3GPP TSG-SA WG6 Meeting #67	S6-252063

Fukuoku, Japan, 17th – 21st May 2025	revision of S6-2xxxxx

Source:	Ericsson

Title:	Enhance to NRM Service on Network Resource Adaptation

Spec:	3GPP TR 23.700-35 v0.3.0

Agenda item:	9.3

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

According to the gaps identified in clause 4.1 (TR 23.700-35), the study focuses on the enhancements of NRM APIs to design NRM services for multi-modal traffic applications. The open issues of the current NRM APIs include:

- The existing input parameters of the SS_NetworkResourceAdaptation API are not suffcient to support the latest network XR capabilities to be used by the XR service, e.g., lack of the support of PDU set handling.

It is important for the NRM APIs to provide sufficient input parameters in the request to add necessary information to support the latest network capabilities. It enables the application server to request the required network resource adaptation as per the nature of the application traffic to the NRM server. This helps the NRM server to ease the usage of 5G network resources with clarity on the traffic type and the priorities.

This paper proposes enhacements of the NRM  network resource adaptation service to support multi-modal traffic. This enables the application server to request network resource adaptation by specifying the type of the traffic along with the priority of the traffics of the application and the QoS requirements for each traffic type of the application. This solution can be useful for the NRM server in the further steps on PDU set handling.

2. Reason for Change

To resolve the gaps identified in clause 4.1(TR 23.700-35) on the insufficient input parameters of SS_NetworkResourceAdaptation API that makes it incapable to support the latest network capabilities such as multi-modality for different applications (e.g, XR). Enhacements of the NRM network resource adaptation service is required that can provide sufficient input parameters to support multi-modal application traffic.

3. Conclusions

This pCR proposes enhacements of the NRM network resource adaptation service to support multi-modal application traffic with input parameters and procedures that helps the applications with latest network capabilities to request QoS requirements as per the traffic type.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35 v0.3.0.

* * * First Change * * * *

### 8.1.Y	Solution#Y: Enhancements of NRM network resource adaptation service

#### 8.1.Y.1	Solution Description

It is important for the NRM APIs to provide sufficient input parameters to support latest network capabilities such as multi-modal application traffic. The existing NRM network resource adaptation service can be enhanced for multi-modal traffic by providing additional input parameters that enables the application client/server to request network resource adaptation by specifying the traffic type along with the priority of the traffics of the application and the QoS requirements for each traffic type of the application. This solution can be useful for the NRM server in the further steps on PDU set handling.

The application server sends the network resource adaptation request with resource adaptation requirements to the NRM server. The resource adaptation requirements are with input parameters specifying the traffic type(s) e.g., audio, video, sensor data along with the priority of the traffic flows. This helps the NRM server for prioritized resource allocation. For example, consider a gaming application with multiple traffic types such as audio, video, haptics etc, the proposed solution allows the application to prioritize the for eg., video type with the highest priority.

The enhanced NRM network resource adaptation service for multi-modal traffic allows the application client/server to add the QoS requirements for each traffic types separately. For example, consider the above mentioned gaming application, QoS requirements such as high reliablility and low latency can be requested for the "video" traffic type and high data rate and low latency can be requested for the "haptic" traffic type.

The enhanced NRM network resource adaptation service for multi-modal traffic can send traffic type specific QoS requirements to the 5GC and can be mapped with the AFSessionWithQoS for multi-modal service. The multi-modal service requirements information for each data flow can be mapped with the QoS requirements of different traffic types.

The proposed input parameters to the enhanced NRM network resource adaptation service for multi-modal traffic makes it suitable for scenarios with single or multiple traffic types, irrespective of the application type.

The following clauses specify the procedures and information flows for gaps identified in clause 4.1 to support the enhancements of the NRM network resource adaptation service for multi-modal traffic.

#### 8.1.Y.2	Impact to existing NRM information flows

The information flows of NRM network resource adaptation service in 3GPP TS 23.434 [3] can be enhanced (highlighted in bold italics) as follows.

### 14.3.2 Information flows #### 14.3.2.1 Network resource adaptation request Table 14.3.2.1-1 describes the information flow network resource adaptation request from the VAL/SEALDD server to the NRM server. Table 14.3.2.1-1: Network resource adaptation request Information element; Status; Description. Requester Identity; M; The identity of the VAL server performing the request.. List of VAL UE IDs; O (see NOTE 1); List consisting of one or more VAL UE IDs for whom the network resource adaptation occurs.. VAL group ID; O (see NOTE 1); The VAL group ID for whom the network resource adaptation occurs.. Resource adaptation requirement(s); M; The resource adaptation requirement corresponds to the VAL service QoS requirements as applied for a UE or group of UEs (E.g. bandwidth, resource).. >Traffic type; O; The different types of traffic involved (e.g, audio, video, haptics etc).. >Priority; O (see NOTE 2); Indicates the priority of the traffic type.. >QoS requirement; M; The QoS requirement corresponds to the VAL service QoS requirement as per the traffic type (e.g, data rate, GBR, MBR, latency, reliability, bandwidth, etc., and/or its combinations).. BDT Reference ID; O; Indicates the Background data transfer Reference ID provided by the 3GPP network. NOTE 1: Either of the information elements should be present. NOTE 2: Priority may be present only when the Traffic type is present.. #### 14.3.2.2 Network resource adaptation response Table 14.3.2.2-1 describes the information flow network resource adaptation response from the NRM server to the VAL server. Table 14.3.2.2-1: Network resource adaptation response Information element; Status; Description. Result; M; Result includes success or failure of the network resource adaptation with the underlying network. The response can also include an updated value for some of the parameters included in the network resource adaptation request (e.g. negotiation of resource offering). Cause; O (see NOTE); Cause of failure (e.g. QoS requirement for traffic type cannot be satisfied).. NOTE: The information element shall be present only when the result is failure..

#### 8.1.Y.3	Solution evaluation

Editor's Note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Changes * * * *