3GPP TSG-SA WG6 Meeting #67	S6-252198

Fukuoka, Japan 19th – 23rd May 2025	(revision of S6-252xxx)

Source:	China Mobile

Title:	Pseudo-CR on Key Issue on Support of virtual number

Spec:	3GPP TR 23.700-57 v0.1.0

Agenda item:	9.6

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

Virtual number is a widely used value added calling service which the real number of user is hid by the service provider to protect the privacy of user. When enables MMTel service to application providers/Vertical service provider, providing virtual number service for the end user is needed to be considered as a necessary value added service.

The Key Issue on virtual number is introduced in the Rel-19 eMMTel_App SID and documented in 3GPP TR 23.700-92 as Key Issue#5. This virtual number feature is also included in the Rel-19 exception sheet (i.e. S6-245646) of MMTel_App WID. The solution of this KI is not provided in Rel-19 due to the lack of IMS session management capabilities provided by the network. In Rel-19, IMS session management capabilities  are introduced to IMS by SA2 in Annex AA.2.4 of 3GPP TS 23.228. Therefore, it is proposed to study this Key Issue again in Rel-20.

This pCR proposes new Key Issue for TS23_700_57.

2. Reason for Change

Contribution with new Key Issue.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-57 v0.1.0.

* * * First Change * * * *

4.x	Key Issue #X: Support of virtual number

4.x.1	Description

A virtual number is a widely used value-added calling service where the user's real number is hidden by the service provider to protect their privacy, especially in the A2P/P2A scenarios. This virtual number is used by one or both parties in a call and is not used in multi-device scenario, i.e. virtual number is not used by the secondary devices, as specified in GSMA PRD NG.110 [ng110]. This feature is also differs from the third party user identity in IMS sessions specified in Annex AF of 3GPP TS 23.228 [4] which allows the called party to use a third-party-specific identity in an IMS session.

This feature differs from the existing Terminating Identification Restriction (TIR) service specified in 3GPP TS 22.173 [22173]. The virtual number looks like a real phone number rather than an anonymous number. A Virtual number may either pre-configured to the caller or assigned to the caller dynamically on demand of a user. There may be one or multiple virtual numbers for the same caller/callee in different calls or some caller may share a same virtual number. In addition, in some cases the callee can use the virtual number to call the caller back after their call is finished or to call the caller back when the call is not answered, but in some cases the virtual number cannot used to call back the caller.

Some examples of virtual number usage in different use cases are listed below. (This is not an exhaustive list and the arrows in the figures below points from the caller to the callee.):

Figure 4.x.1-1 AX mode of virtual number

a)	AX mode of virtual number is shown in Figure 4.x.1-1. When a caller A1 wants to hide its real number in every call, A1 can request a virtual number from the MMTel service provider, i.e. MNO, in AX mode. The MMTel service provider will assign a virtual number, e.g. a MSISDN for this specific usage, to A1. This virtual number applies to A1 in every call until A1 cancels this service from MMTel service provider. If AX mode virtual number applies, the following characteristics apply to the call:

-	outgoing calls: when A1 calls other users, other users, e.g. B1, B2 and B3, will see the caller number is x1 and number of A1 will be hid;

-	incoming calls: when other users call the virtual number x1, the call will go to A1.

Figure 4.x.1-2 AXB mode of virtual number

b)	AXB mode of virtual number is shown in Figure 4.x.1-2. In this mode, the virtual number applies to a specific caller-callee pair. If user A1 wants to hide its real number in the call with user B1only, A1 can request a virtual number x1 from the MMTel service provider, i.e. MNO, in AXB mode. If user A2 wants to hide its real number in the call with user B2 only, A2 can also requests a virtual number x1 from the MMTel service provider, i.e. MNO, in AXB mode. The virtual number x1 can be re-used in different caller-callee pair and applies to all calls between this caller-callee pair until A1 or A2 cancels this service from MMTel service provider. If AXB mode virtual number applies, the following characteristics apply to the call:

-	outgoing calls: when A1 calls B1, the callee B1 will see the caller number is x1 and number of A1 will be hid, when A1 calls other callees, the callees will see the real number of A1. Similarly, when A2 calls B21, the callee B2 will see the caller number is x1 and number of A2 will be hid, when A2 calls other callees, the callees will see the real number of A2.

-	incoming calls: when B1 calls the virtual number x1, the call will go to A1. Similarly, when B2 calls the virtual number x1, the call will go to A2. If users other than B1 and B2 call the virtual number x1, the call will be rejected by the MMTel service provider, i.e. MNO.

-	When A1 wants to use virtual number in the call with different users, e.g. B1, B2 or B3, different virtual number x1, x2 and x3 are needed to be requested from the MMTel service provider.

Figure 4.x.1-3 XB mode of virtual number

c)	XB mode of virtual number is shown in Figure 4.x.1-3. This mode may used by a group of users in a enterprise. If a enterprise wants to hide the every single number in the enterprise and different members in the enterprise can share the same virtual number in different time, the enterprise can request a virtual number from the MMTel service provider, i.e. MNO, in XB mode. This virtual number applies to the group of users in the enterprise until the enterprise cancels this service from MMTel service provider. If XB mode virtual number applies, the following characteristics apply to the call:

-	outgoing calls: XB mode can not apply.

-	incoming calls: when other users call the virtual number x1, the call will go to A1, A2 or A3 respectively in different time.

Figure 4.x.1-4 AXE mode of virtual number

d)	AXE mode of virtual number is shown in Figure 4.x.1-4. This mode may used by a group of users in a enterprise to reduce the cost of number and the call can reach every specific member in the enterprise. The enterprise can request  a virtual number and a series of extension from the MMTel service provider, i.e. MNO, in AXE mode. This virtual number plus the extensions applies to the group of users in the enterprise until the enterprise cancels this service from MMTel service provider. If AXE mode virtual number applies, the following characteristics apply to the call:

-	outgoing calls: AXE mode can not apply.

-	incoming calls: when other users call the virtual number x1 plus extension 101, 102 or 103, the call will go to A1, A2 or A3 respectively.

Figure 4.x.1-5 AXYB mode of virtual number

e)	AXYB mode of virtual number is shown in Figure 4.x.1-5. This mode is an enhancement of AX mode. When both caller A1 and callee B1 want to hide the real number in every call, A1 and B1 can request a virtual number x1 and y1 from the MMTel service provider respectively, i.e. MNO, in AXYB mode. In this mode, A1 and B1 do not know the real number of the other party. If AXYB mode virtual number applies, the following characteristics apply to the call:

-	outgoing calls: when A1 wants to call B1 or B2, A1 should call the number y1 or y2 respectively. B1 and B2 will see the caller number is x1 and number of A1 will be hid;

-	incoming calls: when B1 or B2 wants to call A1, B1 or B2 should call the number x1.A1 will see virtual number y1 or y2 respectively and number of B1 and B2 will be hid.

The user may request the network to assign virtual number(s), and the enterprise may request the network to assign a virtual number(s) for its employee(s). The virtual number(s) will be assigned in one or multiple modes. The network may assign a virtual number to the user on demand, and maintains the mapping between the user’s real number and the virtual number according to the requested mode(s). Considering the high flexibility requirement of the virtual number assignment and maintaining, this feature is not suitable to be handled in IMS.

4.x.2	Open Issue

This key issue aims to study how to support the virtual number service in the MMTel Enabler layer:

1.	How to manage the virtual number in MMTel Enabler layer, e.g. how to assign a virtual number(s) on demand of a user in one or multiple requested mode.

2.	How to maintain the relationship between the virtual number and real number in the requested mode(s) and the related number translation procedures in the call.

Editor's note 1:	Whether the capabilities specified in Annex AA.2.4 of 3GPP 23.228 can support the virtual number studied by SA6 is FFS. Coordination with SA2 is needed.

Editor's note 2	Whether the virtual number related procedures have impact on the procedures in 5GC/IMS is FFS and coordination with SA2 is needed.

Editor's note 3	When the solution of virtual number is proposed, the charging aspect is needed to be studied by SA5

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.392: "Application enablement aspects for MMTel".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[5]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[ng110]	GSMA NG.110: "Multi Device".

[22173]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".