3GPP TSG-SA WG6 Meeting #68	S6-253119

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

Source:	China Mobile

Title:	Solution update to address the ENs on SEALDD enabled data transmission quality guarantee based on traffic characteristics

Spec:	3GPP TR 23.700-51 v0.2.0

Agenda item:	9.5

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

Solution update to address the ENs on SEALDD enabled data transmission quality guarantee based on traffic characteristics.

2. Reason for Change

This paper proposes to address the following three ENs, and adds the solution evaluation.

For “Editor's note: the traffic characteristic analysis service in ADAES server is FFS”, considering the real traffic does not go through the ADAES server, so it is proposed to utilize the NWDAF’s analytics or prediction of user plane performance service.

Regarding “Editor's note: the enhancement for rate control service in SEALDD is FFS”, it is proposed to utilize traffic characteristics to calculate the suggested traffic transmission bandwidth in rate control service.

Regarding “Editor's note: the enhancement for NRM service in SEAL is FFS”, considering there is no existing NRM service that could be utilized to do the transmission enhancement based on traffic characteristic, this EN is removed.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51 v0.2.0.

## 5.3	Solution #2: Transmission enhancement based on traffic characteristics

This solution provides a solution to enhance the transmission for XR traffic based on their traffic characteristics.

### 5.3.1	Architecture Impacts

None

### 5.3.2	Solution description

XR service traffic of different types may exhibit different traffic characteristics,

For Video traffic, taking 2K/60FPS, GOP:30, H.264 encoded DL video traffic as an example, the traffic characteristics including:

- Bitrate:  (2K=2560×1440(Pixels/Frame)) × 1.5 bytes (YUV420) × 60 FPS × 8 ÷ 100 ≈ 26.5 Mbps)

- Traffic Periodicity‌: the frame inter arrival time is 16.67 ms, then the Traffic Periodicity‌ for I frame is 0.5s.

- Packet Size: the size of I frame is generally 2-3 times larger than P frame.

For Audio traffic, characteristics including:

- Bitrate: can be calculated as the sampling rate × bit depth × number of channels;

For Haptic traffic, characteristics including:

- transmission interval: For some tactile sensor, it will have sampling rate (e.g., 1 kHz), so that the data output will be sent at fixed intervals.

In summary, the traffic characteristics can be calculated by SEALDD server are as follows:

For video, the Bitrate can be calculated based on the following equation:

    - Bitrate(kbps)= (Resolution×Raw Frame Size(Resolution×bpp)×FPS×Compression Ratio)/ GOP​×8×10−3

    - Frame inter arrival time= 1/FPS

For audio, the Bitrate can be calculated based on the following equation:

    - Bitrate(kbps)=sampling rate × bit depth × number of channels

Based on the characteristics, the SEALDD layer could enhance the transmission for that traffic based on the 5GC services and SEALDD layer services.

#### 5.3.2.1	SEALDD enabled data transmission quality guarantee based on traffic characteristics

Figure 5.3.2.1-1: SEALDD enabled data transmission quality guarantee based on traffic characteristics

1.	VAL server sends request for SEALDD enabled data transmission quality guarantee based on traffic characteristics. The request may include the media configuration information (i.e., Video: Resolution, encoded method (e.g., Compression Ratio, Color Encoding Model (bits per pixel, bpp)), Frames Per Second (FPS), group of pictures (GOP); Audio: sampling rate, bit depth, number of channels; Haptic: sampling rate, etc.).

2a.	SEALDD server calculates the traffic characteristics based on received media configuration information.

2b.	If the received media information is not enough to calculate the traffic characteristics, the SEALDD server request analytics or prediction of user plane performance (i.e. average/maximum traffic rate) from NWDAF (or via NEF) as defined in TS 23.288[x], clause 6.14.4.

NOTE: the Bitrate calculation based on traffic rate is implementation based, depending on specific transmission protocol.

3a.	Based on the traffic characteristics from step2, SEALDD server could do the transmission enhancement by request the rate control based on the bitrate. The traffic characteristics could be used to calculate the suggested traffic transmission bandwidth to the SEALDD client, as defined in TS 23.433[6], clause 9.8.2.1, step7.

3b.	SEALDD server request AF Session with required QoS as defined in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3] can be reused to provision the protocol description and the calculated Traffic Periodicity, bitrate.

### 5.3.3	Solution evaluation

This solution address KI#2, by analyzing the XR traffic characteristics, and providing a solution to enhance the transmission for XR traffic based on their traffic characteristics. This solution has no architecture imapct

.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[6]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[7]	3GPP TR 26.926: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

[8]	3GPP TS 23.542: "Application layer support for Personal IoT Network".

[x]	3GPP TS 23.288: " Architecture enhancements for 5G System (5GS) to support network data analytics services ".

…

* * * Next Change * * * *

<Proposed change in revision marks>