3GPP TSG-SA WG6 Meeting #68	S6-253135

Gothenburg, Sweden 25th – 29th August 2025

Source:	China Mobile

Title:	New KI on efficient content delivery over satellite access

Spec:	3GPP TR 23.700-02 v0.0.0

Agenda item:	9.12

Document for:	Approval

Contact:	Tianji Jiang, tianjijiang@chinamobile.com

1. Introduction

Add new KI to enhance the application enablement layer for the support of efficient content delivery over the satellite access.

2. Reason for Change

The 3GPP stage-1 spec. TS 22.261 provides the service requirments for the support of efficient content delivery. Emphasizing the video-based services and personal data storage applications, it brings up that the in-network content caching can be provided by an operator, a third-party or both, to improve user experience, reduce backhaul resource usage and utilize radio resource efficiently. The operation of in-network caching includes the location management of the content cache as well as the efficient delivery of content to and from the appropriate content caching applications. The location requirement includes the support of content caching applications in both the network (e.g., AN/access network and CN/core network) and the UE (e.g., terminal equipment, smartphone, etc.).

When the services are extended to accommodate the satellite access, the stage-1 spec. requires, quoted as:

A 5G system with satellite access optimize the delivery of content from a content caching application by taking advantage of satellites in supporting ubiquitous service on very large to global coverages.

This extension suggests three aspects: (1) ubiquitous coverage over a large to global scope implies the support of distributed content caching applications; (2) the distributedness necessitates the cache deployment in AN, CN, and even in UE; and (3) efficient content delivery from application servers with the consideration of satellite access characteristics.

The SEAL spec. TS 23.434 references the data delivery enabler, i.e., SEALDD, to ease the diversified data delivery demands for VAL applications. SEALDD offers not only the advanced data delivery capabilities, but also the storage capacities. The SEALDD spec. TS 23.433 clauses 4.6 and 9.5 indicate that the SEALDD server supports the data storage and storage management for VAL client/server, SEALDD client and other SEALDD servers (via the SEALDD-E reference point). The VAL client and VAL server may coordinate to determine the usage of the SEALDD storage service. As we can project, while the content caching service may be leveraged via the storage service as provided by the SEALDD server, the normal deployment locations of SEALDD servers makes the distributedness of content caches less desirable, which posts challenge to the required ubiquitous coverage over a large to global scope for satellite access. In TS 23.434 upon describing the NRM handling the satellite S&F services, the clause 21.3.2.2 says a VAL UE may report (via the NRM client on the UE) the maximum data storage quota which indicates the maximum data storage quota per application layer on the UE for all of services. This suggests a content caching application might be deployable on a UE for better distributedness.

TS 23.434 clause 21, SEAL services over satellite access, implies the satellite footprint or satellite coverage availability information (SCAI) as defined in SA2 TS 23.501 can be leveraged for efficient content delivery. It describes a CM (i.e., Configuration Management) server can provide the SCAI to consumers (e.g., VAL UE, CM client, VAL server, etc.) to guide to minimize the service impact (e.g. indicate when to trigger the UL/DL service flow if the VAL UE is accessing the satellite). TR 23.700-01 indicates that supplying the discontinuous coverage pattern between UEs and AS/AFs to the application layer will help applications design themselves for handling discontinuity accordingly. For instance, if the intermittent connectivity pattern is predicted in advance and then exposed to the application enablement layer, then the application layer could use this information to better schedule content delivery to distributed caching applications (e.g. UEs supporting caching applications may be given precedence during a connectivity available interval, UEs with large caching demand may be better off utilizing longer connecvitiy interval, etc.).

This Key issue is proposed to study the enhancement of the application enablement architecture for the support of efficient content delivery over the satellite access:

- Whether and how to enhance the application enablement architecture layer to support the deployment of content caching applications for efficient content delivery.

- Whether and how to enhance the application enablers (e.g., SEALDD, NRM, etc.) to optimize the content delivery over the satellite connectivity by leveraging the satellite access characteristics (e.g., satellite ephemeris, SCAI or satellite coverage availability information as defined in SA2 TS 23.501, QoS parameters for traffic over the satellite access, etc.).

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-02 v0.0.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x1]	3GPP TS 22.261: "Service requirements for the 5G system".

[x2]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[x3]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[x4]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[x5]	3GPP TR 23.700-01: "Study on application enablement for Satellite access enabled 5G Services”.

* * * Next Change (all new texts) * * * *

## 4.x	Key issue #x: Application enablement layer enhancement for efficient content delivery over satellite access

The 3GPP stage-1 spec. TS 22.261[x1] provides the service requirments for the support of efficient content delivery. Emphasizing the video-based services and personal data storage applications, it brings up that the in-network content caching can be provided by an operator, a third-party or both, to improve user experience, reduce backhaul resource usage and utilize radio resource efficiently. The operation of in-network caching includes the location management of the content cache as well as the efficient delivery of content to and from the appropriate content caching applications. The location requirement includes the support of content caching applications in both the network (e.g., AN/access network and CN/core network) and the UE (e.g., terminal equipment, smartphone, etc.).

When the services are extended to accommodate the satellite access, the stage-1 spec. requires, quoted as:

A 5G system with satellite access optimize the delivery of content from a content caching application by taking advantage of satellites in supporting ubiquitous service on very large to global coverages.

This extension suggests three aspects: (1) ubiquitous coverage over a large to global scope implies the support of distributed content caching applications; (2) the distributedness necessitates the cache deployment in AN, CN, and even in UE; and (3) efficient content delivery from application servers with the consideration of satellite access characteristics.

The 3GPP SEAL spec. TS 23.434[x3] references the data delivery enabler, i.e., SEALDD, to ease the diversified data delivery demands for VAL applications. SEALDD offers not only the advanced data delivery capabilities, but also the storage capacities. The SEALDD spec. The 3GPP TS 23.433[x2] clauses 4.6 and 9.5 indicate that the SEALDD server supports the data storage and storage management for VAL client/server, SEALDD client and other SEALDD servers (via the SEALDD-E reference point). The VAL client and VAL server may coordinate to determine the usage of the SEALDD storage service. As we can project, while the content caching service may be leveraged via the storage service as provided by the SEALDD server, the normal deployment locations of SEALDD servers makes the distributedness of content caches less desirable, which posts challenge to the required ubiquitous coverage over a large to global scope for satellite access. In TS 23.434[x3] upon describing the NRM handling the satellite S&F services, the clause 21.3.2.2 says a VAL UE may report (via the NRM client on the UE) the maximum data storage quota which indicates the maximum data storage quota per application layer on the UE for all of services. This suggests a content caching application might be deployable on a UE for better distributedness.

TS 23.434[x3] clause 21, SEAL services over satellite access, implies the satellite footprint or satellite coverage availability information (SCAI) as defined in 3GPP TS 23.501[x4] can be leveraged for efficient content delivery. It describes a CM (i.e., Configuration Management) server can provide the SCAI to consumers (e.g., VAL UE, CM client, VAL server, etc.) to guide to minimize the service impact (e.g. indicate when to trigger the UL/DL service flow if the VAL UE is accessing the satellite). The 3GPP TR 23.700-01[x5] indicates that supplying the discontinuous coverage pattern between UEs and AS/AFs to the application layer will help applications design themselves for handling discontinuity accordingly. For instance, if the intermittent connectivity pattern is predicted in advance and then exposed to the application enablement layer, then the application layer could use this information to better schedule content delivery to distributed caching applications (e.g. UEs supporting caching applications may be given precedence during a connectivity available interval, UEs with large caching demand may be better off utilizing longer connecvitiy interval, etc.).

This Key issue is proposed to study the enhancement of the application enablement architecture for the support of efficient content delivery over the satellite access:

- Whether and how to enhance the application enablement architecture layer to support the deployment of content caching applications for efficient content delivery.

- Whether and how to enhance the application enablers (e.g., SEALDD, NRM, etc.) to optimize the content delivery over the satellite connectivity by leveraging the satellite access characteristics (e.g., satellite ephemeris, SCAI or satellite coverage availability information as defined in the 3GPP TS 23.501[x4], QoS parameters for traffic over the satellite access, etc.).

* * * End Change * * * *