3GPP TSG-SA6 Meeting #68	S6-253148

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

Source:	Jio Platforms

Title:	Enhancements to AIMLE data management assistance to extend assistance on external data

Spec:	3GPP TR 23.700-83

Agenda item:	9.4

Document for:	Approval

Contact:	Raghvendra Bhushan <Raghvendra.Bhushan@ril.com>

1. Introduction

The Key Issue#6, on the study aspects on support enhancements related to diverse AIMLE deployments scenarios includes:

- Whether and how to enhance data management assistance among AIMLE Clients, edge AIMLE servers, and central AIMLE servers in diverse (e.g., hierarchical and distributed) AIMLE deployments scenarios.

The data management assistance procedure mentioned in 8.15 has a precondition that “UE application data for AI/ML operations have been collected and dataset identifier have been assigned to each dataset. EVEX mechanism can be reused for data collection as described in 3GPP TS 26.531 [10].”. However, the scenario where a UE needs to perform data analysis and data preparation on external data collected by some other UE and hosted on a trusted URL by the network operator ensuring data privacy.

SA2 is working on data collection mechanism in 23.700-04 and a data collection function is introduced. So, a framework can be designed to process and analyse raw data collected by DCF on some other high capability AIMLE clients supporting high performance data analysis or preparation functions. Hence, this paper proposes a new mechanism “external data management assistance” as an extension to existing “AIMLE data management assistance” defined in clause 8.15 of TS 23.482. The data collection may follow aspects mentioned in clause mentioned in 6.2.8 and 6.2.9 of TS 23.288

2. Reason for Change

To introduce external data management assistence in 3GPP data management framework.This data may be collected by UE/AIMLE client (not supporting required analysis function) and processed by some other UE/AIMLE clients supporting required data analysis/data preparation functions.

3. Conclusions

This paper proposes solution for an AIMLE client to process external data on hosted on some trusted URL. This data may be collected by a data collection function. The data sharing to AIMLE clients shall be done ensuring data security

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0.

* * * First Change * * * *

## 8.15	AIMLE data management assistance

### 8.15.1	General

AIMLE data management assistance is the process of the AIMLE server assisting AIMLE service consumers with managing data operations performed by VAL clients. The data operations include data preparation and data analysis. The AIMLE server offloads the AIMLE service consumer from interacting with AIMLE clients to manage the data operations. The VAL clients perform the actual data preparation and data analysis operations and send the outputs to the AIMLE server for aggregation.

The following clauses specify procedures, information flows, and APIs for AIMLE data management assistance.

### 8.15.2	Procedure

Pre-conditions:

1.	AIMLE clients have registered with AIMLE server.

2.	UE application data for AI/ML operations have been collected and dataset identifier have been assigned to each dataset. EVEX mechanism can be reused for data collection as described in 3GPP TS 26.531 [10].

Figure 8.15.2-1: AIMLE data management assistance

1.	An AIMLE service consumer (e.g., VAL server) makes a request for AIMLE data management assistance. The request includes information as described in Table 8.15.3.1-1.

2.	The AIMLE server authenticates and authorizes the request. If authorized, the AMILE server assigns an identifier for the subscription.

3.	The AIMLE server sends an AIMLE data management assistance subscription response to the AIMLE service consumer. The response includes information as described in Table 8.15.3.2-1.

4.	The AIMLE server sends Client data processing trigger requests to AIMLE clients. The request can be for data preparation or data analysis. The request includes information as described in Table 8.15.3.4-1.

5.	Each AIMLE client sends the requirements to trigger data management for the VAL client to perform the requested data operation. The VAL client performs the operation locally.

NOTE:	The data preparation and data analysis functions operate in a similar manner as ML models. The AIMLE layer is able to transport the functions to the VAL clients and the VAL clients performs the associated function on the application data as part of data preparation or data analysis in a similar manner as ML training.

6.	After the data operation completes, each AIMLE client sends a response to the AIMLE server with information as described in Table 8.15.3.5-1.

7.	The AIMLE server aggregates the output of each AIMLE client. Aggregation includes combining or performing a statistical operation on the received data. Data received from the AIMLE clients can be numerical or categorical, which allows the AIMLE server to aggregate the output data from the AIMLE clients. If necessary, steps 4–7 is repeated to complete all the required data operations.

8.	The AIMLE server sends an AIMLE data management assistance notification to the AIMLE service consumer with information as described in Table 8.15.3.3-1.

### 8.15.2.X	AIMLE external data management assistance

### 8.15.2.X.1	General

The AIMLE external data management assistance manages data operations performed by VAL clients on data collected by extenal sources (like IoT devices). This data is shared to AIMLE client by AIMLE server or VAL server. The AIMLE client downloads the dataset from trusted URL and updates its registeration details with new dataset ID

### 8.15.2.X.2	Procedure

Pre-conditions:

1.	AIMLE clients have registered with AIMLE server.

2.	For external data management, VAL Server/AIMLE service consumer has endpoint of data on which analysis or preparation is required or it at least has the dataset ID for AIMLE server to identify a dataset

Figure 8.15.2.X.1-1: AIMLE external data management assistance

- The VAL server sends AIML data assistance request to AIMLE server. This request may contain URL of data along with external data set identifier and checksum. If URL is not sent by VAL server, the AIMLE server shall identify the URL of dataset using the dataset ID shared by VAL server. The request includes information as described in Table 8.15.2.X.2.1-1

- The AIMLE server authenticates and authorizes the request. If authorized, the AIMLE prepares distribution of URLs and data set ID to AIMLE clients.

- The AIMLE server sends an AIMLE data management assistance subscription response to the AIMLE service consumer. The response includes information as described in Table 8.15.2.X.2.2-1

- The AIMLE server sends client data processing trigger request containing the URL and dataset identifier of external data to be processed by UE along with checksum.

- For external dataset management, The AIMLE client downloads the dataset and updates its registration with new dataset ID as defined in clause 8.7.3.2

- Steps 5 to 8 of clause 8.15.2 shall follow further.

### 8.15.3	Information flows

#### 8.15.3.1	AIMLE data management assistance subscription request

Table 8.15.3.1-1 shows the request sent by an AIMLE service consumer to an AIMLE server for the AIMLE data management assistance subscription procedure.

Table 8.15.3.1-1: AIMLE data management assistance subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Data management operations | M | An indicator showing what data management type is being requested: data preparation, data analysis. |
| Data management requirements | M | Requirements for the data management request: |
| External Data management Requirements | O | URL of external data, checksum, Auth details and data schema |
| >Data preparation requirements | O (NOTE 1) | Data preparation requirements as detailed in Table 8.15.3.1-2. |
| >Data analysis requirements | O (NOTE 1) | Data analysis requirements as detailed in Table 8.15.3.1-3. |
| AIMLE clients list | O (NOTE 2) | A list of AIMLE clients for which data management should be performed. The list may be specified by AIMLE client set identifier. |
| AIMLE client selection criteria | O (NOTE 2) | Selection criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2. |
| NOTE 1: At least one of the information elements shall be provided. NOTE 2: At least one of the information elements shall be provided. |  |  |

Table 8.15.3.1-2: Data preparation requirements

| Information element | Status | Description |
| --- | --- | --- |
| Dataset identifier | M | An identifier for the dataset. |
| External Dataset identifier | O (NOTE) | An identifier for the external dataset. |
| Data preparation requirements | M | Requirements for data preparation. |
| > Dataset ID | M | The identifier for the dataset. |
| > Dataset feature ID | M | Identifier or name of dataset feature to process. |
| > Data preparation function | M | This indicates the function which prepares the data, and it could be an identifier of a function if the function is available locally at the UE or an executable included in the request. |
| NOTE: Mandatory for external data management. |  |  |

Table 8.15.3.1-3: Data analysis requirements

| Information element | Status | Description |
| --- | --- | --- |
| Dataset identifier | M | An identifier for the dataset. |
| External Dataset identifier | O (NOTE) | An identifier for the external dataset |
| Dataset analysis requirements | M | Requirements for data analysis. |
| > Dataset ID | M | The identifier for the dataset. |
| > Dataset feature ID | M | Identifier or name of dataset feature. |
| > Data analysis function | M | This indicates the function which performs the data analysis, and it could be an identifier of a function if the function is available locally at the UE or an executable included in the request. |
| NOTE: Mandatory for external data management. |  |  |

#### 8.15.3.2	AIMLE data management assistance subscription response

Table 8.15.3.2-1 shows the response sent by the AIMLE server to the AIMLE service consumer for the AIMLE data management assistance subscription procedure.

Table 8.15.3.2-1: AIMLE data management assistance subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the data management operation |
| Subscription identifier | M | An identifier for the subscription. |
| Expiration time | O | Expiration time for the subscription |

#### 8.15.3.3	AIMLE data management assistance notify

Table 8.15.3.3-1 shows the notification sent by the AIMLE server to the AIMLE service consumer for the AIMLE data management assistance subscription procedure.

Table 8.15.3.3-1: AIMLE data management assistance notify

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the data management operation |
| Aggregated data preparation outputs | O (NOTE 1) (NOTE 2) | Provides outputs for data preparation: dataset identifier, dataset features, and prepared data output. |
| Aggregated data analysis outputs | O (NOTE 1) (NOTE 2) | Provides outputs for data analysis: dataset identifier, statistical outputs for each feature, list of outlier and anomaly values, and feature correlation information. |
| Timestamp | O | Timestamp of the data management notification |
| NOTE 1: At least one of the information elements shall be provided in the output. NOTE 2: The output format can be numerical or categorical. If categorical, the format can be nominal or ordinal. |  |  |

#### 8.15.3.4	Client data processing trigger request

Table 8.15.3.4-1 shows the request sent by the AIMLE server to AIMLE clients for the AIMLE client data processing trigger procedure.

Table 8.15.3.4-1: Client data processing trigger request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Data management type | M | An indicator showing what data management type is being requested: data preparation, data analysis. |
| External Data management Requirements | O (NOTE X) | URL of external data, checksum, Auth details and data schema |
| Data management requirements | M | Requirements for the data management request: |
| >Data preparation requirements | O (NOTE) | Data collection requirements as detailed in Table 8.15.3.1-2. |
| >Data analysis requirements | O (NOTE) | Data collection requirements as detailed in Table 8.15.3.1-3. |
| Operational schedule | O | A schedule to perform the requested data management operation. |
| NOTE: At least one of the information elements shall be provided. NOTE X: Mandatory for external data management. |  |  |

#### 8.15.3.5	Client data processing trigger response

Table 8.15.3.5-1 shows the response sent by AIMLE clients to the AIMLE server for the Client data processing trigger procedure.

Table 8.15.3.5-1: Client data processing trigger response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the data management operation |
| Data preparation outputs | O (NOTE 1) | The output data after performing data preparation. One output data is generated for each requirement. (NOTE 2) |
| Data analysis outputs | O (NOTE 1) | The output data generated by data analysis. One output data is generated for each requirement. (NOTE 2) |
| Timestamp | O | Timestamp of the data management operation |
| NOTE 1: At least one of the information elements shall be provided in the output. NOTE 2: The output format can be numerical or categorical. If categorical, the format can be nominal or ordinal. The AIMLE server is able to process either type of data. |  |  |

# *** End of Changes ***