3GPP TSG-SA6 Meeting #68	S6-253154

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0046 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modifications to ML model capability evaluation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Jio Platforms |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-08-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The output of an AIMLE training can be evaluated on two parameters i.e. the generated output and the time taken to generate output. In asynchronous training, a merging alogrithm will merge the stale output of UE/AIMLE client to a model with low weights as the model has changed due to early updates of fast AIMLE clients. Hence, capability of a AIMLE client to train a ML model shall be based on output it produced for a AIML training along with the time it took to generate the output. This will also let AIMLE server compute time required to train model for multiple training iterations by the client. In TS 23.482, there is a service to evaluate AIMLE client’s capability to train a ML model in clause 8.19. This service considers only output generated by client to judge if a client has sufficient capability to train a ML model or not. The client output generation time shall also be considered to check if a client is capable of training a model or not. The dependency on output reception time can be deceptive as it involves network delays. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Its proposed to add a “Test duration” parameter in “ML Model training capability evaluation response” to indicate how much time was taken by client to generate the output. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The client capability to train a ML model will not be correctly evaluated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 8.19.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

#### 8.19.3.2	ML model training capability evaluation response

Table 8.19.3.2-1 shows the response sent by FL members (AIMLE Clients which are deployed on UEs) to AIMLE server for the ML model training capability evaluation.

Table 8.19.3.2-1: ML model training capability evaluation response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the evaluation: success, fail. - Success means join the FL training process. - Fail means not join the FL training process. |
| Test result | O | The test result of the ML model training capability evaluation. The "test result" shall be provided when the "status" is "success". |
| Test Duration | O | The time duration taken by UE to generate test result |
| Fail reason | O | The reason of the ML model training capability evaluation fail. The "fail reason" shall be provided when the "status" is "fail". |

* * * End of Changes * * * *