3GPP TSG-SA WG6 Meeting #68	S6-253264

Göteborg, Sweden, 25th – 29th August 2025	revision of S6-25xxxx

Source:	Ericsson

Title:	New solution for KI#2 on ML Model Performance Evaluation with information collection from ML model consumer

Spec:	3GPP TR 23.700-83 v0.2.0

Agenda item:	9.4

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

During the lifecycle of an AIML application, it is crucial to evaluate the performance of the ML model to ensure that it aligns with the expectations for the AIML application. Consequently, TR 23.700-83 v0.2.0 identifies the following key issue to be addressed:

Key Issue #2: Support enhancements for evaluation and determination of ML model performance and correctness within the AIML Enablement Layer

It is anticipated that during AIMLE Phase 2 in Release 20, procedures will be developed to foster the evaluation of the ML model in response to Key Issue #2. This document follows the same direction and proposes a new solution on ML model performance evaluation considering ML model performance information collection from ML model consumer. Two methods are considered for collecting the ML model performance information, including communication between AIMLE server and the ML model consumer via A-DCCF and direct communication between the AIMLE server and the ML model consumer.

2. Reason for Change

To improve the support for ML model performance evaluation functionally.

3. Conclusions

This paper proposes solution on ML model performance evaluation with information collection from ML model consumer.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0.

* * * First Change * * * *

## 7.Y	Solution#Y: ML Model Performance Evaluation with information collection from ML model consumer

### 7.Y.1	Solution Description

This clause describes the procedure and information flows for supporting ML model performance evaluation with information collection from ML model consumer.

#### 7.Y.1.1	Procedure

Figure 7.Y.1.1-1: ML model performance evaluation with information collection from ML model consumer

Figure 7.Y.1.1-1 illustrates the procedure on ML model performance with information collection from ML model consumer. The corresponding procedure in detail is as follow:

1. The VAL server sends to the AIMLE server the ML model evaluation subscription request message containing the ML model ID, the VAL Service ID, and the purpose on the ML model the information as specified in Table 7.Y.1.2.1-1.

2. The AIMLE server sends the ML model performance evaluation subscription response message to the VAL server, containing a success message if the VAL server is authorized to subscribe. Otherwise, an error message is sent to the VAL server. The requested message includes the information specified in Table 7.Y.1.2.2-1.

3. The AIMLE sends to the A-DCCF the ML model performance information collection request to get the ML performance of the model. The message contains the information as specified in Table 7.Y.1.2.3-1.

4. The A-DCCF determine the list of ML model consumers based on ML model consumer list, the ML model ID, and the info on the purpose on the use the ML model, and the VAL service ID.

5. The A-DCCF sends to the ML model consumer the information collection request message to obtain the ML model performance of the respective ML model consumer. The information collection request message contains the ML model info (e.g., ML model ID and the purpose on use the ML model) and it is specified in Table 7.Y.1.2.4-1.

6. The ML model consumer generate the ML model performance information based on usage of the ML model.

7. Upon generating the ML model performance, the ML model consumer sends to the A-DCCF the information collection response message containing information as specified in Table 7.Y.1.2.5-1.

8. Upon receiving the Information collection response, the A-DCCF aggregates the information received from each ML model Consumer.

9. The A-DCCF contacts the AIMLE server with the ML model performance information collected from the ML model consumer. The message contains the ML model info (e.g., ML model ID), the purpose on the usage of ML model, ML model consumer info, the ML model performance metrics and it is specified in Table 7.Y.1.2.6-1.

10. Based on the ML model performance received in the previous step, the AIMLE server computes the ML model performance metrics based on the information provided by the ML consumers.

11. The AIMLE server sends to the VAL server the ML model performance evaluation subscription notification containing the information about the performance of the requested ML model. The detail information contains in the notification is as specified in Table 7.Y.1.2.7-1.

Alternatively, the AIMLE server collects ML model performance information from the ML model consumer directly.

#### 7.Y.1.2	Information Flows

##### 7.Y.1.2.1 ML model evaluation subscription request

Table 7.Y.1.2.1-1 describes information elements for the ML model evaluation subscription request from the VAL server to the AIMLE server.

Table 7.Y.1.2.1-1: ML model evaluation subscription request

| Information element | Status | Description |
| --- | --- | --- |
| ML model ID | M | The ML model identifier |
| VAL service ID | M | The identifier of the VAL service |
| Purpose on the ML model | M | Describes the purpose of the ML model (e.g., prediction, classification) |

##### 7.Y.1.2.2 ML model evaluation subscription response

Table 7.Y.1.2.2-1 describes information elements for the ML model evaluation subscription response from the AIMLE server to the VAL server.

Table 7.Y.1.2.2-1: ML model evaluation subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Model ID | M | The identifier of the Model |
| VAL service ID | M | The identifier of the VAL service |
| Request successful | O (NOTE) | ML model evaluation request successful message |
| Error message | O (NOTE) | ML model evaluation request error message |
| NOTE: At least one of these IE shall be present. |  |  |

##### 7.Y.1.2.3 ML model performance information collection request

Table 7.Y.1.2.3-1 describes information elements for the ML model performance information request from the AIMLE server to the A-DCCF.

Table 7.Y.1.2.3-1: ML model performance information collection request

| Information element | Status | Description |
| --- | --- | --- |
| ML model ID | M | The ML model identifier |
| ML consumer List | M | The list of consumers that uses the ML model identified by the ML model ID |
| VAL service ID | M | Indicates the VAL service ID |
| Purpose on the usage of ML model | O | The ML model purpose (e.g., prediction, classification) |

##### 7.Y.1.2.4 Information collection request

Table 7.Y.1.2.4-1 describes information elements for the information collection request from the A-DCCF to the ML model consumer.

Table 7.Y.1.2.4-1: Information collection request

| Information element | Status | Description |
| --- | --- | --- |
| ML model ID | O (NOTE) | The ML model identifier |
| Analytics ID | O (NOTE) | The identifier of the analytics service. |
| Data source IDs | M | The Identifier of the data sources |
| Purpose on the usage of ML model | O | The ML model purpose (e.g., prediction, classification) |
| Interest time period of the historical data | O | Interest time period of the historical data. |
| NOTE: At least one of the IE shall be provided. |  |  |

##### 7.Y.1.2.5 Information collection response

Table 7.Y.1.2.5-1 describes information elements for the Information collection response from the ML model Consumer to the A-DCCF.

Table 7.Y.1.2.5-1: Information Collection response

| Information element | Status | Description |
| --- | --- | --- |
| ML model ID | O (NOTE) | The ML model identifier |
| Analytics ID | O (NOTE) | The identifier of the analytics service. |
| ML model performance information | M | It contains the aggregated information regarding the performance of the ML model identified by the model ID on the data sources (e.g., accuracy, F1 score, AUC). |
| Data source IDs | O | The Identifier of the data sources used to collect the ML model performance |
| Interest time period of the historical data | O | Interest time period of the historical data. |
| NOTE: At least one of the IE shall be provided. |  |  |

##### 7.Y.1.2.6 ML model performance information collection response

Table 7.Y.1.2.6-1 describes information elements for the ML model performance information response from the A-DCCF to the AIMLE server.

Table 7.Y.1.2.6-1: ML model performance information collection response

| Information element | Status | Description |
| --- | --- | --- |
| ML model ID | M | The ML model identifier |
| ML consumer list | M | The list of consumers that uses the ML model identified by the ML model ID |
| ML model performance indication | M | It contains the information regarding the performance of the ML model identified by the model ID. (e.g., accuracy, F1 score, ROC, AUC) |
| Purpose on the usage of ML model | O | The ML model purpose (e.g., prediction, classification) |

##### 7.Y.1.2.7 ML model evaluation notification

Table 7.Y.1.2.7-1 describes information elements for the ML model evaluation notification from the AIMLE server to the VAL server.

Table 7.Y.1.2.7-1: ML model evaluation notification

| Information element | Status | Description |
| --- | --- | --- |
| Model ID | M | The identifier of the model |
| VAL service ID | M | The identifier of the VAL service |
| ML model performance information | M | It contains the aggregated information regarding the performance of the ML model identified by the Model ID on the data sources. (e.g., accuracy, F1 score, AUC) |

### 7.Y.2	Architecture Impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 7.Y.3	Corresponding APIs

Editor's Note:	The corresponding APIs for supporting the solution is FFS.

### 7.Y.4	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

* * * End of Changes * * * *