3GPP TSG-SA WG6 Meeting #68	S6-253314

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

Source:	Huawei, Hisilicon

Title:	Key Issue 1 AIoT enabler architecture

Spec:	3GPP TR 23.700-26 v0.0.0

Agenda item:	9.8

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This solution aims to address the issues identified in Key Issue 1 and provide a high-level architecture and functional model.

2. Reason for Change

AIoT services provide raw data that has significant gaps in semantics, granularity and business context, compared to the requirements of industry applications, making it unsuitable for direct use. For the downlink task data, the same problem also exists. It is necessary to duild a enabler layer to translate the raw data from AIoT into business events, and to translate business task to AIoT inventory task, to achieve compatibility with the applications.

3. Conclusions

To ensure the usability and effectiveness of the entire AIoT system, it is necessary to establish AIoT enable server which connects the AIoT network and the APP.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 v0.0.0.

* * * First Change * * * *

x.y	Solution #x: High – level architecture and funcitonal model

The AIoT network provides fundamental tag inventory service with typical output AIoT data is about "Tag X was seen at Reader Y at Time Z". However, the upper-layer applications require high-level, semantic-rich information such as "which business object, at what time, in which logical location, experienced what meaningful business event, and associated with which business attributes". To ensure better adaptation and integration between the AIoT network and the application layer, it is necessary to build AIoT-enabled service capabilities to bridge the gap between AIoT basic data and APP business data.

As shown in the figure x.y-1, AIoT-enabled services encompass two major foundational capabilities:

Figure x.y-1 AIoT enabler architecture and functional model

AIoT enabler server supports the following functionalities:

AIoT service provisioning: which is used to maintain foundational data, including AIoT tag data, AIoT regional data, business object data, business regional data, and scenario configurations. Through the maintenance of foundational data, it achieves the initial association between AIoT data and business data, thereby infusing business semantics into AIoT data.

AIoT task control: which is used to manage the conversion between the APP layer and the AIoT network, including the downstream between the business tasks and AIoT inventory tasks, and the uplinkstream of AIoT inventory results and business results. It achieves semantic translation of uplink and downlink data, enabling data adaptation.

The entire end-to-end data processing workflow is illustrated in the following figure x.y-2 :

Figure x.y-2 AIoT enabler architecture and functional model

1.	Perform basic configurations with the application layer business requirements, including the registration of AIoT tags, AIoT regions, business objects, and business areas. Then, associate AIoT tags with business objects, as well as AIoT regions with business areas. Simultaneously, establish the scenario-based processing model for the application layer business. For instance, in the scenario of goods outbound, AIoT data will be processed according to the outbound logic. After completing the basic configuration, the application layer can invoke AIoT services based on business requirements.

2.	AIoT Inventory Task Generation and Distribution. The AIoT Enabled Server receives business tasks issued by the application layer and, in conjunction with scenario-based processing models. The AIoT enabler server transforms these business tasks into AIoT inventory tasks. For example, the application layer business task "Execute Task 1 at Checkpoint 1" is translated into the AIoT inventory task " inventory command with service area 1(corresponding to Checkpoint 1)."

3.	The AIoT network begins inventorying and reports the results to the AIoT Enabled Server.

4.	The AIoT-enabled server receives the AIoT inventory results and, based on the AIoT business configuration information and scenario processing models, performs scenario-based processing on the AIoT inventory results to form business events. For example, the AIoT inventory result "Tag X was seen at Reader Y at Time Z" is translated into "Cargo 1 has completed receipt at Checkpoint 1." Depending on the different requirements at the application layer, the AIoT-enabled server can directly report the business event or further process it in conjunction with business logic to form a business outcome, such as "The receipt task at Checkpoint 1 has been successfully completed," and then report it.