3GPP TSG-SA WG6 Meeting #68	S6-253334

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

Source:	Lenovo

Title:	Solution on support for ML model split learning using relays

Spec:	3GPP TR 23.700-83 v0.2.0

Agenda item:	9.4

Document for:	Approval

Contact:	Manos Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

The paper proposes a solution on supporting ML model split learning use cases.

2. Reason for Change

In Stage 1 Rel-19, further use cases have been identified for Split AI/ML operations in 3GPP TR 22.876, for:

- Split AI/ML operation between AI/ML endpoints for AI inference by leveraging direct device connection

- Local AI/ML model split on factory robots

One of the key differences as in below figure was the use of a relay UE to support meeting the performance requirements in case of the source UE being in bad channel conditions or lack of resources. This relaying for the factory robot use cases could be due to the joint local processing at the robot side to minimize the end to end latency for the AI inference process.

Figure below (TR 22.876) shows the case when using direct device connection (sidelink) to realize the proximity-based work task offloading. In this case, the data rate on Uu need not be increased while the original UE’s computation load is offloaded.

As part of AIMLAPP, support for split AI/ML operations has been introduced in 3GPP TS 23.482. The current framework is for single AIMLE client-to-network interaction and does not consider multiple AIMLE clients collaborating in a split AI/ML scenario. If multiple AIMLE clients were to collaboratively execute split AIML operations within the same pipeline involving an Edge Server, the operation continuity may be disrupted if one client moves out of the service area during execution. Therefore, it is important to study the service continuity and optimization for split AIML operations involving multiple AIMLE clients.

In addition to the above multi-client scenario, emerging AI/ML applications require collaborative inference across multiple computing nodes (UE(s), edge(s), and cloud(s)) rather than a single operation. For example, in an autonomous driving or V2X scenario, a VAL UE (e.g. a connected vehicle) may only execute part of a model locally and rely on a roadside edge server or even a cloud server to complete the remaining inference. This is due to the vehicle’s limited computing resources and the need for low-latency results to ensure safe and efficient driving decisions.

In Release-19, basic support for split ML model inference between an AIMLE client and an AIMLE server was introduced in 3GPP TS 23.482 [2]). However, those solutions assumed a relatively static allocation of inference tasks and did not fully explore more dynamic or collaborative inference involving multiple nodes. Therefore, it is necessary to study whether and how to enhance the AIML application enablement architecture and functions to support cloud-edge-device collaborative inference considering the conditions changes on these computing nodes (e.g. due to variations in edge server load).

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0.

* * * First Change * * * *

## 7.x	Solution #x: Support for ML model split learning using relays

### 7.x.1	Solution description

This solution addresses Key Issue #4. The solution provides a mechanism for identifying and determining an intermediate entity (UE, edge function) to serve as relay for a split learning operation, and particularly a split AI/ML operation.

Pre-conditions:

- A split AIML pipeline is created, and the UE and VAL server has an ongoing connection session for undertaking jointly the split learning operation.

- One or more VAL UEs or edge nodes (e.g. EAS) have registered their capabilities in the ML repository / AIMLE server.

Editor’s Note: It is FFS how one or more VAL UEs register their capabilities as candidate intermediate nodes for the split learning operation.

Figure 7.x.1-1: Support for split relay operation relaying

- A VAL UE detects that a ML model performance is degraded or expected to degrade. The detection can have different flavours either based on UE mobility and channel conditions or due to load/energy status and degradation of the ongoing ML task. More specifically, the detections can be due to:

- VAL client notifies the AIMLE client that the task is delaying or expected to delay.

- AIMLE client utilizes ADAE analytics on VAL session performance (based on TS 23.436) and predicts the downgrade of the link between the VAL UE and the VAL server

- High load or energy consumption at the VAL UE side, due to high app load or lower layer processing

- UE mobility prediction to an area with no coverage or poor coverage or bad channel conditions.

- The AIMLE client determines a requirement for updating the pipeline to include a relay/intermediate node and sends a discovery request to the AIMLE server.

The discovery request includes discovery criteria for the selection of the relay, including capability of the VAL UE (apps onboard, workloads supported, processing power and capacity, energy consumption limitations/restrictions), the status of the radio capability of the UE (RATs/spectrum supported, channel conditions, interference limitations), whether the UE is static/low or high mobile UE, whether it supports dual-connectivity (or multi-connectivity), whether it can be connected to the server via non-3GPP means (Wifi, NTN,..), the mobility of the UE or relative mobility to the UE of interest (source UE), etc.

The requirement can include also requested topology/option for the split model operation for which the relay/intermediate nodes need to be discovered. Alternatively, the model type may be provided in the request if there is an association between the type and the topology at the server side (as pre-configuration).

NOTE: Split model topology refers to how a model is divided and distributed across different devices or computational units for training or inference. These topologies can be broadly categorized into data parallelism, model parallelism, and hybrid approaches.

- The AIMLE server requests from ML repository and receives the registered nodes (available AIMLE clients, EASs to serve as candidate relays) with their roles as requested at the discovery criteria.

- The AIMLE server sends a discovery response to the AIMLE client including the discovered nodes and their capabilities. Such response includes the split learning operation related information such as VAL UE capability and availability information for acting as intermediate / relay node for the Split Operation and the type of relay, the supported split model topologies, VAL policies on the split operation etc.

- The AIMLE client after receiving the list of discovered AIMLE clients, it checks whether the other VAL UEs in the list are active and in close proximity. Hence it sends a groupcast/broadcast or unicast message via AIMLE-PC5 to all listed AIMLE clients, to receive reporting on the channel conditions and availability related to the sidelink radio conditions. The other AIMLE clients may response if available and the source AIMLE client will establish a connection/session with them if they fulfil the KPI (e.g. latency for the handshake is less than a pre-defined threshold).

The sidelink status check may also be in form of requesting from the target UEs additional /supplementary information on the channel status, load, energy and mobility information locally. The status check can be also using ADAE analytics for VAL UE-to-UE session performance stats/predictions as specified in TS 23.436.

- The AIMLE client of the VAL UE of interest (source UE) selects one of the candidate intermediate/relay nodes (edge or UE based relays) given criteria which can be equivalent to the discovery criteria and the performance KPI for the split learning operation. This may be based on local or VAL UE or server policies on the selection, e.g. to select UEs to meet KPIs with higher energy /load remaining capacity or prefer static UEs or edge nodes vs UEs.

Then the AIMLE client sends the selected intermediate node (AIMLE client or EAS) to the AIMLE server, including the identifier of the selected node and the time to activate (time of validity) for the addition to the pipeline, as well as to a pipeline ID (of the pipeline to be modified).

- AIMLE server associates the selected intermediate node in the AIML pipeline. This can be performed in different ways based on whether the intermediate node is a UE or an edge node:

- If the relay is a UE (AIMLE or VAL client), the AIML pipeline update adds the relay UE as another entity in the existing pipeline with the associated flag as relay/intermediate and the type of relay and the layers that it is going to process (or this can be an indication of the type of processing that the relay will undertake if this is pre-configured, e.g. “relay_split_X corresponds to relay node handling layers 5-15”)

- If the relay is an edge node (EAS/edge AIMLE) then the AIMLE pipeline update add the edge node as another entity in the pipeline or creates two further pipelines (VAL UE-edge node, edge node to VAL server) which are coupled/associated with the first pipeline for the give AIML task/ split learning operation.

- After step 7 (or this can be before previous steps given the scenario) the AIMLE server notifies the VAL server on the addition of the intermediate / relay node to the pipeline (or to the creation of dependent/partner pipelines to the existing pipeline).

### 7.x.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.x.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.x.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *