3GPP TSG-SA WG6 Meeting #68	S6-253411

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253025)

Source:	Netherlands Police

Title:	Pseudo-CR on Scenario for Discreet monitoring of remotely initiated MCVideo push

Spec:	3GPP TR 23700-37-020

Agenda item:	9.1

Document for:	Approval

Contact:	keesdotverweijatpolitiedotnl

1. Introduction

A scenario detailing remotely initiated MCVideo push is added to clarify the usecase and identify technical impact. The text originates from TR 23.784, but has been enhanced as required.

2. Reason for Change

Scenario for remotely initiated MCVideo push is missing.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23700-37-020 v 0.2.0.

* * * First Change * * * *

## 4.6	Scenario 5: Discreet monitoring of remotely initiated MCVideo push

This scenario describes the case where authorized MC user A requests discreet monitoring for communications involving MC user B where MC user B is within the authority of authorized MC user A, and where MC user C causes a remotely initiated MCVideo push from target MC user B to MC user D. The scenario is illustrated in figure 4.6-1 below.

Figure 4.6-1: Discreet monitoring of remotely initiated MCVideo push

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. MC system A verifies that MC user A is authorized to perform discreet monitoring on MC user B. This aspect takes place before any communications involving MC user B can be subject to discreet monitoring.

-	MC user C initiates a remotely initiated MCVideo push procedure, wherein target MC user B pushes video to MC user D.

-	MC system A provides MC service client A with information concerning the initiation of the MCVideo push including the identities of MC users C and D, and provides the MCVideo media sent from target MC user B to MC user D.

NOTE 1:	Authorized MC user A may receive separate notifications when the MCVideo push is initiated, and when the MCVideo is provided.

NOTE 2:	The provision of MCVideo media to authorized MC user A may be in real time, or on demand with intermediate storage in the MCVideo server. This aspect may need an additional requirement on media storage server.

NOTE 3:	The scenario is the same where MC user C causes MC user D to push MCVideo media towards target MC user B; MC system A also sends the pushed video from MC user D to authorized MC user A.

NOTE 4:	The scenario is the same where MC user C remotely initiates a private call between target MC user B and MC user D.

* * * End of Changes * * * *