3GPP TSG-SA WG6 Meeting #68	S6-253653

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253058)

Source:	InterDigital, Samsung

Title:	New solution on roaming

Spec:	3GPP TR 23.700-83 v0.2.0

Agenda item:	9.4

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This pCR proposes a new solution for KI#3 on usage of AIML enablement services when roaming.

2. Reason for Change

This contribution proposes a solution to Key Issue #3 on support for cross-PLMN and roaming scenarios.

The solution proposes enhancements to the AIMLE client registration procedure for enabling support of AIMLE services for roaming UE use cases.

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0.

* * * First Change * * * *

## 7.x	Solution #x: Support of AIMLE services for roaming UEs

### 7.x.1	Solution description

#### 7.x.1.1 Introduction

This solution proposes enhancements to address Key Issue #3 – Open Issue #2 on support of AIMLE services for roaming UEs.

This solution proposes enhancing AIMLE client registration in roaming scenarios by enabling the H-AIMLE server to indicate to a registering AIMLE client whether it is allowed to use AIMLE services while in a visited network.

The solution assumes the following roaming models:

- Home Routed (HR) roaming architecture

- Local Breakout (LBO) roaming architecture

In the HR roaming architecture, the traffic of the AIMLE client is routed to the H-AIMLE server where the UE obtains AIMLE services from the home network.

In the LBO roaming architecture, the traffic of the AIMLE client is routed to the H-AIMLE Server or to the V-AIMLE Server dependent on the AIMLE server endpoint used by the AIMLE client. The AIMLE client is provided with V-AIMLE server information when registering with the H-AIMLE server as described in this solution.

#### 7.x.1.2 Impact to existing AIMLE client registration

The AIMLE client registration procedures and information flows in 3GPP TS 23.482 are enhanced (highlighted in bold italics) as follows.

# 2 References The following documents contain provisions which, through reference in this text, constitute provisions of the present document. - References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific. - For a specific reference, subsequent revisions do not apply. - For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document. [1] 3GPP TR 21.905: "Vocabulary for 3GPP Specifications". [2] 3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services". [3] 3GPP TS 28.104: "Management and orchestration; Management Data Analytics". [4] 3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service". [5] 3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows". [6] 3GPP TS 23.501: "System Architecture for the 5G System; Stage 2". [7] 3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access". [8] 3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management". [9] 3GPP TS 23.502: "Procedures for the 5G System (5GS)". [10] 3GPP TS 22.261: "Service requirements for the 5G system". [11] 3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture". [12] 3GPP TS 23.558: "Architecture for enabling Edge Applications". [13] 3GPP TS 23.273:"5G System (5GS) Location Services (LCS); Stage 2". [14] 3GPP TS 33.434: "Security aspects of Service Enabler Architecture Layer (SEAL) for verticals". [15] 3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

## 8.7 AIMLE client registration ### 8.7.1 General Prior to participation in AI/ML operations, AI/ML capable UEs register to an AIMLE server and provide an AIMLE client profile and optionally a list of supported services. The AIMLE server uses information provided by the AIMLE client to discover and select suitable AIMLE clients for requested AI/ML operations. When a UE is roaming and the roaming architecture is home routed (HR) (e.g., based on roaming agreement), the UE registers with an AIMLE server in the home network and AIMLE services provided by the visited network are not used. When a UE is roaming and the roaming architecture is local breakout (LBO) (e.g., based on roaming agreement), the UE first registers with an AIMLE server in the home network, then the UE may perform an AIMLE server registration with an AIMLE server in the visited network if AIMLE services usage is allowed in the visited network. NOTE: Usage of AIMLE service provided by a visited network is dependent on a roaming agreement. The following clauses specify procedures, information flows, and APIs for AIMLE client registration. ### 8.7.2 Procedures #### 8.7.2.1 General The following are supported for AIMLE client registration: - AIMLE client registration procedure; - AIMLE client registration update procedure; and - AIMLE client de-registration procedure #### 8.7.2.2 AIMLE client registration Pre-conditions: 1. The AIMLE client has been pre-configured or has discovered the address (e.g., URI) of the AIMLE server. 2. The AIMLE client has been pre-configured with an AIMLE client profile. Figure 8.7.2.2-1: AIMLE client registration 1. The AIMLE client sends an AIMLE client registration request to the AIMLE server, the registration request includes information as described in Table 8.7.3.2-1. The AIMLE client indicates in the registration request its AI/ML capabilities such as supported ML model types and supported AI/ML operations, supported AIMLE client task capabilities with compute and task performance capabilities to assist with performing AIMLE client discovery and AIMLE client selection. If the UE is roaming, the registration request includes the serving PLMN identifier. 2. The AIMLE server validates the registration request and performs an authentication and authorization check to determine if the AIMLE client is permitted to register to the AIMLE server and participate in AI/ML operations. Upon successful authorization, the AIMLE server saves the context of the AIMLE client registration in the ML repository. When the PLMN identifier is provided, the AIMLE server may verify the serving PLMN information from the AIMLE client, using the 3GPP core network services (for example, NEF monitoring event API as described in 3GPP TS 23.502 [9] and 3GPP TS 23.682 [15]), to obtain the UE roaming status and serving PLMN identifier. Further verify and confirm the serving PLMN information of the AIMLE client. If the UE is roaming, the AIMLE server determines if the registering AIMLE client is allowed to consume AIMLE services for the PLMN identifier and determines allowed AIMLE service operations. 3. The AIMLE server returns an AIMLE client registration response to the AIMLE client with the status of the request. If the AIMLE server has determined that the UE is allowed to consume AIMLE services with the PLMN identifier, the response includes the V-AIMLE server connectivity information and a list of allowed AIML service operations. If the UE is roaming and the response includes V-AIMLE server connectivity information, the AIMLE client registers with the V-AIMLE server. If the AIMLE client has successfully registered with the V-AIMLE server, the AIMLE client sends a registration update to the AIMLE server in the home network to indicate that it is consuming AIMLE services in the visited network. #### 8.7.2.3 AIMLE client registration update Pre-conditions: 1. The AIMLE client has already registered with the AIMLE Server. Figure 8.7.2.3-1: AIMLE client registration update 1. The AIMLE client sends an AIMLE client registration update request to the AIMLE server, the registration update shall include registration identifier, updated list of supported services and updated AIMLE client profile as described in Table 8.7.3.4-1. The AIML client removes services from the list of services identified by Service ID(s) when it ceases to provide a service and adds to the service when it starts to provide a service. If the UE roaming network has changed, the registration request includes the updated serving PLMN identifier. 2. The AIMLE server validates the registration update and performs an authentication and authorization check. Upon successful authorization, the AIMLE server updates the context of the AIMLE client registration in the ML repository. When the PLMN identifier is updated, the AIMLE server may verify the serving PLMN information from the AIMLE client, using the 3GPP core network services (for example, NEF monitoring event API as described in 3GPP TS 23.502 [9] and 3GPP TS 23.682 [15]), to obtain the UE roaming status and serving PLMN identifier. Further verify and confirm the serving PLMN information of the AIMLE client. If the UE roaming network has changed, the AIMLE server determines if the AIMLE client is allowed to consume AIMLE services for the updated PLMN identifier and determines allowed AIMLE service operations. 3. The AIMLE server returns an AIMLE client registration update response to the AIMLE client with the status of the update. If the AIMLE server has determined that the UE is allowed to consume AIMLE services with the updated PLMN identifier, the response includes the V-AIMLE server connectivity information and a list of allowed operations with the V-AIMLE server. If the UE is roaming and the response includes V-AIMLE server connectivity information, the AIMLE client registers with the V-AIMLE server. If the AIMLE client has successfully registered with the V-AIMLE server, the AIMLE client sends a registration update to the AIMLE server in the home network to indicate that it is consuming AIMLE services in the updated visited network. #### 8.7.2.4 AIMLE client de-registration Pre-conditions: 1. The AIMLE client has already registered with the AIMLE Server. Figure 8.7.2.4-1: AIMLE client de-registration 1. The AIMLE client sends an AIMLE client de-registration request to the AIMLE server. The request shall include registration identifier as described in Table 8.7.3.6-1. 2. The AIMLE server validates the deregistration request and performs an authentication and authorization check to determine whether the AIMLE client can deregister from AI/ML operations. Upon successful authorization, the AIMLE server removes the context of the AIMLE client registration from the ML repository. 3. The AIMLE server returns an AIMLE client de-registration response to the AIMLE client with the status of the request. If the UE is roaming and has previously registered with a V-AIMLE server, the AIMLE client de-registers with the V-AIMLE server. ### 8.7.3 Information flows #### 8.7.3.1 General The following information flows are specified for AIMLE client registration: - AIMLE client registration request and response; - AIMLE client registration update request and response; and - AIMLE client de-registration request and response #### 8.7.3.2 AIMLE client registration request Table 8.7.3.2-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration request. Table 8.7.3.2-1: AIMLE client registration request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. List of supported profiles; M; Supported AIML client profile(s). For each client profile provided in the list, the supported service information is provided.. > AIMLE client profile; M; Information about the capability of the AIMLE client to support AI/ML operations for the VAL service ID as detailed in Table 8.7.3.2-2.. > List of supported services; M; List of VAL service IDs with corresponding permissions.. > VAL service ID; M; The identifier of the VAL service.. > Service permission level; O; Service permission level (e.g., premium resource usage, standard resource usage, limited resource usage).. > PLMN Identifier; O; The identifier of the PLMN where the UE is currently registered.. Table 8.7.3.2-2: AIMLE client profile Information element; Status; Description. Supported AI/ML model types; O; AI/ML model types supported by the AIMLE client (e.g., decision trees, linear regression, neural networks).. Supported AI/ML operations; M; AI/ML operations supported by the AIMLE client such as training, model transfer, model inference, model offload, model split, continue perform intermediate AI/ML operation/task).. AIMLE client time schedule configurations; O; Indicates the availability schedule of the AIMLE client for the AIML service, e.g., the AIMLE client is (not) available to participate in the AIML operations (e.g. ML model training) in the given time slot(s) and/or day(s) of the week.. AIMLE client location configurations; O; Indicates the location-based configurations of the AIMLE client for the AIML service, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID.. AIMLE client capabilities; M; AIMLE client capability information (e.g. ML application type, allowed resource usage level, supported roaming and roaming service operations).. Dataset availability; O; Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers.. Data capabilities; O; A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis functions.. AIMLE client task capability; O See NOTE; Indicates the AIML task performing capabilities. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g., Green task, energy-efficient, low costs). NOTE: The Green and Energy-efficient task performance may not be applicable to a UE.. #### 8.7.3.3 AIMLE client registration response Table 8.7.3.3-1 shows the AIMLE client registration response sent by the AIMLE server to the AIMLE client. Table 8.7.3.3-1: AIMLE client registration response Information element; Status; Description. Successful response; O (NOTE); Indicates that the registration was successful.. > Registration ID; M; The identifier of the registration.. > Expiration time; O; Indicates the expiration time of the updated registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the updated registration never expires.. > Roaming configuration; O; The configuration for using AIMLE services in the visited network. >> Roaming connectivity information; O; The connectivity information to a V-AIMLE server.. >> Allowed roaming operations; O; The list of allowed operations with the V-AIMLE server. Failure response; O (NOTE); Indicates that the registration request failed.. > Cause; O; Indicates the cause of registration request failure. NOTE: One of the IEs shall be present.. #### 8.7.3.4 AIMLE client registration update request Table 8.7.3.4-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration update request. Table 8.7.3.4-1: AIMLE client registration update request Information element; Status; Description. Registration identifier; M; Identifier of the existing registration for which the update request applies.. AIMLE client profile; O; Update information about the capability of the AIMLE client to support AI/ML operations as detailed in Table 8.7.3.2-2.. List of supported services; O; Update to supported service information. Services identified by their VAL Service ID are either removed from the list when the AIMLE client ceases to provide a service and added when the AIMLE client starts to provide a service.. > VAL service ID; M; The identifier of the VAL service.. > Service permission level; O; Service permission level (e.g., premium resource usage, standard resource usage, limited resource usage).. > PLMN Identifier; O; The identifier of the PLMN where the UE is currently registered.. > Roaming AIMLE service indication; O; The indication that the AIMLE client is registered with the AIMLE server in the visited network (e.g., the V-AIMLE server indicated by the H-AIMLE server).. #### 8.7.3.5 AIMLE client registration update response Table 8.7.3.5-1 shows the AIMLE client de-registration update response sent by the AIMLE server to the AIMLE client. Table 8.7.3.5-1: AIMLE client registration update response Information element; Status; Description. Successful response; O (NOTE); Indicates that the registration update request was successful.. > Expiration time; O; Indicates the expiration time of the updated registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the updated registration never expires.. > Roaming configuration; O; The configuration for using AIMLE services in the visited network. >> Roaming connectivity information; O; The connectivity information to a V-AIMLE server.. >> Allowed roaming operations; O; The list of allowed operations with the V-AIMLE server. Failure response; O (NOTE); Indicates that the registration update request failed.. > Cause; O; Indicates the cause of registration update request failure. NOTE: One of the IEs shall be present.. #### 8.7.3.6 AIMLE client de-registration request Table 8.7.3.6-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client de-registration request. Table 8.7.3.6-1: AIMLE client de-registration request Information element; Status; Description. Registration ID; M; Identifier of the existing registration for which the update request applies.. #### 8.7.3.7 AIMLE client de-registration response Table 8.7.3.7-1 shows the AIMLE client de-registration response sent by the AIMLE server to the AIMLE client. Table 8.7.3.7-1: AIMLE client de-registration response Information element; Status; Description. Successful response; O (NOTE); Indicates that the de-registration request was successful.. Failure response; O (NOTE); Indicates that the de-registration request failed.. > Cause; O; Indicates the cause of de-registration request failure. NOTE: One of the IEs shall be present..

### 7.X.2	Architecture Impacts

No architecture impact has been identified.

### 7.X.3	Corresponding APIs

API updates have been described in clause 7.x.1.2.

### 7.X.4	Solution evaluation

Editor's Note:		The solution evaluation is FFS.

* * * End of Changes * * * *