3GPP TSG-SA WG6 Meeting #68	S6-253676

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253062)

Source:	InterDigital

Title:	Pseudo-CR on hierarchical ML model training

Spec:	3GPP TR 23.700-83 v0.2.0

Agenda item:	9.4

Document for:	Approval

Contact:	Quang Ly <Quang.Ly@InterDigital.com>

1. Introduction

This contribution proposes the addition of hierarchical ML model training procedure as a solution to Key Issue #6 to support ML model training in hierarchical AIMLE deployments in edge data networks.

2. Reason for Change

ML model training procedure is defined in clause 8.3 of 3GPP TS 23.482, where an AIMLE server supports ML model training based on a request from a VAL server. In a hierarchical AIMLE deployment scenario (such as described in Annex A.4 of 3GPP TS 23.482), ML model training may require members/clients across multiple EDN service areas to complete. The members/clients in different EDN service areas may be managed by corresponding edge deployed AIMLE servers, rather than directly managed by the central AIMLE server. As a result, the central AIMLE server needs to distribute the ML model training request to the edge AIMLE servers in different EDN service areas. Therefore, additional interactions between the edge deployed AIMLE servers and the central AIMLE server need to be defined to support hierarchical ML model training.

This contribution proposes a solution to Key Issue #6: support for architecture and functions enhancement on diverse AIMLE deployments scenarios. Particularly, a hierarchical ML model training procedure is proposed to support hierarchical AIMLE deployment in edge data networks as indicated in Annex A.4 of 3GPP TS 23.482.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0.

* * * First Change * * * *

## 7.x	Solution #x: ML model training in a hierarchical AIMLE deployment

### 7.x.1	Solution description

This solution is related to Key Issue #6: support for architecture and functions enhancement on diverse AIMLE deployments scenarios.

The solution proposes ML model training procedures considering AIMLE deployments in edge data networks as described in Annex A.4 of 3GPP TS 23.482 [2]. This solution makes no assumptions on the cardinality of ML repository(ies) deployed at the EDN or at the centralized DN.

ML model training procedure is described in Clause 8.3 of 3GPP TS 23.482 [2]. In a hierarchical AIMLE deployment scenario, a VAL server requests the central AIMLE server for ML model training. Since the member clients that will participate in the training process may be distributed in different EDN service areas, the central AIMLE server can distribute the training request to the corresponding edge AIMLE servers.

Figure 7.x.1-1 describes a procedure for central and edge AIMLE servers to support ML model training based on the request from a VAL server in a hierarchical AIMLE deployment.

Pre-conditions:

1.	AIMLE clients have registered with the corresponding edge deployed AIMLE servers as described in clause 8.7.2.2 of 3GPP TS 23.482 [2]. The edge deployed AIMLE servers receive AIMLE client profiles associated with each registered AIMLE client in the EDN service area.

2.	The edge deployed AIMLE servers have registered with the central AIMLE server as described in clause 7.9 (Solution #9).

Figure 7.x.1-1: ML model training in a hierarchical AIMLE deployment

1.	A VAL server sends an ML model training request to a central AIMLE server, as described in step 1 of Figure 8.3.2-1 in TS 23.482. The request includes information described in Table 8.3.3.1-1 in TS 23.482 [2].

2.	The central AIMLE server checks whether the VAL server is authorized to perform the ML model training request, as described in step 2 of Figure 8.3.2-1 in TS 23.482 [2]. The central AIMLE server uses information from the AIMLE server registration procedure described in clause 7.9 to identify edge AIMLE servers that can fulfil the ML model training requirements.

3.	Based on the ML model training request and the identified edge AIMLE servers, the central AIMLE server determines to distribute the training request to the identified edge AIMLE servers. The central AIMLE server identifies the EDN service areas based on information in the ML model training request (e.g. training type, list of member clients, member selection criteria) and the AIMLE client profiles (e.g. supported AI/ML operations, AIMLE client capabilities) received from the AIMLE server registrations. For example, the central AIMLE server determines the distribution of the request based on the required number of clients in the request and the number of available clients in each EDN service area or based on the dataset identifiers in the request and the dataset availability of clients in each EDN service area. The central AIMLE server sends an Edge ML model training request to the edge AIMLE server in each of the identified EDN service areas.

4.	The edge AIMLE server sends an Edge ML model training response to the central AIMLE server.

5.	The central AIMLE server returns the success or failure response, as described in step 3 of Figure 8.3.2-1 in TS 23.482 [2]. The response includes information described in Table 8.3.3.2-1 in TS 23.482 [2].

6. 	The edge AIMLE server notifies the central AIMLE server of the progress of the ML model training, including the list of member clients selected or de-selected for the ML model training, the training output, or errors encountered during the training process.

7.	The central AIMLE server aggregates the notifications received from the edge AIMLE servers and sends the aggregated notification to the VAL server. The notification includes information described in Table 8.3.3.3-1 in TS 23.482 [2].

### 7.x.2	Architecture impacts

No architectural impact has been identified.

### 7.x.3	Corresponding APIs

#### 7.x.3.1	Edge ML model training subscription request

Table 7.x.3.1-1 describes the information flow from the central AIMLE server to the edge AIMLE server for subscribing to ML model training in the EDN service area.

Table 7.x.3.1-1: Edge ML model training subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the central AIMLE server of the edge ML training request. |
| Original requestor identity | O | Identity of the VAL server of the original ML training request. |
| Edge training identifier | M | Identifier to associate with the original ML training request (from the VAL server to the central AIMLE server). |
| Training type | M | Identifies whether the VFL or HFL training to be performed. |
| List of member clients | O (NOTE) | List of member clients in the EDN service area to be utilized for training the ML model. |
| Member selection criteria | O (NOTE) | Identifies the criteria that needs to be continuously monitored for selecting the member clients (e.g., AIMLE clients in a particular location, member availability duration, new member clients registering with the required capabilities). |
| ML model information | M | Identifies the ML model (specified in the original request or selected by the central AIMLE server) that has to be distributed to the selected member clients in the EDN service area for training. The information is described in Table-8.11.4.1 of TS 23.482 [2]. |
| Operational requirements | O | Requirements for the training operations, e.g. latency, QoS requirements, etc. |
| Operational schedule | O | A schedule for when training is to occur. |
| Reporting requirements | O | Indicates what information regarding the training operations needs to be sent to the central AIMLE server by the edge AIMLE server (e.g. update related to new member clients selected or de-selected). The reporting requirements includes a specific schedule for sending notifications. |
| NOTE: At least one of these IEs shall be present. |  |  |

#### 7.x.3.2	Edge ML model training subscription response

Table 7.x.3.2-1 describes the information flow of Edge ML model training subscription response from the edge AIMLE server to the central AIMLE server.

Table 7.x.3.2-1: Edge ML model training subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE 1) | Indicates that the Edge ML model training request was successful. |
| > Edge training identifier | M | Identifier to associate with the original ML training request (from the VAL server to the central AIMLE server). |
| > EDN service area identifier | M | The identifier of the EDN service area associated with the edge AIMLE server. |
| Failure response | O (NOTE 1) | Indicates that the Edge ML model training request was failure. |
| > Cause | M | Reason for the failure. |
| NOTE: Only one of these information elements shall be present. |  |  |

#### 7.x.3.3	Edge ML model training notification

Table 7.x.3.3-1 describes the information flow of Edge ML model training notification from the edge AIMLE server to the central AIMLE server.

Table 7.x.3.3-1: Edge ML model training notification

| Information element | Status | Description |
| --- | --- | --- |
| Edge training identifier | M | Identifier to associate with the original ML training request (from the VAL server to the central AIMLE server). |
| EDN service area identifier | M | The identifier of the EDN service area associated with the registering AIMLE server. |
| EDN service area information | O | The information of the EDN service area (e.g., location, area, edge load of the area, etc.) associated with the edge AIMLE server. |
| Percentage completion | M | Indicates a completion percentage for the training. |
| List of AIMLE clients | O (NOTE) | Indicates the list of AIMLE clients selected or de-selected for the ML model training in the EDN service area. |
| Training output | O (NOTE) | Output of training, e.g., ML model parameters for the training. |
| Errors list | O (NOTE) | A list of errors, if any, encountered during training process. |
| NOTE: At least one of these IEs shall be present. |  |  |

### 7.x.4	Solution evaluation

Editor's Note:		The solution evaluation is FFS.

* * * End of Change * * * *