3GPP TSG-SA WG6 Meeting #68	Revised to S6-253744

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253397)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on Solution evaluation forTechnical solutions #7

Spec:	3GPP TR 23.700-35 v1.0.0

Agenda item:	9.3

Document for:	Approval

Contact:	Yang Yanmei yangyanmei@huawei.com

1. Introduction

This contribution is to propose Solution evaluation for Technical solutions #7 and make some changes on this solution.

2. Reason for Change

The changes to the solution cover the following aspects:

- Remove unclear, but unnessary description text.

- Correct VAL UE IP address retrival mechnism

- Add parameters which is important for multi-modal applications,

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35 v1.0.0.

* * * First Change * * * *

8.1.7	Solution#7: Enhancements of NRM network resource adaptation service

8.1.7.1	Solution Description

It is important for the NRM services to provide sufficient input parameters to support latest network capabilities such as multi-modal application traffic. The existing NRM network resource adaptation service can be enhanced for multi-modal traffic by providing additional input parameters that enables the application client/server to request network resource adaptation by specifying the traffic type along with the priority of the traffics of the application and the QoS requirements for each traffic type of the application.

The application server sends the network resource adaptation request with resource adaptation requirements to the NRM server. The resource adaptation requirements are with input parameters specifying the traffic type(s) e.g., audio, video, sensor data along with the priority of the traffic flows.

In some cases, the application may need to indicate the priority of one traffic flow compared to other proposed traffic flows.

The enhanced NRM network resource adaptation service for multi-modal traffic allows the application client/server to add the QoS requirements for each traffic types separately. For example, consider the above mentioned gaming application, QoS requirements such as high reliablility and low latency can be requested for the "video" traffic type and high data rate and low latency can be requested for the "haptic" traffic type.

The enhanced NRM network resource adaptation service for multi-modal traffic can send traffic type specific QoS requirements to the 5GC and can be mapped with the AFSessionWithQoS for multi-modal service. The multi-modal service requirements information for each data flow can be mapped with the QoS requirements of different traffic types.

The proposed input parameters to the enhanced NRM network resource adaptation service for multi-modal traffic makes it suitable for scenarios with single or multiple traffic types, irrespective of the application type.

The following clauses specify the procedures and information flows for gaps identified in clause 4.1 to support the enhancements of the NRM network resource adaptation service for multi-modal traffic.

8.1.7.2.1	Impact to existing NRM information flows

The information flows of NRM network resource adaptation service in 3GPP TS 23.434 [3] can be enhanced (highlighted in bold italics) as follows.

14.3.3.3.1 Network resource adaptation 14.3.3.3.1.1 General This subclause describes the procedure for network resource adaptation using PCC procedures. This procedure satisfies the requirements for requesting unicast resources and modification to already allocated unicast resources to VAL communications. 14.3.3.3.1.2 Procedure Figure 14.3.3.3.1.2-1 illustrates the procedure for the network resource adaptation. Figure 14.3.3.3.1.2-1: Network resource adaptation - The VAL server (or SEALDD server) sends a network resource adaptation request to the NRM server for one or more users belonging to one or more VAL services, and may comprise one or more VAL UEs that will have updated resource requirement. This requirement may be in the form of exact resources /resource pools to be used or indication of bandwidth increase/decrease for the corresponding VAL UEs or set of VAL UEs. The BDT Reference ID may also be included for setting BDT policies. The resource adaptation requirement(s) IE in the request may include traffic typeto indicate the different types of traffic flows involved (for e.g. audio, video, haptic), priority of the traffic types (the VAL server can set the priority of the traffic types involved using the priority IE), QoS requirement per traffic type. NOTE: The SEALDD server can use the NRM netwok resource adaption service as per the procedure defined in 3GPP TS 23.433 clause 9.11 SEALDD Background data transfer. 2. The NRM server processes the request and applies / enforces the resource adaptation per VAL UE. The NRM server retrieves UE IP address by using event monitoring capability for PDU session status (or PDN connectivity status) and initiates the PCC procedures for the traffic flows associated to each VAL UE Editor NOTE 1: It is FFS if UE IP address can be sent from VAL server. Editor NOTE 2: Whether and how multi-modal service ID needs to be get by NRM server, and used in this step. 4. The NRM server provides a network resource adaptation response to the VAL server, providing information on the fulfilment of the network resource adaptation request. This will include information either per VAL UE or per set of VAL UEs, as indicated by the request of the VAL server in step 1. If the QoS requirement for traffic type(s) cannot be satisfied, the cause of failure may be included in the response. 14.3.2 Information flows 14.3.2.1 Network resource adaptation request Table 14.3.2.1-1 describes the information flow network resource adaptation request from the VAL/SEALDD server to the NRM server. Table 14.3.2.1-1: Network resource adaptation request Information element; Status; Description. Requester Identity; M; The identity of the VAL server performing the request.. List of VAL UE IDs; O (see NOTE 1); List consisting of one or more VAL UE IDs for whom the network resource adaptation occurs.. VAL group ID; O (see NOTE 1); The VAL group ID for whom the network resource adaptation occurs.. Resource adaptation requirement(s); M; The resource adaptation requirement corresponds to the VAL service QoS requirements as applied for a UE or group of UEs (E.g. bandwidth, resource).. >Traffic type; O; The type of the traffic (e.g, audio, video, haptics etc).. >Priority; O (see NOTE 2); Indicates the priority of the traffic type.. >QoS requirement; M; The QoS requirement corresponds to the VAL service QoS requirement as per the traffic type (e.g, data rate, GBR, MBR, latency, reliability, bandwidth, etc., and/or its combinations).. BDT Reference ID; O; Indicates the Background data transfer Reference ID provided by the 3GPP network. NOTE 1: Either of the information elements should be present. NOTE 2: Priority may be present only when the Traffic type is present.. 14.3.2.2 Network resource adaptation response Table 14.3.2.2-1 describes the information flow network resource adaptation response from the NRM server to the VAL server. Table 14.3.2.2-1: Network resource adaptation response Information element; Status; Description. Result; M; Result includes success or failure of the network resource adaptation with the underlying network. The response can also include an updated value for some of the parameters included in the network resource adaptation request (e.g. negotiation of resource offering). Cause; O (see NOTE); Cause of failure (e.g. QoS requirement for traffic type cannot be satisfied).. NOTE: The information element shall be present only when the result is failure..

8.1.7.2	Solution evaluation

Editor's Note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

This solution provide mechanism to enable NRM to support network resource adaptation service for multi-modal applications. There are two some open issues for UE IP address and multi-modal service ID in solution.

It has no architectrure impact.

* * * Next Change * * * *