3GPP TSG-SA WG6 Meeting #69	S6-254655

Wuhan, China 13th – 17th October 2025	(revision of S6-254119)

Source:	Telefónica, Apple, Fogus, UMA

Title:	TR 23.947 Overview of Frameworks EDGEAPP

Spec:	3GPP TR 23.947

Agenda item:	9.14

Document for:	Approval

Contact:	w_featherstone@apple.com

1. Introduction

This contribution provides the Overview of Frameworks EDGEAPP text for the Guidelines for 3GPP Application Enablement usage (AppEnable_EXT) technical report.

2. Reason for Change

This contribution proposes text for the Overview of Frameworks EDGEAPP of 3GPP TR 23.947.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.947, version 0.1.0.

* * * First Change * * * *

## 4.3	Edge Application Enablement (EDGEAPP)

Edge Application Enablement (EDGEAPP) has been specified by 3GPP to enable applications to be deployed and managed at the edge of the network (closer to end users) to achieve ultra-low latency and high bandwidth efficiency, 3GPP TS 23.558 [4]. By offloading computation and services from centralized cloud to edge nodes (e.g. servers co-located with base stations or local data centers), EDGEAPP enables use cases like real-time video analytics, AR/VR, V2X, and industrial automation that require immediate processing and minimal backhaul delay. The key goal of EDGEAPP is to make this edge deployment transparent to application developers – applications should not need major redesign to benefit from edge computing. The framework accomplishes this by providing a standardized set of functions that handle discovery of edge services, provisioning and relocation of application instances, and continuity of sessions as users move between edge sites. In essence, EDGEAPP extends the 3GPP system with an application-layer control plane for edge computing, while hiding network complexity from the application itself.

The EDGEAPP architecture introduces several new functional entities (as depicted in Figure 4.3-1), which can be grouped into client-side and network-side components:

-	Application Client (AC): The end-user application component that consumes a service. In an edge computing scenario, this could be, for example, a video streaming app, a game client, or an IoT application running on a UE (User Equipment). The AC is typically developed by the Application Service Provider and contains the core application logic for the end user. Importantly, the AC itself does not directly interface with the edge network functions; instead, it interacts with the Edge Enabler Client via local APIs on the device. This design keeps the AC simple and focused on the service logic, while offloading networking tasks to the Edge Enabler Client.

-	Edge Enabler Client (EEC): A client-side middleware function residing on the UE (or whatever device hosts the AC). The EEC is the bridge between the application client and the edge system. It exposes a set of APIs (referred to as EDGE-5 interfaces in 3GPP) to the AC, through which the AC can request edge services (such as registering to the edge system, discovering available edge application servers, etc.). Internally, the EEC communicates with the network’s edge enablement functions: it registers the device/application with an Edge Enabler Server, keeps track of the current “serving” edge zone, and orchestrates any changes needed when the user moves. From the developer’s perspective, the EEC is crucial – by calling the EEC’s APIs, the developer’s application can utilize edge capabilities without needing to implement complex protocols. The EEC also handles obtaining any needed network authorization (for example, via CAPIF tokens when calling edge services) on behalf of the AC.

-	Edge Enabler Server (EES): A network-side function that is typically deployed at an edge data center or site serving a particular geographical zone. The EES is the core control-plane entity in the EDGEAPP framework. It manages edge application services within its zone and interfaces with EECs on UEs. Key responsibilities of the EES include registering new Edge Application Servers (EAS) when they become available in its zone, handling UE registrations (via EECs) so that the system knows which UEs are using edge services in that area and facilitating service discovery for UEs (telling an EEC which local EAS can provide the service the AC needs). The EES also monitors UE mobility (through integration with the cellular network or via the EEC’s updates) – when a UE is about to move or has moved to another zone, the EES can trigger Application Context Relocation (ACR) procedures to ensure continuity of the service session. In summary, the EES is analogous to a local edge broker/manager that coordinates between UEs and application servers at the edge.

-	Edge Application Server (EAS): The actual application server instance that runs on the edge node, providing the service logic to be consumed by the AC. This could be, for example, an instance of a video analytics engine, a game server, or a data caching server, deployed in the edge cloud. The EAS is owned by the application provider (ASP) and hosts the service that the AC uses. Each EAS registers with the local EES so that the system knows the service it offers and can advertise it to interested clients. The EAS relies on the EES for certain lifecycle operations; for instance, the EES can instruct the EAS to transfer state or shut down when a user leaves the zone. The EAS can also consume network and edge services itself – for example, an EAS might call an mobile network operator provided API (like a location service via CAPIF) to augment its application or use EES-provided APIs for retrieving information about UEs or network conditions. In EDGEAPP terminology, there are reference points (like EDGE-3 and EDGE-7) for EAS to interact with the EES and potentially with operator network exposures.

-	Edge Configuration Server (ECS): A centralized function that oversees the configuration and coordination of the edge system across multiple edge sites. The ECS can be thought of as the master controller in a multi-edge deployment. It maintains a global view of all edge sites (each with an EES and perhaps multiple EAS instances). The ECS provides services such as: answering queries from EECs or EESs about where certain application services are available (e.g., if a UE in zone A needs a service that’s currently only in zone B, the ECS can help route or instantiate it), bootstrapping new edge nodes (EESs) into the system, and storing subscription information or policies that apply network wide. In some scenarios, the EEC can contact the ECS at initial registration (EDGE-4 interface) to obtain information like the address of its serving EES if the UE is not preconfigured. The EESs also typically interface with the ECS (EDGE-6) to report new EAS deployments or to get updates on global configurations. The ECS, being a central point, can use CAPIF/NEF to interface with core network functions – for example, to fetch network resource exposure (like cell congestion status) or to publish its own APIs for other systems. The Edge Computing Service Provider (ECSP), for example the mobile network operator, is responsible for running the ECS, EES, and providing the infrastructure on which EAS instances run.

Figure 4.3-1: EDGEAPP Architecture, as specified in 3GPP TS 23.558 [4]

The typical sequence of interactions in EDGEAPP to deliver an edge-augmented service is outlined below (note that some steps can vary depending on deployment options, but this reflects a common scenario in a single operator’s network):

1.	Edge Infrastructure Registration: Before any end-user device participates, the edge nodes must be set up. Each Edge Enabler Server (EES) that comes online (for a new edge site) registers with the Edge Configuration Server (ECS). The ECS thus maintains a registry of all EESs and their capabilities/zones. This registration can involve mutual authentication and authorization – often incorporating CAPIF if the ECS’s APIs are published via CAPIF. In our context, the operator ensures all EES instances are known to the ECS (either through provisioning or dynamic discovery in larger deployments). Similarly, when an application provider wants to deploy an Edge Application Server (EAS) in a zone, they might coordinate with the operator such that the EAS gets registered with the local EES. The EES will record what service the EAS offers (e.g. “VideoAnalyticsService v1”) and possibly pass that information up to the ECS for global awareness. At this stage, CAPIF can be used: for example, the ECS and EES can onboard to CAPIF as API providers and invokers so that their APIs (like “register EAS” or “query edge info”) are accessible securely. By the end of this step, the edge control-plane is prepared: EES know about local EAS services, and the ECS knows about all EESs (and possibly their EAS listings).

2.	UE Discovery of Edge Service (Registration): When a user’s device (UE) powers on or an application starts, the Edge Enabler Client on the UE needs to discover which edge it should connect to. There are multiple ways this can happen. The UE could have a pre-provisioned address for an EES (perhaps provided by the operator’s subscription or via a device management system); or the UE could perform a DNS query or receive an indication from the network (e.g., during attach, the network might provide an “edge pointer”). Another method is the bootstrap via ECS: the EEC contacts the central ECS to ask, “where is my serving EES?” and the ECS responds with the address of the optimal EES for that UE (based on the UE’s location or subscription). Once the EEC knows which Edge Enabler Server to use, it proceeds to register with that EES. EEC-EES registration involves the EEC introducing the AC (application client) and itself to the EES: essentially saying “this UE/application wants to use edge services in your zone.” The EES authenticates the EEC (possibly via network credentials or certificates; security can rely on 3GPP credentials or CAPIF tokens if the EEC is acting as an invoker) and then creates a context for that AC on the edge. After a successful registration, the EES knows this UE (often by a unique Edge UE ID) and can start providing it with services like discovery notifications. The EEC will typically get a confirmation that it is registered. From the AC developer’s viewpoint, this whole discovery and registration process is triggered by a simple API call like Eeec_ACRegistration() from the AC to the EEC – the EEC and the network handle the rest (including contacting ECS or using CAPIF as needed).

3.	Service Discovery and Consumption: After registration, the AC can request an actual application service at the edge. Suppose the AC needs a “Video Analytics” service. The AC (through an API call to EEC, e.g. Eeec_EASDiscovery(serviceType)) asks the EEC to find an appropriate Edge Application Server (EAS) that provides that service. The EES receives this request via the EEC and checks whether an EAS in its zone offers the requested service. If yes, it selects the optimal EAS (there can be multiple instances or versions) and returns the details (e.g. the EAS’s IP address or domain and any required info to connect) to the EEC, which then passes it to the AC. If no local EAS can serve it, the EES might query the ECS to see if the service is available in another zone or if a new instance should be instantiated. Assuming an EAS is found in the local zone (Edge Zone 1 for example), the AC can then start communicating with that EAS to consume the service. Typically, the AC’s requests will still go through the EEC as a proxy (the exact data plane path can vary it could be AC → EEC (on device) → direct IP to EAS, or AC → EEC → EES → EAS, depending on whether the EES needs to anchor the traffic). In any case, the AC’s traffic to the EAS is application-layer communication (e.g. HTTP requests, video stream) and the EES/EEC ensure it is routed to the edge rather than a distant server.

4.	It’s worth noting the security aspect here: if the EAS’s API is protected (which would be the usual scenario expected), the AC/EEC can need to obtain an authorization token to invoke it. This is where CAPIF integration comes in – the EAS can expose its service API via CAPIF (with the EAS acting as an API provider and the EEC or AC as the invoker). In practice, the EEC might use CAPIF to get a token for the EAS’s API and include it in AC’s requests. However, these steps are typically hidden from the developer; they happen under the hood. The developer just sees that after discovery; the AC can call the EAS (likely the EEC provides a local API call that internally forwards to the EAS). In summary, the AC starts consuming the edge service, enjoying much lower latency because the EAS is nearby and potentially tailored to local conditions.

5.	Mobility and Service Continuity: A key feature of EDGEAPP is handling what happens when a user moves out of the coverage of one edge site to another (e.g., driving from one city to the next). Without EDGEAPP, the session to the first edge server would break or see a drastic performance drop. EDGEAPP’s design allows for Application Context Relocation (ACR) to preserve service continuity. In our example, as the UE (with AC and EEC) moves from Edge Zone 1 to Edge Zone 2, the system must transfer the ongoing session from the current EAS (in Zone 1) to a new EAS in Zone 2 (that provides the same service). The EES in Zone 1, detecting that the UE is leaving (possibly via network triggers or the UE’s EEC notifying loss of connection), coordinates with the EES in Zone 2 and the central ECS to set up a target EAS. If a suitable EAS is already running in Zone 2, it can prepare to take over the session; if not, the operator might instantiate one on the fly (depending on how dynamic the system is – this is more advanced). The state of the application (the “application context”) can be transferred from EAS1 to EAS2 – this could involve state sync or handing off relevant data (the exact method can depend on application design; EDGEAPP can signal the EAS to push state). Once Zone 2’s EAS is ready, the EES2 (Edge Enabler Server in Zone 2) will finalize the relocation: the EEC on the UE is informed of the new EAS endpoint (via a notification or updated discovery result). The AC then seamlessly continues communication with EAS2. From the AC’s viewpoint, it might just receive an event or callback that a new server endpoint is now in use – the content of the service continues with minimal disruption. The AC does not need to re-register or rediscover manually; the EEC and EES handle that transition. EDGEAPP thus ensures that mobility across edges is managed gracefully, which is critical for use cases like vehicular connectivity or users walking across a city with an AR application. This continuity is achieved while still enforcing security (if a new token is needed for the new EAS, the EEC would obtain it via CAPIF as well, transparently to the AC).

* * * End of Changes * * * *