3GPP TSG-SA WG6 Meeting #69	S6-254664

Wuhan, China 13th – 17th October 2025	(revision of S6-254069)

Source:	InterDigital

Title:	New solution on KI#3

Spec:	3GPP TR 23.700-26

Agenda item:	9.8

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This contribution proposes a new solution for KI#3 on enabling provisioning and presence monitoring for AIoT devices.

2. Reason for Change

This contribution proposes a new solution for KI#3 which is related to aspects of provisioning information and enabling presence monitoring of AIoT devices by the application enablement layer.

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 v0.1.0.

* * * First Change * * * *

## 7.x	Solution #x: Support of AIoT device presence information subscription

### 7.x.1	Solution description

#### 7.x.1.1 Introduction

This solution addresses Key Issue #3 on provisioning information in the application enablement layer and enables presence monitoring of AIoT devices.

This solution proposes that an AIoT Enablement Server (AIOTES) offers a service to authorized consumers (e.g., VAL Servers, VAL clients) for subscribing to AIoT device presence information.

This solution proposes to:

- define a new procedure and information flows to enable provisioning of AIoT presence information subscription in the AIOTES for presence reporting;

- define a new procedure and information flow for detecting events and reporting presence of AIoT devices.

#### 7.x.1.2 AIoT presence information subscription

Figure 7.x.1.2-1 describes a procedure for AIoT presence information subscription.

Figure 7.x.1.2-1. AIoT Presence Information Subscription

1.	The requestor (e.g., VAL server) sends an AIoT presence information subscription request to the AIOTES. The request includes the requestor identifier, information about AIoT device(s) [device identifier, area information] and information about the presence reporting configuration [notification endpoint, frequency, schedule, location indication, change only indication].

2.	Upon receiving the request from the requestor, the AIOTES validates if the requestor is authorized for the request. If the requestor is authorized, the AIOTES allocates a presence reporting subscription identifier, and stores the associated AIoT device information and presence reporting configuration.

The AIOTES initiates the presence monitoring by sending the Nnef_AIoT_Inventory Request as described in 3GPP TS 23.369 [3]. Based on the AIoT devices information and reporting configuration, the AIOTES includes the AIOTES identifier, the AIOTES notification endpoint, the AIoT device identifiers and may include Area Information, Location information indication and number of AIoT devices as input parameters for Nnef_AIoT_Inventory Request. If the reporting configuration includes schedule, the AIOTES sends the Nnef_AIoT_Inventory Request according to the time schedule (e.g., delays the request based on reporting time schedule).

The AIOTES configures the reporting based on the presence reporting configuration. If the reporting configuration includes frequency, the AIOTES schedules periodic reporting of the monitored presence to the notification endpoint according to frequency. If the reporting configuration includes change only indication, the AIOTES sends the notification only if a change in the presence information has happened since the previous notification.

If the AIOTES sends the Nnef_AIoT_Inventory Request and the request is successful, the AIOTES stores the returned transaction identifier.

3.	The AIOTES sends an AIoT Information Provisioning response to the VAL server. If the request was authorized, the response includes an indication of success for the request, the presence reporting session identifier and an indication that the presence monitoring has started. Otherwise, the response includes an indication of failure and may include a failure cause.

#### 7.x.1.3 AIoT presence information notification

Figure 7.x.1.3-1 describes a procedure for AIoT presence information notification.

Figure 7.x.1.3-1. AIoT Presence Information Notification

- The AIOTES detects a presence reporting event. The presence reporting event may be one of receiving a Nnef_AIoT_Notify message or detecting a reporting event based on the reporting configuration.

If the AIOTES receives a Nnef_AIoT_Notify message as described in 3GPP TS 23.369 [3], the AIOTES identifies retrieves the subscription information using the AF transaction identifier. The AIOTES stores or updates the received AIoT device presence information locally and prepares the information to be included in the AIoT Presence Information notification. If the change only indication is present in the reporting configuration, the AIOTES determines, for each device, if the received presence information has changed and the AIoT Presence Information notification is sent only if the AIOTES has determined a change. If the Nnef_AIoT_Notify message includes a last report indication, the AIOTES may send a Nnef_AIoT_Inventory Request as described in step 2 of clause 7.x.1.2 to continue receiving inventory notifications.

If the AIOTES detects a reporting event which is the periodic reporting frequency timer expiration. The AIOTES prepares the information to be included in the AIoT Presence Information notification based on the AIoT device presence information locally available.

If the AIOTES detects a reporting event which is a scheduled reporting time event. If the reporting event is to start reporting, the AIOTES starts inventory command by sending the Nnef_AIoT_Inventory Request as described in step 2 of clause 7.x.1.2. If the detected reporting event is to stop reporting, the AIOTES sends a last notification to the requestor to indicate that reporting is stopped and stops sending notifications to the requestor.

2.	The AIOTES sends an AIoT presence information notification to the endpoint configured in the presence reporting configuration.

The notification includes a presence report including the AIoT device identifiers, the presence status of the AIoT devices and may include the location information (e.g., as indicated in Nnef_AIoT_Notify), a duration since the last presence status change. The presence report includes a presence reporting status (e.g., started or stopped) if the detected event scheduled reporting time event.

### 7.x.2	Architecture Impacts

This solution proposes that a new functional entity, as an AIoT Enablement Server (AIOTES) which is a SEAL Server, is supported in the application enablement layer.

### 7.x.3	Corresponding APIs

#### 7.x.3.1	AIoT Presence Information Provisioning request

Table 7.x.3.1-1 describes the AIoT Presence Information Subscription request.

Table 7.x.3.1-1: AIoT Presence Information Subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| AIoT device information | M | The information about AIoT devices for the subscription. |
| > AIoT device identifiers | M | The list of device identifiers for which the presence is monitored. |
| > Area information | O | The area information where the presence is monitored. |
| Reporting configuration | M | The configuration for presence reporting. |
| > Notification endpoint | M | The endpoint information (e.g., URL, URI, IP address) where presence reports should be sent. |
| > Frequency | O | The frequency at which the presence is reported to the requestor. |
| > Schedule | O | The time schedule (e.g., duration, time of day, period) at which presence reporting is required. |
| > Location information indication | O | The indication that location information is requested in the presence report. |
| > Change only indication | O | The indication that reporting only includes device for which the presence status has changed. |

#### 7.x.3.2	AIoT Presence Information Subscription response

Table 7.x.3.2-1 describes the AIoT Presence Information Subscription response.

Table 7.x.3.2-1: AIoT Presence Information Subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | The indication that the request was successful. |
| > Subscription identifier | M | The identifier for the subscription. |
| Failure response | O (NOTE) | The indication that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.x.3.3	AIoT Presence Information notification

Table 7.x.3.5-1 describes the AIoT Presence Information notification.

Table 7.x.3.5-1: AIoT Presence Information notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The identifier for the subscription. |
| Presence report | M | The presence report information. |
| > AIoT device identifiers | M | The list of device identifiers for which presence is reported |
| > Presence status | M | The status of presence which is one of present or absent or unknown. |
| > Location information | O | The location information of the AIoT device. |
| > Presence status duration | O | The duration since the last presence status change. |
| > Presence reporting status | O | The status of presence reporting which may be an indication that presence reporting has started or stopped. |

* * * End of Changes * * * *