3GPP TSG-SA WG6 Meeting #69	S6-254706

Wuhan, China 13th – 17th October 2025	(revision of S6-254288,4522)

Source:	Lenovo

Title:	New Solution to Key Issue #4

Spec:	3GPP TR 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Manos Pateromichelakis (epateromiche@lenovo.com)

1. Introduction

This paper proposes a new solution for ADAE analytics related to UE energy predictions.

2. Reason for Change

This paper proposes a new solution for ADAE analytics related to UE energy predictions.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * * *

## 6.x	Solution #x: ADAES Support for AIMLE client energy sustainability analytics

### 6.x.1	Solution Description

This solution introduces a new analytics event for “AIMLE client energy sustainability analytics” as requested from the Consumer who can be the VAL Server or AIMLE server to the ADAES. These analytics may be used for predicting whether an AI/ML client at the VAL UE can be considered a candidate in a ML training or inference task, or FL process based on its expected energy status/capability. Such energy analytics can be used for identifying whether the energy consumption is sustainable for a given AIMLE service, or a recommendation on whether the AIMLE client is applicable or suitable for the AI/ML task based on the energy prediction or sustainability indicator.

This clause describes the procedure for supporting the AIMLE client energy analytics in ADAES as new analytics service.

Figure 6.x.1-1: ADAES support for AIMLE client energy analytics

1.	The Consumer (AIMLE or VAL server) sends an AIMLE client energy sustainability analytics request to ADAES to perform analytics on the AIMLE client Energy Consumption who is expected to undertake an AIML task, Event ID= “AIMLE client energy sustainability analytics”, for a given service area and a given time window. This request includes a VAL service ID or AIMLE service ID for which the analytics apply, as well as the consumer identifier and the analytics KPI. Such request can include a predicted or expected route of the VAL UE hosting the AIMLE client, and the type of analytics requested. The analytics type can be requesting whether the energy consumption is sustainable for a given AIMLE service.

Such request can also include an AI/ML task or operation or process (ML training or inference or FL task) for which the prediction/stats apply.

2.	The ADAES authorizes the consumer’s request.

3.	The ADAES maps the AIMLE client energy analytics event ID to a list of data collection event identifiers, and a list of data producer IDs. Such mapping may be preconfigured by OAM or may be determined by ADAES based on the analytics event type/vertical type and/or data producer profile.

ADAES determines and maps the analytics event to the energy data to acquire and the data producers which can be functionalities in the target VAL UE(s) and/or data stored in A-ADRF.

Optionally, the ADAES (or A-ADRF) keeps per AIMLE client. This energy profile keeps stats or energy rating of the AIMLE client or the data source in the UE (e.g. AI app) which can be used as a metric to identify whether the AIMLE client at the VAL UE is energy demanding source; hence applicable or not to be considered as capable of undertaking an AI/ML task.

NOTE: If Energy Profile is used, this information may either be provided by the VAL UE (or the VAL server) as part of the energy data or can be quantified at the ADAES.

4a.	The ADAES requests and receives from the VAL UE(s) of interest energy data using the AIMLE client Energy Data Collection API. Such Energy data collection can include measurements or stats on the energy consumption per AIMLE client.

Editor’s Note: Further details on the energy data collection from the VAL UE is FFS.

4b.	In addition, or complementary to AIMLE client energy data collection, the ADAES may also request and receive from the ADAEC of the UE, performance statistics related to the application layer or enabler layer sessions (based on existing ADAE capabilities on VAL session analytics).

5.	The ADAES calculates the expected AIMLE client energy consumption based on the received energy/ performance data in step 4, based on the request.

6.	The ADAES obtains the corresponding trained ML model based on procedure in 3GPP TS 23.482 clause 8.3.2 and performs analytics to derive the predicted UE energy consumption sustainability at the target area and time horizon.

7.	The ADAES sends the analytics output via the AIMLE client energy sustainability analytics API to the consumer (VAL or AIMLE server) as analytics response with the analytics output data (based on the type of analytics in the request). The analytics output may include:

- identification on whether the energy consumption is sustainable for a given VAL or AIMLE service, where this service can be an AI/ML service

- a recommendation on whether the AIMLE client is applicable or suitable for the AI/ML task based on the energy prediction or sustainability indicator.

The ADAES and/or the AIMLE (as consumer) may also store the energy data and/or analytics to a repository being A-ADRF or ML repository (if this applies to an ML task).

Based on step 7, the consumer (AIMLE or VAL server) can use these analytics as input to trigger the pro-active selection or re-selection or removal of the AIMLE client from the list of UEs which are capable of undertaking an AI/ML process/task/operation, given the energy factor.

### 6.x.2	Architecture impacts

This clause will provide the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 6.x.3	Solution evaluation

This clause will provide an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * * *