3GPP TSG-SA WG6 Meeting #70	S6-255401

Dallas, United States 17th – 21st November 2025	(revision of S6-255241)

Source:	Lenovo

Title:	EN resolution in Solution #7

Spec:	3GPP TR 23.700-83 v1.1.0

Agenda item:	9.4

Document for:	Approval

Contact:	Manos Pateromichelakis (epateromiche@lenovo.com)

1. Introduction

This contribution provides an EN resolution to solution evaluaton of Sol#7.

2. Reason for Change

This contribution provides an EN resolution to solution evaluaton of Sol#7.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v1.1.0.

* * * First Change * * * *

## 7.7	Solution #7: Solution on discovering AIMLE clients in cross-PLMN scenarios

### 7.7.1	Solution description

This solution addresses key issue #3 open issues 1), 3), 4).

According to 3GPP TS 23.434, one of the business models is related to the SEAL provider having PLMN operator service arrangements with multiple home PLMN operators. Additionally, according to [AR-4.2.2-b], the SEAL shall support deployments in which SEAL services are deployed only outside of PLMN network.

This solution corresponds to the following use case:

-	An AI/ML service runs in a certain service area, which can be an edge or cloud service area or geographical area, where UEs connected to different PLMNs are present. Such service can be considered multi-operator AIMLE/VAL service. VAL UEs are assumed to have AIMLE clients installed and active and connect to AIMLE server via different PLMN. The AI/ML service can be an application enablement service (AIMLE client to server) where AIMLE-UU can be over multiple MNO networks.

-	In Figure 7.7.1-1, an example is shown where UE#1 is connected to PLMN#1 and UE#2 to PLMN#2. Both UEs are connected to the same AIMLE server. Different VAL server may connect to UE#1 VAL client and different VAL server to UE#2; however, both VAL servers are connected to the same AIMLE server.

-	One example of such scenario is that AI operation running for a V2X scenario, where VAL UEs of a traffic safety or optimization service (AI-enhanced) are needed and belong to different networks.

Figure 7.7.1-1: Use case for multi-operator AIMLE scenario

The solution provides a mechanism for discovering AI/ML participants (AIMLE clients, VAL clients) in an AIMLE-assisted operation (e.g. ML model training, FL, TL) in a given AIMLE service area. Discovery of AIMLE clients is an important step in the AI/ML process for distributed, federated, split AI/ML, and transfer learning. Due to the nature of such learning, VAL servers need to discover suitable AIMLE clients to fulfil the requirements for the AI/ML application. The VAL server can then use the discovered AIMLE clients to select a set of AIMLE clients to perform AI/ML operations.

In cross-PLMN scenario as in Figure 7.7.1-2, the following enhancements are provided (over AIMLE-S or SEAL-X interface if the AIMLE consumer is another SEAL server, e.g. ADAES).

8.8.2.1	AIMLE client discovery

Pre-conditions:

1.	AIMLE clients that support AI/ML operations have registered with the AIMLE server and included their AIMLE client profiles and optionally a list of supported services.

2.	The AIMLE server can access a ML repository to obtain AIMLE client profiles and supported services associated with AIMLE clients.

Figure 8.8.2.1-1: AIMLE client discovery

1.	A VAL server sends an AIMLE client discovery request to an AIMLE server to discover a list of AIMLE clients that are available to participate in AI/ML operations (e.g., is available and have required data to train an ML model). The request may also include AIML client task capability requirements to discover clients who can perform the AIML tasks like AIML model training/offload/split with compute requirements and task performance preference like green task performance. The AIMLE client discovery request includes information as described in Table 8.8.3.1-1.

This request also includes information on the serving PLMN for the communication of the VAL service, as well as the allowed MNO info from other MNO network from which the AIMLE consumer / VAL server can discover AIMLE clients / VAL UEs.

2.	The AIMLE server performs authentication and authorization checks to determine if the requestor is able to discover AIMLE clients for the VAL UEs connected to the serving PLMN. If the requestor is authorized, the AIMLE server performs the following to discover AIMLE clients fulfilling the provided AIMLE client discovery criteria.

The AIMLE server also obtains candidate UEs connected to the serving PLMN from the ML repository.

From the list of candidate UEs, the AIMLE server discovers a list of UEs (i.e., AIMLE clients) that fulfils the discovery criteria based on supported services and AIMLE client profiles.

The AIMLE server determines whether the discovered UEs fulfil the location information in the discovery criteria by using their identifiers to determine their location and includes in the discovery response only those UEs that meet the requirements. The AIMLE server may use SEAL-LMS (as in 3GPP TS 23.434 [5] clause 9.3.9, or 3GPP TS 23.434 [5] clause 9.3.10) or 3GPP 5G Core Network Services (such as GMLC as in 3GPP TS 23.273 [13] and NEF as in 3GPP TS 23.273 [13] or 3GPP TS 23.502 [9]) to determine the UEs which fulfil the location requirement.

If the available AIMLE clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required minimum number of AIMLE clients for AI/ML operation (e.g., split AI/ML), then the AIMLE server may discover the remaining required AIMLE clients from other AIMLE servers over AIML-E reference point and include them in the response message.

3.	The AIMLE server sends an AIMLE client discovery response that includes information in Table 8.8.3.2-1.

If the required number of AIMLE clients are not included in the response message from the AIMLE server (i.e., the AIMLE clients in the response message are less than the required minimum number of AIMLE clients), then the AIML service consumer may discover the remaining required AIMLE clients from other AIML enablement server indicating the required number of AIMLE clients. The response may also include information related to supported tasks with guaranteed task KPIs, available AIML tasks service duration, AIML task expected KPI like latency.

If the available AIMLE clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required minimum number of AIMLE clients for AI/ML operation (e.g., split AI/ML) or the AIMLE clients are not available or capable for undertaking the operation, then the AIMLE server determines to discover and discovers the remaining required AIMLE clients connected to other PLMNs based on the allowed MNO information and the permissions as in step 1. The AIMLE server also obtains candidate UEs connected to the target/destination PLMN (other than the serving PLMN) from the ML repository.

The AIMLE server may also determine if a candidate UE is suitable based on roaming status and roaming policies.

### 7.7.2	Architecture Impacts

The architecture impact is the required enhancement of the AIMLE functional model to support multi-operator scenarios (as in clause 6.2).

### 7.7.3	Corresponding APIs

This subclause provides a summary on the corresponding API for solution #7. This provides enhancements to the following information flows based on TS 23.482 clause 8.8.3.1 (bold italics is new):

- Table 8.8.3.1-1 on AIMLE client discovery request

- Table 8.8.3.1-2: AIMLE client discovery criteria

- Table 8.8.3.2-1: AIMLE client discovery response

Table 8.8.3.1-1: AIMLE client discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g., VAL server). |
| AIMLE client discovery criteria | M | Discovery criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2. |
| Number of required AIMLE clients | O | Indicates the requested number of AIMLE clients to be discovered based on the discovery criteria. |
| Cross-PLMN discovery requirement | O | This parameter indicates whether the discover may span across multiple MNO networks or not. It can be seen as a multi-PLMN flag for the discovery or an indicator to further look in the AIMLE client discovery criteria for additional PLMNs |

Table 8.8.3.1-2: AIMLE client discovery criteria

| Information element | Status | Description |
| --- | --- | --- |
| Service requirement | M | Information about the required service |
| > VAL Service ID | M | VAL Service ID that the client is required to support. This identifies the service associated with the requester. |
| > Service permission level | O | Required corresponding service permission level (e.g., premium resource usage standard resource usage, limited resource usage). |
| Requested ML model types | O | Requested ML model types (decision trees, linear regression, neural networks). |
| Requested AIML operations | M | Requested role for AI/ML operations such as model training, model transfer, model inference, model offload, model split. |
| Application layer AIMLE client capabilities | M | Application layer AIMLE client capability information (e.g., ML application type like FL/TL/SL, client availability to support AIML operations at the UE, AIMLE drop off rate). |
| Dataset requirements | O | Information about dataset. |
| > Dataset availability | O | Dataset availability, including dataset identifiers, dataset size, age, list of dataset features. |
| > Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g., raw data), supported data processing capabilities (e.g., processed data) and supported exploratory data analysis functions. |
| AIML client task capability requirements | O | Indicates the AIML task requirements to discover the AIML clients for performing AIML tasks. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g. Green task, Energy-efficient, low costs) |
| AIMLE client velocity | O | Indicates the AIMLE client velocity. It includes mobile (e.g., high, low), static. |
| Location information | O | Indicates the location information (e.g., Cell Identity, Tracking Area Identity, GPS Coordinates or civic addresses, VAL service area ID) to discover the AIMLE clients. |
| AIMLE client QoS requirements | O | Indicates the AIMLE client QoS information (like PLR, bandwidth, latency jitter) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above or below) to discover the AIMLE clients. |
| Allowed MNO info | M | Information of the allowed operator(s) (e.g. MNO name, PLMN ID) from which its subscriber can consume the AIMLE services. |
| > capability limitations | O | Capability limitations per PLMN (which is not the service PLMN) are based on the service agreement between the vertical / ASP and the allowed MNO network and can provide limitations on the capabilities to be supported by the entities connected to that PLMN. One example is the reduced performance or energy limitations or exposure limitations when utilizing the access from the target PLMN (which is not the serving). |
| >Serving PLMN info | M | Information on the serving operator (MNO name, PLMN ID) where the subscriber is registered / connected. |

Table 8.8.3.2-1: AIMLE client discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| List of AIMLE client IDs | M | A list of AIMLE client IDs that matches the AIML discovery criteria. |
| > PLMN ID/name per AIMLE client | O | The PLMN in which the VAL UE is connected/registered. |
| AIML supported tasks | O | Indicates the discovered AIML task providers fulfilling the AIML task and AIML task requirements. It includes expected AIML task KPIs like latency, available AIML tasks service duration, list of AIML task providers like IDs or URI, and supported tasks. |

### 7.7.4	Solution evaluation

This solution addresses Key Issue #3 and introduces the capability to support discovery of AIMLE clients in cross-PLMN scenarios. This capability mainly introduces enhancements to AIMLE existing procedure as in clause 8.8 of 3GPP TS 23.482.

For checking the UE roaming status, it is assumed that Solution #10 is re-used.

ΝΟΤΕ: How to check the VAL UE roaming status will be further discussed in normative phase.

This solution is feasible and doesn't introduce any dependency to 3GPP network systems.

* * * End of Changes * * * *