| 3GPP TR 23.757 V0.3.0 (2020-01) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on architectural enhancements for 5G multicast-broadcast services (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Abbreviations	8

4	Architectural Assumptions and Principles	8

4.1	Common architectural requirements and principles	8

4.2	Specific architectural requirements and principles	9

4.3	Baseline functionality	9

5	Key Issues	9

5.1	Key Issue #1: MBS session management	9

5.1.1	Description	9

5.2	Key Issue #2: Definition of Service Levels	10

5.2.1	Description	10

5.3	Key Issue #3: Levels of authorization for Multicast communication services	10

5.3.1	Description	10

5.4	Key Issue #4: QoS level support for Multicast and Broadcast communication services	10

5.4.1	Description	10

5.5	Key Issue #5: Support of Broadcast TV Video and Radio communication services	10

5.5.1	Description	10

5.6	Key Issue #6: Local MBS service	11

5.6.1	Description	11

5.7	Key Issue #7: Reliable delivery mode switching between unicast and multicast	11

5.7.1	Description	11

5.8	Key Issue #8: Reliable switching between unicast and broadcast delivery methods	12

5.8.1	Description	12

5.9	Key Issue #9: Minimizing the interruption of public safety services upon transition between NR/5GC and E-UTRAN/EPC	12

5.9.1	Description	12

6	Solutions	12

6.0	Mapping of solutions to key issues	12

6.1	Solution #1: Multicast service levels	13

6.1.1	Functional description	13

6.1.2	Procedures	13

6.1.3	Impacts on services, entities and interfaces	13

6.2	Solution #2: MBS Session setup using flexible radio resources	14

6.2.1	Functional Description	14

6.2.2	Procedures	14

6.2.2.1	Requesting to allocate radio resource for MBS service	14

6.2.2.2	Releasing radio resource for MBS service	16

6.2.3	Impacts on services, entities and interfaces	16

6.3	Solution #3: Integrated Multicast and Unicast Transport	17

6.3.1	Functional Description	17

6.3.1.1	System Architecture	17

6.3.1.2	Multicast Session Context and Multicast flow characteristics	17

6.3.2	Procedures	18

6.3.2.1	Multicast context and Multicast flow setup/modification via PDU Session Modification procedure	18

6.3.3	Impacts on services, entities and interfaces	21

6.4	Solution #4: Multicast session management with dedicated MBS network functions	21

6.4.1	Functional description	21

6.4.2	Procedures	21

6.4.2.1	High-level procedures for multicast services	21

6.4.2.2	User MBS Multicast Session Establishment	23

6.4.2.3	User MBS Multicast Session Termination	24

6.4.3	Impacts on services, entities and interfaces	25

6.5	Solution #5: Broadcast Session Start	25

6.5.1	Functional description	25

6.5.2	Procedures	26

6.5.3	Impacts on services, entities and interfaces	27

6.6	Solution #6: Multicast service initiation	28

6.6.1	Functional description	28

6.6.2	Procedures	28

6.6.2.1	MBS Session initiation procedure	28

6.6.3	Impacts on services, entities and interfaces	29

6.7	Solution #7: Local multicast service discovery	29

6.7.1	Functional description	29

6.7.2	Procedures	29

6.7.3	Impacts on services, entities and interfaces	30

7	Evaluation	30

8	Conclusions	30

Annex A: Architecture alternatives	31

A.1	Baseline architecture 1: 5G MBS system architecture based on unicast 5GC	31

A.1.1	Transport aspects	31

A.1.2	Service Layer aspects	32

A.2	Baseline architecture 2: 5G MBS system architecture based on dedicated MBS Function	34

A.2.1	General	34

A.2.2	Reference Architecture	34

Annex B (informative): Change history	37

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

This Technical Report studies and evaluates architectural enhancements to the 5G System to address the following objective.

Objective A: Enabling general MBS services over 5GS.

Support general multicast and broadcast communication services, e.g., transparent IPv4/IPv6 multicast delivery, IPTV, software delivery over wireless, group communications and IoT applications, V2X applications, public safety.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 22.101: "Service aspects; Service principles".

[4]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[5]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[6]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[7]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[8]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Broadcast communication service: A communication service in which the same service and the same specific content data are provided simultaneously to all UEs in a geographical area (i.e., all UEs in the broadcast coverage area are authorized to receive the data).

Broadcast service area: The area within which data of one or multiple Broadcast session(s) are sent.

Broadcast session: A session to deliver the broadcast communication service. A broadcast session is characterised by the content to send and the geographical area where to distribute it.

Multicast service area: The area within which data of one or multiple Multicast session(s) are sent.

Editor's note:	The definitions of broadcast service area and multicast service area may be updated further according to 5MBS study.

MBS session: A multicast session or a broadcast session.

Multicast communication service: A communication service in which the same service and the same specific content data are provided simultaneously to a dedicated set of UEs (i.e., not all UEs in the multicast coverage are authorized to receive the data).

NOTE:	The transport in the 5GC for broadcast and multicast services will be determined as part of this study. A multicast communication service could for instance use a lower layer unicast or multicast transfer in the 5GC and the access interfaces.

Multicast session: A session to deliver the multicast communication service. A multicast session is characterised by the content to send, by the list of UEs that may receive the service and optionally by a multicast area where to distribute it.

Receive Only Mode: A UE configuration option that allows a UE to receive only broadcast service without the need to access and register with the PLMN offering the MBS service. Use of Receive Only Mode does not require USIM for the UE.

Shared MBS Network: A network shared by multiple PLMNs that provides multicast or broadcast services. At least the northbound data entrance point in the 5GS is shared.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

MBS	Multicast/Broadcast Service.

# 4	Architectural Assumptions and Principles

## 4.1	Common architectural requirements and principles

The following common architectural requirements and principles apply:

-	Solutions shall build on the 5G System architectural principles as in TS 23.501 [2], including flexibility and modularity for newly introduced functionalities.

-	The system shall provide an efficient transport for a variety of multicast and broadcast services.

-	Solutions shall minimize impact to existing external services.

-	Only NR of NG-RAN connected to 5GC is considered as RAT.

-	Architecture reference models defined in TS 23.501 [2] clause 4.2 are used as the baseline architecture for supporting multicast and broadcast services in this study. In particular, Figure 4.1-1 shows the MBS architecture with 5G UE, NG-RAN and 5GC.

Figure 4.1-1: High level MBS architecture

Editor's note:	The impact on RAN is to be analysed by and coordinated with the relevant RAN WGs.

## 4.2	Specific architectural requirements and principles

The following IPTV requirements and principles apply:

-	Solutions to IPTV shall minimize the impact to IPTV network and STB.

-	Solutions to IPTV shall reuse IGMP/MLD message via user plane to join/leave an IPTV channel group as Release 16.

-	Solutions to IPTV shall provide an efficient mechanism for the UE to join/leave an IP channel group, including reducing latency and signalling.

## 4.3	Baseline functionality

The sequence of events to establish and deliver a MBS session is assumed as follows:

- Negotiation between application/service layer and 5G CN for 5G MBS service

Editor's Note: It is FFS whether this step is optional or is always required.

2.	UEs participate in receiving MBS flow (for Multicast session).

3.	Establishment of MBS flow transport (what used to be session start)

4.	MBS data delivery

5.	UEs stop receiving MBS flow (for Multicast session)

6.	Release of MBS flow transport (what used to be session stop)

# 5	Key Issues

## 5.1	Key Issue #1: MBS session management

### 5.1.1	Description

To transfer the service data flow of multicast and broadcast communication services, MBS sessions need to be established and maintained. This key issue will study the following aspects:

-	When and how to trigger the establishment of an MBS session?

-	Which network function(s) is/are responsible for establishing and maintaining the MBS session (including interaction between 5GC and 3rd party)?

-	Which information/parameters are used to establish an MBS session (e.g., session related IDs, QoS information, broadcast service area or multicast service area, etc.)?

-	Which information is provided to the gNB when a UE joins a MBS multicast Session? How does the gNB associate the UE with the established MBS multicast session?

-	Which information/parameters can be updated for an established MBS session (e.g. QoS information, broadcast service area or multicast service area, etc.)?

-	Which network function(s) is/are responsible for determining the broadcast service area or multicast service area used for MBS sessions? How is the broadcast service area or the multicast service area determined (e.g. pre-defined, determined during an on-going MBS session, etc.)?

## 5.2	Key Issue #2: Definition of Service Levels

### 5.2.1	Description

5G system is envisioned to address different use cases. In some of these use cases, a complete 5G system becomes a part of another system, for instance the support of IPTV where 5G system is integrated into an IPTV network from which it receives multicast, where the role of the 5GS is primarily to provide an efficient "transparent" transport of a broadcast/multicast service. Other use cases offering for multicast or broadcast services are in vertical domains, for which the "transparent" mode is not possible or feasible and a full-service mode should be defined.

This key issue will study the service levels for the broadcast and multicast support of the 5GS.

NOTE:	For different service levels, different solution proposals can be provided as part of other key issues.

## 5.3	Key Issue #3: Levels of authorization for Multicast communication services

### 5.3.1	Description

The 5GS is expected to support different use cases of multicast services. The mobile network operators (MNO) and/or application service providers (ASP) may want to provide different levels of authorization (e.g. at session or service level) for the UE to access multicast communication services.

This key issue will study the following aspects:

-	Define and study how to support the necessary level(s) of authorization for UEs to access multicast communication services.

-	How can a UE join/leave (including authorised or revoked to access) a multicast communication service?

## 5.4	Key Issue #4: QoS level support for Multicast and Broadcast communication services

### 5.4.1	Description

Different multicast and broadcast communication services have potentially different QoS requirements. For this key issue the QoS framework of TS 23.501 [2] clause 5.7 is taken as baseline. This key issue aims at studying:

-	QoS requirements, including packet error rate, delay budget, MBR or GBR;

-	If a need for different QoS levels is determined by the QoS requirement analysis:

-	how to provide a means to support the identified QoS levels;

-	which NF determines the QoS levels of multicast and broadcast services;

-	which entity enforces the required QoS and how.

## 5.5	Key Issue #5: Support of Broadcast TV Video and Radio communication services

### 5.5.1	Description

This key issue aims at addressing:

-	E-UTRA of NG-RAN based support of broadcast TV, video and radio communication;

-	Allow Video/TV/Radio broadcast communication services to devices with no MNO subscription (e.g., "Free to air TV").

A solution to this key issue shall provide means for MNO to enable this service based on policy.

A solution for this key issue shall provide:

-	Support of Free-to-Air service via Receive Only Mode device support.

-	Support of Shared MBS functionality to allow dedicated MBS network.

-	Exposure of MBS service and transport (similar to support of xMB external reference point).

NOTE: This key issue is not addressed within the Rel-17 timeframe.

## 5.6	Key Issue #6: Local MBS service

### 5.6.1	Description

For V2X, Public Safety and other service provided locally, there may be a multicast or broadcast communication services only available in a local service area (e.g., per cell, TA or other geographic metric) and for a specific time (e.g., during event hours, emergency situation, or operation hours).

In this situation, it is also expected that different application servers can be assigned to control the multicast or broadcast communication service(s) in each service area. UEs may need to be configured to receive information about available multicast or broadcast service in the area where they are residing.

Therefore, for supporting local multicast and broadcast communication services, the following aspects will be studied:

-	Whether additional specific functionality and information (e.g. geographical range for broadcast service area/multicast service area) is required to provide local multicast and broadcast communication services in 5G system in a resource efficient manner, and if so, how.

-	How to support UEs if necessary, to discover and receive multicast and broadcast communication services which is available locally in 5G system in a resource efficient manner.

## 5.7	Key Issue #7: Reliable delivery mode switching between unicast and multicast

### 5.7.1	Description

This key issue aims at providing support for dynamic delivery mode switching in the 5GS. Depending on the number of devices receiving a specific content, their location, and RAN considerations, it may be necessary to support reliable and efficient delivery mode switching between unicast and multicast modes, i.e., to be able to dynamically transfer a unicast session to multicast delivery mode and vice versa. In addition, when a UE is receiving a multicast session, it may move across NG-RAN nodes and it is possible that the UE moves from a NG-RAN node that supports MBS to one that does not support MBS, or vice versa.

The following aspects will be studied:

-	Triggers for delivery mode switching in 5GS.

-	How delivery mode switching between unicast and multicast modes is performed in the 5GS (including the UE) while supporting service continuity.

NOTE:	During the study of this key issue, RAN WGs, SA4 and SA6 will be involved, if needed.

## 5.8	Key Issue #8: Reliable switching between unicast and broadcast delivery methods

### 5.8.1	Description

When a UE is receiving a session, it may move from a NG-RAN node that supports MBS to a NG-RAN node that does not support MBS, or vice versa.

The following aspect will be studied:

-	Triggers for switching between unicast and broadcast delivery methods.

-	How switching between unicast and broadcast delivery methods is performed in the 5GS while supporting service continuity.

## 5.9	Key Issue #9: Minimizing the interruption of public safety services upon transition between NR/5GC and E-UTRAN/EPC

### 5.9.1	Description

The approved RAN WID on NR Multicast Broadcast and Multicast (RP-193248) states in the objectives section: “Architecture: it is the one in Figure 4.1-1 in TR 23.757 v0.2.0: High level MBS architecture, with the further restriction that only NR in NG-RAN (i.e. connected to 5GC) is considered as RAT”. This means that there is no planned support for MBS over E-UTRA connected to 5GC.

This KI applies to PLMNs that have E-UTRAN/EPC based eMBMS deployments and 5G MBS over NR/5GC, with Public Safety services.

There may be UEs receiving a service subject to multicast delivery over NR/5GC that move to E-UTRAN/EPC and use eMBMS, and vice versa. How to handle these services via both EPC and 5GC, and how to handle mobility across RATs should be considered.

Therefore, solutions that address this key issue focused on public safety services only should:

-	Allow an AF (Public Safety GCS AS) to provide the same Multicast/Broadcast service to UEs camping on E-UTRAN (eMBMS), and UEs over NR connected to 5GC (via 5G MBS solution).

-	Define procedures for UEs performing inter-CN type mobility between EPC and 5GC during a multicast session.

The goal is to minimize service interruption and packet loss and achieve rapid re-connection during transitions between various systems, as much as possible.

NOTE 1:	For the impacts to Public Safety services, work is expected to be done mainly in SA6.

NOTE 2:	Scheduling time to discuss solutions for this key issue will take place after concluding Key Issues #1, #2, and #7 that will define basic architecture and procedures for MBS in 5GS.

# 6	Solutions

## 6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 MBS session management | 2 Service levels definition | 3 Levels of authorization for MC | 4 QoS for MC and BC | 5 BC TV and Radio services | 6 Local MBS | 7 MC-UC delivery mode switch | 8 BC-UC delivery method switch | 9 IWK with EPC/eMBMS for Public Safety |
| 1 |  | x |  |  |  |  |  |  |  |
| 2 | x |  |  |  |  |  |  |  |  |
| 3 | x |  |  |  |  |  |  |  |  |
| 4 | x |  |  |  |  |  |  |  |  |
| 5 | x |  |  |  |  |  |  |  |  |
| 6 | x |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  | x |  |  |  |

## 6.1	Solution #1: Multicast service levels

### 6.1.1	Functional description

The following service levels for the multicast communication service are defined:

Editor's note: Terminology to be confirmed. How the Service Level maps to Transport Only mode and Full-Service mode of operation as defined in TS 23.246 [4] is FFS.

Basic Service Level

Requirements for the basic service level are mandatory to be supported. The following requirements are defined:

-	Media are transported transparently through the 5GS;

-	Request to receive the multicast service;

-	Efficient packet distribution from the 5GS ingress to (R)AN node(s).

Editor's Note: It is FFS if the list is complete.

Enhanced Service Level

The requirements for the Basic service level also apply for the Enhanced Service Level. Additional requirements for the Enhanced Service Level are listed below. Different requirements out of this set may be necessary to address specific use cases.

Editor's Note: list to be completed.

### 6.1.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

It is expected that procedures to address the requirements for the service levels defined in this solution proposal are proposed as part of other solution proposals.

### 6.1.3	Impacts on services, entities and interfaces

It is expected that an impact analysis to address the requirements for the service levels defined in this solution proposal are proposed as part of other solution proposals.

## 6.2	Solution #2: MBS Session setup using flexible radio resources

### 6.2.1	Functional Description

This Solution assumes the architectural alternative 2 (see Annex A.2).

This solution addresses Key Issue #1.

NOTE 1: The solution assumes that the RAN can be notified by the UE of its interested MBS service.

In this solution for Multicast,

- When no MBS Session context exists in NG-RAN for an MBS service,

-	If a UE is interested in an MBS service, the UE notifies NG-RAN of the interested MBS, or a UE previously using MBS service in an old NG-RAN is moved to a new NG-RAN;

-	for the first UE initiating the MBS service interest, the (new) NG-RAN notifies the M-AMF for MBS service to initiate MBS Session creation towards the (new) NG-RAN;

-	if the MBS Session does not exist for the service in the M-AMF, the M-AMF notifies the MB-SMF to initiate the MBS Session later.

- When MBS Session context exists in NG-RAN for an MBS service,

-	NG-RAN detects that no UE is using the MBS service (e.g. due to UE moving to a different NG-RAN), it notifies the M-AMF, so that the M-AMF can initiate MBS Session stop towards the NG-RAN.

Based on the proposal above that NG-RAN can dynamically allocate the radio resource for MBS service, when UE moves from one NG-RAN to another, the multicast service can continue without switching to the unicast.

NOTE 2:	Further interaction with RAN WGs, and SA4 and SA6 will be triggered, as needed.

### 6.2.2	Procedures

#### 6.2.2.1	Requesting to allocate radio resource for MBS service

Transport Only mode is used as an example in the flows.

M-AMF is MBS specific and not associated with the UE.

Figure 6.2.2.1-1: Requesting to allocate radio resource for MBS service

NOTE 1:	Procedure (A) can happen prior to, in parallel with, or after Steps 0, 4, 5 and 6. MBS service related configuration (e.g., TMGI allocation) occurs prior UE starting MBS service setup towards 5GS.

Editor's Note: How the TMGI is provided to the UE is FFS (e.g. from the AF, via PCF etc.).

0. 	UE interacted with the Application Server, and the MBS Session will be started some time later.

1.	The Application Server starts MBS Session.

2.	The MB-SMF requests the MB-UPF to allocate IP address and port for receiving downlink traffic. The MB-SMF also requests MB-UPF to allocate the multicast address and C-TEID if the multicast address and C-TEID allocation is done by the MB-UPF.

3.	The MB-SMF responds to the Application Server with the IP address and port which the AS can send packets to.

4.	The UE notifies the NG-RAN that the UE is interested in a specific MBS service represented by TMGI.

Editor's Note: It is FFS whether UE expresses interest in a specific MBS service unconditionally (i.e., even if radio resources for this specific MBS service have already been allocated or even when the UE is in an area where the MBS service is not available). Whether RRC signalling can be used by UE to express interest in a specific MBS service depends on work in RAN WGs.

5.	No radio resource has been allocated for the MBS service, and the NG-RAN notifies the M-AMF of its interest. If radio resource has been allocated, step 5 to step 11 are skipped.

6.	The MBS Session for the MBS service is not started yet in the M-AMF, and the M-AMF stores the info that NG-RAN has interest in a specific MBS service and notifies the SMF of its interest in an MBS Service. If the MBS session has been started in the M-AMF, step 6 to step 9 are skipped.

Editor's Note: How the M-AMF discovers the MB-SMF is FFS.

Editor's Note: A check whether the UE is authorized to access the MBS service is FFS.

7.	If the MBS Session is already started, the MB-SMF immediately initiates the MBS Session towards the M-AMF, otherwise, the MB-SMF wait for the MBS Session start from MBSF/AF and then initiates MBS Session towards the M-AMF.

8-9.MB-SMF initiates the MBS Session Start Request towards the M-AMF including the multicast address and C-TEID.

10-11. The M-AMF sends the MBS Session Request also to the NG-RAN.

#### 6.2.2.2	Releasing radio resource for MBS service

Figure 6.2.2.2-1: Releasing radio resource for MBS service

1.	The NG-RAN detects that no UE is using an MBS service, e.g., due to UE moved to another NG-RAN.

2.	The NG RAN informs M-AMF that no UE within the NG-RAN is using an MBS service.

2a. If, for all NG-RAN managed by M-AMF, there are no UEs using the MBS service, M-AMF further informs MB-SMF about it. Otherwise, step 2b is skipped.

2b.	 The MB-SMF stops the MBS Session towards the M-AMF.

3.	The M-AMF stops the MBS Session towards the NG-RAN.

4.	The NG-RAN releases the radio resource and leave the multicast group.

### 6.2.3	Impacts on services, entities and interfaces

Editor's Note: This clause describes impacts to services, entities and interfaces.

## 6.3	Solution #3: Integrated Multicast and Unicast Transport

### 6.3.1	Functional Description

#### 6.3.1.1	System Architecture

This solution addresses Key Issue 1 and proposes a system architecture that reuses as much as possible the system architecture and procedures of current 5GS unicast system architecture. The architecture functional entities are described in Annex A.1 “5G MBS system architecture based on unicast 5GC”.

#### 6.3.1.2	Multicast Session Context and Multicast flow characteristics

The proposed multicast communication service session management is an extension of the existing solution for IPTV in TS 23.316 [7].

The Multicast Session context is identified by a Multicast Session context ID and is used to represent information about the group of UEs receiving Multicast flows with the same Multicast Session context ID.

In case of IP PDU session type, the Multicast session context ID represents one IP multicast group address (i.e. any source multicast or source specific multicast). The packet filters for all Multicast flows within the Multicast context shall have the same destination and, in case of source specific multicast, also source IP addresses. The default Multicast flow shall allow for any source and destination ports and any protocols. An AF may request the PCF/NEF to create policies for Multicast flows within the Multicast Session context to meet the needs of application service flows that use different ports and protocols.

Figure 6.3.1.2-1: Multicast Session Context, UE group and Multicast flow model

On N3 or MB-N3, the Multicast flows within one Multicast Session context use the same shared tunnel.

At the NG-RAN, the Multicast flow identifier maps to radio bearer. If the QoS characteristics of Multicast flows allow, NG-RAN can also map several Multicast flows within a Multicast Session context to one radio bearer. The characteristics of the radio bearer depends on RAN decision on whether to deliver the Multicast flow content via unicast or point-to-multipoint (PTM) transmission.

The 5G QoS model is extended to support Multicast flow. At Session Management level for a particular UE the Multicast session context may exist in association with at least one PDU session, and can be set-up during the PDU session establishment or modification procedure. It may be modified at any point via PDU session modification procedure. Note that the Multicast session context is common to all UEs configured with the Multicast session context, but the associated PDU session context is specific to each UE.

The Multicast context ID and Multicast flow ID are assigned by the SMF. The SMF provides the Multicast flow information (packet filters, etc.) to the UPF.

If point-to-point (PTP) tunnelling is used in N3, the SMF provides RAN with Multicast context ID, Multicast flows and associated QoS information. The RAN responds with downlink tunnel information for the Multicast context. The SMF configures UPF with Multicast flows, associated QoS information and the downlink tunnel information.

If point-to-multipoint tunnelling is used (MB-N3), the SMF provides the UPF with MB-N3 tunnelling information. The SMF provides Multicast flow Id and associated QoS information and MB-N3 tunnel information to the RAN.

Editor's Note: Whether both point-to-point and point-to-multipoint tunnelling in N3 needs to be supported or only one of the two variants is FFS.

Figure 6.3.1.2-1 depicts the user plane path for a Multicast flow.

Figure 6.3.1.2-1: Multicast context / multicast flow user plane model

When multicast traffic is established, there is no need for unicast transport in N3. In AS, when RAN decides to switch to PTM transmission, the DRB previously allocated to the QoS Flow is not used anymore.

An UPF, based on configuration received from SMF, identifies a packet as belonging to a Multicast flow, in which case it delivers it to one or multiple RAN nodes via a shared tunnel identified by a shared TEID associated with the Multicast session context to which the Multicast flow belongs to.

The AN delivers download data for the Multicast flow via broadcast or unicast over the air.

Each Multicast session context is managed by one SMF.

Editor's note: How to handle the case where a UE requests to establish a Multicast session context associated with a PDU session, but the Multicast session context is managed by a different SMF, is FFS.

### 6.3.2	Procedures

#### 6.3.2.1	Multicast context and Multicast flow setup/modification via PDU Session Modification procedure

The Multicast context and Multicast flow setup/modification uses an enhanced PDU session modification procedure for unicast traffic defined in TS 23.502 [8].

Figure 6.3.2-1: PDU Session modification for multicast

1.	The content provider announces the availability of multicast using higher layers (e.g., application layer). The announcement includes at least the multicast address of a multicast group that UE can join.
The content provider may also send a request to reserve resources for the corresponding multicast session to the NEF and communicate the related multicast address.

The content provider may invoke the services provided by the PCF or NEF to provision the multicast information. The multicast information is used to identify (e.g., IP Address of multicast data) and reserve resources for the multicast. Multicast information may further include: QoS requirements, UE authorization information, service area identifying the service scope, and start and end time of MBS. The existing procedure defined in TS 23.502 [8], section 4.15.6.2 could be used as starting point.

Editor's note: Other parameters provided by content provider are FFS.

Editor's note: Signalling interactions between the content provider and PCF for multicast session policies is FFS.

NOTE 1:	The request to reserve resources for the corresponding multicast session is optional and can be replaced by configured data based on commercial agreements. If IP multicast is used in the external network, the content provider does not require information where to send the multicast data.

2.	The UE registers in the PLMN (see subclause 4.2.2.2 of 3GPP TS 23.502 [8]) and request the establishment of a PDU session (see subclause 4.3.2.2 of 3GPP TS 23.502 [8]). The AMF obtains information from the UDM whether the UE can join multicast sessions as part of the SMF Selection Subscription data. If so, for direct discovery, the AMF selects an SMF capable of handling multicast sessions based on locally configured data or a corresponding SMF capability stored in the NRF.

3. Alternative 1: user plane signalling

3a. The UE joins the multicast group

3b.	The UPF is a multicast capable router. The reception of the join message triggers the UPF to notify the SMF. The UPF can be optimized to send the notification only when the UE’s status in regard to number of multicast groups UE has joined changes, i.e., when the UE joins or leaves a group. The SMF initiates PDU session modification procedure upon the reception of the notification from the UPF.

4.	Alternative 2: control plane signalling

4a.	The UE sends the PDU Session Establishment/Modification Request either upon a request from higher layers or upon a detection by lower layers of UE joining a multicast group (i.e., detection of IGMP or MLR and detection of the change of content of these messages). The PDU Session Modification Request shall include information about multicast group, which UE wants to join, such as multicast addresses listed in the IGMP and MLR messages. This information is needed for configuration of the UPF with appropriate packet filters.

4b.	The AMF invokes Nsmf_PDUSession_UpdateSMContext (SM Context ID, N1 SM container (PDU Session Modification Request with the multicast information)).

5.	The SMF checks whether a multicast context for the multicast group (address) exists in the system, i.e., whether there is a UE that already joined the multicast group. If the multicast context for the multicast group does not exist, then the SMF creates it when the first UE joins the multicast group. If it is the first UE joining the multicast group, the UPF may also have to join the multicast tree towards the content provider.

6.	The SMF requests the AMF to transfer a message to the RAN node carrying transparently to RAN node an indication that the UE joined the multicast session identified by the multicast address using the Namf_Communication service.

Editor's note: Whether the N1N2 message transfer service operation or a new service operation is used is FFS. The related Session Management signalling towards the UE, e.g., providing information regarding Multicast Context and Multicast Flows is also FFS. It is also FFS if additional dedicated signalling towards the RAN node to establish a multicast distribution session is required.

7.	The session modification request is sent to the RAN. The request is sent in the UE context using the currently standardized message enhanced with multicast related information, which includes a multicast group identity (e.g., multicast address itself, Multicast Session context ID, or multicast flow information such as multicast QoS Flow ID and associating QoS information). The RAN is using the multicast group identity to determine that the session modification procedures of two or more UEs correspond to one multicast group. In other words, the RAN learns what UEs are receiving the same multicast from the multicast group identity. When the RAN receives a session modification request for previously unknown multicast group identity, the RAN is configured to serve this multicast group.

Editor's note: It is FFS which information is used by NG-RAN to determine the mapping between a Multicast flow ID and a (unicast) DRB of a specific UE.

Editor's note: It is FFS whether RAN awareness about the group of UEs receiving the same multicast traffic poses an issue for certain Public Safety scenarios.

8.	The RAN performs the necessary access network resource modification such as configuration of broadcast bearers.

Editor's note: The details of this procedure should be studied in the RAN WGs.

9.	The RAN sends the session modification response that may include downlink tunnel information, see step 6.

10.	The AMF transfers the possible downlink tunnel information received in step 9 to the SMF. The SMF stores the information about the multicast distribution session towards the RAN node serving the UE and the possible received downlink tunnel information if it did not previously store that information when handling a multicast service request of another UE served by the same RAN node.

Editor's note: Whether the Nsmf_PDUSession_UpdateSMContext service operation or a response to the Namf_Communication service operation is used is FFS.

11.	If the RAN node serving the UE does not yet receive the multicast distribution session, the SMF handling the multicast distribution session sends an N4 session modification request to the UPF including tunnel information if unicast tunnelling is used.

12.	The UPF receives multicast PDUs according to the configuration in step 11.

13.	The UPF sends multicast PDUs in the N3/N9 tunnel associated to the multicast distribution session to the RAN. There is only one tunnel per multicast distribution session and RAN node, i.e., all associated PDU sessions share this tunnel.

14.	The RAN selects multicast or unicast radio bearers to deliver the multicast PDUs to UEs that joined the multicast group.

15. The RAN performs the transmission using the selected bearer.

### 6.3.3	Impacts on services, entities and interfaces

SMF: The SMF must handle a multicast context and the enhanced PDU session procedures.

UPF: If the UE joins multicast group via user plane, the UPF must support a new capability to trigger a user plane event in a response to the reception of a join message. The UPF should also act as multicast capable router but this functionality was already introduced in TS 23.316 [7].

RAN: The RAN must support the PDU session procedures and store UEs’ association with multicast group in a context as received from the SMF. The RAN should be able to select PTP or PTM bearers that are used for multicast data transmission to UEs.

N3: A tunnel on this interface, which is configured when the first UE joins a multicast group and PDU session modification is performed, should be used to deliver multicast data from the UPF to the RAN.

PCF: The PCF interacts with Content Provider to receive QoS requirements, UE authorization information, service area, and start and end time of MBS sessions.

UE: It needs to indicate the MBS service information as part of the user plan join message (e.g., IGMP join), or of the control plan message (e.g., PDU Session modification request).

## 6.4	Solution #4: Multicast session management with dedicated MBS network functions

### 6.4.1	Functional description

This solution addresses KI#1 "MBS session management".

This solution decouples the possible PDU session user plane with the 5G MBS multicast session user plane. The architecture of this solution is based on alternative 2 (see Annex A.2).

### 6.4.2	Procedures

#### 6.4.2.1	High-level procedures for multicast services

Figure 6.4.2.1-1 shows the high-level procedure for multicast services. In this figure, the Internal Content Provider is deployed by MNO itself hence belongs to the PLMN domain, which does not need NEF to access MBS services, while the External Content Provider is a third party Application Server hence does not belong to the PLMN domain, which needs NEF to access MBS services.

Figure 6.4.2.1-1: High-level procedures for multicast services

<Stage 1> Multicast service configuration:

1.	The Content Provider performs TMGI management procedures over MB2 interface as described in clause 5.1.2.2 of TS 23.468 [5], or performs service management procedures over xMB interface as described in clause 5.3 (except 5.3.6) of TS 26.348 [6]. The NEF shall support MB2 interface or xMB interface to interact with external Content Provider, and invoke service operators of the MBSF for service association accordingly.

2.	UE may interact with the application server of a Content Provider over user plan for a multicast service. This can be used when the Content Provider does not need service announcement as described in TS 23.246 [4].

3.	The Content Provider performs MBS Bearer management procedures over MB2 interface as described in clause 5.1.2.3 and clause 5.1.2.4 of TS 23.468 [5], or performs session management procedures over xMB interface as described in clause 5.4 of TS 26.348 [6]. The NEF shall support MB2 interface or xMB interface to interact with external Content Provider, and invoke service operators of the MB-SCF for service parameters provisioning, e.g. QoS requirements, service area, etc., and subscribe events exposed from the MBSF.

<Stage 2> Multicast service operation:

4.	The UE that interests in a multicast session initiates a User MBS Multicast Session Establishment or Termination procedures with AMF, SMF, MB-SMF, MB-UPF, and MB-SCF. The MB-SMF interacts with the MBSF to perform MB service control for the UE, and interacts with the MB-UPF to create and modify the MBS session paths accordingly.

5.	If events have been subscribed by the internal Content Provider or the NEF, when condition of events are met, the MBSF performs MBMS Delivery Status Indication procedure with the internal Content Provider over MB2 interface as described in clause 5.1.2.5 of TS 23.468 [5], or performs Service Notification procedure with the internal Content Provider over xMB interface as described in clause 5.3.6 of TS 26.348 [6], or performs MB-SCF service notification with the NEF. The NEF shall support MB2 interface or xMB interface to interact with external Content Provider, and notify the event to the external Content Provider accordingly.

Editor's note: What entity allocates IP multicast address used is FFS.

The following two sub-clauses use External Content Provider as an example to describe how to establish and terminate a MBS session in step 4 of Stage 2. For Internal Content Provider, it can directly interact with MBSF without NEF involved.

#### 6.4.2.2	User MBS Multicast Session Establishment

Figure 6.4.2.2-1 shows the procedure for 5G MBS multicast session establishment:

Figure 6.4.2.2-1: User MBS Multicast Session Establishment procedure

1.	The UE sends an IGMP (IPv4) or MLD (IPv6) Join message over user plan of a PDU session to signal its interest in receiving a particular multicast 5G MBS session service identified by an IP multicast address.

2.	The UPF detects the IP data of IGMP or MLD, and forwards the IP data to the SMF according to the PDR installed in the UPF.

3.	The SMF selects a MB-SMF, e.g., based on local policy, subscription data, or UE location.

4.	The SMF invokes Nmbsmf_service_operator1 (SUPI, GUAMI, PDU session ID, N2 terminating point, UE location, IP data) and get response. The PDU session ID correspond to the PDU session for transferring the IGMP (IPv4) or MLD (IPv6) Join message.

5.	The MB-SMF resolves the IP data to get the IP multicast address and message type etc., and invokes Nmbsf_service_operator1 (SUPI, IP multicast address) to indicate a user join.

6.	The MBSF may invokes Nudm_SDM_Get service operator to authorize the user for the multicast session based on subscription, or the MBSF may notify the Content Provider of user join based on the event subscription. When receiving the user-join event, the Content Provider performs bearer management procedure or session management procedure to update the service parameters with information whether it allows the user to join the session or not, the MBSF waits for the authorization result.

7.	The MBSF responses to the Nmbsf_service_operator1 with the authorization result and may include security information in the service operator if required by the Content Provider.

8.	If authorization success, the MB-SMF selects a MB-UPF for the multicast session.

9.	If the RAN node is not in the 5G MBS session, the MB-SMF updates the MB-UPF to include the RAN node into the paths of the 5G MBS session. If the MB-SMF received security information from the MB-SCF and the RAN node is the first downlink endpoint of the 5G MBS session, the MB-SMF also sends the security information to the MB-UPF.

10.	If authorization success and the MB-SMF has not subscribed N1-MB message notification to the AMF, the MB-SMF invokes Namf_Communication_N1MessageSubscribe (MB-SMF ID, N1-MB message types, [SUPI]) service operator and get response.

NOTE:	If the deployment case is that multiple MB-SMF can subscribe to one AMF for N1-MB message notification, then the subscribe service operator shall include SUPI.

11.	If authorization success, the MB-SMF generates a N1 message1 ([PDU Session ID], [security information]) for the 5G MBS session to indicate the UE of the multicast session establishment. The N1 message1 actually is a N1-MB message, which may also include PDU session ID received from the SMF and security information received from the MB-SCF. The MB-SMF invokes Namf_Communication_N1N2MessageTransfer (N1 message1, [N2 message1]). The N2 message actually is a N2-MB message, which includes PDU session ID and information to RAN node, e.g., for shared CN tunnel setup, service QoS modification, etc.

12.	The AMF forwards the N1 message1 ([PDU Session ID], [security information]) to the UE. If the N2 message1 received, the AMF forwards the N2 message1 to the RAN node.

13. After receiving the N2 message1, the RAN may response a N2 message2 with the information for the 5G MBS session and the PDU session ID.

14.	The AMF invokes Nsmf_PDUSession_UpdateSMContext (N2 message2) to the SMF.

15.	The MB-SMF invokes Nmbscf_service operator2 including the N2 message2.

16.	The MB-SMF may update the MB-UPF for the 5G MBS session according to the received N2 message2.

17. The MB-SCF responses to the MB-SMF.

18. The MB-SMF responses to the AMF.

Editor's note: The actual service operator names of MB-SMF and MBSF need to be determined during normative phase.

#### 6.4.2.3	User MBS Multicast Session Termination

Figure 6.4.2.3-1 shows the procedure for 5G MBS multicast session termination:

Figure 6.4.2.3-1: User MBS Multicast Session Termination procedure

1.	The UE sends an IGMP (IPv4) or MLD (IPv6) Leave message over user plan of a PDU session to leave a particular multicast service identified by an IP multicast address.

2.	The UPF detects the IP data of IGMP or MLD, and forwards the IP data to the SMF according to the PDR installed in the UPF.

3.	The SMF selects a MB-SMF, e.g., based on local policy, subscription data, or UE's current location.

4.	The SMF invokes Nmbsmf_service_operator1 (SUPI, GUAMI, PDU session ID, N2 terminating point, UE location, IP data) and get response. The PDU session ID corresponds to the PDU session for transferring the IGMP (IPv4) or MLD (IPv6) Leave message.

5.	The MB-SMF gets the IP multicast address and invokes Nmbsf_service_operator2 (SUPI, IP multicast address) to indicate a user leaving.

6.	The MBSF may invokes Nudm_SDM_Get service operator to authorize the user for the multicast session based on subscription, or the MB-SCF may notify the Content Provider of user leaving based on the event subscription. When receiving the user-leaving event, the Content Provider performs bearer management procedure or session management procedure to update the service parameters with information whether the user has joined the multicast session or not, the MB-SCF waits for the authorization result.

7.	The MBSF responses to the Nmbsf_service_operator2 with the authorization result.

8.	If authorization success and the MB-SMF has not subscribed N1-MB message notification to the AMF, the MB-SMF invokes Namf_Communication_N1MessageSubscribe (MB-SMF ID, N1-MB message types, [SUPI]) service operator and get response.

NOTE:	If the deployment case is that multiple MB-SMF can subscribe to one AMF for N1-MB message notification, then the subscribe service operator shall include SUPI.

9.	If authorization success, the MB-SMF may generate a N2 message3 and invokes Namf_Communication_N1N2MessageTransfer (N2 message3). The N2 message3 actually is a N2-MB message, which includes information to RAN node, e.g., for shared CN tunnel release, service QoS modification, etc.

10.	The AMF forwards the N2 message3 to the RAN.

Editor's note: The actual service operator names of MB-SMF and MB-SCF need to be determined during normative phase.

### 6.4.3	Impacts on services, entities and interfaces

Editor's note: This clause describes impacts to services, entities and interfaces.

## 6.5	Solution #5: Broadcast Session Start

### 6.5.1	Functional description

This solution enhances the existing 5GC architecture to provide broadcast service. The 5GC is responsible to manage the broadcast service based on, e.g., broadcast service information from AF. The baseline architecture 1 in Annex A.1 is assumed.

When the AF requests the 5GS to provide a broadcast service, the AF provides to the PCF directly or via NEF the broadcast service related information, including service identifier, service area, QoS requirement information. The PCF derives the involved SMFs to support the requested service based on e.g. the service identifier, service area. The PCF sends the service session start to the involved SMFs with the service identifier, the DNN info, slicing information, QoS profiles etc. The involved SMF(s) select the involved AMF(s) and involved PSA UPF and other UPF(s) according to the service identifier. The AMF(s) send service session start to the involved RAN node(s) with the service identifier.

Editor´s Note: It is FFS whether (and why) several SMFs need to be involved.

Editor’s Note: How an SMF selects an AMF and why such functions are needed is FFS.

There is one PSA UPF targeting N6 for one broadcast service, and the PSA UPA is responsible to distribute the broadcast traffic to multiple UPFs or RANs within the service area over N9 or N3 tunnels configured by SMF(s). The PSA UPF can be configured by multiple SMFs for one broadcast service and each N9 or N3 tunnel can only be configured by one SMF based on local configuration. Taking Figure 6.5.1-1 as an example, SMF1 configures UPF1, RAN1 and corresponding N9-1, N3-1 tunnel, and SMF2 configures UPF2, RAN2 and corresponding N9-2, N3-2 tunnel.

For QoS aspect, the SMF1 and SMF2 obtain the uniform QoS profiles from PCF and provide to the PSA UPF, then the PSA UPF enforces the QoS for the broadcast service.

Figure 6.5.1-1: Multiple SMFs for one broadcast service

### 6.5.2	Procedures

Figure 6.5.2-1: Broadcast Session Start procedure

0: The broadcast service is announced in application layer, the UE receives the broadcast service information from application server.

Editor's note: It is FFS how and when the AF obtains the radio-level identifier of the broadcast service (e.g. TMGI) that is provided to the UE.

1: The AF requests to start one specific MBS Service to PCF or via NEF, providing the MBS assistance information, MBS service identifier, service area, service QoS requirement etc.

2: The PCF selects involved SMF(s) for the MBS service based on the received MBS assistance (MBS service identifier, or service area) and local pre-configuration and the PCF generates the QoS profiles for the MBS service.

3: The PCF sends session start request to the involved SMF(s) determined in Step 2, providing the service identifier, service area, DNN information, S-NSSAI information, and QoS profile.

4-5: The SMF(s) determines the involved AMF(s) based on the MBS service identifier or service area and local configuration and sends session start request to AMF(s) with MBS identifier, including N2 message (service identifier, QoS parameters) for the MBS service.

Editor's note: It is FFS if this AMF is for that dedicated UE and already selected as per Architecture option 1 via Unicast session establishment, then is it the same AMF for the UE or dedicated AMF for Session Start?

6-7: The AMF(s) select involved RAN nodes based on the MBS service identifier and local configuration and establishing context for the MBS service.

Editor's note: It is FFS if AMF selects the involved RAN nodes and in that case is the AMF dedicated for MBS session?

8: If the RAN accepts the request, the RAN allocates the DL tunnel address information for the MBS service and responds to AMF/SMF.

9: the SMF establishes session towards UPFs, including the DL tunnel address and QoS parameters.

10: The SMF responds to the PCF the MBS session is established.

11: The PCF responds to the AF the MBS service is started.

### 6.5.3	Impacts on services, entities and interfaces

In order to support this solution, the involved network entities need to support the following functionality.

PCF:

-	Receives the broadcast service information from AF, e.g. service identifier, service area, service requirements.

-	Determines the broadcast area and the involved SMFs for the target broadcast service.

-	Determines the policy and QoS profile for the broadcast service.

-	Determines DNN and slicing.

-	Authorization management.

SMF:

-	Receives the broadcast service information from the PCF, e.g. service identifier, QoS profile

-	Determines the involved PSA UPF and other UPFs;

-	Determines the involved AMF(s);

-	Manage the N3/N9 tunnels for the broadcast service.

AMF:

-	Receives the broadcast service information from the SMF, e.g. service identifier.

-	Determines the involved RAN nodes for the target broadcast service.

PSA UPF:

-	The anchor UPF connects to broadcast service based DNN.

-	Distributes the broadcast traffic to the involved UPFs configured by SMF.

-	Enforce broadcast QoS rule.

RAN node:

-	Allocate the N3 tunnel address for the broadcast service.

-	Transmitting broadcast traffic with satisfying the QoS requirements.

## 6.6	Solution #6: Multicast service initiation

### 6.6.1	Functional description

This solution addresses Key Issue 1 and proposes a multicast service initiation procedure based on the MBS reference architecture alternative 2 (see Annex A.2).

### 6.6.2	Procedures

#### 6.6.2.1	MBS Session initiation procedure

Figure 6.6.2.1-1: MBS session initiation procedure

The figure depicts MBS session initiation procedure as follows.

1.	Third party contents provider (or AF) requests MBS service(s) to MBSF (optionally via NEF) with service characteristics (e.g. service type, service area, target service UE group, etc.). At this step MBSF may check whether the AF is authorized to start the MBS service with UDM based on the target service UE group ID. If the MBS session is authorized, MBS session information (e.g. MBS session ID, address information of the MBSU, etc.) is selected by the MBSF and notified to the content provider.

Editor's note: It is FFS if a service area in addition to a UE group is mandatory. Service type requires more explanation.

2.	MBSF initiates a multicast (shared) PDU session setup to MB-SMF so that the MB-SMF may get service authorization including authorized QoS info from PCF. The MBSF establishes a transport tunnel between MBSU and MB-UPF for multicast/broadcast delivery of the MBS data.

Editor's note: The roles of PCF and UDM in this step need to be clarified. Is the PCF extended to perform policy control for a UE group?
It is also FFS how the MB-SMF is selected.

3.	The MB-SMF selects AMF(s), which in turn discovers and selects appropriate NG-RAN(s) for multicast/broadcast delivery of MBS data. To start the multicast/broadcast delivery, the NG-RAN establishes a multicast (shared) tunnel to the MB-UPF selected in the step 2.

Editor's note: Criteria for the node selection should be added.

4.	Based on the step 3, MBS service announcement for multicast/broadcast delivery can be delivered to UEs, which includes the MBS service specific parameters e.g. DNN, MBS session ID, target service UE group ID, multicast IP address, RAN related information for broadcast/multicast delivery, etc.

Editor's note: How to perform MBS service announcement is FFS RAN related information for broadcast/multicast delivery is decided by RAN WGs. Can the service announcement also be performed on application level by the AF?

Editor's note: whether additional UE interactions to join the multicast session are required is FFS.

### 6.6.3	Impacts on services, entities and interfaces

Editor's note: This clause describes impacts to services, entities and interfaces.

## 6.7	Solution #7: Local multicast service discovery

### 6.7.1	Functional description

This solution addresses KI#6 "Local MBS service".

This solution reuses UE Configuration Update procedure to provision local MBS multicast service area information to the UE, who is a member of the multicast service. When the UE, who is interesting with the multicast service, enters into the multicast service area, it detects the broadcast signalling (i.e. SIB) for local multicast service discovery.  The baseline architecture 2 in Annex A.2 is assumed.

Editor´s note: 	Network function names should be aligned with architecture 2. Architecture 2 also shows no PCF,

### 6.7.2	Procedures

Figure 6.7.2-1 shows the procedure of 5G MBS multicast service discovery for receiving multicast data:

Figure 6.7.2-1: Local multicast service discovery procedure

1.	The UE may register to the server application of the local MBS Content Provider and may get information for receiving the local multicast service data from the Content Provider, e.g., application layer security information, TMGI etc. The TMGI is allocated by MBSF.

2.	The Content Provider create or update the parameters of the local multicast service associated with the user with service boundary and service information, e.g., IP multicast address, TMGI, URL, etc.

Editor's note: It is FFS whether and how the content provider should consider the current UE location. A signalling flood to update many unaffected UEs should be avoided.

3.	The NEF stores or updates the information related to the local multicast service associated with the user.

4.	The UDR invokes Nudr_DM_Notify (UE ID, service information, service boundary) to notify the PCF the change of the service parameters. The PCF may translate the service boundary information into TAI list or cell IDs.

Editor's note: Whether the PCF is a proper NF for local multicast service parameters provisioning is FFS. If other NF is used instead of PCF, how for the NF to provide the service parameters to the UE is FFS, e.g. using UE Configuration Update procedure for transparent UE Policy delivery, etc.

Editor's note: Whether UDR can be skipped is FFS.

5.	The PCF initiates UE configuration update procedure to notify the UE of the service parameters, which includes TMGI, service boundary, etc.

6.	If the UE is interesting with the local multicast service, the UE starts to detect whether it is in the Service Boundary, e.g. based on the TAI received in the SIB message.

Editor's note: Detail of step 6 for why not use TMGI detection in SIB to discover the local multicast service is FFS.

Editor´s note: Usage of TAI is FFS. TA planning could be impacted by the requirements for local multicast service areas.

7.	When the UE is in the local multicast service area, if the UE does not receive multicast service data, or lack of information for receiving the multicast service data, e.g. network layer security information, the UE initiates multicast session establishment procedure to join the local multicast session.

### 6.7.3	Impacts on services, entities and interfaces

Editor's note: This clause describes impacts to services, entities and interfaces.

# 7	Evaluation

# 8	Conclusions

Annex A:
Architecture alternatives

# A.1	Baseline architecture 1: 5G MBS system architecture based on unicast 5GC

## A.1.1	Transport aspects

It is assumed that the 5G MBS system architecture reuses as much as possible the system architecture and procedures of current 5GS unicast system architecture for Multicast Transport.

Figure A.1-1 shows the 5G system architecture for integrated Multicast transport with unicast. The solution relies on enhancing the existing 5GS network functions, NG-RAN and UE currently only supporting unicast transport, to support Multicast transport.

Figure A.1.1-1: 5GS enhancement for Multicast support

The following new functionality is added to the current 5GC NFs, NG-RAN and UE:

-	NEF:

-	5G MBS service exposure.

-	Negotiation of 5G MBS service with AF, including QoS, 5G MBS service area.

-	PCF:

-	Support policies for Multicast services, including QoS parameters like 5QI, MBR, GBR.

-	Provide policy information regarding the MBS session to SMF.

-	Receive MBS service information from AF, directly (operator owned) or indirectly via NEF.

-	Note it is possible to establish multicast transport without interaction with NEF.

-	SMF:

-	Control of MBS transport, based on received MBS policies from PCF.

-	Configuration of the UPF for MBS flows and for point to point or point-to-multipoint transfer.

-	Configuration of the RAN for MBS flows and QoS information.

-	SM configuration at the UE for MBS flows.

-	An SMF may be used for both unicast and MBS.

-	UPF:

-	Support of packet filtering of MBS flows, and delivery of MBS flows to RAN via point to point or point-to-multipoint N3.

-	Receive 5G MBS flow configuration from SMF.

-	Detection of IGMP packets and notification to SMF. (If UE joining is performed via IGMP).

-	A UPF may receive both unicast and MBS flows.

-	A UPF may be configured to deliver a same flow as MBS flow in a certain area using point-to-multipoint tunnelling and as unicast flow to specific UEs (i.e. for legacy UEs).

-	NG-RAN:

-	Reception of MBS flows via N3 and delivery over-the-air.

-	Switch between multicast and unicast delivery of MBS flows.

-	UEs configuration for MBS flow reception at AS layer. (TBD how UE AS layer configuration of 5G MBS works)

-	UE:

-	Support of UE policy configuration extension to MBS.

-	Support of SM extension for MBS flows.

-	Signalling for joining MBS flow (via SM signalling or user plane IGMP Join).

-	MBS support at AS layer.

## A.1.2	Service Layer aspects

Orthogonal to the description of the multicast flow user plane model described in clause A.1.1 a service layer can be supported on top.

The service layer is fully separate from the Multicast transport. This allows for applications that do not require a service layer to establish a multicast transport directly via Nnef (control plane and N6 (user plane data)

Figure A.1.2-1 shows an example for service layer support of multicast/broadcast using xMB/MB2 as entry point. A new Network Function, called Multicast Service Function (MSF) is introduced. The MSF provides only Service layer functionality and requests the 5G system (via Npcf or Nnef) for the underlying Multicast transport necessary for the Multicast service. The MSF has the following functionality:

-	Entry point for both control plane service layer signalling and user plane data, e.g. xMB/MB2. Interaction can happen directly with external AF or via NEF.

-	MSF Control Plane (MSF-C):

-	Multicast service configuration.

-	xMB-C/MB2-C termination.

-	Codec configuration (if needed).

-	MSF User Plane (MSF-U):

-	xMB-U/MB2-U termination.

-	Encoding of data at service layer.

-	Multicast service layer data packets delivery via N6.

Editor’s note: the need for any other service layer functionality is FFS.

Figure A.1.2-1: 5G MBS system architecture with Multicast Service Function

Note that an Application may not need any specific service layer functionality, in which case the Application may use directly Nnef for Multicast session configuration/negotiation and N6 for multicast data delivery, as depicted in Figure A.1.2-2.

Figure A.1.2-2: MBS system with direct Application Server/Function interaction.

# A.2	Baseline architecture 2: 5G MBS system architecture based on dedicated MBS Function

## A.2.1	General

Existing 5GS architecture is reused if the MBS user service is delivered to the UE via unicast. To support multicast/broadcast MBS user service delivery in 5GS, new functional components and necessary enhancement to the existing entities are described in this clause.

## A.2.2	Reference Architecture

Editor's Note: MB-SMF and MB-UPF, M-AMF, MBSF and MBSU are functional components, whether to introduce standalone entities or reuse existing entities to cater for the functions is FFS.

Editor’s Note: EPC supports various different SGi-mb options for eMBMS. The selection of one or more N6 options for MBS is FFS.

To support MBS in 5GS user service delivery, two variants exist: one for Transport Only Mode, and the other for Full Service Mode as defined in TS 23.246 [4] clause 7.5.

Editor’s Note: These two services modes need to be mapped to 5GS MBS context.

Figure A.2.2-1 illustrates single architecture for MBS in 5GS.

UE

M

-

AMF

MB

-

UPF

MB

-

SMF

NEF

MBSF

MBSU

Application

NG

-

RA

N

Uu

N

2

MB

-

N

3

MB

-

N11

N6mb_C

N

6

Nx

Ny

NxMB

-

U

NxMB

-

C

N33

Figure A.2.2-1: 5GS Architecture supporting MBS

Editor’s Note: whether MBSU should be involved for Transport Only Mode is FFS.

Enhancement to existing entities and new functional components are as follows:

-	UE, NG-RAN and M-AMF are enhanced to support MBS.

-  	MB-SMF (Multicast/Broadcast Session Management Function) and MB-UPF (Multicast/Broadcast User Plane Function) are new functional components in 5GS,

- 	MBSF (Multicast/Broadcast Service Function) is a new Network Function to handle signalling part to cater for the service layer capability in the Transport-Only and the Full Service Mode. It also provides an interface to the Application Server or content provider in the Transport Only mode.

- 	MBSU (Multicast/Broadcast Service User plane) is new entity to handle the payload part to cater for the service layer capability.

-	NEF is an existing NF, the role/functions associated with NEF is FFS.

-	PCF may be enhanced, further details FFS.

Enhancement to existing interfaces and new interfaces are as follows:

-	Uu interface

Editor's note:	Whether Uu interface is impacted depends on RAN evaluation.

-	N2 interface is impacted to handle MBS Session.

-	N6 interface supports the MBS user plane.

-	MB-N11, implemented as NF Services.

-	MB-N3.

-	Nx.

Editor's note:	Whether to enhance existing N4, N3 and N11 or to introduce a new interfaces depends on the discussion whether standalone MB-SMF and MB-UPF are to be introduced.

-	Ny: new (optional) interface between the new MBSF and MBSU to provide the service layer capability in Full Service Mode.

-	N6mb-C: new interface between the new MBSF and MB-SMF, implemented as NF Services.

-	NxMB-C and NxMB-U: new interface between the new MBSF/MBSU and NEF. NxMB-C is implemented as NF Services.

The difference between transport only and full service mode of operation relies on the transparent delivery of the MBS service via 3GPP networks.

Editor’s Note: Impacts and functional distribution difference between the two modes of operation is FFS.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-10 | SA2#135 | S2-1910692 | - | - | - | S2-1910692: Proposed skeleton agreed at S2#135 | 0.0.0 |
| 2019-10 | SA2#135 |  | - | - | - | Inclusion of documents agreed in SA2#135: S2-1910694. S2-1910742, S2-1910824, S2-1910830, S2-1910826, S2-1910827, S2-1910828, S2-1910507, S2-1910696 | 0.1.0 |
| 2019-11 | SA2#136 |  | - | - | - | Inclusion of documents agreed in SA2#136: S2-1911638, S2-1912694, S2-1912699, S2-1912444, S2-1912698, S2-1912743, S2-1912696, S2-1912744 | 0.2.0 |
| 2020-20 | SA2#136AH |  | - | - | - | Inclusion of documents agreed in SA2#136AH: S2-2001353, S2-2001377, S2-2001538, S2-2001700 (S2-2001537rev1), S2-2001699 (S2-2001536rev3), S2-2001701 (S2-2001539rev3), S2-2001702 (S2-2001540rev3), S2-2001703 (S2-2001541rev2), S2-2001704 (S2-2001542rev3), S2-2001705 (S2-2001543rev4), S2-2001706 (S2-2001544rev1), S2-2001707 (S2-2001545rev7), S2-2001708 (S2-2001546rev5), S2-2001709 (S2-2001547rev3) | 0.3.0 |