| 3GPP TS 24.486 V0.4.0 (2020-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Vehicle-to-Everything (V2X) Application Enabler (VAE) layer; Protocol aspects; Stage 3 (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms and abbreviations	8

3.1	Terms	8

3.2	Abbreviations	8

4	General description	8

5	SEAL services	9

6	VAE procedures	9

6.1	General	9

6.2	V2X UE registration procedure	9

6.2.1	Client procedure	9

6.2.2	Server procedure	9

6.2.3	Abnormal cases	10

6.3	V2X UE de-registration procedure	10

6.3.1	Client procedure	10

6.3.2	Server procedure	10

6.4	Application level location tracking procedure	10

6.4.1	Client procedure	10

6.4.2	Server procedure	11

6.5	V2X message delivery procedure	11

6.5.1	Client procedure	11

6.5.1.1	Reception of a V2X message	11

6.5.1.2	Reception of a V2X message reception report	12

6.5.1.3	Sending of a V2X message reception report	12

6.5.1.4	Sending of a V2X message	12

6.5.2	Server procedure	13

6.5.2.1	Reception of a V2X message	13

6.5.2.2	Reception of a V2X message reception report	13

6.5.2.3	Sending of a V2X message reception report	13

6.5.2.4	Sending of a V2X message to target geografical areas	13

6.5.2.4	Sending of a V2X message to a V2X group	14

6.6	V2X service discovery procedure	14

6.6.1	Client procedure	14

6.6.2	Server procedure	14

6.7	V2X service continuity procedure	15

6.7.1	Client procedure	15

6.7.2	Server procedure	15

6.8	V2X application resource management procedure	15

6.9	File distribution procedure	15

6.10	Dynamic group management procedure	15

6.11	Network monitoring by the V2X UE procedure	15

7	Provisioning of parameters by the VAE server	16

7.1	General	16

7.2	V2X USD provisioning	16

7.3	PC5 parameters provisioning	16

8	Coding	16

8.1	General	16

8.2	Application unique ID	16

8.3	Structure	16

8.4	XML schema	17

8.5	Data semantics	17

8.6	MIME types	18

8.7	IANA registration template	18

Annex A (informative): Change history	19

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the protocols for application layer support for V2X services as specified in 3GPP TS 23.286 [3] for:

a)	V2X application communication among UEs (over the V5-AE interface); and

b)	V2X application communication between the UE and the V2X application server (over the V1-AE interface).

The present specification defines the associated procedures for V2X application communication between the UE and the V2X application server and among UEs.

The present specification defines the usage and interactions of the VAE layer with SEAL services.

The present specification also defines the message format, message contents, error handling and system parameters applied by the protocols for the VAE layer.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[3]	3GPP TS 23.286: "Application layer support for V2X services; Functional architecture and information flows".

[4]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[5]	3GPP TS 24.544: "Group Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

[6]	3GPP TS 24.545: "Location Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

[7]	3GPP TS 24.546: "Configuration Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

[8]	3GPP TS 24.547: "Identity Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

[9]	3GPP TS 24.548: "Network Resource Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

[10]	ETSI TS 102 965 (V1.4.1): "Intelligent Transport Systems (ITS); Application Object Identifier (ITS-AID); Registration".

[11]	ETSI EN 302 637-3: "Intelligent Transport Systems (ITS); Vehicular Communications; Basic Set of Applications; Part 3: Specifications of Decentralized Environmental Notification Basic Service".

[12]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

[13]	ISO TS 17419: "Intelligent Transport Systems - Cooperative systems - Classification and management of ITS applications in a global context".

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

V2X application enabler client: An entity that provides the client side functionalities corresponding to the V2X application enabler layer.

V2X application enabler server: An entity that provides the server side functionalities corresponding to the V2X application enabler layer.

V2X service identifier: An identifier of a V2X service, e.g. PSID or ITS-AIDs of the V2X application.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.286 [3] apply:

V2X service

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.434 [4] apply:

SEAL service

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

SEAL	Service Enabler Architecture Layer for Verticals

VAE	V2X Application Enabler

VAE-C	V2X Application Enabler Client

VAE-S	V2X Application Enabler Server

# 4	General description

The UE can contain a VAE client (VAE-C). The VAE-C communicates with the VAE server (VAE-S) over the V1-AE interface. Furthermore, the VAE-C of a UE can communicate with the VAE-C of another UE over the V5-AE interface.

The VAE layer supports UEs in the LTE-Uu communication range assigning a ProSe Layer-2 Group ID for application layer V2X dynamic group formation (on-network dynamic group creation procedure as defined in clause 6.10).

Additionally, the VAE layer supports UEs in assigning a ProSe Layer-2 Group ID for application layer V2X dynamic group formation (off-network dynamic group creation procedure as defined in clause 6.10).

By means of using the V1-AE interface:

a)	V2X UE registration and de-registration towards the VAE-S can be provided as defined by clause 6.2 and 6.3;

b) 	application level location tracking can be provided as defined by clause 6.4;

c)	V2X service delivery can be provided as defined by clause 6.5;

d)	V2X service discovery information can be provided as defined by clause 6.6;

e)	V2X service continuity can be provided as defined by clause 6.7;

f)	V2X application resource management can be provided as defined by clause 6.8;

g)	file distribution can be provided as defined by clause 6.9;

h)	dynamic local service information for V2X service continuity can be obtained as defined by clause 6.10;

i)	network monitoring by the V2X UE can be provided as defined by clause 6.11;

j)	V2X USD provisioning can be provided as defined by clause 7.2; and

k)	PC5 parameters provisioning can be provided as defined by clause 7.3.

# 5	SEAL services

The VAE layer utilizes SEAL services to support V2X services. The SEAL services are specified in 3GPP TS 24.544 [5], 3GPP TS 24.545 [6], 3GPP TS 24.546 [7], 3GPP TS 24.547 [8] and 3GPP TS 24.548 [9]. Interactions between the VAE layer and the SEAL services are described in detail in clause 6.

# 6	VAE procedures

## 6.1	General

## 6.2	V2X UE registration procedure

### 6.2.1	Client procedure

Upon receiving a request from a V2X application to register for receiving V2X messages from the V2X AS, the VAE client shall send an HTTP POST request according to procedures specified in IETF RFC 2616 [12]. In the HTTP POST request, the VAE client:

a)	shall set the Request-URI to the URI included in the received HTTP response message for V2X service discovery procedure (see clause 6.5);

b)	shall include a Content-Type header field set to "application/vnd.3gpp.VAE-registration-info+xml";

c)	shall include an application/vnd.3gpp.VAE-registration-info+xml MIME body and in the <registration-info> root element:

1)	shall include a <identity> element with a <V2X-UE-id> child element set to the identity of the UE which requests the registration; and

2)	shall include a <service> element with a <V2X-service-id> child element set to the identity of the V2X service which is interested in registering for receiving V2X messages.

### 6.2.2	Server procedure

Upon reception of an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.VAE-registration-info+xml"; and

b)	an application/vnd.3gpp.VAE-registration-+xml MIME body with a <registration-info> root element, the VAE server:

1)	shall store the received registration information; and

2)	shall reply with a HTTP response with a <result> element of the <registration-info> element set to a value "success" or "fail".

### 6.2.3	Abnormal cases

Editor’s note:	This clause will describe the abnormal cases for the V2X registration procedure.

## 6.3	V2X UE de-registration procedure

### 6.3.1	Client procedure

Upon receiving a request from a V2X application to de-register for receiving certain V2X message types from the V2X AS, the VAE client shall send an HTTP POST request according to procedures specified in IETF RFC 2616 [12]. In the HTTP POST request, the VAE client:

a)	shall set the Request-URI to the URI included in the received HTTP response message for registration request;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.VAE-de-registration-info+xml";

c)	shall include an application/vnd.3gpp.VAE-registration-info+xml MIME body and in the <de-registration-info> root element:

1)	shall include a <identity> element with a <V2X-UE-id> child element set to the identity of a UE which requests the registration; and

2)	shall include a <service> element with a <V2X-MSG-type> child element set to the types of V2X messages that the UE is no longer interested in receiving.

### 6.3.2	Server procedure

Upon reception of an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.VAE-de-registration-info+xml"; and

b)	an application/vnd.3gpp.VAE-registration-+xml MIME body with a <de-registration-info> root element, the VAE server:

1)	shall store the received registration information; and

2)	shall reply with a HTTP response.

## 6.4	Application level location tracking procedure

### 6.4.1	Client procedure

Upon entering a new geographical area if the V2X UE has been provisioned with geographical identifier groups (see clause 7) and the V2X UE has subscribed to a certain geographical area identifier group in order to receive V2X messages for this area, the VAE client shall send an HTTP POST request according to procedures specified in IETF RFC 2616 [12]. In the HTTP POST request, the VAE client:

a)	shall set the Request-URI to the URI included in the received HTTP response message for V2X UE registration procedure (see clause 6.2);

b)	shall include a Content-Type header field set to "application/vnd.3gpp.VAE-subscribe-location-tracking-info+xml";

c)	shall include an application/vnd.3gpp.VAE-location-tracking-info+xml MIME body and in the <location-tracking-info> root element:

1)	shall include a <identity> element with a <V2X-UE-id> child element set to the identity of the UE which requests the registration; and

2)	shall include a <geographical-identifier> element with a <geo-id> child element set to the identity of the geographical area to be subscribed.

Upon a successful subscription to a geographical area, the VAE client shall send an HTTP POST request according to procedures specified in IETF RFC 2616 [12]. In the HTTP POST request, the VAE client:

a)	shall set the Request-URI to the URI included in the received HTTP response message for the successful subscription of the geographical area;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.VAE-unsubscribe-location-tracking-info+xml";

c)	shall include an application/vnd.3gpp.VAE-location-tracking-info+xml MIME body and in the <location-tracking-info> root element:

1)	shall include a <identity> element with a <V2X-UE-id> child element set to the identity of the UE which requests the registration; and

2)	shall include a <geographical-identifier> element with a <geo-id> child element set to the identity of the geographical area to be unsubscribed.

### 6.4.2	Server procedure

Upon reception of an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.VAE-subscribe-location-tracking-info+xml"; and

b)	an application/vnd.3gpp.VAE-registration-+xml MIME body with a <location-tracking-info> root element, the VAE server:

1)	shall store the received geographical area information; and

2)	shall reply with a HTTP response with a <result> element of the <subscription-info> element set to a value "success" or "fail".

Upon reception of an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.VAE-unsubscribe-location-tracking-info+xml"; and

b)	an application/vnd.3gpp.VAE-registration-+xml MIME body with a <location-tracking-info> root element, the VAE server:

1)	shall remove the received geographical area information; and

2)	shall reply with a HTTP response with a <result> element of the <unsubscription-info> element set to a value "success" or "fail".

## 6.5	V2X message delivery procedure

### 6.5.1	Client procedure

#### 6.5.1.1	Reception of a V2X message

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.vae-message-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.vae-message-info+xml"; and

c)	an application/vnd.3gpp.vae-message-info+xml MIME body with either a <identity> element or a <group> element, a <payload>element and a <service> element included in the <message-info> root element;

the VAE client:

a)	shall provide the received information to the V2X application identified by the service indicated in the V2X message if the identity or group of theV2X message matches the identity of the V2X UE or the group of the VAE client; and

b)	shall send a V2X message reception report as specified in clause 6.5.1.3 if the <message-reception-ind> is included in the received V2X message.

#### 6.5.1.2	Reception of a V2X message reception report

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.vae-message-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.vae-message-info+xml"; and

c)	an application/vnd.3gpp.vae-message-info+xml MIME body with a <result> element included in the <message-info> root element;

the VAE client:

a)	shall provide the received result to the V2X application identified by the service indicated in the V2X message of the sent V2X message.

#### 6.5.1.3	Sending of a V2X message reception report

In order to send a V2X message reception report, the VAE client shall send a HTTP POST request message according to procedures specified in IETF RFC 2616 [12]. In the HTTP POST request message, the VAE client:

a)	shall set the Request-URI to the URI included in the received HTTP response message for reception of a V2X message (see clause 6.5.1.1);

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-message-info+xml"; and

c)	shall include an application/vnd.3gpp.vae-message-info+xml MIME body and a <result> element of the <message-info> element set to a value "success" or "fail".

#### 6.5.1.4	Sending of a V2X message

In order to send a V2X message, the VAE client shall send a HTTP POST request message according to procedures specified in IETF RFC 2616 [12]. In the HTTP POST request message, the VAE client:

a)	shall set the Request-URI to the URI included in the received HTTP response message for V2X service discovery procedure (see clause 6.5);

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-message-info+xml";

c)	shall include an application/vnd.3gpp.vae-message-info+xml MIME body and in the <message-info> root element:

1)	shall include a <identity> element with a <V2X-UE-id> child element set to the identity of the UE which requests the sending of the V2X message;

2)	shall include a <service> element with a <V2X-service-id> child element set to the identity of the V2X service which is interested in sending the V2X message.

3)	may include a <geographical-identifier> element with one or more <geo-id> child elements set to the identity of the geographical locations of the V2X UE; and

4)	may include a <message-reception-ind> element to indicate to the VAE server that a reception report is required.

### 6.5.2	Server procedure

#### 6.5.2.1	Reception of a V2X message

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.vae-message-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.vae-message-info+xml"; and

c)	an application/vnd.3gpp.vae-message-info+xml MIME body with either a <identity> element or a <group> element, a <payload>element and a <service> element included in the <message-info> root element;

the VAE server:

a)	shall provide the received information to the V2X application server identified by the service indicated in the V2X message; and

b)	shall send a V2X message reception report as specified in clause 6.5.2.3 if the <message-reception-ind> is included in the received V2X message.

#### 6.5.2.2	Reception of a V2X message reception report

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.vae-message-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.vae-message-info+xml"; and

c)	an application/vnd.3gpp.vae-message-info+xml MIME body with a <result> element included in the <message-info> root element;

the VAE servert:

a)	shall provide the received result to the V2X application server identified by the service indicated in the V2X message of the sent V2X message.

#### 6.5.2.3	Sending of a V2X message reception report

In order to send a V2X message reception report, the VAE server shall send a HTTP POST request message according to procedures specified in IETF RFC 2616 [12]. In the HTTP POST request message, the VAE server:

a)	shall set the Request-URI to the URI included in the received HTTP response message for reception of a V2X message (see clause 6.5.2.1);

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-message-info+xml"; and

c)	shall include an application/vnd.3gpp.vae-message-info+xml MIME body and a <result> element of the <message-info> element set to a value "success" or "fail".

#### 6.5.2.4	Sending of a V2X message to target geografical areas

In order to send a V2X message received from a V2X application server to target geographical areas, the VAE server shall send a HTTP POST request message according to procedures specified in IETF RFC 2616 [12]. In the HTTP POST request message, the VAE server:

a)	shall set the Request-URI to the URI included in the received HTTP response message for V2X service discovery procedure (see clause 6.5);

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-message-info+xml";

c)	shall include an application/vnd.3gpp.vae-message-info+xml MIME body and in the <message-info> root element:

1)	shall include a <identity> element with a <V2X-UE-id> child element set to the identity of the UE which requests the sending of the V2X message;

2)	shall include a <service> element with a <V2X-service-id> child element set to the identity of the V2X service which is interested in sending the V2X message.

3)	may include a <geographical-identifier> element with a <geo-id> child element set to the identity of the geographical location of of the V2X UE; and

4)	may include a <message-reception-ind> element to indicate to the VAE server that a reception report is required.

#### 6.5.2.4	Sending of a V2X message to a V2X group

In order to send a V2X message received from a V2X application server, the VAE server shall send a HTTP POST request message according to procedures specified in IETF RFC 2616 [12] to each VAE client which has subscribed to the V2X message delivery service. In the HTTP POST request message, the VAE server:

a)	shall set the Request-URI to the URI of each VAE client subscribed for V2X message delivery service;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-message-info+xml";

c)	shall include an application/vnd.3gpp.vae-message-info+xml MIME body and in the <message-info> root element:

1)	shall include a <identity> element with a <V2X-UE-id> child element set to the identity of the UE which requests the sending of the V2X message;

2)	shall include a <service> element with a <V2X-service-id> child element set to the identity of the V2X service which is interested in sending the V2X message.

3)	may include a <geographical-identifier> element with a <geo-id> child element set to the identity of the geographical location; and

4)	may include a <message-reception-ind> element to indicate to the VAE client that a reception report is required.

## 6.6	V2X service discovery procedure

### 6.6.1	Client procedure

In order to discover V2X service information from a VAE server (e.g., available VAE services identified by V2X service indetities), the VAE client shall send an HTTP POST request according to procedures specified in IETF RFC 2616 [12]. In the HTTP POST request, the VAE client:

a)	shall set the Request-URI to the URI provided by the SEAL layer;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.VAE-service-discovery-info+xml";

c)	shall include an application/vnd.3gpp.VAE-service-discovery-info+xml MIME body and in the <service-discovery-info> root element:

1)	shall include a <identity> element with a <V2X-UE-id> child element set to the identity of the UE which requests the registration.

### 6.6.2	Server procedure

Upon reception of an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.VAE-service-discovery-info+xml"; and

b)	an application/vnd.3gpp.VAE-registration-+xml MIME body with a <service-discovery-info> root element, the VAE server:

1)	shall reply with a HTTP response with a <result> element of the <service-discovery-info> element set to a value "success" or "fail", and amay include a <service-discovery-info> element which provides the V2X UE service discovery data.

## 6.7	V2X service continuity procedure

### 6.7.1	Client procedure

In order to obtaining dynamic local V2X service information from a VAE server, the VAE client shall send an HTTP POST request according to procedures specified in IETF RFC 2616 [12]. In the HTTP POST request, the VAE client:

a)	shall set the Request-URI to the URI included in the received HTTP response message for V2X service discovery procedure (see clause 6.6);

b)	shall include a Content-Type header field set to "application/vnd.3gpp.VAE-local-service-info+xml";

c)	shall include an application/vnd.3gpp.VAE-local-service- -info+xml MIME body and in the <local-service-info> root element:

1)	shall include a <identity> element with a <V2X-UE-id> child element set to the identity of the UE which requests the local service discovery information; and

2)	shall include a <geographical-identifier> element with a <geo-id> child element set to the identity of the geographical location for which the local service information is requested.

### 6.7.2	Server procedure

Upon reception of an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.VAE-local-service- info+xml"; and

b)	an application/vnd.3gpp.VAE-registration-+xml MIME body with a <local-service-info> root element, the VAE server:

1)	shall determines the local service information (e.g. V2X server USD(s), V2X USD) corresponding to the geographic location information received in <geographical-identifier>;

2)	shall reply with a HTTP response with a <result> element of the <local-service-info> element set to a value "success" or "fail", and if the result is "success", the VAE server shall include a <local-service-info-content> element which provides the local service information to the VAE client, and

## 6.8	V2X application resource management procedure

Editor’s note: This clause will describe the procedure for V2X application resource management based on TS 23.286.

## 6.9	File distribution procedure

Editor’s note: This clause will describe the procedure for file distribution based on TS 23.286.

## 6.10	Dynamic group management procedure

Editor’s note: This clause will describe the procedure for dynamic group management based on TS 23.286.

## 6.11	Network monitoring by the V2X UE procedure

Editor’s note: This clause will describe the procedure for network monitoring by the V2X UE based on TS 23.286.

# 7	Provisioning of parameters by the VAE server

## 7.1	General

The VAE server can provision network related information to a VAE client over the V1-AE interface:

a)	V2X USD provisioning in order to provision V2X USDs for receiving MBMS based V2X traffic; and

b)	PC5 parameters provisioning in order to provide PC5 parameters configuration data.

## 7.2	V2X USD provisioning

Editor’s note: This clause will provide description of V2X USD provisioning.

## 7.3	PC5 parameters provisioning

Editor’s note: This clause will provide description of PC5 parameters provisioning.

# 8	Coding

## 8.1	General

This clause specifies the coding to enable a VAE-C and a VAE-S to communicate.

## 8.2	Application unique ID

Editor’s note:	This clause will contain the application unique ID for VAE layer.

## 8.3	Structure

The VAE document shall conform to the XML schema described in clause 8.4.

The <VAE-info> element shall be the root element of the VAE document.

The <VAE-info> element shall include at least one of the following:

a)	an <identity> element;

b)	a <registration-info> element;

c)	a <de-registration-info> element;

d)	a <location-tracking-info> element;

e)	a <message-info> element; or

f)	a <service-discovery> element.

The <identity> element shall include a <V2X-UE-id> child element.

The <service-discovery> element shall include a <result> element and may include a <service-discovery-info> element.

The <service-discovery-info> element shall include an <identity> element.

The <registration-info> element shall include at least one of the following:

a)	an <identity> element;

b)	a <service> element; or

c)	a <result> element.

The <service> element shall include a <V2X-service-id> or a <V2X-MSG-type> child element.

The <de-registration-info> element shall include the following:

a)	an <identity> element; and

b)	a <service> element.

The <location-tracking-info> element shall include one of the following:

a)	an <identity> element shall include a <V2X-UE-id> element;

b)	a <geographical-identifier> element shall include a <geo-id> element; or

c)	a <result> element.

The <geographical-identifier> element shall include one or more <geo-id> elements which each shall include:

a)	a <polygon-area> element; and

b)	an <ellipsoid-arc-area> element.

The <message-info> element shall include at least one of the following:

a)	an <identity> element shall include a <V2X-UE-id> element;

b)	a <group> element shall include a <V2X-group-id>;

c) 	a <payload> element;

d)	a <service> element shall include a <V2X-service-id>;

e)	a <geographical-identifier> element shall include a <geo-id> element;

f)	a <message-reception-ind> element; or

g)	a <result> element.

The <group> element shall include a <V2X-group-id> child element.

## 8.4	XML schema

Editor’s note:	This clause will describe the XML schema for VAE layer.

## 8.5	Data semantics

The <VAE-info> element is the root element of the XML document. The <VAE-info> element contains the <identity>, <registration-info>, <de-registration-info>, <location-tracking.info>, <message-info> and <service-discovery> sub-elements.

<identity> is a mandatory element used to include the identity of a VAL client. The <identity> element contains a <V2X-UE-id> attribute that contains the identity of the VAL client.

The <registration-info> element contains the <result> sub-element and may include a <service-discovery-info> sub-element.

<result> is a mandatory element which indicates a value "success" or "fail".

<de-registration-info> is an optional element used to include the de-V2X registration information. The <de-registration-info> element contains the <identity> and <service> sub-elements.

<service> is a mandatory element used to include the types of V2X messages that the UE is no longer interested in receiving. The <service> element contains either a <V2X-service-id> attribute that contains one or more identifiers of V2X service identifiers as specified in ETSI TS 102 965 [10] and ISO TS 17419 [13] or a <V2X-MSG-type> attribute that contains one or more identifiers of a V2X service identifiers as specified in ETSI TS 102 965 [10] and ISO TS 17419 [13].

<service-discovery> is a mandatory element used to include the V2X service discovery response information. The <service-discovery-info> element contains an <identity> sub-element.

<geographical-identifier>, an optional element specifying one or more geographical area identifiers. This element consists of one or more <geo-id> elements. The <geo-id> element has the following sub-elements:

a)	<polygon-area>, an optional element specifying the area as a polygon specified in clause 5.2 of 3GPP TS 23.032 [2]; and

b)	<ellipsoid-arc-area>, an optional element specifying the area as an ellipsoid arc specified in clause 5.7 of 3GPP TS 23.032 [2].

<group> is an optional element used to include the identity of a VAL group. The <group> element contains a <V2X-group-id> attribute that contains the group identity of a set of VAL clients according to the VAL service.

<payload> is an optional element used to include the payload of the V2X message as specified in ETSI TS 102 965 [11].

<message-reception-ind> is an optional element used to indicate that a reception report is required to be sent.

## 8.6	MIME types

Editor’s note:	This clause will describe the MIME types.

## 8.7	IANA registration template

Editor’s note:	This clause will describe the IANA registration template.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-08 | CT1#119 | C1-194367 |  |  |  | Draft skeleton provided by the rapporteur. | 0.0.0 |
| 2019-09 | CT1#119 |  |  |  |  | Implementing the following p-CR agreed by CT1: C1-194368 | 0.1.0 |
| 2019-09 | CT1 e-mail review |  |  |  |  | Correction done by the rapporteur to the title of clause 3 | 0.1.1 |
| 2019-10 | CT1#120 |  |  |  |  | Implementing the following p-CRs agreed by CT1: C1-196373, C1-196376, C1-196618, C1-196859 | 0.2.0 |
| 2019-11 | CT1#121 |  |  |  |  | Implementing the following p-CRs agreed by CT1: C1-198550, C1-198624 Corrections done by the rapporteur. | 0.3.0 |
| 2020-03 | CT1#122-e |  |  |  |  | Implementing the following p-CRs agreed by CT1: C1-200530, C1-200532, C1-200533, C1-200622, C1-200623, C1-200624, C1-200903, C1-200905, C1-200906, C1-200944 Corrections done by the rapporteur. | 0.4.0 |