| 3GPP TS 24.545 V16.0.0 (2020-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Location Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification; (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms and abbreviations	8

3.1	Terms	8

3.2	Abbreviations	8

4	General description	8

5	Functional entities	8

5.1	SEAL location management client (SLM-C)	8

5.2	SEAL location management server (SLM-S)	9

6	Location management procedures	9

6.1	General	9

6.2	On-network procedures	9

6.2.1	General	9

6.2.1.1	Authenticated identity in HTTP request	9

6.2.2	Event-triggered location reporting procedure	9

6.2.2.1	General	9

6.2.2.2	Client procedure	9

6.2.2.2.1	Fetching location reporting configuration	9

6.2.2.2.2	Location reporting	10

6.2.2.3	Server procedure	10

6.2.2.3.1	Fetching location reporting configuration	10

6.2.2.3.2	Location reporting	11

6.2.3	On-demand location reporting procedure	11

6.2.3.1	Client procedure	11

6.2.3.2	Server procedure	12

6.2.4	Client-triggered or VAL server-triggered location reporting procedure	12

6.2.4.1	Client procedure	12

6.2.4.2	Server procedure	13

6.2.5	Location reporting event-triggered configuration cancel procedure	13

6.2.5.1	Client procedure	13

6.2.5.2	Server procedure	13

6.2.6	Location information subscription procedure	13

6.2.6.1	VAL server procedure	14

6.2.6.1.1	SIP based procedure	14

6.2.6.1.2	HTTP based procedure	14

6.2.6.2	Server procedure	15

6.2.6.2.1	SIP based procedure	15

6.2.6.2.2	HTTP based procedure	15

6.2.7	Event-triggered location information notification procedure	15

6.2.7.1	Client procedure	16

6.2.7.2	Server procedure	16

6.2.8	On-demand usage of location information procedure	16

6.2.8.1	VAL server procedure	16

6.2.8.2	Server procedure	17

6.2.9	Query list of users based on location	17

6.2.9.1	Client procedure	17

6.2.9.2	Server procedure	18

6.3	Off-network procedures	18

7	Coding	18

7.1	General	18

7.2	Application unique ID	19

7.3	Structure	19

7.4	XML schema	22

7.5	Data semantics	22

7.6	MIME types	28

7.7	IANA registration template	28

Annex A (informative): Change history	29

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the protocol aspects for the location management capability of SEAL to support vertical applications (e.g. V2X) over the 3GPP system.

The present document is applicable to the user equipment (UE) supporting the location management client functionality as described in 3GPP TS 23.434 [4], to the application server supporting the location management server functionality as described in 3GPP TS 23.434 [4] and to the application server supporting the vertical application server (VAL server) functionality as defined in the specific vertical application service (VAL service) specifications.

NOTE:	The specification of the VAL server for a specific VAL service is out of scope of present document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.003: "Numbering, addressing and identification".

[3]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[4]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[5]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[6]	3GPP TS 24.547: "Identity management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

[7]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

[8]	IETF RFC 3261 (June 2002): "SIP: Session Initiation Protocol".[9]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[10]	IETF RFC 6050 (November 2010): "A Session Initiation Protocol (SIP) Extension for the Identification of Services"

[11]	IETF RFC 6665 (July 2012): "SIP-Specific Event Notification".

[12]	OMA OMA-TS-XDM_Group-V1_1_1-20170124-A: "Group XDM Specification".

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

SEAL location management client: An entity that provides the client side functionalities corresponding to the SEAL location management service.

SEAL location management server: An entity that provides the server side functionalities corresponding to the SEAL location management service.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.434 [4] apply:

SEAL client

SEAL server

SEAL service

VAL server

VAL service

VAL user

Vertical

Vertical application

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

SCEF	Service Capability Exposure Function

SEAL	Service Enabler Architecture Layer for verticals

SLM-C	SEAL Location Management Client

SLM-S	SEAL Location Management Server

VAL	Vertical Application Layer

# 4	General description

Location management is a SEAL service that provides the location management related capabilities to one or more vertical applications. The present document enables a SEAL location management client (SLM-C) and a VAL server that communicate with a SEAL location management server (SLM-S).

# 5	Functional entities

## 5.1	SEAL location management client (SLM-C)

The SLM-C functional entity acts as the application client for location management related transactions. To be compliant with the procedures in the present document the SLM-C:

a)	shall support the role of XCAP client as specified in IETF RFC 4825 [9];

b)	shall support the role of XDMC as specified in OMA OMA-TS-XDM_Core-V2_1 [12]; and

c)	shall support the location management procedures in clause 6.2.

## 5.2	SEAL location management server (SLM-S)

The SLM-S is a functional entity used to provide location management supported within the vertical application layer. To be compliant with the procedures in the present document the SLM-S:

a)	shall support the role of XCAP server as specified in IETF RFC 4825 [9];

b)	shall support the role of XDMS as specified in OMA OMA-TS-XDM_Core-V2_1 [12]; and

c)	shall support the location management procedures in clause 6.2.

# 6	Location management procedures

## 6.1	General

## 6.2	On-network procedures

### 6.2.1	General

#### 6.2.1.1	Authenticated identity in HTTP request

Upon receiving an HTTP request, the SLM-S shall authenticate the identity of the sender of the HTTP request is authorized as specified in 3GPP TS 24.547 [6], and if authentication is successful, the SLM-S shall use the identity of the sender of the HTTP request as an authenticated identity.

### 6.2.2	Event-triggered location reporting procedure

#### 6.2.2.1	General

The SLM-C sends a location reporting configuration request when it needs to fetch location reporting configuration from the SLM-S.

The SLM-C sends a location report when at least one of the trigger criteria is fulfilled. To send the location report the SLM-C can use an appropriate HTTP request message.

If a location reporting trigger is met, the SLM-C checks if the minimum-report-interval timer is running. If the timer is running, the SLM-C waits until the timer expires. When the minimum-report-interval timer expires, the SLM-C:

a)	shall send a location information report as specified in clause 6.2.2.2, if any of the reporting triggers are still met.

Editor’s note:	It is FFS how the minimum-report-interval timer value is available in the UE.

#### 6.2.2.2	Client procedure

##### 6.2.2.2.1	Fetching location reporting configuration

In order to fetch location reporting configuration, the SLM-C shall send an HTTP GET request message according to procedures specified in IETF RFC 2616 [7] "Fetch a Document". In the HTTP GET request message, the SLM-C:

a)	shall set the Request-URI to the URI identifying the XML document to be fetched. In the Request-URI;

1)	the "auid" is set to specific VAL service identity; and

2)	the document selector is set to a document URI pointing to the location reporting configuration document; and

b)	shall set X-3GPP-Intended-Identity header to the VAL user identity requesting for location reporting configuration.

Editor’s notes:	The use of the X-3GPP-Intended-Identity header will be evaluated based on stage 2 security requirements.

Upon receiving an HTTP 200 (OK) response from the SLM-S containing:

a)	a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml"; and

b)	an application/vnd.3gpp.seal-location-info+xml MIME body with a <configuration> element included in the <location-info> root element;

the SLM-C:

a)	shall store the content of the <configuration> elements;

b)	shall set the location reporting triggers accordingly; and

c)	shall start the minimum-report-interval timer.

##### 6.2.2.2.2	Location reporting

In order to report the location information, the SLM-C shall send a HTTP POST request message according to procedures specified in IETF RFC 2616 [7]. In the HTTP POST request message, the SLM-C:

a)	shall set the Request-URI to the URI included in the received HTTP response message for location report configuration;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml";

c)	shall include an application/vnd.3gpp.seal-location-info+xml MIME body and in the <location-info> root element:

1)	shall include a <identity> element with a <VAL-user-id> child element set to the identity of the VAL user for location report; and

2)	shall include a <report> element and, if the report was triggered by a location request, include the <report-id> attribute set to the value of the <request-id> attribute in the received request. The <report> element:

i)	shall include a <trigger-id> child element set to the value of each <trigger-id> value of the triggers that have been met; and

ii)	shall include the location reporting elements corresponding to the triggers that have been met;

d)	shall set the minimum-report-interval timer to the minimum-report-interval time and start this timer; and

e)	shall reset all the trigger criteria for location reporting.

#### 6.2.2.3	Server procedure

#### 6.2.2.3.1	Fetching location reporting configuration

Upon receiving of an HTTP GET request where the Request-URI of the HTTP GET request identifies a location reporting configuration document as specified in the specific vertical application, the SLM-S:

a)	shall determine the identity of the sender of the received HTTP GET request as specified in clause 6.2.1.1, and:

1)	if the identity of the sender of the received HTTP GET request is not authorized to fetch requested configuration document, shall respond with a HTTP 403 (Forbidden) response to the HTTP GET request and skip rest of the steps;

b)	shall support handling an HTTP GET request from a SLM-C according to procedures specified in IETF RFC 4825 [9] "GET Handling".

c) 	shall generate an HTTP 200 (OK) response according to IETF RFC 2616 [7]. In the HTTP 200 (OK) response message, the SLM-S:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml";

2)	shall include an application/vnd.3gpp.seal-location-info+xml MIME body and in the <location-info> root element:

i)	shall include an <identity> element with a <VAL-user-id> child element set to the identity of the VAL user requesting for location reporting configuration;

ii)	shall include a <configuration> element which shall include at least one of the followings:

A)	the location reporting elements which are requested;

B)	a <triggering-criteria> child element which provides the triggers for the SLM-C to request a location report as described in clause 7; and

C)	a <minimum-interval-length>child element specifying the minimum time between consecutive reports. The value is given in seconds;

3)	shall include the <trigger-id> attribute where defined for the sub-elements defining the trigger criterion; and

d)	shall send the HTTP 200 (OK) response towards the SLM-C.

##### 6.2.2.3.2	Location reporting

Upon reception of an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml"; and

b)	an application/vnd.3gpp.seal-location-info+xml MIME body with a <report> element included in the <location-info> root element;

where the Request-URI of the HTTP POST request identifies an element of a XML document as specified in application usage of the specific vertical application, the SLM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 6.2.1.1; and

1)	if the identity of the sender of the received HTTP POST request is not authorized to obtain location information of another VAL user, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps; and

2)	shall support handling an HTTP POST request from a SLM-C according to procedures specified in IETF RFC 4825 [9] where the Request-URI of the HTTP POST request identifies an element of XML document as specified in application usage of the specific vertical application. The SLM-S:

i)	shall store the received location information of the reporting SLM-C; and

ii)	shall use the location information as needed.

NOTE:	The <report> element contains the event triggering identity in the location information report from the VAL client, and can contain location information.

### 6.2.3	On-demand location reporting procedure

#### 6.2.3.1	Client procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-location-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml";

c)	an application/vnd.3gpp.seal-location-info+xml MIME body with a <request> element included in the <location-info> root element;

the SLM-C:

a)	may send a location report as specified in clause 6.2.2.2.2.

#### 6.2.3.2	Server procedure

If the SLM-S needs to request the SLM-C to report its location, the SLM-S shall generate an HTTP POST request according to procedures specified in IETF RFC 2616 [7]. The SLM-S:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SLM-C;

b)	shall include an Accept header field set to "application/vnd.3gpp.seal-location-info+xml";

c)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml";

d)	shall include an application/vnd.3gpp.seal-location-info+xml MIME body and in the <location-info> root element:

1)	shall include a <requested-identity> element with a <VAL-user-id> child element set to the identity of the VAL user whose location is requested;

2)	shall include a <request> element; and

e)	shall send the HTTP POST request as specified in IETF RFC 2616 [7].

### 6.2.4	Client-triggered or VAL server-triggered location reporting procedure

#### 6.2.4.1	Client procedure

Upon receiving a request from a VAL user to obtain the location information of another VAL user, the SLM-C shall send an HTTP POST request according to procedures specified in IETF RFC 2616 [7]. In the HTTP POST request, the SLM-C:

a)	shall set the Request-URI to the URI included in the received HTTP response message for location report configuration;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml"; and

c)	shall include an application/vnd.3gpp.seal-location-info+xml MIME body and in the <location-info> root element:

1)	shall include a <identity> element with a <VAL-user-id> child element set to the identity of the VAL user which requests the location report;

2)	shall include a <requested-identity> element with a <VAL-user-id> child element set to the identity of the VAL user for which a location report is requested. The VAL user should belong to the same VAL service as the identity of the VAL user which requests the location report; and

3)	a <report-request> element which shall include at least one of the followings:

i)	a <immediate-report-indicator> child element to indicate that an immediate location report is required;

ii)	the location reporting elements which are requested;

iii)	a <triggering-criteria> child element which indicate a specified location trigger criteria to send the location report; and

iv)	a <minimum-interval-length>child element specifying the minimum time between consecutive reports. The value is given in seconds.;

#### 6.2.4.2	Server procedure

Upon reception of an HTTP POST request where the Request-URI of the HTTP POST request identifies an element of a XML document as specified in application usage of the specific vertical application, the SLM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 6.2.1.1 and;

1)	if the identity of the sender of the received HTTP POST request is not authorized to obtain location information of another VAL user, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps; and

2)	shall support handling an HTTP POST request from a SLM-C according to procedures specified in IETF RFC 4825 [9] where the Request-URI of the HTTP POST request identifies an element of XML document as specified in application usage of the specific vertical application. Depending on the information specified by the HTTP POST request, the SLM-S initiates either an event-triggered location reporting procedure as specified in clause 6.2.2.2 or an on-demand location reporting procedure as specified in clause 6.2.2.3 for providing the SLM-C with the location of the requested VAL user.

### 6.2.5	Location reporting event-triggered configuration cancel procedure

#### 6.2.5.1	Client procedure

Upon receiving an HTTP POST request containing:

a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml"; and

b)	an application/vnd.3gpp.seal-location-info+xml MIME body with a <configuration> element included in the <location-info> root element, which has none of child elements;

the SLM-C:

a)	shall delete the content of the <configuration> elments;

b)	shall stop the location reporting; and

c)	shall generate an HTTP 200 (OK) response to the received HTTP POST request message according to IETF RFC 2616 [7].

#### 6.2.5.2	Server procedure

The SLM-S may cancel the location reporting event triggers configuration in the SLM-C by generatiing an HTTP POST request message according to procedures specified in IETF RFC 2616 [7]. The SLM-S:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SLM-C;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml";

c)	shall include an application/vnd.3gpp.seal-location-info+xml MIME body and in the <location-info> root element:

1)	shall include an <identity> element with a <VAL-user-id> child element set to the identity of the VAL user for location reporting event triggers configuration cancellation;

2)	shall include a <configuration> element which shall not include any child element; and

d)	shall send the HTTP POST request as specified in IETF RFC 2616 [7].

### 6.2.6	Location information subscription procedure

NOTE:	The VAL service will use the same identity which has been authenticated by VAL service with SIP core using SIP based REGISTER message. If VAL service do not support SIP protocol, then HTTP based method needs to be used.

#### 6.2.6.1	VAL server procedure

##### 6.2.6.1.1	SIP based procedure

In order to subscribe location information of one or more VAL users or VAL UEs, if VAL server supports SIP,  the VAL server shall generate an initial SIP SUBSCRIBE request according to 3GPP TS 24.229 [5] and IETF RFC 6665 [11]. In the SIP SUBSCRIBE request, the VAL server:

a)	shall set the Request-URI to the public service identity identifying the originating SLM-S serving the VAL server;

b)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.seal" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [10];

c)	shall set the Event header field to the 'seal-location-info' value;

Editor’s note: the details of the event package ‘seal-location-info’ is FFS.

d)	shall include an application/vnd.3gpp.seal-location-info+xml MIME body and in the <location-info> root element;

1)	shall include an <identity> element with a <VAL-user-id> child element set to the identity of the VAL server which requests the location information subscription;

2)	shall include a <subscription> element which shall include:

i)	an <identities-list> element with one or more  <VAL-user-id> child elements set to the identities of the VAL users whose location information is requested;

ii)	an <interval-length> element specifying the time between consecutive reports. The value is given in seonds;

e)	if the VAL server wants to receive the current status and later notification, shall set the Expires header field according to IETF RFC 6665 [11], to 4294967295; and

NOTE:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [8].

f)	if the VAL server wants to fetch the current state only, shall set the Expires header field according to IETF RFC 6665 [11], to zero.

##### 6.2.6.1.2	HTTP based procedure

If VAL server does not support SIP, the VAL server shall send an HTTP POST request to the SLM-S according to procedures specified in IETF RFC 2616 [7]. In the HTTP POST request message, the VAL server:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SLM-S;

b)	shall include an Accept header field set to "application/vnd.3gpp.seal-location-info+xml";

c)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml";

d)	shall include an application/vnd.3gpp.seal-location-info+xml MIME body and in the <location-info> root element;

1)	shall include an <identity> element with a <VAL-user-id> child element set to the identity of the VAL server which requests the location information subscription; and

2)	shall include a <subscription> element which shall include:

i)	an <identities-list> element with one or more  <VAL-user-id> child elements set to the identities of the VAL users whose location information is requested; and.

ii)	an <interval-length> element specifying the time between consecutive reports. The value is given in seonds.

#### 6.2.6.2	Server procedure

##### 6.2.6.2.1	SIP based procedure

Upon receiving a SIP SUBSCRIBE request such that:

a)	Request-URI of the SIP SUBSCRIBE request contains the public service identity identifying the SLM-S of the served VAL server;

b)	the ICSI value "urn:urn-7:3gpp-service.ims.icsi.seal" (coded as specified in 3GPP TS 24.229 [5]), in a P-Asserted-Service header field according to IETF RFC 6050 [10];

c)	the Event header field of the SIP SUBSCRIBE request contains the 'seal-location-info' event type; and

d)	the SIP SUBSCRIBE request contains an application/vnd.3gpp.seal-location-info+xml MIME body with an <subscription> element included in the <location-info> root element;

the SLM-S:

a)	shall identify the served VAL user ID in the <identity> element of the application/ vnd.3gpp.seal-location-info+xml MIME body of the SIP SUBSCRIBE request;

b)	if the Request-URI of the SIP SUBSCRIBE request contains the public service identity identifying the SLM-S serving the VAL server, shall identify the originating VAL user ID from public user identity in the P-Asserted-Identity header field of the SIP SUBSCRIBE request;

c)	if the originating VAL user ID is different than the served VAL user ID, shall send a 403 (Forbidden) response and shall not continue with the rest of the steps; and

d)	shall generate a 200 (OK) response to the SIP SUBSCRIBE request according to 3GPP TS 24.229 [5], IETF RFC 6665 [11].

##### 6.2.6.2.2	HTTP based procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-location-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml";

c)	an application/vnd.3gpp.seal-location-info+xml MIME body with a <subscription> element included in the <location-info> root element;

the SLM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 6.2.1.1; and

1)	if the identity of the sender of the received HTTP POST request is not authorized to subscribe location information of another VAL user or VAL UE, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps; and

b)	shall support handling an HTTP POST request from a SLM-C according to procedures specified in IETF RFC 4825 [9] "POST Handling";

c)	shall generate an HTTP 200 (OK) response according to IETF RFC 2616 [7].

### 6.2.7	Event-triggered location information notification procedure

NOTE:	The SLM-C will use the same identity which has been authenticated by VAL service with SIP core using SIP based REGISTER message. If VAL service do not support SIP protocol, then HTTP based method needs to be used.

#### 6.2.7.1	Client procedure

Upon receiving a SIP NOTIFY request containing an application/vnd.3gpp.seal-location-info+xml MIME body with a <notification> element included in the <location-info> root element, or an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml"; and

b)	an application/vnd.3gpp.seal-location-info+xml MIME body with a <notification> element included in the <location-info> root element;

the SLM-C:

a)	shall store the received location information; and

b)	may share the information to a group or to another VAL user or VAL UE.

#### 6.2.7.2	Server procedure

In order to nitify the subscriber about the location information report, the SLM-S:

a)	shall generate an application/vnd.3gpp.seal-location-info+xml MIME body containing:

1)	an <identity> element with a <VAL-user-id> child element set to the identity of the VAL user which subscribed to location of another VAL user or VAL UE; and

2)	a <notification> element which shall include:

i)	an <identities-list> element with one or more <VAL-user-id> child elements set to the identities of the VAL users whose location information needs to be notified;

ii)	a <trigger-id> element set to the value of each <trigger-id> value of the triggers that have been met; and

iii)	a <reports> element containing one or more <loc-info-report> elements. The <loc-info-report> shall include:

A)	a <VAL-user-id> element set to the identity of the VAL user whose location information needs to be notified; and

B)	the latest location information corresponding to the VAL user; and

b)	if SLM-C supports SIP, shall send a SIP NOTIFY request according to 3GPP TS 24.229 [5] and IETF RFC 6665 [11] with the constructed application/vnd.3gpp.seal-location-info+xml MIME body;

c)	if SLM-C does not support SIP, shall send an HTTP POST request message to the SLM-C according to procedures specified in IETF RFC 2616 [7] with the constructed application/vnd.3gpp.seal-location-info+xml MIME body and an Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml".

### 6.2.8	On-demand usage of location information procedure

#### 6.2.8.1	VAL server procedure

If the VAL server needs to request UE location information, the VAL server shall send an HTTP POST request to the SLM-S according to procedures specified in IETF RFC 2616 [7]. In the HTTP POST request message, the VAL server:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SLM-S;

b)	shall include an Accept header field set to "application/vnd.3gpp.seal-location-info+xml";

c)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml";

d)	shall include an application/vnd.3gpp.seal-location-info+xml MIME body and in the<location-info> root element:

1)	shall include an <identity> element with a <VAL-user-id> child element set to the identity of the VAL server which requests the location information; and

2)	shall include an <identities-list> element with one or more  <VAL-user-id> child elements set to the identities of the VAL users whose location information is requested;

Upon receiving an HTTP 200 (OK) response from the SLM-S containing:

a)	a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml"; and

b)	an application/vnd.3gpp.seal-location-info+xml MIME body with a <reports> element included in the <location-info> root element;

the VAL server:

a)	shall store the received location information; and

b)	may share the information to a group or to another VAL user or VAL UE.

#### 6.2.8.2	Server procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-location-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml";

c)	an application/vnd.3gpp.seal-location-info+xml MIME body with an < identities-list > element included in the <location-info> root element;

the SLM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 6.2.1.1; and

1)	if the identity of the sender of the received HTTP POST request is not authorized to obtain location information of another VAL user, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps; and

b)	shall support handling an HTTP POST request from a SLM-C according to procedures specified in IETF RFC 4825 [9] "POST Handling";

c)	shall generate an HTTP 200 (OK) response according to IETF RFC 2616 [7]. In the HTTP 200 (OK) response message, the SLM-S:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml";

2)	shall include an application/vnd.3gpp.seal-location-info+xml MIME body and in the <location-info> root element:

i)	shall include an <identity> element with a <VAL-user-id> child element set to the identity of the VAL user for location reporting configuration;

ii)	an <identities-list> element with one or more  <VAL-user-id> child elements set to the identities of the VAL users whose location information is requested;

iii)	a <reports> element containing one or more <loc-info-report> elements. The <loc-info-report> contains a <VAL-user-id> element set to the identity of the VAL user in the requested-identity-list and the latest location information corresponding to the VAL user; and

d)	shall send an HTTP 200 (OK) response towards the VAL server.

### 6.2.9	Query list of users based on location

#### 6.2.9.1	Client procedure

The procedure defined in this clause can be used by SEAL server to query list of users based on given geolocation area.

In order to query the list of users based on given geolocation area, the client shall send an HTTP POST request message according to procedures specified in IETF RFC 2616 [7]. In the HTTP POST request message, the SLM-C:

a)	shall set the Request-URI to the URI corresponding to the identity of the SEAL server;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml"; and

c)	shall include an application/vnd.3gpp.seal-location-info+xml MIME body and in the <location-info> root element:

1)	shall include an <identity> element with a <VAL-user-id> child element set to the identity of the SEAL server querying list of users; and

2)	shall include an <location-based-query> element with a <polygon-area> child element or an <ellipsoid-arc-area> child element.

#### 6.2.9.2	Server procedure

Upon reception of an HTTP POST request containing:

a)	a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml"; and

b)	an application/vnd.3gpp.seal-location-info+xml MIME body with a < location-based-query> element included in the <location-info> root element;

the SLM-S:

a)	shall authorize the identity of the sender of the received HTTP POST request; and

1)	if the identity of the sender of the received HTTP POST request is not authorized to obtain list of users based on given geolocation area, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps;

b)	shall generate the list of users who are currently available in requested geographical area; and

c)	shall send an HTTP 200 (OK) response message to SLM-C. In the HTTP 200 (OK) response message, the SLM-S:

1)	 shall generate an application/vnd.3gpp.seal-location-info+xml MIME body containing:

i)	an <identity> element with a <VAL-user-id> child element set to the identity of the SEAL server querying list of users; and

ii)	a <location-based-response> element which shall include:

A)	an <identities-list> element with one or more <VAL-user-id> child elements set to the identities of the VAL users to be queried;

## 6.3	Off-network procedures

The off-network procedures are out of scope of the present document in this release of the specification.

# 7	Coding

## 7.1	General

This clause specifies the coding to enable an SLM-C and an SLM-S to communicate.

## 7.2	Application unique ID

Editor’s note:	This clause will contain the application unique ID for location management.

## 7.3	Structure

The location management document shall conform to the XML schema described in clause 7.4.

The <location-info> element shall be the root element of the SEALLocationManagement document.

The <location-info> element shall include at least one of the following:

a)	an <identity> element;

b)	a <subscription> element;

c)	a <notification> element;

d)	a <report> element;

e)	a <configuration> element; or

f) 	a <request> element.

The <identity> element shall include one of the following:

a)	a <VAL-user-id> element may include a <VAL-client-id> element; or

b)	a <VAL-group-id> element.

The <subscription> element shall include:

a)	an <identities-list> element which shall include:

1)	one or more <VAL-user-id> elements; and

b)	an <interval-length> element;

The <notification> element shall include:

a)	an <identities-list> element which shall include:

1)	one or more <VAL-user-id> elements;

b)	a <trigger-id> element; and

c)	a <reports> element containing one or more <loc-info-report> elements. The <loc-info-report> element shall include:

1)	a <VAL-user-id> element;

2)	a <latest-location> element, which shall include at least one of the following sub-elements:

i)	a <latest-serving-NCGI> element;

ii)	a <neighbouring-NCGI> element;

iii)	an <mbms-service-area-id> element;

iv)	an <mbsfn-area> element; or

v)	a <latest-coordinate> element;

The <report> element shall contain a <report-id> attribute. The <report> shall include:

a)	a <trigger-id> element; and

b)	a <current-location> element which shall include at least one of the following:

1)	a <current-serving-NCGI> element;

2)	a <neighbouring-NCGI> element;

3)	a <mbms-service-area-id> element; or

4)	a <current-coordinate> element.

The <configuration> element includes:

a)	a <location-information> element including:

1)	a <current-serving-NCGI> element;

2)	a <neighbouring-NCGI> element;

3)	an <mbms-service-area-id> element;

4)	an <mbsfn-area-id> element; or

5)	a <current-geographical-coordinate> element;

b)	a <triggering-criteria> element shall include at least one of the following sub-elements:

1)	a <cell-change> element shall include one of the following sub-elements:

i)	an <any-cell-change> element shall include a <trigger-id> element;

ii)	an <enter-specific-cell> element shall include a <trigger-id> element; and

iii)	an <exit-specific-cell> element include a <trigger-id> element;

2)	a <tracking-area-change> element shall include one of the following sub-elements:

i)	an <any-tracking-area-change> element shall include a <trigger-id> element;

ii)	an <enter-specific-tracking-area> element shall include a <trigger-id> element; and

iii)	an <exit-specific-trackin-area> element shall include a <trigger-id> element;

3)	a <plmn-change> element shall include one of the following sub-elements:

i)	an <any-plmn-change> element shall include a <trigger-id> element;

ii)	an <enter-specific-plmn>element shall include a <trigger-id> element; and

iii)	an <exit-specific-plmn> element shall include a <trigger-id> element;

4)	an <mbms-sa-change> element shall include one of the following sub-elements:

i)	an <any-mbms-sa-change> element shall include a <trigger-id> element;

ii)	an <enter-specific-mbms-sa> element shall include a <trigger-id> element; and

iii)	an <exit-specific-mbms-sa> element shall include a <trigger-id> element;

5)	an <mbsfn-area-change> element shall include one of the following sub-elements:

i)	an <any-mbsfn-area-change> element shall include a <trigger-id> element;

ii)	an <enter-specific-mbsfn-area> element shall include a <trigger-id> element; and

iii)	an <exit-specific-mbsfn-area> element shall include a <trigger-id> element;

6)	a <periodic-report> element shall include a <trigger-id> element;

7)	a <travelled-distance> element shall include a <trigger-id> element;

8)	a <vertical-application-event> element shall include one of the following sub-elements:

i)	an <initial-log-on> element shall include a <trigger-id> element;

ii)	a <location-configuration-received> element shall include a <trigger-id> element; and

iii)	an <any-other-event>, an optional element specifying that any other application signalling event than initial-log-on and location-configuration-received triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

9)	a <geographical-area-change> element shall include one of the following sub-elements:

i)	an <any-area-change> element shall include a <trigger-id> element;

ii)	an <enter-specific-area> element shall include the following sub-element:

A)	a <geographical-area> element shall include the following two sub-elements:

I)	a <polygon-area> element shall include a <trigger-id> element; and

II)	an <ellipsoid-arc-area> element shall include a <trigger-id> element;

iii)	an <exit-specific-area-type> element shall include a <trigger-id> element;c)	a <minimum-interval-length> element;

The <request> shall contain a <request-id> attribute.

The <requested-identity> element shall include one of the following sub-elements:

a)	a <VAL-user-id> element may include a <VAL-client-id> element; or

b)	a <VAL-group-id> element.

The <report-request> element shall include at leat one of the following sub-elements:

a)	an <immediate-report-indicator > element; and

b)	a <current-location> element which shall include at least one of the following sub-elements:

1)	a <current-serving-NCGI> element;

2)	a <neighbouring-NCGI> element;

3)	an <mbms-service-area-id> element; or

4)	a <current-coordinate> element;

c)	a <triggering-criteria> element shall include at least one of the following sub-elements:

1)	a <cell-change> element shall include one of the following sub-elements:

i)	an <any-cell-change> element shall include a <trigger-id> element;

ii)	a <enter-specific-cell> element shall include a <trigger-id> element; and

iii)	an <exit-specific-cell> element include a <trigger-id> element;

2)	a <tracking-area-change> element shall include one of the following sub-elements:

i)	an <any-tracking-area-change> element shall include a <trigger-id> element;

ii)	an <enter-specific-tracking-area> element shall include a <trigger-id> element; and

iii)	an <exit-specific-trackin-area> element shall include a <trigger-id> element;

3)	a <plmn-change> element shall include one of the following sub-elements:

i)	an <any-plmn-change> element shall include a <trigger-id> element;

ii)	an <enter-specific-plmn>element shall include a <trigger-id> element; and

iii)	an <exit-specific-plmn> element shall include a <trigger-id> element;

4)	an <mbms-sa-change> element shall include one of the following sub-elements:

i)	an <any-mbms-sa-change> element shall include a <trigger-id> element;

ii)	an <enter-specific-mbms-sa> element shall include a <trigger-id> element; and

iii)	an <exit-specific-mbms-sa> element shall include a <trigger-id> element;

5)	an <mbsfn-area-change> element shall include one of the following sub-elements:

i)	an <any-mbsfn-areaChange> element shall include a <trigger-id> element;

ii)	an <enter-specific-mbsfn-area> element shall include a <trigger-id> element; and

iii)	an <exit-specific-mbsfn-area> element shall include a <trigger-id> element;

6)	a <periodic-report> element shall include a <trigger-id> element;

7)	a <travelled-distance> element shall include a <trigger-id> element;

8)	a <vertical-application-event> element shall include one of the following sub-elements:

i)	an <initial-log-on> element shall include a <trigger-id> element;

ii)	a <location-configuration-received> element shall include a <trigger-id> element; and

iii)	an <any-other-event>, an optional element specifying that any other application signalling event than initial-log-on and location-configuration-received triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

9)	a <geographical-area-change> element shall include one of the following sub-elements:

i)	an <any-area-change> element shall include a <trigger-id> element;

ii)	an <enter-specific-area> element shall include the following sub-element:

A)	a <geographical-area> element shall include the following two sub-elements:

I)	a <polygon-area> element shall include a <trigger-id> element; and

II)	an <ellipsoid-arc-area> element shall include a <trigger-id> element;

iii)	an <exit-specific-area-type> element shall include a <trigger-id> element.

## 7.4	XML schema

Editor’s note:	This clause will describe the XML schema for location management.

## 7.5	Data semantics

The <location-info> element is the root element of the XML document. The <location-info> element contains the <identity>, <subscription>, <request>, <configuration> and <report> sub-elements.

<identity> is a mandatory element used to include the identity of a VAL user, a VAL client or a VAL group. The <identity> element contains one of following sub-elements:

a)	<VAL-user-id>, an element contains the identity of the VAL user. This element contains an optional <VAL-client-id> attribute that contains the identity of the VAL client; or

b)	<VAL-group-id>, an element contains the group identity of a set of VAL users or VAL clients according to the VAL service.

<subscription> contains the following sub-elements:

a)	<identities-list>, an element contains one or more <VAL-user-id> elements. Each <VAL-user-id> element contains the identity of the VAL user whose location information is requested.

b)	<interval-length>, an element specifying the interval time the SLM-S needs to wait before sending location reports. The value is given in seconds.

<notification> contains the following sub-elements:

a)	<identities-list>, an element contains one or more <VAL-user-id> elements. Each <VAL-user-id> element contains the identity of the VAL user whose location information needs to be notified.

b)	<trigger-id>, an element which can occur multiple times that contains the value of the <trigger-id> attribute associated with a trigger that has fired; and

c)	<reports>, an element contains one or more <loc-info-report> elements. Each <loc-info-report> element contains the following sub-elements:

1)	<VAL-user-id>, an element contains the identity of a VAL user in the identities list;

2)	<latest-location >, an element contains at least one of the following sub-elements:

i)	<latest-serving-NCGI>, an optional element containing the NR cell global identity (NCGI) of the serving cell coded as specified in clause 19.6A in 3GPP TS 23.003 [2];

ii)	<neighbouring-NCGI>, an optional element that can occur multiple times. It contains the NCGI of any neighbouring cell the SLM-C can detect;

iii)	<mbms-service-area-id>, an optional element containing the MBMS service area id the SLM-C is using coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI);

iv)	<mbsfn-area> element, an optional element specifying that the MBSFN area Id needs to be reported; and

v)	<latest-coordinate>, an optional element containing the longitude and latitude coded as specified in clause 6.1 in 3GPP TS 23.032 [3];

1)	<report> is a mandatory element used to include the location report. It contains a <report-id> attribute. The <report-id> attribute is used to return the value in the <request-id> attribute in the <request> element. The <report> element contains the following sub-elements:

a)	<trigger-id>, a mandatory element which can occur multiple times that contain the value of the <trigger-id> attribute associated with a trigger that has fired; and

b)	<current-location>, a mandatory element that contains the location information. The <current-location> element contains the following sub-elements:

1)	<current-serving-NCGI>, an optional element containing the NR cell global identity (NCGI) of the serving cell coded as specified in clause 19.6A in 3GPP TS 23.003 [2];

2)	<neighbouring-NCGI>, an optional element that can occur multiple times. It contains the NCGI of any neighbouring cell the SLM-C can detect;

3)	<mbms-service-area-id>, an optional element containing the MBMS service area id the SLM-C is using coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI); and

4)	<current-coordinate>, an optional element containing the longitude and latitude coded as specified in clause 6.1 in 3GPP TS 23.032 [3].

<request> is an element with a <request-id> attribute. The <request> element is used to request a location report. The value of the <request-id> attribute is returned in the corresponding <report-id> attribute in order to correlate the request and the report.

<requested-identity> is a mandatory element used to include the identity of a VAL user, a VAL client or a VAL group for which a location report is requested. The <requested-identity> element contains one of following sub-elements:

a)	<VAL-user-id>, an element contains the identity of the VAL user. This element contains an optional <VAL-client-id> attribute that contains the identity of the VAL client; or

b)	<VAL-group-id>, an element contains the group identity of a set of VAL users or VAL clients according to the VAL service.

<configuration> is an element with a <configuration-scope> attribute that can have the value "Full" or "Update" . The value "Full" means that the <configuration> element contains the full location configuration which replaces any previous location configuration. The value "Update" means that the location configuration is an addition to any previous location configuration. To remove configuration elements a "Full" configuration is needed. The <configuration> element contains the following sub-elements:

a)	<location-information>, an optional element that specifies the location information. The <location-information> has the sub-elements:

1)	<serving-NCGI>, an optional element containing the NR cell global identity (NCGI) of the serving cell coded as specified in clause 19.6A in 3GPP TS 23.003 [2];

2)	<neighbouring-NCGI>, an optional element that can occur multiple times. It contains the NCGI of any neighbouring cell the SLM-C can detect;

3)	<mbms-service-area-id>, an optional element containing the MBMS service area id that the SLM-C is using. The MBMS service area id is coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI);

4)	<mbsfn-area-id>, an optional element specifying that the MBSFN area id that needs to be reported;

5)	<current-geographical-coordinate>, an optional element containing the longitude and latitude coded as specified in clause 6.1 in 3GPP TS 23.032 [3]; and

b)	<triggering-criteria>, an optional element specifying the triggers for the SLM-C to request a location report of a VAL user, a VAL client or a VAL group. The <triggering-criteria> element contains at least one of the following sub-elements:

1)	<cell-change>, an optional element specifying what cell changes trigger the request for a location report. This element consists of the following sub-elements:

i)	<any-cell-change>, an optional element. The presence of this element specifies that any cell change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-cell>, an optional element specifying an NCGI which when entered triggers a request for alocation report coded as specified in clause 19.6A in 3GPP TS 23.003 [2]. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-cell>, an optional element specifying an NCGI which when exited triggers a request for a location report coded as specified in clause 19.6A in 3GPP TS 23.003 [2]. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

2)	<tracking-area-change>, an optional element specifying what tracking area changes trigger a request for a location report. This element consists of the following sub-elements:

i)	<any-tracking-area-change>, an optional element. The presence of this element specifies that any tracking area change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-tracking-area>, an optional element specifying a tracking area identity coded as specified in clause 19.4.2.3 in 3GPP TS 23.003 [2] which when entered triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-tracking-area>, an optional element specifying a tracking area identity coded as specified in clause 19.4.2.3 in 3GPP TS 23.003 [2] which when exited triggers a request for alocation report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

3)	<plmn-change>, an optional element specifying what PLMN changes trigger a request for a location report. This element consists of the following sub-elements:

i)	<any-plmn-change>, an optional element. The presence of this element specifies that any PLMN change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-plmn>, an optional element specifying a PLMN id (MCC+MNC) coded as specified in 3GPP TS 23.003 [2] which when entered triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-plmn>, an optional element specifying a PLMN id (MCC+MNC) coded as specified in 3GPP TS 23.003 [2] which when exited triggers a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

4)	<mbms-sa-change>, an optional element specifying what MBMS changes trigger location reporting. This element consists of the following sub-elements:

i)	<any-mbms-sa-change>, an optional element. The presence of this element specifies that any MBMS SA change is a trigger for a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-mbms-sa>, an optional element specifying an MBMS service area id which when entered triggers a request for a location report. The MBMS service area id is coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI). This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-mbms-sa>, an optional element specifying an MBMS service area id which when exited triggers a request a location report. The MBMS service area id is coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI). This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

5)	<mbsfn-area-change>, an optional element specifying what MBSFN changes trigger a request for a location report. This element consists of the following sub-elements:

i)	<any-mbsfn-area-change>, an optional element. The presence of this element specifies that any MBSFN area change is a trigger for a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-mbsfn-area>, an optional element specifying an MBSFN area which when entered triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-mbsfn-area>, an optional element specifying an MBSFN area which when exited triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

6)	<periodic-report>, an optional element specifying that periodic request for a location report shall be sent. The value in seconds specifies the reporting interval. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

7)	<travelled-distance>, an optional element specifying that the travelled distance shall trigger a request for a location report. The value in metres specified the travelled distance. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

8)	<vertical-application-event>, an optional element specifying what application signalling events triggers a request for a location report. The <vertical-application-event> element has the following sub-elements:

i)	<initial-log-on>, an optional element specifying that an initial log on triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<location-configuration-received>, an optional element specifying that a received location configuration triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<any-other- event>, an optional element specifying that any other application signalling event than initial-log-on and location-configuration-received triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

9)	<geographical-area-change>, an optional element specifying what geographical are changes trigger a request for a location reporting. This element consists of the following sub-elements:

i)	<any-area-change>, an optional element. The presence of this element specifies that any geographical area change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-area>, an optional element specifying a geographical area which when entered triggers a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string. The <enter-specific-area> element has the following sub-elements:

A)	<geographical-area>, an optional element containing a <trigger-id> attribute and the following two subelements:

I)	<polygon-area>, an optional element specifying the area as a polygon specified in clause 5.2 in 3GPP TS 23.032 [2]; and

II)	<ellipsoid-arc-area>, an optional element specifying the area as an ellipsoid arc specified in clause 5.7 in 3GPP TS 23.032 [2]; and

iii)	<exit-specific-area-type>, an optional element specifying a geographical area which when exited triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string.

c)	<minimum-interval-length>, a mandatory element specifying the minimum time the SLM-C needs to wait between sending location reports. The value is given in seconds;

<report-request> is a mandatory element used to include the requested location report. The <report-request> element contains at least one of the following sub-elements:

a)	<immediate-report-indication>, a mandatory element which can occur multiple times that contain the value of the <trigger-id> attribute associated with a trigger that has fired; and

b)	<current-location>, a mandatory element that contains the location information. The <current-location> element contains the following sub-elements:

1)	<current-serving-NCGI>, an optional element containing the NR cell global identity (NCGI) of the serving cell coded as specified in clause 19.6A in 3GPP TS 23.003 [2];

2)	<neighbouring-NCGI>, an optional element that can occur multiple times. It contains the NCGI of any neighbouring cell the SLM-C can detect;

3)	<mbms-service-area-id>, an optional element containing the MBMS service area id that the SLM-C is using. The MBMS service area id is coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI); and

4)	<current-coordinate>, an optional element containing the longitude and latitude coded as specified in clause 6.1 in 3GPP TS 23.032 [3].

c)	<triggering-criteria>, a mandatory element specifying the triggers for the SLM-C to request a location report of a VAL user, a VAL client or a VAL group. The <triggering-criteria> element contains at least one of the following sub-elements:

1)	<cell-change>, an optional element specifying what cell changes trigger the request for a location report. This element consists of the following sub-elements:

i)	<any-cell-change>, an optional element. The presence of this element specifies that any cell change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-cell>, an optional element specifying an NCGI which when entered triggers a request for alocation report coded as specified in clause 19.6A in 3GPP TS 23.003 [2]. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-cell>, an optional element specifying an NCGI which when exited triggers a request for a location report coded as specified in clause 19.6A in 3GPP TS 23.003 [2]. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

2)	<tracking-area-change>, an optional element specifying what tracking area changes trigger a request for a location report. This element consists of the following sub-elements:

i)	<any-tracking-area-change>, an optional element. The presence of this element specifies that any tracking area change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-tracking-area>, an optional element specifying a tracking area identity coded as specified in clause 19.4.2.3 in 3GPP TS 23.003 [2] which when entered triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-tracking-area>, an optional element specifying a tracking area identity coded as specified in clause 19.4.2.3 in 3GPP TS 23.003 [2] which when exited triggers a request for alocation report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

3)	<plmn-change>, an optional element specifying what PLMN changes trigger a request for a location report. This element consists of the following sub-elements:

i)	<any-plmn-change>, an optional element. The presence of this element specifies that any PLMN change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-plmn>, an optional element specifying a PLMN id (MCC+MNC) coded as specified in 3GPP TS 23.003 [2] which when entered triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-plmn>, an optional element specifying a PLMN id (MCC+MNC) coded as specified in 3GPP TS 23.003 [2] which when exited triggers a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

4)	<mbms-sa-change>, an optional element specifying what MBMS changes trigger location reporting. This element consists of the following sub-elements:

i)	<any-mbms-sa-change>, an optional element. The presence of this element specifies that any MBMS SA change is a trigger for a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-mbms-sa>, an optional element specifying an MBMS service area id which when entered triggers a request for a location report. The MBMS service area id is coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI). This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-mbms-sa>, an optional element specifying an MBMS service area id which when exited triggers a request a location report. The MBMS service area id is coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI). This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

5)	<mbsfn-area-change>, an optional element specifying what MBSFN changes trigger a request for a location report. This element consists of the following sub-elements:

i)	<any-mbsfn-area-change>, an optional element. The presence of this element specifies that any MBSFN area change is a trigger for a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-mbsfn-area>, an optional element specifying an MBSFN area which when entered triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<exit-specific-mbsfn-area>, an optional element specifying an MBSFN area which when exited triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

6)	<periodic-report>, an optional element specifying that periodic request for a location report shall be sent. The value in seconds specifies the reporting interval. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

7)	<travelled-distance>, an optional element specifying that the travelled distance shall trigger a request for a location report. The value in metres specified the travelled distance. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

8)	<vertical-application-event>, an optional element specifying what application signalling events triggers a request for a location report. The <vertical-application-event> element has the following sub-elements:

i)	<initial-log-on>, an optional element specifying that an initial log on triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<location-configuration-received>, an optional element specifying that a received location configuration triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

iii)	<any-other- event>, an optional element specifying that any other application signalling event than initial-log-on and location-configuration-received triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

9)	<geographical-area-change>, an optional element specifying what geographical are changes trigger a request for a location reporting. This element consists of the following sub-elements:

i)	<any-area-change>, an optional element. The presence of this element specifies that any geographical area change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

ii)	<enter-specific-area>, an optional element specifying a geographical area which when entered triggers a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string. The <enter-specific-area> element has the following sub-elements:

A)	<geographical-area>, an optional element containing a <trigger-id> attribute and the following two subelements:

I)	<polygon-area>, an optional element specifying the area as a polygon specified in clause 5.2 in 3GPP TS 23.032 [2]; and

II)	<ellipsoid-arc-area>, an optional element specifying the area as an ellipsoid arc specified in clause 5.7 in 3GPP TS 23.032 [2]; and

iii)	<exit-specific-area-type>, an optional element specifying a geographical area which when exited triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string.

The recipient of the XML ignores any unknown element and any unknown attribute.

## 7.6	MIME types

Editor’s note:	This clause will describe the MIME types.

The MIME type for the LocationInfo document shall be "application/vnd.3gpp.seal-location-info+xml MIME".

## 7.7	IANA registration template

Editor’s note:	This clause will describe the IANA registration template.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-10 | CT1#120 | C1-196855 |  |  |  | Draft skeleton provided by the rapporteur. | 0.0.0 |
| 2019-10 | CT1#120 |  |  |  |  | Implementing the following p-CRs agreed by CT1: C1-196355, C1-196612, C1-196856, C1-196857 | 0.1.0 |
| 2019-11 | CT1#121 |  |  |  |  | Implementing the following p-CRs agreed by CT1: C1-198605, C1-198606, C1-198607, C1-198609, C1-198818, C1-198820 Corrections done by the rapporteur. | 0.2.0 |
| 2020-03 | CT1#122-e |  |  |  |  | Implementing the following p-CRs agreed by CT1: C1-200526, C1-200555, C1-200558, C1-200560, C1-200808, C1-200901, C1-200902, C1-201018, C1-201019 Corrections done by the rapporteur. | 0.3.0 |
| 2020-03 | CT-87e | CP-200169 |  |  |  | Presentation to TSG CT for information andapproval | 1.0.0 |
| 2020-03 | CT-87e |  |  |  |  | Version 16.0.0 created after approval | 16.0.0 |