| 3GPP TS 24.547 V1.1.0 (2020-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Identity management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification; (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPPOrganizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPPonly. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms and abbreviations	7

3.1	Terms	7

3.2	Abbreviations	7

4	General description	7

5	Functional entities	7

5.1	SEAL identity management client (SIM-C)	7

5.2	SEAL identity management server (SIM-S)	8

6	Identity management procedures	8

6.1	General	8

6.2	On-network procedures	8

6.2.1	General	8

6.2.2	User authentication procedure	8

6.2.2.1	SIM-C procedure	8

6.2.2.2	SIM-S procedure	9

6.2.3	Token exchange procedure	10

6.2.3.1	SIM-C procedure	10

6.2.3.2	SIM-S procedure	10

6.3	Off-network procedures	11

Annex A (normative): HTTP entities	12

A.1	Scope	12

A.2	Procedures	12

A.2.1	HTTP client	12

A.2.1.1	General	12

A.2.1.2	HTTP client in UE	12

A.2.1.3	HTTP client in network entity	13

A.2.2	HTTP proxy	13

A.2.2.1	General	13

A.2.2.2	HTTP request method from HTTP client in UE	13

A.2.2.3	HTTP request method from HTTP client in network entity within trust domain	13

A.2.3	HTTP server	14

Annex B (informative): Change history	15

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the protocol aspects for the identity management capability of SEAL to support vertical applications (e.g. V2X) over the 3GPP system.

The present document is applicable to the User Equipment (UE) supporting the identity management client functionality as described in 3GPP TS 23.434 [2], to the application server supporting the identity management server functionality as described in 3GPP TS 23.434 [2] and to the application server supporting the vertical application server (VAL server) functionality as defined in specific vertical application service (VAL service) specifications.

NOTE:	The specification of the VAL server for a specific VAL service is out of scope of the present document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows;".

[3]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[4]	OMA OMA-TS-XDM_Group-V1_1_1-20170124-A: "Group XDM Specification".

[5]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

[6]	W3C.REC-html401-19991224: "HTML 4.01 Specification".

[7]	3GPP TS 33.434: "Service Enabler Architecture Layer (SEAL); Security aspects for Verticals".

[8]	draft-ietf-oauth-token-exchange-11: "OAuth 2.0 Token Exchange".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	IETF RFC 7159: "The JavaScript Object Notation (JSON) Data Interchange Format".

[11]	"OpenID Connect Core 1.0 incorporating errata set 1".

[12]	IETF RFC 2818: "HTTP Over TLS".

[13]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[14]	3GPP TS 24.109: "Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details".

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Authorisation endpoint: A SEAL identity management server protocol endpoint used by the SEAL identity management client to obtain an authorisation grant, as specified in IETF RFC 6749 [9].

SEAL identity management client: An entity that provides the client side functionalities corresponding to the identity management SEAL service.

SEAL identity management server: An entity that provides the server side functionalities corresponding to the identity management SEAL service.

Token endpoint: A SEAL identity management server protocol endpoint used by the SEAL identity management client to exchange an authorisation grant for an access token, as specified in IETF RFC 6749 [9].

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.434 [2] apply:

SEAL client

SEAL server

SEAL service

VAL server

VAL service

VAL user

Vertical

Vertical application

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

SEAL	Service Enabler Architecture Layer for verticals

SIM-C	SEAL Identity Management Client

SIM-S	SEAL Identity Management Server

VAL	Vertical Application Layer

# 4	General description

Identity management is a SEAL service that provides the identity management related capabilities to one or more vertical applications. The present document enables a SEAL identity management client and a VAL server to communicate with a SEAL identity management server. The SEAL identity management server authenticates the VAL user’s identity by verifying the credentials provided by the VAL user. When the VAL user is authenticated it is provided with an access token which is used for accessing different SEAL services.

# 5	Functional entities

## 5.1	SEAL identity management client (SIM-C)

The SIM-C is a functional entity that acts as the application client for VAL user identity related transactions. To be compliant with the procedures in the present document the SIM-C shall:

-	support the role of XCAP client as specified in IETF RFC 4825 [3];

-	support the role of XDMC as specified in OMA OMA-TS-XDM_Core-V2_1 [4];

-	support the user authentication procedure specified in clause 6.2.2; and

-	support the token exchange procedure specified in clause 6.2.3.

## 5.2	SEAL identity management server (SIM-S)

The SIM-S is a functional entity that authenticates the VAL user’s identity by verifying the credentials provided by the VAL user. To be compliant with the procedures in the present document the SIM-S shall:

-	support the role of XCAP server as specified in IETF RFC 4825 [3];

-	support the role of XDMS as specified in OMA OMA-TS-XDM_Core-V2_1 [4];

-	support the user authentication procedure specified in clause 6.2.2; and

-	support the token exchange procedure specified in clause 6.2.3.

# 6	Identity management procedures

## 6.1	General

## 6.2	On-network procedures

### 6.2.1	General

### 6.2.2	User authentication procedure

#### 6.2.2.1	SIM-C procedure

Editor’s Note: This procedure may be updated once a more updated reference to 3GPP TS 33.434 is available.

Upon receiving a request from VAL user to initiate authentication for VAL services, the SIM-C shall:

a)	establish a TLS tunnel to the authorisation endpoint of the SIM-S as specified in 3GPP TS 33.434 [7] using the URL of authorisation endpoint of the SIM-S as provided by the specific VAL service; and

b)	send an OIDC Authentication Request message as specified in the OpenID Connect 1.0 [11] and IETF RFC 6749 [9] using an HTTP GET request method towards the SIM-S according to IETF RFC 2616 [5]. The SIM-C shall include the following parameters as specified in 3GPP TS 33.434 [7] in the query component of the authorization endpoint's URI using the "application/x-www-form-urlencoded" format as specified in W3C.REC-html401-19991224 [6]:

-	client_id;

-	scope;

-	redirect_uri;

-	state;

-	acr_values;

-	code_challenge; and

-	code_challenge_method.

Upon receiving an HTTP 200 (OK) response from the SIM-S, the SIM-C shall:

a)	prompt the VAL service user for their username and password;

b)	generate an HTTP POST request method containing the VAL service user's username and password; and

c)	send the HTTP POST request method towards the SIM-S.

Upon receiving an OIDC Authentication Response message, the SIM-C shall:

a)	establish a TLS tunnel to the token endpoint of the SIM-S as specified in 3GPP TS 33.434 [7]; and

b)	send an OIDC Token Request message as specified in OpenID Connect 1.0 [11] and IETF RFC 6749 [9] using an HTTP POST request method towards the SIM-S according to IETF RFC 2616 [5]. The following parameters are included in the entity body of the HTTP POST request using the "application/x-www-form-urlencoded" format as specified in W3C.REC-html401-19991224 [6] as specified in 3GPP TS 33.434 [7]:

-	grant_type;

-	client_id;

-	redirect_uri; and

-	code_verifier.

Upon receiving an OIDC Token response message from the SIM-S, the SIM-C shall:

a)	validate the id_token, access_token and refresh token in the received OIDC Token Response message as specified in the OpenID Connect 1.0 [11] specification; and

b)	provide the id_token and access_token in the received OIDC Token Response message to the VAL user.

The SIM-C may repeat the entire procedure in this subclause as needed to obtain the necessary authorisation tokens for the VAL service clients, depending on the scope parameter in the Authentication Request message as specified in 3GPP TS 33.434 [7].

#### 6.2.2.2	SIM-S procedure

Editor’s Note: This procedure may be updated once a more updated reference to 3GPP TS 33.434 is available.

Upon receiving an OIDC Authentication Request message as specified in the OpenID Connect 1.0 [11] and IETF RFC 6749 [9] via a secure TLS tunnel between the SIM-C and the authorisation endpoint of the SIM-S, the SIM-S shall:

a)	validate the received OIDC Authentication Request message as specified in the OpenID Connect 1.0 [11] and IETF RFC 6749 [9];

b)	generate an HTTP 200 (OK) response according to IETF RFC 2616 [5] including form data to prompt the VAL service user for their username and password credentials; and

c)	send the HTTP 200 (OK) response towards the SIM-C.

Upon receiving an HTTP POST request method from the SIM-C containing the VAL service user's username and password, the SIM-S authenticates the VAL service user and shall:

a)	generate an OIDC Authentication Response message as specified in OpenID Connect 1.0 [11] and IETF RFC 6749 [9] with the following clarifications:

1)	shall generate an HTTP 302 (FOUND) response according to IETF RFC 2616 [5]; and

2)	shall include the following parameters as specified in 3GPP TS 33.434 [7]:

-	code; and

-	state.

in the query component of the redirection URI contained in the Location header field of the HTTP FOUND request method using the "application/x-www-form-urlencoded" format as specified in W3C.REC-html401-19991224 [6]; and

b)	send the HTTP 302 (FOUND) response towards the SIM-C.

Upon receiving an OIDC Token Request message via a secure TLS tunnel established between the SIM-C and the token endpoint of the SIM-S, the SIM-S shall:

a)	validate the OIDC Token Request message and if valid shall generate an OIDC Token Response message as specified in OpenID Connect 1.0 [11] and IETF RFC 6749 [9] with the following clarifications:

1)	shall generate an HTTP 200 (OK) response according to IETF RFC 2616 [5];

2)	shall based on the received VAL user ID obtained from the received user authentication credentials, determine the VAL service ID of the VAL service user;

3)	shall include an id_token, access_token and refresh_token and VAL service ID as specified in 3GPP TS 33.434 [7]; and

4)	shall include the other required parameters as specified in OpenID Connect 1.0 [11] and IETF RFC 6749 [9]; and

b)	shall send the HTTP 200 (OK) response towards the SIM-C.

### 6.2.3	Token exchange procedure

#### 6.2.3.1	SIM-C procedure

Editor’s Note: This procedure may be updated once a more updated reference to 3GPP TS 33.434 is available.

Upon receiving a request from the VAL user to acquire a security token for authentication of the VAL services, the SIM-C shall:

a)	establish a TLS tunnel to the token endpoint of the SIM-S as specified in 3GPP TS 33.434 [7]; and

b)	send a Token Exchange Request message as specified in 3GPP TS 33.434 [7] and draft-ietf-oauth-token-exchange [8] using an HTTP POST request method towards the SIM-S according to IETF RFC 2616 [5]. The following parameters are  included in the entity body of the HTTP POST request using the "application/x-www-form-urlencoded" format as specified in W3C.REC-html401-19991224 [6] and 3GPP TS 33.434 [7]:

-	grant_type;

-	code;

-	client_id;

-	redirect_uri; and

-	code_verifier.

Upon receipt of an HTTP 200 (OK) response from SIM-S, the SIM-C shall extract the security token contained in the access_token parameter of the received Token Exchange Response message as specified in 3GPP TS 33.434 [7] and draft-ietf-oauth-token-exchange [8] and send it to the VAL user.

#### 6.2.3.2	SIM-S procedure

Editor’s Note: This procedure may be updated once a more updated reference to 3GPP TS 33.434 is available.

Upon receiving a Token Exchange Request message as specified in draft-ietf-oauth-token-exchange [8] via a secure TLS tunnel between the SIM-C and the token endpoint of the SIM-S, the SIM-S shall:

a)	validate the received Token Exchange Request message as specified in draft-ietf-oauth-token-exchange [8]; and

b)	send a Token Exchange Response message as specified in draft-ietf-oauth-token-exchange [8] and IETF RFC 6749 [9] using an HTTP 200 (OK) response to the SIM-C according to IETF RFC 2616 [5]. The following parameters are included in the HTTP 200 (OK) response as specified in 3GPP TS 33.434 [7] and are serialized into a JavaScript Object Notation (JSON) structure as specified in draft-ietf-oauth-token-exchange [8] and IETF RFC 7159 [10]:

-	access_token;

-	issued_token_type;

-	token_type; and

-	expires_in.

## 6.3	Off-network procedures

The off-network procedures are out of scope of the present document in this release of the specification.

Annex A (normative):
HTTP entities

# A.1	Scope

This annex describes the functionality expected from the HTTP entities (i.e. the HTTP client, the HTTP proxy and the HTTP server) defined by 3GPP TS 23.434 [2].

# A.2	Procedures

## A.2.1	HTTP client

### A.2.1.1	General

The HTTP client shall support the client role defined in IETF RFC 2616 [5].

### A.2.1.2	HTTP client in UE

The HTTP client in the UE shall support the client role defined in IETF RFC 2818 [12].

The HTTP client in the UE shall support transport layer security (TLS) as specified in clause 6 of 3GPP TS 33.434 [7].

The HTTP client in the UE is configured with the following parameters:

a)	a home HTTP proxy FQDN;

b)	a home HTTP proxy port;

c)	One of the following TLS tunnel authentication method along with its parameters as specified in 3GPP TS 33.434 [7]:

1)	one-way authentication of the HTTP proxy based on the server certificate;

2)	mutual authentication based on certificates, along with TLS tunnel authentication based on X.509 certificate; and

3)	mutual authentication based on pre-shared key, along with TLS tunnel authentication based on pre-shared key;

The HTTP client in the UE shall establish a TCP connection towards the home HTTP proxy FQDN and the home HTTP proxy port.

The HTTP client in the UE shall establish a TLS tunnel via the TCP connection as specified in 3GPP TS 33.434 [7]. When establishing the TLS tunnel, the HTTP client in the UE shall act as a TLS client and the UE shall perform the TLS tunnel authentication using the TLS authentication method indicated by the TLS tunnel authentication method parameter according to 3GPP TS 33.434 [7]. In order to prevent man-in-the-middle attacks, the HTTP client in the UE shall check the home HTTP proxy FQDN against the server's identity as presented in the received server's certificate message if the TCP connection terminates on the HTTP proxy. The HTTP client in the UE shall check the portion of dereferenced HTTP URL against the server's identity as presented in the received server's certificate message only if the TCP connection terminates on the SIM-S.

NOTE:	The TLS tunnel can be terminated in the HTTP proxy (rather than in the HTTP server providing the dereferenced HTTP URL).

The HTTP client in the UE shall send and receive all HTTP messages via the TLS tunnel.

If the HTTP client in the UE has an access token of the "bearer" token type as specified in IETF RFC 6750 [13], the HTTP client in the UE shall include an Authorization header field with the "Bearer" authentication scheme as specified in IETF RFC 6750 [13] in HTTP requests.

### A.2.1.3	HTTP client in network entity

The HTTP client in the network entity is configured with the following parameters:

a)	a home HTTP proxy FQDN; and

b)	a home HTTP proxy port.

The HTTP client in the network entity shall send and receive all HTTP messages via the home HTTP proxy.

The HTTP client in the network entity shall insert an X-3GPP-Asserted-Identity header field as specified in 3GPP TS 24.109 [14] in the HTTP request and shall set X-3GPP-Asserted-Identity header field to the identity of the HTTP client in the network entity. The identity of the HTTP client in the network entity can be a public service identity, a VAL group ID, or a VAL service ID.

## A.2.2	HTTP proxy

### A.2.2.1	General

The HTTP proxy shall support proxy role defined in IETF RFC 2616 [5].

### A.2.2.2	HTTP request method from HTTP client in UE

The HTTP proxy shall support the server role defined in IETF RFC 2616 [5], and in IETF RFC 2818 [12].

The HTTP proxy shall support transport layer security (TLS) as specified in 3GPP TS 33.434 [7].

The HTTP proxy is configured with the following HTTP proxy parameters:

a)	an FQDN of an HTTP proxy for UEs; and

b)	a TCP port of an HTTP proxy for UEs.

The HTTP proxy shall support establishing TCP connections on the FQDN of HTTP proxy for UEs and the TCP port of HTTP proxy for UEs. The HTTP proxy shall support establishing a TLS tunnel via each such TCP connection as specified in 3GPP TS 33.434 [7]. When establishing the TLS tunnel, the HTTP proxy shall act as the TLS server.

Upon reception of an HTTP request method via a TLS tunnel:

a)	if the HTTP request method contains an X-3GPP-Asserted-Identity header field as specified in 3GPP TS 24.109 [14], the HTTP proxy shall reject the HTTP request method with an HTTP 403 (Forbidden) response and shall not continue with the below steps;

b)	if the HTTP request method contains a Request-URI identifying a resource in a partner's VAL service provider, the HTTP proxy shall forward the HTTP request method according to the Request-URI; and

c)	if the HTTP request method contains a Request-URI identifying a resource in its own VAL service provider, the HTTP proxy shall act as a reverse proxy for the HTTP request method and shall forward the HTTP request method according to the VAL service provider’s policy.

### A.2.2.3	HTTP request method from HTTP client in network entity within trust domain

The HTTP proxy is configured with the following parameters:

a)	a FQDN of an HTTP proxy for trusted entities; and

b)	a TCP port of an HTTP proxy for trusted entities.

Upon receiving an HTTP request method via a TCP connection established on the FQDN of HTTP proxy for UEs and the TCP port of HTTP proxy for UEs, if the TCP connection is between network elements within trusted domain as specified in 3GPP TS 33.434 [7], then:

a)	if the HTTP request method contains a Request-URI identifying a resource in a partner's VAL service provider, the HTTP proxy shall forward the HTTP request method according to the Request-URI; and

b)	if an HTTP request method contains Request-URI identifying a resource in own VAL service provider, the HTTP proxy shall act as reverse proxy for the HTTP request method and shall forward the HTTP request method according to VAL service provider’s policy.

## A.2.3	HTTP server

The HTTP server shall support the server role defined in IETF RFC 2616 [5].

Upon reception of an HTTP request:

a)	if the received HTTP request does not contain an Authorization header field with the "Bearer" authentication scheme and a bearer access token as specified in IETF RFC 6750 [13] and the received HTTP request does not contain an X-3GPP-Asserted-Identity header field as specified in 3GPP TS 24.109 [14], the HTTP server shall reject the request with HTTP 403 (Forbidden) response;

b)	if the received HTTP request contains an Authorization header field with the "Bearer" authentication scheme and a bearer access token as specified in IETF RFC 6750 [13];

a)	the HTTP server shall validate the bearer access token as specified in IETF RFC 6750 [13]; and

b)	the HTTP server shall consider the VAL service ID derived from the bearer access token as the identity of the sender of the HTTP request; and

c)	if the received HTTP request does not contain an Authorization header field with the "Bearer" authentication scheme and a bearer access token as specified in IETF RFC 6750 [13] and the received HTTP request contains an X-3GPP-Asserted-Identity header field as specified in 3GPP TS 24.109 [14], the HTTP server shall consider the URI in the X-3GPP-Asserted-Identity header field as the identity of the sender of the HTTP request.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-10 | CT1#120 | C1-196093 |  |  |  | Draft skeleton provided by the rapporteur. | 0.0.0 |
| 2019-10 | CT1#120 |  |  |  |  | Implementing the following p-CRs agreed by CT1: C1-196850, C1-196865, C1-196866, C1-196867 | 0.1.0 |
| 2019-11 | CT1#121 |  |  |  |  | Implementing the following p-CRs agreed by CT1: C1-198600, C1-198601, C1-198602, C1-198603 | 0.2.0 |
| 2019-12 | CT-86 | CP-193154 |  |  |  | Presentation for information at TSG CT | 1.0.0 |
| 2020-03 | CT1#122-e |  |  |  |  | Implementing the following p-CRs agreed by CT1: C1-200450, C1-200609, C1-200611, C1-200612, C1-200818, C1-201003 | 1.1.0 |