| 3GPP TS 28.555 V1.0.2 (2020-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Policy management for 5G mobile networks; Stage 1 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Concepts and Background	7

4.1 	Policy MnS	7

4.2 	Policy conflict detection and resolution	7

5	Business level requirements	7

5.1	Requirements	7

5.2	Actor roles	8

5.3	Telecommunications resources	8

5.4	Use cases	8

5.4.1	Deploy IMS network functions in centralized or edge DC's	8

5.4.2	Deploy IMS network functions in different DC	8

5.4.3	Deploy IMS network functions in a same DC	9

6	Specification level requirements	10

6.1	Requirements	10

6.2	Use cases	11

Annex <A> (normative):	 <Normative annex for a Technical Specification>	11

Annex <X> (informative): 	Change history	12

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects Management and orchestration of networks, as identified below:

TS 28.555:	Policy management for 5G mobile networks; Stage 1 [2].

TS 28.556:	Policy management for 5G mobile networks; Stage2 and Stage3 [3].

# 1	Scope

The present document specifies the concepts, requirements and use cases for network policy management in 5G networks.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.555: "Management and orchestration; Network policy management for 5G mobile networks; Stage 1".

[3]	3GPP TS 28.556: "Management and orchestration; Network policy management for 5G mobile networks; Stage 2 and Stage 3".

[4]	ETSI GR NFV-IFA 023 (V3.1.1): "Network Functions Virtualisation (NFV); Management and Orchestration; Report on Policy Management in MANO; Release 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

# 4	Concepts and Background

Editor's note: this clause will contain concepts and overview information relevant to policy management service.

## 4.1 	Policy MnS

Introduction of service-based architecture for 5G, expanding the scope of policy management, the policy management for 5G will be built on service-based management architecture.

A Policy MnS producer has the following capabilities:

-	Storing the defined network policy.

-	Detecting policy conflict, reporting the conflict information and give suggestions for solving the conflict.

-	Making the policy execution decisions, which means it take the decision to trigger or perform an action.

-	Evaluating the effectiveness of the network policy execution.

The MnS of various kinds are specified for deployment over many standardized reference interfaces. So, the policy MnS could in principle, be specified for deployment over the same set of standardized reference interfaces, as a replacement of or as an addition to the policy related driven MnS.

## 4.2 	Policy conflict detection and resolution

There may be policy conflict problem, which is a new activated policy may be conflict with an existing activated policy.

When the Policy MnS producer detects the conflict, it can report the conflict information and give suggestions to solve it. GR NFV-IFA 023 [4] descripts policy conflict detection UCs and how to resolve them.

There are multiple scenarios for policy conflict, for example:

1)	If the events are the same and the same condition are triggered (e.g. scaling policy on a VNF), but two policies have different actions, the conflict should be detected and resolved.

2)	If the event and condition of two policies are not be the same, but the different actions (e.g. scaling policy vs. termination policy on a VNF) are towards the same entity. The conflict should be detected and resolved.

3)	If scenario 1) or 2) is enforced by different PFs. The conflict should be detected and resolved.

# 5	Business level requirements

Editor's note: this clause will contain business level requirements for policy management service.

## 5.1	Requirements

REQ-POM_ND-CON-1 3GPP management system should be able to support the capability about the network policy management.

## 5.2	Actor roles

## 5.3	Telecommunications resources

## 5.4	Use cases

### 5.4.1	Deploy IMS network functions in centralized or edge DC's

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | According to the network deployment policy, 3GPP system use the policy to deploy the corresponding network. |  |
| Actors and Roles | Policy MnS consumer is responsible for creating the IMS deployment policy and evaluating the policy execution. Policy MnS producer is responsible for executing the IMS deployment policy and can store the policy. |  |
| Telecom resources | 3GPP management system, NFV MANO (NFVO, VNFM) |  |
| Assumptions | 1. The operator has two types' DCs, core DCs are centralized in some individual region and edge DCs are distributed in most cities. 2. The operator determines core DC supports the control plane network function which handles signalling, determines CSCF should be deployed in core DC. 3. The operator determines edge DC supports the media plane or forwarding plane network function which handles session service and data service, determines SBC should be in edge DC. |  |
| Pre conditions | 1. The operator designs the IMS network deployment requirement and need deploy the network based on the requirement. 2. Management systems (i.e. 3GPP management system and NFV-MANO system) are running normally. |  |
| Begins when | The Policy MnS consumer requests the Policy MnS producer to create the deploy policy (IMS network and determine CSCF should be deployed in core DC and SBC should be in edge DC). |  |
| Step 1 (M) | The Policy MnS consumer activates the policy and notifies related Policy MnS producer. |  |
| Step 2 (M) | The Policy MnS producer follows the policy to design the IMS network then the IMS network is instantiated by the management system. |  |
| Step 3 (M) | The Policy MnS producer informs the Policy MnS consumer the policy has been executed. |  |
| Ends when | The Policy MnS consumer evaluates the effectiveness of the policy execution if needed. The Policy MnS consumer evaluates the effectiveness of the policy according to the result of the policy execution. In this case the effectiveness is CSCF(s) and SBC(s) in the IMS network are deployed in appropriate DCs and on appropriate hardware resource. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post Conditions | The IMS network is correctly configured and normally running. |  |
| Traceability | REQ-POM_ND-CON-1 |  |

### 5.4.2	Deploy IMS network functions in different DC

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | According to the network deployment policy, 3GPP system use the policy to deploy the corresponding network. |  |
| Actors and Roles | Policy MnS consumer is responsible for creating the IMS deployment policy and evaluating the policy execution. Policy MnS producer is responsible for executing the IMS deployment policy and can store the policy. |  |
| Telecom resources | 3GPP management system, NFV MANO (NFVO, VNFM), |  |
| Assumptions | The operator determines to deploy one IMS network with 3 load sharing CSCFs. The operator determines it is not allowed to deploy more than 2 CSCFs in the same core DC. |  |
| Pre conditions | 1. The operator designs the IMS network deployment requirement and need deploy the network based on the requirement. 2. Management systems (i.e. 3GPP management system and NFV-MANO system) are running normally. |  |
| Begins when | The Policy MnS consumer requests the Policy MnS producer to create the policy(deploy one IMS network with 3 load sharing CSCFs, it is not allowed to deploy more than 2 CSCFs in the same core DC). |  |
| Step 1 (M) | The Policy MnS consumer activates the policy and notifies related Policy MnS producer. |  |
| Step 2 (M) | The Policy MnS producer follows the deployment policy to design the IMS network then the IMS network is instantiated by the management system. Note: Whether and how NSD or VNFD supports the policy is out of scope of the present document. |  |
| Step 3 (M) | The Policy MnS producer informs the Policy MnS consumer the policy has been executed. |  |
|  | The Policy MnS consumer evaluates the effectiveness of the policy execution if needed. |  |
| Ends when | The Policy MnS consumer evaluates the effectiveness of the policy execution if needed. The Policy MnS consumer evaluates the effectiveness of the policy according to the result of the policy execution. In this case the effectiveness is all CSCFs in the IMS network are deployed in appropriate DCs. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post Conditions | The IMS network is correctly configured and normally running. |  |
| Traceability | REQ-POM_ND-CON-1 |  |

### 5.4.3	Deploy IMS network functions in a same DC

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | According to the network deployment policy, 3GPP system use the policy to deploy the corresponding network. |  |
| Actors and Roles | Policy MnS consumer is responsible for creating the IMS deployment policy and evaluating the policy execution. Policy MnS producer is responsible for executing the IMS deployment policy and can store the policy. |  |
| Telecom resources | 3GPP management system, NFV MANO (NFVO, VNFM), |  |
| Assumptions | 1. The operator has two types' DCs, core DCs are centralized in some individual region and edge DCs are distributed in most cities. 2. The operator need deploy IMS network in one region and the EPC network has already been running. |  |
| Pre conditions | 1. The operator designs the IMS network deployment requirement and need deploy the network based on the requirement. 2. Management systems (i.e. 3GPP management system and NFV-MANO system) are running normally. 3. The Policy MnS producer has the adjacent location information of the data centre(s) and central office(s). Editor's note: How the Policy MnS producer gets the adjacent location information is out of scope of the present document. |  |
| Begins when | The Policy MnS consumer requests the Policy MnS producer to create the policy(deploy IMS network in one region). |  |
| Step 1 (M) | The Policy MnS consumer delivers the policy to Policy MnS producer and activates the policy if needed. |  |
| Step 2 (M) | The Policy MnS producer follows the policy to design the IMS network and determine the virtualized SBC location after getting the SAE GW location. |  |
| Step 3 (M) | The management systems follow the location constraint to design template (e.g. NSD or VNFD) and instantiate the IMS network. |  |
| Ends when | The Policy MnS producer informs the Policy MnS consumer the policy has been executed. |  |
| Exceptions | The Policy MnS consumer evaluates the effectiveness of the policy execution if needed. The Policy MnS consumer evaluates the effectiveness of the policy according to the result of the policy execution. In this case the effectiveness is the virtualized SBC is deployed adjacently to the running SAE GW. |  |
| Post Conditions | The IMS network is correctly configured and normally running. |  |
| Traceability | REQ-POM_ND-CON-1 |  |

# 6	Specification level requirements

Editor's note: this clause will contain specification level requirements for policy management service.

## 6.1	Requirements

Editor's note: this clause will contain specific requirements for policy management service.

REQ-POM-FUN-01

The Policy Management Service Producer shall provide the capability to allow the Policy Management Service Consumer to create the network policy.

REQ-POM-FUN-02

The Policy Management Service Producer shall provide the capability to allow the Policy Management Service Consumer to delete the network policy.

REQ-POM-FUN-03

The Policy Management Service Producer shall provide the capability to allow the Policy Management Service Consumer to update the network policy.

REQ-POM-FUN-04

The Policy Management Service Producer shall provide the capability to allow the Policy Management Service Consumer to query the network policy.

REQ-POM-FUN-05

The Policy Management Service Producer shall provide the capability to allow the Policy Management Service Consumer to activate the network policy.

REQ-POM-FUN-06

The Policy Management Service Producer shall provide the capability to allow the Policy Management Service Consumer to deactivate the network policy.

REQ-POM-FUN-07

The Policy Management Service Producer should notify the Policy Management Service Consumer when there is a network policy conflict.

REQ-POM-FUN-08

The Policy Management Service Producer should be able to support the capability to execute the activated network policy.

REQ-POM-FUN-09

The Policy Management Service Producer should be able to support the capability to verify whether the policy is executed correctly.

## 6.2	Use cases

Editor's note: this clause will contain specific use cases for policy management service.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-08 | SA5#132e | S5-204441 |  |  |  | Skeleton | 0.0.0 |
| 2020-08 | SA5#132e | S5-204455 S5-204456 S5-204457 S5-204458 S5-204082 S5-204453 S5-204454 |  |  |  | pCR 28.555 add skeleton pCR 28.555 add introduction pCR 28.555 add scope pCR 28.555 add Concepts and background pCR 28.555 add specification level requirements pCR 28.555 add business level requirements pCR 28.555 add high-level use cases | 1.0.0 |
| 2020-09 | SA5#132e |  |  |  |  | EditHelp improvement | 1.0.1 |
| 2020-11 |  |  |  |  |  | No technical change. Replacement of empty document in 3GPP server. | 1.0.2 |