| 3GPP TS 36.101 V16.5.0 (2020-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	32

1	Scope	34

2	References	34

3	Definitions, symbols and abbreviations	34

3.1	Definitions	34

3.2	Symbols	36

3.3	Abbreviations	39

4	General	40

4.1	Relationship between minimum requirements and test requirements	40

4.2	Applicability of minimum requirements	41

4.3	Void	41

4.3A	Applicability of minimum requirements (CA, UL-MIMO, ProSe, Dual Connectivity, UE category 0, UE category M1, UE category M2, UE category 1bis, UE category NB1 and NB2, V2X Communication, MBMS UE)	41

4.4	RF requirements in later releases	43

5	Operating bands and channel arrangement	43

5.1	General	43

5.2	Void	43

5.3	Void	43

5.4	Void	43

5.5	Operating bands	43

5.5A	Operating bands for CA	46

5.5B	Operating bands for UL-MIMO	65

5.5C	Operating bands for Dual Connectivity	65

5.5D	Operating bands for ProSe	68

5.5E	Operating bands for UE category 0, UE category M1 and M2 and UE category 1bis	69

5.5F	Operating bands for category NB1 and NB2	69

5.5G	Operating bands for V2X Communication	69

5.6	Channel bandwidth	70

5.6.1	Channel bandwidths per operating band	71

5.6A	Channel bandwidth for CA	74

5.6A.1	Channel bandwidths per operating band for CA	76

5.6B	Channel bandwidth for UL-MIMO	184

5.6B.1	Void	184

5.6C	Channel bandwidth for Dual Connectivity	184

5.6C.1	Void	184

5.6D	Channel bandwidth for ProSe	184

5.6D.1	Channel bandwidths per operating band for ProSe	184

5.6F	Channel bandwidth for category NB1 and NB2	185

5.6G	Channel bandwidth for V2X Communication	186

5.6G.1	Channel bandwidths per operating band for V2X Communication	186

5.7	Channel arrangement	188

5.7.1	Channel spacing	188

5.7.1A	Channel spacing for CA	188

5.7.1F	Channel spacing for category NB1 and NB2	189

5.7.2	Channel raster	189

5.7.2A	Channel raster for CA	189

5.7.2F	Channel raster for category NB1 and NB2	189

5.7.3	Carrier frequency and EARFCN	189

5.7.3F	Carrier frequency and EARFCN for category NB1 and NB2	192

5.7.4	TX–RX frequency separation	192

5.7.4A	TX–RX frequency separation for CA	193

5.7.4E	TX–RX frequency separation for category M1 and M2	193

5.7.4F	TX–RX frequency separation for category NB1 and NB2	194

6	Transmitter characteristics	194

6.1	General	194

6.2	Transmit power	194

6.2.1	Void	194

6.2.2	UE maximum output power	194

6.2.2A	UE maximum output power for CA	197

6.2.2B	UE maximum output power for UL-MIMO	202

6.2.2C	 Void	205

6.2.2D	UE maximum output power for ProSe	205

6.2.2E	UE maximum output power for Category M1 and M2 UE	205

6.2.2F	UE maximum output power for category NB1 and NB2	206

6.2.2G	UE maximum output power for V2X Communication	207

6.2.3	UE maximum output power for modulation / channel bandwidth	209

6.2.3A	UE Maximum Output power for modulation / channel bandwidth for CA	210

6.2.3B	UE maximum output power for modulation / channel bandwidth for UL-MIMO	215

6.2.3D	UE maximum output power for modulation / channel bandwidth for ProSe	215

6.2.3E	UE maximum output power for modulation / channel bandwidth for category M1 and M2	216

6.2.3F	UE maximum output power for modulation / channel bandwidth for category NB1 and NB2	218

6.2.3G	UE maximum output power for modulation / channel bandwidth for V2X Communication	218

6.2.3G.1	MPR for Power class 3 V2X UE	218

6.2.3G.2	MPR for Power class 2 V2X UE	220

6.2.4	UE maximum output power with additional requirements	220

6.2.4A	UE maximum output power with additional requirements for CA	239

6.2.4A.1	A-MPR for CA_NS_01 for CA_1C	241

6.2.4A.2	A-MPR for CA_NS_02 for CA_1C	241

6.2.4A.3	A-MPR for CA_NS_03 for CA_1C	242

6.2.4A.4	A-MPR for CA_NS_04	243

6.2.4A.5	A-MPR for CA_NS_05 for CA_38C	245

6.2.4A.6	A-MPR for CA_NS_06	246

6.2.4A.7	A-MPR for CA_NS_07	247

6.2.4A.8	A-MPR for CA_NS_08	248

6.2.4A.9	Void	249

6.2.4A.10	A-MPR for CA_NS_10	249

6.2.4B	UE maximum output power with additional requirements for UL-MIMO	252

6.2.4D	UE maximum output power with additional requirements for ProSe	253

6.2.4E	UE maximum output power with additional requirements for category M1 and M2 UE	253

6.2.4F	UE maximum output power with additional requirements for category NB1 and NB2 UE	262

6.2.4G	UE maximum output power with additional requirements for V2X Communication	263

6.2.5	Configured transmitted power	265

6.2.5A	Configured transmitted power for CA	296

6.2.5B	Configured transmitted power for UL-MIMO	300

6.2.5C	Configured transmitted power for Dual Connectivity	300

6.2.5D	Configured transmitted power for ProSe	302

6.2.5F	Configured transmitted Power for category NB1 and NB2	303

6.2.5G	Configured transmitted power for V2X Communication	304

6.3	Output power dynamics	308

6.3.1	(Void)	308

6.3.2	Minimum output power	308

6.3.2.1	Minimum requirement	308

6.3.2A	 UE Minimum output power for CA	308

6.3.2A.1	Minimum requirement for CA	309

6.3.2B	 UE Minimum output power for UL-MIMO	309

6.3.2B.1	Minimum requirement	309

6.3.2C	 Void	309

6.3.2D	UE Minimum output power for ProSe	309

6.3.2F	UE Minimum output power for category NB1 and NB2	310

6.3.2G	UE Minimum output power for V2X Communication	310

6.3.3	Transmit OFF power	310

6.3.3.1.	Minimum requirement	310

6.3.3A	 UE Transmit OFF power for CA	311

6.3.3A.1	Minimum requirement for CA	311

6.3.3B	 UE Transmit OFF power for UL-MIMO	311

6.3.3B.1	Minimum requirement	311

6.3.3D	Transmit OFF power for ProSe	312

6.3.3F	Transmit OFF power for category NB1 and NB2	312

6.3.3G	Transmit OFF power for V2X Communication	312

6.3.4	ON/OFF time mask	313

6.3.4.1	General ON/OFF time mask	313

6.3.4.2	PRACH and SRS time mask	314

6.3.4.2.1	PRACH time mask	314

6.3.4.2.2	SRS time mask	315

6.3.4.3	Slot / Sub frame boundary time mask for subframe TTI	317

6.3.4.4	PUCCH / PUSCH / SRS time mask for subframe TTI	319

6.3.4.5	Symbol / Subslot boundary time mask for subslot TTI	321

6.3.4.6	Subslot PUCCH / subslot PUSCH / SRS time mask for subslot TTI	322

6.3.4.7	Symbol / Slot boundary time mask for slot TTI	325

6.3.4.8	Slot PUCCH / slot PUSCH / SRS time mask for slot TTI	326

6.3.4.9	Consecutive subslot and slot TTI or consecutive subslot and subframe TTI  time mask	326

6.3.4.10	Consecutive subframe and subslot TTI or consecutive slot and subslot TTI time mask	326

6.3.4.11	Consecutive TTI and slot TTI or consecutive slot TTI and TTI time mask	327

6.3.4A	ON/OFF time mask for CA	328

6.3.4B	ON/OFF time mask for UL-MIMO	328

6.3.4D	ON/OFF time mask for ProSe	328

6.3.4D.1	 General time mask for ProSe	328

6.3.4D.2	 PSSS/SSSS time mask	329

6.3.4D.3	 PSSS / SSSS / PSBCH time mask	330

6.3.4D.4	 PSSCH / SRS time mask	330

6.3.4F	ON/OFF time mask for category NB1 and NB2	330

6.3.4F.1	General ON/OFF time mask	330

6.3.4F.2	NPRACH time mask	331

6.3.4G	ON/OFF time mask for V2X Communication	331

6.3.4G.1	PSSS / SSSS / PSBCH time mask	332

6.3.5	Power Control	332

6.3.5.1	Absolute power tolerance	332

6.3.5.1.1	 Minimum requirements	332

6.3.5.2	Relative Power tolerance	333

6.3.5.2.1	Minimum requirements	333

6.3.5.3	Aggregate power control tolerance	334

6.3.5.3.1	Minimum requirement	334

6.3.5A	Power control for CA	334

6.3.5A.1	Absolute power tolerance	334

6.3.5A.1.1	 Minimum requirements	334

6.3.5A.2	Relative power tolerance	335

6.3.5A.2.1	Minimum requirements	335

6.3.5A.3	Aggregate power control tolerance	335

6.3.5A.3.1	 Minimum requirements	335

6.3.5B	Power control for UL-MIMO	336

6.3.5D	Power Control for ProSe	336

6.3.5D.1	Absolute power tolerance	336

6.3.5E	Power control for category M1 and M2	336

6.3.5E.1	Absolute power tolerance	336

6.3.5E.2	Relative Power tolerance	336

6.3.5E.3	Aggregate power control tolerance	337

6.3.5E.3.1	Minimum requirement	337

6.3.5F	Power Control for category NB1 and NB2	337

6.3.5F.1	Absolute power tolerance	338

6.3.5F.2	Relative power tolerance	338

6.3.5F.3	Aggregate power control tolerance for category NB1 and NB2	339

6.3.5F.3.1	Minimum requirement	339

6.3.5G	Power Control for V2X Communication	339

6.3.5G.1	Absolute power tolerance	339

6.4	Void	340

6.5	Transmit signal quality	340

6.5.1	Frequency error	340

6.5.1A 	Frequency error for CA	340

6.5.1B	Frequency error for UL-MIMO	340

6.5.1D	Frequency error for ProSe	340

6.5.1E	Frequency error for UE category M1 and M2	340

6.5.1F	Frequency error for UE category NB1 and NB2	340

6.5.1G	Frequency error for V2X Communication	341

6.5.2	Transmit modulation quality	341

6.5.2.1	 Error Vector Magnitude	341

6.5.2.1.1	Minimum requirement	342

6.5.2.2	 Carrier leakage	342

6.5.2.2.1	 Minimum requirements	342

6.5.2.3	In-band emissions	343

6.5.2.3.1	 Minimum requirements	343

6.5.2.4	 EVM equalizer spectrum flatness	344

6.5.2.4.1	Minimum requirements	344

6.5.2A	Transmit modulation quality for CA	345

6.5.2A.1	Error Vector Magnitude	346

6.5.2A.2	Carrier leakage for CA	346

6.5.2A.2.1	Minimum requirements	346

6.5.2A.3	In-band emissions	346

6.5.2A.3.1	Minimum requirement for CA	346

6.5.2B	Transmit modulation quality for UL-MIMO	349

6.5.2B.1	Error Vector Magnitude	349

6.5.2B.2	Carrier leakage	349

6.5.2B.3	In-band emissions	349

6.5.2B.4	EVM equalizer spectrum flatness for UL-MIMO	349

6.5.2D	Transmit modulation quality for ProSe	349

6.5.2D.1	Error Vector Magnitude	349

6.5.2D.2	Carrier leakage	350

6.5.2D.3	In-band emissions	350

6.5.2D.4	EVM equalizer spectrum flatness for ProSe	350

6.5.2E	Transmit modulation quality for category M1 and M2	350

6.5.2E.1	Error Vector Magnitude	350

The Error Vector Magnitude is defined in section 6.5.2.1.	350

6.5.2E.2	Carrier leakage	350

6.5.2E.2.1	Minimum requirements	350

6.5.2E.3	In-band emissions	350

6.5.2E.3.1	Minimum requirements	350

6.5.2F	Transmit modulation quality for Category NB1 and NB2	352

6.5.2F.1	Error Vector Magnitude	352

6.5.2F.2	Carrier leakage	352

6.5.2F.3	In-band emissions	352

6.5.2G	Transmit modulation quality for V2X Communication	353

6.5.2G.1	Error Vector Magnitude	353

6.5.2G.2	Carrier leakage	354

6.5.2G.3	In-band emissions	354

6.5.2G.4	EVM equalizer spectrum flatness	354

6.6	Output RF spectrum emissions	354

6.6.1	Occupied bandwidth	354

6.6.1.1	Additional minimum requirement for E-UTRA (network signalled value “NS_29”)	355

6.6.1A	Occupied bandwidth for CA	355

6.6.1B	 Occupied bandwidth for UL-MIMO	355

6.6.1F	Occupied bandwidth for category NB1 and NB2	356

6.6.1G	Occupied bandwidth for V2X Communication	356

6.6.2	Out of band emission	356

6.6.2.1	Spectrum emission mask	356

6.6.2.1.1	Minimum requirement	356

6.6.2.1A	Spectrum emission mask for CA	357

6.6.2.2	Additional spectrum emission mask	358

6.6.2.2.1	Minimum requirement (network signalled value "NS_03", “NS_11”, "NS_20", and “NS_21”)	358

6.6.2.2.2	Minimum requirement (network signalled value "NS_04")	359

6.6.2.2.3	Minimum requirement (network signalled value "NS_06" or “NS_07”)	359

6.6.2.2.4	Minimum requirement (network signalled value "NS_33" or “NS_34”)	360

6.6.2.2.5	Minimum requirement (network signalled value “NS_27” and “NS_43”)	360

6.6.2.2.6	Minimum requirement (network signalled value "NS_28”)	361

6.6.2.2.7	Minimum requirement (network signalled value "NS_35")	361

6.6.2.2A	Additional Spectrum Emission Mask for CA	362

6.6.2.2A.1	Minimum requirement (network signalled value "CA_NS_04")	362

6.6.2.2A.2	Minimum requirement CA_66B (network signalled value "CA_NS_09")	362

6.6.2.2A.3	Minimum requirement CA_66C (network signalled value "CA_NS_09")	363

6.6.2.2A.4	Minimum requirement CA_48C (network signalled value "CA_NS_10")	363

6.6.2.3	Adjacent Channel Leakage Ratio	364

6.6.2.3.1	Minimum requirement E-UTRA	364

6.6.2.3.1a	Additional minimum requirement for E-UTRA (network signalled value “NS_29”)	365

6.6.2.3.1A	Void	366

6.6.2.3.1Aa	Void	366

6.6.2.3.2	Minimum requirements UTRA	366

6.6.2.3.2A	Minimum requirement UTRA for CA	367

6.6.2.3.3A	Minimum requirements for CA E-UTRA	369

6.6.2.4	Void	370

6.6.2.4.1	Void	370

6.6.2A	Void	370

6.6.2B	 Out of band emission for UL-MIMO	370

6.6.2C	 Void	370

6.6.2D	 Out of band emission for ProSe	370

6.6.2F	Out of band emission for category NB1 and NB2	371

6.6.2F.1	Spectrum emission mask	371

6.6.2F.2	Additional Spectrum Emission Mask for Category NB1 and NB2	371

6.6.2F.2.1	Minimum requirement (network signalled value "NS_02")	371

6.6.2F.2.2	Minimum requirement (network signalled value "NS_03")	371

6.6.2F.3	Adjacent Channel Leakage Ratio for category NB1 and NB2	372

6.6.2G	 Out of band emission for V2X Communication	372

6.6.3	Spurious emissions	373

6.6.3.1	Minimum requirements	373

6.6.3.1A	Minimum requirements for CA	374

6.6.3.2	Spurious emission band UE co-existence	375

6.6.3.2A	Spurious emission band UE co-existence for CA	383

6.6.3.3	Additional spurious emissions	398

6.6.3.3.1 	Minimum requirement (network signalled value "NS_05")	398

6.6.3.3.2 	Minimum requirement (network signalled value “NS_07”)	399

6.6.3.3.3 	Minimum requirement (network signalled value “NS_08”)	399

6.6.3.3.4 	Minimum requirement (network signalled value “NS_09”)	399

6.6.3.3.5 	Minimum requirement (network signalled value "NS_12")	400

6.6.3.3.6 	Minimum requirement (network signalled value “NS_13”)	400

6.6.3.3.7 	Minimum requirement (network signalled value “NS_14”)	400

6.6.3.3.8 	Minimum requirement (network signalled value “NS_15”)	400

6.6.3.3.9 	Minimum requirement (network signalled value “NS_16”)	401

6.6.3.3.10 	Minimum requirement (network signalled value “NS_17”)	401

6.6.3.3.11 	Minimum requirement (network signalled value “NS_18”)	401

6.6.3.3.12 	Minimum requirement (network signalled value “NS_19”)	401

6.6.3.3.13 	Minimum requirement (network signalled value “NS_11”)	402

6.6.3.3.14	Minimum requirement (network signalled value “NS_20”)	402

6.6.3.3.15	Minimum requirement (network signalled value “NS_21”)	402

6.6.3.3.16	Minimum requirement (network signalled value "NS_22")	403

6.6.3.3.17	Minimum requirement (network signalled value “NS_23”)	403

6.6.3.3.18	Void	404

6.6.3.3.19	Minimum requirement (network signalled value "NS_04")	404

6.6.3.3.20	Minimum requirement (network signalled value “NS_24”)	404

6.6.3.3.21	Minimum requirement (network signalled value “NS_25”)	405

6.6.3.3.22	Minimum requirement (network signalled value “NS_26”)	405

6.6.3.3.23	Minimum requirement (network signalled value “NS_27” and “NS_43”)	405

6.6.3.3.24	Minimum requirement (network signalled value “NS_28”)	405

6.6.3.3.25	Minimum requirement (network signalled value “NS_29”)	406

6.6.3.3.26	Minimum requirement (network signalled value “NS_30”)	406

6.6.3.3.27	Minimum requirement (network signalled value “NS_31”)	407

6.6.3.3.28	Minimum requirement (network signalled value “NS_36”)	408

6.6.3.3.29	Minimum requirement (network signalled value “NS_38”)	408

6.6.3.3.30	Minimum requirement (network signalled value “NS_39”)	408

6.6.3.3.31	Minimum requirement (network signalled value “NS_40” and “NS_41”)	409

6.6.3.3.32	Minimum requirement (network signalled value “NS_42”)	409

6.6.3.3.33	Minimum requirement (network signalled value “NS_44”)	409

6.6.3.3.34	Minimum requirement (network signalled value “NS_45”)	409

6.6.3.3A	Additional spurious emissions for CA	410

6.6.3.3A.1	Minimum requirement for CA_1C (network signalled value "CA_NS_01")	410

6.6.3.3A.2 	Minimum requirement for CA_1C (network signalled value "CA_NS_02")	410

6.6.3.3A.3	Minimum requirement for CA_1C (network signalled value "CA_NS_03")	411

6.6.3.3A.4	Minimum requirement for CA_38C (network signalled value "CA_NS_05")	411

6.6.3.3A.5	Minimum requirement for CA_7C (network signalled value "CA_NS_06")	411

6.6.3.3A.6	Minimum requirement for CA_39C and CA_39C-41A (network signalled value "CA_NS_07")	412

6.6.3.3A.7	Minimum requirement for CA_42C (network signalled value "CA_NS_08")	412

6.6.3.3A.8	Minimum requirement for CA_41C (network signalled value "CA_NS_04")	412

6.6.3.3A.9	Void	413

6.6.3.3A.10	Minimum requirement for CA_48C (network signalled value "CA_NS_10")	413

6.6.3A	Void	413

6.6.3B	 Spurious emission for UL-MIMO	413

6.6.3C	 Void	413

6.6.3D	 Spurious emission for ProSe	413

6.6.3F	Spurious emission for category NB1 and NB2	414

6.6.3G	Spurious emission for V2X Communication	414

6.6A	Void	418

6.6B	Void	418

6.7	Transmit intermodulation	418

6.7.1	Minimum requirement	418

6.7.1A	Minimum requirement for CA	419

6.7.1B	Minimum requirement for UL-MIMO	419

6.7.1F	Minimum requirement for category NB1 and NB2	419

6.7.1G	Minimum requirement for V2X Communication	420

6.8	Void	420

6.8A	Void	420

6.8B	 Time alignment error for UL-MIMO	420

6.8B.1	 Minimum Requirements	420

6.8C	Void	421

6.8D	Void	421

6.8E	Void	421

6.8F	Void	421

6.8G	Time alignment error	421

7	Receiver characteristics	421

7.1	General	421

7.2	Diversity characteristics	422

7.3	Reference sensitivity power level	422

7.3.1	Minimum requirements (QPSK)	422

7.3.1A	Minimum requirements (QPSK) for CA	457

7.3.1B	Minimum requirements (QPSK) for UL-MIMO	557

7.3.1D	Minimum requirements (QPSK) for ProSe	557

7.3.1E	Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis	559

7.3.1F	Minimum requirements for UE category NB1 and NB2	567

7.3.1F.1	Reference sensitivity for UE category NB1 and NB2	567

7.3.1F.2	Void	568

7.3.1G	Minimum requirements (QPSK) for V2X	568

7.3.2	Void	570

7.4	Maximum input level	570

7.4.1	Minimum requirements	571

7.4.1A	Minimum requirements for CA	571

7.4.1B	Minimum requirements for UL-MIMO	572

7.4.1D	Minimum requirements for ProSe	572

7.4.1F	Minimum requirements for category NB1 and NB2	573

7.4.1G	Minimum requirements for V2X	573

7.4A	Void	574

7.4A.1	Void	574

7.5	Adjacent Channel Selectivity (ACS)	574

7.5.1	Minimum requirements	574

7.5.1A	Minimum requirements for CA	575

7.5.1B	Minimum requirements for UL-MIMO	579

7.5.1D	Minimum requirements for ProSe	579

7.5.1F	Minimum requirements for category NB1 and NB2	580

7.5.1G	Minimum requirements for V2X	581

7.6	Blocking characteristics	583

7.6.1	In-band blocking	583

7.6.1.1	Minimum requirements	583

7.6.1.1A	Minimum requirements for CA	585

7.6.1.1D	Minimum requirements for ProSe	588

7.6.1.1F	Minimum requirements for category NB1 and NB2	589

7.6.1.1G	Minimum requirements for V2X	589

7.6.2	Out-of-band blocking	591

7.6.2.1	Minimum requirements	591

7.6.2.1A	Minimum requirements for CA	593

7.6.2.1D	Minimum requirements for ProSe	596

7.6.2.1F	Minimum requirements for category NB1 and NB2	596

7.6.2.1G	Minimum requirements for V2X	597

7.6.3	Narrow band blocking	598

7.6.3.1	Minimum requirements	598

7.6.3.1A	Minimum requirements for CA	599

7.6.3.1D	Minimum requirements for ProSe	600

7.6A	Void	601

7.6B	Blocking characteristics for UL-MIMO	601

7.7	Spurious response	601

7.7.1	Minimum requirements	601

7.7.1A	Minimum requirements for CA	602

7.7.1B	Minimum requirements for UL-MIMO	603

7.7.1D	Minimum requirements for ProSe	603

7.7.1F	Minimum requirements for UE category NB1 and NB2	604

7.7.1G	Minimum requirements for V2X	604

7.8	Intermodulation characteristics	605

7.8.1	Wide band intermodulation	605

7.8.1.1	Minimum requirements	605

7.8.1A	Minimum requirements for CA	606

7.8.1B	Minimum requirements for UL-MIMO	609

7.8.1D	Minimum requirements for ProSe	609

7.8.1F	Minimum requirements for category NB1 and NB2	610

7.8.1G	Minimum requirements	610

7.8.2	Void	611

7.9	Spurious emissions	611

7.9.1	Minimum requirements	612

7.9.1A	Minimum requirements	612

7.10	Receiver image	613

7.10.1	Void	613

7.10.1A	Minimum requirements for CA	613

7.10.1G	Minimum requirements for V2X Communication	613

8	Performance requirement	614

8.1	General	614

8.1.1	Receiver antenna capability	614

8.1.1.1	Simultaneous unicast and MBMS operations	615

8.1.1.2	Dual-antenna receiver capability in idle mode	615

8.1.2	Applicability of requirements	615

8.1.2.1	Applicability of requirements for different channel bandwidths	615

8.1.2.2	Definition of CA capability	615

8.1.2.2A	Definition of dual connectivity capability	620

8.1.2.3	Applicability and test rules for different CA configurations and bandwidth combination sets	621

8.1.2.3A	Applicability and test rules for different dual connectivity configuration and bandwidth combination set	623

8.1.2.3B	Applicability and test rules for different TDD-FDD CA configurations and bandwidth combination sets	624

8.1.2.3C	Applicability and test rules for SDR tests for 4Rx capable UEs	626

8.1.2.3D	Applicability and test rules for different CA with LAA SCell(s) configurations and bandwidth combination sets	626

8.1.2.3E	Applicability and test rules for SDR tests for 8Rx capable UEs	627

8.1.2.4	Test coverage for different number of component carriers	628

8.1.2.5	Applicability of performance requirements for Type B receiver	629

8.1.2.6	Applicability of performance requirements for 4Rx capable UEs	629

8.1.2.6.1	Applicability rule and antenna connection for single carrier tests with 2Rx	629

8.1.2.6.2	Applicability rule and antenna connection for CA and DC tests with 2Rx	631

8.1.2.6.3	Applicability rule and antenna connection for single carrier tests with 4Rx	631

8.1.2.6.4	Applicability rule for 256QAM tests	632

8.1.2.6.5	Applicability rule and antenna connection for CA and DC tests with 4Rx	632

8.1.2.6.6	Applicability rule for Type C with 4Rx	636

8.1.2.6.7	Applicability rule for 1024QAM tests	636

8.1.2.7	Applicability of Enhanced Downlink Control Channel Performance Requirements	636

8.1.2.8	Applicability of performance requirements for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations	638

8.1.2.8A	Applicability of performance requirements for UE supporting coverage enhancement	638

8.1.2.9	Applicability of SDR requirements for CA and LAA	639

8.1.2.10	Applicability of performance requirements for Multi-user Superposed Transmission	640

8.1.2.11	Applicability CRS interference mitigation receivers performance requirements	640

8.1.2.12	Applicability of performance requirements for 8Rx capable UEs	641

8.1.2.12.1	Applicability rule and antenna connection for single carrier PDSCH tests	641

8.1.2.12.2	Applicability rule and antenna connection for control channel tests	647

8.1.2.12.3	Applicability rule and antenna connection for CA and DC tests	647

8.1.3	UE category and UE DL category	648

8.2	Demodulation of PDSCH (Cell-Specific Reference Symbols)	648

8.2.1	FDD (Fixed Reference Channel)	648

8.2.1.1	Single-antenna port performance	649

8.2.1.1.1	Minimum Requirement	649

8.2.1.1.2	Void	655

8.2.1.1.3	Void	655

8.2.1.1.4	Minimum Requirement 1 PRB allocation in presence of MBSFN	655

8.2.1.1.4A	Minimum Requirement 1 PRB allocation in presence of FeMBMS Unicast-mixed Cell under CA	655

8.2.1.2	Transmit diversity performance	656

8.2.1.2.1	Minimum Requirement 2 Tx Antenna Port	656

8.2.1.2.2	Minimum Requirement 4 Tx Antenna Port	657

8.2.1.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	657

8.2.1.2.3A	Minimum Requirement 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	659

8.2.1.2.4	Enhanced Performance Requirement Type A - 2 Tx Antenna Ports with TM3 interference model	661

8.2.1.2.5	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM2 interference model	663

8.2.1.2.6	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM9 interference model	664

8.2.1.2.7	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	665

8.2.1.3	Open-loop spatial multiplexing performance	665

8.2.1.3.1	Minimum Requirement 2 Tx Antenna Port	665

8.2.1.3.1A	Soft buffer management test	669

8.2.1.3.1B	Enhanced Performance Requirement Type C –2Tx Antenna Ports	670

8.2.1.3.1C	Enhanced Performance Requirement Type C - 2 Tx Antenna Ports with TM1 interference	671

8.2.1.3.2	Minimum Requirement 4 Tx Antenna Port	672

8.2.1.3.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	672

8.2.1.3.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	676

8.2.1.3.5	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	678

8.2.1.3.6	Minimum Requirement 2 Tx Antenna Port (network-based CRS interference mitigation)	679

8.2.1.4	Closed-loop spatial multiplexing performance	680

8.2.1.4.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port	680

8.2.1.4.1A	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port	681

8.2.1.4.1B	Enhanced Performance Requirement Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	681

8.2.1.4.1C	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	683

8.2.1.4.1D	Enhanced Performance Requirement Type B - Single-layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	686

8.2.1.4.1E	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports with CRS assistance information	688

8.2.1.4.1F	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Ports with CRS assistance information	689

8.2.1.4.2	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	690

8.2.1.4.2A	Enhanced Performance Requirement Type C – Multi-layer Spatial Multiplexing 2Tx Antenna Ports	691

8.2.1.4.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	691

8.2.1.4.3A	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	695

8.2.1.4.4	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port (Superposed transmission)	697

8.2.1.5	MU-MIMO	698

8.2.1.6	[Control channel performance: D-BCH and PCH]	698

8.2.1.7	Carrier aggregation with power imbalance	698

8.2.1.7.1	Minimum Requirement	698

8.2.1.8	Intra-band non-contiguous carrier aggregation with timing offset	699

8.2.1.8.1	Minimum Requirement	699

8.2.1.9	HST-SFN performance	700

8.2.1.9.1	Minimum Requirement	700

8.2.1.9.2	Minimum Requirement for Rel-16 further enhanced HST	703

8.2.2	TDD (Fixed Reference Channel)	703

8.2.2.1	Single-antenna port performance	704

8.2.2.1.1	Minimum Requirement	704

8.2.2.1.2	Void	709

8.2.2.1.3	Void	709

8.2.2.1.4	Minimum Requirement 1 PRB allocation in presence of MBSFN	709

8.2.2.2	Transmit diversity performance	709

8.2.2.2.1	Minimum Requirement 2 Tx Antenna Port	709

8.2.2.2.2	Minimum Requirement 4 Tx Antenna Port	710

8.2.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	711

8.2.2.2.3A	Minimum Requirement 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	712

8.2.2.2.4	Enhanced Performance Requirement Type A – 2 Tx Antenna Ports with TM3 interference model	714

8.2.2.2.5	Minimum Requirement 2 Tx Antenna Port (when EIMTA-MainConfigServCell-r12 is configured)	716

8.2.2.2.6	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM2 interference model	716

8.2.2.2.7	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM9 interference model	718

8.2.2.2.8	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	719

8.2.2.3	Open-loop spatial multiplexing performance	720

8.2.2.3.1	Minimum Requirement 2 Tx Antenna Port	720

8.2.2.3.1A	Soft buffer management test	723

8.2.2.3.1B	Enhanced Performance Requirement Type C - 2Tx Antenna Ports	723

8.2.2.3.1C	Enhanced Performance Requirement Type C - 2 Tx Antenna Ports with TM1 interference	724

8.2.2.3.2	Minimum Requirement 4 Tx Antenna Port	725

8.2.2.3.3	Minimum Requirement 2Tx antenna port (demodulation subframe overlaps with aggressor cell ABS)	726

8.2.2.3.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	730

8.2.2.3.5	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	732

8.2.2.3.6	Minimum Requirement 2 Tx Antenna Port (network-based CRS interference mitigation)	733

8.2.2.4	Closed-loop spatial multiplexing performance	734

8.2.2.4.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port	734

8.2.2.4.1A	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port	735

8.2.2.4.1B	Enhanced Performance Requirement Type A – Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	735

8.2.2.4.1C	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	737

8.2.2.4.1D	Enhanced Performance Requirement Type B - Single-layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	739

8.2.2.4.1E	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports with CRS assistance information	741

8.2.2.4.1F	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Ports with CRS assistance information	742

8.2.2.4.2	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	744

8.2.2.4.2A	Enhanced Performance Requirement Type C Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	744

8.2.2.4.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	745

8.2.2.4.3A	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	749

8.2.2.4.4	Void	750

8.2.2.4.5	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port (Superposed transmission)	750

8.2.2.5	MU-MIMO	751

8.2.2.6	[Control channel performance: D-BCH and PCH]	751

8.2.2.7	Carrier aggregation with power imbalance	751

8.2.2.7.1	Minimum Requirement	751

8.2.2.8	Intra-band contiguous carrier aggregation with minimum channel spacing	752

8.2.2.8.1	Minimum Requirement	752

8.2.2.9	HST-SFN performance	753

8.2.2.9.1	Minimum Requirement	753

8.2.2.9.2	Minimum Requirement for Rel-16 further enhanced HST	755

8.2.3	TDD FDD CA (Fixed Reference Channel)	756

8.2.3.1	Single-antenna port performance	757

8.2.3.1.1	Minimum Requirement for FDD PCell	757

8.2.3.1.2	Minimum Requirement for TDD PCell	761

8.2.3.2	Open-loop spatial multiplexing performance 2Tx Antenna port	765

8.2.3.2.1	Minimum Requirement for FDD PCell	765

8.2.3.2.1A	Soft buffer management test for FDD PCell	769

8.2.3.2.2	Minimum Requirement for TDD PCell	770

8.2.3.2.2A	Soft buffer management test for TDD PCell	774

8.2.3.3	Closed-loop spatial multiplexing performance 4Tx Antenna Port	775

8.2.3.3.1	Minimum Requirement for FDD PCell	775

8.2.3.3.2	Minimum Requirement for TDD PCell	779

8.2.3.4	Minimum Requirement for Closed-loop spatial multiplexing performance 4Tx Antenna Port for dual connectivity	783

8.2.3.5	HST-SFN performance	785

8.2.3.5.0	General	785

8.2.3.5.1	Minimum Requirement for FDD PCell	785

8.2.3.5.2	Minimum Requirement for TDD PCell	788

8.2.4	LAA	791

8.2.4.1	Closed-loop spatial multiplexing performance 4Tx Antenna Port	791

8.2.4.1.1	FDD PCell (FDD single carrier)	791

8.2.4.1.2	TDD PCell (TDD single carrier)	795

8.3	Demodulation of PDSCH (User-Specific Reference Symbols)	798

8.3.1	FDD	798

8.3.1.1	Single-layer Spatial Multiplexing	799

8.3.1.1A	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model	801

8.3.1.1B	Single-layer Spatial Multiplexing (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	804

8.3.1.1C	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM9 interference model	807

8.3.1.1D	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with CRS interference model	809

8.3.1.1E	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM3 interference model	810

8.3.1.1F	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM10 serving cell configuration and TM9 interference model	811

8.3.1.1G	Single-layer Spatial Multiplexing (CRS assistance information is configured)	813

8.3.1.1H	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	815

8.3.1.1I	Single-layer Spatial Multiplexing (with assistance information for simultaneous transmition interfering PDSCH)	816

8.3.1.2	Dual-Layer Spatial Multiplexing	818

8.3.1.2A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	819

8.3.1.3	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	820

8.3.1.3.1	Minimum requirement with Same Cell ID (with single NZP CSI-RS resource)	820

8.3.1.3.2	Minimum requirements with Same Cell ID (with multiple NZP CSI-RS resources)	822

8.3.1.3.3	Minimum requirement with Different Cell ID and Colliding CRS (with single NZP CSI-RS resource)	824

8.3.1.3.4	Minimum requirement with Different Cell ID and non-colliding CRS (with single NZP CSI-RS resource and CRS assistance information is configured)	826

8.3.1.3.5	Minimum requirements with different Cell ID and non-colliding CRS (with multiple NZP CSI-RS resources and CRS assistance information is configured)	828

8.3.1.3.6	Minimum requirements for QCL Type C and 2 Layers Spatial Multiplexing	831

8.3.1.4	Performance Requirements for semiOpenLoop transmission	833

8.3.2	TDD	835

8.3.2.1	Single-layer Spatial Multiplexing	835

8.3.2.1A	Single-layer Spatial Multiplexing (with multiple CSI-RS configurations)	837

8.3.2.1B	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model	839

8.3.2.1C	Single-layer Spatial Multiplexing (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	842

8.3.2.1D	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM9 interference	845

8.3.2.1E	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with CRS interference model	847

8.3.2.1F	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM3 interference	849

8.3.2.1G	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM10 serving cell configuration and TM9 interference model	850

8.3.2.1H	Single-layer Spatial Multiplexing (CRS assistance information is configured)	852

8.3.2.1I	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	854

8.3.2.1J	Single-layer Spatial Multiplexing (with assistance information for simultaneous transmition interfering PDSCH)	855

8.3.2.2	Dual-Layer Spatial Multiplexing	857

8.3.2.2A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	857

8.3.2.3	Dual-Layer Spatial Multiplexing (with multiple CSI-RS configurations)	858

8.3.2.4	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	859

8.3.2.4.1	Minimum requirement with Same Cell ID (with single NZP CSI-RS resource)	859

8.3.2.4.2	Minimum requirements with Same Cell ID (with multiple NZP CSI-RS resources)	861

8.3.2.4.3	Minimum requirement with Different Cell ID and Colliding CRS (with single NZP CSI-RS resource)	863

8.3.2.4.4	Minimum requirement with Different Cell ID and non-Colliding CRS (with single NZP CSI-RS resource and CRS assistance information is configured)	865

8.3.2.4.5	Minimum requirements with different Cell ID and non-colliding CRS (with multiple NZP CSI-RS resources and CRS assistance information is configured)	867

8.3.2.4.6	Minimum requirements for QCL Type C and 2 Layers Spatial Multiplexing	870

8.3.2.5	Performance Requirements for semiOpenLoop transmission	872

8.3.3	LAA	874

8.3.3.1	Dual-Layer Spatial Multiplexing with DM-RS	874

8.3.3.1.1	FDD PCell (FDD single carrier)	874

8.3.3.1.2	TDD Pcell (TDD single carrier)	878

8.4	Demodulation of PDCCH/PCFICH	882

8.4.1	FDD	882

8.4.1.1	Single-antenna port performance	883

8.4.1.2	Transmit diversity performance	883

8.4.1.2.1	Minimum Requirement 2 Tx Antenna Port	883

8.4.1.2.2	Minimum Requirement 4 Tx Antenna Port	883

8.4.1.2.3 Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	884

8.4.1.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	889

8.4.1.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port under Asynchronous Network	895

8.4.1.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	896

8.4.1.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	897

8.4.1.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	898

8.4.1.2.9	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	899

8.4.2	TDD	900

8.4.2.1	Single-antenna port performance	901

8.4.2.2	Transmit diversity performance	901

8.4.2.2.1	Minimum Requirement 2 Tx Antenna Port	901

8.4.2.2.2	Minimum Requirement 4 Tx Antenna Port	902

8.4.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	902

8.4.2.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	906

8.4.2.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	910

8.4.2.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	911

8.4.2.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	912

8.4.2.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	913

8.4.2.2.9	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	914

8.4.3	LAA	915

8.4.3.1	Transmit diversity performance	915

8.4.3.1.1	FDD Pcell (FDD single carrier)	915

8.4.3.1.2	TDD Pcell (TDD single carrier)	916

8.5	Demodulation of PHICH	917

8.5.1	FDD	918

8.5.1.1	Single-antenna port performance	918

8.5.1.2	Transmit diversity performance	918

8.5.1.2.1	Minimum Requirement 2 Tx Antenna Port	918

8.5.1.2.2	Minimum Requirement 4 Tx Antenna Port	919

8.5.1.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	919

8.5.1.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	921

8.5.1.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports under Asynchronous Network	924

8.5.1.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	925

8.5.1.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	926

8.5.1.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	927

8.5.2	TDD	928

8.5.2.1	Single-antenna port performance	929

8.5.2.2	Transmit diversity performance	929

8.5.2.2.1	Minimum Requirement 2 Tx Antenna Port	929

8.5.2.2.2	Minimum Requirement 4 Tx Antenna Port	930

8.5.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	930

8.5.2.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	932

8.5.2.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	934

8.5.2.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	935

8.5.2.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	936

8.5.2.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	937

8.6	Demodulation of PBCH	938

8.6.1	FDD	938

8.6.1.1	Single-antenna port performance	938

8.6.1.2	Transmit diversity performance	939

8.6.1.2.1	Minimum Requirement 2 Tx Antenna Port	939

8.6.1.2.2	Minimum Requirement 4 Tx Antenna Port	939

8.6.1.2.3	Minimum Requirement 2 Tx Antenna Port under Time Domain Measurement Resource Restriction with CRS Assistance Information	939

8.6.2	TDD	941

8.6.2.1	Single-antenna port performance	941

8.6.2.2	Transmit diversity performance	941

8.6.2.2.1	Minimum Requirement 2 Tx Antenna Port	941

8.6.2.2.2	Minimum Requirement 4 Tx Antenna Port	941

8.6.2.2.3	Minimum Requirement 2 Tx Antenna Port under Time Domain Measurement Resource Restriction with CRS Assistance Information	942

8.7	Sustained downlink data rate provided by lower layers	943

8.7.1	FDD (single carrier and CA)	943

8.7.2	TDD (single carrier and CA)	960

8.7.3	FDD (EPDCCH scheduling)	964

8.7.4	TDD (EPDCCH scheduling)	966

8.7.5	TDD FDD CA	968

8.7.5.1	Minimum Requirement FDD PCell	969

8.7.5.2	Minimum Requirement TDD PCell	977

8.7.6	FDD (DC)	986

8.7.7	TDD (DC)	993

8.7.8	TDD FDD (DC)	996

8.7.9	FDD (4 Rx)	999

8.7.10	TDD (4 Rx)	1001

8.7.11	TDD FDD CA (4 Rx)	1003

8.7.11.1	Void	1005

8.7.12	LAA	1005

8.7.12.1	FDD CA in licensed bands	1005

8.7.12.2	TDD CA in licensed bands	1007

8.7.12.3	TDD-FDD CA in licensed bands	1009

8.7.13	FDD DC (4 Rx)	1012

8.7.14	TDD DC (4 Rx)	1013

8.7.15	TDD FDD DC (4 Rx)	1015

8.7.16	FDD (1024QAM and up to 4Rx supported)	1017

8.7.17	TDD (1024QAM and up to 4 Rx supported)	1020

8.7.18	TDD FDD CA (1024QAM and up to 4 Rx supported)	1022

8.7.19	TDD (8 Rx)	1024

8.8	Demodulation of EPDCCH	1026

8.8.1	Distributed Transmission	1026

8.8.1.1	FDD	1026

8.8.1.1.1	Void	1027

8.8.1.2	TDD	1027

8.8.1.2.1	Void	1028

8.8.2	Localized Transmission with TM9	1028

8.8.2.1	FDD	1028

8.8.2.1.1	Void	1029

8.8.2.1.2	Void	1030

8.8.2.2	TDD	1030

8.8.2.2.1	Void	1031

8.8.2.2.2	Void	1031

8.8.3	Localized transmission with TM10 Type B quasi co-location type	1031

8.8.3.1	FDD	1031

8.8.3.2	TDD	1034

8.8.4	Enhanced Downlink Control Channel Performance Requirements Type A - Localized Transmission with CRS Interference Model	1037

8.8.4.1	FDD	1037

8.8.4.2	TDD	1038

8.8.5	Enhanced Downlink Control Channel Performance Requirements Type A - Distributed Transmission with TM9 Interference Model	1040

8.8.5.1	TDD	1040

8.8.6	Enhanced Downlink Control Channel Performance Requirements Type A - Distributed Transmission with TM3 Interference Model	1041

8.8.6.1	FDD	1041

8.9	Demodulation (single receiver antenna)	1042

8.9.1	PDSCH	1042

8.9.1.1	FDD and half-duplex FDD (Fixed Reference Channel)	1042

8.9.1.1.1	Transmit diversity performance (Cell-Specific Reference Symbols)	1042

8.9.1.1.2	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1043

8.9.1.1.3	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1045

8.9.1.2	TDD (Fixed Reference Channel)	1048

8.9.1.2.1	Transmit diversity performance (Cell-Specific Reference Symbols)	1048

8.9.1.2.2	 Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1049

8.9.1.2.3	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1052

8.9.2	PHICH	1054

8.9.2.1	FDD and half-duplex FDD	1054

8.9.2.1.1	Transmit diversity performance	1054

8.9.2.2	TDD	1054

8.9.2.2.1	Transmit diversity performance	1054

8.9.3	PBCH	1055

8.9.3.1	FDD and half-duplex FDD	1055

8.9.3.1.1	Transmit diversity performance	1055

8.9.3.2	TDD	1055

8.9.3.2.1	Transmit diversity performance	1055

8.9.4	PDCCH/PCFICH	1055

8.9.4.1	FDD and half-duplex FDD	1055

8.9.4.1.1	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1055

8.9.4.1.2	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1056

8.9.4.2	TDD	1057

8.9.4.2.1	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1057

8.9.4.2.2	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1058

8.10	Demodulation (4 receiver antenna ports)	1059

8.10.1	PDSCH	1059

8.10.1.1	FDD (Fixed Reference Channel)	1059

8.10.1.1.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1060

8.10.1.1.1A	Transmit diversity performance wit Enhanced Performance Requirement Type A - 2 Tx Antenna Ports with TM3 interference model	1061

8.10.1.1.2	Open-loop spatial multiplexing performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1062

8.10.1.1.3	Closed-loop spatial multiplexing Enhanced Performance Requirements Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	1062

8.10.1.1.4	Closed-loop spatial multiplexing performance, Dual-Layer Spatial Multiplexing 4 Tx Antenna Port (Cell-Specific Reference Symbols)	1063

8.10.1.1.4A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing with 2Tx Antenna Ports	1064

8.10.1.1.5	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1065

8.10.1.1.5A	Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1068

8.10.1.1.5B	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	1069

8.10.1.1.6	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1070

8.10.1.1.6A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	1072

8.10.1.1.6B	Dual-Layer Spatial Multiplexing with altCQI-Table-1024QAM configured (User-Specific Reference Symbols)	1073

8.10.1.1.7	Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1074

8.10.1.1.7A	Enhanced Performance Requirement Type C - Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1074

8.10.1.1.8	Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas (Cell-Specific Reference Symbols)	1075

8.10.1.1.9	4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1075

8.10.1.1.9A	Enhanced Performance Requirement Type C - 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1077

8.10.1.1.10	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1078

8.10.1.1.11	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 4 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1079

8.10.1.1.12	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing with CRS assistance information (User-Specific Reference Symbols)	1080

8.10.1.1.13	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	1081

8.10.1.1.14	HST-SFN performance	1085

8.10.1.2	TDD (Fixed Reference Channel)	1085

8.10.1.2.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1086

8.10.1.2.1A	Transmit diversity performance with Enhanced Performance Requirement Type A – 2 Tx Antenna Ports with TM3 interference model	1087

8.10.1.2.2	Open-loop spatial multiplexing performance  with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1088

8.10.1.2.3	 Closed-loop spatial multiplexing Enhanced Performance Requirements Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	1088

8.10.1.2.4	Closed-loop spatial multiplexing performance, Dual-Layer Spatial Multiplexing 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1089

8.10.1.2.4A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing with 2Tx Antenna Ports	1090

8.10.1.2.5	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1091

8.10.1.2.5A	Single-layer Spatial Multiplexing (with multiple CSI-RS configurations)	1094

8.10.1.2.5B	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	1095

8.10.1.2.6	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1096

8.10.1.2.6A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	1098

8.10.1.2.6B	Dual-Layer Spatial Multiplexing with altCQI-Table-1024QAM configured (User-Specific Reference Symbols)	1099

8.10.1.2.7	Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1100

8.10.1.2.7A	Enhanced Performance Requirement Type C - Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1100

8.10.1.2.8	Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas	1101

8.10.1.2.9	4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1102

8.10.1.2.9A	Enhanced Performance Requirement Type C - 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1103

8.10.1.2.10	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1104

8.10.1.2.11	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 4 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1106

8.10.1.2.12	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing with CRS assistance information (User-Specific Reference Symbols)	1107

8.10.1.2.13	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	1109

8.10.1.2.14	HST-SFN performance	1113

8.10.2	PDCCH/PCFICH	1114

8.10.2.1	FDD	1114

8.10.2.1.1	Single-antenna port performance	1114

8.10.2.1.2	Transmit diversity performance with 2 Tx Antenna Ports	1114

8.10.2.1.3	Transmit diversity performance with 4 Tx Antenna Ports	1115

8.10.2.1.4	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1115

8.10.2.2	TDD	1117

8.10.2.2.1	Single-antenna port performance	1117

8.10.2.2.2	Transmit diversity performance with 2 Tx Antenna Ports	1117

8.10.2.2.3	Transmit diversity performance with 4 Tx Antenna Ports	1118

8.10.2.2.4	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1118

8.10.3	PHICH	1119

8.10.3.1	 FDD	1119

8.10.3.1.1	Single Tx Antenna Port performance	1119

8.10.3.1.2	Transmit diversity performance with 2 Tx Antenna Ports	1120

8.10.3.1.3	Transmit diversity performance with 4 Tx Antenna Ports	1120

8.10.3.2	TDD	1120

8.10.3.2.1	Single Tx Antenna Port performance	1121

8.10.3.2.2	Transmit diversity performance with 2 Tx Antenna Ports	1121

8.10.3.2.3	Transmit diversity performance with 4 Tx Antenna Ports	1122

8.10.4	ePDCCH	1122

8.10.4.1	Distributed Transmission with 4Rx	1122

8.10.4.1.1	FDD	1122

8.10.4.1.2	TDD	1123

8.10.4.2	Localized Transmission with TM9 and 4Rx	1124

8.10.4.2.1	FDD	1124

8.10.4.2.2	TDD	1125

8.11	Demodulation (UE supporting coverage enhancement)	1126

8.11.1	PDSCH	1127

8.11.1.1	FDD and half-duplex FDD (Fixed Reference Channel)	1127

8.11.1.1.1	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1127

8.11.1.1.2	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1131

8.11.1.1.3	Transmit diversity performance (Cell-Specific Reference Symbols)	1133

8.11.1.2	TDD (Fixed Reference Channel)	1138

8.11.1.2.1	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1138

8.11.1.2.2	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1143

8.11.1.2.3	Transmit diversity performance (Cell-Specific Reference Symbols)	1145

8.11.2	MPDCCH	1149

8.11.2.1	FDD and half-duplex FDD	1150

8.11.2.1.1	CE Mode A	1151

8.11.2.1.2	CE Mode B	1152

8.11.2.1.3	CE Mode A with TM9 interference model	1152

8.11.2.1.4	CE Mode A with CRS interference model	1154

8.11.2.2	TDD	1155

8.11.2.2.1	CE Mode A	1156

8.11.2.2.2	CE Mode B	1157

8.11.2.2.3	CE Mode A with TM9 interference model	1157

8.11.2.2.4	CE Mode A with CRS interference model	1158

8.11.3	PBCH	1159

8.11.3.1	FDD and half-duplex FDD	1160

8.11.3.1.1	Transmit diversity performance	1160

8.11.3.2	TDD	1161

8.11.3.2.1	Transmit diversity performance	1161

8.12	Demodulation of Narrowband IoT	1162

8.12.1	NPDSCH	1162

8.12.1.1	Half-duplex FDD	1162

8.12.1.1.1	Minimum Requirements for In-band	1162

8.12.1.1.2	Minimum Requirements for Standalone/Guard-band	1163

8.12.1.1.3	Minimum Requirements for Standalone for UE Category NB2	1164

8.12.1.2	TDD	1165

8.12.1.2.1	Minimum Requirements for In-band	1166

8.12.1.2.2	Minimum Requirements for Standalone/Guard-band	1167

8.12.1.2.3	Minimum Requirements for Standalone for UE Category NB2	1167

8.12.2	NPDCCH	1168

8.12.2.1	Half-duplex FDD	1168

8.12.2.1.1	Single-antenna performance	1169

8.12.2.1.2	Transmit diversity performance	1170

8.12.2.2	TDD	1170

8.12.2.2.1	Single-antenna performance	1172

8.12.2.2.2	Transmit diversity performance	1172

8.12.3	Demodulation of NPBCH	1172

8.12.3.1	HD-FDD	1173

8.12.3.1.1	Single-antenna port performance with single NPBCH TTI	1173

8.12.3.1.2	Transmit diversity performance	1173

8.12.3.2	TDD	1173

8.12.3.2.1	Single-antenna port performance with single NPBCH TTI	1174

8.12.3.2.2	Transmit diversity performance	1174

8.13	Demodulation of PDSCH CA and DC(4 receiver antenna ports)	1174

8.13.1	FDD (CA and DC)	1174

8.13.1.1	Closed-loop spatial multiplexing performance	1174

8.13.1.1.1	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	1174

8.13.1.1.2	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	1178

8.13.1.1.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port with 256QAM	1180

8.13.1.1.4	Minimum Requirement Four-Layer Spatial Multiplexing 4 Tx Antenna Port	1182

8.13.1.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1183

8.13.1.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port	1183

8.13.1.3	Enhanced Performance Requirements Type A Closed-loop spatial multiplexing	1186

8.13.1.3.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	1186

8.13.1.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1188

8.13.1.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1188

8.13.2	TDD (CA and DC)	1191

8.13.2.1	Closed-loop spatial multiplexing performance	1192

8.13.2.1.1	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	1192

8.13.2.1.2	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	1194

8.13.2.1.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port with 256QAM	1195

8.13.2.1.4	Minimum Requirement Four-Layer Spatial Multiplexing 4 Tx Antenna Port	1196

8.13.2.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1198

8.13.2.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port	1198

8.13.2.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1202

8.13.2.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1202

8.13.3	TDD-FDD (CA and DC)	1204

8.13.3.1	 Closed-loop spatial multiplexing performance 4Tx Antenna Port	1205

8.13.3.1.1	Minimum Requirement for FDD PCell	1205

8.13.3.1.2	Minimum Requirement for TDD PCell	1208

8.13.3.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1212

8.13.3.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port for FDD PCell	1212

8.13.3.2.2	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port for TDD PCell	1215

8.13.3.3	Enhanced Performance Requirements Type A Closed-loop spatial multiplexing	1218

8.13.3.3.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols) for FDD PCell	1218

8.13.3.3.2	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols) for TDD PCell	1220

8.13.3.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1222

8.13.3.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols) for FDD PCell	1222

8.13.3.4.2	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols) for TDD PCell	1225

8.13.3.5	 Closed-loop spatial multiplexing performance 4Tx Antenna Port for DC	1228

8.13.3.5.1	Minimum Requirement for FDD PCell	1228

8.13.3.5.2	Minimum Requirement for TDD PCell	1230

8.13.3.6	 Closed-loop spatial multiplexing performance 4Tx Antenna Port with 256QAM	1232

8.13.3.6.1	Minimum Requirement for FDD PCell	1232

8.13.3.6.2	Minimum Requirement for TDD PCell	1234

8.13.3.7	 Closed-loop spatial multiplexing performance 4Tx Antenna Port with Four layers	1235

8.13.3.7.1	Minimum Requirement for FDD PCell	1235

8.13.3.7.2	Minimum Requirement for TDD PCell	1237

8.14	Demodulation (UE supporting Short TTI)	1239

8.14.1	Slot-PDSCH and Subslot-PDSCH	1239

8.14.1.1	FDD (Fixed Reference Channel)	1239

8.14.1.1.1	Open-loop spatial multiplexing performance	1239

8.14.1.1.2	Closed-loop spatial multiplexing performance (User-Specific Reference Signals)	1240

8.14.1.2	TDD (Fixed Reference Channel)	1242

8.14.1.2.1	Open-loop spatial multiplexing performance	1242

8.14.1.2.2	Closed-loop spatial multiplexing performance (User-Specific Reference Signals)	1243

8.14.2	SPDCCH	1244

8.14.2.1	FDD	1244

8.14.2.1.1	Mimimum requirement	1245

8.14.2.2	TDD	1245

8.14.2.2.1	Mimimum requirement	1246

8.15	Demodulation (8 receiver antenna ports)	1247

8.15.1	PDSCH	1247

8.15.1.1	Void	1247

8.15.1.2	TDD (Fixed Reference Channel)	1247

8.15.1.2.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1247

8.15.1.2.2	Open-loop spatial multiplexing performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1248

8.15.1.2.3	8 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1248

8.15.2	CA	1250

8.15.2.1	Void	1250

8.15.2.2	TDD	1250

8.15.2.2.1	Eight Layer Spatial Multiplexing (User-Specific Reference Symbols)	1250

9	Reporting of Channel State Information	1252

9.1	General	1252

9.1.1	Applicability of requirements	1252

9.1.1.1	Applicability of requirements for different channel bandwidths	1252

9.1.1.2	Applicability and test rules for different CA configurations and bandwidth combination sets	1252

9.1.1.2A	Applicability and test rules for different TDD-FDD CA configurations and bandwidth combination sets	1253

9.1.1.3	Test coverage for different number of componenet carriers	1254

9.1.1.4	Applicability of performance requirements for 4Rx capable UEs	1254

9.1.1.4.1	Applicability rule and antenna connection for single carrier tests with 2Rx	1254

9.1.1.4.2	Applicability rule and antenna connection for CA tests with 2Rx	1256

9.1.1.4.3	Applicability rule and antenna connection for single carrier tests with 4Rx	1257

9.1.1.5	Applicability of requirements for UEs supporting coverage enhancement	1257

9.2	 CQI reporting definition under AWGN conditions	1257

9.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbols)	1257

9.2.1.1	FDD	1257

9.2.1.2	TDD	1258

9.2.1.3	FDD (CSI measurements in case two CSI subframe sets are configured)	1259

9.2.1.4	TDD (CSI measurements in case two CSI subframe sets are configured)	1262

9.2.1.5	FDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1265

9.2.1.6	TDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1268

9.2.1.7	FDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1271

9.2.1.8	TDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1272

9.2.1.9	FDD (Modulation and TBS index Table 3 and 4-bit CQI Table 4 are used)	1272

9.2.1.10	TDD (Modulation and TBS index Table 3 and 4-bit CQI Table 4 are used)	1273

9.2.2	Minimum requirement PUCCH 1-1 (Cell-Specific Reference Symbols)	1274

9.2.2.1	FDD	1274

9.2.2.2	TDD	1275

9.2.3	Minimum requirement PUCCH 1-1 (CSI Reference Symbols)	1276

9.2.3.1	FDD	1276

9.2.3.1A	FDD (With channelMeasRestriction configured)	1277

9.2.3.2	TDD	1278

9.2.3.2A	TDD (With channelMeasRestriction configured)	1279

9.2.4	Minimum requirement PUCCH 1-1 (With Single CSI Process)	1280

9.2.4.1	FDD	1281

9.2.4.1A	FDD (With interferenceMeasRestriction configured)	1283

9.2.4.2	TDD	1286

9.2.4.2A	TDD (With interferenceMeasRestriction configured)	1289

9.2.5	Minimum requirement PUCCH 1-1 (when csi-SubframeSet –r12 and EIMTA-MainConfigServCell-r12 are configured)	1292

9.2.6	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbols)	1295

9.2.6.1	Frame structure type 3 with FDD Pcell	1295

9.2.6.2	Frame structure type 3 with TDD Pcell	1297

9.2.7	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1299

9.2.7.1	Frame structure type 3 wth FDD Pcell	1299

9.2.7.2	Frame structure type 3 wth TDD Pcell	1301

9.3	CQI reporting under fading conditions	1305

9.3.1	Frequency-selective scheduling mode	1305

9.3.1.1	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbols)	1305

9.3.1.1.1	FDD	1305

9.3.1.1.2	TDD	1306

9.3.1.1.3	FDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1307

9.3.1.1.4	TDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1311

9.3.1.1.5	TDD (when csi-SubframeSet –r12 is configured)	1314

9.3.1.2	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1316

9.3.1.2.1	FDD	1316

9.3.1.2.2	TDD	1317

9.3.1.2.3	FDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1319

9.3.1.2.4	TDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1320

9.3.1.2.5	Void	1321

9.3.1.2.6	TDD (when csi-SubframeSet –r12 is configured with one CSI process)	1321

9.3.2	Frequency non-selective scheduling mode	1325

9.3.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol)	1325

9.3.2.1.1	FDD	1325

9.3.2.1.2	TDD	1327

9.3.2.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	1329

9.3.2.2.1	FDD	1329

9.3.2.2.2	TDD	1330

9.3.3	Frequency-selective interference	1332

9.3.3.1	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbol)	1332

9.3.3.1.1	FDD	1332

9.3.3.1.2	TDD	1333

9.3.3.2	Void	1334

9.3.3.2.1	Void	1334

9.3.3.2.2	Void	1334

9.3.4	UE-selected subband CQI	1334

9.3.4.1	Minimum requirement PUSCH 2-0 (Cell-Specific Reference Symbols)	1334

9.3.4.1.1	FDD	1334

9.3.4.1.2	TDD	1335

9.3.4.2	Minimum requirement PUCCH 2-0 (Cell-Specific Reference Symbols)	1336

9.3.4.2.1	FDD	1336

9.3.4.2.2	TDD	1338

9.3.5	Additional requirements for enhanced receiver Type A	1340

9.3.5.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol)	1340

9.3.5.1.1	FDD	1340

9.3.5.1.2	TDD	1341

9.3.5.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	1343

9.3.5.2.1	FDD	1343

9.3.5.2.2	TDD	1346

9.3.6	Minimum requirement (With multiple CSI processes)	1349

9.3.6.1	FDD	1349

9.3.6.2	TDD	1354

9.3.7	Minimum requirement PUSCH 3-2	1358

9.3.7.1	FDD	1358

9.3.7.2	TDD	1359

9.3.8	Additional requirements for enhanced receiver Type B	1361

9.3.8.1	Minimum requirement PUCCH 1-1 (Cell-Specific Reference Symbols)	1361

9.3.8.1.1	FDD	1361

9.3.8.1.2	TDD	1362

9.3.8.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbols)	1364

9.3.8.2.1	FDD	1364

9.3.8.2.2	TDD	1366

9.3.8.3	Minimum requirement with CSI process	1369

9.3.8.3.1	FDD	1369

9.3.8.3.2	TDD	1372

9.4	Reporting of Precoding Matrix Indicator (PMI)	1375

9.4.1	Single PMI	1376

9.4.1.1	Minimum requirement PUSCH 3-1 (Cell-Specific Reference Symbols)	1376

9.4.1.1.1	FDD	1376

9.4.1.1.2	TDD	1377

9.4.1.2	 Minimum requirement PUCCH 2-1 (Cell-Specific Reference Symbols)	1378

9.4.1.2.1	FDD	1378

9.4.1.2.2	TDD	1380

9.4.1.3	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1381

9.4.1.3.1	FDD	1381

9.4.1.3.2	TDD	1384

9.4.1.3.3	FDD (with Class A 12Tx codebook)	1387

9.4.1.3.4	TDD (with Class A 12Tx codebook)	1389

9.4.1.3.5	FDD (with Class A 24Tx codebook)	1392

9.4.1.3.6	TDD (with Class A 24Tx codebook)	1394

9.4.1.4	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	1397

9.4.1.4.1	FDD (with 4Tx enhanced codebook)	1397

9.4.1.4.2	TDD (with 4Tx enhanced codebook)	1399

9.4.1.4.3	FDD (with Class B alternative codebook for one CSI-RS resource configured)	1402

9.4.1.4.4	TDD (with Class B alternative codebook for one CSI-RS resource configured)	1404

9.4.1a	Void	1407

9.4.1a.1	Void	1407

9.4.1a.1.1	Void	1407

9.4.1a.1.2	Void	1407

9.4.2	Multiple PMI	1407

9.4.2.1	Minimum requirement PUSCH 1-2 (Cell-Specific Reference Symbols)	1407

9.4.2.1.1	FDD	1407

9.4.2.1.2	TDD	1408

9.4.2.2	 Minimum requirement PUSCH 2-2 (Cell-Specific Reference Symbols)	1409

9.4.2.2.1	FDD	1409

9.4.2.2.2	TDD	1410

9.4.2.3	Minimum requirement PUSCH 1-2 (CSI Reference Symbol)	1411

9.4.2.3.1	FDD	1411

9.4.2.3.2	TDD	1413

9.4.2.3.3	FDD (with 4Tx enhanced codebook)	1416

9.4.2.3.4	TDD (with 4Tx enhanced codebook)	1418

9.4.2.3.5	FDD (with Class A 16Tx codebook)	1420

9.4.2.3.6	TDD (with Class A 16Tx codebook)	1423

9.4.2.3.7	FDD (with Class A 32Tx codebook)	1426

9.4.2.3.8	TDD (with Class A 32Tx codebook)	1429

9.4.2.3.9	FDD (with Class A 16Tx advanced codebook)	1432

9.4.2.3.10	TDD (with Class A 16Tx advanced codebook)	1435

9.4.3	Void	1438

9.5	Reporting of Rank Indicator (RI)	1438

9.5.1	Minimum requirement (Cell-Specific Reference Symbols)	1438

9.5.1.1	FDD	1438

9.5.1.2	TDD	1439

9.5.2	Minimum requirement (CSI Reference Symbols)	1440

9.5.2.1	FDD	1440

9.5.2.2	TDD	1442

9.5.3	Minimum requirement (CSI measurements in case two CSI subframe sets are configured)	1444

9.5.3.1	FDD	1444

9.5.3.2	TDD	1447

9.5.4	Minimum requirement (CSI measurements in case two CSI subframe sets are configured and CRS assistance information are configured)	1450

9.5.4.1	FDD	1450

9.5.4.2	TDD	1453

9.5.5	Minimum requirement (with CSI process)	1456

9.5.5.1	FDD	1457

9.5.5.2	TDD	1460

9.6	 Additional requirements for carrier aggregation	1463

9.6.1	 Periodic reporting on multiple cells (Cell-Specific Reference Symbols)	1463

9.6.1.1	FDD	1463

9.6.1.2	TDD	1469

9.6.1.3	TDD-FDD CA with FDD PCell	1475

9.6.1.4	TDD-FDD CA with TDD PCell	1480

9.7	CSI reporting (Single receiver antenna)	1486

9.7.1	CQI reporting definition under AWGN conditions	1487

9.7.1.1	FDD and half-duplex FDD	1487

9.7.1.2	TDD	1487

9.7.1.3 FDD (Category 1bis UE)	1488

9.7.1.4 TDD (Category 1bis UE)	1489

9.7.2	CQI reporting under fading conditions	1490

9.7.2.1	FDD and half-duplex FDD	1490

9.7.2.2	TDD	1491

9.7.2.3 FDD (Category 1bis UE)	1492

9.7.2.4 TDD (Category 1bis UE)	1493

9.8	CSI reporting (UE supporting coverage enhancement)	1494

9.8.1	CQI reporting definition under AWGN conditions	1494

9.8.1.1	FDD and half-duplex FDD	1494

9.8.1.2	TDD	1495

9.8.2	UE-selected subband CQI	1496

9.8.2.1	FDD and half-duplex FDD	1496

9.8.2.2	TDD	1500

9.8.3	CQI reporting definition for UE supporting 64QAM under AWGN	1503

9.8.3.1	FDD and half-duplex FDD	1503

9.8.3.2	TDD	1504

9.8.4	CQI reporting definition for UE supporting alternative table under AWGN	1505

9.8.4.1	FDD and half-duplex FDD	1505

9.8.4.2	TDD	1506

9.9	CSI reporting for 4Rx UE	1507

9.9.1	CQI reporting definition under AWGN conditions	1507

9.9.1.1	Minimum requirement PUCCH 1-0 with Rank 1 (Cell-Specific Reference Symbols)	1508

9.9.1.1.1	FDD	1508

9.9.1.1.2	TDD	1508

9.9.1.2	Minimum requirement PUCCH 1-1 with Rank 2 (CSI Reference Symbols)	1509

9.9.1.2.1	FDD	1509

9.9.1.2.2	TDD	1510

9.9.1.3	 Minimum requirement PUCCH 1-1 with Rank 4 (Cell-Specific Reference Symbols)	1511

9.9.1.3.1	FDD	1512

9.9.1.3.2	TDD	1512

9.9.1.4	Minimum requirement PUCCH 1-1 with Rank 3 (CSI Reference Symbols)	1513

9.9.1.4.1	FDD	1513

9.9.1.4.2	TDD	1514

9.9.2	CQI reporting definition under fading conditions	1515

9.9.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol) for enhanced receiver Type A	1515

9.9.2.1.1	FDD	1516

9.9.2.1.2	TDD	1517

9.9.2.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol) for enhanced receiver Type A	1519

9.9.2.2.1	FDD	1519

9.9.2.2.2	TDD	1522

9.9.3	Reporting of Precoding Matrix Indicator (PMI) for 4Rx UE	1525

9.9.3.1	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1526

9.9.3.1.1	TDD	1526

9.9.4	 Reporting of Rank Indicator (RI)	1528

9.9.4.1	Minimum requirement (Cell-Specific Reference Symbols)	1528

9.9.4.1.1	FDD	1528

9.9.4.1.2	TDD	1530

9.9.4.2	Minimum requirement (CSI Reference Symbols)	1531

9.9.4.2.1	FDD	1531

9.9.4.2.2	TDD	1533

9.10	Reporting of CSI-RS Resource Indicator (CRI)	1535

9.10.1	Minimum requirement (PUSCH 3-1)	1536

9.10.1.1	FDD	1536

9.10.1.2	TDD	1538

9.10.2	Minimum requirement (PUSCH 3-1, QCL Type C)	1540

9.10.2.1	FDD	1540

9.10.2.2	TDD	1543

9.11	Reporting of Hybrid Channel state information	1546

9.11.1	Minimum requirement (with eMIMO-Type configured as Class B with more than one CSI-RS resource configured and eMIMO-Type2 as Class B with one CSI-RS resource configured)	1546

9.11.1.1	FDD	1547

9.11.1.2	TDD	1549

9.12	CSI reporting (UE supporting Short TTI)	1551

9.12.1	CQI reporting under fading conditions (Cell-Specific Reference Symbol)	1551

9.12.1.1	FDD	1551

9.12.1.2	TDD	1554

9.12.2	CQI reporting under fading conditions (CSI Reference Symbol)	1555

9.12.2.1	FDD	1555

9.12.2.2	TDD	1558

9.13	CSI reporting for 8Rx UE	1560

9.13.1	CQI reporting definition under AWGN conditions	1560

9.13.1.1	Minimum requirement PUCCH 1-1 with Rank 4 (CSI Reference Symbols)	1560

9.13.1.2.1	Void	1560

9.13.1.2.2	TDD	1560

10	Performance requirement (MBMS)	1561

10.1	FDD (Fixed Reference Channel)	1561

10.1.1	Minimum requirement	1562

10.2	TDD (Fixed Reference Channel)	1563

10.2.1	Minimum requirement	1563

10.3	FDD (Fixed Reference Channel) with FeMBMS	1564

10.3.1	Minimum requirement for FeMBMS Unicast-mixed Cell under CA	1564

10.3.1.1	Minimum requirement with 1.25kHz subcarrier spacing	1564

10.3.1.2	Minimum requirement with 7.5kHz subcarrier spacing	1565

10.3.2	Minimum requirement for FeMBMS Unicast-mixed Cell as Non-Serving Cell	1566

10.3.2.1	Minimum requirement with 1.25kHz subcarrier spacing	1566

10.3.2.2	Minimum requirement with 7.5kHz subcarrier spacing	1567

10.3.3	Minimum requirement for MBMS Dedicated cell	1568

10.3.3.1	Minimum requirement with 1.25kHz subcarrier spacing	1568

10.3.3.2	Minimum requirement with 7.5kHz subcarrier spacing	1569

10.3.3.3	Minimum requirement with 15kHz subcarrier spacing	1570

11	Performance requirement (ProSe Direct Discovery)	1571

11.1	General	1571

11.1.1	Applicability of requirements	1571

11.1.2	Reference DRX configuration	1572

11.2	Demodulation of PSDCH (single link performance)	1572

11.2.1	FDD (in-coverage)	1572

11.2.2	TDD (in-coverage)	1573

11.2.3	FDD (out-of-coverage)	1574

11.3	Power imbalance performance with two links	1574

11.3.1	FDD	1574

11.3.2	TDD	1575

11.4	Multiple timing reference test	1576

11.4.1	FDD	1577

11.5	Maximum Sidelink processes test	1578

11.5.1	FDD	1578

11.5.2	TDD	1579

12	Performance requirement (ProSe Direct Communication)	1581

12.1	General	1581

12.1.1	Applicability of requirements	1581

12.1.1.1	Applicability of requirements for different channel bandwidths	1581

12.1.1.2	Test coverage for different number of component carriers	1581

12.1.1.3	Applicability and test rules for different CA configurations and bandwidth combination sets	1581

12.1.2	Reference DRX configuration	1582

12.2	Demodulation of PSSCH	1582

12.2.1	FDD	1582

12.3	Demodulation of PSCCH	1583

12.3.1	FDD	1584

12.4	Demodulation of PSBCH	1585

12.4.1	FDD	1585

12.5	Power imbalance performance with two links	1585

12.5.1	 FDD	1585

12.6	Multiple timing reference test	1587

12.6.1	FDD	1587

12.7	Maximum Sidelink processes test	1590

12.7.1	FDD	1590

12.8	Sustained downlink data rate with active Sidelink	1591

13	Void	1593

14	Performance requirement (V2X Sidelink Communication)	1593

14.1	General	1593

14.1.1	Applicability of requirements	1593

14.2	Demodulation of PSSCH	1594

14.3	Demodulation of PSCCH	1595

14.4	Power imbalance performance with two links	1595

14.5	Demodulation of PSBCH	1596

14.6	Demodulation of PSSCH with eNB based synchronization	1597

14.7	Soft buffer test	1597

14.8	PSCCH/PSSCH decoding capability test	1598

14.9	Sustained downlink data rate with active sidelink	1599

14.10	Soft buffer test (CA)	1600

14.11	PSCCH/PSSCH decoding capability test (CA)	1601

Annex A (normative):  Measurement channels	1602

A.1	General	1602

A.2	UL reference measurement channels	1602

A.2.1	General	1602

A.2.1.1	Applicability and common parameters	1602

A.2.1.2	Determination of payload size	1602

A.2.1.3	Overview of UL reference measurement channels	1603

A.2.2	Reference measurement channels for FDD	1626

A.2.2.1	Full RB allocation	1626

A.2.2.1.1	QPSK	1626

A.2.2.1.2	16-QAM	1628

A.2.2.1.3	64-QAM	1630

A.2.2.1.4	256 QAM	1631

A.2.2.2	Partial RB allocation	1631

A.2.2.2.1	QPSK	1632

A.2.2.2.2	16-QAM	1635

A.2.2.2.3	64-QAM	1637

A.2.2.2.4	256 QAM	1639

A.2.2.3	Void	1639

A.2.3	Reference measurement channels for TDD	1640

A.2.3.1	Full RB allocation	1640

A.2.3.1.1	QPSK	1640

A.2.3.1.2	16-QAM	1642

A.2.3.1.3	64-QAM	1645

A.2.3.1.4	256 QAM	1645

A.2.3.2	Partial RB allocation	1645

A.2.3.2.1	QPSK	1646

A.2.3.2.2	16-QAM	1650

A.2.3.2.3	64-QAM	1655

A.2.3.2.4	256 QAM	1657

A.2.3.3	Void	1657

A.2.4	Reference measurement channels for UE category NB1	1658

A.2.5	Reference measurement channels for LAA	1659

A.2.5.1	Full RB allocation	1659

A.2.5.1.1	QPSK	1659

A.2.5.1.2	16QAM	1659

A.2.5.1.3	64QAM	1660

A.2.5.2	Partial RB allocation	1660

A.2.5.2.1	QPSK	1660

A.2.5.2.2	16QAM	1661

A.2.5.2.3	64QAM	1661

A.3	DL reference measurement channels	1662

A.3.1	General	1662

A.3.1.1	Overview of DL reference measurement channels	1662

A.3.2	Reference measurement channel for receiver characteristics	1687

A.3.3	Reference measurement channels for PDSCH performance requirements (FDD)	1717

A.3.3.1	Single-antenna transmission (Common Reference Symbols)	1717

A.3.3.2	Multi-antenna transmission (Common Reference Symbols)	1722

A.3.3.2.1	Two antenna ports	1722

A.3.3.2.2	Four antenna ports	1730

A.3.3.3	Reference Measurement Channel for UE-Specific Reference Symbols	1735

A.3.3.3.0	Two antenna ports (no CSI-RS)	1735

A.3.3.3.1	Two antenna port (CSI-RS)	1736

A.3.3.3.2	Four antenna ports (CSI-RS)	1739

A.3.3.3.2A	Eight antenna ports (CSI-RS)	1746

A.3.3.3.3	Twelve antenna port (CSI-RS)	1747

A.3.3.3.4	Sixteen antenna port (CSI-RS)	1748

A.3.3.3.5	Twenty-four antenna port (CSI-RS)	1749

A.3.3.3.6	Thirty-two antenna port (CSI-RS)	1750

A.3.4	Reference measurement channels for PDSCH performance requirements (TDD)	1752

A.3.4.1	Single-antenna transmission (Common Reference Symbols)	1752

A.3.4.2	Multi-antenna transmission (Common Reference Signals)	1760

A.3.4.2.1	Two antenna ports	1760

A.3.4.2.2	Four antenna ports	1773

A.3.4.3	Reference Measurement Channels for UE-Specific Reference Symbols	1780

A.3.4.3.1	Single antenna port (Cell Specific)	1780

A.3.4.3.2	Two antenna ports (Cell Specific)	1781

A.3.4.3.3	Two antenna ports (CSI-RS)	1783

A.3.4.3.4	Four antenna ports (CSI-RS)	1791

A.3.4.3.5	Eight antenna ports (CSI-RS)	1798

A.3.4.3.6	Twelve antenna ports (CSI-RS)	1803

A.3.4.3.7	Sixteen antenna ports (CSI-RS)	1804

A.3.4.3.8	Twenty-four antenna ports (CSI-RS)	1805

A.3.4.3.9	Thirty-two antenna ports (CSI-RS)	1806

A.3.5	Reference measurement channels for PDCCH/PCFICH performance requirements	1808

A.3.5.1	FDD	1808

A.3.5.2	TDD	1808

A.3.5.3	LAA	1809

A.3.6	Reference measurement channels for PHICH performance requirements	1809

A.3.7	Reference measurement channels for PBCH performance requirements	1809

A.3.8	Reference measurement channels for MBMS performance requirements	1810

A.3.8.1	FDD	1810

A.3.8.2	TDD	1814

A.3.9	Reference measurement channels for sustained downlink data rate provided by lower layers	1816

A.3.9.1	FDD	1816

A.3.9.2	TDD	1820

A.3.9.3	FDD (EPDCCH scheduling)	1834

A.3.9.4	TDD (EPDCCH scheduling)	1835

A.3.9.5	LAA	1837

A.3.10	Reference Measurement Channels for EPDCCH performance requirements	1839

A.3.10.1	FDD	1839

A.3.10.2	TDD	1839

A.3.11	Reference Measurement Channels for MPDCCH performance requirements	1839

A.3.11.1	FDD and half-duplex FDD	1839

A.3.11.2	TDD	1840

A.3.12	Reference measurement channels for NPDSCH performance requirements	1841

A.3.12.1	In-band	1841

A.3.12.1.2	Two-antenna transmission	1841

A.3.12.2	Standalone/Guard-band	1843

A.3.12.2.1 Single-antenna transmission	1843

A.3.13	Reference measurement channels for NPDCCH performance requirements	1846

A.3.13.1	Half-duplex FDD	1846

A.3.13.2	TDD	1847

A.3.14	Reference measurement channels for NPBCH performance requirements for Cat NB1 UEs	1847

A.3.15	Reference Measurement Channels for LAA SCell with frame structure Type-3	1848

A.3.15.1	Multi-antenna transmission (Common Reference Symbols)	1848

A.3.15.1.1	Four antenna ports	1848

A.3.15.2	Reference Measurement Channel for UE-Specific Reference Symbols	1848

A.3.15.2.1	Two antenna ports (CSI-RS)	1848

A.3.16	Reference measurement channels for Slot-PDSCH and Subslot-PDSCH performance requirements	1850

A.3.16.1	FDD	1850

A.3.16.2	TDD	1854

A.3.17	Reference measurement channels for SPDCCH performance requirements	1855

A.3.17.1	FDD	1855

A.3.17.2	TDD	1856

A.4	CSI reference measurement channels	1856

A.5	OFDMA Channel Noise Generator (OCNG)	1871

A.5.1	OCNG Patterns for FDD	1871

A.5.1.1	OCNG FDD pattern 1: One sided dynamic OCNG FDD pattern	1871

A.5.1.2	OCNG FDD pattern 2: Two sided dynamic OCNG FDD pattern	1872

A.5.1.3	OCNG FDD pattern 3: 49 RB OCNG allocation with MBSFN in 10 MHz	1872

A.5.1.3A	OCNG FDD pattern 3A: 49 RB OCNG allocation with MBSFN enhancement in 10 MHz	1873

A.5.1.4	OCNG FDD pattern 4: One sided dynamic OCNG FDD pattern for MBMS transmission	1873

A.5.1.4A	OCNG FDD pattern 4A: One sided dynamic OCNG FDD pattern for enhanced MBMS transmission	1874

A.5.1.5	OCNG FDD pattern 5: One sided dynamic 16QAM modulated OCNG FDD pattern	1875

A.5.1.6	OCNG FDD pattern 6: dynamic OCNG FDD pattern when user data is in 2 non-contiguous blocks	1876

A.5.1.8	OCNG FDD pattern 8: Dynamic OCNG FDD pattern for TM10 transmission	1877

A.5.2	OCNG Patterns for TDD	1878

A.5.2.1	OCNG TDD pattern 1: One sided dynamic OCNG TDD pattern	1878

A.5.2.2	OCNG TDD pattern 2: Two sided dynamic OCNG TDD pattern	1879

A.5.2.3	OCNG TDD pattern 3: 49 RB OCNG allocation with MBSFN in 10 MHz	1880

A.5.2.4	OCNG TDD pattern 4: One sided dynamic OCNG TDD pattern for MBMS transmission	1880

A.5.2.5	OCNG TDD pattern 5: One sided dynamic 16QAM modulated OCNG TDD pattern	1881

A.5.2.6	OCNG TDD pattern 6: dynamic OCNG TDD pattern when user data is in 2 non-contiguous blocks	1882

A.5.2.8	OCNG TDD pattern 8: Dynamic OCNG TDD pattern for TM10 transmission	1883

A.5.3	OCNG Patterns for Narrowband IoT	1884

A.5.3.1	Narrowband IoT OCNG pattern 1	1884

A.5.4	OCNG Patterns for frame structure type 3	1885

A.5.4.1	OCNG FS3 pattern 1: One sided dynamic OCNG frame structure type 3 pattern	1885

A.5.4.2	OCNG FS3 pattern 2: Two sided dynamic OCNG frame structure 3 pattern	1886

A.6	Sidelink reference measurement channels	1887

A.6.1	General	1887

A.6.1.1	Overview of ProSe reference measurement channels	1887

A.6.2	Reference measurement channel for receiver characteristics	1888

A.6.3	Reference measurement channels for PSDCH performance requirements	1890

A.6.4	Reference measurement channels for PSCCH performance requirements	1891

A.6.5	Reference measurement channels for PSSCH performance requirements	1891

A.6.6	Reference measurement channels for PSBCH performance requirements	1892

A.7	Sidelink reference resource pool configurations	1893

A.7.1	Reference resource pool configurations for ProSe Direct Discovery demodulation tests	1893

A.7.1.1	FDD	1893

A.7.1.2	TDD	1896

A.7.2	Reference resource pool configurations for ProSe Direct Communication demodulation tests	1898

A.7.2.1	FDD	1898

A.8	V2X reference measurement channels	1903

A.8.1	General	1903

A.8.1.1	Overview of V2X reference measurement channels	1904

A.8.2	Reference measurement channel for receiver characteristics	1904

A.8.3	Reference measurement channel for transmitter characteristics	1906

A.8.4	Reference measurement for PSCCH performance requirements	1909

A.8.5	Reference measurement for PSSCH performance requirements	1910

A.8.6	Reference measurement for PSBCH performance requirements	1910

A.9	V2X reference resource pool configurations	1911

Annex B (normative):  Propagation conditions	1914

B.1	Static propagation condition	1914

B.1.1	UE Receiver with 2Rx	1914

B.1.2	UE Receiver with 4Rx	1914

B.1.3	UE Receiver with 8Rx	1915

B.2	Multi-path fading propagation conditions	1916

B.2.1	Delay profiles	1916

B.2.2	Combinations of channel model parameters	1917

B.2.3	MIMO Channel Correlation Matrices	1918

B.2.3.1	Definition of MIMO Correlation Matrices	1918

B.2.3.2	MIMO Correlation Matrices at High, Medium and Low Level	1924

B.2.3A	MIMO Channel Correlation Matrices using cross polarized antennas	1927

B.2.3A.1	Definition of MIMO Correlation Matrices using cross polarized antennas	1928

B.2.3A.2	Spatial Correlation Matrices using cross polarized antennas at eNB and UE sides	1928

B.2.3A.2.1	Spatial Correlation Matrices at eNB side	1928

B.2.3A.2.2	Spatial Correlation Matrices at UE side	1929

B.2.3A.4	Beam steering approach	1932

B.2.3B	MIMO Channel Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1932

B.2.3B.1	Definition of MIMO Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1933

B.2.3B.2	Spatial Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1934

B.2.3B.2.1	Spatial Correlation Matrices at eNB side	1934

B.2.3B.2.2	Spatial Correlation Matrices at UE side	1934

B.2.3B.3	MIMO Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1934

B.2.3B.4	Beam steering approach	1937

B.2.3B.4A	Beam steering approach with dual cluster beams	1938

B.2.4	Propagation conditions for CQI tests	1939

B.2.4.1	Propagation conditions for CQI tests with multiple CSI processes	1939

B.2.5	Void	1939

B.2.6	MBSFN Propagation Channel Profile	1939

B.2.6.1	Subcarrier spacing 15kHz or 7.5kHz	1939

B.2.6.2	Subcarrier spacing 1.25kHz	1940

B.3	High speed train scenario	1941

B.3A	HST-SFN scenario	1942

B.3B	HST-SFN scenario for 500 km/h speed	1946

B.3C	HST scenario for 500 km/h speed	1946

B.4	Beamforming Model	1946

B.4.1	Single-layer random beamforming (Antenna port 5, 7, or 8)	1946

B.4.1A	Single-layer random beamforming (Antenna port 7, 8, 11 or 13 with enhanced DMRS table configured)	1947

B.4.2	Dual-layer random beamforming (antenna ports 7 and 8)	1947

B.4.3	Generic beamforming model (antenna ports 7-14)	1948

B.4.4	Random beamforming for EPDCCH distributed transmission (Antenna port 107 and 109)	1948

B.4.5	Random beamforming for EPDCCH localized transmission (Antenna port 107, 108, 109 or 110)	1949

B.4.6	Beamforming model for CRI test	1949

B.5	Interference models for enhanced performance requirements Type-A	1950

B.5.1	Dominant interferer proportion	1951

B.5.2	Transmission mode 3 interference model	1951

B.5.3	Transmission mode 4 interference model	1951

B.5.4	Transmission mode 9 interference model	1952

B.6	Interference models for enhanced performance requirements Type-B	1952

B.6.1	Transmission mode 2 interference model	1952

B.6.2	Transmission mode 3 interference model	1952

B.6.3	Transmission mode 4 interference model	1953

B.6.4	Transmission mode 9 interference model	1953

B.6.5	CRS interference model	1954

B.6.6	Random interference model	1954

B.7	Interference models for enhanced downlink control channel performance requirements Type A and B	1955

B.7.1	PDCCH, PCFICH and PHICH interference model	1955

B.8	Burst transmission models for Frame structure type 3	1956

B.8.1	Burst transmission model for one LAA SCell	1956

B.8.2	Burst transmission model for multiple LAA SCell(s)	1956

Annex C (normative):  Downlink Physical Channels	1958

C.1	General	1958

C.2	Set-up	1958

C.3	Connection	1958

C.3.1	Measurement of Receiver Characteristics	1958

C.3.2	Measurement of Performance requirements	1959

C.3.3	Aggressor cell power allocation for Measurement of Performance Requirements when ABS is Configured	1960

C.3.4	Power Allocation for Measurement of Performance Requirements when Quasi Co-location Type B: same Cell ID	1961

C.3.5	Simplified CA testing method	1961

C.3.6	Measurement of Receiver Characteristics for Narrowband IoT	1962

Annex D (normative):  Characteristics of the interfering signal	1963

D.1	General	1963

D.2	Interference signals	1963

Annex E (normative):  Environmental conditions	1964

E.1	 General	1964

E.2	 Environmental	1964

E.2.1	Temperature	1964

E.2.2	Voltage	1964

E.2.3	Vibration	1965

Annex F (normative):  Transmit modulation	1966

F.1	Measurement Point	1966

F.2	Basic Error Vector Magnitude measurement	1966

F.3	Basic in-band emissions measurement	1967

F.4	Modified signal under test	1967

F.5	Window length	1969

F.5.1	Timing offset	1969

F.5.2	Window length	1969

F.5.3	Window length for normal CP	1969

F.5.4	Window length for Extended CP	1970

F.5.5	Window length for PRACH	1970

F.5.F	Window length for category NB1	1971

F.6	Averaged EVM	1971

F.6.F	Averaged EVM for category NB1	1972

F.7	Spectrum Flatness	1972

Annex G (informative):  Reference sensitivity level in lower SNR	1973

G.1	General	1973

G.2	Typical receiver sensitivity performance (QPSK)	1973

G.3	Reference measurement channel for REFSENSE in lower SNR	1979

Annex H (normative):  Modified MPR behavior	1982

H.1	Indication of modified MPR behavior	1982

Annex I (normative):  Supported Post Antenna Gain	1983

I.1	Declared Supported Post Antenna Gain for UE	1983

Annex J (informative):  Change history	1984