3GPP TS 36.133 V15.7.0 (2019-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);
Requirements for support of radio resource management

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	87

1	Scope	88

2	References	88

3	Definitions, symbols and abbreviations	90

3.1	Definitions	90

3.2	Symbols	92

3.3	Abbreviations	93

3.4	Test tolerances	95

3.5	Additional notation	96

3.5.1	Groups of bands	96

3.6	General	98

3.6.1	Applicability of requirements in this specification version	98

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	104

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	106

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	107

3.6.2	Applicability of requirements for EN-DC operation	108

3.6.3	Applicability of requirements for NE-DC operation	109

3.6.4	Applicability of requirements for NGEN-DC operation	110

4	E-UTRAN RRC_IDLE state mobility	110

4.1	Cell Selection	110

4.2	Cell Re-selection	110

4.2.1	Introduction	110

4.2.2	Requirements	110

4.2.2.1	Measurement and evaluation of serving cell	111

4.2.2.2	Void	112

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	112

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	114

4.2.2.5	Measurements of inter-RAT cells	116

4.2.2.5.1	Measurements of UTRAN FDD cells	116

4.2.2.5.2	Measurements of UTRAN TDD cells	118

4.2.2.5.3	Measurements of GSM cells	119

4.2.2.5.4	Measurements of HRPD cells	120

4.2.2.5.5	Measurements of cdma2000 1X	121

4.2.2.5.6	Measurements of NR cells	122

4.2.2.6	Evaluation of cell re-selection criteria	123

4.2.2.7	Maximum interruption in paging reception	123

4.2.2.8	void	124

4.2.2.9	UE measurement capability	124

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	125

4.2.2.10	Reselection to CSG cells	125

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	125

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	126

4.2.2.11	Void	127

4.2.2.12	Void	127

4.2.2.13	Void	127

4.3	Minimization of Drive Tests (MDT)	127

4.3.1	Introduction	128

4.3.2	Measurements	128

4.3.2.1	Requirements	128

4.3.3	Relative Time Stamp Accuracy	128

4.3.3.1	Requirements	128

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	128

4.3.4.1	 Requirements	128

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	129

4.3.5.1	Requirements for timeSinceFailure	129

4.4	MBSFN Measurements	129

4.4.1	Introduction	129

4.4.2	MBSFN RSRP measurements	129

4.4.3	MBSFN RSRQ measurements	129

4.4.4	MCH BLER measurements	130

4.5	Proximity-based Services	130

4.5.1	Introduction	130

4.5.2	Requirements	130

4.5.2.1	Interruptions with ProSe Direct Discovery	130

4.5.2.2	Interruptions with ProSe Direct Communication	130

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	130

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	131

4.6	Cell Selection and Re-selection Requirements for UE category NB1	131

4.6.1	Cell Selection	131

4.6.2	Cell Re-selection	131

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	132

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	132

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	133

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	134

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	135

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	136

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	138

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	139

4.6.2.7	Maximum interruption in paging reception in normal coverage	140

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	140

4.6.2.8	UE measurement capability	140

4.6.2.9	WUS receptions for NB1	141

4.7	Cell Selection and Re-selection Requirements for UE category M1	141

4.7.1	Cell Selection	141

4.7.2	Cell Re-selection	141

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	141

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	141

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	142

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	143

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	145

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	145

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	145

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	145

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	146

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	148

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	149

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in extended coverage	150

4.7.2.3	WUS receptions for UE category M1	150

4.8	Idle State Positioning Measurement Requirements for UE category NB1	150

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	150

4.8.1.1	RSTD Measurement Reporting Delay	152

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	152

4.8.2.1	RSTD Measurement Reporting Delay	154

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	154

4.8.3.1	RSTD Measurement Reporting Delay	155

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	156

4.8.4.1	RSTD Measurement Reporting Delay	157

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	158

4.8.5.1	Measurement Reporting Delay	159

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	159

4.8.6.1	Measurement Reporting Delay	160

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	160

4.8.7.1	Measurement Reporting Delay	161

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	162

4.8.8.1	Measurement Reporting Delay	163

4.9	Idle Mode CA Measurement	164

4.9.1	Introduction	164

4.9.2	Requirement	164

4.9.2.1	Detected cell requirement during state transition and Idle mode	164

4.9.2.2	Measurements of inter-frequency CA candidate cells	164

4.9.2.3	Measurements on serving cell	165

4A	E-UTRAN RRC_INACTIVE state mobility	165

4A.1	Cell Re-selection	165

4A.1.1	Introduction	165

4A.1.2	Requirements	165

4A.1.2.1	UE measurement capability	165

4A.1.2.2	Measurement and evaluation of serving cell	165

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	165

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	165

4A.1.2.5	Evaluation of cell re-selection criteria	166

4A.1.2.6	Maximum interruption in paging reception	166

4A.1.2.7	Measurements of inter-RAT NR cells	166

5	E-UTRAN RRC_CONNECTED state mobility	166

5.1	E-UTRAN Handover	166

5.1.1	Introduction	166

5.1.2	Requirements	166

5.1.2.1	E-UTRAN FDD – FDD	166

5.1.2.1.1	Handover delay	166

5.1.2.1.2	Interruption time	167

5.2.2.2	E-UTRAN FDD – TDD	168

5.2.2.2.1	(Void)	168

5.2.2.2.2	(Void)	168

5.2.2.3	E-UTRAN TDD – FDD	168

5.2.2.3.1	(Void)	168

5.2.2.3.2	(Void)	168

5.2.2.4	E-UTRAN TDD – TDD	168

5.2.2.4.1	Handover delay	168

5.2.2.4.2	Interruption time	169

5.2.2.5	E-UTRAN HD–FDD	170

5.2.2.5.1	Handover delay	170

5.2.2.5.2	Interruption time	171

5.3	Handover to other RATs	172

5.3.1	E-UTRAN - UTRAN FDD Handover	172

5.3.1.1	Introduction	172

5.3.1.1.1	Handover delay	172

5.3.1.1.2	Interruption time	172

5.3.2	E-UTRAN - UTRAN TDD Handover	173

5.3.2.1	Introduction	173

5.3.2.2	Requirements	173

5.3.2.2.1	Handover delay	173

5.3.2.2.2	Interruption time	173

5.3.3	E-UTRAN - GSM Handover	174

5.3.3.1	Introduction	174

5.3.3.2	Requirements	174

5.3.3.2.1	Handover delay	174

5.3.3.2.2	Interruption time	174

5.3.4	E-UTRAN - NR FR1 Handover	175

5.3.4.1	Introduction	175

5.3.4.2	Handover delay	175

5.3.4.3	Interruption time	175

5.3.5	E-UTRAN - NR FR2 Handover	175

5.3.5.1	Introduction	175

5.3.5.2	Handover delay	176

5.3.5.3	Interruption time	176

5.4	Handover to Non-3GPP RATs	176

5.4.1	E-UTRAN – HRPD Handover	176

5.4.1.1	Introduction	176

5.4.1.1.1	Handover delay	176

5.4.1.1.2	Interruption time	177

5.4.2	E-UTRAN – cdma2000 1X Handover	177

5.4.2.1	Introduction	177

5.4.2.1.1	Handover delay	177

5.4.2.1.2	Interruption time	177

5.5	E-UTRAN Handover for Cat-M1 UEs	178

5.5.1	Introduction	178

5.5.2	Requirements in CEModeA	178

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	178

5.5.2.1.1	Handover delay	178

5.5.2.1.2	Interruption time	178

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	179

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	179

5.5.2.3.1	Void	179

5.5.2.3.2	Void	179

5.5.3	Requirements in CEModeB	179

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	179

5.5.3.1.1	Handover delay	179

5.5.3.1.2	Interruption time	179

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	180

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	180

5.6	Void	180

6	RRC Connection Mobility Control	180

6.1	RRC Re-establishment	180

6.1.1	Introduction	180

6.1.2	Requirements	180

6.1.2.1	UE Re-establishment delay requirement	181

6.2	Random Access	181

6.2.1	Introduction	181

6.2.2	Requirements	181

6.2.2.1	Contention based random access	181

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	181

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	182

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	182

6.2.2.1.4	Void	182

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	182

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	182

6.2.2.2	Non-Contention based random access	182

6.2.2.2.1	Correct behaviour when receiving Random Access Response	182

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	182

6.2.3	Requirements for Cat-M1 UEs	182

6.3	RRC Connection Release with Redirection	183

6.3.1	Introduction	183

6.3.2	Requirements	183

6.3.2.1	RRC connection release with redirection to UTRAN FDD	183

6.3.2.2	RRC connection release with redirection to GERAN	183

6.3.2.3	RRC connection release with redirection to UTRAN TDD	184

6.3.2.4	RRC connection release with redirection to NR	184

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	185

6.4.1	Introduction	185

6.4.2	Requirements	185

6.5	RRC Re-establishment for NB-IoT UEs	185

6.5.1	Introduction	185

6.5.2	Requirements	185

6.5.2.1	UE Re-establishment delay requirement in normal coverage	186

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	186

6.6	Random Access for UE category NB1	186

6.6.1	Introduction	186

6.6.2	Requirements	187

6.6.2.1	Correct behaviour when receiving Random Access Response reception	187

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	187

6.6.2.3	Correct behaviour when receiving a NACK on msg3	187

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	187

6.6.2.5	Correct behaviour when contention Resolution timer expires	187

6.6.2.6	MSG3-based channel quality report for UE Category NB1	187

6.6.3	Requirements for NPRACH configuration	188

6.7	RRC Re-establishment for Cat-M1 UEs	188

6.7.1	Introduction	188

6.7.2	Requirements	188

6.7.2.1	UE Re-establishment delay requirement for CEModeA	189

6.7.2.2	UE Re-establishment delay requirement for CEModeB	189

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	190

6.8.1	Introduction	190

6.8.2	Requirements	190

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	190

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	190

6.9.1	Introduction	190

6.9.2	Requirements	191

7	Timing and signalling characteristics	191

7.1	UE transmit timing	191

7.1.1	Introduction	191

7.1.2	Requirements	192

7.2	UE timer accuracy	193

7.2.1	Introduction	193

7.2.2	Requirements	193

7.3	Timing Advance	193

7.3.1	Introduction	193

7.3.2	Requirements	193

7.3.2.1	Timing Advance adjustment delay	193

7.3.2.2	Timing Advance adjustment accuracy	194

7.4	Cell phase synchronization accuracy (TDD)	194

7.4.1	Definition	194

7.4.2	Minimum requirements	194

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	195

7.5.1	Introduction	195

7.5.2	eNodeB Synchronization Requirements	195

7.5.2.1	Synchronized E-UTRAN	195

7.5.2.2	Non-Synchronized E-UTRAN	195

7.6	Radio Link Monitoring	195

7.6.1	Introduction	195

7.6.2	Requirements	197

7.6.2.1	Minimum requirement when no DRX is used	197

7.6.2.2	Minimum requirement when DRX is used	198

7.6.2.3	Minimum requirement at transitions	199

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	199

7.6.2.5	Minimum requirement under IDC Interference	200

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	200

7.7.1	Introduction	200

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	200

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	201

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	202

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	204

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	205

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	205

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	206

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	206

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	206

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	208

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	208

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	209

7.7.14	SCell Activation Delay Requirement for Dormant SCell	210

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	211

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	212

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	213

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	213

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	215

7.8	Interruptions with Carrier Aggregation	217

7.8.1	Introduction	217

7.8.2	Requirements	218

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	218

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	218

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	218

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	218

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	218

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	218

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	219

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	219

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	219

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	220

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	220

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	221

7.8.2.13	Interruptions at SRS carrier based switching	221

7.8.2.14	Interruptions at SCell activation of dormant SCell for intra-band CA	222

7.8.2.15	Interruptions at SCell activation of dormant SCell for inter-band CA	222

7.8.2.16	Interruptions at SCell activation of multiple dormant SCells	222

7.8.2.17	Interruptions during CQI measurement on dormant SCell	223

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	223

7.9.1	Introduction	223

7.9.2	Minimum Requirements for Interband Carrier Aggregation	223

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	223

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	224

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	224

7.10.1	Introduction	224

7.10.2	Requirements	224

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	224

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	224

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	225

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	225

7.11	Radio Link Monitoring for UE Category 0	225

7.11.1	Introduction	225

7.11.2	Requirements for FD-FDD and TDD	227

7.11.2.1	Minimum requirement when no DRX is used	227

7.11.2.2	Minimum requirement when DRX is used	227

7.11.2.3	Minimum requirement at transitions	228

7.11.3	Requirements for HD-FDD	228

7.11.3.1	Minimum requirement when no DRX is used	228

7.11.3.2	Minimum requirement when DRX is used	228

7.11.3.3	Minimum requirement at transitions	229

7.12	Interruptions with Dual Connectivity	229

7.12.1	Introduction	229

7.12.2	Requirements	229

7.12.2.1	Interruptions at PSCell addition/release	229

7.12.2.2	Interruptions at transitions between active and non-active during DRX	230

7.12.2.3	Interruptions at transitions from non-DRX to DRX	230

7.12.2.4	Interruptions at SCell addition/release	230

7.12.2.5	Interruptions at SCell activation/deactivation	230

7.12.2.6	Interruptions during measurements on SCC	231

7.12.2.7	Interruptions at SRS carrier based switching	231

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	232

7.13.1	Definition	232

7.13.2	Minimum requirements	232

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	232

7.14.1	Introduction	232

7.14.2	PSCell Addition Delay Requirement	232

7.14.3	PSCell Release Delay Requirement	233

7.15	Maximum Receive Timing Difference in Dual Connectivity	233

7.15.1	Introduction	233

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	233

7.16	Proximity-based Services	234

7.16.1	Introduction	234

7.16.2	Requirements	234

7.16.2.1	ProSe UE transmission timing	234

7.16.2.1.1	Serving cell or PCell as timing reference	234

7.16.2.1.2	SCell or non-serving cell as timing reference	234

7.16.3	Interruptions with ProSe	234

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	235

7.16.3.2	Interruptions at ProSe Direct Communication configuration	235

7.16.3.3	Interruptions during ProSe Direct Discovery	235

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	235

7.16.3.5	Interruptions during ProSe Direct Communication	236

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	236

7.16.4.1	Measurement and evaluation of selected cell	236

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	236

7.16.5	Selection / Reselection of ProSe relay UE	237

7.16.6	ProSe operation under deactivated SCell	237

7.17	Maximum Transmission Timing Difference in Dual Connectivity	238

7.17.1	Introduction	238

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	238

7.18.1	Introduction	238

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	238

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	238

7.19	Radio Link Monitoring for UE Category M1	238

7.19.1	Introduction	238

7.19.2	Requirements for FD-FDD and TDD CE mode A	239

7.19.2.1	Minimum requirement when no DRX is used	240

7.19.2.2	Minimum requirement when DRX is used	241

7.19.2.3	Minimum requirement at transitions	242

7.19.3	Requirements for HD-FDD with CE mode A	242

7.19.3.1	Minimum requirement when no DRX is used	242

7.19.3.2	Minimum requirement when DRX is used	242

7.19.3.3	Minimum requirement at transitions	244

7.19.4	Requirements for FD-FDD and TDD with CE mode B	244

7.19.4.1	Minimum requirement when no DRX is used	245

7.19.4.2	Minimum requirement when DRX is used	246

7.19.4.3	Minimum requirement at transitions	247

7.19.5	Requirements for HD-FDD with CE mode B	247

7.19.5.1	Minimum requirement when no DRX is used	247

7.19.5.2	Minimum requirement when DRX is used	247

7.19.5.3	Minimum requirement at transitions	249

7.20	UE transmit timing for NB-IoT	249

7.20.1	Introduction	249

7.20.2	Requirements	249

7.21	 UE timer accuracy for NB-IoT	250

7.21.1	Introduction	250

7.21.2	Requirements	250

7.22	Timing Advance for NB-IoT	250

7.22.1	Introduction	250

7.22.2	Requirements	250

7.22.2.1	Timing Advance adjustment delay	250

7.22.2.2	Timing Advance adjustment accuracy	250

7.23	Radio Link Monitoring for Category NB1 UE	250

7.23.1	Introduction	250

7.23.2	Requirements for Category NB1 UE	250

7.23.2.1	Minimum requirement when no DRX is used	251

7.23.2.2	Minimum requirement when DRX is used	251

7.23.2.3	Minimum requirement at transitions	252

7.24	UE transmit timing for Category M1	252

7.24.1	Introduction	252

7.24.2	Requirements	252

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	253

7.25.1	Definition	253

7.25.2	Minimum requirements	253

7.26	UE transmit timing for Category M2	254

7.26.1	Introduction	254

7.26.2	Requirements	254

7.27	UE timer accuracy for category M1	254

7.27.1	Introduction	254

7.27.2	Requirements	254

7.28	Timing Advance for Category M1	254

7.28.1	Introduction	254

7.28.2	Requirements	254

7.29	Interruptions requirements with FeMBMS	254

7.29.1	Introduction	254

7.29.2	Requirements	255

7.30	Numerology switching delay requirements with FeMBMS	255

7.30.1	Introduction	255

7.30.2	Requirements	255

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	255

7.31.1	Introduction	255

7.31.2	NR PSCell Addition Delay Requirement	255

7.31.3	NR PSCell Release Delay Requirement	256

7.32	Interruptions with EN-DC	256

7.32.1	Introduction	256

7.32.2	Requirements	257

7.32.2.1	Interruptions at PSCell addition/release	257

7.32.2.2	Interruptions at transitions between active and non-active during DRX	257

7.32.2.3	Interruptions at transitions from non-DRX to DRX	257

7.32.2.4	Interruptions at SCell addition/release	257

7.32.2.5	Interruptions at SCell activation/deactivation	257

7.32.2.6	Interruptions during measurements on SCC	258

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	258

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	258

7.32.2.7	Interruptions at active BWP switching	258

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	259

7.33.1	Introduction	259

7.33.2	Requirements	259

7.34	Void	259

7.35	Interruptions with SFTD measurements	259

7.35.1	Introduction	259

7.35.2	Requirements	259

7.36	Interruptions with NE-DC	260

7.32.1	Introduction	260

7.36.2	Requirements	260

7.36.2.1	Interruptions at transitions between active and non-active during DRX	260

7.36.2.2	Interruptions at transitions from non-DRX to DRX	260

7.36.2.3	Interruptions at SCell addition/release	260

7.36.2.4	Interruptions at SCell activation/deactivation	260

7.36.2.5	Interruptions during measurements on SCC	261

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	261

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	261

7.36.2.6	Interruptions at active BWP switching	261

8	UE Measurements Procedures in RRC_CONNECTED State	262

8.1	General Measurement Requirements	262

8.1.1	Introduction	262

8.1.2	Requirements	262

8.1.2.1	UE measurement capability	262

8.1.2.1.1	Monitoring of multiple layers using gaps	268

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	270

8.1.2.1.1b	Monitoring of multiple layers using gaps (E-UTRA-NR dual connectivity)	271

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	272

8.1.2.1.2	Network controlled small gap	273

8.1.2.2	E-UTRAN intra frequency measurements	275

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	275

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	279

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	283

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	284

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	285

8.1.2.3	E-UTRAN inter frequency measurements	285

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	286

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	291

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	298

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	298

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	298

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	299

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	301

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	302

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	303

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	310

8.1.2.4	Inter RAT measurements	310

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	310

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	315

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	315

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	319

8.1.2.4.5	E-UTRAN FDD – GSM measurements	319

8.1.2.4.6	E-UTRAN TDD – GSM measurements	324

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	324

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	326

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	326

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	326

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	327

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	327

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	327

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	327

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	329

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	329

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	329

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	329

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	330

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	330

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	332

8.1.2.4.21	E-UTRAN FDD – NR measurements	332

8.1.2.4.22	E-UTRAN TDD – NR measurements	335

8.1.2.4.23	Void	335

8.1.2.4.24	Void	335

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	335

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	337

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	337

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	337

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	339

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	341

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	342

8.1.2.6.5	Void	344

8.1.2.6.6	Void	344

8.1.2.6.7	Void	344

8.1.2.6.8	Void	344

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	344

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	344

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	346

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	348

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	349

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	351

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	353

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	355

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	356

8.1.2.7	E-UTRAN E-CID Measurements	358

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	358

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	359

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	361

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	361

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	362

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	362

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	365

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	368

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	371

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	375

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	375

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	375

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	375

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	376

8.1.2.10	Void	376

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	376

8.2.1	Introduction	376

8.2.2	Requirements	377

8.3	Measurements for E-UTRA carrier aggregation	381

8.3.1	Introduction	381

8.3.2	Measurements of the primary component carrier	381

8.3.3	Measurements of a secondary component carrier	381

8.3.3.1	Measurements of a secondary component carrier with active SCell	381

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	382

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	382

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	383

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	384

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	385

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	385

8.4.1	Introduction	385

8.4.2	Measurements on the primary component carrier	385

8.4.3	Measurements on a secondary component carrier	386

8.4.4	Measurements on both primary component carrier and a secondary component carrier	387

8.4.5	Measurements on different secondary component carriers	388

8.5	Measurements for UE category 0	389

8.5.1	Introduction	389

8.5.2 	Requirements	389

8.5.2.1 	E-UTRAN intra frequency measurements	389

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	389

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	392

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	394

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	398

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	399

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	399

8.6	Discovery signal measurements	400

8.6.1	Introduction	400

8.6.2	Requirements for CRS based discovery signal measurements	400

8.6.2.1	E-UTRAN intra frequency measurements	400

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	400

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	403

8.6.2.2	E-UTRAN inter frequency measurements	405

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	406

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	408

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	411

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	411

8.6.3	Requirements for CSI-RS based discovery signal measurements	411

8.6.3.1	E-UTRAN intra frequency measurements	411

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	412

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	414

8.6.3.2	E-UTRAN inter frequency measurements	416

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	417

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	419

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	422

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	422

8.7	Discovery signal measurements for E-UTRA carrier aggregation	422

8.7.1	Introduction	422

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	423

8.7.2.1	Measurements of the primary component carrier	423

8.7.2.2	Measurements of a secondary component carrier	423

8.7.2.3	Measurements of a secondary component carrier with active SCell	423

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	423

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	423

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	424

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	426

8.7.3.1	Measurements of the primary component carrier	426

8.7.3.2	Measurements of a secondary component carrier	426

8.7.3.3	Measurements of a secondary component carrier with active SCell	426

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	426

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	426

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	428

8.8	Measurements for E-UTRA dual connectivity	429

8.8.1	Introduction	429

8.8.2	Intra-frequency measurements requirements on PCell	429

8.8.3	Intra-frequency measurements requirements on PSCell	430

8.8.4	Inter-frequency and inter-RAT measurement requirements	430

8.8.5	Intra-frequency measurements with autonomous gaps	430

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	430

8.8.5.2	ECGI reporting delay	431

8.8.6	Inter-frequency measurements with autonomous gaps	431

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	431

8.8.6.2	ECGI reporting delay	432

8.8.7	SSTD Measurements	432

8.8.7.1	Introduction	432

8.8.7.2	SSTD Measurement requirements	432

8.8.7.3	SSTD Measurement Reporting Delay	433

8.8.8	Intra-frequency measurements requirements on SCell	433

8.9	MBSFN Measurements	433

8.9.1	Introduction	433

8.9.2	MBSFN RSRP Measurements	433

8.9.3	MBSFN RSRQ Measurements	434

8.9.4	MCH BLER Measurements	434

8.10	Proximity-based Services	434

8.10.1	Introduction	434

8.10.2	Requirements	434

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	434

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	435

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	435

8.11.1	Introduction	435

8.11.2	CRS based discovery signal measurements	436

8.11.2.1	E-UTRAN intra-frequency measurements	436

8.11.2.1.1	Requirements	436

8.11.2.1.1.1	Requirements when no DRX is used	436

8.11.2.1.1.1.1	Measurement Reporting Requirements	437

8.11.2.1.1.2	Requirements when DRX is used	438

8.11.2.1.1.2.1	Measurement Reporting Requirements	440

8.11.2.2	E-UTRAN inter-frequency measurements	441

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	441

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	445

8.11.3	CSI-RS based discovery signal measurements	445

8.11.3.1	E-UTRAN intra-frequency measurements	445

8.11.3.1.1	Requirements	445

8.11.3.1.1.1	Requirements when no DRX is used	445

8.11.3.1.1.1.1	Measurement Reporting Requirements	447

8.11.3.1.1.2	Requirements when DRX is used	447

8.11.3.1.1.2.1	Measurement Reporting Requirements	448

8.11.3.2	E-UTRAN inter-frequency measurements	449

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	449

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	453

8.11.4	RSSI measurements	453

8.11.4.1	E-UTRAN intra-frequency measurements	453

8.11.4.2	E-UTRAN inter-frequency measurements	453

8.11.5	Channel occupancy measurements	453

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	453

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	454

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	454

8.12.1	Introduction	454

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	454

8.12.2.1	Introduction	454

8.12.2.2	Measurements of a secondary component carrier	454

8.12.2.3	Measurements of a secondary component carrier with active SCell	454

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	454

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	455

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	457

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	460

8.12.3.1	Introduction	460

8.12.3.2	Measurements of a secondary component carrier	460

8.12.3.3	Measurements of a secondary component carrier with active SCell	460

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	460

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	460

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	462

8.13	Measurements for UE Category M1	464

8.13.1	Introduction	464

8.13.2	Requirements for UE category M1 with CE mode A	464

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	465

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	465

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	469

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	471

8.13.2.2	Void	475

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	475

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	475

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	478

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	480

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	481

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	481

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	483

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	485

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	486

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	486

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	486

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	487

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	487

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	487

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	488

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	488

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	489

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	490

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	490

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	491

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	495

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	497

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	501

8.13.3	Requirements for UE category M1 with CE mode B	501

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	501

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	501

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	506

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	508

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	514

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	515

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	515

8.13.3.2	Void	516

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	516

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	516

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	519

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	521

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	522

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	522

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	522

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	522

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	523

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	523

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	524

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	524

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	524

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	528

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	531

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	535

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	536

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	536

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	538

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	540

8.14	Measurements for UE category NB1	541

8.14.1	Introduction	541

8.14.2	NB-IoT intra frequency measurements under normal coverage	541

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	541

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	541

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	541

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	541

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	541

8.15	Void	542

8.16	Measurements for UE Category M2	542

8.16.1	Introduction	542

8.16.2	Requirements for UE category M2 with CE mode A	542

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	542

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	543

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	543

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	544

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	544

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	544

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	545

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	545

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	547

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	550

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	550

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	550

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	552

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	555

8.16.3	Requirements for UE category M2 with CE mode B	555

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	555

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	555

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	558

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	560

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	561

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	561

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	563

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	565

8.17	Measurements for E-UTRA – NR Dual Connectivity	566

8.17.1	Introduction	566

8.17.1.1	Measurement Gap Sharing	566

8.17.1A	Intrafrequency Measurements	567

8.17.2	SFTD Measurements	567

8.17.2.1	Introduction	567

8.17.2.2	SFTD Measurement requirements	567

8.17.2.3	SFTD Measurement Reporting Delay	567

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	568

8.17.3.1	Introduction	568

8.17.3.2	E-UTRAN FDD inter frequency measurements	568

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	568

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	569

8.17.3.3	E-UTRAN TDD inter frequency measurements	571

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	571

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	573

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	575

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	575

8.17.4.1.1	NR Inter-RAT cell identification	575

8.17.4.1.2	NR Inter-RAT measurement	577

8.17.4.1.3	NR Inter-RAT measurement reporting	578

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	578

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	578

8.17.5.1	Introduction	578

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	579

8.17.5.2.1	Identification of a new UTRA FDD cell	579

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	579

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	579

8.17.5.2.4	Periodic Reporting	580

8.17.5.2.5	Event Triggered Reporting	580

8.17.5.2.6	Event-triggered Periodic Reporting	580

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	580

8.17.5.3.1	Periodic Reporting	581

8.17.5.3.2	Event Triggered Reporting	581

8.17.5.3.3	Event-triggered Periodic Reporting	582

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	582

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	582

8.17.7.1	Introduction	582

8.17.7.2	Identification of a new UTRA FDD cell for SON	582

8.17.7.2.1	Requirements when no DRX is used	582

8.17.7.2.2	Requirements when DRX is used	583

8.17.7.2.3	Reporting Delay	583

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	584

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	584

8.17.9.1	Introduction	584

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	584

8.17.9.2.1	Identification of a new UTRA FDD cell	584

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	584

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	585

8.17.9.2.4	Periodic Reporting	585

8.17.9.2.5	Event Triggered Reporting	585

8.17.9.2.6	Event-triggered Periodic Reporting	586

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	586

8.17.9.3.1	Periodic Reporting	587

8.17.9.3.2	Event Triggered Reporting	587

8.17.9.3.3	Event-triggered Periodic Reporting	587

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	588

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	588

8.17.11.1	Introduction	588

8.17.11.2	Identification of a new UTRA TDD cell for SON	588

8.17.11.2.1	Requirements when no DRX is used	588

8.17.11.2.2	Requirements when DRX is used	588

8.17.11.2.3	Reporting Delay	589

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	589

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	590

8.17.13.1	Introduction	590

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	590

8.17.13.2.1	GSM carrier RSSI	590

8.17.13.2.2	BSIC verification	590

8.17.13.2.3	Enhanced BSIC verification	592

8.17.13.2.4	Periodic Reporting	592

8.17.13.2.5	Event Triggered Reporting	592

8.17.13.2.6	Event-triggered Periodic Reporting	593

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	593

8.17.13.3.1	GSM carrier RSSI	593

8.17.13.3.2	BSIC verification	593

8.17.13.3.3	Periodic Reporting	595

8.17.13.3.4	Event Triggered Reporting	595

8.17.13.3.5	Event-triggered Periodic Reporting	595

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	595

8.18	Measurements for non-BL/CE UE	595

8.18.1	Introduction	595

8.18.2	Requirements for non-BL/CE UE with CE Mode B	595

8.18.2.1	E-UTRAN intra frequency measurements	595

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	595

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	596

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	596

8.19	Measurements for NR – E-UTRA Dual Connectivity	597

8.19.1	Introduction	597

8.19.2	Intra-frequency Measurements	598

8.19.3	Inter-frequency Measurements	598

8.19.4	Intra-frequency RSTD Measurements	598

8.19.5	Intra-frequency E-CID Measurements	598

9	Measurements performance requirements for UE	598

9.1	E-UTRAN measurements	599

9.1.1	Introduction	599

9.1.2	Intra-frequency RSRP Accuracy Requirements	599

9.1.2.1	Absolute RSRP Accuracy	599

9.1.2.2	Relative Accuracy of RSRP	600

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	601

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	601

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	602

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	603

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	604

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	605

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	606

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	606

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	606

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	607

9.1.2B.1	Introduction	607

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	607

9.1.3	Inter-frequency RSRP Accuracy Requirements	608

9.1.3.1	Absolute RSRP Accuracy	608

9.1.3.2	Relative Accuracy of RSRP	609

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	610

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	610

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	611

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	611

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	612

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	613

9.1.3B.1	Introduction	613

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	613

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	613

9.1.4	RSRP Measurement Report Mapping	614

9.1.5	Intra-frequency RSRQ Accuracy Requirements	614

9.1.5.1	Absolute RSRQ Accuracy	614

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	615

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	616

9.1.5.4	Absolute WB-RSRQ Accuracy	617

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	618

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	619

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	619

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	619

9.1.5B.1	Introduction	619

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	619

9.1.6	Inter-frequency RSRQ Accuracy Requirements	620

9.1.6.1	Absolute RSRQ Accuracy	620

9.1.6.2	Relative Accuracy of RSRQ	621

9.1.6.3	Absolute WB-RSRQ Accuracy	622

9.1.6.4	Relative WB-RSRQ Accuracy	622

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	623

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	624

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	624

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	624

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	625

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	626

9.1.6B.1	Introduction	626

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	626

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	626

9.1.7	RSRQ Measurement Report Mapping	627

9.1.8	Power Headroom	627

9.1.8.1	Period	628

9.1.8.2	Reporting Delay	628

9.1.8.3	Void	628

9.1.8.4	Report Mapping	628

9.1.9	UE Rx – Tx time difference	628

9.1.9.1	Measurement Requirement	628

9.1.9.2	Measurement Report mapping	629

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	630

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	631

9.1.10	Reference Signal Time Difference (RSTD)	632

9.1.10.1	Intra-Frequency Accuracy Requirement	632

9.1.10.2	Inter-Frequency Accuracy Requirement	633

9.1.10.3	RSTD Measurement Report Mapping	635

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	635

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	636

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	637

9.1.11	Carrier aggregation measurement accuracy	638

9.1.11.1	Primary component carrier accuracy requirement	639

9.1.11.2	Secondary component carrier accuracy requirement	639

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	639

9.1.11.4	Secondary component carrier relative accuracy requirement	639

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	639

9.1.13	Measurement accuracy for UE category 0	640

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	640

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	640

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	641

9.1.14	Accuracy requirements for Discovery Signal Measurements	642

9.1.14.1	Introduction	642

9.1.14.2	RSRP measurements in discovery signal occasions	642

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	642

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	642

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	642

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	643

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	644

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	644

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	644

9.1.14.3.3	CSI-RSRP measurement report mapping	645

9.1.14.4	RSRQ measurements in discovery signal occasions	645

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	645

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	646

9.1.15.1.1	Primary component carrier accuracy requirement	646

9.1.15.1.2	Secondary component carrier accuracy requirement	646

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	646

9.1.15.1.4	Secondary component carrier relative accuracy requirement	646

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	646

9.1.15.2.1	Primary component carrier accuracy requirement	646

9.1.15.2.2	Secondary component carrier accuracy requirement	646

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	646

9.1.15.2.4	Secondary component carrier relative accuracy requirement	646

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	647

9.1.17	RS-SINR Measurements	647

9.1.17.1	Measurement Report Mapping	647

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	648

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	648

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	648

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	648

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	649

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	650

9.1.18.1	Introduction	650

9.1.18.2	RSRP measurements	650

9.1.18.2.1	RSRP measurement report mapping	650

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	650

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	651

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	651

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	652

9.1.18.3	RSRQ measurements	652

9.1.18.3.1	RSRQ measurement report mapping	652

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	652

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	653

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	653

9.1.18.4	CSI-RSRP measurements	654

9.1.18.4.1	CSI-RSRP measurement report mapping	654

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	654

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	654

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	655

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	655

9.1.18.5	RSSI measurements	656

9.1.18.5.1	RSSI measurement report mapping	656

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	656

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	657

9.1.18.6	Channel occupancy measurements	657

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	657

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	657

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	658

9.1.19.1	Introduction	658

9.1.19.2	Accuracy requirements for measurements on SCC	658

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	658

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	658

9.1.20	SFN and Subframe Time Difference (SSTD)	659

9.1.20.1	SSTD Accuracy Requirement	659

9.1.20.2	SSTD Measurement Report Mapping	659

9.1.21	Measurement accuracy for UE category M1	660

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	660

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	661

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	662

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	663

9.1.21.5	RSRP Measurement Report Mapping	664

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	664

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	665

9.1.21.8	RSRQ Measurement Report Mapping	666

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	666

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	667

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	668

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	669

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	671

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	671

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	672

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	673

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	675

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	675

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	676

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	677

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	680

9.1.22	Measurement accuracy for UE Category NB1	683

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	683

9.1.22.2	Void	684

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	684

9.1.22.4	Void	685

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	685

9.1.22.6	Void	686

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	686

9.1.22.8	Void	687

9.1.22.9	NRSRP Measurement Report Mapping	687

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	687

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	688

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	689

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	690

9.1.22.14	NRSRQ Measurement Report Mapping	691

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	692

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	692

9.1.23	Power Headroom for UE Category NB1	693

9.1.23.1	Period	693

9.1.23.2	Reporting Delay	693

9.1.23.3	Report Mapping for UE Category NB1	693

9.1.23.3.1	Void	695

9.1.23.3.2	Void	695

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	695

9.1.24	Void	696

9.1.25	Measurement accuracy for UE category M2	696

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	696

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	697

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	698

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	699

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	701

9.1.26	Measurement Accuracy for non-BL CE UE	702

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	703

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	703

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	704

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	705

9.1.26.5	RSRP Measurement Report Mapping	706

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	706

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	706

9.1.26.8	RSRQ Measurement Report Mapping	707

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	707

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	708

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	709

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	710

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	711

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	711

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	711

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	712

9.1.27	SFN and frame Timing Difference (SFTD)	713

9.1.27.1	SFTD Accuracy Requirement	713

9.2	UTRAN FDD Measurements	715

9.2.1	UTRAN FDD CPICH RSCP	715

9.2.2	Void	716

9.2.3	UTRAN FDD CPICH Ec/No	716

9.3	UTRAN TDD Measurements	716

9.3.1	UTRAN TDD P-CCPCH RSCP	717

9.3.2	Void	717

9.3.3	Void	717

9.4	GSM Measurements	717

9.4.1	GSM carrier RSSI	717

9.5	CDMA2000 1x RTT Measurements	717

9.5.1	CDMA2000 1x RTT Pilot Strength	717

9.6	PCMAX,c	718

9.6.1	Report Mapping	718

9.6.2 	Estimation Period	718

9.6.3	Reporting Delay	718

9.7	IEEE802.11 Measurements	718

9.7.1	WLAN RSSI	718

9.7.2	WLAN RSSI Measurement Report Mapping	718

9.8	MBSFN Measurements	719

9.8.1	Introduction	719

9.8.2	MBSFN RSRP	719

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	719

9.8.2.2	MBSFN RSRP measurement report mapping	720

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	720

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	721

9.8.3	MBSFN RSRQ	721

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	721

9.8.3.2	MBSFN RSRQ measurement report mapping	722

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	722

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	723

9.8.4	MCH BLER	723

9.8.4.1	Measurement report mapping for MCH BLER	723

9.8.4.2	Measurement report mapping for MCH Block Number	724

9.9	ProSe Measurements	725

9.9.1	Introduction	725

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	725

9.9.2.1	Absolute S-RSRP Accuracy	725

9.9.2.2	Relative Accuracy of S-RSRP	726

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	727

9.9.3.1	Absolute SD-RSRP Accuracy	727

9.9.3.2	Relative Accuracy of SD-RSRP	727

9.10	V2X Measurements	728

9.10.1	Introduction	728

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	728

9.10.2.1	Absolute S-RSRP Accuracy	728

9.10.2.2	Relative Accuracy of S-RSRP	729

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	729

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	729

9.10.4	S-RSSI Measurement Accuracy Requirements	730

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	730

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	730

9.11	NR Measurements	731

9.11.1	NR SS-RSRP Measurements	731

9.11.2	NR SS-RSRQ Measurements	731

9.11.3	NR SS-SINR Measurements	732

10	Measurements Performance Requirements for E-UTRAN	732

10.1	Received Interference Power	732

10.1.1	Absolute accuracy requirement	732

10.1.2	Relative accuracy requirement	733

10.1.3	Received Interference Power measurement report mapping	733

10.2	Angle of Arrival (AOA)	733

10.2.1	Range/mapping	733

10.3	Timing Advance (TADV)	733

10.3.1	Report mapping	733

11	ProSe Requirements in Any Cell Selection state	734

11.1	Introduction	734

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	734

11.2.1	Introduction	734

11.2.2	ProSe UE transmission timing	734

11.3	Initiation/Cease of SLSS Transmissions	735

11.3.1	Introduction	735

11.3.2	Requirements	735

11.4	Measurements for ProSe in Any Cell Selection State	735

11.4.1	Introduction	735

11.4.2	Requirements	735

11.4.2.1	E-UTRA FDD	735

11.4.2.2	E-UTRA TDD	736

11.5	Selection / Reselection of ProSe Synchronization Reference	736

11.5.1	Introduction	736

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	736

11.5.2.1	Introduction	736

11.5.2.2	Requirements	736

11.6	Void	737

11.7	Selection / Reselection of ProSe relay UE	737

11.7.1	Introduction	737

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	737

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	737

12.1	Introduction	737

12.2	Transmit Timing	738

12.2.1	GNSS as timing reference	738

12.3	Interruption	738

12.4	Reliability of GNSS signal	738

13	V2X Requirements	738

13.1	Introduction	738

13.2	UE Transmit Timing	739

13.2.1	Introduction	739

13.2.2	GNSS as synchronization reference source	739

13.2.3	Serving cell/PCell as synchronization reference source	739

13.2.4	SyncRef UE as synchronization reference source	739

13.3	Initiation/Cease of SLSS Transmissions	739

13.3.1	Introduction	739

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	739

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	740

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	740

13.4	Selection / Reselection of V2X Synchronization Reference Source	741

13.5	Autonomous Resource Selection/Reselection measurements	742

13.5.1	Introduction	742

13.5.2	PSSCH-RSRP measurements	742

13.5.3	S-RSSI measurements	742

13.6	Congestion Control measurements	742

13.7	Interruption	742

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	742

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	742

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	743

13.8	Reliability of GNSS signal	743

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	743

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	744

Annex A (normative):	Test Cases	745

A.1	Purpose of annex	745

A.2	Requirement classification for statistical testing	745

A.2.1	Types of requirements in TS 36.133	745

A.2.1.1	Time and delay requirements on UE higher layer actions	745

A.2.1.2	Measurements of power levels, relative powers and time	745

A.2.1.3	Implementation requirements	746

A.2.1.4	Physical layer timing requirements	746

A.3	RRM test configurations	747

A.3.1	Reference Measurement Channels	747

A.3.1.1	PDSCH	747

A.3.1.1.1	FDD	747

A.3.1.1.2	TDD	751

A.3.1.1.3	FDD for UE category 0	754

A.3.1.1.4	HD-FDD for UE category 0	755

A.3.1.1.5	TDD for UE category 0	756

A.3.1.1.6	Frame Structure 3	757

A.3.1.2	PCFICH/PDCCH/PHICH	758

A.3.1.2.1	FDD	758

A.3.1.2.2	TDD	758

A.3.1.2.3	HD-FDD for UE category 0	759

A.3.1.2.4	FS 3	759

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	759

A.3.1.3.1	FDD in CEModeA	760

A.3.1.3.2	HD-FDD in CEModeA	760

A.3.1.3.3	TDD in CEModeA	761

A.3.1.3.4	FDD in CEModeB	761

A.3.1.3.5	HD-FDD in CEModeB	762

A.3.1.3.6	TDD in CEModeB	762

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	763

A.3.1.4.1	FDD in CEModeA	763

A.3.1.4.2	HD-FDD in CEModeA	764

A.3.1.4.3	TDD in CEModeA	765

A.3.1.4.4	FDD in CEModeB	766

A.3.1.4.5	 HD-FDD in CEModeB	767

A.3.1.4.6	TDD in CEModeB	767

A.3.1.5	NPDSCH Reference Channel for UE category NB1	768

A.3.1.5.1	HD-FDD in-band operation	768

A.3.1.5.2	Void	769

A.3.1.5.3	HD-FDD standalone operation	769

A.3.1.5.4	Void	770

A.3.1.5.5	HD-FDD guard band operation	770

A.3.1.5.6	Void	771

A.3.1.5.7	TDD in-band operation	771

A.3.1.5.8	TDD standalone operation	771

A.3.1.5.9	TDD guard band operation	772

A.3.1.6	NPDCCH Reference Channel for UE category NB1	772

A.3.1.6.1	In-band operation	772

A.3.1.6.2	Void	773

A.3.1.6.3	Standalone operation	773

A.3.1.6.4	Void	773

A.3.1.6.5	Guard band operation	773

A.3.1.6.6	Void	774

A.3.2	OFDMA Channel Noise Generator (OCNG)	774

A.3.2.1	OCNG Patterns for FDD	774

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	775

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	775

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	776

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	777

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	777

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	778

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	778

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	778

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	779

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	780

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	780

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	781

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	781

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	782

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	782

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	783

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	783

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	784

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	784

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	785

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	785

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	786

A.3.2.2	OCNG Patterns for TDD	786

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	787

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	787

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	788

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	788

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	789

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	790

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	791

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	792

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	792

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	793

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	793

A.3.2.3	OCNG Patterns for Narrowband IoT	794

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	795

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	796

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	796

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	797

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	798

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	799

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	800

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	801

A.3.2.4	OCNG Patterns for V2X sidelink	802

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	802

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	803

A.3.3	Reference DRX Configurations	803

A.3.4	ABS Transmission Configurations	804

A.3.4.1	Non-MBSFN ABS Transmission Configurations	804

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	804

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	804

A.3.4.2	MBSFN ABS Transmission Configurations	805

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	805

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	806

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	806

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	806

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	807

A.3.6.1	Introduction	807

A.3.7	Test Cases with Different Channel Bandwidths	807

A.3.7.1	Introduction	807

A.3.7.2	Principle of testing	807

A.3.8	Antenna Configuration	807

A.3.8.1	Antenna connection for 4 Rx capable UEs	807

A.3.8.1.1 Introduction	807

A.3.8.1.2 Principle of testing	807

A.3.8.1.2.1 Single carrier tests	807

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	808

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	809

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	809

A.3.8.2	Antenna connection for 8 Rx capable UEs	809

A.3.8.2.1	Introduction	809

A.3.8.2.2	Principle of testing	809

A.3.8.2.2.1	Single carrier tests	809

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	810

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	810

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	810

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	810

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	810

A.3.9.1	Introduction	810

A.3.9.2	Principle of testing	810

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	811

A.3.10.1	Introduction	811

A.3.10.2	Principle of testing	811

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	811

A.3.11.1	Introduction	811

A.3.11.2	Principle of Testing	811

A.3.12	Proximity-based Services	811

A.3.12.1	Introduction	811

A.3.12.2	Reference DRX configurations for ProSe tests	811

A.3.12.3	Test Cases with Different Channel Bandwidths	812

A.3.12.3.1	Introduction	812

A.3.12.3.2	Principle of testing	812

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	812

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	818

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	820

A.3.12.6.1	FDD	820

A.3.12.7	Reference measurement channels for ProSe Direct Communication	820

A.3.12.7.1	FDD	820

A.3.12.8	ProSe Receive Traffic Generator	821

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	821

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	821

A.3.13	Time Offset between Cells	822

A.3.13.1	Introduction	822

A.3.13.2	Definition	822

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	822

A.3.14.1	Introduction	822

A.3.14.2	Principle of testing	822

A.3.15	Dual connectivity test cases with different combination of duplex mode	822

A.3.15.1	Introduction	822

A.3.15.2	Principle of testing	822

A.3.16	Reference PRACH Configurations	823

A.3.17	Listen before talk model	823

A.3.17.1	Introduction	823

A.3.17.2	Definition	823

A.3.18	Reference NPRACH Configurations	824

A.3.19	Dual connectivity test cases with different bandwidth combinations	825

A.3.19.1	Introduction	825

A.3.19.2	Principle of testing	825

A.3.20	Category M1 UE Test Cases	826

A.3.20.1	Introduction	826

A.3.20.2	Principle of Cat-M1 UE Testing	826

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	827

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	828

A.3.21.1	Introduction	828

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	828

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	829

A.3.22	Category 1bis UE Test Cases	830

A.3.22.1	Introduction	830

A.3.22.2	Principle of Category 1bis UE Testing	830

A.3.23	Category NB2 UE Test Cases	833

A.3.23.1	Introduction	833

A.3.23.2	Principle of Category NB2 UE Testing	833

A.3.24	V2X sidelink communication	835

A.3.24.1	Introduction	835

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	836

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	839

A.3.25	Category M2 UE Test Cases	840

A.3.25.1	Introduction	840

A.3.25.2	Principle of Cat-M2 UE Testing	840

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	841

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	842

A.3.26.1	Introduction	842

A.3.26.2	Principle of testing	842

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	842

A.3.27.1	Introduction	842

A.3.27.2	Principle of INACTIVE cell re-selection Testing	842

A.4	E-UTRAN RRC_IDLE state	842

A.4.2	Cell Re-Selection	842

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	842

A.4.2.1.1	Test Purpose and Environment	842

A.4.2.1.2	Test Requirements	844

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	845

A.4.2.2.1	Test Purpose and Environment	845

A.4.2.2.2	Test Requirements	846

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	847

A.4.2.3.1	Test Purpose and Environment	847

A.4.2.3.2	Test Requirements	848

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	849

A.4.2.4.1	Test Purpose and Environment	849

A.4.2.4.2	Test Requirements	850

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	851

A.4.2.5.1	Test Purpose and Environment	851

A.4.2.5.2	Test Requirements	852

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	853

A.4.2.6.1	Test Purpose and Environment	853

A.4.2.6.2	Test Requirements	854

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	855

A.4.2.7.1	Test Purpose and Environment	855

A.4.2.7.2	Test Requirements	856

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	857

A.4.2.8.1	Test Purpose and Environment	857

A.4.2.8.2	Test Requirements	858

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	859

A.4.2.9.1	Test Purpose and Environment	859

A.4.2.9.2	Test Requirements	859

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	859

A.4.2.10.1	Test Purpose and Environment	859

A.4.2.10.2	Test Requirements	863

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	863

A.4.2.11.1	Test Purpose and Environment	863

A.4.2.11.2	Test Requirements	867

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	867

A.4.2.12.1	Test Purpose and Environment	867

A.4.2.12.2	Test Requirements	869

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	870

A.4.2.13.1	Test Purpose and Environment	870

A.4.2.13.2	Test Requirements	871

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	872

A.4.2.14.1	Test Purpose and Environment	872

A.4.2.14.2	Test Requirements	873

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	874

A.4.2.15.1	Test Purpose and Environment	874

A.4.2.15.2	Test Requirements	875

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	876

A.4.2.16.1	Test Purpose and Environment	876

A.4.2.16.2	Test Requirements	877

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	878

A.4.2.17.1	Test Purpose and Environment	878

A.4.2.17.2	Test Requirements	879

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	880

A.4.2.18.1	Test Purpose and Environment	880

A.4.2.18.2	Test Requirements	882

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	883

A.4.2.19.1	Test Purpose and Environment	883

A.4.2.19.2	Test Requirements	885

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	886

A.4.2.20.1	Test Purpose and Environment	886

A.4.2.20.2	Test Requirements	887

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	888

A.4.2.21.1	Test Purpose and Environment	888

A.4.2.21.2	Test Requirements	889

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	890

A.4.2.22.1	Test Purpose and Environment	890

A.4.2.22.2	Test Requirements	891

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	892

A.4.2.23.1	Test Purpose and Environment	892

A.4.2.23.2	Test Requirements	893

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	894

A.4.2.24.1	Test Purpose and Environment	894

A.4.2.24.2	Test Requirements	896

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	897

A.4.2.25.1	Test Purpose and Environment	897

A.4.2.25.2	Test Requirements	898

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	899

A.4.2.26.1	Test Purpose and Environment	899

A.4.2.26.2	Test Requirements	900

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	901

A.4.2.27.1	Test Purpose and Environment	901

A.4.2.27.2	Test Requirements	902

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	903

A.4.2.28.1	Test Purpose and Environment	903

A.4.2.28.2	Test Requirements	904

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	905

A.4.2.29.1	Test Purpose and Environment	905

A.4.2.29.2	Test Requirements	906

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	907

A.4.2.30.1	Test Purpose and Environment	907

A.4.2.30.2	Test Requirements	908

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	909

A.4.2.31.1	Test Purpose and Environment	909

A.4.2.31.2	Test Requirements	910

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	911

A.4.2.32.1	Test Purpose and Environment	911

A.4.2.32.2	Test Requirements	912

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	913

A.4.2.33.1	Test Purpose and Environment	913

A.4.2.33.2	Test Requirements	914

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	915

A.4.2.34.1	Test Purpose and Environment	915

A.4.2.34.2	Test Requirements	916

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	917

A.4.2.35.1	Test Purpose and Environment	917

A.4.2.35.2	Test Requirements	919

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	920

A.4.2.36.1	Test Purpose and Environment	920

A.4.2.36.2	Test Requirements	922

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	923

A.4.2.37.1	Test Purpose and Environment	923

A.4.2.37.2	Test Requirements	925

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	926

A.4.2.38.1	Test Purpose and Environment	926

A.4.2.38.2	Test Requirements	928

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	929

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	929

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	929

A.4.3.1.1.1	Test Purpose and Environment	929

A.4.3.1.1.2	Test Requirements	931

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	931

A.4.3.1.2.1	Test Purpose and Environment	931

A.4.3.1.2.2	Test Requirements	934

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	934

A.4.3.1.3.1	Test Purpose and Environment	934

A.4.3.1.3.2	Test Requirements	937

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	937

A.4.3.1.4.1	Test Purpose and Environment	937

A.4.3.1.4.2	Test Requirements	938

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	938

A.4.3.1.5.1	Test Purpose and Environment	938

A.4.3.1.5.2	Test Requirements	941

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	942

A.4.3.2.1	Test Purpose and Environment	942

A.4.3.2.1.1	Void	942

A.4.3.2.1.2	1.28Mcps TDD option	942

A.4.3.2.1.3	Void	944

A.4.3.2.2	Test Requirements	944

A.4.3.2.2.1	1.28Mcps TDD option	944

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	944

A.4.3.2A.1	Test Purpose and Environment	944

A.4.3.2A.2	Test Requirements	948

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	948

A.4.3.3.1	Test Purpose and Environment	948

A.4.3.3.2	Test Requirements	951

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	951

A.4.3.3A.1	Test Purpose and Environment	951

A.4.3.3A.2	Test Requirements	956

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	957

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	957

A.4.3.4.1.1	Test Purpose and Environment	957

A.4.3.4.1.2	Test Requirements	959

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	959

A.4.3.4.2.1	Test Purpose and Environment	959

A.4.3.4.2.2	Test Requirements	961

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	961

A.4.3.4.3.1	Test Purpose and Environment	961

A.4.3.4.3.2	Test Requirements	964

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	964

A.4.3.4.4.1	Test Purpose and Environment	964

A.4.3.4.4.2	Test Requirements	968

A.4.4	E-UTRAN to GSM Cell Re-Selection	968

A.4.4.1	E-UTRAN FDD – GSM:	968

A.4.4.1.1	Test Purpose and Environment	968

A.4.4.1.2	Test Requirements	970

A.4.4.2	E-UTRAN TDD – GSM:	970

A.4.4.2.1	Test Purpose and Environment	970

A.4.4.2.2	Test Requirements	972

A.4.5	E-UTRAN to HRPD Cell Re-Selection	973

A.4.5.1	E-UTRAN FDD – HRPD	973

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	973

A.4.5.1.1.1	Test Purpose and Environment	973

A.4.5.1.1.2	Test Requirements	975

A.4.5.2	E-UTRAN TDD – HRPD	975

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	975

A.4.5.2.1.1	Test Purpose and Environment	975

A.4.5.2.1.2	Test Requirements	978

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	978

A.4.6.1	E-UTRAN FDD – cdma2000 1X	978

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	978

A.4.6.1.1.1	Test Purpose and Environment	978

A.4.6.1.1.2	Test Requirements	981

A.4.6.2	E-UTRAN TDD – cdma2000 1X	981

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	981

A.4.6.2.1.1	Test Purpose and Environment	981

A.4.6.2.1.2	Test Requirements	984

A.4.7	Idle State Positioning Measurement for UE category NB1	984

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	984

A.4.7.1.1	Test Purpose and Environment	984

A.4.7.1.2	Test Requirements	988

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	989

A.4.7.2.1	Test Purpose and Environment	989

A.4.7.4.2	Test Requirements	992

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	993

A.4.7.3.1	Test Purpose and Environment	993

A.4.7.3.2	Test Requirements	996

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	997

A.4.7.4.1	Test Purpose and Environment	997

A.4.7.4.2	Test Requirements	1000

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1001

A.5.1	E-UTRAN Handover	1001

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1001

A.5.1.1.1	Test Purpose and Environment	1001

A.5.1.1.2	Test Requirements	1003

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1003

A.5.1.2.1	Test Purpose and Environment	1003

A.5.1.2.2	Test Requirements	1005

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1005

A.5.1.3.1	Test Purpose and Environment	1005

A.5.1.3.2	Test Requirements	1007

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1007

A.5.1.4.1	Test Purpose and Environment	1007

A.5.1.4.2	Test Requirements	1009

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1009

A.5.1.5.1	Test Purpose and Environment	1009

A.5.1.5.2	Test Requirements	1011

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1011

A.5.1.6.1	Test Purpose and Environment	1011

A.5.1.6.2	Test Requirements	1013

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1013

A.5.1.7.1	Test Purpose and Environment	1013

A.5.1.7.2	Test Requirements	1016

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1016

A.5.1.8.1	Test Purpose and Environment	1016

A.5.1.8.2	Test Requirements	1019

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1019

A.5.1.9.1	Test Purpose and Environment	1019

A.5.1.9.2	Test Requirements	1020

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1020

A.5.1.10.1	Test Purpose and Environment	1020

A.5.1.10.2	Test Requirements	1022

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1022

A.5.1.11.1	Test Purpose and Environment	1022

A.5.1.11.2	Test Requirements	1024

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1024

A.5.1.12.1	Test Purpose and Environment	1024

A.5.1.12.2	Test Requirements	1026

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1026

A.5.1.13.1	Test Purpose and Environment	1026

A.5.1.13.2	Test Requirements	1028

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1029

A.5.1.14.1	Test Purpose and Environment	1029

A.5.1.14.2	Test Requirements	1030

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1031

A.5.1.15.1	Test Purpose and Environment	1031

A.5.1.15.2	Test Requirements	1032

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1033

A.5.1.16.1	Test Purpose and Environment	1033

A.5.1.16.2	Test Requirements	1034

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1035

A.5.1.17.1	Test Purpose and Environment	1035

A.5.1.17.2	Test Requirements	1036

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1037

A.5.1.18.1	Test Purpose and Environment	1037

A.5.1.18.2	Test Requirements	1038

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1039

A.5.1.19.1	Test Purpose and Environment	1039

A.5.1.19.2	Test Requirements	1040

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1041

A.5.1.20.1	Test Purpose and Environment	1041

A.5.1.20.2	Test Requirements	1042

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1043

A.5.1.21.1	Test Purpose and Environment	1043

A.5.1.21.2	Test Requirements	1044

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1044

A.5.1.22.1	Test Purpose and Environment	1044

A.5.1.22.2	Test Requirements	1046

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1046

A.5.1.23.1	Test Purpose and Environment	1046

A.5.1.23.2	Test Requirements	1048

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1048

A.5.1.24.1	Test Purpose and Environment	1048

A.5.1.24.2	Test Requirements	1050

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1050

A.5.1.25.1	Test Purpose and Environment	1050

A.5.1.25.2	Test Requirements	1052

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1052

A.5.1.26.1	Test Purpose and Environment	1052

A.5.1.26.2	Test Requirements	1054

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1055

A.5.1.27.1	Test Purpose and Environment	1055

A.5.1.27.2	Test Requirements	1056

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1057

A.5.1.28.1	Test Purpose and Environment	1057

A.5.1.28.2	Test Requirements	1058

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1059

A.5.1.29.1	Test Purpose and Environment	1059

A.5.1.29.2	Test Requirements	1060

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1061

A.5.1.30.1	Test Purpose and Environment	1061

A.5.1.30.2	Test Requirements	1062

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1063

A.5.1.31.1	Test Purpose and Environment	1063

A.5.1.31.2	Test Requirements	1064

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1065

A.5.1.32.1	Test Purpose and Environment	1065

A.5.1.32.2	Test Requirements	1066

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1067

A.5.1.33.1	Test Purpose and Environment	1067

A.5.1.13.2	Test Requirements	1068

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1069

A.5.1.34.1	Test Purpose and Environment	1069

A.5.1.34.2	Test Requirements	1070

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1071

A.5.1.35.1	Test Purpose and Environment	1071

A.5.1.35.2	Test Requirements	1072

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1073

A.5.1.36.1	Test Purpose and Environment	1073

A.5.1.36.2	Test Requirements	1074

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1075

A.5.1.37.1	Test Purpose and Environment	1075

A.5.1.37.2	Test Requirements	1076

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1077

A.5.1.38.1	Test Purpose and Environment	1077

A.5.1.38.2	Test Requirements	1078

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1079

A.5.1.39.1	Test Purpose and Environment	1079

A.5.1.39.2	Test Requirements	1080

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1081

A.5.1.40.1	Test Purpose and Environment	1081

A.5.1.40.2	Test Requirements	1082

A.5.2	E-UTRAN Handover to other RATs	1083

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1083

A.5.2.1.1	Test Purpose and Environment	1083

A.5.2.1.2	Test Requirements	1085

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1086

A.5.2.2.1	Test Purpose and Environment	1086

A.5.2.2.2	Test Requirements	1089

A.5.2.3 E-UTRAN FDD- GSM Handover	1089

A.5.2.3.1	Test Purpose and Environment	1089

A.5.2.3.2	Test Requirements	1090

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1091

A.5.2.4.1	Test Purpose and Environment	1091

A.5.2.4.1.1	Void	1091

A.5.2.4.1.2	1.28 Mcps TDD option	1091

A.5.2.4.1.3	Void	1093

A.5.2.4.2	Test Requirements	1093

A.5.2.4.2.1	Void	1093

A.5.2.4.2.2	1.28 Mcps TDD option	1093

A.5.2.4.2.3	Void	1093

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1093

A.5.2.5.1	Test Purpose and Environment	1093

A.5.2.5.1.3	Void	1096

A.5.2.5.2	Test Requirements	1096

A.5.2.5.2.1	Void	1096

A.5.2.5.2.2	1.28 Mcps TDD option	1096

A.5.2.5.2.3	Void	1096

A.5.2.6	E-UTRAN TDD - GSM Handover	1096

A.5.2.6.1	Test Purpose and Environment	1096

A.5.2.6.2	Test Requirements	1098

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1099

A.5.2.7.1	Test Purpose and Environment	1099

A.5.2.7.2	Test Requirements	1101

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1101

A.5.2.8.1	Test Purpose and Environment	1101

A.5.2.8.2	Test Requirements	1102

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1103

A.5.2.9.1	Test Purpose and Environment	1103

A.5.2.9.2	Test Requirements	1104

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1105

A.5.2.10.1	Test Purpose and Environment	1105

A.5.2.10.2	Test Requirements	1107

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1107

A.5.2.10A.1	Test Purpose and Environment	1107

A.5.2.10A.2	Test Requirements	1110

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1110

A.5.2.10B.1	Test Purpose and Environment	1110

A.5.2.10B.2	Test Requirements	1113

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1113

A.5.2.11.1	Test Purpose and Environment	1113

A.5.2.11.2	Test Requirements	1113

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1114

A.5.3.1	E-UTRAN FDD – HRPD Handover	1114

A.5.3.1.1	Test Purpose and Environment	1114

A.5.3.1.2	Test Requirements	1116

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1116

A.5.3.2.1	Test Purpose and Environment	1116

A.5.3.2.2	Test Requirements	1119

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1119

A.5.3.3.1	Test Purpose and Environment	1119

A.5.3.3.2	Test Requirements	1121

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1122

A.5.3.4.1	Test Purpose and Environment	1122

A.5.3.4.2	Test Requirements	1123

A.5.3.5	E-UTRAN TDD – HRPD Handover	1124

A.5.3.5.1	Test Purpose and Environment	1124

A.5.3.5.2	Test Requirements	1126

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1127

A.5.3.6.1	Test Purpose and Environment	1127

A.5.3.6.2	Test Requirements	1130

A.6	RRC Connection Control	1130

A.6.1	RRC Re-establishment	1130

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1130

A.6.1.1.1	Test Purpose and Environment	1130

A.6.1.1.2	Test Requirements	1131

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1132

A.6.1.2.1	Test Purpose and Environment	1132

A.6.1.2.2	Test Requirements	1133

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1134

A.6.1.3.1	Test Purpose and Environment	1134

A.6.1.3.2	Test Requirements	1135

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1136

A.6.1.4.1	Test Purpose and Environment	1136

A.6.1.4.2	Test Requirements	1137

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1138

A.6.1.5.1	Test Purpose and Environment	1138

A.6.1.5.2	Test Requirements	1138

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1138

A.6.1.6.1	Test Purpose and Environment	1138

A.6.1.6.2	Test Requirements	1140

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1141

A.6.1.7.1	Test Purpose and Environment	1141

A.6.1.7.2	Test Requirements	1142

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1143

A.6.1.8.1	Test Purpose and Environment	1143

A.6.1.8.2	Test Requirements	1144

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1145

A.6.1.9.1	Test Purpose and Environment	1145

A.6.1.9.2	Test Requirements	1146

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1147

A.6.1.10.1	Test Purpose and Environment	1147

A.6.1.10.2	Test Requirements	1148

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1149

A.6.1.11.1	Test Purpose and Environment	1149

A.6.1.11.2	Test Requirements	1150

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1151

A.6.1.12.1	Test Purpose and Environment	1151

A.6.1.12.2	Test Requirements	1152

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1153

A.6.1.13.1	Test Purpose and Environment	1153

A.6.1.13.2	Test Requirements	1154

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1155

A.6.1.14.1	Test Purpose and Environment	1155

A.6.1.14.2	Test Requirements	1156

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1157

A.6.1.15.1	Test Purpose and Environment	1157

A.6.1.15.2	Test Requirements	1159

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1160

A.6.1.16.1	Test Purpose and Environment	1160

A.6.1.16.2	Test Requirements	1162

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1163

A.6.1.17.1	Test Purpose and Environment	1163

A.6.1.17.2	Test Requirements	1164

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1165

A.6.1.18.1	Test Purpose and Environment	1165

A.6.1.18.2	Test Requirements	1166

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1167

A.6.1.19.1	Test Purpose and Environment	1167

A.6.1.19.2	Test Requirements	1168

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1169

A.6.1.20.1	Test Purpose and Environment	1169

A.6.1.20.2	Test Requirements	1170

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1171

A.6.1.21.1	Test Purpose and Environment	1171

A.6.1.21.2	Test Requirements	1172

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1173

A.6.1.22.1	Test Purpose and Environment	1173

A.6.1.22.2	Test Requirements	1174

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1175

A.6.1.23.1	Test Purpose and Environment	1175

A.6.1.23.2	Test Requirements	1177

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1178

A.6.1.24.1	Test Purpose and Environment	1178

A.6.1.24.2	Test Requirements	1180

A.6.2	Random Access	1181

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1181

A.6.2.1.1	Test Purpose and Environment	1181

A.6.2.1.2	Test Requirements	1182

A.6.2.1.2.1	Random Access Response Reception	1182

A.6.2.1.2.2	No Random Access Response Reception	1182

A.6.2.1.2.3	Receiving a NACK on msg3	1182

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1183

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1183

A.6.2.1.2.6	Contention Resolution Timer expiry	1183

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1183

A.6.2.2.1	Test Purpose and Environment	1183

A.6.2.2.2	Test Requirements	1184

A.6.2.2.2.1	Random Access Response Reception	1185

A.6.2.2.2.2	No Random Access Response Reception	1185

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1185

A.6.2.3.1	Test Purpose and Environment	1185

A.6.2.3.2	Test Requirements	1187

A.6.2.3.2.1	Random Access Response Reception	1187

A.6.2.3.2.2	No Random Access Response reception	1187

A.6.2.3.2.3	Receiving a NACK on msg3	1187

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1188

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1188

A.6.2.3.2.6	Contention Resolution Timer expiry	1188

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1188

A.6.2.4.1	Test Purpose and Environment	1188

A.6.2.4.2	Test Requirements	1190

A.6.2.4.2.1	Random Access Response Reception	1190

A.6.2.4.2.2	No Random Access Response Reception	1190

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1190

A.6.2.5.1	Test Purpose and Environment	1190

A.6.2.5.2	Test Requirements	1191

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1191

A.6.2.6.1	Test Purpose and Environment	1191

A.6.2.6.2	Test Requirements	1191

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1192

A.6.2.7.1	Test Purpose and Environment	1192

A.6.2.7.2	Test Requirements	1194

A.6.2.7.2.1	Random Access Response Reception	1194

A.6.2.7.2.2	No Random Access Response Reception	1194

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1194

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1195

A.6.2.8.1	Test Purpose and Environment	1195

A.6.2.8.2	Test Requirements	1197

A.6.2.8.2.1	Random Access Response Reception	1197

A.6.2.8.2.2	No Random Access Response Reception	1197

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1197

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1198

A.6.2.9.1	Test Purpose and Environment	1198

A.6.2.9.2	Test Requirements	1200

A.6.2.9.2.1	Random Access Response Reception	1200

A.6.2.9.2.2	No Random Access Response Reception	1201

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1201

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1202

A.6.2.10.1	Test Purpose and Environment	1202

A.6.2.10.2	Test Requirements	1204

A.6.2.10.2.1	Random Access Response Reception	1204

A.6.2.10.2.2	No Random Access Response Reception	1204

A.6.2.10.2.3	Receiving a NACK on msg3	1205

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1205

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1205

A.6.2.10.2.6	Contention Resolution Timer expiry	1205

A.6.2.10.2.7	PRACH Resource Selection	1205

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1205

A.6.2.11.1	Test Purpose and Environment	1205

A.6.2.11.2	Test Requirements	1207

A.6.2.11.2.1	Random Access Response Reception	1207

A.6.2.11.2.2	No Random Access Response Reception	1208

A.6.2.11.2.3	Receiving a NACK on msg3	1208

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1208

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1208

A.6.2.11.2.6	Contention Resolution Timer expiry	1208

A.6.2.11.2.7	PRACH Resource Selection	1209

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1209

A.6.2.12.1	Test Purpose and Environment	1209

A.6.2.12.2	Test Requirements	1211

A.6.2.12.2.1	Random Access Response Reception	1211

A.6.2.12.2.2	No Random Access Response Reception	1212

A.6.2.12.2.3	Receiving a NACK on msg3	1212

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1212

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1212

A.6.2.12.2.6	Contention Resolution Timer expiry	1212

A.6.2.12.2.7	PRACH Resource Selection	1213

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1213

A.6.2.13.1	Test Purpose and Environment	1213

A.6.2.13.2	Test Requirements	1215

A.6.2.13.2.1	Random Access Response Reception	1215

A.6.2.13.2.2	No Random Access Response Reception	1215

A.6.2.13.2.3	Receiving a NACK on msg3	1216

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1216

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1216

A.6.2.13.2.6	Contention Resolution Timer expiry	1216

A.6.2.13.2.7	PRACH Resource Selection	1216

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1216

A.6.2.14.1	Test Purpose and Environment	1216

A.6.2.14.2	Test Requirements	1218

A.6.2.14.2.1	Random Access Response Reception	1218

A.6.2.14.2.2	No Random Access Response Reception	1219

A.6.2.14.2.3	Receiving a NACK on msg3	1219

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1219

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1219

A.6.2.14.2.6	Contention Resolution Timer expiry	1219

A.6.2.14.2.7	PRACH Resource Selection	1220

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1220

A.6.2.15.1	Test Purpose and Environment	1220

A.6.2.15.2	Test Requirements	1222

A.6.2.15.2.1	Random Access Response Reception	1222

A.6.2.15.2.2	No Random Access Response Reception	1223

A.6.2.15.2.3	Receiving a NACK on msg3	1223

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1223

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1223

A.6.2.15.2.6	Contention Resolution Timer expiry	1223

A.6.2.15.2.7	PRACH Resource Selection	1224

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1224

A.6.2.16.1	Test Purpose and Environment	1224

A.6.2.16.2	Test Requirements	1227

A.6.2.16.2.1	Random Access Response Reception	1227

A.6.2.16.2.2	No Random Access Response Reception	1228

A.6.2.16.2.3	Receiving a NACK on msg3	1228

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1228

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1228

A.6.2.16.2.6	Contention Resolution Timer expiry	1228

A.6.2.16.2.7	NPRACH Resource Selection	1228

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1229

A.6.2.17.1	Test Purpose and Environment	1229

A.6.2.17.2	Test Requirements	1232

A.6.2.17.2.1	Random Access Response Reception	1232

A.6.2.17.2.2	No Random Access Response Reception	1233

A.6.2.17.2.3	Receiving a NACK on msg3	1233

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1233

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1233

A.6.2.17.2.6	Contention Resolution Timer expiry	1233

A.6.2.17.2.7	NPRACH Resource Selection	1233

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1234

A.6.2.18.1	Test Purpose and Environment	1234

A.6.2.18.2	Test Requirements	1237

A.6.2.18.2.1	Random Access Response Reception	1237

A.6.2.18.2.2	No Random Access Response Reception	1238

A.6.2.18.2.3	Receiving a NACK on msg3	1238

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1238

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1238

A.6.2.18.2.6	Contention Resolution Timer expiry	1238

A.6.2.18.2.7	NPRACH Resource Selection	1238

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1239

A.6.2.19.1	Test Purpose and Environment	1239

A.6.2.19.2	Test Requirements	1241

A.6.2.19.2.1	Random Access Response Reception	1241

A.6.2.19.2.2	No Random Access Response Reception	1242

A.6.2.19.2.3	Receiving a NACK on msg3	1242

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1242

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1242

A.6.2.19.2.6	Contention Resolution Timer expiry	1242

A.6.2.19.2.7	NPRACH Resource Selection	1242

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1243

A.6.2.20.1	Test Purpose and Environment	1243

A.6.2.20.2	Test Requirements	1245

A.6.2.20.2.1	Random Access Response Reception	1245

A.6.2.20.2.2	No Random Access Response Reception	1246

A.6.2.20.2.3	Receiving a NACK on msg3	1246

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1246

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1246

A.6.2.20.2.6	Contention Resolution Timer expiry	1246

A.6.2.20.2.7	NPRACH Resource Selection	1246

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1247

A.6.2.21.1	Test Purpose and Environment	1247

A.6.2.21.2	Test Requirements	1249

A.6.2.21.2.1	Random Access Response Reception	1249

A.6.2.21.2.2	No Random Access Response Reception	1250

A.6.2.21.2.3	Receiving a NACK on msg3	1250

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1250

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1250

A.6.2.21.2.6	Contention Resolution Timer expiry	1250

A.6.2.21.2.7	NPRACH Resource Selection	1250

A.6.3	RRC Connection Release with Redirection	1251

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1251

A.6.3.1.1	Test Purpose and Environment	1251

A.6.3.1.2	Test Requirements	1253

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1253

A.6.3.2.1	Test Purpose and Environment	1253

A.6.3.2.2	Test Requirements	1255

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1255

A.6.3.3.1	Test Purpose and Environment	1255

A.6.3.3.2	Test Requirements	1256

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1257

A.6.3.4.1	Test Purpose and Environment	1257

A.6.3.4.2	Test Requirements	1258

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1259

A.6.3.5.1	Test Purpose and Environment	1259

A.6.3.5.2	Test Requirements	1261

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1261

A.6.3.6.1	Test Purpose and Environment	1261

A. 6.3.6.2	Test Requirements	1263

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1263

A.6.3.7.1	Test Purpose and Environment	1263

A.6.3.7.2	Test Requirements	1266

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1266

A.6.3.8.1	Test Purpose and Environment	1266

A.6.3.8.2	Test Requirements	1269

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1269

A.6.3.9.1	Test Purpose and Environment	1269

A.6.3.9.2	Test Requirements	1271

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1271

A.6.3.10.1	Test Purpose and Environment	1271

A.6.3.10.2	Test Requirements	1273

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1273

A.6.3.11.1	Test Purpose and Environment	1273

A.6.3.11.2	Test Requirements	1275

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1275

A.6.3.12.1	Test Purpose and Environment	1275

A.6.3.12.2	Test Requirements	1278

A.7	Timing and Signalling Characteristics	1279

A.7.1	UE Transmit Timing	1279

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1279

A.7.1.1.1	Test Purpose and Environment	1279

A.7.1.1.2	Test Requirements	1280

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1281

A.7.1.2.1	Test Purpose and Environment	1281

A.7.1.2.2	Test Requirements	1283

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1284

A.7.1.3.1	Test Purpose and Environment	1284

A.7.1.3.2	Test Requirements	1286

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1287

A.7.1.4.1	Test Purpose and Environment	1287

A.7.1.4.2	Test Requirements	1288

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1289

A.7.1.4A.1	Test Purpose and Environment	1289

A.7.1.4A.2	Test Requirements	1289

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1289

A.7.1.5.1	Test Purpose and Environment	1289

A.7.1.5.2	Test Requirements	1290

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1290

A.7.1.6.1	Test Purpose and Environment	1290

A.7.1.6.2	Test Requirements	1292

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1293

A.7.1.7.1	Test Purpose and Environment	1293

A.7.1.7.2	Test Requirements	1294

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1295

A.7.1.7A.1	Test Purpose and Environment	1295

A.7.1.7A.2	Test Requirements	1295

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1295

A.7.1.7B.1	Test Purpose and Environment	1295

A.7.1.7B.2	Test Requirements	1295

A.7.1.8	Void	1296

A.7.1.8.1	Void	1296

A.7.1.8.2	Void	1296

A.7.1.9	Void	1296

A.7.1.9.1	Void	1296

A.7.1.9.2	Void	1296

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1296

A.7.1.10.1	Test Purpose and Environment	1296

A.7.1.10.2	Test Requirements	1298

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1298

A.7.1.11.1	Test Purpose and Environment	1298

A.7.1.11.2	Test Requirements	1300

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1300

A.7.1.12.1	Test Purpose and Environment	1300

A.7.1.12.2	Test Requirements	1302

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1302

A.7.1.13.1	Test Purpose and Environment	1302

A.7.1.13.2	Test Requirements	1304

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1305

A.7.1.14.1	Test Purpose and Environment	1305

A.7.1.14.2	Test Requirements	1306

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1306

A.7.1.15.1	Test Purpose and Environment	1306

A.7.1.15.2	Test Requirements	1307

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1308

A.7.1.16.1	Test Purpose and Environment	1308

A.7.1.16.2	Test Requirements	1309

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1310

A.7.1.17.1	Test Purpose and Environment	1310

A.7.1.17.2	Test Requirements	1312

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1313

A.7.1.18.1	Test Purpose and Environment	1313

A.7.1.18.2	Test Requirements	1315

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1316

A.7.1.19.1	Test Purpose and Environment	1316

A.7.1.19.2	Test Requirements	1317

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1317

A.7.1.20.1	Test Purpose and Environment	1317

A.7.1.20.2	Test Requirements	1319

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1319

A.7.1.21.1	Test Purpose and Environment	1319

A.7.1.21.2	Test Requirements	1321

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1321

A.7.1.22.1	Test Purpose and Environment	1321

A.7.1.22.2	Test Requirements	1323

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1323

A.7.1.23.1	Test Purpose and Environment	1323

A.7.1.23.2	Test Requirements	1325

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1325

A.7.1.24.1	Test Purpose and Environment	1325

A.7.1.24.2	Test Requirements	1326

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1327

A.7.1.25.1	Test Purpose and Environment	1327

A.7.1.25.2	Test Requirements	1328

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1329

A.7.1.26.1	Test Purpose and Environment	1329

A.7.1.26.2	Test Requirements	1330

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1331

A.7.1.27.1	Test Purpose and Environment	1331

A.7.1.27.2	Test Requirements	1333

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1333

A.7.1.28.1	Test Purpose and Environment	1333

A.7.1.28.2	Test Requirements	1335

A.7.2	UE Timing Advance	1336

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1336

A.7.2.1.1	Test Purpose and Environment	1336

A.7.2.1.2	Test Requirements	1337

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1338

A.7.2.2.1	Test Purpose and Environment	1338

A.7.2.2.2	Test Requirements	1340

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1340

A.7.2.3.1	Test Purpose and Environment	1340

A.7.2.3.2	Test Requirements	1340

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1340

A.7.2.4.1	Test Purpose and Environment	1340

A.7.2.4.2	Test Requirements	1343

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1343

A.7.2.5.1	Test Purpose and Environment	1343

A.7.2.5.2	Test Requirements	1345

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1345

A.7.2.5A.1	Test Purpose and Environment	1345

A.7.2.5A.2	Test Requirements	1346

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1346

A.7.2.5B.1	Test Purpose and Environment	1346

A.7.2.5B.2	Test Requirements	1346

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1346

A.7.2.6.1	Test Purpose and Environment	1346

A.7.2.6.2	Test Requirements	1349

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1349

A.7.2.7.1	Test Purpose and Environment	1349

A.7.2.7.2	Test Requirements	1351

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1351

A.7.2.8.1	Test Purpose and Environment	1351

A.7.2.8.2	Test Requirements	1353

A.7.2.9.2	Test Requirements	1355

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1355

A.7.2.10.1	Test Purpose and Environment	1355

A.7.2.10.2	Test Requirements	1356

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1356

A.7.2.11.1	Test Purpose and Environment	1356

A.7.2.11.2	Test Requirements	1358

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1358

A.7.2.12.1	Test Purpose and Environment	1358

A.7.2.12.2	Test Requirements	1360

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1360

A.7.2.13.1	Test Purpose and Environment	1360

A.7.2.13.2	Test Requirements	1362

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1362

A.7.2.14.1	Test Purpose and Environment	1362

A.7.2.14.2	Test Requirements	1364

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1364

A.7.2.15.1	Test Purpose and Environment	1364

A.7.2.15.2	Test Requirements	1366

A.7.3	Radio Link Monitoring	1366

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1367

A.7.3.1.1	Test Purpose and Environment	1367

A.7.3.1.2	Test Requirements	1371

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1371

A.7.3.2.1	Test Purpose and Environment	1371

A.7.3.2.2	Test Requirements	1374

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1374

A.7.3.3.1	Test Purpose and Environment	1374

A.7.3.3.2	Test Requirements	1378

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1378

A.7.3.4.1	Test Purpose and Environment	1378

A.7.3.4.2	Test Requirements	1381

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1381

A.7.3.5.1	Test Purpose and Environment	1381

A.7.3.5.2	Test Requirements	1385

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1386

A.7.3.6.1	Test Purpose and Environment	1386

A.7.3.6.2	Test Requirements	1389

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1389

A.7.3.7.1	Test Purpose and Environment	1389

A.7.3.7.2	Test Requirements	1392

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1392

A.7.3.8.1	Test Purpose and Environment	1392

A.7.3.8.2	Test Requirements	1395

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1395

A.7.3.9.2	Test Requirements	1398

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1398

A.7.3.10.1	Test Purpose and Environment	1398

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1401

A.7.3.11.1	Test Purpose and Environment	1401

A.7.3.11.2	Test Requirements	1405

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1405

A.7.3.12.1	Test Purpose and Environment	1405

A.7.3.12.2	Test Requirements	1409

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1409

A.7.3.13.1	Test Purpose and Environment	1409

A.7.3.13.2	Test Requirements	1412

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1412

A.7.3.14.1	Test Purpose and Environment	1412

A.7.3.14.2	Test Requirements	1415

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1415

A.7.3.15.1	Test Purpose and Environment	1415

A.7.3.15.2	Test Requirements	1418

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1418

A.7.3.16.1	Test Purpose and Environment	1418

A.7.3.16.2	Test Requirements	1422

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1422

A.7.3.17.1	Test Purpose and Environment	1422

A.7.3.17.2	Test Requirements	1425

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1425

A.7.3.18.1	Test Purpose and Environment	1425

A.7.3.18.2	Test Requirements	1429

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1429

A.7.3.19.1	Test Purpose and Environment	1429

A.7.3.19.2	Test Requirements	1433

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1433

A.7.3.20.1	Test Purpose and Environment	1433

A.7.3.20.2	Test Requirements	1437

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1437

A.7.3.21.1	Test Purpose and Environment	1437

A.7.3.21.2	Test Requirements	1441

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1441

A.7.3.22.1	Test Purpose and Environment	1441

A.7.3.22.2	Test Requirements	1445

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1445

A.7.3.23.1	Test Purpose and Environment	1445

A.7.3.23.2	Test Requirements	1446

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1446

A.7.3.24.1	Test Purpose and Environment	1446

A.7.3.24.2	Test Requirements	1447

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1447

A.7.3.25.1	Test Purpose and Environment	1447

A.7.3.25.2	Test Requirements	1448

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1448

A.7.3.26.1	Test Purpose and Environment	1448

A.7.3.26.2	Test Requirements	1451

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1451

A.7.3.27.1	Test Purpose and Environment	1451

A.7.3.27.2	Test Requirements	1454

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1454

A.7.3.28.1	Test Purpose and Environment	1454

A.7.3.28.2	Test Requirements	1457

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1457

A.7.3.29.1	Test Purpose and Environment	1457

A.7.3.29.2	Test Requirements	1460

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1460

A.7.3.30.1	Test Purpose and Environment	1460

A.7.3.30.2	Test Requirements	1463

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1463

A.7.3.31.1	Test Purpose and Environment	1463

A.7.3.31.2	Test Requirements	1466

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1466

A.7.3.32.1	Test Purpose and Environment	1466

A.7.3.32.2	Test Requirements	1469

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1469

A.7.3.33.1	Test Purpose and Environment	1469

A.7.3.33.2	Test Requirements	1472

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1472

A.7.3.34.1	Test Purpose and Environment	1472

A.7.3.34.2	Test Requirements	1475

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1475

A.7.3.35.1	Test Purpose and Environment	1475

A.7.3.35.2	Test Requirements	1478

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1478

A.7.3.36.1	Test Purpose and Environment	1478

A.7.3.36.2	Test Requirements	1481

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1481

A.7.3.37.1	Test Purpose and Environment	1481

A.7.3.37.2	Test Requirements	1484

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1484

A.7.3.38.1	Test Purpose and Environment	1484

A.7.3.38.2	Test Requirements	1487

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1487

A.7.3.39.1	Test Purpose and Environment	1487

A.7.3.39.2	Test Requirements	1490

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1491

A.7.3.40.1	Test Purpose and Environment	1491

A.7.3.40.2	Test Requirements	1493

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1494

A.7.3.41.1	Test Purpose and Environment	1494

A.7.3.41.2	Test Requirements	1497

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1497

A.7.3.42.1	Test Purpose and Environment	1497

A.7.3.42.2	Test Requirements	1500

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1500

A.7.3.43.1	Test Purpose and Environment	1500

A.7.3.43.2	Test Requirements	1503

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1503

A.7.3.44.1	Test Purpose and Environment	1503

A.7.3.44.2	Test Requirements	1506

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1507

A.7.3.45.1	Test Purpose and Environment	1507

A.7.3.45.2	Test Requirements	1509

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1510

A.7.3.46.1	Test Purpose and Environment	1510

A.7.3.46.2	Test Requirements	1513

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1513

A.7.3.47.1	Test Purpose and Environment	1513

A.7.3.47.2	Test Requirements	1516

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1516

A.7.3.48.1	Test Purpose and Environment	1516

A.7.3.48.2	Test Requirements	1519

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1519

A.7.3.49.1	Test Purpose and Environment	1519

A.7.3.49.2	Test Requirements	1522

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1522

A.7.3.50.1	Test Purpose and Environment	1522

A.7.3.50.2	Test Requirements	1525

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1525

A.7.3.51.1	Test Purpose and Environment	1525

A.7.3.51.2	Test Requirements	1528

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1528

A.7.3.52.1	Test Purpose and Environment	1528

A.7.3.52.2	Test Requirements	1531

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1531

A.7.3.53.1	Test Purpose and Environment	1531

A.7.3.53.2	Test Requirements	1534

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1534

A.7.3.54.1	Test Purpose and Environment	1534

A.7.3.54.2	Test Requirements	1537

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1537

A.7.3.55.1	Test Purpose and Environment	1537

A.7.3.55.2	Test Requirements	1540

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1540

A.7.3.56.1	Test Purpose and Environment	1540

A.7.3.56.2	Test Requirements	1543

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1543

A.7.3.57.1	Test Purpose and Environment	1543

A.7.3.57.2	Test Requirements	1546

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1546

A.7.3.58.1	Test Purpose and Environment	1546

A.7.3.58.2	Test Requirements	1549

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1549

A.7.3.59.1	Test Purpose and Environment	1549

A.7.3.59.2	Test Requirements	1552

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1552

A.7.3.60.1	Test Purpose and Environment	1552

A.7.3.60.2	Test Requirements	1556

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1556

A.7.3.61.1	Test Purpose and Environment	1556

A.7.3.61.2	Test Requirements	1560

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1560

A.7.3.62.1	Test Purpose and Environment	1560

A.7.3.62.2	Test Requirements	1564

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1564

A.7.3.63.1	Test Purpose and Environment	1564

A.7.3.63.2	Test Requirements	1568

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1568

A.7.3.64.1	Test Purpose and Environment	1568

A.7.3.64.2	Test Requirements	1571

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1572

A.7.3.65.1	Test Purpose and Environment	1572

A.7.3.65.2	Test Requirements	1575

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1576

A.7.3.66.1	Test Purpose and Environment	1576

A.7.3.66.2	Test Requirements	1578

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1579

A.7.3.67.1	Test Purpose and Environment	1579

A.7.3.67.2	Test Requirements	1582

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1583

A.7.3.68.1	Test Purpose and Environment	1583

A.7.3.68.2	Test Requirements	1585

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1585

A.7.3.69.1	Test Purpose and Environment	1585

A.7.3.69.2	Test Requirements	1587

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1587

A.7.3.70.1	Test Purpose and Environment	1587

A.7.3.70.2	Test Requirements	1589

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1589

A.7.3.71.1	Test Purpose and Environment	1589

A.7.3.71.2	Test Requirements	1592

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1592

A.7.3.72.1	Test Purpose and Environment	1592

A.7.3.72.2	Test Requirements	1594

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1594

A.7.3.73.1	Test Purpose and Environment	1594

A.7.3.73.2	Test Requirements	1596

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1596

A.7.3.74.1	Test Purpose and Environment	1596

A.7.3.74.2	Test Requirements	1599

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1599

A.7.3.75.1	Test Purpose and Environment	1599

A.7.3.75.2	Test Requirements	1601

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1602

A.7.3.76.1	Test Purpose and Environment	1602

A.7.3.76.2	Test Requirements	1605

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1605

A.7.3.77.1	Test Purpose and Environment	1605

A.7.3.77.2	Test Requirements	1608

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1608

A.7.3.78.1	Test Purpose and Environment	1608

A.7.3.78.2	Test Requirements	1611

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1611

A.7.3.79.1	Test Purpose and Environment	1611

A.7.3.79.2	Test Requirements	1614

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1614

A.7.3.80.1	Test Purpose and Environment	1614

A.7.3.80.2	Test Requirements	1617

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1617

A.7.3.81.1	Test Purpose and Environment	1617

A.7.3.81.2	Test Requirements	1620

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1620

A.7.3.82.1	Test Purpose and Environment	1620

A.7.3.82.2	Test Requirements	1623

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1623

A.7.3.83.1	Test Purpose and Environment	1623

A.7.3.83.2	Test Requirements	1624

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1625

A.7.3.84.1	Test Purpose and Environment	1625

A.7.3.84.2	Test Requirements	1627

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1627

A.7.3.85.1	Test Purpose and Environment	1627

A.7.3.85.2	Test Requirements	1628

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1629

A.7.3.86.1	Test Purpose and Environment	1629

A.7.3.86.2	Test Requirements	1631

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1631

A.7.3.87.1	Test Purpose and Environment	1631

A.7.3.87.2	Test Requirements	1633

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1634

A.7.3.88.1	Test Purpose and Environment	1634

A.7.3.88.2	Test Requirements	1638

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1638

A.7.3.89.1	Test Purpose and Environment	1638

A.7.3.89.2	Test Requirements	1642

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1642

A.7.3.90.1	Test Purpose and Environment	1642

A.7.3.90.2	Test Requirements	1646

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1646

A.7.3.91.1	Test Purpose and Environment	1646

A.7.3.91.2	Test Requirements	1650

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1650

A.7.3.92.1	Test Purpose and Environment	1650

A.7.3.92.2	Test Requirements	1653

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1654

A.7.3.93.1	Test Purpose and Environment	1654

A.7.3.93.2	Test Requirements	1657

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1658

A.7.3.94.1	Test Purpose and Environment	1658

A.7.3.94.2	Test Requirements	1660

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1661

A.7.3.95.1	Test Purpose and Environment	1661

A.7.3.95.2	Test Requirements	1664

A.7.4	Interruption for Dual Connectivity	1665

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1665

A.7.4.1.1	Test Purpose and Environment	1665

A.7.4.1.2	Test Requirements	1667

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1667

A.7.4.2.1	Test Purpose and Environment	1667

A.7.4.2.2	Test Requirements	1669

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1669

A.7.4.3.1	Test Purpose and Environment	1669

A.7.4.3.2	Test Requirements	1671

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1671

A.7.4.4.1	Test Purpose and Environment	1671

A.7.4.4.2	Test Requirements	1674

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1674

A.7.4.5.1	Test Purpose and Environment	1674

A.7.4.5.2	Test Requirements	1676

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1676

A.7.4.6.1	Test Purpose and Environment	1676

A.7.4.6.2	Test Requirements	1678

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1678

A.7.4.7.1	Test Purpose and Environment	1678

A.7.4.7.2	Test Requirements	1681

A.7.5	Proximity-based Services	1681

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1681

A.7.5.1.1	Test Purpose and Environment	1681

A.7.5.1.2	Test Requirements	1682

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1683

A.7.5.2.1	Test Purpose and Environment	1683

A.7.5.1.2	Test Requirements	1683

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1684

A.7.5.3.1	Test Purpose and Environment	1684

A.7.5.3.2	Test Requirements	1685

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1686

A.7.5.4.1	Test Purpose and Environment	1686

A.7.5.4.2	Test Requirements	1687

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1688

A.7.5.5.1	Test Purpose and Environment	1688

A.7.5.5.2	Test Requirements	1690

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1691

A.7.5.6.1	Test Purpose and Environment	1691

A.7.5.6.2	Test Requirements	1692

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1693

A.7.5.7.1	Test Purpose and Environment	1693

A.7.5.7.2	Test Requirements	1695

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1695

A.7.5.8.1	Test Purpose and Environment	1695

A.7.5.8.2	Test Requirements	1697

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1698

A.7.5.9.1	Test Purpose and Environment	1698

A.7.5.9.2	Test Requirements	1700

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1701

A.7.5.10.1	Test Purpose and Environment	1701

A.7.5.10.2	Test Requirements	1703

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1704

A.7.5.11.1	Test Purpose and Environment	1704

A.7.5.11.2	Test Requirements	1706

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1706

A.7.5.12.1	Test Purpose and Environment	1706

A.7.5.12.2	Test Requirements	1710

A.7.6	Interruption for carrier aggregation	1711

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1711

A.7.6.1.1	Test Purpose and Environment	1711

A.7.6.1.2	Test Requirements	1713

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1713

A.7.6.2.1	Test Purpose and Environment	1713

A.7.6.2.2	Test Requirements	1716

A.8	UE Measurements Procedures	1716

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1716

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1716

A.8.1.1.1	Test Purpose and Environment	1716

A.8.1.1.2	Test Requirements	1718

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1718

A.8.1.2.1	Test Purpose and Environment	1718

A.8.1.2.2	Test Requirements	1720

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1720

A.8.1.3.1	Test Purpose and Environment	1720

A.8.1.3.2	Test Requirements	1723

A.8.1.4	Void	1723

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1723

A.8.1.5.1	Test Purpose and Environment	1723

A.8.1.5.2	Test Requirements	1725

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1725

A.8.1.6.1	Test Purpose and Environment	1725

A.8.1.6.2	Test Requirements	1727

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1727

A.8.1.7.1	Test Purpose and Environment	1727

A.8.1.7.2	Test Requirements	1729

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1730

A.8.1.8.1	Test Purpose and Environment	1730

A.8.1.8.2	Test Requirements	1732

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1733

A.8.1.9.1	Test Purpose and Environment	1733

A.8.1.9.2	Test Requirements	1733

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1733

A.8.1.10.1	Test Purpose and Environment	1733

A.8.1.10.2	Test Requirements	1734

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1734

A.8.1.11.1	Test Purpose and Environment	1734

A.8.1.11.2	Test Requirements	1736

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1737

A.8.1.12.1	Test Purpose and Environment	1737

A.8.1.12.2	Test Requirements	1738

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1739

A.8.1.13.1	Test Purpose and Environment	1739

A.8.1.13.2	Test Requirements	1741

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1741

A.8.1.14.1	Test Purpose and Environment	1741

A.8.1.14.2	Test Requirements	1743

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1744

A.8.1.15.1	Test Purpose and Environment	1744

A.8.1.15.2	Test Requirements	1745

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1746

A.8.1.16.1	Test Purpose and Environment	1746

A.8.1.16.2	Test Requirements	1748

A.8.1.17	Void	1748

A.8.1.18	Void	1748

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1748

A.8.1.19.1	Test Purpose and Environment	1748

A.8.1.19.2	Test Requirements	1750

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1751

A.8.1.20.1	Test Purpose and Environment	1751

A.8.1.20.2	Test Requirements	1753

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1753

A.8.1.21.1	Test Purpose and Environment	1753

A.8.1.21.2	Test Requirements	1755

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1756

A.8.1.22.1	Test Purpose and Environment	1756

A.8.1.22.2	Test Requirements	1758

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1758

A.8.1.23.1	Test Purpose and Environment	1758

A.8.1.23.2	Test Requirements	1759

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1760

A.8.1.24.1	Test Purpose and Environment	1760

A.8.1.24.2	Test Requirements	1762

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1762

A.8.1.25.1	Test Purpose and Environment	1762

A.8.1.25.2	Test Requirements	1764

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1764

A.8.1.26.1	Test Purpose and Environment	1764

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1767

A.8.1.27.1	Test Purpose and Environment	1767

A.8.1.27.2	Test Requirements	1769

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1769

A.8.1.28.1	Test Purpose and Environment	1769

A.8.1.28.2	Test Requirements	1771

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1771

A.8.1.29.1	Test Purpose and Environment	1771

A.8.1.29.2	Test Requirements	1773

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1774

A.8.1.30.1	Test Purpose and Environment	1774

A.8.1.30.2	Test Requirements	1776

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1776

A.8.1.31.1	Test Purpose and Environment	1776

A.8.1.31.2	Test Requirements	1778

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1779

A.8.1.32.1	Test Purpose and Environment	1779

A.8.1.32.2	Test Requirements	1780

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1781

A.8.1.33.1	Test Purpose and Environment	1781

A.8.1.33.2	Test Requirements	1782

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1783

A.8.1.34.1	Test Purpose and Environment	1783

A.8.1.34.2	Test Requirements	1784

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1785

A.8.1.35.1	Test Purpose and Environment	1785

A.8.1.35.2	Test Requirements	1786

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1787

A.8.1.36.1	Test Purpose and Environment	1787

A.8.1.36.2	Test Requirements	1788

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1789

A.8.1.37.1	Test Purpose and Environment	1789

A.8.1.37.2	Test Requirements	1791

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1791

A.8.1.38.1	Test Purpose and Environment	1791

A.8.1.38.2	Test Requirements	1792

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1793

A.8.1.39.1	Test Purpose and Environment	1793

A.8.1.39.2	Test Requirements	1795

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1795

A.8.1.40.1	Test Purpose and Environment	1795

A.8.1.40.2	Test Requirements	1797

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1797

A.8.1.41.1	Test Purpose and Environment	1797

A.8.1.41.2	Test Requirement	1799

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1800

A.8.1.42.1	Test Purpose and Environment	1800

A.8.1.42.2	Test Requirement	1801

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1802

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1802

A.8.2.1.1	Test Purpose and Environment	1802

A.8.2.1.2	Test Requirements	1803

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1804

A.8.2.2.1	Test Purpose and Environment	1804

A.8.2.2.2	Test Requirements	1806

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1806

A.8.2.3.1	Test Purpose and Environment	1806

A.8.2.3.2	Test Requirements	1808

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1809

A.8.2.4.1	Test Purpose and Environment	1809

A.8.2.4.2	Test Requirements	1811

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1811

A.8.2.5.1	Test Purpose and Environment	1811

A.8.2.5.2	Test Requirements	1813

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1814

A.8.2.6.1	Test Purpose and Environment	1814

A.8.2.6.2	Test Requirements	1816

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1817

A.8.2.7.1	Test Purpose and Environment	1817

A.8.2.7.2	Test Requirements	1818

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1819

A.8.2.8.1	Test Purpose and Environment	1819

A.8.2.8.2	Test Requirements	1821

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1821

A.8.2.9.1	Test Purpose and Environment	1821

A.8.2.9.2	Test Requirements	1823

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1824

A.8.2.10.1	Test Purpose and Environment	1824

A.8.2.10.2	Test Requirements	1826

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1826

A.8.2.11.1	Test Purpose and Environment	1826

A.8.2.11.2	Test Requirements	1828

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1828

A.8.2.12.1	Test Purpose and Environment	1828

A.8.2.12.2	Test Requirements	1830

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1831

A.8.2.13.1	Test Purpose and Environment	1831

A.8.2.13.2	Test Requirements	1833

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1833

A.8.2.14.1	Test Purpose and Environment	1833

A.8.2.14.2	Test Requirement	1835

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	1836

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1836

A.8.3.1.1	Test Purpose and Environment	1836

A.8.3.1.2	Test Requirements	1837

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1837

A.8.3.2.1	Test Purpose and Environment	1837

A.8.3.2.2	Test Requirements	1840

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	1840

A.8.3.3.1	Test Purpose and Environment	1840

A.8.3.3.2	Test Requirements	1843

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1843

A.8.3.4.1	Test Purpose and Environment	1843

A.8.3.4.2	Test Requirements	1845

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1845

A.8.3.5.2	Test Requirements	1847

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	1847

A.8.3.6.1	Test Purpose and Environment	1847

A.8.3.6.2	Test Requirements	1849

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1850

A.8.3.7.1	Test Purpose and Environment	1850

A.8.3.7.2	Test Requirements	1852

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1853

A.8.3.8.1	Test Purpose and Environment	1853

A.8.3.8.2	Test Requirements	1856

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1856

A.8.3.9.1	Test Purpose and Environment	1856

A.8.3.9.2	Test Requirements	1859

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1860

A.8.3.10.1	Test Purpose and Environment	1860

A.8.3.10.2	Test Requirements	1861

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	1861

A.8.3.11.1	Test Purpose and Environment	1861

A.8.3.11.2	Test Requirement	1863

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1864

A.8.3.12.1	Test Purpose and Environment	1864

A.8.3.12.2	Test Requirement	1865

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1866

A.8.3.13.1	Test Purpose and Environment	1866

A.8.3.13.2	Test Requirement	1867

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1868

A.8.3.14.1	Test Purpose and Environment	1868

A.8.3.14.2	Test Requirement	1869

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1870

A.8.3.15.1	Test Purpose and Environment	1870

A.8.3.15.2	Test Requirement	1871

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	1872

A.8.3.16.1	Test Purpose and Environment	1872

A.8.3.16.2	Test Requirement	1874

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1874

A.8.3.17.1	Test Purpose and Environment	1874

A.8.3.17.2	Test Requirement	1877

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1877

A.8.3.18.1	Test Purpose and Environment	1877

A.8.3.18.2	Test Requirement	1880

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1880

A.8.3.19.1	Test Purpose and Environment	1880

A.8.3.19.2	Test Requirement	1883

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	1883

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1883

A.8.4.1.1	Test Purpose and Environment	1883

A.8.4.1.2	Test Requirements	1885

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	1885

A.8.4.2.1	Test Purpose and Environment	1885

A.8.4.2.2	Test Requirements	1888

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	1888

A.8.4.3.1	Test Purpose and Environment	1888

A.8.4.3.2	Test Requirements	1890

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1891

A.8.4.4.1	Test Purpose and Environment	1891

A.8.4.4.2	Test Requirements	1893

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1894

A.8.4.5.1	Test Purpose and Environment	1894

A.8.4.5.2	Test Requirements	1896

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	1896

A.8.4.6.1	Test Purpose and Environment	1896

A.8.4.6.2	Test Requirements	1897

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1897

A.8.4.7.1	Test Purpose and Environment	1897

A.8.4.7.2	Test Requirements	1901

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1901

A.8.4.8.1	Test Purpose and Environment	1901

A.8.4.8.2	Test Requirements	1904

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1904

A.8.4.9.1	Test Purpose and Environment	1904

A.8.4.9.2	Test Requirements	1908

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1908

A.8.4.10.1	Test Purpose and Environment	1908

A.8.4.10.2	Test Requirements	1910

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	1910

A.8.4.11.1	Test Purpose and Environment	1910

A.8.4.11.2	Test Requirement	1912

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1912

A.8.4.12.1	Test Purpose and Environment	1912

A.8.4.12.2	Test Requirement	1914

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1915

A.8.4.13.1	Test Purpose and Environment	1915

A.8.4.13.2	Test Requirement	1916

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1917

A.8.4.14.1	Test Purpose and Environment	1917

A.8.4.14.2	Test Requirement	1919

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1919

A.8.4.15.1	Test Purpose and Environment	1919

A.8.4.15.2	Test Requirement	1922

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	1922

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	1922

A.8.5.1.1	Test Purpose and Environment	1922

A.8.5.1.2	Test Requirements	1924

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	1924

A.8.5.2.1	Test Purpose and Environment	1924

A.8.5.2.2	Test Requirements	1926

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	1926

A.8.5.3.1	Test Purpose and Environment	1926

A.8.5.3.2	Test Requirements	1929

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	1929

A.8.5.4.1	Test Purpose and Environment	1929

A.8.5.4.2	Test Requirements	1931

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1931

A.8.5.5.1	Test Purpose and Environment	1931

A.8.5.5.2	Test Requirements	1934

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	1934

A.8.5.6.1	Test Purpose and Environment	1934

A.8.5.6.2	Test Requirements	1935

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	1936

A.8.5.7.1	Test Purpose and Environment	1936

A.8.5.7.2	Test Requirements	1936

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1936

A.8.5.8.1	Test Purpose and Environment	1936

A.8.5.8.2	Test Requirements	1939

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	1939

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	1939

A.8.6.1.1	Test Purpose and Environment	1939

A.8.6.1.2	Test Requirements	1941

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1941

A.8.6.2.1	Test Purpose and Environment	1941

A.8.6.2.2	Test Requirements	1944

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1944

A.8.6.3.1	Test Purpose and Environment	1944

A.8.6.3.2	Test Requirements	1947

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	1947

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	1947

A.8.7.1.1	Test Purpose and Environment	1947

A.8.7.1.1.1	Void	1947

A.8.7.1.1.2	1.28 Mcps TDD option	1947

A.8.7.1.1.3	Void	1949

A.8.7.1.2	Test Requirements	1949

A.8.7.1.2.1	Void	1949

A.8.7.1.2.2	1.28 Mcps TDD option	1949

A.8.7.1.2.3	Void	1949

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	1949

A.8.7.2.1	Test Purpose and Environment	1949

A.8.7.2.2	Test Requirements	1952

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	1953

A.8.7.3.1	Test Purpose and Environment	1953

A.8.7.3.2	Test Parameters	1953

A.8.7.3.3	Test Requirements	1954

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1955

A.8.7.4.1	Test Purpose and Environment	1955

A.8.7.4.2	Test Requirements	1957

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1957

A.8.7.5.1	Test Purpose and Environment	1957

A.8.7.5.2	Test Requirements	1959

A.8.7A	TBD	1959

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1959

A.8.7A.1.1	Test Purpose and Environment	1959

A.8.7A.1.2	Test Requirements	1962

A.8.8	E-UTRAN FDD – GSM Measurements	1962

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	1962

A.8.8.1.1	Test Purpose and Environment	1962

A.8.8.1.2	Test Requirements	1964

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	1964

A.8.8.2.1	Test Purpose and Environment	1964

A.8.8.2.2	Test Requirements	1966

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	1967

A.8.8.3.1	Test Purpose and Environment	1967

A.8.8.3.2	Test Requirements	1968

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	1969

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	1969

A.8.9.1.1	Test Purpose and Environment	1969

A.8.9.1.2	Test Requirements	1970

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1971

A.8.9.2.1	Test Purpose and Environment	1971

A.8.9.2.2	Test Requirements	1973

A.8.10	E-UTRAN TDD – GSM Measurements	1973

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	1973

A.8.10.1.1	Test Purpose and Environment	1973

A.8.10.1.2	Test Requirements	1974

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	1975

A.8.10.2.1	Test Purpose and Environment	1975

A.8.10.2.2	Test Requirements	1977

A.8.11	Monitoring of Multiple Layers	1977

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	1977

A.8.11.1.1	Test Purpose and Environment	1977

A.8.11.1.2	Test Requirements	1979

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	1980

A.8.11.2.1	Test Purpose and Environment	1980

A.8.11.2.2	Test Requirements	1981

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	1981

A.8.11.3.1	Test Purpose and Environment	1981

A.8.11.3.2	Test Requirements	1984

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	1984

A.8.11.4.1	Test Purpose and Environment	1984

A.8.11.4.2	Test Requirements	1987

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1987

A.8.11.5.1	Test Purpose and Environment	1987

A.8.11.5.2	Test Requirements	1989

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1990

A.8.11.6.1	Test Purpose and Environment	1990

A.8.11.6.2	Test Requirements	1992

A.8.12	RSTD Intra-frequency Measurements	1993

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	1993

A.8.12.1.1	Test Purpose and Environment	1993

A.8.12.1.2	Test Requirements	1996

A.8.12.1.2A	Test Requirements for UE Category 1bis	1997

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	1997

A.8.12.2.1	Test Purpose and Environment	1997

A.8.12.2.2	Test Requirements	2001

A.8.12.2.2A	Test Requirements for UE Category 1bis	2002

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2002

A.8.12.3.1	Test Purpose and Environment	2002

A.8.12.3.2	Test Requirements	2006

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2007

A.8.12.4.1	Test Purpose and Environment	2007

A.8.12.4.2	Test Requirements	2010

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2011

A.8.12.5.1	Test Purpose and Environment	2011

A.8.12.5.2	Test Requirements	2014

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2015

A.8.12.6.1	Test Purpose and Environment	2015

A.8.12.6.2	Test Requirements	2018

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2019

A.8.12.7.1	Test Purpose and Environment	2019

A.8.12.7.2	Test Requirements	2022

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2023

A.8.12.8.1	Test Purpose and Environment	2023

A.8.12.8.2	Test Requirements	2026

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2027

A.8.12.9.1	Test Purpose and Environment	2027

A.8.12.9.2	Test Requirements	2030

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2031

A.8.12.10.1	Test Purpose and Environment	2031

A.8.12.10.2	Test Requirements	2034

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2035

A.8.12.11.1	Test Purpose and Environment	2035

A.8.12.11.2	Test Requirements	2039

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2040

A.8.12.12.1	Test Purpose and Environment	2040

A.8.12.12.2	Test Requirements	2043

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2044

A.8.12.13.1	Test Purpose and Environment	2044

A.8.12.13.2	Test Requirements	2047

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2048

A.8.12.14.1	Test Purpose and Environment	2048

A.8.12.14.2	Test Requirements	2052

A.8.13	RSTD Inter-frequency Measurements	2053

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2053

A.8.13.1.1	Test Purpose and Environment	2053

A.8.13.1.2	Test Requirements	2056

A.8.13.1.2A	Test Requirements for UE Category 1bis	2057

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2057

A.8.13.2.1	Test Purpose and Environment	2057

A.8.13.2.2	Test Requirements	2061

A.8.13.2.2A	Test Requirements for UE Category 1bis	2062

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2062

A.8.13.3.1	Test Purpose and Environment	2062

A.8.13.3.2	Test Requirements	2066

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2067

A.8.13.4.1	Test Purpose and Environment	2067

A.8.13.4.2	Test Requirements	2070

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2071

A.8.13.5.1	Test Purpose and Environment	2071

A.8.13.5.2	Test Requirements	2075

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2076

A.8.13.6.1	Test Purpose and Environment	2076

A.8.13.6.2	Test Requirements	2079

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2080

A.8.13.7.1	Test Purpose and Environment	2080

A.8.13.7.2	Test Requirements	2083

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2084

A.8.13.8.1	Test Purpose and Environment	2084

A.8.13.8.2	Test Requirements	2088

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2089

A.8.13.9.1	Test Purpose and Environment	2089

A.8.13.9.2	Test Requirements	2093

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2094

A.8.13.10.1	Test Purpose and Environment	2094

A.8.13.10.2	Test Requirements	2098

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2099

A.8.13.11.1	Test Purpose and Environment	2099

A.8.13.11.2	Test Requirements	2103

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2104

A.8.13.12.1	Test Purpose and Environment	2104

A.8.13.12.2	Test Requirements	2108

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2109

A.8.13.13.1	Test Purpose and Environment	2109

A.8.13.13.2	Test Requirements	2113

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2114

A.8.13.14.1	Test Purpose and Environment	2114

A.8.13.14.2	Test Requirements	2118

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2119

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2119

A.8.14.1.1	Test Purpose and Environment	2119

A.8.14.1.2	Test Requirements	2121

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2121

A.8.14.2.1	Test Purpose and Environment	2121

A.8.14.2.2	Test Requirements	2124

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2124

A.8.14.3.1	Test Purpose and Environment	2124

A.8.14.3.2	Test Requirements	2126

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2127

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2127

A.8.15.1.1	Test Purpose and Environment	2127

A.8.15.1.2	Test Requirements	2128

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2128

A.8.15.2.1	Test Purpose and Environment	2128

A.8.15.2.2	Test Requirements	2131

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2131

A.8.15.3.1	Test Purpose and Environment	2131

A.8.15.3.2	Test Requirements	2133

A.8.16	E-UTRAN Carrier Aggregation Measurements	2134

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2134

A.8.16.1.1	Test Purpose and Environment	2134

A.8.16.1.2	Test Requirements	2136

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2136

A.8.16.2.1	Test Purpose and Environment	2136

A.8.16.2.2	Test Requirements	2138

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2138

A.8.16.3.1	Test Purpose and Environment	2138

A.8.16.3.2	Test Requirements	2140

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2141

A.8.16.3A.1	Test Purpose and Environment	2141

A.8.16.3A.2	Test Requirements	2142

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2143

A.8.16.4.1	Test Purpose and Environment	2143

A.8.16.4.2	Test Requirements	2144

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2145

A.8.16.4A.1	Test Purpose and Environment	2145

A.8.16.4A.2	Test Requirements	2147

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2148

A.8.16.5.1	Test Purpose and Environment	2148

A.8.16.5.2	Test Requirements	2148

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2148

A.8.16.6.1	Test Purpose and Environment	2148

A.8.16.6.2	Test Requirements	2149

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2149

A.8.16.7.1	 Test Purpose and Environment	2149

A.8.16.7.2	 Test Requirements	2150

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2150

A.8.16.8.1	 Test Purpose and Environment	2150

A.8.16.8.2	 Test Requirements	2151

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2151

A.8.16.9.1	Test Purpose and Environment	2151

A.8.16.9.2	Test Requirements	2152

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2152

A.8.16.10.1	Test Purpose and Environment	2152

A.8.16.10.2	Test Requirements	2153

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2153

A.8.16.11.1	 Test Purpose and Environment	2153

A.8.16.11.2	 Test Requirements	2154

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2154

A.8.16.12.1	 Test Purpose and Environment	2154

A.8.16.12.2	 Test Requirements	2154

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2154

A.8.16.13.1	Test Purpose and Environment	2154

A.8.16.13.2	Test Requirements	2155

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2155

A.8.16.14.1	Test Purpose and Environment	2155

A.8.16.14.2	Test Requirements	2156

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2156

A.8.16.15.1	 Test Purpose and Environment	2156

A.8.16.7.2	 Test Requirements	2156

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2156

A.8.16.16.1	 Test Purpose and Environment	2156

A.8.16.16.2	 Test Requirements	2157

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2157

A.8.16.17.1	Test Purpose and Environment	2157

A.8.16.17.2	Test Requirements	2159

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2160

A.8.16.17A.1	Test Purpose and Environment	2160

A.8.16.17A.2	Test Requirements	2160

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2160

A.8.16.17B.1	Test Purpose and Environment	2160

A.8.16.17B.2	Test Requirements	2161

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2161

A.8.16.17C.1	Test Purpose and Environment	2161

A.8.16.17C.2	Test Requirements	2161

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2161

A.8.16.18.1	Test Purpose and Environment	2161

A.8.16.18.2	Test Requirements	2163

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2164

A.8.16.18A.1	Test Purpose and Environment	2164

A.8.16.18A.2	Test Requirements	2164

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2164

A.8.16.18B.1	Test Purpose and Environment	2164

A.8.16.18B.2	Test Requirements	2165

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2165

A.8.16.18C.1	Test Purpose and Environment	2165

A.8.16.18C.2	Test Requirements	2165

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2165

A.8.16.18D.1	Test Purpose and Environment	2165

A.8.16.18D.2	Test Requirements	2166

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2166

A.8.16.19.1	Test Purpose and Environment	2166

A.8.16.19.2	Test Requirements	2168

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2168

A.8.16.19A.1	Test Purpose and Environment	2168

A.8.16.19A.2	Test Requirements	2169

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2169

A.8.16.19B.1	Test Purpose and Environment	2169

A.8.16.19B.2	Test Requirements	2169

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2169

A.8.16.19C.1	Test Purpose and Environment	2169

A.8.16.19C.2	Test Requirements	2170

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2170

A.8.16.20.1	Test Purpose and Environment	2170

A.8.16.20.2	Test Requirements	2172

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2173

A.8.16.20A.1	Test Purpose and Environment	2173

A.8.16.20A.2	Test Requirements	2173

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2173

A.8.16.20B.1	Test Purpose and Environment	2173

A.8.16.20B.2	Test Requirements	2174

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2174

A.8.16.20C.1	Test Purpose and Environment	2174

A.8.16.20C.2	Test Requirements	2174

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2174

A.8.16.20D.1	Test Purpose and Environment	2174

A.8.16.20D.2	Test Requirements	2175

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2175

A.8.16.21.1	Test Purpose and Environment	2175

A.8.16.21.2	Test Requirements	2177

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2177

A.8.16.22.1	Test Purpose and Environment	2177

A.8.16.22.2	Test Requirements	2179

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2179

A.8.16.23.1	Test Purpose and Environment	2179

A.8.16.23.2	Test Requirements	2182

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2182

A.8.16.24.1	Test Purpose and Environment	2182

A.8.16.24.2	Test Requirements	2185

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2185

A.8.16.25.1	Test Purpose and Environment	2185

A.8.16.25.2	Test Requirements	2187

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2187

A.8.16.26.1	Test Purpose and Environment	2187

A.8.16.26.2	Test Requirements	2190

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2190

A.8.16.27.1	Test Purpose and Environment	2190

A.8.16.27.2	Test Requirements	2194

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2194

A.8.16.28.1	Test Purpose and Environment	2194

A.8.16.28.2	Test Requirements	2198

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2198

A.8.16.29.1	Test Purpose and Environment	2198

A.8.16.29.2	Test Requirements	2202

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2202

A.8.16.30.1	Test Purpose and Environment	2202

A.8.16.30.2	Test Requirements	2206

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2206

A.8.16.31.1	Test Purpose and Environment	2206

A.8.16.31.2	Test Requirements	2210

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2210

A.8.16.32.1	Test Purpose and Environment	2210

A.8.16.32.2	Test Requirements	2214

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2214

A.8.16.33.1	Test Purpose and Environment	2214

A.8.16.33.2	Test Requirements	2218

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2218

A.8.16.34.1	Test Purpose and Environment	2218

A.8.16.34.2	Test Requirements	2222

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2222

A.8.16.35.1	Test Purpose and Environment	2222

A.8.16.35.2	Test Requirements	2224

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2225

A.8.16.36.1	Test Purpose and Environment	2225

A.8.16.36.2	Test Requirements	2227

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2228

A.8.16.37.1	Test Purpose and Environment	2228

A.8.16.37.2	Test Requirements	2230

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2231

A.8.16.38.1	Test Purpose and Environment	2231

A.8.16.38.2	Test Requirements	2233

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2234

A.8.16.39.1	Test Purpose and Environment	2234

A.8.16.39.2	Test Requirements	2237

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2237

A.8.16.40.1	Test Purpose and Environment	2237

A.8.16.40.2	Test Requirements	2240

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2240

A.8.16.41.1	Test Purpose and Environment	2240

A.8.16.41.2	Test Requirements	2242

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2243

A.8.16.42.1	Test Purpose and Environment	2243

A.8.16.42.2	Test Requirements	2245

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2246

A.8.16.43.1	Test Purpose and Environment	2246

A.8.16.43.2	Test Requirements	2249

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2249

A.8.16.44.1	Test Purpose and Environment	2249

A.8.16.44.2	Test Requirements	2252

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2252

A.8.16.45.1	Test Purpose and Environment	2252

A.8.16.45.2	Test Requirements	2255

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2255

A.8.16.46.1	Test Purpose and Environment	2255

A.8.16.46.2	Test Requirements	2257

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2258

A.8.16.47.1	Test Purpose and Environment	2258

A.8.16.47.2	Test Requirements	2260

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2261

A.8.16.48.1	Test Purpose and Environment	2261

A.8.16.48.2	Test Requirements	2263

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2264

A.8.16.49.1	Test Purpose and Environment	2264

A.8.16.49.2	Test Requirements	2267

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2267

A.8.16.50.1	Test Purpose and Environment	2267

A.8.16.50.2	Test Requirements	2269

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2270

A.8.16.51.1	Test Purpose and Environment	2270

A.8.16.51.2	Test Requirements	2274

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2274

A.8.16.52.1	Test Purpose and Environment	2274

A.8.16.52.2	Test Requirements	2278

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2278

A.8.16.53.1	Test Purpose and Environment	2278

A.8.16.53.2	Test Requirements	2282

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2282

A.8.16.54.1	Test Purpose and Environment	2282

A.8.16.54.2	Test Requirements	2286

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2286

A.8.16.55.1	Test Purpose and Environment	2286

A.8.16.55.2	Test Requirements	2291

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2291

A.8.16.56.1	Test Purpose and Environment	2291

A.8.16.56.2	Test Requirements	2296

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2296

A.8.16.57.1	Test Purpose and Environment	2296

A.8.16.57.2	Test Requirements	2298

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2299

A.8.16.58.1	Test Purpose and Environment	2299

A.8.16.58.2	Test Requirements	2301

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2302

A.8.16.59.1	Test Purpose and Environment	2302

A.8.16.59.2	Test Requirements	2305

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2306

A.8.16.60.1	Test Purpose and Environment	2306

A.8.16.60.2	Test Requirements	2309

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2310

A.8.16.61.1	Test Purpose and Environment	2310

A.8.16.61.2	Test Requirements	2313

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2313

A.8.16.62.1	Test Purpose and Environment	2313

A.8.16.62.2	Test Requirements	2315

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2316

A.8.16.63.1	Test Purpose and Environment	2316

A.8.16.63.2	Test Requirements	2319

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2320

A.8.16.64.1	Test Purpose and Environment	2320

A.8.16.64.2	Test Requirements	2324

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2324

A.8.16.65.1	Test Purpose and Environment	2324

A.8.16.65.2	Test Requirements	2329

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2329

A.8.16.66.1	Test Purpose and Environment	2329

A.8.16.66.2	Test Requirements	2334

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2334

A.8.16.67.1	Test Purpose and Environment	2334

A.8.16.67.2	Test Requirements	2339

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2339

A.8.16.68.1	Test Purpose and Environment	2339

A.8.16.68.2	Test Requirements	2344

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2344

A.8.16.69.1	Test Purpose and Environment	2344

A.8.16.69.2	Test Requirements	2347

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2347

A.8.16.70.1	Test Purpose and Environment	2347

A.8.16.70.2	Test Requirements	2350

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2350

A.8.16.71.1	Test Purpose and Environment	2350

A.8.16.71.2	Test Requirements	2354

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2354

A.8.16.72.1	Test Purpose and Environment	2354

A.8.16.72.2	Test Requirements	2358

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2358

A.8.16.73.1	Test Purpose and Environment	2358

A.8.16.73.2	Test Requirements	2363

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2363

A.8.16.74.1	Test Purpose and Environment	2363

A.8.16.74.2	Test Requirements	2368

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2368

A.8.16.75.1	Test Purpose and Environment	2368

A.8.16.75.2	Test Requirements	2373

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2373

A.8.16.76.1	Test Purpose and Environment	2373

A.8.16.76.2	Test Requirements	2378

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2378

A.8.16.77.1	Test Purpose and Environment	2378

A.8.16.77.2	Test Requirements	2381

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2381

A.8.16.78.1	Test Purpose and Environment	2381

A.8.16.78.2	Test Requirements	2384

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2385

A.8.16.79.1	Test Purpose and Environment	2385

A.8.16.79.2	Test Requirements	2390

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2390

A.8.16.80.1	Test Purpose and Environment	2390

A.8.16.80.2	Test Requirements	2395

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2395

A.8.16.81.1	Test Purpose and Environment	2395

A.8.16.81.2	Test Requirements	2400

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2400

A.8.16.82.1	Test Purpose and Environment	2400

A.8.16.82.2	Test Requirements	2405

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2405

A.8.16.83.1	Test Purpose and Environment	2405

A.8.16.83.2	Test Requirements	2410

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2410

A.8.16.84.1	Test Purpose and Environment	2410

A.8.16.84.2	Test Requirements	2416

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2416

A.8.16.85.1	Test Purpose and Environment	2416

A.8.16.85.2	Test Requirements	2419

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2419

A.8.16.86.1	Test Purpose and Environment	2419

A.8.16.86.2	Test Requirements	2422

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2422

A.8.16.87.1	Test Purpose and Environment	2422

A.8.16.87.2	Test Requirements	2427

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2427

A.8.16.88.1	Test Purpose and Environment	2427

A.8.16.88.2	Test Requirements	2435

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2435

A.8.16.89.1	Test Purpose and Environment	2435

A.8.16.89.2	Test Requirements	2440

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2440

A.8.16.90.1	Test Purpose and Environment	2440

A.8.16.90.2	Test Requirements	2445

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2445

A.8.16.91.1	Test Purpose and Environment	2445

A.8.16.91.2	Test Requirements	2451

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2451

A.8.16.92.1	Test Purpose and Environment	2451

A.8.16.92.2	Test Requirements	2459

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2459

A.8.16.93.1	Test Purpose and Environment	2459

A.8.16.93.2	Test Requirements	2464

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2464

A.8.16.94.1	Test Purpose and Environment	2464

A.8.16.94.2	Test Requirements	2469

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2469

A.8.16.95.1	Test Purpose and Environment	2469

A.8.16.95.2	Test Requirements	2477

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2477

A.8.16.96.1	Test Purpose and Environment	2477

A.8.16.96.2	Test Requirements	2484

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2484

A.8.16.97.1	Test Purpose and Environment	2484

A.8.16.97.2	Test Requirements	2489

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2489

A.8.16.98.1	Test Purpose and Environment	2489

A.8.16.98.2	Test Requirements	2495

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2495

A.8.16.99.1	Test Purpose and Environment	2495

A.8.16.99.2	Test Requirements	2502

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2502

A.8.16.100.1	Test Purpose and Environment	2502

A.8.16.100.2	Test Requirements	2510

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2510

A.8.16.101.1	Test Purpose and Environment	2510

A.8.16.101.2	Test Requirements	2517

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2517

A.8.16.102.1	Test Purpose and Environment	2517

A.8.16.102.2	Test Requirements	2525

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2525

A.8.16.103.1	Test Purpose and Environment	2525

A.8.16.103.2	Test Requirements	2529

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2530

A.8.16.104.1	Test Purpose and Environment	2530

A.8.16.104.2	Test Requirements	2533

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2534

A.8.16.105.1	Test Purpose and Environment	2534

A.8.16.105.2	Test Requirements	2538

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2539

A.8.16.106.1	Test Purpose and Environment	2539

A.8.16.106.2	Test Requirements	2542

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2542

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2542

A.8.17.1.1	Test Purpose and Environment	2542

A.8.17.1.2	Test Requirements	2548

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2548

A.8.17.2.1	Test Purpose and Environment	2548

A.8.17.2.2	Test Requirements	2554

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2554

A.8.17.3.1	Test Purpose and Environment	2554

A.8.17.3.2	Test Requirements	2555

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2555

A.8.17.4.1	Test Purpose and Environment	2555

A.8.17.4.2	Test Requirements	2556

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2556

A.8.17.5.1	Test Purpose and Environment	2556

A.8.17.5.2	Test Requirements	2557

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2557

A.8.17.6.1	Test Purpose and Environment	2557

A.8.17.6.2	Test Requirements	2558

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2559

A.8.17.7.1	Test Purpose and Environment	2559

A.8.17.7.2	Test Requirements	2559

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2560

A.8.17.8.1	Test Purpose and Environment	2560

A.8.17.8.2	Test Requirements	2560

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2561

A.8.17.9.1	Test Purpose and Environment	2561

A.8.17.9.2	Test Requirements	2562

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2562

A.8.17.10.1	Test Purpose and Environment	2562

A.8.17.10.2	Test Requirements	2567

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2568

A.8.17.11.1	Test Purpose and Environment	2568

A.8.17.11.2	Test Requirements	2574

A.8.18	E-UTRAN TDD – HRPD Measurements	2575

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2575

A.8.18.1.1	Test Purpose and Environment	2575

A.8.18.1.2	Test Requirements	2577

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2577

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2577

A.8.19.1.1	Test Purpose and Environment	2577

A.8.19.1.2	Test Requirements	2578

A.8.20	Inter-frequency/RAT Measurements in CA mode	2579

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2579

A.8.20.1.1	Test Purpose and Environment	2579

A.8.20.1.2	Test Requirements	2580

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2580

A.8.20.2.1	Test Purpose and Environment	2581

A.8.20.2.2	Test Requirements	2582

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2583

A.8.20.2A.1	Test Purpose and Environment	2583

A.8.20.2A.2	Test Requirements	2583

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2583

A.8.20.2B.1	Test Purpose and Environment	2583

A.8.20.2B.2	Test Requirements	2586

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2586

A.8.20.3.1	Test Purpose and Environment	2586

A.8.20.3.2	Test Requirements	2588

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2588

A.8.20.4.1	Test Purpose and Environment	2588

A.8.20.4.1.1	1.28 Mcps TDD option	2588

A.8.20.4.2	Test Requirements	2590

A.8.20.4.2.1	1.28 Mcps TDD option	2590

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2590

A.8.20.4A.1	Test Purpose and Environment	2590

A.8.20.4A.1.1	1.28 Mcps TDD option	2590

A.8.20.4A.2	Test Requirements	2590

A.8.20.4A.2.1	1.28 Mcps TDD option	2590

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2591

A.8.20.4B.1	Test Purpose and Environment	2591

A.8.20.4B.1.1	1.28 Mcps TDD option	2591

A.8.20.4B.2	Test Requirements	2593

A.8.20.4B.2.1	1.28 Mcps TDD option	2593

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2593

A.8.21.1	Test Purpose and Environment	2593

A.8.21.2 Test Requirements	2596

A.8.22	E-UTRAN Discovery Signal Measurements	2596

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2596

A.8.22.1.1	Test Purpose and Environment	2596

A.8.22.1.2	Test Requirements	2599

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2599

A.8.22.2.1	Test Purpose and Environment	2599

A.8.22.2.2	Test Requirements	2602

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2602

A.8.22.3.1	Test Purpose and Environment	2602

A.8.22.3.2	Test Requirements	2605

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2605

A.8.22.4.1	Test Purpose and Environment	2605

A.8.22.4.2	Test Requirements	2608

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2608

A.8.22.5.1	Test Purpose and Environment	2608

A.8.22.5.2	Test Requirements	2611

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2611

A.8.22.6.1	Test Purpose and Environment	2611

A.8.22.6.2	Test Requirements	2614

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2614

A.8.22.7.1	Test Purpose and Environment	2614

A.8.22.7.2	Test Requirements	2617

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2617

A.8.22.8.1	Test Purpose and Environment	2617

A.8.22.8.2	Test Requirements	2620

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2620

A.8.22.9.1	Test Purpose and Environment	2620

A.8.22.9.2	Test Requirements	2622

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2623

A.8.22.10.1	Test Purpose and Environment	2623

A.8.22.10.2	Test Requirements	2625

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2626

A.8.22.11.1	Test Purpose and Environment	2626

A.8.22.11.2	Test Requirements	2628

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2628

A.8.22.12.1	Test Purpose and Environment	2628

A.8.22.12.2	Test Requirements	2631

A.8.23	E-UTRAN Dual Connectivity Measurements	2631

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2631

A.8.23.1.1	Test Purpose and Environment	2631

A.8.23.1.2	Test Requirements	2634

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2634

A.8.23.2.1	Test Purpose and Environment	2634

A.8.23.2.2	Test Requirements	2637

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2637

A.8.23.3.1	Test Purpose and Environment	2637

A.8.23.3.2	Test Requirements	2640

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2640

A.8.23.4.1	Test Purpose and Environment	2640

A.8.23.4.2	Test Requirements	2643

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2643

A.8.23.5.1	Test Purpose and Environment	2643

A.8.23.5.2	Test Requirements	2646

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2646

A.8.23.6.1	Test Purpose and Environment	2646

A.8.23.6.2	Test Requirements	2649

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2649

A.8.23.7.1	Test Purpose and Environment	2649

A.8.23.7.2	Test Requirements	2651

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2652

A.8.23.8.1	Test Purpose and Environment	2652

A.8.23.8.2	Test Requirements	2654

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2655

A.8.23.9.1	Test Purpose and Environment	2655

A.8.23.9.2	Test Requirements	2658

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2658

A.8.23.10.1	Test Purpose and Environment	2658

A.8.23.10.2	Test Requirements	2661

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2661

A.8.23.11.1	Test Purpose and Environment	2661

A.8.23.11.2	Test Requirements	2664

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2664

A.8.23.12.1	Test Purpose and Environment	2664

A.8.23.12.2	Test Requirements	2667

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2667

A.8.23.13.1	Test Purpose and Environment	2667

A.8.23.13.2	Test Requirements	2670

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2670

A.8.23.14.1	Test Purpose and Environment	2670

A.8.23.14.2	Test Requirements	2673

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2673

A.8.23.15.1	Test Purpose and Environment	2673

A.8.23.15.2	Test Requirements	2676

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2676

A.8.23.16.1	Test Purpose and Environment	2676

A.8.23.16.2	Test Requirements	2677

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2678

A.8.23.17.1	Test Purpose and Environment	2678

A.8.23.17.2	Test Requirements	2680

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2680

A.8.23.18.1	Test Purpose and Environment	2680

A.8.23.18.2	Test Requirements	2681

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2682

A.8.23.19.1	Test Purpose and Environment	2682

A.8.23.19.2	Test Requirements	2683

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2684

A.8.23.20.1	Test Purpose and Environment	2684

A.8.23.20.2	Test Requirements	2685

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2686

A.8.23.21.1	Test Purpose and Environment	2686

A.8.23.21.2	Test Requirements	2687

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2688

A.8.23.22.1	Test Purpose and Environment	2688

A.8.23.22.2	Test Requirements	2689

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2690

A.8.23.23.1	Test Purpose and Environment	2690

A.8.23.23.2	Test Requirements	2691

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2692

A.8.23.24.1	Test Purpose and Environment	2692

A.8.23.24.2	Test Requirements	2694

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2695

A.8.23.25.1	Test Purpose and Environment	2695

A.8.23.25.2	Test Requirements	2697

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2698

A.8.23.26.1	Test Purpose and Environment	2698

A.8.23.26.2	Test Requirements	2700

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2701

A.8.23.27.1	Test Purpose and Environment	2701

A.8.23.27.2	Test Requirements	2704

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	2704

A.8.23.28.1	Test Purpose and Environment	2704

A.8.23.28.2	Test Requirements	2707

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2707

A.8.23.29.1	Test Purpose and Environment	2707

A.8.23.29.2	Test Requirements	2710

A.8.24	Proximity-based Services	2710

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2710

A.8.24.1.1	Test Purpose and Environment	2710

A.8.24.1.2	Test Requirements	2711

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2712

A.8.24.2.1	Test Purpose and Environment	2712

A.8.24.2.2	Test Requirements	2713

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2713

A.8.24.3.1	Test Purpose and Environment	2714

A.8.24.3.2	Test Requirements	2715

A.8.25	E-UTRAN-WLAN Measurements	2716

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2716

A.8.25.1.1	Test Purpose and Environment	2716

A.8.25.1.2	Test Requirements	2718

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2718

A.8.25.2.1	Test Purpose and Environment	2718

A.8.25.2.2	Test Requirements	2720

A.8.26 Frame Structure 3 (FS3)	2720

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	2720

A.8.26.1.1	Test Purpose and Environment	2720

A.8.26.1.2	Test Requirements	2722

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	2723

A.8.26.2.1	Test Purpose and Environment	2723

A.8.26.2.2	Test Requirements	2725

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2726

A.8.26.3.1	Test Purpose and Environment	2726

A.8.26.3.2	Test Requirements	2729

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2729

A.8.26.3A.1	Test Purpose and Environment	2729

A.8.26.3A.2	Test Requirements	2732

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	2732

A.8.26.4.1	Test Purpose and Environment	2732

A.8.26.4.2	Test Requirements	2735

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2735

A.8.26.4A.1	Test Purpose and Environment	2735

A.8.26.4A.2	Test Requirements	2738

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2738

A.8.26.5.1	Test Purpose and Environment	2738

A.8.26.5.2	Test Requirements	2741

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2741

A.8.26.5A.1	Test Purpose and Environment	2741

A.8.26.5A.2	Test Requirements	2744

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2744

A.8.26.6.1	Test Purpose and Environment	2744

A.8.26.6.2	Test Requirements	2747

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2747

A.8.26.6A.1	Test Purpose and Environment	2747

A.8.26.6A.2	Test Requirements	2750

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2750

A.8.26.7.1	Test Purpose and Environment	2750

A.8.26.7.2	Test Requirements	2753

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2753

A.8.26.8.1	Test Purpose and Environment	2753

A.8.26.8.2	Test Requirements	2756

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2756

A.8.26.9.1	Test Purpose and Environment	2756

A.8.26.9.2	Test Requirements	2759

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2759

A.8.26.10.1	Test Purpose and Environment	2759

A.8.26.10.2	Test Requirements	2761

A.9	Measurement Performance Requirements	2762

A.9.1	RSRP	2762

A.9.1.1	FDD Intra frequency case	2762

A.9.1.1.1	Test Purpose and Environment	2762

A.9.1.1.2	Test parameters	2762

A.9.1.1.3	Test Requirements	2765

A.9.1.2	TDD Intra frequency case	2765

A.9.1.2.1	Test Purpose and Environment	2765

A.9.1.2.2	Test parameters	2765

A.9.1.2.3	Test Requirements	2766

A.9.1.3	FDD—FDD Inter frequency case	2767

A.9.1.3.1	Test Purpose and Environment	2767

A.9.1.3.2	Test parameters	2767

A.9.1.3.3	Test Requirements	2770

A.9.1.4	TDD—TDD Inter frequency case	2770

A.9.1.4.1	Test Purpose and Environment	2770

A.9.1.4.2	Test parameters	2770

A.9.1.4.3	Test Requirements	2772

A.9.1.5	FDD—TDD Inter frequency case	2773

A.9.1.5.1	Test Purpose and Environment	2773

A.9.1.5.2	Test parameters	2773

A.9.1.5.3	Test Requirements	2774

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2775

A.9.1.6.1	Test Purpose and Environment	2775

A.9.1.6.2	Test parameters	2775

A.9.1.6.3	Test Requirements	2778

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2778

A.9.1.7.1	Test Purpose and Environment	2778

A.9.1.7.2	Test parameters	2778

A.9.1.7.3	Test Requirements	2781

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2781

A.9.1.8.1	Test Purpose and Environment	2781

A.9.1.8.2	Test parameters	2781

A.9.1.8.3	Test Requirements	2785

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2785

A.9.1.9.1	Test Purpose and Environment	2785

A.9.1.9.2	Test parameters	2785

A.9.1.9.3	Test Requirements	2787

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2788

A.9.1.10.1	Test Purpose and Environment	2788

A.9.1.10.2	Test parameters	2788

A.9.1.10.3	Test Requirements	2791

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2791

A.9.1.11.1	Test Purpose and Environment	2791

A.9.1.11.2	Test parameters	2791

A.9.1.11.3	Test Requirements	2795

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2795

A.9.1.12.1	Test Purpose and Environment	2795

A.9.1.12.2	Test parameters	2795

A.9.1.12.3	Test Requirements	2796

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2796

A.9.1.13.1	Test Purpose and Environment	2796

A.9.1.13.2	Test parameters	2796

A.9.1.13.3	Test Requirements	2797

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2797

A.9.1.14.1	Test Purpose and Environment	2797

A.9.1.14.2	Test parameters	2797

A.9.1.14.3	Test Requirements	2801

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2801

A.9.1.15.1	Test Purpose and Environment	2801

A.9.1.15.2	Test parameters	2801

A.9.1.15.3	Test Requirements	2803

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2804

A.9.1.16.1	Test Purpose and Environment	2804

A.9.1.16.2	Test parameters	2804

A.9.1.16.3	Test Requirements	2805

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2805

A.9.1.17.1	Test Purpose and Environment	2805

A.9.1.17.2	Test parameters	2806

A.9.1.17.3	Test Requirements	2807

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2807

A.9.1.18.1	Test Purpose and Environment	2807

A.9.1.18.2	Test parameters	2807

A.9.1.18.3	Test Requirements	2808

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2809

A.9.1.19.1	Test Purpose and Environment	2809

A.9.1.19.2	Test parameters	2809

A.9.1.19.3	Test Requirements	2809

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2809

A.9.1.20.1	Test Purpose and Environment	2809

A.9.1.20.2	Test parameters	2809

A.9.1.20.3	Test Requirements	2811

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2811

A.9.1.21.1	Test Purpose and Environment	2811

A.9.1.21.2	Test parameters	2811

A.9.1.21.3	Test Requirements	2811

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2811

A.9.1.22.1	Test Purpose and Environment	2811

A.9.1.22.2	Test parameters	2812

A.9.1.22.3	Test Requirements	2816

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2816

A.9.1.23.1	Test Purpose and Environment	2816

A.9.1.23.2	Test parameters	2816

A.9.1.23.3	Test Requirements	2820

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	2820

A.9.1.24.1	Test Purpose and Environment	2820

A.9.1.24.2	Test parameters	2820

A.9.1.24.3	Test Requirements	2821

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2821

A.9.1.25.1	Test Purpose and Environment	2821

A.9.1.25.2	Test parameters	2821

A.9.1.25.3	Test Requirements	2824

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2824

A.9.1.26.1	Test Purpose and Environment	2824

A.9.1.26.2	Test parameters	2824

A.9.1.26.3	Test Requirements	2827

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2827

A.9.1.27.1	Test Purpose and Environment	2827

A.9.1.27.2	Test parameters	2827

A.9.1.27.3	Test Requirements	2830

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	2830

A.9.1.28.1	Test Purpose and Environment	2830

A.9.1.28.2	Test parameters	2830

A.9.1.28.3	Test Requirements	2833

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2833

A.9.1.29.1	Test Purpose and Environment	2833

A.9.1.29.2	Test parameters	2833

A.9.1.29.3	Test Requirements	2836

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2836

A.9.1.30.1	Test Purpose and Environment	2836

A.9.1.30.2	Test parameters	2837

A.9.1.30.3	Test Requirements	2840

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2840

A.9.1.31.1	Test Purpose and Environment	2840

A.9.1.31.2	Test parameters	2840

A.9.1.31.3	Test Requirements	2843

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	2843

A.9.1.32.1	Test Purpose and Environment	2843

A.9.1.32.2	Test parameters	2844

A.9.1.32.3	Test Requirements	2847

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2847

A.9.1.33.1	Test Purpose and Environment	2847

A.9.1.33.2	Test parameters	2847

A.9.1.33.3	Test Requirements	2850

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2850

A.9.1.34.1	Test Purpose and Environment	2850

A.9.1.34.2	Test parameters	2850

A.9.1.34.3	Test Requirements	2853

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2853

A.9.1.35.1	Test Purpose and Environment	2853

A.9.1.35.2	Test parameters	2853

A.9.1.35.3	Test Requirements	2856

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2857

A.9.1.36.1	Test Purpose and Environment	2857

A.9.1.36.2	Test parameters	2857

A.9.1.36.3	Test Requirements	2860

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2860

A.9.1.37.1	Test Purpose and Environment	2860

A.9.1.37.2	Test parameters	2860

A.9.1.37.3	Test Requirements	2863

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2864

A.9.1.38.1	Test Purpose and Environment	2864

A.9.1.38.2	Test parameters	2864

A.9.1.38.3	Test Requirements	2867

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2868

A.9.1.39.1	Test Purpose and Environment	2868

A.9.1.39.2	Test parameters	2868

A.9.1.39.3	Test Requirements	2874

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2874

A.9.1.40.1	Test Purpose and Environment	2874

A.9.1.40.2	Test parameters	2875

A.9.1.40.3	Test Requirements	2881

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	2881

A.9.1.41.1	Test Purpose and Environment	2881

A.9.1.41.2	Test parameters	2881

A.9.1.41.3	Test Requirements	2884

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	2884

A.9.1.42.1	Test Purpose and Environment	2884

A.9.1.42.2	Test parameters	2884

A.9.1.42.3	Test Requirements	2887

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	2887

A.9.1.43.1	Test Purpose and Environment	2887

A.9.1.43.2	Test parameters	2887

A.9.1.43.3	Test Requirements	2888

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2889

A.9.1.44.1	Test Purpose and Environment	2889

A.9.1.44.2	Test parameters	2889

A.9.1.44.3	Test Requirements	2894

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2894

A.9.1.45.1	Test Purpose and Environment	2894

A.9.1.45.2	Test parameters	2894

A.9.1.45.3	Test Requirements	2416

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2416

A.9.1.46.1	Test Purpose and Environment	2416

A.9.1.46.2	Test parameters	2416

A.9.1.46.3	Test Requirements	2425

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2426

A.9.1.47.1	Test Purpose and Environment	2426

A.9.1.47.2	Test parameters	2426

A.9.1.47.3	Test Requirements	2435

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2435

A.9.1.48.1	Test Purpose and Environment	2435

A.9.1.48.2	Test parameters	2436

A.9.1.48.3	Test Requirements	2446

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2447

A.9.1.49.1	Test Purpose and Environment	2447

A.9.1.49.2	Test parameters	2447

A.9.1.49.3	Test Requirements	2457

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2458

A.9.1.50.1	Test Purpose and Environment	2458

A.9.1.50.2	Test parameters	2458

A.9.1.50.3	Test Requirements	2465

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2466

A.9.1.51.1	Test Purpose and Environment	2466

A.9.1.51.2	Test parameters	2466

A.9.1.51.3	Test Requirements	2472

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2473

A.9.1.52.1	Test Purpose and Environment	2473

A.9.1.52.2	Test parameters	2473

A.9.1.52.3	Test Requirements	2476

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2476

A.9.1.52A.1	Test Purpose and Environment	2476

A.9.1.52A.2	Test parameters	2476

A.9.1.52A.3	Test Requirements	2477

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2477

A.9.1.53.1	Test Purpose and Environment	2477

A.9.1.53.2	Test parameters	2478

A.9.1.53.3	Test Requirements	2481

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2481

A.9.1.53A.1	Test Purpose and Environment	2481

A.9.1.53A.2	Test parameters	2481

A.9.1.53A.3	Test Requirements	2482

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2482

A.9.1.54.1	Test Purpose and Environment	2482

A.9.1.54.2	Test parameters	2483

A.9.1.54.3	Test Requirements	2485

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2485

A.9.1.55.1	Test Purpose and Environment	2485

A.9.1.55.2	Test parameters	2485

A.9.1.55.3	Test Requirements	2489

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2489

A.9.1.56.1	Test Purpose and Environment	2489

A.9.1.56.2	Test parameters	2489

A.9.1.56.3	Test Requirements	2492

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2493

A.9.1.57.1	Test Purpose and Environment	2493

A.9.1.57.2	Test parameters	2493

A.9.1.57.3	Test Requirements	2496

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2496

A.9.1.57A.1	Test Purpose and Environment	2496

A.9.1.57A.2	Test parameters	2496

A.9.1.57A.3	Test Requirements	2497

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2497

A.9.1.58.1	Test Purpose and Environment	2497

A.9.1.58.2	Test parameters	2498

A.9.1.58.3	Test Requirements	2501

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2501

A.9.1.58A.1	Test Purpose and Environment	2501

A.9.1.58A.2	Test parameters	2501

A.9.1.58A.3	Test Requirements	2502

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2502

A.9.1.59.1	Test Purpose and Environment	2502

A.9.1.59.2	Test parameters	2503

A.9.1.59.3	Test Requirements	2505

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2505

A.9.1.60.1	Test Purpose and Environment	2505

A.9.1.60.2	Test parameters	2505

A.9.1.60.3	Test Requirements	2508

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2509

A.9.1.61.1	Test Purpose and Environment	2509

A.9.1.61.2	Test parameters	2509

A.9.1.61.3	Test Requirements	2512

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2512

A.9.1.62.1	Test Purpose and Environment	2512

A.9.1.62.2	Test parameters	2512

A.9.1.62.3	Test Requirements	2515

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2515

A.9.1.53.1	Test Purpose and Environment	2515

A.9.1.53.2	Test parameters	2515

A.9.1.63.3	Test Requirements	2518

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2518

A.9.1.64.1	Test Purpose and Environment	2518

A.9.1.64.2	Test parameters	2518

A.9.1.64.3	Test Requirements	2519

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2520

A.9.1.65.1	Test Purpose and Environment	2520

A.9.1.65.2	Test parameters	2520

A.9.1.65.3	Test Requirements	2526

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2526

A.9.1.66.1	Test Purpose and Environment	2526

A.9.1.66.2	Test parameters	2526

A.9.1.66.3	Test Requirements	2532

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2532

A.9.1.67.1	Test Purpose and Environment	2532

A.9.1.67.2	Test parameters	2532

A.9.1.67.3	Test Requirements	2533

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2534

A.9.1.68.1	Test Purpose and Environment	2534

A.9.1.68.2	Test parameters	2534

A.9.1.68.3	Test Requirements	2538

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2538

A.9.1.69.1	Test Purpose and Environment	2538

A.9.1.69.2	Test parameters	2538

A.9.1.69.3	Test Requirements	2542

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2543

A.9.1.70.1	Test Purpose and Environment	2543

A.9.1.70.2	Test parameters	2543

A.9.1.70.3	Test Requirements	2555

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2556

A.9.1.71.1	Test Purpose and Environment	2556

A.9.1.71.2	Test parameters	2556

A.9.1.71.3	Test Requirements	2568

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2569

A.9.1.72.1	Test Purpose and Environment	2569

A.9.1.72.2	Test parameters	2569

A.9.1.72.3	Test Requirements	2581

A.9.2	RSRQ	2582

A.9.2.1	FDD Intra frequency case	2582

A.9.2.1.1	Test Purpose and Environment	2582

A.9.2.1.2	Test parameters	2582

A.9.2.1.3	Test Requirements	2585

A.9.2.2	TDD Intra frequency case	2585

A.9.2.2.1	Test Purpose and Environment	2585

A.9.2.2.2	Test parameters	2585

A.9.2.2.3	Test Requirements	2587

A.9.2.3	FDD—FDD Inter frequency case	2587

A.9.2.3.1	Test Purpose and Environment	2587

A.9.2.3.2	Test parameters	2587

A.9.2.3.3	Test Requirements	2590

A.9.2.4	TDD—TDD Inter frequency case	2590

A.9.2.4.1	Test Purpose and Environment	2590

A.9.2.4.2	Test parameters	2590

A.9.2.4.3	Test Requirements	2592

A.9.2.4A	FDD—TDD Inter frequency case	2593

A.9.2.4A.1	Test Purpose and Environment	2593

A.9.2.4A.2	Test parameters	2593

A.9.2.4A.3	Test Requirements	2595

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2595

A.9.2.5.1	Test Purpose and Environment	2595

A.9.2.5.2	Test parameters	2596

A.9.2.5.3	Test Requirements	2599

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2599

A.9.2.6.1	Test Purpose and Environment	2599

A.9.2.6.2	Test parameters	2599

A.9.2.6.3	Test Requirements	2602

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2602

A.9.2.7.1	Test Purpose and Environment	2602

A.9.2.7.2	Test parameters	2602

A.9.2.7.3	Test Requirements	2606

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2606

A.9.2.8.1	Test Purpose and Environment	2606

A.9.2.8.2	Test parameters	2606

A.9.2.8.3	Test Requirements	2609

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2609

A.9.2.9.1	Test Purpose and Environment	2609

A.9.2.9.2	Test parameters	2609

A.9.2.9.3	Test Requirements	2613

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2613

A.9.2.10.1	Test Purpose and Environment	2613

A.9.2.10.2	Test parameters	2613

A.9.2.10.3	Test Requirements	2617

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2617

A.9.2.11.1	Test Purpose and Environment	2617

A.9.2.11.2	Test parameters	2617

A.9.2.11.3	Test Requirements	2618

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2618

A.9.2.12.1	Test Purpose and Environment	2618

A.9.2.12.2	Test parameters	2619

A.9.2.12.3	Test Requirements	2619

A.9.2.13	Void	2620

A.9.2.13.1	Void	2620

A.9.2.13.2	Void	2620

A.9.2.13.3	Void	2620

A.9.2.14	Void	2620

A.9.2.14.1	Void	2620

A.9.2.14.2	Void	2620

A.9.2.14.3	Void	2620

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2620

A.9.2.15.1	Test Purpose and Environment	2620

A.9.2.15.2	Test parameters	2620

A.9.2.15.3	Test Requirements	2624

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2624

A.9.2.16.1	Test Purpose and Environment	2624

A.9.2.16.2	Test parameters	2624

A.9.2.16.3	Test Requirements	2629

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2629

A.9.2.17.1	Test Purpose and Environment	2629

A.9.2.17.2	Test parameters	2629

A.9.2.17.3	Test Requirements	2631

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2631

A.9.2.18.1	Test Purpose and Environment	2631

A.9.2.18.2	Test parameters	2631

A.9.2.18.3	Test Requirements	2634

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2634

A.9.2.19.1	Test Purpose and Environment	2634

A.9.2.19.2	Test parameters	2634

A.9.2.19.3	Test Requirements	2636

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2636

A.9.2.20.1	Test Purpose and Environment	2636

A.9.2.20.2	Test parameters	2636

A.9.2.20.3	Test Requirements	2639

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2639

A.9.2.21.1	Test Purpose and Environment	2639

A.9.2.21.2	Test parameters	2639

A.9.2.21.3	Test Requirements	2642

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2642

A.9.2.22.1	Test Purpose and Environment	2643

A.9.2.22.2	Test parameters	2643

A.9.2.22.3	Test Requirements	2643

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2643

A.9.2.23.1	Test Purpose and Environment	2643

A.9.2.23.2	Test parameters	2643

A.9.2.23.3	Test Requirements	2646

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2646

A.9.2.24.1	Test Purpose and Environment	2646

A.9.2.24.2	Test parameters	2646

A.9.2.24.3	Test Requirements	2647

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2647

A.9.2.25.1	Test Purpose and Environment	2647

A.9.2.25.2	Test parameters	2647

A.9.2.25.3	Test Requirements	2652

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2652

A.9.2.26.1	Test Purpose and Environment	2652

A.9.2.26.2	Test parameters	2652

A.9.2.26.3	Test Requirements	2656

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2656

A.9.2.27.1	Test Purpose and Environment	2656

A.9.2.27.2	Test parameters	2656

A.9.2.27.3	Test Requirements	2657

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2657

A.9.2.28.1	Test Purpose and Environment	2657

A.9.2.28.2	Test parameters	2657

A.9.2.28.3	Test Requirements	2660

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2660

A.9.2.29.1	Test Purpose and Environment	2660

A.9.2.29.2	Test parameters	2660

A.9.2.29.3	Test Requirements	2663

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2663

A.9.2.30.1	Test Purpose and Environment	2663

A.9.2.30.2	Test parameters	2663

A.9.2.30.3	Test Requirements	2666

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2666

A.9.2.31.1	Test Purpose and Environment	2666

A.9.2.31.2	Test parameters	2666

A.9.2.31.3	Test Requirements	2669

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2669

A.9.2.32.1	Test Purpose and Environment	2669

A.9.2.32.2	Test parameters	2669

A.9.2.32.3	Test Requirements	2672

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2672

A.9.2.33.1	Test Purpose and Environment	2672

A.9.2.33.2	Test parameters	2672

A.9.2.33.3	Test Requirements	2676

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2676

A.9.2.34.1	Test Purpose and Environment	2676

A.9.2.34.2	Test parameters	2676

A.9.2.34.3	Test Requirements	2679

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2679

A.9.2.35.1	Test Purpose and Environment	2679

A.9.2.35.2	Test parameters	2679

A.9.2.35.3	Test Requirements	2682

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2682

A.9.2.36.1	Test Purpose and Environment	2682

A.9.2.36.2	Test parameters	2682

A.9.2.36.3	Test Requirements	2683

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2684

A.9.2.37.1	Test Purpose and Environment	2684

A.9.2.37.2	Test parameters	2684

A.9.2.37.3	Test Requirements	2685

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2686

A.9.2.38.1	Test Purpose and Environment	2686

A.9.2.38.2	Test parameters	2686

A.9.2.38.3	Test Requirements	2690

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2690

A.9.2.39.1	Test Purpose and Environment	2690

A.9.2.39.2	Test parameters	2690

A.9.2.39.3	Test Requirements	2694

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2694

A.9.2.40.1	Test Purpose and Environment	2694

A.9.2.40.2	Test parameters	2694

A.9.2.40.3	Test Requirements	2697

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2698

A.9.2.41.1	Test Purpose and Environment	2698

A.9.2.41.2	Test parameters	2698

A.9.2.41.3	Test Requirements	2701

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2701

A.9.2.42.1	Test Purpose and Environment	2701

A.9.2.42.2	Test parameters	2701

A.9.2.42.3	Test Requirements	2704

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2704

A.9.2.43.1	Test Purpose and Environment	2704

A.9.2.43.2	Test parameters	2704

A.9.2.43.3	Test Requirements	2707

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2707

A.9.2.44.1	Test Purpose and Environment	2707

A.9.2.44.2	Test parameters	2707

A.9.2.44.3	Test Requirements	2709

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2709

A.9.2.45.1	Test Purpose and Environment	2709

A.9.2.45.2	Test parameters	2709

A.9.2.45.3	Test Requirements	2713

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2714

A.9.2.46.1	Test Purpose and Environment	2714

A.9.2.46.2	Test parameters	2714

A.9.2.46.3	Test Requirements	2718

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2719

A.9.2.47.1	Test Purpose and Environment	2719

A.9.2.47.2	Test parameters	2719

A.9.2.47.3	Test Requirements	2723

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2723

A.9.2.48.1	Test Purpose and Environment	2723

A.9.2.48.2	Test parameters	2724

A.9.2.48.3	Test Requirements	2728

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2729

A.9.2.49.1	Test Purpose and Environment	2729

A.9.2.49.2	Test parameters	2729

A.9.2.49.3	Test Requirements	2735

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2736

A.9.2.50.1	Test Purpose and Environment	2736

A.9.2.50.2	Test parameters	2736

A.9.2.50.3	Test Requirements	2742

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2742

A.9.2.51.1	Test Purpose and Environment	2742

A.9.2.51.2	Test parameters	2743

A.9.2.51.3	Test Requirements	2747

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2747

A.9.2.52.1	Test Purpose and Environment	2747

A.9.2.52.2	Test parameters	2747

A.9.2.52.3	Test Requirements	2750

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2751

A.9.2.53.1	Test Purpose and Environment	2751

A.9.2.53.2	Test parameters	2751

A.9.2.53.3	Test Requirements	2757

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2758

A.9.2.54.1	Test Purpose and Environment	2758

A.9.2.54.2	Test parameters	2758

A.9.2.54.3	Test Requirements	2764

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2764

A.9.2.55.1	Test Purpose and Environment	2764

A.9.2.55.2	Test parameters	2765

A.9.2.55.3	Test Requirements	2770

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2770

A.9.2.56.1	Test Purpose and Environment	2770

A.9.2.56.2	Test parameters	2770

A.9.2.56.3	Test Requirements	2775

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2775

A.9.2.57.1	Test Purpose and Environment	2775

A.9.2.57.2	Test parameters	2776

A.9.2.57.3	Test Requirements	2780

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2781

A.9.2.58.1	Test Purpose and Environment	2781

A.9.2.58.2	Test parameters	2781

A.9.2.58.3	Test Requirements	2785

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2786

A.9.2.59.1	Test Purpose and Environment	2786

A.9.2.59.2	Test parameters	2786

A.9.2.59.3	Test Requirements	2791

A.9.3	UTRAN FDD CPICH RSCP	2792

A.9.3.1	E-UTRAN FDD	2792

A.9.3.1.1	Test Purpose and Environment	2792

A.9.3.1.2	Parameters	2792

A.9.3.1.3	Test Requirements	2794

A.9.3.2 E-UTRAN TDD	2794

A.9.3.2.1	Test Purpose and Environment	2794

A.9.3.2.2	Parameters	2794

A.9.3.2.3	Test Requirements	2797

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2797

A.9.3.3.1	Test Purpose and Environment	2797

A.9.3.3.2	Parameters	2797

A.9.3.3.3	Test Requirements	2798

A.9.4	UTRAN FDD CPICH Ec/No	2799

A.9.4.1	E-UTRAN FDD	2799

A.9.4.1.1	Test Purpose and Environment	2799

A.9.4.1.2	Parameters	2799

A.9.4.1.3	Test Requirements	2801

A.9.4.2	E-UTRAN TDD	2802

A.9.4.2.1	Test Purpose and Environment	2802

A.9.4.2.2	Parameters	2802

A.9.4.2.3	Test Requirements	2804

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2805

A.9.4.3.1	Test Purpose and Environment	2805

A.9.4.3.2	Parameters	2805

A.9.4.3.3	Test Requirements	2806

A.9.5	UTRAN TDD measurement	2806

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2806

A.9.5.1.1	Test Purpose and Environment	2806

A.9.5.1.2	Test parameters	2806

A.9.5.1.3	Test Requirements	2808

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2808

A.9.5.2.1	Test Purpose and Environment	2808

A.9.5.2.2	Test parameters	2808

A.9.5.2.3	Test Requirements	2810

A.9.6	GSM Carrier RSSI	2811

A.9.6.1	E-UTRAN FDD	2811

A.9.6.1.1	Test Purpose and Environment	2811

A.9.6.1.2	Test Requirements	2812

A.9.6.2	E-UTRAN TDD	2812

A.9.6.2.1	Test Purpose and Environment	2812

A.9.6.2.2	Test Requirements	2814

A.9.7	UE Rx – Tx Time Difference	2815

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2815

A.9.7.1.1	Test Purpose and Environment	2815

A.9.7.1.2	Test parameters	2815

A.9.7.1.3	Test Requirements	2816

A.9.7.2	E-UTRA TDD	2816

A.9.7.2.1	Test Purpose and Environment	2816

A.9.7.2.2	Test parameters	2816

A.9.7.2.3	Test Requirements	2818

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2818

A.9.7.3.1	Test Purpose and Environment	2818

A.9.7.3.2	Test parameters	2818

A.9.7.3.3	Test Requirements	2821

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2821

A.9.7.4.1	Test Purpose and Environment	2821

A.9.7.4.2	Test Parameters	2821

A.9.7.4.3	Test Requirements	2824

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2824

A.9.7.5.1	Test Purpose and Environment	2824

A.9.7.5.2	Test parameters	2824

A.9.7.5.3	Test Requirements	2827

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2827

A.9.7.6.1	Test Purpose and Environment	2827

A.9.7.6.2	Test Parameters	2827

A.9.7.6.3	Test Requirements	2830

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2830

A.9.7.7.1	Test Purpose and Environment	2830

A.9.7.7.2	Test parameters	2830

A.9.7.7.3	Test Requirements	2831

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2832

A.9.7.8.1	Test Purpose and Environment	2832

A.9.7.8.2	Test parameters	2832

A.9.7.8.3	Test Requirements	2833

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2833

A.9.7.9.1	Test Purpose and Environment	2833

A.9.7.9.2	Test parameters	2833

A.9.7.9.3	Test Requirements	2834

A.9.8	RSTD	2835

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2835

A.9.8.1.1	Test Purpose and Environment	2835

A.9.8.1.2	Test Requirements	2837

A.9.8.1.2A	Test Requirements for UE Category 1bis	2837

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2838

A.9.8.2.1	Test Purpose and Environment	2838

A.9.8.2.2	Test Requirements	2841

A.9.8.2.2A	Test Requirements for UE Category 1bis	2841

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2842

A.9.8.3.1	Test Purpose and Environment	2842

A.9.8.3.2	Test Requirements	2844

A.9.8.3.2A	Test Requirements for UE Category 1bis	2844

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2845

A.9.8.4.1	Test Purpose and Environment	2845

A.9.8.4.2	Test Requirements	2847

A.9.8.4.2A	Test Requirements for UE Category 1bis	2847

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2848

A.9.8.5.1	Test Purpose and Environment	2848

A.9.8.5.2	Test Requirements	2850

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2850

A.9.8.6.1	Test Purpose and Environment	2850

A.9.8.6.2	Test Requirements	2853

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2853

A.9.8.7.1	Test Purpose and Environment	2853

A.9.8.7.2	Test Requirements	2854

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2854

A.9.8.8.1	Test Purpose and Environment	2854

A.9.8.8.2	Test Requirements	2855

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2855

A.9.8.9.1	Test Purpose and Environment	2855

A.9.8.9.2	Test Requirements	2856

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2856

A.9.8.10.1	Test Purpose and Environment	2856

A.9.8.10.2	Test Requirements	2857

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2857

A.9.8.11.1	Test Purpose and Environment	2857

A.9.8.11.2	Test Requirements	2858

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2858

A.9.8.12.1	Test Purpose and Environment	2858

A.9.8.12.2	Test Requirements	2859

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2859

A.9.8.13.1	Test Purpose and Environment	2859

A.9.8.13.2	Test Requirements	2860

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2860

A.9.8.14.1	Test Purpose and Environment	2860

A.9.8.14.2	Test Requirements	2867

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2867

A.9.8.15.1	Test Purpose and Environment	2867

A.9.8.15.2	Test Requirements	2873

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2873

A.9.8.16.1	Test Purpose and Environment	2873

A.9.8.16.2	Test Requirements	2876

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2876

A.9.8.17.1	Test Purpose and Environment	2876

A.9.8.17.2	Test Requirements	2879

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2879

A.9.8.18.1	Test Purpose and Environment	2879

A.9.8.18.2	Test Requirements	2882

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2882

A.9.8.19.1	Test Purpose and Environment	2882

A.9.8.19.2	Test Requirements	2885

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2885

A.9.8.20.1	Test Purpose and Environment	2885

A.9.8.20.2	Test Requirements	2888

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2889

A.9.8.21.1	Test Purpose and Environment	2889

A.9.8.21.2	Test Requirements	2892

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2893

A.9.8.22.1	Test Purpose and Environment	2893

A.9.8.22.2	Test Requirements	2896

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2897

A.9.8.23.1	Test Purpose and Environment	2897

A.9.8.23.2	Test Requirements	2900

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2900

A.9.8.24.1	Test Purpose and Environment	2900

A.9.8.24.2	Test Requirements	2904

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2905

A.9.8.25.1	Test Purpose and Environment	2905

A.9.8.25.2	Test Requirements	2908

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2909

A.9.8.26.1	Test Purpose and Environment	2909

A.9.8.26.2	Test Requirements	2911

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2911

A.9.8.27.1	Test Purpose and Environment	2911

A.9.8.27.2	Test Requirements	2914

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2914

A.9.8.28.1	Test Purpose and Environment	2914

A.9.8.28.2	Test Requirements	2917

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2917

A.9.8.29.1	Test Purpose and Environment	2917

A.9.8.29.2	Test Requirements	2921

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2922

A.9.8.30.1	Test Purpose and Environment	2922

A.9.8.30.2	Test Requirements	2925

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2926

A.9.8.31.1	Test Purpose and Environment	2926

A.9.8.31.2	Test Requirements	2930

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2930

A.9.8.32.1	Test Purpose and Environment	2930

A.9.8.32.2	Test Requirements	2933

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2933

A.9.8.33.1	Test Purpose and Environment	2933

A.9.8.33.2	Test Requirements	2936

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2936

A.9.8.34.1	Test Purpose and Environment	2936

A.9.8.34.2	Test Requirements	2939

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2939

A.9.8.35.1	Test Purpose and Environment	2939

A.9.8.35.2	Test Requirements	2942

A.9.9	RSRP and RSRQ on the serving cell	2942

A.9.9.1	FDD Intra frequency serving cell case	2942

A.9.9.1.1	Test Purpose and Environment	2942

A.9.9.1.2	Test parameters	2942

A.9.9.1.3	Test Requirements	2945

A.9.9.2	TDD Intra frequency serving cell case	2945

A.9.9.2.1	Test Purpose and Environment	2945

A.9.9.2.2	Test parameters	2945

A.9.9.2.3	Test Requirements	2947

A.9.10	SSTD	2947

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2947

A.9.10.1.1	Test Purpose and Environment	2947

A.9.10.1.2	Test parameters	2947

A.9.10.1.3	Test Requirements	2949

A.9.10.2	Void	2949

A.9.10.3	Void	2949

A.9.10.4	Void	2949

A.9.11 RSSI	2949

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2949

A.9.11.1.1	Test Purpose and Environment	2949

A.9.11.1.2	Test parameters	2949

A.9.11.1.3	Test Requirements	2952

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2952

A.9.11.2.1	Test Purpose and Environment	2952

A.9.11.2.2	Test parameters	2952

A.9.12	Channel occupancy	2955

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2955

A.9.12.1.1	Test Purpose and Environment	2955

A.9.12.1.2	Test parameters	2955

A.9.12.1.3	Test Requirements	2958

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2958

A.9.12.2.1	Test Purpose and Environment	2958

A.9.12.2.2	Test parameters	2958

A.9.12.2.3	Test Requirements	2961

A.9.13	RS-SINR	2961

A.9.13.1	FDD Intra-Frequency Case	2961

A.9.13.1.1	Test Purpose and Environment	2961

A.9.13.1.2	Test parameters	2961

A.9.13.1.3	Test Requirements	2964

A.9.13.2	TDD Intra-Frequency Case	2964

A.9.13.2.1	Test Purpose and Environment	2964

A.9.13.2.2	Test parameters	2964

A.9.13.2.3	Test Requirements	2967

A.9.13.3	FDD—FDD Inter frequency case	2967

A.9.13.3.1	Test Purpose and Environment	2967

A.9.13.3.2	Test parameters	2967

A.9.13.3.3	Test Requirements	2970

A.9.13.4	TDD—TDD Inter frequency case	2970

A.9.13.4.1	Test Purpose and Environment	2970

A.9.13.4.2	Test parameters	2970

A.9.13.4.3	Test Requirements	2974

A.9.13.5	FDD—TDD Inter frequency case	2975

A.9.13.5.1	Test Purpose and Environment	2975

A.9.13.5.2	Test parameters	2975

A.9.13.5.3	Test Requirements	2981

A.9.13.6	TDD—FDD Inter frequency case	2981

A.9.13.6.1	Test Purpose and Environment	2981

A.9.13.6.2	Test parameters	2981

A.9.13.6.3	Test Requirements	2985

A.9.14	Channel quality reporting accuracy	2985

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	2985

A.9.14.1.1	Test Purpose and Environment	2985

A.9.14.1.2	Test parameters	2986

A.9.14.1.3	Test Requirements	2986

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	2987

A.9.14.2.1	Test Purpose and Environment	2987

A.9.14.2.2	Test parameters	2987

A.9.14.2.3	Test Requirements	2988

A.10	Proximity-based Services in Any Cell Selection State	2988

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2988

A.10.1.1	Test Purpose and Environment	2988

A.10.1.2	Test Requirements	2989

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2990

A.10.2.1	Test Purpose and Environment	2990

A.10.2.2	Test Requirements	2991

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	2991

A.10.3.1	Test Purpose and Environment	2991

A.10.3.2	Test Requirements	2993

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	2994

A.10.4.1	Test Purpose and Environment	2994

A.10.4.2	Test Requirements	2996

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	2996

A.11.1	V2V UE Transmission Timing Accuracy Test	2996

A.11.1.1	Test Purpose and Environment	2996

A.11.1.2	Test requirements	2997

A.11.2	Interruptions due to V2V sidelink communication	2997

A.11.2.1	Test Purpose and Environment	2997

A.11.2.2	Test Requirements	2999

A.12	2999

A.12.1	V2X UE Transmission Timing Accuracy Test	2999

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	2999

A.12.1.1.1	Test Purpose and Environment	2999

A.12.1.1.2	Test requirements	3000

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3001

A.12.1.2.1	Test Purpose and Environment	3001

A.12.1.2.2	Test Requirements	3001

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3002

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3002

A.12.2.1.1	Test Purpose and Environment	3002

A.12.2.1.2	Test Requirements	3003

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3004

A.12.2.2.1	Test Purpose and Environment	3004

A.12.2.2.2	Test Requirements	3005

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3005

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3005

A.12.3.1.1	Test Purpose and Environment	3005

A.12.3.1.2	Test Requirements	3008

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3009

A.12.3.2.1	Test Purpose and Environment	3009

A.12.3.1.2	Test Requirements	3011

A.12.4	Congestion Control Measurement Test for V2X UE	3011

A.12.4.1	Test Purpose and Environment	3011

A.12.4.2	Test Requirements	3013

A.12.5	Interruptions due to V2X Sidelink Communication	3013

A.12.5.1	Test Purpose and Environment	3013

A.12.5.1	Test Requirements	3015

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3015

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3015

A.12.6.1.1	Test Purpose and Environment	3015

A.12.6.1.2	Test Requirements	3017

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3017

A.12.6.2.1	Test Purpose and Environment	3017

A.12.6.1.2	Test Requirements	3019

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3019

A.12.7.1	Test Purpose and Environment	3019

A.12.7.2	Test Requirements	3022

A.12.8	Interruptions due to V2X Carrier Aggregation	3023

A.12.8.1	Interruptions on a FDD PCell	3023

A.12.8.1.1	Test Purpose and Environment	3023

A.12.8.1.2	Test Requirements	3024

A.12.8.2	Interruptions on a TDD PCell	3024

A.12.8.2.1	Test Purpose and Environment	3024

A.12.8.2.2	Test Requirements	3026

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	3027

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3027

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3027

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3027

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3027

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3029

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3030

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3030

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3030

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3030

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3032

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3033

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3033

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3033

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3034

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3034

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3035

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3035

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3035

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3036

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3036

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3037

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3037

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3037

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3037

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3038

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3038

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3038

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3039

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3039

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3039

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3040

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3040

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3041

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3042

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3043

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3044

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3045

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3045

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3045

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3045

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3046

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3046

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3048

B.3	Conditions for measurements performance requirements for UE	3048

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3048

B.3.2	Void	3048

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3048

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3048

B.3.5	Conditions for UE Rx – Tx time difference	3048

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3049

B.3.7	Conditions for inter-frequency RSTD measurements	3049

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3049

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3049

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3049

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3050

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3050

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3050

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3050

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3050

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3050

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3051

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3051

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3051

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3052

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3052

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3052

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3052

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3052

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3052

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3052

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3052

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3052

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3053

B.3.21.1	Conditions for RSRP measurements	3053

B.3.21.2	Conditions for RSRQ measurements	3053

B.3.21.3	Conditions for CSI-RSRP measurements	3053

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3053

B.3.22.1	Conditions for RSRP measurements	3053

B.3.22.2	Void	3054

B.3.22.3	Conditions for CSI-RSRP measurements	3054

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3054

B.3.23.1	Conditions for RSRP measurements	3054

B.3.23.2	Conditions for RSRQ measurements	3054

B.3.23.3	Conditions for CSI-RSRP measurements	3054

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3055

B.3.24.1	Conditions for RSRP measurements	3055

B.3.24.2	Conditions for RSRQ measurements	3055

B.3.24.3	Conditions for CSI-RSRP measurements	3055

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3055

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3055

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3055

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3056

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3056

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3056

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3056

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3056

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3057

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3057

B.4	RRM Requirements Exceptions	3057

B.4.1	General	3057

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3057

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3057

B.4.3.1	Inter-band carrier aggregation	3057

B.4.3.2	Intra-band non-contiguous carrier aggregation	3057

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3058

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3058

B.5.1	Conditions for S-RSRP Accuracy Requirements	3058

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3058

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3059

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3059

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3060

B.6	Conditions for V2X	3060

B.6.1	Test parameters for GNSS signals	3060

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3060

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3060

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3061

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3061

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	3061

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	3061

Annex C (informative):	Change history:	3063