# Annex C:	Appendix to Performance evaluations for R17 performance targets

## C.1	Performance analysis of Rel-16 positioning solutions

Including accuracy and latency (objective 1b) performance, compared to rel17 performance targets

### C.1.1	Positioning accuracy analysis

#### C.1.1.1	Results from source [4]

##### C.1.1.1.1	Description of evaluation scenarios

The scenarios to be presented in this contribution include

- Baseline scenarios with InF-SH/InF-DH with fixed UE/gNB height and without UE/gNB calibration error (Case 0xx series)

- InF-DH with variable UE/gNB height (Case 1xx series)

- InF-SH/In-DH with UE/gNB calibration error (Case 2xx series)

Evaluation assumptions for system level analysis are provided in Table C.1.1.1.1-1 to C.1.1.1.1-5.

Table C.1.1.1.1-1: Rel.16 NR positioning (baseline) - evaluation scenarios and parameters

| Parameter | Case 1 (InF-SH, FR1) | Case 2 (InF-SH, FR1) | Case 3 (InF-SH, FR1) | Case 4 (InF-SH, FR1) | Case 5 (InF-DH, FR1) | Case 6 (InF-DH, FR1) | Case 7 (InF-DH, FR1) | Case 8 (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | In-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based |
| Additional notes, if any | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. |

Table C.1.1.1.1-2: Rel.16 NR positioning (baseline) - evaluation scenarios and parameters

| Parameter | Case 9 (InF-SH, FR2) | Case 10 (InF-SH, FR2) | Case 11 (InF-SH, FR2) | Case 12 (InF-SH, FR2) | Case 13 (InF-DH, FR2) | Case 14 (InF-DH, FR2) | Case 15 (InF-DH, FR2) | Case 16 (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | In-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | Gold, single port | ZC, single port | Gold/ZC, single port | Gold, single port | Gold, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA PSO | DL-TDOA+DL-AoD PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | DL-TDOA PSO | DL-TDOA+DL-AoD PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based |
| Additional notes, if any | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. |

Table C.1.1.1.1-3: Rel.16 NR positioning (modified DH and 3D positioning) - evaluation scenarios and parameters

| Parameter | Case 101 (InF-DH, FR1) | Case 102 (InF-DH, FR1) | Case 103 (InF-DH, FR1) | Case 104 (InF-DH, FR1) | Case 105 (InF-DH, FR2) | Case 106 (InF-DH, FR2) | Case 107 (InF-DH, FR2) | Case 108 (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) |
| Carrier frequency | 3.5GHzHz | 3.5GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100Mhz | 100Mhz | 100Mhz | 100Mhz | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | ZC, single port | Gold/ZC, single port | ZC, single port | Gold/ZC, single port | ZC, single port | Gold/ZC, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based |
| Additional notes, if any | Fixed UE/gNB height. | Fixed UE/gNB height. | UE height within [0.5, 3] gNB height {4, 8} | UE height within [0.5, 3] gNB height {4, 8} | Fixed UE/gNB height. | Fixed UE/gNB height. | UE height within [0.5, 3] gNB height {4, 8} | UE height within [0.5, 3] gNB height {4, 8} |

Table C.1.1.1.1-4: Rel.16 NR positioning (UE/gNB calibration error) - evaluation scenarios and parameters

| Parameter | Case 201 (InF-SH, FR1) | Case 202 (InF-SH, FR1) | Case 203 (InF-SH, FR1) | Case 204 (InF-SH, FR1) | Case 205 (InF-DH, FR1) | Case 206 (InF-DH, FR1) | Case 207 (InF-DH, FR1) | Case 208 (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | In-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHzHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHzHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100Mhz | 100Mhz | 100Mhz | 100Mhz | 100Mhz | 100Mhz | 100Mhz | 100Mhz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | gNB Rx/Tx Time error T1=1.4ns UE Rx/Tx time error T1=5.6ns |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based |
| Additional notes, if any | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. |

Table C.1.1.1.1-5: Rel.16 NR positioning (gNB angle calibration error) - evaluation scenarios and parameters

| Parameter | Case 209 (InF-SH, FR1) | Case 210 (InF-SH, FR1) | Case 211 (InF-SH, FR1) | Case 212 (InF-SH, FR1) |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | In-SH | InF-SH | InF-SH |
| Carrier frequency | 3.5GHzHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100Mhz | 100Mhz | 100Mhz | 100Mhz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | ZC, single port | ZC, single port | ZC, single port | ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-AOA PSO | UL-AOA PSO | UL-AOA PSO | UL-AOA PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | gNB Rx Angle error θ=1° | gNB Rx Angle error θ=2° | gNB Rx Angle error θ=5° |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based |
| Additional notes, if any | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. |

##### C.1.1.1.2	Positioning accuracy evaluation results

Table C.1.1.1.2-1 provides summary of NR positioning evaluations results for horizontal location error under the baseline scenarios.

Table C.1.1.1.2-2 provides summary of NR positioning evaluations results for horizontal location error under the modified DH and 3D positioning.

Table C.1.1.1.2-3 provides summary of NR positioning evaluations results for horizontal location error under UE/gNB calibration error.

Table C.1.1.1.2-4 provides summary of NR positioning evaluations results for vertical location error under the 3D positioning.

Table C.1.1.1.2-1: Rel.16 NR positioning (baseline) - horizontal location error results from [4]

| Cases |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| 1, InF-SH, FR1, DL-TDOA | (Optional) All UEs | 0.077 | 0.153 | 0.419 | 2.789 |
|  | Convex UEs | 0.053 | 0.090 | 0.224 | 1.964 |
| 2, InF-SH, FR1, UL-TDOA | (Optional) All UEs | 0.0871 | 0.1708 | 0.4013 | 1.8678 |
|  | Convex UEs | 0.0621 | 0.1096 | 0.2410 | 1.0277 |
| 3, InF-SH, FR1, UL-TDOA/AoA | (Optional) All UEs | 0.0465 | 0.0731 | 0.1271 | 0.299 |
|  | Convex UEs | 0.0434 | 0.0648 | 0.1116 | 0.2682 |
| 4, InF-SH, FR1, Multi-RTT | (Optional) All UEs | 0.0721 | 0.2035 | 0.6628 | 1.9481 |
|  | Convex UEs | 0.0665 | 0.1798 | 0.5628 | 1.6992 |
| 5, InF-DH422, FR1, DL-TDOA | (Optional) All UEs | 0.861 | 3.448 | 8.103 | 15.212 |
|  | Convex UEs | 0.703 | 2.904 | 8.399 | 15.635 |
| 6, InF-DH422, FR1, UL-TDOA | (Optional) All UEs | 0.4309 | 1.6317 | 4.9693 | 9.9453 |
|  | Convex UEs | 0.3354 | 1.2393 | 4.6237 | 9.6631 |
| 7, InF-DH422, FR1, UL-TDOA/AoA | (Optional) All UEs | 0.0702 | 0.1673 | 0.3825 | 1.0453 |
|  | Convex UEs | 0.0643 | 0.1529 | 0.3206 | 0.8016 |
| 8, InF-DH422, FR1, Multi-RTT | (Optional) All UEs | 0.0699 | 0.1260 | 0.3916 | 7.8992 |
|  | Convex UEs | 0.0667 | 0.1164 | 0.3338 | 7.3110 |
| 9, InF-SH, FR2, DL-TDOA | (Optional) All UEs | 0.0089 | 0.0329 | 0.1935 | 1.1281 |
|  | Convex UEs | 0.0077 | 0.0265 | 0.1586 | 0.9633 |
| 10, InF-SH, FR2, DL-TDOA/AoD | (Optional) All UEs | 0.0118 | 0.0203 | 0.0345 | 0.0918 |
|  | Convex UEs | 0.0143 | 0.0231 | 0.0387 | 0.0654 |
| 11, InF-SH, FR2, UL-TDOA/AoA | (Optional) All UEs | 0.0102 | 0.0191 | 0.0365 | 0.0991 |
|  | Convex UEs | 0.0084 | 0.0156 | 0.0296 | 0.0694 |
| 12, InF-SH, FR2, Multi-RTT | (Optional) All UEs | 0.0049 | 0.0143 | 0.0881 | 0.4697 |
|  | Convex UEs | 0.0048 | 0.0137 | 0.0841 | 0.4496 |
| 13, InF-DH422, FR2, DL-TDOA | (Optional) All UEs | 0.0466 | 0.5795 | 3.6866 | 9.2401 |
|  | Convex UEs | 0.0321 | 0.4003 | 2.7974 | 9.6798 |
| 14, InF- DH422, FR2, DL-TDOA/AoD | (Optional) All UEs | 0.0755 | 0.1861 | 0.4228 | 0.9619 |
|  | Convex UEs | 0.0587 | 0.1452 | 0.3221 | 0.7197 |
| 15, InF-DH422, FR2, UL-TDOA/AoA | (Optional) All UEs | 0.0732 | 0.1728 | 0.3750 | 0.9277 |
|  | Convex UEs | 0.0603 | 0.1309 | 0.2717 | 0.7086 |
| 16, InF- DH422, FR2, Multi-RTT | (Optional) All UEs | 0.0317 | 0.1905 | 1.0572 | 4.1555 |
|  | Convex UEs | 0.0259 | 0.1425 | 0.8398 | 4.2895 |

Table C.1.1.1.2-2: Rel.16 NR positioning (modified DH and 3D positioning) - horizontal location error results from [4]

| Cases |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| 101, InF-DH435, FR1, UL-TDOA/AoA | (Optional) All UEs | 0.0742 | 0.1792 | 0.4392 | 1.3838 |
|  | Convex UEs | 0.0628 | 0.1571 | 0.3844 | 1.3012 |
| 102, InF-DH435, FR1, Multi-RTT | (Optional) All UEs | 0.0713 | 0.1249 | 0.4734 | 8.6557 |
|  | Convex UEs | 0.0663 | 0.1082 | 0.4952 | 9.8411 |
| 103, InF-DH435-3D, FR1, UL-TDOA/AoA | (Optional) All UEs | 0.3405(H) | 1.0103(H) | 2.3157(H) | 5.1203(H) |
|  | Convex UEs | 0.2923(H) | 0.8122(H) | 2.0088(H) | 4.3405(H) |
| 104, InF-DH435-3D, FR1, Multi-RTT | (Optional) All UEs | 0.1721(H) | 1.3411(H) | 7.4143(H) | 15.4966(H) |
|  | Convex UEs | 0.1258(H) | 1.4345(H) | 8.9295(H) | 16.0515(H) |
| 105, InF-DH435, FR2, UL-TDOA/AoA | (Optional) All UEs | 0.0943 | 0.2314 | 0.4973 | 1.3007 |
|  | Convex UEs | 0.0865 | 0.2101 | 0.4707 | 1.1486 |
| 106, InF-DH435, FR2, Multi-RTT | (Optional) All UEs | 0.3364 | 0.9128 | 2.4777 | 5.7209 |
|  | Convex UEs | 0.3079 | 0.8594 | 2.2392 | 5.4600 |
| 107, InF-DH435-3D, FR2, UL-TDOA/AoA | (Optional) All UEs | 0.1297(H) | 0.4173(H) | 1.1042(H) | 2.6878(H) |
|  | Convex UEs | 0.1109(H) | 0.3273(H) | 0.8667(H) | 2.4365(H) |
| 108, InF-DH435-3D, FR2, Multi-RTT | (Optional) All UEs | 0.5012(H) | 2.2343(H) | 6.6096(H) | 15.6304(H) |
|  | Convex UEs | 0.4304(H) | 2.4466(H) | 7.9926(H) | 15.5828(H) |

Table C.1.1.1.2-3: Rel.16 NR positioning (UE/gNB calibration error) - horizontal location error results from [4]

| Cases |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| 201, InF-SH, FR1, DL-TDOA, Group Delay Error | (Optional) All UEs | 0.587 | 0.874 | 1.462 | 3.540 |
|  | Convex UEs | 0.462 | 0.614 | 0.829 | 1.458 |
| 202, InF-SH, FR1, UL-TDOA, Group delay error | (Optional) All UEs | 0.5937 | 0.8672 | 1.3135 | 2.6395 |
|  | Convex UEs | 0.4653 | 0.6145 | 0.8120 | 1.2343 |
| 203, InF-SH, FR1, UL-TDOA/AoA, Group delay error | (Optional) All UEs | 0.0699 | 0.1180 | 0.2034 | 0.3878 |
|  | Convex UEs | 0.0661 | 0.1094 | 0.1750 | 0.3251 |
| 204, InF-SH, FR1, Multi-RTT, Group delay error | (Optional) All UEs | 1.7167 | 2.3902 | 3.3592 | 4.4076 |
|  | Convex UEs | 1.7105 | 2.3963 | 3.3574 | 4.2662 |
| 205, InF-DH422, FR1, DL-TDOA, Group delay error | (Optional) All UEs | 1.719 | 4.034 | 8.588 | 15.330 |
|  | Convex UEs | 1.391 | 3.407 | 8.570 | 15.039 |
| 206, InF-DH422, FR1, UL-TDOA, Group delay error | (Optional) All UEs | 1.0065 | 2.0631 | 4.7037 | 10.459 |
|  | Convex UEs | 0.8741 | 1.5739 | 4.2751 | 9.4102 |
| 207, InF-DH422, FR1, UL-TDOA/AoA, Group delay error | (Optional) All UEs | 0.0810 | 0.184 | 0.4251 | 1.1961 |
|  | Convex UEs | 0.0717 | 0.1577 | 0.3371 | 0.8662 |
| 208, InF-DH422, FR1, Multi-RTT, Group delay error | (Optional) All UEs | 2.2640 | 3.2695 | 4.7279 | 8.9874 |
|  | Convex UEs | 2.2784 | 3.3381 | 5.0728 | 9.5701 |
| 209, InF-SH, FR1, UL-AoA | (Optional) All UEs | 0.0446 | 0.0615 | 0.0879 | 0.1383 |
|  | Convex UEs | 0.0410 | 0.0552 | 0.0758 | 0.1119 |
| 210, InF-SH, FR1, UL-AoA, Angle error 1 degree | (Optional) All UEs | 0.4960 | 0.6675 | 0.8724 | 1.3003 |
|  | Convex UEs | 0.4629 | 0.6280 | 0.8016 | 1.1676 |
| 211, InF-SH, FR1, UL-AoA, Angle error 2 degrees | (Optional) All UEs | 0.9357 | 1.2729 | 1.6915 | 2.6204 |
|  | Convex UEs | 0.8848 | 1.1961 | 1.5487 | 2.1732 |
| 212, InF-SH, FR1, UL-AoA, Angle error 5 degrees | (Optional) All UEs | 2.3274 | 3.1571 | 4.2839 | 5.9433 |
|  | Convex UEs | 2.2187 | 2.9534 | 4.0582 | 5.3982 |

Table C.1.1.1.2-4: Rel.16 NR positioning (modified DH and 3D positioning) - altitude location error results from [4]

| Cases |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| 103, InF-DH435-3D, FR1, UL-TDOA/AoA | (Optional) All UEs | 0.0905(V) | 0.2923(V) | 0.7287(V) | 1.3239(V) |
|  | Convex UEs | 00682(V) | 0.2182(V) | 0.5301(V) | 1.1585(V) |
| 104, InF-DH435-3D, FR1, Multi-RTT | (Optional) All UEs | 0.2176(V) | 0.6510(V) | 1.1742(V) | 1.7622(V) |
|  | Convex UEs | 0.1666(V) | 0.5005(V) | 1.0346(V) | 1.6675(V) |
| 107, InF-DH435-3D, FR2, UL-TDOA/AoA | (Optional) All UEs | 0.0361(V) | 0.1075(V) | 0.2471(V) | 0.5573(V) |
|  | Convex UEs | 0.0288(V) | 0.0796(V) | 0.1879(V) | 0.4593(V) |
| 108, InF-DH435-3D, FR2, Multi-RTT | (Optional) All UEs | 0.3436(V) | 0.9022(V) | 1.5000(V) | 1.9555(V) |
|  | Convex UEs | 0.2871(V) | 0.7927(V) | 1.3316(V) | 1.8800(V) |

Figures C.1.1.1.2-1 to C.1.1.1.2-4 provide positioning evaluations results for the baseline scenario.

Figures C.1.1.1.2-5 to C.1.1.1.2-8 provide positioning evaluation results for the modified DH and 3D positioning (including horizontal and vertical error).

Figures C.1.1.1.2-9 to C.1.1.1.2-11 provide positioning evaluation results for the UE/gNB calibration error.

Figure C.1.1.1.2-1: Rel.16 NR positioning error results (baseline) from [4]

Figure C.1.1.1.2-2: Rel.16 NR positioning error results (baseline) from [4]

Figure C.1.1.1.2-3: Rel.16 NR positioning error results (baseline) from [4]

Figure C.1.1.1.2-4: Rel.16 NR positioning error results (baseline) from [4]

Figure C.1.1.1.2-5: Rel.16 NR positioning error results (modified DH and 2D) from [4]

Figure C.1.1.1.2-6: Rel.16 NR positioning error results (modified DH and 3D) from [4]

Figure C.1.1.1.2-7: Rel.16 NR positioning error results (modified DH and 2D) from [4]

Figure C.1.1.1.2-8: Rel.16 NR positioning error results (modified DH and 3D) from [4]

Figure C.1.1.1.2-9: Rel.16 NR positioning error results (UE/gNB calibration error) from [4]

Figure C.1.1.1.2-10: Rel.16 NR positioning error results (UE/gNB calibration error) from [4]

Figure C.1.1.1.2-11: Rel.16 NR positioning error results (gNB angle calibration error) from [4]

#### C.1.1.2	Results from source [7]

##### C.1.1.2.1	Description of evaluation scenarios

The evaluation scenarios for verifying achievable accuracy based on Rel-16 positioning methods in this contribution  include,

- Baseline scenarios with InF-SH/InF-DH (both FR1 and FR2) with fixed UE/gNB height and without network synchronization, for  comparing results when UEs are uniformly distributed and inside the covex hull over the factory.

- Baseline scenarios with InF-SH/InF-DH (both FR1 and FR2) with fixed UE/gNB height and without network synchronization, for  comparing results when gNBs have different Tx calibration errors (0ns ,0.5 ns, 1 ns and 2 ns).

- InF-SH/InF-DH (both FR1 and FR2) with discrete  gNB (staggered distribution) height and uniformly distributed UE height, for investigating vertical positioning accuracy.

Some scenario common parameters can be found in Table C.1.2.2.1-1. In addition, Table C.1.2.2.1-2 reveals some controlled variables of all simulation cases (Case 1-24).

Table C.1.1.2.1-1 Scenario common parameters

Table C.1.1.2.1-2 All simulation cases for positioning accuracy evaluation based on Rel-16 positioning methods

| Simulation cases | Scenario | FR1/FR2 | UE horizontal drop procedure | gNB Tx calibration error | Clutter parameters |
| --- | --- | --- | --- | --- | --- |
| Case 1 | InF-SH | FR1 | Uniformly distributed | 0ns | {20%, 2m, 10m} |
| Case 2 | InF-SH | FR1 | Inside convex hull | 0ns | {20%, 2m, 10m} |
| Case 3 | InF-SH | FR1 | Inside convex hull | 0.5ns | {20%, 2m, 10m} |
| Case 4 | InF-SH | FR1 | Inside convex hull | 1ns | {20%, 2m, 10m} |
| Case 5 | InF-SH | FR1 | Inside convex hull | 2ns | {20%, 2m, 10m} |
| Case 6 | InF-SH | FR2 | Uniformly distributed | 0ns | {20%, 2m, 10m} |
| Case 7 | InF-SH | FR2 | Inside convex hull | 0ns | {20%, 2m, 10m} |
| Case 8 | InF-SH | FR2 | Inside convex hull | 0.5ns | {20%, 2m, 10m} |
| Case 9 | InF-SH | FR2 | Inside convex hull | 1ns | {20%, 2m, 10m} |
| Case 10 | InF-SH | FR2 | Inside convex hull | 2ns | {20%, 2m, 10m} |
| Case 11 | InF-DH | FR1 | Uniformly distributed | 0ns | {40%, 2m, 2m} |
| Case 12 | InF-DH | FR1 | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 13 | InF-DH | FR1 | Inside convex hull | 0.5ns | {40%, 2m, 2m} |
| Case 14 | InF-DH | FR1 | Inside convex hull | 1ns | {40%, 2m, 2m} |
| Case 15 | InF-DH | FR1 | Inside convex hull | 2ns | {40%, 2m, 2m} |
| Case 16 | InF-DH | FR2 | Uniformly distributed | 0ns | {40%, 2m, 2m} |
| Case 17 | InF-DH | FR2 | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 18 | InF-DH | FR2 | Inside convex hull | 0.5ns | {40%, 2m, 2m} |
| Case 19 | InF-DH | FR2 | Inside convex hull | 1ns | {40%, 2m, 2m} |
| Case 20 | InF-DH | FR2 | Inside convex hull | 2ns | {40%, 2m, 2m} |
| Case 21 | InF-SH | FR1 | Inside convex hull | 0ns | {20%, 2m, 10m} |
| Case 22 | InF-SH | FR2 | Inside convex hull | 0ns | {20%, 2m, 10m} |
| Case 23 | InF-DH | FR1 | Inside convex hull | 0ns | {40%, 3m,5m} |
| Case 24 | InF-DH | FR2 | Inside convex hull | 0ns | {40%, 3m, 5m} |

##### C.1.1.2.2	Positioning accuracy evaluation results

Table C.1.1.2.2-1 provides CDF of horizontal positioning accuracy at some specific percentiles for InF-SH, FR1 scenario, and corresponding CDF curves can be found in Figure C.1.1.2.2-1-1.

Table C.1.1.2.2-2 provides CDF of horizontal positioning accuracy at some specific percentiles for InF-SH, FR2 scenario, and corresponding CDF curves can be found in Figure C.1.1.2.2-1-2.

Table C.1.1.2.2-3 provides CDF of horizontal positioning accuracy at some specific percentiles for InF-DH, FR1 scenario,  and corresponding CDF curves can be found in Figure C.1.1.2.2-1-3.

Table C.1.1.2.2-4 provides CDF of horizontal positioning accuracy at some specific percentiles for InF-DH, FR2 scenario, and corresponding CDF curves can be found in Figure C.1.1.2.2-1-4.

Table C.1.1.2.2-5 provides CDF of vertical positioning accuracy at some specific percentiles  for InF-SH and InF-DH, and corresponding CDF curves can be found in Figure C.1.1.2.2-2.

Table C.1.1.2.2-1 CDF of horizontal positioning accuracy at some specific percentiles for InF-SH, FR1 scenario

| Cases | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50% | 67% | 80% | 90% |
| Case 1 | 0.282 | 0.370 | 0.467 | 0.603 |
| Case 2 | 0.264 | 0.350 | 0.469 | 0.568 |
| Case 3 | 0.301 | 0.411 | 0.549 | 0.704 |
| Case 4 | 0.475 | 0.616 | 0.767 | 0.943 |
| Case 5 | 0.841 | 1.042 | 1.239 | 1.479 |

Table C.1.1.2.2-2 CDF of horizontal positioning accuracy at some specific percentiles for InF-SH, FR2 scenario

| Cases | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50% | 67% | 80% | 90% |
| Case 6 | 0.044 | 0.059 | 0.072 | 0.092 |
| Case 7 | 0.041 | 0.055 | 0.070 | 0.090 |
| Case 8 | 0.171 | 0.223 | 0.257 | 0.300 |
| Case 9 | 0.419 | 0.480 | 0.545 | 0.615 |
| Case 10 | 0.833 | 0.971 | 1.086 | 1.224 |

Table C.1.1.2.2-3 CDF of horizontal positioning accuracy at some specific percentiles for InF-DH, FR1 scenario

| Cases | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50% | 67% | 80% | 90% |
| Case 11 | 0.552 | 5.708 | 10.120 | 12.433 |
| Case 12 | 0.414 | 1.060 | 7.337 | 12.345 |
| Case13 | 0.885 | 1.719 | 7.544 | 12.386 |
| Case 14 | 1.242 | 3.560 | 8.249 | 12.368 |
| Case 15 | 1.632 | 4.127 | 7.701 | 12.458 |

Table C.1.1.2.2-4 CDF of horizontal positioning accuracy at some specific percentiles for InF-DH, FR2 scenario

| Cases | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50% | 67% | 80% | 90% |
| Case 16 | 0.076 | 0.150 | 10.255 | 14.759 |
| Case 17 | 0.062 | 0.097 | 0.174 | 12.174 |
| Case 18 | 0.323 | 0.458 | 0.665 | 10.815 |
| Case 19 | 0.542 | 0.738 | 1.045 | 12.285 |
| Case 20 | 0.958 | 1.220 | 11.075 | 14.845 |

Table C.1.1.2.2-5 CDF of vertical positioning accuracy at some specific percentiles  for InF-SH and InF-DH

| Simulation cases | Vertical positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50% | 67% | 80% | 90% |
| Case 21 | 0.411 | 0.623 | 0.815 | 0.979 |
| Case 22 | 0.185 | 0.293 | 0.362 | 0.459 |
| Case 23 | 0.522 | 0.814 | 1.062 | 1.419 |
| Case 24 | 0.222 | 0.414 | 0.705 | 1.271 |

Figure C.1.1.2.2-1-1 InF-SH, FR1                        Figure C.1.1.2.2-1-2 InF-SH, FR2

Figure C.1.1.2.2-1-3 InF-DH, FR1                         Figure C.1.1.2.2-1-4 InF-DH, FR2

Figure C.1.1.2.2-1 Horizontal positioning accuracy based on Rel-16 DL-TDOA method

Figure C.1.1.2.2-2-1 InF-SH                      Figure C.1.1.2.2-2-2 InF-DH

Figure C.1.1.2.2-2 Vertical positioning accuracy based on Rel-16 DL-TDOA method

Table C.1.1.1.2-1: Rel.16 NR positioning (baseline) - horizontal location error results from [4]

| Cases |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| 1, InF-SH, FR1, DL-TDOA | (Optional) All UEs | 0.077 | 0.153 | 0.419 | 2.789 |
|  | Convex UEs | 0.053 | 0.090 | 0.224 | 1.964 |
| 2, InF-SH, FR1, UL-TDOA | (Optional) All UEs | 0.0871 | 0.1708 | 0.4013 | 1.8678 |
|  | Convex UEs | 0.0621 | 0.1096 | 0.2410 | 1.0277 |
| 3, InF-SH, FR1, UL-TDOA/AoA | (Optional) All UEs | 0.0465 | 0.0731 | 0.1271 | 0.299 |
|  | Convex UEs | 0.0434 | 0.0648 | 0.1116 | 0.2682 |
| 4, InF-SH, FR1, Multi-RTT | (Optional) All UEs | 0.0721 | 0.2035 | 0.6628 | 1.9481 |
|  | Convex UEs | 0.0665 | 0.1798 | 0.5628 | 1.6992 |
| 5, InF-DH422, FR1, DL-TDOA | (Optional) All UEs | 0.861 | 3.448 | 8.103 | 15.212 |
|  | Convex UEs | 0.703 | 2.904 | 8.399 | 15.635 |
| 6, InF-DH422, FR1, UL-TDOA | (Optional) All UEs | 0.4309 | 1.6317 | 4.9693 | 9.9453 |
|  | Convex UEs | 0.3354 | 1.2393 | 4.6237 | 9.6631 |
| 7, InF-DH422, FR1, UL-TDOA/AoA | (Optional) All UEs | 0.0702 | 0.1673 | 0.3825 | 1.0453 |
|  | Convex UEs | 0.0643 | 0.1529 | 0.3206 | 0.8016 |
| 8, InF-DH422, FR1, Multi-RTT | (Optional) All UEs | 0.0699 | 0.1260 | 0.3916 | 7.8992 |
|  | Convex UEs | 0.0667 | 0.1164 | 0.3338 | 7.3110 |
| 9, InF-SH, FR2, DL-TDOA | (Optional) All UEs | 0.0089 | 0.0329 | 0.1935 | 1.1281 |
|  | Convex UEs | 0.0077 | 0.0265 | 0.1586 | 0.9633 |
| 10, InF-SH, FR2, DL-TDOA/AoD | (Optional) All UEs | 0.0118 | 0.0203 | 0.0345 | 0.0918 |
|  | Convex UEs | 0.0143 | 0.0231 | 0.0387 | 0.0654 |
| 11, InF-SH, FR2, UL-TDOA/AoA | (Optional) All UEs | 0.0102 | 0.0191 | 0.0365 | 0.0991 |
|  | Convex UEs | 0.0084 | 0.0156 | 0.0296 | 0.0694 |
| 12, InF-SH, FR2, Multi-RTT | (Optional) All UEs | 0.0049 | 0.0143 | 0.0881 | 0.4697 |
|  | Convex UEs | 0.0048 | 0.0137 | 0.0841 | 0.4496 |
| 13, InF-DH422, FR2, DL-TDOA | (Optional) All UEs | 0.0466 | 0.5795 | 3.6866 | 9.2401 |
|  | Convex UEs | 0.0321 | 0.4003 | 2.7974 | 9.6798 |
| 14, InF- DH422, FR2, DL-TDOA/AoD | (Optional) All UEs | 0.0755 | 0.1861 | 0.4228 | 0.9619 |
|  | Convex UEs | 0.0587 | 0.1452 | 0.3221 | 0.7197 |
| 15, InF-DH422, FR2, UL-TDOA/AoA | (Optional) All UEs | 0.0732 | 0.1728 | 0.3750 | 0.9277 |
|  | Convex UEs | 0.0603 | 0.1309 | 0.2717 | 0.7086 |
| 16, InF- DH422, FR2, Multi-RTT | (Optional) All UEs | 0.0317 | 0.1905 | 1.0572 | 4.1555 |
|  | Convex UEs | 0.0259 | 0.1425 | 0.8398 | 4.2895 |

Table C.1.1.1.2-2: Rel.16 NR positioning (modified DH and 3D positioning) - horizontal location error results from [4]

| Cases |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| 101, InF-DH435, FR1, UL-TDOA/AoA | (Optional) All UEs | 0.0742 | 0.1792 | 0.4392 | 1.3838 |
|  | Convex UEs | 0.0628 | 0.1571 | 0.3844 | 1.3012 |
| 102, InF-DH435, FR1, Multi-RTT | (Optional) All UEs | 0.0713 | 0.1249 | 0.4734 | 8.6557 |
|  | Convex UEs | 0.0663 | 0.1082 | 0.4952 | 9.8411 |
| 103, InF-DH435-3D, FR1, UL-TDOA/AoA | (Optional) All UEs | 0.3405(H) | 1.0103(H) | 2.3157(H) | 5.1203(H) |
|  | Convex UEs | 0.2923(H) | 0.8122(H) | 2.0088(H) | 4.3405(H) |
| 104, InF-DH435-3D, FR1, Multi-RTT | (Optional) All UEs | 0.1721(H) | 1.3411(H) | 7.4143(H) | 15.4966(H) |
|  | Convex UEs | 0.1258(H) | 1.4345(H) | 8.9295(H) | 16.0515(H) |
| 105, InF-DH435, FR2, UL-TDOA/AoA | (Optional) All UEs | 0.0943 | 0.2314 | 0.4973 | 1.3007 |
|  | Convex UEs | 0.0865 | 0.2101 | 0.4707 | 1.1486 |
| 106, InF-DH435, FR2, Multi-RTT | (Optional) All UEs | 0.3364 | 0.9128 | 2.4777 | 5.7209 |
|  | Convex UEs | 0.3079 | 0.8594 | 2.2392 | 5.4600 |
| 107, InF-DH435-3D, FR2, UL-TDOA/AoA | (Optional) All UEs | 0.1297(H) | 0.4173(H) | 1.1042(H) | 2.6878(H) |
|  | Convex UEs | 0.1109(H) | 0.3273(H) | 0.8667(H) | 2.4365(H) |
| 108, InF-DH435-3D, FR2, Multi-RTT | (Optional) All UEs | 0.5012(H) | 2.2343(H) | 6.6096(H) | 15.6304(H) |
|  | Convex UEs | 0.4304(H) | 2.4466(H) | 7.9926(H) | 15.5828(H) |

Table C.1.1.1.2-3: Rel.16 NR positioning (UE/gNB calibration error) - horizontal location error results from [4]

| Cases |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| 201, InF-SH, FR1, DL-TDOA, Group Delay Error | (Optional) All UEs | 0.587 | 0.874 | 1.462 | 3.540 |
|  | Convex UEs | 0.462 | 0.614 | 0.829 | 1.458 |
| 202, InF-SH, FR1, UL-TDOA, Group delay error | (Optional) All UEs | 0.5937 | 0.8672 | 1.3135 | 2.6395 |
|  | Convex UEs | 0.4653 | 0.6145 | 0.8120 | 1.2343 |
| 203, InF-SH, FR1, UL-TDOA/AoA, Group delay error | (Optional) All UEs | 0.0699 | 0.1180 | 0.2034 | 0.3878 |
|  | Convex UEs | 0.0661 | 0.1094 | 0.1750 | 0.3251 |
| 204, InF-SH, FR1, Multi-RTT, Group delay error | (Optional) All UEs | 1.7167 | 2.3902 | 3.3592 | 4.4076 |
|  | Convex UEs | 1.7105 | 2.3963 | 3.3574 | 4.2662 |
| 205, InF-DH422, FR1, DL-TDOA, Group delay error | (Optional) All UEs | 1.719 | 4.034 | 8.588 | 15.330 |
|  | Convex UEs | 1.391 | 3.407 | 8.570 | 15.039 |
| 206, InF-DH422, FR1, UL-TDOA, Group delay error | (Optional) All UEs | 1.0065 | 2.0631 | 4.7037 | 10.459 |
|  | Convex UEs | 0.8741 | 1.5739 | 4.2751 | 9.4102 |
| 207, InF-DH422, FR1, UL-TDOA/AoA, Group delay error | (Optional) All UEs | 0.0810 | 0.184 | 0.4251 | 1.1961 |
|  | Convex UEs | 0.0717 | 0.1577 | 0.3371 | 0.8662 |
| 208, InF-DH422, FR1, Multi-RTT, Group delay error | (Optional) All UEs | 2.2640 | 3.2695 | 4.7279 | 8.9874 |
|  | Convex UEs | 2.2784 | 3.3381 | 5.0728 | 9.5701 |
| 209, InF-SH, FR1, UL-AoA | (Optional) All UEs | 0.0446 | 0.0615 | 0.0879 | 0.1383 |
|  | Convex UEs | 0.0410 | 0.0552 | 0.0758 | 0.1119 |
| 210, InF-SH, FR1, UL-AoA, Angle error 1 degree | (Optional) All UEs | 0.4960 | 0.6675 | 0.8724 | 1.3003 |
|  | Convex UEs | 0.4629 | 0.6280 | 0.8016 | 1.1676 |
| 211, InF-SH, FR1, UL-AoA, Angle error 2 degrees | (Optional) All UEs | 0.9357 | 1.2729 | 1.6915 | 2.6204 |
|  | Convex UEs | 0.8848 | 1.1961 | 1.5487 | 2.1732 |
| 212, InF-SH, FR1, UL-AoA, Angle error 5 degrees | (Optional) All UEs | 2.3274 | 3.1571 | 4.2839 | 5.9433 |
|  | Convex UEs | 2.2187 | 2.9534 | 4.0582 | 5.3982 |

Table C.1.1.1.2-4: Rel.16 NR positioning (modified DH and 3D positioning) - altitude location error results from [4]

| Cases |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| 103, InF-DH435-3D, FR1, UL-TDOA/AoA | (Optional) All UEs | 0.0905(V) | 0.2923(V) | 0.7287(V) | 1.3239(V) |
|  | Convex UEs | 00682(V) | 0.2182(V) | 0.5301(V) | 1.1585(V) |
| 104, InF-DH435-3D, FR1, Multi-RTT | (Optional) All UEs | 0.2176(V) | 0.6510(V) | 1.1742(V) | 1.7622(V) |
|  | Convex UEs | 0.1666(V) | 0.5005(V) | 1.0346(V) | 1.6675(V) |
| 107, InF-DH435-3D, FR2, UL-TDOA/AoA | (Optional) All UEs | 0.0361(V) | 0.1075(V) | 0.2471(V) | 0.5573(V) |
|  | Convex UEs | 0.0288(V) | 0.0796(V) | 0.1879(V) | 0.4593(V) |
| 108, InF-DH435-3D, FR2, Multi-RTT | (Optional) All UEs | 0.3436(V) | 0.9022(V) | 1.5000(V) | 1.9555(V) |
|  | Convex UEs | 0.2871(V) | 0.7927(V) | 1.3316(V) | 1.8800(V) |

Figures C.1.1.1.2-1 to C.1.1.1.2-4 provide positioning evaluations results for the baseline scenario.

Figures C.1.1.1.2-5 to C.1.1.1.2-8 provide positioning evaluation results for the modified DH and 3D positioning (including horizontal and vertical error).

Figures C.1.1.1.2-9 to C.1.1.1.2-11 provide positioning evaluation results for the UE/gNB calibration error.

Figure C.1.1.1.2-1: Rel.16 NR positioning error results (baseline) from [4]

Figure C.1.1.1.2-2: Rel.16 NR positioning error results (baseline) from [4]

Figure C.1.1.1.2-3: Rel.16 NR positioning error results (baseline) from [4]

Figure C.1.1.1.2-4: Rel.16 NR positioning error results (baseline) from [4]

Figure C.1.1.1.2-5: Rel.16 NR positioning error results (modified DH and 2D) from [4]

Figure C.1.1.1.2-6: Rel.16 NR positioning error results (modified DH and 3D) from [4]

Figure C.1.1.1.2-7: Rel.16 NR positioning error results (modified DH and 2D) from [4]

Figure C.1.1.1.2-8: Rel.16 NR positioning error results (modified DH and 3D) from [4]

Figure C.1.1.1.2-9: Rel.16 NR positioning error results (UE/gNB calibration error) from [4]

Figure C.1.1.1.2-10: Rel.16 NR positioning error results (UE/gNB calibration error) from [4]

Figure C.1.1.1.2-11: Rel.16 NR positioning error results (gNB angle calibration error) from [4]

Table C.1.1.1.1-1: Rel.16 NR positioning (baseline) - evaluation scenarios and parameters

| Parameter | Case 1 (InF-SH, FR1) | Case 2 (InF-SH, FR1) | Case 3 (InF-SH, FR1) | Case 4 (InF-SH, FR1) | Case 5 (InF-DH, FR1) | Case 6 (InF-DH, FR1) | Case 7 (InF-DH, FR1) | Case 8 (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | In-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based |
| Additional notes, if any | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. |

Table C.1.1.1.1-2: Rel.16 NR positioning (baseline) - evaluation scenarios and parameters

| Parameter | Case 9 (InF-SH, FR2) | Case 10 (InF-SH, FR2) | Case 11 (InF-SH, FR2) | Case 12 (InF-SH, FR2) | Case 13 (InF-DH, FR2) | Case 14 (InF-DH, FR2) | Case 15 (InF-DH, FR2) | Case 16 (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | In-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | Gold, single port | ZC, single port | Gold/ZC, single port | Gold, single port | Gold, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA PSO | DL-TDOA+DL-AoD PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | DL-TDOA PSO | DL-TDOA+DL-AoD PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based |
| Additional notes, if any | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. |

Table C.1.1.1.1-3: Rel.16 NR positioning (modified DH and 3D positioning) - evaluation scenarios and parameters

| Parameter | Case 101 (InF-DH, FR1) | Case 102 (InF-DH, FR1) | Case 103 (InF-DH, FR1) | Case 104 (InF-DH, FR1) | Case 105 (InF-DH, FR2) | Case 106 (InF-DH, FR2) | Case 107 (InF-DH, FR2) | Case 108 (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) |
| Carrier frequency | 3.5GHzHz | 3.5GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100Mhz | 100Mhz | 100Mhz | 100Mhz | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | ZC, single port | Gold/ZC, single port | ZC, single port | Gold/ZC, single port | ZC, single port | Gold/ZC, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based |
| Additional notes, if any | Fixed UE/gNB height. | Fixed UE/gNB height. | UE height within [0.5, 3] gNB height {4, 8} | UE height within [0.5, 3] gNB height {4, 8} | Fixed UE/gNB height. | Fixed UE/gNB height. | UE height within [0.5, 3] gNB height {4, 8} | UE height within [0.5, 3] gNB height {4, 8} |

Table C.1.1.1.1-4: Rel.16 NR positioning (UE/gNB calibration error) - evaluation scenarios and parameters

| Parameter | Case 201 (InF-SH, FR1) | Case 202 (InF-SH, FR1) | Case 203 (InF-SH, FR1) | Case 204 (InF-SH, FR1) | Case 205 (InF-DH, FR1) | Case 206 (InF-DH, FR1) | Case 207 (InF-DH, FR1) | Case 208 (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | In-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHzHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHzHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100Mhz | 100Mhz | 100Mhz | 100Mhz | 100Mhz | 100Mhz | 100Mhz | 100Mhz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | gNB Rx/Tx Time error T1=1.4ns UE Rx/Tx time error T1=5.6ns |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based |
| Additional notes, if any | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. |

Table C.1.1.1.1-5: Rel.16 NR positioning (gNB angle calibration error) - evaluation scenarios and parameters

| Parameter | Case 209 (InF-SH, FR1) | Case 210 (InF-SH, FR1) | Case 211 (InF-SH, FR1) | Case 212 (InF-SH, FR1) |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | In-SH | InF-SH | InF-SH |
| Carrier frequency | 3.5GHzHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100Mhz | 100Mhz | 100Mhz | 100Mhz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | ZC, single port | ZC, single port | ZC, single port | ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-AOA PSO | UL-AOA PSO | UL-AOA PSO | UL-AOA PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | gNB Rx Angle error θ=1° | gNB Rx Angle error θ=2° | gNB Rx Angle error θ=5° |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based |
| Additional notes, if any | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. |

#### C.1.1.3	 Results from source [8]

##### C.1.1.3.1	Description of evaluation scenarios

Evaluation assumptions for system level analysis are provided from Table C.1.1.3.3-1a to Table C.1.1.3.3-1f.

- Case IDs: 1 to 36.

- Scenarios: InF-SH-2D, InF-DH-2D, InF-SH-3D, InF-DH-3D, IOO.

- Frequency Bands: FR1, FR2.

- Positioning Techniques: R.16 DL-TDOA, R.16 UL-TDOA, R.16 Multi-RTT, R.16 DL-AOD, R.16 UL-TDOA +R.16 UL-AOA.

Note 1：

- InF-SH-2D: 300x150 m, Horizontal: gNB antenna height (8m), UE antenna height (1.5m), low clutter density {20%, 2m, 10m}.

- InF-SH-3D: 300x150 m, Horizontal and vertical: gNB antenna height (4m+8m), UE antenna height uniformly distributed [0.5m, 2m], low clutter density {20%, 2m, 10m}

- InF-DH-2D: 120x60 m, Horizontal: gNB antenna height (8m), UE antenna height (1.5m), high clutter density #1{40%, 2m, 2m}

- InF-DH-3D: 120x60 m, Horizontal and vertical: gNB antenna height (4m+8m), UE antenna height uniformly distributed [0.5m, 3m], high clutter density #2{40%, 3m, 5m}.

Table C.1.1.3.3-1a: Rel.16 NR positioning - evaluation scenarios and parameters [InF-SH-FR1]

| Parameter | [Case 1], [InF-SH-2D], [FR1], [DL-TDOA] | [Case 2], [InF- SH-2D], [FR1], [UL-TDOA] | [Case 3], [InF- SH-2D], [FR1], [Multi-RTT] | [Case 4], [InF-SH-3D], [FR1], [DL-TDOA] | [Case 5], [InF- SH-3D], [FR1], [UL-TDOA] | [Case 6], [InF-SH-3D], [FR1], [UL-TDOA+UL-AOA] | [Case 7], [InF-SH-3D], [FR1], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline) | InF-SH-2D | InF-SH-2D | InF-SH-2D | InF-SH-3D | InF-SH-3D | InF-SH-3D | InF-SH-3D |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No | No | No | No |
| Additional Notes, if any |  |  |  |  |  |  |  |

Table C.1.1.3.3-1b: Rel.16 NR positioning - evaluation scenarios and parameters [InF-DH-FR1]

| Parameter | [Case 8], [InF-DH-2D], [FR1], [DL-TDOA] | [Case 9], [InF- DH-2D], [FR1], [UL-TDOA] | [Case 10], [InF-DH-2D], [FR1], [Multi-RTT] | [Case 11], [InF-DH-3D], [FR1], [DL-TDOA] | [Case 12], [InF-DH-3D], [FR1], [UL-TDOA] | [Case 13], [InF-DH-3D], [FR1], [UL-TDOA+UL-AOA] | [Case 14], [InF-DH-3D], [FR1], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline) | InF-DH-2D | InF-DH-2D | InF-DH-2D | InF-DH-3D | InF-DH-3D | InF-DH-3D | InF-DH-3D |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No | No |
| Precoding assumptions | No | No | No | No | No | No | No |
| Additional Notes, if any |  |  |  |  |  |  |  |

Table C.1.1.3.3-1c: Rel.16 NR positioning - evaluation scenarios and parameters [IOO-FR1]

| Parameter | [Case 15], [IOO], [FR1], [DL-TDOA] | [Case 16], [IOO], [FR1], [UL-TDOA] | [Case 17], [IOO], [FR1], [UL-TDOA+UL-AOA] | [Case 18], [IOO], [FR1], [Multi-RTT] | [Case 19], [IOO], [FR1], [DL-TDOA] | [Case 20], [IOO], [FR1], [UL-TDOA] | [Case 21], [IOO], [FR1], [UL-TDOA+UL-AOA] | [Case 22], [IOO], [FR1], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline) | IOO | IOO | IOO | IOO | IOO | IOO | IOO | IOO |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect | Not Perfect | Not Perfect | Not Perfect | Not Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 | Yes | Yes | Yes | Yes |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No | No | No | No | No |
| Additional Notes, if any |  |  |  |  |  |  |  |  |

Table C.1.1.3.3-1d: Rel.16 NR positioning - evaluation scenarios and parameters [InF-SH-FR2]

| Parameter | [Case 23], [InF-SH-2D], [FR2], [DL-TDOA] | [Case 24], [InF- SH-2D], [FR2], [UL-TDOA] | [Case 25], [InF-SH-3D], [FR2], [DL-TDOA] | [Case 26], [InF- SH-3D], [FR2], [UL-TDOA] |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH-2D | InF-SH-2D | InF-SH-3D | InF-SH-3D |
| Carrier frequency | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 |
| Number of sites | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78dB | 9dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm | R.16 DL-TDOA | R.16 UL-TDOA | R.16 DL-TDOA | R.16 UL-TDOA |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No |
| Additional Notes, if any |  |  |  |  |

Table C.1.1.3.3-1e: Rel.16 NR positioning - evaluation scenarios and parameters [InF-DH-FR2]

| Parameter | [Case 27], [InF-DH-2D], [FR2], [DL-TDOA] | [Case 28], [InF-DH-2D], [FR2], [UL-TDOA] | [Case 29], [InF-DH-3D], [FR2], [DL-TDOA] | [Case 30], [InF-DH-3D], [FR2], [UL-TDOA] |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH-2D | InF-DH-2D | InF-DH-3D | InF-DH-3D |
| Carrier frequency | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 |
| Number of sites | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78dB | 9dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 DL-TDOA | R.16 UL-TDOA |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No |
| Additional Notes, if any |  |  |  |  |

Table C.1.1.3.3-1f: Rel.16 NR positioning - evaluation scenarios and parameters [IOO-FR2]

| Parameter | [Case 31], [IOO], [FR2], [DL-TDOA] | [Case 32], [IOO], [FR2], [UL-TDOA] | [Case 33], [IOO], [FR2], [Multi-RTT] | [Case 34], [IOO], [FR2], [DL-TDOA] | [Case 35], [IOO], [FR2], [UL-TDOA] | [Case 36], [IOO], [FR2], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | IOO | IOO | IOO | IOO | IOO | IOO |
| Carrier frequency | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Not Perfect | Not Perfect | Not Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | Yes | Yes | Yes |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No | No | No |
| Additional Notes, if any |  |  |  |  |  |  |

Table C.1.1.3.3-1a: Rel.16 NR positioning - evaluation scenarios and parameters [InF-SH-FR1]

| Parameter | [Case 1], [InF-SH-2D], [FR1], [DL-TDOA] | [Case 2], [InF- SH-2D], [FR1], [UL-TDOA] | [Case 3], [InF- SH-2D], [FR1], [Multi-RTT] | [Case 4], [InF-SH-3D], [FR1], [DL-TDOA] | [Case 5], [InF- SH-3D], [FR1], [UL-TDOA] | [Case 6], [InF-SH-3D], [FR1], [UL-TDOA+UL-AOA] | [Case 7], [InF-SH-3D], [FR1], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline) | InF-SH-2D | InF-SH-2D | InF-SH-2D | InF-SH-3D | InF-SH-3D | InF-SH-3D | InF-SH-3D |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No | No | No | No |
| Additional Notes, if any |  |  |  |  |  |  |  |

Table C.1.1.3.3-1b: Rel.16 NR positioning - evaluation scenarios and parameters [InF-DH-FR1]

| Parameter | [Case 8], [InF-DH-2D], [FR1], [DL-TDOA] | [Case 9], [InF- DH-2D], [FR1], [UL-TDOA] | [Case 10], [InF-DH-2D], [FR1], [Multi-RTT] | [Case 11], [InF-DH-3D], [FR1], [DL-TDOA] | [Case 12], [InF-DH-3D], [FR1], [UL-TDOA] | [Case 13], [InF-DH-3D], [FR1], [UL-TDOA+UL-AOA] | [Case 14], [InF-DH-3D], [FR1], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline) | InF-DH-2D | InF-DH-2D | InF-DH-2D | InF-DH-3D | InF-DH-3D | InF-DH-3D | InF-DH-3D |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No | No |
| Precoding assumptions | No | No | No | No | No | No | No |
| Additional Notes, if any |  |  |  |  |  |  |  |

Table C.1.1.3.3-1c: Rel.16 NR positioning - evaluation scenarios and parameters [IOO-FR1]

| Parameter | [Case 15], [IOO], [FR1], [DL-TDOA] | [Case 16], [IOO], [FR1], [UL-TDOA] | [Case 17], [IOO], [FR1], [UL-TDOA+UL-AOA] | [Case 18], [IOO], [FR1], [Multi-RTT] | [Case 19], [IOO], [FR1], [DL-TDOA] | [Case 20], [IOO], [FR1], [UL-TDOA] | [Case 21], [IOO], [FR1], [UL-TDOA+UL-AOA] | [Case 22], [IOO], [FR1], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline) | IOO | IOO | IOO | IOO | IOO | IOO | IOO | IOO |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect | Not Perfect | Not Perfect | Not Perfect | Not Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 | Yes | Yes | Yes | Yes |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No | No | No | No | No |
| Additional Notes, if any |  |  |  |  |  |  |  |  |

Table C.1.1.3.3-1d: Rel.16 NR positioning - evaluation scenarios and parameters [InF-SH-FR2]

| Parameter | [Case 23], [InF-SH-2D], [FR2], [DL-TDOA] | [Case 24], [InF- SH-2D], [FR2], [UL-TDOA] | [Case 25], [InF-SH-3D], [FR2], [DL-TDOA] | [Case 26], [InF- SH-3D], [FR2], [UL-TDOA] |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH-2D | InF-SH-2D | InF-SH-3D | InF-SH-3D |
| Carrier frequency | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 |
| Number of sites | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78dB | 9dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm | R.16 DL-TDOA | R.16 UL-TDOA | R.16 DL-TDOA | R.16 UL-TDOA |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No |
| Additional Notes, if any |  |  |  |  |

Table C.1.1.3.3-1e: Rel.16 NR positioning - evaluation scenarios and parameters [InF-DH-FR2]

| Parameter | [Case 27], [InF-DH-2D], [FR2], [DL-TDOA] | [Case 28], [InF-DH-2D], [FR2], [UL-TDOA] | [Case 29], [InF-DH-3D], [FR2], [DL-TDOA] | [Case 30], [InF-DH-3D], [FR2], [UL-TDOA] |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH-2D | InF-DH-2D | InF-DH-3D | InF-DH-3D |
| Carrier frequency | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 |
| Number of sites | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78dB | 9dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 DL-TDOA | R.16 UL-TDOA |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No |
| Additional Notes, if any |  |  |  |  |

Table C.1.1.3.3-1f: Rel.16 NR positioning - evaluation scenarios and parameters [IOO-FR2]

| Parameter | [Case 31], [IOO], [FR2], [DL-TDOA] | [Case 32], [IOO], [FR2], [UL-TDOA] | [Case 33], [IOO], [FR2], [Multi-RTT] | [Case 34], [IOO], [FR2], [DL-TDOA] | [Case 35], [IOO], [FR2], [UL-TDOA] | [Case 36], [IOO], [FR2], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | IOO | IOO | IOO | IOO | IOO | IOO |
| Carrier frequency | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Not Perfect | Not Perfect | Not Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | Yes | Yes | Yes |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No | No | No |
| Additional Notes, if any |  |  |  |  |  |  |

##### C.1.1.3.2	Positioning accuracy evaluation results

Evaluation results are presented from Table C.1.1.3.2-1 to Table C.1.1.3.2-2.

Table C.1.1.3.2-1 provides summary of NR positioning evaluations results for horizontal location error.

Table C.1.1.3.2-1: Rel.16 NR positioning - horizontal location error results from [8]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 1], [InF-SH-2D], [FR1], [DL-TDOA] | Convex UEs | 0.0418 | 0.0603 | 0.0836 | 0.1650 |
|  | (Optional) All UEs | 0.0996 | 0.104 | 0.1196 | 0.3159 |
| [Case 2], [InF-SH-2D], [FR1], [UL-TDOA] | Convex UEs | 0.0430 | 0.0621 | 0.0823 | 0.1551 |
| [Case 3], [InF-SH-2D], [FR1], [Multi-RTT] | Convex UEs | 0.03969 | 0.05828 | 0.08561 | 0.1650 |
| [Case 4], [InF-SH-3D], [FR1], [DL-TDOA] | Convex UEs | 0.079 | 0.1083 | 0.1379 | 0.2045 |
| [Case 5], [InF-SH-3D], [FR1], [UL-TDOA] | Convex UEs | 0.0754 | 0.1143 | 0.1728 | 0.2574 |
| [Case 6], [InF-SH-3D], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.033 | 0.054 | 0.1428 | 0.2677 |
| [Case 7], [InF-SH-3D], [FR1], [Multi-RTT] | Convex UEs | 0.08 | 0.108 | 0.153 | 0.2540 |
| [Case 8], [InF-DH-2D], [FR1], [DL-TDOA] | Convex UEs | 0.0599 | 0.088 | 0.1185 | 0.1693 |
| [Case 9], [InF-DH-2D], [FR1], [UL-TDOA] | Convex UEs | 0.0488 | 0.0695 | 0.0856 | 0.1184 |
| [Case 10], [InF-DH-2D], [FR1], [Multi-RTT] | Convex UEs | 0.04744 | 0.06356 | 0.08704 | 0.1237 |
| [Case 11], [InF-DH-3D], [FR1], [DL-TDOA] | Convex UEs | 0.1201 | 0.2499 | 0.4116 | 0.7089 |
| [Case 12], [InF-DH-3D], [FR1], [UL-TDOA] | Convex UEs | 0.108 | 0.155 | 0.3692 | 0.6937 |
| [Case 13], [InF-DH-3D], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.044 | 0.062 | 0.118 | 0.151 |
| [Case 14], [InF-DH-3D], [FR1], [Multi-RTT] | Convex UEs | 0.115 | 0.179 | 0.391 | 0.692 |
| [Case 15], [IOO], [FR1], [DL-TDOA] | Convex UEs | 0.079 | 0.1028 | 0.1338 | 0.2288 |
| [Case 16], [IOO], [FR1], [UL-TDOA] | Convex UEs | 0.0646 | 0.0984 | 0.127 | 0.1836 |
| [Case 17], [IOO], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.013 | 0.031 | 0.049 | 0.1219 |
| [Case 18], [IOO], [FR1], [Multi-RTT] | Convex UEs | 0.073 | 0.1024 | 0.1331 | 0.283 |
| [Case 19], [IOO], [FR1], [DL-TDOA] | Convex UEs | 7.4192 | 13.0891 | 18.592 | 32.4509 |
| [Case 20], [IOO], [FR1], [UL-TDOA] | Convex UEs | 6.6813 | 12.6521 | 18.281 | 32.0927 |
| [Case 21], [IOO], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 2.4591 | 3.7312 | 5.603 | 9.2356 |
| [Case 22], [IOO], [FR1], [Multi-RTT] | Convex UEs | 0.3828 | 0.4918 | 0.6728 | 1.3668 |
| [Case 23], [InF-SH-2D], [FR2], [DL-TDOA] | Convex UEs | 0.0136 | 0.0223 | 0.028 | 0.0372 |
| [Case 24], [InF-SH-2D], [FR2], [UL-TDOA] | Convex UEs | 0.022 | 0.0296 | 0.041 | 0.0538 |
| [Case 25], [InF-SH-3D], [FR2], [DL-TDOA] | Convex UEs | 0.025 | 0.0324 | 0.046 | 0.0789 |
| [Case 26], [InF-SH-3D], [FR2], [UL-TDOA] | Convex UEs | 0.0223 | 0.0276 | 0.0383 | 0.0817 |
| [Case 27], [InF-DH-2D], [FR2], [DL-TDOA] | Convex UEs | 0.0178 | 0.023 | 0.0277 | 0.0388 |
| [Case 28], [InF-DH-2D], [FR2], [UL-TDOA] | Convex UEs | 0.0204 | 0.0296 | 0.0416 | 0.0553 |
| [Case 29], [InF-DH-3D], [FR2], [DL-TDOA] | Convex UEs | 0.0314 | 0.089 | 0.2996 | 0.7033 |
| [Case 30], [InF-DH-3D], [FR2], [UL-TDOA] | Convex UEs | 0.0282 | 0.0561 | 0.2827 | 0.6848 |
| [Case 31], [IOO], [FR2], [DL-TDOA] | Convex UEs | 0.0166 | 0.0225 | 0.0301 | 0.0406 |
| [Case 32], [IOO], [FR2], [UL-TDOA] | Convex UEs | 0.011 | 0.0179 | 0.0243 | 0.0397 |
| [Case 33], [IOO], [FR2], [Multi-RTT] | Convex UEs | 0.01636 | 0.022 | 0.03 | 0.0402 |
| [Case 34], [IOO], [FR2], [DL-TDOA] | Convex UEs | 7.3189 | 13.0765 | 18.572 | 32.3809 |
| [Case 35], [IOO], [FR2], [UL-TDOA] | Convex UEs | 6.6798 | 12.5968 | 18.2581 | 32.0887 |
| [Case 36], [IOO], [FR2], [Multi-RTT] | Convex UEs | 0.3475 | 0.4718 | 0.6565 | 1.2681 |

Table C.1.1.3.2-2 provides summary of NR positioning evaluations results for vertical location error.

Table C.1.1.3.2-2: Rel.16 NR positioning - altitude location error results from [8]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 4], [InF-SH-3D], [FR1], [DL-TDOA] | Convex UEs | 0.608 | 0.9636 | 1.3383 | 1.8954 |
| [Case 5], [InF-SH-3D], [FR1], [UL-TDOA] | Convex UEs | 0.608 | 1.036 | 1.3383 | 2.0599 |
| [Case 6], [InF-SH-3D], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.8572 | 1.154 | 1.516 | 2.256 |
| [Case 7], [InF-SH-3D], [FR1], [Multi-RTT] | Convex UEs | 0.639 | 0.917 | 1.60 | 2.18 |
| [Case 11], [InF-DH-3D], [FR1], [DL-TDOA] | Convex UEs | 0.461 | 0.8138 | 1.3474 | 2.9917 |
| [Case 12], [InF- DH-3D], [FR1], [UL-TDOA] | Convex UEs | 0.4167 | 0.6408 | 1.0091 | 2.049 |
| [Case 13], [InF- DH-3D], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.334 | 0.486 | 0.593 | 0.633 |
| [Case 14], [InF- DH-3D], [FR1], [Multi-RTT] | Convex UEs | 0.507 | 0.723 | 1.59 | 2.82 |
| [Case 25], [InF-SH-3D], [FR2], [DL-TDOA] | Convex UEs | 0.1581 | 0.277 | 0.3961 | 0.6283 |
| [Case 26], [InF- SH-3D], [FR2], [UL-TDOA] | Convex UEs | 0.2018 | 0.3084 | 0.4611 | 0.8304 |
| [Case 29], [InF-DH-3D], [FR2], [DL-TDOA] | Convex UEs | 0.1337 | 0.2703 | 0.6779 | 3.0578 |
| [Case 30], [InF-DH-3D], [FR2], [UL-TDOA] | Convex UEs | 0.116 | 0.2358 | 0.5051 | 3.1267 |

#### C.1.1.4	Results from source [13]

##### C.1.1.4.1	Description of evaluation scenarios

In this section results are provided for the InF-SH, InF-DH, IOO, and UMi scenarios. In addition, simulations for phase center offset errors were performed.

Table C.1.1.4.1-1 Description of evaluation scenarios

| Parameter | Case 1 [InF-SH, DL-TDOA, FR1, 100 MHz] | Case 2 [InF-DH, DL-TDOA, FR1, 100 MHz] | Case 3 [IOO, DL-TDOA, FR1, 100 MHz] | Case 4 [IOO, DL-TDOA, FR1, 100 MHz] | Case 5 [UMi, DL-TDOA, FR1, 100 MHz] | Case 6 [InF-SH, DL-TDOA, FR1, 100 MHz] | Case 7 [InF-DH, DL-TDOA, FR1, 100 MHz] | Case 8 [Phase Center Offset, FR2] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline | Baseline | Baseline | Baseline | Baseline |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Comb-6, 6 symbol NR PRS | Comb-6, 6 symbol NR PRS | Comb-6, 6 symbol NR PRS | Comb-6, 6 symbol NR PRS | Comb-6, 6 symbol NR PRS | Comb-6, 6 symbol NR PRS | Comb-6, 6 symbol NR PRS | Comb-2, 2 symbol SRS-Pos |
| Reference signal (type of sequence, number of ports, …) | NR PRS, single port | NR PRS, single port | NR PRS, single port | NR PRS, single port | NR PRS, single port | NR PRS, single port | NR PRS, single port | SRS-Pos |
| Number of sites | 18 | 18 | 18 | 18 | 18 | 18 | 18 | Not applicable |
| Number of symbols used per slot per positioning estimate | 6 symbols per slot | 6 symbols per slot | 6 symbols per slot | 6 symbols per slot | 6 symbols per slot | 6 symbols per slot | 6 symbols per slot | Not applicable |
| Number of slots per positioning estimate | 1 slot | 1 slot | 1 slot | 1 slot | 1 slot | 1 slot | 1 slot | 1 slot |
| Power-boosting level | 6 dB | 6 dB | 6 dB | 6 dB | 6 dB | 6 dB | 6 dB | 0 dB |
| Uplink power control (applied/not applied) | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Thresholding, 0.5, (No oversampling) | Thresholding, 0.5, (No oversampling) | Thresholding, 0.5, (Oversampling x4) | Thresholding, 0.5, (No oversampling) | Thresholding, 0.5, (No oversampling) | Thresholding, 0.5, (Oversampling x4) | Thresholding, 0.5, (Oversampling x4) | Not applicable |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Taylor Series, Least Squares | Taylor Series, Least Squares | Series, Least Square | Taylor Series, Least Squares | Taylor Series, Least Squares | Taylor Series, Least Squares | Taylor Series, Least Squares | Not applicable |
| Network synchronization assumptions | Perfect Synchronization | Perfect Synchronization | Perfect Synchronization | Perfect Synchronization | Perfect Synchronization | Perfect Synchronization | Perfect Synchronization | Perfect Syncrhonization |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nr of antenna elements used, etc) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Addiitonal Notes, if any |  |  |  |  |  |  |  | Test UE with 1:8 element antenna array (both co-pol and cross-pol) was simulated for different AoA and beam steering angles. |

##### C.1.1.4.2	Positioning accuracy evaluation results

Table C.1.1.4.2-1 Positioning accuracy evaluation results

| Case | UEs | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 1, [InF-SH, DL-TDOA, FR1] | Convex UEs | 0.98 | 1.47 | 2.13 | 4.35 |
| Case 2, [InF-DH, DL-TDOA, FR1] | Convex UEs | 1.71 | 3.15 | 4.39 | 7.16 |
| Case 3, [IOO, DL-TDOA, FR1] | All UEs | 0.46 | 0.82 | 1.68 | 4.31 |
| Case 4, [IOO, DL-TDOA, FR1] | All UEs | 1.17 | 1.92 | 3.24 | 6.50 |
| Case 5, [UMi, DL-TDOA, FR1] | All UEs | 5.29 | 9.59 | 14.92 | 23.81 |
| Case 6, [InF-SH, DL-TDOA, FR1] | Convex UEs | 0.26 | 0.38 | 0.66 | 1.65 |
| Case 7, [InF-DH, DL-TDOA, FR1] | Convex UEs | 1.29 | 2.04 | 3.34 | 4.99 |

For Case 8 the results depict the errors arising from phase center offsets and are shown in Figure C.1.1.4-1.

Figure C.1.1.4-1: Positioning error simulation results.

#### C.1.1.5 Results from source [5]

##### C.1.1.5.1	Description of evaluation scenarios

Evaluation assumptions for system level analysis are provided in Table C.1.1.5.1-1.1- Table C.1.1.5.1-1.8.

Table C.1.1.5.1-1: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case 1] (InF-SH, FR1) | [Case 2] (InF-SH, FR1) | [Case 3] (InF-SH, FR2) | [Case 4] (InF-SH, FR2) | [Case 5] (InF-DH, FR1) | [Case 6] (InF-DH, FR1) | [Case 7] (InF-DH, FR2) | [Case 8] (InF-DH, FR2) | [Case 9] (InF-DH, FR1) | [Case 10] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (60%, 6, 2) | InF-DH (60%, 6, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) |  |  |  |  |  |  |  |  |  |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence |  |  |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |  |  |

Table C.1.1.5.1-2: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case 11] (InF-SH, FR1) | [Case 12] (InF-SH, FR1) | [Case 13] (InF-SH, FR2) | [Case 14] (InF-SH, FR2) | [Case 15] (InF-DH, FR1) | [Case 16] (InF-DH, FR1) | [Case 17] (InF-DH, FR2) | [Case 18] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) |  |  |  |  |  |  |  |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.1.1.5.1-3: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case 19] (InF-SH, FR1) | [Case 20] (InF-SH, FR1) | [Case 21] (InF-SH, FR2) | [Case 22] (InF-SH, FR2) | [Case 23] (InF-DH, FR1) | [Case 24] (InF-DH, FR1) | [Case 25] (InF-DH, FR2) | [Case 26] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.1.1.5.1-4: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case 27] (InF-SH, FR1) | [Case 28] (InF-SH, FR1) | [Case 29] (InF-SH, FR2) | [Case 30] (InF-SH, FR2) | [Case 31] (InF-DH, FR1) | [Case 32] (InF-DH, FR1) | [Case 33] (InF-DH, FR2) | [Case 34] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.1.1.5.1-5: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case 35] (InF-SH, FR1) | [Case 36] (InF-SH, FR1) | [Case 37] (InF-DH, FR1) | [Case 38] (InF-DH, FR1) | [Case 39] (InF-SH, FR1) | [Case 40] (InF-SH, FR1) | [Case 41] (InF-DH, FR1) | [Case 42] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-AOA Least square first/median peak | UL-AOA Least square first/median peak | UL-AOA Least square first/median peak | UL-AOA Least square first/median peak | UL-TDOA+AOA Least square first/median peak | UL-TDOA+AOA Least square first/median peak | UL-TDOA+AOA Least square first/median peak | UL-TDOA+AOA Least square first/median peak |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.1.1.5.1-6: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case 43] (InF-SH, FR1) | [Case 44] (InF-SH, FR1) | [Case 45] (InF-SH, FR2) | [Case 46] (InF-SH, FR2) | [Case 47] (InF-DH, FR1) | [Case 48] (InF-DH, FR1) | [Case 49] (InF-DH, FR2) | [Case 50] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN + 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | DL:7.78dB UL:6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.1.1.5.1-7: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case 51] (InF-SH, FR1) | [Case 52] (InF-SH, FR1) | [Case 53] (InF-SH, FR2) | [Case 54] (InF-SH, FR2) | [Case 55] (InF-DH, FR1) | [Case 56] (InF-DH, FR1) | [Case 57] (InF-DH, FR2) | [Case 58] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN + 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | DL:7.78dB UL:6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.1.1.5.1-8: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case V1] (InF-SH, FR1) | [Case V2] (InF-DH, FR1) | [Case V3] (InF-SH, FR1) | [Case V4] (InF-DH, FR1) | [Case V5] (InF-SH, FR1) | [Case V6] (InF-DH, FR1) | [Case V7] (InF-SH, FR1) | [Case V8] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH (BS height = 8m UE height =1.5m) | InF-DH (40%, 2, 2) (BS height = 8m UE height =1.5m) | InF-SH (BS height = {4,8}m UE height =1.5m) | InF-DH (40%, 2, 2) (BS height = {4,8}m UE height =1.5m) | InF-SH (BS height =8m UE height =[0.5,2]m) | InF-DH (40%, 2, 2), (BS height =8m UE height =[0.5,2]m) | InF-SH (BS height = {4,8}m UE height =[0.5,2]m) | InF-DH (40%, 2, 2) (BS height = {4,8}m UE height =[0.5,2]m) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-AOA+ZOA taylor series first/median peak | UL-AOA+ZOA taylor series first/median peak | UL-AOA+ZOA taylor series first/median peak | UL-AOA+ZOA taylor series first/median peak | UL-AOA+ZOA taylor series first/median peak | UL-AOA+ZOA taylor series first/median peak | UL-AOA+ZOA taylor series first/median peak | UL-AOA+ZOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |

##### C.1.1.5.2	Positioning accuracy evaluation results

Table C.1.1.5.2-1 provides summary of NR positioning evaluations results for horizontal location error.

Table C.1.1.5.2-1: Rel.16 NR positioning - horizontal location error results from [5]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 1], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.041 | 0.055 | 0.099 | 4.15 |
|  | (Optional) All UEs | 0.059 | 0.17 | 1.40 | 6.73 |
| [Case 2], [SH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 11.49 | 14.48 | 19.44 | 24.06 |
|  | (Optional) All UEs | 13.41 | 18.91 | 25.07 | 35.48 |
| [Case 3], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.010 | 0.013 | 0.021 | 2.97 |
|  | (Optional) All UEs | 0.014 | 0.029 | 0.35 | 4.82 |
| [Case 4], [SH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 11.67 | 13.44 | 17.49 | 23.21 |
|  | (Optional) All UEs | 13.45 | 17.71 | 23.46 | 33.34 |
| [Case 5], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.056 | 0.33 | 2.97 | 5.92 |
|  | (Optional) All UEs | 0.10 | 1.23 | 4.60 | 9.80 |
| [Case 6], [DH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 9.95 | 13.09 | 16.43 | 23.79 |
|  | (Optional) All UEs | 12.65 | 16.29 | 20.59 | 30.46 |
| [Case 7], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.012 | 0.23 | 2.72 | 5.77 |
|  | (Optional) All UEs | 0.023 | 1.06 | 4.08 | 9.67 |
| [Case 8], [DH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 9.39 | 12.89 | 15.05 | 22.90 |
|  | (Optional) All UEs | 11.79 | 14.79 | 17.49 | 22.93 |
| [Case 9], [DH {0.6,6,2}, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 6.13 | 7.76 | 13.38 | 18.71 |
|  | (Optional) All UEs | 6.49 | 10.35 | 15.33 | 24.59 |
| [Case 10], [DH {0.6,6,2}, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 6.63 | 9.01 | 13.6 | 15.09 |
|  | (Optional) All UEs | 8.04 | 12.84 | 15.23 | 23.12 |
| [Case 11], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.038 | 0.049 | 0.069 | 0.094 |
|  | (Optional) All UEs | 0.052 | 0.080 | 0.15 | 0.30 |
| [Case 12], [SH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 11.20 | 14.93 | 18.32 | 26.09 |
|  | (Optional) All UEs | 13.82 | 18.12 | 23.77 | 36.35 |
| [Case 13], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.011 | 0.013 | 0.017 | 0.031 |
|  | (Optional) All UEs | 0.013 | 0.020 | 0.046 | 0.11 |
| [Case 14], [SH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 10.12 | 13.01 | 17.49 | 25.67 |
|  | (Optional) All UEs | 12.27 | 17.54 | 23.28 | 32.26 |
| [Case 15], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.062 | 0.079 | 0.11 | 0.60 |
|  | (Optional) All UEs | 0.093 | 0.15 | 0.25 | 3.23 |
| [Case 16], [DH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 10.14 | 13.00 | 14.93 | 20.30 |
|  | (Optional) All UEs | 12.24 | 15.42 | 20.30 | 31.59 |
| [Case 17], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.011 | 0.015 | 0.018 | 0.049 |
|  | (Optional) All UEs | 0.014 | 0.023 | 0.041 | 0.60 |
| [Case 18], [DH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 9.71 | 12.14 | 15.19 | 20.16 |
|  | (Optional) All UEs | 11.36 | 14.65 | 17.97 | 25.55 |
| [Case 19], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.041 | 0.051 | 0.087 | 4.22 |
|  | (Optional) All UEs | 0.057 | 0.13 | 1.40 | 6.68 |
| [Case 20], [SH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 11.50 | 14.77 | 19.49 | 24.51 |
|  | (Optional) All UEs | 13.81 | 19.44 | 26.32 | 35.51 |
| [Case 21], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.010 | 0.013 | 0.033 | 4.07 |
|  | (Optional) All UEs | 0.015 | 0.036 | 0.54 | 5.61 |
| [Case 22], [SH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 11.70 | 13.82 | 18.11 | 23.21 |
|  | (Optional) All UEs | 13.65 | 18.95 | 23.67 | 33.34 |
| [Case 23], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.060 | 0.19 | 2.77 | 5.85 |
|  | (Optional) All UEs | 0.10 | 1.04 | 4.48 | 9.14 |
| [Case 24], [DH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 9.92 | 13.47 | 16.02 | 22.90 |
|  | (Optional) All UEs | 12.31 | 15.40 | 19.04 | 23.85 |
| [Case 25], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.013 | 0.35 | 2.85 | 5.76 |
|  | (Optional) All UEs | 0.025 | 1.09 | 4.60 | 9.95 |
| [Case 26], [DH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 10.52 | 13.46 | 16.09 | 18.92 |
|  | (Optional) All UEs | 12.22 | 15.26 | 18.72 | 23.64 |
| [Case 27], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.038 | 0.055 | 0.070 | 0.087 |
|  | (Optional) All UEs | 0.055 | 0.078 | 0.13 | 0.29 |
| [Case 28], [SH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 11.23 | 15.14 | 20.06 | 27.70 |
|  | (Optional) All UEs | 13.95 | 18.53 | 24.10 | 36.35 |
| [Case 29], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.011 | 0.013 | 0.017 | 0.032 |
|  | (Optional) All UEs | 0.013 | 0.020 | 0.038 | 0.11 |
| [Case 30], [SH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 11.11 | 14.30 | 18.27 | 25.67 |
|  | (Optional) All UEs | 13.38 | 18.28 | 23.63 | 32.51 |
| [Case 31], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.058 | 0.075 | 0.11 | 0.60 |
|  | (Optional) All UEs | 0.088 | 0.16 | 0.26 | 2.83 |
| [Case 32], [DH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 10.14 | 13.35 | 16.32 | 22.01 |
|  | (Optional) All UEs | 12.14 | 15.29 | 18.76 | 27.05 |
| [Case 33], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.010 | 0.014 | 0.019 | 0.051 |
|  | (Optional) All UEs | 0.015 | 0.023 | 0.052 | 0.89 |
| [Case 34], [DH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 9.93 | 13.20 | 16.64 | 19.74 |
|  | (Optional) All UEs | 12.15 | 15.34 | 18.35 | 24.03 |
| [Case 35], [SH, perfect sync], [FR1], [UL-AOA] | Convex UEs | 1.25 | 2.15 | 3.28 | 5.93 |
|  | (Optional) All UEs | 1.35 | 2.21 | 3.38 | 6.08 |
| [Case 36], [SH, sync error 50ns], [FR1], [UL-AOA] | Convex UEs | 1.31 | 2.24 | 3.75 | 6.20 |
|  | (Optional) All UEs | 1.43 | 2.41 | 4.02 | 6.47 |
| [Case 37], [DH, perfect sync], [FR1], [UL-AOA] | Convex UEs | 2.28 | 3.39 | 4.29 | 5.48 |
|  | (Optional) All UEs | 2.45 | 3.63 | 4.70 | 6.86 |
| [Case 38], [DH, sync error 50ns], [FR1], [UL-AOA] | Convex UEs | 2.45 | 3.56 | 4.45 | 5.76 |
|  | (Optional) All UEs | 2.63 | 3.68 | 4.75 | 7.06 |
| [Case 39], [SH, perfect sync], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.067 | 0.11 | 0.21 | 0.41 |
|  | (Optional) All UEs | 0.078 | 0.14 | 0.25 | 0.41 |
| [Case 40], [SH, sync error 50ns], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.098 | 0.17 | 0.26 | 0.43 |
|  | (Optional) All UEs | 0.11 | 0.21 | 0.33 | 0.63 |
| [Case 41], [DH, perfect sync], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.047 | 0.094 | 0.18 | 0.68 |
|  | (Optional) All UEs | 0.055 | 0.11 | 0.41 | 4.12 |
| [Case 42], [DH, sync error 50ns], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.059 | 0.10 | 0.26 | 0.77 |
|  | (Optional) All UEs | 0.083 | 0.17 | 0.44 | 4.23 |
| [Case 43], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.039 | 0.050 | 0.095 | 4.25 |
|  | (Optional) All UEs | 0.054 | 0.12 | 1.34 | 6.70 |
| [Case 44], [SH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.042 | 0.055 | 0.11 | 4.71 |
|  | (Optional) All UEs | 0.060 | 0.14 | 1.49 | 6.72 |
| [Case 45], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.010 | 0.011 | 0.033 | 3.96 |
|  | (Optional) All UEs | 0.014 | 0.027 | 0.43 | 5.19 |
| [Case 46], [SH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.010 | 0.013 | 0.034 | 4.13 |
|  | (Optional) All UEs | 0.014 | 0.037 | 0.50 | 5.87 |
| [Case 47], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.068 | 0.13 | 2.89 | 5.88 |
|  | (Optional) All UEs | 0.092 | 1.09 | 4.55 | 9.82 |
| [Case 48], [DH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.068 | 0.23 | 3.18 | 6.20 |
|  | (Optional) All UEs | 0.091 | 1.34 | 5.49 | 10.71 |
| [Case 49], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.013 | 0.28 | 2.61 | 5.74 |
|  | (Optional) All UEs | 0.024 | 1.04 | 3.31 | 9.24 |
| [Case 50], [DH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.010 | 0.30 | 2.80 | 6.23 |
|  | (Optional) All UEs | 0.025 | 1.51 | 4.53 | 10.10 |
| [Case 51], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.040 | 0.050 | 0.069 | 0.10 |
|  | (Optional) All UEs | 0.052 | 0.074 | 0.12 | 0.25 |
| [Case 52], [SH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.042 | 0.058 | 0.069 | 0.10 |
|  | (Optional) All UEs | 0.048 | 0.080 | 0.15 | 0.32 |
| [Case 53], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.010 | 0.014 | 0.017 | 0.031 |
|  | (Optional) All UEs | 0.013 | 0.020 | 0.037 | 0.11 |
| [Case 54], [SH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.010 | 0.012 | 0.017 | 0.030 |
|  | (Optional) All UEs | 0.013 | 0.021 | 0.039 | 0.10 |
| [Case 55], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.061 | 0.082 | 0.11 | 0.60 |
|  | (Optional) All UEs | 0.076 | 0.12 | 0.20 | 3.13 |
| [Case 56], [DH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.063 | 0.092 | 0.12 | 0.78 |
|  | (Optional) All UEs | 0.087 | 0.13 | 0.28 | 4.72 |
| [Case 57], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.010 | 0.013 | 0.018 | 0.051 |
|  | (Optional) All UEs | 0.014 | 0.022 | 0.039 | 0.63 |
| [Case 58], [DH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.011 | 0.015 | 0.021 | 0.055 |
|  | (Optional) All UEs | 0.016 | 0.023 | 0.041 | 0.69 |

Table C.1.1.5.2-2 provides summary of NR positioning evaluations results for vertical location error.

Table C.1.1.5.2-2: Rel.16 NR positioning - altitude location error results from [23]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case V1], [SH, perfect sync], [FR1], [ BS height = 8m UE height =1.5m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | Convex UEs | 0.28 | 0.43 | 0.52 | 0.66 |
|  | (Optional) All UEs | 0.48 | 0.54 | 0.58 | 0.90 |
| [Case V2], [DH, perfect sync], [FR1], [ BS height = 8m UE height =1.5m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | Convex UEs | 0.48 | 0.63 | 0.83 | 1.12 |
|  | (Optional) All UEs | 0.44 | 0.63 | 0.83 | 1.27 |
| [Case V3], [SH, perfect sync], [FR1], [ BS height = {4,8}m UE height =1.5m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | Convex UEs | 0.30 | 0.38 | 0.64 | 0.82 |
|  | (Optional) All UEs | 0.33 | 0.48 | 0.68 | 0.98 |
| [Case V4], [DH, perfect sync], [FR1], [ BS height = {4,8}m UE height =1.5m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | Convex UEs | 0.43 | 0.64 | 0.90 | 1.39 |
|  | (Optional) All UEs | 0.46 | 0.74 | 1.13 | 3.61 |
| [Case V5], [SH, perfect sync], [FR1], [ BS height = 8m UE height =[0.5,2]m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | Convex UEs | 0.39 | 0.57 | 0.74 | 1.05 |
|  | (Optional) All UEs | 0.43 | 0.63 | 0.95 | 1.35 |
| [Case V6], [DH, perfect sync], [FR1], [ BS height = 8m UE height =[0.5,2]m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | Convex UEs | 2.07 | 3.19 | 4.83 | 5.46 |
|  | (Optional) All UEs | 2.58 | 4.15 | 4.89 | 5.46 |
| [Case V7], [SH, perfect sync], [FR1], [ BS height = {4,8}m UE height =[0.5,2]m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | Convex UEs | 0.45 | 0.62 | 0.96 | 1.21 |
|  | (Optional) All UEs | 0.45 | 0.70 | 1.06 | 1.33 |
| [Case V8], [DH, perfect sync], [FR1], [ BS height = {4,8}m UE height =[0.5,2]m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | Convex UEs | 3.80 | 4.39 | 4.93 | 9.06 |
|  | (Optional) All UEs | 3.83 | 4.44 | 5.09 | 9.06 |

#### C.1.1.6	Results from source [12]

##### C.1.1.6.1	Description of evaluation scenarios

The following scenarios are included for evaluate the performance of rel16 baseline:

- Baseline scenarios with InF-SH/InF-DH with fixed UE/gNB height and without UE/gNB calibration error

Evaluation assumptions for system level analysis are provided in Table C.1.1.6.1-1.

Table C.1.1.6.1-1: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | Case1 ( InF-SH, FR1) | Case2 ( InF-DH, FR1) |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-DH (40%,2,2) |
| Carrier frequency | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS | DL-PRS |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | Gold, single port |
| Number of sites | 18 | 18 |
| Number of symbols used per occasion | 1 | 1 |
| number of occasions used per positioning estimate | 1 | 1 |
| Power-boosting level | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | ML | ML |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA Chan | DL-TDOA Chan |
| Network synchronization assumptions | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides | alignment assumptions at the tx and rx sides |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based |
| Additional notes, if any |  |  |

##### C.1.1.6.2	Positioning accuracy evaluation results

Table C.1.1.6.2-1 provides summary of NR positioning evaluations results for horizontal location error.

Table C.1.1.1.2-1: Rel.16 NR positioning - horizontal location error results from [12]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case1, (InF-SH, FR1, DL-TDOA) | Convex UEs | 0.26 | 0.4 | 2.14 | 8.58 |
| Case2, (InF-DH, FR1, DL-TDOA) | Convex UEs | 1.86 | 4.95 | 8.75 | 14.9 |

#### C.1.1.7	Results from source [10]

##### C.1.1.7.1	Description of evaluation scenarios

Evaluation assumptions for system level analysis are provided in Table C.1.1.7.1-1.

Table C.1.1.7.1-1: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | Case 1, InF-SH, FR1, DL-TDOA | Case 2, InF-DH, FR1, DL-TDOA | Case 3, InF-SH, FR2, DL-TDOA | Case 4, InF-DH, FR2, DL-TDOA |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 3.5 GHz | 3.5 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 100 MHz | 400 MHz | 400 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS: Comb-2 | DL PRS: Comb-2 | DL PRS: Comb-2 | DL PRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | DL PRS: single port | DL PRS: single port | DL PRS: single port | DL PRS: single port |
| Number of sites | 1 | 1 | 3 | 3 |
| Number of symbols used per occasion | DL PRS: 2 symbols | DL PRS: 2 symbols | DL PRS: 2 symbols | DL PRS: 2 symbols |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | DL PRS: 3 dB | DL PRS: 3 dB | DL PRS: 3 dB | DL PRS: 3 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Ideal synchronization | Ideal synchronization | Ideal synchronization | Ideal synchronization |
| UE/gNB Tx/Rx Calibration Error | No timing calibration error | No timing calibration error | No timing calibration error | No timing calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping | No Tx/Rx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port | Single Tx port | Single Tx port | Single Tx port |
| Additional notes, if any |  |  |  |  |

| Parameter | Case 5, InF-SH, FR1, UL-TDOA | Case 6, InF-DH, FR1, UL-TDOA | Case 7, InF-SH, FR2, UL-TDOA | Case 8, InF-DH, FR2, UL-TDOA |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 3.5 GHz | 3.5 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 100 MHz | 400 MHz | 400 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | UL SRS: Comb-2 | UL SRS: Comb-2 | UL SRS: Comb-2 | UL SRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | UL SRS: single port | UL SRS: single port | UL SRS: single port | UL SRS: single port |
| Number of sites | 1 | 1 | 3 | 3 |
| Number of symbols used per occasion | UL SRS: 1 symbol | UL SRS: 1 symbol | UL SRS: 1 symbol | UL SRS: 1 symbol |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | UL SRS: 3 dB | UL SRS: 3 dB | UL SRS: 3 dB | UL SRS: 3 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Ideal synchronization | Ideal synchronization | Ideal synchronization | Ideal synchronization |
| UE/gNB Tx/Rx Calibration Error | No timing calibration error | No timing calibration error | No timing calibration error | No timing calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping | No Tx/Rx beam sweeping | Rx beam sweeping | Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port | Single Tx port | Single Tx port | Single Tx port |
| Additional notes, if any |  |  |  |  |

| Parameter | Case 9, InF-SH, FR1, Multi-RTT | Case 10, InF-DH, FR1, Multi-RTT | Case 11, InF-SH, FR2, Multi-RTT | Case 12, InF-DH, FR2, Multi-RTT |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 3.5 GHz | 3.5 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 100 MHz | 400 MHz | 400 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port |
| Number of sites | 1 | 1 | 3 | 3 |
| Number of symbols used per occasion | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Ideal synchronization | Ideal synchronization | Ideal synchronization | Ideal synchronization |
| UE/gNB Tx/Rx Calibration Error | No timing calibration error | No timing calibration error | No timing calibration error | No timing calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping | No Tx/Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port | Single Tx port | Single Tx port | Single Tx port |
| Additional notes, if any |  |  |  |  |

##### C.1.1.7.2	Positioning accuracy evaluation results

Table C.1.1.7.2-1 provides summary of NR positioning evaluations results for horizontal location error.

Table C.1.1.7.2-1: Rel.16 NR positioning - horizontal location error results from

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 1, InF-SH, FR1, DL-TDOA | Convex UEs | 0.16 | 0.25 | 0.45 | 0.85 |
| Case 2, InF-DH, FR1, DL-TDOA | Convex UEs | 1.04 | 2 | 3.6 | 6.2 |
| Case 3, InF-SH, FR2, DL-TDOA | Convex UEs | 0.1 | 0.19 | 0.36 | 0.65 |
| Case 4, InF-DH, FR2, DL-TDOA | Convex UEs | 0.87 | 1.9 | 5.17 | 17.3 |
| Case 5, InF-SH, FR1, UL-TDOA | Convex UEs | 0.15 | 0.25 | 0.42 | 0.77 |
| Case 6, InF-DH, FR1, UL-TDOA | Convex UEs | 0.97 | 1.7 | 4.7 | 6.13 |
| Case 7, InF-SH, FR2, UL-TDOA | Convex UEs | 0.6 | 0.09 | 0.27 | 0.9 |
| Case 8, InF-DH, FR2, UL-TDOA | Convex UEs | 0.87 | 2.08 | 3.6 | 16.9 |
| Case 9, InF-SH, FR1, Multi-RTT | Convex UEs | 0.07 | 0.11 | 0.14 | 0.25 |
| Case 10, InF-DH, FR1, Multi-RTT | Convex UEs | 1.35 | 4.2 | 9.6 | 16.3 |
| Case 11, InF-SH, FR2, Multi-RTT | Convex UEs | 0.06 | 0.09 | 0.27 | 0.9 |
| Case 12, InF-DH, FR2, Multi-RTT | Convex UEs | 0.44 | 1.45 | 3.2 | 7.72 |

Table C.1.1.7.2-2 provides summary of NR positioning evaluations results for vertical location error.

Table C.1.1.7.2-2: Rel.16 NR positioning - altitude location error results from [10]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 1, InF-SH, FR1, DL-TDOA | Convex UEs | 1.17 | 6.4 | 8.5 | 8.5 |
| Case 2, InF-DH, FR1, DL-TDOA | Convex UEs | 6.4 | 6.5 | 8.4 | 12.6 |
| Case 3, InF-SH, FR2, DL-TDOA | Convex UEs | 6.3 | 6.4 | 6.5 | 12.88 |
| Case 4, InF-DH, FR2, DL-TDOA | Convex UEs | 10.82 | 13.38 | 29.7 | 63.4 |
| Case 5, InF-SH, FR1, UL-TDOA | Convex UEs | 1.1 | 6.4 | 6.7 | 12.9 |
| Case 6, InF-DH, FR1, UL-TDOA | Convex UEs | 6.4 | 6.5 | 8.45 | 12.9 |
| Case 7, InF-SH, FR2, UL-TDOA | Convex UEs | 6.3 | 6.4 | 6.5 | 13 |
| Case 8, InF-DH, FR2, UL-TDOA | Convex UEs | 10.75 | 13.38 | 29.68 | 62.78 |
| Case 9, InF-SH, FR1, Multi-RTT | Convex UEs | 0.3 | 0.67 | 4.3 | 13.1 |
| Case 10, InF-DH, FR1, Multi-RTT | Convex UEs | 3.2 | 12.88 | 27.75 | 66 |
| Case 11, InF-SH, FR2, Multi-RTT | Convex UEs | 0.37 | 0.51 | 0.89 | 6.45 |
| Case 12, InF-DH, FR2, Multi-RTT | Convex UEs | 0.33 | 1.52 | 5.03 | 7.07 |

| Analysis for FR1 |  |
| --- | --- |
| Analysis for FR2 |  |

Figure C.1.1.7.2-1. Accuracy for Rel 16 positioning solutions