#### C.1.1.8 Results from source [14]

##### C.1.1.8.1 Description of evaluation scenarios

In this section results are provided for the following scenarios.

- InF-SH FR1/ FR2: 300x150 m, gNB height = 8m, UE antenna height = 1.5m, Low clutter density, {20%, 2m, 10m},

- InF-DH FR1/ FR2: 120x60 m, gNB height = 8m, UE antenna height = 1.5m, High clutter density, {40%, 2m, 2m},

- InH-OO FR1/ FR2: 120x50 m, gNB height = 3m.

Evaluation assumptions for system level analysis are provided in Table C.1.1.8.1-1.

Table C.1.1.8.1-1: Rel.16 NR positioning – vertical accuracy performance summary [14]

| Parameter | [Case1, InF- SH, FR2], [Case3, InF-DH, FR2], [Case5, InH-OO, FR2] | [Case2, InF- SH, FR1], [Case4, InF-DH, FR1], [Case6, InH-OO, FR1] |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline (See Table 3) | Baseline (See Table 3) |
| Carrier frequency | 28 GHz | 1.5 GHz |
| Subcarrier spacing | 120 kHz | 30 kHz |
| Reference Signal Transmission Bandwidth | 400 MHz | 100 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-6, 6 symbols | Comb-6, 6 symbols |
| Reference signal (type of sequence, number of ports, …) | Pseudo-random sequence, 1 antenna port |  |
| Number of sites | 18 sites in InF, 12 sites in InH |  |
| Number of symbols used per occasion | 6 | 6 |
| number of occasions used per positioning estimate | 1 | 1 |
| Power-boosting level | 6 dB | 6 dB |
| Uplink power control (applied/not applied) | not applied |  |
| interference modelling (ideal muting, or other) | ideal muting |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Adaptive threshold-based first peak detection for TOA measurement |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA + AoD positioning Select the TRP by Fast peak average ratio (FPAR), Maximum likelihood positioning algorithm, AoD measurement |  |
| Network synchronization assumptions | Perfect synchronization |  |
| UE/gNB Tx/Rx Calibration Error | 0 |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Beam sweeping at Tx |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | DFT Codebook |  |
| Additional notes, if any |  |  |

##### C.1.1.8.2 Positioning accuracy evaluation results

Table C.1.1.8.2-1 provides summary of NR positioning evaluations results for horizontal location error.

Table C.1.1.8.2-1: Rel.16 NR positioning - horizontal location error results from [14]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 1], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.028 | 0.038 | 0.052 | 0.24 |
|  | (Optional) All UEs | 0.036 | 0.059 | 0.12 | 0.39 |
| [Case 2], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.11 | 0.16 | 0.24 | 0.70 |
|  | (Optional) All UEs | 0.14 | 0.21 | 0.37 | 1.06 |
| [Case 3], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.24 | 0.72 | 1.42 | 2.80 |
|  | (Optional) All UEs | 0.30 | 0.68 | 1.31 | 2.83 |
| [Case 4], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.35 | 0.82 | 1.79 | 3.87 |
|  | (Optional) All UEs | 0.31 | 1.14 | 1.93 | 4.29 |
| [Case 5], [InH-OO], [FR2], [400 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.045 | 0.070 | 0.14 | 0.35 |
|  | (Optional) All UEs | 0.068 | 0.13 | 0.28 | 0.50 |
| [Case 6], [InH-OO], [FR1], [100 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.20 | 0.31 | 0.50 | 1.00 |
|  | (Optional) All UEs | 0.21 | 0.36 | 0.64 | 1.17 |

Figure C.1.1.8.2-1: Rel.16 positioning evaluation in InF-SH scenario.

Figure C.1.1.8.2-2: Rel.16 positioning evaluation in InF-DH scenario.

Figure C.1.1.8.2-3: Rel.16 positioning evaluation in InH-OO scenario.

#### C.1.1.9	Results from source [20]

##### C.1.1.9.1	Description of evaluation scenarios

In this section, we present simulation results for positioning accuracies in Rel.16 under baseline assumptions. The scope of evaluation is limited to DL-TDOA and UL-TDOA. Both frequency ranges (FR1 and FR2) are considered.  The scenarios evaluated are:

- Urban Macro (UMa)

- Urban Micro (UMi)

- Indoor Open Office ( IOO)

- Indoor Factory SH

- Indoor Factory DH

Evaluation assumptions for system level analysis are provided in Table C.1.1.9.1-1a-b.

Table C.1.1.9.1-1a: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [20], source1, UMa, FR1, DL-TDOA | [20], source2, UMi, FR1, DL-TDOA | [20], source3, UMi, FR1, UL-TDOA | [20], source4, UMi, FR2, DL-TDOA |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline |  | Baseline |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 272 PRB | 272 PRB | 272 PRB | 272 PRB |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Staggered | Staggered | Staggered | Staggered |
| Reference signal (type of sequence, number of ports, …) | Rel. 16 | Rel. 16 | Rel. 16 | Rel. 16 |
| Number of sites | 19 (3 sectors each) | 19 (3 sectors each) | 19 (3 sectors each) | 19 (3 sectors each) |
| Number of symbols used per occasion | DL: comb-12, 12 symbol | DL: comb-12, 12 symbol | UL: comb-2, 2 symbol | DL: comb-12, 12 symbol |
| number of occasions used per positioning estimate | 1 & 9 | 1 & 9 | 1 & 9 | 1 & 9 |
| Power-boosting level | No power boosting | DL: No power boosting | UL: Power boosting by comb factor. | DL: No power boosting |
| Uplink power control (applied/not applied) | No power control | No power control | No power control | No power control |
| interference modelling (ideal muting, or other) | Idea muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Interpolation by factor 10 | Interpolation by factor 10 | Interpolation by factor 10 | Interpolation by factor 10 |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares |
| Network synchronization assumptions | Perfect synch | Perfect synch | Perfect synch | Perfect synch |
| UE/gNB Tx/Rx Calibration Error | N/A |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No beamforming | No beamforming | No beamforming | Ideal |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Baseline agreed in #101e | Baseline agreed in #101e | Baseline agreed in #101e | Baseline agreed in #101e |
| Additional notes, if any | Two scenarios 50% indoor UEs and 100% outdoors UEs considered | N/A |  | N/A |

Table C.1.1.9.1-1b: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [20], source5, IOO, FR1, DL-TDOA | [20], source6, IOO, FR1, UL-TDOA | [20], source7, IOO, FR2, DL-TDOA | [20], source8, InF-SH, FR1, DL-TDOA | [20], source9, InF-SH , FR1, UL-TDOA |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline |  | Baseline | Baseline (InF SH) |  |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 272 PRB | 272 PRB | 272 PRB | 272 PRB | 272 PRB |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Staggered | Staggered | Staggered | Staggered | Staggered |
| Reference signal (type of sequence, number of ports, …) | Rel. 16 | Rel. 16 | Rel. 16 | Rel. 16 | Rel. 16 |
| Number of sites | 12 (single sector) | 12 (single sector) | 12 (3 sectors each) | 18 (single sector) | 18 (single sector) |
| Number of symbols used per occasion | DL: comb-12, 12 symbol | UL: comb-2, 2 symbol | DL: comb-12, 12 symbol | DL: comb-12, 12 symbol | UL: comb-2, 2 symbol |
| number of occasions used per positioning estimate | 1 & 9 | 1 & 9 | 1 & 9 | 1 | 1 |
| Power-boosting level | DL: No power boosting | UL: Power boosting by comb factor. | DL: No power boosting | Power boosting by comb factor. | Power boosting by comb factor. |
| Uplink power control (applied/not applied) | No power control | No power control | No power control | No power control | No power control |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Idea muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Interpolation by factor 10 | Interpolation by factor 10 | Interpolation by factor 10 | Interpolation by factor 10 | Interpolation by factor 10 |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares |
| Network synchronization assumptions | Perfect synch | Perfect synch | Perfect synch | Perfect synch | Perfect synch |
| UE/gNB Tx/Rx Calibration Error | N/A | N/A | N/A | N/A | N/A |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No beamforming | No beamforming | Ideal | No beamforming | No beamforming |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Baseline agreed in #101e | Baseline agreed in #101e | Baseline agreed in #101e | Baseline agreed in #101e | Baseline agreed in #101e |
| Additional notes, if any | N/A |  | N/A | N/A |  |

Table C.1.1.9.1-1c: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [20], source10, InF-DH, FR1, DL-TDOA | [20], source11, InF-DH , FR1, UL-TDOA | [20], source12, InF-SH, FR2, DL-TDOA | [20], source13, InF-SH , FR2, UL-TDOA | [20], source14, InF-DH, FR2, DL-TDOA | [20], source15, InF-DH , FR2, UL-TDOA |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline (InF DH) | Baseline (InF DH) | Baseline (InF SH) | Baseline (InF SH) | Baseline (InF DH) | Baseline (InF DH) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 272 PRB | 272 PRB | 272 PRB | 272 PRB | 272 PRB | 272 PRB |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Staggered | Staggered | Staggered | Staggered | Staggered | Staggered |
| Reference signal (type of sequence, number of ports, …) | Rel. 16 | Rel. 16 | Rel. 16 | Rel. 16 | Rel. 16 | Rel. 16 |
| Number of sites | 18 (single sector) | 18 (single sector) | 18 (3 sectors each) | 18 (3 sectors each) | 18 (3 sectors each) | 18 (3 sectors each) |
| Number of symbols used per occasion | DL: comb-12, 12 symbol | UL: comb-2, 2 symbol | DL: comb-12, 12 symbol | UL: comb-2, 2 symbol | DL: comb-12, 12 symbol | UL: comb-2, 2 symbol |
| number of occasions used per positioning estimate | 1 | 11 | 1 |  | 1 | 1 |
| Power-boosting level | Power boosting by comb factor. | Power boosting by comb factor. | Power boosting by comb factor. | Power boosting by comb factor. | Power boosting by comb factor. | Power boosting by comb factor. |
| Uplink power control (applied/not applied) | No power control | No power control | No power control | No power control | No power control | No power control |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Interpolation by factor 10 | Interpolation by factor 10 | Interpolation by factor 10 | Interpolation by factor 10 | Interpolation by factor 10 | Interpolation by factor 10 |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares |
| Network synchronization assumptions | Perfect synch | Perfect synch | Perfect synch | Perfect synch | Perfect synch | Perfect synch |
| UE/gNB Tx/Rx Calibration Error | 0ns | 0ns | 0ns and 8ns | 0ns and 8ns | 0ns | 0ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No beamforming | No beamforming | Ideal | Ideal | Ideal | Ideal |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Baseline agreed in #101e | Baseline agreed in #101e | Baseline agreed in #101e | Baseline agreed in #101e | Baseline agreed in #101e | Baseline agreed in #101e |
| Additional notes, if any | N/A |  | N/A |  | N/A |  |

##### C.1.1.9.2	Positioning accuracy evaluation results

Table C.1.1.9.2-1 provides summary of NR positioning evaluations results for horizontal location error.

Table C.1.1.9.2-1: Rel.16 NR positioning - horizontal location error results from [20]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [20], source1, UMa, FR1, DL-TDOA (50% UEs indoor) | Convex UEs | N/A | N/A | N/A | N/A |
|  | (Optional) All UEs | 2.2m (1 occasion) 1.6m (9 occasions) | 3.2m (1 occasion) 2.35m (9 occasions) | 5.3m (1 occasion) 3.4m (9 occasions) | 11.97m (1 occasion) 7.04m (9 occasions) |
| [20], source1, UMa, FR1, DL-TDOA (100% UEs outdoor) | Convex UEs | N/A | N/A | N/A | N/A |
|  | (Optional) All UEs | 1.87m (1 occasion) 1.26m (9 occasions) | 2.5m (1 occasion) 1.61m (9 occasions) | 3.25m (1 occasion) 1.98m (9 occasions) | 4.5m (1 occasion) 2.5m (9 occasions) |
|  | (Optional) All UEs |  |  |  |  |
| [20], source2, UMi, FR1, DL-TDOA | Convex UEs | N/A | N/A | N/A | N/A |
|  | (Optional) All UEs | 0.58m (1 occasion) 0.41m (9 occasions) | 0.8m (1 occasion) 0.59m (9 occasions) | 1.14m (1 occasion) 0.77m (9 occasions) | 1.7m (1 occasion) 1.06m (9 occasions) |
| [20], source3, UMi, FR1, UL-TDOA | Convex UEs | N/A | N/A | N/A | N/A |
|  | (Optional) All UEs | 1.4m (1 occasion) 1.01m (9 occasions) | 1.86m (1 occasion) 1.4m (9 occasions) | 2.39m (1 occasion) 1.84m (9 occasions) | 3.10m (1 occasion) 2.43m (9 occasions) |
| [20], source4, UMi, FR2, DL-TDOA | Convex UEs | N/A | N/A | N/A | N/A |
|  | (Optional) All UEs | 0.05m (1 occasion) 0.02m (9 occasions) | 0.07m (1 occasion) 0.03m (9 occasions) | 0.09m (1 occasion) 0.04m (9 occasions) | 0.13m (1 occasion) 0.06m (9 occasions) |
|  | (Optional) All UEs |  |  |  |  |
| [20], source5, IOO, FR1, DL-TDOA | Convex UEs | N/A | N/A | N/A | N/A |
|  | (Optional) All UEs | 0.41m (1 occasion) 0.26m (9 occasions) | 0.62m (1 occasion) 0.4m (9 occasions) | 0.9m (1 occasion) 0.58m (9 occasions) | 1.51m (1 occasion) 0.93m (9 occasions) |
| [20], source6, IOO, FR1, UL-TDOA | Convex UEs | N/A | N/A | N/A | N/A |
|  | (Optional) All UEs | 0.39m (1 occasion) 0.24m (9 occasions) | 0.61m (1 occasion) 0.36m (9 occasions) | 0.93m (1 occasion) 0.55m (9 occasions) | 1.52m (1 occasion) 0.98m (9 occasions) |
| [20], source7, IOO, FR2, DL-TDOA | Convex UEs | N/A | N/A | N/A | N/A |
|  | (Optional) All UEs | 0.04m (1 occasion) 0.017m (9 occasions) | 0.06m (1 occasion) 0.02m (9 occasions) | 0.1m (1 occasion) 0.04m (9 occasions) | 0.18m (1 occasion) 0.07m (9 occasions) |
|  | (Optional) All UEs |  |  |  |  |
| [20], source8, InF-SH, FR1, DL-TDOA | Convex UEs | 0.06m | 0.089m | 0.13m | 0.19m |
|  | (Optional) All UEs | 0.08m | 0.13m | 0.21m | 0.36m |
| [20], source9, InF-SH, FR1, UL-TDOA | Convex UEs | 0.06m | 0.09m | 0.13m | 0.18m |
|  | (Optional) All UEs | 0.08m | 0.13m | 0.2m | 0.35m |
| [20], source10, InF-DH, FR1, DL-TDOA | Convex UEs | 1.88m | 3.63m | 5.4m | 7m |
|  | (Optional) All UEs | 2.94m | 5.09m | 7.48m | 14.3m |
| [20], source11, InF-DH , FR1, UL-TDOA | Convex UEs | 1.74m | 3.53m | 4.95m | 7.50m |
|  | (Optional) All UEs | 3.03m | 5.06m | 7.91m | 14.55m |
| [20], source12, InF-SH, FR2, DL-TDOA | Convex UEs | 0.0062m (no RX/Tx error) 1.24m (8ns Rx/Tx error) | 0.0088m(no RX/Tx error) 1.88m (8ns Rx/Tx error) | 0.0120m(no RX/Tx error) 2.57m (8ns Rx/Tx error) | 0.0172m(no RX/Tx error) 3.34m (8ns Rx/Tx error) |
|  | (Optional) All UEs | 0.0081m (no RX/Tx error) 1.01m (8ns Rx/Tx error) | 0.0122m (no RX/Tx error) 1.81m (8ns Rx/Tx error) | 0.0196m (no RX/Tx error) 2.66m (8ns Rx/Tx error) | 0.0349m (no RX/Tx error) 3.68m (8ns Rx/Tx error) |
| [20], source13, InF-SH , FR2, UL-TDOA | Convex UEs | 0.0059m (no RX/Tx error) 1.25m (8ns Rx/Tx error) | 0.0081m (no RX/Tx error) 1.90m (8ns Rx/Tx error) | 0.0112m (no RX/Tx error) 2.5m (8ns Rx/Tx error) | 0.0163m (no RX/Tx error) 3.36m (8ns Rx/Tx error) |
|  | (Optional) All UEs | 0.0075m (no RX/Tx error) 1.16m (8ns Rx/Tx error) | 0.0114m (no RX/Tx error) 1.95m (8ns Rx/Tx error) | 0.0174m (no RX/Tx error) 2.76m (8ns Rx/Tx error) | 0.0313m (no RX/Tx error) 3.85m (8ns Rx/Tx error) |
| [20], source14, InF-DH, FR2, DL-TDOA | Convex UEs | 1.78m | 3.28m | 5.08m | 7.06m |
|  | (Optional) All UEs | 2.92m | 5.14m | 7.76m | 14.95m |
| [20], source15, InF-DH , FR2, UL-TDOA | Convex UEs | 1.58m | 3.05m | 4.72m | 6.98m |
|  | (Optional) All UEs | 2.78m | 4.65m | 7.11m | 13.48m |

Simulations results showing accuracies reported in Table C.1.1.9.2-1 are showing in figures C.1.1.9.2-1-16.

Figure C.1.1.9.2-1 UMa FR1 50% UEs indoor DL-TDOA.

Figure C.1.1.9.2-2. UMa FR1 100% UEs outdoor DL-TDOA.

Figure C.1.1.9.2-3. UMi FR1 DL-TDOA.

Figure C.1.1.9.2-4. UMi FR2 DL-TDOA.

Figure C.1.1.9.2-5. UMi FR1 UL-TDOA.

Figure C.1.1.9.2-6. IOO FR1 DL-TDOA.

Figure C.1.1.9.2-7. IOO FR2 DL-TDOA.

Figure C.1.1.9.2-8. IOO FR1 UL-TDOA.

Figure C.1.1.9.2-9 InF-SH FR1 DL-TDOA.

Figure C.1.1.9.2-10. InF-SH FR1 UL-TDOA.

Figure C.1.1.9.2-11. InF-DH FR1 DL-TDOA.

Figure C.1.1.9.2-12. InF-DH FR1 UL-TDOA.

Figure C.1.1.9.2-13 InF-DH FR2 DL-TDOA.

Figure C.1.1.9.2-14 InF-DH FR2 UL-TDOA.

Figure C.1.1.9.2-15. InF-SH FR2 DL-TDOA.

Figure C.1.1.9.2-16. InF-SH FR2 UL-TDOA

#### C.1.1.10	Results from source [17]

##### C.1.1.10.1	Description of evaluation scenarios

Evaluation assumptions for system level analysis are provided in Table C.1.1.10.1-1.

Table C.1.1.10.1-1.

| Parameter |  | Case 1, InF-DH, FR1, DL-TDOA | Case 2, InF-SH, FR1, DL-TDOA | Case 3, InF-DH, FR1, DL-TDOA | Case 4, InF-SH, FR1, DL-TDOA |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |
| Carrier frequency |  | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing |  | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth |  | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb6 | Comb6 | Comb6 | Comb6 |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS | DL-PRS | DL-PRS | DL-PRS |
| Number of sites |  | 18(1-sector) | 18(1-sector) | 18(1-sector) | 18(1-sector) |
| Number of symbols used per occasion |  | 18 | 18 | 18 | 18 |
| number of occasions used per positioning estimate |  | 1 | 1 | 1 | 1 |
| Power-boosting level |  | 0 | 0 | 0 | 0 |
| Uplink power control (applied/not applied) |  | NA | NA | NA | NA |
| interference modelling (ideal muting, or other) |  | Ideal Muting | Ideal muting | Ideal Muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates.. |
| Network synchronization assumptions |  | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error |  | No calibration error | No calibration error | No calibration error | NO calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | In the earliest beam pair is identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles. | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |  |  |  |
| Additional notes, if any |  | All gNB’s are at height 8m and All UEs are at height 1.5m | All gNB’s are at height 8m and All UEs are at height 1.5m | gNBs Heights: Half of gNBs at 4m height and half at 8m height. UEs Heights: either Fixed UE heights at 1.5m or uniformly distributed U[0.5m,2m] | gNBs Heights: Half of gNBs at 4m height and half at 8m height. UEs Heights: either Fixed UE heights at 1.5m or uniformly distributed U[0.5m,2m] |

| Parameter |  | Case 5, InF-SH, FR2, DL-TDOA | Case 6, InF-SH, FR2, multi-RTT | Case 7, InF-DH, FR2, DL-TDOA | Case 8, InF-DH, FR2, multi-RTT |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |
| Carrier frequency |  | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing |  | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth |  | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb2 | Comb2 | Comb2 | Comb2 |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS | DL-PRS&SRS-Pos | DL-PRS | DL-PRS&SRS-Pos |
| Number of sites |  | 18(3-sector) | 18(3-sector) | 18(3-sector) | 18(3-sector) |
| Number of symbols used per occasion |  | 54 (2 FDMed PRS symbols with two TRPs ) | 54 (2 FDMed PRS symbols with two TRPs ) on DL/UL | 54 (2 FDMed PRS symbols with two TRPs ) | 54 (2 FDMed PRS symbols with two TRPs ) on DL/UL |
| number of occasions used per positioning estimate |  | 1 | 1 | 1 | 1 |
| Power-boosting level |  | 0 | 0 | 0 | 0 |
| Uplink power control (applied/not applied) |  | NA | Not applied (23 dBm Tx) | NA | Not applied (23 dBm Tx) |
| interference modelling (ideal muting, or other) |  | Ideal Muting | Ideal muting for DL, interference from 4 UEs in UL | Ideal Muting | Ideal muting for DL, interference from 4 UEs in UL |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | multi-RTT, the algebraic solution proposed in -Norrdine, Abdelmoumen. (55). An Algebraic Solution to the Multilateration Problem. 10.13140/RG.2.1.1681.3602. Equal weight is used in the TOA covariance matrix. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | multi-RTT, the algebraic solution proposed in -Norrdine, Abdelmoumen. (55). An Algebraic Solution to the Multilateration Problem. 10.13140/RG.2.1.1681.3602. Equal weight is used in the TOA covariance matrix. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |
| Network synchronization assumptions |  | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error |  | No calibration error | No calibration error | No calibration error | NO calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. In the strongest beam pair scheme, the strongest RX beam is identified based on the criteria of receiving the strongest path with the serving cell. And Tx beam is further identified in respect to the strongest RX beam per sector. | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. The earliest beam pair is identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles. In the strongest beam pair scheme, the strongest RX beam is identified based on the criteria of receiving the strongest path with the serving cell. And Tx beam is further identified in respect to the strongest RX beam per sector. | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |  |  |  |
| Additional notes, if any |  |  | The UE’s height is lower than gNBs’ height is assumed as prior knowledge in positioning |  | The UE’s height is lower than gNBs’ height is assumed as prior knowledge in positioning |

| Parameter |  | Case 9, UMI, FR1, DL-TDOA or RTT | Case 10, UMI with Δτ, FR1, DL-TDOA and RTT | Case 11, UMA, FR1, DL-TDOA |  |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |
| Carrier frequency |  | 4 GHz | 4 GHz | 4 GHz |  |
| Subcarrier spacing |  | 30KHz | 30KHz | 30KHz |  |
| Reference Signal Transmission Bandwidth |  | 100MHz | 100MHz | 100MHz |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb6 | Comb6 | Comb6 |  |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS | DL-PRS | DL-PRS |  |
| Number of sites |  | 19(3-sector) | 19(3-sector) | 19(3-sector) |  |
| Number of symbols used per occasion |  | 48 | 48 | 48 |  |
| number of occasions used per positioning estimate |  | 1 | 1 | 1 |  |
| Power-boosting level |  | 0 | 0 | 0 |  |
| Uplink power control (applied/not applied) |  | NA | NA | NA |  |
| interference modelling (ideal muting, or other) |  | Ideal Muting | Ideal muting | Ideal Muting |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |  |
| Network synchronization assumptions |  | Perfect Sync | Perfect Sync | Perfect Sync |  |
| UE/gNB Tx/Rx Calibration Error |  | No calibration error | No calibration error | No calibration error |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | NA | NA | NA |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single port wideband Hadamar precoder |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |

| Parameter | Case 12, UMi, FR2, DL-TDOA | Case 13, InH, FR2, DL-TDOA |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |
| Carrier frequency | 30GHz | 30GHz |
| Subcarrier spacing | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb2 | Comb2 |
| Reference signal (type of sequence, number of ports, …) | DL-PRS | DL-PRS |
| Number of sites | 19(3-sector) | 12(3-sector) |
| Number of symbols used per occasion | 58(2 FDMed PRS symbols with two TRPs ) | 36(2 FDMed PRS symbols with two TRPs) |
| number of occasions used per positioning estimate | 1 | 1 |
| Power-boosting level | 0 | 0 |
| Uplink power control (applied/not applied) | NA | NA |
| interference modelling (ideal muting, or other) | Ideal Muting | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |  |
| Network synchronization assumptions | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error | No calibration error | No calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60, 180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. In the strongest beam pair scheme, the strongest RX beam is identified based on the criteria of receiving the strongest path with the serving cell. And Tx beam is further identified in respect to the strongest RX beam per sector. |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |
| Additional notes, if any |  |  |

| Parameter |  | Case 14, UMI with Δτ, FR1, RTT | Case 15, UMI with Δτ, FR1, DL-TDOA | Case 16, UMI without Δτ, FR1, RTT | Case 17, UMI without Δτ, FR1, DL-TDOA |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |
| Carrier frequency |  | 4 GHz | 4 GHz | 4 GHz | 4 GHz |
| Subcarrier spacing |  | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth |  | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb6 | Comb6 | Comb6 | Comb6 |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS | DL-PRS | DL-PRS | DL-PRS |
| Number of sites |  | 19(3-sector) | 19(3-sector) | 19(3-sector) | 19(3-sector) |
| Number of symbols used per occasion |  | 48 | 48 | 48 | 48 |
| number of occasions used per positioning estimate |  | 1 | 1 | 1 | 1 |
| Power-boosting level |  | 0 | 0 | 0 | 0 |
| Uplink power control (applied/not applied) |  | NA | NA | NA | NA |
| interference modelling (ideal muting, or other) |  | Ideal Muting | Ideal muting | Ideal Muting | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |
| Network synchronization assumptions |  | Perfect Sync | Perfect Sync | Perfect Sync |  |
| UE/gNB Tx/Rx Calibration Error |  | The realistic Tx/Rx timing error and/or network sync error according to truncated Gaussian Distribution [-2*T1,2*T1] nsec, as agreed in previous 3GPP RAN1 meetings. The Tx errors at gNB are assumed to be independent |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | NA | NA | NA | NA |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single port wideband Hadamar precoder |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |

| Parameter | Case 18, InF-SH, FR2, DL-TDOA | Case 19, InF-DH, FR2, DL-TDOA | Case 20, UMi, FR2, DL-TDOA | Case 21, InH, FR2, DL-TDOA |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |
| Carrier frequency | 28GHz | 28GHz | 30GHz | 30GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb2 | Comb2 | Comb2 | Comb2 |
| Reference signal (type of sequence, number of ports, …) | DL-PRS | DL-PRS | DL-PRS | DL-PRS |
| Number of sites | 18(3-sector) | 18(3-sector) | 19(3-sector) | 12(3-sector) |
| Number of symbols used per occasion | 54(2 FDMed PRS symbols with two TRPs ) | 54(2 FDMed PRS symbols with two TRPs ) | 58(2 FDMed PRS symbols with two TRPs ) | 36(2 FDMed PRS symbols with two TRPs) |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 0 | 0 | 0 | 0 |
| Uplink power control (applied/not applied) | NA | NA | NA | NA |
| interference modelling (ideal muting, or other) | Ideal Muting | Ideal Muting | Ideal Muting | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. . |  |  |  |
| Network synchronization assumptions | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error | The realistic Tx/Rx timing error and/or network sync error according to truncated Gaussian Distribution [-2*T1,2*T1] nsec, as agreed in previous 3GPP RAN1 meetings The Tx errors at gNB are assumed to be independent and T1 = [0, 0.1 0.2 0.5 1 2]. The UE is assumed to has one panel, and thus no Rx timing error with DL-TDOA. |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. In the strongest beam pair scheme, the strongest RX beam is identified based on the criteria of receiving the strongest path with the serving cell. And Tx beam is further identified in respect to the strongest RX beam per sector. |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |  |  |
| Additional notes, if any |  |  |  |  |

##### C.1.1.10.2	Positioning accuracy evaluation results

Tables C.1.1.10.2-1 provides summary of NR positioning evaluations results for horizontal location error.

Table C.1.1.1.2-1: Rel.16 NR positioning - horizontal location error results from [17]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 1, InF FR1 DH ISD20, 100MHz, Link Quality, DL TDOA | Convex UEs | 0.55m | 6.42m | 15.35m | 46.647m |
|  | (Optional) All UEs | 0.67m | 6.51m | 13.37m | 46.649m |
| Case 1, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA | Convex UEs | 0.04m | 0.07m | 0.16m | 0.044m |
|  | (Optional) All UEs | 0.05m | 0.09m | 0.18m | 0.045m |

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 2, InF FR1 SH ISD50, 100MHz, Link Quality, DL TDOA | Convex UEs | 0.035m | 0.06m | 0.25m | 16.556m |
|  | (Optional) All UEs | 0.04m | 0.09m | 0.49m | 14.647m |
| Case 2, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA | Convex UEs | 0.03m | 0.065 | 0.16m | 0.038m |
|  | (Optional) All UEs | 0.05m | 0.11m | 0.17m | 0.034m |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Variable UE heights | Convex UEs | 0.96m | 2.18m | 4.48m | 12.66m |
|  | (Optional) All UEs | 1.67m | 5.39m | 12.11m | 30.9m |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Fixed UE heights | Convex UEs | 1.33m | 3.56m | 5.32m | 13.1m |
|  | (Optional) All UEs | 1.89m | 3.99m | 7.67m | 17.62m |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, Unequal gNBs heights, Variable UE heights | Convex UEs | 0.03m | 0.05m | 0.1m | 0.22m |
|  | (Optional) All UEs | 0.04m | 0.09m | 0.17m | 0.47m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, Unequal gNBs heights, Fixed UE heights | Convex UEs | 0.03m | 0.05m | 0.08m | 0.12m |
|  | (Optional) All UEs | 0.046m | 0.08m | 0.16m | 0.36m |

| Horizontal Positioning Error (all UEs) | Beam Pair | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 5 InF-SH FR2 DL-TDOA | Earliest | 0.0058 | 0.0077 | 0.011 | 0.019 |
|  | Strongest | 0.0074 | 0.010 | 0.017 | 0.027 |
| Case 6 InF-SH FR2 3d mRTT | Earliest | 0.0061 | 0.0085 | 0.011 | 0.015 |
| Case 7 InF-DH FR2 DL-TDOA | Earliest | 0.0054 | 0.0082 | 0.013 | 0.025 |
|  | Strongest | 0.0084 | 0.0132 | 0.0287 | 9.40 |
| Case 8 InF-DH FR2 3d mRTT | Earliest | 0.0061 | 0.0088 | 0.013 | 0.020 |

| Horizontal Positioning Error (convex Hull) | Beam Pair | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 5 InF-SH FR2 DL-TDOA | Earliest | 0.0043 | 0.0059 | 0.0071 | 0.0087 |
|  | Strongest | 0.0057 | 0.0080 | 0.0104 | 0.0170 |
| Case 7 InF-DH FR2 DL-TDOA | Earliest | 0.0039 | 0.0054 | 0.0070 | 0.0096 |
|  | Strongest | 0.0058 | 0.0098 | 0.0138 | 1.6767 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 9, UMI, FR1, DL-TDOA, Without Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 0.23 | 0.31 | 0.4 | 0.6 |
| Case 9, UMI, FR1, DL-TDOA, Without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | (Optional) All UEs | 0.64 | 1.2 | 2 | 3.2 |
| Case 9, UMI, FR1, RTT, Without Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 0.16 | 0.22 | 0.3 | 0.4 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 3.4 | 5 | 6.2 | 8 |
| Case 10, UMI, FR1,RTT, with Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 4 | 4.2 | 6.9 | 9.4 |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | (Optional) All UEs | 5.7 | 8.1 | 9.9 | 17.3 |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, LQ Algorithm | (Optional) All UEs | 7.9 | 11.5 | 14.3 | 20.1 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 11, UMA, FR1, DL-TDOA, without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | Outdoor UEs | 0.31 | 0.5 | 0.8 | 1.5 |
| Case 11, UMA, FR1, RTT, without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | Indoor UEs | 1.7 | 6.9 | 0.8 | 96 |

| Horizontal Error (All UEs) |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 12 UMi FR2 DL-TDOA | Earliest | 0.011 | 0.018 | 0.025 | 0.040 |
|  | Strongest | 0.13 | 0.80 | 2.63 | 6.32 |
| Case 13 InH FR2 DL-TDOA | Earliest | 0.0058 | 0.0093 | 0.015 | 0.024 |
|  | Strongest | 0.0092 | 0.016 | 0.028 | 0.053 |

| Horizontal Error (convex Hull UEs) |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 12 UMi FR2 DL-TDOA | Earliest | 0.0106 | 0.0178 | 0.0251 | 0.0403 |
|  | Strongest | 0.1333 | 0.7972 | 2.6315 | 6.3233 |
| Case 13 InH FR2 DL-TDOA | Earliest | 0.0035 | 0.0053 | 0.0069 | 0.0113 |
|  | Strongest | 0.0066 | 0.0101 | 0.0160 | 0.0279 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 14, UMI, FR1, RTT, With Δτ, Perfect Sync, Timing Error = 1 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 3.4 | 4.8 | 6 | 7.3 |
| Case 14, UMI, FR1, RTT, With Δτ, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 3.5 | 4.8 | 6 | 7.4 |
| Case 14, UMI, FR1, RTT, With Δτ, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 3.6 | 4.8 | 6 | 7.6 |
| Case 14, UMI, FR1, RTT, With Δτ, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 3.7 | 4.8 | 6 | 7.9 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 10, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 20, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 50, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 10, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 20, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 50, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 10, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 20, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.4 | 5.8 | 7.2 | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 50, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.4 | 5.8 | 7.2 | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 16, UMI, FR1, RTT, Without Δ𝜏, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.38 | 0.52 | 0.66 | 0.87 |
| Case 16, UMI, FR1, RTT, Without Δ𝜏, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.68 | 0.89 | 1.1 | 1.37 |
| Case 16, UMI, FR1, RTT, Without Δ𝜏, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 1.11 | 1.45 | 1.75 | 2.22 |
| Case 16, UMI, FR1,RTT, Without Δτ, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.38 | 0.52 | 0.66 | 0.87 |
| Case 16, UMI, FR1,RTT, Without Δτ, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.68 | 0.89 | 1.1 | 1.37 |
| Case 16, UMI, FR1,RTT, Without Δτ, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 1.11 | 1.45 | 1.75 | 2.22 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.23 | 0.31 | 0.41 | 0.61 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.46 | 0.73 | 0.76 | 1 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 1.2 | 1.55 | 1.89 | 2.3 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.23 | 0.31 | 0.41 | 0.61 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.46 | 0.73 | 0.76 | 1 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 1.2 | 1.55 | 1.89 | 2.3 |

| Case ID | Beam Pair | Tx T1 | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| Case 18 InF-SH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 | 0.01 | 0.01 | 0.01 |
|  |  | 0.1ns | 0.02 | 0.03 | 0.04 | 0.06 |
|  |  | 0.2ns | 0.05 | 0.07 | 0.09 | 0.13 |
|  |  | 0.5ns | 0.14 | 0.19 | 0.27 | 0.40 |
|  |  | 1.0ns | 0.28 | 0.40 | 0.55 | 0.96 |
|  |  | 2.0ns | 0.55 | 0.75 | 1.08 | 1.59 |
|  | Strongest beam pair | 0.0ns | 0.01 | 0.01 | 0.01 | 0.02 |
|  |  | 0.1ns | 0.03 | 0.05 | 0.07 | 0.11 |
|  |  | 0.2ns | 0.06 | 0.10 | 0.14 | 0.25 |
|  |  | 0.5ns | 0.19 | 0.29 | 0.47 | 0.94 |
|  |  | 1.0ns | 0.40 | 0.63 | 1.08 | 2.30 |
|  |  | 2.0ns | 0.82 | 1.22 | 1.87 | 5.19 |
| Case 19 InF-DH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 | 0.01 | 0.01 | 0.02 |
|  |  | 0.1ns | 0.03 | 0.04 | 0.06 | 0.10 |
|  |  | 0.2ns | 0.06 | 0.08 | 0.12 | 0.24 |
|  |  | 0.5ns | 0.16 | 0.24 | 0.40 | 0.76 |
|  |  | 1.0ns | 0.34 | 0.47 | 0.78 | 2.10 |
|  |  | 2.0ns | 0.74 | 1.17 | 2.15 | 6.31 |
|  | Strongest beam pair | 0.0ns | 0.01 | 0.01 | 0.05 | 43.94 |
|  |  | 0.1ns | 0.05 | 0.08 | 3.61 | 36.56 |
|  |  | 0.2ns | 0.10 | 0.21 | 7.91 | 35.28 |
|  |  | 0.5ns | 0.29 | 0.73 | 18.49 | 44.16 |
|  |  | 1.0ns | 0.74 | 1.96 | 18.90 | 40.33 |
|  |  | 2.0ns | 1.73 | 6.42 | 20.03 | 46.31 |
| Case 20 InH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 | 0.01 | 0.02 | 0.03 |
|  |  | 0.1ns | 0.05 | 0.08 | 0.13 | 0.31 |
|  |  | 0.2ns | 0.11 | 0.17 | 0.28 | 0.47 |
|  |  | 0.5ns | 0.23 | 0.40 | 0.60 | 1.16 |
|  |  | 1.0ns | 0.51 | 0.71 | 1.19 | 2.04 |
|  |  | 2.0ns | 0.97 | 1.49 | 2.23 | 4.11 |
|  | Strongest beam pair | 0.0ns | 0.01 | 0.01 | 0.03 | 0.21 |
|  |  | 0.1ns | 0.10 | 0.22 | 0.76 | 7.44 |
|  |  | 0.2ns | 0.22 | 0.48 | 1.94 | 8.22 |
|  |  | 0.5ns | 0.70 | 2.42 | 5.24 | 11.37 |
|  |  | 1.0ns | 1.26 | 3.45 | 6.88 | 14.94 |
|  |  | 2.0ns | 2.08 | 4.38 | 8.21 | 18.87 |
| Case 21 UMi, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 | 0.01 | 0.02 | 0.03 |
|  |  | 0.1ns | 0.04 | 0.05 | 0.08 | 0.13 |
|  |  | 0.2ns | 0.07 | 0.10 | 0.13 | 0.20 |
|  |  | 0.5ns | 0.15 | 0.20 | 0.26 | 0.38 |
|  |  | 1.0ns | 0.32 | 0.42 | 0.54 | 0.73 |
|  |  | 2.0ns | 0.63 | 0.82 | 1.02 | 1.38 |
|  | Strongest beam pair | 0.0ns | 0.47 | 3.08 | 7.73 | 16.21 |
|  |  | 0.1ns | 2.10 | 6.20 | 12.72 | 23.54 |
|  |  | 0.2ns | 1.20 | 4.23 | 9.16 | 16.59 |
|  |  | 0.5ns | 1.69 | 3.81 | 7.18 | 19.30 |
|  |  | 1.0ns | 2.08 | 4.00 | 7.50 | 14.27 |
|  |  | 2.0ns | 2.95 | 6.52 | 12.89 | 21.65 |

Table C.1.1.10.2-2 provides summary of NR positioning evaluations results for vertical location error.

Table C.1.1.10.2-2: Rel.16 NR positioning - altitude location error results from [17]

| Vertical Positioning error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Variable UE heights | Convex UEs | 2.75m | 6.79m | 10.12m | 20.6m |
|  | (Optional) All UEs | 4.6m | 8.91m | 17.21m | 38.9m |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Fixed UE heights | Convex UEs | 2.35m | 6.78m | 12.5m | 18.44m |
|  | (Optional) All UEs | 3.07m | 7.34m | 13.92m | 22.98m |

| Vertical Positioning error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, , Unequal gNBs heights, Variable UE heights | Convex UEs | 0.183m | 0.33m | 0.75m | 1.89m |
|  | (Optional) All UEs | 0.162m | 0.37m | 0.78m | 2.63m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, Unequal gNBs heights, DL TDOA, Unequal gNBs heights, Fixed UE heights | Convex UEs | 0.178m | 0.298m | 0.41m | 0.9m |
|  | (Optional) All UEs | 0.19m | 0.34m | 0.64m | 1.34m |

| Vertical (Across All UEs) | Beam Pair | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 6 InF-SH FR2 3d mRTT | Earliest | 0.037 | 0.048 | 0.061 | 0.084 |
| Case 8 InF-SH FR2 3d mRTT | Earliest | 0.010 | 0.015 | 0.023 | 0.041 |

#### C.1.1.11	Results from source [18]

##### C.1.1.11.1	Description of evaluation scenarios

Table C.1.1.11.1-1: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | Case 1- config 445 (InF-DH, FR1,UL-TDOA) | Case 2- config1011 (UMi, FR1,UL-TDOA) | Case 3- config config1112 (UMi, FR1,UL-TDOA) |
| --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (r =0.4,hc=2m, dClutter=2m) | UMi (ATOA modelling with mu=-8) | UMi (No ATOA modelling) |
| Carrier frequency | 4GHz | 4GHz | 4GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | KTC = 4 nbSymb = 4 | KTC = 8 nbSymb = 8 | KTC = 8 nbSymb = 8 (o/w KTC = 4 nbSymb = 4; or KTC = 2 nbSymb = 1) |
| Reference signal (type of sequence, number of ports, …) | SRS, 1 Port | SRS, 1 Port | SRS, 1 Port |
| Number of sites | 18 | 19 | 19 |
| Number of symbols used per occasion | 4 | 8 | 8/4/1 |
| number of occasions used per positioning estimate | 1 | 1 | 1 |
| Power-boosting level | Power boosting by comb factor | Power boosting by comb factor | Power boosting by comb factor |
| Uplink power control (applied/not applied) | +23dBm | +23dBm | +23dBm |
| interference modelling (ideal muting, or other) | No Interference (no resource sharing) | No Interference (no resource sharing) | No Interference (no resource sharing) |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Levenberg Marquardt | Levenberg Marquardt | Levenberg Marquardt |
| Network synchronization assumptions | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Not applied | Not applied | Not applied |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Power of the first arriving taps | Power of the first arriving taps | Power of the first arriving taps |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Per site (Rx): 3 sectors (3 panels, 5 dual polarized beams 1 TXRU per panel per polarization dimension UE (Tx): Omni pattern per element, 2 dual polarized antennas (1,2,2), random orientation of UE 4 sequential SRS transmissions | Per site (Rx): 3 sectors (= 3 panels, 5 dual polarized beams per panel) 1 TXRU per panel per polarization dimension UE (Tx): Omni antenna One polarization used for UE-TX Random UE orientation (random polarization orientation | Per site (Rx): Omni antenna 1 TXRU per panel per polarization dimension UE (Tx): Omni antenna One polarization used for UE-TX Random UE orientation (random polarization orientation |
| Additional notes, if any | No LOS detection | LOS detection not applicable (number of LOS links not sufficient, all Links are used) | LOS detection not applicable (number of LOS links not sufficient, all Links are used) |

##### C.1.1.11.2	Positioning accuracy evaluation results

| FR1 Results |  |
| --- | --- |
| Figure C.1.1.11.2-1- CDF 2D (horizontal) error for UTDOA in InF for Rel-16 evaluation. | Figure C.1.1.11.2-2 -CDF 2D (horizontal) error for UTDOA in UMi for Rel-16 evaluation |

Table C.1.1.11.2-3: NR positioning enhancements - horizontal location error results from [18]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 1- config ID 445 (FR1-InF DH) | Convex UEs | 2.45 | 4.18 | 6.87 | ≥10 |
| Case 3- config ID 1112 (FR1-UMi) | All UEs | 0.87 | 1.39 | 2.07 | 3.24 |
| Case 2- config ID 1011 (FR1-UMi with ATOA) | All UEs | 3.32 | 5.12 | 7.34 | ≥10 |

Table C.1.1.11.2-3: NR positioning enhancements - altitude location error results from [18]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 1- config ID 445 (FR1-InF DH) | Convex UEs | 4.54 | 6.33 | 6.33 | ≥10 |
| Case 3- config ID 1112 (FR1-UMi) | All UEs | 8.50 | 8.50 | 8.50 | ≥10 |
| Case 2- config ID 1011 (FR1-UMi with ATOA) | All UEs | 8.46 | 8.46 | ≥10 | ≥10 |