3GPP TSG-RAN5 Meeting #109	R5-255553

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4164 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test cases 7.5.18.1 and 7.5.18.2 for FR2 SCPAC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For FR2 subsequent conditional PSCell addition/change, new test cases 7.5.18.1 and 7.5.18.2 are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add minimum conformance requirements. - Add new test cases 7.5.18.1 and 7.5.18.2. - Add principle of testing in NR-DC for TC 7.5.18.1 and 7.5.18.2. - Add cell configuration mapping for TC 7.5.18.1 and 7.5.18.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.18 (new), A.19.5, E.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0661 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

### 7.5.18 Subsequent conditional PSCell addition/change

#### 7.5.18.0 Minimum conformance requirements

##### 7.5.18.0.1 Minimum conformance requirements for Intra-frequency subsequent CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC

This clause defines requirements for the delay within which the UE shall be able to perform subsequent conditional PSCell change in NR-DC. The requirements in this clause are applicable to NR-DC and shall apply for the UE configured with only PCell in FR1.

The UE shall be capable to transmit PRACH preamble towards the new target PSCell no later than in slot n + Tconfig_PSCell_Subsequent_Change_Conditional:

Where:

-	Slot n is the time when UE transmits SN RRCReconfigurationcomplete message for the previous PSCell addition or change.

-	Tconfig_PSCell_Subsequent_Change_Conditional = TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

-	TEvent_DU is the delay uncertainty which is the time from when UE transmits SN RRCReconfigurationcomplete message for the previous PSCell addition or change until a condition exists at the measurement reference point which will trigger the subsequent conditional PSCell change.

-	Tmeasure is the measurements time stated in TS 38.133 [6] clause 8.11E.2.1.

-	TUE_preparation is the UE preparation time for subsequent conditional PSCell change and starts after UE realizes the condition of PSCell change is met and identity of new PSCell is determined. TUE_preparation is up to 10 ms.

-	Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 20 ms when source and target cells are in the same FR, and Tprocessing = 40 ms when source and target cells are in different FRs.

-	T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms.

-	Trs is the SMTC periodicity of the target cell if the UE has been provided with an SMTC configuration for the target cell in PSCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

-	TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in table 8.1-1 of TS 38.213 [8].

The PCell interruption specified in TS 38.133 [6] clause 8.2 is allowed only after the UE starts to execute a subsequent conditional PSCell change.

The normative reference for this requirement is TS 38.133 [6] clauses 8.11E.

##### 7.5.18.0.2 Minimum conformance requirements for Inter-frequency subsequent CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC

This clause defines requirements for the delay within which the UE shall be able to perform subsequent conditional PSCell addition in NR-DC. The requirements in this clause are applicable to NR-DC and shall apply for the UE configured with only PCell in FR1.

Upon receiving RRC Command containing MR-DCRelease configuration and sending an RRCReconfigurationComplete message confirming the release in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell no later than in subframe n + Tconfig_PSCell_Addition_Conditional:

Where:

-	Tconfig_PSCell_Addition_Conditional = TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

-	TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationComplete message confirming the release of SCG configuration until a condition exists at the measurement reference point which will trigger the conditional PSCell addition.

-	Tmeasure is the measurements time stated in TS 38.133 [6] clause 8.9C.2.1.

-	TUE_preparation is the UE preparation time for conditional PSCell addition and starts after UE realizes the condition of PSCell addition is met and identity of the PSCell is determined. TUE_preparation is up to 10 ms.

-	Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 20 ms when PSCell is in FR1, and Tprocessing = 40 ms when PSCell is in FR2.

-	T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms.

-	Trs is the SMTC periodicity of the target cell if the UE has been provided with an SMTC configuration for the target cell in PSCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

-	TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in table 8.1-1 of TS 38.213 [8].

The PCell interruption specified in TS 38.133 [6] clause 8.2 is allowed only after the UE starts to execute a conditional PSCell addition.

The normative reference for this requirement is TS 38.133 [6] clauses 8.9C.

#### 7.5.18.1 Intra-frequency subsequent CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC

Editor’s Note: This test case is incomplete in following aspects:

- NR FR1 - FR2 OTA testability is still FFS

- Connection diagram may require updates for FR1+FR2 scenarios

- Message content is FFS

- TT analysis is missing

7.5.18.1.1	Test purpose

The purpose of this test is to verify that the subsequent conditional NR PSCell change under NR-DC is within the requirements stated in TS 38.133 [6] clause 8.11E.2.

7.5.18.1.2	Test applicability

This test applies to all types of NR-DC UE supporting conditional PSCell addition/change and supporting SCPAC from Release 18 onwards.

7.5.18.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.18.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.18.1.

7.5.18.1.4	Test description

7.5.18.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.18.1.4.1-1.

Table 7.5.18.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 7.5.18.1-1 | PCell (Cell 1): NR FDD, 15 kHz SSB SCS, 10 MHz bandwidth PSCell (Cell 2 and Cell 3): NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz bandwidth |
| 7.5.18.1-2 | PCell (Cell 1): NR TDD, 15 kHz SSB SCS, 10 MHz bandwidth PSCell (Cell 2 and Cell 3): NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz bandwidth |
| 7.5.18.1-3 | PCell (Cell 1): NR TDD, 30 kHz SSB SCS, 40 MHz bandwidth PSCell (Cell 2 and Cell 3): NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz bandwidth |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Test environment parameters are given in Table 7.5.18.1.4.1-2.

Table 7.5.18.1.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clauses 7.2.3 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.18.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | Without LTE link |  |  |

1. The general test parameter settings are set up according to Table 7.5.18.1.4.1-3.

2. Message contents are defined in clause 7.5.18.1.4.3.

3. The test scenario comprises of three NR cells (Cell 1, Cell 2 and Cell 3). Cell 1 (PCell) is on radio channel 1 in FR1, and the test parameters are given in TS 38.133 [6] clause A.3.7A. Cell 1 is used for connection setup with the power level set according to Annex C.1.2 and C.1.3. Cell 1 once setup is not changed across time. Cell 2 and Cell 3 are on radio channel 2 in FR2.

4.	The AoA setup for this test is Setup 2a as defined in clause A.9. The directions in which the UE meets the EIS spherical coverage criteria have been found using one of the search procedures as described in Annex I.

Table 7.5.18.1.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test. One for NR Cell 1, second for NR Cell 2 and Cell 3 |
| Initial Condition | Active PCell |  | Cell 1, Cell 2 | Cell 1: NR PCell on RF channel number 1 Cell 2: NR PSCell on RF channel number 2 |
|  | Neighbour cell |  | Cell 3 | Neighbour cell on RF channel number 2 |
| Final Condition | Active PCell |  | Cell 1, Cell 3 | Cell 1: PCell on RF channel number 1 Cell 3: NR PSCell on RF channel number 2 |
|  | Neighbour Cell |  | Cell 2 | PSCell changed on RF channel number 2 |
| A4 | Hysteresis | dB | 0 | Used to trigger conditional PSCell addition of Cell 2 |
|  | Threshold RSRP | dBm | -118 |  |
|  | Time to Trigger | s | 0 |  |
| A3 |  | dB | 0 | Used to trigger conditional PSCell change from Cell 2 to Cell 3 |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Gap pattern ID |  |  | gp0 | Gaps are configured during T1, T2 and released upon T3 starts. |
| PRACH configuration on Cell 2 and Cell 3 |  |  | FR2 configuration 2 | Captured in A.7.2-1 |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell 2 |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell 3 |
| T1 |  | s | 1 | During this time the PCell is known, and Cell 2 is unknown. |
| T2 |  | s | 5 | During this time Cell 2 meets the PSCell addition condition and UE adds this PSCell. |
| T3 |  | s | 5 | During this time Cell 3 meets the PSCell change condition and UE sends PRACH to Cell 3. |

7.5.18.1.4.2	Test procedure

The test consists of three successive time periods with duration of T1, T2, and T3 respectively.

At the start of T1, only Cell 1 is known to the UE, and the UE does not have any timing information of Cell 2. The SS shall configure subsequent conditional PSCell addition/change with Cell 2 and Cell 3 as target PSCells during T1, at a time earlier than TRRC_delay before the beginning of T2.

At the start of T2, Cell 2 becomes detectable and meets the PSCell addition condition. UE shall be able to measure and detect that the condition is fulfilled, then transmit PRACH to Cell 2 during T2.

Upon PSCell addition complete (UE transmits SN RRCReconfigurationcomplete message), T3 starts. At the start of T3, Cell 3 becomes detectable and meets the PSCell change condition. UE shall be able to measure and detect that the condition is fulfilled, then transmit PRACH to Cell 3 during T3.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.5.18.1.5-1. T1 starts. The SS starts continuously scheduling the UE to perform DL reception in every DL slot on Cell 1 and monitoring corresponding ACK/NACK feedback sent by the UE.

3.	The SS shall transmit a RRCReconfiguration message with conditionalReconfiguration on Cell 1 to configure CPAC execution condition for Cell 2 and Cell 3.

4.	The UE shall transmit a RRCReconfigurationComplete message.

5.	During T1, if SS sends PDSCHs to UE, but SS doesn’t receive corresponding ACK/NACK feedback from the UE, the number of failed iterations is increased by one and skip to step 11.

6.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.5.18.4.5-2. T2 starts.

7.	The UE shall transmit PRACH to Cell 2 less than 6812ms (power class 1) or 4252ms (power class 2/3/4) from the start of T2. Otherwise, the number of failed iterations is increased by one and skip to step 11.

8.	The UE shall transmit SN RRCReconfigurationcomplete message. If SS receives RRCReconfigurationcomplete before T2 expires, the SS shall stop T2 and switch the power settings from T2 to T3 as specified in Table 7.5.18.4.5-2. T3 starts. If SS doesn’t receive RRCReconfigurationcomplete until T2 expires, the number of failed iterations is increased by one and skip to step 11.

9.	The UE shall transmit PRACH to Cell 3 less than 6812ms (power class 1) or 4252ms (power class 2/3/4) from the start of T3. Otherwise, the number of failed iterations is increased by one and skip to step 11.

10.	The SS waits until T3 expires.

11.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

12.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop. Set Cell 3 physical cell identity = ((current Cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

13.	After the RRC connection release, the SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),

or:

-	switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

14.	Repeat steps 2-13 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.5.18.1.4.3	Message contents

FFS

7.5.18.1.5	Test requirements

Table 7.5.18.1.5-1 and table 7.5.18.1.5-2 define the primary level settings for all tests.

Table 7.5.18.1.5-1: Cell specific test parameters (Cell 2 and Cell 3)

| Parameter | Unit | Config | Test |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| NR Channel Number |  | 1,2,3 | 1 for Cell 1 |  |  |
| NR Channel Number |  | 1,2,3 | 2 for Cell 2 and Cell 3 |  |  |
| Duplex Mode |  | 1,2,3 | TDD |  |  |
| TDD configuration |  | 1,2,3 | TDDConf.3.1 |  |  |
| BWchannel | MHz | 1,2,3 | 100: NPRB,c = 66 |  |  |
| Data PRBs allocated |  | 1,2,3 | 48 |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |  |
| TRS Configuration |  | 1,2,3 | TRS.2.1 TDD |  |  |
| PDSCH/PDCCH TCI state |  | 1,2,3 | TCI.State.2 |  |  |
| PDSCH Reference measurement channel |  | 1,2,3 | SR.3.3 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1,2,3 | CR.3.2 TDD |  |  |
| Dedicated CORESET Reference Channel |  | 1,2,3 | CCR.3.7 TDD |  |  |
| OCNG Patterns |  | 1,2,3 | OP.3 |  |  |
| SSB configuration |  | 1,2,3 | SSB.1 FR2 |  |  |
| SMTC configuration |  | 1,2,3 | SMTC.2 |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1,2,3 | 120 |  |  |
| TRS Configuration |  | 1,2,3 | TRS.2.1 TDD |  |  |
| CSI-RS configuration for CSI reporting |  | 1,2,3 | CSI-RS.3.1 TDD |  |  |
| reportConfigType |  | 1,2,3 | periodic |  |  |
| reportQuantity |  | 1,2,3 | cri-RI-PMI-CQI |  |  |
| CSI reporting periodicity | slot | 1,2,3 | 40 |  |  |
| CSI reporting offset | slot | 1,2,3 | 4 |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3 | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Propagation condition |  | 1,2,3 | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |

Table 7.5.18.1.5-2: NR OTA cell specific test parameters (Cell 2 and Cell 3)

| Parameter | Unit | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Angle of arrival configuration |  | Setup 2a according to clause A.9.2.1 |  |  | Setup 2a according to clause A.9.2.1 |  |  |
| Assumption for UE beamsNote 6 |  | Rough |  |  | Rough |  |  |
| ÊsNote 2 | dBm/SCS | -∞ | -81+TT | -81+TT | -∞ | -∞ | -78+TT |
| SSB_RPNote 2, Note 4 | dBm/SCS | -∞ | -81+TT | -81+TT | -∞ | -∞ | -78+TT |
| Ês/IotBBNote 2, Note 7 | dB | -∞ | 4.88+TT | 4.88+TT | -∞ | -∞ | 4.88+TT |
| IoNote 2, Note 4 | dBm/95.04 MHz | N/A | -56.41+TT | -56.41+TT | N/A | N/A | -56.41+TT |
| Note 1: Void Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone. Note 5: Void Note 6: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. Note 7: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [3], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [3] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

TRRC_delay +TEvent_DU for PSCell addition (Cell 2) occurs during T1 as the PSCell addition condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall transmit the PRACH to Cell 2 less than Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms from the start of T2.

The UE shall transmit the PRACH to Cell 3 less than Tconfig_PSCell_Subsequent_Change_ConditionalNOTE 1 from the start of T3.

NOTE 1:	The subsequent conditional PSCell change delay during T3 can be expressed as follows:

Tconfig_PSCell_Subsequent_Change_Conditional = TEvent_DU +Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

Where (only applicable to Cell 3):

TEvent_DU = 0 ms

Where (applicable to both Cell 2 and Cell 3):

Tmeasure = 6720 ms for power class 1 or 4160 for power class 2/3/4

TUE_preparation = 10 ms

Tprocessing = 40 ms

T∆ = 20 ms

TPSCell_ DU = 1*10+10 = 20 ms

During T2, the total delay of PSCell addition for Cell 2 is:

-	Power Class 1: 6720+10+40+20+20+2 = 6812 ms

-	Power Class 2/3/4: 4160+10+40+20+20+2 = 4252ms

During T3, the total delay of PSCell subsequent change for Cell 3 is:

-	Power Class 1: 0+6720+10+40+20+20+2 = 6812 ms

-	Power Class 2/3/4: 0+4160+10+40+20+20+2 = 4252 ms

All the above test requirements shall be fulfilled for the observed PSCell addition and change delay to be counted as correct. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

#### 7.5.18.2 Inter-frequency subsequent CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC

Editor’s Note: This test case is incomplete in following aspects:

- NR FR1 - FR2 OTA testability is still FFS

- Connection diagram may require updates for FR1+FR2 scenarios

- Message content is FFS

- TT analysis is missing

7.5.18.2.1	Test purpose

The purpose of this test is to verify that the subsequent conditional NR PSCell addition under NR-DC is within the requirements stated in TS 38.133 [6] clause 8.9C.2.

7.5.18.2.2	Test applicability

This test applies to all types of NR-DC UE supporting conditional PSCell addition/change and supporting SCPAC from Release 18 onwards.

7.5.18.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.18.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.18.2.

7.5.18.2.4	Test description

7.5.18.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.18.2.4.1-1.

Table 7.5.18.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 7.5.18.2-1 | Cell 1: FDD 15 kHz SSB SCS, 10 MHz bandwidth Cell 2: TDD 120kHz SSB SCS, 100 MHz bandwidth Cell 3: TDD 120kHz SSB SCS, 100 MHz bandwidth |
| 7.5.18.2-2 | Cell 1: TDD 15 kHz SSB SCS, 10 MHz bandwidth Cell 2: TDD 120kHz SSB SCS, 100 MHz bandwidth Cell 3: TDD 120kHz SSB SCS, 100 MHz bandwidth |
| 7.5.18.2-3 | Cell 1: TDD 30 kHz SSB SCS, 40 MHz bandwidth Cell 2: TDD 120kHz SSB SCS, 100 MHz bandwidth Cell 3: TDD 120kHz SSB SCS, 100 MHz bandwidth |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Test environment parameters are given in Table 7.5.18.2.4.1-2.

Table 7.5.18.2.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clauses 7.2.3 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.18.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | Without LTE link |  |  |

1. The general test parameter settings are set up according to Table 7.5.18.2.4.1-3.

2. Message contents are defined in clause 7.5.18.2.4.3.

3. The test scenario comprises of three NR cells (Cell 1, Cell 2 and Cell 3). Cell 1 (PCell) is on radio channel 1 in FR1, and the test parameters are given in TS 38.133 [6] clause A.3.7A. Cell 1 is used for connection setup with the power level set according to Annex C.1.2 and C.1.3. Cell 1 once setup is not changed across time. Cell 2 (candidate PSCell 1) is on radio channel 2 in FR2. Cell 3 (candidate PSCell 2) is on radio channel 3 in FR2.

4.	The AoA setup for this test is Setup 2a as defined in clause A.9. The directions in which the UE meets the EIS spherical coverage criteria have been found using one of the search procedures as described in Annex I.

Table 7.5.18.2.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2, 3 | Three NR radio channels are used for this test. |
| Initial Condition | Active serving cells |  | Cell 1 | Cell 1: NR PCell on RF channel number 1 |
|  | Neighbour cell |  | Cell 2, Cell 3 | Cell 2: NR PSCell on RF channel number 2 Neighbour cell on RF channel number 3 |
| Final Condition | Active serving cells |  | Cell 1, Cell 3 | Cell 1: PCell on RF channel number 1 Cell 3: NR PSCell on RF channel number 3 |
|  | Neighbour Cell |  | Cell 2 | Neighbour cell on RF channel number 2 |
| A4 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A4 |
|  | Threshold RSRP | dBm | -118 | A4 event is used to trigger conditional PSCell addition of Cell 2 and Cell 3 |
|  | Time to Trigger | s | 0 |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Gap pattern ID |  |  | gp0 |  |
| PRACH configuration on Cell 2 and Cell 3 |  |  | FR2 configuration 2 | Captured in A.7.2-1 |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell 2 |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell 3 |
| T1 |  | s | 1 | UE is connected to Cell 1 (NR PCell) on radio channel 1, but is not aware of Cell 2 (NR PSCell 1) on radio channel 2 and Cell 3 (NR PSCell 2) on radio channel 3 |
| T2 |  |  | <7 | During this time PSCell 1 meets the PSCell addition condition and the UE adds the PSCell (Cell 2) |
| T3 |  | s | 1 | During this time the UE releases the PSCell (Cell 2) |
| T4 |  | s | <7 | During this time PSCell 2 meets the PSCell addition condition and the UE adds the PSCell (Cell 3) |

7.5.18.2.4.2	Test procedure

The test consists of three successive time periods with duration of T1, T2, T3 and T4 respectively.

At the start of T1, only Cell 1 is known to the UE, and the UE does not have any timing information of Cell 2 and Cell 3. The SS shall configure subsequent conditional PSCell addition with Cell 2 and Cell 3 as target PSCells during T1, at a time earlier than TRRC_delay before the beginning of T2.

At the start of T2, Cell 2 becomes detectable and meets the PSCell addition condition. UE shall be able to measure and detect that the condition is fulfilled, then transmit PRACH to Cell 2.

Upon PSCell addition complete (UE transmits SN RRCReconfigurationcomplete message), T3 starts.

During T3, the SS shall send a RRCReconfiguration message to the UE to release PSCell (Cell 2) on radio channel 2. Upon PSCell release complete (UE transmits SN RRCReconfigurationcomplete message), T4 starts.

At the start of T4, Cell 3 becomes detectable and meets the PSCell addition condition. UE shall be able to measure and detect that the condition is fulfilled, then transmit PRACH to Cell 3.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.5.18.2.5-1. T1 starts. The SS starts continuously scheduling the UE to perform DL reception in every DL slot on Cell 1 and monitoring corresponding ACK/NACK feedback sent by the UE.

3.	The SS shall transmit a RRCReconfiguration message with conditionalReconfiguration on Cell 1 to configure CPA execution condition for Cell 2 and Cell 3.

4.	The UE shall transmit a RRCReconfigurationComplete message.

5.	During T1, if SS sends PDSCHs to UE, but SS doesn’t receive corresponding ACK/NACK feedback from the UE, the number of failed iterations is increased by one and skip to step 13.

6.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.5.18.4.5-2. T2 starts.

7.	The UE shall transmit PRACH to Cell 2 less than 6812ms (power class 1) or 4252ms (power class 2/3/4) from the start of T2. Otherwise, the number of failed iterations is increased by one and skip to step 13.

8.	The UE shall transmit SN RRCReconfigurationcomplete message. If SS receives RRCReconfigurationcomplete before T2 expires, the SS shall stop T2 and switch the power settings from T2 to T3 as specified in Table 7.5.18.4.5-2. T3 starts. If SS doesn’t receive RRCReconfigurationcomplete until T2 expires, the number of failed iterations is increased by one and skip to step 13.

9.	The SS shall transmit a RRCReconfiguration message to release PSCell.

10.	The UE shall transmit a RRCReconfigurationComplete message. If SS receives RRCReconfigurationcomplete before T3 expires, the SS shall stop T3 and switch the power settings from T3 to T4 as specified in Table 7.5.18.4.5-2. T4 starts. If SS doesn’t receive RRCReconfigurationcomplete until T3 expires, the number of failed iterations is increased by one and skip to step 13.

11.	The UE shall transmit PRACH to Cell 3 less than 6812ms (power class 1) or 4252ms (power class 2/3/4) from the start of T4. Otherwise, the number of failed iterations is increased by one and skip to step 13.

12.	The SS waits until T4 expires.

13.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

14.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop. Set Cell 3 physical cell identity = ((current Cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

15.	After the RRC connection release, the SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),

or:

-	switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

16.	Repeat steps 2-15 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.5.18.2.4.3	Message contents

FFS

7.5.18.2.5	Test requirements

Table 7.5.18.2.5-1 and table 7.5.18.2.5-2 define the primary level settings for all tests.

Table 7.5.18.2.5-1: Cell specific test parameters (Cell 2 and Cell 3)

| Parameter | Unit | Config | Test |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 |
| NR Channel Number |  | 1,2,3 | 2 for cell 2, 3 for cell 3 |  |  |  |
| Duplex Mode |  | 1,2,3 | TDD |  |  |  |
| TDD configuration |  | 1,2,3 | TDDConf.3.1 |  |  |  |
| BWchannel | MHz | 1,2,3 | 100: NPRB,c = 66 |  |  |  |
| Data PRBs allocated |  | 1,2,3 | 48 |  |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| TRS Configuration |  | 1,2,3 | TRS.2.1 TDD |  |  |  |
| PDSCH/PDCCH TCI state |  | 1,2,3 | TCI.State.2 |  |  |  |
| PDSCH Reference measurement channel |  | 1,2,3 | SR.3.3 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | 1,2,3 | CR.3.2 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  | 1,2,3 | CCR.3.7 TDD |  |  |  |
| OCNG Patterns |  | 1,2,3 | OP.3 |  |  |  |
| SSB configuration |  | 1,2,3 | SSB.1 FR2 |  |  |  |
| SMTC configuration |  | 1,2,3 | SMTC.2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1,2,3 | 120 |  |  |  |
| TRS Configuration |  | 1,2,3 | TRS.2.1 TDD |  |  |  |
| CSI-RS configuration for CSI reporting |  | 1,2,3 | CSI-RS.3.1 TDD |  |  |  |
| reportConfigType |  | 1,2,3 | periodic |  |  |  |
| reportQuantity |  | 1,2,3 | cri-RI-PMI-CQI |  |  |  |
| CSI reporting periodicity | slot | 1,2,3 | 40 |  |  |  |
| CSI reporting offset | slot | 1,2,3 | 4 |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3 | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Propagation condition |  | 1,2,3 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |  |

Table 7.5.18.2.5-2: NR OTA cell specific test parameters (Cell 2 and Cell 3)

| Parameter | Unit | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| Angle of arrival configuration |  | Setup 2a according to clause A.9.2.1 |  |  |  | Setup 2a according to clause A.9.2.1 |  |  |  |
| Assumption for UE beamsNote 6 |  | Rough |  |  |  | Rough |  |  |  |
| ÊsNote 2 | dBm/SCS | -∞ | -81+TT | -∞ |  | -∞ |  |  | -81+TT |
| SSB_RPNote 2, Note 4 | dBm/SCS | -∞ | -81+TT | -∞ |  | -∞ |  |  | -81+TT |
| Ês/IotBBNote 2, Note 7 | dB | -∞ | 4.88+TT | -∞ |  | -∞ |  |  | 4.88+TT |
| IoNote 2, Note 4 | dBm/95.04 MHz | N/A | -56.41+TT | N/A |  | N/A |  |  | -56.41+TT |
| Note 1: Void Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone. Note 5: Void Note 6: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. Note 7: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [3], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [3] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |  |

TRRC_delay +TEvent_DU for PSCell addition (Cell 2) occurs during T1 as the PSCell addition condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall transmit the PRACH to Cell 2 less than Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms from the start of T2.

The UE shall transmit the PRACH to Cell 3 less than Tconfig_PSCell_Addition_ConditionalNOTE 1 from the start of T4.

NOTE 1:	The subsequent conditional PSCell addition delay during T4 can be expressed as follows:

Tconfig_PSCell_Addition_Conditional = TEvent_DU +Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

Where (only applicable to Cell 3):

TEvent_DU = 0 ms

Where (applicable to both Cell 2 and Cell 3):

Tmeasure = 6720 ms for power class 1 or 4160 for power class 2/3/4

TUE_preparation = 10 ms

Tprocessing = 40 ms

T∆ = 20 ms

TPSCell_ DU = 1*10+10 = 20 ms

During T2, the total delay of PSCell addition for Cell 2 is:

-	Power Class 1: 6720+10+40+20+20+2 = 6812 ms

-	Power Class 2/3/4: 4160+10+40+20+20+2 = 4252ms

During T4, the total delay of PSCell subsequent addition for Cell 3 is:

-	Power Class 1: 0+6720+10+40+20+20+2 = 6812 ms

-	Power Class 2/3/4: 0+4160+10+40+20+20+2 = 4252 ms

All the above test requirements shall be fulfilled for the observed PSCell addition delay to be counted as correct. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

==============Next change==============

## A.19.5	Principle of Testing in NR-DC

For test cases in clause 7 listed in Table A.19.5-1, the following applies:

-	UE does not have to pass the test case.

Table A.19.5-1: Test cases with the E-UTRA/FR1 – FR2 testability issue (NR-DC)

| Clause | Test case slogan |
| --- | --- |
| 7.3.3.5 | NR SA FR2 NR conditional handover including target MCG and target SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC |
| 7.5.18.1 | Intra-frequency subsequent CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC |
| 7.5.18.2 | Inter-frequency subsequent CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC |
| Note 1: The list of the test case is based on TS 38.133 [6] Table A.3.13A.5-1 and updated based on the RAN5 testability analysis. |  |

==============Next change==============

# E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7

Table E.5-1 defines the cell configuration mapping for NR/5GC FR2 test cases in chapter 7 of this test specification.

Table E.5-1: Cell configuration mapping for SA FR2 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 7.1.1.1 | NR SA FR2 cell re-selection | NR Cell 1 | NR Cell 11 |  |  |  |
| 7.1.1.2 | NR SA FR2-FR2 cell re-selection | NR Cell 1 | NR Cell 23 |  |  |  |
| 7.1.1.3 | NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.4 | NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.5 | NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
|  | << Unchanged rows omitted>> |  |  |  |  |  |
| 7.5.11.1 | NR SA FR2 UE UL carrier RRC reconfiguration delay | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.5.12.1 | Addition and Release Delay of PSCell | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.5.13.1 | NR SA FR2 MAC-CE based active joint TCI state switch | NR Cell 1 |  |  |  |  |
| 7.5.13.2 | NR SA FR2 MAC-CE based active uplink TCI state switch | NR Cell 1 |  |  |  |  |
| 7.5.13.3 | NR SA FR2 MAC-CE based active downlink TCI state switch | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.5.13.6.1 | NR SA FR2 MAC-CE based active uplink TCI state switch for 2 known TCI states | NR Cell 1 |  |  |  |  |
| 7.5.14 | NR SA FR2 PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2 | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.5.16 | NR SA FR2 UE L1-RSRP measurement reporting with Scheduling and Measurement Restrictions in multi-Rx operation | NR Cell 1 |  |  |  |  |
| 7.5.18.1 | Intra-frequency subsequent CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC | NR Cell 10 | NR Cell 1 | NR Cell 2 |  |  |
| 7.5.18.2 | Inter-frequency subsequent CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC | NR Cell 10 | NR Cell 1 | NR Cell 6 |  |  |
| 7.6.1.1 | NR SA FR2 event-triggered reporting without gap in non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.2 | NR SA FR2 event-triggered reporting without gap in DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.3 | NR SA FR2 event-triggered reporting with gap in non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.4 | NR SA FR2 event-triggered reporting with gap in DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.5 | NR SA FR2 event triggered reporting test without gap in non-DRX for UE configured with highSpeedMeasFlagFR2-r17 | NR Cell 1 | NR Cell 2 |  |  |  |
|  | << Unchanged rows omitted>> |  |  |  |  |  |
| 7.7.7.1.1 | NR SA FR2 CSI-RSRP measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |
| 7.7.7.2.1 | NR SA FR2-FR2 CSI-RSRP measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.7.14.1 | NR SA FR2 SSB based L1-RSRP measurement for group-based beam reporting | NR Cell 1 |  |  |  |  |
| 7.7.14.2 | NR SA FR2 CSI-RS based L1-RSRP measurement on resource set with repetition off | NR Cell 1 |  |  |  |  |
| 7.7.15.1 | NR SA FR2 SSB based inter-frequency L1-RSRP measurement | NR Cell 1 | NR Cell 6 |  |  |  |

==============End of change==============