3GPP TSG-RAN5 Meeting #109 	R5-256518

# Dallas, Texas, USA, 17th November 2025 – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4271 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to FR2 multi-Rx TRP-specific CSI-RS-based BFD test case 7.5.5.14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Innovation Center Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Many aspects are still missing in FR2 multi-Rx SA RLM test case 7.5.1.10. This CR implements several of them. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Completed several aspects for test case 7.5.5.14 - Updated editor’s notes for all completed aspects |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case would remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.5.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

#### 7.5.5.14	NR SA FR2 PCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in multi-Rx operation and DRX mode

Editor's Note:

- MU and TT are missing

- Annex F needs to be updated

- Configuration of 3AoA setup and testability of the device is still FFS

- UE positioning procedure is still FFS

- Test configures schedulingRequestID-BFR-SCell-r16, but there is no SCell

7.5.5.14.1	Test purpose

The purpose of this test is to verify that the UE properly detects TRP specific CSI-RS-based beam failure and link recovery in the sets q0,0 and q1,0 for TRP0 and sets q0,1 and q1,1 for TRP1 when DRX is used for an FR2 PCell requirements in clauses 7.5.5.0.5.2, 7.5.5.0.5.4 and 7.5.5.0.5.5.

7.5.5.14.2	Test applicability

This test applies to all types of NR UE release 18 onwards supporting simultaneous reception, singleDCI-SDM-scheme or MultiDCI non overlapping, fastBeamSweepingMultiRx-r18 and long DRX cycle.

7.5.5.14.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.5.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.5.14.

7.5.5.14.4	Test description

The test parameters are given in Tables 7.5.5.14.4-1, 7.5.5.14.4-3 and 7.5.5.14.5-1. Cell 1 is the active PCell in the test. PCell is configured with two TRPs. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.5.5.14.1-3 shows the variation of the downlink SNR of the CSI-RS in set q0,0 and q0,1 in the PCell for TRP0 and TRP1 respectively. Figure 7.5.5.14.4-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in q1,0  for TRP0. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms.

In the test, UE is capable of multi-Rx operation and configured with group-based beam reporting (GBBR) on the cell 1. During T1, UE shall be able to report via beam pair associated with sets q0,0 for TRP0 and q0,1 for TRP1. After T5, UE shall be able to report via new beam pair associated with sets q1,0 for TRP0 and q0,1 for TRP1

During T2 and T3, for beam failure detection, CSI-RS resources in the two sets q0,0 for TRP0 and q0,1 for TRP1 are overlapped on the same OFDM symbol according to CSI-RS configuration in table 7.5.5.14.4.1-3 and A.1.4.2-3, and the conditions in clause 7.5.5.0.5.2 are met, at least including:

-	Both CSI-RSs are not in any CSI-RS resource set with repetition ON

-	The CSI-RS in set q0,0 has same QCL source as the active TCI state of one PDSCH, and the CSI-RS in set q0,1 has same QCL source as the active TCI state of the other PDSCH

-	Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

During T4 and T5, for candidate beam detection, CSI-RS resources in the two sets q1,0 for TRP0 and q0,1 for TRP1 are also overlapped according to CSI-RS configuration in table 7.5.5.14.4.1-3 and and A.1.4.2-3.

Figure 7.5.5.14.4-1: SNR and L1-RSRP variation for beam failure detection and link recovery testing for PCell in DRX mode

7.5.5.14.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.5.14.4.1-1.

Table 7.5.5.14.4-1: Supported test configurations for NR SA FR2 PCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in multi-Rx operation and DRX mode

| Configuration | Description |
| --- | --- |
| 7.5.5.14-1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |

Configure the test equipment and the DUT according to the parameters in Table 7.5.5.14.4.1-2.

Table 7.5.5.14.4.1-2: Initial conditions for NR SA FR2 PCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in multi-Rx operation and DRX mode

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.5.14.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 7.5.5.14.4.1-3.

2.	Message contents are defined in clause 7.5.5.14.4.3.

3.	There is one NR cell specified in the test. Cell 1 is the active PCell cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 7.5.5.14.4.1-3: General test parameters for NR SA FR2 PCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in multi-Rx operation and DRX mode

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PCell |  |  |  | Cell 1 |  |
| RF Channel Number for PCell |  |  |  | 1 |  |
| Duplex mode |  | Config 1 |  | TDD |  |
| BW channel |  | Config 1 |  | 100: NPRB,c = 66 |  |
| DL initial BWP configuration |  | Config 1 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  | Config 1 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  | Config 1 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  | Config 1 |  | ULBWP.1.1 |  |
| TDD Configuration |  | Config 1 |  | TDDConf.3.1 |  |
| CORESET Reference Channel |  | Config 1 |  | CR.3.1 TDD | A.1.2 |
| SSB Configuration |  | Config 1 |  | SSB.3 FR2 | A.3 |
| SMTC Configuration |  | Config 1 |  | SMTC.3 | A.4 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 |  | 120 KHz |  |
| PRACH Configuration |  | Config 1 |  | PRACH.1 FR2 | Table A.7.2-1 |
| CSI-RS configuration for TRP0 |  | Config 1 |  | CSI-RS.3.2 TDD | A.1.4.2 |
| CSI-RS configuration for TRP1 |  | Config 1 |  | CSI-RS.3.6 TDD | A.1.4.2 |
| CSI-RS configuration for CSI reporting |  | Config 1 |  | CSI-RS.3.1 TDD | A.1.4.2 |
| TRS configuration |  | Config 1 |  | TRS.2.1 TDD |  |
| TCI configuration |  | Config 1 |  | CSI-RS.Config.0 |  |
| OCNG parameters |  |  |  | OP.1 |  |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |  |
| OCNG parameters |  |  |  | OP.1 | A.2.1 |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |  |
| Beam failure |  | DCI format |  | 1-0 |  |
| detection transmission parameters |  | Number of Control OFDM symbols |  | 2 |  |
|  |  | Aggregation level | CCE | 8 |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |  |
|  |  | DMRS precoder granularity |  | REG bundle size |  |
|  |  | REG bundle size |  | 6 |  |
| DRX |  |  |  | DRX.3 |  |
| Gap pattern ID |  |  |  | N/A |  |
| schedulingRequestID-BFR-SCell-r16 |  |  |  | Configured |  |
| Periodicity of PUCCH for SR configuration for BFR on SCell |  |  | Slot | 40 | 5 ms |
| Offset of PUCCH for SR configuration for BFR on SCell |  |  | Slot | 5 |  |
| PUCCH parameters for SR configuration for BFR on SCell |  |  |  | Table 8.3.3.1.2-1 in [28] |  |
| CSI-RS Index assigned as BFD RS in set (q0,0) |  |  |  | 0 |  |
| CSI-RS Index assigned as CBD RS in set (q1,0) |  |  |  | 1 |  |
| CSI-RS Index assigned as BFD RS in set (q0,1) |  |  |  | 2 |  |
| CSI-RS Index assigned as CBD RS in set (q1,1) |  |  |  | 3 |  |
| CSI-RS index assigned as RLM RS |  |  |  | 0, 1 |  |
| SSB index assigned as RLM RS |  |  |  | 0, 1 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1 in [6]). |
| rsrp-ThresholdCSI-RS | Config 1 |  | dBm/SCS | -95 | Threshold used for Qin_LR_CSI-RS |
| powerControlOffsetSS |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  | n1 | see TS 38.321 12], clause 5.17 |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see TS 38.321 [12], clause 5.17 |
| T310 Timer |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 (BFD time +CBD time +margin) |  |  | s | 5.69 |  |
| T3 (BFD time +margin) |  |  | s | 5.16 |  |
| T4 |  |  | s | 0 |  |
| T5 (CBD time +margin) |  |  | s | 0.57 |  |
| D1 (CBD time +margin) |  |  | s | 0.53 |  |
| NOTE 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

7.5.5.14.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1 configuring UE with groupBasedBeamReporting-r17 on Cell 1 and periodic CSI reporting.

3.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

4.	Set the parameters of NR Cell 1 according to T1 in Table 7.5.5.14.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

5.	When T1 expires the SS shall change the SNR1 and SNR2 values to T2 as specified in Table 7.5.5.14.5-1. T2 starts.

6.	When T2 expires the SS shall change the SNR1 and SNR2 values to T3 as specified in Table 7.5.5.14.5-1. T3 starts.

7.	When T3 expires the SS shall change the SNR1 and SNR2 values to T4 as specified in Table 7.5.5.14.5-1. T4 starts.

8.	When T4 expires the SS shall change the SNR1 and SNR2 values to T5 as specified in Table 7.5.5.14.5-1. T5 starts.

9.	If the SS:

a) detects uplink power at least in all subframes configured for CSI transmission on Cell 1 with sets q0,0 for TRP0 and sets q0,1 for TRP1 (UE reports via beam pair assoiated with sets q0,0 for TRP0 and q0,1 for TRP1) during time periods T1, T2; and

b) detects uplink power on NR carrier in each slot configured for CSI transmission according to the configured periodic CSI reporting for Cell 1 during the period from time point A to time point B; and

c) detects uplink power at least in all subframes configured for CSI transmission on Cell 1 with sets q0,1 for TRP1 from time point B to time point F; and

d) does not detect a preamble on a beam associated with candidate beam set  q1,0 for TRP0 earlier than time point B; and

e) detects PUCCH with LRR, followed by BFR MAC CE containing beam associated with the candidate beam with sets q1,0 for TRP0 before time point F (D1 after the start of T5); and

f) detects uplink power on NR carrier in all subframes configured for CSI transmission on Cell 1 with sets q0,1 for TRP1 during time priods T3, T4 and T5,

the number of successful tests is increased by one. Otherwise the number of failed tests is increased by one.

10.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 7.5.5.14.5-1.

11.	Wait 1s for the UE to re-establish the connection or continue directly to step 10. If the UE re-establishes the connection within 1s continue to step 11. Otherwise continue to step 10.

12.	Switch the UE on and off. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

13.	Repeat steps 2-12 until the confidence level according to Annex G clause G.2.3 is achieved.

7.5.5.14.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.5.5.14.4.3-1: Common Exception messages for NR SA FR2 PCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in multi-Rx operation and DRX mode

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-8A with Condition CSI-RS BFD Table H.3.1-10 with Condition CSI-RS Table H.3.1-10A Table H.3.1-10B Table H.3.7-1 with Condition DRX.3 Table H.3.14-1 with Condition CSI-RS Table H.3.14-2A Table H.3.14-3 with Condition CSI-RS Table H.3.14-4 with Conditions PERIODIC, 2_MS and CSI-RSRP Table H.3.14-5 with Condition CSI-RS |

Table 7.5.5.14.4.3-2: MAC-CellGroupConfig for NR SA FR2 PCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in multi-Rx operation and DRX mode

| Derivation Path: TS 38.508-1, Table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| schedulingRequestID-BFR-r17 | Not present |  |  |
| schedulingRequestID-BFR2-r17 | Not present |  |  |
| schedulingRequestConfig-v1700 | SchedulingRequest-Config |  |  |
| } |  |  |  |

7.5.5.14.5	Test requirement

Tables 7.5.5.14.4.1-3 and 7.5.5.14.5-1 define the primary level settings including test tolerances for NR SA FR2 PCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in multi-Rx operation and DRX mode.

Table7.5.5.14.5-1: Cell specific test parameters for NR SA FR2 PCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in multi-Rx operation and DRX mode

| Parameter | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  | Setup 6 as specified in clause A.9.6 Note 11 |  |  |  |  |
| Assumption for UE beams Note 10 |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0,0 | dB | 5+TT | -3+TT | -12+TT | -12+TT | -12+TT |
| SNR_CSI-RS of set q0,1 | dB | 5+TT | 5+TT | 5+TT | 5+TT | 5+TT |
| SNR_CSI-RS of set q1,0 | dB | 0.2+TT | 0.2+TT | 20.2+TT | 20.2+TT | 20.2+TT |
| CSI-RS_RP of set q1,0 | dBm/ SCS kHz | -104.5+TT | -104.5+TT | -84.5+TT | -84.5+TT | -84.5+TT |
| Noc | dBm/120 kHz | -104.7+TT |  |  |  |  |
| Propagation condition |  | TDL-A 30 ns 75 Hz |  |  |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 4: Void NOTE 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. NOTE 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. NOTE 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. NOTE 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 7.5.5.14.4-1. NOTE 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. NOTE 10: Information about types of UE beam is given in clause B.2.1.3 and does not limit UE implementation or test system implementation. NOTE 11: AoA1 and AoA3 are for TRP0 of PCell, AoA2 is for TRP1 of PCell. AoA1 and A0A2 are from the set of qualified AoA pairs according to A.3.15.6. |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1 with sets q0,0 for TRP0 and sets q0,1 for TRP1. During T1, UE reports via beam pair assoiated with sets q0,0 for TRP0 and q0,1 for TRP1.

During the period from time point A to time point B the UE shall transmit uplink signal in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3, T4, T5, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1 with sets q0,1 for TRP1.

During T3 the UE shall detect beam failure and initiate link recovery for TRP0. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate sets q1,0 for TRP0 and q0,1 for TRP1.

No later than time point F occurring no later than D1 after the start of T5, the UE shall transmit PUCCH with LRR, followed by BFR MAC CE containing beam associated with the candidate beam with sets q1,0 for TRP0. The UE shall not transmit PUCCH with an LRR with candidate beam set  q1,0 for TRP0 earlier than time point B.

After T5, UE shall be able to report via new beam pair associated with sets q1,0 for TRP0 and q0,1 for TRP1.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90 %.

<< End of Changes >>