3GPP TSG-RAN5 Meeting #109 	R5-256500

Dallas, USA 17th Nov 2025 – 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0121 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to applicability of NB IoT NTN test case 8.3.1.1.2 and 8.3.1.2.1. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Wireless Communication Tech |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Applicability of NB IoT NTN test case 8.3.1.1.2 and 8.3.1.2.1 needs to be updated for Non-Anchor case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Applicability of NB IoT NTN test case 8.3.1.1.2 and 8.3.1.2.1 is updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The specification will remain inconsistent |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.1.2, 8.3.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

## 8.3	Demodulation for IOT NTN UE category NB1 and NB2

### 8.3.1	Half-duplex FDD

<< Skipped sections >>

##### 8.3.1.1.2	Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone mode for category NB1 and NB2

8.3.1.1.2.1	Test purpose

To verify the UE's ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for transmission on one antenna ports.

8.3.1.1.2.2	Test applicability

This test applies to all types of NB-IoT FDD UE release 17 and forward of category NB1 and NB2 that supports satellite access operation. Test 2 requires support of non-anchor mode of operation.

8.3.1.1.2.3	Minimum conformance requirements

The requirements are specified in Table 8.3.1.1.2.3-2, with the addition of the parameters in Table 8.3.1.1.2.3-1 and the downlink physical channel setup according to Annex C.3.2. Using this configuration the fraction of maximum throughput percentage shall meet or exceed the minimum requirements specified in Table 8.3.1.1.2.3-2 for the specified SNR.

Table 8.3.1.1.2.3-1: Test Parameters for NPDSCH under standalone

| Parameter | Unit |  | Test 1, 2 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -99 (Note 2) |
| NPDCCH repetition number |  | subframe | 32 for Test 1; 256 for Test 2. |
| (npdcch-NumRepetitions-r13) |  | subframe | 64 for Test 1; 512 for Test 2. |
| (nPDCCH-startSF-USS-r13) |  |  | 1.5 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.3.1.1.2.3-2: Minimum performance under standalone

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.6 FDD (Note 2) | 32 | EPA5 | 1 | 70% | -3.4 | NB1, NB2 |
| 2 (Note 1) | 200kHz | Non-anchor | R.NB.6-1 FDD (Note 2) | 256 | ETU1 | 1 | 70% | -10.2 | NB1, NB2 |
| Note 1: Applicable to UE supporting Non-Anchor mode of operation. Note 2: R.NB.6 FDD has the same parameters with R.NB.1 FDD in Table A.3.12.1.1-1. R.NB.6-1 FDD has the same parameters with R.NB.2 FDD in Table A.3.12.1.1-1. |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.12.1.1.2.

8.3.1.1.2.4	Test description

8.3.1.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [12] clause 4.1.

Frequencies to be tested: K.2.1.

Channel Bandwidths to be tested: As specified per test number in Table 8.3.1.1.2.3-2.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.9 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to Tables 8.3.1.1-1 and 8.3.1.1.2.3-1 as appropriate.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.0.1, H.1.1, H.2.1 and Annex H.4.2.

4.	Propagation conditions are set according to Annex B.0.

5.	UE location according to TS 36.508 [12] clause 8.2.6.1 is provided to the UE by any preconfigured means.

6.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] table 8.2.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 8.2.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.2.6.3.1.

7.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

8.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 8.3.1.1.2.4.3.

8.3.1.1.2.4.2	Test procedure

1.	SS transmits NPDSCH via NPDCCH DCI format N1 for C_RNTI to transmit the DL RMC according to Tables 8.3.1.1.2.3-1 and 8.3.1.1.2.3-2.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Table 8.3.1.1.2.5-1 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3.5 and G.3.6 in Annex G clause G.3.

4.	Repeat steps from 1 to 3 for each test interval in Tables 8.3.1.1.2.5-1 as appropriate.

8.3.1.1.2.4.3	Message contents

Message contents are same as 8.3.1.1.1.4.3 with the following exceptions:

Table 8.3.1.1.2.4.3-1: NPDCCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 36.508 Table 8.1.6.3-3 NPDCCH-ConfigDedicated-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| NPDCCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |  |
| npdcch-NumRepetitions-r13 |  | r64 for Test 1; r512 for Test 2. |  |  |
| npdcch-StartSF-USS-r13 |  | V1.5 |  |  |
| } |  |  |  |  |

Table 8.3.1.1.2.4.3-2: NB-IoT Physical layer parameters for DCI format N1

| Derivation Path: 36.508 Table 8.1.3.6.1.2-2 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Repetition number | NRep = 32 |  | For Test Number 1 |
| Repetition number | NRep = 256 |  | For Test Number 2 |

8.3.1.1.2.5	Test requirement

Table 8.3.1.1.2.3-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.12.1.1 for each throughput test shall meet or exceed the specified value in Table 8.3.1.1.2.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.3.1.1.2.5-1: Test requirements under standalone with 1 NRS ports

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | 200kHz | Anchor | R.NB.6 FDD (Note 2) | 32 | EPA5 | 1 | 70% | -2.6 |
| 2 (Note 1) | 200kHz | Non-anchor | R.NB.6-1 FDD (Note 2) | 256 | ETU1 | 1 | 70% | -9.4 |
| Note 1: Applicable to UE supporting Non-Anchor mode of operation. Note 2: R.NB.6 FDD has the same parameters with R.NB.1 FDD in Table A.3.12.1.1-1. R.NB.6-1 FDD has the same parameters with R.NB.2 FDD in Table A.3.12.1.1-1. |  |  |  |  |  |  |  |  |

<< Skipped sections >>

##### 8.3.1.2.1	Demodulation of NPDCCH single-antenna performance for category NB1 and NB2

Editor’s Note: This test case has been completed under the current working assumption of minimum test time. Further optimisation to the minimum test time is FFS.

8.3.1.2.1.1	Test purpose

This test verifies the demodulation performance of NPDCCH for single-antenna scenario with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant, remains below a given reference value.

8.3.1.2.1.2	Test applicability

This test applies to all types of NB-IoT FDD UE release 17 and forward of category NB1 and NB2 that supports satellite access operation. Test 2 requires support of non-anchor mode of operation.

8.3.1.2.1.3	Minimum conformance requirements

The purpose of these tests is to verify the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg). The requirements are specified in Table 8.3.1.2.1.3-1, with the parameters in Table 8.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the performance.

Table 8.3.1.2.1.3-1: Minimum performance NPDCCH

| Test number | Deployment mode | Repetition number (R) | Carrier Type | Reference Channel | Propagation Condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | Stand-alone | 128 | Anchor | R.NB.3 FDD | EPA5 | 1 | 1 | -4.9 | NB1, NB2 |
| 2 (Note 1) | Stand-alone | 1024 | Non-anchor | R.NB.3 FDD | ETU1 | 1 | 1 | -11.4 | NB1, NB2 |
| Note 1: Applicable to UE supporting Non-Anchor mode of operation. |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.12.2.1.1.

8.3.1.2.1.4	Test description

8.3.1.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [12] clause 4.1.

Operation mode: Standalone.

Frequencies to be tested: K.2.1.

Channel Bandwidths to be tested: As specified per test number in Table 8.3.1.2.1.3-1.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.9 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to Tables 8.3.1.2-1 and 8.3.1.2.1.3-1 as appropriate.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.0.1, H.1.1, H.2.1 and Annex H.4.2.

4.	Propagation conditions are set according to Annex B.0.

5.	UE location according to TS 36.508 [12] clause 8.2.6.1 is provided to the UE by any preconfigured means.

6.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] table 8.2.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 8.2.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.2.6.3.1.

7.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

8.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 8.3.1.2.1.4.3.

8.3.1.2.1.4.2	Test procedure

1.	SS transmits NPDSCH via NPDCCH DCI format N1 for C_RNTI to transmit the DL RMC according to Tables 8.3.1.2-1, 8.3.1.2-2 and 8.3.1.2.1.3-1. The details of NPDCCH are specified in TS36.521-2 Table A.3.13.1. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition and the SNR according to Tables 8.3.1.2.1.5-1 as appropriate.

3.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.4. Count the number of NACKs, ACKs and statDTXs on the UL NPUSCH during each subtest. Pm-dsg is the radio (statDTX)/(NACK +ACK+statDTX). If Pm-dsg is less than the value specified in table 8.3.1.2.1.5-1, UE pass the subtest. Otherwise fail the UE.

4.	SS release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Configure the Non-anchor carrier according to table 8.3.1.2.1.4.3-1.

5.	Repeat steps from 1 to 3 for subtest 2 in Table 8.3.1.2.1.5-1 as appropriate.

8.3.1.2.1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.5B and 8.1.6 with the following exceptions:

Table 8.3.1.2.1.4.3-1: Configure Non-anchor carrier in subtest 2

| Derivation Path: 36.508 Table 8.1.8.2.1.6-1 PhysicalConfigDedicated-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |  |
| carrierConfigDedicated-NB-r13 ::= SEQUENCE { |  |  | Non-anchor carrier |  |
| dl-CarrierConfig-r13::= SEQUENCE { |  |  |  |  |
| dl-CarrierFreq-r13 |  | Note 1 |  |  |
| downlinkBitmapNonAnchor-r13 |  | NULL |  |  |
| dl-GapNonAnchor-r13 |  | NULL |  |  |
| } |  |  |  |  |
| ul-CarrierConfig-r13 ::= SEQUENCE { |  |  |  |  |
| ul-CarrierFreq-r13 |  | Note 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| npdcch-ConfigDedicated-r13 |  | NPDCCH-ConfigDedicated-NB-DEFAULT | See subclause 8.1.6.3 in 36.508 |  |
| npusch-ConfigDedicated-r13 |  | NPUSCH-ConfigDedicated-NB-DEFAULT | See subclause 8.1.6.3 in 36.508 |  |
| uplinkPowerControlDedicated-r13 |  | UplinkPowerControlDedicated-NB-DEFAULT | See subclause 8.1.6.3 in 36.508 |  |
| } |  |  |  |  |
| Note 1: The frequency of Non-anchor carrier is located at 200KHz higher from the centre of the anchor carrier. |  |  |  |

Table 8.3.1.2.1.4.3-2: NPDCCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 36.508 Table 8.1.6.3-3 NPDCCH-ConfigDedicated-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| NPDCCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |  |
| npdcch-NumRepetitions-r13 |  | R128 for Test 1; r1024 for Test 2. |  |  |
| npdcch-StartSF-USS-r13 |  | V2 for Test 1; V1.5 for Test 2. |  |  |
| npdcch-Offset-USS-r13 |  | zero |  |  |
| } |  |  |  |  |

Table 8.3.1.2.1.4.3-3: NB-IoT Physical layer parameters for DCI format N1

| Derivation Path: 36.508 Table 8.1.3.6.1.2-2 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Repetition number | NRep = 1 |  | For Test Number 1 and 2 |

8.3.1.2.1.5	Test requirement

For the parameters specified in Tables 8.3.1.2-1 and 8.3.1.2.1.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.3.1.2.1.5-1.

Table 8.3.1.2.1.5-1: Test Parameters for NPDCCH performance

| Test number | Deployment mode | Repetition number (Rmax) | Operated carrier | Reference Channel | Propagation Condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | Stand-alone | 128 | Anchor | R.NB.3 FDD | EPA5 | 1 | 1 | -4.1 |
| 2 (Note 1) | Stand-alone | 1024 | Non-anchor | R.NB.3 FDD | ETU1 | 1 | 1 | -10.6 |
| Note 1: Applicable to UE supporting Non-Anchor mode of operation. |  |  |  |  |  |  |  |  |

<< End of Changes >>