3GPP TSG-RAN5 Meeting #109	R5-255622

# Dallas, Texas, USA, 17 – 21 Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 1092 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test point analysis for NS_07 n13 PC2 and NS_03/NS_03U for n25 PC2 and UL-MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, HPUE_NR_FR1_FDD_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing test point analysis for NS_07 n13 PC2 and for NS_03/NS_03U for n25 PC2 and UL-MIMO |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test point analysis into Table 4.1.2.1-1 - NS_07 for n13 PC2 - NS_03/NS_03U for n25 PC2 and UL-MIMO |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test point analysis for NS_07 n13 PC2 and for NS_03/NS_03U for n25 PC2 and UL-MIMO will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 CR 3538 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Requirement CR in R5-255624 (CR 3538) Removed file: 38.521-1_TPanalysis_6.2.3_AMPR_NS_03.zip Added file: 38.521-1_TPanalysis_6.2.3_AMPR_NS_03_v1.zip Removed file: 38.521-1_TPanalysis_6.2.3_AMPR_NS_07_v1.zip Added file: 38.521-1_TPanalysis_6.2.3_AMPR_NS_07_v2.zip |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### 4.1.2	Test point analysis per NS value

#### 4.1.2.1	A-MPR, A-SEM and A-SE FR1 test cases for single carrier and UL MIMO

This section contains information on test point selection for single carrier test cases 6.2.3, Additional Maximum Power Reduction (A-MPR), 6.5.2.3 Additional spectrum emission mask (A-SEM) and 6.5.3.3 Additional spurious emissions (A-SE); and for correspondent UL-MIMO test cases in 6.2D.3 and 6.5D.3.3 in [2].

Selection of test points should include some possible worst combinations based on the A-MPR characteristics specified for each NS value and these shall be selected so that they match with corresponding spectrum emission requirements test points. The number of test points should be realistic.

For pi/2 BPSK with Rel-16 DMRS, the correspondent A-MPR requirements are the same as that for Rel-15 DMRS but the PAPR is lower. Given the UE can pass the requirements using Rel-15 DMRS, it can be expected that the UE can pass the requirements using Rel-16 DMRS. Therefore there is no need to additionally test A-MPR, A-SEM and A-SE for Rel-16 DMRS.

Table 4.1.2.1-1 lists number of test points for A-MPR, A-SEM and A-SE single carrier test cases and for different NS values.

Table 4.1.2.1-1: NS value specific test points for A-MPR, A-SEM and A-SE single carrier

| NS label | Number of test points for A-MPR | Number of test points A-SEM | Number of test points A-SE | Justification | Comments |
| --- | --- | --- | --- | --- | --- |
| NS_03 NS_03U | PC3 and PC2: 144 for Non-SUL testing 72 for SUL testing 6.2D.3: 64 | PC3 and PC2: 144 |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_03_v1.zip” | RAN5#109 |
| NS_04 | 6.2.3: 120 for PC3, PC2 and PC1.5 6.2.3: 149 for PC1 6.2D.3: 112 6.5D.3.3:112 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_04_v4.zip” | RAN5#104 |
| NS_05 | 6.5.3.3: 432 |  |  | “38.521-1_TP analysis_6.5.3.3_TX_Additional_Spurious_Emission_NS_05.zip” | RAN5#87-e |
| NS_05, NS_05U | 6.2.3: 288 for PC3 6.2.3: 396 for PC2 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_05_v3.zip” | RAN5#96-e |
| NS_06 | 6.2.3: PC1: 160 for n14, 144 for n85 PC3 and PC2 144 6.2D.3: 64 |  |  | "38.521-1_TPanalysis_6.2.3_AMPR_NS_06_v5.zip" | RAN5#108 |
| NS_07 | PC3: 162 6.2D.3: 72 6.5D.3.3: 72 PC2: 234 | PC3: 162 PC2: 234 | PC3: 162 PC2: 234 | “38.521-1_TPanalysis_6.2.3_AMPR_NS_07_v2.zip” | RAN5#109 |
| NS_12 | 48 6.2D.3: 18 6.5D.3.3: 18 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_12_13_14_15_v1.zip” | RAN5#106 |
| NS_13 | 30 6.2D.3: 14 6.5D.3.3: 14 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_12_13_14_15_v1.zip” | RAN5#106 |
| NS_14 | 6.2.3: 50 6.5.3.3: 25 6.2D.3: 30 6.5D.3.3: 15 |  | 25 | “38.521-1_TPanalysis_6.2.3_AMPR_NS_12_13_14_15_v1.zip” “38.521-1_TPanalysis_6.5.3.3_TX_Additional_Spurious_Emission_NS_14.zip | RAN5#106 RAN5#96-e |
| NS_15 | 110 6.2D.3: 55 6.5D.3.3: 55 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_12_13_14_15_v1.zip” | RAN5#106 |
| NS_17 | 6.5.3.3: 4 |  |  | “38.521-1_TPanalysis_6.5.3.3_TX_Additional_Spurious_Emission_NS_17_v3.zip” | RAN5#104 |
| NS_18 | PC3 6.2.3: 88 6.5.3.3: 44 PC2 6.2.3: 104 6.5.3.3: 52 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_6.5.3.3_ASE_NS_18_v5.zip” | RAN5#108 |
| NS_21 | 180 | 180 | 180 | “38.521-1_TPanalysis_6.2.3_AMPR_NS_21_v1.zip” | RAN5#98 |
| NS_24 | 6.2.3: 300 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_24.zip” | RAN5#87 |
| NS_27 | 6.2.3: 360 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_27_v5.zip” | RAN5#98 |
| NS_35 | 6.2.3: 144 6.2.3: 72 for UL-MIMO |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_35_v3.zip” | RAN5#104 |
| NS_37 | 6.2.3: 48 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_37.zip” | RAN5#86 |
| NS_38 | 6.2.3: 96 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_38.zip” | RAN5#86 |
| NS_39 | 6.2.3: 54 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_39.zip” | RAN5#86 |
| NS_40 | 6.2.3: 24 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_40.zip” | RAN5#87 |
| NS_41 | 6.2.3: 72 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_41.zip” | RAN5#87 |
| NS_42 | 6.2.3: 108 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_42.zip” | RAN5#87 |
| NS_43 | 6.2.3: PC3 28; PC2 38 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_43_v3.zip” | RAN5#103 |
|  | 6.5.3.3: 81 |  |  | “38.521-1_TP analysis_6.5.3.3_TX_Additional_Spurious_Emission_NS_43.zip” | RAN5#87-e |
| NS_43U | 6.2.3: 36 for PC3, 32 for PC2 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_43U_v3.zip” | RAN5#106 |
| NS_44 | 360 | N/A | 180 | “38.521-1_TPanalysis_6.2.3_AMPR_6.5.3.3_ASE_NS_44.zip” | RAN5#90-e |
| NS_45 | 24 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_45.zip” | RAN5#89-e |
| NS_46 | 88 for PC3 144 for PC1 64 for PC2 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_46_v2.zip” | RAN5#105 |
| NS_47 | 6.2.3: 70 for PC3 63 for PC272 for PC1.5 6.2D.3: 28 for PC3 28 for PC2 32 for PC1.5 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_47_v1.zip” | RAN5#102 |
| NS_48 | 240 for PC3 384 for PC2 | N/A | 96 | “38.521-1_TPanalysis_6.2.3_AMPR_6.5.3.3_ASE_NS_48_v4.zip” | RAN5#105 |
| NS_49 | 280 for PC3 392 for PC2 | N/A | 140 for PC3 191 for PC2 | “38.521-1_TPanalysis_6.2.3_AMPR_6.5.3.3_ASE_NS_49_v3.zip” | RAN5#104 |
| NS_50 | 160 for PC3 336 for PC2 336 for PC1.5 |  | 160 for PC3 336 for PC2 336 for PC1.5 | “38.521-1_TPanalysis_6.2.3_AMPR_6.5.3.3_ASE_NS_50_v2.zip” | RAN5#105 |
| NS_56 | 175 |  | 175 | 38.521-1_TPanalysis_6.2.3_AMPR_6.5.3.3_ASE_NS_56.zip | RAN5#93-e |
| NS_62 |  |  | 2 | 38.521-1_TPanalysis_6.5.3.3_TX_Additional_Spurious_Emission_NS_62 | RAN5#105 |
| NS_100 | 144 for Non-SUL testing 72 for SUL testing |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_100.zip” | RAN5#85 |
| NS_UAV_44 | 360 | N/A | 180 | 38.521-1_TPanalysis_6.2K.3_AMPR_6.5K.3.3_ASE_NS_UAV_44.zip | RAN5#108 |
| NS_UAV_46 | N/A | N/A | 2 | 38.521-1_TPanalysis_6.5K.3.3_TX_ASE_NS_UAV_46 | RAN5#108 |
| NS_UAV_70 | 192 | N/A | 96 | 38.521-1_TPanalysis_6.2K.3_AMPR_6.5K.3.3_ASE_NS_UAV_70.zip | RAN5#108 |

-------- End of changes --------

Remove attached .zip file to TR 38.905: “38.521-1_TPanalysis_6.2.3_AMPR_NS_03.zip”

Add attached .zip file to TR 38.905: “38.521-1_TPanalysis_6.2.3_AMPR_NS_03_v1.zip”

Remove attached .zip file to TR 38.905: “38.521-1_TPanalysis_6.2.3_AMPR_NS_07_v1.zip”

Add attached .zip file to TR 38.905: “38.521-1_TPanalysis_6.2.3_AMPR_NS_07_v2.zip”