| 3GPP TS 36.133 V18.11.0 (2025-09) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	105

1	Scope	107

2	References	107

3	Definitions, symbols and abbreviations	109

3.1	Definitions	109

3.2	Symbols	111

3.3	Abbreviations	112

3.4	Test tolerances	115

3.5	Additional notation	115

3.5.1	Groups of bands	115

3.5.1A	Groups of bands for satellite access	118

3.6	General	119

3.6.1	Applicability of requirements in this specification version	119

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	126

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	127

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	128

3.6.2	Applicability of requirements for EN-DC operation	129

3.6.3	Applicability of requirements for NE-DC operation	130

3.6.4	Applicability of requirements for NGEN-DC operation	131

4	E-UTRAN RRC_IDLE state mobility	131

4.1	Cell Selection	131

4.2	Cell Re-selection	132

4.2.1	Introduction	132

4.2.2	Requirements	132

4.2.2.1	Measurement and evaluation of serving cell	133

4.2.2.2	Void	134

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	134

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	136

4.2.2.5	Measurements of inter-RAT cells	138

4.2.2.5.1	Measurements of UTRAN FDD cells	138

4.2.2.5.2	Measurements of UTRAN TDD cells	140

4.2.2.5.3	Measurements of GSM cells	142

4.2.2.5.4	Measurements of HRPD cells	143

4.2.2.5.5	Measurements of cdma2000 1X	144

4.2.2.5.6	Measurements of NR cells	145

4.2.2.5.7	Measurements of NR cells subject to CCA	148

4.2.2.5.8	Measurements of NR cells for RedCap	150

4.2.2.6	Evaluation of cell re-selection criteria	152

4.2.2.7	Maximum interruption in paging reception	152

4.2.2.8	void	153

4.2.2.9	UE measurement capability	153

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	154

4.2.2.10	Reselection to CSG cells	154

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	154

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	155

4.2.2.11	Void	156

4.2.2.12	Void	156

4.2.2.13	Void	156

4.2.2.14	UE measurement capability for RedCap	156

4.3	Minimization of Drive Tests (MDT)	157

4.3.1	Introduction	157

4.3.2	Measurements	157

4.3.2.1	Requirements	157

4.3.3	Relative Time Stamp Accuracy	157

4.3.3.1	Requirements	157

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	158

4.3.4.1	 Requirements	158

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	158

4.3.5.1	Requirements for timeSinceFailure	158

4.4	MBSFN Measurements	158

4.4.1	Introduction	158

4.4.2	MBSFN RSRP measurements	158

4.4.3	MBSFN RSRQ measurements	159

4.4.4	MCH BLER measurements	159

4.5	Proximity-based Services	159

4.5.1	Introduction	159

4.5.2	Requirements	159

4.5.2.1	Interruptions with ProSe Direct Discovery	159

4.5.2.2	Interruptions with ProSe Direct Communication	159

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	160

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	160

4.6	Cell Selection and Re-selection Requirements for UE category NB1	161

4.6.1	Cell Selection	161

4.6.2	Cell Re-selection	161

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	161

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	162

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	163

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	164

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	166

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	167

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	168

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	169

4.6.2.7	Maximum interruption in paging reception in normal coverage	171

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	171

4.6.2.8	UE measurement capability	171

4.6.2.9	WUS receptions for NB1	171

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	172

4.6.3.1	Introduction	172

4.6.3.2	Requirements on UE synchronization for transmission using PUR	172

4.6.3.3	Requirements on TA validation for transmission using PUR	172

4.6A	Cell Selection and Re-selection Requirements for UE category NB-IoT for Satellite Access	173

4.6A.1	Cell Selection	173

4.6A.2	Cell Re-selection for UE category NB-IoT for Satellite Access	173

4.6A.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	174

4.6A.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	175

4.6A.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	176

4.6A.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	178

4.6A.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	180

4.6A.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	181

4.6A.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	183

4.6A.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	185

4.6A.2.7	Maximum interruption in paging reception in normal coverage	187

4.6A.2.7A	Maximum interruption in paging reception in enhanced coverage	187

4.6A.2.8	UE measurement capability	188

4.6A.2.9	WUS receptions for NB1	188

4.6A.3	Requirements for transmission using preconfigured uplink resources for UE category NB-IoT for Satellite Access	189

4.6A.3.1	Introduction	189

4.6A.3.2	Requirements on UE synchronization for transmission using PUR	189

4.6A.3.3	Requirements on TA validation for transmission using PUR	189

4.7	Cell Selection and Re-selection Requirements for UE category M1	189

4.7.1	Cell Selection	189

4.7.2	Cell Re-selection	189

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	189

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	189

4.7.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	190

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	191

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	193

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	195

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	195

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	196

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	196

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	197

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	198

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	201

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	202

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	203

4.7.2.3	WUS receptions for UE category M1	203

4.7.3	Channel quality report for UE Category M1 in idle mode	203

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	204

4.7.4.1	Introduction	204

4.7.4.2	Requirements on UE synchronization for transmission using PUR	204

4.7.4.3	Requirements on TA validation for transmission using PUR	204

4.7A	Cell Selection and Re-selection Requirements for UE category M1 for Satellite Access	205

4.7A.1	Cell Selection	205

4.7A.2	Cell Re-selection for UE category M1 for Satellite Access	205

4.7A.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	206

4.7A.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	206

4.7A.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	208

4.7A.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	209

4.7A.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	212

4.7A.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	214

4.7A.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	214

4.7A.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	215

4.7A.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	215

4.7A.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	216

4.7A.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	218

4.7A.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	220

4.7A.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	223

4.7A.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	223

4.7A.2.3	WUS receptions for UE category M1	224

4.7A.3	Channel quality report for UE Category M1 in idle mode for Satellite Access	224

4.7A.4	Requirements for transmission using preconfigured uplink resources for UE category M1 for Satellite Access	225

4.7A.4.1	Introduction	225

4.7A.4.2	Requirements on UE synchronization for transmission using PUR	225

4.7A.4.3	Requirements on TA validation for transmission using PUR	225

4.8	Idle State Positioning Measurement Requirements for UE category NB1	225

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	225

4.8.1.1	RSTD Measurement Reporting Delay	227

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	227

4.8.2.1	RSTD Measurement Reporting Delay	228

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	229

4.8.3.1	RSTD Measurement Reporting Delay	230

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	231

4.8.4.1	RSTD Measurement Reporting Delay	232

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	233

4.8.5.1	Measurement Reporting Delay	234

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	234

4.8.6.1	Measurement Reporting Delay	235

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	236

4.8.7.1	Measurement Reporting Delay	237

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	237

4.8.8.1	Measurement Reporting Delay	238

4.9	Idle Mode CA Measurement	239

4.9.1	Introduction	239

4.9.2	Requirement	239

4.9.2.1	Detected cell requirement during state transition and Idle mode	239

4.9.2.2	Measurements of inter-frequency CA candidate cells	240

4.9.2.3	Measurements on serving cell	240

4A	E-UTRAN RRC_INACTIVE state mobility	241

4A.1	Cell Re-selection	241

4A.1.1	Introduction	242

4A.1.2	Requirements	242

4A.1.2.1	UE measurement capability	242

4A.1.2.2	Measurement and evaluation of serving cell	242

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	242

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	242

4A.1.2.5	Evaluation of cell re-selection criteria	242

4A.1.2.6	Maximum interruption in paging reception	242

4A.1.2.7	Measurements of inter-RAT NR cells	242

4A.1.2.8	UE measurement capability for RedCap	242

4A.1.2.9	Measurements of inter-RAT NR cells for RedCap	242

4A.2	Requirements for UE Category M1	243

4A.2.1	Introduction	243

4A.2.2	Cell Selection	243

4A.2.3	Cell Reselection	243

4A.2.3.1	Cell Re-selection requirements for UE category M1 in normal coverage	243

4A.2.3.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	243

4A.2.3.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	244

4A.2.3.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	244

4A.2.3.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	244

4A.2.3.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	244

4A.2.4	Channel quality report for UE Category M1 in idle mode	246

5	E-UTRAN RRC_CONNECTED state mobility	246

5.1	E-UTRAN Handover	247

5.1.1	Introduction	247

5.1.2	Requirements	247

5.1.2.1	E-UTRAN FDD – FDD	247

5.1.2.1.1	Handover delay	247

5.1.2.1.2	Interruption time	247

5.1.2.2	E-UTRAN FDD – TDD	248

5.1.2.2.1	(Void)	249

5.1.2.2.2	(Void)	249

5.1.2.3	E-UTRAN TDD – FDD	249

5.1.2.3.1	(Void)	249

5.1.2.3.2	(Void)	249

5.1.2.4	E-UTRAN TDD – TDD	249

5.1.2.4.1	Handover delay	249

5.1.2.4.2	Interruption time	249

5.1.2.5	E-UTRAN HD–FDD	251

5.1.2.5.1	Handover delay	251

5.1.2.5.2	Interruption time	251

5.1.2.6	E-UTRAN FDD – FDD conditional handover	252

5.1.2.6.1	Handover delay	252

5.1.2.6.2	Measurement time	253

5.1.2.6.3	Preparation time	253

5.1.2.6.4	Interruption time	253

5.1.2.7	E-UTRAN FDD – TDD conditional handover	253

5.1.2.8	E-UTRAN TDD – FDD conditional handover	254

5.1.2.9	E-UTRAN TDD – TDD conditional handover	254

5.2	Void	254

5.3	Handover to other RATs	254

5.3.1	E-UTRAN - UTRAN FDD Handover	254

5.3.1.1	Introduction	254

5.3.1.1.1	Handover delay	254

5.3.1.1.2	Interruption time	254

5.3.2	E-UTRAN - UTRAN TDD Handover	255

5.3.2.1	Introduction	255

5.3.2.2	Requirements	255

5.3.2.2.1	Handover delay	255

5.3.2.2.2	Interruption time	255

5.3.3	E-UTRAN - GSM Handover	256

5.3.3.1	Introduction	256

5.3.3.2	Requirements	256

5.3.3.2.1	Handover delay	256

5.3.3.2.2	Interruption time	256

5.3.4	E-UTRAN - NR FR1 Handover	256

5.3.4.1	Introduction	256

5.3.4.2	Handover delay	257

5.3.4.3	Interruption time	257

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	257

5.3.4A.1	Introduction	257

5.3.4A.2	Handover delay	258

5.3.4A.3	Interruption time	258

5.3.4B	E-UTRAN - NR FR1 Handover for RedCap	259

5.3.4B.1	Introduction	259

5.3.4B.2	Requirements	259

5.3.5	E-UTRAN - NR FR2 Handover	259

5.3.5.1	Introduction	259

5.3.5.2	Handover delay	259

5.3.5.3	Interruption time	259

5.4	Handover to Non-3GPP RATs	260

5.4.1	E-UTRAN – HRPD Handover	260

5.4.1.1	Introduction	260

5.4.1.1.1	Handover delay	260

5.4.1.1.2	Interruption time	260

5.4.2	E-UTRAN – cdma2000 1X Handover	261

5.4.2.1	Introduction	261

5.4.2.1.1	Handover delay	261

5.4.2.1.2	Interruption time	261

5.5	E-UTRAN Handover for Cat-M1 UEs	261

5.5.1	Introduction	261

5.5.2	Requirements in CEModeA	262

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	262

5.5.2.1.1	Handover delay	262

5.5.2.1.2	Interruption time	262

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	262

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	263

5.5.2.3.1	Void	263

5.5.2.3.2	Void	263

5.5.3	Requirements in CEModeB	263

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	263

5.5.3.1.1	Handover delay	263

5.5.3.1.2	Interruption time	263

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	264

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	264

5.5A	E-UTRAN Handover for Cat-M1 UEs for Satellite Access	264

5.5A.1	Introduction	264

5.5A.2	Requirements in CEModeA	264

5.5A.2.1	E-UTRAN FDD – FDD HO for Cat-M1 FDD UEs	264

5.5A.2.1.1	Handover delay	264

5.5A.2.1.2	Interruption time	264

5.5A.2.2	E-UTRAN FDD – FDD HO for Cat-M1 HD – FDD UEs	265

5.5A.2.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs	265

5.5A.2.3.1	Handover delay	265

5.5A.2.3.2	Measurement time	266

5.5A.2.3.3	Preparation time	267

5.5A.2.3.4	Interruption time	267

5.5A.2.4	E-UTRAN FDD – FDD conditional HO for Cat-M1 HD – FDD UEs	268

5.5A.3	Requirements in CEModeB	268

5.5A.3.1	E-UTRAN FDD – FDD HO for Cat-M1 FDD UEs	268

5.5A.3.1.1	Handover delay	268

5.5A.3.1.2	Interruption time	268

5.5A.3.2	E-UTRAN FDD – FDD HO for Cat-M1 HD – FDD UEs	269

5.5A.3.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs	269

5.5A.3.3.1	Handover delay	269

5.5A.3.3.2	Measurement time	270

5.5A.3.3.3	Preparation time	271

5.5A.3.3.4	Interruption time	271

5.5A.3.4	E-UTRAN FDD – FDD conditional HO for Cat-M1 HD – FDD UEs	272

5.6	Void	272

5.7	E-UTRAN DAPS Handover	272

5.7.1	Introduction	272

5.7.2	Requirements	273

5.7.2.1	E-UTRAN FDD – FDD	273

5.7.2.1.1	DAPS Handover delay	273

5.7.2.1.2	Interruption time	273

5.7.2.2	E-UTRAN FDD – TDD	274

5.7.2.3	E-UTRAN TDD – FDD	274

5.7.2.4	E-UTRAN TDD – TDD	274

5.8	EN-DC Handover with PSCell	274

5.8.1	Introduction	274

5.8.1.1	Handover with PSCell Interruption time	274

5.8.1.2	Handover with PSCell - NR PSCell Change Delay requirements	275

5.9	EN-DC Handover with PSCell using CCA	276

5.9.1	Introduction	276

5.9.1.1	Handover with PSCell – E-UTRA HO Interruption time	276

5.9.1.2	Handover with PSCell - NR PSCell Change Delay requirements	276

6	RRC Connection Mobility Control	277

6.1	RRC Re-establishment	277

6.1.1	Introduction	277

6.1.2	Requirements	277

6.1.2.1	UE Re-establishment delay requirement	278

6.2	Random Access	278

6.2.1	Introduction	278

6.2.2	Requirements	278

6.2.2.1	Contention based random access	278

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	278

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	279

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	279

6.2.2.1.4	Void	279

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	279

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	279

6.2.2.2	Non-Contention based random access	279

6.2.2.2.1	Correct behaviour when receiving Random Access Response	279

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	279

6.2.3	Requirements for Cat-M1 UEs	279

6.2.3A	Random Access Requirements for Cat-M1 UEs for Satellite Access	280

6.3	RRC Connection Release with Redirection	280

6.3.1	Introduction	280

6.3.2	Requirements	280

6.3.2.1	RRC connection release with redirection to UTRAN FDD	280

6.3.2.2	RRC connection release with redirection to GERAN	280

6.3.2.3	RRC connection release with redirection to UTRAN TDD	281

6.3.2.4	RRC connection release with redirection to NR	281

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	282

6.3.2.6	RRC connection release with redirection to NR Redcap	283

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	284

6.4.1	Introduction	284

6.4.2	Requirements	284

6.5	RRC Re-establishment for NB-IoT UEs	285

6.5.1	Introduction	285

6.5.2	Requirements	285

6.5.2.1	UE Re-establishment delay requirement in normal coverage	285

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	286

6.5A	RRC Re-establishment for NB-IoT UEs for Satellite Access	286

6.5A.1	Introduction	286

6.5A.2	Requirements	286

6.5A.2.1	UE Re-establishment delay requirement in normal coverage	287

6.5A.2.2	UE Re-establishment delay requirement in enhanced coverage	287

6.6	Random Access for UE category NB1	288

6.6.1	Introduction	288

6.6.2	Requirements	288

6.6.2.1	Correct behaviour when receiving Random Access Response reception	288

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	288

6.6.2.3	Correct behaviour when receiving a NACK on msg3	288

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	288

6.6.2.5	Correct behaviour when contention Resolution timer expires	288

6.6.2.6	MSG3-based channel quality report for UE Category NB1	289

6.6.3	Requirements for NPRACH configuration	289

6.6A	Random Access for UE category NB-IoT for Satellite Access	289

6.6A.1	Introduction	289

6.6A.2	Requirements	290

6.6A.2.1	Correct behaviour when receiving Random Access Response reception	290

6.6A.2.2	Correct behaviour when not receiving Random Access Response reception	290

6.6A.2.3	Correct behaviour when receiving a NACK on msg3	290

6.6A.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	290

6.6A.2.5	Correct behaviour when contention Resolution timer expires	290

6.6A.2.6	MSG3-based channel quality report for UE Category NB1	290

6.6A.3	Requirements for NPRACH configuration	291

6.7	RRC Re-establishment for Cat-M1 UEs	291

6.7.1	Introduction	291

6.7.2	Requirements	291

6.7.2.1	UE Re-establishment delay requirement for CEModeA	292

6.7.2.2	UE Re-establishment delay requirement for CEModeB	292

6.7A	RRC Re-establishment for Cat-M1 UEs for Satellite Access	293

6.7A.1	Introduction	293

6.7A.2	Requirements	293

6.7A.2.1	UE Re-establishment delay requirement for CEModeA	293

6.7A.2.2	UE Re-establishment delay requirement for CEModeB	294

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	294

6.8.1	Introduction	294

6.8.2	Requirements	295

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	295

6.8A	RRC Connection Release with Redirection for UE Category M1 for Satellite Access	295

6.8A.1	Introduction	295

6.8A.2	Requirements	296

6.8A.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	296

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	296

6.9.1	Introduction	296

6.9.2	Requirements	297

6.9A	RRC Connection Redirection to Non-anchor Carrier in NB-IoT for Satellite Access	297

6.9A.1	Introduction	297

7	Timing and signalling characteristics	298

7.1	UE transmit timing	298

7.1.1	Introduction	298

7.1.2	Requirements	299

7.2	UE timer accuracy	300

7.2.1	Introduction	300

7.2.2	Requirements	300

7.3	Timing Advance	300

7.3.1	Introduction	300

7.3.2	Requirements	300

7.3.2.1	Timing Advance adjustment delay	300

7.3.2.2	Timing Advance adjustment accuracy	301

7.4	Cell phase synchronization accuracy (TDD)	301

7.4.1	Definition	301

7.4.2	Minimum requirements	301

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	302

7.5.1	Introduction	302

7.5.2	eNodeB Synchronization Requirements	302

7.5.2.1	Synchronized E-UTRAN	302

7.5.2.2	Non-Synchronized E-UTRAN	302

7.6	Radio Link Monitoring	302

7.6.1	Introduction	302

7.6.2	Requirements	304

7.6.2.1	Minimum requirement when no DRX is used	304

7.6.2.2	Minimum requirement when DRX is used	304

7.6.2.3	Minimum requirement at transitions	306

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	306

7.6.2.5	Minimum requirement under IDC Interference	306

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	306

7.7.1	Introduction	306

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	307

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	308

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	309

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	310

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	311

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	312

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	313

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	313

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	313

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	314

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	315

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	316

7.7.14	SCell Activation Delay Requirement for Dormant SCell	316

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	318

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	318

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	320

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	320

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	322

7.8	Interruptions with Carrier Aggregation	324

7.8.1	Introduction	324

7.8.2	Requirements	324

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	324

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	324

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	324

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	325

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	325

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	325

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	325

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	325

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	326

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	327

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	327

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	327

7.8.2.13	Interruptions at SRS carrier based switching	328

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	329

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	329

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	329

7.8.2.17	Interruptions during CQI measurement on dormant SCell	329

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	330

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	330

7.8.2.20	Interruptions at SCell hibernation	330

7.8.2.21	Interruptions at direct SCell activation and hibernation	330

7.8.2.22	Interruptions during inter-RAT NR measurements without measurement gap	331

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	332

7.9.1	Introduction	332

7.9.2	Minimum Requirements for Interband Carrier Aggregation	332

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	332

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	332

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	333

7.10.1	Introduction	333

7.10.2	Requirements	333

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	333

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	333

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	333

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	334

7.11	Radio Link Monitoring for UE Category 0	334

7.11.1	Introduction	334

7.11.2	Requirements for FD-FDD and TDD	335

7.11.2.1	Minimum requirement when no DRX is used	335

7.11.2.2	Minimum requirement when DRX is used	336

7.11.2.3	Minimum requirement at transitions	336

7.11.3	Requirements for HD-FDD	337

7.11.3.1	Minimum requirement when no DRX is used	337

7.11.3.2	Minimum requirement when DRX is used	337

7.11.3.3	Minimum requirement at transitions	338

7.12	Interruptions with Dual Connectivity	338

7.12.1	Introduction	338

7.12.2	Requirements	338

7.12.2.1	Interruptions at PSCell addition/release	338

7.12.2.2	Interruptions at transitions between active and non-active during DRX	338

7.12.2.3	Interruptions at transitions from non-DRX to DRX	339

7.12.2.4	Interruptions at SCell addition/release	339

7.12.2.5	Interruptions at SCell activation/deactivation	339

7.12.2.6	Interruptions during measurements on SCC	340

7.12.2.7	Interruptions at SRS carrier based switching	340

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	341

7.13.1	Definition	341

7.13.2	Minimum requirements	341

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	341

7.14.1	Introduction	341

7.14.2	PSCell Addition Delay Requirement	341

7.14.3	PSCell Release Delay Requirement	342

7.15	Maximum Receive Timing Difference in Dual Connectivity	342

7.15.1	Introduction	342

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	342

7.16	Proximity-based Services	342

7.16.1	Introduction	342

7.16.2	Requirements	342

7.16.2.1	ProSe UE transmission timing	342

7.16.2.1.1	Serving cell or PCell as timing reference	343

7.16.2.1.2	SCell or non-serving cell as timing reference	343

7.16.3	Interruptions with ProSe	343

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	343

7.16.3.2	Interruptions at ProSe Direct Communication configuration	344

7.16.3.3	Interruptions during ProSe Direct Discovery	344

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	344

7.16.3.5	Interruptions during ProSe Direct Communication	345

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	345

7.16.4.1	Measurement and evaluation of selected cell	345

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	345

7.16.5	Selection / Reselection of ProSe relay UE	346

7.16.6	ProSe operation under deactivated SCell	346

7.17	Maximum Transmission Timing Difference in Dual Connectivity	346

7.17.1	Introduction	346

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	347

7.18.1	Introduction	347

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	347

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	347

7.19	Radio Link Monitoring for UE Category M1	347

7.19.1	Introduction	347

7.19.2	Requirements for FD-FDD and TDD CE mode A	348

7.19.2.1	Minimum requirement when no DRX is used	349

7.19.2.2	Minimum requirement when DRX is used	350

7.19.2.3	Minimum requirement at transitions	351

7.19.3	Requirements for HD-FDD with CE mode A	351

7.19.3.1	Minimum requirement when no DRX is used	352

7.19.3.2	Minimum requirement when DRX is used	352

7.19.3.3	Minimum requirement at transitions	353

7.19.4	Requirements for FD-FDD and TDD with CE mode B	353

7.19.4.1	Minimum requirement when no DRX is used	355

7.19.4.2	Minimum requirement when DRX is used	356

7.19.4.3	Minimum requirement at transitions	357

7.19.5	Requirements for HD-FDD with CE mode B	357

7.19.5.1	Minimum requirement when no DRX is used	357

7.19.5.2	Minimum requirement when DRX is used	358

7.19.5.3	Minimum requirement at transitions	359

7.19A	Radio Link Monitoring for UE Category M1 for Satellite Access	359

7.19A.1	Introduction	359

7.19A.2	Requirements for FD-FDD and CE mode A	359

7.19A.2.1	Minimum requirement when no DRX is used	361

7.19A.2.2	Minimum requirement when DRX is used	362

7.19A.2.3	Minimum requirement at transitions	363

7.19A.3	Requirements for HD-FDD with CE mode A	363

7.19A.3.1	Minimum requirement when no DRX is used	363

7.19A.3.2	Minimum requirement when DRX is used	364

7.19A.3.3	Minimum requirement at transitions	365

7.19A.4	Requirements for HD-FDD with CE mode B	365

7.19A.4.1	Minimum requirement when no DRX is used	365

7.19A.4.2	Minimum requirement when DRX is used	365

7.19A.4.3	Minimum requirement at transitions	366

7.20	UE transmit timing for NB-IoT	367

7.20.1	Introduction	367

7.20.2	Requirements	367

7.20A	UE transmit timing for NB-IoT for Satellite Access	367

7.20A.1	Introduction	367

7.20A.2	Requirements	368

7.21	 UE timer accuracy for NB-IoT	368

7.21.1	Introduction	368

7.21.2	Requirements	368

7.21A	UE timer accuracy for NB-IoT for Satellite Access	369

7.21A.1	Introduction	369

7.21A.2	Requirements	369

7.22	Timing Advance for NB-IoT	369

7.22.1	Introduction	369

7.22.2	Requirements	369

7.22.2.1	Timing Advance adjustment delay	369

7.22.2.2	Timing Advance adjustment accuracy	369

7.22A	Timing Advance for NB-IoT for Satellite Access	370

7.22A.1	Introduction	370

7.22A.2	Requirements	370

7.22A.2.1	Timing Advance adjustment delay	370

7.22A.2.2	Timing Advance adjustment accuracy	370

7.23	Radio Link Monitoring for Category NB1 UE	370

7.23.1	Introduction	370

7.23.2	Requirements for Category NB1 UE	370

7.23.2.1	Minimum requirement when no DRX is used	371

7.23.2.2	Minimum requirement when DRX is used	371

7.23.2.3	Minimum requirement at transitions	372

7.23A	Radio Link Monitoring for Category NB-IoT UE for Satellite Access	372

7.23A.1	Introduction	372

7.23A.2	Requirements for Category NB1 UE	372

7.23A.2.1	Minimum requirement when no DRX is used	373

7.23A.2.2	Minimum requirement when DRX is used	373

7.23A.2.3	Minimum requirement at transitions	374

7.24	UE transmit timing for Category M1	374

7.24.1	Introduction	374

7.24.2	Requirements	374

7.24A	UE transmit timing for Category M1 for Satellite Access	375

7.24A.1	Introduction	375

7.24A.2	Requirements	375

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	376

7.25.1	Definition	376

7.25.2	Minimum requirements	376

7.26	UE transmit timing for Category M2	377

7.26.1	Introduction	377

7.26.2	Requirements	377

7.27	UE timer accuracy for category M1	377

7.27.1	Introduction	377

7.27.2	Requirements	377

7.27A	UE timer accuracy for category M1 for Satellite Access	377

7.27A.1	Introduction	377

7.27A.2	Requirements	377

7.28	Timing Advance for Category M1	378

7.28.1	Introduction	378

7.28.2	Requirements	378

7.28A	Timing Advance for Category M1 for Satellite Access	378

7.28A.1	Introduction	378

7.28A.2	Requirements	378

7.28A.2.1	Timing Advance adjustment delay	378

7.28A.2.2	Timing Advance adjustment accuracy	378

7.29	Interruptions requirements with FeMBMS	378

7.29.1	Introduction	378

7.29.2	Requirements	378

7.30	Numerology switching delay requirements with FeMBMS	379

7.30.1	Introduction	379

7.30.2	Requirements	379

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	379

7.31.1	Introduction	379

7.31.2	NR PSCell Addition Delay Requirement	379

7.31.3	NR PSCell Release Delay Requirement	380

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	380

7.31A.1	Introduction	380

7.31A.2	NR PSCell Addition Delay Requirement	380

7.31A.3	NR PSCell Release Delay Requirement	381

7.32	Interruptions with EN-DC	382

7.32.1	Introduction	382

7.32.2	Requirements	382

7.32.2.1	Interruptions at PSCell addition/release	382

7.32.2.2	Interruptions at transitions between active and non-active during DRX	382

7.32.2.3	Interruptions at transitions from non-DRX to DRX	383

7.32.2.4	Interruptions at SCell addition/release	383

7.32.2.5	Interruptions at SCell activation/deactivation	383

7.32.2.6	Interruptions during measurements on SCC	384

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	384

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	384

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	384

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	384

7.32.2.7	Interruptions at active BWP switching	384

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	385

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	385

7.32.2.10	Interruptions at SCell hibernation	385

7.32.2.11	Interruptions at direct SCell activation and hibernation	385

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	386

7.32.2.13	Interruptions at NR SRS carrier based switching	386

7.32.2.14	Interruptions at NR SCell dormancy	387

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	387

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	387

7.32.2.15	Interruption during NR measurement with autonomous gaps	387

7.32.2.16	Interruptions at SRS carrier based switching	388

7.32.2.17	Interruptions at SCG activation/deactivation	388

7.32.2.18	Interruptions due to NR SRS antenna port switching	388

7.32.2.19	Interruptions at fast SCell activation/deactivation	389

7.32.2.20	Interruptions due to RRM measurements on deactivated NR SCG	389

7.32.2.21	Interruptions during RLM/BFD measurements on deactivated PScell	390

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	390

7.33.1	Introduction	390

7.33.2	Requirements	390

7.34	Void	390

7.35	Interruptions with SFTD measurements	390

7.35.1	Introduction	390

7.35.2	Requirements	390

7.36	Interruptions with NE-DC	391

7.36.1	Introduction	391

7.36.2	Requirements	391

7.36.2.1	Interruptions at transitions between active and non-active during DRX	391

7.36.2.2	Interruptions at transitions from non-DRX to DRX	391

7.36.2.3	Interruptions at SCell addition/release	391

7.36.2.4	Interruptions at SCell activation/deactivation	392

7.36.2.5	Interruptions during measurements on SCC	392

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	392

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	392

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	393

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	393

7.36.2.6	Interruptions at active BWP switching	393

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	394

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	394

7.36.2.9	Interruptions at SCell hibernation	394

7.36.2.10	Interruptions at direct SCell activation and hibernation	394

7.36.2.11	Interruptions at NR SRS carrier based switching	394

7.36.2.12	Interruptions at NR SCell dormancy	395

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	395

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	395

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	396

7.36.2.14	Interruption during NR measurement with autonomous gaps	396

7.36.2.15	Interruptions at SRS carrier based switching	396

7.36.2.16	Interruptions due to NR SRS antenna port switching	397

7.37	Interruptions during NR measurement with autonomous gaps	398

7.37.1	Introduction	398

7.37.2	Requirements	398

7.38 SCG Activation and Deactivation Delay	398

7.38.1	Introduction	398

7.38.2	SCG Activation Delay Requirement	398

7.38.3	SCG Deactivation Delay Requirement	399

8	UE Measurements Procedures in RRC_CONNECTED State	399

8.1	General Measurement Requirements	399

8.1.1	Introduction	399

8.1.2	Requirements	400

8.1.2.1	UE measurement capability	400

8.1.2.1.1	Monitoring of multiple layers using gaps	408

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	409

8.1.2.1.1b	Monitoring of multiple layers using gaps (EN-DC)	410

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	412

8.1.2.1.1d	Monitoring of multiple layers using gaps (RedCap)	413

8.1.2.1.2	Network controlled small gap	413

8.1.2.2	E-UTRAN intra frequency measurements	415

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	415

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	420

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	424

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	425

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	426

8.1.2.3	E-UTRAN inter frequency measurements	426

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	427

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	432

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	439

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	439

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	439

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	440

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	442

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	443

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	444

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	451

8.1.2.4	Inter RAT measurements	451

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	451

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	456

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	456

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	460

8.1.2.4.5	E-UTRAN FDD – GSM measurements	460

8.1.2.4.6	E-UTRAN TDD – GSM measurements	465

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	465

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	467

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	467

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	467

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	468

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	468

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	468

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	468

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	470

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	470

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	470

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	470

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	471

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	471

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	473

8.1.2.4.21	E-UTRAN FDD – NR measurements	473

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	477

8.1.2.4.22	E-UTRAN TDD – NR measurements	481

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	481

8.1.2.4.23	Void	481

8.1.2.4.24	Void	481

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	481

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	483

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	483

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	484

8.1.2.4.29	E-UTRAN FDD – NR measurements without measurement gap	484

8.1.2.4.30	E-UTRAN TDD – NR measurements without measurement gap	489

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	489

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	489

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	491

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	492

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	494

8.1.2.6.5	Void	496

8.1.2.6.6	Void	496

8.1.2.6.7	Void	496

8.1.2.6.8	Void	496

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	496

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	496

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	498

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	500

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	502

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	503

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	505

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	507

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	509

8.1.2.7	E-UTRAN E-CID Measurements	510

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	510

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	512

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	513

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	514

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	514

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	514

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	517

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	520

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	524

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	527

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	527

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	527

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	528

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	528

8.1.2.10	Void	529

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	529

8.2.1	Introduction	529

8.2.2	Requirements	529

8.3	Measurements for E-UTRA carrier aggregation	533

8.3.1	Introduction	533

8.3.2	Measurements of the primary component carrier	533

8.3.3	Measurements of a secondary component carrier	533

8.3.3.1	Measurements of a secondary component carrier with active SCell	533

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	534

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	534

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	535

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	537

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	537

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	537

8.4.1	Introduction	537

8.4.2	Measurements on the primary component carrier	538

8.4.3	Measurements on a secondary component carrier	539

8.4.4	Measurements on both primary component carrier and a secondary component carrier	539

8.4.5	Measurements on different secondary component carriers	540

8.5	Measurements for UE category 0	541

8.5.1	Introduction	541

8.5.2 	Requirements	542

8.5.2.1 	E-UTRAN intra frequency measurements	542

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	542

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	545

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	547

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	551

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	551

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	552

8.6	Discovery signal measurements	553

8.6.1	Introduction	553

8.6.2	Requirements for CRS based discovery signal measurements	553

8.6.2.1	E-UTRAN intra frequency measurements	553

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	553

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	556

8.6.2.2	E-UTRAN inter frequency measurements	558

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	559

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	561

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	564

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	564

8.6.3	Requirements for CSI-RS based discovery signal measurements	564

8.6.3.1	E-UTRAN intra frequency measurements	564

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	565

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	567

8.6.3.2	E-UTRAN inter frequency measurements	569

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	570

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	572

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	575

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	575

8.7	Discovery signal measurements for E-UTRA carrier aggregation	575

8.7.1	Introduction	575

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	576

8.7.2.1	Measurements of the primary component carrier	576

8.7.2.2	Measurements of a secondary component carrier	576

8.7.2.3	Measurements of a secondary component carrier with active SCell	576

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	576

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	576

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	577

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	579

8.7.3.1	Measurements of the primary component carrier	579

8.7.3.2	Measurements of a secondary component carrier	579

8.7.3.3	Measurements of a secondary component carrier with active SCell	579

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	579

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	579

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	581

8.8	Measurements for E-UTRA dual connectivity	582

8.8.1	Introduction	582

8.8.2	Intra-frequency measurements requirements on PCell	582

8.8.3	Intra-frequency measurements requirements on PSCell	583

8.8.4	Inter-frequency and inter-RAT measurement requirements	583

8.8.5	Intra-frequency measurements with autonomous gaps	583

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	583

8.8.5.2	ECGI reporting delay	584

8.8.6	Inter-frequency measurements with autonomous gaps	584

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	584

8.8.6.2	ECGI reporting delay	585

8.8.7	SSTD Measurements	585

8.8.7.1	Introduction	585

8.8.7.2	SSTD Measurement requirements	585

8.8.7.3	SSTD Measurement Reporting Delay	586

8.8.8	Intra-frequency measurements requirements on SCell	586

8.9	MBSFN Measurements	586

8.9.1	Introduction	586

8.9.2	MBSFN RSRP Measurements	586

8.9.3	MBSFN RSRQ Measurements	587

8.9.4	MCH BLER Measurements	587

8.10	Proximity-based Services	587

8.10.1	Introduction	587

8.10.2	Requirements	587

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	587

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	588

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	589

8.11.1	Introduction	589

8.11.2	CRS based discovery signal measurements	589

8.11.2.1	E-UTRAN intra-frequency measurements	589

8.11.2.1.1	Requirements	589

8.11.2.1.1.1	Requirements when no DRX is used	589

8.11.2.1.1.1.1	Measurement Reporting Requirements	591

8.11.2.1.1.2	Requirements when DRX is used	592

8.11.2.1.1.2.1	Measurement Reporting Requirements	594

8.11.2.2	E-UTRAN inter-frequency measurements	595

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	595

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	599

8.11.3	CSI-RS based discovery signal measurements	599

8.11.3.1	E-UTRAN intra-frequency measurements	599

8.11.3.1.1	Requirements	599

8.11.3.1.1.1	Requirements when no DRX is used	599

8.11.3.1.1.1.1	Measurement Reporting Requirements	601

8.11.3.1.1.2	Requirements when DRX is used	601

8.11.3.1.1.2.1	Measurement Reporting Requirements	602

8.11.3.2	E-UTRAN inter-frequency measurements	603

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	603

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	607

8.11.4	RSSI measurements	607

8.11.4.1	E-UTRAN intra-frequency measurements	607

8.11.4.2	E-UTRAN inter-frequency measurements	607

8.11.5	Channel occupancy measurements	608

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	608

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	608

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	608

8.12.1	Introduction	608

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	608

8.12.2.1	Introduction	608

8.12.2.2	Measurements of a secondary component carrier	608

8.12.2.3	Measurements of a secondary component carrier with active SCell	608

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	609

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	609

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	611

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	614

8.12.3.1	Introduction	614

8.12.3.2	Measurements of a secondary component carrier	614

8.12.3.3	Measurements of a secondary component carrier with active SCell	614

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	614

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	614

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	616

8.13	Measurements for UE Category M1	618

8.13.1	Introduction	618

8.13.2	Requirements for UE category M1 with CE mode A	618

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	619

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	619

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	623

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	626

8.13.2.2	Void	630

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	630

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	630

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	633

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	635

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	636

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	636

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	638

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	640

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	641

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	641

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	641

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	642

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	642

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	642

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	643

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	643

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	644

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	645

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	645

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	646

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	650

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	652

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	656

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	656

8.13.3	Requirements for UE category M1 with CE mode B	657

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	657

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	658

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	662

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	665

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	670

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	671

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	671

8.13.3.2	Void	672

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	672

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	672

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	675

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	677

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	678

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	678

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	678

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	679

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	679

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	679

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	680

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	680

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	680

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	685

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	687

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	692

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	692

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	692

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	694

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	697

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	697

8.13A	Measurements for UE Category M1 for Satellite Access	698

8.13A.1	Introduction	698

8.13A.2	Requirements for UE category M1 with CE mode A	698

8.13A.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	699

8.13A.2.1.1	E-UTRAN FDD intra frequency measurements	699

8.13A.2.1.1.1	E-UTRAN intra frequency measurements when no DRX is used	699

8.13A.2.1.1.2	E-UTRAN intra frequency measurements when DRX is used	701

8.13A.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	703

8.13A.2.1.2.1	E-UTRAN intra frequency measurements when no DRX is used	703

8.13A.2.1.2.2	E-UTRAN intra frequency measurements when DRX is used	703

8.13A.2.2	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	705

8.13A.2.2.1	E-UTRAN FDD - FDD inter frequency measurements	705

8.13A.2.2.1.1	E-UTRAN FDD - FDD inter frequency measurements when no DRX is used	705

8.13A.2.2.1.2	E-UTRAN inter frequency measurements when DRX is used	707

8.13A.2.2.2	E-UTRAN inter-frequency measurements for HD-FDD	709

8.13A.2.2.2.1	E-UTRAN inter-frequency measurements when no DRX is used	709

8.13A.2.2.2.2	E-UTRAN inter frequency measurements when DRX is used	709

8.13A.2.3	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	711

8.13A.2.4	Channel quality report for UE Category M1 in connected mode with CE mode A	712

8.13A.3	Requirements for UE category M1 with CE mode B	712

8.13A.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	713

8.13A.3.1.1	E-UTRAN FDD intra frequency measurements	713

8.13A.3.1.1.1	E-UTRAN intra frequency measurements when no DRX is used	713

8.13A.3.1.1.2	E-UTRAN intra frequency measurements when DRX is used	715

8.13A.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	717

8.13A.3.1.2.1	E-UTRAN intra frequency measurements when no DRX is used	717

8.13A.3.1.2.2	E-UTRAN intra frequency measurements when DRX is used	718

8.13A.3.2	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	720

8.13A.3.2.1	E-UTRAN FDD - FDD inter frequency measurements	720

8.13A.3.2.1.1	E-UTRAN FDD - FDD inter frequency measurements when no DRX is used	720

8.13A.3.2.1.2	E-UTRAN inter frequency measurements when DRX is used	722

8.13A.3.2.2	E-UTRAN inter-frequency measurements for HD-FDD	724

8.13A.3.2.2.1	E-UTRAN inter-frequency measurements when no DRX is used	724

8.13A.3.2.2.2	E-UTRAN inter frequency measurements when DRX is used	724

8.13A.3.3	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	726

8.13A.3.4	Channel quality report for UE Category M1 in connected mode with CE mode B	727

8.14	Measurements for UE category NB1	727

8.14.1	Introduction	727

8.14.2	NB-IoT intra frequency measurements under normal coverage	727

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	727

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	727

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	728

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	728

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	728

8.14.4	Connected mode channel quality report for UE Category NB1	728

8.14.5	Connected mode channel quality report for UE Category NB2 supporting 16QAM	728

8.14.6	NB-IoT neighbour cell measurements	729

8.14.6.1	Introduction	729

8.14.6.2	Requirements	729

8.14.6.3	Intra-frequency neighbour cell measurements	729

8.14.6.4	Inter-frequency neighbour cell measurements	730

8.14.6.5	 Requirements for monitoring multiple carriers	731

8.14A	Measurements for UE category NB-IoT for Satellite Access	731

8.14A.1	Introduction	731

8.14A.2	NB-IoT intra frequency measurements under normal coverage	732

8.14A.2.1	NB-IoT intra frequency measurements when no DRX is used	732

8.14A.2.2	NB-IoT intra frequency measurements when DRX is used	732

8.14A.3	NB-IoT intra frequency measurements under enhanced coverage	732

8.14A.3.1	NB-IoT intra frequency measurements when no DRX is used	732

8.14A.3.2	NB-IoT intra frequency measurements when DRX is used	732

8.14A.4	Connected mode channel quality report for UE Category NB1	732

8.15	Void	735

8.16	Measurements for UE Category M2	735

8.16.1	Introduction	735

8.16.2	Requirements for UE category M2 with CE mode A	736

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	736

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	736

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	737

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	737

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	738

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	738

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	738

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	739

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	741

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	743

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	744

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	744

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	746

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	749

8.16.3	Requirements for UE category M2 with CE mode B	749

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	749

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	749

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	752

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	754

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	754

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	755

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	757

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	759

8.17	Measurements for E-UTRA – NR Dual Connectivity	760

8.17.1	Introduction	760

8.17.1.1	Measurement Gap Sharing	760

8.17.1A	Intrafrequency Measurements	760

8.17.2	SFTD Measurements	761

8.17.2.1	Introduction	761

8.17.2.2	SFTD Measurement requirements	761

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	761

8.17.2.3	SFTD Measurement Reporting Delay	762

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	763

8.17.3.1	Introduction	763

8.17.3.2	E-UTRAN FDD inter frequency measurements	763

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	763

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	765

8.17.3.3	E-UTRAN TDD inter frequency measurements	767

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	767

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	768

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	770

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	770

8.17.4.1.1	NR Inter-RAT cell identification	771

8.17.4.1.2	NR Inter-RAT measurement	772

8.17.4.1.3	NR Inter-RAT measurement reporting	773

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	774

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	774

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	774

8.17.4A.1.1	NR Inter-RAT cell identification	775

8.17.4A.1.2	NR Inter-RAT measurement	776

8.17.4A.1.3	NR Inter-RAT measurement reporting	777

8.17.4A.1.4	NR inter-RAT RSSI measurements	777

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	778

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	778

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	779

8.17.5.1	Introduction	779

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	779

8.17.5.2.1	Identification of a new UTRA FDD cell	779

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	779

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	779

8.17.5.2.4	Periodic Reporting	780

8.17.5.2.5	Event Triggered Reporting	780

8.17.5.2.6	Event-triggered Periodic Reporting	780

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	781

8.17.5.3.1	Periodic Reporting	782

8.17.5.3.2	Event Triggered Reporting	782

8.17.5.3.3	Event-triggered Periodic Reporting	782

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	783

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	783

8.17.7.1	Introduction	783

8.17.7.2	Identification of a new UTRA FDD cell for SON	783

8.17.7.2.1	Requirements when no DRX is used	783

8.17.7.2.2	Requirements when DRX is used	783

8.17.7.2.3	Reporting Delay	784

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	785

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	785

8.17.9.1	Introduction	785

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	785

8.17.9.2.1	Identification of a new UTRA FDD cell	785

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	785

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	786

8.17.9.2.4	Periodic Reporting	786

8.17.9.2.5	Event Triggered Reporting	786

8.17.9.2.6	Event-triggered Periodic Reporting	787

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	787

8.17.9.3.1	Periodic Reporting	788

8.17.9.3.2	Event Triggered Reporting	788

8.17.9.3.3	Event-triggered Periodic Reporting	788

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	789

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	789

8.17.11.1	Introduction	789

8.17.11.2	Identification of a new UTRA TDD cell for SON	789

8.17.11.2.1	Requirements when no DRX is used	789

8.17.11.2.2	Requirements when DRX is used	789

8.17.11.2.3	Reporting Delay	790

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	790

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	791

8.17.13.1	Introduction	791

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	791

8.17.13.2.1	GSM carrier RSSI	791

8.17.13.2.2	BSIC verification	791

8.17.13.2.3	Enhanced BSIC verification	793

8.17.13.2.4	Periodic Reporting	793

8.17.13.2.5	Event Triggered Reporting	793

8.17.13.2.6	Event-triggered Periodic Reporting	794

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	794

8.17.13.3.1	GSM carrier RSSI	794

8.17.13.3.2	BSIC verification	794

8.17.13.3.3	Periodic Reporting	796

8.17.13.3.4	Event Triggered Reporting	796

8.17.13.3.5	Event-triggered Periodic Reporting	796

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	796

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	796

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	796

8.17.15.1.1	RSTD Measurement Reporting Delay	797

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	797

8.17.15.2.1	RSTD Measurement Reporting Delay	797

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	797

8.17.15.3.1	RSTD Measurement Reporting Delay	798

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	798

8.17.15.4.1	RSTD Measurement Reporting Delay	798

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	799

8.17.16.1	Introduction	799

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	799

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	799

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	799

8.17.17.1	Introduction	799

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	799

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	799

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	799

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	799

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	799

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	799

8.18	Measurements for non-BL/CE UE	799

8.18.1	Introduction	799

8.18.2	Requirements for non-BL/CE UE with CE Mode B	800

8.18.2.1	E-UTRAN intra frequency measurements	800

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	800

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	800

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	801

8.19	Measurements for NR – E-UTRA Dual Connectivity	802

8.19.1	Introduction	802

8.19.2	Intra-frequency Measurements	802

8.19.3	Inter-frequency Measurements	802

8.19.4	Void	802

8.19.5	Intra-frequency E-CID Measurements	802

8.19.6	Intra-frequency measurements with autonomous gaps	803

8.19.6.1	Introduction	803

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	803

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	803

8.19.7	Inter-frequency measurements with autonomous gaps	803

8.19.7.1	Introduction	803

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	803

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	803

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	803

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	803

8.20	Inter-RAT NR Measurements for RedCap UE	803

8.20.1	Introduction	803

8.20.2	E-UTRAN FDD – NR measurements	803

8.20.2.1	Identification of a new NR cell	803

8.20.2.2	Periodic Reporting	807

8.20.2.3	Event Triggered Reporting	807

8.20.2.4	Event-triggered Periodic Reporting	807

8.20.3	E-UTRAN TDD – NR measurements	807

9	Measurements performance requirements for UE	808

9.1	E-UTRAN measurements	808

9.1.1	Introduction	808

9.1.2	Intra-frequency RSRP Accuracy Requirements	808

9.1.2.1	Absolute RSRP Accuracy	808

9.1.2.2	Relative Accuracy of RSRP	809

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	810

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	810

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	811

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	812

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	813

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	814

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	815

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	815

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	815

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	816

9.1.2B.1	Introduction	816

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	816

9.1.3	Inter-frequency RSRP Accuracy Requirements	817

9.1.3.1	Absolute RSRP Accuracy	817

9.1.3.2	Relative Accuracy of RSRP	818

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	819

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	819

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	820

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	820

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	821

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	822

9.1.3B.1	Introduction	822

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	822

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	822

9.1.4	RSRP Measurement Report Mapping	823

9.1.5	Intra-frequency RSRQ Accuracy Requirements	823

9.1.5.1	Absolute RSRQ Accuracy	823

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	824

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	825

9.1.5.4	Absolute WB-RSRQ Accuracy	826

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	827

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	828

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	828

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	828

9.1.5B.1	Introduction	828

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	828

9.1.6	Inter-frequency RSRQ Accuracy Requirements	829

9.1.6.1	Absolute RSRQ Accuracy	829

9.1.6.2	Relative Accuracy of RSRQ	830

9.1.6.3	Absolute WB-RSRQ Accuracy	831

9.1.6.4	Relative WB-RSRQ Accuracy	831

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	832

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	833

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	833

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	833

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	834

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	835

9.1.6B.1	Introduction	835

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	835

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	835

9.1.7	RSRQ Measurement Report Mapping	836

9.1.8	Power Headroom	836

9.1.8.1	Period	837

9.1.8.2	Reporting Delay	837

9.1.8.3	Void	837

9.1.8.4	Report Mapping	837

9.1.8A	Power Headroom for UE category M1 for satellite access	837

9.1.8A.1	Period	838

9.1.8A.2	Reporting Delay	838

9.1.8A.3	Void	838

9.1.8A.4	Report Mapping	838

9.1.9	UE Rx – Tx time difference	838

9.1.9.1	Measurement Requirement	838

9.1.9.2	Measurement Report mapping	839

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	840

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	841

9.1.10	Reference Signal Time Difference (RSTD)	842

9.1.10.1	Intra-Frequency Accuracy Requirement	842

9.1.10.2	Inter-Frequency Accuracy Requirement	843

9.1.10.3	RSTD Measurement Report Mapping	845

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	845

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	846

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	847

9.1.11	Carrier aggregation measurement accuracy	848

9.1.11.1	Primary component carrier accuracy requirement	849

9.1.11.2	Secondary component carrier accuracy requirement	849

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	849

9.1.11.4	Secondary component carrier relative accuracy requirement	849

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	849

9.1.13	Measurement accuracy for UE category 0	850

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	850

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	850

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	851

9.1.14	Accuracy requirements for Discovery Signal Measurements	852

9.1.14.1	Introduction	852

9.1.14.2	RSRP measurements in discovery signal occasions	852

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	852

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	852

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	852

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	853

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	854

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	854

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	854

9.1.14.3.3	CSI-RSRP measurement report mapping	855

9.1.14.4	RSRQ measurements in discovery signal occasions	855

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	855

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	856

9.1.15.1.1	Primary component carrier accuracy requirement	856

9.1.15.1.2	Secondary component carrier accuracy requirement	856

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	856

9.1.15.1.4	Secondary component carrier relative accuracy requirement	856

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	856

9.1.15.2.1	Primary component carrier accuracy requirement	856

9.1.15.2.2	Secondary component carrier accuracy requirement	856

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	856

9.1.15.2.4	Secondary component carrier relative accuracy requirement	856

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	857

9.1.17	RS-SINR Measurements	857

9.1.17.1	Measurement Report Mapping	857

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	858

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	858

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	858

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	858

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	859

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	860

9.1.18.1	Introduction	860

9.1.18.2	RSRP measurements	860

9.1.18.2.1	RSRP measurement report mapping	860

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	860

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	861

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	861

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	862

9.1.18.3	RSRQ measurements	862

9.1.18.3.1	RSRQ measurement report mapping	862

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	862

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	863

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	863

9.1.18.4	CSI-RSRP measurements	864

9.1.18.4.1	CSI-RSRP measurement report mapping	864

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	864

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	864

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	865

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	865

9.1.18.5	RSSI measurements	866

9.1.18.5.1	RSSI measurement report mapping	866

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	866

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	867

9.1.18.6	Channel occupancy measurements	867

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	867

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	867

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	868

9.1.19.1	Introduction	868

9.1.19.2	Accuracy requirements for measurements on SCC	868

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	868

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	868

9.1.20	SFN and Subframe Time Difference (SSTD)	869

9.1.20.1	SSTD Accuracy Requirement	869

9.1.20.2	SSTD Measurement Report Mapping	869

9.1.21	Measurement accuracy for UE category M1	870

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	870

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	872

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	873

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	875

9.1.21.5	RSRP Measurement Report Mapping	876

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	876

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	877

9.1.21.8	RSRQ Measurement Report Mapping	878

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	878

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	879

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	880

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	881

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	882

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	883

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	884

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	885

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	886

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	887

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	888

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	889

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	892

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	895

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	895

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	898

9.1.21A	Measurement accuracy for UE category M1 for satellite access	899

9.1.21A.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	900

9.1.21A.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	901

9.1.21A.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	902

9.1.21A.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	904

9.1.21A.5	RSRP Measurement Report Mapping	905

9.1.21A.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	905

9.1.21A.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	906

9.1.21A.8	RSRQ Measurement Report Mapping	907

9.1.21A.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	907

9.1.21A.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	908

9.1.21A.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	909

9.1.21A.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	910

9.1.21A.13	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	911

9.1.21A.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	912

9.1.21A.15	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	913

9.1.21A.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	914

9.1.21A.17	Downlink Channel Report Mapping for UE Category M1	915

9.1.21A.18	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	915

9.1.21A.19	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	917

9.1.22	Measurement accuracy for UE Category NB1	918

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	918

9.1.22.2	Void	919

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	919

9.1.22.4	Void	920

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	920

9.1.22.6	Void	921

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	921

9.1.22.8	Void	922

9.1.22.9	NRSRP Measurement Report Mapping	922

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	922

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	923

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	924

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	925

9.1.22.14	NRSRQ Measurement Report Mapping	926

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	927

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	927

9.1.22.17	Channel quality reporting for UE Category NB2 with 16-QAM	928

9.1.22A	Measurement accuracy for UE Category NB1 for satellite access	928

9.1.22A.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	928

9.1.22A.2	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	929

9.1.22A.3	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	930

9.1.22A.4	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	931

9.1.22A.5	NRSRP Measurement Report Mapping	932

9.1.22A.6	NRSRQ Measurement Report Mapping	932

9.1.22A.7	MSG3-based Measurement Report Mapping for UE Category NB1	932

9.1.22A.8	Downlink Channel Quality Measurement Accuracy for UE Category NB1	932

9.1.23	Power Headroom for UE Category NB1	933

9.1.23.1	Period	933

9.1.23.2	Reporting Delay	933

9.1.23.3	Report Mapping for UE Category NB1	933

9.1.23.3.1	Void	935

9.1.23.3.2	Void	935

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	935

9.1.23A	Power Headroom for UE Category NB1 for Satellite Access	936

9.1.23A.1	Period	936

9.1.23A.2	Reporting Delay	936

9.1.23A.3	Report Mapping for UE Category NB1 for Satellite Access	937

9.1.23A.3.1	Void	937

9.1.23A.3.2	Void	937

9.1.23A.4	Report Mapping for UE Category NB1 for UE Power Class 6 for Satellite Access	937

9.1.24	Void	937

9.1.25	Measurement accuracy for UE category M2	937

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	937

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	938

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	939

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	940

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	942

9.1.26	Measurement Accuracy for non-BL CE UE	943

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	944

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	946

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	946

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	948

9.1.26.5	RSRP Measurement Report Mapping	949

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	949

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	949

9.1.26.8	RSRQ Measurement Report Mapping	950

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	950

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	951

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	952

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	953

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	954

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	954

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	954

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	955

9.1.27	SFN and frame Timing Difference (SFTD)	956

9.1.27.1	SFTD Accuracy Requirement	956

9.1.28	SFN and Frame Timing Difference (SFTD) under CCA	958

9.1.28.1	SFTD Accuracy Requirement under CCA	958

9.2	UTRAN FDD Measurements	959

9.2.1	UTRAN FDD CPICH RSCP	959

9.2.2	Void	960

9.2.3	UTRAN FDD CPICH Ec/No	960

9.3	UTRAN TDD Measurements	960

9.3.1	UTRAN TDD P-CCPCH RSCP	961

9.3.2	Void	961

9.3.3	Void	961

9.4	GSM Measurements	961

9.4.1	GSM carrier RSSI	961

9.5	CDMA2000 1x RTT Measurements	961

9.5.1	CDMA2000 1x RTT Pilot Strength	961

9.6	PCMAX,c	962

9.6.1	Report Mapping	962

9.6.2 	Estimation Period	962

9.6.3	Reporting Delay	962

9.7	IEEE802.11 Measurements	962

9.7.1	WLAN RSSI	962

9.7.2	WLAN RSSI Measurement Report Mapping	962

9.8	MBSFN Measurements	963

9.8.1	Introduction	963

9.8.2	MBSFN RSRP	963

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	963

9.8.2.2	MBSFN RSRP measurement report mapping	964

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	964

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	965

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	965

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	965

9.8.3	MBSFN RSRQ	966

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	966

9.8.3.2	MBSFN RSRQ measurement report mapping	966

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	967

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	967

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	967

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	968

9.8.4	MCH BLER	968

9.8.4.1	Measurement report mapping for MCH BLER	968

9.8.4.2	Measurement report mapping for MCH Block Number	969

9.9	ProSe Measurements	970

9.9.1	Introduction	970

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	970

9.9.2.1	Absolute S-RSRP Accuracy	970

9.9.2.2	Relative Accuracy of S-RSRP	971

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	972

9.9.3.1	Absolute SD-RSRP Accuracy	972

9.9.3.2	Relative Accuracy of SD-RSRP	972

9.10	V2X Measurements	973

9.10.1	Introduction	973

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	973

9.10.2.1	Absolute S-RSRP Accuracy	973

9.10.2.2	Relative Accuracy of S-RSRP	974

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	974

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	974

9.10.4	S-RSSI Measurement Accuracy Requirements	975

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	975

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	975

9.11	NR Measurements	976

9.11.1	NR SS-RSRP Measurements	976

9.11.1A	NR SS-RSRP Measurements for DC Idle Mode Measurements	976

9.11.2	NR SS-RSRQ Measurements	977

9.11.2A	NR SS-RSRQ Measurements for DC Idle Mode Measurements	977

9.11.3	NR SS-SINR Measurements	977

9.11.4	NR SS-RSRP Measurements under CCA	978

9.11.5	NR SS-RSRQ Measurements under CCA	978

9.11.6	NR SS-SINR Measurements under CCA	978

9.11.7	NR RSSI Measurements under CCA	978

9.11.8	NR Channel Occupancy Measurements under CCA	979

10	Measurements Performance Requirements for E-UTRAN	979

10.1	Received Interference Power	979

10.1.1	Absolute accuracy requirement	979

10.1.2	Relative accuracy requirement	979

10.1.3	Received Interference Power measurement report mapping	980

10.2	Angle of Arrival (AOA)	980

10.2.1	Range/mapping	980

10.3	Timing Advance (TADV)	980

10.3.1	Report mapping	980

11	ProSe Requirements in Any Cell Selection state	981

11.1	Introduction	981

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	981

11.2.1	Introduction	981

11.2.2	ProSe UE transmission timing	981

11.3	Initiation/Cease of SLSS Transmissions	982

11.3.1	Introduction	982

11.3.2	Requirements	982

11.4	Measurements for ProSe in Any Cell Selection State	982

11.4.1	Introduction	982

11.4.2	Requirements	982

11.4.2.1	E-UTRA FDD	982

11.4.2.2	E-UTRA TDD	983

11.5	Selection / Reselection of ProSe Synchronization Reference	983

11.5.1	Introduction	983

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	983

11.5.2.1	Introduction	983

11.5.2.2	Requirements	983

11.6	Void	984

11.7	Selection / Reselection of ProSe relay UE	984

11.7.1	Introduction	984

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	984

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	984

12.1	Introduction	984

12.2	Transmit Timing	985

12.2.1	GNSS as timing reference	985

12.3	Interruption	985

12.4	Reliability of GNSS signal	985

13	V2X Requirements	985

13.1	Introduction	985

13.2	UE Transmit Timing	986

13.2.1	Introduction	986

13.2.2	GNSS as synchronization reference source	986

13.2.3	Serving cell/PCell as synchronization reference source	986

13.2.4	SyncRef UE as synchronization reference source	986

13.3	Initiation/Cease of SLSS Transmissions	986

13.3.1	Introduction	986

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	986

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	987

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	987

13.4	Selection / Reselection of V2X Synchronization Reference Source	988

13.5	Autonomous Resource Selection/Reselection measurements	989

13.5.1	Introduction	989

13.5.2	PSSCH-RSRP measurements	989

13.5.3	S-RSSI measurements	989

13.6	Congestion Control measurements	989

13.7	Interruption	989

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	989

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	989

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	990

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	990

13.8	Reliability of GNSS signal	990

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	991

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	991

Annex A (normative):	Test Cases	992

A.1	Purpose of annex	992

A.2	Requirement classification for statistical testing	992

A.2.1	Types of requirements in TS 36.133	992

A.2.1.1	Time and delay requirements on UE higher layer actions	992

A.2.1.2	Measurements of power levels, relative powers and time	992

A.2.1.3	Implementation requirements	993

A.2.1.4	Physical layer timing requirements	993

A.3	RRM test configurations	994

A.3.1	Reference Measurement Channels	994

A.3.1.1	PDSCH	994

A.3.1.1.1	FDD	994

A.3.1.1.2	TDD	999

A.3.1.1.3	FDD for UE category 0	1002

A.3.1.1.4	HD-FDD for UE category 0	1003

A.3.1.1.5	TDD for UE category 0	1004

A.3.1.1.6	Frame Structure 3	1005

A.3.1.2	PCFICH/PDCCH/PHICH	1006

A.3.1.2.1	FDD	1006

A.3.1.2.2	TDD	1006

A.3.1.2.3	HD-FDD for UE category 0	1007

A.3.1.2.4	FS 3	1007

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	1007

A.3.1.3.1	FDD in CEModeA	1008

A.3.1.3.2	HD-FDD in CEModeA	1008

A.3.1.3.3	TDD in CEModeA	1009

A.3.1.3.4	FDD in CEModeB	1009

A.3.1.3.5	HD-FDD in CEModeB	1010

A.3.1.3.6	TDD in CEModeB	1010

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	1011

A.3.1.4.1	FDD in CEModeA	1011

A.3.1.4.2	HD-FDD in CEModeA	1012

A.3.1.4.3	TDD in CEModeA	1013

A.3.1.4.4	FDD in CEModeB	1014

A.3.1.4.5	HD-FDD in CEModeB	1015

A.3.1.4.6	TDD in CEModeB	1015

A.3.1.5	NPDSCH Reference Channel for UE category NB1	1016

A.3.1.5.1	HD-FDD in-band operation	1016

A.3.1.5.2	Void	1017

A.3.1.5.3	HD-FDD standalone operation	1017

A.3.1.5.4	Void	1018

A.3.1.5.5	HD-FDD guard band operation	1018

A.3.1.5.6	Void	1019

A.3.1.5.7	TDD in-band operation	1019

A.3.1.5.8	TDD standalone operation	1019

A.3.1.5.9	TDD guard band operation	1020

A.3.1.6	NPDCCH Reference Channel for UE category NB1	1020

A.3.1.6.1	In-band operation	1020

A.3.1.6.2	Void	1021

A.3.1.6.3	Standalone operation	1021

A.3.1.6.4	Void	1021

A.3.1.6.5	Guard band operation	1021

A.3.1.6.6	Void	1022

A.3.2	OFDMA Channel Noise Generator (OCNG)	1022

A.3.2.1	OCNG Patterns for FDD	1022

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	1023

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	1023

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	1024

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	1025

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	1025

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	1026

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	1026

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	1026

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	1027

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	1028

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	1028

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	1029

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	1029

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	1030

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	1030

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	1031

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	1031

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	1032

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	1032

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	1033

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	1033

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	1034

A.3.2.2	OCNG Patterns for TDD	1034

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	1035

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	1035

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	1036

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	1036

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	1037

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	1038

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	1039

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	1040

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	1040

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	1041

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	1042

A.3.2.3	OCNG Patterns for Narrowband IoT	1042

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	1044

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	1045

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	1045

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	1046

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	1047

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	1048

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	1049

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	1050

A.3.2.4	OCNG Patterns for V2X sidelink	1051

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	1051

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	1052

A.3.3	Reference DRX Configurations	1052

A.3.4	ABS Transmission Configurations	1053

A.3.4.1	Non-MBSFN ABS Transmission Configurations	1053

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	1053

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	1053

A.3.4.2	MBSFN ABS Transmission Configurations	1054

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	1054

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	1055

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	1055

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	1055

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	1056

A.3.6.1	Introduction	1056

A.3.7	Test Cases with Different Channel Bandwidths	1056

A.3.7.1	Introduction	1056

A.3.7.2	Principle of testing	1056

A.3.8	Antenna Configuration	1056

A.3.8.1	Antenna connection for 4 Rx capable UEs	1056

A.3.8.1.1 Introduction	1056

A.3.8.1.2 Principle of testing	1056

A.3.8.1.2.1 Single carrier tests	1056

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	1057

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	1058

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	1058

A.3.8.2	Antenna connection for 8 Rx capable UEs	1058

A.3.8.2.1	Introduction	1058

A.3.8.2.2	Principle of testing	1058

A.3.8.2.2.1	Single carrier tests	1058

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	1059

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	1059

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	1059

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	1059

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	1059

A.3.9.1	Introduction	1059

A.3.9.2	Principle of testing	1059

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	1060

A.3.10.1	Introduction	1060

A.3.10.2	Principle of testing	1060

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	1060

A.3.11.1	Introduction	1060

A.3.11.2	Principle of Testing	1060

A.3.12	Proximity-based Services	1060

A.3.12.1	Introduction	1060

A.3.12.2	Reference DRX configurations for ProSe tests	1060

A.3.12.3	Test Cases with Different Channel Bandwidths	1061

A.3.12.3.1	Introduction	1061

A.3.12.3.2	Principle of testing	1061

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	1061

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	1068

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	1071

A.3.12.6.1	FDD	1071

A.3.12.7	Reference measurement channels for ProSe Direct Communication	1071

A.3.12.7.1	FDD	1071

A.3.12.8	ProSe Receive Traffic Generator	1072

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	1072

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	1072

A.3.13	Time Offset between Cells	1073

A.3.13.1	Introduction	1073

A.3.13.2	Definition	1073

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	1073

A.3.14.1	Introduction	1073

A.3.14.2	Principle of testing	1073

A.3.15	Dual connectivity test cases with different combination of duplex mode	1073

A.3.15.1	Introduction	1073

A.3.15.2	Principle of testing	1073

A.3.16	Reference PRACH Configurations	1074

A.3.17	Listen before talk model	1074

A.3.17.1	Introduction	1074

A.3.17.2	Definition	1074

A.3.18	Reference NPRACH Configurations	1075

A.3.19	Dual connectivity test cases with different bandwidth combinations	1076

A.3.19.1	Introduction	1076

A.3.19.2	Principle of testing	1076

A.3.20	Category M1 UE Test Cases	1077

A.3.20.1	Introduction	1077

A.3.20.2	Principle of Cat-M1 UE Testing	1077

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	1078

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	1079

A.3.21.1	Introduction	1079

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	1079

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	1080

A.3.22	Category 1bis UE Test Cases	1081

A.3.22.1	Introduction	1081

A.3.22.2	Principle of Category 1bis UE Testing	1081

A.3.23	Category NB2 UE Test Cases	1085

A.3.23.1	Introduction	1085

A.3.23.2	Principle of Category NB2 UE Testing	1085

A.3.24	V2X sidelink communication	1088

A.3.24.1	Introduction	1088

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	1089

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	1093

A.3.25	Category M2 UE Test Cases	1094

A.3.25.1	Introduction	1094

A.3.25.2	Principle of Cat-M2 UE Testing	1094

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	1095

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	1096

A.3.26.1	Introduction	1096

A.3.26.2	Principle of testing	1096

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	1096

A.3.27.1	Introduction	1096

A.3.27.2	Principle of INACTIVE cell re-selection Testing	1096

A.3.28	Testing related to Satellite access	1096

A.3.28.1	Introduction	1096

A.3.28.2	Principle of testing GSO and NGSO scenarios	1096

A.3.28.2	Principle of testing different RRM requirements	1097

A.3.28.3	Principle of testing different ephemeris formats	1101

A.3.28.4	General setup for SIB31/SIB-31-NB	1101

A.3.28.5	Satellite specific parameters configuration	1101

A.3.28.5.1	Satellite specific configuration for serving cell	1101

A.3.28.5.2	Satellite specific configuration for neighbour cell	1101

A.4	E-UTRAN RRC_IDLE state	1102

A.4.2	Cell Re-Selection	1102

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	1102

A.4.2.1.1	Test Purpose and Environment	1102

A.4.2.1.2	Test Requirements	1104

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	1105

A.4.2.2.1	Test Purpose and Environment	1105

A.4.2.2.2	Test Requirements	1106

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	1107

A.4.2.3.1	Test Purpose and Environment	1107

A.4.2.3.2	Test Requirements	1108

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	1109

A.4.2.4.1	Test Purpose and Environment	1109

A.4.2.4.2	Test Requirements	1110

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	1111

A.4.2.5.1	Test Purpose and Environment	1111

A.4.2.5.2	Test Requirements	1112

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	1113

A.4.2.6.1	Test Purpose and Environment	1113

A.4.2.6.2	Test Requirements	1114

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	1115

A.4.2.7.1	Test Purpose and Environment	1115

A.4.2.7.2	Test Requirements	1116

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	1117

A.4.2.8.1	Test Purpose and Environment	1117

A.4.2.8.2	Test Requirements	1118

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	1119

A.4.2.9.1	Test Purpose and Environment	1119

A.4.2.9.2	Test Requirements	1119

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	1119

A.4.2.10.1	Test Purpose and Environment	1119

A.4.2.10.2	Test Requirements	1123

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	1123

A.4.2.11.1	Test Purpose and Environment	1123

A.4.2.11.2	Test Requirements	1127

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	1127

A.4.2.12.1	Test Purpose and Environment	1127

A.4.2.12.2	Test Requirements	1129

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	1130

A.4.2.13.1	Test Purpose and Environment	1130

A.4.2.13.2	Test Requirements	1131

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1132

A.4.2.14.1	Test Purpose and Environment	1132

A.4.2.14.2	Test Requirements	1133

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	1134

A.4.2.15.1	Test Purpose and Environment	1134

A.4.2.15.2	Test Requirements	1135

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	1136

A.4.2.16.1	Test Purpose and Environment	1136

A.4.2.16.2	Test Requirements	1137

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	1138

A.4.2.17.1	Test Purpose and Environment	1138

A.4.2.17.2	Test Requirements	1139

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	1140

A.4.2.18.1	Test Purpose and Environment	1140

A.4.2.18.2	Test Requirements	1142

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1143

A.4.2.19.1	Test Purpose and Environment	1143

A.4.2.19.2	Test Requirements	1145

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	1146

A.4.2.20.1	Test Purpose and Environment	1146

A.4.2.20.2	Test Requirements	1147

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	1148

A.4.2.21.1	Test Purpose and Environment	1148

A.4.2.21.2	Test Requirements	1149

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	1150

A.4.2.22.1	Test Purpose and Environment	1150

A.4.2.22.2	Test Requirements	1151

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	1152

A.4.2.23.1	Test Purpose and Environment	1152

A.4.2.23.2	Test Requirements	1153

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1154

A.4.2.24.1	Test Purpose and Environment	1154

A.4.2.24.2	Test Requirements	1157

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1158

A.4.2.25.1	Test Purpose and Environment	1158

A.4.2.25.2	Test Requirements	1159

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1160

A.4.2.26.1	Test Purpose and Environment	1160

A.4.2.26.2	Test Requirements	1161

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1162

A.4.2.27.1	Test Purpose and Environment	1162

A.4.2.27.2	Test Requirements	1163

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	1164

A.4.2.28.1	Test Purpose and Environment	1164

A.4.2.28.2	Test Requirements	1165

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	1166

A.4.2.29.1	Test Purpose and Environment	1166

A.4.2.29.2	Test Requirements	1167

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	1168

A.4.2.30.1	Test Purpose and Environment	1168

A.4.2.30.2	Test Requirements	1169

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	1170

A.4.2.31.1	Test Purpose and Environment	1170

A.4.2.31.2	Test Requirements	1171

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	1172

A.4.2.32.1	Test Purpose and Environment	1172

A.4.2.32.2	Test Requirements	1173

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	1174

A.4.2.33.1	Test Purpose and Environment	1174

A.4.2.33.2	Test Requirements	1175

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	1176

A.4.2.34.1	Test Purpose and Environment	1176

A.4.2.34.2	Test Requirements	1177

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	1178

A.4.2.35.1	Test Purpose and Environment	1178

A.4.2.35.2	Test Requirements	1180

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1181

A.4.2.36.1	Test Purpose and Environment	1181

A.4.2.36.2	Test Requirements	1183

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1184

A.4.2.37.1	Test Purpose and Environment	1184

A.4.2.37.2	Test Requirements	1186

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	1187

A.4.2.38.1	Test Purpose and Environment	1187

A.4.2.38.2	Test Requirements	1189

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1190

A.4.2.39.1	Test Purpose and Environment	1190

A.4.2.39.2	Test Requirements	1191

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1192

A.4.2.40.1	Test Purpose and Environment	1192

A.4.2.40.2	Test Requirements	1193

A.4.2.41 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1194

A.4.2.41.1	Test Purpose and Environment	1194

A.4.2.41.2	Test Requirements	1197

A.4.2.42	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1198

A.4.2.42.1	Test Purpose and Environment	1198

A.4.2.42.2	Test Requirements	1200

A.4.2.43	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1201

A.4.2.43.1	Test Purpose and Environment	1201

A.4.2.43.2	Test Requirements	1203

A.4.2.44	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1204

A.4.2.44.1	Test Purpose and Environment	1204

A.4.2.44.2	Test Requirements	1206

A.4.2.45	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1207

A.4.2.45.1	Test Purpose and Environment	1207

A.4.2.45.2	Test Requirements	1209

A.4.2.46	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1210

A.4.2.46.1	Test Purpose and Environment	1210

A.4.2.46.2	Test Requirements	1212

A.4.2.47	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation with UE specific DRX	1213

A.4.2.47.1	Test Purpose and Environment	1213

A.4.2.47.2	Test Requirements	1215

A.4.2.48	E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1216

A.4.2.48.1	Test Purpose and Environment	1216

A.4.2.48.2	Test Requirements	1217

A.4.2.49	E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1218

A.4.2.49.1	Test Purpose and Environment	1218

A.4.2.49.2	Test Requirements	1220

A.4.2.50	E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1221

A.4.2.50.1	Test Purpose and Environment	1221

A.4.2.50.2	Test Requirements	1223

A.4.2.51 E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1224

A.4.2.51.1	Test Purpose and Environment	1224

A.4.2.51.2	Test Requirements	1226

A.4.2.52 E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1227

A.4.2.52.1	Test Purpose and Environment	1227

A.4.2.52.2	Test Requirements	1229

A.4.2.53 E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1230

A.4.2.53.1	Test Purpose and Environment	1230

A.4.2.53.2	Test Requirements	1231

A.4.2.54	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1232

A.4.2.54.1	Test Purpose and Environment	1232

A.4.2.54.2	Test Requirements	1234

A.4.2.55	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1235

A.4.2.55.1	Test Purpose and Environment	1235

A.4.2.55.2	Test Requirements	1237

A.4.2.56	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1238

A.4.2.56.1	Test Purpose and Environment	1238

A.4.2.56.2	Test Requirements	1239

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	1240

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	1240

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	1240

A.4.3.1.1.1	Test Purpose and Environment	1240

A.4.3.1.1.2	Test Requirements	1242

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	1242

A.4.3.1.2.1	Test Purpose and Environment	1242

A.4.3.1.2.2	Test Requirements	1245

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	1245

A.4.3.1.3.1	Test Purpose and Environment	1245

A.4.3.1.3.2	Test Requirements	1248

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	1248

A.4.3.1.4.1	Test Purpose and Environment	1248

A.4.3.1.4.2	Test Requirements	1249

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	1249

A.4.3.1.5.1	Test Purpose and Environment	1249

A.4.3.1.5.2	Test Requirements	1252

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	1253

A.4.3.2.1	Test Purpose and Environment	1253

A.4.3.2.1.1	Void	1253

A.4.3.2.1.2	1.28Mcps TDD option	1253

A.4.3.2.1.3	Void	1255

A.4.3.2.2	Test Requirements	1255

A.4.3.2.2.1	1.28Mcps TDD option	1255

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	1255

A.4.3.2A.1	Test Purpose and Environment	1255

A.4.3.2A.2	Test Requirements	1259

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1259

A.4.3.3.1	Test Purpose and Environment	1259

A.4.3.3.2	Test Requirements	1262

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1262

A.4.3.3A.1	Test Purpose and Environment	1262

A.4.3.3A.2	Test Requirements	1267

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1268

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1268

A.4.3.4.1.1	Test Purpose and Environment	1268

A.4.3.4.1.2	Test Requirements	1270

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1270

A.4.3.4.2.1	Test Purpose and Environment	1270

A.4.3.4.2.2	Test Requirements	1272

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1272

A.4.3.4.3.1	Test Purpose and Environment	1272

A.4.3.4.3.2	Test Requirements	1275

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1275

A.4.3.4.4.1	Test Purpose and Environment	1275

A.4.3.4.4.2	Test Requirements	1279

A.4.4	E-UTRAN to GSM Cell Re-Selection	1279

A.4.4.1	E-UTRAN FDD – GSM:	1279

A.4.4.1.1	Test Purpose and Environment	1279

A.4.4.1.2	Test Requirements	1281

A.4.4.2	E-UTRAN TDD – GSM:	1281

A.4.4.2.1	Test Purpose and Environment	1281

A.4.4.2.2	Test Requirements	1283

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1284

A.4.5.1	E-UTRAN FDD – HRPD	1284

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1284

A.4.5.1.1.1	Test Purpose and Environment	1284

A.4.5.1.1.2	Test Requirements	1286

A.4.5.2	E-UTRAN TDD – HRPD	1286

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1286

A.4.5.2.1.1	Test Purpose and Environment	1286

A.4.5.2.1.2	Test Requirements	1289

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1289

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1289

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1289

A.4.6.1.1.1	Test Purpose and Environment	1289

A.4.6.1.1.2	Test Requirements	1292

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1292

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1292

A.4.6.2.1.1	Test Purpose and Environment	1292

A.4.6.2.1.2	Test Requirements	1295

A.4.7	Idle State Positioning Measurement for UE category NB1	1295

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1295

A.4.7.1.1	Test Purpose and Environment	1295

A.4.7.1.2	Test Requirements	1300

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1301

A.4.7.2.1	Test Purpose and Environment	1301

A.4.7.4.2	Test Requirements	1305

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1306

A.4.7.3.1	Test Purpose and Environment	1306

A.4.7.3.2	Test Requirements	1310

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1311

A.4.7.4.1	Test Purpose and Environment	1311

A.4.7.4.2	Test Requirements	1315

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1316

A.5.1	E-UTRAN Handover	1316

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1316

A.5.1.1.1	Test Purpose and Environment	1316

A.5.1.1.2	Test Requirements	1318

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1318

A.5.1.2.1	Test Purpose and Environment	1318

A.5.1.2.2	Test Requirements	1320

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1320

A.5.1.3.1	Test Purpose and Environment	1320

A.5.1.3.2	Test Requirements	1322

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1322

A.5.1.4.1	Test Purpose and Environment	1322

A.5.1.4.2	Test Requirements	1324

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1324

A.5.1.5.1	Test Purpose and Environment	1324

A.5.1.5.2	Test Requirements	1326

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1326

A.5.1.6.1	Test Purpose and Environment	1326

A.5.1.6.2	Test Requirements	1328

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1328

A.5.1.7.1	Test Purpose and Environment	1328

A.5.1.7.2	Test Requirements	1331

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1331

A.5.1.8.1	Test Purpose and Environment	1331

A.5.1.8.2	Test Requirements	1334

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1334

A.5.1.9.1	Test Purpose and Environment	1334

A.5.1.9.2	Test Requirements	1335

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1335

A.5.1.10.1	Test Purpose and Environment	1335

A.5.1.10.2	Test Requirements	1337

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1337

A.5.1.11.1	Test Purpose and Environment	1337

A.5.1.11.2	Test Requirements	1339

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1339

A.5.1.12.1	Test Purpose and Environment	1339

A.5.1.12.2	Test Requirements	1341

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1341

A.5.1.13.1	Test Purpose and Environment	1341

A.5.1.13.2	Test Requirements	1343

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1344

A.5.1.14.1	Test Purpose and Environment	1344

A.5.1.14.2	Test Requirements	1345

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1346

A.5.1.15.1	Test Purpose and Environment	1346

A.5.1.15.2	Test Requirements	1347

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1348

A.5.1.16.1	Test Purpose and Environment	1348

A.5.1.16.2	Test Requirements	1349

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1350

A.5.1.17.1	Test Purpose and Environment	1350

A.5.1.17.2	Test Requirements	1351

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1352

A.5.1.18.1	Test Purpose and Environment	1352

A.5.1.18.2	Test Requirements	1353

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1354

A.5.1.19.1	Test Purpose and Environment	1354

A.5.1.19.2	Test Requirements	1355

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1356

A.5.1.20.1	Test Purpose and Environment	1356

A.5.1.20.2	Test Requirements	1357

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1358

A.5.1.21.1	Test Purpose and Environment	1358

A.5.1.21.2	Test Requirements	1359

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1359

A.5.1.22.1	Test Purpose and Environment	1359

A.5.1.22.2	Test Requirements	1361

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1361

A.5.1.23.1	Test Purpose and Environment	1361

A.5.1.23.2	Test Requirements	1363

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1363

A.5.1.24.1	Test Purpose and Environment	1363

A.5.1.24.2	Test Requirements	1365

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1365

A.5.1.25.1	Test Purpose and Environment	1365

A.5.1.25.2	Test Requirements	1367

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1368

A.5.1.26.1	Test Purpose and Environment	1368

A.5.1.26.2	Test Requirements	1369

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1370

A.5.1.27.1	Test Purpose and Environment	1370

A.5.1.27.2	Test Requirements	1371

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1372

A.5.1.28.1	Test Purpose and Environment	1372

A.5.1.28.2	Test Requirements	1373

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1374

A.5.1.29.1	Test Purpose and Environment	1374

A.5.1.29.2	Test Requirements	1375

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1376

A.5.1.30.1	Test Purpose and Environment	1376

A.5.1.30.2	Test Requirements	1377

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1378

A.5.1.31.1	Test Purpose and Environment	1378

A.5.1.31.2	Test Requirements	1379

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1380

A.5.1.32.1	Test Purpose and Environment	1380

A.5.1.32.2	Test Requirements	1381

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1382

A.5.1.33.1	Test Purpose and Environment	1382

A.5.1.13.2	Test Requirements	1383

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1384

A.5.1.34.1	Test Purpose and Environment	1384

A.5.1.34.2	Test Requirements	1385

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1386

A.5.1.35.1	Test Purpose and Environment	1386

A.5.1.35.2	Test Requirements	1387

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1388

A.5.1.36.1	Test Purpose and Environment	1388

A.5.1.36.2	Test Requirements	1389

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1390

A.5.1.37.1	Test Purpose and Environment	1390

A.5.1.37.2	Test Requirements	1391

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1392

A.5.1.38.1	Test Purpose and Environment	1392

A.5.1.38.2	Test Requirements	1393

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1394

A.5.1.39.1	Test Purpose and Environment	1394

A.5.1.39.2	Test Requirements	1395

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1396

A.5.1.40.1	Test Purpose and Environment	1396

A.5.1.40.2	Test Requirements	1397

A.5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover	1398

A.5.1.41.1	Test Purpose and Environment	1398

A.5.1.41.2	Test Requirements	1400

A.5.1.42	E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover	1401

A.5.1.42.1	Test Purpose and Environment	1401

A.5.1.42.2	Test Requirements	1402

A.5.1.43	E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover	1403

A.5.1.43.1	Test Purpose and Environment	1403

A.5.1.43.2	Test Requirements	1404

A.5.1.44	E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover	1405

A.5.1.44.1	Test Purpose and Environment	1405

A.5.1.44.2	Test Requirements	1406

A.5.1.45	E-UTRAN FDD - FDD Intra frequency DAPS handover	1407

A.5.1.45.1	Test Purpose and Environment	1407

A.5.1.45.1	Test Requirements	1408

A.5.1.46	E-UTRAN TDD - TDD Intra frequency DAPS handover	1409

A.5.1.46.1	Test Purpose and Environment	1409

A.5.1.46.2	Test Requirements	1410

A.5.1.47	E-UTRAN FDD - FDD Intra frequency conditional handover	1411

A.5.1.47.1	Test Purpose and Environment	1411

A.5.1.47.2	Test Requirements	1412

A.5.1.48	E-UTRAN TDD - TDD Intra frequency conditional handover	1413

A.5.1.48.1	Test Purpose and Environment	1413

A.5.1.48.2	Test Requirements	1414

A.5.1.49	E-UTRAN FDD - FDD Inter frequency conditional handover	1415

A.5.1.49.1	Test Purpose and Environment	1415

A.5.1.49.2	Test Requirements	1416

A.5.1.50	E-UTRAN TDD - TDD Inter frequency conditional handover	1417

A.5.1.50.1	Test Purpose and Environment	1417

A.5.1.50.2	Test Requirements	1418

A.5.1.51	E-UTRAN FDD - TDD Inter frequency conditional handover	1419

A.5.1.51.1	Test Purpose and Environment	1419

A.5.1.51.2	Test Requirements	1421

A.5.1.52	E-UTRAN TDD - FDD Inter frequency conditional handover	1421

A.5.152.1	Test Purpose and Environment	1421

A.5.1.52.2	Test Requirements	1424

A.5.1.53	E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover	1425

A.5.1.53.1	Test Purpose and Environment	1425

A.5.1.53.2	Test Requirements	1427

A.5.1.54	E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover	1428

A.5.1.54.1	Test Purpose and Environment	1428

A.5.1.54.2	Test Requirements	1430

A.5.1.55	E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover	1431

A.5.1.55.1	Test Purpose and Environment	1431

A.5.1.56	E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover	1435

A.5.1.56.1	Test Purpose and Environment	1435

A.5.1.56.2	Test Requirements	1438

A.5.1.57	E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover	1439

A.5.1.57.1	Test Purpose and Environment	1439

A.5.1.57.2	Test Requirements	1441

A.5.1.58	E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover	1441

A.5.1.58.1	Test Purpose and Environment	1441

A.5.1.58.2	Test Requirements	1444

A.5.2	E-UTRAN Handover to other RATs	1444

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1444

A.5.2.1.1	Test Purpose and Environment	1444

A.5.2.1.2	Test Requirements	1446

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1447

A.5.2.2.1	Test Purpose and Environment	1447

A.5.2.2.2	Test Requirements	1450

A.5.2.3 E-UTRAN FDD- GSM Handover	1450

A.5.2.3.1	Test Purpose and Environment	1450

A.5.2.3.2	Test Requirements	1451

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1452

A.5.2.4.1	Test Purpose and Environment	1452

A.5.2.4.1.1	Void	1452

A.5.2.4.1.2	1.28 Mcps TDD option	1452

A.5.2.4.1.3	Void	1454

A.5.2.4.2	Test Requirements	1454

A.5.2.4.2.1	Void	1454

A.5.2.4.2.2	1.28 Mcps TDD option	1454

A.5.2.4.2.3	Void	1454

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1454

A.5.2.5.1	Test Purpose and Environment	1454

A.5.2.5.1.3	Void	1457

A.5.2.5.2	Test Requirements	1457

A.5.2.5.2.1	Void	1457

A.5.2.5.2.2	1.28 Mcps TDD option	1457

A.5.2.5.2.3	Void	1457

A.5.2.6	E-UTRAN TDD - GSM Handover	1457

A.5.2.6.1	Test Purpose and Environment	1457

A.5.2.6.2	Test Requirements	1459

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1460

A.5.2.7.1	Test Purpose and Environment	1460

A.5.2.7.2	Test Requirements	1462

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1462

A.5.2.8.1	Test Purpose and Environment	1462

A.5.2.8.2	Test Requirements	1463

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1464

A.5.2.9.1	Test Purpose and Environment	1464

A.5.2.9.2	Test Requirements	1465

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1466

A.5.2.10.1	Test Purpose and Environment	1466

A.5.2.10.2	Test Requirements	1468

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1468

A.5.2.10A.1	Test Purpose and Environment	1468

A.5.2.10A.2	Test Requirements	1471

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1471

A.5.2.10B.1	Test Purpose and Environment	1471

A.5.2.10B.2	Test Requirements	1474

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1474

A.5.2.11.1	Test Purpose and Environment	1474

A.5.2.11.2	Test Requirements	1474

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1475

A.5.3.1	E-UTRAN FDD – HRPD Handover	1475

A.5.3.1.1	Test Purpose and Environment	1475

A.5.3.1.2	Test Requirements	1477

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1477

A.5.3.2.1	Test Purpose and Environment	1477

A.5.3.2.2	Test Requirements	1480

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1480

A.5.3.3.1	Test Purpose and Environment	1480

A.5.3.3.2	Test Requirements	1483

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1483

A.5.3.4.1	Test Purpose and Environment	1483

A.5.3.4.2	Test Requirements	1485

A.5.3.5	E-UTRAN TDD – HRPD Handover	1485

A.5.3.5.1	Test Purpose and Environment	1485

A.5.3.5.2	Test Requirements	1488

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1488

A.5.3.6.1	Test Purpose and Environment	1488

A.5.3.6.2	Test Requirements	1491

A.6	RRC Connection Control	1491

A.6.1	RRC Re-establishment	1491

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1491

A.6.1.1.1	Test Purpose and Environment	1491

A.6.1.1.2	Test Requirements	1492

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1493

A.6.1.2.1	Test Purpose and Environment	1493

A.6.1.2.2	Test Requirements	1494

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1495

A.6.1.3.1	Test Purpose and Environment	1495

A.6.1.3.2	Test Requirements	1496

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1497

A.6.1.4.1	Test Purpose and Environment	1497

A.6.1.4.2	Test Requirements	1498

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1499

A.6.1.5.1	Test Purpose and Environment	1499

A.6.1.5.2	Test Requirements	1499

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1499

A.6.1.6.1	Test Purpose and Environment	1499

A.6.1.6.2	Test Requirements	1501

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1502

A.6.1.7.1	Test Purpose and Environment	1502

A.6.1.7.2	Test Requirements	1503

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1504

A.6.1.8.1	Test Purpose and Environment	1504

A.6.1.8.2	Test Requirements	1505

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1506

A.6.1.9.1	Test Purpose and Environment	1506

A.6.1.9.2	Test Requirements	1507

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1508

A.6.1.10.1	Test Purpose and Environment	1508

A.6.1.10.2	Test Requirements	1509

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1510

A.6.1.11.1	Test Purpose and Environment	1510

A.6.1.11.2	Test Requirements	1511

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1512

A.6.1.12.1	Test Purpose and Environment	1512

A.6.1.12.2	Test Requirements	1513

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1514

A.6.1.13.1	Test Purpose and Environment	1514

A.6.1.13.2	Test Requirements	1515

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1516

A.6.1.14.1	Test Purpose and Environment	1516

A.6.1.14.2	Test Requirements	1517

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1518

A.6.1.15.1	Test Purpose and Environment	1518

A.6.1.15.2	Test Requirements	1520

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1521

A.6.1.16.1	Test Purpose and Environment	1521

A.6.1.16.2	Test Requirements	1523

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1524

A.6.1.17.1	Test Purpose and Environment	1524

A.6.1.17.2	Test Requirements	1525

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1526

A.6.1.18.1	Test Purpose and Environment	1526

A.6.1.18.2	Test Requirements	1527

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1528

A.6.1.19.1	Test Purpose and Environment	1528

A.6.1.19.2	Test Requirements	1529

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1530

A.6.1.20.1	Test Purpose and Environment	1530

A.6.1.20.2	Test Requirements	1531

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1532

A.6.1.21.1	Test Purpose and Environment	1532

A.6.1.21.2	Test Requirements	1533

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1534

A.6.1.22.1	Test Purpose and Environment	1534

A.6.1.22.2	Test Requirements	1535

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1536

A.6.1.23.1	Test Purpose and Environment	1536

A.6.1.23.2	Test Requirements	1538

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1539

A.6.1.24.1	Test Purpose and Environment	1539

A.6.1.24.2	Test Requirements	1541

A.6.2	Random Access	1542

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1542

A.6.2.1.1	Test Purpose and Environment	1542

A.6.2.1.2	Test Requirements	1543

A.6.2.1.2.1	Random Access Response Reception	1543

A.6.2.1.2.2	No Random Access Response Reception	1543

A.6.2.1.2.3	Receiving a NACK on msg3	1543

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1544

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1544

A.6.2.1.2.6	Contention Resolution Timer expiry	1544

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1544

A.6.2.2.1	Test Purpose and Environment	1544

A.6.2.2.2	Test Requirements	1545

A.6.2.2.2.1	Random Access Response Reception	1546

A.6.2.2.2.2	No Random Access Response Reception	1546

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1546

A.6.2.3.1	Test Purpose and Environment	1546

A.6.2.3.2	Test Requirements	1548

A.6.2.3.2.1	Random Access Response Reception	1548

A.6.2.3.2.2	No Random Access Response reception	1548

A.6.2.3.2.3	Receiving a NACK on msg3	1548

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1549

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1549

A.6.2.3.2.6	Contention Resolution Timer expiry	1549

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1549

A.6.2.4.1	Test Purpose and Environment	1549

A.6.2.4.2	Test Requirements	1551

A.6.2.4.2.1	Random Access Response Reception	1551

A.6.2.4.2.2	No Random Access Response Reception	1551

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1551

A.6.2.5.1	Test Purpose and Environment	1551

A.6.2.5.2	Test Requirements	1552

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1552

A.6.2.6.1	Test Purpose and Environment	1552

A.6.2.6.2	Test Requirements	1552

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1553

A.6.2.7.1	Test Purpose and Environment	1553

A.6.2.7.2	Test Requirements	1555

A.6.2.7.2.1	Random Access Response Reception	1555

A.6.2.7.2.2	No Random Access Response Reception	1555

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1555

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1556

A.6.2.8.1	Test Purpose and Environment	1556

A.6.2.8.2	Test Requirements	1558

A.6.2.8.2.1	Random Access Response Reception	1558

A.6.2.8.2.2	No Random Access Response Reception	1558

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1558

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1559

A.6.2.9.1	Test Purpose and Environment	1559

A.6.2.9.2	Test Requirements	1562

A.6.2.9.2.1	Random Access Response Reception	1562

A.6.2.9.2.2	No Random Access Response Reception	1563

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1563

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1564

A.6.2.10.1	Test Purpose and Environment	1564

A.6.2.10.2	Test Requirements	1566

A.6.2.10.2.1	Random Access Response Reception	1566

A.6.2.10.2.2	No Random Access Response Reception	1567

A.6.2.10.2.3	Receiving a NACK on msg3	1567

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1567

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1567

A.6.2.10.2.6	Contention Resolution Timer expiry	1567

A.6.2.10.2.7	PRACH Resource Selection	1568

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1568

A.6.2.11.1	Test Purpose and Environment	1568

A.6.2.11.2	Test Requirements	1570

A.6.2.11.2.1	Random Access Response Reception	1570

A.6.2.11.2.2	No Random Access Response Reception	1571

A.6.2.11.2.3	Receiving a NACK on msg3	1571

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1571

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1571

A.6.2.11.2.6	Contention Resolution Timer expiry	1571

A.6.2.11.2.7	PRACH Resource Selection	1572

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1572

A.6.2.12.1	Test Purpose and Environment	1572

A.6.2.12.2	Test Requirements	1574

A.6.2.12.2.1	Random Access Response Reception	1574

A.6.2.12.2.2	No Random Access Response Reception	1575

A.6.2.12.2.3	Receiving a NACK on msg3	1575

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1575

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1575

A.6.2.12.2.6	Contention Resolution Timer expiry	1575

A.6.2.12.2.7	PRACH Resource Selection	1576

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1576

A.6.2.13.1	Test Purpose and Environment	1576

A.6.2.13.2	Test Requirements	1578

A.6.2.13.2.1	Random Access Response Reception	1578

A.6.2.13.2.2	No Random Access Response Reception	1579

A.6.2.13.2.3	Receiving a NACK on msg3	1579

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1579

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1579

A.6.2.13.2.6	Contention Resolution Timer expiry	1579

A.6.2.13.2.7	PRACH Resource Selection	1580

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1580

A.6.2.14.1	Test Purpose and Environment	1580

A.6.2.14.2	Test Requirements	1582

A.6.2.14.2.1	Random Access Response Reception	1582

A.6.2.14.2.2	No Random Access Response Reception	1583

A.6.2.14.2.3	Receiving a NACK on msg3	1583

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1583

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1583

A.6.2.14.2.6	Contention Resolution Timer expiry	1583

A.6.2.14.2.7	PRACH Resource Selection	1584

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1584

A.6.2.15.1	Test Purpose and Environment	1584

A.6.2.15.2	Test Requirements	1586

A.6.2.15.2.1	Random Access Response Reception	1586

A.6.2.15.2.2	No Random Access Response Reception	1587

A.6.2.15.2.3	Receiving a NACK on msg3	1587

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1587

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1587

A.6.2.15.2.6	Contention Resolution Timer expiry	1587

A.6.2.15.2.7	PRACH Resource Selection	1588

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1588

A.6.2.16.1	Test Purpose and Environment	1588

A.6.2.16.2	Test Requirements	1591

A.6.2.16.2.1	Random Access Response Reception	1591

A.6.2.16.2.2	No Random Access Response Reception	1592

A.6.2.16.2.3	Receiving a NACK on msg3	1592

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1592

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1592

A.6.2.16.2.6	Contention Resolution Timer expiry	1592

A.6.2.16.2.7	NPRACH Resource Selection	1592

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1593

A.6.2.17.1	Test Purpose and Environment	1593

A.6.2.17.2	Test Requirements	1596

A.6.2.17.2.1	Random Access Response Reception	1596

A.6.2.17.2.2	No Random Access Response Reception	1597

A.6.2.17.2.3	Receiving a NACK on msg3	1597

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1597

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1597

A.6.2.17.2.6	Contention Resolution Timer expiry	1597

A.6.2.17.2.7	NPRACH Resource Selection	1597

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1598

A.6.2.18.1	Test Purpose and Environment	1598

A.6.2.18.2	Test Requirements	1601

A.6.2.18.2.1	Random Access Response Reception	1601

A.6.2.18.2.2	No Random Access Response Reception	1602

A.6.2.18.2.3	Receiving a NACK on msg3	1602

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1602

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1602

A.6.2.18.2.6	Contention Resolution Timer expiry	1602

A.6.2.18.2.7	NPRACH Resource Selection	1602

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1603

A.6.2.19.1	Test Purpose and Environment	1603

A.6.2.19.2	Test Requirements	1605

A.6.2.19.2.1	Random Access Response Reception	1605

A.6.2.19.2.2	No Random Access Response Reception	1606

A.6.2.19.2.3	Receiving a NACK on msg3	1606

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1606

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1606

A.6.2.19.2.6	Contention Resolution Timer expiry	1606

A.6.2.19.2.7	NPRACH Resource Selection	1606

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1607

A.6.2.20.1	Test Purpose and Environment	1607

A.6.2.20.2	Test Requirements	1609

A.6.2.20.2.1	Random Access Response Reception	1609

A.6.2.20.2.2	No Random Access Response Reception	1610

A.6.2.20.2.3	Receiving a NACK on msg3	1610

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1610

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1610

A.6.2.20.2.6	Contention Resolution Timer expiry	1610

A.6.2.20.2.7	NPRACH Resource Selection	1610

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1611

A.6.2.21.1	Test Purpose and Environment	1611

A.6.2.21.2	Test Requirements	1613

A.6.2.21.2.1	Random Access Response Reception	1613

A.6.2.21.2.2	No Random Access Response Reception	1614

A.6.2.21.2.3	Receiving a NACK on msg3	1614

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1614

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1614

A.6.2.21.2.6	Contention Resolution Timer expiry	1614

A.6.2.21.2.7	NPRACH Resource Selection	1614

A.6.3	RRC Connection Release with Redirection	1615

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1615

A.6.3.1.1	Test Purpose and Environment	1615

A.6.3.1.2	Test Requirements	1617

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1617

A.6.3.2.1	Test Purpose and Environment	1617

A.6.3.2.2	Test Requirements	1619

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1619

A.6.3.3.1	Test Purpose and Environment	1619

A.6.3.3.2	Test Requirements	1620

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1621

A.6.3.4.1	Test Purpose and Environment	1621

A.6.3.4.2	Test Requirements	1622

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1623

A.6.3.5.1	Test Purpose and Environment	1623

A.6.3.5.2	Test Requirements	1625

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1625

A.6.3.6.1	Test Purpose and Environment	1625

A. 6.3.6.2	Test Requirements	1628

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1628

A.6.3.7.1	Test Purpose and Environment	1628

A.6.3.7.2	Test Requirements	1631

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1631

A.6.3.8.1	Test Purpose and Environment	1631

A.6.3.8.2	Test Requirements	1634

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1634

A.6.3.9.1	Test Purpose and Environment	1634

A.6.3.9.2	Test Requirements	1636

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1636

A.6.3.10.1	Test Purpose and Environment	1636

A.6.3.10.2	Test Requirements	1638

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1638

A.6.3.11.1	Test Purpose and Environment	1638

A.6.3.11.2	Test Requirements	1640

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1640

A.6.3.12.1	Test Purpose and Environment	1640

A.6.3.12.2	Test Requirements	1643

A.6.3.13	Redirection from E-UTRA to NR FR1 for redcap UE	1643

A.6.3.13.1	Test Purpose and Environment	1643

A.6.3.13.2	Test Parameters	1643

A.6.3.13.3	Test Requirements	1647

A.7	Timing and Signalling Characteristics	1648

A.7.1	UE Transmit Timing	1648

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1648

A.7.1.1.1	Test Purpose and Environment	1648

A.7.1.1.2	Test Requirements	1649

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1650

A.7.1.2.1	Test Purpose and Environment	1650

A.7.1.2.2	Test Requirements	1652

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1653

A.7.1.3.1	Test Purpose and Environment	1653

A.7.1.3.2	Test Requirements	1655

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1656

A.7.1.4.1	Test Purpose and Environment	1656

A.7.1.4.2	Test Requirements	1657

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1658

A.7.1.4A.1	Test Purpose and Environment	1658

A.7.1.4A.2	Test Requirements	1658

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1658

A.7.1.5.1	Test Purpose and Environment	1658

A.7.1.5.2	Test Requirements	1659

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1659

A.7.1.6.1	Test Purpose and Environment	1659

A.7.1.6.2	Test Requirements	1661

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1662

A.7.1.7.1	Test Purpose and Environment	1662

A.7.1.7.2	Test Requirements	1663

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1664

A.7.1.7A.1	Test Purpose and Environment	1664

A.7.1.7A.2	Test Requirements	1664

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1664

A.7.1.7B.1	Test Purpose and Environment	1664

A.7.1.7B.2	Test Requirements	1664

A.7.1.8	Void	1665

A.7.1.8.1	Void	1665

A.7.1.8.2	Void	1665

A.7.1.9	Void	1665

A.7.1.9.1	Void	1665

A.7.1.9.2	Void	1665

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1665

A.7.1.10.1	Test Purpose and Environment	1665

A.7.1.10.2	Test Requirements	1667

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1667

A.7.1.11.1	Test Purpose and Environment	1667

A.7.1.11.2	Test Requirements	1669

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1669

A.7.1.12.1	Test Purpose and Environment	1669

A.7.1.12.2	Test Requirements	1671

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1671

A.7.1.13.1	Test Purpose and Environment	1671

A.7.1.13.2	Test Requirements	1673

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1674

A.7.1.14.1	Test Purpose and Environment	1674

A.7.1.14.2	Test Requirements	1675

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1675

A.7.1.15.1	Test Purpose and Environment	1675

A.7.1.15.2	Test Requirements	1676

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1677

A.7.1.16.1	Test Purpose and Environment	1677

A.7.1.16.2	Test Requirements	1678

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1679

A.7.1.17.1	Test Purpose and Environment	1679

A.7.1.17.2	Test Requirements	1681

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1682

A.7.1.18.1	Test Purpose and Environment	1682

A.7.1.18.2	Test Requirements	1684

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1685

A.7.1.19.1	Test Purpose and Environment	1685

A.7.1.19.2	Test Requirements	1686

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1686

A.7.1.20.1	Test Purpose and Environment	1686

A.7.1.20.2	Test Requirements	1688

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1688

A.7.1.21.1	Test Purpose and Environment	1688

A.7.1.21.2	Test Requirements	1690

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1690

A.7.1.22.1	Test Purpose and Environment	1690

A.7.1.22.2	Test Requirements	1692

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1692

A.7.1.23.1	Test Purpose and Environment	1692

A.7.1.23.2	Test Requirements	1694

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1694

A.7.1.24.1	Test Purpose and Environment	1694

A.7.1.24.2	Test Requirements	1695

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1696

A.7.1.25.1	Test Purpose and Environment	1696

A.7.1.25.2	Test Requirements	1697

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1698

A.7.1.26.1	Test Purpose and Environment	1698

A.7.1.26.2	Test Requirements	1699

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1700

A.7.1.27.1	Test Purpose and Environment	1700

A.7.1.27.2	Test Requirements	1702

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1702

A.7.1.28.1	Test Purpose and Environment	1702

A.7.1.28.2	Test Requirements	1704

A.7.2	UE Timing Advance	1705

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1705

A.7.2.1.1	Test Purpose and Environment	1705

A.7.2.1.2	Test Requirements	1706

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1707

A.7.2.2.1	Test Purpose and Environment	1707

A.7.2.2.2	Test Requirements	1709

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1709

A.7.2.3.1	Test Purpose and Environment	1709

A.7.2.3.2	Test Requirements	1709

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1709

A.7.2.4.1	Test Purpose and Environment	1709

A.7.2.4.2	Test Requirements	1712

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1712

A.7.2.5.1	Test Purpose and Environment	1712

A.7.2.5.2	Test Requirements	1714

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1714

A.7.2.5A.1	Test Purpose and Environment	1714

A.7.2.5A.2	Test Requirements	1715

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1715

A.7.2.5B.1	Test Purpose and Environment	1715

A.7.2.5B.2	Test Requirements	1715

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1715

A.7.2.6.1	Test Purpose and Environment	1715

A.7.2.6.2	Test Requirements	1718

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1718

A.7.2.7.1	Test Purpose and Environment	1718

A.7.2.7.2	Test Requirements	1720

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1720

A.7.2.8.1	Test Purpose and Environment	1720

A.7.2.8.2	Test Requirements	1722

A.7.2.9.2	Test Requirements	1723

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1724

A.7.2.10.1	Test Purpose and Environment	1724

A.7.2.10.2	Test Requirements	1725

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1725

A.7.2.11.1	Test Purpose and Environment	1725

A.7.2.11.2	Test Requirements	1727

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1727

A.7.2.12.1	Test Purpose and Environment	1727

A.7.2.12.2	Test Requirements	1729

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1730

A.7.2.13.1	Test Purpose and Environment	1730

A.7.2.13.2	Test Requirements	1731

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1732

A.7.2.14.1	Test Purpose and Environment	1732

A.7.2.14.2	Test Requirements	1734

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1734

A.7.2.15.1	Test Purpose and Environment	1734

A.7.2.15.2	Test Requirements	1736

A.7.3	Radio Link Monitoring	1736

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1736

A.7.3.1.1	Test Purpose and Environment	1736

A.7.3.1.2	Test Requirements	1740

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1740

A.7.3.2.1	Test Purpose and Environment	1740

A.7.3.2.2	Test Requirements	1744

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1744

A.7.3.3.1	Test Purpose and Environment	1744

A.7.3.3.2	Test Requirements	1748

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1748

A.7.3.4.1	Test Purpose and Environment	1748

A.7.3.4.2	Test Requirements	1752

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1752

A.7.3.5.1	Test Purpose and Environment	1752

A.7.3.5.2	Test Requirements	1755

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1756

A.7.3.6.1	Test Purpose and Environment	1756

A.7.3.6.2	Test Requirements	1759

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1759

A.7.3.7.1	Test Purpose and Environment	1759

A.7.3.7.2	Test Requirements	1762

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1762

A.7.3.8.1	Test Purpose and Environment	1762

A.7.3.8.2	Test Requirements	1765

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1765

A.7.3.9.2	Test Requirements	1768

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1768

A.7.3.10.1	Test Purpose and Environment	1768

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1772

A.7.3.11.1	Test Purpose and Environment	1772

A.7.3.11.2	Test Requirements	1777

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1777

A.7.3.12.1	Test Purpose and Environment	1777

A.7.3.12.2	Test Requirements	1782

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1782

A.7.3.13.1	Test Purpose and Environment	1782

A.7.3.13.2	Test Requirements	1785

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1785

A.7.3.14.1	Test Purpose and Environment	1785

A.7.3.14.2	Test Requirements	1788

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1788

A.7.3.15.1	Test Purpose and Environment	1788

A.7.3.15.2	Test Requirements	1792

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1792

A.7.3.16.1	Test Purpose and Environment	1792

A.7.3.16.2	Test Requirements	1797

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1797

A.7.3.17.1	Test Purpose and Environment	1797

A.7.3.17.2	Test Requirements	1801

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1801

A.7.3.18.1	Test Purpose and Environment	1801

A.7.3.18.2	Test Requirements	1805

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1805

A.7.3.19.1	Test Purpose and Environment	1805

A.7.3.19.2	Test Requirements	1810

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1810

A.7.3.20.1	Test Purpose and Environment	1810

A.7.3.20.2	Test Requirements	1815

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1815

A.7.3.21.1	Test Purpose and Environment	1815

A.7.3.21.2	Test Requirements	1820

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1820

A.7.3.22.1	Test Purpose and Environment	1820

A.7.3.22.2	Test Requirements	1825

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1825

A.7.3.23.1	Test Purpose and Environment	1825

A.7.3.23.2	Test Requirements	1826

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1826

A.7.3.24.1	Test Purpose and Environment	1826

A.7.3.24.2	Test Requirements	1827

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1827

A.7.3.25.1	Test Purpose and Environment	1827

A.7.3.25.2	Test Requirements	1828

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1828

A.7.3.26.1	Test Purpose and Environment	1828

A.7.3.26.2	Test Requirements	1831

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1831

A.7.3.27.1	Test Purpose and Environment	1831

A.7.3.27.2	Test Requirements	1834

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1834

A.7.3.28.1	Test Purpose and Environment	1834

A.7.3.28.2	Test Requirements	1837

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1837

A.7.3.29.1	Test Purpose and Environment	1837

A.7.3.29.2	Test Requirements	1840

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1840

A.7.3.30.1	Test Purpose and Environment	1840

A.7.3.30.2	Test Requirements	1843

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1843

A.7.3.31.1	Test Purpose and Environment	1843

A.7.3.31.2	Test Requirements	1846

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1846

A.7.3.32.1	Test Purpose and Environment	1846

A.7.3.32.2	Test Requirements	1849

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1849

A.7.3.33.1	Test Purpose and Environment	1849

A.7.3.33.2	Test Requirements	1852

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1852

A.7.3.34.1	Test Purpose and Environment	1852

A.7.3.34.2	Test Requirements	1855

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1855

A.7.3.35.1	Test Purpose and Environment	1855

A.7.3.35.2	Test Requirements	1858

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1858

A.7.3.36.1	Test Purpose and Environment	1858

A.7.3.36.2	Test Requirements	1861

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1861

A.7.3.37.1	Test Purpose and Environment	1861

A.7.3.37.2	Test Requirements	1864

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1864

A.7.3.38.1	Test Purpose and Environment	1864

A.7.3.38.2	Test Requirements	1867

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1867

A.7.3.39.1	Test Purpose and Environment	1867

A.7.3.39.2	Test Requirements	1870

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1871

A.7.3.40.1	Test Purpose and Environment	1871

A.7.3.40.2	Test Requirements	1873

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1874

A.7.3.41.1	Test Purpose and Environment	1874

A.7.3.41.2	Test Requirements	1877

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1877

A.7.3.42.1	Test Purpose and Environment	1877

A.7.3.42.2	Test Requirements	1880

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1880

A.7.3.43.1	Test Purpose and Environment	1880

A.7.3.43.2	Test Requirements	1883

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1883

A.7.3.44.1	Test Purpose and Environment	1883

A.7.3.44.2	Test Requirements	1886

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1887

A.7.3.45.1	Test Purpose and Environment	1887

A.7.3.45.2	Test Requirements	1889

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1890

A.7.3.46.1	Test Purpose and Environment	1890

A.7.3.46.2	Test Requirements	1893

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1893

A.7.3.47.1	Test Purpose and Environment	1893

A.7.3.47.2	Test Requirements	1896

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1896

A.7.3.48.1	Test Purpose and Environment	1896

A.7.3.48.2	Test Requirements	1899

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1899

A.7.3.49.1	Test Purpose and Environment	1899

A.7.3.49.2	Test Requirements	1902

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1902

A.7.3.50.1	Test Purpose and Environment	1902

A.7.3.50.2	Test Requirements	1905

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1905

A.7.3.51.1	Test Purpose and Environment	1905

A.7.3.51.2	Test Requirements	1908

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1908

A.7.3.52.1	Test Purpose and Environment	1908

A.7.3.52.2	Test Requirements	1911

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1911

A.7.3.53.1	Test Purpose and Environment	1911

A.7.3.53.2	Test Requirements	1914

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1914

A.7.3.54.1	Test Purpose and Environment	1914

A.7.3.54.2	Test Requirements	1917

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1917

A.7.3.55.1	Test Purpose and Environment	1917

A.7.3.55.2	Test Requirements	1920

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1920

A.7.3.56.1	Test Purpose and Environment	1920

A.7.3.56.2	Test Requirements	1923

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1923

A.7.3.57.1	Test Purpose and Environment	1923

A.7.3.57.2	Test Requirements	1926

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1926

A.7.3.58.1	Test Purpose and Environment	1926

A.7.3.58.2	Test Requirements	1929

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1929

A.7.3.59.1	Test Purpose and Environment	1929

A.7.3.59.2	Test Requirements	1932

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1932

A.7.3.60.1	Test Purpose and Environment	1932

A.7.3.60.2	Test Requirements	1936

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1936

A.7.3.61.1	Test Purpose and Environment	1936

A.7.3.61.2	Test Requirements	1940

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1940

A.7.3.62.1	Test Purpose and Environment	1940

A.7.3.62.2	Test Requirements	1944

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1944

A.7.3.63.1	Test Purpose and Environment	1944

A.7.3.63.2	Test Requirements	1948

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1948

A.7.3.64.1	Test Purpose and Environment	1948

A.7.3.64.2	Test Requirements	1951

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1952

A.7.3.65.1	Test Purpose and Environment	1952

A.7.3.65.2	Test Requirements	1955

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1956

A.7.3.66.1	Test Purpose and Environment	1956

A.7.3.66.2	Test Requirements	1958

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1959

A.7.3.67.1	Test Purpose and Environment	1959

A.7.3.67.2	Test Requirements	1962

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1963

A.7.3.68.1	Test Purpose and Environment	1963

A.7.3.68.2	Test Requirements	1965

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1965

A.7.3.69.1	Test Purpose and Environment	1965

A.7.3.69.2	Test Requirements	1967

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1967

A.7.3.70.1	Test Purpose and Environment	1967

A.7.3.70.2	Test Requirements	1969

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1969

A.7.3.71.1	Test Purpose and Environment	1969

A.7.3.71.2	Test Requirements	1972

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1972

A.7.3.72.1	Test Purpose and Environment	1972

A.7.3.72.2	Test Requirements	1974

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1974

A.7.3.73.1	Test Purpose and Environment	1974

A.7.3.73.2	Test Requirements	1976

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1976

A.7.3.74.1	Test Purpose and Environment	1976

A.7.3.74.2	Test Requirements	1979

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1979

A.7.3.75.1	Test Purpose and Environment	1979

A.7.3.75.2	Test Requirements	1981

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1982

A.7.3.76.1	Test Purpose and Environment	1982

A.7.3.76.2	Test Requirements	1985

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1985

A.7.3.77.1	Test Purpose and Environment	1985

A.7.3.77.2	Test Requirements	1988

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1988

A.7.3.78.1	Test Purpose and Environment	1988

A.7.3.78.2	Test Requirements	1991

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1991

A.7.3.79.1	Test Purpose and Environment	1991

A.7.3.79.2	Test Requirements	1994

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1994

A.7.3.80.1	Test Purpose and Environment	1994

A.7.3.80.2	Test Requirements	1997

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1997

A.7.3.81.1	Test Purpose and Environment	1997

A.7.3.81.2	Test Requirements	2000

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	2000

A.7.3.82.1	Test Purpose and Environment	2000

A.7.3.82.2	Test Requirements	2003

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	2003

A.7.3.83.1	Test Purpose and Environment	2003

A.7.3.83.2	Test Requirements	2004

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	2005

A.7.3.84.1	Test Purpose and Environment	2005

A.7.3.84.2	Test Requirements	2007

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	2007

A.7.3.85.1	Test Purpose and Environment	2007

A.7.3.85.2	Test Requirements	2008

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	2009

A.7.3.86.1	Test Purpose and Environment	2009

A.7.3.86.2	Test Requirements	2011

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	2011

A.7.3.87.1	Test Purpose and Environment	2011

A.7.3.87.2	Test Requirements	2013

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	2014

A.7.3.88.1	Test Purpose and Environment	2014

A.7.3.88.2	Test Requirements	2018

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	2018

A.7.3.89.1	Test Purpose and Environment	2018

A.7.3.89.2	Test Requirements	2022

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	2022

A.7.3.90.1	Test Purpose and Environment	2022

A.7.3.90.2	Test Requirements	2026

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	2026

A.7.3.91.1	Test Purpose and Environment	2026

A.7.3.91.2	Test Requirements	2030

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	2030

A.7.3.92.1	Test Purpose and Environment	2030

A.7.3.92.2	Test Requirements	2033

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	2034

A.7.3.93.1	Test Purpose and Environment	2034

A.7.3.93.2	Test Requirements	2037

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	2038

A.7.3.94.1	Test Purpose and Environment	2038

A.7.3.94.2	Test Requirements	2040

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	2041

A.7.3.95.1	Test Purpose and Environment	2041

A.7.3.95.2	Test Requirements	2044

A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	2045

A.7.3.96.1	Test Purpose and Environment	2045

A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	2046

A.7.3.97.1	Test Purpose and Environment	2046

A.7.3.97.2	Test Requirements	2049

A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	2049

A.7.3.98.1	Test Purpose and Environment	2049

A.7.3.98.2	Test Requirements	2052

A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	2052

A.7.3.99.1	Test Purpose and Environment	2052

A.7.3.99.2	Test Requirements	2055

A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	2055

A.7.3.100.1	Test Purpose and Environment	2055

A.7.3.100.2	Test Requirements	2057

A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	2057

A.7.3.101.1	Test Purpose and Environment	2057

A.7.3.101.2	Test Requirements	2059

A.7.4	Interruption for Dual Connectivity	2059

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2059

A.7.4.1.1	Test Purpose and Environment	2059

A.7.4.1.2	Test Requirements	2062

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2062

A.7.4.2.1	Test Purpose and Environment	2062

A.7.4.2.2	Test Requirements	2064

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	2064

A.7.4.3.1	Test Purpose and Environment	2064

A.7.4.3.2	Test Requirements	2066

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2066

A.7.4.4.1	Test Purpose and Environment	2066

A.7.4.4.2	Test Requirements	2069

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2069

A.7.4.5.1	Test Purpose and Environment	2069

A.7.4.5.2	Test Requirements	2071

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	2071

A.7.4.6.1	Test Purpose and Environment	2071

A.7.4.6.2	Test Requirements	2073

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	2073

A.7.4.7.1	Test Purpose and Environment	2073

A.7.4.7.2	Test Requirements	2077

A.7.5	Proximity-based Services	2077

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	2077

A.7.5.1.1	Test Purpose and Environment	2077

A.7.5.1.2	Test Requirements	2078

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	2079

A.7.5.2.1	Test Purpose and Environment	2079

A.7.5.1.2	Test Requirements	2079

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	2080

A.7.5.3.1	Test Purpose and Environment	2080

A.7.5.3.2	Test Requirements	2081

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2082

A.7.5.4.1	Test Purpose and Environment	2082

A.7.5.4.2	Test Requirements	2083

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	2084

A.7.5.5.1	Test Purpose and Environment	2084

A.7.5.5.2	Test Requirements	2086

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	2087

A.7.5.6.1	Test Purpose and Environment	2087

A.7.5.6.2	Test Requirements	2088

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	2089

A.7.5.7.1	Test Purpose and Environment	2089

A.7.5.7.2	Test Requirements	2091

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	2091

A.7.5.8.1	Test Purpose and Environment	2091

A.7.5.8.2	Test Requirements	2093

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	2094

A.7.5.9.1	Test Purpose and Environment	2094

A.7.5.9.2	Test Requirements	2096

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	2097

A.7.5.10.1	Test Purpose and Environment	2097

A.7.5.10.2	Test Requirements	2099

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	2100

A.7.5.11.1	Test Purpose and Environment	2100

A.7.5.11.2	Test Requirements	2102

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	2102

A.7.5.12.1	Test Purpose and Environment	2102

A.7.5.12.2	Test Requirements	2106

A.7.6	Interruption for carrier aggregation	2107

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	2107

A.7.6.1.1	Test Purpose and Environment	2107

A.7.6.1.2	Test Requirements	2110

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	2110

A.7.6.2.1	Test Purpose and Environment	2110

A.7.6.2.2	Test Requirements	2113

A.8	UE Measurements Procedures	2114

A.8.1	E-UTRAN FDD Intra-frequency Measurements	2114

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2114

A.8.1.1.1	Test Purpose and Environment	2114

A.8.1.1.2	Test Requirements	2115

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	2116

A.8.1.2.1	Test Purpose and Environment	2116

A.8.1.2.2	Test Requirements	2117

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2118

A.8.1.3.1	Test Purpose and Environment	2118

A.8.1.3.2	Test Requirements	2120

A.8.1.4	Void	2120

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2120

A.8.1.5.1	Test Purpose and Environment	2120

A.8.1.5.2	Test Requirements	2122

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2122

A.8.1.6.1	Test Purpose and Environment	2122

A.8.1.6.2	Test Requirements	2124

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2124

A.8.1.7.1	Test Purpose and Environment	2124

A.8.1.7.2	Test Requirements	2126

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2127

A.8.1.8.1	Test Purpose and Environment	2127

A.8.1.8.2	Test Requirements	2129

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	2130

A.8.1.9.1	Test Purpose and Environment	2130

A.8.1.9.2	Test Requirements	2130

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	2130

A.8.1.10.1	Test Purpose and Environment	2130

A.8.1.10.2	Test Requirements	2131

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	2131

A.8.1.11.1	Test Purpose and Environment	2131

A.8.1.11.2	Test Requirements	2133

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2134

A.8.1.12.1	Test Purpose and Environment	2134

A.8.1.12.2	Test Requirements	2135

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2136

A.8.1.13.1	Test Purpose and Environment	2136

A.8.1.13.2	Test Requirements	2138

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	2138

A.8.1.14.1	Test Purpose and Environment	2138

A.8.1.14.2	Test Requirements	2140

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2141

A.8.1.15.1	Test Purpose and Environment	2141

A.8.1.15.2	Test Requirements	2142

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2143

A.8.1.16.1	Test Purpose and Environment	2143

A.8.1.16.2	Test Requirements	2145

A.8.1.17	Void	2145

A.8.1.18	Void	2145

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	2145

A.8.1.19.1	Test Purpose and Environment	2145

A.8.1.19.2	Test Requirements	2147

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	2148

A.8.1.20.1	Test Purpose and Environment	2148

A.8.1.20.2	Test Requirements	2150

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	2150

A.8.1.21.1	Test Purpose and Environment	2150

A.8.1.21.2	Test Requirements	2152

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	2153

A.8.1.22.1	Test Purpose and Environment	2153

A.8.1.22.2	Test Requirements	2155

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	2155

A.8.1.23.1	Test Purpose and Environment	2155

A.8.1.23.2	Test Requirements	2156

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2157

A.8.1.24.1	Test Purpose and Environment	2157

A.8.1.24.2	Test Requirements	2159

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2159

A.8.1.25.1	Test Purpose and Environment	2159

A.8.1.25.2	Test Requirements	2161

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	2161

A.8.1.26.1	Test Purpose and Environment	2161

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2164

A.8.1.27.1	Test Purpose and Environment	2164

A.8.1.27.2	Test Requirements	2166

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2166

A.8.1.28.1	Test Purpose and Environment	2166

A.8.1.28.2	Test Requirements	2168

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2168

A.8.1.29.1	Test Purpose and Environment	2168

A.8.1.29.2	Test Requirements	2170

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2171

A.8.1.30.1	Test Purpose and Environment	2171

A.8.1.30.2	Test Requirements	2173

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	2173

A.8.1.31.1	Test Purpose and Environment	2173

A.8.1.31.2	Test Requirements	2175

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2176

A.8.1.32.1	Test Purpose and Environment	2176

A.8.1.32.2	Test Requirements	2177

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	2178

A.8.1.33.1	Test Purpose and Environment	2178

A.8.1.33.2	Test Requirements	2179

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2180

A.8.1.34.1	Test Purpose and Environment	2180

A.8.1.34.2	Test Requirements	2181

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2182

A.8.1.35.1	Test Purpose and Environment	2182

A.8.1.35.2	Test Requirements	2183

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2184

A.8.1.36.1	Test Purpose and Environment	2184

A.8.1.36.2	Test Requirements	2185

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2186

A.8.1.37.1	Test Purpose and Environment	2186

A.8.1.37.2	Test Requirements	2188

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2188

A.8.1.38.1	Test Purpose and Environment	2188

A.8.1.38.2	Test Requirements	2189

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2190

A.8.1.39.1	Test Purpose and Environment	2190

A.8.1.39.2	Test Requirements	2192

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2192

A.8.1.40.1	Test Purpose and Environment	2192

A.8.1.40.2	Test Requirements	2194

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2194

A.8.1.41.1	Test Purpose and Environment	2194

A.8.1.41.2	Test Requirement	2196

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2196

A.8.1.42.1	Test Purpose and Environment	2196

A.8.1.42.2	Test Requirement	2197

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2197

A.8.1.43.1	Test Purpose and Environment	2197

A.8.1.43.2	Test Requirements	2200

A.8.1.44	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2200

A.8.1.44.1	Test Purpose and Environment	2200

A.8.1.44.2	Test Requirements	2203

A.8.1.45	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2204

A.8.1.45.1	Test Purpose and Environment	2204

A.8.1.45.2	Test Requirements	2206

A.8.1.46	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2207

A.8.1.46.1	Test Purpose and Environment	2207

A.8.1.46.2	Test Requirements	2208

A.8.1.47	TDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2209

A.8.1.47.1	Test Purpose and Environment	2209

A.8.1.47.2	Test Requirements	2211

A.8.1.48	TDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2212

A.8.1.48.1	Test Purpose and Environment	2212

A.8.1.48.2	Test Requirements	2214

A.8.1.49	TDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2215

A.8.1.49.1	Test Purpose and Environment	2215

A.8.1.49.2	Test Requirements	2216

A.8.150	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2217

A.8.1.50.1	Test Purpose and Environment	2217

A.8.1.50.2	Test Requirements	2219

A.8.1.51	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2220

A.8.1.51.1	Test Purpose and Environment	2220

A.8.1.51.2	Test Requirements	2222

A.8.1.52	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2223

A.8.1.52.1	Test Purpose and Environment	2223

A.8.1.52.2	Test Requirements	2224

A.8.1.53	TDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2225

A.8.1.53.1	Test Purpose and Environment	2225

A.8.1.53.2	Test Requirements	2227

A.8.1.54	TDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2228

A.8.1.54.1	Test Purpose and Environment	2228

A.8.1.54.2	Test Requirements	2230

A.8.1.55	TDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2231

A.8.1.55.1	Test Purpose and Environment	2231

A.8.1.55.2	Test Requirements	2232

A.8.2	E-UTRAN TDD Intra-frequency Measurements	2233

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	2233

A.8.2.1.1	Test Purpose and Environment	2233

A.8.2.1.2	Test Requirements	2234

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2234

A.8.2.2.1	Test Purpose and Environment	2234

A.8.2.2.2	Test Requirements	2237

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2237

A.8.2.3.1	Test Purpose and Environment	2237

A.8.2.3.2	Test Requirements	2239

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2240

A.8.2.4.1	Test Purpose and Environment	2240

A.8.2.4.2	Test Requirements	2242

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2242

A.8.2.5.1	Test Purpose and Environment	2242

A.8.2.5.2	Test Requirements	2244

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2245

A.8.2.6.1	Test Purpose and Environment	2245

A.8.2.6.2	Test Requirements	2248

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2249

A.8.2.7.1	Test Purpose and Environment	2249

A.8.2.7.2	Test Requirements	2250

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2251

A.8.2.8.1	Test Purpose and Environment	2251

A.8.2.8.2	Test Requirements	2253

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2253

A.8.2.9.1	Test Purpose and Environment	2253

A.8.2.9.2	Test Requirements	2255

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2256

A.8.2.10.1	Test Purpose and Environment	2256

A.8.2.10.2	Test Requirements	2258

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2258

A.8.2.11.1	Test Purpose and Environment	2258

A.8.2.11.2	Test Requirements	2260

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2260

A.8.2.12.1	Test Purpose and Environment	2260

A.8.2.12.2	Test Requirements	2262

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2263

A.8.2.13.1	Test Purpose and Environment	2263

A.8.2.13.2	Test Requirements	2265

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2265

A.8.2.14.1	Test Purpose and Environment	2265

A.8.2.14.2	Test Requirement	2267

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2267

A.8.2.15.1	Test Purpose and Environment	2267

A.8.2.15.2	Test Requirements	2269

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	2269

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2269

A.8.3.1.1	Test Purpose and Environment	2269

A.8.3.1.2	Test Requirements	2271

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2271

A.8.3.2.1	Test Purpose and Environment	2271

A.8.3.2.2	Test Requirements	2274

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	2274

A.8.3.3.1	Test Purpose and Environment	2274

A.8.3.3.2	Test Requirements	2277

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2277

A.8.3.4.1	Test Purpose and Environment	2277

A.8.3.4.2	Test Requirements	2279

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2279

A.8.3.5.2	Test Requirements	2281

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	2281

A.8.3.6.1	Test Purpose and Environment	2281

A.8.3.6.2	Test Requirements	2283

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2284

A.8.3.7.1	Test Purpose and Environment	2284

A.8.3.7.2	Test Requirements	2286

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2287

A.8.3.8.1	Test Purpose and Environment	2287

A.8.3.8.2	Test Requirements	2290

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2290

A.8.3.9.1	Test Purpose and Environment	2290

A.8.3.9.2	Test Requirements	2294

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2295

A.8.3.10.1	Test Purpose and Environment	2295

A.8.3.10.2	Test Requirements	2296

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	2296

A.8.3.11.1	Test Purpose and Environment	2296

A.8.3.11.2	Test Requirement	2298

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2299

A.8.3.12.1	Test Purpose and Environment	2299

A.8.3.12.2	Test Requirement	2300

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2301

A.8.3.13.1	Test Purpose and Environment	2301

A.8.3.13.2	Test Requirement	2302

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2303

A.8.3.14.1	Test Purpose and Environment	2303

A.8.3.14.2	Test Requirement	2304

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2305

A.8.3.15.1	Test Purpose and Environment	2305

A.8.3.15.2	Test Requirement	2306

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	2307

A.8.3.16.1	Test Purpose and Environment	2307

A.8.3.16.2	Test Requirement	2309

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2309

A.8.3.17.1	Test Purpose and Environment	2309

A.8.3.17.2	Test Requirement	2312

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2312

A.8.3.18.1	Test Purpose and Environment	2312

A.8.3.18.2	Test Requirement	2315

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2315

A.8.3.19.1	Test Purpose and Environment	2315

A.8.3.19.2	Test Requirement	2318

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	2318

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2318

A.8.4.1.1	Test Purpose and Environment	2318

A.8.4.1.2	Test Requirements	2320

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	2320

A.8.4.2.1	Test Purpose and Environment	2320

A.8.4.2.2	Test Requirements	2323

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	2323

A.8.4.3.1	Test Purpose and Environment	2323

A.8.4.3.2	Test Requirements	2325

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2326

A.8.4.4.1	Test Purpose and Environment	2326

A.8.4.4.2	Test Requirements	2328

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2329

A.8.4.5.1	Test Purpose and Environment	2329

A.8.4.5.2	Test Requirements	2331

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	2331

A.8.4.6.1	Test Purpose and Environment	2331

A.8.4.6.2	Test Requirements	2332

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2332

A.8.4.7.1	Test Purpose and Environment	2332

A.8.4.7.2	Test Requirements	2336

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2336

A.8.4.8.1	Test Purpose and Environment	2336

A.8.4.8.2	Test Requirements	2339

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2339

A.8.4.9.1	Test Purpose and Environment	2339

A.8.4.9.2	Test Requirements	2344

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2344

A.8.4.10.1	Test Purpose and Environment	2344

A.8.4.10.2	Test Requirements	2346

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	2346

A.8.4.11.1	Test Purpose and Environment	2346

A.8.4.11.2	Test Requirement	2348

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2348

A.8.4.12.1	Test Purpose and Environment	2348

A.8.4.12.2	Test Requirement	2350

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2351

A.8.4.13.1	Test Purpose and Environment	2351

A.8.4.13.2	Test Requirement	2352

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2353

A.8.4.14.1	Test Purpose and Environment	2353

A.8.4.14.2	Test Requirement	2355

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2355

A.8.4.15.1	Test Purpose and Environment	2355

A.8.4.15.2	Test Requirement	2358

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	2358

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2358

A.8.5.1.1	Test Purpose and Environment	2358

A.8.5.1.2	Test Requirements	2360

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	2360

A.8.5.2.1	Test Purpose and Environment	2360

A.8.5.2.2	Test Requirements	2362

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	2362

A.8.5.3.1	Test Purpose and Environment	2362

A.8.5.3.2	Test Requirements	2365

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	2365

A.8.5.4.1	Test Purpose and Environment	2365

A.8.5.4.2	Test Requirements	2367

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2367

A.8.5.5.1	Test Purpose and Environment	2367

A.8.5.5.2	Test Requirements	2370

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2370

A.8.5.6.1	Test Purpose and Environment	2370

A.8.5.6.2	Test Requirements	2371

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2372

A.8.5.7.1	Test Purpose and Environment	2372

A.8.5.7.2	Test Requirements	2372

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2372

A.8.5.8.1	Test Purpose and Environment	2372

A.8.5.8.2	Test Requirements	2375

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2375

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2375

A.8.6.1.1	Test Purpose and Environment	2375

A.8.6.1.2	Test Requirements	2377

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2377

A.8.6.2.1	Test Purpose and Environment	2377

A.8.6.2.2	Test Requirements	2380

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2380

A.8.6.3.1	Test Purpose and Environment	2380

A.8.6.3.2	Test Requirements	2383

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2383

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2383

A.8.7.1.1	Test Purpose and Environment	2383

A.8.7.1.1.1	Void	2383

A.8.7.1.1.2	1.28 Mcps TDD option	2383

A.8.7.1.1.3	Void	2385

A.8.7.1.2	Test Requirements	2385

A.8.7.1.2.1	Void	2385

A.8.7.1.2.2	1.28 Mcps TDD option	2385

A.8.7.1.2.3	Void	2385

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2385

A.8.7.2.1	Test Purpose and Environment	2385

A.8.7.2.2	Test Requirements	2388

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2389

A.8.7.3.1	Test Purpose and Environment	2389

A.8.7.3.2	Test Parameters	2389

A.8.7.3.3	Test Requirements	2390

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2391

A.8.7.4.1	Test Purpose and Environment	2391

A.8.7.4.2	Test Requirements	2393

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2393

A.8.7.5.1	Test Purpose and Environment	2393

A.8.7.5.2	Test Requirements	2395

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2395

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2395

A.8.7A.1.1	Test Purpose and Environment	2395

A.8.7A.1.2	Test Requirements	2398

A.8.8	E-UTRAN FDD – GSM Measurements	2398

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2398

A.8.8.1.1	Test Purpose and Environment	2398

A.8.8.1.2	Test Requirements	2400

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2400

A.8.8.2.1	Test Purpose and Environment	2400

A.8.8.2.2	Test Requirements	2402

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2403

A.8.8.3.1	Test Purpose and Environment	2403

A.8.8.3.2	Test Requirements	2404

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2405

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2405

A.8.9.1.1	Test Purpose and Environment	2405

A.8.9.1.2	Test Requirements	2406

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2407

A.8.9.2.1	Test Purpose and Environment	2407

A.8.9.2.2	Test Requirements	2409

A.8.10	E-UTRAN TDD – GSM Measurements	2409

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2409

A.8.10.1.1	Test Purpose and Environment	2409

A.8.10.1.2	Test Requirements	2410

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2411

A.8.10.2.1	Test Purpose and Environment	2411

A.8.10.2.2	Test Requirements	2413

A.8.11	Monitoring of Multiple Layers	2413

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2413

A.8.11.1.1	Test Purpose and Environment	2413

A.8.11.1.2	Test Requirements	2415

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2416

A.8.11.2.1	Test Purpose and Environment	2416

A.8.11.2.2	Test Requirements	2417

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2418

A.8.11.3.1	Test Purpose and Environment	2418

A.8.11.3.2	Test Requirements	2420

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2420

A.8.11.4.1	Test Purpose and Environment	2420

A.8.11.4.2	Test Requirements	2423

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2423

A.8.11.5.1	Test Purpose and Environment	2423

A.8.11.5.2	Test Requirements	2425

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2426

A.8.11.6.1	Test Purpose and Environment	2426

A.8.11.6.2	Test Requirements	2428

A.8.12	RSTD Intra-frequency Measurements	2429

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2429

A.8.12.1.1	Test Purpose and Environment	2429

A.8.12.1.2	Test Requirements	2433

A.8.12.1.2A	Test Requirements for UE Category 1bis	2433

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2433

A.8.12.2.1	Test Purpose and Environment	2433

A.8.12.2.2	Test Requirements	2438

A.8.12.2.2A	Test Requirements for UE Category 1bis	2438

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2438

A.8.12.3.1	Test Purpose and Environment	2438

A.8.12.3.2	Test Requirements	2444

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2444

A.8.12.4.1	Test Purpose and Environment	2444

A.8.12.4.2	Test Requirements	2449

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2449

A.8.12.5.1	Test Purpose and Environment	2449

A.8.12.5.2	Test Requirements	2454

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2454

A.8.12.6.1	Test Purpose and Environment	2454

A.8.12.6.2	Test Requirements	2459

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2459

A.8.12.7.1	Test Purpose and Environment	2459

A.8.12.7.2	Test Requirements	2464

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2464

A.8.12.8.1	Test Purpose and Environment	2464

A.8.12.8.2	Test Requirements	2469

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2469

A.8.12.9.1	Test Purpose and Environment	2469

A.8.12.9.2	Test Requirements	2474

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2474

A.8.12.10.1	Test Purpose and Environment	2474

A.8.12.10.2	Test Requirements	2479

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2479

A.8.12.11.1	Test Purpose and Environment	2479

A.8.12.11.2	Test Requirements	2484

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2484

A.8.12.12.1	Test Purpose and Environment	2484

A.8.12.12.2	Test Requirements	2489

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2489

A.8.12.13.1	Test Purpose and Environment	2489

A.8.12.13.2	Test Requirements	2494

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2494

A.8.12.14.1	Test Purpose and Environment	2494

A.8.12.14.2	Test Requirements	2499

A.8.13	RSTD Inter-frequency Measurements	2499

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2499

A.8.13.1.1	Test Purpose and Environment	2499

A.8.13.1.2	Test Requirements	2504

A.8.13.1.2A	Test Requirements for UE Category 1bis	2504

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2504

A.8.13.2.1	Test Purpose and Environment	2504

A.8.13.2.2	Test Requirements	2510

A.8.13.2.2A	Test Requirements for UE Category 1bis	2510

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2510

A.8.13.3.1	Test Purpose and Environment	2510

A.8.13.3.2	Test Requirements	2517

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2517

A.8.13.4.1	Test Purpose and Environment	2517

A.8.13.4.2	Test Requirements	2523

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2523

A.8.13.5.1	Test Purpose and Environment	2523

A.8.13.5.2	Test Requirements	2529

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2529

A.8.13.6.1	Test Purpose and Environment	2529

A.8.13.6.2	Test Requirements	2535

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2535

A.8.13.7.1	Test Purpose and Environment	2535

A.8.13.7.2	Test Requirements	2541

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2541

A.8.13.8.1	Test Purpose and Environment	2541

A.8.13.8.2	Test Requirements	2547

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2547

A.8.13.9.1	Test Purpose and Environment	2547

A.8.13.9.2	Test Requirements	2553

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2553

A.8.13.10.1	Test Purpose and Environment	2553

A.8.13.10.2	Test Requirements	2559

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2559

A.8.13.11.1	Test Purpose and Environment	2559

A.8.13.11.2	Test Requirements	2565

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2565

A.8.13.12.1	Test Purpose and Environment	2565

A.8.13.12.2	Test Requirements	2571

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2571

A.8.13.13.1	Test Purpose and Environment	2571

A.8.13.13.2	Test Requirements	2577

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2577

A.8.13.14.1	Test Purpose and Environment	2577

A.8.13.14.2	Test Requirements	2583

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2583

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2583

A.8.14.1.1	Test Purpose and Environment	2583

A.8.14.1.2	Test Requirements	2585

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2585

A.8.14.2.1	Test Purpose and Environment	2585

A.8.14.2.2	Test Requirements	2588

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2588

A.8.14.3.1	Test Purpose and Environment	2588

A.8.14.3.2	Test Requirements	2590

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2591

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2591

A.8.15.1.1	Test Purpose and Environment	2591

A.8.15.1.2	Test Requirements	2592

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2592

A.8.15.2.1	Test Purpose and Environment	2592

A.8.15.2.2	Test Requirements	2595

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2595

A.8.15.3.1	Test Purpose and Environment	2595

A.8.15.3.2	Test Requirements	2597

A.8.16	E-UTRAN Carrier Aggregation Measurements	2598

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2598

A.8.16.1.1	Test Purpose and Environment	2598

A.8.16.1.2	Test Requirements	2600

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2600

A.8.16.2.1	Test Purpose and Environment	2600

A.8.16.2.2	Test Requirements	2602

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2602

A.8.16.3.1	Test Purpose and Environment	2602

A.8.16.3.2	Test Requirements	2604

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2605

A.8.16.3A.1	Test Purpose and Environment	2605

A.8.16.3A.2	Test Requirements	2606

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2607

A.8.16.4.1	Test Purpose and Environment	2607

A.8.16.4.2	Test Requirements	2608

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2609

A.8.16.4A.1	Test Purpose and Environment	2609

A.8.16.4A.2	Test Requirements	2611

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2612

A.8.16.5.1	Test Purpose and Environment	2612

A.8.16.5.2	Test Requirements	2612

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2612

A.8.16.6.1	Test Purpose and Environment	2612

A.8.16.6.2	Test Requirements	2613

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2613

A.8.16.7.1	 Test Purpose and Environment	2613

A.8.16.7.2	 Test Requirements	2614

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2614

A.8.16.8.1	 Test Purpose and Environment	2614

A.8.16.8.2	 Test Requirements	2615

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2615

A.8.16.9.1	Test Purpose and Environment	2615

A.8.16.9.2	Test Requirements	2616

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2616

A.8.16.10.1	Test Purpose and Environment	2616

A.8.16.10.2	Test Requirements	2617

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2617

A.8.16.11.1	 Test Purpose and Environment	2617

A.8.16.11.2	 Test Requirements	2618

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2618

A.8.16.12.1	 Test Purpose and Environment	2618

A.8.16.12.2	 Test Requirements	2618

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2618

A.8.16.13.1	Test Purpose and Environment	2618

A.8.16.13.2	Test Requirements	2619

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2619

A.8.16.14.1	Test Purpose and Environment	2619

A.8.16.14.2	Test Requirements	2620

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2620

A.8.16.15.1	 Test Purpose and Environment	2620

A.8.16.7.2	 Test Requirements	2620

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2620

A.8.16.16.1	 Test Purpose and Environment	2620

A.8.16.16.2	 Test Requirements	2621

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2621

A.8.16.17.1	Test Purpose and Environment	2621

A.8.16.17.2	Test Requirements	2623

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2624

A.8.16.17A.1	Test Purpose and Environment	2624

A.8.16.17A.2	Test Requirements	2624

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2624

A.8.16.17B.1	Test Purpose and Environment	2624

A.8.16.17B.2	Test Requirements	2625

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2625

A.8.16.17C.1	Test Purpose and Environment	2625

A.8.16.17C.2	Test Requirements	2625

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2625

A.8.16.18.1	Test Purpose and Environment	2625

A.8.16.18.2	Test Requirements	2627

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2628

A.8.16.18A.1	Test Purpose and Environment	2628

A.8.16.18A.2	Test Requirements	2628

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2628

A.8.16.18B.1	Test Purpose and Environment	2628

A.8.16.18B.2	Test Requirements	2629

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2629

A.8.16.18C.1	Test Purpose and Environment	2629

A.8.16.18C.2	Test Requirements	2629

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2629

A.8.16.18D.1	Test Purpose and Environment	2629

A.8.16.18D.2	Test Requirements	2630

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2630

A.8.16.19.1	Test Purpose and Environment	2630

A.8.16.19.2	Test Requirements	2632

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2632

A.8.16.19A.1	Test Purpose and Environment	2632

A.8.16.19A.2	Test Requirements	2633

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2633

A.8.16.19B.1	Test Purpose and Environment	2633

A.8.16.19B.2	Test Requirements	2633

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2633

A.8.16.19C.1	Test Purpose and Environment	2633

A.8.16.19C.2	Test Requirements	2634

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2634

A.8.16.20.1	Test Purpose and Environment	2634

A.8.16.20.2	Test Requirements	2636

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2637

A.8.16.20A.1	Test Purpose and Environment	2637

A.8.16.20A.2	Test Requirements	2637

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2637

A.8.16.20B.1	Test Purpose and Environment	2637

A.8.16.20B.2	Test Requirements	2638

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2638

A.8.16.20C.1	Test Purpose and Environment	2638

A.8.16.20C.2	Test Requirements	2638

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2638

A.8.16.20D.1	Test Purpose and Environment	2638

A.8.16.20D.2	Test Requirements	2639

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2639

A.8.16.21.1	Test Purpose and Environment	2639

A.8.16.21.2	Test Requirements	2641

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2641

A.8.16.22.1	Test Purpose and Environment	2641

A.8.16.22.2	Test Requirements	2643

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2643

A.8.16.23.1	Test Purpose and Environment	2643

A.8.16.23.2	Test Requirements	2646

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2646

A.8.16.24.1	Test Purpose and Environment	2646

A.8.16.24.2	Test Requirements	2649

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2649

A.8.16.25.1	Test Purpose and Environment	2649

A.8.16.25.2	Test Requirements	2652

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2652

A.8.16.26.1	Test Purpose and Environment	2652

A.8.16.26.2	Test Requirements	2655

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2655

A.8.16.27.1	Test Purpose and Environment	2655

A.8.16.27.2	Test Requirements	2660

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2660

A.8.16.28.1	Test Purpose and Environment	2660

A.8.16.28.2	Test Requirements	2665

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2665

A.8.16.29.1	Test Purpose and Environment	2665

A.8.16.29.2	Test Requirements	2670

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2670

A.8.16.30.1	Test Purpose and Environment	2670

A.8.16.30.2	Test Requirements	2675

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2675

A.8.16.31.1	Test Purpose and Environment	2675

A.8.16.31.2	Test Requirements	2680

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2680

A.8.16.32.1	Test Purpose and Environment	2680

A.8.16.32.2	Test Requirements	2685

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2685

A.8.16.33.1	Test Purpose and Environment	2685

A.8.16.33.2	Test Requirements	2690

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2690

A.8.16.34.1	Test Purpose and Environment	2690

A.8.16.34.2	Test Requirements	2695

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2695

A.8.16.35.1	Test Purpose and Environment	2695

A.8.16.35.2	Test Requirements	2697

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2698

A.8.16.36.1	Test Purpose and Environment	2698

A.8.16.36.2	Test Requirements	2700

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2701

A.8.16.37.1	Test Purpose and Environment	2701

A.8.16.37.2	Test Requirements	2703

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2704

A.8.16.38.1	Test Purpose and Environment	2704

A.8.16.38.2	Test Requirements	2706

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2707

A.8.16.39.1	Test Purpose and Environment	2707

A.8.16.39.2	Test Requirements	2710

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2710

A.8.16.40.1	Test Purpose and Environment	2710

A.8.16.40.2	Test Requirements	2713

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2713

A.8.16.41.1	Test Purpose and Environment	2713

A.8.16.41.2	Test Requirements	2715

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2716

A.8.16.42.1	Test Purpose and Environment	2716

A.8.16.42.2	Test Requirements	2718

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2719

A.8.16.43.1	Test Purpose and Environment	2719

A.8.16.43.2	Test Requirements	2722

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2722

A.8.16.44.1	Test Purpose and Environment	2722

A.8.16.44.2	Test Requirements	2725

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2725

A.8.16.45.1	Test Purpose and Environment	2725

A.8.16.45.2	Test Requirements	2728

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2728

A.8.16.46.1	Test Purpose and Environment	2728

A.8.16.46.2	Test Requirements	2730

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2731

A.8.16.47.1	Test Purpose and Environment	2731

A.8.16.47.2	Test Requirements	2733

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2734

A.8.16.48.1	Test Purpose and Environment	2734

A.8.16.48.2	Test Requirements	2736

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2737

A.8.16.49.1	Test Purpose and Environment	2737

A.8.16.49.2	Test Requirements	2740

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2740

A.8.16.50.1	Test Purpose and Environment	2740

A.8.16.50.2	Test Requirements	2742

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2743

A.8.16.51.1	Test Purpose and Environment	2743

A.8.16.51.2	Test Requirements	2748

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2748

A.8.16.52.1	Test Purpose and Environment	2748

A.8.16.52.2	Test Requirements	2753

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2753

A.8.16.53.1	Test Purpose and Environment	2753

A.8.16.53.2	Test Requirements	2757

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2757

A.8.16.54.1	Test Purpose and Environment	2757

A.8.16.54.2	Test Requirements	2761

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2761

A.8.16.55.1	Test Purpose and Environment	2761

A.8.16.55.2	Test Requirements	2767

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2767

A.8.16.56.1	Test Purpose and Environment	2767

A.8.16.56.2	Test Requirements	2773

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2773

A.8.16.57.1	Test Purpose and Environment	2773

A.8.16.57.2	Test Requirements	2775

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2776

A.8.16.58.1	Test Purpose and Environment	2776

A.8.16.58.2	Test Requirements	2778

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2779

A.8.16.59.1	Test Purpose and Environment	2779

A.8.16.59.2	Test Requirements	2783

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2784

A.8.16.60.1	Test Purpose and Environment	2784

A.8.16.60.2	Test Requirements	2787

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2788

A.8.16.61.1	Test Purpose and Environment	2788

A.8.16.61.2	Test Requirements	2791

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2791

A.8.16.62.1	Test Purpose and Environment	2791

A.8.16.62.2	Test Requirements	2793

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2794

A.8.16.63.1	Test Purpose and Environment	2794

A.8.16.63.2	Test Requirements	2797

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2798

A.8.16.64.1	Test Purpose and Environment	2798

A.8.16.64.2	Test Requirements	2802

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2802

A.8.16.65.1	Test Purpose and Environment	2802

A.8.16.65.2	Test Requirements	2807

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2807

A.8.16.66.1	Test Purpose and Environment	2807

A.8.16.66.2	Test Requirements	2812

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2812

A.8.16.67.1	Test Purpose and Environment	2812

A.8.16.67.2	Test Requirements	2817

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2817

A.8.16.68.1	Test Purpose and Environment	2817

A.8.16.68.2	Test Requirements	2822

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2822

A.8.16.69.1	Test Purpose and Environment	2822

A.8.16.69.2	Test Requirements	2826

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2826

A.8.16.70.1	Test Purpose and Environment	2826

A.8.16.70.2	Test Requirements	2830

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2830

A.8.16.71.1	Test Purpose and Environment	2830

A.8.16.71.2	Test Requirements	2835

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2835

A.8.16.72.1	Test Purpose and Environment	2835

A.8.16.72.2	Test Requirements	2840

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2840

A.8.16.73.1	Test Purpose and Environment	2840

A.8.16.73.2	Test Requirements	2847

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2847

A.8.16.74.1	Test Purpose and Environment	2847

A.8.16.74.2	Test Requirements	2853

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2853

A.8.16.75.1	Test Purpose and Environment	2853

A.8.16.75.2	Test Requirements	2859

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2859

A.8.16.76.1	Test Purpose and Environment	2859

A.8.16.76.2	Test Requirements	2865

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2865

A.8.16.77.1	Test Purpose and Environment	2865

A.8.16.77.2	Test Requirements	2869

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2869

A.8.16.78.1	Test Purpose and Environment	2869

A.8.16.78.2	Test Requirements	2871

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2872

A.8.16.79.1	Test Purpose and Environment	2872

A.8.16.79.2	Test Requirements	2879

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2879

A.8.16.80.1	Test Purpose and Environment	2879

A.8.16.80.2	Test Requirements	2886

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2886

A.8.16.81.1	Test Purpose and Environment	2886

A.8.16.81.2	Test Requirements	2892

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2892

A.8.16.82.1	Test Purpose and Environment	2892

A.8.16.82.2	Test Requirements	2898

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2898

A.8.16.83.1	Test Purpose and Environment	2898

A.8.16.83.2	Test Requirements	2903

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2903

A.8.16.84.1	Test Purpose and Environment	2903

A.8.16.84.2	Test Requirements	2909

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2909

A.8.16.85.1	Test Purpose and Environment	2909

A.8.16.85.2	Test Requirements	2913

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2913

A.8.16.86.1	Test Purpose and Environment	2913

A.8.16.86.2	Test Requirements	2916

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2916

A.8.16.87.1	Test Purpose and Environment	2916

A.8.16.87.2	Test Requirements	2922

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2922

A.8.16.88.1	Test Purpose and Environment	2922

A.8.16.88.2	Test Requirements	2930

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2930

A.8.16.89.1	Test Purpose and Environment	2930

A.8.16.89.2	Test Requirements	2937

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2937

A.8.16.90.1	Test Purpose and Environment	2937

A.8.16.90.2	Test Requirements	2944

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2944

A.8.16.91.1	Test Purpose and Environment	2944

A.8.16.91.2	Test Requirements	2950

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2950

A.8.16.92.1	Test Purpose and Environment	2950

A.8.16.92.2	Test Requirements	2958

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2958

A.8.16.93.1	Test Purpose and Environment	2958

A.8.16.93.2	Test Requirements	2965

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2965

A.8.16.94.1	Test Purpose and Environment	2965

A.8.16.94.2	Test Requirements	2972

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2972

A.8.16.95.1	Test Purpose and Environment	2972

A.8.16.95.2	Test Requirements	2980

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2980

A.8.16.96.1	Test Purpose and Environment	2980

A.8.16.96.2	Test Requirements	2989

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2989

A.8.16.97.1	Test Purpose and Environment	2989

A.8.16.97.2	Test Requirements	2996

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2996

A.8.16.98.1	Test Purpose and Environment	2996

A.8.16.98.2	Test Requirements	3004

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	3004

A.8.16.99.1	Test Purpose and Environment	3004

A.8.16.99.2	Test Requirements	3014

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	3014

A.8.16.100.1	Test Purpose and Environment	3014

A.8.16.100.2	Test Requirements	3024

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	3024

A.8.16.101.1	Test Purpose and Environment	3024

A.8.16.101.2	Test Requirements	3034

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	3034

A.8.16.102.1	Test Purpose and Environment	3034

A.8.16.102.2	Test Requirements	3045

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	3045

A.8.16.103.1	Test Purpose and Environment	3045

A.8.16.103.2	Test Requirements	3050

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	3051

A.8.16.104.1	Test Purpose and Environment	3051

A.8.16.104.2	Test Requirements	3055

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	3056

A.8.16.105.1	Test Purpose and Environment	3056

A.8.16.105.2	Test Requirements	3060

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	3061

A.8.16.106.1	Test Purpose and Environment	3061

A.8.16.106.2	Test Requirements	3065

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	3065

A.8.16.107.1	Test Purpose and Environment	3065

A.8.16.107.2	Test Requirements	3067

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	3068

A.8.16.108.1	Test Purpose and Environment	3068

A.8.16.108.2	Test Requirements	3070

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	3071

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	3071

A.8.17.1.1	Test Purpose and Environment	3071

A.8.17.1.2	Test Requirements	3077

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	3077

A.8.17.2.1	Test Purpose and Environment	3077

A.8.17.2.2	Test Requirements	3084

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	3084

A.8.17.3.1	Test Purpose and Environment	3084

A.8.17.3.2	Test Requirements	3085

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	3085

A.8.17.4.1	Test Purpose and Environment	3085

A.8.17.4.2	Test Requirements	3086

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	3086

A.8.17.5.1	Test Purpose and Environment	3086

A.8.17.5.2	Test Requirements	3087

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	3087

A.8.17.6.1	Test Purpose and Environment	3087

A.8.17.6.2	Test Requirements	3088

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	3089

A.8.17.7.1	Test Purpose and Environment	3089

A.8.17.7.2	Test Requirements	3089

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	3090

A.8.17.8.1	Test Purpose and Environment	3090

A.8.17.8.2	Test Requirements	3090

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	3091

A.8.17.9.1	Test Purpose and Environment	3091

A.8.17.9.2	Test Requirements	3092

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	3092

A.8.17.10.1	Test Purpose and Environment	3092

A.8.17.10.2	Test Requirements	3099

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	3100

A.8.17.11.1	Test Purpose and Environment	3100

A.8.17.11.2	Test Requirements	3106

A.8.18	E-UTRAN TDD – HRPD Measurements	3107

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	3107

A.8.18.1.1	Test Purpose and Environment	3107

A.8.18.1.2	Test Requirements	3109

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	3109

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	3109

A.8.19.1.1	Test Purpose and Environment	3109

A.8.19.1.2	Test Requirements	3110

A.8.20	Inter-frequency/RAT Measurements in CA mode	3111

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	3111

A.8.20.1.1	Test Purpose and Environment	3111

A.8.20.1.2	Test Requirements	3112

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3112

A.8.20.2.1	Test Purpose and Environment	3113

A.8.20.2.2	Test Requirements	3114

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	3115

A.8.20.2A.1	Test Purpose and Environment	3115

A.8.20.2A.2	Test Requirements	3115

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	3115

A.8.20.2B.1	Test Purpose and Environment	3115

A.8.20.2B.2	Test Requirements	3118

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	3118

A.8.20.3.1	Test Purpose and Environment	3118

A.8.20.3.2	Test Requirements	3120

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	3120

A.8.20.4.1	Test Purpose and Environment	3120

A.8.20.4.1.1	1.28 Mcps TDD option	3120

A.8.20.4.2	Test Requirements	3122

A.8.20.4.2.1	1.28 Mcps TDD option	3122

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	3122

A.8.20.4A.1	Test Purpose and Environment	3122

A.8.20.4A.1.1	1.28 Mcps TDD option	3122

A.8.20.4A.2	Test Requirements	3122

A.8.20.4A.2.1	1.28 Mcps TDD option	3122

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	3123

A.8.20.4B.1	Test Purpose and Environment	3123

A.8.20.4B.1.1	1.28 Mcps TDD option	3123

A.8.20.4B.2	Test Requirements	3125

A.8.20.4B.2.1	1.28 Mcps TDD option	3125

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	3125

A.8.21.1	Test Purpose and Environment	3125

A.8.21.2 Test Requirements	3129

A.8.22	E-UTRAN Discovery Signal Measurements	3129

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	3129

A.8.22.1.1	Test Purpose and Environment	3129

A.8.22.1.2	Test Requirements	3132

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	3132

A.8.22.2.1	Test Purpose and Environment	3132

A.8.22.2.2	Test Requirements	3135

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	3135

A.8.22.3.1	Test Purpose and Environment	3135

A.8.22.3.2	Test Requirements	3138

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	3138

A.8.22.4.1	Test Purpose and Environment	3138

A.8.22.4.2	Test Requirements	3141

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3141

A.8.22.5.1	Test Purpose and Environment	3141

A.8.22.5.2	Test Requirements	3145

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3145

A.8.22.6.1	Test Purpose and Environment	3145

A.8.22.6.2	Test Requirements	3149

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3149

A.8.22.7.1	Test Purpose and Environment	3149

A.8.22.7.2	Test Requirements	3153

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	3153

A.8.22.8.1	Test Purpose and Environment	3153

A.8.22.8.2	Test Requirements	3157

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	3157

A.8.22.9.1	Test Purpose and Environment	3157

A.8.22.9.2	Test Requirements	3159

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	3160

A.8.22.10.1	Test Purpose and Environment	3160

A.8.22.10.2	Test Requirements	3162

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	3163

A.8.22.11.1	Test Purpose and Environment	3163

A.8.22.11.2	Test Requirements	3166

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	3166

A.8.22.12.1	Test Purpose and Environment	3166

A.8.22.12.2	Test Requirements	3170

A.8.23	E-UTRAN Dual Connectivity Measurements	3170

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	3170

A.8.23.1.1	Test Purpose and Environment	3170

A.8.23.1.2	Test Requirements	3173

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	3173

A.8.23.2.1	Test Purpose and Environment	3173

A.8.23.2.2	Test Requirements	3176

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	3176

A.8.23.3.1	Test Purpose and Environment	3176

A.8.23.3.2	Test Requirements	3179

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	3179

A.8.23.4.1	Test Purpose and Environment	3179

A.8.23.4.2	Test Requirements	3182

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	3182

A.8.23.5.1	Test Purpose and Environment	3182

A.8.23.5.2	Test Requirements	3185

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	3185

A.8.23.6.1	Test Purpose and Environment	3185

A.8.23.6.2	Test Requirements	3188

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	3188

A.8.23.7.1	Test Purpose and Environment	3188

A.8.23.7.2	Test Requirements	3190

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	3191

A.8.23.8.1	Test Purpose and Environment	3191

A.8.23.8.2	Test Requirements	3193

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	3194

A.8.23.9.1	Test Purpose and Environment	3194

A.8.23.9.2	Test Requirements	3197

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	3197

A.8.23.10.1	Test Purpose and Environment	3197

A.8.23.10.2	Test Requirements	3200

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	3200

A.8.23.11.1	Test Purpose and Environment	3200

A.8.23.11.2	Test Requirements	3203

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	3203

A.8.23.12.1	Test Purpose and Environment	3203

A.8.23.12.2	Test Requirements	3206

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	3206

A.8.23.13.1	Test Purpose and Environment	3206

A.8.23.13.2	Test Requirements	3209

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	3209

A.8.23.14.1	Test Purpose and Environment	3209

A.8.23.14.2	Test Requirements	3212

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	3212

A.8.23.15.1	Test Purpose and Environment	3212

A.8.23.15.2	Test Requirements	3215

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	3215

A.8.23.16.1	Test Purpose and Environment	3215

A.8.23.16.2	Test Requirements	3216

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	3217

A.8.23.17.1	Test Purpose and Environment	3217

A.8.23.17.2	Test Requirements	3219

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3219

A.8.23.18.1	Test Purpose and Environment	3219

A.8.23.18.2	Test Requirements	3220

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3221

A.8.23.19.1	Test Purpose and Environment	3221

A.8.23.19.2	Test Requirements	3222

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3223

A.8.23.20.1	Test Purpose and Environment	3223

A.8.23.20.2	Test Requirements	3224

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3225

A.8.23.21.1	Test Purpose and Environment	3225

A.8.23.21.2	Test Requirements	3226

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3227

A.8.23.22.1	Test Purpose and Environment	3227

A.8.23.22.2	Test Requirements	3228

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3229

A.8.23.23.1	Test Purpose and Environment	3229

A.8.23.23.2	Test Requirements	3230

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3231

A.8.23.24.1	Test Purpose and Environment	3231

A.8.23.24.2	Test Requirements	3233

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	3234

A.8.23.25.1	Test Purpose and Environment	3234

A.8.23.25.2	Test Requirements	3236

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3237

A.8.23.26.1	Test Purpose and Environment	3237

A.8.23.26.2	Test Requirements	3239

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3240

A.8.23.27.1	Test Purpose and Environment	3240

A.8.23.27.2	Test Requirements	3243

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	3243

A.8.23.28.1	Test Purpose and Environment	3243

A.8.23.28.2	Test Requirements	3247

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3247

A.8.23.29.1	Test Purpose and Environment	3247

A.8.23.29.2	Test Requirements	3251

A.8.24	Proximity-based Services	3251

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3251

A.8.24.1.1	Test Purpose and Environment	3251

A.8.24.1.2	Test Requirements	3252

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3253

A.8.24.2.1	Test Purpose and Environment	3253

A.8.24.2.2	Test Requirements	3254

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3254

A.8.24.3.1	Test Purpose and Environment	3255

A.8.24.3.2	Test Requirements	3256

A.8.25	E-UTRAN-WLAN Measurements	3257

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3257

A.8.25.1.1	Test Purpose and Environment	3257

A.8.25.1.2	Test Requirements	3259

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3259

A.8.25.2.1	Test Purpose and Environment	3259

A.8.25.2.2	Test Requirements	3261

A.8.26 Frame Structure 3 (FS3)	3261

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	3261

A.8.26.1.1	Test Purpose and Environment	3261

A.8.26.1.2	Test Requirements	3263

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	3264

A.8.26.2.1	Test Purpose and Environment	3264

A.8.26.2.2	Test Requirements	3266

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3267

A.8.26.3.1	Test Purpose and Environment	3267

A.8.26.3.2	Test Requirements	3270

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3270

A.8.26.3A.1	Test Purpose and Environment	3270

A.8.26.3A.2	Test Requirements	3273

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	3273

A.8.26.4.1	Test Purpose and Environment	3273

A.8.26.4.2	Test Requirements	3277

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3277

A.8.26.4A.1	Test Purpose and Environment	3277

A.8.26.4A.2	Test Requirements	3281

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3281

A.8.26.5.1	Test Purpose and Environment	3281

A.8.26.5.2	Test Requirements	3284

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3284

A.8.26.5A.1	Test Purpose and Environment	3284

A.8.26.5A.2	Test Requirements	3288

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3288

A.8.26.6.1	Test Purpose and Environment	3288

A.8.26.6.2	Test Requirements	3292

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3292

A.8.26.6A.1	Test Purpose and Environment	3292

A.8.26.6A.2	Test Requirements	3296

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3296

A.8.26.7.1	Test Purpose and Environment	3296

A.8.26.7.2	Test Requirements	3299

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3299

A.8.26.8.1	Test Purpose and Environment	3299

A.8.26.8.2	Test Requirements	3303

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3303

A.8.26.9.1	Test Purpose and Environment	3303

A.8.26.9.2	Test Requirements	3306

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3306

A.8.26.10.1	Test Purpose and Environment	3306

A.8.26.10.2	Test Requirements	3309

A.9	Measurement Performance Requirements	3311

A.9.1	RSRP	3311

A.9.1.1	FDD Intra frequency case	3311

A.9.1.1.1	Test Purpose and Environment	3311

A.9.1.1.2	Test parameters	3311

A.9.1.1.3	Test Requirements	3314

A.9.1.2	TDD Intra frequency case	3314

A.9.1.2.1	Test Purpose and Environment	3314

A.9.1.2.2	Test parameters	3314

A.9.1.2.3	Test Requirements	3315

A.9.1.3	FDD—FDD Inter frequency case	3316

A.9.1.3.1	Test Purpose and Environment	3316

A.9.1.3.2	Test parameters	3316

A.9.1.3.3	Test Requirements	3319

A.9.1.4	TDD—TDD Inter frequency case	3319

A.9.1.4.1	Test Purpose and Environment	3319

A.9.1.4.2	Test parameters	3319

A.9.1.4.3	Test Requirements	3321

A.9.1.5	FDD—TDD Inter frequency case	3322

A.9.1.5.1	Test Purpose and Environment	3322

A.9.1.5.2	Test parameters	3322

A.9.1.5.3	Test Requirements	3323

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	3324

A.9.1.6.1	Test Purpose and Environment	3324

A.9.1.6.2	Test parameters	3324

A.9.1.6.3	Test Requirements	3327

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	3327

A.9.1.7.1	Test Purpose and Environment	3327

A.9.1.7.2	Test parameters	3327

A.9.1.7.3	Test Requirements	3330

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3330

A.9.1.8.1	Test Purpose and Environment	3330

A.9.1.8.2	Test parameters	3330

A.9.1.8.3	Test Requirements	3334

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3334

A.9.1.9.1	Test Purpose and Environment	3334

A.9.1.9.2	Test parameters	3334

A.9.1.9.3	Test Requirements	3337

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3337

A.9.1.10.1	Test Purpose and Environment	3337

A.9.1.10.2	Test parameters	3337

A.9.1.10.3	Test Requirements	3340

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3340

A.9.1.11.1	Test Purpose and Environment	3340

A.9.1.11.2	Test parameters	3340

A.9.1.11.3	Test Requirements	3344

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3344

A.9.1.12.1	Test Purpose and Environment	3344

A.9.1.12.2	Test parameters	3344

A.9.1.12.3	Test Requirements	3345

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3345

A.9.1.13.1	Test Purpose and Environment	3345

A.9.1.13.2	Test parameters	3345

A.9.1.13.3	Test Requirements	3346

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3346

A.9.1.14.1	Test Purpose and Environment	3346

A.9.1.14.2	Test parameters	3346

A.9.1.14.3	Test Requirements	3350

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3350

A.9.1.15.1	Test Purpose and Environment	3350

A.9.1.15.2	Test parameters	3350

A.9.1.15.3	Test Requirements	3352

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	3353

A.9.1.16.1	Test Purpose and Environment	3353

A.9.1.16.2	Test parameters	3353

A.9.1.16.3	Test Requirements	3354

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	3354

A.9.1.17.1	Test Purpose and Environment	3354

A.9.1.17.2	Test parameters	3355

A.9.1.17.3	Test Requirements	3356

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3356

A.9.1.18.1	Test Purpose and Environment	3356

A.9.1.18.2	Test parameters	3356

A.9.1.18.3	Test Requirements	3357

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3358

A.9.1.19.1	Test Purpose and Environment	3358

A.9.1.19.2	Test parameters	3358

A.9.1.19.3	Test Requirements	3358

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3358

A.9.1.20.1	Test Purpose and Environment	3358

A.9.1.20.2	Test parameters	3358

A.9.1.20.3	Test Requirements	3360

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3360

A.9.1.21.1	Test Purpose and Environment	3360

A.9.1.21.2	Test parameters	3360

A.9.1.21.3	Test Requirements	3360

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3360

A.9.1.22.1	Test Purpose and Environment	3360

A.9.1.22.2	Test parameters	3361

A.9.1.22.3	Test Requirements	3365

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3365

A.9.1.23.1	Test Purpose and Environment	3365

A.9.1.23.2	Test parameters	3365

A.9.1.23.3	Test Requirements	3369

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	3369

A.9.1.24.1	Test Purpose and Environment	3369

A.9.1.24.2	Test parameters	3369

A.9.1.24.3	Test Requirements	3370

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3370

A.9.1.25.1	Test Purpose and Environment	3370

A.9.1.25.2	Test parameters	3370

A.9.1.25.3	Test Requirements	3373

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3373

A.9.1.26.1	Test Purpose and Environment	3373

A.9.1.26.2	Test parameters	3373

A.9.1.26.3	Test Requirements	3376

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3376

A.9.1.27.1	Test Purpose and Environment	3376

A.9.1.27.2	Test parameters	3376

A.9.1.27.3	Test Requirements	3379

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	3379

A.9.1.28.1	Test Purpose and Environment	3379

A.9.1.28.2	Test parameters	3379

A.9.1.28.3	Test Requirements	3382

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3382

A.9.1.29.1	Test Purpose and Environment	3382

A.9.1.29.2	Test parameters	3382

A.9.1.29.3	Test Requirements	3385

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3385

A.9.1.30.1	Test Purpose and Environment	3385

A.9.1.30.2	Test parameters	3386

A.9.1.30.3	Test Requirements	3389

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3389

A.9.1.31.1	Test Purpose and Environment	3389

A.9.1.31.2	Test parameters	3389

A.9.1.31.3	Test Requirements	3392

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	3393

A.9.1.32.1	Test Purpose and Environment	3393

A.9.1.32.2	Test parameters	3393

A.9.1.32.3	Test Requirements	3396

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3396

A.9.1.33.1	Test Purpose and Environment	3396

A.9.1.33.2	Test parameters	3396

A.9.1.33.3	Test Requirements	3399

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3399

A.9.1.34.1	Test Purpose and Environment	3399

A.9.1.34.2	Test parameters	3399

A.9.1.34.3	Test Requirements	3402

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3402

A.9.1.35.1	Test Purpose and Environment	3402

A.9.1.35.2	Test parameters	3402

A.9.1.35.3	Test Requirements	3405

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3406

A.9.1.36.1	Test Purpose and Environment	3406

A.9.1.36.2	Test parameters	3406

A.9.1.36.3	Test Requirements	3409

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3409

A.9.1.37.1	Test Purpose and Environment	3409

A.9.1.37.2	Test parameters	3409

A.9.1.37.3	Test Requirements	3412

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3413

A.9.1.38.1	Test Purpose and Environment	3413

A.9.1.38.2	Test parameters	3413

A.9.1.38.3	Test Requirements	3416

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3417

A.9.1.39.1	Test Purpose and Environment	3417

A.9.1.39.2	Test parameters	3417

A.9.1.39.3	Test Requirements	3423

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3423

A.9.1.40.1	Test Purpose and Environment	3423

A.9.1.40.2	Test parameters	3424

A.9.1.40.3	Test Requirements	3430

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	3430

A.9.1.41.1	Test Purpose and Environment	3430

A.9.1.41.2	Test parameters	3430

A.9.1.41.3	Test Requirements	3433

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	3433

A.9.1.42.1	Test Purpose and Environment	3433

A.9.1.42.2	Test parameters	3433

A.9.1.42.3	Test Requirements	3436

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	3436

A.9.1.43.1	Test Purpose and Environment	3436

A.9.1.43.2	Test parameters	3436

A.9.1.43.3	Test Requirements	3437

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3438

A.9.1.44.1	Test Purpose and Environment	3438

A.9.1.44.2	Test parameters	3438

A.9.1.44.3	Test Requirements	3443

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3443

A.9.1.45.1	Test Purpose and Environment	3443

A.9.1.45.2	Test parameters	3443

A.9.1.45.3	Test Requirements	3432

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3432

A.9.1.46.1	Test Purpose and Environment	3432

A.9.1.46.2	Test parameters	3432

A.9.1.46.3	Test Requirements	3441

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3442

A.9.1.47.1	Test Purpose and Environment	3442

A.9.1.47.2	Test parameters	3442

A.9.1.47.3	Test Requirements	3451

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3451

A.9.1.48.1	Test Purpose and Environment	3451

A.9.1.48.2	Test parameters	3452

A.9.1.48.3	Test Requirements	3462

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3463

A.9.1.49.1	Test Purpose and Environment	3463

A.9.1.49.2	Test parameters	3463

A.9.1.49.3	Test Requirements	3473

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3474

A.9.1.50.1	Test Purpose and Environment	3474

A.9.1.50.2	Test parameters	3474

A.9.1.50.3	Test Requirements	3481

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3482

A.9.1.51.1	Test Purpose and Environment	3482

A.9.1.51.2	Test parameters	3482

A.9.1.51.3	Test Requirements	3488

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3489

A.9.1.52.1	Test Purpose and Environment	3489

A.9.1.52.2	Test parameters	3489

A.9.1.52.3	Test Requirements	3492

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	3492

A.9.1.52A.1	Test Purpose and Environment	3492

A.9.1.52A.2	Test parameters	3492

A.9.1.52A.3	Test Requirements	3493

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3493

A.9.1.53.1	Test Purpose and Environment	3493

A.9.1.53.2	Test parameters	3494

A.9.1.53.3	Test Requirements	3497

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	3497

A.9.1.53A.1	Test Purpose and Environment	3497

A.9.1.53A.2	Test parameters	3497

A.9.1.53A.3	Test Requirements	3498

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3499

A.9.1.54.1	Test Purpose and Environment	3499

A.9.1.54.2	Test parameters	3499

A.9.1.54.3	Test Requirements	3501

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	3501

A.9.1.55.1	Test Purpose and Environment	3501

A.9.1.55.2	Test parameters	3501

A.9.1.55.3	Test Requirements	3505

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	3505

A.9.1.56.1	Test Purpose and Environment	3505

A.9.1.56.2	Test parameters	3505

A.9.1.56.3	Test Requirements	3508

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3509

A.9.1.57.1	Test Purpose and Environment	3509

A.9.1.57.2	Test parameters	3509

A.9.1.57.3	Test Requirements	3512

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	3512

A.9.1.57A.1	Test Purpose and Environment	3512

A.9.1.57A.2	Test parameters	3512

A.9.1.57A.3	Test Requirements	3513

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3513

A.9.1.58.1	Test Purpose and Environment	3513

A.9.1.58.2	Test parameters	3514

A.9.1.58.3	Test Requirements	3517

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	3517

A.9.1.58A.1	Test Purpose and Environment	3517

A.9.1.58A.2	Test parameters	3517

A.9.1.58A.3	Test Requirements	3518

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3518

A.9.1.59.1	Test Purpose and Environment	3518

A.9.1.59.2	Test parameters	3519

A.9.1.59.3	Test Requirements	3521

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	3521

A.9.1.60.1	Test Purpose and Environment	3521

A.9.1.60.2	Test parameters	3521

A.9.1.60.3	Test Requirements	3524

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	3525

A.9.1.61.1	Test Purpose and Environment	3525

A.9.1.61.2	Test parameters	3525

A.9.1.61.3	Test Requirements	3528

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3528

A.9.1.62.1	Test Purpose and Environment	3528

A.9.1.62.2	Test parameters	3528

A.9.1.62.3	Test Requirements	3531

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3531

A.9.1.53.1	Test Purpose and Environment	3531

A.9.1.53.2	Test parameters	3531

A.9.1.63.3	Test Requirements	3534

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3534

A.9.1.64.1	Test Purpose and Environment	3534

A.9.1.64.2	Test parameters	3534

A.9.1.64.3	Test Requirements	3535

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3536

A.9.1.65.1	Test Purpose and Environment	3536

A.9.1.65.2	Test parameters	3536

A.9.1.65.3	Test Requirements	3542

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3542

A.9.1.66.1	Test Purpose and Environment	3542

A.9.1.66.2	Test parameters	3542

A.9.1.66.3	Test Requirements	3548

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3548

A.9.1.67.1	Test Purpose and Environment	3548

A.9.1.67.2	Test parameters	3548

A.9.1.67.3	Test Requirements	3549

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3550

A.9.1.68.1	Test Purpose and Environment	3550

A.9.1.68.2	Test parameters	3550

A.9.1.68.3	Test Requirements	3554

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3554

A.9.1.69.1	Test Purpose and Environment	3554

A.9.1.69.2	Test parameters	3554

A.9.1.69.3	Test Requirements	3558

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3559

A.9.1.70.1	Test Purpose and Environment	3559

A.9.1.70.2	Test parameters	3559

A.9.1.70.3	Test Requirements	3571

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3572

A.9.1.71.1	Test Purpose and Environment	3572

A.9.1.71.2	Test parameters	3572

A.9.1.71.3	Test Requirements	3584

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3585

A.9.1.72.1	Test Purpose and Environment	3585

A.9.1.72.2	Test parameters	3585

A.9.1.72.3	Test Requirements	3597

A.9.1.73	FDD Intra frequency case for CA Idle Mode Measurements	3598

A.9.1.73.1	Test Purpose and Environment	3598

A.9.1.73.2	Test parameters	3598

A.9.1.73.3	Test Requirements	3601

A.9.1.74	FDD—FDD Inter frequency case for CA Idle Mode Measurements for overlapping carrier	3601

A.9.1.74.1	Test Purpose and Environment	3601

A.9.1.74.2	Test parameters	3601

A.9.1.74.3	Test Requirements	3602

A.9.1.75	FDD—FDD Inter frequency case for CA Idle Mode Measurements for non-overlapping carrier	3602

A.9.1.75.1	Test Purpose and Environment	3602

A.9.1.75.2	Test parameters	3603

A.9.1.75.3	Test Requirements	3604

A.9.1.76	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3604

A.9.1.76.1	Test Purpose and Environment	3604

A.9.1.76.2	Test parameters	3604

A.9.1.76.3	Test Requirements	3607

A.9.1.77	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3607

A.9.1.77.1	Test Purpose and Environment	3607

A.9.1.77.2	Test parameters	3607

A.9.1.77.3	Test Requirements	3610

A.9.1.78	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3610

A.9.1.78.1	Test Purpose and Environment	3610

A.9.1.78.2	Test parameters	3610

A.9.1.78.3	Test Requirements	3613

A.9.1.79	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3613

A.9.1.79.1	Test Purpose and Environment	3613

A.9.1.79.2	Test parameters	3613

A.9.1.79.3	Test Requirements	3616

A.9.1.80	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3616

A.9.1.80.1	Test Purpose and Environment	3616

A.9.1.80.2	Test parameters	3616

A.9.1.80.3	Test Requirements	3619

A.9.1.81	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3619

A.9.1.81.1	Test Purpose and Environment	3619

A.9.1.78.2	Test parameters	3619

A.9.1.81.3	Test Requirements	3622

A.9.2	RSRQ	3622

A.9.2.1	FDD Intra frequency case	3622

A.9.2.1.1	Test Purpose and Environment	3622

A.9.2.1.2	Test parameters	3622

A.9.2.1.3	Test Requirements	3625

A.9.2.2	TDD Intra frequency case	3625

A.9.2.2.1	Test Purpose and Environment	3625

A.9.2.2.2	Test parameters	3625

A.9.2.2.3	Test Requirements	3627

A.9.2.3	FDD—FDD Inter frequency case	3627

A.9.2.3.1	Test Purpose and Environment	3627

A.9.2.3.2	Test parameters	3627

A.9.2.3.3	Test Requirements	3630

A.9.2.4	TDD—TDD Inter frequency case	3630

A.9.2.4.1	Test Purpose and Environment	3630

A.9.2.4.2	Test parameters	3630

A.9.2.4.3	Test Requirements	3632

A.9.2.4A	FDD—TDD Inter frequency case	3633

A.9.2.4A.1	Test Purpose and Environment	3633

A.9.2.4A.2	Test parameters	3633

A.9.2.4A.3	Test Requirements	3635

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	3635

A.9.2.5.1	Test Purpose and Environment	3635

A.9.2.5.2	Test parameters	3636

A.9.2.5.3	Test Requirements	3639

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	3639

A.9.2.6.1	Test Purpose and Environment	3639

A.9.2.6.2	Test parameters	3639

A.9.2.6.3	Test Requirements	3642

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	3642

A.9.2.7.1	Test Purpose and Environment	3642

A.9.2.7.2	Test parameters	3642

A.9.2.7.3	Test Requirements	3646

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	3646

A.9.2.8.1	Test Purpose and Environment	3646

A.9.2.8.2	Test parameters	3646

A.9.2.8.3	Test Requirements	3650

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	3650

A.9.2.9.1	Test Purpose and Environment	3650

A.9.2.9.2	Test parameters	3650

A.9.2.9.3	Test Requirements	3654

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	3654

A.9.2.10.1	Test Purpose and Environment	3654

A.9.2.10.2	Test parameters	3654

A.9.2.10.3	Test Requirements	3658

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	3658

A.9.2.11.1	Test Purpose and Environment	3658

A.9.2.11.2	Test parameters	3658

A.9.2.11.3	Test Requirements	3659

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	3659

A.9.2.12.1	Test Purpose and Environment	3659

A.9.2.12.2	Test parameters	3660

A.9.2.12.3	Test Requirements	3660

A.9.2.13	Void	3661

A.9.2.13.1	Void	3661

A.9.2.13.2	Void	3661

A.9.2.13.3	Void	3661

A.9.2.14	Void	3661

A.9.2.14.1	Void	3661

A.9.2.14.2	Void	3661

A.9.2.14.3	Void	3661

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3661

A.9.2.15.1	Test Purpose and Environment	3661

A.9.2.15.2	Test parameters	3661

A.9.2.15.3	Test Requirements	3665

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3665

A.9.2.16.1	Test Purpose and Environment	3665

A.9.2.16.2	Test parameters	3665

A.9.2.16.3	Test Requirements	3670

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	3670

A.9.2.17.1	Test Purpose and Environment	3670

A.9.2.17.2	Test parameters	3670

A.9.2.17.3	Test Requirements	3672

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	3672

A.9.2.18.1	Test Purpose and Environment	3672

A.9.2.18.2	Test parameters	3672

A.9.2.18.3	Test Requirements	3675

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	3675

A.9.2.19.1	Test Purpose and Environment	3675

A.9.2.19.2	Test parameters	3675

A.9.2.19.3	Test Requirements	3677

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	3677

A.9.2.20.1	Test Purpose and Environment	3677

A.9.2.20.2	Test parameters	3677

A.9.2.20.3	Test Requirements	3680

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	3680

A.9.2.21.1	Test Purpose and Environment	3680

A.9.2.21.2	Test parameters	3680

A.9.2.21.3	Test Requirements	3683

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	3683

A.9.2.22.1	Test Purpose and Environment	3684

A.9.2.22.2	Test parameters	3684

A.9.2.22.3	Test Requirements	3684

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	3684

A.9.2.23.1	Test Purpose and Environment	3684

A.9.2.23.2	Test parameters	3684

A.9.2.23.3	Test Requirements	3687

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	3687

A.9.2.24.1	Test Purpose and Environment	3687

A.9.2.24.2	Test parameters	3687

A.9.2.24.3	Test Requirements	3688

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3688

A.9.2.25.1	Test Purpose and Environment	3688

A.9.2.25.2	Test parameters	3688

A.9.2.25.3	Test Requirements	3693

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3693

A.9.2.26.1	Test Purpose and Environment	3693

A.9.2.26.2	Test parameters	3693

A.9.2.26.3	Test Requirements	3697

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	3697

A.9.2.27.1	Test Purpose and Environment	3697

A.9.2.27.2	Test parameters	3697

A.9.2.27.3	Test Requirements	3698

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	3698

A.9.2.28.1	Test Purpose and Environment	3698

A.9.2.28.2	Test parameters	3698

A.9.2.28.3	Test Requirements	3701

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	3701

A.9.2.29.1	Test Purpose and Environment	3701

A.9.2.29.2	Test parameters	3701

A.9.2.29.3	Test Requirements	3704

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	3704

A.9.2.30.1	Test Purpose and Environment	3704

A.9.2.30.2	Test parameters	3704

A.9.2.30.3	Test Requirements	3707

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	3707

A.9.2.31.1	Test Purpose and Environment	3707

A.9.2.31.2	Test parameters	3707

A.9.2.31.3	Test Requirements	3710

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	3710

A.9.2.32.1	Test Purpose and Environment	3710

A.9.2.32.2	Test parameters	3710

A.9.2.32.3	Test Requirements	3713

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	3713

A.9.2.33.1	Test Purpose and Environment	3713

A.9.2.33.2	Test parameters	3713

A.9.2.33.3	Test Requirements	3717

A.9.2.34	FDD—FDD Inter frequency new RSRQ	3717

A.9.2.34.1	Test Purpose and Environment	3717

A.9.2.34.2	Test parameters	3717

A.9.2.34.3	Test Requirements	3720

A.9.2.35	TDD—TDD Inter frequency new RSRQ	3720

A.9.2.35.1	Test Purpose and Environment	3720

A.9.2.35.2	Test parameters	3720

A.9.2.35.3	Test Requirements	3723

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	3723

A.9.2.36.1	Test Purpose and Environment	3723

A.9.2.36.2	Test parameters	3723

A.9.2.36.3	Test Requirements	3724

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	3725

A.9.2.37.1	Test Purpose and Environment	3725

A.9.2.37.2	Test parameters	3725

A.9.2.37.3	Test Requirements	3726

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	3727

A.9.2.38.1	Test Purpose and Environment	3727

A.9.2.38.2	Test parameters	3727

A.9.2.38.3	Test Requirements	3731

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	3731

A.9.2.39.1	Test Purpose and Environment	3731

A.9.2.39.2	Test parameters	3731

A.9.2.39.3	Test Requirements	3735

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3735

A.9.2.40.1	Test Purpose and Environment	3735

A.9.2.40.2	Test parameters	3735

A.9.2.40.3	Test Requirements	3738

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3739

A.9.2.41.1	Test Purpose and Environment	3739

A.9.2.41.2	Test parameters	3739

A.9.2.41.3	Test Requirements	3742

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	3742

A.9.2.42.1	Test Purpose and Environment	3742

A.9.2.42.2	Test parameters	3742

A.9.2.42.3	Test Requirements	3745

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	3745

A.9.2.43.1	Test Purpose and Environment	3745

A.9.2.43.2	Test parameters	3745

A.9.2.43.3	Test Requirements	3748

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	3748

A.9.2.44.1	Test Purpose and Environment	3748

A.9.2.44.2	Test parameters	3748

A.9.2.44.3	Test Requirements	3750

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	3750

A.9.2.45.1	Test Purpose and Environment	3750

A.9.2.45.2	Test parameters	3750

A.9.2.45.3	Test Requirements	3754

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	3755

A.9.2.46.1	Test Purpose and Environment	3755

A.9.2.46.2	Test parameters	3755

A.9.2.46.3	Test Requirements	3759

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	3760

A.9.2.47.1	Test Purpose and Environment	3760

A.9.2.47.2	Test parameters	3760

A.9.2.47.3	Test Requirements	3764

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	3764

A.9.2.48.1	Test Purpose and Environment	3764

A.9.2.48.2	Test parameters	3765

A.9.2.48.3	Test Requirements	3769

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3770

A.9.2.49.1	Test Purpose and Environment	3770

A.9.2.49.2	Test parameters	3770

A.9.2.49.3	Test Requirements	3776

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3777

A.9.2.50.1	Test Purpose and Environment	3777

A.9.2.50.2	Test parameters	3777

A.9.2.50.3	Test Requirements	3783

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	3783

A.9.2.51.1	Test Purpose and Environment	3783

A.9.2.51.2	Test parameters	3784

A.9.2.51.3	Test Requirements	3788

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	3788

A.9.2.52.1	Test Purpose and Environment	3788

A.9.2.52.2	Test parameters	3788

A.9.2.52.3	Test Requirements	3791

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3792

A.9.2.53.1	Test Purpose and Environment	3792

A.9.2.53.2	Test parameters	3792

A.9.2.53.3	Test Requirements	3798

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3799

A.9.2.54.1	Test Purpose and Environment	3799

A.9.2.54.2	Test parameters	3799

A.9.2.54.3	Test Requirements	3805

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3805

A.9.2.55.1	Test Purpose and Environment	3805

A.9.2.55.2	Test parameters	3806

A.9.2.55.3	Test Requirements	3811

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3811

A.9.2.56.1	Test Purpose and Environment	3811

A.9.2.56.2	Test parameters	3811

A.9.2.56.3	Test Requirements	3816

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3816

A.9.2.57.1	Test Purpose and Environment	3816

A.9.2.57.2	Test parameters	3817

A.9.2.57.3	Test Requirements	3821

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3822

A.9.2.58.1	Test Purpose and Environment	3822

A.9.2.58.2	Test parameters	3822

A.9.2.58.3	Test Requirements	3826

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3827

A.9.2.59.1	Test Purpose and Environment	3827

A.9.2.59.2	Test parameters	3827

A.9.2.59.3	Test Requirements	3832

A.9.2.60	FDD Intra frequency case for CA Idle Mode Measurements	3833

A.9.2.60.1	Test Purpose and Environment	3833

A.9.2.60.2	Test parameters	3833

A.9.2.60.3	Test Requirements	3836

A.9.2.61	FDD—FDD Inter frequency case for CA Idle Mode Measurements on overlapping carrier	3836

A.9.2.61.1	Test Purpose and Environment	3836

A.9.2.61.2	Test parameters	3836

A.9.2.61.3	Test Requirements	3839

A.9.2.62	FDD—FDD Inter frequency case for CA Idle Mode Measurements on non-overlapping carrier	3839

A.9.2.62.1	Test Purpose and Environment	3839

A.9.2.62.2	Test parameters	3839

A.9.2.62.3	Test Requirements	3842

A.9.3	UTRAN FDD CPICH RSCP	3842

A.9.3.1	E-UTRAN FDD	3842

A.9.3.1.1	Test Purpose and Environment	3842

A.9.3.1.2	Parameters	3842

A.9.3.1.3	Test Requirements	3845

A.9.3.2 E-UTRAN TDD	3845

A.9.3.2.1	Test Purpose and Environment	3845

A.9.3.2.2	Parameters	3845

A.9.3.2.3	Test Requirements	3848

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	3848

A.9.3.3.1	Test Purpose and Environment	3848

A.9.3.3.2	Parameters	3848

A.9.3.3.3	Test Requirements	3849

A.9.4	UTRAN FDD CPICH Ec/No	3850

A.9.4.1	E-UTRAN FDD	3850

A.9.4.1.1	Test Purpose and Environment	3850

A.9.4.1.2	Parameters	3850

A.9.4.1.3	Test Requirements	3852

A.9.4.2	E-UTRAN TDD	3853

A.9.4.2.1	Test Purpose and Environment	3853

A.9.4.2.2	Parameters	3853

A.9.4.2.3	Test Requirements	3855

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	3856

A.9.4.3.1	Test Purpose and Environment	3856

A.9.4.3.2	Parameters	3856

A.9.4.3.3	Test Requirements	3857

A.9.5	UTRAN TDD measurement	3857

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	3857

A.9.5.1.1	Test Purpose and Environment	3857

A.9.5.1.2	Test parameters	3857

A.9.5.1.3	Test Requirements	3859

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	3859

A.9.5.2.1	Test Purpose and Environment	3859

A.9.5.2.2	Test parameters	3859

A.9.5.2.3	Test Requirements	3861

A.9.6	GSM Carrier RSSI	3862

A.9.6.1	E-UTRAN FDD	3862

A.9.6.1.1	Test Purpose and Environment	3862

A.9.6.1.2	Test Requirements	3863

A.9.6.2	E-UTRAN TDD	3863

A.9.6.2.1	Test Purpose and Environment	3863

A.9.6.2.2	Test Requirements	3865

A.9.7	UE Rx – Tx Time Difference	3866

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	3866

A.9.7.1.1	Test Purpose and Environment	3866

A.9.7.1.2	Test parameters	3866

A.9.7.1.3	Test Requirements	3867

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	3867

A.9.7.2.1	Test Purpose and Environment	3867

A.9.7.2.2	Test parameters	3867

A.9.7.2.3	Test Requirements	3869

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3869

A.9.7.3.1	Test Purpose and Environment	3869

A.9.7.3.2	Test parameters	3869

A.9.7.3.3	Test Requirements	3872

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3872

A.9.7.4.1	Test Purpose and Environment	3872

A.9.7.4.2	Test Parameters	3872

A.9.7.4.3	Test Requirements	3875

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3875

A.9.7.5.1	Test Purpose and Environment	3875

A.9.7.5.2	Test parameters	3875

A.9.7.5.3	Test Requirements	3878

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3878

A.9.7.6.1	Test Purpose and Environment	3878

A.9.7.6.2	Test Parameters	3878

A.9.7.6.3	Test Requirements	3881

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3881

A.9.7.7.1	Test Purpose and Environment	3881

A.9.7.7.2	Test parameters	3881

A.9.7.7.3	Test Requirements	3882

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3883

A.9.7.8.1	Test Purpose and Environment	3883

A.9.7.8.2	Test parameters	3883

A.9.7.8.3	Test Requirements	3884

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3884

A.9.7.9.1	Test Purpose and Environment	3884

A.9.7.9.2	Test parameters	3884

A.9.7.9.3	Test Requirements	3885

A.9.8	RSTD	3886

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	3886

A.9.8.1.1	Test Purpose and Environment	3886

A.9.8.1.2	Test Requirements	3888

A.9.8.1.2A	Test Requirements for UE Category 1bis	3888

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	3889

A.9.8.2.1	Test Purpose and Environment	3889

A.9.8.2.2	Test Requirements	3892

A.9.8.2.2A	Test Requirements for UE Category 1bis	3892

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	3893

A.9.8.3.1	Test Purpose and Environment	3893

A.9.8.3.2	Test Requirements	3895

A.9.8.3.2A	Test Requirements for UE Category 1bis	3895

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	3896

A.9.8.4.1	Test Purpose and Environment	3896

A.9.8.4.2	Test Requirements	3898

A.9.8.4.2A	Test Requirements for UE Category 1bis	3898

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	3899

A.9.8.5.1	Test Purpose and Environment	3899

A.9.8.5.2	Test Requirements	3901

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	3901

A.9.8.6.1	Test Purpose and Environment	3901

A.9.8.6.2	Test Requirements	3904

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	3904

A.9.8.7.1	Test Purpose and Environment	3904

A.9.8.7.2	Test Requirements	3905

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	3905

A.9.8.8.1	Test Purpose and Environment	3905

A.9.8.8.2	Test Requirements	3906

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	3906

A.9.8.9.1	Test Purpose and Environment	3906

A.9.8.9.2	Test Requirements	3907

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	3907

A.9.8.10.1	Test Purpose and Environment	3907

A.9.8.10.2	Test Requirements	3908

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	3908

A.9.8.11.1	Test Purpose and Environment	3908

A.9.8.11.2	Test Requirements	3909

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	3909

A.9.8.12.1	Test Purpose and Environment	3909

A.9.8.12.2	Test Requirements	3910

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	3910

A.9.8.13.1	Test Purpose and Environment	3910

A.9.8.13.2	Test Requirements	3911

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	3911

A.9.8.14.1	Test Purpose and Environment	3911

A.9.8.14.2	Test Requirements	3918

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	3918

A.9.8.15.1	Test Purpose and Environment	3918

A.9.8.15.2	Test Requirements	3924

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	3924

A.9.8.16.1	Test Purpose and Environment	3924

A.9.8.16.2	Test Requirements	3927

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	3927

A.9.8.17.1	Test Purpose and Environment	3927

A.9.8.17.2	Test Requirements	3930

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	3930

A.9.8.18.1	Test Purpose and Environment	3930

A.9.8.18.2	Test Requirements	3933

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	3933

A.9.8.19.1	Test Purpose and Environment	3933

A.9.8.19.2	Test Requirements	3936

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	3936

A.9.8.20.1	Test Purpose and Environment	3936

A.9.8.20.2	Test Requirements	3939

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	3940

A.9.8.21.1	Test Purpose and Environment	3940

A.9.8.21.2	Test Requirements	3943

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	3944

A.9.8.22.1	Test Purpose and Environment	3944

A.9.8.22.2	Test Requirements	3947

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	3948

A.9.8.23.1	Test Purpose and Environment	3948

A.9.8.23.2	Test Requirements	3951

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	3951

A.9.8.24.1	Test Purpose and Environment	3951

A.9.8.24.2	Test Requirements	3955

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	3956

A.9.8.25.1	Test Purpose and Environment	3956

A.9.8.25.2	Test Requirements	3959

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3960

A.9.8.26.1	Test Purpose and Environment	3960

A.9.8.26.2	Test Requirements	3962

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3962

A.9.8.27.1	Test Purpose and Environment	3962

A.9.8.27.2	Test Requirements	3965

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	3965

A.9.8.28.1	Test Purpose and Environment	3965

A.9.8.28.2	Test Requirements	3968

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3968

A.9.8.29.1	Test Purpose and Environment	3968

A.9.8.29.2	Test Requirements	3972

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3973

A.9.8.30.1	Test Purpose and Environment	3973

A.9.8.30.2	Test Requirements	3976

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	3977

A.9.8.31.1	Test Purpose and Environment	3977

A.9.8.31.2	Test Requirements	3981

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	3981

A.9.8.32.1	Test Purpose and Environment	3981

A.9.8.32.2	Test Requirements	3984

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	3984

A.9.8.33.1	Test Purpose and Environment	3984

A.9.8.33.2	Test Requirements	3987

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	3987

A.9.8.34.1	Test Purpose and Environment	3987

A.9.8.34.2	Test Requirements	3990

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	3990

A.9.8.35.1	Test Purpose and Environment	3990

A.9.8.35.2	Test Requirements	3993

A.9.9	RSRP and RSRQ on the serving cell	3993

A.9.9.1	FDD Intra frequency serving cell case	3993

A.9.9.1.1	Test Purpose and Environment	3993

A.9.9.1.2	Test parameters	3993

A.9.9.1.3	Test Requirements	3996

A.9.9.2	TDD Intra frequency serving cell case	3996

A.9.9.2.1	Test Purpose and Environment	3996

A.9.9.2.2	Test parameters	3996

A.9.9.2.3	Test Requirements	3998

A.9.10	SSTD	3998

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	3998

A.9.10.1.1	Test Purpose and Environment	3998

A.9.10.1.2	Test parameters	3998

A.9.10.1.3	Test Requirements	4000

A.9.10.2	Void	4000

A.9.10.3	Void	4000

A.9.10.4	Void	4000

A.9.11 RSSI	4000

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	4000

A.9.11.1.1	Test Purpose and Environment	4000

A.9.11.1.2	Test parameters	4000

A.9.11.1.3	Test Requirements	4003

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	4003

A.9.11.2.1	Test Purpose and Environment	4003

A.9.11.2.2	Test parameters	4003

A.9.12	Channel occupancy	4006

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	4006

A.9.12.1.1	Test Purpose and Environment	4006

A.9.12.1.2	Test parameters	4006

A.9.12.1.3	Test Requirements	4009

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	4009

A.9.12.2.1	Test Purpose and Environment	4009

A.9.12.2.2	Test parameters	4009

A.9.12.2.3	Test Requirements	4012

A.9.13	RS-SINR	4012

A.9.13.1	FDD Intra-Frequency Case	4012

A.9.13.1.1	Test Purpose and Environment	4012

A.9.13.1.2	Test parameters	4012

A.9.13.1.3	Test Requirements	4015

A.9.13.2	TDD Intra-Frequency Case	4015

A.9.13.2.1	Test Purpose and Environment	4015

A.9.13.2.2	Test parameters	4015

A.9.13.2.3	Test Requirements	4018

A.9.13.3	FDD—FDD Inter frequency case	4018

A.9.13.3.1	Test Purpose and Environment	4018

A.9.13.3.2	Test parameters	4018

A.9.13.3.3	Test Requirements	4021

A.9.13.4	TDD—TDD Inter frequency case	4021

A.9.13.4.1	Test Purpose and Environment	4021

A.9.13.4.2	Test parameters	4021

A.9.13.4.3	Test Requirements	4025

A.9.13.5	FDD—TDD Inter frequency case	4026

A.9.13.5.1	Test Purpose and Environment	4026

A.9.13.5.2	Test parameters	4026

A.9.13.5.3	Test Requirements	4032

A.9.13.6	TDD—FDD Inter frequency case	4032

A.9.13.6.1	Test Purpose and Environment	4032

A.9.13.6.2	Test parameters	4032

A.9.13.6.3	Test Requirements	4036

A.9.14	Channel quality reporting accuracy	4036

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	4036

A.9.14.1.1	Test Purpose and Environment	4036

A.9.14.1.2	Test parameters	4037

A.9.14.1.3	Test Requirements	4037

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	4038

A.9.14.2.1	Test Purpose and Environment	4038

A.9.14.2.2	Test parameters	4038

A.9.14.2.3	Test Requirements	4039

A.9.14.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	4039

A.9.14.3.1	Test Purpose and Environment	4039

A.9.14.3.2	Test parameters	4039

A.9.14.3.3	Test Requirements	4040

A.9.14.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	4040

A.9.14.4.1	Test Purpose and Environment	4040

A.9.14.4.2	Test parameters	4040

A.9.14.4.3	Test Requirements	4041

A.10	Proximity-based Services in Any Cell Selection State	4064

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	4064

A.10.1.1	Test Purpose and Environment	4064

A.10.1.2	Test Requirements	4065

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	4066

A.10.2.1	Test Purpose and Environment	4066

A.10.2.2	Test Requirements	4067

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	4067

A.10.3.1	Test Purpose and Environment	4067

A.10.3.2	Test Requirements	4069

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	4070

A.10.4.1	Test Purpose and Environment	4070

A.10.4.2	Test Requirements	4072

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	4072

A.11.1	V2V UE Transmission Timing Accuracy Test	4072

A.11.1.1	Test Purpose and Environment	4072

A.11.1.2	Test requirements	4073

A.11.2	Interruptions due to V2V sidelink communication	4073

A.11.2.1	Test Purpose and Environment	4073

A.11.2.2	Test Requirements	4075

A.12	4075

A.12.1	V2X UE Transmission Timing Accuracy Test	4075

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	4075

A.12.1.1.1	Test Purpose and Environment	4075

A.12.1.1.2	Test requirements	4076

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	4077

A.12.1.2.1	Test Purpose and Environment	4077

A.12.1.2.2	Test Requirements	4077

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	4078

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	4078

A.12.2.1.1	Test Purpose and Environment	4078

A.12.2.1.2	Test Requirements	4079

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	4080

A.12.2.2.1	Test Purpose and Environment	4080

A.12.2.2.2	Test Requirements	4081

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	4081

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	4081

A.12.3.1.1	Test Purpose and Environment	4081

A.12.3.1.2	Test Requirements	4084

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	4085

A.12.3.2.1	Test Purpose and Environment	4085

A.12.3.1.2	Test Requirements	4087

A.12.4	Congestion Control Measurement Test for V2X UE	4087

A.12.4.1	Test Purpose and Environment	4087

A.12.4.2	Test Requirements	4089

A.12.5	Interruptions due to V2X Sidelink Communication	4089

A.12.5.1	Test Purpose and Environment	4089

A.12.5.2	Test Requirements	4091

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	4091

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	4091

A.12.6.1.1	Test Purpose and Environment	4091

A.12.6.1.2	Test Requirements	4093

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	4093

A.12.6.2.1	Test Purpose and Environment	4093

A.12.6.1.2	Test Requirements	4095

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	4095

A.12.7.1	Test Purpose and Environment	4095

A.12.7.2	Test Requirements	4098

A.12.8	Interruptions due to V2X Carrier Aggregation	4099

A.12.8.1	Interruptions on a FDD PCell	4099

A.12.8.1.1	Test Purpose and Environment	4099

A.12.8.1.2	Test Requirements	4100

A.12.8.2	Interruptions on a TDD PCell	4100

A.12.8.2.1	Test Purpose and Environment	4100

A.12.8.2.2	Test Requirements	4102

A.13	E-UTRAN Standalone Tests for UE Category NB for Satellite Access	4103

A.13.1	RRC_IDLE state for satellite access	4103

A.13.1.1	Cell re-selection for satellite access	4103

A.13.1.1.1	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage	4103

A.13.1.1.1.1	Test Purpose and Environment	4103

A.13.1.1.1.2	Test Requirements	4105

A.13.1.1.2	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation	4106

A.13.1.1.2.1	Test Purpose and Environment	4106

A.13.1.1.3	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX	4109

A.13.1.1.3.1	Test Purpose and Environment	4109

A.13.1.1.3.2	Test Requirements	4111

A.13.1.1.4	HD – FDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage	4112

A.13.1.1.4.1	Test Purpose and Environment	4112

A.13.1.1.4.2	Test Requirements	4113

A.13.1.1.5	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in enhanced coverage, location-based cell reselection for NGSO	4114

A.13.1.1.5.1	Test Purpose and Environment	4114

A.13.1.1.5.2	Test Requirements	4115

A.13.1.1.6	HD – FDD Inter frequency case for UE Category NB1 Standalone mode in enhanced coverage, time-based cell reselection for NGSO	4116

A.13.1.1.6.1	Test Purpose and Environment	4116

A.13.1.1.6.2	Test Requirements	4117

A.13.2	Void	4118

A.13.3	RRC connection mobility control for satellite access	4118

A.13.3.1	RRC re-establishment for satellite access	4118

A.13.3.1.1	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage	4118

A.13.3.1.1.1	Test Purpose and Environment	4118

A.13.3.1.1.2	Test Requirements	4119

A.13.3.1.2	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage	4120

A.13.3.1.2.1	Test Purpose and Environment	4120

A.13.3.1.2.2	Test Requirements	4121

A.13.3.1.3	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage	4122

A.13.3.1.3.1	Test Purpose and Environment	4122

A.13.3.1.3.2	Test Requirements	4123

A.13.3.2	Random Access for Satellite Access	4123

A.13.3.2.1	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage	4124

A.13.3.2.1.1	Test Purpose and Environment	4124

A.13.3.2.1.2	Test Requirements	4126

A.13.3.2.2	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in Enhanced Coverage	4128

A.13.3.2.2.1	Test Purpose and Environment	4128

A.13.3.2.2.2	Test Requirements	4129

A.13.3.2.3	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage	4131

A.13.3.2.3.1	Test Purpose and Environment	4131

A.13.3.2.3.2	Test Requirements	4133

A.13.4	Timing and signalling characteristics for satellite access	4135

A.13.4.1	UE transmit timing for satellite access	4135

A.13.4.1.1	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access	4135

A.13.4.1.1.1	Test Purpose and Environment	4135

A.13.4.1.1.2	Test Requirements	4136

A.13.4.1.2	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access	4137

A.13.4.1.2.1	Test Purpose and Environment	4137

A.13.4.1.2.2	Test Requirements	4138

A.13.4.1.3	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access	4139

A.13.4.1.3.1	Test Purpose and Environment	4139

A.13.4.1.3.2	Test Requirements	4140

A.13.4.2	UE timing advance for satellite access	4141

A.13.4.2.1	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access	4141

A.13.4.2.1.1	Test Purpose and Environment	4141

A.13.4.2.1.2	Test Requirements	4143

A.13.4.2.2	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access	4143

A.13.4.2.2.1	Test Purpose and Environment	4143

A.13.4.2.2.2	Test Requirements	4145

A.13.4.3	Radio Link Monitoring for satellite access	4145

A.13.4.3.1	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage	4145

A.13.4.3.1.1	Test Purpose and Environment	4145

A.13.4.3.1.2	Test Requirements	4148

A.13.4.3.2	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage	4148

A.13.4.3.2.1	Test Purpose and Environment	4148

A.13.4.3.2.2	Test Requirements	4151

A.13.4.3.3	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Enhanced Coverage	4151

A.13.4.3.3.1	Test Purpose and Environment	4151

A.13.4.3.3.2	Test Requirements	4154

A.13.4.3.4	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Normal Coverage	4154

A.13.4.3.4.1	Test Purpose and Environment	4154

A.13.4.3.4.2	Test Requirements	4157

A.13.4.3.5	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	4157

A.13.4.3.5.1	Test Purpose and Environment	4157

A.13.4.3.5.2	Test Requirements	4160

A.13.4.3.6	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage	4160

A.13.4.3.6.1	Test Purpose and Environment	4160

A.13.4.3.6.2	Test Requirements	4163

A.13.4.3.7	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	4163

A.13.4.3.7.1	Test Purpose and Environment	4163

A.13.4.3.7.2	Test Requirements	4165

A.13.4.3.8	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage	4166

A.13.4.3.8.1	Test Purpose and Environment	4166

A.13.4.3.8.2	Test Requirements	4168

A.13.5	UE measurement procedures in RRC_CONNECTED state for UE category NB1 for satellite access	4169

A.13.6	Measurement performance requirements for UE for satellite access	4176

A.13.6.1	Void	4176

A.13.6.2	Channel quality reporting accuracy for satellite access	4176

A.13.6.2.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	4176

A.13.6.2.1.1	Test Purpose and Environment	4176

A.13.6.2.1.2	Test parameters	4176

A.13.6.2.1.3	Test Requirements	4177

A.13.6.2.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	4177

A.13.6.2.2.1	Test Purpose and Environment	4177

A.13.6.2.2.2	Test parameters	4177

A.13.6.2.2.3	Test Requirements	4178

A.13.6.2.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	4178

A.13.6.2.3.1	Test Purpose and Environment	4178

A.13.6.2.3.2	Test parameters	4179

A.13.6.2.3.3	Test Requirements	4179

A.13.6.2.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	4180

A.13.6.2.4.1	Test Purpose and Environment	4180

A.13.6.2.4.2	Test parameters	4180

A.13.6.2.4.3	Test Requirements	4181

A.13.6.2.5	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage	4181

A.13.6.2.5.1	Test Purpose and Environment	4181

A.13.6.2.5.2	Test parameters	4181

A.13.6.2.5.3	Test Requirements	4182

A.13.6.2.6	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage	4183

A.13.6.2.6.1	Test Purpose and Environment	4183

A.13.6.2.6.2	Test parameters	4183

A.13.6.2.6.3	Test Requirements	4184

A.14	E-UTRAN Standalone Tests for UE Category M1 for Satellite Access	4184

A.14.1	RRC_IDLE state for satellite access	4184

A.14.1.1	Cell re-selection for satellite access	4184

A.14.1.1.1	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	4184

A.14.1.1.1.1	Test Purpose and Environment	4184

A.14.1.1.1.2	Test Requirements	4186

A.14.1.1.2	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	4187

A.14.1.1.2.1	Test Purpose and Environment	4187

A.14.1.1.2.2	Test Requirements	4189

A.14.1.1.3	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	4190

A.14.1.1.3.1	Test Purpose and Environment	4190

A.14.1.1.3.2	Test Requirements	4192

A.14.1.1.4	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	4193

A.14.1.1.4.1	Test Purpose and Environment	4193

A.14.1.1.4.2	Test Requirements	4195

A.14.1.1.5	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	4196

A.14.1.1.5.1	Test Purpose and Environment	4196

A.14.1.1.5.2	Test Requirements	4198

A.14.1.1.6	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	4199

A.14.1.1.6.1	Test Purpose and Environment	4199

A.14.1.1.6.2	Test Requirements	4201

A.14.1.1.7	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage, time-based triggering	4202

A.14.1.1.7.1	Test Purpose and Environment	4202

A.14.1.1.7.2	Test Requirements	4203

A.14.1.1.8 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage, time-based triggering	4204

A.14.1.1.8.1	Test Purpose and Environment	4204

A.14.1.1.8.2	Test Requirements	4205

A.14.1.1.9	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage, location-based triggering	4206

A.14.1.1.9.1	Test Purpose and Environment	4206

A.14.1.1.9.2	Test Requirements	4207

A.14.1.1.10	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage, location-based triggering	4208

A.14.1.1.10.1	Test Purpose and Environment	4208

A.14.1.1.10.2	Test Requirements	4209

A.14.2	RRC_CONNECTED state mobility for satellite access	4210

A.14.2.1	E-UTRAN handover for satellite access	4210

A.14.2.1.1	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	4210

A.14.2.1.1.1	Test Purpose and Environment	4210

A.14.2.1.1.2	Test Requirements	4212

A.14.2.1.2	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	4213

A.14.2.1.2.1	Test Purpose and Environment	4213

A.14.2.1.2.2	Test Requirements	4214

A.14.2.1.3	E-UTRAN FDD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA	4215

A.14.2.1.3.1	Test Purpose and Environment	4215

A.14.2.1.3.2	Test Requirements	4216

A.14.2.1.4	E-UTRAN HD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA	4217

A.14.2.1.4.1	Test Purpose and Environment	4217

A.14.2.1.4.2	Test Requirements	4218

A.14.2.1.5	E-UTRAN FDD Intra frequency handover for Cat-M1 UEs in CEModeA	4219

A.14.2.1.5.1	Test Purpose and Environment	4219

A.14.2.1.5.2	Test Requirements	4220

14.2.1.6	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	4221

A.14.2.1.6.1	Test Purpose and Environment	4221

A.14.2.1.6.2	Test Requirements	4222

A.14.2.1.7	E-UTRAN FD-FDD Inter frequency handover for Cat-M1 UEs in CEModeA	4223

A.14.2.1.7.1	Test Purpose and Environment	4223

A.14.2.1.7.2	Test Requirements	4224

A.14.2.1.8	E-UTRAN HD-FDD Inter frequency handover for Cat-M1 UEs in CEModeA	4225

A.14.2.1.8.1	Test Purpose and Environment	4225

A.14.2.1.8.2	Test Requirements	4226

A.14.2.1.9	E-UTRAN FDD Inter frequency handover for Cat-M1 UEs in CEModeB	4227

A.14.2.1.9.1	Test Purpose and Environment	4227

A.14.2.1.9.2	Test Requirements	4228

A.14.2.1.10	E-UTRAN HD-FDD Inter frequency handover for Cat-M1 UEs in CEModeB	4229

A.14.2.1.10.1	Test Purpose and Environment	4229

A.14.2.1.10.2	Test Requirements	4230

A.14.2.1.11	E-UTRAN FDD-FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA	4231

A.14.2.1.11.1	Test Purpose and Environment	4231

A.14.2.1.12.2	Test Requirements	4232

A.14.2.1.12	E-UTRAN HD-FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA	4233

A.14.2.1.12.1	Test Purpose and Environment	4233

A.14.2.1.12.2	Test Requirements	4234

A.14.2.1.15	E-UTRAN FDD-FDD Inter frequency location based conditional handover for Cat-M1 UEs in CEModeA	4239

A.14.2.1.15.1	Test Purpose and Environment	4239

A.14.2.1.16.2	Test Requirements	4240

A.14.2.1.16	E-UTRAN HD-FDD Inter frequency time based conditional handover for Cat-M1 UEs in CEModeA	4241

A.14.2.1.16.1	Test Purpose and Environment	4241

A.14.2.1.16.2	Test Requirements	4242

A.14.3	RRC connection mobility control for satellite access	4243

A.14.3.1	RRC re-establishment for satellite access	4243

A.14.3.1.1	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access	4243

A.14.3.1.1.1	Test Purpose and Environment	4243

A.14.3.1.1.2	Test Requirements	4244

A.14.3.1.2	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	4245

A.14.3.1.2.1	Test Purpose and Environment	4245

A.14.3.1.2.2	Test Requirements	4247

A.14.3.1.3	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access	4248

A.14.3.1.3.1	Test Purpose and Environment	4248

A.14.3.1.3.2	Test Requirements	4250

A.14.3.1.4	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access	4251

A.14.3.1.4.1	Test Purpose and Environment	4251

A.14.3.1.4.2	 Test Requirements	4253

A.14.3.2	Random access for satellite access	4254

A.14.3.2.1	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage for satellite access	4254

A.14.3.2.1.1	Test Purpose and Environment	4254

A.14.3.2.1.2	Test Requirements	4256

A.14.3.2.2	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage for satellite access	4257

A.14.3.2.2.1	Test Purpose and Environment	4257

A.14.3.2.2.2	Test Requirements	4259

A.14.3.2.3	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage for satellite access	4261

A.14.3.2.3.1	Test Purpose and Environment	4261

A.14.3.2.3.2	Test Requirements	4263

A.14.3.2.4	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage for satellite access	4264

A.14.3.2.4.1	Test Purpose and Environment	4264

A.14.3.2.4.2	Test Requirements	4266

A.14.4	Timing and signalling characteristics for satellite access	4268

A.14.4.1	UE transmit timing for satellite access	4268

A.14.4.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	4268

A.14.4.1.1.1	Test Purpose and Environment	4268

A.14.4.1.1.2	Test Requirements	4270

A.14.4.1.2	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	4271

A.14.4.1.2.1	Test Purpose and Environment	4271

A.14.4.1.2.2	Test Requirements	4272

A.14.4.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	4273

A.14.4.1.3.1	Test Purpose and Environment	4273

A.14.4.1.3.2	Test Requirements	4274

A.14.4.1.4	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	4275

A.14.4.1.4.1	Test Purpose and Environment	4275

A.14.4.1.4.2	Test Requirements	4276

A.14.4.1.5	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB with segment transmission in NGSO for Satellite Access	4277

A.14.4.1.5.1	Test Purpose and Environment	4277

A.14.4.1.5.2	Test Requirements	4278

A.14.4.2	UE timing advance for satellite access	4279

A.14.4.2.1	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	4279

A.14.4.2.1.1	Test Purpose and Environment	4279

A.14.4.2.1.2	Test Requirements	4281

A.14.4.2.2	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	4281

A.14.4.2.2.1	Test Purpose and Environment	4281

A.14.4.2.2.2	Test Requirements	4284

A.14.4.2.3	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	4284

A.14.4.2.3.1	Test Purpose and Environment	4284

A.14.4.2.3.2	Test Requirements	4285

A.14.4.2.4	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	4285

A.14.4.2.4.1	Test Purpose and Environment	4285

A.14.4.2.4.2	Test Requirements	4287

A.14.4.3	Radio Link Monitoring for satellite access	4288

A.14.4.3.1	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for Satellite access	4288

A.14.4.3.1.1	Test Purpose and Environment	4288

A.14.4.3.1.2	Test Requirements	4291

A.14.4.3.2	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A for Satellite access	4291

A.14.4.3.2.1	Test Purpose and Environment	4291

A.14.4.3.2.2	Test Requirements	4294

A.14.4.3.3	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for Satellite access	4294

A.14.4.3.3.1	Test Purpose and Environment	4294

A.14.4.3.3.2	Test Requirements	4297

A.14.4.3.4	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A for Satellite access	4297

A.14.4.3.4.1	Test Purpose and Environment	4297

A.14.4.3.4.2	Test Requirements	4300

A.14.4.3.5	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	4300

A.14.4.3.5.1	Test Purpose and Environment	4300

A.14.4.3.5.2	 Test Requirements	4303

A.14.4.3.6	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	4303

A.14.4.3.6.1	Test Purpose and Environment	4303

A.14.4.3.6.2	Test Requirements	4306

A.14.4.3.7	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	4306

A.14.4.3.7.1	Test Purpose and Environment	4306

A.14.4.3.8	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	4309

A.14.4.3.8.1 	Test Purpose and Environment	4309

A.14.4.3.8.2	Test Requirements	4312

A.14.5	UE measurement procedures in RRC_CONNECTED state for satellite access	4312

A.14.5.1 Intra-frequency measurements for satellite access	4312

A.14.5.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under AWGN conditions in asynchronous cells for Cat-M1 UE in CEModeA	4312

A.14.5.1.1.1	Test Purpose and Environment	4312

A.14.5.1.1.2	Test Requirements	4314

A.14.5.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under AWGN conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	4315

A.14.5.1.2.1	Test Purpose and Environment	4315

A.14.5.1.2.2	Test Requirements	4317

A.14.5.1.3	E-UTRAN HD-FDD intra-frequency event triggered reporting under AWGN conditions in asynchronous cells for Cat-M1 UE in CEModeA	4317

A.14.5.1.3.1	Test Purpose and Environment	4317

A.14.5.1.3.2	Test Requirements	4319

A.14.5.1.4	E-UTRAN HD-FDD intra-frequency event triggered reporting under AWGN conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	4320

A.14.5.1.4.1	Test Purpose and Environment	4320

A.14.5.1.4.2	Test Requirements	4322

A.14.5.1.5	E-UTRAN FD-FDD Intra-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA with location-based triggering	4322

A.14.5.1.5.1	Test Purpose and Environment	4322

A.14.5.1.5.2	Test Requirements	4324

A.14.5.1.6 	E-UTRAN HD-FDD Intra-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA with location-based triggering	4325

A.14.5.1.6.1	Test Purpose and Environment	4325

A.14.5.1.6.2	Test Requirements	4326

A.14.5.1.7	E-UTRAN HD-FDD Intra-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA when DRX is used with time-based triggering	4327

A.14.5.1.7.1	Test Purpose and Environment	4327

A.14.5.1.7.2	Test Requirements	4328

A.14.5.2 Inter-frequency measurements for satellite access	4329

A.14.5.2.1	E-UTRAN FD-FDD Inter-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA when DRX is used with time-based triggering	4329

A.14.5.2.1.1	Test Purpose and Environment	4329

A.14.5.2.1.2	Test Requirements	4330

A.14.5.2.2	E-UTRAN HD-FDD Inter-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA when DRX is used with time-based triggering	4331

A.14.5.2.2.1	Test Purpose and Environment	4331

A.14.5.2.2.2	Test Requirements	4332

A.14.5.2.3	E-UTRAN HD-FDD Inter-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeB when DRX is used with time-based triggering	4333

A.14.5.2.3.1	Test Purpose and Environment	4333

A.14.5.2.3.2	Test Requirements	4334

A.14.5.2.4	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	4335

A.14.5.2.4.1	Test Purpose and Environment	4335

A.14.5.2.4.2	Test Requirement	4336

A.14.5.2.5	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	4337

A.14.5.2.5.1	Test Purpose and Environment	4337

A.14.5.2.5.2	Test Requirement	4339

A.14.5.2.6	E-UTRAN HD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	4339

A.14.5.2.6.1	Test Purpose and Environment	4339

A.14.5.2.6.2	Test Requirement	4341

A.14.5.2.7	E-UTRAN HD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 in CEModeA in DRX	4341

A.14.5.2.7.1	Test Purpose and Environment	4341

A.14.5.2.7.2	Test Requirement	4343

A.14.5.2.8	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	4343

A.14.5.2.8.1	Test Purpose and Environment	4343

A.14.5.2.8.2	Test Requirement	4345

A.14.5.2.9	E-UTRAN HD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	4346

A.14.5.2.9.1	Test Purpose and Environment	4346

A.14.5.2.9.2	Test Requirement	4347

A.14.6	Measurement performance requirements for UE for satellite access	4348

A.14.6.1	RSRP for satellite access	4348

A.14.6.1.1	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	4348

A.14.6.1.1.1	Test Purpose and Environment	4348

A.14.6.1.1.2	Test parameters	4348

A.14.6.1.1.3	Test Requirements	4350

A.14.6.1.2	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	4350

A.14.6.1.2.1	Test Purpose and Environment	4350

A.14.6.1.2.2	Test parameters	4350

A.14.6.1.2.3	Test Requirements	4352

A.14.6.1.3	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	4352

A.14.6.1.3.1	Test Purpose and Environment	4352

A.14.6.1.3.2	Test parameters	4352

A.14.6.1.3.3	Test Requirements	4353

A.14.6.1.4	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	4354

A.14.6.1.4.1	Test Purpose and Environment	4354

A.14.6.1.4.2	Test parameters	4354

A.14.6.1.4.3	Test Requirements	4355

A.14.6.1.5	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	4355

A.14.6.1.5.1	Test Purpose and Environment	4355

A.14.6.1.5.2	Test parameters	4355

A.14.6.1.5.3	Test Requirements	4356

A.14.6.1.6	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	4356

A.14.6.1.6.1	Test Purpose and Environment	4356

A.14.6.1.6.2	Test parameters	4357

A.14.6.1.6.3	Test Requirements	4358

A.14.6.2	Channel quality reporting accuracy for satellite access	4358

A.14.6.2.1	E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A for Satellite access	4358

A.14.6.2.1.1	Test Purpose and Environment	4358

A.14.6.2.1.2	Test parameters	4358

A.14.6.2.1.3	Test Requirements	4359

A.14.6.2.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A for Satellite access	4360

A.14.6.2.2.1	Test Purpose and Environment	4360

A.14.6.2.2.2	Test parameters	4360

A.14.6.2.2.3	Test Requirements	4361

A.14.6.2.3.2	Test parameters	4361

A.14.6.2.3.3	Test Requirements	4362

A.14.6.2.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B for Satellite access	4363

A.14.6.2.4.1	Test Purpose and Environment	4363

A.14.6.2.4.2	Test parameters	4363

A.14.6.2.4.3	Test Requirements	4364

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	4365

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	4365

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	4365

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	4365

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	4365

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	4367

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	4368

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	4368

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	4368

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	4368

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	4370

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	4371

B.1.9	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1 for satellite access	4371

B.1.10	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1 and NB2 for satellite access	4372

B.1.11	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1 for satellite access	4373

B.1.12	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1 for satellite access	4373

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	4373

B.2.1	Conditions for E-UTRAN intra-frequency measurements	4373

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	4374

B.2.3	Conditions for E-UTRAN inter-frequency measurements	4374

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	4375

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	4375

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	4375

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	4376

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	4376

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4377

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	4377

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	4377

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	4377

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	4378

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	4378

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	4378

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	4379

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	4379

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	4379

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	4380

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	4380

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	4381

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	4382

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	4383

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	4384

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	4385

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	4385

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	4385

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	4385

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	4386

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	4386

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	4388

B.2.24	Conditions for intra-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	4388

B.2.25	Conditions for inter-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	4388

B.2.26	Conditions for E-UTRAN intra-frequency measurements by UE Category M1 for satellite access	4388

B.2.27	Conditions for NB-IoT intra-frequency measurements by UE Category NB1 and NB2 for satellite access	4389

B.2.28	Conditions for E-UTRAN inter-frequency measurements by UE Category M1 for satellite access	4390

B.2.29	Conditions for intra-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1 for satellite access	4390

B.2.30	Conditions for inter-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1 for satellite access	4391

B.3	Conditions for measurements performance requirements for UE	4391

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	4391

B.3.2	Void	4391

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	4392

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	4392

B.3.5	Conditions for UE Rx – Tx time difference	4392

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	4392

B.3.7	Conditions for inter-frequency RSTD measurements	4392

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	4392

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	4393

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	4393

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4393

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4393

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	4393

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	4393

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	4393

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	4394

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	4394

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	4394

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	4394

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	4395

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	4395

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	4395

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	4395

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	4395

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	4395

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	4396

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	4396

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	4396

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	4396

B.3.21.1	Conditions for RSRP measurements	4396

B.3.21.2	Conditions for RSRQ measurements	4396

B.3.21.3	Conditions for CSI-RSRP measurements	4396

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	4397

B.3.22.1	Conditions for RSRP measurements	4397

B.3.22.2	Void	4397

B.3.22.3	Conditions for CSI-RSRP measurements	4397

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	4397

B.3.23.1	Conditions for RSRP measurements	4397

B.3.23.2	Conditions for RSRQ measurements	4398

B.3.23.3	Conditions for CSI-RSRP measurements	4398

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	4398

B.3.24.1	Conditions for RSRP measurements	4398

B.3.24.2	Conditions for RSRQ measurements	4398

B.3.24.3	Conditions for CSI-RSRP measurements	4398

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	4398

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	4399

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	4399

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	4399

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	4400

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	4400

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	4400

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	4400

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	4400

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	4400

B.4	RRM Requirements Exceptions	4401

B.4.1	General	4401

B.4.2	Receiver sensitivity relaxation for UE supporting CA	4401

B.4.3	Receiver sensitivity relaxation for UE configured with CA	4401

B.4.3.1	Inter-band carrier aggregation	4401

B.4.3.2	Intra-band non-contiguous carrier aggregation	4401

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	4401

B.5	Conditions for Measurement Performance Requirements for ProSe UE	4402

B.5.1	Conditions for S-RSRP Accuracy Requirements	4402

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	4402

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	4402

B.5.4	Conditions for SD-RSRP Accuracy Requirements	4403

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	4403

B.6	Conditions for V2X	4404

B.6.1	Test parameters for GNSS signals	4404

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	4404

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	4404

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	4404

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	4405

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	4405

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	4405

B.8	High level test procedure for SAN RRM tests	4407

Annex C (informative):	Change history:	4407