| 3GPP TS 38.101-1 V18.11.0 (2025-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	24

1	Scope	26

2	References	26

3	Definitions, symbols and abbreviations	27

3.1	Definitions	27

3.2	Symbols	28

3.3	Abbreviations	31

4	General	32

4.1	Relationship between minimum requirements and test requirements	32

4.2	Applicability of minimum requirements	32

4.3	Specification suffix information	33

5	Operating bands and channel arrangement	34

5.1	General	34

5.2	Operating bands	34

5.2A	Operating bands for CA	36

5.2A.0	General	36

5.2A.1	Intra-band CA	36

5.2A.2	Inter-band CA	37

5.2A.2.1	Inter-band CA (two bands)	38

5.2A.2.2	Inter-band CA (three bands)	42

5.2A.2.3	Inter-band CA (four bands)	48

5.2A.2.4	Inter-band CA (five bands)	51

5.2A.2.5	Inter-band CA (six bands)	51

5.2B	Operating bands for DC	51

5.2C	Operating band combination for SUL	52

5.2D	Operating bands for UL MIMO	54

5.2E	Operating band for V2X	55

5.2E.1	V2X operating bands	55

5.2E.1A	Sidelink CA operating bands	55

5.2E.1F	Operating bands for Sidelink Unlicensed	55

5.2E.2	V2X operating bands for concurrent operation	56

5.2E.2F	Operating bands for SL-U concurrent operation	56

5.2J	Operating band for ATG	57

5.2K	Operating bands for Aerial UE	57

5.3	UE channel bandwidth	57

5.3.1	General	57

5.3.2	Maximum transmission bandwidth configuration	58

5.3.3	Minimum guardband and transmission bandwidth configuration	58

5.3.4	RB alignment	60

5.3.5	UE channel bandwidth per operating band	60

5.3.6	Asymmetric channel bandwidths	67

5.3A	UE channel bandwidth for CA	68

5.3A.1	General	68

5.3A.2	Maximum transmission bandwidth configuration for CA	68

5.3A.3	Minimum guardband and transmission bandwidth configuration for CA	68

5.3A.4	Void	70

5.3A.5	UE channel bandwidth per operating band for CA	70

5.3E	Channel bandwidth for V2X	71

5.3E.1	General	71

5.3E.1A	Channel bandwidth for Sidelink CA	71

5.3E.1F	Channel bandwidth for Sidelink Unlicensed	72

5.3E.2	Channel bandwidth for V2X concurrent operation	73

5.3E.2F	Channel bandwidth for SL-U concurrent operation	74

5.3I	Channel bandwidth for (e)RedCap	75

5.4	Channel arrangement	75

5.4.1	Channel spacing	75

5.4.1.1	Channel spacing for adjacent NR carriers	75

5.4.2	Channel raster	75

5.4.2.1	NR-ARFCN and channel raster	75

5.4.2.2	Channel raster to resource element mapping	76

5.4.2.3	Channel raster entries for each operating band	76

5.4.3	Synchronization raster	80

5.4.3.1	Synchronization raster and numbering	80

5.4.3.2	Synchronization raster to synchronization block resource element mapping	81

5.4.3.3	Synchronization raster entries for each operating band	81

5.4.4	TX–RX frequency separation	83

5.4A	Channel arrangement for CA	84

5.4A.1	Channel spacing for CA	84

5.4A.2	Channel raster for CA	85

5.4A.3	Synchronization raster for CA	85

5.4A.4	Tx-Rx frequency separation for CA	85

5.4B	Reserved	85

5.4C	Reserved	85

5.4D	Reserved	85

5.4E	Channel arrangement for V2X	85

5.4E.1	Channel spacing	85

5.4E.1A	Channel spacing for Sidelink CA	85

5.4E.1F	Channel spacing for Sidelink Unlicensed	86

5.4E.2	Channel raster	86

5.4E.2.1	NR-ARFCN and channel raster	86

5.4E.2.1A	Void	86

5.4E.2.1F	Void	86

5.4E.2.2	Channel raster to resource element mapping	86

5.4E.2.2A	Void	86

5.4E.2.2F	Void	86

5.4E.2.3	Channel raster entries for each operating band	86

5.4E.2.3A	Void	86

5.4E.2.3F	Void	86

5.4E.2A	Channel raster for Sidelink CA	86

5.4E.2A.1	NR-ARFCN and channel raster for Sidelink CA	86

5.4E.2A.2	Channel raster to resource element mapping for Sidelink CA	87

5.4E.2A.3	Channel raster entries for each operating band for Sidelink CA	87

5.4E.2F	Channel raster for Sidelink Unlicensed	87

5.4E.2F.1	NR-ARFCN and channel raster for Sidelink Unlicensed	87

5.4E.2F.2	Channel raster to resource element mapping for Sidelink Unlicensed	87

5.4E.2F.3	Channel raster entries for Sidelink Unlicensed	87

5.4E.3	Synchronization raster for V2X	87

5.4E.3A	Synchronization raster for Sidelink CA	87

5.4E.3F	Synchronization raster for Sidelink Unlicensed	87

5.4I	Channel arrangement for (e)RedCap	87

5.4I.1	Channel spacing for (e)RedCap	87

5.4I.2	Channel raster for (e)RedCap	88

5.4I.2.1	NR-ARFCN and channel raster	88

5.4I.2.2	Channel raster to resource element mapping	88

5.4I.2.3	Channel raster entries for each operating band	88

5.4I.3	Synchronization raster for (e)RedCap	88

5.4I.4	Tx-Rx frequency separation for (e)RedCap	88

5.5	Void	89

5.5A	Configurations for CA	89

5.5A.0	General	89

5.5A.1	Configurations for intra-band contiguous CA	90

5.5A.2	Configurations for intra-band non-contiguous CA	93

5.5A.3	Configurations for inter-band CA	98

5.5A.3.0	General	98

5.5A.3.1	Configurations for inter-band CA (two bands)	98

Table 5.5A.3.1-1a ~ Table 5.5A.3.1-1e	98

Table 5.5A.3.1-1f ~ Table 5.5A.3.1-1j	115

Table 5.5A.3.1-1k ~ Table 5.5A.3.1-1n	133

5.5A.3.2	Configurations for inter-band CA (three bands)	148

Table 5.5A.3.2-1a	148

Table 5.5A.3.2-1b	209

Table 5.5A.3.2-1c	257

5.5A.3.3	Configurations for inter-band CA (four bands)	279

Table 5.5A.3.3-1a	280

Table 5.5A.3.3-1b	332

5.5A.3.4	Configurations for inter-band CA (five bands)	378

5.5A.3.5	Configurations for inter-band CA (six bands)	401

5.5B	Configurations for DC	402

5.5C	Configurations for SUL	416

5.5D	Reserved	427

5.5E	Void	427

5.5E.1A	Configurations for Sidelink CA	427

5.5E.1A.1	Configurations for Sidelink intra-band contiguous CA	427

6	Transmitter characteristics	428

6.1	General	428

6.1A	General	428

6.1F	General	428

6.1G	(Reserved)	428

6.1H	(Reserved)	428

6.1I	(Reserved)	428

6.1J	General	428

6.1K	(Reserved)	429

6.1L	(Reserved)	429

6.2	Transmitter power	429

6.2.1	UE maximum output power	429

6.2.1I	Void	431

6.2.2	UE maximum output power reduction	431

6.2.3	UE additional maximum output power reduction	434

6.2.3.1	General	434

6.2.3.2	A-MPR for NS_04	438

6.2.3.3	A-MPR for NS_10	442

6.2.3.4	A-MPR for NS_05 and NS_05U	443

6.2.3.5	A-MPR for NS_40	446

6.2.3.6	A-MPR for NS_43 and NS_43U	447

6.2.3.7	A-MPR for NS_03 and NS_03U	449

6.2.3.8	A-MPR for NS_37	450

6.2.3.9	A-MPR for NS_38	451

6.2.3.10	A-MPR for NS_39	451

6.2.3.11	A-MPR for NS_41	452

6.2.3.12	A-MPR for NS_42	452

6.2.3.13	A-MPR for NS_18	453

6.2.3.14	A-MPR for NS_21	454

6.2.3.15	A-MPR for NS_24	456

6.2.3.16	A-MPR for NS_27	457

6.2.3.17	A-MPR for NS_46	459

6.2.3.18	A-MPR for NS_47	463

6.2.3.19	A-MPR for NS_50	465

6.2.3.20	A-MPR for NS_44	468

6.2.3.21	A-MPR for NS_12	469

6.2.3.22	A-MPR for NS_13	469

6.2.3.23	A-MPR for NS_14	470

6.2.3.24	A-MPR for NS_15	470

6.2.3.25	A-MPR for NS_45	471

6.2.3.26	A-MPR for NS_48	472

6.2.3.27	A-MPR for NS_49	474

6.2.3.28	A-MPR for NS_51	476

6.2.3.29	A-MPR for NS_07	477

6.2.3.30	A-MPR for NS_56	478

6.2.3.31	A-MPR for NS_35	479

6.2.3.32	A-MPR for NS_06	479

6.2.4	Configured transmitted power	480

6.2A	Transmitter power for CA	485

6.2A.0	General	485

6.2A.1	UE maximum output power for CA	485

6.2A.1.1	UE maximum output power for Intra-band contiguous CA	485

6.2A.1.2	UE maximum output power for Intra-band non-contiguous CA	486

6.2A.1.3	UE maximum output power for Inter-band CA	486

6.2A.1.4	Void	492

6.2A.1.5	Void	492

6.2A.2	UE maximum output power reduction for CA	492

6.2A.2.1	UE maximum output power reduction for Intra-band contiguous CA	492

6.2A.2.2	UE maximum output power reduction for Intra-band non-contiguous CA	496

6.2A.2.2.0	General	496

6.2A.2.2.1	MPR to meet -30dBm/MHz	497

6.2A.2.2.2	MPR to meet -13dBm/MHz	498

6.2A.2.3	UE maximum output power reduction for Inter-band CA	500

6.2A.2.4	Void	500

6.2A.3	UE additional maximum output power reduction for CA	500

6.2A.3.1	UE additional maximum output power reduction for Intra-band CA	500

6.2A.3.1.1	UE additional maximum output power reduction for Intra-band contiguous CA	500

6.2A.3.1.2	UE additional maximum output power reduction for Intra-band non-contiguous CA	507

6.2A.3.1.3	UE additional maximum output power reduction for Inter-band CA	511

6.2A.4	Configured output power for CA	513

6.2A.4.1	Configured transmitted power level	513

6.2A.4.1.1	Configured transmitted power for Intra-band contiguous CA	513

6.2A.4.1.2	Configured transmitted power for Intra-band non-contiguous CA	516

6.2A.4.1.3	Configured transmitted power for Inter-band CA	518

6.2A.4.1.4	Void	522

6.2A.4.2	ΔTIB,c for CA	522

6.2A.4.2.1	Void	522

6.2A.4.2.2	Void	522

6.2A.4.2.3	ΔTIB,c for Inter-band CA (two bands)	522

6.2A.4.2.4	ΔTIB,c for Inter-band CA (three bands)	527

6.2A.4.2.5	ΔTIB,c for Inter-band CA (four bands)	533

6.2A.4.2.6	ΔTIB,c for Inter-band CA (five bands)	536

6.2A.4.2.7	ΔTIB,c for Inter-band CA (six bands)	537

6.2B	Transmitter power for NR-DC	537

6.2B.0	General	537

6.2B.1	UE maximum output power for NR-DC	537

6.2B.2	UE maximum output power reduction for NR-DC	539

6.2B.3	UE additional maximum output power reduction for NR-DC	539

6.2B.4.1	Configured transmitted power level for NR-DC	539

6.2B.4.2	ΔTIB,c for NR-DC	543

6.2C	Transmitter power for SUL	544

6.2C.1	Configured transmitted power for SUL	544

6.2C.2	ΔTIB,c	544

6.2D	Transmitter power for UL MIMO	547

6.2D.1	UE maximum output power for UL MIMO	547

6.2D.2	UE maximum output power reduction for UL MIMO	549

6.2D.3	UE additional maximum output power reduction for UL MIMO	552

6.2D.4	Configured transmitted power for UL MIMO	552

6.2E	Transmitter power for V2X	553

6.2E.1	UE maximum output power for V2X	553

6.2E.1.1	General	553

6.2E.1.1A	Void	554

6.2E.1.2	UE maximum output power for V2X concurrent operation	554

6.2E.1A	UE maximum output power for Sidelink CA	556

6.2E.1F	UE maximum output power for Sidelink Unlicensed	556

6.2E.1F.1	General	556

6.2E.1.2F	Void	557

6.2E.1F.2	UE Maximum output power for SL-U concurrent operation	557

6.2E.2	UE maximum output power reduction for V2X	558

6.2E.2.1	General	558

6.2E.2.1A	MPR for sidelink CA	558

6.2E.2.2	MPR for Power class 2 and Power class 3 V2X UE	559

6.2E.2.3	MPR for Power class 2 and Power class 3 V2X concurrent operation	561

6.2E.2.4	MPR for Power class 1 UE in Band n14	564

6.2E.2F	UE maximum output power reduction for Sidelink Unlicensed	564

6.2E.2F.1	General	564

6.2E.2F.2	MPR for NR SL-U UE	564

6.2E.2F.3	MPR for SL-U concurrent operation	566

6.2E.3	UE additional maximum output power reduction for V2X	566

6.2E.3.1	General	566

6.2E.3.2	A-MPR for V2X UE by NS_33	567

6.2E.3.3	A-MPR for Power class 3 V2X UE by NS_52	571

6.2E.3.4	A-MPR for V2X concurrent operation	573

6.2E.3F	UE additional maximum output power reduction for Sidelink Unlicensed	573

6.2E.3F.1	General	573

6.2E.3F.2	A-MPR for NS_31	574

6.2E.3F.3	Void	576

6.2E.3F.4	Void	576

6.2E.3F.5	Void	576

6.2E.3F.6	A-MPR for NS_61	576

6.2E.3F.7	A-MPR for SL-U concurrent operation	577

6.2E.3F.8	A-MPR for NS_28	577

6.2E.3F.9	A-MPR for NS_29	578

6.2E.3F.10	A-MPR for NS_30	579

6.2E.3F.11	A-MPR for NS_54	581

6.2E.3F.12	A-MPR for NS_64	582

6.2E.3F.13	A-MPR for NS_65	584

6.2E.3F.14	A-MPR for NS_66	585

6.2E.3F.15	A-MPR for NS_67 or NS_71	586

6.2E.3F.16	A-MPR for NS_68	587

6.2E.3F.17	A-MPR for NS_69	588

6.2E.4	Configured transmitted power for V2X	589

6.2E.4.1	General	589

6.2E.4.2	Configured transmitted power for inter-band V2X concurrent operation	590

6.2E.4.3	Configured transmitted power for intra-band V2X concurrent operation	591

6.2E.4A	Configured transmitted power for Sidelink CA	593

6.2E.4F	Configured transmitted power for Sidelink Unlicensed	594

6.2E.4F.1	General	594

6.2E.4F.2	Configured transmitted power for inter-band concurrent operation	595

6.2F	Transmitter power for shared spectrum channel access	596

6.2F.1	UE maximum output power	596

6.2F.1A	UE maximum output power for CA	597

6.2F.1A.1	UE maximum output power for inter-band CA	597

6.2F.1A.2	UE maximum output power for intra-band contiguous CA	598

6.2F.1A.2.1	Additional requirements for transmit power density for intra-band contiguous CA for CA_NS_53	598

6.2F.1A.2.2	Additional requirements for transmit power density for intra-band contiguous CA for CA_NS_54	598

6.2F.1B	UE maximum output power for NR-DC	599

6.2F.1D	UE maximum output power for UL MIMO	599

6.2F.2	UE maximum output power reduction	599

6.2F.2A	UE maximum output power reduction for CA	602

6.2F.2A.1	UE maximum output power reduction for inter-band CA	602

6.2F.2A.2	UE maximum output power reduction for intra-band contiguous CA	602

6.2F.2D	UE maximum output power reduction for UL MIMO	605

6.2F.3	UE additional maximum output power reduction	605

6.2F.3.1	General	605

6.2F.3.2	A-MPR for NS_28	606

6.2F.3.3	A-MPR for NS_29	607

6.2F.3.4	A-MPR for NS_30	608

6.2F.3.5	A-MPR for NS_31	609

6.2F.3.6	A-MPR for NS_53	610

6.2F.3.7	A-MPR for NS_54	610

6.2F.3.8	A-MPR for NS_58	611

6.2F.3.9	A-MPR for NS_59	612

6.2F.3.10	A-MPR for NS_60	613

6.2F.3.11	A-MPR for NS_61	614

6.2F.3.12	A-MPR for NS_63	615

6.2F.3.13	A-MPR for NS_64	616

6.2F.3.14	A-MPR for NS_65	616

6.2F.3.15	A-MPR for NS_66	617

6.2F.3.16	A-MPR for "NS_67" or "NS_71"	617

6.2F.3.17	A-MPR for NS_68	618

6.2F.3.18	A-MPR for NS_69	618

6.2F.3A	UE additional maximum output power reduction for CA	619

6.2F.3A.1	UE additional maximum output power reduction for inter-band CA	619

6.2F.3A.2	UE additional maximum output power reduction for intra-band CA	619

6.2F.3A.2.0	General	619

6.2F.3A.2.1	UE additional maximum output power reduction for intra-band contiguous CA	619

6.2F.3A.2.2	A-MPR for CA_NS_53	621

6.2F.3A.2.3	A-MPR for CA_NS_54	622

6.2F.3D	UE additional maximum output power reduction for UL MIMO	622

6.2F.4	Configured transmitted power	622

6.2F.4D	Configured transmitted power UL MIMO	622

6.2G	Transmitter power for Tx Diversity	623

6.2G.1	UE maximum output power for Tx Diversity	623

6.2G.2	UE maximum output power reduction for Tx Diversity	623

6.2G.3	UE additional maximum output power reduction for Tx Diversity	623

6.2G.4	Configured transmitted power for Tx Diversity	623

6.2H	Transmitter power for CA with UL MIMO	624

6.2H.1	Transmitter power for intra-band UL contiguous CA with UL MIMO	624

6.2H.1.1	UE maximum output power for intra-band UL contiguous CA with UL MIMO	624

6.2H.1.2	UE maximum output power reduction for intra-band UL contiguous CA with UL MIMO	625

6.2H.1.3	UE additional maximum output power reduction for intra-band UL contiguous CA with UL MIMO	625

6.2H.1.4	Configured transmitted power for intra-band UL contiguous CA with UL MIMO	625

6.2H.2	Void	626

6.2H.3	Transmitter power for inter-band UL CA with UL MIMO	626

6.2H.3.1	UE maximum output power for inter-band UL CA with UL MIMO	626

6.2H.3.2	UE maximum output power reduction for inter-band UL CA with UL MIMO	628

6.2H.3.3	UE additional maximum output power reduction for inter-band UL CA with UL MIMO	628

6.2H.3.4	Configured transmitted power for inter-band UL CA with UL MIMO	628

6.2I	Transmitter power for (e)RedCap	628

6.2I.1	Maximum output power for RedCap	628

6.2J	Transmitter power for ATG	629

6.2J.1	UE maximum output power for ATG	629

6.2J.2	Configured transmitted power for ATG	629

6.2K	Transmitter power for Aerial UE	630

6.2K.1	Maximum output power for Aerial UE	630

6.2K.2	Maximum output power reduction for Aerial UE	630

6.2K.3	Additional maximum output power reduction for Aerial UE	630

6.2K.3.1	General	630

6.2K.3.2	A-MPR for NS_UAV_44	631

6.2K.3.3	A-MPR for NS_UAV_70	632

6.2K.4	Configured transmitted power for Aerial UE	633

6.2L	Transmitter power for CA with Tx Diversity	633

6.2L.1	Void	633

6.2L.2	Void	633

6.2L.3	Transmitter power for inter-band UL CA with Tx Diversity	633

6.2L.3.1	UE maximum output power for inter-band UL CA with Tx Diversity	633

6.2L.3.2	UE maximum output power reduction for inter-band UL CA with Tx Diversity	635

6.2L.3.3	UE additional maximum output power reduction for inter-band UL CA with Tx Diversity	635

6.2L.3.4	Configured transmitted power for inter-band UL CA with Tx Diversity	636

6.3	Output power dynamics	636

6.3.1	Minimum output power	636

6.3.2	Transmit OFF power	636

6.3.3	Transmit ON/OFF time mask	637

6.3.3.1	General	637

6.3.3.2	General ON/OFF time mask	637

6.3.3.3	Transmit power time mask for slot and short or long subslot boundaries	637

6.3.3.4	PRACH time mask	638

6.3.3.5	Void	639

6.3.3.6	SRS time mask	639

6.3.3.7	PUSCH-PUCCH and PUSCH-SRS time masks	640

6.3.3.8	Transmit power time mask for consecutive slot or long subslot transmission and short subslot transmission boundaries	641

6.3.3.9	Transmit power time mask for consecutive short subslot  transmissions boundaries	642

6.3.4	Power control	642

6.3.4.1	General	642

6.3.4.2	Absolute power tolerance	642

6.3.4.3	Relative power tolerance	643

6.3.4.4	Aggregate power tolerance	643

6.3A	Output power dynamics for CA	644

6.3A.1	Minimum output power for CA	644

6.3A.1.1	Minimum output power for intra-band contiguous CA	644

6.3A.1.2	Minimum output power for intra-band non-contiguous CA	644

6.3A.1.3	Minimum output power for inter-band CA	644

6.3A.1.4	Void	644

6.3A.2	Transmit OFF power for CA	644

6.3A.2.1	Transmit OFF power for intra-band contiguous CA	644

6.3A.2.2	Transmit OFF power for intra-band non-contiguous CA	644

6.3A.2.3	Transmit OFF power for inter-band CA	644

6.3A.2.4	Void	645

6.3A.3	Transmit ON/OFF time mask for CA	645

6.3A.3.1	Transmit ON/OFF time mask for intra-band contiguous CA	645

6.3A.3.2	Transmit ON/OFF time mask for intra-band non-contiguous CA	645

6.3A.3.3	Transmit ON/OFF time mask for inter-band CA	645

6.3A.3.3.1	General	645

6.3A.3.3.2	Time mask for switching between two uplink carriers	646

6.3A.3.3.3	Time mask for switching between two uplink carriers with two transmit antenna connectors	647

6.3A.3.3.4	Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	648

6.3A.3.3.5	Time mask for switching between two uplink bands with two transmit antenna connectors	650

6.3A.3.3.6	Time mask for switching across up to four uplink bands	651

6.3A.3.3.6a	Additional requirements for three-band switching with dual TAG	655

6.3A.3.4	Void	655

6.3A.4	Power control for CA	655

6.3A.4.1	Power control for intra-band contiguous CA	655

6.3A.4.1.1	Absolute power tolerance	655

6.3A.4.1.2	Relative power tolerance	655

6.3A.4.1.3	Aggregate power control tolerance	656

6.3A.4.2	Power control for intra-band non-contiguous CA	656

6.3A.4.2.1	Absolute power tolerance	656

6.3A.4.2.1.1	 Minimum requirements	656

6.3A.4.2.2	Relative power tolerance	656

6.3A.4.2.2.1	Minimum requirements	656

6.3A.4.2.3	Aggregate power control tolerance	656

6.3A.4.3	Power control for inter-band CA	656

6.3A.4.4	Void	656

6.3B	Output power dynamics for NR-DC	656

6.3C	Output power dynamics for SUL	657

6.3C.1	Void	657

6.3C.2	Void	657

6.3C.3	Transmit ON/OFF time mask for SUL	657

6.3C.3.0	General	657

6.3C.3.1	Time mask for switching between two uplink carriers	657

6.3C.3.2	Time mask for switching between two uplink carriers with two transmit antenna connectors	658

6.3C.3.3	Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	659

6.3C.3.4	Time mask for switching between two uplink bands with two transmit antenna connectors	661

6.3C.3.5	Time mask for switching across up to four uplink bands	662

6.3C.3.5a	Additional requirements for three-band switching with dual TAG	666

6.3D	Output power dynamics for UL MIMO	666

6.3D.1	Minimum output power for UL MIMO	666

6.3D.2	Transmit OFF power for UL MIMO	666

6.3D.3	Transmit ON/OFF time mask for UL MIMO	666

6.3D.4	Power control for UL MIMO	666

6.3E	Output power dynamics for V2X	667

6.3E.1	Minimum output power for V2X	667

6.3E.1.1	General	667

6.3E.1.1A	Minimum output power for sidelink CA	667

6.3E.1.2	Minimum output power for V2X concurrent operation	667

6.3E.1F	Minimum output power for Sidelink Unlicensed	667

6.3E.1F.1	Minimum output power for SL-U concurrent operation	668

6.3E.2	Transmit OFF power for V2X	668

6.3E.2.1	General	668

6.3E.2.1A	Transmit OFF power for sidelink CA	668

6.3E.2.2	Transmit OFF power for V2X concurrent operation	668

6.3E.2F	Transmit OFF power for Sidelink Unlicensed	668

6.3E.2F.1	Transmit OFF power for SL-U concurrent operation	669

6.3E.3	Transmit ON/OFF time mask for V2X	669

6.3E.3.1	General	669

6.3E.3.1A	Transmit ON/OFF time mask for sidelink CA	669

6.3E.3.2	General time mask	669

6.3E.3.3	S-SSB time mask	669

6.3E.3.4	Transmit ON/OFF time mask for V2X concurrent operation	670

6.3E.3F	Transmit ON/OFF time mask for Sidelink Unlicensed	671

6.3E.3F.1	General	671

6.3E.3F.2	General ON/OFF time mask	671

6.3E.3F.3	S-SSB time mask	672

6.3E.3F.4	Transmit ON/OFF time mask for NR SL-U concurrent operation	672

6.3E.4	Power control for V2X	672

6.3E.4.1	General	672

6.3E.4.1A	Power control for sidelink CA	672

6.3E.4.2	Absolute power tolerance	672

6.3E.4.3	Power control for V2X concurrent operation	673

6.3E.4F	Power control for Sidelink Unlicensed	673

6.3E.4F.1	General	673

6.3E.4F.2	Absolute power tolerance	673

6.3E.4F.3	Power control for SL-U concurrent operation	673

6.3F	Output power dynamics for shared spectrum channel access	673

6.3F.1	Minimum output power	673

6.3F.2	Transmit OFF power	673

6.3F.3	Transmit ON/OFF time mask	673

6.3F.3.1	General	673

6.3F.3.2	General ON/OFF time mask	673

6.3F.3A	General ON/OFF mask for CA	674

6.3F.3A.1	General ON/OFF mask for inter-band CA	674

6.3F.4	Power control	674

6.3F.4.1	General	674

6.3F.4.2	Absolute power tolerance	674

6.3F.4.3	Relative power tolerance	674

6.3F.4.4	Aggregate power tolerance	674

6.3F.4A	Power control for inter-band CA	674

6.3G	Output power dynamics for Tx Diversity	675

6.3G.1	Minimum output power for Tx Diversity	675

6.3G.2	Transmit OFF power for Tx Diversity	675

6.3G.3	Transmit ON/OFF time mask for Tx Diversity	675

6.3G.4	Power control for Tx Diversity	675

6.3H	Output power dynamics for CA with UL MIMO	675

6.3H.1	Output power dynamics for intra-band UL contiguous CA with UL MIMO	675

6.3H.1.1	Minimum output power for intra-band UL contiguous CA with UL MIMO	675

6.3H.1.2	Transmit OFF power for intra-band UL contiguous CA with UL MIMO	675

6.3H.1.3	Transmit ON/OFF time mask for intra-band UL contiguous CA with UL MIMO	675

6.3H.1.4	Power control for intra-band UL contiguous CA with UL MIMO	676

6.3H.3	Output power dynamics for inter-band UL CA with UL MIMO	676

6.3H.3.1	Minimum output power for inter-band UL CA with UL MIMO	676

6.3H.3.2	Transmit OFF power for inter-band UL CA with UL MIMO	676

6.3H.3.3	Transmit ON/OFF time mask for inter-band UL CA with UL MIMO	676

6.3H.3.4	Power control for inter-band UL CA with UL MIMO	676

6.3I	(Reserved)	676

6.3J	Output power dynamics for ATG	676

6.3J.1	Minimum output power for ATG	676

6.3J.2	Transmit OFF power for ATG	677

6.3J.3	Transmit ON/OFF time mask for ATG	677

6.3J.4	Power control for ATG	677

6.3K	(Reserved)	677

6.3L	Output power dynamics for CA with Tx Diversity	677

6.3L.1	Void	677

6.3L.2	Void	677

6.3L.3	Output power dynamics for inter-band UL CA with Tx Diversity	677

6.3L.3.1	Minimum output power for inter-band UL CA with Tx Diversity	677

6.3L.3.2	Transmit OFF power for inter-band UL CA with Tx Diversity	678

6.3L.3.3	Transmit ON/OFF time mask for inter-band UL CA with Tx Diversity	678

6.3L.3.4	Power control for inter-band UL CA with Tx Diversity	678

6.4	Transmit signal quality	678

6.4.1	Frequency error	678

6.4.2	Transmit modulation quality	678

6.4.2.0	General	678

6.4.2.1	Error Vector Magnitude	678

6.4.2.1a	Error Vector Magnitude including symbols with transient period	679

6.4.2.2	Carrier leakage	680

6.4.2.3	In-band emissions	680

6.4.2.4	EVM equalizer spectrum flatness	681

6.4.2.4.1	Requirements for Pi/2 BPSK modulation with powerBoosting-pi2BPSK capability	682

6.4.2.4.2	Requirements for Pi/2 BPSK and QPSK modulation with powerBoosting-pi2BPSK-QPSK-Modified-r18 capability	683

6.4.2.5	Phase continuity requirements for DMRS bundling	684

6.4A	Transmit signal quality for CA	685

6.4A.1	Frequency error for CA	685

6.4A.1.1	Frequency error for intra-band contiguous CA	685

6.4A.1.2	Frequency error for intra-band non-contiguous CA	685

6.4A.1.3	Frequency error for inter-band CA	685

6.4A.1.4	Void	686

6.4A.2	Transmit modulation quality for CA	686

6.4A.2.1	Transmit modulation quality for intra-band contiguous CA	686

6.4A.2.1.0	General	686

6.4A.2.1.1	Error Vector Magnitude	686

6.4A.2.1.2	In-band emissions	686

6.4A.2.1.3	Carrier leakage	689

6.4A.2.2	Transmit modulation quality for intra-band non-contiguous CA	689

6.4A.2.2.0	General	689

6.4A.2.2.1	Error Vector Magnitude	689

6.4A.2.2.2	In-band emissions	690

6.4A.2.2.3	Carrier leakage	690

6.4A.2.3	Transmit modulation quality for inter-band CA	690

6.4A.2.4	Void	691

6.4B	Transmit signal quality for NR-DC	691

6.4C	Transmit signal quality for SUL	691

6.4D	Transmit signal quality for UL MIMO	691

6.4D.0	General	691

6.4D.1	Frequency error for UL MIMO	691

6.4D.2	Transmit modulation quality for UL MIMO	692

6.4D.2.0	General	692

6.4D.2.1	Error Vector Magnitude	692

6.4D.2.2	Carrier leakage	692

6.4D.2.3	In-band emissions	692

6.4D.2.4	EVM equalizer spectrum flatness for UL MIMO	692

6.4D.3	Time alignment error for UL MIMO	692

6.4D.4	Requirements for coherent UL MIMO	693

6.4E	Transmit signal quality for V2X	693

6.4E.1	Frequency error for V2X	693

6.4E.1.1	General	693

6.4E.1.1A	Frequency error for sidelink CA	694

6.4E.1.2	Frequency error for V2X concurrent operation	694

6.4E.1F	Frequency error for Sidelink Unlicensed	694

6.4E.1F.1	Frequency error for SL-U concurrent operation	694

6.4E.2	Transmit modulation quality for V2X	694

6.4E.2.1	General	694

6.4E.2.2	Error Vector Magnitude for V2X	694

6.4E.2.2A	Error Vector Magnitude for sidelink CA	694

6.4E.2.3	Carrier leakage for V2X	694

6.4E.2.3A	Carrier leakage for sidelink CA	695

6.4E.2.4	In-band emissions for V2X	695

6.4E.2.4A	In-band emissions for sidelink CA	695

6.4E.2.5	EVM equalizer spectrum flatness for V2X	695

6.4E.2.6	Transmit modulation quality for V2X concurrent operation	695

6.4E.2F	Transmit modulation quality for Sidelink Unlicensed	695

6.4E.2F.0	General	695

6.4E.2F.1	Error Vector Magnitude	695

6.4E.2F.2	Carrier leakage	695

6.4E.2F.3	In-band emissions	696

6.4E.2F.4	EVM equalizer spectrum flatness	696

6.4E.2F.5	Transmit modulation quality for SL-U concurrent operation	696

6.4F	Transmit signal quality for shared spectrum channel access	696

6.4F.1	Frequency error	696

6.4F.2	Transmit modulation quality	696

6.4F.2.0	General	696

6.4F.2.1	Error Vector Magnitude	696

6.4F.2.2	Carrier leakage	696

6.4F.2.3	In-band emissions	696

6.4F.2.4	EVM equalizer spectrum flatness	697

6.4F.2A	Transmit modulation quality for CA	697

6.4F.2A.1	Transmit modulation quality for inter-band CA	697

6.4G	Transmit signal quality for Tx Diversity	698

6.4G.1	Frequency error for Tx Diversity	698

6.4G.2	 Transmit modulation quality for Tx Diversity	698

6.4G.2.0	General	698

6.4G.2.1	Error Vector Magnitude	698

6.4G.2.2	Carrier leakage	698

6.4G.2.3	In-band emissions	698

6.4G.2.4	EVM equalizer spectrum flatness for Tx Diversity	698

6.4H	Transmit signal quality for CA with UL MIMO	699

6.4H.1	Transmit signal quality for intra-band UL contiguous CA with UL MIMO	699

6.4H.1.1	Frequency error for intra-band UL contiguous CA with UL MIMO	699

6.4H.1.2	Transmit modulation quality for intra-band UL contiguous CA with UL MIMO	699

6.4H.1.2.0	General	699

6.4H.1.2.1	Error Vector Magnitude	699

6.4H.1.2.2	Carrier leakage	699

6.4H.1.2.3	In-band emissions	700

6.4H.1.3	Time alignment error for intra-band UL contiguous CA with UL MIMO	700

6.4H.1.4	Coherent UL MIMO requirement for intra-band UL contiguous CA with UL MIMO	700

6.4H.2	Void	700

6.4H.3	Transmit signal quality for inter-band UL CA with UL MIMO	700

6.4H.3.1	Frequency error for inter-band UL CA with UL MIMO	700

6.4H.3.2	Transmit modulation quality for inter-band UL CA with UL MIMO	700

6.4I	(Reserved)	700

6.4J	Transmit signal quality for ATG	700

6.4J.1	Frequency error for ATG	700

6.4J.2	Transmit modulation quality for ATG	701

6.4K	(Reserved)	701

6.4L	Transmit signal quality for CA with Tx Diversity	701

6.4L.1	Void	701

6.4L.2	Void	701

6.4L.3	Transmit signal quality for inter-band UL CA with Tx Diversity	701

6.4L.3.1	Frequency error for inter-band UL CA with Tx Diversity	701

6.4L.3.2	Transmit modulation quality for inter-band UL CA with Tx Diversity	701

6.5	Output RF spectrum emissions	701

6.5.1	Occupied bandwidth	701

6.5.2	Out of band emission	701

6.5.2.1	General	701

6.5.2.2	Spectrum emission mask	702

6.5.2.3	Additional spectrum emission mask	702

6.5.2.3.1	Requirements for network signalling value "NS_35"	702

6.5.2.3.2	Requirements for network signalling value "NS_04"	703

6.5.2.3.3	Requirements for network signalling values "NS_03" and “NS_03U”	703

6.5.2.3.4	Requirements for network signalling value "NS_06" or “NS_07”	704

6.5.2.3.5	Void	705

6.5.2.3.6	Void	705

6.5.2.3.7	Void	705

6.5.2.3.8	Requirements for network signalling value "NS_27"	705

6.5.2.3.9	Requirements for network signalling value "NS_21"	705

6.5.2.4	Adjacent channel leakage ratio	706

6.5.2.4.1	NR ACLR	706

6.5.2.4.2	UTRA ACLR	706

6.5.3	Spurious emissions	707

6.5.3.0	General	707

6.5.3.1	General spurious emissions	707

6.5.3.2	Spurious emissions for UE co-existence	708

6.5.3.3	Additional spurious emissions	716

6.5.3.3.1	Requirement for network signalling value "NS_04"	716

6.5.3.3.2	Requirement for network signalling value "NS_17"	716

6.5.3.3.3	Requirement for network signalling value "NS_18"	716

6.5.3.3.4	Requirement for network signalling values "NS_05" and “NS_05U”	716

6.5.3.3.5	Requirement for network signalling values "NS_43" and “NS_43U”	717

6.5.3.3.6	Requirement for network signalling value "NS_37"	717

6.5.3.3.7	Requirement for network signalling value "NS_38"	717

6.5.3.3.8	Requirement for network signalling value "NS_39"	717

6.5.3.3.9	Requirement for network signalling value "NS_40"	718

6.5.3.3.10	Requirement for network signalling value "NS_41"	718

6.5.3.3.11	Requirement for network signalling value "NS_42"	718

6.5.3.3.12	Requirement for network signalling value "NS_21"	719

6.5.3.3.13	Requirement for network signalling value "NS_24"	719

6.5.3.3.14	Requirement for network signalling value "NS_27"	719

6.5.3.3.15	Requirement for network signalling value "NS_47"	719

6.5.3.3.16	Requirement for network signalling value "NS_50"	720

6.5.3.3.17	Requirement for network signalling value "NS_12"	720

6.5.3.3.18	Requirement for network signalling value "NS_13"	720

6.5.3.3.19	Requirement for network signalling value "NS_14"	721

6.5.3.3.20	Requirement for network signalling value "NS_15"	721

6.5.3.3.21	Requirement for network signalling value "NS_45"	721

6.5.3.3.22	Requirement for network signalling values "NS_48" and "NS_51"	722

6.5.3.3.23	Requirement for network signalling value "NS_49"	722

6.5.3.3.24	Requirement for network signalling value "NS_44"	722

6.5.3.3.25	Requirement for network signalling value "NS_46"	723

6.5.3.3.26	Requirement for network signalling value "NS_07"	723

6.5.3.3.27	Requirement for network signalling value “NS_56”	723

6.5.3.3.28	Requirement for network signalling value “NS_62”	724

6.5.4	Transmit intermodulation	724

6.5A	Output RF spectrum emissions for CA	725

6.5A.0	General	725

6.5A.1	Occupied bandwidth for CA	725

6.5A.1.1	Void	725

6.5A.1.1a	Occupied bandwidth for Intra-band contiguous CA	725

6.5A.1.2	Occupied bandwidth for Intra-band non-contiguous CA	725

6.5A.1.3	Occupied bandwidth for Inter-band CA	725

6.5A.2	Out of band emission for CA	725

6.5A.2.1	General	725

6.5A.2.2	Spectrum emission mask	726

6.5A.2.2.1	Spectrum emission mask for intra-band contiguous CA	726

6.5A.2.2.2	Spectrum emission mask for intra-band non-contiguous CA	726

6.5A.2.2.3	Spectrum emission mask for Inter-band CA	726

6.5.A.2.2.4	Void	727

6.5A.2.3	Additional spectrum emission mask for CA	727

6.5A.2.3.1	Additional spectrum emission mask for intra-band contiguous CA	727

6.5A.2.3.2	Additional spectrum emission mask for Intra-band non-contiguous CA	728

6.5A.2.3.3	Additional spectrum emission mask for Inter-band CA	728

6.5A.2.4	Adjacent channel leakage ratio	728

6.5A.2.4.1	NR ACLR	728

6.5A.2.4.2	UTRA ACLR	730

6.5A.3	Spurious emission for CA	730

6.5A.3.1	General spurious emissions	730

6.5A.3.2	Spurious emissions for UE co-existence	731

6.5A.3.2.0	General	731

6.5A.3.2.1	Spurious emissions for UE co-existence for intra-band contiguous CA	731

6.5A.3.2.2	Spurious emissions for UE co-existence for intra-band non-contiguous CA	733

6.5A.3.2.3	Spurious emissions for UE co-existence for Inter-band CA	734

6.5A.3.2.4	Void	740

6.5A.3.2.5	Void	740

6.5A.3.2.6	Void	740

6.5A.3.3	Additional spurious emissions for CA	740

6.5A.3.3.1	Additional spurious emissions for intra-band contiguous  CA	740

6.5A.3.3.2	Additional spurious emissions for intra-band non-contiguous CA	741

6.5A.4	Transmit intermodulation for CA	742

6.5A.4.2.1	Transmit intermodulation for intra-band contiguous CA	742

6.5A.4.2.2	Void	742

6.5B	Output RF spectrum emissions for NR-DC	742

6.5D	Output RF spectrum emissions for UL MIMO	742

6.5D.1	Occupied bandwidth for UL MIMO	742

6.5D.2	Out of band emission for UL MIMO	743

6.5D.3	Spurious emission for UL MIMO	743

6.5D.4	Transmit intermodulation for UL MIMO	743

6.5E	Output RF spectrum emissions for V2X	743

6.5E.1	Occupied bandwidth for V2X	743

6.5E.1.1	General	743

6.5E.1.1A	Occupied bandwidth for sidelink CA	744

6.5E.1.2	Occupied bandwidth for V2X concurrent operation	744

6.5E.1F	Occupied bandwidth for Sidelink Unlicensed	744

6.5E.1F.1	Occupied bandwidth for SL-U concurrent operation	744

6.5E.2	Out of band emission for V2X	744

6.5E.2.1	General	744

6.5E.2.2	Spectrum emission mask	744

6.5E.2.2.1	General	744

6.5E.2.2.1A	Spectrum emission mask for sidelink CA	744

6.5E.2.2.2	Spectrum emission mask for V2X concurrent operation	745

6.5E.2.3	Additional Spectrum emission mask	745

6.5E.2.3.1	Requirements for network signalling value "NS_33"	745

6.5E.2.3.2	Requirements for network signalling value "NS_52"	745

6.5E.2.3.3	Requirements for network signalling value "NS_06"	746

6.5E.2.4	Adjacent channel leakage ratio	746

6.5E.2.4.1	General	746

6.5E.2.4.1A	ACLR for sidelink CA	746

6.5E.2.4.2	ACLR for V2X concurrent operation	746

6.5E.2F	Out of band emission for Sidelink Unlicensed	746

6.5E.2F.1	General	746

6.5E.2F.2	Spectrum emission mask for operation with shared spectrum channel access	747

6.5E.2F.2.1	Spectrum emission mask for SL-U concurrent operation	747

6.5E.2F.3	Additional spectrum emission mask	747

6.5E.2F.4	Adjacent channel leakage ratio	747

6.5E.2F.4.1	Adjacent channel leakage ratio for SL-U concurrent operation	747

6.5E.3	Spurious emissions for V2X	747

6.5E.3.1	General spurious emissions	747

6.5E.3.1A	Spurious emissions for sidelink CA	747

6.5E.3.2	Spurious emissions for UE co-existence	747

6.5E.3.2A	Spurious emissions band UE co-existence for sidelink CA	747

6.5E.3.3	Spurious emissions for UE co-existence for V2X concurrent operation	748

6.5E.3.4	Additional spurious emissions requirements for V2X	749

6.5E.3.4.1	General	749

6.5E.3.4.2	Requirements for network signalling value "NS_33"	749

6.5E.3.4.3	Void	749

6.5E.3F	Spurious emissions for Sidelink Unlicensed	749

6.5E.3F.0	General	749

6.5E.3F.1	General spurious emissions	749

6.5E.3F.2	Spurious emissions for UE co-existence	750

6.5E.3F.2.1	Spurious emissions for UE co-existence for SL-U concurrent operation	750

6.5E.3F.3	Additional spurious emissions	750

6.5E.3F.3.0	General	750

6.5E.4	Transmit intermodulation	750

6.5E.4.1	General	750

6.5E.4.1A	Transmit intermodulation for sidelink CA	750

6.5E.4.2	Transmit intermodulation for V2X concurrent operation	750

6.5E.4F	Transmit intermodulation for sidelink Unlicensed	750

6.5E.4F.1	Transmit intermodulation for SL-U concurrent operation	750

6.5F	Output RF spectrum emissions for shared spectrum channel access	751

6.5F.1	Occupied bandwidth	751

6.5F.2	Out of band emission	751

6.5F.2.1	General	751

6.5F.2.2	Spectrum emission mask for operation with shared spectrum channel access	751

6.5F.2.2.0	General	751

6.5F.2.2.1	Spectrum emission mask for non-transmitted channels	752

6.5F.2.3	Additional spectrum emission mask	752

6.5F.2.4	Adjacent channel leakage ratio	752

6.5F.2.4.0	General	752

6.5F.2.4.1	Shared spectrum channel access ACLR	752

6.5F.2.4.2	Additional requirement for network signalled value "NS_29"	752

6.5F.2A	Out of band emission for CA	753

6.5F.2A.1	Spectrum emission mask for CA	753

6.5F.2A.1.1	Spectrum emission mask for Inter-band CA	753

6.5F.2A.1.2	Spectrum emission mask for Intra-band contiguous CA	753

6.5F.2A.1.2.1	General	753

6.5F.2A.1.2.2	Intra-band contiguous CA spectrum emission mask for non-transmitted channels	754

6.5F.2A.2	Adjacent channel leakage ratio for CA	754

6.5F.2A.2.1	Adjacent channel leakage ratio for inter-band CA	754

6.5F.2A.2.2	Adjacent channel leakage ratio for intra-band contiguous CA	754

6.5F.3	Spurious emissions	754

6.5F.3.0	General	754

6.5F.3.1	General spurious emissions	755

6.5F.3.2	Spurious emissions for UE co-existence	755

6.5F.3.3	Additional spurious emissions	755

6.5F.3.3.0	General	755

6.5F.3.3.1	Requirement for network signalling value "NS_28"	755

6.5F.3.3.2	Requirement for network signalling value "NS_29"	755

6.5F.3.3.3	Requirement for network signalling value "NS_30"	757

6.5F.3.3.4	Requirement for network signalling value "NS_31"	758

6.5F.3.3.5	Requirements for network signalling value "NS_53" or "NS_54" or "NS_60" or "NS_66" or "NS_67" or "NS_71"	759

6.5F.3.3.6	Requirements for network signalling value "NS_58"	759

6.5F.3.3.7	Requirements for network signalling value "NS_61"	759

6.5F.3.3.8	Requirements for network signalling value “NS_63” or “NS_69”	759

6.5F.3.3.9	Requirements for network signalling value “NS_64”	760

6.5F.3A	Spurious emissions for CA	760

6.5F.3A.0	General	760

6.5F.3A.1	General spurious emissions	761

6.5F.3A.2	Spurious emissions for UE co-existence	761

6.5F.3A.3	Additional spurious emissions	761

6.5F.3A.3.0	General	761

6.5F.3A.3.1	Requirements for network signalling value "CA_NS_53" or "CA_NS_54"	761

6.5F.4	Transmit intermodulation	761

6.5G	Output RF spectrum emissions for Tx Diversity	761

6.5G.1	Occupied bandwidth for Tx Diversity	761

6.5G.2	Out of band emission for Tx Diversity	762

6.5G.3	Spurious emission for Tx Diversity	762

6.5G.4	Transmit intermodulation for Tx Diversity	762

6.5H	Output RF spectrum emissions for CA with UL MIMO	762

6.5H.1	Output RF spectrum emissions for intra-band UL contiguous CA with UL MIMO	762

6.5H.1.1	Occupied bandwidth for intra-band UL contiguous CA with UL MIMO	762

6.5H.1.2	Out of band emission for intra-band UL contiguous CA with UL MIMO	762

6.5H.1.3	 Spurious emission for intra-band UL contiguous CA with UL MIMO	762

6.5H.1.4	Transmit intermodulation for intra-band UL contiguous CA with UL MIMO	763

6.5H.2	Void	763

6.5H.3	Output RF spectrum emissions for inter-band UL CA with UL MIMO	763

6.5H.3.1	Occupied bandwidth for inter-band UL CA with UL MIMO	763

6.5H.3.2	Out of band emission for inter-band UL CA with UL MIMO	763

6.5H.3.3	Spurious emission for inter-band UL CA with UL MIMO	763

6.5H.3.4	Transmit intermodulation for inter-band UL CA with UL MIMO	763

6.5I	(Reserved)	763

6.5J	Output RF spectrum emissions for ATG	764

6.5J.1	Occupied bandwidth for ATG	764

6.5J.2	Out of band emission for ATG	764

6.5J.2.1	General	764

6.5J.2.2	Spectrum emission mask	764

6.5J.2.3	Adjacent channel leakage ratio	764

6.5J.3	Spurious emissions for ATG	764

6.5K	Output RF spectrum emissions for Aerial UE	765

6.5K.1	Occupied bandwidth for Aerial UE	765

6.5K.2	Out of band emission for Aerial UE	765

6.5K.3	Spurious emissions for Aerial UE	765

6.5K.3.0	General	765

6.5K.3.1	General spurious emissions	765

6.5K.3.2	Spurious emissions for UE co-existence	765

6.5K.3.3	Additional spurious emissions	765

6.5K.3.3.1	Requirement for network signalling value "NS_UAV_44"	765

6.5K.3.3.2	Requirement for network signalling value "NS_UAV_46"	765

6.5K.3.3.3	Requirement for network signalling value “NS_UAV_70"	766

6.5L	Output RF spectrum emissions for CA with Tx Diversity	766

6.5L.1	Void	766

6.5L.2	Void	766

6.5L.3	Output RF spectrum emissions for inter-band UL CA with Tx Diversity	766

6.5L.3.1	Occupied bandwidth for inter-band UL CA with Tx Diversity	766

6.5L.3.2	Out of band emission for inter-band UL CA with Tx Diversity	766

6.5L.3.3	Spurious emission for inter-band UL CA with Tx Diversity	766

6.5L.3.4	Transmit intermodulation for inter-band UL CA with Tx Diversity	766

6.6	Void	766

6.6E	Time alignment error	767

7	Receiver characteristics	769

7.1	General	769

7.1A	General	770

7.1F	General	770

7.1G	(Reserved)	770

7.1H	(Reserved)	770

7.1I	General	770

7.1J	General for ATG	770

7.1K	(Reserved)	770

7.1L	(Reserved)	770

7.2	Diversity characteristics	770

7.2J	Diversity characteristics for ATG	771

7.3	Reference sensitivity	771

7.3.1	General	771

7.3.2	Reference sensitivity power level	771

7.3.3	ΔRIB,c	781

7.3A	Reference sensitivity for CA	781

7.3A.1	General	781

7.3A.2	Reference sensitivity power level for CA	782

7.3A.2.1	Reference sensitivity power level for Intra-band contiguous CA	782

7.3A.2.2	Reference sensitivity power level for Intra-band non-contiguous CA	783

7.3A.2.3	Reference sensitivity power level for Inter-band CA	785

7.3A.2.4	Void	785

7.3A.3	ΔRIB,c for CA	786

7.3A.3.1	General	786

7.3A.3.2	ΔRIB,c for Inter-band CA	786

7.3A.3.2.1	ΔRIB,c for two bands	786

7.3A.3.2.2	Void	788

7.3A.3.2.3	ΔRIB,c for three bands	789

7.3A.3.2.4	ΔRIB,c for four bands	793

7.3A.3.2.5	ΔRIB,c for five bands	796

7.3A.3.2.6	ΔRIB,c for six bands	797

7.3A.3.3	ΔRIB,c for Intra-band CA	797

7.3A.4	Reference sensitivity exceptions due to harmonic interference for CA	797

7.3A.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	810

7.3A.6	Reference sensitivity exceptions due to cross band isolation for CA	856

7.3A.7	Lower-MSD requirements for inter-band CA	861

7.3B	Reference sensitivity for NR-DC	862

7.3C	Reference sensitivity for SUL	862

7.3C.1	General	862

7.3C.2	Reference sensitivity power level for SUL	862

7.3C.3	ΔRIB,c for SUL	865

7.3C.3.1	General	865

7.3C.3.2	SUL band combination	866

7.3C.3.2.1	ΔRIB,c  for two bands	866

7.3C.3.2.2	ΔRIB,c  for three bands	866

7.3C.3.2.3	ΔRIB,c  for four bands	867

7.3D	Reference sensitivity for UL MIMO	867

7.3E	Reference sensitivity for V2X	867

7.3E.1	General	867

7.3E.2	Minimum requirements	868

7.3E.2A	Minimum requirements for Sidelink CA	868

7.3E.2A.1	Reference sensitivity power level for Sidelink CA	868

7.3E.2F	Minimum requirements for Sidelink Unlicensed	869

7.3E.2F.1	General	869

7.3E.2F.2	Reference sensitivity power level	869

7.3E.3	Reference sensitivity power level for V2X concurrent operation	870

7.3E.3.1	General	870

7.3E.3F	Minimum requirements for SL-U concurrent operation	871

7.3E.3F.1	Reference sensitivity power level for SL-U concurrent operation	871

7.3F	Reference sensitivity for shared spectrum channel access	871

7.3F.1	General	871

7.3F.2	Reference sensitivity power level	871

7.3F.3	Void	872

7.3F.4	Void	872

7.3F.4A	Shared spectrum channel access CA	872

7.3F.4A.1	Intra-band contiguous shared spectrum channel access CA	872

7.3F.5	Void	872

7.3F.5.1	Void	872

7.3F.5.2	Void	872

7.3F.5.3	Void	872

7.3G	Reference sensitivity for Tx Diversity	872

7.3G.5	Void	873

7.3G.5.0	Void	873

7.3H	(Reserved)	873

7.3I	Reference sensitivity for (e)RedCap	873

7.3I.1	General	873

7.3I.2	Reference sensitivity power level for RedCap	873

7.3I.3	Reference sensitivity power level for eRedCap	877

7.3J	Reference sensitivity for ATG	877

7.3J.1	General	877

7.3J.2	Reference sensitivity power level	878

7.3K	(Reserved)	878

7.3L	(Reserved)	878

7.4	Maximum input level	878

7.4A	Maximum input level for CA	878

7.4A.1	Maximum input level for Intra-band contiguous CA	878

7.4A.2	Maximum input level for Intra-band non-contiguous CA	879

7.4A.3	Maximum input level for Inter-band CA	879

7.4B	Maximum input level for NR-DC	879

7.4D	Maximum input level for UL MIMO	879

7.4E	Maximum input level for V2X	880

7.4E.1	General	880

7.4E.1A	Maximum input level for Sidelink CA	880

7.4E.1F	General requirement for Sidelink Unlicensed	880

7.4E.2	Maximum input level for V2X concurrent operation	880

7.4E.2F	Maximum input level for SL-U concurrent operation	881

7.4F	(Reserved)	881

7.4G	(Reserved)	881

7.4H	(Reserved)	881

7.4I	(Reserved)	881

7.4J	Maximum input level for ATG	881

7.4K	(Reserved)	881

7.4L	(Reserved)	881

7.5	Adjacent channel selectivity	881

7.5A	Adjacent channel selectivity for CA	884

7.5A.1	Adjacent channel selectivity for Intra-band contiguous CA	884

7.5A.2	Adjacent channel selectivity Intra-band non-contiguous CA	886

7.5A.3	Adjacent channel selectivity Inter-band CA	886

7.5B	Adjacent channel selectivity for NR-DC	886

7.5D	Adjacent channel selectivity for UL MIMO	886

7.5E	Adjacent channel selectivity for V2X	887

7.5E.1	General	887

7.5E.1A	Adjacent channel selectivity requirement for Sidelink CA	888

7.5E.1F	General requirement for Sidelink Unlicensed	889

7.5E.2	Adjacent channel selectivity for V2X concurrent operation	889

7.5E.2F	Adjacent channel selectivity for SL-U concurrent operation	889

7.5F	Adjacent channel selectivity for shared spectrum channel access	889

7.5F.1	General	889

7.5F.1A	Adjacent channel selectivity for shared spectrum channel access CA	890

7.5F.1A.1	Intra-band contiguous shared spectrum channel access CA	890

7.5F.2	Void	891

7.5G	(Reserved)	891

7.5H	(Reserved)	891

7.5I	(Reserved)	891

7.5J	Adjacent channel selectivity for ATG	891

7.5K	(Reserved)	893

7.5L	(Reserved)	893

7.6	Blocking characteristics	893

7.6.1	General	893

7.6.2	In-band blocking	893

7.6.3	Out-of-band blocking	896

7.6.4	Narrow band blocking	899

7.6A	Blocking characteristics for CA	901

7.6A.1	General	901

7.6A.2	In-band blocking for CA	901

7.6A.2.1	In-band blocking for Intra-band contiguous CA	901

7.6A.2.2	In-band blocking for Intra-band non-contiguous CA	902

7.6A.2.3	In-band blocking for Inter-band CA	902

7.6A.3	Out-of-band blocking for CA	903

7.6A.3.1	Out-of-band blocking for Intra-band contiguous CA	903

7.6A.3.2	Out-of-band blocking for Intra-band non-contiguous CA	904

7.6A.3.3	Out-of-band blocking for Inter-band CA	904

7.6A.4	Narrow band blocking for CA	906

7.6A.4.1	Narrow band blocking for Intra-band contiguous CA	906

7.6A.4.2	Narrow band blocking for Intra-band non-contiguous CA	906

7.6A.4.3	Narrow band blocking for Inter-band CA	907

7.6B	Blocking characteristics for NR-DC	907

7.6C	Blocking characteristics for SUL	907

7.6C.1	General	907

7.6C.2	In-band blocking for SUL	907

7.6C.3	Out-of-band blocking for SUL	907

7.6C.4	Narrow band blocking for SUL	908

7.6D	Blocking characteristics for UL MIMO	908

7.6E	Blocking characteristics for V2X	908

7.6E.1	General	908

7.6E.2	In-band blocking	908

7.6E.2.1	General	908

7.6E.2.1A	In-band blocking for Sidelink CA	909

7.6E.2.2	In-band blocking for V2X concurrent operation	910

7.6E.2.2F	In-band blocking for SL-U concurrent operation	910

7.6E.3	Out-of-band blocking	910

7.6E.3.1	General	910

7.6E.3.1A	Out-of-band blocking for Sidelink CA	911

7.6E.3.2	Out-of-band blocking for V2X concurrent operation	912

7.6E.3.2F	Out-of-band blocking for SL-U concurrent operation	912

7.6E.3F	Out-of-band blocking for Sidelink Unlicensed	912

7.6F	Blocking characteristics for shared spectrum channel access	912

7.6F.1	General	912

7.6F.2	In-band blocking	912

7.6F.2.1	General	912

7.6F.2.2	Void	913

7.6F.2A	In-band blocking for shared spectrum CA	913

7.6F.2A.1	Intra-band contiguous shared spectrum channel access CA	913

7.6F.3	Out-of-band blocking	914

7.6F.3.1	General	914

7.6F.3.2	Void	915

7.6F.3A	Out-of-band blocking for shared spectrum CA	915

7.6F.3A.1	Intra-band contiguous shared spectrum channel access CA	915

7.6G	(Reserved)	915

7.6H	(Reserved)	915

7.6I	Blocking characteristics for RedCap	915

7.6J	Blocking characteristics for ATG	916

7.6J.1	General	916

7.6J.2	In-band blocking for ATG	916

7.6J.3	Out-of-band blocking for ATG	916

7.6K	(Reserved)	916

7.6L	(Reserved)	916

7.7	Spurious response	916

7.7A	Spurious response for CA	917

7.7A.1	Spurious response for Intra-band contiguous CA	917

7.7A.2	Spurious response for Intra-band non-contiguous CA	917

7.7A.3	Spurious response for Inter-band CA	918

7.7B	Spurious response for NR-DC	918

7.7D	Spurious response for UL MIMO	918

7.7E	Spurious response for V2X	918

7.7E.1	General	918

7.7E.1A	Spurious response requirements for Sidelink CA	919

7.7E.1F	General requirement for Sidelink Unlicensed	919

7.7E.2	Spurious response for V2X concurrent operation	919

7.7E.2F	Spurious response for SL-U concurrent operation	919

7.7F	Spurious response for shared spectrum channel access	919

7.7F.1	General	919

7.7F.1A	Spurious response for shared spectrum channel access CA	920

7.7F.1A.1	Intra-band contiguous shared spectrum channel access CA	920

7.7F.2	Void	920

7.7G	(Reserved)	920

7.7H	(Reserved)	920

7.7I	(Reserved)	920

7.7J	Spurious response for ATG	921

7.7K	(Reserved)	921

7.7L	(Reserved)	921

7.8	Intermodulation characteristics	921

7.8.1	General	921

7.8.2	Wide band Intermodulation	921

7.8A	Intermodulation characteristics for CA	923

7.8A.1	General	923

7.8A.2	Wide band intermodulation for CA	923

7.8A.2.1	Wide band intermodulation for Intra-band contiguous CA	923

7.8A.2.2	Wide band intermodulation for Intra-band non-contiguous CA	924

7.8A.2.3	Wide band intermodulation for Inter-band CA	924

7.8B	Intermodulation characteristics for NR-DC	925

7.8D	Intermodulation characteristics for UL MIMO	925

7.8E	Intermodulation characteristics for V2X	925

7.8E.1	General	925

7.8E.2	Wide band Intermodulation	925

7.8E.2.1	General	925

7.8E.2.2	Wide band Intermodulation for V2X concurrent operation	926

7.8E.2.2A	Wide band intermodulation for Sidelink CA	926

7.8E.2.2F	Wide band Intermodulation for SL-U concurrent operation	926

7.8E.2F	Wide band Intermodulation for Sidelink Unlicensed	927

7.8F	Intermodulation characteristics for shared spectrum channel access	927

7.8F.1	General	927

7.8F.2	Wide band Intermodulation	927

7.8G	(Reserved)	927

7.8H	(Reserved)	927

7.8I	(Reserved)	928

7.8J	Intermodulation characteristics for ATG	928

7.8J.1	General	928

7.8J.2	Wide band intermodulation for ATG	928

7.8K	(Reserved)	928

7.8L	(Reserved)	928

7.9	Spurious emissions	928

7.9A	Spurious emissions for CA	928

7.9A.1	Void	928

7.9A.2	Void	928

7.9A.3	Spurious emissions for Inter-band CA	928

7.9B	Spurious emissions for NR-DC	929

7.9J	Spurious emissions for ATG	929

7.10	Power imbalance	929

7.10A	Power imbalance for CA	929

7.10A.1	General	929

7.10A.2	Minimum requirement	929

Annex A (normative): Measurement channels	930

A.1	General	930

A.2	UL reference measurement channels	930

A.2.1	General	930

A.2.2	Reference measurement channels	932

A.2.2.1	DFT-s-OFDM Pi/2-BPSK	932

A.2.2.2	DFT-s-OFDM QPSK	934

A.2.2.3	DFT-s-OFDM 16QAM	935

A.2.2.4	DFT-s-OFDM 64QAM	936

A.2.2.5	DFT-s-OFDM 256QAM	938

A.2.2.6	CP-OFDM QPSK	939

A.2.2.7	CP-OFDM 16QAM	941

A.2.2.8	CP-OFDM 64QAM	943

A.2.2.9	CP-OFDM 256QAM	945

A.2.3	Reference measurement channels for TDD	948

A.2.3.1	DFT-s-OFDM Pi/2-BPSK	948

A.2.3.2	DFT-s-OFDM QPSK	948

A.2.3.3	DFT-s-OFDM 16QAM	948

A.2.3.4	DFT-s-OFDM 64QAM	948

A.2.3.5	DFT-s-OFDM 256QAM	948

A.2.3.6	CP-OFDM QPSK	949

A.2.3.7	CP-OFDM 16QAM	949

A.2.3.8	CP-OFDM 64QAM	949

A.2.3.9	CP-OFDM 256QAM	949

A.3	DL reference measurement channels	949

A.3.1	General	949

A.3.2	DL reference measurement channels for FDD	952

A.3.2.1	General	952

A.3.2.2	FRC for receiver requirements for QPSK	953

A.3.2.3	FRC for maximum input level for 64QAM	956

A.3.2.4	FRC for maximum input level for 256 QAM	959

A.3.2.5	FRC for maximum input level for 1024 QAM	962

A.3.3	DL reference measurement channels for TDD	965

A.3.3.1	General	965

A.3.3.2	FRC for receiver requirements for QPSK	966

A.3.3.3	FRC for maximum input level for 64QAM	969

A.3.3.4	FRC for maximum input level for 256 QAM	972

A.3.3.5	FRC for maximum input level for 1024 QAM	975

A.4	CSI reference measurement channels	978

A.5	OFDMA Channel Noise Generator (OCNG)	978

A.5.1	OCNG Patterns for FDD	978

A.5.1.1	OCNG FDD pattern 1: Generic OCNG FDD Pattern for all unused REs	978

A.5.2	OCNG Patterns for TDD	978

A.5.2.1	OCNG TDD pattern 1: Generic OCNG TDD Pattern for all unused REs	978

A.6	Void	979

A.7	V2X reference measurement channels	979

A.7.1	General	979

A.7.2	FRC for V2X receiver requirements for QPSK	979

A.7.3	FRC for maximum input level for 64QAM	981

A.7.4	FRC for maximum input level for 256QAM	982

Annex B (informative): Void	985

Annex C (informative): Downlink physical channels	986

C.1	General	986

C.2	Setup	986

C.3	Connection	986

C.3.1	Measurement of Receiver Characteristics	986

Annex D (normative): Characteristics of the interfering signal	987

D.1	General	987

D.2	Void	987

Annex E (normative):  Environmental conditions	988

E.1	General	988

E.2	Environmental	988

E.2.1	Temperature	988

E.2.2	Voltage	988

E.2.3	Vibration	989

Annex F (normative):  Transmit modulation	990

F.0	General	990

F.1	Measurement Point	990

F.2	Basic Error Vector Magnitude measurement	990

F.3	Basic in-band emissions measurement	991

F.4	Modified signal under test	992

F.5	Window length	994

F.5.1	Timing offset	994

F.5.2	Window length	994

F.5.3	Window length for normal CP	994

F.5.4	Window length for Extended CP	995

F.5.5	Window length for PRACH	996

F.6	Averaged EVM	997

F.7	Spectrum Flatness	998

F.8	EVM measurement for multiple Tx	998

F.9	Phase offset measurement for DMRS bundling	998

F.9.1	Measurement point	998

F.9.2	Symbols used	998

F.9.3	Modified test signal	998

F.9.4	Phase offset measurement	999

F.10	EVM for UL MIMO	999

F10.1	General	999

F10.2	MIMO Equalization	1000

F10.3	Layer processing	1001

Annex G (normative):  Difference of relative phase and power errors	1002

G.0	General	1002

G.1	Measurement Point	1002

G.2	Relative Phase Error Measurement	1002

G.2.1	Symbols and subcarriers used	1002

G.2.2	CFO (carrier frequency offset) correction	1003

G.2.3	Steps of the measurement method	1003

Annex H (informative): Void	1004

Annex I (informative): Void	1005

Annex J (informative): Void	1006

Annex K (informative): Void	1007

Annex L (normative): ModifiedMPR-Behavior	1008

L.1	Indication of modified MPR behavior	1008

Annex M (normative):  Declared Supported Post Antenna Gain for UE	1011

M.1	FRMCS operating bands	1011

Annex N (informative): Change history	1012