Annex A (normative):
RRM test configurations

# A.1	Reference measurement channels

## A.1.0	General

This clause contains the Reference Measurement Channels (RMC) to be used for the RRM test scenarios in Clauses 4 to 18 of this document.

## A.1.1	PDSCH

### A.1.1.1	FDD

Table A.1.1.1-1: PDSCH Reference Measurement Channels for SCS = 15 kHz for FDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | SR.1.1 FDD | SR.1.2 FDD |
| Channel bandwidth | MHz | Defined in test case | Defined in test case by the parameter NRB,c |
| Number of transmitter antennas |  | 1 | 1 |
| Allocated resource blocks for PDSCH Note 1 |  | 24 | Defined by test case |
| Allocated slots per Radio Frame |  | 10 | 10 |
| Radio frame containing SSB | slots | Note 5 | Note 5 |
| Radio frame not containing SSB | slots | 10 | 10 |
| MCS index |  | 4 | 4 |
| Modulation |  | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 |
| Number of control symbols |  | 2 | 2 |
| PDSCH mapping type |  | Type A | Type A |
| Information Bit Payload |  |  |  |
| For slots with RMSI Note 2 | Bits | 1608 | 1608 |
| For slots without RMSI | Bits | 1864 | 1864 |
| Number of Code Blocks per slot |  | 1 | 1 |
| Binary Channel Bits Per slot |  |  |  |
| For slots with RMSI Note 2, 4 | Bits | 5184 | 5184 |
| For slots without RMSI Note 6 | Bits | 6048 | 6048 |
| NOTE 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. NOTE 2: PDSCH is scheduled on the slots with RMSI. NOTE 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. NOTE 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. NOTE 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3. NOTE 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. NOTE 7: When DRX is configured, PDCCH can be scheduled both for downlink assignment and/or UL grant only during ([10ms] – drx-InactivityTimer) from timing when drx-onDurationTimer starts, unless otherwise specified in the test case. |  |  |  |

### A.1.1.2	TDD

Table A.1.1.2-1: PDSCH Reference Measurement Channels for SCS = 15 kHz for TDD

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | SR.1.1 TDD | SR.1.2 TDD |
| Channel bandwidth | MHz | Defined in test case | Defined in test case |
| Number of transmitter antennas |  | 1 | 1 |
| Allocated resource blocks for PDSCH Note 1 |  | 24 | 24 |
| Allocated slots per Radio Frame |  |  |  |
| Radio frame containing SSB | slots | Note 5 | Note 5 |
| Radio frame not containing SSB | slots | 4 | 6 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 4 | 4 |
| Modulation |  | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 |
| Number of control symbols |  | 2 | 2 |
| PDSCH mapping type |  | Type A | Type A |
| Information Bit Payload |  |  |  |
| For slots with RMSI Note 2 | Bits | 1608 | 1608 |
| For slots without RMSI | Bits | 1864 | 1864 |
| For special slots | Bits | N/A | 1128 |
| Number of Code Blocks per slot |  | 1 |  |
| Binary Channel Bits Per slot |  |  |  |
| For slots with RMSI Note 2, 4 | Bits | 5184 | 5184 |
| For slots without RMSI Note 6 | Bits | 6048 | 6048 |
| For special slots Note 6 | Bits | - | 3744 |
| NOTE 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. NOTE 2: PDSCH is scheduled on the slots with RMSI. NOTE 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. NOTE 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. NOTE 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. NOTE 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. NOTE 7: When DRX is configured, PDCCH can be scheduled both for downlink assignment and/or UL grant only during ([10]ms – drx-InactivityTimer) from timing when drx-onDurationTimer starts, unless otherwise specified in the test case. |  |  |  |

Table A.1.1.2-2: PDSCH Reference Measurement Channels for SCS = 30 kHz for TDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | SR.2.1 TDD |
| Channel bandwidth | MHz | Defined in test case |
| Number of transmitter antennas |  | 1 |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |
| Allocated slots per Radio Frame |  |  |
| Radio frame containing SSB | slots | Note 5 |
| Radio frame not containing SSB | slots | 10 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of control symbols |  | 2 |
| PDSCH mapping type |  | Type A |
| Information Bit Payload |  |  |
| For slots with RMSI Note 2 | Bits | 1608 |
| For slots without RMSI | Bits | 1864 |
| Number of Code Blocks per slot |  | 1 |
| Binary Channel Bits Per slot |  |  |
| For slots with RMSI Note 2, 4 | Bits | 5184 |
| For slots without RMSI Note 6 | Bits | 6048 |
| NOTE 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. NOTE 2: PDSCH is scheduled on the slots with RMSI. NOTE 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. NOTE 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. NOTE 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. NOTE 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. NOTE 7: When DRX is configured, PDCCH can be scheduled both for downlink assignment and/or UL grant only during ([10]ms – drxInactivityTimer) from timing when drx-onDurationTimer starts, unless otherwise specified in the test case. |  |  |

Table A.1.1.2-3: PDSCH Reference Measurement Channels for SCS = 120 kHz for TDD

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | SR.3.1 TDD | SR.3.2 TDD | SR.3.3 TDD |
| Channel bandwidth | MHz | 100 | 100 | 100 |
| Number of transmitter antennas |  | 1 | 1 | 1 |
| Allocated resource blocks for PDSCH |  | 24Note 1 | 24Note 7 | 48Note 7 |
| Allocated slots per Radio Frame |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 | Note 8 | Note 5 |
| Radio frame not containing SSB | slots | 48 | 48 | 48 |
| MCS table |  | 64QAM | 64QAM | 64QAM |
| MCS index |  | 4 | 4 | 4 |
| Modulation |  | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 |
| Number of control symbols |  | 2 | 2 | 2 |
| PDSCH mapping type |  | Type A | Type A | Type A |
| Information Bit Payload |  |  |  |  |
| For slots with RMSI | Bits | 1608 | 1608 | 3104 |
| For slots without RMSI | Bits | 1864 | 1864 | 3624 |
| Number of Code Blocks per slot |  | 1 | 1 | 1 |
| Binary Channel Bits Per slot |  |  |  |  |
| For slots with RMSI Note 4 | Bits | 5184 | 5184 | 10368 |
| For slots without RMSI Note 6 | Bits | 6048 | 6048 | 12096 |
| NOTE 1: Allocated in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. NOTE 2: Void NOTE 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. NOTE 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. NOTE 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. NOTE 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditionalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. NOTE 7: Allocated in the same resource blocks as the CORESET. NOTE 8: When DRX is configured, PDSCH is scheduled only while drx-onDurationTimer is running, unless otherwise specified in the test case. |  |  |  |  |

## A.1.2	CORESET for RMSI scheduling

### A.1.2.1	FDD

Table A.1.2.1-1: RMSI CORESET Reference Measurement Channels for SCS = 15 kHz for FDD

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | CR.1.1 FDD | CR.1.2 FDD | CR.1.3 FDD | CR.1.4 FDD |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case |
| Subcarrier spacing for RMSI CORESET | kHz | 15 | 15 | 15 | 15 |
| Allocated resource blocks for RMSI CORESET |  | 24 (NOTE 8) | 12 (NOTE 9) | 15 (NOTE 10) | 20 (NOTE 11) |
| Subcarrier spacing for SSB | kHz | 15 | 15 | 15 | 15 |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1 | Pattern 1 | Pattern 1 | Pattern 1 |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (NOTE 8) | 0 (NOTE 9) | 0 (NOTE 10) | 0 (NOTE 11) |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 | Index 4 | Index 4 | Index 4 |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 |
| Duration of RMSI CORESET | symbols | 2 (NOTE 8) | 2 (NOTE 9) | 3 (NOTE 10) | 3 (NOTE 11) |
| DCI Format Note 1 |  | Note 2 | Note 2 | Note 2 | Note 2 |
| Aggregation level | CCE | 8 | 4 | 8 | 8 |
| DMRS precoder granularity |  | 6 | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| REG bundle size |  | 6 | 6 | 6 | 6 |
| Mapping from REG to CCE |  | Distributed (NOTE 8) | Distributed (NOTE 9) | Non-Distributed (NOTE 10) | Distributed (NOTE 11) |
| Cell ID |  | Note 5 | Note 5 | Note 5 | Note 5 |
| Payload (without CRC) | Bits | Note 6 | Note 6 | Note 6 | Note 6 |
| NOTE 1: DCI formats are defined in TS 38.212 [31]. NOTE 2: DCI format shall depend upon the test configuration. NOTE 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. NOTE 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [8]. NOTE 5: Cell ID shall depend upon the test configuration. NOTE 6: Payload size shall depend upon the test configuration. NOTE 7: Other values can be used to align with GSCN as long as SSB does not overlap the RMC NOTE 8: The configuration of set of resource blocks, slot symbols, RB offset and REG to CCE mapping of control resource set for Type0-PDCCH search space corresponds to index 0 in table 13-1 in TS 38.213 [3] NOTE 9: The configuration of set of resource blocks, slot symbols, RB offset and REG to CCE mapping of control resource set for Type0-PDCCH search space corresponds to index 0 in table 13-0 in TS 38.213 [3] NOTE 10: The configuration of set of resource blocks, slot symbols, RB offset and REG to CCE mapping of control resource set for Type0-PDCCH search space corresponds to index 8 in table 13-0 in TS 38.213 [3] NOTE 11: The configuration of set of resource blocks, slot symbols, RB offset and REG to CCE mapping of control resource set for Type0-PDCCH search space corresponds to index 11 in table 13-0 in TS 38.213 [3] |  |  |  |  |  |

### A.1.2.2	TDD

Table A.1.2.2-1: RMSI CORESET Reference Measurement Channels for SCS = 15 kHz for TDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | CR.1.1 TDD |
| Channel bandwidth | MHz | Defined in test case |
| Subcarrier spacing for RMSI CORESET | kHz | 15 |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 24 |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1 |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 |
| Number of transmitter antennas |  | 1 |
| Duration of RMSI CORESET Note 7 | symbols | 2 |
| DCI Format Note 1 |  | Note 2 |
| Aggregation level | CCE | 8 |
| DMRS precoder granularity |  | 6 |
| REG bundle size |  | 6 |
| Mapping from REG to CCE |  | Distributed |
| Cell ID |  | Note 5 |
| Payload (without CRC) | Bits | Note 6 |
| NOTE 1: DCI formats are defined in TS 38.212 [31]. NOTE 2: DCI format shall depend upon the test configuration. NOTE 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. NOTE 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [8]. NOTE 5: Cell ID shall depend upon the test configuration. NOTE 6: Payload size shall depend upon the test configuration. NOTE 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-1 in TS 38.213 [8]. NOTE 8: Other values can be used to align with GSCN as long as SSB does not overlap the RMC |  |  |

Table A.1.2.2-2: RMSI CORESET Reference Measurement Channels for SCS = 30 kHz for TDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | CR.2.1 TDD |
| Channel bandwidth | MHz | Defined in test case |
| Subcarrier spacing for RMSI CORESET | kHz | 30 |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 24 |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1 |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 |
| Number of transmitter antennas |  | 1 |
| Duration of RMSI CORESET Note 7 | symbols | 2 |
| DCI Format Note 1 |  | Note 2 |
| Aggregation level | CCE | 8 |
| DMRS precoder granularity |  | 6 |
| REG bundle size |  | 6 |
| Mapping from REG to CCE |  | Distributed |
| Cell ID |  | Note 5 |
| Payload (without CRC) | Bits | Note 6 |
| NOTE 1: DCI formats are defined in TS 38.212 [31]. NOTE 2: DCI format shall depend upon the test configuration. NOTE 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. NOTE 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [8]. NOTE 5: Cell ID shall depend upon the test configuration. NOTE 6: Payload size shall depend upon the test configuration. NOTE 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-6 in TS 38.213 [8]. NOTE 8: Other values can be used to align with GSCN as long as SSB does not overlap the RMC |  |  |

Table A.1.2.2-3: RMSI CORESET Reference Measurement Channels for SCS = 120 kHz for TDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | CR.3.1 TDD | CR.3.2 TDD |
| Channel bandwidth | MHz | 100 | 100 |
| Subcarrier spacing for RMSI CORESET | kHz | 120 | 120 |
| Allocated resource blocks for RMSI CORESET |  | 24Note 7 | 48Note 9 |
| Subcarrier spacing for SSB | kHz | 120 | 120 |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1Note 7 | Pattern 1Note 9 |
| Offset between SSB and RMSI CORESET Note 3 | RB | 0 (Note 8) Note 7 | 0 (Note 8)Note 9 |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 | Index 4 |
| Number of transmitter antennas |  | 1 | 1 |
| Duration of RMSI CORESET | symbols | 2Note 7 | 2Note 9 |
| DCI Format Note 1 |  | Note 2 | Note 2 |
| Aggregation level | CCE | 8 | 8 |
| DMRS precoder granularity |  | 6 | 6 |
| REG bundle size |  | 6 | 6 |
| Mapping from REG to CCE |  | Distributed | Distributed |
| Cell ID |  | Note 5 | Note 5 |
| Payload (without CRC) | Bits | Note 6 | Note 6 |
| NOTE 1: DCI formats are defined in TS 38.212 [31]. NOTE 2: DCI format shall depend upon the test configuration. NOTE 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. NOTE 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [8]. NOTE 5: Cell ID shall depend upon the test configuration. NOTE 6: Payload size shall depend upon the test configuration. NOTE 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-8 in TS 38.213 [8]. NOTE 8: Other values can be used to align with GSCN as long as SSB does not overlap the RMC NOTE 9: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 2 in Table 13-10 in TS 38.213 [8]. |  |  |  |

## A.1.3	CORESET for RMC scheduling

### A.1.3.1	FDD

Table A.1.3.1-1: Control Channel RMC for SCS = 15 kHz for FDD

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 FDD | CCR.1.2 FDD | CCR.1.3 FDD | CCR.1.4 FDD | CCR.1.5 FDD | CCR.1.6 FDD | CCR.1.7 FDD | CCR.1.8 FDD |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case | 10 | 3MHz | 3MHz | 5MHz |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 18 | 24 | 18 | 24 | 12 | 12 | 18 |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 |
| monitoringSymbolsWithinSlot |  | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 | 0010000 0000000 | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 |
| REG bundle size |  | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Non-Interleaved | Interleaved |
| Interleave n_shift |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Interleave size |  | 2 | 2 | 2 | 2 | 2 | 2 | N/A | 2 |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Aggregation level | CCE | 4 | 2 | 8 | 4 | 4 | 4 | 4 | 8 |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | 1-0 | 1-0 | 1-0 |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |  |

Table A.1.3.1-2: FFS

Table A.1.3.1-3: Control Channel RMC for SCS = 15 kHz for FDD

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.3.1 FDD | CCR.3.2 FDD |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 48 | 48 |  |  |  |
| Number of transmitter antennas |  | 1 | 1 |  |  |  |
| Duration of CORESET | symbols | 2 | 2 |  |  |  |
| monitoringSymbolsWithinSlot |  | 1000000 0000000 | 1000000 0000000 |  |  |  |
| REG bundle size |  | 6 | 6 |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size |  |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved |  |  |  |
| Interleave n_shift |  | 0 | 0 |  |  |  |
| Interleave size |  | 2 | 2 |  |  |  |
| Beamforming Pre-Coder |  | N/A | N/A |  |  |  |
| Aggregation level | CCE | 8 | 16 |  |  |  |
| DCI formats |  | Note 1 | Note 1 |  |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |

### A.1.3.2	TDD

Table A.1.3.2-1: Control Channel RMC for SCS = 15 kHz for TDD

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 TDD | CCR.1.2 TDD | CCR.1.3 TDD | CCR.1.4 TDD | CCR.1.5 TDD |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case | 10 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 18 | 24 | 18 | 18 |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 | 1 |  |  |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 | 2 |  |  |
| monitoringSymbolsWithinSlot |  | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 | 0010000 0000000 |  |  |
| REG bundle size |  | 6 | 6 | 6 | 6 | 6 |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved |  |  |
| Interleave n_shift |  | 0 | 0 | 0 | 0 | 0 |  |  |
| Interleave size |  | 2 | 2 | 2 | 2 | 2 |  |  |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A | N/A |  |  |
| Aggregation level | CCE | 4 | 2 | 8 | 4 | 4 |  |  |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

Table A.1.3.2-2: Control Channel RMC for SCS = 30 kHz for TDD

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.2.1 TDD | CCR.2.2 TDD | CCR.2.3 TDD | CCR.2.4 TDD |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case |
| Subcarrier spacing for RMSI CORESET | kHz | 30 | 30 | 30 | 30 |
| Allocated resource blocks for CORESET3 |  | 24 | 24 | 18 | 18 |
| Number of transmitter antenna |  | 1 | 1 | 1 | 1 |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 |
| REG bundle size |  | 6 | 6 | 6 | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved |
| Interleave n_shift |  | 0 | 0 | 0 | 0 |
| Interleave size |  | 2 | 2 | 2 | 2 |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A |
| Aggregation level | CCE | 4 | 8 | 4 | 2 |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated RMC is scheduled. |  |  |  |  |  |

Table A.1.3.2-3: Control Channel RMC for SCS = 120 kHz for TDD

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.3.1 TDD | CCR.3.2 TDD | CCR.3.3 TDD | CCR.3.4 TDD | CCR.3.5 TDD | CCR.3.6 TDD | CCR.3.7 TDD |
| Channel bandwidth | MHz | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| Subcarrier spacing for RMSI CORESET | kHz | 120 | 120 | 120 | 120 | 120 | 120 | 120 |
| Allocated resource blocks for CORESETNote 3 |  | 24 | 24 | 24 | 24 | 24 | 24 | 48 |
| Number of transmitter antenna |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| monitoringSlotPeriodicityAndOffsetNote 4 |  | sl160 0 | sl160 0 | sl160 80 | sl160 0 | sl160 0 | sl160 80 | sl160 0 |
| monitoringSymbolsWithinSlot |  | 1100000 0000000 | 0011000 0000000 | 1100000 0000000 | 1000000 0000000 | 0001000 0000000 | 1000000 0000000 | 1100000 0000000 |
| Duration of CORESET | symbols | 1 | 1 | 1 | 2 | 2 | 2 | 1 |
| REG bundle size |  | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved |
| Interleave n_shift |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Interleave size |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Aggregation level | CCE | 4 | 4 | 4 | 8 | 8 | 8 | 4 |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: DCI format shall depend upon the test configuration. NOTE 2: Payload size shall depend upon the test configuration NOTE 3: Allocated in the same resource blocks where the associated PDSCH RMC is scheduled. NOTE 4: monitoringSlotPeriodicityAndOffet can be set to other values depending on test purpose of respective test cases, as needed, e.g. monitoringSlotPeriodicityAndOffet is set to “sl1 0” if it is specifically stated that cell(s) configured with one of the control channel RMCs above shall transmit PDCCHs continuously. |  |  |  |  |  |  |  |  |

Table A.1.3.2-4: Control Channel RMC for SCS = 15 kHz for TDD

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.4.1 TDD | CCR.4.2 TDD |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 48 | 48 |  |  |  |
| Number of transmitter antennas |  | 1 | 1 |  |  |  |
| Duration of CORESET | symbols | 2 | 2 |  |  |  |
| monitoringSymbolsWithinSlot |  | 1000000 0000000 | 1000000 0000000 |  |  |  |
| REG bundle size |  | 6 | 6 |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size |  |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved |  |  |  |
| Interleave n_shift |  | 0 | 0 |  |  |  |
| Interleave size |  | 2 | 2 |  |  |  |
| Beamforming Pre-Coder |  | N/A | N/A |  |  |  |
| Aggregation level | CCE | 8 | 16 |  |  |  |
| DCI formats |  | Note 1 | Note 1 |  |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |

Table A.1.3.2-5: Control Channel RMC for SCS = 30 kHz for TDD

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.5.1 TDD | CCR.5.2 TDD |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case |  |  |  |
| Subcarrier spacing | kHz | 30 | 30 |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 48 | 48 |  |  |  |
| Number of transmitter antennas |  | 1 | 1 |  |  |  |
| Duration of CORESET | symbols | 2 | 2 |  |  |  |
| REG bundle size |  | 6 | 6 |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size |  |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved |  |  |  |
| Interleave n_shift |  | 0 | 0 |  |  |  |
| Interleave size |  | 2 | 2 |  |  |  |
| Beamforming Pre-Coder |  | N/A | N/A |  |  |  |
| Aggregation level | CCE | 8 | 16 |  |  |  |
| DCI formats |  | Note 1 | Note 1 |  |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |

## A.1.4	CSI-RS

### A.1.4.1	FDD

Table A.1.4.1-1: CSI-RS Reference Measurement Channels for SCS = 15 kHz for FDD

|  | CSI-RS.1.1 FDD | CSI-RS.1.2 FDD | CSI-RS.1.3 FDD | CSI-RS.1.4 FDD | CSI-RS.1.5 FDD | CSI-RS.1.6 FDD | CSI-RS.1.7 FDD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic | aperiodic | periodic | periodic |
| Resource Set Config |  |  |  |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| repetition | n.a. | off | off | on | off | n.a. | off |
| aperiodicTriggeringOffset | n.a. | n.a. | 0 | 0 | 6 | n.a. | n.a. |
| trs-Info | n.a. | n.a. | n.a. | n.a. | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |  |  |  |  |
|  |  | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |  | 2 for resource #0 |
|  |  |  |  | 1 for resource #1 |  |  |  |
|  |  |  |  | 2 for resource #2 |  |  |  |
|  |  |  |  | 3 for resource #3 |  |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 | 1 for resource #1 | 0 for resource #0 | 3 for resource #1 |
|  |  |  |  | 5 for resource #5 |  |  |  |
|  |  |  |  | 6 for resource #6 |  |  |  |
|  |  |  |  | 7 for resource #7 |  |  |  |
| powerControlOffset | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Period (slots) | slot5 | slot10 | n.a. | n.a. | n.a. | slot40 | slot10 |
| Offset | 1 | 1 | n.a. | n.a. | n.a. | 1 | 1 |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | n.a. | n.a. | n.a. | TCI.State.0 | TCI.State.0 |
|  |  | TCI.State.1 |  |  |  |  | TCI.State.1 |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 | 000001 | 000001 | 0100 |
| nrofPorts | 2 | 1 | 1 | 1 | 1 | 2 | 1 |
|  |  | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 | Specified in the test case for resource #0 |  | 6 for resource #0 |
|  |  |  |  | 1 for resource #1 |  |  |  |
|  |  |  |  | 2 for resource #2 |  |  |  |
|  |  |  |  | 3 for resource #3 |  |  |  |
| firstOFDMSymbolInTimeDomain | 4 for resource #0 | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 | n.a. | 5 for resource #0 | 10 for resource #1 |
|  |  |  |  | 5 for resource #5 |  |  |  |
|  |  |  |  | 6 for resource #6 |  |  |  |
|  |  |  |  | 7 for resource #7 |  |  |  |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM | noCDM | FD-CDM2 | noCDM |
| density | 1 | 3 | 3 | 3 | 3 | 1 | 3 |
| startingRB | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |  |  |  |  |

Table A.1.4.1-1A: CSI-RS Reference Measurement Channels for SCS=15 kHz for FDD

|  | CSI-RS.1.1A FDD | CSI-RS.1.2A FDD | CSI-RS.1.3A FDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| nzp-CSI-ResourceSetId | 1 | 1 | 1 |
| repetition | off | off | off |
| aperiodicTriggeringOffset | n.a. | 0 | n.a. |
| trs-Info | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |
| nzp-CSI-RS-ResourceId | 12 for resource #0 | 22 for resource #0 | 14 for resource #0 |
|  | 13 for resource #1 | 23 for resource #1 | 15 for resource #1 |
| powerControlOffset | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 |
| Period (slots) | slot20 | n.a. | slot10 |
| Offset | 1 | n.a. | 2 |
| qcl-InfoPeriodicCSI-RS | n.a. | n.a. | n.a. |
| frequencyDomainAllocation | 0001 | 0001 | 0001 |
| nrofPorts | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| cdm-Type | noCDM | noCDM | noCDM |
| density | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

### A.1.4.2	TDD

Table A.1.4.2-1: CSI-RS Reference Measurement Channels for SCS = 15 kHz for TDD

|  | CSI-RS.1.1 TDD | CSI-RS.1.2 TDD | CSI-RS.1.3 TDD | CSI-RS.1.4 TDD | CSI-RS.1.5 TDD | CSI-RS.1.6 TDD |
| --- | --- | --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic | periodic | periodic |
| Resource Set Config |  |  |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 | 0 | 0 |
| repetition | n.a. | off | off | on | n.a. | off |
| aperiodicTriggeringOffset | n.a. | n.a. | 0 | 0 | n.a. | n.a. |
| trs-Info | n.a. | n.a. | n.a. | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |  |  |  |
|  |  | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |  | 2 for resource #0 |
|  |  |  |  | 1 for resource #1 |  |  |
|  |  |  |  | 2 for resource #2 |  |  |
|  |  |  |  | 3 for resource #3 |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 | 0 for resource #0 | 3 for resource #1 |
|  |  |  |  | 5 for resource #5 |  |  |
|  |  |  |  | 6 for resource #6 |  |  |
|  |  |  |  | 7 for resource #7 |  |  |
| powerControlOffset | 0 | 0 | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 | 0 | 0 |
| Period (slots) | slot5 | slot10 | n.a. | n.a. | slot40 | slot10 |
| Offset | 1 | 1 | n.a. | n.a. | 1 | 1 |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | n.a. | n.a. | TCI.State.0 | TCI.State.0 |
|  |  | TCI.State.1 |  |  |  | TCI.State.1 |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 | 000001 | 0100 |
| nrofPorts | 2 | 1 | 1 | 1 | 2 | 1 |
|  |  | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 |  | 6 for resource #0 |
|  |  |  |  | 1 for resource #1 |  |  |
|  |  |  |  | 2 for resource #2 |  |  |
|  |  |  |  | 3 for resource #3 |  |  |
| firstOFDMSymbolInTimeDomain | 4 for resource #0 | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 | 5 for resource #0 | 10 for resource #1 |
|  |  |  |  | 5 for resource #5 |  |  |
|  |  |  |  | 6 for resource #6 |  |  |
|  |  |  |  | 7 for resource #7 |  |  |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM | FD-CDM2 | noCDM |
| density | 1 | 3 | 3 | 3 | 1 | 3 |
| startingRB | 0 | 0 | 0 | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |  |  |  |

Table A.1.4.2-1A: CSI-RS Reference Measurement Channels for SCS=15 kHz for TDD

|  | CSI-RS.1.1A TDD | CSI-RS.1.2A TDD | CSI-RS.1.3A TDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| nzp-CSI-ResourceSetId | 1 | 1 | 1 |
| repetition | off | off | off |
| aperiodicTriggeringOffset | n.a. | 0 | n.a. |
| trs-Info | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |
| nzp-CSI-RS-ResourceId | 12 for resource #0 | 22 for resource #0 | 14 for resource #0 |
|  | 13 for resource #1 | 23 for resource #1 | 15 for resource #1 |
| powerControlOffset | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 |
| Period (slots) | slot20 | n.a. | slot10 |
| Offset | 1 | n.a. | 2 |
| qcl-InfoPeriodicCSI-RS | n.a. | n.a. | n.a. |
| frequencyDomainAllocation | 0001 | 0001 | 0001 |
| nrofPorts | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| cdm-Type | noCDM | noCDM | noCDM |
| density | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

Table A.1.4.2-2: CSI-RS Reference Measurement Channels for SCS = 30 kHz for TDD

|  | CSI-RS.2.1 TDD | CSI-RS.2.2 TDD | CSI-RS.2.3 TDD | CSI-RS.2.4 TDD | CSI-RS.2.5 TDD | CSI-RS.2.6 TDD | CSI-RS.2.7 TDD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic | aperiodic | periodic | periodic |
| Resource Set Config |  |  |  |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| repetition | n.a. | off | off | on | off | n.a. | off |
| aperiodicTriggeringOffset | n.a. | n.a. | 0 | 0 | 6 | n.a. | n.a. |
| trs-Info | n.a. | n.a. | n.a. | n.a. | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |  |  |  |  |
|  |  | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |  | 2 for resource #0 |
|  |  |  |  | 1 for resource #1 |  |  |  |
|  |  |  |  | 2 for resource #2 |  |  |  |
|  |  |  |  | 3 for resource #3 |  |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 | 1 for resource #1 | 0 for resource #0 | 3 for resource #1 |
|  |  |  |  | 5 for resource #5 |  |  |  |
|  |  |  |  | 6 for resource #6 |  |  |  |
|  |  |  |  | 7 for resource #7 |  |  |  |
| powerControlOffset | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Period (slots) | slot10 | slot20 | n.a. | n.a. | n.a. | slot80 | slot20 |
| Offset | 2 | 2 | n.a. | n.a. | n.a. | 2 | 2 |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | n.a. | n.a. | n.a. | TCI.State.0 | TCI.State.0 |
|  |  | TCI.State.1 |  |  |  |  | TCI.State.1 |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 | 000001 | 000001 | 0100 |
| nrofPorts | 2 | 1 | 1 | 1 | 1 | 2 | 1 |
|  |  | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 | Specified in the test case for resource #0 |  | 6 for resource #0 |
|  |  |  |  | 1 for resource #1 |  |  |  |
|  |  |  |  | 2 for resource #2 |  |  |  |
|  |  |  |  | 3 for resource #3 |  |  |  |
| firstOFDMSymbolInTimeDomain | 5 for resource #0 | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 | n.a. | 5 for resource #0 | 10 for resource #1 |
|  |  |  |  | 5 for resource #5 |  |  |  |
|  |  |  |  | 6 for resource #6 |  |  |  |
|  |  |  |  | 7 for resource #7 |  |  |  |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM | noCDM | FD-CDM2 | noCDM |
| density | 1 | 3 | 3 | 3 | 3 | 1 | 3 |
| startingRB | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |  |  |  |  |

Table A.1.4.2-2A: CSI-RS Reference Measurement Channels for SCS=30 kHz for TDD

|  | CSI-RS.2.1A TDD | CSI-RS.2.2A TDD | CSI-RS.2.3A TDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| nzp-CSI-ResourceSetId | 1 | 1 | 1 |
| repetition | off | off | off |
| aperiodicTriggeringOffset | n.a. | 0 | n.a. |
| trs-Info | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |
| nzp-CSI-RS-ResourceId | 12 for resource #0 | 22 for resource #0 | 14 for resource #0 |
|  | 13 for resource #1 | 23 for resource #1 | 15 for resource #1 |
| powerControlOffset | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 |
| Period (slots) | slot40 | n.a. | slot20 |
| Offset | 2 | n.a. | 4 |
| qcl-InfoPeriodicCSI-RS | n.a. | n.a. | n.a. |
| frequencyDomainAllocation | 0001 | 0001 | 0001 |
| nrofPorts | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| cdm-Type | noCDM | noCDM | noCDM |
| density | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

Table A.1.4.2-3: CSI-RS Reference Measurement Channels for SCS = 120 kHz for TDD

|  | CSI-RS.3.1 TDD | CSI-RS.3.2 TDD | CSI-RS.3.3 TDD | CSI-RS.3.4 TDD | CSI-RS.3.5 TDD | CSI-RS.3.6 TDD |
| --- | --- | --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic | periodic | periodic |
| Resource Set Config |  |  |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 | 0 | 0 |
| repetition | n.a. | off | off | on | n.a. | off |
| aperiodicTriggeringOffset | n.a. | n.a. | 4 | 4 | n.a. | n.a. |
| trs-Info | n.a. | n.a. | n.a. | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |  |  |  |
|  |  | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |  | 2 for resource #0 |
|  |  |  |  | 1 for resource #1 |  |  |
|  |  |  |  | 2 for resource #2 |  |  |
|  |  |  |  | 3 for resource #3 |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 | 0 for resource #0 | 3 for resource #1 |
|  |  |  |  | 5 for resource #5 |  |  |
|  |  |  |  | 6 for resource #6 |  |  |
|  |  |  |  | 7 for resource #7 |  |  |
| powerControlOffset | 0 | 0 | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 | 0 | 0 |
| Period (slots) | slot40 | slot80 | n.a. | n.a. | slot320 | slot80 |
| Offset | 8 | 16 | n.a. | n.a. | 8 | 16 |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | n.a. | n.a. | TCI.State.0 | TCI.State.0 |
|  |  | TCI.State.1 |  |  |  | TCI.State.1 |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 | 000001 | 0100 |
| nrofPorts | 2 | 1 | 1 | 1 | 1 | 1 |
|  |  | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 |  | 6 for resource #0 |
|  |  |  |  | 1 for resource #1 |  |  |
|  |  |  |  | 2 for resource #2 |  |  |
|  |  |  |  | 3 for resource #3 |  |  |
| firstOFDMSymbolInTimeDomain | 5 for resource #0 | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 | 5 for resource #0 | 10 for resource #1 |
|  |  |  |  | 5 for resource #5 |  |  |
|  |  |  |  | 6 for resource #6 |  |  |
|  |  |  |  | 7 for resource #7 |  |  |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM | FD-CDM2 | noCDM |
| density | 1 | 3 | 3 | 3 | 1 | 3 |
| startingRB | 0 | 0 | 0 | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |  |  |  |

Table A.1.4.2-3A: CSI-RS Reference Measurement Channels for SCS=120 kHz for TDD

|  | CSI-RS.3.1A TDD | CSI-RS.3.2A TDD | CSI-RS.3.3A TDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| nzp-CSI-ResourceSetId | 1 | 1 | 1 |
| repetition | off | off | off |
| aperiodicTriggeringOffset | n.a. | 6 | n.a. |
| trs-Info | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |
| nzp-CSI-RS-ResourceId | 12 for resource #0 | 22 for resource #0 | 14 for resource #0 |
|  | 13 for resource #1 | 23 for resource #1 | 15 for resource #1 |
| powerControlOffset | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 |
| Period (slots) | slot160 | n.a. | slot80 |
| Offset | 8 | n.a. | 16 |
| qcl-InfoPeriodicCSI-RS | n.a. | n.a. | n.a. |
| frequencyDomainAllocation | 0001 | 0001 | 0001 |
| nrofPorts | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| cdm-Type | noCDM | noCDM | noCDM |
| density | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

## A.1.4A	CSI-RS for tracking

### A.1.4A.1	FR1

#### A.1.4A.1.1	FDD

Table A.1.4A.1.1-1: CSI-RS for tracking FDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | TRS.1.1 FDD | TRS.1.2 FDD |
| Bandwidth |  | BW of Active BWP1 | BW of Active BWP1 |
| SCS | kHz | 15 | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0 | 0 |
| TCI state |  | TCI.State.0 | TCI.State.0 |
| NOTE 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |  |

Table A.1.4A.1.1-2: Aperiodic CSI-RS for tracking

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | TRS.1.3 FDD | TRS.1.4 FDD |
| Bandwidth |  | BW of Active BWPNote 1 | BW of Active BWPNote 1 |
| SCS | kHz | 15 | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 |
| aperiodicTriggeringOffsetL2 | slots | 2 | 2 |
| Aperiodic CSI-RS offset | slots | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 | 0Note 2 |
| TCI state |  | TCI.State.0 | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |  |

#### A.1.4A.1.2	TDD

Table A.1.4A.1.2-1: CSI-RS for TDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | TRS.1.1 TDD | TRS.1.2 TDD |
| Bandwidth |  | BW of Active BWP1 | BW of Active BWP1 |
| SCS | kHz | 15 | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0 | 0 |
| TCI state |  | TCI.State.0 | TCI.State.0 |
| NOTE 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |  |

Table A.1.4A.1.2-2: Aperiodic CSI-RS for tracking

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | TRS.1.3 TDD | TRS.1.4 TDD |
| Bandwidth |  | BW of Active BWPNote 1 | BW of Active BWPNote 1 |
| SCS | kHz | 15 | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 |
| aperiodicTriggeringOffsetL2 | slots | 2 | 2 |
| Aperiodic CSI-RS offset | slots | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 | 0Note 2 |
| TCI state |  | TCI.State.0 | TCI.State.0 |
| Note: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |  |

### A.1.4A.2	FR2

#### A.1.4A.2.1	TDD

Table A.1.4A.2.1-1: CSI-RS for tracking for TDD FR2

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | TRS.2.1 TDD | TRS.2.2 TDD | TRS.2.4 TDD | TRS.2.5 TDD |
| Bandwidth |  | BW of Active BWPNote 1, 2 | BW of Active BWPNote 1,2 | BW of Active BWPNote 1,2 | BW of Active BWPNote 1,2 |
| SCS | kHz | 120 | 120 | 120 | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 1 for CSI-RS resource 1 and 3 l0 = 5 for CSI-RS resource 2 and 4 | l0 = 2 for CSI-RS resource 1 and 3 l0 = 6 for CSI-RS resource 2 and 4 | l0 = 3 for CSI-RS resource 1 and 3 l0 = 7 for CSI-RS resource 2 and 4 | l0 = 4 for CSI-RS resource 1 and 3 l0 = 8 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 80 for CSI-RS resource 1,2,3,4 | 80 for CSI-RS resource 1,2,3,4 | 80 for CSI-RS resource 1,2,3,4 | 80 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0 | 0 | 0 | 0 |
| TCI state |  | TCI.State.0 Note 3 | TCI.State.1 Note 3 | TCI.State.2 | TCI.State.3 |
| NOTE 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases NOTE 2: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. NOTE 3: If unified TCI states are configured, the TCI state ID is defined in each test case and from A.10A.2 Unified TCI state Configuration |  |  |  |  |  |

Table A.1.4A.2.1-2: Aperiodic CSI-RS for tracking for SCS=120kHz Set 1

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.2.3 TDD |
| Bandwidth |  | BW of Active BWPNote 1,3 |
| SCS | kHz | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 1 for CSI-RS resource 1 and 3 l0 = 5 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| aperiodicTriggeringOffsetL2 | slots | 2 |
| Aperiodic CSI-RS offset | slots | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case Note 3: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. |  |  |

## A.1.4B	CSI-IM configurations

### A.1.4B.1	FDD

Table A.1.4B.1-1: CSI-IM Reference Measurement Channels for SCS=15kHz

|  | CSI-IM.1.1 FDD | CSI-IM.1.2 FDD | CSI-IM.1.3 FDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| csi-IM-ResourceSetId | 0 | 0 | 0 |
| Resource Config |  |  |  |
| csi-IM-ResourceId | 0 for resource #0 | 10 for resource #0 | 2 for resource #0 |
|  | 1 for resource #1 | 11 for resource #1 | 3 for resource #1 |
| csi-IM-ResourceElementPattern | pattern1 | pattern1 | pattern1 |
| subcarrierLocation-p1 | s0 | s0 | s0 |
| symbolLocation-p1 | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| Period (slots) | slot20 | n.a. | slot10 |
| Offset | 1 | n.a. | 2 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

### A.1.4B.2	TDD

Table A.1.4B.2-1: CSI-IM Reference Measurement Channels for SCS=15kHz

|  | CSI-IM.1.1 TDD | CSI-IM.1.2 TDD | CSI-IM.1.3 TDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| csi-IM-ResourceSetId | 0 | 0 | 0 |
| Resource Config |  |  |  |
| csi-IM-ResourceId | 0 for resource #0 | 10 for resource #0 | 2 for resource #0 |
|  | 1 for resource #1 | 11 for resource #1 | 3 for resource #1 |
| csi-IM-ResourceElementPattern | pattern1 | pattern1 | pattern1 |
| subcarrierLocation-p1 | s0 | s0 | s0 |
| symbolLocation-p1 | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| Period (slots) | slot20 | n.a. | slot10 |
| Offset | 1 | n.a. | 2 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

Table A.1.4B.2-2: CSI-IM Reference Measurement Channels for SCS=30kHz

|  | CSI-IM.2.1 TDD | CSI-IM.2.2 TDD | CSI-IM.2.3 TDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| csi-IM-ResourceSetId | 0 | 0 | 0 |
| Resource Config |  |  |  |
| csi-IM-ResourceId | 0 for resource #0 | 10 for resource #0 | 2 for resource #0 |
|  | 1 for resource #1 | 11 for resource #1 | 3 for resource #1 |
| csi-IM-ResourceElementPattern | pattern1 | pattern1 | pattern1 |
| subcarrierLocation-p1 | s0 | s0 | s0 |
| symbolLocation-p1 | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| Period (slots) | slot40 | n.a. | slot40 |
| Offset | 2 | n.a. | 4 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

Table A.1.4B.2-3: CSI-RS Reference Measurement Channels for SCS=120kHz

|  | CSI-IM.3.1 TDD | CSI-IM.3.2 TDD | CSI-IM.3.3 TDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| csi-IM-ResourceSetId | 0 | 0 | 0 |
| Resource Config |  |  |  |
| csi-IM-ResourceId | 0 for resource #0 | 10 for resource #0 | 2 for resource #0 |
|  | 1 for resource #1 | 11 for resource #1 | 3 for resource #1 |
| csi-IM-ResourceElementPattern | pattern1 | pattern1 | pattern1 |
| subcarrierLocation-p1 | s0 | s0 | s0 |
| symbolLocation-p1 | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| Period (slots) | slot160 | n.a. | slot80 |
| Offset | 8 | n.a. | 16 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

## A.1.5	TDD UL/DL configuration

Table A.1.5-1: TDD UL/DL configuration for SCS=15kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.1.1 | TDDConf.1.2 |  |
| referenceSubcarrierSpacing | kHz | 15 | 15 |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘DSUU’ S=‘10DL:2GP:2UL’ | ‘DSUU’ S=’ 6DL: 2GP: 6UL’ |  |
| dl-UL-TransmissionPeriodicity | ms | 4 | 4 |  |
| nrofDownlinkSlots |  | 1 | 1 |  |
| nrofDownlinkSymbols |  | 10 | 6 |  |
| nrofUplinkSlot |  | 2 | 2 |  |
| nrofUplinkSymbols |  | 2 | 6 |  |
| TDD UL/DL pattern 2 Note 2 |  | ‘D’ | ‘D’ |  |
| dl-UL-TransmissionPeriodicity | ms | 1 | 1 |  |
| nrofDownlinkSlots |  | 1 | 1 |  |
| nrofDownlinkSymbols |  | 0 | 0 |  |
| nrofUplinkSlot |  | 0 | 0 |  |
| nrofUplinkSymbols |  | 0 | 0 |  |
| NOTE 1: As specified in TS 38.213 [8] and TS 38.331 [13]. NOTE 2: For information |  |  |  |  |

Table A.1.5-2: TDD UL/DL configuration for SCS=30kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.2.1 | TDDConf.2.2 | TDDConf.2.3 |
| referenceSubcarrierSpacing | kHz | 30 | 30 | 30 |
| TDD UL/DL pattern 1 Note 2 |  | ‘3D1S4U’ S=‘6DL:4GP:4UL’ | ‘1D1S2U’ S=’11DL: 1GP:2UL’ | ‘3D1S4U’ S=’4DL:4GP:6UL’ |
| dl-UL-TransmissionPeriodicity | ms | 4 | 2 | 4 |
| nrofDownlinkSlots |  | 3 | 1 | 3 |
| nrofDownlinkSymbols |  | 6 | 11 | 4 |
| nrofUplinkSlot |  | 4 | 2 | 4 |
| nrofUplinkSymbols |  | 4 | 2 | 6 |
| TDD UL/DL pattern 2 Note 2 |  | ‘DD’ | Not configured | ‘DD’ |
| dl-UL-TransmissionPeriodicity | ms | 1 |  | 1 |
| nrofDownlinkSlots |  | 2 |  | 2 |
| nrofDownlinkSymbols |  | 0 |  | 0 |
| nrofUplinkSlot |  | 0 |  | 0 |
| nrofUplinkSymbols |  | 0 |  | 0 |
| NOTE 1: As specified in TS 38.213 [8] and TS 38.331 [13]. NOTE 2: For information |  |  |  |  |

Table A.1.5-3: TDD UL/DL configuration for SCS=120kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.3.1 | TDDConf.3.2 |  |
| referenceSubcarrierSpacing | kHz | 120 | 120 |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘DDDSU’ S=‘10DL:2GP:2UL’ | ‘DDDSU’ S=’ 6DL: 2GP: 6UL’ |  |
| dl-UL-TransmissionPeriodicity | ms | 0.625 | 0.625 |  |
| nrofDownlinkSlots |  | 3 | 3 |  |
| nrofDownlinkSymbols |  | 10 | 6 |  |
| nrofUplinkSlot |  | 1 | 1 |  |
| nrofUplinkSymbols |  | 2 | 6 |  |
| TDD UL/DL pattern 2 Note 2 |  | Not configured | Not configured |  |
| dl-UL-TransmissionPeriodicity | ms | Not configured | Not configured |  |
| nrofDownlinkSlots |  | Not configured | Not configured |  |
| nrofDownlinkSymbols |  | Not configured | Not configured |  |
| nrofUplinkSlot |  | Not configured | Not configured |  |
| nrofUplinkSymbols |  | Not configured | Not configured |  |
| NOTE 1: As specified in TS 38.213 [8] and TS 38.331 [13]. NOTE 2: For information |  |  |  |  |

## A.1.6	PUSCH

This rule applies to NR cell(s), which the UE is connected to. The UE is in RRC_CONNECTED mode.

When signalling or data payloads are expected to be sent on the PUSCH, the UE may be provided in advance with PUSCH resources by the SS. For sake of simplicity, the PUSCH scheduling may also occur continuously over many consecutive subframes. These options shall not be used if:
1) stated otherwise in the test description, or 
2) the transmission of PUSCH and UL scheduling information affects the test purpose (e.g. DRX, PUCCH reception etc.)

# A.1A	Reference measurement channels under CCA

## A.1A.0	General

This clause contains the Reference Measurement Channels (RMC) under CCA to be used for the RRM test scenarios in Clauses 10 to 12 of this document.

## A.1A.1	PDSCH

### A.1A.1.1	FDD

Table A.1A.1.1-1: PDSCH Reference Measurement Channels under CCA for SCS=30kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.1.1 CCA |  |  |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |  |  |  |  |  |  |
| Allocated slots per Radio Frame |  |  |  |  |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 |  |  |  |  |  |  |
| Radio frame not containing SSB | slots | Note 7 |  |  |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |  |  |
| Target Coding Rate |  | 1/3 |  |  |  |  |  |  |
| Number of control symbols |  | 2 |  |  |  |  |  |  |
| PDSCH mapping type |  | Type A |  |  |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | bits | 1608 |  |  |  |  |  |  |
| For slots without RMSI | bits | 1864 |  |  |  |  |  |  |
| Number of Code Blocks per slot |  | 1 |  |  |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2, Note 4 | bits | 5184 |  |  |  |  |  |  |
| For slots without RMSI Note 6 | bits | 6048 |  |  |  |  |  |  |
| Note 1: Allocated outside the discovery burst transmission window in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in TS 38.213 [8]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3A. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. Note 7: PDSCH is transmitted during the RMC burst as specified in A.1A.5. |  |  |  |  |  |  |  |  |

## A.1A.2	CORESET for RMSI scheduling

### A.1A.2.1	TDD

Table A.1A.2.1-1: RMSI CORESET Reference Channel under CCA for SCS=30KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CR.1.1 CCA |  |  |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |  |  |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 48 |  |  |  |  |  |  |
| SSB and RMSI CORESET multiplexing configuration Note 7 |  | Pattern 1 |  |  |  |  |  |  |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) |  |  |  |  |  |  |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 0 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Duration of RMSI CORESET Note 7 | symbols | 2 |  |  |  |  |  |  |
| DCI Format Note 1 |  | Note 2 |  |  |  |  |  |  |
| Aggregation level | CCE | 8 |  |  |  |  |  |  |
| DMRS precoder granularity |  | 6 |  |  |  |  |  |  |
| REG bundle size |  | 6 |  |  |  |  |  |  |
| Mapping from REG to CCE |  | Distributed |  |  |  |  |  |  |
| Cell ID |  | Note 5 |  |  |  |  |  |  |
| Payload (without CRC) | bits | Note 6 |  |  |  |  |  |  |
| Note 1: DCI formats are defined in TS 38.212 [31]. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [8]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 4 in Table 13-4A in TS 38.213 [8]. Note 8: Other values can be used to align with GSCN [13] as long as SSB does not overlap the RMC. |  |  |  |  |  |  |  |  |

## A.1A.3	CORESET for RMC scheduling

### A.1A.3.1	TDD

Table A.1A.3.1-1: Control Channel RMC under CCA with SCS=30KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 CCA | CCR.1.2 CCA | CCR.1.3 CCA |  |  |  |  |
| Channel bandwidth | MHz | 40 | 40 | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 |  |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 24 | 18 |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 |  |  |  |  |
| Duration of CORESET | symbols | 2 | 2 | 2 |  |  |  |  |
| REG bundle size |  | 6 | 6 | 6 |  |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  |  |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved |  |  |  |  |
| Interleave n_shift |  | 0 | 0 | 0 |  |  |  |  |
| Interleave size |  | 2 | 2 | 2 |  |  |  |  |
| Beamforming Pre-Coder |  | N/A | N/A | N/A |  |  |  |  |
| Aggregation level | CCE | 4 | 8 | 4 |  |  |  |  |
| DCI formats |  | Note 1 | Note 1 | Note 1 |  |  |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 |  |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

## A.1A.4	TDD UL/DL configuration

### A.1A.4.1	TDD

Table A.3.1A.4-1: TDD UL/DL configuration under CCA for SCS=30kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.1.1 CCA |  |  |
| referenceSubcarrierSpacing | kHz | N/A |  |  |
| TDD UL/DL pattern 1 Note 2, Note 3 |  | ‘3D1S4U’ S=’6DL:4GP:4UL’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | N/A |  |  |
| nrofDownlinkSlots |  | N/A |  |  |
| nrofDownlinkSymbols |  | N/A |  |  |
| nrofUplinkSlot |  | N/A |  |  |
| nrofUplinkSymbols |  | N/A |  |  |
| TDD UL/DL pattern 2 Note 2, Note 3 |  | ‘DD’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | N/A |  |  |
| nrofDownlinkSlots |  | N/A |  |  |
| nrofDownlinkSymbols |  | N/A |  |  |
| nrofUplinkSlot |  | N/A |  |  |
| nrofUplinkSymbols |  | N/A |  |  |
| Note 1: As specified in TS 38.213 [8] and TS 38.331 [13]. Note 2: Do not configure tdd-UL-DL-ConfigurationCommon using RRC configuration Note 3: The UE will be scheduled via DCI according to the TDD pattern defined in the table. |  |  |  |  |

## A.1A.5	RMC burst transmission model

RMC not conveying RMSI is scheduled during the RMC burst. The length of the transmission burst in slots is defined as N. The burst transmission format is determined according to the steps below:

1.	Select N randomly from a given set of the number of slots S1 = {1,3,5} with equal probability as the total length of RMC burst transmission format.

2.	A uniform random variable from 0 to 1 is generated. If the random variable is less than PCCA_DL, a burst of N fully occupied slots is transmitted. Otherwise, the RMC burst transmission is muted and the muting duration is the same as the number N of slots for determined burst format.

RMC burst transmission is scheduled outside discovery burst transmission window. If transmission occurred in the previous slot, transmission is muted for a duration of one slot. Additionally, if the start time of the candidate RMC burst transmission is within 5 slots of the start of the discovery burst transmission window, RMC transmission is not performed.

# A.2	Reference OCNG configuration

## A.2.1	Generic OFDMA channel noise generator (OCNG)

The OCNG pattern is used in a test for modelling the allocation of unused resourced in the channel bandwidth to virtual UEs (UEs that are not under test). The OCNG pattern simulates both PDCCH and PDSCH transmissions to the virtual UEs. For the cells operating under CCA, OCNG is transmitted only in slots with RMC burst transmission and is not transmitted during the slots for which CCA attempt is unsuccessful or during DBT windows.

Table A.2.1-1: OP.1: Generic OCNG pattern for all unused REs

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| NOTE 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. NOTE 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell, confined to BWoccupied where specified in the test case. |  |  |

Table A.2.1-2: OP.2: Generic OCNG pattern for all unused REs for 2AoA setup

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Probe | Transmitting the serving beam |  |
| Resource allocation | Unused REs (Note 1) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. | Unused REs (Note 2) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| NOTE 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. NOTE 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell, confined to BWoccupied where specified in the test case. NOTE 3: No OCNG is transmitted from the probe transmitting non-serving beam. |  |  |

Table A.2.1-3: OP.3: Generic OCNG pattern for unused REs in the same BW as CORESET

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| NOTE 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. NOTE 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the allocated bandwidth of the CORESET of the serving cell. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. |  |  |

Table A.2.1-4: OP.4: Generic OCNG pattern for all unused REs outside SSB slot(s)

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| NOTE 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. REs for OCNG shall not be allocated in the slot(s) containing SSB of the respective cell. NOTE 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell. REs for OCNG shall not be allocated in the slot(s) containing SSB of the respective cell. |  |  |

Table A.2.1-5: OP.5: Generic OCNG pattern for unused REs in the same BW as CORESET for 2AoA setup

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Probe | Transmitting the serving beam |  |
| Resource allocation | Unused REs (Note 1) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. | Unused REs (Note 2) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| NOTE 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. NOTE 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the allocated bandwidth of the CORESET of the serving cell. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. NOTE 3: No OCNG is transmitted from the probe transmitting non-serving beam. |  |  |

# A.3	Reference SSB configuration

## A.3.1	SSB configuration for FR1

Table A.3.1-1: SSB allocation for FR1

| SSB Parameters |  | Unit | Value |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Pattern |  |  | SSB.1 FR1 | SSB.2 FR1 | SSB.3 FR1 |  | SSB.4 FR1 |  | SSB.5 FR1 | SSB.6 FR1 | SSB.7 FR1 |  | SSB.8 FR1 |  | SSB.13 FR1 |
| Channel bandwidth |  | MHz | 10 | 40 | 10 |  | 40 |  | 10 | 40 | 10 |  | 40 |  | 3 |
| SSB SCS |  | kHz | 15 | 30 | 15 |  | 30 |  | 15 | 30 | 15 |  | 30 |  | 15 |
| SSB periodicity (TSSB) |  | ms | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of SSBs per SS-burst |  |  | 1 | 1 | 2 |  | 2 |  | 1 | 1 | 2 |  | 2 |  | 1 |
| SS/PBCH block index |  |  | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 2 | 3 | 2 | 3 | 0 |
| Indices of symbols containing SSB |  |  | 2-5 | 4-7 or 2-5 Note 2 | 2-5 | 8-11 | 4-7 or 2-5 Note 2 | 8-11 | 2-5 | 4-7 or 2-5 Note 2 | 2-5 | 8-11 | 4-7 or 2-5 Note 2 | 8-11 | 2-5 |
| Indices of slots containing SSB |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |
| Indices of SFN containing SSB |  |  | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |  |  |  |  | SFN mod (max(TSSB,10ms)/10ms) = 1 |  | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |  |  |  |
| RB numbers containing SSB within channel BW |  |  | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |  |  |  |  |  |  |  |  |  |  | (RBJ, RBJ+1,.…, RBJ+11)Note 1 |
|  | NOTE 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [28]. NOTE 2: Symbols 4-7 are chosen if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [28]; Otherwise, symbols 2-5 are chosen. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## A.3.2	SSB configuration for FR2

Table A.3.2-1: SSB allocation for FR2

| SSB Parameters | Unit | Value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Pattern |  | SSB.1 FR2 |  | SSB.2 FR2 |  | SSB.3 FR2 | SSB.4 FR2 | SSB.5 FR2 |  | SSB.6 FR2 |  | SSB.7 FR2 | SSB.8 FR2 | SSB.9 FR2 |  | SSB.10 FR2 |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSB SCS | kHz | 120 |  | 240 |  | 120 | 240 | 120 |  | 240 |  | 120 | 240 | 120 |  | 240 |  |
| SSB periodicity (TSSB) | ms | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of SSBs per SS-burst |  | 2 |  | 2 |  | 1 | 1 | 2 |  | 2 |  | 1 | 1 | 2 |  | 2 |  |
| SS/PBCH block index |  | 0 | 1 | 0 | 1 | 0 | 0 | 2 | 3 | 2 | 3 | 1 | 1 | 2 | 3 | 2 | 3 |
| Indices of symbols containing SSB |  | 4-7 | 8-11 | 8-11 | 12-13, 0-1 | 4-7 | 8-11 | 2-5 | 6-9 | 2-5 | 6-9 | 8-11 | 12-13, 0-1 | 2-5 | 6-9 | 4-7 | 8-11 |
| Indices of slots containing SSB |  | 0 |  | 0 | 0-1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0-1 | 1 | 1 | 1 | 1 |
| Indices of SFN containing SSB |  | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RB numbers containing SSB within channel BW |  | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [28]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# A.3A	Reference SSB configuration under CCA

## A.3A.1	SSB configuration under CCA for FR1

### A.3A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz

Table A.3A.1.1-1: SSB.1 CCA: SSB Pattern 1 for SSB under CCA for SCS=30 kHz in 40 MHz channel

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 40 MHz |
| SSB SCS | 30 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSB indexes per SS-burst (NSSBQCL) | 1 |
| Number of SS/PBCH block candidates per SSB index | 1 |
| SS/PBCH block candidate position | 0 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 2 | 2-5 |
| Slot numbers containing SSB Note 2 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [28]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [8]). They are not settable parameters themselves |  |

### A.3A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz

Table A.3A.1.2-1: SSB.2 CCA: SSB Pattern 2 for SSB under CCA for SCS=30 kHz in 40 MHz channel

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 40 MHz |  |
| SSB SCS | 30 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSB indexes per SS-burst (NSSBQCL) | 1 |  |
| Number of SS/PBCH block candidates per SSB index | 2 |  |
| SS/PBCH block candidate position | 0 | 2 |
| SS/PBCH block index | 0 | 0 |
| Symbol numbers containing SSB Note 2 | 2-5 | 2-5 |
| Slot numbers containing SSB Note 2 | 0 | 1 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [28]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [8]). They are not settable parameters themselves |  |  |

### A.3A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz

Table A.3A.1.3-1: SSB.3 CCA: SSB Pattern 3 for SSB under CCA for SCS=30 kHz in 40 MHz channel

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 40 MHz |  |
| SSB SCS | 30 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSB indexes per SS-burst (NSSBQCL) | 2 |  |
| Number of SS/PBCH block candidates per SSB index | 1 |  |
| SS/PBCH block candidate position | 0 | 1 |
| SS/PBCH block index | 0 | 1 |
| Symbol numbers containing SSB Note 2 | 2-5 | 8-11 |
| Slot numbers containing SSB Note 2 | 0 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [28]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [8]). They are not settable parameters themselves. |  |  |

### A.3A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz

Table A.3A.1.4-1: SSB.4 CCA: SSB Pattern 4 for SSB under CCA for SCS=30 kHz in 40 MHz channel

| SSB Parameters | Values |  |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | 40 MHz |  |  |  |
| SSB SCS | 30 kHz |  |  |  |
| SSB periodicity (TSSB) | 20 ms |  |  |  |
| Number of SSB indexes per SS-burst (NSSBQCL) | 2 |  |  |  |
| Number of SS/PBCH block candidates per SSB index | 2 |  |  |  |
| SS/PBCH block candidate position | 0 | 2 | 1 | 3 |
| SS/PBCH block index | 0 | 0 | 1 | 1 |
| Symbol numbers containing SSB Note 2 | 2-5 | 2-5 | 8-11 | 8-11 |
| Slot numbers containing SSB Note 2 | 0 | 1 | 0 | 1 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |  |  |
| RB numbers containing SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |  |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [28]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [8]). They are not settable parameters themselves. |  |  |  |  |

# A.3B	Reference SSB configuration for RedCap

## A.3B.1	SSB configuration for FR1

Table A.3B.1-1: SSB allocation for FR1

| SSB Parameters |  | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Pattern |  |  | SSB.1 RedCap FR1 | SSB.2 RedCap FR1 | SSB.3 RedCap FR1 | SSB.4 RedCap FR1 | SSB.5 RedCap FR1 | SSB.6 RedCap FR1 | SSB.7 RedCap FR1 |
| Channel bandwidth |  | MHz | 20 | 20 | 20 | 10 | 20 | 10 | 20 |
| SSB SCS |  | kHz | 30 | 30 | 30 | 15 | 30 | 15 | 30 |
| SSB periodicity (TSSB) |  | ms | 20 | 20 | 20 | 20 | 20 | 80 | 80 |
| Number of SSBs per SS-burst |  |  | 1 | 2 | 1 | 1 | 1 | 1 | 1 |
| Symbol numbers containing SSB Note 4 | SSB #0 |  | 4-7 or 2-5 Note 3 | 4-7 or 2-5 Note 3 | 4-7 or 2-5 Note 3 | 2-5 | 4-7 or 2-5 Note 3 | 2-5 | 4-7 or 2-5 Note 3 |
|  | SSB #1 |  | N/A | 8-11 | N/A | N/A | N/A | N/A | N/A |
| Slot numbers containing SSB Note 4 |  |  | 0 |  |  |  |  |  |  |
| SFN containing SSB |  |  | SFN mod (TSSB /10ms) = 0 | SFN mod (TSSB /10ms) = 0 | SFN mod (TSSB /10ms) = 1 | SFN mod (TSSB /10ms) = 0 | SFN mod (TSSB /10ms) = 0 | SFN mod (TSSB /10ms) = 0 | SFN mod (TSSB /10ms) = 0 |
| RB numbers containing SSB within channel BW |  |  | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |  |  |  | (RBJ, RBJ+1,.…, RBJ+19)Note 2 |  |
| NOTE 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [28]. NOTE 2: RBs containing SSB can be configured in any frequency location within the associated bandwidth part except the RBs for allowed synchronization raster defined in TS 38.104 [28]. NOTE 3: Symbols 4-7 are chosen if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [28]; Otherwise, symbols 2-5 are chosen. NOTE 4: These values have been derived from other parameters for information purposes (as per TS 38.213). They are not settable parameters themselves |  |  |  |  |  |  |  |  |  |

## A.3B.2	SSB configuration for FR2

Table A.3B.2-1: SSB allocation for FR2

| SSB Parameters |  | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Pattern |  |  | SSB.1 RedCap FR2 | SSB.2 RedCap FR2 | SSB.3 RedCap FR2 | SSB.4 RedCap FR2 | SSB.5 RedCap FR2 |
| Channel bandwidth |  | MHz | 100 | 100 | 100 | 100 | 100 |
| SSB SCS |  | kHz | 120 | 120 | 120 | 240 | 240 |
| SSB periodicity (TSSB) |  | ms | 20 | 40 | 80 | 40 | 80 |
| Number of SSBs per SS-burst |  |  | 1 | 1 | 1 | 1 | 1 |
| Symbol numbers containing SSB Note 3 | SSB #0 |  | 4-7 | 4-7 | 4-7 | 8-11 | 8-11 |
|  | SSB #1 |  | N/A | N/A | N/A | N/A | N/A |
| Indices of slots containing SSB Note 3 |  |  | 0 |  |  |  |  |
| Indices of SFN containing SSB |  |  | SFN mod (TSSB /10ms) = 0 | SFN mod (TSSB /10ms) = 0 | SFN mod (TSSB /10ms) = 0 | SFN mod (TSSB /10ms) = 0 | SFN mod (TSSB /10ms) = 0 |
| RB numbers containing SSB within channel BW |  |  | (RBJ,RBJ+1,.…,RBJ+19)Note 1 |  | (RBJ,RBJ+1,.…,RBJ+19)Note 2 | (RBJ,RBJ+1,.…,RBJ+19)Note 1 | (RBJ,RBJ+1,.…,RBJ+19)Note 2 |
| NOTE 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [28]. NOTE 2: RBs containing SSB can be configured in any frequency location within the associated bandwidth part except the RBs for allowed synchronization raster defined in TS 38.104 [28]. NOTE 3: These values have been derived from other parameters for information purposes (as per TS 38.213). They are not settable parameters themselves |  |  |  |  |  |  |  |

# A.4	Reference SMTC configuration

Table A.4-1: SMTC configurations set1

| SMTC Parameters | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SMTC Pattern |  | SMTC.1 | SMTC.2 | SMTC.3 | SMTC.4 | SMTC.5 | SMTC.6 | SMTC.7 | SMTC.8 |
| SMTC periodicity | ms | 20 | 20 | 160 | 20 | 20 | 20 | 20 | 10 |
| SMTC offset | ms | 0 | 0 | 0 | 10 | 10 | 17 | 5 | 0 |
| SMTC duration | ms | 1 | 5 | 1 | 1 | 5 | 5 | 5 | 1 |

Table A.4-2: SMTC configurations set2

| SMTC Parameters | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SMTC Pattern |  | SMTC.9 | SMTC.10 | SMTC.11 | SMTC.12 |
| SMTC periodicity | ms | 20 | 80 | 80 | 20 |
| SMTC offset | ms | 13 | 0 | 5 | 4 |
| SMTC duration | ms | 1 | 1 | 5 | 5 |

# A.4A	Reference SMTC configuration for RedCap

Table A.4A-1: SMTC configurations

| SMTC Parameters | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SMTC Pattern |  | SMTC.1 RedCap | SMTC.2 RedCap | SMTC.3 RedCap | SMTC.4 RedCap |  |  |
| SMTC periodicity | ms | 40 | 80 | 40 | 80 |  |  |
| SMTC offset | ms | 0 | 0 | 20 | 0 |  |  |
| SMTC duration | ms | 1 | 1 | 1 | 5 |  |  |

# A.5	Reference DRX configurations

The reference DRX configurations for the NR serving cell are captured in Table A.5-1. The reference DRX configurations for the E-UTRA serving cell for EN-DC and inter-RAT test cases are captured in Table A.5-2.

Table A.5-1: DRX configurations for NR serving cell

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRX Configuration |  | DRX.1 | DRX.2 | DRX.3 | DRX.6 | DRX.7 | DRX.8 | DRX.11 | DRX.X1 | DRX.14 |
| drx-onDurationTimer | ms | 1 | 1 | 6 | 1 | 6 | 6 | 6 | 6 | 6 |
| drx-InactivityTimer | ms | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| drx-RetransmissionTimerDL | slot | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| drx-RetransmissionTimerUL | slot | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| drx-LongCycleStartOffset | ms | 40 | 640 | 40 | 320 | 640 | 320 | 20 | 80 | 160 |
| shortDRX | - | disabled | disabled | disabled | disabled | disabled | disabled | disabled | disabled | disabled |
| TimeAlignmentTimer | ms | 500 | 500 | Infinity | 500 | Infinity | Infinity | Infinity | Infinity | Infinity |
| NOTE 1: The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 38.331 [13]. |  |  |  |  |  |  |  |  |  |  |

Table A.5-2: DRX configurations for E-UTRA serving cell

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DRX Configuration |  | DRX.4 | DRX.5 | DRX.9 | DRX.10 | DRX.12 |
| drx-onDurationTimer | ms | psf2 | psf6 | psf2 | psf6 | psf6 |
| drx-InactivityTimer | ms | psf2 | psf1920 | psf100 | psf1920 | psf2 |
| drx-RetransmissionTimerDL | slot | psf16 | psf16 | psf16 | psf16 | psf16 |
| drx-LongCycleStartOffset | ms | sf160, 0 | sf320. 0 | sf40, 0 | sf640. 0 | sf640, 0 |
| shortDRX | - | disabled | disabled | disabled | disabled | disabled |
| TimeAlignmentTimer | ms | Infinity | Infinity | 500 | 500 | Infinity |
| NOTE 1: The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 36.331 [29] |  |  |  |  |  |  |

# A.6	EN-DC test setup

The purpose of this Annex is to specify the EN-DC configuration for the test cases in Chapters 4 and 5 of this test specification.

## A.6.1	E-UTRA serving cell parameters

This clause defines the cell power levels and other specific cell parameters of the E-UTRA serving cell for EN-DC.

### A.6.1.1	E-UTRA serving cell parameters for EN-DC tests with NR FR1

Table A.6.1.1-1 defines the E-UTRA serving cell parameters for EN-DC tests with NR FR1 cell(s), defined in Chapter 4 of this test specification and for iRAT tests with E-UTRA serving cell and NR FR1 cell(s) defined in Chapter 8 of this test specification. Unless otherwise stated within the test, all measurements in Clauses 4 and 5 are performed only on the NR carrier. The E-UTRA PCell shall be configured to not interfere with NR operation and the E-UTRA PCell signal power shall not be critical to the test purpose.

Table A.6.1.1-1: E-UTRAN cell specific test parameters for EN-DC tests with NR FR1

| Parameter | Unit | E-UTRAN Cell1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel Note 6 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2, Note 6 |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2, Note 6 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG PatternsNote 2 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote 3 | dB |  |
| OCNG_RBNote 3 | dB |  |
| NocNote 4 | dBm/15 kHz | -104 |
| Ês/Noc | dB | 17 |
| Ês/Iot | dB | 17 |
| RSRP Note 5 | dBm/15 kHz | -87 |
| SCH_RP Note 5 | dBm/15 kHz | -87 |
| Io Note 5 | dBm/Ch BW | -59.13 + 10log(NRB,c /50) |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A.1, A.2 and D.1 of TS 36.521-3 [26]. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 6: For E-UTRA anchor configuration, pick 5 MHz as default channel bandwidth setting in the tests as it is supported by all E-UTRA bands. If none of the UE supported EN-DC band combos support 5MHz E-UTRA carrier, pick 20 MHz channel BW or 10 MHz channel BW, in that order, |  |  |

Table A.6.1.1-2: CQI-ReportConfig-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2 CQI-ReportConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | NOT PRESENT |  |  |
| cqi-ReportPeriodic | NOT PRESENT |  |  |
| } |  |  |  |

Table A.6.1.1-3:PhysicalConfigDedicated-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| soundingRS-UL-ConfigDedicated | Not present |  | RBC |
| } |  |  |  |

Table A.6.1.1-4:MAC-MainConfig-RBC: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| timeAlignmentTimerDedicated | Infinity |  |  |

### A.6.1.2	E-UTRA serving cell parameters for EN-DC tests with NR FR2

Table A.6.1.2-1 defines the E-UTRA serving cell parameters for EN-DC tests with NR FR2 cell(s), defined in Chapter 5 of this test specification and for iRAT tests with E-UTRA serving cell and NR FR1 cell(s) defined in Chapter 8 of this test specification. Unless otherwise stated within the test, all measurements in Clauses 5 and 7 are performed only on the NR carrier. The E-UTRA PCell shall configured to not interfere with NR operation and the E-UTRA PCell signal power shall not be critical to the test purpose.

Table A.6.1.2-1: E-UTRAN cell specific test parameters for EN-DC tests with NR FR2

| Parameter | Unit | E-UTRAN Cell1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel Note 5 | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2, Note 5 |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2, Note 5 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG PatternsNote2, Note 5 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 1, A.2 and D.1 of TS 36.521-3 [26]. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: The E-UTRA signal is required only to ensure the E-UTRA link to the DUT in the EN-DC operation. The Test System shall provide a stable and noise-free E-UTRA signal without need of precise propagation modelling, path loss and polarization control. Further details of the E-UTRA signal configuration are not defined as part of the cell specific test parameters, since the E-UTRA link is not under performance verification and is not expected to influence the NR FR2 requirement. NOTE 5: The For E-UTRA anchor configuration, pick 5 MHz as default channel bandwidth setting in the tests as it is supported by all E-UTRA bands. If none of the UE supported EN-DC band combos support 5MHz E-UTRA carrier, pick 20 MHz channel BW or 10 MHz channel BW, in that order, |  |  |

Table A.6.1.2-2: CQI-ReportConfig-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2 CQI-ReportConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | NOT PRESENT |  |  |
| cqi-ReportPeriodic | NOT PRESENT |  |  |
| } |  |  |  |

Table A.6.1.2-3:PhysicalConfigDedicated-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| soundingRS-UL-ConfigDedicated | Not present |  | RBC |
| } |  |  |  |

Table A.6.1.2-4:MAC-MainConfig-RBC: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| timeAlignmentTimerDedicated | Infinity |  |  |

# A.6A	NR FR1-FR2 test setup

Some test cases in clause 7 have NR cells in both FR1 and FR2. Unless otherwise stated within the test, the NR FR1 Cell signal is required only to provide a link to the UE under test. The Test Equipment shall provide a stable and noise-free NR FR1 signal without need of precise propagation modelling, path loss and polarization control. Further details of the NR FR1 signal configuration are not defined as part of the cell specific test parameters, since the NR FR1 link is not under performance verification and is not expected to influence the test purpose.

# A.6B	EN-DC test setup with unlicensed bands

## A.6B.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1

Table A.6B.1-1 defines cell specific test parameters for E-UTRAN cell which can be used in EN-DC test cases or in any test case comprising at least one E-UTRA serving cell with all NR cells under CCA in FR1. Unless otherwise stated within the test, all measurements are performed only on the unlicensed NR carrier. The E-UTRA serving cell shall configured to not interfere with NR operation and the E-UTRA serving cell signal power shall not be critical to the test purpose.

Table A.6B.1-1: E-UTRAN cell specific test parameters for tests with all NR cells user CCA in FR1

| Parameter | Unit | E-UTRAN Cell |
| --- | --- | --- |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel |  | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |
| OCNG PatternsNote2 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |
| PBCH_RA | dB |  |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB | 0 |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
| NocNote4 | dBm/15 kHz | -104 |
| Ês/Noc | dB | 17 |
| Ês/Iot | dB | 17 |
| RSRP Note5 | dBm/15 kHz | -87 |
| SCH_RP Note5 | dBm/15 kHz | -87 |
| Io Note5 | dBm/Ch BW | -59.13+10log(NRB,c /50) |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 respectively. Note 3: OCNG shall be used such that all cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |

# A.7	Reference PRACH configurations

This clause provides the typical PRACH configurations used for all RRM test cases defined in this test specification. Parameters not listed in this clause can be derived from the configuration of each test.

## A.7.1	PRACH configurations for FR1

Table A.7.1-1 defines the PRACH configurations for FR1. Each of the PRACH configurations defined in Table A.7.1-1 have different applicabilities:

-	PRACH.1 FR1 for SSB-based contention based random access in FR1.

-	PRACH.2 FR1 for SSB-based non-contention based random access in FR1.

-	PRACH.3 FR1 for CSI-RS based non-contention based random access in FR1.

-	PRACH.4 FR1 for CSI-RS based non-contention based random access in FR1 to convey BFR.

-	PRACH.6 FR1 for SSB-based contention free random access in FR1.

The PRACH configuration defined in Table A.7.1-2 has the following applicability:

-	PRACH.5 FR1 for LTM early UL synchronization on candidate cell in FR1.

Table A.7.1-1 Parameters for PRACH Configurations for FR1

| Field | Value |  |  |  |  |  | Comment |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PRACH Configuration | PRACH.1 FR1 |  | PRACH.2 FR1 | PRACH.3 FR1 | PRACH.4 FR1 | PRACH.6 FR1 |  |  |
| prach-ConfigurationIndex | 102 |  | 102 | 102 | 102 | 102 |  | 10ms PRACH periodicity and other detailed configuration defined in table 6.3.3.2-2 in TS 38.211 [7]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS |  |  |
| totalNumberOfRA-Preambles | 48 |  | 48 | 48 | 48 | 48 |  | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 |  | 48 | 48 | 48 | 48 |  | No group B. |
| prach-RootSequenceIndex | 0 |  | 0 | 0 | 0 | 0 |  | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 |  | - | - | - | oneFourth, n16 |  | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| ssb-perRACH-Occasion | - |  | oneFourth | oneFourth | oneFourth | - |  | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One |  | One | One | One | One |  | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 |  | RSRP_51 | N/A | RSRP_51 | RSRP_51 |  | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| rsrp-ThresholdCSI-RS | N/A |  | N/A | RSRP_51 | N/A | N/A |  |  |
| ra-ContentionResolutionTimer | sf48 |  | - | - | - | sf48 |  | 48 sub-frames |
| powerRampingStep | dB2 |  | dB2 | dB2 | dB2 | dB2 |  |  |
| preambleReceivedTargetPower | dBm-120 |  | dBm-120 | dBm-120 | dBm-120 | dBm-120 |  |  |
| preambleTransMax | n6 |  | n6 | n6 | n200 | n6 |  | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 |  | sl10 | sl10 | sl1 | sl10 |  | 10 slots |
| zeroCorrelationZoneConfig | 11 |  | 11 | 11 | 11 | 11 |  | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 |  | 2 | 2 | 2 | 2 |  | 20ms, as defined in table 7.2-1 in TS 38.321 [12]. |
| ssb-ResourceList | - |  | present | N/A | N/A | - |  | Associated with SSB index 0 |
| ra-PreambleIndex | - |  | 40 | N/A | N/A | - |  | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | - |  | present | N/A | N/A | - |  | Associated with SSB index 0 |
| ra-PreambleIndex | - |  | 40 | N/A | N/A | - |  | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| csirs-ResourceList | N/A |  | N/A | present | present | N/A |  | Associated with CSI-RS configured |
| ra-PreambleIndex | N/A |  | N/A | 40 | 40 | N/A |  | Associated with CSI-RS configured |
| ra-OccasionList | - |  | - | 1 | 1 | - |  | RA occasions allowed corresponding to CSI-RS |
| ra-ssb-OccasionMaskIndex | - |  | 1 | N/A | N/A | - |  | PRACH occasion index 1 is allowed |
|  |  | NOTE: For further information see Clause 6.3.2 in TS 38.331 [13]. |  |  |  |  |  |  |  |

Table A.7.1-2 Parameters for LTM early UL synchronization PRACH Configuration for FR1

| Field | Value | Comment |
| --- | --- | --- |
| PRACH Configuration | PRACH.5 FR1 |  |
| prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [7]. |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| msg1-FrequencyStart | 0 |  |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 |  |
| powerRampingStep | dB2 |  |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ltm-prach-SubcarrierSpacing | Same as UL carrier SCS |  |
| Note: For further information see clause 6.3.2 in TS 38.331 [13]. |  |  |

## A.7.1A	PRACH configurations for FR1 under CCA

### A.7.1A.1	FR1 PRACH configuration 1 under CCA

Table A.7.1A.1-1 provides PRACH configuration 1 for SSB-based contention based random access in FR1 under CCA.

Table A.7.1A.1-1: Parameters for FR1 PRACH configuration 1 under CCA

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [7]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| ra-ContentionResolutionTimer | sf48 | 48 sub-frames |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-114 | Increased by 6 dB compared with FR1 PRACH configuration 1 for random access test with UL CCA failures. |
| preambleTransMax | n20 | Max number of RA preamble transmission performed before declaring a failure is 20 to account for CCA failures |
| ra-ResponseWindow | Sl20 | 20 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [12]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [13]. |  |  |

### A.7.1A.2	FR1 PRACH configuration 2 under CCA

Table A.7.1A.2-1 provides PRACH configuration 2 for SSB-based contention based random access in FR1 under CCA.

Table A.7.1A.2-1: Parameters for FR1 PRACH configuration 2 under CCA

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [7]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-114 | Increased by 6 dB compared with FR1 PRACH configuration 2 for random access test with UL CCA failures. |
| preambleTransMax | n20 | Max number of RA preamble transmission performed before declaring a failure is 20 to account for CCA failures |
| ra-ResponseWindow | sl20 | 20 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [12]. |
| ssb-ResourceList | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| ra-ssb-OccasionMaskIndex | 1 | PRACH occasion index 1 is allowed |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [13]. |  |  |

## A.7.2	PRACH configurations for FR2

Table A.7.2-1 defines the PRACH configurations for FR2. Each of the PRACH configurations defined in Table A.7.2-1 have different applicabilities:

-	PRACH.1 FR2 for SSB-based contention based random access in FR2.

-	PRACH.2 FR2 for SSB-based non-contention based random access in FR2.

-	PRACH.3 FR2 for CSI-RS based non-contention based random access in FR2.

-	PRACH.4 FR2 for CSI-RS based non-contention based random access in FR2 to convey BFR.

-	PRACH.6 FR2 for SSB-based contention free random access in FR2.

The PRACH configuration defined in Table A.7.2-2 has the following applicability:

-	PRACH.5 FR2 for LTM early UL synchronization on candidate cell in FR2.

Table A.7.2-1 Parameters for PRACH Configurations for FR2

| Field | Value |  |  |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PRACH Configuration | PRACH.1 FR2 | PRACH.2 FR2 |  | PRACH.3 FR2 | PRACH.4 FR2 | PRACH.6 FR2 |  |
| prach-ConfigurationIndex | 190 | 190 |  | 190 | 190 | 190 | Preamble format C2, 10ms PRACH periodicity and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [7]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS | Same as UL carrier SCS |  | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | 48 |  | 48 | 48 | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | 48 |  | 48 | 48 | 48 | No group B. |
| prach-RootSequenceIndex | 0 | 0 |  | 0 | 0 | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | N/A |  | N/A | N/A | oneFourth, n16 | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| ssb-perRACH-Occasion | N/A | oneFourth |  | oneFourth | oneFourth | N/A | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One |  | One | One | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | RSRP_51 |  | N/A | RSRP_51 | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| rsrp-ThresholdCSI-RS | N/A | N/A |  | RSRP_51 | N/A | N/A |  |
| ra-ContentionResolutionTimer | sf48 | N/A |  | N/A | N/A | sf48 | 48 sub-frames |
| powerRampingStep | dB2 | dB2 |  | dB2 | dB2 | dB2 |  |
| preambleReceivedTargetPower | dBm-120 | dBm-120 |  | dBm-120 | dBm-120 | dBm-120 |  |
| preambleTransMax | n6 | n6 |  | n6 | n200 | n6 | Max number of RA preamble transmission performed before declaring a failure |
| ra-ResponseWindow | sl10 | sl10 |  | sl10 | sl40 | sl10 |  |
| zeroCorrelationZoneConfig | 11 | 11 |  | 11 | 11 | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 2 |  | 2 | 2 | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [12]. |
| ssb-ResourceList | - | present |  | N/A | N/A | - | Associated with SSB index 0 |
| ra-PreambleIndex | - | 40 |  | N/A | N/A | - | Associated with SSB index 0 |
| csirs-ResourceList | N/A | present |  | present | present | N/A | Associated with CSI-RS configured |
| ra-PreambleIndex | N/A | 40 |  | 40 | 40 | N/A | Associated with CSI-RS configured |
| ra-OccasionList | - | - |  | 1 | 1 | - | RA occasions allowed corresponding to CSI-RS |
| ra-ssb-OccasionMaskIndex | - | 1 |  | N/A | N/A | - | PRACH occasion index 1 is allowed |
|  |  |  | NOTE: For further information see Clause 6.3.2 in TS 38.331 [13]. |  |  |  |  |

Table A.7.2-2 Parameters for LTM early UL synchronization PRACH Configuration for FR2

| Field | Value | Comment |
| --- | --- | --- |
| PRACH Configuration | PRACH.5 FR2 |  |
| prach-ConfigurationIndex | 190 | Preamble Format C2, with 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [7] |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| msg1-FrequencyStart | 0 |  |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 |  |
| powerRampingStep | dB2 |  |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ltm-prach-SubcarrierSpacing | Same as UL carrier SCS |  |
| Note: For further information see clause 6.3.2 in TS 38.331 [13]. |  |  |

# A.7A	Reference MsgA configurations

This clause provides the typical PRACH and PUSCH configurations for MsgA used for all RRM test cases defined in this test specification. Parameters not listed in this clause can be derived from the configuration of each test.

## A.7A.1	MsgA configurations for FR1

Table A.7A.1-1 defines the MsgA configurations for FR1. Each of the MsgA configurations defined in Table A.7A.1-1 have different applicabilities:

-	MsgA.1 FR1 for SSB-based 2-step contention based random access in FR1.

-	MsgA.2 FR1 for SSB-based 2-step non-contention based random access in FR1.

Table A.7A.1-1 Parameters for MsgA Configurations for FR1

| Field | Value |  | Comment |
| --- | --- | --- | --- |
| MsgA Configuration | MsgA.1 FR1 | MsgA.2 FR1 |  |
| msgA-prach-ConfigurationIndex | 102 | 102 | 10ms PRACH periodicity and other detailed configuration defined in table 6.3.3.2-2 in TS 38.211 [7]. |
| msgA-SubcarrierSpacing | Same as UL carrier SCS | Same as UL carrier SCS |  |
| msgA-totalNumberOfRA-Preambles | 48 | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | 48 | No group B. |
| msgA-PRACH-RootSequenceIndex | 0 | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| msgA-SSB-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msgA-RO-FDM | One | One | One PRACH transmission occasions FDMed in one time instance. |
| ra-ContentionResolutionTimer | sf48 | - | 48 sub-frames |
| msgA-PreamblePowerRampingStep | dB2 | dB2 |  |
| msgA-PreambleReceivedTargetPower | dBm-120 | dBm-120 |  |
| preambleTransMax | n6 | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| msgB-ResponseWindow | sl10 | sl10 | 10 slots |
| msgA-ZeroCorrelationZoneConfig | 11 | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [12]. |
| ssb-ResourceList | - | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | - | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| ra-ssb-OccasionMaskIndex | - | 1 | PRACH occasion index 1 is allowed |
| msgA-MCS | 1 | 1 | MCS index for MsgA PUSCH |
| nrofSlotsMsgA-PUSCH | 1 | 1 | Number of slots containing one or multiple PUSCH occasions |
| nrofMsgA-PO-PerSlot | 1 | 1 | Number of time domain PUSCH occasions in each slot |
| msgA-PUSCH-TimeDomainOffset | 1 | 1 | A single time offset with respect to the start of each PRACH slot, counted as the number of slots |
| PUSCH start symbol | 0 | 0 |  |
| PUSCH allocation length | 14 | 14 |  |
| mappingTypeMsgA-PUSCH | typeA | typeA |  |
| nrofPRBs-PerMsgA-PO | 2 | 2 | Number of RBs per PUSCH occasion |
| nrofMsgA-PO-FDM | One | One | The number of MsgA PUSCH occasions FDMed in one time instance |
| msgA-DMRS-AdditionalPosition | pos1 | pos1 | Position for additional DM-RS |
| msgA-PUSCH-NrofPorts | 0 | 0 | Configure 1 port per CDM group |
| msgA-DeltaPreamble | 2dB | 2dB | Power offset of msgA PUSCH relative to the preamble received target power |
| msgA-Alpha | alpha1 | alpha1 | Alpha value for MsgA PUSCH. Set 1 |
| deltaMCS | Disabled | Disabled | Whether to apply delta MCS |
| NOTE: For further information see clause 6.3.2 in TS 38.331 [13]. |  |  |  |

## A.7A.1A	MsgA configurations for FR1 under CCA

### A.7A.1A.1	FR1 MsgA configuration 1 under CCA

Table A.7A.1A.1-1 provides MsgA configuration 1 for SSB-based contention based random access for 2-step RA in FR1 under CCA.

Table A.7A.1A.1-1: Parameters for FR1 MsgA configuration 1 under CCA

| Field | Value | Comment |
| --- | --- | --- |
| msgA-prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [7]. |
| msgA-SubcarrierSpacing | Same as UL carrier SCS |  |
| msgA-totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| msgA-PRACH-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| msgA-SSB-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| msgA-RO-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| ra-ContentionResolutionTimer | sf48 | 48 sub-frames |
| msgA-PreamblePowerRampingStep | dB2 |  |
| msgA-PreambleReceivedTargetPower | dBm-114 | Increased by 6 dB compared with FR1 MsgA configuration 1 for random access test with UL CCA failures. |
| preambleTransMax | n20 | Max number of RA preamble transmission performed before declaring a failure is 20 to account for CCA failures |
| msgB-ResponseWindow | sl20 | 20 slots |
| msgA-ZeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [12]. |
| msgA-MCS | 1 | MCS index for MsgA PUSCH |
| nrofSlotsMsgA-PUSCH | 1 | Number of slots containing one or multiple PUSCH occasions |
| nrofMsgA-PO-PerSlot | 1 | Number of time domain PUSCH occasions in each slot |
| msgA-PUSCH-TimeDomainOffset | 1 | A single time offset with respect to the start of each PRACH slot, counted as the number of slots |
| PUSCH start symbol | 0 |  |
| PUSCH allocation length | 14 |  |
| mappingTypeMsgA-PUSCH | typeA |  |
| nrofPRBs-PerMsgA-PO | 2 | Number of RBs per PUSCH occasion |
| nrofMsgA-PO-FDM | One | The number of MsgA PUSCH occasions FDMed in one time instance |
| msgA-DMRS-AdditionalPosition | pos1 | Position for additional DM-RS |
| msgA-PUSCH-NrofPorts | 0 | Configure 1 port per CDM group |
| msgA-DeltaPreamble | 3 | Power offset of msgA PUSCH relative to the preamble received target power |
| msgA-Alpha | alpha1 | Alpha value for MsgA PUSCH. Set 1 |
| deltaMCS | Disabled | Whether to apply delta MCS |
| Note: For further information see clause 6.3.2 in TS 38.331 [13]. |  |  |

### A.7A.1A.2	FR1 MsgA configuration 2 under CCA

Table A.7A.1A.2-1 provides MsgA configuration 2 for SSB-based contention based random access for 2-step RA in FR1 under CCA.

Table A.7A.1A.2-1: Parameters for FR1 MsgA configuration 2 under CCA

| Field | Value | Comment |
| --- | --- | --- |
| msgA-prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [7]. |
| msgA-SubcarrierSpacing | Same as UL carrier SCS |  |
| msgA-totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| msgA-PRACH-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msgA-RO-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| msgA-PreamblePowerRampingStep | dB2 |  |
| msgA-PreambleReceivedTargetPower | dBm-114 | Increased by 6 dB compared with FR1 MsgA configuration 2 for random access test with UL CCA failures. |
| preambleTransMax | n20 | Max number of RA preamble transmission performed before declaring a failure is 20 to account for CCA failures |
| msgB-ResponseWindow | sl20 | 20 slots |
| msgA-ZeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [12]. |
| ssb-ResourceList | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| ra-ssb-OccasionMaskIndex | 1 | PRACH occasion index 1 is allowed |
| msgA-MCS | 1 | MCS index for MsgA PUSCH |
| nrofSlotsMsgA-PUSCH | 1 | Number of slots containing one or multiple PUSCH occasions |
| nrofMsgA-PO-PerSlot | 1 | Number of time domain PUSCH occasions in each slot |
| msgA-PUSCH-TimeDomainOffset | 1 | A single time offset with respect to the start of each PRACH slot, counted as the number of slots |
| PUSCH start symbol | 0 |  |
| PUSCH allocation length | 14 |  |
| mappingTypeMsgA-PUSCH | typeA |  |
| nrofPRBs-PerMsgA-PO | 2 | Number of RBs per PUSCH occasion |
| nrofMsgA-PO-FDM | One | The number of MsgA PUSCH occasions FDMed in one time instance |
| msgA-DMRS-AdditionalPosition | pos1 | Position for additional DM-RS |
| msgA-PUSCH-NrofPorts | 0 | Configure 1 port per CDM group |
| msgA-DeltaPreamble | 3 | Power offset of msgA PUSCH relative to the preamble received target power |
| msgA-Alpha | alpha1 | Alpha value for MsgA PUSCH. Set 1 |
| deltaMCS | Disabled | Whether to apply delta MCS |
| Note: For further information see clause 6.3.2 in TS 38.331 [13]. |  |  |

## A.7A.2	MsgA configurations for FR2

Table A.7A.2-1 defines the MsgA configurations for FR2. Each of the MsgA configurations defined in Table A.7A.2-1 have different applicabilities:

-	MsgA.1 FR2 for SSB-based contention based random access in FR2.

-	MsgA.2 FR2 for SSB-based non-contention based random access in FR2.

Table A.7A.2-1 Parameters for MsgA Configurations for FR2

| Field | Value |  | Comment |
| --- | --- | --- | --- |
| MsgA Configuration | MsgA.1 FR2 | MsgA.2 FR2 |  |
| msgA-prach-ConfigurationIndex | 190 | 190 | Preamble Format C2, 10ms PRACH periodicity and other detailed configuration defined in table 6.3.3.2-2 in TS 38.211 [7]. |
| msgA-SubcarrierSpacing | Same as UL carrier SCS | Same as UL carrier SCS |  |
| msgA-totalNumberOfRA-Preambles | 48 | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | 48 | No group B. |
| msgA-PRACH-RootSequenceIndex | 0 | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| msgA-SSB-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msgA-RO-FDM | One | One | One PRACH transmission occasions FDMed in one time instance. |
| ra-ContentionResolutionTimer | sf48 | - | 48 sub-frames |
| msgA-PreamblePowerRampingStep | dB2 | dB2 |  |
| msgA-PreambleReceivedTargetPower | dBm-120 | dBm-120 |  |
| preambleTransMax | n6 | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| msgB-ResponseWindow | sl10 | sl10 | 10 slots |
| msgA-ZeroCorrelationZoneConfig | 11 | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [12]. |
| ssb-ResourceList | - | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | - | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| ra-ssb-OccasionMaskIndex | - | 1 | PRACH occasion index 1 is allowed |
| msgA-MCS | 1 | 1 | MCS index for MsgA PUSCH |
| nrofSlotsMsgA-PUSCH | 1 | 1 | Number of slots containing one or multiple PUSCH occasions |
| nrofMsgA-PO-PerSlot | 1 | 1 | Number of time domain PUSCH occasions in each slot |
| msgA-PUSCH-TimeDomainOffset | 1 | 1 | A single time offset with respect to the start of each PRACH slot, counted as the number of slots |
| PUSCH start symbol | 0 | 0 |  |
| PUSCH allocation length | 10 | 10 |  |
| mappingTypeMsgA-PUSCH | typeA | typeA |  |
| nrofPRBs-PerMsgA-PO | 2 | 2 | Number of RBs per PUSCH occasion |
| nrofMsgA-PO-FDM | One | One | The number of MsgA PUSCH occasions FDMed in one time instance |
| msgA-DMRS-AdditionalPosition | pos1 | pos1 | Position for additional DM-RS |
| msgA-PUSCH-NrofPorts | 0 | 0 | Configure 1 port per CDM group |
| msgA-DeltaPreamble | 2 | 2 | Power offset of msgA PUSCH relative to the preamble received target power |
| msgA-Alpha | alpha1 | alpha1 | Alpha value for MsgA PUSCH. Set 1 |
| deltaMCS | Disabled | Disabled | Whether to apply delta MCS |
| NOTE: For further information see clause 6.3.2 in TS 38.331 [13]. |  |  |  |

# A.8	Reference BWP configurations

This clause provides the typical BWP configurations used for RRM test cases defined in this test specification. For downlink BWP, both initial BWP and dedicated BWP configurations are specified in clause A.8.1 and for uplink BWP, dedicated BWP configurations are specified in clause A.8.2. Parameters not listed in this clause can be derived from the configuration of each test.

## A.8.1	Downlink BWP configurations

Table A.8.1-1 defines the different downlink initial BWP configurations. Table A.8.1-2 defines the different downlink dedicated BWP configurations.

Table A.8.1-1: Downlink BWP patterns for initial BWP configuration

| BWP Parameters | Unit | Values |  |  |
| --- | --- | --- | --- | --- |
| DL BWP |  | DLBWP.0.1 | DLBWP.0.2 |  |
| Starting PRB index |  | 0 | RBc Note 1 |  |
| Bandwidth |  | Same as RF channel defined in each test | same as RMSI CORSET(CORESET #0) defined in each test |  |
| NOTE 1: RBc is the lowest PRB index to guarantee the BWP including CORESET #0 which is defined in Clause A.1.2. |  |  |  |  |

Table A.8.1-2: Downlink BWP patterns for dedicated BWP configuration

| BWP Parameters | Unit | Values |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL BWP |  | DLBWP.1.1 | DLBWP.1.2 |  |  |  | DLBWP.1.3 |  |  |  | DLBWP.1.4 |  | DLBWP.1.7 |  |  |  |
| Starting PRB index |  | 0 | RBb Note 1 |  |  |  | RBa Note 2 |  |  |  | 0 |  | RBc Note 3 |  |  |  |
| SSB SCS | kHz |  | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 | 120 | 240 | 15 | 30 | 120 | 240 |
| Bandwidth | RB | Same as RF channel defined for the serving cell in each test | 25 | 51 | 32 | 48 | 25 | 51 | 32 | 48 | 24 | 24 | 25 | 51 | 32 | 48 |
| NOTE 1: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 2: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 3: RBc is the PRB index having an offset of 6 PRBs with respect to the start of SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## A.8.2	Uplink BWP configurations

Table A.8.2-1 defines the uplink initial BWP configurations. Table A.8.2-2 defines the uplink dedicated BWP configurations.

Table A.8.2-1: Uplink BWP patterns for initial BWP configurations

| BWP Parameters | Values |  |
| --- | --- | --- |
| UL BWP | ULBWP.0.1 | ULBWP.0.2 |
| Staring PRB index | 0 | RBc Note 1 |
| Bandwidth | Same as RF channel defined in each test | same as RMSI CORESET(CORSET #0) defined in each test |
| NOTE 1: RBc is the lowest PRB index to guarantee the BWP including CORESET #0 which is defined in Clause A.1.2. |  |  |

Table A.8.2-2: Uplink BWP patterns for dedicated BWP configurations

| BWP Parameters | Unit | Values |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL BWP |  | ULBWP.1.1 | ULBWP.1.2 |  |  |  | ULBWP.1.3 |  |  |  | ULBWP.1.4 |  | ULBWP.1.7 |  |  |  |
| Staring PRB index |  | 0 | RBb Note 1 |  |  |  | RBa Note 2 |  |  |  | 0 |  | RBc Note 3 |  |  |  |
| SSB SCS | kHz |  | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 | 120 | 240 | 15 | 30 | 120 | 240 |
| Bandwidth | RB | Same as RF channel defined for the serving cell in each test | 25 | 51 | 32 | 48 | 25 | 51 | 32 | 48 | 24 | 24 | 25 | 51 | 32 | 48 |
| NOTE 1: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 2: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 3: RBc is the PRB index having an offset of 6 PRBs with respect to the start of SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# A.8A	Reference BWP configurations for RedCap

This clause provides the typical BWP configurations used for RedCap RRM test cases defined in Annex A. RedCap downlink dedicated BWP configurations are specified in clause A.8A.1 and RedCap uplink dedicated BWP configurations are specified in clause A.8A.2. Other parameters not listed in this clause can be derived from the setup of each test.

## A.8A.1	Downlink BWP configurations

Table A.8A.1-1: Downlink BWP patterns for RedCap dedicated BWP configuration

| BWP Parameters | Unit | Values |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL BWP |  | DLBWP.1.1 RedCap |  |  |  | DLBWP.1.2 RedCap |  |  |  | DLBWP.1.3 RedCap |  |  |  |
| Starting PRB index |  | RBa Note 1 |  |  |  | RBb Note 2 |  |  |  | RBc Note 3 |  |  |  |
| SSB SCS | kHz | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 |
| Bandwidth | RB | 25 | 25 | 32 | 16 | 25 | 25 | 32 | 16 | 25 | 25 | 32 | 16 |
| NOTE 1: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 2: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 3: RBc is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3A. |  |  |  |  |  |  |  |  |  |  |  |  |  |

## A.8A.2	Uplink BWP configurations

Table A.8A.2-1: Uplink BWP patterns for dedicated BWP configurations

| BWP Parameters | Unit | Values |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL BWP |  | ULBWP.1.1 RedCap |  |  |  | ULBWP.1.2 RedCap |  |  |  | ULBWP.1.3 RedCap |  |  |  |
| Starting PRB index |  | RBa Note 1 |  |  |  | RBb Note 2 |  |  |  | RBc Note 3 |  |  |  |
| SSB SCS | kHz | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 |
| Bandwidth | RB | 25 | 25 | 32 | 16 | 25 | 25 | 32 | 16 | 25 | 25 | 32 | 16 |
| NOTE 1: RBa is the same as RBa for DLBWP.1.1 RedCap. NOTE 2: RBb is the same as RBb for DLBWP.1.2 RedCap. NOTE 3: RBc is the same as RBc for DLBWP.1.3 RedCap. |  |  |  |  |  |  |  |  |  |  |  |  |  |

# A.9	Angle of Arrival (AoA) for FR2 RRM test cases

This clause specifies the AoA setups for FR2 RRM test cases in clause 5 and 7. The applicable AoA setup is defined in each test case in clause 5 and 7.

## A.9.1	Setup 1: Single AoA in Rx beam peak direction

There is only one active probe in the test. The DL signals, and noise if applicable, transmitted from the probe, are aligned to the UE Rx beam peak direction (as defined in TS 38.101-2 [3]).

## A.9.2	Setup 2: Single AoA in non Rx beam peak direction

### A.9.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction

There is only one active probe in the test. The DL signals, and noise if applicable, transmitted from the probe, align to a direction (AoA) which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class. The direction (AoA) of the signals shall not be changed between test iterations.

### A.9.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction

There is only one active probe in the test. The DL signals, and noise if applicable, transmitted from the probe, align to a direction (AoA) which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class. For UE power class 3, the direction (AoA) of the signals shall be changed for each test iteration (for UE power classes other than 3, this is FFS). Unless otherwise specified by the test case, the TE shall alternate between a minimum of 33 different AoAs. The minimum separation between AoAs used in consecutive iterations shall be 30° in great-circle distance.

NOTE:	if it is not possible to find at least 33 different test points meeting the required criteria (FFS), the test shall alternate between all the available test points.

## A.9.3	Setup 3: 2 AoAs

There are 2 active probes in the test. The DL signals, and noise if applicable, transmitted from the two active probes, align to directions (AoAs) which are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class. The relative angular offset between the directions (AoAs) of the 2 active probes, shall be changed for each test iteration. Unless otherwise specified by the test case, the TE shall alternate between a minimum of 33 different test points. A test point comprehends the relative angular offset between the active probes, and the AoA of each of the signals from the UE point of view. The relative angular offset between the active probes shall change for consecutive test points.

The applicable set of relative angular offsets between the 2 active probes is given in Table A.9.3-1 for each UE power class.

Table A.9.3-1: Set of relative angular offsets between active probes for each power class

| UE Power class | Relative angular offset between active probes |
| --- | --- |
| 1 | 30°, 60°, 90° and 120° |
| 2 | FFS |
| 3 | 30°, 60°, 90°, 120° and 150° |
| 4 | FFS |

NOTE:	if it is not possible to find at least 33 different test points meeting the required criteria (that is, the two AoAs are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] and the angular offset between them corresponds to one of the offsets in Table A.9.3-1 for the corresponding UE power class), the test shall alternate between all the available test points.

## A.9.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak

### A.9.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction

There are 2 active probes in the test. The DL signals, and noise if applicable, are transmitted from the two active probes. One probe is aligned to the UE Rx beam peak direction as defined in TS 38.101-2 [3]. The second is aligned to a direction (AoA) which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class. The direction (AoA) of the non Rx beam peak signal shall not be changed between test iterations.

### A.9.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction

There are 2 active probes in the test. The DL signals, and noise if applicable, are transmitted from the two active probes. One probe is aligned to the UE Rx beam peak direction as defined in TS 38.101-2 [3]. The second is aligned to a direction (AoA) which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class.

For UE power class 3, the relative angular offset between the directions (AoAs) of the 2 active probes shall be changed for each test iteration, within the probe alignment described above. The applicable set of relative angular offsets between the 2 active probes is given in Table A.9.3-1 for each UE power class. Unless otherwise specified by the test case, the TE shall alternate between a minimum of 5 different test points. A test point comprehends the relative angular offset between the active probes, and the AoA of each of the signals from the UE point of view, one of them being the Rx beam peak direction. The relative angular offset between the active probes shall change for consecutive test points.

NOTE:	if it is not possible to find at least 5 different test points meeting the required criteria (that is, the two AoAs are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] and the angular offset between them corresponds to one of the offsets in Table A.9.3-1 for the corresponding UE power class), the test shall alternate between all the available test points.

### A.9.5	Setup 5: 2 AoAs for simultaneous reception with QCL Type-D

There are 2 active probes in the test. The DL signals, and noise if applicable, are transmitted from the two active probes. The 2 AoAs (AoA1 and AoA2) for simultaneous reception with different QCL-typeD are from the set of AoA pairs, denoted by (AoA1, AoA2) that can support 2 AoA reception for UE declared AoA angular separation and declared orientation in the positioner of the test system according to the spherical coverage requirement for simultaneous reception from multiple directions as defined in clause 7.3K.3 of TS 38.101-2 [3] for UE power class 3 supporting simultaneous reception from multiple directions. The angular separation between the directions (AoA1 and AoA2) of the 2 active probes is declared from Table 7.3K.3-1 in clause 7.3K.3 of TS 38.101-2 [3] and shall not be changed for each test iteration.

AoA1 needs to satisfy the spherical coverage requirement in Table 7.3.4.3-1 of TS 38.101-2 [3].

NOTE: The chosen AoA pair (AoA1, AoA2) is up to RAN5.

### A.9.6	Setup 6: 3 AoAs for simultaneous reception with different QCL Type-D

There are 3 active probes in the test and the DL signals and noise are transmitted from the three active probes.

Out of the three AoAs, one AoA (AoA1) is aligned to a direction which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3]  for UE power class 3 and other 2 AoAs (AoA2 and AoA3) are from the set of AoA pairs, denoted by (AoA2, AoA3) that can support 2 AoA reception for UE declared AoA angular separation and declared orientation in the positioner of the test system according to the spherical coverage requirement for simultaneous reception from multiple directions as defined in clause 7.3K.3 of TS 38.101-2 [3]  for power class 3 supporting simultaneous reception from multiple directions.

The angular offset between the directions of the AoA pair (AoA2, AoA3) is declared from Table 7.3K.3-1 in clause 7.3K.3 of TS 38.101-2 [3] and shall not be changed for each test iteration.

NOTE: The chosen AoA pair (AoA2, AoA3) is up to RAN5.

## A.9.7	Setup 7: 3 AoAs

There are 3 active probes in the test. The DL signals, and noise if applicable, transmitted from the three active probes, align to directions (AoAs) which are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class. The relative angular offset between the directions (AoAs) of the 3 active probes, shall be changed for each test iteration. Any combinations of two relative angular offsets between 2 active probes specified in Table A.9.3-1, is considered as a valid applicable relative angular offsets between 3 active probes for the respective power class.

# A.10	TCI State Configuration

## A.10.1	Introduction

This clause provides the configurations for TCI states towards either SSB or CSI-RS. The TCI states defined in this clause are configured in each test when applicable to indicate that certain DL signals are quasi-collocated with the referenceSignal configured in the TCI states.

## A.10.2	TCI states

Table A.10.2-1: TCI States

| Parameter | TCI.State.0 | TCI.State.1 | TCI.State.2 | TCI.State.3 |
| --- | --- | --- | --- | --- |
| tci-StateId | Id0 | Id1 | Id2 | Id3 |
| qcl-Type1 | typeC | typeC | typeA | typeA |
| qcl-Type2Note1 | typeD | typeD | typeD | typeD |
| referenceSignal | SSB0 | SSB1 | Resource #4 in TRS resource set 1 Note3 | Resource #4 in TRS resource set 2 Note3 |
| NOTE 1: qcl-Type2 of typeD only where applicable. For RRM test cases, this will be only in FR2 NOTE 2: referenceSignal configurations towards which the TCI states are configured are defined in a test-specific manner. NOTE 3: Reference TRS resource sets are defined in A.1.4A, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. |  |  |  |  |

# A.10A	Unified TCI State Configuration

## A.10A.1	Introduction

This clause provides the configurations for unified TCI states towards either SSB or CSI-RS. The DLorJoint TCI states defined in this clause are configured in each test when applicable to indicate that certain DL (and UL, if joint DL/UL operation is configured) signals are QCL’ed with the referenceSignal configured in the TCI states. The UL TCI states defined in this clause are configured in each test when applicable to indicate that certain UL signals are QCL’ed with the referenceSignal configured in the TCI states.

## A.10A.2	DLorJoint TCI states

Table A.10A.2-1: DLorJoint TCI States

| Parameter | DLorJoint TCI.State.0 | DLorJoint TCI.State.1 | DLorJoint TCI.State.2 | DLorJoint TCI.State.3 | DLorJoint TCI.State.4 | DLorJoint TCI.State.5 |
| --- | --- | --- | --- | --- | --- | --- |
| tci-StateUnifiedId | Id0 | Id1 | Id2 | Id3 | Id4 | Id5 |
| qcl-Type1 | typeA | typeA | typeA | typeA | typeC | typeC |
| qcl-Type2Note1 | typeD | typeD | typeD | typeD | typeD | typeD |
| referenceSignal Note2 | Resource #4 in TRS resource set 1 Note3 | Resource #4 in TRS resource set 2 Note 5 | Resource #4 in TRS resource set 1 Note3 | Resource #4 in TRS resource set 2 Note3 | SSB0 | SSB1 from the cell with different PCI |
| pathlossReferenceRS | N/A | N/A | Resource #4 in TRS resource set 1 Note3 | Resource #4 in TRS resource set 1 Note3 | N/A | N/A |
| additionalPCI | N/A | configured Note4 | N/A | N/A | N/A | configured Note4 |
| Note 1: qcl-Type2 of typeD only where applicable. For RRM test cases, this will be only in FR2 Note 2: referenceSignal configurations towards which the TCI states are configured are defined in a test-specific manner. Note 3: Reference TRS resource sets are defined in A.1.4A, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DLorJoint TCI.State.4. Note 4: Only one PCI than serving cell PCI is included in the additionalPCIList, and the additionalPCIIndex is configured as 0. Note 5: Reference TRS resource sets are defined in A.1.4A, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DLorJoint TCI.State.5. |  |  |  |  |  |  |

## A.10A.3	UL TCI states

Table A.10A.3-1: UL TCI States

| Parameter | UL TCI.State.0 | UL TCI.State.1 | UL TCI.State.2 | UL TCI.State.3 | UL TCI.State.4 | UL TCI.State.5 |
| --- | --- | --- | --- | --- | --- | --- |
| ul-TCIState-Id | Id0 | Id1 | Id2 | Id3 | Id4 | Id5 |
| referenceSignal Note1 | SSB0 | SSB1 | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 2 Note2 | Resource #4 in TRS resource set 3 Note2 | Resource #4 in TRS resource set 4 Note2 |
| pathlossReferenceRS | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 2 Note2 | Resource #4 in TRS resource set 3 Note2 | Resource #4 in TRS resource set 4 Note2 |
| additionalPCI | N/A | configured Note3 | N/A | N/A | N/A | N/A |
| Note 1: referenceSignal configurations towards which the UL TCI states are configured are defined in a test-specific manner. Note 2: Reference TRS resource sets are defined in A.1.4A, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. Note 3: Only one PCI than serving cell PCI is included in the additionalPCIList, and the additionalPCIIndex is configured as 0. |  |  |  |  |  |  |

# A.10B	LTM Candidate TCI State Configuration

## A.10B.1	Introduction

This clause provides the configurations for TCI states of LTM candidate cell(s) towards either SSB or TRS. The LTM candidate DLorJoint TCI states defined in this clause are configured in each test when applicable to indicate that certain DL (and UL, if joint DL/UL operation is configured) signals are QCL’ed with the referenceSignal configured in the TCI states. The UL TCI states defined in this clause are configured in each test when applicable to indicate that certain UL signals are QCL’ed with the referenceSignal configured in the TCI states.

## A.10B.2	LTM candidate DLorJoint TCI states

Table B.10B.2-1: LTM candidate DLorJoint TCI States

| Parameter | Candidate DLorJoint TCI.State.0 | Candidate DLorJoint TCI.State.1 | Candidate DLorJoint TCI.State.2 | Candidate DLorJoint TCI.State.3 |
| --- | --- | --- | --- | --- |
| Candidatetci-StateId | Id0 | Id1 | Id2 | Id3 |
| qcl-Type1 | typeA | typeA | typeA | typeA |
| referenceSignal of qcl-Type 1 Note2 | SSB0 | Resource #4 in TRS resource set 1 Note3 | SSB0 | Resource #4 in TRS resource set 1 Note3 |
| qcl-Type2Note1 | typeD | typeD | typeD | typeD |
| referenceSignal of qcl-Type2 Note2 | SSB0 | Resource #4 in TRS resource set 1 Note3 | SSB0 | Resource #4 in TRS resource set 1 Note3 |
| pathlossReferenceRS | SSB0 | SSB0 | N/A | N/A |
| additionalPCI | N/A | N/A | N/A | N/A |
| Note 1: qcl-Type2 of typeD only where applicable. For RRM test cases, this will be only in FR2 Note 2: referenceSignal configurations towards which the TCI states are configured are defined in a test-specific manner. Note 3: Reference TRS resource sets are defined in A.1.4A, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured for a candidate cell in a test case, it is considered as resource set 1. The TCI state of the TRS is the CandidateDLorJoint TCI.State.2. |  |  |  |  |

## A.10B.3	LTM candidate UL TCI states

Table A.10B.3-1: LTM candidate UL TCI States

| Parameter | Candidate UL TCI.State.0 |
| --- | --- |
| Candidateul-TCIState-Id | Id0 |
| referenceSignal Note1 | SSB0 |
| pathlossReferenceRS | SSB0 |
| additionalPCI | N/A |
| Note 1: referenceSignal configurations towards which the UL TCI states are configured are defined in a test-specific manner. |  |

# A.11	NR sidelink communication

## A.11.1	Introduction

This clause also defines the principle and the reference configurations that are applicable to test cases verifying RRM core requirements for V2X sidelink communication.

## A.11.2	Reference resource pool configurations for NR Sidelink Communication

Table A.11.2-1: NR sidelink SL-BWP configuration

| Field | Value | Comment |
| --- | --- | --- |
| SL-BWP-ConfigCommon-r16 |  |  |
| sl-BWP-Generic-r16 |  |  |
| sl-LengthSymbols-r16 | sym14 | All 14 symbols in a slot without S-SSB are used for sidelink |
| sl-StartSymbol-r16 | sym0 | Symbol #0 is the starting symbol used for sidelink in a slot without S-SSB |
| sl-BWP-PoolConfigCommon-r16 |  |  |
| sl-RxPool-r16 |  | Indicates the resource pool for reception on the configured BWP. 1 entry |
| SL-ResourcePool-r16[1] | Set according to Table A.11.2-2 | Entry 1 |
| sl-TxPoolSelectedNormal-r16 |  | Indicates the resources pool for mode 2 sidelink communication on the configured BWP. 1 entry |
| SL-ResourcePoolConfig-r16[1] |  | Entry 1 |
| sl-ResourcePool-r16 | Set according to Table A.11.2-3 |  |
| sl-TxPoolExceptional-r16 | Not present |  |

Table A.11.2-2:NR sidelink resource pool configuration

| Field | Value | Comment |
| --- | --- | --- |
| SL-ResourcePool-r16 |  |  |
| sl-PSCCH-Config-r16 | Set according to Table A.11.3-1 |  |
| sl-PSSCH-Config-r16 | Set according to Table A.11.3-2 |  |
| sl-PSFCH-Config-r16 | Not present |  |
| sl-SyncAllowed-r16 |  | Indicates the allowed synchronization reference(s) which is (are) allowed to use the configured resource pool. |
| gnss-Sync-r16 | true |  |
| gnbEnb-Sync-r16 | true |  |
| ue-Sync-r16 | true |  |
| sl-SubchannelSize-r16 | n10 | Subchannel bandwidth is 10 RB |
| sl-StartRB-Subchannel-r16 | 0 | The offset of lowest RB index of the subchannel with the lowest index in the resource pool with respect to the lowest RB index of a SL BWP |
| sl-NumSubchannel-r16 | 1 | Number of subchannels for SL transmission |
| sl-UE-SelectedConfigRP-r16 |  |  |
| sl-Thres-RSRP-List-r16 | Set according to the specific test configuration | Indicates a list of 64 thresholds, and the threshold should be selected based on the priority in the decoded SCI and the priority in the SCI to be transmitted. A resource is excluded if it is indicated or reserved by a decoded SCI and PSSCH RSRP in the associated data resource is above a threshold. |
| sl-MultiReserveResource-r16 | Not present |  |
| sl-MaxNumPerReserve-r16 | n2 | At most 2 PSCCH/PSSCH resources can be reserved by a single SCI. |
| sl-SensingWindow-r16 | ms100 | Length of resource sensing window specified in TS 38.214 [9] subclause 8.1.4. which is 100ms. |
| sl-SelectionWindowList-r16 |  | Parameter that determines the end of the selection window for each priority level 8 entries |
| SL-SelectionWindowConfig-r16[k,k=1..8] |  | entry k |
| sl-Priority-r16 | k | priority k |
| sl-SelectionWindow-r16 | n20 | Length of resource selection window specified in TS 38.214 [9] subclause 8.1.4. which is 20∙2μslots, where µ=0,1,2,3 refers to SCS 15,30,60,120 kHz respectively |
| sl-ResourceReservePeriodList-r16 | Not present |  |
| sl-RS-ForSensing-r16 | pssch | PSSCH-RSRP measurement is used in the sensing operation. |
| sl-RxParametersNcell | Not present |  |
| sl-ZoneConfigMCR-List-r16 | Not present |  |
| sl-PreemptionEnable-r16 | enabled |  |
| sl-MinMaxMCS-List-r16 |  | 1 entry |
| SL-MinMaxMCS-Config-r16[1] |  | Entry 1 |
| sl-MCS-Table-r16 | qam64 | TS 38.214 [9] Table 5.1.3.1-1 is the MCS table used in the resource pool. |
| sl-TimeResource-r16 | 11111111111111111111 | Every slot in a period of 20 slots during a SFN or DFN cycle can be used for sidelink |

Table A.11.3-3: NR sidelink UE autonomous resource selection configuration

| Field | Value | Comment |
| --- | --- | --- |
| SL-UE-SelectedConfig-r16 |  |  |
| sl-PSSCH-TxConfigList-r16 |  | 1 entry |
| SL-PSSCH-TxConfig-r16[1] |  | Entry 1 |
| sl-TypeTxSync-r16 | Not present | When this filed is absent, the configuration is applicable for all synchronization reference types. |
| sl-ThresUE-Speed-r16 | kmph200 | UE shall apply the parameters in sl-ParametersAboveThres-r16 if UE absolute speed is higher than 200 km/h, otherwise UE shall apply the parameters in sl-ParametersBelowThres-r16 |
| sl-ParametersAboveThres-r16 |  |  |
| sl-MinMCS-PSSCH-r16 | 0 | The minimum MCS index value can be used for PSSCH transmission. |
| sl-MaxMCS-PSSCH-r16 | 15 | The maximum MCS index value can be used for PSSCH transmission. |
| sl-MinSubChannelNumPSSCH-r16 | 1 | The minimum number of subchannels can be used for PSSCH transmission. |
| sl-MaxSubchannelNumPSSCH-r16 | 1 | The maximum number of subchannels can be used for PSSCH transmission. |
| sl-MaxTxTransNumPSSCH-r16 | 1 | The maximum transmission number for PSSCH (including new transmission and retransmission). |
| sl-MaxTxPower-r16 | Not present | Not applicable |
| sl-ParametersBelowThres-r16 |  |  |
| sl-MinMCS-PSSCH-r16 | 4 | Same as above |
| sl-MaxMCS-PSSCH-r16 | 25 | Same as above |
| sl-MinSubChannelNumPSSCH-r16 | 1 | Same as above |
| sl-MaxSubchannelNumPSSCH-r16 | 1 | Same as above |
| sl-MaxTxTransNumPSSCH-r16 | 1 | Same as above |
| sl-MaxTxPower-r16 | Not present | Same as above |
| sl-ProbResourceKeep-r16 | v0dot8 | The probability of UE keeping current resource is 80% when the resource reselection counter reaches 0 (see TS 38.321 [12]). |
| sl-ReselectAfter-r16 | n1 | Resource reselection is triggered after 1 sidelink transmission is skipped (see TS 38.321 [12]). |

## A.11.3	Reference measurement channels for NR Sidelink Communication

Table A.11.3-1: PSCCH Reference Measurement Channels

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Reference channel |  |  | CC.1A HD |
| Channel bandwidth |  | MHz | Note2 |
| Number of PSCCH symbols per slot |  |  | 2 |
| Number of PSCCH RB |  |  | 10 |
| Modulation |  |  | QPSK |
| Information Bit Payload (without CRC) |  | Bits | 26 |
| Information Bit | Number of DMRS ports |  | 0 (1 port) |
|  | Priority |  | As set by higher layers |
|  | Resource reservation period |  | N/A |
|  | Modulation and coding scheme |  | Set as the PSSCH MCS specified in the test |
|  | DMRS pattern |  | 0 (2 DMRS) |
|  | 2nd stage SCI format |  | 00 (SCI format 2-A) |
|  | Beta offset indicator |  | Set as specified in the test |
|  | Frequency resource assignment |  | Set as per PSSCH RB allocation specific in the test |
|  | Time resource assignment |  | Set as per PSSCH slot allocation specific in the test |
|  | Reserved bits |  | Set all these bits to 0 |
| Transport block CRC |  | Bits | 24 |
| Binary Channel Bits (Note 1) |  | Bits | 360 |
| Note 1: Binary channel bits calculated under assumption of 2 CP-OFDM symbols per subframe. Note 2: Channel bandwidth depends on test configuration. |  |  |  |

Table A.11.3-2: PSSCH Reference Measurement Channels

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | CD.1A HD |
| Sidelink transmission mode |  | 2 |
| Channel bandwidth | MHz | Note1 |
| Allocated PSSCH resource blocks |  | 10 |
| Number of PSSCH symbols per slot |  | 10 |
| Modulation |  | QPSK |
| Target Code Rate |  | 1/3 |
| Information Bit Payload (Transport block size) | Bits | 672 |
| Transport block CRC | Bits | 24 |
| Number of PSSCH HARQ retransmissions |  | 0 |
| Binary Channel Bits | Bits | 2160 |
| Note 1: Channel bandwidth depends on test configuration. Note 2: 2nd state SCI and PSFCH are not allocated per slot. |  |  |

# A.12	Discovery Burst Transmission Window configuration under CCA

## A.12.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms

Table A.12.1-1: DBT.1: DBT Window Pattern 1 for DBT Window period = 20 ms and duration = 1 ms

| SMTC Parameters | Values |
| --- | --- |
| Discovery burst transmission window periodicity | 20 ms |
| Discovery burst transmission window offset | 0 ms |
| Discovery burst transmission window duration | 1 ms |

# A.13	Channel bandwidth (CBW) configurations

## A.13.1	DL UE specific CBW

Table A.13.1-1: DL CBW patterns for UE specific CBW configuration

| BWP Parameters | Unit | Values |  |
| --- | --- | --- | --- |
| Reference CBW |  | DLCBW.1.1 | DLCBW.1.2 |
| OffsetToCarrier | RB | RBy Note 2 | RBx Note 1 |
| carrierBandwidth | RB | Same as RF channel defined in each test | Same as RF channel defined in each test |
| Note 1: RBx is offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier. Note that RBx has to be within the CBW of BS. Note 2: RBy is offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier. Note that RBy has to be within the CBW of BS and different with RBx, and CBW should include SSB and CORESET#0. |  |  |  |

## A.13.2	UL UE specific CBW

Table A.13.2-1: UL CBW patterns for UE specific CBW configuration

| BWP Parameters | Unit | Values |  |
| --- | --- | --- | --- |
| Reference CBW |  | ULCBW.1.1 | ULCBW.1.2 |
| OffsetToCarrier | RB | RBy Note 2 | RBx Note 1 |
| carrierBandwidth | RB | Same as RF channel defined in each test | Same as RF channel defined in each test |
| Note 1: RBx is offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier. Note that RBx has to be within the CBW of BS. Note 2: RBy is offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier. Note that RBy has to be within the CBW of BS and different with RBx. |  |  |  |

# A.14	Satellite access configuration

Table A.14.1-1: SSC.1: Satellite specific configuration pattern 1 for serving cell in GSO scenario

| Parameter | SSC.1 |
| --- | --- |
| Interval between adjacent epoch time | 10.24s |
| ntn-UlSyncValidityDuration | 900s |
| cellSpecificKoffset | 258 slots |
| ta-Common | 0 |
| ta-CommonDrift | 0 |
| ta-CommonDriftVariant | 0 |
| ntn-PolarizationDL | linear |
| ntn-PolarizationUL | linear |
| ephemerisInfo | Detailed ephemeris information is provided in TS 38.508-1 [14] |

Table A.14.1-2: SSC.2: Satellite specific configuration pattern 2 for serving cell in NGSO scenario

| Parameter | SSC.2 |
| --- | --- |
| Interval between adjacent epoch time | 2.56s |
| ntn-UlSyncValidityDuration | 5s |
| cellSpecificKoffset | 14 slots |
| ta-Common | 0 |
| ta-CommonDrift | 0 |
| ta-CommonDriftVariant | 0 |
| ntn-PolarizationDL | linear |
| ntn-PolarizationUL | linear |
| ephemerisInfo | Detailed ephemeris information is provided in TS 38.508-1 [14] |

## A.14.2	Satellite specific configuration for neighbour cell

Table A.14.2-1: NSC.1: Satellite specific configuration pattern 1 for neighbour cell in GSO scenario

| Parameter | NSC.1 |
| --- | --- |
| Interval between adjacent epoch time | 10.24s |
| ntn-UlSyncValidityDuration | 900s |
| cellSpecificKoffset | 258 slots |
| ta-Common | 0 |
| ta-CommonDrift | 0 |
| ta-CommonDriftVariant | 0 |
| ntn-PolarizationDL | linear |
| ntn-PolarizationUL | linear |
| ephemerisInfo | Detailed ephemeris information is provided in TS 38.508-1 [14] |

Table A.14.2-2: NSC.2: Satellite specific configuration pattern 2 for neighbour cell in NGSO scenario

| Parameter | NSC.2 |
| --- | --- |
| Interval between adjacent epoch time | 2.56s |
| ntn-UlSyncValidityDuration | 5s |
| cellSpecificKoffset | 14 slots |
| ta-Common | 0 |
| ta-CommonDrift | 0 |
| ta-CommonDriftVariant | 0 |
| ntn-PolarizationDL | linear |
| ntn-PolarizationUL | linear |
| ephemerisInfo | Detailed ephemeris information is provided in TS 38.508-1 [14] |

# A.15	SRS Configurations

## A.15.1	SRS Configuration for SCS=15kHz

Table A.15.1-1: Sounding Reference Symbol Configuration for SCS=15kHz

|  | SRS.1 TDD | SRS.4 TDD | POS-SRS.1 |  |
| --- | --- | --- | --- | --- |
| Field | Value | Value |  | Comment |
| c-SRS | 12 | 12 | Same as NRB,c in the test case |  |
| b-SRS | 0 | 0 | n.a. |  |
| b-hop | 0 | 0 | n.a. | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | neither | neither | No group or sequence hopping |
| freqDomainPosition | 0 | 0 | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 | 0 | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | 0 | 0 | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching | antennaSwitching | n.a. |  |
| startPosition | 1 | 1 | 5 | resourceMapping setting |
| nrofSymbols | 1 | 1 | 4 | SRS symbols belong to the same SRS resource. |
| repetitionFactor | n1 | n1 | n.a. | without repetition. |
| transmissionComb | n2 | n2 | n4 |  |
| combOffset | 0 | 0 | 0 | transmissionComb setting |
| cyclicShift | 0 | 0 | 0 |  |
| nrofSRS-Ports | port1 | port1 | port1 | Number of antenna ports used for SRS resource transmission |
| resourceType | Periodic | Aperiodic | Periodic |  |
| periodicityAndOffset-p | sl40, 1 | N/A | sl160, 20 | SRS transmission periodicity |

## A.15.2	SRS Configuration for SCS=30kHz

Table A.15.2-1: Sounding Reference Symbol Configuration for SCS=30kHz

|  | SRS.2 TDD | SRS.5 TDD | POS-SRS.2 |  |
| --- | --- | --- | --- | --- |
| Field | Value | Value |  | Comment |
| c-SRS | 24 | 24 | Same as NRB,c in the test case |  |
| b-SRS | 0 | 0 | n.a. |  |
| b-hop | 0 | 0 | n.a. | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | neither | neither | No group or sequence hopping |
| freqDomainPosition | 0 | 0 | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 | 0 | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | 0 | 0 | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching | antennaSwitching | n.a. |  |
| startPosition | 3 | 3 | 5 | resourceMapping setting |
| nrofSymbols | 1 | 1 | 4 | SRS symbols belong to the same SRS resource. |
| repetitionFactor | n1 | n1 | n.a. | without repetition. |
| transmissionComb | n2 | n2 | n4 |  |
| combOffset-n2 | 0 | 0 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 | 0 | 0 |  |
| nrofSRS-Ports | port1 | port1 | port1 | Number of antenna ports used for SRS resource transmission |
| resourceType | Periodic | Aperiodic | Periodic |  |
| periodicityAndOffset-p | sl80, 3 | N/A | Sl320, 40 | SRS transmission periodicity |

## A.15.3	SRS Configuration for SCS=30kHz

Table A.15.3-1: Sounding Reference Symbol Configuration for SCS=120kHz

|  | SRS.3 TDD | SRS.6 TDD | POS-SRS.3 |  |
| --- | --- | --- | --- | --- |
| Field | Value | Value |  | Comment |
| c-SRS | 17 | 17 | Same as NRB,c in the test case |  |
| b-SRS | 0 | 0 | n.a. |  |
| b-hop | 0 | 0 | n.a. | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | neither | neither | No group or sequence hopping |
| freqDomainPosition | 0 | 0 | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 | 0 | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | 0 | 0 | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching | antennaSwitching | n.a. |  |
| startPosition | 1 | 1 | 5 | resourceMapping setting |
| nrofSymbols | 1 | 1 | 4 | SRS symbols belong to the same SRS resource. |
| repetitionFactor | n1 | n1 | n.a. | without repetition. |
| transmissionComb | n2 | n2 | n4 |  |
| combOffset-n2 | 0 | 0 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 | 0 | 0 |  |
| nrofSRS-Ports | port1 | port1 | port1 | Number of antenna ports used for SRS resource transmission |
| resourceType | Periodic | Aperiodic | Periodic |  |
| periodicityAndOffset-p | sl320, 3 | N/A | Sl1280, 160 | SRS transmission periodicity |

# A.16	CSI-RS configurations for RRM

## A.16.1	FDD

Table A.16.1-1: CSI-RS RRM Reference Measurement Channels for SCS=15kHz

|  | CSI-RS.RRM.FR1.1 FDD |
| --- | --- |
| CSI-RS-ResourceConfigMobility |  |
| subcarrierSpacing, kHz | 15 |
| CSI-RS-CellMobility |  |
| cellIdnote1 | 0 |
| nrofPRBs | 48 |
| startPRB | 0 |
| density | 3 |
| CSI-RS-Resource-Mobility |  |
| csi-RS-Index | 0 |
| slotConfig: ms20 note2 | slot1 |
| associatedSSB | True |
| ssb-Index note3 | 0 |
| isQuasiColocated | True |
| firstOFDMSymbolInTimeDomain note4 | 10 |
| sequenceGenerationConfig | 0 |
| Others |  |
| nrofPorts | 1 |
| CDM Type | NoCDM |
| EPRE ratio to SSS, dB | 0 |
| Note1: unless specified otherwise Note2: unless specified otherwise Note3: assume the same SS/PBCH block index of the corresponding cell in the test case Note4: unless specified otherwise |  |

## A.16.2	TDD

Table A.16.2-1: CSI-RS RRM Reference Measurement Channels for SCS=15kHz

|  | CSI-RS.RRM.FR1.1 TDD |
| --- | --- |
| CSI-RS-ResourceConfigMobility |  |
| subcarrierSpacing, kHz | 15 |
| CSI-RS-CellMobility |  |
| cellIdnote1 | 0 |
| nrofPRBs | 48 |
| startPRB | 0 |
| density | 3 |
| CSI-RS-Resource-Mobility |  |
| csi-RS-Index | 0 |
| slotConfig: ms20 note2 | slot1 |
| associatedSSB | True |
| ssb-Index note3 | 0 |
| isQuasiColocated | True |
| firstOFDMSymbolInTimeDomain note4 | 10 |
| sequenceGenerationConfig | 0 |
| Others |  |
| nrofPorts | 1 |
| CDM Type | NoCDM |
| EPRE ratio to SSS, dB | 0 |
| Note1: unless specified otherwise Note2: unless specified otherwise Note3: assume the same SS/PBCH block index of the corresponding cell in the test case Note4: unless specified otherwise |  |

Table A.16.2-2: CSI-RS RRM Reference Measurement Channels for SCS=30kHz

|  | CSI-RS.RRM.FR1.2 TDD |
| --- | --- |
| CSI-RS-ResourceConfigMobility |  |
| subcarrierSpacing, kHz | 30 |
| CSI-RS-CellMobility |  |
| cellIdnote1 | 0 |
| nrofPRBs | 48 |
| startPRB | 0 |
| density | 3 |
| CSI-RS-Resource-Mobility |  |
| csi-RS-Index | 0 |
| slotConfig: ms20 note2 | slot1 |
| associatedSSB | True |
| ssb-Index note3 | 0 |
| isQuasiColocated | True |
| firstOFDMSymbolInTimeDomain note4 | 10 |
| sequenceGenerationConfig | 0 |
| Others |  |
| nrofPorts | 1 |
| CDM Type | NoCDM |
| EPRE ratio to SSS, dB | 0 |
| Note1: unless specified otherwise Note2: unless specified otherwise Note3: assume the same SS/PBCH block index of the corresponding cell in the test case Note4: unless specified otherwise |  |

Table A.16.2-3: CSI-RS RRM Reference Measurement Channels for SCS=120kHz

|  | CSI-RS.RRM.FR2.1 TDD |
| --- | --- |
| CSI-RS-ResourceConfigMobility |  |
| subcarrierSpacing, kHz | 120 |
| CSI-RS-CellMobility |  |
| cellIdnote1 | 0 |
| nrofPRBs | 48 |
| startPRB | 0 |
| density | 3 |
| CSI-RS-Resource-Mobility |  |
| csi-RS-Index | 0 |
| slotConfig: ms20 note2 | slot1 |
| associatedSSB | True |
| ssb-Index note3 | 0 |
| isQuasiColocated | True |
| firstOFDMSymbolInTimeDomain note4 | 10 |
| sequenceGenerationConfig | 0 |
| Others |  |
| nrofPorts | 1 |
| CDM Type | NoCDM |
| EPRE ratio to SSS, dB | 0 |
| Note1: unless specified otherwise Note2: unless specified otherwise Note3: assume the same SS/PBCH block index of the corresponding cell in the test case Note4: unless specified otherwise |  |

# A.17	Reference Cell DTX configurations

## A.17.1	Cell DTX Configuration 1: Cell DTX cycle = 160 ms and TAT = Infinity

Table A.17.1-1: DTX.1: Cell DTX cycle = 160 ms and time alignment timer (TAT) = Infinity

| Field | Value |
| --- | --- |
| cellDTXDRX-onDurationTimer | 1 ms |
| cellDTXDRX-CycleStartOffset | 160 ms |
| cellDTXDRXconfigType-r18 | dtx |
| cellDTXDRXactivationStatus | activated |
| Note: This Cell DTX configuration is applicable for NR serving cell. The Cell DTX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 38.331 [13]. |  |

# A.18	Spatial Relation Configuration

## A.18.1	Introduction

This clause provides the configurations for spatial relation towards either SSB or CSI-RS. The spatial relation defined in this clause are configured in each test when applicable to indicate spatial setting for certain UL signals with the referenceSignal configured in the spatial relation.

## A.18.2	Spatial Relation

Table A.18.2-1: PUCCH Spatial Relation

| Parameter | PUCCH.SRI.0 | PUCCH.SRI.1 |
| --- | --- | --- |
| pucch-SpatialRelationInfoId | Id0 | Id1 |
| referenceSignal | SSB0 | SSB1 |
| PUCCH-PathlossReferenceRS | SSB0 | SSB1 |
| Note 1: referenceSignal configurations towards which the spatial relation are configured in a test-specific manner. |  |  |

Table A.18.2-2: SRS Spatial Relation

| Parameter | SRS.SRI0 | SRS.SRI1 |
| --- | --- | --- |
| srs-SpatialRelationInfoId | Id0 | Id1 |
| referenceSignal | SSB0 | SSB1 |
| Note 1: referenceSignal configurations towards which the spatial relation are configured in a test-specific manner. |  |  |

# A.19	Test Cases involving E-UTRA/FR1 and FR2 carriers

## A.19.1	Introduction

The following applies to UE when undergoing tests with a mix of E-UTRA/NR FR1 and NR FR2 carriers in clauses 5, 7 and 8.

## A.19.2	Principle of Testing in EN-DC

For test cases in clause 5 listed in Table A.19.2-1, the following applies:

-	UE does not have to pass the test case.

Table A.19.2-1: Test cases with the E-UTRA/FR1 – FR2 testability issue (EN-DC)

| Clause | Test case slogan |
| --- | --- |
| 5.5.2.7 | E-UTRAN - NR FR2 interruptions at E-UTRA SRS carrier based switching |
| 5.5.3.6 | Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2 |
| 5.5.6.1.2 | EN-DC FR2 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC |
| 5.6.2.5 | EN-DC FR1-FR2 event-triggered reporting in non-DRX |
| 5.6.2.6 | EN-DC FR1-FR2 event-triggered reporting in DRX |
| 5.6.2.7 | EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection |
| 5.6.2.8 | EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection |
| 5.7.1.3 | EN-DC FR1-FR2 SS-RSRP measurement accuracy |
| Note 1: The list of the test case is based on TS 38.133 [6] Table A.3.13A.2-1 and updated based on the RAN5 testability analysis. |  |

## A.19.3	Principle of Testing in SA

For test cases in clause 7 listed in Table A.19.3-1, the following applies:

-	UE does not have to pass the test case.

Table A.19.3-1: Test cases with the E-UTRA/FR1 – FR2 testability issue (SA)

| Clause | Test case slogan |
| --- | --- |
| 7.3.1.1 | NR SA FR1-FR2 Inter-frequency handover; unknown target cell |
| 7.3.1.4 | NR SA FR1-FR2 synchronous DAPS handover |
| 7.3.1.5 | NR SA FR1-FR2 asynchronous DAPS handover |
| 7.5.3.2 | NR SA FR1-FR2 inter-band SCell activation and deactivation delay |
| 7.5.6.1.2 | NR SA FR1-FR2 DCI-based DL active BWP switch in non-DRX |
| 7.5.7.1 | NR SA FR2 addition and release delay of known PSCell |
| 7.5.7.2 | NR SA FR2 addition and release delay of unknown PSCell |
| 7.5.12.1 | NR SA FR2 Addition and Release Delay of PSCell |
| 7.5.14.1 | NR SA FR2 PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2 |
| 7.6.2.5 | NR SA FR1-FR2 event-triggered reporting in non-DRX |
| 7.6.2.6 | NR SA FR1-FR2 event-triggered reporting in DRX |
| 7.6.2.7 | NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection |
| 7.6.2.8 | NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used |
| 7.6.3.6 | NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used |
| 7.7.1.3 | NR SA FR1-FR2 SS-RSRP measurement accuracy |
| Note 1: The list of the test case is based on TS 38.133 [6] Table A.3.13A.3-1 and updated based on the RAN5 testability analysis. |  |

## A.19.4	Principle of Testing in E-UTRA

For test cases in clause 8 listed in Table A.19.4-1, the following applies:

-	UE does not have to pass the test case.

Table A.19.4-1: Test cases with the E-UTRA/FR1 – FR2 testability issue (E-UTRA)

| Clause | Test case slogan |
| --- | --- |
| 8.2.2.2 | E-UTRA – NR FR2 Early Measurement Reporting |
| 8.4.2.5 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX |
| 8.4.2.6 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX |
| 8.4.2.7 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX |
| 8.4.2.8 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX |
| 8.4.3.1 | E-UTRAN - NR SA FR2 event-triggered reporting without gap with interruption |
| Note 1: The list of the test case is based on TS 38.133 [6] Table A.3.13A.4-1 and updated based on the RAN5 testability analysis. |  |

## A.19.5	Principle of Testing in NR-DC

For test cases in clause 7 listed in Table A.19.5-1, the following applies:

-	UE does not have to pass the test case.

Table A.19.5-1: Test cases with the E-UTRA/FR1 – FR2 testability issue (NR-DC)

| Clause | Test case slogan |
| --- | --- |
| 7.3.3.5 | NR SA FR2 NR conditional handover including target MCG and target SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC |
| Note 1: The list of the test case is based on TS 38.133 [6] Table A.3.13A.5-1 and updated based on the RAN5 testability analysis. |  |

Annex B (normative):
Conditions for RRM requirements applicability for operating bands

# B.1	Conditions for NR RRC_IDLE state mobility

## B.1.1	Introduction

In Annex B.1, the following conditions are specified:

-	UE conditions which shall apply for UE intra-frequency idle state mobility test cases in clauses 6.1 and 7.1,

-	UE conditions which shall apply for UE inter-frequency idle state mobility test cases in clauses 6.1 and 7.1.

-	UE conditions which shall apply for RedCap UE intra-frequency idle state mobility test cases in clauses 16.1 and 17.1.

-	UE conditions which shall apply for RedCap UE inter-frequency idle state mobility test cases in clauses 16.1 and 17.1.

## B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection

Editor’s notes for Table B.1.2-2 (RAN4 dependant):

- The value of Y for power class 4 is FFS, where Y4 is the rough/fine beam gain difference in Rx beam peak direction for power class 4

- The value of Z for power class 4 is FFS, where Z4 is the rough/fine beam gain difference in spherical coverage directions for power class 4

This clause defines the following conditions for NR intra-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.1.2-1 for FR1 NR cells.

The conditions are defined in Table B.1.2-2 for FR2 NR cells.

Table B.1.2-1: Conditions for intra-frequency cell re-selection in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -124 | -121 | -4 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_F | -121.5 | -118.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
|  | NR_FDD_FR1_N | -117.5 | -114.5 |  |
| NOTE 1: NR operating band groups are defined in Clause 3A.4. |  |  |  |  |

Table B.1.2-2: Conditions for intra-frequency cell re-selection in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -107.3 | -110.8 | -109.1 | -124.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -107.3 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  |  | n259 |  |  | -105.5 |  |  |  |
|  |  | n260 | -104.3 |  | -106.5 | -122.8+Y4 |  |  |
|  |  | n261 | -107.3 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -99.3 | -99.8 | -98.2 | -115.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -99.3 | -99.8 | -98.2 | -115.8+Z4 |  |  |
|  |  | n259 |  |  | -92.7 |  |  |  |
|  |  | n260 | -96.3 |  | -93.9 | -110.8+Z4 |  |  |
|  |  | n261 | -99.3 | -99.8 | -98.2 | -115.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

## B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection

This clause defines the following conditions for NR inter-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions defined in Table B.1.2-1 for FR1 NR intra-frequency cell re-selection shall also apply for FR1 NR inter-frequency cells in this clause.

The conditions defined in Table B.1.2-2 for FR2 NR intra-frequency cell re-selection shall also apply for FR2 NR inter-frequency cells in this clause.

## B.1.4	Conditions for measurements on NR intra-frequency cells for cell re-selection for RedCap

This clause defines the following conditions for RedCap NR intra-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.1.4-1 and Table B.1.4-2 for 1 Rx and 2 Rx RedCap respectively for FR1 NR cells.

The conditions are defined in Table B.1.4-3 for FR2 NR cells.

Table B.1.4-1: Conditions for intra-frequency cell re-selection in FR1 for RedCap for 2Rx

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_RC_FR1_A, NR_TDD_RC_FR1_A | -124 | -121 | -4 |
|  | NR_FDD_RC_FR1_B | -123.5 | -120.5 |  |
|  | NR_FDD_RC_FR1_C, NR_TDD_RC_FR1_C | -123 | -120 |  |
|  | NR_FDD_RC_FR1_D, NR_TDD_RC_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_RC_FR1_E, NR_TDD_RC_FR1_E | -122 | -119 |  |
|  | NR_FDD_RC_FR1_F | -121.5 | -118.5 |  |
|  | NR_FDD_RC_FR1_G | -121 | -118 |  |
|  | NR_FDD_RC_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in TS.38.133 [6] clause 3.5.2. |  |  |  |  |

Table B.1.4-2: Conditions for intra-frequency cell re-selection in FR1 for RedCap for 1Rx

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_RC_FR1_A, NR_TDD_RC_FR1_A | -124 | -121 | -4 |
|  | NR_FDD_RC_FR1_B | -123.5 | -120.5 |  |
|  | NR_FDD_RC_FR1_C, NR_TDD_RC_FR1_C | -123 | -120 |  |
|  | NR_FDD_RC_FR1_D, NR_TDD_RC_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_RC_FR1_E, NR_TDD_RC_FR1_E | -122 | -119 |  |
|  | NR_FDD_RC_FR1_F | -121.5 | -118.5 |  |
|  | NR_FDD_RC_FR1_G | -121 | -118 |  |
|  | NR_FDD_RC_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.1.4-3: Conditions for intra-frequency cell re-selection in FR2 for RedCap

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  |  |  |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 7 | 1,2,3, 4,5,7 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | -120.4+Y5 | TBD | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | -120.6+Y5 | TBD |  |  |
|  |  | n261 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  | TBD |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | -112.4+Z5 | TBD | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | -112.6+Z5 | TBD |  |  |
|  |  | n261 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  | TBD |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.1.4-3 (RAN4 dependant):

- The value of Y for power class 4 is FFS, where Y4is the rough/fine beam gain difference in Rx beam peak direction for power class 4

- The value of Z for power class 4 is FFS, where Z4 is the rough/fine beam gain difference in spherical coverage directions for power class 4

## B.1.5	Conditions for measurements on NR inter-frequency cells for cell re-selection for RedCap

This clause defines the following conditions for NR inter-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions defined in Table B.1.4-1 and Table B.1.4-2 for 1 Rx and 2 Rx RedCap respectively for FR1 RedCap NR intra-frequency cell re-selection shall also apply for FR1 RedCap NR inter-frequency cells in this clause.

The conditions defined in Table B.1.4-3 for FR2 RedCap NR intra-frequency cell re-selection shall also apply for FR2 RedCap NR inter-frequency cells in this clause.

## B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access

This clause defines the following conditions for NR intra-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band for satellite access.

The conditions are defined in Table B.1.6-1 for FR1 NR cells.

Table B.1.6-1: Conditions for intra-frequency cell re-selection in FR1 for satellite access

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_SAB_FR1_A | -124 | -121 | -4 |
| NOTE 1: NR operating band groups for satellite access are defined in clause 3A.4.1A. |  |  |  |  |

## B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access

This clause defines the following conditions for NR inter-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band for satellite access.

The conditions defined in Table B.1.6-1 for FR1 NR intra-frequency cell re-selection for satellite access shall also apply for FR1 NR inter-frequency cells for satellite access in this clause.

## B.1.8	Conditions for measurements on NR intra-frequency cells for cell re-selection under CCA

This clause defines the following conditions for NR intra-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.1.8-1 for FR1 NR cells under CCA.

Table B.1.8-1: Conditions for intra-frequency cell re-selection in FR1 under CCA

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_CCA_FR1_I | -120 | -117 | -4 |
|  | NR_CCA_FR1_J | -119.5 | -116.5 |  |
| NOTE 1: NR operating band groups are as defined in clause 3A.4.1. |  |  |  |  |

## B.1.9	Conditions for measurements on NR inter-frequency cells for cell re-selection under CCA

This clause defines the following conditions for NR inter-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions defined in Table B.1.8-1 for FR1 NR intra-frequency cells under CCA shall also apply to FR1 NR inter-frequency cells under CCA in this clause.

# B.2	Conditions for NR RRC_CONNECTED state

## B.2.1	Introduction

In Annex B.2, the following conditions are specified:

-	UE conditions which shall apply for UE intra-frequency measurement procedures and measurement performance tests in clauses 4.6, 4.7, 5.6, 5.7, 6.6, 6.7, 7.6 and 7.7,

UE conditions which shall apply for UE inter-frequency measurements procedures and requirements in Clause 9,

-	UE conditions which shall apply for UE intra-frequency measurements performance requirements in Clause 10,

-	UE conditions which shall apply for UE inter-frequency measurements performance requirements in Clause 10.

## B.2.2	Conditions for NR intra-frequency measurements

Editor’s notes for Table B.2.2-2 (RAN4 dependant):

- The value of Y for power class 4 is FFS, where Y4 is the rough/fine beam gain difference in Rx beam peak direction for power class 4

- The value of Z for power class 4 is FFS, where Z4 is the rough/fine beam gain difference in spherical coverage directions for power class 4

This clause defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.2-2 for FR2 NR cells.

Table B.2.2-1: Conditions for intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_TDD_FR1_C | -126 | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  |
|  | NR_FDD_FR1_F | -124.5 | -121.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -121 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 |  |
|  | NR_FDD_FR1_N | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in Clause 3A.4. |  |  |  |  |

Table B.2.2-2: Conditions for intra-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -110.3 | -113.8 | -112.1 | -127.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -110.3 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  |  | n259 |  |  | -108.5 |  |  |  |
|  |  | n260 | -107.3 |  | -109.5 | -125.8+Y4 |  |  |
|  |  | n261 | -110.3 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -102.3 | -102.8 | -101.2 | -118.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -102.3 | -102.8 | -101.2 | -118.8+Z4 |  |  |
|  |  | n259 |  |  | -95.7 |  |  |  |
|  |  | n260 | -99.3 |  | -96.9 | -113.8+Z4 |  |  |
|  |  | n261 | -102.3 | -102.8 | -101.2 | -118.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and spherical coverage values are increased by ∆MBS,n the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

## B.2.3	Conditions for NR inter-frequency measurements

Editor’s notes for Table B.2.3-2 (RAN4 dependant):

- The value of Y for power class 4 is FFS, where Y4 is the rough/fine beam gain difference in Rx beam peak direction for power class 4

- The value of Z for power class 4 is FFS, where Z4 is the rough/fine beam gain difference in spherical coverage directions for power class 4

This clause defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.3-1 for FR1 NR cells.

The conditions are defined in Table B.2.3-2 for FR2 NR cells.

Table B.2.3-1: Conditions for inter-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_F | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
|  | NR_FDD_FR1_N | -118.5 | -115.5 |  |
| NOTE 1: NR operating band groups are defined in Clause 3A.4. |  |  |  |  |

Table B.2.3-2: Conditions for inter-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -108.3 | -111.8 | -110.1 | -125.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -108.3 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  |  | n259 |  |  | -106.5 |  |  |  |
|  |  | n260 | -105.3 |  | -107.5 | -123.8+Y4 |  |  |
|  |  | n261 | -108.3 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -100.3 | -100.8 | -99.2 | -116.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -100.3 | -100.8 | -99.2 | -116.8+Z4 |  |  |
|  |  | n259 |  |  | -93.7 |  |  |  |
|  |  | n260 | -97.3 |  | -94.9 | -111.8+Z4 |  |  |
|  |  | n261 | -100.3 | -100.8 | -99.2 | -116.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

## B.2.4	Conditions for NR L1-RSRP reporting

### B.2.4.1	Conditions for SSB based L1-RSRP reporting

Editor’s notes for Table B.2.4.1-2 (RAN4 dependant):

- The value of Y for power class 4 is FFS, where Y4 is the rough/fine beam gain difference in Rx beam peak direction for power class 4

- The value of Z for power class 4 is FFS, where Z4 is the rough/fine beam gain difference in spherical coverage directions for power class 4

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.1-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.1-2 for FR2 NR cells.

Table B.2.4.1-1: Conditions for SSB based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_F | -121.5 | -118.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
|  | NR_FDD_FR1_N | -117.5 | -114.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3A.4 |  |  |  |  |

Table B.2.4.1-2: Conditions for SSB based L1-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -107.3 | -110.8 | -109.1 | -124.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -107.3 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  |  | n259 |  |  | -105 |  |  |  |
|  |  | n260 | -104.3 |  | -106.5 | -122.8+Y4 |  |  |
|  |  | n261 | -107.3 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -99.3 | -99.8 | -98.2 | -115.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -99.3 | -99.8 | -98.2 | -115.8+Z4 |  |  |
|  |  | n259 |  |  | -92.3 |  |  |  |
|  |  | n260 | -96.3 |  | -93.9 | -110.8+Z4 |  |  |
|  |  | n261 | -99.3 | -99.8 | -98.2 | -115.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

### B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting

Editor’s notes for Table B.2.4.2-2 (RAN4 dependant):

- The value of Y for power class 4 is FFS, where Y4 is the rough/fine beam gain difference in Rx beam peak direction for power class 4

- The value of Z for power class 4 is FFS, where Z4 is the rough/fine beam gain difference in spherical coverage directions for power class 4

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on CSI-RS: CSI-RS_RP and CSI-RS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.2-2 for FR2 NR cells.

Table B.2.4.2-1: Conditions for CSI-RS based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum CSI-RS_RP |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -118 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 | -117.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 | -117 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 | -116.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 | -116 |  |
|  | NR_FDD_FR1_F | -121.5 | -118.5 | -115.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 | -118 | -115 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 | -114.5 |  |
|  | NR_FDD_FR1_N | -117.5 | -114.5 | -111.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

Table B.2.4.2-2: Conditions for CSI-RS based L1-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum CSI-RS_RP Note 2, Note 3 |  |  |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSCSI-RS |  |  |  |  | dB |
|  |  |  | SCSCSI-RS = 60 kHz |  |  |  | SCSCSI-RS = 120 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -110.3 | -113.8 | -112.1 | -127.8+Y4 | (Value for SCSCSI-RS = 60 kHz) +3dB | ≥-3 |
|  |  | n258 | -110.3 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  |  | n259 |  |  | -108.5 |  |  |  |
|  |  | n260 | -107.3 |  | -109.5 | -125.8+Y4 |  |  |
|  |  | n261 | -110.3 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -102.3 | -102.8 | -101.2 | -118.8+Z4 | (Value for SCSCSI-RS = 60 kHz) +3dB | ≥-3 |
|  |  | n258 | -102.3 | -102.8 | -101.2 | -118.8+Z4 |  |  |
|  |  | n259 |  |  | -95.7 |  |  |  |
|  |  | n260 | -99.3 |  | -96.9 | -113.8+Z4 |  |  |
|  |  | n261 | -102.3 | -102.8 | -101.2 | -118.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum CSI-RS Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

## B.2.5	Conditions for RRC connection release with redirection to NR

Editor’s notes for Table B.2.5.2-2:

- The value of Y for power class 4 is FFS, where Y4 is the rough/fine beam gain difference in Rx beam peak direction for power class 4

- The value of Z for power class 4 is FFS, where Z4 is the rough/fine beam gain difference in spherical coverage directions for power class 4

This clause defines the following conditions for RRC connection release with redirection to NR: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.5-1 for FR1 NR cells.

The conditions are defined in Table B.2.5-2 for FR2 NR cells.

Table B.2.5-1: Conditions for RRC connection release with redirection to NR in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_F | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
|  | NR_FDD_FR1_N | -118.5 | -115.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.5-2: Conditions for RRC connection release with redirection to NR in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -108.3 | -111.8 | -110.1 | -125.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -108.3 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  |  | n259 |  |  | -106.5 |  |  |  |
|  |  | n260 | -105.3 |  | -107.5 | -123.8+Y4 |  |  |
|  |  | n261 | -108.3 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -100.3 | -100.8 | -99.2 | -116.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -100.3 | -100.8 | -99.2 | -116.8+Z4 |  |  |
|  |  | n259 |  |  | -93.7 |  |  |  |
|  |  | n260 | -97.3 |  | -94.9 | -111.8+Z4 |  |  |
|  |  | n261 | -100.3 | -100.8 | -99.2 | -116.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

## B.2.6	Conditions for UE transmit timing

### B.2.6.1	Conditions for SSB based UE transmit timing

Editor’s notes for Table B.2.6.1-2:

- The value of Y for power class 4 is FFS, where Y4 is the rough/fine beam gain difference in Rx beam peak direction for power class 4

- The value of Z for power class 4 is FFS, where Z4 is the rough/fine beam gain difference in spherical coverage directions for power class 4

This clause defines the following conditions for UE transmit timing adjustment performed based on SSBs: SSB_RP and SSB Ês/Iot and applicable for a corresponding operating band.

The conditions are defined in Table B.2.6.1-1 for FR1 SSB.

Table B.2.6.1-1: Conditions for SSB based UE transmit timing in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB=15 kHz | SCSSSB=30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

The conditions are defined in Table B.2.6.1-2 for FR2 SSB.

Table B.2.6.1-2: Conditions for SSB based UE transmit timing in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  |  | n259 |  |  | -105.5 |  |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |
|  |  | n261 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
|  |  | n259 |  |  | -92.7 |  |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 |  |  |
|  |  | n261 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOET 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

## B.2.7	Conditions for NR intra-frequency measurements for RedCap

This clause defines the following conditions for RedCap NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.7-1 and Table B.2.7-2 for 1 Rx and 2 Rx RedCap respectively for FR1 NR cells.

The conditions are defined in Table B.2.7-3 for FR2 NR cells.

Table B.2.7-1: Conditions for intra-frequency cell re-selection in FR1 for RedCap for 1 Rx

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_RC_FR1_A, NR_TDD_RC_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_RC_FR1_B | -126.5 | -123.5 |  |
|  | NR_TDD_RC_FR1_C | -126 | -123 |  |
|  | NR_FDD_RC_FR1_D, NR_TDD_RC_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_RC_FR1_E, NR_TDD_RC_FR1_E | -125 | -122 |  |
|  | NR_FDD_RC_FR1_F | -124.5 | -121.5 |  |
|  | NR_FDD_RC_FR1_G | -124 | -121 |  |
|  | NR_FDD_RC_FR1_H | -123.5 | -120.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.7-2: Conditions for intra-frequency cell re-selection in FR1 for RedCap for 2Rx

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_RC_FR1_A, NR_TDD_RC_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_RC_FR1_B | -126.5 | -123.5 |  |
|  | NR_TDD_RC_FR1_C | -126 | -123 |  |
|  | NR_FDD_RC_FR1_D, NR_TDD_RC_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_RC_FR1_E, NR_TDD_RC_FR1_E | -125 | -122 |  |
|  | NR_FDD_RC_FR1_F | -124.5 | -121.5 |  |
|  | NR_FDD_RC_FR1_G | -124 | -121 |  |
|  | NR_FDD_RC_FR1_H | -123.5 | -120.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.7-3: Conditions for intra-frequency measurements in FR2 for RedCap

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  |  |  |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 7 | 1, 2, 3, 4, 5 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | -123.4+Y5 | TBD | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | -123.6+Y5 | TBD |  |  |
|  |  | n261 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  | TBD |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | -115.4+Z5 | TBD | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | -115.6+Z5 | TBD |  |  |
|  |  | n261 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  | TBD |  |  |
| Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.7-2 (RAN4 dependent):

- The value of Y for power class 4 is FFS, where Y4 is the rough/fine beam gain difference in Rx beam peak direction for power class 4

- The value of Z for power class 4 is FFS, where Z4 is the rough/fine beam gain difference in spherical coverage directions for power class 4

## B.2.8	Conditions for NR inter-frequency measurements for RedCap

This clause defines the following conditions for RedCap NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.8-1 and Table B.2.8-2 for 1 Rx and 2 Rx RedCap for FR1 NR cells.

The conditions are defined in Table B.2.8-3 for FR2 NR cells.

Table B.2.8-1: Conditions for inter-frequency measurements in FR1 for 1 Rx RedCap

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_RC_FR1_A, NR_TDD_RC_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_RC_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_RC_FR1_C | -124 | -121 |  |
|  | NR_FDD_RC_FR1_D, NR_TDD_RC_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_RC_FR1_E, NR_TDD_RC_FR1_E | -123 | -120 |  |
|  | NR_FDD_RC_FR1_F | -122.5 | -119.5 |  |
|  | NR_FDD_RC_FR1_G | -122 | -119 |  |
|  | NR_FDD_RC_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.8-2: Conditions for inter-frequency measurements in FR1 for 2 Rx RedCap

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_RC_FR1_A, NR_TDD_RC_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_RC_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_RC_FR1_C | -124 | -121 |  |
|  | NR_FDD_RC_FR1_D, NR_TDD_RC_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_RC_FR1_E, NR_TDD_RC_FR1_E | -123 | -120 |  |
|  | NR_FDD_RC_FR1_F | -122.5 | -119.5 |  |
|  | NR_FDD_RC_FR1_G | -122 | -119 |  |
|  | NR_FDD_RC_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.8-3: Conditions for inter-frequency measurements in FR2 for RedCap

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  |  |  |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 7 | 1, 2, 3, 4, 5 |  |
| Conditions | Rx Beam Peak | n257 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 | -121.4+Y5 | TBD | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 | -121.6+Y5 | TBD |  |  |
|  |  | n261 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  | TBD |  |  |
|  | Spherical coverage Note 1 | n257 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 | -113.4+Z5 | TBD | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 | -113.6+Z5 | TBD |  |  |
|  |  | n261 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 |  | TBD |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.3-2 (RAN4 dependent):

- The value of Y for power class 4 is FFS, where Y4 is the rough/fine beam gain difference in Rx beam peak direction for power class 4

- The value of Z for power class 4 is FFS, where Z4 is the rough/fine beam gain difference in spherical coverage directions for power class 4

## B.2.9	Conditions for NR intra-frequency measurements under CCA

This clause defines the following conditions for NR intra-frequency measurements under CCA and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.9-1 for NR cells under CCA.

Table B.2.9-1: Conditions for intra-frequency measurements under CCA

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_CCA_FR1_I | -123 | -120 | -6 |
|  | NR_CCA_FR1_J | -122.5 | -119.5 |  |
| NOTE 1: NR operating band groups are as defined in clause 3A.4.1. |  |  |  |  |

## B.2.10	Conditions for NR inter-frequency measurements under CCA

This clause defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.10-1 for NR cells under CCA.

Table B.2.10-1: Conditions for inter-frequency measurements under CCA

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_CCA_FR1_I | -121 | -118 | -4 |
|  | NR_CCA_FR1_J | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are as defined in clause 3A.4.1. |  |  |  |  |

## B.2.11	Conditions for NR L1-RSRP reporting under CCA

### B.2.11.1	Conditions for SSB based L1-RSRP reporting

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on SSBs under CCA: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.11.1-1 for NR cells under CCA.

Table B.2.11.1-1: Conditions for SSB based L1-RSRP measurements under CCA

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_CCA_FR1_I | -120 | -117 | -3 |
|  | NR_CCA_FR1_J | -119.5 | -116.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3A.4.1. |  |  |  |  |

## B.2.12 to B.2.16

## B.2.17	Conditions for NR intra-frequency measurements for satellite access

This clause defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band for satellite access.

The conditions are defined in Table B.2.17-1 for FR1 NR cells.

Table B.2.17-1: Conditions for intra-frequency measurements in FR1 for satellite access

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_SAB_FR1_A | -127 | -124 | -6 |
| NOTE 1: NR operating band groups for satellite access are defined in clause 3A.4.1A. |  |  |  |  |

## B.2.18	Conditions for NR inter-frequency measurements for satellite access

This clause defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band for satellite access.

The conditions are defined in Table B.2.18-1 for FR1 NR cells.

Table B.2.18-1: Conditions for inter-frequency measurements in FR1 for satellite access

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_SAB_FR1_A | -125 | -122 | -4 |
| NOTE 1: NR operating band groups for satellite access are defined in clause 3A.4.1A. |  |  |  |  |

## B.2.19	Conditions for NR L1-RSRP reporting for satellite access

### B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band for satellite access.

The conditions are defined in Table B.2.19.1-1 for FR1 NR cells.

Table B.2.19.1-1: Conditions for SSB based L1-RSRP measurements in FR1 for satellite access

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_SAB_FR1_A | -124 | -121 | -3 |
| NOTE 1: NR operating band groups for satellite access are defined in clause 3A.4.1A. |  |  |  |  |

### B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on CSI-RS: CSI-RS_RP and CSI-RS Ês/Iot, applicable for a corresponding operating band for satellite access.

The conditions are defined in Table B.2.19.2-1 for FR1 NR cells.

Table B.2.19.2-1: Conditions for CSI-RS based L1-RSRP measurements in FR1 for satellite access

| Parameter | NR operating band groups Note1 | Minimum CSI-RS_RP |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |
| Conditions | NR_FDD_SAB_FR1_A | -124 | -121 | -118 | -3 |
| NOTE 1: NR operating band groups for satellite access are defined in clause 3A.4.1A. |  |  |  |  |  |

## B.2.20	Conditions for RRC connection release with redirection to NR for satellite access

This clause defines the following conditions for RRC connection release with redirection to NR: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band for satellite access.

The conditions are defined in Table B.2.20-1 for FR1 NR cells.

Table B.2.20-1: Conditions for RRC connection release with redirection to NR in FR1 for satellite access

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_SAB_FR1_A | -125 | -122 | -4 |
| NOTE 1: NR operating band groups for satellite access are defined in clause 3A.4.1A. |  |  |  |  |

# B.3	RRM requirement exceptions

## B.3.1	Introduction

Annex B.3 covers exceptions for the side conditions based on receiver sensitivity for CA, DC, and SUL.

## B.3.2	Receiver sensitivity relaxation for CA

### B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1

For a UE supporting inter-band carrier aggregation configuration with uplink in NR band, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in TS 38.101-1 [18, Clause 7.3A.3], the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB,c defined for the corresponding downlink NR bands.

For a UE supporting CA configuration in FR1, the requirement in this clause applies for both SC and CA operation.

### B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1

#### B.3.2.2.1	Inter-band carrier aggregation

For a UE configured with inter-band carrier aggregation with active uplink in NR band, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in TS 38.101-1, Clause 7.3A.3 [2], the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB,c defined for the corresponding downlink NR bands.

If the relaxation Δ specified in this clause applies, then the relaxation specified in Clause B.3.2.1 should not be applied.

#### B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA

In this clause, requirements exceptions are described for the UE configured with a band in FR1 when it is impacted by UL harmonic interference from another band in FR1 of the same CA configuration.

A relevant side condition (SSB_RP and Io) in a requirement shall be increased by the amount Δ=L2-L1, where L1 is the reference sensitivity level specified in TS 38.101-1 [2], Clause 7.3.2, and L2 is the reference sensitivity level based on the requirements in TS 38.101-1 [2], Clause 7.3A.4, when the following conditions are fulfilled,

-	corresponding downlink component carriers on different NR bands are configured with CA and active,

-	the uplink is configured in the NR low operating band and is active,

-	the uplink configuration is as specified in TS 38.101-1 [2], Clause 7.3A.4, and

-	the exception requirements specified in TS 38.101-1 [2], Clause 7.3A.4 apply.

If the relaxation Δ specified in this clause applies, then the relaxation specified in Clause B.3.2.1 should not be applied.

#### B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

In this clause, requirements exceptions are described for the UE with an inter-band carrier aggregation with uplink assigned to two NR bands.

A relevant side condition (SSB_RP and Io) in a requirement shall be increased by the amount Δ=L2-L1, where L1 is the reference sensitivity level specified in TS 38.101-1 [2], Clause 7.3.2, and L2 is the reference sensitivity level based on the requirements in TS 38.101-1 [2], Clause 7.3A.5, when the following conditions are fulfilled,

-	corresponding downlink component carriers on different bands are configured with CA and active,

-	uplinks are assigned to two NR bands,

-	the exception requirements specified in TS 38.101-1 [2], Clause 7.3A.5 apply.

If the relaxation Δ specified in this clause applies, then the relaxation specified in Clause B.3.2.1 should not be applied.

### B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2

Editor’s note: TBD

### B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2

#### B.3.2.4.1	Intra-band contiguous carrier aggregation

For a UE configured with intra-band contiguous carrier aggregation in NR band in FR2, if there is a relaxation of receiver sensitivity ΔRIB>0 dB as defined in TS 38.101-2 [3], Clause 7.3A.2.1 depending on the aggregated channel bandwidth, the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB defined for the corresponding downlink NR bands.

#### B.3.2.4.2	Intra-band non-contiguous carrier aggregation

For a UE configured with intra-band non-contiguous carrier aggregation in NR band in FR2, if there is a relaxation of receiver sensitivity ΔRIB>0 dB as defined in TS 38.101-2 [3], Clause 7.3A.2.1 depending on the aggregated channel bandwidth, the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB defined for the corresponding downlink NR bands.

## B.3.3	Receiver sensitivity relaxation for DC

Editor’s note: TBD

## B.3.4	Receiver sensitivity relaxation for SUL

### B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1

For a UE supporting a SUL configuration in FR1, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in TS 38.101-1 [2], Clause 7.3C.3, the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB,c defined for the corresponding downlink NR bands.

For a UE supporting a SUL configuration in FR1, the requirement in this clause applies for both SC and SUL operation.

### B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1

#### B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL

In this clause, requirements exceptions are described for the UE with a band in FR1 when it is impacted by UL harmonic interference from another band in FR1 of the same SUL configuration.

A relevant side condition (SSB_RP and Io) in a requirement shall be increased by the amount Δ=L2-L1, where L1 is the reference sensitivity level specified in TS 38.101-1 [2], Clause 7.3.2, and L2 is the reference sensitivity level based on the requirements in TS 38.101-1 [2], Clause 7.3C.2, when the following conditions are fulfilled,

-	a downlink component carrier is configured in NR band and is active,

-	the uplink is configured in the NR low operating band and is active,

-	the uplink configuration is as specified in TS 38.101-1 [2], Clause 7.3C.2, and

-	the exception requirements specified in TS 38.101-1 [2], Clause 7.3C.2 apply.

If the relaxation Δ specified in this clause applies, then the relaxation specified in Clause B.3.4.1 should not be applied.

# B.4	Conditions for NR sidelink

## B.4.1	Test parameters for GNSS signals

This clause defines the reference signal power levels of generated satellites for a corresponding GNSS, which will be used in NR sidelink and satellite access test cases.

Table B.4.1-1: GNSS Reference Signal Power Parameters

| System | Parameters | Unit | Value |
| --- | --- | --- | --- |
|  | Number of generated satellites per system | - | 6 |
| GPSNote 1 | Reference signal power level for all satellites | dBm | -128.5 |
| Galileo | Reference signal power level for all satellites | dBm | -127 |
| GLONASS | Reference signal power level for all satellites | dBm | -131 |
| BDS | Reference signal power level for all satellites | dBm | -133 |
| NOTE 1: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. NOTE 2: The DUT UE does not need to support all systems. The DUT UE shall support at least one system and will be test for the supported systems. |  |  |  |

## B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements

This clause defines the following conditions for PSBCH-RSRP measurement accuracy requirements applicable for a corresponding operating band.

The conditions are defined in Table B.4.2-1 for FR1.

Table B.4.2-1: Conditions for PSBCH-RSRP measurements in FR1

| Parameter | NR sidelink operating band groups Note1 | Minimum S-SSB_RP |  |  | S-SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/SCSS-SSB |  |  | dB |
|  |  | SCSS-SSB = 15kHz | SCSS-SSB = 30kHz | SCSS-SSB = 60kHz |  |
|  | NR_TDD_FR1_B | -126.5 | -123.5 | -120.5 | -6 |
|  | NR_TDD_FR1_J | -122.5 | -119.5 | -116.5 |  |
| NOTE 1: NR sidelink operating band groups are as defined in Section 3A.4 for the corresponding NR operating bands. |  |  |  |  |  |

## B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE

This clause defines the S-SSB_RP and S-SSB Ês/Iot applicable for a corresponding operating band.

The conditions for selection/reselection to intra-frequency SyncRef UE are defined in Table B.4.3-1 for FR1.

Table B.4.3-1: NR sidelink synchronization measurements in FR1

| Parameter | NR sidelink operating band groups Note1 | Minimum S-SSB_RP |  |  | S-SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/SCSS-SSB |  |  | dB |
|  |  | SCSS-SSB = 15kHz | SCSS-SSB = 30kHz | SCSS-SSB = 60kHz |  |
|  | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | 0 |
|  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | 0 |
| NOTE 1: NR sidelink operating band groups are as defined in Section 3A.4 for the corresponding NR operating bands. |  |  |  |  |  |

## B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements

This clause defines the following conditions for L1 SL-RSRP measurement accuracy requirements applicable for a corresponding operating band.

The conditions are defined in Table B.4.4-1 for FR1.

Table B.4.4-1: Conditions for L1 SL-RSRP measurements in FR1

| Parameter | NR sidelink operating band groups Note1 | Minimum L1 SL-RSRP |  |  | Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/SCS |  |  | dB |
|  |  | SCS= 15kHz | SCS= 30kHz | SCS = 60kHz |  |
|  | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | 0 |
|  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 |  |
| NOTE 1: NR sidelink operating band groups are as defined in Section 3A.4 for the corresponding NR operating bands. NOTE 2: The parameter Ês/Iot is the Ês/Iot of PSCCH-DMRS and/or PSSCH-DMRS. NOTE 3: The SCS is for PSCCH and/or PSSCH. |  |  |  |  |  |

# B.5	High level test procedure for SAN RRM tests

The following high-level steps are conducted for test cases for SAN defined in clause 14.

-	A set of ephemeris information are pre-defined for each satellite corresponding to different epoch times in TS 38.508-1 [14].

-	The same ephemeris information will be maintained during the test (constant ephemerisInfo in all SIB19 updates) i.e., SAN RRM test cases are defined with fixed constant Delay and Doppler shift from satellite access node to UE unless otherwise stated.

-	The range from which the selected constant Delay is selected is as follows:

-	For GSO an altitude of 35,786km is considered. The range of the one-way delay between UE and satellite is from 119.375ms to 128.79ms.

-	For NGSO an altitude of 600km and 1200km on a circular orbit are considered. The range of the one-way delay between UE and satellite is from 2ms (lowest value for LEO orbit 600km) to 6.67ms (highest value for LEO orbit 1200km).

-	The constant Doppler shift (i.e. frequency offset) is derived from the same ephemeris (i.e., orbit emulation) as the Delay.

-	UE location is determined for the test. The ephemeris and the UE location should be designed such that elevation angle relative to the UE position shall not be smaller than 30° during entire test time.

-	Test equipment adjusts the time and frequency of transmission according to pre-defined ephemeris (constant in all SIB19 updates) and UE location.

Annex C (normative):
Downlink physical channels and propagation conditions

# C.1	Downlink physical channels

## C.1.1	General

The following clauses describe the downlink physical channels that are needed for setting a connection and channels that are needed during a connection.

## C.1.2	Default downlink signal levels

The downlink power settings in Table C.1.2-1 and Table C.1.2-2 are used unless otherwise specified in a test case for FR1 cell and FR2 cell, respectively. The downlink power settings in Table C.1.2-1 and Table C.1.2-2 are also used for the initial registration for NR /5GC test cases in clauses 6 and 7. For EN-DC test cases in clauses 4 and 5, the E-UTRA power settings used for initial registration are defined in Annex A.6.

For FR1, if the UE has more than one Rx antenna, the downlink signal is applied to each one. All UE Rx antennas shall be connected.

Table C.1.2-1: Default Downlink power levels for NR (FR1)

| SCS (kHz) | Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidth | MHz | 3 |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 |
| 15 | Number of RBs |  | 12 | 15 | 25 | 50 | 75 | 100 | 128 | 160 | 215 | 270 | N/A | N/A | N/A | N/A |
|  | Channel BW power | dBm | -63 | -62 | -60 | -57 | -55 | -54 | -53 | -52 | -51 | -50 | N/A | N/A | N/A | N/A |
| 30 | Number of RBs |  | N/A | N/A | 10 | 24 | 36 | 50 | 64 | 75 | 100 | 128 | 162 | 216 | 243 | 270 |
|  | Channel BW power | dBm | N/A | N/A | -61 | -57 | -55 | -54 | -53 | -52 | -51 | -50 | -49 | -48 | -47 | -47 |
| 60 | Number of RBs |  | N/A | N/A | N/A | 10 | 18 | 24 | 30 | 36 | 50 | 64 | 75 | 100 | 120 | 135 |
|  | Channel BW power | dBm | N/A | N/A | N/A | -58 | -56 | -54 | -53 | -52 | -51 | -50 | -49 | -48 | -47 | -47 |
|  | RS EPRE | dBm/ 15kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 |
| NOTE 1: The channel bandwidth powers are informative, based on -85dBm/15kHz SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. NOTE 2: The power level is specified at each UE Rx antenna. NOTE 3: DL level is applied for any of the Subcarrier Spacing configuration ( ) with the same power spectrum density of -85dBm/15kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table C.1.2-2: Default Downlink power levels for NR (FR2)

| SCS (kHz) |  | Unit | Channel Bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 60 | Number of RBs |  | 66 | 132 | 264 | N/A |
|  | Channel BW power | dBm | -70 | -67 | -64 | N/A |
| 120 | Number of RBs |  | 32 | 66 | 132 | 264 |
|  | Channel BW power | dBm | -70 | -67 | -64 | -61 |
|  | SS/PBCH SSS EPRE | dBm/60kHz | -99 | -99 | -99 | -99 |
| NOTE 1: The channel bandwidth powers are informative, based on -99dBm/60kHz SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. NOTE 2: The power level is specified at the centre of quiet zone. NOTE 3: DL level is applied for any of the Subcarrier Spacing configuration () with the same power spectrum density of –99dBm/60kHz. |  |  |  |  |  |  |

The default signal level uncertainty for FR1 is +/-3dB at each test port, for any level specified. The default signal level uncertainty for FR2 is +/-6dB at centre of quiet zone, for any level specified. If the uncertainty value is critical for the Test purpose, a tighter uncertainty is specified for the related test case in Annex F

## C.1.3	Default connection setup

Table C.1.3-1 describes the downlink physical channels that are required for NR connection setup. For EN-DC test cases in clauses 4 and 5, the required E-UTRA downlink physical channels are defined in TS 36.521-3 [26] Annex C.2.

Table C.1.3-1: Downlink physical channels required for NR connection set-up

| Physical Channel | EPRE Ratio | Note |
| --- | --- | --- |
| PBCH | PBCH_RA = 0 dB |  |
|  | PBCH_RB = 0 dB |  |
| PSS | PSS_RA = 0 dB |  |
| SSS | SSS_RA = 0 dB |  |
| PDCCH | PDCCH_RA = 0 dB |  |
|  | PDCCH_RB = 0 dB |  |
|  | MPDCCH_RB = 0 dB |  |
| PDSCH | PDSCH_RA = 0 dB |  |
|  | PDSCH_RB = 0 dB |  |
| DMRS | TBD |  |
| CSI-RS | TBD |  |
| NOTE 1: No boosting is applied |  |  |

As common PDSCH and PDCCH configuration parameters the parameters in Table A.1 and C.1.3-2 shall be used to bring up the connection setup for FR1 and FR2 NR cell.

Table C.1.3-2: PDSCH and PDCCH configuration

| Parameter | Unit | Value |
| --- | --- | --- |
| Number of HARQ processes |  | 8 (TDD) 16 (FDD) |
| Maximum number of HARQ transmission |  | 5 |

# C.2	Propagation conditions

## C.2.0	General

The propagation conditions and channel models for various environments are specified. For each environment a propagation model is used to evaluate the propagation pathless due to the distance. Channel models are formed by combining delay profiles with a Doppler spectrum, with the addition of correlation properties in the case of a multi-antenna scenario.

## C.2.1	No interference

The downlink connection between the SS and the UE is without AWGN, and has no fading or multipath effects.

## C.2.2	Static propagation conditions

### C.2.2.0	General

The downlink connection between the SS and the UE is an AWGN environment (unless otherwise stated) with no fading or multipath effects.

### C.2.2.1	UE receiver with 2Rx antenna connectors

For 1 port transmission to UE receiver with 2Rx the channel matrix is defined in the frequency domain by

.

For 2 port transmission to UE Receiver with 2Rx the channel matrix is defined in the frequency domain by

For 4 port transmission to UE Receiver with 2Rx the channel matrix is defined in the frequency domain by

### C.2.2.2	UE receiver with 4Rx antenna connectors

For 1 port transmission to UE receiver with 4Rx the channel matrix is defined in the frequency domain by

.

For 2 port transmission to UE Receiver with 4Rx the channel matrix is defined in the frequency domain by

.

For 4 port transmission to UE Receiver with 4Rx the channel matrix is defined in the frequency domain by

.

## C.2.3	Multi-path fading propagation conditions

Same as defined in TS 38.521-4 [20] clause B.2.

Annex D (normative):
Deviations from standard test configuration

# D.1	Test cases with different numerologies

TBD

# D.2	EN-DC test cases with different EN-DC configurations

## D.2.0	General

In clauses 4 and 5, EN-DC test cases may be defined for two component carriers (CCs) as well as for more than two CCs to verify the same RRM requirement.

## D.2.1	Principle of testing

If multiple EN-DC test cases are defined for two CCs as well as for more than two CCs to verify the same type of RRM requirement, and this requirement is dependent on the number of CCs, then from the UE performance point of view the test coverage can be considered fulfilled by executing only the EN-DC test cases with the maximum number of CCs in EN-DC supported by the UE. Otherwise if the same type of RRM requirement is independent of the number of CCs then from the UE performance point of view the test coverage can be considered fulfilled by executing only the EN-DC test cases with two CCs in EN-DC supported by the UE.

Editor’s Note: The maximum number of CCs that can be used in FR2 tests in EN-DC would depend on the test equipment capability.

# D.3	Carrier aggregation test cases with different CA configurations

## D.3.0	General

In clauses 6 and 7, carrier aggregation test cases may be defined for two CCs as well as for more than two CCs to verify the same RRM requirement.

## D.3.1	Principle of testing

If multiple carrier aggregation test cases are defined for two CCs as well as for more than two CCs to verify the same RRM requirement, and the test requirement is dependent on the number of CCs, then from the UE performance point of view the test coverage can be considered fulfilled by executing only the CA test cases with the maximum number of CCs in CA supported by the UE. Otherwise if the same type of RRM requirement is independent of the number of CCs then from the UE performance point of view the test coverage can be considered fulfilled by executing only the CA test cases with at least two CCs in CA supported by the UE.

Editor’s Note: The maximum number of CCs that can be used in FR2 tests in CA would depend on the test equipment capability.

# D.4	Antenna connection for 4Rx capable UEs

## D.4.0	General

All the tests in this test specification are defined for UEs supporting 2Rx. This clause explains how to apply the 2Rx tests in clauses 4 and 6 to UEs supporting 4Rx antenna ports. No tests are currently specified in clauses 4 or 6 which are applicable only to 4Rx antenna ports, so 4Rx capable UEs are always tested by reusing tests which were originally specified for 2Rx UEs. Please notice that 4Rx is in general not supported for the test cases in clauses 5 and 7.

## D.4.1	Principle of testing

### D.4.1.1	Single carrier tests

For 4Rx capable UEs supporting at least one band where 2Rx is supported and 4Rx is not supported, all single carrier tests specified in clauses 4 and 6, except 4.7 and 6.7 shall be tested with 2Rx on any band where 2Rx is supported and 4Rx is not supported, with the antenna connection defined in D.4.2.1. Single carrier tests specified clauses 4.7 and 6.7 are band dependent and shall be tested in all bands supported by the UE, using 2Rx and the antenna connection defined in D.4.2.1 for the bands where 2Rx is supported and 4Rx is not supported, and the antenna connection defined in D.4.2.2 for the bands where 2Rx is not supported.

For 4Rx capable UEs that do not support any band where 2Rx is supported and 4Rx is not supported, all single carrier tests in clauses 4 and 6 shall be tested with 4Rx using the antenna configuration defined in D.4.2.2.For radio link monitoring tests, the SNR levels are modified according to table D.4.1.1-1 and table D.4.1.1-2

Table D.4.1.1-1: Modified parameters for RLM out of sync testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 | Test 4 |
| 4.5.1.1 | -18 | N/A | N/A | N/A |
| 4.5.1.3 | -18 | N/A | N/A | N/A |
| 4.5.1.5 | -18 | N/A | N/A | N/A |
| 4.5.1.7 | -18 | N/A | N/A | N/A |
| 5.5.1.1 | -18 | N/A | N/A | N/A |
| 5.5.1.3 | -18 | N/A | N/A | N/A |
| 5.5.1.5 | -18 | N/A | N/A | N/A |
| 5.5.1.7 | -18 | N/A | N/A | N/A |
| 6.5.1.1 | -18 | N/A | N/A | N/A |
| 6.5.1.3 | -18 | N/A | N/A | N/A |
| 6.5.1.5 | -18 | N/A | N/A | N/A |
| 6.5.1.7 | -18 | N/A | N/A | N/A |
| 7.5.1.1 | -18 | N/A | N/A | N/A |
| 7.5.1.3 | -18 | N/A | N/A | N/A |
| 7.5.1.5 | -18 | N/A | N/A | N/A |
| 7.5.1.7 | -18 | N/A | N/A | N/A |

Table D.4.1.1-2: Modified parameters for RLM in sync single carrier testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |  | SNR during T4 (dB) |  |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 1 | Test 2 |
| 4.5.1.2 | -18 | N/A | -8 | N/A |
| 4.5.1.4 | -18 | N/A | -8 | N/A |
| 4.5.1.6 | -18 | N/A | -8 | N/A |
| 4.5.1.8 | -18 | N/A | -8 | N/A |
| 5.5.1.2 | -18 | N/A | -8 | N/A |
| 5.5.1.4 | -18 | N/A | -8 | N/A |
| 5.5.1.6 | -18 | N/A | -8 | N/A |
| 5.5.1.8 | -18 | N/A | -8 | N/A |
| 6.5.1.2 | -18 | N/A | -8 | N/A |
| 6.5.1.4 | -18 | N/A | -8 | N/A |
| 6.5.1.6 | -18 | N/A | -8 | N/A |
| 6.5.1.8 | -18 | N/A | -8 | N/A |
| 7.5.1.2 | -18 | N/A | -8 | N/A |
| 7.5.1.4 | -18 | N/A | -8 | N/A |
| 7.5.1.6 | -18 | N/A | -8 | N/A |
| 7.5.1.8 | -18 | N/A | -8 | N/A |

Table D.4.1.1-3: Modified parameters for Beam Failure Detection and Link Recovery testing with 4 RX antenna connection

| Test case | SNR for RS in set q0 during T3, T4 and T5.(dB) |
| --- | --- |
|  | Test 1 |
| 4.5.5.1 | -15 |
| 4.5.5.2 | -15 |
| 4.5.5.3 | -15 |
| 4.5.5.4 | -15 |
| 5.5.5.1 | -15 |
| 5.5.5.2 | -15 |
| 5.5.5.3 | -15 |
| 5.5.5.4 | -15 |
| 6.5.5.1 | -15 |
| 6.5.5.2 | -15 |
| 6.5.5.3 | -15 |
| 6.5.5.4 | -15 |
| 7.5.5.1 | -15 |
| 7.5.5.2 | -15 |
| 7.5.5.3 | -15 |
| 7.5.5.4 | -15 |

### D.4.1.2	Carrier aggregation tests

For carrier aggregation tests, the antenna connection is selected independently for each cell, the PCell and the SCell(s). If a cell (either PCell or any of the SCell(s)) is on a band where 2Rx is supported and 4Rx is not supported, antenna connection in Clause D.4.2.1 shall be used for this cell. If the cell is on a band where 2Rx is not supported, antenna connection in clause D.4.2.2 shall be used for this cell.

### D.4.1.3	EN-DC tests

For all EN-DC tests, the antenna connection is selected independently for each cell. For the E-UTRA PCell, the antenna connection specified in D.4.2.3 shall be used if the PCell is on an E-UTRA band where 2Rx is supported and 4Rx is not supported, and the antenna connection specified in D.4.2.4 shall be used if the PCell is on an E-UTRA band not supporting 2Rx.

For the NR PSCell and SCell(s), the principle of testing is the same as in D.4.1.2.

## D.4.2	Antenna connection

### D.4.2.1	Antenna connection for NR bands where 2Rx is supported

For NR bands where 2Rx is supported and 4Rx is not supported, the UE shall decide via manufacturer declaration and antenna port configuration which 2 of the 4 antenna ports shall be connected with the downlink signal from the SS. The remaining 2 antenna ports shall be connected to zero input. The parameters and test requirements remain unmodified.

### D.4.2.2	Antenna connection for NR bands where only 4Rx is supported

For NR bands where only 4Rx is supported, all 4Rx antenna ports shall be connected to the downlink signal from the SS. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring tests specified in D.4.1.1 and D.4.1.2, the parameters and test requirements remain unmodified.

### D.4.2.3	Antenna connection for E-UTRA bands where 2Rx is supported

For E-UTRAN bands where 2Rx is supported and 4Rx is not supported, the UE shall decide via manufacturer declaration and antenna port configuration which 2 of the 4 antenna ports shall be connected with the downlink signal from the SS. The remaining 2Rx antenna ports shall be connected to zero input. The parameters and test requirements remain unmodified.

### D.4.2.4	Antenna connection for E-UTRA bands where only 4Rx is supported

For E-UTRAN bands where only 4Rx is supported, all 4Rx antenna ports shall be connected to the downlink signal from the SS. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring tests specified in TS 36.521-1 [26] clause 3A.4.1.2.1 and 3A.4.1.2.2, the parameters and test requirements remain unmodified.

# D.4A	Antenna configurations with unlicensed bands for 4Rx capable UEs

## D.4A.1	Introduction

All tests in clauses 10, 11, 12 and 13 are specified for UEs supporting 2RX. In this clause, the antenna connection method for applying 2RX tests to UEs supporting 4RX antenna ports is specified. No tests are currently specified in clauses 10, 11, 12 or 13 which are applicable only to 4RX antenna ports, so 4RX capable UEs are always tested by reusing tests which were originally specified for 2RX UEs.

## D.4A.2	Principle of Testing

### D.4A.2.1	Single Carrier Tests

For 4RX capable UEs supporting at least one 2RX band, the, all single carrier tests specified in clauses 10, 11, 12 and 13 except those in 10.5, 11.6, 12.5 and 13.4, shall be tested on any band where 2RX is supported with the antenna connection specified in D.4A.2.4. For single carrier tests specified in clauses 10.5, 11.6, 12.5 or 13.4, all tests shall be tested with the antenna connection specified in D.4A.2.4 for bands where 2RX is supported, and the antenna connection specified in D.4A.2.5 for bands where 4RX is supported.

For 4RX capable UEs which do not support any 2RX band, all tests specified in clauses 10, 11, 12 and 13 shall be tested using the antenna connection specified in clause D.4A.2.5. For radio link monitoring tests, the SNR levels are modified according to table D.4A.2.1-1 and table D.4A.2.1-2.

Table D.4A.2.1-1: Modified parameters for RLM out of sync testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 | Test 4 |
| 10.3.1.2 | -18 | N/A | N/A | N/A |
| 10.3.1.4 | TBD | N/A | N/A | N/A |
| 11.4.1.2 | -18 | N/A | N/A | N/A |
| 11.4.1.4 | TBD | N/A | N/A | N/A |

Table D.4A.2.1-2: Modified parameters for RLM in sync single carrier testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |  | SNR during T4 (dB) |  |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 1 | Test 2 |
| 10.3.1.3 | -18 | N/A | -8 | N/A |
| 10.3.1.5 | TBD | N/A | TBD | N/A |
| 11.4.1.3 | -18 | N/A | -8 | N/A |
| 11.4.1.5 | TBD | N/A | TBD | N/A |

Table D.4A.2.1-3: Modified parameters for Beam Failure Detection and Link Recovery testing with 4 RX antenna connection

| Test case | SNR for RS in set q0 during T3, T4 and T5 (dB) |  |
| --- | --- | --- |
|  | Test 1 | Test 2 |
| 10.3.4.1 | -15 | N/A |
| 10.3.4.2 | -15 | N/A |
| 11.4.4.1 | -15 | N/A |
| 11.4.4.2 | -15 | N/A |

### D.4A.2.2	Carrier Aggregation Tests

All carrier aggregation tests are performed using the antenna connection in clause D.4A.2.4 for the PCell antenna connection if the PCell is on a band where 2RX is supported or the antenna connection in D.4A.2.5 for the PCell antenna connection if the PCell is on a band where 4RX is supported.

All carrier aggregation tests are performed using the antenna connection in clause D.4A.2.4 for the SCell antenna connection if an SCell is on band where 2RX is supported or the testing procedure in D.4A.2.5 for the SCell antenna connection if an SCell is on a band where 4RX is supported.

### D.4A.2.3	EN-DC Tests

All EN-DC tests are performed using the antenna connection in clause D.4A.2.6 for the PCell antenna connection if the PCell is on a band where 2RX is supported or the antenna connection in D.4A.2.7 for the PCell antenna connection if the PCell is on a band where 4RX is supported.

All EN-DC tests are performed using the antenna connection in clause D.4A.2.4 for the PSCell or SCell antenna connection if an SCell is on band where 2RX is supported or the testing procedure in D.4A.2.5 for the SCell antenna connection if an SCell or PSCell is on a band where 4RX is supported.

### D.4A.2.4	Antenna connection for bands where 2RX is supported

For bands where 2RX is supported, it is left to the UE declaration and AP configuration to decide which 2 of the 4 Rx ports are connected with data source from system simulator. The remaining 2 Rx ports shall be connected with zero input. No test parameters or requirements are modified.

### D.4A.2.5	Antenna connection for bands where 4RX is supported

For bands where 4RX is supported, all 4 RX antennas are connected with data source from system simulator. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring thresholds described in clauses D.4A.2.1 and D.4A.2.2, no test parameters or requirements are modified.

### D.4A.2.6	EN-DC LTE Antenna connection for bands where 2RX is supported

For bands where LTE 2RX is supported, it is left to the UE declaration and AP configuration to decide which 2 of the 4 Rx ports are connected with data source from system simulator. The remaining 2 Rx ports shall be connected with zero input. No test parameters or requirements are modified.

### D.4A.2.7	EN-DC LTE Antenna connection for bands where 4RX is supported

For bands where LTE 4RX is supported, all 4 RX antennas are connected with data source from system simulator. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring thresholds described in clauses A.3.8.1.2.1 and A.3.8.1.2.2 of TS 36.133 [23], no test parameters or requirements are modified.

# D.4B	Antenna connection for 8Rx capable UEs

## D.4B.1	Introduction

All tests in clause 4 and 6 are specified for UEs supporting 2Rx. In this clause, the antenna connection method for applying 2Rx tests or 4Rx tests to UEs supporting 8Rx antenna ports is specified. No tests are currently specified in clause 4 or 6 which are applicable only to 8Rx antenna ports, so 8Rx capable UEs are always tested by reusing tests which were originally specified for 2Rx UEs or 4Rx UEs.

## D.4B.1	Principle of testing

### D.4B.1.1	Single carrier tests

For 8Rx capable UEs supporting at least one band where 2Rx is supported and either 4Rx or 8Rx is not supported, all single carrier tests specified in clause 4 and 6 except those in clause 4.7 and 6.7 shall be tested on any band where 2Rx is supported and 8Rx is not supported with the antenna connection specified in clause D.4B.2.1. For single carrier tests specified in clause 4.7 or 6.7, all tests shall be tested with the antenna connection specified in clause D.4B.2.1 for bands where 2Rx is supported and 8Rx is not supported, and the antenna connection specified in clause D.4B.2.3 for bands where 8Rx is supported.

For 8Rx capable UEs supporting at least one band where 4Rx is supported and 8Rx is not supported but without supporting any band where only 2Rx is supported, all single carrier tests specified in clause 4 and 6 except those in clause 4.7 and 6.7 shall be tested on any band where 4Rx is supported and 8Rx is not supported with the antenna connection specified in clause D.4B.2.2. For single carrier tests specified in clause 4.7 or 6.7, all tests shall be tested with the antenna connection specified in clause D.4B.2.2 for bands where 4Rx is supported and 8Rx is not supported, and the antenna connection specified in D.4B.2.3 for bands where 8Rx is supported. For radio link monitoring tests, the SNR levels are modified according to Table D.4.1.1-1 and Table D.4.1.1-2. For beam failure detection and link recovery tests, the SNR levels are modified according to Table D.4.1.1-3.

For 8Rx capable UEs which do not support any band where 8Rx is not supported, all tests specified in clauses A.4 and A.6 shall be tested using the antenna connection specified in clause D.4B.2.3. For radio link monitoring tests, the SNR levels are modified according to Table D.4.1.1-1 and Table D.4.1.1-2. For beam failure detection and link recovery tests, the SNR levels are modified according to Table D.4.1.1-3.

### D.4B.1.2	Carrier aggregation tests

All carrier aggregation tests are performed using the antenna connection in clause D.4B.2.1 for the PCell antenna connection if the PCell is on a band where 2Rx is supported and either 4Rx or 8Rx is not supported, or using the antenna connection in clause D.4B.2.2 for the PCell antenna connection if the PCell is on a band where 4Rx is supported and 8Rx is not supported, or using the antenna connection in D.4B.2.3 for the PCell antenna connection if the PCell is on a band where 8Rx is supported.

All carrier aggregation tests are performed using the antenna connection in clause D.4B.2.1 for the SCell antenna connection if an SCell is on band where 2Rx is supported and either 4Rx or 8Rx is not supported, or using the antenna connection in clause D.4B.2.2 for the SCell antenna connection if an SCell is on a band where 4Rx is supported and 8Rx is not supported, or using the antenna connection in clause D.4B.2.3 for the SCell antenna connection if an SCell is on a band where 8Rx is supported.

### D.4B.1.3	EN-DC tests

All EN-DC tests are performed using the antenna connection in clause D.4B.2.4 for the PCell antenna connection if the PCell is on a band where 2Rx is supported and either 4Rx or 8Rx is not supported, or using the antenna connection in clause D.4B.2.5 for the PCell antenna connection if the PCell is on a band where 4Rx is supported and 8Rx is not supported, or using the antenna connection in clause D.4B.2.6 for the PCell antenna connection if the PCell is on a band where 8Rx is supported.

All EN-DC tests are performed using the antenna connection in clause D.4B.2.1 for the PSCell or SCell antenna connection if an SCell or PSCell is on band where 2Rx is supported and either 4Rx or 8Rx is not supported, or using antenna connection in clause D.4B.2.2 for the PSCell or SCell antenna connection if an SCell or PSCell is on a band where 4Rx is supported and 8Rx is not supported, or using antenna connection in D.4B.2.3 for the PSCell or SCell antenna connection if an SCell or PSCell is on a band where 8Rx is supported.

## D.4B.2	Antenna connection

### D.4B.2.1	Antenna connection for NR bands where 2Rx is supported

For bands where 2Rx is supported and either 4Rx or 8Rx is not supported, it is left to the UE declaration and antenna port configuration to decide which 2 of the 8 Rx ports are connected with data source from system simulator. The remaining 6 Rx ports shall be connected with zero input. No test parameters or requirements are modified.

### D.4B.2.2	Antenna connection for NR bands where 4Rx is supported

For bands where 4Rx is supported and 8Rx is not supported, it is left to the UE declaration and antenna port configuration to decide which 4 of the 8 Rx ports are connected with data source from system simulator. The remaining 4 Rx ports shall be connected with zero input. Except for the modifications to radio link monitoring thresholds and beam failure detection thresholds described in clause D.4A.2.1, no test parameters or requirements are modified.

### D.4B.2.3	Antenna connection for bands where 8Rx is supported

For bands where 8Rx is supported, all 8 Rx antennas are connected with data source from system simulator. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring thresholds described in clause D.4A.2.1, no test parameters or requirements are modified.

### D.4B.2.4	Antenna connection for E-UTRA bands where 2Rx is supported

For E-UTRAN bands where 2Rx is supported and 4Rx or 8Rx is not supported, it is left to the UE declaration and antenna port configuration to decide which 2 of the 8 Rx ports are connected with data source from system simulator. The remaining 6 Rx ports shall be connected with zero input. No test parameters or requirements are modified.

### D.4B.2.5	Antenna connection for E-UTRA bands where 4Rx is supported

For bands E-UTRAN where 4Rx is supported and 8Rx is not supported, it is left to the UE declaration and antenna port configuration to decide which 4 of the 8 Rx ports are connected with data source from system simulator. The remaining 4 Rx ports shall be connected with zero input. Except for the modifications to radio link monitoring thresholds described in TS 36.133 [23] clause A.3.8.1.2.1 and A.3.8.1.2.2, no test parameters or requirements are modified.

### D.4B.2.6	Antenna connection for E-UTRA bands where 8Rx is supported

For bands E-UTRAN where 8Rx is supported, all 8 Rx antennas are connected with data source from system simulator. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring thresholds described in TS 36.133 [23] clause A.3.8.1.2.1 and A.3.8.1.2.2, no test parameters or requirements are modified.

# D.5	Test Cases with Different Channel Bandwidths

## D.5.1	Test Cases with Different E-UTRA Channel Bandwidths

### D.5.1.1	Introduction

Test cases involving E-UTRA cell(s) may be defined with different E-UTRA channel bandwidths to verify the same type of RRM requirement.

### D.5.1.2	Principle of testing

If multiple test cases involving E-UTRA cell(s) are defined with different E-UTRA channel bandwidths to verify the same type of RRM requirement that is E-UTRA channel bandwidth independent, then the UE needs to be tested with only one channel bandwidth in each E-UTRA cell and with the same bandwidth in all the E-UTRA cells used in the test case.

# D.6	Test Cases for Synchronous and Asynchronous DC Operations

## D.6.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations

### D.6.1.1	Introduction

This clause defines a principle, which is applicable to test cases verifying RRM requirements for EN-DC operation in synchronous and asynchronous scenarios.

Test cases may be defined in both synchronous EN-DC and asynchronous EN-DC scenarios to verify the same type of RRM requirement.

### D.6.1.2	Principle of Testing

If EN-DC test cases are defined in both synchronous and asynchronous EN-DC scenarios to verify the same type of RRM requirement then the UE capable of both synchronous and asynchronous EN-DC operations needs to be tested with one of the tests in either synchronous or asynchronous EN-DC scenarios.

# D.7	CCA model

## D.7.1	Introduction

The CCA model is used in some RRM test cases with at least one cell on a carrier frequency with CCA. The intention with the CCA model is to emulate in the test equipment the behaviour of a gNB or UE which performs channel measurement to check that the channel is clear prior to performing one or more downlink or uplink transmissions.

## D.7.2	CCA model for operation on a carrier frequency with CCA in FR1

### D.7.2.1	DL CCA model

The same DL CCA model is applicable regardless of whether DRX cycle is used or not with the following differences:

-	The counter, lCCA, is used to monitor the number of unavailable DBT samples withing an evaluation window, WCCA_DL. DBT samples outside of the evaluation window WCCA_DL are discarded.

-	If DRX cycle is not used then prior to each DBT window, the test equipment shall determine whether the DL CCA attempt is successful (i.e., the corresponding signals have to be transmitted), based on probability PCCA_DL of successful DL CCA configured in the corresponding test case. If DRX cycle is not used, then the DL CCA model shall increment the counter lCCA for every unavailable DBT sample due to DL CCA failure.

-	If DRX cycle is used, then the DL CCA model shall increment the counter, lCCA, once per DRX cycle for a DRX cycle if the first DBT sample in that DRX cycle is unavailable due to DL CCA failure. DL CCA failures in a DRX cycle are determined as follows:

-	The test system in the first DBT window of each DRX cycle determines whether the DL CCA attempt is successful or not using the principle as follows:

-	If the DL CCA is successful then the test system shall transmit in all DBT windows within that DRX cycle.

-	If the DL CCA is not successful then the test system shall not transmit in any of the DBT windows within that DRX cycle. In this case lCCA is increased by 1.

-	The parameters, LCCA_DL, LCCA_UL, WCCA_DL and WCCA_UL can be used as in non-DRX tests.

If the CCA attempt is successful for a transmission, then the test equipment shall transmit also other remaining transmissions, according to the configuration, within the same DBT window.

If the CCA attempt is not successful for a transmission within the DBT window, the test equipment shall determine whether the CCA attempt is successful for the next configured transmission, based on probability PCCA_DL.

The probability can be different in different time intervals Ti during a test case. One probability value (per cell) applies at any time point during a test; one or more probability values can be configured in the entire test, one value PCCA_DL per time interval Ti where i≥1, and the multiple time intervals (when i>1) do not overlap (e.g., PCCA_DL=1.0 in T1 and PCCA_DL=0.75 in T2).

For semi-static channel access configuration, a single value PCCA_DL is used to configure the probability of CCA success in different time intervals Ti during a test realization. An additional limit LCCA_DL is used to determine the maximum number of unavailable DBT samples within an evaluation window WCCA_DL. If the number of unavailable DBT samples on the last WCCA_DL DBT samples is larger or equal to LCCA_DL, the CCA attempt is considered successful for transmission.

For dynamic channel access configuration, the parameters PCCA_DL_1 and PCCA_DL_2 are used to configure the probability of CCA success on the first and second SSB candidate positions, respectively, in different time intervals Ti during a test realization. An additional limit LCCA_DL is used to determine the maximum number of unavailable DBT samples within an evaluation window WCCA_DL. If the number of unavailable DBT samples on the last WCCA_DL DBT samples is larger or equal to LCCA_DL, the CCA attempt is considered successful for transmission.

For semi-static channel access configuration or for dynamic channel access configuration where one candidate SSB position is modelled, prior to each discovery burst transmission window within a time interval Ti of the test, the test equipment shall:

1 - Generate a uniform random variable p1 from the range [0, 1] for the first candidate position.

2 - Transmit the discovery burst based on p1 in the first candidate position. If p1 ≤ PCCA_DL, the discovery burst is transmitted at the first candidate SSB location; else if lCCA is larger than or equal to LCCA_DL, the discovery burst is transmitted at the first candidate SSB location, otherwise the discovery burst is muted. If DRX cycle is used, then the decision whether the discover burst is muted or not is repeated for the rest of the DRX cycle.

For dynamic channel access configuration where two candidate SSB positions are modelled, prior to each discovery burst transmission window within a time interval Ti of the test, the test equipment shall:

1 - Generate a uniform random variable p1 from the range [0, 1] for the first candidate position.

2 - Transmit the discovery burst based on p1 in the first candidate position: if p1 ≤ PCCA_DL1, the discovery burst is transmitted at first candidate SSB location, else the test equipment shall:

a - Generate a uniform random variable p2 from the range [0, 1] for the second candidate SSB position.

b - Transmit the discovery burst based on p2 in the second candidate position. If p2 ≤ PCCA_DL2, the discovery burst is transmitted at the second candidate SSB location; else if lCCA is larger than or equal to LCCA_DL, the discovery burst is transmitted at the second candidate SSB location, otherwise the discovery burst is muted. If DRX cycle is used, then the decision whether the discover burst is muted or not is repeated for the rest of the DRX cycle.

The above steps are repeated for each discovery burst transmission window in each time interval Ti of the test. The limit LCCA_DL and window WCCA_DL is a configuration parameter for each test case.

In many test cases, the requirement under a test depends on the number of configured SSB transmissions which are not available during the test due to CCA failure, so the test equipment shall track how many such signal occasions are not transmitted in DL during the test period.

### D.7.2.2	UL CCA model

For UL CCA, the modelling approach is based on probability PCCA_UL of successful CCA. Probability PCCA_UL is configured in the corresponding test case, based on a set SCCA_UL of possible values including 75 % and 87% as typical values for dynamic and semi-static channel access configurations, 0% to model consistent UL CCA failures, and 100% to model no UL CCA failures.

Consistent UL CCA failures are modelled by configuring a low value for PCCA_UL, e.g., PCCA_UL = 0%.

In the same time interval Ti during the same test case, PCCA_UL can be different from PCCA_DL.

The probability can be different in different time intervals Ti during a test case. One probability value applies at any time point during a test; one or more probability values can be configured in the entire test, one value PCCA_UL per time interval Ti where I ≥ 1, and the multiple time intervals (when I > 1) do not overlap (e.g., PCCA_UL = 1.0 in T1 and PCCA_UL = 0.75 in T2).

TCCA µs prior to each UL transmission burst in the test, the test equipment (TE) shall generate a uniform random variable p from the range [0, 1]. If p<PCCA_UL, the TE transmits an OCNG noise pattern with an energy level X per LBT measurement BW, within the UE BW scheduled/configured for the UL transmission for at-least TCCA µs. Where TCCA µs is energy detection time for accessing the uplink channel as defined in section 5.1.1 of TS 37.106 [36]. Where:

-	X is 10.5Note1 dB above the energy detection threshold defined in section 5.1.1 of TS 37.106 [36].

-	TCCA is the channel sensing period depending on CCA category for the next UL transmission.

The TE shall count the number of UL CCA failures, and no further UL CCA failures are modelled if the number of failures exceeds the limit LCCA_UL within a window WCCA_UL. For each UL CCA failure generated by the model, the TE shall monitor the corresponding UL resource for the desired UL signal and based on when and/or whether the TE received the desired UL signal, it deems the test case to pass or fail.

In many cases, the requirement under a test depends on the number of configured signal occasions which are not available during the test, so the test equipment shall track how many such signal occasions are not transmitted in UL during the test period.

Note1: Includes measurement uncertainty for OCNG noise as defined in Table F.1.1.2-1.

## D.7.3	Void

# D.8	Test Cases in SA and EN-DC Operations

## D.8.0	General

This clause defines a principle which is applicable to test cases verifying RRM requirements in standalone (SA) or EN-DC operations. RRM test cases may be defined in SA and EN-DC operations to verify the same RRM requirement.

The normative reference for this section is TS 38.133 [6] clause A.3.13.

Editor’s note: this clause may need to define further for NE-DC and NR-DC test cases, which subjects to the test cases defined in the future.

## D.8.1	Principle of testing

If test cases are defined in both SA and EN-DC operations to verify the same RRM requirement then the UE capable of both SA and EN-DC operations needs to verify that RRM requirement by performing test case(s) in either SA operation or in EN-DC operation.

If test cases are defined in both SA and EN-DC operations to verify at least one common RRM requirement then the UE capable of both SA and EN-DC operations needs to verify RRM requirements by performing test case(s) in either SA operation or in EN-DC operation provided that the performed test case(s):

-	verifies the largest number of RRM requirements and

-	verifies at least all RRM requirements covered in the test case(s), which is not performed.

Annex E (normative):
Cell configuration mapping

# E.0	General

The cells used in TS 38.533 do not correspond to the cells defined in TS 38.508-1 [14] clause 4.4.2. This annex describes the mapping between the test cases in TS 38.533 and the cells defined in TS 38.508-1 [14]. The test case shall apply the RF parameters as defined in TS 38.533 according to the column heading.

NOTE:	For example if the second cell in a test case is an inter-frequency cell then Cell3 from TS 38.508-1 [14] clause 4.4.2 is used with the radio parameters as defined for Cell2 in TS 38.533.

# E.1	Test frequency selection

## E.1.0	General

The requirements defined in this test specification comprise EN-DC and NR/5GC test cases. The test cases are defined with a single NR cell or with multiple NR cells. The multi-cell test cases can be either intra-frequency, i.e. the NR cells defined by the test are overlapping in the frequency domain, or inter-frequency, i.e. the NR cells defined by the test have different centre frequencies, separated from each other by a frequency value bigger than the respective cell bandwidths. This clause describes the general rule on how to select the test frequencies for the NR RRM test cases in this test specification.

## E.1.1	E-UTRA PCell for EN-DC test cases

Unless otherwise stated, the E-UTRA PCell for EN-DC test cases shall be configured using the test frequency “Mid” as defined in TS 36.508 [25] for the corresponding E-UTRA band.

In case that the “Mid” test frequency overlaps with any of the NR test frequencies required by the test case, the E-UTRA PCell shall be shifted to an additional frequency within the E-UTRA same band. If the E-UTRA band channel bandwidth is not sufficient to allocate a non-overlapping E-UTRA PCell, the auxiliary band as defined in TS 36.521-3 [26] clause 3 shall be used.

## E.1.2	Test cases with one NR cell

Unless otherwise stated, for NR test cases with one NR cell, this cell shall be configured using the test frequency “Mid” as defined in TS 38.508-1 [14] for the corresponding band under test.

## E.1.3	Test cases with more than one NR cell

### E.1.3.1	Intra-frequency test cases

Unless otherwise stated, multi-cell intra-frequency test cases shall be tested using the test frequency “Mid” as defined in TS 38.508-1 [14] for the corresponding NR band under test.

### E.1.3.2	Inter-frequency test cases

For NR/5GC and EN-DC multi-cell inter-frequency test cases in FR1, unless otherwise stated, the serving cell shall be configured using the test frequency “Low” as defined in TS 38.508-1 [14] for the corresponding band under test. Any inter-frequency neighbour cell shall be configured using the test frequency “High” as defined in TS 38.508-1 [14] for the corresponding band under test. For the 2nd inter-frequency neighbour cell shall be configured using the test frequency “Mid” as defined in TS 38.508-1 [14] for the corresponding band under test.

For NR/5GC multi-cell inter-frequency test cases in FR2, unless otherwise stated, the serving cell (and any other neighbour cell in the same frequency carrier) shall be configured using the test frequency “Mid” as defined in TS 38.508-1 [14], clause 7.2.3.2 for the corresponding band under test. Any inter-frequency neighbour cell shall be configured using a non-overlapping test frequency adjacent to the serving cell frequency, as defined in TS 38.508-1 [14], clause 7.2.3.2.

For EN-DC multi-cell inter-frequency test cases in FR2, unless otherwise stated, the PSCell (and any other neighbour cell in the same frequency carrier) shall be configured using the test frequency “Mid” as defined in TS 38.508-1 [14] , clause 7.2.3.2 for the corresponding band under test. Any inter-frequency neighbour cell shall be configured using a non-overlapping test frequency adjacent to the PSCell frequency, as defined in TS 38.508-1 [14] , clause 7.2.3.2.

## E.1.4	Carrier aggregation test cases

### E.1.4.1	Inter-band carrier aggregation

For inter-band carrier aggregation test cases, each of the component carriers and their respective neighbour cells shall be configured following the same principles defined in E.1.2 and E.1.3.

### E.1.4.2	Intra-band contiguous carrier aggregation

For intra-band contiguous carrier aggregation with 2 CCs in FR1, unless otherwise specified, the serving cells (and any other neighbour cell(s) in the same frequency carriers) shall be configured using the test frequency “Low” as defined in TS 38.508-1 [14], subclause 4.3.1.1.3 for the corresponding band under test (SpCC = CC1 and SCC = CC2).

For intra-band contiguous carrier aggregation with 2 CCs in FR2, unless otherwise specified, the serving cells (and any other neighbour cell(s) in the same frequency carriers) shall be configured using the test frequency “Mid” as defined in TS 38.508-1 [14], subclause 4.3.1.2.3 for the corresponding band under test (SpCC = CC1 and SCC = CC2).

### E.1.4.3	Intra-band non-contiguous carrier aggregation

For intra-band non-contiguous carrier aggregation in FR1, unless otherwise specified, the test frequency selection shall be done following the maximum Wgap principle, i.e. selecting the test frequencies (of the test frequencies defined in TS 38.508-1 [14]) with the widest frequency separation within the band under test.

For intra-band non-contiguous carrier aggregation in FR2, the test frequency selection is TBD.

## E.1.5	E-UTRA – NR inter RAT test cases

Unless otherwise stated, the E-UTRA serving/neighbour cell for E-UTRA – NR inter-RAT test cases shall be configured using the test frequency “Mid” as defined in TS 36.508 [25] for the corresponding E-UTRA band.

In case that the “Mid” test frequency overlaps with any of the NR test frequencies required by the test case, the E-UTRA cell shall be shifted to an additional frequency within the E-UTRA same band. If the E-UTRA band channel bandwidth is not sufficient to allocate a non-overlapping E-UTRA cell, the auxiliary band as defined in TS 36.521-3 [26] clause 3 shall be used.

## E.1.6	Intra-band EN-DC test cases

### E.1.6.1	Intra-band non-contiguous EN-DC

For intra-band non-contiguous EN-DC in FR1, unless otherwise specified, the test frequency selection shall be done following the maximum Wgap principle, i.e. selecting the test frequencies (of the test frequencies defined in TS 38.508-1 [14]) with the widest frequency separation within the band under test.

#### E.1.6.1.1	Inter-frequency neighbour cell

Unless otherwise stated, any inter-frequency neighbour cell shall be configured using the test frequency “Mid” as defined in TS 38.508-1 [14] for the corresponding NR band under test. If the NR band channel bandwidth is not sufficient to allocate a non-overlapping the neighbour cell, the auxiliary band as defined in TBD shall be used.

### E.1.6.2	Intra-band contiguous EN-DC

#### E.1.6.1.1	E-UTRA PCell

Unless otherwise stated, the E-UTRA PCell for intra-band contiguous EN-DC test cases shall be configured using same test frequency, e.g., “Mid” as for NR PSCell as defined in TS 38.508-1 [14].

## E.1.7	NR sidelink test cases

Unless otherwise stated, for NR sidelink test cases the NR sidelink carrier shall be configured using the test frequency “Mid” as defined in TS 38.508-1 [14] for the corresponding band under test.

For con-current operation, unless otherwise stated, each of the NR Uu carriers and their respective cells shall be configured following the sample principles defined in E.1.2 and E.1.3.

# E.2	Cell configuration mapping for EN-DC FR1 test cases in Chapter 4

Table E.2-1 defines the cell configuration mapping for EN-DC FR1 test cases in chapter 4 of this test specification.

Table E.2-1: Cell configuration mapping for EN-DC FR1 RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 4.3.2.2.1 | EN-DC FR1 contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.3.2.2.2 | EN-DC FR1 non-contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.3.2.2.3 | EN-DC FR1 2-step contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.3.2.2.4 | EN-DC FR1 2-step non-contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.4.1.1 | EN-DC FR1 UE transmit timing accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.4.3.1 | EN-DC FR1 timing advance adjustment accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.1 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.2 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.3 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM-RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.4 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM-RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.5 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM-RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.6 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM-RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.7 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM-RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.8 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM-RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.9 | EN-DC FR1 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM-RS for UE fulfilling relaxed measurement criterion | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.2.1 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.2.2 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.2.3 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.2.4 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.2.5 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 4.5.2.6 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 4.5.2.8 | EN-DC FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.2.9 | EN-DC FR1 interruptions at E-UTRA SRS carrier based switching | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 4.5.2.10 | EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.3.1 | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.3.2 | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.3.3 | EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.3.5 | EN-DC FR1 direct SCell activation at SCell addition of known SCell | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.3.6 | EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.3.7 | EN-DC FR1 fast SCell Activation of known SCell for 640 ms SCell measurement cycle | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
| 4.5.3.11 | EN-DC FR1 TRS-based SCell Activation of SSB-less SCell in FR1 collocated inter-band in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 10 |  | inter-band |
| 4.5.3.12 | EN-DC FR1 Inter-band SSB-less SCell activation using A-TRS | LTE Cell 1 | NR Cell 1 | NR Cell 10 |  | inter-band |
| 4.5.4.1 | EN-DC FR1 UE UL carrier RRC reconfiguration delay | LTE Cell 1 | NR Cell 1 | NR Cell 33 |  |  |
| 4.5.5.1 | EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.2 | EN-DC FR1 SSB-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.3 | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.4 | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.5 | EN-DC FR1 SCell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.6 | EN-DC FR1 SCell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.7 | EN-DC FR1 PSCell TRP specific SSB-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.8 | EN-DC FR1 SCell TRP specific CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  |  |
| 4.5.6.1.1 | EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.6.1.2 | EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.5.6.2.1 | EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.6.3.1 | Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  |  |
| 4.5.6.5.1 | E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  |  |
| 4.5.7.1 | EN-DC FR1 addition and release delay of known PSCell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.8.1 | EN-DC FR1 interruptions at switching between two uplink carriers | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.9.1 | UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.10.1 | EN-DC FR1 PSCell activation and deactivation delay | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.11.1 | EN-DC FR1 Addition and Release Delay of PSCell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.1.1 | EN-DC FR1 event-triggered reporting without gap in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.2 | EN-DC FR1 event-triggered reporting without gap in DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.3 | EN-DC FR1 event-triggered reporting with gap in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.4 | EN-DC FR1 event-triggered reporting with gap in DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.5 | EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.6 | EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.7 | EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.8 | EN-DC FR1 event triggered reporting cell without SSB time index detection in DRX for UE configured with highSpeedMeasCA-SCell-r17 | LTE Cell 1 | NR Cell 1 | NR Cell 6 | NR Cell 13 | Intra-band |
|  |  |  |  | NR Cell 10 | NR Cell 30 | Inter-band |
| 4.6.2.1 | EN-DC FR1-FR1 event-triggered reporting in non-DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.6.2.2 | EN-DC FR1-FR1 event-triggered reporting in DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.6.2.5 | EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.6.2.6 | EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.6.2.9 | EN-DC FR1-FR1 event triggered reporting without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.6.4.1 | EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.4.2 | EN-DC FR1 SSB-based L1-RSRP measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.4.3 | EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.4.4 | EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.5.1 | EN-DC FR1 SRS-RSRP measurement with non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.7.1 | EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.7.2 | EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.7.3 | EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.1.1.1 | EN-DC FR1 SS-RSRP absolute measurement accuracy | LTE Cell 1 | NR Cell 489 | NR Cell 1 |  |  |
| 4.7.1.1.2 | EN-DC FR1 SS-RSRP relative measurement accuracy | LTE Cell 1 | NR Cell 489 | NR Cell 1 |  |  |
| 4.7.1.2.1 | EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.1.2.2 | EN-DC FR1-FR1 SS-RSRP relative measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.2.1 | EN-DC FR1 SS-RSRQ measurement accuracy | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.7.2.2.1 | EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.2.2.2 | EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.3.1 | EN-DC FR1 SS-SINR measurement accuracy | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.7.3.2.1 | EN-DC FR1-FR1 SS-SINR absolute measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.3.2.2 | EN-DC FR1-FR1 SS-SINR relative measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.4.1.1 | EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.4.1.2 | EN-DC FR1 SSB based L1-RSRP relative measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.4.2.1 | EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.4.2.2 | EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.5.1 | EN-DC FR1 SFTD measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.6.1 | EN-DC SRS-RSRP measurement accuracy with FR1 serving cell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.7.1 | EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.7.2 | EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.7.3 | EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.8.1.1 | EN-DC intra-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell | LTE Cell 1 | NR Cell 489 | NR Cell 1 |  |  |
| 4.7.8.1.2 | EN-DC intra-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell | LTE Cell 1 | NR Cell 489 | NR Cell 1 |  |  |
| 4.7.8.2.1 | EN-DC inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.8.2.2 | EN-DC inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| NOTE 1: Intra-frequency cells are selected such that serving and neighbour cells satisfy (PCIneighbor-PCIserving) mod 3 ≠0. |  |  |  |  |  |  |

# E.3	Cell configuration mapping for EN-DC FR2 test cases in Chapter 5

Table E.3-1 defines the cell configuration mapping for EN-DC FR2 test cases in chapter 5 of this test specification.

Table E.3-1: Cell configuration mapping for EN-DC FR2 RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 5.3.2.2.1 | EN-DC FR2 contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.3.2.2.2 | EN-DC FR2 non-contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.3.2.2.3 | EN-DC FR2 2-step contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.3.2.2.4 | EN-DC FR2 2-step non-contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.4.1.1 | EN-DC FR2 UE transmit timing accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.4.3.1 | EN-DC FR2 timing advance adjustment accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.1 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode | LTE Cell 1 | NR Cell 6 |  |  |  |
| 5.5.1.2 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode | LTE Cell 1 | NR Cell 6 |  |  |  |
| 5.5.1.3 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM-RS in DRX mode | LTE Cell 1 | NR Cell 6 |  |  |  |
| 5.5.1.4 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM-RS in DRX mode | LTE Cell 1 | NR Cell 6 |  |  |  |
| 5.5.1.5 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM-RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.6 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM-RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.7 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM-RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.8 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM-RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.9 | EN-DC FR2 radio link monitoring UE scheduling restrictions | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.10 | EN-DC FR2 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM-RS for UE fulfilling relaxed measurement criterion | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.2.1 | EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.2.2 | EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.2.3 | EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 5.5.2.4 | EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 5.5.2.5 | EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 5.5.2.6 | EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 5.5.2.7 | EN-DC FR2 interruptions at E-UTRA SRS carrier based switching | LTE Cell 1 | NR Cell 1 | LTE Cell 3 |  |  |
| 5.5.2.8 | EN-DC FR2 interruptions at NR SRS carrier based switching | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 5.5.3.1 | EN-DC FR2 SCell activation and deactivation intra-band in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
| 5.5.3.7 | EN-DC FR2 direct SCell activation at SCell addition of known SCell | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
| 5.5.3.8 | EN-DC FR2 fast SCell Activation of SCell in FR2 intra-band | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
| 5.5.5.1 | EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.2 | EN-DC FR2 SSB-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.3 | EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.4 | EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.5 | EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.6 | EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  |  |
| 5.5.5.7 | EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  |  |
| 5.5.5.8 | EN-DC FR2 PSCell CSI-RS-based TRP specific Beam Failure Detection and Link Recovery in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.9 | EN-DC FR2 SSB-based beam failure detection and link recovery in DRX mode for UE fulfilling relaxed measurement criterion | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.6.2.1 | EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.6.3.1 | E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.5.6.4.1 | E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.5.7.1 | Addition and Release Delay of NR PSCell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.8.1 | EN-DC FR2 MAC-CE based active TCI state switch | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.8.2 | EN-DC FR2 RRC based active TCI state switch | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.9.1.1 | EN-DC PSCell FR2 uplink spatial relation switch for a known spatial relation | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.9.2.1 | EN-DC PSCell FR2 uplink spatial relation switch associated with a known DL-RS | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.11.1 | EN-DC FR2 MAC-CE based active joint TCI state switch | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.11.2 | EN-DC FR2 MAC-CE based active uplink TCI state switch | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.11.3 | EN-DC FR2 MAC-CE based active downlink TCI state switch | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  |  |
| 5.5.12.1 | EN-DC FR2 PSCell activation and deactivation delay | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.13.1 | EN-DC FR2 Addition and Release Delay of NR PSCell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.1.1 | EN-DC FR2 event-triggered reporting without gap in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.6.1.2 | EN-DC FR2 event-triggered reporting without gap in DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.6.1.3 | EN-DC FR2 event-triggered reporting with gap in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.6.1.4 | EN-DC FR2 event-triggered reporting with gap in DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.6.2.1 | EN-DC FR2-FR2 event-triggered reporting in non-DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.2 | EN-DC FR2-FR2 event-triggered reporting in DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.3 | EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.4 | EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.5 | EN-DC FR1-FR2 event-triggered reporting in non-DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.6 | EN-DC FR1-FR2 event-triggered reporting in DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.7 | EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.8 | EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.3.1 | EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.3.2 | EN-DC FR2 SSB-based L1-RSRP measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.3.3 | EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.3.4 | EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.6.1 | EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.6.2 | EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.6.3 | EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.7.1 | EN-DC event triggered reporting test without gap under non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.6.8.1 | EN-DC event triggered reporting tests for NR FR2 cell when DRX is used | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.7.1.1 | EN-DC FR2 SS-RSRP measurement accuracy | LTE Cell 1 | NR Cell 489 | NR Cell 1 |  |  |
| 5.7.1.2 | EN-DC FR2-FR2 SS-RSRP measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.7.1.3 | EN-DC FR1-FR2 SS-RSRP measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.7.2.1 | EN-DC FR2 SS-RSRQ measurement accuracy | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.7.2.2 | EN-DC FR2-FR2 SS-RSRQ measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.7.3.1 | EN-DC FR2 SS-SINR measurement accuracy | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.7.3.2 | EN-DC FR2-FR2 SS-SINR measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.7.6.1 | EN-DC FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.7.6.2 | EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.7.6.3 | EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.7.7.1 | EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | LTE Cell 1 | NR Cell 489 | NR Cell 1 |  |  |
| 5.7.7.2 | EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |

# E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6

Table E.4-1 defines the cell configuration mapping for NR/5GC FR1 test cases in chapter 6 of this test specification.

Table E.4-1: Cell configuration mapping for SA FR1 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 |  | 38.533 NR Cell3 |  | 38.533 NR Cell4 |  | CA Type |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.1.1.1 | NR SA FR1 cell re-selection | NR Cell 1 | NR Cell 11 |  |  |  |  |  |  |  |
| 6.1.1.2 | NR SA FR1-FR1 cell re-selection | NR Cell 6 | NR Cell 23 |  |  |  |  |  |  |  |
| 6.1.1.3 | NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 11 |  |  |  |  |  |  |  |
| 6.1.1.4 | NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 11 |  |  |  |  |  |  |  |
| 6.1.1.5 | NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 6 | NR Cell 23 |  |  |  |  |  |  |  |
| 6.1.1.6 | NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 6 | NR Cell 23 |  |  |  |  |  |  |  |
| 6.1.1.7 | NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | NR Cell 11 |  |  |  |  |  |  |  |
| 6.1.1.8 | NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17 | NR Cell 1 | NR Cell 11 |  |  |  |  |  |  |  |
| 6.1.1.9 | NR SA FR1 cell re-selection for UE operating on a cell with less than 5MHz BW | NR Cell 1 | NR Cell 11 |  |  |  |  |  |  |  |
| 6.2.1 | NR SA FR1 TA Validation for CG-SDT | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.3.1.1 | NR SA FR1 handover with known target cell | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.3.1.2 | NR SA FR1 handover with unknown target cell | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.3.1.3 | NR SA FR1-FR1 Handover with unknown Target Cell | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.3.1.7 | NR SA FR1 synchronous DAPS handover | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.3.1.8 | NR SA FR1 asynchronous DAPS handover | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.3.1.9 | NR SA FR1 Intra-band inter-frequency synchronous DAPS handover | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.3.1.10 | NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.3.1.11 | NR SA FR1 Inter-band inter-frequency synchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |  |  |  |  |
| 6.3.1.12 | NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |  |  |  |  |
| 6.3.1.18 | NR SA FR1 handover with unknown target cell operating with 12 PRB SSB bandwidth | NR Cell 6 | NR Cell 23 |  |  |  |  |  |  |  |
| 6.3.2.1.1 | NR SA FR1 RRC re-establishment | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.3.2.1.2 | NR SA FR1 - FR1 RRC re-establishment | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.3.2.1.3 | NR SA FR1 RRC re-establishment without serving cell timing | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.3.2.2.1 | NR SA FR1 contention based random access | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.3.2.2.2 | NR SA FR1 non-contention based random access | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.3.2.2.3 | NR SA FR1 2-step contention based random access | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.3.2.2.4 | NR SA FR1 2-step non-contention based random access | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.3.2.3.1 | NR SA FR1 RRC connection release with redirection | NR Cell 6 | NR Cell 23 |  |  |  |  |  |  |  |
| 6.3.2.4.1 | PDCCH-order RACH on neighbor cell in FR1 when RACH BW is within active UL BWP | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.3.2.4.2 | PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.3.2.4.3 | PDCCH-order RACH on neighbor cell without L1-RSRP measurement in FR1 when RACH BW is within active UL BWP | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.3.3.1 | NR SA FR1 conditional handover | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.3.3.2 | NR SA FR1-FR1 conditional handover | NR Cell 1 | NR Cell 6 |  |  |  |  |  |  |  |
| 6.3.3.3 | SA FR1 conditional handover including target MCG and target SCG | NR Cell 1 | NR Cell 3 |  | NR Cell 2 |  | NR Cell 12 |  |  |  |
| 6.3.3.4 | NR-SA FR1 conditional handover including target MCG and candidate SCG | NR Cell 1 | NR Cell 3 |  | NR Cell 2 |  | NR Cell 12 |  |  |  |
| 6.3.3.6 | NR SA FR1 Intra-frequency NES triggering conditional handover | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.3.3.7 | NR SA FR1 Inter-frequency NES-based conditional handover | NR Cell 1 | NR Cell 6 |  |  |  |  |  |  |  |
| 6.3.4.1 | RACH-based Intra-frequency PCell switch from FR1 to FR1 | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.3.4.2 | RACH based Inter-frequency LTM PCell switch from FR1 to FR1 | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.3.4.3 | RACH-less Intra-frequency PCell switch from FR1 to FR1 | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.3.4.4 | NR SA FR1 RACH-less Intra-frequency PCell switch from FR1 to FR1 without L1-RSRP measurement | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.3.5.1 | NR SA FR1 RACH-based intra-frequency LTM PSCell switch from FR1 to FR1 | NR Cell 1 | NR Cell 6 |  | NR Cell 2 |  |  |  |  |  |
| 6.4.3.1 | NR SA FR1 timing advance adjustment accuracy | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.1.1 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM-RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.1.2 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM-RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.1.3 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM-RS in DRX mode | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.1.4 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM-RS in DRX mode | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.1.5 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM-RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.1.6 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM-RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.1.7 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM-RS in DRX mode | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.1.8 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM-RS in DRX mode | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.1.9 | SA FR1 radio link monitoring out-of-sync Test for PSCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.1.13 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM-RS in DRX mode for UE operating on a cell with less than 5MHz BW | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.1.14 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM-RS in non-DRX mode for UE operating on a cell with less than 5MHz BW | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.1.15 | NR SA FR1 radio link monitoring In-sync Test for PCell with 3MHz Channel Bandwidth configured with SSB-based RLM-RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.1.16 | NR SA FR1 Radio Link Monitoring In-sync Test for PCell with 3MHz Channel Bandwidth configured with SSB-based RLM-RS in DRX mode | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.2.1 | NR SA FR1 interruptions during measurements on deactivated NR SCC | NR Cell 1 | NR Cell 6 |  |  |  |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  |  |  |  | inter-band |  |
| 6.5.2.2 | SA FR1 interruptions at NR SRS carrier based switching | NR Cell 1 | NR Cell 6 |  |  |  |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  |  |  |  | inter-band |  |
| 6.5.3.1 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  |  |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  |  |  |  | inter-band |  |
| 6.5.3.2 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  |  |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  |  |  |  | inter-band |  |
| 6.5.3.3 | NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX | NR Cell 1 | NR Cell 6 |  |  |  |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  |  |  |  | inter-band |  |
| 6.5.3.4 | NR SA FR1 direct SCell activation at SCell addition of known SCell | NR Cell 1 | NR Cell 6 |  |  |  |  |  | intra-band |  |
| 6.5.3.5 | NR SA FR1 direct SCell activation at handover with known SCell | NR Cell 1 | NR Cell 6 |  | NR Cell 2 |  |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  |  |  |  | inter-band |  |
| 6.5.3.10 | NR SA FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  |  |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  |  |  |  | inter-band |  |
| 6.5.3.11 | NR SA FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  |  |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  |  |  |  | inter-band |  |
| 6.5.3.15 | NR SA FR1 TRS based SCell Activation of SSB-less SCell in inter-band CA in non-DRX | NR Cell 1 | NR Cell 10 |  |  |  |  |  | Inter-band |  |
| 6.5.3.16 | NR SA FR1 Inter-band SSB-less SCell Activation based on A-TRS in non-DRX | NR Cell 1 | NR Cell 10 |  |  |  |  |  | Inter-band |  |
| 6.5.4.1 | NR SA FR1 UE UL carrier RRC reconfiguration delay | NR Cell 1 | NR Cell 33 |  |  |  |  |  |  |  |
| 6.5.5.1 | NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.5.2 | NR SA FR1 SSB-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.5.3 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.5.4 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.5.5 | NR SA FR1 SCell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | NR Cell 1 | NR Cell 10 |  |  |  |  |  |  |  |
| 6.5.5.6 | NR SA FR1 SCell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | NR Cell 1 | NR Cell 10 |  |  |  |  |  |  |  |
| 6.5.5.7 | NR SA FR1 PCell TRP Specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.6.1.1 | NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX | NR Cell 1 | NR Cell 6 |  |  |  |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  |  |  |  | inter-band |  |
| 6.5.6.1.2 | NR SA FR1 DCI-based DL active BWP switch in non-DRX | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.6.2.1 | NR SA FR1 RRC-based DL active BWP switch in non-DRX | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.5.6.3.1 | NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA | NR Cell 1 | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |
| 6.5.6.5.1 | RRC based BWP switch on multiple CCs | NR Cell 1 | NR Cell 6 |  |  |  |  |  |  |  |
| 6.5.7.1 | NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA | NR Cell 1 | NR Cell 10 |  |  |  |  |  | inter-band |  |
| 6.5.7.2 | NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA | NR Cell 1 | NR Cell 10 |  |  |  |  |  | inter-band |  |
| 6.5.7A.1 | NR SA FR1 DL interruptions at switching between two uplink carriers in FDD-TDD CA | NR Cell 1 | NR Cell 10 |  |  |  |  |  | inter-band |  |
| 6.5.7A.2 | NR SA FR1 DL interruptions at switching between two uplink carriers in TDD-TDD CA | NR Cell 1 | NR Cell 10 |  |  |  |  |  | inter-band |  |
| 6.5.7B.1 | NR SA FR1 DL Interruptions at switching between two uplink bands in FDD-TDD CA | NR Cell 1 | NR Cell 10 |  | NR Cell 31 |  |  |  | inter-band |  |
| 6.5.7B.2 | NR SA FR1 DL Interruptions at switching between two uplink bands in TDD-TDD CA | NR Cell 1 | NR Cell 10 |  | NR Cell 31 |  |  |  | inter-band |  |
| 6.5.7C.1 | DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA | NR Cell 1 | NR Cell 10 |  | NR Cell 31 |  |  |  | Inter-band |  |
| 6.5.7C.2 | DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA | NR Cell 1 | NR Cell 10 |  | NR Cell 31 |  |  |  | Inter-band |  |
| 6.5.7D.1 | NR SA FR1 DL interruptions at switching across three uplink bands in TDD-TDD CA for single TAG | NR Cell 1 | NR Cell 10 |  | NR Cell 500 |  |  |  | Inter-band |  |
| 6.5.7D.2 | NR SA FR1 DL interruptions at switching across four uplink bands in FDD-TDD CA for single TAG | NR Cell 1 | NR Cell 10 |  | NR Cell 28 |  | NR Cell 500 |  | Inter-band |  |
| 6.5.7D.3 | NR SA FR1 DL interruptions at switching across three uplink bands in FDD-TDD CA for two TAGs | NR Cell 1 | NR Cell 10 |  | NR Cell 500 |  |  |  | Inter-band |  |
| 6.5.7D.4 | NR SA FR1 DL interruptions at switching across four uplink bands in TDD-TDD CA for two TAGs | NR Cell 1 | NR Cell 10 |  | NR Cell 28 |  | NR Cell 500 |  | Inter-band |  |
| 6.5.8.1 | UE specific CBW change on PCell in FR1 in non-DRX | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.6.1.1 | SA event triggered reporting tests without gap under non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.6.1.2 | SA event triggered reporting tests without gap under DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.6.1.3 | SA event triggered reporting tests with per-UE gaps under non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.6.1.4 | SA event triggered reporting tests with per-UE gaps under DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.6.1.5 | SA event triggered reporting tests without gap under non-DRX with SSB index reading | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.6.1.6 | SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.6.1.7 | NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.6.1.8 | NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-SCell-r17 | NR Cell 1 | NR Cell 6 |  | NR Cell 13 |  |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  | NR Cell 30 |  |  |  | inter-band |  |
| 6.6.1.12 | NR SA FR1 event triggered reporting with SSB index detection without measurement gap in non-DRX mode for UE operating with 12 PRB SSB bandwidth | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.6.1.9 | NR SA FR1 event triggered reporting with MUSIM gap configured | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.6.1.13 | NR SA FR1 event triggered reporting tests without gap in DRX under Cell DTX | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.6.2.1 | NR SA FR1-FR1 event-triggered reporting in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.6.2.2 | NR SA FR1-FR1 event-triggered reporting in DRX | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.6.2.5 | NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.6.2.6 | NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.6.2.9 | NR SA FR1-FR1 event triggered reporting tests with additional mandatory gap pattern | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.6.2.10 | NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is used | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.6.2.11 | NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is not used | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.6.2.12 | NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.6.2.13 | NR SA FR1-FR1 event triggered reporting using MUSIM gap with lower and higher priority | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.6.2.14 | NR SA FR1-FR1 event triggered reporting using MUSIM gap with shorter MGRP | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.6.2.15 | NR SA FR1 event triggered reporting without SSB index detection in DRX mode for UE operating on a cell with less than 5MHz BW | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.6.4.1 | NR SA FR1 SSB-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.6.4.2 | NR SA FR1 SSB-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.6.4.3 | NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.6.4.4 | NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.6.4.5 | NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.6.4.6 | NR SA FR1 Inter-cell SSB based L1-RSRP measurements on PCell in DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.6.6.1 | NR SA FR1 SRS-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.6.8.1 | NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.6.8.2 | NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.6.8.3 | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.7.8.1 | NR SA FR1 SRS-RSRP measurement accuracy | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.6.9.1 | NR SA FR1 Idle mode CA/DC measurement for FR1 | NR Cell 1 | NR Cell 6 |  |  |  |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  |  |  |  | inter-band |  |
| 6.6.9.2 | NR SA FR1 Idle mode fast CA/DC eEMR measurement | NR Cell 1 | NR Cell 6 |  |  |  |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  |  |  |  | inter-band |  |
| 6.6.9.3 | NR SA FR1 Idle mode fast CA/DC cell reselection measurement without valid reporting | NR Cell 1 | NR Cell 6 |  |  |  |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  |  |  |  | inter-band |  |
| 6.6.9.4 | NR SA FR1 Idle mode fast CA/DC cell reselection measurement with valid reporting | NR Cell 1 | NR Cell 6 |  |  |  |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  |  |  |  | inter-band |  |
| 6.6.17.1 | NR SA FR1 event triggered reporting tests with autonomous activation/deactivation Pre-MG | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.6.17.2 | NR SA FR1 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.6.18.1 | NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers | NR Cell 6 | NR Cell 3 |  | NR Cell 1 |  |  |  |  |  |
| 6.6.18.2 | NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers | NR Cell 6 | NR Cell 3 |  | NR Cell 1 |  |  |  |  |  |
| 6.6.18.4 | NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used | NR Cell 6 | NR Cell 3 |  | NR Cell 13 |  |  |  |  |  |
| 6.6.19.1 | NR SA FR1 event-triggered reporting tests with NCSG under non-DRX in FR1 | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.6.19.2 | NR SA FR1 event-triggered reporting tests for FR1 with NCSG for inter-frequency measurement | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.6.19.4 | NR SA FR1 Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX | NR Cell 1 | NR Cell 6 |  | NR Cell 13 |  |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  | NR Cell 30 |  |  |  | inter-band |  |
| 6.6.20.1 | UE Rx-Tx time difference measurement with PRS for RTT-based PDC | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.6.21.1 | UE Rx-Tx time difference measurement with TRS for RTT-based PDC | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.6.22.1 | NR SA FR1 and FR1-FR1 event-triggered reporting with concurrent Type-2 measurement gap and Pre-MG | NR Cell 6 | NR Cell 13 |  | NR Cell 3 |  |  |  |  |  |
| 6.6.22.2 | NR SA FR1 event-triggered reporting with two concurrent Pre-MGs | NR Cell 6 | NR Cell 13 |  | NR Cell 3 |  |  |  |  |  |
| 6.6.23.1 | NR SA FR1-FR1 event-triggered reporting with concurrent Type-2 measurement gap and NCSG | NR Cell 6 | NR Cell 3 |  | NR Cell 1 |  |  |  |  |  |
| 6.6.23.2 | NR SA FR1-FR1 event-triggered reporting with two concurrent NCSGs | NR Cell 6 | NR Cell 3 |  | NR Cell 1 |  |  |  |  |  |
| 6.6.23.3 | NR SA FR1 and FR1-FR1 event triggered reporting with concurrent Type-2 measurement gap and NCSG | NR Cell 6 |  | NR Cell 3 |  | NR Cell 12 |  |  |  | Intra-band |
|  |  | NR Cell 1 |  | NR Cell 10 |  | NR Cell 30 |  |  |  | Inter-band |
| 6.6.24.1 | NR SA FR1 event-triggered reporting without gap with interruptions | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.6.24.2 | NR SA FR1-FR1 event-triggered reporting without gap with interruption with SSB index reading | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.6.24.3 | NR SA FR1-FR1 event-triggered reporting without gap with interruption | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.6.24.4 | NR SA FR1-FR1 event-triggered reporting without gap without interruption | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.6.24.5 | NR SA FR1 event-triggered reporting without gap without interruption | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.6.26.1 | Intra-frequency SSB based L1-RSRP measurement | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.6.27.1 | Inter-frequency SSB based L1-RSRP measurement with measurement gap | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.6.28.1 | Inter-frequency SSB based L1-RSRP measurement without measurement gap | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.7.1.1.1 | NR SA FR1 SS-RSRP absolute measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |  |  |  |  |
| 6.7.1.1.2 | NR SA FR1 SS-RSRP relative measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |  |  |  |  |
| 6.7.1.2.1 | NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.7.1.2.2 | NR SA FR1-FR1 SS-RSRP relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.7.1.3.1 | NR SA FR1-FR2 SS-RSRP absolute measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.7.1.3.2 | NR SA FR1-FR2 SS-RSRP relative measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.7.2.1 | NR SA FR1 SS-RSRQ measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.7.2.2.1 | NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.7.2.2.2 | NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.7.3.1 | NR SA FR1 SS-SINR measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |  |  |  |
| 6.7.3.2.1 | NR SA FR1-FR1 SS-SINR absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.7.3.2.2 | NR SA FR1-FR1 SS-SINR relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.7.4.1.1 | NR SA FR1 SSB based L1-RSRP absolute measurement accuracy | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.7.4.1.2 | NR SA FR1 SSB based L1-RSRP relative measurement accuracy | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.7.4.2.1 | NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.7.4.2.2 | NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.7.9.1 | NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.7.9.2 | NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.7.9.3 | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |  |  |  |  |
| 6.7.10.1.1 | NR SA FR1 CSI-RSRP absolute measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |  |  |  |  |
| 6.7.10.1.2 | NR SA FR1 CSI-RSRP relative measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |  |  |  |  |
| 6.7.10.2.1 | NR SA FR1-FR1 CSI-RSRP absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.7.10.2.2 | NR SA FR1-FR1 CSI-RSRP relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.7.17.1.1 | Inter-frequency L1-RSRP absolute accuracy requirements for neighbour cell | NR Cell 6 | NR Cell 3 |  |  |  |  |  |  |  |
| 6.7.17.1.2 | Void |  |  |  |  |  |  |  |  |  |

Table E.4-2 defines the cell configuration mapping for NR/5GC FR1 – E-UTRA Inter-RAT test cases (serving cell in NR) in chapter 6 of this test specification.

Table E.4-2: Cell configuration mapping for SA FR1 – E-UTRA Inter-RAT RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 LTE Cell2 | 38.533 LTE Cell3 | 38.533 LTE Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6.1.2.1 | NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.1.2.2 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.1.2.3 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.1.2.4 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.1.2.5 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | LTE Cell 11 |  |  |  |
| 6.3.1.4 | NR SA FR1 – E-UTRA handover with known target cell | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.3.1.5 | NR SA FR1 – E-UTRA handover with unknown target cell | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.3.2.3.2 | NR SA FR1 – E-UTRA RRC connection release with redirection | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.3.1 | NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.3.2 | NR SA FR1 – E-UTRAN event-triggered reporting in DRX | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.3.3 | NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.15.1 | NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.19.3 | NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.25.1 | NR SA FR1 - E-UTRAN event-triggered reporting without gap without interruption | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.25.2 | NR SA FR1 - E-UTRAN event-triggered reporting without interruption outside gap | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.25.3 | NR SA FR1 - E-UTRAN event-triggered reporting without gap when CRS contained within active DL BWP | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.7.5.1 | NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.7.6.1 | NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.7.7.1 | NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | NR Cell 1 | LTE Cell 1 |  |  |  |

Table E.4-3: Cell configuration mapping for SA FR1 – UTRA FDD Inter-RAT RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 UTRA Cell2 | 38.533 UTRA Cell3 | 38.533 UTRA Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6.3.1.6 | NR SA FR1 – UTRA cell re-selection to higher priority UTRA | NR Cell 1 | UTRA Cell 1 |  |  |  |
| 6.6.5.1 | NR SA FR1 – UTRAN event-triggered reporting in non-DRX | NR Cell 1 | UTRA Cell 1 |  |  |  |

Table E.4-4 defines the multi-cell configuration mapping for NR/5GC FR1 – E-UTRA Inter-RAT test cases (serving cell in NR) in chapter 6 of this test specification.

Table E.4-4: Multi-cell configuration mapping for SA FR1 – E-UTRA Inter-RAT RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 LTE Cell3 | 38.533 LTE Cell 4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6.6.18.3 | NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1 | NR Cell 6 | NR Cell 3 | LTE Cell 1 |  |  |

# E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7

Table E.5-1 defines the cell configuration mapping for NR/5GC FR2 test cases in chapter 7 of this test specification.

Table E.5-1: Cell configuration mapping for SA FR2 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 7.1.1.1 | NR SA FR2 cell re-selection | NR Cell 1 | NR Cell 11 |  |  |  |
| 7.1.1.2 | NR SA FR2-FR2 cell re-selection | NR Cell 1 | NR Cell 23 |  |  |  |
| 7.1.1.3 | NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.4 | NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.5 | NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.6 | NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.7 | NR SA FR2 cell re-selection for power class 6 UE configured with highSpeedMeasFlagFR2-r17 | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.8 | NR SA FR2 inter-frequency cell re-selection for power class 6 UE configured with highSpeedMeasFlagFR2-r17 | NR Cell 1 | NR Cell 23 |  |  |  |
| 7.2.1.1 | TA validation for CG-SDT in FR2 | NR Cell 1 |  |  |  |  |
| 7.3.1.1 | NR SA FR1-FR2 Inter-frequency handover; unknown target cell | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.3.1.2 | NR SA FR2 Intra-frequency handover; unknown target cell | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.1.3 | NR SA FR2-FR2 Inter-frequency handover; unknown target cell | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.3.1.4 | NR SA FR1-FR2 synchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.3.1.5 | NR SA FR1-FR2 asynchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.3.2.1.1 | NR SA FR2 RRC re-establishment | NR Cell 1 |  |  |  |  |
| 7.3.2.1.2 | NR SA FR2-FR2 re-establishment | NR Cell 1 |  |  |  |  |
| 7.3.2.1.3 | NR SA FR2 RRC re-establishment without serving cell timing | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.2.2.1 | NR SA FR2 contention based random access | NR Cell 1 |  |  |  |  |
| 7.3.2.2.2 | NR SA FR2 non-contention based random access | NR Cell 1 |  |  |  |  |
| 7.3.2.2.4 | NR SA FR2 2-step non-contention based random access | NR Cell 1 |  |  |  |  |
| 7.3.2.3.1 | NR SA FR2-FR2 RRC connection release with redirection | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.3.2.4.1 | NR SA FR2 PDCCH-order RACH on neighbor cell when RACH BW is within active BWP | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.2.4.2 | NR SA FR2 PDCCH-order RACH on inter-frequency neighbor cell | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.3.3.1 | NR SA FR2 conditional handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.3.2 | NR SA FR2-FR2 conditional handover | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.3.4.1 | NR SA FR2 RACH based Intra-frequency PCell switch from FR2 to FR2 | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.4.2 | NR SA FR2 RACH-less Intra-frequency PCell switch from FR2 to FR2 | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.4.3 | NR SA FR2 RACH-based Inter-frequency LTM PCell switch from FR2 to FR2 | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.3.3.3 | NR SA FR2 intra-frequency NES triggering conditional handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.3.4 | NR SA FR2-FR1 inter-frequency NES triggering conditional handover | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.3.3.5 | NR SA FR2 NR conditional handover including target MCG and target SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC | NR Cell 1 | NR Cell 2 | NR Cell 3 | NR Cell 12 |  |
| 7.3.3.6 | NR SA FR2 conditional handover including target MCG and candidate SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC | NR Cell 1 | NR Cell 6 | NR Cell 3 | NR Cell 13 |  |
| 7.3.5.1 | NR SA FR2 RACH-based Intra-frequency LTM PSCell switch from FR2 to FR2 | NR Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 7.4.1.1 | NR SA FR2 UE transmit timing accuracy | NR Cell 1 |  |  |  |  |
| 7.4.1.3 | NR SA FR2 UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 | NR Cell 1 |  |  |  |  |
| 7.4.3.1 | NR SA FR2 timing advance adjustment accuracy | NR Cell 1 |  |  |  |  |
| 7.5.1.1 | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM-RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 7.5.1.2 | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM-RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 7.5.1.3 | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM-RS in DRX mode | NR Cell 1 |  |  |  |  |
| 7.5.1.4 | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM-RS in DRX mode | NR Cell 1 |  |  |  |  |
| 7.5.1.9 | NR SA FR2 radio link monitoring UE scheduling restrictions | NR Cell 1 |  |  |  |  |
| 7.5.3.3 | SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 7.5.3.4 | NR SA FR2 direct SCell activation at SCell addition of known SCell | NR Cell 1 | NR Cell 6 |  |  |  |  |
| 7.5.3.5 | NR SA FR2 direct SCell activation at handover with known SCell | NR Cell 1 | NR Cell 6 | NR Cell 2 |  |  |  |
| 7.5.3.13 | NR SA FR2 SCell Activation for SCell in FR2 intra-band in non-DRX | NR Cell 1 | NR Cell 6 |  |  | intra-band |
| 7.5.3.14 | NR SA FR2 SCell Activation for known SCell in FR2 inter-band | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.5.5.1 | NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |
| 7.5.5.2 | NR SA FR2 SSB-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |
| 7.5.5.3 | NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |
| 7.5.5.4 | NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |
| 7.5.5.6 | NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.5.5.7 | NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.5.5.8 | FFS |  |  |  |  |  |
| 7.5.5.9 | NR SA FR2 SCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.5.5.10 | NR SA FR2 PCell TRP specific SSB-based Beam Failure Detection and Link Recovery in non-DRX | NR Cell 1 |  |  |  |  |
| 7.5.6.1 | NR SA FR2 DCI-based DL active BWP switch in non-DRX | NR Cell 1 |  |  |  |  |
| 7.5.8.1.1 | NR SA PCell FR2 MAC-CE based active TCI state switch for a known TCI state | NR Cell 1 |  |  |  |  |  |
| 7.5.8.2.1 | NR SA PCell FR2 RRC based active TCI state switch for a known TCI state | NR Cell 1 |  |  |  |  |  |
| 7.5.8.3.1 | NR SA FR2 HST active TCI state switch for a known TCI state | NR Cell 1 |  |  |  |  |  |
| 7.5.8.3.2 | NR SA FR2 HST active TCI state switch for PC6 UE supporting tci-StateSwitchInd-r18 a known TCI state | NR Cell 1 |  |  |  |  |  |
| 7.5.8.4 | NR SA FR2 DCI based active TCI state switch with m-DCI for simultaneous reception | NR Cell 1 |  |  |  |  |  |
| 7.5.8.5 | NR SA FR2 Single-DCI based active TCI state switch with known target TCI states for simultaneous reception | NR Cell 1 |  |  |  |  |  |
| 7.5.9.1.1 | NR PCell FR2 uplink spatial relation switch associated with known DL-RS (MAC-CE based) | NR Cell 1 |  |  |  |  |  |
| 7.5.9.2.1 | NR PCell FR2 uplink spatial relation switch associated with known DL-RS (RRC based) | NR Cell 1 |  |  |  |  |  |
| 7.5.11.1 | NR SA FR2 UE UL carrier RRC reconfiguration delay | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.5.12.1 | Addition and Release Delay of PSCell | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.5.13.1 | NR SA FR2 MAC-CE based active joint TCI state switch | NR Cell 1 |  |  |  |  |
| 7.5.13.2 | NR SA FR2 MAC-CE based active uplink TCI state switch | NR Cell 1 |  |  |  |  |
| 7.5.13.3 | NR SA FR2 MAC-CE based active downlink TCI state switch | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.5.13.6.1 | NR SA FR2 MAC-CE based active uplink TCI state switch for 2 known TCI states | NR Cell 1 |  |  |  |  |
| 7.5.14 | NR SA FR2 PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2 | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.6.1.1 | NR SA FR2 event-triggered reporting without gap in non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.2 | NR SA FR2 event-triggered reporting without gap in DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.3 | NR SA FR2 event-triggered reporting with gap in non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.4 | NR SA FR2 event-triggered reporting with gap in DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.5 | NR SA FR2 event triggered reporting test without gap in non-DRX for UE configured with highSpeedMeasFlagFR2-r17 | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.2.1 | NR SA FR2-FR2 event-triggered reporting in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.2 | NR SA FR2-FR2 event-triggered reporting in DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.3 | NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.4 | NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.5 | NR SA FR1-FR2 event-triggered reporting in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.6 | NR SA FR1-FR2 event-triggered reporting in DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.7 | NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.8 | NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.10 | SA FR2-FR2 event triggered reporting test without gap under non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.11 | SA FR2-FR2 event triggered reporting test without gap under DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.20 | NR SA FR2-FR2 event triggered reporting using MUSIM gap with higher priority | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.21 | NR SA FR2-FR2 event triggered reporting using MUSIM gap with shorter MGRP | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.3.1 | NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |
| 7.6.3.2 | NR SA FR2 SSB-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |
| 7.6.3.3 | NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |
| 7.6.3.4 | NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |
| 7.6.3.5 | NR SA FR2 SSB based L1-RSRP measurement test when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17 | NR Cell 1 |  |  |  |  |
| 7.6.3.6 | NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used | NR Cell 10 | NR Cell 1 |  |  | Inter-band |
| 7.6.6.1 | NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 7.6.6.2 | NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 7.6.6.3 | NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 7.6.13.1 | UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2 | NR Cell 1 |  |  |  |  |  |
| 7.6.13.2 | NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using TRS | NR Cell 1 |  |  |  |  |  |
| 7.6.14.1 | NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and autonomous activation/deactivation | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 7.6.14.2 | NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 7.6.15.1 | NR SA FR2 event triggered reporting tests with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements | NR Cell 1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.15.2 | NR SA FR2 event triggered reporting tests with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2) | NR Cell 1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.15.3 | NR SA FR2 event triggered reporting tests with concurrent measurement gaps with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement | NR Cell 1 | NR Cell 6 | NR Cell 2 |  |  |  |
| 7.6.16.1 | NR SA FR2 event triggered reporting tests with per-UE NCSG under non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 7.6.16.2 | NR SA FR2 event triggered reporting tests on inter-frequency measurement with NCSG when DRX is not used(PCell in FR2) | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 7.6.16.3 | NR SA FR2 event triggered reporting test on deactivated SCell measurement via NCSG in non-DRX | NR Cell 1 | NR Cell 6 | NR Cell 13 |  | Intra-band |  |
| 7.6.17.1 | NR SA FR2 and FR2-FR2 event-triggered reporting with concurrent Type-2 measurement gap and Pre-MG | NR Cell 1 | NR Cell 2 | NR Cell 6 |  |  |  |
| 7.6.17.2 | NR SA FR2-FR2 event-triggered reporting with two concurrent Pre-MGs | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 7.6.18.1 | NR SA FR2 event-triggered reporting with concurrent Type-2 measurement gap and NCSG | NR Cell 1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.19.1 | NR SA FR2 event-triggered reporting without interruption outside gap | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 7.6.19.2 | NR SA FR2-FR2 SA event triggered reporting test without gap without interruption | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 7.6.19.3 | NR SA FR2-event triggered reporting test without gap without interruption under non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 7.6.20.1 | NR SA FR2 Intra-frequency SSB based L1-RSRP measurement | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 7.6.21.1 | NR SA FR2 Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM | NR Cell 1 | NR Cell 6 |  |  |  |  |
| 7.6.22.1 | NR SA FR2 Inter-frequency SSB based L1-RSRP measurement without measurement gap in FR2 | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 7.7.1.1 | NR SA FR2 SS-RSRP measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |
| 7.7.1.2 | NR SA FR2-FR2 SS-RSRP measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.7.1.3.1 | NR SA FR1-FR2 SS-RSRP absolute measurement accuracy | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.7.1.3.2 | Void |  |  |  |  |  |
| 7.7.2.1 | NR SA FR2 SS-RSRQ measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.7.2.2 | NR SA FR2-FR2 SS-RSRQ measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.7.3.1 | NR SA FR2 SS-SINR measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.7.3.2 | NR SA FR2-FR2 SS-SINR measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.7.6.1 | NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 7.7.6.2 | NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 7.7.6.3 | NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 7.7.7.1.1 | NR SA FR2 CSI-RSRP measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |
| 7.7.7.2.1 | NR SA FR2-FR2 CSI-RSRP measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.7.14.1 | NR SA FR2 SSB based L1-RSRP measurement for group-based beam reporting | NR Cell 1 |  |  |  |  |
| 7.7.14.2 | NR SA FR2 CSI-RS based L1-RSRP measurement on resource set with repetition off | NR Cell 1 |  |  |  |  |
| 7.7.15.1 | NR SA FR2 SSB based inter-frequency L1-RSRP measurement | NR Cell 1 | NR Cell 6 |  |  |  |

# E.6	Cell configuration mapping for E-UTRAN – SA test cases in Chapter 8

Table E.6-1 defines the cell configuration mapping for E-UTRAN - NR/5GC FR1 Inter-RAT test cases (serving cell in LTE) in chapter 8 of this test specification.

Table E.6-1: Cell configuration mapping for E-UTRA – SA FR1 Inter-RAT RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 8.2.1.1 | E-UTRA – NR FR1 cell re-selection to higher priority NR target cell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.2.1.2 | E-UTRA – NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16 | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.2.2.1 | E-UTRA – NR FR1 Early Measurement Reporting | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.3.1.1 | E-UTRA – NR FR1 handover with unknown target cell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.1.1 | E-UTRA – NR FR1 SFTD measurement delay in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.1.2 | E-UTRA – NR FR1 SFTD measurement delay in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.1 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.2 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.3 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.4 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.9 | E-UTRA – NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16 | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.3.2 | E-UTRAN - NR SA FR1 event-triggered reporting without gap | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.1.1 | E-UTRA – NR FR1 SFTD measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.1.1.1 | E-UTRA SS-RSRP absolute measurement accuracy of a NR FR1 neighbour cell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.2.1 | Void | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.3.1 | E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell | LTE Cell 1 | NR Cell 1 |  |  |  |

Table E.6-2 defines the cell configuration mapping for E-UTRAN - NR/5GC FR2 Inter-RAT test cases (serving cell in LTE) in chapter 8 of this test specification.

Table E.6-2: Cell configuration mapping for E-UTRA – SA FR2 Inter-RAT RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 8.2.2.2 | E-UTRA – NR FR2 Early Measurement Reporting | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.5 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.6 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.7 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.8 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.3.1 | E-UTRAN - NR SA FR2 event-triggered reporting without gap with interruption | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.1.2 | E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.2.2 | SS-RSRQ | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.3.2 | E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell | LTE Cell 1 | NR Cell 1 |  |  |  |

# E.7	Cell configuration mapping for NR sidelink test cases in Chapter 9

Table E.7-1 defines the cell configuration mapping for NR PC5 – NR SA FR1 Uu con-current test cases (serving cell in FR1) in chapter 9 of this test specification.

Table E.7-1: Cell configuration mapping for NR PC5 – NR SA FR1 Uu con-current RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 9.1.1.3 | NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source | NR Cell 1 |  |  |  |  |
| 9.1.2.1 | NR SA FR1 initiation/cease of S-SSB transmission for FR1 NR cell as synchronization reference source | NR Cell 1 |  |  |  |  |
| 9.1.5.1 | NR SA FR1 congestion control measurement for concurrent operation | NR Cell 1 |  |  |  |  |
| 9.1.6.1 | NR SA FR1 interruption to WAN due to NR sidelink communication | NR Cell 1 |  |  |  |  |

Table E.7-2 defines the cell configuration mapping for NR PC5 – NR SA FR2 Uu con-current test cases (serving cell in FR2) in chapter 9 of this test specification.

Table E.7-2: Cell configuration mapping for NR PC5 – SA FR2 con-current RRM testing

FFS

# E.8	Cell configuration mapping for EN-DC FR1 Shared Spectrum test cases in Chapter 10

Table E.8-1 defines the cell configuration mapping for EN-DC FR1 Shared Spectrum test cases in chapter 10 of this test specification.

Table E.8-1: Cell configuration mapping for EN-DC FR1 Shared Spectrum RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | 38.533 LTE Cell5 | CA Type |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10.1.1.1.1 | EN-DC FR1 4-step RA type contention-based random access for NR PSCell with CCA | LTE Cell 1 | NR Cell 1 |  |  |  |  |
| 10.1.1.1.2 | EN-DC FR1 4-step RA type non-contention based random access for NR PSCell with CCA | LTE Cell 1 | NR Cell 1 |  |  |  |  |
| 10.1.1.1.3 | EN-DC FR1 2-step RA type contention-based random access for NR PSCell with CCA | LTE Cell 1 | NR Cell 1 |  |  |  |  |
| 10.1.1.1.4 | EN-DC FR1 2-step RA type non-contention based random access for NR PSCell with CCA | LTE Cell 1 | NR Cell 1 |  |  |  |  |
| 10.1.2 | EN-DC FR1 Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  | LTE Cell 2 |  |
| 10.2.1.1 | EN-DC FR1 UE Transmit Timing Test with PSCell under DL CCA | LTE Cell 1 | NR Cell 1 |  |  |  |  |
| 10.2.2.1 | EN-DC FR1 UE Timing Advance Adjustment Accuracy with PSCell under DL CCA | LTE Cell 1 | NR Cell 1 |  |  |  |  |
| 10.3.1.2 | EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode under CCA | LTE Cell 1 | NR Cell 1 |  |  |  |  |
| 10.3.1.3 | EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode under CCA | LTE Cell 1 | NR Cell 1 |  |  |  |  |
| 10.3.3.1 | EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  |  | Intra-band |
|  |  |  |  | NR Cell 10 |  |  | Inter-band |
| 10.3.3.2 | EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  |  | Intra-band |
|  |  |  |  | NR Cell 10 |  |  | Inter-band |
| 10.3.3.3 | EN-DC FR1 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  |  | Intra-band |
|  |  |  |  | NR Cell 10 |  |  | Inter-band |
| 10.3.4.1 | EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |  |
| 10.3.4.2 | EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |  |
| 10.3.5.1 | EN-DC FR1 UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA | LTE Cell 1 | NR Cell 1 |  |  |  |  |
| 10.3.5.2.1 | EN-DC FR1 E-UTRAN – NR PSCell under CCA FR1 DL active BWP switch in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |  |
| 10.3.5.2.2 | EN-DC FR1 E-UTRAN – NR PSCell under CCA FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  |  | Intra-band |
|  |  |  |  | NR Cell 10 |  |  | Inter-band |
| 10.3.5.3.1 | EN-DC FR1 E-UTRAN – NR PSCell under CCA FR1 DL active BWP switch in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |  |
| 10.3.6.1 | EN-DC FR1 Addition and Release Delay of known NR PSCell on the carrier under CCA | LTE Cell 1 | NR Cell 1 |  |  |  |  |
| 10.4.1.1 | EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under non-DRX and CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 10.4.1.4 | EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under DRX and CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 10.4.2.3 | EN-DC FR1 event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 10.4.2.4 | EN-DC FR1 event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 10.4.2.5 | EN-DC FR1 event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 10.4.2.6 | EN-DC FR1 event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 10.4.2.7 | EN-DC FR1 event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 10.4.2.8 | EN-DC FR1 event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 10.4.2.9 | EN-DC FR1 event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 10.4.2.10 | EN-DC FR1 event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 10.4.3.1 | EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is not used | LTE Cell 1 | NR Cell 1 |  |  |  |  |
| 10.4.3.2 | EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is used | LTE Cell 1 | NR Cell 1 |  |  |  |  |
| 10.4.3.3 | EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used | LTE Cell 1 | NR Cell 1 |  |  |  |  |
| 10.4.3.4 | EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used | LTE Cell 1 | NR Cell 1 |  |  |  |  |
| 10.4.4.1 | EN-DC FR1 E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 10.4.4.2 | EN-DC FR1 E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 10.4.4.3 | EN-DC FR1 NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 10.4.4.4 | EN-DC FR1 NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 10.5.1.1 | EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 10.5.1.2 | EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 10.5.2.1 | EN-DC FR1 intra-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 10.5.2.2 | EN-DC FR1-FR1 inter-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 10.5.3.1 | EN-DC FR1 intra-frequency SS-SINR measurement accuracy on PSCC under CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 10.5.3.2 | EN-DC FR1-FR1 inter-frequency SS-SINR measurement accuracy on PSCC under CCA | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 10.5.3.3 | EN-DC FR1 intra-frequency SS-SINR measurement accuracy on SCC under CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 10.5.4.1 | EN-DC FR1 SSB based L1-RSRP measurement | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 10.5.5.1 | EN-DC FR1 RSSI measurement accuracy on PSCC with CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 10.5.5.2 | EN-DC FR1 RSSI measurement accuracy on SCC with CCA | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  |  | Intra-band |
|  |  |  |  | NR Cell 10 |  |  | Inter-band |
| 10.5.5.3 | EN-DC FR1 Inter-frequency RSSI measurement accuracy on a carrier with CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 10.5.6.1 | EN-DC FR1 Channel occupancy measurement accuracy on PSCC with CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 10.5.6.2 | EN-DC FR1 Channel occupancy measurement accuracy on SCC with CCA | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  |  | Intra-band |
|  |  |  |  | NR Cell 10 |  |  | Inter-band |
| 10.5.6.3 | EN-DC FR1 Inter-frequency channel occupancy measurement accuracy on a carrier with CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |  |
| NOTE 1: Intra-frequency cells are selected such that serving and neighbour cells satisfy (PCIneighbor-PCIserving) mod 3 ≠0. |  |  |  |  |  |  |  |

# E.9	Cell configuration mapping for SA FR1 Shared Spectrum test cases in Chapter 11

Table E.9-1 defines the cell configuration mapping for SA FR1 Shared Spectrum test cases in chapter 11 of this test specification.

Table E.9-1: Cell configuration mapping for SA FR1 Shared Spectrum RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 LTE Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 11.1.1.1 | NR SA FR1 Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell | NR Cell 1 | NR Cell 11 |  |  |  |
| 11.1.1.2 | NR SA FR1 Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell | NR Cell 1 | NR Cell 23 |  |  |  |
| 11.1.2.1 | NR SA FR1 Cell re-selection to NR with source NR carrier frequency under CCA | NR Cell 1 | NR Cell 23 |  |  |  |
| 11.1.3.1 | NR SA FR1 Cell re-selection from NR carrier with target NR carrier frequency under CCA | NR Cell 1 | NR Cell 23 |  |  |  |
| 11.1.4.1 | NR SA FR1 Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA | NR Cell 1 |  |  | LTE Cell 1 |  |
| 11.1.4.2 | NR SA FR1 Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA | NR Cell 1 |  |  | LTE Cell 1 |  |
| 11.2.1.1 | NR SA FR1 Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell | NR Cell 1 | NR Cell 2 |  |  |  |
| 11.2.1.2 | NR SA FR1 Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell | NR Cell 1 | NR Cell 2 |  |  |  |
| 11.2.1.3 | NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.2.1.4 | NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.2.1.5 | NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.2.1.6 | NR SA FR1 Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.2.1.7 | NR SA FR1 carrier under CCA - E-UTRAN handover with known target cell | NR Cell 1 |  |  | LTE Cell 1 |  |
| 11.2.1.8 | NR SA FR1 carrier under CCA - E-UTRAN handover with unknown target cell | NR Cell 1 |  |  | LTE Cell 1 |  |
| 11.2.2.1.1 | NR SA FR1 Intra-frequency RRC Re-establishment with CCA in FR1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 11.2.2.1.2 | NR SA FR1 Inter-frequency RRC Re-establishment with CCA in FR1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.2.2.1.3 | NR SA FR1 Intra-frequency RRC Re-establishment with CCA without serving cell timing | NR Cell 1 | NR Cell 2 |  |  |  |
| 11.2.2.1.4 | NR SA FR1 Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.2.2.2.1 | NR SA FR1 4-step RA type contention-based random access for NR PCell with CCA | NR Cell 1 |  |  |  |  |
| 11.2.2.2.2 | NR SA FR1 4-step RA type non-contention based random access for NR PSCell with CCA | NR Cell 1 |  |  |  |  |
| 11.2.2.2.3 | NR SA FR1 2-step RA type contention-based random access for NR PCell with CCA | NR Cell 1 |  |  |  |  |
| 11.2.2.2.4 | NR SA FR1 2-step RA type non-contention-based random access for NR PCell with CCA | NR Cell 1 |  |  |  |  |
| 11.2.2.3.1 | NR SA FR1 Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.2.2.3.2 | NR SA FR1 Redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.3.1.1 | NR SA FR1 UE Transmit Timing Test with PCell under DL CCA | NR Cell 1 |  |  |  |  |
| 11.3.2.1 | NR SA FR1 UE Timing Advance Adjustment Accuracy with PCell under DL CCA | NR Cell 1 |  |  |  |  |
| 11.4.1.2 | NA SA FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode under CCA | NR Cell 1 |  |  |  |  |
| 11.4.1.3 | NR SA FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode under CCA | NR Cell 1 |  |  |  |  |
| 11.4.3.1 | NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  | Intra-band |
|  |  |  | NR Cell 10 |  |  | Inter-band |
| 11.4.3.2 | NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  | Intra-band |
|  |  |  | NR Cell 10 |  |  | Inter-band |
| 11.4.3.3 | NR SA FR1 SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA | NR Cell 1 | NR Cell 6 |  |  | Intra-band |
|  |  |  | NR Cell 10 |  |  | Inter-band |
| 11.4.4.1 | NR SA FR1 E-UTRAN – NR interruptions during SCell operations with CCA | NR Cell 1 |  |  |  |  |
| 11.4.4.2 | NR SA FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle | NR Cell 1 |  |  |  |  |
| 11.4.5.1 | NR SA FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle | NR Cell 1 |  |  |  |  |
| 11.4.5.2.1 | NR SA FR1 DCI-based DL active BWP switch of PCell with non-DRX under CCA | NR Cell 1 | NR Cell 6 |  |  | Intra-band |
|  |  |  | NR Cell 10 |  |  | Inter-band |
| 11.4.5.2.2 | NR SA FR1 DCI-based DL active BWP switch with non-DRX under CCA | NR Cell 1 |  |  |  |  |
| 11.4.5.3.1 | NR SA FR1 DL active BWP switch of Cell with non-DRX | NR Cell 1 |  |  |  |  |
| 11.5.1.1 | NR SA FR1 Event-triggered reporting tests on PCC under CCA without gaps under non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 11.5.1.2 | NR SA FR1 Event-triggered reporting tests on PCC under CCA without gaps under DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 11.5.2.3 | NR SA FR1 Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.5.2.4 | NR SA FR1 Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.5.2.5 | NR SA FR1 Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.5.2.6 | NR SA FR1 Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.5.2.7 | NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.5.2.8 | NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.5.2.9 | NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.5.2.10 | NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.5.3.1 | NR SA FR1 - E-UTRAN event-triggered reporting in non-DRX | NR Cell 1 |  |  | LTE Cell 1 |  |
| 11.5.3.2 | NR SA FR1 - E-UTRAN event-triggered reporting in DRX | NR Cell 1 |  |  | LTE Cell 1 |  |
| 11.5.4.1 | NR SA FR1 SSB based L1-RSRP measurement when DRX is not used | NR Cell 1 |  |  |  |  |
| 11.5.4.2 | NR SA FR1 SSB based L1-RSRP measurement when DRX is used | NR Cell 1 |  |  |  |  |
| 11.5.4.3 | NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used | NR Cell 1 |  |  |  |  |
| 11.5.4.4 | NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used | NR Cell 1 |  |  |  |  |
| 11.6.1.1 | NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA | NR Cell 489 | NR Cell 1 |  |  |  |
| 11.6.1.2 | NR SA FR1 Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA | NR Cell 489 | NR Cell 6 | NR Cell 3 |  | Intra-band |
|  |  |  | NR Cell 10 |  |  | Inter-band |
| 11.6.2.1 | NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA | NR Cell 1 | NR Cell 2 |  |  |  |
| 11.6.2.2 | NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.6.2.3 | NR SA FR1 Intra-frequency measurement accuracy on SCC | NR Cell 1 | NR Cell 2 |  |  |  |
| 11.6.2.4 | NR SA FR1 Inter-frequency measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.6.3.1 | NR SA FR1 Intra-frequency SS-SINR measurement accuracy on a carrier frequency with CCA | NR Cell 1 | NR Cell 2 |  |  |  |
| 11.6.3.2 | NR SA FR1 Inter-frequency SS-SINR measurement accuracy on a carrier frequency with CCA | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.6.3.3 | NR SA FR1 Intra-frequency SS-SINR measurement accuracy on SCC on a carrier frequency with CCA | NR Cell 1 | NR Cell 6 |  |  | Intra-band |
|  |  |  | NR Cell 10 |  |  | Inter-band |
| 11.6.3.4 | NR SA FR1 Inter-frequency measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 11.6.4.1 | NR SA FR1 SSB based L1-RSRP measurement accuracy with CCA | NR Cell 1 |  |  |  |  |
| 11.6.5.1 | NR SA FR1 Intra-frequency RSSI measurement accuracy on PCC with CCA | NR Cell 1 | NR Cell 2 |  |  |  |
| 11.6.5.2 | NR SA FR1 Intra-frequency RSSI measurement accuracy on SCC with CCA | NR Cell 1 | NR Cell 6 |  |  | Intra-band |
|  |  |  | NR Cell 10 |  |  | Inter-band |
| 11.6.5.3 | NR SA FR1 Inter-frequency RSSI measurement accuracy on a carrier with CCA | NR Cell 1 | NR Cell 2 |  |  |  |
| 11.6.6.1 | NR SA FR1 Intra-frequency channel occupancy measurement accuracy on PCC with CCA | NR Cell 1 | NR Cell 2 |  |  |  |
| 11.6.6.2 | NR SA FR1 Intra-frequency channel occupancy measurement accuracy on SCC with CCA | NR Cell 1 | NR Cell 6 |  |  | Intra-band |
|  |  |  | NR Cell 10 |  |  | Inter-band |
| 11.6.6.3 | NR SA FR1 Inter-frequency channel occupancy measurement accuracy on a carrier with CCA | NR Cell 1 | NR Cell 2 |  |  |  |
| NOTE 1: Intra-frequency cells are selected such that serving and neighbour cells satisfy (PCIneighbor-PCIserving) mod 3 ≠0. |  |  |  |  |  |  |

# E.10	Cell configuration mapping for E-UTRAN – SA FR1 Shared Spectrum test cases in Chapter 12

Table E.10-1 defines the cell configuration mapping for E-UTRAN – SA FR1 Shared Spectrum test cases in Chapter 12 of this test specification.

Table E.10-1: Cell configuration mapping for E-UTRAN – SA FR1 Shared Spectrum RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 12.1.1.1 | E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA | LTE Cell 1 | NR Cell 1 |  |  |  |
| 12.2.1.1 | E-UTRAN - NR with CCA handover with known target cell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 12.4.1.1 | E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 12.4.2.1 | E-UTRAN-NR FR1 inter-RAT event triggered reporting without SSB time index detection when DRX is not used under CCA | LTE Cell 1 | NR Cell 1 |  |  |  |
| 12.4.2.2 | E-UTRAN-NR FR1 inter-RAT event triggered reporting without SSB time index detection when DRX is used under CCA | LTE Cell 1 | NR Cell 1 |  |  |  |
| 12.4.2.3 | E-UTRAN-NR FR1 inter-RAT event triggered reporting with SSB time index detection when DRX is not used under CCA | LTE Cell 1 | NR Cell 1 |  |  |  |
| 12.4.2.4 | E-UTRAN-NR FR1 inter-RAT event triggered reporting with SSB time index detection when DRX is used under CCA | LTE Cell 1 | NR Cell 1 |  |  |  |
| 12.5.1.1 | E-UTRA - NR FR1 SFTD measurement accuracy under CCA | LTE Cell 1 | NR Cell 1 |  |  |  |
| NOTE 1: Intra-frequency cells are selected such that serving and neighbour cells satisfy (PCIneighbor-PCIserving) mod 3 ≠0. |  |  |  |  |  |  |

# E.11	Cell configuration mapping for SA FR1 with SCell in Shared Spectrum test cases in Chapter 13

Table E.11-1 defines the cell configuration mapping for SA FR1 with SCell in Shared Spectrum test cases in Chapter 13 of this test specification.

Table E.11-1: Cell configuration mapping for SA FR1 with SCell in Shared Spectrum RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 13.2.2.1 | NR SA FR1 SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle | NR Cell 1 | NR Cell 10 |  |  | Intra-band |
|  |  |  | NR Cell 6 |  |  | Inter-band |
| 13.2.2.2 | NR SA FR1 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle | NR Cell 1 | NR Cell 10 |  |  | Intra-band |
|  |  |  | NR Cell 6 |  |  | Inter-band |
| 13.2.2.3 | NR SA FR1 SCell Activation and Deactivation of unknown SCell under CCA | NR Cell 1 | NR Cell 10 |  |  | Intra-band |
|  |  |  | NR Cell 6 |  |  | Inter-band |
| 13.3.1.1 | NR SA FR1 Event-triggered reporting tests on SCC under CCA without gaps under non-DRX | NR Cell 1 | NR Cell 10 | NR Cell 2 |  | Intra-band |
|  |  |  | NR Cell 6 |  |  | Inter-band |
| 13.3.1.2 | NR SA FR1 Event-triggered reporting tests on SCC under CCA without gaps under DRX | NR Cell 1 | NR Cell 10 | NR Cell 2 |  | Intra-band |
|  |  |  | NR Cell 6 |  |  | Inter-band |
| 13.3.1.3 | NR SA FR1 Event-triggered reporting tests on SCC under CCA with per-UE gaps under non-DRX | NR Cell 1 | NR Cell 10 | NR Cell 2 |  | Intra-band |
|  |  |  | NR Cell 6 |  |  | Inter-band |
| 13.3.1.4 | NR SA FR1 Event-triggered reporting tests on SCC under CCA with per-UE gaps under DRX | NR Cell 1 | NR Cell 10 | NR Cell 2 |  | Intra-band |
|  |  |  | NR Cell 6 |  |  | Inter-band |
| 13.3.2.3 | NR SA FR1-FR1 Event triggered reporting tests on a carrier with CCA without SSB time index detection when DRX is not used | NR Cell 1 | NR Cell 10 | NR Cell 2 |  | Intra-band |
|  |  |  | NR Cell 6 |  |  | Inter-band |
| 13.3.2.4 | NR SA FR1-FR1 Event triggered reporting tests on a carrier with CCA without SSB time index detection when DRX is used | NR Cell 1 | NR Cell 10 | NR Cell 2 |  | Intra-band |
|  |  |  | NR Cell 6 |  |  | Inter-band |
| 13.3.2.5 | NR SA FR1-FR1 Event triggered reporting tests on a carrier with CCA with SSB time index detection when DRX is not used | NR Cell 1 | NR Cell 10 | NR Cell 2 |  | Intra-band |
|  |  |  | NR Cell 6 |  |  | Inter-band |
| 13.3.2.6 | NR SA FR1-FR1 Event triggered reporting tests on a carrier with CCA with SSB time index detection when DRX is used | NR Cell 1 | NR Cell 10 | NR Cell 2 |  | Intra-band |
|  |  |  | NR Cell 6 |  |  | Inter-band |
| 13.3.3.1 | NR SA FR1 SSB based L1-RSRP measurement for SCell under CCA when DRX is not used | NR Cell 1 | NR Cell 10 |  |  | Intra-band |
|  |  |  | NR Cell 6 |  |  | Inter-band |
| 13.3.3.2 | NR SA FR1 SSB based L1-RSRP measurement for SCell under CCA when DRX is used | NR Cell 1 | NR Cell 10 |  |  | Intra-band |
|  |  |  | NR Cell 6 |  |  | Inter-band |
| 13.4.1.1 | NR SA FR1 SS-RSRP measurement accuracy on a carrier with CCA | NR Cell 1 | NR Cell 489 | NR Cell 6 |  |  |
| 13.4.2.1 | NR SA FR1 SS-RSRQ measurement accuracy on a carrier with CCA | NR Cell 1 | NR Cell 489 | NR Cell 6 |  |  |
| 13.4.3.1 | NR SA FR1 SS-SINR measurement accuracy on a carrier with CCA | NR Cell 1 | NR Cell 489 | NR Cell 6 |  |  |
| 13.4.4.1 | NR SA FR1 L1-RSRP measurement accuracy for beam reporting on a carrier with CCA | NR Cell 1 | NR Cell 6 |  |  |  |
| 13.4.5.1 | NR SA FR1 RSSI measurement accuracy on a carrier with CCA | NR Cell 1 | NR Cell 6 |  |  |  |
| 13.4.5.2 | NR SA FR1-FR1 RSSI measurement accuracy on a carrier with CCA | NR Cell 1 | NR Cell 6 |  |  |  |
| 13.4.6.1 | NR SA FR1 channel occupancy measurement accuracy on a carrier with CCA | NR Cell 1 | NR Cell 6 |  |  |  |
| 13.4.6.2 | NR SA FR1-FR1 channel occupancy measurement accuracy on a carrier with CCA | NR Cell 1 | NR Cell 6 |  |  |  |
| NOTE 1: Intra-frequency cells are selected such that serving and neighbour cells satisfy (PCIneighbor-PCIserving) mod 3 ≠0. |  |  |  |  |  |  |

# E.12	Cell configuration mapping for SA Satellite Access test cases in Chapter 14

Table E.12-1 defines the cell configuration mapping for SA Satellite Access test cases in Chapter 14 of this test specification.

Table E.12-1: Cell configuration mapping for SA Satellite Access RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 14.1.1 | NR SA FR1 Cell reselection for Satellite Access | NR Cell 1 | NR Cell 11 |  |  |  |
| 14.1.2 | NR SA FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access | NR Cell 1 | NR Cell 11 |  |  |  |
| 14.1.3 | NR SA FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access | NR Cell 1 | NR Cell 11 |  |  |  |
| 14.1.4 | NR SA FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access | NR Cell 1 | NR Cell 11 |  |  |  |
| 14.1.5 | NR SA FR1-FR1 Cell reselection for Satellite Access | NR Cell 1 | NR Cell 23 |  |  |  |
| 14.1.6 | NR SA FR1-FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access | NR Cell 1 | NR Cell 23 |  |  |  |
| 14.1.7 | NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access | NR Cell 1 | NR Cell 23 |  |  |  |
| 14.1.8 | NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access | NR Cell 1 | NR Cell 23 |  |  |  |
| 14.1.9 | NR SA FR1-FR1 Cell reselection for UE fulfilling low mobility relaxed measurement criterion for Satellite Access | NR Cell 1 | NR Cell 23 |  |  |  |
| 14.1.10 | NR SA FR1-FR1 Cell reselection for UE fulfilling not-at-cell edge relaxed measurement criterion for Satellite Access | NR Cell 1 | NR Cell 23 |  |  |  |
| 14.2.1.1 | NR SA FR1 Handover for Satellite Access | NR Cell 1 | NR Cell 2 |  |  |  |
| 14.2.1.2 | NR SA FR1-FR1 Handover for Satellite Access | NR Cell 6 | NR Cell 3 |  |  |  |
| 14.2.1.3 | NR SA FR1 Time-based Conditional Handover for NR Satellite Access | NR Cell 1 | NR Cell 2 |  |  |  |
| 14.2.1.4 | SA FR1-FR1 SAN time-based conditional Handover for NR satellite access | NR Cell 6 | NR Cell 3 |  |  |  |
| 14.2.1.5 | SA FR1 SAN distance-based conditional Handover for NR satellite access | NR Cell 1 | NR Cell 2 |  |  |  |
| 14.2.1.6 | SA FR1-FR1 SAN distance-based conditional Handover for NR satellite access | NR Cell 6 | NR Cell 3 |  |  |  |
| 14.2.1.8 | NR SA FR1-FR1 RACH-less Handover for Satellite Access | NR Cell 6 | NR Cell 3 |  |  |  |
| 14.2.2.1.1 | NR SA FR1 RRC Re-establishment for Satellite Access | NR Cell 1 | NR Cell 2 |  |  |  |
| 14.2.2.1.2 | NR SA FR1-FR1 RRC Re-establishment for Satellite Access | NR Cell 6 | NR Cell 3 |  |  |  |
| 14.2.2.2.1 | NR SA FR1 4-step contention based random access for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.2.2.2.2 | NR SA FR1 4-step non-contention based random access for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.2.2.3.1 | NR SA FR1-FR1 RRC connection release with redirection for Satellite Access | NR Cell 6 | NR Cell 23 |  |  |  |
| 14.2.2.4 | NR SA FR1 RACH-based Hard Satellite switching with re-synchronization for Satellite Access | NR Cell 1 | NR Cell 1 |  |  |  |
| 14.2.2.5 | NR SA FR1 RACH-less Soft Satellite switching with re-synchronization for Satellite Access | NR Cell 1 | NR Cell 1 |  |  |  |
| 14.3.1.1 | NR SA FR1 UE transmit timing accuracy for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.3.2.1 | NR SA FR1 timing advance adjustment accuracy for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.4.1.1 | NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with SSB-based RLM-RS in non-DRX mode for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.4.1.2 | NR SA FR1 Radio Link Monitoring In-sync for PCell configured with SSB-based RLM-RS in non-DRX mode for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.4.1.3 | NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with SSB-based RLM-RS in DRX mode for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.4.1.4 | NR SA FR1 Radio Link Monitoring In-sync for PCell configured with SSB-based RLM-RS in DRX mode for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.4.1.5 | NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with CSI-RS-based RLM-RS in non-DRX mode for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.4.1.6 | NR SA FR1 Radio Link Monitoring In-sync for PCell configured with CSI-RS-based RLM-RS in non-DRX mode for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.4.1.7 | NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with CSI-RS-based RLM-RS in DRX mode for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.4.1.8 | NR SA FR1 Radio Link Monitoring In-sync for PCell configured with CSI-RS-based RLM-RS in DRX mode for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.4.2.1 | NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with SSB-based BFD and LR in non-DRX mode for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.4.2.2 | NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with SSB-based BFD and LR in DRX mode for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.4.2.3 | NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in non-DRX mode for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.4.2.4 | NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in DRX mode for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.4.3.1.1 | NR SA FR1 DCI-based DL active BWP switch with non-DRX for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.4.3.2.1 | NR SA FR1 RRC-based DL active BWP switch with non-DRX for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.4.4.1 | NR SA FR1 UE specific CBW change on PCell in non-DRX for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.4.5.1 | NR SA FR1 MAC-CE based pathloss reference signal switch delay for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.5.1.1 | SA FR1 event triggered reporting tests without gap under non-DRX for NR satellite access | NR Cell 1 | NR Cell 2 |  |  |  |
| 14.5.1.2 | SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access | NR Cell 1 | NR Cell 2 |  |  |  |
| 14.5.1.3 | SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access | NR Cell 1 | NR Cell 2 |  |  |  |
| 14.5.1.4 | SA FR1 event triggered reporting tests with single measurement gap under non-DRX for satellite access for NR satellite access | NR Cell 1 | NR Cell 2 |  |  |  |
| 14.5.1.5 | SA FR1 event triggered reporting tests with FNO concurrent gaps under DRX for satellite access for NR satellite access | NR Cell 1 | NR Cell 2 |  |  |  |
| 14.5.1.6 | SA FR1 event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access for NR satellite access | NR Cell 1 | NR Cell 2 |  |  |  |
| 14.5.2.1 | NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access | NR Cell 6 | NR Cell 3 |  |  |  |
| 14.5.2.2 | NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access | NR Cell 6 | NR Cell 3 |  |  |  |
| 14.5.2.3 | NR SA FR1-FR1 Event-triggered reporting with SSB time index detection when DRX is not used with single gap for satellite access | NR Cell 6 | NR Cell 3 |  |  |  |
| 14.5.2.4 | NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access | NR Cell 6 | NR Cell 3 |  |  |  |
| 14.5.2.6 | NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access | NR Cell 6 | NR Cell 3 |  |  |  |
| 14.5.2.7 | NR SA FR1-FR1 Event triggered reporting without gap under non-DRX for Satellite Access | NR Cell 6 | NR Cell 3 |  |  |  |
| 14.5.2.8 | NR SA FR1-FR1 Event triggered reporting without gap under DRX for Satellite Access | NR Cell 6 | NR Cell 3 |  |  |  |
| 14.5.3.1 | NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.5.3.2 | NR SA FR1 SSB based L1-RSRP measurement when DRX is used for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.5.3.3 | NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.5.3.4 | NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.6.1.1 | NR SA FR1 SS-RSRP measurement accuracy for Satellite Access | NR Cell 1 | NR Cell 2 |  |  |  |
| 14.6.1.2 | NR SA FR1-FR1 SS-RSRP measurement accuracy for Satellite Access | NR Cell 6 | NR Cell 3 |  |  |  |
| 14.6.2.1 | NR SA FR1 SS-RSRQ measurement accuracy for Satellite Access | NR Cell 1 | NR Cell 2 |  |  |  |
| 14.6.2.2 | NR SA FR1-FR1 SS-RSRQ measurement accuracy for Satellite Access | NR Cell 6 | NR Cell 3 |  |  |  |
| 14.6.3.1 | NR SA FR1 SS-SINR Measurement Accuracy for Satellite Access | NR Cell 1 | NR Cell 2 |  |  |  |
| 14.6.3.2 | NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access | NR Cell 6 | NR Cell 3 |  |  |  |
| 14.6.4.1 | NR SA FR1 SSB based L1-RSRP Measurement Accuracy for Satellite Access | NR Cell 1 |  |  |  |  |
| 14.6.4.2 | NR SA FR1 CSI-RS based L1-RSRP Measurement Accuracy for Satellite Access | NR Cell 1 |  |  |  |  |
| NOTE 1: Intra-frequency cells are selected such that serving and neighbour cells satisfy (PCIneighbor-PCIserving) mod 3 ≠0. |  |  |  |  |  |  |

# E.13

# E.14	Cell configuration mapping for SA FR1 test cases for RedCap in Chapter 16

Table E.14-1 defines the cell configuration mapping for NR/5GC FR1 test cases in chapter 16 of this test specification.

Table E.14-1: Cell configuration mapping for SA FR1 RRM testing for RedCap

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 |
| --- | --- | --- | --- | --- | --- |
| 16.1.1.1 | NR SA FR1 Cell reselection for 1 Rx UE | NR Cell 1 | NR Cell 11 |  |  |
| 16.1.1.2 | NR SA FR1 Cell reselection for 2 Rx UE | NR Cell 1 | NR Cell 11 |  |  |
| 16.1.1.3 | NR SA FR1-FR1 Cell reselection for 1 Rx UE | NR Cell 6 | NR Cell 23 |  |  |
| 16.1.1.4 | NR SA FR1-FR1 Cell reselection for 2 Rx UE | NR Cell 6 | NR Cell 23 |  |  |
| 16.1.1.5 | NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE | NR Cell 1 | NR Cell 11 |  |  |
| 16.1.1.6 | NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | NR Cell 1 | NR Cell 11 |  |  |
| 16.1.1.7 | NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE | NR Cell 6 | NR Cell 23 |  |  |
| 16.1.1.8 | NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | NR Cell 6 | NR Cell 23 |  |  |
| 16.2.1.1 | NR SA FR1 CG-SDT for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.2.1.2 | NR SA FR1 CG-SDT for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.3.1.1 | NR SA FR1 handover with known target cell for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.3.1.2 | NR SA FR1 handover with known target cell for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.3.1.3 | NR SA FR1 handover with unknown target cell for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.3.1.4 | NR SA FR1 handover with unknown target cell for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.3.1.5 | NR SA FR1-FR1 handover with unknown target cell for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.3.1.6 | NR SA FR1-FR1 handover with unknown target cell for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.3.2.1.1 | NR SA FR1 RRC Re-establishment for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.3.2.1.2 | NR SA FR1 RRC Re-establishment for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.3.2.1.3 | NR SA FR1-FR1 RRC Re-establishment for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.3.2.1.4 | NR SA FR1-FR1 RRC Re-establishment for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.3.2.1.5 | NR SA FR1 RRC Re-establishment for 1 Rx UE without serving cell timing | NR Cell 1 | NR Cell 2 |  |  |
| 16.3.2.1.6 | NR SA FR1 RRC Re-establishment for 2 Rx UE without serving cell timing | NR Cell 1 | NR Cell 2 |  |  |
| 16.3.2.2.1 | NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE | Cell 1 |  |  |  |
| 16.3.2.2.2 | NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE | Cell 1 |  |  |  |
| 16.3.2.2.3 | NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE | Cell 1 |  |  |  |
| 16.3.2.2.4 | NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE | Cell 1 |  |  |  |
| 16.3.2.2.5 | NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE | Cell 1 |  |  |  |
| 16.3.2.2.6 | NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE | Cell 1 |  |  |  |
| 16.3.2.2.7 | NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE | Cell 1 |  |  |  |
| 16.3.2.2.8 | NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE | Cell 1 |  |  |  |
| 16.3.2.3.1 | NR SA FR1-FR1 RRC connection release with redirection for 1 Rx UE | NR Cell 6 | NR Cell 23 |  |  |
| 16.3.2.3.2 | NR SA FR1-FR1 RRC connection release with redirection for 2 Rx UE | NR Cell 6 | NR Cell 23 |  |  |
| 16.4.1.1 | NR SA FR1 NR UE Transmit Timing accuracy for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.4.1.2 | NR SA FR1 NR UE Transmit Timing accuracy for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.4.3.1 | NR SA FR1 timing advance adjustment accuracy for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.4.3.2 | NR SA FR1 timing advance adjustment accuracy for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.1 | NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM-RS in non-DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.2 | NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM-RS in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.3 | NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM-RS in non-DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.4 | NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM-RS in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.5 | NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM-RS in DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.6 | NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM-RS in DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.7 | NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM-RS in DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.8 | NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM-RS in DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.9 | NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.10 | NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.11 | NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.12 | NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.13 | NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.14 | NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.15 | NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.1.16 | NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.2.1 | NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.2.2 | NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.2.3 | NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.2.4 | NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.2.5 | NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.2.6 | NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.2.7 | NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.2.8 | NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.3.1.1 | NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.3.1.2 | NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.3.2.1 | NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.3.2.2 | NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.5.4.1 | NR SA FR1 UE specific CBW change on PCell in non-DRX for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.5.4.2 | NR SA FR1 UE specific CBW change on PCell in non-DRX for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.6.1.1 | NR SA FR1 Event triggered reporting tests without gap under non-DRX for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.2 | NR SA FR1 Event triggered reporting tests without gap under non-DRX for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.3 | NR SA FR1 Event triggered reporting tests without gap under DRX for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.4 | NR SA FR1 Event triggered reporting tests without gap under DRX for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.5 | NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.6 | NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.7 | NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.8 | NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.9 | NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.10 | NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.11 | NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.1.12 | NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.2.1 | NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.2 | NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.3 | NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.4 | NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.5 | NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.6 | NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.7 | NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.8 | NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.9 | NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.10 | NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.11 | NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.2.12 | NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.6.4.1 | NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.6.4.2 | NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.6.4.3 | NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.6.4.4 | NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.6.4.5 | NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.6.4.6 | NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.6.4.7 | NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.6.4.8 | NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.6.5.1 | NR SA FR1 CGI identification of NR neighbour cell for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.6.5.2 | NR SA FR1 CGI identification of NR neighbour cell for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.7.1.1.1 | NR SA FR1 SS-RSRP absolute measurement accuracy for 1 Rx UE | NR Cell 489 | NR Cell 1 |  |  |
| 16.7.1.1.2 | NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE | NR Cell 489 | NR Cell 1 |  |  |
| 16.7.1.2.1 | NR SA FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE | NR Cell 489 | NR Cell 1 |  |  |
| 16.7.1.2.2 | NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE | NR Cell 489 | NR Cell 1 |  |  |
| 16.7.1.3.1 | NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.7.1.3.2 | NR SA FR1-FR1 SS-RSRP relative measurement accuracy for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.7.1.4.1 | NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.7.1.4.2 | NR SA FR1-FR1 SS-RSRP relative measurement accuracy for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.7.2.1 | NR SA FR1 SS-RSRQ absolute measurement accuracy for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.7.2.2 | NR SA FR1 SS-RSRQ absolute measurement accuracy for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.7.2.3.1 | NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.7.2.3.2 | NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.7.2.4.1 | NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.7.2.4.2 | NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.7.3.1 | NR SA FR1 SS-SINR absolute measurement accuracy for 1 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.7.3.2 | NR SA FR1 SS-SINR absolute measurement accuracy for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |
| 16.7.3.3.1 | NR SA FR1-FR1 SS-SINR absolute measurement accuracy for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.7.3.3.2 | NR SA FR1-FR1 SS-SINR relative measurement accuracy for 1 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.7.3.4.1 | NR SA FR1-FR1 SS-SINR absolute measurement accuracy for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.7.3.4.2 | NR SA FR1-FR1 SS-SINR relative measurement accuracy for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |
| 16.7.4.1.1 | NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.7.4.1.2 | NR SA FR1 SSB based L1-RSRP relative measurement accuracy for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.7.4.2.1 | NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.7.4.2.2 | NR SA FR1 SSB based L1-RSRP relative measurement accuracy for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.7.4.3.1 | NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.7.4.3.2 | NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for 1 Rx UE | NR Cell 1 |  |  |  |
| 16.7.4.4.1 | NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for 2 Rx UE | NR Cell 1 |  |  |  |
| 16.7.4.4.2 | NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for 2 Rx UE | NR Cell 1 |  |  |  |

Table E.14-2 defines the cell configuration mapping for NR/5GC FR1 – E-UTRA Inter-RAT test cases (serving cell in NR) for RedCap in chapter 16 of this test specification.

Table E.14-2: Cell configuration mapping for SA FR1 – E-UTRA Inter-RAT RRM testing for RedCap

| TC | Description | 38.533 NR Cell1 | 38.533 LTE Cell2 | 38.533 LTE Cell3 | 38.533 LTE Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 16.1.2.1 | NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1 RX UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.1.2.2 | NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2 RX UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.1.2.3 | NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1 RX UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.1.2.4 | NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2 RX UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.1.2.5 | NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.1.2.6 | NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.3.1.7 | NR SA FR1 - E-UTRA handover with known target cell for 1 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.3.1.8 | NR SA FR1 - E-UTRA handover with known target cell for 2 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.3.1.9 | NR SA FR1 - E-UTRA handover with unknown target cell for 1 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.3.1.10 | NR SA FR1 - E-UTRA handover with unknown target cell for 2 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.3.2.3.3 | NR SA FR1 - E-UTRA RRC connection release with redirection for 1 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.3.2.3.4 | NR SA FR1 - E-UTRA RRC connection release with redirection for 2 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.6.3.1 | NR - E-UTRA event-triggered reporting in non-DRX for 1 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.6.3.2 | NR - E-UTRA event-triggered reporting in non-DRX for 2 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.6.3.3 | NR - E-UTRA event-triggered reporting in DRX for 1 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.6.3.4 | NR - E-UTRA event-triggered reporting in DRX for 2 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.6.5.3 | NR SA FR1 - E-UTRA CGI Identification of E-UTRA cell for 1 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.6.5.4 | NR SA FR1 - E-UTRA CGI Identification of E-UTRA cell for 2 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.7.5.1 | NR SA FR1 - E-UTRA RSRP absolute measurement accuracy for 1 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.7.5.2 | NR SA FR1 - E-UTRA RSRP absolute measurement accuracy for 2 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.7.6.1 | NR SA FR1 - E-UTRA RSRQ absolute measurement accuracy for 1 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |
| 16.7.6.2 | NR SA FR1 - E-UTRA RSRQ absolute measurement accuracy for 2 Rx UE | NR Cell 1 | LTE Cell 1 |  |  |  |

# E.15	Cell configuration mapping for SA FR2 test cases for RedCap in Chapter 17

Table E.15-1 defines the cell configuration mapping for NR/5GC FR2 test cases in chapter 17 of this test specification.

Table E.15-1: Cell configuration mapping for SA FR2 RRM testing for RedCap

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 17.1.1.1 | NR SA FR2 Cell reselection for 2 Rx UE | NR Cell 1 | NR Cell 11 |  |  |  |
| 17.1.1.2 | NR SA FR2-FR2 Cell reselection for 2 Rx UE | NR Cell 6 | NR Cell 23 |  |  |  |
| 17.1.1.3 | NR SA FR2 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | NR Cell 1 | NR Cell 11 |  |  |  |
| 17.1.1.4 | NR SA FR2-FR2 Cell reselection for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE | NR Cell 6 | NR Cell 23 |  |  |  |
| 17.2.1.1 | NR SA FR2 TA validation for CG-SDT for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.3.1.1 | NR SA FR2 handover with unknown target cell for 2 Rx | NR Cell 1 | NR Cell 2 |  |  |  |
| 17.3.1.2 | NR SA FR2-FR2 handover with unknown target cell for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |  |
| 17.3.2.1.1 | NR SA FR2 Intra-frequency RRC Re-establishment in FR2 | NR Cell 1 |  |  |  |  |
| 17.3.2.1.2 | NR SA FR2-FR2 Inter-frequency RRC Re-establishment in FR2 | NR Cell 1 |  |  |  |  |
| 17.3.2.1.3 | NR SA FR2 RRC re-establishment without serving cell timing | NR Cell 1 | NR Cell 2 |  |  |  |
| 17.3.2.2.1 | NR SA FR2 4-step RA type contention based random access for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.3.2.2.2 | NR SA FR2 4-step RA type non-contention based random access for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.3.2.2.3 | NR SA FR2 2-step RA type contention based random access for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.3.2.2.4 | NR SA FR2 2-step RA type non-contention based random access for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.3.2.3.1 | NR SA FR2-FR2 RRC connection release with redirection for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |  |
| 17.4.1.1 | NR SA FR2 NR UE Transmit Timing accuracy for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.4.3.1 | NR SA FR2 timing advance adjustment accuracy for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.1.1 | NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM-RS in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.1.2 | NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM-RS in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.1.3 | NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM-RS in DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.1.4 | NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM-RS in DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.1.5 | NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.1.6 | NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.1.7 | NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.1.8 | NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.1.9 | NR SA FR2 Radio Link Monitoring Scheduling Restrictions on FR2 for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.2.1 | NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.2.2 | NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.2.3 | NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.2.4 | NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.2.5 | NR SA FR2 Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.3.1.1 | NR SA FR2 DCI-based and Timer-based DL active BWP switch for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.3.2.1 | NR SA FR2 RRC-based DL active BWP switch for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.4.1.1 | NR SA FR2 MAC-CE based active TCI state switch for a known TCI state for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.4.2.1 | NR SA FR2 RRC-CE based active TCI state switch for a known TCI state for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.5.1.1 | NR SA FR2 MAC-CE based spatial relation switch associated with known DL-RS for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.5.2.1 | NR SA FR2 RRC-based spatial relation switch associated with a known DL-RS for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.5.6.1 | NR SA FR2 UE specific CBW change on PCell for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.6.1.1 | NR SA FR2 event triggered reporting without gap in non-DRX for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |  |
| 17.6.1.2 | NR SA FR2 event triggered reporting without gap in DRX for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |  |
| 17.6.1.3 | NR SA FR2 event triggered reporting with gap in non-DRX for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |  |
| 17.6.1.4 | NR SA FR2 event triggered reporting with gap in DRX for 2 Rx UE | NR Cell 1 | NR Cell 2 |  |  |  |
| 17.6.2.1 | NR SA FR2-FR2 event triggered reporting in non-DRX for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |  |
| 17.6.2.2 | NR SA FR2-FR2 event triggered reporting in DRX for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |  |
| 17.6.2.3 | NR SA FR2-FR2 event triggered reporting with SSB time index detection in non-DRX for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |  |
| 17.6.2.4 | NR SA FR2-FR2 event triggered reporting with SSB time index detection in DRX for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |  |
| 17.6.3.1 | NR SA FR2 SSB based L1-R SRP measurement in non-DRX for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.6.3.2 | NR SA FR2 SSB based L1-RSRP measurement in DRX for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.6.3.3 | NR SA FR2 CSI-RS based L1-RSRP measurement in non-DRX for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.6.3.4 | NR SA FR2 CSI-RS based L1-RSRP measurement in DRX for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.6.4.1 | NR SA FR2-FR2 CGI identification of NR neighbour cell for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |  |
| 17.7.1.1 | NR SA FR2 SS-RSRP measurement accuracy for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.7.1.2 | NR SA FR2-FR2 SS-RSRP measurement accuracy for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |  |
| 17.7.2.1 | NR SA FR2 SS-RSRQ measurement accuracy for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.7.2.2 | NR SA FR2-FR2 SS-RSRQ measurement accuracy for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |  |
| 17.7.3.1 | NR SA FR2 SSB based L1-RSRP measurement accuracy for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.7.3.2 | NR SA FR2 CSI-RS based L1-RSRP measurement accuracy for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.7.4.1 | NR SA FR2 SS-SINR measurement accuracy for 2 Rx UE | NR Cell 1 |  |  |  |  |
| 17.7.4.2 | NR SA FR2-FR2 SS-SINR measurement accuracy for 2 Rx UE | NR Cell 6 | NR Cell 3 |  |  |  |

# E.16	Cell configuration mapping for E-UTRAN – SA test cases for RedCap in Chapter 18

Table E.16-1 defines the cell configuration mapping for E-UTRAN - NR/5GC FR1 Inter-RAT test cases for RedCap (serving cell in LTE) in chapter 18 of this test specification.

Table E.16-1: Cell configuration mapping for E-UTRA – SA FR1 Inter-RAT RRM testing for RedCap

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 18.1.1.1 | E-UTRA - NR SA FR1 E-UTRA cell re-selection to higher priority NR target Cell for 2 Rx UE | LTE Cell 1 | NR Cell 1 |  |  |  |
| 18.2.1.1 | E-UTRA - NR SA FR1 handover for 2 Rx UE | LTE Cell 1 | NR Cell 1 |  |  |  |
| 18.2.2.1 | E-UTRA - NR SA FR1 RRC connection release with redirection for 2 Rx UE | LTE Cell 1 | NR Cell 1 |  |  |  |
| 18.3.1.1 | E-UTRA - NR SA FR1 event triggered reporting without SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 18.3.1.2 | E-UTRA - NR SA FR1 event triggered reporting without SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 18.3.1.3 | E-UTRA - NR SA FR1 event triggered reporting with SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 18.3.1.4 | E-UTRA - NR SA FR1 event triggered reporting with SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 18.3.1.5 | E-UTRA - NR SA FR2 event triggered reporting without SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 18.3.1.6 | E-UTRA - NR SA FR2 event triggered reporting without SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 18.3.1.7 | E-UTRA - NR SA FR2 event triggered reporting with SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 18.3.1.8 | E-UTRA - NR SA FR2 event triggered reporting with SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |

# E.17	Cell configuration mapping for SA FR1 test cases for ATG in Chapter 19

Table E.17-1 defines the cell configuration mapping for NR/5GC FR1 test cases for ATG in chapter 19 of this test specification.

Table E.17-1: Cell configuration mapping for SA FR1 RRM testing for ATG

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 19.2.1.1 | Intra-frequency handover from FR1 to FR1; known target cell | NR Cell 1 | NR Cell 2 |  |  |  |
| 19.2.1.2 | Inter-frequency handover from FR1 to FR1; unknown target cell | NR Cell 6 | NR Cell 3 |  |  |  |
| 19.2.2.1 | Intra-frequency distance-based conditional Handover from FR1 to FR1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 19.2.2.2 | Inter-frequency distance-based conditional Handover from FR1 to FR1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 19.2.3.1.1 | Intra-frequency RRC Re-establishment in FR1 for ATG | NR Cell 1 | NR Cell 2 |  |  |  |
| 19.2.3.1.2 | Inter-frequency RRC Re-establishment in FR1 with unknown target cell without serving cell timing for ATG | NR Cell 6 | NR Cell 3 |  |  |  |
| 19.2.3.3.1 | NR SA FR1-FR1 RRC connection release with Redirection for ATG | NR Cell 6 | NR Cell 23 |  |  |  |
| 19.4.1.1 | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM-RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 19.4.1.2 | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM-RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 19.4.1.3 | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode | NR Cell 1 |  |  |  |  |
| 19.4.1.4 | Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode | NR Cell 1 |  |  |  |  |
| 19.6.1.1.1 | NR SA FR1 SS-RSRP absolute measurement accuracy for ATG | NR Cell 489 | NR Cell 1 |  |  |  |
| 19.6.1.1.2 | NR SA FR1 SS-RSRP relative measurement accuracy for ATG | NR Cell 489 | NR Cell 1 |  |  |  |
| 19.6.1.2.1 | NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for ATG | NR Cell 6 | NR Cell 3 |  |  |  |
| 19.6.1.2.2 | NR SA FR1-FR1 SS-RSRP relative measurement accuracy for ATG | NR Cell 6 | NR Cell 3 |  |  |  |
| 19.6.2.1 | NR SA FR1 SS-RSRQ absolute measurement accuracy for ATG | NR Cell 1 | NR Cell 2 |  |  |  |
| 19.6.2.2.1 | NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for ATG | NR Cell 6 | NR Cell 3 |  |  |  |
| 19.6.2.2.2 | NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for ATG | NR Cell 6 | NR Cell 3 |  |  |  |
| 19.6.3.1 | NR SA FR1 SS-SINR absolute measurement accuracy for ATG | NR Cell 1 | NR Cell 2 |  |  |  |
| 19.6.3.2.1 | NR SA FR1-FR1 SS-SINR absolute measurement accuracy for ATG | NR Cell 6 | NR Cell 3 |  |  |  |
| 19.6.3.2.2 | NR SA FR1-FR1 SS-SINR relative measurement accuracy for ATG | NR Cell 6 | NR Cell 3 |  |  |  |
| 19.6.4.1.1 | NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for ATG | NR Cell 1 |  |  |  |  |
| 19.6.4.1.2 | NR SA FR1 SSB based L1-RSRP relative measurement accuracy for ATG | NR Cell 1 |  |  |  |  |
| 19.6.4.2.1 | NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for ATG | NR Cell 1 |  |  |  |  |
| 19.6.4.2.2 | NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for ATG | NR Cell 1 |  |  |  |  |
| 19.6.5.1.1 | NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG | NR Cell 1 |  |  |  |  |
| 19.6.5.1.2 | NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG | NR Cell 1 |  |  |  |  |
| 19.6.5.2 | NR SA FR1 L1-SINR absolute measurement accuracy with SSB based CMR and dedicated IMR for ATG | NR Cell 1 |  |  |  |  |
| 19.6.5.3.1 | NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG | NR Cell 1 |  |  |  |  |
| 19.6.5.3.2 | NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG | NR Cell 1 |  |  |  |  |
| 19.6.6.1.1 | NR SA FR1 CSI-RSRP absolute measurement accuracy for ATG | NR Cell 489 | NR Cell 1 |  |  |  |
| 19.6.1.1.2 | NR SA FR1 CSI-RSRP relative measurement accuracy for ATG | NR Cell 489 | NR Cell 1 |  |  |  |
| 19.6.7.1 | NR SA FR1 CSI-RSRQ absolute measurement accuracy for ATG | NR Cell 1 | NR Cell 2 |  |  |  |
| 19.6.8.1 | NR SA FR1 CSI-SINR absolute measurement accuracy for ATG | NR Cell 1 | NR Cell 2 |  |  |  |

Annex F (normative):
Measurement uncertainties and test tolerances

The requirements of this clause apply to all tests in the present document.

# F.1	Measurement uncertainties and test tolerances for FR1 and FR2

## F.1.1	Acceptable uncertainty of test system (normative)

See TS 38.521-1 [17] annex F.1.

### F.1.1.1	Measurement of test environments

See TS 38.521-1 [17] Annex F1.1.

### F.1.1.2	Measurement of RRM requirements

This clause defines the maximum test system uncertainty for the RRM requirements. The maximum uncertainty values allowed for the typical RRM measurement uncertainty contributors is defined in Table F.1.1.2-1 and Table F.1.1.2-2. Unless explicitly stated for a particular test case, these maximum uncertainty values should be used as starting point to perform the test tolerance analysis in TR 38.903 [22] for each of the test cases. Specific test cases might require a tighter measurement uncertainty value for some of the contributors. Exceptions to the general values in Table F.1.1.2-1 and Table F.1.1.2-2 shall be handled case by case.

Table F.1.1.2-1: Maximum allowed measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Cell BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc averaged over BWConfig | dB | ±1.5 |  |
| AWGN absolute power, Noc for Measurement PRB | dB | ±1.5 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB105 for CSI-RS-based, or configured RBs in the SRS resource for CLI-based |
| Ratio of cell X signal / AWGN, Êsx / Noc averaged over BWConfig | dB | ±0.3 | Same as in LTE |
| Ratio of cell X signal / AWGN, Êsx / Noc for Measurement PRB | dB | ±0.3 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB105 for CSI-RS-based, or configured RBs in the SRS resource for CLI-based |
| Fading profile uncertainty | dB | ±0.5 for 1 Tx ±0.7 for 2 Tx | Depends on fading profile, can be referenced from TS 38.101-4 [20] |
| AWGN and signal flatness* | dB | ±2.0 | Same as in LTE, can be referenced from TS 38.101-4 [20] |
| Uplink absolute power measurement | dB | ±1.5 | Same as in TS 38.521-1 [17] |
| Uplink relative power measurement | dB | ±0.7 | Same as in TS 38.521-1 [17] |
| Uplink signal transmit timing relative to downlink | Tc | ±112 | Calculated as 25% of tightest UE core requirement for FR1 (25% of 7*64*Tc) |
| Uplink signal transmit timing relative to downlink for satellite access | Tc | ±352 | Calculated as 25% of tightest UE core requirement for FR1 for satellite access (25% of 22*64*Tc) |
| Relative transmit timing accuracy during UE timing adjustment | Tc | ±88 | Calculated as 25% of tightest UE core requirement for FR1 (25% of 5.5*64*Tc) |
| Timing Advance Adjustment accuracy | Tc | ±88 | Calculated as 25% of tightest UE core requirement for FR1 (25% of 5.5*64*Tc) |
| Note 1: The values in this table are specified per cell. Multi-cell test cases need to combined these values in the TT analysis in TR 38.903 Note 2: These values apply for cell BW ≤ 40 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. Note 3: Void |  |  |  |

Table F.1.1.2-2: Maximum allowed measurement uncertainty values for the test system for FR2 (up to 40GHz) and Cell BW ≤ 400 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc averaged over BWConfig | dB | PC3: ±5.654,5 PC1: 5.654,5 PC6: 5.654,5 |  |
| AWGN absolute power, Noc for Measurement PRB | dB | PC3: ±5.654,5 PC1: 5.654,5 PC6: 5.654,5 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based, or configured RBs in the SRS resource for CLI-based |
| Ratio of cell X signal / AWGN, Êsx / Noc averaged over BWConfig | dB | ±0.3 |  |
| Ratio of cell X signal / AWGN, Êsx / Noc for Measurement PRB | dB | ±0.3 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based, or configured RBs in the SRS resource for CLI-based |
| ÊsxNote 3 absolute power, Noc averaged over BWConfig | dB | PC3: ±5.654,5 PC1: 5.654,5 PC6: 5.654,5 |  |
| ÊsxNote 3 absolute power, Noc for Measurement PRB | dB | PC3: ±5.654,5 PC1: 5.654,5 PC6: 5.654,5 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based, or configured RBs in the SRS resource for CLI-based |
| Relative power between any two Ês levels at the same frequency | dB | ±2.0 |  |
| Fading profile uncertainty | dB | ±0.5 for 1 Tx ±0.7 for 2 Tx |  |
| AWGN and signal flatness* | dB | ±3.6 | Same as in Demod Performance test cases, can be referenced from TS 38.521-4 [20] |
| Uplink absolute power measurement | dB | TBD |  |
| Uplink relative power measurement | dB | TBD |  |
| Uplink signal transmit timing relative to downlink | Tc | ±48 |  |
| Relative transmit timing during UE timing adjustment | Tc | ±40 |  |
| Timing Advance Adjustment | Tc | ±40 |  |
| NOTE 1: The values in this table are specified per cell. Multi-cell test cases need to combine these values in the TT analysis in TR 38.903 NOTE 2: These values apply for cell BW ≤ 400 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. NOTE 3: Applies for test cases that use Ês without AWGN NOTE 4: Total Expanded MU for IFF for Max device size ≤ 30cm in TR 38.903 [22] Table E.3.1.3-2 for PC3 UEs, Table E.3.1.3-3 for PC1 UEs and Table E.3.1.3-4 for PC6 UEs. NOTE 5: If the TT analysis for a specific test case based on this MU value results in an unsolvable conflict, making the test case untestable, even after the alternative solutions listed in TR 38.903 [22] clause A.4 have been considered for the test case in TS 38.133 [6] Annex A, the TT analysis shall be repeated using a lower MU value, taking into account the values defined in clause E.3.1 TR 38.903 [22]. The test case will be applicable for the subset of the test systems meeting this reduced MU Threshold. |  |  |  |

The maximum test system uncertainty for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.1.2-3.

The maximum test system uncertainty for the NE-DC FR1 test cases in chapter 4 is defined in Table F.1.1.2-3a.

The maximum test system uncertainty for the NR/5GC FR1 test cases in chapter 6 is defined in Table F.1.1.2-4.

The maximum test system uncertainty for the EN-DC FR2 test cases in chapter 5 is defined in Table F.1.1.2-5.

The maximum test system uncertainty for the NR/5GC FR2 test cases in chapter 7 is defined in Table F.1.1.2-6.

The maximum test system uncertainty for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases in chapter 8 is defined in Table F.1.1.2-7.

The maximum test system uncertainty for the FR1 NR sidelink test cases in chapter 9 is defined in Table F.1.1.2-8.

The maximum test system uncertainty for the EN-DC FR1 test cases for shared spectrum in chapter 10 is defined in Table F.1.1.2-12.

The maximum test system uncertainty for the NR/5GC FR1 test cases for shared spectrum in chapter 11 is defined in Table F.1.1.2-13.

The maximum test system uncertainty for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases for shared spectrum in chapter 12 is defined in Table F.1.1.2-14.

The maximum test system uncertainty for the NR/5GC FR1 test cases with SCell in shared spectrum in chapter 13 is defined in Table F.1.1.2-15.

The maximum test system uncertainty for SA FR1 Satellite access test cases in chapter 14 is defined in Table F.1.1.2-16.

The maximum test system uncertainty for the NR/5GC FR1 test cases for RedCap in chapter 16 is defined in Table F.1.1.2-9.

The maximum test system uncertainty for the NR/5GC FR2 test cases for RedCap in chapter 17 is defined in Table F.1.1.2-10.

The maximum test system uncertainty for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases for RedCap in chapter 18 is defined in Table F.1.1.2-11.

The maximum test system uncertainty for the NR/5GC FR1 test cases for ATG in chapter 19 is defined in Table F.1.1.2-17.

Table F.1.1.2-3: Maximum test system uncertainty for RRM requirements for EN-DC FR1 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 4.3.2.2.1 Contention based random access test in FR1 for PSCell in EN-DC | Noc ±1.5 dB Ês2 / Noc ±0.3 dB Uplink absolute power measurement ±1.5 dB Uplink relative power measurement ±0.7 dB ±112Tc Uplink signal transmit timing relative to downlink | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 4.3.2.2.2 Non-contention based random access test in FR1 for PSCell in EN-DC | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 |
| 4.3.2.2.3 EN-DC FR1 2-step contention based random access | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 |
| 4.3.2.2.4 EN-DC FR1 2-step non-contention based random access | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 |
| 4.4.1.1 EN-DC FR1 UE transmit timing accuracy | Noc ±1.5 dB Ês2 / Noc ±0.3 dB ±112Tc Uplink signal transmit timing relative to downlink | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy | Noc ±1.5 dB Ês2 / Noc ±0.3 dB ±88Tc Timing Advance Adjustment accuracy | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 4.5.1.1 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR Meas PRB are the measurement PRB for SSB-based RLM #RBJ to RBJ+19 |
| 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.3 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM-RS in DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.4 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM-RS in DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.5 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM-RS in non-DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.6 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM-RS in non-DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.7 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM-RS in DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.8 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM-RS in DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.9 EN-DC FR1 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM-RS for UE fulfilling relaxed measurement criterion | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.2.1 EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | ± 0.6dB (AWGN conditions) | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 4.5.2.2 EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | Same as 4.5.2.1 | Same as 4.5.2.1 |
| 4.5.2.3 EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | Noc2 ±1.5 dB Noc3 ±1.5 dB Ês2 / Noc2 ±0.3 dB Ês3 / Noc3 ±0.3 dB | Note: Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |
| 4.5.2.4 EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | Same as 4.5.2.3 | Same as 4.5.2.3 |
| 4.5.2.5 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | Noc1 ±1.5 dB Noc2 ±1.5 dB Noc3 ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB | Note: Noc1 is the AWGN on E-UTRAN cell 1 frequency Ês1 / Noc1 is the ratio of E-UTRAN cell 1 signal / AWGN Noc2 is the AWGN on NR cell 2 frequency Ês2 / Noc2 is the ratio of NR cell 2 signal / AWGN Noc3 is the AWGN on E-UTRAN cell 3 frequency Ês3 / Noc3 is the ratio of E-UTRAN cell 3 signal / AWGN |
| 4.5.2.6 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | Same as 4.5.2.5 | Same as 4.5.2.5 |
| 4.5.2.8 | Same as 4.5.2.1 | Same as 4.5.2.1 |
| 4.5.2.9 | Same as 4.5.2.1 | Same as 4.5.2.1 |
| 4.5.2.10 EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell | Noc ±1.5 dB Ês2 / Noc ±0.3 dB | Note: Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 4.5.3.1 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.5.3.2 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | Same as 4.5.3.1 | Same as 4.5.3.1 |
| 4.5.3.3 EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX | Same as 4.5.3.1 | Same as 4.5.3.1 |
| 4.5.3.4 SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Noc4 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Average Ês4 / Noc4 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Noc4 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB Meas PRB Ês4 / Noc4 ±0.3 dB | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on cell 4 frequency Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.5.3.5 EN-DC FR1 direct SCell activation at SCell addition of known SCell | Same as 4.5.3.1 | Same as 4.5.3.1 |
| 4.5.3.6 EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc1 ±0.3 dB Average Ês3 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB | Note: Noc2 is the AWGN on cell 2 frequency Noc3 is the AWGN on cell 3 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 for CSI-RSRP Meas PRB are # RB0 to # RB105. |
| 4.5.3.7 EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle | Same as 4.5.3.6 | Same as 4.5.3.6 |
| 4.5.3.11 EN-DC FR1 TRS-based SCell Activation of SSB-less SCell in FR1 collocated inter-band in non-DRX | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.5.3.12 EN-DC FR1 Inter-band SSB-less SCell activation using A-TRS | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB | Note: Noc2 is the AWGN on cell 2 frequency Noc3 is the AWGN on cell 3 frequency Ês2 / Noc3 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19. |
| 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay | Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB Noc3 ±1.5 dB Ês3 / Noc3 ±0.3 dB | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |
| 4.5.5.1 EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.5.5.2 EN-DC FR1 SSB-based beam failure detection and link recovery in DRX | Same as 4.5.5.1 | Same as 4.5.5.1 |
| 4.5.5.3 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |
| 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | Same as 4.5.5.3 | Same as 4.5.5.3 |
| 4.5.5.5 EN-DC FR1 SCell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |
| 4.5.5.6 EN-DC FR1 SCell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | Same as 4.5.5.5 | Same as 4.5.5.5 |
| 4.5.5.7 EN-DC FR1 PSCell TRP specific SSB-based beam failure detection and link recovery in non-DRX | Same as 4.5.5.1 | Same as 4.5.5.1 |
| 4.5.5.8 EN-DC FR1 SCell TRP specific CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | Same as 4.5.5.5 | Same as 4.5.5.5 |
| 4.5.6.1.1 EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | ±0.6 dB | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 4.5.6.1.2 EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC | Same as 4.5.6.1.1 | Same as 4.5.6.1.1 |
| 4.5.6.2.1 EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | ±0.6 dB | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 4.5.6.3.1 Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs | ±0.6 dB | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 4.5.6.4.1 E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs | Same as 4.5.6.3.1 | Same as 4.5.6.3.1 |
| 4.5.7.1 EN-DC FR1 addition and release delay of known PSCell | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.5.8.1 EN-DC FR1 interruptions at switching between two uplink carriers | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB | Noc is the AWGN on cell 2 frequency Ês / Noc is the ratio of cell 2 signal / AWGN |
| 4.5.9.1 UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN-DC | ±1.0 dB | The following parameters are proposed to be controlled: - AWGN flatness and signal flatness ±2 dB - Signal-to-noise ratio uncertainty ±0.3 dB Test System uncertainty on PSCell = SQRT (0.32 + (0.25 * 2)2) = 0.58 dB, rounded to 1.0 dB. |
| 4.5.10.1 EN-DC FR1 PSCell activation and deactivation delay | Same as 4.5.7.1 | Same as 4.5.7.1 |
| 4.5.11.1 EN-DC FR1 conditional PSCell addition delay | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.6.1.1 EN-DC FR1 event-triggered reporting without gap in non-DRX | Noc ±1.5 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB | Note: Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |
| 4.6.1.2 EN-DC FR1 event-triggered reporting without gap in DRX | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.3 EN-DC FR1 event-triggered reporting with gap in non-DRX | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.4 EN-DC FR1 event-triggered reporting with gap in DRX | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.5 EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.6 EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.7 EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.8 EN-DC FR1 event triggered reporting cell without SSB time index detection in DRX for UE configured with highSpeedMeasCA-SCell-r17 | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 2 signal / AWGN Ês2 / Noc2 is the ratio of cell 3 signal / AWGN Ês3 / Noc2 is the ratio of cell 4 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.6.2.1 EN-DC FR1-FR1 event-triggered reporting in non-DRX | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB | Noc2 is the AWGN on NR freq2 Noc3 is the AWGN on NR freq3 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.6.2.2 EN-DC FR1-FR1 event-triggered reporting in DRX | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 4.6.2.9 EN-DC FR1-FR1 event triggered reporting without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 4.6.4.1 EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.6.4.2 EN-DC FR1 SSB-based L1-RSRP measurement in DRX | Same as 4.6.4.1 | Same as 4.6.4.1 |
| 4.6.4.3 EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |
| 4.6.4.4 EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX | Same as 4.6.4.3 | Same as 4.6.4.3 |
| 4.6.4.5 EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 4.6.4.1 | Same as 4.6.4.1 |
| 4.6.5.1 EN-DC FR1 SRS-RSRP measurement with non-DRX | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB | Average Noc is the AWGN on virtual UE frequency (NR) on NR freq1 Ês / Noc is the ratio of virtual UE signal / AWGN over the SRS resource RBs |
| 4.6.5.2 EN-DC FR1 CLI-RSSI measurement with non-DRX | Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc2 is the AWGN on cell 2 (NR) frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 4.6.6.1 EN-DC intra-frequency CGI identification of NR FR1 cell with autonomous gaps in synchronous EN-DC | Noc ±1.5 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB | Note: Ês2 / Noc is the ratio of Cell 2 signal / AWGN Ês3 / Noc is the ratio of Cell 3 signal / AWGN |
| 4.6.7.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on Cell 2 frequency Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |
| 4.6.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on Cell 2 frequency Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |
| 4.6.7.3 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on Cell 2 frequency Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |
| 4.7.1.1.1 EN-DC FR1 SS-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Average Ês3 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês2 / Noc ±0.3 dB Meas PRB Ês3 / Noc ±0.3 dB | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 2 SNR Ês3 / Noc is the cell 3 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.7.1.1.2 EN-DC FR1 SS-RSRP relative measurement accuracy | Same as 4.7.1.1.1 | Same as 4.7.1.1.1 |
| 4.7.1.2.1 EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês2 / Noc1 ±0.3 dB Average Ês3 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc1 ±0.3 dB Meas PRB Ês3 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês2 / Noc1 is the cell 2 SNR Ês3 / Noc2 is the cell 3 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.7.1.2.2 EN-DC FR1-FR1 SS-RSRP relative measurement accuracy | Same as 4.7.1.2.1 | Same as 4.7.1.2.1 |
| 4.7.2.1 EN-DC FR1 SS-RSRQ measurement accuracy | Same as 4.7.1.1.1 | Same as 4.7.1.1.1 |
| 4.7.2.2.1 EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy | Same as 4.7.1.2.1 | Same as 4.7.1.2.1 |
| 4.7.2.2.2 EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy | Same as 4.7.1.2.1 | Same as 4.7.1.2.1 |
| 4.7.3.1 EN-DC FR1 SS-SINR measurement accuracy | Same as 4.7.1.1.1 | Same as 4.7.1.1.1 |
| 4.7.3.2.1 EN-DC FR1-FR1 SS-SINR absolute measurement accuracy | Same as 4.7.1.2.1 | Same as 4.7.1.2.1 |
| 4.7.3.2.2 EN-DC FR1-FR1 SS-SINR relative measurement accuracy | Same as 4.7.1.2.1 | Same as 4.7.1.2.1 |
| 4.7.4.1.1 EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 2 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.7.4.1.2 EN-DC FR1 SSB based L1-RSRP relative measurement accuracy | Same as 4.7.4.1.1 | Same as 4.7.4.1.1 |
| 4.7.4.2.1 EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 4.7.4.1.1 | Same as 4.7.4.1.1 |
| 4.7.4.2.2 EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 4.7.4.1.1 | Same as 4.7.4.1.1 |
| 4.7.5.1 EN-DC FR1 SFTD measurement accuracy | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.8 dB | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 Noc2 is the AWGN on E-UTRA freq2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RB22 to RB27 |
| 4.7.6.1 EN-DC SRS-RSRP measurement accuracy with FR1 serving cell | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB | Average Noc is the AWGN on virtual UE frequency (NR) on NR freq1 Ês / Noc is the ratio of virtual UE signal / AWGN over the SRS resource RBs |
| 4.7.7.1.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB | Noc is the AWGN on NR freq1 Ês2 / Noc is the SNR for the CSI-RS#0 & CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |
| 4.7.7.1.2 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy | Same as 4.7.7.1.1 | Same as 4.7.7.1.1 |
| 4.7.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB | Noc is the AWGN on NR freq1 Ês2 / Noc is the SNR for the SSB#0 & SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |
| 4.7.7.3.1 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Same as 4.7.7.1.1 | Same as 4.7.7.1.1 |
| 4.7.7.3.2 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy | Same as 4.7.7.1.1 | Same as 4.7.7.1.1 |
| 4.7.8.1 EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell | Same as 4.7.1.1.1 | Same as 4.7.1.1.1 |
| 4.7.8.2 EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | Same as 4.7.1.2.1 | Same as 4.7.1.2.1 |

Table F.1.1.2-3a: Maximum test system uncertainty for RRM requirements for NE-DC FR1 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 4A.1.1.1 NE-DC FR1 addition and release delay of known PSCell | Noc ±1.5 dB Ês / Noc ±0.3 dB | Note: Ês / Noc is the ratio of E-UTRA cell signal / AWGN |
| 4A.2.1.1 NE-DC FR1 SFTD accuracy | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.8 dB | Note: Ês / Noc is the ratio of NR cell signal / AWGN Ês2 / Noc2 is the ratio of E-UTRA cell signal / AWGN |

Table F.1.1.2-4: Maximum test system uncertainty for RRM requirements for SA FR1 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.1.1.2 NR SA FR1-FR1 cell re-selection | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.1.1.3 NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 6.1.1.4 NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 6.1.1.5 NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 6.1.1.6 NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 6.1.1.7 NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 6.1.1.8 NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17 | Same as 6.1.1.2 | Same as 6.1.1.2 |  |
| 6.1.1.9 NR SA FR1 cell reselection to intra-frequency NR case for UE operating on a cell with less than 5MHz BW | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 6.1.2.1 NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 (NR) frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 (E-UTRAN) frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.1.2.2 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 6.1.2.3 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 6.1.2.4 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 6.1.2.5 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 6.2.1 NR SA FR1 TA Validation for CG-SDT | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Ês / Noc ±0.3 dB | Note: Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.3.1.1 NR SA FR1 handover with known target cell | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.3.1.2 NR SA FR1 handover with unknown target cell | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 6.3.1.3 NR SA FR1-FR1 Handover with unknown Target Cell | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.3.1.4 NR SA FR1 – E-UTRA handover with known target cell | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 6.3.1.5 NR SA FR1 – E-UTRA handover with unknown target cell | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 6.3.1.6 NR SA FR1 – UTRAN FDD handover with known target cell | Noc ±1.5 dB Ês / Noc ±0.3 dB Ioc ±0.7 dB Îor / Ioc ± 0.3dB CPICH Ec / Ior ± 0.1dB SCH Ec / Ior ± 0.1dB | Note: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the ratio of cell 2 CPICH code level / Ior SCH Ec / Ior is the ratio of cell 2 SCH code level / Ior |
| 6.3.1.7 NR SA FR1 synchronous DAPS handover | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 6.3.1.8 NR SA FR1 asynchronous DAPS handover | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 6.3.1.9 NR SA FR1 Intra-band inter-frequency synchronous DAPS handover | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 6.3.1.10 NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 6.3.1.11 NR SA FR1 Inter-band inter-frequency synchronous DAPS handover | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 6.3.1.12 NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 6.3.1.18 NR SA FR1 Intra-frequency handover for unknown target cell operating with 12 PRB SSB bandwidth | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.3.2.1.1 NR SA FR1 RRC re-establishment | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 6.3.2.1.2 NR SA FR1 – FR1 RRC re-establishment | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 6.3.2.1.3 NR SA FR1 RRC re-establishment without serving cell timing | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.3.2.2.1 | Noc ±1.5 dB Ês / Noc ±0.3 dB Uplink absolute power measurement ±1.5 dB Uplink relative power measurement ±0.7 dB ±112Tc Uplink signal transmit timing relative to downlink | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.3.2.2.2 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 6.3.2.2.3 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 6.3.2.2.4 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 6.3.2.3.1 NR SA FR1 RRC connection release with redirection | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 6.3.2.3.2 NR SA FR1 – E-UTRA RRC connection release with redirection | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 6.3.2.4.1 PDCCH-order RACH on neighbor cell in FR1 when RACH BW is within active UL BWP | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB ±112Tc Uplink signal transmit timing relative to downlink | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for Cell 1 Ês1 / Noc is the SNR for Cell 2 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.3.2.4.2 PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB ±112Tc Uplink signal transmit timing relative to downlink | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.3.2.4.3 PDCCH-order RACH on neighbour cell without L1-RSRP measurement in FR1 when RACH BW is within active UL BWP | Same as 6.3.2.4.1 | Same as 6.3.2.4.1 |
| 6.3.3.1 NR SA FR1 conditional handover | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 6.3.3.2 NR SA FR1-FR1 conditional handover | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 6.3.3.3 NR-DC SA FR1 conditional handover including target MCG and target SCG | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc1 ±0.3 dB Average Ês4 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc1 ±0.3 dB Meas PRB Ês4 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 and cell 3 frequency Noc2 is the AWGN on cell 2 and cell 4 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc1 is the ratio of cell 3 signal / AWGN Ês4 / Noc2 is the ratio of cell 4 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19. |
| 6.3.3.4 NR-DC SA FR1 conditional handover including target MCG and candidate SCG | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc1 ±0.3 dB Average Ês4 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc1 ±0.3 dB Meas PRB Ês4 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 and cell 3 frequency Noc2 is the AWGN on cell 2 and cell 4 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc1 is the ratio of cell 3 signal / AWGN Ês4 / Noc2 is the ratio of cell 4 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19. |
| 6.3.3.6 NR SA FR1 Intra-frequency NES triggering conditional handover | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 6.3.3.7 NR SA FR1-FR1 Inter-frequency NES-based conditional handover | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 6.3.4.1 RACH-based Intra-frequency PCell switch from FR1 to FR1 | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.3.4.2 RACH-based Inter-frequency PCell switch from FR1 to FR1 | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq 1 Noc2 is the AWGN on NR freq 2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.3.4.3 RACH-less Intra-frequency PCell switch from FR1 to FR1 | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Noc is the AWGN on NR freq1 Ês1 / Noc is the SNR for Cell 1 Ês2 / Noc is the SNR for Cell 2 Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.3.4.4 NR SA FR1 RACH-less Intra-frequency PCell switch from FR1 to FR1 without L1-RSRP measurement | Same as 6.3.4.3 | Same as 6.3.4.3 |
| 6.4.1.1 EN-DC FR1 UE transmit timing accuracy | Noc ±1.5 dB Ês1 / Noc ±0.3 dB ±112Tc Uplink signal transmit timing relative to downlink | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.4.3.1 NR SA FR1 timing advance adjustment accuracy | Noc ±1.5 dB Ês1 / Noc ±0.3 dB ±88Tc Timing Advance Adjustment accuracy | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.5.1.1 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM-RS in non-DRX mode | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês / Noc ±0.3 dB Fading profile ±0.7 dB | Noc is the AWGN on cell 1 frequency Ês / Noc is the SNR Meas PRB are the measurement PRB for SSB-based RLM #RBJ to RBJ+19 |
| 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM-RS in non-DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.3 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM-RS in DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.4 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM-RS in DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.5 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM-RS in non-DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.6 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM-RS in non-DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.7 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM-RS in DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.8 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM-RS in DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.9 SA FR1 radio link monitoring out-of-sync Test for PSCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.13 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM-RS in DRX mode for UE operating on a cell with less than 5MHz BW | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.14 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM-RS in non-DRX mode for UE operating on a cell with less than 5MHz BW | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.16 NR SA FR1 Radio Link Monitoring In-sync Test for PCell with 3MHz Channel Bandwidth configured with SSB-based RLM-RS in DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC | Noc1 ±1.5 dB Noc2 ±1.5 dB Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.5.2.2 | Same as 4.5.2.1 | Same as 4.5.2.1 |
| 6.5.3.1 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.5.3.2 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5.3.3 NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5.3.4 NR SA FR1 direct SCell activation at SCell addition of known SCell | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5.3.5 NR SA FR1 direct SCell activation at handover with known SCell | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc1 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc1 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.5.3.10 NR SA FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 for CSI-RSRP Meas PRB are # RB0 to # RB105. |
| 6.5.3.11 NR SA FR1 fast SCell Activation of known SCell in non-DRX for 640 ms SCell measurement cycle | Same as 6.5.3.10 | Same as 6.5.3.10 |
| 6.5.3.16 NR SA FR1 Inter-band SSB-less SCell Activation based on A-TRS in non-DRX | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19. |
| 6.5.4.1 NR SA FR1 UE UL carrier RRC reconfiguration delay | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.5.5.1 NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Fading profile ±0.7 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.5.5.2 NR SA FR1 SSB-based beam failure detection and link recovery in DRX | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 6.5.5.3 NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Fading profile ±0.7 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |
| 6.5.5.4 NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX | Same as 6.5.5.3 | Same as 6.5.5.3 |
| 6.5.5.5 NR SA FR1 SCell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the SSB and CSI-RS Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.5.5.6 NR SA FR1 SCell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | Same as 6.5.5.5 | Same as 6.5.5.5 |
| 6.5.5.7 NR SA FR1 PCell TRP Specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX | Same as 6.5.5.3 | Same as 6.5.5.3 |
| 6.5.6.1.1 NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX | ±0.6 dB | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 6.5.6.1.2 NR SA FR1 DCI-based DL active BWP switch in non-DRX | Same as 6.5.6.1.1 | Same as 6.5.6.1.1 |
| 6.5.6.2.1 NR SA FR1 RRC-based DL active BWP switch in non-DRX | ±0.6 dB | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 6.5.6.3.1 NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA | Same as 6.5.6.2.1 | Same as 6.5.6.2.1 |
| 6.5.6.5.1 NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA | Same as 6.5.6.2.1 | Same as 6.5.6.2.1 |
| 6.5.7.1 NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA | Noc1 ±1.5 dB Noc2 ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.5.7.2 NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA | Same as 6.5.7.1 | Same as 6.5.7.1 |
| 6.5.7A.1 NR SA FR1 DL interruptions at switching between two uplink carriers in FDD-TDD CA | Same as 6.5.7.1 | Same as 6.5.7.1 |
| 6.5.7A.2 NR SA FR1 DL interruptions at switching between two uplink carriers in TDD-TDD CA | Same as 6.5.7.1 | Same as 6.5.7.1 |
| 6.5.7B.1 NR SA FR1 DL interruptions at switching between two uplink bands in FDD-TDD CA | Noc1 ±1.5 dB Noc2 ±1.5 dB Noc3 ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |
| 6.5.7B.2 NR SA FR1 DL interruptions at switching between two uplink bands in TDD-TDD CA | Noc1 ±1.5 dB Noc2 ±1.5 dB Noc3 ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |
| 6.5.7C.1 NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA | Same as 6.5.7B.1 | Same as 6.5.7B.1 |
| 6.5.7D.1 NR SA FR1 DL interruptions at switching across three uplink bands in TDD-TDD CA for single TAG | Same as 6.5.7B.1 | Same as 6.5.7B.1 |
| 6.5.7D.2 NR SA FR1 DL interruptions at switching across four uplink bands in FDD-TDD CA for single TAG | Noc1 ±1.5 dB Noc2 ±1.5 dB Noc3 ±1.5 dB Noc4 ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB Ês4 / Noc ±0.3 dB | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Ês4 / Noc4 is the ratio of cell 4 signal / AWGN |
| 6.5.7D.3 NR SA FR1 DL interruptions at switching across three uplink bands in FDD-TDD CA for two TAGs | Same as 6.5.7B.1 | Same as 6.5.7B.1 |
| 6.5.7D.4 NR SA FR1 DL interruptions at switching across four uplink bands in TDD-TDD CA for two TAGs | Same as 6.5.7D.2 | Same as 6.5.7D.2 |
| 6.5.8.1 UE specific CBW change on PCell in FR1 in non-DRX | Test System uncertainty on Cell1: ±0.6 dB | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty: ±0.3 dB, 2. Effect of AWGN flatness and signal flatness: ±2.0 dB. Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) = 0.58dB, round up to 0.6dB. |
| 6.6.1.1 SA event triggered reporting tests without gap under non-DRX | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.6.1.2 SA event triggered reporting tests without gap under DRX | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.3 SA event triggered reporting tests with per-UE gaps under non-DRX | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.4 SA event triggered reporting tests with per-UE gaps under DRX | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.5 SA event triggered reporting tests without gap under non-DRX with SSB index reading | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.6 SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.7 NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.8 NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-SCell-r17 | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.6.1.9 NR SA FR1 event triggered reporting with MUSIM gap configured | Same as 6.6.1.1 | Same as 6.6.1.1 |  |
| 6.6.1.12 NR SA FR1 event triggered reporting with SSB index detection without measurement gap in non-DRX mode for UE operating with 12 PRB SSB bandwidth | Same as 6.6.1.1 | Same as 6.6.1.1 |  |
| 6.6.1.13 NR SA FR1 event triggered reporting tests without gap in DRX under Cell DTX | Same as 6.6.1.1 | Same as 6.6.1.1 |  |
| 6.6.2.1 NR SA FR1-FR1 event-triggered reporting in non-DRX | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.6.2.2 NR SA FR1-FR1 event-triggered reporting in DRX | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.5 NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.6 NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.9 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.10 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.11 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.12 NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 | Same as 6.6.2.1 | Same as 6.6.2.1 |  |
| 6.6.2.13 NR SA FR1-FR1 event triggered reporting using MUSIM gap with lower and higher priority | Same as 6.6.2.1 | Same as 6.6.2.1 |  |
| 6.6.2.14 NR SA FR1-FR1 event triggered reporting using MUSIM gap with shorter MGRP | Same as 6.6.2.1 | Same as 6.6.2.1 |  |
| 6.6.2.15 NR SA FR1 event triggered reporting without SSB index detection in DRX mode for UE operating on a cell with less than 5MHz BW | Same as 6.6.2.1 | Same as 6.6.2.1 |  |
| 6.6.3.1 NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX | Freq 1 Noc ±1.5 dB Freq 2 Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.6.3.2 NR SA FR1 – E-UTRAN event-triggered reporting in DRX | Same as 6.6.3.1 | Same as 6.6.3.1 |
| 6.6.3.3 NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 | Freq 1 Noc ±1.5 dB Freq 2 Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.6.4.1 NR SA FR1 SSB-based L1-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.6.4.2 NR SA FR1 SSB-based L1-RSRP measurement in DRX | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 6.6.4.3 NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |
| 6.6.4.4 NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX | Same as 6.6.4.3 | Same as 6.6.4.3 |
| 6.6.4.5 NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 6.6.4.6 NR SA FR1 Inter-cell SSB based L1-RSRP measurements on PCell in DRX | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 6.6.5.1 NR SA FR1 – UTRAN event-triggered reporting in non-DRX | Noc ±1.5 dB Ês / Noc ±0.3 dB Ioc ±0.7 dB Îor / Ioc ± 0.3dB CPICH Ec / Ior ± 0.1dB SCH Ec / Ior ± 0.1dB | Note: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the ratio of cell 2 CPICH code level / Ior SCH Ec / Ior is the ratio of cell 2 SCH code level / Ior |
| 6.6.6.1 NR SA FR1 SRS-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB | Average Noc is the AWGN on virtual UE frequency (NR) on NR freq1 Ês / Noc is the ratio of virtual UE signal / AWGN over the SRS resource RBs |  |
| 6.6.7.1 SA intra-frequency CGI identification of NR neighbor cell in FR1 | Noc ±1.5 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB | Ês2 / Noc is the ratio of Cell 2 signal / AWGN Ês3 / Noc is the ratio of Cell 3 signal / AWGN |
| 6.6.7.2 Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA | Noc1 ±1.5 dB Noc2 ±1.5 dB Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on Cell 1 frequency. Noc1 is the AWGN on Cell 2 frequency. Ês1 / Noc1 is the ratio of Cell 1 signal / AWGN. Ês2 / Noc2 is the ratio of Cell 2 signal / AWGN. |
| 6.6.8.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on Cell 1 frequency Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |
| 6.6.8.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on Cell 1 frequency Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |
| 6.6.8.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on Cell 1 frequency Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |
| 6.6.9.1 NR SA FR1 Idle mode CA/DC measurement for FR1 | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.6.15.1 NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on LTE freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.6.17.1 NR SA FR1 event triggered reporting tests with autonomous activation/deactivation Pre-MG | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.6.17.2 NR SA FR1 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation | Same as 6.6.17.1 | Same as 6.6.17.1 |
| 6.6.18.1 NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Noc3 is the AWGN on NR freq3 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.6.18.2 NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers | Same as 6.6.18.1 | Same as 6.6.18.1 |
| 6.6.18.3 NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1 | Freq 1 Noc ±1.5 dB Freq 2 Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.6.18.4 NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used | Freq 1 Noc ±1.5 dB Freq 2 Noc ±1.5 dB Freq 3 Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |
| 6.6.19.1 NR SA FR1 event-triggered reporting tests with NCSG under non-DRX in FR1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.19.2 NR SA FR1 event-triggered reporting tests for FR1 with NCSG for inter-frequency measurement | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.19.3 NR SA FR1 NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG | Same as 6.6.3.1 | Same as 6.6.3.1 |
| 6.6.19.4 NR SA FR1 Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX | Same as 6.6.1.8 | Same as 6.6.1.8 |
| 6.6.20.1 UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB | Noc1 is the AWGN on NR freq Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.6.21.1 UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.6.24.1 NR SA FR1 event-triggered reporting without gap with interruptions | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.24.2 NR SA FR1-FR1 event-triggered reporting without gap with interruption with SSB index | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.24.3 NR SA FR1-FR1 event-triggered reporting without gap with interruption | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.24.4 NR SA FR1-FR1 event-triggered reporting without gap without interruption | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.24.5 NR SA FR1 event-triggered reporting without gap without interruption | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.26.1 Intra-frequency SSB based L1-RSRP measurement | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Noc is the AWGN on NR freq Ês1 / Noc is the SNR for the Cell 1 Ês2 / Noc is the SNR for the Cell 2 Meas PRBs are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.6.27.1 Inter-frequency SSB based L1-RSRP measurement with measurement gap | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq 1 Noc2 is the AWGN on NR freq 2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRBs are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.6.28.1 Inter-frequency SSB based L1-RSRP measurement without measurement gap | Same as 6.6.27.1 | Same as 6.6.27.1 |
| 6.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB meas PRB Ês1 / Noc ±0.3 dB meas PRB Ês2 / Noc ±0.3 dB | Noc is the AWGN on NR freq1 Ês1 / Noc is the cell 1 SNR Ês2 / Noc is the cell 2 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 6.7.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB meas PRB Ês1 / Noc1 ±0.3 dB meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the cell 2 SNR Ês2 / Noc2 is the cell 3 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.7.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 6.7.2.1 NR SA FR1 SS-RSRQ measurement accuracy | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 6.7.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 6.7.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 6.7.3.1 NR SA FR1 SS-SINR measurement accuracy | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 6.7.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 6.7.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 6.7.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.8 dB | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 1 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.7.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 6.7.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 6.7.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 6.7.5.1 NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês / Noc ±0.8 dB | Noc is the AWGN on E-UTRA freq1 Ês / Noc is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for RSRP #RB22 to RB27 |
| 6.7.6.1 NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | Same as 6.7.5.1 | Same as 6.7.5.1 |
| 6.7.7.1 NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | Same as 6.7.5.1 | Same as 6.7.5.1 |
| 6.7.8.1 NR SA FR1 SRS-RSRP measurement accuracy | Averaqe Noc ±1.5 dB Average Ês / Noc ±0.3 dB | Average Noc is the AWGN on virtual UE frequency (NR) on NR freq1 Ês / Noc is the ratio of virtual UE signal / AWGN over the SRS resource RBs |  |
| 6.7.9.1.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB | Noc is the AWGN on NR freq1 Ês2 / Noc is the SNR for the CSI-RS#0 & CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |
| 6.7.9.1.2 NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy | Same as 6.7.9.1.1 | Same as 6.7.9.1.1 |
| 6.7.9.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB | Noc is the AWGN on NR freq1 Ês2 / Noc is the SNR for the SSB#0 & SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |
| 6.7.9.3.1 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Same as 6.7.9.1.1 | Same as 6.7.9.1.1 |
| 6.7.9.3.2 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy | Same as 6.7.9.1.1 | Same as 6.7.9.1.1 |
| 6.7.17.1.1 Inter-frequency L1-RSRP absolute accuracy requirements for neighbour cell in FR1 | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq 1 Noc2 is the AWGN on NR freq 2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRBs are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |

Table F.1.1.2-5: Maximum test system uncertainty for RRM requirements for EN-DC FR2 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 5.3.2.2.1 EN-DC FR2 contention based random access | Average Ês ±6.0 dB Meas PRB Ês ±6.0 dB Uplink absolute power measurement ±6.0 dB Uplink relative power measurement ±FFS dB ±[48]Tc Uplink signal transmit timing relative to downlink | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.3.2.2.2 EN-DC FR2 non-contention based random access | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 |
| 5.4.1.1 EN-DC FR2 UE transmit timing accuracy | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB ±48Tc Uplink signal transmit timing relative to downlink ±40Tc Relative during UE timing adjustment | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.4.1.2 NR UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 | Same as 5.4.1.1 | Same as 5.4.1.1 |
| 5.4.3.1 EN-DC FR2 UE timing advance adjustment accuracy | Noc ±5.65 dB, f<=40.8 GHz Ês1 / Noc ±0.3 dB ±40Tc timing advance adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.5.1.1 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode | Average Noc ±5.65 dB, f<=40.8 GHz Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Fading profile ±0.5 dB for 1Tx | Ês1 / Noc is the SNR of RLM-RS1 Ês2 / Noc is the SNR of RLM-RS2 |
| 5.5.1.2 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 5.5.1.3 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM-RS in DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 5.5.1.4 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM-RS in DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 5.5.1.5 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM-RS in non-DRX mode | Average Noc ±5.65 dB, f<=40.8 GHz Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Fading profile ±0.5 dB for 1Tx | Ês1 / Noc is the SNR of RLM-RS1 Ês2 / Noc is the SNR of RLM-RS2 |
| 5.5.1.6 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM-RS in non-DRX mode | Same as 5.5.1.5 | Same as 5.5.1.5 |
| 5.5.1.7 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM-RS in DRX mode | Same as 5.5.1.5 | Same as 5.5.1.5 |
| 5.5.1.8 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM-RS in DRX mode | Same as 5.5.1.5 | Same as 5.5.1.5 |
| 5.5.1.9 EN-DC FR2 radio link monitoring UE scheduling restrictions | Average Noc ±5.65 dB, f<=40.8 GHz Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB | Ês1 / Noc is the SNR of SSB #0 Ês2 / Noc is the SNR of SSB #1 |
| 5.5.1.10 EN-DC FR2 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM-RS for UE fulfilling relaxed measurement criterion | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 5.5.2.1 EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.5.2.2 EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | Same as 5.5.2.1 | Same as 5.5.2.1 |
| 5.5.2.3 EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | Noc ±5.65 dB, f<=40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.5.2.4 EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | Same as 5.5.2.4 | Same as 5.5.2.4 |
| 5.5.3.1 EN-DC FR2 SCell activation and deactivation intra-band in non-DRX | Average Noc2 ±5.65 dB Average Noc3 ±5.65 dB Average Ês2 / Noc ±0.3 dB Average Ês3 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |
| 5.5.3.6 Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2 | Same as 5.5.6.2.1 | Same as 5.5.6.2.1 |
| 5.5.3.7 EN-DC FR2 direct SCell activation at SCell addition of known SCell | Same as 5.5.3.1 | Same as 5.5.3.1 |
| 5.5.3.8 EN-DC FR2 fast SCell Activation of SCell in FR2 intra-band | Average Noc2 ±5.65 dB Average Noc3 ±5.65 dB Average Ês2 / Noc ±0.3 dB Average Ês3 / Noc ±0.3 dB Meas PRB Average Noc2 ±5.65 dB Meas PRB Average Noc3 ±5.65 dB Meas PRB Average Ês2 / Noc ±0.3 dB Meas PRB Average Ês3 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 for CSI-RSRP Meas PRB are # RB0 to # RB274. |
| 5.5.5.1 EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX | Noc averaged over BWConfig, ±5.65 dB, f<=40.8 GHz Noc averaged over measured PRBs, ±5.65 dB, f<=40.8 GHz Ês / Noc averaged over BWConfig: ±0.3 dB Ês / Noc averaged over measured PRBs: ±0.3 dB Fading profile ±0.7 dB, 2Tx | Noc averaged over BWConfig is the AWGN absolute power averaged over BWConfig Noc averaged over measured PRBs is the AWGN absolute power averaged over measured PRBs Ês / Noc averaged over BWConfig is the Ratio of cell signal / AWGN averaged over BWConfig Ês / Noc averaged over measured PRBs is the Ratio of cell signal / AWGN averaged over measured PRBs |
| 5.5.5.2 EN-DC FR2 SSB-based beam failure detection and link recovery in DRX | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.5.3 EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB Fading profile ±0.7 dB for 2Tx | Note: Noc is the AWGN on cell 2 frequency Ês2 / Noc is the SNR for the CSI-RS |
| 5.5.5.4 EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX | Same as 5.5.5.3 | Same as 5.5.5.3 |
| 5.5.5.5 EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.5.6 EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | Noc ±5.65 dB, f<=40.8 GHz Ês3 / Noc ±0.3 dB Fading profile ±0.7 dB for 2Tx | Note: Noc is the AWGN on cell 3 frequency Ês3 / Noc is the SNR for the CSI-RS |
| 5.5.5.7 EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | Same as 5.5.5.6 | Same as 5.5.5.6 |
| 5.5.5.8 EN-DC FR2 CSI-RS-based PSCell TRP specific Beam Failure Detection and Link Recovery in DRX mode | Same as 5.5.5.3 | Same as 5.5.5.3 |
| 5.5.5.9 EN-DC FR2 SSB-based beam failure detection and link recovery in DRX mode for UE fulfilling relaxed measurement criterion | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.6.2.1 EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | ±1.0 dB | Overall system uncertainty for fading conditions comprises two quantities: 1. gNB emulator Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (gNB emulator Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) gNB emulator Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±3.6 dB |
| 5.5.6.3.1 E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC | Same as 5.5.6.2.1 | Same as 5.5.6.2.1 |
| 5.5.6.4.1 E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time | Same as 5.5.6.2.1 | Same as 5.5.6.2.1 |
| 5.5.6.5.1 E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC | Same as 5.5.6.2.1 | Same as 5.5.6.2.1 |
| 5.5.7.1 Addition and Release Delay of NR PSCell | Ês averaged over BW ±5.65 dB, f <= 40.8 GHz Ês averaged over measured PRB ±5.65 dB, f <= 40.8 GHz | Ês averaged over BW is the absolute power of cell 2 signal averaged over BW Ês averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB |
| 5.5.9.1.1 EN-DC PSCell FR2 uplink spatial relation switch for a known spatial relation | ±1.0 dB | The following parameters are proposed to be controlled: - AWGN flatness and signal flatness ±3.6 dB - Signal-to-noise ratio uncertainty ±0.3 dB Test System uncertainty on PSCell = SQRT (0.32 + (0.25 * 3.6)2) = 0.95 dB, rounded to 1.0 dB. |
| 5.5.9.2.1 EN-DC PSCell FR2 uplink spatial relation switch associated with a known DL-RS | Same as 5.5.9.1.1 | Same as 5.5.9.1.1 |
| 5.5.12.1 EN-DC FR2 PSCell activation and deactivation delay | Average Ês2 ±5.65 dB, f <= 40.8 GHz | Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.5.13.1 EN-DC FR2 Addition and Release Delay of NR PSCell | Ês averaged over BW ±5.65 dB, f <= 40.8 GHz Ês averaged over measured PRB ±5.65 dB, f <= 40.8 GHz | Ês averaged over BW is the absolute power of cell 2 signal averaged over BW Ês averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB |
| 5.6.1.1 EN-DC FR2 event-triggered reporting without gap in non-DRX | Ês2 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês2 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz Ês3 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês3 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz | Ês2 averaged over BW is the absolute power of cell 2 signal averaged over BW Ês2 averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB Ês3 averaged over BW is the absolute power of cell 3 signal averaged over BW Ês3 averaged over measured PRB is the absolute power of cell 3 signal averaged over measured PRB |
| 5.6.1.2 EN-DC FR2 event-triggered reporting without gap in DRX | Noc ±5.65 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |
| 5.6.1.3 EN-DC FR2 event-triggered reporting without gap in non-DRX | Same as 5.6.1.1 | Same as 5.6.1.1 |
| 5.6.1.4 EN-DC FR2 event-triggered reporting with gap in DRX | Same as 5.6.1.2 | Same as 5.6.1.2 |
| 5.6.2.1 EN-DC FR2-FR2 event-triggered reporting in non-DRX | Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB Meas PRB Ês2 ±5.65 dB | Ês1 is NR cell 2 signal Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.2.2 EN-DC FR2-FR2 event-triggered reporting in DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc ±0.3 dB Ês2 / Noc over meas PRBs ±0.3 dB | Ês1 is NR cell 2 signal Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.2.3 EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.1 | Same as 5.6.2.1 |
| 5.6.2.4 EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.2 | Same as 5.6.2.2 |
| 5.6.2.5 EN-DC FR1-FR2 event-triggered reporting in non-DRX | Average Ês2 ±5.65, f <= 40.8 GHz | Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.2.6 EN-DC FR1-FR2 event-triggered reporting in DRX | Noc ±5.65 dB, f <= 40.8 GHz Ês2 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.6.2.7 EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 5.6.2.8 EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 5.6.3.1 EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.3.2 EN-DC FR2 SSB-based L1-RSRP measurement in DRX | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 5.6.3.3 EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-RSRP #RBJ to RBJ+65 |
| 5.6.3.4 EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 5.6.3.3 | Same as 5.6.3.3 |
| 5.6.4.1 EN-DC FR2 SRS-RSRP measurement in non-DRX | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell2 signal/AWGN |
| 5.6.6.1 EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-SINR #RBJ to RBJ+275 |
| 5.6.6.2 EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-SINR#RBJ to RBJ+19 |
| 5.6.6.3 EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Same as 5.6.6.1 | Same as 5.6.6.1 |
| 5.6.8.1 EN-DC event triggered reporting tests for NR FR2 cell when DRX is used | Average Noc1 ±5.65 dB Average Noc2 ±5.65 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Average Noc1 ±5.65 dB Meas PRB Average Noc2 ±5.65 dB Meas PRB Average Ês1 / Noc ±0.3 dB Meas PRB Average Ês2 / Noc ±0.3 dB | Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 for CSI-RSRP Meas PRB are # RB0 to # RB274. |
| 5.7.1.1 EN-DC FR2 SS-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Ês2 / Noc is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 2 signal Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.1.2 EN-DC FR2-FR2 SS-RSRP measurement accuracy | Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over measPRBs ±5.65 dB, f <= 40.8 GHz Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over measPRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on the NR freq 1 Noc2 is the AWGN on the NR freq 2 Ês1 / Noc1 is the ratio of cell 2 signal / AWGN Ês2 / Noc2 is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.1.3 EN-DC FR1-FR2 SS-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês2 / Noc is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.2.1 EN-DC FR2 SS-RSRQ measurement accuracy | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Ês2 / Noc is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.2.2 EN-DC FR2-FR2 SS-RSRQ measurement accuracy | Same as 5.7.1.2 | Same as 5.7.1.2 |
| 5.7.3.1 EN-DC FR2 SS-SINR measurement accuracy | Same as 5.7.2.1 | Same as 5.7.2.1 |
| 5.7.3.2 EN-DC FR2-FR2 SS-SINR measurement accuracy | Same as 5.7.1.2 | Same as 5.7.1.2 |
| 5.7.4.1 EN-DC FR2 SSB based L1-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.4.2 EN-DC FR2 CSI-RS based L1-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for CSI-RS-RSRP Test 2 Ês1 is NR cell 2 signal Meas PRB is the measurement PRB for CSI-RS-RSRP |
| 5.7.5.1 EN-DC FR2 SRS-RSRP measurement accuracy | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell2 signal/AWGN |
| 5.7.6.1 EN-DC FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for CSI-RS-RSRP |
| 5.7.6.2 EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SSB or CSI-RS |
| 5.7.6.3 EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for CSI-RS-RSRP |
| 5.7.7.1 EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Ês2 / Noc is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 or CSI-RS #RB0 to RB274 Test 2 Ês1 is NR cell 2 signal Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 or CSI-RS #RB0 to RB274 |
| 5.7.7.2 EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over measPRBs ±5.65 dB, f <= 40.8 GHz Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over measPRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on the NR freq 1 Noc2 is the AWGN on the NR freq 2 Ês1 / Noc1 is the ratio of cell 2 signal / AWGN Ês2 / Noc2 is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 or CSI-RS #RB0 to RB274 |
| 5.7.8.1 EN-DC intra-frequency FR2 CSI-RSRQ measurement accuracy | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Ês2 / Noc is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for CSI-RSRP |
| 5.7.8.2 EN-DC inter-frequency FR2 CSI-RSRQ measurement accuracy | Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over meas PRBs ±5.65 dB, f <= 40.8 GHz Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over meas PRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on the NR freq 1 Noc2 is the AWGN on the NR freq 2 Ês1 / Noc1 is the ratio of cell 2 signal / AWGN Ês2 / Noc2 is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for CSI-RSRP |
| 5.7.9.1 EN-DC intra-frequency FR2 CSI-SINR measurement accuracy | Same as 5.7.8.1 | Same as 5.7.8.1 |
| 5.7.9.2 EN-DC inter-frequency FR2 CSI-SINR measurement accuracy | Same as 5.7.8.2 | Same as 5.7.8.2 |

Table F.1.1.2-6: Maximum test system uncertainty for RRM requirements for SA FR2 test cases

| Subclause |  | Maximum Test System Uncertainty1 |  | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- | --- | --- |
| 7.1.1.1 NR SA FR2 cell re-selection |  | Noc, averaged over BWConfig, ±5.65 dB Noc, averaged over Measurement PRB, ±5.65 dB Es1/Noc averaged over BWConfig ±0.3 dB Es1/Noc averaged over Measurement PRB ±0.3 dB Es2/Noc averaged over BWConfig ±0.3 dB Es2/Noc averaged over Measurement PRB ±0.3 dB |  | Noc averaged over BWConfig is the AWGN absolute power of RF channel 1 averaged over BWConfig Noc averaged over Measurement PRB is the AWGN absolute power of RF channel 1 averaged over Measurement PRB Es1/Noc averaged over BWConfig is Ratio of cell 1 signal / AWGN averaged over BWConfig Es1/Noc averaged over Measurement PRB is Ratio of cell 1 signal / AWGN averaged over Measurement PRB Es2/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |  |
| 7.1.1.2 NR SA FR2-FR2 cell re-selection |  | Noc1, averaged over BWConfig, ±5.65 dB Noc1, averaged over Measurement PRB, ±5.65 dB Noc2, averaged over BWConfig, ±5.65 dB Noc2, averaged over Measurement PRB, ±5.65 dB Es1/Noc1 averaged over BWConfig ±0.3 dB Es1/Noc1 averaged over Measurement PRB ±0.3 dB Es2/Noc2 averaged over BWConfig ±0.3 dB Es2/Noc2 averaged over Measurement PRB ±0.3 dB |  | Noc1 averaged over BWConfig is the AWGN absolute power of RF channel 1 averaged over BWConfig Noc1 averaged over Measurement PRB is the AWGN absolute power of RF channel 1 averaged over Measurement PRB Noc2 averaged over BWConfig is the AWGN absolute power of RF channel 2 averaged over BWConfig Noc2 averaged over Measurement PRB is the AWGN absolute power of RF channel 2 averaged over Measurement PRB Es1/Noc1 averaged over BWConfig is Ratio of cell 1 signal / AWGN averaged over BWConfig Es1/Noc1 averaged over Measurement PRB is Ratio of cell 1 signal / AWGN averaged over Measurement PRB Es2/Noc2 averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc2 averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |  |
| 7.1.1.3 NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | Same as 7.1.1.1 |  | Same as 7.1.1.1 |  |
| 7.1.1.4 NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | Same as 7.1.1.1 |  | Same as 7.1.1.1 |  |
| 7.1.1.5 NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | Same as 7.1.1.2 |  | Same as 7.1.1.2 |  |
| 7.1.1.6 NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | Same as 7.1.1.2 |  | Same as 7.1.1.2 |  |
| 7.3.1.2 NR SA FR2 Intra-frequency handover; unknown target cell |  | Average Noc over BW, ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc over BW, ±0.3 dB, f <= 40.8 GHz Average Ês1 / Noc over meas PRB, ±0.3 dB, f <= 40.8 GHz Average Ês2 / Noc over BW, ±0.3 dB, f <= 40.8 GHz Average Ês2 / Noc over meas PRB, ±0.3 dB, f <= 40.8 GHz |  | Average Noc over BW is AWGN absolute power averaged over BW. Average Noc over meas PRB is AWGN absolute power averaged over measured PRB. Average Ês1 / Noc over BW is the ratio of cell 1 signal / AWGN averaged over BW Average Ês1 / Noc over measured PRB is the ratio of cell 1 signal / AWGN averaged over measured PRB Average Ês2 / Noc over BW is the ratio of cell 2 signal / AWGN averaged over BW. Average Ês2 / Noc over measured PRB is the ratio of cell 2 signal / AWGN averaged over measured PRB. Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.3.1.3 NR SA FR2-FR2 Inter-frequency handover; unknown target cell |  | Average Noc1 over BW ±5.65 dB, f <= 40.8 GHz Noc1 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Noc2 over BW ±5.65 dB, f <= 40.8 GHz Noc2 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc1 over BW, ±0.3 dB, f <= 40.8 GHz Average Ês1 / Noc1 over meas PRB, ±0.3 dB, f <= 40.8 GHz Average Ês2 / Noc2 over BW, ±0.3 dB, f <= 40.8 GHz Average Ês2 / Noc2 over meas PRB, ±0.3 dB, f <= 40.8 GHz |  | Average Noc1 over BW is AWGN absolute power averaged over BW on freq 1. Average Noc1 over meas PRB is AWGN absolute power averaged over measured PRB on freq 1. Average Noc2 over BW is AWGN absolute power averaged over BW on freq 2. Average Noc2 over meas PRB is AWGN absolute power averaged over measured PRB on freq 2. Average Ês1 / Noc1 over BW is the ratio of cell 1 signal / AWGN averaged over BW on freq 1. Average Ês1 / Noc1 over measured PRB is the ratio of cell 1 signal / AWGN averaged over measured PRB on freq 1. Average Ês2 / Noc2 over BW is the ratio of cell 2 signal / AWGN averaged over BW on freq 2. Average Ês2 / Noc2 over measured PRB is the ratio of cell 2 signal / AWGN averaged over measured PRB on freq 2. Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.3.1.4 NR SA FR1-FR2 synchronous DAPS handover |  | Noc, averaged over BWConfig, ±5.65 dB Noc, averaged over Measurement PRB, ±5.65 dB Es2/Noc averaged over BWConfig ±0.3 dB Es2/Noc averaged over Measurement PRB ±0.3 dB |  | Noc averaged over BWConfig is the AWGN absolute power of RF channel 2 averaged over BWConfig Noc averaged over Measurement PRB is the AWGN absolute power of RF channel 2 averaged over Measurement PRB Es2/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |  |
| 7.3.1.5 NR SA FR1-FR2 asynchronous DAPS handover |  | Same as 7.3.1.4 |  | Same as 7.3.1.4 |  |
| 7.3.2.1.1 NR SA FR2 RRC re-establishment |  | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc ±0.3 dB Ês2 / Noc over meas PRBs ±0.3 dB |  | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.3.2.1.2 NR SA FR2 - FR2 RRC re-establishment |  | Average Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc1 ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Ês2 / Noc over meas PRBs ±0.3 dB |  | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.3.2.1.3 NR SA FR2 RRC re-establishment without serving cell timing |  | Same as 7.3.2.1.1 |  | Same as 7.3.2.1.1 |  |
| 7.3.2.2.1 NR SA FR2 contention based random access |  | Average Ês ±6.0 dB Meas PRB Ês ±6.0 dB Uplink absolute power measurement ±6.0 dB Uplink relative power measurement ±FFS dB ±[48]Tc Uplink signal transmit timing relative to downlink |  | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |
| 7.3.2.2.2 NR SA FR2 non-contention based random access |  | Same as 7.3.2.2.1 |  | Same as 7.3.2.2.1 |  |
| 7.3.2.3.1 NR SA FR2-FR2 RRC connection release with redirection |  | Same as 7.3.1.3 |  | Same as 7.3.1.3 |  |
| 7.3.3.1 NR SA FR2 conditional handover |  | Noc averaged over BWConfig, ±5.65 dB Noc, averaged over Measurement PRB, ±5.65 dB Es1/Noc averaged over BWConfig ±0.3 dB Es1/Noc averaged over Measurement PRB ±0.3 dB Es2/Noc ±0.3 dB Noc, averaged over BWConfig, ±5.65 dB Es2/Noc averaged over Measurement PRB ±0.3 dB |  | Noc averaged over BWConfig,is the AWGN absolute power of RF channel 1 averaged over BWConfig, Noc averaged over Measurement PRB,is the AWGN absolute power of RF channel 1 averaged over Measurement PRB, Es1/Noc averaged over BWConfig is Ratio of cell 1 signal / AWGN averaged over BWConfig Es1/Noc averaged over Measurement PRB is Ratio of cell 1 signal / AWGN averaged over Measurement PRB Es2/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |  |
| 7.3.3.3 |  | Same as 7.3.3.1 |  | Same as 7.3.3.1 |  |
| 7.3.4.1 NR SA FR2 RACH based Intra-frequency PCell switch from FR2 to FR2 |  | Same as 7.3.1.2 |  | Same as 7.3.1.2 |  |
| 7.3.4.2 NR SA FR2 RACH-less Intra-frequency PCell switch from FR2 to FR2 |  | Same as 7.3.1.2 |  | Same as 7.3.1.2 |  |
| 7.3.4.3 NR SA FR2 RACH-based Inter-frequency LTM PCell switch from FR2 to FR2 |  | Same as 7.3.1.3 |  | Same as 7.3.1.3 |  |
| 7.4.1.1 SA FR2 UE transmit timing accuracy |  | Noc ±5.65 dB Ês1 / Noc ±0.3 dB ±48Tc Uplink signal transmit timing relative to downlink ±40Tc Relative during UE timing adjustment |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |
| 7.4.1.3 SA FR2 UE transmit timing accuracy |  | Noc ±5.65 dB Ês1 / Noc ±0.3 dB ±48Tc Uplink signal transmit timing relative to downlink ±40Tc Relative during UE timing adjustment |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |
| 7.4.3.1 NR SA FR2 UE timing advance adjustment accuracy |  | Noc ±5.65 dB Ês1 / Noc ±0.3 dB ±40Tc timing advance adjustment |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |
| 7.5.1.1 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM-RS in non-DRX mode |  | Same as 5.5.1.1 |  | Same as 5.5.1.1 |  |
| 7.5.1.2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM-RS in non-DRX mode |  | Same as 5.5.1.1 |  | Same as 5.5.1.1 |  |
| 7.5.1.3 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM-RS in DRX mode |  | Same as 5.5.1.1 |  | Same as 5.5.1.1 |  |
| 7.5.1.4 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM-RS in DRX mode |  | Same as 5.5.1.1 |  | Same as 5.5.1.1 |  |
| 7.5.1.9 EN-DC FR2 radio link monitoring UE scheduling restrictions |  | Same as 5.5.1.1 |  | Same as 5.5.1.1 |  |
| 7.5.3.4 NR SA FR2 direct SCell activation at SCell addition of known SCell | Average Noc1 ±5.65 dB Average Noc2 ±5.65 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB |  | Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |
| 7.5.3.5 NR SA FR2 direct SCell activation at handover with known SCell | Average Noc1 ±5.65 dB Average Noc2 ±5.65 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Average Ês3 / Noc ±0.3 dB |  | Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc1 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN |  |
| 7.5.3.13 NR SA FR2 SCell Activation of SCell in FR2 intra-band in non-DRX | Average Noc1 ±5.65 dB Average Noc2 ±5.65 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Average Noc1 ±5.65 dB Meas PRB Average Noc2 ±5.65 dB Meas PRB Average Ês1 / Noc ±0.3 dB Meas PRB Average Ês2 / Noc ±0.3 dB |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 for CSI-RSRP Meas PRB are # RB0 to # RB274. |  |
| 7.5.3.14 NR SA FR2 SCell Activation for known SCell in FR2 inter-band | Noc1 averaged over BWConfig, ±5.65 dB Noc1, averaged over Measurement PRB, ±5.65 dB Noc2 averaged over BWConfig, ±5.65 dB Noc2, averaged over Measurement PRB, ±5.65 dB Es1/Noc1 averaged over BWConfig ±0.3 dB Es1/Noc1 averaged over Measurement PRB ±0.3 dB Es2/Noc2 ±0.3 dB Noc, averaged over BWConfig, ±5.65 dB Es2/Noc2 averaged over Measurement PRB ±0.3 dB |  | Noc1 averaged over BWConfig,is the AWGN absolute power of RF channel 1 averaged over BWConfig Noc1 averaged over Measurement PRB,is the AWGN absolute power of RF channel 1 averaged over Measurement PRB Noc2 averaged over BWConfig,is the AWGN absolute power of RF channel 2 averaged over BWConfig, Noc2 averaged over Measurement PRB,is the AWGN absolute power of RF channel 2 averaged over Measurement PRB, Es1/Noc averaged over BWConfig is Ratio of cell 1 signal / AWGN averaged over BWConfig Es1/Noc averaged over Measurement PRB is Ratio of cell 1 signal / AWGN averaged over Measurement PRB Es2/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |  |
| 7.5.5.1 NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX |  | Same as 5.5.5.1 |  | Same as 5.5.5.1 |  |
| 7.5.5.2 NR SA FR2 SSB-based beam failure detection and link recovery in DRX |  | Same as 5.5.5.1 |  | Same as 5.5.5.1 |  |
| 7.5.5.3 NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX |  | Noc ±5.65 dB, f<=40.8 GHz Ês1 / Noc ±0.3 dB Fading profile ±0.7 dB for 2Tx |  | Note: Noc is the AWGN on cell 1 frequency Ês1 / Noc is the SNR for the CSI-RS |  |
| 7.5.5.4 NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX |  | Same as 7.5.5.3 |  | Same as 7.5.5.3 |  |
| 7.5.5.5 NR SA FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX |  | Same as 5.5.5.1 |  | Same as 5.5.5.1 |  |
| 7.5.5.6 NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX |  | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB Fading profile ±0.7 dB for 2Tx |  | Note: Noc is the AWGN on cell 2 frequency Ês2 / Noc is the SNR for the CSI-RS |  |
| 7.5.5.7 NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX |  | Same as 7.5.5.6 |  | Same as 7.5.5.6 |  |
| 7.5.5.9 NR SA FR2 SCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX |  | Same as 7.5.5.6 |  | Same as 7.5.5.6 |  |
| 7.5.5.10 NR SA FR2 PCell TRP specific SSB-based Beam Failure Detection and Link Recovery in non-DRX |  | Same as 5.5.5.1 |  | Same as 5.5.5.1 |  |
| 7.5.6.2.1 NR SA FR2 RRC-based DL active BWP switch in non-DRX |  | ±1.0 dB |  | Overall system uncertainty for fading conditions comprises two quantities: 1. gNB emulator Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (gNB emulator Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) gNB emulator Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±3.6 dB |  |
| 7.5.6.3.1 Rel16 test requirements for BWP switching on multiple CCs |  | Same as 7.5.6.2.1 |  | Same as 7.5.6.2.1 |  |
| 7.5.6.5.1 Rel16 test requirements for active BWP switch |  | Same as 7.5.6.2.1 |  | Same as 7.5.6.2.1 |  |
| 7.5.8.1.1 NR SA PCell FR2 MAC-CE based active TCI state switch for a known TCI state |  | Noc ±5.65 dB, f<=40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB No fading |  | Noc is the AWGN on cell 1 and 2 frequency Ês1 / Noc is Ratio of cell 1 signal / AWGN averaged over BW Ês2 / Noc is Ratio of cell 2 signal / AWGN averaged over BW |  |
| 7.5.8.2.1 NR SA PCell FR2 RRC based active TCI state switch for a known TCI state |  | Same as 7.5.8.1.1 |  | Same as 7.5.8.1.1 |  |
| 7.5.9.1.1 EN-DC PSCell FR2 uplink spatial relation switch for a known spatial relation |  | ±1.0 dB |  | The following parameters are proposed to be controlled: - AWGN flatness and signal flatness ±3.6 dB - Signal-to-noise ratio uncertainty ±0.3 dB Test System uncertainty on PSCell = SQRT (0.32 + (0.25 * 3.6)2) = 0.95 dB, rounded to 1.0 dB. |  |
| 7.5.9.2.1 EN-DC PSCell FR2 uplink spatial relation switch associated with a known DL-RS |  | Same as 7.5.9.1.1 |  | Same as 7.5.9.1.1 |  |
| 7.5.11.1 UE UL carrier RRC reconfiguration delay |  | Noc ±5.65 dB, f<=40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB No fading |  | Note: Noc is the AWGN on cell 1 and 2 frequencies Ês1 / Noc is Ratio of cell 1 signal / AWGN averaged over BW Ês2 / Noc is Ratio of cell 2 signal / AWGN averaged over BW |  |
| 7.5.12.1 Addition and Release Delay of PSCell |  | Ês averaged over BW ±5.65 dB, f <= 40.8 GHz Ês averaged over measured PRB ±5.65 dB, f <= 40.8 GHz |  | Ês averaged over BW is the absolute power of cell 2 signal averaged over BW Ês averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB |  |
| 7.5.13.6.1 NR SA FR2 MAC-CE based active uplink TCI state switch for 2 known TCI states |  | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc ±0.3 dB Ês2 / Noc over meas PRBs ±0.3 dB Average Ês3 / Noc ±0.3 dB Ês3 / Noc over meas PRBs ±0.3 dB |  | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Ês2 / Noc is the SNR for the SSB#2 Ês3 / Noc is the SNR for the SSB#3 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 for 120kHz SCSSSB |  |
| 7.5.14 NR SA FR2 PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2 |  | Noc averaged over BWConfig, ±5.65 dB Noc, averaged over Measurement PRB, ±5.65 dB Es/Noc averaged over BWConfig ±0.3 dB Es/Noc averaged over Measurement PRB ±0.3 dB |  | Noc averaged over BWConfig,is the AWGN absolute power of RF channel 2 averaged over BWConfig Noc averaged over Measurement PRB,is the AWGN absolute power of RF channel 2 averaged over Measurement PRB Es/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |  |
| 7.6.1.1 NR SA FR2 event-triggered reporting without gap in non-DRX |  | Ês1 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês1 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz Ês2 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês2 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz |  | Ês1 averaged over BW is the absolute power of cell 1 signal averaged over BW Ês1 averaged over measured PRB is the absolute power of cell 1 signal averaged over measured PRB Ês2 averaged over BW is the absolute power of cell 2 signal averaged over BW Ês2 averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB |  |
| 7.6.1.2 NR SA FR2 event-triggered reporting without gap in DRX |  | Noc averaged over BW, ±5.65 dB, f <= 40.8 GHz Noc averaged over measured PRB, ±5.65 dB, f <= 40.8 GHz Ês1 / Noc averaged over BW, ±0.3 dB, f <= 40.8 GHz Ês1 / Noc averaged over measured PRB, ±0.3 dB, f <= 40.8 GHz Ês2 / Noc averaged over BW, ±0.3 dB, f <= 40.8 GHz Ês2 / Noc averaged over measured PRB, ±0.3 dB, f <= 40.8 GHz |  | Noc averaged over BW is AWGN absolute power averaged over BW. Noc averaged over measured PRB is AWGN absolute power averaged over measured PRB. Ês1 / Noc averaged over BW is the ratio of cell 1 signal / AWGN averaged over BW Ês1 / Noc averaged over measured PRB is the ratio of cell 1 signal / AWGN averaged over measured PRB Ês2 / Noc averaged over BW is the ratio of cell 2 signal / AWGN averaged over BW. Ês2 / Noc averaged over measured PRB is the ratio of cell 2 signal / AWGN averaged over measured PRB. |  |
| 7.6.1.3 NR SA FR2 event-triggered reporting without gap in non-DRX |  | Same as 7.6.1.1 |  | Same as 7.6.1.1 |  |
| 7.6.1.4 NR SA FR2 event-triggered reporting with gap in DRX |  | Same as 7.6.1.2 |  | Same as 7.6.1.2 |  |
| 7.6.2.1 NR SA FR2-FR2 event-triggered reporting in non-DRX |  | Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB Meas PRB Ês2 ±5.65 dB |  | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.6.2.2 NR SA FR2-FR2 event-triggered reporting in DRX |  | Average Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc1 ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Ês2 / Noc2 over meas PRBs ±0.3 dB |  | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.6.2.3 NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection |  | Same as 7.6.2.1 |  | Same as 7.6.2.1 |  |
| 7.6.2.4 NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection |  | Same as 7.6.2.2 |  | Same as 7.6.2.2 |  |
| 7.6.2.5 NR SA FR1-FR2 event-triggered reporting in non-DRX |  | Average Ês2 ±5.65 dB, f <= 40.8 GHz |  | Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.6.2.6 NR SA FR1-FR2 event-triggered reporting in DRX |  | Noc ±5.65 dB, f <= 40.8 GHz Ês2 / Noc ±0.3 dB |  | Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 7.6.2.7 NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection |  | Same as 7.6.2.5 |  | Same as 7.6.2.5 |  |
| 7.6.2.8 NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection |  | Same as 7.6.2.6 |  | Same as 7.6.2.6 |  |
| 7.6.2.10 |  | Same as 7.6.2.2 |  | Same as 7.6.2.2 |  |
| 7.6.2.11 |  | Same as 7.6.2.2 |  | Same as 7.6.2.2 |  |
| 7.6.2.20 NR SA FR2-FR2 event triggered reporting using MUSIM gap with higher priority |  | Same as 7.6.2.1 |  | Same as 7.6.2.1 |  |
| 7.6.3.1 NR SA FR2 SSB-based L1-RSRP measurement in non-DRX |  | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB |  | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.6.3.2 NR SA FR2 SSB-based L1-RSRP measurement in DRX |  | Same as 7.6.3.1 |  | Same as 7.6.3.1 |  |
| 7.6.3.3 NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX |  | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB |  | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-RSRP |  |
| 7.6.3.4 NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX |  | Same as 7.6.3.3 |  | Same as 7.6.3.3 |  |
| 7.6.3.6 NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used |  | Same as 7.6.6.2 |  | Same as 7.6.6.2 |  |
| 7.6.4.1 NR SA FR2 SRS-RSRP measurement in non-DRX |  | Same as 5.6.4.1 |  | Same as 5.6.4.1 |  |
| 7.6.6.1 NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy |  | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB |  | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-SINR #RBJ to RBJ+275 |  |
| 7.6.6.2 NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy |  | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB |  | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |  |
| 7.6.6.3 NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy |  | Same as 7.6.6.1 |  | Same as 7.6.6.1 |  |
| 7.6.13.1 NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using PRS |  | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for PRS-RSRP #RBJ to RBJ+19 |  |
| 7.6.13.2 NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using TRS |  | Same as 7.6.13.1 |  | Same as 7.6.13.1 |  |
| 7.6.14.1 NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and autonomous activation/deactivation |  | Ês1 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês1 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz Ês2 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês2 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz |  | Ês1 averaged over BW is the absolute power of cell 1 signal averaged over BW Ês1 averaged over measured PRB is the absolute power of cell 1 signal averaged over measured PRB Ês2 averaged over BW is the absolute power of cell 2 signal averaged over BW Ês2 averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB |  |
| 7.6.14.2 NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation |  | Same as 7.6.14.1 |  | Same as 7.6.14.1 |  |
| 7.6.15.1 NR SA FR2 SA event triggered reporting tests with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements |  | Ês1 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês1 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz Ês2 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês2 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz Ês3 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês3 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz |  | Ês1 averaged over BW is the absolute power of cell 1 signal averaged over BW Ês1 averaged over measured PRB is the absolute power of cell 1 signal averaged over measured PRB Ês2 averaged over BW is the absolute power of cell 2 signal averaged over BW Ês2 averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB Ês3 averaged over BW is the absolute power of cell 2 signal averaged over BW Ês3 averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB |  |
| 7.6.15.2 NR SA FR2 event triggered reporting tests with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2) |  | Same as 7.6.15.1 |  | Same as 7.6.15.1 |  |
| 7.6.15.3 NR SA FR2 event triggered reporting tests with concurrent measurement gaps with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement |  | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Meas PRB Ês3 / Noc ±0.3 dB |  | Noc is the AWGN on the NR freq 1 and 2 Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.6.16.1 NR SA FR2 event triggered reporting tests with per-UE NCSG under non-DRX |  | Same as 7.6.1.1 |  | Same as 7.6.1.1 |  |
| 7.6.16.2 NR SA FR2 event triggered reporting tests on inter-frequency measurement with NCSG when DRX is not used(PCell in FR2) |  | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB |  | Noc is the AWGN on the NR freq 1 and 2 Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.6.16.3 NR SA FR2 event triggered reporting tests on deactivated SCell measurement via NCSG in non-DRX |  | Ês2 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês2 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz Ês3 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês3 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz |  | Ês2 averaged over BW is the absolute power of cell 2 signal averaged over BW Ês2 averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB Ês3 averaged over BW is the absolute power of cell 3 signal averaged over BW Ês3 averaged over measured PRB is the absolute power of cell 3 signal averaged over measured PRB |  |
| 7.6.20.1 NR SA FR2 Intra-frequency SSB based L1-RSRP measurement |  | Average Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz |  | Ês1 is NR Cell 1 signal Ês2 is NR Cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.6.21.1 NR SA FR2 Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM |  | Average Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz |  | Ês1 is NR Cell 1 signal Ês2 is NR Cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.7.1.1 NR SA FR2 SS-RSRP measurement accuracy |  | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz |  | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.7.1.2 NR SA FR2-FR2 SS-RSRP measurement accuracy |  | Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over measPRBs ±5.65 dB, f <= 40.8 GHz Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over measPRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB |  | Noc1 is the AWGN on the NR freq 1 Noc2 is the AWGN on the NR freq 2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.7.1.3 NR SA FR1-FR2 SS-RSRP measurement accuracy |  | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz |  | Test 1 Noc is the AWGN on the NR freq Ês2 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.7.2.1 NR SA FR2 SS-RSRQ measurement accuracy |  | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB |  | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.7.2.2 NR SA FR2-FR2 SS-RSRQ measurement accuracy |  | Same as 7.7.1.2 |  | Same as 7.7.1.2 |  |
| 7.7.3.1 NR SA FR2 SS-SINR measurement accuracy |  | Same as 7.7.2.1 |  | Same as 7.7.2.1 |  |
| 7.7.3.3 NR SA FR2-FR2 SS-SINR measurement accuracy |  | Same as 7.7.1.2 |  | Same as 7.7.1.2 |  |
| 7.7.4.1 NR SA FR2 SSB based L1-RSRP measurement accuracy |  | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz |  | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 1 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.7.4.2 NR SA FR2 CSI-RS based L1-RSRP measurement accuracy |  | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz |  | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for CSI-RS-RSRP Test 2 Ês1 is NR cell 1 signal Meas PRB is the measurement PRB for CSI-RS-RSRP |  |
| 7.7.5.1 NR SA FR2 SRS-RSRP measurement accuracy |  | Same as 5.7.5.1 |  | Same as 5.7.5.1 |  |
| 7.7.6.1 NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy |  | Same as 7.6.6.1 |  | Same as 7.6.6.1 |  |
| 7.7.6.2 NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy |  | Same as 7.6.6.2 |  | Same as 7.6.6.2 |  |
| 7.7.6.3 NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy |  | Same as 7.6.6.1 |  | Same as 7.6.6.1 |  |
| 7.7.8.1 NR SA intra-frequency FR2 CSI-RSRQ measurement accuracy |  | Same as 5.7.8.1 |  | Same as 5.7.8.1 |  |
| 7.7.8.2 NR SA inter-frequency FR2 CSI-RSRQ measurement accuracy |  | Same as 5.7.8.2 |  | Same as 5.7.8.2 |  |
| 7.7.9.1 NR SA intra-frequency FR2 CSI-SINR measurement accuracy |  | Same as 5.7.9.1 |  | Same as 5.7.9.1 |  |
| 7.7.9.2 NR SA inter-frequency FR2 CSI-SINR measurement accuracy |  | Same as 5.7.9.2 |  | Same as 5.7.9.2 |  |
| 7.7.15.1 NR SA FR2 SSB based inter-frequency L1-RSRP measurement |  | Test 1 : Average Noc1 ±5.65 dB, f <= 40.8 GHz Average Noc2 ±5.65 dB, f <= 40.8 GHz Average Ês1/Noc1 ±0.3 dB, f<=40.8 GHz Average Ês2/Noc2 ±0.3 dB, f<=40.8 GHz Meas PRB Noc1 ±5.65 dB, f <= 40.8 GHz Meas PRB Noc2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1/Noc1 ±0.3 dB, f<=40.8 GHz Meas PRB Ês2/Noc2 ±0.3 dB, f<=40.8 GHz Test 2 : Average Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz |  | Test 1 : Noc1 is the AWGN on the NR freq 1 Noc2 is the AWGN on the NR freq 2 Ês1/Noc1 is the ratio of Cell 1 signal/AWGN Ês2/Noc2 is the ratio of Cell 2 signal/AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 : Ês1 is NR Cell 1 signal Ês2 is NR Cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |

Table F.1.1.2-7: Maximum test system uncertainty for RRM requirements for E-UTRA – NR inter-RAT test cases with E-UTRA serving cell

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 8.2.1.1 E-UTRA – NR FR1 cell re-selection to higher priority NR target cell | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.2.1.2 E-UTRA – NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16 | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.2.2.1 E-UTRA – NR FR1 Early Measurement Reporting | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.2.2.2 E-UTRA – NR FR2 Early Measurement Reporting | Average Noc1 ±5.65 dB Average Noc2 ±5.65 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±5.65 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.3.1.1 E-UTRA – NR FR1 handover with known target cell | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.4.1.1 E-UTRA – NR FR1 SFTD measurement delay in non-DRX | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.4.1.2 E-UTRA – NR FR1 SFTD measurement delay in DRX | Same as 8.4.1.1 | Same as 8.4.1.1 |
| 8.4.2.1 E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.4.2.2 E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.3 E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.4 E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.5 E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX | Average Noc2 ±5.65 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±5.65 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.4.2.6 E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX | Average Noc2 ±5.65 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±5.65 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.4.2.7 E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX | Same as 8.4.2.6 | Same as 8.4.2.6 |
| 8.4.2.8 E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX | Same as 8.4.2.6 | Same as 8.4.2.6 |
| 8.4.2.9 E-UTRA – NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16 | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB |
| 8.5.1.1 E-UTRA – NR FR1 SFTD measurement accuracy | Average Ês1 / Noc1 ±0.8 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Ês1 / Noc1 ±0.8 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.5.2.1.1.1 E-UTRA SS-RSRP absolute measurement accuracy of a NR FR1 neighbour cell | Average Noc ±1.5 dB Noc over meas PRBs #RBJ-J+19 ±1.5 dB Average Ês / Noc ±0.3 dB meas PRB Ês1 / Noc1 ±0.8 dB | Noc is the AWGN on the NR freq Ês / Noc is the cell SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.5.2.1.2 E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell | Test 1 Average Noc ±5.65 dB Noc over meas PRBs ±5.65 dB Average Ês / Noc ±0.3 dB Ês / Noc over meas PRBs ±0.3 dB Test 2 Average Es ±5.65 dB Es over meas PRBs ±5.65 dB | Test 1 Noc is the AWGN on the NR freq Ês / Noc is the cell SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.5.2.2.1 E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell | Same as 8.5.2.1.1.1 | Same as 8.5.2.1.1.1 |
| 8.5.2.2.2 E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR2 neighbour cell | Same as 8.5.2.1.2 | Same as 8.5.2.1.2 |
| 8.5.2.3.1 E-UTRA SS-SINR absolute measurement accuracy of a NR FR1 neighbour cell | Same as 8.5.2.1.1.1 | Same as 8.5.2.1.1.1 |
| 8.5.2.3.2 E-UTRA SS-SINR absolute measurement accuracy of a NR FR2 neighbour cell | Same as 8.5.2.1.2 | Same as 8.5.2.1.2 |

Table F.1.1.2-8: Maximum test system uncertainty for RRM requirements for FR1 NR sidelink test cases

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 9.1.1.1 NR SA FR1 UE transmit timing accuracy for GNSS as synchronization reference source | Sidelink signal transmit timing relative to GNSS timing: ±192 Tc | Tc is the basic timing unit defined in TS 38.211 [7], Tc = 1/(480000 x 4096) seconds |
| 9.1.1.2 NR SA FR1 UE transmit timing accuracy for SyncRef UE as synchronization reference source | Sidelink signal transmit timing relative to SyncRef UE: ±128 Tc for SL SCS = 30kHz Average Noc: ±1.5 dB Average Ês1 / Noc: ±0.3 dB Meas PRB Noc: ±1.5 dB Meas PRB Ês1 / Noc: ±0.3 dB | Tc is the basic timing unit defined in TS 38.211 [7], Tc = 1/(480000 x 4096) seconds Noc is the AWGN absolute power on PC5 carrier. Ês1 / Noc is the ratio of SyncRef UE 1 signal / AWGN Meas PRB is the measurement PRB for PSBCH-RSRP #RBJ to RBJ+10 |
| 9.1.1.3 NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source | Sidelink signal transmit timing relative to downlink timing: ±192 Tc for DL SCS = 15kHz and SL SCS = 30kHz ±160 Tc for DL SCS = 30kHz and SL SCS = 30kHz Average Noc: ±1.5 dB Average Ês1 / Noc: ±0.3 dB Meas PRB Noc: ±1.5 dB Meas PRB Ês1 / Noc: ±0.3 dB | Tc is the basic timing unit defined in TS 38.211 [7], Tc = 1/(480000 x 4096) seconds Noc is the AWGN absolute power on Uu carrier. Ês1 / Noc is the ratio of Cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 9.1.2.1 NR SA FR1 initiation/cease of S-SSB transmission for FR1 NR cell as synchronization reference source | Average Noc: ±1.5 dB Average Ês1 / Noc: ±0.3 dB Meas PRB Noc: ±1.5 dB Meas PRB Ês1 / Noc: ±0.3 dB | Noc is the AWGN absolute power on Uu carrier. Ês1 / Noc is the ratio of Cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 9.1.2.2 NR SA FR1 initiation/cease of S-SSB transmission for SyncRef UE as synchronization reference source | Average Noc: ±1.5 dB Average Ês1 / Noc: ±0.3 dB Meas PRB Noc: ±1.5 dB Meas PRB Ês1 / Noc: ±0.3 dB | Noc is the AWGN absolute power on PC5 carrier. Ês1 / Noc is the ratio of SyncRef UE 1 signal / AWGN Meas PRB is the measurement PRB for PSBCH-RSRP #RBJ to RBJ+10 |
| 9.1.3.1 NR SA FR1 synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source | Average Noc: ±1.5 dB Average Êsi / Noc: ±0.3 dB, i=1,2,3 Meas PRB Noc: ±1.5 dB Meas PRB Êsi / Noc: ±0.3 dB, i=1,2,3 | Noc is the AWGN absolute power on PC5 carrier. Êsi / Noc is the ratio of SyncRef UE i signal / AWGN, i = 1,2,3 Meas PRB is the measurement PRB for PSBCH-RSRP #RBJ to RBJ+10 |
| 9.1.3.2 NR SA FR1 synchronization reference selection/reselection for FR1 NR Cell configured as the highest priority synchronization reference source | Average Noc: ±1.5 dB Average Êsi / Noc: ±0.3 dB, i=1,2 Meas PRB Noc: ±1.5 dB Meas PRB Êsi / Noc: ±0.3 dB, i=1,2 | Noc is the AWGN absolute power on PC5 carrier. Êsi / Noc is the ratio of SyncRef UE i signal / AWGN, i = 1,2 Meas PRB is the measurement PRB for PSBCH-RSRP #RBJ to RBJ+10 |
| 9.1.4.1 NR SA FR1 L1 SL-RSRP measurement for autonomous resource selection/reselection | Average Noc: ±1.5 dB Average Êsi / Noc: ±0.3 dB, i=0,1,...,49 Meas PRB Noc: ±1.5 dB Meas PRB Êsi / Noc: ±0.3 dB, i=0,1,...,49 | Noc is the AWGN absolute power on PC5 carrier. Êsi / Noc is the ratio of active sidelink UE i signal / AWGN, i = 0,1,...,49 Meas PRB is the measurement PRB for SL-RSRP #RBJ to RBJ+599 |
| 9.1.4.2 NR SA FR1 L1 SL-RSRP measurement for resource pre-emption | Average Noc: ±1.5 dB Average Ês1 / Noc: ±0.3 dB, Meas PRB Noc: ±1.5 dB Meas PRB Ês1 / Noc: ±0.3 dB, | Noc is the AWGN absolute power on PC5 carrier. Ês1 / Noc is the ratio of active sidelink UE 1 signal / AWGN, Meas PRB is the measurement PRB for SL-RSRP #RBJ to RBJ+119 |
| 9.1.4.3 NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation | Average Noc: ±1.5 dB Average Êsi / Noc: ±0.3 dB, i=0,1,...,129 Meas PRB Noc: ±1.5 dB Meas PRB Êsi / Noc: ±0.3 dB, i=0,1,...,129 | Noc is the AWGN absolute power on PC5 carrier. Êsi / Noc is the ratio of active sidelink UE i signal / AWGN, i = 0,1,...,129 Meas PRB is the measurement PRB for SL-RSRP #RBJ to RBJ+119 |
| 9.1.5.1 NR SA FR1 congestion control measurement for concurrent operation | Average Noc: ±1.5 dB Average Êsi / Noc: ±0.3 dB, i=0,1,2,3 Meas PRB Noc: ±1.5 dB Meas PRB Êsi / Noc: ±0.3 dB, i=0,1,2,3 | Noc is the AWGN absolute power on PC5 carrier. Êsi / Noc is the ratio of active sidelink UE i signal / AWGN, i = 0,1,2,3 Meas PRB is the measurement PRB for SL-RSSI #RBJ to RBJ+119 |
| 9.1.5.2 NR SA FR1 congestion control measurement for PC5-only operation | Same as 9.1.5.1 | Same as 9.1.5.1 |
| 9.1.6.1 NR SA FR1 interruption to WAN due to NR sidelink communication | ± 0.6dB (AWGN conditions) | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |

Table F.1.1.2-9: Maximum test system uncertainty for RRM requirements for SA FR1 test cases for RedCap

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 16.1.1.1 NR SA FR1 Cell reselection for 1 Rx UE | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 16.1.1.2 NR SA FR1 Cell reselection for 2 Rx UE | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 16.1.1.3 NR SA FR1-FR1 Cell reselection for 1 Rx UE | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 16.1.1.4 NR SA FR1-FR1 Cell reselection for 2 Rx UE | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 16.1.1.5 NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE | Same as 6.1.1.4 | Same as 6.1.1.4 |
| 16.1.1.6 NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 6.1.1.4 | Same as 6.1.1.4 |
| 16.1.1.7 NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE | Same as 6.1.1.5 | Same as 6.1.1.5 |
| 16.1.1.8 NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 6.1.1.5 | Same as 6.1.1.5 |
| 16.1.2.1 NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1 RX UE | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 16.1.2.2 NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2 RX UE | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 16.1.2.3 NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1 RX UE | Same as 6.1.2.2 | Same as 6.1.2.2 |
| 16.1.2.4 NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2 RX UE | Same as 6.1.2.2 | Same as 6.1.2.2 |
| 16.1.2.5 NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE | Same as 6.1.2.2 | Same as 6.1.2.2 |
| 16.1.2.6 NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 6.1.2.2 | Same as 6.1.2.2 |
| 16.2.1.1 NR SA FR1 CG-SDT for 1 Rx UE | Same as 6.2.1 | Same as 6.2.1 |
| 16.2.1.2 NR SA FR1 CG-SDT for 2 Rx UE | Same as 6.2.1 | Same as 6.2.1 |
| 16.3.1.1 NR SA FR1 handover with known target cell for 1 Rx UE | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 16.3.1.2 NR SA FR1 handover with known target cell for 2 Rx UE | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 16.3.1.3 NR SA FR1 handover with unknown target cell for 1 Rx UE | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 16.3.1.4 NR SA FR1 handover with unknown target cell for 2 Rx UE | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 16.3.1.5 NR SA FR1-FR1 handover with unknown target cell for 1 Rx UE | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 16.3.1.6 NR SA FR1-FR1 handover with unknown target cell for 2 Rx UE | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 16.3.1.7 NR SA FR1 - E-UTRA handover with known target cell for 1 Rx UE | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 16.3.1.8 NR SA FR1 - E-UTRA handover with known target cell for 2 Rx UE | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 16.3.1.9 NR SA FR1 - E-UTRA handover with unknown target cell for 1 Rx UE | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 16.3.1.10 NR SA FR1 - E-UTRA handover with unknown target cell for 2 Rx UE | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 16.3.2.1.1 NR SA FR1 RRC Re-establishment for 1 Rx UE | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 16.3.2.1.2 NR SA FR1 RRC Re-establishment for 2 Rx UE | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 16.3.2.1.3 NR SA FR1-FR1 RRC Re-establishment for 1 Rx UE | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 16.3.2.1.4 NR SA FR1-FR1 RRC Re-establishment for 2 Rx UE | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 16.3.2.1.5 NR SA FR1 RRC Re-establishment for 1 Rx UE without serving cell timing | Same as 6.3.2.1.3 | Same as 6.3.2.1.3 |
| 16.3.2.1.6 NR SA FR1 RRC Re-establishment for 2 Rx UE without serving cell timing | Same as 6.3.2.1.3 | Same as 6.3.2.1.3 |
| 16.3.2.2.1 NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.2 NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.3 NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.4 NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.5 NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.6 NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.7 NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.8 NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.3.1 NR SA FR1-FR1 RRC connection release with redirection for 1 Rx UE | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 16.3.2.3.2 NR SA FR1-FR1 RRC connection release with redirection for 2 Rx UE | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 16.3.2.3.3 NR SA FR1 - E-UTRA RRC connection release with redirection for 1 Rx UE | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 16.3.2.3.4 NR SA FR1 - E-UTRA RRC connection release with redirection for 2 Rx UE | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 16.4.1.1 NR SA FR1 NR UE Transmit Timing accuracy for 1 Rx UE | Same as 6.4.1.1 | Same as 6.4.1.1 |
| 16.4.1.2 NR SA FR1 NR UE Transmit Timing accuracy for 2 Rx UE | Same as 6.4.1.1 | Same as 6.4.1.1 |
| 16.4.3.1 NR SA FR1 timing advance adjustment accuracy for 1 Rx UE | Same as 6.4.3.1 | Same as 6.4.3.1 |
| 16.4.3.2 NR SA FR1 timing advance adjustment accuracy for 2 Rx UE | Same as 6.4.3.1 | Same as 6.4.3.1 |
| 16.5.1.1 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM-RS in non-DRX mode for 1 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.2 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM-RS in non-DRX mode for 2 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.3 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM-RS in non-DRX mode for 1 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.4 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM-RS in non-DRX mode for 2 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.5 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM-RS in DRX mode for 1 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.6 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM-RS in DRX mode for 2 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.7 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM-RS in DRX mode for 1 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.8 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM-RS in DRX mode for 2 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.9 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.10 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.11 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.12 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.13 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.14 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.15 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.1.16 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 16.5.2.1 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 16.5.2.2 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 16.5.2.3 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 16.5.2.4 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 16.5.2.5 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE | Same as 6.5.5.3 | Same as 6.5.5.3 |
| 16.5.2.6 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE | Same as 6.5.5.3 | Same as 6.5.5.3 |
| 16.5.2.7 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE | Same as 6.5.5.3 | Same as 6.5.5.3 |
| 16.5.2.8 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE | Same as 6.5.5.3 | Same as 6.5.5.3 |
| 16.5.3.1.1 NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE | Same as 6.5.6.1.2 | Same as 6.5.6.1.2 |
| 16.5.3.1.2 NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE | Same as 6.5.6.1.2 | Same as 6.5.6.1.2 |
| 16.5.3.2.1 NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE | Same as 6.5.6.2.1 | Same as 6.5.6.2.1 |
| 16.5.3.2.2 NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE | Same as 6.5.6.2.1 | Same as 6.5.6.2.1 |
| 16.5.4.1 NR SA FR1 UE specific CBW change on PCell in non-DRX for 1 Rx UE | Same as 6.5.8.1 | Same as 6.5.8.1 |
| 16.5.4.2 NR SA FR1 UE specific CBW change on PCell in non-DRX for 2 Rx UE | Same as 6.5.8.1 | Same as 6.5.8.1 |
| 16.6.1.1 NR SA FR1 Event triggered reporting tests without gap under non-DRX for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.2 NR SA FR1 Event triggered reporting tests without gap under non-DRX for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.3 NR SA FR1 Event triggered reporting tests without gap under DRX for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.4 NR SA FR1 Event triggered reporting tests without gap under DRX for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.5 NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.6 NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.7 NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.8 NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.9 NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.10 NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.11 NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.12 NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.2.1 NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.2 NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.3 NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.4 NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.5 NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.6 NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.7 NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.8 NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.9 NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.10 NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.11 NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 1 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.12 NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.3.1 NR - E-UTRA event-triggered reporting in non-DRX for 1 Rx UE | Same as 6.6.3.1 | Same as 6.6.3.1 |
| 16.6.3.2 NR - E-UTRA event-triggered reporting in non-DRX for 2 Rx UE | Same as 6.6.3.2 | Same as 6.6.3.2 |
| 16.6.3.3 NR - E-UTRA event-triggered reporting in DRX for 1 Rx UE | Same as 6.6.3.1 | Same as 6.6.3.1 |
| 16.6.3.4 NR - E-UTRA event-triggered reporting in DRX for 2 Rx UE | Same as 6.6.3.2 | Same as 6.6.3.2 |
| 16.6.4.1 NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 16.6.4.2 NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 16.6.4.3 NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 1 Rx UE | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 16.6.4.4 NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 2 Rx UE | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 16.6.4.5 NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE | Same as 6.6.4.3 | Same as 6.6.4.3 |
| 16.6.4.6 NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE | Same as 6.6.4.3 | Same as 6.6.4.3 |
| 16.6.4.7 NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE | Same as 6.6.4.3 | Same as 6.6.4.3 |
| 16.6.4.8 NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE | Same as 6.6.4.3 | Same as 6.6.4.3 |
| 16.6.5.1 NR SA FR1 CGI identification of NR neighbour cell for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.5.2 NR SA FR1 CGI identification of NR neighbour cell for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.5.3 NR SA FR1 - E-UTRA CGI Identification of E-UTRA cell for 1 Rx UE | Noc1 ±1.5 dB Noc2 ±1.5 dB Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on Cell 1 frequency. Noc1 is the AWGN on Cell 2 frequency. Ês1 / Noc1 is the ratio of Cell 1 signal / AWGN. Ês2 / Noc2 is the ratio of Cell 2 signal / AWGN. |
| 16.6.5.4 NR SA FR1 - E-UTRA CGI Identification of E-UTRA cell for 2 Rx UE | Same as 16.6.5.3 | Same as 16.6.5.3 |
| 16.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy for 1 Rx UE | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 16.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE | Same as 6.7.1.1.2 | Same as 6.7.1.1.2 |
| 16.7.1.2.1 NR SA FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 16.7.1.2.2 NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE | Same as 6.7.1.1.2 | Same as 6.7.1.1.2 |
| 16.7.1.3.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for 1 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 16.7.1.3.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy for 1 Rx UE | Same as 6.7.1.2.2 | Same as 6.7.1.2.2 |
| 16.7.1.4.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 16.7.1.4.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy for 2 Rx UE | Same as 6.7.1.2.2 | Same as 6.7.1.2.2 |
| 16.7.2.1 NR SA FR1 SS-RSRQ absolute measurement accuracy for 1 Rx UE | Same as 6.7.2.1 | Same as 6.7.2.1 |
| 16.7.2.2 NR SA FR1 SS-RSRQ absolute measurement accuracy for 2 Rx UE | Same as 6.7.2.1 | Same as 6.7.2.1 |
| 16.7.2.3.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for 1 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 16.7.2.3.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for 1 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 16.7.2.4.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for 2 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 16.7.2.4.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for 2 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 16.7.3.1 NR SA FR1 SS-SINR absolute measurement accuracy for 1 Rx UE | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 16.7.3.2 NR SA FR1 SS-SINR absolute measurement accuracy for 2 Rx UE | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 16.7.3.3.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy for 1 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 16.7.3.3.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy for 1 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 16.7.3.4.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy for 2 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 16.7.3.4.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy for 2 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 16.7.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for 1 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy for 1 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.4.2.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for 2 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.4.2.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy for 2 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.4.3.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for 1 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.4.3.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for 1 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.4.4.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for 2 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.4.4.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for 2 Rx UE | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 16.7.5.1 NR SA FR1 - E-UTRA RSRP absolute measurement accuracy for 1 Rx UE | Same as 6.7.5.1 | Same as 6.7.5.1 |
| 16.7.5.2 NR SA FR1 - E-UTRA RSRP absolute measurement accuracy for 2 Rx UE | Same as 6.7.5.1 | Same as 6.7.5.1 |
| 16.7.6.1 NR SA FR1 - E-UTRA RSRQ absolute measurement accuracy for 1 Rx UE | Same as 6.7.5.1 | Same as 6.7.5.1 |
| 16.7.6.2 NR SA FR1 - E-UTRA RSRQ absolute measurement accuracy for 2 Rx UE | Same as 6.7.5.1 | Same as 6.7.5.1 |

Table F.1.1.2-10: Maximum test system uncertainty for RRM requirements for SA FR2 test cases for RedCap

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 17.1.1.1 NR SA FR2 Cell reselection for 2 Rx UE | Same as 7.1.1.1 | Same as 7.1.1.1 |
| 17.1.1.2 NR SA FR2-FR2 Cell reselection for 2 Rx UE | Same as 7.1.1.1 | Same as 7.1.1.1 |
| 17.1.1.3 NR SA FR2 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 7.1.1.1 | Same as 7.1.1.1 |
| 17.1.1.4 NR SA FR2-FR2 Cell reselection for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE | Same as 7.1.1.1 | Same as 7.1.1.1 |
| 17.3.1.1 NR SA FR2 handover with unknown target cell for 2 Rx | Same as 7.3.1.2 | Same as 7.3.1.2 |
| 17.3.1.2 NR SA FR2-FR2 handover with unknown target cell for 2 Rx UE | Same as 7.3.1.3 | Same as 7.3.1.3 |
| 17.3.2.1.1 NR SA FR2 Intra-frequency RRC Re-establishment in FR2 | Same as 7.3.2.1.1 | Same as 7.3.2.1.1 |
| 17.3.2.1.2 NR SA FR2-FR2 Inter-frequency RRC Re-establishment in FR2 | Same as 7.3.2.1.2 | Same as 7.3.2.1.2 |
| 17.3.2.1.3 NR SA FR2 RRC re-establishment without serving cell timing | Same as 7.3.2.1.3 | Same as 7.3.2.1.3 |
| 17.3.2.3.1 NR SA FR2-FR2 RRC connection release with redirection for 2 Rx UE | Same as 7.3.2.3 | Same as 7.3.2.3 |
| 17.4.1.1 NR SA FR2 NR UE Transmit Timing accuracy for 2 Rx UE | Same as 7.4.1.1 | Same as 7.4.1.1 |
| 17.5.1.1 NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM-RS in non-DRX mode for 2 Rx UE | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.2 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM-RS in non-DRX mode for 2 Rx UE | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.3 NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM-RS in DRX mode for 2 Rx UE | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.4 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM-RS in DRX mode for 2 Rx UE | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.5 NR SA FR2 Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM-RS in non-DRX mode for 2 Rx UE | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.6 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.7 NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.8 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.9 NR SA FR2 radio link monitoring UE scheduling restrictions on FR2 for 2 Rx UE | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.2.1 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE | Same as 7.5.5.1 | Same as 7.5.5.1 |
| 17.5.2.2 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE | Same as 7.5.5.2 | Same as 7.5.5.2 |
| 17.5.2.3 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE | Same as 7.5.5.3 | Same as 7.5.5.3 |
| 17.5.2.4 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE | Same as 7.5.5.4 | Same as 7.5.5.4 |
| 17.5.2.5 NR SA FR2 Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE | Same as 7.5.5.5 | Same as 7.5.5.5 |
| 17.5.3.1.1 NR SA FR2 DCI-based and Timer-based DL active BWP switch for 2 Rx UE | Noc ±5.65dB Ês / Noc ±0.3 dB AWGN and signal flatness: ±3.0 dB | Ês / Noc is the ratio of cell 1 signal / AWGN |
| 17.5.3.2.1 NR SA FR2 RRC-based DL active BWP switch for 2 Rx UE | Same as 17.5.3.1.1 | Same as 17.5.3.1.1 |
| 17.5.4.1.1 NR SA FR2 MAC-CE based active TCI state switch for a known TCI state for 2 Rx UE | Same as 7.5.8.1.1 | Same as 7.5.8.1.1 |
| 17.5.4.2.1 NR SA FR2 RRC-CE based active TCI state switch for a known TCI state for 2 Rx UE | Same as 7.5.8.2.1 | Same as 7.5.8.2.1 |
| 17.5.5.1.1 NR SA FR2 MAC-CE based spatial relation switch associated with known DL-RS for 2 Rx UE | Same as 7.5.9.1.1 | Same as 7.5.9.1.1 |
| 17.5.5.2.1 NR SA FR2 RRC-based spatial relation switch associated with a known DL-RS for 2 Rx UE | Same as 7.5.9.2.1 | Same as 7.5.9.2.1 |
| 17.6.1.1 NR SA FR2 event triggered reporting without gap in non-DRX for 2 Rx UE | Same as 7.6.1.1 | Same as 7.6.1.1 |
| 17.6.1.2 NR SA FR2 event triggered reporting without gap in DRX for 2 Rx UE | Same as 7.6.1.2 | Same as 7.6.1.2 |
| 17.6.1.3 NR SA FR2 event triggered reporting with gaps in non-DRX for 2 Rx UE | Same as 7.6.1.1 | Same as 7.6.1.1 |
| 17.6.1.4 NR SA FR2 event triggered reporting with gaps in DRX for 2 Rx UE | Same as 7.6.1.2 | Same as 7.6.1.2 |
| 17.6.2.1 NR SA FR2-FR2 event triggered reporting in non-DRX for 2 Rx UE | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 17.6.2.2 NR SA FR2-FR2 event triggered reporting in DRX for 2 Rx UE | Same as 7.6.2.2 | Same as 7.6.2.2 |
| 17.6.2.3 NR SA FR2-FR2 event triggered reporting with SSB time index detection in non-DRX for 2 Rx UE | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 17.6.2.4 NR SA FR2-FR2 event triggered reporting with SSB time index detection in DRX for 2 Rx UE | Same as 7.6.2.2 | Same as 7.6.2.2 |
| 17.6.3.1 NR SA FR2 SSB based L1-RSRP measurement in non-DRX for 2 Rx UE | Same as 7.6.3.1 | Same as 7.6.3.1 |
| 17.6.3.2 NR SA FR2 SSB based L1-RSRP measurement in DRX for 2 Rx UE | Same as 7.6.3.1 | Same as 7.6.3.1 |
| 17.6.3.3 NR SA FR2 CSI-RS based L1-RSRP measurement in non-DRX for 2 Rx UE | Same as 7.6.3.3 | Same as 7.6.3.3 |
| 17.6.3.4 NR SA FR2 CSI-RS based L1-RSRP measurement in DRX for 2 Rx UE | Same as 7.6.3.3 | Same as 7.6.3.3 |
| 17.7.1.1 NR SA FR2 SS-RSRP measurement accuracy for 2 Rx UE | Same as 7.7.1.1 | Same as 7.7.1.1 |
| 17.7.1.2 NR SA FR2-FR2 SS-RSRP measurement accuracy for 2 Rx UE | Same as 7.7.1.2 | Same as 7.7.1.2 |
| 17.7.2.1 NR SA FR2 SS-RSRQ measurement accuracy for 2 Rx UE | Same as 7.7.2.1 | Same as 7.7.2.1 |
| 17.7.2.2 NR SA FR2-FR2 SS-RSRQ measurement accuracy for 2 Rx UE | Same as 7.7.2.2 | Same as 7.7.2.2 |
| 17.7.3.1 NR SA FR2 SSB based L1-RSRP measurement accuracy for 2 Rx UE | Same as 7.7.4.1 | Same as 7.7.4.1 |
| 17.7.3.2 NR SA FR2 CSI-RS based L1-RSRP measurement accuracy for 2 Rx UE | Same as 7.7.4.2 | Same as 7.7.4.2 |
| 17.7.4.1 NR SA FR2 SS-SINR measurement accuracy for 2 Rx UE | Same as 7.7.3.1 | Same as 7.7.3.1 |
| 17.7.4.2 NR SA FR2-FR2 SS-SINR measurement accuracy for 2 Rx UE | Same as 7.7.3.2 | Same as 7.7.3.2 |

Table F.1.1.2-11: Maximum test system uncertainty for RRM requirements for E-UTRA – NR inter-RAT test cases with E-UTRA serving cell for RedCap

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 18.1.1.1 E-UTRA - NR SA FR1 E-UTRA cell re-selection to higher priority NR target Cell for 2 Rx UE | Same as 8.2.1.1 | Same as 8.2.1.1 |
| 18.2.1.1 E-UTRA - NR SA FR1 handover for 2 Rx UE | Same as 8.3.1.1 | Same as 8.3.1.1 |
| 18.2.2.1 E-UTRA - NR SA FR1 RRC connection release with redirection for 2 Rx UE | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 18.3.1.1 E-UTRA - NR SA FR1 event triggered reporting without SSB time index detection in non-DRX | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 18.3.1.2 E-UTRA - NR SA FR1 event triggered reporting without SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 18.3.1.3 E-UTRA - NR SA FR1 event triggered reporting with SSB time index detection in non-DRX | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 18.3.1.4 E-UTRA - NR SA FR1 event triggered reporting with SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 |

Table F.1.1.2-12: Maximum test system uncertainty for RRM requirements for EN-DC FR1 test cases for shared spectrum

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 10.1.1.1.1 EN-DC FR1 4-step RA type contention-based random access for NR PSCell with CCA | Noc ±1.5 dB Ês2 / Noc ±0.3 dB Uplink absolute power measurement ±1.5 dB Uplink relative power measurement ±0.7 dB ±112Tc Uplink signal transmit timing relative to downlink | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 10.1.1.1.2 EN-DC FR1 4-step RA type non-contention based random access for NR PSCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 10.1.1.1.3 EN-DC FR1 2-step RA type contention-based random access for NR PSCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 10.1.1.1.4 EN-DC FR1 2-step RA type non-contention based random access for NR PSCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 10.1.2 EN-DC FR1 Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Average Ês3 / Noc ±0.3 dB Average Ês4 / Noc ±0.3 dB | Noc1 is the AWGN on E-UTRAN freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc is the cell 1 SNR Ês2 / Noc is the cell 2 SNR Ês3 / Noc is the cell 3 SNR Ês4 / Noc is the cell 4 SNR |
| 10.2.1.1 EN-DC FR1 UE Transmit Timing Test with PSCell under DL CCA | Same as 4.4.1.1 | Same as 4.4.1.1 |
| 10.2.2.1 EN-DC FR1 UE Timing Advance Adjustment Accuracy with PSCell under DL CCA | Same as 4.4.3.1 | Same as 4.4.3.1 |
| 10.3.1.2 EN-DC FR1 Radio link monitoring out-of-sync test for PSCell under CCA configured with SSB-based RLM-RS in non-DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 10.3.1.3 EN-DC FR1 Radio link monitoring in-sync test for PSCell under CCAconfigured with SSB-based RLM-RS in non-DRX mode | Same as 4.5.1.2 | Same as 4.5.1.2 |
| 10.3.3.1 EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle | Same as 4.5.3.1 | Same as 4.5.3.1 |
| 10.3.3.2 EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle | Same as 4.5.3.1 | Same as 4.5.3.1 |
| 10.3.3.3 EN-DC FR1 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA | Same as 4.5.3.1 | Same as 4.5.3.1 |
| 10.3.4.1 EN-DC FR1 EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell under CCA configured with SSB-based BFD and LR in non-DRX mode | Same as 4.5.5.1 | Same as 4.5.5.1 |
| 10.3.4.2 EN-DC FR1 EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell under CCA configured with SSB-based BFD and LR in DRX mode | Same as 4.5.5.1 | Same as 4.5.5.1 |
| 10.3.5.1 EN-DC FR1 UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA | 1.5dB | Measurement uncertainty for OCNG noise as defined in Table F.1.1.2-1 |
| 10.3.5.2.1 EN-DC FR1 E-UTRAN – NR PSCell under CCA FR1 DL active BWP switch in non-DRX in synchronous EN-DC | Same as 4.5.6.1.1 | Same as 4.5.6.1.1 |
| 10.3.5.2.2 EN-DC FR1 E-UTRAN – NR PSCell under CCA FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC | Same as 4.5.6.1.1 | Same as 4.5.6.1.1 |
| 10.3.5.3.1 EN-DC FR1 E-UTRAN – NR PSCell under CCA FR1 DL active BWP switch in non-DRX in synchronous EN-DC | Same as 4.5.6.1.1 | Same as 4.5.6.1.1 |
| 10.3.6.1 EN-DC FR1 Addition and Release Delay of known NR PSCell on the carrier under CCA | Same as 4.5.7.1 | Same as 4.5.7.1 |
| 10.4.1.1 EN-DC FR1 Event-triggered reporting tests on PSCC under CCA without gaps under non-DRX | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 10.4.1.4 EN-DC FR1 Event-triggered reporting tests on PSCC under CCA with per-UE gaps under DRX | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 10.4.2.3 EN-DC FR1 event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 10.4.2.4 EN-DC FR1 event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 10.4.2.5 EN-DC FR1 event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 10.4.2.6 EN-DC FR1 event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 10.4.2.7 EN-DC FR1 event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 10.4.2.8 EN-DC FR1 event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 10.4.2.9 EN-DC FR1 event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 10.4.2.10 EN-DC FR1 event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 10.4.3.1 EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is not used | Same as 4.6.4.1 | Same as 4.6.4.1 |
| 10.4.3.2 EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is used | Same as 4.6.4.1 | Same as 4.6.4.1 |
| 10.4.3.3 EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used | Same as 4.6.4.1 | Same as 4.6.4.1 |
| 10.4.3.4 EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used | Same as 4.6.4.1 | Same as 4.6.4.1 |
| 10.4.4.1 EN-DC FR1 E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used | Same as 12.4.2.1 | Same as 12.4.2.1 |
| 10.4.4.2 EN-DC FR1 E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used | Same as 12.4.2.1 | Same as 12.4.2.1 |
| 10.4.4.3 EN-DC FR1 NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used | Same as 12.4.2.1 | Same as 12.4.2.1 |
| 10.4.4.4 EN-DC FR1 NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used | Same as 12.4.2.1 | Same as 12.4.2.1 |
| 10.5.1.1 EN-DC FR1 Intra-frequency measurement accuracy on a CCA serving cell | Same as 4.7.1.1.1 | Same as 4.7.1.1.1 |
| 10.5.1.2 EN-DC FR1 Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell | Same as 4.7.1.2.1 | Same as 4.7.1.2.1 |
| 10.5.2.1 EN-DC FR1 Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell | Same as 4.7.2.1 | Same as 4.7.2.1 |
| 10.5.2.2 EN-DC FR1 Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell | Same as 4.7.2.2.1 | Same as 4.7.2.2.1 |
| 10.5.3.1 EN-DC FR1 Intra-frequency measurement accuracy on PSCC under CCA | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Average Ês3 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês2 / Noc ±0.8 dB Meas PRB Ês3 / Noc ±0.8 dB | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 2 SNR Ês3 / Noc is the cell 3 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 10.5.3.2 EN-DC FR1 Inter-frequency measurement accuracy on PSCC under CCA | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês2 / Noc1 ±0.3 dB Average Ês3 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc1 ±0.8 dB Meas PRB Ês3 / Noc2 ±0.8 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês2 / Noc1 is the cell 2 SNR Ês3 / Noc2 is the cell 3 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 10.5.3.3 EN-DC FR1 Intra-frequency measurement accuracy on SCC under CCA | Same as 10.5.3.1 | Same as 10.5.3.1 |
| 10.5.4.1 EN-DC FR1 SSB based L1-RSRP measurement accuracy with CCA | Same as 4.7.4.1.1 | Same as 4.7.4.1.1 |
| 10.5.5.1 EN-DC FR1 RSSI measurement accuracy on PSCC with CCA | Average Noc ±1.5 dB | Noc is the AWGN on NR freq1 |
| 10.5.5.2 EN-DC FR1 RSSI measurement accuracy on SCC with CCA | Same as 10.5.5.1 | Same as 10.5.5.1 |
| 10.5.5.3 EN-DC FR1 Inter-frequency RSSI measurement accuracy on a carrier with CCA | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 |
| 10.5.6.1 EN-DC FR1 Channel occupancy measurement accuracy on PSCC with CCA | Average Noc ±1.5 dB | Noc is the AWGN on NR freq1 |
| 10.5.6.2 EN-DC FR1 Channel occupancy measurement accuracy on SCC with CCA | Same as 10.5.6.1 | Same as 10.5.6.1 |
| 10.5.6.3 EN-DC FR1 Inter-frequency channel occupancy measurement accuracy on a carrier with CCA | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 |

Table F.1.1.2-13: Maximum test system uncertainty for RRM requirements for SA FR1 test cases for shared spectrum

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 11.1.1.1 NR SA FR1 Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 11.1.1.2 NR SA FR1 Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 11.1.2.1 Cell re-selection to NR with source NR carrier frequency under CCA | Same as 11.1.1.2 | Same as 11.1.1.2 |
| 11.1.3.1 NR SA FR1 Cell re-selection to NR with source NR carrier frequency under CCA | Same as 11.1.1.2 | Same as 11.1.1.2 |
| 11.1.4.1 NR SA FR1 Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 (NR) frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 (E-UTRAN) frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 11.1.4.2 NR SA FR1 Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA | Same as 11.1.4.1 | Same as 11.1.4.1 |
| 11.2.1.1 NR SA FR1 Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 11.2.1.2 NR SA FR1 Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell | Same as 11.2.1.1 | Same as 11.2.1.1 |
| 11.2.1.3 NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 11.2.1.4 NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell | Same as 11.2.1.3 | Same as 11.2.1.3 |
| 11.2.1.5 NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell | Same as 11.2.1.3 | Same as 11.2.1.3 |
| 11.2.1.6 NR SA FR1 Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell | Same as 11.2.1.3 | Same as 11.2.1.3 |
| 11.2.1.7 NR SA FR1 carrier under CCA - E-UTRAN handover with known target cell | Same as 11.1.4.1 | Same as 11.1.4.1 |
| 11.2.1.8 NR SA FR1 carrier under CCA - E-UTRAN handover with unknown target cell | Same as 11.1.4.1 | Same as 11.1.4.1 |
| 11.2.2.1.1 NR SA FR1 Intra-frequency RRC Re-establishment with CCA in FR1 | Same as 11.1.1.1 | Same as 11.1.1.1 |
| 11.2.2.1.2 NR SA FR1 Inter-frequency RRC Re-establishment with CCA in FR1 | Same as 11.1.1.2 | Same as 11.1.1.2 |
| 11.2.2.1.3 NR SA FR1 Intra-frequency RRC Re-establishment with CCA without serving cell timing | Same as 11.1.1.1 | Same as 11.1.1.1 |
| 11.2.2.1.4 NR SA FR1 Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA | Same as 11.1.1.2 | Same as 11.1.1.2 |
| 11.2.2.2.1 NR SA FR1 4-step RA type contention-based random access for NR PCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 11.2.2.2.2 NR SA FR1 4-step RA type non-contention based random access for NR PSCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 11.2.2.2.3 NR SA FR1 2-step RA type contention-based random access for NR PCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 11.2.2.2.4 NR SA FR1 2-step RA type non-contention-based random access for NR PCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 11.2.2.3.1 NR SA FR1 Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA | Same as 11.2.2.3.2 | Same as 11.2.2.3.2 |
| 11.2.2.3.2 NR SA FR1 Redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 11.3.1.1 NR SA FR1 UE Transmit Timing Test with PCell under DL CCA | Same as 6.4.1.1 | Same as 6.4.1.1 |
| 11.3.2.1 NR SA FR1 UE Timing Advance Adjustment Accuracy with PCell under DL CCA | Same as 6.4.3.1 | Same as 6.4.3.1 |
| 11.4.1.2 NR SA FR1 Radio link monitoring out-of-sync test for PCell under CCA configured with SSB-based RLM-RS in non-DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 11.4.1.3 NR SA FR1 Radio link monitoring in-sync test for PCell under CCA configured with SSB-based RLM-RS in non-DRX mode | Same as 6.5.1.2 | Same as 6.5.1.2 |
| 11.4.3.1 NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 11.4.3.2 NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 11.4.3.3 NR SA FR1 SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 11.4.4.1 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell under CCA configured with SSB-based BFD and LR in non-DRX mode | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 11.4.4.2 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell under CCA configured with SSB-based BFD and LR in DRX mode | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 11.4.5.1 NR SA FR1 UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA | 1.5dB | Measurement uncertainty for OCNG noise as defined in Table F.1.1.2-1 |
| 11.4.5.2.1 NR SA FR1 - NR FR1 DL active BWP switch of PCell under CCA with non-DRX | Same as 6.5.6.1.1 | Same as 6.5.6.1.1 |
| 11.4.5.2.2 NR SA FR1 DCI-based DL active BWP switch with non-DRX under CCA | Same as 6.5.6.1.1 | Same as 6.5.6.1.1 |
| 11.4.5.3.1 NR SA FR1 DL active BWP switch with non-DRX | Same as 6.5.6.1.1 | Same as 6.5.6.1.1 |
| 11.5.1.1 SA FR1 Event-triggered reporting tests on PCC without gaps under non-DRX | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 11.5.1.2 SA FR1 Event-triggered reporting tests on PCC without gaps under DRX | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 11.5.2.3 NR SA FR1-FR1 Event triggered reporting tests with CCA without SSB time index detection when DRX is not used | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.4 NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.5 NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.6 NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.7 NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.8 NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.9 NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.10 NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.3.1 NR SA FR1 - E-UTRAN event-triggered reporting in non-DRX | Same as 6.6.3.1 | Same as 6.6.3.1 |
| 11.5.3.2 NR SA FR1 - E-UTRAN event-triggered reporting in DRX | Same as 6.6.3.1 | Same as 6.6.3.1 |
| 11.5.4.1 NR SA FR1 SSB based L1-RSRP measurement when DRX is not used | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 11.5.4.2 NR SA FR1 SSB based L1-RSRP measurement when DRX is used | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 11.5.4.3 NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 11.5.4.4 NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 11.6.1.1 NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 11.6.1.2 NR SA FR1 Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 11.6.2.1 NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA | Same as 6.7.2.1 | Same as 6.7.2.1 |
| 11.6.2.2 NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA | Same as 6.7.2.2.1 | Same as 6.7.2.2.1 |
| 11.6.2.3 NR SA FR1 Intra-frequency measurement accuracy on SCC | Same as 10.5.2.1 | Same as 10.5.2.1 |
| 11.6.2.4 NR SA FR1 Inter-frequency measurement accuracy | FFS | FFS |
| 11.6.3.1 NR SA FR1 Intra-frequency SS-SINR measurement accuracy on a carrier frequency with CCA | Same as 10.5.3.1 | Same as 10.5.3.1 |
| 11.6.3.2 NR SA FR1 Inter-frequency SS-SINR measurement accuracy on a carrier frequency with CCA | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB meas PRB Ês1 / Noc1 ±0.8 dB meas PRB Ês2 / Noc2 ±0.8 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the cell 2 SNR Ês2 / Noc2 is the cell 3 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 11.6.3.3 NR SA FR1 Intra-frequency SS-SINR measurement accuracy on SCC on a carrier frequency with CCA | Same as 10.5.3.1 | Same as 10.5.3.1 |
| 11.6.3.4 NR SA FR1 Intra-frequency SS-SINR measurement accuracy on SCC on a carrier frequency with CCA | FFS | FFS |
| 11.6.4.1 NR SA FR1 SSB based L1-RSRP measurement accuracy with CCA | Same as 10.5.4.1 | Same as 10.5.4.1 |
| 11.6.5.1 NR SA FR1 Intra-frequency RSSI measurement accuracy on PCC with CCA | Average Noc ±1.5 dB | Noc is the AWGN on NR freq1 |
| 11.6.5.2 NR SA FR1 Intra-frequency RSSI measurement accuracy on SCC with CCA | Same as 11.6.5.1 | Same as 11.6.5.1 |
| 11.6.5.3 NR SA FR1 Inter-frequency RSSI measurement accuracy on a carrier with CCA | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 |
| 11.6.6.1 NR SA FR1 Intra-frequency channel occupancy measurement accuracy on PCC with CCA | Average Noc ±1.5 dB | Noc is the AWGN on NR freq1 |
| 11.6.6.2 NR SA FR1 Intra-frequency channel occupancy measurement accuracy on SCC with CCA | Same as 11.6.6.1 | Same as 11.6.6.1 |
| 11.6.6.3 NR SA FR1 Inter-frequency channel occupancy measurement accuracy on a carrier with CCA | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 |

Table F.1.1.2-14: Maximum test system uncertainty for RRM requirements for E-UTRAN – SA FR1 Shared Spectrum test cases

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 12.1.1.1 E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA | Same as 12.4.2.1 | Same as 12.4.2.1 |
| 12.2.1.1 E-UTRAN - NR with CCA handover with known target cell | Same as 12.4.2.1 | Same as 12.4.2.1 |
| 12.4.2.1 E-UTRAN-NR FR1 inter-RAT event triggered reporting without SSB time index detection when DRX is not used under CCA | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 12.4.2.2 E-UTRAN-NR FR1 inter-RAT event triggered reporting without SSB time index detection when DRX is used under CCA | Same as 12.4.2.1 | Same as 12.4.2.1 |
| 12.4.2.3 E-UTRAN-NR FR1 inter-RAT event triggered reporting with SSB time index detection when DRX is not used under CCA | Same as 12.4.2.1 | Same as 12.4.2.1 |
| 12.4.2.4 E-UTRAN-NR FR1 inter-RAT event triggered reporting with SSB time index detection when DRX is used under CCA | Same as 12.4.2.1 | Same as 12.4.2.1 |

Table F.1.1.2-15: Maximum test system uncertainty for RRM requirements for SA FR1 with SCell in Shared Spectrum test cases

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 13.2.2.1 NR SA FR1 SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 13.2.2.2 NR SA FR1 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 13.2.2.3 NR SA FR1 SCell Activation and Deactivation of unknown SCell under CCA | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 13.3.1.1 NR SA FR1 Event-triggered reporting tests on SCC under CCA without gaps under non-DRX | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Average Ês3 / Noc ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês3 / Noc ±0.3 dB | Noc1 is the AWGN on frequency 1 Noc2 is the AWGN on frequency 2 Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN Meas PRB on cell 3 (in frequency 2) are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 13.3.1.2 NR SA FR1 Event-triggered reporting tests on SCC under CCA without gaps under DRX | Same as 13.3.1.1 | Same as 13.3.1.1 |
| 13.3.1.3 NR SA FR1 Event-triggered reporting tests on SCC under CCA with per-UE gaps under non-DRX | Same as 13.3.1.1 | Same as 13.3.1.1 |
| 13.3.1.4 NR SA FR1 Event-triggered reporting tests on SCC under CCA with per-UE gaps under DRX | Same as 13.3.1.1 | Same as 13.3.1.1 |
| 13.3.2.3 NR SA FR1 Event triggered reporting tests on a carrier with CCA without SSB time index detection when DRX is not used | Average Noc1 ±1.5 dB Average Noc3 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês3 / Noc3 ±0.3 dB | Noc1 is the AWGN on frequency 1 Noc3 is the AWGN on frequency 3 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB on cell 3 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 13.3.2.4 NR SA FR1 Event triggered reporting tests on a carrier with CCA without SSB time index detection when DRX is used | Same as 13.3.2.3 | Same as 13.3.2.3 |
| 13.3.2.5 NR SA FR1 Event triggered reporting tests on a carrier with CCA with SSB time index detection when DRX is not used | Same as 13.3.2.3 | Same as 13.3.2.3 |
| 13.3.2.6 NR SA FR1 Event triggered reporting tests on a carrier with CCA with SSB time index detection when DRX is used | Same as 13.3.2.3 | Same as 13.3.2.3 |
| 13.3.3.1 NR SA FR1 SSB based L1-RSRP measurement for SCell under CCA when DRX is not used | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 13.3.3.2 NR SA FR1 SSB based L1-RSRP measurement for SCell under CCA when DRX is used | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 13.4.1.1 NR SA FR1 SS-RSRP measurement accuracy on a carrier with CCA | Same as 10.5.1.1 | Same as 10.5.1.1 |
| 13.4.2.1 NR SA FR1 SS-RSRQ measurement accuracy on a carrier with CCA | Same as 10.5.2.1 | Same as 10.5.2.1 |
| 13.4.3.1 NR SA FR1 SS-SINR measurement accuracy on a carrier with CCA | Same as 10.5.3.1 | Same as 10.5.3.1 |
| 13.4.4.1 NR SA FR1 L1-RSRP measurement accuracy for beam reporting on a carrier with CCA | Same as 10.5.4.1 | Same as 10.5.4.1 |
| 13.4.5.1 NR SA FR1 RSSI measurement accuracy on a carrier with CCA | Same as 10.5.5.1 | Same as 10.5.5.1 |
| 13.4.5.2 NR SA FR1-FR1 RSSI measurement accuracy on a carrier with CCA | Same as 10.5.5.3 | Same as 10.5.5.3 |
| 13.4.6.1 NR SA FR1 channel occupancy measurement accuracy on a carrier with CCA | Same as 10.5.6.1 | Same as 10.5.6.1 |
| 13.4.6.2 NR SA FR1-FR1 channel occupancy measurement accuracy on a carrier with CCA | Same as 10.5.6.3 | Same as 10.5.6.3 |

Table F.1.1.2-16: Maximum test system uncertainty for RRM requirements for SA FR1 Satellite Access test cases

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 14.1.1 NR SA FR1 Cell Reselection for Satellite Access | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 14.1.2 NR SA FR1 Cell Reselection for Satellite Access | Same as 14.1.1 | Same as 14.1.1 |
| 14.1.3 NR SA FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access | Same as 14.1.1 | Same as 14.1.1 |
| 14.1.4 NR SA FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access | Same as 14.1.1 | Same as 14.1.1 |
| 14.1.5 NR SA FR1-FR1 Cell Reselection for Satellite Access | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 14.1.6 NR SA FR1-FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access | Same as 14.1.5 | Same as 14.1.5 |
| 14.1.7 NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access | Same as 14.1.5 | Same as 14.1.5 |
| 14.1.8 NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access | Same as 14.1.5 | Same as 14.1.5 |
| 14.1.9 NR SA FR1-FR1 Cell reselection for UE fulfilling low mobility relaxed measurement criterion for Satellite Access | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 14.1.10 NR SA FR1-FR1 Cell reselection for UE fulfilling not-at-cell edge relaxed measurement criterion for Satellite Access | Same as 14.1.9 | Same as 14.1.9 |
| 14.2.1.1 NR SA FR1 Handover for Satellite Access | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 14.2.1.2 NR SA FR1-FR1 Handover for Satellite Access | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 14.2.1.3 NR SA FR1 Time-based Conditional Handover for NR Satellite Access | Same as 14.2.1.1 | Same as 14.2.1.1 |
| 14.2.1.4 NR SA FR1-FR1 Time-based Conditional Handover for NR Satellite Access | Same as 14.2.1.2 | Same as 14.2.1.2 |
| 14.2.1.5 NR SA FR1 Distance-based Conditional Handover for Satellite Access | Same as 14.2.1.1 | Same as 14.2.1.1 |
| 14.2.1.6 NR SA FR1-FR1 Distance-based Conditional Handover for Satellite Access | Same as 14.2.1.2 | Same as 14.2.1.2 |
| 14.2.2.1.1 NR SA FR1 RRC Re-establishment for Satellite Access | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 14.2.2.1.2 NR SA FR1-FR1 RRC Re-establishment for Satellite Access | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 14.3.1.1 NR SA FR1 UE transmit timing accuracy for Satellite Access | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB ±352Tc Uplink signal transmit timing relative to downlink for satellite access ±88Tc Relative transmit timing accuracy during UE timing adjustment | Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 14.3.2.1 NR SA FR1 timing advance adjustment accuracy for Satellite Access | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB ±88Tc Timing Advance Adjustment accuracy | Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 14.4.1.1 NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with SSB-based RLM-RS in non-DRX mode for Satellite Access | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês / Noc ±0.3 dB Fading profile ±0.7 dB | Noc is the AWGN on cell 1 frequency Ês / Noc is the SNR Meas PRB are the measurement PRB for SSB-based RLM #RBJ to RBJ+19 |
| 14.4.1.2 NR SA FR1 Radio Link Monitoring In-sync for PCell configured with SSB-based RLM-RS in non-DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.1.3 NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with SSB-based RLM-RS in DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.1.4 NR SA FR1 Radio Link Monitoring In-sync for PCell configured with SSB-based RLM-RS in DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.1.5 NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with CSI-RS-based RLM-RS in non-DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.1.6 NR SA FR1 Radio Link Monitoring In-sync for PCell configured with CSI-RS-based RLM-RS in non-DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.1.7 NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with CSI-RS-based RLM-RS in DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.1.8 NR SA FR1 Radio Link Monitoring In-sync for PCell configured with CSI-RS-based RLM-RS in DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.2.1 NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with SSB-based BFD and LR in non-DRX mode for Satellite Access | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Fading profile ±0.7 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 14.4.2.2 NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with SSB-based BFD and LR in DRX mode for Satellite Access | Same as 14.4.2.1 | Same as 14.4.2.1 |
| 14.4.2.3 NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in non-DRX mode for Satellite Access | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Fading profile ±0.7 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |
| 14.4.2.4 NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in DRX mode for Satellite Access | Same as 14.4.2.3 | Same as 14.4.2.3 |
| 14.4.3.1.1 NR SA FR1 DCI-based DL active BWP switch with non-DRX for Satellite Access | Average Ês / Iot ±0.6 dB | Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 14.4.3.2.1 NR SA FR1 RRC-based DL active BWP switch with non-DRX for Satellite Access | Same as 14.4.3.1.1 | Same as 14.4.3.1.1 |
| 14.4.4.1 NR SA FR1 UE specific CBW change on PCell in non-DRX for Satellite Access | Same as 14.4.3.1.1 | Same as 14.4.3.1.1 |
| 14.5.1.1 SA FR1 event triggered reporting tests without gap under non-DRX for NR satellite access | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 14.5.1.2 SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.1.3 SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.1.4 SA FR1 event triggered reporting tests with single measurement gap under non-DRX for satellite access for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.1.5 SA FR1 event triggered reporting tests with FNO concurrent gaps under DRX for satellite access for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.1.6 SA FR1 event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.2.1 NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 14.5.2.2 NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access | Same as 14.5.2.1 | Same as 14.5.2.1 |
| 14.5.2.3 NR SA FR1-FR1 Event-triggered reporting with SSB time index detection when DRX is not used with single gap for satellite access | Same as 14.5.2.1 | Same as 14.5.2.1 |
| 14.5.2.4 NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 14.5.2.6 NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access | Same as 14.5.2.4 | Same as 14.5.2.4 |
| 14.6.3.1 NR SA FR1 SS-SINR Measurement Accuracy for Satellite Access | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB meas PRB Ês1 / Noc ±0.8 dB meas PRB Ês2 / Noc ±0.8 dB | Noc is the AWGN on NR freq1 Ês1 / Noc is the cell 1 SNR Ês2 / Noc is the cell 2 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 14.6.3.2 NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access | Same as 14.6.3.1 | Same as 14.6.3.1 |
| 14.6.4.1 NR SA FR1 SSB based L1-RSRP Measurement Accuracy for Satellite Access | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.8 dB | Noc is the AWGN on NR freq1 Ês1 / Noc is the cell 1 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 14.6.4.2 NR SA FR1 CSI-RS based L1-RSRP Measurement Accuracy for Satellite Access | Same as 14.6.4.1 | Same as 14.6.4.1 |

Table F.1.1.2-17: Maximum test system uncertainty for RRM requirements for SA FR1 Air-to-ground test cases

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 19.2.1.1 Intra-frequency handover from FR1 to FR1; known target cell | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 19.2.1.2 Inter-frequency handover from FR1 to FR1; unknown target cell | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 19.6.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.2.1 NR SA FR1 SS-RSRQ absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.3.1 NR SA FR1 SS-SINR absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |

## F.1.2	Interpretation of measurement results (normative)

See TS 38.521-1 [17] Annex F.2.

## F.1.3	Test Tolerance and Derivation of Test Requirements (informative)

See TS 38.521-1 [17] Annex F.3.

### F.1.3.1	Measurement of test environments

See TS 38.521-1 [17] Annex F.3.1.

### F.1.3.2	Measurement of RRM requirements

Because the relationships between the test system uncertainties and the test tolerances are often complex, it is not always possible to give a simple derivation of the test requirement in this document. The analysis is recorded in 3GPP TR 38 903 [22].

The derivation of the test requirements for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.3.2-1.

The derivation of the test requirements for the NE-DC FR1 test cases in chapter 4 is defined in Table F.1.3.2-1a.

The derivation of the test requirements for the NR/5GC FR1 test cases in chapter 6 is defined in Table F.1.3.2-2.

The derivation of the test requirements for the EN-DC FR2 test cases in chapter 5 is defined in Table F.1.3.2-3.

The derivation of the test requirements for the NR/5GC FR2 test cases in chapter 7 is defined in Table F.1.3.2-4.

The derivation of the test requirements for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases in chapter 8 is defined in Table F.1.3.2-5.

The derivation of the test requirements for the FR1 NR sidelink test cases in chapter 9 is defined in Table F.1.3.2-6.

The derivation of the test requirements for the EN-DC FR1 test cases for shared spectrum in chapter 10 is defined in Table F.1.3.2-10.

The derivation of the test requirements for the NR/5GC FR1 test cases for shared spectrum in chapter 11 is defined in Table F.1.3.2-11.

The derivation of the test requirements for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases for shared spectrum in chapter 12 is defined in Table F.1.3.2-12.

The derivation of the test requirements for the NR/5GC FR1 test cases with SCell in shared spectrum in chapter 13 is defined in Table F.1.3.2-13.

The derivation of the test requirements for SA FR1 Satellite access test cases in chapter 14 is defined in Table F.1.3.2-14.

The derivation of the test requirements for the NR/5GC FR1 test cases for RedCap in chapter 16 is defined in Table F.1.3.2-7.

The derivation of the test requirements for the NR/5GC FR2 test cases for RedCap in chapter 17 is defined in Table F.1.3.2-8.

The derivation of the test requirements for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases for RedCap in chapter 18 is defined in Table F.1.3.2-9.

The derivation of the test requirements for the NR/5GC FR1 test cases for ATG in chapter 19 is defined in Table F.1.3.2-15.

Table F.1.3.2-1: Derivation of test requirements for EN-DC FR1 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 4.3.2.2.1 | Absolute uplink power: Normal conditions ±17dB Relative uplink power step: Normal conditions ±2.5dB Uplink timing: 15kHz SCS Te ±12*64*Tc 30kHz SCS Te ±8*64*Tc | 2.1dB 0.7dB 112Tc 112Tc | Absolute uplink power: Normal conditions ±19.1dB Relative uplink power step: Normal conditions ±3.2dB Uplink timing: 15kHz SCS Te ±880*Tc 30kHz SCS Te ±624*Tc |
| 4.3.2.2.2 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 |
| 4.3.2.2.3 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 |
| 4.3.2.2.4 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 |
| 4.4.1.1 EN-DC FR1 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Max step size Tq: 5.5*64*Tc Min adjust rate Tp: 5.5*64*Tc Max adjust rate: 5.5*64*Tc Ês2 / Noc: +3.00dB Noc = -98 dBm/15 kHz (Config 1,2,3) Test 2 (with DRX): ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Ês2 / Noc: +3.00dB | ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.5*64Tc -3.6*64*Tc +1.1*64*Tc +0.3 dB +1.5 dB ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.3dB | Test 1 (10MHz Ch BW): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Max step size Tq: 6.0*64*Tc Min adjust rate: 1.9*64*Tc Max adjust rate: 6.6*64*Tc Ês2 / Noc: +3.30dB Noc = -98 dBm/15 kHz (Config 1,2,3) +1.5 dB Test 2 (with DRX): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Ês2 / Noc: +3.30dB |
| 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE Timing Advance Adjustment Accuracy for 15kHz SCS = ±256 Tc+ TT UE Timing Advance Adjustment Accuracy for 30kHz SCS = ±256 Tc+ TT | 0 0 0 +/- 88 Tc +/- 88 Tc | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE TAAA for 15kHz SCS = ±344 Tc UE TAAA for 30kHz SCS = ±344 Tc |
| 4.5.1.1 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode | SNR during T1: 1dB T2: -7dB T3: -15dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |
| 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause D.4.1.1 |
| 4.5.1.3 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM-RS in DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.4 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM-RS in DRX mode | Same as 4.5.1.2 | Same as 4.5.1.2 | Same as 4.5.1.2 |
| 4.5.1.5 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM-RS in non-DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.6 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM-RS in non-DRX mode | Same as 4.5.1.2 | Same as 4.5.1.2 | Same as 4.5.1.2 |
| 4.5.1.7 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM-RS in DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.8 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM-RS in DRX mode | Same as 4.5.1.2 | Same as 4.5.1.2 | Same as 4.5.1.2 |
| 4.5.1.9 EN-DC FR1 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM-RS for UE fulfilling relaxed measurement criterion | SNR during T1: 1dB T2: 1dB T3: -15dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 1.8dB T2: 1.8dB T3: -15.8dB |
| 4.5.2.1 EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | SNRs as specified | 0.6dB | Formula: SNR + TT |
| 4.5.2.2 EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | Same as 4.5.2.1 | Same as 4.5.2.1 | Same as 4.5.2.1 |
| 4.5.2.3 EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |
| 4.5.2.4 EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | Same as 4.5.2.3 | Same as 4.5.2.3 | Same as 4.5.2.3 |
| 4.5.2.5 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |
| 4.5.2.6 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | Same as 4.5.2.5 | Same as 4.5.2.5 | Same as 4.5.2.5 |
| 4.5.2.8 | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |
| 4.5.2.9 | Same as 4.5.2.8 | Same as 4.5.2.8 | Same as 4.5.2.8 |
| 4.5.2.10 EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell | During T1: Noc: -104dBm/15kHz Ês2 / Noc: +17dB | During T1: 0dB 0dB | During T1: Noc: -104dBm/15kHz Ês2 / Noc: +17dB |
| 4.5.3.1 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |
| 4.5.3.2 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | Same as 4.5.3.1 | Same as 4.5.3.1 | Same as 4.5.3.1 |
| 4.5.3.3 EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX | Same as 4.5.3.1 | Same as 4.5.3.1 | Same as 4.5.3.1 |
| 4.5.3.4 SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Noc4: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: -infinity Ês4 / Noc4: -infinity During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Noc4: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB Ês4 / Noc4: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Noc4: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB Ês4 / Noc4: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB 0dB | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Noc4: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: -infinity Ês4 / Noc4: -infinity During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Noc4: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB Ês4 / Noc4: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Noc4: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB Ês4 / Noc4: +17dB |
| 4.5.3.5 EN-DC FR1 direct SCell activation at SCell addition of known SCell | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |
| 4.5.3.6 EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB In signalling: A3-offset = -15dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB In signalling: 0 dB | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB In signalling: A3-offset = -15dB |
| 4.5.3.7 EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 640 ms SCell measurement cycle | Same as 4.5.3.6 | Same as 4.5.3.6 | Same as 4.5.3.6 |
| 4.5.3.11 EN-DC FR1 TRS-based SCell Activation of SSB-less SCell in FR1 collocated inter-band in non-DRX | Config 1,2,4,5: Noc2: -104dBm/15kHz Noc3: -120dBm/15kHz Ês2 / Noc2: 19dB Ês3 / Noc3: (35-ΔEPRE) dB Config 3,6: Noc2: -104dBm/15kHz Noc3: -120dBm/15kHz Ês2 / Noc2: 19dB Ês3 / Noc3: (35-ΔEPRE) dB | Config 1,2,4,5: 0dB 0dB -2dB 1dB Config 3,6: 0dB 0dB -1dB +2dB | Config 1,2,4,5: Noc2: -104dBm/15kHz Noc3: -120dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: (36-ΔEPRE) dB Config 3,6: Noc2: -104dBm/15kHz Noc3: -120dBm/15kHz Ês2 / Noc2: +18dB Ês3 / Noc3: (37-ΔEPRE) dB |
| 4.5.3.12 EN-DC FR1 Inter-band SSB-less SCell activation using A-TRS | Test Config 1,2,4,5: During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +10dB Ês3 / Noc3: +22dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +10dB Ês3 / Noc3: +22dB Test Config 3,6: During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês2 / Noc2: +10dB Ês3 / Noc3: +22dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês2 / Noc2: +10dB Ês3 / Noc3: +22dB | Test Config 1,2,4, 5: During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB Test Config 3,6: During T1: 0dB 0dB 0dB -4dB During T2: 0dB 0dB 0dB -4dB | Test Config 1,2,4,5: During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +10dB Ês3 / Noc3: +22dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +10dB Ês3 / Noc3: +22dB Test Config 3,6: During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +10dB Ês3 / Noc3: +18dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc3: +10dB Ês3 / Noc3: +18dB |
| 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay | During T1: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T2: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T3: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T2: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T3: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB |
| 4.5.5.1 EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |
| 4.5.5.2 EN-DC FR1 SSB-based beam failure detection and link recovery in DRX | Same as 4.5.5.1 | Same as 4.5.5.1 | Same as 4.5.5.1 |
| 4.5.5.3 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |
| 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | Same as 4.5.5.3 | Same as 4.5.5.3 | Same as 4.5.5.3 |
| 4.5.5.5 EN-DC FR1 SCell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in section D.4.1.1 q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |
| 4.5.5.6 EN-DC FR1 SCell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | Same as 4.5.5.5 | Same as 4.5.5.5 | Same as 4.5.5.5 |
| 4.5.5.7 EN-DC FR1 PSCell TRP specific SSB-based beam failure detection and link recovery in non-DRX | q0,0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q0,1 SSB SNR during: T1-T5: 5dB q1,0 and q1,1, SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1,0 and q1,1, SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1,0 and q1,1, SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB | q0,0, Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q0,1, Offset during: T1-T5: +0.8dB q1,0 and q1,1, SSB during T1: 0dB -0.2dB q1,0 and q1,1, SSB during T2: 0dB -0.2dB q1,0 and q1,1, SSB during T3, T4 and T5: 0dB +0.2dB | q0,0, SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q0,1, SNR during: T1-T5: +5.8dB q1,0 and q1,1, SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1,0 and q1,1, SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1,0 and q1,1, SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |
| 4.5.5.8 EN-DC FR1 SCell TRP specific CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | q0,0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q0,1 CSI-RS SNR during: T1-T5: 5dB q1,0 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1,0 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1,0 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB q1,1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1,1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1,1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q0,1, Offset during: T1-T5: +0.8dB q1,0 SSB during T1: 0dB -0.2dB q1,0 SSB during T2: 0dB -0.2dB q1,0 SSB during T3, T4 and T5: 0dB +0.2dB q1,1 SSB during T1: 0dB -0.2dB q1,1 SSB during T2: 0dB -0.2dB q1,1 SSB during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in section D.4.1.1 q0,1, SNR during: T1-T5: +5.8dB q1,0 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1,0 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1,0 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB q1,1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1,1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1,1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |
| 4.5.6.1.1 EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB | During T1: 0dB 0dB During T2: 0dB 0dB During T3: 0dB 0dB | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |
| 4.5.6.1.2 EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB |
| 4.5.6.2.1 EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB | During T1: 0dB 0dB | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |
| 4.5.6.3.1 Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB |
| 4.5.6.4.1 E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB: +17dB |
| 4.5.7.1 EN-DC FR1 addition and release delay of known PSCell | For Test Configurations 1,2,4,5 Noc: -85dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -93 dBm/SCS For Test Configurations 3,6 Noc: -85dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -93 dBm/SCS | 0dB 0dB -3dB -4dB 0dB -3dB | Noc: -85dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS Noc: -89dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS |  |
| 4.5.8.1 EN-DC FR1 interruptions at switching between two uplink carriers | Noc: -104dBm/15kHz Ês / Noc: 14dB | 0dB 0dB | Noc: -104dBm/15kHz Ês / Noc: 14dB |  |
| 4.5.9.1 UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC | Noc: -104 dBm/15kHz Ês / Noc: 17 dB | 0dB 0dB | Noc: -104 dBm/15kHz Ês / Noc: 17 dB |  |
| 4.5.10.1 EN-DC FR1 PSCell activation and deactivation delay | For Test Configurations 1,2,4,5 Noc: -85dBm/15kHz Ês / Noc: 0.0dB For Test Configurations 3,6 Noc: -82dBm/15kHz Ês / Noc: 0.0dB | -3.6dB 0dB -3.6dB 0dB | Noc: -88.6dBm/15kHz Ês / Noc: 0.0dB Noc: -85.6dBm/15kHz Ês / Noc: 0.0dB |  |
| 4.5.11.1 EN-DC FR1 conditional PSCell addition delay | For Test Configurations 1,2,4,5 Noc: -85 dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS For Test Configurations 3,6 Noc: -85 dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -93 dBm/SCS | 0dB 0dB 0dB -3.6dB 0dB -3dB | Noc: -85 dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS Noc: -88.6dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS |  |
| 4.6.1.1 EN-DC FR1 event-triggered reporting without gap in non-DRX | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |
| 4.6.1.2 EN-DC FR1 event-triggered reporting without gap in DRX | Same as 4.6.1.1 | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.3 EN-DC FR1 event-triggered reporting with gap in non-DRX | Same as 4.6.1.1 | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.4 EN-DC FR1 event-triggered reporting with gap in DRX | Same as 4.6.1.1 | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.5 EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | Same as 4.6.1.1 | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.6 EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | Same as 4.6.1.1 | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.7 EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 4.6.1.1 | Same as 4.6.1.1 | Same as 4.6.1.1 |
| 4.6.1.8 EN-DC FR1 event triggered reporting cell without SSB time index detection in DRX for UE configured with highSpeedMeasCA-SCell-r17 | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Ês2 / Noc: +4.00dB | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |
| 4.6.2.1 EN-DC FR1-FR1 event-triggered reporting in non-DRX | During T1: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB |
| 4.6.2.2 EN-DC FR1-FR1 event-triggered reporting in DRX | Same as 4.6.2.1 | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Same as 4.6.2.1 | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Same as 4.6.2.1 | Same as 4.6.2.1 | Same as 4.6.2.1 |
| 4.6.2.9 EN-DC FR1-FR1 event triggered reporting without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 | Same as 4.6.2.1 | Same as 4.6.2.1 | Same as 4.6.2.1 |  |
| 4.6.4.1 EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SSB#0 SS-RSRP values ± 10dB Reported SSB#1 SS-RSRP values ± 10dB Reported Differential SS-RSRP values ± 3dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB Via mapping | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 SSB#1: RSRP_55 to RSRP_75 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 SSB#1: RSRP_58 to RSRP_78 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 |
| 4.6.4.2 EN-DC FR1 SSB-based L1-RSRP measurement in DRX | Same as 4.6.4.1 | Same as 4.6.4.1 | Same as 4.6.4.1 |
| 4.6.4.3 EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-RSRP values ± 10dB Reported Differential CSI-RSRP values ± 3dB | 0dB 0dB 0.5dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 CSI-RS#1: RSRP_55 to RSRP_75 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 CSI-RS#1: RSRP_58 to RSRP_78 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 |
| 4.6.4.4 EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX | Same as 4.6.4.3 | Same as 4.6.4.3 | Same as 4.6.4.3 |
| 4.6.4.5 EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 4.6.4.1 | Same as 4.6.4.1 | Same as 4.6.4.1 |
| 4.6.5.1 EN-DC FR1 SRS-RSRP measurement with non-DRX | Test Configuration 1: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +4dB Test Configuration 2: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +4dB | Test Config 1: T1: 0dB 0dB T2: 0dB +5.25dB Test Config 2: T1: 0dB 0dB T2: 0dB +4.75dB | Test Configuration 1: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +9.25dB Test Configuration 2: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +8.75dB |  |
| 4.6.5.2 EN-DC FR1 CLI-RSSI measurement with non-DRX | During T1: Noc on CLI RSSI: -116 dBm/15kHz Io on CLI RSSI : -97.43 dBm/ 1.08 MHz During T2: Noc on CLI RSSI: -108dBm/15kHz Io on CLI RSSI : -89.43 dBm/ 1.08 Mhz | During T1: 0dB 0dB During T2: 0dB 0.5dB | During T1: Noc: -116 dBm/15kHz Io on CLI RSSI : -97.43 dBm/ 1.08 MHz During T2: Noc: -116 dBm/15kHz Io on CLI RSSI : -88.93 dBm/ 1.08 MHz |  |
| 4.6.6.1 EN-DC intra-frequency CGI identification of NR FR1 cell with autonomous gaps in synchronous EN-DC | During T1: Noc: -98dBm/15kHz ÊS2 / Noc: +4.00dB ÊS3 / Noc: -infinity During T2: Noc: -98dBm/15kHz ÊS2 / Noc: +4.00dB ÊS3 / Noc: +4.00dB | During T1: 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz ÊS2 / Noc: +4.00dB ÊS3 / Noc: -infinity During T2: Noc: -98dBm/15kHz ÊS2 / Noc: +4.00dB ÊS3 / Noc: +4.00dB |  |
| 4.6.7.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-SINR values ± 5.5dB Reported Differential CSI-SINR values ± 4.5dB | 0dB 0dB 0dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 CSI-RS#1: SINR_41 to SINR_64 CSI-RS#0: DIFFSINR_0 to DIFFSINR_7 |  |
| 4.6.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SS-SINR values ±4.5dB Reported Differential SS-SINR values ±3.5dB | 0dB 0dB 0dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 SSB#1: SINR_43 to SINR_62 SSB#0: DIFFSINR_0 to DIFFSINR_6 |  |
| 4.6.7.3 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-SINR values ±4dB Reported Differential CSI-SINR values ±3dB | 0dB 0.5dB 0dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.50dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 CSI-RS#1: SINR_44 to SINR_61 CSI-RS#0: DIFFSINR_0 to DIFFSINR_5 |  |
| 4.7.1.1.1 EN-DC FR1 SS-RSRP absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |
|  | Test 1: Noc: -106dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported RSRP values: ±6dB | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping | Test 1: Noc: -107.5dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_44 to RSRP_56 Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_60 to RSRP_79 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -0.8dB RSRP_36 to RSRP_48 RSRP_36 to RSRP_48 RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 RSRP_38 to RSRP_50 RSRP_39 to RSRP_51 RSRP_39 to RSRP_51 depending on operating band |
|  | TEST CONFIGURATION 3, 6 |  |  |
|  | Test 1: Noc: -110dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±4.5dB N.A Test 2: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -113dBm111dBm/30kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported RSRP values: ±6dB | Test 1: -0.8dB 0dB +0.4dB Via mapping N/A Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB-1dB 0dB +0.2dB Via mapping | Test 1: Noc: -110.8dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_41 to RSRP_53 N/A Test 2: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 3: Noc: -113dBm112dBm/30kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -0.8dB RSRP_37 38 to RSRP_4950 RSRP_37 38 to RSRP_4950 RSRP_38 39 to RSRP_5051 RSRP_38 39 to RSRP_5051 RSRP_39 40 to RSRP_5152 RSRP_40 41 to RSRP_5253 RSRP_40 41 to RSRP_5253 depending on operating band |
| The derivation of the RSRP values takes into account the uncertainty in Cell 3 SS-RSRP from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |
| 4.7.1.1.2 EN-DC FR1 SS-RSRP relative measurement accuracy | Test 1: Noc: Test configuration 1, 2, 4, 5:-106dBm/15kHz Test configuration 3, 6:-110dBm/30kHzN/A Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported relative SS-RSRP values: ±3dB Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported relative SS-RSRP values: ±3dB Test 3: Noc: -116dBm114dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported relative SS-RSRP values: ±3dB | Test 1: 0 dB 0 dB 0 dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping | Test 1: Noc: -106 dBm/15kHz Noc: -110 dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 3: Noc: -116dBm114dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: 0dB RSRP_x-7 to RSRP_x+1 |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 2 and Cell 3 RSRP from Noc, Ês2 / Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |
| 4.7.1.2.1 EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported RSRP values: ±4.5dB | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_62 to RSRP_81 Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 depending on operating band |
|  | TEST CONFIGURATION 3, 6 |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported RSRP values: ±4.5dB | Test 1: -1.35dB -1.35dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 RSRP_37 to RSRP_50 RSRP_38 to RSRP_51 RSRP_39 to RSRP_51 depending on operating band |
| The derivation of the RSRP values takes into account the uncertainty in Cell 3 SS-RSRP from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3.0dB wider at each end for Test 1, and 4.5 dB wider at each for Test 2. |  |  |  |
| 4.7.1.2.2 EN-DC FR1-FR1 SS-RSRP relative measurement accuracy | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported relative RSRP values: ±4.5dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported relative RSRP values: ±4.5dB | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_x-7 to RSRP_x+7 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_x-31 to RSRP_x-18 |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 2 and Cell 3 RSRP from Noc, Ês2 / Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 1.5dB wider at each end. |  |  |  |
| 4.7.2.1 EN-DC FR1 SS-RSRQ measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |
|  | Test 1: Noc: -85dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -101dBm/15kHz Ês2 / Noc: -2.9dB Ês3 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB Reported RSRQ values: ±3.5dB | Test 1: -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -86.5dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: Noc: -101dBm/15kHz Ês2 / Noc: -2.9dB Ês3 / Noc: -2.9dB RSRQ_46 to RSRQ_60 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB RSRQ_44 to RSRQ_59 |
|  | TEST CONFIGURATION 3, 6 |  |  |
|  | Test 1: Noc: -91dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB Reported RSRQ values: ±3.5dB | Test 1: -1.6dB 0dB 0dB Via mapping Test 2: N/A Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -92.6dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB RSRQ_44 to RSRQ_59 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 3 SS-RSRQ from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the SS-RSRQ values are 1.5dB wider at each end for Test 1, and 0.5 dB wider at each for Tests 2 and 3. |  |  |  |
| 4.7.2.2.1 EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |
|  | TEST CONFIGURATION 3, 6 |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 3 SS-RSRQ from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1.5dB wider at each end. |  |  |  |
| 4.7.2.2.2 EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |
|  | TEST CONFIGURATION 3, 6 |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 and Cell 3 SS-RSRQ from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1dB wider at each end. |  |  |  |
| 4.7.3.1 EN-DC FR1 SS-SINR measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -4dB Ês3 / Noc: -4dB Reported SINR values: ±3.5dB | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -3.5dB Ês3 / Noc: -3.5dB SINR_28 to SINR_45 |
|  | TEST CONFIGURATION 3, 6 |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -4dB Ês3 / Noc: -4dB Reported SINR values: ±3.5dB | Test 1: -0.2dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping | Test 1: Noc: -93.2dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -3.5dB Ês3 / Noc: -3.5dB SINR_28 to SINR_45 |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each end. |  |  |  |
| 4.7.3.2.1 EN-DC FR1-FR1 SS-SINR absolute measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -4dB Ês3 / Noc2: -4dB Reported SINR values: ±3.5dB | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3.2dB Ês3 / Noc2: -3.2dB SINR_32 to SINR_49 |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |
| 4.7.3.2.2 EN-DC FR1-FR1 SS-SINR relative measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported SINR values: ±3.5dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB Reported SINR values: ±3.5dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -4dB Ês3 / Noc2: -4dB Reported SINR values: ±4dB | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB SINR_x-10 to SINR_x+10 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB SINR_x-10 to SINR_x+10 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3.2dB Ês3 / Noc2: -3.2dB SINR_x-11 to SINR_x+11 |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each for Tests 1 and 2 and 0dB wider at each end for Test 3. |  |  |  |
| 4.7.4.1.1 EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±3dB additionally for extreme conditions) Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±4.5dB additionally for extreme conditions) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB RSRP_62 to RSRP_82 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_31 to RSRP_44 RSRP_31 to RSRP_45 RSRP_32 to RSRP_45 RSRP_32 to RSRP_46 RSRP_33 to RSRP_46 RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 depending on operating band |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±3dB additionally for extreme conditions) Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±4.5dB additionally for extreme conditions) | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB RSRP_63 to RSRP_84 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 RSRP_35 to RSRP_48 RSRP_35 to RSRP_49 RSRP_36 to RSRP_49 RSRP_37 to RSRP_50 RSRP_37 to RSRP_51 depending on operating band |
| 4.7.4.1.2 EN-DC FR1 SSB based L1-RSRP relative measurement accuracy |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 |
| 4.7.4.2.1 EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 4.7.4.1.1 | Same as 4.7.4.1.1 | Same as 4.7.4.1.1 |
| 4.7.4.2.2 EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 4.7.4.1.2 | Same as 4.7.4.1.2 | Same as 4.7.4.1.2 |
| 4.7.5.1 EN-DC FR1 SFTD measurement accuracy | Noc1: -104dBm/15kHz Ês1 / Noc1: -3.0dB Noc2: -104dBm/15kHz Ês2 / Noc2: 17.0dB | 0dB 0.3dB 0dB 0dB | Noc1: -104dBm/15kHz Ês1 / Noc1: -2.7dB Noc2: -104dBm/15kHz Ês2 / Noc2: 17.0dB |
| 4.7.6.1 EN-DC FR1 SRS-RSRP measurement accuracy | Test Configuration 1: Test 1: Noc = -106dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±3dB Test 2: Noc = -88dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±6.5dB Test 3: Noc = -114+ΔBG_offset dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±3dB Test Configuration 2: Test 1: Test 1 is not used when configuring SCS 30kHz Test 2: Noc = -91dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±7dB Test 3: Noc = -114+ ΔBG_offset dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±3.5dB | Test Config 1: Test 1: 0dB +0.5dB Via mapping Test 2: 0dB +0.5dB Via mapping Test 3: 0dB +0.75dB for BG_A +0.5dB otherwise Via mapping Test Config 2: Test 1: N/A Test 2: 0dB +0.5dB Via mapping Test 3: 0dB +1.25dB for BG_A +0.5dB otherwise Via mapping | Test Config 1: Test 1: Noc = -106dBm/15kHz Ês / Noc = +1.5dB Reported SRS-RSRP: 30 to 41 Test 2: Noc = -88dBm/15kHz Ês / Noc = +1.5dB Reported SRS-RSRP: 45 to 62 Test 3: Noc = -114+ΔBG_offset dBm/15kHz Ês / Noc = +1.75dB for BG_A Ês / Noc = +1.5dB otherwise Reported SRS-RSRP: BG_A: 23 to 33 BG_C: 23 to 34 BG_D: 24 to 34 BG_E: 24 to 35 Test Config 2: Test 1: N/A Test 2: Noc = -91dBm/15kHz Ês / Noc = +1.5dB Reported SRS-RSRP: 44 to 63 Test 3: Noc = -114+ΔBG_offset dBm/15kHz Ês / Noc = +2.25dB for BG_A Ês / Noc = +1.5dB otherwise Reported SRS-RSRP: BG_A: 26 to 37 BG_C: 26 to 37 BG_D: 26 to 38 BG_E: 27 to 38 |  |
| 4.7.7.1.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±5.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±5.5dB (±1dB additionally for extreme conditions) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_28 to L1-SINR_57 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_27 to L1-SINR_58 |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±5.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±5.5dB (±1dB additionally for extreme conditions) | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_28 to L1-SINR_57 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_27 to L1-SINR_58 |
| 4.7.7.1.2 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy |  |  |  |
| Same as 4.7.7.1.1 | Same as 4.7.7.1.1 | Same as 4.7.7.1.1 | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB L1-SINR_x-9 to L1-SINR_x+9 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB L1-SINR_x-9 to L1-SINR_x+9 |
| Same as 4.7.7.1.1 | Same as 4.7.7.1.1 | Same as 4.7.7.1.1 | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB L1-SINR_x-9 to L1-SINR_x+9 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB L1-SINR_x-9 to L1-SINR_x+9 |
| 4.7.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported SS-SINR values: ±4.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported SS-SINR values: ±4.5dB (±1dB additionally for extreme conditions) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_30 to L1-SINR_55 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_29 to L1-SINR_56 |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_30 to L1-SINR_55 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_29 to L1-SINR_56 |
| 4.7.7.3.1 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_30 to L1-SINR_55 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_29 to L1-SINR_56 |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_30 to L1-SINR_55 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_29 to L1-SINR_56 |
| 4.7.7.3.2 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy |  |  |  |
| Same as 4.7.7.1.2 | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 |
| Same as 4.7.7.1.2 | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 |
| 4.7.8.1 EN-DC FR1 CSI-RSRP absolute measurement accuracy |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |
|  |  | Test 1: Noc: -106dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported RSRP values: ±6dB | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping | Test 1: Noc: -107.5dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_44 to RSRP_56 Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_60 to RSRP_79 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -0.8dB RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 RSRP_37 to RSRP_50 RSRP_38 to RSRP_50 RSRP_38 to RSRP_51 RSRP_39 to RSRP_51 RSRP_39 to RSRP_52 RSRP_40 to RSRP_52 depending on operating band |
|  |  | TEST CONFIGURATION 3, 6 |  |  |
|  |  | Test 2: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -111dBm/30kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported RSRP values: ±6dB | Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping | Test 2: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 3: Noc: -111dBm/30kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -0.8dB RSRP_39 to RSRP_51 RSRP_39 to RSRP_51 RSRP_40 to RSRP_52 RSRP_40 to RSRP_52 RSRP_41 to RSRP_53 RSRP_41 to RSRP_53 RSRP_41 to RSRP_53 RSRP_41 to RSRP_53 depending on operating band |
| The derivation of the RSRP values takes into account the uncertainty in Cell 3 CSI-RSRP from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The CSI-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |  |
| 4.7.8.2 EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |
|  |  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -115dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported RSRP values: ±4.5dB | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_62 to RSRP_81 Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -115dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 depending on operating band |
|  |  | TEST CONFIGURATION 3, 6 |  |  |
|  |  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -115dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported RSRP values: ±4.5dB | Test 1: -1.35dB -1.35dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -115dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 RSRP_37 to RSRP_50 RSRP_38 to RSRP_50 RSRP_38 to RSRP_51 RSRP_39 to RSRP_51 depending on operating band |
| The derivation of the RSRP values takes into account the uncertainty in Cell 3 CSI-RSRP from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The CSI-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3.0dB wider at each end for Test 1, and 4.5 dB wider at each for Test 2. |  |  |  |  |

Table F.1.3.2-1a: Derivation of test requirements for NE-DC FR1 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 4A.1.1.1 NE-DC FR1 addition and release delay of known PSCell | Noc: -104dBm/15kHz Ês / Noc: +17dB B1 Threshold Test Configuration 1, 2, 4, 5: -96 dBm/15kHz Test Configuration 3,6: -93 dBm/15kHz | 0dB 0dB -1dB -4dB | Noc: -104dBm/15kHz Ês / Noc: +17dB Test Configuration 1, 2, 4, 5: -97 dBm/15kHz Test Configuration 3,6: -97 dBm/15kHz |
| 4A.2.1.1 NE-DC FR1 SFTD accuracy | Noc1: Test Configuration 1, 2, 4, 5: -104dBm/15kHz Test Configuration 3,6: -101dBm/15kHz Ês1 / Noc1: -3dB Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB | 0dB 0dB 0.3dB 0dB 0dB | Noc1: Test Configuration 1, 2, 4, 5: -104dBm/15kHz Test Configuration 3,6: -101dBm/15kHz Ês1 / Noc1: -2.7dB Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |

Table F.1.3.2-2: Derivation of test requirements for NR SA FR1 RRM tests

| Test |  | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |
| 6.1.1.2 NR SA FR1-FR1 cell re-selection |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -2dB 1.6dB 0.4dB During T2: 0dB 0dB 1.6dB 0dB During T3: 0dB 0dB 1.6dB 1.6dB | During T1: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB |
| 6.1.1.3 NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB In Signalling SSearchDeltaP = 3dB | During T1: 0dB -0.45dB 0dB During T2: 0dB 0dB -0.45dB In Signalling 3dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +12.55dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +12.55dB In Signalling SSearchDeltaP = 6dB |
| 6.1.1.4 NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB -0.45dB 0dB During T2: 0dB 0dB -0.45dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +12.55dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +12.55dB |
| 6.1.1.5 NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 48dB Thresh_X_LowP of Cell 1 = 50dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 44dB SSearchDeltaP of Cell 1 = 3dB SSearchDeltaP of Cell 2 = 3dB | During T1: 0dB 0dB 0dB 0.3dB During T2: 0dB 0dB 0dB -2.25dB In signalling: 0dB -4dB -2dB 0dB 0dB 2dB 0dB 12dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -3.7dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc1: -98dBm/15kHz Ês2 / Noc2: +9.75dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 44dB Thresh_X_LowP of Cell 1 = 48dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 46dB SSearchDeltaP of Cell 1 = 3dB SSearchDeltaP of Cell 2 = 15dB |
| 6.1.1.6 NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB In signalling: SSearchThresholdP of Cell 1 = 50dB SSearchThresholdP of Cell 2 = 50dB | During T1: 0dB 2dB -2dB 0.35dB During T2: 0dB 0dB -2dB 4dB In signalling: 0dB -16dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +16dB Noc2: -100dBm/15kHz Ês2 / Noc2: -3.65dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc1: -100dBm/15kHz Ês2 / Noc2: +16dB In signalling: SSearchThresholdP of Cell 1 = 50dB SSearchThresholdP of Cell 2 = 34dB |
| 6.1.1.7 NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |
| 6.1.1.8 NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17 |  | Same as 6.1.1.2 | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 6.1.1.9 NR SA FR1 cell reselection to intra-frequency NR case for UE operating on a cell with less than 5MHz BW |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |
| 6.1.2.1 NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 1.6dB 1.6dB During T3: 0dB -2dB 1.6dB 0.4dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB During T3: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB |
| 6.1.2.2 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |
| 6.1.2.3 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |
| 6.1.2.4 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |
| 6.1.2.5 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 |  | Same as 6.1.2.2 | Same as 6.1.2.2 | Same as 6.1.2.2 |
| 6.2.1 NR SA FR1 TA Validation for CG-SDT |  | During T1: Noc = -100 dBm/15kHz Es/Noc = 0 dB During T2: Noc = -100 dBm/15kHz Es/Noc = 12dB During T3: Noc = -100 dBm/15kHz Es/Noc = 0dB During T4: Noc = -100 dBm/15kHz Es/Noc = 14dB During T5: Noc = -100 dBm/15kHz Es/Noc = 14dB During T6: Noc = -100 dBm/15kHz Es/Noc = 0dB In Signalling cg-SDT-RSRP-ChangeThreshold = 8dB cg-SDT-RSRP-ThresholdSSB = -110 dBm/SCS | During T1: 0 dB 0 dB During T2: 0 dB 0 dB During T3: 0 dB 0 dB During T4: 0 dB 0 dB During T5: 0 dB 0 dB During T6: 0 dB 0.25 dB for Config 1,2,4 0 dB for Config 3 In Signalling 0 dB 0 dB | During T1: Noc = -100 dBm/15kHz Es/Noc = 0 dB During T2: Noc = -100 dBm/15kHz Es/Noc = 12 dB During T3: Noc = -100 dBm/15kHz Es/Noc = 0 dB During T4: Noc = -100 dBm/15kHz Es/Noc = 14 dB During T5: Noc = -100 dBm/15kHz Es/Noc = 14 dB During T6: Noc = -100 dBm/15kHz Es/Noc = 0.25 dB for Config 1,2,4 Es/Noc = 0 dB for Config 3 In Signalling cg-SDT-RSRP-ChangeThreshold = 8dB cg-SDT-RSRP-ThresholdSSB = -110 dBm/SCS |
| 6.3.1.1 NR SA FR1 handover with known target cell |  | TEST CONFIGURATION 1, 2 A3-Offset: 0dB | -1dB | A3-Offset: -1dB |
|  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB -1dB During T3: 0dB 0dB 0dB -1dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |
|  |  | TEST CONFIGURATION 3 A3-Offset: 0dB | -1dB | A3-Offset: -1dB |
|  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB -1dB During T3: 0dB 0dB 0dB -1dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |
| 6.3.1.2 NR SA FR1 handover with unknown target cell |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB |
| 6.3.1.3 NR SA FR1-FR1 Handover with unknown Target Cell |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB |
| 6.3.1.4 NR SA FR1 – E-UTRA handover with known target cell |  | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: -infinity During T2: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +8dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +8dB | During T1: 0dB 0dB 1.55dB 0dB During T2: 0dB 0dB -1.55dB 1.55dB During T3: 0dB 0dB -1.55dB 1.55dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.55dB Ês2 / Noc2: -infinity During T2: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -5.55dB Ês2 / Noc2: +9.55dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -5.55dB Ês2 / Noc2: +9.55dB |
| 6.3.1.5 NR SA FR1 – E-UTRA handover with unknown target cell |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +7dB |
| 6.3.1.6 NR SA FR1 – UTRAN FDD handover with known target cell |  | During T1: Noc: -100dBm/15kHz Ês / Noc: 12dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -100dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T3: Noc: -100dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB | During T1: -1.54dB 0dB 0dB 0dB 0dB 0dB During T2: -1.54dB 0dB 0dB 0dB 0dB 0dB During T3: -1.54dB 0dB 0dB 0dB 0dB 0dB | During T1: Noc: -101.54dBm/15kHz Ês / Noc: 12dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -101.54dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T3: Noc: -101.54dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB |
| 6.3.1.7 NR SA FR1 synchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB In signalling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 1dB During T3: 0dB 0dB 1dB During T4: 0dB 0dB 1dB During T5: 0dB 0dB 1dB In signalling: -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signalling: A3-offset: -1dB |
| 6.3.1.8 NR SA FR1 asynchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB In signalling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 1dB During T3: 0dB 0dB 1dB During T4: 0dB 0dB 1dB During T5: 0dB 0dB 1dB In signalling: -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signalling: A3-offset: -1dB |
| 6.3.1.9 NR SA FR1 Intra-band inter-frequency synchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB In signalling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 3.7dB During T3: 0dB 0dB 3.7dB During T4: 0dB 0dB 3.7dB During T5: 0dB 0dB 3.7dB In signalling: -3dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB In signalling: A3-offset: -3dB |
| 6.3.1.10 NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB In signalling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 3.7dB During T3: 0dB 0dB 3.7dB During T4: 0dB 0dB 3.7dB During T5: 0dB 0dB 3.7dB In signalling: -3dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB In signalling: A3-offset: -3dB |
| 6.3.1.11 NR SA FR1 Inter-band inter-frequency synchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB In signalling: A3-offset: -6dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB During T4: 0dB 0dB 0dB During T5: 0dB 0dB 0dB In signalling: -1dB for config 1,2,4,5,9 In signalling: -4dB for config 3,6 In signalling: 2dB for config 7,8 | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB In signalling: -7dB for config 1,2,4,5,9 In signalling: -10dB for config 3,6 In signalling: -4dB for config 7,8 |
| 6.3.1.12 NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB In signalling: A3-offset: -4dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB During T4: 0dB 0dB 0dB During T5: 0dB 0dB 0dB In signalling: -3dB for config 1,2,4,5,9 In signalling: -6dB for config 3,6 In signalling: 0dB for config 7,8 | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB In signalling: -7dB for config 1,2,4,5,9 In signalling: -10dB for config 3,6 In signalling: -4dB for config 7,8 |
| 6.3.1.18 NR SA FR1 Intra-frequency handover for unknown target cell operating with 12 PRB SSB bandwidth |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB |
| 6.3.2.1.1 NR SA FR1 RRC re-establishment |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |
| 6.3.2.1.2 NR SA FR1 - FR1 RRC re-establishment |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB |
| 6.3.2.1.3 NR SA FR1 RRC re-establishment without serving cell timing |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |
| 6.3.2.2.1 |  | Absolute uplink power: Normal conditions ±17dB Relative uplink power step: Normal conditions ±2.5dB Uplink timing: 15kHz SCS Te ±12*64*Tc 30kHz SCS Te ±8*64*Tc | 2.1dB 0.7dB 112Tc 112Tc | Absolute uplink power: Normal conditions ±19.1dB Relative uplink power step: Normal conditions ±3.2dB Uplink timing: 15kHz SCS Te ±880*Tc 30kHz SCS Te ±624*Tc |
| 6.3.2.2.2 |  | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 6.3.2.2.3 |  | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 6.3.2.2.4 |  | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 6.3.2.3.1 NR SA FR1 RRC connection release with redirection |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |
| 6.3.2.3.2 NR SA FR1 – E-UTRA RRC connection release with redirection |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |
| 6.3.2.4.1 PDCCH-order RACH on neighbor cell in FR1 when RACH BW is within active UL BWP |  | Noc: -98dBm/SCS Ês0 / Noc: +8dB Ês1 / Noc: +8dB In signalling: A3-offset: -6dB PRACH Timing: ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS | 0dB 0dB 0dB In signalling: 0dB PRACH Timing: ±112Tc ±112Tc | Noc: -98dBm/SCS Ês0 / Noc: +8dB Ês1 / Noc: +8dB In signalling: A3-offset: -6dB PRACH Timing: ±880 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±624 Tc for 30 KHz SSB SCS,30 kHz UL SCS |
| 6.3.2.4.2 PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM |  | Noc: -98dBm/SCS Ês0 / Noc: +8dB Ês1 / Noc: +4dB In signalling: A3-offset: -6dB PRACH Timing: ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS | 0dB 0dB 0dB In signalling: -5dB PRACH Timing: ±112Tc ±112Tc | Noc: -98dBm/SCS Ês0 / Noc: +8dB Ês1 / Noc: +4dB In signalling: A3-offset: -11dB PRACH Timing: ±880 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±624 Tc for 30 KHz SSB SCS,30 kHz UL SCS |
| 6.3.2.4.3 PDCCH-order RACH on neighbour cell without L1-RSRP measurement in FR1 when RACH BW is within active UL BWP |  | Same as 6.3.2.4.1 | Same as 6.3.2.4.1 | Same as 6.3.2.4.1 |
| 6.3.3.1 NR SA FR1 conditional handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11dB In signalling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB -1dB In signalling: -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signalling: A3-offset: -1dB |
| 6.3.3.2 NR SA FR1-FR1 conditional handover |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.7dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +6.7dB |
| 6.3.3.3 NR-DC SA FR1 conditional handover including target MCG and target SCG |  | Test Config 1, 2: During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: -infinity Ês4 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: +5dB Ês4 / Noc2: +5dB Test Config 3: During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: -infinity Ês4 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: +5dB Ês4 / Noc2: +5dB | Test Config 1, 2: During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB Test Config 3: During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB | Test Config 1,2: During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: -infinity Ês4 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: +5dB Ês4 / Noc2: +5dB Test Config 3: During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: -infinity Ês4 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: +5dB Ês4 / Noc2: +5dB |
| 6.3.3.4 NR-DC SA FR1 conditional handover including target MCG and candidate SCG |  | Test Config 1, 2: During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: -infinity Ês4 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: +5dB Ês4 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: +5dB Ês4 / Noc2: +5dB Test Config 3: During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: -infinity Ês4 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: +5dB Ês4 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: +5dB Ês4 / Noc2: +5dB | Test Config 1, 2: During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB +1.5dB 0dB During T3: 0dB 0dB 0dB 0dB +1.5dB +1.5dB Test Config 3: During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB +1.5dB 0dB During T3: 0dB 0dB 0dB 0dB +1.5dB +1.5dB | Test Config 1,2: During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: -infinity Ês4 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: +6.5dB Ês4 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: +6.5dB Ês4 / Noc2: +6.5dB Test Config 3: During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: -infinity Ês4 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: +6.5dB Ês4 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB Ês3 / Noc1: +6.5dB Ês4 / Noc2: +6.5dB |
| 6.3.3.6 NR SA FR1 Intra-frequency NES triggering conditional handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11dB In signalling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB -1dB In signalling: -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signalling: A3-offset: -1dB |
| 6.3.3.7 NR SA FR1-FR1 Inter-frequency NES-based conditional handover |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB In signalling: A3-offset for CHO: 6dB A3-offset for NES CHO: -4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.7dB In signalling: CHO: 4dB NES CHO: 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +6.7dB In signalling: A3-offset for CHO: 10dB A3-offset for NES CHO: -4dB |
| 6.3.4.1 RACH-based Intra-frequency PCell switch from FR1 to FR1 |  | During T1~T4: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB In signalling: A3-offset = -6dB | During T1~T4: 0dB 0dB 0dB 0dB In signalling: 0 dB | During T1~T4: 0dB 0dB +8dB +8dB In signalling: A3-offset = -6dB |
| 6.3.4.2 RACH-based Inter-frequency PCell switch from FR1 to FR1 |  | During T1~T4: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB In signalling: A3-offset = -6dB | During T1~T4: 0dB 0dB 0dB 0dB In signalling: -1 dB | During T1~T4: 0dB 0dB +8dB +8dB In signalling: A3-offset = -7dB |
| 6.3.4.3 RACH-less Intra-frequency PCell switch from FR1 to FR1 |  | During T1~T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB In signalling: A3-offset: -6dB | During T1~T5: 0dB 0dB 0dB In signalling: 0dB | During T1~T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB In signalling: A3-offset: -6dB |
| 6.3.4.4 NR SA FR1 RACH-less Intra-frequency PCell switch from FR1 to FR1 without L1-RSRP measurement |  | Same as 6.3.4.3 | Same as 6.3.4.3 | Same as 6.3.4.3 |
| 6.4.1.1 EN-DC FR1 UE transmit timing accuracy |  | Test 1 (no DRX): Uplink timing: ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Max step size Tq: 5.5*64*Tc Min adjust rate Tp: 5.5*64*Tc Max adjust rate: 5.5*64*Tc Ês2 / Noc: +3.00dB Noc = -98 dBm/15 kHz (Config 1,2,3) Test 2 (with DRX): ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Ês1 / Noc: +3.00dB | ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.5*64Tc -3.6*64*Tc +1.1*64*Tc +0.3 dB +1.5 dB ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.3dB | Test 1 (10MHz Ch BW): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Max step size Tq: 6.0*64*Tc Min adjust rate: 1.9*64*Tc Max adjust rate: 6.6*64*Tc Ês1 / Noc: +3.30dB Noc = -98 dBm/15 kHz (Config 1,2,3) +1.5 dB Test 2 (with DRX): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Ês1 / Noc: +3.30dB |
| 6.4.3.1 NR SA FR1 timing advance adjustment accuracy |  | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE Timing Advance Adjustment Accuracy for 15kHz SCS = ±256 Tc+ TT UE Timing Advance Adjustment Accuracy for 30kHz SCS = ±256 Tc+ TT | 0 0 0 +/- 88 Tc +/- 88 Tc | Noc = -98 dBm/15 kHz (Config 1, 2) Noc = -95 dBm/15 kHz (Config 3) Êsx / Noc = 3 dB UE TAAA for 15kHz SCS = ±344 Tc UE TAAA for 30kHz SCS = ±344 Tc |
| 6.5.1.1 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM-RS in non-DRX mode |  | SNR during T1: 1dB T2: -7dB T3: -15dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |
| 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM-RS in non-DRX mode |  | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause D.4.1.1 |
| 6.5.1.3 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM-RS in DRX mode |  | Same as 6.5.1.1 | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.4 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM-RS in DRX mode |  | Same as 6.5.1.2 | Same as 6.5.1.2 | Same as 6.5.1.2 |
| 6.5.1.5 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM-RS in non-DRX mode |  | Same as 6.5.1.1 | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.6 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM-RS in non-DRX mode |  | Same as 6.5.1.2 | Same as 6.5.1.2 | Same as 6.5.1.2 |
| 6.5.1.7 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM-RS in DRX mode |  | Same as 6.5.1.1 | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.8 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM-RS in DRX mode |  | Same as 6.5.1.2 | Same as 6.5.1.2 | Same as 6.5.1.2 |
| 6.5.1.9 SA FR1 radio link monitoring out-of-sync Test for PSCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion |  | SNR during T1: 1dB T2: 1dB T3: -15dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 1.8dB T2: 1.8dB T3: -15.8dB |
| 6.5.1.13 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM-RS in DRX mode for UE operating on a cell with less than 5MHz BW |  | SNR during T1: 4dB T2: -4dB T3: -12dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 4.8dB T2: -3.2dB T3: -12.8dB |
| 6.5.1.14 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM-RS in non-DRX mode for UE operating on a cell with less than 5MHz BW |  | SNR during T1: 2dB T2: -6dB T3: -14dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 2.8dB T2: -5.2dB T3: -14.8dB |
| 6.5.1.16 NR SA FR1 Radio Link Monitoring In-sync Test for PCell with 3MHz Channel Bandwidth configured with SSB-based RLM-RS in DRX mode |  | SNR during T1: 4dB T2: -4dB T3: -12dB T4: 0dB T5: 5.5dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB | SNR during T1: 4.8dB T2: -3.2dB T3: -12.8dB T4: -0.8dB T5: 6.3dB |
| 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |
| 6.5.2.2 |  | Same as 6.5.2.1 | Same as 6.5.2.1 | Same as 6.5.2.1 |
| 6.5.3.1 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |
| 6.5.3.2 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle |  | Same as 6.5.3.1 | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5.3.3 NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX |  | Same as 6.5.3.1 | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5.3.4 NR SA FR1 direct SCell activation at SCell addition of known SCell |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |
| 6.5.3.5 NR SA FR1 direct SCell activation at handover with known SCell |  | During T1: Noc1: -98 dBm/15 kHz Noc2: -98 dBm/15 kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB Ês3 / Noc1: +8dB During T2: Noc1: -98 dBm/15 kHz Noc2: -98 dBm/15 kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB Ês3 / Noc1: +8dB During T3: Noc1: -98 dBm/15 kHz Noc2: -98 dBm/15 kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB Ês3 / Noc1: +8dB In signalling: A3-offset = -15dB | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB In signalling: 0dB | During T1: Noc1: -98 dBm/15 kHz Noc2: -98 dBm/15 kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB Ês3 / Noc1: +8dB During T2: Noc1: -98 dBm/15 kHz Noc2: -98 dBm/15 kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB Ês3 / Noc1: +8dB During T3: Noc1: -98 dBm/15 kHz Noc2: -98 dBm/15 kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB Ês3 / Noc1: +8dB In signalling: A3-offset = -15dB |
| 6.5.3.10 NR SA FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB In signalling: A3-offset = -15dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB In signalling: 0 dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB In signalling: A3-offset = -15dB |
| 6.5.3.11 NR SA FR1 fast SCell Activation of known SCell in non-DRX for 640 ms SCell measurement cycle |  | Same as 6.5.3.10 | Same as 6.5.3.10 | Same as 6.5.3.10 |
| 6.5.3.16 NR SA FR1 Inter-band SSB-less SCell Activation based on A-TRS in non-DRX |  | Test Config 1, 2: During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +22dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +22dB Test Config 3: During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +22dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +22dB | Test Config 1, 2: During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB Test Config 3: During T1: 0dB 0dB 0dB -4dB During T2: 0dB 0dB 0dB -4dB | Test Config 1,2: During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +22dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +22dB Test Config 3: During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +18dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +18dB |
| 6.5.5.1 NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX |  | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |
| 6.5.5.2 NR SA FR1 SSB-based beam failure detection and link recovery in DRX |  | Same as 6.5.5.1 | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 6.5.5.3 NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês1 / Noc1: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 CSI-RS during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |
| 6.5.5.4 NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX |  | Same as 6.5.5.3 | Same as 6.5.5.3 | Same as 6.5.5.3 |
| 6.5.5.5 NR SA FR1 SCell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX |  | On the SCell: q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10dB | On the SCell: Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB | On the SCell: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |
| 6.5.5.6 NR SA FR1 SCell CSI-RS-based beam failure detection and SSB-based link recovery in DRX |  | Same as 6.5.5.5 | Same as 6.5.5.5 | Same as 6.5.5.5 |
| 6.5.5.7 NR SA FR1 PCell TRP Specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX |  | q0,0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q0,1 CSI-RS SNR during: T1-T5: 5dB q1,0 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1,0 CSI-RS during T2: Noc2: -98dBm/15kHz Ês1 / Noc1: -10dB q1,0 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês1 / Noc1: +10dB | q0,0 Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q0,1 Offset during: T1-T5: +0.8dB q1,0 CSI-RS during T1: 0dB -0.2dB q1,0 CSI-RS during T2: 0dB -0.2dB q1,0 CSI-RS during T3, T4 and T5: 0dB +0.2dB | q0,0 SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q0,1 SNR during: T1-T5: +5.8dB q1,0 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1,0 CSI-RS during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1,0 CSI-RS during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |
| 6.5.6.1.1 NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |
| 6.5.6.1.2 NR SA FR1 DCI-based DL active BWP switch in non-DRX |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T2: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T3: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB | During T1: 0dB 0dB During T2: 0dB 0dB During T3: 0dB 0dB | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T2: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T3: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |
| 6.5.6.2.1 NR SA FR1 RRC-based DL active BWP switch in non-DRX |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB | During T1: 0dB 0dB | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |
| 6.5.6.3.1 NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA |  | Same as 6.5.6.2.1 | Same as 6.5.6.2.1 | Same as 6.5.6.2.1 |
| 6.5.6.5.1 NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA |  | Same as 6.5.6.2.1 | Same as 6.5.6.2.1 | Same as 6.5.6.2.1 |
| 6.5.7.1 NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA |  | Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +14dB Reported CSI-RSRP values ± 10dB | 0dB 0dB 0dB 0dB Via mapping | Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +14dB CSI-RS#0: RSRP_65 to RSRP_86 |
| 6.5.7.2 NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA |  | Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc: +14dB Ês2 / Noc: +14dB Reported CSI-RSRP values ± 10dB | 0dB 0dB 0dB 0dB Via mapping | Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc: +14dB Ês2 / Noc: +14dB CSI-RS#0: RSRP_65 to RSRP_86 |
| 6.5.7A.1 NR SA FR1 DL interruptions at switching between two uplink carriers in FDD-TDD CA |  | Same as 6.5.7.1 | Same as 6.5.7.1 | Same as 6.5.7.1 |
| 6.5.7A.2 NR SA FR1 DL interruptions at switching between two uplink carriers in TDD-TDD CA |  | Same as 6.5.7.2 | Same as 6.5.7.2 | Same as 6.5.7.2 |
| 6.5.7B.1 NR SA FR1 DL interruptions at switching between two uplink bands in FDD-TDD CA |  | Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +14dB Ês3 / Noc: +14dB Reported CSI-RSRP values ± 10dB | 0dB 0dB 0dB 0dB 0dB 0dB Via mapping | Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +14dB Ês3 / Noc: +14dB CSI-RS#0: RSRP 65 to RSRP_86 |
| 6.5.7B.2 NR SA FR1 DL interruptions at switching between two uplink bands in TDD-TDD CA |  | Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc: +14dB Ês2 / Noc: +14dB Ês3 / Noc: +14dB Reported CSI-RSRP values ± 10dB | 0dB 0dB 0dB 0dB 0dB 0dB Via mapping | Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc: +14dB Ês2 / Noc: +14dB Ês3 / Noc: +14dB CSI-RS#0: RSRP_65 to RSRP_86 |
| 6.5.7C.1 NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA |  | Same as 6.5.7B.1 | Same as 6.5.7B.1 | Same as 6.5.7B.1 |
| 6.5.7C.2 NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA |  | Same as 6.5.7B.2 | Same as 6.5.7B.2 | Same as 6.5.7B.2 |
| 6.5.7D.1 NR SA FR1 DL interruptions at switching across three uplink bands in TDD-TDD CA for single TAG |  | Same as 6.5.7B.1 | Same as 6.5.7B.1 | Same as 6.5.7B.1 |
| 6.5.7D.2 NR SA FR1 DL interruptions at switching across four uplink bands in FDD-TDD CA for single TAG |  | Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Noc4: -104dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +17dB Ês3 / Noc: +14dB Ês3 / Noc: +14dB Reported CSI-RSRP values ± 10dB | 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB Via mapping | Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Noc4: -104dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +17dB Ês3 / Noc: +14dB Ês3 / Noc: +14dB CSI-RS#0: RSRP_65 to RSRP_86 |
| 6.5.7D.3 NR SA FR1 DL interruptions at switching across three uplink bands in FDD-TDD CA for two TAGs |  | Same as 6.5.7B.1 | Same as 6.5.7B.1 | Same as 6.5.7B.1 |
| 6.5.7D.4 NR SA FR1 DL interruptions at switching across four uplink bands in TDD-TDD CA for two TAGs |  | Same as 6.5.7D.2 | Same as 6.5.7D.2 | Same as 6.5.7D.2 |
| 6.5.8.1 UE specific CBW change on PCell in FR1 in non-DRX |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB | During T1: 0dB 0dB | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |
| 6.6.1.1 SA event triggered reporting tests without gap under non-DRX |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |
| 6.6.1.2 SA event triggered reporting tests without gap under DRX |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.3 SA event triggered reporting tests with per-UE gaps under non-DRX |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.4 SA event triggered reporting tests with per-UE gaps under DRX |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.5 SA event triggered reporting tests without gap under non-DRX with SSB index reading |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.6 SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.7 NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.8 NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-SCell-r17 |  | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Ês2 / Noc: +4.00dB | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Ês2 / Noc: +4.00dB |
| 6.6.1.9 NR SA FR1 event triggered reporting with MUSIM gap configured |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.12 NR SA FR1 event triggered reporting with SSB index detection without measurement gap in non-DRX mode for UE operating with 12 PRB SSB bandwidth |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.13 NR SA FR1 event triggered reporting tests without gap in DRX under Cell DTX |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.2.1 NR SA FR1-FR1 event-triggered reporting in non-DRX |  | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB |
| 6.6.2.2 NR SA FR1-FR1 event-triggered reporting in DRX |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.5 NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.6 NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.9 |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.10 |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.11 |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.12 NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.13 NR SA FR1-FR1 event triggered reporting using MUSIM gap with lower and higher priority |  | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB During T3: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T4: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB During T4: 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB During T3: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T4: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB |
| 6.6.2.14 NR SA FR1-FR1 event triggered reporting using MUSIM gap with shorter MGRP |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.15 NR SA FR1 event triggered reporting without SSB index detection in DRX mode for UE operating on a cell with less than 5MHz BW |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.3.1 NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX |  | During T1: Freq 1 Noc: -104dBm/15kHz Freq 2 Noc: -104dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -104dBm/15kHz Freq 2 Noc: -104dBm/15kHz Ês1 / Noc: 0dB Ês2 / Noc: +17.00dB | During T1: 0dB 0dB +1.55dB 0dB During T2: 0dB 0dB -1.55dB +1.55dB | During T1: Freq 1 Noc: -104dBm/15kHz Freq 2 Noc: -104dBm/15kHz Ês1 / Noc: +17.55dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -104+TTdBm/15kHz Freq 2 Noc: -104+TTdBm/15kHz Ês1 / Noc: -1.55dB Ês2 / Noc: +18.55dB |
| 6.6.3.2 NR SA FR1 – E-UTRAN event-triggered reporting in DRX |  | Same as 6.6.3.1 | Same as 6.6.3.1 | Same as 6.6.3.1 |
| 6.6.3.3 NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 |  | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +18.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -2dB Ês2 / Noc: +19.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +18.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -2dB Ês2 / Noc: +19.00dB |
| 6.6.4.1 NR SA FR1 SSB-based L1-RSRP measurement in non-DRX |  | During T1: Noc: -98dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SSB#0 SS-RSRP values ± 10dB Reported SSB#1 SS-RSRP values ± 10dB Reported Differential SS-RSRP values ± 3dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB Via mapping | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 SSB#1: RSRP_55 to RSRP_75 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 SSB#1: RSRP_58 to RSRP_78 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 |
| 6.6.4.2 NR SA FR1 SSB-based L1-RSRP measurement in DRX |  | Same as 6.6.4.1 | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 6.6.4.3 NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  | Noc: -98dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-RSRP values ± 10dB Reported Differential CSI-RSRP values ± 3dB | 0dB 0dB 0.5dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 CSI-RS#1: RSRP_55 to RSRP_75 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 CSI-RS#1: RSRP_58 to RSRP_78 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 |
| 6.6.4.4 NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX |  | Same as 6.6.4.3 | Same as 6.6.4.3 | Same as 6.6.4.3 |
| 6.6.4.5 NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 |  | Same as 6.6.4.1 | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 6.6.4.6 NR SA FR1 Inter-cell SSB based L1-RSRP measurements on PCell in DRX |  | Same as 6.6.4.1 | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 6.6.5.1 NR SA FR1 – UTRAN event-triggered reporting in non-DRX |  | During T1: Noc: -106dBm/15kHz Ês / Noc: 18dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -106dBm/15kHz Ês / Noc: -2dB Ioc: -70dBm/3.84MHz Îor / Ioc: -1.8dB CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB | During T1: Noc: -106dBm/15kHz Ês / Noc: 18dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -106dBm/15kHz Ês / Noc: -2dB Ioc: -70dBm/3.84MHz Îor / Ioc: -1.8dB CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB |
| 6.6.6.1 NR SA FR1 SRS-RSRP measurement with non-DRX |  | Test Configuration 1: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +4dB Test Configuration 2: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +4dB | Test Config 1: T1: 0dB 0dB T2: 0dB +5.25dB Test Config 2: T1: 0dB 0dB T2: 0dB +4.75dB | Test Configuration 1: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +9.25dB Test Configuration 2: T1: Noc = -98dBm/15kHz Ês / Noc = -infinity dB T2: Noc = -98dBm/15kHz Ês / Noc = +8.75dB |
| 6.6.7.1 SA intra-frequency CGI identification of NR neighbor cell in FR1 |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |
| 6.6.7.2 Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA |  | During T1 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: 18dB Es2/Noc2: -infinity During T2 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7dB During T3 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7dB | During T1 0dB 0dB 0dB 0dB During T2 0dB 0dB 0dB 0dB During T3 0dB 0dB 0dB 0dB | During T1 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: 18dB Es2/Noc2: -infinity During T2 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7dB During T3 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7dB |
| 6.6.8.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-SINR values ±5.5dB Reported Differential CSI-SINR values ±4.5dB | 0dB 0dB 0dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 CSI-RS#1: SINR_41 to SINR_64 CSI-RS#0: DIFFSINR_0 to DIFFSINR_7 |
| 6.6.8.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SS-SINR values ±4dB Reported Differential SS-SINR values ±3dB | 0dB 0.5dB 0dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.50dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 SSB#1: SINR_44 to SINR_61 SSB#0: DIFFSINR_0 to DIFFSINR_5 |
| 6.6.8.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-SINR values ±4.5dB Reported Differential CSI-SINR values ±3.5dB | 0dB 0dB 0dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 CSI-RS#1: SINR_43 to SINR_62 CSI-RS#0: DIFFSINR_0 to DIFFSINR_6 |
| 6.6.9.1 NR SA FR1 Idle mode CA/DC measurement for FR1 |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB Reported SS-RSRP and SS-RSRQ | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB Via mapping | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB Configuration 1 and 2: Cell 1: RSRP_54 to RSRP_77 RSRQ_55 to RSRQ_72 Cell 2: RSRP_47 to RSRP_70 RSRQ_51 to RSRQ_67 Configuration 3: Cell 1: RSRP_57 to RSRP_80 RSRQ_55 to RSRQ_72 Cell 2: RSRP_50 to RSRP_73 RSRQ_51 to RSRQ_67 |
| 6.6.15.1 NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: -3dB During T4: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +8dB During T5: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB Thresh_X_HighP 48dB Reported SS-RSRP and SS-RSRQ | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB During T4: 0dB 0dB 0dB 0dB During T5: 0dB 0dB 0dB 0dB +4dB Via Mapping | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: -3dB During T4: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +8dB During T5: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB 52dB Configuration 1 and 2: Cell 1: RSRP_52 to RSRP_75 RSRQ_54 to RSRQ_71 Cell 2: RSRP_39 to RSRP_62 RSRQ_6 to RSRQ_27 Configuration 3: Cell 1: RSRP_55 to RSRP_78 RSRQ_54 to RSRQ_71 Cell 2: RSRP_39 to RSRP_62 RSRQ_6 to RSRQ_27 |
| 6.6.17.1 NR SA FR1 event triggered reporting tests with autonomous activation/deactivation Pre-MG |  | During T1, T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1, T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |
| 6.6.17.2 NR SA FR1 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation |  | Same as 6.6.17.1 | Same as 6.6.17.1 | Same as 6.6.17.1 |
| 6.6.18.1 NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers |  | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Freq 3 Noc3: -98dBm/15kHz Freq 1 Ês1 / Noc1: +4.00dB Freq 2 Ês2 / Noc2: -infinity Freq 3 Ês3 / Noc3: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Freq 3 Noc3: -98dBm/15kHz Freq 1 Ês1 / Noc1: +4.00dB Freq 2 Ês2 / Noc2: +7.00dB Freq 3 Ês3 / Noc3: +7.00dB | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Freq 3 Noc3: -98dBm/15kHz Freq 1 Ês1 / Noc1: +4.00dB Freq 2 Ês2 / Noc2: -infinity Freq 3 Ês3 / Noc3: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Freq 3 Noc3: -98dBm/15kHz Freq 1 Ês1 / Noc1: +4.00dB Freq 2 Ês2 / Noc2: +7.00dB Freq 3 Ês3 / Noc3: +7.00dB |
| 6.6.18.2 NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers |  | Same as 6.6.18.1 | Same as 6.6.18.1 | Same as 6.6.18.1 |
| 6.6.18.3 NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1 |  | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -104dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity Ês3 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -104dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Ês3 / Noc: +17.00dB In signalling: b2-Threshold2EUTRA: -95dB | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB In signalling: -6dB | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -104dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity Ês3 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -104dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Ês3 / Noc: +17.00dB In signalling: b2-Threshold2EUTRA: -101dB |
| 6.6.18.4 NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used |  | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz PRS Ês1 / Noc: -infinity PRS Ês2 / Noc: N/A PRS Ês3 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz PRS Ês1 / Noc: -3.00dB PRS Ês2 / Noc: N/A PRS Ês3 / Noc: -10.00dB In signalling: A3-Offset: -6dB | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB In signalling: -9dB | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz PRS Ês1 / Noc: -infinity PRS Ês2 / Noc: N/A PRS Ês3 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz PRS Ês1 / Noc: -3.00dB PRS Ês2 / Noc: NA PRS Ês3 / Noc: -10.00dB In signalling: A3-Offset: -15dB |
| 6.6.19.1 NR SA FR1 event-triggered reporting tests with NCSG under non-DRX in FR1 |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.19.2 NR SA FR1 event-triggered reporting tests for FR1 with NCSG for inter-frequency measurement |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.19.3 NR SA FR1 NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG |  | Same as 6.6.3.1 | Same as 6.6.3.1 | Same as 6.6.3.1 |
| 6.6.19.4 NR SA FR1 Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX |  | Same as 6.6.1.8 | Same as 6.6.1.8 | Same as 6.6.1.8 |
| 6.6.20.1 UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA |  | TEST CONFIGURATION 1, 2 |  |  |
|  |  | During T1: Noc : -98dBm/15kHz Ês / Noc: infinity During T2: Noc: -98dBm/15kHz Ês / Noc: -2dB | During T1: 0 dB 0 dB During T2: 0 dB 0 dB | During T1: Noc : -98dBm/15kHz Ês / Noc: infinity During T2: Noc: -98dBm/15kHz Ês / Noc: -2dB |
|  |  | TEST CONFIGURATION 3 |  |  |
|  |  | During T1: Noc: -95dBm/30kHz Ês / Noc: infinity During T2: Noc1: -95dBm/30kHz Ês1 / Noc1: -2dB | During T1: 0 dB 0 dB During T2: 0 dB 0 dB | During T1: Noc: -95dBm/30kHz Ês / Noc: infinity During T2: Noc1: -95dBm/30kHz Ês1 / Noc1: -2dB |
| 6.6.21.1 UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA |  | TEST CONFIGURATION 1, 2 |  |  |
|  |  | During T1: Noc : -98 dBm/15kHz Ês / Noc: infinity During T2: Noc: -98 dBm/15kHz Ês / Noc: -3 dB | During T1: 0 dB 0 dB During T2: 0 dB 0 dB | During T1: Noc : -98 dBm/15kHz Ês / Noc: infinity During T2: Noc: -98 dBm/15kHz Ês / Noc: -3 dB |
|  |  | TEST CONFIGURATION 3 |  |  |
|  |  | During T1: Noc: -95 dBm/30kHz Ês / Noc: infinity During T2: Noc1: -95 dBm/30kHz Ês1 / Noc1: -3 dB | During T1: 0 dB 0 dB During T2: 0 dB 0 dB | During T1: Noc: -95 dBm/30kHz Ês / Noc: infinity During T2: Noc1: -95 dBm/30kHz Ês1 / Noc1: -3 dB |
| 6.6.24.1 NR SA FR1 event-triggered reporting without gap with interruptions |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.24.2 NR SA FR1-FR1 event-triggered reporting without gap with interruption with SSB index |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.24.3 NR SA FR1-FR1 event-triggered reporting without gap with interruption |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.24.4 NR SA FR1-FR1 event-triggered reporting without gap without interruption |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.24.5 NR SA FR1 event-triggered reporting without gap without interruption |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.26.1 Intra-frequency SSB based L1-RSRP measurement |  | Test configuration 1, 2 During T1: Noc: -101dBm/15kHz Ês1 / Noc: +3dB Ês2 / Noc: +3dB During T2: Noc: -101dBm/15kHz Ês1 / Noc: +13.5dB Ês2 / Noc: +13.5dB Reported Cell 2 SS-RSRP values ± 10dB Test configuration 3 During T1: Noc: -101dBm/15kHz Ês1 / Noc: +3dB Ês2 / Noc: +3dB During T2: Noc: -101dBm/15kHz Ês1 / Noc: +13.5dB Ês2 / Noc: +13.5dB Reported Cell 2 SS-RSRP values ± 10dB | Test configuration 1,2 During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB Via mapping Test configuration 3 During T1: 0dB 0dB 0dB During T2: -1.2dB 0dB 0dB Via mapping | Test configuration 1,2 During T1: Noc: -101dBm/15kHz Ês1 / Noc: +3dB Ês2 / Noc: +3dB During T2: Noc: -101dBm/15kHz Ês1 / Noc: +13.5dB Ês2 / Noc: +13.5dB Cell 2: RSRP_59 to RSRP_79 Test configuration 3 During T1: Noc: -101dBm/15kHz Ês1 / Noc: +3dB Ês2 / Noc: +3dB During T2: Noc: -102.2dBm/15kHz Ês1 / Noc: +13.5dB Ês2 / Noc: +13.5dB Cell 2: RSRP_61 to RSRP_81 |
| 6.6.27.1 Inter-frequency SSB based L1-RSRP measurement with measurement gap |  | In signalling: A3-Offset: -6dB During T1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: 0dB During T2: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +8dB Reported Cell 2 SS-RSRP values ± 10dB | In signalling: -1dB During T1 : 0dB 0dB 0dB 0dB During T2 : 0dB 0dB 0dB 0dB Via mapping | In signalling: A3-Offset: -7dB During T1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: 0dB During T2: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +8dB Test configuration 1,2 Cell 2: RSRP_60 to RSRP_80 Test configuration 3 Cell 2: RSRP_63 to RSRP_83 |
| 6.6.28.1 Inter-frequency SSB based L1-RSRP measurement without measurement gap |  | In signalling: A3-Offset: -6dB Test configuration 1,2 During T1: Noc1: -101dBm/15kHz Noc2: -101dBm/15kHz Ês1 / Noc1: 3dB Ês2 / Noc2: 3dB During T2: Noc1: -101dBm/15kHz Noc2: -101dBm/15kHz Ês1 / Noc1: +13.5dB Ês2 / Noc2: +13.5dB Reported Cell 2 SS-RSRP values ± 10dB Test configuration 3 During T1: Noc1: -101dBm/15kHz Noc2: -101dBm/15kHz Ês1 / Noc1: 3dB Ês2 / Noc2: 3dB During T2: Noc1: -101dBm/15kHz Noc2: -101dBm/15kHz Ês1 / Noc1: +13.5dB Ês2 / Noc2: +13.5dB Reported Cell 2 SS-RSRP values ± 10dB | In signalling: -1dB Test configuration 1,2 During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB Via mapping Test configuration 3 During T1: 0dB 0dB 0dB 0dB During T2: -1.2dB -1.2dB 0dB 0dB Via mapping | In signalling: A3-Offset: -7dB Test configuration 1,2 During T1: Noc1: -101dBm/15kHz Noc2: -101dBm/15kHz Ês1 / Noc1: 3dB Ês2 / Noc2: 3dB During T2: Noc1: -101dBm/15kHz Noc2: -101dBm/15kHz Ês1 / Noc1: +13.5dB Ês2 / Noc2: +13.5dB Cell 2: RSRP_59 to RSRP_79 Test configuration 3 During T1: Noc1: -101dBm/15kHz Noc2: -101dBm/15kHz Ês1 / Noc1: 3dB Ês2 / Noc2: 3dB During T2: Noc1: -102.2dBm/15kHz Noc2: -102.2dBm/15kHz Ês1 / Noc1: +13.5dB Ês2 / Noc2: +13.5dB Cell 2: RSRP_61 to RSRP_81 |
| 6.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy |  | TEST CONFIGURATION 1, 2, |  |  |
|  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping | Test 1: Noc: -107.5dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_44 to RSRP_56 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_60 to RSRP_79 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0.8dB RSRP_36 to RSRP_48 RSRP_36 to RSRP_48 RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 RSRP_38 to RSRP_50 RSRP_39 to RSRP_51 RSRP_39 to RSRP_51 depending on operating band |
|  |  | TEST CONFIGURATION 3 |  |  |
|  |  | Test 1: Noc: -110dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB N/A Test 2: Noc: -91dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -113dBm111dBm/30kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB | Test 1: -0.8dB 0dB +0.4dB Via mapping N/A Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB-1dB 0dB +0.2dB Via mapping | Test 1: Noc: -110.8dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_41 to RSRP_53 N/A Test 2: Noc: -91dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 3: Noc: -113dBm112dBm/30kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0.8dB RSRP_37 38 to RSRP_4950 RSRP_37 38 to RSRP_4950 RSRP_38 39 to RSRP_5051 RSRP_38 39 to RSRP_5051 RSRP_39 40 to RSRP_5152 RSRP_40 41 to RSRP_5253 RSRP_40 41 to RSRP_5253 depending on operating band |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |  |
| 6.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy |  | Test 1: Noc: Test configuration 1, 2:-106dBm/15kHz Test configuration 3: Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported rel. SS-RSRP values: ±3dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported rel. SS-RSRP values: ±3dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported re. SS-RSRP values: ±3dB | Test 1: 0 dB 0 dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping | Test 1: Noc: -106 dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB RSRP_x-7 to RSRP_x+1 |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |
| 6.7.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_62 to RSRP_81 Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 depending on operating band |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  | Test 1: -1.35dB -1.35dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 RSRP_37 to RSRP_50 RSRP_38 to RSRP_51 RSRP_39 to RSRP_51 depending on operating band |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3.0 dB wider at each end for Test 1, and 4.5 dB wider at each for Test 2. |  |  |  |  |
| 6.7.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported relative RSRP values: ±4.5dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported relative RSRP values: ±4.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_x-7 to RSRP_x+7 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_x-31 to RSRP_x-18 |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 1.5dB wider at each end. |  |  |  |  |
| 6.7.2.1 NR SA FR1 SS-RSRQ measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc: -85dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -101dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  | Test 1: -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -86.5dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: Noc: -101dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_46 to RSRQ_60 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB RSRQ_44 to RSRQ_59 |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc: -91dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  | Test 1: -1.6dB 0dB 0dB Via mapping Test 2: N/A Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -92.6dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB RSRQ_44 to RSRQ_59 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the SS-RSRQ values are 1.5dB wider at each end for Test 1, and 0.5 dB wider at each for Tests 2 and 3. |  |  |  |  |
| 6.7.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1.5dB wider at each end. |  |  |  |  |
| 6.7.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 1 and Cell 2 SS-RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1dB wider at each end. |  |  |  |  |
| 6.7.3.1 NR SA FR1 SS-SINR measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -4dB Ês2 / Noc: -4dB Reported SINR values: ±3.5dB |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB SINR_28 to SINR_45 |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -4dB Ês2 / Noc: -4dB Reported SINR values: ±3.5dB |  | Test 1: -0.2dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping | Test 1: Noc: -93.2dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB SINR_28 to SINR_45 |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each end. |  |  |  |  |
| 6.7.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Reported SINR values: ±3.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB SINR_32 to SINR_49 |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |  |
| 6.7.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported SINR values: ±3.5dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Reported SINR values: ±3.5dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Reported SINR values: ±4dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_x-10 to SINR_x+10 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_x-10 to SINR_x+10 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB SINR_x-11 to SINR_x+11 |
| The derivation of the SINR values takes into account the uncertainty in Cell 1 and Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each for Tests 1 and 2 and 0dB wider at each end for Test 3. |  |  |  |  |
| 6.7.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy |  |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±3dB additionally for extreme conditions) Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±4.5dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: 10dB RSRP_62 to RSRP_82 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_31 to RSRP_44 RSRP_31 to RSRP_45 RSRP_32 to RSRP_45 RSRP_32 to RSRP_46 RSRP_33 to RSRP_46 RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 depending on operating band |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±3dB additionally for extreme conditions) Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±4.5dB additionally for extreme conditions) |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96dBm/15kHz Ês1 / Noc1: 10dB RSRP_63 to RSRP_84 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 RSRP_35 to RSRP_48 RSRP_35 to RSRP_49 RSRP_36 to RSRP_49 RSRP_37 to RSRP_50 RSRP_37 to RSRP_51 depending on operating band |
| 6.7.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy |  |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: 10dB DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96dBm/15kHz Ês1 / Noc1: 10dB DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 |
| 6.7.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 6.7.4.1.1 |  | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 6.7.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.2 |  | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 |
| 6.7.5.1 NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | Test 1: Noc: -91.65dBm/15kHz Ês / Noc: +10.0dB Reported RSRP values: ±8dB (±3dB additionally for extreme conditions) Test 2: Noc: -117dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RSRP values: ±4.5dB (±4.5dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc: -91.65dBm/15kHz Ês / Noc: 10dB RSRP_49 to RSRP_69 Test 2: Noc: -117dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_30 RSRP_18 to RSRP_30 Depending on operating band |
| 6.7.6.1 NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | Test 1: Noc: -83dBm/15kHz Ês / Noc: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc: -104.7dBm/15kHz Ês / Noc: -4.0dB Reported RSRQ values: ±3.5dB (±0.5dB additionally for extreme conditions) Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RSRQ values: ±3.5dB (±0.5dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping Test 3: 0dB 0.8dB Via mapping | Test 1: Noc: -83dBm/15kHz Ês / Noc: -1.75dB RSRQ_4 to RSRQ_16 Test 2: Noc: -104.7dBm/15kHz Ês / Noc: -3.2dB RSRQ_0 to RSRQ_16 Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB RSRQ_0 to RSRQ_16 |
| 6.7.7.1 NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | Test 1: Noc: -88dBm/15kHz Ês / Noc: -1.75dB Reported RS-SINR values: ±3.0dB (±1.0dB additionally for extreme conditions) Test 2: Noc: -108.5dBm/15kHz Ês / Noc: 20.0dB Reported RS-SINR values: ±3.0dB (±1.0dB additionally for extreme conditions) Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RS-SINR values: ±3.5dB (±0.5dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0.8dB Via mapping | Test 1: Noc: -88dBm/15kHz Ês / Noc: -1.75dB SINR_35 to SINR_51 Test 2: Noc: -108.5dBm/15kHz Ês / Noc: 20.0dB SINR_79 to SINR_94 Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB SINR_32 to SINR_49 |
| 6.7.8.1 NR SA FR1 SRS-RSRP measurement accuracy | Test Configuration 1: Test 1: Noc = -106dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±3dB Test 2: Noc = -88dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±6.5dB Test 3: Noc = -114+ΔBG_offset dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±3dB Test Configuration 2: Test 1: Test 1 is not used when configuring SCS 30kHz Test 2: Noc = -91dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±7dB Test 3: Noc = -114+ ΔBG_offset dBm/15kHz Ês / Noc = +1dB Reported SRS-RSRP values: ±3.5dB |  | Test Config 1: Test 1: 0dB +0.5dB Via mapping Test 2: 0dB +0.5dB Via mapping Test 3: 0dB +0.75dB for BG_A +0.5dB otherwise Via mapping Test Config 2: Test 1: N/A Test 2: 0dB +0.5dB Via mapping Test 3: 0dB +1.25dB for BG_A +0.5dB otherwise Via mapping | Test Config 1: Test 1: Noc = -106dBm/15kHz Ês / Noc = +1.5dB Reported SRS-RSRP: 30 to 41 Test 2: Noc = -88dBm/15kHz Ês / Noc = +1.5dB Reported SRS-RSRP: 45 to 62 Test 3: Noc = -114+ΔBG_offset dBm/15kHz Ês / Noc = +1.75dB for BG_A Ês / Noc = +1.5dB otherwise Reported SRS-RSRP: BG_A: 23 to 33 BG_C: 23 to 34 BG_D: 24 to 34 BG_E: 24 to 35 Test Config 2: Test 1: N/A Test 2: Noc = -91dBm/15kHz Ês / Noc = +1.5dB Reported SRS-RSRP: 44 to 63 Test 3: Noc = -114+ΔBG_offset dBm/15kHz Ês / Noc = +2.25dB for BG_A Ês / Noc = +1.5dB otherwise Reported SRS-RSRP: BG_A: 26 to 37 BG_C: 26 to 37 BG_D: 26 to 38 BG_E: 27 to 38 |
| 6.7.9.1.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy |  |  |  |  |
| Test Configuration 1,2,4,5 |  | Same as 4.7.7.1 | Same as 4.7.7.1 | Same as 4.7.7.1 |
| Test Configuration 3,6 |  | Same as 4.7.7.1 | Same as 4.7.7.1 | Same as 4.7.7.1 |
| 6.7.9.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  |  |  |  |
| Test Configuration 1,2,4,5 |  | Same as 4.7.7.2 | Same as 4.7.7.2 | Same as 4.7.7.2 |
| Test Configuration 3,6 |  | Same as 4.7.7.2 | Same as 4.7.7.2 | Same as 4.7.7.2 |
| 6.7.9.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy |  |  |  |  |
| Test Configuration 1,2,4,5 |  | Same as 4.7.7.3 | Same as 4.7.7.3 | Same as 4.7.7.3 |
| Test Configuration 3,6 |  | Same as 4.7.7.3 | Same as 4.7.7.3 | Same as 4.7.7.3 |
| 6.7.17.1.1 Inter-frequency L1-RSRP absolute accuracy requirements for neighbour cell in FR1 |  | Test configuration 1, 2 Test 1 : Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Normal condition : Reported Cell 2 SS-RSRP values ± 10dB Extreme condition : Reported Cell 2 SS-RSRP values ± 13dB Test 2 : Noc1: -109dBm/15kHz + ΔBG_offset Noc2: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13dB Ês2 / Noc2: -3dB Normal condition : Reported Cell 2 SS-RSRP values ± 6.5dB Extreme condition : Reported Cell 2 SS-RSRP values ± 11dB | Test configuration 1, 2 Test 1 : 0dB 0dB 0dB 0dB Via mapping Test 2 : 0dB 0dB 0dB 0.3dB Via mapping | Test configuration 1, 2 Test 1 : Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Normal condition: RSRP_62 to RSRP_82 Extreme condition: RSRP_59 to RSRP_85 Test 2: Noc1: -109dBm/15kHz + ΔBG_offset Noc2: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13dB Ês2 / Noc2: -2.7dB Normal condition: RSRP_30 to RSRP_43 RSRP_31 to RSRP_44 RSRP_31 to RSRP_44 RSRP_32 to RSRP_45 RSRP_32 to RSRP_45 RSRP_33 to RSRP_46 RSRP_33 to RSRP_46 RSRP_34 to RSRP_47 RSRP_37 to RSRP_50 Depending on band groups Extreme condition: RSRP_26 to RSRP_48 RSRP_26 to RSRP_48 RSRP_27 to RSRP_49 RSRP_27 to RSRP_49 RSRP_28 to RSRP_50 RSRP_28 to RSRP_50 RSRP_29 to RSRP_51 RSRP_29 to RSRP_51 RSRP_32 to RSRP_54 Depending on band groups |
|  |  | Test configuration 3 Test 1 : Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Normal condition : Reported Cell 2 SS-RSRP values ± 10dB Extreme condition : Reported Cell 2 SS-RSRP values ± 13dB Test 2 : Noc1: -109dBm/15kHz + ΔBG_offset Noc2: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13dB Ês2 / Noc2: -3dB Normal condition : Reported Cell 2 SS-RSRP values ± 6.5dB Extreme condition : Reported Cell 2 SS-RSRP values ± 11dB | Test configuration 3 Test 1 : -1.35dB -1.35dB 0dB 0dB Via mapping Test 2 : 0dB 0dB 0dB 0.3dB Via mapping | Test configuration 3 Test 1 : Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Normal condition: RSRP_63 to RSRP_84 Extreme condition: RSRP_60 to RSRP_87 Test 2: Noc1: -109dBm/15kHz + ΔBG_offset Noc2: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13dB Ês2 / Noc2: -2.7dB Normal condition: RSRP_33 to RSRP_46 RSRP_34 to RSRP_47 RSRP_34 to RSRP_47 RSRP_35 to RSRP_48 RSRP_35 to RSRP_48 RSRP_36 to RSRP_49 RSRP_36 to RSRP_49 RSRP_37 to RSRP_50 RSRP_40 to RSRP_53 Depending on band groups Extreme condition: RSRP_29 to RSRP_51 RSRP_29 to RSRP_51 RSRP_30 to RSRP_52 RSRP_30 to RSRP_52 RSRP_31 to RSRP_53 RSRP_31 to RSRP_53 RSRP_32 to RSRP_54 RSRP_32 to RSRP_54 RSRP_35 to RSRP_57 Depending on band groups |

Table F.1.3.2-3: Derivation of test requirements for EN-DC FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 5.3.2.2.1 EN-DC FR2 contention based random access | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 5.3.2.2.2 EN-DC FR2 non-contention based random access | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 |
| 5.4.1.1 EN-DC FR2 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB | ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB | Test 1 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB |
| 5.4.1.2 NR UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 | Same as 5.4.1.1 | Same as 5.4.1.1 | Same as 5.4.1.1 |
| 5.4.3.1 EN-DC FR2 UE timing advance adjustment accuracy | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE Timing Advance Adjustment Accuracy for 120kHz SCS = ±32 Tc | 0dB 0dB +/- 40 Tc | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE TAAA for 120kHz SCS = ±72 Tc |
| 5.5.1.1 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB |
| 5.5.1.2 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -4.5 dB During T5: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: +2 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB During T4: -2.7 dB 0 dB 0 dB During T5: -2.7 dB 0 dB 2.1 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -4.5 dB During T5: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: +4.1 dB |
| 5.5.1.3 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM-RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB |
| 5.5.1.4 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM-RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: -14 dB | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB During T4: 0 dB -0.4 dB -0.4 dB During T5: 0 dB 1.3 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.9 dB Ês2 / Noc: -15.4 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: 3.3 dB Ês2 / Noc: -15.4 dB |
| 5.5.1.5 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM-RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB |
| 5.5.1.6 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM-RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -4.5 dB During T5: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: +2 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB During T4: -2.7 dB 0 dB 0 dB During T5: -2.7 dB 0 dB 2.1 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -4.5 dB During T5: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: +4.1 dB |
| 5.5.1.7 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM-RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB |
| 5.5.1.8 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM-RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -104.7dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -104.7dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: -15 dB | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB During T4: 0 dB -0.4 dB -0.4 dB During T5: 0 dB 1.3 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.9 dB Ês2 / Noc: -15.4 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: 3.3 dB Ês2 / Noc: -15.4 dB |
| 5.5.1.9 EN-DC FR2 radio link monitoring UE scheduling restrictions | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB | During T1: -2.7 dB 2.1 dB 2.1 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB |
| 5.5.1.10 EN-DC FR2 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM-RS for UE fulfilling relaxed measurement criterion | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T3: Noc: -104.7dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: 0 dB 1.25 dB 1.25 dB During T2: 0 dB 1.25 dB 1.25 dB During T3: 0 dB -0.5 dB -0.5 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.25 dB Ês2 / Noc: +3.25 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.25 dB Ês2 / Noc: +3.25 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.5 dB Ês2 / Noc: -15.5 dB |
| 5.5.2.3 EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | During T1: Noc1: -111.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc: +12 dB Ês2 / Noc: +5 dB | During T1: 0 dB 0 dB 0 dB 0 dB | During T1: Noc1: -111.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc: +12 dB Ês2 / Noc: +5 dB |
| 5.5.2.3 EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | During T1: Noc1: -111.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc: +12 dB Ês2 / Noc: +5 dB | During T1: 0 dB 0 dB 0 dB 0 dB | During T1: Noc1: -111.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc: +12 dB Ês2 / Noc: +5 dB |
| 5.5.3.1 EN-DC FR2 SCell activation and deactivation intra-band in non-DRX | During T1: Noc2: -104.7dBm/15kHz Noc3: -104.7dBm/15kHz Ês2 / Noc2: +7dB Ês3 / Noc3: +7dB During T2: Noc2: -104.7dBm/15kHz Noc3: -104.7dBm/15kHz Ês2 / Noc2: +7dB Ês3 / Noc3: +7dB During T3: Noc2: -104.7dBm/15kHz Noc3: -104.7dBm/15kHz Ês2 / Noc2: +7dB Ês3 / Noc3: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc2: -104.7dBm/15kHz Noc3: -104.7dBm/15kHz Ês2 / Noc2: +7dB Ês3 / Noc3: +7dB During T2: Noc2: -104.7dBm/15kHz Noc3: -104.7dBm/15kHz Ês2 / Noc2: +7dB Ês3 / Noc3: +7dB During T3: Noc2: -104.7dBm/15kHz Noc3: -104.7dBm/15kHz Ês2 / Noc2: +7dB Ês3 / Noc3: +7dB |
| 5.5.3.6 Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2 | During T1: Noc3: -112dBm/15kHz Noc4: N/A Ês3 / Noc3: +14dB Ês4 / Noc4: NA During T2: Noc3: -112dBm/15kHz Noc4: -112dBm/15kHz Ês3 / Noc3: +14dB Ês4 / Noc4: +14dB During T3: Noc3: -112dBm/15kHz Noc4: -112dBm/15kHz Ês3 / Noc3: +14dB Ês4 / Noc4: +14dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc3: -112dBm/15kHz Noc4: N/A Ês3 / Noc3: +14dB Ês4 / Noc4: NA During T2: Noc3: -112dBm/15kHz Noc4: -112dBm/15kHz Ês3 / Noc3: +14dB Ês4 / Noc4: +14dB During T3: Noc3: -112dBm/15kHz Noc4: -112dBm/15kHz Ês3 / Noc3: +14dB Ês4 / Noc4: +14dB |
| 5.5.3.7 EN-DC FR2 direct SCell activation at SCell addition of known SCell | Same as 5.5.3.1 | Same as 5.5.3.1 | Same as 5.5.3.1 |
| 5.5.3.8 EN-DC FR2 fast SCell Activation for SCell in FR2 intra-band | During T1: Noc2: -104.7 dBm/15kHz Noc3: -104.7 dBm/15kHz Ês2 / Noc2: +7 dB Ês3 / Noc3: +7 dB During T2: Noc2: -104.7 dBm/15kHz Noc3: -104.7 dBm/15kHz Ês2/ Noc2: +7 dB Ês3/ Noc3: +7 dB In signalling: A3-offset = -15dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB In signalling: 0dB | During T1: Noc2: -104.7 dBm/15kHz Noc3: -104.7 dBm/15kHz Ês2 / Noc2: +7 dB Ês3 / Noc3: +7 dB During T2: Noc2: -104.7 dBm/15kHz Noc3: -104.7 dBm/15kHz Ês2/ Noc2: +7 dB Ês3/ Noc3: +7 dB In signalling: A3-offset = -15dB |
| 5.5.5.1 EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX | During T1: SNR_SSB q0: 5 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: -3 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20.2 dB In signalling: rsrp-ThresholdSSB: -95 dBm/120kHz for Config 1,2 rsrp-ThresholdSSB: -92 dBm/240kHz for Config 3,4 | +8.70 dB 0 dB +8.70 dB 0 dB 0 dB -0.2 dB -14 dB -14 dB | During T1: SNR_SSB q0: 13.7 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: 5.7 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20 dB In signalling: rsrp-ThresholdSSB: -109 dBm/120kHz for Config 1,2 srp-ThresholdSSB: -106 dBm/240kHz for Config 3,4 |
| 5.5.5.2 EN-DC FR2 SSB-based beam failure detection and link recovery in DRX | Same as 5.5.5.1 | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.5.3 EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdSSB: -95 dBm/SCS kHz | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdSSB: -109.0 dBm/SCS kHz |
| 5.5.5.4 EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX | Same as 5.5.5.3 | Same as 5.5.5.3 | Same as 5.5.5.3 |
| 5.5.5.5 EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Same as 5.5.5.1 | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.5.6 EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês3 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês3 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdBFR: -95 dBm/SCS kHz | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês3 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês3 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdBFR: -109.0 dBm/SCS kHz |
| 5.5.5.7 EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | Same as 5.5.5.6 | Same as 5.5.5.6 | Same as 5.5.5.6 |
| 5.5.5.8 EN-DC FR2 PSCell CSI-RS-based TRP specific Beam Failure Detection and Link Recovery in DRX mode | Noc = -104.7 dBm/120 kHz Ês3 / Noc for CSI-RS set q0,0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês3 / Noc for CSI-RS set q0,1: T1-T5: 5 dB Ês3 / Noc for CSI-RS set q1,0 and q1,1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdBFR: -95 dBm/SCS kHz | 0 dB q0,0: 8.7 dB 8.7 dB 0 dB 0 dB 0 dB q0,1: 8.7 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz q0,0: Ês3 / Noc for CSI-RS set q0,0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês3 / Noc for CSI-RS set q0,1 T1-T5: 13.7 dB Ês3 / Noc for CSI-RS set q1,0 and q1,1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdBFR: -109.0 dBm/SCS kHz |
| 5.5.5.9 EN-DC FR2 SSB-based beam failure detection and link recovery in DRX mode for UE fulfilling relaxed measurement criterion | During T1: SNR_SSB q0: 6 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: 6 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20.2 dB In signalling: rsrp-ThresholdSSB: -95 dBm/120kHz for Config 1,2 rsrp-ThresholdSSB: -92 dBm/240kHz for Config 3,4 | +8.70 dB 0 dB +8.70 dB 0 dB 0 dB -0.2 dB -14 dB -14 dB | During T1: SNR_SSB q0: 14.70 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: 14.70 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20 dB In signalling: rsrp-ThresholdSSB: -109 dBm/120kHz for Config 1,2 srp-ThresholdSSB: -106 dBm/240kHz for Config 3,4 |
| 5.5.6.2.1 EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | During T1: Noc2: -112dBm/15kHz Ês2 / Iot2: +18dB | During T1: 0dB 0dB | During T1: Noc2: -112dBm/15kHz Ês2 / Iot2: +18dB |
| 5.5.6.3.1 E-UTRAN - NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC | Same as 5.5.6.2.1 | Same as 5.5.6.2.1 | Same as 5.5.6.2.1 |
| 5.5.6.5.1 E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC | During T1: Noc1: -111.7dBm/15kHz Noc2: -111.7dBm/15kHz Ês1 / Iot1: +18dB Ês2 / Iot2: +18dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc1: -111.7dBm/15kHz Noc2: -111.7dBm/15kHz Ês1 / Iot1: +18dB Ês2 / Iot2: +18dB |
| 5.5.7.1 Addition and Release Delay of NR PSCell | During T1: Ês: -infinity During T2: Ês: -81 dBm/120kHz During T3: Ês: -81 dBm/120kHz During T4: Ês: -81 dBm/120kHz | During T1: 0dB During T2: 0dB During T3: 0dB During T4: 0dB | During T1: Ês: -infinity During T2: Ês: -81 dBm/120kHz During T3: Ês: -81 dBm/120kHz During T4: Ês: -81 dBm/120kHz |
| 5.5.9.1.1 EN-DC PSCell FR2 uplink spatial relation switch for a known spatial relation | During T1: Ês / Noc: 1 dB During T2: Ês / Noc: 1 dB During T3: Ês / Noc: -infinity dB During T4: Ês / Noc: 1 dB | During T1: 0 dB During T2: 0 dB During T3: 0 dB During T4: 0 dB | During T1: Ês / Noc: 1 dB During T2: Ês / Noc: 1 dB During T3: Ês / Noc: -infinity dB During T4: Ês / Noc: 1 dB |
| 5.5.9.2.1 EN-DC PSCell FR2 uplink spatial relation switch associated with a known DL-RS | During T1: Ês / Noc: 1 dB During T2: Ês / Noc: 1 dB During T3: Ês / Noc: -infinity dB During T4: Ês / Noc: 1 dB | During T1: 0 dB During T2: 0 dB During T3: 0 dB During T4: 0 dB | During T1: Ês / Noc: 1 dB During T2: Ês / Noc: 1 dB During T3: Ês / Noc: -infinity dB During T4: Ês / Noc: 1 dB |
| 5.5.12.1 EN-DC FR2 PSCell activation and deactivation delay | During T1: Ês2: - Infinity During T2-T4: Ês2: -81 dBm/120kHz | During T1: 0dB During T2-T4: 0dB | During T1: Ês2: - Infinity During T2-T4: Ês2: -81 dBm/120kHz |
| 5.5.13.1 EN-DC FR2 Addition and Release Delay of NR PSCell | During T1: Ês: -infinity During T2~T4: Ês: -81dBm/SCS | During T1: 0dB During T2~T4: 0dB | During T1: Ês: -infinity During T2~T4: Ês: -81dBm/SCS |
| 5.6.1.1 EN-DC FR2 event-triggered reporting without gap in non-DRX | During T1: Ês2: -86dBm/120kHz Ês3: -infinity During T2: Ês2: -86dBm/120kHz Ês3: -86dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês2: -86dBm/120kHz Ês3: -infinity During T2: Ês2: -86dBm/120kHz Ês3: -86dBm/120kHz |
| 5.6.1.2 EN-DC FR2 event-triggered reporting without gap in DRX | During T1: Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +4.00dB Signalled A3-offset: -6.0 | During T1: -3.5dB 0dB 0dB During T2: -3.5dB 0dB 0dB Signalled A3-offset: -1.0 | During T1: Noc: -101.5dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Noc: -101.5dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +4.00dB Signalled A3-offset: -7.0 |
| 5.6.1.3 EN-DC FR2 event-triggered reporting with gap in non-DRX | Same as 5.6.1.1 | Same as 5.6.1.1 | Same as 5.6.1.1 |
| 5.6.1.4 EN-DC FR2 event-triggered reporting with gap in DRX | Same as 5.6.1.2 | Same as 5.6.1.2 | Same as 5.6.1.2 |
| 5.6.2.1 EN-DC FR2-FR2 event-triggered reporting in non-DRX | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -87 dBm/120kHz Ês2: -- infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz |
| 5.6.2.2 EN-DC FR2-FR2 event-triggered reporting in DRX | During T1: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +9.00dB Signalled A3-offset: -6.0 | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB Signalled A3-offset: -6.0 | During T1: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +9.00dB Signalled A3-offset: -12.0 |
| 5.6.2.3 EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.1 | Same as 5.6.2.1 | Same as 5.6.2.1 |
| 5.6.2.4 EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.2 | Same as 5.6.2.2 | Same as 5.6.2.2 |
| 5.6.2.5 EN-DC FR1-FR2 event-triggered reporting in non-DRX | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz | During T1: 0dB During T2: 0dB | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz |
| 5.6.2.6 EN-DC FR1-FR2 event-triggered reporting in DRX | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB |
| 5.6.2.7 EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.5 | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 5.6.2.8 EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.6 | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 5.6.3.1 EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB |
| 5.6.3.2 EN-DC FR2 SSB-based L1-RSRP measurement in DRX | Same as 5.6.3.1 | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 5.6.3.3 EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-RSRP values ± 29dB Reported Differential CSI-RSRP values ± 7dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: RSRP_40 to RSRP_99 CSI-RS#0: DIFFRSRP_1 to DIFFRSRP_7 |
| 5.6.3.4 EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 5.6.3.3 | Same as 5.6.3.3 | Same as 5.6.3.3 |
| 5.6.4.1 EN-DC FR2 SRS-RSRP measurement in non-DRX | During T1: Noc: -98 dBm/15kHz Neighbour cell UE Es/Noc: -infinity During T2: Noc: -98 dBm/15kHz Neighbour cell UE Es/Noc: 4dB In signalling: i1-threshold: -103dBm/SCS | During T1: -2.5dB 3dB During T2: -2.5dB 3dB -9dB | During T1: Noc: -100.5 dBm/15kHz Neighbour cell UE Es/Noc: -infinity During T2: Noc: -100.5 dBm/15kHz Neighbour cell UE Es/Noc: 7dB i1-threshold: -112dBm/SCS |
| 5.6.6.1 EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-SINR values ± 5.5dB Reported Differential CSI-SINR values ± 4.5dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: SINR_51 to SINR_74 CSI-RS#0: DIFFSINR_4 to DIFFSINR_13 |
| 5.6.6.2 EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported SS-SINR values ± 4.0dB Reported Differential SS-SINR values ± 3.0dB | During T1: 0dB 1.5dB 0dB During T2: 0dB 1.5dB 0dB Via mapping | During T1: Noc: -105dBm/15kHz Ês0 / Noc: +1.50dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: +1.50dB Ês1 / Noc: +9.00dB SSB#1: SINR_54 to SINR_71 SSB#0: DIFFSINR_4 to DIFFSINR_10 |
| 5.6.6.3 EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-SINR values ± 4.5dB Reported Differential CSI-SINR values ± 3.5dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: SINR_53 to SINR_72 CSI-RS#0: DIFFSINR_5 to DIFFSINR_12 |
| 5.6.8.1 EN-DC event triggered reporting tests for NR FR2 cell when DRX is used | During T1: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: +6 dB Ês2 / Noc2: +6 dB During T2: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: +6 dB Ês2 / Noc2: +6 dB In signalling: A3-offset = -6dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB In signalling: 0dB | During T1: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: +6 dB Ês2 / Noc2: +6 dB During T2: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: +6 dB Ês2 / Noc2: +6 dB In signalling: A3-offset = -6dB |
| 5.7.1.1 EN-DC FR2 SS-RSRP measurement accuracy | UE PC1 |  |  |
|  | Test 1: Noc: -91.60dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês2: -108.2dBm/120kHz n257 Ês3: -108.2dBm/120kHz n260 Ês2: -105.2dBm/120kHz n260 Ês3: -105.2dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 6dB 6dB 6dB 6dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB Cell 2: RSRP_61 to RSRP_126 Cell 3: RSRP_56 to RSRP_126 Cell 3: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês2: -102.2dBm/120kHz n257 Ês3: -102.2dBm/120kHz n260 Ês2: -99.2dBm/120kHz n260 Ês3: -99.2dBm/120kHz Band dependent. See test case tables in clause 5.7.1.1.5 |
|  | UE PC3 |  |  |
|  | Test 1: Noc: -91.60dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês2: -110.0dBm/120kHz n257 Ês3: -110.0dBm/120kHz n260 Ês2: -107.4dBm/120kHz n260 Ês3: -107.4dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 7.70dB 7.70dB 7.70dB 7.70dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB Cell 2: RSRP_50 to RSRP_108 Cell 3: RSRP_46 to RSRP_103 Cell 3: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês2: -102.3dBm/120kHz n257 Ês3: -102.3dBm/120kHz n260 Ês2: -99.7dBm/120kHz n260 Ês3: -99.7dBm/120kHz Band dependent. See test case tables in clause 5.7.1.1.5 |
| 5.7.1.2 EN-DC FR2-FR2 SS-RSRP measurement accuracy | Test 1 Config 1: Noc1: -90.60dBm/15kHz Noc2: -90.60dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 1: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 1 Config 2: Noc1: -93.70dBm/15kHz Noc2: -93.70dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 2: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1 Config 1: -5.70dB -5.70dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 1: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping Test 1 Config 2: -5.60dB -5.60dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 2: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping | Test 1 Config 1: Noc1: -96.30dBm/15kHz Noc2: -96.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 2 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 3: RSRP_52 to RSRP_109 Cell 3: n257: RSRP_x-23 to RSRP_x+26 n260: RSRP_x-23 to RSRP_x+27 Test 2 Config 1: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 2 n257: RSRP_33 to RSRP_101 n260: RSRP_34 to RSRP_104 Cell 3: n257: RSRP_32 to RSRP_87 n260: RSRP_34 to RSRP_90 Cell 3: n257: RSRP_x-37 to RSRP_x+17 n260: RSRP_x-37 to RSRP_x+19 Test 1 Config 2: Noc1: -99.30dBm/15kHz Noc2: -99.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 12 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 3: RSRP_52 to RSRP_109 Cell 3: n257: RSRP_x-23 to RSRP_x+26 n260: RSRP_x-23 to RSRP_x+27 Test 2 Config 2: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 2 n257: RSRP_36 to RSRP_104 n260: RSRP_37 to RSRP_107 Cell 3: n257: RSRP_35 to RSRP_90 n260: RSRP_37 to RSRP_93 Cell 3: n257: RSRP_x-37 to RSRP_x+17 n260: RSRP_x-37 to RSRP_x+19 |
| 5.7.1.3 EN-DC FR1-FR2 SS-RSRP measurement accuracy | Test 1: Noc: -90dBm/15kHz Ês3 / Noc: +5.0dB Reported absolute RSRP values: ±8dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês3: -98.2dBm/120kHz n260 Ês3: -93.9dBm/120kHz Reported absolute RSRP values: ±8dBFor extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.50dB 0dB Via Mapping Test 2: 5.50dB 5.50dB Via Mapping | Test 1: Noc: -95.50dBm/15kHz Ês3 / Noc: +5dB Band dependent. See test case tables in clause 5.7.1.3.5 Test 2: n257 Ês3: -92.7dBm/120kHz n260 Ês3: -88.4dBm/120kHz Band dependent. See test case tables in clause 5.7.1.3.5 |
| 5.7.2.1 EN-DC FR2 SS-RSRQ measurement accuracy | Test 1: Noc: -95.0dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Test 2: Noc: -95.0dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: -5.70dB 0dB 0dB Via Mapping Test 2: -1.70dB 0dB 0dB Via Mapping | Test 1: Noc: -100.70dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Cell 3: All bands: RSRQ_41 to RSRQ_73 Test 2: Noc: -96.70dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Cell 3: n257, n258, n261: RSRQ_35 to RSRQ_71 n260: RSRQ_34 to RSRQ_71 |
| 5.7.2.2 EN-DC FR2-FR2 SS-RSRQ measurement accuracy | Test 1: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Reported relative RSRQ values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Test 2: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported relative RSRQ values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: -1.90dB -1.90dB 0dB 0dB Via Mapping Via Mapping Test 2: -1.41dB -1.41dB 0dB 0dB Via Mapping Via Mapping | Test 1: Noc1: -95.93dBm/15kHz Noc2: -95.93dBm /15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Cell 2 and Cell 3: RSRQ_41 to RSRQ_73 Cell 3: RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -95.44dBm/15kHz Noc2: -95.44dBm /15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Cell 2 and Cell 3: RSRQ_37 to RSRQ_74 Cell 3: RSRQ_x-9 to RSRQ_x+9 |
| 5.7.3.1 EN-DC FR2 SS-SINR measurement accuracy | Test 1: Noc: -105.0dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +2.66dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc: -105.0dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB Via Mapping Test 2: 0dB 0.2dB 0.2dB Via Mapping | Test 1: Noc: -105.0dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +2.66dB Cell 3: n257, n258, n261: SINR_22 to RSRQ_58 n260: SINR_21 to RSRQ_58 Test 2: Noc: -105.0dBm/15kHz Ês2 / Noc: -2.8dB Ês3 / Noc: -2.8dB Cell 3: n257, n258, n261: SINR_18 to RSRQ_55 n260: SINR_18 to RSRQ_54 |
| 5.7.3.2 EN-DC FR2-FR2 SS-SINR measurement accuracy | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported absolute SINR values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB 0dB Via Mapping Via Mapping Test 2: -0.1dB -0.1dB 0dB 0dB Via Mapping Via Mapping Test 3: 0dB 0dB 0.9dB 0.9dB Via Mapping Via Mapping | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Cell 2 and Cell 3: n257, n258, n261: SINR_27 to SINR_62 n260: SINR_27 to SINR_61 Cell 3: SINR_x-8 to SINR_x+8 Test 2: Noc1: -105.1dBm/15kHz Noc2: -105.1dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Cell 2 and Cell 3: n257, n258, n261: SINR_48 to SINR_87 n260: SINR_48 to SINR_86 Cell 3: SINR_x-17 to SINR_x+17 Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -2.1dB Ês2 / Noc2: -2.1dB Cell 2 and Cell 3: n257, n258, n261: SINR_23 to SINR_60 n260: SINR_22 to SINR_59 Cell 3: SINR_x-9 to SINR_x+9 |
| 5.7.4.1 EN-DC FR2 SSB based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 SSB#0 Ês: -108.1dBm/120kHz n257 SSB#1 Ês: -105.5dBm/120kHz n260 SSB#0 Ês: -108.1dBm/120kHz n260 SSB#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -4.1dB 0dB 0.4dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -1.6dB SSB#0: RSRP_42 to RSRP_101 SSB#1: RSRP_31 to RSRP_89 SSB#1-SSB#0: -9 to -2 Test 2: n257 SSB#0 Ês: -102.4dBm/120kHz n257 SSB#1 Ês: -99.8dBm/120kHz n260 SSB#0 Ês: -102.4dBm/120kHz n260 SSB#1 Ês: -99.8dBm/120kHz SSB#0 and SSB#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 SSB#1-SSB#0: -4 to +4 |
| 5.7.4.2 EN-DC FR2 CSI-RS based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 CSI-RS#0 Ês: -108.1dBm/120kHz n257 CSI-RS#1 Ês: -105.5dBm/120kHz n260 CSI-RS#0 Ês: -108.1dBm/120kHz n260 CSI-RS#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -4.1dB 0dB 0.2dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -1.8dB CSI-RS#0: RSRP_42 to RSRP_101 CSI-RS#1: RSRP_30 to RSRP_89 CSI-RS#1-CSI-RS#0: -9 to -2 Test 2: n257 CSI-RS#0 Ês: -102.4dBm/120kHz n257 CSI-RS#1 Ês: -99.8dBm/120kHz n260 CSI-RS#0 Ês: -102.4dBm/120kHz n260 CSI-RS#1 Ês: -99.8dBm/120kHz CSI-RS#0 and CSI-RS#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 CSI-RS#1-CSI-RS#0: -4 to +4 |
| 5.7.5.1 EN-DC FR2 SRS-RSRP measurement accuracy | During T1: Noc: -100 dBm/15kHz SRS Es/Noc: 2dB During T2: Noc: -100 dBm/15kHz Reported absolute SRS-RSRP values: -89 ± 11.5dB | During T1: 0dB 0dB During T2: 0dB Via mapping | During T1: Noc: -100 dBm/15kHz SRS Es/Noc: 2dB During T2: Noc: -100 dBm/15kHz Reported SRS-RSRP range n257: SRS_RSRP_50 to SRS_RSRP_85 n260: SRS_RSRP_50 to SRS_RSRP_85 |
| 5.7.6.1 EN-DC FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Test 1: Noc: -100dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -2.0dB Reported absolute SINR values: ±5.5dB Reported relative SINR values: ±4.5dB For extreme conditions allow ±1dB + FFS MU additionally | Test 1: -4.1dB 0dB 0.2dB | Test 1: Noc: -104.1dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -1.8dB CSI-RS#0: SINR_53 to SINR_76 CSI-RS#1-CSI-RS#0: 6 to 15 |
| 5.7.6.2 EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Test 1: Noc: -100dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: 0.0dB CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: 0.0dB Reported absolute SINR values: ±4dB Reported relative SINR values: ±3dB For extreme conditions allow ±1dB + FFS MU additionally | Test 1: -4.1dB 0dB 0.2dB 0dB 0.2dB | Test 1: Noc: -104.1dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: 1.2dB CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: 1.2dB SSB#0: SINR_56 to SINR_73 SSB#1-SSB#0: 5 to 12 |
| 5.7.6.3 EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Test 1: Noc: -100dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -2.0dB Reported absolute SINR values: ±4.5dB Reported relative SINR values: ±3.5dB For extreme conditions allow ±1dB + FFS MU additionally | Test 1: -4.1dB 0dB 0.2dB | Test 1: Noc: -104.1dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -1.8dB CSI-RS#0: SINR_55 to SINR_74 CSI-RS#1-CSI-RS#0: 7 to 15 |
| 5.7.7.1 EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | Test 1: Noc: -91.60dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês2: -110.0dBm/120kHz n257 Ês3: -110.0dBm/120kHz n260 Ês2: -107.4dBm/120kHz n260 Ês3: -107.4dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 7.70dB 7.70dB 7.70dB 7.70dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB Cell 2: RSRP_50 to RSRP_108 Cell 3: RSRP_46 to RSRP_103 Cell 3: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês2: -102.3dBm/120kHz n257 Ês3: -102.3dBm/120kHz n260 Ês2: -99.7dBm/120kHz n260 Ês3: -99.7dBm/120kHz Band dependent. See test case tables in clause 5.7.7.1.5 |
| 5.7.7.2 EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | Test 1: Noc1: -90.60dBm/15kHz Noc2: -90.60dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.70dB -5.70dB 0dB 0dB Via Mapping Via Mapping Test 2: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping | Test 1: Noc1: -96.30dBm/15kHz Noc2: -96.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 2 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 3: RSRP_52 to RSRP_109 Cell 3: n257: RSRP_x-23 to RSRP_x+26 n260: RSRP_x-23 to RSRP_x+27 Test 2: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 2 n257: RSRP_33 to RSRP_101 n260: RSRP_34 to RSRP_104 Cell 3: n257: RSRP_32 to RSRP_87 n260: RSRP_34 to RSRP_90 Cell 3: n257: RSRP_x-37 to RSRP_x+17 n260: RSRP_x-37 to RSRP_x+19 |
| 5.7.8.1 EN-DC intra-frequency FR2 CSI-RSRQ measurement accuracy | Test 1: Noc: -95.0dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported absolute RSRQ values: ±2.5dB Test 2: Noc: -95.0dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Reported absolute RSRQ values: ±3.5dB | Test 1: -5.70dB 0dB 0dB Via Mapping Test 2: -1.70dB 0dB 0dB Via Mapping | Test 1: Noc: -100.70dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Cell 3: n257, n258, n261: RSRQ_52 to RSRQ 62 n260: RSRQ_51 to RSRQ_62 Test 2: Noc: -96.70dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Cell 3: All bands: RSRQ_46 to RSRQ_60 |
| 5.7.8.2 EN-DC inter-frequency FR2 CSI-RSRQ measurement accuracy | Test 1: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported absolute RSRQ values: ±2.5dB Reported relative RSRQ values: ±3dB Test 2: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute RSRQ values: ±3.5dB Reported relative RSRQ values: ±4dB | Test 1: -1.90dB -1.90dB 0dB 0dB Via Mapping Via Mapping Test 2: -1.41dB -1.41dB 0dB 0dB Via Mapping Via Mapping | Test 1: Noc1: -95.93dBm/15kHz Noc2: -95.93dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Cell 2: All bands: RSRQ_52 to RSRQ_62 Cell 3: All bands: RSRQ_52 to RSRQ_62 Cell 3: RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -95.44dBm/15kHz Noc2: -95.44dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Cell 2: n257, n258, n261: RSRQ_48 to RSRQ_63 n260: RSRQ_48 to RSRQ_62 Cell 3: n257, n258, n261: RSRQ_48 to RSRQ_63 n260: RSRQ_48 to RSRQ_62 Cell 3: RSRQ_x-9 to RSRQ_x+9 |
| 5.7.9.1 EN-DC intra-frequency FR2 CSI-SINR measurement accuracy | Test 1: Noc: -105.0dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Reported absolute SINR values: ±3dB Test 2: Noc: -105.0dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Reported absolute SINR values: ±3.5dB | Test 1: 0dB 0dB 0dB Via Mapping Test 2: 0dB 0.2dB 0.2dB Via Mapping | Test 1: Noc: -105.0dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Cell 3: All bands: SINR_32 to SINR_48 Test 2: Noc: -1050dBm/15kHz Ês1 / Noc: -2.8dB Ês2 / Noc: -2.8dB Cell 3: n257, n258, n261: SINR_29 to SINR_45 n260: SINR_28 to SINR_45 |
| 5.7.9.2 EN-DC inter-frequency FR2 CSI-SINR measurement accuracy | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Reported absolute SINR values: ±3dB Reported relative SINR values: ±3.5dB Test 2: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Reported absolute SINR values: ±3dB Reported relative RSRQ values: ±3.5dB Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute SINR values: ±3.5dB Reported relative SINR values: ±4dB | Test 1: 0dB 0dB 0dB 0dB Via Mapping Via Mapping Test 2: -0.1dB -0.1dB 0dB 0dB Via Mapping Via Mapping Test 3: 0dB 0dB 0.9dB 0.9dB Via Mapping Via Mapping | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Absolute accuracy: Cell 2: n257, n258, n261: SINR_37 to SINR_52 n260: SINR_36 to SINR_52 Cell 3: n257, n258, n261: SINR_37 to SINR_52 n260: SINR_36 to SINR_52 Cell 3: SINR_x-8 to SINR_x+8 Test 2: Noc1: -105.1dBm/15kHz Noc2: -105.1dBm/15kHz Ês1 / Noc1: 11dB Ês2 / Noc2: 11dB Cell 2: n257, n258, n261: SINR_54 to SINR_81 n260: SINR_54 to SINR_80 Cell 3: n257, n258, n261: SINR_54 to SINR_81 n260: SINR_54 to SINR_80 Cell 3: SINR_x-17 to SINR_x+17 Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -2.1dB Ês2 / Noc2: -2.1dB Cell 2: n257, n258, n261: SINR_33 to SINR_50 n260: SINR_32 to SINR_50 Cell 3: n257, n258, n261: SINR_33 to SINR_50 n260: SINR_32 to SINR_50 Cell 3: SINR_x-9 to SINR_x+9 |

Table F.1.3.2-4: Derivation of test requirements for NR SA FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] |  | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- | --- |
| 7.1.1.1 NR SA FR2 cell re-selection | UE PC1 |  |  |  |
|  | During T1: Noc = -102 dBm/15kHz Es1/Noc = 8dB Es2/Noc = -infinity During T2: Noc = -102 dBm/15kHz Es1/Noc = -3dB Es2/Noc = 1.5dB During T3: Noc = -102 dBm/15kHz Es1/Noc = 1.5dB Es2/Noc = -3dB In signalling: SintraSearchP = 50 Qrxlevmin = -138dBm/SCS for config. 1 Qrxlevmin = -135dBm/SCS for config. 2 |  | During T1: 0dB 0dB N/A During T2: 0dB 0.5dB 1.0dB During T3: 0dB 1.0dB 0.5dB In signalling: 12 37 dB 37 dB | During T1: Noc = -102 dBm/15kHz Es1/Noc = 8dB Es2/Noc = -infinity During T2: Noc = -102 dBm/15kHz Es1/Noc = -2.50dB Es2/Noc = 2.5dB During T3: Noc = -102 dBm/15kHz Es1/Noc = 2.5dB Es2/Noc = -2.50dB In signalling: SintraSearchP = 62 Qrxlevmin = -101dBm/SCS for config. 1 Qrxlevmin = -98dBm/SCS for config. 2 |
|  | UE PC3 |  |  |  |
|  | During T1: Noc = -102 dBm/15kHz Es1/Noc = 8dB Es2/Noc = -infinity During T2: Noc = -102 dBm/15kHz Es1/Noc = -3dB Es2/Noc = 1.5dB During T3: Noc = -102 dBm/15kHz Es1/Noc = 1.5dB Es2/Noc = -3dB In signalling: Qrxlevmin = -138dBm/SCS for config. 1 Qrxlevmin = -135dBm/SCS for config. 2 |  | During T1: 0dB 0dB N/A During T2: 0dB 0.1dB 0.55dB During T3: 0dB 0.55dB 0.1dB In signalling: 18 dB 18 dB | During T1: Noc = -102 dBm/15kHz Es1/Noc = 8dB Es2/Noc = -infinity During T2: Noc = -102 dBm/15kHz Es1/Noc = -2.90dB Es2/Noc = 2.05dB During T3: Noc = -102 dBm/15kHz Es1/Noc = 2.05dB Es2/Noc = -2.90dB In signalling: Qrxlevmin = -120dBm/SCS for config. 1 Qrxlevmin = -117dBm/SCS for config. 2 |
| 7.1.1.2 NR SA FR2-FR2 cell re-selection | During T1: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -10.5dB During T2: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -infinity During T3: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 8dB Es2/Noc2 = 8.5dB In signalling: Cell 2 Qrxlevmin = -138dBm/SCS for config. 1 Cell 2 Qrxlevmin = -135dBm/SCS for config. 2 Cell 2 SnonintraSearchP = 50dB Cell 1 Threshx, highP = 48dB Cell 2 Threshserving, lowP = 44dB Cell 2 Threshx, lowP = 50dB |  | During T1: 0dB 0dB 0dB 7.6dB During T2: 0dB 0dB 0dB N/A During T3: 0dB 0dB 0dB 0dB In signalling: 16 dB 16 dB 12dB -16dB 18dB -32dB | During T1: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -2.9dB During T2: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -infinity During T3: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 8dB Es2/Noc2 = 8.5dB In signalling: Qrxlevmin = -124dBm/SCS for config. 1 Qrxlevmin = -121dBm/SCS for config. 2 Cell 2 SnonintraSearchP = 62dB Cell 1 Threshx, highP = 32dB Cell 2 Threshserving, lowP = 62dB Cell 2 Threshx, lowP = 18dB |
| 7.1.1.3 NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | During T1: Noc = -102 dBm/15kHz Es1/Noc = -3dB Es2/Noc = 1.5dB During T2: Noc = -102 dBm/15kHz Es1/Noc = 1.5dB Es2/Noc = -3dB In signalling: Qrxlevmin = -140dBm/SCS for config. 1 Qrxlevmin = -137dBm/SCS for config. 2 |  | During T1: 0dB 0.1dB 0.55dB During T2: 0dB 0.55dB 0.1dB In signalling: 20dB 20dB | During T1: Noc = -102 dBm/15kHz Es1/Noc = -2.90dB Es2/Noc = 2.05dB During T2: Noc = -102 dBm/15kHz Es1/Noc = 2.05dB Es2/Noc = -2.90dB In signalling: Qrxlevmin = -120dBm/SCS for config. 1 Qrxlevmin = -117dBm/SCS for config. 2 |
| 7.1.1.4 NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | During T1: Noc = -102 dBm/15kHz Es1/Noc = -3dB Es2/Noc = 1.5dB During T2: Noc = -102 dBm/15kHz Es1/Noc = 1.5dB Es2/Noc = -3dB In signalling: Qrxlevmin = -140dBm/SCS for config. 1 Qrxlevmin = -137dBm/SCS for config. 2 SsearchThresholdP = 35dB |  | During T1: 0dB 0.1dB 0.55dB During T2: 0dB 0.55dB 0.1dB In signalling: 20dB 20dB -27dB | During T1: Noc = -102 dBm/15kHz Es1/Noc = -2.90dB Es2/Noc = 2.05dB During T2: Noc = -102 dBm/15kHz Es1/Noc = 2.05dB Es2/Noc = -2.90dB In signalling: Qrxlevmin = -120dBm/SCS for config. 1 Qrxlevmin = -117dBm/SCS for config. 2 SsearchThresholdP = 8dB |
| 7.1.1.5 NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | During T1: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -10.5dB During T2: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 8dB Es2/Noc2 = 8.5dB In signalling: Cell 2 Qrxlevmin = -138dBm/SCS for config. 1 Cell 2 Qrxlevmin = -135dBm/SCS for config. 2 Cell 2 SnonintraSearchP = 50dB Cell 1 Threshx, highP = 48dB Cell 2 Threshserving, lowP = 44dB Cell 2 Threshx, lowP = 50dB SSearchDeltaP = 6dB |  | During T1: 0dB 0dB 0dB 7.6dB During T2: 0dB 0dB 0dB 0dB In signalling: 16 dB 16 dB 12dB -16dB 18dB -32dB 6dB | During T1: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -2.9dB During T2: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 8dB Es2/Noc2 = 8.5dB In signalling: Qrxlevmin = -124dBm/SCS for config. 1 Qrxlevmin = -121dBm/SCS for config. 2 Cell 2 SnonintraSearchP = 62dB Cell 1 Threshx, highP = 32dB Cell 2 Threshserving, lowP = 62dB Cell 2 Threshx, lowP = 18dB SSearchDeltaP = 12dB |
| 7.1.1.6 NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | During T1: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -10.5dB During T2: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 8dB Es2/Noc2 = 8.5dB In signalling: Cell 2 Qrxlevmin = -138dBm/SCS for config. 1 Cell 2 Qrxlevmin = -135dBm/SCS for config. 2 Cell 2 SnonintraSearchP = 50dB Cell 1 Threshx, highP = 48dB Cell 2 Threshserving, lowP = 44dB Cell 2 Threshx, lowP = 50dB Cell 2 SsearchThresholdP = 29dB |  | During T1: 0dB 0dB 0dB 7.6dB During T2: 0dB 0dB 0dB 0dB In signalling: 16 dB 16 dB 12dB -16dB 18dB -32dB -17dB | During T1: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -2.9dB During T2: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 8dB Es2/Noc2 = 8.5dB In signalling: Qrxlevmin = -124dBm/SCS for config. 1 Qrxlevmin = -121dBm/SCS for config. 2 Cell 2 SnonintraSearchP = 62dB Cell 1 Threshx, highP = 32dB Cell 2 Threshserving, lowP = 62dB Cell 2 Threshx, lowP = 18dB Cell 2 SsearchThresholdP = 12dB |
| 7.3.1.2 NR SA FR2 Intra-frequency handover; unknown target cell | During T1: Noc: -104.7dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.7dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +7.00dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -104.7dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.7dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +7.00dB |
| 7.3.1.3 NR SA FR2-FR2 Inter-frequency handover; unknown target cell | During T1: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +5.00dB Ês2 / Noc2: -infinity During T2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +5.00dB Ês2 / Noc2: +5.00dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +5.00dB Ês2 / Noc2: -infinity During T2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +5.00dB Ês2 / Noc2: +5.00dB |
| 7.3.1.4 NR SA FR1-FR2 synchronous DAPS handover | During T1: Noc -104.7dBm/15kHz Es2/Noc -infinity During T2-T5: Noc -104.7 dBm/15kHz Es2/Noc 10 dB |  | During T1: 0 dB 0 dB During T2-T5: 0 dB 0 dB | During T1: Noc -104.7dBm/15kHz Es2/Noc -infinity During T2-T5: Noc -104.7 dBm/15kHz Es2/Noc 10 dB |
| 7.3.1.5 NR SA FR1-FR2 asynchronous DAPS handover | Same as 7.3.1.4 |  | Same as 7.3.1.4 | Same as 7.3.1.4 |
| 7.3.2.1.1 NR SA FR2 RRC re-establishment | During T1: Noc = -104.7 dBm/15 kHz Ês1 / Noc = 4 dB Ês2 / Noc = 2 dB During T2: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB During T3: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês1 / Noc = 4 dB Ês2 / Noc = 2 dB During T2: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB During T3: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB |
| 7.3.2.1.2 NR SA FR2 - FR2 RRC re-establishment | During T1: Noc1 = -92.1 dBm/15 kHz Noc2 = -92.1 dBm/15 kHz Ês1 / Noc1 = 0 dB Ês2 / Noc 2= - Infinity During T2: Noc1 = -92.1 dBm/15 kHz Noc2 = -92.1 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = - Infinity During T3: Noc1 = -92.1 dBm/15 kHz Noc2 = -92.1 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = 0 dB |  | During T1: -0.2dB -0.2dB 0dB 0dB During T2: -0.2dB -0.2dB 0dB 0dB During T3: -0.2dB -0.2dB 0dB 0dB | During T1: Noc1 = -92.3 dBm/15 kHz Noc2 = -92.3 dBm/15 kHz Ês1 / Noc1 = 0 dB Ês2 / Noc 2= - Infinity During T2: Noc1 = -92.3 dBm/15 kHz Noc2 = -92.3 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = - Infinity During T3: Noc1 = -92.3 dBm/15 kHz Noc2 = -92.3 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = 0 dB |
| 7.3.2.1.3 NR SA FR2 RRC re-establishment without serving cell timing | During T1: Noc = -104.7 dBm/15 kHz Ês1 / Noc = 5 dB Ês2 / Noc = - infinity During T2: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = - infinity During T3: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 5 dB |  | During T1: 0dB 0dB N/A During T2: 0dB N/A N/A During T3: 0dB N/A 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês1 / Noc = 5 dB Ês2 / Noc = - infinity During T2: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = - infinity During T3: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 5 dB |
| 7.3.2.2.1 NR SA FR2 contention based random access | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc |  | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 7.3.2.2.2 NR SA FR2 non-contention based random access | Same as 7.3.2.2.1 |  | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 |
| 7.3.2.3.1 NR SA FR2-FR2 RRC connection release with redirection | Same as 7.3.1.3 |  | Same as 7.3.1.3 | Same as 7.3.1.3 |
| 7.3.3.1 NR SA FR2 conditional handover | During T1: Noc -104.7dBm/15kHz Es1/Noc 6 dB Es2/Noc -infinity During T2: Noc -104.7dBm/15kHz Es1/Noc 6 dB Es2/Noc 11dB In signalling: A3-offset = -1dB |  | During T1 -1.5dB 0dB 0dB During T2 -1.5dB 0dB 0dB In signalling: -1dB | During T1: Noc -106.2 dBm/15kHz Es1/Noc 6 dB Es2/Noc -infinity During T2: Noc -106.2 dBm/15kHz Es1/Noc 6 dB Es2/Noc 11dB In signalling: A3-offset = -2dB |
| 7.3.3.3 | Same as 7.3.3.1 |  | Same as 7.3.3.1 | Same as 7.3.3.1 |
| 7.3.4.1 NR SA FR2 RACH based Intra-frequency PCell switch from FR2 to FR2 | During T1~T4: Noc: -104.7dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +7.00dB |  | During T1~T4: 0dB 0dB 0dB | During T1~T4: Noc: -104.7dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +7.00dB |
| 7.3.4.2 NR SA FR2 RACH-less Intra-frequency PCell switch from FR2 to FR2 | During T1~T5: Noc: -104.7dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +7.00dB |  | During T1~T5: 0dB 0dB 0dB | During T1~T5: Noc: -104.7dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +7.00dB |
| 7.3.4.3 NR SA FR2 RACH-based Inter-frequency LTM PCell switch from FR2 to FR2 | During T1~T4: Noc: -104.7dBm/15kHz Ês1 / Noc: +5.00dB Ês2 / Noc: +5.00dB |  | During T1~T4: 0dB 0dB 0dB | During T1~T4: Noc: -104.7dBm/15kHz Ês1 / Noc: +5.00dB Ês2 / Noc: +5.00dB |
| 7.4.1.1 SA FR2 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB |  | ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB | Test 1 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB |
| 7.4.1.3 NR SA FR2 UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 | Test 1 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 3 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 4 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB |  | ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB | Test 1 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 3 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 4 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB |
| 7.4.3.1 NR SA FR2 UE timing advance adjustment accuracy | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE Timing Advance Adjustment Accuracy for 120kHz SCS = ±32 Tc |  | 0dB 0dB +/- 40 Tc | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE TAAA for 120kHz SCS = ±72 Tc |
| 7.5.1.1 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM-RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB |  | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB |
| 7.5.1.2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM-RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -4.5 dB During T5: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: +2 dB |  | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB During T4: -2.7 dB 0 dB 0 dB During T5: -2.7 dB 0 dB 2.1 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -4.5 dB During T5: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: +4.1 dB |
| 7.5.1.3 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM-RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB |  | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB |
| 7.5.1.4 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM-RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: -14 dB |  | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB During T4: 0 dB -0.4 dB -0.4 dB During T5: 0 dB 1.3 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.9 dB Ês2 / Noc: -15.4 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: 3.3 dB Ês2 / Noc: -15.4 dB |
| 7.5.1.9 NR SA FR2 radio link monitoring UE scheduling restrictions | Same as 5.5.1.9 |  | Same as 5.5.1.9 | Same as 5.5.1.9 |
| 7.5.3.4 NR SA FR2 direct SCell activation at SCell addition of known SCell |  | During T1: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc2: +14 dB During T2: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc2: +14 dB During T3: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc2: +14 dB In signalling: A3-offset = -15dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB In signalling: 0dB | During T1: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc2: +14 dB During T2: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc2: +14 dB During T3: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc2: +14 dB In signalling: A3-offset = -15dB |
| 7.5.3.5 NR SA FR2 direct SCell activation at SCell addition of known SCell |  | During T1: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc1: +14 dB Ês3 / Noc2: +14 dB During T2: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc1: +14 dB Ês3 / Noc2: +14 dB During T3: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc1: +14 dB Ês3 / Noc2: +14 dB In signalling: A3-offset = -15dB | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB In signalling: 0dB | During T1: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc1: +14 dB Ês3 / Noc2: +14 dB During T2: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc1: +14 dB Ês3 / Noc2: +14 dB During T3: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc1: +14 dB Ês3 / Noc2: +14 dB In signalling: A3-offset = -15dB |
| 7.5.3.13 NR SA FR2 SCell Activation for SCell in FR2 intra-band in non-DRX |  | During T1: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: +7 dB Ês2 / Noc2: +7 dB During T2: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: +7 dB Ês2 / Noc2: +7 dB In signalling: A3-offset = -15dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB In signalling: 0dB | During T1: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: +7 dB Ês2 / Noc2: +7 dB During T2: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: +7 dB Ês2 / Noc2: +7 dB In signalling: A3-offset = -15dB |
| 7.5.3.14 NR SA FR2 SCell Activation for known SCell in FR2 inter-band | During T1: Noc1: -92.1 dBm/15kHz Noc2: -92.1 dBm/15kHz Ês1 / Noc1: 0 dB Ês2 / Noc2: 0 B During T2: Noc1: -92.1 dBm/15kHz Noc2: -92.1 dBm/15kHz Ês1 / Noc1: 0 dB Ês2 / Noc2: 0 B In signalling: A3-offset = -15dB |  | During T1: -4.6dB -4.6dB 0dB 0dB During T2: -4.6dB -4.6dB 0dB 0dB In signalling: 0dB | During T1: Noc1: -96.7 dBm/15kHz Noc2: -96.7 dBm/15kHz Ês1 / Noc1: 0 dB Ês2 / Noc2: 0 B During T2: Noc1: -96.7 dBm/15kHz Noc2: -96.7 dBm/15kHz Ês1 / Noc1: 0 dB Ês2 / Noc2: 0 B In signalling: A3-offset = -15dB |
| 7.5.5.1 NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX | During T1: SNR_SSB q0: 5 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: -3 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20.2 dB In signalling: rsrp-ThresholdSSB: -95 dBm/120kHz for Config 1 rsrp-ThresholdSSB: -92 dBm/240kHz for Config 2 |  | +8.70 dB 0 dB +8.70 dB 0 dB 0 dB -0.2 dB -14 dB -14 dB | During T1: SNR_SSB q0: 13.7 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: 5.7 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20 dB In signalling: rsrp-ThresholdSSB: -109 dBm/120kHz for Config 1 srp-ThresholdSSB: -106 dBm/240kHz for Config 2 |
| 7.5.5.2 NR SA FR2 SSB-based beam failure detection and link recovery in DRX | Same as 7.5.5.1 |  | Same as 7.5.5.1 | Same as 7.5.5.1 |
| 7.5.5.3 NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês1 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês1 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdSSB: -95 dBm/SCS kHz |  | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês1 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês1 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdSSB: -109.0 dBm/SCS kHz |
| 7.5.5.4 NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX | Same as 7.5.5.3 |  | Same as 7.5.5.3 | Same as 7.5.5.3 |
| 7.5.5.5 NR SA FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Same as 7.5.5.1 |  | Same as 7.5.5.1 | Same as 7.5.5.1 |
| 7.5.5.6 NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdBFR: -95 dBm/SCS kHz |  | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdBFR: -109.0 dBm/SCS kHz |
| 7.5.5.7 NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | Same as 7.5.5.6 |  | Same as 7.5.5.6 | Same as 7.5.5.6 |
| 7.5.5.9 NR SA FR2 SCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX | Noc = -104.7 dBm/120 kHz Ês1 / Noc for CSI-RS set q0,0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês1 / Noc for CSI-RS set q0,1 T1-T5: 5 dB Ês1 / Noc for CSI-RS set q1,0 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdSSB: -95 dBm/SCS kHz |  | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 8.7dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês1 / Noc for CSI-RS set q0,0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês1 / Noc for CSI-RS set q0,1 T1-T5: 13.7 dB Ês1 / Noc for CSI-RS set q1,0 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdSSB: -109.0 dBm/SCS kHz |
| 7.5.5.10 NR SA FR2 PCell TRP specific SSB-based Beam Failure Detection and Link Recovery in non-DRX | During T1: SNR_SSB q0,0: 5 dB SNR_SSB q0,1: 5 dB SNR_SSB q1,0: 0.2 dB During T2: SNR_SSB q0,0: -3 dB SNR_SSB q0,1: 5 dB SNR_SSB q1,0: 0.2 dB During T3-T5: SNR_SSB q0,0: -12 dB SNR_SSB q0,1: 5 dB SNR_SSB q1,0: 20.2 dB In signalling: rsrp-ThresholdSSB: -95 dBm/120kHz for Config 1 rsrp-ThresholdSSB: -92 dBm/240kHz for Config 2 |  | +8.70 dB +8.70 dB 0 dB +8.50 dB +8.70 dB 0 dB 0 dB +8.70 dB -0.2 dB -14 dB -14 dB | During T1: SNR_SSB q0,0: 13.7 dB SNR_SSB q0,1: 13.7 dB SNR_SSB q1,0: 0.2 dB During T2: SNR_SSB q0,0: 5.5 dB SNR_SSB q0,1: 13.7 dB SNR_SSB q1,0: 0.2 dB During T3-T5: SNR_SSB q0,0: -12 dB SNR_SSB q0,1: 13.7 dB SNR_SSB q1,0: 20 dB In signalling: rsrp-ThresholdSSB: -109 dBm/120kHz for Config 1 srp-ThresholdSSB: -106 dBm/240kHz for Config 2 |
| 7.5.6.2.1 NR SA FR2 RRC-based DL active BWP switch in non-DRX | During T1: Noc2: -112dBm/15kHz Ês2 / Iot2: +18dB |  | During T1: 0dB 0dB | During T1: Noc2: -112dBm/15kHz Ês2 / Iot2: +18dB |
| 7.5.6.3.1 Active BWP switch on multiple SCells with non-DRX in SA | During T1/T2/T3: Noc1: -111.7dBm/15kHz Noc2: -111.7dBm/15kHz Noc3: -111.7dBm/15kHz Ês1 / Iot1: +7dB Ês2 / Iot2: +7dB Ês3 / Iot3: +7dB |  | During T1/T2/T3: 0dB 0dB 0dB 0dB 0dB 0dB | During T1/T2/T3: Noc1: -111.7dBm/15kHz Noc2: -111.7dBm/15kHz Noc3: -111.7dBm/15kHz Ês1 / Iot1: +7dB Ês2 / Iot2: +7dB Ês3 / Iot3: +7dB |
| 7.5.6.5.1 Active BWP switch on multiple SCells with non-DRX in SA | During T1: Noc1: -111.7dBm/15kHz Noc2: -111.7dBm/15kHz Ês1 / Iot1: +18dB Ês2 / Iot2: +18dB |  | During T1: 0dB 0dB 0dB 0dB | During T1: Noc1: -111.7dBm/15kHz Noc2: -111.7dBm/15kHz Ês1 / Iot1: +18dB Ês2 / Iot2: +18dB |
| 7.5.8.1.1 NR SA PCell FR2 MAC-CE based active TCI state switch for a known TCI state | During T1: SSB#0: Ês / Iot, BB: +8.3dB SSB#0: Ês: -80.6dB SSB#1: Ês / Iot, BB: - infinity SSB#1: Ês: - infinity During T2: SSB#0: Ês / Iot, BB: +8.3dB SSB#0: Ês: -80.6dB SSB#1: Ês / Iot, BB: +8.3dB SSB#1: Ês: -80.6dB |  | During T1: 0dB -0.3dB 0dB 0dB During T2: 0dB -0.3FFSdB 0dB -0.3FFSdB | During T1: SSB#0: Ês / Iot, BB: +8.3dB SSB#0: Ês: -80.9dB SSB#1: Ês / Iot, BB: - infinity SSB#1: Ês: - infinity During T2: SSB#0: Ês / Iot, BB: +8.3dB SSB#0: Ês: -80.9dB SSB#1: Ês / Iot, BB: +8.3dB SSB#1: Ês: -80.9dB |
| 7.5.8.2.1 NR SA PCell FR2 RRC based active TCI state switch for a known TCI state | Same as 7.5.8.1.1 |  | Same as 7.5.8.1.1 | Same as 7.5.8.1.1 |
| 7.5.9.1.1 EN-DC PSCell FR2 uplink spatial relation switch for a known spatial relation | During T1: Ês / Noc: 1 dB During T2: Ês / Noc: 1 dB During T3: Ês / Noc: -infinity dB During T4: Ês / Noc: 1 dB |  | During T1: 0 dB During T2: 0 dB During T3: 0 dB During T4: 0 dB | During T1: Ês / Noc: 1 dB During T2: Ês / Noc: 1 dB During T3: Ês / Noc: -infinity dB During T4: Ês / Noc: 1 dB |
| 7.5.9.2.1 EN-DC PSCell FR2 uplink spatial relation switch associated with a known DL-RS | During T1: Ês / Noc: 1 dB During T2: Ês / Noc: 1 dB During T3: Ês / Noc: -infinity dB During T4: Ês / Noc: 1 dB |  | During T1: 0 dB During T2: 0 dB During T3: 0 dB During T4: 0 dB | During T1: Ês / Noc: 1 dB During T2: Ês / Noc: 1 dB During T3: Ês / Noc: -infinity dB During T4: Ês / Noc: 1 dB |
| 7.5.11.1 NR SA FR2 UE UL carrier RRC reconfiguration delay | During T1: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: +7dB During T2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: +7dB During T3: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: +7dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: +7dB During T2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: +7dB During T3: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: +7dB |
| 7.5.12.1 Addition and Release Delay of PSCell | During T1: Ês: -infinity During T2~T4: Ês: -81dBm/SCS |  | During T1: 0dB During T2~T4: 0dB | During T1: Ês: -infinity During T2~T4: Ês: -81dBm/SCS |
| 7.5.13.6.1 NR SA FR2 MAC-CE based active uplink TCI state switch for 2 known TCI states | During T1: Noc: -83.10dBm/120kHz Ês0 / Noc: 1 dB Ês1/ Noc: -infinity Ês2 / Noc: -infinity Ês3 / Noc: -infinity During T2: Noc: -83.10dBm/120kHz Ês0 / Noc: 1 dB Ês1/ Noc: 1 dB Ês2 / Noc: 1 dB Ês3 / Noc: 1 dB |  | During T1: -3dB 2.4dB 0dB 0dB 0dB During T2: -3dB 2.4dB 2.4dB 2.4dB 2.4dB | During T1: Noc: -86.10dBm/120kHz Ês0 / Noc: 3.4 dB Ês1/ Noc: -infinity Ês2 / Noc: -infinity Ês3 / Noc: -infinity During T2: Noc: -86.10dBm/120kHz Ês0 / Noc: 3.4 dB Ês1/ Noc: 3.4 dB Ês2 / Noc: 3.4 dB Ês3 / Noc: 3.4 dB |
| 7.5.14 NR SA FR2 PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2 | During T1~T4: Noc: -104.7 dBm/15kHz Ês / Noc: 7 dB |  | During T1~T4: 0.9dB 0dB | During T1~T4: Noc: -103.8 dBm/15kHz Ês / Noc: 7 dB |
| 7.6.1.1 NR SA FR2 event-triggered reporting without gap in non-DRX | During T1: Ês1: -86dBm/120kHz Ês2: -infinity During T2: Ês1: -86dBm/120kHz Ês2: -86dBm/120kHz |  | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -86dBm/120kHz Ês2: -infinity During T2: Ês1: -86dBm/120kHz Ês2: -86dBm/120kHz |
| 7.6.1.2 NR SA FR2 event-triggered reporting without gap in DRX | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Signalled A3-offset: -6.0 |  | During T1: -3.5dB 0dB 0dB During T2: -3.5dB 0dB 0dB Signalled A3-offset: -1.0 | During T1: Noc: -101.5dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -101.5dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Signalled A3-offset: -7.0 |
| 7.6.1.3 NR SA FR2 event-triggered reporting with gap in non-DRX | Same as 7.6.1.1 |  | Same as 7.6.1.1 | Same as 7.6.1.1 |
| 7.6.1.4 NR SA FR2 event-triggered reporting with gap in DRX | Same as 7.6.1.2 |  | Same as 7.6.1.2 | Same as 7.6.1.2 |
| 7.6.2.1 NR SA FR2-FR2 event-triggered reporting in non-DRX | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz |  | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz |
| 7.6.2.2 NR SA FR2-FR2 event-triggered reporting in DRX | During T1: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +9.00dB Signalled A3-offset: -6.0 |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB Signalled A3-offset: -6.0 | During T1: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +9.00dB Signalled A3-offset: -12.0 |
| 7.6.2.3 NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 7.6.2.1 |  | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 7.6.2.4 NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 7.6.2.2 |  | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 7.6.2.5 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1) | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz |  | During T1: 0dB During T2: 0dB | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz |
| 7.6.2.6 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1) | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB |  | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB |
| 7.6.2.7 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1) | Same as 5.6.2.5 |  | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 7.6.2.8 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1) | Same as 5.6.2.6 |  | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 7.6.2.10 | During T1: Noc: -102dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -Infinity During T2: Noc: -102dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +8.00dB |  | During T1: -0.7dB 0dB 0dB During T2: -0.7dB 0dB 0dB | During T1: Noc: -102.7dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -Infinity During T2: Noc: -102.7dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +8.00dB |
| 7.6.2.11 | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -Infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  | During T1: -1.2dB 0dB 0dB During T2: -1.2dB 0dB 0dB | During T1: Noc: -99.2dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -Infinity During T2: Noc: -99.2dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |
| 7.6.2.20 NR SA FR2-FR2 event triggered reporting using MUSIM gap with higher priority | Same as 7.6.2.1 |  | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 7.6.3.1 NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | Same as 5.6.3.1 |  | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 7.6.3.2 NR SA FR2 SSB-based L1-RSRP measurement in DRX | Same as 5.6.3.1 |  | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 7.6.3.3 NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-RSRP values ± 29dB Reported Differential CSI-RSRP values ± 7dB |  | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: RSRP_40 to RSRP_99 CSI-RS#0: DIFFRSRP_1 to DIFFRSRP_7 |
| 7.6.3.4 NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 7.6.3.3 |  | Same as 7.6.3.3 | Same as 7.6.3.3 |
| 7.6.3.6 NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used | Same as 7.6.6.2 |  | Same as 7.6.6.2 | Same as 7.6.62 |
| 7.6.4.1 NR SA FR2 SRS-RSRP measurement in non-DRX | Same as 5.6.4.1 |  | Same as 5.6.4.1 | Same as 5.6.4.1 |
| 7.6.6.1 NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Same as 5.6.6.1 |  | Same as 5.6.6.1 | Same as 5.6.6.1 |
| 7.6.6.2 NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported SS-SINR values ± 4.5dB Reported Differential SS-SINR values ± 3.5dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB Via mapping | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB SSB#1: SINR_53 to SINR_72 SSB#0: DIFFSINR_5 to DIFFSINR_12 |
| 7.6.6.3 NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-SINR values ± 4.0dB Reported Differential CSI-SINR values ± 3.0dB |  | 0dB 1.5dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: +1.50dB Ês1 / Noc: +9.00dB CSI-RS#1: SINR_54 to SINR_71 CSI-RS#0: DIFFSINR_4 to DIFFSINR_10 |
| 7.6.13.1 NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using PRS | During T1: Noc : -98 dBm/15kHz Ês / Noc: infinity During T2: Noc: -98 dBm/15kHz Ês / Noc: -2 dB |  | During T1: 0 dB 0 dB During T2: 0 dB 0 dB | During T1: Noc : -98 dBm/15kHz Ês / Noc: infinity During T2: Noc: -98 dBm/15kHz Ês / Noc: -2 dB |
| 7.6.13.2 NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using TRS | Same as 7.6.13.1 |  | Same as 7.6.13.1 | Same as 7.6.13.1 |
| 7.6.14.1 NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and autonomous activation/deactivation | During T1 – T2: Ês1: -89dBm/120kHz Ês2: -infinity During T3: Ês1: -89dBm/120kHz Ês2: -89dBm/120kHz Signalled A4-Thresholds: -120 dBm/SCS |  | During T1 - T2: 0dB 0dB During T3: 0dB 0dB 0 dB | During T1 – T2: Ês1: -89dBm/120kHz Ês2: -infinity During T3: Ês1: -89dBm/120kHz Ês2: -89dBm/120kHz Signalled A4-Thresholds: -120 dBm/SCS |
| 7.6.14.2 NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation | During T1 – T2: Ês1: -89dBm/120kHz Ês2: -infinity During T3: Ês1: -89dBm/120kHz Ês2: -89dBm/120kHz Signalled A3-Offset: -11 dB |  | During T1 – T2: 0dB 0dB During T3: 0dB 0dB 0 dB | During T1 – T2: Ês1: -89dBm/120kHz Ês2: -infinity During T3: Ês1: -89dBm/120kHz Ês2: -89dBm/120kHz Signalled A3-Offset: -11 dB |
| 7.6.15.1 NR SA FR2 SA event triggered reporting tests with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements | During T1: Ês1: -87dBm/120kHz Ês2: -infinity Ês3: -infinity During T2: Ês1: -87dBm/120kHz Ês2: -87dBm/120kHz Ês3: -87dBm/120kHz Signalled offsetMO: 16 dB Signalled A3-Offset: -11 dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB Signalled offsetMO: 0dB Signalled A3-offset: 0 dB | During T1: Ês1: -87dBm/120kHz Ês2: -infinity Ês3: -infinity During T2: Ês1: -87dBm/120kHz Ês2: -87dBm/120kHz Ês3: -87dBm/120kHz Signalled offsetMO: 16dB Signalled A3-offset: -11dB |
| 7.6.15.2 NR SA FR2 event triggered reporting tests with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2) | Same as 7.6.15.1 |  | Same as 7.6.15.1 | Same as 7.6.15.1 |
| 7.6.15.3 NR SA FR2 event triggered reporting tests with concurrent measurement gaps with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement | During T1: Noc1: -102.0dBm/15kHz Noc2: -104.7dBm/15kHz Noc3: -102.0dBm/15kHz SSB Ês1: -89.7dBm/120kHz SSB Ês2: -infinity SSB Ês3: -infinity PRS Ês1: -infinity PRS Ês3: -infinity During T2: Noc1: -102.0dBm/15kHz Noc2: -104.7dBm/15kHz Noc3: -102.0dBm/15kHz SSB Ês1: -89.7dBm/120kHz SSB Ês2: -86.7dBm/120kHz SSB Ês3: -86.7dBm/120kHz PRS Ês1: -96dBm/120kHz PRS Ês3: -103dBm/120kHz Signalled A3-Offset: -6 dB |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB -0.6dB 0dB 0.2dB Signalled A3-offset: -6 dB | During T1: Noc1: -102.0dBm/15kHz Noc2: -104.7dBm/15kHz Noc3: -102.0dBm/15kHz SSB Ês1: -89.7dBm/120kHz SSB Ês2: -infinity SSB Ês3: -infinity PRS Ês1: -infinity PRS Ês3: -infinity During T2: Noc1: -102.0dBm/15kHz Noc2: -104.7dBm/15kHz Noc3: -102.0dBm/15kHz SSB Ês1: -89.7dBm/120kHz SSB Ês2: -86.7dBm/120kHz SSB Ês3: -87.3dBm/120kHz PRS Ês1: -96dBm/120kHz PRS Ês3: -102.8dBm/120kHz Signalled A3-offset: -12dB |
| 7.6.16.1 NR SA FR2 event triggered reporting tests with per-UE NCSG under non-DRX | Same as 7.6.1.1 |  | Same as 7.6.1.1 | Same as 7.6.1.1 |
| 7.6.16.2 NR SA FR2 event triggered reporting tests on inter-frequency measurement with NCSG when DRX is not used(PCell in FR2) | During T1: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +6 dB Ês2 / Noc2: -infinity dB During T2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +6 dB Ês2 / Noc2: +9 dB Signalled offsetMO: 16 dB Signalled A3-Offset: -11 dB |  | During T1: 0 dB 0 dB 2.7 dB 0 dB During T2: 0 dB 0 dB 2.7 dB 0 dB 0 dB 0 dB | During T1: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +8.7 dB Ês2 / Noc2: -infinity dB During T2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +8.7 dB Ês2 / Noc2: +9 dB Signalled offsetMO: 16 dB Signalled A3-Offset: -11 dB |
| 7.6.16.3 NR SA FR2 event triggered reporting tests on deactivated SCell measurement via NCSG in non-DRX | During T1: Ês2: -89dBm/120kHz Ês3: -infinity During T2: Ês2: -89dBm/120kHz Ês3: -89dBm/120kHz Signalled A3-Offset: -11 dB |  | During T1: 0 dB 0 dB During T2: 0 dB 0 dB 0 dB | During T1: Ês2: -89dBm/120kHz Ês3: -infinity During T2: Ês2: -89dBm/120kHz Ês3: -89dBm/120kHz Signalled A3-Offset: -11 dB |
| 7.6.20.1 NR SA FR2 Intra-frequency SSB based L1-RSRP measurement | Signalled A3-Offset: -15dB During T1: Ês/IotBB1: -1.5dB Ês/IotBB2: -1.5dB During T2: Ês/IotBB1: +16.5dB Ês/IotBB2: +16.5dB |  | 0dB During T1: 1.35dB 1.35dB During T2: 0dB 0dB | Signalled A3-Offset: -15dB During T1: Ês/IotBB1: -0.15dB Ês/IotBB2: -0.15dB During T2: Ês/IotBB1: +16.5dB Ês/IotBB2: +16.5dB |
| 7.6.21.1 NR SA FR2 Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM | Signalled A3-Offset: -15dB Signalled Cell 2 offsetMO: 16dB During T1: Ês/IotBB1: 16.5dB Ês/IotBB2: -1.5dB During T2: Ês/IotBB1: +16.5dB Ês/IotBB2: +16.5dB |  | 0dB 0dB During T1: 0dB 6dB During T2: 0dB 0dB | Signalled A3-Offset: -15dB Signalled Cell 2 offsetMO: 16dB During T1: Ês/IotBB1: 16.5dB Ês/IotBB2: 4.5dB During T2: Ês/IotBB1: +16.5dB Ês/IotBB2: +16.5dB |
| 7.7.1.1 NR SA FR2 SS-RSRP measurement accuracy | UE PC1 |  |  |  |
|  | Test 1: Noc: -91.60dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês2: -108.2dBm/120kHz n257 Ês3: -108.2dBm/120kHz n260 Ês2: -105.2dBm/120kHz n260 Ês3: -105.2dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally |  | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 6dB 6dB 6dB 6dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB Cell 2: RSRP_61 to RSRP_126 Cell 3: RSRP_56 to RSRP_126 Cell 3: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês2: -102.2dBm/120kHz n257 Ês3: -102.2dBm/120kHz n260 Ês2: -99.2dBm/120kHz n260 Ês3: -99.2dBm/120kHz Band dependent. See test case tables in clause 5.7.1.1.5 |
|  | UE PC3 |  |  |  |
|  | Test 1: Noc: -91.60dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês1: -110.0dBm/120kHz n257 Ês2: -110.0dBm/120kHz n260 Ês1: -107.4dBm/120kHz n260 Ês2: -107.4dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally |  | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 7.70dB 7.70dB 7.70dB 7.70dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB Cell 1: RSRP_50 to RSRP_108 Cell 2: RSRP_46 to RSRP_103 Cell 2: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês1: -102.3dBm/120kHz n257 Ês2: -102.3dBm/120kHz n260 Ês1: -99.7dBm/120kHz n260 Ês2: -99.7dBm/120kHz Band dependent. See test case tables in clause 5.7.1.1.5 |
| 7.7.1.2 NR SA FR2-FR2 SS-RSRP measurement accuracy | Test 1 Config 1: Noc1: -90.60dBm/15kHz Noc2: -90.60dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 1: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 1 Config 2: Noc1: -93.70dBm/15kHz Noc2: -93.70dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 2: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally |  | Test 1 Config 1: -5.70dB -5.70dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 1: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping Test 1 Config 2: -5.60dB -5.60dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 2: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping | Test 1 Config 1: Noc1: -96.30dBm/15kHz Noc2: -96.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 1 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 2: RSRP_52 to RSRP_109 Cell 2: n257: RSRP_x-23 to RSRP_x+26 n260: RSRP_x-23 to RSRP_x+27 Test 2 Config 1: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 1 n257: RSRP_33 to RSRP_101 n260: RSRP_34 to RSRP_104 Cell 2: n257: RSRP_32 to RSRP_87 n260: RSRP_34 to RSRP_90 Cell 2: n257: RSRP_x-37 to RSRP_x+17 n260: RSRP_x-37 to RSRP_x+19 Test 1 Config 2: Noc1: -99.30dBm/15kHz Noc2: -99.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 1 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 2: RSRP_52 to RSRP_109 Cell 2: n257: RSRP_x-23 to RSRP_x+26 n260: RSRP_x-23 to RSRP_x+27 Test 2 Config 2: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 1 n257: RSRP_36 to RSRP_104 n260: RSRP_37 to RSRP_107 Cell 2: n257: RSRP_35 to RSRP_90 n260: RSRP_37 to RSRP_93 Cell 2: n257: RSRP_x-37 to RSRP_x+17 n260: RSRP_x-37 to RSRP_x+19 |
| 7.7.1.3 NR SA FR1-FR2 SS-RSRP measurement accuracy | Test 1: Noc: -90dBm/15kHz Ês2 / Noc: +5.0dB Reported absolute RSRP values: ±8dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês2: -98.2dBm/120kHz n260 Ês2: -93.9dBm/120kHz Reported absolute RSRP values: ±8dBFor extreme conditions allow ±3dB+ FFS MU additionally |  | Test 1: -5.50dB 0dB Via Mapping Test 2: 5.50dB 5.50dB Via Mapping | Test 1: Noc: -95.50dBm/15kHz Ês2 / Noc: +5dB Band dependent. See test case tables in clause 7.7.1.3.5 Test 2: n257 Ês2: -92.7dBm/120kHz n260 Ês2: -88.4dBm/120kHz Band dependent. See test case tables in clause 7.7.1.3.5 |
| 7.7.2.1 NR SA FR2 SS-RSRQ measurement accuracy | Test 1: Noc: -95.0dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Test 2: Noc: -95.0dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally |  | Test 1: -5.70dB 0dB 0dB Via Mapping Test 2: -1.70dB 0dB 0dB Via Mapping | Test 1: Noc: -100.70dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Cell 2: All bands: RSRQ_41 to RSRQ_73 Test 2: Noc: -96.70dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Cell 2: n257, n258, n261: RSRQ_35 to RSRQ_71 n260: RSRQ_34 to RSRQ_71 |
| 7.7.2.2 NR SA FR2-FR2 SS-RSRQ measurement accuracy | Test 1: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Reported relative RSRQ values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Test 2: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported relative RSRQ values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally |  | Test 1: -1.90dB -1.90dB 0dB 0dB Via Mapping Via Mapping Test 2: -1.41dB -1.41dB 0dB 0dB Via Mapping Via Mapping | Test 1: Noc1: -95.93dBm/15kHz Noc2: -95.93dBm /15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Cell 1 and Cell 2: RSRQ_41 to RSRQ_73 Cell 2: RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -95.44dBm/15kHz Noc2: -95.44dBm /15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Cell 1 and Cell 2: RSRQ_37 to RSRQ_74 Cell 2: RSRQ_x-9 to RSRQ_x+9 |
| 7.7.3.1 NR SA FR2 SS-SINR measurement accuracy | Test 1: Noc: -105.0dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc: -105.0dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally |  | Test 1: 0dB 0dB 0dB Via Mapping Test 2: 0dB 0.2dB 0.2dB Via Mapping | Test 1: Noc: -105.0dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Cell 2: n257, n258, n261: SINR_22 to RSRQ_58 n260: SINR_21 to RSRQ_58 Test 2: Noc: -105.0dBm/15kHz Ês1 / Noc: -2.8dB Ês2 / Noc: -2.8dB Cell 2: n257, n258, n261: SINR_18 to RSRQ_55 n260: SINR_18 to RSRQ_54 |
| 7.7.3.2 NR SA FR2-FR2 SS-SINR measurement accuracy | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported absolute SINR values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally |  | Test 1: 0dB 0dB 0dB 0dB Via Mapping Via Mapping Test 2: -0.1dB -0.1dB 0dB 0dB Via Mapping Via Mapping Test 3: 0dB 0dB 0.9dB 0.9dB Via Mapping Via Mapping | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Cell 1 and Cell 2: n257, n258, n261: SINR_27 to SINR_62 n260: SINR_27 to SINR_61 Cell 2: SINR_x-8 to SINR_x+8 Test 2: Noc1: -105.1dBm/15kHz Noc2: -105.1dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Cell 1 and Cell 2: n257, n258, n261: SINR_48 to SINR_87 n260: SINR_48 to SINR_86 Cell 2: SINR_x-17 to SINR_x+17 Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -2.1dB Ês2 / Noc2: -2.1dB Cell 1 and Cell 2: n257, n258, n261: SINR_23 to SINR_60 n260: SINR_22 to SINR_59 Cell 2: SINR_x-9 to SINR_x+9 |
| 7.7.4.1 NR SA FR2 SSB based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 SSB#0 Ês: -108.1dBm/120kHz n257 SSB#1 Ês: -105.5dBm/120kHz n260 SSB#0 Ês: -108.1dBm/120kHz n260 SSB#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally |  | Test 1: -4.1dB 0dB 0.4dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -1.6dB SSB#0: RSRP_42 to RSRP_101 SSB#1: RSRP_31 to RSRP_89 SSB#1-SSB#0: -9 to -2 Test 2: n257 SSB#0 Ês: -102.4dBm/120kHz n257 SSB#1 Ês: -99.8dBm/120kHz n260 SSB#0 Ês: -102.4dBm/120kHz n260 SSB#1 Ês: -99.8dBm/120kHz SSB#0 and SSB#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 SSB#1-SSB#0: -4 to +4 |
| 7.7.4.2 NR SA FR2 CSI-RS based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 CSI-RS#0 Ês: -108.1dBm/120kHz n257 CSI-RS#1 Ês: -105.5dBm/120kHz n260 CSI-RS#0 Ês: -108.1dBm/120kHz n260 CSI-RS#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally |  | Test 1: -4.1dB 0dB 0.2dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -1.8dB CSI-RS#0: RSRP_42 to RSRP_101 CSI-RS#1: RSRP_30 to RSRP_89 CSI-RS#1-CSI-RS#0: -9 to -2 Test 2: n257 CSI-RS#0 Ês: -102.4dBm/120kHz n257 CSI-RS#1 Ês: -99.8dBm/120kHz n260 CSI-RS#0 Ês: -102.4dBm/120kHz n260 CSI-RS#1 Ês: -99.8dBm/120kHz CSI-RS#0 and CSI-RS#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 CSI-RS#1-CSI-RS#0: -4 to +4 |
| 7.7.5.1 NR SA FR2 SRS-RSRP measurement accuracy | Same as 5.7.5.1 |  | Same as 5.7.5.1 | Same as 5.7.5.1 |
| 7.7.6.1 NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Noc: -100dBm/15kHz Ês0 / Noc: 10.00dB Ês1 / Noc: -2.00dB Reported CSI-SINR values ± 5.5dB Reported Differential CSI-SINR values ± 4.5dB |  | -4.1dB 0dB 0.2dB Via mapping | Noc: -104.1dBm/15kHz Ês0 / Noc: 10.00dB Ês1 / Noc: -1.8dB CSI-RS#0: SINR_53 to SINR_76 CSI-RS#0: DIFFSINR_6 to DIFFSINR_15 |
| 7.7.6.2 NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | Noc: -100dBm/15kHz Ês0 / Noc: 10.00dB Ês1 / Noc: -2.00dB Reported SS-SINR values ± 4.5dB Reported Differential SS-SINR values ± 3.5dB |  | -4.1dB 0dB 0.2dB Via mapping | Noc: -104.1dBm/15kHz Ês0 / Noc: 10.00dB Ês1 / Noc: -1.8dB SSB#0: SINR_55 to SINR_74 SSB#0: DIFFSINR_7 to DIFFSINR_15 |
| 7.7.6.3 NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Noc: -100dBm/15kHz Ês0 / Noc: 10.00dB Ês1 / Noc: 0.00dB Reported CSI-SINR values ± 4.5dB Reported Differential CSI-SINR values ± 3.5dB |  | -4.1dB 0dB 0.2dB Via mapping | Noc: -104.1dBm/15kHz Ês0 / Noc: 10.00dB Ês1 / Noc: 0.2dB CSI-RS#0: SINR_55 to SINR_74 CSI-RS#0: DIFFSINR_5 to DIFFSINR_13 |
| 7.7.8.1 NR SA intra-frequency FR2 CSI-RSRQ measurement accuracy | Same as 5.7.8.1 |  | Same as 5.7.8.1 | Same as 5.7.8.1 |
| 7.7.8.2 NR SA inter-frequency FR2 CSI-RSRQ measurement accuracy | Same as 5.7.8.2 |  | Same as 5.7.8.2 | Same as 5.7.8.2 |
| 7.7.9.1 NR SA intra-frequency FR2 CSI-SINR measurement accuracy | Same as 5.7.9.1 |  | Same as 5.7.9.1 | Same as 5.7.9.1 |
| 7.7.9.2 NR SA inter-frequency FR2 CSI-SINR measurement accuracy | Same as 5.7.9.2 |  | Same as 5.7.9.2 | Same as 5.7.9.2 |
| 7.7.15.1 NR SA FR2 SSB based inter-frequency L1-RSRP measurement | Test configuration 1 Signalled a4-Threshold: -120dBm Signalled offsetMO: 16dB Test 1: Noc1: -100dBm/15kHz Noc2: -100dBm/15kHz Ês1/Noc1 : +10.0dB Ês2/Noc2: -2.0dB Reported absolute L1-RSRP values: Normal condition: ±8.5dB Extreme condition: ±11.5dB+FFS MU Test 2: n257 Ês1: -109.1dBm/120kHz n259 Ês1: -105.5dBm/120kHz n260 Ês1: -106.5dBm/120kHz n257 Ês2: -109.1dBm/120kHz n259 Ês2: -105.5dBm/120kHz n260 Ês2: -106.5dBm/120kHz Reported absolute L1-RSRP values: Normal condition: ±6.5dB/±8.5dB Extreme condition: (±9.5dB/±11.5dB) + FFS MU |  | Test configuraiton 1 0dB 0dB Test 1: -4.1dB 0dB 0dB 0dB Via mapping Test 2: 3.65dB 3.65dB 3.65dB 6.65dB 6.65dB 6.65dB Via mapping | Test configuaiton 1 Signalled a4-Threshold: -120dBm Signalled offsetMO: 16dB Test 1: Noc1: -104.1dBm/15kHz Noc2: -100dBm/15kHz Ês1/Noc1: +10.0dB Ês2/Noc2: -2dB Normal condition: RSRP_39 to RSRP_98 Test 2: n257 Ês1: -105.45dBm/120kHz n259 Ês1: -101.85dBm/120kHz n260 Ês1: -102.85dBm/120kHz n257 Ês2: -102.45dBm/120kHz n259 Ês2: -98.85dBm/120kHz n260 Ês2: -99.85dBm/120kHz Normal condition: n257: RSRP_32 to RSRP_88 n259: RSRP_36 to RSRP_92 n260: RSRP_35 to RSRP_91 |
|  | Test configuration 2 Signalled a4-Threshold: -120dBm Signalled offsetMO: 16dB Test 1: Noc1: -100dBm/15kHz Noc2: -100dBm/15kHz Ês1/Noc1: +10.0dB Ês2/Noc2: -2.0dB Reported absolute L1-RSRP values: Normal condition: ±8.5dB Extreme condition: ±11.5dB+FFS MU Test 2: n257 Ês1: -106.1dBm/240kHz n259 Ês1: -102.5dBm/240kHz n260 Ês1: -103.5dBm/240kHz n257 Ês2: -106.1dBm/240kHz n259 Ês2: -102.5dBm/240kHz n260 Ês2: -103.5dBm/240kHz Reported absolute L1-RSRP values: Normal condition: ±6.5dB/±8.5dB Extreme condition: (±9.5dB/±11.5dB) + FFS MU |  | Test configuraiton 2 0dB 0dB Test 1: -4.1dB 0dB 0dB 0dB Via mapping Test 2: 3.65dB 3.65dB 3.65dB 6.65dB 6.65dB 6.65dB Via mapping | Test configuaiton 2 Signalled a4-Threshold: -120dBm Signalled offsetMO: 16dB Test 1: Noc1: -104.1dBm/15kHz Noc2: -100dBm/15kHz Ês1/Noc1: +10.0dB Ês2/Noc2: -2dB Normal condition: RSRP_42 to RSRP_101 Test 2: n257 Ês1: -102.45dBm/240kHz n259 Ês1: -98.85dBm/240kHz n260 Ês1: -99.85dBm/240kHz n257 Ês2: -99.45dBm/240kHz n259 Ês2: -95.85dBm/240kHz n260 Ês2: -96.85dBm/240kHz Normal condition: n257: RSRP_35 to RSRP_91 n259: RSRP_39 to RSRP_95 n260: RSRP_38 to RSRP_94 |

Table F.1.3.2-5: Derivation of test requirements for E-UTRA – NR inter-RAT test cases with E-UTRA serving cell RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 8.2.1.1 E-UTRA – NR FR1 cell re-selection to higher priority NR target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -1.90dB 0dB 0.35dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 1.55dB 1.55dB | During T1: Noc1: -98dBm/15kHz Noc2: -99.90dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -3.65dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.55dB Ês2 / Noc2: +13.55dB |
| 8.2.1.2 E-UTRA – NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16 | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: 14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB | During T1: -1.90dB -1.90dB 0.35dB 3.45dB During T2: -1.90dB -1.90dB 3.45dB 0dB | During T1: Noc1: -99.9dBm/15kHz Noc2: -99.9dBm/15kHz Ês1 / Noc1: -3.65dB Ês2 / Noc2: 17.45dB During T2: Noc1: -99.9dBm/15kHz Noc2: -99.9dBm/15kHz Ês1 / Noc1: +15.45dB Ês2 / Noc2: +14dB |
| 8.2.2.1 E-UTRA – NR FR1 Early Measurement Reporting | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB Reported SS-RSRP and SS-RSRQ for NR Cell 2 | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB +0.3dB During T3: 0dB 0dB 0dB +0.3dB Via mapping | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -3.7dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -3.7dB Configuration 1 and 2: RSRP_44 to RSRP_66 RSRQ_44 to RSRQ_65 Configuration 3: RSRP_47 to RSRP_69 RSRQ_44 to RSRQ_65 |
| 8.2.2.2 E-UTRA – NR FR2 Early Measurement Reporting | During T1: Noc1: -98dBm/15kHz Noc2: -107dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB Signalled Thresh_X_HighP: 48dB Reported SS-RSRP and SS-RSRQ for NR Cell 2 | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB +2.6dB During T3: 0dB 0dB 0dB +2.6dB +3dB Via mapping | During T1: Noc1: -98dBm/15kHz Noc2: -107dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -1.4dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -1.4dB 51dB RSRP_28 to RSRP_86 RSRQ_44 to RSRQ_65 |
| 8.3.1.1 E-UTRA – NR FR1 handover with known target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB -1.55dB +1.55dB During T3: 0dB 0dB -1.55dB +1.55dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5.45dB Ês2 / Noc2: +1.55dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5.45dB Ês2 / Noc2: +1.55dB |
| 8.4.1.1 E-UTRA – NR FR1 SFTD measurement delay in non-DRX | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +4dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +4dB |
| 8.4.1.2 E-UTRA – NR FR1 SFTD measurement delay in DRX | Same as 8.4.1.1 | Same as 8.4.1.1 | Same as 8.4.1.1 |
| 8.4.2.1 E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +7dB | During T1: 0dB0dB0dB0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +7dB |
| 8.4.2.2 E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.3 E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.4 E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.5 E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX | During T1: Ês2: - Infinity During T2: Ês2: -80.6 dBm/120kHz In signalling: b1-ThresholdNR: -112 dBm/SCS | During T1: 0dB During T2: 0dB In signalling: -6dB | During T1: Ês2: - Infinity During T2: Ês2: -80.6 dBm/120kHz In signalling: b1-ThresholdNR: -118 dBm/SCS |
| 8.4.2.6 E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX | During T1: Noc: -104.7dBm/15kHz Ês / Noc: -infinity During T2: Noc: -104.7dBm/15kHz Ês / Noc: +8dB | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Noc: -104.7dBm/15kHz Ês / Noc: -infinity During T2: Noc: -104.7dBm/15kHz Ês / Noc: +8dB |
| 8.4.2.7 E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX | Same as 8.4.2.6 | Same as 8.4.2.6 | Same as 8.4.2.6 |
| 8.4.2.8 E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX | Same as 8.4.2.6 | Same as 8.4.2.6 | Same as 8.4.2.6 |
| 8.4.2.9 E-UTRA – NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16 | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +7dB |
| 8.5.1.1 E-UTRA – NR FR1 SFTD measurement accuracy | Test 1 Ês1 / Noc1: -3dB Ês2 / Noc2: -3dB | Test 1: +0.8dB +0.3dB | Test 1: Ês1 / Noc1: -2.2dB Ês2 / Noc2: -2.7dB |
| 8.5.2.1.1.1 E-UTRA SS-RSRP absolute measurement accuracy of a NR FR1 neighbour cell |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Reported RSRP values: ±8dB (±3.0dB additionally for extreme conditions) Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4.0dB Reported RSRP values: ±4.5dB (±4.5dB additionally for extreme conditions) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB RSRP_62 to RSRP_82 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB RSRP_30 to RSRP_43 RSRP_31 to RSRP_43 RSRP_31 to RSRP_44 RSRP_32 to RSRP_44 RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 depending on operating band |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Reported RSRP values: ±8dB (±3.0dB additionally for extreme conditions) Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4.0dB Reported RSRP values: ±4.5dB (±4.5dB additionally for extreme conditions) | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96.00dBm/15kHz Ês1 / Noc1: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |
| 8.5.2.1.2 E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell | Test 1: Noc: -105dBm/15kHz Ês / Noc 11dB Reported RSRP values: ±8dB (±3dB additionally for extreme conditions) Test 2: Ês FR2a PC3: -109.1dBm/SCS Reported RSRP values: ±8dB Ês FR2b PC3: -106.5dBm/SCS Reported RSRP values: ±8dB Ês FR2c PC3: -105.5dBm/SCS Reported RSRP values: ±8dB (±3dB additionally for extreme conditions in all frequency ranges) | Test 1: -0.1dB 0dB Via mapping Test 2: +5.65dB Via mapping +5.65dB Via mapping +5.65dB Via mapping | Test 1: Noc: -105.1dBm/15kHz Ês / Noc: +11.0dB RSRP_48 to RSRP_105 Test 2: Ês: -103.45dBm/SCS RSRP_29 to RSRP_87 Ês: -100.85dBm/SCS RSRP_32 to RSRP_89 Ês: -99.85dBm/SCS RSRP_33 to RSRP_90 |
| 8.5.2.2.1 E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc1: -106dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) | Test 1: -1.5dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0dB Via mapping | Test 1: Noc1: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 2: Noc1: -106dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 |
| Test Configuration 3,6 | Test 1: Noc1: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc1: -113dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) | Test 1: -1.53dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0dB Via mapping | Test 1: Noc1: -87.80dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 2: Noc1: -113dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 |
| 8.5.2.2.2 E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR2 neighbour cell | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc1 FR2a PC3: -115.1dBm/15kHz Noc1 FR2b PC3: -112.5dBm/15kHz Noc1 FR2c PC3: -111.5dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions in all frequency ranges) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 5.3dB Via mapping | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB RSRQ_23 to RSRQ_95 Test 2: Noc1: -115.1dBm/15kHz Noc1: -112.5dBm/15kHz Noc1: -111.5dBm/SCS Ês1 / Noc1: 3.55dB RSRQ_26 to RSRQ_98 |
| 8.5.2.3.1 E-UTRA SS-SINR absolute measurement accuracy of a NR FR1 neighbour cell | Test 1: Noc1: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Ês1 / Noc1: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Reported SINR values: ±3.5dB | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Ês1 / Noc1: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Ês1 / Noc1: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB SINR_32 to SINR_49 |
| 8.5.2.3.2 E-UTRA SS-SINR absolute measurement accuracy of a NR FR2 neighbour cell | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB Reported SINR values: ±3dB (±1dB additionally for extreme conditions) Test 2: Noc1: -105dBm/15kHz Ês1 / Noc1: 11dB Reported SINR values: ±3dB (±1dB additionally for extreme conditions) Test 3: Noc1 FR2a PC3: -115.1dBm/15kHz Noc1 FR2b PC3: -112.5dBm/15kHz Noc1 FR2c PC3: -111.5dBm/15kHz Ês1 / Noc1: -1dB Reported SINR values: ±3dB (±1dB additionally for extreme conditions in all frequency ranges) | Test 1: 0dB 0dB Via mapping Test 2: -0.1dB 0dB Via mapping Test 3: 0dB 0dB 0dB 4.6dB Via mapping | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB SINR_9 to SINR_82 Test 2: Noc1: -105.1dBm/15kHz Ês1 / Noc1: 11dB SINR_32 to SINR_105 Test 3: Noc1: -115.1dBm/15kHz Noc1: -112.5dBm/15kHz Noc1: -111.5dBm/SCS Ês1 / Noc1: 3.6dB SINR_17 to SINR_90 |

Table F.1.3.2-6: Derivation of test requirements for FR1 NR sidelink RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 9.1.1.1 NR SA FR1 UE transmit timing accuracy for GNSS as synchronization reference source | Sidelink transmit timing error limit: ±12*64 Tc | ±3*64 Tc | ±15*64 Tc |
| 9.1.1.2 NR SA FR1 UE transmit timing accuracy for SyncRef UE as synchronization reference source | Sidelink transmit timing error limit: ±8*64 Tc Noc = -95 dBm/30 kHz Es1/Noc = 3 dB | ±2*64 Tc 0dB 0dB | ±10*64 Tc Noc = -95 dBm/30 kHz Es1/Noc = 3 dB |
| 9.1.1.3 NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source | Uu Test Configuration 1, 2 |  |  |
|  | Sidelink transmit timing error limit: ±12*64 Tc Noc = -98 dBm/15 kHz Es1/Noc = 3 dB | ±3*64 Tc 0dB 0dB | ±15*64 Tc Noc = -98 dBm/15 kHz Es1/Noc = 3 dB |
|  | Uu Test Configuration 3 |  |  |
|  | Sidelink transmit timing error limit: ±10*64 Tc Noc = -98 dBm/15 kHz Es1/Noc = 3 dB | ±3*64 Tc 0dB 0dB | ±13*64 Tc Noc = -98 dBm/15 kHz Es1/Noc = 3 dB |
| 9.1.2.1 NR SA FR1 initiation/cease of S-SSB transmission for FR1 NR cell as synchronization reference source | Uu Test Configuration 1, 2 |  |  |
|  | During T1: Noc = -110 dBm/15 kHz Es1/Noc = 4.50 dB During T2: Noc = -110 dBm/15 kHz Es1/Noc = -4.50 dB During T3: Noc = -110 dBm/15 kHz Es1/Noc = 4.50 dB In signalling: syncTxThreshIC = -110.00 dBm/SCS | -1.55dB 3.10dB -1.55dB 0dB -1.55dB 3.10dB 0dB | During T1: Noc = -111.55 dBm/15 kHz Es1/Noc = 7.60 dB During T2: Noc = -111.55 dBm/15 kHz Es1/Noc = -4.50 dB During T3: Noc = -111.55 dBm/15 kHz Es1/Noc = 7.60 dB syncTxThreshIC = -110.00 dBm/SCS |
|  | Uu Test Configuration 3 |  |  |
|  | During T1: Noc = -110 dBm/15 kHz Es1/Noc = 4.50 dB During T2: Noc = -110 dBm/15 kHz Es1/Noc = -4.50 dB During T3: Noc = -110 dBm/15 kHz Es1/Noc = 4.50 dB In signalling: syncTxThreshIC = -110.00 dBm/SCS | -4.55dB 3.10dB -4.55dB 0dB -4.55dB 3.10dB 0dB | During T1: Noc = -114.55 dBm/15 kHz Es1/Noc = 7.60 dB During T2: Noc = -114.55 dBm/15 kHz Es1/Noc = -4.50 dB During T3: Noc = -114.55 dBm/15 kHz Es1/Noc = 7.60 dB syncTxThreshIC = -110.00 dBm/SCS |
| 9.1.2.2 NR SA FR1 initiation/cease of S-SSB transmission for SyncRef UE as synchronization reference source | During T1: Noc = -98 dBm/30 kHz Es1/Noc = 5.50 dB During T2: Noc = -98 dBm/30 kHz Es1/Noc = -3.50 dB During T3: Noc = -98 dBm/30 kHz Es1/Noc = 5.50 dB In signalling: syncTxThreshOoC = -100.00 dBm/SCS | -1.35dB 0dB -4.55dB 0dB -1.35dB 0dB 0dB | During T1: Noc = -99.35 dBm/30 kHz Es1/Noc = 5.50 dB During T2: Noc = -102.55 dBm/30 kHz Es1/Noc = -3.50 dB During T3: Noc = -99.35 dBm/30 kHz Es1/Noc = 5.50 dB syncTxThreshOoC = -100.00 dBm/SCS |
| 9.1.3.1 NR SA FR1 synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source | During T1: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = -infinity Es3/Noc = -infinity During T2: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = 0 dB Es3/Noc = -infinity During T3: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = 0 dB Es3/Noc = 3 dB | 0dB 0.35dB N/A N/A 0dB 0dB 0.35dB N/A 0dB 0dB 0dB 0.35dB | During T1: Noc = -95 dBm/30 kHz Es1/Noc = 0.35 dB Es2/Noc = -infinity Es3/Noc = -infinity During T2: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = 0.35 dB Es3/Noc = -infinity During T3: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = 0 dB Es3/Noc = 3.35 dB |
| 9.1.3.2 NR SA FR1 synchronization reference selection/reselection for FR1 NR Cell configured as the highest priority synchronization reference source | During T1: Noc = -95 dBm/30 kHz Es1/Noc = -infinity Es2/Noc = -infinity During T2: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = -infinity During T3: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = 0 dB | 0dB N/A N/A 0dB 0.35dB N/A 0dB 0dB 0.35dB | During T1: Noc = -95 dBm/30 kHz Es1/Noc = -infinity Es2/Noc = -infinity During T2: Noc = -95 dBm/30 kHz Es1/Noc = 0.35 dB Es2/Noc = -infinity During T3: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = 0.35 dB |
| 9.1.4.1 NR SA FR1 L1 SL-RSRP measurement for autonomous resource selection/reselection | During T1: Noc = -110 dBm/30 kHz Esi/Noc = 10 dB, i = 20~29 Esi/Noc = 0 dB, i = 0~19, 30~49 During T2: Noc = -121 dBm/30 kHz Esi/Noc = 10 dB, i = 20~29 Esi/Noc = 20 dB, i = 0~19, 30~49 In signalling: SL-Thres-RSRP = -106 dBm/SCS | 0dB 10.55dB 0.35dB 0dB 0dB 10.55dB 6dB | During T1: Noc = -110 dBm/30 kHz Esi/Noc = 20.55 dB, i = 20~29 Esi/Noc = 0.35 dB, i = 0~19, 30~49 During T2: Noc = -121 dBm/30 kHz Esi/Noc = 10 dB, i = 20~29 Esi/Noc = 30.55 dB, i = 0~19, 30~49 In signalling: SL-Thres-RSRP = -100 dBm/SCS |
| 9.1.4.2 NR SA FR1 L1 SL-RSRP measurement for resource pre-emption | During T1: Noc = -infinity Es1/Noc = -infinity During T2: Noc = -100 dBm/30 kHz Es1/Noc = 5 dB In signalling: SL-Thres-RSRP = -106 dBm/SCS | N/A N/A 0dB 0dB 0dB | During T1: Noc = -infinity Es1/Noc = -infinity During T2: Noc = -100 dBm/30 kHz Es1/Noc = 5 dB In signalling: SL-Thres-RSRP = -106 dBm/SCS |
| 9.1.4.3 NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation | During T1: Noc = -103 dBm/30 kHz Esi/Noc = 12dB, i = 0~29 Esi/Noc = 2 dB, i = 30~49 Esi/Noc = 25.50 dB, i = 50~64, 85~99 Esi/Noc = 12 dB, i = 65~84 Esi/Noc = -infinity, i = 100~129 During T2: Noc = -103 dBm/30 kHz Esi/Noc = -infinity, i = 0~29 Esi/Noc = 2 dB, i = 30~49 Esi/Noc = 25.5 dB, i = 50~64, 85~99 Esi/Noc = 12 dB, i = 65~84 Esi/Noc = 25.5dB, i = 100~129 In signalling: SL-Thres-RSRP = -96 dBm/SCS | -7.05dB 2.1dB 0dB 7.1dB 2.1dB N/A -7.05dB N/A 0dB 7.1dB 2.1dB 7.55dB -6dB | During T1: Noc = -110.05 dBm/30 kHz Esi/Noc = 14.1 dB, i = 0~29 Esi/Noc = 2 dB, i = 30~49 Esi/Noc = 32.60 dB, i = 50~64, 85~99 Esi/Noc = 14.1 dB, i = 65~84 Esi/Noc = -infinity, i = 100~129 During T2: Noc = -110.05 dBm/30 kHz Esi/Noc = -infinity, i = 0~29 Esi/Noc = 2 dB, i = 30~49 Esi/Noc = 32.60 dB, i = 50~64, 85~99 Esi/Noc = 14.1 dB, i = 65~84 Esi/Noc = 33.05dB, i = 100~129 In signalling: SL-Thres-RSRP = -102 dBm/SCS |
| 9.1.5.1 NR SA FR1 congestion control measurement for concurrent operation | During T1: Noc = -103 dBm/30kHz Esi/Noc = 4.35 dB, i = 0,1,2,3 During T2: Noc = -103 dBm/30kHz Esi/Noc = 10.32 dB, i = 0,1,2,3 In signalling: threshS-RSSI-CBR = -74 dBm/subchannel | -1.55dB 0dB -1.55dB 3.30dB 0dB | During T1: Noc = -104.55 dBm/30kHz Esi/Noc = 4.35 dB, i = 0,1,2,3 During T2: Noc = -104.55 dBm/30kHz Esi/Noc = 13.62 dB, i = 0,1,2,3 In signalling: threshS-RSSI-CBR = -74 dBm/subchannel |
| 9.1.5.2 NR SA FR1 congestion control measurement for PC5-only operation | Same as 9.1.5.1 | Same as 9.1.5.1 | Same as 9.1.5.1 |
| 9.1.6.1 NR SA FR1 interruption to WAN due to NR sidelink communication | N/A | N/A | N/A |

Table F.1.3.2-7: Derivation of test requirements for NR SA FR1 RRM tests for RedCap

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 16.1.1.1 NR SA FR1 Cell reselection for 1 Rx UE | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB Qoffsets, n = 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB In signalling -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB Qoffsets, n = -1dB |
| 16.1.1.2 NR SA FR1 Cell reselection for 2 Rx UE | Same as 6.1.1.1 | Same as 6.1.1.1 | Same as 6.1.1.1 |
| 16.1.1.3 NR SA FR1-FR1 Cell reselection for 1 Rx UE | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -4dB 1.6dB 0.3dB During T2: 0dB 0dB 1.6dB 0dB During T3: 0dB 0dB 1.6dB 1.6dB | During T1: Noc1: -98dBm/15kHz Noc2: -102dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.7dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB |
| 16.1.1.4 NR SA FR1-FR1 Cell reselection for 2 Rx UE | Same as 6.1.1.2 | Same as 6.1.1.2 | Same as 6.1.1.2 |
| 16.1.1.5 NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB In signalling: Qoffsets, n = 0dB SintrasearchP = 60dB SSearchDeltaP = 3dB | During T1: 0dB 0dB 0.45dB During T2: 0dB 0.45dB 0dB -1dB 2dB 3dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB In signalling: Qoffsets, n = -1dB SintrasearchP = 62dB SSearchDeltaP = 6dB |
| 16.1.1.6 NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 6.1.1.4 | Same as 6.1.1.4 | Same as 6.1.1.4 |
| 16.1.1.7 NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 48dB Thresh_X_LowP of Cell 1 = 50dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 44dB SSearchDeltaP of Cell 1 = 3dB SSearchDeltaP of Cell 2 = 3dB | During T1: 0dB 0dB 0dB 0.3dB During T2: 0dB 0dB 0dB -2.25dB In signalling: 0dB -4dB -2dB 0dB 0dB 2dB 0dB 12dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -3.7dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc1: -98dBm/15kHz Ês2 / Noc2: +9.75dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 44dB Thresh_X_LowP of Cell 1 = 48dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 46dB SSearchDeltaP of Cell 1 = 3dB SSearchDeltaP of Cell 2 = 15dB |
| 16.1.1.8 NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 6.1.1.5 | Same as 6.1.1.5 | Same as 6.1.1.5 |
| 16.1.2.1 NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1 RX UE | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB During T3: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB Qrxlevmin of Cell 2: -141 | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.6dB During T2: 0dB 1.6dB -2dB 0.4dB In signalling: +1dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +13.6dB During T3: Noc1: -98dBm/15kHz Ês1 / Noc1: +15.6dB Noc2: -100dBm/15kHz Ês2 / Noc2: -3.6dB Qrxlevmin of Cell 2: -140 |
| 16.1.2.2 NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2 RX UE | Same as 6.1.2.1 | Same as 6.1.2.1 | Same as 6.1.2.1 |
| 16.1.2.3 NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1 RX UE | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14Db Qrxlevmin of Cell 1: 140 (137) | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB In signalling: +1dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB Qrxlevmin of Cell 1: 139 (136) |
| 16.1.2.4 NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2 RX UE | Same as 6.1.2.2 | Same as 6.1.2.2 | Same as 6.1.2.2 |
| 16.1.2.5 NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE | Same as 16.1.2.5 | Same as 16.1.2.5 | Same as 16.1.2.5 |
| 16.1.2.6 NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 6.1.2.2 | Same as 6.1.2.2 | Same as 6.1.2.2 |
| 16.2.1.1 NR SA FR1 CG-SDT for 1 Rx UE | During T1: Noc = -100 dBm/15kHz Es/Noc = 0dB During T2: Noc = -100 dBm/15kHz Es/Noc = 12dB During T3: Noc = -100 dBm/15kHz Es/Noc = 0dB During T4: Noc = -100 dBm/15kHz Es/Noc = 14dB During T5: Noc = -100 dBm/15kHz Es/Noc = 14dB During T6: Noc =-100 dBm/15kHz Es/Noc = 0dB In Signalling cg-SDT-RSRP-ChangeThreshold = 8dB cg-SDT-RSRP-ThresholdSSB = -110 dBm/SCS | During T1: 0 dB 0 dB During T2: 0 dB 0.43 dB During T3: 0 dB 0 dB During T4: 0 dB 0 dB During T5: 0 dB 0 dB During T6: 0 dB -0.43 dB In Signalling 0 dB -5 dB for Config 1,2,4 -2 dB for Config 3 | During T1: Noc = -100 dBm/15kHz Es/Noc = 0dB During T2: Noc = -100 dBm/15kHz Es/Noc = 12.43dB During T3: Noc = -100 dBm/15kHz Es/Noc = 0dB During T4: Noc = -100 dBm/15kHz Es/Noc = 14dB During T5: Noc = -100 dBm/15kHz Es/Noc = 14dB During T6: Noc = -100 dBm/15kHz Es/Noc = -0.43dB In Signalling cg-SDT-RSRP-ChangeThreshold = 8dB cg-SDT-RSRP-ThresholdSSB = -115 dBm/SCS for config 1,2 & 4 -112 dBm/SCS for config 3 |
| 16.2.1.2 NR SA FR1 CG-SDT for 2 Rx UE | Same as 6.2.1 | Same as 6.2.1 | Same as 6.2.1 |
| 16.3.1.1 NR SA FR1 handover with known target cell for 1 Rx UE | During T1 Noc: -98 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: -infinity During T2 Noc: -98 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: 11 dB During T3 Noc: -98 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: 11 dB In signalling A3-Offset 0 dB | 0 dB 0 dB 0 dB 0 dB 0 dB -1.85dB 0 dB 0 dB -1.85dB -2.5dB | During T1 Noc: -98 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: -infinity During T2 Noc: -98 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: 9.15 dB During T3 Noc: -98 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: 19.15 dB In signalling A3-Offset -2.5 dB |
| 16.3.1.2 NR SA FR1 handover with known target cell for 2 Rx UE | Same as 6.3.1.1 | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 16.3.1.3 NR SA FR1 handover with unknown target cell for 1 Rx UE | Same as 6.3.1.2 | Same as 6.3.1.2 | Same as 6.3.1.2 |
| 16.3.1.4 NR SA FR1 handover with unknown target cell for 2 Rx UE | Same as 6.3.1.2 | Same as 6.3.1.2 | Same as 6.3.1.2 |
| 16.3.1.5 NR SA FR1-FR1 handover with unknown target cell for 1 Rx UE | Same as 6.3.1.3 | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 16.3.1.6 NR SA FR1-FR1 handover with unknown target cell for 2 Rx UE | Same as 6.3.1.3 | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 16.3.1.7 NR SA FR1 - E-UTRA handover with known target cell for 1 Rx UE | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +8dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +8dB In Signalling b2-Threshold1: -96dBm for NR SCS 15kHz and -93dBm for NR SCS 30kHz | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 2.55dB During T3: 0dB 0dB 0dB 0dB 3dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +10.55dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +8dB In Signalling b2-Threshold1: -93dBm for NR SCS 15kHz and -90dBm for NR SCS 30kHz |
| 16.3.1.8 NR SA FR1 - E-UTRA handover with known target cell for 2Rx UE | Same as 6.3.1.4 | Same as 6.3.1.4 | Same as 6.3.1.4 |
| 16.3.1.9 NR - E-UTRA handover with unknown target cell for 1 Rx UE | Same as 6.3.1.5 | Same as 6.3.1.5 | Same as 6.3.1.5 |
| 16.3.1.10 NR - E-UTRA handover with unknown target cell for 2 Rx UE | Same as 6.3.1.5 | Same as 6.3.1.5 | Same as 6.3.1.5 |
| 16.3.2.1.1 NR SA FR1 RRC Re-establishment for 1 Rx UE | Same as 6.3.2.1.1 | Same as 6.3.2.1.1 | Same as 6.3.2.1.1 |
| 16.3.2.1.2 NR SA FR1 RRC Re-establishment for 2 Rx UE | Same as 6.3.2.1.1 | Same as 6.3.2.1.1 | Same as 6.3.2.1.1 |
| 16.3.2.1.3 NR SA FR1-FR1 RRC Re-establishment for 1 Rx UE | Same as 6.3.2.1.2 | Same as 6.3.2.1.2 | Same as 6.3.2.1.2 |
| 16.3.2.1.4 NR SA FR1-FR1 RRC Re-establishment for 2 Rx UE | Same as 6.3.2.1.2 | Same as 6.3.2.1.2 | Same as 6.3.2.1.2 |
| 16.3.2.1.5 NR SA FR1 RRC Re-establishment for 1 Rx UE without serving cell timing | Same as 6.3.2.1.3 | Same as 6.3.2.1.3 | Same as 6.3.2.1.3 |
| 16.3.2.1.6 NR SA FR1 RRC Re-establishment for 2 Rx UE without serving cell timing | Same as 6.3.2.1.3 | Same as 6.3.2.1.3 | Same as 6.3.2.1.3 |
| 16.3.2.2.1 NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE | TEST CONFIGURATION 1,4 Noc: -98dBm/15kHz Es0/Noc: 3dB Es1/Noc:-17dB First PRACH preamble transmit power: -22dBm Absolute power tolerance ± 9dB Relative power tolerance: ± 2.5dB PRACH transmit timing error ± 768*Tc In signalling rsrp-ThresholdSSB: -105dB | 0dB 1.55dB 0dB Via calculating ± 11.1dB ±0.7dB ±112Tc 0dB | Noc: -98dBm/15kHz Es0/Noc: 4.55dB Es1/Noc:-17dB First PRACH preamble transmit power: -23.55dBm Absolute power tolerance ± 20.1dB Relative power tolerance: ± 3.2dB PRACH transmit timing error ± 880*Tc In signalling rsrp-ThresholdSSB: -105dB |
|  | TEST CONFIGURATION 2 Noc: -98dBm/15kHz Es0/Noc: 3dB Es1/Noc:-17dB First PRACH preamble transmit power: -25dBm Absolute power tolerance ± 9dB Relative power tolerance: ± 2.5dB PRACH transmit timing error ± 768*Tc In signalling rsrp-ThresholdSSB: -105dB | 0dB 1.55dB 0dB Via calculating ± 11.1dB ±0.7dB ±112Tc 0dB | Noc: -98dBm/15kHz Es0/Noc: 4.55dB Es1/Noc:-17dB First PRACH preamble transmit power: -26.55dBm Absolute power tolerance ± 20.1dB Relative power tolerance: ± 3.2dB PRACH transmit timing error ± 880*Tc In signalling rsrp-ThresholdSSB: -105dB |
|  | TEST CONFIGURATION 3 Noc: -101dBm/15kHz Es0/Noc: 3dB Es1/Noc:-17dB First PRACH preamble transmit power: -22dBm Absolute power tolerance ± 9dB Relative power tolerance: ± 2.5dB PRACH transmit timing error ± 768*Tc In signalling rsrp-ThresholdSSB: -105dB | 0dB 1.55dB 0dB Via calculating ± 11.1dB ±0.7dB ±112Tc 0 dB | Noc: -101dBm/15kHz Es0/Noc: 4.55dB Es1/Noc:-17dB First PRACH preamble transmit power: -23.55dBm Absolute power tolerance ± 20.1dB Relative power tolerance: ± 3.2dB PRACH transmit timing error ± 624*Tc In signalling rsrp-ThresholdSSB: -105dB |
| 16.3.2.2.2 NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.3 NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE | Same as 16.3.2.2.1 | Same as 16.3.2.2.1 | Same as 16.3.2.2.1 |
| 16.3.2.2.4 NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.5 NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE | TEST CONFIGURATION 1,4 Noc: -98dBm/15kHz Es0/Noc: 3dB Es1/Noc:-17dB First PRACH preamble transmit power: -22dBm First MsgA PUSCH transmit power: 6.6 dBm Absolute power tolerance ± 9dB Relative power tolerance: ± 2.5dB PRACH transmit timing error ± 768*Tc In signalling rsrp-ThresholdSSB: -105dB | 0dB 1.55dB 0dB Via calculating Via calculating 0dB ± 11.1dB ±0.7dB ±112Tc 0dB | Noc: -98dBm/15kHz Es0/Noc: 4.55dB Es1/Noc:-17dB First PRACH preamble transmit power: -23.55dBm First MsgA PUSCH transmit power: 6.6 dBm Absolute power tolerance ± 20.1dB Relative power tolerance: ± 3.2dB PRACH transmit timing error ± 880*Tc In signalling rsrp-ThresholdSSB: -105dB |
|  | TEST CONFIGURATION 2 Noc: -98dBm/15kHz Es0/Noc: 3dB Es1/Noc:-17dB First PRACH preamble transmit power: -25dBm First MsgA PUSCH transmit power: 6.6 dBm Absolute power tolerance ± 9dB Relative power tolerance: ± 2.5dB PRACH transmit timing error ± 768*Tc In signalling rsrp-ThresholdSSB: -105dB | 0dB 1.55dB 0dB Via calculating ± 11.1dB Via calculating 0 dB ± 11.1dB ±0.7dB ±112Tc 0dB | Noc: -98dBm/15kHz Es0/Noc: 4.55dB Es1/Noc:-17dB First PRACH preamble transmit power: -26.55dBm First MsgA PUSCH transmit power: 6.6 dBm Absolute power tolerance ± 20.1dB Relative power tolerance: ± 3.2dB PRACH transmit timing error ± 880*Tc In signalling rsrp-ThresholdSSB: -105dB |
|  | TEST CONFIGURATION 3 Noc: -101dBm/15kHz Es0/Noc: 3dB Es1/Noc:-17dB First PRACH preamble transmit power: -22dBm First MsgA PUSCH transmit power: 9.6 dBm Absolute power tolerance ± 9dB Relative power tolerance: ± 2.5dB PRACH transmit timing error ± 768*Tc In signalling rsrp-ThresholdSSB: -105dB | 0dB 1.55dB 0dB Via calculating Via calculating ± 11.1dB ±0.7dB ±112Tc 0 dB | Noc: -101dBm/15kHz Es0/Noc: 4.55dB Es1/Noc:-17dB First PRACH preamble transmit power: -23.55dBm First MsgA PUSCH transmit power: 9.6 dBm Absolute power tolerance ± 20.1dB Relative power tolerance: ± 3.2dB PRACH transmit timing error ± 624*Tc In signalling rsrp-ThresholdSSB: -105dB |
| 16.3.2.2.6 NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.7 NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE | TEST CONFIGURATION 1 Noc: -101dBm/15kHz Es0/Noc: 3dB Es1/Noc:-17dB First PRACH preamble transmit power: -22dBm First MsgA PUSCH transmit power: 9.6 dBm Absolute power tolerance ± 9dB Relative power tolerance: ± 2.5dB PRACH transmit timing error ± 768*Tc In signalling rsrp-ThresholdSSB: -105dB | 0dB 1.55dB 0dB Via calculating Via calculating ± 11.1dB ±0.7dB ±112Tc 0 dB | Noc: -101dBm/15kHz Es0/Noc: 4.55dB Es1/Noc:-17dB First PRACH preamble transmit power: -23.55dBm First MsgA PUSCH transmit power: 9.6 dBm Absolute power tolerance ± 20.1dB Relative power tolerance: ± 3.2dB PRACH transmit timing error ± 624*Tc In signalling rsrp-ThresholdSSB: -105dB |
|  | TEST CONFIGURATION 2 Noc: -98dBm/15kHz Es0/Noc: 3dB Es1/Noc:-17dB First PRACH preamble transmit power: -22dBm First MsgA PUSCH transmit power: 6.6 dBm Absolute power tolerance ± 9dB Relative power tolerance: ± 2.5dB PRACH transmit timing error ± 768*Tc In signalling rsrp-ThresholdSSB: -105dB | 0dB 1.55dB 0dB Via calculating Via calculating 0dB ± 11.1dB ±0.7dB ±112Tc 0dB | Noc: -98dBm/15kHz Es0/Noc: 4.55dB Es1/Noc:-17dB First PRACH preamble transmit power: -23.55dBm First MsgA PUSCH transmit power: 6.6 dBm Absolute power tolerance ± 20.1dB Relative power tolerance: ± 3.2dB PRACH transmit timing error ± 880*Tc In signalling rsrp-ThresholdSSB: -105dB |
| 16.3.2.2.8 NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.3.1 NR SA FR1-FR1 RRC connection release with redirection for 1 Rx UE | Same as 6.3.2.3.1 | Same as 6.3.2.3.1 | Same as 6.3.2.3.1 |
| 16.3.2.3.2 NR SA FR1-FR1 RRC connection release with redirection for 2 Rx UE | Same as 6.3.2.3.1 | Same as 6.3.2.3.1 | Same as 6.3.2.3.1 |
| 16.3.2.3.3 NR SA FR1 - E-UTRA RRC connection release with redirection for 1 Rx UE | Same as 6.3.2.3.2 | Same as 6.3.2.3.2 | Same as 6.3.2.3.2 |
| 16.3.2.3.4 NR SA FR1 - E-UTRA RRC connection release with redirection for 2 Rx UE | Same as 6.3.2.3.2 | Same as 6.3.2.3.2 | Same as 6.3.2.3.2 |
| 16.4.1.1 NR SA FR1 NR UE Transmit Timing accuracy for 1 Rx UE | Same as 6.4.1.1 | Same as 6.4.1.1 | Same as 6.4.1.1 |
| 16.4.1.2 NR SA FR1 NR UE Transmit Timing accuracy for 2 Rx UE | Same as 6.4.1.1 | Same as 6.4.1.1 | Same as 6.4.1.1 |
| 16.4.3.1 NR SA FR1 timing advance adjustment accuracy for 1 Rx UE | Same as 6.4.3.1 | Same as 6.4.3.1 | Same as 6.4.3.1 |
| 16.4.3.2 NR SA FR1 timing advance adjustment accuracy for 2 Rx UE | Same as 6.4.3.1 | Same as 6.4.3.1 | Same as 6.4.3.1 |
| 16.5.1.1 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM-RS in non-DRX mode for 1 Rx UE | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered |
| 16.5.1.2 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM-RS in non-DRX mode for 2 Rx UE | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered |
| 16.5.1.3 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM-RS in non-DRX mode for 1 Rx UE | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered |
| 16.5.1.4 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM-RS in non-DRX mode for 2 Rx UE | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered |
| 16.5.1.5 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM-RS in DRX mode for 1 Rx UE | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered |
| 16.5.1.6 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM-RS in DRX mode for 2 Rx UE | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered |
| 16.5.1.7 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM-RS in DRX mode for 1 Rx UE | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered |
| 16.5.1.8 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM-RS in DRX mode for 2 Rx UE | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered |
| 16.5.1.9 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered |
| 16.5.1.10 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered |
| 16.5.1.11 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered |
| 16.5.1.12 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered |
| 16.5.1.13 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered |
| 16.5.1.14 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered | Same as 6.5.1.1 with the exception: - 4Rx is not considered |
| 16.5.1.15 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered |
| 16.5.1.16 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered | Same as 6.5.1.2 with the exception: - 4Rx is not considered |
| 16.5.2.1 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE | Same ad 16.5.2.3 | Same ad 16.5.2.3 | Same ad 16.5.2.3 |
| 16.5.2.2 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE | Same as 6.5.5.1 with the exception: - 4Rx is not considered | Same as 6.5.5.1 with the exception: - 4Rx is not considered | Same as 6.5.5.1 with the exception: - 4Rx is not considered |
| 16.5.2.3 NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 1 Rx UE | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB at T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB at T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB at T3-5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB at T1: 0dB -3.25dB q1 SSB at T2: 0dB -3.25dB q1 SSB at T3-5: 0dB +3.25dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB q1 SSB at T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -13.25dB q1 SSB at T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -13.25dB q1 SSB at T3-5: Noc1: -98dBm/15kHz Ês1 / Noc1: +13.25dB |
| 16.5.2.4 NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 2 Rx UE | Same as 6.5.5.1 with the exception: - 4Rx is not considered | Same as 6.5.5.1 with the exception: - 4Rx is not considered | Same as 6.5.5.1 with the exception: - 4Rx is not considered |
| 16.5.2.5 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T3-5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 CSI-RS during T1: 0dB -3.25dB q1 CSI-RS during T2: 0dB -3.25dB q1 CSI-RS during T3-5: 0dB +3.25dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -13.25dB q1 CSI-RS during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -13.25dB q1 CSI-RS during T3-5: Noc1: -98dBm/15kHz Ês1 / Noc1: +13.25dB |
| 16.5.2.6 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE | Same as 6.5.5.3 with the exception: - 4Rx is not considered | Same as 6.5.5.3 with the exception: - 4Rx is not considered | Same as 6.5.5.3 with the exception: - 4Rx is not considered |
| 16.5.2.7 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE | Same ad 16.5.2.5 | Same as 16.5.2.5 | Same ad 16.5.2.5 |
| 16.5.2.8 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE | Same as 6.5.5.3 with the exception: - 4Rx is not considered | Same as 6.5.5.3 with the exception: - 4Rx is not considered | Same as 6.5.5.3 with the exception: - 4Rx is not considered |
| 16.5.3.1.1 NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE | Same as 6.5.6.1.2 | Same as 6.5.6.1.2 | Same as 6.5.6.1.2 |
| 16.5.3.1.2 NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE | Same as 6.5.6.1.2 | Same as 6.5.6.1.2 | Same as 6.5.6.1.2 |
| 16.5.3.2.1 NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE | Same as 6.5.6.2.1 | Same as 6.5.6.2.1 | Same as 6.5.6.2.1 |
| 16.5.3.2.2 NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE | Same as 6.5.6.2.1 | Same as 6.5.6.2.1 | Same as 6.5.6.2.1 |
| 16.5.4.1 NR SA FR1 UE specific CBW change on PCell in non-DRX for 1 Rx UE | Same as 6.5.8.1 | Same as 6.5.8.1 | Same as 6.5.8.1 |
| 16.5.4.2 NR SA FR1 UE specific CBW change on PCell in non-DRX for 2 Rx UE | Same as 6.5.8.1 | Same as 6.5.8.1 | Same as 6.5.8.1 |
| 16.6.1.1 NR SA FR1 Event triggered reporting tests without gap under non-DRX for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.2 NR SA FR1 Event triggered reporting tests without gap under non-DRX for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.3 NR SA FR1 Event triggered reporting tests without gap under DRX for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.4 NR SA FR1 Event triggered reporting tests without gap under DRX for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.5 NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.6 NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.7 NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.8 NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.9 NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.10 NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.11 NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.12 NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.2.1 NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB |
| 16.6.2.2 NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.3 NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE | Same as 16.6.2.1 | Same as 16.6.2.1 | Same as 16.6.2.1 |
| 16.6.2.4 NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.5 NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE | Same as 16.6.2.1 | Same as 16.6.2.1 | Same as 16.6.2.1 |
| 16.6.2.6 NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.7 NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE | Same as 16.6.2.1 | Same as 16.6.2.1 | Same as 16.6.2.1 |
| 16.6.2.8 NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.9 NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE | Same as 16.6.2.1 | Same as 16.6.2.1 | Same as 16.6.2.1 |
| 16.6.2.10 NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.2.11 NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 1 Rx UE | Same as 16.6.2.1 | Same as 16.6.2.1 | Same as 16.6.2.1 |
| 16.6.2.12 NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 2 Rx UE | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 16.6.3.1 NR - E-UTRA event-triggered reporting in non-DRX for 1 Rx UE | During T1: Freq 1 Noc: -104dBm/15kHz Freq 2 Noc: -104dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -104dBm/15kHz Freq 2 Noc: -104dBm/15kHz Ês1 / Noc: 0dB Ês2 / Noc: +17.00dB In signalling: Configs 1,2,4,5: b2-Threshold1 = -96dBm b2-Threshold2EUTRAN = -95dBm Configs 3,6: b2-Threshold1 = -93dBm b2-Threshold2EUTRAN = -95dBm | During T1: 0dB 0dB +3.55dB 0dB During T2: 0dB 0dB -1.55dB +1.55dB In signalling: Configs 1,2,4,5: +1dB -1dB Configs 3,6: +1dB -1dB | During T1: Freq 1 Noc: -104dBm/15kHz Freq 2 Noc: -104dBm/15kHz Ês1 / Noc: +19.55dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -104dBm/15kHz Freq 2 Noc: -104dBm/15kHz Ês1 / Noc: -1.55dB Ês2 / Noc: +18.55dB In signalling: Configs 1,2,4,5: b2-Threshold1 = -95dBm b2-Threshold2EUTRAN = -96dBm Configs 3,6: b2-Threshold1 = -92dBm b2-Threshold2EUTRAN = -96dBm |
| 16.6.3.2 NR - E-UTRA event-triggered reporting in non-DRX for 2 Rx UE | During T1: Freq 1 Noc: -104dBm/15kHz Freq 2 Noc: -104dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -104dBm/15kHz Freq 2 Noc: -104dBm/15kHz Ês1 / Noc: 0dB Ês2 / Noc: +17.00dB | During T1: 0dB 0dB +1.55dB 0dB During T2: 0dB 0dB -1.55dB +1.55dB | During T1: Freq 1 Noc: -104dBm/15kHz Freq 2 Noc: -104dBm/15kHz Ês1 / Noc: +17.55dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -104dBm/15kHz Freq 2 Noc: -104dBm/15kHz Ês1 / Noc: -1.55dB Ês2 / Noc: +18.55dB |
| 16.6.3.3 NR - E-UTRA event-triggered reporting in DRX for 1 Rx UE | Same as 16.6.3.1 | Same as 16.6.3.1 | Same as 16.6.3.1 |
| 16.6.3.4 NR - E-UTRA event-triggered reporting in DRX for 2 Rx UE | Same as 16.6.3.2 | Same as 16.6.3.2 | Same as 16.6.3.2 |
| 16.6.4.1 NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE | During T1: Noc: -94.65 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: -infinity During T2: Noc: -94.65 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: 3 dB Test Configuration 1,2,4 Reported SSB#1 absolute L1-RSRP values: -88.15 ± 13.03 dBm/SCS Reported SSB#0 differential L1-RSRP values: -6.5 ± 6.42 dB Test Configuration 3 Reported SSB#1 absolute L1-RSRP values: -85.15 ± 13.03 dBm/SCS Reported SSB#0 differential L1-RSRP values: -6.5 ± 6.42 dB | 0 dB 0 dB 0 dB 0 dB 0 dB 3.5 dB Via mapping | During T1: Noc: -94.65 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: -infinity During T2: Noc: -94.65 dBm/15kHz Es1/Noc: 0 dB Es2/Noc: 6.5 dB Test Configuration 1,2,4 Reported SSB#1 absolute L1-RSRP Range: RSRP_55 to RSRP_81 Reported SSB#0 differential L1-RSRP range: DIFFRSRP_0 to DIFFRSRP_6 Test Configuration 3 Reported SSB#1 absolute L1-RSRP Range: RSRP_58 to RSRP_84 Reported SSB#0 differential L1-RSRP range: DIFFRSRP_0 to DIFFRSRP_6 |
| 16.6.4.2 NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE | Same as 6.6.4.1 | Same as 6.6.4.1 | Same as 6.6.4.1 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.6.4.3 NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 1 Rx UE | Same as 16.6.4.1 | Same as 16.6.4.1 | Same as 16.6.4.1 |
| 16.6.4.4 NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 2 Rx UE | Same as 6.6.4.1 | Same as 6.6.4.1 | Same as 6.6.4.1 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.6.4.5 NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-RSRP values ± 13.03dB Reported Differential CSI-RSRP values ± 6.42dB Same as 6.6.4.3 | 0dB 0dB 3.5dB Via mapping Same as 6.6.4.3 | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +6.50dB For configuration 1,2,4 CSI-RS#1: RSRP_55 to RSRP_81 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_6 For configuration 3 CSI-RS#1: RSRP_58 to RSRP_84 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_6 Same as 6.6.4.3 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.6.4.6 NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE | Same as 6.6.4.3 | Same as 6.6.4.3 | Same as 6.6.4.3 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.6.4.7 NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE | Same as 6.6.4.3 Same as 16.6.4.5 | Same as 16.6.4.5Same as 6.6.4.3 | Same as 16.6.4.5Same as 6.6.4.3 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.6.4.8 NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE | Same as 6.6.4.3 | Same as 6.6.4.3 | Same as 6.6.4.3 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.6.5.3 NR SA FR1 - E-UTRA CGI Identification of E-UTRA cell for 1 Rx UE | During T1 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: 18dB Es2/Noc2: -infinity During T2 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7dB During T3 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7dB | During T1 -0.55dB 0dB 0dB 0dB During T2 -0.55dB 0dB 0dB 0.55dB During T3 -0.55dB 0dB 0dB 0.55dB | During T1 Noc1: -106.55dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: 18dB Es2/Noc2: -infinity During T2 Noc1: -106.55dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7.55dB During T3 Noc1: -106.55dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7.55dB |
| 16.6.5.4 NR SA FR1 - E-UTRA CGI Identification of E-UTRA cell for 2 Rx UE | During T1 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: 18dB Es2/Noc2: -infinity During T2 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7dB During T3 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7dB | During T1 0dB 0dB 0dB 0dB During T2 0dB 0dB 0dB 0dB During T3 0dB 0dB 0dB 0dB | During T1 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: 18dB Es2/Noc2: -infinity During T2 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7dB During T3 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -2dB Es2/Noc2: 7dB |
| 16.7.1.1.1 NR SA FR1 SS- | TEST CONFIGURATION 1, 2, 4 |  |  |
| RSRP absolute measurement accuracy for 1 Rx UE | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±5.5dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±9dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB Reported RSRP values: ±5.5dB | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -107.5dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_43 to RSRP_57 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_59 to RSRP_80 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0dB RSRP_35 to RSRP_50 RSRP_36 to RSRP_50 RSRP_36 to RSRP_51 RSRP_37 to RSRP_51 RSRP_37 to RSRP_52 RSRP_38 to RSRP_52 RSRP_38 to RSRP_53 RSRP_39 to RSRP_53 depending on operating band |
|  | TEST CONFIGURATION 3 |  |  |
|  | Test 1: N/A Test 2: Noc: -94dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±9dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB Reported RSRP values: ±5.5dB | Test 1: N/A Test 2: 0dB 0dB +0.4dB Via mapping Test 3: -1.2dB 0dB 0dB Via mapping | Test 1: N/A Test 2: Noc: -94dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_56 to RSRP_77 Test 3: Noc: -115.2dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0dB RSRP_37 to RSRP_51 RSRP_38 to RSRP_52 RSRP_38 to RSRP_52 RSRP_39 to RSRP_53 RSRP_39 to RSRP_53 RSRP_40 to RSRP_54 RSRP_40 to RSRP_54 RSRP_41 to RSRP_55 depending on operating band |
| 16.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE | Test 1: Noc: Test configuration 1, 2, 4: -106dBm/15kHz Test configuration 3: N/A Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported rel. SS-RSRP values: ±3dB Test 2: Noc: Test configuration 1, 2, 4: -88dBm/15kHz Test configuration 3 -94dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported rel. SS-RSRP values: ±3dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB Reported re. SS-RSRP values: ±3dB | Test 1: 0 dB N/A 0 dB +1.0dB Via mapping Test 2: 0dB 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -106 dBm/15kHz N/A Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 2: Noc: -88dBm/15kHz -94dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB RSRP_x-7 to RSRP_x+1 |
| 16.7.1.2.1 NR SA FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.7.1.2.2 NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE | Same as 6.7.1.1.2 | Same as 6.7.1.1.2 | Same as 6.7.1.1.2 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.7.1.3.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for 1 Rx UE | TEST CONFIGURATION 1, 2, 4 |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -115dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_61 to RSRP_82 Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -115dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_31 to RSRP_46 RSRP_32 to RSRP_46 RSRP_32 to RSRP_47 RSRP_33 to RSRP_47 RSRP_33 to RSRP_48 RSRP_34 to RSRP_49 RSRP_35 to RSRP_49 depending on operating band |
|  | TEST CONFIGURATION 3 |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -115dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_64 to RSRP_85 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -115dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_34 to RSRP_49 RSRP_35 to RSRP_49 RSRP_36 to RSRP_50 RSRP_36 to RSRP_50 RSRP_36 to RSRP_51 RSRP_37 to RSRP_52 RSRP_38 to RSRP_52 depending on operating band |
| 16.7.1.3.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy for 1 Rx UE | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -115dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_x-8 to RSRP_x+8 Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -115dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_x-34 to RSRP_x-15 |
| 16.7.1.4.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.7.1.4.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy for 2 Rx UE | Same as 6.7.1.2.2 | Same as 6.7.1.2.2 | Same as 6.7.1.2.2 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.7.2.1 NR SA FR1 SS-RSRQ absolute measurement accuracy for 1 Rx UE | TEST CONFIGURATION 1, 2, 4 |  |  |
|  | Test 1: Noc: -85dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±3.5dB Test 2: Noc: -101dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±4.5dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±4.5dB | Test 1: -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -86.5dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_50 to RSRQ_64 Test 2: Noc: -101dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_44 to RSRQ_62 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB RSRQ_42 to RSRQ_61 |
|  | TEST CONFIGURATION 3 |  |  |
|  | Test 1: Noc: -91dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB | Test 1: -1.6dB 0dB 0dB Via mapping Test 2: N/A Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -86.5dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_50 to RSRQ_64 Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB RSRQ_42 to RSRQ_61 |
| 16.7.2.2 NR SA FR1 SS-RSRQ absolute measurement accuracy for 2 Rx UE | Same as 6.7.2.1 | Same as 6.7.2.1 | Same as 6.7.2.1 with following exceptions: - Test requirements for configuration 1,2 also apply to configuration 4 |
| 16.7.2.3.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for 1 Rx UE | Test configuration 1,2,4: Test 1 Noc1: -80.18dBm/15kHz Noc2: -80.18dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 2 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB | Test 1 -1.5dB -1.5dB 0dB 0dB Via mapping Via mapping Test 2 0dB 0dB 0dB 0dB Via mapping Via mapping Test 3 0dB 0dB 0dB 0dB Via mapping Via mapping | Test 1 Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_50 to RSRQ_64 Extreme condition, RSRQ_47 to RSRQ_67 Test 2 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_50 to RSRQ_64 Extreme condition, RSRQ_47 to RSRQ_67 Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, RSRQ_50 to RSRQ_64 Extreme condition, RSRQ_47 to RSRQ_67 |
|  | Test configuration 3: Test 1 Noc1: -86.27dBm/15kHz Noc2: -86.27dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 2 Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB | Test 1 0dB 0dB 0dB 0dB Via mapping Via mapping Test 2 0dB 0dB 0dB 0dB Via mapping Via mapping Test 3 0dB 0dB 0dB 0dB Via mapping Via mapping | Test 1 Noc1: -86.27dBm/15kHz Noc2: -86.27dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_50 to RSRQ_64 Extreme condition, RSRQ_47 to RSRQ_67 Test 2 Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_50 to RSRQ_64 Extreme condition, RSRQ_47 to RSRQ_67 Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, RSRQ_50 to RSRQ_64 Extreme condition, RSRQ_47 to RSRQ_67 |
| 16.7.2.3.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for 1 Rx UE | Test configuration 1,2,4: Test 1 Noc1: -80.18dBm/15kHz Noc2: -80.18dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±4.25dB Extreme condition, Reported RSRQ values: ±5.25dB Test 2 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±4.25dB Extreme condition, Reported RSRQ values: ±5.25dB Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±4.21dB Extreme condition, Reported RSRQ values: ±5.21dB | Test 1 -1.5dB -1.5dB 0dB 0dB Via mapping Via mapping Test 2 0dB 0dB 0dB 0dB Via mapping Via mapping Test 3 0dB 0dB 0dB 0dB Via mapping Via mapping | Test 1 Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_x-9 to RSRQ_x+9 Extreme condition, RSRQ_x-11 to RSRQ_x+11 Test 2 Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_x-9 to RSRQ_x+9 Extreme condition, RSRQ_x-11 to RSRQ_x+11 Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, RSRQ_x-13 to RSRQ_x+4 Extreme condition, RSRQ_x-15 to RSRQ_x+6 |
|  | Test configuration 3: Test 1 Noc1: -86.27dBm/15kHz Noc2: -86.27dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 2 Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB | Test 1 0dB 0dB 0dB 0dB Via mapping Via mapping Test 2 0dB 0dB 0dB 0dB Via mapping Via mapping Test 3 0dB 0dB 0dB 0dB Via mapping Via mapping | Test 1 Noc1: -86.27dBm/15kHz Noc2: -86.27dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_x-9 to RSRQ_x+9 Extreme condition, RSRQ_x-11 to RSRQ_x+11 Test 2 Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_x-9 to RSRQ_x+9 Extreme condition, RSRQ_x-11 to RSRQ_x+11 Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, RSRQ_x-13 to RSRQ_x+4 Extreme condition, RSRQ_x-15 to RSRQ_x+6 |
| 16.7.2.4.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for 2 Rx UE | Test configuration 1,2,4: Same as 6.7.2.2.2 test configuration 1,2 | Same as 6.7.2.2.2 test configuration 1,2 | Same as 6.7.2.2.2 test configuration 1,2 |
|  | Test configuration 3: Test 1 Noc1: -86.27dBm/15kHz Noc2: -86.27dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 2 Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB | Test 1 0dB 0dB 0dB 0dB Via mapping Via mapping Test 2 0dB 0dB 0dB 0dB Via mapping Via mapping Test 3 0dB 0dB 0dB 0dB Via mapping Via mapping | Test 1 Noc1: -86.27dBm/15kHz Noc2: -86.27dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_50 to RSRQ_64 Extreme condition, RSRQ_47 to RSRQ_67 Test 2 Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_50 to RSRQ_64 Extreme condition, RSRQ_47 to RSRQ_67 Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, RSRQ_50 to RSRQ_64 Extreme condition, RSRQ_47 to RSRQ_67 |
| 16.7.2.4.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for 2 Rx UE | Test configuration 1,2,4: Same as 6.7.2.2.2 test configuration 1, 2 | Same as 6.7.2.2.2 test configuration 1, 2 | Same as 6.7.2.2.2 test configuration 1, 2 |
|  | Test configuration 3: Test 1 Noc1: -86.27dBm/15kHz Noc2: -86.27dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 2 Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, Reported RSRQ values: ±3.68dB Extreme condition, Reported RSRQ values: ±5.18dB | Test 1 0dB 0dB 0dB 0dB Via mapping Via mapping Test 2 0dB 0dB 0dB 0dB Via mapping Via mapping Test 3 0dB 0dB 0dB 0dB Via mapping Via mapping | Test 1 Noc1: -86.27dBm/15kHz Noc2: -86.27dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_x-7 to RSRQ_x+7 Extreme condition, RSRQ_x-9 to RSRQ_x+9 Test 2 Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, RSRQ_x-7 to RSRQ_x+7 Extreme condition, RSRQ_x-9 to RSRQ_x+9 Test 3 Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Es1/Noc1: 3dB Es2/Noc2: -1.75dB Normal condition, RSRQ_x-11 to RSRQ_x+2 Extreme condition, RSRQ_x-13 to RSRQ_x+4 |
| 16.7.3.1 NR SA FR1 SS-SINR absolute measurement accuracy for 1 Rx UE | Test configuration 1, 2, 4: Same as 6.7.3.1 test configuration 1, 2 | Same as 6.7.3.1 test configuration 1, 2 | Same as 6.7.3.1 test configuration 1, 2 |
|  | Test configuration 3: Test 1 Noc: -93dBm/15kHz Es1/Noc: 4.54dB Es2/Noc: 2.66dB Normal condition, Reported SINR values: ±4.40dB Extreme condition, Reported SINR values: ±4.90dB Test 2 Noc: -116dBm/15kHz + ΔBG_offset Es1/Noc: -4dB Es2/Noc: -4dB Normal condition, Reported SINR values: ±4.17dB Extreme condition, Reported SINR values: ±4.67dB | 0dB 0dB 0dB Via mapping Via mapping 0dB 0.5dB 0.5dB Via mapping Via mapping | Test 1 Noc: -93dBm/15kHz Es1/Noc: 4.54dB Es2/Noc: 2.66dB Normal condition, SINR_29 to SINR_51 Extreme condition, SINR_28 to SINR_52 Test 2 Noc: -116dBm/15kHz + ΔBG_offset Es1/Noc: -3.5dB Es2/Noc: -3.5dB Normal condition, SINR_26 to SINR_47 Extreme condition, SINR_25 to SINR_48 |
| 16.7.3.2 NR SA FR1 SS-SINR absolute measurement accuracy for 2 Rx UE | Same as 16.7.3.1 | Same as 16.7.3.1 | Same as 16.7.3.1 |
| 16.7.3.3.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy for 1 Rx UE | Test 1 Noc1: -88dBm/15kHz Noc2: -88dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported SINR values: ±4.30dB Extreme condition, Reported SINR values: ±5.30dB Test 2 Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Es1/Noc1: 20dB Es2/Noc2: 20dB Normal condition, Reported SINR values: ±4.30dB Extreme condition, Reported SINR values: ±5.30dB Test 3 Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Es1/Noc1: -4dB Es2/Noc2: -4dB Normal condition, Reported SINR values: ±4.80dB Extreme condition, Reported SINR values: ±5.30dB | 0dB 0dB 0dB 0dB Via mapping Via mapping 0dB 0dB 0dB 0dB Via mapping Via mapping 0dB 0dB 0.35dB 0.35dB Via mapping Via mapping | Test 1 Noc1: -88dBm/15kHz Noc2: -88dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, SINR_34 to SINR_52 Extreme condition, SINR_32 to SINR_54 Test 2 Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Es1/Noc1: 20dB Es2/Noc2: 20dB Normal condition, SINR_78 to SINR_95 Extreme condition, SINR_76 to SINR_97 Test 3 Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Es1/Noc1: -3.65dB Es2/Noc2: -3.65dB Normal condition, SINR_30 to SINR_49 Extreme condition, SINR_29 to SINR_50 |
| 16.7.3.3.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy for 1 Rx UE | Test 1 Noc1: -88dBm/15kHz Noc2: -88dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, Reported SINR values: ±4.92dB Extreme condition, Reported SINR values: ±5.42dB Test 2 Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Es1/Noc1: 20dB Es2/Noc2: 20dB Normal condition, Reported SINR values: ±4.92dB Extreme condition, Reported SINR values: ±5.42dB Test 3 Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Es1/Noc1: -4dB Es2/Noc2: -4dB Normal condition, Reported SINR values: ±5.42dB Extreme condition, Reported SINR values: ±5.42dB | 0dB 0dB 0dB 0dB Via mapping Via mapping 0dB 0dB 0dB 0dB Via mapping Via mapping 0dB 0dB 0.35dB 0.35dB Via mapping Via mapping | Test 1 Noc1: -88dBm/15kHz Noc2: -88dBm/15kHz Es1/Noc1: -1.75dB Es2/Noc2: -1.75dB Normal condition, SINR_x-10 to SINR_x+10 Extreme condition, SINR_x-11 to SINR_x+11 Test 2 Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Es1/Noc1: 20dB Es2/Noc2: 20dB Normal condition, SINR_x-10 to SINR_x+10 Extreme condition, SINR_x-11 to SINR_x+11 Test 3 Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Es1/Noc1: -3.65dB Es2/Noc2: -3.65dB Normal condition, SINR_x-11 to SINR_x+11 Extreme condition, SINR_x-11 to SINR_x+11 |
| 16.7.3.4.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy for 2 Rx UE | Same as 6.7.3.2.1 | Same as 6.7.3.2.1 | Same as 6.7.3.2.1 |
| 16.7.3.4.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy for 2 Rx UE | Same as 6.7.3.2.2 | Same as 6.7.3.2.2 | Same as 6.7.3.2.2 |
| 16.7.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for 1 Rx UE | Test configuration 1, 2, 4: Test 1 Noc: -94.65dBm/15kHz Es/Noc: 10dB Normal condition, Reported L1-RSRP values: ±13.03dB Extreme condition, Reported L1-RSRP values: ±16.03dB Test 2 Noc: -117dBm/15kHz + ΔBG_offset Es/Noc: -3dB Normal condition, Reported L1-RSRP values: ±13.03dB Extreme condition, Reported L1-RSRP values: ±16.03dB | 0dB 0dB Via mapping Via mapping 0dB 0.35dB Via mapping Via mapping | Test 1 Noc: -94.65dBm/15kHz Es/Noc: 10dB Normal condition, RSRP_59 to RSRP_85 Extreme condition, RSRP_56 to RSRP_88 Test 2 Noc: -117dBm/15kHz + ΔBG_offset Es/Noc: -2.65dB Normal condition: RSRP_27 to RSRP_46 RSRP_28 to RSRP_47 RSRP_28 to RSRP_47 RSRP_29 to RSRP_48 RSRP_29 to RSRP_48 RSRP_30 to RSRP_49 RSRP_30 to RSRP_49 RSRP_31 to RSRP_50 Depending on band groups Extreme condition: RSRP_23 to RSRP_51 RSRP_23 to RSRP_51 RSRP_24 to RSRP_52 RSRP_24 to RSRP_52 RSRP_25 to RSRP_53 RSRP_25 to RSRP_53 RSRP_26 to RSRP_54 RSRP_26 to RSRP_54 Depending on band groups |
|  | Test configuration 3: Test 1 Noc: -94.65dBm/15kHz Es/Noc: 10dB Normal condition, Reported L1-RSRP values: ±13.03dB Extreme condition, Reported L1-RSRP values: ±16.03dB Test 2 Noc: -117dBm/15kHz + ΔBG_offset Es/Noc: -3dB Normal condition, Reported L1-RSRP values: ±13.03dB Extreme condition, Reported L1-RSRP values: ±16.03dB | 0dB 0dB Via mapping Via mapping 0dB 0.35dB Via mapping Via mapping | Test 1 Noc: -94.65dBm/15kHz Es/Noc: 10dB Normal condition, RSRP_62 to RSRP_88 Extreme condition, RSRP_59 to RSRP_91 Test 2 Noc: -117dBm/15kHz + ΔBG_offset Es/Noc: -2.65dB Normal condition: RSRP_30 to RSRP_49 RSRP_31 to RSRP_50 RSRP_31 to RSRP_50 RSRP_32 to RSRP_51 RSRP_32 to RSRP_51 RSRP_33 to RSRP_52 RSRP_33 to RSRP_52 RSRP_34 to RSRP_53 Depending on band groups Extreme condition: RSRP_26 to RSRP_54 RSRP_26 to RSRP_54 RSRP_27 to RSRP_55 RSRP_27 to RSRP_55 RSRP_28 to RSRP_56 RSRP_28 to RSRP_56 RSRP_29 to RSRP_57 RSRP_29 to RSRP_57 Depending on band groups |
| 16.7.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy for 1 Rx UE | Test configuration 1, 2, 4: Test 1 Noc: -94.65dBm/15kHz Es/Noc: 10dB Normal condition, Reported differential L1-RSRP values: ±6.6dB Extreme condition, Reported differential L1-RSRP values: ±7.6dB Test 2 Noc: -117dBm/15kHz + ΔBG_offset Es/Noc: -3dB Normal condition, Reported differential L1-RSRP values: ±6.6dB Extreme condition, Reported differential L1-RSRP values: ±7.6dB | 0dB 0dB Via mapping Via mapping 0dB 0.35dB Via mapping Via mapping | Test 1 Noc: -94.65dBm/15kHz Es/Noc: 10dB Normal condition, DIFFRSRP_0 to DIFFRSRP_3 Extreme condition, DIFFRSRP_0 to DIFFRSRP_3 Test 2 Noc: -117dBm/15kHz + ΔBG_offset Es/Noc: -2.65dB Normal condition, DIFFRSRP_0 to DIFFRSRP_3 Extreme condition, DIFFRSRP_0 to DIFFRSRP_3 |
|  | Test configuration 3: Test 1 Noc: -94.65dBm/15kHz Es/Noc: 10dB Normal condition, Reported differential L1-RSRP values: ±6.6dB Extreme condition, Reported differential L1-RSRP values: ±7.6dB Test 2 Noc: -117dBm/15kHz + ΔBG_offset Es/Noc: -3dB Normal condition, Reported differential L1-RSRP values: ±6.6dB Extreme condition, Reported differential L1-RSRP values: ±7.6dB | 0dB 0dB Via mapping Via mapping 0dB 0.35dB Via mapping Via mapping | Test 1 Noc: -94.65dBm/15kHz Es/Noc: 10dB Normal condition, DIFFRSRP_0 to DIFFRSRP_3 Extreme condition, DIFFRSRP_0 to DIFFRSRP_3 Test 2 Noc: -117dBm/15kHz + ΔBG_offset Es/Noc: -2.65dB Normal condition, DIFFRSRP_0 to DIFFRSRP_3 Extreme condition, DIFFRSRP_0 to DIFFRSRP_3 |
| 16.7.4.2.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for 2 Rx UE | Test configuration 1, 2, 4: Same as 6.7.4.1.1 | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
|  | Test configuration 3: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±3dB additionally for extreme conditions) Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±4.5dB additionally for extreme conditions) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB RSRP_65 to RSRP_85 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 RSRP_35 to RSRP_48 RSRP_35 to RSRP_49 RSRP_36 to RSRP_49 RSRP_37 to RSRP_50 RSRP_37 to RSRP_51 depending on operating band |
| 16.7.4.2.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy for 2 Rx UE | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 |
| 16.7.4.3.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for 1 Rx UE | Same as 16.7.4.1.1 | Same as 16.7.4.1.1 | Same as 16.7.4.1.1 |
| 16.7.4.3.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for 1 Rx UE | Same as 16.7.4.1.2 | Same as 16.7.4.1.2 | Same as 16.7.4.1.2 |
| 16.7.4.4.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for 2 Rx UE | Same as 16.7.4.2.1 | Same as 16.7.4.2.1 | Same as 16.7.4.2.1 |
| 16.7.4.4.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for 2 Rx UE | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 |
| 16.7.5.1 NR SA FR1 - E-UTRA RSRP absolute measurement accuracy for 1 Rx UE | Test 1 Noc2: -91.65dBm/15kHz Es2/Noc2: 10dB Normal condition, Reported LTE RSRP values: ±10.53dB Extreme condition, Reported LTE RSRP values: ±13.53dB Test 2 Noc2: -117dBm/15kHz + ΔBG_offset Es2/Noc2: -4dB Normal condition, Reported LTE RSRP values: ±7.03dB Extreme condition, Reported LTE RSRP values: ±11.53dB | 0dB 0dB Via mapping Via mapping 0dB 0.35dB Via mapping Via mapping | Noc2: -91.65dBm/15kHz Es2/Noc2: 10dB Normal condition, RSRP_48 to RSRP_69 Extreme condition, RSRP_45 to RSRP_72 Noc2: -117dBm/15kHz + ΔBG_offset Es2/Noc2: -3.65dB Normal condition, RSRP_13 to RSRP_27 RSRP_13 to RSRP_27 RSRP_14 to RSRP_28 RSRP_14 to RSRP_28 RSRP_15 to RSRP_29 RSRP_16 to RSRP_30 RSRP_16 to RSRP_30 Depending on band groups Extreme condition, RSRP_8 to RSRP_31 RSRP_9 to RSRP_32 RSRP_9 to RSRP_32 RSRP_10 to RSRP_33 RSRP_10 to RSRP_33 RSRP_11 to RSRP_34 RSRP_12 to RSRP_35 Depending on band groups |
| 16.7.5.2 NR SA FR1 - E-UTRA RSRP absolute measurement accuracy for 2 Rx UE | Same as 6.7.5.1 | Same as 6.7.5.1 | Same as 6.7.5.1 |
| 16.7.6.1 NR SA FR1 - E-UTRA RSRQ absolute measurement accuracy for 1 Rx UE | Test 1 Noc2: -83dBm/15kHz Es2/Noc2: -1.75dB Normal condition, Reported LTE RSRQ values: ±3.68dB Extreme condition, Reported LTE RSRQ values: ±5.18dB Test 2 Noc2: -104.7dBm/15kHz Es2/Noc2: -4dB Normal condition, Reported LTE RSRQ values: ±4.72dB Extreme condition, Reported LTE RSRQ values: ±5.22dB Test 3 Noc2: -119.5dBm/15kHz + ΔBG_offset Es2/Noc2: -4dB Normal condition, Reported LTE RSRQ values: ±4.72dB Extreme condition, Reported LTE RSRQ values: ±5.22dB | 0dB 0dB Via mapping Via mapping 0dB 0.35dB Via mapping Via mapping 0dB 0.35dB Via mapping Via mapping | Noc2: -83dBm/15kHz Es2/Noc2: -1.75dB Normal condition, RSRQ_3 to RSRQ_17 Extreme condition, RSRP_0 to RSRP_20 Noc2: -104.7dBm/15kHz Es2/Noc2: -3.65dB Normal condition, RSRQ_0 to RSRQ_17 Extreme condition, RSRP_0 to RSRP_18 Noc2: -119.5dBm/15kHz + ΔBG_offset Es2/Noc2: -3.65dB Normal condition, RSRQ_0 to RSRQ_17 Extreme condition, RSRP_0 to RSRP_18 |
| 16.7.6.2 NR SA FR1 - E-UTRA RSRQ absolute measurement accuracy for 2 Rx UE | Same as 6.7.6.1 | Same as 6.7.6.1 | Same as 6.7.6.1 |

Table F.1.3.2-8: Derivation of test requirements for NR SA FR2 RRM tests for RedCap

| Test | Minimum requirement in TS 38.133 [6] |  | Test tolerance (TT) |  | Test requirement in TS 38.533 |
| --- | --- | --- | --- | --- | --- |
| 17.1.1.1 NR SA FR2 Cell reselection for 2 Rx UE | Same as 7.1.1.1 | Same as 7.1.1.1 |  | Same as 7.1.1.1 |  |
| 17.1.1.2 NR SA FR2-FR2 Cell reselection for 2 Rx UE | Same as 7.1.1.2 | Same as 7.1.1.2 |  | Same as 7.1.1.2 |  |
| 17.1.1.3 NR SA FR2 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 7.1.1.3 | Same as 7.1.1.3 |  | Same as 7.1.1.3 |  |
| 17.1.1.4 NR SA FR2-FR2 Cell reselection for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE | Same as 7.1.1.5 | Same as 7.1.1.5 |  | Same as 7.1.1.5 |  |
| 17.3.1.1 NR SA FR2 handover with unknown target cell for 2 Rx | Same as 7.3.1.2 | Same as 7.3.1.2 |  | Same as 7.3.1.2 |  |
| 17.3.1.2 NR SA FR2-FR2 handover with unknown target cell for 2 Rx UE | Same as 7.3.1.3 | Same as 7.3.1.3 |  | Same as 7.3.1.3 |  |
| 17.3.2.1.1 NR SA FR2 Intra-frequency RRC Re-establishment in FR2 | Same as 7.3.2.1.1 |  | Same as 7.3.2.1.1 |  | Same as 7.3.2.1.1 |
| 17.3.2.1.2 NR SA FR2-FR2 Inter-frequency RRC Re-establishment in FR2 | Same as 7.3.2.1.2 |  | Same as 7.3.2.1.2 |  | Same as 7.3.2.1.2 |
| 17.3.2.1.3 NR SA FR2 RRC re-establishment without serving cell timing | Same as 7.3.2.1.3 |  | Same as 7.3.2.1.3 |  | Same as 7.3.2.1.3 |
| 17.3.2.3.1 NR SA FR2-FR2 RRC connection release with redirection for 2 Rx UE | Same as 7.3.2.3 |  | Same as 7.3.2.3 |  | Same as 7.3.2.3 |
| 17.4.1.1 NR SA FR2 NR UE Transmit Timing accuracy for 2 Rx UE | Same as 7.4.1.1 |  | Same as 7.4.1.1 |  | Same as 7.4.1.1 |
| 17.5.1.1 NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM-RS in non-DRX mode for 2 Rx UE | Same as 7.5.1.1 |  | Same as 7.5.1.1 |  | Same as 7.5.1.1 |
| 17.5.1.2 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM-RS in non-DRX mode for 2 Rx UE | Same as 7.5.1.2 |  | Same as 7.5.1.2 |  | Same as 7.5.1.2 |
| 17.5.1.3 NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM-RS in DRX mode for 2 Rx UE | Same as 7.5.1.1 |  | Same as 7.5.1.1 |  | Same as 7.5.1.1 |
| 17.5.1.4 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM-RS in DRX mode for 2 Rx UE | Same as 7.5.1.4 |  | Same as 7.5.1.4 |  | Same as 7.5.1.4 |
| 17.5.1.5 NR SA FR2 Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM-RS in non-DRX mode for 2 Rx UE | Same as 7.5.1.5 |  | Same as 7.5.1.5 |  | Same as 7.5.1.5 |
| 17.5.1.6 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE | Same as 7.5.1.6 |  | Same as 7.5.1.6 |  | Same as 7.5.1.6 |
| 17.5.1.7 NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE | Same as 7.5.1.7 |  | Same as 7.5.1.7 |  | Same as 7.5.1.7 |
| 17.5.1.8 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE | Same as 7.5.1.8 |  | Same as 7.5.1.8 |  | Same as 7.5.1.8 |
| 17.5.1.9 NR SA FR2 radio link monitoring UE scheduling restrictions | Same as 7.5.1.9 |  | Same as 7.5.1.9 |  | Same as 7.5.1.9 |
| 17.5.2.1 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE | Same as 7.5.5.1 |  | Same as 7.5.5.1 |  | Same as 7.5.5.1 |
| 17.5.2.2 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE | Same as 7.5.5.2 |  | Same as 7.5.5.2 |  | Same as 7.5.5.2 |
| 17.5.2.3 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE | Same as 7.5.5.3 |  | Same as 7.5.5.3 |  | Same as 7.5.5.3 |
| 17.5.2.4 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE | Same as 7.5.5.4 |  | Same as 7.5.5.4 |  | Same as 7.5.5.4 |
| 17.5.2.5 NR SA FR2 Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE | Same as 7.5.5.1 |  | Same as 7.5.5.1 |  | Same as 7.5.5.1 |
| 17.5.3.1.1 NR SA FR2 DCI-based and Timer-based DL active BWP switch for 2 Rx UE | T1 to T3 Noc: -112 dBm/15kHz Ês / Noc: 18dB |  | 0dB 0dB |  | T1 to T3 Noc: -112 dBm/15kHz Ês / Noc: 18dB |
| 17.5.3.2.1 NR SA FR2 RRC-based DL active BWP switch for 2 Rx UE | Same as 17.5.3.1.1 |  | Same as 17.5.3.1.1 |  | Same as 17.5.3.1.1 |
| 17.5.4.1.1 NR SA FR2 MAC-CE based active TCI state switch for a known TCI state for 2 Rx UE | Same as 7.5.8.1.1 |  | Same as 7.5.8.1.1 |  | Same as 7.5.8.1.1 |
| 17.5.4.2.1 NR SA FR2 RRC-CE based active TCI state switch for a known TCI state for 2 Rx UE | Same as 7.5.8.2.1 |  | Same as 7.5.8.2.1 |  | Same as 7.5.8.2.1 |
| 17.5.5.1.1 NR SA FR2 MAC-CE based spatial relation switch associated with known DL-RS for 2 Rx UE | Same as 7.5.9.1.1 |  | Same as 7.5.9.1.1 |  | Same as 7.5.9.1.1 |
| 17.5.5.2.1 NR SA FR2 RRC-based spatial relation switch associated with a known DL-RS for 2 Rx UE | Same as 7.5.9.2.1 |  | Same as 7.5.9.2.1 |  | Same as 7.5.9.2.1 |
| 17.6.1.1 NR SA FR2 event triggered reporting without gap in non-DRX for 2 Rx UE | Same as 7.6.1.1 |  | Same as 7.6.1.1 |  | Same as 7.6.1.1 |
| 17.6.1.2 NR SA FR2 event triggered reporting without gap in DRX for 2 Rx UE | Same as 7.6.1.2 |  | Same as 7.6.1.2 |  | Same as 7.6.1.2 |
| 17.6.1.3 NR SA FR2 event triggered reporting with gap in non-DRX for 2 Rx UE | Same as 7.6.1.1 |  | Same as 7.6.1.1 |  | Same as 7.6.1.1 |
| 17.6.1.4 NR SA FR2 event triggered reporting with gap in DRX for 2 Rx UE | Same as 7.6.1.2 |  | Same as 7.6.1.2 |  | Same as 7.6.1.2 |
| 17.6.2.1 NR SA FR2-FR2 event triggered reporting in non-DRX for 2 Rx UE | Same as 7.6.2.1 |  | Same as 7.6.2.1 |  | Same as 7.6.2.1 |
| 17.6.2.2 NR SA FR2-FR2 event triggered reporting in DRX for 2 Rx UE | Same as 7.6.2.2 |  | Same as 7.6.2.2 |  | Same as 7.6.2.2 |
| 17.6.2.3 NR SA FR2-FR2 event triggered reporting with SSB time index detection in non-DRX for 2 Rx UE | Same as 7.6.2.1 |  | Same as 7.6.2.1 |  | Same as 7.6.2.1 |
| 17.6.2.4 NR SA FR2-FR2 event triggered reporting with SSB time index detection in DRX for 2 Rx UE | Same as 7.6.2.2 |  | Same as 7.6.2.2 |  | Same as 7.6.2.2 |
| 17.6.3.1 NR SA FR2 SSB based L1-R SRP measurement in non-DRX for 2 Rx UE | Same as 7.6.3.1 |  | Same as 7.6.3.1 |  | Same as 7.6.3.1 |
| 17.6.3.2 NR SA FR2 SSB based L1-RSRP measurement in DRX for 2 Rx UE | Same as 7.6.3.1 |  | Same as 7.6.3.1 |  | Same as 7.6.3.1 |
| 17.6.3.3 NR SA FR2 CSI-RS based L1-RSRP measurement in non-DRX for 2 Rx UE | Same as 7.6.3.3 |  | Same as 7.6.3.3 |  | Same as 7.6.3.3 |
| 17.6.3.4 R SA FR2 CSI-RS based L1-RSRP measurement in DRX for 2 Rx UE | Same as 7.6.3.3 |  | Same as 7.6.3.3 |  | Same as 7.6.3.3 |
| 17.7.1.1 NR SA FR2 SS-RSRP measurement accuracy for 2 Rx UE | Same as 7.7.1.1 |  | Same as 7.7.1.1 |  | Same as 7.7.1.1 |
| 17.7.1.2 NR SA FR2-FR2 SS-RSRP measurement accuracy for 2 Rx UE | Same as 7.7.1.2 |  | Same as 7.7.1.2 |  | Same as 7.7.1.2 |
| 17.7.2.1 NR SA FR2 SS-RSRQ measurement accuracy for 2 Rx UE | Same as 7.7.2.1 |  | Same as 7.7.2.1 |  | Same as 7.7.2.1 |
| 17.7.2.2 NR SA FR2-FR2 SS-RSRQ measurement accuracy for 2 Rx UE | Same as 7.7.2.2 |  | Same as 7.7.2.2 |  | Same as 7.7.2.2 |
| 17.7.3.1 NR SA FR2 SSB based L1-RSRP measurement accuracy for 2 Rx UE | Same as 7.7.4.1 |  | Same as 7.7.4.1 |  | Same as 7.7.4.1 |
| 17.7.3.2 NR SA FR2 CSI-RS based L1-RSRP measurement accuracy for 2 Rx UE | Same as 7.7.4.2 |  | Same as 7.7.4.2 |  | Same as 7.7.4.2 |
| 17.7.4.1 NR SA FR2 SS-SINR measurement accuracy for 2 Rx UE | Same as 7.7.3.1 |  | Same as 7.7.3.1 |  | Same as 7.7.3.1 |
| 17.7.4.2 NR SA FR2-FR2 SS-SINR measurement accuracy for 2 Rx UE | Same as 7.7.3.2 |  | Same as 7.7.3.2 |  | Same as 7.7.3.2 |

Table F.1.3.2-9: Derivation of test requirements for E-UTRA – NR inter-RAT test cases with E-UTRA serving cell RRM tests for RedCap

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 18.1.1.1 E-UTRA - NR SA FR1 E-UTRA Cell reselection to higher priority NR target Cell in FR1 | Same as 8.2.1.1 | Same as 8.2.1.1 | Same as 8.2.1.1 |
| 18.2.1.1 E-UTRA - NR SA FR1 handover for 2 Rx UE | Same as 8.3.1.1 | Same as 8.3.1.1 | Same as 8.3.1.1 |
| 18.2.2.1 E-UTRA - NR SA FR1 RRC connection release with redirection for 2 Rx UE | Same as 6.3.2.3.1 | Same as 6.3.2.3.1 | Same as 6.3.2.3.1 |
| 18.3.1.1 E-UTRA - NR SA FR1 event triggered reporting without SSB time index detection in non-DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 18.3.1.2 E-UTRA - NR SA FR1 event triggered reporting without SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 18.3.1.3 E-UTRA - NR SA FR1 event triggered reporting with SSB time index detection in non-DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 18.3.1.4 E-UTRA - NR SA FR1 event triggered reporting with SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 18.3.1.5 E-UTRA - NR SA FR2 event triggered reporting without SSB time index detection in non-DRX | Same as 8.4.2.5 | Same as 8.4.2.5 | Same as 8.4.2.5 |
| 18.3.1.6 E-UTRA - NR SA FR2 event triggered reporting without SSB time index detection in DRX | Same as 8.4.2.6 | Same as 8.4.2.6 | Same as 8.4.2.6 |
| 18.3.1.7 E-UTRA - NR SA FR2 event triggered reporting with SSB time index detection in non-DRX | Same as 8.4.2.6 | Same as 8.4.2.6 | Same as 8.4.2.6 |
| 18.3.1.8 E-UTRA - NR SA FR2 event triggered reporting with SSB time index detection in DRX | Same as 8.4.2.6 | Same as 8.4.2.6 | Same as 8.4.2.6 |

Table F.1.3.2-10: Derivation of test requirements for EN-DC FR1 RRM tests for shared spectrum

| Test |  | Minimum requirement in TS 38.133 [6] |  | Test tolerance (TT) |  | Test requirement in TS 38.533 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10.1.1.1.1 EN-DC FR1 4-step RA type contention-based random access for NR PSCell with CCA |  | Absolute uplink power: Normal conditions ±17dB Relative uplink power step: Normal conditions ±2.5dB Uplink timing: 15kHz SCS Te ±12*64*Tc 30kHz SCS Te ±8*64*Tc |  | 2.1dB 0.7dB 112Tc 112Tc |  | Absolute uplink power: Normal conditions ±19.1dB Relative uplink power step: Normal conditions ±3.2dB Uplink timing: 15kHz SCS Te ±880*Tc 30kHz SCS Te ±624*Tc |  |
| 10.1.1.1.2 EN-DC FR1 4-step RA type non-contention based random access for NR PSCell with CCA |  | Same as 10.1.1.1.1 |  | Same as 10.1.1.1.1 |  | Same as 10.1.1.1.1 |  |
| 10.1.1.1.3 EN-DC FR1 2-step RA type contention-based random access for NR PSCell with CCA |  | Same as 10.1.1.1.1 |  | Same as 10.1.1.1.1 |  | Same as 10.1.1.1.1 |  |
| 10.1.1.1.4 EN-DC FR1 2-step RA type non-contention based random access for NR PSCell with CCA |  | Same as 10.1.1.1.1 |  | Same as 10.1.1.1.1 |  | Same as 10.1.1.1.1 |  |
| 10.1.2 EN-DC FR1 Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +8.00dB Noc2: -98dBm/15kHz Ês3 / Noc: -infinity Ês4 / Noc: -infinity During T2 and T3: Noc1: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +8.00dB Noc2: -98dBm/15kHz Ês3 / Noc: +11.00dB Ês4 / Noc: +11.00dB In signalling: E-UTRAN_A3-offset = 0dB NR_A3-offset = 0dB |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2 and T3: 0dB 0dB 0dB 0dB -1.00dB -1.00dB In signalling: -1.00dB -1.00dB |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +8.00dB Noc2: -98dBm/15kHz Ês3 / Noc: -infinity Ês4 / Noc: -infinity During T2 and T3: Noc1: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +8.00dB Noc2: -98dBm/15kHz Ês3 / Noc: +10.00dB Ês4 / Noc: +10.00dB In signalling: E-UTRAN_A3-offset = -1dB NR_A3-offset = -1dB |  |
| 10.2.1.1 EN-DC FR1 UE Transmit Timing Test with PSCell under DL CCA |  | Same as 4.4.1.1 |  | Same as 4.4.1.1 |  | Same as 4.4.1.1 |  |
| 10.2.2.1 EN-DC FR1 UE Timing Advance Adjustment Accuracy with PSCell under DL CCA |  | Same as 4.4.1.1 |  | Same as 4.4.1.1 |  | Same as 4.4.1.1 |  |
| 10.3.1.2 EN-DC FR1 Radio link monitoring out-of-sync test for PSCell under CCA configured with SSB-based RLM-RS in non-DRX mode |  | SNR during T1: 1dB T2: -7dB T3: -15dB |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |  |
| 10.3.1.3 EN-DC FR1 Radio link monitoring in-sync test for PSCell under CCA configured with SSB-based RLM-RS in non-DRX mode |  | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause D.4A.2.1 |  |
| 10.3.3.1 EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle |  | During T1 to T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  | During T1 to T3: 0dB 0dB 0dB 0dB |  | During T1 to T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |
| 10.3.3.2 EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle |  | Same as 10.3.3.1 |  | Same as 10.3.3.1 |  | Same as 10.3.3.1 |  |
| 10.3.3.3 EN-DC FR1 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA |  | Same as 10.3.3.1 |  | Same as 10.3.3.1 |  | Same as 10.3.3.1 |  |
| 10.3.4.1 EN-DC FR1 EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell under CCA configured with SSB-based BFD and LR in non-DRX mode |  | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4A.2.1 q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |  |
| 10.3.4.2 EN-DC FR1 EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell under CCA configured with SSB-based BFD and LR in DRX mode |  | Same as 10.3.4.1 |  | Same as 10.3.4.1 |  | Same as 10.3.4.1 |  |
| 10.3.5.1 EN-DC FR1 UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB During T2: 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |
| 10.3.5.2.1 EN-DC FR1 E-UTRAN – NR PSCell under CCA FR1 DL active BWP switch in non-DRX in synchronous EN-DC |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB During T2: 0dB 0dB During T3: 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |
| 10.3.5.2.2 EN-DC FR1 E-UTRAN – NR PSCell under CCA FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC |  | Same as 10.3.5.2.1 |  | Same as 10.3.5.2.1 |  | Same as 10.3.5.2.1 |  |
| 10.3.5.3.1 EN-DC FR1 E-UTRAN – NR PSCell under CCA FR1 DL active BWP switch in non-DRX in synchronous EN-DC |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |
| 10.3.6.1 EN-DC FR1 Addition and Release Delay of known NR PSCell on the carrier under CCA |  | Noc: -85dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -93 dBm/SCS |  | -3.6dB 0dB -3dB |  | Noc: -88.6dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS |  |
| 10.4.1.1 EN-DC FR1 Event-triggered reporting tests on PSCC under CCA without gaps under non-DRX |  | During T1: Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 1.2dB 0dB |  | During T1: Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês2 / Noc: +5.20dB Ês3 / Noc: +7.00dB |  |
| 10.4.1.4 EN-DC FR1 Event-triggered reporting tests on PSCC under CCA with per-UE gaps under DRX |  | Same as 10.4.1.1 |  | Same as 10.4.1.1 |  | Same as 10.4.1.1 |  |
| 10.4.2.3 EN-DC FR1 event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used |  | During T1: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  | During T1: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB |  |
| 10.4.2.4 EN-DC FR1 event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  |
| 10.4.2.5 EN-DC FR1 event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  |
| 10.4.2.6 EN-DC FR1 event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  |
| 10.4.2.7 EN-DC FR1 event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  |
| 10.4.2.8 EN-DC FR1 event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  |
| 10.4.2.9 EN-DC FR1 event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  |
| 10.4.2.10 EN-DC FR1 event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  | Same as 10.4.2.3 |  |
| 10.4.3.1 EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is not used |  | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SSB#0 SS-RSRP values ± 10dB Reported SSB#1 SS-RSRP values ± 10dB Reported Differential SS-RSRP values ± 3dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB Via mapping |  | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB SSB#1: RSRP_58 to RSRP_78 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 |  |
| 10.4.3.2 EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is used |  | Same as 10.4.3.1 |  | Same as 10.4.3.1 |  | Same as 10.4.3.1 |  |
| 10.4.3.3 EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used |  | Same as 10.4.3.1 |  | Same as 10.4.3.1 |  | Same as 10.4.3.1 |  |
| 10.4.3.4 EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used |  | Same as 10.4.3.1 |  | Same as 10.4.3.1 |  | Same as 10.4.3.1 |  |
| 10.4.4.1 EN-DC FR1 E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used |  | Same as 12.4.2.1 |  | Same as 12.4.2.1 |  | Same as 12.4.2.1 |  |
| 10.4.4.2 EN-DC FR1 E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used |  | Same as 12.4.2.1 |  | Same as 12.4.2.1 |  | Same as 12.4.2.1 |  |
| 10.4.4.3 EN-DC FR1 NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used |  | Same as 12.4.2.1 |  | Same as 12.4.2.1 |  | Same as 12.4.2.1 |  |
| 10.4.4.4 EN-DC FR1 NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used |  | Same as 12.4.2.1 |  | Same as 12.4.2.1 |  | Same as 12.4.2.1 |  |
| 10.5.1.1 EN-DC FR1 Intra-frequency measurement accuracy on a CCA serving cell |  | TEST CONFIGURATION 1, 2 |  |  |  |  |  |
|  |  | Test 1: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±8dB Test 2: Noc: -107dBm/30kHz Ês2 / Noc: +1.0dB Ês3 / Noc: 0dB Reported RSRP values: ±6dB |  | Test 1: 0dB 0dB +0.4dB Via mapping Test 2: -1.0dB 0dB +0.2dB Via mapping |  | Test 1: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 2: Noc: -118dBm/30kHz Ês2 / Noc: +1.0dB Ês3 / Noc: +0.2dB RSRP_43 to RSRP_55 RSRP_43 to RSRP_55 depending on operating band |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 3 SS-RSRP from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |
| 10.5.1.2 EN-DC FR1 Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell |  | TEST CONFIGURATION 1, 2 |  |  |  |  |  |
|  |  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -103dBm/15kHz Noc2: -111dBm/15kHz Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  | Test 1: -1.35dB -1.35dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc: -103dBm/15kHz Noc1: -111dBm/15kHz Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_47 to RSRP_60 RSRP_40 to RSRP_52 depending on operating band |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3.0 dB wider at each end for Test 1, and 4.5 dB wider at each for Test 2. |  |  |  |  |  |  |  |
| 10.5.2.1 EN-DC FR1 Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell |  | TEST CONFIGURATION 1, 2 |  |  |  |  |  |
|  |  | Test 1: Noc: -91dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: N/A Test 3: Noc: -110dBm/15kHz Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  | Test 1: -1.6dB 0dB 0dB Via mapping Test 2: N/A Test 3: 0dB 0dB 0dB Via mapping |  | Test 1: Noc: -92.6dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: N/A Test 3: Noc: -110dBm/15kHz Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB RSRQ_44 to RSRQ_59 |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the SS-RSRQ values are 1.5dB wider at each end for Test 1, and 0.5 dB wider at each for Tests 2 and 3. |  |  |  |  |  |  |  |
| 10.5.2.2 EN-DC FR1 Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell |  | TEST CONFIGURATION 1, 2 |  |  |  |  |  |
|  |  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: NR_CCA_FR1_I: Noc1: -112dBm/15kHz Noc2: -112dBm/15kHz Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB NR_CCA_FR1_J: Noc1: -111.5dBm/15kHz Noc2: -111.5dBm/15kHz Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: NR_CCA_FR1_I: 0dB 0dB 0dB 0dB NR_CCA_FR1_J: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -113dBm/15kHz Noc2: -113dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: NR_CCA_FR1_I: Noc1: -112dBm/15kHz Noc2: -112dBm/15kHz Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB NR_CCA_FR1_J: Noc1: -111.5dBm/15kHz Noc2: -111.5dBm/15kHz Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 RSRQ_52 to RSRQ_62 depending on operating band |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1.5dB wider at each end. |  |  |  |  |  |  |  |
| 10.5.3.1 EN-DC FR1 Intra-frequency measurement accuracy on PSCC under CCA |  | TEST CONFIGURATION 1, 2 |  |  |  |  |  |
|  |  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Reported SINR values: ±3.5dB Test 2: Noc: -112dBm/15kHz Ês1 / Noc: -4dB Ês2 / Noc: -4dB Reported SINR values: ±3.5dB |  | Test 1: -0.2dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping |  | Test 1: Noc: -93.2dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB SINR_31 to SINR_49 Test 2: Noc: -112dBm/15kHz Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB SINR_28 to SINR_45 |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each end. |  |  |  |  |  |  |  |
| 10.5.3.2 EN-DC FR1 Inter-frequency measurement accuracy on PSCC under CCA |  | TEST CONFIGURATION 1, 2 |  |  |  |  |  |
|  |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Reported SINR values: ±3dB Test 3: Noc1: -115.5dBm/15kHz Noc2: -115.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Reported SINR values: ±3.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_79 to SINR_94 Test 3: Noc1: -115.5dBm/15kHz Noc2: -115.5dBm/15kHz Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB SINR_32 to SINR_49 |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |  |  |  |  |
| 10.5.3.3 EN-DC FR1 Intra-frequency measurement accuracy on SCC under CCA |  | Same as 10.5.3.1 |  | Same as 10.5.3.1 |  | Same as 10.5.3.1 |  |
| 10.5.4.1 EN-DC FR1 SSB based L1-RSRP measurement accuracy with CCA |  |  |  |  |  |  |  |
| Test Configuration 1,2 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±3dB additionally for extreme conditions) Test 2: Noc1: -113dBm/15kHz Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±4.5dB additionally for extreme conditions) |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Ês1 / Noc1: 10dB RSRP_63 to RSRP_84 Test 2: Noc1: -113dBm/15kHz Ês1 / Noc1: -2.2dB RSRP_38 to RSRP_51 |  |
| 10.5.5.1 EN-DC FR1 RSSI measurement accuracy on PSCC with CCA |  | Slots where RSSI is measured: Noc: -87.00dBm/SCS Ês / Noc: -infinity Slots where RSSI is not measured: Noc: -106.00dBm/SCS Ês / Noc: +2.50dB Normal conditions: Reported RSSI values: ±5.5dB Extreme conditions: Reported RSSI values: ±8.5dB |  | Slots where RSSI is measured: 0dB 0dB Slots where RSSI is not measured: 0dB 0dB Via mapping |  | Slots where RSSI is measured: Noc: -87.00dBm/SCS Ês / Noc: -infinity Slots where RSSI is not measured: Noc: -106.00dBm/SCS Ês / Noc: +2.50dB RSSI_35 to RSSI_49 RSSI_32 to RSSI_52 |  |
| 10.5.5.2 EN-DC FR1 RSSI measurement accuracy on SCC with CCA |  | Same as 10.5.5.1 |  | Same as 10.5.5.1 |  | Same as 10.5.5.1 |  |
| 10.5.5.3 EN-DC FR1 Inter-frequency RSSI measurement accuracy on a carrier with CCA |  | Slots where RSSI is measured: Noc1: -106.00dBm/SCS Ês1 / Noc1: +2.50dB Noc2: -87.00dBm/SCS Ês2 / Noc2: -infinity Slots where RSSI is not measured: Noc1: -106.00dBm/SCS Ês1 / Noc1: +2.50dB Noc2: -106.00dBm/SCS Ês2 / Noc2: +2.50dB Normal conditions: Reported RSSI values: ±5.5dB Extreme conditions: Reported RSSI values: ±8.5dB |  | Slots where RSSI is measured: 0dB 0dB 0dB 0dB Slots where RSSI is not measured: 0dB 0dB 0dB 0dB Via mapping |  | Slots where RSSI is measured: Noc1: -106.00dBm/SCS Ês1 / Noc1: +2.50dB Noc2: -87.00dBm/SCS Ês2 / Noc2: -infinity Slots where RSSI is not measured: Noc1: -106.00dBm/SCS Ês1 / Noc1: +2.50dB Noc2: -106.00dBm/SCS Ês2 / Noc2: +2.50dB RSSI_35 to RSSI_49 RSSI_32 to RSSI_52 |  |
| 10.5.6.1 EN-DC FR1 Channel occupancy measurement accuracy on PSCC with CCA |  | Slots where RSSI is measured: Noc: -87.00dBm/SCS Ês / Noc: -infinity Slots where RSSI is not measured: Noc: -106.00dBm/SCS Ês / Noc: +2.50dB channelOccupancyThreshold = -83dBm |  | Slots where RSSI is measured: 0dB 0dB Slots where RSSI is not measured: 0dB 0dB 0dB |  | Slots where RSSI is measured: Noc: -87.00dBm/SCS Ês / Noc: - infinity Slots where RSSI is not measured: Noc: -106.00dBm/SCS Ês / Noc: +2.50dB channelOccupancyThreshold = -83dBm |  |
| 10.5.6.2 EN-DC FR1 Channel occupancy measurement accuracy on SCC with CCA |  | Same as 10.5.6.1 |  | Same as 10.5.6.1 |  | Same as 10.5.6.1 |  |
| 10.5.6.3 EN-DC FR1 Inter-frequency channel occupancy measurement accuracy on a carrier with CCA |  | Slots where RSSI is measured: Noc1: -106.00dBm/SCS Ês1 / Noc1: +2.50dB Noc2: -87.00dBm/SCS Ês2 / Noc2: -infinity Slots where RSSI is not measured: Noc1: -106.00dBm/SCS Ês1 / Noc1: +2.50dB Noc2: -106.00dBm/SCS Ês2 / Noc2: +2.50dB channelOccupancyThreshold = -83dBm |  | Slots where RSSI is measured: 0dB 0dB 0dB 0dB Slots where RSSI is not measured: 0dB 0dB 0dB 0dB 0dB |  | Slots where RSSI is measured: Noc1: -106.00dBm/SCS Ês1 / Noc1: +2.50dB Noc2: -87.00dBm/SCS Ês2 / Noc2: -infinity Slots where RSSI is not measured: Noc1: -106.00dBm/SCS Ês1 / Noc1: +2.50dB Noc2: -106.00dBm/SCS Ês2 / Noc2: +2.50dB channelOccupancyThreshold = -83dBm |  |

Table F.1.3.2-11: Derivation of test requirements for SA FR1 RRM tests for shared spectrum

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 11.1.1.1 NR SA FR1 Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |
| 11.1.1.2 NR SA FR1 Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -2dB 1.6dB 0.4dB During T2: 0dB 0dB 1.6dB 0dB During T3: 0dB 0dB 1.6dB 1.6dB | During T1: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB |
| 11.1.2.1 Cell re-selection to NR with source NR carrier frequency under CCA | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -2dB 1.6dB 0.4dB During T2: 0dB 0dB 1.6dB 0dB During T3: 0dB 0dB 1.6dB 1.6dB | During T1: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB |
| 11.1.3.1 NR SA FR1 Cell re-selection to NR with source NR carrier frequency under CCA | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -2dB 1.6dB 0.4dB During T2: 0dB 0dB 1.6dB 0dB During T3: 0dB 0dB 1.6dB 1.6dB | During T1: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB |
| 11.1.4.1 NR SA FR1 Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 1.6dB 1.6dB During T3: 0dB -2dB 1.6dB 0.4dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB During T3: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB |
| 11.1.4.2 NR SA FR1 Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |
| 11.2.1.1 NR SA FR1 Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell | During T1: Noc: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB -1dB During T3: 0dB 0dB -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |
| 11.2.1.2 NR SA FR1 Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB |
| 11.2.1.3 NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB |
| 11.2.1.4 NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB -1dB During T3: 0dB 0dB 0dB -1dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |
| 11.2.1.5 NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB |
| 11.2.1.6 NR SA FR1 Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB |
| 11.2.1.7 NR SA FR1 carrier under CCA - E-UTRAN handover with known target cell | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +8dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +8dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.55dB During T3: 0dB 0dB 0dB 1.55dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +9.55dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +9.55dB |
| 11.2.1.8 NR SA FR1 carrier under CCA - E-UTRAN handover with unknown target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +7dB |
| 11.2.2.1.1 NR SA FR1 Intra-frequency RRC Re-establishment with CCA in FR1 | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |
| 11.2.2.1.2 NR SA FR1 Inter-frequency RRC Re-establishment with CCA in FR1 | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: +4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +4dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: +4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +4dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +4dB |
| 11.2.2.1.3 NR SA FR1 Intra-frequency RRC Re-establishment with CCA without serving cell timing | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |
| 11.2.2.1.4 NR SA FR1 Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB |
| 11.2.2.2.1 NR SA FR1 4-step RA type contention-based random access for NR PCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 11.2.2.2.2 NR SA FR1 4-step RA type non-contention based random access for NR PSCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 11.2.2.2.3 NR SA FR1 2-step RA type contention-based random access for NR PCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 11.2.2.2.4 NR SA FR1 2-step RA type non-contention-based random access for NR PCell with CCA | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 | Same as 10.1.1.1.1 |
| 11.2.2.3.1 NR SA FR1 Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA | Same as 11.2.2.3.2 | Same as 11.2.2.3.2 | Same as 11.2.2.3.2 |
| 11.2.2.3.2 NR SA FR1 Redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4 dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4 dB |
| 11.3.1.1 NR SA FR1 UE Transmit Timing Test with PCell under DL CCA | Same as 6.4.1.1 | Same as 6.4.1.1 | Same as 6.4.1.1 |
| 11.3.2.1 NR SA FR1 UE Timing Advance Adjustment Accuracy with PCell under DL CCA | Same as 6.4.3.1 | Same as 6.4.3.1 | Same as 6.4.3.1 |
| 11.4.1.2 NR SA FR1 Radio link monitoring out-of-sync test for PCell under CCA configured with SSB-based RLM-RS in non-DRX mode | Same as 10.3.1.2 | Same as 10.3.1.2 | Same as 10.3.1.2 |
| 11.4.1.3 NR SA FR1 Radio link monitoring in-sync test for PCell under CCA configured with SSB-based RLM-RS in non-DRX mode | Same as 10.3.1.3 | Same as 10.3.1.3 | Same as 10.3.1.3 |
| 11.4.3.1 NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle | Same as 10.3.3.1 | Same as 10.3.3.1 | Same as 10.3.3.1 |
| 11.4.3.2 NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle | Same as 10.3.3.1 | Same as 10.3.3.1 | Same as 10.3.3.1 |
| 11.4.3.3 NR SA FR1 SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA | Same as 10.3.3.1 | Same as 10.3.3.1 | Same as 10.3.3.1 |
| 11.4.4.1 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell under CCA configured with SSB-based BFD and LR in non-DRX mode | Same as 10.3.4.1 | Same as 10.3.4.1 | Same as 10.3.4.1 |
| 11.4.4.2 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell under CCA configured with SSB-based BFD and LR in DRX mode | Same as 10.3.4.1 | Same as 10.3.4.1 | Same as 10.3.4.1 |
| 11.4.5.1 NR SA FR1 UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA | Same as 10.3.5.1 | Same as 10.3.5.1 | Same as 10.3.5.1 |
| 11.4.5.2.1 NR SA FR1 - NR FR1 DL active BWP switch of PCell under CCA with non-DRX | Same as 10.3.5.2.1 | Same as 10.3.5.2.1 | Same as 10.3.5.2.1 |
| 11.4.5.2.2 NR SA FR1 DCI-based DL active BWP switch with non-DRX under CCA | Same as 10.3.5.2.1 | Same as 10.3.5.2.1 | Same as 10.3.5.2.1 |
| 11.4.5.3.1 NR SA FR1 DL active BWP switch of Cell with non-DRX | Same as 10.3.5.3.1 | Same as 10.3.5.3.1 | Same as 10.3.5.3.1 |
| 11.5.1.1 SA FR1 Event-triggered reporting tests on PCC without gaps under non-DRX | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 11.5.1.2 SA FR1 Event-triggered reporting tests on PCC without gaps under DRX | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 11.5.2.3 NR SA FR1-FR1 Event triggered reporting tests with CCA without SSB time index detection when DRX is not used | Same as 10.4.2.3 | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.4 NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used | Same as 10.4.2.3 | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.5 NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used | Same as 10.4.2.3 | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.6 NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used | Same as 10.4.2.3 | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.7 NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used | Same as 10.4.2.3 | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.8 NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used | Same as 10.4.2.3 | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.9 NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used | Same as 10.4.2.3 | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.2.10 NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used | Same as 10.4.2.3 | Same as 10.4.2.3 | Same as 10.4.2.3 |
| 11.5.3.1 NR SA FR1 - E-UTRAN event-triggered reporting in non-DRX | During T1: Noc1: -106dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +18dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -2dB Ês2 / Noc2: +17 dB | During T1: 0dB 0dB +1.55dB 0dB During T2: 0dB 0dB 0dB +1.55dB | During T1: Noc1: -106dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +19.55dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -2dB Ês2 / Noc2: +18.55 dB |
| 11.5.3.2 NR SA FR1 - E-UTRAN event-triggered reporting in DRX | Same as 11.5.3.1 | Same as 11.5.3.1 | Same as 11.5.3.1 |
| 11.5.4.1 NR SA FR1 SSB based L1-RSRP measurement when DRX is not used | Same as 10.4.3.1 | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 11.5.4.2 NR SA FR1 SSB based L1-RSRP measurement when DRX is used | Same as 10.4.3.1 | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 11.5.4.3 NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used | Same as 10.4.3.1 | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 11.5.4.4 NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used | Same as 10.4.3.1 | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 11.6.1.1 NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA | Same as 10.5.1.1 | Same as 10.5.1.1 | Same as 10.5.1.1 |
| 11.6.1.2 NR SA FR1 Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA | Test 2: Noc: -94.00dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: NR_CCA_FR1_I: Noc: -110.00dBm/30kHz Ês1 / Noc: +1.0dB Ês2 / Noc: +0.0dB NR_CCA_FR1_J: Noc: -109.50dBm/30kHz Ês1 / Noc: +1.0dB Ês2 / Noc: +0.0dB Normal conditions: Reported RSRP values: ±4.5dB | Test 2: 0dB 0dB +0.4dB Via mapping Test 2: NR_CCA_FR1_I: -1.0dB 0dB +0.2dB NR_CCA_FR1_J: -1.5dB 0dB +0.2dB Via mapping | Test 1: Noc: -91dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 2: NR_CCA_FR1_I: Noc: -111dBm/30kHz Ês1 / Noc: +1.0dB Ês2 / Noc: +0.2dB NR_CCA_FR1_J: Noc: -111dBm/30kHz Ês1 / Noc: +1.0dB Ês2 / Noc: +0.2dB NR_CCA_FR1_I: RSRP_43 to RSRP_55 NR_CCA_FR1_J: RSRP_43 to RSRP_55 |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |
| 11.6.2.1 NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA | Same as 10.5.2.1 | Same as 10.5.2.1 | Same as 10.5.2.1 |
| 11.6.2.2 NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA | Same as 10.5.2.2 | Same as 10.5.2.2 | Same as 10.5.2.2 |
| 11.6.2.3 NR SA FR1 Intra-frequency measurement accuracy on SCC | Same as 10.5.2.1 | Same as 10.5.2.1 | Same as 10.5.2.1 |
| 11.6.2.4 NR SA FR1 Inter-frequency measurement accuracy | FFS | FFS | FFS |
| 11.6.3.1 NR SA FR1 Intra-frequency SS-SINR measurement accuracy on a carrier frequency with CCA | Same as 10.5.3.1 | Same as 10.5.3.1 | Same as 10.5.3.1 |
| 11.6.3.2 NR SA FR1 Inter-frequency SS-SINR measurement accuracy on a carrier frequency with CCA | Same as 10.5.3.2 | Same as 10.5.3.2 | Same as 10.5.3.2 |
| 11.6.3.3 NR SA FR1 Intra-frequency SS-SINR measurement accuracy on SCC on a carrier frequency with CCA | Same as 10.5.3.1 | Same as 10.5.3.1 | Same as 10.5.3.1 |
| 11.6.3.4 NR SA FR1 Intra-frequency SS-SINR measurement accuracy on SCC on a carrier frequency with CCA | FFS | FFS | FFS |
| 11.6.4.1 NR SA FR1 SSB based L1-RSRP measurement accuracy with CCA | Same as 10.5.4.1 | Same as 10.5.4.1 | Same as 10.5.4.1 |
| 11.6.5.1 NR SA FR1 Intra-frequency RSSI measurement accuracy on PCC with CCA | Same as 10.5.5.1 | Same as 10.5.5.1 | Same as 10.5.5.1 |
| 11.6.5.2 NR SA FR1 Intra-frequency RSSI measurement accuracy on SCC with CCA | Same as 10.5.5.1 | Same as 10.5.5.1 | Same as 10.5.5.1 |
| 11.6.5.3 NR SA FR1 Inter-frequency RSSI measurement accuracy on a carrier with CCA | Same as 10.5.5.3 | Same as 10.5.5.3 | Same as 10.5.5.3 |
| 11.6.6.1 NR SA FR1 Intra-frequency channel occupancy measurement accuracy on PCC with CCA | Same as 10.5.6.1 | Same as 10.5.6.1 | Same as 10.5.6.1 |
| 11.6.6.2 NR SA FR1 Intra-frequency channel occupancy measurement accuracy on SCC with CCA | Same as 10.5.6.1 | Same as 10.5.6.1 | Same as 10.5.6.1 |
| 11.6.6.3 NR SA FR1 Inter-frequency channel occupancy measurement accuracy on a carrier with CCA | Same as 10.5.6.3 | Same as 10.5.6.3 | Same as 10.5.6.3 |

Table F.1.3.2-12: Derivation of test requirements for E-UTRAN – SA FR1 Shared Spectrum test cases

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 12.1.1.1 E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -1.90dB 0dB 0.35dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 1.55dB 1.55dB | During T1: Noc1: -98dBm/15kHz Noc2: -99.90dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -3.65dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.55dB Ês2 / Noc2: +13.55dB |
| 12.2.1.1 E-UTRAN - NR with CCA handover with known target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB -2.55dB 0.55dB During T3: 0dB 0dB -2.55dB 0.55dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4.45dB Ês2 / Noc2: +0.55dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4.45dB Ês2 / Noc2: +0.55dB |
| 12.4.2.1 E-UTRAN-NR FR1 inter-RAT event triggered reporting without SSB time index detection when DRX is not used under CCA | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +7dB |
| 12.4.2.2 E-UTRAN-NR FR1 inter-RAT event triggered reporting without SSB time index detection when DRX is used under CCA | Same as 12.4.2.1 | Same as 12.4.2.1 | Same as 12.4.2.1 |
| 12.4.2.3 E-UTRAN-NR FR1 inter-RAT event triggered reporting with SSB time index detection when DRX is not used under CCA | Same as 12.4.2.1 | Same as 12.4.2.1 | Same as 12.4.2.1 |
| 12.4.2.4 E-UTRAN-NR FR1 inter-RAT event triggered reporting with SSB time index detection when DRX is used under CCA | Same as 12.4.2.1 | Same as 12.4.2.1 | Same as 12.4.2.1 |

Table F.1.3.2-13: Derivation of test requirements for SA FR1 with SCell in Shared Spectrum test cases

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 13.2.2.1 NR SA FR1 SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle | Same as 10.3.3.1 | Same as 10.3.3.1 | Same as 10.3.3.1 |
| 13.2.2.2 NR SA FR1 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle | Same as 10.3.3.1 | Same as 10.3.3.1 | Same as 10.3.3.1 |
| 13.2.2.3 NR SA FR1 SCell Activation and Deactivation of unknown SCell under CCA | Same as 10.3.3.1 | Same as 10.3.3.1 | Same as 10.3.3.1 |
| 13.3.1.1 NR SA FR1 Event-triggered reporting tests on SCC under CCA without gaps under non-DRX | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB Ês3 / Noc: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB Ês3 / Noc: +7dB | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB Ês3 / Noc: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB Ês3 / Noc: +7dB |
| 13.3.1.2 NR SA FR1 Event-triggered reporting tests on SCC under CCA without gaps under DRX | Same as 13.3.1.1 | Same as 13.3.1.1 | Same as 13.3.1.1 |
| 13.3.1.3 NR SA FR1 Event-triggered reporting tests on SCC under CCA with per-UE gaps under non-DRX | Same as 13.3.1.1 | Same as 13.3.1.1 | Same as 13.3.1.1 |
| 13.3.1.4 NR SA FR1 Event-triggered reporting tests on SCC under CCA with per-UE gaps under DRX | Same as 13.3.1.1 | Same as 13.3.1.1 | Same as 13.3.1.1 |
| 13.3.2.3 NR SA FR1 Event triggered reporting tests on a carrier with CCA without SSB time index detection when DRX is not used | During T1: Noc1: -98dBm/15kHz Noc3: -98dBm/15kHz Ês1 / Noc1: +4dB Ês3 / Noc3: -infinity During T2: Noc1: -98dBm/15kHz Noc3: -98dBm/15kHz Ês1 / Noc1: +4dB Ês3 / Noc3: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | D During T1: Noc1: -98dBm/15kHz Noc3: -98dBm/15kHz Ês1 / Noc1: +4dB Ês3 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc3: -98dBm/15kHz Ês1 / Noc1: +4dB Ês3 / Noc3: +7dB |
| 13.3.2.4 NR SA FR1 Event triggered reporting tests on a carrier with CCA without SSB time index detection when DRX is used | Same as 13.3.2.3 | Same as 13.3.2.3 | Same as 13.3.2.3 |
| 13.3.2.5 NR SA FR1 Event triggered reporting tests on a carrier with CCA with SSB time index detection when DRX is not used | Same as 13.3.2.3 | Same as 13.3.2.3 | Same as 13.3.2.3 |
| 13.3.2.6 NR SA FR1 Event triggered reporting tests on a carrier with CCA with SSB time index detection when DRX is used | Same as 13.3.2.3 | Same as 13.3.2.3 | Same as 13.3.2.3 |
| 13.3.3.1 NR SA FR1 SSB based L1-RSRP measurement for SCell under CCA when DRX is not used | Same as 10.4.3.1 | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 13.3.3.2 NR SA FR1 SSB based L1-RSRP measurement for SCell under CCA when DRX is used | Same as 10.4.3.1 | Same as 10.4.3.1 | Same as 10.4.3.1 |
| 13.4.1.1 NR SA FR1 SS-RSRP measurement accuracy on a carrier with CCA | Same as 10.5.1.1 | Same as 10.5.1.1 | Same as 10.5.1.1 |
| 13.4.2.1 NR SA FR1 SS-RSRQ measurement accuracy on a carrier with CCA | Same as 10.5.2.1 | Same as 10.5.2.1 | Same as 10.5.2.1 |
| 13.4.3.1 NR SA FR1 SS-SINR measurement accuracy on a carrier with CCA | Same as 10.5.3.1 | Same as 10.5.3.1 | Same as 10.5.3.1 |
| 13.4.4.1 NR SA FR1 L1-RSRP measurement accuracy for beam reporting on a carrier with CCA | Same as 10.5.4.1 | Same as 10.5.4.1 | Same as 10.5.4.1 |
| 13.4.5.1 NR SA FR1 RSSI measurement accuracy on a carrier with CCA | Same as 10.5.5.1 | Same as 10.5.5.1 | Same as 10.5.5.1 |
| 13.4.5.2 NR SA FR1-FR1 RSSI measurement accuracy on a carrier with CCA | Same as 10.5.5.3 | Same as 10.5.5.3 | Same as 10.5.5.3 |
| 13.4.6.1 NR SA FR1 channel occupancy measurement accuracy on a carrier with CCA | Same as 10.5.6.1 | Same as 10.5.6.1 | Same as 10.5.6.1 |
| 13.4.6.2 NR SA FR1-FR1 channel occupancy measurement accuracy on a carrier with CCA | Same as 10.5.6.3 | Same as 10.5.6.3 | Same as 10.5.6.3 |

Table F.1.3.2-14: Derivation of test requirements for SA FR1 Satellite Access test cases

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 14.1.1 NR SA FR1 Cell Reselection for Satellite Access | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |
| 14.1.2 NR SA FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access | Same as 14.1.1 | Same as 14.1.1 | Same as 14.1.1 |
| 14.1.3 NR SA FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +16dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +16.45dB |
| 14.1.4 NR SA FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access | Same as 14.1.3 | Same as 14.1.3 | Same as 14.1.3 |
| 14.1.5 NR SA FR1-FR1 Cell Reselection for Satellite Access | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -2dB 1.6dB 0.4dB During T2: 0dB 0dB 1.6dB 0dB During T3: 0dB 0dB 1.6dB 1.6dB | During T1: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB |
| 14.1.6 NR SA FR1-FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -2dB 1.6dB 0.4dB During T2: 0dB 0dB 1.6dB 0dB During T3: 0dB 0dB 1.6dB 1.6dB | During T1: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB |
| 14.1.7 NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +16dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: +16dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +16dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: +16dB |
| 14.1.8 NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access | Same as 14.1.7 | Same as 14.1.7 | Same as 14.1.7 |
| 14.1.9 NR SA FR1-FR1 Cell reselection for UE fulfilling low mobility relaxed measurement criterion for Satellite Access | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 48dB Thresh_X_LowP of Cell 1 = 50dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 44dB SSearchDeltaP of Cell 1 = 3dB SSearchDeltaP of Cell 2 = 3dB | During T1: 0dB 0dB 0dB 0.3dB During T2: 0dB 0dB 0dB -2.25dB In signalling: 0dB -4dB -2dB 0dB 0dB 2dB 0dB 12dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -3.7dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +9.75dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 44dB Thresh_X_LowP of Cell 1 = 48dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 46dB SSearchDeltaP of Cell 1 = 3dB SSearchDeltaP of Cell 2 = 15dB |
| 14.1.10 NR SA FR1-FR1 Cell reselection for UE fulfilling not-at-cell edge relaxed measurement criterion for Satellite Access | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB In signalling: SSearchThresholdP of Cell 1 = 50dB SSearchThresholdP of Cell 2 = 50dB | During T1: 0dB 2dB -2dB 0.35dB During T2: 0dB 0dB -2dB 4dB In signalling: 0dB -16dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +16dB Noc2: -100dBm/15kHz Ês2 / Noc2: -3.65dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -100dBm/15kHz Ês2 / Noc2: +16dB In signalling: SSearchThresholdP of Cell 1 = 50dB SSearchThresholdP of Cell 2 = 34dB |
| 14.2.1.1 NR SA FR1 Handover for Satellite Access | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB Signaled A3-Offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB -1dB During T3: 0dB 0dB -1dB -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +10.00dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +10.00dB Signaled A3-Offset: -1dB |
| 14.2.1.2 NR SA FR1-FR1 Handover for Satellite Access | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +9.00dB During T3: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +9.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +9.00dB During T3: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +9.00dB |
| 14.2.1.3 NR SA FR1 Time-based Conditional Handover for NR Satellite Access | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB Signaled A3-Offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB -1dB -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +10.00dB Signaled A3-Offset: -1dB |
| 14.2.1.4 NR SA FR1-FR1 Time-based Conditional Handover for NR Satellite Access | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +9.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc2: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +9.00dB |
| 14.2.1.5 NR SA FR1 Distance-based Conditional Handover for Satellite Access | Same as 14.2.1.3 | Same as 14.2.1.3 | Same as 14.2.1.3 |
| 14.2.1.6 NR SA FR1-FR1 Distance-based Conditional Handover for Satellite Access | Same as 14.2.1.4 | Same as 14.2.1.4 | Same as 14.2.1.4 |
| 14.2.2.1.1 NR SA FR1 RRC Re-establishment for Satellite Access | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |
| 14.2.2.1.2 NR SA FR1-FR1 RRC Re-establishment for Satellite Access | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB |
| 14.3.1.1 NR SA FR1 UE transmit timing accuracy for Satellite Access | Test 1 and Test 2: Uplink timing: ±29*64 Tc Ês / Noc: +3.00dB Noc = -98dBm/15kHz Test 1 (no DRX): Max step size Tq: 5.5*64*Tc Min adjust rate Tp: 5.5*64*Tc Max adjust rate: 5.5*64*Tc | Test 1 and Test 2: ±5.5*64*Tc +0.3 dB 0 dB Test 1 (no DRX): +1.375*64Tc -4.475*64*Tc +1.975*64*Tc | Test 1 and Test 2: Uplink timing: ±34.5*64*Tc Ês / Noc: +3.30dB Noc = -98dBm/15kHz Test 1 (no DRX): Max step size Tq: 6.875*64*Tc Min adjust rate: 1.025*64*Tc Max adjust rate: 7.475*64*Tc |
| 14.3.2.1 NR SA FR1 timing advance adjustment accuracy for Satellite Access | Ês / Noc: +3.00dB Noc = -98dBm/15kHz Timing Advance Accuracy: ±256*Tc | +0.3 dB 0 dB ±88*Tc | Ês / Noc: +3.30dB Noc = -98dBm/15kHz Timing Advance Accuracy: ±344*Tc |
| 14.4.1.1 NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with SSB-based RLM-RS in non-DRX mode for Satellite Access | SNR during T1: 1dB T2: -7dB T3: -15dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |
| 14.4.1.2 NR SA FR1 Radio Link Monitoring In-sync for PCell configured with SSB-based RLM-RS in non-DRX mode for Satellite Access | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB |
| 14.4.1.3 NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with SSB-based RLM-RS in DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.1.4 NR SA FR1 Radio Link Monitoring In-sync for PCell configured with SSB-based RLM-RS in DRX mode for Satellite Access | Same as 14.4.1.2 | Same as 14.4.1.2 | Same as 14.4.1.2 |
| 14.4.1.5 NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with CSI-RS-based RLM-RS in non-DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.1.6 NR SA FR1 Radio Link Monitoring In-sync for PCell configured with CSI-RS-based RLM-RS in non-DRX mode for Satellite Access | Same as 14.4.1.2 | Same as 14.4.1.2 | Same as 14.4.1.2 |
| 14.4.1.7 NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with CSI-RS-based RLM-RS in DRX mode for Satellite Access | Same as 14.4.1.1 | Same as 14.4.1.1 | Same as 14.4.1.1 |
| 14.4.1.8 NR SA FR1 Radio Link Monitoring In-sync for PCell configured with CSI-RS-based RLM-RS in DRX mode for Satellite Access | Same as 14.4.1.2 | Same as 14.4.1.2 | Same as 14.4.1.2 |
| 14.4.2.1 NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with SSB-based BFD and LR in non-DRX mode for Satellite Access | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |
| 14.4.2.2 NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with SSB-based BFD and LR in DRX mode for Satellite Access | Same as 14.4.2.1 | Same as 14.4.2.1 | Same as 14.4.2.1 |
| 14.4.2.3 NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in non-DRX mode for Satellite Access | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 CSI-RS during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |
| 14.4.2.4 NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in DRX mode for Satellite Access | Same as 14.4.2.3 | Same as 14.4.2.3 | Same as 14.4.2.3 |
| 14.4.3.1.1 NR SA FR1 DCI-based DL active BWP switch with non-DRX for Satellite Access | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB | During T1: 0dB 0dB | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |
| 14.4.3.2.1 NR SA FR1 RRC-based DL active BWP switch with non-DRX for Satellite Access | Same as 14.4.3.1.1 | Same as 14.4.3.1.1 | Same as 14.4.3.1.1 |
| 14.4.4.1 NR SA FR1 UE specific CBW change on PCell in non-DRX for Satellite Access | Same as 14.4.3.1.1 | Same as 14.4.3.1.1 | Same as 14.4.3.1.1 |
| 14.5.1.1 SA FR1 event triggered reporting tests without gap under non-DRX for NR satellite access | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |
| 14.5.1.2 SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.1.3 SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.1.4 SA FR1 event triggered reporting tests with single measurement gap under non-DRX for satellite access for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.1.5 SA FR1 event triggered reporting tests with FNO concurrent gaps under DRX for satellite access for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.1.6 SA FR1 event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access for NR satellite access | Same as 14.5.1.1 | Same as 14.5.1.1 | Same as 14.5.1.1 |
| 14.5.2.1 NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +7.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +7.00dB |
| 14.5.2.2 NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access | Same as 14.5.2.1 | Same as 14.5.2.1 | Same as 14.5.2.1 |
| 14.5.2.3 NR SA FR1-FR1 Event-triggered reporting with SSB time index detection when DRX is not used with single gap for satellite access | Same as 14.5.2.1 | Same as 14.5.2.1 | Same as 14.5.2.1 |
| 14.5.2.4 NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: -infinity Ês3 / Noc2: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +7.00dB Ês3 / Noc2: +7.00dB | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: -infinity Ês3 / Noc2: -infinity During T2: Freq 1 Noc1: -98dBm/15kHz Freq 2 Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +7.00dB Ês3 / Noc2: +7.00dB |
| 14.5.2.6 NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access | Same as 14.5.2.4 | Same as 14.5.2.4 | Same as 14.5.2.4 |
| 14.6.3.1 NR SA FR1 SS-SINR Measurement Accuracy for Satellite Access | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz Ês1 / Noc: -4dB Ês2 / Noc: -4dB Reported SINR values: ±3.5dB | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB SINR_28 to SINR_45 |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each end. |  |  |  |
| 14.6.3.2 NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Absolute Reported SINR values: ±3dB Relative Reported SINR values: ±3.5dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Absolute Reported SINR values: ±3dB Relative Reported SINR values: ±3.5dB Test 3: Noc1: -119.5dBm/15kHz Noc2: -119.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Absolute Reported SINR values: ±3.5dB Relative Reported SINR values: ±4dB | Test 1: 0dB 0dB 0dB 0dB Via mapping Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Via mapping Test 3: 0dB 0dB 1.9dB 1.9dB Via mapping Via mapping | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_35 to SINR_51 SINR_x-10 to SINR_x+10 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_79 to SINR_94 SINR_x-10 to SINR_x+10 Test 3: Noc1: -119.5dBm/15kHz Noc2: -119.5dBm/15kHz Ês1 / Noc1: -2.1dB Ês2 / Noc2: -2.1dB SINR_34 to SINR_51 SINR_x-11 to SINR_x+11 |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |
| 14.6.4.1 NR SA FR1 SSB based L1-RSRP Measurement Accuracy for Satellite Access | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±3dB additionally for extreme conditions) Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB Test 2: Noc1: -117dBm/15kHz Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±4.5dB additionally for extreme conditions) Reported differential L1-RSRP values for normal conditions: -4.6dB to 0dB Reported differential L1-RSRP values for extreme conditions: -5.6dB to 0dB | Test 1: 0dB 0dB Via mapping Via mapping Via mapping Test 2: 0dB 0.8dB Via mapping Via mapping Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: 10dB RSRP_62 to RSRP_82 DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 Test 2: Noc1: -117dBm/15kHz Ês1 / Noc1: -2.2dB RSRP_31 to RSRP_44 DIFFRSRP_0 to DIFFRSRP_2 DIFFRSRP_0 to DIFFRSRP_2 |
| 14.6.4.2 NR SA FR1 CSI-RS based L1-RSRP Measurement Accuracy for Satellite Access | Same as 14.6.4.1 | Same as 14.6.4.1 | Same as 14.6.4.1 |

Table F.1.3.2-15: Derivation of test requirements for RRM requirements for SA FR1 Air-to-ground test cases

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 19.2.1.1 Intra-frequency handover from FR1 to FR1; known target cell | Same as 6.3.1.1 | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 19.2.1.2 Inter-frequency handover from FR1 to FR1; unknown target cell | Same as 6.3.1.3 | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 19.6.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.2.1 NR SA FR1 SS-RSRQ absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.3.1 NR SA FR1 SS-SINR absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |

Annex G (normative):
Statistical testing

# G.1	General

The test requirements are expressed as absolute requirements with a single value stating the requirement or expressed as a success rate. The statistical nature depends on the type of test requirement. Some have large statistical variations, while others are not statistical in nature at all. When testing a parameter with a statistical nature, a confidence level is set. This establishes the probability that a Device Under Test (DUT) passing the test actually meets the test requirement and determines how many times a test have to be repeated and what the pass and fail criteria is. This Annex describes how to set the statistical significance.

# G.2	Statistical testing of delay and UE measurement performance in RRM tests

## G.2.1	General

The RRM tests are either of deterministic or of statistical nature. The pass fail limits in tests of statistical nature are expressed as a limit (e.g. delay limit) and a success ratio applicable for the limit. The success ratio is 90% uniform (the complement is the error ratio ER = 10%).

## G.2.2	Design of the test

The test is defined by the following design principles (see TS 36.521-1 clause G.X, Theory):

1) The early decision concept is applied.

2) A second limit is introduced: bad DUT factor M>1

To decide the test pass:

Supplier risk is applied based on the bad DUT quality

To decide the test fails

Customer risk is applied based on the specified DUT quality

The test is defined by the following parameters:

1) Limit ER = 0.1 (success ratio = 90%)

2) Bad DUT factor M=1.5 (selectivity)

3) Confidence level CL = 95% (for specified DUT and bad DUT-quality)

## G.2.3	Numerical definition of the pass fail limits

Editor’s Note:

-	Further investigate the technical details behind this statistical method to ensure that this is applicable for FR2 radiated test cases.

Table G.2.3-1: pass fail limits

| ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 33 | NA | 43 | 408 | 283 | 86 | 737 | 644 | 129 | 1056 | 1021 |
| 1 | 46 | NA | 44 | 416 | 291 | 87 | 745 | 653 | 130 | 1064 | 1030 |
| 2 | 58 | NA | 45 | 424 | 299 | 88 | 752 | 661 | 131 | 1071 | 1039 |
| 3 | 69 | NA | 46 | 432 | 307 | 89 | 760 | 670 | 132 | 1078 | 1048 |
| 4 | 79 | NA | 47 | 440 | 315 | 90 | 767 | 679 | 133 | 1086 | 1057 |
| 5 | 89 | NA | 48 | 447 | 324 | 91 | 775 | 687 | 134 | 1093 | 1066 |
| 6 | 99 | NA | 49 | 455 | 332 | 92 | 782 | 696 | 135 | 1100 | 1074 |
| 7 | 109 | NA | 50 | 463 | 340 | 93 | 790 | 705 | 136 | 1108 | 1083 |
| 8 | 118 | NA | 51 | 471 | 348 | 94 | 797 | 713 | 137 | 1115 | 1092 |
| 9 | 127 | NA | 52 | 478 | 356 | 95 | 804 | 722 | 138 | 1122 | 1101 |
| 10 | 136 | 39 | 53 | 486 | 365 | 96 | 812 | 731 | 139 | 1130 | 1110 |
| 11 | 145 | 45 | 54 | 494 | 373 | 97 | 819 | 739 | 140 | 1137 | 1119 |
| 12 | 154 | 51 | 55 | 502 | 381 | 98 | 827 | 748 | 141 | 1144 | 1128 |
| 13 | 163 | 58 | 56 | 509 | 389 | 99 | 834 | 757 | 142 | 1152 | 1137 |
| 14 | 172 | 64 | 57 | 517 | 398 | 100 | 842 | 766 | 143 | 1159 | 1147 |
| 15 | 180 | 71 | 58 | 525 | 406 | 101 | 849 | 774 | 144 | 1166 | 1155 |
| 16 | 189 | 78 | 59 | 532 | 414 | 102 | 857 | 783 | 145 | 1174 | 1164 |
| 17 | 197 | 85 | 60 | 540 | 423 | 103 | 864 | 792 | 146 | 1181 | 1173 |
| 18 | 206 | 92 | 61 | 548 | 431 | 104 | 871 | 801 | 147 | NA | 1182 |
| 19 | 214 | 99 | 62 | 555 | 440 | 105 | 879 | 809 | 148 |  |  |
| 20 | 223 | 106 | 63 | 563 | 448 | 106 | 886 | 818 | 149 |  |  |
| 21 | 231 | 113 | 64 | 571 | 456 | 107 | 894 | 827 | 150 |  |  |
| 22 | 239 | 120 | 65 | 578 | 465 | 108 | 901 | 836 | 151 |  |  |
| 23 | 248 | 128 | 66 | 586 | 473 | 109 | 909 | 844 | 152 |  |  |
| 24 | 256 | 135 | 67 | 594 | 482 | 110 | 916 | 853 | 153 |  |  |
| 25 | 264 | 142 | 68 | 601 | 490 | 111 | 923 | 862 | 154 |  |  |
| 26 | 272 | 150 | 69 | 609 | 499 | 112 | 931 | 871 | 155 |  |  |
| 27 | 281 | 157 | 70 | 616 | 507 | 113 | 938 | 880 | 156 |  |  |
| 28 | 289 | 165 | 71 | 624 | 516 | 114 | 946 | 888 | 157 |  |  |
| 29 | 297 | 173 | 72 | 632 | 524 | 115 | 953 | 897 | 158 |  |  |
| 30 | 305 | 180 | 73 | 639 | 533 | 116 | 960 | 906 | 159 |  |  |
| 31 | 313 | 188 | 74 | 647 | 541 | 117 | 968 | 915 | 160 |  |  |
| 32 | 321 | 196 | 75 | 654 | 550 | 118 | 975 | 924 | 161 |  |  |
| 33 | 329 | 204 | 76 | 662 | 558 | 119 | 983 | 933 | 162 |  |  |
| 34 | 337 | 211 | 77 | 669 | 567 | 120 | 990 | 941 | 163 |  |  |
| 35 | 345 | 219 | 78 | 677 | 575 | 121 | 997 | 950 | 164 |  |  |
| 36 | 353 | 227 | 79 | 684 | 584 | 122 | 1005 | 959 | 165 |  |  |
| 37 | 361 | 235 | 80 | 692 | 592 | 123 | 1012 | 968 | 166 |  |  |
| 38 | 369 | 243 | 81 | 700 | 601 | 124 | 1019 | 977 | 167 |  |  |
| 39 | 377 | 251 | 82 | 707 | 610 | 125 | 1027 | 986 | 168 |  |  |
| 40 | 385 | 259 | 83 | 715 | 618 | 126 | 1034 | 994 | 169 |  |  |
| 41 | 393 | 267 | 84 | 722 | 627 | 127 | 1042 | 1003 |  |  |  |
| 42 | 400 | 275 | 85 | 730 | 635 | 128 | 1049 | 1012 |  |  |  |

The first column is the number of errors (ne = number of exceeded delays or number of wrong reports)

The second column is the number of samples for the pass limit (nsp , ns=Number of samples= number of successes + number of exceedings or number of reports)

The third column is the number of samples for the fail limit (nsf)

## G.2.4	Pass fail decision rules

The pass fail decision rules apply for a single test, comprising one component in the test vector. The over all Pass /Fail conditions are defined in clause G.2.6

Having observed 0 errors, pass the test at 33+ samples, otherwise continue

Having observed 1 error, pass the test at 46+ samples, otherwise continue

Having observed 2 errors, pass the test at 58+ samples, otherwise continue

Having observed 10 errors, pass the test at 136+ samples, fail the test at 39 samples, otherwise continue

Having observed 146 errors, pass the test at 1181+ samples, fail the test at 1173- samples, otherwise continue

Having observed 147 errors, fail the test at 1182- samples,

Where x+ means: x or more, x- means x or less

NOTE 1: an ideal DUT passes after 33 samples. The maximum test time is 1181 samples.

## G.2.5	Void

## G.2.6	Test conditions for delay tests and UE measurement performance

Table G.2.6-1: test conditions

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test | Over all Pass/Fail condition |
| --- | --- | --- | --- |
| All tests in clauses 4.4.3, 4.5, 4.6, 5.4.3, 5.5, 5.6, 6.1, 6.2, 6.3.1, 6.3.2.1, 6.3.2.3, 6.4.3, 6.5, 6.6, 7.1, 7.2, 7.3.1, 7.3.2.1, 7.3.2.3, 7.4.3, 7.5, 7.6, 10.1.2, 10.2.2, 10.3, 10.4, 11.1, 11.2.1, 11.2.2.1, 11.2.2.3, 11.3.2, 11.4, 11.5, 12.1, 12.2, 12.3, 12.4, 13.1.2, 13.2, 13.3, 16.1, 16.2, 16.3.1, 16.3.2.1, 16.3.2.3, 16.5, 16.6, 17.1, 17.2, 17.3.1, 17.3.2.1, 17.3.2.3, 17.4.3, 17.5, 17.6, 18.1, 18.2, 18.3 are delay tests of statistical nature while 4.3.2.2, 4.4.1, 5.3.2.2, 5.4.1, 6.3.2.2, 6.4.1, 7.3.2.2, 7.4.1, 10.1.1.1, 10.2.1.1, 11.2.2.2, 11.3.1, 13.1.1, 16.3.2.2, 16.4.1, 17.3.2.2, 17.4.1 are not applicable, since they are deterministic. | Test procedure in all statistical tests ensures independency | 1 per operating band (if tested, see 3A.3) | Full set of environmental conditions (5) per operating band |
| All tests in clauses 4.7, 5.7, 6.7 and 7.7, 10.5, 11.6, 12.5, 13.4, 16.7, 17.7 are UE level reports of statistical nature | Independency is assumed, although Layer 1 filtering is applied to the reported results | Full set of environmental conditions (5) per operating band | Full set of environmental conditions (5) per operating band |

## G.2.7	Additional considerations for tests in bands subject to CCA failure

The scope of shared spectrum RRM tests is similar to their corresponding licensed carrier tests. However, the unlicensed channel accessibility is subject to a probabilistic Clear Channel Assessment (CCA) model and therefore is not guaranteed. This model applies independently to DL (PCCA_DL_i) and UL (PCCA_UL) and is defined in section D.7.

The SS has knowledge on whether a given DL or UL slot is subject to CCA failure:

-	In DL slots subject to CCA failure the SS will not schedule the slot

-	In UL slots subject to CCA failure the SS will occupy the channel according to section D.7.2.2

Due to the statistical nature of the CCA model, iterations can fail. In the event the SS detects a failure caused by CCA failure, the SS shall declare that iteration as inconclusive (remove it from the count) and continue testing.

Table G.2.7-1 lists test cases from sections 10, 11, 12 and 13 that have been considered as untestable due to issues related with how the CCA model impacts the test outcome. Unless otherwise listed in Table G.2.7-1, all tests from sections 10, 11, 12 and 13 can be executed following the statistical methodology defined in clause G.2.3.

Table G.2.7-1: Test cases in bands subject to CCA failure with testability issues

| Test Case | Channel Access Mode | PCCA_DL_i | PCCA_UL | Testability Issue Reason |
| --- | --- | --- | --- | --- |
| 11.2.2.2.1 | Semi-static | N/A | 0.87 | SS may not be able to identify whether a missing PRACH was caused by CCA failure as UE does not have a fixed schedule |
|  | Dynamic | N/A | 0.75 |  |
| 11.2.2.2.2 | Semi-static | N/A | 0.87 | SS may not be able to identify whether a missing PRACH was caused by CCA failure as UE does not have a fixed schedule |
|  | Dynamic | N/A | 0.75 |  |
| 11.2.2.2.3 | Semi-static | N/A | 0.87 | SS may not be able to identify whether a missing PRACH was caused by CCA failure as UE does not have a fixed schedule |
|  | Dynamic | N/A | 0.75 |  |
| 11.2.2.2.4 | Semi-static | N/A | 0.87 | SS may not be able to identify whether a missing PRACH was caused by CCA failure as UE does not have a fixed schedule |
|  | Dynamic | N/A | 0.75 |  |

# G.3	Statistical testing of NR sidelink CBR measurement tests

## G.3.1	General

The CBR measurement tests are of statistical nature. The pass fail limits in tests of statistical nature are expressed as a limit (e.g. delay limit) and a success ratio applicable for the limit. The success ratio is 98% (the complement is the error ratio ER = 2%).

## G.3.2	Design of the test

The test is defined by the following design principles (see TS 36.521-1 clause G.7, Theory):

- The early decision concept is applied.

- A second limit is introduced: bad DUT factor M>1

To decide the test pass:

- Supplier risk is applied based on the bad DUT quality

To decide the test fails

- Customer risk is applied based on the specified DUT quality

The test is defined by the following parameters:

- Limit ER = 0.02 (success ratio = 98%)

- Bad DUT factor M=1.5 (selectivity)

- Confidence level CL = 95% (for specified DUT and bad DUT-quality)

## G.3.3	Numerical definition of the pass fail limits

Table G.3.3-1: pass fail limits for ER = 0.02

| ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 206 | N/A | 43 | 1970 | 1389 | 86 | 6370 | 3300 | 129 | 6413 | 5409 |
| 1 | 287 | N/A | 44 | 2005 | 1431 | 87 | 6371 | 3348 | 130 | 6414 | 5459 |
| 2 | 345 | N/A | 45 | 2041 | 1468 | 88 | 6372 | 3396 | 131 | 6415 | 5511 |
| 3 | 398 | 10 | 46 | 2078 | 1511 | 89 | 6373 | 3443 | 132 | 6416 | 5560 |
| 4 | 447 | 23 | 47 | 2113 | 1555 | 90 | 6374 | 3492 | 133 | 6417 | 5613 |
| 5 | 494 | 39 | 48 | 2149 | 1600 | 91 | 6375 | 3537 | 134 | 6418 | 5660 |
| 6 | 540 | 57 | 49 | 6333 | 1637 | 92 | 6376 | 3585 | 135 | 6419 | 5712 |
| 7 | 585 | 78 | 50 | 6334 | 1683 | 93 | 6377 | 3631 | 136 | 6420 | 5765 |
| 8 | 629 | 102 | 51 | 6335 | 1728 | 94 | 6378 | 3683 | 137 | 6421 | 5811 |
| 9 | 671 | 130 | 52 | 6336 | 1768 | 95 | 6379 | 3728 | 138 | 6422 | 5866 |
| 10 | 714 | 155 | 53 | 6337 | 1813 | 96 | 6380 | 3778 | 139 | 6423 | 5915 |
| 11 | 756 | 182 | 54 | 6338 | 1857 | 97 | 6381 | 3828 | 140 | 6424 | 5970 |
| 12 | 797 | 212 | 55 | 6339 | 1900 | 98 | 6382 | 3874 | 141 | 6425 | 6020 |
| 13 | 838 | 243 | 56 | 6340 | 1938 | 99 | 6383 | 3922 | 142 | 6426 | 6069 |
| 14 | 879 | 276 | 57 | 6341 | 1982 | 100 | 6384 | 3971 | 143 | 6427 | 6124 |
| 15 | 918 | 306 | 58 | 6342 | 2028 | 101 | 6385 | 4018 | 144 | N/A | 6427 |
| 16 | 959 | 342 | 59 | 6343 | 2072 | 102 | 6386 | 4070 | 145 |  |  |
| 17 | 998 | 375 | 60 | 6344 | 2116 | 103 | 6387 | 4118 | 146 |  |  |
| 18 | 1037 | 408 | 61 | 6345 | 2161 | 104 | 6388 | 4169 | 147 |  |  |
| 19 | 1077 | 442 | 62 | 6346 | 2203 | 105 | 6389 | 4216 | 148 |  |  |
| 20 | 1115 | 480 | 63 | 6347 | 2246 | 106 | 6390 | 4264 | 149 |  |  |
| 21 | 1154 | 517 | 64 | 6348 | 2292 | 107 | 6391 | 4314 | 150 |  |  |
| 22 | 1192 | 553 | 65 | 6349 | 2338 | 108 | 6392 | 4358 | 151 |  |  |
| 23 | 1230 | 591 | 66 | 6350 | 2382 | 109 | 6393 | 4413 | 152 |  |  |
| 24 | 1269 | 629 | 67 | 6351 | 2427 | 110 | 6394 | 4462 | 153 |  |  |
| 25 | 1306 | 666 | 68 | 6352 | 2473 | 111 | 6395 | 4506 | 154 |  |  |
| 26 | 1344 | 706 | 69 | 6353 | 2516 | 112 | 6396 | 4561 | 155 |  |  |
| 27 | 1382 | 741 | 70 | 6354 | 2559 | 113 | 6397 | 4611 | 156 |  |  |
| 28 | 1420 | 777 | 71 | 6355 | 2608 | 114 | 6398 | 4661 | 157 |  |  |
| 29 | 1457 | 815 | 72 | 6356 | 2652 | 115 | 6399 | 4712 | 158 |  |  |
| 30 | 1494 | 855 | 73 | 6357 | 2694 | 116 | 6400 | 4760 | 159 |  |  |
| 31 | 1531 | 898 | 74 | 6358 | 2741 | 117 | 6401 | 4807 | 160 |  |  |
| 32 | 1568 | 936 | 75 | 6359 | 2789 | 118 | 6402 | 4854 | 161 |  |  |
| 33 | 1605 | 976 | 76 | 6360 | 2836 | 119 | 6403 | 4909 | 162 |  |  |
| 34 | 1642 | 1015 | 77 | 6361 | 2883 | 120 | 6404 | 4960 | 163 |  |  |
| 35 | 1679 | 1059 | 78 | 6362 | 2928 | 121 | 6405 | 5010 | 164 |  |  |
| 36 | 1715 | 1098 | 79 | 6363 | 2971 | 122 | 6406 | 5062 | 165 |  |  |
| 37 | 1752 | 1138 | 80 | 6364 | 3018 | 123 | 6407 | 5110 | 166 |  |  |
| 38 | 1788 | 1180 | 81 | 6365 | 3068 | 124 | 6408 | 5158 | 167 |  |  |
| 39 | 1825 | 1220 | 82 | 6366 | 3115 | 125 | 6409 | 5204 | 168 |  |  |
| 40 | 1861 | 1263 | 83 | 6367 | 3162 | 126 | 6410 | 5256 | 169 |  |  |
| 41 | 1897 | 1302 | 84 | 6368 | 3203 | 127 | 6411 | 5305 |  |  |  |
| 42 | 1933 | 1344 | 85 | 6369 | 3251 | 128 | 6412 | 5358 |  |  |  |

The first column is the number of errors (ne = number of exceeded delays or number of wrong reports)

The second column is the number of samples for the pass limit (nsp , ns=Number of samples= number of successes + number of wrong reports)

The third column is the number of samples for the fail limit (nsf)

## G.3.4	Pass fail decision rules

The pass/fail decision rules apply for a single test, comprising one component in the test vector. The over all pass/fail conditions are defined in clause G.2.6:

Having observed 0 error, pass the test at 206+ samples, otherwise continue,

Having observed 1 error, pass the test at 287+ samples, otherwise continue,

Having observed 2 errors, pass the test at 345+ samples, otherwise continue,

Having observed 3 errors, pass the test at 398+ samples, fail the test at 10- samples, otherwise continue,

…

Having observed 143 errors, pass the test at 6427+ samples, fail the test at 6124- samples, otherwise continue,

Having observed 144 errors, fail the test at 6427- samples,

Where x+ means: x or more, x- means x or less.

NOTE 1: an ideal DUT passes after 206 samples. The maximum test time is 6427 samples.

# G.4	Theory to derive the numbers in Table G.2.3-1 (informative)

TS 36.521-1 Anne4 G.X applies.

# G.5	Theory to derive the numbers in Table G.3.3-1 (informative)

TS 36.521-1 Annex G.7 applies except that:

-	36.521-1 Annex G.7.9 is replaced by G.5.1 and G.5.2.

## G.5.1	Algorithm to calculate pass and fail probabilities

Following basic assumptions for statistical testing are introduced in this clause:

-	Result of single iteration of test procedure loop obeys Bernoulli distribution with error ratio p, i.e. the probability that the TE observes expected UE behaviour in single iteration (counted as a success) is 1-p and the probability that the TE doesn't observe expected UE behaviour in single iteration (counted as a fail) is p.

-	Results of different iterations are independent and identically distributed (i.i.d).

-	Early decision concept is applied, i.e., the TE repeats multiple iterations of the test procedure loop. For each iteration the TE records its result (count as a success if the TE observes the expected behavior, count as a fail otherwise) and its index in all iterations. When the i-th fail is observed in ei-th iteration, the TE compares ei with corresponding pass limit Np,i and fail limit Nf,i. The TE shall:

-	terminate the test and decide the DUT passes the test if ei>Np,i

-	terminate the test and decide the DUT fails the test if ei≤Nf,i,

-	continue the test and make no decision if Nf,i<ei≤Np,i.

For a more intuitive understanding, we use the state transition diagram shown in Figure G.5.1-1 to describe the entire process of statistical testing. In Figure 1, X-axis and Y-axis denote the success count and fail count in iterations already executed respectively. And the point with coordinate i,j (referred as state i,j thereafter) is used to indicates that the DUT has executed i+j iterations which includes i successes and j fails, where i,j≥0.

Figure G.5.1-1: Visualization of early decision statistical testing process. In this example, the TE verdicts that the DUT passes the test after executing 15 iterations of test procedure loop. The results of these 15 iterations are “s”, “f”, “f”, “s”, “s”, “s”, “f”, “s”, “s”, “s”, “f”, “s”, “s” and “s” in order (“s” = “success” and “f” = “fail”)

From Figure G.5.1-1 it’s straightforward to get following observations:

-	The DUT is always in state 0,0 at the beginning of statistical test since no iteration is executed at that moment.

-	Assume the DUT is in state i,j after executing i+j iterations. Then,

-	if the i+j+1-th iteration counts a success (with probability 1-p), the success count shall increase 1 and fail count shall remain unchanged after the i+j+1-th iteration. As a result, the DUT will be in state i+1,j after the i+j+1-th iteration. We call that the DUT moves from state i,j to state i+1,j, depicted as moving right by one in the diagram.

-	if the i+j+1-th iteration counts a fail (with probability p), the success count remains unchanged and the fail count increases 1 after the i+j+1-th iteration. Then the DUT will be in state i,j+1 after the i+j+1-th iteration. We call that the DUT moves from state i,j to state i,j+1, depicted as moving up by one in the diagram.

-	The TE shall evaluate whether pass criteria or fail criteria is satisfied every time the DUT has executed a new iteration, i.e. the DUT moves to a new state. Specifically speaking,

-	If pass criterion is satisfied when DUT is in state i,j, the TE makes the decision that the DUT passes the test. Correspondingly, state i,j is referred as a passing state (depicted as green points in Figure G.5.1-1);

-	If fail criterion is satisfied when DUT is in state i,j, the TE makes the decision that the DUT fails the test. Correspondingly, state i,j is referred as a failure state (depicted as red points in Figure G.5.1-1).

-	If neither pass criterion nor fail criterion is satisfied when DUT is in state i,j, the TE makes no decision and continues the test. Correspondingly, state i,j is referred as a non-decidable state (depicted as hollow points in Figure G.5.1-1).

In this way, the statistical testing is represented as a process in which the DUT starts from state 0,0, moves along certain path in the diagram, and finally stops in certain passing state or failure state.

Obviously, there must be a close relationship between the pass/fail states and the pass/fail limits used for the test. The following two conclusions precisely describes the mapping between pass/fail limits and pass/fail states.

Conclusion 1: The DUT is decided to pass the test if and only if the DUT moves from state 0,0 to a state belongs to passing state set P=Np,i+1-i,i: i=0,1,2,⋯.

Proof: We prove sufficiency first, and then necessity.

-	Sufficiency: If the DUT is in state Np,i+1-i,i, it means that the DUT has executed Np,i+1 iterations which includes i success and Np,i+1-i fails. Clearly, the TE can only get the i+1-th fail as early as the Np,i+1+1-th iteration, then we have ei+1>Np,i+1. This is just the pass criterion when the TE collecting the i+1-th fail.

-	Necessity: If the TE verdicts the DUT to pass when it collects the i+1-th fail, following inequations must have been satisfied according to the pass criterion:

e1≤Np,1, e2≤Np,2,⋯, ei≤Np,i,ei+1>Np,i+1.

The DUT is in state ei-i, i when the i-th fail happens in the ei-th iteration. Then the next Np,i+1-ei iterations after the ei-th iteration must be counted as success since ei+1>Np,i+1. Then the DUT will be in state Np,i+1-i,i after executing these Np,i+1-ei iterations.

Conclusion 2: The DUT is decided to fail the test if and only if the DUT moves from state 0,0 to a state belongs to failure set F=j,i+1: Nf,i-i<j≤Nf,i+1-i+1, i=0,1,2,⋯, where Nf,0≜-1 for sake of simplify.

Proof of conclusion 2 is quite similar as the proof of conclusion 1 and is omitted here. Conclusion 1 and 2 have shown the equivalence between pass/fail limits and pass/fail state sets. As a result, the probability that the DUT pass the test for given error ratio p, passing state set P and failure state set F, denoted as PpP,F, is given by

PpP,F=ProbDUT moves from state0,0 to state i,j which belongs to P.

Furthermore, it’s easy to prove that any path from state0,0 to passing state i,j won’t pass through any passing state or failure state except the ending state. This is because if the path contains a passing state or failure state i',j' which is not the ending state, the test will be terminated immediately when the DUT is in state i',j'. Then i',j' shall be the ending state, i.e. i',j'=i,j. This contradicts the assumption that state i',j' is not the ending state. This also means that two events “DUT moves from state (0,0) to passing state (i,j)” and “DUT moves from state (0,0) to passing state (i',j')” are disjoint if i',j'≠i,j. Then we have,

PpP,F=i,j∈PProbDUT moves from state0,0 to passing state i,j=i,j∈PTi,jpP,F,

where Ti,jpP,F is defined as the transition probability from state 0,0 to state i,j, i.e.

Ti,jpP,F≜ProbDUT moves from state0,0 to state i,j.

Similarly, the probability that the DUT fail the test for given error ratio p, passing state set P and failure state set F, denoted as FpP,F, is derived as

FpP,F=i,j∈FTi,jpP,F

From the discussion above we can observe that the calculation of the PpP,F and FpP,F is finally transformed into the calculation of the transition probability Ti,jpP,F. Assuming DUT is in state i,j after the i+j-th iteration, i,j≥0. Considering that the i+j-th iteration can only have two possible results: success or fail, we can derive that the DUT can only be in state i-1,j or state i,j-1 after executing i+j-1 iterations. Then we have:

Ti,jpP,F=ProbDUT moves from state0,0 to state i,j, passing i-1,j+ProbDUT moves from state0,0 to state i,j, passing state i,j-1=ProbDUT moves from state0,0 to i-1,jProbDUT moves from state i-1,j to i,j+ProbDUT moves from state0,0 to i,j-1ProbDUT moves from state i,j-1 to i,j

The first equation above holds because the event “DUT moves from state (0,0) to state (i,j), passing state (i,j-1)” and event “DUT moves from state (0,0) to state (i,j), passing state (i-1,j)” are disjoint. And the second equation above holds because results of different iterations are independent.

Furthermore, we have following obvious observations:

If i=0, obviously the event “DUT moves from state 0,0 to state i-1,j” won’t happen since state i-1,j doesn’t exist in this case. Otherwise, the probability that this event occurs equals to  Ti-1,jpP,F according to the definition of transition probability. Then we have

ProbDUT moves from state0,0 to i-1,j=Ti-1,jpP,FIi>0,

Similarly, we can also derive that

ProbDUT moves from state0,0 to i,j-1=Ti,j-1pP,FIj>0,

Where, Ii>0 and Ij>0 are indicator functions,

Ii>0=1,if i>00,otherwise,  Ij>0=1,if j>00,otherwise

If i-1,j∈P∪F, the event “DUT moves from state i-1,j to state i,j” won’t happen since the test will be terminated when DUT reaches state i-1,j. Otherwise, this event is equivalent to the event “result of the i+j-th iteration is a success”. Then we have

ProbDUT moves from state i-1,j to i,j=1-pIi-1,j∉P∪F

Similarly, we can also derive that

ProbDUT moves from state i,j-1 to i,j=pIi,j-1∉P∪F.

where,

Ii-1,j∉P∪F=1,i-1,j∉P∪F0,otherwise,  Ii,j-1∉P∪F=1,if i,j-1∉P∪F0,otherwise

Finally, we have shown that following conclusion holds.

Conclusion 3: Transition probability Ti,jpP,F can be calculated by

Ti,jpP,F=Ti-1,jpP,F1-pIi>0Ii-1,j∉P∪F+Ti,j-1pP,FpIj>0Ii,j-1∉P∪F.

The conclusion above shows that Ti,jpP,F can be calculated in a dynamic programming manner: Clearly, we can calculate Ti,jpP,F using the above equation if we know the value of Ti-1,jpP,F and Ti,j-1pP,F; For the same reason, we can calculate Ti-1,jpP,F and Ti,j-1pP,F using the equation above if we know the value of  Ti-2,jpP,F, Ti-1,j-1pP,F and Ti,j-2pP,F; Repeating the recursion above, at last all we need to know is only T0,0pP,F. And we know T0,0pP,F=1 since UE always is in state 0,0 when test starts.

Using the recursion above, we can finally obtain Ti,jpP,F and further PpP,F and FpP,F for given p, P and F.

Assume the limit-risk mapping given in 36.521-1 is still used. Then the customer risk Rc and supplier risk Rs for given passing state set P and failure state set F is the probability that baseline bad DUT (error ratio = Mp, where M is the bad DUT factor) passes the test and the probability that baseline good DUT (error ratio = mp, where m is the bad DUT factor) fails the test respectively. Then we have

Rc=i,j∈PTi,jMpP,F,Rs=i,j∈FTi,jmpP,F.

## G.5.2	Method of designing pass/fail limit to approach given risk probability

For given error ratio p, confidence level PCL, good DUT factor m and bad DUT factor M, pass limit Np,i,i=1,2,⋯ and fail limit Nf,i:i=1,2,⋯ can be obtained by using the method listed below. Note that the limit-risk mapping in 36.521-1 is assumed.

Step 1: Initialize as follows:

- Set P and F, passing state set and failure state set respectively, as P, F=∅.

- Set Rc and Rs, customer risk and supplier risk respectively, as Rc,Rs=0;

- Set n, number of executed iterations, as n=0;

- Set S, non-decidable state set after n-th iteration, as S=0,0;

- Set Ti,jmpP,F and T0,0MpP,F, transition probability to state 0,0 for baseline good DUT and for baseline bad DUT respectively, as Ti,jmpP,F=1, T0,0MpP,F=1.

- Select Δc and Δs, step size for customer risk and supplier risk respectively, as Δc≤1-PCL, Δs≤1-PCL.

Figure G.5.2-1: Demonstration of step 1

Figure G.5.2-1 is the demonstration of step 1. The reason to initialize =0,0, Ti,jmpP,F=1, T0,0MpP,F=1 is because the DUT is always in state 0,0 before executing any iteration. And we have P, F=∅ since no passing state or failure state is selected yet.

Step 2: Calculate set S', the set of the state can be reached after the n+1-th iteration, as S'=i,j:i-1,j∈ S or i,j-1∈S. Then for all state i,j∈S', calculate Ti,jMpP,F and Ti,jmpP,F as follows:

Ti,jMpP,F=Ti-1,jMpP,F1-MpIi>0Ii-1,j∉P∪F+Ti,j-1MpP,FMpIj>0Ii,j-1∉P∪F,

Ti,jmpP,F=Ti-1,jmpP,F1-mpIi>0Ii-1,j∉P∪F+Ti,j-1mpP,FmpIj>0Ii,j-1∉P∪F.

Figure G.5.2-2: Demonstration of step 2

See Figure G.5.2-2 for better understanding of step 2. If DUT doesn’t pass or fail the test after the n-th iteration then DUT must be in one state in S. It’s easy to observe that the DUT must be in one state in S'. Then the transition probability to state in S' can be calculated by using Conclusion 3. For example, the only way to move to state D in Figure 2 is to first move to state A then count a fail in the n+1-th iteration, then we have TDp=TApp. On the other hand, to move to state E, the DUT can first move to state B then count a success in the n+1-th iteration or first move to state C then count a fail in the n+1-the iteration, i.e. TEp=TBp1-p+TCpp.

Step 3: First, try finding an integer ne which satisfies n+1-ne,ne∈S', Rc+i,j∈S'j≤neTi,jMpP,F≤1-PCL and Rs+i,j∈S'j>neTi,jmpP,F≤1-PCL. If such a ne exists, let

Rc=Rc+i,j∈S',j≤neTi,jMpP,F,Rs=Rs+i,j∈S',j>neTi,jmpP,F,

P=P∪i,j:i,j∈S',j≤ne, F=F∪i,j:i,j∈S',j>ne,

If such a ne doesn’t exist, instead try finding integers np and nf which satisfy

n+1-np,np∈S',n+1-nf,nf∈S',

minicΔc,1-PCL<Rc+i,j∈S',j≤npTi,jMpP,F≤minic+1Δc,1-PCL,

minisΔs,1-PCL<Rs+i,j∈S',j≥nfTi,jmpP,F≤minis+1Δs,1-PCL.

Then,

-	if such a np exists, set

Rc=Rc+i,j∈S',j≤npTi,jMpP,F,  P=P∪i,j:i,j∈S',j≤np,  ic=ic+1.

-	if such a nf exists, set

Rs=Rs+i,j∈S',j≥nfTi,jmpP,F,  F=F∪i,j:i,j∈S',j≥nf,  is=is+1.

Two possible cases in step 3 are depicted in figure G.5.2-3 and figure G.5.2-4 respectively. If ne exists, even we add all states i,j∈S' with j≤ne to P and all states i,j∈S' with j>ne to F, the customer risk and supplier risk are still no larger than 1-PCL (the increment of the customer risk and supplier risk, are denoted as blue and green triangle in figure 4 respectively).

Figure G.5.2-3: Demonstration of step 3 (ne exists)

If ne doesn’t exist, instead we try to find state set i,j:i,j∈S',j≤np which satisfies that the total transition probability to any state in the set is approximately Δc. If such a set exists we can add states in this set to P without breaking customer risk threshold. Similarly, we can also try to a state seti,j:i,j∈S',j≥nf which satisfies that the total transition probability to any state in the set is approximately Δs and add states in this set to F.

Figure G.5.2-4: Demonstration of step 3 (ne doesn’t exist)

Step 4: Let S=S'\P∪F, n=n+1. Then go back to Step 2 if S≠∅, otherwise go to Step 5.

If a state i,j∈S' is selected as a passing state or failing state then it’s no longer a non-decidable state. Then we need to exclude it from non-decidable state set after n+1-the iteration. If all states in S' are excluded, clearly the test surely will terminate after the n+1-the iteration and the designing is finished. Otherwise, we need to go back to step 2 and repeat procedures above.

Step 5: Sort all states i,j in P in the ascending order of j and all states i,j in F in the ascending order of i, i.e. we denote

P=A0,0, A1,1, A2,2, A3,3⋯,  F=0,B0, 1,B1,2,B2,3,B3,⋯

Then construct passing state set P' and failure state set F' as

P'=Aj',j:j=0,1,2,⋯,  F'=i,Bi':i=0,1,2⋯

Where Aj'=mink≥jAk,  Bi'=mink≥iBi

Purpose of step 5 is to further optimize the passing state and failure state obtained in step 2 to 4. To understand step 5, we only need to note that all path to passing state A,B must also pass state C,D. On the other hand, if a path passes state D or E, it must eventually reach one passing state in A, B and C. Then one can derive that

i,j∈A,B,CTi,jpP,F=i,j∈A,D,ETi,jpP,F

In other words, if P=A,B,C is a passing set which satisfies customer risk requirements, then P'=A,D,E is also a passing set satisfying customer risk requirements since it has exactly the same customer risk as P. But P' is a better choice than P since it requires fewer iterations to make decision.

For the same reason, we can also derive that failure set F'=F,G,L,M,N,K has same supplier risk with F=F,G,H,I,J,K but requires fewer iterations to fail the test.

Figure G.5.2-5: Demonstration of step 3 (ne doesn’t exist).

Step 6: Construct pass limit Np,i and fail limit Nf,i as follows:

Nf,1=maxBi=1i+Bi,Nf,2=maxBi=2i+Bi,Nf,3=maxBi=3i+Bi,⋯

At last, we can convert pass/fail state sets to pass/fail limits by using conclusion 3 in Annex G.5.1.

Annex H (normative):
Default message contents for RRM

# H.1	Void

# H.2	System information blocks message content exceptions

## H.2.1	System information blocks message contents exceptions for NR intra-frequency cell re-selection

SIB2: for NR intra-frequency cell re-selection

Table H.2.1-1: SIB2: NR intra-frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| rangeToBestCell | Not present |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 | -140 is actual value in dBm (-70 * 2 dBm) | SCS15 |
|  | -69 | -138 is actual value in dBm (-69 * 2 dBm) | SCS30 |
|  | -69 | -138 is actual value in dBm (-69 * 2 dBm) | SCS120 |
|  | -68 | -136 is actual value in dBm (-68 * 2 dBm) | SCS240 |
| s-IntraSearchP | 30 | 60 is actual value in dB (30 * 2 dB) |  |
| smtc | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.n |  | SMTC.n |
| deriveSSB-IndexFromCell | false |  | Asynchronous cells |
|  | true |  | Synchronous cells |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SMTC.n | SMTC pattern n according to TS 38.133 [6] A.3.11 |
| Synchronous cells | SSB indices of neigibour cells can be derived from timing of serving cell |
| Asynchronous cells | SSB indices of neigibour cells cannot be derived from timing of serving cell |

SIB3: for NR intra-frequency cell re-selection

Table H.2.1-2: SIB3: NR intra-frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB3 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList SEQUENCE (SIZE (1..maxCellIntra)) OF SEQUENCE { |  |  |  |
| IntraFreqNeighCellInfo ::= SEQUENCE{ |  |  |  |
| physCellId | Set according to the neighbour cell PCI |  |  |
| q-OffsetCell | dB0 | 0 is actual value in dB (0 * 2 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

SIB1: for NR intra-frequency cell re-selection

Table H.2.1-3: SIB1: NR intra-frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 | -140 is actual value in dBm (-70 * 2 dBm) | SCS15 |
|  | -69 | -138 is actual value in dBm (-69 * 2 +1 dBm) | SCS30 |
|  | -69 | -138 is actual value in dBm (-69 * 2 dBm) | SCS120 |
|  | -68 | -136 is actual value in dBm (-68 * 2 dBm) | SCS240 |
| } |  |  |  |
| servingCellConfigCommon SEQUENCE { |  |  |  |
| highSpeedConfig-r16 SEQUENCE { |  |  |  |
| highSpeedMeasFlag-r16 | true |  | HighSpeedMeas |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| HighSpeedMeas | highSpeedMeasFlag-r16 is configured. UE shall apply the enhanced measurement requirements to support high speed up to 500 km/h as specified in TS 38.133 [14] |

## H.2.2	System information blocks message contents exceptions for NR inter-frequency cell re-selection

SIB2: for NR inter-frequency cell re-selection

Table H.2.2-1: SIB2: NR inter-frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| rangeToBestCell | Not present |  |  |
| } |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | 25 | 50 is actual value in dB (25 * 2 dB) |  |
| threshServingLowP | 22 | 44 is actual value in dB (22 * 2 dB) |  |
| cellReselectionPriority | 4 |  | NR Cell 1 |
|  | 5 |  | NR Cell 2 |
| } |  |  |  |
| } |  |  |  |

SIB4: for NR inter-frequency cell re-selection

Table H.2.2-2: SIB4: NR inter-frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 Entry |  |  |
| dl-CarrierFreq[1] | Downlink NR SSB ARFCN of NR Cell 2 |  | NR Cell 1 |
|  | Downlink NR SSB ARFCN of NR Cell 1 |  | NR Cell 2 |
| smtc | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.n |  | SMTC.n |
| deriveSSB-IndexFromCell[1] | false |  | Asynchronous cells |
|  | true |  | Synchronous cells |
| q-RxLevMin[1] | -70 | -140 is actual value in dBm (-70 * 2 dBm) | dBm/15kHz |
|  | -69 | -137 is actual value in dBm (-69 * 2 +1 dBm) | dBm/30kHz |
| threshX-HighP[1] | 24 | 48 is actual value in dB (24 * 2 dB) |  |
| threshX-LowP[1] | 25 | 50 is actual value in dB (25 * 2 dB) |  |
| cellReselectionPriority[1] | 5 |  | NR Cell 1 |
|  | 4 |  | NR Cell 2 |
| q-OffsetFreq[1] | dB0 | 0 is actual value in dB (0 * 2 dB). |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB.n FRm | SSB pattern n in FRm according to TS 38.133 [6] A.3.10 |
| Synchronous cells | SSB indices of neighbour cells can be derived from timing of serving cell |
| Asynchronous cells | SSB indices of neighbour cells cannot be derived from timing of serving cell |

SIB1: for NR inter-frequency cell re-selection

Table H.2.2-3: SIB1: NR inter-frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 | -140 is actual value in dBm (-70 * 2 dBm) | dBm/15kHz |
|  | -69 | -137 is actual value in dBm (-69 * 2 +1 dBm) | dBm/30kHz |
| } |  |  |  |
| } |  |  |  |

## H.2.3	System information blocks message contents exceptions for NR inter-RAT cell re-selection

SIB1: for inter-RAT NR – E-UTRA cell re-selection

Table H.2.3-1: SIB1: Inter-RAT NR – E-UTRA cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 | -140 is actual value in dBm (-70 * 2 dBm) | SCS15 |
|  | -69 | -137 is actual value in dBm (-69 * 2 +1 dBm) | SCS30 |
| } |  |  |  |
| } |  |  |  |

SIB2: for inter-RAT NR – E-UTRA cell re-selection

Table H.2.3-2: SIB2: Inter-RAT NR – E-UTRA cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | 25 | 50 is actual value in dB (25 * 2 dB) |  |
| threshServingLowP | 22 | 44 is actual value in dB (22 * 2 dB) |  |
| cellReselectionPriority | 4 |  | higher priority |
|  | 5 |  | lower priority |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| smtc | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.n |  | SMTC.n |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SMTC.n | SMTC pattern n according to TS 38.133 [6] A.3.11 |
| higher priority | NR cell re-selection to higher priority E-UTRA |
| lower priority | NR cell re-selection to lower priority E-UTRA |

SIB5: for inter-RAT NR – E-UTRA cell re-selection

Table H.2.3-3: SIB5: Inter-RAT NR – E-UTRA cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF SEQUENCE { | 1 Entry |  |  |
| carrierFreq[1] | Downlink EUTRA ARFCN of E-UTRA Cell 1 |  |  |
| eutra-FreqNeighCellList[1] SEQUENCE (SIZE (1..maxCellEUTRA)) OF SEQUENCE { |  |  |  |
| physCellId | E-UTRA physical cell identity |  |  |
| dummy | dB0 |  |  |
| } |  |  |  |
| presenceAntennaPort1[1] | FALSE |  |  |
| cellReselectionPriority[1] | 5 |  | higher priority |
|  | 4 |  | lower priority |
| threshX-High | 24 | 48 is actual value in dB (24 * 2 dB) |  |
| threshX-Low | 25 | 50 is actual value in dB (25 * 2 dB) |  |
| q-RxLevMin | -70 | -140 is actual value in dBm (-70 * 2 dBm) |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| higher priority | NR cell re-selection to higher priority E-UTRA |
| lower priority | NR cell re-selection to lower priority E-UTRA |

# H.3	RRC message content exceptions

## H.3.1	RRC messages and information elements contents exceptions for NR measurement configuration

RRCReconfiguration

To setup NR Measurement Configuration.

Table H.3.1-1: RRCReconfiguration: NR measurement Configuration

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition NR_MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| measConfig | MeasConfig-DEFAULT | Measurement configuration |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| needForInterruptionConfigNR-r18 | enabled |  | NFICN |
|  | disabled |  | NNFICN |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NFICN | UE shall report interruption requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message |
| NNFICN | UE shall not report interruption requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message |

MeasConfig-DEFAULT

Configuration for NR measurement.

Table H.3.1-2: MeasConfig-DEFAULT: Configuration of NR measurement

| Derivation path: TS 38.508-1 [14], Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF SEQUENCE { | n entries | n=1 without condition n=2 for INTER-FREQ OR INTER-RAT OR Deactivated SCell |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with Condition INTRA-FREQ MO |  |  |
| } |  |  |  |
| measObjectId[2] | 2 |  | INTER-FREQ OR INTER-RAT OR Deactivated SCell |
| measObject[2] CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with Condition INTER-FREQ MO |  | INTER-FREQ |
| measObjectNR | MeasObjectNR-DEFAULT with Condition Deactivated SCell |  | Deactivated SCell |
| measObjectEUTRA | MeasObjectEUTRA-DEFAULT |  | INTER-RAT |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | ReportConfigId |  |  |
| reportConfig[1] CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-DEFAULT |  |  |
|  | ReportConfigNR-DEFAULT |  | INTER-FREQ OR Deactivated SCell |
| reportConfigInterRAT | ReportConfigInterRAT-DEFAULT |  | INTER-RAT |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | MeasId |  |  |
| measObjectId[1] | 1 |  |  |
|  | 2 |  | INTER-FREQ OR INTER-RAT OR Deactivated SCell |
| reportConfigId[1] | ReportConfigId |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
|  | QuantityConfig | TS 38.508-1 Table 4.6.3-127 | L3 FILTERING NEEDED |
| measGapConfig | MeasGapConfig-DEFAULT |  | GAP NEEDED |
| measGapSharingConfig | Not present |  |  |
| interFrequencyConfig-NoGap-r16 | Not present |  |  |
| effectiveMeasWindowConfig-r18 CHOICE { |  |  |  |
| setup | MeasWindowConfig-r18-DEFAULT |  | EMW_Setup |
| release | null |  | EMW_Release |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |  |
| --- | --- | --- |
| GAP NEEDED | Measurement gap on the NR Cell is needed for measurement |  |
| INTER-FREQ | Configuration for inter-frequency NR measurement tests |  |
| INTER-RAT | Configuration for inter-RAT EUTRA measurement tests |  |
| Deactivated SCell |  | Configuration for measurement on deactivated SCell tests |
| L3 FILTERING NEEDED |  | L3 filtering is needed for measurement |
| EMW_Setup |  | Setup effective measurement window in NR for E-UTRA measurements |
| EMW_Release |  | Release effective measurement window in NR for E-UTRA measurements |

MeasObjectNR-DEFAULT

NR measurement object configuration.

Table H.3.1-3: MeasObjectNR-DEFAULT: NR intra-frequency measurement object configuration

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR of the SSB associated to serving cell | frequency of the serving cell | INTRA-FREQ MO |
|  | ARFCN-ValueNR of the SSB associated to SCell |  | Deactivated SCell |
|  | ARFCN-ValueNR of the SSB associated to inter-frequency neighbour cell |  | INTER-FREQ MO |
| refFreqCSI-RS | ARFCN-ValueNR with condition DL_PointA |  | CSI-RS MOBILITY |
| referenceSignalConfig SEQUENCE { |  |  |  |
| ssb-ConfigMobility SEQUENCE { |  |  |  |
| ssb-ToMeasure | Not present |  |  |
|  | SSB-ToMeasure specified in TS.38.508-1 [14] Table 7.3.1-31 with condition SSB.7 FR2 OR SSB.8 FR2 |  | SSB.7 FR2 OR SSB.8 FR2 |
| deriveSSB-IndexFromCell | false |  |  |
|  | true |  | Synchronous cells OR RLM |
| ssb-PositionQCL-Common-r16 | n1 |  | SSB.1 CCA SSB.2 CCA |
|  | n2 |  | SSB.3 CCA SSB.4 CCA |
| } |  |  |  |
| csi-rs-ResourceConfigMobility CHOICE { |  |  | CSI-RS MOBILITY |
| setup | CSI-RS-ResourceConfigMobility-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |
| absThreshSS-BlocksConsolidation SEQUENCE { |  |  |  |
| thresholdRSRP | 0 | SS-RSRP < -156dB | NOT SS-SINR |
| thresholdSINR | 0 | SS-SINR<-23 | SS-SINR |
| } |  |  |  |
| measCycleSCell-v1530 | sf640 |  | Deactivated SCell |
| rmtc-Config-r16 SEQUENCE { |  |  | RSSI |
| rmtc-Periodicity-r16 | ms40 |  |  |
| measDurationSymbols-r16 | sym14or12 |  |  |
| rmtc-SubframeOffset-r16 | 20 |  |  |
| ref-SCS-CP-r16 | kHz15 |  |  |
| } |  |  |  |
| associatedMeasGapSSB-r17 | Not present |  |  |
|  | MeasGapId-r17 |  | MGe |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| INTRA-FREQ MO | Configuration for NR MO associated to intra-frequency carrier |
| INTER-FREQ MO | Configuration for NR MO associated to inter-frequency carrier |
| Synchronous cells | SSB indices of neighbour cells can be derived from timing of serving cell |
| RLM | Configuration for RLM tests |
| Deactivated SCell | Configuration for SCell activation and deactivation tests |
| SS-SINR | Configuration for SS-SINR tests |
| MGe | Indicates measurement gap enhancements configuration |
| SSB.X CCA | SSB pattern SSB.X CCA configured as per section A.3A (X = {1,2,3,4}) |
| RSSI | Configuration associated to RSSI or Channel Occupancy measurements |
| CSI-RS MOBILITY | Configuration for CSI-RS based measurement tests |

MeasObjectEUTRA-DEFAULT

EUTRA measurement object configuration

Table H.3.1-3A: MeasObjectEUTRA-DEFAULT: InterRAT EUTRA measurement object configuration

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-74 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA ::= SEQUENCE { |  |  |  |
| cellsToAddModListEUTRAN SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF SEQUENCE{ |  |  |  |
| cellIndexEUTRA | 1 |  |  |
| physCellId | Physical Cell ID of the E-UTRA cell | Annex E |  |
| cellIndividualOffset | dB0 |  |  |
| } |  |  |  |
| eutra-PresenceAntennaPort1 | false |  |  |
| associatedMeasGap-r17 | Not present |  |  |
|  | MeasGapId-r17 |  | MGe |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MGe | Indicates measurement gap enhancements configuration |

CSI-RS-ResourceConfigMobility-DEFAULT

CSI-RS based measurement resource configuration

Table H.3.1-3B: CSI-RS-ResourceConfigMobility-DEFAULT

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-44 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceConfigMobility ::= SEQUENCE { |  |  |  |
| csi-RS-CellList-Mobility SEQUENCE (SIZE (1..maxNrofCSI-RS-CellsRRM)) OF CSI-RS-CellMobility { | 2 entries |  |  |
| CSI-RS-CellMobility[1] SEQUENCE { |  | Entry 1 |  |
| cellId | PhysCellId of NR SpCell |  |  |
| csi-rs-MeasurementBW SEQUENCE { |  |  |  |
| nrofPRBs | 48 |  |  |
| startPRB | 3 |  |  |
| } |  |  |  |
| density | d3 |  |  |
| csi-rs-ResourceList-Mobility SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility { | 1 entry |  |  |
| CSI-RS-Resource-Mobility[1] SEQUENCE { |  | Entry 1 |  |
| csi-RS-Index | 0 |  |  |
| slotConfig CHOICE { |  |  |  |
| ms20 | 1 |  |  |
| } |  |  |  |
| associatedSSB SEQUENCE { |  |  |  |
| ssb-Index | 0 |  |  |
| isQuasiColocated | true |  |  |
| } |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 1111 |  |  |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 10 |  |  |
| sequenceGenerationConfig | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| CSI-RS-CellMobility[2] SEQUENCE { |  | Entry 2 |  |
| cellId | PhysCellId of NR neighbour cell |  |  |
| csi-rs-MeasurementBW SEQUENCE { |  |  |  |
| nrofPRBs | 48 |  |  |
| startPRB | 3 |  |  |
| } |  |  |  |
| density | d3 |  |  |
| csi-rs-ResourceList-Mobility SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility { | 1 entry |  |  |
| CSI-RS-Resource-Mobility[1] SEQUENCE { |  | Entry 1 |  |
| csi-RS-Index | 0 |  |  |
| slotConfig CHOICE { |  |  |  |
| ms20 | 1 |  |  |
| } |  |  |  |
| associatedSSB SEQUENCE { |  |  |  |
| ssb-Index | 0 |  |  |
| isQuasiColocated | true |  |  |
| } |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 1111 |  |  |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 10 |  |  |
| sequenceGenerationConfig | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

ReportConfigNR-DEFAULT

NR Report Configuration

Table H.3.1-4: ReportConfigNR-DEFAULT(a3-offset): NR report configuration for event A3

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | A3-offset*2 | The actual value is field value * 0.5 dB |  |
| rsrq | A3-offset*2 | The actual value is field value * 0.5 dB | RSRQ |
| sinr | A3-offset*2 | The actual value is field value * 0.5 dB | SINR |
| } |  |  |  |
| hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportInterval | ms120 |  | RSSI |
| reportQuantityRS-Indexes SEQUENCE { |  |  | SSB Index |
| rsrp | true |  |  |
| rsrq | false |  |  |
|  | true |  | RSRQ |
| sinr | false |  |  |
|  | true |  | SINR |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| maxNrofRS-IndexesToReport | 2 |  | SSB Index |
| measRSSI-ReportConfig-r16 SEQUENCE { |  |  |  |
| channelOccupancyThreshold-r16 | RSSI-Range-r16 | Integer (0..76), actual value is field value -101 dBm | RSSI |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB Index | To include SSB Index |
| RSRQ | RSRQ trigger quantity |
| SINR | SINR trigger quantity |
| RSSI | RSSI or Channel Occupancy measurements |

Table H.3.1-4AA: ReportConfigNR-DEFAULT(a4-threshold): NR report configuration for event A4

| Derivation Path: 38.508-1 [14], Table 4.6.3-142 with condition EVENT_A4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA4 SEQUENCE { |  |  |  |
| a4-Threshold CHOICE { |  |  |  |
| rsrp | a4-threshold+156 | For thresholds, the actual value is (IE value – 156) dBm |  |
| } |  |  |  |
| hysteresis | 0 |  |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantityRS-Indexes SEQUENCE { |  |  | SSB Index |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| maxNrofRS-IndexesToReport | 2 |  | SSB Index |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB Index | To include SSB Index |

ReportConfigInterRAT-DEFAULT

InterRAT NR Report Configuration.

Table H.3.1-4A: ReportConfigInterRAT- DEFAULT (b2-Thres1, b2-Thres2): InterRAT NR report configuration for FR1 to E-UTRAN handover with b2-Threshold1 = b2-Thres1 and b2-Threshold2EUTRA  = b2-Thres2 dBm

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-141 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB2 SEQUENCE { |  |  |  |
| b2-Threshold1 CHOICE { |  |  |  |
| rsrp | b2-Thres1 | b2-Thres1 – 156 dBm |  |
| } |  |  |  |
| b2-Threshold2EUTRA CHOICE { |  |  |  |
| rsrp | b2-Thres2 | b2-Thres2 – 140 dBm |  |
| } |  |  |  |
| hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

QuantityConfig-DEFAULT

NR quantity configuration when L3 filtering is not used.

Table H.3.1-5: QuantityConfig-DEFAULT: NR quantity configuration when L3 filtering is not used

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-127 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig::= SEQUENCE { |  |  |  |
| quantityConfigNR-List SEQUENCE (SIZE (1..maxNrofQuantityConfig)) OF SEQUENCE { | 1 entry |  |  |
| quantityConfigCell[1] SEQUENCE { |  |  |  |
| ssb-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 | No L3 filtering |  |
| filterCoefficientRSRQ | fc0 | No L3 filtering |  |
| filterCoefficientRS-SINR | fc0 | No L3 filtering |  |
| } |  |  |  |
| csi-RS-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 | No L3 filtering |  |
| filterCoefficientRSRQ | fc0 | No L3 filtering |  |
| filterCoefficientRS-SINR | fc0 | No L3 filtering |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfigRS-Index | Not present |  |  |
| quantityConfigRS-Index SEQUENCE { |  |  | SSB Index |
| ssb-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 | No L3 filtering |  |
| filterCoefficientRSRQ | fc0 | No L3 filtering |  |
| filterCoefficientRS-SINR | fc0 | No L3 filtering |  |
| } |  |  |  |
| csi-RS-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 | No L3 filtering |  |
| filterCoefficientRSRQ | fc0 | No L3 filtering |  |
| filterCoefficientRS-SINR | fc0 | No L3 filtering |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfigEUTRA | Not present |  |  |
| quantityConfigEUTRA SEQUENCE { |  |  | INTER-RAT |
| filterCoefficientRSRP | fc0 |  |  |
| filterCoefficientRSRQ | fc0 |  |  |
| filterCoefficientRS-SINR | fc0 |  |  |

| Condition | Explanation |
| --- | --- |
| INTER-RAT | Configuration for inter-RAT EUTRA measurement tests |
| SSB Index | SSB Index (RS Index) is reported |

MeasGapConfig-DEFAULT

Measurement gap configuration.

Table H.3.1-6: MeasGapConfig: measurement gap configuration

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-70 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapFR2 CHOICE { |  |  | gapFR2 |
| setup SEQUENCE { |  |  |  |
| gapOffset | 39 |  | Pattern #13 |
| mgl | ms5dot5 |  | Pattern #13 |
| mgrp | ms40 |  | Pattern #13 |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| gapFR1 CHOICE { |  |  | gapFR1 |
| setup SEQUENCE { |  |  |  |
| gapOffset | 39 |  | Pattern #2 |
|  | 19 |  | Pattern #4 |
| mgl | ms3 |  | Pattern #2 |
|  | ms6 |  | Pattern #4 |
| mgrp | ms40 |  | Pattern #2 |
|  | ms20 |  | Pattern #4 |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| gapUE CHOICE { |  |  | gapUE |
| setup SEQUENCE { |  |  |  |
| gapOffset | 39 |  | Pattern #0 OR Pattern #2 OR Pattern #13 |
|  | 0 |  | RLM OR BFD |
| mgl | ms6 |  | Pattern #0 OR RLM OR BFD |
|  | ms3 |  | Pattern #2 OR Pattern #3 |
|  | ms5dot5 |  | Pattern #13 |
| mgrp | ms40 |  | Pattern #0 OR Pattern #2 OR Pattern #13 OR RLM OR BFD |
|  | ms80 |  | Pattern #3 |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 SEQUENCE{ | 1 entry |  | MGe |
| GapConfig-r17[1] SEQUENCE{ |  |  |  |
| measGapId-r17 | 1 |  | Pattern #0 |
|  | 2 |  | Pattern #1 OR Pattern #13 |
|  | 3 |  | Pattern #24 |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 39 |  | Pattern #0 OR Pattern #13 |
|  | 19 |  | Pattern #1 |
|  | 11 |  | Pattern #24 |
| mgl-r17 | ms6 |  | Pattern #0 OR Pattern #1 |
|  | ms5dot5 |  | Pattern #13 |
|  | ms10 |  | Pattern #24 |
| mgrp-r17 | ms40 |  | Pattern #0 OR Pattern #13 |
|  | ms80 |  | Pattern #1 OR Pattern #24 |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
|  | true |  | Pre-CFG |
| ncsgInd-r17 | Not present |  |  |
|  | true |  | NCSG |
| gapAssociationPRS-r17 | Not present |  |  |
|  | true |  | PRS |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Pattern #0 | Measurement gap pattern #0 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |
| Pattern #1 | Measurement gap pattern #1 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |
| Pattern #2 | Measurement gap pattern #2 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |
| Pattern #3 | Measurement gap pattern #3 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |
| Pattern #4 | Measurement gap pattern #4 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |
| Pattern #13 | Measurement gap pattern #13 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |
| Pattern #24 | Measurement gap pattern #24 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |
| RLM | Measurement gap pattern for RLM tests |
| BFD | Measurement gap pattern for Beam Failure Dectection tests |
| gapFR2 | Indicates measurement gap configuration that applies to FR2 only. gapFR2 cannot be configured together with gapUE. In (NG)EN-DC or NE-DC, gapFR2 can only be set up by NR RRC |
| gapFR1 | Indicates measurement gap configuration that applies to FR1 only. gapFR1 cannot be configured together with gapUE. In (NG)EN-DC, gapFR1 cannot be set up by NR RRC |
| gapUE | Indicates measurement gap configuration that applies to all frequencies (FR1 and FR2). If gapUE is configured, then neither gapFR1 nor gapFR2 can be configured. In (NG)EN-DC, gapUE cannot be set up by NR RRC. |
| MGe | Indicates measurement gap enhancements configuration |
| Pre-CFG | Pre-configured measurement gap |
| NCSG | Measurement gap pattern for NCSG (network controlled small gap) tests |
| PRS | Measurement gap associated with PRS measurement |

MeasResults-DEFAULT

measurement result for NR measurements.

Table H.3.1-7: MeasResults: measurement result for NR measurements

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-79 with condition A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| measResults SEQUENCE { |  |  |  |
| measId | MeasId |  |  |
| measResultServingMOList | Not Checked |  | CSI-RS MOBILITY |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF SEQUENCE { | 2 entries |  |  |
| servCellId[1] | ServCellIndex of NR SpCell |  |  |
| measResultServingCell[1] SEQUENCE { |  |  |  |
| physCellId | PhysCellId of NR SpCell |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | (0..127) |  | SINR |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| servCellId[2] | ServCellIndex of NR SCell |  | Deactivated SCell |
| measResultServingCell[2] SEQUENCE { |  |  | Deactivated SCell |
| physCellId | PhysCellId of NR SCell |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | (0..127) |  | SINR |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE(1..maxCellReport)) OF SEQUENCE { | 1 entry |  | INTRA-FREQ OR INTER-FREQ |
| physCellId[1] | PhysCellId of NR neighbour Cell |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  | RSRQ |
| sinr | (0..127) |  | SINR |
| } |  |  |  |
| resultsSSB-Cell | Not Checked |  | CSI-RS MOBILITY |
| resultsCSI-RS-Cell SEQUENCE { |  |  | CSI-RS MOBILITY |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  | RSRQ |
| sinr | (0..127) |  | SINR |
| } |  |  |  |
| } |  |  |  |
| rsIndexResults | Not Checked |  | CSI-RS MOBILITY |
| rsIndexResults SEQUENCE { | n entires of ResultsPerSSB-Index | ResultsPerSSB-IndexList | SSB Index |
| resultsPerSSB-Index SEQUENCE { | entry [1] |  |  |
| ssb-Index | SSB-Index | an SS-Block within an SS-Burst |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  | INTER-RAT |
| eutra-PhysCellId [1] | PhysCellId of E-UTRA neighbour Cell |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrp | (0..97) |  |  |
| rsrq | (0..34) |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB Index | To include SSB Index |
| INTRA-FREQ | Configuration for intra-frequency NR measurement tests |
| INTER-FREQ | Configuration for inter-frequency NR measurement tests |
| INTER-RAT | Configuration for inter-RAT EUTRA measurement tests |
| Deactivated SCell | Configuration for measurement on Deactivated SCell tests |
| RSRQ | RSRQ measurement reporting |
| SINR | SINR measurement reporting |
| CSI-RS MOBILITY | Configuration for CSI-RS based measurement tests |

RadioLinkMonitoringConfig -DEFAULT

Default configuration for RLM resources.

Table H.3.1-8: RadioLinkMonitoringConfig-DEFAULT: Default configuration for RLM and BFD resources

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF { | n entries | n = 1 for FR1, n = 2 for FR2 with 2SSB or second resource set of TRS, n = 1 otherwise | SSB RLM OR CSI-RS RLM |
| RadioLinkMonitoringRS[1] SEQUENCE { |  |  |  |
| radioLinkMonitoringRS-Id | 0 |  |  |
| purpose | rlf |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 | Index of SSB #0 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| RadioLinkMonitoringRS[2] SEQUENCE { |  |  | FR2 |
| radioLinkMonitoringRS-Id | 1 |  |  |
| purpose | rlf |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 1 | Index of SSB #1 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) with condition SECOND_SET | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF { | 2 entries |  | SSB BFD OR CSI-RS BFD |
| RadioLinkMonitoringRS[1] SEQUENCE { |  |  |  |
| radioLinkMonitoringRS-Id | 0 |  |  |
| purpose | both |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 | Index of SSB #0 | SSB BFD |
| csi-RS-Index | NZP-CSI-RS-ResourceId for BM(0) | TS 38.508-1[14], Table 7.3.1-7E | CSI-RS BFD |
| } |  |  |  |
| } |  |  |  |
| RadioLinkMonitoringRS[2] SEQUENCE { |  |  |  |
| radioLinkMonitoringRS-Id | 1 |  |  |
| purpose | rlf |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 1 | Index of SSB #1 | SSB BFD |
| csi-RS-Index | NZP-CSI-RS-ResourceId for BM(1) | TS 38.508-1[14], Table 7.3.1-7E | CSI-RS BFD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| beamFailureInstanceMaxCount | n1 |  | SSB BFD OR CSI-RS BFD |
| beamFailureDetectionTimer | pbfd4 |  | SSB BFD OR CSI-RS BFD |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB RLM | Used when SSB based RLM is configured in test case |
| CSI-RS RLM | Used when CSI-RS based RLM is configured in test case |
| SSB BFD | Used when SSB based BFD is configured in test case |
| CSI-RS BFD | Used when CSI-RS based BFD is configured in test case |
| 2SSB | For configuration with 2 SS Blocks |

Table H.3.1-8A: RadioLinkMonitoringConfig-Release 17: Release 17 configuration for TRP specific RLM and BFD resources

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| beamFailure-r17 ::= SEQUENCE { |  |  |  |
| failureDetectionSet1-r17::= SEQUENCE { |  |  |  |
| bfdResourcesToAddModList-r17 SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF BeamLinkMonitoringRS-r17 { | n entries | n = 1 for FR1, n = 2 for FR2 | SSB RLM OR CSI-RS RLM |
| BeamLinkMonitoringRS-r17[1] ::= SEQUENCE { |  | entry 1 |  |
| beamLinkMonitoringRS-Id-r17 | 0 |  |  |
| detectionResource-r17 CHOICE { |  |  |  |
| ssb-Index | 0 | Index of SSB #0 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| BeamLinkMonitoringRS-r17[2] ::= SEQUENCE { |  | entry 2 |  |
| beamLinkMonitoringRS-Id-r17 | 1 |  |  |
| detectionResource-r17 CHOICE { |  |  |  |
| ssb-Index | 1 | Index of SSB #1 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) with condition SECOND_SET | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bfdResourcesToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF BeamLinkMonitoringRS-Id-r17 { | 2 entries |  | SSB BFD OR CSI-RS BFD |
| BeamLinkMonitoringRS-Id-r17[1] | 0 | entry 1 |  |
| BeamLinkMonitoringRS-Id-r17[2] | 1 | entry 2 |  |
| } |  |  |  |
| beamFailureInstanceMaxCount-r17 | n1 |  | SSB BFD OR CSI-RS BFD |
| beamFailureDetectionTimer-r17 | pbfd4 |  | SSB BFD OR CSI-RS BFD |
| } |  |  |  |
| failureDetectionSet2-r17::= SEQUENCE { |  |  |  |
| bfdResourcesToAddModList-r17 SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF BeamLinkMonitoringRS-r17 { | n entries | n = 1 for FR1, n = 2 for FR2 | SSB RLM OR CSI-RS RLM |
| BeamLinkMonitoringRS-r17[1] ::= SEQUENCE { |  | entry 1 |  |
| beamLinkMonitoringRS-Id-r17 | 2 |  |  |
| detectionResource-r17 CHOICE { |  |  |  |
| ssb-Index | 2 | Index of SSB #2 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| BeamLinkMonitoringRS-r17[2] ::= SEQUENCE { |  | entry 2 |  |
| beamLinkMonitoringRS-Id-r17 | 3 |  |  |
| detectionResource-r17 CHOICE { |  |  |  |
| ssb-Index | 3 | Index of SSB #3 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) with condition SECOND_SET | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bfdResourcesToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OBeamLinkMonitoringRS-Id-r17 F { | 2 entries |  | SSB BFD OR CSI-RS BFD |
| BeamLinkMonitoringRS-Id-r17[1] | 2 | entry 1 |  |
| BeamLinkMonitoringRS-Id-r17[2] | 3 | entry 2 |  |
| } |  |  |  |
| beamFailureInstanceMaxCount-r17 | n1 |  | SSB BFD OR CSI-RS BFD |
| beamFailureDetectionTimer-r17 | pbfd4 |  | SSB BFD OR CSI-RS BFD |
| } |  |  |  |
| additionalPCI-r17 | not present |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB RLM | Used when SSB based RLM is configured in test case |
| CSI-RS RLM | Used when CSI-RS based RLM is configured in test case |
| SSB BFD | Used when SSB based BFD is configured in test case |
| CSI-RS BFD | Used when CSI-RS based BFD is configured in test case |

RLF-TimersAndConstants-DEFAULT

Default parameters for RLM related timers and counters.

Table H.3.1-9: RLF-TimersAndConstants-DEFAULT

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms0 |  |  |
| } |  |  |  |

BeamFailureRecoveryConfig-DEFAULT

Default configuration for CBD and contention-free RACH in link recovery.

Table H.3.1-10: BeamFailureRecoveryConfig-DEFAULT

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BeamFailureRecoveryConfig ::= SEQUENCE { |  |  |  |
| rootSequenceIndex-BFR | 0 |  |  |
| rach-ConfigBFR | RACH-ConfigGeneric specified in TS.38.508-1 [14] Table 7.3.1-14 |  | SSB |
|  | RACH-ConfigGeneric specified in TS.38.508-1 [14] Table 7.3.1-14 with condition PRACH.4 FR1 |  | CSI-RS and FR1 |
|  | RACH-ConfigGeneric specified in TS.38.508-1 [14] Table 7.3.1-14 with condition PRACH.4 FR2 |  | CSI-RS and FR1 |
| rsrp-ThresholdSSB | 58 | Actual value is 58 -156 = -98 dBm | SCS15 |
|  | 61 | Actual value is 61 -156 = -95 dBm | SCS30 |
|  | 47 | Actual value is 47 -156 = -109 dBm | SCS120 |
|  | 50 | Actual value is 50 -156 = -106 dBm | SCS240 |
| candidateBeamRSList SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF CHOICE { | 1 entry |  |  |
| ssb[1] SEQUENCE { |  |  | SSB |
| ssb | 1 |  |  |
| ra-PreambleIndex | 40 |  |  |
| } |  |  |  |
| csi-RS[1] SEQUENCE { |  |  | CSI-RS |
| csi-RS | NZP-CSI-RS-ResourceId of the CSI-RS resource used for CBD |  |  |
| ra-OccasionList | 1 |  |  |
| ra-PreambleIndex | 40 |  |  |
| } |  |  |  |
| } |  |  |  |
| ssb-perRACH-Occasion | oneFourth |  |  |
| ra-ssb-OccasionMaskIndex | Not present |  |  |
|  | 1 |  | PRACH.2 FR1 OR PRACH.2 FR2 |
| recoverySearchSpaceId | SearchSpaceId of the search space used for BFD RAR in DL active BWP |  |  |
| ra-Prioritization | Not present |  |  |
| beamFailureRecoveryTimer | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB | Configuration for SSB based CBD |
| CSI-RS | Configuration for CSI-RS based CBD |

BWP-UplinkDedicated

Default BWP configuration for Beam Failure Recovery

Table H.3.1-10A: BWP-UplinkDedicated

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| beamFailureRecoveryConfig | BeamFailureRecoveryConfig-DEFAULT |  |  |
| } |  |  |  |

RACH-ConfigCommon

Default RACH configuration for Beam Failure Recovery

Table H.3.1-10B: RACH-ConfigCommon

| Derivation Path: TS 38.508-1 [14] Table 7.3.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| oneFourth | n40 |  |  |
| } |  |  |  |
| } |  |  |  |

Table H.3.1-11: Void

BeamFailureRecoverySCellConfig-DEFAULT

Default configuration for CBD and contention-free RACH in link recovery.

Table H.3.1-12: BeamFailureRecoverySCellConfig-DEFAULT

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-6AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BeamFailureRecoveryRSConfig-r16 ::= SEQUENCE { |  |  |  |
| rsrp-ThresholdBFR-r16 | 58 | -98dBm | SCS15 |
|  | 61 | -95dBm | SCS30 |
|  | 47 | -109dBm | SCS120 |
| candidateBeamRS-List-r16 SEQUENCE (SIZE(1..maxNrofCandidateBeams-r16)) OF CandidateBeamRS-r16 { | 1 entry |  |  |
| CandidateBeamRS-r16[1] SEQUENCE { |  | entry 1 |  |
| candidateBeamConfig-r16 CHOICE { |  |  |  |
| ssb-r16 | 1 |  | SSB |
| csi-RS-r16 | NZP-CSI-RS-ResourceId of the CSI-RS resource used for CBD |  | CSI-RS |
| } |  |  |  |
| servingCellId | ServCellIndex of the SCell |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table H.3.1-12A: BeamFailureRecoverySpCellConfig-DEFAULT

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BeamFailureRecoveryRSConfig-r16 ::= SEQUENCE { |  |  |  |
| rsrp-ThresholdBFR-r16 | 58 | -98dBm | SCS15 |
|  | 61 | -95dBm | SCS30 |
|  | 47 | -109dBm | SCS120 |
| candidateBeamRS-List2-r17 SEQUENCE (SIZE(1..maxNrofCandidateBeams-r16)) OF CandidateBeamRS-r16 { | 1 entry |  |  |
| CandidateBeamRS-r16[1] SEQUENCE { |  | entry 1 |  |
| candidateBeamConfig-r16 CHOICE { |  |  |  |
| ssb-r16 | 1 |  | SSB |
| csi-RS-r16 | NZP-CSI-RS-ResourceId of the CSI-RS resource used for CBD |  | CSI-RS |
| } |  |  |  |
| servingCellId | ServCellIndex of the SpCell |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

BWP-DownlinkDedicated

Table H.3.1-13: BWP-DownlinkDedicated

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| beamFailureRecoverySCellConfig-r16 CHOICE { |  |  |  |
| setup | BeamFailureRecoverySCellConfig-DEFAULT |  | SCell |
| } |  |  |  |
| } |  |  |  |
| beamFailureRecoverySpCellConfig-r17 CHOICE { |  |  |  |
| setup | BeamFailureRecoverySpCellConfig-DEFAULT |  | SpCell |
| } |  |  |  |
| } |  |  |  |

MeasWindowConfig-DEFAULT

Table H.3.1-14: MeasWindowConfig-DEFAULT

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-80C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasWindowConfig-r18 ::= SEQUENCE { |  |  |  |
| windowOffsetPeriodicity CHOICE { |  |  |  |
| periodicityMs40 | 0 |  | MS40 |
| periodicityMs80 | 0 |  | MS80 |
| } |  |  |  |
| windowDuration | ms5 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MS40 | 40 ms for measurement window periodicity |
| MS80 | 80 ms for measurement window periodicity |

NeedForInterruptionNR

Table H.3.1-15: NeedForInterruptionNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NeedForInterruptionNR-r18 ::= SEQUENCE { |  |  |  |
| interruptionIndication-r18 | no-gap-with-interruption |  | NGWI |
|  | no-gap-no-interruption |  | NGNI |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NGWI | interruption is needed to perform SSB based measurements without measurement gap |
| NGNI | interruption is not needed to perform SSB based measurements without measurement gap |

## H.3.2	RRC messages and information elements contents exceptions for handover

Table H.3.2-1: Void

Table H.3.2-2: RRCReconfiguration-HO

| Derivation Path: TS 38.508-1 [14], Table 4.8.1-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions SRB_NR_PDCP and DRBn and Re-establish_PDCP |  | RBConfig_KeyChange |
|  | RadioBearerConfig with conditions DRBn and Recover_PDCP |  | RBConfig_NoKeyChange |
| secondaryCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | Table H.3.2-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RBConfig_KeyChange | RadioBearerConfig to perform Intra-NR handover with security key change |
| RBConfig_NoKeyChange | RadioBearerConfig to perform Intra-NR handover without security key change |

Table H.3.2-3: CellGroupConfig

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-19 with condition PCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| smtc | SMTC specified in TS.38.508-1 [14] Table 7.3.1-3 with condition SMTC.n | SMTC.n is the SMTC reference configuration used in test case |  |
| } |  |  |  |
| } |  |  |  |

## H.3.3	RRC messages and information elements contents exceptions for NR inter-RAT handover

MobilityFromNRCommand

For Inter-RAT NR handover.

Table H.3.3-1: MobilityFromNRCommand: InterRAT NR handover

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityFromNRCommand::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| mobilityFromNRCommand SEQUENCE { |  |  |  |
| targetRAT-Type | eutra |  |  |
| targetRAT-MessageContainer | OCTET STRING including the RRCConnectionReconfiguration message according to TS 36.508 [25], table 4.6.1-8 with condition HO-TO-EUTRA |  |  |
| nas-SecurityParamFromNR | The 4 LSB of the downlink NAS COUNT |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

MobilityFromEUTRACommand

Table H.3.3-2: MobilityFromEUTRACommand: inter-RAT handover to NR Cell

| Derivation Path:TS 36.508 [25] Table 4.6.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityFromEUTRACommand ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| mobilityFromEUTRACommand-r9 SEQUENCE { |  |  |  |
| purpose CHOICE { |  |  |  |
| handover SEQUENCE { |  |  |  |
| targetRAT-Type | nr |  |  |
| targetRAT-MessageContainer | OCTET STRING containing RRCReconfiguration according to TS 38.508-1 [14] with Condition NR |  |  |
| nas-SecurityParamFromEUTRA | Not present |  |  |
| systemInformation | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## H.3.4	E-UTRA RRC messages and information elements contents exceptions

RRCConnectionReconfiguration

Includes the nr-SecondaryCellGroupConfig-r15 to convey NR RRCReconfiguration message as specified in TS 38.331 [13].

Table H.3.4-1: RRCConnectionReconfiguration: NR RRC Reconfiguration in EN-DC

| Derivation Path: 36.508 [25], Table 4.6.1-8 with condition MCG_and_SCG |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | RRCReconfiguration |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table H.3.4-1a: RRCConnectionReconfiguration for measurement configuration

For measurement configuration provided by NR PSCell under EN-DC.

| Derivation Path: 36.508 [25], Table 4.6.1-8 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
|  | MeasConfig-DEFAULT | Table H.3.4-4 | GAP_NEEDED |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | RRCReconfiguration | Table H.3.1-1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GAP_NEEDED | Measurement gap is needed in Test |

ULInformationTransferMRDC

uplink transfer of MR DC information for transferring the NR RRC Measurement Report message in EN-DC.

Table H.3.4-2: ULInformationTransferMRDC: uplink transfer of MR DC information

| Derivation Path: 36.508 [25], Table 4.6.1-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ULInformationTransferMRDC ::= SEQUENCE { |  |  |  |
| ul-DCCH-MessageNR-r15 | OCTET STRING including the MeasurementReport |  |  |
| } |  |  |  |

RRCConnectionReconfigurationComplete

Includes the scg-ConfigResponseNR to convey NR RRCReconfigurationComplete message as specified in TS 38.331 [13].

Table H.3.4-3: RRCConnectionReconfigurationComplete: NR RRC Reconfiguration Complete in EN-DC

| Derivation Path: 36.508 [25], Table 4.6.1-9: with condition MCG_and_SCG |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReconfigurationComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| scg-ConfigResponseNR-r15 | OCTET STRING including the RRCReconfigurationComplete message according to TS 38.508-1 [14], Table 4.6.1-14. |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

MeasConfig-DEFAULT

Configures measurement gap that applies to FR1 only in EN-DC as specified in TS 38.331 [13].

Table H.3.4-4: MeasConfig-DEFAULT

| Derivation Path: 36.508 [25],Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] CHOICE { |  |  |  |
| measObjectEUTRA | MeasObjectEUTRA-GENERIC(Freq) | Freq is the frequency of E-UTRA PCell |  |
| } |  |  |  |
| } |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | INTER-RAT NR |
| measObjectId[1] | 1 |  |  |
| measObject[1] CHOICE { |  |  |  |
| measObjectEUTRA | MeasObjectEUTRA-GENERIC(Freq) | Freq is the frequency of E-UTRA PCell |  |
| } |  |  |  |
| measObjectId[2] | 2 |  |  |
| measObject[2] CHOICE { |  |  |  |
| measObjectNR-r15 | MeasObjectNR | Table H.3.4-6 |  |
| } |  |  |  |
| } |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 3 entries |  | E-UTRA_Deactivated_SCell |
| measObjectId[1] | 1 |  |  |
| measObject[1] CHOICE { |  |  |  |
| measObjectEUTRA | MeasObjectEUTRA-GENERIC(Freq) | Freq is the frequency of E-UTRA PCell Table H.3.4-9 |  |
| } |  |  |  |
| measObjectId[2] | 2 |  |  |
| measObject[2] CHOICE { |  |  |  |
| measObjectEUTRA | MeasObjectEUTRA-GENERIC(Freq) | Freq is the frequency of E-UTRA SCell Table H.3.4-9 | E-UTRA_Deactivated_SCell |
| } |  |  |  |
| measObjectId[3] | 3 |  |  |
| measObject[3] CHOICE { |  |  |  |
| measObjectNR-r15 | MeasObjectNR | Table H.3.4-6 |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  | INTER-RAT NR OR E-UTRAN_Deactivated_SCell |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] CHOICE { |  |  |  |
| reportConfigEUTRA | ReportConfigEUTRA-A4(Thres) | Set Thres according to test parameters specified in test cases | EVENT A4 |
|  | ReportConfigEUTRA-A3(A3-offset) | Table H.3.4-10 | E-UTRA_Deactivated_SCell |
| reportConfigInterRAT | ReportConfigInterRAT-B2-NR(EUTRA-Thres, NR-Thres) | Set EUTRA-Thres and NR-Thres according to test parameters specified in test cases | EVENT B2 |
|  | ReportConfigInterRAT-B1-NR(EUTRA-Thres, NR-Thres) | Set EUTRA-Thres and NR-Thres according to test parameters specified in test cases | EVENT B1 |
|  | ReportConfigInterRAT-SFTD with Condition INTER-RAT | TS 36.508 [25] Table 4.6.6-8B | SFTD |
|  | ReportConfigInterRAT-PERIODICAL | Table 4.6.6-9 in TS 36.508 [25] | PERIODICAL |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  | INTER-RAT NR OR E-UTRA_Deactivated_SCell |
| measId[1] | 1 |  |  |
| measObjectId[1] | 2 |  |  |
| reportConfigId[1] | 1 |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-FR1 | Table H.3.4-5 |  |
|  | Not present |  | GAPLESS |
| fr1-Gap-r15 | false |  | gapUE |
|  | true |  | gapFR1 |
|  | Not present |  | E-UTRA_Deactivated_SCell |
| quantityConfig | QuantityConfig-DEFAULT | Table H.3.4-11 | E-UTRA_Deactivated_SCell |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| gapUE | fr1-Gap-r15 set to false indicates the gap is applicable for measurements on FR1 and FR2. E-UTRAN includes this field only when the UE is configured with (NG)EN-DC. |
| gapFR1 | fr1-Gap-r15 set to true indicates the gap is only applicable for measurements on FR1. E-UTRAN includes this field only when the UE is configured with (NG)EN-DC. |
| INTER-RAT NR | Measurement configuration for inter-RAT NR measurements |
| EVENT B2 | For event B2 triggered measurement reporting test cases |
| EVENT B1 | For event B1 triggered measurement reporting test cases |
| EVENT A4 | For event A4 triggered measurement reporting |
| SFTD | For inter-RAT SFTD measurement test cases |
| GAPLESS | MG is not needed in test |
| PERIODICAL | For periodic measurement reporting test cases |
| E-UTRA_Deactivated_SCell | For test cases with an LTE deactivated SCell |

MeasGapConfig

Configures measurement gap by LTE RRC in EN-DC as specified in TS 38.331 [13].

Table H.3.4-5: MeasGapConfig

| Derivation Path: 36.508 [25],Table 4.6.6-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset CHOICE { |  |  |  |
| gp0 | 39 | TGRP = 40 ms | Pattern #0 |
|  | 0 |  | BFD |
| gp4-r15 | 19 | TGRP = 20 ms | Pattern #4 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Pattern #0 | gp0 corresponds to gap offset of Gap Pattern Id "0" with MGRP = 40ms |
| Pattern #4 | gp4-r15 corresponds to gap offset of Gap Pattern Id "4" with MGRP = 40ms (see TS 38.133, Table 9.1.2-1). It can be applied for (NG)EN-DC, see TS 38.133, Table 9.1.2-2. |
| BFD | Measurement gap pattern for Beam Failure Dectection tests |

Table H.3.4-6: MeasObjectNR

| Derivation Path: TS 36.508 [25] Table 4.6.6-2B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| rs-ConfigSSB-r15 ::= SEQUENCE { |  |  |  |
| measTimingConfig-r15 ::= SEQUENCE { |  |  |  |
| periodicityAndOffset-r15 CHOICE { |  |  |  |
| sf20-r15 | 0 |  | SMTC.1 or SMTC.2 |
|  | 10 |  | SMTC.4 or SMTC.5 |
|  | 17 |  | SMTC.6 |
| sf160-r15 | 0 |  | SMTC.3 |
| } |  |  |  |
| ssb-Duration-r15 | sf1 |  | SMTC.1, SMTC.3 or SMTC.4 |
|  | sf5 |  | SMTC.2, SMTC.5 or SMTC.6 |
| } |  |  |  |
| subcarrierSpacingSSB-r15 | kHz15 |  | SSB.1 FR1, SSB.3 FR1 or SSB.5 FR1 |
|  | kHz30 |  | SSB.2 FR1, SSB.4 FR1 or SSB.6 FR1 |
|  | kHz120 |  | SSB.1 FR2, SSB.3 FR2, SSB.5 FR2 or SSB.7 FR2 |
|  | KHz240 |  | SSB.2 FR2, SSB.4 FR2, SSB.6 FR2 or SSB.8 FR2 |
| } |  |  |  |
| deriveSSB-IndexFromCell-r15 | Not present |  |  |
|  | true |  | Synchronous cells |
|  | false |  | Asynchronous cells |
| } |  |  |  |

Table H.3.4-7: RRCConnectionReconfiguration for iRAT measurement configuration

| Derivation Path: 36.508 [25], Table 4.6.1-8 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |
| measConfig | MeasConfig-DEFAULT with Conditions INTER RAT NR and PERIODICAL | Table H.3.4-4 |  |
|  | MeasConfig-DEFAULT with Conditions INTER RAT NR and SFTD | Table H.3.4-4 | INTER-RAT and SFTD |
|  | MeasConfig-DEFAULT with Conditions INTER RAT NR and EVENT B2 | Table H.3.4-4 | INTER-RAT and EVENT B2 |
|  | MeasConfig-DEFAULT with Conditions INTER RAT NR and EVENT B1 | Table H.3.4-4 | INTER-RAT and EVENT B1 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table H.3.4-8: ReportConfigInterRAT-B2-NR(EUTRA-Thres, NR-Thres)

| Derivation Path: 36.508 [25], Table 4.6.6-8A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT-B2-NR ::= SEQUENCE { |  |  |  |
| maxReportRS-Index-r15 | 2 |  | SSB-Index |
| reportQuantityRS-IndexNR-r15 SEQUENCE { |  |  | SSB-Index |
| ss-rsrp | TRUE |  |  |
| ss-rsrq | FALSE |  |  |
| ss-sinr | FALSE |  |  |
| } |  |  |  |
| reportRS-IndexResultsNR | TRUE |  | SSB-Index |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB-Index | To include SSB Index |

Table H.3.4-9: MeasObjectEUTRA-GENERIC(Freq)

| Derivation Path: 36.508 [25],Table 7.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| measCycleSCell-r10 | sf640 |  | E-UTRA_Deactivated_SCell |
| } |  |  |  |

Table H.3.4-10: ReportConfigEUTRA-A3(a3-offset)

| Derivation Path: 36.508 [25],Table 4.6.6-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A3 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset | A3-offset *2 | The actual value is field value * 0.5 dB |  |
| } |  |  |  |
| } |  |  |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity | sameAsTriggerQUantity |  |  |
| maxReportCells | 2 |  |  |
| reportInterval | ms480 |  |  |
| reportAmount | r2 |  |  |
| } |  |  |  |

Table H.3.4-11: QuantityConfig-DEFAULT

| Derivation Path: 36.508 [25],Table 4.6.6-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| quantityConfigEUTRA SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 |  |  |
| filterCoefficientRSRQ | fc0 |  |  |
| } |  |  |  |
| quantityConfigNRList-r15 (SIZE (1..maxQuantSetsNR-r15)) OF SEQUENCE { | 1 entries |  |  |
| measQuantityCellNR-r15[1] SEQUENCE { |  |  |  |
| filterCoeff-RSRP-r15 | fc0 |  |  |
| filterCoeff-RSRQ-r15 | fc0 |  |  |
| filterCoefficient-SINR-r13 | fc0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## H.3.5	RRC messages and information elements contents exceptions for NR radio link monitoring (RLM)

CSI-RS information elements contents exception for NR RLM SSB-Based test cases

Table H.3.5-1 to H.3.5-3: Void

Table H.3.5-4: CSI-ReportConfig

| Derivation Path: TS 38.508[14] Table 7.3.1-12F |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| reportSlotConfig ::= CHOICE { |  |  |  |
| slots5 | 0 |  | SCS15kHz_FDD |
| slots5 | 2 |  | SCS15kHz_TDD |
| slots10 | 4 |  | SCS30kHz |
| slots40 | 16 |  | SCS120kHz |
| } |  |  |  |
| pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF{ |  |  |  |
| PUCCH_CSI_Resource[0] SEQUENCE { |  |  |  |
| uplinkBandwidthPartId | BWP-Id |  |  |
| pucch_Resource | 9 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table H.3.5-5 to H.3.5-7: Void

BWP-DownlinkDedicated: to setup radioLinkMonitoringConfig Configuration

Table H.3.5-8: Void

RadioLinkMonitoringConfig: Configuration for RLM-RS

Table H.3.5-9: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | n entries | n = 1 for FR1, n = 2 for FR2 |  |
| radioLinkMonitoringRS-Id[1] | 0 |  |  |
| purpose[1] | rlf |  |  |
| detectionResource[1] CHOICE { |  |  |  |
| ssb-Index | 0 | Index of SSB #0 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| RadioLinkMonitoringRS[2] SEQUENCE { |  |  | FR2 |
| radioLinkMonitoringRS-Id | 1 |  |  |
| purpose | rlf |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 1 | Index of SSB #1 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) with condition SECOND_SET | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| failureDetectionResourcesToReleaseList | Not present |  |  |
| beamFailureInstanceMaxCount | Not present |  |  |
| beamFailureDetectionTimer | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB RLM | Configuration for SSB based RLM test cases |
| CSI-RS RLM | Configuration for CSI-RS based RLM test cases |

Table H.3.5-10: Void

## H.3.6	RRC messages and IE content exceptions for L1-RSRP measurement for beam reporting

ServingCellConfig: Default generic configuration for enabling CSI measurements and reporting

Table H.3.6-1: Void

CSI-ReportConfig: Default generic configuration for L1-RSRP measurements

Table H.3.6-2: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-12G |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| csi-IM-ResourcesForInterference | Not present |  |  |
| nzp-CSI-RS-ResourcesForInterference | Not present |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  | PERIODIC |
| reportSlotConfig CHOICE { |  |  |  |
| slot80 | 7 | Periodicity 80 slots and offset 7 | SCS30 |
|  | 2 | Periodicity 80 slots and offset 2 | SCS15 |
| slot320 | 4 | Periodicity 320 slots and offset 4 | FR2 |
| pucch-CSI-ResourceList SEQUENCE { | 1 entry |  |  |
| (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| uplinkBandwidthPartId | BWP-Id of active BWP |  |  |
| pucch-Resource | 8 | PUCCH-format2 as configured in TS 38.508-1 [14], Table 4.6.3-112 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| ssb-Index-RSRP | NULL |  | SS-RSRP |
| cri-RSRP | NULL |  | CSI-RSRP |
| } |  |  |  |
| timeRestrictionForChannelMeasurements | configured |  |  |
| groupBasedBeamReporting CHOICE { |  |  |  |
| disabled SEQUENCE { |  |  |  |
| nrofReportedRS | n2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PERIODIC | Configuration for periodic reporting |
| SS-RSRP | L1-RSRP measurement based on SSB |
| CSI-RSRP | L1-RSRP measurement based on CSI-RS |

CSI-ResourceConfig: Default generic resource configuration for L1-RSRP measurements

Table H.3.6-3: CSI-ResourceConfig

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-12B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF { | 1 entry |  | CSI-RS |
| NZP-CSI-RS-ResourceSetId[0] | 0 |  |  |
| } |  |  |  |
| csi-SSB-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF { | 1 entry |  | SSB |
| CSI-SSB-ResourceSetId[0] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB | Configuration for SSB based CSI |
| CSI-RS | Configuration for CSI-RS based CSI |

Table H.3.6-4 to H.3.6-9: Void

CSI-MeasConfig: Default generic resource configuration for L1-RSRP measurements

Table H.3.6-10: CSI-MeasConfig

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| csi-SSB-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet { |  |  |  |
| CSI-SSB-ResourceSet[1] | CSI-SSB-ResourceSet-RRM | TS 38.508-1 Table 7.3.1-29 |  |
| } |  |  |  |
| } |  |  |  |

## H.3.6A	RRC messages and IE content exceptions for L1-SINR measurement for beam reporting

ServingCellConfig: Default generic configuration for enabling CSI measurements and reporting.

CSI-ReportConfig: Default generic configuration for L1-SINR measurements.

Table H.3.6A-1: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-12G |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| csi-IM-ResourcesForInterference | CSI-IM-ResourceConfigId-IMR |  | CSI-IM_IMR |
|  | CSI-IM-ResourceConfigId-IMR with condition APERIODIC |  | CSI-IM_IMR AND APERIODIC |
| nzp-CSI-RS-ResourcesForInterference | CSI-ResourceConfigId-IMR |  | CSI-RS_IMR |
|  | CSI-ResourceConfigId-IMR with condition APERIODIC |  | CSI-RS_IMR AND APERIODIC |
| reportConfigType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  | PERIODIC |
| reportSlotConfig CHOICE { |  |  |  |
| slot80 | 7 | Periodicity 80 slots and offset 7 | SCS30 |
|  | 2 | Periodicity 80 slots and offset 2 | SCS15 |
|  | 4 | Periodicity 80 slots and offset 4 | FR2 |
| pucch-CSI-ResourceList SEQUENCE { | 1 entry |  |  |
| (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| uplinkBandwidthPartId | BWP-Id of active BWP |  |  |
| pucch-Resource | 8 | PUCCH-format2 as configured in TS 38.508-1 [14], Table 4.6.3-112 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| aperiodic SEQUENCE { |  |  | APERIODIC |
| reportSlotOffsetList SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF { | 1 entry |  |  |
| INTEGER[1] | 26 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| timeRestrictionForChannelMeasurements | configured |  |  |
| groupBasedBeamReporting CHOICE { |  |  |  |
| disabled SEQUENCE { |  |  |  |
| nrofReportedRS | n2 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity-r16 CHOICE { |  |  |  |
| cri-SINR-r16 | NULL |  | CSI-SINR |
| ssb-Index- SINR-r16 | NULL |  | SS-SINR |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PERIODIC | Configuration for periodic reporting |
| APERIODIC | Configuration for aperiodic reporting |
| SS-SINR | L1-SINR measurement based on SSB |
| CSI- SINR | L1-SINR measurement based on CSI-RS |
| CSI-RS_IMR | Configuration for NZP-CSI-RS based IMR |
| CSI-IM_IMR | Configuration for CSI-IM based IMR |

CSI-ResourceConfig for CMR: Default generic resource configuration for L1-SINR measurements.

Table H.3A.6-2: CSI-ResourceConfig for CMR

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-12B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF { | 1 entry |  | CSI-RS |
| NZP-CSI-RS-ResourceSetId[1] | 0 |  |  |
| } |  |  |  |
| csi-SSB-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF { | 1 entry |  | SSB |
| CSI-SSB-ResourceSetId[1] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id of active BWP |  |  |
| resourceType | aperiodic |  | APERIODIC |
|  | periodic |  | PERIODIC |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB | Configuration for SSB based CMR |
| CSI-RS | Configuration for CSI-RS based CMR |

CSI-ResourceConfig for IMR: For NZP-CSI-RS resource configured as dedicated IMR, refer to Table H.3A.6-3; For CSI-IM resource configured as dedicated IMR, refer to Table H.3A.6-4.

Table H.3A.6-3: CSI-ResourceConfig for NZP-CSI-RS as IMR

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-12B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId-IMR |  |  |
|  | CSI-ResourceConfigId-IMR with condition APERIODIC |  | APERIODIC |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | NZP-CSI-RS-ResourceSetId-IMR |  |  |
|  | NZP-CSI-RS-ResourceSetId-IMR with condition APERIODIC |  | APERIODIC |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id of active BWP |  |  |
| resourceType | aperiodic |  | APERIODIC |
|  | periodic |  | PERIODIC |
| } |  |  |  |

Table H.3A.6-3a: NZP-CSI-RS-ResourceSet for IMR

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-11B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp-CSI-ResourceSetId | NZP-CSI-RS-ResourceSetId-IMR |  |  |
|  | NZP-CSI-RS-ResourceSetId-IMR with condition APERIODIC |  | APERIODIC |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId for IMR(0) | INTEGER 12 |  |
|  | NZP-CSI-RS-ResourceId for IMR(0) with condition APERIODIC | INTEGER 22 | APERIODIC |
| NZP-CSI-RS-ResourceId[2] | NZP-CSI-RS-ResourceId for IMR(1) | INTEGER 13 |  |
|  | NZP-CSI-RS-ResourceId for IMR(1) with condition APERIODIC | INTEGER 23 | APERIODIC |
| } |  |  |  |
| aperiodicTriggeringOffset | 6 |  | APERIODIC |
| } |  |  |  |

Table H.3A.6-3b: NZP-CSI-RS-Resource for IMR(Id)

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-7B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | NZP-CSI-RS-ResourceId for IMR(Id) |  |  |
|  | NZP-CSI-RS-ResourceId for IMR(Id) with condition APERIODIC |  | APERIODIC |
| resourceMapping SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 |  |  |
| } |  |  |  |
| nrofPorts | p1 |  |  |
| firstOFDMSymbolInTimeDomain | 6 |  | Id = 0 |
|  | 10 |  | Id = 1 |
|  | 7 |  | Id = 0 AND APERIODIC |
|  | 11 |  | Id = 1 AND APERIODIC |
| cdm-Type | noCDM |  |  |
| density CHOICE { |  |  |  |
| three | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation-RRM |  | See TS 38.508-1 [14], Table 7.3.1-10 |
| } |  |  |  |
| powerControlOffset | 0 |  |  |
| powerControlOffsetSS | db0 |  |  |
| scramblingID | 0 |  |  |
| periodicityAndOffset { |  |  |  |
| slots20 | 1 |  | SCS15 |
| slots40 | 2 |  | SCS30 |
| } |  |  |  |
| periodicityAndOffset | Not present |  | APERIODIC |
| qcl-InfoPeriodicCSI-RS | Not present |  |  |
| } |  |  |  |

Table H.3A.6-3c: NZP-CSI-RS-ResourceSetId-IMR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSetId | 1 |  |  |

Table H.3A.6-4: CSI-ResourceConfig for CSI-IM as IMR

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-12B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-IM-ResourceConfigId-IMR |  |  |
|  | CSI-IM-ResourceConfigId-IMR with condition APERIODIC |  | APERIODIC |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| csi-IM-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig) OF { | 1 entry |  |  |
| CSI-IM-ResourceSetId[1] | CSI-IM-ResourceSetId-IMR |  |  |
|  | CSI-IM-ResourceSetId-IMR with condition APERIODIC |  | APERIODIC |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id of active BWP |  |  |
| resourceType | aperiodic |  | APERIODIC |
|  | periodic |  | PERIODIC |
| } |  |  |  |

Table H.3A.6-4a: CSI-IM-ResourceSet for IMR

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSet ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceSetId | CSI-IM-ResourceSetId-IMR |  |  |
|  | CSI-IM-ResourceSetId-IMR with condition APERIODIC |  | APERIODIC |
| csi-IM-Resources SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourcesPerSet)) OF CSI-IM-ResourceId { | 2 entries |  |  |
| CSI-IM-ResourceId[1] | CSI-IM-ResourceId for IMR(0) | INTEGER 0 |  |
|  | CSI-IM-ResourceId for IMR(0) with condition APERIODIC | INTEGER 10 | APERIODIC |
| CSI-IM-ResourceId[2] | CSI-IM-ResourceId for IMR(1) | INTEGER 1 |  |
|  | CSI-IM-ResourceId for IMR(1) with condition APERIODIC | INTEGER 11 | APERIODIC |
| } |  |  |  |
| } |  |  |  |

Table H.3A.6-4b: CSI-IM-Resource for IMR(Id)

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-Resource ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceId | CSI-IM-ResourceId for IMR(Id) |  |  |
|  | CSI-IM-ResourceId for IMR(Id) with condition APERIODIC |  | APERIODIC |
| csi-IM-ResourceElementPattern CHOICE { |  |  |  |
| pattern1 SEQUENCE { |  |  |  |
| subcarrierLocation-p1 | s0 |  |  |
| symbolLocation-p1 | 6 |  | Id = 0 |
|  | 10 |  | Id = 1 |
|  | 7 |  | Id = 0 AND APERIODIC |
|  | 11 |  | Id = 1 AND APERIODIC |
| } |  |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation-RRM |  | See TS 38.508-1 [14], Table 7.3.1-10 |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for CSI |  |  |
| slots20 | 1 |  | SCS15 |
| slots40 | 2 |  | SCS30 |
| } |  |  |  |
| periodicityAndOffset | Not present |  | APERIODIC |
| } |  |  |  |

Table H.3A.6-4c: CSI-IM-ResourceSetId-IMR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSetId | 0 |  |  |

## H.3.7	RRC messages and information elements contents exceptions for NR cell search when DRX is used

MAC-CellGroupConfig: DRX configuration for NR serving cell

Table H.3.7-1: MAC-CellGroupConfig: NR intra-frequency cell search when DRX is used

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms1 |  | DRX.1 OR DRX.2 OR DRX.6 |
|  | ms6 |  | DRX.3 OR DRX.7 OR DRX.8 OR DRX.11 OR DRX.14 |
| } |  |  |  |
| drx-InactivityTimer | ms1 |  |  |
| drx-RetransmissionTimerDL | sl1 |  |  |
| drx-RetransmissionTimerUL | sl1 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms20 | 0 |  | DRX.11 |
| ms40 | 0 |  | DRX.1 or DRX.3 |
|  | 10 |  | (DRX.1 and Gap) OR (DRX.1 and Offset) OR (DRX.3 and Offset) |
|  | 15 |  | DRX.1 and Gap and INTER-FREQ |
|  | 20 |  | DRX.3 and Gap |
| ms320 | 0 |  | DRX.6 OR DRX.8 |
|  | 10 |  | DRX.6 and Gap |
| ms160 | 0 |  | DRX.14 |
| ms640 | 0 |  | DRX.2 OR DRX.7 |
|  | 10 |  | (DRX.2 and Gap) OR (DRX.7 and Offset) OR (DRX.7 and Gap) |
|  | 15 |  | (DRX.2 OR DRX.7) and Gap and INTER-FREQ |
| } |  |  |  |
| shortDRX | not present |  |  |
| drx-SlotOffset | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| tag-Config SEQUENCE { |  |  |  |
| tag-ToReleaseList | Not present |  |  |
| tag-ToAddModList SEQUENCE (SIZE (1..maxNrofTAGs)) OF SEQUENCE { | 1 entry |  |  |
| tag-Id[1] | 0 |  |  |
| timeAlignmentTimer[1] | ms500 |  | DRX.1 OR DRX.2 OR DRX.6 |
|  | infinity |  | DRX.3 OR DRX.7 OR DRX.8 OR DRX.11 OR DRX.14 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DRX.1 | DRX Configuration 1 according to TS 38.133 [6] A.3.3.1 |
| DRX.2 | DRX Configuration 2 according to TS 38.133 [6] A.3.3.2 |
| DRX.3 | DRX Configuration 3 according to TS 38.133 [6] A.3.3.3 |
| DRX.6 | DRX Configuration 6 according to TS 38.133 [6] A.3.3.6 |
| DRX.7 | DRX Configuration 7 according to TS 38.133 [6] A.3.3.7 |
| DRX.8 | DRX Configuration 8 according to TS 38.133 [6] A.3.3.8 |
| DRX.14 | DRX Configuration 14 according to TS 38.133 [6] A.3.3.14 |
| INTER-FREQ | Configuration for inter-frequency NR measurement tests |
| Gap | Configuration for tests with measurement gap |
| Offset | Tests that requires offset for DRX to eliminate overlap of DRX and SMTC |

MAC-MainConfig: DRX configuration for E-UTRAN serving cell

Table H.3.7-2: MAC-MainConfig

| Derivation Path: 36.508, Table 4.8.2.1.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-MainConfig ::= SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| onDurationTimer | psf2 |  | DRX.4 OR DRX.9 |
|  | psf6 |  | DRX.5 OR DRX.10 OR DRX.12 |
| drx-InactivityTimer | psf2 |  | DRX.4 OR DRX.12 |
|  | psf1920 |  | DRX.5 OR DRX.10 |
|  | psf100 |  | DRX.9 |
| longDRX-CycleStartOffset CHOICE { |  |  |  |
| sf160 | 0 |  | DRX.4 |
| sf320 | 0 |  | DRX.5 |
|  | 10 |  | DRX.5 and Gap |
| sf40 | 0 |  | DRX.9 |
|  | 10 |  | DRX.9 and Gap |
| sf640 | 0 |  | DRX.10 OR DRX.12 |
|  | 10 |  | DRX.12 and Gap |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| timeAlignmentTimerDedicated | infinity |  | DRX.4 OR DRX.5 OR DRX.12 |
|  | sf500 |  | DRX.9 OR DRX.10 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DRX.4 | DRX Configuration 4 according to TS 38.133 [6] A.3.3.4 |
| DRX.5 | DRX Configuration 5 according to TS 38.133 [6] A.3.3.5 |
| DRX.9 | DRX Configuration 9 according to TS 38.133 [6] A.3.3.9 |
| DRX.10 | DRX Configuration 10 according to TS 38.133 [6] A.3.3.10 |
| DRX.12 | DRX Configuration 12 according to TS 38.133 [6] A.3.3.12 |

## H.3.8	RRC messages and information elements contents exceptions for NR RRC reconfiguration delay

ServingCellConfigCommonSIB: information elements content exception for RRC reconfiguration delay test cases to specific both NR uplink and supplementary uplink for SCell.

Table H.3.8-1: ServingCellConfigCommonSIB-Procedure Delay

| Derivation Path: TS 38.508-1 [14], table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB |  |  |
| uplinkConfigCommon | UplinkConfigCommonSIB | NR uplink |  |
|  | UplinkConfigCommonSIB with condition SUL_NUL |  | SUL |
| supplementaryUplink | UplinkConfigCommonSIB with condition SUL_SUL |  | SUL |
| n-TimingAdvanceOffset | Not present |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ’0100 0000’B | When carrier frequency is smaller than or equal to 3 GHz, only the 4 leftmost bits are valid; |  |
| groupPresence | Not present |  |  |
| } |  |  |  |
| ssb-PeriodicityServingCell | ms20 |  |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon |  | FR1_TDD |
| ss-PBCH-BlockPower | 0 |  |  |
| } |  |  |  |

RRCReconfiguration: information elements contents exception for RRC reconfiguration delay test cases to specific radio bearers and cell groups.

Table H.3.8-2: RRCReconfiguration-Procedure Delay

| Derivation Path: TS 38.508-1 [14], table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions SRB2 and DRB1 |  |  |
| } |  |  |  |
| secondaryCellGroup | CellGroupConfig with condition EN-DC_SCell_add | OCTET STRING (CONTAINING CellGroupConfig) |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig with conditions EN-DC | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## H.3.9	RRC messages and information elements contents exceptions for UL timing

Table H.3.9-1: PUSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF { | 2 entries |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| startSymbolAndLength | 41 | Start symbol(S)=0, Length(L)=13 |  |
| } |  |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | addressed by Msg3 PUSCH time resource allocation field of the Random Access Response acc. to TS 38.213 [22] Table 8.2-1. |  |
| startSymbolAndLength | 41 | Start symbol(S)=0, Length(L)=13 |  |
| } |  |  |  |
| } |  |  |  |

## H.3.10	RRC messages and information elements contents exceptions for Shared Spectrum Access

Table H.3.10-1: ServingCellConfigCommon for Shared Spectrum Access

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst CHOICE { |  |  |  |
| mediumBitmap | 10000000 |  | SSB.1 CCA |
|  | 10100000 |  | SSB.2 CCA |
|  | 11000000 |  | SSB.3 CCA |
|  | 11110000 |  | SSB.4 CCA |
| } |  |  |  |
| channelAccessMode-r16 CHOICE { |  |  |  |
| dynamic | NULL |  | Dynamic |
| semiStatic SEQUENCE { |  |  | SemiStatic |
| period | ms2 |  |  |
| } |  |  |  |
| } |  |  |  |
| discoveryBurstWindowLength-r16 | ms1 | DBT.1 |  |
| ssb-PositionQCL-r16 | n1 |  | SSB.1 CCA SSB.2 CCA |
|  | n2 |  | SSB.3 CCA SSB.4 CCA |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB.X CCA | SSB pattern SSB.X CCA configured as per section A.3A (X = {1,2,3,4}) |
| Dynamic | Dynamic channel access |
| SemiStatic | Semi-static channel access |

Table H.3.10-2: SSB-MTC for Shared Spectrum Access

| Derivation Path: Table 38.508-1 Table 7.3.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SSB-MTC ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sf20 | 0 |  | SMTC.1 SMTC.2 |
|  | 10 |  | SMTC.4 SMTC.5 |
| } |  |  |  |
| duration | sf1 |  | SMTC.1 SMTC.4 |
|  | sf5 |  | SMTC.2 SMTC.5 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SMTC.X | SMTC pattern SMTC.X configured as per section A.4 (X = {1,2,4,5}) |

Table H.3.10-3: MeasResultForRSSI for Shared Spectrum Access

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-78AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultForRSSI ::= SEQUENCE { |  |  |  |
| rssi-Result-r16 | measResultRSSI | RSSI-Range-r16, integer (0..76) | RSSI |
| channelOccupancy-r16 | measResultChannelOccupancy | Integer (0..100) | Channel_Occupancy |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RSSI | RSSI measurements |
| Channel_Occupancy | Channel Occupancy measurements |

## H.3.11	RRC messages and information elements contents exceptions for UL Tx switching

CellGroupConfig-R16-R17

Rel-16 1T-2T or Rel-17 2T-2T UL Tx switch configuration for PCell and 1 SCell or 2 intra-band contiguous SCells.

Table H.3.11-1: CellGroupConfig-R16-R17

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| rlf-TimersAndConstants | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig-TxSwitch | Table H.3.11-3 PCell (Cell 1) |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | 1 |  |  |
| sCellConfigCommon | ServingCellConfigCommon with condition SCell_add | TS 38.508-1 [14] Table 4.6.3-168 |  |
| sCellConfigDedicated | ServingCellConfig-TxSwitch | Table H.3.11-3 1st SCell (Cell 2) |  |
| } |  |  |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 2 entries |  | 2SCELLS |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | 1 |  |  |
| sCellConfigCommon | ServingCellConfigCommon with condition SCell_add | TS 38.508-1 [14] Table 4.6.3-168 |  |
| sCellConfigDedicated | ServingCellConfig-TxSwitch | Table H.3.11-3 1st SCell (Cell 2) |  |
| } |  |  |  |
| SCellConfig[2] SEQUENCE { |  | entry 2 |  |
| sCellIndex | 2 |  |  |
| sCellConfigCommon | ServingCellConfigCommon with condition SCell_add | TS 38.508-1 [14] Table 4.6.3-168 |  |
| sCellConfigDedicated | ServingCellConfig-TxSwitch | Table H.3.11-3 2nd SCell (Cell 3) |  |
| } |  |  |  |
| } |  |  |  |
| bap-Address-r16 | Not present |  |  |
| bh-RLC-ChannelToAddModList-r16 | Not present |  |  |
| bh-RLC-ChannelToReleaseList-r16 | Not present |  |  |
| f1c-TransferPath-r16 | Not present |  |  |
| simultaneousTCI-UpdateList1-r16 | Not present |  |  |
| simultaneousTCI-UpdateList2-r16 | Not present |  |  |
| simultaneousSpatial-UpdatedList1-r16 | Not present |  |  |
| simultaneousSpatial-UpdatedList2-r16 | Not present |  |  |
| uplinkTxSwitchingOption-r16 | switchedUL |  | pc_uplinkTxSwitching_OptionSupport_switchedUL_r16 OR pc_uplinkTxSwitching_OptionSupport_both_r16 OR pc_uplinkTxSwitching_OptionSupport_switchedUL_r17 OR pc_uplinkTxSwitching_OptionSupport_both_r17 |
|  | dualUL |  | pc_uplinkTxSwitching_OptionSupport_dualUL_r16 OR pc_uplinkTxSwitching_OptionSupport_dualUL_r17 |
| uplinkTxSwitchingPowerBoosting-r16 | Not present |  |  |
| uplinkTxSwitching-2T-Mode-r17 | Not present |  | 1T-2T |
|  | enabled |  | 2T-2T |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2SCELLS | In case two intra-band contiguous SCells are used in UL Tx switching tests |
| 1T-2T | For 1T-2T UL Tx switching tests |
| 2T-2T | For 2T-2T UL Tx switching tests |

CellGroupConfig-R18

Rel-18 UL Tx switch configuration for PCell and 2 or 3 inter-band SCells over 3 or 4 bands.

Table H.3.11-2: CellGroupConfig-R18

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  | DualTAGs |
| tag-Config SEQUENCE { |  |  |  |
| tag-ToReleaseList | Not present |  |  |
| tag-ToAddModList SEQUENCE (SIZE (1..maxNrofTAGs)) OF TAG { | 2 entries |  |  |
| TAG[1] SEQUENCE { |  | entry 1 |  |
| tag-Id | 0 |  |  |
| timeAlignmentTimer | infinity |  |  |
| } |  |  |  |
| TAG[2] SEQUENCE { |  | entry 2 |  |
| tag-Id | 1 |  |  |
| timeAlignmentTimer | infinity |  |  |
| } |  |  |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| rlf-TimersAndConstants | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig-TxSwitch | Table H.3.11-3 |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 2 entries |  | 1T1T-2T |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | 1 | 1st SCell (Cell 2) |  |
| sCellConfigCommon | ServingCellConfigCommon with condition SCell_add | TS 38.508-1 [14] Table 4.6.3-168 |  |
| sCellConfigDedicated | ServingCellConfig-TxSwitch | Table H.3.11-3 |  |
| } |  |  |  |
| } |  |  |  |
| SCellConfig[2] SEQUENCE { |  | entry 2 |  |
| sCellIndex | 2 | 2nd SCell (Cell 3) |  |
| sCellConfigCommon | ServingCellConfigCommon with condition SCell_add | TS 38.508-1 [14] Table 4.6.3-168 |  |
| sCellConfigDedicated | ServingCellConfig-TxSwitch | Table H.3.11-3 |  |
| } |  |  |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 3 entries |  | 1T1T-1T1T |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | 1 | 1st SCell (Cell 2) |  |
| sCellConfigCommon | ServingCellConfigCommon with condition SCell_add | TS 38.508-1 [14] Table 4.6.3-168 |  |
| sCellConfigDedicated | ServingCellConfig-TxSwitch | Table H.3.11-3 |  |
| } |  |  |  |
| } |  |  |  |
| SCellConfig[2] SEQUENCE { |  | entry 2 |  |
| sCellIndex | 2 | 2nd SCell (Cell 3) |  |
| sCellConfigCommon | ServingCellConfigCommon with condition SCell_add | TS 38.508-1 [14] Table 4.6.3-168 |  |
| sCellConfigDedicated | ServingCellConfig-TxSwitch | Table H.3.11-3 |  |
| } |  |  |  |
| } |  |  |  |
| SCellConfig[3] SEQUENCE { |  | entry 3 |  |
| sCellIndex | 3 | 3rd SCell (Cell 4) |  |
| sCellConfigCommon | ServingCellConfigCommon with condition SCell_add | TS 38.508-1 [14] Table 4.6.3-168 |  |
| sCellConfigDedicated | ServingCellConfig-TxSwitch | Table H.3.11-3 |  |
| } |  |  |  |
| } |  |  |  |
| ncr-FwdConfig-r18 | Not present |  |  |
| autonomousDenialParameters-r18 | Not present |  |  |
| nonCollocatedTypeMRDC-r18 | Not present |  |  |
| nonCollocatedTypeNR-CA-r18 | Not present |  |  |
| uplinkTxSwitchingMoreBands-r18 CHOICE { |  |  | 1T1T-2T |
| setup SEQUENCE{ |  |  |  |
| uplinkTxSwitchingBandList-r18 SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FreqBandIndicatorNR { | 3 entries |  |  |
| FreqBandIndicatorNR[1] | FreqBandIndicatorNR of PCell (Cell 1) | entry 1 Band A |  |
| FreqBandIndicatorNR[2] | FreqBandIndicatorNR of 1st SCell (Cell 2) | entry 2 Band B |  |
| FreqBandIndicatorNR[3] | FreqBandIndicatorNR of 2nd SCell (Cell 3) | entry 3 Band C |  |
| } |  |  |  |
| uplinkTxSwitchingBandPairList-r18 SEQUENCE (SIZE (1.. maxULTxSwitchingBandPairs)) OF UplinkTxSwitchingBandPairConfig-r18 { | 3 entries |  |  |
| UplinkTxSwitchingBandPairConfig-r18[1] SEQUENCE { |  | entry 1 |  |
| bandInfoUL1-r18 | 1 | Band A |  |
| bandInfoUL2-r18 | 3 | Band C |  |
| switchingOptionConfigForBandPair-r18 | switchedUL |  | pc_uplinkTxSwitching_OptionSupport_switchedUL_r18 OR pc_uplinkTxSwitching_OptionSupport_both_r18 |
|  | dualUL |  | pc_uplinkTxSwitching_OptionSupport_dualUL_r18 |
| switching2T-Mode-r18 | Not present |  |  |
| switchingPeriodConfigForBandPair-r18 | Not present | 210 us | pc_switchingPeriod_210us_r18 |
|  | n35us | 35 us | pc_switchingPeriod_35us_r18 |
|  | n140us | 140 us | pc_switchingPeriod_140us_r18 |
| } |  |  |  |
| UplinkTxSwitchingBandPairConfig-r18[2] SEQUENCE { |  | entry 2 |  |
| bandInfoUL1-r18 | 2 | Band B |  |
| bandInfoUL2-r18 | 3 | Band C |  |
| switchingOptionConfigForBandPair-r18 | switchedUL |  | pc_uplinkTxSwitching_OptionSupport_switchedUL_r18 OR pc_uplinkTxSwitching_OptionSupport_both_r18 |
|  | dualUL |  | pc_uplinkTxSwitching_OptionSupport_dualUL_r18 |
| switching2T-Mode-r18 | Not present |  |  |
| switchingPeriodConfigForBandPair-r18 | Not present | 210 us | pc_switchingPeriod_210us_r18 |
|  | n35us | 35 us | pc_switchingPeriod_35us_r18 |
|  | n140us | 140 us | pc_switchingPeriod_140us_r18 |
| } |  |  |  |
| UplinkTxSwitchingBandPairConfig-r18[3] SEQUENCE { |  | entry 3 |  |
| bandInfoUL1-r18 | 1 | Band A |  |
| bandInfoUL2-r18 | 2 | Band B |  |
| switchingOptionConfigForBandPair-r18 | dualUL |  |  |
| switching2T-Mode-r18 | Not present |  |  |
| switchingPeriodConfigForBandPair-r18 | Not present | 210 us | pc_switchingPeriod_210us_r18 |
|  | n35us | 35 us | pc_switchingPeriod_35us_r18 |
|  | n140us | 140 us | pc_switchingPeriod_140us_r18 |
| } |  |  |  |
| } |  |  |  |
| uplinkTxSwitchingAssociatedBandDualUL-List-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| uplinkTxSwitchingMoreBands-r18 CHOICE { |  |  | 1T1T-1T1T |
| setup SEQUENCE{ |  |  |  |
| uplinkTxSwitchingBandList-r18 SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FreqBandIndicatorNR { | 4 entries |  |  |
| FreqBandIndicatorNR[1] | FreqBandIndicatorNR of PCell (Cell 1) | entry 1 |  |
| FreqBandIndicatorNR[2] | FreqBandIndicatorNR of 1st SCell (Cell 2) | entry 2 |  |
| FreqBandIndicatorNR[3] | FreqBandIndicatorNR of 2nd SCell (Cell 3) | entry 3 |  |
| FreqBandIndicatorNR[4] | FreqBandIndicatorNR of 3rd SCell (Cell 4) | entry 4 |  |
| } |  |  |  |
| uplinkTxSwitchingBandPairList-r18 SEQUENCE (SIZE (1.. maxULTxSwitchingBandPairs)) OF UplinkTxSwitchingBandPairConfig-r18 { | 6 entries |  |  |
| UplinkTxSwitchingBandPairConfig-r18[1] SEQUENCE { |  | entry 1 |  |
| bandInfoUL1-r18 | 1 | Band A |  |
| bandInfoUL2-r18 | 3 | Band C |  |
| switchingOptionConfigForBandPair-r18 | switchedUL |  | pc_uplinkTxSwitching_OptionSupport_switchedUL_r18 OR pc_uplinkTxSwitching_OptionSupport_both_r18 |
|  | dualUL |  | pc_uplinkTxSwitching_OptionSupport_dualUL_r18 |
| switching2T-Mode-r18 | Not present |  |  |
| switchingPeriodConfigForBandPair-r18 | Not present | 210 us | pc_switchingPeriod_210us_r18 |
|  | n35us | 35 us | pc_switchingPeriod_35us_r18 |
|  | n140us | 140 us | pc_switchingPeriod_140us_r18 |
| } |  |  |  |
| UplinkTxSwitchingBandPairConfig-r18[2] SEQUENCE { |  | entry 2 |  |
| bandInfoUL1-r18 | 2 | Band B |  |
| bandInfoUL2-r18 | 3 | Band C |  |
| switchingOptionConfigForBandPair-r18 | switchedUL |  | pc_uplinkTxSwitching_OptionSupport_switchedUL_r18 OR pc_uplinkTxSwitching_OptionSupport_both_r18 |
|  | dualUL |  | pc_uplinkTxSwitching_OptionSupport_dualUL_r18 |
| switching2T-Mode-r18 | Not present |  |  |
| switchingPeriodConfigForBandPair-r18 | Not present | 210 us | pc_switchingPeriod_210us_r18 |
|  | n35us | 35 us | pc_switchingPeriod_35us_r18 |
|  | n140us | 140 us | pc_switchingPeriod_140us_r18 |
| } |  |  |  |
| UplinkTxSwitchingBandPairConfig-r18[3] SEQUENCE { |  | entry 3 |  |
| bandInfoUL1-r18 | 1 | Band A |  |
| bandInfoUL2-r18 | 2 | Band B |  |
| switchingOptionConfigForBandPair-r18 | dualUL |  |  |
| switching2T-Mode-r18 | Not present |  |  |
| switchingPeriodConfigForBandPair-r18 | Not present | 210 us | pc_switchingPeriod_210us_r18 |
|  | n35us | 35 us | pc_switchingPeriod_35us_r18 |
|  | n140us | 140 us | pc_switchingPeriod_140us_r18 |
| } |  |  |  |
| UplinkTxSwitchingBandPairConfig-r18[4] SEQUENCE { |  | entry 4 |  |
| bandInfoUL1-r18 | 1 | Band A |  |
| bandInfoUL2-r18 | 4 | Band D |  |
| switchingOptionConfigForBandPair-r18 | switchedUL |  | pc_uplinkTxSwitching_OptionSupport_switchedUL_r18 OR pc_uplinkTxSwitching_OptionSupport_both_r18 |
|  | dualUL |  | pc_uplinkTxSwitching_OptionSupport_dualUL_r18 |
| switching2T-Mode-r18 | Not present |  |  |
| switchingPeriodConfigForBandPair-r18 | Not present | 210 us | pc_switchingPeriod_210us_r18 |
|  | n35us | 35 us | pc_switchingPeriod_35us_r18 |
|  | n140us | 140 us | pc_switchingPeriod_140us_r18 |
| } |  |  |  |
| UplinkTxSwitchingBandPairConfig-r18[5] SEQUENCE { |  | entry 5 |  |
| bandInfoUL1-r18 | 2 | Band B |  |
| bandInfoUL2-r18 | 4 | Band D |  |
| switchingOptionConfigForBandPair-r18 | switchedUL |  | pc_uplinkTxSwitching_OptionSupport_switchedUL_r18 OR pc_uplinkTxSwitching_OptionSupport_both_r18 |
|  | dualUL |  | pc_uplinkTxSwitching_OptionSupport_dualUL_r18 |
| switching2T-Mode-r18 | Not present |  |  |
| switchingPeriodConfigForBandPair-r18 | Not present | 210 us | pc_switchingPeriod_210us_r18 |
|  | n35us | 35 us | pc_switchingPeriod_35us_r18 |
|  | n140us | 140 us | pc_switchingPeriod_140us_r18 |
| } |  |  |  |
| UplinkTxSwitchingBandPairConfig-r18[6] SEQUENCE { |  | entry 6 |  |
| bandInfoUL1-r18 | 3 | Band C |  |
| bandInfoUL2-r18 | 4 | Band D |  |
| switchingOptionConfigForBandPair-r18 | dualUL |  |  |
| switching2T-Mode-r18 | Not present |  |  |
| switchingPeriodConfigForBandPair-r18 | Not present | 210 us | pc_switchingPeriod_210us_r18 |
|  | n35us | 35 us | pc_switchingPeriod_35us_r18 |
|  | n140us | 140 us | pc_switchingPeriod_140us_r18 |
| } |  |  |  |
| } |  |  |  |
| uplinkTxSwitchingAssociatedBandDualUL-List-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DualTAGs | For tests in which two TAGs are configured |
| 1T1T-2T | For Band A (1T)+Band B (1T) to Band C (2T) UL Tx switching tests |
| 1T1T-1T1T | For Band A (1T)+Band B (1T) to Band C (1T)+Band D (1T) UL Tx switching tests |

ServingCellConfig-TxSwitch

To provide dedicated serving cell configurations for PCell/SCell(s) in UL Tx switching tests.

Table H.3.11-3: ServingCellConfig-TxSwitch

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 with condition DLBWP_add and ULBWP_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 1 entry |  |  |
| BWP-Uplink[1] SEQUENCE { |  | entry 1 |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| srs-Config CHOICE { |  |  |  |
| setup | SRS-Config-TxSwitch | Table H.3.11-4 Associated conditions shall be specified in test |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| enablePL-RS-UpdateForPUSCH-SRS-r16 | Not present |  |  |
| enableDefaultBeamPL-ForPUSCH0-0-r16 | Not present |  |  |
| enableDefaultBeamPL-ForPUCCH-r16 | Not present |  |  |
| enableDefaultBeamPL-ForSRS-r16 | Not present |  |  |
| uplinkTxSwitching-r16 | Not present |  | 1T1T-2T OR 1T1T-1T1T |
| uplinkTxSwitching-r16 CHOICE { |  |  | 1T-2T OR 2T-2T |
| setup SEQUENCE { |  |  |  |
| uplinkTxSwitchingPeriodLocation-r16 | true |  | PCELL |
|  | false |  |  |
| uplinkTxSwitchingCarrier-r16 | carrier1 |  | PCELL |
|  | carrier2 |  |  |
| } |  |  |  |
| } |  |  |  |
| mpr-PowerBoost-FR2-r16 | Not present |  |  |
| } |  |  |  |
| csi-MeasConfig | CSI-MeasConfig-TxSwitch | Table H.3.11-5 Associated conditions shall be specified in test |  |
| tag-Id | 0 |  |  |
|  | 1 |  | 2ND_TAG |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 1T-2T | For 1T-2T UL Tx switching |
| 2T-2T | For 2T-2T UL Tx switching |
| 1T1T-2T | For Band A (1T)+Band B (1T) to Band C (2T) UL Tx switching |
| 1T1T-1T1T | For Band A (1T)+Band B (1T) to Band C (1T)+Band D (1T) UL Tx switching |
| PCELL | Configuration for PCell in UL Tx switching tests |
| 2ND_TAG | For Cell in 2nd TAG |

SRS-Config-TxSwitch

To provide SRS configurations for PCell/SCell(s) in UL Tx switching tests.

Table H.3.11-4: SRS-Config-TxSwitch

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-182 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-ResourceSetToAddModList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet { | 1 entry |  |  |
| SRS-ResourceSet[1] SEQUENCE { |  | entry 1 |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| associatedCSI-RS | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource { | 1 entry |  |  |
| SRS-Resource[1] SEQUENCE { |  | entry 1 |  |
| nrofSRS-Ports | ports2 |  | 2TX |
|  | port1 |  |  |
| resourceMapping SEQUENCE { |  |  |  |
| nrofSymbols | n2 |  |  |
| } |  |  |  |
| freqHopping SEQUENCE { |  |  |  |
| c-SRS | 14 |  | FDD |
|  | 25 |  | TDD |
| } |  |  |  |
| groupOrSequenceHopping | neither |  |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| periodicityAndOffset-p CHOICE { |  |  |  |
| sl10 | 6 |  | FDD AND SwitchFrom |
| sl20 | 5 |  | TDD AND SwitchFrom |
| sl20 | 3 |  | TDD AND SwitchTo |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX | For Bands supporting 2Tx UL-MIMO |
| SwitchFrom | For the switch-from Cells in UL Tx switching tests |
| SwitchTo | For the switch-to Cells in UL Tx switching tests |

CSI-MeasConfig-TxSwitch

To provide aperiodic CSI-RS resource configurations on PCell/SCell(s) and aperiodic L1-RSRP reporting configurations on PCell in UL Tx switching tests.

Table H.3.11-5: CSI-MeasConfig-TxSwitch

| Derivation Path: TS 38.508-1 [14] Table 7.3.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | 1 entry |  |  |
| NZP-CSI-RS-Resource [1] | NZP-CSI-RS-Resource-TxSwitch | entry 1 Table H.3.11-6 |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet-TxSwitch | entry 1 Table H.3.11-7 |  |
| } |  |  |  |
| csi-IM-ResourceToAddModList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | 1 entry |  |  |
| CSI-ResourceConfig[1] | CSI-ResourceConfig-TxSwitch | entry 1 Table H.3.11-8 |  |
| } |  |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { | 1 entry |  |  |
| CSI-ReportConfig[1] | CSI-ReportConfig-TxSwitch(6) | entry 1 Table H.3.11-9 reportSlotOffset = 6 | FDD-TDD AND SwitchFrom |
|  | CSI-ReportConfig-TxSwitch(11) | entry 1 Table H.3.11-9 reportSlotOffset = 11 | TDD-TDD AND SwitchFrom |
|  | CSI-ReportConfig-TxSwitch(11) | entry 1 Table H.3.11-9 reportSlotOffset = 11 | FDD-TDD AND SwitchTo |
|  | CSI-ReportConfig-TxSwitch(10) | entry 1 Table H.3.11-9 reportSlotOffset = 10 | TDD-TDD AND SwitchTo |
| } |  |  |  |
| reportTriggerSize | 1 |  |  |
| aperiodicTriggerStateList CHOICE { |  |  |  |
| setup | CSI-AperiodicTriggerStateList-TxSwitch | Table H.3.11-10 |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SwitchFrom | For the switch-from Cells in UL Tx switching tests |
| SwitchTo | For the switch-to Cells in UL Tx switching tests |
| FDD-TDD | For FDD-TDD UL Tx switching tests |
| TDD-TDD | For TDD-TDD UL Tx switching tests |

NZP-CSI-RS-Resource-TxSwitch

To provide aperiodic CSI-RS resource configurations for PCell/SCell(s) in UL Tx switching tests.

Table H.3.11-6: NZP-CSI-RS-Resource-TxSwitch

| Derivation Path: TS 38.508-1[14] Table 7.3.1-7B with condition APERIODIC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | 0 |  |  |
| resourceMapping SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 000001 |  |  |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 8 |  | FDD AND ( pc_switchingPeriod_210us_r16 OR pc_switchingPeriod_210us_r17 OR pc_switchingPeriod_210us_r18) AND ( pc_uplinkTxSwitching_DL_Interruption_r16 OR pc_uplinkTxSwitching_DL_Interruption_r17 OR pc_uplinkTxSwitching_DL_Interruption_r18) |
|  | 4 |  | TDD AND ( pc_switchingPeriod_210us_r16 OR pc_switchingPeriod_210us_r17 OR pc_switchingPeriod_210us_r18) AND ( pc_uplinkTxSwitching_DL_Interruption_r16 OR pc_uplinkTxSwitching_DL_Interruption_r17 OR pc_uplinkTxSwitching_DL_Interruption_r18) |
|  | 9 |  | FDD AND ( pc_switchingPeriod_140us_r16 OR pc_switchingPeriod_140us_r17 OR pc_switchingPeriod_140us_r18) AND ( pc_uplinkTxSwitching_DL_Interruption_r16 OR pc_uplinkTxSwitching_DL_Interruption_r17 OR pc_uplinkTxSwitching_DL_Interruption_r18) |
|  | 5 |  | TDD AND ( pc_switchingPeriod_140us_r16 OR pc_switchingPeriod_140us_r17 OR pc_switchingPeriod_140us_r18) AND ( pc_uplinkTxSwitching_DL_Interruption_r16 OR pc_uplinkTxSwitching_DL_Interruption_r17 OR pc_uplinkTxSwitching_DL_Interruption_r18) |
|  | 10 |  | FDD AND ( pc_switchingPeriod_35us_r16 OR pc_switchingPeriod_35us_r17 OR pc_switchingPeriod_35us_r18) AND ( pc_uplinkTxSwitching_DL_Interruption_r16 OR pc_uplinkTxSwitching_DL_Interruption_r17 OR pc_uplinkTxSwitching_DL_Interruption_r18) |
|  | 8 |  | TDD AND ( pc_switchingPeriod_35us_r16 OR pc_switchingPeriod_35us_r17 OR pc_switchingPeriod_35us_r18) AND ( pc_uplinkTxSwitching_DL_Interruption_r16 OR pc_uplinkTxSwitching_DL_Interruption_r17 OR pc_uplinkTxSwitching_DL_Interruption_r18) |
|  | 12 |  | FDD AND NOT ( pc_uplinkTxSwitching_DL_Interruption_r16 OR pc_uplinkTxSwitching_DL_Interruption_r17 OR pc_uplinkTxSwitching_DL_Interruption_r18) |
|  | 10 |  | TDD AND NOT ( pc_uplinkTxSwitching_DL_Interruption_r16 OR pc_uplinkTxSwitching_DL_Interruption_r17 OR pc_uplinkTxSwitching_DL_Interruption_r18) |
| } |  |  |  |
| powerControlOffset | -6 |  |  |
| powerControlOffsetSS | db6 | 6dB power boost |  |
| } |  |  |  |

Table H.3.11-7: NZP-CSI-RS-ResourceSet-TxSwitch

| Derivation Path: TS 38.508-1 [14] Table 7.3.1-11B with condition APERIODIC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp-CSI-ResourceSetId | 0 |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceId[1] | 0 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

CSI-ResourceConfig-TxSwitch

To provide CSI resource configurations for PCell/SCell(s) in UL Tx switching tests.

Table H.3.11-8: CSI-ResourceConfig-TxSwitch

| Derivation Path: TS 38.508-1 [14] Table 7.3.1-12B with condition APERIODIC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | 0 |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | 0 | Entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | 1 |  |  |
| } |  |  |  |

CSI-ReportConfig-TxSwitch (slotOffset)

To provide aperiodic CSI reporting configuration on serving cell. slotoffset is the configured reportSlotOffset (in slots).

Table H.3.11-9: CSI-ReportConfig-TxSwitch (slotOffset)

| Derivation Path: TS 38.508[14] Table 7.3.1-12G with condition APERIODIC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | 0 |  |  |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF { | 1 entry |  |  |
| INTEGER[1] | slotOffset |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

CSI-AperiodicTriggerStateList-TxSwitch

To provide aperiodic CSI reporting triggering state configuration on serving cell.

Table H.3.11-10: CSI-AperiodicTriggerStateList-TxSwitch

| Derivation Path: 38.508-1 [14] Table 7.3.1-12J |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-AperiodicTriggerStateList ::= SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF CSI-AperiodicTriggerState { | 1 entry |  |  |
| CSI-AperiodicTriggerState[1] SEQUENCE { |  | entry 1 |  |
| associatedReportConfigInfoList SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo { | 1 entry |  |  |
| CSI-AssociatedReportConfigInfo[k: k=1,..,m] SEQUENCE { |  | entry k For k-th switch-from cell |  |
| reportConfigId | 0 |  |  |
| resourcesForChannel CHOICE { |  |  |  |
| nzp-CSI-RS SEQUENCE { |  |  |  |
| resourceSet | 0 |  |  |
| qcl-info SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | 0 | QCL to SSB #0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

TDD-UL-DL-ConfigCommon-TxSwitch

To provide common TDD UL/DL configuration for UL Tx switch tests

Table H.3.11-11: TDD-UL-DL-ConfigCommon-TxSwitch

| Derivation Path: 38.508-1[14] Table 7.3.1-1 with condition TDDConf.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| pattern1 SEQUENCE { |  |  |  |
| nrofDownlinkSymbols | 11 |  |  |
| nrofUplinkSymbols | 2 |  |  |
| } |  |  |  |
| } |  |  |  |

## H.3.12	RRC messages and information elements contents exceptions for L1/L2-triggered mobility

RRCReconfiguration

To setup NR Measurement Configuration.

Table H.3.12-1: RRCReconfiguration: LTM Configuration

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition LTM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | Not present |  |  |
| masterCellGroup SEQUENCE { |  |  | L1-RSRP |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| csi-MeasConfig SEQUENCE { |  |  |  |
| ltm-CSI-ReportConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfig-r18 { | 1 entry |  |  |
| LTM-CSI-ReportConfig-r18[1] | LTM-CSI-ReportConfig-r18 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ltm-Config-r18 CHOICE { |  |  |  |
| setup | LTM-Config-r18 | LTM Configuration |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| L1-RSRP | Neighbour cell L1-RSRP measurement and report |

LTM-Config-r18

Configuration for LTM.

Table H.3.12-2: LTM-Config-r18: Configuration of LTM

| Derivation path: TS 38.508-1 [14], Table 4.6.3-67D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Config-r18 ::= SEQUENCE { |  |  |  |
| ltm-ReferenceConfiguration-r18 | Not present |  |  |
| ltm-CandidateToReleaseList-r18 | Not present |  |  |
| ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 { | 1 entry |  |  |
| LTM-Candidate-r18[1] | LTM-Candidate-r18 | entry 1 |  |
| } |  |  |  |
| ltm-ServingCellNoResetID-r18 | Not present |  |  |
| ltm-CSI-ResourceConfig-r18 | Not present |  |  |
| ltm-CSI-ResourceConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18 { | 1 entry |  | L1-RSRP |
| LTM-CSI-ResourceConfig-r18[1] | LTM-CSI-ResourceConfig-r18 | entry 1 |  |
| } |  |  |  |
| ltm-CSI-ResourceConfigToReleaseList-r18 | Not present |  |  |
| attemptLTM-Switch-r18 | Not present |  |  |
|  | true |  | Fast Recovery |
| ltm-ServingCellUE-MeasuredTA-ID-r18 | Not present |  | TA_UE-BASED |
|  | 1 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Fast Recovery | LTM fast recovery |
| TA_UE-BASED | Early UL synchronization – UE based TA measurement |
| L1-RSRP | Neighbour cell L1-RSRP measurement and report |

LTM-Candidate-r18

Configuration for LTM candidate.

Table H.3.12-3: LTM-Candidate-r18: Configuration for LTM candidate

| Derivation path: TS 38.508-1 [14], Table 4.6.3-67C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Candidate-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateId-r18 | LTM-CandidateId-r18 |  |  |
| ltm-CandidatePCI-r18 | PhysCellId |  |  |
| ltm-SSB-Config-r18 | LTM-SSB-Config-r18 |  |  |
| ltm-CandidateConfig-r18 | Not present |  |  |
| ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) { |  |  | Candidate Config |
| RRCReconfiguration SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig specified in TS 38.508-1 [14] Table 4.6.3-19 with condition PCell_change |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ltm-ConfigComplete-r18 | true |  |  |
| ltm-EarlyUL-SyncConfig-r18 | Not present |  |  |
|  | EarlyUL-SyncConfig-r18 |  | TA_PDCCH-ORDERED |
| ltm-EarlyUL-SyncConfigSUL-r18 | Not present |  |  |
| ltm-TCI-Info-r18 | Not present |  |  |
|  | LTM-TCI-Info-r18 |  | TCI_INFO |
| ltm-NoResetID-r18 | Not present |  |  |
| ltm-UE-MeasuredTA-ID-r18 | Not present |  |  |
|  | 1 |  | TA_UE-BASED |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Candidate Config | LTM candidate configuration |
| TA_PDCCH-ORDERED | Early UL synchronization – PDCCH-order RACH |
| TCI_INFO | LTM TCI State List |
| TA_UE-BASED | LTM UE based TA measurement |

LTM-CSI-ResourceConfig-r18

Configuration for LTM CSI resource.

Table H.3.12-4: LTM-CSI-ResourceConfig-r18: Configuration for LTM CSI resource

| Derivation path: TS 38.508-1 [14], Table 4.6.3-67G |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-CSI-ResourceConfig-r18 ::= SEQUENCE { |  |  |  |
| ltm-CSI-ResourceConfigId-r18 | LTM-CSI-ResourceConfigId-r18 |  |  |
| ltm-CSI-SSB-ResourceSet-r18 | LTM-CSI-SSB-ResourceSet-r18 |  |  |
| } |  |  |  |

LTM-CSI-ReportConfig-r18

Configuration for LTM CSI report.

Table H.3.12-5: LTM-CSI-ReportConfig-r18: Configuration for LTM CSI report

| Derivation path: TS 38.508-1 [14], Table 4.6.3-67E |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-CSI-ReportConfig-r18 ::= SEQUENCE { |  |  |  |
| ltm-CSI-ReportConfigId-r18 | LTM-CSI-ReportConfigId |  |  |
| ltm-ResourcesForChannelMeasurement-r18 | LTM-CSI-ResourceConfigId |  |  |
| ltm-ReportConfigType-r18 CHOICE { |  |  |  |
| periodic-r18 SEQUENCE { |  |  |  |
| reportSlotConfig-r18 CHOICE { |  |  |  |
| slots80 | 2 | Periodicity 80 slots and offset 2 | SCS15 |
|  | 7 | Periodicity 80 slots and offset 7 | SCS30 |
| slots320 | 4 | Periodicity 320 slots and offset 4 | SCS120 |
|  | 9 | Periodicity 320 slots and offset 9 | SCS240 |
| pucch-CSI-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource { | 1 entry |  |  |
| uplinkBandwidthPartId | BWP-Id of active BWP |  |  |
| pucch-Resource | pucch-ResourceId | PUCCH-format2 as configured in TS 38.508-1 [14], Table 4.6.3-112 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ltm-ReportContent-r18 | LTM-ReportContent-r18 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS15 | Configuration with 15 kHz SSB SCS |
| SCS30 | Configuration with 30 kHz SSB SCS |
| SCS120 | Configuration with 120 kHz SSB SCS |
| SCS240 | Configuration with 240 kHz SSB SCS |

LTM-CSI-SSB-ResourceSet-r18

Configuration for LTM CSI SSB resource set.

Table H.3.12-6: LTM-CSI-SSB-ResourceSet-r18: Configuration for LTM CSI SSB resource set

| Derivation path: TS 38.508-1 [14], Table 4.6.3-67HA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-CSI-SSB-ResourceSet-r18 ::= SEQUENCE { |  |  |  |
| ltm-CSI-SSB-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index { | 1 entry |  |  |
| SSB-Index[1] | SSB-Index | entry 1 |  |
| } |  |  |  |
| ltm-CandidateIdList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18 { | 1 entry |  |  |
| LTM-CandidateId-r18[1] | LTM-CandidateId-r18 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

LTM-QCL-Info-r18

Configuration for LTM QCL information.

Table H.3.12-7: LTM-QCL-Info-r18: Configuration for LTM QCL information

| Derivation path: TS 38.508-1 [14], Table 4.6.3-67HB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-QCL-Info-r18 ::= SEQUENCE { |  |  |  |
| referenceSignal-r18 CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  | SSB |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) specified in TS 38.508-1 [14], table 7.3.1-11 |  | TRS |
| } |  |  |  |
| qcl-Type-r18 | typeA |  | Type A |
|  | typeD |  | Type D |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB | Configuration for SSB based LTM QCL |
| TRS | Configuration for TRS based LTM QCL |
| Type A | Indicates qcl-type1 of typeA to both FR1 and FR2 test cases |
| Type D | Indicates qcl-type2 of typeD to FR2 test cases only |

LTM-ReportContent-r18

Configuration for LTM report content.

Table H.3.12-8: LTM-ReportContent-r18: Configuration for LTM report content

| Derivation path: TS 38.508-1 [14], Table 4.6.3-67HC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-ReportContent-r18 ::= SEQUENCE { |  |  |  |
| nrOfReportedCells-r18 | n1 |  |  |
| nrOfReportedRS-PerCell-r18 | n1 |  |  |
| spCellInclusion-r18 | Not present |  |  |
|  | true |  | Include SpCell |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Include SpCell | L1-RSRP measurement report associated to current SpCell |

LTM-SSB-Config-r18

Configuration for LTM SSB.

Table H.3.12-9: LTM-SSB-Config-r18: Configuration for LTM SSB

| Derivation path: TS 38.508-1 [14], Table 4.6.3-67HD |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-SSB-Config-r18 ::= SEQUENCE { |  |  |  |
| ssb-Frequency-r18 | ARFCN-ValueNR with condition DL_SSB |  |  |
| subcarrierSpacing-r18 | SubcarrierSpacing |  |  |
| ssb-Periodicity-r18 | ms20 |  |  |
| ssb-PositionsInBurst-r18 CHOICE { |  |  |  |
| shortBitmap | 0100 |  | FR1 AND SSB#1 AND (1.88GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=1.88GHZ) |
|  | 1000 |  | FR1 AND SSB#0 AND (1.88GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=1.88GHz) |
| mediumBitmap | 01000000 |  | FR1 AND SSB#1 AND (1.88GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
|  | 10000000 |  | FR1 AND SSB#0 AND (1.88GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
| longBitmap | 0100000000000000000000000000000000000000000000000000000000000000 |  | FR2 AND SSB#1 |
|  | 1000000000000000000000000000000000000000000000000000000000000000 |  | FR2 AND SSB#0 |
| } |  |  |  |
| ss-PBCH-BlockPower-r18 | 0 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FREQ<=1.88GHz | Frequency range <= 1.88 GHz |
| 1.88GHz<FREQ<=3GHz | Frequency range > 1.88 GHz and <= 3 GHz |
| FREQ>3GHz | Frequency range > 3 GHz |
| SSB#N | Cell configured with SSB-Index set to N as defined in TS 38.508-1 [14] Table 4.4.2-2 |

LTM-TCI-Info-r18

Configuration for LTM TCI information.

Table H.3.12-10: LTM-TCI-Info-r18: Configuration for LTM TCI information

| Derivation path: TS 38.508-1 [14], Table 4.6.3-67I |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-TCI-Info-r18 ::= SEQUENCE { |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF CandidateTCI-State-r18 { | n entries | The number of entries depends on the condition |  |
| CandidateTCI-State-r18[1] | TCI-StateId-LTM(2) |  |  |
| CandidateTCI-State-r18[2] | TCI-StateId-LTM(0) |  | Joint AND Early DL Sync |
|  | TCI-StateId-LTM(1) |  | Joint |
|  | TCI-StateId-LTM(3) |  | Separate |
| CandidateTCI-State-r18[3] | TCI-StateId-LTM(1) |  | Joint AND Early DL Sync |
| } |  |  |  |
| ltm-DL-OrJointTCI-StateToReleaseList-r18 | Not present |  |  |
| ltm-UL-TCI-StateToAddModList-r18 | Not present |  |  |
| ltm-UL-TCI-StateToAddModList-r18 SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF CandidateTCI-UL-State-r18 { | 1 entry |  | Separate |
| CandidateTCI-UL-State-r18[1] | CandidateTCI-UL-State-r18 | entry 1 |  |
| } |  |  |  |
| ltm-UL-TCI-StateToReleaseList-r18 | Not present |  |  |
| ltm-NZP-CSI-RS-ResourceToAddModList-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | n entries | n=4 |  |
| NZP-CSI-RS-Resource[k, k=1..n] | NZP-CSI-RS-Resource for TRS (k) specified in TS 38.508-1 [14] Table 7.3.1-7 with qcl-InfoPeriodicCSI-RS = TCI-StateId-LTM(2) | entry [k, k=1..n] |  |
| } |  |  |  |
| ltm-NZP-CSI-RS-ResourceToReleaseList-r18 | Not present |  |  |
| ltm-NZP-CSI-RS-ResourceSetToAddModList-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet for TRS specified in TS 38.508-1 [14] Table 7.3.1-11 with NZP-CSI-RS-ResourceSetId-TRS = 0 | entry 1 |  |
| } |  |  |  |
| ltm-NZP-CSI-RS-ResourceSetToReleaseList-r18 | Not present |  |  |
| pathlossReferenceRS-ToAddModList-r18 SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-r17 { | 1 entry |  |  |
| PathlossReferenceRS-r17[1] | PathlossReferenceRS-r17 | entry 1 |  |
| } |  |  |  |
| pathlossReferenceRS-ToReleaseList-r18 | Not present |  |  |
| unifiedTCI-StateType-r18 | joint |  | Joint OR (Joint AND Early DL Sync) |
|  | separate |  | Separate |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Early DL Sync | For Early DL Sync tests |
| Separate | LTM candidate configuration is configured with ltm-DL-OrJointTCI-StateToAddModList for DL TCI states and ltm-UL-TCI-StatesToAddModList for UL TCI states |
| Joint | LTM candidate configuration is configured with ltm-DL-OrJointTCI-StateToAddModList for joint TCI states for UL and DL operation |

CandidateTCI-State-r18

Configuration for candidate TCI state.

Table H.3.12-11: CandidateTCI-State-r18: Configuration for candidate TCI state

| Derivation path: TS 38.508-1 [14], Table 4.6.3-15B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CandidateTCI-State-r18 ::= SEQUENCE { |  |  |  |
| tci-StateId-r18 | TCI-StateId LTM(Id) |  |  |
| qcl-Type1-r18 | LTM-QCL-Info-r18 with conditions SSB and Type A |  | Id = 0 or 2 |
|  | LTM-QCL-Info-r18 with conditions TRS and Type A |  | Id = 1 or 3 |
| qcl-Type2-r18 | LTM-QCL-Info-r18 with conditions SSB and Type D |  | FR2 AND (Id = 0 or 2) |
|  | LTM-QCL-Info-r18 with conditions TRS and Type D |  | FR2 AND (Id = 1 or 3) |
| pathlossReferenceRS-Id-r18 | Not present |  | Id = 2 or 3 |
|  | PathlossReferenceRS-Id-r17 |  | Id = 0 or 1 |
| tag-Id-ptr-r18 | Not present |  |  |
| ul-powerControl-r18 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR2 | Indicates qcl-Type2 of type D, which applies to FR2 only |
| Id | The ID number of the TCI state |

EarlyUL-SyncConfig-r18

Configuration for early UL synchronization.

Table H.3.12-12: EarlyUL-SyncConfig-r18: Configuration for early UL synchronization

| Derivation path: TS 38.508-1 [14], Table 4.6.3-56CA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EarlyUL-SyncConfig-r18 ::= SEQUENCE { |  |  |  |
| frequencyInfoUL-r18 | FrequencyInfoUL |  |  |
| rach-ConfigGeneric-r18 | RACH-ConfigGeneric specified in TS 38.508-1 [14] Table 7.3.1-14 |  |  |
| bwp-GenericParameters-r18 | BWP |  |  |
| ssb-PerRACH-Occasion-r18 | oneFourth |  |  |
| prach-RootSequenceIndex-r18 CHOICE { |  |  |  |
| l839 | Set according to TS 38.508-1 [14] table 4.4.2-2 |  |  |
| l139 | Set according to TS 38.508-1 [14] table 4.4.2-2 |  |  |
| } |  |  |  |
| ltm-PRACH-SubcarrierSpacing-r18 | SubcarrierSpacing |  |  |
| n-TimingAdvanceOffset-r18 | n25600 |  |  |
| ltm-tdd-UL-DL-ConfigurationCommon-r18 | Not present |  | FDD |
|  | TDD-UL-DL-ConfigCommon |  | TDD |
| ltm-restrictedSetConfig-r18 | unrestrictedSet |  |  |
| } |  |  |  |

PathlossReferenceRS-r17

Configuration for pathloss of reference RS.

Table H.3.12-13: PathlossReferenceRS-r17: Configuration for pathloss of reference RS

| Derivation path: TS 38.508-1 [14], Table 4.6.3-89AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PathlossReferenceRS-r17 ::= SEQUENCE { |  |  |  |
| pathlossReferenceRS-Id-r17 | PathlossReferenceRS-Id-r17 |  |  |
| referenceSignal-r17 CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| additionalPCI-r17 | Not present |  |  |
| } |  |  |  |

ReferenceConfiguration-r18

Configuration for LTM reference.

Table H.3.12-14: ReferenceConfiguration-r18: Void

TDD-UL-DL-ConfigCommon

Configuration for cell specific Uplink/Downlink TDD configuration.

Table H.3.12-15: TDD-UL-DL-ConfigCommon: Configuration for cell specific Uplink/Downlink TDD configuration

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-192 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| SubcarrierSpacing | kHz15 |  | TDDConf.1.1, TDDConf.1.2 |
|  | kHz30 |  | TDDConf.2.1, TDDConf.2.2, TDDConf.2.3 |
|  | kHz120 |  | TDDConf.3.1, TDDConf.3.2 |
| pattern1 | Refer to Table A.1.5-1 |  | TDDConf.1.1 |
|  | Refer to Table A.1.5-1 |  | TDDConf.1.2 |
|  | Refer to Table A.1.5-2 |  | TDDConf.2.1 |
|  | Refer to Table A.1.5-2 |  | TDDConf.2.2 |
|  | Refer to Table A.1.5-2 |  | TDDConf.2.3 |
|  | Refer to Table A.1.5-3 |  | TDDConf.3.1 |
|  | Refer to Table A.1.5-3 |  | TDDConf.3.2 |
| pattern2 | Refer to Table A.1.5-1 |  | TDDConf.1.1 |
|  | Refer to Table A.1.5-1 |  | TDDConf.1.2 |
|  | Refer to Table A.1.5-2 |  | TDDConf.2.1 |
|  | Refer to Table A.1.5-2 |  | TDDConf.2.3 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TDDConf.1.1 | TDD UL/DL configuration #1 for SCS=15kHz as defined in Table A.1.5-1 |
| TDDConf.1.2 | TDD UL/DL configuration #2 for SCS=15kHz as defined in Table A.1.5-1 |
| TDDConf.2.1 | TDD UL/DL configuration #1 for SCS=30kHz as defined in Table A.1.5-2 |
| TDDConf.2.2 | TDD UL/DL configuration #2 for SCS=30kHz as defined in Table A.1.5-2 |
| TDDConf.2.3 | TDD UL/DL configuration #3 for SCS=30kHz as defined in Table A.1.5-2 |
| TDDConf.3.1 | TDD UL/DL configuration #1 for SCS=120kHz as defined in Table A.1.5-3 |
| TDDConf.3.2 | TDD UL/DL configuration #2 for SCS=120kHz as defined in Table A.1.5-3 |

## H.3.13	RRC messages and information elements contents exceptions for Satellite Access

Table H.3.13-1: ServingCellConfigCommon for Handover for Satellite Access

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 | NTN-Config-r17 | Table H.3.13-2 |  |
| } |  |  |  |

Table H.3.13-2: NTN-Config for Handover for Satellite Access

| Derivation Path: TS 38.508-1 [14], clause 4.6.3-84C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NTN-Config-r17 ::= SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ntn-UlSyncValidityDuration-r17 | s900 |  | GSO |
|  | s5 |  | NGSO |
|  | s10 |  | NGSO_T_CHO |
|  | s20 |  | NGSO_D_CHO |
| cellSpecificKoffset-r17 | 239 |  | GSO |
|  | 4 |  | NGSO or NGSO_T_CHO or NGSO_D_CHO |
| kmac-r17 | 0 |  |  |
| ta-Info-r17 SEQUENCE { |  |  |  |
| ta-Common-r17 | 0 |  |  |
| ta-CommonDrift-r17 | 0 |  |  |
| ta-CommonDriftVariant-r17 | 0 |  |  |
| } |  |  |  |
| ntn-PolarizationDL-r17 | linear |  |  |
| ntn-PolarizationUL-r17 | linear |  |  |
| ephemerisInfo-r17 | EphemerisInfo | Ephemeris info in TS 38.508-1 [14] section 7.6.2 |  |
| ta-Report-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |
| NGSO_T_CHO | Time-based Conditional Handover for NGSO scenario |
| NGSO_D_CHO | Distance-based Conditional Handover for NGSO scenario |

Annex I (normative):
RRM OTA procedures

Annex I defines the RRM OTA procedures for the permitted testing methodologies defined in [32].

# I.0	Test applicability per permitted test method

The applicability of each permitted test method for the different RRM Angle of Arrival (AoA) setups as defined in Clause A.9 is defined in Table I.0-1.

Table I.0-1: AoA Test Setup applicability per permitted test method

| AoA Test Setup | D > 5cm or No declaration | D ≤ 5cm |
| --- | --- | --- |
| Setup 1 | IFF, Enhanced IFF | DFF, IFF, Enhanced IFF, IFF+DFF |
| Setup 2a | IFF, Enhanced IFF | DFF, IFF, Enhanced IFF, IFF+DFF |
| Setup 2b | IFF, Enhanced IFF | DFF, IFF, Enhanced IFF, IFF+DFF |
| Setup 3 | Enhanced IFF | DFF, Enhanced IFF, IFF+DFF |
| Setup 4a | Enhanced IFF | DFF, Enhanced IFF, IFF+DFF |
| Setup 4b | Enhanced IFF | DFF, Enhanced IFF, IFF+DFF |
| NOTE1: D =The diameter of the smallest sphere that encloses the radiating parts of the phase coherent array antenna(s) active at any one time during the test., declared by UE vendor as per Table 4.3.9-9 in TS 38.508-2 NOTE2: DFF indicates both DFF and DFF simplification in TR 38.810. NOTE3: For DFF and DFF part of IFF+DFF, minimum range length needs to meet the requirement as specified in TS.38.508-1 Annex B.2.2-4 with the declared D |  |  |

# I.1	Direct far field (DFF)

## I.1.1	RX beam peak direction search

The Rx beam peak direction search can be found using any of the following options:

-	Same measurement procedure as in clause Annex K.1.2 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

## I.1.2	Search for directions corresponding to the EIS spherical coverage percentile

The test directions corresponding to the EIS spherical coverage percentile can be found using any of the following options:

-	EIS spherical coverage scan procedure as described in Annex K.1.6 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

# I.2	Direct far field (DFF) simplification

## I.2.1	RX beam peak direction search

The Rx beam peak direction search can be found using any of the following options:

-	Same measurement procedure as in clause Annex K.2.2 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

## I.2.2	Search for directions corresponding to the EIS spherical coverage percentile

The test directions corresponding to the EIS spherical coverage percentile can be found using any of the following options:

-	EIS spherical coverage scan procedure as described in Annex K.1.6 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

# I.3	Indirect far field (IFF)

## I.3.1	RX beam peak direction search

The Rx beam peak direction search can be found using any of the following options:

-	Same measurement procedure as in clause Annex K.3.2 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

## I.3.2	Search for directions corresponding to the EIS spherical coverage percentile

The test directions corresponding to the EIS spherical coverage percentile can be found using any of the following options:

-	EIS spherical coverage scan procedure as described in Annex K.1.6 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

# I.3A	Enhanced indirect far field (Enhanced IFF)

## I.3A.1	RX beam peak direction search

Same as in clause I.3.1 for IFF.

## I.3A.2	Search for directions corresponding to the EIS spherical coverage percentile

Same as in clause I.3.2 for IFF.

# I.4	Procedures to search test directions for RRM FR2

## I.4.1	RSRPB-based scan with fallback option to Rx beam peak direction search

This clause provides RSRPB-based scan to find the Rx beam peak of the UE for Setup 1, single AoA cases and one of the test directions for Setup 4. It also provides a method to find test directions fulfilling the EIS Spherical Coverage (TS 38.521-2 [18] clause 7.3.4.3) for Setup 2, 3 and 4. Other approaches are not precluded. RSRPB-based peak beam scan greatly reduces the time to find the peak beam position compared to traditional EIS scan using either constant step size or constant density. The procedure can be used with DFF as well as IFF methodology. The procedure is achieved implementing the following steps:

1.	Enable periodic RSRPB reporting from the UE

2.	Set of grid points for the UE scan can be user defined set or entire sphere.

3.	Set the DL SSB_RP (at the centre of the QZ) to -95 dBm / SCS. For each grid point, record RSRPB first by connecting SS to the DUT through the measurement antenna with PolLink=  polarization to form the Rx beam towards the measurement antenna.

4.	Wait for BEAM_SELECT_WAIT_TIME before recording the RSRPB reports.

4a	For each of the grid points, calculate the RSRPB result for PolLink= as the linear sum of the 2 RSRPB reports (one per receiver branch).

4b	Repeat Steps 3, 4 and 4a with PolLink= .

5.	Once the grid points scan is completed, sort the grid points based on the linear sum of the RSRPB results for PolLink= and PolLink= .

6.	For the top 10 grid points, run the REFSENS throughput test as per the test condition defined in TS 38.521-2 [18] clause 7.3.2.

7.	Select the grid point with the best REFSENS result among the 10 points from Step 6 as the Rx beam peak for RRM test cases using AoA Setup 1 or 4.

8.	Use the EIS result for the RRM beam peak to offset the RSRPB results to obtain an estimate of the EIS Spherical coverage (SC) map.

9.	From the EIS map in Step 8, select all the points meeting the EIS SC criteria (as defined in TS 38.521-2 [18] Clause 7.3.4.3) as potential test directions for AoA Setup 2a, 2b, 3 and 4. In order to confirm that the grid point is suitable, run the REFSENS throughput test as per the test condition defined in TS 38.521-2 [18] clause 7.3.2 and verify that the EIS SC criteria is met.

If not enough grid points are found using this method, fallback to using the method in Annex I.1, I.2, I.3 or I.3A as applicable.

# I.5	Re-Positioning Approach for RRM FR2

This clause provides guidance on the applicability of the re-positioning approach which was originally introduced to reduce measurement uncertainties due to positioner blocking as described in Annex N.3 of [18]. Illustrations of the applicability in the following clauses are based on example 1-AoA and 2-AoA capable RRM test systems shown in Figure I.5-1. The probes shown in blue and red are shown for simplicity as typical microwave probes antennas; any of these probes could be implemented as probe&reflector combinations to yield applicable IFF, Enhanced IFF, IFF+DFF test setups, see Clause I.0. The number of locations of the 2 AoA probes are chosen arbitrarily, i.e., systems with fewer and different probe locations are applicable.

Figure I.5-1: Example combined-axes RRM system setup to support 1 AoA (top) and 1 AoA & 2 AoA test cases (bottom)

The re-positioning approach is applicable to RRM test cases if, during the test case, the pedestal is not obstructing the beam path and it is not in closer proximity to the measurement antenna/reflector than the DUT [18], i.e., for the selected DUT alignment options and orientations only the directions of test (AoA) in the upper hemisphere away from the positioner are tested.

## I.5.1	1 AoA Test Cases

The 1 AoA test cases follow Setups 1 and 2, see Clauses A.9.1 and A.9.2, respectively. Here, the re-positioning approach is applicable only when the pedestal is not obstructing the beam path and that the pedestal is not in closer proximity to the measurement antenna/reflector than the DUT. Therefore, for the selected DUT alignment option and orientation, only the upper hemisphere away from the positioner shall be tested for all the test iterations (if applicable), i.e., the positioner shall always be in the back hemisphere of the 1 AoA probe for each test iteration.

This requirement is illustrated next using the example 1 AoA RRM systems from Figure I.5-1. For the example systems in Figure I.5-1 with P0 (shown in blue) used as active probe, the directions of testing (AoA) within the yellow highlighted upper hemisphere shall be considered so the pedestal is not obstructing the beam path. In this case, the re-positioning approach applies and the optimized QoQZ MU is applicable to the assessed MU of the test system. The different test conditions and orientations from Clause N.2 of [18] can be used for each test iteration to prevent the pedestal from obstructing the beam path.

When the directions of testing (AoA) are outside the yellow highlighted upper hemisphere, i.e. the pedestal is obstructing the beam path, the re-positioning approach no longer applies. In this case, the calculated MU for QoQZ shall include the full range of rotation angles described in the QoQZ procedure, Annex O of [18].

Figure I.5.1-1: Ranges of possible directions of testing (AoA) for 1 AoA TCs that are applicable with re-positioning concept

## I.5.2	2 AoA Test Cases

The 2 AoA test cases follow Setups 3 and 4, see Clauses A.9.3 and A.9.4, respectively. Here, the re-positioning approach is applicable only when the pedestal is not obstructing the beam paths between each of the 2 active AoA probes and the pedestal is not in closer proximity to the measurement antennas/reflectors than the DUT. Therefore, for the selected DUT alignment option and orientation, the positioner shall always be in the back hemispheres of each of the active AoA probe for each test iteration.

This requirement is illustrated next using the example 2 AoA RRM system from Figure I.5-1. Figure I.5.2-1 shows the acceptable directions of test for the re-positioner concept separately for each active probe. The directions of testing (AoAs) within the respective yellow highlighted upper hemisphere shall be considered so the pedestal is not obstructing any of the beam paths. In this case, the re-positioning approach applies and the optimized QoQZ MU is applicable to the assessed MU of the test system. The different test conditions and orientations from Clause N.2 of TS 38.521-2 [18] can be used for each test iteration to prevent the pedestal from obstructing the beam paths.

When the directions of testing (AoAs) are outside the yellow highlighted upper hemisphere for at least one active probe, i.e., the pedestal is obstructing at least one beam path, the re-positioning approach no longer applies. In this case, the calculated MU for QoQZ shall include the full range of rotation angles described in the QoQZ procedure, Annex O of [18].

Figure I.5.2-1: Ranges of possible directions of testing (AoAs) for each active probe

Annex J (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-08 | RAN5#76 | R5-175205 | - | - | - | Introduction of TS 38.533 | 0.0.1 |
| 2018-08 | RAN5#80 | R5-184115 | - | - | - | Added specification structure including headers up to third level | 0.0.2 |
| 2018-11 | RAN5#81 | R5-186706 | - | - | - | Added references and common sections | 0.1.0 |
| 2018-11 | RAN5#81 | R5-187847 | - | - | - | Added RMCs, OCGN, SMTC and SSB configurations to Annex A | 0.1.0 |
| 2018-11 | RAN5#81 | R5-187996 | - | - | - | Added test cases 6.7.1.1.1 to 6.7.1.2.2 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-187997 | - | - | - | Added test cases 4.6.2.1 to 4.6.2.8 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-187998 | - | - | - | Added test cases 5.6.2.1 to 5.6.2.4 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-187999 | - | - | - | Added test cases 6.6.2.1 to 6.6.2.8 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-188000 | - | - | - | Added test cases 7.6.2.1 to 7.6.2.4 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-188001 | - | - | - | Added test case 4.4.1.1 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-188002 | - | - | - | Added test cases 4.7.1.1.1 to 4.7.1.2.2 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-188005 | - | - | - | Added Annexes B to H | 0.1.0 |
| 2018-11 | RAN5#81 | R5-188011 | - | - | - | Added test case 4.4.3.1 | 0.1.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190448 | - | - | - | Updating FR1 MU for timing measurements | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190476 | - | - | - | Addition of band group power offsets | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190477 | - | - | - | Update of the annexes | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190478 | - | - | - | Changes to 4.7.1.x tests | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190479 | - | - | - | Addition of 4.7.2.x tests | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190480 | - | - | - | Addition of 4.7.4.x tests | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190481 | - | - | - | Changes to 6.7.1.x tests | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190482 | - | - | - | Addition of 6.7.2.x tests | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190483 | - | - | - | Addition of 6.7.4.x tests | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190512 | - | - | - | Addition of EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM-RS in non-DRX mode test case 4.5.1.5 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190513 | - | - | - | Addition of EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM-RS in non-DRX mode test case 4.5.1.6 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190514 | - | - | - | Addition of EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM-RS in DRX mode test case 4.5.1.7 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190515 | - | - | - | Addition of EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM-RS in DRX mode test case 4.5.1.8 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190516 | - | - | - | Addition of EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC test case 4.5.2.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190517 | - | - | - | Addition of EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC test case 4.5.2.2 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190518 | - | - | - | Addition of EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC test case 4.5.2.3 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190519 | - | - | - | Addition of EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC test case 4.5.2.4 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190520 | - | - | - | Addition of EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC test case 4.5.2.5 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190582 | - | - | - | Annex F correction | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190803 | - | - | - | Update Annex G in TS 38.533 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190874 | - | - | - | Addition of NR test case 6.6.1.1-reporting without gap non-DRX | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190875 | - | - | - | Addition of NR test case 6.6.1.2-reporting without gap DRX | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190876 | - | - | - | Addition of NR test case 6.6.1.3-with gap non DRX | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190877 | - | - | - | Addition of NR test case 6.6.1.4-with gap DRX | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190878 | - | - | - | Addition of NR test case 6.6.1.5-without gap non DRX SBI reading | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190879 | - | - | - | Addition of NR test case 6.6.1.6-with gap non DRX SBI reading | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190880 | - | - | - | CR to 38.533 annex for event triggered reporting test cases | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190881 | - | - | - | Addition of EN-DC FR1 event triggered reporting test case 4.6.1.5 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190882 | - | - | - | Addition of EN-DC FR1 event triggered reporting test case 4.6.1.6 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190883 | - | - | - | Addition of EN-DC FR1 event triggered reporting test case 4.6.1.3 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190884 | - | - | - | Addition of EN-DC FR1 event triggered reporting test case 4.6.1.4 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190885 | - | - | - | Addition of default config for event triggered test cases | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190886 | - | - | - | Introduction of 5G RRM TC 4.5.3.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190887 | - | - | - | Introduction of 5G RRM TC 4.5.3.2 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190888 | - | - | - | Introduction of 5G RRM TC 4.5.3.3 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190889 | - | - | - | Introduction of 5G RRM TC 4.6.1.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190890 | - | - | - | Introduction of 5G RRM TC 4.6.1.2 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190891 | - | - | - | Introduction of 5G RRM TC 5.5.3.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190892 | - | - | - | Introduction of 5G RRM TC 6.6.3.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190893 | - | - | - | pCR for Addition of TC 6.5.1.3 NR SA FR1 RLM OOS in DRX | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190894 | - | - | - | pCR for Addition of TC 6.4.3.1 NR SA FR1 TAA Accuracy | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190895 | - | - | - | pCR for Modification of TC 4.4.3.1 EN-DC FR1 TAA accuracy | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190896 | - | - | - | pCR for Addition of TC 6.5.1.1 NR SA FR1 RLM OOS | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190897 | - | - | - | pCR for Addition of TC 4.5.1.1 EN-DC FR1 RLM OOS | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190898 | - | - | - | pCR for Addition of TC 4.5.1.3 EN-DC FR1 RLM OOS in DRX | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190899 | - | - | - | Update to EN-DC FR1 transmit timing accuracy test | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190900 | - | - | - | Addition of EN-DC FR1 RLM IS non-DRX test with SSB-based RLM-RS | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190901 | - | - | - | Addition of EN-DC FR1 RLM IS DRX test with SSB-based RLM-RS | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190902 | - | - | - | Addition of NR SA FR1 RLM IS non-DRX test with SSB-based RLM-RS | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190903 | - | - | - | Addition of NR SA FR1 RLM IS DRX test with SSB-based RLM-RS | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190904 | - | - | - | Correction of RRM 5G Test Cases 4.6.2 - EN-DC FR1-FR1 Inter-frequency measurements | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190905 | - | - | - | Correction of RRM 5G Test Cases 6.6.2 - NR SA FR1-FR1 Inter-frequency measurements | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190906 | - | - | - | Correction of RRM 5G Test Cases 7.6.2 - NR SA FR2-FR2 Inter-frequency measurements | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190907 | - | - | - | Addition of RRM Test Cases 4.5.2.6: EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190908 | - | - | - | Addition of RRM Test Cases 5.5.2.1: EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190909 | - | - | - | Addition of RRM Test Cases 5.5.2.2: EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190910 | - | - | - | Addition of RRM Test Cases 5.5.2.3: EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190911 | - | - | - | Addition of RRM Test Cases 5.5.2.4: EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190912 | - | - | - | Addition of RRM Test Cases 5.5.2.5: EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190913 | - | - | - | Addition of RRM Test Cases 5.5.2.6: EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190914 | - | - | - | Addition of RRM Test Cases 6.1.1.1: NR SA FR1 cell re-selection | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190915 | - | - | - | Addition of RRM Test Cases 6.1.1.2: NR SA FR1-FR1 cell re-selection | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190916 | - | - | - | Addition of cell re-selection to higher priority E-UTRAN test case 6.1.2.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190917 | - | - | - | Addition of cell re-selection to lower priority E-UTRAN test case 6.1.2.2 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190918 | - | - | - | Addition of SA NR to E-UTRAN handover test case 6.3.1.4 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190919 | - | - | - | Addition of SA NR to E-UTRAN handover test case 6.3.1.5 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190920 | - | - | - | Addition of NR SA FR1 UE UL carrier RRC reconfiguration delay test case 6.5.4.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190921 | - | - | - | Addition of NR SA FR1 CSI-RS based RLM out-of-sync non-DRX test case 6.5.1.5 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190922 | - | - | - | Addition of NR SA FR1 CSI-RS based RLM in-sync non-DRX test case 6.5.1.6 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190923 | - | - | - | Addition of NR SA FR1 CSI-RS based RLM out-of-sync in DRX test case 6.5.1.7 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190924 | - | - | - | Addition of NR SA FR1 CSI-RS based RLM in-sync in DRX test case 6.5.1.8 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190987 | - | - | - | 38.533 Common Section updates to clarify leverage across architecture options | 0.2.0 |
| 2019-03 | RAN5#82 | R5-191484 | - | - | - | Correction Annex G | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191485 | - | - | - | Correction NSA Options | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191486 | - | - | - | Modifications NSA FR1 SS-RSRP tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191487 | - | - | - | Modifications NSA FR1 SS-RSRQ tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191488 | - | - | - | Modifications NSA FR1 L1-RSRP tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191489 | - | - | - | Modifications SA FR1 SS-RSRP tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191490 | - | - | - | Modifications SA FR1 SS-RSRQ tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191491 | - | - | - | Modifications SA FR1 L1-RSRP tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191492 | - | - | - | Addition NSA FR1 BWP switch tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191493 | - | - | - | Addition SA FR1 BWP switch tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191494 | - | - | - | Addition NSA FR2 BWP switch tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191495 | - | - | - | Addition SA FR2 BWP switch tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191720 | - | - | - | addition of cell mapping for BFD and measurement | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191924 | - | - | - | Correction of default message contents for RRM | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191926 | - | - | - | Addition of event-triggered reporting Test Cases to Cell configuration mapping in Annex E | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191930 | - | - | - | Correction of 5G RRM Test Case 4.6.2.3 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191931 | - | - | - | Correction of 5G RRM Test Case 4.6.2.4 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191934 | - | - | - | Correction of 5G RRM Test Case 4.6.2.7 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191935 | - | - | - | Correction of 5G RRM Test Case 4.6.2.8 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191936 | - | - | - | Addition of Minimum conformance requirements 5.6.2.0 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191937 | - | - | - | Correction of 5G RRM Test Case 5.6.2.1 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191938 | - | - | - | Correction of 5G RRM Test Case 5.6.2.2 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191939 | - | - | - | Correction of 5G RRM Test Case 5.6.2.3 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191940 | - | - | - | Correction of 5G RRM Test Case 5.6.2.4 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191945 | - | - | - | Addition of Minimum conformance requirements 7.6.2.0 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191946 | - | - | - | Correction of 5G RRM Test Case 7.6.2.1 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191947 | - | - | - | Correction of 5G RRM Test Case 7.6.2.2 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191948 | - | - | - | Correction of 5G RRM Test Case 7.6.2.3 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191949 | - | - | - | Correction of 5G RRM Test Case 7.6.2.4 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191950 | - | - | - | Correction of 5G RRM Test Case 7.6.2.5 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191951 | - | - | - | Correction of 5G RRM Test Case 7.6.2.6 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191952 | - | - | - | Correction of 5G RRM Test Case 7.6.2.7 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191953 | - | - | - | Correction of 5G RRM Test Case 7.6.2.8 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192062 | - | - | - | Update of EN-DC FR1 event triggered reporting test case 4.6.1.6 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192063 | - | - | - | Update of EN-DC FR1 event triggered reporting test case 4.6.1.3 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192064 | - | - | - | Update of EN-DC FR1 event triggered reporting test case 4.6.1.4 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192221 | - | - | - | Update on RRC_Connected generic procedure within RRM tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192477 | - | - | - | Introduction of FR1 EN-DC Contention based random access Test case | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192478 | - | - | - | Introduction of FR1 EN-DC non-Contention based random access Test case | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192479 | - | - | - | Introduction of FR1 standalone Contention based random access Test case | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192480 | - | - | - | Introduction of FR1 standalone Non-contention based random access Test case | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192481 | - | - | - | Updated to 5G RRM TC 4.6.1.1 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192482 | - | - | - | Updated to 5G RRM TC 4.6.1.2 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192483 | - | - | - | Addition of NR test case 6.7.1.3.1-absolute RSRP | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192484 | - | - | - | Addition of NR test case 6.7.1.3.2-relative RSRP | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192485 | - | - | - | Addition of NR test case 6.5.5.3 FR1 CSI-RS BFD nonDRX | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192486 | - | - | - | Addition of NR test case 6.5.5.4 FR1 CSI-RS BFD DRX | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192487 | - | - | - | Addition of NR test case 7.5.5.1 FR2 SSB BFD nonDRX | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192488 | - | - | - | Addition of NR test case 7.5.5.2 FR2 SSB BFD DRX | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192489 | - | - | - | Addition of NR test case 7.5.5.3 FR2 CSI-RS BFD nonDRX | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192490 | - | - | - | Addition of NR test case 7.5.5.4 FR2 CSI-RS BFD DRX | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192492 | - | - | - | Correction of 5G RRM Test Case 4.6.2.1 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192493 | - | - | - | Correction of 5G RRM Test Case 4.6.2.2 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192494 | - | - | - | Correction of 5G RRM Test Case 4.6.2.5 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192495 | - | - | - | Correction of 5G RRM Test Case 4.6.2.6 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192496 | - | - | - | Correction of 5G RRM Test Case 6.6.2.1 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192497 | - | - | - | Correction of 5G RRM Test Case 6.6.2.2 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192498 | - | - | - | Correction of 5G RRM Test Case 6.6.2.3 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192499 | - | - | - | Correction of 5G RRM Test Case 6.6.2.4 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192500 | - | - | - | Update of EN-DC FR1 event triggered reporting test case 4.6.1.5 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192503 | - | - | - | Update to EN-DC FR1 transmit timing accuracy test | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192674 | - | - | - | Update to EN-DC FR1 RLM IS non-DRX test with SSB-based RLM-RS | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192675 | - | - | - | Update to EN-DC FR1 RLM IS DRX test with SSB-based RLM-RS | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192676 | - | - | - | Update to NR SA FR1 RLM IS non-DRX test with SSB-based RLM-RS | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192677 | - | - | - | Update to NR SA FR1 RLM IS DRX test with SSB-based RLM-RS | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192678 | - | - | - | pCR for modification of TAAA TC for EN-DC FR1 | 0.3.0 |
| 2019-03 | RAN#83 | RP-190161 | - | - | - | Presented to the RAN#83 plenary for 1-step approval | 1.0.0 |
| 2019-03 | RAN#83 | - | - | - | - | Upgraded to Rel-15 with small editorial changes | 15.0.0 |
| 2019-06 | RAN#84 | R5-193578 | 0100 | - | F | Update the test case number of TC 4.3.2.2.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193755 | 0102 | - | F | Update of Minimum conformance requirements 6.6.1.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193758 | 0104 | - | F | Update of EN-DC RLM in-sync in non-DRX test case 4.5.1.6 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193759 | 0105 | - | F | Update of EN-DC RLM out-of-sync in DRX test case 4.5.1.7 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193760 | 0106 | - | F | Update of EN-DC RLM in-sync in DRX test case 4.5.1.8 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193761 | 0107 | - | F | Addition of Minimum conformance requirements 6.3.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193762 | 0108 | - | F | Addition of NR RRM TC 6.3.2.1.1-Intra Freq RRC Re-establishment | 15.1.0 |
| 2019-06 | RAN#84 | R5-193763 | 0109 | - | F | Addition of FR1-FR1 re-establishment test case 6.3.2.1.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193764 | 0110 | - | F | Addition of FR1 RRC connection release with redirection test case 6.3.2.3.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193765 | 0111 | - | F | Addition of FR1-E-UTRA RRC connection release with redirection test case 6.3.2.3.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193766 | 0112 | - | F | Update of TC 6.5.1.5 SA FR1 CSI-RS RLM OOS non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193767 | 0113 | - | F | Update of TC 6.5.1.6 SA FR1 CSI-RS RLM IS non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193768 | 0114 | - | F | Update of TC 6.5.1.7 SA FR1 CSI-RS RLM OOS DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193769 | 0115 | - | F | Update of TC 6.5.1.8 SA FR1 CSI-RS RLM IS DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193770 | 0116 | - | F | Addition of Minimum conformance requirements 6.5.5.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193771 | 0117 | - | F | Update of TC 6.5.5.3 SA FR1 CSI-RS BFD non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193772 | 0118 | - | F | Update of TC 6.5.5.4 SA FR1 CSI-RS BFD DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193773 | 0119 | - | F | Addition of Minimum conformance requirements 7.5.5.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193774 | 0120 | - | F | Update of TC 7.5.5.1 SA FR2 SSB BFD non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193775 | 0121 | - | F | Update of TC 7.5.5.2 SA FR2 SSB BFD DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193776 | 0122 | - | F | Update of TC 7.5.5.3 SA FR2 CSI-RS BFD non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193777 | 0123 | - | F | Update of TC 7.5.5.4 SA FR2 CSI-RS BFD DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193778 | 0124 | - | F | Addition of Minimum conformance requirements 7.6.1.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193779 | 0125 | - | F | Addition of 7.6.1.1 SA FR2 RRM measurement no-gap non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193780 | 0126 | - | F | Addition of 7.6.1.2 SA FR2 RRM measurement no-gap DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193781 | 0127 | - | F | Addition of 7.6.1.3 SA FR2 RRM measurement gap non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193782 | 0128 | - | F | Addition of 7.6.1.4 SA FR2 RRM measurement gap DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193784 | 0130 | - | F | Update of FR1 cell re-selection test case 6.1.1.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193785 | 0131 | - | F | Update of FR1-FR1 cell re-selection test case 6.1.1.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193786 | 0132 | - | F | Update of FR1-EUTRA higher priority cell re-selection test case 6.1.2.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193787 | 0133 | - | F | Update of FR1-EUTRA lower priority cell re-selection test case 6.1.2.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193788 | 0134 | - | F | Update of FR1-EUTRA handover known cell test case 6.3.1.4 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193790 | 0136 | - | F | Update of 6.6.1.1 SA FR1 RRM measurement no-gap non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193791 | 0137 | - | F | Update of 6.6.1.2 SA FR1 RRM measurement no-gap DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193792 | 0138 | - | F | Update of 6.6.1.3 SA FR1 RRM measurement gap non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193793 | 0139 | - | F | Update of 6.6.1.4 SA FR1 RRM measurement gap DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193794 | 0140 | - | F | Update of FR1 event-triggered without gap with SSB index test case 6.6.1.5 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193795 | 0141 | - | F | Update of FR1 event-triggered with gap with SSB index test case 6.6.1.6 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193812 | 0145 | - | F | Addition of 6.3.2.2 minimum conformance requirements | 15.1.0 |
| 2019-06 | RAN#84 | R5-193872 | 0146 | - | F | Introduction of TC 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193949 | 0148 | - | F | Correction of PRACH Configurations | 15.1.0 |
| 2019-06 | RAN#84 | R5-194328 | 0153 | - | F | Additional of new reference used in RRM test spec | 15.1.0 |
| 2019-06 | RAN#84 | R5-194329 | 0154 | - | F | Correction of reference spec number in RRM spec | 15.1.0 |
| 2019-06 | RAN#84 | R5-194494 | 0165 | - | F | Addition missing Editor's note 4.5.2.5 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194549 | 0173 | - | F | Correction of 5G RRM Test Case 5.6.2.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194550 | 0174 | - | F | Correction of 5G RRM Test Case 5.6.2.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194551 | 0175 | - | F | Correction of 5G RRM Test Case 5.6.2.3 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194552 | 0176 | - | F | Correction of 5G RRM Test Case 5.6.2.4 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194553 | 0177 | - | F | Correction of 5G RRM Test Case 5.6.2.5 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194555 | 0179 | - | F | Correction of 5G RRM Test Case 5.6.2.7 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194556 | 0180 | - | F | Correction of 5G RRM Test Case 5.6.2.8 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194557 | 0181 | - | F | Correction of Minimum conformance requirements 6.6.2.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194562 | 0186 | - | F | Correction of Minimum conformance requirements 7.6.2.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194564 | 0188 | - | F | Correction of 5G RRM Test Case 7.6.2.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194565 | 0189 | - | F | Correction of 5G RRM Test Case 7.6.2.3 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194566 | 0190 | - | F | Correction of 5G RRM Test Case 7.6.2.4 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194567 | 0191 | - | F | Correction of 5G RRM Test Case 7.6.2.5 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194568 | 0192 | - | F | Correction of 5G RRM Test Case 7.6.2.6 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194569 | 0193 | - | F | Correction of 5G RRM Test Case 7.6.2.7 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194570 | 0194 | - | F | Correction of 5G RRM Test Case 7.6.2.8 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194571 | 0195 | - | F | Correction of Minimum conformance requirements 7.3.2.1.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194572 | 0196 | - | F | Correction of 5G RRM Test Case 7.3.2.1.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194573 | 0197 | - | F | Correction of 5G RRM Test Case 7.3.2.1.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194700 | 0208 | - | F | Clean-up in EN-DC FR1 transmit timing accuracy test | 15.1.0 |
| 2019-06 | RAN#84 | R5-194702 | 0210 | - | F | Update to EN-DC FR1 RLM IS DRX test with SSB-based RLM-RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-194703 | 0211 | - | F | Update to NR SA FR1 RLM IS non-DRX test with SSB-based RLM-RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-194704 | 0212 | - | F | Update to NR SA FR1 RLM IS DRX test with SSB-based RLM-RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-194710 | 0214 | - | F | Update to E-UTRA configuration for RRM EN-DC tests to align with core spec update | 15.1.0 |
| 2019-06 | RAN#84 | R5-194713 | 0216 | - | F | Update to EN-DC FR1 RLM IS DRX test with SSB-based RLM-RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195013 | 0156 | 1 | F | Updated to EN-DC FR1 RRM tests in clause 4.5.3 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195016 | 0198 | 1 | F | Test tolerance and measurement uncertainty in Annex F for Inter-Freq measurement test cases | 15.1.0 |
| 2019-06 | RAN#84 | R5-195017 | 0129 | 1 | F | Addition of default configuration in Annex H | 15.1.0 |
| 2019-06 | RAN#84 | R5-195018 | 0143 | 1 | F | Update of 6.3.2.2.1 random access | 15.1.0 |
| 2019-06 | RAN#84 | R5-195019 | 0144 | 1 | F | Update of 6.3.2.2.2 non-contention random access | 15.1.0 |
| 2019-06 | RAN#84 | R5-195020 | 0151 | 1 | F | Updated to EN-DC FR1 RRM tests in clause 4.6.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195021 | 0207 | 1 | F | Modification of EN-DC FR1 TAAA | 15.1.0 |
| 2019-06 | RAN#84 | R5-195024 | 0199 | 1 | F | Modification of EN-DC FR1 SSB RLM OOS in non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-195025 | 0200 | 1 | F | Modification of EN-DC FR1 SSB RLM OOS in DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-195026 | 0201 | 1 | F | Modification of NR SA FR1 TAAA | 15.1.0 |
| 2019-06 | RAN#84 | R5-195027 | 0202 | 1 | F | Modification of SA FR1 SSB RLM OOS in non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-195028 | 0203 | 1 | F | Modification of SA FR1 SSB RLM OOS in DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-195029 | 0204 | 1 | F | Modification of EN-DC FR2 TAAA | 15.1.0 |
| 2019-06 | RAN#84 | R5-195030 | 0205 | 1 | F | Modification of EN-DC FR2 SSB RLM OOS in non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-195031 | 0206 | 1 | F | Modification of EN-DC FR2 SSB RLM OOS in DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-195032 | 0219 | 1 | F | Update to EN-DC FR2 RLM IS non-DRX test with SSB-based RLM-RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195033 | 0220 | 1 | F | Update to EN-DC FR2 RLM IS DRX test with SSB-based RLM-RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195034 | 0221 | 1 | F | Addition of EN-DC FR2 Transmit Timing Accuracy tests | 15.1.0 |
| 2019-06 | RAN#84 | R5-195035 | 0209 | 1 | F | Update to EN-DC FR1 RLM IS non-DRX test with SSB-based RLM-RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195036 | 0150 | 1 | F | Correction to EN-DC FR1 UE transmit timing accuracy TC 4.4.1.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195037 | 0155 | 1 | F | Updated to cell configuration mapping table for RRM tests | 15.1.0 |
| 2019-06 | RAN#84 | R5-195038 | 0147 | 1 | F | Introduction of TC 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC | 15.1.0 |
| 2019-06 | RAN#84 | R5-195039 | 0178 | 1 | F | Correction of 5G RRM Test Case 5.6.2.6 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195040 | 0166 | 1 | F | Correction of default message content for RRM in Annex H | 15.1.0 |
| 2019-06 | RAN#84 | R5-195041 | 0217 | 1 | F | Update to NR SA FR1 RLM IS non-DRX test with SSB-based RLM-RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195042 | 0218 | 1 | F | Update to NR SA FR1 RLM IS DRX test with SSB-based RLM-RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195043 | 0215 | 1 | F | Update to EN-DC FR1 RLM IS non-DRX test with SSB-based RLM-RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195044 | 0187 | 1 | F | Correction of 5G RRM Test Case 7.6.2.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195045 | 0103 | 1 | F | Update of EN-DC RLM out-of-sync in non-DRX test case 4.5.1.5 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195099 | 0223 | - | F | RRM implementation of FR2 UL demod OTA tests using single pol Rx TE | 15.1.0 |
| 2019-06 | RAN#84 | R5-195173 | 0167 | 1 | F | Correction of Minimum conformance requirements 4.6.2.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195174 | 0168 | 1 | F | Correction of 5G RRM Test Case 4.6.2.1 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195175 | 0169 | 1 | F | Correction of 5G RRM Test Case 4.6.2.2 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195176 | 0170 | 1 | F | Correction of 5G RRM Test Case 4.6.2.3 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195177 | 0171 | 1 | F | Correction of 5G RRM Test Case 4.6.2.4 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195179 | 0172 | 1 | F | Correction of Minimum conformance requirements 5.6.2.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195182 | 0142 | 1 | F | Addition of FR1-E-UTRAN event-triggered reporting in DRX test case 6.6.3.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195185 | 0158 | 1 | F | Update of FR1 Test tolerance and uncertainties in AnnexF | 15.1.0 |
| 2019-06 | RAN#84 | R5-195186 | 0182 | 1 | F | Correction of 5G RRM Test Case 6.6.2.1 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195187 | 0183 | 1 | F | Correction of 5G RRM Test Case 6.6.2.2 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195188 | 0184 | 1 | F | Correction of 5G RRM Test Case 6.6.2.3 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195189 | 0185 | 1 | F | Correction of 5G RRM Test Case 6.6.2.4 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195445 | 0152 | 2 | F | Updated to SA FR1 RRM tests in clause 6.6.3 | 15.1.0 |
| 2019-06 | RAN#84 | - | - | - | - | Administrative release upgrade to match the release of 3GPP TS 38.508-1 and TS 38.521-1 which were upgraded at RAN#84 to Rel-16 due to Rel-16 relevant CR(s) | 16.0.0 |
| 2019-09 | RAN#85 | R5-195563 | 0228 | - | F | Addition of 5.6.1.0 minimum requirements | 16.1.0 |
| 2019-09 | RAN#85 | R5-195568 | 0233 | - | F | Update of 6.1.1.2 inter-freq cell re-selection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195569 | 0234 | - | F | Update of 6.1.2.1 inter-RAT cell re-selection to higher priority | 16.1.0 |
| 2019-09 | RAN#85 | R5-195571 | 0236 | - | F | Update of 6.3.1.4 inter-RAT handover to known cell | 16.1.0 |
| 2019-09 | RAN#85 | R5-195573 | 0238 | - | F | Update Test Tolerance of 6.3.2.1.1 intra-freq RRC re-establishment | 16.1.0 |
| 2019-09 | RAN#85 | R5-195574 | 0239 | - | F | Update Test Tolerance of 6.3.2.1.2 inter-freq RRC re-establishment | 16.1.0 |
| 2019-09 | RAN#85 | R5-195575 | 0240 | - | F | Update Test Tolerance of 6.3.2.3.1 NR RRC redirection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195576 | 0241 | - | F | Update Test Tolerance of 6.3.2.3.2 inter-RAT RRC redirection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195577 | 0242 | - | F | Update of 6.5.1.5 RLM out-of-sync non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195578 | 0243 | - | F | Update of 6.5.1.6 RLM in-sync non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195579 | 0244 | - | F | Update of 6.5.1.7 RLM out-of-sync in DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195580 | 0245 | - | F | Update of 6.5.1.8 RLM in-sync in DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195581 | 0246 | - | F | Update of 6.5.5.3 CSI-RS-based BFD non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195582 | 0247 | - | F | Update of 6.5.5.4 CSI-RS-based BFD in DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195596 | 0248 | - | F | Addition of minimum conformance requirements for FR2 EN-DC CSI-RS based RLM | 16.1.0 |
| 2019-09 | RAN#85 | R5-195601 | 0253 | - | F | Addition of minimum conformance requirements for FR2 EN-DC CSI-RS based BFD | 16.1.0 |
| 2019-09 | RAN#85 | R5-195604 | 0256 | - | F | Addition of minimum conformance requirements for SA FR2 reselection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195605 | 0257 | - | F | Addition of NR test case 7.1.1.1-intra freq reselection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195606 | 0258 | - | F | Addition of NR test case 7.1.1.2-inter freq reselection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195608 | 0260 | - | F | Addition of NR test case 8.4.2.1-without SBI non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195609 | 0261 | - | F | Addition of NR test case 8.4.2.2-without SBI DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195610 | 0262 | - | F | Addition of NR test case 8.4.2.3-with SBI non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195611 | 0263 | - | F | Addition of NR test case 8.4.2.4-with SBI DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195612 | 0264 | - | F | Addition of minimum conformance requirements for FR1 EN-DC CSI-RS based RLM | 16.1.0 |
| 2019-09 | RAN#85 | R5-195623 | 0275 | - | F | Correction of NR test case 5.5.2.1-interruption transition DRX sync | 16.1.0 |
| 2019-09 | RAN#85 | R5-195624 | 0276 | - | F | Correction of NR test case 5.5.2.2-interruption transition DRX async | 16.1.0 |
| 2019-09 | RAN#85 | R5-195625 | 0277 | - | F | Correction of NR test case 5.5.2.3-interruption NR deactivated SCell sync | 16.1.0 |
| 2019-09 | RAN#85 | R5-195626 | 0278 | - | F | Correction of NR test case 5.5.2.4-interruption NR deactivated SCell async | 16.1.0 |
| 2019-09 | RAN#85 | R5-195627 | 0279 | - | F | Correction of NR test case 5.5.2.5-interruption LTE deactivated SCell sync | 16.1.0 |
| 2019-09 | RAN#85 | R5-195628 | 0280 | - | F | Correction of NR test case 5.5.2.6-interruption LTE deactivated SCell async | 16.1.0 |
| 2019-09 | RAN#85 | R5-195629 | 0281 | - | F | Correction of NR test case 7.5.5.1-SSB BFD non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195630 | 0282 | - | F | Correction of NR test case 7.5.5.2-SSB BFD DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195631 | 0283 | - | F | Correction of NR test case 7.5.5.3-CSI-RS BFD non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195632 | 0284 | - | F | Correction of NR test case 7.5.5.4-CSI-RS BFD DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195633 | 0285 | - | F | Correction of NR test case 7.6.1.1-without gap non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195634 | 0286 | - | F | Correction of NR test case 7.6.1.2-without gap DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195636 | 0288 | - | F | Correction of NR test case 7.6.1.4-with gap DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-196140 | 0295 | - | F | Update NR operating band groups | 16.1.0 |
| 2019-09 | RAN#85 | R5-196250 | 0296 | - | F | Correction to EN-DC FR1 event-triggered reporting without gap TC 4.6.1.1 and 4.6.1.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196503 | 0301 | - | F | Updated to EN-DC FR1 RRM tests in clause 4.5.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196504 | 0302 | - | F | Updated to EN-DC FR2 RRM tests in clause 5.5.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196547 | 0304 | - | F | Update of FR1 Test tolerance and uncertainties in AnnexF | 16.1.0 |
| 2019-09 | RAN#85 | R5-196601 | 0306 | - | F | Correction of the reference for test frequencies and test mode - Chapter 4 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196602 | 0307 | - | F | Correction of the reference for test frequencies and test mode - Chapter 6 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196603 | 0308 | - | F | Editorial Corrections to section 5.5 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196659 | 0309 | - | F | Editorial corrections of Annex A | 16.1.0 |
| 2019-09 | RAN#85 | R5-196660 | 0310 | - | F | Correction of RRM Test Case 7.3.2.1.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196661 | 0311 | - | F | Correction of RRM Test Case 7.3.2.1.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196662 | 0312 | - | F | Correction of cell configuration mapping for RRM Test Cases in Annex E.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196666 | 0316 | - | F | Correction of RRM Test Case 5.6.2.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196667 | 0317 | - | F | Correction of RRM Test Case 5.6.2.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196668 | 0318 | - | F | Correction of RRM Test Case 5.6.2.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196669 | 0319 | - | F | Correction of RRM Test Case 5.6.2.4 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196670 | 0320 | - | F | Correction of RRM Test Case 5.6.2.5 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196671 | 0321 | - | F | Correction of RRM Test Case 5.6.2.6 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196672 | 0322 | - | F | Correction of RRM Test Case 5.6.2.7 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196673 | 0323 | - | F | Correction of RRM Test Case 5.6.2.8 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196674 | 0324 | - | F | Addition of minimum conformance requirements for L1-RSRP measurement RRM FR1 test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-196675 | 0325 | - | F | Addition of SSB based L1-RSRP measurement RRM test case 4.6.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196676 | 0326 | - | F | Addition of SSB based L1-RSRP measurement RRM test case 4.6.3.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196677 | 0327 | - | F | Addition of CSI-RS based L1-RSRP measurement RRM test case 4.6.3.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196679 | 0329 | - | F | Addition of minimum conformance requirements for L1-RSRP measurement RRM FR2 test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-196680 | 0330 | - | F | Addition of SSB based L1-RSRP measurement RRM test case 5.6.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196681 | 0331 | - | F | Addition of SSB based L1-RSRP measurement RRM test case 5.6.3.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196682 | 0332 | - | F | Addition of CSI-RS based L1-RSRP measurement RRM test case 5.6.3.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196735 | 0334 | - | F | Editorial to fix conversion issues with two equations | 16.1.0 |
| 2019-09 | RAN#85 | R5-196736 | 0335 | - | F | Adding Chapter 8 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196737 | 0336 | - | F | New test 4.5.7.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196738 | 0337 | - | F | New test 4.5.5.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196739 | 0338 | - | F | New test 4.5.5.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196740 | 0339 | - | F | New test 4.7.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196741 | 0340 | - | F | New test 4.7.3.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196742 | 0341 | - | F | New test 6.7.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196743 | 0342 | - | F | New test 6.7.3.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196925 | 0351 | - | F | Modification of EN-DC FR2 TAAA Section 5.4.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196943 | 0360 | - | F | Addition of CSI-RS based L1-RSRP measurement RRM test case 5.6.3.4 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197361 | 0357 | 1 | F | Modification of SA FR1 TAAA Section 6.4.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197364 | 0290 | 1 | F | Update of NR UE Tx Timing Accuracy Test | 16.1.0 |
| 2019-09 | RAN#85 | R5-197366 | 0235 | 1 | F | Update Test Tolerance of 6.1.2.2 inter-RAT cell re-selection to lower priority | 16.1.0 |
| 2019-09 | RAN#85 | R5-197396 | 0346 | 1 | F | Update NSA Event trigger reporting test case | 16.1.0 |
| 2019-09 | RAN#85 | R5-197397 | 0352 | 1 | F | Modification of EN-DC FR2 SSB-RLM OOS Section 5.5.1.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197398 | 0353 | 1 | F | Modification of EN-DC FR2 SSB-RLM OOS in DRX Section 5.5.1.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197399 | 0354 | 1 | F | Modification of Synchronous EN-DC FR2 RRC DL BWP Switch in non-DRX Section 5.5.6.2.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197400 | 0355 | 1 | F | Modification of SA FR1 Handover with unknown Target Cell Section 6.3.1.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197401 | 0356 | 1 | F | Modification of SA FR1-FR1 Handover with unknown Target Cell Section 6.3.1.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197402 | 0224 | 1 | F | Update of 4.6.1.3 event with gap non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197403 | 0225 | 1 | F | Update of 4.6.1.4 event with gap in DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197404 | 0226 | 1 | F | Update of 4.6.1.5 event without gap with ssb index | 16.1.0 |
| 2019-09 | RAN#85 | R5-197405 | 0227 | 1 | F | Update of 4.6.1.6 event with gap with ssb index | 16.1.0 |
| 2019-09 | RAN#85 | R5-197406 | 0314 | 1 | F | Correction of 5G RRM Inter Frequency measurements EN-DC test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197407 | 0229 | 1 | F | Addition of 5.6.1.1 event without gap non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197408 | 0230 | 1 | F | Addition of 5.6.1.2 event without gap in DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197409 | 0231 | 1 | F | Addition of 5.6.1.3 event with gap non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197410 | 0232 | 1 | F | Addition of 5.6.1.4 event with gap in DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197411 | 0249 | 1 | F | Addition of NR test case 5.5.1.5-CSI-RS RLM OOS non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197412 | 0250 | 1 | F | Addition of NR test case 5.5.1.6-CSI-RS RLM IS non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197413 | 0251 | 1 | F | Addition of NR test case 5.5.1.7-CSI-RS RLM OOS DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197414 | 0252 | 1 | F | Addition of NR test case 5.5.1.8-CSI-RS RLM IS DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197415 | 0254 | 1 | F | Addition of NR test case 5.5.5.3-CSI-RS BFD non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197416 | 0255 | 1 | F | Addition of NR test case 5.5.5.4-CSI-RS BFD DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197417 | 0259 | 1 | F | Addition of minimum conformance requirements for inter-RAT NR measurements | 16.1.0 |
| 2019-09 | RAN#85 | R5-197418 | 0289 | 1 | F | Correction of default configuration in annex H | 16.1.0 |
| 2019-09 | RAN#85 | R5-197419 | 0303 | 1 | F | Updated to cell configuration mapping table for RRM tests | 16.1.0 |
| 2019-09 | RAN#85 | R5-197420 | 0313 | 1 | F | Correction of default message content for RRM in Annex H | 16.1.0 |
| 2019-09 | RAN#85 | R5-197422 | 0305 | 1 | F | Clarification on NE-DC tests for RRM | 16.1.0 |
| 2019-09 | RAN#85 | R5-197423 | 0343 | 1 | F | Addition RRM FR2 test setups into TS 38.533 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197424 | 0345 | 1 | F | Update TDD UL-DL Config based on TS 38.133 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197425 | 0347 | 1 | F | Modification of EN-DC FR1 UE Transmit Timing Section 4.4.1.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197426 | 0265 | 1 | F | Correction of NR test case 4.5.1.5-CSI-RS RLM OOS non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197427 | 0266 | 1 | F | Correction of NR test case 4.5.1.6-CSI-RS RLM IS non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197428 | 0267 | 1 | F | Correction of NR test case 4.5.1.7-CSI-RS RLM OOS DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197429 | 0268 | 1 | F | Correction of NR test case 4.5.1.8-CSI-RS RLM IS DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197430 | 0269 | 1 | F | Correction of NR test case 4.5.2.1-interruption transition DRX sync | 16.1.0 |
| 2019-09 | RAN#85 | R5-197431 | 0270 | 1 | F | Correction of NR test case 4.5.2.2-interruption transition DRX async | 16.1.0 |
| 2019-09 | RAN#85 | R5-197432 | 0271 | 1 | F | Correction of NR test case 4.5.2.3-interruption NR deactivated SCell sync | 16.1.0 |
| 2019-09 | RAN#85 | R5-197433 | 0272 | 1 | F | Correction of NR test case 4.5.2.4-interruption NR deactivated SCell async | 16.1.0 |
| 2019-09 | RAN#85 | R5-197434 | 0273 | 1 | F | Correction of NR test case 4.5.2.5-interruption LTE deactivated SCell sync | 16.1.0 |
| 2019-09 | RAN#85 | R5-197435 | 0274 | 1 | F | Correction of NR test case 4.5.2.6-interruption LTE deactivated SCell async | 16.1.0 |
| 2019-09 | RAN#85 | R5-197436 | 0328 | 1 | F | Addition of CSI-RS based L1-RSRP measurement RRM test case 4.6.3.4 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197437 | 0315 | 1 | F | Correction of 5G RRM Inter Frequency measurements SA test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197570 | 0294 | 1 | F | Annex E and F FR1 Test tolerance update for SS-RSRP | 16.1.0 |
| 2019-09 | RAN#85 | R5-197584 | 0344 | 1 | F | Addition Annex for RRM OTA procedures in TS 38.533 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197607 | 0287 | 1 | F | Correction of NR test case 7.6.1.3-with gap non-DRX. | 16.1.0 |
| 2019-09 | RAN#85 | R5-197611 | 0237 | 1 | F | Update of 6.3.1.5 inter-RAT handover to unknown cell | 16.1.0 |
| 2019-09 | RAN#85 | R5-197612 | 0291 | 1 | F | Update of SSB-based RLM in-sync test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197624 | 0297 | 1 | F | Correction to PRACH configurations for FR1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197651 | 0348 | 2 | F | Modification of EN-DC FR1 TAAA Section 4.4.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197652 | 0349 | 2 | F | Modification of EN-DC FR1 SSB-RLM OOS Section 4.5.1.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197653 | 0350 | 2 | F | Modification of EN-DC FR1 SSB-RLM OOS DRX Section 4.5.1.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197654 | 0358 | 2 | F | Modification of SA FR1 SSB RLM OOS in non-DRX Section 6.5.1.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197655 | 0359 | 2 | F | Modification of SA FR1 SSB RLM OOS in DRX Section 6.5.1.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197656 | 0292 | 2 | F | FR1 Test tolerance update for SS-RSRP NSA FR1 test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197657 | 0300 | 2 | F | Correction to EN-DC FR1 radio link monitoring TC 4.5.1.1 and 4.5.1.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197658 | 0293 | 2 | F | FR1 Test tolerance update for SS-RSRP SA FR1 test cases | 16.1.0 |
| 2019-12 | RAN#86 | R5-197794 | 0369 | - | F | Update Test Tolerance of 4.5.2.1 interruptions active and non-active in sync | 16.2.0 |
| 2019-12 | RAN#86 | R5-197795 | 0370 | - | F | Update Test Tolerance of 4.5.2.2 interruptions active and non-active in async | 16.2.0 |
| 2019-12 | RAN#86 | R5-197804 | 0379 | - | F | Update Test Tolerance of 6.6.3.1 inter-RAT measurement non-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-197805 | 0380 | - | F | Update Test Tolerance of 6.6.3.2 inter-RAT measurement DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-197813 | 0382 | - | F | Addition of minimum conformance requirements 5.5.5.0.1-SSB based BFD | 16.2.0 |
| 2019-12 | RAN#86 | R5-197814 | 0383 | - | F | Addition of minimum conformance requirements 8.2.1.0-inter-RAT reselection | 16.2.0 |
| 2019-12 | RAN#86 | R5-197815 | 0384 | - | F | Addition of minimum conformance requirements 8.3.1.0-inter-RAT handover | 16.2.0 |
| 2019-12 | RAN#86 | R5-197816 | 0385 | - | F | Addition of minimum conformance requirements 8.4.1.0-inter-RAT SFTD delay | 16.2.0 |
| 2019-12 | RAN#86 | R5-197817 | 0386 | - | F | Addition of minimum conformance requirements 8.5.1.0-inter-RAT SFTD accuracy | 16.2.0 |
| 2019-12 | RAN#86 | R5-197820 | 0389 | - | F | Addition of NR test case 8.2.1.1-high priority NR reselection | 16.2.0 |
| 2019-12 | RAN#86 | R5-198029 | 0394 | - | F | Update NSA SS-RSRP tests for 4Rx connection diagram | 16.2.0 |
| 2019-12 | RAN#86 | R5-198032 | 0397 | - | F | Align Annex D to TS 38.133 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198289 | 0400 | - | F | Correction of PRACH index in 6.1.1.1, 6.3.2.2.1 and 6.3.2.2.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198415 | 0404 | - | F | Correction to NR SA FR1 timing advance adjustment accuracy TC 6.4.3.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198416 | 0405 | - | F | Correction to intra-frequency event-triggered reporting TC 6.6.1.1, 6.6.1.2, 6.6.1.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198421 | 0410 | - | F | Correction to FR1 SA random access test cases | 16.2.0 |
| 2019-12 | RAN#86 | R5-198472 | 0412 | - | F | Correct message contents for 5.6.2.x tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-198477 | 0416 | - | F | Correction to Annex E | 16.2.0 |
| 2019-12 | RAN#86 | R5-198518 | 0418 | - | F | Update PUSCH symbol length as message exception in 6.4.1.1 in TS 38.533 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198519 | 0419 | - | F | Update 4.4.3.1 test requirements | 16.2.0 |
| 2019-12 | RAN#86 | R5-198545 | 0420 | - | F | Update 4.7.1.1.1 test requirements in TS 38.533 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198549 | 0421 | - | F | Updated to cell configuration mapping table for RRM tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-198550 | 0422 | - | F | Minimum conformance requirements updated for 5G RRM Inter-RAT measurements tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-198551 | 0423 | - | F | Minimum conformance requirements updated for 5G RRM SCell activation and deactivation delay tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-198558 | 0428 | - | F | 5G RRM Spec 3GPP style correction in clause 4 and 5 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198572 | 0429 | - | F | Correction of minimum conformance requirements 4.6.2.0 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198575 | 0432 | - | F | Correction of minimum conformance requirements 5.6.3.0 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198577 | 0434 | - | F | Correction of clause 3 definitions, symbols and abbreviations | 16.2.0 |
| 2019-12 | RAN#86 | R5-198578 | 0435 | - | F | Correction of RRM Test Case 4.6.3.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198581 | 0438 | - | F | Correction of RRM Test Case 4.6.3.4 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198582 | 0439 | - | F | Correction of RRM Test Case 5.6.2.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198583 | 0440 | - | F | Correction of RRM Test Case 5.6.2.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198584 | 0441 | - | F | Correction of RRM Test Case 5.6.2.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198585 | 0442 | - | F | Correction of RRM Test Case 5.6.2.4 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198586 | 0443 | - | F | Correction of RRM Test Case 5.6.2.5 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198587 | 0444 | - | F | Correction of RRM Test Case 5.6.2.6 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198588 | 0445 | - | F | Correction of RRM Test Case 5.6.2.7 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198589 | 0446 | - | F | Correction of RRM Test Case 5.6.2.8 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198594 | 0451 | - | F | Correction of RRM Test Case 7.6.2.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198595 | 0452 | - | F | Correction of RRM Test Case 7.6.2.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198596 | 0453 | - | F | Correction of RRM Test Case 7.6.2.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198597 | 0454 | - | F | Correction of RRM Test Case 7.6.2.4 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198598 | 0455 | - | F | Correction of RRM Test Case 7.6.2.5 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198599 | 0456 | - | F | Correction of RRM Test Case 7.6.2.6 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198601 | 0458 | - | F | Correction of RRM Test Case 7.6.2.8 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198692 | 0471 | - | F | Addition of RRM Test Case 6.6.4.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198693 | 0472 | - | F | Addition of RRM Test Case 6.6.4.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198694 | 0473 | - | F | Addition of RRM Test Case 6.6.4.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198695 | 0474 | - | F | Addition of RRM Test Case 6.6.4.4 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198696 | 0475 | - | F | Addition of RRM Test Case 7.6.3.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198697 | 0476 | - | F | Addition of RRM Test Case 7.6.3.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198698 | 0477 | - | F | Addition of RRM Test Case 7.6.3.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198699 | 0478 | - | F | Addition of RRM Test Case 7.6.3.4 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199364 | 0365 | 1 | F | Update Test Tolerance of 4.5.1.5 RLM OOS non-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199365 | 0367 | 1 | F | Update Test Tolerance of 4.5.1.7 RLM OOS DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199366 | 0375 | 1 | F | Update Test Tolerance of 6.5.1.5 SA RLM OOS non-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199367 | 0377 | 1 | F | Update Test Tolerance of 6.5.1.7 SA RLM OOS DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199368 | 0366 | 1 | F | Update Test Tolerance of 4.5.1.6 RLM in-sync non-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199369 | 0368 | 1 | F | Update Test Tolerance of 4.5.1.8 RLM in-sync DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199370 | 0376 | 1 | F | Update Test Tolerance of 6.5.1.6 SA RLM in-sync non-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199371 | 0378 | 1 | F | Update Test Tolerance of 6.5.1.8 SA RLM in-sync DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199379 | 0417 | 1 | F | Update 4.4.1.1 test procedure in TS 38.533 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199380 | 0479 | 1 | F | Update 4.5.1.1 and 4.5.1.2 in TS 38.533 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199381 | 0406 | 1 | F | Correction to NR SA FR1 SS-RSRP absolute and relative measurement accuracy Intra-frequency measurements | 16.2.0 |
| 2019-12 | RAN#86 | R5-199389 | 0402 | 1 | F | Correction to NR SA Cell re-selection tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-199390 | 0436 | 1 | F | Correction of RRM Test Case 4.6.3.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199391 | 0437 | 1 | F | Correction of RRM Test Case 4.6.3.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199392 | 0387 | 1 | F | Addition of NR test case 5.5.5.1-ENDC FR2 SSB BFD no-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199393 | 0388 | 1 | F | Addition of NR test case 5.5.5.2-ENDC FR2 SSB BFD DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199394 | 0431 | 1 | F | Correction of minimum conformance requirements 5.6.2.0 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199395 | 0395 | 1 | F | Update SA SS-RSRP tests for 4Rx connection diagram | 16.2.0 |
| 2019-12 | RAN#86 | R5-199396 | 0403 | 1 | F | Correction to NR SA FR1 UE transmit timing accuracy TC 6.4.1.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199397 | 0457 | 1 | F | Correction of RRM Test Case 7.6.2.7 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199398 | 0361 | 1 | F | Add AoA Setup 4 for FR2 RRM Test cases | 16.2.0 |
| 2019-12 | RAN#86 | R5-199399 | 0390 | 1 | F | Addition of NR test case 8.3.1.1-known handover | 16.2.0 |
| 2019-12 | RAN#86 | R5-199400 | 0391 | 1 | F | Addition of NR test case 8.4.1.1-SFTD delay non-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199401 | 0392 | 1 | F | Addition of NR test case 8.4.1.2-SFTD delay DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199402 | 0393 | 1 | F | Addition of NR test case 8.5.1.1-SFTD accuracy | 16.2.0 |
| 2019-12 | RAN#86 | R5-199403 | 0396 | 1 | F | Align Annex A to TS 38.133 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199404 | 0413 | 1 | F | Clean up RRM message contents in Annex H | 16.2.0 |
| 2019-12 | RAN#86 | R5-199405 | 0424 | 1 | F | New 5G RRM Inter-RAT measurement TC 8.4.2.5 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199406 | 0425 | 1 | F | New 5G RRM Inter-RAT measurement TC 8.4.2.6 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199407 | 0426 | 1 | F | New 5G RRM Inter-RAT measurement TC 8.4.2.7 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199408 | 0427 | 1 | F | New 5G RRM Inter-RAT measurement TC 8.4.2.8 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199409 | 0433 | 1 | F | Correction of message content in Annex H | 16.2.0 |
| 2019-12 | RAN#86 | R5-199436 | 0470 | 1 | F | Introduction of n29 and n65 to 38.533 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199499 | 0401 | 1 | F | Correction to EN-DC FR1 UE transmit timing accuracy TC 4.4.1.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199515 | 0381 | 1 | F | Update Test Tolerance in Annex F | 16.2.0 |
| 2019-12 | RAN#86 | R5-199517 | 0481 | 1 | F | Update General parameters in test case 4.5.1.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199533 | 0482 | 1 | F | Update NSA FR2 RLM IS non-DRX test | 16.2.0 |
| 2019-12 | RAN#86 | R5-199534 | 0483 | 1 | F | Update NSA FR2 RLM IS test with DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199535 | 0484 | 1 | F | Update NSA FR2 Timing Accuracy Test | 16.2.0 |
| 2019-12 | RAN#86 | R5-199544 | 0399 | 1 | F | Add applicable test methods for RRM FR2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199550 | 0463 | 1 | F | Addition of NR SA FR1 Handover with known Target Cell Section 6.3.1.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199551 | 0464 | 1 | F | Modification of NR SA FR1 Handover with unknown Target Cell Section 6.3.1.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199552 | 0465 | 1 | F | Modification of NR SA FR1-FR1 Handover with unknown Target Cell Section 6.3.1.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199583 | 0409 | 1 | F | Correction to FR1 NSA random access test cases | 16.2.0 |
| 2020-03 | RAN#87 | R5-200822 | 0544 | - | F | Correction to Test Applicability for 4.3.2.2.2 Non-contention based random access test in FR1 for PSCell in EN-DC | 16.3.0 |
| 2020-03 | RAN#87 | R5-200823 | 0545 | - | F | Correction to SRS Configuration for 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy | 16.3.0 |
| 2020-03 | RAN#87 | R5-200829 | 0551 | - | F | Correction to Message Exception for 4.6.1.5 EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | 16.3.0 |
| 2020-03 | RAN#87 | R5-200833 | 0555 | - | F | Correction to Test Applicability for 6.3.2.2.2 Non-Contention based random access test in FR1 for NR standalone | 16.3.0 |
| 2020-03 | RAN#87 | R5-200834 | 0556 | - | F | Correction to SRS Configuration for 6.4.3.1 NR SA FR1 timing advance adjustment accuracy | 16.3.0 |
| 2020-03 | RAN#87 | R5-200839 | 0561 | - | F | Correction to Message Exception for 6.6.1.5 NR SA FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | 16.3.0 |
| 2020-03 | RAN#87 | R5-200277 | 0488 | - | F | Test tolerance update intra-frequency SS-SINR NSA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200278 | 0489 | - | F | Test tolerance update inter-frequency SS-SINR NSA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200279 | 0490 | - | F | Test tolerance update inter-frequency SS-RSRP SA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200281 | 0492 | - | F | Test tolerance update inter-frequency SS-RSRQ SA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200282 | 0493 | - | F | Test tolerance update intra-frequency SS-SINR SA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200283 | 0494 | - | F | Test tolerance update inter-frequency SS-SINR SA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200306 | 0497 | - | F | Reference SSB configuration correction | 16.3.0 |
| 2020-03 | RAN#87 | R5-200421 | 0508 | - | F | Correction to Active UL BWP for intra-frequency event triggered reporting with gap | 16.3.0 |
| 2020-03 | RAN#87 | R5-200422 | 0509 | - | F | Correction to EN-DC FR1 event-triggered reporting test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-200423 | 0510 | - | F | Correction to FR1 NR SA E-UTRA cell re-selection test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-200425 | 0512 | - | F | Correction to NR SA FR1 SS-RSRP measurement accuracy tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-200427 | 0514 | - | F | Correction to the simulated cell for cell-reselection test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-200428 | 0515 | - | F | Correction to UE transmit timing accuracy test case | 16.3.0 |
| 2020-03 | RAN#87 | R5-200564 | 0524 | - | F | Correction to Statistical testing of delay and UE measurement performance in RRM tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-200606 | 0526 | - | F | Cell mapping update measurement tests Annex E | 16.3.0 |
| 2020-03 | RAN#87 | R5-200607 | 0527 | - | F | Re-submission R5-197804 not implemented | 16.3.0 |
| 2020-03 | RAN#87 | R5-200608 | 0528 | - | F | Corrections to SS-RSRP meas accuracy NSA tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-200610 | 0530 | - | F | Correct message contents measurement tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-200705 | 0533 | - | F | Correction to TC 6.1.1.2 FR1 inter-freq re-selection | 16.3.0 |
| 2020-03 | RAN#87 | R5-200708 | 0536 | - | F | Update of TC 6.3.2.3.1 FR1 RRC redirection | 16.3.0 |
| 2020-03 | RAN#87 | R5-200709 | 0537 | - | F | Update of TC 6.3.2.3.2 inter-freq RRC redirection | 16.3.0 |
| 2020-03 | RAN#87 | R5-200785 | 0539 | - | F | Update of EN-DC SSB-based RLM TC 4.5.1.1, 4.5.1.2, 4.5.1.3 and 4.5.1.4 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200786 | 0540 | - | F | Update of EN-DC CSI-RS-based RLM TC 4.5.1.5, 4.5.1.6, 4.5.1.7 and 4.5.1.8 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200787 | 0541 | - | F | Update of NR CSI-RS-based RLM TC 6.5.1.5, 6.5.1.6, 6.5.1.7 and 6.5.1.8 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200818 | 0543 | - | F | Correction to cell mapping Annex E | 16.3.0 |
| 2020-03 | RAN#87 | R5-200858 | 0569 | - | F | Corrections to Table H.3.1-8 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200916 | 0538 | 1 | F | Update to test applicability per permitted test method | 16.3.0 |
| 2020-03 | RAN#87 | R5-200986 | 0507 | 1 | F | Core alignment for event-triggered reporting test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-200987 | 0534 | 1 | F | Update of TC 6.3.2.1.1 FR1 RRC re-establishment | 16.3.0 |
| 2020-03 | RAN#87 | R5-200988 | 0535 | 1 | F | Update of TC 6.3.2.1.2 inter-freq RRC re-establishment | 16.3.0 |
| 2020-03 | RAN#87 | R5-201030 | 0529 | 1 | F | Corrections to SS-RSRP meas accuracy SA tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-201039 | 0485 | 1 | F | Test tolerance update inter-frequency SS-RSRP NSA | 16.3.0 |
| 2020-03 | RAN#87 | R5-201040 | 0486 | 1 | F | Test tolerance update intra-frequency SS-RSRQ NSA | 16.3.0 |
| 2020-03 | RAN#87 | R5-201041 | 0491 | 1 | F | Test tolerance update intra-frequency SS-RSRQ SA | 16.3.0 |
| 2020-03 | RAN#87 | R5-201056 | 0511 | 1 | F | Correction to message contents in 6.6.1.5 and 6.6.1.6 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201057 | 0513 | 1 | F | Correction to test description of RRM TC 4.6.1.3 and 4.6.1.6 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201058 | 0516 | 1 | F | Update of maximum test system uncertainty for FR1 RRM Test | 16.3.0 |
| 2020-03 | RAN#87 | R5-201063 | 0517 | 1 | F | Update FR1 Test Tolerance of 4.5.2.3 interruptions on NR SCC in sync | 16.3.0 |
| 2020-03 | RAN#87 | R5-201064 | 0518 | 1 | F | Update FR1 Test Tolerance of 4.5.2.4 interruptions on NR SCC in async | 16.3.0 |
| 2020-03 | RAN#87 | R5-201071 | 0552 | 1 | F | Correction to Test Applicability for 4.6.2.2 EN-DC FR1-FR1 event-triggered reporting in DRX | 16.3.0 |
| 2020-03 | RAN#87 | R5-201072 | 0553 | 1 | F | Correction to Test Applicability for 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | 16.3.0 |
| 2020-03 | RAN#87 | R5-201073 | 0554 | 1 | F | Correction to Test Applicability for 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | 16.3.0 |
| 2020-03 | RAN#87 | R5-201078 | 0562 | 1 | F | Correction to Test Applicability for 6.6.2.1 NR SA FR1-FR1 event-triggered reporting in non-DRX | 16.3.0 |
| 2020-03 | RAN#87 | R5-201080 | 0564 | 1 | F | Correction to Test Applicability for 6.6.2.5 NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | 16.3.0 |
| 2020-03 | RAN#87 | R5-201081 | 0565 | 1 | F | Correction to Test Applicability for 6.6.2.6 NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | 16.3.0 |
| 2020-03 | RAN#87 | R5-201082 | 0571 | 1 | F | Correction to Test Parameters for 4.6.1.1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201083 | 0572 | 1 | F | Correction to Test Parameters for 4.6.1.2 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201160 | 0531 | 1 | F | Correct message contents event triggered measurement tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-201166 | 0567 | 1 | F | Update RLM non DRX test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-201167 | 0568 | 1 | F | Update 4.5.1.4 test case | 16.3.0 |
| 2020-03 | RAN#87 | R5-201168 | 0570 | 1 | F | Clarification 4.4.1.1 test procedure | 16.3.0 |
| 2020-03 | RAN#87 | R5-201169 | 0521 | 1 | F | Update of FR1 Test Tolerance in Annex F | 16.3.0 |
| 2020-03 | RAN#87 | R5-201190 | 0542 | 1 | F | Update of NR SSB-based RLM TC 6.5.1.1, 6.5.1.2, 6.5.1.3 and 6.5.1.4 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201240 | 0563 | 2 | F | Correction to Test Applicability for 6.6.2.2 NA SA FR1-FR1 event-triggered reporting in DRX | 16.3.0 |
| 2020-03 | RAN#87 | R5-201242 | 0532 | 2 | F | Correction to Test Applicability for 4.6.2.1 EN-DC FR1-FR1 event-triggered reporting in non-DRX | 16.3.0 |
| 2020-03 | RAN#87 | R5-200276 | 0487 | - | F | Test tolerance update inter-frequency SS-RSRQ NSA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200284 | 0495 | - | F | Test tolerance update measurement tests Annex F | 16.3.0 |
| 2020-03 | RAN#87 | R5-201044 | 0496 | - | F | Test Tolerance and Measurement Uncertainty in Annex F for L1-RSRP measurement test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-201045 | 0499 | 1 | F | Correction of RRM Test Case 4.6.3.1 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201046 | 0500 | 1 | F | Correction of RRM Test Case 4.6.3.2 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201047 | 0501 | 1 | F | Correction of RRM Test Case 4.6.3.3 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201048 | 0502 | 1 | F | Correction of RRM Test Case 4.6.3.4 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201049 | 0503 | 1 | F | Correction of RRM Test Case 6.6.4.1 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201050 | 0504 | 1 | F | Correction of RRM Test Case 6.6.4.2 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201051 | 0505 | 1 | F | Correction of RRM Test Case 6.6.4.3 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201052 | 0506 | 1 | F | Correction of RRM Test Case 6.6.4.4 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201070 | 0548 | 1 | F | Correction to Message Exception for 4.5.1.3 EN-DC FR1 radio link monitoring Out-of-sync test for PSCell configured with SSB-based RLM-RS in DRX mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201074 | 0557 | 1 | F | Correction to Test Parameter for 6.5.1.1 NR SA FR1 radio link monitoring Out-of-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201075 | 0558 | 1 | F | Correction to Test Procedure for 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM-RS in non-DRX mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201076 | 0559 | 1 | F | Correction to Message Exception for 6.5.1.3 NR SA FR1 radio link monitoring Out-of-sync test for PSCell configured with SSB-based RLM-RS in DRX mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201077 | 0560 | 1 | F | Correction to Message Exception for 6.5.1.4 NR SA FR1 radio link monitoring In-sync test for PSCell configured with SSB-based RLM-RS in DRX mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201105 | 0546 | 1 | F | Correction to Test Parameters for 4.5.1.1 EN-DC FR1 radio link monitoring Out-of-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201106 | 0547 | 1 | F | Correction to Test Procedure for 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode | 16.3.0 |
| 2020-06 | RAN#88 | R5-201395 | 0576 | - | F | Correction to NR test case 7.7.1.3.2 SS-RSRP relative accuracy | 16.4.0 |
| 2020-06 | RAN#88 | R5-201396 | 0577 | - | F | Correction to NR test case 8.2.1.1 higher priority NR cell reselection | 16.4.0 |
| 2020-06 | RAN#88 | R5-201398 | 0579 | - | F | Correction to NR test case 8.4.1.1 SFTD non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201399 | 0580 | - | F | Correction to NR test case 8.4.1.2 SFTD DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201400 | 0581 | - | F | Correction to NR test case 8.4.2.1 event-triggered without SSB index non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201401 | 0582 | - | F | Correction to NR test case 8.4.2.2 event-triggered without SSB index DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201402 | 0583 | - | F | Correction to NR test case 8.4.2.3 event-triggered with SSB index non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201403 | 0584 | - | F | Correction to NR test case 8.4.2.4 event-triggered with SSB index DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201404 | 0585 | - | F | Correction to NR test case 8.5.1.1 SFTD accuracy | 16.4.0 |
| 2020-06 | RAN#88 | R5-201405 | 0586 | - | F | Addition to minimum conformance requirement 5.5.5.0.3 - scheduling restriction for BFD and CBD | 16.4.0 |
| 2020-06 | RAN#88 | R5-201407 | 0588 | - | F | Addition to minimum conformance requirement 5.5.8.0 - TCI-state switch | 16.4.0 |
| 2020-06 | RAN#88 | R5-201408 | 0589 | - | F | Addition to NR test case 5.5.8.1 MAC-CE based TCI switch | 16.4.0 |
| 2020-06 | RAN#88 | R5-201409 | 0590 | - | F | Addition to NR test case 5.5.8.2 RRC based TCI switch | 16.4.0 |
| 2020-06 | RAN#88 | R5-201412 | 0593 | - | F | Addition to minimum conformance requirements 7.5.1.0.5 - Scheduling restriction for RLM | 16.4.0 |
| 2020-06 | RAN#88 | R5-201414 | 0595 | - | F | Addition to minimum Conformance Requirements 7.5.6.1.0 - DCI-based DL active BWP switch | 16.4.0 |
| 2020-06 | RAN#88 | R5-201415 | 0596 | - | F | Addition to NR test case 7.5.6.1.3 FR2 DCI-based DL active BWP switch non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201619 | 0603 | - | F | Corrections to 6.7.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201676 | 0614 | - | F | Correction to inter-freq measurement TCs for SSB configuration | 16.4.0 |
| 2020-06 | RAN#88 | R5-201677 | 0615 | - | F | Correction to TC 6.3.2.3.1 for the test procedure | 16.4.0 |
| 2020-06 | RAN#88 | R5-201679 | 0617 | - | F | Update of TC 5.6.1.1 event-triggered without gap non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201680 | 0618 | - | F | Update of TC 5.6.1.2 event-triggered without gap DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201681 | 0619 | - | F | Update of TC 5.6.1.3 event-triggered with gap non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201682 | 0620 | - | F | Update of TC 5.6.1.4 event-triggered with gap DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201686 | 0624 | - | F | Update of TC 8.4.2.5 FR2 event-triggered without SSB index non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201687 | 0625 | - | F | Update of TC 8.4.2.6 FR2 event-triggered without SSB index DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201688 | 0626 | - | F | Update of TC 8.4.2.7 FR2 event-triggered with SSB index non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201689 | 0627 | - | F | Update of TC 8.4.2.8 FR2 event-triggered with SSB index DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201821 | 0634 | - | F | Correction of SS-SINR applicability -NSA | 16.4.0 |
| 2020-06 | RAN#88 | R5-201822 | 0635 | - | F | Correction of SS-SINR applicability -SA | 16.4.0 |
| 2020-06 | RAN#88 | R5-201824 | 0637 | - | F | Corrections to 4.7.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202054 | 0644 | - | F | Minimum Requirements SS-RSRP iRAT | 16.4.0 |
| 2020-06 | RAN#88 | R5-202055 | 0645 | - | F | Addition 8.5.2.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202056 | 0646 | - | F | Addition 8.5.2.1.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202057 | 0647 | - | F | Minimum Requirements SS-RSRQ iRAT | 16.4.0 |
| 2020-06 | RAN#88 | R5-202058 | 0648 | - | F | Addition 8.5.2.2.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202059 | 0649 | - | F | Addition 8.5.2.2.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202060 | 0650 | - | F | Minimum Requirements SS-SINR iRAT | 16.4.0 |
| 2020-06 | RAN#88 | R5-202061 | 0651 | - | F | Addition 8.5.2.3.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202062 | 0652 | - | F | Addition 8.5.2.3.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202063 | 0653 | - | F | Minimum requirements SS-RSRP NSA FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202064 | 0654 | - | F | Addition 5.7.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202065 | 0655 | - | F | Addition 5.7.1.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202066 | 0656 | - | F | Addition 5.7.1.3 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202067 | 0657 | - | F | Minimum requirements SS-RSRQ NSA FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202068 | 0658 | - | F | Addition 5.7.2.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202069 | 0659 | - | F | Addition 5.7.2.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202070 | 0660 | - | F | Minimum requirements SS-SINR NSA FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202071 | 0661 | - | F | Addition 5.7.3.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202072 | 0662 | - | F | Addition 5.7.3.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202073 | 0663 | - | F | Minimum requirements SS-RSRP SA FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202074 | 0664 | - | F | Addition 7.7.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202075 | 0665 | - | F | Addition 7.7.1.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202076 | 0666 | - | F | Minimum requirements SS-RSRQ SA FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202077 | 0667 | - | F | Addition 7.7.2.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202078 | 0668 | - | F | Addition 7.7.2.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202079 | 0669 | - | F | Minimum requirements SS-SINR SA FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202080 | 0670 | - | F | Addition 7.7.3.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202081 | 0671 | - | F | Addition 7.7.3.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202091 | 0672 | - | F | Correction of OCNG configurations and CSI-RS for tracking in Annex A | 16.4.0 |
| 2020-06 | RAN#88 | R5-202092 | 0673 | - | F | Cell configuration for SA FR2 event-triggered reporting test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202093 | 0674 | - | F | Correction of MeasGapConfig in Annex H | 16.4.0 |
| 2020-06 | RAN#88 | R5-202095 | 0676 | - | F | Correction of AoA configuration for FR2 SA Inter-freq measurement test cases in clause 5.6.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202096 | 0677 | - | F | Correction of RRM Test Case 7.6.2.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202097 | 0678 | - | F | Correction of RRM Test Case 7.6.2.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202098 | 0679 | - | F | Correction of RRM Test Case 7.6.2.3 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202099 | 0680 | - | F | Correction of RRM Test Case 7.6.2.4 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202100 | 0681 | - | F | Correction of RRM Test Case 7.6.2.5 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202101 | 0682 | - | F | Correction of RRM Test Case 7.6.2.6 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202102 | 0683 | - | F | Correction of RRM Test Case 7.6.2.7 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202103 | 0684 | - | F | Correction of RRM Test Case 7.6.2.8 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202263 | 0704 | - | F | Core alignment in 4.6.1.5 and 4.6.1.6 Event Triggered Reporting with SSB time index detection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202265 | 0706 | - | F | Correction on EN-DC FR1 radio link monitoring out-of-sync test | 16.4.0 |
| 2020-06 | RAN#88 | R5-202273 | 0714 | - | F | Correction to NR SA FR1 - E-UTRA RRC Connection release with redirection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202277 | 0718 | - | F | Editorial Correction to FR2 Interruption test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202385 | 0720 | - | F | Addition of RRM Cell configuration mapping table for EN-DC FR1 Test Case 4.5.4.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202387 | 0721 | - | F | Addition of RRC message content exceptions for UE UL Carrier Reconfiguration Delay | 16.4.0 |
| 2020-06 | RAN#88 | R5-202389 | 0722 | - | F | Addition of RRM Test Case 4.5.4.1 in Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-202415 | 0728 | - | F | Alignment 5.4.1.1 test procedure with 4.4.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202418 | 0730 | - | F | Update 6.5.1.1 and CSI-ReportConfig in Annex H | 16.4.0 |
| 2020-06 | RAN#88 | R5-202488 | 0736 | - | F | Correction to the simulated cell for cell-reselection test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202496 | 0744 | - | F | Correction of RRM Test Case 6.4.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202497 | 0745 | - | F | Title Correction of Annex D.4.2.4 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202700 | 0723 | 1 | F | Addition of new RRM Test Case 4.5.4.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202701 | 0675 | 1 | F | Editorial correction of RRM TT in Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-202744 | 0708 | 1 | F | Correction to EN-DC FR1 radio link monitoring tests | 16.4.0 |
| 2020-06 | RAN#88 | R5-202745 | 0727 | 1 | F | Clarification cell2 SS-RSRP in 4.7.1.2.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202746 | 0587 | 1 | F | Addition to NR test case 5.5.5.5 scheduling available restriction | 16.4.0 |
| 2020-06 | RAN#88 | R5-202747 | 0707 | 1 | F | Correction on NR SA FR1 - E-UTRAN event-triggered reporting tests | 16.4.0 |
| 2020-06 | RAN#88 | R5-202748 | 0731 | 1 | F | Update 6.5.1.2 message content exception and test procedure | 16.4.0 |
| 2020-06 | RAN#88 | R5-202749 | 0737 | 1 | F | Correction of RRM Test Case 6.1.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202750 | 0738 | 1 | F | Correction of RRM Test Case 6.1.1.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202751 | 0621 | 1 | F | Update of TC 7.1.1.1 intra-freq cell re-selection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202752 | 0622 | 1 | F | Update of TC 7.1.1.2 inter-freq cell re-selection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202753 | 0724 | 1 | F | Addition of new RRM Test Cases in clause 7.5.3 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202754 | 0725 | 1 | F | Addition of new RRM Test Cases in clause 7.5.7 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202812 | 0685 | 1 | F | Correction to Test Applicability for 4.6.1.3 EN-DC FR1 event-triggered reporting with gap in non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-202813 | 0686 | 1 | F | Correction to Test Applicability for 4.6.1.4 EN-DC FR1 event-triggered reporting with gap in DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-202814 | 0687 | 1 | F | Correction to Test Applicability & Message Exception for 4.6.1.6 EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202815 | 0688 | 1 | F | Correction to Test Applicability for 6.6.1.3 NR SA FR1 event-triggered reporting with gap in non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-202816 | 0689 | 1 | F | Correction to Test Applicability for 6.6.1.4 NR SA FR1 event-triggered reporting with gap in DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-202817 | 0732 | 1 | F | Correction to Test Requirement for 4.4.3.1 by updating TT values from Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-202833 | 0591 | 1 | F | Addition to NR test case 6.3.2.1.3 FR1-FR1 RRC re-establishment without serving cell timing | 16.4.0 |
| 2020-06 | RAN#88 | R5-202834 | 0575 | 1 | F | Correction to NR test case 7.7.1.3.1 SA FR2 SS-RSRP absolute accuracy | 16.4.0 |
| 2020-06 | RAN#88 | R5-202835 | 0592 | 1 | F | Addition to NR test case 7.3.2.1.3 FR2-FR2 RRC re-establishment without serving cell timing | 16.4.0 |
| 2020-06 | RAN#88 | R5-202836 | 0594 | 1 | F | Addition to NR test case 7.5.1.9 FR2 RLM scheduling restrictions | 16.4.0 |
| 2020-06 | RAN#88 | R5-202837 | 0578 | 1 | F | Correction to NR test case 8.3.1.1 handover to known cell | 16.4.0 |
| 2020-06 | RAN#88 | R5-202838 | 0597 | 1 | F | Addition of Default Configuration in Annex H | 16.4.0 |
| 2020-06 | RAN#88 | R5-202839 | 0628 | 1 | F | Update of Annex D.4.1.1 for parameters of BFD with 4RX | 16.4.0 |
| 2020-06 | RAN#88 | R5-202840 | 0630 | 1 | F | Update of Cell mapping in Annex E | 16.4.0 |
| 2020-06 | RAN#88 | R5-202841 | 0633 | 1 | F | Add auxiliary bands for RRM inter-frequency SA tests | 16.4.0 |
| 2020-06 | RAN#88 | R5-202909 | 0697 | 1 | F | Correction to Test Requirements in 4.4.1.1, Table 4.4.1.1.5-5 to include TT values from Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-202910 | 0698 | 1 | F | Correction to Test Requirements in 6.4.1.1, Table 6.4.1.1.5-5 to include TT values from Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-202911 | 0734 | 1 | F | Correction to Test Requirement for 6.4.3.1 by updating TT values from Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-202912 | 0735 | 1 | F | Annex I RRM OTA procedures update to add RSRPB based UE Positioning Method for FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202913 | 0703 | 1 | F | Clarification of disabling Tx diversity for FR2 UE for FR2 RRM testing | 16.4.0 |
| 2020-06 | RAN#88 | R5-202914 | 0719 | 1 | F | Updates of FR2 MU and TT in TS 38.533 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202950 | 0693 | 1 | F | Correction to Message Exception for 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202951 | 0694 | 1 | F | Correction to Message Exception for 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202961 | 0690 | 1 | F | Correction to Test Applicability and Message Exception for 6.6.1.6 NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading | 16.4.0 |
| 2020-06 | RAN#88 | R5-202969 | 0639 | 1 | F | Corrections to 4.7.2.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202970 | 0641 | 1 | F | Corrections to 4.7.3.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202971 | 0642 | 1 | F | Corrections to 4.7.3.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202972 | 0598 | 1 | F | Corrections to 6.7.1.2 and core spec alignment | 16.4.0 |
| 2020-06 | RAN#88 | R5-202973 | 0599 | 1 | F | Corrections to 6.7.2.1 and core spec alignment | 16.4.0 |
| 2020-06 | RAN#88 | R5-202974 | 0600 | 1 | F | Corrections to 6.7.2.2 and core spec alignment | 16.4.0 |
| 2020-06 | RAN#88 | R5-202975 | 0601 | 1 | F | Corrections to 6.7.3.1 and core spec alignment | 16.4.0 |
| 2020-06 | RAN#88 | R5-202976 | 0602 | 1 | F | Corrections to 6.7.3.2 and core spec alignment | 16.4.0 |
| 2020-06 | RAN#88 | R5-202977 | 0638 | 1 | F | Corrections to 4.7.1.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202978 | 0640 | 1 | F | Corrections to 4.7.2.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202982 | 0710 | 1 | F | Correction to FR1 UE transmit timing accuracy | 16.4.0 |
| 2020-06 | RAN#88 | R5-202983 | 0709 | 1 | F | Correction to EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous ENDC | 16.4.0 |
| 2020-06 | RAN#88 | R5-202984 | 0711 | 1 | F | Correction to FR2 UE transmit timing accuracy | 16.4.0 |
| 2020-06 | RAN#88 | R5-202985 | 0705 | 1 | F | Core alignment in 6.6.1.2 NR SA FR1 event-triggered reporting without gap in DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-202986 | 0713 | 1 | F | Correction to NR E-UTRA reselection tests | 16.4.0 |
| 2020-06 | RAN#88 | R5-202987 | 0715 | 1 | F | Correction to NR SA FR1 handover test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202996 | 0604 | 1 | F | Update FR1 Test Tolerance of 4.5.2.5 E-UTRAN SCC in sync | 16.4.0 |
| 2020-06 | RAN#88 | R5-202997 | 0605 | 1 | F | Update FR1 Test Tolerance of 4.5.2.6 E-UTRAN SCC in async | 16.4.0 |
| 2020-06 | RAN#88 | R5-202998 | 0606 | 1 | F | Update FR1 Test Tolerance of 4.5.3.1 SCell activation 160ms | 16.4.0 |
| 2020-06 | RAN#88 | R5-202999 | 0607 | 1 | F | Update FR1 Test Tolerance of 4.5.3.2 SCell activation 320ms | 16.4.0 |
| 2020-06 | RAN#88 | R5-203000 | 0608 | 1 | F | Update FR1 Test Tolerance of 4.5.3.3 unknown SCell activation | 16.4.0 |
| 2020-06 | RAN#88 | R5-203091 | 0623 | 1 | F | Update of 4.5.3.0 minimum requirements for SCell activation | 16.4.0 |
| 2020-06 | RAN#88 | R5-203092 | 0613 | 1 | F | Update FR1 Test Tolerance of 6.5.4.1 UL carrier RRC reconfiguration delay | 16.4.0 |
| 2020-06 | RAN#88 | R5-203093 | 0629 | 1 | F | Update of FR1 Test Tolerance in Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-203095 | 0609 | 1 | F | Addition FR1 Test Tolerance of 6.5.5.1 SSB BFD non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-203096 | 0610 | 1 | F | Addition FR1 Test Tolerance of 6.5.5.2 SSB BFD DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-203097 | 0611 | 1 | F | Update FR1 Test Tolerance of 6.5.5.3 CSI-RS BFD non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-203098 | 0612 | 1 | F | Update FR1 Test Tolerance of 6.5.5.4 CSI-RS BFD DRX | 16.4.0 |
| 2020-09 | RAN#89 | R5-203234 | 0747 | - | F | Update TC 8.5.2.1.1.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-203235 | 0748 | - | F | Update TC 8.5.2.2.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-203249 | 0758 | - | F | Annex E and F update with TT results for 8.5.2.x.x iRAT periodic reporting test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-203250 | 0759 | - | F | Annex E and F update with TT results for 4.7.4.x.x EN-DC L1-RSRP periodic reporting test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-203252 | 0761 | - | F | Void unnecessary TC 8.5.2.1.1.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203267 | 0765 | - | F | [EDITORIAL] Fix formatting issues throwing DOCX conversion errors in TS 38.533 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203268 | 0766 | - | F | Correction of test applicability for EN-DC event-triggered tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-203269 | 0767 | - | F | Correction of test applicability for NR SA event-triggered tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-203270 | 0768 | - | F | Adding annex table for iRAT measurement configuration | 16.5.0 |
| 2020-09 | RAN#89 | R5-203271 | 0769 | - | F | Clarification on the usage of auxiliary bands | 16.5.0 |
| 2020-09 | RAN#89 | R5-203272 | 0770 | - | F | Remove IE includeBeamMeasurements from 6.6.1.5 and 6.6.1.6 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203280 | 0772 | - | F | n26 addition to group of bands in TS.38.533 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203299 | 0773 | - | F | Corrected test description in non-DRX test case 6.5.1.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203300 | 0774 | - | F | Correction of comment for monitoringSymbolsWithinSlot | 16.5.0 |
| 2020-09 | RAN#89 | R5-203301 | 0775 | - | F | Correction of delta offset application for Cell 2 in test case 4.7.1.2.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203303 | 0777 | - | F | PUSCH allocation correction in test case 6.4.3.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203304 | 0778 | - | F | Correction to the RRC Reconfiguration message for measurements | 16.5.0 |
| 2020-09 | RAN#89 | R5-203306 | 0779 | - | F | Update extreme conditions limits in TC 4.7.1.2.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203307 | 0780 | - | F | Update extreme conditions limits in TC 6.7.1.2.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203308 | 0781 | - | F | Update extreme conditions limits in TC 4.7.1.2.1 and TC 6.7.1.2.1 - Annex F | 16.5.0 |
| 2020-09 | RAN#89 | R5-203309 | 0782 | - | F | Correction power offset for Band Group F | 16.5.0 |
| 2020-09 | RAN#89 | R5-203317 | 0786 | - | F | Removing SearchSpaceId and ControlResourceSetId modification | 16.5.0 |
| 2020-09 | RAN#89 | R5-203319 | 0788 | - | F | Unification of cell frequency for FR1-FR1 test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-203732 | 0790 | - | F | Set ss-sinr request to false for EN-DC RRM measurement accuracy tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-203733 | 0791 | - | F | Set ss-sinr request to false for NR SA RRM measurement accuracy tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-203832 | 0798 | - | F | Update of 4.5.6.1.0 minimum requirement for DCI-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-203835 | 0801 | - | F | Update of 4.5.6.2.0 minimum requirement for RRC-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-203837 | 0803 | - | F | Update of FR1 Test Tolerance in 6.3.2.1.3 RRC re-establishment without serving cell timing | 16.5.0 |
| 2020-09 | RAN#89 | R5-203838 | 0804 | - | F | Addition of 6.5.2.0 minimum requirement for interruptions during measurements on NR SCC | 16.5.0 |
| 2020-09 | RAN#89 | R5-203839 | 0805 | - | F | Update of 6.5.2.1 interruptions during measurements on deactivated NR SCC | 16.5.0 |
| 2020-09 | RAN#89 | R5-203840 | 0806 | - | F | Addition of new Test Case 6.5.3.1 SCell activation and deactivation for 160ms cycle | 16.5.0 |
| 2020-09 | RAN#89 | R5-203841 | 0807 | - | F | Addition of new Test Case 6.5.3.2 SCell activation and deactivation for 320ms cycle | 16.5.0 |
| 2020-09 | RAN#89 | R5-203842 | 0808 | - | F | Addition of new Test Case 6.5.3.3 SCell activation and deactivation of unknown SCell | 16.5.0 |
| 2020-09 | RAN#89 | R5-203847 | 0813 | - | F | Update of 6.5.6.1.0 minimum requirement for DCI-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-203850 | 0816 | - | F | Update of 6.5.6.2.0 minimum requirement for RRC-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-203853 | 0819 | - | F | Correction to FR1 intra-frequency measurement with gap test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-203855 | 0821 | - | F | Update of Cell mapping in Annex E | 16.5.0 |
| 2020-09 | RAN#89 | R5-204034 | 0827 | - | F | CR to 38.533 on RRM Test Cases in clause 7.5.3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204036 | 0828 | - | F | CR to 38.533 on RRM Test Cases in clause 7.5.7 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204068 | 0829 | - | F | Correction to FR2 timing advance adjustment accuracy | 16.5.0 |
| 2020-09 | RAN#89 | R5-204069 | 0830 | - | F | Correction to EN-DC FR2 interruptions at transitions between active and non-active during DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204070 | 0831 | - | F | Addition of E-UTRA Test frequency selection rule for IRAT test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204071 | 0832 | - | F | Correction on event-triggered reporting with gap in DRX and DRX configuration settings for NR serving cell | 16.5.0 |
| 2020-09 | RAN#89 | R5-204072 | 0833 | - | F | Correction to EN-DC and SA FR1-FR1 SS-RSRP and SS-RSRQ periodical measurement accuracy | 16.5.0 |
| 2020-09 | RAN#89 | R5-204073 | 0834 | - | F | Correction to SS-RSRQ periodical measurement accuracy | 16.5.0 |
| 2020-09 | RAN#89 | R5-204075 | 0836 | - | F | Correction to message configuration for radio link monitoring tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204076 | 0837 | - | F | Correction to NR SA FR1 E-UTRA RRC connection release with redirection | 16.5.0 |
| 2020-09 | RAN#89 | R5-204077 | 0838 | - | F | Correction to SRS PeriodicityAndOffset for Timing Advance tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204079 | 0840 | - | F | Correction to CSI-RS for tracking for TDD FR2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204116 | 0841 | - | F | Correction of EN-DC FR2 SSB-based L1-RSRP measurement test case 5.6.3.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204118 | 0843 | - | F | Correction of EN-DC FR2 CSI-RS-based L1-RSRP measurement test case 5.6.3.3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204119 | 0844 | - | F | Correction of EN-DC FR2 CSI-RS-based L1-RSRP measurement test case 5.6.3.4 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204122 | 0847 | - | F | Correction of NR SA FR2 SSB-based L1-RSRP measurement test case 7.6.3.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204123 | 0848 | - | F | Correction of NR SA FR2 SSB-based L1-RSRP measurement test case 7.6.3.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204124 | 0849 | - | F | Correction of NR SA FR2 CSI-RS-based L1-RSRP measurement test case 7.6.3.3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204125 | 0850 | - | F | Correction of NR SA FR2 CSI-RS-based L1-RSRP measurement test case 7.6.3.4 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204126 | 0851 | - | F | Correction of minimum conformance requirements 4.6.4.0 for EN-DC FR1 L1-RSRP measurement test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204127 | 0852 | - | F | Correction of minimum conformance requirements 5.6.3.0 for EN-DC FR2 L1-RSRP measurement test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204128 | 0853 | - | F | Correction of minimum conformance requirements 7.6.3.0 for SA FR2 L1-RSRP measurement test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204220 | 0854 | - | F | Update missing SMTC and SSB configurations in H.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204221 | 0855 | - | F | Update missing SMTC and SSB configurations in H.3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204222 | 0856 | - | F | Editorial: correct table placement in H.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204285 | 0859 | - | F | Correction to Message Exceptions for 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | 16.5.0 |
| 2020-09 | RAN#89 | R5-204337 | 0874 | - | F | Addition of 4.5.5.0 minimum requirement for BFR | 16.5.0 |
| 2020-09 | RAN#89 | R5-204730 | 0792 | 1 | F | Update of PDSCH RMC in Annex A.1.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204733 | 0793 | 1 | F | Correction of RRM Test Case 6.1.1.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204734 | 0824 | 1 | F | Correction of RRM Test Case 6.1.1.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204735 | 0825 | 1 | F | Correction of RRM Test Case 7.1.1.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204736 | 0826 | 1 | F | Correction of RRM Test Case 7.1.1.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204775 | 0751 | 1 | F | Update TC 4.7.4.1.2 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204776 | 0753 | 1 | F | Update TC 4.7.4.2.2 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204777 | 0771 | 1 | F | Correction of measurement gap configuration for intra-frequency even-triggered tests needing gap | 16.5.0 |
| 2020-09 | RAN#89 | R5-204778 | 0755 | 1 | F | Update TC 6.7.4.1.2 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204779 | 0757 | 1 | F | Update TC 6.7.4.2.2 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204780 | 0762 | 1 | F | Correction to the test procedure of NR SA event triggered tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204781 | 0763 | 1 | F | Correction to the test procedure of NR SA event triggered tests inter-freq | 16.5.0 |
| 2020-09 | RAN#89 | R5-204782 | 0764 | 1 | F | Correction to the test procedure of NR SA event triggered tests inter-RAT | 16.5.0 |
| 2020-09 | RAN#89 | R5-204783 | 0749 | 1 | F | Update TC 8.5.2.3.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204784 | 0760 | 1 | F | Annex E and F update with TT results for 6.7.4.x.x NR SA L1-RSRP periodic reporting test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204785 | 0785 | 1 | F | Correction in A.1.4 CSI-RS for nzp-CSI-RS-ResourceId values | 16.5.0 |
| 2020-09 | RAN#89 | R5-204786 | 0820 | 1 | F | Update of FR1 Test Tolerance in Annex F | 16.5.0 |
| 2020-09 | RAN#89 | R5-204787 | 0823 | 1 | F | Correction to default configurations in Annex H | 16.5.0 |
| 2020-09 | RAN#89 | R5-204872 | 0857 | 1 | F | Correction to Test Procedure and sync to latest RAN4 in 4.4.1.1 EN-DC FR1 UE transmit timing accuracy | 16.5.0 |
| 2020-09 | RAN#89 | R5-204873 | 0858 | 1 | F | Correction to Test Procedure and sync to latest RAN4 in 6.4.1.1 NR SA FR1 UE transmit timing accuracy | 16.5.0 |
| 2020-09 | RAN#89 | R5-204874 | 0866 | 1 | F | Correction to 6.5.1.1 NR SA FR1 RLM OOS test with SSB-based RLM-RS in non-DRX mode, sync to latest RAN4 Core Spec | 16.5.0 |
| 2020-09 | RAN#89 | R5-204875 | 0867 | 1 | F | Correction to 6.5.1.2 NR SA FR1 RLM IS test with SSB-based RLM-RS in non-DRX mode, sync to latest RAN4 Core Spec | 16.5.0 |
| 2020-09 | RAN#89 | R5-204876 | 0868 | 1 | F | Correction to 6.5.1.3 NR SA FR1 RLM OOS test with SSB-based RLM-RS in DRX mode, sync to latest RAN4 Core Spec | 16.5.0 |
| 2020-09 | RAN#89 | R5-204877 | 0869 | 1 | F | Correction to 6.5.1.4 NR SA FR1 RLM IS test with SSB-based RLM-RS in DRX mode, sync to latest RAN4 Core Spec | 16.5.0 |
| 2020-09 | RAN#89 | R5-204878 | 0750 | 1 | F | Update TC 4.7.4.1.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204879 | 0752 | 1 | F | Update TC 4.7.4.2.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204880 | 0754 | 1 | F | Update TC 6.7.4.1.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204881 | 0756 | 1 | F | Update TC 6.7.4.2.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204882 | 0776 | 1 | F | Gap Pattern Id correction in section 6.6.2 to align to core spec 38.133 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204883 | 0787 | 1 | F | Test Procedure correction in test case 6.5.1.3 for definition of Point C | 16.5.0 |
| 2020-09 | RAN#89 | R5-204884 | 0789 | 1 | F | Addition of new RRM Test Cases in clause 7.5.5 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204885 | 0783 | 1 | F | CR to update the RRM FR2 MUs in Annex F | 16.5.0 |
| 2020-09 | RAN#89 | R5-204886 | 0784 | 1 | F | Add information on the AoA change during test for RRM FR2 tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204941 | 0835 | 1 | F | Correction to connection diagram for radio link monitoring tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204942 | 0842 | 1 | F | Correction of EN-DC FR2 SSB-based L1-RSRP measurement test case 5.6.3.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204943 | 0845 | 1 | F | Correction of FR2 condition tables in Annex B | 16.5.0 |
| 2020-09 | RAN#89 | R5-204944 | 0846 | 1 | F | Introduction of band n259 in clause 3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204985 | 0794 | 1 | F | Update of FR1 Test Tolerance in 4.5.5.1 SSB based BFR in non-DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204986 | 0795 | 1 | F | Update of FR1 Test Tolerance in 4.5.5.2 SSB based BFR in DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204987 | 0796 | 1 | F | Addition of new Test Case 4.5.5.3 CSI-RS based BFR non-DRX with FR1 Test Tolerance | 16.5.0 |
| 2020-09 | RAN#89 | R5-204988 | 0797 | 1 | F | Update of FR1 Test Tolerance in 4.5.5.4 CSI-RS based BFR in DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204989 | 0799 | 1 | F | Update of FR1 Test Tolerance in 4.5.6.1.1 DCI-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-204990 | 0800 | 1 | F | Update of FR1 Test Tolerance in 4.5.6.1.2 DCI-based BWP switch with SCell | 16.5.0 |
| 2020-09 | RAN#89 | R5-204991 | 0802 | 1 | F | Update of FR1 Test Tolerance in 4.5.6.2.1 RRC-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-204992 | 0809 | 1 | F | Update of FR1 Test Tolerance in 6.5.5.1 SSB based BFR in non-DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204993 | 0810 | 1 | F | Update of FR1 Test Tolerance in 6.5.5.2 SSB based BFR in DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204994 | 0811 | 1 | F | Update of FR1 Test Tolerance in 6.5.5.3 CSI-RS based BFR in non-DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204995 | 0812 | 1 | F | Update of FR1 Test Tolerance in 6.5.5.4 CSI-RS based BFR in DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204996 | 0814 | 1 | F | Update of FR1 Test Tolerance in 6.5.6.1.1 DCI-based BWP switch with SCell | 16.5.0 |
| 2020-09 | RAN#89 | R5-204997 | 0815 | 1 | F | Addition of new Test Case 6.5.6.1.2 DCI-based BWP switch with FR1 Test Tolerance | 16.5.0 |
| 2020-09 | RAN#89 | R5-204998 | 0817 | 1 | F | Update of FR1 Test Tolerance in 6.5.6.2.1 RRC-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-204999 | 0818 | 1 | F | Update of test parameters in 6.3.1.4 inter-RAT handover | 16.5.0 |
| 2020-09 | RAN#89 | R5-205000 | 0822 | 1 | F | Update of default configurations for RMC scheduling in Annex A | 16.5.0 |
| 2020-12 | RAN#90 | R5-205051 | 0879 | - | F | Message contents for RRM iRAT | 16.6.0 |
| 2020-12 | RAN#90 | R5-205059 | 0884 | - | F | Message contents for RRM PSCell addition | 16.6.0 |
| 2020-12 | RAN#90 | R5-205063 | 0885 | - | F | Update RRM 8.5.2.1.2 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-205068 | 0890 | - | F | Correction of the measObjectID, measID and reportConfigID in Annex H | 16.6.0 |
| 2020-12 | RAN#90 | R5-205070 | 0892 | - | F | Remove includeBeamMeasurements requirement for TC4.6.1.5 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205257 | 0907 | - | F | NR Band n53 addition to RRM group of bands | 16.6.0 |
| 2020-12 | RAN#90 | R5-205296 | 0914 | - | F | Add FR2 Fading MU values | 16.6.0 |
| 2020-12 | RAN#90 | R5-205813 | 0920 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205814 | 0921 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205815 | 0922 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.3 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205816 | 0923 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.4 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205817 | 0924 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.5 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205818 | 0925 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.6 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205819 | 0926 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.7 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205820 | 0927 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.8 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205821 | 0928 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205822 | 0929 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205823 | 0930 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.3 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205824 | 0931 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.4 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205826 | 0933 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.6 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205827 | 0934 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.7 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205828 | 0935 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.8 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205834 | 0939 | - | F | Correction of message content exceptions in Annex H | 16.6.0 |
| 2020-12 | RAN#90 | R5-205953 | 0946 | - | F | Update of FR1 TT in 8.4.1.1 inter-RAT SFTD in non-DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-205954 | 0947 | - | F | Update of FR1 TT in 8.4.1.2 inter-RAT SFTD in DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-205960 | 0953 | - | F | Addition of test case 5.5.1.9 EN-DC FR2 radio link monitoring UE scheduling restrictions | 16.6.0 |
| 2020-12 | RAN#90 | R5-205961 | 0954 | - | F | Update of test case 5.5.3.1 EN-DC FR2 SCell activation and deactivation | 16.6.0 |
| 2020-12 | RAN#90 | R5-205963 | 0956 | - | F | Update of 7.3.2.1.3 SA FR2 RRC re-establishment without serving cell timing | 16.6.0 |
| 2020-12 | RAN#90 | R5-205967 | 0960 | - | F | Update of Annex E for cell mapping | 16.6.0 |
| 2020-12 | RAN#90 | R5-205975 | 0964 | - | F | Correction to default configuration in Annex H | 16.6.0 |
| 2020-12 | RAN#90 | R5-205976 | 0965 | - | F | Correction to SSB reference configuration | 16.6.0 |
| 2020-12 | RAN#90 | R5-206100 | 0968 | - | F | Correction to EN-DC FR1 interruptions at transitions between active and non-active during DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-206102 | 0970 | - | F | Correction to FR1 NSA SS-SINR measurement TCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-206105 | 0973 | - | F | Correction to message exception for inter-RAT reselection TCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-206106 | 0974 | - | F | Editorial correction of RSRP tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206111 | 0976 | - | F | Update of Annex C.1.3 of default connection setup | 16.6.0 |
| 2020-12 | RAN#90 | R5-206156 | 0977 | - | F | Addition of missing PRACH configuration 4 for FR1 and FR2 applicability | 16.6.0 |
| 2020-12 | RAN#90 | R5-206214 | 0979 | - | F | Alignment test parameter table with TS 38.133 in SA cell reselection test case | 16.6.0 |
| 2020-12 | RAN#90 | R5-206215 | 0980 | - | F | Alignment test parameter table with TS 38.133 in SA handover test case | 16.6.0 |
| 2020-12 | RAN#90 | R5-206216 | 0981 | - | F | Addition RLM IE into event trigger reporting CSI-RS based RLM | 16.6.0 |
| 2020-12 | RAN#90 | R5-206217 | 0982 | - | F | Addition RLM IE into event trigger reporting SSB based | 16.6.0 |
| 2020-12 | RAN#90 | R5-206219 | 0984 | - | F | Update message content section in SS-SINR test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206221 | 0985 | - | F | Update to Section 5.4.1.1 NSA FR2 Transmit Timing Accuracy | 16.6.0 |
| 2020-12 | RAN#90 | R5-206222 | 0986 | - | F | Update to Section 5.4.3.1 NSA FR2 Timing Advance Accuracy | 16.6.0 |
| 2020-12 | RAN#90 | R5-206606 | 0998 | - | F | Update of CSI-RS RMC configuration in Annex A | 16.6.0 |
| 2020-12 | RAN#90 | R5-206672 | 0918 | 1 | F | Editorial CR to clarify SS-RSRP dBm / 15 kHz for PRACH tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206673 | 0941 | 1 | F | Correction of RRM TC 4.7.2.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206674 | 0943 | 1 | F | Update of message contents for EN-DC RLM test case | 16.6.0 |
| 2020-12 | RAN#90 | R5-206675 | 0969 | 1 | F | Correction to FR1 Inter frequency Event triggered Reporting test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206677 | 0987 | 1 | F | Update to Message Exceptions for 4.3.2.2.2 EN-DC Non Contention Based Random Access | 16.6.0 |
| 2020-12 | RAN#90 | R5-206678 | 0989 | 1 | F | Update to 4.5.1.1 NSA FR1 OOS RLM to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206679 | 0990 | 1 | F | Update to 4.5.1.2 NSA FR1 IS RLM to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206680 | 0991 | 1 | F | Update to 4.5.1.3 NSA FR1 OOS RLM in DRX to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206681 | 0992 | 1 | F | Update to 4.5.1.4 NSA FR1 IS RLM in DRX to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206682 | 0993 | 1 | F | Update to 6.5.1.1 SA FR1 OOS RLM to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206683 | 0994 | 1 | F | Update to 6.5.1.2 SA FR1 IS RLM to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206684 | 0995 | 1 | F | Update to 6.5.1.3 SA FR1 OOS RLM in DRX to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206685 | 0996 | 1 | F | Update to 6.5.1.4 SA FR1 IS RLM in DRX to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206686 | 0952 | 1 | F | Update of 5.5.1 EN-DC FR2 radio link monitoring | 16.6.0 |
| 2020-12 | RAN#90 | R5-206687 | 0955 | 1 | F | Update of 5.5.5 EN-DC FR2 Beam failure detection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206688 | 0891 | 1 | F | Corrections to re-selection test procedure | 16.6.0 |
| 2020-12 | RAN#90 | R5-206689 | 0896 | 1 | F | Correction of RRM TC 6.1.1.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206690 | 0897 | 1 | F | Correction of RRM TC 6.1.2.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206691 | 0899 | 1 | F | Correction of RRM TC 6.3.2.2.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206692 | 0910 | 1 | F | Correction of RRM TC 6.5.5.3 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206693 | 0911 | 1 | F | Correction of RRM TC 6.5.5.4 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206694 | 0919 | 1 | F | Editorial CR to clarify SS-RSRP dBm / 15 kHz for SA PRACH tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206695 | 0942 | 1 | F | Correction of RRM TC 6.7.2.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206696 | 0961 | 1 | F | Update of FR1 TT in 6.1.1.1 intra-freq re-selection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206697 | 0962 | 1 | F | Correction of clause 6.1 cell re-selection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206698 | 0967 | 1 | F | Correction to NR SA RLM out-of-sync test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206699 | 0983 | 1 | F | Clarification in message exception for SA RLM out of synch test cases in non DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-206700 | 0900 | 1 | F | Correction of RRM TC 7.1.1.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206701 | 0932 | 1 | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.5 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206702 | 0936 | 1 | F | Correction of NR SA FR2 RRC re-establishment test case 7.3.2.1.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206703 | 0937 | 1 | F | Correction of NR SA FR2-FR2 RRC re-establishment test case 7.3.2.1.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206704 | 0957 | 1 | F | Update of clause 7.5.5 SA FR2 Beam failure detection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206705 | 0958 | 1 | F | Update of clause 7.6.1 SA FR2 Intra-frequency measurements | 16.6.0 |
| 2020-12 | RAN#90 | R5-206706 | 0889 | 1 | F | Correction of the message contents for RRM re-selection test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206707 | 0917 | 1 | F | Correction to the aggregation level in the CORESET for RMC scheduling | 16.6.0 |
| 2020-12 | RAN#90 | R5-206708 | 0938 | 1 | F | Correction of clause 3.3 Abbreviations | 16.6.0 |
| 2020-12 | RAN#90 | R5-206709 | 0963 | 1 | F | Update of Annex H for cell re-selection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206797 | 0882 | 1 | F | Complete RRM 4.5.7.1 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206798 | 0875 | 1 | F | Complete RRM 6.7.5.1 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206799 | 0916 | 1 | F | Add FR2 PRACH Test cases 7.3.2.2.1 and 7.3.2.2.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206800 | 0944 | 1 | F | Update of FR1 TT in 8.2.1.1 inter-RAT cell re-selection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206801 | 0945 | 1 | F | Update of FR1 TT in 8.3.1.1 inter-RAT handover | 16.6.0 |
| 2020-12 | RAN#90 | R5-206802 | 0948 | 1 | F | Update of FR1 TT in 8.4.2.1 inter-RAT event-triggered reporting without SSB time index in non-DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-206803 | 0949 | 1 | F | Update of FR1 TT in 8.4.2.2 inter-RAT event-triggered reporting without SSB time index in DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-206804 | 0950 | 1 | F | Update of FR1 TT in 8.4.2.3 inter-RAT event-triggered reporting with SSB time index in non-DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-206805 | 0951 | 1 | F | Update of FR1 TT in 8.4.2.4 inter-RAT event-triggered reporting with SSB time index in DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-206806 | 0878 | 1 | F | Annex E and F iRAT measurement accuracy tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206807 | 0888 | 1 | F | Annex E and F iRAT FR2 measurement accuracy tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206808 | 0959 | 1 | F | Update of Annex F for Test Tolerance | 16.6.0 |
| 2020-12 | RAN#90 | R5-206834 | 0915 | 1 | F | Update FR2 Downlink and Es/Noc MU values | 16.6.0 |
| 2020-12 | RAN#90 | R5-206843 | 0876 | 1 | F | Complete RRM 6.7.6.1 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206844 | 0886 | 1 | F | Update RRM 8.5.2.2.2 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206871 | 0894 | 1 | F | Correction of RRM TC 4.3.2.2.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206872 | 0905 | 1 | F | New DRX configuration for RLM TC | 16.6.0 |
| 2020-12 | RAN#90 | R5-206895 | 0971 | 1 | F | Correction to message configuration for NSA radio link monitoring CSI-RS Based in DRX mode tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206906 | 0880 | 1 | F | Complete RRM 4.7.5.1 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206907 | 0877 | 1 | F | Complete RRM 6.7.7.1 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206908 | 0972 | 1 | F | Correction to message configuration for SA radio link monitoring CSI RS Based RLM-RS | 16.6.0 |
| 2020-12 | RAN#90 | R5-206909 | 0887 | 1 | F | Update RRM 8.5.2.3.2 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206910 | 0881 | 1 | F | Annex E and F 4.7.5.1 SFTD measurement accuracy test | 16.6.0 |
| 2021-03 | RAN#91 | R5-210134 | 0999 | - | F | Update of first preamble power for EN-DC TC 4.3.2.2.1 and 4.3.2.2.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210135 | 1000 | - | F | Update of first preamble power for SA TC 6.3.2.2.1 and 6.3.2.2.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210136 | 1001 | - | F | Update of UE initial state for SA RLM TC 6.5.1.x | 16.7.0 |
| 2021-03 | RAN#91 | R5-210137 | 1002 | - | F | Update of process delay for SFTD measurement TC 8.4.1.x | 16.7.0 |
| 2021-03 | RAN#91 | R5-210138 | 1003 | - | F | Update of cell frequency for TC 6.7.2.2.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210171 | 1004 | - | F | Editorial update of SS-RSRP for TC 6.7.1.2.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210173 | 1006 | - | F | Update of RRC message for TC 6.4.3.1 and 6.5.2.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210176 | 1009 | - | F | Update of SCell activation and CSI reporting time for EN-DC TC 5.5.3.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210177 | 1010 | - | F | Update of CSI reporting time for SA TC 7.5.3.1 and 7.5.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210178 | 1011 | - | F | Update of PRACH configuration for SA TC 7.3.2.2.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210179 | 1012 | - | F | Update of PRACH configuration for EN-DC TC 4.5.5.3 and 4.5.5.4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210180 | 1013 | - | F | Update of PRACH configuration for EN-DC TC 5.5.5.x | 16.7.0 |
| 2021-03 | RAN#91 | R5-210181 | 1014 | - | F | Update of PRACH configuration for SA TC 6.5.5.3 and 6.5.5.4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210182 | 1015 | - | F | Update of PRACH configuration for SA TC 7.5.5.x | 16.7.0 |
| 2021-03 | RAN#91 | R5-210183 | 1016 | - | F | Update of prach-ConfigurationIndex for FR1 PRACH configuration 4 in A.7 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210279 | 1017 | - | F | Addition of minimum conformance requirements for R16 NR HST cell re-selection in 6.1.1.0 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210281 | 1019 | - | F | Addition of minimum conformance requirements for R16 NR HST measurement in 6.6.1.0 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210283 | 1021 | - | F | Update of Annex H.2.1-3 for R16 NR HST | 16.7.0 |
| 2021-03 | RAN#91 | R5-210434 | 1025 | - | F | Correct cell mapping for 5.4.3.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210437 | 1027 | - | F | Annex E and F 4.5.7.1 PSCell addition test | 16.7.0 |
| 2021-03 | RAN#91 | R5-210440 | 1029 | - | F | Update of TT analysis results in Annex F for FR2 iRAT test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210442 | 1030 | - | F | Update TT results for SS-RSRP measurement accuracy test cases chapter 4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210443 | 1031 | - | F | Update TT results for SS-RSRP measurement accuracy test cases chapter 6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210444 | 1032 | - | F | Update TT results for SS-RSRP measurement accuracy test cases Annex F | 16.7.0 |
| 2021-03 | RAN#91 | R5-210446 | 1033 | - | F | Update TT results for SS-RSRQ measurement accuracy test cases chapter 4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210447 | 1034 | - | F | Update TT results for SS-RSRQ measurement accuracy test cases chapter 6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210448 | 1035 | - | F | Update TT results for SS-RSRQ measurement accuracy test cases Annex F | 16.7.0 |
| 2021-03 | RAN#91 | R5-210456 | 1039 | - | F | Update Annex I to TS 38.533 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210457 | 1040 | - | F | Update NR frequency band groups for FR1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210458 | 1041 | - | F | Correction RLM config for event triggered test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210465 | 1046 | - | F | Editorial correction of title of clause 6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210466 | 1047 | - | F | Corrections to 6.3.2.2.1 and 6.3.2.2.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210467 | 1048 | - | F | Corrections to 4.3.2.2.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210470 | 1049 | - | F | Clarification on SSB Index to use in the PUxCH-PowerControl for RRM tests with more than one SSB | 16.7.0 |
| 2021-03 | RAN#91 | R5-210471 | 1050 | - | F | Clarification on SSB Index to use in the PUxCH-PowerControl for RRM tests with more than one SSB - EN-DC | 16.7.0 |
| 2021-03 | RAN#91 | R5-210472 | 1051 | - | F | Editorial: correct title of PRACH test cases to match RRM work plan - EN-DC FR1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210473 | 1052 | - | F | Editorial: correct title of PRACH test cases to match RRM work plan - SA FR1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210474 | 1053 | - | F | Editorial: correct title of PRACH test cases to match RRM work plan - SA FR2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210475 | 1054 | - | F | Editorial: correct title of PRACH test cases to match RRM work plan - Annexes | 16.7.0 |
| 2021-03 | RAN#91 | R5-210507 | 1056 | - | F | Correction of test applicability for long DRX cycle related test cases in section 4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210508 | 1057 | - | F | Correction of test applicability for long DRX cycle related test cases in section 5 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210509 | 1058 | - | F | Correction of test applicability for long DRX cycle related test cases in section 6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210510 | 1059 | - | F | Correction of test applicability for long DRX cycle related test cases in section 7 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210511 | 1060 | - | F | Correction of test applicability for long DRX cycle related test cases in section 8 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210528 | 1062 | - | F | Correction to test procedure in test case 6.5.1.4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210529 | 1063 | - | F | Correction to Interruptions during measurements on deactivated NR SCC test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210530 | 1064 | - | F | Correction to message configuration for NSA CSI-RS-based RLM-RS test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210531 | 1065 | - | F | Correction to NR SA RLM out-of-sync test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210532 | 1066 | - | F | Correction to NR SA FR1 RRC Re-establishment test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210533 | 1067 | - | F | Correction to 4.5.1.2 and 6.5.1.2 PDCCH Aggregation Level | 16.7.0 |
| 2021-03 | RAN#91 | R5-210535 | 1069 | - | F | Correction to FR1 NSA SS-SINR measurement test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210536 | 1070 | - | F | Correction to L1-RSRP test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210537 | 1071 | - | F | Correction to the procedure to add a step for establishing SRB2 and DRB | 16.7.0 |
| 2021-03 | RAN#91 | R5-210538 | 1072 | - | F | Update of DRX configuration in FR1 Event-triggered Test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210601 | 1073 | - | F | Clarification of BWP1 and BWP2 in 6.6.1.3, 6.6.1.4, 6.6.1.6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210608 | 1077 | - | F | Correction in 4.5.2.1 and 4.5.2.2 test procedure | 16.7.0 |
| 2021-03 | RAN#91 | R5-210613 | 1079 | - | F | Correction in SIB5 for iRAT cell reselection | 16.7.0 |
| 2021-03 | RAN#91 | R5-210614 | 1080 | - | F | Correction in 6.1.2.2 test procedure | 16.7.0 |
| 2021-03 | RAN#91 | R5-210814 | 1085 | - | F | Finalise FR2 Timing MU values | 16.7.0 |
| 2021-03 | RAN#91 | R5-210816 | 1086 | - | F | Update of FR2 Tx Timing Test case 5.4.1.1 Test Tolerances | 16.7.0 |
| 2021-03 | RAN#91 | R5-210851 | 1089 | - | F | Update of FR1 TT for 4.5.5.1 and 4.5.5.2 SSB based LR | 16.7.0 |
| 2021-03 | RAN#91 | R5-210852 | 1090 | - | F | Update of FR1 TT for 4.5.5.3 and 4.5.5.4 CSI-RS based LR | 16.7.0 |
| 2021-03 | RAN#91 | R5-210853 | 1091 | - | F | Correction to 4.6.4.3 and 4.6.4.4 L1-RSRP reporting delay | 16.7.0 |
| 2021-03 | RAN#91 | R5-210857 | 1095 | - | F | Update of FR1 TT for 6.5.5.1 and 6.5.5.2 SSB based LR | 16.7.0 |
| 2021-03 | RAN#91 | R5-210858 | 1096 | - | F | Update of FR1 TT for 6.5.5.3 and 6.5.5.4 CSI-RS based LR | 16.7.0 |
| 2021-03 | RAN#91 | R5-210860 | 1098 | - | F | Correction to 6.6.4.3 and 6.6.4.4 L1-RSRP reporting delay | 16.7.0 |
| 2021-03 | RAN#91 | R5-210863 | 1101 | - | F | Update of 6.5.3.1 SCell activation and deactivation | 16.7.0 |
| 2021-03 | RAN#91 | R5-210864 | 1102 | - | F | Update of D.4 antenna configuration | 16.7.0 |
| 2021-03 | RAN#91 | R5-210866 | 1104 | - | F | Correction to CSI-RS RMC | 16.7.0 |
| 2021-03 | RAN#91 | R5-210867 | 1105 | - | F | Correction to default configuration on L1-RSRP reporting in Annex H | 16.7.0 |
| 2021-03 | RAN#91 | R5-210959 | 1113 | - | F | Update to 6.5.4.1 NR SA FR1 UE UL carrier RRC reconfiguration delay | 16.7.0 |
| 2021-03 | RAN#91 | R5-211049 | 1116 | - | F | Additional of default downlink level for FR2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211219 | 1124 | - | F | Correction of the minimum conformance requirements for Inter-RAT measurements 6.6.3.0 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211220 | 1125 | - | F | Addition of new RRM iRAT measurement Inter-RAT measurements test case 6.6.3.3 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211255 | 1131 | - | F | Clarification of BWP1 and BWP2 in 4.6.1.3, 4.6.1.4, 4.6.1.6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211614 | 1007 | 1 | F | Update of SCell activation and CSI reporting time for EN-DC TC 4.5.3.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211615 | 1026 | 1 | F | Complete RRM 4.5.7.1 including TT analysis results | 16.7.0 |
| 2021-03 | RAN#91 | R5-211616 | 1087 | 1 | F | Update Test Tolerance for FR1 RLM Test Cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211617 | 1092 | 1 | F | Correction to EN-DC radio link monitoring | 16.7.0 |
| 2021-03 | RAN#91 | R5-211618 | 1093 | 1 | F | Update of 4.5.3.1 SCell activation and deactivation | 16.7.0 |
| 2021-03 | RAN#91 | R5-211619 | 1108 | 1 | F | Update to 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay | 16.7.0 |
| 2021-03 | RAN#91 | R5-211620 | 1109 | 1 | F | Update to 4.7.5.1 EN-DC FR1 SFTD measurement accuracy | 16.7.0 |
| 2021-03 | RAN#91 | R5-211621 | 1117 | 1 | F | Correction of applied TT for EN-DC FR1 L1-RSRP measurement test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211622 | 1023 | 1 | F | Update 5.4.3.1 with TT analysis results | 16.7.0 |
| 2021-03 | RAN#91 | R5-211623 | 1119 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.1 including TT | 16.7.0 |
| 2021-03 | RAN#91 | R5-211624 | 1120 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.3 including TT | 16.7.0 |
| 2021-03 | RAN#91 | R5-211625 | 1008 | 1 | F | Update of CSI reporting time for SA TC 6.5.3.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211626 | 1094 | 1 | F | Correction to SA radio link monitoring | 16.7.0 |
| 2021-03 | RAN#91 | R5-211627 | 1110 | 1 | F | Update to 6.3.2.2.1 Contention based random access test in FR1 for NR standalone | 16.7.0 |
| 2021-03 | RAN#91 | R5-211628 | 1111 | 1 | F | Update to 6.3.2.2.2 Non-Contention based random access test in FR1 for NR standalone | 16.7.0 |
| 2021-03 | RAN#91 | R5-211629 | 1112 | 1 | F | Update to 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC | 16.7.0 |
| 2021-03 | RAN#91 | R5-211630 | 1114 | 1 | F | Update to 6.6.3.1 NR SA FR1 - E-UTRAN event-triggered reporting in non-DRX | 16.7.0 |
| 2021-03 | RAN#91 | R5-211631 | 1115 | 1 | F | Update to 6.6.3.2 NR SA FR1 - E-UTRAN event-triggered reporting in DRX | 16.7.0 |
| 2021-03 | RAN#91 | R5-211632 | 1118 | 1 | F | Correction of applied TT for SA FR1 L1-RSRP measurement test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211633 | 1121 | 1 | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.1 including TT | 16.7.0 |
| 2021-03 | RAN#91 | R5-211634 | 1122 | 1 | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.3 including TT | 16.7.0 |
| 2021-03 | RAN#91 | R5-211635 | 1028 | 1 | F | Complete FR2 iRAT measurement accuracy test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211636 | 1024 | 1 | F | Update Annex F for 5.4.3.1 and 7.4.3.1 with TT analysis results | 16.7.0 |
| 2021-03 | RAN#91 | R5-211637 | 1038 | 1 | F | Update DL AWGN MU for RRM FR2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211638 | 1084 | 1 | F | Update FR2 Downlink and Uplink MU values | 16.7.0 |
| 2021-03 | RAN#91 | R5-211639 | 1103 | 1 | F | Update of Annex F for Test Tolerance | 16.7.0 |
| 2021-03 | RAN#91 | R5-211640 | 1123 | 1 | F | Correction of MTSU and applied TT in Annex F | 16.7.0 |
| 2021-03 | RAN#91 | R5-211641 | 1130 | 1 | F | Addition of new RMCs and OCNGs into Annex A | 16.7.0 |
| 2021-03 | RAN#91 | R5-211722 | 1042 | 1 | F | Correction EN-DC FR1 timing tests | 16.7.0 |
| 2021-03 | RAN#91 | R5-211723 | 1043 | 1 | F | Corrections to 5.4.1.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211724 | 1061 | 1 | F | Clarification of SNR for 4RX UE in RLM Test Cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211725 | 1076 | 1 | F | Correction in 4.7.3.2.1 test parameters | 16.7.0 |
| 2021-03 | RAN#91 | R5-211726 | 1005 | 1 | F | Update of RRC message for TC 6.3.1.4, 6.3.1.5 and 6.3.2.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211727 | 1045 | 1 | F | Corrections NR SA FR1 timing test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211728 | 1097 | 1 | F | Correction to NR TC 6.6.3.2-Inter-RAT DRX | 16.7.0 |
| 2021-03 | RAN#91 | R5-211729 | 1099 | 1 | F | Update of 6.5.1.2 and 6.5.1.4 SSB based RLM | 16.7.0 |
| 2021-03 | RAN#91 | R5-211730 | 1083 | 1 | F | Clarification of BWP1 and BWP2 in 7.6.1.3 and 7.6.1.4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211731 | 1044 | 1 | F | Addition of Serving Cell Config for RRM timing test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211827 | 1018 | 1 | F | Addition of new test case 6.1.1.7 for R16 NR HST | 16.7.0 |
| 2021-03 | RAN#91 | R5-211828 | 1020 | 1 | F | Addition of new test case 6.6.1.7 for R16 NR HST | 16.7.0 |
| 2021-03 | RAN#91 | R5-211829 | 1022 | 1 | F | Addition of cell configuration mapping in Annex E for R16 NR HST | 16.7.0 |
| 2021-03 | RAN#91 | R5-211830 | 1126 | 1 | F | Correction of the minimum conformance requirements for E-UTRA - NR FR1 Cell reselection 8.2.1.0 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211831 | 1127 | 1 | F | Addition of new RRM E-UTRA - NR FR1 Cell reselection test case 8.2.1.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211832 | 1128 | 1 | F | Correction of the minimum conformance requirements for E-UTRA - NR Inter-RAT event triggered reporting tests 8.4.2.0 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211833 | 1129 | 1 | F | Addition of new RRM E-UTRA - NR Inter-RAT event triggered reporting test case 8.4.2.9 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211889 | 1036 | 1 | F | Update TT results for 6.3.1.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211890 | 1037 | 1 | F | Update TT results for 6.3.1.1 Annex F | 16.7.0 |
| 2021-06 | RAN#92 | R5-212024 | 1132 | - | F | Complete 5.7.1.1 with TT analysis results | 16.8.0 |
| 2021-06 | RAN#92 | R5-212025 | 1133 | - | F | Complete 5.7.2.1 with TT analysis results | 16.8.0 |
| 2021-06 | RAN#92 | R5-212026 | 1134 | - | F | Complete 5.7.3.1 with TT analysis results | 16.8.0 |
| 2021-06 | RAN#92 | R5-212256 | 1138 | - | F | Core alignment of TCI state of CSI-RS for TRS | 16.8.0 |
| 2021-06 | RAN#92 | R5-212257 | 1139 | - | F | Introducing additional band for n38 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212265 | 1147 | - | F | Updates to DRX configuration in FR1 Event-triggered Test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212266 | 1148 | - | F | Core alignment of BWP configuration parameter in 6.5.2.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212267 | 1149 | - | F | Correction to CSI-RS based L1-RSRP measurement tests | 16.8.0 |
| 2021-06 | RAN#92 | R5-212612 | 1153 | - | F | Addition of new test case 6.6.4.5 for R16 NR HST | 16.8.0 |
| 2021-06 | RAN#92 | R5-212613 | 1154 | - | F | Addition of cell configuration mapping in Annex E for TC 6.6.4.5 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212712 | 1158 | - | F | Remove obsolete message contents tables after Chapter 7 alignment | 16.8.0 |
| 2021-06 | RAN#92 | R5-212715 | 1161 | - | F | Correcting measurement accuracy iRAT test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212716 | 1162 | - | F | Corrections to 5.4.1.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212717 | 1163 | - | F | Corrections to 5.4.3.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212718 | 1164 | - | F | Corrections to 4.4.1.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212719 | 1165 | - | F | Corrections to 4.4.3.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212720 | 1166 | - | F | Corrections to 6.4.1.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212721 | 1167 | - | F | Corrections to 6.4.3.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212726 | 1171 | - | F | Correct applicability of Interruption EN-DC test cases with CA | 16.8.0 |
| 2021-06 | RAN#92 | R5-212727 | 1172 | - | F | Correct applicability of SCell activation EN-DC test cases with CA | 16.8.0 |
| 2021-06 | RAN#92 | R5-212728 | 1173 | - | F | Correct applicability of Interruption SA test cases with CA | 16.8.0 |
| 2021-06 | RAN#92 | R5-212729 | 1174 | - | F | Correct applicability of SCell activation SA test cases with CA | 16.8.0 |
| 2021-06 | RAN#92 | R5-212731 | 1175 | - | F | Addition of RRM FR2 EN-DC PRACH test case 5.3.2.2.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212732 | 1176 | - | F | Addition of RRM FR2 EN-DC PRACH test case 5.3.2.2.2 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212737 | 1177 | - | F | Update to test applicability of RRM TC6.5.4.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212745 | 1178 | - | F | Update to RRM Antenna connection for 4Rx capable Ues | 16.8.0 |
| 2021-06 | RAN#92 | R5-212895 | 1180 | - | F | Correction to FR1 EN-DC interruption during SCell measurement test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212896 | 1181 | - | F | Correction to FR1 EN-DC beam failure recovery test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212900 | 1185 | - | F | Correction to FR1 NR SA TC 6.5.3.1-SCell activation | 16.8.0 |
| 2021-06 | RAN#92 | R5-212901 | 1186 | - | F | Correction to FR1 NR SA beam failure recovery test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212904 | 1189 | - | F | Correction to RMSI CORESET reference configuration in Annex A | 16.8.0 |
| 2021-06 | RAN#92 | R5-212905 | 1190 | - | F | Correction to CSI-RS reference configuration in annex A | 16.8.0 |
| 2021-06 | RAN#92 | R5-212906 | 1191 | - | F | Correction to TRS reference configuration in annex A | 16.8.0 |
| 2021-06 | RAN#92 | R5-212909 | 1194 | - | F | Correction to BWP reference configuration in Annex A | 16.8.0 |
| 2021-06 | RAN#92 | R5-212910 | 1195 | - | F | Correction of minimum requirements for L1-RSRP reporting test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212911 | 1196 | - | F | Addition of FR1 EN-DC TC 4.6.4.5-HST L1-RSRP | 16.8.0 |
| 2021-06 | RAN#92 | R5-213183 | 1205 | - | F | Extension of Annex I for EIS spherical coverage points | 16.8.0 |
| 2021-06 | RAN#92 | R5-213227 | 1212 | - | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.5 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213228 | 1213 | - | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.6 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213229 | 1214 | - | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.7 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213230 | 1215 | - | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.8 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213240 | 1219 | - | F | Test frequency selection for Inter-frequency RRM test cases in Annex E | 16.8.0 |
| 2021-06 | RAN#92 | R5-213345 | 1222 | - | F | Update on iRAT measurement 6.6.3.2 test case | 16.8.0 |
| 2021-06 | RAN#92 | R5-213346 | 1223 | - | F | Update on Annex H.3.1 for iRAT measurement | 16.8.0 |
| 2021-06 | RAN#92 | R5-213351 | 1224 | - | F | Correction to Coreset and Search Space RMC for FR2 RRM | 16.8.0 |
| 2021-06 | RAN#92 | R5-213352 | 1225 | - | F | Correction to Message Exceptions for 6.3.2.2.2 Non-contention based random access test in FR1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-213353 | 1226 | - | F | Correction to PDSCH RMC when DRX is configured | 16.8.0 |
| 2021-06 | RAN#92 | R5-213354 | 1227 | - | F | Correction to intra and inter-frequency reselection test cases with asynchronous cells | 16.8.0 |
| 2021-06 | RAN#92 | R5-213847 | 1179 | 1 | F | Addition of inter-frequency measurements configuration for FR1 EN-DC BFD and RLM test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-213848 | 1183 | 1 | F | Addition of inter-frequency measurements configuration for FR1 NR SA BFD and RLM test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-213849 | 1184 | 1 | F | Correction to FR1 NR SA interruption during SCell measurement test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-213850 | 1193 | 1 | F | Correction to PRACH reference configuration in Annex H | 16.8.0 |
| 2021-06 | RAN#92 | R5-213921 | 1145 | 1 | F | Correction to test procedure for SSB-based L1-RSRP measurement reporting | 16.8.0 |
| 2021-06 | RAN#92 | R5-213922 | 1159 | 1 | F | Remove obsolete message contents tables after Chapter 7 alignment - EN-DC | 16.8.0 |
| 2021-06 | RAN#92 | R5-213923 | 1168 | 1 | F | Corrections to RLM test cases EN-DC | 16.8.0 |
| 2021-06 | RAN#92 | R5-213924 | 1141 | 1 | F | Correction to FR2 RRM test channel reference for Event triggered reporting test | 16.8.0 |
| 2021-06 | RAN#92 | R5-213925 | 1142 | 1 | F | Correction to FR2 RRM test channel reference for RLM and BFD tests | 16.8.0 |
| 2021-06 | RAN#92 | R5-213926 | 1143 | 1 | F | Correction to FR2 RRM test channel reference for UL timing test | 16.8.0 |
| 2021-06 | RAN#92 | R5-213927 | 1208 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.5 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213928 | 1209 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.6 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213929 | 1210 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.7 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213930 | 1211 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.8 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213931 | 1156 | 1 | F | Correct reference to cell mapping for SA test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-213932 | 1160 | 1 | F | Correcting typos in PRACH test case | 16.8.0 |
| 2021-06 | RAN#92 | R5-213933 | 1169 | 1 | F | Corrections to RLM test cases SA | 16.8.0 |
| 2021-06 | RAN#92 | R5-213934 | 1135 | 1 | F | Annex F for EN-DC and SA FR2 Measurement Accuracy | 16.8.0 |
| 2021-06 | RAN#92 | R5-213935 | 1157 | 1 | F | Align Annex H of TS 38.533 to Clause 7 of TS 38.508-1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-213936 | 1202 | 1 | F | Correction to AoA Test Setup applicability per permitted test method | 16.8.0 |
| 2021-06 | RAN#92 | R5-213937 | 1204 | 1 | F | Adapt Annex F to indicate applicability of the RRM FR2 TT analysis results | 16.8.0 |
| 2021-06 | RAN#92 | R5-213938 | 1207 | 1 | F | Correction of MTSU and applied TT in Annex F for FR2 inter-freq measurement test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-214009 | 1151 | 1 | F | Addition of 5G-SRVCC RRM TC 6.3.1.6-handover | 16.8.0 |
| 2021-06 | RAN#92 | R5-214010 | 1152 | 1 | F | Addition of 5G-SRVCC RRM TC 6.6.5.1-envent triggered reporting non-DRX | 16.8.0 |
| 2021-06 | RAN#92 | R5-214060 | 1182 | 1 | F | Correction to FR1 EN-DC BWP switching test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-214061 | 1198 | 1 | F | Update of CSI reporting time for EN-DC FR1 TC 4.5.3.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-214062 | 1199 | 1 | F | Update of CSI reporting time for EN-DC FR2 TC 5.5.3.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-214063 | 1203 | 1 | F | Adapt RRM FR2 test cases to indicate applicability of the TT analysis results | 16.8.0 |
| 2021-06 | RAN#92 | R5-214064 | 1146 | 1 | F | Corrections to NR SA FR1 RRC Re-establishment tests | 16.8.0 |
| 2021-06 | RAN#92 | R5-214065 | 1187 | 1 | F | Correction to FR1 NR SA BWP switch test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-214066 | 1200 | 1 | F | Update of CSI reporting time for SA FR1 TC 6.5.3.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-214067 | 1201 | 1 | F | Update of CSI reporting time for SA FR2 TC 7.5.3.1 and 7.5.3.2 | 16.8.0 |
| 2021-06 | RAN#92 | R5-214068 | 1188 | 1 | F | Correction to NR inter-RAT measurement test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-214090 | 1197 | 1 | F | Correction to Annex A.9 on the selection of test direction | 16.8.0 |
| 2021-06 | RAN#92 | R5-214091 | 1206 | 1 | F | Extension of the RSRPB Rx Beak Peak Search method | 16.8.0 |
| 2021-06 | RAN#92 | R5-214092 | 1221 | 1 | F | Update on test frequencies selection for inter-frequency test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-214108 | 1216 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.1 and 5.6.2.3 | 16.8.0 |
| 2021-06 | RAN#92 | R5-214109 | 1217 | 1 | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.1 and 7.6.2.3 | 16.8.0 |
| 2021-06 | RAN#92 | R5-214111 | 1144 | 1 | F | Correction to Interruptions during measurements on deactivated NR SCC | 16.8.0 |
| 2021-06 | RAN#92 | R5-214113 | 1140 | 1 | F | Correction to FR2 event-triggered reporting of intra-frequency cell | 16.8.0 |
| 2021-09 | RAN#93 | R5-214195 | 1232 | - | F | Completion 7.7.2.2 including TT analysis results | 16.9.0 |
| 2021-09 | RAN#93 | R5-214197 | 1234 | - | F | Annex E and F update for FR2 inter-frequency periodic measurements tests | 16.9.0 |
| 2021-09 | RAN#93 | R5-214198 | 1235 | - | F | Completion 7.7.1.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214199 | 1236 | - | F | Completion 7.7.2.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214200 | 1237 | - | F | Completion 7.7.3.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214339 | 1239 | - | F | Correction to 4.5.1.1 message contents and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214342 | 1242 | - | F | Correction to 4.5.1.4 core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214343 | 1243 | - | F | Correction to 4.5.1.5 message contents and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214345 | 1245 | - | F | Correction to 4.4.1.1 and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214347 | 1247 | - | F | Editorial corrections and core spec alignment for 5.7.1.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214348 | 1248 | - | F | Editorial corrections and core spec alignment for 5.7.2.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214349 | 1249 | - | F | Editorial corrections and core spec alignment for 5.7.3.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214350 | 1250 | - | F | Core spec alignment for 5.6.1.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214351 | 1251 | - | F | Core spec alignment for 5.6.1.2 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214352 | 1252 | - | F | Core spec alignment for 5.6.1.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214353 | 1253 | - | F | Core spec alignment for 5.6.1.4 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214354 | 1254 | - | F | Correction to 5.6.2.1 message contents | 16.9.0 |
| 2021-09 | RAN#93 | R5-214355 | 1255 | - | F | Correction to 5.6.2.2 message contents | 16.9.0 |
| 2021-09 | RAN#93 | R5-214356 | 1256 | - | F | Correction to 5.6.2.3 message contents | 16.9.0 |
| 2021-09 | RAN#93 | R5-214357 | 1257 | - | F | Correction to 5.6.2.4 message contents | 16.9.0 |
| 2021-09 | RAN#93 | R5-214358 | 1258 | - | F | Correction to 5.6.2.5 message contents | 16.9.0 |
| 2021-09 | RAN#93 | R5-214359 | 1259 | - | F | Correction to 5.6.2.6 message contents | 16.9.0 |
| 2021-09 | RAN#93 | R5-214360 | 1260 | - | F | Correction to 5.6.2.7 message contents | 16.9.0 |
| 2021-09 | RAN#93 | R5-214361 | 1261 | - | F | Correction to 5.6.2.8 message contents | 16.9.0 |
| 2021-09 | RAN#93 | R5-214363 | 1263 | - | F | Correction to 6.5.1.1 message contents and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214365 | 1265 | - | F | Correction to 6.5.1.5 message contents and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214367 | 1267 | - | F | Correction to 6.3.1.6 and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214368 | 1268 | - | F | Correction to 6.5.2.1 and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214370 | 1270 | - | F | Correction to 6.7.5.1 and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214371 | 1271 | - | F | Correction to 6.7.6.1 and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214372 | 1272 | - | F | Correction to 6.7.7.1 and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214373 | 1273 | - | F | Core spec alignment for 7.6.1.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214374 | 1274 | - | F | Core spec alignment for 7.6.1.2 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214375 | 1275 | - | F | Core spec alignment for 7.6.1.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214376 | 1276 | - | F | Core spec alignment for 7.6.1.4 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214378 | 1278 | - | F | Correction to 8.3.1.1 and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214395 | 1280 | - | F | Addition of FR1 mobility enhancement TC 6.3.1.9-Intra-band inter-frequency sync DAPS HO in SA for FR1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214407 | 1281 | - | F | Clarification of test procedure for 4.3.2.2.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214408 | 1282 | - | F | Corrections to 4.7.1.x.y SS-RSRP test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214409 | 1283 | - | F | Corrections to 4.7.2.x SS-RSRQ test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214410 | 1284 | - | F | Corrections to 4.7.3.x SS-SINR test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214411 | 1285 | - | F | Corrections to 4.5.1.6 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214412 | 1286 | - | F | Corrections to 4.5.1.8 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214413 | 1287 | - | F | Core spec alignment of EN-DC FR2 PRACH | 16.9.0 |
| 2021-09 | RAN#93 | R5-214414 | 1288 | - | F | Corrections to 6.5.1.2 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214415 | 1289 | - | F | Corrections to 6.5.1.4 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214416 | 1290 | - | F | Corrections to 6.7.1.x.y SS-RSRP test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214417 | 1291 | - | F | Corrections to 6.7.2.x SS-RSRQ test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214418 | 1292 | - | F | Corrections to 6.7.3.x SS-SINR test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214419 | 1293 | - | F | Corrections to 6.5.1.6 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214420 | 1294 | - | F | Corrections to 6.5.1.8 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214421 | 1295 | - | F | Align 6.3.2.2.x to core spec | 16.9.0 |
| 2021-09 | RAN#93 | R5-214422 | 1296 | - | F | Align 6.4.3.1 to core spec | 16.9.0 |
| 2021-09 | RAN#93 | R5-214423 | 1297 | - | F | Align 6.6.2.x to core spec | 16.9.0 |
| 2021-09 | RAN#93 | R5-214424 | 1298 | - | F | Align 6.6.3.x to core spec | 16.9.0 |
| 2021-09 | RAN#93 | R5-214426 | 1300 | - | F | Correction to Table H.3.5-9 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214427 | 1301 | - | F | Clarification CSI-ReportConfig from Annex H | 16.9.0 |
| 2021-09 | RAN#93 | R5-214519 | 1302 | - | F | Addition of FR1 mobility enhancement TC 6.3.1.10-Intra-band inter-frequency asynchronous DAPS HO in SA for FR1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214520 | 1303 | - | F | Addition of FR1 mobility enhancement TC 6.3.1.11-Inter-band inter-frequency sync DAPS HO in SA for FR1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214521 | 1304 | - | F | Addition of FR1 mobility enhancement TC 6.3.1.12-Inter-band inter-frequency asynchronousDAPS HO in SA for FR1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214522 | 1305 | - | F | Update of applicability for RLM TC 4.6.1.3 and 4.6.1.6 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214525 | 1308 | - | F | Update of applicability for RLM TC 6.6.1.3 and 6.6.1.6 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214526 | 1309 | - | F | Correction of cell configuration for SA FR1 TC 6.3.2.1.2 and 6.3.2.1.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214527 | 1310 | - | F | Correction of specific message content for SA FR2 TC 7.6.1.4 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214528 | 1311 | - | F | Correction of cell configuration for SA FR2 TC 7.3.2.1.1, 7.3.2.1.2 and 7.3.2.1.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214529 | 1312 | - | F | Update of TCI configuration for SA FR2 TC 7.6.2.1 and 7.6.2.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214530 | 1313 | - | F | Correction of non-existent config for SA FR2 TC 7.7.1.3.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214681 | 1319 | - | F | Core alignment for DRX configuration | 16.9.0 |
| 2021-09 | RAN#93 | R5-214682 | 1320 | - | F | Correction to FR2 event-triggered reporting in DRX test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214685 | 1323 | - | F | Correction to Inter-RAT SFTD measurement delay and event triggered reporting tests | 16.9.0 |
| 2021-09 | RAN#93 | R5-214687 | 1325 | - | F | Correction to 4.5.7.1EN-DC FR1 addition and release delay of known PSCell | 16.9.0 |
| 2021-09 | RAN#93 | R5-214688 | 1326 | - | F | Correction to NR SA FR1 - E-UTRAN event-triggered reporting tests | 16.9.0 |
| 2021-09 | RAN#93 | R5-214689 | 1327 | - | F | Correction to the number of entries in the measObjectToAddModList | 16.9.0 |
| 2021-09 | RAN#93 | R5-214693 | 1331 | - | F | Clean up on editor notes for FR2 test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214711 | 1335 | - | F | Add minimum conformance requirements for DAPS handover | 16.9.0 |
| 2021-09 | RAN#93 | R5-214832 | 1337 | - | F | Correction to 5G-SRVCC RRM TC 6.3.1.6-handover | 16.9.0 |
| 2021-09 | RAN#93 | R5-214833 | 1338 | - | F | Correction to 5G-SRVCC RRM TC 6.6.5.1-envent triggered reporting non-DRX | 16.9.0 |
| 2021-09 | RAN#93 | R5-214921 | 1340 | - | F | Alignment HO test case 6.3.1.2 with core requirements | 16.9.0 |
| 2021-09 | RAN#93 | R5-214922 | 1341 | - | F | Correction re-establishment test cases 6.3.2.1.x | 16.9.0 |
| 2021-09 | RAN#93 | R5-214923 | 1342 | - | F | Remove gapUE and gapFR1 from iRAT test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214924 | 1343 | - | F | Annex A.6.1 for iRAT test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214948 | 1344 | - | F | Addition of FR-1 NSA CLI Measurement test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214983 | 1349 | - | F | Correction to FR1 EN-DC TCs-RLM | 16.9.0 |
| 2021-09 | RAN#93 | R5-214985 | 1351 | - | F | Correction to FR2 EN-DC TCs-RLM | 16.9.0 |
| 2021-09 | RAN#93 | R5-214986 | 1352 | - | F | Correction to FR2 EN-DC TCs-SCell activation | 16.9.0 |
| 2021-09 | RAN#93 | R5-214987 | 1353 | - | F | Correction to FR1 NR SA TC 6.5.2.1-SCell interruption | 16.9.0 |
| 2021-09 | RAN#93 | R5-214989 | 1355 | - | F | Correction to FR1 NR SA TCs-cell reselection | 16.9.0 |
| 2021-09 | RAN#93 | R5-214991 | 1357 | - | F | Correction to FR1 NR SA TCs-RLM | 16.9.0 |
| 2021-09 | RAN#93 | R5-214993 | 1359 | - | F | Correction to FR2 NR SA TCs-cell reselection | 16.9.0 |
| 2021-09 | RAN#93 | R5-214994 | 1360 | - | F | Correction to FR2 NR SA TCs-SCell activation | 16.9.0 |
| 2021-09 | RAN#93 | R5-214996 | 1362 | - | F | Correction to LTE SA TCs-cell reselection | 16.9.0 |
| 2021-09 | RAN#93 | R5-214997 | 1363 | - | F | Correction to LTE SA TCs-inter-RAT delay | 16.9.0 |
| 2021-09 | RAN#93 | R5-214999 | 1365 | - | F | Correction to cell mapping for CA TCs | 16.9.0 |
| 2021-09 | RAN#93 | R5-215005 | 1367 | - | F | Addition of minimum requirements for FR1 CHO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215006 | 1368 | - | F | Addition of minimum requirements for FR2 CHO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215007 | 1369 | - | F | Addition of NR Mob_Enh RRM TC 6.3.1.7-intra freq sync DAPS HO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215008 | 1370 | - | F | Addition of NR Mob_Enh RRM TC 6.3.1.8-intra freq async DAPS HO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215009 | 1371 | - | F | Addition of NR Mob_Enh RRM TC 6.3.3.1-intra freq CHO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215010 | 1372 | - | F | Addition of NR Mob_Enh RRM TC 6.3.3.2-inter freq CHO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215011 | 1373 | - | F | Addition of NR Mob_Enh RRM TC 7.3.1.4-inter band sync DAPS HO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215012 | 1374 | - | F | Addition of NR Mob_Enh RRM TC 7.3.1.5-inter band async DAPS HO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215013 | 1375 | - | F | Addition of NR Mob_Enh RRM TC 7.3.3.1-intra freq CHO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215014 | 1376 | - | F | Addition of NR Mob_Enh RRM TC 7.3.3.2-inter freq CHO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215020 | 1378 | - | F | Addition of minimum requirements for inter-freq relaxed measurement | 16.9.0 |
| 2021-09 | RAN#93 | R5-215022 | 1380 | - | F | Addition of minimum requirements for intra-freq relaxed measurement | 16.9.0 |
| 2021-09 | RAN#93 | R5-215023 | 1381 | - | F | Addition of NR PS RRM TC 6.1.1.3 - intra-freq cell reselection low mobility | 16.9.0 |
| 2021-09 | RAN#93 | R5-215024 | 1382 | - | F | Addition of NR PS RRM TC 6.1.1.4 - intra-freq cell reselection non-cell-edge | 16.9.0 |
| 2021-09 | RAN#93 | R5-215025 | 1383 | - | F | Addition of NR PS RRM TC 6.1.1.5 - inter-freq cell reselection low mobility | 16.9.0 |
| 2021-09 | RAN#93 | R5-215026 | 1384 | - | F | Addition of NR PS RRM TC 6.1.1.6 - inter-freq cell reselection non-cell-edge | 16.9.0 |
| 2021-09 | RAN#93 | R5-215036 | 1388 | - | F | Correction to minimum requirements for inter-RAT cell reselection with highSpeedMeasFlag | 16.9.0 |
| 2021-09 | RAN#93 | R5-215037 | 1389 | - | F | Correction to minimum requirements for intra-frequency measurement with highSpeedMeasFlag | 16.9.0 |
| 2021-09 | RAN#93 | R5-215038 | 1390 | - | F | Correction to NR HST RRM TC 6.1.1.7-HST intra-freq cell reselection | 16.9.0 |
| 2021-09 | RAN#93 | R5-215039 | 1391 | - | F | Addition of cell mapping for NR HST RRM TCs | 16.9.0 |
| 2021-09 | RAN#93 | R5-215243 | 1398 | - | F | Update of test case 6.6.4.5 for R16 NR HST | 16.9.0 |
| 2021-09 | RAN#93 | R5-215244 | 1399 | - | F | Editorial change of RRM test case 6.6.1.7 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215352 | 1401 | - | F | Correction to test frequeny selection for intra-band EN-DC | 16.9.0 |
| 2021-09 | RAN#93 | R5-215354 | 1403 | - | F | Correction to test requirement for 8.4.2.4 and error in writing for 8.4.2.x | 16.9.0 |
| 2021-09 | RAN#93 | R5-215355 | 1404 | - | F | Correction to DRX configuration for eliminating overlap between DRX and SMTC | 16.9.0 |
| 2021-09 | RAN#93 | R5-215360 | 1405 | - | F | Correction to test case 6.3.1.1 and 6.3.1.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215365 | 1406 | - | F | Core spec alignment to add CCR configuration for EN-DC PRACH | 16.9.0 |
| 2021-09 | RAN#93 | R5-215366 | 1407 | - | F | Core spec alignment to add CCR configuration for EN-DC Timing | 16.9.0 |
| 2021-09 | RAN#93 | R5-215367 | 1408 | - | F | Core spec alignment to add CCR configuration for EN-DC event triggered | 16.9.0 |
| 2021-09 | RAN#93 | R5-215368 | 1409 | - | F | Addition of minimum conformance requirements of cell re-selection with relaxed measurement criterion in FR2 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215392 | 1410 | - | F | Add test case 4.3.2.2.3 for EN-DC FR1 2-step PRACH | 16.9.0 |
| 2021-09 | RAN#93 | R5-215393 | 1411 | - | F | Add test case 4.3.2.2.4 for EN-DC FR1 2-step PRACH | 16.9.0 |
| 2021-09 | RAN#93 | R5-215394 | 1412 | - | F | Add test case 5.3.2.2.3 for EN-DC FR2 2-step PRACH | 16.9.0 |
| 2021-09 | RAN#93 | R5-215395 | 1413 | - | F | Add test case 5.3.2.2.4 for EN-DC FR2 2-step PRACH | 16.9.0 |
| 2021-09 | RAN#93 | R5-215396 | 1414 | - | F | Add test case 6.3.2.2.3 for SA FR1 2-step PRACH | 16.9.0 |
| 2021-09 | RAN#93 | R5-215397 | 1415 | - | F | Add test case 6.3.2.2.4 for SA FR1 2-step PRACH | 16.9.0 |
| 2021-09 | RAN#93 | R5-215398 | 1416 | - | F | Add 2-Step PRACH test cases to Annex E | 16.9.0 |
| 2021-09 | RAN#93 | R5-215412 | 1417 | - | F | Addition of cell configuration for RRM HST test cases in Annex E | 16.9.0 |
| 2021-09 | RAN#93 | R5-215413 | 1418 | - | F | Correction of RRM HST Inter-RAT measurements test case 6.6.3.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215414 | 1419 | - | F | Correction of RRM HST E-UTRA NR FR1 Cell reselection test case 8.2.1.2 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215415 | 1420 | - | F | Correction of RRM HST E-UTRA NR Inter-RAT event triggered reporting test case 8.4.2.9 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215420 | 1425 | - | F | Correction of RRM EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX test case 5.6.3.4 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215425 | 1430 | - | F | Correction of RRM EN-DC FR2-FR2 event-triggered reporting in DRX test case 5.6.2.2 including Test Tolerance | 16.9.0 |
| 2021-09 | RAN#93 | R5-215426 | 1431 | - | F | Correction of RRM EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection test case 5.6.2.4 including Test Tolerance | 16.9.0 |
| 2021-09 | RAN#93 | R5-215427 | 1432 | - | F | Correction of RRM SA FR2-FR2 event-triggered reporting in DRX test case 7.6.2.2 including Test Tolerance | 16.9.0 |
| 2021-09 | RAN#93 | R5-215428 | 1433 | - | F | Correction of RRM SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection test case 7.6.2.4 including Test Tolerance | 16.9.0 |
| 2021-09 | RAN#93 | R5-215429 | 1434 | - | F | Editorial correction of RRM FR2 EN-DC event triggered measurement test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-215430 | 1435 | - | F | Editorial correction of RRM FR2 SA event triggered measurement test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-215431 | 1436 | - | F | Correction of Measurement Uncertainty and Test Tolerance in Annex F for RRM test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-215436 | 1437 | - | F | Add MsgA configuration to the TS 38.533 annexes | 16.9.0 |
| 2021-09 | RAN#93 | R5-215614 | 1438 | - | F | Update on 6.3.1.1 to be aligned with TS 38.133 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215620 | 1439 | - | F | MTSU and TT mapping related to Max Device Size in TS 38.533 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215622 | 1441 | - | F | Updates on 5.6.1.3 and 5.6.1.4 CSI-RS RLM test cases test applicability | 16.9.0 |
| 2021-09 | RAN#93 | R5-215624 | 1443 | - | F | Correction in 5.6.1.1, 5.6.1.3 test procedure to configure iterations | 16.9.0 |
| 2021-09 | RAN#93 | R5-215625 | 1444 | - | F | Correction in 5.6.2.1, 5.6.2.2, 5.6.2.3, 5.6.2.4 test procedure to configure iterations | 16.9.0 |
| 2021-09 | RAN#93 | R5-215655 | 1451 | - | F | Update to applicability statement to include gap pattern id 13 for applicable NSA event triggered test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-215656 | 1452 | - | F | TRS configuration update to NSA FR1 TC 6.5.4.1.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215903 | 1238 | 1 | F | Change to EN-DC L1-RSRP test cases to add evaluation rules | 16.9.0 |
| 2021-09 | RAN#93 | R5-215904 | 1240 | 1 | F | Correction to 4.5.1.2 core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-215905 | 1241 | 1 | F | Correction to 4.5.1.3 message contents and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-215906 | 1244 | 1 | F | Correction to 4.5.1.7 message contents and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-215907 | 1329 | 1 | F | Correction to FR1 and FR2 event-triggered reporting with gap tests | 16.9.0 |
| 2021-09 | RAN#93 | R5-215908 | 1262 | 1 | F | Change to SA L1-RSRP test cases to add evaluation rules | 16.9.0 |
| 2021-09 | RAN#93 | R5-215909 | 1264 | 1 | F | Correction to 6.5.1.3 message contents and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-215910 | 1266 | 1 | F | Correction to 6.5.1.7 message contents and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-215911 | 1356 | 1 | F | Correction to FR1 NR SA TCs-inter-RAT accuracy | 16.9.0 |
| 2021-09 | RAN#93 | R5-215912 | 1402 | 1 | F | Correction to test procedure for 6.1.2.2 IRAT ReSelection | 16.9.0 |
| 2021-09 | RAN#93 | R5-215913 | 1277 | 1 | F | Change title of iRAT test cases for clarity | 16.9.0 |
| 2021-09 | RAN#93 | R5-215914 | 1361 | 1 | F | Correction to LTE SA TC 8.5.1.1-SFTD accuracy | 16.9.0 |
| 2021-09 | RAN#93 | R5-215915 | 1279 | 1 | F | Change title of iRAT test cases for clarity - Annexes | 16.9.0 |
| 2021-09 | RAN#93 | R5-215916 | 1333 | 1 | F | Update to Annex H.3.4 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215917 | 1366 | 1 | F | Correction to default configuration-Annex H | 16.9.0 |
| 2021-09 | RAN#93 | R5-215918 | 1421 | 1 | F | Correction of CSI-ReportConfig in Annex H | 16.9.0 |
| 2021-09 | RAN#93 | R5-215932 | 1377 | 1 | F | Addition of cell mapping for Mob_Enh RRM TCs | 16.9.0 |
| 2021-09 | RAN#93 | R5-215938 | 1316 | 1 | F | Addition of NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | 16.9.0 |
| 2021-09 | RAN#93 | R5-215939 | 1385 | 1 | F | Addition of cell mapping for NR PS RRM TCs | 16.9.0 |
| 2021-09 | RAN#93 | R5-215940 | 1392 | 1 | F | Addition of minimum conformance requirements of inter-RAT cell re-selection with relaxed measurement criterion | 16.9.0 |
| 2021-09 | RAN#93 | R5-215941 | 1395 | 1 | F | Update of Annex E and Annex F for test cases with relaxed measurement criterion | 16.9.0 |
| 2021-09 | RAN#93 | R5-215948 | 1386 | 1 | F | Addition of NR HST RRM TC 4.6.1.7-intra-freq DRX highSpeedMeasFlag | 16.9.0 |
| 2021-09 | RAN#93 | R5-215949 | 1387 | 1 | F | Addition of NR HST RRM TC 6.1.2.5-intra-freq cell reselection highSpeedMeasFlag | 16.9.0 |
| 2021-09 | RAN#93 | R5-216042 | 1328 | 1 | F | Correction to FR1 Beam Failure Detection and Link Recovery tests | 16.9.0 |
| 2021-09 | RAN#93 | R5-216043 | 1339 | 1 | F | Correction message contents 4.3.2.2.2 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216044 | 1345 | 1 | F | Update to applicability of test cases on CSI-RS based RLM | 16.9.0 |
| 2021-09 | RAN#93 | R5-216045 | 1445 | 1 | F | Updates to DCI based BWP switch NSA FR1 TC 4.5.6.1.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216046 | 1446 | 1 | F | Updates to DCI based BWP switch NSA FR1 2DLCA TC 4.5.6.1.2 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216047 | 1447 | 1 | F | Updates to RRC based BWP switch NSA FR1 TC 4.5.6.2.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216048 | 1228 | 1 | F | Completion 5.7.1.2 including TT analysis results | 16.9.0 |
| 2021-09 | RAN#93 | R5-216049 | 1229 | 1 | F | Completion 5.7.2.2 including TT analysis results | 16.9.0 |
| 2021-09 | RAN#93 | R5-216050 | 1230 | 1 | F | Completion 5.7.3.2 including TT analysis results | 16.9.0 |
| 2021-09 | RAN#93 | R5-216051 | 1322 | 1 | F | Correction to 6.5.3.1 NR SA FR1 SCell activation and deactivation of known SCell | 16.9.0 |
| 2021-09 | RAN#93 | R5-216052 | 1231 | 1 | F | Completion 7.7.1.2 including TT analysis results | 16.9.0 |
| 2021-09 | RAN#93 | R5-216053 | 1233 | 1 | F | Completion 7.7.3.2 including TT analysis results | 16.9.0 |
| 2021-09 | RAN#93 | R5-216054 | 1346 | 1 | F | Update to applicability of test cases requiring gap pattern ID 4 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216055 | 1299 | 1 | F | Completion Annex C.2.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216098 | 1336 | 1 | F | Correction to EN-DC FR2 interruptions at transitions between active and non-active during DRX | 16.9.0 |
| 2021-09 | RAN#93 | R5-216099 | 1324 | 1 | F | Correction to 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC | 16.9.0 |
| 2021-09 | RAN#93 | R5-216100 | 1330 | 1 | F | Correction to the propagation condition of NR cell for Inter RAT test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-216101 | 1318 | 1 | F | Addition of BWP definition for FR2 SSB SCS240kHz | 16.9.0 |
| 2021-09 | RAN#93 | R5-216123 | 1448 | 1 | F | Updates to DCI based BWP switch SA FR1 2DLCA TC 6.5.6.1.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216124 | 1449 | 1 | F | Updates to DCI based BWP switch SA FR1 TC 6.5.6.1.2 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216125 | 1450 | 1 | F | Updates to RRC based BWP switch SA FR1 TC 6.5.6.2.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216132 | 1347 | 1 | F | Correction to FR1 EN-DC TC 4.5.7.1-PSCell addition | 16.9.0 |
| 2021-09 | RAN#93 | R5-216133 | 1348 | 1 | F | Correction to FR1 EN-DC TCs-BWP switching | 16.9.0 |
| 2021-09 | RAN#93 | R5-216134 | 1350 | 1 | F | Correction to FR1 EN-DC TCs-SCell activation | 16.9.0 |
| 2021-09 | RAN#93 | R5-216135 | 1354 | 1 | F | Correction to FR1 NR SA TCs-BWP switching | 16.9.0 |
| 2021-09 | RAN#93 | R5-216136 | 1358 | 1 | F | Correction to FR1 NR SA TCs-SCell activation | 16.9.0 |
| 2021-09 | RAN#93 | R5-216137 | 1454 | 1 | F | Correction to UL BWP configuration for SA FR1 TC 6.5.2.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216138 | 1364 | 1 | F | Correction to LTE SA TCs-SFTD delay | 16.9.0 |
| 2021-09 | RAN#93 | R5-216143 | 1393 | 1 | F | Addition of 6.1.2.3 inter-RAT cell re-selection with relaxed measurement with low mobility | 16.9.0 |
| 2021-09 | RAN#93 | R5-216144 | 1394 | 1 | F | Addition of 6.1.2.4 inter-RAT cell re-selection with relaxed measurement with not at cell edge | 16.9.0 |
| 2021-09 | RAN#93 | R5-216145 | 1422 | 1 | F | Correction of RRM EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX test case 5.6.3.1 including Test Tolerance | 16.9.0 |
| 2021-09 | RAN#93 | R5-216146 | 1423 | 1 | F | Correction of RRM EN-DC FR2 SSB-based L1-RSRP measurement in DRX test case 5.6.3.2 including Test Tolerance | 16.9.0 |
| 2021-09 | RAN#93 | R5-216147 | 1424 | 1 | F | Correction of RRM EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX test case 5.6.3.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216148 | 1426 | 1 | F | Correction of RRM NR SA FR2 SSB-based L1-RSRP measurement in non-DRX test case 7.6.3.1 including Test Tolerance | 16.9.0 |
| 2021-09 | RAN#93 | R5-216149 | 1427 | 1 | F | Correction of RRM NR SA FR2 SSB-based L1-RSRP measurement in DRX test case 7.6.3.2 including Test Tolerance | 16.9.0 |
| 2021-09 | RAN#93 | R5-216360 | 1428 | 1 | F | Correction of RRM NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX test case 7.6.3.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216361 | 1429 | 1 | F | Correction of RRM NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX test case 7.6.3.4 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216364 | 1314 | 1 | F | Addition of NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | 16.9.0 |
| 2021-09 | RAN#93 | R5-216365 | 1315 | 1 | F | Addition of NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | 16.9.0 |
| 2021-09 | RAN#93 | R5-216366 | 1317 | 1 | F | Addition of NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | 16.9.0 |
| 2021-09 | RAN#93 | R5-215968 | 1397 | 1 | B | Introduction of n24 | 17.0.0 |
| 2021-12 | RAN#94 | R5-216602 | 1455 | - | F | Update of TC 6.7.1.1.2 | 17.1.0 |
| 2021-12 | RAN#94 | R5-216839 | 1458 | - | F | Correction to CSI-RS based beam failure detection test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-216840 | 1459 | - | F | Correction to Initial DL and UL BWP for test case 6.3.2.1.2 | 17.1.0 |
| 2021-12 | RAN#94 | R5-216841 | 1460 | - | F | Correction to SRS configuration for FR1 timing test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-216842 | 1461 | - | F | Correction to SRS configuration for FR2 timing test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-216843 | 1462 | - | F | Correction to FR1 CA RRM tests for generic channel BW configurations | 17.1.0 |
| 2021-12 | RAN#94 | R5-216844 | 1463 | - | F | Correction to FR1 Active BWP Switch Delay TCs | 17.1.0 |
| 2021-12 | RAN#94 | R5-216845 | 1464 | - | F | Correction to FR1 Redirection test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-216846 | 1465 | - | F | Correction to message exceptions for CSI-RS based BFR tests | 17.1.0 |
| 2021-12 | RAN#94 | R5-216857 | 1470 | - | F | Correction to FR2 EN-DC RRM TCs - RLM | 17.1.0 |
| 2021-12 | RAN#94 | R5-216860 | 1473 | - | F | Correction to FR2 EN-DC RRM TCs using Table H.3.1-4 for event A4 | 17.1.0 |
| 2021-12 | RAN#94 | R5-216861 | 1474 | - | F | Correction to FR1 NR SA RRM TC 6.1.1.1 - intra-freq reselection | 17.1.0 |
| 2021-12 | RAN#94 | R5-216862 | 1475 | - | F | Correction to FR1 NR SA RRM TC 6.7.6.1 - inter-RAT meas | 17.1.0 |
| 2021-12 | RAN#94 | R5-216864 | 1477 | - | F | Correction to FR1 NR SA RRM TCs 6.5.2.1 - interruption | 17.1.0 |
| 2021-12 | RAN#94 | R5-216866 | 1479 | - | F | Correction to FR2 NR SA RRM TCs using Table H.3.1-4 for event A4 | 17.1.0 |
| 2021-12 | RAN#94 | R5-216868 | 1481 | - | F | Correction to SSB reference configurations | 17.1.0 |
| 2021-12 | RAN#94 | R5-216869 | 1482 | - | F | Correction to Annex H for RRM TCs | 17.1.0 |
| 2021-12 | RAN#94 | R5-216873 | 1485 | - | F | Correction to Mob_enh RRM TC 6.3.3.1 - FR1 intra CHO | 17.1.0 |
| 2021-12 | RAN#94 | R5-216874 | 1486 | - | F | Correction to Mob_enh RRM TC 6.3.3.2 - FR1 inter CHO | 17.1.0 |
| 2021-12 | RAN#94 | R5-216876 | 1488 | - | F | Correction to Mob_enh RRM TC 7.3.1.5 - inter-FR DAPS async | 17.1.0 |
| 2021-12 | RAN#94 | R5-216886 | 1492 | - | F | Correction to PS RRM TC 6.1.1.3 - intra low mobility | 17.1.0 |
| 2021-12 | RAN#94 | R5-216887 | 1493 | - | F | Correction to PS RRM TC 6.1.1.4 - intra not cell edge | 17.1.0 |
| 2021-12 | RAN#94 | R5-216888 | 1494 | - | F | Correction to PS RRM TC 6.1.1.5 - inter low mobility | 17.1.0 |
| 2021-12 | RAN#94 | R5-216889 | 1495 | - | F | Correction to PS RRM TC 6.1.1.6 - inter not cell edge | 17.1.0 |
| 2021-12 | RAN#94 | R5-216890 | 1496 | - | F | Correction to Annex F for PS RRM TCs | 17.1.0 |
| 2021-12 | RAN#94 | R5-216900 | 1497 | - | F | Correction to SRVCC RRM TC 6.3.1.6 - UTRA handover | 17.1.0 |
| 2021-12 | RAN#94 | R5-216902 | 1499 | - | F | Correction to Annex F for SRVCC RRM TCs | 17.1.0 |
| 2021-12 | RAN#94 | R5-216912 | 1500 | - | F | Correction to HST RRM TC 4.6.1.7 - intra-freq meas | 17.1.0 |
| 2021-12 | RAN#94 | R5-216913 | 1501 | - | F | Correction to HST RRM TC 4.6.4.5 - L1-RSRP | 17.1.0 |
| 2021-12 | RAN#94 | R5-216914 | 1502 | - | F | Correction to HST RRM TC 6.1.2.5 - inter-RAT reselection | 17.1.0 |
| 2021-12 | RAN#94 | R5-216915 | 1503 | - | F | Correction to Annex F for HST RRM TCs | 17.1.0 |
| 2021-12 | RAN#94 | R5-217070 | 1505 | - | F | Correction of message configuration for EN-DC FR1 SFTD TC 4.7.5.1 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217122 | 1510 | - | F | Reinstate wrongly deleted Table H.3.5-4 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217123 | 1511 | - | F | Correct B2 Threshold in H.3.1-4A | 17.1.0 |
| 2021-12 | RAN#94 | R5-217124 | 1512 | - | F | Corrections to 4.3.2.2.1 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217131 | 1515 | - | F | Corrections to L1-RSRP EN-DC test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-217132 | 1516 | - | F | Corrections to L1-RSRP NR SA test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-217145 | 1517 | - | F | Correct reference to test frequencies | 17.1.0 |
| 2021-12 | RAN#94 | R5-217203 | 1519 | - | F | Addition of minimum requirements for EN-DC L1-SINR measurement for beam reporting | 17.1.0 |
| 2021-12 | RAN#94 | R5-217207 | 1523 | - | F | Addition of minimum requirements for NR SA FR1 L1-SINR measurement for beam reporting | 17.1.0 |
| 2021-12 | RAN#94 | R5-217211 | 1527 | - | F | Addition of the abbreviations for Rel-16 NR MIMO | 17.1.0 |
| 2021-12 | RAN#94 | R5-217213 | 1529 | - | F | Addition of Annex A.1.4B CSI-IM Reference Measurement Channels | 17.1.0 |
| 2021-12 | RAN#94 | R5-217214 | 1530 | - | F | Addition of Annex H.3.6A RRC messages and IE exceptions for L1-SINR measurement for beam reporting | 17.1.0 |
| 2021-12 | RAN#94 | R5-217228 | 1539 | - | F | Correct of RRM Test Cases for test applicability | 17.1.0 |
| 2021-12 | RAN#94 | R5-217229 | 1540 | - | F | Correct of RRM Test Cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-217255 | 1541 | - | F | Addition of FR-1 NSA CLI accuracy test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-217258 | 1542 | - | F | Corrections to event-triggered iRAT | 17.1.0 |
| 2021-12 | RAN#94 | R5-217395 | 1545 | - | F | Update of 6.1.2.3 and 6.1.2.4 for MU and TT | 17.1.0 |
| 2021-12 | RAN#94 | R5-217396 | 1546 | - | F | Update of Annex F for test cases with relaxed measurement criterion | 17.1.0 |
| 2021-12 | RAN#94 | R5-217436 | 1550 | - | F | Correction to E-UTRA - NR FR1 cell re-selection to higher priority NR target cell | 17.1.0 |
| 2021-12 | RAN#94 | R5-217438 | 1551 | - | F | Correction to SIB4 exceptions in Annex H | 17.1.0 |
| 2021-12 | RAN#94 | R5-217439 | 1552 | - | F | Correction to FR2 event-triggered reporting with gap test | 17.1.0 |
| 2021-12 | RAN#94 | R5-217441 | 1553 | - | F | Correction to test procedure in 8.2.1.1 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217443 | 1555 | - | F | Correction to test requirement in 8.3.1.1 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217520 | 1559 | - | F | Addition of RMC CORESET to FR2 random access tests | 17.1.0 |
| 2021-12 | RAN#94 | R5-217570 | 1561 | - | F | Correction of RRM HST Inter-RAT measurements test case 6.6.3.3 including Test Tolerance | 17.1.0 |
| 2021-12 | RAN#94 | R5-217572 | 1563 | - | F | Correction of RRM HST E-UTRA NR Inter-RAT event triggered reporting test case 8.4.2.9 including Test Tolerance | 17.1.0 |
| 2021-12 | RAN#94 | R5-217573 | 1564 | - | F | Correction of RRM EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX test case 5.6.3.3 including Test Tolerance | 17.1.0 |
| 2021-12 | RAN#94 | R5-217574 | 1565 | - | F | Correction of RRM EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX test case 5.6.3.4 including Test Tolerance | 17.1.0 |
| 2021-12 | RAN#94 | R5-217575 | 1566 | - | F | Correction of RRM NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX test case 7.6.3.3 including Test Tolerance | 17.1.0 |
| 2021-12 | RAN#94 | R5-217576 | 1567 | - | F | Correction of RRM NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX test case 7.6.3.4 including Test Tolerance | 17.1.0 |
| 2021-12 | RAN#94 | R5-217577 | 1568 | - | F | Correction of SA FR2 RRC re-establishment test case 7.3.2.1.1 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217580 | 1571 | - | F | Correction of Measurement Uncertainty and Test Tolerance in Annex F for RRM test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-217585 | 1572 | - | F | Correction of Minimum conformance requirements for FR1 re-establishment test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-217586 | 1573 | - | F | Correction of Minimum conformance requirements for FR2 re-establishment test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-217661 | 1574 | - | F | Update on FR2 Radio Link monitoring SSB based RLM test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-217663 | 1575 | - | F | Update on FR2 radio link monitoring CSI-RS based RLM test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-217736 | 1576 | - | F | Update to SA Handover delay requirements | 17.1.0 |
| 2021-12 | RAN#94 | R5-218207 | 1577 | - | F | Correct 5.7.3.2 / 7.7.3.2 test case limits based on TT analysis results | 17.1.0 |
| 2021-12 | RAN#94 | R5-218215 | 1457 | 1 | F | Correction to Mob_enh RRM TC 6.3.1.9 - inter-freq sync DAPS HO | 17.1.0 |
| 2021-12 | RAN#94 | R5-218216 | 1491 | 1 | F | Correction to Annex F for Mob_enh RRM TCs | 17.1.0 |
| 2021-12 | RAN#94 | R5-218217 | 1504 | 1 | F | Correction to Mob_enh RRM TC 6.3.1.10 - inter-freq async DAPS HO | 17.1.0 |
| 2021-12 | RAN#94 | R5-218250 | 1469 | 1 | F | Correction to FR1 EN-DC RRM TCs 4.5.2.X - interruption | 17.1.0 |
| 2021-12 | RAN#94 | R5-218251 | 1513 | 1 | F | Corrections to RLM EN-DC test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-218252 | 1548 | 1 | F | Correction to EN-DC FR1 addition and release delay of known PSCell | 17.1.0 |
| 2021-12 | RAN#94 | R5-218253 | 1471 | 1 | F | Correction to FR2 EN-DC RRM TCs - RRC based | 17.1.0 |
| 2021-12 | RAN#94 | R5-218254 | 1506 | 1 | F | Correction of T2 timer value for SA FR1 TC 6.3.2.1.3 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218255 | 1514 | 1 | F | Corrections to RLM NR SA test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-218256 | 1478 | 1 | F | Correction to FR2 NR SA RRM TCs - RLM | 17.1.0 |
| 2021-12 | RAN#94 | R5-218257 | 1507 | 1 | F | Correction of RRC re-establishment delay for SA FR2 TC 7.3.2.1.1 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218258 | 1508 | 1 | F | Correction of T2 timer value for SA FR2 TC 7.3.2.1.2 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218259 | 1509 | 1 | F | Correction of T2 timer value for SA FR2 TC 7.3.2.1.3 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218260 | 1569 | 1 | F | Correction of SA FR2 - FR2 RRC re-establishment test case 7.3.2.1.2 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218261 | 1480 | 1 | F | Correction to LTE SA RRM TCs - NR inter-RAT accuracy | 17.1.0 |
| 2021-12 | RAN#94 | R5-218294 | 1544 | 1 | F | Update of Annex for 4.5.8.1 EN-DC FR1 DL interruptions at switching between two uplink carriers | 17.1.0 |
| 2021-12 | RAN#94 | R5-218296 | 1483 | 1 | F | Correction to Mob_enh RRM TC 6.3.1.7 - intra DAPS sync | 17.1.0 |
| 2021-12 | RAN#94 | R5-218297 | 1484 | 1 | F | Correction to Mob_enh RRM TC 6.3.1.8 - intra DAPS async | 17.1.0 |
| 2021-12 | RAN#94 | R5-218298 | 1487 | 1 | F | Correction to Mob_enh RRM TC 7.3.1.4 - inter-FR DAPS sync | 17.1.0 |
| 2021-12 | RAN#94 | R5-218299 | 1490 | 1 | F | Correction to Mob_enh RRM TC 7.3.3.2 - FR2 inter CHO | 17.1.0 |
| 2021-12 | RAN#94 | R5-218313 | 1520 | 1 | F | Addition of 4.6.7.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX | 17.1.0 |
| 2021-12 | RAN#94 | R5-218314 | 1521 | 1 | F | Addition of 4.6.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX | 17.1.0 |
| 2021-12 | RAN#94 | R5-218315 | 1522 | 1 | F | Addition of 4.6.7.3 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX | 17.1.0 |
| 2021-12 | RAN#94 | R5-218316 | 1524 | 1 | F | Addition of 6.6.8.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | 17.1.0 |
| 2021-12 | RAN#94 | R5-218317 | 1525 | 1 | F | Addition of 6.6.8.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | 17.1.0 |
| 2021-12 | RAN#94 | R5-218318 | 1526 | 1 | F | Addition of 6.6.8.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | 17.1.0 |
| 2021-12 | RAN#94 | R5-218319 | 1528 | 1 | F | Addition of CSI-RS Reference Measurement Channels based on IMR | 17.1.0 |
| 2021-12 | RAN#94 | R5-218320 | 1531 | 1 | F | Addition of 4.5.5.5 EN-DC FR1 SCell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | 17.1.0 |
| 2021-12 | RAN#94 | R5-218321 | 1532 | 1 | F | Addition of 4.5.5.6 EN-DC FR1 SCell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | 17.1.0 |
| 2021-12 | RAN#94 | R5-218322 | 1533 | 1 | F | Addition of 4.7.7.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | 17.1.0 |
| 2021-12 | RAN#94 | R5-218323 | 1535 | 1 | F | Addition of 4.7.7.3 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | 17.1.0 |
| 2021-12 | RAN#94 | R5-218324 | 1536 | 1 | F | Addition of 6.7.9.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | 17.1.0 |
| 2021-12 | RAN#94 | R5-218325 | 1537 | 1 | F | Addition of 6.7.9.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | 17.1.0 |
| 2021-12 | RAN#94 | R5-218326 | 1538 | 1 | F | Addition of 6.7.9.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | 17.1.0 |
| 2021-12 | RAN#94 | R5-218327 | 1547 | 1 | F | Addition of cell configuration MU and TT for Enhancement on MIMO | 17.1.0 |
| 2021-12 | RAN#94 | R5-218336 | 1498 | 1 | F | Correction to SRVCC RRM TC 6.6.5.1 - UTRA meas | 17.1.0 |
| 2021-12 | RAN#94 | R5-218351 | 1562 | 1 | F | Correction of RRM HST E-UTRA NR FR1 Cell reselection test case 8.2.1.2 including Test Tolerance | 17.1.0 |
| 2021-12 | RAN#94 | R5-218352 | 1570 | 1 | F | Correction of Measurement Uncertainty and Test Tolerance in Annex F for RRM HST test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-218372 | 1543 | 1 | F | Addition of 4.5.8.1 EN-DC FR1 DL interruptions at switching between two uplink carriers | 17.1.0 |
| 2021-12 | RAN#94 | R5-218439 | 1468 | 1 | F | Correction to FR1 EN-DC RRM TCs - RRC based | 17.1.0 |
| 2021-12 | RAN#94 | R5-218440 | 1560 | 1 | F | Test Procedure update for FR1 Event Triggered test cases with CDRX | 17.1.0 |
| 2021-12 | RAN#94 | R5-218441 | 1472 | 1 | F | Correction to FR2 EN-DC RRM TCs 5.5.2.X - interruption | 17.1.0 |
| 2021-12 | RAN#94 | R5-218442 | 1558 | 1 | F | Update to FR2 interruption tests 5.5.2.1 and 5.5.2.2 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218443 | 1456 | 1 | F | Addition of clause 3A.6 for UE with Multiband Capability | 17.1.0 |
| 2021-12 | RAN#94 | R5-218444 | 1556 | 1 | F | Clarification on cl 3A.1.1 test coverage across 5G NR architecture options for RRM | 17.1.0 |
| 2021-12 | RAN#94 | R5-218461 | 1489 | 1 | F | Correction to Mob_enh RRM TC 7.3.3.1 - FR2 intra CHO | 17.1.0 |
| 2021-12 | RAN#94 | R5-218464 | 1534 | 1 | F | Addition of 4.7.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | 17.1.0 |
| 2022-03 | RAN#95 | R5-220096 | 1578 | - | F | Correction to Mob_enh RRM TC 6.3.1.11 - inter-band sync DAPS HO including test tolerance | 17.2.0 |
| 2022-03 | RAN#95 | R5-220097 | 1579 | - | F | Correction to Mob_enh RRM TC 6.3.1.12 - inter-band async DAPS HO including test tolerance | 17.2.0 |
| 2022-03 | RAN#95 | R5-220109 | 1580 | - | F | Update of SA FR1 TC 4.5.3.1 and 4.5.3.3 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220110 | 1581 | - | F | Update of SA FR1 TC 6.5.3.1 and 6.5.3.3 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220136 | 1582 | - | F | Correction to Annex F for Mob_enh RRM TCs | 17.2.0 |
| 2022-03 | RAN#95 | R5-220160 | 1583 | - | F | Add test case 7.3.2.2.4 for SA FR2 2-step PRACH | 17.2.0 |
| 2022-03 | RAN#95 | R5-220167 | 1584 | - | F | Updates to RRM HST Test Case 6.6.1.7 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220168 | 1585 | - | F | Updates to RRM HST Test Case 6.6.4.5 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220196 | 1586 | - | F | Update to table E.5-1 to add the cell configurations for one 2-step RACH test case | 17.2.0 |
| 2022-03 | RAN#95 | R5-220278 | 1587 | - | F | Clarifications on 5G NR connectivity options for RRM | 17.2.0 |
| 2022-03 | RAN#95 | R5-220282 | 1588 | - | F | Test case update for FR1 CLI-RSSI measurement with non-DRX | 17.2.0 |
| 2022-03 | RAN#95 | R5-220283 | 1589 | - | F | Annexure update for test tolerance for FR1 CLI-RSSI measurement with non-DRX | 17.2.0 |
| 2022-03 | RAN#95 | R5-220689 | 1592 | - | F | Correction to FR1 EN-DC RRM TCs - SCell activation | 17.2.0 |
| 2022-03 | RAN#95 | R5-220697 | 1600 | - | F | Correction to FR2 EN-DC RRM TC 5.6.1.2 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220699 | 1602 | - | F | Correction to FR2 EN-DC RRM TC 5.6.1.4 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220701 | 1604 | - | F | Correction to FR1 NR SA RRM TC 6.5.2.1 - interruption SCC | 17.2.0 |
| 2022-03 | RAN#95 | R5-220702 | 1605 | - | F | Correction to FR1 NR SA RRM TCs - SCell activation | 17.2.0 |
| 2022-03 | RAN#95 | R5-220726 | 1623 | - | F | Addition of band group for NR SL RRM test | 17.2.0 |
| 2022-03 | RAN#95 | R5-220727 | 1624 | - | F | Addition of minimum requirements for NR SL UE Tx timing TCs | 17.2.0 |
| 2022-03 | RAN#95 | R5-220728 | 1625 | - | F | Addition of NR SL RRM TC 9.1.1.1 - Tx Timing GNSS | 17.2.0 |
| 2022-03 | RAN#95 | R5-220729 | 1626 | - | F | Addition of NR SL RRM TC 9.1.1.2 - Tx Timing SyncRef UE | 17.2.0 |
| 2022-03 | RAN#95 | R5-220730 | 1627 | - | F | Addition of NR SL RRM TC 9.1.1.3 - Tx Timing NR Cell | 17.2.0 |
| 2022-03 | RAN#95 | R5-220731 | 1628 | - | F | Addition of minimum requirements for NR SL S-SSB Tx TCs | 17.2.0 |
| 2022-03 | RAN#95 | R5-220734 | 1631 | - | F | Addition of minimum requirements for NR SL SyncRef reselection TCs | 17.2.0 |
| 2022-03 | RAN#95 | R5-220737 | 1634 | - | F | Addition of minimum requirements for NR SL L1 SL-RSRP TCs | 17.2.0 |
| 2022-03 | RAN#95 | R5-220738 | 1635 | - | F | Addition of NR SL RRM TC 9.1.4.1 - Resource sensing | 17.2.0 |
| 2022-03 | RAN#95 | R5-220741 | 1638 | - | F | Addition of minimum requirements for NR SL Congestion control TCs | 17.2.0 |
| 2022-03 | RAN#95 | R5-220742 | 1639 | - | F | Addition of NR SL RRM TC 9.1.5.1 - SL-RSSI con-current | 17.2.0 |
| 2022-03 | RAN#95 | R5-220744 | 1641 | - | F | Addition of minimum requirements for NR SL interruption TCs | 17.2.0 |
| 2022-03 | RAN#95 | R5-220745 | 1642 | - | F | Addition of NR SL RRM TC 9.1.6.1 - WAN interruption | 17.2.0 |
| 2022-03 | RAN#95 | R5-220747 | 1644 | - | F | Addition of side conditions for NR SL RRM test in Annex B | 17.2.0 |
| 2022-03 | RAN#95 | R5-220748 | 1645 | - | F | Correction to Annex E for NR SL RRM TCs | 17.2.0 |
| 2022-03 | RAN#95 | R5-220749 | 1646 | - | F | Correction to Annex G for NR SL RRM TCs | 17.2.0 |
| 2022-03 | RAN#95 | R5-220798 | 1647 | - | F | Update to 5.5.5.0 FR2 BFD minimum requirements | 17.2.0 |
| 2022-03 | RAN#95 | R5-220801 | 1650 | - | F | Update to 7.5.5.0 FR2 BFD minimum requirements | 17.2.0 |
| 2022-03 | RAN#95 | R5-220804 | 1653 | - | F | Update to Annex E for eMIMO test cases | 17.2.0 |
| 2022-03 | RAN#95 | R5-220910 | 1657 | - | F | Correction to TRS Configuration of 6.3.2.1.3 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220911 | 1658 | - | F | Correction to test procedure of 6.1.1.2 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220912 | 1659 | - | F | Correction to reference of test frequency in 6.3.1.x NR only test cases | 17.2.0 |
| 2022-03 | RAN#95 | R5-220918 | 1661 | - | F | Correction to SRS configuration condition for FR1 timing test cases | 17.2.0 |
| 2022-03 | RAN#95 | R5-220920 | 1662 | - | F | Correction to DRX setting of 8.4.2.9 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220921 | 1663 | - | F | Correction to message exception for handover test cases | 17.2.0 |
| 2022-03 | RAN#95 | R5-220923 | 1665 | - | F | Correction to BWP switch delay test cases | 17.2.0 |
| 2022-03 | RAN#95 | R5-220924 | 1666 | - | F | Correction to message exception for 4.7.5.1 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220927 | 1668 | - | F | Correction to 6.3.2.3.1 NR SA FR1 RRC connection release with redirection | 17.2.0 |
| 2022-03 | RAN#95 | R5-220928 | 1669 | - | F | Correction to NR-EUTRA and EUTRA-NR reselection with high speed | 17.2.0 |
| 2022-03 | RAN#95 | R5-220929 | 1670 | - | F | Correction to message exception for 8.5.1.1 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220930 | 1671 | - | F | Correction to ssb-ResourceList for 5.3.2.2.2 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220931 | 1672 | - | F | Correction to SMTC configuration in 6.1.1.7 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220933 | 1674 | - | F | Correction to T HARQ setting for 6.5.3.1 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220938 | 1678 | - | F | Correction to DRX setting of 6.6.1.7 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220939 | 1679 | - | F | Correction to message exception of 4.6.1.7 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220940 | 1680 | - | F | Correction to test procedure for 8.2.1.2 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220941 | 1681 | - | F | Correction to SIB1 message exceptions of 4.6.1.7 and 4.6.4.5 | 17.2.0 |
| 2022-03 | RAN#95 | R5-220942 | 1682 | - | F | Addition of new test case 7.7.6.1 NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | 17.2.0 |
| 2022-03 | RAN#95 | R5-220943 | 1683 | - | F | Addition of new test case 7.7.6.2 NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | 17.2.0 |
| 2022-03 | RAN#95 | R5-220944 | 1684 | - | F | Addition of new test case 7.7.6.3 NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | 17.2.0 |
| 2022-03 | RAN#95 | R5-220986 | 1688 | - | F | Update of 4.6.7.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX | 17.2.0 |
| 2022-03 | RAN#95 | R5-220988 | 1690 | - | F | Update of 4.6.7.3 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX | 17.2.0 |
| 2022-03 | RAN#95 | R5-220989 | 1691 | - | F | Update of 6.6.8.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | 17.2.0 |
| 2022-03 | RAN#95 | R5-220991 | 1693 | - | F | Update of 6.6.8.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | 17.2.0 |
| 2022-03 | RAN#95 | R5-220992 | 1694 | - | F | Update of Annex for EN-DC FR1 L1-SINR measurement test cases | 17.2.0 |
| 2022-03 | RAN#95 | R5-220993 | 1695 | - | F | Update of Annex for NR SA FR1 L1-SINR measurement test cases | 17.2.0 |
| 2022-03 | RAN#95 | R5-221001 | 1696 | - | F | Correction to Annex H.3.6A RRC messages for L1-SINR measurement | 17.2.0 |
| 2022-03 | RAN#95 | R5-221008 | 1698 | - | F | Editorial corrections for EditHelp comments s1 to s3 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221009 | 1699 | - | F | Editorial corrections for EditHelp comments s4 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221010 | 1700 | - | F | Editorial corrections to revert headers to h6 for s4 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221011 | 1701 | - | F | Editorial corrections for EditHelp comments s5 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221012 | 1702 | - | F | Editorial corrections to revert headers to h6 for s5 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221013 | 1703 | - | F | Editorial corrections for EditHelp comments s6.1 to s6.3 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221014 | 1704 | - | F | Editorial corrections for EditHelp comments s6.4 to s6.5 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221015 | 1705 | - | F | Editorial corrections for EditHelp comments s7 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221016 | 1706 | - | F | Editorial corrections for EditHelp comments s8 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221017 | 1707 | - | F | Editorial corrections to revert headers to h6 for s8 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221018 | 1708 | - | F | Editorial corrections for EditHelp comments Annexes | 17.2.0 |
| 2022-03 | RAN#95 | R5-221019 | 1709 | - | F | New section for NE-DC RRM test cases | 17.2.0 |
| 2022-03 | RAN#95 | R5-221020 | 1710 | - | F | Minimum requirements for EN-DC FR1 L1-RSRP test cases | 17.2.0 |
| 2022-03 | RAN#95 | R5-221021 | 1711 | - | F | Minimum requirements for EN-DC FR2 L1-RSRP test cases | 17.2.0 |
| 2022-03 | RAN#95 | R5-221024 | 1714 | - | F | Correct message contents reference iRAT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221025 | 1715 | - | F | Add auxiliary band for n39 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221026 | 1716 | - | F | Corrections to 5.7.x.x | 17.2.0 |
| 2022-03 | RAN#95 | R5-221027 | 1717 | - | F | Corrections to 8.4.2.4 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221028 | 1718 | - | F | Corrections to 8.2.1.1 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221029 | 1719 | - | F | Corrections to 8.5.2.2.1 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221031 | 1721 | - | F | Correction reportAmount for B2 report | 17.2.0 |
| 2022-03 | RAN#95 | R5-221033 | 1723 | - | F | Corrections to message contents for 8.4.2.3 and 8.4.2.4 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221187 | 1728 | - | F | Correct message content for RLM-SSB Based FR2 5.5.1.1 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221188 | 1729 | - | F | Correct message content for RLM-SSB Based FR2 5.5.1.2 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221189 | 1730 | - | F | Correct message content for RLM-SSB Based FR2 5.5.1.3 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221190 | 1731 | - | F | Correct message content for RLM-SSB Based FR2 5.5.1.4 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221191 | 1732 | - | F | Correct message content for FR2 interruption | 17.2.0 |
| 2022-03 | RAN#95 | R5-221254 | 1733 | - | F | Update 4.5.7.1 test procedure | 17.2.0 |
| 2022-03 | RAN#95 | R5-221256 | 1734 | - | F | Update number of HARQ processes for FDD | 17.2.0 |
| 2022-03 | RAN#95 | R5-221289 | 1736 | - | F | Correction of RRM re-establishment test case 7.3.2.1.1 including Test Tolerance | 17.2.0 |
| 2022-03 | RAN#95 | R5-221290 | 1737 | - | F | Correction of RRM re-establishment test case 7.3.2.1.2 including Test Tolerance | 17.2.0 |
| 2022-03 | RAN#95 | R5-221291 | 1738 | - | F | Test Tolerance addition in Annex F for RRM FR2 re-establishment test cases | 17.2.0 |
| 2022-03 | RAN#95 | R5-221297 | 1739 | - | F | Addition of Minimum conformance requirements for DL interruptions at switching between two uplink carriers | 17.2.0 |
| 2022-03 | RAN#95 | R5-221298 | 1740 | - | F | Addition of UL switching test case 6.5.7.1 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221300 | 1742 | - | F | Cell configuration mapping for UL switching test cases | 17.2.0 |
| 2022-03 | RAN#95 | R5-221301 | 1743 | - | F | Addition of Idle mode CA/DC measurement test case 6.6.9.1 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221302 | 1744 | - | F | Addition of Idle Mode measurements of inter-RAT CA candidate cells for early reporting test case 6.6.15.1 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221370 | 1748 | - | F | Update to PDSCH reference measurement channels for RRM test cases with DRX config | 17.2.0 |
| 2022-03 | RAN#95 | R5-221637 | 1613 | 1 | F | Correction to FR2 NR SA RRM TC 7.6.1.2 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221638 | 1615 | 1 | F | Correction to FR2 NR SA RRM TC 7.6.1.4 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221645 | 1619 | 1 | F | Correction to Mob_enh RRM TC 7.3.1.4 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221646 | 1620 | 1 | F | Correction to Mob_enh RRM TC 7.3.1.5 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221650 | 1685 | 1 | F | Completion 4.5.5.5 including TT analysis results | 17.2.0 |
| 2022-03 | RAN#95 | R5-221651 | 1686 | 1 | F | Completion 4.5.5.6 including TT analysis results | 17.2.0 |
| 2022-03 | RAN#95 | R5-221652 | 1687 | 1 | F | Addition of cell configuration MU and TT for Enhancement on MIMO | 17.2.0 |
| 2022-03 | RAN#95 | R5-221653 | 1689 | 1 | F | Update of 4.6.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX | 17.2.0 |
| 2022-03 | RAN#95 | R5-221654 | 1692 | 1 | F | Update of 6.6.8.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | 17.2.0 |
| 2022-03 | RAN#95 | R5-221655 | 1656 | 1 | F | Correction to NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221713 | 1591 | 1 | F | Correction to FR1 EN-DC RRM TCs - interruption SCC | 17.2.0 |
| 2022-03 | RAN#95 | R5-221714 | 1746 | 1 | F | Introduce Test case for LTE PSCell addition and release in NE-DC | 17.2.0 |
| 2022-03 | RAN#95 | R5-221715 | 1593 | 1 | F | Correction to FR2 EN-DC RRM TCs - RLM | 17.2.0 |
| 2022-03 | RAN#95 | R5-221716 | 1594 | 1 | F | Correction to FR2 EN-DC RRM TC 5.5.5.1 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221717 | 1595 | 1 | F | Correction to FR2 EN-DC RRM TC 5.5.5.2 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221718 | 1596 | 1 | F | Correction to FR2 EN-DC RRM TC 5.5.5.3 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221719 | 1597 | 1 | F | Correction to FR2 EN-DC RRM TC 5.5.5.4 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221720 | 1598 | 1 | F | Correction to FR2 EN-DC RRM TC 5.5.5.5 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221721 | 1599 | 1 | F | Correction to FR2 EN-DC RRM TC 5.6.1.1 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221722 | 1601 | 1 | F | Correction to FR2 EN-DC RRM TC 5.6.1.3 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221723 | 1712 | 1 | F | New test case 5.7.4.1 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221724 | 1713 | 1 | F | New test case 5.7.4.2 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221725 | 1725 | 1 | F | Update to RRM test case with CDRX | 17.2.0 |
| 2022-03 | RAN#95 | R5-221726 | 1720 | 1 | F | Corrections to 6.7.x.x | 17.2.0 |
| 2022-03 | RAN#95 | R5-221727 | 1607 | 1 | F | Correction to FR2 NR SA RRM TC 7.5.5.1 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221728 | 1608 | 1 | F | Correction to FR2 NR SA RRM TC 7.5.5.2 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221729 | 1609 | 1 | F | Correction to FR2 NR SA RRM TC 7.5.5.3 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221730 | 1610 | 1 | F | Correction to FR2 NR SA RRM TC 7.5.5.4 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221731 | 1611 | 1 | F | Correction to FR2 NR SA RRM TC 7.5.5.5 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221732 | 1612 | 1 | F | Correction to FR2 NR SA RRM TC 7.6.1.1 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221733 | 1614 | 1 | F | Correction to FR2 NR SA RRM TC 7.6.1.3 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221734 | 1667 | 1 | F | Correction to message exception for 8.4.2.x with SSB index detection | 17.2.0 |
| 2022-03 | RAN#95 | R5-221735 | 1726 | 1 | F | Test procedure update to L2NR Cell reselection test case | 17.2.0 |
| 2022-03 | RAN#95 | R5-221736 | 1616 | 1 | F | Correction to Annex F for FR2 SSB based BFD TCs | 17.2.0 |
| 2022-03 | RAN#95 | R5-221737 | 1617 | 1 | F | Correction to Annex F for FR2 SSB based intra-freq measurement TCs | 17.2.0 |
| 2022-03 | RAN#95 | R5-221738 | 1618 | 1 | F | Correction to default configuration in Annex H | 17.2.0 |
| 2022-03 | RAN#95 | R5-221739 | 1722 | 1 | F | Updates to CSI-ReportConfig for FR2 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221740 | 1724 | 1 | F | Update report config for iRAT test cases | 17.2.0 |
| 2022-03 | RAN#95 | R5-221741 | 1745 | 1 | F | Correction of clause 3 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221742 | 1749 | 1 | F | Correction to test frequencies for intra-band contiguous CA | 17.2.0 |
| 2022-03 | RAN#95 | R5-221798 | 1741 | 1 | F | Addition of UL switching test case 6.5.7.2 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221812 | 1621 | 1 | F | Correction to Mob_enh RRM TC 7.3.3.1 with TT | 17.2.0 |
| 2022-03 | RAN#95 | R5-221813 | 1622 | 1 | F | Correction to Annex F for Mob_enh RRM TCs | 17.2.0 |
| 2022-03 | RAN#95 | R5-221821 | 1629 | 1 | F | Addition of NR SL RRM TC 9.1.2.1 - SLSS Tx NR Cell | 17.2.0 |
| 2022-03 | RAN#95 | R5-221822 | 1630 | 1 | F | Addition of NR SL RRM TC 9.1.2.2 - SLSS Tx SyncRef UE | 17.2.0 |
| 2022-03 | RAN#95 | R5-221823 | 1632 | 1 | F | Addition of NR SL RRM TC 9.1.3.1 - SyncRef reselection GNSS | 17.2.0 |
| 2022-03 | RAN#95 | R5-221824 | 1633 | 1 | F | Addition of NR SL RRM TC 9.1.3.2 - SyncRef reselection Cell | 17.2.0 |
| 2022-03 | RAN#95 | R5-221825 | 1636 | 1 | F | Addition of NR SL RRM TC 9.1.4.2 - Resource pre-emption | 17.2.0 |
| 2022-03 | RAN#95 | R5-221826 | 1637 | 1 | F | Addition of NR SL RRM TC 9.1.4.3 - Resource re-evaluation | 17.2.0 |
| 2022-03 | RAN#95 | R5-221827 | 1640 | 1 | F | Addition of NR SL RRM TC 9.1.5.2 - SL-RSSI PC5 only | 17.2.0 |
| 2022-03 | RAN#95 | R5-221828 | 1643 | 1 | F | Addition of RMC for NR SL RRM test in Annex A | 17.2.0 |
| 2022-03 | RAN#95 | R5-221833 | 1648 | 1 | F | Addition of 5.5.5.6 FR2 SCell BFD in non-DRX for NSA | 17.2.0 |
| 2022-03 | RAN#95 | R5-221834 | 1649 | 1 | F | Addition of 5.5.5.7 FR2 SCell BFD in DRX for NSA | 17.2.0 |
| 2022-03 | RAN#95 | R5-221835 | 1651 | 1 | F | Addition of 7.5.5.6 FR2 SCell BFD in non-DRX for SA | 17.2.0 |
| 2022-03 | RAN#95 | R5-221836 | 1652 | 1 | F | Addition of 7.5.5.7 FR2 SCell BFD in DRX for SA | 17.2.0 |
| 2022-03 | RAN#95 | R5-221837 | 1654 | 1 | F | Update to Annex F for eMIMO test cases | 17.2.0 |
| 2022-03 | RAN#95 | R5-221838 | 1655 | 1 | F | Update to Annex H for eMIMO test cases | 17.2.0 |
| 2022-03 | RAN#95 | R5-221839 | 1697 | 1 | F | Update of RRM Test Cases for UE Enhancements on MIMO | 17.2.0 |
| 2022-03 | RAN#95 | R5-221859 | 1664 | 1 | F | Correction to DRX setting of 4.6.4.5 and 6.6.4.5 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221893 | 1590 | 1 | F | Add test case 7.3.2.2.3 for NR SA FR2 2-step RACH | 17.2.0 |
| 2022-03 | RAN#95 | R5-221923 | 1660 | 1 | F | Correction to test procedure of SFTD measurement accuracy | 17.2.0 |
| 2022-03 | RAN#95 | R5-221924 | 1677 | 1 | F | Correction to BWP configuration for 4.5.6.1.2 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221925 | 1673 | 1 | F | Correction to active BWP ID and TRS configuration in 6.5.6.1.1 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221926 | 1675 | 1 | F | Correction to CSI-RS offset for 6.5.3.1 | 17.2.0 |
| 2022-03 | RAN#95 | R5-221927 | 1676 | 1 | F | Correction to CSI report offset for 6.5.3.1 | 17.2.0 |
| 2022-06 | RAN#96 | R5-222490 | 1752 | - | F | Correction to test time in 6.1.1.7 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222491 | 1753 | - | F | Editorial correction in 6.1.2.5 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222492 | 1754 | - | F | Correction to physical cell identity in 6.6.3.2 and 6.6.3.3 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222493 | 1755 | - | F | Correction to test procedure in 8.2.1.2 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222494 | 1756 | - | F | Correction to test procedure in 8.5.1.1 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222505 | 1758 | - | F | Correction to test parameters in 5.6.1.x and 5.6.2.x | 17.3.0 |
| 2022-06 | RAN#96 | R5-222507 | 1760 | - | F | Correction to DRX offset setting in 6.6.1.7 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222508 | 1761 | - | F | Correction to DRX offset setting in 6.6.3.3 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222509 | 1762 | - | F | Correction to DRX offset setting in 8.4.2.x | 17.3.0 |
| 2022-06 | RAN#96 | R5-222517 | 1765 | - | F | Add minimum requirements for 7.7.4 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222522 | 1769 | - | F | Remove incorrect references - Chapter 4 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222523 | 1770 | - | F | Remove incorrect references - Chapter 5 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222524 | 1771 | - | F | Remove incorrect references - Chapter 6 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222525 | 1772 | - | F | Remove incorrect references - Chapter 7 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222526 | 1773 | - | F | Remove incorrect references - Chapter 8 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222527 | 1774 | - | F | Corrections to 6.6.3.1 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222528 | 1775 | - | F | Corrections to 8.4.1.2 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222529 | 1776 | - | F | Modification to the asynchronous / synchronous cells conditions | 17.3.0 |
| 2022-06 | RAN#96 | R5-222531 | 1778 | - | F | Clean-up asynchronous / synchronous cells conditions for IRAT | 17.3.0 |
| 2022-06 | RAN#96 | R5-222532 | 1779 | - | F | Editorial correction to H.3.4-5 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222533 | 1780 | - | F | Corrections to 5.6.1.3 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222534 | 1781 | - | F | Corrections to 5.6.1.4 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222535 | 1782 | - | F | Correction to H.3.4-7 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222536 | 1783 | - | F | Editorial correction 4.5.5.x | 17.3.0 |
| 2022-06 | RAN#96 | R5-222633 | 1791 | - | F | Correction to NR SL RRM TCs | 17.3.0 |
| 2022-06 | RAN#96 | R5-222636 | 1792 | - | F | Correction to FR1 EN-DC TC 4.7.5.1 - SFTD | 17.3.0 |
| 2022-06 | RAN#96 | R5-222638 | 1794 | - | F | Correction to FR1 NR SA TCs 6.1.2.2 - low priority reselection | 17.3.0 |
| 2022-06 | RAN#96 | R5-222640 | 1796 | - | F | Correction to inter-RAT TC 8.5.1.1 - SFTD | 17.3.0 |
| 2022-06 | RAN#96 | R5-222882 | 1803 | - | F | Editorial update to minimum requirement in 6.5.7.0 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222887 | 1804 | - | F | Update to minimum requirements for BFR | 17.3.0 |
| 2022-06 | RAN#96 | R5-222890 | 1807 | - | F | Update to FR2 SCell BFD test cases | 17.3.0 |
| 2022-06 | RAN#96 | R5-222962 | 1810 | - | F | Corrections AoA setup references | 17.3.0 |
| 2022-06 | RAN#96 | R5-222964 | 1812 | - | F | Corrections 7.6.3.1 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222965 | 1813 | - | F | Corrections 7.6.3.2 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222966 | 1814 | - | F | Corrections 7.6.3.3 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222967 | 1815 | - | F | Corrections 7.6.3.4 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222968 | 1816 | - | F | Update to 4.3.2.2.3 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222969 | 1817 | - | F | Update to 4.3.2.2.4 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222970 | 1818 | - | F | Update to 6.3.2.2.3 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222971 | 1819 | - | F | Update to 6.3.2.2.4 | 17.3.0 |
| 2022-06 | RAN#96 | R5-222998 | 1821 | - | F | Update of 4.6.7 EN-DC FR1 L1-SINR measurement procedure | 17.3.0 |
| 2022-06 | RAN#96 | R5-222999 | 1822 | - | F | Update of 6.6.8 NR SA FR1 L1-SINR measurement procedure | 17.3.0 |
| 2022-06 | RAN#96 | R5-223001 | 1824 | - | F | Addition of 5.6.6.1 EN-DC FR2 L1-SINR measurement including Test Tolerance | 17.3.0 |
| 2022-06 | RAN#96 | R5-223002 | 1825 | - | F | Addition of 5.6.6.2 EN-DC FR2 L1-SINR measurement including Test Tolerance | 17.3.0 |
| 2022-06 | RAN#96 | R5-223004 | 1827 | - | F | Addition of 7.6.6.1 NR SA FR2 L1-SINR measurement including Test Tolerance | 17.3.0 |
| 2022-06 | RAN#96 | R5-223007 | 1830 | - | F | Addition of Annex E and Annex F for FR2 L1-SINR measurement | 17.3.0 |
| 2022-06 | RAN#96 | R5-223214 | 1835 | - | F | Editorial correction to 5G RRM TCs | 17.3.0 |
| 2022-06 | RAN#96 | R5-223246 | 1836 | - | F | Correction to Annexes for RRM SCell activation test cases | 17.3.0 |
| 2022-06 | RAN#96 | R5-223305 | 1837 | - | F | Update to eMIMO test cases 4.5.5.5 and 4.5.5.6 | 17.3.0 |
| 2022-06 | RAN#96 | R5-223605 | 1826 | 1 | F | Addition of 5.6.6.3 EN-DC FR2 L1-SINR measurement including Test Tolerance | 17.3.0 |
| 2022-06 | RAN#96 | R5-223606 | 1829 | 1 | F | Addition of 7.6.6.3 NR SA FR2 L1-SINR measurement including Test Tolerance | 17.3.0 |
| 2022-06 | RAN#96 | R5-223607 | 1750 | 1 | F | Correction to EN-DC FR2 RLM tests for PSCell configured with CSI-RS-based RLM-RS in non-DRX including TT | 17.3.0 |
| 2022-06 | RAN#96 | R5-223615 | 1763 | 1 | F | Completing 5.7.4.1 including TT analysis | 17.3.0 |
| 2022-06 | RAN#96 | R5-223616 | 1766 | 1 | F | Completing 7.7.4.1 including TT analysis | 17.3.0 |
| 2022-06 | RAN#96 | R5-223707 | 1823 | 1 | F | Addition of minimum requirements for EN-DC FR2 L1-SINR measurement for beam reporting | 17.3.0 |
| 2022-06 | RAN#96 | R5-223710 | 1786 | 1 | F | Introduction of EN-DC FR2 SRS-RSRP measurement in non-DRX test case 5.6.4.1 | 17.3.0 |
| 2022-06 | RAN#96 | R5-223711 | 1787 | 1 | F | Introduction of EN-DC FR2 SRS-RSRP measurement accuracy test case 5.7.5.1 | 17.3.0 |
| 2022-06 | RAN#96 | R5-223712 | 1797 | 1 | F | Introduction of NR SA FR2 SRS-RSRP measurement in non-DRX test case 7.6.4.1 | 17.3.0 |
| 2022-06 | RAN#96 | R5-223713 | 1798 | 1 | F | Introduction of NR SA FR2 SRS-RSRP measurement accuracy test case 7.7.5.1 | 17.3.0 |
| 2022-06 | RAN#96 | R5-223850 | 1777 | 1 | F | Corrections to 4.7.5.1 | 17.3.0 |
| 2022-06 | RAN#96 | R5-223851 | 1784 | 1 | F | Corrections to 4.5.5.1 | 17.3.0 |
| 2022-06 | RAN#96 | R5-223853 | 1808 | 1 | F | Remove condition asynchronous cells | 17.3.0 |
| 2022-06 | RAN#96 | R5-223854 | 1757 | 1 | F | Correction to CSI-RS for tracking in 5.6.1.2 | 17.3.0 |
| 2022-06 | RAN#96 | R5-223855 | 1764 | 1 | F | Completing 5.7.4.2 including TT analysis | 17.3.0 |
| 2022-06 | RAN#96 | R5-223856 | 1790 | 1 | F | Update to FR2 interruption test case 5.5.2.1 | 17.3.0 |
| 2022-06 | RAN#96 | R5-223857 | 1793 | 1 | F | Correction to FR2 EN-DC BFD TCs | 17.3.0 |
| 2022-06 | RAN#96 | R5-223858 | 1809 | 1 | F | Corrections to 6.6.3.2 | 17.3.0 |
| 2022-06 | RAN#96 | R5-223859 | 1751 | 1 | F | Editorial reference correction to NR SA FR2 cell re-selection test requirements | 17.3.0 |
| 2022-06 | RAN#96 | R5-223860 | 1767 | 1 | F | Completing 7.7.4.2 including TT analysis | 17.3.0 |
| 2022-06 | RAN#96 | R5-223861 | 1795 | 1 | F | Correction to FR2 NR SA BFD TCs | 17.3.0 |
| 2022-06 | RAN#96 | R5-223862 | 1831 | 1 | F | Correction of RRM test case 7.7.1.1 | 17.3.0 |
| 2022-06 | RAN#96 | R5-223863 | 1768 | 1 | F | Annex F for L1-RSRP meas accuracy test cases | 17.3.0 |
| 2022-06 | RAN#96 | R5-223864 | 1788 | 1 | F | Alignment of RMC note for DRX test cases | 17.3.0 |
| 2022-06 | RAN#96 | R5-223870 | 1785 | 2 | F | Corrections to 4.5.5.2 | 17.3.0 |
| 2022-06 | RAN#96 | R5-223874 | 1759 | 1 | F | Correction to Active UL BWP-2 Configuration in 4.5.6.1.1 and 6.5.6.1.2 | 17.3.0 |
| 2022-06 | RAN#96 | R5-223878 | 1832 | 1 | F | Correction of UL switching test case 6.5.7.1 including Test Tolerance | 17.3.0 |
| 2022-06 | RAN#96 | R5-223879 | 1833 | 1 | F | Correction of UL switching test case 6.5.7.2 including Test Tolerance | 17.3.0 |
| 2022-06 | RAN#96 | R5-223880 | 1805 | 1 | F | Addition of eMIMO test case 6.5.5.5 | 17.3.0 |
| 2022-06 | RAN#96 | R5-223881 | 1806 | 1 | F | Addition of eMIMO test case 6.5.5.6 | 17.3.0 |
| 2022-06 | RAN#96 | R5-223882 | 1828 | 1 | F | Addition of 7.6.6.2 NR SA FR2 L1-SINR measurement including Test Tolerance | 17.3.0 |
| 2022-06 | RAN#96 | R5-223886 | 1789 | 1 | F | Update to FR1 SCell activation and deactivation test cases | 17.3.0 |
| 2022-07 | RAN#96 | R5-223615 | 1763 | 1 | F | addition of missing changes from R5-223615 in Tables 5.7.4.1.5-2, /-3, /-4 | 17.3.1 |
| 2022-09 | RAN#97 | R5-223926 | 1838 | - | F | Update to 2-step RACH RRM TC 7.3.2.2.4-FR2 non-contention based random access | 17.4.0 |
| 2022-09 | RAN#97 | R5-223965 | 1841 | - | F | Complete 5.7.1.3 including TT analysis results | 17.4.0 |
| 2022-09 | RAN#97 | R5-223966 | 1842 | - | F | Complete 7.7.1.3 including TT analysis results | 17.4.0 |
| 2022-09 | RAN#97 | R5-223977 | 1844 | - | F | Complete TC 4.3.2.2.3 including TT analysis results | 17.4.0 |
| 2022-09 | RAN#97 | R5-223978 | 1845 | - | F | Complete TC 4.3.2.2.4 including TT analysis results | 17.4.0 |
| 2022-09 | RAN#97 | R5-223981 | 1848 | - | F | Annexes for 2-step RACH test cases | 17.4.0 |
| 2022-09 | RAN#97 | R5-224006 | 1851 | - | F | Update of SA FR1 TC 6.3.2.1.1 and 6.3.2.1.2 | 17.4.0 |
| 2022-09 | RAN#97 | R5-224182 | 1854 | - | F | Addition of new test case 4.7.7.1.2 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy | 17.4.0 |
| 2022-09 | RAN#97 | R5-224186 | 1858 | - | F | Addition of new test case 4.7.7.3.2 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy | 17.4.0 |
| 2022-09 | RAN#97 | R5-224468 | 1866 | - | F | Corrections to 5.6.1.3 and 5.6.1.4 | 17.4.0 |
| 2022-09 | RAN#97 | R5-224469 | 1867 | - | F | Corrections to 5.5.5.1 | 17.4.0 |
| 2022-09 | RAN#97 | R5-224470 | 1868 | - | F | Corrections to 5.5.5.2 | 17.4.0 |
| 2022-09 | RAN#97 | R5-224471 | 1869 | - | F | Corrections to 5.5.5.3 | 17.4.0 |
| 2022-09 | RAN#97 | R5-224472 | 1870 | - | F | Corrections for BFD LR test cases | 17.4.0 |
| 2022-09 | RAN#97 | R5-224474 | 1872 | - | F | Core spec alignment 6.7.x.1 test cases | 17.4.0 |
| 2022-09 | RAN#97 | R5-224476 | 1874 | - | F | Correction to CSI-RS.3.2 TDD | 17.4.0 |
| 2022-09 | RAN#97 | R5-224501 | 1877 | - | F | Correction to Mob_enh RRM TCs 7.3.1.X - FR2 DAPS HO | 17.4.0 |
| 2022-09 | RAN#97 | R5-224505 | 1878 | - | F | Correction to minimum requirements in 9.1.1.0 - SL Tx timing error | 17.4.0 |
| 2022-09 | RAN#97 | R5-224506 | 1879 | - | F | Correction to NR SL RRM TC 9.1.1.1 - GNSS with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-224507 | 1880 | - | F | Correction to NR SL RRM TC 9.1.1.2 - SyncRef UE with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-224508 | 1881 | - | F | Correction to NR SL RRM TC 9.1.1.3 - gNB with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-224509 | 1882 | - | F | Correction to NR SL RRM TC 9.1.2.1 - S-SSB Tx gNB with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-224510 | 1883 | - | F | Correction to NR SL RRM TC 9.1.2.2 - S-SSB Tx SyncRef UE with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-224511 | 1884 | - | F | Correction to minimum requirements in 9.1.3.0 - SyncRef reselection | 17.4.0 |
| 2022-09 | RAN#97 | R5-224512 | 1885 | - | F | Correction to NR SL RRM TC 9.1.3.1 - GNSS highest priority with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-224514 | 1887 | - | F | Correction to NR SL RRM TC 9.1.4.1 - resource selection with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-224515 | 1888 | - | F | Correction to NR SL RRM TC 9.1.4.2 - resource pre-emption with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-224517 | 1890 | - | F | Correction to NR SL RRM TC 9.1.5.1 - CBR concurrent with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-224518 | 1891 | - | F | Correction to NR SL RRM TC 9.1.5.2 - CBR PC5 only with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-224519 | 1892 | - | F | Correction to NR SL RRM TC 9.1.6.1 - WAN interruption with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-224546 | 1903 | - | F | Correction to NR HST RRM TC 6.1.1.7 - intra-freq reselection | 17.4.0 |
| 2022-09 | RAN#97 | R5-224553 | 1910 | - | F | Correction to NR HST RRM TC 8.2.1.2 - intra-RAT reselection | 17.4.0 |
| 2022-09 | RAN#97 | R5-224635 | 1912 | - | F | Correction to condition DRX.7 in H.3.7 | 17.4.0 |
| 2022-09 | RAN#97 | R5-224637 | 1914 | - | F | Correction to test requirement for SFTD measurement accuracy | 17.4.0 |
| 2022-09 | RAN#97 | R5-224645 | 1915 | - | F | Correction to measurement delay in 4.6.4.5 | 17.4.0 |
| 2022-09 | RAN#97 | R5-224646 | 1916 | - | F | Correction to RMSI CORESET Reference Measurement Channels in A.1.2 | 17.4.0 |
| 2022-09 | RAN#97 | R5-224647 | 1917 | - | F | Correction to SS-SINR condition in 5.7.3.x | 17.4.0 |
| 2022-09 | RAN#97 | R5-224648 | 1918 | - | F | Correction to Starting PRB index for ULBWP.1.3 in A.8.2 | 17.4.0 |
| 2022-09 | RAN#97 | R5-224768 | 1921 | - | F | Addition of minimum requirement for L1-SINR measurement in 5.7.6.0 | 17.4.0 |
| 2022-09 | RAN#97 | R5-224801 | 1926 | - | F | Update to FR2 L1-RSRP measurement test cases 5.6.3.x | 17.4.0 |
| 2022-09 | RAN#97 | R5-224804 | 1928 | - | F | Test Tolerances and test case introduction for FR2 RLM test cases 7.5.1.x | 17.4.0 |
| 2022-09 | RAN#97 | R5-225145 | 1935 | - | F | Correction of the measurement accuracy test cases in Annex F | 17.4.0 |
| 2022-09 | RAN#97 | R5-225146 | 1936 | - | F | Correction of UL switching test case 6.5.7.1 including Test Tolerance | 17.4.0 |
| 2022-09 | RAN#97 | R5-225147 | 1937 | - | F | Correction of UL switching test case 6.5.7.2 including Test Tolerance | 17.4.0 |
| 2022-09 | RAN#97 | R5-225148 | 1938 | - | F | Addition of Test Tolerance for UL switching test cases in Annex F of TS 38.533 | 17.4.0 |
| 2022-09 | RAN#97 | R5-225149 | 1939 | - | F | Correction of SA FR2-FR2 SS-RSRP measurement accuracy test case 7.7.1.2 including Test Tolerance | 17.4.0 |
| 2022-09 | RAN#97 | R5-225152 | 1942 | - | F | Correction of NR SA FR1 SA Idle mode CA/DC measurement for FR1 test case 6.6.9.1 | 17.4.0 |
| 2022-09 | RAN#97 | R5-225153 | 1943 | - | F | Correction of Idle Mode inter-RAT CA/DC Measurements test case 6.6.15.1 | 17.4.0 |
| 2022-09 | RAN#97 | R5-225154 | 1944 | - | F | Addition of E-UTRA - NR FR1 Early Measurement Reporting 8.2.2.1 test case | 17.4.0 |
| 2022-09 | RAN#97 | R5-225213 | 1946 | - | F | Update on number of HARQ processes for FDD RRM test cases | 17.4.0 |
| 2022-09 | RAN#97 | R5-225230 | 1952 | - | F | Update 6.1.1.3 test procedure | 17.4.0 |
| 2022-09 | RAN#97 | R5-225617 | 1839 | 1 | F | Correction to EN-DC FR2 RLM tests for PSCell configured with CSI-RS-based RLM-RS in DRX including TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-225622 | 1859 | 1 | F | Addition of new test case 6.7.9.3.2 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy | 17.4.0 |
| 2022-09 | RAN#97 | R5-225623 | 1861 | 1 | F | Completion 7.7.6.2 including TT analysis results | 17.4.0 |
| 2022-09 | RAN#97 | R5-225624 | 1862 | 1 | F | Completion 7.7.6.3 including TT analysis results | 17.4.0 |
| 2022-09 | RAN#97 | R5-225628 | 1846 | 1 | F | Complete TC 6.3.2.2.3 including TT analysis results | 17.4.0 |
| 2022-09 | RAN#97 | R5-225629 | 1847 | 1 | F | Complete TC 6.3.2.2.4 including TT analysis results | 17.4.0 |
| 2022-09 | RAN#97 | R5-225630 | 1927 | 1 | F | Test Tolerances for FR2 RLM test cases 5.5.1.x | 17.4.0 |
| 2022-09 | RAN#97 | R5-225631 | 1933 | 1 | F | Correction of ENDC FR2 SSB based L1-RSRP measurement accuracy test case 5.7.4.1 including Test Tolerance | 17.4.0 |
| 2022-09 | RAN#97 | R5-225632 | 1934 | 1 | F | Correction of ENDC FR2 CSI-RS based L1-RSRP measurement accuracy test case 5.7.4.2 including Test Tolerance | 17.4.0 |
| 2022-09 | RAN#97 | R5-225633 | 1940 | 1 | F | Correction of SA FR2 SSB based L1-RSRP measurement accuracy test case 7.7.4.1 including Test Tolerance | 17.4.0 |
| 2022-09 | RAN#97 | R5-225634 | 1941 | 1 | F | Correction of SA FR2 CSI-RS based L1-RSRP measurement accuracy test case 7.7.4.2 including Test Tolerance | 17.4.0 |
| 2022-09 | RAN#97 | R5-225635 | 1843 | 1 | F | Annex F for 5.7.1.3 and 7.7.1.3 | 17.4.0 |
| 2022-09 | RAN#97 | R5-225690 | 1853 | 1 | F | Completion 4.7.7.1.1 and 6.7.9.1.1 including TT analysis results | 17.4.0 |
| 2022-09 | RAN#97 | R5-225691 | 1855 | 1 | F | Addition of new test case 6.7.9.1.2 NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy | 17.4.0 |
| 2022-09 | RAN#97 | R5-225692 | 1856 | 1 | F | Completion 4.7.7.2 and 6.7.9.2 including TT analysis results | 17.4.0 |
| 2022-09 | RAN#97 | R5-225693 | 1857 | 1 | F | Completion 4.7.7.3.1 and 6.7.9.3.1 including TT analysis results | 17.4.0 |
| 2022-09 | RAN#97 | R5-225725 | 1955 | 1 | F | Update to Annex E and F for eMIMO test cases | 17.4.0 |
| 2022-09 | RAN#97 | R5-225748 | 1945 | 1 | F | Addition of E-UTRA - NR FR2 Early Measurement Reporting 8.2.2.2 test case | 17.4.0 |
| 2022-09 | RAN#97 | R5-225815 | 1863 | 1 | F | Correction on test applicability of HST TCs | 17.4.0 |
| 2022-09 | RAN#97 | R5-225816 | 1864 | 1 | F | Correction message contents 4.6.4.3 and 4.6.4.4 | 17.4.0 |
| 2022-09 | RAN#97 | R5-225817 | 1947 | 1 | F | Clarification on SFTD delay in 4.7.5.1 | 17.4.0 |
| 2022-09 | RAN#97 | R5-225818 | 1931 | 1 | F | Correction of ENDC FR2 SS-RSRP measurement accuracy test case 5.7.1.1 | 17.4.0 |
| 2022-09 | RAN#97 | R5-225819 | 1949 | 1 | F | Updates on 5.7.4.1 test procedure and test requirements | 17.4.0 |
| 2022-09 | RAN#97 | R5-225820 | 1951 | 1 | F | Updates on 5.7.4.2 test procedure and test requirements | 17.4.0 |
| 2022-09 | RAN#97 | R5-225821 | 1871 | 1 | F | Correction message contents 6.6.4.3 and 6.6.4.4 | 17.4.0 |
| 2022-09 | RAN#97 | R5-225822 | 1930 | 1 | F | Editorial correction to 5G RRM test case 7.6.1.x | 17.4.0 |
| 2022-09 | RAN#97 | R5-225823 | 1948 | 1 | F | Clarification on SFTD delay in 8.5.1.1 | 17.4.0 |
| 2022-09 | RAN#97 | R5-225824 | 1950 | 1 | F | Corrections to Interruptions at SCell addition/release RRM tests | 17.4.0 |
| 2022-09 | RAN#97 | R5-225846 | 1860 | 1 | F | Completion 7.7.6.1 including TT analysis results | 17.4.0 |
| 2022-09 | RAN#97 | R5-225847 | 1922 | 1 | F | Addition of new test case 5.7.6.1 EN-DC FR2 L1-SINR measurement | 17.4.0 |
| 2022-09 | RAN#97 | R5-225848 | 1923 | 1 | F | Addition of new test case 5.7.6.2 EN-DC FR2 L1-SINR measurement | 17.4.0 |
| 2022-09 | RAN#97 | R5-225849 | 1924 | 1 | F | Addition of new test case 5.7.6.3 EN-DC FR2 L1-SINR measurement | 17.4.0 |
| 2022-09 | RAN#97 | R5-225850 | 1932 | 1 | F | Correction of ENDC FR2-FR2 SS-RSRP measurement accuracy test case 5.7.1.2 including Test Tolerance | 17.4.0 |
| 2022-09 | RAN#97 | R5-225851 | 1886 | 1 | F | Correction to NR SL RRM TC 9.1.3.2 - Cell highest priority with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-225852 | 1889 | 1 | F | Correction to NR SL RRM TC 9.1.4.3 - resource re-evaluation with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-225853 | 1893 | 1 | F | Correction to Annex F for NR SL RRM TCs | 17.4.0 |
| 2022-09 | RAN#97 | R5-225855 | 1894 | 1 | F | Correction to NR PS RRM TC 7.1.1.3 - intra-freq reselection low mobility with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-225856 | 1895 | 1 | F | Correction to NR PS RRM TC 7.1.1.4 - intra-freq reselection not-at-cell-edge with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-225857 | 1896 | 1 | F | Correction to NR PS RRM TC 7.1.1.5 - inter-freq reselection low mobility with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-225858 | 1897 | 1 | F | Correction to NR PS RRM TC 7.1.1.6 - inter-freq reselection not-at-cell-edge with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-225859 | 1898 | 1 | F | Correction to Annex F for NR PS RRM TCs | 17.4.0 |
| 2022-09 | RAN#97 | R5-225864 | 1908 | 1 | F | Correction to NR SA FR2 RRM TC 7.1.1.1 - intra-freq reselection with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-225865 | 1909 | 1 | F | Correction to NR SA FR2 RRM TC 7.1.1.2 - inter-freq reselection with TT | 17.4.0 |
| 2022-09 | RAN#97 | R5-225866 | 1911 | 1 | F | Correction to Annex F for NR RRM TCs | 17.4.0 |
| 2022-09 | RAN#97 | R5-225871 | 1919 | 1 | F | Correction to test parameters in 5.6.3.x | 17.4.0 |
| 2022-09 | RAN#97 | R5-225872 | 1925 | 1 | F | Update of eMIMO test case 6.5.5.5 and 6.5.5.6 | 17.4.0 |
| 2022-09 | RAN#97 | R5-225876 | 1876 | 1 | F | Correction to Mob_enh RRM TCs 6.3.1.X - FR1 DAPS HO | 17.4.0 |
| 2022-09 | RAN#97 | R5-225884 | 1899 | 1 | F | Correction to EN-DC FR1 RRM TC 4.5.2.X - interruption | 17.4.0 |
| 2022-09 | RAN#97 | R5-225885 | 1900 | 1 | F | Correction to EN-DC FR1 RRM TC 4.5.3.X - SCell activation | 17.4.0 |
| 2022-09 | RAN#97 | R5-225886 | 1901 | 1 | F | Correction to EN-DC FR1 RRM TC 4.5.6.1.2 - BWP switching | 17.4.0 |
| 2022-09 | RAN#97 | R5-225887 | 1904 | 1 | F | Correction to NR SA FR1 RRM TC 6.5.2.1 - interruption | 17.4.0 |
| 2022-09 | RAN#97 | R5-225888 | 1905 | 1 | F | Correction to NR SA FR1 RRM TC 6.5.3.X - SCell activation | 17.4.0 |
| 2022-09 | RAN#97 | R5-225889 | 1906 | 1 | F | Correction to NR SA FR1 RRM TC 6.5.6.1.1 - BWP switching | 17.4.0 |
| 2022-12 | RAN#98 | R5-225962 | 1956 | - | F | Adding NR bands n91, n92, n93 and n94 to NR band group for FR1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226113 | 1959 | - | F | Addition of minimum requirements for RedCap RRM TCs 16.3.1.0 - FR1 HO | 17.5.0 |
| 2022-12 | RAN#98 | R5-226114 | 1960 | - | F | Addition of RedCap RRM TC 16.3.1.7 - inter-RAT HO 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-226116 | 1962 | - | F | Addition of RedCap RRM TC 16.3.1.9 - inter-RAT blind HO 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-226118 | 1964 | - | F | Addition of minimum requirements for RedCap RRM TCs 16.3.2.1.0 - FR1 Reestablish | 17.5.0 |
| 2022-12 | RAN#98 | R5-226119 | 1965 | - | F | Addition of RedCap RRM TC 16.3.2.1.1 - intra reestablish 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-226120 | 1966 | - | F | Addition of RedCap RRM TC 16.3.2.1.2 - intra reestablish 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226121 | 1967 | - | F | Addition of RedCap RRM TC 16.3.2.1.3 - inter reestablish 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-226122 | 1968 | - | F | Addition of RedCap RRM TC 16.3.2.1.4 - inter reestablish 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226123 | 1969 | - | F | Addition of RedCap RRM TC 16.3.2.1.5 - no timing reestablish 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-226124 | 1970 | - | F | Addition of RedCap RRM TC 16.3.2.1.6 - no timing reestablish 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226125 | 1971 | - | F | Addition of minimum requirements for RedCap RRM TCs 16.3.2.2.0 - RACH | 17.5.0 |
| 2022-12 | RAN#98 | R5-226126 | 1972 | - | F | Addition of RedCap RRM TC 16.3.2.2.1 - 4 step CBRA 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-226127 | 1973 | - | F | Addition of RedCap RRM TC 16.3.2.2.2 - 4 step CBRA 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226128 | 1974 | - | F | Addition of RedCap RRM TC 16.3.2.2.3 - 4 step CFRA 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-226129 | 1975 | - | F | Addition of RedCap RRM TC 16.3.2.2.4 - 4 step CFRA 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226134 | 1980 | - | F | Addition of minimum requirements for RedCap RRM TCs 16.3.2.3.0 - Redirection | 17.5.0 |
| 2022-12 | RAN#98 | R5-226135 | 1981 | - | F | Addition of RedCap RRM TC 16.3.2.3.1 - NR redirection 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-226139 | 1985 | - | F | Addition of minimum requirements for RedCap RRM TCs 16.4.1.0 - Tx timing | 17.5.0 |
| 2022-12 | RAN#98 | R5-226142 | 1988 | - | F | Addition of minimum requirements for RedCap RRM TCs 16.5.1.0 - RLM | 17.5.0 |
| 2022-12 | RAN#98 | R5-226143 | 1989 | - | F | Addition of RedCap RRM TC 16.5.1.1 - OOS non-DRX 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-226144 | 1990 | - | F | Addition of RedCap RRM TC 16.5.1.2 - OOS non-DRX 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226145 | 1991 | - | F | Addition of RedCap RRM TC 16.5.1.3 - IS non-DRX 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-226146 | 1992 | - | F | Addition of RedCap RRM TC 16.5.1.4 - IS non-DRX 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226147 | 1993 | - | F | Addition of RedCap RRM TC 16.5.1.7 - IS DRX 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-226148 | 1994 | - | F | Addition of RedCap RRM TC 16.5.1.8 - IS DRX 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226149 | 1995 | - | F | Addition of minimum requirements for RedCap RRM TCs 16.5.2.0 - BFD | 17.5.0 |
| 2022-12 | RAN#98 | R5-226150 | 1996 | - | F | Addition of RedCap RRM TC 16.5.2.1 - SSB BFR 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-226152 | 1998 | - | F | Addition of minimum requirements for RedCap RRM TCs 16.6.1.0 - Intra-freq measurement | 17.5.0 |
| 2022-12 | RAN#98 | R5-226153 | 1999 | - | F | Addition of RedCap RRM TC 16.6.1.3 - no gap DRX 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-226154 | 2000 | - | F | Addition of RedCap RRM TC 16.6.1.4 - no gap DRX 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226155 | 2001 | - | F | Addition of RedCap RRM TC 16.6.1.7 - gap DRX 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-226156 | 2002 | - | F | Addition of RedCap RRM TC 16.6.1.8 - gap DRX 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226157 | 2003 | - | F | Addition of RedCap RRM TC 16.6.1.11 - gap non-DRX SSB index 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-226158 | 2004 | - | F | Addition of RedCap RRM TC 16.6.1.12 - gap non-DRX SSB index 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226159 | 2005 | - | F | Addition of minimum requirements for RedCap RRM TCs 16.6.4.0 - L1-RSRP | 17.5.0 |
| 2022-12 | RAN#98 | R5-226160 | 2006 | - | F | Addition of RedCap RRM TC 16.6.4.1 - L1-RSRP non-DRX 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-226162 | 2008 | - | F | Addition of RedCap RRM TC 16.6.4.3 - L1-RSRP DRX 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-226164 | 2010 | - | F | Addition of minimum requirements for RedCap RRM TCs 17.3.2.1.0 - Reestablish | 17.5.0 |
| 2022-12 | RAN#98 | R5-226165 | 2011 | - | F | Addition of RedCap RRM TC 17.3.2.1.1 - intra reestablish with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226166 | 2012 | - | F | Addition of RedCap RRM TC 17.3.2.1.2 - inter reestablish with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226167 | 2013 | - | F | Addition of RedCap RRM TC 17.3.2.1.3 - no timing reestablish | 17.5.0 |
| 2022-12 | RAN#98 | R5-226168 | 2014 | - | F | Addition of minimum requirements for RedCap RRM TCs 17.5.1.0 - RLM | 17.5.0 |
| 2022-12 | RAN#98 | R5-226169 | 2015 | - | F | Addition of RedCap RRM TC 17.5.1.1 - OOS non-DRX with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226170 | 2016 | - | F | Addition of RedCap RRM TC 17.5.1.2 - IS non-DRX with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226171 | 2017 | - | F | Addition of RedCap RRM TC 17.5.1.4 - IS DRX with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226172 | 2018 | - | F | Addition of minimum requirements for RedCap RRM TCs 17.6.1.0 - Intra-freq measurement | 17.5.0 |
| 2022-12 | RAN#98 | R5-226173 | 2019 | - | F | Addition of RedCap RRM TC 17.6.1.1 - no gap non-DRX with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226174 | 2020 | - | F | Addition of minimum requirements for RedCap RRM TCs 18.2.1.0 - L2N HO | 17.5.0 |
| 2022-12 | RAN#98 | R5-226175 | 2021 | - | F | Addition of RedCap RRM TC 18.2.1.1 - L2N HO with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226176 | 2022 | - | F | Addition of minimum requirements for RedCap RRM TCs 18.2.2.0 - L2N redirection | 17.5.0 |
| 2022-12 | RAN#98 | R5-226177 | 2023 | - | F | Addition of RedCap RRM TC 18.2.2.1 - L2N redirection with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-226178 | 2024 | - | F | Correction to Clause 3 for RedCap RRM TCs | 17.5.0 |
| 2022-12 | RAN#98 | R5-226180 | 2026 | - | F | Correction to Annex B for RedCap RRM TCs | 17.5.0 |
| 2022-12 | RAN#98 | R5-226181 | 2027 | - | F | Correction to Annex E for RedCap RRM TCs | 17.5.0 |
| 2022-12 | RAN#98 | R5-226183 | 2029 | - | F | Correction to Annex G for RedCap RRM TCs | 17.5.0 |
| 2022-12 | RAN#98 | R5-226207 | 2030 | - | F | Correction to NR RRM TCs 4.5.2.X and 6.5.2.X - interruption | 17.5.0 |
| 2022-12 | RAN#98 | R5-226208 | 2031 | - | F | Correction to NR RRM TCs 4.5.3.X and 6.5.3.X - SCell activation | 17.5.0 |
| 2022-12 | RAN#98 | R5-226209 | 2032 | - | F | Correction to NR RRM TCs 4.5.6.1.2 and 6.5.6.1.1 - BWP switching | 17.5.0 |
| 2022-12 | RAN#98 | R5-226212 | 2035 | - | F | Correction to cell mapping for RRM TC 6.3.2.3.1 - redirection | 17.5.0 |
| 2022-12 | RAN#98 | R5-226233 | 2039 | - | F | Correction of Inter-RAT TC 8.2.1.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226374 | 2041 | - | F | Correction to FR2 RRC-based DL active BWP switch | 17.5.0 |
| 2022-12 | RAN#98 | R5-226497 | 2044 | - | F | Corrections to 4.6.4.4 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226501 | 2048 | - | F | Corrections to 5.6.3.3 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226502 | 2049 | - | F | Corrections to 6.6.4.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226503 | 2050 | - | F | Corrections to 6.6.4.2 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226504 | 2051 | - | F | Corrections to 6.6.4.3 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226505 | 2052 | - | F | Corrections to 6.6.4.4 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226506 | 2053 | - | F | Corrections to 6.5.5.3 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226507 | 2054 | - | F | Corrections to 6.5.5.4 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226508 | 2055 | - | F | Corrections to 6.5.6.2.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226509 | 2056 | - | F | Corrections to 8.4.2.9 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226510 | 2057 | - | F | Corrections to 8.2.1.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226511 | 2058 | - | F | Corrections to 8.2.1.2 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226516 | 2060 | - | F | Update to RRM TC 4A.1.1.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226647 | 2063 | - | F | Update to test case 6.6.3.1 and 6.6.3.2 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226649 | 2065 | - | F | Addition of test case 7.4.3.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226650 | 2066 | - | F | Addition of test case 7.5.2.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226652 | 2067 | - | F | Addition of test 7.3.1.1, 7.3.1.2, 7.3.1.3 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226653 | 2068 | - | F | Update to tests 5.5.2.3 and 5.5.2.4 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226698 | 2076 | - | F | Update on the delta(Max-Min) rules for 5.7.1.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226699 | 2077 | - | F | Update on the delta(Max-Min) rules for 5.7.1.2 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226775 | 2078 | - | F | Addition of CCA model, RMC into Annex for NR-U | 17.5.0 |
| 2022-12 | RAN#98 | R5-227003 | 2085 | - | F | Update on the delta(Max-Min) rules for 7.7.1.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227004 | 2086 | - | F | Update on the delta(Max-Min) rules for 7.7.1.2 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227005 | 2087 | - | F | Corrections to 7.5.1.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227006 | 2088 | - | F | Corrections to 7.5.1.2 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227007 | 2089 | - | F | Corrections to 7.5.1.3 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227008 | 2090 | - | F | Corrections to 7.5.1.4 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227096 | 2094 | - | F | Correction to RadioLinkMonitoringConfig | 17.5.0 |
| 2022-12 | RAN#98 | R5-227097 | 2095 | - | F | Correction to CSI-Report periodicity in procedure of 5.7.4.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227098 | 2096 | - | F | Correction to EUTRA PCell and NR PSCell interruption requirement in 4.5.6.1.2 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227100 | 2098 | - | F | Correction to radioBearerConfig in RRCReconfiguration-HO | 17.5.0 |
| 2022-12 | RAN#98 | R5-227182 | 2101 | - | F | Correction of minimum conformance requirements for E-UTRA - NR Early Measurement Reporting test cases | 17.5.0 |
| 2022-12 | RAN#98 | R5-227188 | 2105 | - | F | Minimum conformance requirements for EN-DC FR1 interruptions at SRS carrier-based switching test cases | 17.5.0 |
| 2022-12 | RAN#98 | R5-227189 | 2106 | - | F | Addition of EN-DC FR1 interruptions at NR SRS carrier based switching test case 4.5.2.8 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227190 | 2107 | - | F | Addition of EN-DC FR1 interruptions at NR SRS carrier based switching test case 4.5.2.9 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227191 | 2108 | - | F | Minimum conformance requirements for EN-DC FR2 interruptions at SRS carrier-based switching test cases | 17.5.0 |
| 2022-12 | RAN#98 | R5-227192 | 2109 | - | F | Addition of EN-DC FR2 interruptions at E-UTRA SRS carrier-based switching test case 5.5.2.7 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227193 | 2110 | - | F | Addition of EN-DC FR2 interruptions at NR SRS carrier-based switching test case 5.5.2.8 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227194 | 2111 | - | F | Minimum conformance requirements for SA FR1 interruptions at SRS carrier-based switching test cases | 17.5.0 |
| 2022-12 | RAN#98 | R5-227196 | 2113 | - | F | Minimum conformance requirements for Inter-frequency measurement without measurement gaps | 17.5.0 |
| 2022-12 | RAN#98 | R5-227197 | 2114 | - | F | Addition of SA event triggered reporting with additional mandatory gap pattern test case 6.6.2.9 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227198 | 2115 | - | F | Addition of SA FR1 event triggered reporting without gap when DRX is used test case 6.6.2.10 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227199 | 2116 | - | F | Addition of SA event triggered reporting tests for FR1 without gap when DRX is not used test case 6.6.2.11 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227207 | 2123 | - | F | Correction of minimum conformance requirements for intra-freq measurement cases | 17.5.0 |
| 2022-12 | RAN#98 | R5-227211 | 2127 | - | F | Addition of NR SA FR1 Cell reselection RedCap test case 16.1.1.2 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227212 | 2128 | - | F | Addition of NR SA FR1 Cell reselection RedCap test case 16.1.1.3 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227213 | 2129 | - | F | Addition of NR SA FR1 Cell reselection RedCap test case 16.1.1.4 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227214 | 2130 | - | F | Addition of NR SA FR1 Cell reselection RedCap test case 16.1.1.5 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227215 | 2131 | - | F | Addition of NR SA FR1 Cell reselection RedCap test case 16.1.1.6 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227216 | 2132 | - | F | Addition of NR SA FR1 Cell reselection RedCap test case 16.1.1.7 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227217 | 2133 | - | F | Addition of NR SA FR1 Cell reselection RedCap test case 16.1.1.8 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227335 | 2138 | - | F | Correction in 6.1.1.4 test case | 17.5.0 |
| 2022-12 | RAN#98 | R5-227337 | 2139 | - | F | Correction in 6.1.2.4 test case | 17.5.0 |
| 2022-12 | RAN#98 | R5-227340 | 2142 | - | F | Correction on MU definition for BWP switch test cases | 17.5.0 |
| 2022-12 | RAN#98 | R5-227342 | 2144 | - | F | Updates to L1-RSRP NSA FR1 test cases | 17.5.0 |
| 2022-12 | RAN#98 | R5-227347 | 2147 | - | F | Updates to L1-RSRP SA FR2 test cases | 17.5.0 |
| 2022-12 | RAN#98 | R5-227990 | 2135 | 1 | F | Updates to CLI Measurement test case 5.6.4.1, 5.7.5.1, 7.6.4.1 and 7.7.5.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227643 | 1957 | 2 | F | Update of EN-DC FR1 TC 4.5.7.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-226817 | 2079 | - | F | Update of RRM Test Case 5.5.3.1 EN-DC FR2 SCell activation and deactivation intra-band in non-DRX | 17.5.0 |
| 2022-12 | RAN#98 | R5-227645 | 2093 | 1 | F | Correction to FR2 NSA RLM test cases | 17.5.0 |
| 2022-12 | RAN#98 | R5-227817 | 2071 | 1 | F | Editorial update to RACH tests | 17.5.0 |
| 2022-12 | RAN#98 | R5-227827 | 2025 | 1 | F | Correction to Annex A for RedCap RRM TCs | 17.5.0 |
| 2022-12 | RAN#98 | R5-227828 | 2126 | 1 | F | Addition of NR SA FR1 Cell reselection RedCap test case 16.1.1.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227829 | 2134 | 1 | F | Addition of NR - E-UTRA cell re-selection test cases for RedCap | 17.5.0 |
| 2022-12 | RAN#98 | R5-227830 | 2112 | 1 | F | Addition of NR FR1 interruptions at SRS carrier-based switching test case 6.5.2.2 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227831 | 2080 | 1 | F | Modification of description of tables of test requirement for EN-DC FR2 eMIMO measurement | 17.5.0 |
| 2022-12 | RAN#98 | R5-227832 | 2082 | 1 | F | Modification of description of tables of test requirement for NR SA FR2 eMIMO measurement | 17.5.0 |
| 2022-12 | RAN#98 | R5-227833 | 2059 | 1 | F | Corrections to 6.1.2.5 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227834 | 2141 | 1 | F | Update on priority cell configuration in 8.2.1.2 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227835 | 2061 | 1 | F | Update to RRM TC 4A.1.2 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227836 | 2062 | 1 | F | Update to RRM TC 4A.2.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227837 | 2081 | 1 | F | Modification of description of tables of test requirement for EN-DC FR2 L1-RSRP measurement | 17.5.0 |
| 2022-12 | RAN#98 | R5-227838 | 2083 | 1 | F | Modification of description of tables of test requirement for NR SA FR2 L1-RSRP measurement | 17.5.0 |
| 2022-12 | RAN#98 | R5-227839 | 2092 | 1 | F | Correction to FR2 NSA BFR test cases | 17.5.0 |
| 2022-12 | RAN#98 | R5-227844 | 2104 | 1 | F | Addition of CADC enhancement test cases to Annex E and F including Test Tolerance | 17.5.0 |
| 2022-12 | RAN#98 | R5-227845 | 2028 | 1 | F | Correction to Annex F for RedCap RRM TCs | 17.5.0 |
| 2022-12 | RAN#98 | R5-227861 | 2117 | 1 | F | Correction of minimum conformance requirements for intra-freq measurement cases to include HST_enh | 17.5.0 |
| 2022-12 | RAN#98 | R5-227862 | 2118 | 1 | F | Addition of EN-DC FR1 intra-freq measurement HST_enh test case 4.6.1.8 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227863 | 2119 | 1 | F | Correction of minimum conformance requirements for inter-freq measurement cases to include HST_enh | 17.5.0 |
| 2022-12 | RAN#98 | R5-227864 | 2120 | 1 | F | Addition of EN-DC FR1 inter-freq measurement HST_enh test case 4.6.2.9 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227865 | 2121 | 1 | F | Correction of minimum conformance requirements for NR SA reselection cases | 17.5.0 |
| 2022-12 | RAN#98 | R5-227866 | 2122 | 1 | F | Addition of EN-DC FR1 Cell reselection in HST scenario test case 6.1.1.8 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227867 | 2124 | 1 | F | Addition of SA FR1 intra-freq measurement HST_enh test case 6.6.1.8 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227868 | 2125 | 1 | F | Addition of SA FR1 intra-freq measurement HST_enh test case 6.6.2.12 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227977 | 2099 | 1 | F | Correction of NR SA FR1 Idle mode CA/DC measurement for FR1 test case 6.6.9.1 including Test Tolerance | 17.5.0 |
| 2022-12 | RAN#98 | R5-227978 | 2100 | 1 | F | Correction of Idle Mode inter-RAT CA/DC Measurements test case 6.6.15.1 including Test Tolerance | 17.5.0 |
| 2022-12 | RAN#98 | R5-227979 | 2102 | 1 | F | Correction of E-UTRA - NR FR1 Early Measurement Reporting 8.2.2.1 test case including Test Tolerance | 17.5.0 |
| 2022-12 | RAN#98 | R5-227980 | 2103 | 1 | F | Correction of E-UTRA - NR FR2 Early Measurement Reporting 8.2.2.2 test case including Test Tolerance | 17.5.0 |
| 2022-12 | RAN#98 | R5-227992 | 2073 | 1 | F | Addition of BFD and Link recovery test cases for NR-U | 17.5.0 |
| 2022-12 | RAN#98 | R5-227993 | 2136 | 1 | F | Addition of EN-DC FR1 RLM test cases under CCA configuration | 17.5.0 |
| 2022-12 | RAN#98 | R5-227994 | 2072 | 1 | F | Update to L1-RSRP measurement accuracy test cases | 17.5.0 |
| 2022-12 | RAN#98 | R5-227995 | 2038 | 1 | F | Correction of SA FR1 TC 6.3.2.2.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227996 | 2064 | 1 | F | Addition of test case 7.4.1.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227998 | 2069 | 1 | F | Update to tests 4.5.3.1 and 6.5.3.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-227999 | 2137 | 1 | F | Update on message content for 5.6.1.1 and 5.6.1.3 | 17.5.0 |
| 2022-12 | RAN#98 | R5-228000 | 2143 | 1 | F | Corrections to EN-DC FR2 radio link monitoring TCs 5.5.1.1, 5.1.2, 5.5.1.3 and 5.5.1.4 | 17.5.0 |
| 2022-12 | RAN#98 | R5-228001 | 2036 | 1 | F | Correction to Mob_enh RRM TCs 6.3.1.x and 7.3.1.x - DAPS HO | 17.5.0 |
| 2022-12 | RAN#98 | R5-228002 | 2070 | 1 | F | Update to CSI-RS RRM tables | 17.5.0 |
| 2022-12 | RAN#98 | R5-228003 | 1958 | 1 | F | Update of SA FR2 TC 7.5.7.1 | 17.5.0 |
| 2022-12 | RAN#98 | R5-228004 | 2033 | 1 | F | Correction to NR RRM TCs 4.6.4.X and 6.6.4.X - CSI-RS based L1-RSRP | 17.5.0 |
| 2022-12 | RAN#98 | R5-228013 | 1961 | 1 | F | Addition of RedCap RRM TC 16.3.1.8 - inter-RAT HO 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-228014 | 1963 | 1 | F | Addition of RedCap RRM TC 16.3.1.10 - inter-RAT blind HO 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-228015 | 1976 | 1 | F | Addition of RedCap RRM TC 16.3.2.2.5 - 2 step CBRA 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-228016 | 1977 | 1 | F | Addition of RedCap RRM TC 16.3.2.2.6 - 2 step CBRA 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-228017 | 1978 | 1 | F | Addition of RedCap RRM TC 16.3.2.2.7 - 2 step CFRA 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-228018 | 1979 | 1 | F | Addition of RedCap RRM TC 16.3.2.2.8 - 2 step CFRA 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-228019 | 1982 | 1 | F | Addition of RedCap RRM TC 16.3.2.3.2 - NR redirection 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-228020 | 1983 | 1 | F | Addition of RedCap RRM TC 16.3.2.3.3 - LTE redirection 1 Rx | 17.5.0 |
| 2022-12 | RAN#98 | R5-228021 | 1984 | 1 | F | Addition of RedCap RRM TC 16.3.2.3.4 - LTE redirection 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-228022 | 1986 | 1 | F | Addition of RedCap RRM TC 16.4.1.1 - Tx timing 1 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-228023 | 1987 | 1 | F | Addition of RedCap RRM TC 16.4.1.2 - Tx timing 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-228024 | 1997 | 1 | F | Addition of RedCap RRM TC 16.5.2.2 - SSB BFR 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-228025 | 2007 | 1 | F | Addition of RedCap RRM TC 16.6.4.2 - L1-RSRP non-DRX 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-228026 | 2009 | 1 | F | Addition of RedCap RRM TC 16.6.4.4 - L1-RSRP DRX 2 Rx with TT | 17.5.0 |
| 2022-12 | RAN#98 | R5-228027 | 2140 | 1 | F | Corrections to SA FR1 SCell activation and deactivation delay TCs 6.5.3.1, 6.5.3.2 and 6.5.3.3 | 17.5.0 |
| 2022-12 | RAN#98 | R5-228028 | 2146 | 1 | F | Updates to L1-RSRP SA FR1 test cases | 17.5.0 |
| 2022-12 | RAN#98 | R5-228029 | 2145 | 1 | F | Updates to L1-RSRP NSA FR2 test cases | 17.5.0 |
| 2023-03 | RAN#99 | R5-230045 | 2148 | - | F | Addition of minimum requirements for FR1 6.6.18.0 - concurrent gaps | 17.6.0 |
| 2023-03 | RAN#99 | R5-230046 | 2149 | - | F | Addition of test case 6.6.18.1 - non-overlapping scenario | 17.6.0 |
| 2023-03 | RAN#99 | R5-230047 | 2150 | - | F | Correction to table E.4-1 for concurrent gap TCs. | 17.6.0 |
| 2023-03 | RAN#99 | R5-230076 | 2151 | - | F | Adding NR bands n100, n101 to NR band group for FR1 | 17.6.0 |
| 2023-03 | RAN#99 | R5-230185 | 2154 | - | F | Addition of test case 6.6.18.2 - partial-overlapping scenario | 17.6.0 |
| 2023-03 | RAN#99 | R5-230431 | 2161 | - | F | Update of NR Inter-RAT event triggered reporting tests for FR2 test cases 8.4.2.5 including Test tolerance | 17.6.0 |
| 2023-03 | RAN#99 | R5-230433 | 2162 | - | F | Update of NR Inter-RAT event triggered reporting tests for FR2 test cases including Test Tolerance | 17.6.0 |
| 2023-03 | RAN#99 | R5-230459 | 2170 | - | F | Addition of RedCap RRM TC 16.3.1.1 - intra known HO 1Rx | 17.6.0 |
| 2023-03 | RAN#99 | R5-230465 | 2176 | - | F | Addition of RedCap RRM TC 16.4.3.1 - TA accuracy 1Rx with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-230466 | 2177 | - | F | Addition of RedCap RRM TC 16.4.3.2 - TA accuracy 2Rx with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-230467 | 2178 | - | F | Addition of RedCap RRM TC 16.5.1.9 - OOS non-DRX 1Rx | 17.6.0 |
| 2023-03 | RAN#99 | R5-230470 | 2181 | - | F | Addition of RedCap RRM TC 16.5.1.12 - IS non-DRX 2Rx with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-230471 | 2182 | - | F | Addition of RedCap RRM TC 16.5.1.13 - OOS DRX 1Rx | 17.6.0 |
| 2023-03 | RAN#99 | R5-230474 | 2185 | - | F | Addition of RedCap RRM TC 16.5.1.16 - IS DRX 2Rx with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-230475 | 2186 | - | F | Addition of RedCap RRM TC 16.5.2.5 - BFR non-DRX 1Rx | 17.6.0 |
| 2023-03 | RAN#99 | R5-230476 | 2187 | - | F | Addition of RedCap RRM TC 16.5.2.6 - BFR non-DRX 2Rx with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-230477 | 2188 | - | F | Addition of RedCap RRM TC 16.5.2.7 - BFR DRX 1Rx | 17.6.0 |
| 2023-03 | RAN#99 | R5-230478 | 2189 | - | F | Addition of RedCap RRM TC 16.5.2.8 - BFR DRX 2Rx with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-230483 | 2194 | - | F | Addition of RedCap RRM TC 16.6.1.5 - intra gap based non-DRX 1Rx | 17.6.0 |
| 2023-03 | RAN#99 | R5-230484 | 2195 | - | F | Addition of RedCap RRM TC 16.6.1.6 - intra gap based non-DRX 2Rx with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-230487 | 2198 | - | F | Addition of RedCap RRM TC 16.6.4.5 - CSI-RS L1-RSRP 1Rx | 17.6.0 |
| 2023-03 | RAN#99 | R5-230488 | 2199 | - | F | Addition of RedCap RRM TC 16.6.4.6 - CSI-RS L1-RSRP 2Rx with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-230489 | 2200 | - | F | Addition of RedCap RRM TC 16.6.4.7 - CSI-RS L1-RSRP DRX 1Rx | 17.6.0 |
| 2023-03 | RAN#99 | R5-230491 | 2202 | - | F | Addition of RedCap RRM TC 17.3.2.2.1 - 4-step CBRA | 17.6.0 |
| 2023-03 | RAN#99 | R5-230492 | 2203 | - | F | Addition of RedCap RRM TC 17.3.2.2.2 - 4-step CFRA | 17.6.0 |
| 2023-03 | RAN#99 | R5-230493 | 2204 | - | F | Addition of RedCap RRM TC 17.3.2.2.3 - 2-step CBRA | 17.6.0 |
| 2023-03 | RAN#99 | R5-230494 | 2205 | - | F | Addition of RedCap RRM TC 17.3.2.2.4 - 2-step CFRA | 17.6.0 |
| 2023-03 | RAN#99 | R5-230495 | 2206 | - | F | Addition of RedCap RRM TC 17.5.1.9 - RLM scheduling restriction | 17.6.0 |
| 2023-03 | RAN#99 | R5-230499 | 2210 | - | F | Addition of RedCap RRM TC 17.6.1.3 - intra gap-based non-DRX | 17.6.0 |
| 2023-03 | RAN#99 | R5-230500 | 2211 | - | F | Addition of RedCap RRM TC 17.6.1.4 - intra gap-based DRX | 17.6.0 |
| 2023-03 | RAN#99 | R5-230502 | 2213 | - | F | Addition of RedCap RRM TC 17.6.3.2 - SSB L1-RSRP DRX | 17.6.0 |
| 2023-03 | RAN#99 | R5-230503 | 2214 | - | F | Addition of RedCap RRM TC 17.6.3.3 - CSI-RS L1-RSRP non-DRX | 17.6.0 |
| 2023-03 | RAN#99 | R5-230504 | 2215 | - | F | Addition of RedCap RRM TC 17.6.3.4 - CSI-RS L1-RSRP DRX | 17.6.0 |
| 2023-03 | RAN#99 | R5-230505 | 2216 | - | F | Addition of RedCap RRM TC 18.3.1.1 - FR1 NR meas no-DRX with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-230506 | 2217 | - | F | Addition of RedCap RRM TC 18.3.1.2 - FR1 NR meas DRX with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-230507 | 2218 | - | F | Addition of RedCap RRM TC 18.3.1.3 - FR1 NR meas no-DRX SBI with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-230508 | 2219 | - | F | Addition of RedCap RRM TC 18.3.1.4 - FR1 NR meas DRX SBI with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-230509 | 2220 | - | F | Addition of RedCap RRM TC 18.3.1.5 - FR2 NR meas no-DRX | 17.6.0 |
| 2023-03 | RAN#99 | R5-230510 | 2221 | - | F | Addition of RedCap RRM TC 18.3.1.6 - FR2 NR meas DRX | 17.6.0 |
| 2023-03 | RAN#99 | R5-230511 | 2222 | - | F | Addition of RedCap RRM TC 18.3.1.7 - FR2 NR meas no-DRX SBI | 17.6.0 |
| 2023-03 | RAN#99 | R5-230512 | 2223 | - | F | Addition of RedCap RRM TC 18.3.1.8 - FR2 NR meas DRX SBI | 17.6.0 |
| 2023-03 | RAN#99 | R5-230513 | 2224 | - | F | Correction to Annex A for RedCap RRM TCs | 17.6.0 |
| 2023-03 | RAN#99 | R5-230514 | 2225 | - | F | Correction to Annex E for RedCap RRM TCs | 17.6.0 |
| 2023-03 | RAN#99 | R5-230515 | 2226 | - | F | Correction to Annex F for RedCap RRM TCs | 17.6.0 |
| 2023-03 | RAN#99 | R5-230528 | 2228 | - | F | Correction to Annex A for RRM enhancement TCs | 17.6.0 |
| 2023-03 | RAN#99 | R5-230529 | 2229 | - | F | Correction to Annex E for RRM enhancement TCs | 17.6.0 |
| 2023-03 | RAN#99 | R5-230530 | 2230 | - | F | Correction to Annex F for RRM enhancement TCs | 17.6.0 |
| 2023-03 | RAN#99 | R5-230532 | 2231 | - | F | Correction to FR1 NR SA RRM TC 6.3.1.3 - inter unknown HO | 17.6.0 |
| 2023-03 | RAN#99 | R5-230725 | 2235 | - | F | Correction to FR2 EN-DC test case 5.3.2.2.x | 17.6.0 |
| 2023-03 | RAN#99 | R5-230774 | 2236 | - | F | Update to RRC based BWP switch in FR2 | 17.6.0 |
| 2023-03 | RAN#99 | R5-230777 | 2237 | - | F | Addition of TT for SA FR2 handover test cases | 17.6.0 |
| 2023-03 | RAN#99 | R5-230780 | 2238 | - | F | Addition of SA FR2-FR2 RRC Connection Release with Redirection test case | 17.6.0 |
| 2023-03 | RAN#99 | R5-230782 | 2239 | - | F | Addition of NR-U NSA intra-frequency event-triggered measurement reporting test cases | 17.6.0 |
| 2023-03 | RAN#99 | R5-230783 | 2240 | - | F | Addition of NR-U SA FR1 RLM and BFR test cases | 17.6.0 |
| 2023-03 | RAN#99 | R5-230784 | 2241 | - | F | Update to NR-U NSA RLM and BFR test cases | 17.6.0 |
| 2023-03 | RAN#99 | R5-230867 | 2252 | - | F | Correct Test procedure for RLM-SSB Based FR2 5.5.1.4 | 17.6.0 |
| 2023-03 | RAN#99 | R5-230929 | 2253 | - | F | Update TC 4A.1.1.1 with TT analysis results | 17.6.0 |
| 2023-03 | RAN#99 | R5-230930 | 2254 | - | F | Update TC 4A.2.1.1 with TT analysis results | 17.6.0 |
| 2023-03 | RAN#99 | R5-230931 | 2255 | - | F | Minimum requirements for TC 4A.2.1.1 | 17.6.0 |
| 2023-03 | RAN#99 | R5-230932 | 2256 | - | F | Update Annex F for NE-DC test cases | 17.6.0 |
| 2023-03 | RAN#99 | R5-230938 | 2257 | - | F | Removal of Editor Note for EN-DC FR2 L1-SINR measurement test cases | 17.6.0 |
| 2023-03 | RAN#99 | R5-230939 | 2258 | - | F | Removal of Editor Note for NR SA FR2 L1-SINR measurement test cases | 17.6.0 |
| 2023-03 | RAN#99 | R5-230985 | 2260 | - | F | Clarification on the test procedure of 5.7.1.2 and 7.7.1.2 | 17.6.0 |
| 2023-03 | RAN#99 | R5-230987 | 2262 | - | F | Correction to message exceptions of 5.7.4.2 | 17.6.0 |
| 2023-03 | RAN#99 | R5-230988 | 2263 | - | F | Correction to configuration number of 6.3.2.3.2 | 17.6.0 |
| 2023-03 | RAN#99 | R5-230989 | 2264 | - | F | Correction to Offset value in CSI-RS RMCs table | 17.6.0 |
| 2023-03 | RAN#99 | R5-230990 | 2265 | - | F | Correction to L1-RSRP report delay requirement | 17.6.0 |
| 2023-03 | RAN#99 | R5-231005 | 2267 | - | F | Correction to 4.5.2.6 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231006 | 2268 | - | F | Correction to 4.5.3.x | 17.6.0 |
| 2023-03 | RAN#99 | R5-231007 | 2269 | - | F | Correction to 4.5.5.3 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231009 | 2271 | - | F | Correction to 4.5.6.1.x | 17.6.0 |
| 2023-03 | RAN#99 | R5-231010 | 2272 | - | F | Correction to 4.5.7.1 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231011 | 2273 | - | F | Corrections to 5.6.1.x | 17.6.0 |
| 2023-03 | RAN#99 | R5-231012 | 2274 | - | F | Corrections 5.6.3.3 and 5.6.3.4 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231013 | 2275 | - | F | Correction to RLM test cases EN-DC FR2 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231014 | 2276 | - | F | Correction to RLM test cases NR FR1 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231015 | 2277 | - | F | Correction to 6.5.3.x | 17.6.0 |
| 2023-03 | RAN#99 | R5-231016 | 2278 | - | F | Correction to 6.5.6.1.x | 17.6.0 |
| 2023-03 | RAN#99 | R5-231017 | 2279 | - | F | Correction to RLM test cases NR FR2 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231018 | 2280 | - | F | Correction in Annex F for 5.6.3.4 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231019 | 2281 | - | F | Corrections to 16.3.2.3.2 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231020 | 2282 | - | F | Corrections to 16.6.1.8 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231025 | 2284 | - | F | Addition of eMG TC 6.6.18.3 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231026 | 2285 | - | F | Addition of eMG TC 6.6.18.4 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231111 | 2286 | - | F | Addition of the editors note regarding the principle of testing on mix of E-UTRA and NR FR2 carriers in clause 5 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231112 | 2287 | - | F | Addition of the editors note regarding the principle of testing on mix of E-UTRA and NR FR2 carriers in clause 7 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231114 | 2289 | - | F | Editorial corrections of RedCap test cases | 17.6.0 |
| 2023-03 | RAN#99 | R5-231123 | 2298 | - | F | Addition of NR - E-UTRA cell re-selection test case 16.1.2.1 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231124 | 2299 | - | F | Addition of NR - E-UTRA cell re-selection test case 16.1.2.2 including Test Tolerance | 17.6.0 |
| 2023-03 | RAN#99 | R5-231125 | 2300 | - | F | Addition of NR - E-UTRA cell re-selection test case 16.1.2.3 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231126 | 2301 | - | F | Addition of NR - E-UTRA cell re-selection test case 16.1.2.4 including Test Tolerance | 17.6.0 |
| 2023-03 | RAN#99 | R5-231127 | 2302 | - | F | Addition of NR - E-UTRA cell re-selection test case 16.1.2.5 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231128 | 2303 | - | F | Addition of NR - E-UTRA cell re-selection test case 16.1.2.6 including Test Tolerance | 17.6.0 |
| 2023-03 | RAN#99 | R5-231129 | 2304 | - | F | Addition of new NR SA FR1 Event triggered reporting RedCap test case | 17.6.0 |
| 2023-03 | RAN#99 | R5-231130 | 2305 | - | F | Addition of NR SA FR2 Cell reselection to FR2 intra-frequency NR case for 2 Rx 17.1.1.1 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231131 | 2306 | - | F | Addition of NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for 2 Rx 17.1.1.2 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231132 | 2307 | - | F | Addition of NR SA FR2 Cell reselection to FR2 intra-frequency NR for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE 17.1.1.3 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231133 | 2308 | - | F | Addition of NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE 17.1.1.4 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231134 | 2309 | - | F | Addition of E-UTRA - NR SA FR1 E-UTRA Cell reselection to higher priority NR target Cell in FR1 18.1.1.1 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231135 | 2310 | - | F | Annex E and F correction for RedCap reselection test cases including Test Tolerance | 17.6.0 |
| 2023-03 | RAN#99 | R5-231144 | 2312 | - | F | Correction of E-UTRA - NR FR1 Early Measurement Reporting 8.2.2.1 test case including Test Tolerance | 17.6.0 |
| 2023-03 | RAN#99 | R5-231150 | 2315 | - | F | Correction of EN-DC FR1 HST event triggered reporting test case 4.6.1.8 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231155 | 2320 | - | F | Correction of cell mapping in Annex E for FR1 HST test cases | 17.6.0 |
| 2023-03 | RAN#99 | R5-231223 | 2321 | - | F | Correction of NR SA FR1 Idle mode CA/DC measurement for FR1 test case 6.6.9.1 - resubmission | 17.6.0 |
| 2023-03 | RAN#99 | R5-231234 | 2323 | - | F | Corrections in 7.6.2.2 and 7.6.2.4 Test Procedures | 17.6.0 |
| 2023-03 | RAN#99 | R5-231239 | 2324 | - | F | Update Message Contents 4.5.2.5 and 4.5.2.6 test cases | 17.6.0 |
| 2023-03 | RAN#99 | R5-231240 | 2325 | - | F | Update 5.6.2.4 test applicability | 17.6.0 |
| 2023-03 | RAN#99 | R5-231271 | 2327 | - | F | Update Message Contents 8.4.2.7 and 8.4.2.8 test cases | 17.6.0 |
| 2023-03 | RAN#99 | R5-231272 | 2328 | - | F | Correction to CSI RS based L1-measurement tests 4.6.4.3, 4.6.4.4,6.6.4.3 and 6.6.4.4 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231274 | 2330 | - | F | Updated correct Event A4 in test procedure for EN-DC FR1-FR2 event-triggered reporting | 17.6.0 |
| 2023-03 | RAN#99 | R5-231275 | 2331 | - | F | Update of SA FR1 TC 6.1.1.1 and 6.1.2.1 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231276 | 2332 | - | F | Correction in Measurement uncertainty table Annex F | 17.6.0 |
| 2023-03 | RAN#99 | R5-231317 | 2334 | - | F | Update to test case 6.6.3.1 and 6.6.3.2 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231706 | 2270 | 1 | F | Correction to 4.5.5.4 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231707 | 2259 | 1 | F | Correction to firstActiveUplinkBWP-Id in uplinkConfig for non-contention RA TCs | 17.6.0 |
| 2023-03 | RAN#99 | R5-231708 | 2232 | 1 | F | Correction to FR1 NR SA RRM TC 6.7.3.2.1 - SS-SINR | 17.6.0 |
| 2023-03 | RAN#99 | R5-231709 | 2316 | 1 | F | Correction of EN-DC FR1 HST event triggered reporting test case 4.6.2.9 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231710 | 2319 | 1 | F | Correction of SA FR1 HST event triggered reporting test case 6.6.2.12 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231711 | 2243 | 1 | F | Addition of CG-SDT test case | 17.6.0 |
| 2023-03 | RAN#99 | R5-231712 | 2152 | 1 | F | Addition of CG-SDT RRM test case for FR2 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231713 | 2245 | 1 | F | Add new RRC messages and information elements contents for TS38.533 Annex H.3 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231714 | 2246 | 1 | F | add test case for TS38.533 clause 4.5.5.7 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231715 | 2247 | 1 | F | add test case for TS38.533 clause 4.5.5.8 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231716 | 2248 | 1 | F | add test case for TS38.533 clause 5.5.5.8 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231717 | 2249 | 1 | F | add test case for TS38.533 clause 6.5.5.7 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231718 | 2250 | 1 | F | add test case for TS38.533 clause 7.5.5.9 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231719 | 2251 | 1 | F | add test case for TS38.533 clause 7.5.5.10 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231720 | 2169 | 1 | F | Addition of pass fail limits for CBR test cases | 17.6.0 |
| 2023-03 | RAN#99 | R5-231721 | 2207 | 1 | F | Addition of RedCap RRM TC 17.5.2.3 - CSI-RS BFR non-DRX | 17.6.0 |
| 2023-03 | RAN#99 | R5-231722 | 2208 | 1 | F | Addition of RedCap RRM TC 17.5.2.4 - CSI-RS BFR DRX | 17.6.0 |
| 2023-03 | RAN#99 | R5-231723 | 2209 | 1 | F | Addition of RedCap RRM TC 17.5.2.5 - BFR scheduling restriction | 17.6.0 |
| 2023-03 | RAN#99 | R5-231724 | 2212 | 1 | F | Addition of RedCap RRM TC 17.6.3.1 - SSB L1-RSRP non-DRX | 17.6.0 |
| 2023-03 | RAN#99 | R5-231725 | 2283 | 1 | F | Corrections to 16.6.1.12 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231726 | 2163 | 1 | F | Addition of test case 4.5.3.5 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231727 | 2164 | 1 | F | Addition of test case 5.5.3.7 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231728 | 2165 | 1 | F | Addition of test case 6.5.3.4 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231729 | 2166 | 1 | F | Addition of test case 6.5.3.5 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231730 | 2167 | 1 | F | Addition of test case 7.5.3.4 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231731 | 2168 | 1 | F | Addition of test case 7.5.3.5 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231732 | 2290 | 1 | F | Correction of NR SA FR1 Cell reselection RedCap test case 16.1.1.1 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231733 | 2292 | 1 | F | Correction of NR SA FR1 Cell reselection RedCap test case 16.1.1.3 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231734 | 2294 | 1 | F | Correction of NR SA FR1 Cell reselection RedCap test case 16.1.1.5 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231735 | 2296 | 1 | F | Correction of NR SA FR1 Cell reselection RedCap test case 16.1.1.7 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231736 | 2153 | 1 | F | Adding test case 7.5.11 for UE UL carrier RRC reconfiguration delay | 17.6.0 |
| 2023-03 | RAN#99 | R5-231743 | 2311 | 1 | F | Correction of Idle Mode inter-RAT CA/DC Measurements test case 6.6.15.1 including Test Tolerance | 17.6.0 |
| 2023-03 | RAN#99 | R5-231744 | 2313 | 1 | F | Correction of E-UTRA - NR FR2 Early Measurement Reporting 8.2.2.2 test case including Test Tolerance | 17.6.0 |
| 2023-03 | RAN#99 | R5-231746 | 2295 | 1 | F | Correction of NR SA FR1 Cell reselection RedCap test case 16.1.1.6 including Test Tolerance | 17.6.0 |
| 2023-03 | RAN#99 | R5-231747 | 2297 | 1 | F | Correction of NR SA FR1 Cell reselection RedCap test case 16.1.1.8 including Test Tolerance | 17.6.0 |
| 2023-03 | RAN#99 | R5-231748 | 2314 | 1 | F | Correction of test tolerance for CADC enhancement test cases in Annex F | 17.6.0 |
| 2023-03 | RAN#99 | R5-231759 | 2183 | 1 | F | Addition of RedCap RRM TC 16.5.1.14 - OOS DRX 2Rx with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-231760 | 2291 | 1 | F | Correction of NR SA FR1 Cell reselection RedCap test case 16.1.1.2 including Test Tolerance | 17.6.0 |
| 2023-03 | RAN#99 | R5-231761 | 2293 | 1 | F | Correction of NR SA FR1 Cell reselection RedCap test case 16.1.1.4 including Test Tolerance | 17.6.0 |
| 2023-03 | RAN#99 | R5-231764 | 2160 | 1 | F | Update of RRM Test Case 5.5.3.1 EN-DC FR2 SCell activation and deactivation intra-band in non-DRX including Test Tolerance | 17.6.0 |
| 2023-03 | RAN#99 | R5-231767 | 2233 | 1 | F | Introduction of CLI Measurement test case 5.6.4.2, 5.7.5.2, 7.6.4.2, 7.7.5.2, 6.6.6.1 and 6.7.8.1 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231855 | 2155 | 1 | F | Addition of PRS based UE Rx-Tx measurement FR1 SA test case | 17.6.0 |
| 2023-03 | RAN#99 | R5-231856 | 2157 | 1 | F | Addition of PRS based UE Rx-Tx measurement FR2 SA test case | 17.6.0 |
| 2023-03 | RAN#99 | R5-231872 | 2335 | 1 | F | Update to RRM applicability rules and test optimization - 38.533 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231875 | 2261 | 1 | F | Correction to NSA FR2 RLM test cases | 17.6.0 |
| 2023-03 | RAN#99 | R5-231884 | 2227 | 1 | F | Addition of RRM enh TC 6.5.8.1 - PCell CBW change with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-231893 | 2156 | 2 | F | Addition of TRS based UE Rx-Tx measurement SA FR1 test case | 17.6.0 |
| 2023-03 | RAN#99 | R5-231896 | 2242 | 1 | F | Update to HST RRM test cases | 17.6.0 |
| 2023-03 | RAN#99 | R5-231950 | 2317 | 1 | F | Correction of SA FR1 HST reselection test case 6.1.1.8 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231951 | 2318 | 1 | F | Correction of SA FR1 HST event triggered reporting test case 6.6.1.8 | 17.6.0 |
| 2023-03 | RAN#99 | R5-231954 | 2179 | 1 | F | Addition of RedCap RRM TC 16.5.1.10 - OOS non-DRX 2Rx with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-231955 | 2180 | 1 | F | Addition of RedCap RRM TC 16.5.1.11 - IS non-DRX 1Rx | 17.6.0 |
| 2023-03 | RAN#99 | R5-231956 | 2184 | 1 | F | Addition of RedCap RRM TC 16.5.1.15 - IS DRX 1Rx | 17.6.0 |
| 2023-03 | RAN#99 | R5-231957 | 2190 | 1 | F | Addition of RedCap RRM TC 16.5.4.1 - CBW change 1Rx with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-231958 | 2191 | 1 | F | Addition of RedCap RRM TC 16.5.4.2 - CBW change 2Rx with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-231963 | 2201 | 1 | F | Addition of RedCap RRM TC 16.6.4.8 - CSI-RS L1-RSRP DRX 2Rx with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-231976 | 2171 | 1 | F | Addition of RedCap RRM TC 16.3.1.2 - intra known HO 2Rx with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-231977 | 2172 | 1 | F | Addition of RedCap RRM TC 16.3.1.3 - intra unknown HO 1Rx | 17.6.0 |
| 2023-03 | RAN#99 | R5-231978 | 2173 | 1 | F | Addition of RedCap RRM TC 16.3.1.4 - intra unknown HO 2Rx with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-231979 | 2174 | 1 | F | Addition of RedCap RRM TC 16.3.1.5 - inter unknown HO 1Rx | 17.6.0 |
| 2023-03 | RAN#99 | R5-231980 | 2175 | 1 | F | Addition of RedCap RRM TC 16.3.1.6 - inter unknown HO 2Rx with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-231981 | 2192 | 2 | F | Addition of RedCap RRM TC 16.6.1.1 - gapless non-DRX 1Rx | 17.6.0 |
| 2023-03 | RAN#99 | R5-231982 | 2193 | 2 | F | Addition of RedCap RRM TC 16.6.1.2 - gapless non-DRX 2Rx with TT | 17.6.0 |
| 2023-03 | RAN#99 | R5-231983 | 2196 | 2 | F | Addition of RedCap RRM TC 16.6.1.9 - intra gapless SBI 1Rx | 17.6.0 |
| 2023-03 | RAN#99 | R5-231984 | 2197 | 2 | F | Addition of RedCap RRM TC 16.6.1.10 - intra gapless SBI 2Rx with TT | 17.6.0 |
| 2023-03 | RAN#99 | - | - | - | - | coversheet correction | 17.6.1 |
| 2023-06 | RAN#100 | R5-232097 | 2340 | - | F | Addition of minimum requirements for 6.6.19.0 - FR1 NCSG | 17.7.0 |
| 2023-06 | RAN#100 | R5-232098 | 2341 | - | F | Addition of NCSG RRM test case 6.6.19.1 | 17.7.0 |
| 2023-06 | RAN#100 | R5-232099 | 2342 | - | F | Addition of NCSG RRM test case 6.6.19.2 | 17.7.0 |
| 2023-06 | RAN#100 | R5-232100 | 2343 | - | F | Addition of NCSG RRM test case 6.6.19.3 | 17.7.0 |
| 2023-06 | RAN#100 | R5-232101 | 2344 | - | F | Addition of NCSG RRM test case 6.6.19.4 | 17.7.0 |
| 2023-06 | RAN#100 | R5-232102 | 2345 | - | F | Correction to table E.4-1 and E.4-2 for NCSG TCs | 17.7.0 |
| 2023-06 | RAN#100 | R5-232104 | 2346 | - | F | Addition of minimum requirements for 6.6.17.0 | 17.7.0 |
| 2023-06 | RAN#100 | R5-232130 | 2349 | - | F | Addition to CG-SDT RRM test case for FR2 | 17.7.0 |
| 2023-06 | RAN#100 | R5-232171 | 2350 | - | F | Core spec alignment for RedCap TCs 16.6.1.8 and 16.6.1.12 | 17.7.0 |
| 2023-06 | RAN#100 | R5-232172 | 2351 | - | F | Correction to Test frequencies reference for RedCap TCs in chapter 16 | 17.7.0 |
| 2023-06 | RAN#100 | R5-232174 | 2353 | - | F | Core spec alignment for SMTC value for TC 6.1.1.1 | 17.7.0 |
| 2023-06 | RAN#100 | R5-232175 | 2354 | - | F | Correction to Test frequencies reference for active BWP switch TCs: 4.5.6.1.1, 4.5.6.1.2, 4.5.6.2.1 | 17.7.0 |
| 2023-06 | RAN#100 | R5-232307 | 2369 | - | F | Correction to EN-DC FR2 RLM tests for PSCell configured with CSI-RS-based RLM-RS including TT | 17.7.0 |
| 2023-06 | RAN#100 | R5-232336 | 2370 | - | F | Update concurrent gap test cases 6.6.18.1 and 6.6.18.2 | 17.7.0 |
| 2023-06 | RAN#100 | R5-232426 | 2371 | - | F | Correction to table E.4-1 for Pre-MG TCs | 17.7.0 |
| 2023-06 | RAN#100 | R5-232460 | 2374 | - | F | Correction to RedCap RRM TC 16.5.2.x SSB BFR | 17.7.0 |
| 2023-06 | RAN#100 | R5-232461 | 2375 | - | F | Correction to RedCap RRM TC 16.6.1.x CDSSB intraFreq | 17.7.0 |
| 2023-06 | RAN#100 | R5-232462 | 2376 | - | F | Correction to RedCap RRM TC 16.6.1.x NCDSSB intraFreq | 17.7.0 |
| 2023-06 | RAN#100 | R5-232463 | 2377 | - | F | Correction to RedCap RRM TC 17.5.1.2 SSB InSync noDRX | 17.7.0 |
| 2023-06 | RAN#100 | R5-232467 | 2381 | - | F | Correction to RedCap RRM TC 17.6.1.1 intraFreq noDRX | 17.7.0 |
| 2023-06 | RAN#100 | R5-232469 | 2383 | - | F | Correction to RedCap RRM TC 17.6.1.3 gap intraFreq noDRX with TT | 17.7.0 |
| 2023-06 | RAN#100 | R5-232470 | 2384 | - | F | Correction to RedCap RRM TC 17.6.1.4 gap intraFreq DRX with TT | 17.7.0 |
| 2023-06 | RAN#100 | R5-232472 | 2386 | - | F | Correction to RedCap RRM TC 17.6.3.2 SSB L1RSRP DRX with TT | 17.7.0 |
| 2023-06 | RAN#100 | R5-232473 | 2387 | - | F | Correction to RedCap RRM TC 17.6.3.3 CSIRS L1RSRP noDRX with TT | 17.7.0 |
| 2023-06 | RAN#100 | R5-232474 | 2388 | - | F | Correction to RedCap RRM TC 17.6.3.4 CSIRS L1RSRP DRX with TT | 17.7.0 |
| 2023-06 | RAN#100 | R5-232475 | 2389 | - | F | Correction to RedCap RRM TC 18.2.2.1 L2N Redirection | 17.7.0 |
| 2023-06 | RAN#100 | R5-232480 | 2394 | - | F | Correction to Annex A for RedCap RRM TCs | 17.7.0 |
| 2023-06 | RAN#100 | R5-232482 | 2396 | - | F | Correction to Annex F for RedCap RRM TCs | 17.7.0 |
| 2023-06 | RAN#100 | R5-232489 | 2397 | - | F | Correction to RRM enh TC 6.5.8.1 CBW change | 17.7.0 |
| 2023-06 | RAN#100 | R5-232491 | 2399 | - | F | Correction to EN DC RRM TC 4.6.2.x FR1 interFreq | 17.7.0 |
| 2023-06 | RAN#100 | R5-232492 | 2400 | - | F | Correction to NR SA RRM TC 6.6.2.x FR1 interFreq | 17.7.0 |
| 2023-06 | RAN#100 | R5-232494 | 2402 | - | F | Correction to Annex H for NR RRM TCs | 17.7.0 |
| 2023-06 | RAN#100 | R5-232530 | 2403 | - | F | Update of eMG TC 6.6.18.3 | 17.7.0 |
| 2023-06 | RAN#100 | R5-232531 | 2404 | - | F | Update of eMG TC 6.6.18.4 | 17.7.0 |
| 2023-06 | RAN#100 | R5-232532 | 2405 | - | F | Update of E.4 for MG enhancements | 17.7.0 |
| 2023-06 | RAN#100 | R5-232533 | 2406 | - | F | Update of H.3.1 for MG enhancements | 17.7.0 |
| 2023-06 | RAN#100 | R5-232916 | 2426 | - | F | Correction of EN-DC event reporting with highSpeedMeasCA-SCell-r17 test case 4.6.1.8 including Test Tolerance | 17.7.0 |
| 2023-06 | RAN#100 | R5-232917 | 2427 | - | F | Correction of EN-DC event reporting with highSpeedMeasCA-SCell-r17 test case 4.6.2.9 including Test Tolerance | 17.7.0 |
| 2023-06 | RAN#100 | R5-232918 | 2428 | - | F | Correction of SA FR1 HST reselection test case 6.1.1.8 including Test Tolerance | 17.7.0 |
| 2023-06 | RAN#100 | R5-232920 | 2430 | - | F | Correction of SA event reporting with highSpeedMeasCA-SCell-r17 test case 6.6.2.12 including Test Tolerance | 17.7.0 |
| 2023-06 | RAN#100 | R5-232921 | 2431 | - | F | Correction of Annex F for HST test cases including Test Tolerance | 17.7.0 |
| 2023-06 | RAN#100 | R5-232923 | 2433 | - | F | Correction of RedCap test case 17.1.1.1 | 17.7.0 |
| 2023-06 | RAN#100 | R5-232924 | 2434 | - | F | Correction of RedCap test case 17.1.1.2 | 17.7.0 |
| 2023-06 | RAN#100 | R5-232925 | 2435 | - | F | Correction of RedCap test case 17.1.1.3 | 17.7.0 |
| 2023-06 | RAN#100 | R5-232926 | 2436 | - | F | Correction of RedCap test case 17.1.1.4 | 17.7.0 |
| 2023-06 | RAN#100 | R5-232982 | 2439 | - | F | Correction to CSI RS based L1-RSRP measurement test 5.6.3.4 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233011 | 2440 | - | F | Correction to message exception in FR1 BWP Switching test cases | 17.7.0 |
| 2023-06 | RAN#100 | R5-233012 | 2441 | - | F | Correction to ssb-ToMeasure setting in 5.6.1.1 and 5.6.1.3 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233013 | 2442 | - | F | Correction to CellGroupConfig in 5.5.6.2.1 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233015 | 2444 | - | F | Correction to entries of FR2 RLM config in Annex H | 17.7.0 |
| 2023-06 | RAN#100 | R5-233019 | 2447 | - | F | Corrections to RRM HST 6.1.1.7 test case | 17.7.0 |
| 2023-06 | RAN#100 | R5-233020 | 2448 | - | F | Introduction of 6.5.1.9 power saving enhancement test case | 17.7.0 |
| 2023-06 | RAN#100 | R5-233134 | 2449 | - | F | Correction in RRM TC 4.5.2.5 TC Procedure | 17.7.0 |
| 2023-06 | RAN#100 | R5-233136 | 2450 | - | F | Removal of duplicated table in A.1.4.2-3 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233137 | 2451 | - | F | Core spec alignment for TC 5.5.1.7 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233140 | 2454 | - | F | PDCCH Config correction for FR2 beam failure detection and link recovery TCs | 17.7.0 |
| 2023-06 | RAN#100 | R5-233298 | 2461 | - | F | Addition of NR SA FR2 BFD and LR TT for RedCap | 17.7.0 |
| 2023-06 | RAN#100 | R5-233507 | 2367 | 1 | F | Addition of MAC-CE based active TCI state switch test case for HST FR2 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233604 | 2359 | 1 | F | Completion of SA FR2 direct SCell activation at handover test case | 17.7.0 |
| 2023-06 | RAN#100 | R5-233605 | 2361 | 1 | F | Completion of SA FR2 direct SCell activation test case | 17.7.0 |
| 2023-06 | RAN#100 | R5-233606 | 2362 | 1 | F | Completion of ENDC FR2 direct SCell activation test case | 17.7.0 |
| 2023-06 | RAN#100 | R5-233607 | 2366 | 1 | F | Addition of UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2 test case | 17.7.0 |
| 2023-06 | RAN#100 | R5-233608 | 2339 | 1 | F | Addition of Pre-MG RRM test case 6.6.17.2 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233609 | 2338 | 1 | F | Addition of Pre-MG RRM test case 6.6.17.1 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233610 | 2352 | 1 | F | Core spec alignment for SMTC value for RedCap TC 16.1.1.1 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233611 | 2395 | 1 | F | Correction to Annex E for RedCap RRM TCs | 17.7.0 |
| 2023-06 | RAN#100 | R5-233612 | 2415 | 1 | F | Addition of RedCap RLM OOS test cases 16.5.1.5 and 16.5.1.6 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233613 | 2416 | 1 | F | Addition of RedCap SSB-based BFD and LR in DRX mode test cases 16.5.2.3 and 16.5.2.4 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233614 | 2417 | 1 | F | Addition of RedCap DCI-based DL active BWP switch test cases 16.5.3.1.1 and 16.5.3.1.2 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233615 | 2418 | 1 | F | Addition of RedCap RRC-based DL active BWP switch test cases 16.5.3.2.1 and 16.5.3.2.2 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233616 | 2432 | 1 | F | Correction of RedCap NR SA FR1 - E-UTRA Cell reselection test cases in clause 16.1.2 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233617 | 2437 | 1 | F | Annex E correction for Redcap FR2 reselection cases | 17.7.0 |
| 2023-06 | RAN#100 | R5-233618 | 2348 | 1 | F | Adding test case 7.5.3.3 for SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX | 17.7.0 |
| 2023-06 | RAN#100 | R5-233619 | 2419 | 1 | F | Addition of power savings RLM OOS test case 4.5.1.9 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233620 | 2420 | 1 | F | Addition of power savings RLM OOS test case 5.5.1.10 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233621 | 2411 | 1 | F | Addition of NR-U EN-DC SS-RSRP measurement performance test cases | 17.7.0 |
| 2023-06 | RAN#100 | R5-233622 | 2412 | 1 | F | Update to NR-U frequency bands | 17.7.0 |
| 2023-06 | RAN#100 | R5-233623 | 2413 | 1 | F | Addition of NR-U SA SS-RSRP measurement performance test cases | 17.7.0 |
| 2023-06 | RAN#100 | R5-233624 | 2355 | 1 | F | Clarification to test procedure for EN-DC active BWP switch TCs: 4.5.6.1.1, 4.5.6.1.2, 4.5.6.2.1 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233625 | 2423 | 1 | F | Update to SCell activation and deactivation test cases | 17.7.0 |
| 2023-06 | RAN#100 | R5-233626 | 2455 | 1 | F | T3 correction of SSB_RP for TC 5.5.5.1 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233627 | 2443 | 1 | F | Editorial correction to reference of AoA setup | 17.7.0 |
| 2023-06 | RAN#100 | R5-233628 | 2459 | 1 | F | Update of RRM Test Case 5.5.3.1 EN-DC FR2 SCell activation and deactivation intra-band in non-DRX | 17.7.0 |
| 2023-06 | RAN#100 | R5-233629 | 2356 | 1 | F | Clarification to test procedure for SA active BWP switch TCs: 6.5.6.1.1, 6.5.6.1.2, 6.5.6.2.1 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233630 | 2424 | 1 | F | Correction to test applicability for SA FR2 test cases | 17.7.0 |
| 2023-06 | RAN#100 | R5-233645 | 2410 | 1 | F | Addition of NR SA FR2 active TCI state switch test cases 7.5.8.1.1 and 7.5.8.2.1 with TT | 17.7.0 |
| 2023-06 | RAN#100 | R5-233647 | 2385 | 1 | F | Correction to RedCap RRM TC 17.6.3.1 SSB L1RSRP noDRX with TT | 17.7.0 |
| 2023-06 | RAN#100 | R5-233648 | 2408 | 1 | F | Update to CLI tests 4.6.5.1 and 6.6.6.1 with TTs | 17.7.0 |
| 2023-06 | RAN#100 | R5-233649 | 2409 | 1 | F | Update to CLI tests 4.7.6.1 and 6.7.8.1 with TTs | 17.7.0 |
| 2023-06 | RAN#100 | R5-233650 | 2422 | 1 | F | Annex E and F updates for CLI-based test cases including TTs | 17.7.0 |
| 2023-06 | RAN#100 | R5-233654 | 2378 | 1 | F | Correction to RedCap RRM TC 17.5.2.3 CSIRS BFR noDRX with TT | 17.7.0 |
| 2023-06 | RAN#100 | R5-233655 | 2379 | 1 | F | Correction to RedCap RRM TC 17.5.2.4 CSIRS BFR DRX with TT | 17.7.0 |
| 2023-06 | RAN#100 | R5-233656 | 2380 | 1 | F | Correction to RedCap RRM TC 17.5.2.5 BFR restriction with TT | 17.7.0 |
| 2023-06 | RAN#100 | R5-233657 | 2382 | 1 | F | Addition of RedCap RRM TC 17.6.1.2 intraFreq DRX with TT | 17.7.0 |
| 2023-06 | RAN#100 | R5-233658 | 2429 | 1 | F | Correction of SA event reporting with highSpeedMeasCA-SCell-r17 test case 6.6.1.8 including Test Tolerance | 17.7.0 |
| 2023-06 | RAN#100 | R5-233660 | 2363 | 1 | F | Update to PRS based UE Rx-Tx measurement FR1 SA test case | 17.7.0 |
| 2023-06 | RAN#100 | R5-233661 | 2364 | 1 | F | Update to PRS based UE Rx-Tx measurement FR2 SA test case | 17.7.0 |
| 2023-06 | RAN#100 | R5-233662 | 2365 | 1 | F | Update to TRS based UE Rx-Tx measurement FR1 SA test case | 17.7.0 |
| 2023-06 | RAN#100 | R5-233663 | 2347 | 1 | F | UE UL carrier RRC reconfiguration delay test tolerances for FR2 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233665 | 2368 | 1 | F | Correction to FR2 BFD and LR including TT | 17.7.0 |
| 2023-06 | RAN#100 | R5-233666 | 2460 | 1 | F | Correction to FR2 BFD and LR | 17.7.0 |
| 2023-06 | RAN#100 | R5-233667 | 2421 | 1 | F | TT update for test cases 6.6.3.1 and 6.6.3.2 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233729 | 2452 | 1 | F | Core spec alignment for antenna configuration for 4.5.3 TCs | 17.7.0 |
| 2023-06 | RAN#100 | R5-233730 | 2453 | 1 | F | Core spec alignment for antenna configuration for 6.5.3 TCs | 17.7.0 |
| 2023-06 | RAN#100 | R5-233739 | 2456 | 1 | F | Update to FR1 CG-SDT test case | 17.7.0 |
| 2023-06 | RAN#100 | R5-233740 | 2414 | 1 | F | Update to RRM idle mode HST test cases | 17.7.0 |
| 2023-06 | RAN#100 | R5-233741 | 2457 | 1 | F | Correction for multi-TRP test case 7.5.5.9 | 17.7.0 |
| 2023-06 | RAN#100 | R5-233742 | 2458 | 1 | F | Update to L2N latency test cases | 17.7.0 |
| 2023-06 | RAN#100 | R5-233747 | 2357 | 1 | F | Completion of EN-DC FR1 direct SCell activation test case | 17.7.0 |
| 2023-06 | RAN#100 | R5-233748 | 2358 | 1 | F | Completion of SA FR1 direct SCell activation test case | 17.7.0 |
| 2023-06 | RAN#100 | R5-233749 | 2360 | 1 | F | Completion SA FR1 direct SCell activation at handover test case | 17.7.0 |
| 2023-06 | RAN#100 | R5-233752 | 2390 | 1 | F | Correction to RedCap RRM TC 18.3.1.5 interRAT noDRX with TT | 17.7.0 |
| 2023-06 | RAN#100 | R5-233753 | 2391 | 1 | F | Correction to RedCap RRM TC 18.3.1.6 interRAT DRX with TT | 17.7.0 |
| 2023-06 | RAN#100 | R5-233754 | 2392 | 1 | F | Correction to RedCap RRM TC 18.3.1.7 interRAT noDRX SBI with TT | 17.7.0 |
| 2023-06 | RAN#100 | R5-233755 | 2393 | 1 | F | Correction to RedCap RRM TC 18.3.1.8 interRAT DRX SBI with TT | 17.7.0 |
| 2023-06 | RAN#100 | R5-233771 | 2373 | 1 | F | Correction to RedCap RRM TC 16.3.1.x NCDSSB HO | 17.7.0 |
| 2023-06 | RAN#100 | R5-233773 | 2398 | 1 | F | Correction to Annex A for RRM enh TCs | 17.7.0 |
| 2023-06 | RAN#100 | R5-233775 | 2401 | 2 | F | Correction to NR SA RRM TC 8.4.2.x FR2 interRAT | 17.7.0 |
| 2023-09 | RAN#101 | R5-233889 | 2467 | - | F | Update CORESET RMC in Annex for Redcap TC 16.5.3.2.1 and 16.5.3.2.2 with core spec alignment | 17.8.0 |
| 2023-09 | RAN#101 | R5-233891 | 2468 | - | F | Correction of 6.6.19.1 NCSG test case with Test Tolerance | 17.8.0 |
| 2023-09 | RAN#101 | R5-233892 | 2469 | - | F | Correction of 6.6.19.2 NCSG test case with Test Tolerance | 17.8.0 |
| 2023-09 | RAN#101 | R5-233894 | 2471 | - | F | Correction of 6.6.19.4 NCSG test case with Test Tolerance | 17.8.0 |
| 2023-09 | RAN#101 | R5-233967 | 2472 | - | F | Adding TTs into 7.5.3.3 - SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX | 17.8.0 |
| 2023-09 | RAN#101 | R5-233999 | 2476 | - | F | Correction of Redcap test case 16.6.1.3, 16.6.1.4, 16.6.1.7 and 16.6.1.8 | 17.8.0 |
| 2023-09 | RAN#101 | R5-234000 | 2477 | - | F | Correction of Redcap test case 16.1.2.1, 16.1.2.2, 16.1.2.3 and 16.1.2.4 | 17.8.0 |
| 2023-09 | RAN#101 | R5-234086 | 2481 | - | F | Correction of test step number and reference for RRM Test Case | 17.8.0 |
| 2023-09 | RAN#101 | R5-234331 | 2482 | - | F | Correction in minimum conformance requirements for intra-frequency cell re-selection to handle HST FR2 | 17.8.0 |
| 2023-09 | RAN#101 | R5-234332 | 2483 | - | F | Addition of intra-frequency cell re-selection NR FR2 test case for UE power class 6 | 17.8.0 |
| 2023-09 | RAN#101 | R5-234337 | 2486 | - | F | Completion of propagation delay compensation FR1 SA test cases | 17.8.0 |
| 2023-09 | RAN#101 | R5-234338 | 2487 | - | F | Completion of propagation delay compensation FR2 SA test cases | 17.8.0 |
| 2023-09 | RAN#101 | R5-234356 | 2497 | - | F | Addition of 16.6.7.1 test case: NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 1 Rx UE | 17.8.0 |
| 2023-09 | RAN#101 | R5-234357 | 2498 | - | F | Addition of 16.6.7.2 test case: NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 2 Rx UE | 17.8.0 |
| 2023-09 | RAN#101 | R5-234367 | 2507 | - | F | Cell configuration mapping for the MRDC and CPAC test case | 17.8.0 |
| 2023-09 | RAN#101 | R5-234385 | 2508 | - | F | Correction to FR2 RLM tests for PSCell configured with SSB-based RLM-RS including TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234386 | 2509 | - | F | Correction to TC 17.5.1.2 RedCap NR SA FR2 RLM including TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234388 | 2510 | - | F | Correction to 5.5.1.6 EN-DC FR2 RLM test for PSCell configured with CSI-RS-based RLM-RS including TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234397 | 2514 | - | F | Correction to RedCap RRM TC 16.3.1.1_intra freq HO known 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234398 | 2515 | - | F | Correction to RedCap RRM TC 16.3.1.3_intra freq HO unknown 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234399 | 2516 | - | F | Correction to RedCap RRM TC 16.3.1.4_intra freq HO unknown 2Rx | 17.8.0 |
| 2023-09 | RAN#101 | R5-234400 | 2517 | - | F | Correction to RedCap RRM TC 16.3.1.5_inter freq HO unknown 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234401 | 2518 | - | F | Correction to RedCap RRM TC 16.5.1.1_SSB OOS no DRX 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234402 | 2519 | - | F | Correction to RedCap RRM TC 16.5.1.3_SSB IS no DRX 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234404 | 2521 | - | F | Correction to RedCap RRM TC 16.5.1.7_SSB IS DRX 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234405 | 2522 | - | F | Correction to RedCap RRM TC 16.5.1.9_CSIRS OOS no DRX 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234406 | 2523 | - | F | Correction to RedCap RRM TC 16.5.1.11_CSIRS IS no DRX 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234407 | 2524 | - | F | Correction to RedCap RRM TC 16.5.1.13_CSIRS OOS DRX 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234408 | 2525 | - | F | Correction to RedCap RRM TC 16.5.1.15_CSIRS IS DRX 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234409 | 2526 | - | F | Correction to RedCap RRM TC 16.6.1.1_no gap no DRX 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234410 | 2527 | - | F | Correction to RedCap RRM TC 16.6.1.3_no gap DRX 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234411 | 2528 | - | F | Correction to RedCap RRM TC 16.6.1.5_gap no DRX 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234412 | 2529 | - | F | Correction to RedCap RRM TC 16.6.1.7_gap DRX 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234413 | 2530 | - | F | Correction to RedCap RRM TC 16.6.1.9_no gap no DRX SBI 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234414 | 2531 | - | F | Correction to RedCap RRM TC 16.6.1.11_no gap DRX SBI 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234415 | 2532 | - | F | Correction to RedCap RRM TC 16.6.4.1_SSB L1 RSRP no DRX 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234416 | 2533 | - | F | Correction to RedCap RRM TC 16.6.4.3_SSB L1 RSRP DRX 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234417 | 2534 | - | F | Correction to RedCap RRM TC 16.6.4.5_CSIRS L1 RSRP no DRX 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234418 | 2535 | - | F | Correction to RedCap RRM TC 16.6.4.7_CSIRS L1 RSRP DRX 1Rx with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234428 | 2537 | - | F | Addition of RRM enh TC 4.5.6.3.1 - EN DC dual DCI BWP switch | 17.8.0 |
| 2023-09 | RAN#101 | R5-234429 | 2538 | - | F | Addition of RRM enh TC 4.5.6.5.1 - EN DC dual RRC BWP switch | 17.8.0 |
| 2023-09 | RAN#101 | R5-234430 | 2539 | - | F | Addition of RRM enh TC 6.5.6.3.1 - NR SA dual DCI BWP switch | 17.8.0 |
| 2023-09 | RAN#101 | R5-234431 | 2540 | - | F | Addition of RRM enh TC 6.5.6.5.1 - NR SA dual RRC BWP switch | 17.8.0 |
| 2023-09 | RAN#101 | R5-234432 | 2541 | - | F | Correction to Annex E for RRM enh TCs | 17.8.0 |
| 2023-09 | RAN#101 | R5-234436 | 2544 | - | F | Correction to NR SA RRM TC 6.7.1.2.1_RSRP absolute accuracy | 17.8.0 |
| 2023-09 | RAN#101 | R5-234437 | 2545 | - | F | Correction to FR2 NR inter-RAT measurement TCs 8.4.2.X | 17.8.0 |
| 2023-09 | RAN#101 | R5-234440 | 2547 | - | F | Correction to power saving RRM TCs 6.1.1.x_relaxed measurements | 17.8.0 |
| 2023-09 | RAN#101 | R5-234646 | 2548 | - | F | Addition of NTN intra-frequency measurement test cases | 17.8.0 |
| 2023-09 | RAN#101 | R5-234647 | 2549 | - | F | Addition of NTN general sections | 17.8.0 |
| 2023-09 | RAN#101 | R5-234650 | 2550 | - | F | Update for test case 4.5.1.9 including TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234652 | 2551 | - | F | Update for test case 5.5.1.10 including TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-234653 | 2552 | - | F | Addition of NR-U general sections | 17.8.0 |
| 2023-09 | RAN#101 | R5-234654 | 2553 | - | F | Addition of NR-U interruption test case | 17.8.0 |
| 2023-09 | RAN#101 | R5-234655 | 2554 | - | F | Addition of NR-U SCell activation and deactivation test cases | 17.8.0 |
| 2023-09 | RAN#101 | R5-234656 | 2555 | - | F | Addition of NR-U SS-RSRQ measurement accuracy test cases | 17.8.0 |
| 2023-09 | RAN#101 | R5-234657 | 2556 | - | F | Addition of NR-U SS-SINR measurement accuracy test cases | 17.8.0 |
| 2023-09 | RAN#101 | R5-234658 | 2557 | - | F | Update to NR-U test cases | 17.8.0 |
| 2023-09 | RAN#101 | R5-234659 | 2558 | - | F | Update to NR-U annexes E and H | 17.8.0 |
| 2023-09 | RAN#101 | R5-234662 | 2559 | - | F | Annex E and F updates for Redcap RLM, BFR and BWP switch test cases | 17.8.0 |
| 2023-09 | RAN#101 | R5-234664 | 2560 | - | F | Update to Redcap RLM, BFR and BWP switch tests including TTs | 17.8.0 |
| 2023-09 | RAN#101 | R5-234671 | 2561 | - | F | Update to TT analysis for L2N RedCap latency test cases 18.3.1.x | 17.8.0 |
| 2023-09 | RAN#101 | R5-234677 | 2564 | - | F | Addition of TT analysis for TCI state switch test cases 7.5.8.1.1 and 7.5.8.2.1 | 17.8.0 |
| 2023-09 | RAN#101 | R5-234681 | 2566 | - | F | Update to TT analysis for L2N latency test cases 8.4.2.x | 17.8.0 |
| 2023-09 | RAN#101 | R5-234682 | 2567 | - | F | Addition of CSI-RS based RLM SA FR2 Test Cases | 17.8.0 |
| 2023-09 | RAN#101 | R5-234776 | 2571 | - | F | Test tolerances and measurement uncertainty for DL interruptions testcases 6.5.7A.2 and 6.5.7B.2 | 17.8.0 |
| 2023-09 | RAN#101 | R5-234807 | 2572 | - | F | Addition of minimum requirements for Fast SCell Activation | 17.8.0 |
| 2023-09 | RAN#101 | R5-234808 | 2573 | - | F | Addition of minimum requirements for Conditional PSCell addition | 17.8.0 |
| 2023-09 | RAN#101 | R5-234810 | 2575 | - | F | Addition of new test case 7.5.3.14 for Fast SCell Activation | 17.8.0 |
| 2023-09 | RAN#101 | R5-234811 | 2576 | - | F | Addition of new test case 7.5.12.1 for Conditional PSCell addition | 17.8.0 |
| 2023-09 | RAN#101 | R5-234812 | 2577 | - | F | Addition of new test case 7.5.15.1 for SCG activation and deactivation | 17.8.0 |
| 2023-09 | RAN#101 | R5-234813 | 2578 | - | F | Addition of CSI-RS configuration for Fast SCell activation | 17.8.0 |
| 2023-09 | RAN#101 | R5-234855 | 2580 | - | F | Addition of 5.5.11.1 new test case for unified joint TCI activation | 17.8.0 |
| 2023-09 | RAN#101 | R5-234856 | 2581 | - | F | Addition of 5.5.11.2 new test case for unified UL TCI activation | 17.8.0 |
| 2023-09 | RAN#101 | R5-234857 | 2582 | - | F | Addition of 5.5.11.3 new test case for unified DL TCI activation | 17.8.0 |
| 2023-09 | RAN#101 | R5-234858 | 2583 | - | F | Addition of 7.5.13.1 new test case for unified joint TCI activation | 17.8.0 |
| 2023-09 | RAN#101 | R5-234859 | 2584 | - | F | Addition of 7.5.13.2 new test case for unified UL TCI activation | 17.8.0 |
| 2023-09 | RAN#101 | R5-234860 | 2585 | - | F | Addition of 7.5.13.3 new test case for unified DL TCI activation | 17.8.0 |
| 2023-09 | RAN#101 | R5-234919 | 2591 | - | F | Correction to Cell 2 PCI for next iteration in 16.1.1.1 | 17.8.0 |
| 2023-09 | RAN#101 | R5-234921 | 2593 | - | F | Correction to test requirements in RedCap Cell reselection test cases | 17.8.0 |
| 2023-09 | RAN#101 | R5-234924 | 2596 | - | F | Correction to RedCap Event triggered measurement test cases | 17.8.0 |
| 2023-09 | RAN#101 | R5-234977 | 2608 | - | F | Addition of cell configuration mapping for interruptions at SRS carrier based switching test cases to Annex E | 17.8.0 |
| 2023-09 | RAN#101 | R5-234992 | 2623 | - | F | Addition of NR Event triggered reporting test case 16.6.2.9 for RedCap UE | 17.8.0 |
| 2023-09 | RAN#101 | R5-234995 | 2626 | - | F | Addition of NR Event triggered reporting test case 16.6.2.12 for RedCap UE | 17.8.0 |
| 2023-09 | RAN#101 | R5-234997 | 2628 | - | F | Addition of cell configuration mapping for event triggered measurement RedCap test cases to Annex E | 17.8.0 |
| 2023-09 | RAN#101 | R5-235107 | 2646 | - | F | TC 4.6.1.2 alignment with SA FR1 test case on DRX configuration | 17.8.0 |
| 2023-09 | RAN#101 | R5-235108 | 2647 | - | F | Correction in 5.5.1.5 and 5.5.1.6 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235110 | 2648 | - | F | Correction in 5.5.1.7 and 5.5.1.8 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235117 | 2652 | - | F | Correction in 8.4.2.8 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235166 | 2654 | - | F | Addition of clause 5.5.9.0 minimum conformance requirements for uplink spatial relation switch delay | 17.8.0 |
| 2023-09 | RAN#101 | R5-235167 | 2655 | - | F | Addition of EN-DC PSCell FR2 uplink spatial relation switch for a known spatial relation test case 5.5.9.1.1 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235168 | 2656 | - | F | Addition of EN-DC PSCell FR2 uplink spatial relation switch associated with a known DL-RS test case 5.5.9.2.1 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235170 | 2658 | - | F | Addition of NR PCell FR2 uplink spatial relation switch with known DL-RS test case 7.5.9.1.1 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235171 | 2659 | - | F | Addition of NR PCell FR2 uplink spatial relation switch with known DL-RS test case 7.5.9.2.1 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235172 | 2660 | - | F | Addition of test cases 5.5.9.1.1 and 5.5.9.2.1 to Table E.3-1, and test cases 7.5.9.1.1 and 7.5.9.2.1 to Table E.5-1 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235463 | 2602 | 1 | F | Correction of interruptions at NR SRS carrier based switching test case 4.5.2.8 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235464 | 2603 | 1 | F | Correction of interruptions at E-UTRA SRS carrier based switching test case 4.5.2.9 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235465 | 2568 | 2 | F | Update to gap-based measurement reporting tests | 17.8.0 |
| 2023-09 | RAN#101 | R5-235466 | 2600 | 1 | F | Correction to BWP Switch Delay test cases | 17.8.0 |
| 2023-09 | RAN#101 | R5-235695 | 2466 | 1 | F | Correction of RedCap test case 16.6.1.7 and 16.6.1.11 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235696 | 2562 | 1 | F | Update to stationary measurement idle mode RedCap tests | 17.8.0 |
| 2023-09 | RAN#101 | R5-235697 | 2627 | 1 | F | Correction of the conditions for 1Rx RedCap UE | 17.8.0 |
| 2023-09 | RAN#101 | R5-235698 | 2631 | 1 | F | Removal of redundant Note 4 in 16.6.4.2 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235699 | 2633 | 1 | F | Alignment of Annex E14 to Test procedure for RedCap Redirection TCs | 17.8.0 |
| 2023-09 | RAN#101 | R5-235700 | 2638 | 1 | F | Corrections to RedCap RA TCs 16.3.2.2.2 and 16.3.2.2.4 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235701 | 2639 | 1 | F | Typos corrections for RedCap TCs | 17.8.0 |
| 2023-09 | RAN#101 | R5-235702 | 2645 | 1 | F | Core Spec alignment for RedCap RLM Tcs | 17.8.0 |
| 2023-09 | RAN#101 | R5-235703 | 2499 | 1 | F | Addition of NR SA FR2 SSB based Inter-cell L1-RSRP measurement in non-DRX test case | 17.8.0 |
| 2023-09 | RAN#101 | R5-235704 | 2579 | 1 | F | Addition of minimum requirement for unified TCI state switch delay | 17.8.0 |
| 2023-09 | RAN#101 | R5-235705 | 2586 | 1 | F | Update to Annex for feMIMO | 17.8.0 |
| 2023-09 | RAN#101 | R5-235706 | 2495 | 1 | F | Addition of DL interruptions at switching between two uplink bands with two transmit antenna connectors for 6.5.7C.1 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235707 | 2484 | 1 | F | Corrections in MAC-CE based active TCI state switch test case for HST FR2 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235708 | 2485 | 1 | F | Handle of PC6 devices via HighSpeedMeasFlagFR2-r17 signalling for 7.4.1.1 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235709 | 2478 | 1 | F | Addition of new test case 4.5.2.10 EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell | 17.8.0 |
| 2023-09 | RAN#101 | R5-235710 | 2479 | 1 | F | Addition of new test case 4.5.10.1 EN-DC FR1 PSCell activation and deactivation delay | 17.8.0 |
| 2023-09 | RAN#101 | R5-235711 | 2480 | 1 | F | Addition of new test case 5.5.12.1 EN-DC FR2 PSCell activation and deactivation delay | 17.8.0 |
| 2023-09 | RAN#101 | R5-235712 | 2574 | 1 | F | Addition of new test case 5.5.13.1 for Conditional PSCell addition | 17.8.0 |
| 2023-09 | RAN#101 | R5-235713 | 2542 | 1 | F | Editorial correction to NR RRM TCs | 17.8.0 |
| 2023-09 | RAN#101 | R5-235714 | 2569 | 1 | F | Update to Random Access applicability | 17.8.0 |
| 2023-09 | RAN#101 | R5-235715 | 2635 | 1 | F | Typo correction for 4.5.2.6 Test purpose | 17.8.0 |
| 2023-09 | RAN#101 | R5-235716 | 2643 | 1 | F | Editorial changes to correct formatting | 17.8.0 |
| 2023-09 | RAN#101 | R5-235717 | 2651 | 1 | F | Correction to firstActiveDownlinkBWP-Id for non-contention based RA TCs | 17.8.0 |
| 2023-09 | RAN#101 | R5-235718 | 2632 | 1 | F | Removal of inexistent Config IDs 6.6.4.x | 17.8.0 |
| 2023-09 | RAN#101 | R5-235719 | 2642 | 1 | F | Core Spec alignment for Table 6.6.3.3.5-1 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235720 | 2492 | 1 | F | Correction in DCCA test case titles | 17.8.0 |
| 2023-09 | RAN#101 | R5-235721 | 2634 | 1 | F | Alignment of Annex E Cell configuration mapping to Test procedure for TC 6.3.2.3.1 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235722 | 2563 | 1 | F | Update to RRM tests with testability issues | 17.8.0 |
| 2023-09 | RAN#101 | R5-235723 | 2661 | 1 | F | Addition of test cases 5.5.9.1.1 and 5.5.9.2.1 to Table F.1.3.2-3 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235724 | 2536 | 1 | F | Correction to Annex F for RedCap RRM TCs | 17.8.0 |
| 2023-09 | RAN#101 | R5-235725 | 2609 | 1 | F | Correction of NR SA FR1 Cell reselection RedCap test case 16.1.1.3 including test tolerance | 17.8.0 |
| 2023-09 | RAN#101 | R5-235727 | 2611 | 1 | F | Correction of NR SA FR1 Cell reselection RedCap test case 16.1.1.7 including test tolerance | 17.8.0 |
| 2023-09 | RAN#101 | R5-235728 | 2612 | 1 | F | Correction of NR SA FR1 - E-UTRA Cell reselection RedCap test case 16.1.2.1 including test tolerance | 17.8.0 |
| 2023-09 | RAN#101 | R5-235729 | 2613 | 1 | F | Correction of NR SA FR1- E-UTRA Cell reselection RedCap test case 16.1.2.3 including test tolerance | 17.8.0 |
| 2023-09 | RAN#101 | R5-235733 | 2649 | 1 | F | Introduction of 5.5.5.9 power saving enhancement test case | 17.8.0 |
| 2023-09 | RAN#101 | R5-235734 | 2650 | 1 | F | Update on 6.5.1.9 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235735 | 2662 | 1 | F | Addition of test cases 7.5.9.1.1 and 7.5.9.2.1 to Table F.1.3.2-4 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235738 | 2463 | 1 | F | Correction to 6.6.17.2 for editor's note, confirm TT=0, RRC msg content | 17.8.0 |
| 2023-09 | RAN#101 | R5-235739 | 2465 | 1 | F | Update Annex F for gap enhancement test cases 6.6.18.1 and 6.6.18.2 including TTs | 17.8.0 |
| 2023-09 | RAN#101 | R5-235741 | 2543 | 1 | F | Correction to FR1 L1 RSRP relative accuracy TCs with TT | 17.8.0 |
| 2023-09 | RAN#101 | R5-235742 | 2546 | 1 | F | Correction to Annex F for FR1 L1 RSRP relative accuracy TCs | 17.8.0 |
| 2023-09 | RAN#101 | R5-235744 | 2462 | 1 | F | Correction to 6.6.17.1 for editor's note, confirm TT=0, update Annex F for pre-MG TCs | 17.8.0 |
| 2023-09 | RAN#101 | R5-235745 | 2464 | 1 | F | Update concurrent gap test cases 6.6.18.1 and 6.6.18.2 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235840 | 2657 | 1 | F | Addition of clause 7.5.9.0 minimum conformance requirements for uplink spatial relation switch delay | 17.8.0 |
| 2023-09 | RAN#101 | R5-235881 | 2475 | 1 | F | Core spec alignment for Redcap test case 16.5.1.5, 16.5.1.6, 16.5.2.3, 16.5.3.2.1 and 16.5.3.2.2 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235882 | 2592 | 1 | F | Removal of square brackets from test parameters of RedCap test cases | 17.8.0 |
| 2023-09 | RAN#101 | R5-235883 | 2597 | 1 | F | Correction to CORESET RMC settings in 16.6.1.8 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235884 | 2614 | 1 | F | Correction of NR SA FR1- E-UTRA Cell reselection RedCap test case 16.1.2.5 including test tolerance | 17.8.0 |
| 2023-09 | RAN#101 | R5-235885 | 2615 | 1 | F | Addition of NR Event triggered reporting test case 16.6.2.1 for RedCap UE | 17.8.0 |
| 2023-09 | RAN#101 | R5-235886 | 2616 | 1 | F | Addition of NR Event triggered reporting test case 16.6.2.2 for RedCap UE | 17.8.0 |
| 2023-09 | RAN#101 | R5-235887 | 2617 | 1 | F | Addition of NR Event triggered reporting test case 16.6.2.3 for RedCap UE | 17.8.0 |
| 2023-09 | RAN#101 | R5-235888 | 2618 | 1 | F | Addition of NR Event triggered reporting test case 16.6.2.4 for RedCap UE | 17.8.0 |
| 2023-09 | RAN#101 | R5-235889 | 2621 | 1 | F | Addition of NR Event triggered reporting test case 16.6.2.7 for RedCap UE | 17.8.0 |
| 2023-09 | RAN#101 | R5-235890 | 2622 | 1 | F | Addition of NR Event triggered reporting test case 16.6.2.8 for RedCap UE | 17.8.0 |
| 2023-09 | RAN#101 | R5-235891 | 2624 | 1 | F | Addition of NR Event triggered reporting test case 16.6.2.10 for RedCap UE | 17.8.0 |
| 2023-09 | RAN#101 | R5-235892 | 2625 | 1 | F | Addition of NR Event triggered reporting test case 16.6.2.11 for RedCap UE | 17.8.0 |
| 2023-09 | RAN#101 | R5-235894 | 2653 | 1 | F | Update in Redcap Radio Link Monitoring In-sync Test in DRX mode for 2 Rx | 17.8.0 |
| 2023-09 | RAN#101 | R5-235895 | 2473 | 1 | F | Modification to CG-SDT RRM test case for FR2 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235896 | 2587 | 1 | F | TT update for test cases 4.5.5.7& 4.5.5.8 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235897 | 2588 | 1 | F | TT update for test cases 5.5.5.8 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235898 | 2589 | 1 | F | TT update for test cases 6.5.5.7 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235899 | 2590 | 1 | F | TT update for test cases 7.5.5.9& 7.5.5.10 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235900 | 2604 | 1 | F | Correction of interruptions at E-UTRA SRS carrier based switching test case 5.5.2.7 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235901 | 2605 | 1 | F | Correction of interruptions at NR SRS carrier based switching test case 5.5.2.5 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235902 | 2606 | 1 | F | Correction of interruptions at NR SRS carrier based switching test case 6.5.2.2 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235903 | 2607 | 1 | F | Addition of SRS configuration for carrier based switching test cases to Annex A | 17.8.0 |
| 2023-09 | RAN#101 | R5-235904 | 2493 | 1 | F | Addition of NR SA FR1 DL interruptions at switching between two uplink carriers for 6.5.7A.2 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235905 | 2494 | 1 | F | Addition of NR SA FR1 DL interruptions at switching between two uplink bands for 6.5.7B.2 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235906 | 2496 | 1 | F | Addition of DL interruptions at switching between two uplink bands with two transmit antenna connectors for 6.5.7C.2 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235907 | 2470 | 1 | F | Correction of 6.6.19.3 NCSG test case with Test Tolerance | 17.8.0 |
| 2023-09 | RAN#101 | R5-235908 | 2500 | 1 | F | Addition of EN-DC FR1 test case for the addition and release delay of PSCell | 17.8.0 |
| 2023-09 | RAN#101 | R5-235909 | 2501 | 1 | F | Addition of EN-DC FR1 fast SCell activation test case for 160 ms SCell measurement cycle | 17.8.0 |
| 2023-09 | RAN#101 | R5-235910 | 2502 | 1 | F | Addition of EN-DC FR1 fast SCell activation test case for 640 ms SCell measurement cycle | 17.8.0 |
| 2023-09 | RAN#101 | R5-235911 | 2503 | 1 | F | Addition of EN-DC FR2 fast SCell in intra-band activation test case | 17.8.0 |
| 2023-09 | RAN#101 | R5-235912 | 2504 | 1 | F | Addition of NR SA FR1 fast SCell activation test case for 160 ms SCell measurement cycle | 17.8.0 |
| 2023-09 | RAN#101 | R5-235913 | 2505 | 1 | F | Addition of NR SA FR1 fast SCell activation test case for 640 ms SCell measurement cycle | 17.8.0 |
| 2023-09 | RAN#101 | R5-235914 | 2506 | 1 | F | Addition of NR SA FR2 fast SCell in intra-band activation test case | 17.8.0 |
| 2023-09 | RAN#101 | R5-235916 | 2488 | 1 | F | TT analysis of NR SA FR1 direct SCell activation at handover test case | 17.8.0 |
| 2023-09 | RAN#101 | R5-235917 | 2489 | 1 | F | TT analysis of NR SA FR2 direct SCell activation test case | 17.8.0 |
| 2023-09 | RAN#101 | R5-235918 | 2490 | 1 | F | TT analysis of NR SA FR2 direct SCell activation at handover test case | 17.8.0 |
| 2023-09 | RAN#101 | R5-235919 | 2491 | 1 | F | Measurement uncertainty and test tolerance updation of direct SCell activation test cases | 17.8.0 |
| 2023-09 | RAN#101 | R5-235946 | 2601 | 1 | F | Correction to csi-SSB-ResourceToAddModList for SSB-based CSI-Reporting test cases | 17.8.0 |
| 2023-09 | RAN#101 | R5-235948 | 2565 | 1 | F | Update to FR2 RLM test cases | 17.8.0 |
| 2023-09 | RAN#101 | R5-235963 | 2630 | 1 | F | Correction of Redcap test case 16.4.1.1, 16.4.1.2, 16.4.3.1, 16.4.3.2, 16.5.3.1.1 and 16.5.3.1.2 | 17.8.0 |
| 2023-09 | RAN#101 | R5-235964 | 2629 | 2 | F | Test tolerance correction in Annex F for 1Rx RedCap test cases | 17.8.0 |
| 2023-09 | RAN#101 | - | - | - | - | Administrative release upgrade to match the release of 3GPP TS 38.522 which was upgraded at RAN#101 to Rel-18 due to Rel-18 relevant CR(s) | 18.0.0 |
| 2023-12 | RAN#102 | R5-236160 | 2665 | - | F | Update of MG enhancements TC 6.6.17.x | 18.1.0 |
| 2023-12 | RAN#102 | R5-236163 | 2668 | - | F | Update of measGapId-r17 for MG enhancements TC | 18.1.0 |
| 2023-12 | RAN#102 | R5-236196 | 2669 | - | F | Editorial correction for 6.7.3.2.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236198 | 2670 | - | F | Addition of SMTC.7 and SMTC.8 into reference SMTC configuration in Annex | 18.1.0 |
| 2023-12 | RAN#102 | R5-236199 | 2671 | - | F | Addition of minimum requirements for FR2 7.6.15.0 - concurrent gaps | 18.1.0 |
| 2023-12 | RAN#102 | R5-236200 | 2672 | - | F | Addition of new test case 7.6.15.2 for overlapping concurrent gaps | 18.1.0 |
| 2023-12 | RAN#102 | R5-236201 | 2673 | - | F | Addition of new test case 7.6.15.3 for concurrent gaps with PRS measurement | 18.1.0 |
| 2023-12 | RAN#102 | R5-236202 | 2674 | - | F | Update cell mapping table for FR2 concurrent gaps new test cases | 18.1.0 |
| 2023-12 | RAN#102 | R5-236209 | 2676 | - | F | Addition of NR SA FR2 SSB-based L1-RSRP measurement test case for PC6 devices | 18.1.0 |
| 2023-12 | RAN#102 | R5-236212 | 2677 | - | F | Addition of NR-U redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA test case | 18.1.0 |
| 2023-12 | RAN#102 | R5-236213 | 2678 | - | F | Addition of NR-U redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA test case | 18.1.0 |
| 2023-12 | RAN#102 | R5-236216 | 2679 | - | F | Completion of NR SA FR1 DL interruptions test case 6.5.7C.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236217 | 2680 | - | F | Completion of NR SA FR1 DL interruptions test case 6.5.7C.2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236227 | 2684 | - | F | Completion of EN-DC FR1 fast SCell activation test case for 160 ms SCell measurement cycle | 18.1.0 |
| 2023-12 | RAN#102 | R5-236228 | 2685 | - | F | Completion of EN-DC FR1 fast SCell activation test case for 640 ms SCell measurement cycle | 18.1.0 |
| 2023-12 | RAN#102 | R5-236230 | 2687 | - | F | Completion of EN-DC FR2 fast SCell in intra-band activation test case | 18.1.0 |
| 2023-12 | RAN#102 | R5-236231 | 2688 | - | F | Completion of NR SA FR1 fast SCell activation test case for 160 ms SCell measurement cycle | 18.1.0 |
| 2023-12 | RAN#102 | R5-236232 | 2689 | - | F | Completion of NR SA FR1 fast SCell activation test case for 640 ms SCell measurement cycle | 18.1.0 |
| 2023-12 | RAN#102 | R5-236233 | 2690 | - | F | Completion of NR SA FR2 fast SCell in intra-band activation test case | 18.1.0 |
| 2023-12 | RAN#102 | R5-236362 | 2694 | - | F | Correction of TRP specific BFR test cases | 18.1.0 |
| 2023-12 | RAN#102 | R5-236383 | 2695 | - | F | Addition of new test case 7.6.16.3 for NCSG | 18.1.0 |
| 2023-12 | RAN#102 | R5-236384 | 2696 | - | F | Addition of new test case 7.6.16.2 for NCSG | 18.1.0 |
| 2023-12 | RAN#102 | R5-236385 | 2697 | - | F | Addition of minimum requirements for 7.6.16.0 - FR2 NCSG | 18.1.0 |
| 2023-12 | RAN#102 | R5-236386 | 2698 | - | F | Update cell mapping table for FR2 NCSG new test cases | 18.1.0 |
| 2023-12 | RAN#102 | R5-236454 | 2703 | - | F | Update cell mapping table for FR2 Pre-MG new test cases | 18.1.0 |
| 2023-12 | RAN#102 | R5-236455 | 2704 | - | F | Addition of minimum requirements for 7.6.14.0 - FR2 Pre-MG | 18.1.0 |
| 2023-12 | RAN#102 | R5-236682 | 2709 | - | F | Addition of RedCap RRM TC 16.6.5.3 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236704 | 2731 | - | F | Addition of RedCap RRM TC 16.7.6.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236705 | 2732 | - | F | Addition of RedCap RRM TC 16.7.6.2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236709 | 2736 | - | F | Addition of RedCap RRM TC 17.3.2.3.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236711 | 2738 | - | F | Addition of RedCap RRM TC 17.4.3.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236712 | 2739 | - | F | Addition of RedCap RRM TC 17.5.1.5 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236713 | 2740 | - | F | Addition of RedCap RRM TC 17.5.1.6 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236715 | 2742 | - | F | Addition of RedCap RRM TC 17.5.1.8 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236721 | 2748 | - | F | Correction to RedCap RRM TC 16.3.2.1.1 with TT | 18.1.0 |
| 2023-12 | RAN#102 | R5-236722 | 2749 | - | F | Correction to RedCap RRM TC 16.3.2.1.3 with TT | 18.1.0 |
| 2023-12 | RAN#102 | R5-236724 | 2751 | - | F | Correction to RedCap RRM TC 16.3.2.2.1 with TT | 18.1.0 |
| 2023-12 | RAN#102 | R5-236725 | 2752 | - | F | Correction to RedCap RRM TC 16.3.2.2.3 with TT | 18.1.0 |
| 2023-12 | RAN#102 | R5-236726 | 2753 | - | F | Correction to RedCap RRM TC 16.3.2.2.5 with TT | 18.1.0 |
| 2023-12 | RAN#102 | R5-236727 | 2754 | - | F | Correction to RedCap RRM TC 16.3.2.2.7 with TT | 18.1.0 |
| 2023-12 | RAN#102 | R5-236728 | 2755 | - | F | Correction to RedCap RRM TC 16.3.2.3.1 with TT | 18.1.0 |
| 2023-12 | RAN#102 | R5-236730 | 2757 | - | F | Correction to RedCap RRM TC 16.5.2.1 with TT | 18.1.0 |
| 2023-12 | RAN#102 | R5-236731 | 2758 | - | F | Correction to RedCap RRM TC 16.5.2.5 with TT | 18.1.0 |
| 2023-12 | RAN#102 | R5-236732 | 2759 | - | F | Correction to RedCap RRM TC 16.5.2.7 with TT | 18.1.0 |
| 2023-12 | RAN#102 | R5-236734 | 2761 | - | F | Correction to RedCap RRM TC 16.6.1.x | 18.1.0 |
| 2023-12 | RAN#102 | R5-236736 | 2763 | - | F | Corrections to BWP RMCs and configurations for RedCap RRM TCs | 18.1.0 |
| 2023-12 | RAN#102 | R5-236737 | 2764 | - | F | Corrections to Annex E for RedCap RRM TCs | 18.1.0 |
| 2023-12 | RAN#102 | R5-236748 | 2766 | - | F | Correction to RRM enhancement TC 6.5.8.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236749 | 2767 | - | F | Correction to FR1 EN-DC TC 4.6.4.3 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236750 | 2768 | - | F | Correction to FR1 NR SA TC 6.6.4.3 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236751 | 2769 | - | F | Correction to CORESET RMC | 18.1.0 |
| 2023-12 | RAN#102 | R5-236819 | 2773 | - | F | Addition of new test case 7.6.15.1 for SA event triggered reporting tests with concurrent gaps | 18.1.0 |
| 2023-12 | RAN#102 | R5-236821 | 2775 | - | F | Addition of CCR settings for FR1 Link recovery procedures | 18.1.0 |
| 2023-12 | RAN#102 | R5-236822 | 2776 | - | F | Correction to SNR on RLM-RS in 6.5.1.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236825 | 2777 | - | F | Addition of SSB index assigned as BFD-RS to CSI-RS-based RLM for 2Rx | 18.1.0 |
| 2023-12 | RAN#102 | R5-236916 | 2786 | - | F | Correction to message contents for 4.5.2.3 and 4.5.2.4 | 18.1.0 |
| 2023-12 | RAN#102 | R5-236992 | 2787 | - | F | Correction of FR2 EN-DC Inter-freq accuracy measurement test case 5.7.1.2 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-236993 | 2788 | - | F | Correction of FR2 SA Inter-freq accuracy measurement test case 7.7.1.2 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-236994 | 2789 | - | F | TT analysis correction for FR2 Inter-freq accuracy measurement test cases in Annex F | 18.1.0 |
| 2023-12 | RAN#102 | R5-236997 | 2792 | - | F | Correction of NR SA FR2 Cell reselection RedCap test case 17.1.1.1 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-236998 | 2793 | - | F | Correction of NR SA FR2 Cell reselection RedCap test case 17.1.1.2 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-236999 | 2794 | - | F | Correction of NR SA FR2 Cell reselection RedCap test case 17.1.1.3 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237000 | 2795 | - | F | Correction of NR SA FR2 Cell reselection RedCap test case 17.1.1.4 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237015 | 2809 | - | F | Correction of SA event triggered reporting with additional mandatory gap pattern test case 6.6.2.9 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237018 | 2812 | - | F | Addition of cell configuration mapping for rel-16 RRM test cases | 18.1.0 |
| 2023-12 | RAN#102 | R5-237020 | 2814 | - | F | Correction of Inter-RAT FR1 Cell reselection RedCap test case 16.1.2.6 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237095 | 2816 | - | F | Correction to Cell Configuration Maping for TC 6.3.3.2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237096 | 2817 | - | F | Missing DRX.X1 configuration addition to Annex A.5 and formatting adaption | 18.1.0 |
| 2023-12 | RAN#102 | R5-237097 | 2818 | - | F | Core Spec alignment for RRM TC 6.5.1.9 Enh Power Saving | 18.1.0 |
| 2023-12 | RAN#102 | R5-237098 | 2819 | - | F | Core spec alignment for TC 5.5.1.1 and 5.5.1.2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237101 | 2822 | - | F | Addition of minimum conformance requirement for FR1 RedCap 16.7.1.x test cases | 18.1.0 |
| 2023-12 | RAN#102 | R5-237103 | 2824 | - | F | Corrections for references of RedCap 2-step RA TC 16.3.2.2.6 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237104 | 2825 | - | F | Message content exception addition for RedCap 2-step RA TCs | 18.1.0 |
| 2023-12 | RAN#102 | R5-237183 | 2846 | - | F | Addition of clause 4.5.9.0 minimum conformance requirements for UE specific CBW change | 18.1.0 |
| 2023-12 | RAN#102 | R5-237184 | 2847 | - | F | Addition of UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN-DC test case 4.5.9.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237185 | 2848 | - | F | Addition of clause 4.6.6.0 minimum conformance requirements for measurements with autonomous gaps | 18.1.0 |
| 2023-12 | RAN#102 | R5-237187 | 2850 | - | F | Addition of clause 5.5.6.3.0 minimum conformance requirements for simultaneous DCI-based and timer-based active BWP switch on multiple CCs | 18.1.0 |
| 2023-12 | RAN#102 | R5-237189 | 2852 | - | F | Addition of clause 5.5.6.4.0 minimum conformance requirements for SCell dormancy switch | 18.1.0 |
| 2023-12 | RAN#102 | R5-237191 | 2854 | - | F | Addition of clause 5.5.10.0 minimum conformance requirements for UE specific CBW change on FR2 NR Pscell | 18.1.0 |
| 2023-12 | RAN#102 | R5-237192 | 2855 | - | F | Addition of UE specific CBW change on FR2 NR PSCell test case 5.5.10.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237193 | 2856 | - | F | Addition of clause 5.6.5.0 minimum conformance requirements for measurements with autonomous gaps | 18.1.0 |
| 2023-12 | RAN#102 | R5-237195 | 2858 | - | F | Addition of clause 6.6.7.0 minimum conformance requirements for NR measurements with autonomous gaps | 18.1.0 |
| 2023-12 | RAN#102 | R5-237198 | 2861 | - | F | Addition of clause 7.5.2.0.2 minimum conformance requirements for UE specific CBW change | 18.1.0 |
| 2023-12 | RAN#102 | R5-237200 | 2863 | - | F | Addition of clause 7.5.6.3.0 minimum conformance requirements for simultaneous DCI-based and timer-based active BWP switch on multiple CCs | 18.1.0 |
| 2023-12 | RAN#102 | R5-237202 | 2865 | - | F | Addition of clause 7.5.10.0 minimum conformance requirements for UE specific CBW change | 18.1.0 |
| 2023-12 | RAN#102 | R5-237203 | 2866 | - | F | Addition of NR FR2 UE specific CBW change of PCell with non-DRX in SA test case 7.5.10.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237205 | 2868 | - | F | Addition of clause 7.6.5.0 minimum conformance requirements for NR measurements with autonomous gaps | 18.1.0 |
| 2023-12 | RAN#102 | R5-237218 | 2870 | - | F | Update on relative angular offset in 2AoA testing for PC1 devices | 18.1.0 |
| 2023-12 | RAN#102 | R5-237220 | 2872 | - | F | Update on 16.4.1.1 test case | 18.1.0 |
| 2023-12 | RAN#102 | R5-237222 | 2874 | - | F | Update on DRX configuration in message content | 18.1.0 |
| 2023-12 | RAN#102 | R5-237470 | 2827 | 1 | F | Corrections to UE position provision for NR-NTN | 18.1.0 |
| 2023-12 | RAN#102 | R5-237688 | 2830 | 1 | F | Addition of NTN inter-frequency measurement test cases | 18.1.0 |
| 2023-12 | RAN#102 | R5-237689 | 2831 | 1 | F | Addition of NTN parallel measurement gap-based inter-frequency test cases | 18.1.0 |
| 2023-12 | RAN#102 | R5-237702 | 2779 | 1 | F | Correction for 38.533 Annex | 18.1.0 |
| 2023-12 | RAN#102 | R5-237703 | 2772 | 1 | F | Addition of new test case 7.6.14.1 for SA event triggered reporting tests with Pre-MG | 18.1.0 |
| 2023-12 | RAN#102 | R5-237704 | 2774 | 1 | F | Addition of new test case 7.6.16.1 for SA event triggered reporting tests with NCSG | 18.1.0 |
| 2023-12 | RAN#102 | R5-237705 | 2707 | 1 | F | Addition of RedCap RRM TC 16.2.1.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237706 | 2708 | 1 | F | Addition of RedCap RRM TC 16.2.1.2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237707 | 2711 | 1 | F | Addition of RedCap RRM TC 16.7.2.3_1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237708 | 2712 | 1 | F | Addition of RedCap RRM TC 16.7.2.3_2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237709 | 2771 | 1 | F | Addition of powerControlOffsetSS in 16.5.2.2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237710 | 2821 | 1 | F | Typo correction for SNR values at T3 - T5 for TC 16.5.2.4 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237711 | 2823 | 1 | F | Addition of FR1 RedCap SS-RSRP intra-frequency measurement accuracy test cases | 18.1.0 |
| 2023-12 | RAN#102 | R5-237712 | 2810 | 1 | F | Correction of SA event triggered reporting tests for FR1 without gap when DRX is used 6.6.2.10 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237713 | 2811 | 1 | F | Correction of SA event triggered reporting tests for FR1 without gap when DRX is not used 6.6.2.11 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237714 | 2813 | 1 | F | Correction of Annex H to cover additional mandatory gap pattern with MGL 3ms | 18.1.0 |
| 2023-12 | RAN#102 | R5-237715 | 2849 | 1 | F | Addition of EN-DC intra-frequency CGI identification of NR FR1 cell with autonomous gaps in synchronous EN-DC test case 4.6.6.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237716 | 2851 | 1 | F | Addition of E-UTRAN - NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC test case 5.5.6.3.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237717 | 2853 | 1 | F | Addition of E-UTRAN - NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time test case 5.5.6.4.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237718 | 2857 | 1 | F | Addition of EN-DC inter-frequency CGI identification of NR neighbour cell in FR2 test case 5.6.5.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237719 | 2820 | 1 | F | Addition of SA FR1 NR-U SS-RSRQ measurement accuracy test cases | 18.1.0 |
| 2023-12 | RAN#102 | R5-237720 | 2699 | 1 | F | Correct of RRM Test Case | 18.1.0 |
| 2023-12 | RAN#102 | R5-237737 | 2836 | 1 | F | Addition of NR-U inter-frequency RSSI and Channel Occupancy tests | 18.1.0 |
| 2023-12 | RAN#102 | R5-237738 | 2837 | 1 | F | Addition of NR-U EN-DC unlicensed event triggered inter-frequency test cases | 18.1.0 |
| 2023-12 | RAN#102 | R5-237739 | 2838 | 1 | F | Addition of NR-U EN-DC inter-frequency licensed cell reporting test cases | 18.1.0 |
| 2023-12 | RAN#102 | R5-237740 | 2839 | 1 | F | Addition of NR-U EN-DC DCI-based and RRC-based Active BWP Switch test cases | 18.1.0 |
| 2023-12 | RAN#102 | R5-237741 | 2840 | 1 | F | Addition of NR-U EN-DC PSCell addition and release delay test case | 18.1.0 |
| 2023-12 | RAN#102 | R5-237742 | 2713 | 1 | F | Addition of RedCap RRM TC 16.7.2.4_1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237743 | 2714 | 1 | F | Addition of RedCap RRM TC 16.7.2.4_2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237744 | 2715 | 1 | F | Addition of RedCap RRM TC 16.7.3.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237745 | 2716 | 1 | F | Addition of RedCap RRM TC 16.7.3.2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237746 | 2717 | 1 | F | Addition of RedCap RRM TC 16.7.3.3_1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237747 | 2718 | 1 | F | Addition of RedCap RRM TC 16.7.3.3_2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237748 | 2719 | 1 | F | Addition of RedCap RRM TC 16.7.3.4_1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237749 | 2720 | 1 | F | Addition of RedCap RRM TC 16.7.3.4_2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237750 | 2729 | 1 | F | Addition of RedCap RRM TC 16.7.5.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237751 | 2730 | 1 | F | Addition of RedCap RRM TC 16.7.5.2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237752 | 2733 | 1 | F | Addition of RedCap RRM TC 17.2.1.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237753 | 2734 | 1 | F | Addition of RedCap RRM TC 17.3.1.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237754 | 2735 | 1 | F | Addition of RedCap RRM TC 17.3.1.2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237755 | 2737 | 1 | F | Addition of RedCap RRM TC 17.4.1.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237756 | 2741 | 1 | F | Addition of RedCap RRM TC 17.5.1.7 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237757 | 2743 | 1 | F | Addition of RedCap RRM TC 17.5.4.1.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237758 | 2744 | 1 | F | Addition of RedCap RRM TC 17.5.4.2.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237759 | 2745 | 1 | F | Addition of RedCap RRM TC 17.6.4.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237760 | 2859 | 1 | F | Addition of SA intra-frequency CGI identification of NR neighbor cell in FR1 test case 6.6.7.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237761 | 2860 | 1 | F | Addition of identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA test case 6.6.7.2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237762 | 2862 | 1 | F | Addition of SA interruptions at NR SRS carrier-based switching test case 7.5.2.2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237763 | 2864 | 1 | F | Addition of Active BWP switch on multiple SCells with non-DRX in SA test case 7.5.6.3.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237764 | 2867 | 1 | F | Addition of SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17) test case 7.6.2.9 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237765 | 2869 | 1 | F | Addition of SA inter-frequency CGI reporting in autonomous gaps test (PCell in FR2) test case 7.6.5.1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237766 | 2835 | 1 | F | Applicability update for r17 HST inter-frequency tests | 18.1.0 |
| 2023-12 | RAN#102 | R5-237767 | 2843 | 1 | F | Applicability update to FR2 inter-frequency tests | 18.1.0 |
| 2023-12 | RAN#102 | R5-237768 | 2841 | 1 | F | Editorial changes to 38.533 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237769 | 2844 | 1 | F | Applicability update for r16 supported gap pattern | 18.1.0 |
| 2023-12 | RAN#102 | R5-237770 | 2845 | 1 | F | Update to RRM test 6.3.1.4 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237785 | 2681 | 1 | F | Completion of NR SA FR1 intra-frequency CGI identification of NR neighbour cell for 1 Rx UE | 18.1.0 |
| 2023-12 | RAN#102 | R5-237786 | 2747 | 1 | F | Correction to RedCap RRM TC 16.3.1.9 with TT | 18.1.0 |
| 2023-12 | RAN#102 | R5-237787 | 2750 | 1 | F | Correction to RedCap RRM TC 16.3.2.1.5 with TT | 18.1.0 |
| 2023-12 | RAN#102 | R5-237788 | 2756 | 1 | F | Correction to RedCap RRM TC 16.3.2.3.3 with TT | 18.1.0 |
| 2023-12 | RAN#102 | R5-237789 | 2790 | 1 | F | Correction of NR SA FR1 Cell reselection RedCap test case 16.1.1.1 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237790 | 2791 | 1 | F | Annex F correction for RedCap test cases including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237793 | 2683 | 1 | F | Completion of 7.6.3.6 test case with message content section and test tolerance values. | 18.1.0 |
| 2023-12 | RAN#102 | R5-237803 | 2686 | 1 | F | Completion of EN-DC FR1 test case for the conditional addition delay of PSCell | 18.1.0 |
| 2023-12 | RAN#102 | R5-237804 | 2691 | 1 | F | Measurement uncertainty and test tolerance updation of fast SCell activation and CPAC test cases | 18.1.0 |
| 2023-12 | RAN#102 | R5-237805 | 2700 | 1 | F | Update of RRM Test Case 4.5.2.10 EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell | 18.1.0 |
| 2023-12 | RAN#102 | R5-237806 | 2701 | 1 | F | Update of RRM Test Case 4.5.10.1 EN-DC FR1 PSCell activation and deactivation delay | 18.1.0 |
| 2023-12 | RAN#102 | R5-237807 | 2702 | 1 | F | Update of RRM Test Case 5.5.12.1 EN-DC FR2 PSCell activation and deactivation delay | 18.1.0 |
| 2023-12 | RAN#102 | R5-237808 | 2781 | 1 | F | Update of 5.5.13.1 Conditional PSCell Addition including test tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237809 | 2782 | 1 | F | Update of 7.5.12.1 Conditional PSCell Addition including test tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237810 | 2785 | 1 | F | Update of Annex F for MR-DC enhancement test cases including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237811 | 2842 | 1 | F | Update to SA FR2 CSI-RS based RLM test cases including TT | 18.1.0 |
| 2023-12 | RAN#102 | R5-237827 | 2664 | 1 | F | Modification to CG-SDT RRM test case for FR2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237830 | 2808 | 1 | F | Editorial correction of test cases 7.1.1.x | 18.1.0 |
| 2023-12 | RAN#102 | R5-237833 | 2832 | 1 | F | Addition of NTN intra-frequency gap-based measurement test cases | 18.1.0 |
| 2023-12 | RAN#102 | R5-237865 | 2828 | 1 | F | Satllite access configuration update | 18.1.0 |
| 2023-12 | RAN#102 | R5-237866 | 2829 | 1 | F | Updates to High level test procedure for SAN RRM tests | 18.1.0 |
| 2023-12 | RAN#102 | R5-237886 | 2796 | 1 | F | Correction of NR SA FR1-FR1 Event triggered reporting with DRX and without SSB time index for 1Rx test case 16.6.2.1 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237887 | 2797 | 1 | F | Correction of NR SA FR1-FR1 Event triggered reporting with DRX and without SSB time index for 2Rx test case 16.6.2.2 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237888 | 2798 | 1 | F | Correction of NR SA FR1-FR1 Event triggered reporting without DRX and without SSB time index for 1Rx test case 16.6.2.3 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237889 | 2799 | 1 | F | Correction of NR SA FR1-FR1 Event triggered reporting without DRX and without SSB time index for 2Rx test case 16.6.2.4 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237890 | 2800 | 1 | F | Correction of NR SA FR1-FR1 Event triggered reporting without DRX and with SSB time index for 1Rx test case 16.6.2.5 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237891 | 2801 | 1 | F | Correction of NR SA FR1-FR1 Event triggered reporting without DRX and with SSB time index for 2Rx UE test case 16.6.2.6 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237892 | 2802 | 1 | F | Correction of NR SA FR1-FR1 Event triggered reporting with DRX and SSB time index for 1Rx UE test case 16.6.2.7 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237893 | 2803 | 1 | F | Correction of NR SA FR1-FR1 Event triggered reporting with DRX and SSB time index for 2Rx UE test case 16.6.2.8 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237894 | 2804 | 1 | F | Correction of NR SA FR1-FR1 Event triggered reporting with additional mandatory gap pattern for 1Rx test case 16.6.2.9 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237895 | 2805 | 1 | F | Correction of NR SA FR1-FR1 Event triggered reporting with additional mandatory gap pattern for 2Rx UEtest case 16.6.2.10 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237896 | 2806 | 1 | F | Correction of NR SA FR1-FR1 Event triggered reporting without measurement gap for 1Rx UE test case 16.6.2.11 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237897 | 2815 | 1 | F | SMTC configuration for RedCap | 18.1.0 |
| 2023-12 | RAN#102 | R5-237914 | 2682 | 1 | F | Completion of NR SA FR1 intra-frequency CGI identification of NR neighbour cell for 2 Rx UE | 18.1.0 |
| 2023-12 | RAN#102 | R5-237915 | 2710 | 1 | F | Addition of RedCap RRM TC 16.6.5.4 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237916 | 2721 | 1 | F | Addition of RedCap RRM TC 16.7.4.1_1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237917 | 2722 | 1 | F | Addition of RedCap RRM TC 16.7.4.1_2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237918 | 2723 | 1 | F | Addition of RedCap RRM TC 16.7.4.2_1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237919 | 2724 | 1 | F | Addition of RedCap RRM TC 16.7.4.2_2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237920 | 2725 | 1 | F | Addition of RedCap RRM TC 16.7.4.3_1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237921 | 2726 | 1 | F | Addition of RedCap RRM TC 16.7.4.3_2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237922 | 2727 | 1 | F | Addition of RedCap RRM TC 16.7.4.4_1 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237923 | 2728 | 1 | F | Addition of RedCap RRM TC 16.7.4.4_2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237924 | 2746 | 1 | F | Correction to RedCap RRM TC 16.3.1.7 with TT | 18.1.0 |
| 2023-12 | RAN#102 | R5-237925 | 2762 | 1 | F | Correction to RedCap RRM TC 16.6.7.1 and 16.6.7.2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237926 | 2765 | 1 | F | Corrections to Annex F for RedCap RRM TCs | 18.1.0 |
| 2023-12 | RAN#102 | R5-237927 | 2826 | 1 | F | Corrections for RedCap RA TCs | 18.1.0 |
| 2023-12 | RAN#102 | R5-237938 | 2705 | 1 | F | Addition of new test case 7.6.14.2 for FR2 Pre-MG | 18.1.0 |
| 2023-12 | RAN#102 | R5-237942 | 2807 | 1 | F | Correction of NR SA FR1-FR1 Event triggered reporting without measurement gap for 2Rx UE test case 16.6.2.12 including Test Tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237950 | 2778 | 1 | F | Correction to message exception in 16.5.3.1.2 and 16.5.3.2.2 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237951 | 2693 | 1 | F | Updating unified TCI switch test cases to add MU and TT | 18.1.0 |
| 2023-12 | RAN#102 | R5-237953 | 2783 | 1 | F | Update of 7.5.14 PSCell activation including test tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237954 | 2780 | 1 | F | Update of 7.5.3.14 Fast SCell activation including test tolerance | 18.1.0 |
| 2023-12 | RAN#102 | R5-237959 | 2675 | 1 | F | Addition of new NR SA FR2 event triggered reporting test case for PC6 devices | 18.1.0 |
| 2023-12 | RAN#102 | R5-237960 | 2833 | 1 | F | Update to FR1 CG-SDT test case including TTs | 18.1.0 |
| 2023-12 | RAN#102 | R5-237962 | 2834 | 1 | F | Update to RRM tests 16.3.1.7 and 16.3.1.8 | 18.1.0 |
| 2023-12 | RAN#102 | R5-237966 | 2871 | 1 | F | Update on 6.5.1.9 RLM for relaxed measurement criteria | 18.1.0 |
| 2024-03 | RAN#103 | R5-240026 | 2875 | - | F | Editorial corrections for 6.7.3.1 and 6.7.3.2.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240369 | 2884 | - | F | Addition of NR SA FR1 DL interruptions at switching between two uplink carriers for 6.5.7A.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240372 | 2887 | - | F | Test tolerances and measurement uncertainty for DL interruptions test cases 6.5.7A.1, 6.5.7B.1 and 6.5.7C.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240381 | 2888 | - | F | Update of test procedure in 4.5.10.1 PSCell activation for EN-DC | 18.2.0 |
| 2024-03 | RAN#103 | R5-240382 | 2889 | - | F | Correction to 4.5.11.1 EN-DC FR1 Conditional PSCell Addition | 18.2.0 |
| 2024-03 | RAN#103 | R5-240383 | 2890 | - | F | Update of measurement configuration for 5.5.13.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240384 | 2891 | - | F | Update of measurement configuration for 7.5.12.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240385 | 2892 | - | F | Update of message contents for 4.5.3.7 and 6.5.3.11 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240449 | 2893 | - | F | Resubmission of Annex B Corrections to BWP RMCs and configurations for RedCap RRM TCs | 18.2.0 |
| 2024-03 | RAN#103 | R5-240518 | 2894 | - | F | Update to NR-U redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA test case | 18.2.0 |
| 2024-03 | RAN#103 | R5-240519 | 2895 | - | F | Update to NR-U redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA test case | 18.2.0 |
| 2024-03 | RAN#103 | R5-240681 | 2901 | - | F | Addition of RedCap RRM test case 17.5.3.1.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240682 | 2902 | - | F | Addition of RedCap RRM test case 17.5.3.2.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240683 | 2903 | - | F | Addition of RedCap RRM test case 17.5.5.1.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240684 | 2904 | - | F | Addition of RedCap RRM test case 17.5.5.2.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240685 | 2905 | - | F | Addition of RedCap RRM test case 17.5.6.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240686 | 2906 | - | F | Addition of RedCap RRM test case 17.7.1.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240687 | 2907 | - | F | Addition of RedCap RRM test case 17.7.1.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240688 | 2908 | - | F | Addition of RedCap RRM test case 17.7.2.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240689 | 2909 | - | F | Addition of RedCap RRM test case 17.7.2.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240690 | 2910 | - | F | Addition of RedCap RRM test case 17.7.3.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240691 | 2911 | - | F | Addition of RedCap RRM test case 17.7.3.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240692 | 2912 | - | F | Addition of RedCap RRM test case 17.7.4.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240695 | 2915 | - | F | Correction to RedCap RRM test case 16.5.2.3 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240696 | 2916 | - | F | Correction to RedCap RRM test case 16.5.4.1 and 16.5.4.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240697 | 2917 | - | F | Correction to RedCap RRM test case 17.4.1.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240698 | 2918 | - | F | Correction to RedCap RRM test case 17.6.4.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240699 | 2919 | - | F | Correction to SSB time offset in NCD-SSB test cases | 18.2.0 |
| 2024-03 | RAN#103 | R5-240700 | 2920 | - | F | Correction to RRM enh test case 6.5.8.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240707 | 2927 | - | F | Correction to RedCap RRM test cases 16.3.2.2.X with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240709 | 2929 | - | F | Correction to RedCap RRM test case 16.6.5.4 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240710 | 2930 | - | F | Correction to RedCap RRM test case 16.7.2.3.1 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240714 | 2934 | - | F | Correction to RedCap RRM test case 16.7.3.1 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240715 | 2935 | - | F | Correction to RedCap RRM test case 16.7.3.2 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240716 | 2936 | - | F | Correction to RedCap RRM test case 16.7.3.3.1 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240717 | 2937 | - | F | Correction to RedCap RRM test case 16.7.3.3.2 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240718 | 2938 | - | F | Correction to RedCap RRM test case 16.7.3.4.1 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240720 | 2940 | - | F | Correction to RedCap RRM test case 16.7.4.1.1 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240721 | 2941 | - | F | Correction to RedCap RRM test case 16.7.4.1.2 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240722 | 2942 | - | F | Correction to RedCap RRM test case 16.7.4.2.1 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240723 | 2943 | - | F | Correction to RedCap RRM test case 16.7.4.2.2 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240724 | 2944 | - | F | Correction to RedCap RRM test case 16.7.4.3.1 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240725 | 2945 | - | F | Correction to RedCap RRM test case 16.7.4.3.2 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240726 | 2946 | - | F | Correction to RedCap RRM test case 16.7.4.4.1 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240727 | 2947 | - | F | Correction to RedCap RRM test case 16.7.4.4.2 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240728 | 2948 | - | F | Correction to RedCap RRM test case 16.7.5.1 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240729 | 2949 | - | F | Correction to RedCap RRM test case 16.7.5.2 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240730 | 2950 | - | F | Correction to RedCap RRM test case 16.7.6.1 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240731 | 2951 | - | F | Correction to RedCap RRM test case 16.7.6.2 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240758 | 2955 | - | F | Correction to FR1 4-step RACH test cases with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240759 | 2956 | - | F | Correction to FR1 2-step RACH test cases with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240760 | 2957 | - | F | Correction to FR1 inter-frequency SS SINR relative accuracy test cases with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240761 | 2958 | - | F | Correction to FR1 L1 RSRP absolute accuracy test cases with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240762 | 2959 | - | F | Correction to FR1 LTE RSRP accuracy test case 6.7.5.1 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-240790 | 2961 | - | F | Addition of Test Tolerance into Annex F for RedCap test cases | 18.2.0 |
| 2024-03 | RAN#103 | R5-240793 | 2962 | - | F | Correction of Test tolerance for FR2 Inter frequency test case | 18.2.0 |
| 2024-03 | RAN#103 | R5-240798 | 2967 | - | F | Addition of NR SA FR2-FR2 Event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) test case 17.6.2.1 including Test Tolerance | 18.2.0 |
| 2024-03 | RAN#103 | R5-240799 | 2968 | - | F | Addition of NR SA FR2-FR2 Event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2) test case 17.6.2.2 including Test Tolerance | 18.2.0 |
| 2024-03 | RAN#103 | R5-240800 | 2969 | - | F | Addition of NR SA FR2-FR2 Event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2) test case 17.6.2.3 including Test Tolerance | 18.2.0 |
| 2024-03 | RAN#103 | R5-240801 | 2970 | - | F | Addition of NR SA FR2-FR2 Event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) test case 17.6.2.4 including Test Tolerance | 18.2.0 |
| 2024-03 | RAN#103 | R5-240802 | 2971 | - | F | Addition of cell configuration mapping for event triggered measurement test cases for RedCap | 18.2.0 |
| 2024-03 | RAN#103 | R5-240803 | 2972 | - | F | Addition of NR_U test case 11.5.1.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240804 | 2973 | - | F | Addition of NR_U test case 11.5.1.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240805 | 2974 | - | F | Addition of NR_U test case 11.5.1.3 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240806 | 2975 | - | F | Addition of NR_U test case 11.5.1.4 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240807 | 2976 | - | F | Addition of NR_U test case 11.5.1.9 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240809 | 2978 | - | F | Addition of NR_U test case 11.5.2.4 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240811 | 2979 | - | F | Addition of NR_U test case 11.5.2.5 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240812 | 2980 | - | F | Addition of NR_U test case 11.5.2.6 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240813 | 2981 | - | F | Addition of NR_U test case 11.5.2.7 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240814 | 2982 | - | F | Addition of NR_U test case 11.5.2.8 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240815 | 2983 | - | F | Addition of NR_U test case 11.5.2.9 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240816 | 2984 | - | F | Addition of NR_U test case 11.5.2.10 | 18.2.0 |
| 2024-03 | RAN#103 | R5-240920 | 2987 | - | F | Cell Mapping correction for TC 6.5.6.1.1 in Annex E | 18.2.0 |
| 2024-03 | RAN#103 | R5-240921 | 2988 | - | F | Cell Mapping correction for TC 6.6.1.8 in Annex E | 18.2.0 |
| 2024-03 | RAN#103 | R5-241114 | 2990 | - | F | Correction to OCNG references in 16.1.1 and 16.1.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241115 | 2991 | - | F | Correction to title of RedCap test cases | 18.2.0 |
| 2024-03 | RAN#103 | R5-241116 | 2992 | - | F | Correction to PRACH config in 16.5.2.4 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241120 | 2996 | - | F | Correction to QuantityConfigu-DEFAULT in H.3.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241160 | 3005 | - | F | Correction to EN-DC FR1 Beam Failure TCs 4.5.5.x | 18.2.0 |
| 2024-03 | RAN#103 | R5-241162 | 3006 | - | F | Correction to EN-DC FR2 Beam Failure TCs 5.5.5.x | 18.2.0 |
| 2024-03 | RAN#103 | R5-241198 | 3008 | - | F | Addition of new test case 4.7.8.2 EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | 18.2.0 |
| 2024-03 | RAN#103 | R5-241200 | 3009 | - | F | Correct of EN-DC FR1 addition and release delay of known PSCell for test case 4.5.7.1 including Test Tolerance | 18.2.0 |
| 2024-03 | RAN#103 | R5-241211 | 3012 | - | F | Annex E.14 correction for new RedCap test cases | 18.2.0 |
| 2024-03 | RAN#103 | R5-241213 | 3014 | - | F | Correction of NR SA FR1 SS-RSRP absolute measurement accuracy for RedCap 1 Rx UE test case 16.7.1.1.1 including Test Tolerance | 18.2.0 |
| 2024-03 | RAN#103 | R5-241214 | 3015 | - | F | Correction of NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE test case 16.7.1.1.2 including Test Tolerance | 18.2.0 |
| 2024-03 | RAN#103 | R5-241215 | 3016 | - | F | Correction of NR SA FR1 SS-RSRP absolute measurement accuracy for RedCap 2 Rx UE test case 16.7.1.2.1 including Test Tolerance | 18.2.0 |
| 2024-03 | RAN#103 | R5-241216 | 3017 | - | F | Correction of NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE test case 16.7.1.2.2 including Test Tolerance | 18.2.0 |
| 2024-03 | RAN#103 | R5-241217 | 3018 | - | F | Addition of NR SA FR1 SS-RSRQ measurement accuracy for 1 Rx UE test case 16.7.2.1 including Test Tolerance | 18.2.0 |
| 2024-03 | RAN#103 | R5-241218 | 3019 | - | F | Addition of NR SA FR1 SS-RSRQ measurement accuracy for 2 Rx UE test case 16.7.2.2 including Test Tolerance | 18.2.0 |
| 2024-03 | RAN#103 | R5-241219 | 3020 | - | F | Addition of RRM RedCap test cases 16.7.1.3.1, 16.7.1.3.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241220 | 3021 | - | F | Addition of RRM RedCap test cases 16.7.1.4.1, 16.7.1.4.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241227 | 3022 | - | F | Editorial Correction to RedCap PRACH TC 16.3.2.2.8 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241228 | 3023 | - | F | Editorial corrections for RedCap test cases 18.3.1.x | 18.2.0 |
| 2024-03 | RAN#103 | R5-241233 | 3026 | - | F | Editorial correction in Table 5.6.1.3.5-2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241235 | 3028 | - | F | Correction to BFD-LR test case 5.5.5.5 message content | 18.2.0 |
| 2024-03 | RAN#103 | R5-241238 | 3031 | - | F | Editor correction to Test Procedure 5.5.1.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241241 | 3032 | - | F | Addition of SA FR1 NR-U intra-frequency SS-SINR measurement accuracy test cases | 18.2.0 |
| 2024-03 | RAN#103 | R5-241242 | 3033 | - | F | Addition of SA FR1 NR-U inter-frequency SS-SINR measurement accuracy test cases | 18.2.0 |
| 2024-03 | RAN#103 | R5-241243 | 3034 | - | F | Addition of SA FR1 NR-U intra-frequency RSSI measurement accuracy test cases | 18.2.0 |
| 2024-03 | RAN#103 | R5-241264 | 3035 | - | F | Core Spec alignment for RedCap test cases 18.3.1.x | 18.2.0 |
| 2024-03 | RAN#103 | R5-241265 | 3036 | - | F | General update to NTN tests | 18.2.0 |
| 2024-03 | RAN#103 | R5-241266 | 3037 | - | F | Addition of NTN timing accuracy test 14.3.1.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241267 | 3038 | - | F | Addition of NTN time-based CHO tests 14.2.1.x | 18.2.0 |
| 2024-03 | RAN#103 | R5-241268 | 3039 | - | F | Addition of NTN intra-frequency cell reselection tests 14.1.x | 18.2.0 |
| 2024-03 | RAN#103 | R5-241269 | 3040 | - | F | Addition of NTN SS-SINR measurement accuracy tests 14.6.3.x | 18.2.0 |
| 2024-03 | RAN#103 | R5-241272 | 3042 | - | F | Addition of NR-U SA BWP switch tests including TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-241273 | 3043 | - | F | Addition of UL active BWP switch test 10.3.5.1 including TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-241274 | 3044 | - | F | Update of NR-U Annex E and F including TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-241275 | 3045 | - | F | Update to NR-U general sections | 18.2.0 |
| 2024-03 | RAN#103 | R5-241276 | 3046 | - | F | Addition of NR-U UE Timing Tests 10.2.x | 18.2.0 |
| 2024-03 | RAN#103 | R5-241277 | 3047 | - | F | Addition of NR-U L1-RSRP accuracy test 10.5.4.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241278 | 3048 | - | F | Addition of NR-U RSSI measurement accuracy tests 10.5.5.x | 18.2.0 |
| 2024-03 | RAN#103 | R5-241279 | 3049 | - | F | Addition of NR-U Channel occupancy measurement accuracy tests 10.5.6.x | 18.2.0 |
| 2024-03 | RAN#103 | R5-241280 | 3050 | - | F | Addition of NR-U L1-RSRP tests 10.4.3.x | 18.2.0 |
| 2024-03 | RAN#103 | R5-241281 | 3051 | - | F | Addition of NR-U IRAT tests 10.4.4.x | 18.2.0 |
| 2024-03 | RAN#103 | R5-241282 | 3052 | - | F | Update to NSA NR-U tests including TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-241283 | 3053 | - | F | Update to SA NR-U tests including TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-241284 | 3054 | - | F | Update to OCNG noise power in UL CCA model | 18.2.0 |
| 2024-03 | RAN#103 | R5-241285 | 3055 | - | F | Core alignment for NR-U common sections | 18.2.0 |
| 2024-03 | RAN#103 | R5-241295 | 3056 | - | F | Cleanup on FR1 CG-SDT test 6.2.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241296 | 3057 | - | F | Addition of RedCap SA FR2 SSB RLM OOS in DRX test case 17.5.1.3 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241297 | 3058 | - | F | Addition of RedCap SA FR2 SSB BFR test cases 17.5.2.1 and 17.5.2.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241298 | 3059 | - | F | Update to RRM test 16.3.1.8 including TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-241299 | 3060 | - | F | Correction to stationary relaxed measurement criterion tests | 18.2.0 |
| 2024-03 | RAN#103 | R5-241300 | 3061 | - | F | Update to Re-establishment test cases | 18.2.0 |
| 2024-03 | RAN#103 | R5-241301 | 3062 | - | F | Applicability update for several RedCap tests | 18.2.0 |
| 62024-03 | RAN#103 | R5-241304 | 3063 | - | F | Update to RRM test 6.3.1.4 including TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-241311 | 3064 | - | F | Addition of Clause 4.7.9.0 minimum conformance requirements for CSI-RSRQ | 18.2.0 |
| 2024-03 | RAN#103 | R5-241312 | 3065 | - | F | Addition of EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 4.7.9.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241313 | 3066 | - | F | Addition of EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 4.7.9.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241314 | 3067 | - | F | Addition of Clause 4.7.10.0 minimum conformance requirements for CSI-SINR | 18.2.0 |
| 2024-03 | RAN#103 | R5-241315 | 3068 | - | F | Addition of EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 4.7.10.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241316 | 3069 | - | F | Addition of EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell l test case 4.7.10.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241317 | 3070 | - | F | Addition of Clause 5.7.8.0 minimum conformance requirements for CSI-RSRQ | 18.2.0 |
| 2024-03 | RAN#103 | R5-241318 | 3071 | - | F | Addition of EN-DC intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell test case 5.7.8.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241319 | 3072 | - | F | Addition of EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 5.7.8.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241320 | 3073 | - | F | Addition of Clause 5.7.9.0 minimum conformance requirements for CSI-SINR | 18.2.0 |
| 2024-03 | RAN#103 | R5-241321 | 3074 | - | F | Addition of EN-DC intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 5.7.9.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241322 | 3075 | - | F | Addition of EN-DC inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 5.7.9.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241323 | 3076 | - | F | Addition of Clause 6.7.11.0 minimum conformance requirements for CSI-RSRQ | 18.2.0 |
| 2024-03 | RAN#103 | R5-241324 | 3077 | - | F | Addition of SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 6.7.11.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241325 | 3078 | - | F | Addition of SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 6.7.11.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241326 | 3079 | - | F | Addition of Clause 6.7.12.0 minimum conformance requirements for CSI-SINR | 18.2.0 |
| 2024-03 | RAN#103 | R5-241327 | 3080 | - | F | Addition of SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 6.7.12.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241328 | 3081 | - | F | Addition of SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 6.7.12.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241329 | 3082 | - | F | Addition of Clause 7.7.8.0 minimum conformance requirements for CSI-RSRQ | 18.2.0 |
| 2024-03 | RAN#103 | R5-241330 | 3083 | - | F | Addition of SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell test case 7.7.8.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241331 | 3084 | - | F | Addition of SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 7.7.8.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241332 | 3085 | - | F | Addition of Clause 7.7.9.0 minimum conformance requirements for CSI-SINR | 18.2.0 |
| 2024-03 | RAN#103 | R5-241333 | 3086 | - | F | Addition of SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell test case 7.7.9.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241335 | 3088 | - | F | Addition of test cases 4.5.9.1, 5.5.6.3.1, and 5.5.6.4.1 to Annex E | 18.2.0 |
| 2024-03 | RAN#103 | R5-241447 | 3091 | - | F | Correction to Annex F for R15 RRM test cases | 18.2.0 |
| 2024-03 | RAN#103 | R5-241670 | 2925 | 1 | F | Correction to RedCap RRM test case 16.2.1.1 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-241671 | 2926 | 1 | F | Correction to RedCap RRM test case 16.2.1.2 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-241672 | 2952 | 2 | F | Correction to Annex F for RedCap RRM test cases | 18.2.0 |
| 2024-03 | RAN#103 | R5-241673 | 2953 | 1 | F | Correction to SDT RRM test case 6.2.1 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-241677 | 2898 | 1 | F | Correction to FR2 SA event triggered reporting tests with Pre-MG including TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-241678 | 2899 | 1 | F | Correction to FR2 SA event triggered reporting tests with concurrent gaps including TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-241679 | 2900 | 1 | F | Correction to FR2 SA event triggered reporting tests with NCSG including TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-241826 | 2985 | 1 | F | Removal of Editor Note affecting to TC 5.5.5.9 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241827 | 2977 | 1 | F | Addition of NR_U test case 11.5.2.3 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241828 | 3041 | 1 | F | Core alignment for NSA NR-U tests | 18.2.0 |
| 2024-03 | RAN#103 | R5-241829 | 2921 | 1 | F | Correction to RRM enh test case 6.6.2.9 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241830 | 3007 | 1 | F | Addition of new test case 4.7.8.1 EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell | 18.2.0 |
| 2024-03 | RAN#103 | R5-241831 | 3087 | 1 | F | Addition of SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell test case 7.7.9.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241832 | 2886 | 1 | F | Cell configuration mapping for DL interruptions test cases 6.5.7A.1 and 6.5.7B.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241833 | 2989 | 1 | F | Correct Table 4.3.2.2.1.4.1-2 & Table 4.3.2.2.2.4.1-2 & Table 4.3.2.2.3.4.1-2 & Table 4.3.2.2.4.4.1-2 of test frequency | 18.2.0 |
| 2024-03 | RAN#103 | R5-241834 | 2995 | 1 | F | Correction to test applicability of 4.6.1.7 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241835 | 3025 | 1 | F | Correction for test case 5.5.6.2.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241836 | 3027 | 1 | F | Corrections to BFD-LR test case 5.5.5.5 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241837 | 3024 | 1 | F | Corrections to test procedure for 2-step RACH TC 6.3.2.2.4 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241838 | 2881 | 1 | F | Correction to Table H.3.1-8A | 18.2.0 |
| 2024-03 | RAN#103 | R5-241839 | 2882 | 1 | F | Correction to Table H.3.1-12 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241840 | 2883 | 1 | F | Correction to Table H.3.1-12A | 18.2.0 |
| 2024-03 | RAN#103 | R5-241862 | 2897 | 1 | F | Introducing the Re-Positioning Concept for FR2 RRM TCs | 18.2.0 |
| 2024-03 | RAN#103 | R5-241886 | 2928 | 1 | F | Correction to RedCap RRM test case 16.6.5.3 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-241893 | 3089 | 1 | F | Addition of test cases 4.5.9.1, 5.5.6.3.1, and 5.5.6.4.1 to Annex F | 18.2.0 |
| 2024-03 | RAN#103 | R5-241896 | 2885 | 1 | F | Addition of NR SA FR1 DL interruptions at switching between two uplink bands for 6.5.7B.1 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241901 | 3010 | 1 | F | Correct of Test Tolerance into Annex F for EN-DC FR1 addition and release delay of known PSCell | 18.2.0 |
| 2024-03 | RAN#103 | R5-241963 | 2939 | 1 | F | Correction to RedCap RRM test case 16.7.3.4.2 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-241964 | 2998 | 1 | F | Correction to test configuration in 16.4.1.2 and 16.5.2.2 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241965 | 2999 | 1 | F | Correction to PDCCH level in CSI-RS based RLM test cases for RedCap | 18.2.0 |
| 2024-03 | RAN#103 | R5-241967 | 2997 | 1 | F | Correction to PDCCH level in 6.5.1.9 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241994 | 2923 | 1 | F | Correction to EN-DC and NR SA SCell activation test cases | 18.2.0 |
| 2024-03 | RAN#103 | R5-241995 | 2924 | 1 | F | Correction to PCI updating formulas in RRM test cases | 18.2.0 |
| 2024-03 | RAN#103 | R5-241996 | 3029 | 1 | F | Removal of Editors note for RRM FR2 RLM in-sync test cases | 18.2.0 |
| 2024-03 | RAN#103 | R5-241997 | 3030 | 1 | F | Revision of Editors note for RRM FR2 RLM test cases | 18.2.0 |
| 2024-03 | RAN#103 | R5-241998 | 3003 | 1 | F | Correction to TRS Configuration in 6.3.2.1.3 | 18.2.0 |
| 2024-03 | RAN#103 | R5-241999 | 3004 | 1 | F | Correction to PDCCH level in 6.5.1.8 | 18.2.0 |
| 2024-03 | RAN#103 | R5-242021 | 3002 | 1 | F | Correction to test parameters in 5.6.1.3 and 5.6.1.4 | 18.2.0 |
| 2024-03 | RAN#103 | R5-242033 | 2963 | 1 | F | Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 1 Rx UE test case 16.6.3.1 including Test Tolerance | 18.2.0 |
| 2024-03 | RAN#103 | R5-242034 | 2964 | 1 | F | Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2 Rx UE test case 16.6.3.2 including Test Tolerance | 18.2.0 |
| 2024-03 | RAN#103 | R5-242035 | 2965 | 1 | F | Addition of NR - E-UTRA event-triggered reporting in DRX in FR1 for 1 Rx UE test case 16.6.3.3 including Test Tolerance | 18.2.0 |
| 2024-03 | RAN#103 | R5-242036 | 2966 | 1 | F | Addition of NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2Rx UE test case 16.6.3.4 including Test Tolerance | 18.2.0 |
| 2024-03 | RAN#103 | R5-242037 | 2879 | 1 | F | Correction of Redcap inter-RAT E-UTRAN CGI test cases | 18.2.0 |
| 2024-03 | RAN#103 | R5-242039 | 2913 | 1 | F | Correction to NCD-SSB and SMTC RMCs | 18.2.0 |
| 2024-03 | RAN#103 | R5-242040 | 2914 | 1 | F | Correction to RedCap inter-frequency measurement test cases | 18.2.0 |
| 2024-03 | RAN#103 | R5-242041 | 2931 | 1 | F | Correction to RedCap RRM test case 16.7.2.3.2 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-242042 | 2932 | 1 | F | Correction to RedCap RRM test case 16.7.2.4.1 with TT | 18.2.0 |
| 2024-03 | RAN#103 | R5-242043 | 2933 | 1 | F | Correction to RedCap RRM test case 16.7.2.4.2 with TT | 18.2.0 |
| 2024-03 | RAN#103 | - | - | - | - | addition of small missing changes of R5-241965, R5-242040, R5-240718, R5-241963 | 18.2.1 |
| 2024-06 | RAN#104 | R5-242371 | 3094 | - | F | Introduction of band n106 to NR band group for FR1 | 18.3.0 |
| 2024-06 | RAN#104 | R5-242579 | 3096 | - | F | Correction to TC titles for event triggered reporting with measurement gap | 18.3.0 |
| 2024-06 | RAN#104 | R5-242609 | 3100 | - | F | Editorial correction to 7.6.3.6 test case | 18.3.0 |
| 2024-06 | RAN#104 | R5-242611 | 3101 | - | F | Incorrect clause number for test case 7.6.13.1 | 18.3.0 |
| 2024-06 | RAN#104 | R5-242612 | 3102 | - | F | Addition of cell configuration mapping for HST FR2 test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-242636 | 3107 | - | F | Correction of NR SA event triggered reporting tests with additional mandatory gap pattern test case 6.6.2.9 including Test Tolerance | 18.3.0 |
| 2024-06 | RAN#104 | R5-242637 | 3108 | - | F | Correction of NR SA event triggered reporting tests for FR1 without gap when DRX is used test case 6.6.2.10 including Test Tolerance | 18.3.0 |
| 2024-06 | RAN#104 | R5-242638 | 3109 | - | F | Correction of NR SA event triggered reporting tests for FR1 without gap when DRX is not used test case 6.6.2.11 including Test Tolerance | 18.3.0 |
| 2024-06 | RAN#104 | R5-242641 | 3112 | - | F | Test Tolerance correction in Annex F for RRM enhancement test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-242643 | 3114 | - | F | Correction of NR_U SA Inter_RAT measurements test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-242646 | 3116 | - | F | Cell configuration correction in Annex E for SA FR2 RRM enhancement test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-242647 | 3117 | - | F | Minimum conformance requirements for FR2 SA event triggered reporting without gap | 18.3.0 |
| 2024-06 | RAN#104 | R5-242727 | 3126 | - | F | Correct Table 4.5.2.1.4.1-2 & Table 4.5.2.2.4.1-2 & Table 4.5.2.3.4.1-2 & Table 4.5.2.4.4.1-2 & Table 4.5.2.5.4.1-2 of test frequency | 18.3.0 |
| 2024-06 | RAN#104 | R5-242867 | 3129 | - | F | Addition of MCE RRM test case 6.5.7D.1 | 18.3.0 |
| 2024-06 | RAN#104 | R5-242868 | 3130 | - | F | Addition of MCE RRM test case 6.5.7D.2 | 18.3.0 |
| 2024-06 | RAN#104 | R5-242869 | 3131 | - | F | Addition of MCE RRM test case 6.5.7D.3 | 18.3.0 |
| 2024-06 | RAN#104 | R5-242870 | 3132 | - | F | Correction to Annex E for MCE RRM test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-242872 | 3133 | - | F | Correction to R15 L1-RSRP absolute accuracy RRM test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-242875 | 3136 | - | F | Correction to R15 SCell activation test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-242876 | 3137 | - | F | Correction to R16 relaxed measurement RRM test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-242878 | 3138 | - | F | Correction to RedCap RRM test case 16.1.1.5 and 16.1.1.7 | 18.3.0 |
| 2024-06 | RAN#104 | R5-242883 | 3143 | - | F | Correction to RedCap RRM test case 17.3.1.1 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-242884 | 3144 | - | F | Correction to RedCap RRM test case 17.3.1.2 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-242885 | 3145 | - | F | Correction to RedCap RRM test case 17.3.2.3.1 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-242887 | 3147 | - | F | Correction to RedCap RRM test case 17.4.3.1 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-242892 | 3152 | - | F | Correction to RedCap RRM test case 17.5.4.1.1 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-242893 | 3153 | - | F | Correction to RedCap RRM test case 17.5.4.2.1 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-242895 | 3155 | - | F | Correction to RedCap RRM test case 17.5.5.2.1 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-242904 | 3164 | - | F | Correction to Annex E for RedCap RRM test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-242979 | 3167 | - | F | Editorial correction of references to TS 38.101-1 in section 7 | 18.3.0 |
| 2024-06 | RAN#104 | R5-242980 | 3168 | - | F | Editorial correction of references to TS 38.101-1 in section 6.7 | 18.3.0 |
| 2024-06 | RAN#104 | R5-242981 | 3169 | - | F | Editorial correction of references to TS 38.101-1 in section 6.6 | 18.3.0 |
| 2024-06 | RAN#104 | R5-242982 | 3170 | - | F | Editorial correction of references to TS 38.101-1 in section 5 | 18.3.0 |
| 2024-06 | RAN#104 | R5-242983 | 3171 | - | F | Editorial correction of references to TS 38.101-1 in section 4 | 18.3.0 |
| 2024-06 | RAN#104 | R5-242986 | 3173 | - | F | Editorial correction to title of R17 RedCap test case 16.6.4.7 | 18.3.0 |
| 2024-06 | RAN#104 | R5-242987 | 3174 | - | F | Test procedure correction for R17 RedCap NCD-SSB test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-242988 | 3175 | - | F | Addition of SA FR1 NR-U RSSI minimum conformance requirements | 18.3.0 |
| 2024-06 | RAN#104 | R5-242989 | 3176 | - | F | Addition of SA FR1 NR-U RSSI measurement accuracy test case 11.6.5.3 | 18.3.0 |
| 2024-06 | RAN#104 | R5-242990 | 3177 | - | F | Addition of SA FR1 NR-U SSB based L1-RSRP measurement test case 11.6.4.1 | 18.3.0 |
| 2024-06 | RAN#104 | R5-242991 | 3178 | - | F | Addition of SA FR1 NR-U channel occupancy inter-frequency test case 11.6.6.3 | 18.3.0 |
| 2024-06 | RAN#104 | R5-242992 | 3179 | - | F | Correction to frequencies table for RRM FR2 SA test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-242993 | 3180 | - | F | Corrections to message contents for SA FR2 event triggered intra-frequency test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-242994 | 3181 | - | F | Corrections to message contents for SA FR2 timing test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-243019 | 3183 | - | F | Correction to K1 in 4.6.4.3 and 6.6.4.3 for FDD and TDD SCS 30 kHz | 18.3.0 |
| 2024-06 | RAN#104 | R5-243020 | 3184 | - | F | Correction to DCI trigger in 6.6.4.3 for FDD and TDD SCS 15 kHz | 18.3.0 |
| 2024-06 | RAN#104 | R5-243025 | 3186 | - | F | Correction to SMTC settings in 16.6.2.9 and 16.6.2.10 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243095 | 3187 | - | F | Removal of Editors note for applicability of RedCap SS-RSRP test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-243097 | 3189 | - | F | Addition of SA FR1 NR-U channel occupancy intra-frequency test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-243235 | 3192 | - | F | Addition of Clause 4.5.6.4.0 minimum conformance requirements for simultaneous RRC-based active BWP switch on multiple CCs | 18.3.0 |
| 2024-06 | RAN#104 | R5-243236 | 3193 | - | F | Addition of Clause 4.6.8.0 minimum conformance requirements for CSI-RS based intra-frequency measurement | 18.3.0 |
| 2024-06 | RAN#104 | R5-243237 | 3194 | - | F | Addition of Clause 4.6.9.0 minimum conformance requirements for CSI-RS based inter-frequency measurement | 18.3.0 |
| 2024-06 | RAN#104 | R5-243238 | 3195 | - | F | Addition of Clause 5.5.3.0 minimum conformance requirements for SCell activation and deactivation delay | 18.3.0 |
| 2024-06 | RAN#104 | R5-243239 | 3196 | - | F | Addition of Clause 5.5.6.5.0 minimum conformance requirements for simultaneous RRC-based active BWP switch on multiple CC | 18.3.0 |
| 2024-06 | RAN#104 | R5-243329 | 3213 | - | F | Addition of EN-DC FR1 NR-U Random Access and Handover test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-243341 | 3216 | - | F | Corrections to NR FR1 RedCap E-UTRA cell re-selection TCs 16.1.2.1, 16.1.2.3 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243342 | 3217 | - | F | Editorial correction in Redcap 16.6.2.1 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243343 | 3218 | - | F | Correction to FR1 RedCap E-UTRA - NR inter - RAT Handover TC 18.2.1.1 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243344 | 3219 | - | F | Editorial correction in Annex E for FR1 RedCap TCs 16.6.2.3, 16.6.2.4 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243359 | 3220 | - | F | Addition of NTN Timing Advance test 14.3.2.1 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243360 | 3221 | - | F | Addition of NTN distance-based CHO tests 14.2.1.5 and 14.2.1.6 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243361 | 3222 | - | F | Addition of NTN inter-frequency Measurement Initiation Cell Reselection tests 14.1.7 and 14.1.8 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243362 | 3223 | - | F | Addition of NTN L1-RSRP Measurement Accuracy tests 14.6.4.1 and 14.6.4.2 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243363 | 3224 | - | F | Update to NTN general sections | 18.3.0 |
| 2024-06 | RAN#104 | R5-243364 | 3225 | - | F | Update to NR-U Cases 10.4.1.1 and 10.4.1.4 including TTs | 18.3.0 |
| 2024-06 | RAN#104 | R5-243365 | 3226 | - | F | Update to NR-U Timing Accuracy Test 10.2.1.1 including TTs | 18.3.0 |
| 2024-06 | RAN#104 | R5-243366 | 3227 | - | F | Update to NR-U Timing Advance Test 10.2.2.1 including TTs | 18.3.0 |
| 2024-06 | RAN#104 | R5-243367 | 3228 | - | F | Update to NR-U L1-RSRP measurement reporting tests 10.4.3.x including TTs | 18.3.0 |
| 2024-06 | RAN#104 | R5-243368 | 3229 | - | F | Update to SCell activation and deactivation tests 10.3.3.x including TTs | 18.3.0 |
| 2024-06 | RAN#104 | R5-243369 | 3230 | - | F | Addition of NR-U EN-DC PSCell addition and release delay test case including TTs | 18.3.0 |
| 2024-06 | RAN#104 | R5-243370 | 3231 | - | F | Addition of NR-U SA FR1 UE transmit timing and timing advance test cases including TTs | 18.3.0 |
| 2024-06 | RAN#104 | R5-243371 | 3232 | - | F | Addition of NR-U SA FR1 SCell activation and deactivation test cases including TTs | 18.3.0 |
| 2024-06 | RAN#104 | R5-243373 | 3234 | - | F | Removal of interruption during SCell operation NR-U tests | 18.3.0 |
| 2024-06 | RAN#104 | R5-243377 | 3238 | - | F | Addition of testing methodology for Shared Spectrum | 18.3.0 |
| 2024-06 | RAN#104 | R5-243378 | 3239 | - | F | Clean up of pending items in section 10 NR-U tests | 18.3.0 |
| 2024-06 | RAN#104 | R5-243379 | 3240 | - | F | Update to Annex H for NR-U | 18.3.0 |
| 2024-06 | RAN#104 | R5-243380 | 3241 | - | F | Clean up of pending items in section 11 NR-U tests | 18.3.0 |
| 2024-06 | RAN#104 | R5-243381 | 3242 | - | F | Editorial update to Annex D for NR-U | 18.3.0 |
| 2024-06 | RAN#104 | R5-243382 | 3243 | - | F | Update to OCNG modelling for NR-U | 18.3.0 |
| 2024-06 | RAN#104 | R5-243384 | 3245 | - | F | Correction to stationary relaxed measurement criterion tests | 18.3.0 |
| 2024-06 | RAN#104 | R5-243388 | 3249 | - | F | Editorial correction to section 18 title | 18.3.0 |
| 2024-06 | RAN#104 | R5-243391 | 3252 | - | F | Correction to gap-based measurement tests | 18.3.0 |
| 2024-06 | RAN#104 | R5-243393 | 3254 | - | F | Correction to Handover tests | 18.3.0 |
| 2024-06 | RAN#104 | R5-243703 | 3190 | 1 | F | Update to Unified TCI state test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-243704 | 3191 | 1 | F | Correcting propagation AWGN in test cases without Noc | 18.3.0 |
| 2024-06 | RAN#104 | R5-243705 | 3119 | 1 | F | Adding TRP specific Minimum conformance requirements | 18.3.0 |
| 2024-06 | RAN#104 | R5-243706 | 3105 | 1 | F | Correction of FR2 interruptions at SRS carrier based switching test case 5.5.2.7 and 5.5.2.8 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243707 | 3121 | 1 | F | Addition of new test case 5.6.7.1 EN-DC event triggered reporting test without gap under non-DRX | 18.3.0 |
| 2024-06 | RAN#104 | R5-243708 | 3122 | 1 | F | Addition of new test case 5.6.8.1 EN-DC event triggered reporting tests for NR FR2 cell when DRX is used | 18.3.0 |
| 2024-06 | RAN#104 | R5-243709 | 3123 | 1 | F | Addition of new test case 5.7.7.1 EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | 18.3.0 |
| 2024-06 | RAN#104 | R5-243710 | 3124 | 1 | F | Addition of new test case 5.7.7.2 EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | 18.3.0 |
| 2024-06 | RAN#104 | R5-243711 | 3250 | 1 | F | Update to gap-based measurement reporting tests for HST | 18.3.0 |
| 2024-06 | RAN#104 | R5-243712 | 3214 | 1 | F | Correction to RRM RedCap TC 16.5.1.7 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243713 | 3172 | 1 | F | Corrections to R17 RedCap 18.3.1.x test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-243735 | 3236 | 1 | F | Update to SA measurement accuracy NR-U tests including TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-243744 | 3097 | 1 | F | Correction to 7.6.15.3 FR2 SA event triggered reporting tests with concurrent gaps including TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-243748 | 3115 | 1 | F | Correction of NR_U SA L1-RSRP measurements test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-243749 | 3099 | 1 | F | Completion of NR-U redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA test case including test tolerance | 18.3.0 |
| 2024-06 | RAN#104 | R5-243751 | 3098 | 1 | F | Completion of NR-U redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA test case including test tolerance | 18.3.0 |
| 2024-06 | RAN#104 | R5-243754 | 3103 | 1 | F | Correction of interruptions at NR SRS carrier based switching test case 4.5.2.8 including Test Tolerance | 18.3.0 |
| 2024-06 | RAN#104 | R5-243755 | 3104 | 1 | F | Correction of interruptions at E-UTRA SRS carrier based switching test case 4.5.2.9 including Test Tolerance | 18.3.0 |
| 2024-06 | RAN#104 | R5-243756 | 3106 | 1 | F | Correction of SA FR1 interruptions at NR SRS carrier based switching test case 6.5.2.2 including Test Tolerance | 18.3.0 |
| 2024-06 | RAN#104 | R5-243757 | 3110 | 1 | F | Addition of FR2 SA event triggered reporting test without gap under non-DRX test case 7.6.2.10 including Test Tolerance | 18.3.0 |
| 2024-06 | RAN#104 | R5-243758 | 3111 | 1 | F | Addition of FR2 SA event triggered reporting test without gap under DRX test case 7.6.2.11 including Test Tolerance | 18.3.0 |
| 2024-06 | RAN#104 | R5-243774 | 3146 | 1 | F | Correction to RedCap RRM test case 17.4.1.1 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-243775 | 3154 | 1 | F | Correction to RedCap RRM test case 17.5.5.1.1 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-243776 | 3156 | 1 | F | Correction to RedCap RRM test case 17.7.1.1 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-243777 | 3157 | 1 | F | Correction to RedCap RRM test case 17.7.1.2 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-243778 | 3158 | 1 | F | Correction to RedCap RRM test case 17.7.2.1 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-243779 | 3159 | 1 | F | Correction to RedCap RRM test case 17.7.2.2 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-243780 | 3160 | 1 | F | Correction to RedCap RRM test case 17.7.3.1 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-243781 | 3161 | 1 | F | Correction to RedCap RRM test case 17.7.3.2 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-243782 | 3162 | 1 | F | Correction to RedCap RRM test case 17.7.4.1 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-243783 | 3163 | 1 | F | Addition of RedCap RRM test case 17.7.4.2 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-243815 | 3197 | 1 | F | Addition of message contents to test case 4.7.9.1 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243842 | 3198 | 1 | F | Addition of message contents to test case 4.7.9.2 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243843 | 3199 | 1 | F | Addition of message contents to test case 4.7.10.1 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243844 | 3200 | 1 | F | Addition of message contents to test case 4.7.10.2 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243845 | 3201 | 1 | F | Addition of message contents to test case 5.7.8.1 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243846 | 3202 | 1 | F | Addition of message contents to test case 5.7.8.2 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243847 | 3203 | 1 | F | Addition of message contents to test case 5.7.9.1 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243848 | 3204 | 1 | F | Addition of message contents to test case 5.7.9.2 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243849 | 3205 | 1 | F | Addition of message contents to test case 6.7.11.1 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243850 | 3206 | 1 | F | Addition of message contents to test case 6.7.11.2 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243851 | 3207 | 1 | F | Addition of message contents to test case 6.7.12.1 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243852 | 3208 | 1 | F | Addition of message contents to test case 6.7.12.2 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243853 | 3209 | 1 | F | Addition of message contents to test case 7.7.8.1 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243854 | 3210 | 1 | F | Addition of message contents to test case 7.7.8.2 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243855 | 3211 | 1 | F | Addition of message contents to test case 7.7.9.1 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243856 | 3212 | 1 | F | Addition of message contents to test case 7.7.9.2 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243880 | 3118 | 1 | F | Addition test case for Inter-cell SSB based L1-RSRP Reporting | 18.3.0 |
| 2024-06 | RAN#104 | R5-243892 | 3237 | 1 | F | Update to NSA event-triggered measurement NR-U tests including TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-243901 | 3251 | 1 | F | Update to Rel.17 HST CA measurement reporting tests | 18.3.0 |
| 2024-06 | RAN#104 | R5-243902 | 3253 | 1 | F | Correction to SUL test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-243903 | 3247 | 1 | F | Correction to re-establishment test cases for RedCap | 18.3.0 |
| 2024-06 | RAN#104 | R5-243904 | 3244 | 1 | F | Correction to gap-based measurement tests for RedCap | 18.3.0 |
| 2024-06 | RAN#104 | R5-243909 | 3215 | 1 | F | Correction to FR1 NSA Event Trigger TC 4.6.1.8 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243916 | 3134 | 1 | F | Correction to R15 RACH RRM test cases and RMCs | 18.3.0 |
| 2024-06 | RAN#104 | R5-243917 | 3135 | 1 | F | Correction to R15 reestablishment RRM test case 6.3.2.1.1 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243918 | 3139 | 1 | F | Correction to RedCap RRM test case 16.1.2.5 and 16.1.2.6 | 18.3.0 |
| 2024-06 | RAN#104 | R5-243919 | 3255 | 1 | F | Correction to RedCap inter-RAT measurement test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-243994 | 3233 | 2 | F | Update to Annex E and F for NR-U RRM tests | 18.3.0 |
| 2024-06 | RAN#104 | R5-243995 | 3235 | 2 | F | Update to NSA measurement accuracy NR-U tests including TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-244001 | 3127 | 1 | F | Correction to SDT RRM test case 6.2.1 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-244002 | 3125 | 1 | F | Addition of MU and test tolerance in Annex F for Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | 18.3.0 |
| 2024-06 | RAN#104 | R5-244003 | 3166 | 2 | F | Update of Rel-16 RRM test case EN-DC CSI-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | 18.3.0 |
| 2024-06 | RAN#104 | R5-244005 | 3092 | 1 | F | Addition of TT for MG enhancements TC 6.6.18.3 | 18.3.0 |
| 2024-06 | RAN#104 | R5-244006 | 3093 | 2 | F | Addition of TT for MG enhancements TC 6.6.18.4 | 18.3.0 |
| 2024-06 | RAN#104 | R5-244012 | 3165 | 2 | F | Correction to Annex F for RedCap RRM test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-244013 | 3095 | 2 | F | Update test requirement for Redcap inter-frequency test cases | 18.3.0 |
| 2024-06 | RAN#104 | R5-244014 | 3140 | 1 | F | Correction to RedCap RRM test case 16.2.1.1 with TT | 18.3.0 |
| 2024-06 | RAN#104 | R5-244015 | 3141 | 1 | F | Correction to RedCap RRM test case 16.2.1.2 with TT | 18.3.0 |
| 2024-09 | RAN#105 | R5-244110 | 3259 | - | F | Correction to RedCap RRM - 16.6.1.1 test case | 18.4.0 |
| 2024-09 | RAN#105 | R5-244151 | 3264 | - | F | Addition of minimum requirements for FR1 LTM intra L1-RSRP measurement | 18.4.0 |
| 2024-09 | RAN#105 | R5-244153 | 3266 | - | F | Addition of minimum requirements for FR1 LTM inter L1-RSRP measurement with GAP | 18.4.0 |
| 2024-09 | RAN#105 | R5-244155 | 3268 | - | F | Addition of minimum requirements for FR1 LTM inter L1-RSRP measurement without GAP | 18.4.0 |
| 2024-09 | RAN#105 | R5-244157 | 3270 | - | F | Addition of minimum requirements for FR1 LTM inter L1-RSRP measurement accuracy | 18.4.0 |
| 2024-09 | RAN#105 | R5-244160 | 3273 | - | F | Addition of new band groups for FR1 LTM L1-RSRP measurement accuracy test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-244161 | 3274 | - | F | Editorial correction of RRC IE used in ReportConfigNR-DEFAULT for LTM L1-RSRP measurement test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-244285 | 3275 | - | F | Addition of minimum requirements for 6.3.4.0 - FR1 LTM PCell switch | 18.4.0 |
| 2024-09 | RAN#105 | R5-244286 | 3276 | - | F | Addition of new test case 6.3.4.1 for FR1 LTM intra-f Rach-based PCell switch | 18.4.0 |
| 2024-09 | RAN#105 | R5-244287 | 3277 | - | F | Addition of new test case 6.3.4.2 for FR1 LTM inter-f Rach-based PCell switch | 18.4.0 |
| 2024-09 | RAN#105 | R5-244288 | 3278 | - | F | Correction to table E.4-1 for LTM FR1 TCs | 18.4.0 |
| 2024-09 | RAN#105 | R5-244292 | 3281 | - | F | Addition of LTM Candidate TCI State Configurations to align with core specs | 18.4.0 |
| 2024-09 | RAN#105 | R5-244293 | 3282 | - | F | Editorial correction of Redcap inter-RAT CGI test case 16.6.5.4 | 18.4.0 |
| 2024-09 | RAN#105 | R5-244318 | 3287 | - | F | Correction of Redcap RRM-16.7.4.0.3 and 16.7.4.0.4 CSI-RS L1-RSRP accuracy requirements | 18.4.0 |
| 2024-09 | RAN#105 | R5-244334 | 3288 | - | F | Editorial correction of 1Rx RedCap RRM - L1-RSRP measurement for beam reporting | 18.4.0 |
| 2024-09 | RAN#105 | R5-244349 | 3289 | - | F | Correction on test requirements in Redcap RLM test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-244350 | 3290 | - | F | Addition of new sub-clause 6.3.2.4.0 for LTM | 18.4.0 |
| 2024-09 | RAN#105 | R5-244351 | 3291 | - | F | Addition of new test case 6.3.2.4.1 for LTM | 18.4.0 |
| 2024-09 | RAN#105 | R5-244352 | 3292 | - | F | Addition of new test case 6.3.2.4.2 for LTM | 18.4.0 |
| 2024-09 | RAN#105 | R5-244353 | 3293 | - | F | Addition of new test case 6.3.2.4.3 for LTM | 18.4.0 |
| 2024-09 | RAN#105 | R5-244361 | 3294 | - | F | Editorial correction to clause 4.5.3.0 and TC 4.5.2.3 | 18.4.0 |
| 2024-09 | RAN#105 | R5-244390 | 3295 | - | F | Introduction of bands n31 and n72 to NR band group for FR1 | 18.4.0 |
| 2024-09 | RAN#105 | R5-244549 | 3296 | - | F | Addition of Rel-18 inter-frequency cell reselection HST FR2 test case | 18.4.0 |
| 2024-09 | RAN#105 | R5-244550 | 3297 | - | F | Addition of Rel-18 HST FR2 active TCI state switch delay test case | 18.4.0 |
| 2024-09 | RAN#105 | R5-244556 | 3299 | - | F | Addition of FR1 intra-frequency handover for less than 5MHz test case | 18.4.0 |
| 2024-09 | RAN#105 | R5-244557 | 3300 | - | F | Addition of RLM OOS in DRX mode for UE operating on a cell with less than 5MHz bandwidth test case | 18.4.0 |
| 2024-09 | RAN#105 | R5-244558 | 3301 | - | F | Addition of RLM OOS in non-DRX mode for UE operating on a cell with less than 5MHz bandwidth test case | 18.4.0 |
| 2024-09 | RAN#105 | R5-244559 | 3302 | - | F | Addition of less than 5 MHz RMC and CORESET RMSI scheduling | 18.4.0 |
| 2024-09 | RAN#105 | R5-244562 | 3303 | - | F | Addition of SA FR2 UE Transmit Timing with 2 TAGs for multi-DCI multi-TRP operation test case | 18.4.0 |
| 2024-09 | RAN#105 | R5-244565 | 3306 | - | F | Addition of Test Case: TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX | 18.4.0 |
| 2024-09 | RAN#105 | R5-244605 | 3310 | - | F | Correction to MCE RRM test case 6.5.7D.1 with TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-244606 | 3311 | - | F | Correction to MCE RRM test case 6.5.7D.2 with TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-244607 | 3312 | - | F | Correction to MCE RRM test case 6.5.7D.3 with TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-244610 | 3315 | - | F | Correction to Annex H for Tx switch RRM test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-244613 | 3317 | - | F | Correction to EN-DC BFR test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-244614 | 3318 | - | F | Correction to NR SA BFR test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-244615 | 3319 | - | F | Correction to NR SA CSI-RS based L1-RSRP test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-244617 | 3321 | - | F | Correction to NR SA RRM test case 7.3.2.1.1 | 18.4.0 |
| 2024-09 | RAN#105 | R5-244619 | 3322 | - | F | Correction to R16 Tx switch RRM test case 4.5.8.1 with TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-244620 | 3323 | - | F | Correction to R16 Tx switch RRM test case 6.5.7.1 with TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-244621 | 3324 | - | F | Correction to R16 Tx switch RRM test case 6.5.7.2 with TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-244622 | 3325 | - | F | Correction to Annex F for R16 Tx switch RRM test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-244624 | 3326 | - | F | Correction to R17 Tx switch RRM test case 6.5.7A.1 with TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-244625 | 3327 | - | F | Correction to R17 Tx switch RRM test case 6.5.7A.2 with TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-244626 | 3328 | - | F | Correction to R17 Tx switch RRM test case 6.5.7B.1 | 18.4.0 |
| 2024-09 | RAN#105 | R5-244627 | 3329 | - | F | Correction to R17 Tx switch RRM test case 6.5.7B.2 with TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-244628 | 3330 | - | F | Correction to R17 Tx switch RRM test case 6.5.7C.1 | 18.4.0 |
| 2024-09 | RAN#105 | R5-244629 | 3331 | - | F | Correction to R17 Tx switch RRM test case 6.5.7C.2 with TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-244630 | 3332 | - | F | Correction to Annex F for R17 Tx switch RRM test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-244632 | 3333 | - | F | Correction to RedCap RRM test case 16.1.1.7 | 18.4.0 |
| 2024-09 | RAN#105 | R5-244633 | 3334 | - | F | Correction to RedCap BFR test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-244634 | 3335 | - | F | Correction to RedCap RRM test case 17.5.1.3 with TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-244635 | 3336 | - | F | Correction to RedCap RRM test case 17.5.1.5 with TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-244636 | 3337 | - | F | Correction to RedCap RRM test case 17.5.1.6 with TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-244637 | 3338 | - | F | Correction to RedCap RRM test case 17.5.1.7 with TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-244638 | 3339 | - | F | Correction to RedCap RRM test case 17.5.1.8 with TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-244639 | 3340 | - | F | Correction to RedCap RRM test case 17.5.3.1.1 with TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-244640 | 3341 | - | F | Correction to RedCap RRM test case 17.5.3.2.1 with TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-244759 | 3343 | - | F | Cell Configuration Mapping for RRM Enhancements Test Cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-244769 | 3351 | - | F | Correction of Interruptions at NR SRS Carrier Based Switching Test Case 4.5.2.8 | 18.4.0 |
| 2024-09 | RAN#105 | R5-244770 | 3352 | - | F | Correction of Interruptions at E-UTRA SRS Carrier Based Switching Test Case 4.5.2.9 | 18.4.0 |
| 2024-09 | RAN#105 | R5-244771 | 3353 | - | F | Correction of SA FR1 Interruptions at NR SRS Carrier Based Switching Test Case 6.5.2.2 | 18.4.0 |
| 2024-09 | RAN#105 | R5-244778 | 3357 | - | F | Addition of MU and test tolerance in Annex F for Rel-16 RRM EN-DC event triggered reporting test for NR FR2 cell when DRX is used | 18.4.0 |
| 2024-09 | RAN#105 | R5-244781 | 3359 | - | F | Addition of MU and test tolerance in Annex F for Rel-16 RRM EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | 18.4.0 |
| 2024-09 | RAN#105 | R5-244784 | 3361 | - | F | Addition of MU and test tolerance in Annex F for Rel-16 RRM EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | 18.4.0 |
| 2024-09 | RAN#105 | R5-244810 | 3362 | - | F | Update of FR2 Random Access test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-244832 | 3364 | - | F | Correction to message exception in 16.5.3.1.1 and 16.5.3.2.1 | 18.4.0 |
| 2024-09 | RAN#105 | R5-244835 | 3367 | - | F | Addition of clarification notes in 16.7.2.1 and 16.7.2.2 | 18.4.0 |
| 2024-09 | RAN#105 | R5-244836 | 3368 | - | F | Correction to test requirement in 16.7.4.1.1 | 18.4.0 |
| 2024-09 | RAN#105 | R5-244840 | 3372 | - | F | Editorial correction to 6.7.1.2.2 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245036 | 3374 | - | F | Typo correction to A3 offset in TC 7.6.1.2 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245040 | 3378 | - | F | Corrections to msg content of BFD-LR test cases 4.5.5.1,4.5.5.2,5.5.5.1,5.5.5.2,6.5.5.1 and 6.5.5.2 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245042 | 3380 | - | F | Update to NR-U UE random access tests with PSCell under DL CCA including Test Tolerance | 18.4.0 |
| 2024-09 | RAN#105 | R5-245044 | 3382 | - | F | Update to NR-U UE FR1 SA Intra-frequency measurement accuracy on SCC TC 11.6.1.2 including Test Tolerance | 18.4.0 |
| 2024-09 | RAN#105 | R5-245074 | 3384 | - | F | Editorial correction of 3GPP style issue | 18.4.0 |
| 2024-09 | RAN#105 | R5-245123 | 3386 | - | F | Addition of RRC message content for Further NR mobility enhancements | 18.4.0 |
| 2024-09 | RAN#105 | R5-245131 | 3388 | - | F | Correction for new reference SSB configuration | 18.4.0 |
| 2024-09 | RAN#105 | R5-245132 | 3389 | - | F | Correction of test configurations and test cases for FeMIMO | 18.4.0 |
| 2024-09 | RAN#105 | R5-245242 | 3395 | - | F | Corrections to NRU measurement accuracy test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-245243 | 3396 | - | F | Addition of new test cases for SS-RSRP measurement accuracy for Satellite Access | 18.4.0 |
| 2024-09 | RAN#105 | R5-245244 | 3397 | - | F | Addition of new test cases for SS-RSRQ measurement accuracy for Satellite Access | 18.4.0 |
| 2024-09 | RAN#105 | R5-245245 | 3398 | - | F | Editorial corrections to Table 14.0.2-1 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245248 | 3401 | - | F | Editorial corrections to Initial Conditions for RRM FR2 SA test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-245252 | 3404 | - | F | Correction of MU values for RRM FR2 PC1 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245267 | 3405 | - | F | Correction of SS-RSRP RedCap test case 16.7.1.4.2 including Test Tolerance | 18.4.0 |
| 2024-09 | RAN#105 | R5-245268 | 3406 | - | F | Correction of SS-RSRP RedCap test case 16.7.1.4.1 including Test Tolerance | 18.4.0 |
| 2024-09 | RAN#105 | R5-245269 | 3407 | - | F | Correction of SS-RSRP RedCap test case 16.7.1.3.2 including Test Tolerance | 18.4.0 |
| 2024-09 | RAN#105 | R5-245272 | 3410 | - | F | Correction of FR2 Event Triggered test cases including TT for PC1 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245273 | 3411 | - | F | Correction of FR2 Timing Advance test cases including TT for PC1 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245274 | 3412 | - | F | Correction of FR2 Uplink Timing test cases including TT for PC1 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245278 | 3416 | - | F | Annex E.14 correction for new RedCap test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-245279 | 3417 | - | F | Annex F correction for RRM FR2 PC1 test cases including Test Tolerance | 18.4.0 |
| 2024-09 | RAN#105 | R5-245281 | 3418 | - | F | Correction to Editors Notes in Annex B | 18.4.0 |
| 2024-09 | RAN#105 | R5-245316 | 3419 | - | F | Update in 16.7.1.1.1 to align Noc level to TT analysis | 18.4.0 |
| 2024-09 | RAN#105 | R5-245318 | 3420 | - | F | Update in 16.7.2.2 to align Noc level | 18.4.0 |
| 2024-09 | RAN#105 | R5-245319 | 3421 | - | F | Annex F correction for RedCap test cases including Test Tolerance | 18.4.0 |
| 2024-09 | RAN#105 | R5-245333 | 3422 | - | F | Update on 4.6.2.9 DRX configuration | 18.4.0 |
| 2024-09 | RAN#105 | R5-245337 | 3423 | - | F | Update on pdcch-DMRS-SCID for NSA FR1 BFR test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-245338 | 3424 | - | F | Update on pdcch-DMRS-SCID for NSA FR2 BFR test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-245346 | 3427 | - | F | Addition of Annex E for NR-NTN test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-245347 | 3428 | - | F | Correction to RRM test 6.6.2.9 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245349 | 3430 | - | F | Update to SDT test 6.2.1 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245351 | 3432 | - | F | Core alignment for section 11 tests | 18.4.0 |
| 2024-09 | RAN#105 | R5-245352 | 3433 | - | F | Addition of chapter 13 tests on a secondary carrier with CCA | 18.4.0 |
| 2024-09 | RAN#105 | R5-245353 | 3434 | - | F | Update to Annex E and F for NR-U tests | 18.4.0 |
| 2024-09 | RAN#105 | R5-245354 | 3435 | - | F | Addition of NR-U SA FR1 cell reselection test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-245355 | 3436 | - | F | Addition of NR-U SA FR1 Handover test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-245356 | 3437 | - | F | Addition of NR-U SA FR1 RRC re-establishment test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-245357 | 3438 | - | F | Addition of chapter 12 inter-RAT tests on target carrier with CCA | 18.4.0 |
| 2024-09 | RAN#105 | R5-245358 | 3439 | - | F | Update to PSCell HO test 10.1.2 including TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-245359 | 3440 | - | F | Addition of NR-U SA FR1 Inter-RAT cell reselection test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-245360 | 3441 | - | F | Editorial corrections to Annex D.7 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245361 | 3442 | - | F | Core alignment for gap-based FR2 inter-RAT measurement tests | 18.4.0 |
| 2024-09 | RAN#105 | R5-245362 | 3443 | - | F | Update to SUL tests including TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-245365 | 3446 | - | F | Update to RedCap SA FR2 SSB BFR test cases 17.5.2.1 and 17.5.2.2 including TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-245368 | 3449 | - | F | Update to RedCap 2Rx event-triggered test cases 16.6.3.2 and 16.6.3.4 including TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-245369 | 3450 | - | F | Update to Annex F for RedCap event-triggered test cases 16.6.3.x | 18.4.0 |
| 2024-09 | RAN#105 | R5-245370 | 3451 | - | F | Correction to RedCap BFR 1Rx test cases 16.5.2.1 and 16.5.2.3 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245727 | 3470 | 1 | F | Update of test titles for RedCap UE | 18.4.0 |
| 2024-09 | RAN#105 | R5-245731 | 3308 | 2 | F | Correction to CSI-RS L3 measurement RRM test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-245732 | 3457 | 2 | F | Update to Clause 5.7.8.1 ENDC and Clause 7.7.8.1 SA intra-frequency CSI-RSRQ measurement accuracy | 18.4.0 |
| 2024-09 | RAN#105 | R5-245733 | 3458 | 2 | F | Update to Clause 5.7.8.2 ENDC and Clause 7.7.8.2 SA inter-frequency CSI-RSRQ measurement accuracy | 18.4.0 |
| 2024-09 | RAN#105 | R5-245734 | 3459 | 2 | F | Update to Clause 5.7.9.1 ENDC and Clause 7.7.9.1 SA intra-frequency CSI-SINR measurement accuracy | 18.4.0 |
| 2024-09 | RAN#105 | R5-245735 | 3460 | 2 | F | Update to Clause 5.7.9.2 ENDC and Clause 7.7.9.2 SA inter-frequency CSI-SINR measurement accuracy | 18.4.0 |
| 2024-09 | RAN#105 | R5-245742 | 3262 | 1 | F | Correction to FR2 RRM - 5.5.5.5 test case | 18.4.0 |
| 2024-09 | RAN#105 | R5-245743 | 3263 | 1 | F | Correction to FR2 RRM - 7.5.5.5 test case | 18.4.0 |
| 2024-09 | RAN#105 | R5-245859 | 3425 | 1 | F | Addition of NR-NTN RLM test cases 14.4.1.x | 18.4.0 |
| 2024-09 | RAN#105 | R5-245860 | 3426 | 1 | F | Addition of NR-NTN Re-establishment test cases 14.2.2.1.x | 18.4.0 |
| 2024-09 | RAN#105 | R5-245880 | 3307 | 1 | F | Addition of Test Case: NR SA FR1 SSB-less SCell Activation SSB-less inter-band CA based on A-TRS in non-DRX | 18.4.0 |
| 2024-09 | RAN#105 | R5-245881 | 3309 | 1 | F | Correction to Annex D for 8Rx capable UEs | 18.4.0 |
| 2024-09 | RAN#105 | R5-245882 | 3298 | 1 | F | Addition of Test Case: Cell reselection to FR1 intra-frequency NR for UE operating on a cell with less than 5MHz BW | 18.4.0 |
| 2024-09 | RAN#105 | R5-245883 | 3313 | 1 | F | Correction to Annex A for MCE RRM test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-245884 | 3305 | 1 | F | Annexure A changes for adding SA FR2 UL TCI state switch test case | 18.4.0 |
| 2024-09 | RAN#105 | R5-245885 | 3304 | 1 | F | Addition of NR SA FR2 PCell uplink TCI state switch for two known TCI states test case | 18.4.0 |
| 2024-09 | RAN#105 | R5-245886 | 3265 | 1 | F | Addition of new test case 6.6.26.1 for FR1 LTM intra L1-RSRP measurement | 18.4.0 |
| 2024-09 | RAN#105 | R5-245887 | 3267 | 1 | F | Addition of new test case 6.6.27.1 for FR1 LTM inter L1-RSRP measurement with gap | 18.4.0 |
| 2024-09 | RAN#105 | R5-245888 | 3269 | 1 | F | Addition of new test case 6.6.28.1 for FR1 LTM inter L1-RSRP measurement without gap | 18.4.0 |
| 2024-09 | RAN#105 | R5-245889 | 3271 | 1 | F | Addition of new test case 6.7.17.1.1 for FR1 LTM inter L1-RSRP absolute accuracy | 18.4.0 |
| 2024-09 | RAN#105 | R5-245890 | 3272 | 1 | F | Addition of new test case 6.7.17.1.2 for FR1 LTM inter L1-RSRP relative accuracy | 18.4.0 |
| 2024-09 | RAN#105 | R5-245891 | 3279 | 1 | F | Addition of new test case 6.3.4.3 for FR1 LTM intra-f Rach-less PCell switch | 18.4.0 |
| 2024-09 | RAN#105 | R5-245892 | 3280 | 1 | F | Update of FR1 and FR2 PRACH configurations to align with core specs | 18.4.0 |
| 2024-09 | RAN#105 | R5-245893 | 3452 | 1 | F | Add CSI-RS.RRM profiles to 38.533 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245894 | 3345 | 1 | F | Introduction of new test case 6.7.10.1.1 CSI-RSRP Intra-frequency Absolute Measurement Accuracy | 18.4.0 |
| 2024-09 | RAN#105 | R5-245895 | 3346 | 1 | F | Introduction of new test case 6.7.10.1.2 CSI-RSRP Intra-frequency Relative Measurement Accuracy | 18.4.0 |
| 2024-09 | RAN#105 | R5-245896 | 3347 | 1 | F | Introduction of new test case 6.7.10.2.1 CSI-RSRP Inter-frequency Absolute Measurement Accuracy | 18.4.0 |
| 2024-09 | RAN#105 | R5-245897 | 3348 | 1 | F | Introduction of new test case 6.7.10.2.2 CSI-RSRP Inter-frequency Relative Measurement Accuracy | 18.4.0 |
| 2024-09 | RAN#105 | R5-245898 | 3349 | 1 | F | Introduction of new test case 7.7.7.1.1 CSI-RSRP Intra-frequency Measurement Accuracy | 18.4.0 |
| 2024-09 | RAN#105 | R5-245899 | 3350 | 1 | F | Introduction of new test case 7.7.7.2.1 NR SA FR1-FR1 CSI-RSRP Measurement Accuracy | 18.4.0 |
| 2024-09 | RAN#105 | R5-245900 | 3344 | 1 | F | Introduction of new test case 5.5.7.1 Addition and Release Delay of NR PSCell | 18.4.0 |
| 2024-09 | RAN#105 | R5-245901 | 3373 | 1 | F | Addition of NR-U UE random access test with PCell under DL CCA | 18.4.0 |
| 2024-09 | RAN#105 | R5-245902 | 3316 | 1 | F | Correction to EN-DC RRM test case 4.5.7.1 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245903 | 3444 | 1 | F | Core alignment for r17 HST tests with DRX | 18.4.0 |
| 2024-09 | RAN#105 | R5-245906 | 3402 | 1 | F | Test parameters alignment FR2 Event-Triggered SA 7.6.1.1 to NSA 5.6.1.1 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245908 | 3366 | 1 | F | Correction to title of 16.7.1.x | 18.4.0 |
| 2024-09 | RAN#105 | R5-245909 | 3369 | 1 | F | Correction to test requirements in FR1 L1-RSRP absolute measurement for RedCap | 18.4.0 |
| 2024-09 | RAN#105 | R5-245910 | 3370 | 1 | F | Correction to test requirement in 16.3.1.1 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245911 | 3371 | 1 | F | Correction to test parameters in 16.3.2.3.1 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245912 | 3385 | 1 | F | Update to test case titles to match work plan | 18.4.0 |
| 2024-09 | RAN#105 | R5-245930 | 3356 | 1 | F | Update of Rel-16 RRM test case EN-DC event triggered reporting test when DRX is used | 18.4.0 |
| 2024-09 | RAN#105 | R5-245931 | 3358 | 1 | F | Update of Rel-16 RRM test case EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | 18.4.0 |
| 2024-09 | RAN#105 | R5-245932 | 3360 | 1 | F | Update of Rel-16 RRM test case EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | 18.4.0 |
| 2024-09 | RAN#105 | R5-245945 | 3314 | 1 | F | Correction to Annex F for MCE RRM test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-245952 | 3413 | 1 | F | Correction of Relative Measurement Accuracy test cases including TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-245953 | 3414 | 1 | F | Correction of Absolute Measurement Accuracy test cases including TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-245954 | 3409 | 1 | F | Correction of FR2 Intra-frequency measurement accuracy test cases including TT for PC1 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245955 | 3391 | 1 | F | Update the position of core requirements for test case 7.5.14 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245956 | 3342 | 1 | F | Correction to Annex F for RedCap RRM test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-245957 | 3283 | 1 | F | Correction to Redcap RRM TC 16.6.4.5 and 16.6.4.7_CSI-RS L1 RSRP 1Rx with TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-245958 | 3284 | 1 | F | Correction of Redcap CSI-RS based BFD and LR 16.5.2.5 and 16.5.2.7 test case with Test Tolerance | 18.4.0 |
| 2024-09 | RAN#105 | R5-245959 | 3285 | 1 | F | Correction of 16.7.4.3.1 Redcap CSI-RS based L1-RSRP accuracy for 1RX test case with Test Tolerance | 18.4.0 |
| 2024-09 | RAN#105 | R5-245960 | 3286 | 1 | F | Correction of 16.7.4.3.2 Redcap CSI-RS based L1-RSRP accuracy for 1RX test case with Test Tolerance | 18.4.0 |
| 2024-09 | RAN#105 | R5-245961 | 3377 | 1 | F | Correction to cell2 SS-RSRP for test 1 in TC 16.7.3.1 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245962 | 3383 | 1 | F | Update to NR2L FR1 measurement accuracy TC 16.7.5.2 | 18.4.0 |
| 2024-09 | RAN#105 | R5-245963 | 3408 | 1 | F | Correction of SS-RSRP RedCap test case 16.7.1.3.1 including Test Tolerance | 18.4.0 |
| 2024-09 | RAN#105 | R5-245995 | 3363 | 1 | F | Correction to CellGroupConfig for Non-Contention Based Random Access | 18.4.0 |
| 2024-09 | RAN#105 | R5-246015 | 3320 | 1 | F | Correction to EN-DC and NR SA CFRA test cases and RMCs | 18.4.0 |
| 2024-09 | RAN#105 | R5-246016 | 3260 | 1 | F | Correction to RedCap RRM - 16.5.4.1 and 16.5.4.2 test cases | 18.4.0 |
| 2024-09 | RAN#105 | R5-246017 | 3261 | 1 | F | Correction to RedCap RRM - 16.6.3.x test case | 18.4.0 |
| 2024-09 | RAN#105 | R5-246020 | 3379 | 1 | F | Update to NR-U ENDC FR1 Channel occupancy measurement on PSCell under DL CCA including Test Tolerance | 18.4.0 |
| 2024-09 | RAN#105 | R5-246021 | 3381 | 1 | F | Update to NR-U ENDC FR1 RSSI measurement accuracy on PSCell under DL CCA including Test Tolerance | 18.4.0 |
| 2024-09 | RAN#105 | R5-246022 | 3431 | 1 | F | Core alignment for section 10 tests | 18.4.0 |
| 2024-09 | RAN#105 | R5-246029 | 3390 | 1 | F | Correction to 6.6.4.6 NR SA FR1 Inter-cell SSB based L1-RSRP measurements on PCell in DRX | 18.4.0 |
| 2024-09 | RAN#105 | R5-246045 | 3445 | 1 | F | Update to r17 HST reselection test 6.1.1.8 | 18.4.0 |
| 2024-09 | RAN#105 | R5-246046 | 3375 | 1 | F | Correction to Antenna config in TC 16.6.2.9 | 18.4.0 |
| 2024-09 | RAN#105 | R5-246053 | 3448 | 1 | F | Update to RedCap 1Rx event-triggered test cases 16.6.3.1 and 16.6.3.3 including TT | 18.4.0 |
| 2024-09 | RAN#105 | R5-246064 | 3376 | 1 | F | Corrections to BFD-LR test cases 4.5.5.1,4.5.5.2,5.5.5.1,5.5.5.2,6.5.5.1 and 6.5.5.2 | 18.4.0 |
| 2024-12 | RAN#106 | R5-246172 | 3472 | - | F | Update TT value to LTM intra TC 6.3.2.4.1 and TC 6.3.2.4.3 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246182 | 3474 | - | F | Update LTM TC 6.6.26.1 including TTs | 18.5.0 |
| 2024-12 | RAN#106 | R5-246183 | 3475 | - | F | Update LTM TC 6.6.27.1 including TTs | 18.5.0 |
| 2024-12 | RAN#106 | R5-246184 | 3476 | - | F | Update LTM TC 6.6.28.1 including TTs | 18.5.0 |
| 2024-12 | RAN#106 | R5-246185 | 3477 | - | F | Update LTM TC 6.7.17.1.1 including TTs | 18.5.0 |
| 2024-12 | RAN#106 | R5-246186 | 3478 | - | F | Update Annex F for LTM L1-RSRP test cases including TTs | 18.5.0 |
| 2024-12 | RAN#106 | R5-246187 | 3479 | - | F | Update message contents and test applicability for LTM TC 6.6.26.1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246188 | 3480 | - | F | Update message contents and test applicability for LTM TC 6.6.27.1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246189 | 3481 | - | F | Update message contents and test applicability for LTM TC 6.6.28.1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246190 | 3482 | - | F | Update message contents and test applicability for LTM TC 6.7.17.1.1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246192 | 3483 | - | F | Update TT to Annex F for LTM PDCCH order TCs | 18.5.0 |
| 2024-12 | RAN#106 | R5-246196 | 3484 | - | F | Update LTM TC 6.3.4.3 including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246197 | 3485 | - | F | Update LTM TC 6.3.4.1 including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246198 | 3486 | - | F | Update LTM TC 6.3.4.2 including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246201 | 3487 | - | F | Correction to LTM test case 6.3.4.3 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246202 | 3488 | - | F | Update message contents for LTM TC 6.3.4.3 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246203 | 3489 | - | F | Core alignment for LTM RACHless TC 6.3.4.3 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246207 | 3491 | - | F | Update message contents for LTM TC 6.3.4.1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246208 | 3492 | - | F | Update message contents for LTM TC 6.3.4.2 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246209 | 3493 | - | F | Correction to RedCap RRM - SS-SINR accuracy requirement for 1RX | 18.5.0 |
| 2024-12 | RAN#106 | R5-246311 | 3494 | - | F | Removal of LTM L1-RSRP relative accuracy TC 6.7.17.1.2 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246320 | 3496 | - | F | Update test requirement for Redcap inter-frequency test cases | 18.5.0 |
| 2024-12 | RAN#106 | R5-246352 | 3497 | - | F | Correction to TC 6.6.18.3 and 6.6.18.4 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246393 | 3498 | - | F | Core spec alignment for LTM cell switch TC 6.3.4.1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246433 | 3500 | - | F | Update RRC message content for Further NR mobility enhancements | 18.5.0 |
| 2024-12 | RAN#106 | R5-246492 | 3503 | - | F | Core spec alignment update to LTM PDCCH order TCs | 18.5.0 |
| 2024-12 | RAN#106 | R5-246493 | 3504 | - | F | Update test applicability to clause 6.3.2.4 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246494 | 3505 | - | F | Update message content to clause 6.3.2.4 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246495 | 3506 | - | F | Correction to Inter-RAT NR cell re-selection for RedCap including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246719 | 3508 | - | F | Correction to FR1 NR SA re-establishment RRM test case 6.3.2.1.3 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246720 | 3509 | - | F | Correction to FR2 NR SA re-establishment RRM test case 7.3.2.1.3 with TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246722 | 3511 | - | F | Correction to Annex F for FR2 RRM test cases | 18.5.0 |
| 2024-12 | RAN#106 | R5-246727 | 3513 | - | F | Correction to FR1 NR SA 2-step RACH RRM test cases 6.3.2.2.3 & 6.3.2.2.4 with TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246732 | 3515 | - | F | Correction to RedCap inter-RAT reselection test cases 16.1.2.1 & 16.1.2.2 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246734 | 3517 | - | F | Correction to RedCap inter-frequency measurement test cases 16.6.2.5 & 16.6.2.6 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246735 | 3518 | - | F | Correction to FR1 RedCap inter-RAT measurement RRM test cases 16.6.3.X | 18.5.0 |
| 2024-12 | RAN#106 | R5-246737 | 3520 | - | F | Correction to FR2 RedCap re-establishment RRM test cases 17.3.2.1.3 with TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246738 | 3521 | - | F | Correction to FR2 RedCap RLM scheduling restriction test case 17.5.1.9 with TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246809 | 3522 | - | F | Test Tolerance , Statistical analysis update and core spec alignment to NR-U FR1 E-UTRAN-NR inter-RAT measurements under CCA | 18.5.0 |
| 2024-12 | RAN#106 | R5-246822 | 3529 | - | F | Update to propagation conditions for NR-NTN RLM test cases 14.4.1.x | 18.5.0 |
| 2024-12 | RAN#106 | R5-246823 | 3530 | - | F | Addition of Annex F for NR-NTN test cases | 18.5.0 |
| 2024-12 | RAN#106 | R5-246824 | 3531 | - | F | Update to NR-NTN event-triggered reporting test cases 14.5.1.x including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246825 | 3532 | - | F | Addition of NR-NTN cell reselection test cases 14.1.1 and 14.1.5 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246826 | 3533 | - | F | Addition of NR-NTN cell reselection enhanced requirement test case 14.1.6 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246827 | 3534 | - | F | Addition of NR-NTN idle mode relaxed measurement test cases 14.1.9 and 14.1.10 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246828 | 3535 | - | F | Update to EN-DC 2-step RACH NR-U tests | 18.5.0 |
| 2024-12 | RAN#106 | R5-246829 | 3536 | - | F | Addition of NR-U L1-RSRP tests 13.3.3.1 and 13.3.3.2 including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246830 | 3537 | - | F | Addition of NR-U inter-RAT cell reselection test 12.1.1.1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246831 | 3538 | - | F | Addition of NR-U inter-RAT handover test 12.2.1.1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246833 | 3540 | - | F | Update to NR-U inter-RAT tests 12.4.2.1, 12.4.2.2, 12.4.2.3 and 12.4.2.4 including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246834 | 3541 | - | F | Update to NR-U SS-RSRP test 13.4.1.1 including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246836 | 3543 | - | F | Update to NR-U SS-SINR test 13.4.3.1 including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246837 | 3544 | - | F | Update to NR-U L1-RSRP test 13.4.4.1 including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246838 | 3545 | - | F | Update to NR-U RSSI tests 13.4.5.1 and 13.4.5.2 including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246839 | 3546 | - | F | Update to NR-U channel occupancy tests 13.4.6.1 and 13.4.6.2 including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246840 | 3547 | - | F | Addition of NR-U SCell activation and deactivation tests 13.2.2.1, 13.2.2.2 and 13.2.2.3 including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246841 | 3548 | - | F | Addition of NR-U inter-frequency tests 13.3.2.3, 13.3.2.4, 13.3.2.5 and 13.3.2.6 including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246842 | 3549 | - | F | Addition of NR-U intra-frequency measurement tests 13.3.1.x including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246843 | 3550 | - | F | Update to Annex F for NR-U test cases | 18.5.0 |
| 2024-12 | RAN#106 | R5-246844 | 3551 | - | F | Update to 4-step RACH NR-U test cases including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246845 | 3552 | - | F | Update to 2-step RACH NR-U test cases including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246846 | 3553 | - | F | Update to Annex E for NR-U test cases | 18.5.0 |
| 2024-12 | RAN#106 | R5-246848 | 3555 | - | F | Update to r17 HST reselection test 6.1.1.8 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246849 | 3556 | - | F | Clarification on FR2 RLM test cases | 18.5.0 |
| 2024-12 | RAN#106 | R5-246881 | 3557 | - | F | Addition of minimum conformance requirements for MUSIM gap configured | 18.5.0 |
| 2024-12 | RAN#106 | R5-246882 | 3558 | - | F | Addition of NR SA FR1 event triggered reporting with MUSIM gap configured test case 6.6.1.9 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246883 | 3559 | - | F | Addition of NR SA FR1-FR1 event triggered reporting using MUSIM gap with lower and higher priority test case 6.6.2.13 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246884 | 3560 | - | F | Correction to Cell configuration mapping for MUSIM gap test cases | 18.5.0 |
| 2024-12 | RAN#106 | R5-246925 | 3561 | - | F | Addition of minimum requirements for NR FR1 NES-based CHO test cases | 18.5.0 |
| 2024-12 | RAN#106 | R5-246929 | 3565 | - | F | Correction to test applicability for 6.6.1.2 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246954 | 3566 | - | F | Addition of spatial relation configuration to 38.533 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246955 | 3567 | - | F | Addition of TC4.5.3.4 SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation deactivation command | 18.5.0 |
| 2024-12 | RAN#106 | R5-246956 | 3568 | - | F | Addition of TC4.5.6.4.1 E-UTRAN - NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs | 18.5.0 |
| 2024-12 | RAN#106 | R5-246957 | 3569 | - | F | Addition of TC5.5.3.6 Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246958 | 3570 | - | F | Addition of TC5.5.6.5.1 E-UTRAN - NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC | 18.5.0 |
| 2024-12 | RAN#106 | R5-246959 | 3571 | - | F | Addition of TC7.5.6.5.1 Active BWP switch on multiple SCells with non-DRX in SA | 18.5.0 |
| 2024-12 | RAN#106 | R5-246963 | 3574 | - | F | Correct a typo in the title of Annex A.2 | 18.5.0 |
| 2024-12 | RAN#106 | R5-246976 | 3575 | - | F | Completion of Test Case: Cell reselection to FR1 intra-frequency NR for UE operating on a cell with less than 5MHz BW including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246978 | 3577 | - | F | Addition of test tolerance for cell reselection and handover test cases when cell is operating in less than 5 MHz | 18.5.0 |
| 2024-12 | RAN#106 | R5-246983 | 3578 | - | F | Completion of SA FR2 UE Transmit Timing with 2 TAGs for multi-DCI multi-TRP operation test case including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-246985 | 3580 | - | F | Annex A changes for adding SA FR2 UL TCI state switch test case | 18.5.0 |
| 2024-12 | RAN#106 | R5-246986 | 3581 | - | F | Addition of test tolerance for test cases of NR SA FR2 MAC-CE based active uplink TCI state switch and UE Transmit Timing for multi-TRP operation | 18.5.0 |
| 2024-12 | RAN#106 | R5-246990 | 3583 | - | F | Update to TRS based SCell Activation of SSB-less SCell in inter-band CA in non-DRX test case | 18.5.0 |
| 2024-12 | RAN#106 | R5-247046 | 3586 | - | F | Addition of RRC message content for Further measurement gap enhancements | 18.5.0 |
| 2024-12 | RAN#106 | R5-247127 | 3587 | - | F | Correction in Message contents for FR2-FR2 event-triggered reporting in DRX test cases. | 18.5.0 |
| 2024-12 | RAN#106 | R5-247128 | 3588 | - | F | Correction A3-Offset for TC 7.6.1.1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247186 | 3589 | - | F | Correction to message contents for FR1 SA absolute measurement accuracy | 18.5.0 |
| 2024-12 | RAN#106 | R5-247187 | 3590 | - | F | Correction to message contents for BFD-LR 1Rx RedCap test cases | 18.5.0 |
| 2024-12 | RAN#106 | R5-247189 | 3592 | - | F | Update of test requirements for RedCap test cases in Annex F | 18.5.0 |
| 2024-12 | RAN#106 | R5-247194 | 3593 | - | F | Correction to TDD configuration in RedCap handover test cases | 18.5.0 |
| 2024-12 | RAN#106 | R5-247195 | 3594 | - | F | Correction to CORESET Reference Channel in 16.3.2.3.1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247197 | 3596 | - | F | Correction to general test parameters of active BWP switch delay for RedCap | 18.5.0 |
| 2024-12 | RAN#106 | R5-247198 | 3597 | - | F | Correction to Active BWP Dedicated CORESET RMC in 16.6.1.7 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247264 | 3598 | - | F | Introduction of band n254 to NR band group for FR1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247284 | 3599 | - | F | Addition of new test cases for SSB based L1-RSRP measurement for Satellite Access | 18.5.0 |
| 2024-12 | RAN#106 | R5-247292 | 3603 | - | F | Editorial corrections to Annex F.1.1.2 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247293 | 3604 | - | F | Correction of Table H.3.1-10 BeamFailureRecoveryConfig | 18.5.0 |
| 2024-12 | RAN#106 | R5-247295 | 3606 | - | F | Core spec alignment for SMTC FR2 patterns | 18.5.0 |
| 2024-12 | RAN#106 | R5-247298 | 3609 | - | F | Editors note removal for RedCap test case 16.6.5.1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247299 | 3610 | - | F | Alignment of FR1 RedCap L1-RSRP absolute measurement test cases to TT analysis | 18.5.0 |
| 2024-12 | RAN#106 | R5-247437 | 3619 | - | F | Correction to EN-DC FR2 event triggered reporting TCs 5.6.1.2, 5.6.1.4 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247440 | 3622 | - | F | Correction to E-UTRA event-triggered reporting of a NR FR2 neighbour cell TC 8.4.2.5 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247442 | 3623 | - | F | Correction to NR FR1 RedCap Cell reselection TC 16.1.1.2 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247472 | 3643 | - | F | Introduction of NR SA FR2-FR1 inter-frequency NES triggering conditional handover test case 7.3.3.4 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247473 | 3644 | - | F | Correction of Annex E for NES HO test cases | 18.5.0 |
| 2024-12 | RAN#106 | R5-247474 | 3645 | - | F | Correction of Annex E for NR mobility enhancements test cases | 18.5.0 |
| 2024-12 | RAN#106 | R5-247475 | 3646 | - | F | Introduction of RACH-based intra-frequency LTM PSCell switch from FR1 to FR1 test case 6.3.5.1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247476 | 3647 | - | F | Introduction of RACH-based Intra-frequency LTM PSCell switch from FR2 to FR2 test case 7.3.5.1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247488 | 3651 | - | F | Introduction of Intra-frequency SSB based L1-RSRP measurement in FR2 7.6.22.1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247509 | 3652 | - | F | Update to Annex B for NR-U test cases | 18.5.0 |
| 2024-12 | RAN#106 | R5-247685 | 3507 | 2 | F | Correction to FR2 EN-DC RLM measurement restriction test case 5.5.1.9 with TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-247686 | 3510 | 2 | F | Correction to FR2 NR SA RLM measurement restriction test case 7.5.1.9 with TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-247687 | 3595 | 1 | F | Addition of CORESET RMC for RLM 1Rx RedCap test cases | 18.5.0 |
| 2024-12 | RAN#106 | R5-247807 | 3584 | 1 | F | Update to NR SA FR1 SSB-less SCell Activation SSB-less inter-band CA based on A-TRS in non-DRX test case | 18.5.0 |
| 2024-12 | RAN#106 | R5-247808 | 3642 | 1 | F | Introduction of NR SA FR2 intra-frequency NES triggering conditional handover test case 7.3.3.3 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247809 | 3490 | 1 | F | Addition of new test case 6.3.4.4 for FR1 LTM intra-f Rach-less PCell switch without L1-RSRP measurement | 18.5.0 |
| 2024-12 | RAN#106 | R5-247810 | 3499 | 1 | F | Core spec alignment for LTM cell switch TC 6.3.4.2 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247811 | 3539 | 1 | F | General maintenance for NR-U chapter 10 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247813 | 3620 | 1 | F | Correction to EN-DC FR2 Inter-frequency measurements TCs 5.6.2.2 and 5.6.2.4 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247814 | 3607 | 1 | F | Corrections to spec implementation, including editorial corrections | 18.5.0 |
| 2024-12 | RAN#106 | R5-247815 | 3608 | 1 | F | Correction for RRM FR2 SS-RSRP measurement accuracy test cases for PC1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247836 | 3562 | 1 | F | Addition of new test case 6.3.3.6 NR FR1 intra-frequency NES-based CHO->>ok! | 18.5.0 |
| 2024-12 | RAN#106 | R5-247863 | 3527 | 1 | F | Test Tolerance update to NR-U FR1 handover tests under CCA | 18.5.0 |
| 2024-12 | RAN#106 | R5-247864 | 3528 | 1 | F | Test Tolerance update to NR-U FR1 RRC Re-establishment with CCA tests and addition of TC 11.2.2.1.3 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247865 | 3601 | 1 | F | Corrections to NRU SS-SINR measurement accuracy test cases including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-247866 | 3625 | 1 | F | Correction of SA FR1 event-triggered reporting on PCC without gaps under non-DRX test case 11.5.1.1 including Test Tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247867 | 3626 | 1 | F | Correction of SA FR1 event-triggered reporting on PCC without gaps under DRX test case 11.5.1.2 including Test Tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247879 | 3615 | 1 | F | Addition beam failure and link recovery TC 14.4.2.1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247880 | 3616 | 1 | F | Addition beam failure and link recovery TC 14.4.2.2 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247881 | 3617 | 1 | F | Addition beam failure and link recovery TC 14.4.2.3 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247882 | 3624 | 1 | F | Annex F update including TT analysis for NR-NTN BFR test cases | 18.5.0 |
| 2024-12 | RAN#106 | R5-247885 | 3649 | 1 | F | Correction of EN-DC FR2 Addition and Release Delay of NR PSCell test case 5.5.7.1 including Test Tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247886 | 3650 | 1 | F | Annex F correction for RRM enh TC 5.5.7.1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247901 | 3576 | 1 | F | Completion of FR1 intra-frequency handover for less than 5MHz test case including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-247904 | 3579 | 1 | F | Completion of NR SA FR2 PCell uplink TCI state switch for two known TCI states test case including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-247907 | 3471 | 1 | F | Correction of FR2 Event Triggered test cases including TT for PC1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247908 | 3613 | 1 | F | Annex F correction for RRM FR2 PC1 test cases including Test Tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247920 | 3563 | 1 | F | Addition of new test case 6.3.3.7 NR FR1 inter-frequency NES-based CHO | 18.5.0 |
| 2024-12 | RAN#106 | R5-247921 | 3564 | 1 | F | Addition of new test case 6.6.1.13 under Cell DTX | 18.5.0 |
| 2024-12 | RAN#106 | R5-247933 | 3572 | 1 | F | Addition of three TDD UL DL configurations in A.1.5 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247934 | 3573 | 1 | F | TCs titles alignment with RAN4 specification | 18.5.0 |
| 2024-12 | RAN#106 | R5-247943 | 3501 | 1 | F | Update of RRM TC titles for RedCap UE | 18.5.0 |
| 2024-12 | RAN#106 | R5-247945 | 3618 | 1 | F | Addition beam failure and link recovery TC 14.4.2.4 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247946 | 3542 | 1 | F | Update to NR-U SS-RSRQ test 13.4.2.1 including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-247947 | 3600 | 1 | F | Corrections to NRU SS-RSRQ measurement accuracy test cases including TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-247948 | 3627 | 1 | F | Correction of NR SA FR1 Event triggered reporting for FR1 with CCA without SSB time index detection when DRX is not used test case 11.5.2.3 including Test Tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247949 | 3628 | 1 | F | Correction of NR SA FR1 Event triggered reporting for FR1 with CCA without SSB time index detection when DRX is used test case 11.5.2.4 including Test Tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247950 | 3629 | 1 | F | Correction of NR SA FR1 Event triggered reporting for FR1 with CCA with SSB time index detection when DRX is not used test case 11.5.2.5 including Test Tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247951 | 3630 | 1 | F | Correction of NR SA FR1 Event triggered reporting for FR1 with CCA with SSB time index detection when DRX is used test case 11.5.2.6 including Test Tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247952 | 3631 | 1 | F | Correction of NR SA FR1 Event triggered reporting for FR1 without SSB time index detection when DRX is not used test case 11.5.2.7 including Test Tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247953 | 3632 | 1 | F | Correction of NR SA FR1 Event triggered reporting for FR1 without SSB time index detection when DRX is used test case 11.5.2.8 including Test Tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247954 | 3633 | 1 | F | Correction of NR SA FR1 Event triggered reporting for FR1 with SSB time index detection when DRX is not used test case 11.5.2.9 including Test Tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247955 | 3634 | 1 | F | Correction of NR SA FR1 Event triggered reporting for FR1 with SSB time index detection when DRX is used test case 11.5.2.10 including Test Tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247956 | 3635 | 1 | F | Correction of NR SA FR1 - E-UTRAN event-triggered reporting in non-DRX test case 11.5.3.1 including Test Tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247957 | 3636 | 1 | F | Correction of NR SA FR1 - E-UTRAN event-triggered reporting in DRX test case 11.5.3.2 including Test Tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247958 | 3637 | 1 | F | Correction of NR SA FR1 SSB based L1-RSRP measurement when DRX is not used test case 11.5.4.1 including Test Tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247959 | 3638 | 1 | F | Correction of NR SA FR1 SSB based L1-RSRP measurement when DRX is used test case 11.5.4.2 including Test Tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247960 | 3639 | 1 | F | Correction of NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used test case 11.5.4.3 including Test Tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247961 | 3640 | 1 | F | Correction of NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used test case 11.5.4.4 including Test Tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247962 | 3641 | 1 | F | Correction of Annex F for SA NRU measurement test cases including test tolerance | 18.5.0 |
| 2024-12 | RAN#106 | R5-247976 | 3612 | 1 | F | Correction of FR2 re-selection test case 7.1.1.1 including TT for PC1 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247980 | 3473 | 1 | F | Update TT value to LTM inter TC 6.3.2.4.2 | 18.5.0 |
| 2024-12 | RAN#106 | R5-247984 | 3582 | 1 | F | Addition of EN-DC FR1 Inter-band SSB-less SCell activation using A-TRS test case | 18.5.0 |
| 2024-12 | RAN#106 | R5-247985 | 3523 | 1 | F | Test Tolerance update to NR-U FR1 cell reselection with both NR carrier under CCA | 18.5.0 |
| 2024-12 | RAN#106 | R5-247986 | 3524 | 1 | F | Test Tolerance update to NR-U FR1 cell reselection with source NR carrier under CCA | 18.5.0 |
| 2024-12 | RAN#106 | R5-247987 | 3525 | 1 | F | Test Tolerance update to NR-U FR1 cell reselection with target NR carrier under CCA | 18.5.0 |
| 2024-12 | RAN#106 | R5-247988 | 3526 | 1 | F | Test Tolerance update to NR-U FR1 cell reselection to E-UTRAN target cell tests | 18.5.0 |
| 2024-12 | RAN#106 | R5-247998 | 3512 | 1 | F | Correction to FR1 EN-DC 2-step RACH RRM test cases 4.3.2.2.3 & 4.3.2.2.4 with TT | 18.5.0 |
| 2024-12 | RAN#106 | R5-248001 | 3514 | 1 | F | Correction to Annex A for Rel-16 2-step RACH RRM test cases | 18.5.0 |
| 2024-12 | RAN#106 | R5-248002 | 3516 | 1 | F | Correction to FR1 RedCap 2-step RACH RRM test cases 16.3.2.2.X with TT | 18.5.0 |
| 2025-03 | RAN#107 | R5-250047 | 3653 | - | F | Addition of minimum requirements for FR2 LTM intra L1-RSRP measurement | 18.6.0 |
| 2024-03 | RAN#107 | R5-250048 | 3654 | - | F | Addition of new test case 7.6.20.1 for FR2 LTM intra L1-RSRP measurement | 18.6.0 |
| 2024-03 | RAN#107 | R5-250049 | 3655 | - | F | Addition of minimum requirements for FR2 LTM inter L1-RSRP measurement with gap | 18.6.0 |
| 2024-03 | RAN#107 | R5-250051 | 3657 | - | F | Addition of minimum requirements for FR2 LTM inter L1-RSRP measurement accuracy | 18.6.0 |
| 2024-03 | RAN#107 | R5-250052 | 3658 | - | F | Addition of new test case 7.7.15.1 for FR1 LTM inter L1-RSRP measurement accuracy | 18.6.0 |
| 2024-03 | RAN#107 | R5-250053 | 3659 | - | F | Addition of cell mapping for FR2 LTM L1-RSRP test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-250054 | 3660 | - | F | Deletion of 6.7.17.1.2 from cell mapping table | 18.6.0 |
| 2024-03 | RAN#107 | R5-250055 | 3661 | - | F | Editorial corrections of FR1 LTM test cases title | 18.6.0 |
| 2024-03 | RAN#107 | R5-250058 | 3662 | - | F | Addition of minimum requirements for E-UTRAN event-triggered without measurement gaps | 18.6.0 |
| 2024-03 | RAN#107 | R5-250059 | 3663 | - | F | Addition of new test case 6.6.25.1 for E-UTRAN event-triggered without gap without interruption | 18.6.0 |
| 2024-03 | RAN#107 | R5-250060 | 3664 | - | F | Editorial corrections of 7.6.4.1 and 7.6.4.2 | 18.6.0 |
| 2024-03 | RAN#107 | R5-250061 | 3665 | - | F | Editorial corrections of 7.7.1.2 and 7.7.4.1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-250161 | 3666 | - | F | Update to RRC IEs of Further NR mobility enhancements | 18.6.0 |
| 2024-03 | RAN#107 | R5-250166 | 3667 | - | F | Update to LTM RRM Clause 6.3.2.4 | 18.6.0 |
| 2024-03 | RAN#107 | R5-250264 | 3670 | - | F | Addition of minimum requirements for FR1 NR needforgap | 18.6.0 |
| 2024-03 | RAN#107 | R5-250265 | 3671 | - | F | Addition of new test case 6.6.24.1 for FR1 NR needforgap | 18.6.0 |
| 2024-03 | RAN#107 | R5-250266 | 3672 | - | F | Addition of new test case 6.6.24.2 for FR1 NR needforgap | 18.6.0 |
| 2024-03 | RAN#107 | R5-250267 | 3673 | - | F | Addition of new test case 6.6.24.3 for FR1 NR needforgap | 18.6.0 |
| 2024-03 | RAN#107 | R5-250269 | 3675 | - | F | Addition of new test case 6.6.24.5 for FR1 NR needforgap | 18.6.0 |
| 2024-03 | RAN#107 | R5-250271 | 3676 | - | F | Addition of cell mapping for measurement gap enhancements test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-250397 | 3677 | - | F | Update RRC message content for Further NR mobility enhancements | 18.6.0 |
| 2024-03 | RAN#107 | R5-250398 | 3678 | - | F | Update to LTM TC 6.3.4.4 including TT | 18.6.0 |
| 2024-03 | RAN#107 | R5-250403 | 3679 | - | F | Correction to FR1 LTM PCell Switch test case | 18.6.0 |
| 2024-03 | RAN#107 | R5-250406 | 3680 | - | F | Update the cell configuration mapping for FR1 LTM test case 6.3.4.4 | 18.6.0 |
| 2024-03 | RAN#107 | R5-250480 | 3683 | - | F | Update to Annexure to address less than 5 MHz test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-250486 | 3688 | - | F | Annex F changes to include test tolerance for A-TRS based SSB-less SCell activation test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-250489 | 3689 | - | F | Addition of NR-DC SA FR1 conditional handover including target MCG and target SCG test case | 18.6.0 |
| 2024-03 | RAN#107 | R5-250490 | 3690 | - | F | Addition of NR-DC SA FR1 conditional handover including target MCG and candidate SCG test case | 18.6.0 |
| 2024-03 | RAN#107 | R5-250496 | 3696 | - | F | Addition of multiple AoA test setups for multiRx test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-250531 | 3701 | - | F | Addition of new test case 6.6.2.15 for inter-frequency event triggered reporting without SSB index detection in DRX mode with less than 5MHz bandwidth | 18.6.0 |
| 2024-03 | RAN#107 | R5-250596 | 3705 | - | F | Correction to TC4.5.6.4.1 with TT | 18.6.0 |
| 2024-03 | RAN#107 | R5-250600 | 3707 | - | F | Modification to Annex F for test cases 4.5.3.4, 4.5.6.3.1, 4.5.6.4.1, 4.6.6.1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-250668 | 3708 | - | F | Clarification to Non-Contention Based Random Access TCs | 18.6.0 |
| 2024-03 | RAN#107 | R5-250670 | 3709 | - | F | Correction to RRC_connection release with redirection for RedCap TC 18.2.2.1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-250671 | 3710 | - | F | Clarification on NCD-SSB absolute frequency definition | 18.6.0 |
| 2024-03 | RAN#107 | R5-250687 | 3713 | - | F | Addition of ATG RRM test case 19.6.1.1.2 - intra freq SS RSRP relative accuracy | 18.6.0 |
| 2024-03 | RAN#107 | R5-250688 | 3714 | - | F | Addition of ATG RRM test case 19.6.1.2.1 - inter freq SS RSRP absolute accuracy | 18.6.0 |
| 2024-03 | RAN#107 | R5-250689 | 3715 | - | F | Addition of ATG RRM test case 19.6.1.2.2 - inter freq SS RSRP relative accuracy | 18.6.0 |
| 2024-03 | RAN#107 | R5-250690 | 3716 | - | F | Addition of ATG RRM test case 19.6.2.1 - intra freq SS RSRQ absolute accuracy | 18.6.0 |
| 2024-03 | RAN#107 | R5-250691 | 3717 | - | F | Addition of ATG RRM test case 19.6.2.2.1 - inter freq SS RSRQ absolute accuracy | 18.6.0 |
| 2024-03 | RAN#107 | R5-250692 | 3718 | - | F | Addition of ATG RRM test case 19.6.2.2.2 - inter freq SS RSRQ relative accuracy | 18.6.0 |
| 2024-03 | RAN#107 | R5-250693 | 3719 | - | F | Addition of ATG RRM test case 19.6.3.1 - intra freq SS SINR absolute accuracy | 18.6.0 |
| 2024-03 | RAN#107 | R5-250694 | 3720 | - | F | Addition of ATG RRM test case 19.6.3.2.1 - inter freq SS SINR absolute accuracy | 18.6.0 |
| 2024-03 | RAN#107 | R5-250695 | 3721 | - | F | Addition of ATG RRM test case 19.6.3.2.2 - inter freq SS SINR relative accuracy | 18.6.0 |
| 2024-03 | RAN#107 | R5-250696 | 3722 | - | F | Addition of ATG RRM test case 19.6.4.1.1 - SSB L1 RSRP absolute accuracy | 18.6.0 |
| 2024-03 | RAN#107 | R5-250697 | 3723 | - | F | Addition of ATG RRM test case 19.6.4.1.2 - SSB L1 RSRP relative accuracy | 18.6.0 |
| 2024-03 | RAN#107 | R5-250698 | 3724 | - | F | Addition of ATG RRM test case 19.6.4.2.1 - CSIRS L1 RSRP absolute accuracy | 18.6.0 |
| 2024-03 | RAN#107 | R5-250699 | 3725 | - | F | Addition of ATG RRM test case 19.6.4.2.2 - CSIRS L1 RSRP relative accuracy | 18.6.0 |
| 2024-03 | RAN#107 | R5-250700 | 3726 | - | F | Addition of ATG RRM test case 19.6.5.1.1 - CSIRS L1 SINR absolute accuracy no dedicated IMR | 18.6.0 |
| 2024-03 | RAN#107 | R5-250701 | 3727 | - | F | Addition of ATG RRM test case 19.6.5.1.2 - CSIRS L1 SINR relative accuracy no dedicated IMR | 18.6.0 |
| 2024-03 | RAN#107 | R5-250702 | 3728 | - | F | Addition of ATG RRM test case 19.6.5.2 - SSB L1 SINR absolute accuracy dedicated IMR | 18.6.0 |
| 2024-03 | RAN#107 | R5-250703 | 3729 | - | F | Addition of ATG RRM test case 19.6.5.3.1 - CSIRS L1 SINR absolute accuracy dedicated IMR | 18.6.0 |
| 2024-03 | RAN#107 | R5-250704 | 3730 | - | F | Addition of ATG RRM test case 19.6.5.3.2 - CSIRS L1 SINR relative accuracy dedicated IMR | 18.6.0 |
| 2024-03 | RAN#107 | R5-250706 | 3732 | - | F | Correction to Rel-18 UL Tx switching RRM test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-250711 | 3735 | - | F | Correction to Rel-16 UL Tx switching RRM test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-250712 | 3736 | - | F | Correction to Annex H for UL Tx switching RRM test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-250715 | 3739 | - | F | Correction to RedCap RRM test cases 16.1.1.1 and 16.1.1.5 with TT | 18.6.0 |
| 2024-03 | RAN#107 | R5-250716 | 3740 | - | F | Correction to RedCap RRM test cases 16.3.1.1 with TT | 18.6.0 |
| 2024-03 | RAN#107 | R5-250717 | 3741 | - | F | Correction to Annex F for RedCap RRM test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-250725 | 3742 | - | F | Editorial Correction to NR SA FR1-FR1 Handover Test Case | 18.6.0 |
| 2024-03 | RAN#107 | R5-250726 | 3743 | - | F | Editorial Correction to NR SA FR1 handover | 18.6.0 |
| 2024-03 | RAN#107 | R5-250727 | 3744 | - | F | Editorial Correction to NR SA FR1 NES handover | 18.6.0 |
| 2024-03 | RAN#107 | R5-250767 | 3749 | - | F | Correction to NR SA FR1 event triggered reporting with MUSIM gap configured test case 6.6.1.9 with Message contents | 18.6.0 |
| 2024-03 | RAN#107 | R5-250768 | 3750 | - | F | Correction to NR SA FR1-FR1 event triggered reporting using MUSIM gap with lower and higher priority test case 6.6.2.13 with message contents | 18.6.0 |
| 2024-03 | RAN#107 | R5-250769 | 3751 | - | F | Addition of NR SA FR1-FR1 event triggered reporting using MUSIM gap with shorter MGRP test case 6.6.2.14 | 18.6.0 |
| 2024-03 | RAN#107 | R5-250770 | 3752 | - | F | Addition of NR SA FR2-FR2 event triggered reporting using MUSIM gap with higher priority test case 7.6.2.20 | 18.6.0 |
| 2024-03 | RAN#107 | R5-250771 | 3753 | - | F | Addition of NR SA FR2-FR2 event triggered reporting using MUSIM gap with shorter MGRP test case 7.6.2.21 | 18.6.0 |
| 2024-03 | RAN#107 | R5-250772 | 3754 | - | F | Correction to Cell configuration mapping for MUSIM gap test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-250773 | 3755 | - | F | Correction of minimum conformance requirements for MUSIM gap configured | 18.6.0 |
| 2024-03 | RAN#107 | R5-250778 | 3758 | - | F | Addition of TDCP amplitude measurement accuracy EN-DC test case | 18.6.0 |
| 2024-03 | RAN#107 | R5-250779 | 3759 | - | F | Addition of TDCP amplitude measurement accuracy SA test case | 18.6.0 |
| 2024-03 | RAN#107 | R5-250810 | 3760 | - | F | Addition of new test case 4.5.3.11 SCell activation of SSB-less Cell | 18.6.0 |
| 2024-03 | RAN#107 | R5-250812 | 3762 | - | F | Addition of new test case 6.3.3.7 NES-based conditional handover | 18.6.0 |
| 2024-03 | RAN#107 | R5-250825 | 3764 | - | F | Correction to Annex E for MCE RRM test case | 18.6.0 |
| 2024-03 | RAN#107 | R5-250921 | 3770 | - | F | Addition of new test cases for CSI-RS based L1-RSRP measurement for Satellite Access | 18.6.0 |
| 2024-03 | RAN#107 | R5-250922 | 3771 | - | F | Applicability update for L1-RSRP and L3 measurement accuracy test cases for SAN | 18.6.0 |
| 2024-03 | RAN#107 | R5-250923 | 3772 | - | F | Message content alignment for RRM FR2 Rel-15 test case 7.6.1.1. | 18.6.0 |
| 2024-03 | RAN#107 | R5-250925 | 3774 | - | F | Editorial corrections to measurement procedures minimum conformance requirements | 18.6.0 |
| 2024-03 | RAN#107 | R5-250927 | 3776 | - | F | Completion of RRM NRU channel occupancy measurement accuracy test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-250928 | 3777 | - | F | Update of RRM NRU inter-frequency measurement accuracy test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-250929 | 3778 | - | F | Completion of RRM NRU RSSI measurement accuracy test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-250959 | 3780 | - | F | Correction to message exception of RLM 1Rx test cases for RedCap | 18.6.0 |
| 2024-03 | RAN#107 | R5-250961 | 3782 | - | F | Correction to test requirements for RedCap random access test cases in Annex F | 18.6.0 |
| 2024-03 | RAN#107 | R5-250962 | 3783 | - | F | Core spec alignment for SMTC config of TC 16.6.1.10 | 18.6.0 |
| 2024-03 | RAN#107 | R5-250997 | 3784 | - | F | Modification of Annex H.3.1 to include a new table for RACH-ConfigCommon to be used for BFR TCs | 18.6.0 |
| 2024-03 | RAN#107 | R5-250998 | 3785 | - | F | Correction to Beam Failure detection and link recovery TCs | 18.6.0 |
| 2024-03 | RAN#107 | R5-251001 | 3786 | - | F | Cell configuration mapping for MG enhancement test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-251002 | 3787 | - | F | Addition of Minimum conformance requirements for event-triggered reporting test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-251003 | 3788 | - | F | Addition of NR SA FR2 and FR2-FR2 event-triggered reporting with concurrent Type-2 measurement gap and Pre-MG test case 7.6.17.1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251004 | 3789 | - | F | Addition of NR SA FR2-FR2 event-triggered reporting with two concurrent Pre-MGs test case 7.6.17.2 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251007 | 3792 | - | F | Addition of NR SA FR2-FR2 SA event triggered reporting test without gap without interruption test case 7.6.19.1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251008 | 3793 | - | F | Addition of NR SA FR2-event triggered reporting test without gap without interruption under non-DRX test case 7.6.19.2 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251009 | 3794 | - | F | Addition of E-UTRAN - NR SA FR2 event-triggered reporting without gap with interruption test case 7.6.19.3 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251012 | 3795 | - | F | Correction of NR SA FR2 Inter-frequency SSB based L1-RSRP measurement without measurement gap test case 7.6.22.1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251020 | 3802 | - | F | Minimum conformance requirements correction for NR SA FR2 NES triggering conditional handover test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-251032 | 3805 | - | F | Addition of E-UTRAN - NR SA FR2 event-triggered reporting without gap with interruption test case 8.4.3.1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251040 | 3806 | - | F | Correction of RRM propagation conditions for RLM RedCap FR2 test cases with fading | 18.6.0 |
| 2024-03 | RAN#107 | R5-251041 | 3807 | - | F | Corrections of RRM propagation conditions for RLM Rel-15 FR2 test cases with fading | 18.6.0 |
| 2024-03 | RAN#107 | R5-251042 | 3808 | - | F | Completion of RRM NRU L1-RSRP test case 11.6.4.1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251105 | 3810 | - | F | Corrections in 16.7.1.3 and 16.7.1.4 on derived parameters | 18.6.0 |
| 2024-03 | RAN#107 | R5-251106 | 3811 | - | F | Corrections in TCs 4.7.1.2 and 6.7.1.2 on derived parameters | 18.6.0 |
| 2024-03 | RAN#107 | R5-251154 | 3815 | - | F | Update to NR-NTN SSB based L1-RSRP Measurement Accuracy test cases 14.6.4.1 and 14.6.4.2 including TT | 18.6.0 |
| 2024-03 | RAN#107 | R5-251155 | 3816 | - | F | Update to NR-NTN SS-SINR Measurement Accuracy test cases 14.6.3.1 and 14.6.3.2 including TT | 18.6.0 |
| 2024-03 | RAN#107 | R5-251156 | 3817 | - | F | Correction in RedCap CSI-RS based L1-RSRP measurement test case 16.6.4.5 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251158 | 3819 | - | F | Update to TE connection diagram for RedCap 2RX PRACH test cases 16.3.2.2.x | 18.6.0 |
| 2024-03 | RAN#107 | R5-251159 | 3820 | - | F | Update to TE and UE connection diagram for PRACH test cases 6.3.2.2.x | 18.6.0 |
| 2024-03 | RAN#107 | R5-251160 | 3821 | - | F | Update to RedCap 4-step RA type PRACH test cases 16.3.2.2.x | 18.6.0 |
| 2024-03 | RAN#107 | R5-251163 | 3822 | - | F | Addition of NR-NTN DCI-based BWP switch test case 14.4.3.1.1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251164 | 3823 | - | F | Addition of NR-NTN RRC-based BWP switch test case 14.4.3.2.1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251165 | 3824 | - | F | Addition of NR-NTN CBW change test case 14.4.4.1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251167 | 3826 | - | F | Update to NR-NTN two-gap inter-frequency test cases 14.5.2.4 and 14.5.2.6 including TT | 18.6.0 |
| 2024-03 | RAN#107 | R5-251170 | 3829 | - | F | Addition of NR-NTN intra-frequency Handover test case 14.2.1.1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251174 | 3833 | - | F | Update to NR-NTN intra-frequency test cases 14.5.1.x | 18.6.0 |
| 2024-03 | RAN#107 | R5-251175 | 3834 | - | F | Update to NR-NTN two-gap intra-frequency test cases 14.5.1.5 and 14.5.1.6 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251177 | 3836 | - | F | General corrections for NR-U RRM | 18.6.0 |
| 2024-03 | RAN#107 | R5-251178 | 3837 | - | F | Addition of NR-U SFTD measurement test 12.4.1.1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251179 | 3838 | - | F | Addition of NR-U SFTD measurement test 12.5.1.1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251180 | 3839 | - | F | Update to RSSI and Channel Occupancy tests | 18.6.0 |
| 2024-03 | RAN#107 | R5-251181 | 3840 | - | F | Update to Annex E for NR-U test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-251186 | 3845 | - | F | Update to applicability statement of RRM tests 16.6.2.9 and 16.6.2.10 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251187 | 3846 | - | F | Correction to RedCap test cases configuring NCD-SSB 16.6.1.x | 18.6.0 |
| 2024-03 | RAN#107 | R5-251437 | 3681 | 2 | F | Update of TBD parameters RLM OOS in DRX and non-DRX mode for UE operating on a cell with less than 5MHz bandwidth test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-251438 | 3682 | 2 | F | Addition of RLM IS in DRX mode for UE operating on a cell with less than 5MHz bandwidth test case | 18.6.0 |
| 2024-03 | RAN#107 | R5-251562 | 3768 | 1 | F | Core Spec Alignment for Test Configuration ID for NR-NTN test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-251563 | 3769 | 1 | F | Addition of new Inter-frequency event triggered test cases for SAN | 18.6.0 |
| 2024-03 | RAN#107 | R5-251564 | 3841 | 1 | F | Addition of r18 NR-NTN RACH-based hard satellite switching test case 14.2.2.4 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251565 | 3842 | 1 | F | Addition of r18 NR-NTN RACH-less soft satellite switching test case 14.2.2.5 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251582 | 3761 | 1 | F | Update of minimum requirements for NES-based conditional handover | 18.6.0 |
| 2024-03 | RAN#107 | R5-251583 | 3803 | 1 | F | Correction of NR SA FR2 intra-frequency NES triggering conditional handover test case 7.3.3.3 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251584 | 3804 | 1 | F | Correction of NR SA FR2-FR1 inter-frequency NES triggering conditional handover test case 7.3.3.4 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251585 | 3712 | 1 | F | Addition of ATG RRM test case 19.6.1.1.1 - intra freq SS RSRP absolute accuracy | 18.6.0 |
| 2024-03 | RAN#107 | R5-251586 | 3731 | 1 | F | Correction to Annex E for ATG RRM test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-251587 | 3745 | 1 | F | Addition of Intra-frequency handover from FR1 to FR1 for ATG | 18.6.0 |
| 2024-03 | RAN#107 | R5-251588 | 3746 | 1 | F | Addition of Inter-frequency handover from FR1 to FR1 for ATG | 18.6.0 |
| 2024-03 | RAN#107 | R5-251589 | 3747 | 1 | F | Addition of Intra-frequency distance-based conditional handover from FR1 to FR1 for ATG | 18.6.0 |
| 2024-03 | RAN#107 | R5-251590 | 3748 | 1 | F | Addition of Inter-frequency distance-based conditional handover from FR1 to FR1 for ATG | 18.6.0 |
| 2024-03 | RAN#107 | R5-251591 | 3797 | 1 | F | Addition of Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM-RS in non-DRX mode test case 19.4.1.1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251592 | 3798 | 1 | F | Addition of Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM-RS in non-DRX mode test case 19.4.1.2 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251593 | 3799 | 1 | F | Addition of Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode test case 19.4.1.3 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251594 | 3800 | 1 | F | Addition of Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode test case 19.4.1.4 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251595 | 3700 | 1 | F | Addition of new test case 6.6.1.12 for intra-frequency event triggered reporting without gap with less than 5MHz bandwidth | 18.6.0 |
| 2024-03 | RAN#107 | R5-251596 | 3692 | 1 | F | Addition of SSB based L1-RSRP measurement for group-based beam reporting test case | 18.6.0 |
| 2024-03 | RAN#107 | R5-251597 | 3693 | 1 | F | Addition of CSI-RS based L1-RSRP measurement for group-based beam reporting test case | 18.6.0 |
| 2024-03 | RAN#107 | R5-251598 | 3695 | 1 | F | Addition of NR SA FR2 single DCI based active TCI state switch test case | 18.6.0 |
| 2024-03 | RAN#107 | R5-251599 | 3843 | 1 | F | Addition of FR2 multi-Rx RLM test case 5.5.1.11 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251600 | 3844 | 1 | F | Addition of FR2 multi-Rx RLM test case 7.5.1.10 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251601 | 3763 | 1 | F | Addition of MCE RRM test case 6.5.7D.4 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251602 | 3765 | 1 | F | Correction to MCE RRM test cases 6.5.7D.1, 6.5.7D.2 and 6.5.7D.3 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251603 | 3674 | 1 | F | Addition of new test case 6.6.24.4 for FR1 NR needforgap | 18.6.0 |
| 2024-03 | RAN#107 | R5-251604 | 3790 | 1 | F | Addition of NR SA FR2 event-triggered reporting with concurrent Type-2 measurement gap and NCSG test case 7.6.18.1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251605 | 3812 | 1 | F | Update to NR-U FR1 IRAT test cases 12.1.1.1 and 12.2.1.1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251606 | 3813 | 1 | F | Test Tolerance update in Annex F for NR-U cell reselection test cases 12.1.1.1 and 12.2.1.1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251611 | 3656 | 1 | F | Addition of new test case 7.6.21.1 for FR2 LTM inter L1-RSRP measurement with gap | 18.6.0 |
| 2024-03 | RAN#107 | R5-251612 | 3697 | 1 | F | Addition of minimum requirements and new test case 7.3.4.1 for FR2 LTM PCell switch from FR2 to FR2 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251613 | 3698 | 1 | F | Addition of new test case 7.3.4.2 for FR2 LTM PCell switch from FR2 to FR2 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251614 | 3699 | 1 | F | Addition of new test case 7.3.4.3 for FR2 LTM PCell switch from FR2 to FR2 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251615 | 3691 | 1 | F | Addition of NR-DC SA FR2 conditional handover including target MCG and target SCG test case | 18.6.0 |
| 2024-03 | RAN#107 | R5-251616 | 3668 | 1 | F | Update of RRM TC titles for RedCap UE | 18.6.0 |
| 2024-03 | RAN#107 | R5-251617 | 3733 | 1 | F | Correction to EN-DC SS-RSRP measurement absolute accuracy test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-251618 | 3779 | 1 | F | Correction to q-OffsetCell in NR cell re-selection test cases for RedCap | 18.6.0 |
| 2024-03 | RAN#107 | R5-251619 | 3781 | 1 | F | Correction to titles of RedCap test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-251620 | 3702 | 1 | F | Update to RedCap SS-SINR and SS-RSRQ test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-251621 | 3734 | 1 | F | Correction to NR SA SS-RSRP measurement absolute accuracy test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-251622 | 3738 | 1 | F | Correction to RedCap SS-RSRP measurement absolute accuracy test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-251644 | 3686 | 1 | F | Correction and update to TRS and A-TRS based SSB-less SCell Activation test cases including TT | 18.6.0 |
| 2024-03 | RAN#107 | R5-251645 | 3687 | 1 | F | Update to SA FR1 A-TRS based SSB-less SCell Activation test cases including TT | 18.6.0 |
| 2024-03 | RAN#107 | R5-251646 | 3756 | 1 | F | Addition of test case for NR UE Transmit Timing Test | 18.6.0 |
| 2024-03 | RAN#107 | R5-251648 | 3703 | 1 | F | Correction to TC4.5.3.4 with TT analysis | 18.6.0 |
| 2024-03 | RAN#107 | R5-251649 | 3704 | 1 | F | Correction to TC4.5.6.3.1 with TT analysis | 18.6.0 |
| 2024-03 | RAN#107 | R5-251651 | 3827 | 1 | F | Update to NR-NTN SSB-based RLM test cases 14.4.1.1, 14.4.1.2, 14.4.1.3 and 14.4.1.4 including TT | 18.6.0 |
| 2024-03 | RAN#107 | R5-251652 | 3828 | 1 | F | Update to NR-NTN CSI-RS-based RLM test cases 14.4.1.5, 14.4.1.6, 14.4.1.7 and 14.4.1.8 including TT | 18.6.0 |
| 2024-03 | RAN#107 | R5-251653 | 3830 | 1 | F | Addition of NR-NTN inter-frequency Handover test case 14.2.1.2 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251654 | 3825 | 1 | F | Update to NR-NTN inter-frequency test cases 14.5.2.x including TT | 18.6.0 |
| 2024-03 | RAN#107 | R5-251655 | 3767 | 1 | F | Correction of FR2 Rel-15 Random Access Test Cases Including TT for PC1 | 18.6.0 |
| 2024-03 | RAN#107 | R5-251657 | 3814 | 1 | F | Update to NR-NTN cell reselection test cases 14.1.9 and 14.1.10 including TT | 18.6.0 |
| 2024-03 | RAN#107 | R5-251658 | 3835 | 1 | F | Update to Annex F for NR-NTN test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-251691 | 3684 | 1 | F | Completion of message contents in SA FR2 UE transmit timing test case | 18.6.0 |
| 2024-03 | RAN#107 | R5-251741 | 3831 | 1 | F | Update to NR-NTN intra-frequency time-based CHO test case 14.2.1.3 including TT | 18.6.0 |
| 2024-03 | RAN#107 | R5-251742 | 3832 | 1 | F | Update to NR-NTN inter-frequency time-based CHO test case 14.2.1.4 including TT | 18.6.0 |
| 2024-03 | RAN#107 | R5-251744 | 3706 | 1 | F | Correction to TC4.6.6.1 with TT analysis | 18.6.0 |
| 2024-03 | RAN#107 | R5-251748 | 3737 | 1 | F | Correction to Rel-17 UL Tx switching RRM test cases | 18.6.0 |
| 2024-03 | RAN#107 | R5-251751 | 3685 | 1 | F | Correction of clause number for the 3 AoA setup in TCI state switch test case | 18.6.0 |
| 2024-03 | RAN#107 | R5-251752 | 3694 | 1 | F | Addition of NR SA FR2 DCI based active TCI state switch with m-DCI test case | 18.6.0 |
| 2025-03 | RAN#107 | - | - | - | - | accept ed revision marks in clause 1-4 | 18.6.1 |
| 2025-06 | RAN#108 | R5-251819 | 3847 | - | F | Update LTM TC 7.6.20.1 including TTs | 18.7.0 |
| 2025-06 | RAN#108 | R5-251820 | 3848 | - | F | Update LTM TC 7.6.21.1 including TTs | 18.7.0 |
| 2025-06 | RAN#108 | R5-251821 | 3849 | - | F | Update LTM TC 7.7.15.1 including TTs | 18.7.0 |
| 2025-06 | RAN#108 | R5-251822 | 3850 | - | F | Update Annex F for FR2 LTM L1-RSRP test cases including TTs | 18.7.0 |
| 2025-06 | RAN#108 | R5-251823 | 3851 | - | F | Addition of new test case 6.6.25.2 and 6.6.25.3 for E-UTRAN event-triggered reporting | 18.7.0 |
| 2025-06 | RAN#108 | R5-251830 | 3854 | - | F | Update of test parameters and requirements for LTM L1-RSRP test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-251832 | 3855 | - | F | Correction to R18 Less than 5M test cases 6.6.1.12 and 6.6.2.15 | 18.7.0 |
| 2025-06 | RAN#108 | R5-251833 | 3856 | - | F | Update to R18 Less than 5M test cases TC 6.6.1.12 including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-251834 | 3857 | - | F | Update to R18 Less than 5M test cases TC 6.6.2.15 including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-251837 | 3858 | - | F | Correction to R18 LTM FR2 test cases 7.3.4.1, 7.3.4.2 and 7.3.4.3 | 18.7.0 |
| 2025-06 | RAN#108 | R5-251841 | 3859 | - | F | Update to R18 LTM FR2 test case 7.3.4.1 including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-251842 | 3860 | - | F | Update to R18 LTM FR2 test case 7.3.4.2 including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-251843 | 3861 | - | F | Update to R18 LTM FR2 test case 7.3.4.3 including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-251847 | 3863 | - | F | Completion of SSB-based RLM IS 6.5.1.16 test case including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-251849 | 3865 | - | F | Addition of test tolerance and measurement uncertainty for less than 5 MHz test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-251854 | 3866 | - | F | Addition of MU values for RRM FR2 PC6 | 18.7.0 |
| 2025-06 | RAN#108 | R5-251860 | 3869 | - | F | Update message contents for LTM L1-RSRP test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-251861 | 3870 | - | F | Update of SMTC config for concurrent gap TC 6.6.18.2 | 18.7.0 |
| 2025-06 | RAN#108 | R5-251868 | 3873 | - | F | Addition of cell configuration for ATG RRM test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-251990 | 3877 | - | F | Update to needforgap test cases 6.6.24.x | 18.7.0 |
| 2025-06 | RAN#108 | R5-251994 | 3879 | - | F | Core spec alignment for UL TX switching 6.5.7.1 | 18.7.0 |
| 2025-06 | RAN#108 | R5-252172 | 3881 | - | F | Correction of RMSI CORESET configuration for less than 5 MHz test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-252178 | 3885 | - | F | Correction of test requirements of SA FR2 CHO and PSCell change | 18.7.0 |
| 2025-06 | RAN#108 | R5-252179 | 3886 | - | F | Addition of NR-DC SA FR2 CHO with candidate SCG test case | 18.7.0 |
| 2025-06 | RAN#108 | R5-252375 | 3892 | - | F | Correction to TC5.5.3.6 with TT analysis | 18.7.0 |
| 2025-06 | RAN#108 | R5-252378 | 3894 | - | F | Correction to TC7.5.6.5.1 with TT analysis | 18.7.0 |
| 2025-06 | RAN#108 | R5-252382 | 3896 | - | F | Correction to TC6.5.6.3.1 with TT analysis | 18.7.0 |
| 2025-06 | RAN#108 | R5-252384 | 3897 | - | F | Correction to TC6.5.6.5.1 with TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-252386 | 3898 | - | F | Correction to TC6.6.7.1 with TT analysis | 18.7.0 |
| 2025-06 | RAN#108 | R5-252388 | 3899 | - | F | Correction to TC6.6.7.2 with TT analysis | 18.7.0 |
| 2025-06 | RAN#108 | R5-252444 | 3904 | - | F | Addition of new test case 6.5.13.1 unified TCI for mDCI mTRP | 18.7.0 |
| 2025-06 | RAN#108 | R5-252445 | 3905 | - | F | Addition of new test case 7.5.13.4 joint TCI for sDCI mTRP | 18.7.0 |
| 2025-06 | RAN#108 | R5-252446 | 3906 | - | F | Addition of new test case 7.5.13.5 separate TCI for sDCI mTRP | 18.7.0 |
| 2025-06 | RAN#108 | R5-252488 | 3908 | - | F | Editorial Correction to Clause numbers for RRM test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-252573 | 3910 | - | F | Addition of new FR1 RRM TC 6.3.3.5 | 18.7.0 |
| 2025-06 | RAN#108 | R5-252574 | 3911 | - | F | Addition of new FR1 RRM TC 6.5.12.1 | 18.7.0 |
| 2025-06 | RAN#108 | R5-252693 | 3924 | - | F | Correction to ATG SS-RSRP accuracy test cases 19.6.1.X with TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-252694 | 3925 | - | F | Correction to ATG SS-RSRQ accuracy test cases 19.6.2.X with TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-252695 | 3926 | - | F | Correction to ATG SS-SINR accuracy test cases 19.6.3.X with TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-252696 | 3927 | - | F | Removal of duplications in ATG L1-SINR accuracy test cases 19.6.5.X | 18.7.0 |
| 2025-06 | RAN#108 | R5-252697 | 3928 | - | F | Correction to Annex E for ATG RRM test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-252698 | 3929 | - | F | Correction to Annex F for ATG RRM test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-252710 | 3930 | - | F | Addition of MG_enh2 RRM test case 8.4.3.2 FR1 inter-RAT without gap | 18.7.0 |
| 2025-06 | RAN#108 | R5-252711 | 3931 | - | F | Addition of MG_enh2 RRM test cases 6.6.22.X concurrent gap including Pre-MG | 18.7.0 |
| 2025-06 | RAN#108 | R5-252712 | 3932 | - | F | Addition of MG_enh2 RRM test cases 6.6.23.X concurrent gap including NCSG | 18.7.0 |
| 2025-06 | RAN#108 | R5-252713 | 3933 | - | F | Correction to Annex E for MG_enh2 test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-252715 | 3934 | - | F | Correction to 2-step RACH RMCs in Annex A | 18.7.0 |
| 2025-06 | RAN#108 | R5-252716 | 3935 | - | F | Correction to 2-step RACH RMCs for NR-U in Annex A | 18.7.0 |
| 2025-06 | RAN#108 | R5-252723 | 3941 | - | F | Specification quality improvement for 38.533 clause 9 | 18.7.0 |
| 2025-06 | RAN#108 | R5-252724 | 3942 | - | F | Correction to FR1 EN-DC L1 and L3 RSRP absolute accuracy test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-252725 | 3943 | - | F | Correction to FR1 NR SA L1 and L3 RSRP absolute accuracy test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-252726 | 3944 | - | F | Correction to Inter-RAT FR1 NR RSRP absolute accuracy test case | 18.7.0 |
| 2025-06 | RAN#108 | R5-252776 | 3950 | - | F | Correction of NR SA NES triggering conditional handover test cases including test tolerance | 18.7.0 |
| 2025-06 | RAN#108 | R5-252779 | 3951 | - | F | Addition of new CADC fast measurement test cases 6.6.9.x | 18.7.0 |
| 2025-06 | RAN#108 | R5-252782 | 3952 | - | F | Correction of NR SA FR2 Inter-frequency SSB based L1-RSRP measurement without measurement gap test case 7.6.22.1 | 18.7.0 |
| 2025-06 | RAN#108 | R5-252783 | 3953 | - | F | Annex H correction for LTM test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-252785 | 3955 | - | F | Addition of new NR SA FR2 PDCCH-order RACH test cases 7.3.2.4.x | 18.7.0 |
| 2025-06 | RAN#108 | R5-252804 | 3960 | - | F | Test Tolerance update for intra-frequency NR cell reselection for Satellite Access test 14.1.1 and 14.1.2. | 18.7.0 |
| 2025-06 | RAN#108 | R5-252806 | 3962 | - | F | Test Tolerance update for inter-frequency NR cell reselection for Satellite Access test 14.1.5. | 18.7.0 |
| 2025-06 | RAN#108 | R5-252807 | 3963 | - | F | Test Tolerance update for inter-frequency NR cell reselection for Satellite Access test 14.1.6. | 18.7.0 |
| 2025-06 | RAN#108 | R5-252829 | 3968 | - | F | Addition of NR NTN 14.2.2.3.1 test case | 18.7.0 |
| 2025-06 | RAN#108 | R5-252877 | 3970 | - | F | Correction to NR SA SS-RSRP measurement accuracy for Satellite Access TCs | 18.7.0 |
| 2025-06 | RAN#108 | R5-252887 | 3976 | - | F | Correction of Connection Diagram for RedCap RA test case with CSI-RS | 18.7.0 |
| 2025-06 | RAN#108 | R5-252938 | 3982 | - | F | Update to r18 NR-NTN satellite switching tests 14.2.2.4 and 14.2.2.5 | 18.7.0 |
| 2025-06 | RAN#108 | R5-252939 | 3983 | - | F | Addition of r18 NR-NTN RACH-less Handover test 14.2.1.8 | 18.7.0 |
| 2025-06 | RAN#108 | R5-252940 | 3984 | - | F | Update to Annex E for r18 NR-NTN test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-252941 | 3985 | - | F | Update to NR-NTN measurement reporting tests | 18.7.0 |
| 2025-06 | RAN#108 | R5-252942 | 3986 | - | F | Addition of NR-NTN MAC-CE based pathloss RS switch test 14.4.5.1 | 18.7.0 |
| 2025-06 | RAN#108 | R5-252944 | 3988 | - | F | Update to general section of NR-NTN | 18.7.0 |
| 2025-06 | RAN#108 | R5-252945 | 3989 | - | F | Update to NR-NTN intra-frequency re-establishment test 14.2.2.1.1 including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-252946 | 3990 | - | F | Update to NR-NTN inter-frequency re-establishment test 14.2.2.1.2 including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-252947 | 3991 | - | F | Update to NR-NTN DCI-based BWP switch test 14.4.3.1.1 including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-252948 | 3992 | - | F | Update to NR-NTN RRC-based BWP switch test 14.4.3.2.1 including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-252949 | 3993 | - | F | Update to NR-NTN UE-specific CBW change test 14.4.4.1 including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-252950 | 3994 | - | F | Update to NR-NTN Transmit Timing test 14.3.1.1 including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-252951 | 3995 | - | F | Update to NR-NTN Timing Advance test 14.3.2.1 including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-252952 | 3996 | - | F | Update to Annex F for NR-NTN test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-252953 | 3997 | - | F | Addition of Annex H common message contents for NR-NTN | 18.7.0 |
| 2025-06 | RAN#108 | R5-252954 | 3998 | - | F | Update to NR-NTN Handover test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-252956 | 4000 | - | F | Addition of FR1-NTN timing measurement MU | 18.7.0 |
| 2025-06 | RAN#108 | R5-252957 | 4001 | - | F | Update to NR-U SFTD test case 12.4.1.1 including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-252961 | 4005 | - | F | Correction for L1-RSRP reporting tests in FR2 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253476 | 3889 | 1 | F | Addition of new test case 14.2.2.2.1 and 14.2.2.2.2 for NR NTN | 18.7.0 |
| 2025-06 | RAN#108 | R5-253477 | 3902 | 1 | F | Addition of NR SA FR1 inter-RAT Cell reselection with NTN Carrier for Satellite Access in 14.1.11 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253478 | 3903 | 1 | F | Addition of NR SA FR1-FR1 Cell re-selection with TN carrier for Satellite Access in 14.1.12 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253492 | 3878 | 1 | F | Update of RRM distance-based conditional Handover test cases in FR1 for ATG | 18.7.0 |
| 2025-06 | RAN#108 | R5-253493 | 3921 | 1 | F | Addition of ATG intra-frequency CSI-RSRP accuracy test cases 19.6.6.1.1 and 19.6.6.1.2 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253494 | 3922 | 1 | F | Addition of ATG intra-frequency CSI-RSRQ accuracy test case 19.6.7.1 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253495 | 3923 | 1 | F | Addition of ATG intra-frequency CSI-SINR accuracy test case 19.6.8.1 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253496 | 3958 | 1 | F | Addition of ATG SA event triggered reporting test cases 19.5.1.x | 18.7.0 |
| 2025-06 | RAN#108 | R5-253497 | 3957 | 1 | F | Correction of ATG Radio Link Monitoring test cases 19.4.1.x | 18.7.0 |
| 2025-06 | RAN#108 | R5-253498 | 3864 | 1 | F | Addition of RLM IS non DRX less than 5 MHz test case | 18.7.0 |
| 2025-06 | RAN#108 | R5-253499 | 3909 | 1 | F | Addition of test requirement for L1-RSRP measurement for group-based beam reporting test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-253500 | 3980 | 1 | F | Addition of multi-Rx TRP-specific BFR test 7.5.5.14 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253501 | 3965 | 1 | F | Update of NR operating band configuration table for band n13 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253502 | 3890 | 1 | F | Update to 6.3.2.4 and add abbreviation for LTM | 18.7.0 |
| 2025-06 | RAN#108 | R5-253503 | 3956 | 1 | F | Correction of event-triggered reporting test cases 7.6.19.x | 18.7.0 |
| 2025-06 | RAN#108 | R5-253504 | 3913 | 1 | F | Correction to RRC messages and IEs in H.3.12 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253505 | 3974 | 1 | F | Removal of p-max from message content exceptions for FR2 PRACH tests | 18.7.0 |
| 2025-06 | RAN#108 | R5-253506 | 4004 | 1 | F | Update to FR2 BFR tests | 18.7.0 |
| 2025-06 | RAN#108 | R5-253507 | 3936 | 1 | F | Correction to Tx switching RRM test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-253508 | 3959 | 1 | F | Correction for NR SA FR1 even triggered reporting with gap in DRX TC 6.6.1.4 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253509 | 3945 | 1 | F | Correction to FR1 RedCap L1 and L3 RSRP absolute accuracy test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-253510 | 3912 | 1 | F | Addition of new FR1 RRM TC 6.5.12.2 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253511 | 3888 | 1 | F | RRM Spec Quality Improvement clause 7 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253512 | 3901 | 1 | F | Spec quality improvement - clauses 1-4 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253513 | 3972 | 1 | F | RRM Spec quality improvement draft CR for clause 5 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253514 | 3937 | 1 | F | Specification quality improvement for 38.533 clause 6.1 to 6.3 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253515 | 3938 | 1 | F | Specification quality improvement for 38.533 clause 6.4 and 6.5 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253516 | 3939 | 1 | F | Specification quality improvement for 38.533 clause 6.6 and 6.7 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253517 | 3940 | 1 | F | Specification quality improvement for 38.533 clause 8 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253518 | 3947 | 1 | F | RRM Spec quality improvement draft CR clause 10 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253519 | 3948 | 1 | F | RRM Spec quality improvement draft CR clause 11 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253520 | 3949 | 1 | F | RRM Spec quality improvement draft CR clause 12 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253521 | 3971 | 1 | F | RRM Spec quality improvement draft CR for clause 13 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253522 | 3914 | 1 | F | RRM Spec quality improvement CR for clause 16 FR1 RedCap | 18.7.0 |
| 2025-06 | RAN#108 | R5-253523 | 3915 | 1 | F | RRM Spec quality improvement CR for clause 17 FR2 RedCap | 18.7.0 |
| 2025-06 | RAN#108 | R5-253524 | 3967 | 1 | F | RRM Spec quality improvement for clause 18 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253525 | 3973 | 1 | F | RRM Spec quality improvement draft CR for Annexes | 18.7.0 |
| 2025-06 | RAN#108 | R5-253526 | 3966 | 1 | F | RRM Spec quality improvement for clause 14 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253529 | 3919 | 1 | F | Correction of NR SA FR2-FR2 event triggered reporting using MUSIM gap with higher priority test case 7.6.2.20 with TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-253537 | 3874 | 1 | F | Update to RRM Intra-frequency handover test case in FR1 for ATG including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-253538 | 3875 | 1 | F | Update to RRM Inter-frequency handover test cases in FR1 for ATG including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-253542 | 3961 | 1 | F | Test Tolerance update for Measurement Initiation based intra-frequency NR cell reselection for Satellite Access test 14.1.3 and 14.1.4. | 18.7.0 |
| 2025-06 | RAN#108 | R5-253546 | 3893 | 1 | F | Correction to TC5.5.6.5.1 with TT analysis | 18.7.0 |
| 2025-06 | RAN#108 | R5-253547 | 3895 | 1 | F | Correction to TC7.5.6.3.1 with TT analysis | 18.7.0 |
| 2025-06 | RAN#108 | R5-253548 | 3900 | 1 | F | Annex F modification for some TCs | 18.7.0 |
| 2025-06 | RAN#108 | R5-253554 | 3876 | 1 | F | Test Tolerance Update in Annex F for RRM Intra-Frequency and Inter-Frequency handover test case in FR1 for ATG | 18.7.0 |
| 2025-06 | RAN#108 | R5-253560 | 3916 | 1 | F | Correction to NR SA FR1 event triggered reporting with MUSIM gap configured test case 6.6.1.9 with TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-253561 | 3917 | 1 | F | Correction to NR SA FR1-FR1 event triggered reporting using MUSIM gap with lower and higher priority test case 6.6.2.13 with TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-253562 | 3918 | 1 | F | Correction to NR SA FR1-FR1 event triggered reporting using MUSIM gap with shorter MGRP test case 6.6.2.14 with TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-253565 | 3891 | 1 | F | Update to 4.5.3.11 SCell activation of SSB-less Cell including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-253569 | 3852 | 1 | F | Update FR1 NR needforgap 6.6.24.1 and 6.6.24.5 including TTs | 18.7.0 |
| 2025-06 | RAN#108 | R5-253570 | 3853 | 1 | F | Update FR1 NR needforgap 6.6.24.2, 6.6.24.3 and 6.6.24.4 including TTs | 18.7.0 |
| 2025-06 | RAN#108 | R5-253571 | 4002 | 1 | F | Update to NR-U SFTD test case 12.5.1.1 including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-253572 | 4003 | 1 | F | Update to Annex F for NR-U test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-253589 | 3981 | 1 | F | Addition of multi-RX L1-RSRP with scheduling restrictions test 7.5.16 | 18.7.0 |
| 2025-06 | RAN#108 | R5-253597 | 3880 | 1 | F | Introduction of new test case 19.2.3.3 SA RRC Connection Release with Redirection for ATG | 18.7.0 |
| 2025-06 | RAN#108 | R5-253598 | 3871 | 1 | F | Addition of Intra-frequency RRC Re-establishment in FR1 for ATG | 18.7.0 |
| 2025-06 | RAN#108 | R5-253599 | 3872 | 1 | F | Addition of Inter-frequency RRC Re-establishment in FR1 with unknown target cell without serving cell timing for ATG | 18.7.0 |
| 2025-06 | RAN#108 | R5-253617 | 3969 | 1 | F | Test Tolerance update in NR NTN Beam Failure Detection and link recovery test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-253623 | 3964 | 1 | F | Test Tolerance update for Time based and Location based Measurement Initiation inter-frequency NR cell reselection for Satellite Access test 14.1.7 and 14.1.8. | 18.7.0 |
| 2025-06 | RAN#108 | R5-253624 | 3987 | 1 | F | Update to NR-NTN measurement accuracy tests | 18.7.0 |
| 2025-06 | RAN#108 | R5-253625 | 3999 | 1 | F | Update to SMTC profiles for NR-NTN | 18.7.0 |
| 2025-06 | RAN#108 | R5-253630 | 3862 | 1 | F | Completion of RLM OOS for less than 5MHz bandwidth test cases including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-253633 | 3887 | 1 | F | Missing AoA configuration for different TRPs in SA FR2 UE transmit timing test case | 18.7.0 |
| 2025-06 | RAN#108 | R5-253634 | 3882 | 1 | F | Completion of NR-DC SA FR1 CHO test case including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-253635 | 3883 | 1 | F | Update to NR-DC SA FR1 CHO with target MCG and candidate SCG test case including TT | 18.7.0 |
| 2025-06 | RAN#108 | R5-253636 | 3884 | 1 | F | Addition of test tolerance and measurement uncertainty for NR-DC SA FR1 CHO test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-253639 | 3867 | 1 | F | Addition of test tolerance analysis for DCI based TCI switch multi-Rx test cases | 18.7.0 |
| 2025-06 | RAN#108 | R5-253644 | 3946 | 1 | F | FR1 and EUTRA - FR2 testability issue CR | 18.7.0 |