3GPP TSG-SA4 Meeting ad hoc post #131-bis-e	S4aI250078

Online, 7 May 2025 – 2 June 2025

Agenda item: 	2.5

Source: 	Dolby Laboratories, Inc.

Title: 	[AMD_PRO-MED] WT#2: CMMF End-to-End Examples

Document for	Approval

# 1	Overview

Coded Multisource Media Format (CMMF) [1] is one method that can be used to enable Media delivery from multiple service endpoints/locations within the 5GMS System. This document provides two complete end-to-end examples showing how to provision and stream media using CMMF within the 5GMS System. Throughout these examples, features that are required but not yet supported within the 5GMS specifications will be highlighted. Furthermore, each example will focus on a relatively complex deployment scenario with the understanding that these examples can be simplified to other deployments. Finally, proposals to implement the required, missing features identified within the examples is provided. In addition, a proposed schema for a general Media Player Entry document that can be used to supplement existing, traditional Media Player Entry documents (e.g., MPDs, HLS playlists, URL to a video clip, etc.) is also proposed.

# 2	General Media Player Entry schema

The deployment scenarios provided later in this document use a high-level/meta Media Player Entry document to signal the necessary parameters to configure a streaming session that uses CMMF. This general Media Player Entry document is intended to augment a traditional Media Player Entry such as an MPD, HLS playlist, URL to a video file with features that may not otherwise be supported.

A proposed schema for this general Media Player Entry document with CMMF configuration parameters is provided in table 2-1.

Table 2-1: General Media Player Entry schema example

| Schema | Description |
| --- | --- |
| { "mediaResourceInformation": { "mediaResource": string, "contentType": string, }, "downlinkConfiguration": [ { "mediaResourcePathPattern": string, "serviceLocation": [ { "baseURL": string, "requestPathPattern": string, "mappedPath": string, }, ], "cmmfConfiguration": { "cmmfVersion": number, "cmmfCodeType": number, "cmmfProfile": string, "cmmfProfileDescription": string, }, }, ], } | REQUIRED. A media resource such as an MPD, HLS playlist, or URL(s) to a media resource. If this paramater takes the value of "*\n", the media resource or its URL is provided via a process external to this configuration. REQUIRED. The MIME content type of the mediaResource parameter. For example, the MIME content type for a media Resource with value "*\n" is "text/plain"; and the MIME content type of a URL, or list of URLs, is "text/uri-list". OPTIONAL. A regular expression against which the path of the media resource URL shall be compared. If true, this configuration applies. If it is not defined, this configuration applies to all media resource URLs. OPTIONAL. Base URL of the service location exposed by the 5GMSd AS. Base URLs defined here overwrite the base URLs of the media resource (if they exist) identified above. OPTIONAL. A regular expression against which the path of the media resource URL shall be compared, including the leading “/”, and up to and including the final “/” shall be compared. (Any leaf path element following the final “/” shall be excluded from this comparison.) OPTIONAL. A replacement for the portion of the media resource path that matches the requestPathPattern. REQUIRED if CMMF in use. Version of the CMMF specification in use. OPTIONAL. The CMMF code_type used (see clause 6.1.4.11 of ETSI TS 103 973 [1]). Note: The cmmfCodeType is provided within the bitstream_header() subatom of the CMMF object distributed on M4d. OPTIONAL. The CMMF profile_type (see clause 6.1.4.11 of ETSI TS 103 973 [1]). Note: the cmmfProfile is provided within the bitstream_header() subatom of the CMMF object distributed on M4d. OPTIONAL. The CMMF profile_description (see clause 6.1.4.12 of ETSI TS 103 973 [1]). Note: The cmmfProfileDescription is provided within the bitstream_header() subatom of the CMMF object distributed on M4d. |

NOTE:	The above schema can be extended to provide other configuration information. For example, Content Steering Server [2] URLs, protocol types and versions, etc. that extend existing “traditional” Media Player Entries (e.g., MPD, HLS playlists, URLs to video clips, etc.).

# 3	Implementation Scenario 1: Media delivery from multiple service locations using CMMF

## 3.1			Overview

The first implementation scenario shows how CMMF can be used to enable a basic deployment where a single Provisioning Session is configured by the 5GMSd Application Provider to expose multiple service locations. This scenario assumes the 5GMSd Application Provider provisions the system in the following manner:

1.	A 5GMSd AS is provisioned to ingest media resources at reference point M2d, create different CMMF “versions” or “stripes” of each ingested media resource, and serve those CMMF “versions” or “stripes” from multiple service locations exposed at reference point M4d.

2.	Different media assets (e.g., described by a DASH MPD or HLS playlist) are presented to the 5GMSd-Aware Application in the form of a content library. After selection of one media asset, the MPDs or HLS playlist is provided to the 5GMSd Client from the 5GMSd Application Provider without modification (i.e., baseURLs are not added to MPDs to communicate the location of 5GMSd AS service locations exposed at reference point M4d, nor is any CMMF configuration information embedded within these documents). The MPD or HLS playlist may be obtained by the 5GMSd Client via either reference point M8d or reference point M4d. Furthermore, each MPD or HLS playlist describes different content types (e.g., video, audio, subtitles, etc.) required to be obtained by the 5GMSd Client during a streaming session.

3.	A general Media Player Entry document is created (either by the 5GMSd Application Provider or by the 5GMSd AS using a Content Preparation Template) to communicate streaming session configuration information (e.g., 5GMSd AS service location URLs, CMMF configuration information, etc.). This Media Player Entry document, along with the selected DASH MPD or HLS playlist, is used by the 5GMSd Client to initiate a streaming session.

4.	The URL of this general Media Player Entry document may be communicated to the 5GMSd Client via reference point M8d or within the Service Access Information at reference point M5d.

The implementation scenario is illustrated in figure 3-1. For the purposes of this example, it will be assumed that:

-	The Media Application Provider creates one Provisioning Session. The Content Hosting Configuration defines three distributions where each one is configured to individually prepare content using a Content Preparation Template and exposes a service location at M4d.

-	The configured distributions are labelled “A”, “B”, and “C”. Distribution A distributes one “version” or “stripe” of the CMMF-encoded media resources, Distribution B distributes a different “version” or “stripe”, etc. The “version” or “stripe” distributed from each configured distribution are labelled “CMMF-A”, “CMMF-B”, and “CMMF-C” respectively.

-	To provide increased robustness in the case of server failures, network congestion, etc. service locations exposed by the 5GMSd AS at reference point M4d should not be physically located with each other.

-	Pull-based protocols are used to deliver media resource from the 5GMSd Application Provider to 5GMSd Clients at reference points M2d and M4d.

-	The URL of the general Media Player Entry is communicated to the 5GMSd Client within the Service Access Information available at reference point M5d, and the general Media Player Entry can be obtained from any service location exposed at M4d.

-	The MPD URL is obtained from the 5GMSd Application Provider via reference point M8d as a relative path to the MPD resource. This relative path is combined with a base URL parameter provided in the general Media Player Entry document.

-	The MPD is obtained from the 5GMSd Application Provider via reference points M2d and M4d without modification (e.g., baseURLs are not added to the MPD listing 5GMSd AS service locations, etc.).

-	For the purposes of this example, it is assumed that the MPD resource describes segmented media (i.e., each segment of video, audio, etc. is uniquely addressable).

Media Player Entry, MPD, and CMMF-C

Media Player Entry, MPD, and CMMF-B

Media Player Entry, MPD, and CMMF-A

MPD URL

Figure 3-1: Basic deployment scenario for 5GMSd AS ingest, content preparation, and delivery using CMMF

The remainder of this clause provides an implementation of the 5GMS APIs and protocols to realize the implementation scenario.

## 3.2			Provisioning and Content Hosting Configurations

The procedures for provisioning the Content Hosting Configurations follow those specified in clauses 5.3, 5.4, and 7 of TS 26.501 [3]. This clause provides example values used to configure the three Provisioning Sessions to realize the implementation scenario.

### 3.2.1	Provisioning Session provisioning and configuration

To configure the 5GMSd AS, one provisioning session is created by the 5GMSd Application Provider using the create Provisioning Session resource operation specified in clause 5.2.2.3 of TS 26.510 [4] and the API specified in clause 8.2 of TS 26.510. Example values of the provisioning session API parameters used are shown in table 3.2.1-1.

Table 3.2.1-1: ProvisioningSession resource parameters

| Property name | Property value | Assigned by |
| --- | --- | --- |
| provisioningSessionId | provisioning.session.a | Media AF |
| provisioningSessionType | MS_DOWNLINK | Media Application Provider |
| externalServiceId | com.‌provider.‌service |  |
| appId | dash.downlink.streaming |  |
| NOTE: Values for provisioningSessionId, externalServiceId, and appId are notional and may not reflect actual values. |  |  |

### 3.2.2	Content Preparation Templates provisioning and configuration

The Media Application Provider provisions one CMMF encoder Content Preparation Template within the provisioning.session.a Provisioning Session using the create Content Preparation Template resource operation specified in clause 5.2.5.2 of TS 26.510 and the API defined in clause 8.5 of TS 26.510. While these CMMF encoder Content Preparation Templates have yet to be defined, assume that they have the following functionality:

-	Content Preparation is triggered upon the receipt of a pull-based request at a service location exposed at reference point M4d.

-	The path of the requested resource is available to the Content Preparation Template associated with the service location that received a request.

-	Source content (e.g., original video, audio, etc.) is ingested by the 5GMS AS at reference point M2d and encoded within a CMMF object by the Content Preparation Template. A sub-path URL added by the 5GMSd Client into the M4d request URL is used to determine which CMMF “version” or “stripe” is generated. For the purposes of this example, the Content Preparation Template will generate the CMMF-A “version” or “stripe” if a M4d request URL containing the sub-path cmmf-a is received; it will generate the CMMF-B “version” or “stripe” if the URL contains the sub-path cmmf-b is received; and it will generate the CMMF-C “version” or “stripe” if the URL contains the sub-path cmmf-c is received.

-	Upon completion of the CMMF encoding operation, the CMMF object is made available to the 5GMS AS for caching and/or delivery to the requesting 5GMSd Client at the service location where the request was received.

Upon success provisioning of the Content Preparation Template, the Media AF provides the cmmf.content.preparation.template Content Preparation Template ID to the Media Application Provider.

### 3.2.3	Server Certificates provisioning and configuration

The Media Application Provider provisions the Server Certificates for the Provisioning Session using the create Server Certificate resource operation specified in clause 5.2.4.2 of TS 26.510 and the API defined in clause 8.4 of TS 26.510. Example values for the certificates generated by the Media AF are provided in table 3.2.3-1.

Table 3.2.3-1: Server Certificates

| CN | subjectAltName | certificateId |
| --- | --- | --- |
| *.com-provider-service. ms.as.3gppservices.org | *.com-provider-service. ms.as.3gppservices.org | server.certificate.a |
| NOTE: The canonical 5GMS AS authority at reference point M4 defined in clause 6.0.2.2 of TS 26.512 [5] needs to be extended to include a distribution ID in order to differentiate between more than one content distributions defined within a single Content Hosting Configuration. As an example, the canonical domain name for service locations exposed at M4d will have the following canonical domain name: {distributionId}.{modifiedExternalServiceId}.ms.as.3gppservices.org. |  |  |

### 3.2.4	Content Hosting provisioning and configuration

The Media Application Provider provisions the Content Hosting Configuration for each Provisioning Session using the create Content Hosting Configuration resource operation specified in clause 5.2.8.2 of TS 26.510 and the API defined in clause 8.8 of TS 26.510. Provisioning the Content Hosting Configuration in each 5GMS AS is performed according to:

-	The example base URL of the Media Application Provider’s origin server is https://origin.media-application-provider.com.

-	The general Media Player Entry document is available at 5gms-media-player-entry.json. Furthermore, the Media Player Entry document has a MIME content type application/json.

Table 3.2.4-1 provides example values for the Content Hosting Configuration API parameters.

Table 3.2.4-1: ContentHostingConfiguration resource parameters

| Property name |  |  | Property value | Assigned by |
| --- | --- | --- | --- | --- |
| name |  |  | content-hosting-configuration-a | Media Application Provider |
| ingestConfiguration |  |  |  |  |
|  | mode |  | PULL | Media Application Provider |
|  | protocol |  | urn:3gpp:‌5gms:‌content-protocol:‌http-pull |  |
|  | baseURL |  | https://origin.media-application-provider.com |  |
| distributionConfiguration |  |  |  |  |
|  | affinityGroup |  | affinity.group.a | Media Application Provider |
|  | contentPreparation TemplateId |  | cmmf.content.preparation.template |  |
|  | certificateId |  | server.certificate.a |  |
|  | canonical DomainName |  | distribution-a.com-provider-service.ms.as. 3gppservices.org | Media AF |
|  | baseURL |  | https://distribution-a.com-provider-service.ms.as. 3gppservices.org |  |
|  | entryPoint |  |  |  |
|  |  | relativePath | 5gms-media-player-entry.json | Media Application Provider |
|  |  | contentType | application/json |  |
|  | pathRewriteRule |  |  |  |
|  |  | requestPathPattern | (cmmf-a/)$ | Media Application Provider |
|  |  | mappedPath | $ |  |
| distributionConfiguration |  |  |  |  |
|  | affinityGroup |  | affinity.group.b | Media Application Provider |
|  | contentPreparation TemplateId |  | cmmf.content.preparation.template |  |
|  | certificateId |  | server.certificate.a |  |
|  | canonical DomainName |  | distribution-b.com-provider-service.ms.as. 3gppservices.org | Media AF |
|  | baseURL |  | https://distribution-b.com-provider-service.ms.as. 3gppservices.org |  |
|  | entryPoint |  |  |  |
|  |  | relativePath | 5gms-media-player-entry.json | Media Application Provider |
|  |  | contentType | application/json |  |
|  | pathRewriteRule |  |  |  |
|  |  | requestPathPattern | (cmmf-b/)$ | Media Application Provider |
|  |  | mappedPath | $ |  |
| distributionConfiguration |  |  |  |  |
|  | affinityGroup |  | affinity.group.c | Media Application Provider |
|  | contentPreparation TemplateId |  | cmmf.content.preparation.template |  |
|  | certificateId |  | server.certificate.a |  |
|  | canonical DomainName |  | distribution-c.com-provider-service.ms.as. 3gppservices.org | Media AF |
|  | baseURL |  | https://distribution-c.com-provider-service.ms.as. 3gppservices.org |  |
|  | entryPoint |  |  |  |
|  |  | relativePath | 5gms-media-player-entry.json | Media Application Provider |
|  |  | contentType | application/json |  |
|  | pathRewriteRule |  |  |  |
|  |  | requestPathPattern | (cmmf-c/)$ | Media Application Provider |
|  |  | mappedPath | $ |  |
| NOTE: The DistributionConfiguration.affinityGroup property listed above is new to the ContentHostingConfiguration resource. The Media Application Provider may assign an affinity group label indicating that reference point M4 service locations exposed by this distribution configuration are to be deployed with service locations exposed by other distribution configurations with the same affinity group label. Service locations exposed by distribution configurations with different affinity group labels are not intended to be deployed together. If this property is omitted, deployment of service locations for this distribution configuration is at the discretion of the Media AF. |  |  |  |  |

## 3.3			Streaming session configuration

### 3.3.1	Overview

A Media Player Entry in this example consists of two parts:

-	A general Media Player Entry document as specified in clause 2 that contains streaming session configuration information.

-	An MPD or HLS playlist provided by the Media Application Provider that has not been modified to include the base URLs of the distributions provisioned within the Content Hosting Configurations.

### 3.3.2	Example general Media Player Entry

For the purposes of this deployment scenario, an example realization of the general Media Player Entry specified in clause 2 is shown in figure 3.3.2-1.

Table 3.3.2-1: CMMF Media Entry Point example

{ "mediaResourceInformation": { "mediaResource": "*\n", "contentType": "text/plain" "downlinkConfiguration": [ { "serviceLocation": [ { "baseURL": "https://distribution-a.com-provider-service.ms.as. 3gppservices.org", "requestPathPattern": "$", "mappedPath": "cmmf-a/", }, { "baseURL": "https://distribution-b.com-provider-service.ms.as. 3gppservices.org", "requestPathPattern": "$", "mappedPath": "cmmf-b/", }, { "baseURL": "https://distribution-c.com-provider-service.ms.as. 3gppservices.org", "requestPathPattern": "$", "mappedPath": "cmmf-c/", }, ], "cmmfConfiguration": { "cmmfVersion": 1, "cmmfCodeType": 0, "cmmfProfile": 3gpp:5gms, "cmmfProfileDescription": "", }, }, ], }

NOTE 2:	The CMMF profile and profile description used in the above table have not yet been defined.

The example JSON document may be obtained by the 5GMSd Client from https://distribution-a.com-provider-service.ms.as.3gppservices.org/5gms-media-player-entry.json, https://distribution-b.com-provider-service.ms.as.3gppservices.org/5gms-media-player-entry.json, or https://distribution-c.com-provider-service.ms.as.3gppservices.org/5gms-media-player-entry.json.

The wildcard value for the mediaResourceInformation.mediaResource parameter indicates that the provided streaming configuration is applied to any MPD or HLS playlist nominated by the 5GMSd-Aware Application. For the purposes of this deployment scenario, it is assumed that the 5GMSd-Aware Application nominates the MPD with URL /example.mpd. In such a case, the 5GMSd Client appends this relative path to each of the serviceLocation.baseURLs listed in table 3.3.2-1 in order to obtain the selected MPD.

NOTE 3:	M7 and M11 APIs exist for DASH Media Players for the 5GMSd-Aware Application to nominate an MPD via the attach()method defined in clause 13.2.3.3 of TS 26.512 [5].

### 3.3.3	Service Access Information

While optional, the URL of the Media Player Entry document described in table 3.3.2-1 may be provided to the 5GMSd Client at reference point M5d within the Service Access Information. The Service Access Information can be obtained from the 5GMSd AF using the Service Access Information API as defined in clause 9.2 of TS 26.510. Table 3.3.3-1 provides example values for the Service Access Information API.

Table 3.3.3-1: ServiceAccessInformation resource parameters

| Property name |  |  | Example Value |
| --- | --- | --- | --- |
| provisioningSessionId |  |  | provisioning.session.a |
| provisioningSessionType |  |  | MS_DOWNLINK |
| locationReporting |  |  | false |
| streamingAccess |  |  |  |
|  | entryPoint |  |  |
|  |  | locator | https://distribution-a.com-provider-service.ms.as. 3gppservices.org/5gms-media-player-entry.json |
|  |  | contentType | application/json |
|  |  | protocol | urn:3gpp:‌5gms:‌content-protocol:‌http-pull |
|  | entryPoint |  |  |
|  |  | locator | https://distribution-b.com-provider-service.ms.as. 3gppservices.org/5gms-media-player-entry.json |
|  |  | contentType | application/json |
|  |  | protocol | urn:3gpp:‌5gms:‌content-protocol:‌http-pull |
|  | entryPoint |  |  |
|  |  | locator | https://distribution-c.com-provider-service.ms.as. 3gppservices.org/5gms-media-player-entry.json |
|  |  | contentType | application/json |
|  |  | protocol | urn:3gpp:‌5gms:‌content-protocol:‌http-pull |

### 3.3.4	Example MPD

The deployment scenario assumes that a media asset (and its associated MPD) is selected from a catalog or library of media assets. Upon selection of an asset, the URL of the associated MPD is provided to the 5GMSd Client at reference point M7d. The 5GMSd may use this URL to then obtain the MPD at reference point M4d. An example MPD is provided in table 3.3.4-1. Note that this MPD has not been updated by the 5GMSd Application Provider, nor a Content Preparation Template provisioned within the 5GMSd AS, to include the base URLs of the provisioned distributions in the Content Hosting Configuration.

Table 3.3.4-1: Example MPD

<?xml version="1.0"?> <MPD profiles="urn:3GPP:PSS:profile:DASH10" type="static" minBufferTime="PT10S" mediaPresentationDuration="PT2H" availabilityStartTime="2010-04-01T09:30:47Z" availabilityEndTime="2010-04-07T09:30:47Z" xsi:schemaLocation="urn:mpeg:dash:schema:mpd:2011 3GPP-Rel10-MPD.xsd" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:dash:schema:mpd:2011"> <ProgramInformation moreInformationURL="http://www.example.com"> <Title>Example</Title> </ProgramInformation> <Period start="PT0S"> <AdaptationSet mimeType="video/3gpp"> <ContentComponent contentType="video"/> <ContentComponent contentType="audio" lang="en"/> <Representation codecs="s263, samr" bandwidth="256000" id="256"> <BaseURL>"rep1"</BaseURL> <SegmentList duration="1000" timescale="100"> <Initialization sourceURL="seg-init.3gp"/> <SegmentURL media="seg-1.3gp"/> <SegmentURL media="seg-2.3gp"/> <SegmentURL media="seg-3.3gp"/> </SegmentList> </Representation> <Representation codecs="mp4v.20.9, mp4a.E1" bandwidth="128000" id="128"> <BaseURL>"rep2"</BaseURL> <SegmentList duration="10"> <Initialization sourceURL="seg-init.3gp"/> <SegmentURL media="seg-1.3gp"/> <SegmentURL media="seg-2.3gp"/> <SegmentURL media="seg-3.3gp"/> </SegmentList> </Representation> </AdaptationSet> </Period> <Period start="PT30S"> <SegmentTemplate duration="10" initialization="seg-init-$RepresentationId$.3gp" media="http://example.com/$RepresentationId$/$Number$.3gp"/> <AdaptationSet mimeType="video/3gpp" codecs="mp4v.20.9, mp4a.E1"> <ContentComponent contentType="video"/> <ContentComponent contentType="audio" lang="en"/> <Representation bandwidth="256000" id="1"/> <Representation bandwidth="128000" id="2"/> </AdaptationSet> </Period> </MPD>

### 3.3.5	Mapping the example MPD with the generic Media Player Entry

Once the 5GMSd Client has obtained both the generic Media Player Entry document shown in table 3.3.2-1 and the example MPD shown in table 3.3.4-1, it maps the information contained within the MPD with the information contained in the generic Media Player Entry. Table 3.3.5-1 shows a complete example of this mapping for each segment URL listed within the MPD.

Table 3.3.5-1: Example segment URL mapping

| MPD URL | Generic Media Player Entry |  |  | M4d Mapped URL |
| --- | --- | --- | --- | --- |
|  | baseURL | request Path Pattern | mapped Path |  |
| rep1/seg-init.3gp | https://distribution-a.com-provider-service.ms.as. 3gppservices.org | $ | cmmf-a/ | https://distribution-a.com-provider-service .ms.as.3gppservices.org/rep1/cmmf-a/seg-init.3gp |
| rep1/seg-1.3gp |  |  |  | https://distribution-a.com-provider-service .ms.as.3gppservices.org/rep1/cmmf-a/seg-1.3gp |
| rep1/seg-2.3gp |  |  |  | https://distribution-a.com-provider-service .ms.as.3gppservices.org/rep1/cmmf-a/seg-2.3gp |
| rep1/seg-3.3gp |  |  |  | https://distribution-a.com-provider-service .ms.as.3gppservices.org/rep1/cmmf-a/seg-3.3gp |
| rep2/seg-init.3gp |  |  |  | https://distribution-a.com-provider-service .ms.as.3gppservices.org/rep2/cmmf-a/seg-init.3gp |
| rep2/seg-1.3gp |  |  |  | https://distribution-a.com-provider-service .ms.as.3gppservices.org/rep2/cmmf-a/seg-1.3gp |
| rep2/seg-2.3gp |  |  |  | https://distribution-a.com-provider-service .ms.as.3gppservices.org/rep2/cmmf-a/seg-2.3gp |
| rep2/seg-3.3gp |  |  |  | https://distribution-a.com-provider-service .ms.as.3gppservices.org/rep2/cmmf-a/seg-3.3gp |
| seg-init-1.3gp |  |  |  | https://distribution-a.com-provider-service .ms.as.3gppservices.org/cmmf-a/seg-init-1.3gp |
| http://example.com/1/1.3gp |  |  |  | https://distribution-a.com-provider-service .ms.as.3gppservices.org/1/cmmf-a/1.3gp |
| http://example.com/1/2.3gp |  |  |  | https://distribution-a.com-provider-service .ms.as.3gppservices.org/1/cmmf-a/2.3gp |
| http://example.com/1/3.3gp |  |  |  | https://distribution-a.com-provider-service .ms.as.3gppservices.org/1/cmmf-a/3.3gp |
| seg-init-2.3gp |  |  |  | https://distribution-a.com-provider-service .ms.as.3gppservices.org/cmmf-a/seg-init-2.3gp |
| http://example.com/2/1.3gp |  |  |  | https://distribution-a.com-provider-service .ms.as.3gppservices.org/2/cmmf-a/1.3gp |
| http://example.com/2/2.3gp |  |  |  | https://distribution-a.com-provider-service .ms.as.3gppservices.org/2/cmmf-a/2.3gp |
| http://example.com/2/3.3gp |  |  |  | https://distribution-a.com-provider-service .ms.as.3gppservices.org/2/cmmf-b/3.3gp |
| rep1/seg-init.3gp | https://distribution-b.com-provider-service.ms.as. 3gppservices.org | $ | cmmf-b/ | https://distribution-b.com-provider-service .ms.as.3gppservices.org/rep1/cmmf-b/seg-init.3gp |
| rep1/seg-1.3gp |  |  |  | https://distribution-b.com-provider-service .ms.as.3gppservices.org/rep1/cmmf-b/seg-1.3gp |
| rep1/seg-2.3gp |  |  |  | https://distribution-b.com-provider-service .ms.as.3gppservices.org/rep1/cmmf-b/seg-2.3gp |
| rep1/seg-3.3gp |  |  |  | https://distribution-b.com-provider-service .ms.as.3gppservices.org/rep1/cmmf-b/seg-3.3gp |
| rep2/seg-init.3gp |  |  |  | https://distribution-b.com-provider-service .ms.as.3gppservices.org/rep2/cmmf-b/seg-init.3gp |
| rep2/seg-1.3gp |  |  |  | https://distribution-b.com-provider-service .ms.as.3gppservices.org/rep2/cmmf-b/seg-1.3gp |
| rep2/seg-2.3gp |  |  |  | https://distribution-b.com-provider-service .ms.as.3gppservices.org/rep2/cmmf-b/seg-2.3gp |
| rep2/seg-3.3gp |  |  |  | https://distribution-b.com-provider-service .ms.as.3gppservices.org/rep2/cmmf-b/seg-3.3gp |
| seg-init-1.3gp |  |  |  | https://distribution-b.com-provider-service .ms.as.3gppservices.org/cmmf-b/seg-init-1.3gp |
| http://example.com/1/1.3gp |  |  |  | https://distribution-b.com-provider-service .ms.as.3gppservices.org/1/cmmf-b/1.3gp |
| http://example.com/1/2.3gp |  |  |  | https://distribution-b.com-provider-service .ms.as.3gppservices.org/1/cmmf-b/2.3gp |
| http://example.com/1/3.3gp |  |  |  | https://distribution-b.com-provider-service .ms.as.3gppservices.org/1/cmmf-b/3.3gp |
| seg-init-2.3gp |  |  |  | https://distribution-b.com-provider-service .ms.as.3gppservices.org/cmmf-b/seg-init-2.3gp |
| http://example.com/2/1.3gp |  |  |  | https://distribution-b.com-provider-service .ms.as.3gppservices.org/2/cmmf-b/1.3gp |
| http://example.com/2/2.3gp |  |  |  | https://distribution-b.com-provider-service .ms.as.3gppservices.org/2/cmmf-b/2.3gp |
| http://example.com/2/3.3gp |  |  |  | https://distribution-b.com-provider-service .ms.as.3gppservices.org/2/cmmf-b/3.3gp |
| rep1/seg-init.3gp | https://distribution-a.com-provider-service.ms.as. 3gppservices.org | $ | cmmf-c/ | https://distribution-c.com-provider-service .ms.as.3gppservices.org/rep1/cmmf-c/seg-init.3gp |
| rep1/seg-1.3gp |  |  |  | https://distribution-c.com-provider-service .ms.as.3gppservices.org/rep1/cmmf-c/seg-1.3gp |
| rep1/seg-2.3gp |  |  |  | https://distribution-c.com-provider-service .ms.as.3gppservices.org/rep1/cmmf-c/seg-2.3gp |
| rep1/seg-3.3gp |  |  |  | https://distribution-c.com-provider-service .ms.as.3gppservices.org/rep1/cmmf-c/seg-3.3gp |
| rep2/seg-init.3gp |  |  |  | https://distribution-c.com-provider-service .ms.as.3gppservices.org/rep2/cmmf-c/seg-init.3gp |
| rep2/seg-1.3gp |  |  |  | https://distribution-c.com-provider-service .ms.as.3gppservices.org/rep2/cmmf-c/seg-1.3gp |
| rep2/seg-2.3gp |  |  |  | https://distribution-c.com-provider-service .ms.as.3gppservices.org/rep2/cmmf-c/seg-2.3gp |
| rep2/seg-3.3gp |  |  |  | https://distribution-c.com-provider-service .ms.as.3gppservices.org/rep2/cmmf-c/seg-3.3gp |
| seg-init-1.3gp |  |  |  | https://distribution-c.com-provider-service .ms.as.3gppservices.org/cmmf-c/seg-init-1.3gp |
| http://example.com/1/1.3gp |  |  |  | https://distribution-c.com-provider-service .ms.as.3gppservices.org/1/cmmf-c/1.3gp |
| http://example.com/1/2.3gp |  |  |  | https://distribution-c.com-provider-service .ms.as.3gppservices.org/1/cmmf-c/2.3gp |
| http://example.com/1/3.3gp |  |  |  | https://distribution-c.com-provider-service .ms.as.3gppservices.org/1/cmmf-c/3.3gp |
| seg-init-2.3gp |  |  |  | https://distribution-c.com-provider-service .ms.as.3gppservices.org/cmmf-c/seg-init-2.3gp |
| http://example.com/2/1.3gp |  |  |  | https://distribution-c.com-provider-service .ms.as.3gppservices.org/2/cmmf-c/1.3gp |
| http://example.com/2/2.3gp |  |  |  | https://distribution-c.com-provider-service .ms.as.3gppservices.org/2/cmmf-c/2.3gp |
| http://example.com/2/3.3gp |  |  |  | https://distribution-c.com-provider-service .ms.as.3gppservices.org/2/cmmf-c/3.3gp |

## 3.4		End-to-End procedures and request URL mapping

### 3.4.1	Procedures for downlink streaming using CMMF

Figure 3.4.1-1, which was replicated from clause 5.2.6 of TS 26.501, provides the general procedure used to provision, request, prepare, and distribute content within the deployment scenario.

Figure 3.4.1-1: High-level procedure for downlink streaming from multiple service locations within the 5GMS System

The steps describing the procedure are provided below. These steps have been altered from those provided in clause 5.2.6 of TS 26.501 to be consistent with the deployment scenario.

1.	The 5GMSd Application Provider provisions the 5GMS System at reference point M1d as described in clause 3.2. The Content Hosting Configuration includes configurations for three distributions where each exposes a service location with a different base URL at reference point M4d. The Service Announcement also provides a reference to the Service Access Information requiring Media Session Handling at reference point M5d (see step 5).

2.	The 5GMSd-Aware Application triggers the Service Announcement and Service and Content Discovery procedure. The Service and Content Discovery procedure only involves the 5GMSd-Aware Application and the 5GMSd Application Provider. The 5GMSd Application Provider communicates a catalogue or library of media assets to the 5GMSd-Aware Application where each item in the catalogue includes a URL to a MPD that has not been updated to reflect the provisioned 5GMS System configuration (e.g., service location base URLs, etc.).

3.	A media content item is selected.

4.	The 5GMSd-Aware Application triggers the 5GMSd Client to initiate the 5G Media Streaming Service.

5.	The Media Session Handler interacts with the 5GMSd AF to acquire the whole Service Access Information. The Service Access Information includes the URLs to the general Media Player Entry shown in table 3.3.2-1.

6.	The Media Session Handler provides the Media Player Entry URLs to the Media Player. For the purposes of this deployment scenario, a precedence order for these Media Player Entries does not exist.

7.	The Media Player is invoked with the selected MPD URL to start media access and playback via the attach()method defined in clause 13.2.3.3 of TS 26.512.

8.	The Media Player establishes the transport session for acquiring the general Media Player Entry and the MPD.

9.	The Media Player requests the both the general Media Player Entry shown in table 3.3.2-1 and the MPD shown in table 3.3.4-1.

10.	The Media Player receives the general Media Player Entry and the MPD.

11.	The Media Player processes the general Media Player Entry and the MPD. From the Media Player Entry, the Media Player determines the multiple service location configuration, including the locations of the available service locations where content can be accessed and the method in which it should access this content. The presence of CMMF configuration information indicates that CMMF multi-source delivery shall be used to access media resources at reference point M4d. The Media Player further determines the number of needed transport sessions for media acquisition to each service location and identifies or creates a mapping between each media resource communicated in the MPD to the request URLs that will be used at reference point M4d. This mapping is shown in table 3.3.5-1. Furthermore, the Media Player initializes the media pipelines for each media stream.

12.	The Media Player notifies the Media Session Handler about the Media Player Entry.

13.	Not applicable for this deployment scenario.

14.	The Media Player configures the media playback pipeline.

15.	The Media Player establishes the necessary transport sessions for the content according to the streaming session configuration information contained in the general Media Player Entry and the MPD. These transport sessions are established between the Media Player and each of the service locations. The Media Player may establish one transport session for each media component (audio, video, etc) and possibly additional transport sessions for other media representations to each service location.

16.	The Media Player notifies the Media Session Handler that it is ready to commence playback and optionally provides transport session parameters.

16a.	The Media Player selects an adaption set and representation from the MPD shown in table 3.3.4-1 and identifies the URL to use when requesting the media resource at reference point M4d. The 5GMSd Client then adds or replaces the media resources’ base URL with one of the service location base URLs contained in the general Media Player Entry document shown in table 3.3.2-1. It also uses the requestPathPattern parameter associated with the selected service location to identify where to insert a sub-path URL identified by the mappedPath parameter. The resulting URL is used to request a CMMF-encoded media resource from the appropriate service location exposed at reference point M4d.

17.	The Media Player requests and obtains the initialization information. This initialization information is obtained from any one service location or a combination of service locations. The Media Player repeats this step for each required initialization segment.

18.	The Media Player requests and obtains the media segments using the same method as described in step 16.a. These media segments may be obtained from any one service location or a combination of service locations. The received information is put into the appropriate media rendering pipeline.

19.	Previous steps are repeated according to the information contained in the MPD.

### 3.4.2	Media processing procedures for downlink media streaming using CMMF

The procedure used by the 5GMSd AS when handling a request receive at a service location exposed at reference point M4d is provided in figure 3.4.2-1. This figure has been replicated from figure 7.2-1 in TS 26.501.

Figure 3.4.2-1: Media processing procedures for downlink media streaming

The steps describing the procedure are provided below. These steps have been altered from those provided in clause 7.2 of TS 26.501 to be consistent with the deployment scenario.

1 - 4. 	The 5GMSd Application Provider provisions the 5GMS System according to clause 3.2. This includes provisioning a Content Preparation Template to perform CMMF encoding (see table 3.2.2-1), a Content Hosting Configuration that defines three distribution configurations where each distribution exposes a different service location at reference point M4d (see table 3.2.4-1), etc.

5.	A request for a CMMF-encoded media resource is sent by the 5GMSd Client to one or more service locations. At each service location, the received request is processed by the 5GMSd AS. The 5GMSd AS either serves the requested CMMF-encoded media resource from cache or propagates the request for the original media resource to the 5GMSd Application Provider.

6.	If the request for the CMMF-encoded media resource results in a cache miss, the request URL is processed by the upstream 5GMSd AS using the Content Hosting Configuration defined in table 3.2.4-1. The 5GMSd AS applies the PathRewriteRule associated with the service location at which the request was received in order to obtain the path URL that is used to request the media resource at reference point M2d. The media resource is then obtained from the 5GMSd Application Provider at reference point M2d. The 5GMSd AS may cache the media resource for future processing and delivery to another 5GMSd Client.

7.	Content Preparation Template cmmf.content.preparation.template as described in clause 3.2.2 is applied to the media resource to produce the appropriate CMMF-encoded “version” or “stripe” of the media resource associated with the service location serving the request. The 5GMSd AS may cache the CMMF-encoded media resource for future delivery to another 5GMSd Client.

8.	The 5GMSd AS responds to the M4d request from the service location where the request was received with the appropriate CMMF-encoded “version” or “stripe” of the media resource. Once the 5GMSd Client obtains enough CMMF-encoded data from all active requests for different “versions” or “stripes” of the same desired media resource, the 5GMSd Client decodes and recovers the desired media resource associated with the original URL communicated within the MPD.

### 3.4.3	End-to-end URL mapping

Table 3.4.3-1 provides an example of the end-to-end mapping for requests initiated by the Media Player for a subset of the URLs provided in the example MPD shown in table 3.3.4-1.

Table 3.4.3-1: End-to-End URL mapping example

| MPD URL | M4d Request URL | M2d Request URL |
| --- | --- | --- |
| rep1/seg-1.3gp | https://distribution-a.com-provider-service .ms.as.3gppservices.org/rep1/cmmf-a/seg-1.3gp | https://origin.media- application-provider.com/ rep1/seg-1.3gp |
|  | https://distribution-b.com-provider-service .ms.as.3gppservices.org/rep1/cmmf-b/seg-1.3gp |  |
|  | https://distribution-c.com-provider-service .ms.as.3gppservices.org/rep1/cmmf-c/seg-1.3gp |  |
| http://example.com/1/1.3gp | https://distribution-a.com-provider-service .ms.as.3gppservices.org/1/cmmf-a/1.3gp | https://origin.media- application-provider.com/ 1/1.3gp |
|  | https://distribution-b.com-provider-service .ms.as.3gppservices.org/1/cmmf-b/1.3gp |  |
|  | https://distribution-c.com-provider-service .ms.as.3gppservices.org/1/cmmf-c/1.3gp |  |

# 4	Implementation Scenario 2: 5GMSd AS ingest, content preparation, and delivery using 5GMSd AS service chaining at reference point M10d.

The second implementation scenario shows how CMMF can be used to enable the deployment example provided in annex A.16.3 of TS 26.501. This scenario assumes the 5GMSd Application Provider provisions the system in the following manner:

1.	A 5GMSd AS is provisioned to serve as an origin shield intended to reduce requests for content at reference point M2d. Furthermore, the creation of CMMF objects from ingested content at reference point M2d using Content Preparation is performed within this 5GMSd AS.

2.	Two 5GMSd AS’s are provisioned to serve 5GMSd Clients from exposed service locations at reference point M4d. Both 5GMSd AS’s are configured to ingest CMMF encoded content (unique to their exposed service locations) from the 5GMSd AS serving as the origin shield at reference M10d.

3.	Different media assets (e.g., described by a DASH MPD or HLS playlist) are presented to the 5GMSd-Aware Application in the form of a content library. After selection of one media asset, the MPD or HLS playlist is provided to the 5GMSd Client from the 5GMSd Application Provider without modification (i.e., baseURLs are not added to MPD to communicate the location of 5GMSd AS service locations exposed at reference point M4d, nor is any CMMF configuration information embedded within these documents). The MPD or HLS playlist may be obtained by the 5GMSd Client via either reference point M8d or reference point M4d. Furthermore, each MPD or HLS playlist describes different content types (e.g., video, audio, subtitles, etc.) required to be obtained by the 5GMSd Client during a streaming session.

4.	A general Media Player Entry document, as specified in clause 2, is created (either by the 5GMSd Application Provider or by the 5GMSd AS using a Content Preparation Template) to communicate streaming session configuration information (e.g., 5GMSd AS service location URLs, CMMF configuration information, etc.). This Media Player Entry document, along with the selected DASH MPD or HLS playlist, is used by the 5GMSd Client to initiate a streaming session.

6.	The URL of this general Media Player Entry document may be communicated to the 5GMSd Client via reference point M8d or within the Service Access Information at reference point M5d.

The implementation scenario is illustrated in figure 4-1 which is a labelled version of figure A.16.3-1 in TS 26.501. For the purposes of this example, it will be assumed that:

-	The Media Application Provider creates three Provisioning Sessions. Two in the Trusted DN 5GMS AS and one in the External DN 5GMSd AS.

-	The Provisioning Session in the Trusted DN 5GMSd AS (labelled “Provisioning Session (PS) A” ) will serve as the origin server, in addition to being responsible for creating CMMF objects from content ingested at reference point M2d.

-	Two Provisioning Sessions in the Trusted DN 5GMSd AS and the External DN 5GMSd AS are responsible for distributing CMMF-encoded content to 5GMSd Clients from service locations exposed at reference point M4d. The former is labelled “PS B” and the latter is labelled “PS C”. PS B distributes one “version” or “stripe” of the CMMF-encoded content (labelled “CMMF-B”), and PS C distributes a second “version” or “stripe of the CMMF-encoded content (labelled “CMMF-C”).

-	The URL of the general Media Player Entry is communicated to the 5GMSd Client within the Service Access Information available at reference point M5d.

-	Pull-based protocols are used to deliver media resource from the 5GMSd Application Provider to 5GMSd Clients at reference points M2d, M10d and M4d.

-	The general Media Player Entry is obtained by the 5GMSd Client at reference point M4d from PS B.

-	The MPD URL is obtained from the 5GMSd Application Provider via reference point M8d as a relative path to the MPD resource. This relative path is combined with a base URL parameter provided in the general Media Player Entry document.

-	The MPD is obtained from the 5GMSd Application Provider via reference points M2d and M4d without modification (e.g., baseURLs are not added to the MPD listing 5GMSd AS service locations, etc.).

-	For the purposes of this example, it is assumed that the MPD resource describes segmented media (i.e., each segment of video, audio, etc. is uniquely addressable).

Media Player Entry and MPD

MPD URL

CMMF-C

CMMF-B

PS C

PS B

PS A

Figure 4-1: Centralized 5GMSd AS content preparation and ingest deployment scenario

The remainder of this clause provides an implementation of the 5GMS APIs and protocols to realize the implementation scenario.

## 4.2			Provisioning and Content Hosting Configurations

The procedures for provisioning the Content Hosting Configurations follow those specified in clauses 5.3, 5.4, and 7 of TS 26.501. This clause provides example values used to configure the three Provisioning Sessions to realize the implementation scenario.

### 4.2.1	Provisioning Session provisioning and configuration

To configure the three 5GMS AS’s, three provisioning sessions are created by the 5GMSd Application Provider using the create Provisioning Session resource operation specified in clause 5.2.2.3 of TS 26.510 and the API specified in clause 8.2 of TS 26.510, one for each 5GMS AS. In the interest of space, example values for the three provisioning session API parameters used are shown in a single table, table 4.2.1-1.

Table 4.2.1-1: ProvisioningSession resource parameters

| Property name | PS A (Trusted DN) | PS B (Trusted DN) | PS C (External DN) | Assigned by |
| --- | --- | --- | --- | --- |
| provisioningSessionId | provisioning. session.a | provisioning. session.b | provisioning. session.c | Media AF |
| provisioningSession Type | MS_DOWNLINK | MS_DOWNLINK | MS_DOWNLINK | Media Application Provider |
| externalServiceId | com.‌provider.‌service.a | com.‌provider.‌service.b | com.‌provider.‌service.c |  |
| appId | dash.downlink. streaming | dash.downlink. streaming | dash.downlink. streaming |  |
| NOTE: Values for provisioningSessionId, externalServiceId, and appId are notional and may not reflect actual values. |  |  |  |  |

### 4.2.2	Content Preparation Templates provisioning and configuration

The Media Application Provider provisions the CMMF encoder Content Preparation Template within the provisioning.session.a Provisioning Session using the create Content Preparation Template resource operation specified in clause 5.2.5.2 of TS 26.510 and the API defined in clause 8.5 of TS 26.510. While the CMMF encoder Content Preparation Template has yet to be defined, assume that it has the following functionality:

-	Content Preparation is triggered upon the receipt of a pull-based request at reference point M10d.

-	The path of the requested resource is available to the Content Preparation Template.

-	Source content (e.g., original video, audio, etc.) ingested by the Trusted DN 5GMS AS at reference point M2d is encoded by the Content Preparation Template within a CMMF object. A sub-path URL added by the 5GMSd Client into the M4d request URL is used to determine which CMMF “version” or “stripe” is generated. For the purposes of this example, the Content Preparation Template will generate the CMMF-B “version” or “stripe” if a M4d request URL containing the sub-path cmmf-b is received; and it will generate the CMMF-C “version” or “stripe” if the URL contains the sub-path cmmf-c is received.

-	Upon completion of the CMMF encoding operation, the CMMF object is made available to the 5GMS AS for caching and/or delivery to the requesting entity.

Upon successful provisioning of the Content Preparation Template, the Media AF provides the cmmf.content.preparation.template Content Preparation Template ID to the Media Application Provider.

### 4.2.3	Server Certificates provisioning and configuration

The Media Application Provider provisions the Server Certificates for each Provisioning Session using the create Server Certificate resource operation specified in clause 5.2.4.2 of TS 26.510 and the API defined in clause 8.4 of TS 26.510. Example values for the certificates generated by the Media AF are provided in table 4.2.3-1.

Table 4.2.3-1: Server Certificates

| provisioningSessionId | CN | subjectAltName | certificateId |
| --- | --- | --- | --- |
| provisioning.session.a | *.com-provider-service-a. ms.as.3gppservices.org | *.com-provider-service-a. ms.as.3gppservices.org | server. certificate.a |
| provisioning.session.b | *.com-provider-service-b. ms.as.3gppservices.org | *.com-provider-service-b. ms.as.3gppservices.org | server. certificate.b |
| provisioning.session.c | *.com-provider-service-c. ms.as.3gppservices.org | *.com-provider-service-c. ms.as.3gppservices.org | server. certificate.c |
| NOTE: While not applicable to this example, the canonical 5GMS AS authority at reference point M4 defined in clause 6.0.2.2 of TS 26.512 needs to be extended to include a distribution ID in order to differentiate between more than one content distributions defined within a single Content Hosting Configuration. As an example, the canonical domain name for service locations exposed at M4d will have the following canonical domain name: {distributionId}.{modifiedExternalServiceId}.ms.as.3gppservices.org. |  |  |  |

### 4.2.4	Content Hosting provisioning and configuration

The Media Application Provider provisions the Content Hosting Configuration for each Provisioning Session using the create Content Hosting Configuration resource operation specified in clause 5.2.8.2 of TS 26.510 and the API defined in clause 8.8 of TS 26.510. Provisioning the Content Hosting Configuration in each 5GMS AS is performed according to:

-	The example base URL of the Media Application Provider’s origin server is https://origin.media-application-provider.com.

-	The creation of the Content Hosting Configuration for Provisioning Session provisioning.session.a is completed prior to the creation of the Content Hosting Configuration for Provisioning Sessions provisioning.session.b and provisioning.session.c.

-	The general Media Player Entry document is available at 5gms-media-player-entry.json. Furthermore, the Media Player Entry document has a MIME content type application/json.

Table 4.2.4-1 provides example values for the Content Hosting Configuration API parameters for all three Provisioning Sessions.

Table 4.2.4-1: ContentHostingConfiguration resource parameters

| Property name |  |  | PS A |  | PS B |  | PS C |  | Assigned by |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| name |  |  | content-hosting-configuration-a |  | content-hosting-configuration-b |  | content-hosting-configuration-c |  | Media Application Provider |
| ingestConfiguration |  |  |  |  |  |  |  |  |  |
|  | mode |  |  | PULL |  | PULL |  | PULL | Media Application Provider |
|  | protocol |  |  | urn:3gpp:‌5gms:‌content-protocol:‌http-pull |  | urn:3gpp:‌5gms:‌content-protocol:‌http-pull |  | urn:3gpp:‌5gms:‌content-protocol:‌http-pull |  |
|  | baseURL |  |  | https://origin. media-application-provider.com |  | https:// distribution-a .com-provider-service-a.ms.as. 3gppservices.org |  | https:// distribution-a. com-provider-service-a.ms.as. 3gppservices.org |  |
|  | distributionConfiguration |  |  |  |  |  |  |  |  |
|  | contentPreparation TemplateId |  |  | cmmf.content. preparation. template |  |  |  |  | Media Application Provider |
|  | certificateId |  |  | server. certificate.a |  | server. certificate.b |  | server. certificate.c |  |
|  | canonical DomainName |  |  | distribution-a. com-provider-service-a.ms.as. 3gppservices.org |  | distribution-a.com-provider-service-b.ms.as. 3gppservices.org |  | distribution-a.com-provider-service-c.ms.as. 3gppservices.org | Media AF |
|  | baseURL |  |  | https://distribution-a.com-provider-service-a.ms.as. 3gppservices.org |  | https://distribution-a.com-provider-service-b.ms.as. 3gppservices.org |  | https://distribution-a.com-provider-service-c.ms.as. 3gppservices.org |  |
|  | entryPoint |  |  |  |  |  |  |  |  |
|  |  | relativePath |  |  |  | 5gms-media-player-entry.json |  |  | Media Application Provider |
|  |  | contentType |  |  |  | application/json |  |  |  |
|  | pathRewriteRule |  |  |  |  |  |  |  |  |
|  |  | requestPathPattern |  | (cmmf-[a-c]/)$ |  |  |  |  | Media Application Provider |
|  |  | mappedPath |  | $ |  |  |  |  |  |
| NOTE: The DistributionConfiguration.affinityGroup property is not defined in any of the Content Hosting Configurations listed above since only one DistributionConfiguration is defined for each Content Hosting Configuration within this example. |  |  |  |  |  |  |  |  |  |

## 4.3			Streaming session configuration

### 4.3.1	Overview

A Media Player Entry in this example consists of two parts:

-	A general Media Player Entry document using the schema specified in clause 2 that contains streaming session configuration information.

-	An MPD or HLS playlist provided by the Media Application Provider that has not been modified to include the base URLs of the distributions provisioned within the Content Hosting Configurations.

### 4.3.2	Example general Media Player Entry

For the purposes of this deployment scenario, an example realization of the above Media Player Entry is shown in figure 4.3.2-1.

Table 4.3.2-1: CMMF Media Entry Point example

{ "mediaResourceInformation": { "mediaResource": "*\n", "contentType": "text/plain" "downlinkConfiguration": [ { "serviceLocation": [ { "baseURL": "https://distribution-a.com-provider-service-b.ms.as. 3gppservices.org", "requestPathPattern": "$", "mappedPath": "cmmf-b/", }, { "baseURL": "https://distribution-a.com-provider-service-c.ms.as. 3gppservices.org", "requestPathPattern": "$", "mappedPath": "cmmf-c/", }, ], "cmmfConfiguration": { "cmmfVersion": 1, "cmmfCodeType": 0, "cmmfProfile": 3gpp:5gms, "cmmfProfileDescription": "", }, }, ], }

NOTE 2:	The CMMF profile and profile description used in the above table have not yet been defined.

The example JSON document may be obtained by the 5GMSd Client from https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/5gms-media-player-entry.json.

The wildcard value for the mediaResourceInformation.mediaResource parameter indicates that the provided streaming configuration is applied to any MPD or HLS playlist nominated by the 5GMSd-Aware Application. For the purposes of this deployment scenario, it is assumed that the 5GMSd-Aware Application nominates the MPD with URL /example.mpd. In such a case, the 5GMSd Client appends this relative path to either of the serviceLocation.baseURLs listed in table 4.3.2-1 in order to obtain the selected MPD.

NOTE 3:	M7 and M11 APIs exist for DASH Media Players that allow the 5GMSd-Aware Application to nominate an MPD via the attach()method defined in clause 13.2.3.3 of TS 26.512.

### 4.3.3	Service Access Information

While optional, the URL of the Media Player Entry document described in table 4.3.2-1 may be provided to the 5GMSd Client at reference point M5d within the Service Access Information. The Service Access Information can be obtained from the 5GMSd AF using the Service Access Information API as defined in clause 9.2 of TS 26.510. Table 4.3.2-1 provides example values for the Service Access Information API.

Table 4.3.3-1: ServiceAccessInformation resource parameters

| Property name |  |  | Example Value |
| --- | --- | --- | --- |
| provisioningSessionId |  |  | provisioning.session.b |
| provisioningSessionType |  |  | MS_DOWNLINK |
| locationReporting |  |  | false |
| streamingAccess |  |  |  |
|  | entryPoint |  |  |
|  |  | locator | https://distribution-a.com-provider-service-b.ms.as. 3gppservices.org/5gms-media-player-entry.json |
|  |  | contentType | application/json |
|  |  | protocol | urn:3gpp:‌5gms:‌content-protocol:‌http-pull |

### 4.3.4	Example MPD

The deployment scenario assumes that a media asset (and its associated MPD) is selected from a catalog or library of media assets. Upon selection of an asset, the URL of the associated MPD is provided to the 5GMSd Client at reference point M7d. The 5GMSd may use this URL to then obtain the MPD at reference point M4d. An example MPD is provided in table 4.3.4-1. Note that this MPD has not been updated by the 5GMSd Application Provider, nor a Content Preparation Template provisioned within the 5GMSd AS, to include the base URLs of the provisioned distributions in the Content Hosting Configuration.

Table 4.3.4-1: Example MPD

<?xml version="1.0"?> <MPD profiles="urn:3GPP:PSS:profile:DASH10" type="static" minBufferTime="PT10S" mediaPresentationDuration="PT2H" availabilityStartTime="2010-04-01T09:30:47Z" availabilityEndTime="2010-04-07T09:30:47Z" xsi:schemaLocation="urn:mpeg:dash:schema:mpd:2011 3GPP-Rel10-MPD.xsd" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:dash:schema:mpd:2011"> <ProgramInformation moreInformationURL="http://www.example.com"> <Title>Example</Title> </ProgramInformation> <Period start="PT0S"> <AdaptationSet mimeType="video/3gpp"> <ContentComponent contentType="video"/> <ContentComponent contentType="audio" lang="en"/> <Representation codecs="s263, samr" bandwidth="256000" id="256"> <BaseURL>"rep1"</BaseURL> <SegmentList duration="1000" timescale="100"> <Initialization sourceURL="seg-init.3gp"/> <SegmentURL media="seg-1.3gp"/> <SegmentURL media="seg-2.3gp"/> <SegmentURL media="seg-3.3gp"/> </SegmentList> </Representation> <Representation codecs="mp4v.20.9, mp4a.E1" bandwidth="128000" id="128"> <BaseURL>"rep2"</BaseURL> <SegmentList duration="10"> <Initialization sourceURL="seg-init.3gp"/> <SegmentURL media="seg-1.3gp"/> <SegmentURL media="seg-2.3gp"/> <SegmentURL media="seg-3.3gp"/> </SegmentList> </Representation> </AdaptationSet> </Period> <Period start="PT30S"> <SegmentTemplate duration="10" initialization="seg-init-$RepresentationId$.3gp" media="http://example.com/$RepresentationId$/$Number$.3gp"/> <AdaptationSet mimeType="video/3gpp" codecs="mp4v.20.9, mp4a.E1"> <ContentComponent contentType="video"/> <ContentComponent contentType="audio" lang="en"/> <Representation bandwidth="256000" id="1"/> <Representation bandwidth="128000" id="2"/> </AdaptationSet> </Period> </MPD>

### 4.3.5	Mapping the example MPD with the generic Media Player Entry

Once the 5GMSd Client has obtained both the generic Media Player Entry document shown in table 4.3.2-1 and the example MPD shown in table 4.3.4-1, it maps the information contained within the MPD with the information contained in the generic Media Player Entry. Table 4.3.5-1 shows a complete example of this mapping for each segment URL listed within the MPD.

Table 4.3.5-1: Example segment URL mapping

| MPD URL | Generic Media Player Entry |  |  | M4d Mapped URL |
| --- | --- | --- | --- | --- |
|  | baseURL | request Path Pattern | mapped Path |  |
| rep1/seg-init.3gp | https://distribution-a.com-provider-service-b.ms.as. 3gppservices.org | $ | cmmf-b/ | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/rep1/cmmf-b/seg-init.3gp |
| rep1/seg-1.3gp |  |  |  | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/rep1/cmmf-b/seg-1.3gp |
| rep1/seg-2.3gp |  |  |  | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/rep1/cmmf-b/seg-2.3gp |
| rep1/seg-3.3gp |  |  |  | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/rep1/cmmf-b/seg-3.3gp |
| rep2/seg-init.3gp |  |  |  | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/rep2/cmmf-b/seg-init.3gp |
| rep2/seg-1.3gp |  |  |  | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/rep2/cmmf-b/seg-1.3gp |
| rep2/seg-2.3gp |  |  |  | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/rep2/cmmf-b/seg-2.3gp |
| rep2/seg-3.3gp |  |  |  | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/rep2/cmmf-b/seg-3.3gp |
| seg-init-1.3gp |  |  |  | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/cmmf-b/seg-init-1.3gp |
| http://example.com/1/1.3gp |  |  |  | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/1/cmmf-b/1.3gp |
| http://example.com/1/2.3gp |  |  |  | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/1/cmmf-b/2.3gp |
| http://example.com/1/3.3gp |  |  |  | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/1/cmmf-b/3.3gp |
| seg-init-2.3gp |  |  |  | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/cmmf-b/seg-init-2.3gp |
| http://example.com/2/1.3gp |  |  |  | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/2/cmmf-b/1.3gp |
| http://example.com/2/2.3gp |  |  |  | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/2/cmmf-b/2.3gp |
| http://example.com/2/3.3gp |  |  |  | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/2/cmmf-b/3.3gp |
| rep1/seg-init.3gp | https://distribution-a.com-provider-service-c.ms.as. 3gppservices.org | $ | cmmf-c/ | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/rep1/cmmf-c/seg-init.3gp |
| rep1/seg-1.3gp |  |  |  | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/rep1/cmmf-c/seg-1.3gp |
| rep1/seg-2.3gp |  |  |  | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/rep1/cmmf-c/seg-2.3gp |
| rep1/seg-3.3gp |  |  |  | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/rep1/cmmf-c/seg-3.3gp |
| rep2/seg-init.3gp |  |  |  | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/rep2/cmmf-c/seg-init.3gp |
| rep2/seg-1.3gp |  |  |  | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/rep2/cmmf-c/seg-1.3gp |
| rep2/seg-2.3gp |  |  |  | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/rep2/cmmf-c/seg-2.3gp |
| rep2/seg-3.3gp |  |  |  | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/rep2/cmmf-c/seg-3.3gp |
| seg-init-1.3gp |  |  |  | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/cmmf-c/seg-init-1.3gp |
| http://example.com/1/1.3gp |  |  |  | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/1/cmmf-c/1.3gp |
| http://example.com/1/2.3gp |  |  |  | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/1/cmmf-c/2.3gp |
| http://example.com/1/3.3gp |  |  |  | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/1/cmmf-c/3.3gp |
| seg-init-2.3gp |  |  |  | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/cmmf-c/seg-init-2.3gp |
| http://example.com/2/1.3gp |  |  |  | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/2/cmmf-c/1.3gp |
| http://example.com/2/2.3gp |  |  |  | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/2/cmmf-c/2.3gp |
| http://example.com/2/3.3gp |  |  |  | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/2/cmmf-c/3.3gp |

## 4.4		End-to-End procedures and request URL mapping

### 4.4.1	Procedures for downlink streaming using CMMF

The procedures for downlink streaming using CMMF largely follow those described in clause 3.4.1.

### 4.4.2	Media processing procedures for downlink media streaming using CMMF

Figure 4.4.2-1, which was replicated from annex A.16.3 of TS 26.501, provides the general procedure used to request, prepare, and distribute content within the deployment example.

Figure 4.4.2-1: Procedure for centralized 5GMSd AS content preparation and ingest

The steps describing the procedure are provided below. These steps have been altered from those provided in annex A.16.3 to be consistent with the deployment scenario.

1.	The Media Player selects an adaption set, representation, and segment from the MPD shown in table 4.3.4-1 and identifies the URL to use when requesting the media resource at reference point M4d. The 5GMSd Client then adds or replaces the media resources’ base URL with one of the service location base URLs contained in the general Media Player Entry document shown in table 4.3.2-1. It also uses the requestPathPattern parameter associated with the selected service location to identify where to insert a sub-path URL identified by the mappedPath parameter. The resulting URL is used to request a CMMF-encoded media resource from either the Trusted DN 5GMSd AS or the External DN 5GMSd AS at reference point M4d.

2.	The received request is processed by the appropriate downstream 5GMSd AS using either Content Hosting Configuration content-hosting-configuration-b or content-hosting-configuration-c depending on where the request was sent. The downstream 5GMSd AS either serves the requested CMMF-encoded media resource from cache or propagates the request to the upstream 5GMSd AS at reference point M10d. The path URL is not altered by the downstream 5GMSd AS between reference points M4d and M10d since both Content Hosting Configurations did not define a PathRewriteRule.

3.	The received request is processed by the upstream 5GMSd AS using the Content Hosting Configuration defined within the provisioning.session.c Provisioning Session. The 5GMSd AS applies the PathRewriteRule defined in table 4.2.4-1 for the Content Hosting Configuration labelled content-hosting-configuration-a to the path URL in order to obtain the path URL that is used to request the media resource at reference point M2d. The 5GMSd AS determines if the media resource can be served from cache (the description of CachingConfiguration.urlPatternFilter in table 8.8.3.3 of TS 26.510 indicates that the media resource is cached in the form ingested at reference point M2d). If not, the media resource is requested from the 5GMSd Application Provider at reference point M2d.

4.	The 5GMSd Application Provider responds with the requested media resource.

5.	Content Preparation Template cmmf.content.preparation.template is applied to the media resource to produce the CMMF-encoded media resource requested at M10d.

6.	The upstream 5GMSd AS responds to the M10d request with the appropriate CMMF-encoded “version” or “stripe” of the media resource. Upon receipt of this CMMF-encoded media resource, the downstream 5GMSd AS may cache it for future delivery to another 5GMSd Client.

7.	The downstream 5GMSd AS responds to the M4d request with the appropriate CMMF-encoded “version” or “stripe” of the media resource. Once the 5GMSd Client obtains enough CMMF-encoded data from all active requests for different “versions” or “stripes” of the same desired media resource, the 5GMSd Client decodes and recovers the desired media resource associated with the original URL communicated within the MPD.

### 4.4.3	End-to-end URL mapping

Table 4.4.3-1 provides an example of the end-to-end mapping for requests initiated by the Media Player for a subset of the URLs provided in the example MPD shown in table 4.3.4-1.

Table 4.4.3-1: End-to-End URL mapping example

| MPD URL | M4d Request URL | M10d Request URL | M2d Request URL |
| --- | --- | --- | --- |
| rep1/seg-1.3gp | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/ rep1/cmmf-b/seg-1.3gp | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/ rep1/cmmf-b/seg-1.3gp | https://origin.media- application-provider.com/ rep1/seg-1.3gp |
|  | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/ rep1/cmmf-c/seg-1.3gp | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/ rep1/cmmf-c/seg-1.3gp |  |
| http://example.com/1/1.3gp | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/ 1/cmmf-b/1.3gp | https://distribution-a.com-provider-service-b.ms.as.3gppservices.org/ 1/cmmf-b/1.3gp | https://origin.media- application-provider.com/ 1/1.3gp |
|  | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/ 1/cmmf-c/1.3gp | https://distribution-a.com-provider-service-c.ms.as.3gppservices.org/ 1/cmmf-c/1.3gp |  |

# 5	Identified 5GMS stage-3 required updates

The following updates to 5GMS stage-3 specifications were identified:

1.	The specification of a high-level/meta Media Player Entry is needed to communicate streaming session configuration information that is not otherwise supported in “traditional” Media Player Entry documents (e.g., MPDs, HLS playlists, URLs to video clips, etc.). A proposed Media Player Entry is described in clause 2 that supports the communication of CMMF related configuration information. Furthermore, the proposed schema can be easily extended to include other information such as content steering information, DRM information, etc. 

It is proposed to define a new Media Player Entry schema within an annex of TS 26.512, in addition to providing examples of different types of Media Player Entry documents.

2.	The canonical 5GMS AS authority at reference point M4 defined in clause 6.0.2.2 of TS 26.512 needs to be extended to include a distribution ID to differentiate between more than one content distribution defined within a single Content Hosting Configuration. It is proposed that the canonical domain name for service locations exposed at M4 have the following domain name:

{distributionId}.{modifiedExternalServiceId}.ms.as.3gppservices.org

NOTE 1:	A new distributionId parameter may also need to be defined within the ContentHostingConfiguration.DistributionConfiguration resource specified in table 8.8.3.1-1 of TS 26.510.

It is proposed to extend the canonical 5GMS AS authority defined in TS 26.512 to include a distribution ID and define a new distributionId parameter within the ContentHostingConfiguration.DistributionConfiguration resource specified in TS 26.510.

3.	A new parameter (affinityGroup) in the DistributionConfiguration resource of the Content Hosting Configuration was introduced allowing the 5GMSd Application Provider to influence the physical deployment of service locations exposed by the 5GMSd AS at reference point M4d. The introduction of this new parameter has already been captured in both clause 5.19 of TS 26.804 [6] and TS 26.501. 

No new changes to TS 26.510 nor TS 26.512 that have not already been identified and approved are necessary.

4.	In the deployment scenario described in clause 4, the 5GMS System was explicitly setup to use reference point M10d. Protocols applicable to this reference point need to be described in TS 26.510 and TS 26.512. This includes, at a minimum, downlink pull-based protocols, although downlink push-based and uplink protocols may also be required.

It is proposed to define, at a minimum, downlink pull-based protocols for reference point M10d. Further discussion is required to determine if other protocols should also be defined.

NOTE 2:	Defining downlink push-based protocols at reference point M10d may require updates to the Content Hosting Configuration API and ContentHostingConfiguration resource specified in TS 26.510. For example, the Content Hosting Configuration will need to support configuring a distribution to push media resources to a downstream entity (e.g., a downstream 5GMSd AS).

NOTE 3:	Defining uplink pull-based and push-based protocols at reference point M10 will require defining reference point M10u in TS 26.501. It may also require updates to the Content Publishing API and ContentPublishingConfiguration resource specified in TS 26.510.

5.	Clauses 3.2.2 and 4.2.2 assumed that creation of CMMF-encoded media resources were possible via a provisioned Content Preparation Template. However, neither a CMMF profile nor a Content Preparation Template has been specified within a 3GPP specification or elsewhere to do this processing. Both a CMMF profile and a Content Preparation Template need to be defined.

It is proposed to define both a CMMF profile and a Content Preparation Template for the purposes of streaming within the 5GMS System within one or more annexes to TS 26.512.

6.	While not discussed above, the collaboration scenario described in annex A.16.4 of TS 26.501 (decentralized 5GMSd AS content preparation and ingest) cannot currently be achieved without additional changes to TS 26.510 and/or TS 26.512 that have not already been identified. The collaboration scenario either requires the ability for a Content Hosting Configuration to ingest media from more than one location at reference points M2d and M10d and/or the acknowledgement that a distribution defined within a Content Hosting Configuration can ingest media resources cached in distributions also defined within the same Content Hosting Configuration. 

In the case of the former, the ContentHostingConfiguration resource only allows the Media Application Provider to define one media ingest endpoint within a Content Hosting Configuration. A proposed solution is to change the IngestConfiguration resource so that it is an array of IngestConfiguration resources similar to how the DistributionConfiguration resource is defined.

For the latter, two options have been identified. The first involves clarifying within clause 5.2.8 of TS 26.510 that one distribution may obtain media resources cached within other distributions defined within the same Content Hosting Configuration. The second involves adding new parameters to the DistributionConfiguration resource specified in table 8.8.3.1-1 of TS 26.510 that allows the Media Application Provider to link distributions defined in the same Content Hosting Configuration together. 

The proposed path forward requires further discussion.

# 6	Summary

This document provided two complete end-to-end examples showing how to provision and stream media using CMMF within the 5GMS System. Throughout these examples, features that are required but not yet supported within the 5GMS specifications were highlighted. For each of these features, a way forward is proposed to add these features within TS 26.510 and TS 26.512. Finally, a schema was also proposed for a general Media Player Entry document that can be used to supplement existing, traditional Media Player Entry documents (e.g., MPDs, HLS playlists, URL to a video clip, etc.) for the purposes of communicating streaming session configuration information that may not otherwise be supported.

# 7	Proposal

Approve implementation of items 1, 2, 4, and 5 listed in clause 5 within TS 26.510 and TS 26.512.

# 8	References

[1] 	ETSI TS 103 973: "Coded Multisource Media Format (CMMF) for Content Distribution and Delivery", October 2024.

[2] 	ETSI TS 103 998: "Publicly Available Specification (PAS); DASH-IF: Content Steering for DASH", January 2024.

[3]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[4] 	3GPP TS 26.510: "Media delivery; interactions and APIs for provisioning and media session handling".

[5] 	3GPP TS 26.512: "5G Media Streaming (5GMS) Protocols".

[6]	3GPP TS 26.804: "Study on 5G media streaming extensions".