3GPP TSG SA4-e (AH) MBS SWG post 131-bis-e	S4aI250080

Online, 7 May 2025

Agenda item: 	2.3

Source: 	Qualcomm Germany

Title: 	On NTN intended service area in the Service Announcement

Document for	Agreement

# Introduction

During SA4#131-bis-e, 3GPP SA4 received a straightforward LS from RAN2 in S4-250491

RAN2 is working on a solution to provide UEs with geographical information of where within an NTN cell an MBS broadcast service is applicable. This area is called the “intended service area” (ISA) whose geographical definition is provided to the UE via SIB and is associated with an identifier (ISA ID). The attached CR includes the changes introduced by SA2 to adopt the ISA for NTN access.

RAN2 would like to ask whether including the ISA in the Service Announcement would impact in SA4 specifications and would kindly request SA4 to confirm this can be supported within the Rel-19 timeframe.

RAN2 kindly request SA4 to confirm whether the inclusion of the ISA in the Service Announcement can be supported within the Rel-19 timeframe.

Meanwhile, questions to SA2 and RAN2 have been compiled by SA4 in a draft reply in S4aI250077:

1.	Please clarify which Cell Ids/TAIs to include in the MBS User Service Announcement for NR NTN. SA4 assumes that Cell Ids/TAIs are obtained from local configuration within MBSF/NEF.

2.	Please clarify whether an AF (internal or external) can select whether to use NR NTN for the MBS Broadcast Session or not.

3.	Please clarify how to obtain the Intended Service Area, when Cell Ids/TAIs are used by the AF for MBS Service Area description.

4.	Please clarify whether ISA ID is obtainable from RAN so that the local configuration can also contain the ISA IDs for a given MBS Service Area.

# Analysis

After consultation with RAN2 and SA2 experts, the following answers to the above questions are considered.

1.	Please clarify which Cell Ids/TAIs to include in the MBS User Service Announcement for NR NTN. SA4 assumes that Cell Ids/TAIs are obtained from local configuration within MBSF/NEF.

o	Cell IDs and TAIs in NTN are no different to what is there in TN. The MBS Service Area parameter defined in TS 29.571 includes this information and is included in TS 26.517. Clause 5.9.4.4 of TS 29.571 defines the MbsServiceArea

o	Support of ISA and ISA ID is totally optional, and UE may not support it and network has no idea about it, then the cell IDs, TAI must be present for those UEs who do not support ISA as legacy behaviour. ISA is just additional information to assist UE for power saving in searching/scanning. It does not mean we can cut off any existing information. The intendedServAreaList is actually already present in MbsServiceArea type.

2.	Please clarify whether an AF (internal or external) can select whether to use NR NTN for the MBS Broadcast Session or not.

o	There is no change to selection procedure. If the MBS service is needed in a particular area and that area is served by NTN, then AF chooses NTN.

3.	Please clarify how to obtain the Intended Service Area, when Cell Ids/TAIs are used by the AF for MBS Service Area description.

o	The mapped cell ID would be determined by NG-RAN so the AF can keep using existing cell IDs as in TN.

o	The AF can continue to use regular cell ID as AF sends service announcement to UE and UE does not understand Mapped Cell ID. The AMF can convert the regular cell ID to Mapped cell ID when it sends MBS Session Setup message to RAN. How AF knows ISA, is left to AF’s OAM configuration of ISA or Cell ID or TAI.

4.	Please clarify whether ISA ID is obtainable from RAN so that the local configuration can also contain the ISA IDs for a given MBS Service Area.

o	Note that RAN2 has not asked on ISA ID. When NG-RAN broadcasts ISA in system information, it locally gives an ID or INDEX to each area, called ISA ID. AF does not need to know ISA ID. AF needs to provide existing information cell IDs, TA list PLUS additionally if possible, ISA (geographic) as complementary information.

o	If at all, the ISA ID may be used to reduce signaling overhead in service announcement, i.e., adding ISA ID instead of ISA. But this seems to be unnecessary complexity as it would require that the ISA ID is synchronized between NG-RAN (the one who broadcasts SIB) and AF (the one who includes it in USD). It is much simpler to not to use ISA ID and focus on action of the incoming.

It is unclear in which direction we are going.

Please bear in mind that TS 29.571 defines the service area and has added the intendedServAreaList.

| 2025-03 | CT#107 | CP-250046 | 0630 | 1 | B | MBS broadcast support for NTN | 19.2.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

#### 5.9.4.4	Type: MbsServiceArea

Table 5.9.4.4-1: Definition of type MbsServiceArea

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ncgiList | array(NcgiTai) | C | 1..N | List of NR cell ids with their pertaining TAIs (NOTE 1, NOTE 3). |
| taiList | array(Tai) | C | 1..N | List of tracking area Ids (NOTE 1). |
| intendedServAreaList | array(IntendedServiceArea) | O | 1..N | List of NR NTN Intended Service Areas (NOTE 2) |
| NOTE 1: The MBS Service Area consists of the union of the cells in the tracking areas listed in the taiList IE and the cells listed in the ncgiList IE. At least one of the ncgiList IE and taiList IE shall be present. NOTE 2: The NR NTN Intended Service Area may be sent by the NEF/MBSF to the NG-RAN, via the MB-SMF and AMF, to further restrict the MBS service area. It contains information that shall only be used by the NG-RAN, i.e. the MB-SMF and AMF shall transfer this information transparently. See 3GPP TS 23.247 [72]. NOTE 3: In case of NR NTN, the cell IDs contained in the MBS service area may correspond to mapped cell IDs as defined in 3GPP TS 38.300 [73]. |  |  |  |  |

Also note that TS 26.517 already supports the delivery of the MbsServiceArea in the AvailabilityInformation

Table 5.2.9-1: Semantics of AvailabilityInformation data type

| Property name | Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mbsService‌Areas | array(Mbs‌Service‌Area) | O | 1..N | The Target service areas of this MBS Distribution Session, as defined in table 4.5.8-1 of TS 26.502 [6]. |
| mbs‌FSA‌Id | MbsFsaId | C | 0..1 | (Broadcast MBS Distribution Session only.) The MBS Frequency Selection Area (FSA) Identifier of the (broadcast) MBS Distribution Session in the parent service area, as defined in table 4.5.8-1 of TS 26.502 [6]. |
| nrParameters | array(Nr‌ParameterSet) | M | 1..N | (Broadcast MBS Distribution Session only.) The New Radio transmission parameters in the parent service areas, expressed using the data type specified in table 5.2.9-2. |
| nrRedCapUEInfo | NrRedCapUeInfo | O | 0..1 | (Broadcast MBS Distribution Session only.) Indicates whether the MBS session is suitable for consumption by NR RedCap UEs and/or non-RedCap UEs as defined by Target UE classes in TS 26.502 [6]. The data type and its enumerated values are specified in TS 29.571 [30]. If omitted, no information is known and the value NON_REDCAP_UE_ONLY may be assumed. |

And hence the support for delivering the ISA in the Service Announcement is implicitly supported within the Rel-19 timeframe, we just to upgrade the openapi version to the latest release.

# Proposal

It is proposed that we simplify the response to the LS in S4-250491 significantly thanking RAN2 for the request, and responding that we confirm that the inclusion of the ISA in the Service Announcement is implicitly supported by inheriting an updated data type for the MbsServiceArea from TS 29.571 in Rel-19. We will update the openapi version accordingly.