| WI | # | Feature group | Components | Prerequisite feature groups | Need for gNB to know whether the feature is supported by the UE (what happens if gNB does not know?) | Consequences if the feature is not supported by the UE | Type (See R4-17121 19) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | RAN5 implication | Remarks | Responsible WG | Recommendation for TSG-RAN | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| System parameter | 1-1 | 60kHz of subcarrier spacing for FR1 | 1) 60kHz subcarrier spacing for data channel in FR1 |  | Yes | UE does not support 60kHz of subcarrier spacing for data channel in FR1 | Type 4 | No Need | Applicable only to FR1 |  |  | RAN4 |  | Optional |
|  | 1-2 | 64QAM modulation for FR2 PDSCH | 1) 64QAM modulation for FR2 PDSCH |  | Yes | 64QAM for PUSCH is not possible |  | No need | Applicable only to FR2 |  | Capability can be discussed in future, e.g. when low cost device (e.g. IoT) and/or higher frequency band in FR2 are introduced | RAN4 | Mandatory without capability | Mandatory without capability |
|  | 1-3 | 64QAM for PUSCH | 1) 64QAM for PUSCH |  | Yes | 64QAM for PUSCH is not possible |  | No need | No need |  | Capability can be discussed in future, e.g. when low cost device (e.g. IoT) and/or higher frequency band in FR2 are introduced | RAN4 | Mandatory without capability | Mandatory without capability |
|  | 1-4 | 256QAM for PDSCH | 1) 256QAM for PDSCH |  | Yes | 256QAM for PDSCH is not possible | Type 4 for FR1 Type 1 for FR2 | No need | Yes |  | For FR1, it can be revisited in the future whether the 256QAM is mandated in all UE types or categories For FR2, RAN4 agreed that no BS and UE requirements will be introduced in Rel.15. | RAN4 | Mandatory with capability for FR1 Optional for FR2 | Mandatory with capability for FR1 Optional for FR2 |
|  | 1-5 | 256QAM for PUSCH | 1) 256QAM for PUSCH |  | Yes | 256QAM for PUSCH is not possible | Type1 for FR1 Type 1 for FR2 | No need | Yes |  | For FR1, RAN4 can further discuss to mandate 256QAM for PUSCH for FR1 in future release. For FR2, RAN4 agreed that no BS and UE requirements will be introduced in Rel.15. | RAN4 | Optional for FR1 Optional for FR2 | Optional for FR1 Optional for FR2 |
|  | 1-6 | pi/2-BPSK for PUSCH | 1) pi/2-BPSK for PUSCH |  | Yes | pi/2-BPSK for PUSCH is not possible | Type 4 | No need | Yes |  | RAN4 will define the same minimum requirements for pulse-shaped pi/2 BPSK and non pulse-shaped pi/2 BPSK for FR2. | RAN4 | Optional for FR1 | Optional for FR1 For FR2, mandatory with capability |
|  | 1-7 | pi/2-BPSK for PUCCH format 3/4 | 1) pi/2-BPSK for PUCCH format ¾ |  | Yes | pi/2-BPSK for PUCCH format 3/4 is not possible | Type 4 | No need | Yes |  |  | RAN4 | Optional for FR1 | Optional for FR1 For FR2, mandatory with capability |
|  | 1-8 | Active BWP switching delay | 1) Support of active BWP switching delay specified in TS38.133, candidate values set: {type1, type2} |  | Yes | UE does not support BWP switching delay | Type 4 | No need | No need |  | For this feature, RAN4 also sent another LS (R4-1803283) Network cannot configure the shorter delay for certain UE type | RAN4 | Mandatory to report type 1 or type 2 | Mandatory to report type 1 or type 2 |
|  | 1-10 | Support of EN-DC with LTE-NR coexistence in UL sharing from UE perspective | 1) LTE and NR UL Transmission in the shared carrier via TDM only 2) LTE and NR UL Transmission in the shared carrier via FDM only 3) LTE and NR UL transmission in the shared carrier via FDM or TDM |  | Yes | UE does not support of EN-DC with LTE-NR coexistence in UL sharing from UE perspective | Type 3 (per band combination) | No Need | Applicable only to FR1 |  |  | RAN4 | Optional | Optional |
|  | 1-11 | Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective | Support of switching type between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective. Type 1: <0.5us Type 2: <20us | 1-10 | Yes | UE does not support UL subcarrier alignment between LTE and NR for EN-DC with LTE-NR coexistence in UL | Type 3 (per band combination) | No Need | Applicable only to FR1 |  | This feature is the switching time between LTE UL and NR UL in the same carrier Per band combination signalling (Type 3) UE Capability signalling elements. 1: <0.5us switching type. 2: <20us switching type. | RAN4 |  | Mandatory to report switching time type 1 or type 2 if UE reports its capability in 1-10 as 1) LTE and NR UL Transmission in the shared carrier via TDM only, or 3) LTE and NR UL transmission in the shared carrier via FDM or TDM |
|  | 1-12 | 7.5kHz UL raster shift | 1) 7.5kHz UL raster shift |  | Yes | UE does not support 7.5kHz UL raster shift | N/A | N/A | No need |  |  | RAN4 | Mandatory in the SUL bands with uplink sharing either from UE perspective or from network perspective 7.5KHz raster shift as mandatory without capability signalling. 7.5kHz UL raster shift is mandatory in Bands n1, n2, n3, n5, n7, n8, n20, n28, n66. RAN4 can revisit the above bands in the future release. 7.5KHz raster shift is not mandatory for other LTE refarming band except the bands which were agreed to support 7.5kHz UL raster shift as mandatory | Mandatory in the SUL bands with uplink sharing either from UE perspective or from network perspective 7.5KHz raster shift as mandatory without capability signalling. 7.5kHz UL raster shift is mandatory in the uplink of all FDD bands defined in Table 5.2-1 of TS38.101-1. RAN4 can revisit the above bands in the future release. 7.5KHz raster shift is not mandatory for other LTE refarming band except the bands which were agreed to support 7.5kHz UL raster shift as mandatory |
| UE RF | 2-1 | Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports within a single CC | 1) FR1 channel bandwidths in TS38.101-1 Table 5.3.5-1 2) FR2 channel bandwidths in TS38.101-2 Table 5.3.5-1 |  | Yes | For FR1, all the bandwidths listed in TS38.101-1 v15.0.0 Table 5.3.5-1 for each band shall be mandatory with a single CC For FR2, UE does not support some UE channel bandwidths | Type 1 | No Need | No Need |  | UE capability signalling shall follow RP-172832 (Per-band capability signalling, separately for DL and UL and for each SCS) Whether a bandwidth newly introduced in future is mandatory for UE shall be discussed case by case. | RAN4 | For FR1, all the bandwidths listed in TS38.101-1 v15.0.0 Table 5.3.5-1 for each band shall be mandatory with a single CC. The bandwidths listed in the slide #3 of R4-1805985 are mandatory with a single CC. 90MHz is optional for n41, n77, n78. For FR2, the set of mandatory CBW is 50, 100, 200 MHz. | For FR1, all the bandwidths listed in TS38.101-1 v15.0.0 Table 5.3.5-1 for each band shall be mandatory with a single CC. The bandwidths listed in the slide #3 of R4-1805985 are mandatory with a single CC. 90MHz is optional for n41, n77, n78. For FR2, the set of mandatory CBW is 50, 100, 200 MHz. |
|  | 2-2 | Simultaneous reception or transmission with same or different numerologies in CA | 1) Support of simultaneous reception or transmission with same or different numerologies in CA |  | Yes | UE does not support simultaneous reception or transmission with same or different numerologies in CA | Type 3 | No need | No need |  | From RAN4 perspective UE shall be able to signal the supported SCS per CC for each band combination Same numerology for intra-band NR CA including both continuous and non-continuous is mandatory support for Rel15 The capability of supporting SCS within the single carrier in the CA configuration will be signalled separately, i.e., there is no need to mandatory UE to support mixed numerologies in CA case If a UE supports inter-band NR CA including both FR1 band(s) and FR2 band(s), the UE shall support two mixed numerologies between FR1 band(s) and FR2 band(s) in DL and UL with capability signaling | RAN4 |  | Same numerology for intra-band NR CA including both continuous and non-continuous is mandatory with capability in both FR1 and FR2. Two mixed numerologies between FR1 band(s) and FR2 band(s) in DL and UL are mandatory with capability if UE supports inter-band NR CA including both FR1 band(s) and FR2 band(s). Optional for other cases. |
|  | 2-3 | Non-contiguous intra-band CA frequency separation class for FR2 | 1) Support of frequency separation classes to handle the total frequency span for DL for intra-band non-contiguous CA 2) Support of frequency separation classes to handle the total frequency span for UL for intra-band non-contiguous CA |  | Yes | UE does not support non-contiguous intra-band CA for FR2 | Type 1 | No need | Applicable only to FR2 |  | UE signals the supported Frequency separation classes with per band granularity (Type 1) based on R4-1803363 Separate Frequency separation classes can be signalled for DL and UL | RAN4 | Mandatory to report for UE to support non-continuous CA in FR2 | Mandatory to report for UE to support non-continuous CA in FR2 |
|  | 2-4 | Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) | 1) Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) |  | Yes | UE does not support simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) | Type 3 (Per band combination) | No Need | No Need |  | For TDD-FDD and TDD-TDD band combinations for which simultaneous RxTx capability is agreed to be supported, corresponding capability indication must be set to “supported”. Band combinations for which simultaneous RxTx capability is mandatory EN-DC combinations (Both FR1 LTE – FR1 NR and FR1 LTE- FR2 NR) are captured in TS 38.101-3 | RAN4 | Mandatory/Optional support depends on band combination and captured in TS 38.101-3 | Mandatory/Optional support depends on band combination and captured in TS 38.101-3 |
|  | 2-5 | Simultaneous reception and transmission for inter band CA (TDD-TDD or TDD-FDD) | 1) Simultaneous reception and transmission for inter band CA (TDD-TDD or TDD-FDD) |  | Yes | UE does not support simultaneous reception and transmission for inter band CA (TDD-TDD or TDD-FDD) | Type 3 (Per band combination) | No Need | No Need |  | For TDD-FDD and TDD-TDD band combinations for which simultaneous RxTx capability is agreed to be supported, corresponding capability indication must be set to “supported”. Band combinations for which simultaneous RxTx capability is mandatory are captured in TS 38.101-1, TS 38.101-2 and TS 38.101-3 | RAN4 | Mandatory/Optional support depends on band combination and captured in TS 38.101-1, TS 38.101-2 and TS 38.101-3 | Mandatory/Optional support depends on band combination and captured in TS 38.101-1, TS 38.101-2 and TS 38.101-3 |
|  | 2-6 | Asynchronous FDD-FDD intra-band EN-DC DC | Asynchronous FDD-FDD intra-band EN-DC |  | Yes | UE does not support asynchronous FDD-FDD intra-band EN-DC | Type 3 (Per band combination) | Applicable only to FDD | Applicable only to FR1 |  |  | RAN4 | Optional | Optional |
|  | 2-7 | Almost contiguous UL CP-OFDM | 1) Support of almost contiguous UL CP-OFDM transmissions |  |  | UE does not support almost contiguous UL CP-OFDM | Type 4 | No need | Yes |  | RAN4 had defined the requirements for “Almost contiguous UL CP-OFDM” in Rel-15. | RAN4 | Optional | Optional |
|  | 2-9 | UE power class | 1) Support of [non-default] FR1 UE power class 2) Support of FR2 UE power class 3) Support of FR1 UE power class for EN-DC 4) Support of FR1 UE power class for NR-CA |  | Yes | gNB cannot know the UE power class | Type 1 and Type 3 (see remarks column) | No Need | No Need |  | Capability signalling - FR1 UE power class ( Type 1 (per band)) - FR2 UE power class ( Type 1 (per band)) - FR1 UE power class for EN-DC (Type 3 (per band combination)) - FR1 UE power class for NR CA (Type 3 (per band combination)) Note that default power class for each component is indicated in TS38.101-1/2/3. If the default power class is not indicated, UE shall report supported power class. | RAN4 | Mandatory to report non-default power class if UE supports | Mandatory to report non-default power class if UE supports |
|  | 2-10 | Simultaneous reception and transmission for SA SUL band combinations | 1) Simultaneous reception and transmission for SA SUL band combinations |  | Yes | UE does not support simultaneous reception and transmission for SA SUL band combinations | Type 3 (Per band combination) | No Need | No Need |  |  | RAN4 | Mandatory/Optional support depends on band combination and captured in TS 38.101-1 | Mandatory/Optional support depends on band combination and captured in TS 38.101-1 |
|  | 2-11 | Multiple frequency band indication | 1) Multiple frequency band indication |  | Yes | UE does not support multiple frequency band indication | Type 4 | No Need | No Need |  | Per UE capability | RAN4 | Mandatory [with or without] capability | Mandatory without capability |
|  | 2-12 | Modified MPR behaviour | 1) Modified MPR behaviour |  | Yes | UE does not support modified MPR behaviour | Type 1 | No Need | No need |  | Per band capability | RAN4 | Optional | Optional |
|  | 2-13 | Multiple NS/P-Max | 1) Multiple NS/P-Max |  | Yes | UE does not support multiple NS/P-Max | Type 4 | No Need | No need |  | Per UE capability | RAN4 | Mandatory without capability | Mandatory without capability |
|  | 2-14 | Maximum uplink duty cycle for FR1 power class 2 UE | 1) Maximum percentage of uplink symbols can be scheduled within a certain evaluation period. The value range is {60%, 70%, 80%, 90%, 100%}. If the field is absent, 50% shall be applied. |  | Yes | The network cannot know maximum percentage of uplink symbols can be scheduled for high power UE (i.e. power class 2 UE) | Type 1 | No Need | Applicable only to FR1 |  | Per band capability. If this capability is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%, or this capability is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than this capability, apply all requirements for the default power class. The evaluation period is up to UE implementation, no less than one radio frame. UE do not need to do UL duty cycle calculation when it’s transmit power is below 23dBm and all the UL/DL configurations can be scheduled. | RAN4 | Optional | Optional |
|  | 2-15 | Power boosting for Pi/2 BPSK for power class 3 UE | 1) Power boosting for Pi/2 BPSK for power class 3 UE in TDD bands n40, n77, n78 and n79 with duty cycle less than 40% | 1-6, 1-7 | Yes | Power class 3 UE does not support power boosting for Pi/2 BPSK in TDD bands n40, n77, n78 and n79 with duty cycle less than 40% | Type 1 | Applicable only to TDD | Applicable only to FR1 |  | Per band capability | RAN4 |  | Optional |
|  | 2-16 | Maximum uplink duty cycle for FR2 power class 3 UE | 1) Maximum percentage of uplink transmission time that can be scheduled within a certain evaluation period to ensure compliance with applicable electromagnetic energy absorption requirements. The value range is {2%, 10%, 20%, 30%, 40%, 50%, 60%, 70%, 80%, 90%, 100%}. |  | Yes | The network cannot know maximum percentage of uplink signals can be scheduled for FR2 handheld UE (i.e. power class 3 UE) | Type 1 | No Need | Applicable only to FR2 |  | Per band capability. Default value is FFS For a FR2 UE when the percentage of uplink transmission time scheduled by the network within a certain evaluation period is larger than its capability, UE could do power back off as in TS38.101-2. | RAN4 | Mandatory to report non-default uplink duty cycle if UE supports |  |
|  | 2-17 | PA architectures | 1) Single PA 2) Dual PA |  | Yes | The network cannot know A-MPR/MPR and MSD values for dual uplink, and switching time between LTE UL and NR UL in single switched UL operation mode for intra-band non-contiguous EN-DC | Type 3 | No Need | No Needed |  | Per band per band combination capability The following requirements are involved by this capability - A-MPR/MPR and MSD values for dual uplink. Whether two sets of requirements will be introduced in RAN4 can be further discussed for each specific band combination - Switching time between LTE UL and NR UL in single switched UL operation mode for intra-band non-contiguous EN-DC | RAN4 | Mandatory to report for UE to support intra-band EN-DC, intra-band UL CA and FDM-based ULSUP |  |
| Baseband | 3-1 | Independent measurement gap configurations for FR1 and FR2 | 1) measurement gaps for FR1 and FR2 are configured independently |  | Yes | UE does not support independent gap configuration between FR1 and FR2 | Type 4 | No Need | No Need |  |  | RAN4 | Optional | Optional |
|  | 3-2 | Simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement | 1) Simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement |  | Yes | UE does not support simultaneous reception of data and SS block with different numerologies | Type 4 | No Need | Yes |  |  | RAN4 | Optional | Optional |
|  | 3-3 | Short measurement gap | 1) Measurement gap patterns with short MGL (gap pattern#2, 3, 6, 7, 8, 10) are supported for E-UTRAN measurement. Gap patterns #6, 7, 8, 10 only apply to E-UTRAN measurement when MO includes both E-UTRAN and NR. |  | Yes | UE does not support measurement gap patterns with short MGL (gap pattern#2, 3, 6, 7, 8, 10) for E-UTRAN measurement. Gap patterns #6, 7, 8, 10 only apply to E-UTRAN measurement when MO includes both E-UTRAN and NR | Type 4 | No need | No need |  | Per UE capability Note that this capability is signalled as a part of supportedGapPattern in TS38.306. | RAN4 | Optional | Optional |
|  | 3-4 | SU-MIMO Interference Mitigation advanced receiver | 1) R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2 with 2 RX antennas 2) R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2, 3, and 4 with 4 RX antennas |  | No | SU-MIMO Interference Mitigation advanced receiver capability is not possible | Type 4 | No need | No need |  | UE supporting the feature is required to meet the Enhanced Receiver Type [TBD] requirements in TS 38.101-4 | RAN4 | Optional without capability signalling | Optional without capability signalling |