| 3GPP TS 29.558 V19.5.0 (2025-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Enabling Edge Applications; Application Programming Interface (API) specification; Stage 3; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	22

1	Scope	24

2	References	24

3	Definitions of terms, symbols and abbreviations	25

3.1	Terms	25

3.2	Symbols	25

3.3	Abbreviations	25

4	Overview	26

5	Services offered by Edge Enabler Server	26

5.1	Introduction	26

5.2	Eees_EASRegistration Service	28

5.2.1	Service Description	28

5.2.2	Service Operations	29

5.2.2.1	Introduction	29

5.2.2.2	Eees_EASRegistration_Request	29

5.2.2.2.1	General	29

5.2.2.2.2	Service consumer registering to EES using Eees_EASRegistration_Request operation	29

5.2.2.3	Eees_EASRegistration_Update	29

5.2.2.3.1	General	29

5.2.2.3.2	Service consumer updating registration information using Eees_EASRegistration_Update operation	30

5.2.2.4	Eees_EASRegistration_Deregister	30

5.2.2.4.1	General	30

5.2.2.4.2	Service consumer deregistering from EES using Eees_EASRegistration_Deregister operation	30

5.3	Eees_UELocation Service	31

5.3.1	Service Description	31

5.3.2	Service Operations	31

5.3.2.1	Introduction	31

5.3.2.2	Eees_UELocation_Get	31

5.3.2.2.1	General	31

5.3.2.2.2	Service consumer obtaining UE location information from EES using Eees_UELocation_Get operation	31

5.3.2.2.3	User consent management	32

5.3.2.3	Eees_UELocation_Subscribe	32

5.3.2.3.1	General	32

5.3.2.3.2	Service consumer subscribing to continuous UE(s) location reporting from EES using Eees_UELocation_Subscribe operation	32

5.3.2.3.3	User consent management	33

5.3.2.4	Eees_UELocation_Notify	34

5.3.2.4.1	General	34

5.3.2.4.2	EES notifying the UE(s) location reporting to service consumer using Eees_UELocation_Notify operation	34

5.3.2.4.3	EES notifying the service consumer about user consent revocation using Eees_UELocation_Notify operation	34

5.3.2.5	Eees_UELocation_UpdateSubscription	35

5.3.2.5.1	General	35

5.3.2.5.2	Service consumer updating continuous UE(s) location reporting subscription at EES using Eees_UELocation_UpdateSubscribe operation	35

5.3.2.5.3	User consent management	35

5.3.2.6	Eees_UELocation_Unsubscribe	36

5.3.2.6.1	General	36

5.3.2.6.2	Service consumer unsubscribing to continuous UE(s) location reporting from EES using Eees_UELocation_Unsubscribe operation	36

5.4	Eees_UEIdentifier Service	37

5.4.1	Service Description	37

5.4.2	Service Operations	37

5.4.2.1	Introduction	37

5.4.2.2	Eees_UEIdentifier_Get	37

5.4.2.2.1	General	37

5.4.2.2.1A	Service consumer obtaining UE Identifier Information using the "Get" custom operation	37

5.4.2.2.2	EAS obtaining UE identifier from EES using Eees_UEIdentifier_Fetch custom operation	38

5.5	Eees_AppClientInformation Service	38

5.5.1	Service Description	38

5.5.2	Service Operations	39

5.5.2.1	Introduction	39

5.5.2.2	Eees_AppClientInformation_Subscribe	39

5.5.2.2.1	General	39

5.5.2.2.2	Service consumer subscribing to AC information reporting from the EES using the Eees_AppClientInformation_Subscribe operation	39

5.5.2.3	Eees_AppClientInformation_Notify	40

5.5.2.3.1	General	40

5.5.2.3.2	EES notifying the AC information to the service consumer using Eees_AppClientInformation_Notify operation	40

5.5.2.4	Eees_AppClientInformation_UpdateSubscription	40

5.5.2.4.1	General	40

5.5.2.4.2	Service consumer updating AC information reporting subscription at the EES using Eees_AppClientInformation_UpdateSubscribe operation	40

5.5.2.5	Eees_AppClientInformation_Unsubscribe	41

5.5.2.5.1	General	41

5.5.2.5.2	Service consumer unsubscribing to AC information reporting from the EES using Eees_AppClientInformation_Unsubscribe operation	41

5.6	Eees_SessionWithQoS Service	42

5.6.1	Service Description	42

5.6.2	Service Operations	42

5.6.2.1	Introduction	42

5.6.2.2	Eees_SessionWithQoS_Create	42

5.6.2.2.1	General	42

5.6.2.2.2	Service consumer requesting reservation of resources for a data session between AC and service consumer with specific QoS using Eees_SessionWithQoS operation	42

5.6.2.3	Eees_SessionWithQoS_Update	43

5.6.2.3.1	General	43

5.6.2.3.2	Service consumer updating QoS of a data session between AC and service consumer using Eees_SessionWithQoS_Update operation	43

5.6.2.4	Eees_SessionWithQoS_Revoke	44

5.6.2.4.1	General	44

5.6.2.4.2	Service consumer revoking QoS of a data session between AC and service consumer using Eees_SessionWithQoS_Revoke operation	44

5.6.2.5	Eees_SessionWithQoS_Notify	45

5.6.2.5.1	General	45

5.6.2.5.2	EES notifying QoS of a data session between AC and service consumer using Eees_SessionWithQoS_Notify operation	45

5.7	Eees_EASDiscovery Service	45

5.7.1	Service Description	45

5.7.2	Service Operations	45

5.7.2.1	Introduction	45

5.7.2.2	Eees_EASDiscovery_TEasDiscRequest	46

5.7.2.2.1	General	46

5.7.2.2.2	Service consumer requesting T-EAS discovery information using Eees_EASDiscovery_TEasDiscRequest operation	46

5.8	Eees_ACRManagementEvent Service	46

5.8.1	Service Description	46

5.8.2	Service Operations	46

5.8.2.1	Introduction	46

5.8.2.2	Eees_ACRManagementEvent_Subscribe	47

5.8.2.2.1	General	47

5.8.2.2.2	Service consumer requesting to get notifications of ACR management events using Eees_ACRManagementEvent_Subscribe service operation	47

5.8.2.3	Eees_ACRManagementEvent_UpdateSubscription	47

5.8.2.3.1	General	47

5.8.2.3.2	Service consumer updating an existing Individual ACR Management Events Subscription using Eees_ACRManagementEvent_UpdateSubscription service operation	48

5.8.2.4	Eees_ACRManagementEvent_Unsubscribe	48

5.8.2.4.1	General	48

5.8.2.4.2	Service consumer deleting an existing Individual ACR Management Events Subscription using Eees_ACRManagementEvent_Unsubscribe service operation	48

5.8.2.5	Eees_ACRManagementEvent_Notify	49

5.8.2.5.1	General	49

5.8.2.5.2	EES notifying ACR management events using Eees_ACRManagementEvent_Notify operation	49

5.8.2.5.3	EES notifying the availability of user path management events monitoring via the 3GPP 5GC network using Eees_ACRManagementEvent_Notify operation	49

5.9	Eees_AppContextRelocation Service	50

5.9.1	Service Description	50

5.9.2	Service Operations	50

5.9.2.1	Introduction	50

5.9.2.2	Eees_AppContextRelocation_SelectedTargetEAS_Declare	50

5.9.2.2.1	General	50

5.9.2.2.2	S-EAS informing the S-EES about the selected T-EAS using Eees_AppContextRelocation_SelectedTargetEAS_Declare operation	50

5.9.2.3	Eees_AppContextRelocation_ACRDetermination_Request	51

5.9.2.3.1	General	51

5.9.2.3.2	Request the S-EES to determine the ACR using Eees_AppContextRelocation_ACRDetermination_Request operation	51

5.10	Eees_EECContextRelocation Service	51

5.10.1	Service Description	51

5.10.2	Service Operations	51

5.10.2.1	Introduction	51

5.10.2.2	Eees_EECContextRelocation_Pull	52

5.10.2.2.1	General	52

5.10.2.2.2	Service consumer pulling the EEC context information from the EES using the Eees_EECContextRelocation_Pull operation	52

5.10.2.3	Eees_EECContextRelocation_Push	52

5.10.2.3.1	General	52

5.10.2.3.2	Service consumer pushing the EEC context information to the EES using the Eees_EECContextRelocation_Push operation	52

5.11	Eees_EELManagedACR Service	53

5.11.1	Service Description	53

5.11.2	Service Operations	53

5.11.2.1	Introduction	53

5.11.2.2	Eees_EELManagedACR_Request	53

5.11.2.2.1	General	53

5.11.2.2.2	EEL Managed ACR Request	53

5.11.2.3	Eees_EELManagedACR_Subscribe	54

5.11.2.3.1	General	54

5.11.2.3.2	Subscribe to ACT status information reporting	54

5.11.2.4	Eees_EELManagedACR_Notify	54

5.11.2.4.1	General	54

5.11.2.4.2	ACT Status Notification	54

5.12	Eees_ACRStatusUpdate Service	55

5.12.1	Service Description	55

5.12.2	Service Operations	55

5.12.2.1	Introduction	55

5.12.2.2	Eees_ACRStatusUpdate_Request	55

5.12.2.2.1	General	55

5.12.2.2.2	ACR Status Update Request	55

5.13	Eees_ACRParameterInformation Service	56

5.13.1	Service Description	56

5.13.2	Service Operations	56

5.13.2.1	Introduction	56

5.13.2.2	Eees_ACRParameterInformation_Request	56

5.13.2.2.1	General	56

5.13.2.2.2	ACR Parameters Information Request	56

5.14	Eees_CommonEASAnnouncement Service	56

5.14.1	Service Description	56

5.14.2	Service Operations	57

5.14.2.1	Introduction	57

5.14.2.2	Eees_CommonEASAnnouncement_Declare	57

5.14.2.2.1	General	57

5.14.2.2.2	Common EAS Information Declaration	57

5.15	Eees_TrafficInfluenceEAS Service	57

5.15.1	Service Description	57

5.15.2	Service Operations	58

5.15.2.1	Introduction	58

5.15.2.2	Eees_TrafficInfluenceEAS_Manage	58

5.15.2.2.1	General	58

5.15.2.2.2	Application Traffic Influence Initiation	58

5.15.2.2.3	Application Traffic Influence Update	58

5.15.2.2.4	Application Traffic Influence Cancellation	59

5.15.2.2.5	Application Traffic Influence Retrieval	59

5.16	Eees_EASInformationProvisioning Service	60

5.16.1	Service Description	60

5.16.2	Service Operations	60

5.16.2.1	Introduction	60

5.16.2.2	Eees_EASInformationProvisioning_Declare	60

5.16.2.2.1	General	60

5.16.2.2.2	EAS Information Provisioning Information Declaration	60

6	Services offered by Edge Configuration Server	61

6.1	Introduction	61

6.2	Eecs_EESRegistration Service	62

6.2.1	Service Description	62

6.2.2	Service Operations	62

6.2.2.1	Introduction	62

6.2.2.2	Eecs_EESRegistration_Request	62

6.2.2.2.1	General	62

6.2.2.2.2	EES registering to ECS using Eecs_EESRegistration_Request operation	62

6.2.2.3	Eecs_EESRegistration_Update	63

6.2.2.3.1	General	63

6.2.2.3.2	EES updating registration information using Eecs_EESRegistration_Update operation	63

6.2.2.4	Eecs_EESRegistration_Deregister	63

6.2.2.4.1	General	63

6.2.2.4.2	EES deregistering from ECS using Eecs_EESRegistration_Deregister operation	64

6.3	Eecs_TargetEESDiscovery Service	64

6.3.1	Service Description	64

6.3.2	Service Operations	64

6.3.2.1	Introduction	64

6.3.2.2	Eecs_TargetEESDiscovery_Request	64

6.3.2.2.1	General	64

6.3.2.2.2	Service consumer fetching the target Enabler Server information from the ECS using Eecs_TargetEESDiscovery_Request operation	64

6.4	Eecs_EASInfoManagement Service	65

6.4.1	Service Description	65

6.4.2	Service Operations	65

6.4.2.1	Introduction	65

6.4.2.2	Eecs_EASInfoManagement_Get	66

6.4.2.2.1	General	66

6.4.2.2.2	Common EAS Binding Information Retrieval	66

6.4.2.3	Eecs_EASInfoManagement_Store	66

6.4.2.3.1	General	66

6.4.2.3.2	Common EAS Binding Information Storage	66

6.4.2.4	Eecs_EASInfoManagement_Update	67

6.4.2.4.1	General	67

6.4.2.4.2	Common EAS Binding Information Update	67

6.4.2.5	Eecs_EASInfoManagement_Delete	67

6.4.2.5.1	General	67

6.4.2.5.2	Common EAS Binding information Deletion	68

6.5	Eecs_ECSServiceProvisioning	68

6.5.1	Service Description	68

6.5.2.1	Introduction	68

6.5.2.2	Eecs_ECSServiceProvisioning_Request	69

6.5.2.2.1	General	69

6.5.2.2.2	Service Provisioning Information Retrieval Request	69

6.5.2.3	Eecs_ECSServiceProvisioning_Subscribe	69

6.5.2.3.1	General	69

6.5.2.3.2	Service Provisioning Subscription Creation	69

6.5.2.4	Eecs_ECSServiceProvisioning_UpdateSubscription	69

6.5.2.4.1	General	69

6.5.2.4.2	Service Provisioning Subscription Update	70

6.5.2.5	Eecs_ECSServiceProvisioning_Unsubscribe	70

6.5.2.5.1	General	70

6.5.2.5.2	Service Provisioning Subscription Deletion	70

6.5.2.6	Eecs_ECSServiceProvisioning_Notify	70

6.5.2.6.1	General	70

6.5.2.6.2	Service Provisioning Notification	71

6.6	Eecs_ECSDiscovery Service	71

6.6.1	Service Description	71

6.6.2	Service Operations	71

6.6.2.1	Introduction	71

6.6.2.2	Eecs_ECSDiscovery_Request	71

6.6.2.2.1	General	71

6.6.2.2.2	Service consumer fetching partner ECS information from the ECS using the Eecs_ECSDiscovery_Request operation	71

6.6.2.3	Eecs_ECSDiscovery_Request	72

6.6.2.3.1	General	72

6.6.2.3.2	ECS notifying the ECS information to the service consumer using Eecs_ECSDiscovery_Notification operation	72

6.7	Eecs_ACREvents	72

6.7.1	Service Description	72

6.7.2	Service Operations	72

6.7.2.1	Introduction	72

6.7.2.2	Eecs_ACREvents_Subscribe	73

6.7.2.2.1	General	73

6.7.2.2.2	ACR Events Subscription Creation	73

6.7.2.3	Eecs_ACREvents_Update	73

6.7.2.3.1	General	73

6.7.2.3.2	ACR Events Subscription Update	73

6.7.2.4	Eecs_ACREvents_Unsubscribe	74

6.7.2.4.1	General	74

6.7.2.4.2	ACR Events Subscription Deletion	74

6.7.2.5	Eecs_ACREvents_Notify	74

6.7.2.5.1	General	74

6.7.2.5.2	ACR Events Notification	75

6A	Services offered by the Cloud Application Server (CAS)	75

6A.1	Introduction	75

6A.2	Ecas_SelectedEES Service	75

6A.2.1	Service Description	75

6A.2.2	Service Operations	76

6A.2.2.1	Introduction	76

6A.2.2.2	Ecas_SelectedEES_Request	76

6A.2.2.2.1	General	76

6A.2.2.2.2	Service consumer informing the CAS of the selected EES using Ecas_SelectedEES_Declare operation	76

6B	Services offered by the Cloud Enabler Server (CES)	76

6B.1	Introduction	76

7	Information applicable to several APIs	77

7.1	General	77

7.2	Data Types	77

7.2.1	General	77

7.2.2	Referenced structured data types	78

7.2.3	Referenced simple data types and enumerations	78

7.3	Usage of HTTP	78

7.4	Content type	78

7.5	URI structure	78

7.5.1	Resource URI structure	78

7.5.2	Custom operations URI structure	79

7.6	Notifications	79

7.7	Error handling	79

7.8	Feature negotiation	79

7.9	HTTP headers	79

7.10	Conventions for Open API specification files	80

8	Edge Enabler Server API Definitions	80

8.1	Eees_EASRegistration API	80

8.1.1	Introduction	80

8.1.2	Resources	80

8.1.2.1	Overview	80

8.1.2.2	Resource: EAS Registrations	81

8.1.2.2.1	Description	81

8.1.2.2.2	Resource Definition	81

8.1.2.2.3	Resource Standard Methods	81

8.1.2.2.3.1	POST	81

8.1.2.2.4	Resource Custom Operations	82

8.1.2.3	Resource: Individual EAS Registration	82

8.1.2.3.1	Description	82

8.1.2.3.2	Resource Definition	82

8.1.2.3.3	Resource Standard Methods	82

8.1.2.3.3.1	GET	82

8.1.2.3.3.2	PUT	83

8.1.2.3.3.3	DELETE	84

8.1.2.3.3.4	PATCH	85

8.1.2.3.4	Resource Custom Operations	86

8.1.3	Custom Operations without associated resources	86

8.1.4	Notifications	87

8.1.5	Data Model	87

8.1.5.1	General	87

8.1.5.2	Structured data types	88

8.1.5.2.1	Introduction	88

8.1.5.2.2	Type: EASRegistration	88

8.1.5.2.3	Type: EASProfile	89

8.1.5.2.4	Type: EASServiceKPI	92

8.1.5.2.5	Type: EndPoint	92

8.1.5.2.6	Type: EASRegistrationPatch	93

8.1.5.2.7	Type: TransContSuppDetails	93

8.1.5.2.8	Type: EASBundleInfo	93

8.1.5.2.9	Type: EASBdlReqs	94

8.1.5.2.10	Type: CoordinatedAcrReqs	94

8.1.5.2.11	Type: AssociatedDevice	95

8.1.5.3	Simple data types and enumerations	95

8.1.5.3.1	Introduction	95

8.1.5.3.2	Simple data types	95

8.1.5.3.3	Enumeration: PermissionLevel	95

8.1.5.3.4	Enumeration: EASCategory	95

8.1.5.3.5	Enumeration: TransportProtocol	96

8.1.5.3.6	Enumeration: BdlType	96

8.1.5.3.7	Enumeration: Affinity	96

8.1.5.3.8	Enumeration: FailureAction	96

8.1.5.3.9	Enumeration: EASStatus	96

8.1.5.3.10	Enumeration: DeviceType	97

8.1.6	Error Handling	97

8.1.6.1	General	97

8.1.6.2	Protocol Errors	97

8.1.6.3	Application Errors	97

8.1.7	Feature negotiation	97

8.2	Eees_UELocation API	98

8.2.1	Introduction	98

8.2.2	Resources	98

8.2.2.1	Overview	98

8.2.2.2	Resource: Location Information Subscriptions	99

8.2.2.2.1	Description	99

8.2.2.2.2	Resource Definition	99

8.2.2.2.3	Resource Standard Methods	100

8.2.2.2.3.1	POST	100

8.2.2.2.4	Resource Custom Operations	100

8.2.2.3	Resource: Individual Location Information Subscription	101

8.2.2.3.1	Description	101

8.2.2.3.2	Resource Definition	101

8.2.2.3.3	Resource Standard Methods	101

8.2.2.3.3.1	GET	101

8.2.2.3.3.2	PATCH	102

8.2.2.3.3.3	PUT	103

8.2.2.3.3.4	DELETE	104

8.2.2.3.4	Resource Custom Operations	105

8.2.3	Custom Operations without associated resources	105

8.2.3.1	Overview	105

8.2.3.2	Operation: Fetch	106

8.2.3.2.1	Description	106

8.2.3.2.2	Operation Definition	106

8.2.4	Notifications	107

8.2.4.1	General	107

8.2.4.2	Location Information Notification	108

8.2.4.2.1	Description	108

8.2.4.2.2	Target URI	108

8.2.4.2.3	Standard Methods	108

8.2.4.2.3.1	POST	108

8.2.4.3	User Consent Revocation Notification	109

8.2.4.3.1	Description	109

8.2.4.3.2	Target URI	109

8.2.4.3.3	Standard Methods	109

8.2.5	Data Model	110

8.2.5.1	General	110

8.2.5.2	Structured data types	112

8.2.5.2.1	Introduction	112

8.2.5.2.2	Type: LocationSubscription	112

8.2.5.2.3	Type: LocationSubscriptionPatch	114

8.2.5.2.4	Type: LocationNotification	114

8.2.5.2.5	Type: LocationEvent	114

8.2.5.2.6	Type: LocationRequest	115

8.2.5.2.7	Type: LocationResponse	115

8.2.5.2.8	Type: ConsentRevocNotif	115

8.2.5.2.9	Type: ConsentRevoked	116

8.2.5.3	Simple data types and enumerations	116

8.2.6	Error Handling	116

8.2.6.1	General	116

8.2.6.2	Protocol Errors	116

8.2.6.3	Application Errors	116

8.2.7	Feature negotiation	116

8.3	Eees_UEIdentifier API	117

8.3.1	Introduction	117

8.3.2	Resources	117

8.3.3	Custom Operations without associated resources	117

8.3.3.1	Overview	117

8.3.3.2	Operation: Fetch	118

8.3.3.2.1	Description	118

8.3.3.2.2	Operation Definition	118

8.3.3.3	Operation: Get	119

8.3.3.3.1	Description	119

8.3.3.3.2	Operation Definition	119

8.3.4	Notifications	120

8.3.5	Data Model	120

8.3.5.1	General	120

8.3.5.2	Structured data types	121

8.3.5.2.1	Introduction	121

8.3.5.2.2	Type: UserInformation	121

8.3.5.2.3	Type: UserInfo	122

8.3.5.2.4	Type: UeIdInfo	122

8.3.5.2.5	Type: UeId	123

8.3.5.3	Simple data types and enumerations	123

8.3.5.3.1	Introduction	123

8.3.5.3.2	Simple data types	123

8.3.6	Error Handling	123

8.3.6.1	General	123

8.3.6.2	Protocol Errors	123

8.3.6.3	Application Errors	123

8.3.7	Feature negotiation	124

8.4	Eees_AppClientInformation API	124

8.4.1	Introduction	124

8.4.2	Resources	124

8.4.2.1	Overview	124

8.4.2.2	Resource: Application Client Information Subscriptions	125

8.4.2.2.1	Description	125

8.4.2.2.2	Resource Definition	125

8.4.2.2.3	Resource Standard Methods	126

8.4.2.2.3.1	POST	126

8.4.2.2.4	Resource Custom Operations	126

8.4.2.3	Resource: Individual Application Client Information Subscription	126

8.4.2.3.1	Description	126

8.4.2.3.2	Resource Definition	126

8.4.2.3.3	Resource Standard Methods	127

8.4.2.3.3.1	GET	127

8.4.2.3.3.2	PATCH	128

8.4.2.3.3.3	PUT	129

8.4.2.3.3.4	DELETE	130

8.4.2.3.4	Resource Custom Operations	131

8.4.3	Custom Operations without associated resources	131

8.4.4	Notifications	131

8.4.4.1	General	131

8.4.4.2	AC Information Notification	131

8.4.4.2.1	Description	131

8.4.4.2.2	Target URI	131

8.4.4.2.3	Standard Methods	131

8.4.4.2.3.1	POST	131

8.4.5	Data Model	132

8.4.5.1	General	132

8.4.5.2	Structured data types	135

8.4.5.2.1	Introduction	135

8.4.5.2.2	Type: ACInfoSubscription	135

8.4.5.2.3	Type: ACInfoSubscriptionPatch	136

8.4.5.2.4	Type: ACFilters	136

8.4.5.2.5	Type: ACInfoNotification	137

8.4.5.2.6	Type: ACInformation	137

8.4.5.2.7	Type: EASBdlInd	138

8.4.5.3	Simple data types and enumerations	138

8.4.5.3.1	Introduction	138

8.4.5.3.2	Simple data types	138

8.4.5.3.3	Enumeration: TrigCondParams	139

8.4.6	Error Handling	139

8.4.6.1	General	139

8.4.6.2	Protocol Errors	139

8.4.6.3	Application Errors	139

8.4.7	Feature negotiation	139

8.5	Eees_SessionWithQoS API	140

8.5.1	Introduction	140

8.5.2	Resources	140

8.5.2.1	Overview	140

8.5.2.2	Resource: Sessions with QoS	141

8.5.2.2.1	Description	141

8.5.2.2.2	Resource Definition	141

8.5.2.2.3	Resource Standard Methods	141

8.5.2.2.3.1	POST	141

8.5.2.2.3.2	GET	142

8.5.2.2.4	Resource Custom Operations	143

8.5.2.3	Resource: Individual Session with QoS	143

8.5.2.3.1	Description	143

8.5.2.3.2	Resource Definition	143

8.5.2.3.3	Resource Standard Methods	144

8.5.2.3.3.1	PATCH	144

8.5.2.3.3.2	PUT	145

8.5.2.3.3.3	DELETE	146

8.5.2.3.3.4	GET	147

8.5.2.3.4	Resource Custom Operations	148

8.5.3	Custom Operations without associated resources	148

8.5.4	Notifications	148

8.5.4.1	General	148

8.5.4.2	User Plane Event Notification	148

8.5.4.2.1	Description	148

8.5.4.2.2	TargetURI	148

8.5.4.2.3	Standard Methods	148

8.5.4.2.3.1	POST	148

8.5.5	Data Model	149

8.5.5.1	General	149

8.5.5.2	Structured data types	151

8.5.5.2.1	Introduction	151

8.5.5.2.2	Type: SessionWithQoS	151

8.5.5.2.3	Type: SessionWithQoSPatch	154

8.5.5.2.4	Type: UserPlaneEventNotification	154

8.5.5.3	Simple data types and enumerations	155

8.5.6	Error Handling	155

8.5.6.1	General	155

8.5.6.2	Protocol Errors	155

8.5.6.3	Application Errors	155

8.5.7	Feature negotiation	155

8.6	Eees_ACRManagementEvent API	155

8.6.1	Introduction	155

8.6.2	Resources	156

8.6.2.1	Overview	156

8.6.2.2	Resource: ACR Management Events Subscriptions	157

8.6.2.2.1	Description	157

8.6.2.2.2	Resource Definition	157

8.6.2.2.3	Resource Standard Methods	157

8.6.2.2.3.1	POST	157

8.6.2.2.3.2	GET	158

8.6.2.2.4	Resource Custom Operations	159

8.6.2.3	Resource: Individual ACR Management Events Subscription	159

8.6.2.3.1	Description	159

8.6.2.3.2	Resource Definition	159

8.6.2.3.3	Resource Standard Methods	159

8.6.2.3.3.1	PATCH	159

8.6.2.3.3.2	PUT	160

8.6.2.3.3.3	DELETE	161

8.6.2.3.3.4	GET	162

8.6.2.3.4	Resource Custom Operations	163

8.6.3	Custom Operations without associated resources	163

8.6.4	Notifications	164

8.6.4.1	General	164

8.6.4.2	ACR Management Events Notification	164

8.6.4.2.1	Description	164

8.6.4.2.2	Notification definition	164

8.6.4.3	User Plane Path Change Availability Notification	165

8.6.4.3.1	Description	165

8.6.4.3.2	Target URI	165

8.6.4.3.3	Standard Methods	166

8.6.5	Data Model	166

8.6.5.1	General	166

8.6.5.2	Structured data types	169

8.6.5.2.1	Introduction	169

8.6.5.2.2	Type: AcrMgntEventsSubscription	169

8.6.5.2.3	Type: AcrMgntEventSubsc	172

8.6.5.2.4	Type: AcrMgntEventsSubscriptionPatch	174

8.6.5.2.5	Type: AcrMgntEventsNotification	175

8.6.5.2.6	Type: AcrMgntEventReport	176

8.6.5.2.7	Type: FailureAcrMgntEventInfo	178

8.6.5.2.8	Type: TargetUeIdentification	179

8.6.5.2.9	Type: UpPathChangeInfo	179

8.6.5.2.10	Type: IndUeIdentification	179

8.6.5.2.11	Type: AvailabilityNotif	180

8.6.5.2.12	Type: SelectedACRScenarios	180

8.6.5.2.13	Type: ACRParameters	180

8.6.5.2.14	Type: TrafficFilterInfo	180

8.6.5.2.15	Type: EasAckInformation	181

8.6.5.2.16	Type: EasInBundleInfo	181

8.6.5.3	Simple data types and enumerations	181

8.6.5.3.1	Introduction	181

8.6.5.3.2	Simple data types	181

8.6.5.3.3	Enumeration: AcrMgntEvent	181

8.6.5.3.4	Enumeration: AcrMgntEventFilter	182

8.6.5.3.5	Enumeration: ActStatus	182

8.6.5.3.6	Enumeration: AcrMgntEventFailureCode	182

8.6.5.3.7	Enumeration: AvailabilityStatus	182

8.6.5.3.8	Enumeration: ResultCode	183

8.6.5.3.9	Enumeration: InOutArea	183

8.6.6	Error Handling	183

8.6.6.1	General	183

8.6.6.2	Protocol Errors	183

8.6.6.3	Application Errors	183

8.6.7	Feature negotiation	183

8.7	Eees_EECContextRelocation API	184

8.7.1	API URI	184

8.7.1A	Usage of HTTP	184

8.7.2	Resources	184

8.7.2.1	Overview	184

8.7.2.2	Resource: EEC Contexts	185

8.7.2.2.1	Description	185

8.7.2.2.2	Resource Definition	185

8.7.2.2.3	Resource Standard Methods	185

8.7.2.2.3.1	GET	185

8.7.2.2.3.2	POST	186

8.7.2.2.4	Resource Custom Operations	187

8.7.3	Custom Operations without associated resources	187

8.7.4	Notifications	187

8.7.5	Data Model	187

8.7.5.1	General	187

8.7.5.2	Structured data types	188

8.7.5.2.1	Introduction	188

8.7.5.2.2	Type: SessionContexts	188

8.7.5.2.3	Type: IndividualSessionContext	188

8.7.5.2.4	Type: EECContextPush	189

8.7.5.2.5	Type: EECContext	190

8.7.5.2.6	Type: EECContextPushRes	190

8.7.5.2.7	Type: ImplicitRegDetails	191

8.7.5.2.8	Type: EECSrvContinuitySupport	191

8.7.5.3	Simple data types and enumerations	191

8.7.5.3.1	Introduction	191

8.7.5.3.2	Simple data types	191

8.7.5.4	Data types describing alternative data types or combinations of data types	191

8.7.5.5	Binary data	192

8.7.5.5.1	Binary Data Types	192

8.7.6	Error Handling	192

8.7.6.1	General	192

8.7.6.2	Protocol Errors	192

8.7.6.3	Application Errors	192

8.7.7	Feature negotiation	192

8.8	Eees_EELManagedACR API	193

8.8.1	Introduction	193

8.8.2	Usage of HTTP	193

8.8.3	Resources	193

8.8.3.1	Overview	193

8.8.3.2	Resource: ACT Status Subscriptions	194

8.8.3.2.1	Description	194

8.8.3.2.2	Resource Definition	194

8.8.3.2.3	Resource Standard Methods	194

8.8.3.2.3.1	GET	195

8.8.3.2.3.2	POST	195

8.8.3.2.4	Resource Custom Operations	196

8.8.3.3	Resource: Individual ACT Status Subscription	196

8.8.3.3.1	Description	196

8.8.3.3.2	Resource Definition	196

8.8.3.3.3	Resource Standard Methods	196

8.8.3.3.3.1	GET	197

8.8.3.3.4	Resource Custom Operations	197

8.8.4	Custom Operations without associated resources	198

8.8.4.1	Overview	198

8.8.4.2	Operation: RequestEELManagedACR	198

8.8.4.2.1	Description	198

8.8.4.2.2	Operation Definition	198

8.8.5	Notifications	199

8.8.5.1	General	199

8.8.5.2	ACT Status Notification	199

8.8.5.2.1	Description	199

8.8.5.2.2	Target URI	199

8.8.5.2.3	Standard Methods	200

8.8.5.2.3.1	POST	200

8.8.6	Data Model	201

8.8.6.1	General	201

8.8.6.2	Structured data types	201

8.8.6.2.1	Introduction	201

8.8.6.2.2	Type: EELACRReq	201

8.8.6.2.3	Type: EELACRResp	202

8.8.6.2.4	Type: ACTStatusSubsc	202

8.8.6.2.5	Type: ACTStatusNotif	202

8.8.6.3	Simple data types and enumerations	202

8.8.6.3.1	Introduction	202

8.8.6.3.2	Simple data types	202

8.8.6.4	Data types describing alternative data types or combinations of data types	203

8.8.6.5	Binary data	203

8.8.6.5.1	Binary Data Types	203

8.8.7	Error Handling	203

8.8.7.1	General	203

8.8.7.2	Protocol Errors	203

8.8.7.3	Application Errors	203

8.8.8	Feature negotiation	203

8.9	Eees_ACRStatusUpdate API	203

8.9.1	Introduction	203

8.9.2	Usage of HTTP	204

8.9.3	Resources	204

8.9.4	Custom Operations without associated resources	204

8.9.4.1	Overview	204

8.9.4.2	Operation: RequestACRUpdate	205

8.9.4.2.1	Description	205

8.9.4.2.2	Operation Definition	205

8.9.5	Notifications	205

8.9.6	Data Model	206

8.9.6.1	General	206

8.9.6.2	Structured data types	206

8.9.6.2.1	Introduction	206

8.9.6.2.2	Type: ACRUpdateData	207

8.9.6.2.3	Type: ACRDataStatus	207

8.9.6.2.4	Type: ACTResultInfo	208

8.9.6.2.5	Type: AppGroupInfo	208

8.9.6.3	Simple data types and enumerations	208

8.9.6.3.1	Introduction	208

8.9.6.3.2	Simple data types	208

8.9.6.3.3	Enumeration: ACTResult	208

8.9.6.3.4	Enumeration: E3SubscsStatus	208

8.9.6.3.5	Enumeration: ACTFailureCause	209

8.9.6.4	Data types describing alternative data types or combinations of data types	209

8.9.6.5	Binary data	209

8.9.6.5.1	Binary Data Types	209

8.9.7	Error Handling	209

8.9.7.1	General	209

8.9.7.2	Protocol Errors	209

8.9.7.3	Application Errors	209

8.9.8	Feature negotiation	210

8.10	Eees_ACRParameterInformation API	210

8.10.1	Introduction	210

8.10.2	Usage of HTTP	210

8.10.3	Resources	210

8.10.4	Custom Operations without associated resources	211

8.10.4.1	Overview	211

8.10.4.2	Operation: Request	211

8.10.4.2.1	Description	211

8.10.4.2.2	Operation Definition	211

8.10.5	Notifications	212

8.10.6	Data Model	212

8.10.6.1	General	212

8.10.6.2	Structured data types	213

8.10.6.2.1	Introduction	213

8.10.6.2.2	Type: ACRParamsInfo	213

8.10.6.3	Simple data types and enumerations	213

8.10.6.3.1	Introduction	213

8.10.6.3.2	Simple data types	213

8.10.6.4	Data types describing alternative data types or combinations of data types	213

8.10.6.5	Binary data	214

8.10.6.5.1	Binary Data Types	214

8.10.7	Error Handling	214

8.10.7.1	General	214

8.10.7.2	Protocol Errors	214

8.10.7.3	Application Errors	214

8.10.8	Feature negotiation	214

8.11	Eees_CommonEASAnnouncement API	214

8.11.1	Introduction	214

8.11.2	Usage of HTTP	215

8.11.3	Resources	215

8.11.4	Custom Operations without associated resources	215

8.11.4.1	Overview	215

8.11.4.2	Operation: Declare	215

8.11.4.2.1	Description	215

8.11.4.2.2	Operation Definition	216

8.11.5	Notifications	216

8.11.6	Data Model	216

8.11.6.1	General	216

8.11.6.2	Structured data types	217

8.11.6.2.1	Introduction	217

8.11.6.2.2	Type: CommonEASInfo	217

8.11.6.2.3	Type: CommonEASInfoDecResp	218

8.11.6.2.4	Type: GrpConnInfo	218

8.11.6.3	Simple data types and enumerations	218

8.11.6.3.1	Introduction	218

8.11.6.3.2	Simple data types	218

8.11.6.4	Data types describing alternative data types or combinations of data types	218

8.11.6.5	Binary data	219

8.11.6.5.1	Binary Data Types	219

8.11.7	Error Handling	219

8.11.7.1	General	219

8.11.7.2	Protocol Errors	219

8.11.7.3	Application Errors	219

8.11.8	Feature negotiation	219

8.12	Eees_TrafficInfluenceEAS API	219

8.12.1	Introduction	219

8.12.2	Usage of HTTP	220

8.12.3	Resources	220

8.12.3.1	Overview	220

8.12.3.2	Resource: Application Traffic Influence	221

8.12.3.2.1	Description	221

8.12.3.2.2	Resource Definition	221

8.12.3.2.3	Resource Standard Methods	221

8.12.3.2.3.1	POST	221

8.12.3.2.4	Resource Custom Operations	222

8.12.3.3	Resource: Individual Application Traffic Influence	222

8.12.3.3.1	Description	222

8.12.3.3.2	Resource Definition	222

8.12.3.3.3	Resource Standard Methods	222

8.12.3.3.3.1	GET	222

8.12.3.3.3.2	PUT	223

8.12.3.3.3.3	PATCH	224

8.12.3.3.3.4	DELETE	226

8.12.3.3.4	Resource Custom Operations	226

8.12.4	Notifications	227

8.12.5	Data Model	227

8.12.5.1	General	227

8.12.5.2	Structured data types	227

8.12.5.2.1	Introduction	227

8.12.5.2.2	Type: AppTrafficInfluence	228

8.12.5.2.3	Type: AppTrafficInfluencePatch	229

8.12.5.3	Simple data types and enumerations	229

8.12.5.3.1	Introduction	229

8.12.5.3.2	Simple data types	229

8.12.5.4	Data types describing alternative data types or combinations of data types	229

8.12.5.5	Binary data	229

8.12.5.5.1	Binary Data Types	229

8.12.6	Error Handling	230

8.12.6.1	General	230

8.12.6.2	Protocol Errors	230

8.12.6.3	Application Errors	230

8.12.7	Feature negotiation	230

8A	CAS API Definitions	230

8A.1	Ecas_SelectedEES API	230

8A.1.1	Introduction	230

8A.1.2	Usage of HTTP	231

8A.1.3	Resources	231

8A.1.4	Custom Operations without associated resources	231

8A.1.4.1	Overview	231

8A.1.4.2	Operation: Declare	231

8A.1.4.2.1	Description	231

8A.1.4.2.2	Operation Definition	231

8A.1.5	Notifications	232

8A.1.6	Data Model	232

8A.1.6.1	General	232

8A.1.6.2	Structured data types	233

8A.1.6.2.1	Introduction	233

8A.1.6.2.2	Type: SelEESDecInfo	233

8A.1.6.3	Simple data types and enumerations	233

8A.1.6.3.1	Introduction	233

8A.1.6.3.2	Simple data types	233

8A.1.6.4	Data types describing alternative data types or combinations of data types	233

8A.1.6.5	Binary data	234

8A.1.6.5.1	Binary Data Types	234

8A.1.7	Error Handling	234

8A.1.7.1	General	234

8A.1.7.2	Protocol Errors	234

8A.1.7.3	Application Errors	234

8A.1.8	Feature negotiation	234

9	Edge Configuration Server API Definitions	234

9.1	Eecs_EESRegistration API	234

9.1.1	Introduction	234

9.1.2	Resources	235

9.1.2.1	Overview	235

9.1.2.2	Resource: EES Registrations	236

9.1.2.2.1	Description	236

9.1.2.2.2	Resource Definition	236

9.1.2.2.3	Resource Standard Methods	236

9.1.2.2.3.1	POST	236

9.1.2.2.4	Resource Custom Operations	237

9.1.2.3	Resource: Individual EES Registration	237

9.1.2.3.1	Description	237

9.1.2.3.2	Resource Definition	237

9.1.2.3.3	Resource Standard Methods	237

9.1.2.3.3.1	GET	237

9.1.2.3.3.2	PUT	238

9.1.2.3.3.3	DELETE	239

9.1.2.3.3.4	PATCH	240

9.1.2.3.4	Resource Custom Operations	241

9.1.3	Custom Operations without associated resources	241

9.1.4	Notifications	241

9.1.5	Data Model	241

9.1.5.1	General	241

9.1.5.2	Structured data types	243

9.1.5.2.1	Introduction	243

9.1.5.2.2	Type: EESRegistration	243

9.1.5.2.3	Type: EESProfile	244

9.1.5.2.4	Type: EESRegistrationPatch	246

9.1.5.2.5	Type: ServiceArea	247

9.1.5.2.6	Type: TopologicalServiceArea	247

9.1.5.2.7	Type: GeographicalServiceArea	247

9.1.5.2.8	Type: EASInstantiationInfo	247

9.1.5.2.9	Type: InstantiationCriteria	248

9.1.5.2.10	Type: EDNInfo	248

9.1.5.2.11	Type: LoadInfo	248

9.1.5.2.12	Type: UeServSatInfo	249

9.1.5.2.13	Type: AppSatCovAvailInfo	249

9.1.5.3	Simple data types and enumerations	249

9.1.5.3.1	Introduction	249

9.1.5.3.2	Simple data types	249

9.1.5.3.3	Enumeration: ACRScenario	249

9.1.5.3.4	Enumeration: InstantiationStatus	250

9.1.6	Error Handling	250

9.1.6.1	General	250

9.1.6.2	Protocol Errors	250

9.1.6.3	Application Errors	250

9.1.7	Feature negotiation	250

9.2	Eecs_TargetEESDiscovery API	251

9.2.1	Introduction	251

9.2.2	Resources	251

9.2.2.1	Overview	251

9.2.2.2	Resource: EES Profiles	252

9.2.2.2.1	Description	252

9.2.2.2.2	Resource Definition	252

9.2.2.2.3	Resource Standard Methods	252

9.2.2.2.3.1	GET	252

9.2.2.2.4	Resource Custom Operations	254

9.2.3	Custom Operations without associated resources	254

9.2.4	Notifications	254

9.2.5	Data Model	254

9.2.5.1	General	254

9.2.5.2	Structured data types	255

9.2.5.2.1	Introduction	255

9.2.5.2.2	Type: TunnelInfo	255

9.2.5.2.3	Type: CandidateDnaisInfo	256

9.2.5.3	Simple data types and enumerations	256

9.2.6	Error Handling	256

9.2.7	Feature negotiation	256

9.3	Eecs_EASInfoManagement API	257

9.3.1	Introduction	257

9.3.2	Usage of HTTP	257

9.3.3	Resources	258

9.3.3.1	Overview	258

9.3.3.2	Resource: Common EAS Bindings	258

9.3.3.2.1	Description	258

9.3.3.2.2	Resource Definition	258

9.3.3.2.3	Resource Standard Methods	259

9.3.3.2.3.1	GET	259

9.3.3.2.3.2	POST	260

9.3.3.2.4	Resource Custom Operations	261

9.3.3.3	Resource: Individual Common EAS Binding	261

9.3.3.3.1	Description	261

9.3.3.3.2	Resource Definition	261

9.3.3.3.3	Resource Standard Methods	261

9.3.3.3.3.1	GET	261

9.3.3.3.3.2	PUT	262

9.3.3.3.3.3	PATCH	263

9.3.3.3.3.4	DELETE	264

9.3.4	Custom Operations without associated resources	265

9.3.5	Notifications	265

9.3.6	Data Model	266

9.3.6.1	General	266

9.3.6.2	Structured data types	266

9.3.6.2.1	Introduction	266

9.3.6.2.2	Type: CommonEASBindReq	267

9.3.6.2.3	Type: CommonEASBindResp	267

9.3.6.2.4	Type: CommonEASBinding	268

9.3.6.2.5	Type: CommEasBdlInfo	269

9.3.6.2.6	Type: CommonEASBindingPatch	269

9.3.6.3	Simple data types and enumerations	270

9.3.6.3.1	Introduction	270

9.3.6.3.2	Simple data types	270

9.3.6.4	Data types describing alternative data types or combinations of data types	270

9.3.6.4.1	Type: ProblemDetailsEIMExt	270

9.3.6.5	Binary data	270

9.3.6.5.1	Binary Data Types	270

9.3.7	Error Handling	270

9.3.7.1	General	270

9.3.7.2	Protocol Errors	270

9.3.7.3	Application Errors	270

9.3.8	Feature negotiation	271

9.4	Eecs_ECSServiceProvisioning API	271

9.4.1	Introduction	271

9.4.2	Usage of HTTP	271

9.4.3	Resources	272

9.4.3.1	Overview	272

9.4.3.2	Resource: Service Provisioning Subscriptions	272

9.4.3.2.1	Description	272

9.4.3.2.2	Resource Definition	272

9.4.3.2.3	Resource Standard Methods	273

9.4.3.2.3.2	POST	273

9.4.3.2.4	Resource Custom Operations	273

9.4.3.3	Resource: Individual Service Provisioning Subscription	273

9.4.3.3.1	Description	273

9.4.3.3.2	Resource Definition	274

9.4.3.3.3	Resource Standard Methods	274

9.4.3.3.3.1	GET	274

9.4.3.3.3.2	PUT	275

9.4.3.3.3.3	PATCH	276

9.4.3.3.3.4	DELETE	277

9.4.4	Custom Operations without associated resources	278

9.4.4.1	Overview	278

9.4.4.2	Operation: Request	279

9.4.4.2.1	Description	279

9.4.4.2.2	Operation Definition	279

9.4.5	Notifications	280

9.4.5.1	General	280

9.4.5.2	Service Provisioning Notification	281

9.4.5.2.1	Description	281

9.4.5.2.2	Target URI	281

9.4.5.2.3	Standard Methods	281

9.4.6	Data Model	282

9.4.6.1	General	282

9.4.6.2	Structured data types	283

9.4.6.2.1	Introduction	283

9.4.6.2.2	Type: ServProvReq	283

9.4.6.2.3	Type: ServProvResp	283

9.4.6.2.4	Type: ServProvSubsc	284

9.4.6.2.5	Type: ServProvSubscPatch	284

9.4.6.2.6	Type: ServProvNotif	285

9.4.6.2.7	Type: FederationInfo	285

9.4.6.2.8	Type: AppInfo	285

9.4.6.2.9	Type: AppGrpProfile	285

9.4.6.3	Simple data types and enumerations	285

9.4.6.3.1	Introduction	285

9.4.6.3.2	Simple data types	285

9.4.6.4	Data types describing alternative data types or combinations of data types	286

9.4.6.5	Binary data	286

9.4.6.5.1	Binary Data Types	286

9.4.7	Error Handling	286

9.4.7.1	General	286

9.4.7.2	Protocol Errors	286

9.4.7.3	Application Errors	286

9.4.8	Feature negotiation	286

9.4.9	Security	287

9.5	Eecs_ECSDiscovery API	287

9.5.1	Introduction	287

9.5.2	Resources	287

9.5.2.1	Overview	287

9.5.2.2	Resource: ECS Information	288

9.5.2.2.1	Description	288

9.5.2.2.2	Resource Definition	288

9.5.2.2.3	Resource Standard Methods	288

9.5.2.2.4	Resource Custom Operations	288

9.5.2.2.4.1	Overview	288

9.5.2.2.4.2	Operation: Discover	289

9.5.2.2.4.2.1	Description	289

9.5.2.2.4.2.2	Operation Definition	289

9.5.3	Custom Operations without associated resources	290

9.5.4	Notifications	290

9.5.4.1	General	290

9.5.4.2	ECS Discovery Notification	290

9.5.4.2.1	Description	290

9.5.4.2.2	Target URI	290

9.5.4.2.3	Standard Methods	290

9.5.4.2.3.1	POST	290

9.5.5	Data Model	291

9.5.5.1	General	291

9.5.5.2	Structured data types	292

9.5.5.2.1	Introduction	292

9.5.5.2.2	Type: EcsInfoDiscoveryReq	293

9.5.5.2.3	Type: EcsInfoDiscoveryResp	293

9.5.5.2.4	Type: EcsInfo	293

9.5.5.2.5	Type: ECSProfile	294

9.5.5.2.6	Type: SupportedPlmn	294

9.5.5.2.7	Type: SupportedEcsp	294

9.5.5.2.8	Type: PduConfiguration	294

9.5.5.2.9	Type: EcsInfoDiscNotif	295

9.5.5.3	Simple data types and enumerations	295

9.5.5.3.1	Introduction	295

9.5.5.3.2	Simple data types	295

9.5.5.4	Data types describing alternative data types or combinations of data types	295

9.5.5.5	Binary data	295

9.5.5.5.1	Binary Data Types	295

9.5.6	Error Handling	295

9.5.6.1	General	295

9.5.6.2	Protocol Errors	295

9.5.6.3	Application Errors	295

9.5.7	Feature negotiation	296

9.6	Eecs_ACREvents API	296

9.6.1	Introduction	296

9.6.2	Usage of HTTP	296

9.6.3	Resources	296

9.6.3.1	Overview	296

9.6.3.2	Resource: ACR Events Subscriptions	297

9.6.3.2.1	Description	297

9.6.3.2.2	Resource Definition	297

9.6.3.2.3	Resource Standard Methods	298

9.6.3.2.3.2	POST	298

9.6.3.2.4	Resource Custom Operations	298

9.6.3.3	Resource: Individual ACR Events Subscription	298

9.6.3.3.1	Description	298

9.6.3.3.2	Resource Definition	298

9.6.3.3.3	Resource Standard Methods	299

9.6.3.3.3.1	GET	299

9.6.3.3.3.2	PUT	300

9.6.3.3.3.3	PATCH	301

9.6.3.3.3.4	DELETE	302

9.6.3.3.4	Resource Custom Operations	303

9.6.4	Custom Operations without associated resources	303

9.6.5	Notifications	303

9.6.5.1	General	303

9.6.5.2	ACR Events Notification	304

9.6.5.2.1	Description	304

9.6.5.2.2	Target URI	304

9.6.5.2.3	Standard Methods	304

9.6.6	Data Model	305

9.6.6.1	General	305

9.6.6.2	Structured data types	306

9.6.6.2.1	Introduction	306

9.6.6.2.2	Type: AcrEventsSubsc	306

9.6.6.2.3	Type: AcrEventsSubscPatch	307

9.6.6.2.4	Type: AcrEventsNotif	307

9.6.6.3	Simple data types and enumerations	307

9.6.6.3.1	Introduction	307

9.6.6.3.2	Simple data types	307

9.6.6.3.3	Enumeration: AcrEvent	307

9.6.6.4	Data types describing alternative data types or combinations of data types	308

9.6.6.5	Binary data	308

9.6.6.5.1	Binary Data Types	308

9.6.7	Error Handling	308

9.6.7.1	General	308

9.6.7.2	Protocol Errors	308

9.6.7.3	Application Errors	308

9.6.8	Feature negotiation	308

9.6.9	Security	309

10	Using Common API Framework	309

10.1	General	309

10.2	Security	309

11	Security	310

Annex A (normative): OpenAPI specification	310

A.1	General	310

A.2	Eees_EASRegistration API	310

A.3	Eees_UELocation API	319

A.4	Eees_UEIdentifier API	327

A.5	Eees_AppClientInformation API	330

A.6	Eees_SessionWithQoS API	336

A.7	Eees_ACRManagementEvent API	343

A.8	Eees_EECContextRelocation API	355

A.9	Eees_EELManagedACR API	358

A.10	Eees_ACRStatusUpdate API	362

A.11	Eecs_EESRegistration API	365

A.12	Eecs_TargetEESDiscovery API	373

A.13	Eees_ACRParameterInformation API	376

A.14	Ecas_SelectedEES API	378

A.15	Eees_CommonEASAnnouncement API	379

A.16	Eecs_EASInfoManagement API	381

A.17	Eees_TrafficInfluenceEAS API	387

A.18	Eecs_ECSServiceProvisioning API	392

A.19	Eecs_ECSDiscovery API	398

A.20	Eecs_ACREvents API	402

Annex B (informative): Change history	408

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the APIs for enabling the edge applications over 3GPP networks. The application layer architecture, functional requirements, procedures and information flows necessary for enabling edge applications over 3GPP networks are specified in 3GPP TS 23.558 [2]. The APIs are specified as RESTful APIs except for custom operations wherever required.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[3]	Open API: "OpenAPI Specification Version 3.0.0.", https://spec.openapis.org/oas/v3.0.0.

[4]	3GPP TR 21.900: "Technical Specification Group working methods".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	3GPP TS 29.122: "T8 reference point for Northbound Application Programming Interfaces (APIs)".

[7]	IETF RFC 6455: "The Websocket Protocol".

[8]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[9]	Void.

[10]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[11]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[12]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[13]	3GPP TS 29.523: "5G System; Policy Control Event Exposure Service; Stage 3".

[14]	3GPP TS 24.558: "Enabling Edge Applications; Protocol specification".

[15]	3GPP TS 29.214: "Policy and charging control over Rx reference point".

[16]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[17]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs".

[18]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[19]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[20]	3GPP TS 33.558: "Security aspects of enhancement of support for enabling edge applications; Stage 2".

[21]	Void.

[22]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[23]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[24]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[25]	Void.

[26]	3GPP TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the following terms and its definitions given in 3GPP TS 23.558 [2] shall apply:

Application Context

Application Context Relocation

Application Context Transfer

Application Server

Edge Computing Service Provider

Edge Data Network

EEC Context

Edge Hosting Environment

Instantiable EAS

Partner ECS

Partner ECSP

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Application Client

ACID	Application Client Identification

ACR	Application Context Relocation

AEF	API Exposing Function

AF	Application Function

ASP	Application Service Provider

CAPIF	Common API Framework

CAS	Cloud Application Server

CES	Cloud Enabler Server

DN	Data Network

DNAI	Data Network Access Identifier

DNN	Data Network Name

EAS	Edge Application Server

EASID	Edge Application Server Identification

ECI	Edge and Cloud Interworking

ECS	Edge Configuration Server

ECSP	Edge Computing Service Provider

EDN	Edge Data Network

EEC	Edge Enabler Client

EECID	Edge Enabler Client Identification

EEL	Edge Enabler Layer

EES	Edge Enabler Server

EESID	Edge Enabler Server Identification

EHE	Edge Hosting Environment

FQDN	Fully Qualified Domain Name

GPSI	Generic Public Subscription Identifier

LADN	Local Area Data Network

NEF	Network Exposure Function

NID	Network Identifier

PCF	Policy Control Function

S-EAS	Source Edge Application Server

S-EES	Source Edge Enabler Server

SCEF	Service Capability Exposure Function

SEAL	Service Enabler Architecture Layer for Verticals

SEALDD	SEAL Data Delivery

SMF	Session Management Function

SNPN	Stand-alone Non-Public Network

SSID	Service Set Identifier

T-EAS	Target Edge Application Server

T-EES	Target Edge Enabler Server

TAI	Tracking Area Identity

UAS	Uncrewed Aerial System

# 4	Overview

3GPP TS 23.558 [2] has specified the application layer architecture, requirements, procedures, information flows and the APIs, in order to support the edge applications over the 3GPP systems. Various features are defined to ensure the efficient use and deployment of edge applications, some of which include, registration, discovery, service provisioning, capability exposure, support for service continuity and support for roaming and federation.

The present document specifies the north-bound APIs in detail, needed to support the services offered by EES and ECS over EDGE-3/6/9/10 interfaces and by CES and CAS over ECI-1/2/3/4 interfaces for enabling the edge applications over 3GPP network.

# 5	Services offered by Edge Enabler Server

## 5.1	Introduction

The table 5.1-1 lists the EES APIs below the service name. A service description clause for each API gives a general description of the related API.

Table 5.1-1: List of EES Service APIs

| Service Name | Service Operations | Operation Semantics | Known Consumer(s) |
| --- | --- | --- | --- |
| Eees_EASRegistration | Request | Request/Response | EAS |
|  | Update | Request/Response | EAS |
|  | Deregister | Request/Response | EAS |
| Eees_UELocation | Get | Request/Response | EAS |
|  | Subscribe | Subscribe/Notify | EAS |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |
| Eees_UEIdentifier | Get | Request/Response | EAS, EEC |
| Eees_AppClientInformation | Subscribe | Subscribe/Notify | EAS |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |
| Eees_SessionWithQoS | Create | Request/Response | EAS |
|  | Update | Request/Response | EAS |
|  | Revoke | Request/Response | EAS |
|  | Notify | Subscribe/Notify | EAS |
| Eees_EASDiscovery | TEasDiscRequest | Request/Response | EAS, EES, CAS |
| Eees_ACRManagementEvent | Subscribe | Subscribe/Notify | EAS, CAS, EES |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |
| Eees_EECContextRelocation | Push | Request/Response | EES, CES |
|  | Pull | Request/Response | EES |
| Eees_EELManagedACR | Request | Request/Response | EAS, EES |
|  | Subscribe | Subscribe/Notify | EAS, EES |
|  | Notify |  |  |
| Eees_ACRStatusUpdate | Request | Request/Response | EAS, CAS, EES |
| Eees_AppContextRelocation | ACRDetermination_Request | Request/Response | EAS, EES |
|  | SelectedTargetEAS_Declare | Request/Response | EAS, CAS, EES |
| Eees_ACRParameterInformation | Request | Request/Response | EES, CES |
| Eees_CommonEASAnnouncement | Declare | Request/Response | EES |
| Eees_TrafficInfluenceEAS | Manage | Request/Response | EAS |
| Eees_EASInformationProvisioning | Declare | Request/Response | EES |

Table 5.1-2 summarizes the corresponding EES APIs defined in this specification.

Table 5.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Eees_EASRegistration | 5.2 | EES EAS registration service | TS29558_Eees_EASRegistration.yaml | eees-easregistration | A.2 |
| Eees_UELocation | 5.3 | EES Service for fetching UE location information | TS29558_Eees_UELocation.yaml | eees-uelocation | A.3 |
| Eees_UEIdentifier | 5.4 | EES Service for fetching UE identifier. | TS29558_Eees_UEIdentifier.yaml | eees-ueidentifier | A.4 |
| Eees_AppClientInformation | 5.5 | EES Service to obtain the capabilities of the ACs. | TS29558_Eees_AppClientInformation.yaml | eees-appclientinformation | A.5 |
| Eees_SessionWithQoS | 5.6 | EES Service to setup data session between AC and EAS with specific QoS. | TS29558_Eees_SessionWithQoS.yaml | eees-session-with-qos | A.6 |
| Eees_ACRManagementEvent | 5.8 | EES Service to receive notification related to ACR management events. | TS29558_Eees_ACRManagementEvent.yaml | eees-acrmgntevent | A.7 |
| Eees_EECContextRelocation | 5.10 | EES Service to push or pull EEC context information. | TS29558_Eees_EECContextRelocation.yaml | eees-eeccontextreloc | A.8 |
| Eees_EELManagedACR | 5.11 | EES Service to request for handling of ACR related operations and receive ACT notifications. | TS29558_Eees_EELManagedACR.yaml | eees-eel-acr | A.9 |
| Eees_ACRStatusUpdate | 5.12 | EES Service to update the status of ACR. | TS29558_Eees_ACRStatusUpdate.yaml | eees-acrstatus-update | A.10 |
| Eees_ACRParameterInformation | 5.13 | EES Service to send ACR parameters information. | TS29558_Eees_ACRParameterInformation.yaml | eees-acr-param | A.13 |
| Eees_CommonEASAnnouncement | 5.14 | EES Common EAS Announcement Service | TS29558_Eees_CommonEASAnnouncement.yaml | eees-cea | A.15 |
| Eees_TrafficInfluenceEAS | 5.15 | Service to trigger application traffic influence | TS29558_Eees_TrafficInfluenceEAS.yaml | eees-tie | A.17 |

## 5.2	Eees_EASRegistration Service

### 5.2.1	Service Description

The Eees_EASRegistration API, as defined in 3GPP TS 23.558 [2], allows an service consumer via Eees interface to register, update its registration and deregister at a given EES.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The service operations defined for the Eees_EASRegistration API are shown in the table 5.2.2.1-1.

Table 5.2.2.1-1: Operations of the Eees_EASRegistration API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_EASRegistration_Request | This service operation is used by the service consumer to register itself to a given EES. | e.g., EAS |
| Eees_EASRegistration_Update | This service operation is used by the service consumer to update its registration information at EES. | e.g., EAS |
| Eees_EASRegistration_Deregister | This service operation is used by the service consumer to deregister itself from a given EES. | e.g., EAS |

#### 5.2.2.2	Eees_EASRegistration_Request

##### 5.2.2.2.1	General

This service operation is used by service consumer to register itself to a given EES.

##### 5.2.2.2.2	Service consumer registering to EES using Eees_EASRegistration_Request operation

To register itself at the EES, the service consumer shall send an HTTP POST request message to the EES targeting the "EAS Registrations" collection resource, with the request body including the EASRegistration data structure as specified in clause 8.1.2.2.3.1.

Upon reception of the HTTP POST request message, the EES shall:

1.	verify the identity of the service consumer and check if the service consumer is authorized to register itself at EES;

2.	if the service consumer is authorized to register at the EES, then the EES shall:

a.	create a new "Individual EAS Registration" resource with the received registration information; and

b.	respond with an HTTP "201 Created" status code with the response body including a representation of the created "Individual EAS Registration" resource within the EASRegistration data structure and an HTTP "Location" header field containing the URI of the created resource, as specified in clause 8.1.2.2.3.1;

and

3.	On failure, the EES shall take proper error handling actions, as specified in clause 8.1.6, and respond with an appropriate error status code.

If an expiration time was provided for the resource as part of the created resource representation in step 2b above, then to maintain the registration at the EES, the service consumer shall send a registration update request (as defined in clause 5.2.2.3) to update the registration prior to the expiration time. If the registration update request is not sent before the expiry time, then the EES shall delete the corresponding "Individual EAS Registration" resource.

#### 5.2.2.3	Eees_EASRegistration_Update

##### 5.2.2.3.1	General

This service operation is used by service consumer to update its registration information at a given EES.

##### 5.2.2.3.2	Service consumer updating registration information using Eees_EASRegistration_Update operation

To update its registration information at the EES, the service consumer shall send an HTTP PUT or PATCH request message to the EES targeting the corresponding "Individual EAS Registration" resource, with the request body including either:

-	the EASRegistration data structure as specified in clause 8.1.2.3.3.2, in case of HTTP PUT; or

-	the EASRegistrationPatch data structure as specified in clause 8.1.2.3.3.4, in case of HTTP PATCH.

This request shall not replace the "easId" attribute of the existing resource.

Upon reception of the HTTP PUT or PATCH message, the EES shall:

1.	verify the identity of the service consumer and check if the service consumer is authorized to update the targeted registration; and

2.	if the service consumer is authorized, then the EES shall:

a.	update the corresponding "Individual EAS Registration" resource accordingly;

b.	upon success, respond to the EAS with either:

-	an HTTP "204 No Content" status code; or

-	an HTTP "200 OK" status code with the response body including a representation of the updated resource within the EASRegistration data structure as specified in clauses 8.1.2.3.3.2 and 8.1.2.3.3.4.

On failure, the EES shall take proper error handling actions, as specified in clause 8.1.6, and respond with an appropriate error status code.

If the EES determines that the received HTTP PUT or PATCH request needs to be redirected, the EES may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative EES towards which the message should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

If a new or updated expiration time was provided for the resource as part of the updated resource representation in step 2b above, then to maintain the registration at the EES, the service consumer shall send a registration update request (as defined in clause 5.2.2.3) to update the registration prior to the expiration time. If the registration update request is not sent before the expiry time, then the EES shall delete the corresponding "Individual EAS Registration" resource.

#### 5.2.2.4	Eees_EASRegistration_Deregister

##### 5.2.2.4.1	General

This service operation is used by service consumer to deregister itself from a given EES.

##### 5.2.2.4.2	Service consumer deregistering from EES using Eees_EASRegistration_Deregister operation

To deregister itself from the EES, the service consumer shall send an HTTP DELETE request message to the EES targeting the corresponding "Individual EAS Registration" resource as specified in clause 8.1.2.3.3.3.

Upon reception of the HTTP DELETE request, the EES shall:

1.	verify the identity of the service consumer and check if the service consumer is authorized to delete the targeted registration;

2.	if the service consumer is authorized, then the EES shall delete the corresponding "Individual EAS Registration" resource; and

3.	respond with an "204 Not Content" status code.

On failure, the EES shall take proper error handling actions, as specified in clause 8.1.6, and respond with an appropriate error status code.

If the EES determines that the received HTTP DELETE request needs to be redirected, the EES may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative EES towards which the message should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

## 5.3	Eees_UELocation Service

### 5.3.1	Service Description

The Eees_UELocation API, as defined in 3GPP TS 23.558 [2], allows a service consumer via the Eees interface to obtain the UE location information either via one-time request/response or a subscription for continuous reporting.

### 5.3.2	Service Operations

#### 5.3.2.1	Introduction

The service operations defined for the Eees_UELocation API are shown in the table 5.3.2.1-1.

Table 5.3.2.1-1: Operations of the Eees_UELocation API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_UELocation_Get | This service operation is used by the service consumer to request UE location information to the EES. | e.g., EAS |
| Eees_UELocation_Subscribe | This service operation is used by the service consumer to subscribe to continuous reporting of UE location information at the EES. | e.g., EAS |
| Eees_UELocation_Notify | This service operation is used by the EES to notify about UE location information. | EES |
| Eees_UELocation_UpdateSubscription | This service operation is used by the service consumer to update an existing subscription for continuous reporting of UE location information at the EES. | e.g., EAS |
| Eees_UELocation_Unsubscribe | This service operation is used by the service consumer to remove an existing subscription for continuous reporting of UE location information at the EES. | e.g., EAS |

#### 5.3.2.2	Eees_UELocation_Get

##### 5.3.2.2.1	General

This service operation is used by a service consumer to obtain a UE's location information from the EES.

##### 5.3.2.2.2	Service consumer obtaining UE location information from EES using Eees_UELocation_Get operation

To obtain a UE's location information from the EES, the service consumer shall send an HTTP POST message to the EES targeting the URI of the "Fetch" custom operation (i.e., "{apiRoot}/eees-uelocation/<apiVersion>/fetch") as specified in clause 8.2.3.2, with the request body including the LocationRequest data structure.

Upon reception of the HTTP POST request, the EES shall:

1.	verify the identity of the service consumer and check if the service consumer is authorized to obtain UE location information; and

2.	if the service consumer is authorized, then the EES shall:

a.	consider the location granularity information received in the request message to obtain the UE's location information;

b.	check if a valid locally cached UE location information is available;

c.	if valid UE location information is not available in local cache, then the EES shall obtain the UE location information by consuming the relevant 3GPP core network capabilities; and

d.	upon success, respond to the service consumer with an HTTP "200 OK" status code with the response body including the LocationResponse data structure.

On failure, the EES shall take proper error handling actions, as specified in clause 8.2.6, and respond to the service consumer with an appropriate error status code.

If the EES determines that the received HTTP POST request needs to be redirected, the EES may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative EES where the message should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

##### 5.3.2.2.3	User consent management

Based on local regulations' requirements and/or operator policies, user consent management specified in Annex V of 3GPP TS 33.501 [6] may be required for accessing the Eees_UELocation API. When it is the case and the EES is accessing the 3GPP 5GC network services directly, the EES shall act as the consent enforcement entity, as specified in clause 5.1.3 of 3GPP TS 33.558 [20].

When user consent management and enforcement shall be undertaken for the Eees_UELocation API, then:

-	the EES shall check user consent for the targeted UE by retrieving the user consent subscription data via the Nudm_SDM service API of the UDM, as specified in clause 5.2.2.2.24 of 3GPP TS 29.503 [22], and:

-	if user consent is not granted for the targeted UE, the EES shall reject the request and respond to the service consumer with an HTTP "403 Forbidden" status code with the response body containing a ProblemDetails data structure with the "cause" attribute including the "USER_CONSENT_NOT_GRANTED" application error; and

-	if user consent is granted for the targeted UE, the EES shall accept the request to retrieve UE location information and process it as specified in clause 5.3.2.2.2.

#### 5.3.2.3	Eees_UELocation_Subscribe

##### 5.3.2.3.1	General

This service operation is used by the service consumer to subscribe for continuous UE(s) location reporting.

##### 5.3.2.3.2	Service consumer subscribing to continuous UE(s) location reporting from EES using Eees_UELocation_Subscribe operation

To subscribe to continuous UE(s) location information reporting at the EES, the service consumer shall send an HTTP POST message to the EES targeting the "Location Information Subscriptions" collection resource, with the request body including the LocationSubscription data structure, as specified in clause 8.2.2.2.3.1.

Upon reception of the HTTP POST request message from the service consumer, the EES shall:

1.	verify the identity of the service consumer and check if the service consumer is authorized to subscribe to continuous UE(s) location reporting; and

2.	if the service consumer is authorized, then the EES shall;

a.	create a corresponding new "Individual Location Information Subscription" resource; and

b.	return an HTTP "201 Created" status code with the response body including a representation of the created resource within the LocationSubscription data structure, and an HTTP "Location" header field containing the URI of the created resource.

On failure, the EES shall take proper error handling actions, as specified in clause 8.2.6, and respond to the service consumer with an appropriate error status code.

The EES shall obtain the UE location information by consuming the relevant 3GPP core network capabilities from the NEF as specified in 3GPP TS 29.522 [10], or from SCEF as specified in 3GPP TS 29.122 [6] or from LCS (Location Service) as specified in 3GPP TS 23.271 [23] or 3GPP TS 23.273 [24]. The EES may also consume the UE mobility analytics from the NEF as specified in 3GPP TS 29.522 [10] or from the NWDAF as specified in 3GPP TS 29.520 [12].

If the expiration time is provided, then to maintain the registration, the service consumer shall send a subscription update request (as described in clause 5.3.2.5) prior to the expiration time. If the subscription update request is not sent before the expiry time, then the EES shall treat the subscription as unsubscribed and delete the corresponding resource.

##### 5.3.2.3.3	User consent management

Based on local regulations' requirements and/or operator policies, user consent management specified in Annex V of 3GPP TS 33.501 [6] may be required for accessing the Eees_UELocation API. When it is the case and the EES is accessing the 3GPP 5GC network services directly, the EES shall act as the consent enforcement entity, as specified in clause 5.1.3 of 3GPP TS 33.558 [20].

When user consent management and enforcement shall be undertaken for the Eees_UELocation API, then:

-	if the service consumer does not support the "UserConsentRevocation" feature or does not indicate its support for this feature in the HTTP POST request to create a new "Individual Location Information Subscription" resource, the EES shall reject the request and respond to the service consumer with an HTTP "403 Forbidden" status code with the response body containing a ProblemDetails data structure with the "cause" attribute including the "CONSENT_REVOCATION_NOT_SUPPORTED" application error;

-	if the service consumer indicates its support for the "UserConsentRevocation" feature in the HTTP POST request to create a new "Individual Location Information Subscription" resource, the EES shall:

-	check user consent for the targeted UE(s) by retrieving the user consent subscription data via the Nudm_SDM service API of the UDM as specified in clause 5.2.2.2.24 of 3GPP TS 29.503 [22];

-	subscribe to user consent revocation notifications only for those UE(s) for which user consent is granted also using the Nudm_SDM service API of the UDM;

-	accept the request for the creation of the "Individual Location Information Subscription" resource only for the UE(s) for which user consent is granted; and

-	if user consent is not granted for all the targeted UE(s), the EES shall reject the request and respond to the service consumer with an HTTP "403 Forbidden" status code with the response body containing a ProblemDetails data structure with the "cause" attribute including the "USER_CONSENT_NOT_GRANTED" application error;

-	the service consumer shall provide within the payload body of the HTTP POST request to create a new "Individual Location Information Subscription" resource the URI via which it desires to receive user consent revocation notifications within the "revocationNotifUri" attribute of the LocationSubscription data structure;

-	the service consumer may update this URI in subsequent HTTP PUT/PATCH requests to update/modify the corresponding "Individual Location Information Subscription" resource;

-	when becoming aware of user consent revocation for one or several UE(s), the EES shall:

-	stop processing the data related to the concerned UE(s);

-	send a user consent revocation notification to the service consumer by sending an HTTP POST request with the request body including the ConsentRevocNotif data structure that shall contain the user consent revocation information (e.g., UE(s) for which user consent was revoked, etc.);

-	remove the concerned UE(s) from the "Individual Location Information Subscription" resource and from the related subscriptions at the 3GPP 5GC network entities, if any; and

-	unsubscribe from user consent revocation notifications for the concerned UE(s) at the UDM;

-	at the reception of a user consent revocation notification from the EES, the service consumer shall take the necessary actions to stop processing the data related to the concerned UE(s); and

-	if user consent is revoked for all the UE(s), the service consumer shall delete the corresponding "Individual Location Information Subscription" resource, as specified in clause 5.3.2.6.

#### 5.3.2.4	Eees_UELocation_Notify

##### 5.3.2.4.1	General

This service operation is used by the EES to send UE(s) location information notifications or user consent revocation notifications to the service consumer.

##### 5.3.2.4.2	EES notifying the UE(s) location reporting to service consumer using Eees_UELocation_Notify operation

The EES determines to notify the service consumer with the UE location information, when the UE location information is available either in the local cache or received from the 3GPP core network.

To notify on UE(s) location information events, the EES shall send an HTTP POST request message targeting the Notification Destination URI received during the corresponding subscription creation/update request as defined in clauses 5.3.2.3.2 and 5.3.2.5.2, with the request body including the LocationNotification data structure.

TheLocationNotification includes location information of each UE with accuracy, and timestamp. The location information of each UE may be the actual location change or predictive location report from the UE mobility analytics report from the NEF as specified in 3GPP TS 29.522 [10], from the SCEF as specified in 3GPP TS 29.122 [6], from LCS (Location Service) as specified in 3GPP TS 29.271 [23] or 3GPP TS 29.273 [24], or from the NWDAF as specified in 3GPP TS 29.520 [12]. The EES may adapt the UE location information in the format requested by the service consumer in the subscription request.

Upon reception the HTTP POST request message and successful processing, the service consumer shall return an HTTP "204 No Content" status code to acknowledge the successful reception of the notification.

On failure, the service consumer shall take proper error handling actions, as specified in clause 8.2.6, and respond to the EES with an appropriate error status code.

If the service consumer determines that the received HTTP POST message needs to be redirected, the service consumer may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

##### 5.3.2.4.3	EES notifying the service consumer about user consent revocation using Eees_UELocation_Notify operation

The EES determines to notify the service consumer about user consent revocation, when becoming aware of user consent revocation for one or several UE(s) from the UDM, as specified in 3GPP TS 29.503 [22].

To notify user consent revocation, the EES shall send an HTTP POST request message using the revocation notification URI received from the service consumer during the corresponding subscription creation/update/modification request as specified in clause 5.3.2.3.2 and 5.3.2.5.2, with the request body including the ConsentRevocNotif data structure.

Upon reception of the HTTP POST request and successful processing, the service consumer shall return an HTTP "204 No Content" status code to acknowledge the successful reception of the notification and take the necessary actions to stop processing the data related to the UE(s) for which user consent was revoked.

On failure, the service consumer shall take proper error handling actions, as specified in clause 8.2.6, and respond to the EES with an appropriate error status code.

If the service consumer determines that the received HTTP POST request message needs to be redirected, the service consumer may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

#### 5.3.2.5	Eees_UELocation_UpdateSubscription

##### 5.3.2.5.1	General

This service operation is used by the service consumer to update its location information subscription at the EES.

##### 5.3.2.5.2	Service consumer updating continuous UE(s) location reporting subscription at EES using Eees_UELocation_UpdateSubscribe operation

To update an existing continuous UE(s) location information reporting subscription at the EES, the service consumer shall send an HTTP PATCH or PUT request message to the EES targeting the corresponding "Individual Location Information Subscription" resource, with the request body including either:

-	the requested modifications to the resource within the LocationSubscriptionPatch data structure as specified in clause 8.2.2.3.3.2, in case the HTTP PATCH method is used; or

-	the updated representation of the resource within the LocationSubscription data structure as specified in clause 8.2.2.3.3.3, in case the HTTP PUT method is used.

The HTTP PUT request shall not update the "easId", "ueId" and "groupId" attributes of the existing resource.

Upon reception of the HTTP PATCH or PUT message from the service consumer, the EES shall:

1.	verify the identity of the service consumer and check if the service consumer is authorized to updated the targeted location information subscription;

2.	if the service consumer is authorized, then the EES shall:

a.	update the targeted "Individual Location Information Subscription" resource accordingly;

b.	upon success, return either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Location Information Subscription" resource within the LocationSubscription data structure; or

-	an HTTP "204 No Content" status code.

On failure, the EES shall take proper error handling actions, as specified in clause 8.2.6, and respond to the service consumer with an appropriate error status code.

If the EES determines that the received HTTP PATCH or PUT request needs to be redirected, the EES may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative EES where the message should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

If the expiration time is provided for the first time or updated, then to maintain the subscription, the service consumer shall send another subscription update request prior to subscription expiry time. If the subscription update request is not received before the expiry time, then the EES shall treat service consumer subscription as unsubscribed and delete the corresponding resource.

##### 5.3.2.5.3	User consent management

Based on local regulations' requirements and/or operator policies, user consent management specified in Annex V of 3GPP TS 33.501 [6] may be required for accessing the Eees_UELocation API. When it is the case and the EES is accessing the 3GPP 5GC network services directly, the EES shall act as the consent enforcement entity, as specified in clause 5.1.3 of 3GPP TS 33.558 [20].

When user consent management and enforcement shall be undertaken for the Eees_UELocation API, then:

-	the service consumer may update/modify the revocation notification URI when updating/modifying an existing "Individual Location Information Subscription" resource;

-	at the reception of a request to update/modify an existing "Individual Location Information Subscription" resource, the EES shall:

-	if user consent is still granted for all the concerned UE(s), i.e., no user consent revocation notification is received by the EES from the UDM, then process the request as specified in clause 5.3.2.5.2; and

-	otherwise, if the EES is made aware by the UDM that user consent is not granted for a subset or all the UE(s), then:

-	stop processing the data related to the concerned UE(s) (i.e., for which user consent was revoked);

-	unsubscribe from user consent revocation notifications for the concerned UE(s) at the UDM;

-	if user consent is revoked for only a subset of the UE(s):

-	remove the concerned UE(s) from the concerned "Individual Location Information Subscription" resource and from the related subscriptions at the 3GPP 5GC network entities, if any; and

-	process the request as specified in clause 5.3.2.5.2 only for the subset of UE(s) for which user consent was not revoked;

and

-	if user consent is not anymore granted for all the UE(s):

-	reject the request and respond to the service consumer with an HTTP "403 Forbidden" status code with the response body containing a ProblemDetails data structure with the "cause" attribute including the "USER_CONSENT_NOT_GRANTED" application error;

-	send a user consent revocation notification to the service consumer by sending an HTTP POST request with the request body including the ConsentRevocNotif data structure that shall contain the user consent revocation information (e.g., UE(s) for which user consent was revoked, etc.); and

-	remove the UE(s) from the related subscriptions at the 3GPP 5GC network entities, if any;

and

-	the service consumer shall:

-	at the reception of the user consent revocation notification from the EES, take the necessary actions to stop processing the data related to the concerned UE(s); and

-	if user consent is revoked for all the UE(s), delete the corresponding "Individual Location Information Subscription" resource, as specified in clause 5.3.2.6.

#### 5.3.2.6	Eees_UELocation_Unsubscribe

##### 5.3.2.6.1	General

This service operation is used by the service consumer to unsubscribe from an existing UE(s) location information subscription.

##### 5.3.2.6.2	Service consumer unsubscribing to continuous UE(s) location reporting from EES using Eees_UELocation_Unsubscribe operation

To unsubscribe its location information subscription from the EES, the service consumer shall send HTTP DELETE message to the EES, on the resource URI identifying the "Individual Location Information Subscription" resource representation as specified in clause 8.2.2.3.3.4. Upon receiving the HTTP DELETE request, the EES shall:

1.	verify the identity of the service consumer and check if the service consumer is authorized to unsubscribe the Individual Location Information Subscription resource;

2.	if the service consumer is authorized to unsubscribe the Individual Location Information Subscription resource, then the EES shall unsubscribe the service consumer subscription identified by the subscriptionId from the EES and delete the resource representing Individual Location Information Subscription resource represented by subscriptionId;

3.	return the "204 Not Content" message to the service consumer, indicating the successful removal of the subscription information and may unsubscribe to the corresponding 3GPP core network service subscriptions.

On failure, the EES shall take proper error handling actions, as specified in clause 8.2.6, and respond to the service consumer with an appropriate error status code.

If the EES determines that the received HTTP DELETE message needs to be redirected, the EES may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative EES where the message should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

## 5.4	Eees_UEIdentifier Service

### 5.4.1	Service Description

The Eees_UEIdentifier API, as defined in 3GPP TS 23.558 [2], allows a service consumer to obtain an identifier uniquely identifying a UE from the EES.

### 5.4.2	Service Operations

#### 5.4.2.1	Introduction

The service operations defined for Eees_UEIdentifier API are shown in the table 5.4.2.1-1.

Table 5.4.2.1-1: Operations of the Eees_UEIdentifier API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_UEIdentifier_Get | This service operation is used by the service consumer to request UE identifier information. | e.g., EAS |

#### 5.4.2.2	Eees_UEIdentifier_Get

##### 5.4.2.2.1	General

This service operation is used by a service consumer to obtain a UE's identifier (UE ID, as specified in 3GPP TS 23.558 [2]) from the EES that uses 3GPP CN capability to retrieve the UE Identifier which is specific to the service consumer.

The following procedures using the "Eees_UEIdentifier_Get" service operation service operation are supported:

-	Service consumer obtaining UE Identifier Information using the "Get" custom operation.

##### 5.4.2.2.1A	Service consumer obtaining UE Identifier Information using the "Get" custom operation

In order to obtain a UE's Identifier from the EES, the service consumer shall send an HTTP POST request message to the EES targeting the "Get" custom operation URI (i.e., "{apiRoot}/eees-ueidentifier/<apiVersion>/get") and the request body including the UserInfo data structure as defined in clause 8.3.3.3.

Upon reception of the HTTP POST request message from the service consumer, the EES shall:

1.	verify the identity of the service consumer and check if the service consumer is authorized to obtain the UE's Identifier; and

2.	if the service consumer is authorized, the EES shall:

a.	invoke the Nnef_UEId service as specified in clause 4.4.32 of 3GPP TS 29.522 [10] to obtain the UE's Identifier based on the received user information, and may derive the coresponding DNN and/or S-NSSAI based on the verified identity of the service consumer; and

b.	upon successful retrieval of the UE's Identifier, respond to the service consumer with an HTTP "200 OK" status code along with the retrieved UE identifier information within the UeIdInfo data structure.

On failure or if the EES receives an error code from the NEF, the EES shall take proper error handling actions, as specified in clause 8.3.6, and respond to the service consumer with an appropriate error status code.

If the EES determines the received HTTP POST request needs to be redirected, the EES may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative EES where the request message should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

##### 5.4.2.2.2	EAS obtaining UE identifier from EES using Eees_UEIdentifier_Fetch custom operation

This procedure and the corresponding custom operation are deprecated. The procedure in clause 5.4.2.2.1A should be used instead.

To obtain an UE's Identifier from the EES, the EAS shall send a HTTP POST message to the EES with the request URI set to "{apiRoot}/eees-ueidentifier/<apiVersion>/fetch" and the request body with the UserInformation data structure including the information about the user or UE available with EAS, for which the UE identifier is requested.

Upon receiving the HTTP POST message from the EAS, the EES shall:

1.	Process the EAS UE Identifier request;

2.	verify the identity of the EAS and check if the EAS is authorized to obtain the UE Identifier;

3.	if the EAS is authorized to obtain the UE's Identifier information, then the EES shall;

a.	invoke the Nnef_UEId service as specified in clause 4.4.32 of 3GPP TS 29.522 [10] to obtain the UE identifier based on the user information received, and may derive the corresponding DNN and/or S-NSSAI based on the verified identity of the EAS;

b.	upon successful retrieval of UE identifier, respond with "200 OK" along with the retrieved UE identifier to the EAS. If the UE identifier cannot be successfully retrieved or an internal error or an error in the HTTP POST request, the EES shall send an HTTP error response as specified in clause 7.7.

If the EES determines the received HTTP POST request needs to be redirected, the EES may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative EES where the request message should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

## 5.5	Eees_AppClientInformation Service

### 5.5.1	Service Description

The Eees_AppClientInformation API, as defined in 3GPP TS 23.558 [2], allows a service consumer via Eees interface to obtain the information about the capabilities of the ACs based on certain filter criteria.

### 5.5.2	Service Operations

#### 5.5.2.1	Introduction

The service operations defined for Eees_AppClientInformation API are shown in the table 5.5.2.1-1.

Table 5.5.2.1-1: Operations of the Eees_AppClientInformation API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_AppClientInformation_Subscribe | This service operation is used by a service consumer to subscribe to the EES for AC information reporting. | e.g., EAS |
| Eees_AppClientInformation_Notify | This service operation is used by the EES to notify the service consumer about the AC information. | EES |
| Eees_AppClientInformation_UpdateSubscription | This service operation is used by the service consumer to update an existing subscription for AC information reporting at the EES. | e.g., EAS |
| Eees_AppClientInformation_Unsubscribe | This service operation is used by the service consumer to delete an existing subscription for AC information reporting at the EES. | e.g., EAS |

#### 5.5.2.2	Eees_AppClientInformation_Subscribe

##### 5.5.2.2.1	General

This service operation is used by the service consumer to subscribe for AC(s) information reporting.

##### 5.5.2.2.2	Service consumer subscribing to AC information reporting from the EES using the Eees_AppClientInformation_Subscribe operation

To subscribe to AC information reporting at the EES, the service consumer shall send an HTTP POST request to the EES targeting the "Application Client Information Subscriptions" collection resource, with the request body containing the ACInfoSubscription data structure, as specified in clause 8.4.2.2.3.1.

Upon reception of the HTTP POST request from the service consumer, the EES shall:

1.	verify the identity of the service consumer and check if the service consumer is authorized to subscribe for the AC information reporting; and

2.	if the service consumer is authorized, then the EES shall:

a.	create a new "Individual Application Client Information Subscription" resource with the AC Information Subscription as specified in clause 8.4.2.1; and

b.	respond to the service consumer with an HTTP "201 Created" status code including an HTTP Location header field containing the URI of the created "Individual Application Client Information Subscription" resource, and the response body containing a representation of the created resource within the ACInfoSubscription data structure.

On failure:

-	the EES shall take proper error handling actions, as specified in clause 8.4.6, and respond to the service consumer with an appropriate error status code; and

-	if the targeted service consumer (e.g., EAS, CAS) is not registered at the EES when at the reception of the subscription creation request, the EES shall respond with an HTTP "403 Forbidden" status code with the response body containing the ProblemDetails data structure with the "cause" attribute set to "REGISTRATION_REQUIRED" application error.

If the expiration time is provided, then to maintain the registration, the service consumer shall send a subscription update request (as described in clause 5.5.2.4) prior to the expiration time. If the subscription update request is not sent before the expiry time, then the EES shall treat the subscription as unsubscribed and delete the corresponding resource.

#### 5.5.2.3	Eees_AppClientInformation_Notify

##### 5.5.2.3.1	General

This service operation is used by the EES to send AC information reporting notifications to the service consumer.

##### 5.5.2.3.2	EES notifying the AC information to the service consumer using Eees_AppClientInformation_Notify operation

The EES determines to notify the service consumer with the AC information matching the filter criteria, when the AC information updates are available to the EES, e.g., EES receives a registration request from the EEC.

To notify the AC(s) information, the EES shall send an HTTP POST request message targeting the Notification Destination URI received in the corresponding subscription creation/update request, With the request body including the ACInfoNotification data structure.

Upon reception of the HTTP POST request message and successful processing, the service consumer shall return an HTTP "204 No Content" message to acknowledge the successful reception of the notification.

On failure, the service consumer shall take proper error handling actions, as specified in clause 8.4.6, and respond to the EES with an appropriate error status code.

If the service consumer determines that the received HTTP POST message needs to be redirected, the service consumer may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

#### 5.5.2.4	Eees_AppClientInformation_UpdateSubscription

##### 5.5.2.4.1	General

This service operation is used by the service consumer to update an existing AC information reporting subscription at the EES.

##### 5.5.2.4.2	Service consumer updating AC information reporting subscription at the EES using Eees_AppClientInformation_UpdateSubscribe operation

To update an existing AC information reporting subscription at the EES, the service consumer shall send a HTTP PATCH or PUT request message to the EES targeting the corresponding "Individual Application Client Information" resource, with the request body including either:

-	the requested modifications to the resource within the ACInfoSubscriptionPatch data structure as specified in clause 8.4.2.3.3.2, in case the HTTP PATCH method is used; or

-	the updated representation of the resource within the ACInfoSubscription data structure as specified in clause 8.4.2.3.3.3, in case the HTTP PUT method is used.

The HTTP PUT request shall not udpate the "easId" property of the existing resource.

Upon reception of the HTTP PATCH or PUT request message from the service consumer, the EES shall:

1.	verify the identity of the service consumer and check if the service consumer is authorized to updated the targeted AC information reporting subscription; and

2.	if the service consumer is authorized, then the EES shall:

a.	update the targeted "Individual Application Client Information" resource accordingly; and

b.	return either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Application Client Information" resource within the ACInfoSubscription data structure; or

-	an HTTP "204 No Content" status code.

On failure:

-	the EES shall take proper error handling actions, as specified in clause 8.4.6, and respond to the service consumer with an appropriate error status code; and

-	if the service consumer (e.g., EAS, CAS) is not registered at the EES when sending the AC information reporting subscription update request, the EES shall respond with an HTTP "403 Forbidden" status code with the response body containing the ProblemDetails data structure with the "cause" attribute set to "REGISTRATION_REQUIRED" application error.

If the EES determines that the received HTTP PATCH or PUT message needs to be redirected, the EES may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative EES where the message should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

If the expiration time is provided for the first time or updated, then to maintain the subscription, the service consumer shall send another subscription update request prior to the subscription expiry time. If the subscription update request is not sent before the expiry time, then the EES shall treat the subscription as unsubscribed and delete the corresponding resource.

#### 5.5.2.5	Eees_AppClientInformation_Unsubscribe

##### 5.5.2.5.1	General

This service operation is used by the service consumer to unsubscribe from an existing AC information reporting subscription.

##### 5.5.2.5.2	Service consumer unsubscribing to AC information reporting from the EES using Eees_AppClientInformation_Unsubscribe operation

To unsubscribe from AC information reporting at the EES, the service consumer shall send an HTTP DELETE request message to the EES targeting the corresponding "Individual Application Client Information Subscription" resource as specified in clause 8.4.2.3.3.4.

Upon reception of the HTTP DELETE request, the EES shall:

1.	verify the identity of the service consumer and check if the service consumer is authorized to unsubscribe from the targeted AC information reporting subscription; and

2.	if the service consumer is authorized, then the EES shall:

-	delete the targeted "Individual Application Client Information Subscription" resource; and

-	return an HTTP "204 Not Content" status code.

On failure:

-	the EES shall take proper error handling actions, as specified in clause 8.4.6, and respond to the service consumer with an appropriate error status code; and

-	if the service consumer is not registered at the EES when sending the AC information reporting subscription deletion request, the EES shall respond with an HTTP "403 Forbidden" status code with the response body containing the ProblemDetails data structure with the "cause" attribute set to "REGISTRATION_REQUIRED" application error.

If the EES determines that the received HTTP DELETE request needs to be redirected, the EES may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative EES where the message should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

## 5.6	Eees_SessionWithQoS Service

### 5.6.1	Service Description

The Eees_SessionWithQoS API, as defined in 3GPP TS 23.558 [2], allows a service consumerto support the setup and management of a data session between the AC and service consumer with a specific QoS.

### 5.6.2	Service Operations

#### 5.6.2.1	Introduction

The service operations defined for Eees_SessionWithQoS API are shown in the table 5.6.2.1-1.

Table 5.6.2.1-1: Operations of the Eees_SessionWithQoS API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_SessionWithQoS_Create | The service operation is used by the service consumer to request the creation of a session with QoS. | e.g., EAS |
| Eees_SessionWithQoS_Update | The service operation is used by the service consumer to request the update of an existing session with QoS. | e.g., EAS |
| Eees_SessionWithQoS_Revoke | The service operation is used by the service consumer to request the update of an existing session with QoS. | e.g., EAS |
| Eees_SessionWithQoS_Notify | The service operation is used by the EES to notify the service consumer about a user plane event associated with an existing established session with QoS. | EES |

#### 5.6.2.2	Eees_SessionWithQoS_Create

##### 5.6.2.2.1	General

This service operation is used by a service consumer to request the creation of a session with QoS.

##### 5.6.2.2.2	Service consumer requesting reservation of resources for a data session between AC and service consumer with specific QoS using Eees_SessionWithQoS operation

To request the establishment of a session with QoS, the service consumer shall send an HTTP POST request message to the EES targeting the "Sessions with QoS" collection resource as specified in clause 8.5.2.2.3.1, with the request body including the SessionWithQoS data structure.

Upon reception of the HTTP POST request message from the service consumer:

1.	the EES shall verify the identity of the service consumer and check if the service consumer is authorized to request teh creation of the session with QoS;

2.	if the service consumer is authorized, then the EES shall:

a.	if the request is for a group of UEs identified by group ID (i.e., via the "intGrpId" or "extGrpId" attribute) or for a single UE identified via the "ueId" attribute, interact with the SCEF (as specified in 3GPP TS 29.122 [6]) or the NEF (as specified in 3GPP TS 29.522 [10]) by invoking the MonitroingEvent API with the monitoring type set to "PDN_CONNECTIVITY_STATUS" to request to be notified when the 3GPP network detects that the UE's PDN connection or PDU session is set up or torn down;

b.	if the request is for a single UE identified by the IP address or the IP address is obtained in step 2a, interact directly with the PCRF (as specified in 3GPP TS 29.214 [15]) or the PCF (as specified in 3GPP TS 29.514 [16]), or via the SCEF (as specified in 3GPP TS 29.122 [6]) or the NEF (as specified in 3GPP TS 29.522 [10]) by invoking the AsSessionWithQoS API, to provide the specific QoS information to the PCF;

c.	if one of the subscribed event(s) is "UP_PATH_CHG", "ACR_MONITORING" and/or "ACR_FACILITATION" event:

a)	if the "EdgeApp_2" feature is supported, the service consumer may provide the traffic filter information in the "trafFilterInfo" attribute in the request body and:

i)	the EES may invoke the Nnef_PfdManagement API as described in clause 4.4.6 of 3GPP TS 29.522 [10] and clause 4.4.10 of 3GPP TS 29.122 [18] with the same Application Identifier that is used for requesting user plane path management events monitoring as defined further below;

ii)	if the Application Identifier is not provided by the service consumer, the EES may map the service consumer ID into the Application Identifier that is used to invoke the Nnef_PfdManagement API;

iii)	if the invocation of the PFD Management procedures towards the 3GPP network fails (e.g., the PFD Management service is not supported by the 3GPP Core Network), the EES shall reject the request with an appropriate error response; and

iv)	if the PFD Management service is not supported by the 3GPP Core Network, the EES shall reject the request with an HTTP "403 Forbidden" status code with the response body containing the ProblemDetails data structure including the "cause" attribute set to the "PFD_MNGT_NOT_SUPPORTED" application error;

and

3.	upon reception of a successful response from the 3GPP network and successful processing of the request,	create a new "Individual Session with QoS" resource and respond to the service consumer with an HTTP "201 Created" status code with the response body including a representation of the created resource within the SessionWithQoS data structure, and an HTTP Location header field containing the URI of the created resource.

On failure, the EES shall take proper error handling actions, as specified in clause 8.5.6, and respond to the service consumer with an appropriate error status code.

#### 5.6.2.3	Eees_SessionWithQoS_Update

##### 5.6.2.3.1	General

This service operation is used by a service consumer to request the update of an existing session with QoS.

##### 5.6.2.3.2	Service consumer updating QoS of a data session between AC and service consumer using Eees_SessionWithQoS_Update operation

To request the update of an existing session with QoS, the service consumer shall send an HTTP PATCH or PUT request message to the EES targeting the corresponding "Individual Session with QoS" resource, with the request body including either;

-	the requested modifications to the resource within the SessionWithQoSPatch data structure as specified in clause 8.5.2.3.3.1, in case the HTTP PATCH method is used; or

-	the updated representation of the resource within the SessionWithQoS data structure as specified in clause 8.5.2.3.3.2, in case the HTTP PUT method is used.

The HTTP PUT request shall not update the "easId", "ueId", "ueIpv4Addr", "ueIpv6Addr", "ipDomain", "intGrpId", "extGrpId", "dnn" and/or "snssai" attributes.

Upon reception of the HTTP PATCH or PUT message from the service consumer:

1.	the EES shall verify the identity of the service consumer and check if the service consumer is authorized to request the update of the targeted session with QoS; and

2.	if the service consumer is authorized, then:

a.	if one of the subscribed event(s) is "UP_PATH_CHG", "ACR_MONITORING" and/or "ACR_FACILITATION" event:

i.	if the "EdgeApp_2" feature is supported, the service consumer may provide the traffic filter information in the "trafFilterInfo" attribute in the request body and:

-	the EES may invoke the Nnef_PfdManagement API as described in clause 4.4.6 of 3GPP TS 29.522 [10] and clause 4.4.10 of 3GPP TS 29.122 [18] with the same Application Identifier that is used for requesting user plane path management event monitoring as defined further below;

-	if the Application Identifier is not provided by the service consumer, the EES may map the service consumer's ID into the Application Identifier that is used to invoke the Nnef_PfdManagement API;

-	if the invocation of the PFD Management procedures towards the 3GPP network fails (e.g., the PFD Management service is not supported by the 3GPP Core Network), the EES shall reject the request with an appropriate error response; and

-	if the PFD Management service is not supported by the 3GPP Core Network, the EES shall reject the request with an HTTP "403 Forbidden" status code with the response body containing the ProblemDetails data structure including the "cause" attribute set to the "PFD_MNGT_NOT_SUPPORTED" application error;

and

b.	upon success, the EES shall:

a.	interact with the 3GPP network to update the associated data session; and

b.	upon reception of a successful response from 3GPP network, respond to the service consumer with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Session with QoS" resource within the SessionWithQoS data structure; or

-	an HTTP "204 No Content" status code.

On failure, the EES shall take proper error handling actions, as specified in clause 8.5.6, and respond to the service consumer with an appropriate error status code.

If the EES determines that the received HTTP PATCH or PUT message needs to be redirected, the EES may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative EES where the message should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

#### 5.6.2.4	Eees_SessionWithQoS_Revoke

##### 5.6.2.4.1	General

This service operation is used by service consumer to request the deletion of an existing session with QoS.

##### 5.6.2.4.2	Service consumer revoking QoS of a data session between AC and service consumer using Eees_SessionWithQoS_Revoke operation

To request the deletion of an existing session with QoS, the service consumer shall send an HTTP DELETE request message to the EES targeting the corresponding "Individual Session with QoS" resource as specified in clause 8.5.2.3.3.3.

Upon reception of the HTTP DELETE request, the EES shall:

1.	verify the identity of the service consumer and check if the service consumer is authorized to request the deletion of the targeted session with QoS; and

2.	if the service consumer is authorized, then the EES shall:

a.	interact with the 3GPP core network to delete the associated data session;

b.	upon reception of a successful response from the 3GPP core network, delete the corresponding "Individual Session with QoS" resource; and

c.	return an HTTP "204 No Content" status code.

On failure, the EES shall take proper error handling actions, as specified in clause 8.5.6, and respond to the service consumer with an appropriate error status code.

If the EES determines that the received HTTP DELETE message needs to be redirected, the EES may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative EES where the message should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

#### 5.6.2.5	Eees_SessionWithQoS_Notify

##### 5.6.2.5.1	General

This service operation is used by EES to send user plane event notification information related to an existing session with QoS.

##### 5.6.2.5.2	EES notifying QoS of a data session between AC and service consumer using Eees_SessionWithQoS_Notify operation

The EES determines to notify the user plane event notification information to the service consumer when the EES receives the notification of the user plane event information from the 3GPP core network.

To notify the user plane event notification information, the EES shall send an HTTP POST request message targeting the Notification Destination URI received during the creation/update of the corresponding session with QoS, with the request body including the  UserPlaneEventNotification data structure.

Upon reception of the HTTP POST request message and successful processing of the request, the service consumer shall process the event report information and return an HTTP "204 No Content" status code to acknowledge the successful reception of the notification.

On failure, the service consumer shall take proper error handling actions, as specified in clause 8.5.6, and respond to the EES with an appropriate error status code.

If the service consumer determines that the received HTTP POST message needs to be redirected, the service consumer may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

## 5.7	Eees_EASDiscovery Service

### 5.7.1	Service Description

The Eees_EASDiscovery service API enables a service consumer to:

-	request EAS discovery.

### 5.7.2	Service Operations

#### 5.7.2.1	Introduction

The service operations defined for Eees_EASDiscovery API are shown in the table 5.7.2.1-1.

Table 5.7.2.1-1: Operations of the Eees_EASDiscovery API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_EASDiscovery_TEasDiscRequest | This service operation is used by the service consumer to request EAS discovery information. | EES, EAS, CAS |

#### 5.7.2.2	Eees_EASDiscovery_TEasDiscRequest

##### 5.7.2.2.1	General

This service operation is used by the service consumer to request for T-EAS discovery information, as specified in 3GPP TS 23.558 [2].

##### 5.7.2.2.2	Service consumer requesting T-EAS discovery information using Eees_EASDiscovery_TEasDiscRequest operation

To request for T-EAS discovery, the service consumer shall send an HTTP POST request to the EES as specified in clause 6.3.2.4.4.2 of 3GPP TS 24.558 [14]. The body of the POST message shall include the EasDiscoveryReq data structure as specified in clause 6.3.5.2.2 of 3GPP TS 24.558 [14].

Upon reception of the HTTP POST request from the service consumer, the EES shall:

a)	process the EAS discovery request information;

b)	the EES verifies and checks if the service consumer is authorized to discover the requested EAS(s) from the EES;

c)	if the service consumer is authorized to discover the requested EAS(s) from the EES, then upon successful processing of the request, the EES responds with an HTTP "200 OK" status code with the response body including the EasDiscoveryResp data structure as specified in clause 6.3.5.2.3 of 3GPP TS 24.558 [14] and in clause 8.8.3.2 of 3GPP TS 23.558 [2]. If the successful processing of the request does not result in finding a matching EAS (i.e. there is no client side error), the EES responds with an HTTP "204 No Content" status code.

If the EES determines that the received HTTP POST request needs to be redirected, the EES may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative EES where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

On failure, the EES shall take proper error handling actions, as specified in clause 6.3.6 of 3GPP TS 24.558 [14], and respond to the EAS or EES with an appropriate error status code.

## 5.8	Eees_ACRManagementEvent Service

### 5.8.1	Service Description

The Eees_ACRManagementEvent API, as defined in 3GPP TS 23.558 [2], allows a service consumer to support (un)subscription to the notifications of ACR management events.

### 5.8.2	Service Operations

#### 5.8.2.1	Introduction

The service operation defined for Eees_ACRManagementEvent API is shown in the table 5.8.2.1-1.

Table 5.8.2.1-1: Operations of the Eees_ACRManagementEvent API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_ACRManagementEvent_Subscribe | The service operation is used by the service consumer to subscribe to notifications of ACR management event(s). | e.g., EAS, CAS, EES |
| Eees_ACRManagementEvent_UpdateSubscription | The service operation is used by the service consumer to request a modification of an existing subscription of notifications of ACR management event(s). | e.g., EAS, CAS, EES |
| Eees_ACRManagementEvent_Unsubscribe | The service operation is used by the service consumer to unsubscribe to an existing subscription of notifications of ACR management event(s). | e.g., EAS, CAS, EES |
| Eees_ACRManagementEvent_Notify | The service operation is used by the EES to notify the service consumer about ACR management event(s) if detected. | EES |

#### 5.8.2.2	Eees_ACRManagementEvent_Subscribe

##### 5.8.2.2.1	General

This service operation is used by the service consumer to subscribe to notifications of ACR management events (see also clauses 8.6.3.2.2 and 8.18.2.4.6 of 3GPP TS 23.558 [2]).

##### 5.8.2.2.2	Service consumer requesting to get notifications of ACR management events using Eees_ACRManagementEvent_Subscribe service operation

In order to subscribe to notifications of ACR management events, the service consumer shall send a HTTP POST request message to the EES targeting the "ACR Management Events Subscriptions" resource as specified in clause 8.6.2.2.3.1. The body of the HTTP POST request message shall include the AcrMgntEventsSubscription data type as specified in clause 8.6.5.2.2.

Upon reception of the HTTP POST request message from the service consumer:

1.	the EES shall verify the identity of the service consumer and check if the service consumer is authorized to subscribe to notifications of ACR management events;

2a.	if the service consumer is authorized and upon successful processing of the request, the EES shall create a new "Individual ACR Management Events Subscription" resource and respond to the service consumer with an HTTP "201 Created" status code, with the response body including the representation of the created "Individual ACR Management Events Subscription" resource. The URI of the created resource shall also be included within an HTTP Location header field;

and

2b.	on failure:

ii)	the EES shall take proper error handling actions, as specified in clause 8.6.6, and respond to the service consumer with an appropriate error status code; and

iii)	if the "EdgeApp_2" feature is supported and the invocation of the PFD Management procedures towards the 3GPP network during the processing of the request as part of step 2a above fails because the PFD Management service is not supported by the 3GPP Core Network, the EES shall reject the request with an HTTP "403 Forbidden" status code with the response body containing the ProblemDetails data structure including the "cause" attribute set to the "PFD_MNGT_NOT_SUPPORTED" application error.

#### 5.8.2.3	Eees_ACRManagementEvent_UpdateSubscription

##### 5.8.2.3.1	General

This service operation is used by the service consumer to request updating an existing Individual ACR Management Events Subscription (see also clauses 8.6.3.2.4 and 8.18.2.4.6 of 3GPP TS 23.558 [2]).

##### 5.8.2.3.2	Service consumer updating an existing Individual ACR Management Events Subscription using Eees_ACRManagementEvent_UpdateSubscription service operation

To request the modification or update of an existing "Individual ACR Management Events Subscription" resource, the service consumer shall send an HTTP PATCH request (for partial modification) or PUT request (for update via full replacement) message to the EES targeting the URI of the corresponding "Individual ACR Management Events Subscription" resource, as specified in clause 8.6.2.3.3.1 (for an HTTP PATCH) message and in clause 8.6.2.3.3.2 (for an HTTP PUT message). This request shall not update the "easId" property of the existing resource.

Upon receiving the HTTP PATCH or PUT request message from the service consumer:

1.	the EES shall check whether the service consumer is authorized to update this existing "Individual ACR Management Events Subscription" resource;

2a.	if the service consumer is authorized and upon successful processing of the request, the EES shall update the targeted existing "Individual ACR Management Events Subscription" resource accordingly and respond to the service consumer with either an HTTP "204 No Content" status code, or an HTTP "200 OK" status code with the response body includingthe updated "Individual ACR Management Events" Subscription resource.

2b. on failure, the EES shall take proper error handling actions, as specified in clause 8.6.6, and respond to the service consumer with an appropriate error status code; and

2c. if the EES determines that the received HTTP PATCH or PUT message needs to be redirected, the EES may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative EES towards which the message should be redirected. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

#### 5.8.2.4	Eees_ACRManagementEvent_Unsubscribe

##### 5.8.2.4.1	General

This service operation is used by the service consumer to delete an existing ACR Management Events Subscription (see also clauses 8.6.3.2.5 and 8.18.2.4.6 of 3GPP TS 23.558 [2]).

##### 5.8.2.4.2	Service consumer deleting an existing Individual ACR Management Events Subscription using Eees_ACRManagementEvent_Unsubscribe service operation

To delete an existing "Individual ACR Management Events Subscription", the service consumer shall send a HTTP DELETE request message to the EES targeting the concerned "Individual ACR Management Events Subscription" resource as specified in clause 8.6.2.3.3.3. Upon reception of the HTTP DELETE request:

1.	the EES shall verify the identity of the service consumer and check if the service consumer is authorized to delete the targeted "Individual ACR Management Events Subscription" resource;

2a.	if the service consumer is authorized and upon successful processing of the request, the EES shall delete the "Individual ACR Management Events Subscription"; and respond with an HTTP "204 No Content" status code to the service consumer, indicating the successful deletion of this "Individual ACR Management Events Subscription" resource;

2b.	on failure, the EES shall take proper error handling actions, as specified in clause 8.6.6, and respond to the service consumer with an appropriate error status code; and

2c.	if the EES determines that the received HTTP DELETE request message needs to be redirected, the EES may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative EES towards which the message should be redirected. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

#### 5.8.2.5	Eees_ACRManagementEvent_Notify

##### 5.8.2.5.1	General

This service operation is used by EES to send ACR management event notification information or the availability of user path management events monitoring via the 3GPP 5GC network to the service consumer (see also clauses 8.6.3.2.3 and 8.18.2.4.6 of 3GPP TS 23.558 [2]).

##### 5.8.2.5.2	EES notifying ACR management events using Eees_ACRManagementEvent_Notify operation

In order to notify on ACR management event(s), the EES shall send an HTTP POST request message targeting the Notification URI received during the creation or update/modification of the corresponding subscription as defined in clauses 5.8.2.2.2 and 5.8.2.2.3. The body of HTTP POST message shall include the AcrMgntEventsNotification data structure as specified in clause 8.6.5.2.4.

Upon reception of the HTTP POST message, the service consumer shall process the event report information, as specified in clause 8.6.3.2.3 of 3GPP TS 23.558 [2], and respond to the EES with either:

1.	an HTTP "204 No Content" message; or

2.	an HTTP "200 OK" status code,with the response body including EasAckInformation data type as specified in clause 8.6.5.2.15.

On failure, the service consumer shall take proper error handling actions, as specified in clause 8.6.6, and respond to the EES with an appropriate error status code.

If the service consumer determines that the received HTTP POST request needs to be redirected, the service consumer may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative service consumer where the notification should be redirected. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

##### 5.8.2.5.3	EES notifying the availability of user path management events monitoring via the 3GPP 5GC network using Eees_ACRManagementEvent_Notify operation

In order to notify of the availability of user path management events monitoring via the 3GPP 5GC network, the EES shall send an HTTP POST request message targeting the notification URI "{notificationDestination}/report-availability", where the Notification Destination URI is the one received during the creation or modification of the corresponding ACR management event subscription as defined in clauses 5.8.2.2.2 and 5.8.2.2.3. The body of HTTP POST request message shall include the AvailabilityNotif data type as specified in clause 8.6.5.2.11.

NOTE:	The service consumer is subscribed by default to the reception of notifications on the availability of user path management events monitoring via the 3GPP 5GC network.

Upon reception of the HTTP POST request message, the service consumer shall process the received information and respond to the EES with an HTTP "204 No Content" status code.

On failure, the service consumer shall take proper error handling actions, as specified in clause 8.6.6, and respond to the EES with an appropriate error status code.

If the service consumer determines that the received HTTP POST request needs to be redirected, the service consumer may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

## 5.9	Eees_AppContextRelocation Service

### 5.9.1	Service Description

The Eees_AppContextRelocation API, allows the service consumer  to declare to the S-EES about the T-EAS selected by the service consumer, or allows the service consumer to request ACR determination to the S-EES.

### 5.9.2	Service Operations

#### 5.9.2.1	Introduction

The service operation defined for Eees_AppContextRelocation API is shown in the table 5.9.2.1-1.

Table 5.9.2.1-1: Operations of the Eees_AppContextRelocation API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_AppContextRelocation_SelectedTargetEAS_Declare | This service operation is used by the service consumer to inform the S-EES about selection of the T-EAS. | EAS, CAS, EES |
| Eees_AppContextRelocation_ACRDetermination_Request | This service operation is used by the service consumer to request the S-EES to determine the ACR. | EAS, EES |

#### 5.9.2.2	Eees_AppContextRelocation_SelectedTargetEAS_Declare

##### 5.9.2.2.1	General

This service operation is used by the service consumer to declare the selected T-EAS information to the S-EES, as specified in 3GPP TS 23.558 [2].

##### 5.9.2.2.2	S-EAS informing the S-EES about the selected T-EAS using Eees_AppContextRelocation_SelectedTargetEAS_Declare operation

To declare the selected T-EAS information to the S-EES, the service consumer shall send an HTTP POST request message to the EES targeting the URI "{apiRoot}/eees-appctxtreloc/v1/declare" as specified in clause 6.5.3.4 of 3GPP TS 24.558 [14]. The request body of the POST request message (i.e. "Declare" custom operation) includes the AcrDecReq data structure as specified in clause 6.5.5.2.4 of 3GPP TS 24.558 [14].

Upon receiving the HTTP POST message from the service consumer, the EES shall:

1.	Process the service consumer request;

2.	verify the identity of the service consumer and check if the service consumer is authorized to declare the T-EAS information;

3.	if the service consumer is authorized to declare T-EAS information, then the EES shall send successful declaration response message to service consumer.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

If the EES determines that the received HTTP POST message needs to be redirected, the EES may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative EES where the message should be redirected. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

#### 5.9.2.3	Eees_AppContextRelocation_ACRDetermination_Request

##### 5.9.2.3.1	General

This service operation is used by service consumer to request the S-EES to determine the ACR, as specified in the Eees_AppContextRelocation_Request operation with ACR determination part in 3GPP TS 23.558 [2].

##### 5.9.2.3.2	Request the S-EES to determine the ACR using Eees_AppContextRelocation_ACRDetermination_Request operation

To request the S-EES to determine the ACR, the service consumer shall send a HTTP POST message to the S-EES on the custom URI "{apiRoot}/eees-appctxtreloc/v1/determine" as specified in clause 6.5.3.2 of 3GPP TS 24.558 [14]. The request body of the POST message includes the AcrDetermReq data structure with information about the S-EAS end point information and may include other optional information on UE and/or EASID as specified in clause 6.5.5.2.2 of 3GPP TS 24.558 [14].

Upon receiving the HTTP POST message from the service consumer, the EES shall:

1. process the request;

2. verify the identity of the service consumer and check if it is authorized to request ACR determination;

3. if the service consumer is authorized to request the ACR , then the EES shall;

a.	send successful ACR determination response to S-EAS with an HTTP "204 No Content" status code;

b.	additionally, the S-EES determines the T-EES via the Discover T-EAS procedure and may notify the EEC with target information and/or ACR result notification as specified in 3GPP TS 24.558 [14];

c.	if the ACR determination handling met with failure case , the S-EES shall send a proper HTTP error status code and may include additional error information in the POST response body.

If the EES determines that the received HTTP POST message needs to be redirected, the EES may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative EES where the message should be redirected. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

## 5.10	Eees_EECContextRelocation Service

### 5.10.1	Service Description

The Eees_EECContextRelocation API enables a service consumer (e.g., S-EES, T-EES, CES) to perform the EEC context relocation, as defined in clause 8.9 of 3GPP TS 23.558 [2]. The Eees_EECContextRelocation_Pull service operation enables a service consumer (e.g., T-EES) to pull the EEC context from the EES (e.g., S-EES), as specified by Eees_EECContextPull API in 3GPP TS 23.558 [2]. The Eees_EECContextRelocation_Push service operation enables a service consumer (e.g., S-EES, CES) to push the EEC context to the EES (e.g., T-EES), as specified by Eees_EECContextPush API in 3GPP TS 23.558 [2].

### 5.10.2	Service Operations

#### 5.10.2.1	Introduction

The service operation defined for Eees_EECContextRelocation API is shown in the table 5.10.2.1-1.

Table 5.10.2.1-1: Operations of the Eees_EECContextRelocation API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_EECContextRelocation_Pull | This service operation is used by a service consumer to pull the EEC context information from the EES. | EES |
| Eees_EECContextRelocation_Push | This service operation is used by a service consumer to push the EEC context information to EES. | EES, CES |

#### 5.10.2.2	Eees_EECContextRelocation_Pull

##### 5.10.2.2.1	General

This service operation is used by a service consumer to pull the EEC context information from the EES (see also clause 8.9 of 3GPP TS 23.558 [2]).

##### 5.10.2.2.2	Service consumer pulling the EEC context information from the EES using the Eees_EECContextRelocation_Pull operation

In order to pull the EEC context information from the EES, the service consumer shall send an HTTP GET request message to the EES targeting the URI of the "EEC Contexts" collection resource and including the relevant query parameters, as specified in the clause 8.7.2.2.3.1.

Upon reception of the HTTP GET message from the service consumer, the EES shall:

1.	validate the request and verify whether the service consumer is authorized to relocate the EEC context;

2.	if the service consumer is authorised, process the request;

3.	upon success, respond with an HTTP "200 OK" status code, with the response body including the EECContext data type containing the EEC context information corresponding to the received query parameters; and

4.	on failure, the EES shall take proper error handling actions, as specified in clause 8.7.6, and respond to the service consumer with an appropriate error status code.

#### 5.10.2.3	Eees_EECContextRelocation_Push

##### 5.10.2.3.1	General

This service operation is used by a service consumer to push the EEC context information to the EES, (see also clause 8.9 of 3GPP TS 23.558 [2]).

##### 5.10.2.3.2	Service consumer pushing the EEC context information to the EES using the Eees_EECContextRelocation_Push operation

In order to push the EEC context information to the EES, the service consumer shall send an HTTP POST request message to the EES, targeting the URI of the "EEC Contexts" collection resource, as specified in the clause 8.7.2.2.3.2, with the request body including the EECContextPush data structure defined in clause 8.7.5.2.4.

Upon reception of the HTTP POST request message from the service consumer, the EES shall:

1.	validate the request and verify whether the service consumer is authorized to relocate the EEC context;

2.	if the service consumer is authorised, then process the request;

3.	upon success, the EES shall store the received EEC Context and respond with either:

-	an HTTP "200 OK" status code with the response body including the EECContextPushRes data structure; or

-	an HTTP "204 No content" status code;

and

4.	on failure, the EES shall take proper error handling actions, as specified in clause 8.7.6, and respond to the service consumer with an appropriate error status code.

## 5.11	Eees_EELManagedACR Service

### 5.11.1	Service Description

The Eees_EELManagedACR service exposed by the EES enables a service consumer to:

-	request the EES to handle all the operations of an ACR;

-	subscribe to ACT status reporting during an EEL Managed ACR; and

-	receive notifications from the EES on ACT status during an EEL Managed ACR.

### 5.11.2	Service Operations

#### 5.11.2.1	Introduction

The service operations defined for Eees_EELManagedACR API are shown in the table 5.11.2.1-1.

Table 5.11.2.1-1: Eees_EELManagedACR Service Operations

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_EELManagedACR_Request | This service operation enables a service consumer to request the EES to handle all the operations of an ACR. | e.g., EAS, EES |
| Eees_EELManagedACR_Subscribe | This service operation enables a service consumer to subscribe to ACT status reporting during an EEL Managed ACR. | e.g., EAS, EES |
| Eees_EELManagedACR_Notify | This service operation enables a service consumer to receive notifications from the EES on ACT status during an EEL Managed ACR. | e.g., EES, EES |

#### 5.11.2.2	Eees_EELManagedACR_Request

##### 5.11.2.2.1	General

This service operation is used by a service consumer to request the EES to handle all the operations of an ACR.

The following procedures are supported by the "Eees_EELManagedACR_Request" service operation:

-	EEL Managed ACR Request procedure.

##### 5.11.2.2.2	EEL Managed ACR Request

The EEL Managed ACR Request procedure enables a service consumer to request the EES to handle all the operations of an ACR (see also clauses 8.8.3.6 and clause 8.18.2.4.7 of 3GPP°TS°23.558°[2]).

1.	The service consumer shall send for this purpose an HTTP POST request (custom operation: "Request") to the EES, with the request URI set to "{apiRoot}/eees-eel-acr/<apiVersion>/request-eelacr" and the request body including the EELACRReq data structure as defined in clause 8.8.6.2.2.

2a.	Upon success, the EES shall respond with an HTTP "200 OK" status code with the response body including the feedback from the EES within the EELACRResp data structure as defined in clause 8.8.6.2.3.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body.

#### 5.11.2.3	Eees_EELManagedACR_Subscribe

##### 5.11.2.3.1	General

This service operation is used by a service consumer to subscribe to ACT status information reporting at the EES.

The following procedures are supported by the "Eees_EELManagedACR_Subscribe" service operation:

-	Subscribe to ACT status information reporting.

##### 5.11.2.3.2	Subscribe to ACT status information reporting

This procedure enables a service consumer to request the creation of a subscription to ACT status information reporting (see also clauses 8.8.3.6 and clause 8.18.2.4.7 of 3GPP°TS°23.558°[2]).

1.	In order to subscribe to ACT status reporting, the service consumer shall send an HTTP POST request to the EES, with the request URI set to "{apiRoot}/eees-eel-acr/<apiVersion>/subscriptions" and the request body including the ACTStatusSubsc data structure as defined in clause 8.8.6.2.4.

2a.	Upon success, the EES shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created Individual ACT Status Subscription resource within the ACTStatusSubsc data structure as defined in clause 8.8.6.2.4.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body.

#### 5.11.2.4	Eees_EELManagedACR_Notify

##### 5.11.2.4.1	General

This service operation is used by an EES to notify a previously subscribed service consumer on ACT status information.

The following procedures are supported by the "Eees_EELManagedACR_Notify" service operation:

-	ACT Status Notification.

##### 5.11.2.4.2	ACT Status Notification

The ACT Status Notification procedure enables an EES to notify a previously subscribed service consumer on ACT status information (see also clauses 8.8.3.6 and clause 8.18.2.4.7 of 3GPP°TS°23.558°[2]).

1.	The EES shall send for this purpose an HTTP POST request to the service consumer with the request URI set to "{notificationUri}/act-status", where the "notificationUri" is set to the value received from the service consumer during the ACT status reporting subscription procedure defined in clause 5.11.2.3, and the request body including the ACTStatusNotif data structure as defined in clause 8.8.6.2.5.

2a.	Upon success, the service consumer shall respond with an HTTP "204 No Content" status code to acknowledge the reception of the notification to the EES.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body.

If the service consumer is not able to handle the notification request or determines that the received HTTP POST request needs to be redirected, the service consumer may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

## 5.12	Eees_ACRStatusUpdate Service

### 5.12.1	Service Description

The Eees_ACRStatusUpdate service exposed by the EES enables a service consumer to:

-	update the information related to ACR (e.g., indicate the status of ACT, update the notification target address); and

-	receive from the EES the status of EDGE-3 subscription relocation, including subscription ID update for EDGE-3 subscriptions.

### 5.12.2	Service Operations

#### 5.12.2.1	Introduction

The service operations defined for Eees_ACRStatusUpdate API are shown in the table 5.12.2.1-1.

Table 5.12.2.1-1: Eees_ACRStatusUpdate Service Operations

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_ACRStatusUpdate_Request | This service operation enables a service consumer to update the information related to ACR (e.g., indicate the status of ACT, update the notification target address) at the EES. | e.g., EAS, CAS, EES |

#### 5.12.2.2	Eees_ACRStatusUpdate_Request

##### 5.12.2.2.1	General

This service operation is used by a service consumer to update the information related to ACR (e.g., indicate the status of ACT, update the notification target address) at the EES.

The following procedures are supported by the "Eees_ACRStatusUpdate_Request" service operation:

-	ACR Status Update Request procedure.

##### 5.12.2.2.2	ACR Status Update Request

The ACR Status Update Request procedure enables a service consumer to request to update the EES to request to update the information related to ACR (see also clause 8.8.3.8 of 3GPP°TS°23.558°[2]).

1.	The service consumer shall send for this purpose an HTTP POST request (custom operation: "Request") to the EES, with the request URI set to "{apiRoot}/eees-acrstatus-update/<apiVersion>/request-acrupdate" and the request body including the ACRUpdateData data structure defined in clause 8.9.6.2.2.

2.	Upon receiving the HTTP POST request message from the service consumer, the EES shall check whether the service consumer is autorized to update the ACR status. Then:

2a.	Upon success, the EES shall respond with an HTTP "200 OK" status code with the response body including the feedback from the EES within the ACRDataStatus data structure defined in clause 8.9.6.2.3, or an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body.

If the EES determines that the received HTTP POST request needs to be redirected, the EES may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative EES where the message should be redirected. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

## 5.13	Eees_ACRParameterInformation Service

### 5.13.1	Service Description

The Eees_ACRParameterInformation service exposed by the EES enables a service consumer (e.g., S-EES, CES) to:

-	send ACR parameters information to the EES (e.g., T-EES).

### 5.13.2	Service Operations

#### 5.13.2.1	Introduction

The service operations defined for the Eees_ACRParameterInformation API are shown in the table 5.13.2.1-1.

Table 5.13.2.1-1: Eees_ACRParameterInformation Service Operations

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_ACRParameterInformation_Request | This service operation enables a service consumer to send ACR parameters information to the EES. | e.g., EES, CES |

#### 5.13.2.2	Eees_ACRParameterInformation_Request

##### 5.13.2.2.1	General

This service operation is used by a service consumer to send ACR parameters information to the EES.

The following procedures are supported by the "Eees_ACRParameterInformation_Request" service operation:

-	ACR Parameters Information Request.

##### 5.13.2.2.2	ACR Parameters Information Request

The ACR Parameters Information Request procedure enables a service consumer to send ACR parameters information to the EES (see also clause 8.8.3.9 of 3GPP°TS°23.558°[2]).

1.	The service consumer shall send for this purpose an HTTP POST request to the EES targeting the URI of the corresponding custom operation (i.e., "Request"), with the request body including the ACRParamsInfo data structure defined in clause 8.10.6.2.2.

2.	Upon reception of the HTTP POST request message from the service consumer, the EES shall check whether the service consumer is autorized to send the ACR parameters information. Then:

2a.	upon success, the EES shall respond with an HTTP "204 No Content" status code;

2b.	on failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information, as defined in clause 8.10.7, should be returned in the HTTP POST response body; and

2c.	if the EES is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative target URI of the custom operation located in an alternative EES, as defined in clause 5.2.10 of 3GPP TS 29.122 [2].

## 5.14	Eees_CommonEASAnnouncement Service

### 5.14.1	Service Description

The Eees_CommonEASAnnouncement service exposed by the EES enables a service consumer (e.g., announcing EES) to:

-	send common EAS information to the EES (e.g., receiving EES).

### 5.14.2	Service Operations

#### 5.14.2.1	Introduction

The service operations defined for the Eees_CommonEASAnnouncement API are shown in the table 5.14.2.1-1.

Table 5.14.2.1-1: Eees_CommonEASAnnouncement Service Operations

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_CommonEASAnnouncement_Declare | This service operation enables a service consumer to send common EAS information to the EES. | e.g., EES |

#### 5.14.2.2	Eees_CommonEASAnnouncement_Declare

##### 5.14.2.2.1	General

This service operation is used by a service consumer to send common EAS information to the EES.

The following procedures are supported by the "Eees_CommonEASAnnouncement_Declare" service operation:

-	Common EAS Information Declaration.

##### 5.14.2.2.2	Common EAS Information Declaration

This procedure enables a service consumer to send common EAS information to the EES (see also clause 8.19.2 of 3GPP°TS°23.558°[2]).

1.	In order to declare Common EAS Information, the service consumer shall send an HTTP POST request to the EES targeting the URI of the corresponding custom operation (i.e., "Declare"), with the request body including the CommonEASInfo data structure defined in clause 8.11.6.2.2.

2.	Upon reception of the HTTP POST request message from the service consumer, the EES shall check whether the service consumer is authorized to declare common EAS information. Then:

2a.	upon success, the EES shall respond with either:

-	an HTTP "204 No Content" status code indicating that the common EAS information is successfully received and processed; or

-	an HTTP "200 OK" status code indicating that the common EAS information is successfully received and processed, and the response body containing common EAS declaration related information within the CommonEASInfoDecResp data structure;

2b.	on failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information, as defined in clause 8.11.7, should be returned in the HTTP POST response body; and

2c.	If the EES is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative target URI of the custom operation located in an alternative EES, as defined in clause 5.2.10 of 3GPP TS 29.122 [2].

## 5.15	Eees_TrafficInfluenceEAS Service

### 5.15.1	Service Description

The Eees_TrafficInfluenceEAS service exposed by the EES enables a service consumer to:

-	request to initiate/update/cancel/retrieve EAS application traffic influence.

### 5.15.2	Service Operations

#### 5.15.2.1	Introduction

The service operations defined for the Eees_TrafficInfluenceEAS API are shown in the table 5.15.2.1-1.

Table 5.15.2.1-1: Eees_TrafficInfluenceEAS Service Operations

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_TrafficInfluenceEAS_Manage | This service operation enables a service consumer to request to initiate/update/cancel/retrieve EAS application traffic influence. | e.g., EAS |

#### 5.15.2.2	Eees_TrafficInfluenceEAS_Manage

##### 5.15.2.2.1	General

This service operation enables a service consumer to request the EES to initiate/update/cancel/retrieve EAS application traffic influence.

The following procedures are supported by the "Eees_TrafficInfluenceEAS_Manage" service operation:

-	Application Traffic Influence Initiation.

-	Application Traffic Influence Update.

-	Application Traffic Influence Cancellation.

-	Application Traffic Influence Retrieval.

##### 5.15.2.2.2	Application Traffic Influence Initiation

This procedure enables a service consumer to request the EES to initiate EAS application traffic influence (see also clause 8.6.7 of 3GPP°TS°23.558°[2]).

1.	The service consumer shall send for this purpose an HTTP POST request to the EES targeting the URI of the "Application Traffic Influence Instances" collection resource, with the request body including the AppTrafficInfluence data structure defined in clause 8.12.5.2.2.

2.	Upon reception of the HTTP POST request message from the service consumer, the EES shall check whether the service consumer is authorized to request application traffic influence. Then:

2a.	if the service consumer is authorized:

-	the EES shall trigger the relevant traffic influence procedures towards the 3GPP core network as specified in clause 8.6.7.2 of 3GPP°TS°23.558°[2]; and

-	upon success, the EES shall respond to the service consumer with an HTTP "201 Created" status code with the response body including a representation of the created "Individual Application Traffic Influence Instance" resource, and an HTTP Location header field containing the URI of the created resource.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information, as defined in clause 8.12.6, should be returned in the HTTP POST response body.

##### 5.15.2.2.3	Application Traffic Influence Update

This procedure enables a service consumer to request the EES to update an existing EAS application traffic influence.

1.	The service consumer shall send for this purpose an HTTP PUT/PATCH request to the EES targeting the URI of the "Individual Application Traffic Influence Instance" resource with the request body including either:

-	the updated representation of the resource within the AppTrafficInfluence data structure as specified in clause 8.12.3.3.3.2, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the AppTrafficInfluencePatch data structure as specified in clause 8.12.3.3.3.3, in case the HTTP PATCH method is used.

2.	Upon reception of the HTTP PUT/PATCH request message from the service consumer, the EES shall check whether the update of the existing "Individual Application Traffic Influence Instance" resource is authorized or not:

2a.	if the update is authorized:

-	the EES shall trigger the relevant traffic influence procedures towards the 3GPP core network; and

-	upon success, the EES shall respond to the service consumer with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Application Traffic Influence Instance" resource within the AppTrafficInfluence data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information, as defined in clause 8.12.6, should be returned in the HTTP PUT/PATCH response body.

##### 5.15.2.2.4	Application Traffic Influence Cancellation

This procedure enables a service consumer to request the EES to cancel EAS application traffic influence.

1.	The service consumer shall send for this purpose an HTTP DELETE request to the EES targeting the URI of the "Individual Application Traffic Influence Instance" resource as specified in clause 8.12.3.3.3.4.

2.	Upon reception of the HTTP DELETE request message from the service consumer, the EES shall check whether the deletion of the existing "Individual Application Traffic Influence" resource is authorized or not:

2a.	if the deletion is authorized:

-	the EES shall trigger the relevant traffic influence procedures towards the 3GPP core network; and

-	upon success, the EES shall respond to the service consumer with with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information, as defined in clause 8.12.6, should be returned in the HTTP DELETE response body.

##### 5.15.2.2.5	Application Traffic Influence Retrieval

This procedure enables a service consumer to the EES to retrieve an existing EAS application traffic influence.

1.	The service consumer shall send for this purpose an HTTP GET request to the EES targeting the URI of the "Individual Application Traffic Influence Instance" resource as specified in clause 8.12.3.3.3.1.

2.	Upon reception of the HTTP GET request message from the service consumer, the EES shall check whether the retrieval of the existing Individual Application Traffic Influence Instance is authorized or not:

2a.	if the retrieval is authorized:

-	upon success, the EES shall respond to the service consumer with an HTTP "200 OK" status code with the response body including the representation of the requested "Individual Application Traffic Influence Instance" resource.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information, as defined in clause 8.12.6, should be returned in the HTTP GET response body.

## 5.16	Eees_EASInformationProvisioning Service

### 5.16.1	Service Description

The Eees_EASInformationProvisioning API enables a service consumer (e.g., EES) to:

-	declare the received EAS Information provisioning information to the EES.

### 5.16.2	Service Operations

#### 5.16.2.1	Introduction

The service operations defined for Eees_EASInformationProvisioning API are shown in the table 5.16.2.1-1.

Table 5.16.2.1-1: Operations of the Eees_EASInformationProvisioning API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_EASInformationProvisioning_Declare | This service operation is used by the service consumer to declare the received EAS Information provisioning information. | e.g., EES |

#### 5.16.2.2	Eees_EASInformationProvisioning_Declare

##### 5.16.2.2.1	General

This service operation is used by the service consumer to declare the received EAS Information provisioning information to the EES, as specified in clause 8.18.2.4.5 of 3GPP TS 23.558 [2].

##### 5.16.2.2.2	EAS Information Provisioning Information Declaration

To declare the received EAS Information provisioning information, the service consumer shall send an HTTP POST request message to the EES targeting the URI of the "Declare" custom operation, with the request body including the EASInfoProvReq data structure as defined in clause 6.6.3.2.2 of 3GPP TS 24.558 [14].

Upon receiving the HTTP POST message from the service consumer, the EES shall:

1.	verify the identity of the service consumer and check if the service consumer is authorized to perform this operation; and

2.	if the service consumer is authorized, then the EES shall process the request and respond with either:

-	an HTTP "200 OK" status code with the response body including EAS information provisioning related information within the EASInfoProvResp data structure; or

-	an HTTP "204 No Content" status code.

If the EES determines that the received HTTP POST message needs to be redirected, the EES may respond instead with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative EES towards which the request message should be redirected. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body, as specified in clause 6.6.6 of 3GPP TS 24.558 [14].

# 6	Services offered by Edge Configuration Server

## 6.1	Introduction

The table 6.1-1 lists the Edge Configuration Server APIs below the service name. A service description clause for each API gives a general description of the related API.

Table 6.1-1: List of ECS Service APIs

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eecs_EESRegistration | Request | Request/Response | e.g., EES |
|  | Update | Request/Response | e.g., EES |
|  | Deregister | Request/Response | e.g., EES |
| Eecs_TargetEESDiscovery (NOTE) | Request | Request/Response | e.g., EES, CES |
| Eecs_EASInfoManagement | Get | Request/Response | e.g., EES |
|  | Store | Request/Response | e.g., EES |
|  | Update | Request/Response | e.g., EES |
|  | Remove | Request/Response | e.g., EES |
| Eecs_ECSServiceProvisioning | Request | Request/Response | e.g., ECS |
|  | Subscribe | Subscribe/Notify |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |
|  | Notify |  |  |
| Eecs_ECSDiscovery | Request | Request/Response | ECS |
| Eecs_ACREvents | Subscribe | Subscribe/Notify | e.g., EES |
|  | Update |  |  |
|  | Unsubscribe |  |  |
|  | Notify |  |  |
| NOTE: In this release of the specification, this API is extended to enable to discover the target Enabler Server, which can be either the target EES or the target CES, not only the target EES, in order to support cloud enabler services. |  |  |  |

Table 6.1-2 summarizes the corresponding Edge Configuration Server APIs defined in this specification.

Table 6.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Eecs_EESRegistration | 9.1 | ECS EES registration service. | TS29558_Eecs_EESRegistration.yaml | eecs-eesregistration | A.11 |
| Eecs_TargetEESDiscovery | 9.2 | ECS Service to discover the target EES information. | TS29558_Eecs_TargetEESDiscovery.yaml | eecs-targeteesdiscovery | A.12 |
| Eecs_EASInfoManagement | 9.3 | ECS EAS Information Management Service. | TS29558_Eecs_EASInfoManagement.yaml | eecs-eim | A.16 |
| Eecs_ECSServiceProvisioning | 9.4 | ECS Service Provisioning Service | TS29558_Eecs_ECSServiceProvisioning.yaml | eecs-esp | A.18 |
| Eecs_ECSDiscovery | 9.5 | Eecs_ECSDiscovery | TS29558_Eecs_ECSDiscovery.yaml | eecs-ecsdiscovery | A.19 |
| Eecs_ACREvents | 9.6 | Eecs_ACREvents | TS29558_Eecs_ACREvents.yaml | eecs-ae | A.20 |

NOTE:	In this release of the specification, the ECS registration service is not specified and the profiles of the ECSs of a partner ECSP are assumed to be available at an ECS of this partner ECSP. The Eecs_ECSDiscovery service API is defined to enable the discovery of partner ECS information at the partner ECSP.

## 6.2	Eecs_EESRegistration Service

### 6.2.1	Service Description

The Eecs_EESRegistration API, as defined in 3GPP TS 23.558 [2], allows an Edge Enabler Server via Eecs interface to register, update its registration and deregister at a given Edge Configuration Server.

### 6.2.2	Service Operations

#### 6.2.2.1	Introduction

The service operation defined for Eecs_EESRegistration API is shown in the table 6.2.2.1-1.

Table 6.2.2.1-1: Operations of the Eecs_EESRegistration API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eecs_EESRegistraiton_Request | This service operation is used by the EES to register itself to a given ECS. | EES |
| Eecs_EESRegistraiton_Update | This service operation is used by the EES to update its registration information at ECS. | EES |
| Eecs_EESRegistraiton_Deregister | This service operation is used by the EES to deregister itself from a given ECS. | EES |

#### 6.2.2.2	Eecs_EESRegistration_Request

##### 6.2.2.2.1	General

This service operation is used by EES to register itself to a given ECS.

##### 6.2.2.2.2	EES registering to ECS using Eecs_EESRegistration_Request operation

To register itself as an Edge Enabler Server at the ECS, the EES shall send an HTTP POST message to the Edge Configuration Server on the "EES Registrations" collection resource. The body of the HTTP POST message shall include the EES profile information, may include proposed expiration time for the registration, as specified in clause 9.1.2.2.3.1.

Upon receiving the HTTP POST message from the EES, the ECS shall:

1.	Process the EES registration request information;

2.	verify the identity of the Edge Enabler Server and check if the EES is authorized to register itself at ECS ;

3.	if the EES is authorized to register to ECS, then the ECS shall;

a.	store the EES profile and create a new resource with the EES registration information as specified in clause 9.1.2.1;

b.	return the EES registration information, the resource URI of the EES registration information, in the "201 Created" response message. It shall include a Location HTTP header field. The Location header field shall contain the URI of the created registration i.e. {apiRoot}/eecs-eesregistration/<apiVersion>/registrations/{registrationId}. The response message may include expiration time to indicate when the EES registration will automatically expire.

On failure, the ECS shall take proper error handling actions, as specified in clause 9.1.6, and respond to the EES with an appropriate error status code.

If the expiration time is provided, then to maintain the registration, the EES shall send a registration update request (as described in clause 6.2.2.3) prior to the expiration time. If the registration update request is not sent before the expiry time, then the ECS shall treat the EES as deregistered and remove the corresponding EES registration resource.

#### 6.2.2.3	Eecs_EESRegistration_Update

##### 6.2.2.3.1	General

This service operation is used by EES to update its registration information at a given ECS.

##### 6.2.2.3.2	EES updating registration information using Eecs_EESRegistration_Update operation

To update the EES registration information at the ECS, the EES shall send a HTTP PUT or PATCH message to the Edge Configuration Server on resource URI identifying the individual EES registration resource representation, as specified in clause 9.1.2.3.3.2 for HTTP PUT message and in clause 9.1.2.3.3.4 for HTTP PATCH message. The HTTP PUT message shall replace all properties in the existing resource with the EES registration information in the request. The EESRegistration data type in the request body of the HTTP PUT message shall include the EES profile information, and may include proposed expiration time to update the registration. This request shall not replace the eesId property of the existing resource.

The HTTP PATCH request message in the EESRegistrationPatch data type includes parameters (EES Profile, and/or expiry time) that need to be replaced in the existing Individual EAS registration resource.  This request shall not replace the eesId property of the existing resource.

Upon receiving the HTTP PUT or PATCH message from the EES, the ECS shall:

1. check the registration update message from the EES to see if the EES is authorized to modify the requested registration resource;

2. if the EES is authorized to update the registration information:

a. update the resource identified by Resource URI of the EES registration information with the updated EES registration information received in the HTTP PUT or PATCH request message;

b. return the updated EES registration information in the "200 OK" response message or respond with the "204 No Content" message indicating to the EES that the EES registration information is updated successfully. In the response message, the ECS may provide an updated expiration time to indicate to the EES when the updated registration will automatically expire.

On failure, the ECS shall take proper error handling actions, as specified in clause 9.1.6, and respond to the EES with an appropriate error status code.

If the ECS determines that the received HTTP PUT or PATCH request needs to be redirected, the ECS may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative ECS where the message should be redirected. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

If the expiration time is provided, then to maintain the registration, the EES shall send a registration update prior to registration expiry time. If the registration update request is not sent before the expiry time, then the ECS shall treat EES as deregistered and remove the corresponding EES registration resource.

#### 6.2.2.4	Eecs_EESRegistration_Deregister

##### 6.2.2.4.1	General

This service operation is used by EES to deregister itself from a given ECS.

##### 6.2.2.4.2	EES deregistering from ECS using Eecs_EESRegistration_Deregister operation

To deregister itself from the ECS, the EES shall send HTTP DELETE message to the ECS, on the resource URI identifying the individual EES registration resource representation as specified in clause 9.1.2.3.3.3. Upon receiving the HTTP DELETE request, the ECS shall:

1. verify the identity of the EES and check if the EES is authorized to deregister the EES registration information;

2. if the EES is authorized to deregister the EES registration information, then the ECS shall deregister the EES profile from the ECS and delete the resource representing EES registration information.

3. return the "204 Not Content" message to the EES, indicating the successful deregistration of the EES information.

On failure, the ECS shall take proper error handling actions, as specified in clause 9.1.6, and respond to the EES with an appropriate error status code.

If the ECS determines that the received HTTP DELETE request needs to be redirected, the ECS may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative ECS where the message should be redirected. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

## 6.3	Eecs_TargetEESDiscovery Service

### 6.3.1	Service Description

The Eecs_TargetEESDiscovery API, as defined in 3GPP TS 23.558 [2], allows a service consumer (e.g., EES, ECS) to retrieve the target Enabler Server (e.g., T-EES, target CES) information at the ECS.

### 6.3.2	Service Operations

#### 6.3.2.1	Introduction

The service operation defined for Eecs_TargetEESDiscovery API is shown in the table 6.3.2.1-1.

Table 6.3.2.1-1: Operations of the Eecs_TargetEESDiscovery API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eecs_TargetEESDiscovery_Request | This service operation is used by a service consumer to retrieve the target Enabler Server information. | EES, CES |

#### 6.3.2.2	Eecs_TargetEESDiscovery_Request

##### 6.3.2.2.1	General

This service operation is used by a service consumer to retrieve the target Enabler Server information from the ECS.

##### 6.3.2.2.2	Service consumer fetching the target Enabler Server information from the ECS using Eecs_TargetEESDiscovery_Request operation

To retrieve the target Enabler Server information from the ECS, the service consumer shall send a HTTP GET message to the ECS on the "EES Profiles" collection resource as specified in clause 9.2.2.2.3.1. The service consumer includes the discovery filter information in the query parameters of the GET message, as specified in clause 9.2.2.2.3.1, to assist the ECS to determine the target Enabler Server that has the Application Server (e.g., EAS, CAS) available to serve the given AC in the UE.

Upon receiving the HTTP GET message from the service consumer, the ECS shall:

1. Process the request information;

2. verify the identity of the service consumer and check if it is authorized to retrieve the target Enabler Server information;

3. if the service consumer is authorized to retrieve the target Enabler Server information, then the ECS shall;

a. determine the target Enabler Server based on the provided query parameters in the request. If UE location information is not included in the request or not known to the ECS, then the ECS may fetch the UE location information from the 3GPP core network as specified in 3GPP TS 29.522 [10];

b. returns an HTTP "200 OK" status code with the response body including the EDN configuration and the list of T-EES(s) or target CES(s) information to the service consumer. The list of target Enabler Server includes the endpoint information to reach the target Enabler Server.

and

4.	on failure, the ECS shall take proper error handling actions, as specified in clause 9.2.6, and respond to the service consumer with an appropriate error status code.

## 6.4	Eecs_EASInfoManagement Service

### 6.4.1	Service Description

The Eecs_EASInfoManagement API exposed by the ECS, as defined in 3GPP TS 23.558 [2], enables a service consumer to:

-	retrieve the Common EAS Binding information; and

-	store, update and delete the Common EAS Binding information or receive the existing Common EAS Binding information.

NOTE:	Within an ECSP's edge deployment, this service API is exposed by ECS(s) that play the role of center of information, as specified in 3GPP TS 23.558 [2].

### 6.4.2	Service Operations

#### 6.4.2.1	Introduction

The service operations defined for Eecs_EASInfoManagement API are shown in the table 6.4.2.1-1.

Table 6.4.2.1-1: Eecs_EASInfoManagement API Service Operations

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eecs_EASInfoManagement_Get | This service operation enables a service consumer to retrieve Common EAS Binding information. | e.g., EES |
| Eecs_EASInfoManagement_Store | This service operation enables a service consumer to store the Common EAS Binding information or receive the existing Common EAS Binding information. | e.g., EES |
| Eecs_EASInfoManagement_Update | This service operation enables a service consumer to update an existing Common EAS Binding information. | e.g., EES |
| Eecs_EASInfoManagement_Delete | This service operation enables a service consumer to delete an existing Common EAS Binding information. | e.g., EES |

#### 6.4.2.2	Eecs_EASInfoManagement_Get

##### 6.4.2.2.1	General

This service operation is used by a service consumer to retrieve Common EAS Binding information from the ECS.

The following procedures are supported by the "Eecs_EASInfoManagement_Get" service operation:

-	Common EAS Binding Information Retrieval.

##### 6.4.2.2.2	Common EAS Binding Information Retrieval

1.	In order to retrieve the Common EAS Binding information from an ECS, the service consumer shall send an HTTP GET request message to the ECS targeting the URI of the "Common EAS Bindings" collection resource, with query parameters as defined in clause 9.3.3.2.3.1.

2a.	Upon reception of the HTTP GET request message from the service consumer, the ECS shall:

-	check the identity of the service consumer and whether the service consumer is authorized to retrieve the Common EAS Binding information; and

-	if the service consumer is authorized and upon successful processing of the request, respond with either:

-	an HTTP "200 OK" status code, with the response body including the requested Common EAS Binding information within the CommonEASBindResp data structure; or

-	an HTTP "204 No Content" status code, if there is no Common EAS Binding information corresponding to the received query parameters.

2b.	On failure, the ECS shall take proper error handling actions, as specified in clause 9.3.7, and respond to the service consumer with an appropriate error status code.

#### 6.4.2.3	Eecs_EASInfoManagement_Store

##### 6.4.2.3.1	General

This service operation is used by the service consumer to store the Common EAS information to the ECS.

The following procedures are supported by the "Eecs_EASInfoManagement_Store" service operation:

-	Common EAS Binding Information Storage.

##### 6.4.2.3.2	Common EAS Binding Information Storage

1.	In order to store the Common EAS Binding information at the ECS, the service consumer shall send an HTTP POST request message to the ECS targeting the URI of the "Common EAS Bindings" collection resource, with the request body including the CommonEASBindReq data structure as defined in clause 9.3.3.2.3.2.

2a.	Upon reception of the HTTP POST request message from the service consumer, the ECS shall:

-	check the identity of the service consumer and whether the service consumer is authorized to store the Common EAS Binding information at the ECS; and

-	if the service consumer is authorized and upon successful processing of the request, respond with an HTTP "201 Created" status code with the response body including the stored Common EAS Binding information within the CommonEASBindResp data structure.

2b.	On failure, the ECS shall take proper error handling actions, as specified in clause 9.3.7, and respond to the service consumer with an appropriate error status code. In particular:

-	if there is already an existing Common EAS Binding information for the targeted application group, the ECS shall reject the request and respond with an HTTP "403 Forbidden" status code with the response body including the ProblemDetailsEIMExt data structure containing:

-	the ProblemDetails data structure containing the "cause" attribute set to the "EXISTING_COMMON_EAS" application error; and

-	the CommonEASBinding data structure containing the existing Common EAS Binding information.

#### 6.4.2.4	Eecs_EASInfoManagement_Update

##### 6.4.2.4.1	General

This service operation is used by the service consumer to request the update of an existing "Individual Common EAS Binding" resource at the ECS.

The following procedures are supported by the "Eecs_EASInfoManagement_Update" service operation:

-	Common EAS Binding Information Update.

##### 6.4.2.4.2	Common EAS Binding Information Update

In order to update existing Common EAS Binding information at the ECS, the service consumer shall send an HTTP PUT/PATCH request message to the ECS targeting the URI of the corresponding "Individual Common EAS Binding" resource, with the request body including either:

-	the updated representation of the resource within the CommonEASBind data structure as defined in clause 9.3.3.3.3.2, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the CommonEASBindPatch data structure as defined in clause 9.3.3.3.3.3, in case the HTTP PATCH method is used;

Upon reception of the HTTP PUT/PATCH request message from the service consumer, the ECS shall:

-	verify the identity of the service consumer and check whether the service consumer is authorized to update the Common EAS Binding information at the ECS or not;

-	if the service consumer is authorized and upon successful processing of the request, respond with either:

-	an HTTP "200 OK" status code, with the response body including the updated representation of the "Individual Common EAS Binding" resource within the CommonEASBindResp data structure; or

-	an HTTP "204 No Content" status code.

On failure, the ECS shall take proper error handling actions, as specified in clause 9.3.7, and respond to the service consumer with an appropriate error status code.

If the ECS determines that the received HTTP PUT/PATCH request need to be redirected, the ECS may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative ECS where the message should be redirected. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

#### 6.4.2.5	Eecs_EASInfoManagement_Delete

##### 6.4.2.5.1	General

This service operation is used by the service consumer to request the deletion of an existing Common EAS Binding at the ECS.

The following procedures are supported by the "Eecs_EASInfoManagement_Delete" service operation:

-	Common EAS Binding Information Deletion.

##### 6.4.2.5.2	Common EAS Binding information Deletion

In order to delete existing Common EAS Binding information at the ECS, the service consumer shall send HTTP DELETE request message to the ECS targeting the URI of the corresponding "Individual Common EAS Binding" resource.

Upon receiving the HTTP DELETE request message, the ECS shall:

1.	verify the identity of the service consumer and check whether the service consumer is authorized to delete the Common EAS Binding information at the ECS; and

2.	if the service comsumer is authorized and upon successful processing of the request, respond with an HTTP "204 No Content" status code.

On failure, the ECS shall take proper error handling actions, as specified in clause 9.3.7, and respond to the EES with an appropriate error status code.

If the ECS determines that the received HTTP DELETE request needs to be redirected, the ECS may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative ECS towards which the request should be redirected. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6].

## 6.5	Eecs_ECSServiceProvisioning

### 6.5.1	Service Description

The Eecs_ECSServiceProvisioning service exposed by the ECS (e.g., partner ECS) enables a service consumer (e.g., ECS) to:

-	request to retrieve service provisioning information;

-	create/update/delete a service provisioning subscription; and

-	receive service provisioning event(s) related notifications.

This API shall be exposed by an ECS (e.g., partner ECS) only in roaming or federation scenarios.

#### 6.5.2.1	Introduction

The service operations defined for the Eecs_ECSServiceProvisioning service are shown in table 6.5.2.1-1.

Table 6.5.2.1-1: Eecs_ECSServiceProvisioning Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Eecs_ECSServiceProvisioning_Request | This service operation enables a service consumer to request to retrieve service provisioning information from the ECS. | e.g., ECS |
| Eecs_ECSServiceProvisioning_Subscribe | This service operation enables a service consumer to request the creation of a Service Provisioning Subscription at the ECS. | e.g., ECS |
| Eecs_ECSServiceProvisioning_UpdateSubscription | This service operation enables a service consumer to request the update of an existing Service Provisioning Subscription at the ECS. | e.g., ECS |
| Eecs_ECSServiceProvisioning_Unsubscribe | This service operation enables a service consumer to request the deletion of an existing Service Provisioning Subscription at the ECS. | e.g., ECS |
| Eecs_ECSServiceProvisioning_Notify | This service operation enables a service consumer to receive service provisioning event(s) related notifications from the ECS. | ECS |

#### 6.5.2.2	Eecs_ECSServiceProvisioning_Request

##### 6.5.2.2.1	General

This service operation is used by a service consumer to request to retrieve service provisioning information from the ECS.

The following procedures are supported by the "Eecs_ECSServiceProvisioning_Request" service operation:

-	Service Provisioning Information Retrieval Request.

##### 6.5.2.2.2	Service Provisioning Information Retrieval Request

This procedure enables a service consumer to send a request to the ECS to request to retrieve service provisioning information (see also clauses 8.17 and 8.3.3.2.2 of 3GPP°TS°23.558°[2]).

1.	In order to request to retrieve service provisioning information, the service consumer shall send an HTTP POST request to the ECS targeting the URI of the corresponding custom operation (i.e., "Request"), with the request body including the ServProvReq data structure.

2a.	Upon success, the ECS shall respond with an HTTP "200 OK" status code with the response body containing the requested service provisioning information within the ServProvResp data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 9.4.7.

#### 6.5.2.3	Eecs_ECSServiceProvisioning_Subscribe

##### 6.5.2.3.1	General

This service operation is used by a service consumer to request the creation of a Service Provisioning Subscription at the ECS.

The following procedures are supported by the "Eecs_ECSServiceProvisioning_Subscribe" service operation:

-	Service Provisioning Subscription Creation.

##### 6.5.2.3.2	Service Provisioning Subscription Creation

This procedure enables a service consumer to send a request to the ECS to request the creation of a Service Provisioning Subscription (see also clause 8.17 of 3GPP°TS°23.558°[2]).

1.	In order to create a new Service Provisioning Subscription, the service consumer shall send an HTTP POST request to the ECS targeting the URI of the "Service Provisioning Subscriptions" collection resource, with the request body including the ServProvSubsc data structure.

2a.	Upon success, the ECS shall respond with an HTTP "201 Created" status code, with the response body containing a representation of the created "Individual Service Provisioning Subscription" resource within the ServProvSubsc data structure, and an HTTP "Location" header field containing the URI of the created resource.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 9.4.7.

#### 6.5.2.4	Eecs_ECSServiceProvisioning_UpdateSubscription

##### 6.5.2.4.1	General

This service operation is used by a service consumer to request the update of an existing Service Provisioning Subscription at the ECS.

The following procedures are supported by the "Eecs_ECSServiceProvisioning_UpdateSubscription" service operation:

-	Service Provisioning Subscription Update.

##### 6.5.2.4.2	Service Provisioning Subscription Update

This procedure enables a service consumer to send a request to the ECS to request the update of an existing Service Provisioning Subscription (see also clause 8.17 of 3GPP°TS°23.558°[2]).

1.	In order to request the update of an existing Service Provisioning Subscription, the service consumer shall send an HTTP PUT/PATCH request to the ECS, targeting the URI of the corresponding "Individual Service Provisioning Subscription" resource, with the request body including either:

-	the updated representation of the resource within the ServProvSubsc data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the ServProvSubscPatch data structure, in case the HTTP PATCH method is used.

2a.	Upon success, the ECS shall update the targeted "Individual Service Provisioning Subscription" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Service Provisioning Subscription" resource within the ServProvSubsc data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 9.4.7.

#### 6.5.2.5	Eecs_ECSServiceProvisioning_Unsubscribe

##### 6.5.2.5.1	General

This service operation is used by a service consumer to request the deletion of an existing Service Provisioning Subscription at the ECS.

The following procedures are supported by the "Eecs_ECSServiceProvisioning_Unsubscribe" service operation:

-	Service Provisioning Subscription Deletion.

##### 6.5.2.5.2	Service Provisioning Subscription Deletion

This procedure enables a service consumer to send a request to the ECS to request the deletion of an existing Service Provisioning Subscription (see also clause 8.17 of 3GPP°TS°23.558°[2]).

1.	In order to request the deletion of an existing Service Provisioning Subscription, the service consumer shall send an HTTP DELETE request to the ECS targeting the corresponding "Individual Service Provisioning Subscription" resource.

2a.	Upon success, the ECS shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 9.4.7.

#### 6.5.2.6	Eecs_ECSServiceProvisioning_Notify

##### 6.5.2.6.1	General

This service operation is used by an ECS to notify a previously subscribed service consumer on:

-	service provisioning related event(s).

The following procedures are supported by the "Eecs_ECSServiceProvisioning_Notify" service operation:

-	Service Provisioning Notification.

##### 6.5.2.6.2	Service Provisioning Notification

This procedure enables the ECS to send a request to notify a previously subscribed service consumer on service provisioning related event(s) (see also clause 8.17 of 3GPP°TS°23.558°[2]).

1.	In order to notify a previously subscribed service consumer on service provisioning related event(s), the ECS shall send an HTTP POST request to the service consumer with the request URI set to "{notifUri}", where the "notifUri" is set to the value received from the service consumer during the creation/update of the corresponding Service Provisioning Subscription using the procedures defined in clauses 6.5.2.3 and 6.5.2.4, and the request body including the ServProvNotif data structure.

2a.	Upon success, the service consumer shall respond to the ECS with an HTTP "204 No Content" status code to acknowledge the successful reception and processing of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 9.4.7.

## 6.6	Eecs_ECSDiscovery Service

### 6.6.1	Service Description

The Eecs_ECSDiscovery API, as defined in 3GPP TS 23.558 [2], allows a service consumer (e.g., partner ECS) to:

-	retrieve partner ECS information from the ECS; and

-	receive partner ECS information update notifications.

### 6.6.2	Service Operations

#### 6.6.2.1	Introduction

The service operation defined for Eecs_ECSDiscovery API is shown in the table 6.6.2.1-1.

Table 6.6.2.1-1: Operations of the Eecs_ECSDiscovery API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eecs_ECSDiscovery_Request | This service operation is used by the service consumer to retrieve partner ECS information. | e.g., ECS |
| Eecs_ECSDiscovery_Notify | This service operation is used by the ECS to notify a previously subscribed service consumer on partner ECS information. | e.g., ECS |

#### 6.6.2.2	Eecs_ECSDiscovery_Request

##### 6.6.2.2.1	General

This service operation is used by the service consumer to retrieve partner ECS information from the ECS.

##### 6.6.2.2.2	Service consumer fetching partner ECS information from the ECS using the Eecs_ECSDiscovery_Request operation

To retrieve partner ECS information, the service consumer shall send an HTTP POST request message to the ECS targeting the "ECS Information" collection resource, with the request body including the ECSInfoDiscoveryReq data structure.

Upon reception of the HTTP POST message from the service consumer, the ECS shall:

1.	verify the identity of the service consumer and check if it is authorized to trigger this request;

2.	if the service consumer ECS is authorized, the ECS shall;

a.	determine the ECS information that may be shared with the service consumer based on the federation information and the ECSP policies; and

b.	return an HTTP "200 OK" status code with the response body including the ECSInfoDiscoveryResp data structure containing the requested ECS information.

On failure, the ECS shall take proper error handling actions, as specified in clause 9.5.6, and respond to the service consumer with an appropriate error status code.

#### 6.6.2.3	Eecs_ECSDiscovery_Request

##### 6.6.2.3.1	General

This service operation is used by the service consumer to notify a previously subscribed service consumer on partner ECS information.

##### 6.6.2.3.2	ECS notifying the ECS information to the service consumer using Eecs_ECSDiscovery_Notification operation

When the ECS determines to notify the service consumers with updated ECS information, the ECS shall send an HTTP POST request message targeting the notification URI received in the corresponding ECS discovery request, with the request body including the EcsInfoDiscNotif data structure.

Upon reception the HTTP POST message, the service consumer shall process the notification and shall respond to the ECS with an HTTP "204 No Content" status code.

On failure, the service consumer shall take proper error handling actions, as specified in clause 9.5.6, and respond to the ECS with an appropriate error status code.

## 6.7	Eecs_ACREvents

### 6.7.1	Service Description

The Eecs_ACREvents API exposed by the ECS, as defined in 3GPP TS 23.558 [2], enables a service consumer to:

-	create/update/delete a ACR Events Subscription; and

-	receive ACR event(s) related notifications.

### 6.7.2	Service Operations

#### 6.7.2.1	Introduction

The service operations defined for the Eecs_ACREvents service are shown in table 6.7.2.1-1.

Table 6.7.2.1-1: Eecs_ACREvents Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Eecs_ACREvents_Subscribe | This service operation enables a service consumer to request the creation of a ACR Events Subscription at the ECS. | e.g., EES |
| Eecs_ACREvents_Update | This service operation enables a service consumer to request the update of an existing ACR Events Subscription at the ECS. | e.g., EES |
| Eecs_ACREvents_Unsubscribe | This service operation enables a service consumer to request the deletion of an existing ACR Events Subscription at the ECS. | e.g., EES |
| Eecs_ACREvents_Notify | This service operation enables a service consumer to receive ACR event(s) related notifications from the ECS. | ECS |

#### 6.7.2.2	Eecs_ACREvents_Subscribe

##### 6.7.2.2.1	General

This service operation is used by a service consumer to request the creation of a ACR Events Subscription at the ECS.

The following procedures are supported by the "Eecs_ACREvents_Subscribe" service operation:

-	ACR Events Subscription Creation.

##### 6.7.2.2.2	ACR Events Subscription Creation

This procedure enables a service consumer to send a request to the ECS to request the creation of a ACR Events Subscription.

In order to create a new ACR Events Subscription, the service consumer shall send an HTTP POST request message to the ECS targeting the URI of the "ACR Events Subscriptions" collection resource, with the request body including the AcrEventsSubsc data structure as specified in clause 9.6.3.2.3.2.

Upon reception of the HTTP POST request message from the service consumer, the ECS shall verify the identity of the service consumer and check whether the service consumer is authorized to create a new ACR Events Subscription. Then:

-	if the service consumer is authorized and upon successful processing of the request, the ECS shall respond with an HTTP "201 Created" status code, with the response body containing a representation of the created "Individual ACR Events Subscription" resource within the AcrEventsSubsc data structure, and an HTTP Location header field containing the URI of the created resource; and

-	on failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 9.6.7.

#### 6.7.2.3	Eecs_ACREvents_Update

##### 6.7.2.3.1	General

This service operation is used by a service consumer to request the update of an existing ACR Events Subscription at the ECS.

The following procedures are supported by the "Eecs_ACREvents_Update" service operation:

-	ACR Events Subscription Update.

##### 6.7.2.3.2	ACR Events Subscription Update

This procedure enables a service consumer to send a request to the ECS to request the update of an existing ACR Events Subscription.

In order to request the update of an existing ACR Events Subscription, the service consumer shall send an HTTP PUT or PATCH request message to the ECS, targeting the URI of the corresponding "Individual ACR Events Subscription" resource, with the request body including either:

-	the updated representation of the resource within the AcrEventsSubsc data structure as specified in clause 9.6.3.3.3.2, in case the HTTP PUT method is used; or

-	the requested modification to the resource within the AcrEventsSubscPatch data structure as specified in clause 9.6.3.3.3.3, in case the HTTP PATCH method is used.

Upon reception of the HTTP PUT or PATCH request message from the service consumer, the ECS shall check whether the service consumer is autorized to update the ACR Events Subscription. Then:

-	if the service consumer is authorized and upon successful processing of the request, the ECS shall update the targeted "Individual ACR Events Subscription" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual ACR Events Subscription" resource within the AcrEventsSubsc data structure; or

-	an HTTP "204 No Content" status code;

and

-	on failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 9.6.7.

#### 6.7.2.4	Eecs_ACREvents_Unsubscribe

##### 6.7.2.4.1	General

This service operation is used by a service consumer to request the deletion of an existing ACR Events Subscription at the ECS.

The following procedures are supported by the "Eecs_ACREvents_Unsubscribe" service operation:

-	ACR Events Subscription Deletion.

##### 6.7.2.4.2	ACR Events Subscription Deletion

This procedure enables a service consumer to send a request to the ECS to request the deletion of an existing ACR Events Subscription.

In order to request the deletion of an existing ACR Events Subscription, the service consumer shall send an HTTP DELETE request message to the ECS targeting the corresponding "Individual ACR Events Subscription" resource as specified in clause 9.6.3.3.3.4.

Upon reception of the HTTP DELETE request message from the service consumer, the ECS shall verify the identity of the service consumer and check whether the service consumer is authorized to delete the ACR Events Subscription. Then:

-	if the service consumer is authorized and the deletion is successful, the ECS shall respond with an HTTP "204 No Content" status code to the service consumer; and

-	on failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 9.6.7.

#### 6.7.2.5	Eecs_ACREvents_Notify

##### 6.7.2.5.1	General

This service operation is used by an ECS to notify a previously subscribed service consumer on:

-	ACR related event(s).

The following procedures are supported by the "Eecs_ACREvents_Notify" service operation:

-	ACR Events Notification.

##### 6.7.2.5.2	ACR Events Notification

This procedure enables the ECS to send a request to notify a previously subscribed service consumer on ACR related event(s).

In order to notify a previously subscribed service consumer on ACR Events related event(s), the ECS shall send an HTTP POST request message to the service consumer with the request URI set to "{notifUri}", where the "notifUri" is set to the value received from the service consumer during the creation/update of the corresponding ACR Events Subscription using the procedures defined in clauses 6.7.2.2 and 6.7.2.3, and the request body including the AcrEventsNotif data structure as specified in clause 9.6.5.2.

Upon successful reception of the HTTP POST request message, the service consumer shall respond to the ECS with an HTTP "204 No Content" status code to acknowledge the successful reception and processing of the notification.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 9.6.7.

# 6A	Services offered by the Cloud Application Server (CAS)

## 6A.1	Introduction

The table 6A.1-1 lists the CAS APIs below with the service name. A service description clause for each API gives a general description of the related API.

Table 6A.1-1: List of CAS Service APIs

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Ecas_SelectedEES | Declare | Request/Response | EES |

Table 6A.1-2 summarizes the corresponding CAS APIs defined in this specification.

Table 6A.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Ecas_SelectedEES | 8A.1 | The service consumer declares the selected EES information to the CAS. | TS29558_Ecas_SelectedEES.yaml | ecas-selected-ees | A.14 |

## 6A.2	Ecas_SelectedEES Service

### 6A.2.1	Service Description

The Ecas_SelectedEES API, as defined in 3GPP TS 23.558 [2], allows a service consumer (e.g., EES) to inform the CAS of the selected EES during ACR.

### 6A.2.2	Service Operations

#### 6A.2.2.1	Introduction

The service operation defined for Ecas_SelectedEES API is shown in the table 6A.2.2.1-1.

Table 6A.2.2.1-1: Operations of the Ecas_SelectedEES API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Ecas_SelectedEES_Declare | This service operation is used by the service consumer to inform the CAS of the selected EES during the ACR from EAS to CAS. | e.g., EES |

#### 6A.2.2.2	Ecas_SelectedEES_Request

##### 6A.2.2.2.1	General

This service operation is used by a service consumer to inform the CAS of the selected EES.

##### 6A.2.2.2.2	Service consumer informing the CAS of the selected EES using Ecas_SelectedEES_Declare operation

To inform the CAS of the selected EES during the ACR, the service consumer shall send an HTTP POST request to the CAS targeting the URI of the corresponding custom operation (i.e., "Declare"), with the request body including the SelEESDecInfo data structure defined in clause 8A.1.6.2.2.

Upon reception of the HTTP POST request message from the service consumer, the CAS shall:

1.	check whether the service consumer is authorized to declare the selected EES information;

2.	if the service consumer is authorized, process the request, store the received information and respond with an HTTP "204 No Content" status code; and

3.	on failure, the CAS shall take proper error handling actions, as specified in clause 8A.1.7, and respond to the service consumer with an appropriate error status code.

# 6B	Services offered by the Cloud Enabler Server (CES)

## 6B.1	Introduction

Table 6B.1-1 lists the CES APIs defined in this specification.

Table 6B.1-1: List of CES Service APIs

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |

Table 6B.1-2 summarizes the corresponding CES APIs defined in this specification.

Table 6B.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |

Table 6B.1-3 lists the EES APIs that are defined in this specification and may be reused (i.e., exposed) by the CES.

Table 6B.1-3: API Descriptions of service APIs reused by the CES

| Service Name | Clause | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- |
| Eees_EASRegistration | 5.2 | TS29558_Eees_EASRegistration.yaml | eees-easregistration | A.2 |
| Eees_UELocation | 5.3 | TS29558_Eees_UELocation.yaml | eees-uelocation | A.3 |
| Eees_UEIdentifier | 5.4 | TS29558_Eees_UEIdentifier.yaml | eees-ueidentifier | A.4 |
| Eees_AppClientInformation | 5.5 | TS29558_Eees_AppClientInformation.yaml | eees-appclientinformation | A.5 |
| Eees_SessionWithQoS | 5.6 | TS29558_Eees_SessionWithQoS.yaml | eees-session-with-qos | A.6 |
| Eees_ACRManagementEvent | 5.8 | TS29558_Eees_ACRManagementEvent.yaml | eees-acrmgntevent | A.7 |
| Eees_EECContextRelocation | 5.10 | TS29558_Eees_EECContextRelocation.yaml | eees-eeccontextreloc | A.8 |
| Eees_EELManagedACR | 5.11 | TS29558_Eees_EELManagedACR.yaml | eees-eel-acr | A.9 |
| Eees_ACRStatusUpdate | 5.12 | TS29558_Eees_ACRStatusUpdate.yaml | eees-acrstatus-update | A.10 |
| Eees_ACRParameterInformation | 5.13 | TS29558_Eees_ACRParameterInformation.yaml | eees-acr-param | A.13 |
| Eees_CommonEASAnnouncement | 5.14 | TS29558_Eees_CommonEASAnnouncement.yaml | eees-ceas | A.15 |
| Eees_TrafficInfluenceEAS | 5.15 | TS29558_Eees_TrafficInfluenceEAS.yaml | eees-tie | A.17 |
| Eees_EASDiscovery | (NOTE) | TS24558_Eees_EASDiscovery.yaml | eees-easdiscovery | (NOTE) |
| Eees_AppContextRelocation | (NOTE) | TS24558_AppContextRelocation | eees-appctxtreloc | (NOTE) |
| NOTE: These APIs are defined in 3GPP TS 24.558 [14]. |  |  |  |  |

NOTE:	In the provisions defining the above APIs reused by the CES and listed in Table 6B.1-3, the CES takes the role of the EES.

# 7	Information applicable to several APIs

## 7.1	General

The EDGEAPP APIs as specified in this document allow secure access to the capabilities provided by the functional entities, Edge Enabler Server and Edge Configuration Server.

This document specifies the procedures triggered at different functional entities as a result of API invocation requests and event notifications. The stage-2 level requirements and signalling flows are defined in 3GPP TS 23.558 [2].

Several design aspects, as mentioned in the following clauses, are specified in 3GPP TS 29.122 [6] and referenced by this specification.

## 7.2	Data Types

### 7.2.1	General

This clause defines structured data types, simple data types and enumerations that are applicable to several APIs defined in the present specification and can be referenced from data structures defined in the subsequent clauses.

In addition, data types that are defined in OpenAPI 3.0.0 Specification [3] can also be referenced from data structures defined in the subsequent clauses.

NOTE:	As a convention, data type's names in the present specification are with an upper-case letter in the beginning. Parameters are with a lower-case letter in the beginning. As an exception, data types that are also defined in OpenAPI 3.0.0 Specification [3] can use a lower-case case letter in the beginning for consistency.

Table 7.2.1-1 specifies data types re-used by APIs in this specification from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the APIs of this specification.

Table 7.2.1-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Uri | 3GPP TS 29.122 [6] |  |

### 7.2.2	Referenced structured data types

Table 7.2.2-1 lists structured data types defined in this specification referenced by multiple services:

Table 7.2.2-1: Referenced Structured Data Types

| Data type | Reference | Description |
| --- | --- | --- |

### 7.2.3	Referenced simple data types and enumerations

Following simple data types defined in Table 7.2.3-1 are applicable to several APIs in this document:

Table 7.2.3-1: Simple data types applicable to several APIs

| Type name | Reference | Description |
| --- | --- | --- |

## 7.3	Usage of HTTP

For all the APIs in this document, the usage of HTTP shall be as specified in clause 5.2.2 of 3GPP TS 29.122 [6], with the clarification that SCA/AS is the functional entity invoking an EDGEAPP API.

For all the APIs, to provide the interface protection over EDGE-3/6/9/10 interfaces, HTTP over TLS shall be used as specified in 3GPP TS 33.558 [20].

## 7.4	Content type

The content type for the HTTP messages shall be as specified in clause 5.2.3 of 3GPP TS 29.122 [6].

## 7.5	URI structure

### 7.5.1	Resource URI structure

The resource URI structure of all the APIs specified in this document shall be as specified in clause 5.2.4 of 3GPP TS 29.122 [6].

### 7.5.2	Custom operations URI structure

The custom operation definition is in Annex C of 3GPP TS 29.501 [5].

The URI of a custom operation which is associated with a resource shall have the following structure:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>/<custOpName>

Custom operations can also be associated with the service instead of a resource. The URI of a custom operation which is not associated with a resource shall have the following structure:

{apiRoot}/<apiName>/<apiVersion>/<custOpName>

In the above URI structures, "apiRoot", "apiName", "apiVersion" and "apiSpecificResourceUriPart" are as defined in clause 7.5.1 and "custOpName" represents the name of the custom operation as defined in clause 5.1.3.2 of 3GPP TS 29.501 [5].

## 7.6	Notifications

The functional entities

-	shall support the delivery of notifications using a separate HTTP connection towards an address;

-	may support testing delivery of notifications; and

-	may support the delivery of notification using WebSocket protocol (see IETF RFC 6455 [7]),

as described in 3GPP TS 29.122 [6], with the following clarifications:

-	the SCEF is EES for Eees APIs; and

-	the SCS/AS is the Subscriber entity invoking an EDGEAPP API.

## 7.7	Error handling

Response bodies for error handling, as described in clause 5.2.6 of 3GPP TS 29.122 [6], are applicable to all APIs in the present specification unless specified otherwise, with the following clarifications:

-	the SCEF is EES for Eees APIs or ECS for Eecs APIs ; and

-	the SCS/AS is the functional entity invoking an EDGEAPP API

## 7.8	Feature negotiation

The functional entity invoking an API (i.e. the EAS server) and the EDGEAPP server use feature negotiation procedures defined in clause 5.2.7 of 3GPP TS 29.122 [6] to negotiate the supported features, with the following clarifications:

-	description of the SCEF applies to the EES for Eees APIs or ECS for Eecs APIs; and

-	description of the SCS/AS applies to the functional entity invoking an EDGEAPP API.

## 7.9	HTTP headers

The HTTP headers described in 3GPP TS 29.122 [6] are applicable to all APIs in this document.

## 7.10	Conventions for Open API specification files

The conventions for Open API specification files as specified in clause 5.2.9 of 3GPP TS 29.122 [6] shall be applicable for all APIs in this document.

# 8	Edge Enabler Server API Definitions

## 8.1	Eees_EASRegistration API

### 8.1.1	Introduction

The Eees_EASRegistration service shall use the Eees_EASRegistration API.

The API URI of the Eees_EASRegistration API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure as defined in clause 7.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in clause 7.5.

-	The <apiName> shall be "eees-easregistration".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.1.2.

### 8.1.2	Resources

#### 8.1.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 8.1.2.1-1 depicts the resource URIs structure for the Eees_EASRegistration API.

Figure 8.1.2.1-1: Resource URI structure of the Eees_EASRegistration API

Table 8.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| EAS Registrations | /registrations | POST | Creates a new EAS Registration at the EES. |
| Individual EAS Registration | /registrations/{registrationId} | GET | Retrieve an existing "Individual EAS Registration" resource. |
|  |  | PUT | Update an existing "Individual EAS Registration" resource. |
|  |  | DELETE | Remove an existing "Individual EAS Registration" resource. |
|  |  | PATCH | Modify an existing "Individual EAS Registration" resource. |

NOTE:	When this API is exposed by a CES as specified in clause 6B, the "EAS" is replaced by the "CAS" in the clauses under clause 8.1 and the OpenAPI description in Annex A.2. In other words, the "EAS Registrations" resource contains the collection of CAS Registrations managed by the CES and the "Individual EAS Registration" resource contains a CAS Registration managed by the CES.

#### 8.1.2.2	Resource: EAS Registrations

##### 8.1.2.2.1	Description

This resource represents the Collection of EAS Registrations managed by the EES.

##### 8.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/eees-easregistration/<apiVersion>/registrations

This resource shall support the resource URI variables defined in the table 8.1.2.2.2-1.

Table 8.1.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5. |

##### 8.1.2.2.3	Resource Standard Methods

###### 8.1.2.2.3.1	POST

This method shall support the URI query parameters specified in table 8.1.2.2.3.1-1.

Table 8.1.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.1.2.2.3.1-2 and the response data structures and response codes specified in table 8.1.2.2.3.1-3.

Table 8.1.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EASRegistration | M | 1 | Represents the parameters of the EAS Registration to be created. |

Table 8.1.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EASRegistration | M | 1 | 201 Created | Successful case. The EAS Registration is successfully created at EES and a representation of the created "Individual EAS Registration" resource is returned in the response body. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 8.1.2.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/eees-easregistration/<apiVersion>/registrations/{registrationId} |

##### 8.1.2.2.4	Resource Custom Operations

There are no resource Custom Operations defined for this resource in this release of the specification.

#### 8.1.2.3	Resource: Individual EAS Registration

##### 8.1.2.3.1	Description

This "Individual EAS Registration" resource represents an individual EAS registration managed by the EES.

##### 8.1.2.3.2	Resource Definition

Resource URI: {apiRoot}/eees-easregistration/<apiVersion>/registrations/{registrationId}

This resource shall support the resource URI variables defined in the table 8.1.2.3.2-1.

Table 8.1.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5. |
| registrationId | string | Represents the identifier of the "Individual EAS Registration" resource. |

##### 8.1.2.3.3	Resource Standard Methods

###### 8.1.2.3.3.1	GET

This method shall support the URI query parameters specified in table 8.1.2.3.3.1-1.

Table 8.1.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.1.2.3.3.1-2 and the response data structures and response codes specified in table 8.1.2.3.3.1-3.

Table 8.1.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.1.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EASRegistration | M | 1 | 200 OK | Successful case. The requested "Individual EAS Registration" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 8.1.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative EES. |

Table 8.1.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative EES. |

###### 8.1.2.3.3.2	PUT

This method enables to update an existing "Individual EAS Registration" resource at the EES. The values of the "suppFeat" within the EASRegistration data type and the "easId" attribute within the EASProfile data type shall not be updated.

This method shall support the URI query parameters specified in the table 8.1.2.3.3.2-1.

Table 8.1.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.1.2.3.3.2-2 and the response data structures and response codes specified in table 8.1.2.3.3.2-3.

Table 8.1.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EASRegistration | M | 1 | Represents the updated representation of the "Individual EAS Registration" resource. |

Table 8.1.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EASRegistration | M | 1 | 200 OK | Successful case. The "Individual EAS Registration" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual EAS Registration" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status code for the HTTP PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 8.1.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative EES. |

Table 8.1.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative EES. |

###### 8.1.2.3.3.3	DELETE

This method shall support the URI query parameters specified in the table 8.1.2.3.3.3-1.

Table 8.1.2.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.1.2.3.3.3-2 and the response data structures and response codes specified in table 8.1.2.3.3.3-3.

Table 8.1.2.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.1.2.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual EAS Registration" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 8.1.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative EES. |

Table 8.1.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative EES. |

###### 8.1.2.3.3.4	PATCH

This method shall support the URI query parameters specified in the table 8.1.2.3.3.4-1.

Table 8.1.2.3.3.4-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.1.2.3.3.4-2 and the response data structures and response codes specified in table 8.1.2.3.3.4-3.

Table 8.1.2.3.3.4-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EASRegistrationPatch | M | 1 | Represents the parameters to request the modification of the "Individual EAS Registration" resource. |

Table 8.1.2.3.3.4-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EASRegistration | M | 1 | 200 OK | Successful case. The "Individual EAS Registration" resource is successfully modified and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual EAS Registration" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 8.1.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative EES. |

Table 8.1.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative EES. |

##### 8.1.2.3.4	Resource Custom Operations

There are no resource Custom Operations defined for this resource in this release of the specification.

### 8.1.3	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 8.1.4	Notifications

There are no notifications defined for this API in this release of the specification.

### 8.1.5	Data Model

#### 8.1.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API

Table 8.1.5.1-1 specifies the data types defined specifically for the Eees_EASRegistration API service.

Table 8.1.5.1-1: Eees_EASRegistration API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| Affinity | 8.1.5.3.7 | Represents the affinity requirements. | EdgeApp_2 |
| AssociatedDevice | 8.1.5.2.11 | Represents the associated device along with the UE. | MetaverseApp |
| BdlType | 8.1.5.3.6 | Represents the bundle type. | EdgeApp_2 |
| CoordinatedAcrReqs | 8.1.5.2.10 | Represents the coordinated ACR related requirements. | EdgeApp_2 |
| DeviceType | 8.1.5.3.10 | Represent the device type. | MetaverseApp |
| EASBdlReqs | 8.1.5.2.9 | Represents bundle requirements. | EdgeApp_2 |
| EASBundleInfo | 8.1.5.2.8 | Represents bundle information. | EdgeApp_2 |
| EASCategory | 8.1.5.3.4 | Represents the EAS category. |  |
| EASProfile | 8.1.5.2.3 | Represents the EAS Profile. |  |
| EASRegistration | 8.1.5.2.2 | Represents the registration information of an EAS. |  |
| EASRegistrationPatch | 8.1.5.2.6 | Represents the requested modifications to an EAS's registration. |  |
| EASServiceKPI | 8.1.5.2.4 | Represents the service KPIs. |  |
| EASStatus | 8.1.5.3.9 | Represents the EAS status. | EdgeApp_3 |
| EndPoint | 8.1.5.2.5 | Represents the end point information. |  |
| FailureAction | 8.1.5.3.8 | Represents the bundle related failure action during ACR. | EdgeApp_2 |
| PermissionLevel | 8.1.5.3.3 | Represents the service permission level. |  |
| TransContSuppDetails | 8.1.5.2.7 | Represents the detailed information about the EAS (e.g. SEALDD Server) capability for seamless transport layer service contnuity. | SEALDDSupport |
| TransportProtocol | 8.1.5.3.5 | Represents the transport layer protocol. | SEALDDSupport |

Table 8.1.5.1-2 specifies data types re-used by the Eees_EASRegistration API service from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Eees_EASRegistration API.

Table 8.1.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ACRScenario | Clause 9.1.5.3.3 | Represent the ACR scenarios. |  |
| BitRate | 3GPP TS 29.571 [8] | Represents a bit rate. |  |
| DateTime | 3GPP TS 29.122 [6] | Represents a date and a time. |  |
| DateTimeRm | 3GPP TS 29.571 [8] | Represents the same as the DataTime data structure but with the OpenAPI "nullable" property set to "true". |  |
| DurationSec | 3GPP TS 29.122 [6] | Represents a time duration expressed in units of seconds. |  |
| Fqdn | 3GPP TS 29.571 [8] | Represents an FQDN. |  |
| Ipv4Addr | 3GPP TS 29.122 [6] | Represents the IPv4 address. |  |
| Ipv6Addr | 3GPP TS 29.122 [6] | Represents the IPv6 address. |  |
| PlmnIdNid | 3GPP TS 29.571 [8] | Represents the network identifier. | EdgeApp_2 |
| ServiceArea | Clause 9.1.5.2.5 | Represents the topological and geographical service area information. |  |
| RouteToLocation | 3GPP TS 29.571 [8] | Represents the routing information. |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [6] | Represents a time schedule. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported features. |  |
| Uinteger | 3GPP TS 29.571 [8] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.122 [6] | Represents a URI. |  |

#### 8.1.5.2	Structured data types

##### 8.1.5.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 8.1.5.2.2	Type: EASRegistration

Table 8.1.5.2.2-1: Definition of type EASRegistration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easProf | EASProfile | M | 1 | The profile information of the EAS. |  |
| expTime | DateTime | O | 0..1 | Identifies the expiration time for the EAS registration. If the expiration time is not present, then it indicates that the registration of EAS never expires. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported optional features of the API as described in clause 7.8. This attribute shall be present only when feature negotiation is required. |  |

##### 8.1.5.2.3	Type: EASProfile

Table 8.1.5.2.3-1: Definition of type EASProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easId | string | M | 1 | The application identifier of the EAS (e.g. URI, FQDN). |  |
| endPt | EndPoint | M | 1 | Endpoint information (URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |  |
| allowedPlmnId | PlmnIdNid | O | 0..1 | Contains the allowed PLMN ID from which the subscriber can consume the services of the EAS. (NOTE 4, NOTE 5) | EdgeApp_2 |
| allowedPlmnIds | array(PlmnIdNid) | O | 1..N | Contains the allowed PLMN ID(s) from which the subscriber can consume the services of the EAS. (NOTE 5) | EdgeApp_3 |
| easBdlInfos | array(EASBundleInfo) | O | 1..N | Represents the list of bundle(s) to which the EAS (identified by the "easId" attribute) belongs, and for each one of them, the corresponding EAS bundle information. (NOTE 3) | EdgeApp_2 |
| acIds | array(string) | O | 1..N | Identities of the Application Clients that can be served by the EAS. |  |
| provId | string | O | 0..1 | Identifier of the ASP that provides the EAS. |  |
| type | EASCategory | O | 0..1 | The EAS type with the 3GPP standardized value set. (NOTE 1). |  |
| flexEasType | string | O | 0..1 | The EAS type with the flexible value set. (NOTE 1) |  |
| scheds | array(ScheduledCommunicationTime) | O | 1..N | The availability schedule of the EAS. |  |
| svcArea | ServiceArea | O | 0..1 | The list of geographical and topological areas that the EAS serves. ACs in the UE that are outside the area shall not be served. |  |
| svcKpi | EASServiceKPI | O | 0..1 | Service characteristics provided by the EAS. |  |
| permLvl | array(PermissionLevel) | O | 1..N | Level of service permissions supported by the EAS. |  |
| easFeats | array(string) | O | 1..N | Service specific features supported by the EAS (e.g. single vs multi-player gaming service). |  |
| svcContSupp | array(ACRScenario) | O | 1..N | The ACR scenarios supported by the EAS for service continuity. If this attribute is not present, then the EAS does not support service continuity. |  |
| svcContSuppExt1 | array(EASBundleInfo) | O | 1..N | Represents the information related to the EAS ability to handle bundled EAS coordinated ACRs. This attribute may be present only when the "svcContSupp" attribute is also present. When this attribute is present, it indicates that the EAS (identified by the "easId" attribute) is able to handle bundled EAS coordinated ACRs and contains the information of the bundle(s) for which the EAS is able to handle bundled EAS coordinated ACRs. | EdgeApp_2 |
| transContSupp | TransContSuppDetails | O | 0..1 | Represents the detailed information about the EAS (e.g. SEALDD Server) capability for seamless transport layer service continuity. If this attribute is not present, then the EAS does not support the seamless transport layer service continuity capability. | SEALDDSupport |
| appLocs | array(RouteToLocation) | O | 1..N | List of DNAI(s) and the corresponding N6 traffic routing information/routing profile ID, associated with the EAS. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |  |
| avlRep | DurationSec | O | 0..1 | The period indicating to the EES, how often the EES needs to check the EAS's availability after a successful registration. |  |
| status | string | O | 0..1 | EAS status (e.g. Enabled, Disabled, Overload Warning, etc.) (NOTE 6) |  |
| statusVal | EASStatus | O | 0..1 | Contains the EAS status. (NOTE 6) | EdgeApp_3 |
| genCtxDur | DurationSec | O | 0..1 | Contains the general context holding time duration, which indicates the time duration during which the EAS holds the application context in case of an ACR for service continuity planning. | EdgeApp_2 |
| easSyncSupp | boolean | O | 0..1 | Indicates whether the EAS supports content synchronization between EASs. - When set to "true", it indicates that content synchronization between EASs is supported by the EAS. - When set to "false", it indicates that content synchronization between EASs is not supported by the EAS. - The default value when this attribute is omitted is "false". | EdgeApp_2 |
| assoDevList | array(AssociatedDevice) | O | 1..N | Contains the list of associated devices required along with the UE in order to provide the service to the user. | MetaverseApp |
| NOTE 1: The "flexEasType" attribute and the "type" attribute are mutually exclusive. Either one of them may be provided. The same attribute should be used when this data type is conveyed over the EDGE-1 and EDGE-3 interfaces (i.e. for the Eees_EASRegistration and the Eees_EASDiscovery APIs). NOTE 2: Void. NOTE 3: Within any instance of the EASBundleInfo data structure used to encode an array element of this attribute, the "easIdsList" attribute may be present only when the "bdlId" attribute is also present and the "bdlType" attribute is set to "PROXY". NOTE 4: When the "EdgeApp_3" feature is not supported, the "allowedPlmnId" attribute shall include only the MNO information of the leading ECSP in case of edge node sharing scenario and the "allowedPlmnIds" attribute shall not be present. NOTE 5: When the "EdgeApp_3" feature is supported and the "allowedPlmnIds" attribute is present, then the "allowedPlmnId" attribute shall not be present. NOTE 6: When the "EdgeApp_3" feature is supported, then either the "statusVal" attribute or the "status" attribute may be present. |  |  |  |  |  |

##### 8.1.5.2.4	Type: EASServiceKPI

Table 8.1.5.2.4-1: Definition of type EASServiceKPI

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| maxReqRate | Uinteger | O | 0..1 | Maximum request rate from the Application Client supported by the EAS. The minimum and maximum value shall be 0 and 100 respectively. |  |
| maxRespTime | Uinteger | O | 0..1 | The maximum response time, in the units of milliseconds, advertised for the AC's service requests. This includes the round trip time of the request and response packet for communication between the AC and EAS, the processing time at the EAS and time required by EAS to consume any 3GPP core network capabilities. |  |
| avail | Uinteger | O | 0..1 | Advertised percentage of time the server is available for the AC's use. The minimum and maximum value shall be 0 and 100 respectively. |  |
| avlComp | Uinteger | O | 0..1 | The maximum compute resource available for the AC. |  |
| avlGraComp | Uinteger | O | 0..1 | The maximum graphical compute resource available for the AC. |  |
| avlMem | Uinteger | O | 0..1 | The maximum memory resource available for the AC. |  |
| avlStrg | Uinteger | O | 0..1 | The maximum storage resource available for the AC. |  |
| connBand | BitRate | O | 0..1 | The connection bandwidth in Kbit/s advertised for the AC's use. |  |

##### 8.1.5.2.5	Type: EndPoint

Table 8.1.5.2.5-1: Definition of type EndPoint

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| fqdn | Fqdn | C | 0..1 | Fully Qualified Domain Name of the Edge server. (NOTE) |  |
| ipv4Addrs | array(Ipv4Addr) | C | 1..N | IPv4 addresses of the Edge server. (NOTE) |  |
| ipv6Addrs | array(Ipv6Addr) | C | 1..N | IPv6 addresses of the Edge server. (NOTE) |  |
| uri | Uri | C | 0..1 | URI information of the Edge server (NOTE) |  |
| NOTE: One of the addressing parameters (fqdn, ipv4Addrs, ipv6Addrs, uri attributes) shall be included. |  |  |  |  |  |

##### 8.1.5.2.6	Type: EASRegistrationPatch

Table 8.1.5.2.6-1: Definition of type EASRegistrationPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easProf | EASProfile | O | 0..1 | The profile information of the EAS. |  |
| expTime | DateTimeRm | O | 0..1 | Identifies the expiration time for the EAS registration. If the expiration time is not present, then it indicates that the registration of EAS never expires. |  |

##### 8.1.5.2.7	Type: TransContSuppDetails

Table 8.1.5.2.7-1: Definition of type TransContSuppDetails

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| transProtocs | array(TransportProtocol) | M | 1..N | Indicates the transport layer protocols supported for EAS (e.g. SEALDD Server) context transfer using the seamless transport layer service continuity capability. |  |

##### 8.1.5.2.8	Type: EASBundleInfo

Table 8.1.5.2.8-1: Definition of type EASBundleInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| bdlType | BdlType | M | 1 | Represents the bundle type. |  |
| bdlId | string | C | 0..1 | Contains the identifier of the bundle. (NOTE) |  |
| easIdsList | array(string) | C | 1..N | Contains the list of the identifier(s) of the EAS(s) constituting the bundle. (NOTE) |  |
| easBdlReqs | EASBdlReqs | O | 0..1 | Contains the Bundle Requirements. |  |
| mainEasId | string | O | 0..1 | Represents the identifier of the EAS that plays the role of the main EAS within the bundle. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

##### 8.1.5.2.9	Type: EASBdlReqs

Table 8.1.5.2.9-1: Definition of type EASBdlReqs

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| coordinatedEasDisc | boolean | O | 0..1 | Indicates whether or not coordinated EAS discovery is required (i.e., when the EAS discovery request for one of the EAS(s) of the bundle is processed, then the corresponding EAS discovery response should include information of all the EAS(s) of the bundle). - When set to "true", it indicates that coordianted EAS discovery is required. - When set to "false", it indicates that coordinated EAS discovery is not required. - The default value when this attribute is omitted is "false". |  |
| coordinatedAcr | CoordinatedAcrReqs | O | 0..1 | Represents the coordinated ACR requirements for the EAS bundle. |  |
| affinity | Affinity | O | 0..1 | Represents the affinity requirements of the EAS bundle. |  |

##### 8.1.5.2.10	Type: CoordinatedAcrReqs

Table 8.1.5.2.10-1: Definition of type CoordinatedAcrReqs

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| coordinatedAcrInd | boolean | M | 1 | Indicates whether or not coordinated ACR is required (i.e., when ACR is initiated for one of the EAS(s) of the bundle, then ACR should be initiated for all the other EAS(s) of the bundle). - When set to "true", it indicates that coordianted ACR is required. - When set to "false", it indicates that coordianted ACR is not required. |  |
| failureAction | FailureAction | O | 0..1 | Indicates the action to be taken when ACR for one or more EAS(s) of the EAS bundle fails, i.e., whether the ACR for all the other EAS(s) of the bundle shall proceed or be cancelled. |  |

##### 8.1.5.2.11	Type: AssociatedDevice

Table 8.1.5.2.11-1: Definition of type AssociatedDevice

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| devInfo | string | C | 0..1 | Contains the information about the associated device. (NOTE) |  |
| devType | DeviceType | C | 0..1 | Contains the type of device (e.g. haptic device, joy stick). (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

#### 8.1.5.3	Simple data types and enumerations

##### 8.1.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 8.1.5.3.2	Simple data types

The simple data types defined in table 8.1.5.3.2-1 shall be supported.

Table 8.1.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 8.1.5.3.3	Enumeration: PermissionLevel

Table 8.1.5.3.3-1: Enumeration PermissionLevel

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| TRIAL | Level of service permission supported is TRIAL. |  |
| GOLD | Level of service permission supported is GOLD. |  |
| SILVER | Level of service permission supported is SILVER. |  |
| OTHER | Any other level of service permissions supported. |  |

##### 8.1.5.3.4	Enumeration: EASCategory

Table 8.1.5.3.4-1: Enumeration EASCategory

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UAS | Indicates that the EAS category is for UAS services. |  |
| V2X | Indicates that the EAS category is for V2X services. |  |
| APP_ENABLER | Indicates that the EAS category is for Application Enabler (e.g., SEAL) services. |  |
| OTHER | Indicates any other EAS category. |  |

##### 8.1.5.3.5	Enumeration: TransportProtocol

Table 8.1.5.3.5-1: Enumeration TransportProtocol

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| QUIC | Indicates the QUIC protocol. |  |
| TCP | Indicate the Transmission Control Protocol (TCP) protocol. |  |
| TCP_TLS | Indicates the Transmission Control Protocol (TCP) with Transport Layer Security (TLS) protocol. |  |

##### 8.1.5.3.6	Enumeration: BdlType

Table 8.1.5.3.6-1: Enumeration BdlType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DIRECT | Indicates that the Bundle type is direct bundle. The AC interacts with multiple EASs of the bundle directly (i.e., with no coordination between the EAS(s)). |  |
| PROXY | Indicates that the Bundle type is proxy bundle. The AC interacts with only one EAS of the bundle, which in turn coordinates with the other EAS(s) of the bundle to provide the services required by the AC. |  |

##### 8.1.5.3.7	Enumeration: Affinity

Table 8.1.5.3.7-1: Enumeration Affinity

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| STRONG | Indicates that the affinity is strong, i.e., all the EASs of the bundle shall be in the same EDN. |  |
| PREFERRED | Indicates that the affinity is preferred, i.e., it is preferred to have all the EASs of the bundle in the same EDN, but it is not essential. |  |
| WEAK | Indicates that the affinity is weak, i.e., it is not essential to have all the EASs of the bundle in the same EDN. |  |

##### 8.1.5.3.8	Enumeration: FailureAction

Table 8.1.5.3.8-1: Enumeration FailureAction

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| CANCEL | Indicates that ACR shall be cancelled for the other EAS(s) of the bundle for which ACR is not failed. |  |
| PROCEED | Indicates that ACR shall proceed for the other EAS(s) of the bundle for which ACR is not failed. |  |

##### 8.1.5.3.9	Enumeration: EASStatus

Table 8.1.5.3.9-1: Enumeration EASStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ENABLED | Indicates that the EAS status is enabled. |  |
| DISABLED | Indicates that the EAS status is disabled. |  |
| OVERLOAD_WARNING | Indicates that the EAS status is overload warning. |  |
| OTHER | Indicates the EAS status is another status. |  |

##### 8.1.5.3.10	Enumeration: DeviceType

Table 8.1.5.3.10-1: Enumeration DeviceType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| HAPTIC_DEVICE | Indicates that the device type is haptic device. |  |
| JOY_STICK | Indicates that the device type is joy stick. |  |
| OTHER | Indicates any other device type. |  |

### 8.1.6	Error Handling

#### 8.1.6.1	General

For the Eees_EASRegistration API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [6]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [6] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [6].

In addition, the requirements in the following clauses are applicable for the Eees_EASRegistration API.

#### 8.1.6.2	Protocol Errors

No specific protocol errors for the Eees_EASRegistration API are specified.

#### 8.1.6.3	Application Errors

The application errors defined for the Eees_EASRegistration API are listed in Table 8.1.6.3-1.

Table 8.1.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

### 8.1.7	Feature negotiation

The optional features listed in table 8.1.7-1 are defined for the Eees_EASRegistration API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [6].

Table 8.1.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | SEALDDSupport | This feature indicates the support of the SEALDD functionality related enhancements. The following functionalities are supported: - Support the EAS capability for seamless transport layer service continuity within the EAS profile. |
| 2 | EdgeApp_2 | This feature indicates the support of the first set of enhancements to the Edge Applications functionalities. The following functionalities are supported: - Support of the indication of the general context holding time. - Support of the EAS bundle functionality. - Support of the indication of the EAS ability to handle bundled EAS ACRs within the EAS profile. - Support of allowed MNO information in the EAS Profile. - Support the provisioning of the indication on EAS support for content synchronization. |
| 3 | EdgeApp_3 | This feature indicates the support of the second set of enhancements to the Edge Applications Enabler Layer. Within this feature, the following enhancements are covered: - Support multiple allowed PLMNs for an EAS. - Support the further EAS status values and a new encoding for it. |
| 4 | MetaverseApp | This feature indicates the support of the first set of enhancements to the Edge Applications functionalities to support metaverse applications. The following sub-functionalities are supported: - Support of associated devices required along with the UE. |

## 8.2	Eees_UELocation API

### 8.2.1	Introduction

The Eees_UELocation service shall use the Eees_UELocation API.

The API URI of the Eees_UELocation API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure as defined in clause 7.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in clause 7.5.

-	The <apiName> shall be "eees-uelocation".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.2.2.

### 8.2.2	Resources

#### 8.2.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 8.2.2.1-1 depicts the resource URIs structure for the Eees_UELocation API.

Figure 8.2.2.1-1: Resource URI structure of the Eees_UELocation API

Table 8.2.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.2.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Location Information Subscriptions | /subscriptions | POST | Creates a subscription for continuous reporting of UE(s) location information to the EAS. |
| Individual Location Information Subscription | /subscriptions/{subscriptionId} | GET | Retrieves the Individual location information subscription information identified by subscriptionId. |
|  |  | PATCH | Partially updates the Individual location information subscription identified by subscriptionId. |
|  |  | PUT | Fully replace the Individual location information subscription identified by subscriptionId. |
|  |  | DELETE | Removes the Individual location information subscription identified by subscriptionId. |

#### 8.2.2.2	Resource: Location Information Subscriptions

##### 8.2.2.2.1	Description

This resource represents all location information subscriptions at a given EES.

##### 8.2.2.2.2	Resource Definition

Resource URI: {apiRoot}/eees-uelocation/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in the table 8.2.2.2.2-1.

Table 8.2.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |

##### 8.2.2.2.3	Resource Standard Methods

###### 8.2.2.2.3.1	POST

This method creates the location information subscription at the EES for continuous reporting of UE(s) location information. This method shall support the URI query parameters specified in the table 8.2.2.2.3.1-1.

Table 8.2.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.2.2.2.3.1-2 and the response data structures and response codes specified in table 8.2.2.2.3.1-3.

Table 8.2.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationSubscription | M | 1 | Create a new location information subscription. |

Table 8.2.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationSubscription | M | 1 | 201 Created | The individual location information subscription resource created successfully. The information about the confirmed subscription at the EES is provided in the response body. The URI of the created resource shall be returned in the "Location" HTTP header. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. NOTE 2: Failure cases are described in clause 8.2.6.3. |  |  |  |  |

Table 8.2.2.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/eees-uelocation/<apiVersion>/subscriptions/{subscriptionId} |

##### 8.2.2.2.4	Resource Custom Operations

None.

#### 8.2.2.3	Resource: Individual Location Information Subscription

##### 8.2.2.3.1	Description

This resource represents the individual location information subscription of an EAS at a given EES.

##### 8.2.2.3.2	Resource Definition

Resource URI: {apiRoot}/eees-uelocation/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in the table 8.2.2.3.2-1.

Table 8.2.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| subscriptionId | string | Identifies an individual location information subscription. |

##### 8.2.2.3.3	Resource Standard Methods

###### 8.2.2.3.3.1	GET

This method retrieves the location information subscription information at EES. This method shall support the URI query parameters specified in the table 8.2.2.3.3.1-1.

Table 8.2.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.2.2.3.3.1-2 and the response data structures and response codes specified in table 8.2.2.3.3.1-3.

Table 8.2.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.2.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationSubscription | M | 1 | 200 OK | The location information subscription information is returned by the EES. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.2.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.2.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

###### 8.2.2.3.3.2	PATCH

This method partially updates the location information subscription information at the EES. This method shall support the URI query parameters specified in the table 8.2.2.3.3.2-1.

Table 8.2.2.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.2.2.3.3.2-2 and the response data structures and response codes specified in table 8.2.2.3.3.2-3.

Table 8.2.2.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationSubscriptionPatch | M | 1 | Request to partially update the individual location information subscription matching the subscriptionId at the EES. |

Table 8.2.2.3.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationSubscription | M | 1 | 200 OK | The individual Location information subscription matching the subscriptionId was modified successfully and the updated Location subscription information is returned in the response. |
| n/a |  |  | 204 No Content | The individual Location information subscription matching the subscriptionId was modified successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The manadatory HTTP error status code for the PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. NOTE 2: Failure cases are described in clause 8.2.6.3. |  |  |  |  |

Table 8.2.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.2.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

###### 8.2.2.3.3.3	PUT

This method updates the location information subscription information at the EES by completely replacing the existing subscription data (except easId, ueId, groupId). This method shall support the URI query parameters specified in the table 8.2.2.3.3.3-1.

Table 8.2.2.3.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.2.2.3.3.3-2 and the response data structures and response codes specified in table 8.2.2.3.3.3-3.

Table 8.2.2.3.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationSubscription | M | 1 | Details of individual location information subscription matching the subscriptionId to be updated at the EES. |

Table 8.2.2.3.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationSubscription | M | 1 | 200 OK | The individual Location information subscription matching the subscriptionId was modified successfully and the updated Location subscription information is returned in the response. |
| n/a |  |  | 204 No Content | The individual Location information subscription matching the subscriptionId was modified successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The manadatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. NOTE 2: Failure cases are described in clause 8.2.6.3. |  |  |  |  |

Table 8.2.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.2.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

###### 8.2.2.3.3.4	DELETE

This method removes the location information subscription information from the EES. This method shall support the URI query parameters specified in the table 8.2.2.3.3.4-1.

Table 8.2.2.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.2.2.3.3.4-2 and the response data structures and response codes specified in table 8.2.2.3.3.4-3.

Table 8.2.2.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.2.2.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual Location information subscription matching the subscriptionId is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status code for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.2.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.2.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

##### 8.2.2.3.4	Resource Custom Operations

None.

### 8.2.3	Custom Operations without associated resources

#### 8.2.3.1	Overview

The structure of the custom operation URIs of the Eees_UELocation API is shown in Figure 8.2.3.1-1.

Figure 8.2.3.1-1: Custom operation URI structure of the Eees_UELocation API

Custom operations used for this API are summarized in table 8.2.3.1-1.

Table 8.2.3.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Fetch | /fetch | POST | Fetch an UE location information. |

#### 8.2.3.2	Operation: Fetch

##### 8.2.3.2.1	Description

This custom operation allows the EAS to fetch an UE's location information from the EES.

##### 8.2.3.2.2	Operation Definition

This operation shall support the request data structures and response codes and data structures specified in tables 8.2.3.2.2-1 and 8.2.3.2.2-2.

Table 8.2.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationRequest | M | 1 | Parameters to request to fetch the UE location information. |

Table 8.2.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationResponse | M | 1 | 200 OK | Upon success, the UE location information returned by the EES. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The manadatory HTTP error status code for the POST method listed in the Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. NOTE 2: Failure cases are described in clause 8.2.6.3. |  |  |  |  |

Table 8.2.3.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.2.3.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

### 8.2.4	Notifications

#### 8.2.4.1	General

Table 8.2.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Location Information Notification | {notificationDestination} | POST | Notifies the subscriber EAS the UE(s) location information. |
| User Consent Revocation Notification | {revocationNotifUri} | POST | The user consent revocation notification from the EES to the EAS to inform about the revocation of user consent for one or several UE(s). |

#### 8.2.4.2	Location Information Notification

##### 8.2.4.2.1	Description

Location Information Notification is used by the EES to notify an EAS with location information of UE(s). The EES shall subscribe to the location information for UE(s) via the Individual Location Information Subscription resource.

##### 8.2.4.2.2	Target URI

The callback URI {notificationDestination} shall be used with the callback URI variables defined in table 8.2.4.2.2-1.

Table 8.2.4.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notificationDestination | Uri | Callback reference provided by the EAS during the UE location information subscription creation/update/modification procedure. |

##### 8.2.4.2.3	Standard Methods

###### 8.2.4.2.3.1	POST

This method shall support the request data structures specified in table 8.2.4.2.3.1-1 and the response data structures and response codes specified in table 8.2.4.2.3.1-2.

Table 8.2.4.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationNotification | M | 1 | Notification of UE(s) location information. |

Table 8.2.4.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative EAS where the notification should be sent. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative EAS where the notification should be sent. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.2.4.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative EAS towards which the notification should be redirected. |

Table 8.2.4.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative EAS towards which the notification should be redirected. |

#### 8.2.4.3	User Consent Revocation Notification

##### 8.2.4.3.1	Description

The User Consent Revocation Notification is used by the EES to report the revocation of user consent for one or several UE(s) to the EAS.

##### 8.2.4.3.2	Target URI

The Callback URI "{revocationNotifUri}" shall be used with the callback URI variables defined in table 8.2.4.3.2-1.

Table 8.2.4.3.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| revocationNotifUri | Uri | Callback reference provided by the EAS during the UE location information subscription creation or modification procedure if the UserConsentRevocation feature is supported, within the LocationSubscription data structure as specified in clauses 8.2.2.2.3.1, 8.2.2.3.3.3 and 8.2.5.2.2, or the LocationSubscriptionPatch data structure as specified in clauses 8.2.2.3.3.2, and 8.2.5.2.3. |

##### 8.2.4.3.3	Standard Methods

8.2.4.3.3.1	POST

This method shall support the request data structures specified in table 8.2.4.3.3.1-1 and the response data structures and response codes specified in table 8.2.4.3.3.1-2.

Table 8.2.4.3.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ConsentRevocNotif | M | 1 | Contains the user consent revocation information. |

8.2.4.3.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The user consent revocation notification is successfully received. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative EAS where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative EAS where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the HTTP POST method listed in Table 5.2.6-1 of TS 29.122 [6] also apply. |  |  |  |  |

Table 8.2.4.3.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| Location | string | 1 | An alternative URI representing the end point of an alternative EAS towards which the notification should be redirected. |

Table 8.2.4.3.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| Location | string | 1 | An alternative URI representing the end point of an alternative EAS towards which the notification should be redirected. |

### 8.2.5	Data Model

#### 8.2.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API

Table 8.2.5.1-1 specifies the data types defined specifically for the Eees_UELocation API service.

Table 8.2.5.1-1: Eees_UELocation API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ConsentRevoked | 8.2.5.2.9 | Represents the information related to revoked user consent(s). | UserConsentRevocation |
| ConsentRevocNotif | 8.2.5.2.8 | Represents the user consent revocation information conveyed in a user consent revocation notification. | UserConsentRevocation |
| LocationSubscription | 8.2.5.2.2 | Represents the location information subscription. |  |
| LocationSubscriptionPatch | 8.2.5.2.3 | Used to request the partial update of location information subscription. |  |
| LocationNotification | 8.2.5.2.4 | UE location information notification from EES to EAS. |  |
| LocationEvent | 8.2.5.2.5 | Location information related an Individual UE. |  |
| LocationRequest | 8.2.5.2.6 | UE location information request |  |
| LocationResponse | 8.2.5.2.7 | Contains the response to a UE location information retrieval request. |  |

Table 8.2.5.1-2 specifies data types re-used by the Eees_UELocation API service.

Table 8.2.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Gpsi | 3GPP TS 29.571 [8] | Used to identify the UE in the query parameter, for which location information is queried. |  |
| LocationQoS | 3GPP TS 29.572 [11] | Used to indicate the location quality of service, of the location information queried. |  |
| LocationInfo | 3GPP TS 29.122 [6] | The location information related to the UE . |  |
| DateTime | 3GPP TS 29.122 [6] | Used to capture the expiration time of EAS subscription for location information reporting. |  |
| ReportingInformation | 3GPP TS 29.523 [13] | Used to indicate the reporting requirement, only the following information are applicable: - immRep - notifMethod - maxReportNbr - monDur - repPeriod |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of optional features. |  |
| UcPurpose | 3GPP TS 29.503 [22] | Represents the purpose of a user consent. | UserConsentRevocation |
| Uri | 3GPP TS 29.122 [6] |  |  |
| WebsockNotifConfig | 3GPP TS 29.122 [6] | Following differences apply: - The SCEF is the EES; and - The SCS/AS is the subscribing EAS. |  |
| UeMobilityExposure | 3GPP TS 29.522 [10] | The predictive location information related to the UE, |  |
| GroupId | 3GPP TS 29.571 [8] | Used to present the internal group identifier in location subscription. |  |
| ExternalId | TS 29.122 [6] | Represents an external identifier. |  |
| ExternalGroupId | 3GPP TS 29.571 [8] | Used to present the external group identifier in location subscription. |  |
| Accuracy | 3GPP TS 29.122 [6] | Used by EAS to indicate the desired granularity of the requested location information. |  |

#### 8.2.5.2	Structured data types

##### 8.2.5.2.1	Introduction

##### 8.2.5.2.2	Type: LocationSubscription

Table 8.2.5.2.2-1: Definition of type LocationSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easId | string | M | 1 | The application identifier of the EAS (e.g. URI, FQDN) subscribing for location information report. |  |
| ueId | Gpsi | C | 0..1 | Identifier of the UE for which the location information reporting is subscribed for. (NOTE) |  |
| intGrpId | GroupId | C | 0..1 | The internal group identifier, identifying the group of UEs for which the location information reporting is subscribed for. (NOTE) |  |
| extGrpId | ExternalGroupId | C | 0..1 | The external group identifier, identifying the group of UEs for which the location information reporting is subscribed for. (NOTE) |  |
| expTime | DateTime | O | 0..1 | Indicates the expiration time of the subscription. If the expiration time is not present, then it indicates that the EAS subscription never expires. |  |
| locGran | Accuracy | O | 0..1 | Indicates the format of the location information that the EAS supports. EAS wishes to receive the location information report from the EES in the indicated format. |  |
| locQos | LocationQoS | O | 0..1 | Indicates the location QoS as specified in 3GPP TS 29.572 [11]. |  |
| eventReq | ReportingInformation | O | 0..1 | Represents the reporting requirements of the location information event subscription. |  |
| notificationDestination | Uri | C | 0..1 | URI where the location information notification should be delivered to. This attribute shall be present in HTTP POST message to EES and shall be present in HTTP PUT request. |  |
| revocationNotifUri | Uri | C | 0..1 | Contains the URI via which the EAS desires to receive user consent revocation notifications. If the "UserConsentRevocation" feature is supported by the EAS, this attribute shall be present in an HTTP POST request/response, and HTTP PUT request/response, and HTTP PATCH responses, and may be present in an HTTP PATCH request, if the revocation notification needs to be updated. | UserConsentRevocation |
| requestTestNotification | boolean | O | 0..1 | Set to true by Subscriber to request the EES to send a test notification as defined in clause 7.6. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in clause 7.6. | Notification_websocket |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported optional features of the API as described in clause 7.8. This attribute shall be provided in the HTTP POST request to create a new subscription resource, if at least one feature is supported, and in the HTTP POST response of successful resource creation, if it was present in the associated HTTP POST request. |  |
| NOTE: Only one of UE Identifier (ueId), Internal group identifier (intGrpId), and External group identifier (extGrpId) shall be included. |  |  |  |  |  |

NOTE:	To protect the privacy of the user, the MSISDN can be used as GPSI only after obtaining user's consent.

##### 8.2.5.2.3	Type: LocationSubscriptionPatch

Table 8.2.5.2.3-1: Definition of type LocationSubscriptionPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventReq | ReportingInformation | O | 0..1 | The reporting requirements of the location information event subscription to be updated. |  |
| expTime | DateTime | O | 0..1 | Indicates the proposed expiration time of the subscription. |  |
| notificationDestination | Uri | O | 0..1 | Updated URI where the location information notification should be delivered to. |  |
| revocationNotifUri | Uri | O | 0..1 | Contains the updated URI via which the EAS desires to receive user consent revocation notifications. This attribute may be present if the "UserConsentRevocation" feature is supported by the EAS. | UserConsentRevocation |
| locGran | Accuracy | O | 0..1 | Updated format of the location information that the EAS supports. |  |
| locQos | LocationQoS | O | 0..1 | Updated location QoS, as specified in 3GPP TS 29.572 [11]. |  |

##### 8.2.5.2.4	Type: LocationNotification

Table 8.2.5.2.4-1: Definition of type LocationNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subId | string | M | 1 | String identifying the individual Location information subscription for which the location notification is delivered. |  |
| locEvs | array(LocationEvent) | M | 1..N | List of notifications that include the location information of the UE(s). |  |

##### 8.2.5.2.5	Type: LocationEvent

Table 8.2.5.2.5-1: Definition of type LocationEvent

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueId | Gpsi | M | 1 | Identifier of the UE for which the location information is reported. |  |
| locInf | LocationInfo | C | 0..1 | Actual Location information of the UE. (NOTE) |  |
| locInfPred | UeMobilityExposure | C | 0..1 | Predictive location information of the UE. (NOTE) |  |
| NOTE: Only one of "locInf" or "locInfPred" shall be included. |  |  |  |  |  |

##### 8.2.5.2.6	Type: LocationRequest

Table 8.2.5.2.6-1: Definition of type LocationRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueId | Gpsi | M | 1 | Identifier of the UE for which the location information is requested. |  |
| gran | Accuracy | O | 0..1 | Format in which the location information is requested by the EAS. |  |
| locQos | LocationQoS | O | 0..1 | Indicates the location quality of service of the requested location information. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported optional features of the API as described in clause 7.8. This attribute shall be provided in the HTTP POST request to retrieve UE location information, if at least one feature is supported. |  |

##### 8.2.5.2.7	Type: LocationResponse

Table 8.2.5.2.7-1: Definition of type LocationResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueLocation | LocationInfo | M | 1 | Contains the requested UE location information. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported optional features of the API as described in clause 7.8. This attribute shall be provided in the HTTP POST response for UE location information retrieval, if it was present in the associated HTTP POST request. |  |

##### 8.2.5.2.8	Type: ConsentRevocNotif

Table 8.2.5.2.8-1: Definition of type ConsentRevocNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subscriptionId | string | M | 1 | Contains the identifier of the subscription to which the notification is related. |  |
| consentsRevoked | array(ConsentRevoked) | M | 1..N | Indicates the revoked user consents. |  |

##### 8.2.5.2.9	Type: ConsentRevoked

Table 8.2.5.2.9-1: Definition of type ConsentRevoked

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ucPurpose | UcPurpose | M | 1 | Identifies the purpose of the revoked user consent. |  |
| ueId | Gpsi | C | 0..1 | Contains the identifier of the UE for which user consent was revoked. (NOTE) |  |
| externalId | ExternalId | C | 0..1 | Indicates the user for which user consent was revoked. (NOTE) |  |
| NOTE: One of the "ueId" or "externalId" attributes shall be present. |  |  |  |  |  |

#### 8.2.5.3	Simple data types and enumerations

None.

### 8.2.6	Error Handling

#### 8.2.6.1	General

For the Eees_UELocation API, HTTP error handling shall be supported as specified in clause 7.7. In addition, the requirements in the following clauses are applicable for the Eees_UELocation API.

#### 8.2.6.2	Protocol Errors

No specific protocol errors for the Eees_UELocation API are specified.

#### 8.2.6.3	Application Errors

The application errors defined for the Eees_UELocation API are listed in Table 8.2.6.3-1.

Table 8.2.6.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| CONSENT_REVOCATION_NOT_SUPPORTED | 403 Forbidden | Indicates that the request is rejected because user consent management and enforcement is not supported by the client. |
| USER_CONSENT_NOT_GRANTED | 403 Forbidden | Indicates that the request is rejected because user consent is not granted. |

### 8.2.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.2.7-1 lists the supported features for Eees_UELocation API.

Table 8.2.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to clause 7.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 7.6. This feature requires that the Notification_test_event feature is also supported. |
| 3 | UserConsentRevocation | This feature indicates the support of user consent revocation management and enforcement (e.g. stop data processing). |
| 4 | enNB1 | This feature indicates the support of enhancements to this application layer API in Rel-18. |

## 8.3	Eees_UEIdentifier API

### 8.3.1	Introduction

The Eees_UEIdentifier service shall use the Eees_UEIdentifier API.

The API URI of the Eees_UEIdentifier API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure as defined in clause 7.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in clause 7.5.

-	The <apiName> shall be "eees-ueidentifier".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.3.2.

### 8.3.2	Resources

There are no resources defined for this API in this release of the specification.

### 8.3.3	Custom Operations without associated resources

#### 8.3.3.1	Overview

The structure of the custom operation URIs of the Eees_UEIdentifier API is shown in Figure 8.3.3.1-1.

Figure 8.3.3.1-1: Custom operation URI structure of the Eees_UEIdentifier API

Table 8.3.3.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the Eees_UEIdentifier API.

Table 8.3.3.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Fetch | /fetch | POST | Fetch the identifier of a UE. This custom operation is deprecated. The "Get" operation should be used instead. |
| Get | /get | POST | EAS fetch the UE Identifier information. |

NOTE 1:	Based on SA3 specified security mechanisms for EDGE-1 and EDGE-3 interfaces, the EES can identify the initiator of the API (EEC or EAS) and apply the appropriate security procedures as specified in 3GPP TS 33.558 [20].

NOTE 2:	The same service API can be implemented on two different interfaces, i.e. EDGE-1 and EDGE-3, which are for separate endpoints, i.e. the EEC and the EAS.

#### 8.3.3.2	Operation: Fetch

This operation is deprecated. The operation defined in clause 8.3.3.3 should be used instead.

##### 8.3.3.2.1	Description

This custom operation allows the EAS to fetch a UE's identifier, which is UE ID as specified in 3GPP TS 23.558 [2], from the EES for a given UE information.

##### 8.3.3.2.2	Operation Definition

This operation shall support the request data structures and the response data structure and response codes specified in the tables 8.3.3.2.2-1 and 8.3.3.2.2-2.

Table 8.3.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UserInformation | M | 1 | Information about the User or the UE, available at the EAS. |

Table 8.3.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| Gpsi | M | 1 | 200 OK | The UE Identifier (UE ID), returned by the Edge Enabler Server. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6] |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.3.3.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI located in an alternative EES. |

Table 8.3.3.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI located in an alternative EES. |

#### 8.3.3.3	Operation: Get

##### 8.3.3.3.1	Description

This custom operation allows the service consumer to retrieve UE Identifier information from the EES for a given User information as specified in 3GPP TS 23.558 [2].

##### 8.3.3.3.2	Operation Definition

This operation shall support the request data structures and the response data structure and response codes specified in the tables 8.3.3.3.2-1 and 8.3.3.3.2-2.

Table 8.3.3.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UserInfo | M | 1 | Information about the User or the UE, provided by the service consumer. |

Table 8.3.3.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UeIdInfo | M | 1 | 200 OK | The operation is successful and the corresponding UE Identifier information, returned by the Edge Enabler Server is included in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6] |
| NOTE: The mandatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.3.3.3.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI located in an alternative EES. |

Table 8.3.3.3.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI located in an alternative EES. |

### 8.3.4	Notifications

None.

### 8.3.5	Data Model

#### 8.3.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API

Table 8.3.5.1-1 specifies the data types defined specifically for the Eees_UEIdentifier API service.

Table 8.3.5.1-1: Eees_UEIdentifier API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| UserInformation | 8.3.5.2.2 | Information about the User or the UE, that used by EES to use 3GPP CN capability to retrieve the EAS specific UE identifier. Deprecated. |  |
| UserInfo | 8.3.5.2.3 | Information about the User or the UE, that used by EES to retrieve the UE identifier Information. |  |
| UeIdInfo | 8.3.5.2.4 | UE Identifier Information, including list of UE Identifier related information. |  |
| UeId | 8.3.5.2.5 | UE identifier. |  |
| UeIdType | 8.3.5.3.3 | Identifies the UE Identifier type. |  |

Table 8.3.5.1-2 specifies data types re-used by the Eees_UEIdentifier API service.

Table 8.3.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Gpsi | 3GPP TS 29.571 [8] | Used to identify the UE with GPSI. |  |
| IpAddr | 3GPP TS 29.571 [8] | IP address of the UE. |  |
| Port | 3GPP TS 29.122 [6] | Identifies a port, unsigned integer with valid values between 0 and 65535. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of optional features defined in Table 8.3.7-1. |  |

#### 8.3.5.2	Structured data types

##### 8.3.5.2.1	Introduction

##### 8.3.5.2.2	Type: UserInformation

This data type is deprecated.

Table 8.3.5.2.2-1: Definition of type UserInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easId | string | M | 1 | The application identifier of the EAS (e.g. URI, FQDN) requesting the UE Identifier information. |  |
| easProviderId | string | O | 0..1 | Identifier of the ASP that provides the EAS. |  |
| ipAddr | IpAddr | M | 1 | IP address of the UE. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported optional features of the API as described in clause 7.8. This attribute shall be provided in the HTTP POST request and success response. |  |

##### 8.3.5.2.3	Type: UserInfo

Table 8.3.5.2.3-1: Definition of type UserInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | string | C | 0..1 | Contains the identifier of the service consumer that is sending the request. (NOTE 7) | EdgeApp_3 |
| easIds | array(string) | C | 1..N | The list of EAS Identifiers for which the UE IDs are requested by the service consumer for the given user information (e.g. IP address). (NOTE 1) |  |
| easProviderId | string | O | 0..1 | Identifier of the ASP that provides the EAS. |  |
| ueId | Gpsi | C | 0..1 | Identify the UE with GPSI. (NOTE 2, NOTE 4) |  |
| ipAddr | IpAddr | C | 0..1 | IP address of the UE. (NOTE 3, NOTE 4) |  |
| portNumber | Port | C | 0..1 | Indicates the UDP or TCP port number associated with the UE IP address as provided in the "ipAddr" attribute. (NOTE 5) |  |
| appPortId | Port | O | 0..1 | Identifier of the Application Port ID associated with the EAS. (NOTE 6) |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported feature(s) among the ones defined in clause 8.3.7. This attribute shall be present only when feature negotiation needs to take place. |  |
| NOTE 1: This attribute shall be present when the service consumer is an EAS. If this attribute is not present, it shall be interpreted by the EES that it is an EEC that is requesting the UE ID. NOTE 2: This attribute may be present only if the service consumer is an EEC. NOTE 3: This attribute shall be present when the service consumer is an EAS. When the service consumer is an EEC, this attribute, if provided, may contain both UE's private IPv6 address and UE's private IPv4 address. NOTE 4: At least one of these attributes shall be present. NOTE 5: This attribute shall be present only when the service consumer is an EAS and the EAS recognizes the UE IP address is a public IP address different from the actual UE IP address (private IP address). NOTE 6: This attribute may be present only if the service consumer is an EAS. NOTE 7: This attribute shall be present when the service consumer is an EAS. |  |  |  |  |  |

##### 8.3.5.2.4	Type: UeIdInfo

Table 8.3.5.2.4-1: Definition of type UeIdInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueIds | array(UeId) | M | 1..N | Represents the UE Identifier(s). |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported feature(s) among the ones defined in clause 8.3.7. This attribute shall be present only when feature negotiation needs to take place. |  |

##### 8.3.5.2.5	Type: UeId

Table 8.3.5.2.5-1: Definition of type UeId

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| edgeUeId | string | C | 0..1 | Represents the EES generated EDGE UE Identifier of the UE. This attribute shall be provided only if it is the Edge UE ID that needs to be returned. (NOTE) |  |
| afSpecUeId | Gpsi | C | 0..1 | Identifier of the AF specific UE Identifier in the GPSI fomat of External ID. This attribute shall be provided only if it is the AF-specific UE ID that needs to be returned. (NOTE) |  |
| easId | string | C | 0..1 | Represents the identifier of the EAS for which the provided UE ID is related. This attribute shall be present only if the "easIds" attribute was provided in the corresponding request. |  |
| NOTE: These attributes are mutually exclusive. Either one of them shall be present. |  |  |  |  |  |

#### 8.3.5.3	Simple data types and enumerations

##### 8.3.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 8.3.5.3.2	Simple data types

The simple data types defined in table 8.3.5.3.2-1 shall be supported.

Table 8.3.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 8.3.6	Error Handling

#### 8.3.6.1	General

For the Eees_UEIdentifier API, HTTP error handling shall be supported as specified in clause 7.7. In addition, the requirements in the following clauses are applicable for the Eees_UEIdentifier API.

#### 8.3.6.2	Protocol Errors

No specific protocol errors for the Eees_UEIdentifier API are specified.

#### 8.3.6.3	Application Errors

The application errors defined for the Eees_UEidentifier API are listed in Table 8.3.6.3-1.

Table 8.3.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

### 8.3.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.3.7-1 lists the supported features for Eees_UEIdentifier API.

Table 8.3.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | EdgeApp_3 | This feature indicates the support of the second set of enhancements to the Edge Applications Enabler Layer. Within this feature, the following enhancements are covered: - Support of providing the requestor's identifier within the UE Identifier retrieval request. |

## 8.4	Eees_AppClientInformation API

### 8.4.1	Introduction

The Eees_AppClientInformation service shall use the Eees_AppClientInformation API.

The API URI of the Eees_AppClientInformation API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure as defined in clause 7.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in clause 7.5.

-	The <apiName> shall be "eees-appclientinformation".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.4.2.

### 8.4.2	Resources

#### 8.4.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 8.4.2.1-1 depicts the resource URIs structure for the Eees_AppClientInformation API.

Figure 8.4.2.1-1: Resource URI structure of the Eees_AppClientInformation API

Table 8.4.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.4.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Application Client Information Subscriptions | /subscriptions | POST | Creates a subscription for reporting of AC information to the EAS. |
| Individual Application Client Information Subscription | /subscriptions/{subscriptionId} | GET | Retrieves the Individual AC information subscription information identified by subscriptionId. |
|  |  | PATCH | Partially updates the Individual AC information subscription identified by subscriptionId. |
|  |  | PUT | Fully replaces the Individual AC information subscription identified by subscriptionId. |
|  |  | DELETE | Removes the Individual AC information subscription identified by subscriptionId. |

#### 8.4.2.2	Resource: Application Client Information Subscriptions

##### 8.4.2.2.1	Description

This resource represents all AC information subscriptions at a given EES.

##### 8.4.2.2.2	Resource Definition

Resource URI: {apiRoot}/eees-appclientinformation/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in the table 8.4.2.2.2-1.

Table 8.4.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |

##### 8.4.2.2.3	Resource Standard Methods

###### 8.4.2.2.3.1	POST

This method creates the AC information subscription at the EES for reporting of the AC capabilities. This method shall support the URI query parameters specified in the table 8.4.2.2.3.1-1.

Table 8.4.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.4.2.2.3.1-2 and the response data structures and response codes specified in table 8.4.2.2.3.1-3.

Table 8.4.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ACInfoSubscription | M | 1 | Create a new AC information subscription. |

Table 8.4.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ACInfoSubscription | M | 1 | 201 Created | The individual AC information subscription resource created successfully. The information about the confirmed subscription at the EES is provided in the response body. The URI of the created resource shall be retruned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.4.2.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/eees-appclientinformation/<apiVersion>/subscriptions/{subscriptionId} |

##### 8.4.2.2.4	Resource Custom Operations

None.

#### 8.4.2.3	Resource: Individual Application Client Information Subscription

##### 8.4.2.3.1	Description

This resource represents the individual application client information subscription of an EAS at a given EES.

##### 8.4.2.3.2	Resource Definition

Resource URI: {apiRoot}/eees-appclientinformation/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in the table 8.4.2.3.2-1.

Table 8.4.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| subscriptionId | string | Identifies an individual AC information subscription. |

##### 8.4.2.3.3	Resource Standard Methods

###### 8.4.2.3.3.1	GET

This method retrieves the AC information subscription information at EES. This method shall support the URI query parameters specified in the table 8.4.2.3.3.1-1.

Table 8.4.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.4.2.3.3.1-2 and the response data structures and response codes specified in table 8.4.2.3.3.1-3.

Table 8.4.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.4.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ACInfoSubscription | M | 1 | 200 OK | The AC information subscription information is returned by the EES. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.4.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.4.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

###### 8.4.2.3.3.2	PATCH

This method partially updates the individual AC information subscription information at the EES. This method shall support the URI query parameters specified in the table 8.4.2.3.3.2-1.

Table 8.4.2.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.4.2.3.3.2-2 and the response data structures and response codes specified in table 8.4.2.3.3.2-3.

Table 8.4.2.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ACInfoSubscriptionPatch | M | 1 | Request to partially update the individual AC information subscription matching the subscriptionId at the EES. |

Table 8.4.2.3.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ACInfoSubscription | M | 1 | 200 OK | The individual AC information subscription matching the subscriptionId was modified successfully and the updated AC information subscription information is returned in the response. |
| n/a |  |  | 204 No Content | The individual AC information subscription matching the subscriptionId was modified successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.4.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.4.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

###### 8.4.2.3.3.3	PUT

This method updates the AC information subscription information at the EES by completely replacing the existing subscription data (except easId). This method shall support the URI query parameters specified in the table 8.4.2.3.3.3-1.

Table 8.4.2.3.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.4.2.3.3.3-2 and the response data structures and response codes specified in table 8.4.2.3.3.3-3.

Table 8.4.2.3.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ACInfoSubscription | M | 1 | Details of individual AC information subscription matching the subscriptionId to be updated at the EES. |

Table 8.4.2.3.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ACInfoSubscription | M | 1 | 200 OK | The individual AC information subscription matching the subscriptionId was modified successfully and the updated AC subscription information is returned in the response. |
| n/a |  |  | 204 No Content | The individual AC information subscription matching the subscriptionId was modified successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.4.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.4.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

###### 8.4.2.3.3.4	DELETE

This method removes the AC information subscription information from the EES. This method shall support the URI query parameters specified in the table 8.4.2.3.3.4-1.

Table 8.4.2.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.4.2.3.3.4-2 and the response data structures and response codes specified in table 8.4.2.3.3.4-3.

Table 8.4.2.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.4.2.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual AC information subscription matching the subscriptionId is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status code for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.4.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.4.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

##### 8.4.2.3.4	Resource Custom Operations

None.

### 8.4.3	Custom Operations without associated resources

None.

### 8.4.4	Notifications

#### 8.4.4.1	General

Table 8.4.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| AC Information Notification | {notificationDestination} | POST | Notifies the subscriber EAS the AC information. |

#### 8.4.4.2	AC Information Notification

##### 8.4.4.2.1	Description

AC Information Notification is used by the EES to notify an EAS with AC information matching the filter criteria.

##### 8.4.4.2.2	Target URI

The callback URI {notificationDestination} shall be used with the callback URI variables defined in table 8.4.4.2.2-1.

Table 8.4.4.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notificationDestination | Uri | Callback reference provided by the EAS during AC information subscription creation/update/modification procedure. |

##### 8.4.4.2.3	Standard Methods

###### 8.4.4.2.3.1	POST

This method shall support the request data structures specified in table 8.4.4.2.3.1-1 and the response data structures and response codes specified in table 8.4.4.2.3.1-2.

Table 8.4.4.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ACInfoNotification | M | 1 | Notification of AC(s) information. |

Table 8.4.4.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative EAS where the notification should be sent. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative EAS where the notification should be sent. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.2.4.4.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative EAS towards which the notification should be redirected. |

Table 8.2.4.4.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative EAS towards which the notification should be redirected. |

### 8.4.5	Data Model

#### 8.4.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API

Table 8.4.5.1-1 specifies the data types defined specifically for the Eees_AppClientInformation API service.

Table 8.4.5.1-1: Eees_AppClientInformation API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ACInfoSubscription | 8.4.5.2.2 | Represents the AC information subscription. |  |
| ACInfoSubscriptionPatch | 8.4.5.2.3 | Used to request the partial update of AC information subscription. |  |
| ACFilters | 8.4.5.2.4 | Used to list the set of characteristics to discover the ACs. |  |
| ACInfoNotification | 8.4.5.2.5 | AC information notification matching the filter criteria. |  |
| ACInformation | 8.4.5.2.6 | Used to represent the AC information in the AC information notification. |  |
| EASBdlInd | 8.4.5.2.7 | Represents the EAS Bundle indication information. | EdgeApp_2 |
| TrigCondParams | 8.4.5.3.3 | Represents the notification triggering conditions. | EdgeApp_2 |

Table 8.4.5.1-2 specifies data types re-used by the Eees_AppClientInformation API service.

Table 8.4.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.122 [6] | Used to capture the expiration time of EAS subscription for application client information reporting. |  |
| ReportingInformation | 3GPP TS 29.523 [13] | Used to indicate the reporting requirement, only the following information are applicable: - immRep - notifMethod - maxReportNbr - monDur - repPeriod |  |
| Uri | 3GPP TS 29.122 [6] |  |  |
| WebsockNotifConfig | 3GPP TS 29.122 [6] | Following differences apply: - The SCEF is the EES; and - The SCS/AS is the subscribing EAS. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of optional features. |  |
| LocationArea5G | 3GPP TS 29.122 [6] | Used to define the geographic and topological area. |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [6] | Used to define the operation schedule of AC. |  |
| Gpsi | 3GPP TS 29.571 [8] | Used to identify the UE in the query parameter, for which location information is queried. |  |
| ACProfile | 3GPP TS 24.558 [14] | Used to represent the profile the information of the Application Client. |  |
| ServiceArea | Clause 9.1.5.2.5 | Used to represent the EAS service area in ACFilters data type. |  |
| ACServiceKPIs | 3GPP TS 24.558 [14] | Used to represent the minimum and maximum AC service KPI information of the Application Client. |  |
| EASBdlReqs | Clause 8.1.5.2.9 | Used to represent the EAS bundle requirements | EdgeApp_2 |
| BdlType | Clause 8.1.5.3.6 | Used to represent the EAS bundle type. | EdgeApp_2 |

#### 8.4.5.2	Structured data types

##### 8.4.5.2.1	Introduction

##### 8.4.5.2.2	Type: ACInfoSubscription

Table 8.4.5.2.2-1: Definition of type ACInfoSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easId | string | M | 1 | The application identifier of the EAS (e.g. URI, FQDN) subscribing for AC information report. |  |
| acFltrs | array(ACFilters) | O | 1..N | Filters to retrieve the information about particular ACs. |  |
| expTime | DateTime | O | 0..1 | Indicates the expiration time of the subscription. If the expiration time is not present, then it indicates that the EAS subscription never expires. |  |
| eventReq | ReportingInformation | O | 0..1 | Represents the reporting requirements of the AC information subscription. (NOTE 1) |  |
| notificationDestination | Uri | C | 0..1 | URI where the notification on information about particular ACs should be delivered to. This attribute shall be present in HTTP POST message to EES and maybe present in HTTP PUT request. |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by Subscriber to request the EES to send a test notification as defined in clause 7.6. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in clause 7.6. | Notification_websocket |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported feature(s) among the ones defined in clause 8.4.7. This attribute shall be present only when feature negotiation needs to take place. |  |
| trigCondParams | array(TrigCondParams) | O | 1..N | Contains the notification triggering conditions. (NOTE 2) | EdgeApp_2 |
| NOTE 1: The applicable values of the ReportingInformation data type are, "immRep", "notifMethod", "maxReportNbr", "monDur", "repPeriod". NOTE 2: This attribute may be present only when the "eventReq" attribute is present and the "notifMethod" attribute within the provided "eventReq" attribute is present and set to "ON_EVENT_DETECTION". When the "eventReq" attribute is present and the "notifMethod" attribute within the provided "eventReq" attribute is present and set to "ON_EVENT_DETECTION" and this attribute is either not present or present and set to "UNSPECIFIED", then the triggering conditions to be used are implementation specific. |  |  |  |  |  |

##### 8.4.5.2.3	Type: ACInfoSubscriptionPatch

Table 8.4.5.2.3-1: Definition of type ACInfoSubscriptionPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventReq | ReportingInformation | O | 0..1 | The reporting requirements of the AC information event subscription to be updated. |  |
| expTime | DateTime | O | 0..1 | Indicates the proposed expiration time of the subscription. |  |
| notificationDestination | Uri | O | 0..1 | Updated URI where the AC information notification should be delivered to. |  |
| acFltrs | array(ACFilters) | O | 1..N | Filters to retrieve the information about particular ACs. |  |
| trigCondParams | array(TrigCondParams) | O | 1..N | Contains the updated notification triggering conditions. (NOTE) | EdgeApp_2 |
| NOTE: This attribute may be present only when the "eventReq" attribute is present and the "notifMethod" attribute within the provided "eventReq" attribute is present and set to "ON_EVENT_DETECTION". When the "eventReq" attribute is present and the "notifMethod" attribute within the provided "eventReq" attribute is present and set to "ON_EVENT_DETECTION" and this attribute is either not present or present and set to "UNSPECIFIED", then the triggering conditions to be used are implementation specific. |  |  |  |  |  |

##### 8.4.5.2.4	Type: ACFilters

Table 8.4.5.2.4-1: Definition of type ACFilters

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| acTypesList | array(string) | O | 1..N | List of AC types or categories to be matched. |  |
| ecspIdsList | array(string) | O | 1..N | The list of identifiers of the ECSPs associated with the EEC. |  |
| acIdsList | array(string) | O | 1..N | List of the identifiers of the AC(s) to be matched. |  |
| svcArea | ServiceArea | O | 0..1 | EAS service area for identifying the UEs with matching expected geographical and topological location(s). |  |
| maxAcKpi | ACServiceKPIs | O | 0..1 | Maximum value of the AC service KPIs to identify the matched ACs. The service KPI values in this attribute need to be higher than the service KPIs information of the AC profiles provided during registrations and/or discovery need to be lower. |  |
| minAcKpi | ACServiceKPIs | O | 0..1 | Minimum value of the AC service KPIs to identify the matched ACs. The service KPI values in this attribute need to be lower than the service KPIs information of the AC profiles provided during registrations and/or discovery need to be lower. |  |
| opSchds | array(ScheduledCommunicationTime) | O | 1..N | The operation schedule of the EAS to be matched with operation schedule of the AC. |  |
| ueIds | array(Gpsi) | O | 1..N | List of UE identifiers hosting the AC. |  |
| locInfs | LocationArea5G | O | 0..1 | List of location(s) of the UE(s) hosting the AC. |  |
| easBundInd | EASBdlInd | O | 0..1 | Contains the EAS bundle indication related information. (NOTE) | EdgeApp_2 |
| NOTE: If the "bdlId" attribute or the "easBdlReqs" attribute within the "easBundInd" attribute are not included, then the presence of this attribute indicates that all ACs that include EAS bundle information are targeted. |  |  |  |  |  |

##### 8.4.5.2.5	Type: ACInfoNotification

Table 8.4.5.2.5-1: Definition of type ACInfoNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subId | string | M | 1 | String identifying the individual AC information subscription for which the AC information notification is delivered. |  |
| acInfs | array(ACInformation) | M | 1..N | List of notifications that include the information of the ACs matching the filter criteria. |  |

##### 8.4.5.2.6	Type: ACInformation

Table 8.4.5.2.6-1: Definition of type ACInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| acProfs | array(ACProfile) | M | 1..N | List of ACs profile information. |  |
| ueIds | array(Gpsi) | O | 1..N | List of UE identifiers hosting the ACs. |  |
| ueLocInfs | LocationArea5G | O | 0..1 | Location information of the UEs hosting the AC. |  |

##### 8.4.5.2.7	Type: EASBdlInd

Table 8.4.5.2.7-1: Definition of type EASBdlInd

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| bdlType | BdlType | C | 0..1 | Represents the EAS bundle type. (NOTE) |  |
| bdlId | string | C | 0..1 | Contains the identifier of the EAS bundle for identifying ACs with EAS bundle information in the AC Profile and with matching bundle EAS identification information. (NOTE) |  |
| easBdlReqs | EASBdlReqs | C | 0..1 | Contains the EAS Bundle Requirements for identifying ACs with EAS bundle information in the AC Profile and with matching bundle requirements. (NOTE) |  |
| parBundDet | boolean | C | 0..1 | Indicates partial bundle determination, for identifying ACs which include EAS bundle information in the AC Profile but for which only an EAS subset has been determined by EES. - When set to "true", it indicates that the partial bundle determination is requested. - When set to "false", it indicates that the partial bundle determination is not requested. - The default value when this attribute is omitted is "false". (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

#### 8.4.5.3	Simple data types and enumerations

##### 8.4.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 8.4.5.3.2	Simple data types

The simple data types defined in table 8.4.5.3.2-1 shall be supported.

Table 8.4.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 8.4.5.3.3	Enumeration: TrigCondParams

Table 8.4.5.3.3-1: Enumeration TrigCondParams

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ANALYTICS_INFO | Indicates the resource load predictive analytics trigger condition. |  |
| EEC_REGISTRATION | Indicates the EEC registration trigger condition. |  |
| EAS_DISCOVERY | Indicates the EAS discovery trigger condition. |  |
| UNSPECIFIED | Indicates that the triggering conditions are unspecified. |  |

### 8.4.6	Error Handling

#### 8.4.6.1	General

For the Eees_AppClientInformation API, HTTP error responses, protocol errors and application errors shall be supported as specified in clause 7.7.

In addition, the requirements in the following clauses are applicable for the Eees_AppClientInformation API.

#### 8.4.6.2	Protocol Errors

No specific protocol errors for the Eees_AppClientInformation API are specified.

#### 8.4.6.3	Application Errors

The application errors defined for the Eees_AppClientInformation API are listed in Table 8.4.6.3-1.

Table 8.4.6.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| REGISTRATION_REQUIRED | 403 Forbidden | Indicates that registration at the EES is required for the EAS to be authorized to perform the operation. |

### 8.4.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.4.7-1 lists the supported features for Eees_AppClientInformation API.

Table 8.4.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to clause 7.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 7.6. This feature requires that the Notification_test_event feature is also supported. |
| 3 | EdgeApp_2 | This feature indicates the support of the enhancement to the Edge Applications. Within this feature the following enhancements are covered: - Support of the provisioning of the triggering conditions parameters. - Support of provisioning the EAS bundle indication information. |

## 8.5	Eees_SessionWithQoS API

### 8.5.1	Introduction

The Eees_SessionWithQoS service shall use the Eees_SessionWithQoS API.

The API URI of the Eees_SessionWithQoS API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure as defined in clause 7.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in clause 7.5.

-	The <apiName> shall be "eees-session-with-qos".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.5.2.

### 8.5.2	Resources

#### 8.5.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 8.5.2.1-1 depicts the resource URIs structure for the Eees_SessionWithQoS API.

Figure 8.5.2.1-1: Resource URI structure of the Eees_SessionWithQoS API

Table 8.5.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.5.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Sessions with QoS | /sessions | POST | Create a new individual Session with QoS |
|  |  | GET | Read all subscription resources for given EAS. |
| Individual Session with QoS | /sessions/{sessionId} | PUT | Fully replace an existing Individual Session with QoS resource identified by a sessionId. |
|  |  | PATCH | Partially update an existing Individual Session with QoS resource identified by a sessionId |
|  |  | DELETE | Remove an Individual Session with QoS resource identified by a sessionId. |
|  |  | GET | Read a subscription resource for a sessionId. |

#### 8.5.2.2	Resource: Sessions with QoS

##### 8.5.2.2.1	Description

This resource represents session information of all the data sessions with a specific QoS setting at a given Edge Enabler Server.

##### 8.5.2.2.2	Resource Definition

Resource URI: {apiRoot}/eees-session-with-qos/<apiVersion>/sessions

This resource shall support the resource URI variables defined in the table 8.5.2.2.2-1.

Table 8.5.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |

##### 8.5.2.2.3	Resource Standard Methods

###### 8.5.2.2.3.1	POST

This method requests resources for a data session between AC and EAS with a specific QoS and may create the session information subscription at the Edge Enabler Server for receiving the user plane event notification of the session information. This method shall support the URI query parameters specified in table 8.5.2.2.3.1-1.

Table 8.5.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.5.2.2.3.1-2 and the response data structures and response codes specified in table 8.5.2.2.3.1-3.

Table 8.5.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SessionWithQoS | M | 1 | Parameters to create a subscription for a session with required QoS for the service requirement. |

Table 8.5.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SessionWithQoS | M | 1 | 201 Created | The session is successfully set up with requested QoS, and the session information is provided in the response body. The URI of the created resource shall be returned in the "Location" HTTP header. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. NOTE 2: Failure cases are described in clause 8.5.6.3. |  |  |  |  |

Table 8.5.2.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/eees-session-with-qos/<apiVersion>/sessions/{sessionId} |

###### 8.5.2.2.3.2	GET

The GET method allows to read all active subscriptions for a given EAS. The EAS shall initiate the HTTP GET request message and the EES shall respond to the message. This method shall support the URI query parameters specified in table 8.5.2.2.3.2-1.

Table 8.5.2.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| eas-id | string | M | 1 | Represents the application identifier of the EAS , e.g. URI, FQDN, that is querying the status of subscriptions. |

This method shall support the request data structures specified in table 8.5.2.2.3.2-2 and the response data structures and response codes specified in table 8.5.2.2.3.2-3.

Table 8.5.2.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.5.2.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(SessionWithQoS) | M | 1..N | 200 OK | The subscription information related to the request URI is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.5.2.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.5.2.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

##### 8.5.2.2.4	Resource Custom Operations

None.

#### 8.5.2.3	Resource: Individual Session with QoS

##### 8.5.2.3.1	Description

This resource represents an individual session information of the data session with a specific QoS setting at a given Edge Enabler Server.

##### 8.5.2.3.2	Resource Definition

Resource URI: {apiRoot}/eees-session-with-qos/<apiVersion>/sessions/{sessionId}

This resource shall support the resource URI variables defined in the table 8.5.2.3.2-1.

Table 8.5.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5. |
| sessionId | string | Contains the identifier of a Session with QoS. |

##### 8.5.2.3.3	Resource Standard Methods

###### 8.5.2.3.3.1	PATCH

This method partially updates the QoS of the data session between AC and EAS. This method shall support the URI query parameters specified in the table 8.5.2.3.3.1-1.

Table 8.5.2.3.3.1-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.5.2.3.3.1-2 and the response data structures and response codes specified in table 8.5.2.3.3.1-3.

Table 8.5.2.3.3.1-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SessionWithQoSPatch | M | 1 | Request to partially update the data session between AC and EAS with a specific QoS |

Table 8.5.2.3.3.1-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SessionWithQoS | M | 1 | 200 OK | The individual Session with QoS is successfully modified and the updated session with QoS context information is returned in the response |
| n/a |  |  | 204 No Content | The individual Session with QoS is successfully modified. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The manadatory HTTP error status code for the PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. NOTE 2: Failure cases are described in clause 8.5.6.3. |  |  |  |  |

Table 8.5.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.5.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

###### 8.5.2.3.3.2	PUT

This method requests modification of QoS of the data session between AC and EAS and may modify the subscription of the event monitoring by subscribing to new events or removing subscriptions to existing events at the Edge Enabler Server for receiving the user plane event notification of the session information. This method shall support the URI query parameters specified in the table 8.5.2.3.3.2-1.

Table 8.5.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.5.2.3.3.2-2 and the response data structures and response codes specified in table 8.5.2.3.3.2-3.

Table 8.5.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SessionWithQoS | M | 1 | Parameters to create a subscription for a session with required QoS for the service requirement. |

Table 8.5.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SessionWithQoS | M | 1 | 200 OK | The individual Session with QoS is successfully modified and the updated session with QoS context information is returned in the response. |
| n/a |  |  | 204 No Content | The individual Session with QoS is successfully modified. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The manadatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. NOTE 2: Failure cases are described in clause 8.5.6.3. |  |  |  |  |

Table 8.5.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.5.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

###### 8.5.2.3.3.3	DELETE

This method revokes the data session between AC and EAS with a specific QoS and unsubscribes to the related session with user plane event notification. This method shall support the URI query parameters specified in table 8.5.2.3.3.3-1.

Table 8.5.2.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.5.2.3.3.3-2 and the response data structures and response codes specified in table 8.5.2.3.3.3-3.

Table 8.5.2.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.5.2.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual Session with QoS resource matching the sessionId is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.5.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.5.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

###### 8.5.2.3.3.4	GET

The GET method allows to read a subscription. The EAS shall initiate the HTTP GET request message and the EES shall respond to the message. This method shall support the URI query parameters specified in table 8.5.2.3.3.4-1.

Table 8.5.2.3.3.4-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.5.2.3.3.4-2 and the response data structures and response codes specified in table 8.5.2.3.3.4-3.

Table 8.5.2.3.3.4-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.5.2.3.3.4-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SessionWithQoS | M | 1 | 200 OK | The subscription information related to the request URI is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.5.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.5.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

##### 8.5.2.3.4	Resource Custom Operations

None.

### 8.5.3	Custom Operations without associated resources

None.

### 8.5.4	Notifications

#### 8.5.4.1	General

Table 8.5.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| User Plane Event Notification | {notificationDestination} | POST | Notifies the EAS the subscribed user plane event(s). |

#### 8.5.4.2	User Plane Event Notification

##### 8.5.4.2.1	Description

##### 8.5.4.2.2	TargetURI

The callback URI {notificationDestination} shall be used with the callback URI variables defined in table 8.5.4.2.2-1.

Table 8.5.4.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notificationDestination | Uri | Callback reference provided by the EAS during session with QoS creation/update/modification procedure. |

##### 8.5.4.2.3	Standard Methods

###### 8.5.4.2.3.1	POST

This method shall support the request data structures specified in table 8.5.4.2.3.1-1 and the response data structures and response codes specified in table 8.5.4.2.3.1-2.

Table 8.5.4.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UserPlaneEventNotification | M | 1 | Notification of the user plane event on the data session. |

Table 8.5.4.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative EAS where the notification should be sent. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative EAS where the notification should be sent. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.5.4.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative EAS towards which the notification should be redirected. |

Table 8.5.4.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative EAS towards which the notification should be redirected. |

### 8.5.5	Data Model

#### 8.5.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API

Table 8.5.5.1-1 specifies the data types defined specifically for the Eees_SessionWithQoS API service.

Table 8.5.5.1-1: Eees_SessionWithQoS API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| SessionWithQoS | 8.5.5.2.2 |  |  |
| SessionWIthQoSPatch | 8.5.5.2.3 |  |  |
| UserPlaneEventNotification | 8.5.5.2.4 |  |  |

Table 8.5.5.1-2 specifies data types re-used by the Eees_SessionWithQoS API service.

Table 8.5.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Gpsi | 3GPP TS 29.571 [8] | Used to identify the UE, for which the related session with QoS is requested to. |  |
| Ipv4Addr | 3GPP TS 29.122 [6] | Identifying the IPv4 address of the UE. |  |
| Ipv6Addr | 3GPP TS 29.122 [6] | Identifying the IPv6 address of the UE. |  |
| UserPlaneEvent | 3GPP TS 29.122 [6] | Indicates the event reported by the EES. |  |
| SponsorInformation | 3GPP TS 29.122 [6] | Indicates a sponsor information |  |
| QosMonitoringInformation | 3GPP TS 29.122 [6] | Indicates the Qos Monitoring information |  |
| QosMonitoringInformationRm | 3GPP TS 29.122 [6] | Represents the same as the QosMonitoringInformation data type but with the "nullable: true" property. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of optional features. |  |
| Uri | 3GPP TS 29.122 [6] |  |  |
| WebsockNotifConfig | 3GPP TS 29.122 [6] | This type represents configuration for the delivery of notifications over Websockets. |  |
| Dnn | 3GPP TS 29.571 [8] | Identifies a DNN. |  |
| GroupId | 3GPP TS 29.571 [8] | Used to present the internal group identifier in the AS session with QoS subscription/request. |  |
| ExternalGroupId | 3GPP TS 29.571 [8] | Used to present the external group identifier in the AS session with QoS subscription/request. |  |
| Snssai | 3GPP TS 29.571 [8] | Identifies a S-NSSAI |  |
| FlowDescription | 3GPP TS 29.514 [16] | Identifies an IP flow description. |  |
| BitRate | 3GPP TS 29.571 [8] | Represents a bit rate. |  |
| BitRateRm | 3GPP TS 29.571 [8] | This data type is defined in the same way as the "BitRate" data type, but with the OpenAPI "nullable: true" property. |  |
| UserPlaneEventReport | 3GPP TS 29.122 [6] | Represents an event report for user plane. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of optional features. |  |
| TrafficFilterInfo | 8.6.5.2.12 | Represents the traffic filter information. | EdgeApp_2 |

#### 8.5.5.2	Structured data types

##### 8.5.5.2.1	Introduction

##### 8.5.5.2.2	Type: SessionWithQoS

Table 8.5.5.2.2-1: Definition of type SessionWithQoS

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| self | Uri | C | 0..1 | Link to the "Individual Session with QoS" resource. Shall only be present in the HTTP GET response on the "Sessions with QoS" resource. |  |
| easId | string | M | 1 | The application identifier of the EAS (e.g. URI, FQDN). |  |
| ueIpv4Addr | Ipv4Addr | O | 0..1 | IPv4 address of the UE. (NOTE 1) |  |
| ueIpv6Addr | Ipv6Addr | O | 0..1 | IPv6 address of the UE. (NOTE 1) |  |
| ipDomain | string | O | 0..1 | Identifies the IP domain. The attribute may only be provided if the ueIpv4Addr attribute is present. |  |
| ueId | Gpsi | O | 0..1 | Identifier of the UE for which related session with QoS is requested to. (NOTE 1) |  |
| intGrpId | GroupId | O | 0..1 | The internal group identifier, identifying the group of UEs for which related session with QoS is requested to. (NOTE 1) |  |
| extGrpId | ExternalGroupId | O | 0..1 | The external group identifier, identifying the group of UEs for which the related session with QoS is requested to. (NOTE 1) |  |
| ipFlows | array(FlowDescription) | M | 1..N | Contains the flow description for the Uplink and/or Downlink IP flows. (NOTE 3) |  |
| qosReference | string | O | 0..1 | Identifies a pre-defined QoS information (NOTE 2) |  |
| trafFilterInfo | TrafficFilterInfo | O | 0..1 | Represents the traffic filter information. This attribute may be present only if the "event" attribute is set to "UP_PATH_CHG", "ACR_MONITORING" and/or "ACR_FACILITATION". (NOTE 3) | EdgeApp_2 |
| altQosReference | array(string) | O | 1..N | Identifies an ordered list of pre-defined QoS information. The lower the index of the array for a given entry, the higher the priority. |  |
| events | array(UserPlaneEvent) | O | 1..N | Indicates the events subscribed by the EAS. |  |
| sponsorInformation | SponsorInformation | O | 0..1 | Describes the sponsor information. |  |
| qosMonInfo | QosMonitoringInformation | O | 0..1 | Qos Monitoring information. It may be present when the event "QOS_MONITORING" is subscribed. |  |
| notificationDestination | Uri | C | 0..1 | URI where the event notification shall be delivered to. This attribute shall be present if the "events" attribute is included. |  |
| dnn | Dnn | O | 0..1 | Dnn of the PDU session, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |
| snssai | Snssai | O | 0..1 | S-NSSAI of the PDU session. |  |
| maxbrUl | BitRate | O | 0..1 | Indicates the (requested) maximum bandwidth in uplink. (NOTE 2) |  |
| maxbrDl | BitRate | O | 0..1 | Indicates the (requested) maximum bandwidth in downlink. (NOTE 2) |  |
| disUeNotif | boolean | O | 0..1 | Indicates to disable QoS flow parameters signalling to the UE when the SMF is notified by the NG-RAN of changes in the fulfilled QoS situation when it is included and set to "true". The fulfilled situation is either the QoS profile or an Alternative QoS Profile. The default value "false" shall apply, if the attribute is not present and has not been supplied previously. |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by Subscriber to request the EES to send a test notification as defined in 3GPP TS 29.122 [6]. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in 3GPP TS 29.122 [6]. | Notification_websocket |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported feature(s) among the ones defined in clause 8.5.7. This attribute shall be present only when feature negotiation needs to take place. |  |
| NOTE 1: Only one of UE IP address (ipv4Addr or ipv6Addr), UE Identifier (ueId), Internal group identifier (intGrpId), or External group identifier (extGrpId) shall be included. NOTE 2: Only one of requested QoS (qosReference) or Requested bandwidth (maxbuUl and/or maxbtDl) shall be included. NOTE 3: If the "EdgeApp_2” feature is supported, the "ipFlows" attribute within the "trafFilterInfo” attribute shall take precedence over the "ipFlows" attribute when both are provided. |  |  |  |  |  |

##### 8.5.5.2.3	Type: SessionWithQoSPatch

Table 8.5.5.2.3-1: Definition of type SessionWithQoSPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ipFlows | array(FlowDescription) | O | 1..N | Contains the flow description for the Uplink and/or Downlink IP flows. (NOTE) |  |
| trafFilterInfo | TrafficFilterInfo | O | 0..1 | Represents the traffic filter information. This attribute may be present only if the "event" attribute is set to "UP_PATH_CHG","ACR_MONITORING" and/or "ACR_FACILITATION". (NOTE) | EdgeApp_2 |
| qosReference | string | O | 0..1 | Identifies a pre-defined QoS information. |  |
| altQosReference | array(string) | O | 1..N | Identifies an ordered list of pre-defined QoS information. The lower the index of the array for a given entry, the higher the priority. |  |
| events | array(UserPlaneEvent) | O | 1..N | Indicates the event subscribed by the EAS. |  |
| sponsorInformation | SponsorInformation | O | 0..1 | Describes the sponsor information such as who is sponsoring the traffic. |  |
| qosMonInfo | QosMonitoringInformationRm | O | 0..1 | Qos Monitoring information. It can be present when the event "QOS_MONITORING" is subscribed. |  |
| notificationDestination | Uri | O | 0..1 | URI where the monitoring event notification should be delivered to. |  |
| maxbrUl | BitRateRm | O | 0..1 | Indicates the (requested) maximum bandwidth in uplink. |  |
| maxbrDl | BitRateRm | O | 0..1 | Indicates the (requested) maximum bandwidth in downlink. |  |
| disUeNotif | boolean | O | 0..1 | Indicates to disable QoS flow parameters signalling to the UE when the SMF is notified by the NG-RAN of changes in the fulfilled QoS situation when it is included and set to "true". The fulfilled situation is either the QoS profile or an Alternative QoS Profile. The default value "false" shall apply, if the attribute is not present and has not been supplied previously. |  |
| NOTE: If the "EdgeApp_2” feature is supported, the "ipFlows" attribute within the "trafFilterInfo” attribute shall take precedence over the "ipFlows" attribute when both are provided. |  |  |  |  |  |

##### 8.5.5.2.4	Type: UserPlaneEventNotification

Table 8.5.5.2.4-1: Definition of type UserPlaneEventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sessionId | string | M | 1 | String identifying the individual data session information for which the QoS event notification is delivered. |  |
| eventReports | array(UserPlaneEventReport) | M | 1..N | List of user plane event reports and application information that include the QoS event information of the data session. |  |

#### 8.5.5.3	Simple data types and enumerations

None.

### 8.5.6	Error Handling

#### 8.5.6.1	General

For the Eees_SessionWithQoS API, HTTP error handling shall be supported as specified in clause 7.7. In addition, the requirements in the following clauses are applicable for the Eees_SessionWithQoS API.

#### 8.5.6.2	Protocol Errors

No specific protocol errors for the Eees_SessionWithQoS API are specified.

#### 8.5.6.3	Application Errors

The application errors defined for the Eees_SessionWithQoS API are listed in Table 8.5.6.3-1.

Table 8.5.6.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| PFD_MNGT_NOT_SUPPORTED | 403 Forbidden | Indicates that the AS Session with QoS Subscription creation/update is rejected because PFD Management is needed to fulfill the request but it is not supported by the 3GPP network. |

### 8.5.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.5.7-1 lists the supported features for Eees_SessionWithQoS API.

Table 8.5.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to clause 7.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 7.6. This feature requires that the Notification_test_event feature is also supported. |

## 8.6	Eees_ACRManagementEvent API

### 8.6.1	Introduction

The Eees_ACRManagementEvent service shall use the Eees_ACRManagementEvent API.

The API URI of the Eees_ACRManagementEvent API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure as defined in clause 7.5, i.e:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in clause 7.5.

-	The <apiName> shall be "eees-acrmgntevent".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.6.2.

### 8.6.2	Resources

#### 8.6.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 8.6.2.1-1 depicts the resource URIs structure for the Eees_ACRManagementEvent API.

Figure 8.6.2.1-1: Resource URI structure of the Eees_ACRManagementEvent API

Table 8.6.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.6.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| ACR Management Events Subscriptions | /subscriptions | GET | Query all the subscriptions. |
|  |  | POST | Create a new Individual ACR Management Events Subscription resource. |
| Individual ACR Management Events Subscription | /subscriptions/{subscriptionId} | GET | Query an existing Individual ACR Management Events Subscription resource identified by a subscriptionId. |
|  |  | PUT | Fully replace an existing Individual ACR Management Events Subscription resource identified by a subscriptionId. |
|  |  | PATCH | Partially update an existing Individual ACR Management Events Subscription resource identified by a subscriptionId. |
|  |  | DELETE | Remove an Individual ACR Management Events Subscription resource identified by a subscriptionId. |

#### 8.6.2.2	Resource: ACR Management Events Subscriptions

##### 8.6.2.2.1	Description

This resource represents ACR Management Events Subscriptions at a given Edge Enabler Server.

##### 8.6.2.2.2	Resource Definition

Resource URI: {apiRoot}/eees-acrmgntevent/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in the table 8.6.2.2.2-1.

Table 8.6.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |

##### 8.6.2.2.3	Resource Standard Methods

###### 8.6.2.2.3.1	POST

This method requests to create an Individual ACR Management Event Subscription resource at the EES for receiving the notifications of ACR management events. This method shall support the URI query parameters specified in table 8.6.2.2.3.1-1.

Table 8.6.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.6.2.2.3.1-2 and the response data structures and response codes specified in table 8.6.2.2.3.1-3.

Table 8.6.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AcrMgntEventsSubscription | M | 1 | Parameters to create a subscription for notifications of ACR management events. |

Table 8.6.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AcrMgntEventsSubscription | M | 1 | 201 Created | An Individual ACR Management Events Subscription resource is successfully created, and the subscription information is provided in the response body. The URI of the created resource shall be returned in the "Location" HTTP header. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. NOTE 2: Failure cases are described in clause 8.6.6.3. |  |  |  |  |

Table 8.6.2.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/eees-acrmgntevent/<apiVersion>/subscriptions/{subscriptionId} |

###### 8.6.2.2.3.2	GET

This method retrieves all the ACR Management Events Subscriptions information at EES. This method shall support the URI query parameters specified in the table 8.6.2.2.3.2-1.

Table 8.6.2.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supp-feat | SupportedFeatures | O | 0..1 | Contains the list of supported feature(s) among the ones defined in in clause 8.6.7. This query parameter shall be present only when feature negotiation needs to take place. |

This method shall support the request data structures specified in table 8.6.2.2.3.2-2 and the response data structures and response codes specified in table 8.6.2.2.3.2-3.

Table 8.6.2.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.6.2.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(AcrMgntEventsSubscription) | M | 1..N | 200 OK | All the ACR Management Events Subscriptions information is returned by the EES. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.6.2.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.6.2.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

##### 8.6.2.2.4	Resource Custom Operations

None.

#### 8.6.2.3	Resource: Individual ACR Management Events Subscription

##### 8.6.2.3.1	Description

This resource represents an existing Individual ACR Management Events Subscription at a given EES.

##### 8.6.2.3.2	Resource Definition

Resource URI: {apiRoot}/eees-acrmgntevent/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in the table 8.6.2.3.2-1.

Table 8.6.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5. |
| subscriptionId | string | Contains the identifier of an ACR Management Events Subscription. |

##### 8.6.2.3.3	Resource Standard Methods

###### 8.6.2.3.3.1	PATCH

This method partially updates an existing Individual ACR Management Events Subscription. This method shall support the URI query parameters specified in the table 8.6.2.3.3.1-1.

Table 8.6.2.3.3.1-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.6.2.3.3.1-2 and the response data structures and response codes specified in table 8.6.2.3.3.1-3.

Table 8.6.2.3.3.1-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AcrMgntEventsSubscriptionPatch | M | 1 | Request to partially update an existing Individual ACR Management Events Subscription. |

Table 8.6.2.3.3.1-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AcrMgntEventsSubscription | M | 1 | 200 OK | The Individual ACR Management Events Subscription is successfully modified and the updated subscription information is returned in the response. |
| n/a |  |  | 204 No Content | The Individual ACR Management Events Subscription is successfully modified. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. NOTE 2: Failure cases are described in clause 8.6.6.3. |  |  |  |  |

Table 8.6.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.6.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

###### 8.6.2.3.3.2	PUT

This method requests fully replacement of an existing Individual ACR Management Events Subscription at the EES. The request shall not change the values of the "easId", "tgtUeId", "requestTestNotification", "websockNotifConfig" and/or "suppFeat" attributes within the AcrMgntEventsSubscription data type. This method shall support the URI query parameters specified in the table 8.6.2.3.3.2-1.

Table 8.6.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.6.2.3.3.2-2 and the response data structures and response codes specified in table 8.6.2.3.3.2-3.

Table 8.6.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AcrMgntEventsSubscription | M | 1 | Parameters to replace an existing Individual ACR Management Events Subscription. |

Table 8.6.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AcrMgntEventsSubscription | M | 1 | 200 OK | The existing Individual ACR Management Events Subscription is successfully replaced and the updated subscription information is returned in the response. |
| n/a |  |  | 204 No Content | The existing Individual ACR Management Events Subscription is successfully modified. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. NOTE 2: Failure cases are described in clause 8.6.6.3 |  |  |  |  |

Table 8.6.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.6.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

###### 8.6.2.3.3.3	DELETE

This method deletes an existing Individual ACR Management Events Subscription. This method shall support the URI query parameters specified in table 8.6.2.3.3.3-1.

Table 8.6.2.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.6.2.3.3.3-2 and the response data structures and response codes specified in table 8.6.2.3.3.3-3.

Table 8.6.2.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.6.2.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The existing Individual ACR Management Events Subscription is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during subscription termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during subscription termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.6.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.6.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

###### 8.6.2.3.3.4	GET

This method retrieves the location information subscription information at EES. This method shall support the URI query parameters specified in the table 8.6.2.3.3.4-1.

Table 8.6.2.3.3.4-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supp-feat | SupportedFeatures | O | 0..1 | The features supported by the service consumer. |

This method shall support the request data structures specified in table 8.6.2.3.3.4-2 and the response data structures and response codes specified in table 8.6.2.3.3.4-3.

Table 8.6.2.3.3.4-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.6.2.3.3.4-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AcrMgntEventsSubscription | M | 1 | 200 OK | The Individual ACR Management Events Subscription is returned by the EES. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.6.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.6.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

##### 8.6.2.3.4	Resource Custom Operations

None.

### 8.6.3	Custom Operations without associated resources

None.

### 8.6.4	Notifications

#### 8.6.4.1	General

Table 8.6.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| ACR Management Events Notification | {notificationDestination} | POST | Notifies the service consumer the subscribed ACR management event(s). |
| User Plane Path Change Availability Notification | {notificationDestination}/report-availability | report-availability (POST) | Notifies the service consumer of the availability of user plane path management events monitoring via the 3GPP 5GC network (e.g. due to UE mobility from 5GC to EPC). |

#### 8.6.4.2	ACR Management Events Notification

##### 8.6.4.2.1	Description

##### 8.6.4.2.2	Notification definition

The POST method is used by the EES for the notifications of subscribed ACR management event(s) and the callback URI shall be provided by the service consumer during the creation of the Individual ACR Management Events Subscription resource.

Callback URI: {notificationDestination}

This method shall support the URI query parameters specified in table 8.6.4.2.2-1.

Table 8.6.4.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 8.6.4.2.2-2 and the response data structures and response codes specified in table 8.6.4.2.2-3.

Table 8.6.4.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AcrMgntEventsNotification | M | 1 | Notification of the ACR management events. |

Table 8.6.4.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| EasAckInformation | M | 1 | 200 OK | The receipt of the Notification is acknowledged and includes the acknowledgement information. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.6.4.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

Table 8.6.4.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

#### 8.6.4.3	User Plane Path Change Availability Notification

##### 8.6.4.3.1	Description

The User Plane Path Change Availability Notification is used by the EES to notify the service consumer of the availability of user plane path management events monitoring via the 3GPP 5GC network (e.g. due to UE mobility from 5GC to EPC).

##### 8.6.4.3.2	Target URI

The Callback URI "{notificationDestination}/report-availability" shall be used with the callback URI variables defined in table 8.6.4.3.2-1.

Table 8.6.4.3.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notificationDestination | Uri | Callback reference provided by the service consumer during the ACR Management Event subscription creation/update/modification procedure. |

##### 8.6.4.3.3	Standard Methods

8.6.4.3.3.1	POST

This method shall support the request data structures specified in table 8.6.4.3.3.1-1 and the response data structures and response codes specified in table 8.6.4.3.3.1-2.

Table 8.6.4.3.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AvailabilityNotif | M | 1 | Notification of the ACR management events. |

Table 8.6.4.3.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The notification is successfully received. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.6.4.3.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

Table 8.6.4.3.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

### 8.6.5	Data Model

#### 8.6.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API

Table 8.6.5.1-1 specifies the data types defined specifically for the Eees_ACRManagementEvent API service.

Table 8.6.5.1-1: Eees_ACRManagementEvent API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AcrMgntEventFailureCode | 8.6.5.3.6 | Represents the reason for ACR Management subscription failure for an event. |  |
| AcrMgntEventFilter | 8.6.5.3.4 | Represents the ACR Management Event filter. |  |
| AcrMgntEventsSubscription | 8.6.5.2.2 | Represents an Individual ACR Management Events Subscription. |  |
| AcrMgntEventSubsc | 8.6.5.2.3 | Represents an ACR Management Event Subscription. |  |
| AcrMgntEventsSubscriptionPatch | 8.6.5.2.4 | Represents a modification request of Individual ACR Management Events Subscription. |  |
| AcrMgntEventsNotification | 8.65.2.5 | Represents the ACR management events notification. |  |
| AcrMgntEventReport | 8.6.5.2.6 | Represents an ACR management event report. |  |
| ACRParameters | 8.6.5.2.13 | Represents ACR parameters. | EdgeApp_2 |
| ActStatus | 8.6.5.3.5 | Represents ACT status, i.e. ACT start or stop. |  |
| AvailabilityNotif | 8.6.5.2.11 | Represents the availability information of user plane path management events monitoring via the 3GPP 5GC network. |  |
| AvailabilityStatus | 8.6.5.3.7 | Represents the availability status. |  |
| EasAckInformation | 8.6.5.2.15 | Represents the acknowledgement information from EAS in response to ACR notifications. | EdgeApp_2 |
| EasInBundleInfo | 8.6.5.2.16 | Represents EAS in a bundle information. | EdgeApp_2 |
| FailureAcrMgntEventInfo | 8.6.5.2.7 | Represents a failure ACR management event. |  |
| AcrMgntEvent | 8.6.5.3.3 | Represents the ACR Management event. |  |
| IndtUeIdentification | 8.6.5.2.10 | Contains individual UE identification information. |  |
| ResultCode | 8.6.5.3.8 | Contains the EAS acknowledgement response message. | EdgeApp_2 |
| SelectedACRScenarios | 8.6.5.2.12 | Represents the selected ACR scenario(s) applicable for a given combination of AC and UE. | EdgeApp_2 |
| TargetUeIdentification | 8.6.5.2.8 | Contains target UE(s) identification information. |  |
| TrafficFilterInfo | 8.6.5.2.14 | Represents the traffic filter information. | EdgeApp_2 |
| InOutArea | 8.6.5.3.9 | Contains UE information on whether the UE moved into or out of a service area | EdgeApp_3 |
| UpPathChangeInfo | 8.6.5.2.9 | Contains user plane path change information. |  |

Table 8.6.5.1-2 specifies data types re-used by the Eees_ACRManagementEvent API service.

Table 8.6.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ACRScenario | Clause 9.1.5.3.3 | Represent the ACR scenarios. | EdgeApp_2 |
| EasCharacteristics | 3GPP TS 24.558 [14] | Represents the EAS characteristics. |  |
| DateTime | 3GPP TS 29.122 [6] | Contains a date and a time. |  |
| Dnai | 3GPP TS 29.571 [8] | Identifies a DNAI. |  |
| DnaiChangeType | 3GPP TS 29.571 [8] | Contains the type of a DNAI change. |  |
| DomainNameProtocol | 3GPP TS 29.122 [6] | Represents the additional protocol and protocol field for domain names to be matched. | EdgeApp_2 |
| EndPoint | Clause 8.1.5.2.5 | Represents the endpoint information. |  |
| ExternalGroupId | 3GPP TS 29.571 [8] | Represents an external group of UEs. |  |
| ExternalId | 3GPP TS 29.122 [6] | Represents an external identifier of a UE. |  |
| FlowDescription | 3GPP TS 29.514 [16] | Represents an IP flow description. | EdgeApp_2 |
| Gpsi | 3GPP TS 29.571 [8] | Represents a GPSI. |  |
| GroupId | 3GPP TS 29.571 [8] | Represents an internal group of UEs. |  |
| IpAddr | 3GPP TS 29.571 [8] | Represents a UE IP Address. |  |
| Ipv4Addr | 3GPP TS 29.122 [6] | Identifying the IPv4 address of the UE. |  |
| Ipv6Prefix | 3GPP TS 29.571 [8] | Identifies an IPv6 Prefix. |  |
| ReportingInformation | 3GPP TS 29.523 [13] | Represents the type of reporting the subscription requires. |  |
| RouteToLocation | 3GPP TS 29.571 [8] | Describes the traffic routes to the locations of the application. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of optional features. |  |
| ServiceArea | Clause 9.1.5.2.5 | Represents the topological and geographic areas. | EdgeApp_2 |
| Uri | 3GPP TS 29.122 [6] | Represents a URI. |  |
| WebsockNotifConfig | 3GPP TS 29.122 [6] | This type represents configuration for the delivery of notifications over Websockets. |  |

#### 8.6.5.2	Structured data types

##### 8.6.5.2.1	Introduction

##### 8.6.5.2.2	Type: AcrMgntEventsSubscription

Table 8.6.5.2.2-1: Definition of type AcrMgntEventsSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| self | Uri | C | 0..1 | Link to the "Individual ACR Management Events Subscription" resource. Shall only be present in the HTTP GET response on the "ACR Management Events Subscriptions" resource. |  |
| easId | string | M | 1 | Contains the application identifier of the EAS (e.g. URI, FQDN). |  |
| eventSubscs | array(AcrMgntEventSubsc) | M | 1..N | Indicates the subscribed ACR management events and the related information. |  |
| evtReq | ReportingInformation | O | 0..1 | Indicates the event reporting information. (NOTE 1, NOTE 2, NOTE 3) |  |
| notificationDestination | Uri | M | 1 | Contains the URI towards which the event notifications shall be delivered. |  |
| eventReports | array(AcrMgntEventReport) | C | 1..N | Represents ACR management event report(s). This attribute shall be present in an HTTP POST response if the immediate reporting indication in the "immRep" attribute within the "evtReq" attribute sets to true in the corresponding HTTP POST request, and the report(s) are available. This attribute may be present in an HTTP PUT or PATCH response when the report(s) are available. |  |
| availabilityInfo | AvailabilityNotif | C | 0..1 | Indicates the availability information of user plane path management events monitoring via the 3GPP 5GC network. This attribute may only be present in an HTTP POST/PUT/PATCH response, if the availability status information is available. |  |
| failEventReports | array(FailureAcrMgntEventInfo) | O | 1..N | Represents the event(s) for which the subscription is not successful including the failure reason(s). |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by the EAS to request the EES to send a test notification as defined in 3GPP TS 29.122 [6]. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in 3GPP TS 29.122 [6]. | Notification_websocket |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the list of Supported features. This attribute shall be present in the HTTP POST request if at least one feature is supported by the consumer. It shall be present in an HTTP POST response if it was provided in the corresponding HTTP POST request, or in the HTTP GET response if the "supp-feat" attribute query parameter is included in the corresponding HTTP GET request. |  |
| NOTE 1: The "evtReq" of an AcrMgntEventSubsc data structure within the "eventSubscs" attribute takes precedence over the "evtReq" attribute defined in this data structure when both are present. NOTE 2: When the "evtReq" is not provided in an AcrMgntEventSubsc data structure within the "eventSubscs" attribute, the "evtReq" attribute defined in this data structure applies for the corresponding ACR management event subscription. NOTE 3: This attribute is not applicable to neither the "ACT_START_STOP" event nor the "ACR_SELECTION" event when one of them or both of them is/are part of the subscribed event(s) within the "eventSubscs" attribute. |  |  |  |  |  |

##### 8.6.5.2.3	Type: AcrMgntEventSubsc

Table 8.6.5.2.3-1: Definition of type AcrMgntEventSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | AcrMgntEvent | M | 1 | Indicates the subscribed ACR management event. |  |
| eventFilter | AcrMgntEventFilter | O | 0..1 | Represents the event filter for the subscribed ACR management event. This attribute may be provided only if the "event" attribute is set to "ACR_MONITORING". |  |
| evtReq | ReportingInformation | O | 0..1 | Indicates the event reporting requirements (e.g. reporting periodicity, immediate reporting indication, etc). (NOTE 1, NOTE 2, NOTE 3) |  |
| tgtUeId | TargetUeIdentification | C | 0..1 | Contains the UE identification information. This attribute shall be provided only if the "event" attribute is set to either "UP_PATH_CHG", "OUT_OF_SERVICE_AREA" (if the "EdgeApp_3" feature is supported), "ACR_MONITORING" or "ACR_FACILITATION". |  |
| appGrpId | string | O |  | Contains the targeted application group identifier. This attribute may be present only when the "event" attribute is set to either "ACR_MONITORING" or "ACR_FACILITATION". | EdgeApp_3 |
| dnaiChgType | DnaiChangeType | O | 0..1 | Identifies a type of notification regarding UP path management event. This attribute may be provided only if the "event" attribute is set to "UP_PATH_CHG". |  |
| easAckInd | boolean | O | 0..1 | Identifies whether EAS acknowledgement of UP path change event notifications is to be expected. Set to "true" if the EAS acknowledgement is expected. Set to "false" if the EAS acknowledgement is not expected. Default value is "false". This attribute may be provided only if the "event" attribute is set to "UP_PATH_CHG". |  |
| easChars | array(EasCharacteristics) | O | 1..N | Represents a list of EAS characteristics. This attribute may be provided only if the "event" attribute is set to either "ACR_MONITORING" or "ACR_FACILITATION". |  |
| trafFilterInfo | TrafficFilterInfo | O | 0..1 | Represents the traffic filter information. This attribute may be present only if the "event" attribute is set to "UP_PATH_CHG", "ACR_MONITORING" and/or "ACR_FACILITATION". | EdgeApp_2 |
| servContPlanInd | boolean | O | 0..1 | Contains the service continuity planning indication (i.e., whether or not the EES shall monitor whether the UE(s) enter the predicted location). - When set to "true", it indicates that service continuity planning is required. - When set to "false", it indicates that service continuity planning is not required. - The default value when this attribute is omitted is false. This attribute may be present only when the "event" attribute is set to "ACR_MONITORING" or "ACR_FACILITATION". | EdgeApp_2 |
| easAckSvcCont | boolean | O | 0..1 | Indicates Whether the EAS will provide an acknowledgement as a response to the notification of ACR management notification related to service continuity planning. - When set to "true", it indicates that the EAS acknowledgement is expected. - When set to "false", it indicates the EAS acknowledgement is not expected. - The default value is "false", when this attribute is omitted. This attribute may be present only if the "event" attribute is set to "ACR_MONITORING" or "ACR_FACILITATION". | EdgeApp_2 |
| NOTE 1: The "evtReq" takes precedence over the "evtReq" attribute of the AcrMgntEventsSubscription data structure when both are present. NOTE 2: This attribute shall not be present when the "event" attribute is set to "ACT_START_STOP" or "ACR_SELECTION". NOTE 3: For the "OUT_OF_SERVICE_AREA" event, the event triggered reporting is prefered. When using the periodical reporting, care needs to be taken to avoid congestion. |  |  |  |  |  |

##### 8.6.5.2.4	Type: AcrMgntEventsSubscriptionPatch

Table 8.6.5.2.4-1: Definition of type AcrMgntEventsSubscriptionPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventSubscs | array(AcrMgntEventSubsc) | O | 1..N | Indicates the subscribed ACR management events. |  |
| evtReq | ReportingInformation | O | 0..1 | Indicates the event reporting information. (NOTE) |  |
| notificationDestination | Uri | O | 0..1 | URI where the event notification shall be delivered to. |  |
| NOTE: This attribute is not applicable to neither the "ACT_START_STOP" event nor the "ACR_SELECTION" event when one of them or both or them is/are part of the existing or newly subscribed event(s) within the "eventSubscs" attribute. |  |  |  |  |  |

##### 8.6.5.2.5	Type: AcrMgntEventsNotification

Table 8.6.5.2.5-1: Definition of type AcrMgntEventsNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subpId | string | M | 1 | String identifying the Individual ACR Management Events Subscription to which the notification is related. |  |
| eventReports | array(AcrMgntEventReport) | M | 1..N | Represents a list of ACR management event `report(s). |  |

##### 8.6.5.2.6	Type: AcrMgntEventReport

Table 8.6.5.2.6-1: Definition of type AcrMgntEventReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | AcrMgntEvent | M | 1 | Indicates the subscribed ACR management event. |  |
| timeStamp | DateTime | O | 0..1 | Represents the time stamp of the detected event. |  |
| easEndPoint | EndPoint | C | 0..1 | Represents the endpoint information of the EAS. This attribute shall be provided either: - when the "event" attribute is set to either "ACR_MONITORING" or "ACR_FACILITATION". In such case, it shall contain the endpoint information of the T-EAS; or - when the "event" attribute is set to "ACT_START_STOP". In such case, it shall contain the endpoint information of the EAS to/from which the ACT needs to be started/stopped. |  |
| actStatus | ActStatus | C | 0..1 | Represents the reported ACT status, i.e. ACT start or ACT stop. This attribute shall be provided only when the "event" attribute is set to "ACT_START_STOP". |  |
| acId | string | C | 0..1 | Represents the identifier of the concerned AC. This attribute shall be present only when the "event" attribute is set to "ACT_START_STOP", or when the "EdgeApp_2" feature is supported, "ACR_SELECTION". |  |
| ueId | TargetUeIdentification | C | 0..1 | Represents the identifier of the UE. This attribute shall be present only when the "event" attribute is set to "ACT_START_STOP" or "ACR_SELECTION". (NOTE) | EdgeApp_2 |
| selACRScen | array(SelectedACRScenarios) | C | 1..N | Represents the selected ACR scenario(s) for a given combination of AC and UE. This attribute shall be provided only when the "event" attribute is set to "ACR_SELECTION". | EdgeApp_2 |
| easInBdlInfoList | array(EasInBundleInfo) | C | 1..N | Represents the list of EAS in a bundle related information. This attribute shall be provided only when the "event" attribute is set to "ACR_SELECTION". | EdgeApp_2 |
| acrParams | ACRParameters | O | 0..1 | Represents the ACR Parameters. This attribute may be present only when the "actStatus" attribute is present and set to "ACT_START". | EdgeApp_2 |
| upPathChgInfo | UpPathChangeInfo | C | 0..1 | Represents the UP Path change information. This attribute shall be provided when the "event" attribute is set to "UP_PATH_CHG". |  |
| servContPlanInd | boolean | O | 0..1 | Contains the service continuity planning indication (i.e., whether or not the EES shall monitor whether the UE(s) enter the predicted location or not). - When set to "true", it indicates that service continuity planning shall be performed. - When set to "false", it indicates that service continuity planning shall not be performed. - The default value when this attribute is omitted is false. This attribute may be present only when the "event" attribute is set to "ACT_START_STOP". | EdgeApp_2 |
| inOutOfServArea | InOutArea | C | 0..1 | Contains the information on whether the UE moved into or out of the service area. This attribute shall be present only when the "event" attribute is set to "OUT_OF_SERVICE_AREA". | EdgeApp_3 |
| ueIds | array(TargetUeIdentification) | C | 1..N | Represents the list of UE identifiers in an application group. This attribute shall be present only when the "event" attribute is set to "ACR_FACILITATION", "UP_PATH_CHG", "ACR_MONITORING", or "OUT_OF_SERVICE_AREA". | EdgeApp_3 |
| NOTE: Only the GPSI, within the "gpsi" attribute, or the Edge UE ID, within the "edgeUeId" attribute, are applicable for this attribute as it shall convey the identifier of a single UE. |  |  |  |  |  |

##### 8.6.5.2.7	Type: FailureAcrMgntEventInfo

Table 8.6.5.2.7-1: Definition of type FailureAcrMgntEventInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | AcrMgntEvent | M | 1 | Indicates the subscribed ACR management event. |  |
| failureCode | AcrMgntEventFailureCode | M | 1 | Identifies the failure reason. |  |

##### 8.6.5.2.8	Type: TargetUeIdentification

Table 8.6.5.2.8-1: Definition of type TargetUeIdentification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsi | Gpsi | C | 0..1 | Represents external UE identifier. (NOTE) |  |
| edgeUeId | string | C | 0..1 | Represents the Edge UE ID. (NOTE) | EdgeApp_2 |
| intGrpId | GroupId | C | 0..1 | Represents a group of UEs identified by an Internal Group Identifier. (NOTE) |  |
| extGrpId | ExternalGroupId | C | 0..1 | Represents a group of UEs identified by an External Group Identifier. (NOTE) |  |
| ueIpAddr | IpAddr | C | 0..1 | Represents the UE IP address. (NOTE) |  |
| NOTE: These attributes are mutually excusive. Either one of them shall be present. |  |  |  |  |  |

##### 8.6.5.2.9	Type: UpPathChangeInfo

Table 8.6.5.2.9-1: Definition of type UpPathChangeInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueId | IndUeIdentification | M | 1 | Contains the concerned UE's identification information. |  |
| dnaiChgType | DnaiChangeType | M | 1 | Identifies the type of notification regarding UP path management event. |  |
| sourceTrafficRoute | RouteToLocation | O | 0..1 | Identifies the N6 traffic routing information associated to the source DNAI. |  |
| targetTrafficRoute | RouteToLocation | O | 0..1 | Identifies the N6 traffic routing information associated to the target DNAI. |  |
| sourceDnai | Dnai | O | 0..1 | Source DN Access Identifier. |  |
| targetDnai | Dnai | O | 0..1 | Target DN Access Identifier. |  |
| srcUeIpv4Addr | Ipv4Addr | O | 0..1 | The IPv4 Address of the served UE for the source DNAI. |  |
| srcUeIpv6Prefix | Ipv6Prefix | O | 0..1 | The Ipv6 Address Prefix of the served UE for the source DNAI. |  |
| tgtUeIpv4Addr | Ipv4Addr | O | 0..1 | The IPv4 Address of the served UE for the target DNAI. |  |
| tgtUeIpv6Prefix | Ipv6Prefix | O | 0..1 | The Ipv6 Address Prefix of the served UE for the target DNAI. |  |

##### 8.6.5.2.10	Type: IndUeIdentification

Table 8.6.5.2.10-1: Definition of type IndUeIdentification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsi | Gpsi | C | 0..1 | Represents the GPSI of the UE. |  |
| externalId | ExternalId | C | 0..1 | Represents the External Identifier of the UE. |  |
| ueIpAddr | IpAddr | C | 0..1 | Represents the UE IP address. |  |
| NOTE: Only one of the above attributes shall be present. |  |  |  |  |  |

##### 8.6.5.2.11	Type: AvailabilityNotif

Table 8.6.5.2.11-1: Definition of type AvailabilityNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| availabilityStatus | AvailabilityStatus | M | 1 | Indicates the availability information of user plane path management events monitoring via the 3GPP 5GC network. |  |

##### 8.6.5.2.12	Type: SelectedACRScenarios

Table 8.6.5.2.12-1: Definition of type SelectedACRScenarios

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| acrList | array(ACRScenario) | M | 0..N | Indicates the list of selected ACR scenario(s) for the combination of AC (identified by the "acId" attribute) and UE (identified by the "ueId" attribute. If no ACR scenario applies, an empty array is provided within this attribute. |  |
| acId | string | M | 1 | The identifier of the AC for which the selected ACR scenario(s) provided within the "acrList" attribute apply. |  |
| ueId | Gpsi | M | 1 | Represents identifier of the UE for which the selected ACR scenario(s) provided within the "acrList" attribute apply. |  |

##### 8.6.5.2.13	Type: ACRParameters

Table 8.6.5.2.13-1: Definition of type ACRParameters

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| predictExpTime | DateTime | O | 0..1 | The estimated time at which the UE may reach the Predicted/Expected UE location or EAS service area at the latest. |  |

##### 8.6.5.2.14	Type: TrafficFilterInfo

Table 8.6.5.2.14-1: Definition of type TrafficFilterInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ipFlows | array(FlowDescription) | C | 1..N | Represents IP Flows descriptions. (NOTE) |  |
| uris | array(string) | C | 1..N | Indicates URI(s) matching criteria. (NOTE) |  |
| domainNames | array(string) | C | 1..N | Indicates domain name(s) matching criteria. (NOTE) |  |
| dnProtocol | DomainNameProtocol | O | 0..1 | Indicates the additional protocol and protocol field for domain names to be matched. This attribute may only be provided when "domainNames" attribute is present. |  |
| NOTE: At least one of these attributes shall be present. If more than one of these attributes are present, the traffic flow is matched only when every provided attribute has a matching value. |  |  |  |  |  |

##### 8.6.5.2.15	Type: EasAckInformation

Table 8.6.5.2.15-1: Definition of type EasAckInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| resCode | ResultCode | M | 1 | Indicates the acceptance or rejection of the ACT. |  |

##### 8.6.5.2.16	Type: EasInBundleInfo

Table 8.6.5.2.16-1: Definition of type EasInBundleInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easId | string | M | 1 | Contains the identifier of the EAS that is within an EAS bundle. |  |
| dnais | array(Dnai) | O | 1..N | Contains the List of DNAI(s) associated with the EAS. |  |
| svcArea | ServiceArea | O | 0..1 | Contains the list of geographical and/or topological area(s) that the EAS serves. |  |

#### 8.6.5.3	Simple data types and enumerations

##### 8.6.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 8.6.5.3.2	Simple data types

The simple data types defined in table 8.6.5.3.2-1 shall be supported.

Table 8.6.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 8.6.5.3.3	Enumeration: AcrMgntEvent

Table 8.6.5.3.3-1: Enumeration AcrMgntEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UP_PATH_CHG | Indicates that the ACR Management Event is user plane path change. |  |
| ACR_MONITORING | Indicates that the ACR Management Event is ACR monitoring. |  |
| ACR_FACILITATION | Indicates that the ACR Management Event is ACR facilitation. |  |
| ACT_START_STOP | Indicates that the ACR Management Event is ACT start/stop. |  |
| ACR_SELECTION | Indicates that the ACR Management Event is ACR selection. | EdgeApp_2 |
| OUT_OF_SERVICE_AREA | Indicates that the ACR Management Event is out of service area. | EdgeApp_3 |

##### 8.6.5.3.4	Enumeration: AcrMgntEventFilter

Table 8.6.5.3.4-1: Enumeration AcrMgntEventFilter

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| INTRA_EDN_MOBILITY | Indicates that the ACR Management Event filter is intra-EDN mobility. |  |
| INTER_EDN_MOBILITY | Indicates that the ACR Management Event filter is inter-EDN mobility. |  |

##### 8.6.5.3.5	Enumeration: ActStatus

Table 8.6.5.3.5-1: Enumeration ActStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ACT_START | Indicates ACT start. |  |
| ACT_STOP | Indicates ACT stop. |  |

##### 8.6.5.3.6	Enumeration: AcrMgntEventFailureCode

Table 8.6.5.3.6-1: Enumeration AcrMgntEventFailureCode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| 3GPP_UP_PATH_CHANGE_MON_NOT_AVAILABLE | Indicates that the ACR Management Event Subscription failed because user plane path management event notifications from the 3GPP network is not available. This value is only applicable for the "UP_PATH_CHG", "ACR_MONITORING" and "ACR_FACILITATION" events. |  |
| 3GPP_SERVICE_AREA_MON_NOT_AVAILABLE | Indicates that the ACR Management Event Subscription failed because the area of interest location monitoring from the 3GPP network is not available. This value is only applicable for the "OUT_OF_SERVICE_AREA" event. |  |
| OTHER_REASONS | Indicates that the ACR Management Event Subscription failed for other reasons. This value is applicable for all events. |  |

##### 8.6.5.3.7	Enumeration: AvailabilityStatus

Table 8.6.5.3.7-1: Enumeration AvailabilityStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| AVAILABLE | Indicates availability. |  |
| NOT_AVAILABLE | Indicates unavailability. |  |

##### 8.6.5.3.8	Enumeration: ResultCode

Table 8.6.5.3.8-1: Enumeration ResultCode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ACCEPTED | Indicates acceptance of the ACT. |  |
| REJECTED | Indicates rejection of the ACT. |  |

##### 8.6.5.3.9	Enumeration: InOutArea

Table 8.6.5.3.9-1: Enumeration InOutArea

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| MOVED_INTO_AREA | Indicates that the UE moved into the service area. |  |
| MOVED_OUT_OF_AREA | Indicates that the UE moved out of the service area. |  |

### 8.6.6	Error Handling

#### 8.6.6.1	General

For the Eees_ACRManagementEvent API, HTTP error handling shall be supported as specified in clause 7.7. In addition, the requirements in the following clauses are applicable for the Eees_ACRManagementEvent API.

#### 8.6.6.2	Protocol Errors

No specific protocol errors for the Eees_ACRManagementEvent API are specified.

#### 8.6.6.3	Application Errors

The application errors defined for the Eees_ACRManagementEvent API are listed in Table 8.6.6.3-1.

Table 8.6.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| PFD_MNGT_NOT_SUPPORTED | 403 Forbidden | Indicates that the ACR Management Events Subscription creation/update is rejected because PFD Management is needed to fulfill the request but it is not supported by the 3GPP network. | EdgeApp_2 |

### 8.6.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.6.7-1 lists the supported features for Eees_ACRManagementEvent API.

Table 8.6.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to clause 7.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 7.6. This feature requires that the Notification_test_event feature is also supported. |
| 3 | EdgeApp_2 | This feature indicates the support of the enhancements to the Edge Applications. Within this feature the following enhacements are covered: - Support of the "ACR Selection" event reporting. - Support of additional ACT Start/Stop event related information. - Support of the service continuity planning support indication. - Support of the EAS acknowledgement for service continuity planning indication. - Support of EAS bundle information reporting. - Support of the Edge UE ID as a possible UE ID. |
| 4 | EdgeApp_3 | This feature indicates the support of the second set of enhancements to the Edge Applications Enabler Layer. Within this feature the following enhancements are covered: - Support of the "OUT_OF_SERVICE_AREA" event reporting. - Support of the reporting of the list of UE IDs of an application group. - Support of the common EAS relocation. |

## 8.7	Eees_EECContextRelocation API

### 8.7.1	API URI

The Eees_EECContextRelocation service shall use the Eees_EECContextRelocation API.

The request URIs used in HTTP requests shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "eees-eeccontextreloc".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.7.2.

### 8.7.1A	Usage of HTTP

The provisions of clause 5.2.2 of 3GPP TS 29.122 [6] shall apply for the Eees_EECContextRelocation API.

### 8.7.2	Resources

#### 8.7.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 8.7.2.1-1 depicts the resource URIs structure for the Eees_EECContextRelocation API.

Figure 8.7.2.1-1: Resource URI structure of the Eees_EECContextRelocation API

Table 8.7.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.7.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| EEC Contexts | /eec-contexts | GET | Pull the EEC Context information. |
|  |  | POST | Push the EEC Context information. |

#### 8.7.2.2	Resource: EEC Contexts

##### 8.7.2.2.1	Description

This resource represents the collection of EEC Contexts managed by the EES.

##### 8.7.2.2.2	Resource Definition

Resource URI: {apiRoot}/eees-eeccontextreloc/<apiVersion>/eec-contexts

This resource shall support the resource URI variables defined in the table 8.7.2.2.2-1.

Table 8.7.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |

##### 8.7.2.2.3	Resource Standard Methods

###### 8.7.2.2.3.1	GET

This method allows a service consumer to pull the EEC Context information from the EES.

This method shall support the URI query parameters specified in table 8.7.2.2.3.1-1.

Table 8.7.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ees-id | string | M | 1 | The identifier of the requesting entity (e.g., T-EES, CES). |
| eec-cntx-id | string | M | 1 | Unique identifier of the EEC Context to authorize the transfer. |
| sess-cntxs | SessionContexts | O | 0..1 | List of service session context information being requested. (NOTE) |
| NOTE: Within each array element of the "sessCntxs" attribute (encoded using the IndividualSessionContext data structure) provided within this query parameter, the "acId" and "acrList" attributes shall not be applicable. |  |  |  |  |

This method shall support the request data structures specified in table 8.7.2.2.3.1-2 and the response data structures and response codes specified in table 8.7.2.2.3.1-3.

Table 8.7.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.7.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EECContext | M | 1 | 200 OK | Successful case. The EEC context information matching the query parameters in the request is returned. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

###### 8.7.2.2.3.2	POST

This method allows a service consumer to push the EEC Context to the EES.

This method shall support the URI query parameters specified in table 8.7.2.2.3.2-1.

Table 8.7.2.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.7.2.2.3.2-2 and the response data structures and response codes specified in table 8.7.2.2.3.2-3.

Table 8.7.2.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EECContextPush | M | 1 | Represents the EEC Context information to be pushed. |

Table 8.7.2.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The EEC context information is successfully pushed to the EES, and no content is returned in the response body. |
| EECContextPushRes | M | 1 | 200 OK | Successful case. The EEC context information is successfully pushed to the EES and related information is returned in the response body. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

##### 8.7.2.2.4	Resource Custom Operations

There are no resource custom operations defined for this API in this release of the specification.

### 8.7.3	Custom Operations without associated resources

There are no custom Operations without associated resources defined for this API in this release of the specification.

### 8.7.4	Notifications

There are no notifications defined for this API in this release of the specification.

### 8.7.5	Data Model

#### 8.7.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API

Table 8.7.5.1-1 specifies the data types defined specifically for the Eees_EECContextRelocation API service.

Table 8.7.5.1-1: Eees_EECContextRelocation API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| EECContext | 8.7.5.2.5 | To represent the EEC context information to be relocated. |  |
| EECContextPush | 8.7.5.2.4 | To represent the EEC context information to be pushed. |  |
| EECContextPushRes | 8.7.5.2.6 | Represents the EEC context push relocation response. |  |
| EECSrvContinuitySupport | 8.7.5.2.8 | Represents the EEC service continuity support and the supported ACR scenarios. |  |
| ImplicitRegDetails | 8.7.5.2.7 | Represents the EEC implicit registration information. |  |
| IndividualSessionContext | 8.7.5.2.3 | To represent single Service session context information. |  |
| SessionContexts | 8.7.5.2.2 | To represent the Service session contexts information in the EEC context pull request. |  |

Table 8.7.5.1-2 specifies data types re-used by the Eees_EECContextRelocation API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Eees_EECContextRelocation API.

Table 8.7.5.1-2: Eees_EECContextRelocation API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| EndPoint | 8.1.5.2.5 | To represent the end point information of an entity. |  |
| ACProfile | 3GPP TS 24.558 [14] | Represents the application clients profiles in EEC context. |  |
| ACRScenario | 9.1.5.3.3 | Represents the ACR scenarios. | EdgeApp_2 |
| Gpsi | 3GPP TS 29.571 [8] | Represents the identifier of a UE in GPSI format. |  |
| LocationArea5G | 3GPP TS 29.122 [6] | Represents the location information of the UE in the EEC context. |  |
| DateTime | 3GPP TS 29.122 [6] | Represents a date and a time. |  |

#### 8.7.5.2	Structured data types

##### 8.7.5.2.1	Introduction

The data type for the Eees_EECContextRelocation API are defined in the clauses below.

##### 8.7.5.2.2	Type: SessionContexts

Table 8.7.5.2.2-1: Definition of type SessionContexts

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sessCntxs | array(IndividualSessionContext) | M | 1..N | Contains the list of service session context information. |  |

##### 8.7.5.2.3	Type: IndividualSessionContext

Table 8.7.5.2.3-1: Definition of type IndividualSessionContext

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easId | string | M | 1 | Contains the application identifier (e.g., URI, FQDN) of the Application Server (e.g., EAS, CAS) providing the application services. |  |
| endPt | EndPoint | M | 1 | Contains the end point information of the Application Server provided within the "easId" attribute. |  |
| acId | string | O | 0..1 | Contains the identifier of the AC for which the service session information is provided. |  |
| acrList | array(ACRScenario) | O | 1..N | Contains the list of the selected ACR scenario(s). | EdgeApp_2 |
| eecId | string | O | 0..1 | Contains the identifier of the EEC. | enNB1 |
| appGrpId | string | O | 0..1 | Contains the application group identifier. | EdgeApp_3 |

##### 8.7.5.2.4	Type: EECContextPush

Table 8.7.5.2.4-1: Definition of type EECContextPush

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eesId | string | M | 1 | Contains the identifier of the requesting entity (e.g., S-EES, CES) pushing the EEC context. |  |
| eecCntx | EECContext | M | 1 | Contains the EEC Context to be relocated. |  |
| tgtEas | EndPoint | O | 0..1 | Contains the endpoint information of the selected Application Server (e.g., T-EAS, CAS). | EdgeApp_2 |
| acrScenariosSelReq | boolean | O | 0..1 | Contains the ACR scenarios selection request. - When set to "true", this attribute indicates to the EES to select the ACR scenarios list. - When set to "false", this attribute indicates to the EES not to select the ACR scenarios list. - The default value when this attribute is omitted is "false". | EdgeApp_2 |
| eecCntxs | array(EECContext) | C | 1..N | Contains the list of the additional EEC Context(s) to be relocated. | EdgeApp_3 |

##### 8.7.5.2.5	Type: EECContext

Table 8.7.5.2.5-1: Definition of type EECContext

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eecId | string | M | 1 | Contains the unique identifier of the EEC |  |
| cntxId | string | M | 1 | Contains the unique identifier assigned to the EEC Context |  |
| ueId | Gpsi | O | 0..1 | Contains the identifier of the UE hosting the EEC. |  |
| e1Subs | array(string) | O | 1..N | Contains the list of subscription IDs for the capability exposure for the EEC ID. |  |
| ueLoc | LocationArea5G | O | 0..1 | Contains the latest available location information of the UE hosting the EEC. |  |
| acProfs | array(ACProfile) | O | 1..N | Contains the list of ACs profiles. |  |
| sessCntxs | SessionContexts | O | 0..1 | Contains the list of associated Service Session Contexts. (NOTE) |  |
| eecSrvContSupp | EECSrvContinuitySupport | O | 0..1 | Represents the service continuity support related information. | EdgeApp_2 |
| ueMobSuppInd | boolean | O | 0..1 | Contains the UE Mobility Support indication. - When set to "true", this attribute indicates that UE Mobility Support is required. - When set to "false" or omitted, this attribute indicates that UE Mobility Support is not required. - The default value when this attribute is omitted is "false". | EdgeApp_2 |
| NOTE: Within each array element of the "sessCntxs" attribute (encoded using the IndividualSessionContext data structure) provided within this attribute, the "eecId" attribute shall not be applicable. |  |  |  |  |  |

##### 8.7.5.2.6	Type: EECContextPushRes

Table 8.7.5.2.6-1: Definition of type EECContextPushRes

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| implReg | ImplicitRegDetails | C | 0..1 | Contains implicit registration details. (NOTE) |  |
| selAcrScenariosList | array(ACRScenario) | O | 1..N | Contains the list of ACR scenarios selected by the EES. | EdgeApp_2 |
| NOTE: This attribute shall be included if the EES has performed implicit registration of the EEC. |  |  |  |  |  |

##### 8.7.5.2.7	Type: ImplicitRegDetails

Table 8.7.5.2.7-1: Definition of type ImplicitRegDetails

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| regId | string | M | 1 | Represents the registration ID of the EEC whose EEC context is pushed |  |
| expTime | DateTime | O | 0..1 | Represents the expiration time of the registration. If absent, then it indicates that the registration of the EEC never expires. |  |

##### 8.7.5.2.8	Type: EECSrvContinuitySupport

Table 8.7.5.2.8-1: Definition of type EECSrvContinuitySupport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| srvContSupp | boolean | M | 1 | This attribute is to indicate EEC service continuity support. When set to "true", this attribute indicates EEC supports service continuity. When set to "false", this attribute indicates EEC does not support service continuity. |  |
| acrScenarios | array(ACRScenario) | C | 1..N | This attribute indicates the list of ACR scenarios supported by the EEC. This attribute shall be present only when the "srvContSupp" attribute is set to "true". |  |

#### 8.7.5.3	Simple data types and enumerations

##### 8.7.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 8.7.5.3.2	Simple data types

The simple data types defined in table 8.7.5.3.2-1 shall be supported.

Table 8.7.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 8.7.5.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 8.7.5.5	Binary data

##### 8.7.5.5.1	Binary Data Types

Table 8.7.5.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 8.7.6	Error Handling

#### 8.7.6.1	General

For the Eees_EECContextRelocation API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [6]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [6] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [6].

In addition, the requirements in the following clauses are applicable for the Eees_EECContextRelocation API.

#### 8.7.6.2	Protocol Errors

No specific protocol errors for the Eees_EECContextRelocation API are specified.

#### 8.7.6.3	Application Errors

The application errors defined for the Eees_EECContextRelocation API are listed in Table 8.7.6.3-1.

Table 8.7.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

### 8.7.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.7.7-1 lists the supported features for Eees_EECContextRelocation API.

Table 8.7.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | EdgeApp_2 | This feature indicates the support of the phase 2 of the definition of EDGE applications support. Within this feature, the following enhancements are covered: - Support the "ACR Selection" event subscription and reporting. |
| 2 | enNB1 | This feature indicates the support of the enhancements of this application layer API. Within this feature, the following enhancements are covered: - Support the the EEC ID within the IndividualSessionContext data type. |
| 3 | EdgeApp_3 | This feature indicates the support of the second set of enhancements to the Edge Applications Enabler Layer. Within this feature, the following enhancements are covered: - Support the application group identifier within the IndividualSessionContext data type. - Support more than one element of the EEC Context(s) to be relocated within EECContextPush data type. |

## 8.8	Eees_EELManagedACR API

### 8.8.1	Introduction

The Eees_EELManagedACR service shall use the Eees_EELManagedACR API.

The API URI of the Eees_EELManagedACR API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [6], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

-	The <apiName> shall be "eees-eel-acr".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

### 8.8.2	Usage of HTTP

The provisions of clause 5.2.2 of 3GPP TS 29.122 [6] shall apply for the Eees_EELManagedACR API.

### 8.8.3	Resources

#### 8.8.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 8.8.3.1-1 depicts the resource URIs structure for the Eees_EELManagedACR API.

Figure 8.8.3.1-1: Resource URIs structure of the Eees_EELManagedACR API

Table 8.8.3.1-1 provides an overview of the resources and applicable HTTP methods for the Eees_EELManagedACR API.

Table 8.8.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| ACT Status Subscriptions | /subscriptions | GET | Retrieve all the active ACT Status Subscription resources managed by the EES. |
|  |  | POST | Request the creation of a subscription to ACT status reporting during an EEL Managed ACR. |
| Individual ACT Status Subscription | /subscriptions/{subscriptionId} | GET | Retrieve an Individual ACT Status Subscription resource identified by the provided subscription identifier. |

#### 8.8.3.2	Resource: ACT Status Subscriptions

##### 8.8.3.2.1	Description

This resource represents the collection of ACT Status Subscriptions managed by the EES.

##### 8.8.3.2.2	Resource Definition

Resource URI: {apiRoot}/eees-eel-acr/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 8.8.3.2.2-1.

Table 8.8.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 of 3GPP TS 29.122 [6]. |

##### 8.8.3.2.3	Resource Standard Methods

The following clauses specify the standard methods supported by the resource.

###### 8.8.3.2.3.1	GET

The GET method allows a service consumer to retrieve all the active ACT Status Subscriptions managed by the EES.This method shall support the URI query parameters specified in table 8.8.3.2.3.1-1.

Table 8.8.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 8.8.3.2.3.1-2 and the response data structures and response codes specified in table 8.8.3.2.3.1-3.

Table 8.8.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.8.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(ACTStatusSubsc) | M | 0..N | 200 OK | Successful case. All the active ACT Status Subscriptions managed by the EES are returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status code for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.8.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.8.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

###### 8.8.3.2.3.2	POST

The POST method allows a service consumer to request the creation of a subscription to ACT status reporting at the EES. This method shall support the URI query parameters specified in table 8.8.3.2.3.2-1.

Table 8.8.3.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 8.8.3.2.3.2-2 and the response data structures and response codes specified in table 8.8.3.2.3.2-3.

Table 8.8.3.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ACTStatusSubsc | M | 1 | Represents the parameters to request the creation of a subscription to ACT status reporting. |

Table 8.8.3.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ACTStatusSubsc | M | 1 | 201 Created | Successful case. The subscription is successfully created and a representation of the created Individual ACT Status Subscription resource is returned. An HTTP "Location" header that contains the resource URI of the created Individual ACT Status Subscription resource shall also be included. |
| NOTE: The mandatory HTTP error status code for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.8.3.2.3.2-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/eees-eel-act/<apiVersion>/subscriptions/{subscriptionId} |

##### 8.8.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 8.8.3.3	Resource: Individual ACT Status Subscription

##### 8.8.3.3.1	Description

This resource represents an Individual ACT Status subscription managed by the EES.

##### 8.8.3.3.2	Resource Definition

Resource URI: {apiRoot}/eees-eel-acr/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 8.8.3.3.2-1.

Table 8.8.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 of 3GPP TS 29.122 [6]. |
| subscriptionId | string | Represents the subscription identifier. |

##### 8.8.3.3.3	Resource Standard Methods

The following clauses specify the standard methods supported by the resource.

###### 8.8.3.3.3.1	GET

The GET method allows a service consumer to retrieve an ACT status subscription identified by the subscription identifier included in the request URI (i.e. within the "/{subscriptionId}" path segment).This method shall support the URI query parameters specified in table 8.8.3.3.3.1-1.

Table 8.8.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 8.8.3.3.3.1-2 and the response data structures and response codes specified in table 8.8.3.3.3.1-3.

Table 8.8.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.8.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ACTStatusSubsc | M | 1 | 200 OK | Successful case. The requested Individual ACT Status Subscription resource is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.8.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

Table 8.8.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative EES. |

##### 8.8.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 8.8.4	Custom Operations without associated resources

#### 8.8.4.1	Overview

The structure of the custom operation URIs of the Eees_EELManagedACR API is shown in Figure 8.8.4.1-1.

Figure 8.8.4.1-1: Custom operation URI structure of the Eees_EELManagedACR API

Table 8.8.4.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the Eees_EELManagedACR API.

Table 8.8.4.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| RequestEELManagedACR | /request-eelacr | POST | Enables a service consumer to request the EES to handle all the operations of an ACR. |

#### 8.8.4.2	Operation: RequestEELManagedACR

##### 8.8.4.2.1	Description

The custom operation enables a service consumer to request the EES to handle all the operations of an ACR.

##### 8.8.4.2.2	Operation Definition

This operation shall support the request data structures and the response data structures and response codes specified in tables 8.8.4.2.2-1 and 8.8.4.2.2-2.

Table 8.8.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EELACRReq | M | 1 | Parameters to request the EES (i.e. S-EES) to handle all the operations of an ACR. |

Table 8.8.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EELACRResp | M | 1 | 200 OK | The requested EEL Managed ACR initiation was successfully received and processed. The response body contains the feedback of the EES. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6] |
| NOTE: The manadatory HTTP error status code for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.8.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI located in an alternative EES. |

Table 8.8.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI located in an alternative EES. |

### 8.8.5	Notifications

#### 8.8.5.1	General

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [6].

Table 8.8.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| ACT Status Notification | {notificationUri}/act-status | act-status (POST) | This service operation enables an EES to notify a previously subscribed a service consumer on ACT status information. |

#### 8.8.5.2	ACT Status Notification

##### 8.8.5.2.1	Description

The ACT Status Notification is used by an EES to notify a previously subscribed service consumer on ACT status information.

##### 8.8.5.2.2	Target URI

The Callback URI "{notificationUri}/act-status" shall be used with the callback URI variables defined in table 8.8.5.2.2-1.

Table 8.8.5.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notificationUri | Uri | String formatted as a URI containing the Callback URI. |

##### 8.8.5.2.3	Standard Methods

###### 8.8.5.2.3.1	POST

This method shall support the request data structures specified in table 8.8.5.2.3.1-1 and the response data structures and response codes specified in table 8.8.5.2.3.1-2.

Table 8.8.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ACTStatusNotif | M | 1 | Represents an ACT status notification. |

Table 8.8.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The ACT status notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.8.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

Table 8.8.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

### 8.8.6	Data Model

#### 8.8.6.1	General

This clause specifies the application data model supported by the API.

Table 8.8.6.1-1 specifies the data types defined for the Eees_EELManagedACR API.

Table 8.8.6.1-1: Eees_EELManagedACR API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| EELACRReq | 8.8.6.2.2 | Represents the parameters to request the EES (e.g. S-EES) to handle all the operations of an ACR. |  |
| EELACRResp | 8.8.6.2.3 | Represents the feedback of the EES on EEL Managed ACR request. |  |
| ACTStatusSubsc | 8.8.6.2.4 | Represents the parameters to request the creation of a subscription to ACT status reporting. |  |
| ACTStatusNotif | 8.8.6.2.5 | Represents an ACT status notification. |  |

Table 8.8.6.1-2 specifies data types re-used by the Eees_EELManagedACR API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Eees_EELManagedACR API.

Table 8.8.6.1-2: Eees_EELManagedACR API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ACTResult | Clause 8.11.6.3.3 | Represents the result of ACT. |  |
| EasCharacteristics | 3GPP TS 24.558 [14] | Represents the EAS characteristics. |  |
| Gpsi | 3GPP TS 29.571 [8] | Represents the identifier of a UE. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features. |  |
| Uri | 3GPP TS 29.122 [6] | Represents a URI. |  |

#### 8.8.6.2	Structured data types

##### 8.8.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 8.8.6.2.2	Type: EELACRReq

Table 8.8.6.2.2-1: Definition of type EELACRReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueId | Gpsi | M | 1 | Contains the UE identifier in the form of a GPSI. |  |
| easCharacs | array(EasCharacteristics) | M | 1..N | Contains a set of EAS characteristics to be used to determine the required EASs. |  |
| appCtxtStoreAddr | Uri | O | 0..1 | Contains the URI via which the Application Context can be accessed for ACT. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 8.8.8. This parameter shall be provided if at least one feature is supported by the service consumer. |  |

##### 8.8.6.2.3	Type: EELACRResp

Table 8.8.6.2.3-1: Definition of type EELACRResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appCtxtStoreAddr | Uri | C | 0..1 | Contains the URI via which the Application Context can be accessed for ACT. This attribute shall be included if it was not received from the service consumer in the related EEL Managed ACR request. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the list of negotiated supported features. This parameter shall be provided by the EES in the response to a request in which the service consumer provided the list of features that it supports. |  |

##### 8.8.6.2.4	Type: ACTStatusSubsc

Table 8.8.6.2.4-1: Definition of type ACTStatusSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easId | string | M | 1 | Contains the identifier of the service consumer (e.g., EAS, EES). |  |
| notificationUri | Uri | M | 1 | Contains the URI via which the service consumer desires to receive ACT status notifications. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 8.8.8. This parameter shall be provided if at least one feature is supported by the service consumer. |  |

##### 8.8.6.2.5	Type: ACTStatusNotif

Table 8.8.6.2.5-1: Definition of type ACTStatusNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subscriptionId | string | M | 1 | Subscription identifier. |  |
| actStatus | ACTResult | M | 1 | Contains the reported ACT status. |  |

#### 8.8.6.3	Simple data types and enumerations

##### 8.8.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 8.8.6.3.2	Simple data types

The simple data types defined in table 8.8.6.3.2-1 shall be supported.

Table 8.8.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 8.8.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 8.8.6.5	Binary data

##### 8.8.6.5.1	Binary Data Types

Table 8.8.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 8.8.7	Error Handling

#### 8.8.7.1	General

For the Eees_EELManagedACR API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [6]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [6] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [6].

In addition, the requirements in the following clauses are applicable for the Eees_EELManagedACR API.

#### 8.8.7.2	Protocol Errors

No specific protocol errors for the Eees_EELManagedACR API are specified.

#### 8.8.7.3	Application Errors

The application errors defined for the Eees_EELManagedACR API are listed in Table 8.8.7.3-1.

Table 8.8.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

### 8.8.8	Feature negotiation

The optional features in table 8.8.8-1 are defined for the Eees_EELManagedACR API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [6].

Table 8.8.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 8.9	Eees_ACRStatusUpdate API

### 8.9.1	Introduction

The Eees_ACRStatusUpdate service shall use the Eees_ACRStatusUpdate API.

The API URI of the Eees_ACRStatusUpdate API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [6], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

-	The <apiName> shall be "eees-acrstatus-update".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

### 8.9.2	Usage of HTTP

The provisions of clause 5.2.2 of 3GPP TS 29.122 [6] shall apply for the Eees_ACRStatusUpdate API.

### 8.9.3	Resources

There are no resources defined for this API in this release of the specification.

### 8.9.4	Custom Operations without associated resources

#### 8.9.4.1	Overview

The structure of the custom operation URIs of the Eees_ACRStatusUpdate API is shown in Figure 8.9.4.1-1.

Figure 8.9.4.1-1: Custom operation URI structure of the Eees_ACRStatusUpdate API

Table 8.9.4.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the Eees_ACRStatusUpdate API.

Table 8.9.4.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| RequestACRUpdate | /request-acrupdate | POST | Enables a service consumer to update the information related to ACR (e.g. indicate the status of ACT, update the notification target address) at the EES. |

#### 8.9.4.2	Operation: RequestACRUpdate

##### 8.9.4.2.1	Description

The custom operation enables a service consumer to update the information related to ACR (e.g. indicate the status of ACT, update the notification target address) at the EES.

##### 8.9.4.2.2	Operation Definition

This operation shall support the request data structures and the response data structures and response codes specified in tables 8.9.4.2.2-1 and 8.9.4.2.2-2.

Table 8.9.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ACRUpdateData | M | 1 | Parameters to update the information related to ACR (e.g. indicate the status of ACT, update the notification target address). |

Table 8.9.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ACRDataStatus | M | 1 | 200 OK | The communicated ACR update information was successfully received. The response body contains the feedback of the EES. |
| n/a |  |  | 204 No Content | The communicated ACR update information was successfully received. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6] |
| NOTE: The manadatory HTTP error status code for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 8.9.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI located in an alternative EES. |

Table 8.9.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI located in an alternative EES. |

### 8.9.5	Notifications

There are no notifications defined for this API in this release of the specification.

### 8.9.6	Data Model

#### 8.9.6.1	General

This clause specifies the application data model supported by the API.

Table 8.9.6.1-1 specifies the data types defined for the Eees_ACRStatusUpdate API.

Table 8.9.6.1-1: Eees_ACRStatusUpdate API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| ACRUpdateData | 8.9.6.2.2 | Represents the parameters to update the information related to ACR (e.g. indicate the status of ACT, update the notification target address). |  |
| ACRDataStatus | 8.9.6.2.3 | Represents the ACR status information. |  |
| ACTFailureCause | 8.9.6.3.5 | Represents the cause of ACT failure. |  |
| ACTResult | 8.9.6.3.3 | Represents the result of ACT. |  |
| ACTResultInfo | 8.9.6.2.4 | Represents the result of ACT and the related information. |  |
| AppGroupInfo | 8.9.6.2.5 | Represents the application group information. | EdgeApp_3 |
| E3SubscsStatus | 8.9.6.3.4 | Represents the status of the initialization of EDGE-3 subscriptions. |  |

Table 8.9.6.1-2 specifies data types re-used by the Eees_ACRStatusUpdate API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Eees_ACRStatusUpdate API.

Table 8.9.6.1-2: Eees_ACRStatusUpdate API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| EndPoint | Clause 8.1.5.2.5 | Represents the endpoint information. |  |
| Gpsi | 3GPP TS 29.571 [8] | Represents the identifier of a UE. |  |
| Uri | 3GPP TS 29.122 [6] | Represents a URI. |  |

#### 8.9.6.2	Structured data types

##### 8.9.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 8.9.6.2.2	Type: ACRUpdateData

Table 8.9.6.2.2-1: Definition of type ACRUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easId | string | M | 1 | Contains the application identifier of the service consumer (e.g. URI, FQDN) that is sending the ACR status update request. |  |
| acId | string | O | 0..1 | Contains the identifier of the concerned AC. |  |
| actResultInfo | ACTResultInfo | C | 0..1 | Contains the status of ACT, i.e. whether it was successful or failed, and the related information. This attribute may be included if the service consumer is the S-EAS, CAS or the T-EAS. In the case of an EEL Managed ACR, this attribute shall not be included by a T-EAS acting as the service consumer. (NOTE) |  |
| e3SubscIds | array(string) | C | 1..N | Contains a list of EDGE-3 subscription identifiers. This attribute may be included only if the service consumer sending the request is the T-EAS. (NOTE) |  |
| e3NotificationUri | Uri | C | 0..1 | Contains the updated notification URI via which the service consumer desires to receive notifications (related to EDGE-3 subscriptions) from the EES. This attribute may be included only if the service consumer sending the request is the T-EAS. (NOTE) |  |
| NOTE: At least one of the "actResultInfo", "e3SubscIds" or "e3NotificationUri" attributes shall be present. |  |  |  |  |  |

##### 8.9.6.2.3	Type: ACRDataStatus

Table 8.9.6.2.3-1: Definition of type ACRDataStatus

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| e3SubscsStatus | E3SubscsStatus | M | 1 | Contains the status of the initialization of EDGE-3 subscriptions, i.e. whether it was successful or failed. |  |
| e3SubscIds | array(string) | O | 1..N | Contains an updated list of EDGE-3 subscription identifiers. The absence of a subscription identifier implies no change for this subscription identifier. This attribute may be provided if the "e3SubscsStatus" attribute is set to "SUCCESSFUL". |  |

##### 8.9.6.2.4	Type: ACTResultInfo

Table 8.9.6.2.4-1: Definition of type ACTResultInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| actResult | ACTResult | M | 1 | Contains the status of ACT, i.e. whether it was successful or failed. |  |
| actFailureCause | ACTFailureCause | C | 0..1 | Contains the cause of ACT failure. This attribute shall be provided only if the "actResult" attribute is set to "FAILED". |  |
| appGrpInfo | AppGroupInfo | O | 0..1 | Contains the Application Group information. | EdgeApp_3 |
| ueId | Gpsi | M | 1 | Contains the identifier of the concerned UE. |  |
| easEndPoint | EndPoint | M | 1 | Contains the endpoint of the other EAS to or from which the ACT was performed. |  |

##### 8.9.6.2.5	Type: AppGroupInfo

Table 8.9.6.2.5-1: Definition of type AppGroupInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appGroupId | string | M | 1 | Contains the application group identifier. |  |
| ueIds | array(Gpsi) | M | 1..N | Contains a list of UE ID(s) in the Application Group. |  |

#### 8.9.6.3	Simple data types and enumerations

##### 8.9.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 8.9.6.3.2	Simple data types

The simple data types defined in table 8.9.6.3.2-1 shall be supported.

Table 8.9.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 8.9.6.3.3	Enumeration: ACTResult

The enumeration ACTResult represents the result of ACT. It shall comply with the provisions defined in table 8.9.6.3.3-1.

Table 8.9.6.3.3-1: Enumeration ACTResult

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SUCCESSFUL | Indicates that the ACT was successful. |  |
| FAILED | Indicates that the ACT failed. |  |

##### 8.9.6.3.4	Enumeration: E3SubscsStatus

The enumeration E3SubscsStatus represents the status of the initialization of EDGE-3 subscriptions. It shall comply with the provisions defined in table 8.9.6.3.4-1.

Table 8.9.6.3.4-1: Enumeration E3SubscsStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SUCCESSFUL | Indicates that the initialization of EDGE-3 subscriptions was successful. |  |
| FAILED | Indicates that the initialization of EDGE-3 subscriptions failed. |  |

##### 8.9.6.3.5	Enumeration: ACTFailureCause

The enumeration ACTFailureCause represents the cause of ACT failure. It shall comply with the provisions defined in table 8.9.6.3.5-1.

Table 8.9.6.3.5-1: Enumeration ACTFailureCause

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ACR_CANCELLATION | Indicates that the ACT failed due to the cancellation of the ACR. |  |
| OTHER | Indicates that the ACT failed for other reasons. |  |

#### 8.9.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 8.9.6.5	Binary data

##### 8.9.6.5.1	Binary Data Types

Table 8.9.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 8.9.7	Error Handling

#### 8.9.7.1	General

For the Eees_ACRStatusUpdate API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [6]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [6] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [6].

In addition, the requirements in the following clauses are applicable for the Eees_ACRStatusUpdate API.

#### 8.9.7.2	Protocol Errors

No specific protocol errors for the Eees_ACRStatusUpdate API are specified.

#### 8.9.7.3	Application Errors

The application errors defined for the Eees_ACRStatusUpdate API are listed in Table 8.9.7.3-1.

Table 8.9.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

### 8.9.8	Feature negotiation

The optional features in table 8.9.8-1 are defined for the Eees_ACRStatusUpdate API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [6].

Table 8.9.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | EdgeApp_3 | This feature indicates the support of the second set of enhancements to the Edge Applications Enabler Layer. Within this feature, the following enhancements are covered: - support of common EAS relocation. |

## 8.10	Eees_ACRParameterInformation API

### 8.10.1	Introduction

The Eees_ACRParameterInformation service shall use the Eees_ACRParameterInformation API.

The API URI of the Eees_ACRParameterInformation API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [6], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

-	The <apiName> shall be "eees-acr-param".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

NOTE:	When 3GPP TS 29.122 [6] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.5, the EES takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

### 8.10.2	Usage of HTTP

The provisions of clause 5.2.2 of 3GPP TS 29.122 [6] shall apply for the Eees_ACRParameterInformation API.

### 8.10.3	Resources

There are no resources defined for this API in this release of the specification.

### 8.10.4	Custom Operations without associated resources

#### 8.10.4.1	Overview

The structure of the custom operation URIs of the Eees_ACRParameterInformation API is shown in Figure 8.10.4.1-1.

Figure 8.10.4.1-1: Custom operation URI structure of the Eees_ACRParameterInformation API

Table 8.10.4.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the Eees_ACRParameterInformation API.

Table 8.10.4.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Request | /send-acrparamsinfo | POST | Enables a service consumer to send ACR parameters information to the EES. |

#### 8.10.4.2	Operation: Request

##### 8.10.4.2.1	Description

The custom operation enables a service consumer to send ACR parameters information to the EES.

##### 8.10.4.2.2	Operation Definition

This operation shall support the request data structures and the response data structures and response codes specified in tables 8.10.4.2.2-1 and 8.10.4.2.2-2.

Table 8.10.4.2.2-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ACRParamsInfo | M | 1 | Contains the ACR parameters information to be provisioned/updated. |

Table 8.10.4.2.2-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The ACR parameters information is successfully received and processed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 8.10.4.2.2-3: Headers supported by the 307 Response Code

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative EES. |

Table 8.10.4.2.2-4: Headers supported by the 308 Response Code

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative EES. |

### 8.10.5	Notifications

There are no notifications defined for this API in this release of the specification.

### 8.10.6	Data Model

#### 8.10.6.1	General

This clause specifies the application data model supported by the API.

Table 8.10.6.1-1 specifies the data types defined for the Eees_ACRParameterInformation API.

Table 8.10.6.1-1: Eees_ACRParameterInformation API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| ACRParamsInfo | 8.10.6.2.2 | Represents the ACR parameters information. |  |

Table 8.10.6.1-2 specifies data types re-used by the Eees_ACRParameterInformation API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Eees_ACRParameterInformation API.

Table 8.10.6.1-2: Eees_ACRParameterInformation API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ACRParameters | Clause 8.6.5.2.13 | Represents ACR parameters. |  |
| EndPoint | Clause 8.1.5.2.5 | Represents the endpoint information. |  |

#### 8.10.6.2	Structured data types

##### 8.10.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 8.10.6.2.2	Type: ACRParamsInfo

Table 8.10.6.2.2-1: Definition of type ACRParamsInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | string | M | 1 | Contains the identifier of the service consumer that is sending the request. |  |
| eecId | string | M | 1 | Contains the identifier of the concerned EEC. |  |
| acId | string | M | 1 | Contains the identifier of the concerned AC. |  |
| sAsEndPoint | EndPoint | M | 1 | Contains the endpoint information of the source Application Server (e.g., S-EAS, CAS). |  |
| tAsEndPoint | EndPoint | M | 1 | Contains the endpoint information of the target Application Server (e.g., T-EAS, CAS). |  |
| acrParams | ACRParameters | M | 1 | Contains the ACR Parameters. |  |

#### 8.10.6.3	Simple data types and enumerations

##### 8.10.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 8.10.6.3.2	Simple data types

The simple data types defined in table 8.10.6.3.2-1 shall be supported.

Table 8.10.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 8.10.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 8.10.6.5	Binary data

##### 8.10.6.5.1	Binary Data Types

Table 8.10.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 8.10.7	Error Handling

#### 8.10.7.1	General

For the Eees_ACRParameterInformation API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [6]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [6] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [6].

In addition, the requirements in the following clauses are applicable for the Eees_ACRParameterInformation API.

#### 8.10.7.2	Protocol Errors

No specific protocol errors for the Eees_ACRParameterInformation API are specified.

#### 8.10.7.3	Application Errors

The application errors defined for the Eees_ACRParameterInformation API are listed in Table 8.10.7.3-1.

Table 8.10.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

### 8.10.8	Feature negotiation

The optional features in table 8.10.8-1 are defined for the Eees_ACRParameterInformation API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [6].

Table 8.10.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 8.11	Eees_CommonEASAnnouncement API

### 8.11.1	Introduction

The Eees_CommonEASAnnouncement service shall use the Eees_CommonEASAnnouncement API.

The API URI of the Eees_CommonEASAnnouncement API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [6], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

-	The <apiName> shall be "eees-cea".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

NOTE:	When 3GPP TS 29.122 [6] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.5, the EES takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

### 8.11.2	Usage of HTTP

The provisions of clause 5.2.2 of 3GPP TS 29.122 [6] shall apply for the Eees_CommonEASAnnouncement API.

### 8.11.3	Resources

There are no resources defined for this API in this release of the specification.

### 8.11.4	Custom Operations without associated resources

#### 8.11.4.1	Overview

The structure of the custom operation URIs of the Eees_CommonEASAnnouncement API is shown in Figure 8.11.4.1-1.

Figure 8.11.4.1-1: Custom operation URI structure of the Eees_CommonEASAnnouncement API

Table 8.11.4.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the Eees_CommonEASAnnouncement API.

Table 8.11.4.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Declare | /declare | POST | Enables a service consumer to send common EAS information to the EES. |

#### 8.11.4.2	Operation: Declare

##### 8.11.4.2.1	Description

The custom operation enables a service consumer to send common EAS information to the EES.

##### 8.11.4.2.2	Operation Definition

This operation shall support the request data structures and the response data structures and response codes specified in tables 8.11.4.2.2-1 and 8.11.4.2.2-2.

Table 8.11.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| CommonEASInfo | M | 1 | Contains the common EAS information to be declared. |

Table 8.11.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The common EAS information is successfully received and no content is returned in the response body. |
| CommonEASInfoDecResp | M | 1 | 200 OK | Successful case. The common EAS information is successfully received and common EAS declaration related information is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative EES. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 8.11.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative EES. |

Table 8.11.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative EES. |

### 8.11.5	Notifications

There are no notifications defined for this API in this release of the specification.

### 8.11.6	Data Model

#### 8.11.6.1	General

This clause specifies the application data model supported by the API.

Table 8.11.6.1-1 specifies the data types defined for the Eees_CommonEASAnnouncement API.

Table 8.10.6.1-1: Eees_CommonEASAnnouncement API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| CommonEASInfo | 8.11.6.2.2 | Represents the common EAS information. |  |
| CommonEASInfoDecResp | 8.11.6.2.3 | Represents the response to a common EAS information declaration request. |  |
| GrpConnInfo | 8.11.6.2.4 | Represents the group connection information. |  |

Table 8.11.6.1-2 specifies data types re-used by the Eees_CommonEASAnnouncement API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Eees_CommonEASAnnouncement API.

Table 8.11.6.1-2: Eees_CommonEASAnnouncement API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| EASServiceKPI | 8.1.5.2.4 | Represents the EAS Service KPIs. | EdgeApp_3 |
| EDNInfo | 9.1.5.2.10 | Represents EDN related information. | EdgeApp_3 |
| EndPoint | 8.1.5.2.5 | Represents the end point information of an entity. |  |
| Gpsi | 3GPP TS 29.571 [8] | Represents a GPSI. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported features. |  |

#### 8.11.6.2	Structured data types

##### 8.11.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 8.11.6.2.2	Type: CommonEASInfo

Table 8.11.6.2.2-1: Definition of type CommonEASInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | string | M | 1 | Contains the identifier of the service consumer (e.g., announcing EES) that is sending the request. |  |
| requestorEndPt | EndPoint | O | 0..1 | Contains the endpoint information of the service consumer (e.g., announcing EES). |  |
| easId | string | M | 1 | Contains the identifier of the selected common EAS. |  |
| easEndPt | EndPoint | M | 1 | Contains the endpoint information of the selected common EAS. |  |
| serviceKpisList | EASServiceKPI | O | 0..1 | Contains the list of service KPI(s) of the selected common EAS. | EdgeApp_3 |
| ednInfo | EDNInfo | C | 0..1 | Contains information related to the EDN where the selected common EAS resides. When the "EdgeApp_3" feature is supported, this attribute shall be present. | EdgeApp_3 |
| appGrpId | string | M | 1 | Contains the targeted application group identifier. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 8.11.8. This attribute shall be present only when feature negotiation needs to take place. |  |

##### 8.11.6.2.3	Type: CommonEASInfoDecResp

Table 8.11.6.2.3-1: Definition of type CommonEASInfoDecResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| grpConnInfo | array(GrpConnInfo) | C | 1..N | Contains the group connection information, i.e., whether the UE(s) in the group should connect to the selected common EAS or not. (NOTE) |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 8.11.8. This attribute shall be present only when feature negotiation needs to take place. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

##### 8.11.6.2.4	Type: GrpConnInfo

Table 8.11.6.2.4-1: Definition of type GrpConnInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueId | Gpsi | M | 1 | Contains the GPSI of the UE. |  |
| connInd | boolean | M | 1 | Contains the common EAS connection indication, i.e., whether or not the UE identified by the "ueId" attribute should connect to the selected common EAS. - When set to "true", it indicates that UE should connect to the selected common EAS. - When set to "false", it indicates that UE should not connect to the selected common EAS. |  |

#### 8.11.6.3	Simple data types and enumerations

##### 8.11.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 8.11.6.3.2	Simple data types

The simple data types defined in table 8.11.6.3.2-1 shall be supported.

Table 8.11.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 8.11.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 8.11.6.5	Binary data

##### 8.11.6.5.1	Binary Data Types

Table 8.11.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 8.11.7	Error Handling

#### 8.11.7.1	General

For the Eees_CommonEASAnnouncement API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [6]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [6] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [6].

In addition, the requirements in the following clauses are applicable for the Eees_CommonEASAnnouncement API.

#### 8.11.7.2	Protocol Errors

No specific protocol errors for the Eees_CommonEASAnnouncement API are specified.

#### 8.11.7.3	Application Errors

The application errors defined for the Eees_CommonEASAnnouncement API are listed in Table 8.11.7.3-1.

Table 8.11.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

### 8.11.8	Feature negotiation

The optional features in table 8.11.8-1 are defined for the Eees_CommonEASAnnouncement API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [6].

Table 8.11.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | EdgeApp_3 | This feature indicates the support of the second set of enhancements to the Edge Applications Enabler Layer. Within this feature, the following enhancements are covered: - Support of the provisioning of the service KPIs and the EDN information of the selected common EAS in the selected common EAS announcement request. |

## 8.12	Eees_TrafficInfluenceEAS API

### 8.12.1	Introduction

The Eees_TrafficInfluenceEAS service shall use the Eees_TrafficInfluenceEAS API.

The API URI of the Eees_TrafficInfluenceEAS API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [6], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

-	The <apiName> shall be "eees-tie".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

### 8.12.2	Usage of HTTP

The provisions of clause 5.2.2 of 3GPP TS 29.122 [6] shall apply for the Eees_TrafficInfluenceEAS API.

### 8.12.3	Resources

#### 8.12.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 8.12.3.1-1 depicts the resource URIs structure for the Eees_TrafficInfluenceEAS API.

Figure 8.12.3.1-1: Resource URIs structure of the Eees_TrafficInfluenceEAS API

Table 8.12.3.1-1 provides an overview of the resources and applicable HTTP methods for the Eees_TrafficInfluenceEAS API.

Table 8.12.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Application Traffic Influence Instances | /instances | POST | Request the creation of an Application Traffic Influence Instance. |
| Individual Application Traffic Influence Instance | /instances/{instanceId} | GET | Request the retrieval of an "Individual Application Traffic Influence Instance" resource. |
|  |  | PUT | Request the update of an existing "Individual Application Traffic Influence Instance" resource. |
|  |  | PATCH | Request the modification of an existing "Individual Application Traffic Influence Instance" resource. |
|  |  | DELETE | Request the cancelation of an existing "Individual Application Traffic Influence Instance" resource. |

#### 8.12.3.2	Resource: Application Traffic Influence

##### 8.12.3.2.1	Description

This resource represents the collection of Application Traffic Influence Instances managed by the EES.

##### 8.12.3.2.2	Resource Definition

Resource URI: {apiRoot}/eees-tie/<apiVersion>/instances

This resource shall support the resource URI variables defined in table 8.12.3.2.2-1.

Table 8.12.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 of 3GPP TS 29.122 [6]. |

##### 8.12.3.2.3	Resource Standard Methods

The following clauses specify the standard methods supported by the resource.

###### 8.12.3.2.3.1	POST

The POST method allows a service consumer to request the creation of an Application Traffic Influence Instance.

This method shall support the URI query parameters specified in table 8.12.3.2.3.1-1.

Table 8.12.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 8.12.3.2.3.1-2 and the response data structures and response codes specified in table 8.12.3.2.3.1-3.

Table 8.12.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppTrafficInfluence | M | 1 | Represents the parameters to request the creation of an Application Traffic Influence Instance. |

Table 8.12.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppTrafficInfluence | M | 1 | 201 Created | Successful case. The application traffic influence is successfully created and a representation of the created "Individual Application Traffic Influence Instance" resource is returned. An HTTP "Location" header that contains the URI of the created "Individual Application Traffic Influence Instance" resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 8.12.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/eees-tie/<apiVersion>/instances/{instanceId} |

##### 8.12.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 8.12.3.3	Resource: Individual Application Traffic Influence

##### 8.12.3.3.1	Description

This resource represents the collection of Individual Application Traffic Influence Instances managed by the EES.

##### 8.12.3.3.2	Resource Definition

Resource URI: {apiRoot}/eees-tie/<apiVersion>/instances/{instanceId}

This resource shall support the resource URI variables defined in table 8.12.3.3.2-1.

Table 8.12.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 of 3GPP TS 29.122 [6]. |
| instanceId | string | Contains the identifier of an "Individual Application Traffic Influence Instance " resource. |

##### 8.12.3.3.3	Resource Standard Methods

The following clauses specify the standard methods supported by the resource.

###### 8.12.3.3.3.1	GET

This method requests retrieval of an existing Application Traffic Influence Instance.

This method shall support the URI query parameters specified in the table 8.12.3.3.3.2-1.

Table 8.12.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 8.12.3.3.3.1-2 and the response data structures and response codes specified in table 8.12.3.3.3.1-3.

Table 8.12.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.12.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppTrafficInfluence | M | 1 | 200 OK | Successful case. The "Individual Application Traffic Influence Instance" resource is successfully returned by the EES. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 8.12.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EES. |

Table 8.12.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EES. |

###### 8.12.3.3.3.2	PUT

This method requests modification of an existing Application Traffic Influence Instance.

This method shall support the URI query parameters specified in the table 8.12.3.3.3.2-1.

Table 8.12.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.12.3.3.3.2-2 and the response data structures and response codes specified in table 8.12.3.3.3.2-3.

Table 8.12.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppTrafficInfluence | M | 1 | Request to update the Application Traffic Influence Instance. |

Table 8.12.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppTrafficInfluence | M | 1 | 200 OK | Successful case. The "Individual Application Traffic Influence Instance" resource is successfully updated and the updated representation of the resource is returned in the response. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Application Traffic Influence Instance" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status codes for the HTTP PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 8.12.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EES. |

Table 8.12.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EES. |

###### 8.12.3.3.3.3	PATCH

This method modifies an existing Application Traffic Influence Instance.

This method shall support the URI query parameters specified in the table 8.12.3.3.3.3-1.

Table 8.12.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.12.3.3.3.3-2 and the response data structures and response codes specified in table 8.12.3.3.3.3-3.

Table 8.12.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppTrafficInfluencePatch | M | 1 | Request to modify the Application Traffic Influence Instance. |

Table 8.12.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppTrafficInfluence | M | 1 | 200 OK | Successful case. The "Individual Application Traffic Influence Instance" resource is successfully modified and the updated representation of the resource is returned in the response |
| n/a |  |  | 204 No Content | Successful case. The "Individual Application Traffic Influence Instance" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status codes for the PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 8.12.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EES. |

Table 8.12.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EES. |

###### 8.12.3.3.3.4	DELETE

This method cancels an existing Application Traffic Influence Instance.

This method shall support the URI query parameters specified in table 8.12.3.3.3.4-1.

Table 8.12.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.12.3.3.3.4-2 and the response data structures and response codes specified in table 8.12.3.3.3.4-3.

Table 8.12.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.12.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The "Individual Application Traffic Influence Instance" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative EES. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status codes for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 8.12.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EES. |

Table 8.12.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EES. |

##### 8.12.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 8.12.4	Notifications

There are no notifications defined for this API in this release of the specification.

### 8.12.5	Data Model

#### 8.12.5.1	General

This clause specifies the application data model supported by the API.

Table 8.12.5.1-1 specifies the data types defined for the Eees_TrafficInfluenceEAS API.

Table 8.12.5.1-1: Eees_TrafficInfluenceEAS API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AppTrafficInfluence | 8.12.5.2.2 | Represents the application traffic influence information. |  |
| AppTrafficInfluencePatch | 8.12.5.2.3 | Represents the parameters to request the modification of an application traffic influence instance. |  |

Table 8.12.5.1-2 specifies data types re-used by the Eees_TrafficInfluenceEAS API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Eees_TrafficInfluenceEAS API.

Table 8.12.5.1-2: Eees_TrafficInfluenceEAS API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported features. |  |
| TargetUeIdentification | 8.6.5.2.8 | Indicates the target UE. |  |

#### 8.12.5.2	Structured data types

##### 8.12.5.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 8.12.5.2.2	Type: AppTrafficInfluence

Table 8.12.5.2.2-1: Definition of type AppTrafficInfluence

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | string | M | 1 | Contains the identifier of the service consumer that is sending the request. |  |
| tgtUes | array(TargetUeIdentification) | C | 1..N | Indicates the target UE(s). (NOTE 1) |  |
| addTgtUes | array(TargetUeIdentification) | C | 1..N | Indicates the target UE(s) to be added. (NOTE 2) |  |
| deleTgtUes | array(TargetUeIdentification) | C | 1..N | Indicates the target UE(s) to be deleted. (NOTE 2) |  |
| anyUe | boolean | C | 0..1 | Indicates whether the request applies to any UE. - "true": Indicates that the request applies to any UE. - "false": Indicates that the request does not apply to any UE. - The default value when this attribute is omitted is "false". (NOTE 1) (NOTE 2) |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 8.12.7. This attribute shall be present only when feature negotiation needs to take place. |  |
| NOTE 1: Either the "anyUe" attribute set to the value "true" or the "tgtUes" attribute shall be present in the Application Traffic Influence creation request. The "edgeUeId" attribute within the TargetUeIdentification data type is not applicable for the "tgtUes" attribute. NOTE 2: Either the "anyUe" attribute set to the value "true" or the "addTgtUes" and/or "deleTgtUes" attributes shall be present in the Application Traffic Influence update request. The "edgeUeId" attribute within the TargetUeIdentification data type is not applicable for the "addTgtUes" and "deleTgtUes" attributes. |  |  |  |  |  |

##### 8.12.5.2.3	Type: AppTrafficInfluencePatch

Table 8.12.5.2.3-1: Definition of type AppTrafficInfluencePatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| addTgtUes | array(TargetUeIdentification) | O | 1..N | Indicates the target UE(s) to be added. (NOTE) |  |
| deleTgtUes | array(TargetUeIdentification) | O | 1..N | Indicates the target UE(s) to be deleted. (NOTE) |  |
| anyUe | boolean | O | 0..1 | Indicates whether the request applies to any UE. - "true": Indicates that the request applies to any UE. - "false": Indicates that the request does not apply to any UE. (NOTE) |  |
| NOTE: Either the "anyUe" attribute set to the value "true" or the "addTgtUes" and/or "deleTgtUes" attributes may be present. If the "anyUe" attribute is present and set to the "true" and the existing resource representation contains the "tgtUes" attribute, then the "tgtUes" attribute shall be deleted from the resource representation. If the "addTgtUes" and/or "deleTgtUes" attributes is present and the existing resource representation contains the "anyUe" attribute set to "true", the "anyUe" attribute shall either be deleted from the resource representation or its value changed to "false". |  |  |  |  |  |

#### 8.12.5.3	Simple data types and enumerations

##### 8.12.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 8.12.5.3.2	Simple data types

The simple data types defined in table 8.12.5.3.2-1 shall be supported.

Table 8.12.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 8.12.5.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 8.12.5.5	Binary data

##### 8.12.5.5.1	Binary Data Types

Table 8.12.5.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 8.12.6	Error Handling

#### 8.12.6.1	General

For the Eees_TrafficInfluenceEAS API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [6]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [6] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [6].

In addition, the requirements in the following clauses are applicable for the Eees_TrafficInfluenceEAS API.

#### 8.12.6.2	Protocol Errors

No specific protocol errors for the Eees_TrafficInfluenceEAS API are specified.

#### 8.12.6.3	Application Errors

The application errors defined for the Eees_TrafficInfluenceEAS API are listed in Table 8.12.6.3-1.

Table 8.12.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

### 8.12.7	Feature negotiation

The optional features in table 8.10.7-1 are defined for the Eees_TrafficInfluenceEAS API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [6].

Table 8.12.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

# 8A	CAS API Definitions

## 8A.1	Ecas_SelectedEES API

### 8A.1.1	Introduction

The Ecas_SelectedEES service shall use the Ecas_SelectedEES API.

The API URI of the Ecas_SelectedEES API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [6], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

-	The <apiName> shall be "ecas-selected-ees".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the CAS takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

### 8A.1.2	Usage of HTTP

The provisions of clause 5.2.2 of 3GPP TS 29.122 [6] shall apply for the Ecas_SelectedEES API.

### 8A.1.3	Resources

There are no resources defined for this API in this release of the specification.

### 8A.1.4	Custom Operations without associated resources

#### 8A.1.4.1	Overview

The structure of the custom operation URIs of the Ecas_SelectedEES API is shown in Figure 8A.1.4.1-1.

Figure 8A.1.4.1-1: Custom operation URI structure of the Ecas_SelectedEES API

Table 8A.1.4.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the Ecas_SelectedEES API.

Table 8A.1.4.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Declare | /declare | POST | Enables a service consumer to declare the selected target EES related information. |

#### 8A.1.4.2	Operation: Declare

##### 8A.1.4.2.1	Description

The custom operation enables a service consumer to inform the CAS about the selected target EES related information during an ACR procedure from EAS to CAS.

##### 8A.1.4.2.2	Operation Definition

This operation shall support the request data structures and the response data structures and response codes specified in tables 8A.1.4.2.2-1 and 8A.1.4.2.2-2.

Table 8A.1.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SelEESDecInfo | M | 1 | Contains the parameters to declare the selected target EES related information. |

Table 8A.1.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The selected EES declaration request is successfully received and processed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative CAS. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative CAS. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 8A.1.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative CAS. |

Table 8A.1.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative CAS. |

### 8A.1.5	Notifications

There are no notifications defined for this API in this release of the specification.

### 8A.1.6	Data Model

#### 8A.1.6.1	General

This clause specifies the application data model supported by the API.

Table 8A.1.6.1-1 specifies the data types defined for the Ecas_SelectedEES API.

Table 8A.1.6.1-1: Ecas_SelectedEES API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| SelEESDecInfo | 8A.1.6.2.2 | Represent the selected target EES related information. |  |

Table 8A.1.6.1-2 specifies data types re-used by the Ecas_SelectedEES API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Ecas_SelectedEES API.

Table 8A.1.6.1-2: Ecas_SelectedEES API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| EndPoint | Clause 8.1.5.2.5 | Represents the endpoint information. |  |
| Gpsi | 3GPP TS 29.571 [8] | Used to identify the UE with GPSI. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |

#### 8A.1.6.2	Structured data types

##### 8A.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 8A.1.6.2.2	Type: SelEESDecInfo

Table 8A.1.6.2.2-1: Definition of type SelEESDecInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueId | Gpsi | M | 1 | Contains the identifier of the UE. |  |
| seleEesId | string | M | 1 | Contains the identifier of the selected EES. |  |
| seleEndpoint | EndPoint | M | 1 | Contains Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the selected EES. |  |
| easId | string | M | 1 | Contains the identifier of the concerned EAS. |  |
| acId | string | O | 0..1 | Contains the identifier of the concerned AC. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 8A.1.8. This attribute shall be present only when feature negotiation needs to take place. |  |

#### 8A.1.6.3	Simple data types and enumerations

##### 8A.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 8A.1.6.3.2	Simple data types

The simple data types defined in table 8A.1.6.3.2-1 shall be supported.

Table 8A.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 8A.1.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 8A.1.6.5	Binary data

##### 8A.1.6.5.1	Binary Data Types

Table 8A.1.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 8A.1.7	Error Handling

#### 8A.1.7.1	General

For the Ecas_SelectedEES API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [6]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [6] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [6].

In addition, the requirements in the following clauses are applicable for the Ecas_SelectedEES API.

#### 8A.1.7.2	Protocol Errors

No specific protocol errors for the Ecas_SelectedEES API are specified.

#### 8A.1.7.3	Application Errors

The application errors defined for the Ecas_SelectedEES API are listed in Table 8A.1.7.3-1.

Table 8A.1.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

### 8A.1.8	Feature negotiation

The optional features in table 8A.1.8-1 are defined for the Ecas_SelectedEES API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [6].

Table 8A.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

# 9	Edge Configuration Server API Definitions

## 9.1	Eecs_EESRegistration API

### 9.1.1	Introduction

The Eecs_EESRegistration service shall use the Eecs_EESRegistration API.

The API URI of the Eecs_EESRegistration API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure as defined in clause 7.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in clause 7.5.

-	The <apiName> shall be "eecs-eesregistration".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 9.1.2.

### 9.1.2	Resources

#### 9.1.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 9.1.2.1-1 depicts the resource URIs structure for the Eecs_EESRegistration API.

Figure 9.1.2.1-1: Resource URI structure of the Eecs_EESRegistration API

Table 9.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 9.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| EES Registrations | /registrations | POST | Registers a new EES at the Edge Configuration Server. |
| Individual EES Registration | /registrations/{registrationId} | GET | Fetch an individual EES registration resource. |
|  |  | PUT | Fully replace an individual EES registration resource. |
|  |  | DELETE | Remove an individual EES registration resource. |
|  |  | PATCH | Partially update an individual EES registration resource. |

#### 9.1.2.2	Resource: EES Registrations

##### 9.1.2.2.1	Description

This resource represents all the Edge Enabler Servers that are registered at a given Edge Configuration Server.

##### 9.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/eecs-eesregistration/<apiVersion>/registrations

This resource shall support the resource URI variables defined in the table 9.1.2.2.2-1.

Table 9.1.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |

##### 9.1.2.2.3	Resource Standard Methods

###### 9.1.2.2.3.1	POST

This method shall support the URI query parameters specified in table 9.1.2.2.3.1-1.

Table 9.1.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 9.1.2.2.3.1-2 and the response data structures and response codes specified in table 9.1.2.2.3.1-3.

Table 9.1.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EESRegistration | M | 1 | EES registration request information. |

Table 9.1.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EESRegistration | M | 1 | 201 Created | EES information is registered successfully at ECS. EES information registered with ECS is provided in the response body. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 9.1.2.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/eecs-eesregistration/<apiVersion>/registrations/{registrationId} |

##### 9.1.2.2.4	Resource Custom Operations

None.

#### 9.1.2.3	Resource: Individual EES Registration

##### 9.1.2.3.1	Description

This Individual EES Registration resource represents an individual EES registered at a given Edge Configuration Server.

##### 9.1.2.3.2	Resource Definition

Resource URI: {apiRoot}/eecs-eesregistration/<apiVersion>/registrations/{registrationId}

This resource shall support the resource URI variables defined in the table 9.1.2.3.2-1.

Table 9.1.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| registrationId | string | The EES registration identifier. |

##### 9.1.2.3.3	Resource Standard Methods

###### 9.1.2.3.3.1	GET

This method retrieves the EES information registered at Edge Configuration Server. This method shall support the URI query parameters specified in table 9.1.2.3.3.1-1.

Table 9.1.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 9.1.2.3.3.1-2 and the response data structures and response codes specified in table 9.1.2.3.3.1-3.

Table 9.1.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 9.1.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EESRegistration | M | 1 | 200 OK | The EES registration information at the Edge Configuration Server. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 9.1.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ECS. |

Table 9.1.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ECS. |

###### 9.1.2.3.3.2	PUT

This method updates the EES registration information at Edge Configuration Server by completely replacing the existing registration data (except the value of "eesId" within EESProfile data type and the value of "suppFeat" attribute within the EESRegistration data type). This method shall support the URI query parameters specified in the table 9.1.2.3.3.2-1.

Table 9.1.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 9.1.2.3.3.2-2 and the response data structures and response codes specified in table 9.1.2.3.3.2-3.

Table 9.1.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EESRegistration | M | 1 | Details of the EES registration information to be updated |

Table 9.1.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EESRegistration | M | 1 | 200 OK | The EES registration information updated successfully and the updated EES registration information is returned in the response. |
| n/a |  |  | 204 No Content | The EES registration information was updated successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 9.1.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ECS. |

Table 9.1.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ECS. |

###### 9.1.2.3.3.3	DELETE

This method deregisters an EES registration from the ECS. This method shall support the URI query parameters specified in the table 9.1.2.3.3.3-1.

Table 9.1.2.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 9.1.2.3.3.3-2 and the response data structures and response codes specified in table 9.1.2.3.3.3-3.

Table 9.1.2.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 9.1.2.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual EES registration information matching the registrationId is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 9.1.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ECS. |

Table 9.1.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ECS. |

###### 9.1.2.3.3.4	PATCH

This method partially updates the EES registration information at Edge Configuration Server. This method shall support the URI query parameters specified in the table 9.1.2.3.3.4-1.

Table 9.1.2.3.3.4-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 9.1.2.3.3.4-2 and the response data structures and response codes specified in table 9.1.2.3.3.4-3.

Table 9.1.2.3.3.4-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EESRegistrationPatch | M | 1 | Details of the EES registration information to be updated |

Table 9.1.2.3.3.4-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EESRegistration | M | 1 | 200 OK | The Individual EES registration information was updated successfully and the updated EES registration information is returned in the response. |
| n/a |  |  | 204 No Content | The Individual EES registration information was updated successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] also apply. |  |  |  |  |

Table 9.1.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ECS. |

Table 9.1.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ECS. |

##### 9.1.2.3.4	Resource Custom Operations

None.

### 9.1.3	Custom Operations without associated resources

None.

### 9.1.4	Notifications

None.

### 9.1.5	Data Model

#### 9.1.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API

Table 9.1.5.1-1 specifies the data types defined specifically for the Eecs_EESRegistration API service.

Table 9.1.5.1-1: Eecs_EESRegistration API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| EDNInfo | 9.1.5.2.10 | Represents EDN related information | EdgeApp_2 |
| EESRegistration | 9.1.5.2.2 | The EES registration information on ECS. |  |
| EESProfile | 9.1.5.2.3 | The profile information related to the EES in the EESRegistration data type. |  |
| ACRScenario | 9.1.5.3.3 | The ACR scenarios supported by EES. |  |
| EESRegistrationPatch | 9.1.5.2.4 | To partially update the EES Registration information. |  |
| ServiceArea | 9.1.5.2.5 | The topological and geographic areas served by EES. |  |
| TopologicalServiceArea | 9.1.5.2.6 | The topological areas served by EES. |  |
| GeographicalServiceArea | 9.1.5.2.7 | The geographic areas served by EES. |  |
| EASInstantiationInfo | 9.1.5.2.8 | Represents the EAS instantiation information. | EdgeApp_2 |
| InstantiationCriteria | 9.1.5.2.9 | Represents the instantiation cirteria. | EdgeApp_2 |
| InstantiationStatus | 9.1.5.3.4 | Represents the instantiation status. | EdgeApp_2 |
| LoadInfo | 9.1.5.2.11 | Represents the load information. | EdgeApp_3 |
| AppSatCovAvailInfo | 9.1.5.2.13 | Represents the Application Satellite Coverage Availability information data. | 5GSatApp_1 |
| UeServSatInfo | 9.1.5.2.12 | Represents the UE serving satellite information. | 5GSatApp_1 |

Table 9.1.5.1-2 specifies data types re-used by the Eecs_EESRegistration API service.

Table 9.1.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of optional features defined in table 9.1.7-1. |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [6] | Represents the schedule information. |  |
| DateTime | 3GPP TS 29.122 [6] | Represents a date and a time. |  |
| Ecgi | 3GPP TS 29.571 [8] | Represents an EUTRA cell identifier. |  |
| Ncgi | 3GPP TS 29.571 [8] | Represents an NR cell identifier. |  |
| Tai | 3GPP TS 29.571 [8] | Represents a tracking area identity. |  |
| GeographicArea | 3GPP TS 29.572 [11] | Identifies the geographical information of the user(s). |  |
| CivicAddress | 3GPP TS 29.572 [11] | Identifies the civic address information of the user(s). |  |
| PlmnIdNid | 3GPP TS 29.571 [8] | Identifies the network: PLMN Identifier or the SNPN Identifier (the PLMN Identifier and the NID). Used to indicate the serving network as part of topological service areas. |  |
| Dnn | 3GPP TS 29.571 [8] | Represents a DNN. | EdgeApp_2 |
| Dnai | 3GPP TS 29.571 [8] | Used to represent the list of DNAI(s) information associated with EES. |  |
| TimeWindow | 3GPP TS 29.122 [6] | Represents the time window. |  |
| DateTimeRm | 3GPP TS 29.571 [8] | Used to capture the expiration time EES registration patch. |  |
| EndPoint | 8.1.5.2.5 | The end point information of the Edge Enabler Server in the EES profile. |  |
| EASBundleInfo | 8.1.5.3.8 | Represents EAS bundle information. | EdgeApp_2 |
| AscaiInfo | 3GPP TS 29.549 [26] | Represents the Application Satellite Coverage Availability Information. | 5GSatApp_1 |

#### 9.1.5.2	Structured data types

##### 9.1.5.2.1	Introduction

##### 9.1.5.2.2	Type: EESRegistration

Table 9.1.5.2.2-1: Definition of type EESRegistration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eesProf | EESProfile | M | 1 | The profile information of the EES. |  |
| expTime | DateTime | O | 0..1 | Identifies the expiration time for the EES registration. If the expiration time is not present, then it indicates that the registration of EES never expires. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported feature(s) among the ones defined in clause 9.1.7. This attribute shall be present only when feature negotiation needs to take place. |  |

##### 9.1.5.2.3	Type: EESProfile

Table 9.1.5.2.3-1: Definition of type EESProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eesId | string | M | 1 | The identifier of the EES |  |
| endPt | EndPoint | M | 1 | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |  |
| easIds | array(string) | O | 1..N | The application identities of the Edge Application Servers (e.g. URI, FQDN) registered with the EES or expected to be registered with the EES. |  |
| easBdlInfos | map(array(EASBundleInfo)) | O | 1..N(1..M) | Represents the list of EAS bundles to which each EAS among the ones identified by the "easIds" attribute belongs. The key of the map shall be the identifier of the EAS to which the provided EAS bundle information within the map value corresponds. (NOTE) | EdgeApp_2 |
| allowedMNOsInfo | map(array(PlmnIdNid)) | O | 1..N(1..M) | Contains the per-EAS allowed MNO information, i.e., the list of the allowed PLMN ID(s) for an EAS among the ones identified by the "easIds" attribute. Only the subscribers of the allowed PLMN(s) can consume the EES services for a concerned EAS. The key of the map shall be the identifier of the EAS to which the provided allowed MNO information within the map value corresponds. | EdgeApp_3 |
| ednInfoSets | EDNInfo | O | 0..1 | Contains EDN related information for the EDN where the EES resides. | EdgeApp_2 |
| easInstInfo | map(EASInstantiationInfo) | O | 1..N | Contains the EAS instantiation information for each EAS identified by the "easIds" attribute. The key of the map shall be the EAS ID to which the provided instantiation information within the map value relates. | EdgeApp_2 |
| provId | string | O | 0..1 | Identifier of the ECSP that provides the EES provider. |  |
| svcArea | ServiceArea | O | 0..1 | The list of geographical and topological areas that the EES serves. EECs in the UE that are outside the area shall not be served. |  |
| appLocs | array(Dnai) | O | 1..N | List of DNAI(s) associated with the EES. This is a list of potential locations of the applications. It is a subset of the DNAI(s) associated with the EDN, where the EES resides. |  |
| svcContSupp | array(ACRScenario) | O | 1..N | The ACR scenarios supported by the EES for service continuity. If this attribute is not present, then the EES does not support service continuity. |  |
| svcContSuppExt1 | array(EASBundleInfo) | O | 1..N | Represents the information related to the EES ability to handle bundled EAS coordinated ACRs. This attribute may be present only when the "svcContSupp" attribute is also present. When this attribute is present, it indicates that the EES (identified by the "eesId" attribute) is able to handle bundled EAS coordinated ACRs and contains the information of the EAS bundle(s) for which the EES is able to handle bundled EAS coordinated ACRs. | EdgeApp_2 |
| eecRegConf | boolean | M | 1 | Set to "true" if the EEC is required to register on the EES to use edge services. Set to "false" if EEC is not required to register on the EES to use edge services. |  |
| easLoadInfoList | map(LoadInfo) | O | 1..N | Represents the list of load information per application identified by the "easIds" attribute. The load information provided within each map value corresponds to the load information of all the the EAS(s) providing the particular application. . The key of the map shall be set to the the EAS ID (among the ones provided within the "easIds" attribute) for which the load information provided within the map value corresponds. | EdgeApp_3 |
| ueServingSat | UeServSatInfo | O | 0..1 | Contains UE serving satellite information. | 5GSatApp_1 |
| NOTE: Within each EASBundleInfo encoded map entry of this attribute, the "mainEasId" attribute shall not be present. |  |  |  |  |  |

##### 9.1.5.2.4	Type: EESRegistrationPatch

Table 9.1.5.2.4-1: Definition of type EESRegistrationPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eesProf | EESProfile | O | 0..1 | The profile information of the EES. |  |
| expTime | DateTimeRm | O | 0..1 | Identifies the expiration time for the EES registration. If the expiration time is not present, then it indicates that the registration of EES never expires. |  |
| NOTE: The value of the "eesId" attribute within the EESProfile data type shall be the same as the one within the EES Registration data type during the creation of resource procedure. |  |  |  |  |  |

##### 9.1.5.2.5	Type: ServiceArea

Table 9.1.5.2.5-1: Definition of type ServiceArea

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| topServAr | TopologicalServiceArea | O | 0..1 | The topological service areas. (NOTE) |  |
| geoServAr | GeographicalServiceArea | O | 0..1 | The geographical service areas. |  |
| NOTE: The "topServAr" attribute is not applicable for untrusted functional elements (EESs and/or EASs deployed outside the MNO trust domain). |  |  |  |  |  |

##### 9.1.5.2.6	Type: TopologicalServiceArea

Table 9.1.5.2.6-1: Definition of type TopologicalServiceArea

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ecgis | array(Ecgi) | O | 1..N | This IE contains a list of E-UTRA cell identities. |  |
| ncgis | array(Ncgi) | O | 1..N | This IE contains a list of NR cell identities. |  |
| tais | array(Tai) | O | 1..N | This IE contains a list of tracking area identities. |  |
| plmnIds | array(PlmnIdNid) | O | 1..N | List of serving network (PLMN or SNPN) identities . For a SNPN, a NID together with a PLMN ID identifies the SNPN. (NOTE) |  |
| NOTE: A combination of these information elements should not have duplicate or overlapping information for the same topological Service Area. |  |  |  |  |  |

##### 9.1.5.2.7	Type: GeographicalServiceArea

Table 9.1.5.2.7-1: Definition of type GeographicalServiceArea

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| geoArs | array(GeographicArea) | O | 1..N | Identifies a list of geographic area of the user where the UE is located. |  |
| civicAddrs | array(CivicAddress) | O | 1..N | Identifies a list of civic addresses of the user where the UE is located. |  |

##### 9.1.5.2.8	Type: EASInstantiationInfo

Table 9.1.5.2.8-1: Definition of type EASInstantiationInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easId | string | M | 1 | The application identities of the Edge Application Servers. |  |
| status | InstantiationStatus | M | 1 | Contains the instantiation status of the EAS. |  |
| instCrit | InstantiationCriteria | C | 0..1 | Contains the criteria information upon which the EAS can be instantiated. This attribute shall be present only when the value of the "status" attribute is set to "INSTANTIABLE". |  |

##### 9.1.5.2.9	Type: InstantiationCriteria

Table 9.1.5.2.9-1: Definition of type InstantiationCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| instantiationTime | DateTime | C | 0..1 | Identifies the time at which the EAS shall be instantiated. (NOTE) |  |
| instWindows | array(TimeWindow) | C | 1..N | The time windows at which the EAS shall be instantiated. (NOTE) |  |
| scheds | array(ScheduledCommunicationTime) | C | 1..N | The schedules at which the EAS shall be instantiated. (NOTE) |  |
| NOTE : These attributes are mutually exclusive. Either one of them shall be present. |  |  |  |  |  |

##### 9.1.5.2.10	Type: EDNInfo

Table 9.1.5.2.10-1: Definition of type EDNInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dnn | Dnn | M | 1 | Contains the DNN identifying the EDN |  |
| dnais | array(Dnai) | O | 1..N | Contains the DNAI(s) associated with the EDN. |  |

##### 9.1.5.2.11	Type: LoadInfo

Table 9.1.5.2.11-1: Definition of type LoadInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| loadLevel | integer | O | 0..1 | Represents the laod level, expressed as a percentage. Minimum = 0. Maximum = 100. |  |
| loadWarningStat | boolean | O | 0..1 | Represents the status of load warning. - "true" indicates that load warning is activated. - "false" indicates that load warning is not activated. - The default value is "false" when this attribute is omitted. |  |

##### 9.1.5.2.12	Type: UeServSatInfo

Table 9.1.5.2.12-1: Definition of type UeServSatInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dynServAreaInd | boolean | O | 0..1 | Indicates whether the EES service area is mobile or not. - "true" indicates that the EES service area is mobile (e.g., case of a a MEO or LEO satellite). - "false" indicates that the EES service area is not mobile (e.g., case of a a GEO satellite). - The default value is "false" when this attribute is omitted. |  |
| satId | string | M | 1 | Contains the identifier of the satellite, which also identifies the satellite EDN. |  |
| trajId | string | O | 0..1 | Contains the identifier of the trajectory of the satellite, which also identifies the trajectory of the EES. |  |
| ascaiInfo | AppSatCovAvailInfo | O | 0..1 | Contains the Applications Satellite Coverage Availability Information of the satellite on which the EES is onboard. |  |

##### 9.1.5.2.13	Type: AppSatCovAvailInfo

Table 9.1.5.2.13-1: Definition of type AppSatCovAvailInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| covAvailInfo | AscaiInfo | M | 1 | Contains the Application Satellite Coverage Availability Information of the satellite. |  |

#### 9.1.5.3	Simple data types and enumerations

##### 9.1.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 9.1.5.3.2	Simple data types

The simple data types defined in table 9.1.5.3.2-1 shall be supported.

Table 9.1.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 9.1.5.3.3	Enumeration: ACRScenario

The enumeration ACRScenario represents the ACR scenarios supported. It shall comply with the provisions defined in table 9.1.5.3.3-1.

Table 9.1.5.3.3-1: Enumeration ACRScenario

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| EEC_INITIATED | Represents the EEC initiated ACR scenario. |  |
| EEC_EXECUTED_VIA_SOURCE_EES | Represents the EEC ACR scenario executed via the S-EES. |  |
| EEC_EXECUTED_VIA_TARGET_EES | Represents the EEC ACR scenario executed via the T-EES. |  |
| SOURCE_EAS_DECIDED | Represents the EEC ACR scenario where the S-EAS decides to perform ACR. |  |
| SOURCE_EES_EXECUTED | Represents the EEC ACR scenario where S-EES executes the ACR. |  |
| EEL_MANAGED_ACR | Represents the EEC ACR scenario where the ACR is managed by the Edge Enabler Layer. |  |

##### 9.1.5.3.4	Enumeration: InstantiationStatus

The enumeration InstantiationStatus represents the instantiation status of the EAS. It shall comply with the provisions defined in table 9.1.5.3.4-1.

Table 9.1.5.3.4-1: Enumeration InstantiationStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| INSTANTIATED | Indicates that the EAS is instantiated. |  |
| INSTANTIABLE | Indicates that the EAS is instantiable but not yet instantiated. |  |

### 9.1.6	Error Handling

#### 9.1.6.1	General

For the Eecs_EESRegistration API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [6]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [6] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [6].

In addition, the requirements in the following clauses are applicable for the Eecs_EESRegistration API.

#### 9.1.6.2	Protocol Errors

No specific protocol errors for the Eecs_EESRegistration API are specified.

#### 9.1.6.3	Application Errors

The application errors defined for the Eecs_EESRegistration API are listed in Table 9.1.6.3-1.

Table 9.1.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

### 9.1.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8 Table 9.1.7-1 lists the supported features for Eecs_EESRegistration API.

Table 9.1.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | EdgeApp_2 | This feature indicates the support of the enhancements to the Edge Applications. Within this feature, the following enhancements are covered: - Support of EAS bundle information. - Support of EAS instantiation information management. - Support of EDN related information provisioning within the EES profile. - Support of the indication of the EAS ability to handle bundled EAS ACRs within the EAS profile. |
| 2 | EdgeApp_3 | This feature indicates the support of the second set of enhancements to the Edge Applications Enabler Layer. Within this feature, the following enhancements are covered: - Support of the per-EAS allowed MNO information in the EES Profile. - Support of EAS load information reporting within the EES Profile. |
| 3 | 5GSatApp_1 | This feature indicates the support of the first set of enhancements to the Edge Enabler Layer to support 5G Satellite applications. Within this feature, the following enhancements are covered: - support of EES profile with satellite information. |

## 9.2	Eecs_TargetEESDiscovery API

### 9.2.1	Introduction

The Eecs_TargetEESDiscovery service shall use the Eecs_TargetEESDiscovery API.

The API URI of the Eecs_TargetEESDiscovery API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure as defined in clause 7.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in clause 7.5.

-	The <apiName> shall be "eecs-targeteesdiscovery".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 9.2.2.

### 9.2.2	Resources

#### 9.2.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 9.2.2.1-1 depicts the resource URIs structure for the Eecs_TargetEESDiscovery API.

Figure 9.2.2.1-1: Resource URI structure of the Eecs_TargetEESDiscovery API

Table 9.2.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 9.2.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| EES Profiles (NOTE) | /ees-profiles | GET | Retrieve the target Enabler Server information. |
| NOTE: In this release of the specification, this resource is extended to manage also the CES profiles, not only the EES profiles, in order to support cloud enabler services. |  |  |  |

#### 9.2.2.2	Resource: EES Profiles

##### 9.2.2.2.1	Description

This resource allows a service consumer to retrieve the target Enabler Server information from the ECS.

##### 9.2.2.2.2	Resource Definition

Resource URI: {apiRoot}/eecs-targeteesdiscovery/<apiVersion>/ees-profiles

This resource shall support the resource URI variables defined in the table 9.2.2.2.2-1.

Table 9.2.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |

##### 9.2.2.2.3	Resource Standard Methods

###### 9.2.2.2.3.1	GET

This method allows the service consumer to fetch the target Enabler Server information as specified in 3GPP TS 23.558 [2], from the ECS with a given discovery filters.

This method shall support the URI query parameters specified in table 9.2.2.2.3.1-1.

Table 9.2.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ees-id | string | M | 1 | Unique identifier of the target Enabler Server. |  |
| eas-id | string | M | 1 | Represents the application identifier of the source Application Server (e.g., S-EAS or CAS), e.g. URI, FQDN. |  |
| target-dnai | Dnai | O | 0..1 | The DNAI information associated with the potential target Enabler Server(s) and/or target Application Server(s). (NOTE) |  |
| cand-dnais-info | CandidateDnaisInfo | O | 0..1 | Contains the candidate DNAI(s) related information. This query parameter is applicable only for the common EAS case. (NOTE) | EdgeApp_3 |
| ue-id | Gpsi | O | 0..1 | Identifier of the UE. |  |
| ue-location | LocationArea5G | O | 0..1 | The location information of the UE. |  |
| eec-srv-cont-supp | EECSrvContinuitySupport | O | 0..1 | Indicates whether the EEC supports service continuity or not and the related service continuity support information. | EdgeApp_2 |
| ac-svc-cont-supp | array(ACRScenario) | O | 1..N | Indicates that the AC supports service continuity and contains the related service continuity support information (i.e., supported ACR scenarios). | EdgeApp_2 |
| bdl-id | string | O | 0..1 | Contains the identifier of the EAS bundle. This query parameter may be present only when the "bdl-type" query parameter is also present and set to "PROXY". | EdgeApp_2 |
| bdl-type | BdlType | O | 0..1 | Contains the EAS bundle type. | EdgeApp_2 |
| eas-ids-list | array(string) | O | 1..N | Contains the list of the identifier(s) of the EAS(s) associated with the EAS bundle. | EdgeApp_3 |
| ens-ind | boolean | O | 0..1 | Indicates whether edge node sharing is requested. - When set to "true", it indicates that edge node sharing is requested. - When set to "false" (default if omitted), it indicates that node sharing is not requested. | EdgeApp_2 |
| app-grp-id | string | O | 0..1 | Contains the application group identifier. When this query parameter is provided, then it indicates that the request is for the retrieval of an EES list for the announcement of common EAS. | EdgeApp_2 |
| supp-feats | SupportedFeatures | C | 0..1 | Contains the list of supported feature(s) among the ones defined in clause 9.2.7. This query parameter shall be present only when feature negotiation needs to take place. |  |
| serving-mno-info | PlmnIdNid | O | 0..1 | Contains the serving MNO information, i.e., the MNO that is serving the subscriber. | EdgeApp_3 |
| pred-exp-time | DateTime | O | 0..1 | Contains the prediction expiration time, i.e., the estimated time at which the UE should reach the predicted/expected location or EAS service area at the latest. | EdgeApp_3 |
| tunnel-info | TunnelInfo | O | 0..1 | Contains the target tunnel information. | EdgeApp_2_Ext1 |
| NOTE: When the "EdgeApp_3" feature is supported, these query parameters are mutually exclusive. |  |  |  |  |  |

This method shall support the request data structures specified in table 9.2.2.2.3.1-2 and the response data structures and response codes specified in table 9.2.2.2.3.1-3.

Table 9.2.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 9.2.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ECSServProvResp | M | 1 | 200 OK | The EDN configuration and the target Enabler Server information determined by the ECS based on the query parameters. |
| NOTE: The mandatory HTTP error status code for the HTTP GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

##### 9.2.2.2.4	Resource Custom Operations

None.

### 9.2.3	Custom Operations without associated resources

None.

### 9.2.4	Notifications

None.

### 9.2.5	Data Model

#### 9.2.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API.

Table 9.2.5.1-1 specifies the data types defined specifically for the Eecs_TargetEESDiscovery API.

Table 9.2.5.1-1: Eecs_TargetEESDiscovery API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| CandidateDnaisInfo | 9.5.5.2.3 | Represents the candidate DNAI(s) related information. | EdgeApp_3 |
| TunnelInfo | 9.2.5.2.2 | Contains the tunnel information. | EdgeApp_2_Ext1 |

Table 9.2.5.1-2 specifies data types re-used by the Eecs_TargetEESDiscovery API service.

Table 9.2.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ACRScenario | Clause 9.1.5.3.3 | Represents ACR scenarios. | EdgeApp_2 |
| BdlType | Clause 8.1.5.3.6 | Represent EAS Bundle type. | EdgeApp_2 |
| DateTime | 3GPP TS 29.122 [6] | Represents a date and a time. | EdgeApp_3 |
| Dnai | 3GPP TS 29.571 [8] | Used to indicate the target DNAI information. |  |
| ECSServProvResp | 3GPP TS 24.558 [14] | The response to the target EES discovery request, which includes the EDN configuration along with list of EES(s) information. |  |
| EECSrvContinuitySupport | Clause 8.7.5.2.8 | Represent service continuity support related information for an EEC. | EdgeApp_2 |
| Gpsi | 3GPP TS 29.571 [8] | Used to identify the UE in the query parameter. |  |
| LocationArea5G | 3GPP TS 29.122 [6] | Used to indicate the location information of the UE in the query parameter. |  |
| PlmnIdNid | 3GPP TS 29.571 [8] | Represents the identifier of the network, i.e., PLMN or SNPN. | EdgeApp_3 |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported feature(s) and is used to negotiate the applicability of optional features. |  |
| TunnelAddress | 3GPP TS 29.571 [8] | Represents the tunnel address information. | EdgeApp_2_Ext1 |

#### 9.2.5.2	Structured data types

##### 9.2.5.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 9.2.5.2.2	Type: TunnelInfo

Table 9.2.5.2.2-1: Definition of type TunnelInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| servProvId | string | M | 1 | Contains the identifier of the service provider. |  |
| endPoint | TunnelAddress | M | 1 | Contains the endpoint address of the tunnel server. |  |

##### 9.2.5.2.3	Type: CandidateDnaisInfo

Table 9.2.5.2.3-1: Definition of type CandidateDnaisInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| candidateDnais | array(Dnai) | M | 1..N | Contains the candidate DNAI(s). |  |
| candDnaisPrioInd | boolean | O | 0..1 | Indicates whether the candidate DNAI(s) provided within the "candidateDnais" attribute are in descending priority order, i.e., the lower the array index of the "candidateDnais" attribute the higher the priority of the respective DNAI. - "true" indicates that the candidate DNAI(s) provided within the "candidateDnais" attribute are in descending priority order. - "false" indicates that the candidate DNAI(s) provided within the "candidateDnais" attribute are not in descending priority order. - The default value is "false" if this attribute is omitted. |  |

#### 9.2.5.3	Simple data types and enumerations

None.

### 9.2.6	Error Handling

General error responses are defined in clause 7.7.

### 9.2.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 9.2.7-1 lists the supported features for Eecs_TargetEESDiscovery API.

Table 9.2.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | EdgeApp_2 | This feature indicates the support of the enhancements to the Edge Applications. Within this feature, the following enhancements are covered: - Support T-EAS discovery based on EEC service continuity information and/or AC service continuity information. - Support EAS bundle information management. - Support of requesting edge node sharing. - Support of T-EES discovery based on application group identifier. |
| 2 | EdgeApp_3 | This feature indicates the support of the second set of enhancements to the Edge Applications Enabler Layer. Within this feature, the following enhancements are covered: - Support of the allowed MNO information in the T-EES discovery request. - Support of the prediction expiration time in the T-EES discovery request. - Support of the list of the identifier(s) of the EAS(s) associated with the EAS bundle in the T-EES discovery request. - Support target EES discovery based on the candidate DNAI(s) information to support the common EAS case. |
| 3 | EdgeApp_2_Ext1 | This feature indicates the support of the enhancements to the Edge Applications. Within this feature, the following enhancements are covered: - Support of the tunnel information in the T-EES discovery request. |

## 9.3	Eecs_EASInfoManagement API

### 9.3.1	Introduction

The Eecs_EASInfoManagement service shall use the Eecs_EASInfoManagement API.

The API URI of the Eecs_EASInfoManagement API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [6], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

-	The <apiName> shall be "eecs-eim".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

### 9.3.2	Usage of HTTP

The provisions of clause 5.2.2 of 3GPP TS 29.122 [6] shall apply for the Eecs_EASInfoManagement API.

### 9.3.3	Resources

#### 9.3.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 9.3.3.1-1 depicts the resource URIs structure for the Eecs_EASInfoManagement API.

Figure 9.3.3.1-1: Resource URI structure of the Eecs_EASInfoManagement API

Table 9.3.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 9.3.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Common EAS Bindings | /bindings | GET | Retrieve common EAS binding information. |
|  |  | POST | Store a new common EAS binding information. |
| Individual Common EAS Binding | /bindings/{bindingId} | GET | Retrieve an existing "Individual Common EAS Binding" resource. |
|  |  | PUT | Update an existing "Individual Common EAS Binding" resource. |
|  |  | DELETE | Delete an existing "Individual Common EAS Binding" resource. |

#### 9.3.3.2	Resource: Common EAS Bindings

##### 9.3.3.2.1	Description

This resource represents the Common EAS Bindings managed by the ECS.

##### 9.3.3.2.2	Resource Definition

Resource URI: {apiRoot}/eecs-eim/<apiVersion>/bindings

This resource shall support the resource URI variables defined in the table 9.3.3.2.2-1.

Table 9.3.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 9.3.1. |

##### 9.3.3.2.3	Resource Standard Methods

###### 9.3.3.2.3.1	GET

The HTTP GET method allows a service consumer to retrieve common EAS binding information.

This method shall support the URI query parameters specified in table 9.3.3.2.3.1-1.

Table 9.3.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestor-id | string | M | 1 | Represents the identifier of the service consumer. |  |
| app-group-id | string | M | 1 | Contains the application group associated with the EAS ID. |  |
| eas-id | string | M | 1 | Represents the identifier of the EAS, e.g., URI, FQDN. |  |
| eas-bdl-Id-list | array(string) | O | 1..N | Contains a list of associated EAS IDs in an EAS bundle. | EdgeApp_3 |
| eas-bdl-id | string | O | 0..1 | Represents the EAS bundle ID. (NOTE) | EdgeApp_3 |
| main-eas-id | string | O | 0..1 | Represents the identifier of the main EAS in an EAS bundle. (NOTE) | EdgeApp_3 |
| supp-feats | SupportedFeatures | C | 0..1 | Contains the list of supported feature(s) among the ones defined in clause 9.3.8. This query parameter shall be present only when feature negotiation needs to take place. |  |
| NOTE: These query parameters are mutually exclusive. Only one of them may be present. |  |  |  |  |  |

This method shall support the request data structures specified in table 9.3.3.2.3.1-2 and the response data structures and response codes specified in table 9.3.3.2.3.1-3.

Table 9.3.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 9.3.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CommonEASBindResp | M | 1 | 200 OK | Successful case. The requested Common EAS Binding Information stored in the ECS is returned. |
| n/a |  |  | 204 No Content | Successful case. There is no Common EAS Binding Information corresponding to the received query parameters. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.3.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

Table 9.3.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

###### 9.3.3.2.3.2	POST

The HTTP POST method allows a service consumer to register the common EAS binding information.

This method shall support the URI query parameters specified in table 9.3.3.2.3.2-1.

Table 9.3.3.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 9.3.3.2.3.2-2 and the response data structures and response codes specified in table 9.3.3.2.3.2-3.

Table 9.3.3.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| CommonEASBindReq | M | 1 | Represents the Common EAS Binding Information. |

Table 9.3.3.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CommonEASBindResp | M | 1 | 201 Created | Successful case. The Common EAS Binding information is successfully stored. |
| ProblemDetailsEIMExt | O | 0..1 | 403 Forbidden | The request is rejected because there is an existing Common EAS Binding information stored in the ECS for the application group. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. NOTE 2: Failure cases are described in clause 9.3.7.3. |  |  |  |  |

##### 9.3.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 9.3.3.3	Resource: Individual Common EAS Binding

##### 9.3.3.3.1	Description

This resource represents a Common EAS Binding managed by the ECS.

##### 9.3.3.3.2	Resource Definition

Resource URI: {apiRoot}/eecs-eim/<apiVersion>/bindings/{bindingId}

This resource shall support the resource URI variables defined in the table 9.3.3.3.2-1.

Table 9.3.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| bindingId | string | Represents the identifier of the "Individual Common EAS Binding" resource. |

##### 9.3.3.3.3	Resource Standard Methods

###### 9.3.3.3.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing "Individual Common EAS Binding" resource at the ECS.

This method shall support the URI query parameters specified in table 9.3.3.3.3.1-1.

Table 9.3.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 9.3.3.3.3.1-2 and the response data structures and response codes specified in table 9.3.3.3.3.1-3.

Table 9.3.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 9.3.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CommonEASBindingResp | M | 1 | 200 OK | Successful case. The requested "Individual Common EAS Binding" resource is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status code for the HTTP GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.3.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

Table 9.3.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

###### 9.3.3.3.3.2	PUT

The HTTP PUT method enables a service consumer to update an existing "Individual Common EAS Binding" resource at the ECS.

This method shall support the URI query parameters specified in the table 9.3.3.3.3.2-1.

Table 9.3.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 9.3.3.3.3.2-2 and the response data structures and response codes specified in table 9.3.3.3.3.2-3.

Table 9.3.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| CommonEASBinding | M | 1 | Contains the updated representation of the "Individual Common EAS Binding" resource. |

Table 9.3.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CommonEASBinding | M | 1 | 200 OK | Successful case. The "Individual Common EAS Binding" resource is successfully updated and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Common EAS Binding" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status code for the HTTP PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.3.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

Table 9.3.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

###### 9.3.3.3.3.3	PATCH

The HTTP PATCH method allows a service consumer to request the modification of an existing "Individual Common EAS Binding" resource at the ECS.

This method shall support the URI query parameters specified in table 9.3.3.3.3.3-1.

Table 9.3.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 9.3.3.3.3.3-2 and the response data structures and response codes specified in table 9.3.3.3.3.3-3.

Table 9.3.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| CommonEASBindingPatch | M | 1 | Represents the parameters to request the modification of the "Individual Common EAS Binding" resource. |

Table 9.3.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CommonEASBinding | M | 1 | 200 OK | Successful case. The "Individual Common EAS Binding" resource is successfully modified and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Common EAS Binding" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.3.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

Table 9.3.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

###### 9.3.3.3.3.4	DELETE

The HTTP DELETE method enables a service consumer to delete an existing "Individual Common EAS Binding" resource at the ECS.

This method shall support the URI query parameters specified in the table 9.3.3.3.3.4-1.Table 9.3.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 9.3.3.3.3.3-2 and the response data structures and response codes specified in table 9.3.3.3.3.3-3.

Table 9.3.3.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 9.3.3.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual Common EAS Binding" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status code for the HTTP DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.3.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

Table 9.3.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

### 9.3.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this resource in this release of the specification.

### 9.3.5	Notifications

There are no notifications defined for this API in this release of the specification.

### 9.3.6	Data Model

#### 9.3.6.1	General

This clause specifies the application data model supported by the API.

Table 9.3.6.1-1 specifies the data types defined for the Eecs_EASInfoManagement API.

Table 9.3.6.1-1: Eecs_EASInfoManagement API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| CommonEASBinding | 9.3.6.2.4 | Represents the common EAS binding information. |  |
| CommonEASBindingPatch | 9.3.6.2.6 | Represents the requested modifications to common EAS binding information | EdgeApp_3 |
| CommonEASBindReq | 9.3.6.2.2 | Represents the common EAS binding information registration request. |  |
| CommonEASBindResp | 9.3.6.2.3 | Represents the common EAS binding information registration or retrieval response. |  |
| CommonEasBdlInfo | 9.3.6.2.5 | Represents the common EAS bundle information. | EdgeApp_3 |
| ProblemDetailsEIMExt | 9.3.6.4.1 | Represents an extension to the ProblemDetails data structure with additional EAS Info Management related error information. |  |

Table 9.3.6.1-2 specifies data types re-used by the Eecs_EASInfoManagement API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Eecs_EASInfoManagement API.

Table 9.3.6.1-2: Eecs_EASInfoManagement API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| EASServiceKPI | 8.1.5.2.4 | Represent the EAS Service KPIs. | EdgeApp_3 |
| EDNInfo | 9.1.5.2.10 | Represent the EDN related information. |  |
| EndPoint | 8.1.5.2.5 | Represents the endpoint information. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |

#### 9.3.6.2	Structured data types

##### 9.3.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 9.3.6.2.2	Type: CommonEASBindReq

Table 9.3.6.2.2-1: Definition of type CommonEASBindReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | string | M | 1 | Contains the identifier of the service consumer that is sending the request. |  |
| binding | CommonEASBinding | M | 1 | Contains the common EAS Binding information. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported feature(s) among the ones defined in clause 9.3.8. This attribute shall be present only when feature negotiation needs to take place. |  |

##### 9.3.6.2.3	Type: CommonEASBindResp

Table 9.3.6.2.3-1: Definition of type CommonEASBindResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| binding | CommonEASBinding | M | 1 | Contains the common EAS Binding information. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported feature(s) among the ones defined in clause 9.3.8. This attribute shall be present only when feature negotiation needs to take place. |  |

##### 9.3.6.2.4	Type: CommonEASBinding

Table 9.3.6.2.4-1: Definition of type CommonEASBinding

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appGroupId | string | M | 1 | Contains the identifier of the application group for which the common EAS binding information is provided. |  |
| easId | string | M | 1 | Contains the identifier of the common EAS. |  |
| easEndpoints | array(EndPoint) | M | 1..N | Contains the endpoint(s) information of the common EAS. |  |
| assocEasEndpoints | array(EndPoint) | O | 1..N | Contains the associated EAS endpoint(s) corresponding to the requested EAS ID(s). This attribute may be present only in a Common EAS Binding Information Storage/Retrieval response. | EdgeApp_3 |
| eesEndpoint | EndPoint | C | 0..1 | Contains the endpoint information of the common EES. Shall be provided in storage response in the POST method. |  |
| ednInfo | EDNInfo | C | 0..1 | Contains the information of EDN where the common EAS resides. This attribute shall be present in a Common EAS Binding Information storage/update request. When the "EdgeApp_3" is supported, this attribute may also be present in a Common EAS Binding Information Retrieval response. |  |
| easSvcKpi | EASServiceKPI | O | 0..1 | Contains the EAS service KPIs. This attribute may be present only in a Common EAS Binding Information Storage/Update/Retrieval request or in a Common EAS Binding Information Retrieval response. | EdgeApp_3 |
| commEasBdlInfo | CommEasBdlInfo | O | 0..1 | Contains information of the EAS bundle to which the common EAS belongs. This attribute may be present only in a Common EAS Binding Information Storage request. | EdgeApp_3 |

##### 9.3.6.2.5	Type: CommEasBdlInfo

Table 9.3.6.2.5-1: Definition of type CommEasBdlInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easIdsList | array(string) | M | 1..N | Contains the list of the identifier(s) of the EAS(s) constituting the EAS bundle where the common EAS resides. |  |
| easEndpoints | map(EndPoint) | M | 1..N | Contains the list of the endpoint(s) of the EAS(s) constituting the EAS bundle where the common EAS resides. The key of the map shall be set to the identifier or the EAS (among the ones provided within the "easIdsList" attribute) for which the endpoint information provided in the map value is related. |  |
| mainEasId | string | O | 0..1 | Represents the identifier of the EAS that plays the role of the main EAS within the EAS bundle where the common EAS resides. (NOTE) |  |
| bdlId | string | O | 0..1 | Contains the identifier of the EAS bundle where the common EAS resides. (NOTE) |  |
| NOTE: These attributes are mutually exclusive. Only one of them may be present. |  |  |  |  |  |

##### 9.3.6.2.6	Type: CommonEASBindingPatch

Table 9.3.6.2.6-1: Definition of type CommonEASBindingPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easEndpoints | array(EndPoint) | O | 1..N | Contains the endpoint(s) information of the common EAS. |  |
| assocEasEndpoints | array(EndPoint) | O | 1..N | Contains the associated EAS endpoint(s) corresponding to the requested EAS ID(s). This attribute may be present only in a Common EAS Binding Information Storage/Retrieval response. |  |
| eesEndpoint | EndPoint | C | 0..1 | Contains the endpoint information of the common EES. Shall be provided in storage response in the POST method. |  |
| ednInfo | EDNInfo | C | 0..1 | Contains the information of EDN where the common EAS resides. This attribute shall be present in a Common EAS Binding Information storage/update request. |  |
| commEasBdlInfo | CommEasBdlInfo | O | 0..1 | Contains information of the EAS bundle to which the common EAS belongs. This attribute may be present only in a Common EAS Binding Information Storage request. |  |

#### 9.3.6.3	Simple data types and enumerations

##### 9.3.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 9.3.6.3.2	Simple data types

The simple data types defined in table 9.3.6.3.2-1 shall be supported.

Table 9.3.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 9.3.6.4	Data types describing alternative data types or combinations of data types

##### 9.3.6.4.1	Type: ProblemDetailsEIMExt

Table 9.3.6.4.1-1: Definition of type ProblemDetailsEIMExt as a list of to be combined data types

| Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- |
| ProblemDetails | 1 | Represents error related information. |  |
| CommonEASBinding | 1 | Represents the existing common EAS binding information for the application group. |  |

#### 9.3.6.5	Binary data

##### 9.3.6.5.1	Binary Data Types

Table 9.3.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 9.3.7	Error Handling

#### 9.3.7.1	General

For the Eecs_EASInfoManagement API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [6]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [6] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [6].

In addition, the requirements in the following clauses are applicable for the Eecs_EASInfoManagement API.

#### 9.3.7.2	Protocol Errors

No specific protocol errors for the Eecs_EASInfoManagement API are specified.

#### 9.3.7.3	Application Errors

The application errors defined for the Eecs_EASInfoManagement API are listed in Table 9.3.7.3-1.

Table 9.3.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| EXISTING_COMMON_EAS | 403 Forbidden | The common EAS binding information registration request is rejected because there is an existing Common EAS Binding information for the concerned application group. |  |

### 9.3.8	Feature negotiation

The optional features in table 9.3.8-1 are defined for the Eecs_EASInfoManagement API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [6].

Table 9.3.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | EdgeApp_3 | This feature indicates the support of the second set of enhancements to the Edge Applications Enabler Layer. Within this feature, the following enhancements are covered: - Support of common EAS bundle information during Common EAS Binding information retrieval and storage. - Support of conveying the EAS Service KPIs during Common EAS Binding information retrieval and storage. - Support of conveying the EDN information during Common EAS Binding information retrieval. |

## 9.4	Eecs_ECSServiceProvisioning API

### 9.4.1	Introduction

The Eecs_ECSServiceProvisioning service shall use the Eecs_ECSServiceProvisioning API.

The API URI of the Eecs_ECSServiceProvisioning Service API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [6], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

-	The <apiName> shall be "eecs-esp".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

NOTE:	When 3GPP TS 29.122 [6] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 9.4, the ECS takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

### 9.4.2	Usage of HTTP

The provisions of clause 5.2 of 3GPP TS 29.122 [6] shall apply for the Eecs_ECSServiceProvisioning API.

### 9.4.3	Resources

#### 9.4.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 9.4.3.1-1 depicts the resource URIs structure for the Eecs_ECSServiceProvisioning API.

Figure 9.4.3.1-1: Resource URIs structure of the Eecs_ECSServiceProvisioning API

Table 9.4.3.1-1 provides an overview of the resources and applicable HTTP methods for the Eecs_ECSServiceProvisioning API.

Table 9.4.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Service Provisioning Subscriptions | /subscriptions | POST | Request the creation of a Service Provisioning Subscription. |
| Individual Service Provisioning Subscription | /sbscriptions/{subscId} | GET | Retrieve an existing "Individual Service Provisioning Subscription" resource. |
|  |  | PUT | Request the update of an existing "Individual Service Provisioning Subscription" resource. |
|  |  | PATCH | Request the modification of an existing "Individual Service Provisioning Subscription" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual Service Provisioning Subscription" resource. |

#### 9.4.3.2	Resource: Service Provisioning Subscriptions

##### 9.4.3.2.1	Description

This resource represents the collection of Service Provisioning Subscriptions managed by the ECS.

##### 9.4.3.2.2	Resource Definition

Resource URI: {apiRoot}/eecs-esp/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 9.4.3.2.2-1.

Table 9.4.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 9.4.1. |

##### 9.4.3.2.3	Resource Standard Methods

###### 9.4.3.2.3.2	POST

The HTTP POST method allows a service consumer to request the creation of a Service Provisioning Subscription at the ECS.

This method shall support the URI query parameters specified in table 9.4.3.2.3.2-1.

Table 9.4.3.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 9.4.3.2.3.2-2 and the response data structures and response codes specified in table 9.4.3.2.3.2-3.

Table 9.4.3.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServProvSubsc | M | 1 | Represents the parameters to request the creation of a Service Provisioning Subscription. |

Table 9.4.3.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServProvSubsc | M | 1 | 201 Created | Successful case. The Service Provisioning Subscription is successfully created and a representation of the created "Individual Service Provisioning Subscription" resource shall be returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.4.3.2.3.2-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/eecs-esp/<apiVersion>/subscriptions/{subscId} |

##### 9.4.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 9.4.3.3	Resource: Individual Service Provisioning Subscription

##### 9.4.3.3.1	Description

This resource represents a Service Provisioning Subscription managed by the ECS.

##### 9.4.3.3.2	Resource Definition

Resource URI: {apiRoot}/eecs-esp/<apiVersion>/subscriptions/{subscId}

This resource shall support the resource URI variables defined in table 9.4.3.3.2-1.

Table 9.4.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 9.4.1. |
| subscId | string | Represents the identifier of the "Individual Service Provisioning Subscription" resource. |

##### 9.4.3.3.3	Resource Standard Methods

###### 9.4.3.3.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing "Individual Service Provisioning Subscription" resource at the ECS.

This method shall support the URI query parameters specified in table 9.4.3.3.3.1-1.

Table 9.4.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 9.4.3.3.3.1-2 and the response data structures and response codes specified in table 9.4.3.3.3.1-3.

Table 9.4.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 9.4.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServProvSubsc | M | 1 | 200 OK | Successful case. The requested "Individual Service Provisioning Subscription" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.4.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

Table 9.4.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

###### 9.4.3.3.3.2	PUT

The HTTP PUT method allows a service consumer to request the update of an existing "Individual Service Provisioning Subscription" resource at the ECS.

This method shall support the URI query parameters specified in table 9.4.3.3.3.2-1.

Table 9.4.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 9.4.3.3.3.2-2 and the response data structures and response codes specified in table 9.4.3.3.3.2-3.

Table 9.4.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServProvSubsc | M | 1 | Represents the updated representation of the "Individual Service Provisioning Subscription" resource. |

Table 9.4.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServProvSubsc | M | 1 | 200 OK | Successful case. The "Individual Service Provisioning Subscription" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Service Provisioning Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.4.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

Table 9.4.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

###### 9.4.3.3.3.3	PATCH

The HTTP PATCH method allows a service consumer to request the modification of an existing "Individual Service Provisioning Subscription" resource at the ECS.

This method shall support the URI query parameters specified in table 9.4.3.3.3.3-1.

Table 9.4.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 9.4.3.3.3.3-2 and the response data structures and response codes specified in table 9.4.3.3.3.3-3.

Table 9.4.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServProvSubscPatch | M | 1 | Represents the parameters to request the modification of the "Individual Service Provisioning Subscription" resource. |

Table 9.4.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServProvSubsc | M | 1 | 200 OK | Successful case. The "Individual Service Provisioning Subscription" resource is successfully modified and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Service Provisioning Subscription" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.4.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

Table 9.4.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

###### 9.4.3.3.3.4	DELETE

The HTTP DELETE method allows a service consumer to request the deletion of an existing "Individual Service Provisioning Subscription" resource at the ECS.

This method shall support the URI query parameters specified in table 9.4.3.3.3.4-1.

Table 9.4.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 9.4.3.3.3.4-2 and the response data structures and response codes specified in table 9.4.3.3.3.4-3.

Table 9.4.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 9.4.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual Service Provisioning Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.4.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

Table 9.4.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

### 9.4.4	Custom Operations without associated resources

#### 9.4.4.1	Overview

The structure of the custom operation URIs of the Eecs_ECSServiceProvisioning API is shown in Figure 9.4.4.1-1.

Figure 9.4.4.1-1: Custom operation URI structure of the Eecs_ECSServiceProvisioning API

Table 9.4.4.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the Eecs_ECSServiceProvisioning API.

Table 9.4.4.1-1: Custom operations without associated resources

| Custom operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Request | /request | POST | Enables a service consumer to request service provisioning information. |

The custom operations shall support the URI variables defined in table 9.4.4.1-2.

Table 9.4.4.1-2: URI variables for this custom operation

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 9.4.1. |

#### 9.4.4.2	Operation: Request

##### 9.4.4.2.1	Description

The custom operation enables a service consumer to request service provisioning information to the ECS.

##### 9.4.4.2.2	Operation Definition

This operation shall support the request data structures specified in table 9.4.4.2.2-1 and the response data structures and response codes specified in table 9.4.4.2.2-2.

Table 9.4.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServProvReq | M | 1 | Contains the parameters to request service provisioning information. |

Table 9.4.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServProvResp | M | 1 | 200 OK | Successful case. The requested service provisioning information shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The requested service provisioning information does not exist. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6] |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.4.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative ECS. |

Table 9.4.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative ECS. |

### 9.4.5	Notifications

#### 9.4.5.1	General

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [6].

Table 9.4.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Service Provisioning Notification | {notifUri} | POST | This service operation enables an ECS to notify a previously subscribed service consumer on service provisioning event(s). |

#### 9.4.5.2	Service Provisioning Notification

##### 9.4.5.2.1	Description

The Service Provisioning Notification is used by the ECS to notify a previously subscribed service consumer on service provisioning event(s).

##### 9.4.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 9.4.5.2.2-1.

Table 9.4.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Represents the callback URI encoded as a string formatted as a URI. |

##### 9.4.5.2.3	Standard Methods

9.4.5.2.3.1	POST

This method shall support the request data structures specified in table 9.4.5.2.3.1-1 and the response data structures and response codes specified in table 9.4.5.2.3.1-2.

Table 9.4.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServProvNotif | M | 1 | Represents the Service Provisioning Notification. |

Table 9.4.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Service Provisioning Notification is successfully received and processed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.4.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

Table 9.4.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

### 9.4.6	Data Model

#### 9.4.6.1	General

This clause specifies the application data model supported by the API.

Table 9.4.6.1-1 specifies the data types defined for the Eecs_ECSServiceProvisioning API.

Table 9.4.6.1-1: Eecs_ECSServiceProvisioning API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AppGrpProfile | 9.4.6.2.9 | Represents the application group profile. | EdgeApp_3 |
| AppInfo | 9.4.6.2.8 | Represents the application information. | EdgeApp_3 |
| FederationInfo | 9.4.6.2.7 | Represents federation agreements related information. |  |
| ServProvNotif | 9.4.6.2.6 | Represents a Service Provisioning Notification. |  |
| ServProvReq | 9.4.6.2.2 | Represents a Service Provisioning information retrieval request. |  |
| ServProvResp | 9.4.6.2.3 | Represents a Service Provisioning information retrieval response. |  |
| ServProvSubsc | 9.4.6.2.4 | Represents a Service Provisioning Subscription. |  |
| ServProvSubscPatch | 9.4.6.2.5 | Represents the requested modifications to a Service Provisioning Subscription. |  |

Table 9.4.6.1-2 specifies data types re-used by the Eecs_ECSServiceProvisioning API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the Eecs_ECSServiceProvisioning API.

Table 9.4.6.1-2: Eecs_ECSServiceProvisioning API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ACProfile | 3GPP TS 24.558 [14] | Represents an AC profile. |  |
| ConnectivityInfo | 3GPP TS 24.558 [14] | Represents connectivity information. |  |
| DateTime | 3GPP TS 29.122 [6] | Represents a date and a time. |  |
| DurationSec | 3GPP TS 29.122 [6] | Represents a time duration in seconds. |  |
| EDNConfigInfo | 3GPP TS 24.558 [14] | Represents EDN related configuration information. |  |
| GeographicArea | 3GPP TS 29.572 [11] | Represents the geographical information. | EdgeApp_3 |
| LocationInfo | 3GPP TS 29.122 [6] | Represents location information. |  |
| SupportedFeatures | 3GPP TS 29.571 [18] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| Uinteger | 3GPP TS 29.571 [8] | Represents an unsigned integer. | EdgeApp_3 |
| Uri | 3GPP TS 29.122 [6] | Represents the URI. |  |

#### 9.4.6.2	Structured data types

##### 9.4.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

##### 9.4.6.2.2	Type: ServProvReq

Table 9.4.6.2.2-1: Definition of type ServProvReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| fedInfo | array(FederationInfo) | O | 1..N | Contains federation agreements related information. |  |
| acProfs | array(ACProfile) | O | 1..N | Contains the AC profile(s) information indicating the required service(s). (NOTE 1) |  |
| appInfos | array(AppInfo) | O | 1..N | Contains the list of application information. (NOTE 1, NOTE 2) | EdgeApp_3 |
| connInfo | array(ConnectivityInfo) | O | 1..N | Contains the set(s) of connectivity information where the services are required. |  |
| locInfo | LocationInfo | O | 0..1 | Contains the location information of the concerned UE. (NOTE 2) |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 9.4.8. This attribute shall be present only when feature negotiation needs to take place. |  |
| NOTE 1: When the feature "EdgeApp_3" feature is supported, only one of these attributes may be present. NOTE 2: When the "appInfos" attribute is present, then the "expGeoAreas" attribute within the "appInfos" attribute should take precedence over the "locInfo" attribute. |  |  |  |  |  |

##### 9.4.6.2.3	Type: ServProvResp

Table 9.4.6.2.3-1: Definition of type ServProvResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ednConfigInfo | array(EDNConfigInfo) | M | 1..N | Contains the list of EDN configuration information. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 9.4.8. This attribute shall be present only when feature negotiation needs to take place. |  |

##### 9.4.6.2.4	Type: ServProvSubsc

Table 9.4.6.2.4-1: Definition of type ServProvSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | M | 1 | Contains the URI via which the service provisioning event(s) notifications shall be delivered. |  |
| fedInfo | array(FederationInfo) | O | 1..N | Contains list of federation agreements related information. |  |
| acProfs | array(ACProfile) | O | 1..N | Contains the AC profile(s) information indicating the required service(s). |  |
| connInfo | array(ConnectivityInfo) | O | 1..N | Contains the set(s) of connectivity information where the services are required. |  |
| locInfo | LocationInfo | O | 0..1 | Contains the location information of the concerned UE. |  |
| expTime | DateTime | O | 0..1 | Contains the expiration time of the subscription. If this attribute is not present, this means that the subscription does not expires until explicitly terminated by the service consumer. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 9.4.8. This attribute shall be present only when feature negotiation needs to take place. |  |

##### 9.4.6.2.5	Type: ServProvSubscPatch

Table 9.4.6.2.5-1: Definition of type ServProvSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | O | 0..1 | Contains the updated URI via which the service provisioning event(s) notifications shall be delivered. |  |
| fedInfo | array(FederationInfo) | O | 1..N | Contains the updated list of federation agreements related information. |  |
| acProfs | array(ACProfile) | O | 1..N | Contains the updated AC profile(s) information indicating the required services. |  |
| connInfo | array(ConnectivityInfo) | O | 1..N | Contains the updated set(s) of connectivity information where the services are required. |  |
| locInfo | LocationInfo | O | 0..1 | Contains the updated location information of the concerned UE. |  |
| expTime | DateTime | O | 0..1 | Contains the updated expiration time of the subscription. |  |

##### 9.4.6.2.6	Type: ServProvNotif

Table 9.4.6.2.6-1: Definition of type ServProvNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subscId | string | M | 1 | Contains the identifier of the subscription to which the notification is related. |  |
| ednConfigInfo | array(EDNConfigInfo) | M | 1..N | Contains the list of EDN configuration information. |  |
| lifetime | DurationSec | O | 0..1 | Contains the time duration that the the provided EDN configuration information can be cached. |  |

##### 9.4.6.2.7	Type: FederationInfo

Table 9.4.6.2.7-1: Definition of type FederationInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| federationId | string | M | 1 | Contains the identifier of the federation. |  |

##### 9.4.6.2.8	Type: AppInfo

Table 9.4.6.2.8-1: Definition of type AppInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| acProf | ACProfile | M | 1 | Contains the AC profile information indicating the required service(s). |  |
| appGrpProf | AppGrpProfile | O | 0..1 | Contains the application group profile associated with the AC profile. |  |

##### 9.4.6.2.9	Type: AppGrpProfile

Table 9.4.6.2.9-1: Definition of type AppGrpProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appGrpId | string | M | 1 | Contains the targeted application group identifier. |  |
| e2eRespTime | Uinteger | O | 0..1 | Contains the required response time for end to end communications between ACs on two UEs via EAS(s), expressed in units of milliseconds. |  |
| easId | string | M | 1 | Contains the identifier of the EAS. |  |
| expGeoAreas | array(GeographicArea) | O | 1..N | Contains the expected location(s) (e.g., route) of the group of UE(s) during the AC's operation schedule. |  |

#### 9.4.6.3	Simple data types and enumerations

##### 9.4.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 9.4.6.3.2	Simple data types

The simple data types defined in table 9.4.6.3.2-1 shall be supported.

Table 9.4.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 9.4.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 9.4.6.5	Binary data

##### 9.4.6.5.1	Binary Data Types

Table 9.4.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 9.4.7	Error Handling

#### 9.4.7.1	General

For the Eecs_ECSServiceProvisioning API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [6]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [6] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [6].

In addition, the requirements in the following clauses are applicable for the Eecs_ECSServiceProvisioning API.

#### 9.4.7.2	Protocol Errors

No specific protocol errors for the Eecs_ECSServiceProvisioning API are specified.

#### 9.4.7.3	Application Errors

The application errors defined for the Eecs_ECSServiceProvisioning API are listed in Table 9.4.7.3-1.

Table 9.4.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

### 9.4.8	Feature negotiation

The optional features listed in table 9.4.8-1 are defined for the Eecs_ECSServiceProvisioning API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [6].

Table 9.4.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | EdgeApp_3 | This feature indicates the support of the second set of enhancements to the Edge Applications Enabler Layer. Within this feature, the following enhancements are covered: - Support the retrieval of common EAS based on application group profile. |

### 9.4.9	Security

The provisions of clause 6 of 3GPP TS 29.122 [6] shall apply for the Eecs_ECSServiceProvisioning API.

## 9.5	Eecs_ECSDiscovery API

### 9.5.1	Introduction

The Eecs_ECSDiscovery service shall use the Eecs_ECSDiscovery API.

The API URI of the Eecs_ECSDiscovery API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [6], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

-	The <apiName> shall be "eecs-ecsdiscovery".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

### 9.5.2	Resources

#### 9.5.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 9.5.2.1-1 depicts the resource URIs structure for the Eecs_ECSDiscovery API.

Figure 9.5.2.1-1: Resource URI structure of the Eecs_ECSDiscovery API

Table 9.5.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 9.5.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| ECS Information | /ecs-info | Discover | Retrieve partner ECS information. |

#### 9.5.2.2	Resource: ECS Information

##### 9.5.2.2.1	Description

This resource represents the collection of ECS Information managed by the ECS.

##### 9.5.2.2.2	Resource Definition

Resource URI: {apiRoot}/eecs-ecsdiscovery/<apiVersion>/ecs-info

This resource shall support the resource URI variables defined in the table 9.5.2.2.2-1.

Table 9.5.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 9.5.1. |

##### 9.5.2.2.3	Resource Standard Methods

There are no resource standard methods defined for this resource in this release of the specification.

##### 9.5.2.2.4	Resource Custom Operations

###### 9.5.2.2.4.1	Overview

Table 9.5.2.4.4.1-1 specifies the custom operations defined on this resource.

Table 9.5.2.2.4.1-1: Custom operations

| Operation name | Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Discover | /ecs-info/discover | POST | Request partner ECS information |

###### 9.5.2.2.4.2	Operation: Discover

9.5.2.2.4.2.1	Description

The custom operation allows a service consumer to request ECS information from the ECS.

9.5.2.2.4.2.2	Operation Definition

This operation shall support the request of data structures specified in table 9.5.2.2.4.2.2-1 and the response data structure and response codes specified in table 9.5.2.2.4.2.2-2.

Table 9.5.2.2.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ECSInfoDiscoveryReq | M | 1 | Contains the necessary information to request ECS information. |

Table 9.5.2.2.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ECSInfoDiscoveryResp | M | 1 | 200 OK | The requested ECS discovery information was successfully returned. |
| n/a |  |  | 204 No Content | The processing of the request is successful but no matching ECS was found. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource custom operation located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource custom operation located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 9.5.2.2.4.2.2-3: Headers supported by the 307 Response Code on this resource custom operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource custom operation located in an alternative ECS. |

Table 9.5.2.2.4.2.2-4: Headers supported by the 308 Response Code on this resource custom operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource custom operation located in an alternative ECS. |

### 9.5.3	Custom Operations without associated resources

There are no custom Operations without associated resources defined for this API in this release of the specification.

### 9.5.4	Notifications

#### 9.5.4.1	General

Table 9.5.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| ECS Discovery Notification | {notifUri} | POST | Enables the ECS to notify a previously subscribed service consumer on partner ECS information. |

#### 9.5.4.2	ECS Discovery Notification

##### 9.5.4.2.1	Description

The ECS Discovery Notification service operation is used by the ECS to notify a previously subscribed service consumer on partner ECS information.

##### 9.5.4.2.2	Target URI

The callback URI {notifUri} shall be used with the callback URI variables defined in table 9.5.4.2.2-1.

Table 9.5.4.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notifUri | Uri | Represents the callback URI encoded as a string formatted as a URI. |

##### 9.5.4.2.3	Standard Methods

###### 9.5.4.2.3.1	POST

This method shall support the request data structures specified in table 9.5.4.2.3.1-1 and the response data structures and response codes specified in table 9.5.4.2.3.1-2.

Table 9.5.4.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EcsInfoDiscNotif | M | 1 | Notification of ECS information. |

Table 9.5.4.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI representing the end point of an alternative service consumer towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.5.4.4.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

Table 9.5.4.4.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

### 9.5.5	Data Model

#### 9.5.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API.

Table 9.5.5.1-1 specifies the data types defined specifically for the Eecs_ECSDiscovery API.

Table 9.5.5.1-1: Eecs_ECSDiscovery API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| EcsInfo | 9.5.5.2.4 | Represents the discovered ECS information. |  |
| EcsInfoDiscNotif | 9.5.5.2.9 | Represents the ECS Discovery notification information |  |
| EcsInfoDiscoveryReq | 9.5.5.2.2 | Represents the ECS Discovery request information. |  |
| EcsInfoDiscoveryResp | 9.5.5.2.3 | Represents the ECS Discovery response information. |  |
| ECSProfile | 9.5.5.2.5 | Represents the profile information related to the ECS in the ECSRegistration data type. |  |
| PduConfiguration | 9.5.5.2.8 | Represents information to establish PDU session with ECS. |  |
| SupportedEcsp | 9.5.5.2.7 | Represents the ECSP Information associated to PLMN. |  |
| SupportedPlmn | 9.5.5.2.6 | Represents the PLMN information. |  |

Table 9.5.5.1-2 specifies data types re-used by the Eecs_ECSDiscovery API service, including a reference to their respective specifications, and when needed, a short description of their use within the Eecs_ECSDiscovery API.

Table 9.5.5.1-2: Eecs_ECSDiscovery re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ACProfile | 3GPP TS 24.558 [14] | Represents the AC profiles filter information. |  |
| ConnectivityInfo | 3GPP TS 24.558 [14] | Represents to represent the connectivity information of the UE. |  |
| DateTime | 3GPP TS 29.122 [6] | Represents a date and a time. |  |
| Dnn | 3GPP TS 29.571 [8] | Represents the Dnn information |  |
| EndPoint | 8.1.5.2.5 | Represents the end point information of the ECS in the ECS profile. |  |
| LocationInfo | 3GPP TS 29.122 [6] | Represents the location information of the UE in the ECS discovery request and subscriptions. |  |
| PlmnIdNid | 3GPP TS 29. 571 [8] | Represents the network, i.e., PLMN Identifier or the SNPN Identifier (the PLMN Identifier and the NID). Used to indicate the PLMN supporting information in ECS profile. |  |
| Snssai | 3GPP TS 29.571 [8] | Represents the Snssai information. |  |
| SpatialValidityCond | 3GPP TS 29.571 [8] | Represents the spatial validity conditions. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| Uri | 3GPP TS 29.122 [6] | Represents a URI. |  |

#### 9.5.5.2	Structured data types

##### 9.5.5.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 9.5.5.2.2	Type: EcsInfoDiscoveryReq

Table 9.5.5.2.2-1: Definition of type EcsInfoDiscoveryReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ecsAddr | EndPoint | M | 1 | Represents end point information (e.g. URI, FQDN, IP address) of the requesting ECS. |  |
| acProfs | array(ACProfile) | O | 1..N | AC Profiles filter information about the required services. |  |
| connInf | array(ConnectivityInfo) | O | 1..N | List of connectivity information for the UE. |  |
| ueLoc | array(LocationInfo) | O | 1..N | Location information of the UE for which the services are required. (NOTE) |  |
| notifUri | Uri | O | 0..1 | Contains the URI via which the ECS Discovery notifications shall be delivered. |  |
| duration | DateTime | O | 0..1 | Represents the time duration over which the service consumer expects the ECS to provide ECS Discovery notifications. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents a list of Supported features used as described in clause 9.5.7. This attribute shall be present only when feature negotiation needs to take place. |  |
| appGrpId | string | O | 0..1 | Contains the application group identifier. | EdgeApp_3 |
| NOTE: In this release of the specification, only one element within the array data structure may be provided. |  |  |  |  |  |

##### 9.5.5.2.3	Type: EcsInfoDiscoveryResp

Table 9.5.5.2.3-1: Definition of type EcsInfoDiscoveryResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ecsInfo | array(EcsInfo) | M | 1..N | Contains the list of partner ECS information. |  |

##### 9.5.5.2.4	Type: EcsInfo

Table 9.5.5.2.4-1: Definition of type EcsInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ecs | ECSProfile | M | 1 | Contains the ECS profile information. |  |
| lifeTime | DateTime | O | 0..1 | Indicates the time duration for which the provided ECS information is valid. |  |

##### 9.5.5.2.5	Type: ECSProfile

Table 9.5.5.2.5-1: Definition of type ECSProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| endPt | EndPoint | M | 1 | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the ECS. |  |
| ecspId | string | O | 0..1 | The identifier of the ECSP (e.g. the mobile network operator or 3rd party service provider) that provides the ECS. |  |
| splVal | SpatialValidityCond | O | 0..1 | The spatial validity conditions. |  |
| suppPlmns | array(SupportedPlmn) | O | 1..N | List of PLMNs and the associated ECSPs for which the ECS can provide the EDN configuration information. |  |

##### 9.5.5.2.6	Type: SupportedPlmn

Table 9.5.5.2.6-1: Definition of type SupportedPlmn

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| plmnId | PlmnIdNid | O | 0..1 | The identifier of the PLMN for which EDN configuration information can be provided by ECS. |  |
| suppEcsps | array(SupportedEcsp) | O | 1..N | The information of ECSP(s) associated to the PLMN identified by "plmnId" attribute. |  |
| pduConf | PduConfiguration | O | 0..1 | DNN and S-NSSAI information for roaming UEs to establish PDU sessions with the ECS. |  |

##### 9.5.5.2.7	Type: SupportedEcsp

Table 9.5.5.2.7-1: Definition of type SupportedEcsp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ecspId | string | M | 1 | The identifier of an ECSP |  |
| easIds | array(string) | M | 1..N | The list of EAS IDs available or expected to be available through the ECSP identified by "ecspId" attribute. |  |

##### 9.5.5.2.8	Type: PduConfiguration

Table 9.5.5.2.8-1: Definition of type PduConfiguration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| snssai | Snssai | M | 1 | Indicates the S-NSSAI information to establish PDU sessions with the ECS. |  |
| dnn | Dnn | M | 1 | Indicates the DNN information to establish PDU sessions with the ECS. |  |

##### 9.5.5.2.9	Type: EcsInfoDiscNotif

Table 9.5.5.2.9-1: Definition of type EcsInfoDiscNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ecsInfo | array(EcsInfo) | M | 1..N | Contains the list of partner ECS information. |  |

#### 9.5.5.3	Simple data types and enumerations

##### 9.5.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 9.5.5.3.2	Simple data types

The simple data types defined in table 9.5.5.3.2-1 shall be supported.

Table 9.5.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 9.5.5.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 9.5.5.5	Binary data

##### 9.5.5.5.1	Binary Data Types

Table 9.5.5.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 9.5.6	Error Handling

#### 9.5.6.1	General

For the Eecs_ECSDiscovery API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [3]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [3] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [3].

In addition, the requirements in the following clauses are applicable for the Eecs_ECSDiscovery API.

#### 9.5.6.2	Protocol Errors

No specific protocol errors for the Eecs_ECSDiscovery API are specified.

#### 9.5.6.3	Application Errors

The application errors defined for the Eecs_ECSDiscovery service are listed in Table 9.5.6.3-1.

Table 9.5.6.1-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

### 9.5.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 9.5.7-1 lists the supported features for Eecs_ECSDiscovery API.

Table 9.5.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | EdgeApp_3 | This feature indicates the support of the second set of enhancements to the Edge Applications Enabler Layer. Within this feature, the following enhancements are covered: - Support of providing the application group identifier in the ECS Information discovery request. |

## 9.6	Eecs_ACREvents API

### 9.6.1	Introduction

The Eecs_ACREvents service shall use the Eecs_ACREvents API.

The API URI of the Eecs_ACREvents Service API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [6], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

-	The <apiName> shall be "eecs-ae".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [6].

### 9.6.2	Usage of HTTP

The provisions of clause 5.2.2 of 3GPP TS 29.122 [6] shall apply for the Eecs_ACREvents API.

### 9.6.3	Resources

#### 9.6.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 9.6.3.1-1 depicts the resource URIs structure for the Eecs_ACREvents API.

Figure 9.6.3.1-1: Resource URIs structure of the Eecs_ACREvents API

Table 9.6.3.1-1 provides an overview of the resources and applicable HTTP methods for the Eecs_ACREvents API.

Table 9.6.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| ACR Events Subscriptions | /subscriptions | POST | Request the creation of a ACR Events Subscription. |
| Individual ACR Events Subscription | /sbscriptions/{subscriptionId} | GET | Retrieve an existing "Individual ACR Events Subscription" resource. |
|  |  | PUT | Request the update of an existing "Individual ACR Events Subscription" resource. |
|  |  | PATCH | Request the modification of an existing "Individual ACR Events Subscription" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual ACR Events Subscription" resource. |

#### 9.6.3.2	Resource: ACR Events Subscriptions

##### 9.6.3.2.1	Description

This resource represents the collection of ACR Events Subscriptions managed by the ECS.

##### 9.6.3.2.2	Resource Definition

Resource URI: {apiRoot}/eecs-ae/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 9.6.3.2.2-1.

Table 9.6.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 9.6.1. |

##### 9.6.3.2.3	Resource Standard Methods

###### 9.6.3.2.3.2	POST

The HTTP POST method allows a service consumer to request the creation of a ACR Events Subscription at the ECS.

This method shall support the URI query parameters specified in table 9.6.3.2.3.2-1.

Table 9.6.3.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 9.6.3.2.3.2-2 and the response data structures and response codes specified in table 9.6.3.2.3.2-3.

Table 9.6.3.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AcrEventsSubsc | M | 1 | Represents the parameters to request the creation of a ACR Events Subscription. |

Table 9.6.3.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AcrEventsSubsc | M | 1 | 201 Created | Successful case. The ACR Events Subscription is successfully created and a representation of the created "Individual ACR Events Subscription" resource shall be returned. An HTTP Location header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.6.3.2.3.2-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/eecs-ae/<apiVersion>/subscriptions/{subscriptionId} |

##### 9.6.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 9.6.3.3	Resource: Individual ACR Events Subscription

##### 9.6.3.3.1	Description

This resource represents a ACR Events Subscription managed by the ECS.

##### 9.6.3.3.2	Resource Definition

Resource URI: {apiRoot}/eecs-ae/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 9.6.3.3.2-1.

Table 9.6.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 9.6.1. |
| subscriptionId | string | Represents the identifier of the "Individual ACR Events Subscription" resource. |

##### 9.6.3.3.3	Resource Standard Methods

###### 9.6.3.3.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing "Individual ACR Events Subscription" resource at the ECS.

This method shall support the URI query parameters specified in table 9.6.3.3.3.1-1.

Table 9.6.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 9.6.3.3.3.1-2 and the response data structures and response codes specified in table 9.6.3.3.3.1-3.

Table 9.6.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 9.6.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AcrEventsSubsc | M | 1 | 200 OK | Successful case. The requested "Individual ACR Events Subscription" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.6.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

Table 9.6.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

###### 9.6.3.3.3.2	PUT

The HTTP PUT method allows a service consumer to request the update of an existing "Individual ACR Events Subscription" resource at the ECS.

This method shall support the URI query parameters specified in table 9.6.3.3.3.2-1.

Table 9.6.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 9.6.3.3.3.2-2 and the response data structures and response codes specified in table 9.6.3.3.3.2-3.

Table 9.6.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AcrEventsSubsc | M | 1 | Represents the updated representation of the "Individual ACR Events Subscription" resource. |

Table 9.6.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AcrEventsSubsc | M | 1 | 200 OK | Successful case. The "Individual ACR Events Subscription" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual ACR Events Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.6.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

Table 9.6.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

###### 9.6.3.3.3.3	PATCH

The HTTP PATCH method allows a service consumer to request the modification of an existing "Individual ACR Events Subscription" resource at the ECS.

This method shall support the URI query parameters specified in table 9.6.3.3.3.3-1.

Table 9.6.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 9.6.3.3.3.3-2 and the response data structures and response codes specified in table 9.6.3.3.3.3-3.

Table 9.6.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AcrEventsSubscPatch | M | 1 | Represents the parameters to request the modification of the "Individual ACR Events Subscription" resource. |

Table 9.6.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AcrEventsSubsc | M | 1 | 200 OK | Successful case. The "Individual ACR Events Subscription" resource is successfully modified and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual ACR Events Subscription" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.6.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

Table 9.6.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

###### 9.6.3.3.3.4	DELETE

The HTTP DELETE method allows a service consumer to request the deletion of an existing "Individual ACR Events Subscription" resource at the ECS.

This method shall support the URI query parameters specified in table 9.6.3.3.3.4-1.

Table 9.6.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 9.6.3.3.3.4-2 and the response data structures and response codes specified in table 9.6.3.3.3.4-3.

Table 9.6.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 9.6.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual ACR Events Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ECS. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.6.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

Table 9.6.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ECS. |

##### 9.6.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 9.6.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this resource in this release of the specification.

### 9.6.5	Notifications

#### 9.6.5.1	General

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [6].

Table 9.6.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| ACR Events Notification | {notifUri} | POST | This service operation enables an ECS to notify a previously subscribed service consumer on ACR event(s). |

#### 9.6.5.2	ACR Events Notification

##### 9.6.5.2.1	Description

The ACR Events Notification is used by the ECS to notify a previously subscribed service consumer on ACR related event(s).

##### 9.6.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 9.6.5.2.2-1.

Table 9.6.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Represents the callback URI encoded as a string formatted as a URI. |

##### 9.6.5.2.3	Standard Methods

9.6.5.2.3.1	POST

This method shall support the request data structures specified in table 9.6.5.2.3.1-1 and the response data structures and response codes specified in table 9.6.5.2.3.1-2.

Table 9.6.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AcrEventsNotif | M | 1 | Represents the ACR Events Notification. |

Table 9.6.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The ACR Events Notification is successfully received and processed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [6] shall also apply. |  |  |  |  |

Table 9.6.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

Table 9.6.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

### 9.6.6	Data Model

#### 9.6.6.1	General

This clause specifies the application data model supported by the API.

Table 9.6.6.1-1 specifies the data types defined for the Eecs_ACREvents API.

Table 9.6.6.1-1: Eecs_ACREvents API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AcrEventsNotif | 9.6.6.2.4 | Represents an ACR Events Notification. |  |
| AcrEventsSubsc | 9.6.6.2.2 | Represents an ACR Events Subscription. |  |
| AcrEventsSubscPatch | 9.6.6.2.3 | Represents the requested modifications to an ACR Events subscription. |  |
| AcrEvent | 9.6.6.3.3 | Represents the ACR event. |  |

Table 9.6.6.1-2 specifies data types re-used by the Eecs_ACREvents API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Eecs_ACREvents API.

Table 9.6.6.1-2: Eecs_ACREvents API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.122 [6] | Represents a date and a time. |  |
| DateTimeRm | 3GPP TS 29.571 [8] | Represents the same as the DateTime data type but with the OpenAPI "nullable:true" property. |  |
| EDNInfo | Clause 9.1.5.2.10 | Represent EDN related information. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| Uinteger | 3GPP TS 29.571 [8] | Used to express the maximum response time of EAS service KPI. |  |
| Uri | 3GPP TS 29.122 [6] | Represents a URI. |  |

#### 9.6.6.2	Structured data types

##### 9.6.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 9.6.6.2.2	Type: AcrEventsSubsc

Table 9.6.6.2.2-1: Definition of type AcrEventsSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | string | M | 1 | Contains the identifier of the service consumer that is sending the request. |  |
| events | array(AcrEvent) | M | 1..N | Contains the subscribed ACR Event(s). |  |
| ednInfo | EDNInfo | O | 0..1 | Contains the EDN configuration information. This attribute may be present only when the subscribed event is "EDN_OVERLOAD". |  |
| notifUri | Uri | M | 1 | Contains the URI via which the ACR event(s) notifications shall be delivered. |  |
| expTime | DateTime | O | 0..1 | Contains the proposed expiration time of the subscription. If this attribute is not present, this means that the subscription does not expires until explicitly terminated by the service consumer. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported feature(s) among the ones defined in clause 9.6.8. This attribute shall be present only when feature negotiation needs to take place. |  |

##### 9.6.6.2.3	Type: AcrEventsSubscPatch

Table 9.6.6.2.3-1: Definition of type AcrEventsSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| events | array(AcrEvent) | O | 1..N | Contains the ACR event(s) the subscription is related. |  |
| ednInfo | EDNInfo | O | 0..1 | Contains the EDN configuration information. This attribute may be present only when the subscribed event is "EDN_OVERLOAD". |  |
| notifUri | Uri | O | 0..1 | Contains the URI via which the ACR event(s) notifications shall be delivered. |  |
| expTime | DateTimeRm | O | 0..1 | Contains the updated expiration time of the subscription. If this attribute is set to "NULL", this means that it is requested that the subscription does not expires until explicitly terminated by the service consumer. |  |

##### 9.6.6.2.4	Type: AcrEventsNotif

Table 9.6.6.2.4-1: Definition of type AcrEventsNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| events | array(AcrEvent) | O | 1..N | Contains the reported ACR event(s) the subscription is related. |  |
| subscriptionId | string | M | 1 | Contains the identifier of the subscription to which the notification is related. |  |
| relocRatio | Uinteger | C | 0..1 | Contains the relocation ratio expressed as a percentage. Minimum: 1. Maximum: 100. The attribute shall be present only when the "EDN_OVERLOAD" is present within the "events" attribute. |  |

#### 9.6.6.3	Simple data types and enumerations

##### 9.6.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 9.6.6.3.2	Simple data types

The simple data types defined in table 9.6.6.3.2-1 shall be supported.

Table 9.6.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 9.6.6.3.3	Enumeration: AcrEvent

The enumeration Event represents the ACR events. It shall comply with the events defined in table 9.6.6.3.3-1.

Table 9.6.6.3.3-1: Enumeration AcrEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| EDN_OVERLOAD | Indicates the ACR event is EDN overload. |  |

#### 9.6.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 9.6.6.5	Binary data

##### 9.6.6.5.1	Binary Data Types

Table 9.6.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 9.6.7	Error Handling

#### 9.6.7.1	General

For the Eecs_ACREvents API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [6]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [6] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [6].

In addition, the requirements in the following clauses are applicable for the Eecs_ACREvents API.

#### 9.6.7.2	Protocol Errors

No specific protocol errors for the Eecs_ACREvents API are specified.

#### 9.6.7.3	Application Errors

The application errors defined for the Eecs_ACREvents API are listed in Table 9.6.7.3-1.

Table 9.6.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

### 9.6.8	Feature negotiation

The optional features in table 9.6.8-1 are defined for the Eecs_ACREvents API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [6].

Table 9.6.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 9.6.9	Security

The provisions of clause 6 of 3GPP TS 29.122 [6] shall apply for the Eecs_ACREvents API.

# 10	Using Common API Framework

## 10.1	General

EES may expose its services to EAS with support of CAPIF. Also, the EES may also re-expose the network capabilities of the 3GPP core network to the EAS(s) with support of CAPIF architecture, as specified in 3GPP TS 23.558 [2]. When CAPIF is used with EES services, the EES shall support the following as defined in 3GPP TS 29.222 [17]:

-	the API exposing function and related APIs over CAPIF-2/2e and CAPIF-3/3e reference points;

-	the API publishing function and related APIs over CAPIF-4/4e reference point;

-	the API management function and related APIs over CAPIF-5/5e reference point; and

-	at least one of the security methods for authentication and authorization, and related security mechanisms.

The EAS supports the role of API Invoker as specified in 3GPP TS 29.222 [17]. In a centralized deployment as defined in 3GPP TS 23.222 [17], where the CAPIF core function and API provider domain functions are co-located, the interactions between the CAPIF core function and API provider domain functions may be independent of CAPIF-3/3e, CAPIF-4/4e and CAPIF-5/5e reference points.

When CAPIF is used with an EES service, the EES shall register all the features for northbound APIs in the CAPIF Core Function.

The EAS may expose its services to other EAS(s) using CAPIF as specified in 3GPP TS 23.558 [2]. When CAPIF is used for exposure of EAS services to other EAS(s), the above procedure shall be applicable with the following differences:

-	the provisions related to the EES apply to the EAS;

-	the provisions related to the EAS apply to the other EAS(s) as consumer(s) of the exposed EAS service APIs.

## 10.2	Security

When CAPIF is used for managing the exposure of the EEL APIs, before invoking an API exposed by the EEL entities (e.g., EES, EAS, ECS), the service consumer (e.g., EAS), acting as an API Invoker, shall negotiate the security method (PKI, TLS-PSK or OAUTH2) with the CAPIF Core Function and ensure that the EEL entities (e.g., EES, ECS) have enough credential to authenticate the service consumer as defined in clauses 5.6.2.2 and 6.2.2.2 of 3GPP TS 29.222 [17].

If PKI or TLS-PSK is used as the selected security method between the service consumer and the EEL entities, upon API invocation, the EEL entities shall retrieve the authorization information from the CAPIF Core Function as described in clause 5.6.2.4 of 3GPP TS 29.222 [17].

As indicated in 3GPP TS 33.122 [18], the access to the EEL APIs may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [19]), where the CAPIF Core Function (see 3GPP TS 29.222 [17]) plays the role of the authorization server.

If OAuth2 is used as the selected security method between the service consumer and the EEL entities, then the service consumer, prior to consuming services offered by the APIs exposed by the EEL entities, shall obtain a "token" from the authorization server, by invoking the Obtain_Authorization service, as described in clause 5.6.2.3.2 of 3GPP TS 29.222 [17].

The EES APIs do not define any scopes for OAuth2 authorization in the present specification. For the definition and handling of scopes for OAuth2 authorization in CAPIF, see 3GPP TS 29.222 [17].

It is the EEL entities responsibility to check whether the service consumer is authorized to use an API based on the provided "token". Once the EEL entities verifies the "token", it shall check whether the EEL entities identifier in the "token" matches its own published identifier, whether the API name in the "token" matches its own published API name and whether the granted scope (see 3GPP TS 29.222 [17]) in the "token" is authorized. If those checks are passed, the service consumer has full authority to access any resource(s) and/or operation(s) for the invoked service API and that are within the limits of the granted scope in the "token".

NOTE:	For the aforementioned security methods, the EEL entities need to apply admission control according to access control policies after performing the authorization checks.

# 11	Security

The authentication and authorization between EES and ECS shall be as specified in 3GPP TS 33.558 [20].

The authentication and authorization in EES capability exposure shall be as specified in 3GPP TS 33.558 [20]. When CAPIF is used, the aspects specified in clause 10 shall be used.

The security credentials to be used for verification and authorization of various API requests from EAS and EES shall be as specified in 3GPP TS 33.558 [20].

Annex A (normative):
OpenAPI specification

# A.1	General

This annex is based on the OpenAPI 3.0.0 specification [3] and provides corresponding representations of all APIs defined in the present specification in YAML format.

This Annex shall take precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API.

NOTE:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification file contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5B of the 3GPP TR 21.900 [4] and clause 5.3.1 of the 3GPP TS 29.501 [5] for further information).

# A.2	Eees_EASRegistration API

openapi: 3.0.0

info:

title: EES EAS Registration_API

description: |

API for EAS Registration.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: 1.2.0

externalDocs:

description: >

3GPP TS 29.558 V19.5.0 Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

servers:

- url: '{apiRoot}/eees-easregistration/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.558.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/registrations:

post:

summary: Creates a new Individual EAS Registration resource

operationId: CreateEASRegistration

tags:

- EAS Registrations (Collection)

description: Registers a new EAS at an EES.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EASRegistration'

responses:

'201':

description: >

Created. The EAS information is registered successfully at the EES.

content:

application/json:

schema:

$ref: '#/components/schemas/EASRegistration'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/registrations/{registrationId}:

get:

summary: Retrieve an existing Individual EAS Registration resource.

operationId: ReadIndEASRegistration

tags:

- Individual EAS Registration (Document)

description: Retrieve an Individual EAS registration resource.

parameters:

- name: registrationId

in: path

description: Represents the identifier of the Individual EAS Registration resource.

required: true

schema:

type: string

responses:

'200':

description: OK. The requested Individual EAS Registration resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/EASRegistration'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an existing Individual EAS Registration resource.

operationId: UpdateIndEASRegistration

tags:

- Individual EAS Registration (Document)

description: Fully replace an existing EAS Registration resource.

parameters:

- name: registrationId

in: path

description: Represents the identifier of the Individual EAS Registration resource.

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EASRegistration'

responses:

'200':

description: >

OK. The Individual EAS Registration resource is successfully updated and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/EASRegistration'

'204':

description: >

No Content. The Individual EAS Registration resource is successfully updated and no

content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an existing Individual EAS Registration resource.

operationId: ModifyIndEASRegistration

tags:

- Individual EAS Registration (Document)

description: Partially update an existing EAS Registration resource.

parameters:

- name: registrationId

in: path

description: Represents the identifier of the Individual EAS Registration resource..

required: true

schema:

type: string

requestBody:

description: Partial update of an existing EAS registration resource.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/EASRegistrationPatch'

responses:

'200':

description: >

OK. The Individual EAS Registration resource is successfully modified and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/EASRegistration'

'204':

description: >

No Content. The Individual EAS Registration resource is successfully modified and no

content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual EAS Registration resource.

operationId: DeleteIndEASRegistration

tags:

- Individual EAS Registration (Document)

description: Delete an existing EAS registration at EES.

parameters:

- name: registrationId

in: path

description: Represents the identifier of the Individual EAS Registration resource..

required: true

schema:

type: string

responses:

'204':

description: The individual EAS registration is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

EASRegistration:

type: object

description: Represents an EAS registration information.

properties:

easProf:

$ref: '#/components/schemas/EASProfile'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- easProf

EASProfile:

type: object

description: Represents the EAS profile information.

properties:

easId:

type: string

description: Identifier of the EAS.

endPt:

$ref: '#/components/schemas/EndPoint'

allowedPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

allowedPlmnIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

minItems: 1

easBdlInfos:

type: array

items:

$ref: '#/components/schemas/EASBundleInfo'

minItems: 1

acIds:

type: array

items:

type: string

minItems: 1

description: Identities of application clients that are served by the EAS.

provId:

type: string

description: Identifier of the ASP that provides the EAS.

type:

$ref: '#/components/schemas/EASCategory'

flexEasType:

type: string

description: The EAS type with flexible value set.

scheds:

type: array

items:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

description: The availability schedule of the EAS.

svcArea:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ServiceArea'

svcKpi:

$ref: '#/components/schemas/EASServiceKPI'

permLvl:

type: array

items:

$ref: '#/components/schemas/PermissionLevel'

minItems: 1

description: level of service permissions supported by the EAS.

easFeats:

type: array

items:

type: string

minItems: 1

description: Service specific features supported by EAS.

appLocs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

minItems: 1

description: List of DNAI(s) and the N6 traffic information associated with the EAS.

svcContSupp:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

minItems: 1

description: The ACR scenarios supported by the EAS for service continuity.

svcContSuppExt1:

type: array

items:

$ref: '#/components/schemas/EASBundleInfo'

minItems: 1

description: >

Represents the information related to the EAS ability to handle

bundled EAS coordinated ACRs.

This attribute may be present only when the "svcContSupp" attribute is also present.

When this attribute is present, it indicates that the EAS (identified

by the "easId" attribute) is able to handle bundled by the "easId" coordinated ACRs and

contains the information of the EAS bundle(s) for which the EAS is able to handle

bundled EAS coordinated ACRs.

transContSupp:

$ref: '#/components/schemas/TransContSuppDetails'

avlRep:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

status:

type: string

description: EAS status information.

statusVal:

$ref: '#/components/schemas/EASStatus'

genCtxDur:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

easSyncSupp:

type: boolean

default: false

description: >

Set to true to indicate that content synchronization between EASs is

supported.

Set to false to indicate that content synchronization between EASs is

not supported.

The default value when this attribute is omitted is false.

assoDevList:

type: array

items:

$ref: '#/components/schemas/AssociatedDevice'

minItems: 1

required:

- easId

- endPt

allOf:

- not:

required: [ type, flexEasType ]

- not:

required: [ status, statusVal ]

EASRegistrationPatch:

type: object

description: >

Represents the requested modification to an

EAS registration information.

properties:

easProf:

$ref: '#/components/schemas/EASProfile'

expTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTimeRm'

EASServiceKPI:

type: object

description: Represents the EAS service KPI information.

properties:

maxReqRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

maxRespTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avail:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avlComp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avlGraComp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avlMem:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avlStrg:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

connBand:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

EASBundleInfo:

type: object

description: Represents the EAS bundle information.

properties:

bdlType:

$ref: '#/components/schemas/BdlType'

bdlId:

type: string

description: Indicates a bundle ID.

easIdsList:

type: array

items:

type: string

minItems: 1

easBdlReqs:

$ref: '#/components/schemas/EASBdlReqs'

mainEasId:

type: string

required:

- bdlType

anyOf:

- required: [bdlId]

- required: [easIdsList]

EASBdlReqs:

type: object

description: Represents the EAS bundle requirements.

properties:

coordinatedEasDisc:

type: boolean

default: false

description: >

Set to true to indicate that coordianted EAS discovery is required.

Set to false to indicate that coordianted EAS discovery is not required.

The default value when this attribute is omitted is false.

coordinatedAcr:

$ref: '#/components/schemas/CoordinatedAcrReqs'

affinity:

$ref: '#/components/schemas/Affinity'

CoordinatedAcrReqs:

type: object

description: >

Represents the coordinated ACR related requirements for an EAS bundle.

properties:

coordinatedAcrInd:

type: boolean

default: false

description: >

Set to true to indicate that coordianted ACR is required.

Set to false to indicate that coordianted ACR is not required.

failureAction:

$ref: '#/components/schemas/FailureAction'

required:

- coordinatedAcrInd

EndPoint:

type: object

description: Represents the end point information.

properties:

fqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

ipv4Addrs:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

minItems: 1

description: IPv4 addresses of the edge server.

ipv6Addrs:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

minItems: 1

description: IPv6 addresses of the edge server.

uri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

oneOf:

- required: [uri]

- required: [fqdn]

- required: [ipv4Addrs]

- required: [ipv6Addrs]

AssociatedDevice:

type: object

description: Represents the associated device along with the UE.

properties:

devInfo:

type: string

devType:

$ref: '#/components/schemas/DeviceType'

anyOf:

- required: [devInfo]

- required: [devType]

PermissionLevel:

anyOf:

- type: string

enum:

- TRIAL

- GOLD

- SILVER

- OTHER

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Indicates the level of service permissions.

Possible values are:

- TRIAL: Level of service permission supported is TRIAL.

- GOLD: Level of service permission supported is GOLD.

- SILVER: Level of service permission supported is SILVER.

- OTHER: Any other level of service permissions supported.

EASCategory:

anyOf:

- type: string

enum:

- UAS

- V2X

- APP_ENABLER

- OTHER

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Indicates the category or type of the EAS.

Possible values are:

- UAS: Indicates that the EAS category is for UAS services.

- V2X: Indicates that the EAS category is for V2X services.

- APP_ENABLER: Indicates that the EAS category is for Application Enabler (e.g., SEAL)

services.

- OTHER: Indicates any other EAS category.

TransContSuppDetails:

type: object

description: >

Represents the detailed information about the EAS (e.g. SEALDD Server) capability for

seamless transport layer service continuity.

properties:

transProtocs:

type: array

items:

$ref: '#/components/schemas/TransportProtocol'

minItems: 1

description: >

Indicates the transport layer protocols supported for EAS context transfer using

the seamless transport layer service continuity capability.

required:

- transProtocs

TransportProtocol:

anyOf:

- type: string

enum:

- QUIC

- TCP

- TCP_TLS

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Indicates the transport layer protocol.

Possible values are:

- QUIC: Indicates the QUIC protocol.

- TCP: Indicates the Transmission Control (TCP) Protocol.

- TCP_TLS: Indicates the Transmission Control Protocol (TCP) with Transport Layer Security

(TLS) protocol.

BdlType:

anyOf:

- type: string

enum:

- DIRECT

- PROXY

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the EAS Bundle type.

Possible values are:

- DIRECT: Indicates that the EAS Bundle type is direct bundle.

- PROXY: Indicates that the EAS Bundle type is proxy bundle.

Affinity:

anyOf:

- type: string

enum:

- STRONG

- PREFERRED

- WEAK

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the affinity requirements of an EAS bundle.

Possible values are:

- STRONG: Indicates that the affinity is strong, i.e., all the EASs of the bundle shall be

in the same EDN.

- PREFERRED: Indicates that the affinity is preferred, i.e., it is preferred to have all the

EASs of the bundle in the same EDN, but it is not essential.

- WEAK: Indicates that the affinity is weak, i.e., it is not essential to have all the EASs

of the bundle in the same EDN.

FailureAction:

anyOf:

- type: string

enum:

- CANCEL

- PROCEED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the EAS bundle related failure action during ACR.

Possible values are:

- CANCEL: Indicates that ACR shall be cancelled for the other EAS(s) of the bundle for which

ACR is not failed.

- PROCEED: Indicates that ACR shall proceed for the other EAS(s) of the bundle for which

ACR is not failed.

EASStatus:

anyOf:

- type: string

enum:

- ENABLED

- DISABLED

- OVERLOAD_WARNING

- OTHER

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the EAS status.

Possible values are:

- ENABLED: Indicates that the EAS status is enabled

- DISABLED: Indicates that the EAS status is disabled

- OVERLOAD_WARNING: Indicates that the EAS status is overload warning.

- OTHER: Indicates the EAS status is another status.

DeviceType:

anyOf:

- type: string

enum:

- HAPTIC_DEVICE

- JOY_STICK

- OTHER

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the device type.

Possible values are:

- HAPTIC_DEVICE: Indicates that the device type is haptic device.

- JOY_STICK: Indicates that the device type is joy stick

- OTHER: Indicates any other device type.

# A.3	Eees_UELocation API

openapi: 3.0.0

info:

title: EES UE Location Information_API

description: |

API for EES UE Location Information.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: 1.2.0

externalDocs:

description: >

3GPP TS 29.558 V19.5.0 Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eees-uelocation/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.558.

paths:

/fetch:

post:

summary: Fetch an UE location information.

operationId: FetchUELocation

tags:

- Fetch an UE location information

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocationRequest'

responses:

'200':

description: OK (The requested location information)

content:

application/json:

schema:

$ref: '#/components/schemas/LocationResponse'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions:

post:

summary: Creates a new Individual Location Information Subscription resource

operationId: CreateLocationInfoSubscription

tags:

- Location Information Subscriptions (Collection)

description: >

Create a Subscription resource for continious reporting of UE location

information to the EAS.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocationSubscription'

responses:

'201':

description: >

Created (The individual location information subscription resource

is created successfully)

content:

application/json:

schema:

$ref: '#/components/schemas/LocationSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

LocationInformationNotification:

'{$request.body#/notificationDestination}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocationNotification'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

UserConsentRevocationNotif:

'{$request.body#/revocationNotifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ConsentRevocNotif'

responses:

'204':

description: No Content (successful notification).

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: Read an Individual Location Information Subscription resource

operationId: ReadIndLocationInfoSubscription

tags:

- Individual Location Information Subscription (Document)

description: Retrieve an Individual location information subscription information.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

responses:

'200':

description: OK (Successfully get the location information subscription).

content:

application/json:

schema:

$ref: '#/components/schemas/LocationSubscription'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an Individual Location Information Subscription resource

operationId: UpdateIndLocationInfoSubscription

tags:

- Individual Location Information Subscription (Document)

description: Fully replace an existing Individual location information Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocationSubscription'

responses:

'200':

description: OK (The individual location information subscription was modified successfully).

content:

application/json:

schema:

$ref: '#/components/schemas/LocationSubscription'

'204':

description: No Content.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an Individual Location Information Subscription resource

operationId: ModifyIndLocationInfoSubscription

tags:

- Individual Location Information Subscription (Document)

description: Partially update an existing Individual location information Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

requestBody:

description: Partial update an existing Individual AC information Subscription.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/LocationSubscriptionPatch'

responses:

'200':

description: >

OK (The Individual location information Subscription is successfully

modified and the updated subscription information is returned in the response).

content:

application/json:

schema:

$ref: '#/components/schemas/LocationSubscription'

'204':

description: >

No Content (The individual location information subscription was modified successfully).

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an Individual Location Information Subscription resource

operationId: DeleteIndLocationInfoSubscription

tags:

- Individual Location Information Subscription (Document)

description: Delete an existing Individual location information Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

responses:

'204':

description: The individual subscription is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

# Components

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

LocationSubscription:

type: object

description: Represents an Individual Location Information Subscription.

properties:

easId:

type: string

description: Identifier of the EAS subscribing for location information report.

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

intGrpId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

extGrpId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExternalGroupId'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

locGran:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/Accuracy'

locQos:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LocationQoS'

eventReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: >

Set to true by the EAS to request the EES to send a test notification.

Set to false or omitted otherwise.

revocationNotifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

oneOf:

- required: [ueId]

- required: [intGrpId]

- required: [extGrpId]

required:

- easId

LocationSubscriptionPatch:

type: object

description: Represents the partial update of Individual AC Information Subscription.

properties:

eventReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

revocationNotifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

locGran:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/Accuracy'

locQos:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LocationQoS'

LocationNotification:

type: object

description: Represents the filters information for AC Information Subscription.

properties:

subId:

type: string

description: >

Identifier of the location information subscription for which the

location information notification is related to.

locEvs:

type: array

items:

$ref: '#/components/schemas/LocationEvent'

minItems: 1

description: List of notifications with location information.

required:

- subId

- locEvs

LocationEvent:

type: object

description: Location Information event notification.

properties:

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

locInf:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

locInfPred:

$ref: 'TS29522_AnalyticsExposure.yaml#/components/schemas/UeMobilityExposure'

oneOf:

- required: [locInf]

- required: [locInfPred]

required:

- ueId

LocationRequest:

type: object

description: To request location information request.

properties:

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

gran:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/Accuracy'

locQos:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LocationQoS'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- ueId

LocationResponse:

type: object

description: Contains the response  location information request.

properties:

ueLocation:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- ueLocation

ConsentRevocNotif:

description: >

Represents the user consent revocation information conveyed in a user consent

revocation notification.

type: object

properties:

subscriptionId:

type: string

description: >

Contains the identifier of the subscription to which the notification is related.

consentsRevoked:

type: array

items:

$ref: '#/components/schemas/ConsentRevoked'

minItems: 1

required:

- subscriptionId

- consentsRevoked

ConsentRevoked:

description: Represents the information related to a revoked user consent.

type: object

properties:

ucPurpose:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/UcPurpose'

externalId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalId'

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

required:

- ucPurpose

oneOf:

- required: [externalId]

- required: [ueId]

# A.4	Eees_UEIdentifier API

openapi: 3.0.0

info:

title: EES UE Identifier Service

version: 1.2.0

description: |

EES UE Identifier Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.558 V19.5.0; Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eees-ueidentifier/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

paths:

/fetch:

post:

deprecated: true

summary: Fetch the identifier of an UE.

operationId: FetchUEId

tags:

- Fetch UE Identifier

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UserInformation'

responses:

'200':

description: >

The communicated ACR update information was successfully received.

The response body contains the feedback of the EES.

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/get:

post:

summary: Get the identifier of an UE.

operationId: GetUEId

tags:

- Get UE Identifier

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UserInfo'

responses:

'200':

description: >

The operation is successful and the corresponding UE Identifier information, returned

by the Edge Enabler Server is included in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/UeIdInfo'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

UserInformation:

deprecated: true

description: >

Represents information about the User or the UE, that used by EES to use 3GPP CN capability

to retrieve the EAS specific UE identifier.

type: object

properties:

easId:

description: >

The application identifier of the EAS, e.g. URI, FQDN, requesting the UE Identifier

information

type: string

easProviderId:

description: Identifier of the ASP that provides the EAS.

type: string

ipAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- easId

- ipAddr

UserInfo:

description: >

Represents information about the User or the UE, that used by EES to retrieve the UE

Identifier information.

type: object

properties:

requestorId:

type: string

easIds:

type: array

items:

type: string

minItems: 1

description: >

The additional list of EAS Identifier for which the UE IDs are requested for by EAS or

EEC given the User information (e.g. IP address).

easProviderId:

description: Identifier of the ASP that provides the EAS.

type: string

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

ipAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

appPortId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Port'

portNumber:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Port'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

anyOf:

- required: [ueId]

- required: [ipAddr]

UeIdInfo:

description: >

Represents UE Identifier Information, including list of UE Identifier related information.

type: object

properties:

ueIds:

type: array

items:

$ref: '#/components/schemas/UeId'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- ueIds

UeId:

description: >

Represents UE Identifier Information, including list of UE Identifier related information.

type: object

properties:

edgeUeId:

description: Represents EDGE UE Identifier.

type: string

afSpecUeId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

easId:

description: >

The application identifier of the EAS, e.g. URI, FQDN, requesting the UE Identifier

Information.

type: string

oneOf:

- required: [edgeUeId]

- required: [afSpecUeId]

# A.5	Eees_AppClientInformation API

openapi: 3.0.0

info:

title: EES Application Client Information_API

description: |

API for EES Application Client Information.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: 1.2.0

externalDocs:

description: >

3GPP TS 29.558 V19.5.0 Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eees-appclientinformation/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.558.

paths:

/subscriptions:

post:

summary: Creates a new Individual Application Client Information Subscriptions resource

operationId: CreateAppClientInfoSubscription

tags:

- Application Client Information Subscriptions (Collection)

description: Create a Subscription resource for reporting of AC information to the EAS.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ACInfoSubscription'

responses:

'201':

description: >

Created (The individual AC information subscription resource is created successfully)

content:

application/json:

schema:

$ref: '#/components/schemas/ACInfoSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

ACInformationNotification:

'{$request.body#/notificationDestination}':

post:

requestBody: # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ACInfoNotification'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: Read an Individual Application Client Information Subscriptions resource

operationId: ReadIndAppClientInfoSubscription

tags:

- Individual Application Client Information Subscription (Document)

description: Retrieve an Individual AC information subscription information.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

responses:

'200':

description: OK (Successfully get the AC information subscription).

content:

application/json:

schema:

$ref: '#/components/schemas/ACInfoSubscription'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an Individual Application Client Information Subscriptions resource

operationId: UpdateIndAppClientInfoSubscription

tags:

- Individual Application Client Information Subscription (Document)

description: Fully replace an existing Individual AC information Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ACInfoSubscription'

responses:

'200':

description: OK (The individual AC information subscription was modified successfully).

content:

application/json:

schema:

$ref: '#/components/schemas/ACInfoSubscription'

'204':

description: No Content.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an Individual Application Client Information Subscriptions resource

operationId: ModifyIndAppClientInfoSubscription

tags:

- Individual Application Client Information Subscription (Document)

description: Partially update an existing Individual AC information Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

requestBody:

description: Partial update an existing Individual AC information Subscription.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/ACInfoSubscriptionPatch'

responses:

'200':

description: >

OK (The Individual AC information Subscription is successfully modified

and the updated subscription information is returned in the response).

content:

application/json:

schema:

$ref: '#/components/schemas/ACInfoSubscription'

'204':

description: >

No Content (The individual AC information subscription was modified successfully).

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an Individual Application Client Information Subscriptions resource

operationId: DeleteIndAppClientInfoSubscription

tags:

- Individual Application Client Information Subscription (Document)

description: Delete an existing Individual AC information Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

responses:

'204':

description: The individual subscription is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

# Components

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ACInfoSubscription:

type: object

description: Represents an Individual AC Information Subscription.

properties:

easId:

type: string

description: Identifier of the EAS subscribing for AC information report.

acFltrs:

type: array

items:

$ref: '#/components/schemas/ACFilters'

minItems: 1

description: Filters to retrieve the information about specific ACs.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

eventReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: >

Set to true by the EAS to request the EES to send a test notification.

Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

trigCondParams:

type: array

items:

$ref: '#/components/schemas/TrigCondParams'

minItems: 1

description: >

Represents the notification triggering conditions of the AC information subscription.

required:

- easId

ACInfoSubscriptionPatch:

type: object

description: Represents the partial update of Individual AC Information Subscription.

properties:

acFltrs:

type: array

items:

$ref: '#/components/schemas/ACFilters'

minItems: 1

description: Filters to retrieve the information about specific ACs.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

eventReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

trigCondParams:

type: array

items:

$ref: '#/components/schemas/TrigCondParams'

minItems: 1

description: >

Represents the notification triggering conditions of the AC information subscription.

ACFilters:

type: object

description: Represents the filters information for AC Information Subscription.

properties:

acTypesList:

type: array

items:

type: string

minItems: 1

ecspIdsList:

type: array

items:

type: string

minItems: 1

acIdsList:

type: array

items:

type: string

minItems: 1

svcArea:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ServiceArea'

maxAcKpi:

$ref: 'TS24558_Eees_EECRegistration.yaml#/components/schemas/ACServiceKPIs'

minAcKpi:

$ref: 'TS24558_Eees_EECRegistration.yaml#/components/schemas/ACServiceKPIs'

opSchds:

type: array

items:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

description: Operation schedule of EAS to be matched with operation schedule of the AC.

ueIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minItems: 1

description: List of UE identifiers hosting the AC.

locInfs:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

easBundInd:

$ref: '#/components/schemas/EASBdlInd'

ACInfoNotification:

type: object

description: AC Information notification.

properties:

subId:

type: string

description: >

Identifier of the AC information subscription for which this notification is related to.

acInfs:

type: array

items:

$ref: '#/components/schemas/ACInformation'

minItems: 1

description: Notifications that include the ACs information matching filter criteria.

required:

- subId

- acInfs

ACInformation:

type: object

description: AC Information matching the filter criteria.

properties:

acProfs:

type: array

items:

$ref: 'TS24558_Eees_EECRegistration.yaml#/components/schemas/ACProfile'

minItems: 1

description: List of profile information of ACs.

ueIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minItems: 1

description: List of UE identifiers hosting the AC.

ueLocInfs:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

required:

- acProfs

EASBdlInd:

type: object

description: Represents the EAS bundle indication information.

properties:

bdlType:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/BdlType'

bdlId:

type: string

description: Indicates a bundle ID.

easBdlReqs:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EASBdlReqs'

parBundDet:

type: boolean

default: false

description: >

Set to true to indicate that the partial bundle determination is requested.

Set to false to indicate that the partial bundle determination is not requested.

Set to false or omitted otherwise.

anyOf:

- required: [bdlType]

- required: [bdlId]

- required: [easBdlReqs]

- required: [parBundDet]

TrigCondParams:

anyOf:

- type: string

enum:

- ANALYTICS_INFO

- EEC_REGISTRATION

- EAS_DISCOVERY

- UNSPECIFIED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the trigger condition parameter.

Possible values are:

- ANALYTICS_INFO: Indicates that the notification triggering condition is the analytics

information.

- EEC_REGISTRATION: Indicates that the notification triggering condition is the EEC

registration.

- EAS_DISCOVERY: Indicates that the notification triggering condition is the EAS

discovery.

- UNSPECIFIED: Indicates that the triggering conditions are unspecified.

# A.6	Eees_SessionWithQoS API

openapi: 3.0.0

info:

title: EES Session with QoS API

description: |

API for EES Session with Qos service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: 1.2.0

externalDocs:

description: >

3GPP TS 29.558 V19.5.0 Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eees-session-with-qos/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.558.

paths:

/sessions:

post:

summary: Create a new Individual Session with QoS resource

operationId: CreateIndSessionWithQoS

tags:

- Sessions with QoS (Collection)

description: >

Request reservation of resources for a data session between AC and EAS with a specific QoS.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SessionWithQoS'

responses:

'201':

description: Created (Successful creation)

content:

application/json:

schema:

$ref: '#/components/schemas/SessionWithQoS'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

notificationDestination:

'{$request.body#/notificationDestination}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UserPlaneEventNotification'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

get:

summary: Read all Sessions with QoS resource

operationId: ReadAllSessionsWithQoS

tags:

- Sessions with QoS (Collection)

description: Retrieve all the Session With QoS information.

parameters:

- name: eas-id

in: query

description: Identifier of the EAS which querying the status of subscriptions.

required: true

schema:

type: string

responses:

'200':

description: OK (Successful get all of the active subscriptions)

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/SessionWithQoS'

minItems: 1

description: >

All the subscription information related to the request URI is returned

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/sessions/{sessionId}:

put:

summary: Update an Individual Session with QoS resource

operationId: UpdateIndSessionWithQoS

tags:

- Individual Session with QoS (Document)

description: >

Fully replace an existing Individual Session with QoS resource identified by a sessionId.

parameters:

- name: sessionId

in: path

description: Session Id.

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SessionWithQoS'

responses:

'200':

description: >

The individual Session with QoS is successfully modified and the updated

session with QoS context information is returned in the response.

content:

application/json:

schema:

$ref: '#/components/schemas/SessionWithQoS'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an Individual Session with QoS resource

operationId: ModifyIndSessionWithQoS

tags:

- Individual Session with QoS (Document)

parameters:

- name: sessionId

in: path

description: session Id.

required: true

schema:

type: string

requestBody:

description: >

Partial update an existing Individual Session with QoS resource identified by a sessionId.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/SessionWithQoSPatch'

responses:

'200':

description: >

The individual Session with QoS is successfully modified and the updated

session with QoS context information is returned in the response.

content:

application/json:

schema:

$ref: '#/components/schemas/SessionWithQoS'

'204':

description: No Content.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an Individual Session with QoS resource

operationId: DeleteIndSessionWithQoS

tags:

- Individual Session with QoS (Document)

description: Remove an Individual Session with QoS resource identified by a sessionId.

parameters:

- name: sessionId

in: path

description: session Id.

required: true

schema:

type: string

responses:

'204':

description: >

The individual Session with QoS resource matching the sessionId is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

get:

summary: Read Individual Session with QoS resource

operationId: ReadIndSessionWithQoS

tags:

- Individual Session with QoS (Document)

description: Read a subscription resource for a sessionId.

parameters:

- name: sessionId

in: path

description: Session Id.

required: true

schema:

type: string

responses:

'200':

description: The subscription information related to the request URI is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SessionWithQoS'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

# Components

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

SessionWithQoS:

type: object

description: Represents an Individual Session with QoS Subscription.

properties:

self:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

easId:

type: string

description: Identifier of an EAS.

ueIpv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

ueIpv6Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

ipDomain:

type: string

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

intGrpId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

extGrpId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExternalGroupId'

ipFlows:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/FlowDescription'

minItems: 1

description: Contains the flow description for the Uplink and/or Downlink IP flows.

trafFilterInfo:

$ref: 'TS29558_Eees_ACRManagementEvent.yaml#/components/schemas/TrafficFilterInfo'

qosReference:

type: string

description: Identifies a pre-defined QoS information.

altQosReference:

type: array

items:

type: string

description: >

Identifies an ordered list of pre-defined QoS information.

The lower the index of the array for a given entry, the higher the priority.

events:

type: array

items:

$ref: 'TS29122_AsSessionWithQoS.yaml#/components/schemas/UserPlaneEvent'

description: Indicates the events subscribed by the EAS.

sponsorInformation:

$ref: 'TS29122_CommonData.yaml#/components/schemas/SponsorInformation'

qosMonInfo:

$ref: 'TS29122_AsSessionWithQoS.yaml#/components/schemas/QosMonitoringInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

maxbrUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

maxbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

disUeNotif:

type: boolean

description: >

Indicates to disable QoS flow parameters signalling to the UE when the SMF is notified

by the NG-RAN of changes in the fulfilled QoS situation when it is included and set to

"true". The fulfilled situation is either the QoS profile or an Alternative QoS Profile.

The default value "false" shall apply, if the attribute is not present and has not been

supplied previously.

requestTestNotification:

type: boolean

description: >

Set to true by Subscriber to request the EES to send a test notification

as defined in 3GPP TS 29.122. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- easId

- ipFlows

SessionWithQoSPatch:

type: object

description: Represents a modification request of Individual Session with QoS Subscription.

properties:

ipFlows:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/FlowDescription'

minItems: 1

description: Contains the flow description for the Uplink and/or Downlink IP flows.

trafFilterInfo:

$ref: 'TS29558_Eees_ACRManagementEvent.yaml#/components/schemas/TrafficFilterInfo'

qosReference:

type: string

description: Identifies a pre-defined QoS information.

altQosReference:

type: array

items:

type: string

description: >

Identifies an ordered list of pre-defined QoS information.

The lower the index of the array for a given entry, the higher the priority.

events:

type: array

items:

$ref: 'TS29122_AsSessionWithQoS.yaml#/components/schemas/UserPlaneEvent'

description: Indicates the events subscribed by the EAS.

sponsorInformation:

$ref: 'TS29122_CommonData.yaml#/components/schemas/SponsorInformation'

qosMonInfo:

$ref: 'TS29122_AsSessionWithQoS.yaml#/components/schemas/QosMonitoringInformationRm'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

maxbrUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRateRm'

maxbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRateRm'

disUeNotif:

type: boolean

UserPlaneEventNotification:

type: object

description: Represents the user plane event notification.

properties:

sessionId:

type: string

description: >

String identifying the individual data session information for which

the QoS event notification is delivered.

eventReports:

type: array

items:

$ref: 'TS29122_AsSessionWithQoS.yaml#/components/schemas/UserPlaneEventReport'

minItems: 1

description: >

Contains the flow description for the Uplink and/or Downlink IP flows.

required:

- sessionId

- eventReports

# A.7	Eees_ACRManagementEvent API

openapi: 3.0.0

info:

title: EES ACR Management Event_API

description: |

API for EES ACR Management Event.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: 1.2.0

externalDocs:

description: >

3GPP TS 29.558 V19.5.0 Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eees-acrmgntevent/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.558.

paths:

/subscriptions:

post:

summary: Creates a new Individual ACR Management Events Subscription

operationId: CreateACRMngEventSubscr

tags:

- ACR Management Events Subscriptions (Collection)

description: Create an Individual ACR Management Event Subscription resource.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

callbacks:

ACRManagementEventsNotification:

'{$request.body#/notificationDestination}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsNotification'

responses:

'200':

description: OK (The notification is received successfully)

content:

application/json:

schema:

$ref: '#/components/schemas/EasAckInformation'

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

UPPathChangeAvailabilityNotif:

'{$request.body#/notificationDestination}/report-availability':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AvailabilityNotif'

responses:

'204':

description: No Content. The notification is successful received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Created (Successful creation)

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'204':

description: >

Successful case. The resource has been successfully created and no

additional content is to be sent in the response message.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

get:

summary: Read all ACR Management Events Subscriptions

operationId: GetACRMngEventSubscrs

tags:

- ACR Management Events Subscriptions (Collection)

description: Retrieve all the ACR Management Events Subscriptions information.

parameters:

- name: supp-feat

in: query

description: Features supported by the service consumer.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: OK (Successful get all of the active subscriptions)

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

minItems: 1

description: All the active ACR management events subscriptions

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: Read an Individual ACR Management Events Subscription

operationId: GetIndACRMngEventSubscr

tags:

- Individual ACR Management Events Subscription (Document)

description: Retrieve an Individual ACR Management Events Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

- name: supp-feat

in: query

description: Features supported by the service consumer.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: OK (Successful get the active subscription).

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an Individual ACR Management Events Subscription

operationId: UpdateIndACRMngEventSubscr

tags:

- Individual ACR Management Events Subscription (Document)

description: Fully replace an existing Individual ACR Management Events Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

responses:

'200':

description: OK (Successful get the active subscription).

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an Individual ACR Management Events Subscription

operationId: ModifyIndACRMngEventSubscr

tags:

- Individual ACR Management Events Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

requestBody:

description: Partial update an existing Individual ACR Management Events Subscription.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscriptionPatch'

responses:

'200':

description: >

The Individual ACR Management Events Subscription is successfully modified

and the updated subscription information is returned in the response.

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

'204':

description: No Content.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an Individual ACR Management Events Subscription

operationId: DeleteIndACRMngEventSubscr

tags:

- Individual ACR Management Events Subscription (Document)

description: Delete an existing Individual ACR Management Events Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

responses:

'204':

description: The individual subscription is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

# Components

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

AcrMgntEventsSubscription:

type: object

description: Represents an Individual ACR Management Events Subscription.

properties:

self:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

easId:

type: string

description: Identifier of the service consumer.

eventSubscs:

type: array

items:

$ref: '#/components/schemas/AcrMgntEventSubsc'

minItems: 1

description: The subscribed ACR management events.

evtReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

eventReports:

type: array

items:

$ref: '#/components/schemas/AcrMgntEventReport'

minItems: 1

description: The ACR management event report(s).

availabilityInfo:

$ref: '#/components/schemas/AvailabilityNotif'

failEventReports:

type: array

items:

$ref: '#/components/schemas/FailureAcrMgntEventInfo'

minItems: 1

description: Failure event reports.

requestTestNotification:

type: boolean

description: >

Set to true to request the EES to send a test notification.

Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- easId

- eventSubscs

- notificationDestination

AcrMgntEventSubsc:

type: object

description: Represents an ACR Management Event Subscription.

properties:

event:

$ref: '#/components/schemas/AcrMgntEvent'

eventFilter:

$ref: '#/components/schemas/AcrMgntEventFilter'

evtReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

tgtUeId:

$ref: '#/components/schemas/TargetUeIdentification'

appGrpId:

type: string

dnaiChgType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DnaiChangeType'

easAckInd:

type: boolean

description: >

Identifies whether EAS acknowledgement of UP path change event notifications is to be

expected. Set to "true" if the EAS acknowledgement is expected. Set to "false" if

the EAS acknowledgement is not expected. Default value is "false" if ommited.

This attribute may be provided only if the "event" attribute is set to "UP_PATH_CHG".

easChars:

type: array

items:

$ref: 'TS24558_Eees_EASDiscovery.yaml#/components/schemas/EasCharacteristics'

minItems: 1

description: A list of EAS characteristics.

trafFilterInfo:

$ref: '#/components/schemas/TrafficFilterInfo'

servContPlanInd:

type: boolean

description: >

Represents the service continuity planning indication (i.e., whether or not the EES

shall monitor whether the UE(s) enter the predicted location).

When set to true, it indicates that service continuity planning is required.

When set to false, it indicates that Service continuity planning is not required.

The default value when this attribute is omitted is false.

easAckSvcCont:

type: boolean

description: >

Indicates that the EAS will provide an acknowledgement as a response to the notification

of ACR management notification related to service continuity planning. Set to "true" if

the EAS acknowledgement is expected. Default value is "false". This attribute may be

provided only if the "event" attribute is set to "ACR_MONITORING" and/or

"ACR_FACILITATION".

required:

- event

AcrMgntEventsSubscriptionPatch:

type: object

description: >

Represents a modification request of Individual ACR Management Events Subscription.

properties:

eventSubscs:

type: array

items:

$ref: '#/components/schemas/AcrMgntEventSubsc'

minItems: 1

description: The subscribed ACR management events.

evtReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

AcrMgntEventsNotification:

type: object

description: Represents the ACR management events notification.

properties:

subpId:

type: string

description: >

String identifying the Individual ACR Management Events Subscription

for which the notification is delivered.

eventReports:

type: array

items:

$ref: '#/components/schemas/AcrMgntEventReport'

minItems: 1

description: A list of ACR management event reports.

required:

- subpId

- eventReports

AcrMgntEventReport:

type: object

description: Represents an ACR management event report.

properties:

event:

$ref: '#/components/schemas/AcrMgntEvent'

timeStamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

upPathChgInfo:

$ref: '#/components/schemas/UpPathChangeInfo'

easEndPoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

actStatus:

$ref: '#/components/schemas/ActStatus'

acrParams:

$ref: '#/components/schemas/ACRParameters'

acId:

type: string

ueId:

$ref: '#/components/schemas/TargetUeIdentification'

selACRScen:

type: array

items:

$ref: '#/components/schemas/SelectedACRScenarios'

minItems: 1

easInBdlInfoList:

type: array

items:

$ref: '#/components/schemas/EasInBundleInfo'

minItems: 1

description: Represents the list of EAS in a bundle related information.

servContPlanInd:

type: boolean

description: >

Represents the service continuity planning indication (i.e., whether or not the EES will

monitor whether the UE(s) enter the predicted location).

When set to true, it indicates that service continuity planning will be performed.

When set to false, it indicates that Service continuity planning will not be performed.

The default value when this attribute is omitted is false.

inOutOfServArea:

$ref: '#/components/schemas/InOutArea'

ueIds:

type: array

items:

$ref: '#/components/schemas/TargetUeIdentification'

minItems: 1

required:

- event

ACRParameters:

type: object

description: Represents the ACR parameters.

properties:

predictExpTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

FailureAcrMgntEventInfo:

type: object

description: Represents a failure ACR management event.

properties:

event:

$ref: '#/components/schemas/AcrMgntEvent'

failureCode:

$ref: '#/components/schemas/AcrMgntEventFailureCode'

required:

- event

- failureCode

TargetUeIdentification:

description: Identifies the target UE information.

type: object

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

edgeUeId:

type: string

intGrpId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

extGrpId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExternalGroupId'

ueIpAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

oneOf:

- required: [gpsi]

- required: [edgeUeId]

- required: [intGrpId]

- required: [extGrpId]

- required: [ueIpAddr]

AvailabilityNotif:

type: object

description: >

Represents the availability information of user plane path management events monitoring

via the 3GPP 5GC network.

properties:

availabilityStatus:

$ref: '#/components/schemas/AvailabilityStatus'

required:

- availabilityStatus

UpPathChangeInfo:

description: Represents user plane path change information.

type: object

properties:

ueId:

$ref: '#/components/schemas/IndUeIdentification'

dnaiChgType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DnaiChangeType'

sourceTrafficRoute:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

targetTrafficRoute:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

sourceDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

targetDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

srcUeIpv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

srcUeIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

tgtUeIpv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

tgtUeIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

required:

- ueId

- dnaiChgType

IndUeIdentification:

description: Represents identification information of a UE.

type: object

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

externalId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalId'

ueIpAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

oneOf:

- required: [gpsi]

- required: [externalId]

- required: [ueIpAddr]

TrafficFilterInfo:

description: Represents the Traffic Filter Information.

type: object

properties:

ipFlows:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/FlowDescription'

minItems: 1

description: Contains the flow description for the Uplink and/or Downlink IP flows.

uris:

type: array

items:

type: string

minItems: 1

description: Indicates URI(s) matching criteria.

domainNames:

type: array

items:

type: string

minItems: 1

description: Indicates Domain Name matching criteria.

dnProtocol:

$ref: 'TS29122_PfdManagement.yaml#/components/schemas/DomainNameProtocol'

anyOf:

- required: [ipFlows]

- required: [uris]

- required: [domainNames]

SelectedACRScenarios:

type: object

description: >

Represents the selected ACR scenario(s) applicable for a given combination of AC and UE.

properties:

acrList:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

minItems: 0

acId:

type: string

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

required:

- acrList

- acId

- ueId

EasAckInformation:

type: object

description: Represents the EAS acknowledgement information.

properties:

resCode:

$ref: '#/components/schemas/ResultCode'

required:

- resCode

EasInBundleInfo:

type: object

description: >

Represents EAS in a bundle information.

properties:

easId:

type: string

description: Contains the identifier of the EAS that is within an EAS bundle.

dnais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

svcArea:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ServiceArea'

required:

- easId

#

# ENUMERATIONS DATA TYPES

#

AcrMgntEvent:

anyOf:

- type: string

enum:

- UP_PATH_CHG

- ACR_MONITORING

- ACR_FACILITATION

- ACT_START_STOP

- ACR_SELECTION

- OUT_OF_SERVICE_AREA

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the ACR Management event.

Possible values are:

- UP_PATH_CHG: Indicates that ACR Management Event is the User plane path change event.

- ACR_MONITORING: Indicates that ACR Management Event is the ACR monitoring event.

- ACR_FACILITATION: Indicates that ACR Management Event is the ACR facilitation event.

- ACT_START_STOP: Indicates that ACR Management Event is the ACT start/stop event.

- ACR_SELECTION: Indicates that ACR Management Event is the ACR selection event.

- OUT_OF_SERVICE_AREA: Indicates that ACR Management Event is the out of service area event.

AcrMgntEventFilter:

anyOf:

- type: string

enum:

- INTRA_EDN_MOBILITY

- INTER_EDN_MOBILITY

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the filtering criteria for the ACR Management event.

Possible values are:

- INTRA_EDN_MOBILITY: Indicates that the ACR Management Event filter is intra-EDN mobility.

- INTER_EDN_MOBILITY: Indicates that the ACR Management Event filter is inter-EDN mobility.

ActStatus:

anyOf:

- type: string

enum:

- ACT_START

- ACT_STOP

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the ACT status.

Possible values are:

- ACT_START: Indicates ACT start.

- ACT_STOP: Indicates ACT stop.

AcrMgntEventFailureCode:

anyOf:

- type: string

enum:

- 3GPP_UP_PATH_CHANGE_MON_NOT_AVAILABLE

- 3GPP_SERVICE_AREA_MON_NOT_AVAILABLE

- OTHER_REASONS

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the failure reason for the ACR Management event.

Possible values are:

- 3GPP_UP_PATH_CHANGE_MON_NOT_AVAILABLE: Indicates that the ACR Management Event

Subscription failed because user plane path management event notifications from the 3GPP

network is NOT available. This value is only applicable for the "UP_PATH_CHG",

"ACR_MONITORING" and "ACR_FACILITATION" events.

- 3GPP_SERVICE_AREA_MON_NOT_AVAILABLE: Indicates that ACR Management Event Subscription

failed because area of interest location monitoring from the 3GPP network is not

available. This value is only applicable for the "OUT_OF_SERVICE_AREA" event.

- OTHER_REASONS: Indicates that the ACR Management Event Subscription failed for other

reasons. This value is applicable for all events.

AvailabilityStatus:

anyOf:

- type: string

enum:

- AVAILABLE

- NOT_AVAILABLE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the availability status.

Possible values are:

- AVAILABLE: Indicates availability.

- NOT_AVAILABLE: Indicates unavailability.

ResultCode:

anyOf:

- type: string

enum:

- ACCEPTED

- REJECTED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the result code of ACT acceptance.

Possible values are:

- ACCEPTED: Indicates acceptance of the ACT.

- REJECTED: Indicates rejection of the ACT.

InOutArea:

anyOf:

- type: string

enum:

- MOVED_INTO_AREA

- MOVED_OUT_OF_AREA

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the presence status.

Possible values are:

- MOVED_INTO_AREA: Indicates that the UE moved into the service area.

- MOVED_OUT_OF_AREA: Indicates that the UE moved out of the service area.

# A.8	Eees_EECContextRelocation API

openapi: 3.0.0

info:

title: EES EEC Context Relocation API

version: 1.2.0

description: |

API for EEC Context Relocation.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.558 V19.5.0 Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eees-eeccontextreloc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.558.

paths:

/eec-contexts:

post:

summary: Push EEC Context information.

operationId: PushEecContexts

tags:

- EEC contexts (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EECContextPush'

responses:

'200':

description: >

OK. The EEC context has been successfully pushed to the EES and related

information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/EECContextPushRes'

'204':

description: No Content. The EEC context has been successfully pushed to the EES.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

get:

summary: Pull EEC Context information.

operationId: PullEecContexts

tags:

- EEC contexts (Collection)

parameters:

- name: ees-id

in: query

description: Unique identifier of the requesting entity.

required: true

schema:

type: string

- name: eec-cntx-id

in: query

description: Unique identifier of the EEC context.

required: true

schema:

type: string

- name: sess-cntxs

in: query

description: List of service session context information being requested.

required: false

content:

application/json:

schema:

$ref: '#/components/schemas/SessionContexts'

responses:

'200':

description: >

OK. The EEC context information matching the query parameters in the request

shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/EECContext'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

SessionContexts:

type: object

description: Represents the list of service session contexts information.

properties:

sessCntxs:

type: array

items:

$ref: '#/components/schemas/IndividualSessionContext'

minItems: 1

description: List of service session contexts information.

required:

- sessCntxs

IndividualSessionContext:

type: object

description: Represents a single service session context information.

properties:

easId:

type: string

description: Identifier of the Application Server providing the application services.

endPt:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

acId:

type: string

description: Identifier of the AC for which the service session information is provided.

acrList:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

minItems: 1

description: The list of selected ACR scenarios.

eecId:

type: string

appGrpId:

type: string

required:

- easId

- endPt

EECContextPush:

type: object

description: Represents the EEC context push request data.

properties:

eesId:

type: string

description: Identifier of the entity that is pushing the EEC context information.

eecCntx:

$ref: '#/components/schemas/EECContext'

tgtEas:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

acrScenariosSelReq:

type: boolean

description: >

Set to true if the EEC indicates to select the ACR scenarisos.

Set to false if it is not required to select the ACR scenarios.

The default value when omitted is false.

eecCntxs:

type: array

items:

$ref: '#/components/schemas/EECContext'

minItems: 1

required:

- eesId

- eecCntx

EECContextPushRes:

type: object

description: Represents the EEC context push response data.

properties:

implReg:

$ref: '#/components/schemas/ImplicitRegDetails'

selAcrScenariosList:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

minItems: 1

ImplicitRegDetails:

type: object

description: Represents the EEC implicit registration details.

properties:

regId:

type: string

description: Identifier of the EEC registration.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

required:

- regId

EECContext:

type: object

description: Represents the EEC context information.

properties:

eecId:

type: string

description: Unique idenitfier of the EEC.

cntxId:

type: string

description: Unique idenitfier assigned to the EEC context.

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

e1Subs:

type: array

items:

type: string

minItems: 1

description: List of subscription IDs for the capability expsoure for the EEC ID.

ueLoc:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

acProfs:

type: array

items:

$ref: 'TS24558_Eees_EECRegistration.yaml#/components/schemas/ACProfile'

minItems: 1

description: List AC profiles.

sessCntxs:

$ref: '#/components/schemas/SessionContexts'

eecSrvContSupp:

$ref: '#/components/schemas/EECSrvContinuitySupport'

ueMobSuppInd:

type: boolean

description: >

Set to true to indicate that UE Mobility support is required.

Set to false to indicate that UE mobility support is not required.

The default value when omitted is false.

required:

- eecId

- cntxId

EECSrvContinuitySupport:

type: object

description: Represents the EEC service continuity support details.

properties:

srvContSupp:

type: boolean

description: >

Set to true if EEC supports service continuity.

Set to false if EEC does not supports service continuity.

acrScenarios:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

minItems: 1

description: The list of ACR scenarios supported by EEC.

required:

- srvContSupp

# A.9	Eees_EELManagedACR API

openapi: 3.0.0

info:

title: EES EEL Managed ACR Service

version: 1.2.0

description: |

EES EEL Managed ACR Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.558 V19.5.0; Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eees-eel-acr/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

paths:

/request-eelacr:

post:

summary: Request the EES to handle all the operations of an ACR.

operationId: RequestEELManagedACR

tags:

- Request EEL Managed ACR

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EELACRReq'

responses:

'200':

description: >

The requested EEL Managed ACR initiation was successfully received and

processed. The response body contains the feedback of the EES.

content:

application/json:

schema:

$ref: '#/components/schemas/EELACRResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions:

get:

summary: Retrieve all the active ACT Status Subscriptions managed by the EES.

operationId: GetACTStatusSubscriptions

tags:

- ACT Status Subscriptions (Collection)

responses:

'200':

description: >

OK. All the active ACT status subscriptions managed by the EES shall

be returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/ACTStatusSubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Request the creation of a subscription to ACT status reporting.

operationId: CreateACTStatusSubsc

tags:

- ACT Status Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ACTStatusSubsc'

responses:

'201':

description: >

Created. The subscription is successfully created and a representation of the

created Individual ACT Status Subscription resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/ACTStatusSubsc'

headers:

Location:

description: >

Contains the URI of the created Individual ACT Status Subscription resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

ACTStatusNotification:

'{$request.body#/notificationUri}/act-status':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ACTStatusNotif'

responses:

'204':

description: >

No Content. The ACT status notification is successfully received

and acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: Retrieve an ACT status subscription resource.

operationId: GetACTStatusSubscription

tags:

- Individual ACT Status Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Individual ACT Status Subscription identifier.

required: true

schema:

type: string

responses:

'200':

description: >

OK. The requested real-time UAV status subscription resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/ACTStatusSubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

EELACRReq:

description: >

Represents the parameters to request the EES to handle all the

operations of an ACR.

type: object

properties:

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

easCharacs:

type: array

items:

$ref: 'TS24558_Eees_EASDiscovery.yaml#/components/schemas/EasCharacteristics'

minItems: 1

appCtxtStoreAddr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- ueId

- easCharacs

EELACRResp:

description: Represents the feedback of the EES on EEL Managed ACR request.

type: object

properties:

appCtxtStoreAddr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

ACTStatusSubsc:

description: >

Represents the parameters to request the creation of a subscription to ACT

status reporting.

type: object

properties:

easId:

type: string

description: Contains the identifier of the service consumer.

notificationUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- easId

- notificationUri

ACTStatusNotif:

description: Represents an ACT status notification.

type: object

properties:

subscriptionId:

type: string

description: Subscription identifier.

actStatus:

$ref: 'TS29558_Eees_ACRStatusUpdate.yaml#/components/schemas/ACTResult'

required:

- subscriptionId

- actStatus

# ENUMS:

# A.10	Eees_ACRStatusUpdate API

openapi: 3.0.0

info:

title: EES ACR Status Update Service

version: 1.2.0

description: |

EES ACR Status Update Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.558 V19.5.0; Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eees-acrstatus-update/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

paths:

/request-acrupdate:

post:

summary: Request to update the information related to ACR (e.g. indicate the status of ACT, update the notification target address).

operationId: RequestACRUpdate

tags:

- Request ACR Update

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ACRUpdateData'

responses:

'200':

description: >

The communicated ACR update information was successfully received.

The response body contains the feedback of the EES.

content:

application/json:

schema:

$ref: '#/components/schemas/ACRDataStatus'

'204':

description: >

No Content. The communicated ACR update information was successfully

received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ACRUpdateData:

description: >

Represents the parameters to update the information related to ACR (e.g. indicate

the status of ACT, update the notification target address).

type: object

properties:

easId:

type: string

acId:

type: string

actResultInfo:

$ref: '#/components/schemas/ACTResultInfo'

e3SubscIds:

type: array

items:

type: string

minItems: 1

e3NotificationUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

required:

- easId

anyOf:

- required: [actResultInfo]

- required: [e3SubscIds]

- required: [e3NotificationUri]

ACRDataStatus:

description: Represents the ACR status information.

type: object

properties:

e3SubscsStatus:

$ref: '#/components/schemas/E3SubscsStatus'

e3SubscIds:

type: array

items:

type: string

minItems: 1

required:

- e3SubscsStatus

ACTResultInfo:

description: Represents the result of ACT and the related information.

type: object

properties:

actResult:

$ref: '#/components/schemas/ACTResult'

actFailureCause:

$ref: '#/components/schemas/ACTFailureCause'

appGrpInfo:

$ref: '#/components/schemas/AppGroupInfo'

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

easEndPoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

required:

- actResult

- ueId

- easEndPoint

AppGroupInfo:

description: >

Represents the Application Group information.

type: object

properties:

appGroupId:

type: string

ueIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minItems: 1

required:

- appGroupId

- ueIds

# ENUMS:

ACTResult:

anyOf:

- type: string

enum:

- SUCCESSFUL

- FAILED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the result of ACT.

Possible values are:

- SUCCESSFUL: Indicates that the ACT was successful.

- FAILED: Indicates that the ACT failed.

E3SubscsStatus:

anyOf:

- type: string

enum:

- SUCCESSFUL

- FAILED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the status of the initialization of EDGE-3 subscriptions.

Possible values are:

- SUCCESSFUL: Indicates that the initialization of EDGE-3 subscriptions was successful.

- FAILED: Indicates that the initialization of EDGE-3 subscriptions failed.

ACTFailureCause:

anyOf:

- type: string

enum:

- ACR_CANCELLATION

- OTHER

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the cause of ACT failure.

Possible values are:

- ACR_CANCELLATION: Indicates that the ACT failed due to the cancellation of the ACR.

- OTHER: Indicates that the ACT failed for other reasons.

# A.11	Eecs_EESRegistration API

openapi: 3.0.0

info:

title: ECS EES Registration_API

description: |

API for EES Registration.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: 1.2.0

externalDocs:

description: >

3GPP TS 29.558 V19.5.0 Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eecs-eesregistration/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.558.

paths:

/registrations:

post:

summary: Create a new EES Registration

operationId: CreateEESRegistration

tags:

- EES Registrations (Collection)

description: Registers a new EES at the Edge Configuration Server.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EESRegistration'

responses:

'201':

description: EES information is registered successfully at ECS.

content:

application/json:

schema:

$ref: '#/components/schemas/EESRegistration'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/registrations/{registrationId}:

get:

summary: Get an Individual EES Registration

operationId: GetIndEESReg

tags:

- Individual EES Registration (Document)

description: Retrieve an Individual EES registration resource.

parameters:

- name: registrationId

in: path

description: Registration Id.

required: true

schema:

type: string

responses:

'200':

description: OK (The EES registration information at the Edge Configuration Server).

content:

application/json:

schema:

$ref: '#/components/schemas/EESRegistration'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an Individual EES Registration

operationId: UpdateIndEESReg

tags:

- Individual EES Registration (Document)

description: Fully replace an existing EES Registration resource.

parameters:

- name: registrationId

in: path

description: EES Registration Id.

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EESRegistration'

responses:

'200':

description: OK (The EES registration information is updated successfully).

content:

application/json:

schema:

$ref: '#/components/schemas/EESRegistration'

'204':

description: >

No Content. The individual EES registration information is updated successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an Individual EES Registration

operationId: ModifyIndEESReg

tags:

- Individual EES Registration (Document)

description: Partially update an existing EES Registration resource.

parameters:

- name: registrationId

in: path

description: EES registration Id.

required: true

schema:

type: string

requestBody:

description: Partial update an existing EES registration resource.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/EESRegistrationPatch'

responses:

'200':

description: >

The Individual EES registration is successfully modified and

the updated registration information is returned in the response.

content:

application/json:

schema:

$ref: '#/components/schemas/EESRegistration'

'204':

description: >

No Content. The individual EES registration information is updated successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an Individual EES Registration

operationId: DeleteIndEESReg

tags:

- Individual EES Registration (Document)

description: Delete an existing EES registration at ECS.

parameters:

- name: registrationId

in: path

description: Registration Id.

required: true

schema:

type: string

responses:

'204':

description: The individual EES registration is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

EESRegistration:

type: object

description: Represents an EES registration information.

properties:

eesProf:

$ref: '#/components/schemas/EESProfile'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- eesProf

EESProfile:

type: object

description: Represents the EES profile information.

properties:

eesId:

type: string

description: Identifier of the EES.

endPt:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

easIds:

type: array

items:

type: string

minItems: 1

description: Application identifiers of EASs that are registered with EES.

easBdlInfos:

type: object

additionalProperties:

type: array

items:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EASBundleInfo'

minItems: 1

minProperties: 1

description: >

The key used in this map for each entry is the EAS ID of the concerned EAS.

Within each EASBundleInfo encoded map entry of this attribute, the "mainEasId" attribute

shall not be present.

allowedMNOsInfo:

type: object

additionalProperties:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

minItems: 1

minProperties: 1

description: >

The key of the map shall be the identifier of the EAS to which the provided allowed MNO

information within the map value corresponds.

ednInfoSets:

$ref: '#/components/schemas/EDNInfo'

easInstInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/EASInstantiationInfo'

minProperties: 1

description: >

Represents the EAS instantiation information for the EAS(s) registered at the EES.

The key of the map shall be the EAS ID to which the provided instantiation information

within the map value relates.

provId:

type: string

description: Identifier of the ECSP that provides the EES provider.

svcArea:

$ref: '#/components/schemas/ServiceArea'

appLocs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

description: List of DNAI(s) associated with the EES.

svcContSupp:

type: array

items:

$ref: '#/components/schemas/ACRScenario'

minItems: 1

description: The ACR scenarios supported by the EES for service continuity.

svcContSuppExt1:

type: array

items:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EASBundleInfo'

minItems: 1

description: >

Represents the information related to the EES ability to handle bundled EAS coordinated

ACRs.

This attribute may be present only when the "svcContSupp" attribute is also present.

When this attribute is present, it indicates that the EES (identified by the "eesId"

attribute) is able to handle bundled EAS coordinated ACRs and contains the information

of the EAS bundle(s) for which the EES is able to handle bundled EAS coordinated ACRs.

eecRegConf:

type: boolean

description: >

Set to true if the EEC is required to register to the EES to use edge service.

Set to false if the EEC is not required to register to use edge services.

ueServingSat:

$ref: '#/components/schemas/UeServSatInfo'

easLoadInfoList:

type: object

additionalProperties:

$ref: '#/components/schemas/LoadInfo'

minProperties: 1

description: >

Represents the load information for each EAS identified by the "easIds" attribute.

The key of the map shall be set to the identifier of the EAS (among the ones provided

within the "easIds" attribute) for which the load info provided within the map value

corresponds.

required:

- eesId

- endPt

- eecRegConf

EESRegistrationPatch:

type: object

description: Represents partial update request of individual EES registration information.

properties:

eesProf:

$ref: '#/components/schemas/EESProfile'

expTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTimeRm'

ServiceArea:

type: object

description: Represents a service area information of the EdgeApp entity.

properties:

topServAr:

$ref: '#/components/schemas/TopologicalServiceArea'

geoServAr:

$ref: '#/components/schemas/GeographicalServiceArea'

TopologicalServiceArea:

type: object

description: Represents topological service area information.

properties:

ecgis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

minItems: 1

description: A list of E-UTRA cell identities.

ncgis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

minItems: 1

description: A list of NR cell identities.

tais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

minItems: 1

description: A list of tracking area identities.

plmnIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

minItems: 1

description: A list of serving network identities.

GeographicalServiceArea:

type: object

description: Represents geographical service area information.

properties:

geoArs:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

minItems: 1

description: A list of geographic area information.

civicAddrs:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CivicAddress'

minItems: 1

description: A list of civic address information.

EASInstantiationInfo:

type: object

description: Represents the EAS instantiation information.

properties:

easId:

type: string

description: Identifier of the EAS.

status:

$ref: '#/components/schemas/InstantiationStatus'

instCrit:

$ref: '#/components/schemas/InstantiationCriteria'

required:

- easId

- status

InstantiationCriteria:

type: object

description: Represents the instantiation criteria for an EAS.

properties:

instantiationTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

instWindows:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

minItems: 1

description: A list time windows at which the EAS is instantiated.

scheds:

type: array

items:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

description: Represents the EAS instantiation schedule.

oneOf:

- required: [instantiationTime]

- required: [instWindows]

- required: [scheds]

EDNInfo:

type: object

description: Represents EDN related information.

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

dnais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

required:

- dnn

LoadInfo:

type: object

description: Represents the load information of an EEL entity.

properties:

loadLevel:

type: integer

minimum: 0

maximum: 100

loadWarningStat:

type: boolean

description: >

true indicates that load warning is activated.

false indicates that load warning is not activated.

The default value is "false" when this attribute is omitted.

UeServSatInfo:

type: object

description: Represents the EDN related information for the satellite.

properties:

dynServAreaInd:

type: boolean

description: >

Indicates whether the EES service area is mobile or not.

true indicates that the EES service area is mobile (e.g., case of a MEO or LEO

satellite).

false indicates that the EES service area is not mobile (e.g., case of a GEO

satellite).

The default value is "false" when this attribute is omitted.

satId:

type: string

trajId:

type: string

ascaiInfo:

$ref: '#/components/schemas/AppSatCovAvailInfo'

required:

- satId

AppSatCovAvailInfo:

type: object

description: Represents the Application Satellite Coverage Availability Information data.

properties:

covAvailInfo:

$ref: 'TS29549_SS_ASCAIInfoRetrieval.yaml#/components/schemas/AscaiInfo'

required:

- covAvailInfo

ACRScenario:

anyOf:

- type: string

enum:

- EEC_INITIATED

- EEC_EXECUTED_VIA_SOURCE_EES

- EEC_EXECUTED_VIA_TARGET_EES

- SOURCE_EAS_DECIDED

- SOURCE_EES_EXECUTED

- EEL_MANAGED_ACR

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the ACR scenarios supported by EES.

Possible values are:

- EEC_INITIATED: Represents the EEC initiated ACR scenario.

- EEC_EXECUTED_VIA_SOURCE_EES: Represents the EEC ACR scenario executed via the S-EES.

- EEC_EXECUTED_VIA_TARGET_EES: Represents the EEC ACR scenario executed via the T-EES.

- SOURCE_EAS_DECIDED: Represents the EEC ACR scenario where the S-EAS decides to perform

ACR.

- SOURCE_EES_EXECUTED: Represents the EEC ACR scenario where S-EES executes the ACR.

- EEL_MANAGED_ACR: Represents the EEC ACR scenario where the ACR is managed by the

Edge Enabler Layer.

InstantiationStatus:

anyOf:

- type: string

enum:

- INSTANTIATED

- INSTANTIABLE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the instantiation status information of an EAS.

Possible values are:

- INSTANTIATED: Indicates that the EAS status is instantiated.

- INSTANTIABLE: Indicates that the EAS status is instantiable but not yet instantiated.

# A.12	Eecs_TargetEESDiscovery API

openapi: 3.0.0

info:

title: ECS Target EES Discovery API

description: |

API for Target EES Discovery.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: 1.2.0

externalDocs:

description: >

3GPP TS 29.558 V19.5.0 Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eecs-targeteesdiscovery/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.558.

paths:

/ees-profiles:

get:

summary: Read all the targeted Enabler Server Profiles

operationId: GetEESProfiles

tags:

- EES Profiles (Collection)

description: Retrieve the T-EES information.

parameters:

- name: ees-id

in: query

description: Unique identifier of the source Enabler Server.

required: true

schema:

type: string

- name: eas-id

in: query

description: Unique identifier of the source Application Server.

required: true

schema:

type: string

- name: target-dnai

in: query

description: >

The DNAI information associated with the potential target Enabler Server(s)

and/or target Application Server(s).

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

- name: cand-dnais-info

in: query

description: >

Contains the candidate DNAI(s) related information.

This query parameter is applicable only for the common EAS case.

When the "EdgeApp_3" feature is supported, this query parameter and the target-dnai

Query parameter are mutually exclusive.

required: false

style: form

explode: false

schema:

type: array

items:

$ref: '#/components/schemas/CandidateDnaisInfo'

minItems: 1

- name: ue-id

in: query

description: Identifier of the UE.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

- name: ue-location

in: query

description: The location information of the UE.

required: false

content:

application/json:

schema:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

- name: eec-srv-cont-supp

in: query

description: >

Indicates whether the EEC supports service continuity or not and the related service

continuity support information.

required: false

schema:

$ref: 'TS29558_Eees_EECContextRelocation.yaml#/components/schemas/EECSrvContinuitySupport'

- name: ac-svc-cont-supp

in: query

description: >

Indicates that the AC supports service continuity and contains the related service

continuity support information (i.e., supported ACR scenarios).

required: false

schema:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

minItems: 1

- name: bdl-id

in: query

description: >

Contains EAS bundle identifier.

required: false

schema:

type: string

- name: bdl-type

in: query

description: >

Contains EAS bundle type.

required: false

schema:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/BdlType'

- name: eas-ids-list

in: query

description: >

Contains the list of the identifier(s) of the EAS(s) associated with the EAS bundle.

required: false

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: ens-ind

in: query

description: indicates whether edge node sharing is requested.

required: false

schema:

type: boolean

description: >

Set to true if edge node sharing is requested.

Set to false if edge node sharing is not requested.

The default value when omitted is false.

- name: app-grp-id

in: query

description: >

Contains the application group identifier.

required: false

schema:

type: string

- name: supp-feats

in: query

description: Contains the list of supported feature(s).

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: serving-mno-info

in: query

description: >

Contains the serving MNO information, i.e., the MNO that is serving the subscriber.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

- name: pred-exp-time

in: query

description: >

Contains the prediction expiration time.

required: false

schema:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

- name: tunnel-info

in: query

description: >

Contains the target tunnel information.

required: false

content:

application/json:

schema:

$ref: '#/components/schemas/TunnelInfo'

responses:

'200':

description: >

The EDN configuration and the Enabler Server information determined by the ECS.

content:

application/json:

schema:

$ref: 'TS24558_Eecs_ServiceProvisioning.yaml#/components/schemas/ECSServProvResp'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

#

# STRUCTURED DATA TYPES

#

schemas:

TunnelInfo:

description: >

Represents the tunnel information.

type: object

properties:

servProvId:

type: string

endPoint:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TunnelAddress'

required:

- servProvId

- endPoint

CandidateDnaisInfo:

description: >

Represents the candidate DNAI(s) related information.

type: object

properties:

candidateDnais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

candDnaisPrioInd:

type: boolean

description: >

Indicates whether the candidate DNAI(s) provided within the candidateDnais attribute are

in descending priority order, i.e., the lower the array index of the candidateDnais

attribute the higher the priority of the respective DNAI.

true indicates that the candidate DNAI(s) provided within the candidateDnais attribute

are in descending priority order.

false indicates that the candidate DNAI(s) provided within the candidateDnais attribute

are not in descending priority order.

The default value is false if this attribute is omitted.

required:

- candidateDnais

# A.13	Eees_ACRParameterInformation API

openapi: 3.0.0

info:

title: EES ACR Parameters Information Service

version: 1.0.0

description: |

EES ACR Parameters Information Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.558 V18.6.0; Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eees-acr-param/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

paths:

/send-acrparamsinfo:

post:

summary: Request to send ACR parameters information.

operationId: Request

tags:

- Send ACR Parameter Information

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ACRParamsInfo'

responses:

'204':

description: >

No Content. The ACR parameter information is successfully received and processed.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ACRParamsInfo:

description: >

Represents the ACR parameters information.

type: object

properties:

requestorId:

type: string

eecId:

type: string

acId:

type: string

sAsEndPoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

tAsEndPoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

acrParams:

$ref: 'TS29558_Eees_ACRManagementEvent.yaml#/components/schemas/ACRParameters'

required:

- requestorId

- eecId

- acId

- sAsEndPoint

- tAsEndPoint

- acrParams

# A.14	Ecas_SelectedEES API

openapi: 3.0.0

info:

title: Selected EES Service

version: 1.0.0

description: |

Selected EES Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.558 V18.6.0; Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ecas-selected-ees/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

paths:

/declare:

post:

summary: Declare the selected EES to the CAS.

operationId: DeclareSelectedEES

tags:

- Declare Selected EES

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SelEESDecInfo'

responses:

'204':

description: >

No Content. The Seleted EES Declaration request is received

successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

SelEESDecInfo:

description: >

Represents the information elements for the selected EES declaration.

type: object

properties:

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

seleEesId:

type: string

seleEndpoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

easId:

type: string

acId:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- ueId

- seleEesId

- seleEndpoint

- easId

# A.15	Eees_CommonEASAnnouncement API

openapi: 3.0.0

info:

title: EES Common EAS Announcement Service

version: 1.1.0

description: |

EES Common EAS Announcement Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.558 V19.5.0; Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eees-cea/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

paths:

/declare:

post:

summary: Request to declare common EAS information.

operationId: Declare

tags:

- Declare common EAS Information

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/CommonEASInfo'

responses:

'204':

description: >

No Content. The common EAS information is successfully received and no content is

returned in the response body.

'200':

description: >

OK. The common EAS information is successfully received and common EAS declaration

related information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/CommonEASInfoDecResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

CommonEASInfo:

description: >

Represents the common EAS information.

type: object

properties:

requestorId:

type: string

description: Represents the identifier of the announcing EES sending the request.

requestorEndPt:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

easId:

type: string

description: Represents the EAS ID of the selected common EAS

easEndPt:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

serviceKpisList:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EASServiceKPI'

ednInfo:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/EDNInfo'

appGrpId:

type: string

description: Represents the application group identifier.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- requestorId

- easId

- easEndPt

- appGrpId

CommonEASInfoDecResp:

description: >

Represents the Represents the response to a common EAS information declaration request.

type: object

properties:

grpConnInfo:

type: array

items:

$ref: '#/components/schemas/GrpConnInfo'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

anyOf:

- required: [grpConnInfo]

GrpConnInfo:

description: >

Represents the group connection information.

type: object

properties:

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

connInd:

type: boolean

description: >

Represents the common EAS connection indication, i.e., whether or not the UE identified

by the ueId attribute should connect to the selected common EAS.

When set to true, it indicates that the UE should connect to the selected common EAS.

When set to false, it indicates that the UE should not connect to the selected common

EAS.

required:

- ueId

- connInd

# A.16	Eecs_EASInfoManagement API

openapi: 3.0.0

info:

title: EAS Information Management

version: 1.1.0

description: |

EAS Information Management Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.558 V19.5.0; Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eecs-eim/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

paths:

/bindings:

get:

summary: Get EAS information.

operationId: GetEASInfo

tags:

- Get EAS information

parameters:

- name: requestor-id

in: query

description: Represents the Identifier of the service consumer.

required: true

schema:

type: string

- name: app-group-id

in: query

description: Represents the Application group identifier.

required: true

schema:

type: string

- name: eas-id

in: query

description: EAS Identifier.

required: true

schema:

type: string

- name: eas-bdl-id-list

in: query

required: false

schema:

type: array

items:

type: string

minItems: 1

- name: eas-bdl-id

in: query

required: false

schema:

type: string

- name: main-eas-id

in: query

required: false

schema:

type: string

- name: supp-feats

in: query

description: Contains the list of supported feature(s).

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: OK. Resource representation is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/CommonEASBindResp'

'204':

description: >

No Content. There is no Common EAS Binding Information corresponding to the received

query parameters.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Request to store the Common EAS information.

operationId: StoreEasInfo

tags:

- Store EAS Information

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/CommonEASBindReq'

responses:

'201':

description: >

Created. The Common EAS Binding information is successfully stored.

content:

application/json:

schema:

$ref: '#/components/schemas/CommonEASBindResp'

headers:

Location:

description: >

Contains the URI of the created Common EAS Binding information resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

description: >

Forbidden. There is an existing Common EAS information stored.

content:

application/json:

schema:

$ref: '#/components/schemas/ProblemDetailsEIMExt'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/bindings/{bindingId}:

parameters:

- name: bindingId

in: path

description: _Represents the identifier of the Individual Common EAS Binding resource.

required: true

schema:

type: string

get:

summary: Retrieve an Individual Common EAS Binding resource.

operationId: GetIndCommonEasBinding

tags:

- Individual Common EAS Binding (Document)

responses:

'200':

description: The requested Individual Common EAS Binding resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/CommonEASBindResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an existing Individual Common EAS Binding resource.

operationId: UpdateCommonEasBinding

tags:

- Individual Common EAS Binding (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/CommonEASBinding'

responses:

'200':

description: >

OK. The Individual Common EAS Binding Instance is successfully updated and a

representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/CommonEASBinding'

'204':

description: >

No Content. The Individual Common EAS Binding Instance is successfully

updated and no content is returned in the response body

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Common EAS Binding resource.

operationId: ModifyIndCommonEasBinding

tags:

- Individual Common EAS Binding (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/CommonEASBindingPatch'

responses:

'200':

description: >

OK. The Individual Common EAS Binding resource is successfully modified

and a representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/CommonEASBinding'

'204':

description: >

No Content. The Individual Common EAS Binding resource is successfully

modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual Common EAS Binding resource.

operationId: DeleteEasInfo

tags:

- Individual Common EAS Binding (Document)

responses:

'204':

description: >

No Content. The Individual Common EAS Binding resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

CommonEASBindReq:

description: >

Represents the information elements for the Common EAS information

type: object

properties:

requestorId:

type: string

binding:

$ref: '#/components/schemas/CommonEASBinding'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- requestorId

- binding

CommonEASBindResp:

description: >

Represents the response information elements for the Common EAS information.

type: object

properties:

binding:

$ref: '#/components/schemas/CommonEASBinding'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- binding

CommonEASBinding:

description: Represents the common EAS Binding information.

type: object

properties:

appGroupId:

type: string

easId:

type: string

easEndpoints:

type: array

items:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

minItems: 1

assocEasEndpoints:

type: array

items:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

minItems: 1

eesEndpoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

ednInfo:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/EDNInfo'

easSvcKpi:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EASServiceKPI'

commEasBdlInfo:

$ref: '#/components/schemas/CommEasBdlInfo'

required:

- appGroupId

- easId

- easEndpoints

CommEasBdlInfo:

description: Represents the common EAS bundle information.

type: object

properties:

easIdsList:

type: array

items:

type: string

minItems: 1

easEndpoints:

type: object

additionalProperties:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

minProperties: 1

description: >

Contains the list of the endpoint(s) of the EAS(s) constituting the EAS bundle where the

common EAS resides.

The key of the map shall be set to the identifier or the EAS (among the ones provided

within the "easIdsList" attribute) for which the endpoint information provided in the

map value is related..

mainEasId:

type: string

bdlId:

type: string

required:

- easIdsList

- easEndpoints

not:

required: [mainEasId, bdlId]

CommonEASBindingPatch:

description: Represents the requested modification to the common EAS Binding information.

type: object

properties:

easEndpoints:

type: array

items:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

minItems: 1

assocEasEndpoints:

type: array

items:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

minItems: 1

eesEndpoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

ednInfo:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/EDNInfo'

commEasBdlInfo:

$ref: '#/components/schemas/CommEasBdlInfo'

ProblemDetailsEIMExt:

description: >

Extends ProblemDetails to also include existing common EAS binding information for the

application group.

allOf:

- $ref: 'TS29122_CommonData.yaml#/components/schemas/ProblemDetails'

- $ref: '#/components/schemas/CommonEASBinding'

# A.17	Eees_TrafficInfluenceEAS API

openapi: 3.0.0

info:

title: EES Application Traffic Influence Service

version: 1.0.0

description: |

EES Application Traffic Influence Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.558 V18.6.0; Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eees-tie/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

paths:

/instances:

post:

summary: Create application traffic influence trigger from EAS.

operationId: CreateTraffInfluInstance

tags:

- Traffic Influence (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AppTrafficInfluence'

responses:

'201':

description: Create a new individual Application Traffic Influence Instance.

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

content:

application/json:

schema:

$ref: '#/components/schemas/AppTrafficInfluence'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/instances/{instanceId}:

parameters:

- name: instanceId

in: path

description: Instance Id.

required: true

schema:

type: string

get:

summary: Read an Individual Application Traffic Influence Instance.

operationId: GetIndTraffInfluInstance

tags:

- Individual ACR Management Events Subscription (Document)

responses:

'200':

description: OK. The Individual Application Traffic Influence Instance is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/AppTrafficInfluence'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an Individual Application Traffic Influence Instance.

operationId: UpdateIndTraffInfluInstance

tags:

- Individual Traffic Influence Instance (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AppTrafficInfluence'

responses:

'200':

description: >

The Individual Application Traffic Influence Instance is successfully modified and the

representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/AppTrafficInfluence'

'204':

description: No Content.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an Individual Application Traffic Influence Instance.

operationId: ModifyIndTraffInfluInstance

tags:

- Individual Traffic Influence Instance (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/AppTrafficInfluencePatch'

responses:

'200':

description: >

The Individual Application Traffic Influence Instance is successfully modified and the

representation of the updated resource is returned in the response.

content:

application/json:

schema:

$ref: '#/components/schemas/AppTrafficInfluence'

'204':

description: No Content.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an Individual Traffic Influence Instance.

operationId: DeleteIndTraffInfluInstance

tags:

- Individual Traffic Influence Instance (Document)

responses:

'204':

description: >

The Individual Traffic Influence Instance is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

AppTrafficInfluence:

description: >

Represents the application traffic influence information.

type: object

properties:

requestorId:

type: string

description: Contains the identifier of the service consumer that is sending the request.

tgtUes:

type: array

items:

$ref: 'TS29558_Eees_ACRManagementEvent.yaml#/components/schemas/TargetUeIdentification'

minItems: 1

addTgtUes:

type: array

items:

$ref: 'TS29558_Eees_ACRManagementEvent.yaml#/components/schemas/TargetUeIdentification'

minItems: 1

deleTgtUes:

type: array

items:

$ref: 'TS29558_Eees_ACRManagementEvent.yaml#/components/schemas/TargetUeIdentification'

minItems: 1

anyUe:

type: boolean

description: >

Indicates whether the request applies to any UE.

true indicates that the request applies to any UE.

False indicates that the request does not apply to any UE.

The default value when this attribute is omitted is false.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

AppTrafficInfluencePatch:

description: >

Represents the update of application traffic influence information.

type: object

properties:

addTgtUes:

type: array

items:

$ref: 'TS29558_Eees_ACRManagementEvent.yaml#/components/schemas/TargetUeIdentification'

minItems: 1

deleTgtUes:

type: array

items:

$ref: 'TS29558_Eees_ACRManagementEvent.yaml#/components/schemas/TargetUeIdentification'

minItems: 1

anyUe:

type: boolean

description: >

Indicates whether the request applies to any UE.

true indicates that the request applies to any UE.

False indicates that the request does not apply to any UE.

# A.18	Eecs_ECSServiceProvisioning API

openapi: 3.0.0

info:

title: ECS Service Provisioning Service

version: 1.1.0

description: |

ECS Service Provisioning Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.558 V19.5.0; Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

servers:

- url: '{apiRoot}/eecs-esp/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/request:

post:

summary: Enables to request service provisioning information.

operationId: ServProvRetReq

tags:

- Service Provisioning Information Retrieval Request

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServProvReq'

responses:

'200':

description: >

OK. The requested service provisioning information shall be returned in the response

body.

content:

application/json:

schema:

$ref: '#/components/schemas/ServProvResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions:

post:

summary: Request the creation of a Service Provisioning Subscription.

operationId: CreateServProvSubsc

tags:

- Service Provisioning Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServProvSubsc'

responses:

'201':

description: >

Created. The Service Provisioning Subscription is successfully created and a

representation of the created Individual Service Provisioning Subscription resource

shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/ServProvSubsc'

headers:

Location:

description: >

Contains the URI of the created Individual Service Provisioning Subscription

resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

ServProvNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServProvNotif'

responses:

'204':

description: >

No Content. The Service Provisioning Notification is successfully received and

acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscId}:

parameters:

- name: subscId

in: path

description: >

Represents the identifier of the Individual Service Provisioning Subscription resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Service Provisioning Subscription resource.

operationId: GetIndServProvSubsc

tags:

- Individual Service Provisioning Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual Service Provisioning Subscription resource shall be

returned.

content:

application/json:

schema:

$ref: '#/components/schemas/ServProvSubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Service Provisioning Subscription resource.

operationId: UpdateIndServProvSubsc

tags:

- Individual Service Provisioning Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServProvSubsc'

responses:

'200':

description: >

OK. The Individual Service Provisioning Subscription resource is successfully updated

and a representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ServProvSubsc'

'204':

description: >

No Content. The Individual Service Provisioning Subscription resource is successfully

updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Service Provisioning Subscription resource.

operationId: ModifyIndServProvSubsc

tags:

- Individual Service Provisioning Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/ServProvSubscPatch'

responses:

'200':

description: >

OK. The Individual Service Provisioning Subscription resource is successfully modified

and a representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ServProvSubsc'

'204':

description: >

No Content. The Individual Service Provisioning Subscription resource is successfully

modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Service Provisioning Subscription resource.

operationId: DeleteIndServProvSubsc

tags:

- Individual Service Provisioning Subscription (Document)

responses:

'204':

description: >

No Content. The Individual Service Provisioning Subscription resource is successfully

deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

ServProvReq:

description: >

Represents a Service Provisioning information retrieval request.

type: object

properties:

fedInfo:

type: array

items:

$ref: '#/components/schemas/FederationInfo'

minItems: 1

acProfs:

type: array

items:

$ref: 'TS24558_Eees_EECRegistration.yaml#/components/schemas/ACProfile'

minItems: 1

appInfos:

type: array

items:

$ref: '#/components/schemas/AppInfo'

minItems: 1

connInfo:

type: array

items:

$ref: 'TS24558_Eecs_ServiceProvisioning.yaml#/components/schemas/ConnectivityInfo'

minItems: 1

locInfo:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

ServProvResp:

description: >

Represents a Service Provisioning information retrieval response.

type: object

properties:

ednConfigInfo:

type: array

items:

$ref: 'TS24558_Eecs_ServiceProvisioning.yaml#/components/schemas/EDNConfigInfo'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- ednConfigInfo

ServProvSubsc:

description: >

Represents a Service Provisioning Subscription.

type: object

properties:

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

fedInfo:

type: array

items:

$ref: '#/components/schemas/FederationInfo'

minItems: 1

acProfs:

type: array

items:

$ref: 'TS24558_Eees_EECRegistration.yaml#/components/schemas/ACProfile'

minItems: 1

connInfo:

type: array

items:

$ref: 'TS24558_Eecs_ServiceProvisioning.yaml#/components/schemas/ConnectivityInfo'

minItems: 1

locInfo:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notifUri

ServProvSubscPatch:

description: >

Represents the requested modifications to a Service Provisioning Subscription.

type: object

properties:

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

fedInfo:

type: array

items:

$ref: '#/components/schemas/FederationInfo'

minItems: 1

acProfs:

type: array

items:

$ref: 'TS24558_Eees_EECRegistration.yaml#/components/schemas/ACProfile'

minItems: 1

connInfo:

type: array

items:

$ref: 'TS24558_Eecs_ServiceProvisioning.yaml#/components/schemas/ConnectivityInfo'

minItems: 1

locInfo:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

ServProvNotif:

description: >

Represents a Service Provisioning Notification.

type: object

properties:

subscId:

type: string

ednConfigInfo:

type: array

items:

$ref: 'TS24558_Eecs_ServiceProvisioning.yaml#/components/schemas/EDNConfigInfo'

minItems: 1

lifetime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

required:

- subscId

- ednConfigInfo

FederationInfo:

description: >

Represents federation agreements related information.

type: object

properties:

federationId:

type: string

required:

- federationId

AppInfo:

description: >

Represents the application informations.

type: object

properties:

acProf:

$ref: 'TS24558_Eees_EECRegistration.yaml#/components/schemas/ACProfile'

appGrpProf:

$ref: '#/components/schemas/AppGrpProfile'

required:

- acProf

AppGrpProfile:

description: >

Represents application group profile.

type: object

properties:

appGrpId:

type: string

e2eRespTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

easId:

type: string

expGeoAreas:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

minItems: 1

description: A list of geographic area information.

required:

- appGrpId

- easId

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

# A.19	Eecs_ECSDiscovery API

openapi: 3.0.0

info:

title: ECS ECS Discovery Service

version: 1.1.0

description: |

API for ECS Discovery

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.558 V19.5.0 Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eecs-ecsdiscovery/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.558.

paths:

/ecs-info/discover:

post:

description: >

Provides ECS information requested by the service consumer.

operationId: GetECSDiscInfo

tags:

- ECS Information (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EcsInfoDiscoveryReq'

responses:

'200':

description: The requested ECS discovery information was returned successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/EcsInfoDiscoveryResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

ECSDiscNotif:

'{request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EcsInfoDiscNotif'

responses:

'204':

description: No Content (successful notification).

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

EcsInfoDiscoveryReq:

type: object

description: Represents the ECS Discovery request information.

properties:

ecsAddr:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

acProfs:

type: array

items:

$ref: 'TS24558_Eees_EECRegistration.yaml#/components/schemas/ACProfile'

minItems: 1

connInf:

type: array

items:

$ref: 'TS24558_Eecs_ServiceProvisioning.yaml#/components/schemas/ConnectivityInfo'

minItems: 1

ueLoc:

type: array

items:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

minItems: 1

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

duration:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

appGrpId:

type: string

required:

- ecsAddr

EcsInfoDiscoveryResp:

type: object

description: Represents the ECS Discovrey response.

properties:

ecsInfo:

type: array

items:

$ref: '#/components/schemas/EcsInfo'

minItems: 1

required:

- ecsInfo

EcsInfo:

type: object

description: Represents the discovered ECS information.

properties:

ecs:

$ref: '#/components/schemas/ECSProfile'

lifeTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

required:

- ecs

ECSProfile:

type: object

description: Represents the ECS profile information.

properties:

endPt:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

ecspId:

type: string

splVal:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SpatialValidityCond'

suppPlmns:

type: array

items:

$ref: '#/components/schemas/SupportedPlmn'

minItems: 1

required:

- endPt

SupportedPlmn:

type: object

description: Represents supported PLMN and the related ECSPs information.

properties:

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

suppEcsps:

type: array

items:

$ref: '#/components/schemas/SupportedEcsp'

minItems: 1

pduConf:

$ref: '#/components/schemas/PduConfiguration'

SupportedEcsp:

type: object

description: Represents the ECSPs information.

properties:

ecspId:

type: string

easIds:

type: array

items:

type: string

minItems: 1

required:

- ecspId

- easIds

PduConfiguration:

type: object

description: Represents the PDU configuration information of the ECS.

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

required:

- snssai

- dnn

EcsInfoDiscNotif:

type: object

description: Represents the ECS Discovery notification information.

properties:

ecsInfo:

type: array

items:

$ref: '#/components/schemas/EcsInfo'

minItems: 1

required:

- ecsInfo

# A.20	Eecs_ACREvents API

openapi: 3.0.0

info:

title: ACR Events Service

version: 1.0.0

description: |

ACR Events Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.558 V19.5.0; Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

servers:

- url: '{apiRoot}/eecs-ae/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/subscriptions:

post:

summary: Request the creation of an ACR Events Subscription.

operationId: CreateAcrEventsSubsc

tags:

- ACR Events Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AcrEventsSubsc'

responses:

'201':

description: >

Created. The ACR Events Subscription is successfully created and a

representation of the created Individual ACR Events Subscription resource

shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/AcrEventsSubsc'

headers:

Location:

description: >

Contains the URI of the created Individual ACR Events Subscription

resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

AcrEventsNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AcrEventsNotif'

responses:

'204':

description: >

No Content. The ACR Events Notification is successfully received and

acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual ACR Events Subscription resource.

operationId: GetIndAcrEventsSubsc

tags:

- Individual ACR Events Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual ACR Events Subscription resource shall be

returned.

content:

application/json:

schema:

$ref: '#/components/schemas/AcrEventsSubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual ACR Events Subscription resource.

operationId: UpdateIndAcrEventsSubsc

tags:

- Individual ACR Events Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AcrEventsSubsc'

responses:

'200':

description: >

OK. The Individual ACR Events Subscription resource is successfully updated

and a representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/AcrEventsSubsc'

'204':

description: >

No Content. The Individual ACR Events Subscription resource is successfully

updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual ACR Events Subscription resource.

operationId: ModifyIndAcrEventsSubsc

tags:

- Individual ACR Events Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/AcrEventsSubscPatch'

responses:

'200':

description: >

OK. The Individual ACR Events Subscription resource is successfully modified

and a representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/AcrEventsSubsc'

'204':

description: >

No Content. The Individual ACR Events Subscription resource is successfully

modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual ACR Events Subscription resource.

operationId: DeleteIndAcrEventsSubsc

tags:

- Individual ACR Events Subscription (Document)

responses:

'204':

description: >

No Content. The Individual ACR Events Subscription resource is successfully

deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

AcrEventsSubsc:

description: >

Represents a ACR Events Subscription Request.

type: object

properties:

requestorId:

type: string

events:

type: array

items:

$ref: '#/components/schemas/AcrEvent'

minItems: 1

ednInfo:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/EDNInfo'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- requestorId

- events

- notifUri

AcrEventsSubscPatch:

description: >

Represents the requested modifications to a ACR Events Subscription.

type: object

properties:

events:

type: array

items:

$ref: '#/components/schemas/AcrEvent'

minItems: 1

ednInfo:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/EDNInfo'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

expTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTimeRm'

AcrEventsNotif:

description: >

Represents a ACR Events Notification.

type: object

properties:

events:

type: array

items:

$ref: '#/components/schemas/AcrEvent'

minItems: 1

subscriptionId:

type: string

relocRatio:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- subscriptionId

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

AcrEvent:

anyOf:

- type: string

enum:

- EDN_OVERLOAD

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the ACR event.

Possible values are:

- EDN_OVERLOAD: Indicates that ACR event is EDN overload.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-01 | CT3#113e | C3-210300 |  |  |  | TS skeleton for Enabling Edge Applications, Application Programming Interface (API) specification; Stage 3. | 0.0.0 |
| 2021-01 | CT3#113e | C3-210322 |  |  |  | Inclusion of documents agreed in CT3#113e: C3-210182, C3-210301, C3-210321 | 0.1.0 |
| 2021-03 | CT3#114e | C3-211508 |  |  |  | Inclusion of documents agreed in CT3#114e: C3-211365, C3-211503, C3-211504, C3-211505, C3-211506, C3-211507 | 0.2.0 |
| 2021-04 | CT3#115e | C3-212520 |  |  |  | Inclusion of documents agreed in CT3#115e: C3-212367, C3-212368, C3-212369, C3-212370 | 0.3.0 |
| 2021-05 | CT3#116e | C3-213328 |  |  |  | Inclusion of documents agreed in CT3#116e: C3-213324, C3-213325, C3-213326, C3-213327, C3-213547 | 0.4.0 |
| 2021-08 | CT3#117e | C3-214571 |  |  |  | Inclusion of documents agreed in CT3#117e: C3-214387, C3-214513, C3-214389, C3-214570, C3-214388, C3-214390, C3-214391, C3-214392 | 0.5.0 |
| 2021-09 | CT#93e | CP-212164 |  |  |  | Version 1.0.0 created for presentation for information. | 1.0.0 |
| 2021-10 | CT3#118e | C3-215471 |  |  |  | Inclusion of documents agreed in CT3#118e: C3-215236, C3-215338, C3-215339, C3-215340, C3-215341, C3-215342, C3-215377, C3-215378 | 1.1.0 |
| 2021-11 | CT3#119e | C3-216515 |  |  |  | Inclusion of documents agreed in CT3#119e: C3-216182, C3-216347, C3-216348, C3-216385, C3-216386, C3-216396 | 1.2.0 |
| 2022-01 | CT3#119-Bis-e | C3-220447 |  |  |  | Inclusion of documents agreed in CT3#119-Bis-e: C3-220133, C3-220419, C3-220420, C3-220421, C3-220459, C3-220275, C3-220276 | 1.3.0 |
| 2022-02 | CT3#120e | C3-221510 |  |  |  | Inclusion of documents agreed in CT3#120e: C3-221401, C3-221635, C3-221408, C3-221636, C3-221407, C3-221637, C3-221630, C3-221631, C3-221632, C3-221633, C3-221634, C3-221410, C3-221726, C3-221727, C3-221687 | 1.4.0 |
| 2022-04 | CT3#121e | C3-222481 |  |  |  | Inclusion of documents agreed in CT3#121e: C3-222136, C3-222137, C3-222139, C3-222141, C3-222143, C3-222362, C3-222363, C3-222364, C3-222409, C3-222410, C3-222411, C3-222449, C3-222508, C3-222576 | 1.5.0 |
| 2022-05 | CT3#122e | C3-223504 |  |  |  | Inclusion of documents agreed in CT3#122e: C3-223195, C3-223321, C3-223323, C3-223325, C3-223326, C3-223327, C3-223331, C3-223334, C3-223336, C3-223337, C3-223575, C3-223606, C3-223607, C3-223608, C3-223646, C3-223647, C3-223648, C3-223649, C3-223650, C3-223675, C3-223676, C3-223677, C3-223691, C3-223756, C3-223781, C3-223792, C3-223793, C3-223794 | 1.6.0 |
| 2022-06 | CT#96 | CP-221100 |  |  |  | Presentation to TSG CT for approval | 2.0.0 |
| 2022-06 | CT#96 | CP-221100 |  |  |  | Approved by TSG CT | 17.0.0 |
| 2022-09 | CT#97e | CP-222097 | 0001 | 1 | F | Corrections to Eees_EASDiscovery_TeasDiscRequest operation | 17.1.0 |
| 2022-09 | CT#97e | CP-222097 | 0002 | 1 | F | Add failure handling descriptions for EES services | 17.1.0 |
| 2022-09 | CT#97e | CP-222097 | 0003 | 1 | F | Add failure handling descriptions for ECS services | 17.1.0 |
| 2022-09 | CT#97e | CP-222097 | 0004 | 1 | F | Add redirect descriptions for EES services | 17.1.0 |
| 2022-09 | CT#97e | CP-222097 | 0005 | 1 | F | Add redirect descriptions for ECS services | 17.1.0 |
| 2022-09 | CT#97e | CP-222097 | 0006 | 1 | F | Corrections on ACRDataStatus and ACTResultInfo | 17.1.0 |
| 2022-09 | CT#97e | CP-222097 | 0007 |  | F | Corrections on HTTP methods in EES services | 17.1.0 |
| 2022-09 | CT#97e | CP-222097 | 0008 |  | F | Correction on OpenenAPI Eees_ EECContextRelocation | 17.1.0 |
| 2022-09 | CT#97e | CP-222097 | 0009 |  | F | Corrections on OpenAPI Eees_EELManagedACR | 17.1.0 |
| 2022-09 | CT#97e | CP-222098 | 0010 | 1 | F | Corrections to EES APIs definition | 17.1.0 |
| 2022-09 | CT#97e | CP-222098 | 0011 | 1 | F | Corrections to Eees_EELManagedACR API definition | 17.1.0 |
| 2022-09 | CT#97e | CP-222097 | 0012 |  | F | Remove unused reference to OMA | 17.1.0 |
| 2022-09 | CT#97e | CP-222205 | 0014 | 1 | F | Correcting the name of the data type related to availability status information | 17.1.0 |
| 2022-09 | CT#97e | CP-222206 | 0015 | 2 | F | Clarifications to the "easId" | 17.1.0 |
| 2022-09 | CT#97e | CP-222098 | 0017 | 1 | F | Corrections to user consent revocation management for the Eees_UELocation API | 17.1.0 |
| 2022-09 | CT#97e | CP-222121 | 0018 |  | F | Update of info and externalDocs fields | 17.1.0 |
| 2022-12 | CT#98e | CP-223170 | 0020 | 1 | F | Miscellaneous corrections in EES, ECS services | 17.2.0 |
| 2022-12 | CT#98e | CP-223170 | 0021 | 1 | F | Correction in Eees_EECContextRelocation , Eees_EELManagedACR API | 17.2.0 |
| 2022-12 | CT#98e | CP-223170 | 0022 | 1 | F | Correction in Eees_UEIdentifier, Eees_ACRManagementEvent API | 17.2.0 |
| 2022-12 | CT#98e | CP-223170 | 0023 | 1 | F | Corrections on easId verification in update procedures in EES services | 17.2.0 |
| 2022-12 | CT#98e | CP-223170 | 0027 | 1 | F | Correction on user consent procedure description | 17.2.0 |
| 2022-12 | CT#98e | CP-223170 | 0033 | 1 | F | Editor’s note resolution for Eees_UEIdentifier | 17.2.0 |
| 2022-12 | CT#98e | CP-223170 | 0034 |  | F | Essential correction to Application Client Information | 17.2.0 |
| 2022-12 | CT#98e | CP-223170 | 0035 |  | F | Correction on attribute name within EndPoint | 17.2.0 |
| 2022-12 | CT#98e | CP-223170 | 0036 |  | F | Correction on request URI for Eees_EECContextRelocation_Push | 17.2.0 |
| 2022-12 | CT#98e | CP-223170 | 0037 | 1 | F | Corrections on Location information report | 17.2.0 |
| 2022-12 | CT#98e | CP-223170 | 0039 | 1 | F | Corrections on the Notification Destination URI | 17.2.0 |
| 2022-12 | CT#98e | CP-223170 | 0040 | 1 | F | Corrections on the Revocation Notification URI | 17.2.0 |
| 2022-12 | CT#98e | CP-223170 | 0041 | 1 | F | Missing supported features for Eees_UEIdentifier API | 17.2.0 |
| 2022-12 | CT#98e | CP-223170 | 0042 | 1 | F | Corrections for data types of Eees_ACRManagementEvent service | 17.2.0 |
| 2022-12 | CT#98e | CP-223170 | 0043 |  | F | Corrections for data types of Eees_SessionWithQoS service | 17.2.0 |
| 2022-12 | CT#98e | CP-223170 | 0044 | 1 | F | Corrections on presence of the attribute in UserInformation data type | 17.2.0 |
| 2022-12 | CT#98e | CP-223188 | 0050 |  | F | Update of info and externalDocs fields | 17.2.0 |
| 2022-12 | CT#98e | CP-223200 | 0024 | 1 | F | Correction on eesId verification in update procedure in ECS service | 18.0.0 |
| 2022-12 | CT#98e | CP-223200 | 0025 | 1 | F | Correction on ACRManagementEvent_Subscribe service operation | 18.0.0 |
| 2022-12 | CT#98e | CP-223200 | 0026 | 1 | F | Corrections on EES UE location information description | 18.0.0 |
| 2022-12 | CT#98e | CP-223198 | 0028 |  | F | Remove duplicated description for EELManagedACR | 18.0.0 |
| 2022-12 | CT#98e | CP-223185 | 0029 | 1 | F | Adding operationId and tags fields | 18.0.0 |
| 2022-12 | CT#98e | CP-223185 | 0030 | 1 | F | Eees_ACRManagementEvent and Eees_ACRStatusUpdate APIs: enumeration definitions | 18.0.0 |
| 2022-12 | CT#98e | CP-223185 | 0031 | 1 | F | Corrections on the description of PUT and PATCH | 18.0.0 |
| 2022-12 | CT#98e | CP-223199 | 0038 | 1 | F | Corrections on the apiNames | 18.0.0 |
| 2022-12 | CT#98e | CP-223185 | 0045 | 1 | F | Add the missing description fields of some attributes in the OpenAPI file | 18.0.0 |
| 2022-12 | CT#98e | CP-223190 | 0051 |  | F | Update of info and externalDocs fields | 18.0.0 |
| 2023-03 | CT#99 | CP-230156 | 0055 | 1 | B | Updates on location reporting in Eees_UELocation API | 18.1.0 |
| 2023-03 | CT#99 | CP-230156 | 0056 | 1 | F | Correction of the description fields in enumerations | 18.1.0 |
| 2023-03 | CT#99 | CP-230169 | 0057 | 1 | B | Seamless service continuity support for transport layer in the EAS | 18.1.0 |
| 2023-03 | CT#99 | CP-230138 | 0060 | 1 | A | Corrections to the EAS Type definition | 18.1.0 |
| 2023-03 | CT#99 | CP-230139 | 0061 | 1 | B | EAS profile update with general context holding time | 18.1.0 |
| 2023-03 | CT#99 | CP-230162 | 0063 |  | F | Update of info and externalDocs fields | 18.1.0 |
| 2023-06 | CT#100 | CP-231139 | 0064 | 1 | F | Updates to the EAS type for UAS services | 18.2.0 |
| 2023-06 | CT#100 | CP-231185 | 0065 | 1 | B | Completing the support of the seamless transport layer service continuity functionality | 18.2.0 |
| 2023-06 | CT#100 | CP-231144 | 0068 | 3 | B | ACR scenario re-selection after successful ACR | 18.2.0 |
| 2023-06 | CT#100 | CP-231310 | 0069 | 2 | B | ACR Selection event and update ACR start stop event | 18.2.0 |
| 2023-06 | CT#100 | CP-231144 | 0070 | 1 | B | EEC context update with UE mobility requirement | 18.2.0 |
| 2023-06 | CT#100 | CP-231237 | 0071 | 4 | B | Update EES Profile with Instantiable EAS information | 18.2.0 |
| 2023-06 | CT#100 | CP-231144 | 0072 | 1 | B | Completion of the definition of the general context holding time duration | 18.2.0 |
| 2023-06 | CT#100 | CP-231321 | 0073 | 3 | B | Defining the EAS bundle information | 18.2.0 |
| 2023-06 | CT#100 | CP-231328 | 0075 | 2 | B | Enhancements to ACR management events | 18.2.0 |
| 2023-06 | CT#100 | CP-231144 | 0076 | 1 | B | Exposure of EAS service APIs via CAPIF | 18.2.0 |
| 2023-06 | CT#100 | CP-231144 | 0077 |  | B | Support for federation and roaming | 18.2.0 |
| 2023-06 | CT#100 | CP-231312 | 0078 | 2 | B | Updates to Eees_ACRManagementEvent API | 18.2.0 |
| 2023-06 | CT#100 | CP-231144 | 0079 | 1 | B | Updates to Eees_SessionWithQoS API | 18.2.0 |
| 2023-06 | CT#100 | CP-231144 | 0080 |  | B | Supporting SNPN for the EES Topological Service Area | 18.2.0 |
| 2023-06 | CT#100 | CP-231153 | 0084 | 1 | A | Correction to EndPoint | 18.2.0 |
| 2023-06 | CT#100 | CP-231153 | 0086 |  | A | Correct data type in Eees_EASRegistration API | 18.2.0 |
| 2023-06 | CT#100 | CP-231144 | 0088 | 1 | B | Add consumer for Eees_UEIdentifier and support EDGE-10 | 18.2.0 |
| 2023-06 | CT#100 | CP-231139 | 0090 |  | F | Corrections on certain terminology | 18.2.0 |
| 2023-06 | CT#100 | CP-231139 | 0091 |  | F | Corrections on supported feature in EDGEAPP APIs | 18.2.0 |
| 2023-06 | CT#100 | CP-231144 | 0093 | 1 | B | Definition of the service description clauses of the Eees_ACRParameterInformation API | 18.2.0 |
| 2023-06 | CT#100 | CP-231144 | 0094 | 1 | B | Definition of the API clauses of the Eees_ACRParameterInformation API | 18.2.0 |
| 2023-06 | CT#100 | CP-231144 | 0095 | 1 | B | Definition of the OpenAPI description of the Eees_ACRParameterInformation API | 18.2.0 |
| 2023-06 | CT#100 | CP-231320 | 0097 | 1 | A | Corrections to the ACT start/stop event related information | 18.2.0 |
| 2023-06 | CT#100 | CP-231134 | 0098 | 1 | D | Clarifications to the EAS ID definition | 18.2.0 |
| 2023-06 | CT#100 | CP-231141 | 0100 |  | F | Update of info and externalDocs fields | 18.2.0 |
| 2023-09 | CT#101 | CP-232089 | 0101 | 1 | B | Service continuity support related updates Eecs_TargetEESDiscovery_Request service operation | 18.3.0 |
| 2023-09 | CT#101 | CP-232089 | 0102 | 1 | B | Composite EAS support related updates | 18.3.0 |
| 2023-09 | CT#101 | CP-232089 | 0103 | 1 | B | Resolving the SelectedACRScenarios data type Editor's Notes | 18.3.0 |
| 2023-09 | CT#101 | CP-232089 | 0104 |  | B | Supporting the indication of service continuity planning | 18.3.0 |
| 2023-09 | CT#101 | CP-232089 | 0105 | 1 | F | Corrections to the definition of the Eees_ACRParameterInformation API | 18.3.0 |
| 2023-09 | CT#101 | CP-232089 | 0106 |  | F | Instantiable EAS information related updates | 18.3.0 |
| 2023-09 | CT#101 | CP-232089 | 0107 | 1 | B | Selected EES declaration service descriptions and procedures | 18.3.0 |
| 2023-09 | CT#101 | CP-232089 | 0108 | 1 | B | Selected EES declaration service API Definitions | 18.3.0 |
| 2023-09 | CT#101 | CP-232254 | 0109 | 2 | B | Selected EES declaration service OpenAPI file definitions | 18.3.0 |
| 2023-09 | CT#101 | CP-232089 | 0113 | 1 | B | Update the Eecs_TargetEESDiscovery to support ECI-3 | 18.3.0 |
| 2023-09 | CT#101 | CP-232253 | 0114 | 2 | B | Update the Eecs_TargetEESDiscovery API to support ENS | 18.3.0 |
| 2023-09 | CT#101 | CP-232089 | 0115 | 1 | B | Update the Eees_EASDiscovery_TEasDiscRequest to support EAS synchronization | 18.3.0 |
| 2023-09 | CT#101 | CP-232089 | 0116 | 1 | B | Update to ACR parameter information procedure to support ECI-4 | 18.3.0 |
| 2023-09 | CT#101 | CP-232089 | 0117 | 1 | B | Update to EEC context relocation to support ECI-4 | 18.3.0 |
| 2023-09 | CT#101 | CP-232089 | 0118 |  | B | Common EAS announcement | 18.3.0 |
| 2023-09 | CT#101 | CP-232085 | 0121 |  | F | Update of info and externalDocs fields | 18.3.0 |
| 2023-12 | CT#102 | CP-233241 | 0089 | 5 | B | Updates to Eees_UEIdentifier API | 18.4.0 |
| 2023-12 | CT#102 | CP-233241 | 0122 |  | B | Remove the remaining Editor's Note on EAS bundle requirements | 18.4.0 |
| 2023-12 | CT#102 | CP-233241 | 0123 |  | B | Remove the remaining Editor's Note on the ACRScenario data type | 18.4.0 |
| 2023-12 | CT#102 | CP-233241 | 0124 | 3 | B | Remove the remaining Editor's Notes on the "servContPlanInd" attribute | 18.4.0 |
| 2023-12 | CT#102 | CP-233241 | 0125 |  | B | Remove the remaining Editor's Note on the "acIds" attribute | 18.4.0 |
| 2023-12 | CT#102 | CP-233241 | 0126 | 2 | B | Complete the definition of the ACR Selection event | 18.4.0 |
| 2023-12 | CT#102 | CP-233241 | 0127 |  | B | Resolve the remaining Editor's Note on the "teasEndPoint" attribute | 18.4.0 |
| 2023-12 | CT#102 | CP-233273 | 0129 | 1 | A | Corrections to boolean type definitions | 18.4.0 |
| 2023-12 | CT#102 | CP-233241 | 0130 | 1 | B | ACR Management subscription API update | 18.4.0 |
| 2023-12 | CT#102 | CP-233241 | 0132 | 1 | B | ACRManagementEvent - EAS ack and service continuity | 18.4.0 |
| 2023-12 | CT#102 | CP-233241 | 0133 | 1 | B | Updates to Common EAS Announcement service | 18.4.0 |
| 2023-12 | CT#102 | CP-233241 | 0134 | 1 | B | Eees_EASRegistration - EAS sync and Bundle updates | 18.4.0 |
| 2023-12 | CT#102 | CP-233241 | 0136 | 1 | B | Retreive EES using application group identifier | 18.4.0 |
| 2023-12 | CT#102 | CP-233287 | 0138 | 1 | F | SEALDD feature related updates | 18.4.0 |
| 2023-12 | CT#102 | CP-233241 | 0139 | 1 | B | Corrections and updates to the EAS bundle information | 18.4.0 |
| 2023-12 | CT#102 | CP-233241 | 0140 | 1 | B | Adding EDN information to the EES profile | 18.4.0 |
| 2023-12 | CT#102 | CP-233241 | 0141 | 1 | B | Support indicating the EAS ability of handling bundled EAS ACR | 18.4.0 |
| 2023-12 | CT#102 | CP-233241 | 0142 |  | F | Corrections to the definition of the general context holding time duration | 18.4.0 |
| 2023-12 | CT#102 | CP-233241 | 0143 | 1 | B | Resolve ENs in Eees_ACRManagementEvent API | 18.4.0 |
| 2023-12 | CT#102 | CP-233241 | 0144 | 1 | B | Resolve ENs in Eees_SessionWithQoS API | 18.4.0 |
| 2023-12 | CT#102 | CP-233242 | 0145 | 1 | B | Update ACR management event notification | 18.4.0 |
| 2023-12 | CT#102 | CP-233231 | 0146 |  | F | Miscellaneous Corrections | 18.4.0 |
| 2023-12 | CT#102 | CP-233241 | 0147 | 1 | B | Instantiation criteria update and add missing abbreviations | 18.4.0 |
| 2023-12 | CT#102 | CP-233237 | 0149 |  | F | Update of info and externalDocs fields | 18.4.0 |
| 2024-03 | CT#103 | CP-240168 | 0110 | 5 | B | EAS Information Management service descriptions and procedures | 18.5.0 |
| 2024-03 | CT#103 | CP-240168 | 0111 | 5 | B | EAS Information Management service API Definitions | 18.5.0 |
| 2024-03 | CT#103 | CP-240168 | 0112 | 3 | B | EAS Information Management service OpenAPI Definitions | 18.5.0 |
| 2024-03 | CT#103 | CP-240168 | 0150 | 1 | B | Updates to EAS ID in ACR management event notification | 18.5.0 |
| 2024-03 | CT#103 | CP-240168 | 0151 | 1 | B | Resolve EN for interworking with cloud services | 18.5.0 |
| 2024-03 | CT#103 | CP-240168 | 0153 | 1 | B | Complete the definition of the Eees_CommonEASAnnouncement API | 18.5.0 |
| 2024-03 | CT#103 | CP-240168 | 0154 |  | B | Complete the definition of the new ACR Selection event and the updates to the ACT Start/Stop event | 18.5.0 |
| 2024-03 | CT#103 | CP-240168 | 0155 | 2 | B | Updates to the support of the cloud entities and services | 18.5.0 |
| 2024-03 | CT#103 | CP-240168 | 0156 | 1 | B | Application Port ID for Eees_UEIdentifier API | 18.5.0 |
| 2024-03 | CT#103 | CP-240168 | 0157 | 1 | F | Correction for Eees_UEIdentifier API | 18.5.0 |
| 2024-03 | CT#103 | CP-240168 | 0158 | 1 | F | Correction for the typo | 18.5.0 |
| 2024-03 | CT#103 | CP-240168 | 0159 | 1 | B | Trigger parameters for Eees_AppClientInformation API | 18.5.0 |
| 2024-03 | CT#103 | CP-240168 | 0160 | 4 | B | Application traffic influence trigger from EAS | 18.5.0 |
| 2024-03 | CT#103 | CP-240168 | 0161 | 1 | F | Corrections to Eees_UEIdentifier API | 18.5.0 |
| 2024-03 | CT#103 | CP-240168 | 0162 | 1 | F | CAS decided ACR for CESless case | 18.5.0 |
| 2024-03 | CT#103 | CP-240168 | 0163 | 1 | F | AC information exposure updates | 18.5.0 |
| 2024-03 | CT#103 | CP-240168 | 0164 | 1 | F | Miscellaneous alignments | 18.5.0 |
| 2024-03 | CT#103 | CP-240197 | 0166 | 1 | A | Correction to incorrect attribute name capitalization | 18.5.0 |
| 2024-03 | CT#103 | CP-240173 | 0167 | 1 | F | Corrections to ConsentRevoked | 18.5.0 |
| 2024-03 | CT#103 | CP-240166 | 0168 |  | F | Update of info and externalDocs fields | 18.5.0 |
| 2024-06 | CT#104 | CP-241106 | 0119 | 7 | B | ECS Registration and Deregistration | 18.6.0 |
| 2024-06 | CT#104 | CP-241256 | 0120 | 10 | B | ECS Discovery | 18.6.0 |
| 2024-06 | CT#104 | CP-241106 | 0152 | 4 | B | Definition of Eecs_ECSServiceProvisioning service API | 18.6.0 |
| 2024-06 | CT#104 | CP-241083 | 0169 |  | F | Several OpenAPI Corrections | 18.6.0 |
| 2024-06 | CT#104 | CP-241084 | 0170 | 1 | B | Complete the definition of the EAS ID and EAS type to support application layer framworks deployment | 18.6.0 |
| 2024-06 | CT#104 | CP-241083 | 0171 | 1 | B | Updates to the definition of the IndividualSessionContext data type | 18.6.0 |
| 2024-06 | CT#104 | CP-241106 | 0172 |  | F | Various corrections to the definition of the Eees_TrafficInfluenceEAS API | 18.6.0 |
| 2024-06 | CT#104 | CP-241106 | 0174 | 1 | F | Update to CES re-used service APIs | 18.6.0 |
| 2024-06 | CT#104 | CP-241106 | 0176 | 1 | B | Updates to EAS Profile | 18.6.0 |
| 2024-06 | CT#104 | CP-241106 | 0178 | 1 | B | Updates to EAS synchronization | 18.6.0 |
| 2024-06 | CT#104 | CP-241135 | 0179 | 1 | A | Correction in Eees_UELocation API | 18.6.0 |
| 2024-06 | CT#104 | CP-241084 | 0180 | 1 | F | Correction on P and Cardinality column in the Data type | 18.6.0 |
| 2024-06 | CT#104 | CP-241084 | 0182 | 1 | F | Error in redirection handling of UE Identifier request message | 18.6.0 |
| 2024-06 | CT#104 | CP-241106 | 0183 | 2 | F | Various essential corrections | 18.6.0 |
| 2024-06 | CT#104 | CP-241084 | 0184 | 1 | B | Updates error handling in Eees_UEIdentifier API | 18.6.0 |
| 2024-06 | CT#104 | CP-241106 | 0185 | 1 | B | Updates to Eees_TrafficInfluenceEAS service | 18.6.0 |
| 2024-06 | CT#104 | CP-241106 | 0186 |  | F | Correction of data type of ProblemDetailsEIMExt | 18.6.0 |
| 2024-06 | CT#104 | CP-241086 | 0188 |  | F | Update of info and externalDocs fields | 18.6.0 |
| 2024-09 | CT#105 | CP-242161 | 0208 | 1 | F | Various updates and corrections | 18.7.0 |
| 2024-09 | CT#105 | CP-242120 | 0220 |  | F | Update of info and externalDocs fields | 18.7.0 |
| 2024-09 | CT#105 | CP-242116 | 0192 | 1 | B | Updates to the EAS discovery procedures to support edge node sharing | 19.0.0 |
| 2024-09 | CT#105 | CP-242116 | 0193 |  | B | Updates to support the prediction expiration time in ACR scenarios | 19.0.0 |
| 2024-09 | CT#105 | CP-242116 | 0194 | 1 | B | Support service KPIs and EDN information in common EAS announcement procedures | 19.0.0 |
| 2024-09 | CT#105 | CP-242116 | 0195 | 1 | B | Support group connection information synchronization in common EAS announcement procedures | 19.0.0 |
| 2024-09 | CT#105 | CP-242116 | 0196 | 1 | B | Support new service consumers for the Eees_ACRManagementEvent API | 19.0.0 |
| 2024-09 | CT#105 | CP-242116 | 0197 | 1 | B | Support new service consumers for the Eees_EELManagedACR API | 19.0.0 |
| 2024-09 | CT#105 | CP-242116 | 0198 | 1 | B | Support new service consumers for the Eees_ACRStatusUpdate API | 19.0.0 |
| 2024-09 | CT#105 | CP-242116 | 0200 | 1 | B | Support common EAS relocation | 19.0.0 |
| 2024-09 | CT#105 | CP-242116 | 0201 | 1 | B | Support Update operation and common EAS bundle in Eecs_EASInfoManagement Service | 19.0.0 |
| 2024-09 | CT#105 | CP-242116 | 0202 | 1 | B | Support Remove operation in Eecs_EASInfoManagement Service | 19.0.0 |
| 2024-09 | CT#105 | CP-242116 | 0203 | 1 | B | Eecs_ACREvents Service and Procedures | 19.0.0 |
| 2024-09 | CT#105 | CP-242116 | 0204 | 1 | B | Eecs_ACREvents API definitions | 19.0.0 |
| 2024-09 | CT#105 | CP-242116 | 0205 | 1 | B | Eecs_ACREvents OpenAPI definitions | 19.0.0 |
| 2024-09 | CT#105 | CP-242116 | 0206 | 1 | B | Updates to support EAS load information reporting | 19.0.0 |
| 2024-09 | CT#105 | CP-242116 | 0207 | 1 | B | Add EES as consumer for Edge Node Sharing | 19.0.0 |
| 2024-09 | CT#105 | CP-242116 | 0209 | 1 | B | Support of Requestor Identifier for Eees_UEIdentifier API | 19.0.0 |
| 2024-09 | CT#105 | CP-242114 | 0210 | 1 | B | API requester identifier | 19.0.0 |
| 2024-09 | CT#105 | CP-242219 | 0211 | 2 | B | Out of service area ACR management event | 19.0.0 |
| 2024-09 | CT#105 | CP-242116 | 0212 | 1 | B | Provides list of UE IDs in ACR management notification | 19.0.0 |
| 2024-09 | CT#105 | CP-242116 | 0213 | 1 | B | Service continuity for common EAS serving UE(s) of the same application group | 19.0.0 |
| 2024-09 | CT#105 | CP-242117 | 0214 | 1 | B | Update EES as consumer of EES_AppContextRelocation service | 19.0.0 |
| 2024-09 | CT#105 | CP-242116 | 0218 | 1 | B | Eecs_ECSServiceProvisioning – Common EAS in partner ECSP | 19.0.0 |
| 2024-09 | CT#105 | CP-242113 | 0221 |  | F | Update of info and externalDocs fields | 19.0.0 |
| 2024-12 | CT#106 | CP-243085 | 0222 |  | F | Various necessary updates and corrections to EDGEAPP APIs | 19.1.0 |
| 2024-12 | CT#106 | CP-243096 | 0223 | 1 | F | Corrections to the encoding of EAS load level | 19.1.0 |
| 2024-12 | CT#106 | CP-243096 | 0224 | 2 | B | Updates and corrections to the allowed MNO information for an EAS | 19.1.0 |
| 2024-12 | CT#106 | CP-243096 | 0225 |  | F | Eees_ACRStatusUpdate – Minor correction | 19.1.0 |
| 2024-12 | CT#106 | CP-243096 | 0226 | 1 | B | Application group profile – end to end response time | 19.1.0 |
| 2024-12 | CT#106 | CP-243109 | 0228 |  | A | Corrections on the federation agreements related information | 19.1.0 |
| 2024-12 | CT#106 | CP-243085 | 0229 | 1 | F | Corrections on Eecs_ECSDiscovery API | 19.1.0 |
| 2024-12 | CT#106 | CP-243086 | 0230 | 2 | F | Corrections on the Eecs_EASInfoManagement API | 19.1.0 |
| 2024-12 | CT#106 | CP-243085 | 0231 |  | F | Corrections on the Uri description | 19.1.0 |
| 2024-12 | CT#106 | CP-243086 | 0233 | 1 | F | Corrections to Eees_EECContextRelocation Service | 19.1.0 |
| 2024-12 | CT#106 | CP-243085 | 0235 | 1 | F | Corrections to EES consumes Eees_AppContextRelocation service | 19.1.0 |
| 2024-12 | CT#106 | CP-243096 | 0236 | 1 | B | Add Application Group Id in the Eees_ACRManagementEvent API | 19.1.0 |
| 2024-12 | CT#106 | CP-243096 | 0237 | 1 | B | Updates to the bundle EAS functionality | 19.1.0 |
| 2024-12 | CT#106 | CP-243096 | 0238 | 1 | F | Terminology alignment for bundled EAS coordinated ACRs | 19.1.0 |
| 2024-12 | CT#106 | CP-243096 | 0239 | 1 | F | Event Reporting Information clarification for out of service area event | 19.1.0 |
| 2024-12 | CT#106 | CP-243083 | 0240 | 1 | B | Tunnel information in retrieve T-EES procedure | 19.1.0 |
| 2024-12 | CT#106 | CP-243086 | 0241 | 1 | F | EDGEAPP – Correcting structured data types in query parameters | 19.1.0 |
| 2024-12 | CT#106 | CP-243096 | 0242 | 1 | B | ACR status update – Application group ID | 19.1.0 |
| 2024-12 | CT#106 | CP-243096 | 0243 | 1 | B | ECS service provisioning information – Location clarification | 19.1.0 |
| 2024-12 | CT#106 | CP-243096 | 0245 | 1 | B | Resolve ENs on AppGroupInfo data type | 19.1.0 |
| 2024-12 | CT#106 | CP-243281 | 0246 | 2 | B | Resolve EN on AppGrpProfile data type | 19.1.0 |
| 2024-12 | CT#106 | CP-243139 | 0248 | 1 | B | Support of the EES profile for satellite information | 19.1.0 |
| 2024-12 | CT#106 | CP-243126 | 0251 |  | A | Corrections on the AC Filters | 19.1.0 |
| 2024-12 | CT#106 | CP-243109 | 0253 |  | A | Corrections on the end point | 19.1.0 |
| 2024-12 | CT#106 | CP-243109 | 0255 |  | A | Corrections on the UE Id | 19.1.0 |
| 2024-12 | CT#106 | CP-243096 | 0256 |  | F | Corrections on the status of load warning | 19.1.0 |
| 2024-12 | CT#106 | CP-243086 | 0257 | 1 | F | Corrections on the API names | 19.1.0 |
| 2024-12 | CT#106 | CP-243087 | 0258 | 1 | F | Various corrections on Eees service | 19.1.0 |
| 2024-12 | CT#106 | CP-243147 | 0261 |  | F | Update of info and externalDocs fields | 19.1.0 |
| 2025-03 | CT#107 | CP-250101 | 0263 | 1 | F | Removal of suppFeat attribute in AcrEventsSubscPatch data type | 19.2.0 |
| 2025-03 | CT#107 | CP-250101 | 0264 | 1 | B | Updates to Eecs_EASInfoManagement API | 19.2.0 |
| 2025-03 | CT#107 | CP-250085 | 0265 |  | F | Corrections to LocationSubscription and UeIdInfo data types | 19.2.0 |
| 2025-03 | CT#107 | CP-250085 | 0266 | 1 | F | Corrections to TrafficFilterInfo data type | 19.2.0 |
| 2025-03 | CT#107 | CP-250085 | 0267 | 1 | F | Corrections to EcsInfoDiscoveryReq data type | 19.2.0 |
| 2025-03 | CT#107 | CP-250101 | 0268 |  | F | Correct cardinality of the allowedMNOsInfo attribute | 19.2.0 |
| 2025-03 | CT#107 | CP-250101 | 0269 | 1 | B | Eecs_ECSDiscovery – Support of Application group identifier | 19.2.0 |
| 2025-03 | CT#107 | CP-250101 | 0270 | 1 | B | Eees_EASRegistration – Minor updates | 19.2.0 |
| 2025-03 | CT#107 | CP-250130 | 0272 |  | F | Update of info and externalDocs fields | 19.2.0 |
| 2025-06 | CT#108 | CP-251084 | 0273 | 1 | F | Complete the updates to ensure easy extensibility for the Eees_EASRegistration API | 19.3.0 |
| 2025-06 | CT#108 | CP-251100 | 0274 |  | F | Feature description alignment | 19.3.0 |
| 2025-06 | CT#108 | CP-251096 | 0275 | 1 | B | Complete the updates to the EES profile to support satellite information | 19.3.0 |
| 2025-06 | CT#108 | CP-251084 | 0276 | 1 | F | Correction to the incorrect description of the boolean type for the mandatory attribute | 19.3.0 |
| 2025-06 | CT#108 | CP-251100 | 0277 | 2 | B | Resolving EN for Eees_EASInformationProvisioning Service | 19.3.0 |
| 2025-06 | CT#108 | CP-251128 | 0278 | 1 | B | EAS profile update to support metaverse services requiring multiple devices | 19.3.0 |
| 2025-06 | CT#108 | CP-251100 | 0279 | 1 | B | Define the candidate DNAIs related information for the common EAS cases | 19.3.0 |
| 2025-06 | CT#108 | CP-251232 | 0281 |  | F | Update of info and externalDocs fields | 19.3.0 |
| 2025-09 | CT#109 | CP-252084 | 0282 | 1 | B | Complete the definition of the Satellite Coverage Availability information | 19.4.0 |
| 2025-09 | CT#109 | CP-252105 | 0283 | 1 | F | Correction to Eees_EASDiscovery API | 19.4.0 |
| 2025-09 | CT#109 | CP-252075 | 0284 | 1 | F | Corrections to Supported Features | 19.4.0 |
| 2025-09 | CT#109 | CP-252114 | 0285 |  | F | Update of info and externalDocs fields | 19.4.0 |
| 2025-12 | CT#110 | CP-253026 | 0286 | 1 | B | Updates and corrections to the CAPIF related Security clause | 19.5.0 |
| 2025-12 | CT#110 | CP-253036 | 0287 |  | F | Removal of unused references in TS 29.558 | 19.5.0 |
| 2025-12 | CT#110 | CP-253027 | 0288 | 1 | F | Corrections to Eees_SessionWithQoS API | 19.5.0 |
| 2025-12 | CT#110 | CP-253027 | 0289 |  | F | Corrections to other EES APIs | 19.5.0 |
| 2025-12 | CT#110 | CP-253027 | 0290 | 1 | F | Corrections to ECS APIs | 19.5.0 |
| 2025-12 | CT#110 | CP-253065 | 0291 | 1 | F | Update of info and externalDocs fields | 19.5.0 |