| 3GPP TS 29.530 V19.1.0 (2026-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Application Function Artificial Intelligence/Machine Learning (AI/ML) Services; Stage 3 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2026, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

1	Scope	10

2	References	10

3	Definitions, symbols and abbreviations	11

3.1	Definitions	11

3.2	Symbols	11

3.3	Abbreviations	11

4	Overview	11

5	Services offered by the AF	12

5.1	Introduction	12

5.2	Naf_VFLTraining Service	12

5.2.1	Service Description	12

5.2.1.1	Overview	12

5.2.1.2	Service Architecture	13

5.2.1.3	Network Functions	13

5.2.1.3.1	Application Function (AF)	13

5.2.1.3.2	NF Service Consumers	13

5.2.2	Service Operations	14

5.2.2.1	Introduction	14

5.2.2.2	Naf_VFLTraining_Subscribe	14

5.2.2.2.1	General	14

5.2.2.2.2	VFL Training Subscription Creation	14

5.2.2.2.3	VFL Training Subscription Update	15

5.2.2.3	Naf_VFLTraining_Unsubscribe	15

5.2.2.3.1	General	15

5.2.2.3.2	VFL Training Subscription Deletion	16

5.2.2.4	Naf_VFLTraining_Notify	16

5.2.2.4.1	General	16

5.2.2.4.2	VFL Training Notification	16

5.3	Naf_VFLInference Service	17

5.3.1	Service Description	17

5.3.1.1	Overview	17

5.3.1.2	Service Architecture	17

5.3.1.3	Network Functions	18

5.3.1.3.1	Application Function (AF)	18

5.3.1.3.2	NF Service Consumers	18

5.3.2	Service Operations	18

5.3.2.1	Introduction	18

5.3.2.2	Naf_VFLInference_Subscribe service operation	18

5.3.2.2.1	General	18

5.3.2.2.2	Subscription for VFL inference event notifications	18

5.3.2.2.3	Update subscription for event notifications	19

5.3.2.2.4	Partial update subscription for event notifications	20

5.3.2.3	Naf_VFLInference_Unsubscribe service operation	21

5.3.2.3.1	General	21

5.3.2.3.2	Unsubscribe from VFL inference notifications	21

5.3.2.4	Naf_VFLInference_Notify service operation	22

5.3.2.4.1	General	22

5.3.2.4.2	Notification about subscribed event	22

5.4	Naf_Training Service	23

5.4.1	Service Description	23

5.4.1.1	Overview	23

5.4.1.2	Service Architecture	23

5.4.1.3	Network Functions	24

5.4.1.3.1	Application Function (AF)	24

5.4.1.3.2	NF Service Consumers	24

5.4.2	Service Operations	24

5.4.2.1	Introduction	24

5.4.2.2	Naf_Training_Subscribe	25

5.4.2.2.1	General	25

5.4.2.2.2	Training Subscription Creation	25

5.4.2.2.3	Training Subscription Update	25

5.4.2.3	Naf_Training_Unsubscribe	26

5.4.2.3.1	General	26

5.4.2.3.2	Training Subscription Deletion	26

5.4.2.4	Naf_Training_Notify	27

5.4.2.4.1	General	27

5.4.2.4.2	Training Notification	27

5.5	Naf_Inference Service	27

5.5.1	Service Description	27

5.5.1.1	Overview	27

5.5.1.2	Service Architecture	28

5.5.1.3	Network Functions	28

5.5.1.3.1	Application Function (AF)	28

5.5.1.3.2	NF Service Consumers	28

5.5.2	Service Operations	29

5.5.2.1	Introduction	29

5.5.2.2	Naf_Inference_Subscribe	29

5.5.2.2.1	General	29

5.5.2.2.2	Inference Subscription Creation	29

5.5.2.2.3	Inference Subscription Update	30

5.5.2.3	Naf_Inference_Unsubscribe	31

5.5.2.3.1	General	31

5.5.2.3.2	Inference Subscription Deletion	31

5.5.2.4	Naf_Inference_Notify	32

5.5.2.4.1	General	32

5.5.2.4.2	Inference Notification	32

6	API Definitions	32

6.1	Naf_VFLTraining Service API	32

6.1.1	Introduction	32

6.1.2	Usage of HTTP	33

6.1.2.1	General	33

6.1.2.2	HTTP standard headers	33

6.1.2.2.1	General	33

6.1.2.2.2	Content type	33

6.1.2.3	HTTP custom headers	33

6.1.3	Resources	33

6.1.3.1	Overview	33

6.1.3.2	Resource: VFL Training Subscriptions	34

6.1.3.2.1	Description	34

6.1.3.2.2	Resource Definition	34

6.1.3.2.3	Resource Standard Methods	35

6.1.3.2.4	Resource Custom Operations	35

6.1.3.3	Resource: Individual VFL Training Subscription	35

6.1.3.3.1	Description	35

6.1.3.3.2	Resource Definition	35

6.1.3.3.3	Resource Standard Methods	36

6.1.4	Custom Operations without associated resources	41

6.1.5	Notifications	41

6.1.5.1	General	41

6.1.5.2	VFL Training Notification	41

6.1.5.2.1	Description	41

6.1.5.2.2	Target URI	41

6.1.5.2.3	Standard Methods	41

6.1.6	Data Model	42

6.1.6.1	General	42

6.1.6.2	Structured data types	43

6.1.6.2.1	Introduction	43

6.1.6.2.2	Type: VflTrainingSubs	44

6.1.6.2.3	Type: VflTrainingSubsPatch	44

6.1.6.3	Simple data types and enumerations	45

6.1.6.3.1	Introduction	45

6.1.6.3.2	Simple data types	45

6.1.6.4	Data types describing alternative data types or combinations of data types	45

6.1.6.5	Binary data	45

6.1.6.5.1	Binary Data Types	45

6.1.7	Error Handling	45

6.1.7.1	General	45

6.1.7.2	Protocol Errors	45

6.1.7.3	Application Errors	45

6.1.8	Feature negotiation	46

6.1.9	Security	46

6.1.10	HTTP redirection	46

6.2	Naf_VFLInference Service API	46

6.2.1	Introduction	46

6.2.2	Usage of HTTP	47

6.2.2.1	General	47

6.2.2.2	HTTP standard headers	47

6.2.2.2.1	General	47

6.2.2.2.2	Content type	47

6.2.2.3	HTTP custom headers	47

6.2.3	Resources	47

6.2.3.1	Resource Structure	47

6.2.3.2	Resource: VFL Inference Subscriptions	48

6.2.3.2.1	Description	48

6.2.3.2.2	Resource definition	48

6.2.3.2.3	Resource Standard Methods	49

6.2.3.2.4	Resource Custom Operations	49

6.2.3.3	Resource: Individual VFL Inference Subscription	49

6.2.3.3.1	Description	49

6.2.3.3.2	Resource definition	49

6.2.3.3.3	Resource Standard Methods	50

6.2.3.3.4	Resource Custom Operations	54

6.2.4	Custom Operations without associated resources	54

6.2.5	Notifications	55

6.2.5.1	General	55

6.2.5.2	VFL Inference Event Notification	55

6.2.5.2.1	Description	55

6.2.5.2.2	Operation Definition	55

6.2.6	Data Model	56

6.2.6.1	General	56

6.2.6.2	Structured data types	57

6.2.6.2.1	Introduction	57

6.2.6.2.2	Type VflInferSub	57

6.2.6.2.3	Type VflInferSubPatch	58

6.2.6.3	Simple data types and enumerations	58

6.2.6.3.1	Introduction	58

6.2.6.3.2	Simple data types	58

6.2.7	Error handling	58

6.2.7.1	General	58

6.2.7.2	Protocol Errors	58

6.2.7.3	Application Errors	58

6.2.8	Feature negotiation	59

6.2.9	Security	59

6.2.10	HTTP redirection	59

6.3	Naf_Training Service API	59

6.3.1	Introduction	59

6.3.2	Usage of HTTP	60

6.3.2.1	General	60

6.3.2.2	HTTP standard headers	60

6.3.2.2.1	General	60

6.3.2.2.2	Content type	60

6.3.2.3	HTTP custom headers	60

6.3.3	Resources	60

6.3.3.1	Overview	60

6.3.3.2	Resource: Training Subscriptions	61

6.3.3.2.1	Description	61

6.3.3.2.2	Resource Definition	61

6.3.3.2.3	Resource Standard Methods	61

6.3.3.2.4	Resource Custom Operations	62

6.3.3.3	Resource: Individual Training Subscription	62

6.3.3.3.1	Description	62

6.3.3.3.2	Resource Definition	62

6.3.3.3.3	Resource Standard Methods	63

6.3.4	Custom Operations without associated resources	67

6.3.5	Notifications	68

6.3.5.1	General	68

6.3.5.2	Training Notification	68

6.3.5.2.1	Description	68

6.3.5.2.2	Target URI	68

6.3.5.2.3	Standard Methods	68

6.3.6	Data Model	69

6.3.6.1	General	69

6.3.6.2	Structured data types	70

6.3.6.2.1	Introduction	70

6.3.6.2.2	Type: TrainEventsSubsc	71

6.3.6.2.3	Type: TrainEventsSubscPatch	71

6.3.6.2.4	Type: EventSubsc	72

6.3.6.2.5	Type: MlModelMonitorInfo	72

6.3.6.2.6	Type: TrainEventsNotif	73

6.3.6.2.7	Type: EventNotif	73

6.3.6.3	Simple data types and enumerations	74

6.3.6.3.1	Introduction	74

6.3.6.3.2	Simple data types	74

6.3.6.4	Data types describing alternative data types or combinations of data types	74

6.3.6.5	Binary data	74

6.3.6.5.1	Binary Data Types	74

6.3.7	Error Handling	74

6.3.7.1	General	74

6.3.7.2	Protocol Errors	74

6.3.7.3	Application Errors	74

6.3.8	Feature negotiation	75

6.3.9	Security	75

6.3.10	HTTP redirection	75

6.4	Naf_Inference Service API	75

6.4.1	Introduction	75

6.4.2	Usage of HTTP	76

6.4.2.1	General	76

6.4.2.2	HTTP standard headers	76

6.4.2.2.1	General	76

6.4.2.2.2	Content type	76

6.4.2.3	HTTP custom headers	76

6.4.3	Resources	76

6.4.3.1	Overview	76

6.4.3.2	Resource: Inference Subscriptions	77

6.4.3.2.1	Description	77

6.4.3.2.2	Resource Definition	77

6.4.3.2.3	Resource Standard Methods	78

6.4.3.2.4	Resource Custom Operations	78

6.4.3.3	Resource: Individual Inference Subscription	78

6.4.3.3.1	Description	78

6.4.3.3.2	Resource Definition	79

6.4.3.3.3	Resource Standard Methods	79

6.4.3.2.4	Resource Custom Operations	83

6.4.4	Custom Operations without associated resources	83

6.4.5	Notifications	83

6.4.5.1	General	83

6.4.5.5	Inference notification	83

6.4.5.5.1	Description	83

6.4.5.5.2	Target URI	83

6.4.5.5.3	Standard Methods	83

6.4.6	Data Model	84

6.4.6.1	General	84

6.4.6.2	Structured data types	86

6.4.6.2.1	Introduction	86

6.4.6.2.2	Type InferEventSubsc	86

6.4.6.2.3	Type InferEventSubscPatch	87

6.4.6.2.4	Type InferAnaSub	88

6.4.6.2.5	Type InferNotif	88

6.4.6.2.6	Type InferResult	89

6.4.6.3	Simple data types and enumerations	89

6.4.6.3.1	Introduction	89

6.4.6.3.2	Simple data types	89

6.4.7	Error Handling	89

6.4.7.1	General	89

6.4.7.2	Protocol Errors	89

6.4.7.3	Application Errors	90

6.4.8	Feature negotiation	90

6.4.9	Security	90

6.4.10	HTTP redirection	90

Annex A (normative): OpenAPI specification	91

A.1	General	91

A.2	Naf_VFLTraining API	91

A.3	Naf_VFLInference API	96

A.4	Naf_Training API	101

A.5	Naf_Inference API	107

Annex B (informative): Withdrawn API versions	113

B.1	General	113

B.2	Naf_VFLTraining API	113

B.3	Naf_VFLInference API	113

B.4	Naf_Training API	113

B.5	Naf_Inference API	114

Annex C (normative): ABNF grammar for 3GPP SBI HTTP custom headers	115

C.1	General	115

Annex D (informative): Change history	115

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document specifies the stage 3 protocol and data model for the Naf Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the AF.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.288 [14], 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 9113: "HTTP/2".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 9457: "Problem Details for HTTP APIs".

[14]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[15]	3GPP TS 29.552: "5G System; Network Data Analytics signalling flows; Stage 3".

[16]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[17]	3GPP TS 29.523: "5G System; Policy Control Event Exposure Service; Stage 3".

[18]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[19]	IETF RFC 9112: "HTTP/1.1".

[20]	IETF RFC 9110: "HTTP Semantics".

[21]	IETF RFC 9111: "HTTP Caching".

[22]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[23]	3GPP TS 29.554: "5G System; Background Data Transfer Policy Control Service; Stage 3".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purpose of the present document, the terms and definitions given in clause 3 of 3GPP TS 23.288 [14] also apply, including the ones referencing other specifications.

3.2	Symbols

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AF	Application Function

AI/ML	Artificial Intelligence/Machine Learning

GPSI	Generic Public Subscription Identifier

NEF	Network Exposure Function

NWDAF	Network Data Analytics Function

REST	Representational State Transfer

VFL	Vertical Federated Learning

4	Overview

The Application Function Artificial Intelligence/Machine Learning (AI/ML) Services, as defined in 3GPP TS 23.288 [14], are provided by the Application Function (AF).

The following AI/ML services are specified for the AF:

Table 4.1-1: AI/ML Services provided by AF

| Service Name | Description | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- | --- |
| Naf_VFLTraining | This service is provided by an AF acting as VFL client and enables an NF service consumer to request the AF to participate in VFL model training as VFL client and train a local model. | Subscribe(NOTE 1) | Subscribe / Notify | NWDAF, NEF |
|  |  | Unsubscribe |  |  |
|  |  | Notify |  |  |
| Naf_VFLInference | This service is provided by AF acting as VFL client and enables an NF service consumer to subscribe/unsubscribe for a VFL inference. | Subscribe(NOTE 2) | Subscribe / Notify | NWDAF, NEF |
|  |  | Unsubscribe |  |  |
|  |  | Notify |  |  |
| Naf_Inference | This service is provided by AF acting as VFL server and enables an NF service consumer to subscribe/unsubscribe for a VFL inference. | Subscribe(NOTE 3) | Subscribe / Notify | NWDAF, NEF |
|  |  | Unsubscribe |  |  |
|  |  | Notify |  |  |
| Naf_Training | This service is provided by AF acting as VFL server and enables an NF service consumer to subscribe/unsubscribe for a VFL training. | Subscribe | Subscribe / Notify | NWDAF, NEF |
|  |  | Unsubscribe |  |  |
|  |  | Notify |  |  |
| NOTE 1: This service implements also the Naf_VFLTraining_Request as specified in 3GPP TS 23.288 [14] by using immediate and one-time reporting requirement. NOTE 2: This service implements also the Naf_VFLInference_Request as specified in 3GPP TS 23.288 [14] by using immediate and one-time reporting requirement. NOTE 3: This service implements also the Naf_Inference_Request as specified in 3GPP TS 23.288 [14] by using immediate and one-time reporting requirement. |  |  |  |  |

5	Services offered by the AF

5.1	Introduction

The AF offers to other NFs the following services:

-	Naf_VFLTraining;

-	Naf_VFLInference;

-	Naf_Training;

-	Naf_Inference.

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Naf_VFLTraining | 5.2 | AF VFL Training service | TS29530_Naf_VFLTraining.yaml | naf-vfl-train | A.2 |
| Naf_VFLInference | 5.3 | AF VFL Inference service | TS29530_Naf_VFLInference.yaml | naf-vflinference | A.3 |
| Naf_Training | 5.4 | AF training service | TS29530_Naf_Training.yaml | naf-train | A.4 |
| Naf_Inference | 5.5 | AF Inference service | TS29530_Naf_Inference.yaml | naf-inference | A.5 |

5.2	Naf_VFLTraining Service

5.2.1	Service Description

5.2.1.1	Overview

The Naf_VFLTraining service exposed by the AF acting as VFL client enables an NF service consumer to:

-	request the creation/update of a VFL Training Subscription; and

-	receive VFL Training related event(s) reporting.

5.2.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Network Data Analytics Exposure architecture is defined in 3GPP TS 23.288 [14]. The VFL signalling flows are defined in 3GPP TS 29.552 [15].

The Naf_VFLTraining service is part of the Naf service-based interface exhibited by the trusted Application Function (AF) or untrusted Application Function (AF).

Known consumers of the Naf_VFLTraining service are:

-	Network Data Analytics Function (NWDAF) when the AF is trusted.

-	Network Exposure Function (NEF) when the AF is untrusted.

Figure 5.2.1.2-1: Reference Architecture for the Naf_VFLTraining service; SBI representation

Figure 5.2.1.2-2: Reference Architecture for the Naf_VFLTraining service: reference point representation

5.2.1.3	Network Functions

5.2.1.3.1	Application Function (AF)

The Application Function (AF) acting as VFL client provides VFL training for different analytics events to NF service consumers.

The Application Function (AF) acting as VFL client allows NF service consumers to subscribe to and unsubscribe from VFL training event notifications.

5.2.1.3.2	NF Service Consumers

The Network Data Analytics Function (NWDAF) and Network Exposure Function (NEF) support (un)subscription to the notification of different VFL training events.

5.2.2	Service Operations

5.2.2.1	Introduction

The service operations defined for the Naf_VFLTraining service are shown in table 5.2.2.1-1.

Table 5.2.2.1-1: Naf_VFLTraining Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Naf_VFLTraining_Subscribe | This service operation enables the NF service consumer to request the creation/update of a VFL Training Subscription. | e.g., NWDAF, NEF |
| Naf_VFLTraining_Unsubscribe | This service operation enables the NF service consumer to request the deletion of a VFL Training Subscription. | e.g., NWDAF, NEF |
| Naf_VFLTraining_Notify | This service operation enables the NF service consumer to receive VFL Training related event(s) reporting. | AF |

5.2.2.2	Naf_VFLTraining_Subscribe

5.2.2.2.1	General

This service operation is used by an NF service consumer to request the creation/update of a VFL Training Subscription at the AF.

The following procedures are supported by the "Naf_VFLTraining_Subscribe" service operation:

-	VFL Training Subscription Creation.

-	VFL Training Subscription Update.

5.2.2.2.2	VFL Training Subscription Creation

Figure 5.2.2.2.2-1 depicts a scenario where an NF service consumer sends a request to the AF to request the creation of a VFL Training Subscription (see also clause 6.2H of 3GPP°TS°23.288°[14]).

Figure 5.2.2.2.2-1: Procedure for VFL Training Subscription Creation

1.	In order to subscribe to VFL Training, the NF service consumer shall send an HTTP POST request to the AF targeting the URI of the "VFL Training Subscriptions" collection resource, with the request body including the VflTrainingSubs data structure.

2a.	Upon success, the AF shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual VFL Training Subscription" resource within the VflTrainingSubs data structure, and an HTTP "Location" header field containing the URI of the created resource.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.7.

5.2.2.2.3	VFL Training Subscription Update

Figure 5.2.2.2.3-1 depicts a scenario where an NF service consumer sends a request to the AF to request the update of an existing VFL Training Subscription (see also clause 6.2H of 3GPP°TS°23.288°[14]).

Figure 5.2.2.2.3-1: Procedure for VFL Training Subscription Update

1.	In order to request the update of an existing VFL Training Subscription, the NF service consumer shall send an HTTP PUT/PATCH request to the AF, targeting the URI of the corresponding "Individual VFL Training Subscription" resource, with the request body including either:

-	the updated representation of the resource within the VflTrainingSubs data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the VflTrainingSubsPatch data structure, in case the HTTP PATCH method is used.

2a.	Upon success, the AF shall update the targeted "Individual VFL Training Subscription" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual VFL Training Subscription" resource within the VflTrainingSubs data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 6.1.7.

5.2.2.3	Naf_VFLTraining_Unsubscribe

5.2.2.3.1	General

This service operation is used by an NF service consumer to request the deletion of a VFL Training Subscription at the AF.

The following procedures are supported by the "Naf_VFLTraining_Unsubscribe" service operation:

-	VFL Training Subscription Deletion.

5.2.2.3.2	VFL Training Subscription Deletion

Figure 5.2.2.3.2-1 depicts a scenario where an NF service consumer sends a request to the AF to delete an existing VFL Training Subscription (see also clause 6.2H of 3GPP°TS°23.288°[14]).

Figure 5.2.2.3.2-1: Procedure for VFL Training Subscription Deletion

1.	In order to request the deletion of an existing VFL Training Subscription, the NF service consumer shall send an HTTP DELETE request to the AF targeting the URI of the corresponding "Individual VFL Training Subscription" resource.

2a.	Upon success, the AF shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.1.7.

5.2.2.4	Naf_VFLTraining_Notify

5.2.2.4.1	General

This service operation is used by the AF to notify a previously subscribed service consumer on:

-	VFL Training report(s).

The following procedures are supported by the "Naf_VFLTraining_Notify" service operation:

-	VFL Training Notification.

5.2.2.4.2	VFL Training Notification

Figure 5.2.2.4.2-1 depicts a scenario where the AF sends a request to notify a previously subscribed service consumer on VFL Training report(s) (see also clause 6.2H of 3GPP°TS°23.288°[14]).

Figure 5.2.2.4.2-1: Procedure for VFL Training Notification

1.	In order to notify a previously subscribed service consumer on VFL Training report(s), the AF shall send an HTTP POST request to the NF service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the NF service consumer during the creation/update of the corresponding VFL Training Subscription using the procedures defined in clauses 5.2.2.2, and the request body including the VflTrainingNotify data structure.

2a.	Upon success, the NF service consumer shall respond to the AF with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.7.

5.3	Naf_VFLInference Service

5.3.1	Service Description

5.3.1.1	Overview

The Naf_VFLInference service as defined in 3GPP TS 23.288 [14], is provided by the trusted Application Function (AF) or untrusted Application Function (AF) acting as VFL client.

This service allows the NF service consumers acting as VFL servers to:

-	subscribe to and unsubscribe from different VFL inference events;

-	modify VFL inference subscriptions; and

-	be notified about events for corresponding VFL inference subscriptions.

5.3.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Network Data Analytics Exposure architecture is defined in 3GPP TS 23.288 [14]. The VFL signalling flows are defined in 3GPP TS 29.552 [15].

The Naf_VFLInference service is part of the Naf service-based interface exhibited by the trusted Application Function (AF) or untrusted Application Function (AF).

Known consumers of the Naf_VFLInference service are:

-	Network Data Analytics Function (NWDAF) when the AF is trusted.

-	Network Exposure Function (NEF) when the AF is untrusted.

Figure 5.3.1.2-1: Reference Architecture for the Naf_VFLInference service; SBI representation

Figure 5.3.1.2-2: Reference Architecture for the Naf_VFLInference service: reference point representation

5.3.1.3	Network Functions

5.3.1.3.1	Application Function (AF)

The Application Function (AF), provides VFL inference for different analytics events to NF service consumers.

The Application Function (AF) allows NF service consumers to subscribe to and unsubscribe from one-time, periodic notification when a VFL inference event is detected.

5.3.1.3.2	NF Service Consumers

The Network Data Analytics Function (NWDAF) and Network Exposure Function (NEF) support (un)subscription to the notification of different VFL inference events.

5.3.2	Service Operations

5.3.2.1	Introduction

Table 5.3.2.1-1: Operations of the Naf_VFLInference service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Naf_VFLInference_Subscribe | This service operation is used by an NF service consumer to subscribe to VFL inference events. | NF service consumer (NWDAF,NEF) |
| Naf_VFLInference_Unsubscribe | This service operation is used by an NF service consumer to unsubscribe from VFL inference event notifications. | NF service consumer (NWDAF,NEF) |
| Naf_VFLInference_Notify | This service operation is used by the AF to notify the VFL inference results to the NF service consumer which has subscribed to the event notifications. | AF |

5.3.2.2	Naf_VFLInference_Subscribe service operation

5.3.2.2.1	General

The Naf_VFLInference_Subscribe service operation is used by an NF service consumer to request AF VFL client(s) to subscribe or update subscription for VFL inference event notifications from the AF acting as VFL client.

5.3.2.2.2	Subscription for VFL inference event notifications

Figure 5.3.2.2.2-1 shows a scenario where the NF service consumer sends a request to the AF to subscribe for VFL inference event notification(s) (as shown in 3GPP TS 23.288 [14]).

Figure 5.3.2.2.2-1: NF service consumer subscribes to VFL inference notifications

The NF service consumer shall invoke the Naf_VFLInference_Subscribe service operation to subscribe to VFL inference event notification(s). The NF service consumer shall send an HTTP POST request with "{apiRoot}/naf-vflinference/<apiVersion>/subscriptions" as Resource URI representing the "VFL Inference Subscriptions", as shown in figure 5.3.2.2.2-1, step 1, to create a subscription for an "Individual VFL Inference Subscription" according to the information in message body.

The VflInferSub data structure provided in the request body shall include:

-	an URI where to receive the requested notifications as the "notifUri" attribute;

-	a notification correlation identifier assigned by the NF service consumer for the requested notifications as "notifCorrId" attribute; and

-	a description of the subscribed analytics event(s) as the "vflInferAnaSubs" attribute.

and may include:

-	the VFL reporting information as the "reportingReqs" attribute; and

Upon the reception of an HTTP POST request with: "{apiRoot}/naf-vflinference/<apiVersion>/subscriptions" as Resource URI and VflInferSub data structure as request body, the AF shall create a new subscription and store the subscription.

If the AF created an "Individual VFL Inference Subscription" resource, the AF shall respond with "201 Created" with the message body containing a representation of the created subscription, as shown in figure 5.3.2.2.2-1, step 2. The AF shall include a Location HTTP header field. The Location header field shall contain the URI of the created subscription i.e. "{apiRoot}/naf-vflinference/<apiVersion>/subscriptions/{subscriptionId}".

If the immediate reporting indication is applicable during the event subscription considering the "reportingReqs" attribute in the VflInferSub and VflInferAnaSub data structures, the AF shall include the intermediate VFL inference results of the subscribed events, if available, as the "vflInferResults" attribute in the HTTP POST response.

If any error occurs when processing the HTTP POST request, the AF shall send an HTTP error response as specified in clause 6.2.7.

5.3.2.2.3	Update subscription for event notifications

Figure 5.3.2.2.3-1 shows a scenario that the NF service consumer sends an HTTP PUT request to the AF to modify an existing VFL inference subscription (as shown in 3GPP TS 23.288 [14]).

Figure 5.3.2.2.3-1: Modification of VFL inference events subscription information using HTTP PUT

The NF service consumer shall invoke the Naf_VFLInference_Subscribe service operation to modify an existing VFL inference subscription. The NF service consumer shall send an HTTP PUT request with: "{apiRoot}/naf-vflinference/<apiVersion>/subscriptions/{subscriptionId}" as Resource URI, where "{subscriptionId}" is the subscriptionId of the existing VFL inference subscription to be modified, to update an "Individual VFL Inference Subscription" according to the information in the message body. The VflInferSub data structure provided in the request body shall include the same contents as described in clause 5.3.2.2.2.

Upon receipt of an HTTP PUT request with: "{apiRoot}/naf-vflinference/<apiVersion>/subscriptions/{subscriptionId}" as Resource URI and VflInferSub data type as request body, if the request is successfully processed and accepted, the AF shall:

-	modify the concerned subscription; and

-	store the subscription.

If the AF successfully processed and accepted the received HTTP PUT request, the AF shall update an "Individual VFL Inference Subscription" resource, and shall respond with:

-	HTTP "204 No Content" response (as shown in figure 5.3.2.2.3-1, step 2a); or

-	HTTP "200 OK" response (as shown in figure 5.3.2.2.3-1, step 2b) with a response body containing a representation of the updated subscription in the VflInferSub data type.

If the immediate reporting indication is applicable during the event subscription update considering the "reportingReqs" attribute in the VflInferSub and VflInferAnaSub data structures, the AF shall include the reports of the subscribed events, if available, as the "vflInferResults" attribute in the HTTP PUT response.

If any error occurs when processing the HTTP PUT request, the AF shall send an HTTP error response as specified in clause 6.2.7.

If the AF determines that the received HTTP PUT request needs to be redirected, the AF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

5.3.2.2.4	Partial update subscription for event notifications

Figure 5.3.2.2.4-1 shows a scenario that the NF service consumer sends an HTTP PATCH request to the AF to partial modify an existing VFL inference subscription (as shown in 3GPP TS 23.288 [14]).

Figure 5.3.2.2.4-1: Partial modification of VFL inference subscription information using HTTP PATCH

The NF service consumer shall invoke the Naf_VFLInference_Subscribe service operation to partial modify an existing VFL inference subscription. The NF service consumer shall send an HTTP PATCH request with: "{apiRoot}/naf-vflinference/<apiVersion>/subscriptions/{subscriptionId}" as Resource URI, where "{subscriptionId}" is the subscriptionId of the existing VFL inference subscription to be modified, to update an "Individual VFL Inference Subscription" according to the information in the message body.

Upon receipt of an HTTP PATCH request with: "{apiRoot}/naf-vflinference/<apiVersion>/subscriptions/{subscriptionId}" as Resource URI and VflInferSubPatch data type as request body, if the request is successfully processed and accepted, the AF shall:

-	partial modify the concerned subscription; and

-	store the subscription.

If the AF successfully processed and accepted the received HTTP PATCH request, the AF shall partially update an "Individual VFL Inference Subscription" resource, and shall respond with:

-	HTTP "204 No Content" response (as shown in figure 5.3.2.2.4-1, step 2a); or

-	HTTP "200 OK" response (as shown in figure 5.3.2.2.4-1, step 2b) with a response body containing a representation of the updated subscription in the VflInferSub data type.

If the immediate reporting indication is applicable during the event subscription update considering the "reportingReqs" attribute within the in the VflInferSub and VflInferAnaSub data structures on the event subscription or the event subscription update, the AF shall include the reports of the subscribed events, if available, as the "vflInferResults" attribute in the HTTP PATCH response.

If any error occurs when processing the HTTP PATCH request, the AF shall send an HTTP error response as specified in clause 6.2.7.

If the AF determines that the received HTTP PATCH request needs to be redirected, the AF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

5.3.2.3	Naf_VFLInference_Unsubscribe service operation

5.3.2.3.1	General

The Naf_VFLInference_Unsubscribe service operation is used by an NF service consumer to unsubscribe from VFL inference notifications.

5.3.2.3.2	Unsubscribe from VFL inference notifications

Figure 5.3.2.3.2-1 shows a scenario where the NF service consumer sends a request to the AF to unsubscribe from a VFL inference notification (see also 3GPP TS 23.288 [14]).

Figure 5.3.2.3.2-1: NF service consumer unsubscribes from VFL inference notifications

The NF service consumer shall invoke the Naf_VFLInference_Unsubscribe service operation to unsubscribe from VFL inference event notifications. The NF service consumer shall send an HTTP DELETE request with: "{apiRoot}/naf-vflinference/<apiVersion>/subscriptions/{subscriptionId}" as Resource URI, where "{subscriptionId}" is the subscriptionId of the existing VFL inference subscription that is to be deleted.

Upon the reception of an HTTP DELETE request, if the AF successfully processed and accepted the received HTTP DELETE request, the AF shall:

-	remove the corresponding subscription; and

-	respond with HTTP "204 No Content" status code.

If the AF determines the received HTTP DELETE request needs to be redirected, the AF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

If errors occur when processing the HTTP DELETE request, the AF shall send an HTTP error response as specified in clause 6.2.7.

5.3.2.4	Naf_VFLInference_Notify service operation

5.3.2.4.1	General

The Naf_VFLInference_Notify service operation is used by an AF to notify NF consumers about subscribed VFL inference events.

5.3.2.4.2	Notification about subscribed event

Figure 5.3.2.4.2-1 shows a scenario where the AF sends a request to the NF Service Consumer to notify for VFL inference event notifications (see also 3GPP TS 23.288 [14]).

Figure 5.3.2.4.2-1: AF notifies the subscribed VFL inference event

The AF shall invoke the Naf_VFLInference_Notify service operation to notify about a subscribed VFL inference event. The AF shall send an HTTP POST request with "{notifUri}" received in the Naf_VFLInference_Subscribe service operation as Resource URI, as shown in figure 5.3.2.4.2-1, step 1. The VflInferNotif data structure provided in the request body that shall include:

-	a notification correlation identifier as "notifCorrId" attribute;

-	description of the notified event as "vflInferResults" attribute.

Upon the reception of an HTTP POST request, if the NF service consumer successfully processed and accepted the received HTTP POST request, the NF Service Consumer shall store the notification and respond with HTTP "204 No Content" status code.

If the NF service consumer determines the received HTTP POST request needs to be redirected, the NF service consumer shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

If errors occur when processing the HTTP POST request, the NF service consumer shall send an HTTP error response as specified in clause 6.2.7.

5.4	Naf_Training Service

5.4.1	Service Description

5.4.1.1	Overview

The Naf_Training service exposed by the AF acting as VFL server and enables an NF service consumer to:

-	request the creation/update of a Training subscription; and

-	receive Training related event(s) reporting.

5.4.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Network Data Analytics Exposure architecture is defined in 3GPP TS 23.288 [14]. The VFL signalling flows are defined in 3GPP TS 29.552 [15].

The Naf_Training service is part of the Naf service-based interface exhibited by the trusted Application Function (AF) or untrusted Application Function (AF).

Known consumers of the Naf_Training service are:

-	Network Data Analytics Function (NWDAF) when the AF is trusted.

-	Network Exposure Function (NEF) when the AF is untrusted.

Figure 5.4.1.2-1: Reference Architecture for the Naf_Training service; SBI representation

Figure 5.4.1.2-2: Reference Architecture for the Naf_Training service: reference point representation

5.4.1.3	Network Functions

5.4.1.3.1	Application Function (AF)

The Application Function (AF) acting as VFL server provides training for different analytics events to NF service consumers.

The Application Function (AF) acting as VFL server allows NF service consumers to subscribe to and unsubscribe from training event notifications.

5.4.1.3.2	NF Service Consumers

The Network Data Analytics Function (NWDAF) and Network Exposure Function (NEF) support (un)subscription to the notification of different training events.

5.4.2	Service Operations

5.4.2.1	Introduction

The service operations defined for the Naf_Training service are shown in table 5.4.2.1-1.

Table 5.4.2.1-1: Naf_Training Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Naf_Training_Subscribe | This service operation enables the NF service consumer to request the creation/update of a Training Subscription. | e.g., NWDAF, NEF |
| Naf_Training_Unsubscribe | This service operation enables the NF service consumer to request the deletion of a Training Subscription. | e.g., NWDAF, NEF |
| Naf_Training_Notify | This service operation enables the NF service consumer to receive Training related event(s) reporting. | AF |

5.4.2.2	Naf_Training_Subscribe

5.4.2.2.1	General

This service operation is used by an NF service consumer to request the creation/update of a Training Subscription at the AF.

The following procedures are supported by the "Naf_Training_Subscribe" service operation:

-	Training Subscription Creation.

-	Training Subscription Update.

5.4.2.2.2	Training Subscription Creation

Figure 5.4.2.2.2-1 depicts a scenario where an NF service consumer sends a request to the AF to request the creation of a Training Subscription (see also clause 6.2H of 3GPP°TS°23.288°[14]).

Figure 5.4.2.2.2-1: Procedure for Training Subscription Creation

1.	In order to subscribe to Training, the NF service consumer shall send an HTTP POST request to the AF targeting the URI of the "Training Subscriptions" collection resource, with the request body including the TrainEventsSubsc data structure.

2a.	Upon success, the AF shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual Training Subscription" resource within the TrainEventsSubsc data structure, and an HTTP "Location" header field containing the URI of the created resource.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.3.7.

5.4.2.2.3	Training Subscription Update

Figure 5.4.2.2.3-1 depicts a scenario where an NF service consumer sends a request to the AF to request the update of an existing Training Subscription (see also clause 6.2H of 3GPP°TS°23.288°[14]).

Figure 5.4.2.2.3-1: Procedure for Training Subscription Update

1.	In order to request the update of an existing Training Subscription, the NF service consumer shall send an HTTP PUT/PATCH request to the AF, targeting the URI of the corresponding "Individual Training Subscription" resource, with the request body including either:

-	the updated representation of the resource within the TrainEventsSubsc data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the TrainEventsSubscPatch data structure, in case the HTTP PATCH method is used.

2a.	Upon success, the AF shall update the targeted "Individual Training Subscription" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Training Subscription" resource within the TrainEventsSubsc data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 6.3.7.

5.4.2.3	Naf_Training_Unsubscribe

5.4.2.3.1	General

This service operation is used by an NF service consumer to request the deletion of a Training Subscription at the AF.

The following procedures are supported by the "Naf_Training_Unsubscribe" service operation:

-	Training Subscription Deletion.

5.4.2.3.2	Training Subscription Deletion

Figure 5.4.2.3.2-1 depicts a scenario where an NF service consumer sends a request to the AF to delete an existing Training Subscription (see also clause 6.2H of 3GPP°TS°23.288°[14]).

Figure 5.4.2.3.2-1: Procedure for Training Subscription Deletion

1.	In order to request the deletion of an existing Training Subscription, the NF service consumer shall send an HTTP DELETE request to the AF targeting the URI of the corresponding "Individual Training Subscription" resource.

2a.	Upon success, the AF shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.3.7.

5.4.2.4	Naf_Training_Notify

5.4.2.4.1	General

This service operation is used by the AF to notify a previously subscribed service consumer on:

-	Training report(s).

The following procedures are supported by the "Naf_Training_Notify" service operation:

-	Training Notification.

5.4.2.4.2	Training Notification

Figure 5.4.2.4.2-1 depicts a scenario where the AF sends a request to notify a previously subscribed service consumer on Training report(s) (see also clause 6.2H of 3GPP°TS°23.288°[14]).

Figure 5.4.2.4.2-1: Procedure for Training Notification

1.	In order to notify a previously subscribed service consumer on Training report(s), the AF shall send an HTTP POST request to the NF service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the NF service consumer during the creation/update of the corresponding Training Subscription using the procedures defined in clauses 5.4.2.2, and the request body including the TrainEventsNotif data structure.

2a.	Upon success, the NF service consumer shall respond to the AF with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.3.7.

5.5	Naf_Inference Service

5.5.1	Service Description

5.5.1.1	Overview

The Naf_Inference service as defined in 3GPP TS 23.288 [14], is provided by the Application Function (AF) acting as VFL server.

This service allows the NF service consumers to:

-	subscribe to and unsubscribe from different inference events;

-	modify inference subscriptions; and

-	be notified about events for corresponding inference subscriptions.

5.5.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Network Data Analytics Exposure architecture is defined in 3GPP TS 23.288 [14]. The VFL signalling flows are defined in 3GPP TS 29.552 [15].

The Naf_Inference service is part of the Naf service-based interface exhibited by the trusted Application Function (AF) or untrusted Application Function (AF).

Known consumers of the Naf_Inference service are:

-	Network Data Analytics Function (NWDAF) when the AF is trusted.

-	Network Exposure Function (NEF) when the AF is untrusted.

Figure 5.5.1.2-1: Reference Architecture for the Naf_Inference service; SBI representation

Figure 5.5.1.2-2: Reference Architecture for the Naf_Inference service: reference point representation

5.5.1.3	Network Functions

5.5.1.3.1	Application Function (AF)

The Application Function (AF) acting as VFL server provides inference for different analytics events to NF service consumers.

The Application Function (AF) acting as VFL server allows NF service consumers to subscribe to and unsubscribe from inference events.

5.5.1.3.2	NF Service Consumers

The Network Data Analytics Function (NWDAF) and Network Exposure Function (NEF) support (un)subscription to the notification of different inference events.

5.5.2	Service Operations

5.5.2.1	Introduction

Service operations defined for the Naf_Inference Service are shown in table 5.5.2-1.

Table 5.5.2-1: Naf_Inference Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Naf_Inference_Subscribe | This service operation is used by an NF service consumer to subscribe to, or modify a subscription in the AF for Inference event notifications. | NF Consumer (i.e., NWDAF, NEF) |
| Naf_Inference_Unsubscribe | This service operation is used by an NF service consumer to unsubscribe from Inference event notifications. | NF Consumer (i.e., NWDAF, NEF) |
| Naf_Inference_Notify | This service operation is used by the AF to report Inference related event(s) to the NF service consumer which has subscribed to the event report service. | AF |

5.5.2.2	Naf_Inference_Subscribe

5.5.2.2.1	General

The Naf_Inference_Subscribe service operation is used by an NF service consumer to request AF VFL servers(s) to subscribe or update subscription for inference event notifications from the AF acting as VFL server.

The following procedures are supported by the "Naf_Inference_Subscribe" service operation:

-	Inference Subscription Creation.

-	Inference Subscription Update.

5.5.2.2.2	Inference Subscription Creation

Figure 5.5.2.2.2-1 a scenario where an NF service consumer sends a request to the AF to request the creation of an Inference Subscription.

Figure 5.5.2.2.2-1: Procedure for Inference subscription

In order to subscribe to Inference event notifications, the NF service consumer shall send an HTTP POST request to the AF targeting the URI of the "Inference Subscriptions" collection resource, with the request body including the InferEventSubsc data structure as defined in clause 6.4.6.2.2.

Upon the reception of an HTTP POST request with: "{apiRoot}/naf-inference/<apiVersion>/subscriptions" as Resource URI and InferEventSubsc data structure as request body:

-	If the VFL server is a trusted AF:

-	If no VFL model is already trained, the VFL server initiates VFL Training by sending Nnwdaf_VFLTraining_Subscribe towards NWDAF(s) acting as VFL Client(s).

-	If VFL model is already trained, the VFL server may decide to initiate the VFL inference procedure towards the selected NWDAF VFL client(s) by sending Nnwdaf_VFLInference_Subscribe.

-	If the VFL server is an untrusted AF:

-	If no VFL model is already trained, the VFL server initiates VFL Training by sending for each NWDAF acting as VFL client an Nnef_VFLTraining_Subscribe request towards NEF indicating the identity of the NWDAF acting as VFL Client.

-	If VFL model is already trained, the VFL server may decide to initiate the VFL inference procedure. For each selected NWDAF VFL client the VFL server sends an Nnef_VFLInference_Subscribe request to the NEF indicating the identity of the NWDAF acting as VFL Client.

If no NWDAF VFL Client(s) are selected, the VFL server may generate the VFL inference results based only on its local trained ML model associated with the determined VFL correlation ID.

Upon success, the AF shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual Inference Subscription" resource within the InferEventsSubsc data structure, and an HTTP "Location" header field containing the URI of the created resource.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.4.7.

5.5.2.2.3	Inference Subscription Update

Figure 5.5.2.2.3-1 illustrates the modification of an existing subscription.

Figure 5.5.2.2.3-1: Modification of an existing subscription

In order to request the update of an existing inference subscription, the NF service consumer shall send an HTTP PUT/PATCH request to the AF, targeting the URI of the corresponding "Individual Inference Subscription" resource, with the request body including either:

-	the updated representation of the resource within the InferEventSubsc data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the InferEventSubscPatch data structure, in case the HTTP PATCH method is used.

Upon receipt of the HTTP PUT/PATCH request from the NF service consumer the AF shall:-

-	if the VFL server is a trusted AF and there are NWDAF VFL subscriptions associated to the AF inference subscription, the VFL updates those NWDAF VFL inference subscriptions by sending an Nnwdaf_VFLInference_Subscribe request towards each applicable NWDAF VFL client(s);

-	if the VFL server is an untrusted AF and there are NWDAF VFL subscriptions associated to the AF inference subscription, the VFL updates those NWDAF VFL inference subscriptions by sending for each applicable NWDAF client an Nnef_VFLInference_Subscribe request to the NEF indicating the identity of the NWDAF VFL Client;

Upon success, the AF shall update the targeted "Individual Inference Subscription" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Inference Subscription" resource within the InferEventsSubsc data structure; or

-	an HTTP "204 No Content" status code.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 6.4.7.

5.5.2.3	Naf_Inference_Unsubscribe

5.5.2.3.1	General

The Naf_Inference_Unsubscribe service operation is used by an NF service consumer to subscribe from inference notifications.

The following procedures are supported by the "Naf_Inference_Unsubscribe" service operation:

-	Inference Subscription Deletion.

5.5.2.3.2	Inference Subscription Deletion

Figure 5.5.2.3.2-1 shows a scenario where the NF service consumer sends a request to the AF to unsubscribe from an inference notification (see also 3GPP TS 23.288 [14]).

Figure 5.5.2.3.2-1: NF service consumer unsubscribes from inference notifications

In order to request the deletion of an existing Inference Subscription, the NF service consumer shall send an HTTP DELETE request to the AF targeting the URI of the corresponding "Individual Inference Subscription" resource.

Upon success, the AF shall respond with an HTTP "204 No Content" status code.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.4.7.

5.5.2.4	Naf_Inference_Notify

5.5.2.4.1	General

The Naf_Inference_Notify service operation is used by an AF to notify NF consumers about subscribed inference events.

The following procedures are supported by the "Naf_Inference_Notify" service operation:

-	Inference Notification.

5.5.2.4.2	Inference Notification

Figure 5.5.2.4.2-1 shows a scenario where the AF sends a request to the NF Service Consumer to notify for inference event notifications (see also 3GPP TS 23.288 [14]).

Figure 5.5.2.4.2-1: AF notifies the subscribed inference event

In order to notify a previously subscribed service consumer on Inference  event(s), the AF shall send an HTTP POST request to the NF service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the NF service consumer during the creation/update of the corresponding Individual Inference Subscription resource using the procedures defined in clauses 5.5.2.2.2 and 5.5.2.2.3, and the request body including the InferNotif data structure.

Upon success, the NF service consumer shall respond to the AF with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.4.7.

6	API Definitions

6.1	Naf_VFLTraining Service API

6.1.1	Introduction

The Naf_VFLTraining shall use the Naf_VFLTraining API.

The API URI of the Naf_VFLTraining API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "naf-vfl-train".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clauses 6.1.3 and 6.1.4.

6.1.2	Usage of HTTP

6.1.2.1	General

HTTP/2, IETF RFC 9113 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Naf_VFLTraining API is contained in Annex A.

6.1.2.2	HTTP standard headers

6.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

6.1.2.2.2	Content type

If the AF is untrusted, support of HTTP/1.1 (IETF RFC 9112 [19], IETF RFC 9110 [20] and IETF RFC 9111[21] over TLS is mandatory and support of HTTP/2 (IETF RFC 9113 [11]) over TLS is recommended. TLS shall be used as specified in clause 12.3 and clause 13.1 of 3GPP TS 33.501 [8].

If the AF is trusted, HTTP/2, IETF RFC 9113 [11], shall be used as specified in clause 5.2 of 3GPP TS 29.500 [4].

HTTP/2, IETF RFC 9113 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Naf_VFLTraining API is contained in Annex A.

6.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

6.1.3	Resources

6.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.3.1-1 depicts the resource URIs structure for the Naf_VFLTraining API.

Figure 6.1.3.1-1: Resource URI structure of the Naf_VFLTraining API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| VFL Training Subscriptions | /subscriptions | POST | Create a new VFL Training Subscription. |
| Individual VFL Training Subscription | /subscriptions/{subscriptionId} | GET | Retrieve an existing "Individual VFL Training Subscription" resource. |
|  |  | PUT | Request the update of an existing "Individual VFL Training Subscription" resource. |
|  |  | PATCH | Request the modification of an existing "Individual VFL Training Subscription" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual VFL Training Subscription" resource. |

6.1.3.2	Resource: VFL Training Subscriptions

6.1.3.2.1	Description

This resource represents the collection of VFL Training Subscription(s) managed by the AF.

6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/naf-vfl-train/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |

6.1.3.2.3	Resource Standard Methods

6.1.3.2.3.1	POST

The HTTP POST method allows a service consumer to request the creation of a VFL Training Subscription at the AF.

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VflTrainingSubs | M | 1 | Represents the parameters to request the creation of a VFL Training Subscription. |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VflTrainingSubs | M | 1 | 201 Created | Successful case. The VFL Training Subscription is successfully created and a representation of the created "Individual VFL Training Subscription" resource shall be returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: Failure cases are described in clause 6.1.7. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/naf-vfl-train/<apiVersion>/subscriptions/{subscriptionId} |

6.1.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

6.1.3.3	Resource: Individual VFL Training Subscription

6.1.3.3.1	Description

This resource represents a VFL Training Subscription managed by the AF.

6.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/naf-vfl-train/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |
| subscriptionId | string | Represents the unique identifier of the "Individual VFL Training Subscription" resource. |

6.1.3.3.3	Resource Standard Methods

6.1.3.3.3.1	GET

The GET method allows an NF service consumer to retrieve an existing "Individual VFL Training Subscription" resource managed by the AF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VflTrainingSubs | M | 1 | 200 OK | Successful case. The requested "Individual VFL Training Subscription" resource is returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

Table 6.1.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

6.1.3.3.3.2	PUT

The PUT method allows an NF service consumer to request the update of an existing "Individual VFL Training Subscription" resource managed by the AF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.2-1.

Table 6.1.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.3.3.3.2-3.

Table 6.1.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VflTrainingSubs | M | 1 | Contains the updated representation of the "Individual VFL Training Subscription" resource. |

Table 6.1.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VflTrainingSubs | M | 1 | 200 OK | Successful case. The "Individual VFL Training Subscription" resource is successfully updated and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual VFL Training Subscription" resource is successfully updated and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 3) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). NOTE 3: Failure cases are described in clause 6.1.7. |  |  |  |  |

Table 6.1.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

Table 6.1.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

6.1.3.3.3.3	PATCH

The PATCH method allows an NF service consumer to request the modification of an existing "Individual VFL Training Subscription" resource managed by the AF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.3-1.

Table 6.1.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.3-2 and the response data structures and response codes specified in table 6.1.3.3.3.3-3.

Table 6.1.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VflTrainingSubsPatch | M | 1 | Contains the parameters to request the modification of the "Individual VFL Training Subscription" resource. |

Table 6.1.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VflTrainingSubs | M | 1 | 200 OK | Successful case. The "Individual VFL Training Subscription" resource is successfully modified and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual VFL Training Subscription" resource is successfully modified and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 3) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). NOTE 3: Failure cases are described in clause 6.1.7. |  |  |  |  |

Table 6.1.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

Table 6.1.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

6.1.3.3.3.4	DELETE

The DELETE method allows an NF service consumer to request the deletion of an existing "Individual VFL Training Subscription" resource managed by the AF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.4-1.

Table 6.1.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.4-2 and the response data structures and response codes specified in table 6.1.3.3.3.4-3.

Table 6.1.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual VFL Training Subscription" resource is successfully deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

Table 6.1.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

6.1.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

6.1.5	Notifications

6.1.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

Table 6.1.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| VFL Training Notification | {notifUri} | POST | Enables the AF to notify a previously subscribed NF service consumer on VFL Training report(s). |

6.1.5.2	VFL Training Notification

6.1.5.2.1	Description

The VFL Training Notification is used by the AF to notify a previously subscribed NF service consumer on VFL Training report(s).

6.1.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.1.5.2.2-1.

Table 6.1.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Represents the callback URI encoded as a string formatted as a URI. |

6.1.5.2.3	Standard Methods

6.1.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.5.2.3.1-1 and the response data structures and response codes specified in table 6.1.5.2.3.1-2.

Table 6.1.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VflTrainingNotify | M | 1 | Represents the VFL Training Notification. |

Table 6.1.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The VFL Training Notification is successfully received. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF service consumer (service) instance towards which the notification request is redirected. |

Table 6.1.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF service consumer (service) instance towards which the notification request is redirected. |

6.1.6	Data Model

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Naf_VFLTraining service-based interface protocol.

Table 6.1.6.1-1: Naf_VFLTraining API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| VflTrainingSubs | 6.1.6.2.2 | Represents a VFL Training Subscription. |  |
| VflTrainingSubsPatch | 6.1.6.2.3 | Represents the requested modifications to a VFL Training Subscription. |  |

Table 6.1.6.1-2 specifies data types re-used by the Naf_VFLTraining service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Naf_VFLTraining service-based interface.

Table 6.1.6.1-2: Naf_VFLTraining API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ReportingInformation | 3GPP TS 29.523 [17] | Represents the event reporting requirements. |  |
| SupportedFeatures | 3GPP TS 29.571 [16] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| Uri | 3GPP TS 29.571 [16] | Represents a URI. |  |
| VflTrainingNotify | 3GPP TS 29.520 [18] | Represents a VFL Training Notification. |  |
| VflTrainingSub | 3GPP TS 29.520 [18] | Represents a VFL Training set. |  |

6.1.6.2	Structured data types

6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.1.6.2.2	Type: VflTrainingSubs

Table 6.1.6.2.2-1: Definition of type VflTrainingSubs

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| vflTrainSubs | array(VflTrainingSub) | M | 1..N | Contains the subscribed VFL training set(s). |  |
| notifUri | Uri | C | 0..1 | Represents the URI via which VFL Training related notifications shall be delivered. It shall be present if the "vflPreFlag" attribute is not included or set to "false". |  |
| notifCorrId | string | C | 0..1 | Represents the notification Correlation Identifier. It shall be present if the "vflPreFlag" attribute is not included or set to "false". |  |
| vflPreFlag | boolean | C | 0..1 | Indicates the subscription is for preparation of VFL, when it is included and set to "true". The default value is "false" if omitted. It shall be present when the service is for preparation of VFL. (NOTE) |  |
| reportingReqs | ReportingInformation | O | 0..1 | Contains the reporting requirements applicable for VFL Training related reporting. |  |
| trainReports | array(VflTrainingNotify) | O | 1..N | Contains the VFL Training related event(s) report(s). This attribute may be present only if immediate reporting was requested via the "reportingReqs" attribute. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.1.8. This attribute shall be present only when feature negotiation is required. |  |
| NOTE: When the service is for VFL preparation (the "vflPreFlag" attribute sets to "true"), the NF service producer shall send no notification until it is updated with the value of the "vflPreFlag" attribute as "false", and the "trainReports" attribute shall be absent in the HTTP POST response. |  |  |  |  |  |

6.1.6.2.3	Type: VflTrainingSubsPatch

Table 6.1.6.2.3-1: Definition of type VflTrainingSubsPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| vflTrainSubs | array(VflTrainingSub) | O | 1..N | Contains the updated subscribed VFL training set(s). |  |
| notifUri | Uri | O | 0..1 | Contains the updated URI via which VFL Training related notifications shall be delivered. |  |
| notifCorrId | string | O | 0..1 | Notification Correlation Identifier. |  |
| reportingReqs | ReportingInformation | O | 0..1 | Contains the reporting requirements applicable for VFL Training related reporting. |  |

6.1.6.3	Simple data types and enumerations

6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

6.1.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

6.1.6.5	Binary data

6.1.6.5.1	Binary Data Types

Table 6.1.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

6.1.7	Error Handling

6.1.7.1	General

For the Naf_VFLTraining API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Naf_VFLTraining API.

6.1.7.2	Protocol Errors

No specific procedures for the Naf_VFLTraining service are specified.

6.1.7.3	Application Errors

The application errors defined for the Naf_VFLTraining service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| OVERLOAD | 403 Forbidden | Indicates the AF is overloaded. |  |
| NOT_AVAILABLE_FOR_VFL_PROCESS | 403 Forbidden | Indicates the AF is not available for the VFL process anymore. |  |
| VFL_REQS_NOT_MET | 403 Forbidden | Indicates the VFL training requirements are not met. |  |
| NOTE: Including a "ProblemDetails" data structure with the "cause" attribute in the HTTP response is optional unless explicitly mandated in the service operation clauses. |  |  |  |

6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the Naf_VFLTraining API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

6.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Naf_VFLTraining API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Naf_VFLTraining API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in clause 5.4.2.2 of 3GPP TS 29.510 [10].

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Naf_VFLTraining service.

The Naf_VFLTraining API defines a single scope "naf-vfl-train" for the entire service, and it does not define any additional scopes at resource or operation level.

6.1.10	HTTP redirection

An HTTP request may be redirected to a different AF service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different AF producer instance will return the NF Instance ID of the new AF producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an AF redirects a service request to a different AF using an HTTP "307 Temporary Redirect" or "308 Permanent Redirect" status code, the identity of the new AF towards which the service request is redirected shall be indicated in the "3gpp-Sbi-Target-Nf-Id" header of the HTTP "307 Temporary Redirect" or "308 Permanent Redirect" response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

6.2	Naf_VFLInference Service API

6.2.1	Introduction

The Naf_VFLInference service shall use the Naf_VFLInference API.

The API URI of the Naf_VFLInference API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in each HTTP requests from the NF service consumer towards the AF shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The<apiName> shall be "naf-vflinference".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 5.5.3.

6.2.2	Usage of HTTP

6.2.2.1	General

If the AF is untrusted, support of HTTP/1.1 (IETF RFC 9112 [19], IETF RFC 9110 [20] and IETF RFC 9111[21] over TLS is mandatory and support of HTTP/2 (IETF RFC 9113 [11]) over TLS is recommended. TLS shall be used as specified in clause 12.3 and clause 13.1 of 3GPP TS 33.501 [8].

If the AF is trusted, HTTP/2, IETF RFC 9113 [11], shall be used as specified in clause 5.2 of 3GPP TS 29.500 [4].

HTTP/2, IETF RFC 9113 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Naf_VFLInference is contained in Annex A.

6.2.2.2	HTTP standard headers

6.2.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

6.2.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 9457 [13].

6.2.2.3	HTTP custom headers

The Naf_VFLInference service API shall support mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] and may support HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4].

In this release of the specification, no specific custom headers are defined for the Naf_VFLInference service API.

6.2.3	Resources

6.2.3.1	Resource Structure

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.2.3.1-1 depicts the resource URIs structure for the Naf_VFLInference API.

Figure 6.2.3.1-1: Resource URI structure of the Naf_VFLInference API

Table 6.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| VFL Inference Subscriptions | /subscriptions | POST | Creates a new Individual VFL Inference Subscription resource. |
| Individual VFL Inference Subscription | /subscriptions/{subscriptionId} | GET | Retrieve an existing Individual VFL Inference Subscription identified by subresource {subscriptionId}. |
|  |  | PUT | Updates an existing Individual VFL Inference Subscription identified by subresource {subscriptionId}. |
|  |  | PATCH | Modifies an existing Individual VFL Inference Subscription identified by subresource {subscriptionId}. |
|  |  | DELETE | Deletes an Individual VFL Inference Subscription identified by subresource {subscriptionId}. |

6.2.3.2	Resource: VFL Inference Subscriptions

6.2.3.2.1	Description

The VFL Inference Subscriptions resource represents all VFL Inference subscriptions to the Naf_VFLInference service at a given AF. The resource allows an NF service consumer to create a new Individual VFL Inference Subscription resource.

6.2.3.2.2	Resource definition

Resource URI: {apiRoot}/naf-vflinference/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.2.3.2.2-1.

Table 6.2.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1 |

6.2.3.2.3	Resource Standard Methods

6.2.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.2.3.2.3.1-1.

Table 6.2.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.3.2.3.1-3.

Table 6.2.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VflInferSub | M | 1 | Creates a new Individual VFL Inference Subscription resource. |

Table 6.2.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VflInferSub | M | 1 | 201 Created | The creation of an Individual VFL Inference Subscription resource is confirmed and a representation of that resource is returned. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: Failure cases are described in clause 6.2.7. |  |  |  |  |

Table 6.2.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/naf-vflinference/<apiVersion>/subscriptions/{subscriptionId} |

6.2.3.2.4	Resource Custom Operations

None in this release of the specification.

6.2.3.3	Resource: Individual VFL Inference Subscription

6.2.3.3.1	Description

The Individual VFL Inference Subscription resource represents a single VFL inference subscription to the Naf_VFLInference service at a given AF.

6.2.3.3.2	Resource definition

Resource URI: {apiRoot}/naf-vflinference/<apiVersion>/subscriptions/{subscriptionId}

The <apiVersion> shall be set as described in clause 6.2.1.

This resource shall support the resource URI variables defined in table 6.2.3.3.2-1.

Table 6.2.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1. |
| subscriptionId | string | Identifies a VFL inference subscription to the Naf_VFLInference service. |

6.2.3.3.3	Resource Standard Methods

6.2.3.3.3.1	GET

The GET method allows an NF service consumer to retrieve an existing "Individual VFL Inference Subscription" resource managed by the AF.

This method shall support the URI query parameters specified in table 6.2.3.3.3.1-1.

Table 6.2.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.1-2 and the response data structures and response codes specified in table 6.2.3.3.3.1-3.

Table 6.2.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VflInferSub | M | 1 | 200 OK | Successful case. The requested "Individual VFL Inference Subscription" resource is returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.2.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

Table 6.2.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

6.2.3.3.3.2	PUT

This method shall support the URI query parameters specified in table 6.2.3.3.3.2-1.

Table 6.2.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.2-2 and the response data structures and response codes specified in table 6.2.3.3.3.2-3.

Table 6.2.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VflInferSub | M | 1 | Parameters to replace a subscription to VFL Subscription resource. |

Table 6.2.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VflInferSub | M | 1 | 200 OK | The Individual VFL Subscription resource was modified successfully, and a representation of that resource is returned. |
| n/a |  |  | 204 No Content | The Individual VFL Subscription resource was modified successfully. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 3) |
| NOTE 1: The mandatory HTTP error status codes for the PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). NOTE 3: Failure cases are described in clause 6.2.7. |  |  |  |  |

Table 6.2.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the request is redirected |

Table 6.2.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the request is redirected |

6.2.3.3.3.3	PATCH

This method shall support the URI query parameters specified in table 6.2.3.3.3.3-1.

Table 6.2.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.3-2 and the response data structures and response codes specified in table 6.2.3.3.3.3-3.

Table 6.2.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VflInferSubPatch | M | 1 | Partial update of parameters to a subscription to VFL Inference Subscription resource. |

Table 6.2.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VflInferSub | M | 1 | 200 OK | The Individual VFL Inference Subscription resource was partial modified successfully and a representation of that resource is returned. |
| n/a |  |  | 204 No Content | The Individual VFL Inference Subscription resource was partial modified successfully. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 3) |
| NOTE 1: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). NOTE 3: Failure cases are described in clause 6.2.7. |  |  |  |  |

Table 6.2.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the request is redirected |

Table 6.2.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the request is redirected |

6.2.3.3.3.4	DELETE

This method shall support the URI query parameters specified in table 6.2.3.3.3.4-1.

Table 6.2.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.4-2 and the response data structures and response codes specified in table 6.2.3.3.3.4-3.

Table 6.2.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |

Table 6.2.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case: The Individual VFL Subscription resource matching the subscriptionId was deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.2.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.2.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the request is redirected. |

6.2.3.3.4	Resource Custom Operations

None in this release of the specification.

6.2.4	Custom Operations without associated resources

None in this release of the specification.

6.2.5	Notifications

6.2.5.1	General

Notifications shall comply with clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

Table 6.2.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Event Notification | {notifUri} | POST | Report one or several observed Events. |

6.2.5.2	VFL Inference Event Notification

6.2.5.2.1	Description

The VFL Inference Event Notification is used by the AF to report one or several observed VFL Inference Events to a NF service consumer that has subscribed to such Notifications via the Individual VFL Inference Subscription Resource.

6.2.5.2.2	Operation Definition

Callback URI: {notifUri}

The operation shall support the callback URI variables defined in table 6.2.5.2.2-1, the request data structures specified in table 6.2.5.2.2-2 and the response data structure and response codes specified in table 6.2.5.2.2-3.

Table 6.2.5.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notifUri | Uri | The Notification Uri as assigned within the Individual VFL Inference Subscription. |

Table 6.2.5.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VflInferNotif | M | 1 | Provides Information about observed events. |

Table 6.2.5.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |  |

Table 6.2.5.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the notification request is redirected. |

Table 6.2.5.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the notification request is redirected |

6.2.6	Data Model

6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the Naf_VFLInference service-based interface protocol.

Table 6.2.6.1-1: Naf_VFLInference specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| VflInferSub | 6.2.6.2.2 | Represents a VFL inference subscription. |  |
| VflInferSubPatch | 6.2.6.2.3 | Represents parameters to request the modification of a VFL inference subscription. |  |

Table 6.2.6.1-2 specifies data types re-used by the Naf_VFLInference service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Naf_VFLInference service-based interface.

Table 6.2.6.1-2: Naf_VFLInference re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ReportingInformation | 3GPP TS 29.523 [17] | Represents the type of reporting a subscription requires. |  |
| RedirectResponse | 3GPP TS 29.571 [16] | Contains redirection related information. |  |
| SupportedFeatures | 3GPP TS 29.571 [16] | Represents the list of supported features. |  |
| Uri | 3GPP TS 29.571 [16] | Represents a URI. |  |
| VflInferAnaSub | 3GPP TS 29.520 [18] | Represents the VFL inference subscription information for the subscribed analytics ID. |  |
| VflInferNotif | 3GPP TS 29.520 [18] | Represents notification of a VFL inference subscription. |  |
| VflInferResult | 3GPP TS 29.520 [18] | Represents intermediate VFL inference result per target. |  |

6.2.6.2	Structured data types

6.2.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.2.6.2.2	Type VflInferSub

Table 6.2.6.2.2-1: Definition of type VflInferSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifCorrId | string | M | 1 | The value of Notification Correlation ID in the corresponding notification. |  |
| notifUri | Uri | M | 1 | URI at which the NF service consumer requests to receive notifications. |  |
| reportingReqs | ReportingInformation | O | 0..1 | Reporting requirement information of the VFL inference subscription. |  |
| suppFeats | SupportedFeatures | C | 0..1 | List of Supported features used as described in clause 6.2.8. It shall be supplied by NF service consumer in the POST requests that request the creation of a VFL Subscriptions resource and shall be supplied by the AF in the reply of corresponding request. |  |
| vflInferAnaSubs | array(VflInferAnaSub) | M | 1..N | Contains the VFL inference subscription information for the subscribed analytics ID(s). (NOTE) |  |
| vflInferResults | array(VflInferResult) | O | 1..N | Represents intermediate VFL inference results. This attribute may be present only if immediate reporting was requested via the "reportingReqs" attribute. |  |
| NOTE: The "intGroupIds" and "supis" attributes of the data type are not applicable for the AF services if the AF is an untrusted AF. The "exterGroupIds" and "gpsis" attributes of the data type are not applicable for the AF services if the AF is a trusted AF. |  |  |  |  |  |

6.2.6.2.3	Type VflInferSubPatch

Table 6.2.6.2.3-1: Definition of type VflInferSubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | O | 0..1 | URI at which the NF service consumer requests to receive notifications. |  |
| notifCorrId | string | O | 0..1 | The value of Notification Correlation ID in the corresponding notification. |  |
| reportingReqs | ReportingInformation | O | 0..1 | Reporting requirements of the VFL inference subscription. |  |
| vflInferAnaSubs | array(VflInferAnaSub) | O | 1..N | Contains the VFL inference subscription information for the subscribed analytics ID(s). (NOTE) |  |
| NOTE: The "intGroupIds" and "supis" attributes of the data type are not applicable for the AF services if the AF is an untrusted AF. The "exterGroupIds" and "gpsis" attributes of the data type are not applicable for the AF services if the AF is a trusted AF. |  |  |  |  |  |

6.2.6.3	Simple data types and enumerations

6.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.2.6.3.2	Simple data types

The simple data types defined in table 5.1.6.3.2-1 shall be supported.

Table 6.2.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

6.2.7	Error handling

6.2.7.1	General

For the Naf_VFLInference API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Naf_VFLInference API.

6.2.7.2	Protocol Errors

No specific procedures for the Naf_VFLInference service are specified.

6.2.7.3	Application Errors

The application errors defined for the Naf_VFLInference API are listed in table 6.2.7.3-1.

Table 6.2.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| OVERLOAD | 403 Forbidden | Indicates the AF is overloaded. |  |
| NOT_AVAILABLE_FOR_VFL_PROCESS | 403 Forbidden | Indicates the AF is not available for the VFL process anymore. |  |
| VFL_REQS_NOT_MET | 403 Forbidden | Indicates the VFL inference requirements are not met. |  |
| NOTE: Including a "ProblemDetails" data structure with the "cause" attribute in the HTTP response is optional unless explicitly mandated in the service operation clauses. |  |  |  |

6.2.8	Feature negotiation

The optional features in table 6.2.8-1 are defined for the Naf_VFLInference API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

6.2.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Naf_VFLInference API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Naf_VFLInference API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Naf_VFLInference service.

The Naf_VFLInference API defines a single scope "naf-vflinference" for the entire service, and it does not define any additional scopes at resource or operation level.

6.2.10	HTTP redirection

An HTTP request may be redirected to a different AF service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different AF producer instance will return the NF Instance ID of the new AF producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an AF redirects a service request to a different AF using an HTTP 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new AF towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the HTTP 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

6.3	Naf_Training Service API

6.3.1	Introduction

The Naf_Training shall use the Naf_Training API.

The API URI of the Naf_Training API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "naf-train".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clauses 6.3.3 and 6.3.4.

6.3.2	Usage of HTTP

6.3.2.1	General

HTTP/2, IETF RFC 9113 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Naf_Training API is contained in Annex A.

6.3.2.2	HTTP standard headers

6.3.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

6.3.2.2.2	Content type

If the AF is untrusted, support of HTTP/1.1 (IETF RFC 9112 [19], IETF RFC 9110 [20] and IETF RFC 9111[21] over TLS is mandatory and support of HTTP/2 (IETF RFC 9113 [11]) over TLS is recommended. TLS shall be used as specified in clause 12.3 and clause 13.1 of 3GPP TS 33.501 [8].

If the AF is trusted, HTTP/2, IETF RFC 9113 [11], shall be used as specified in clause 5.2 of 3GPP TS 29.500 [4].

HTTP/2, IETF RFC 9113 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Naf_Training API is contained in Annex A.

6.3.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

6.3.3	Resources

6.3.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.3.3.1-1 depicts the resource URIs structure for the Naf_Training API.

Figure 6.3.3.1-1: Resource URI structure of the Naf_Training API

Table 6.3.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.3.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Training Subscriptions | /subscriptions | POST | Create a new Training Subscription. |
| Individual Training Subscription | /subscriptions/{subscriptionId} | GET | Retrieve an existing "Individual Training Subscription" resource. |
|  |  | PUT | Request the update of an existing "Individual Training Subscription" resource. |
|  |  | PATCH | Request the modification of an existing "Individual Training Subscription" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual Training Subscription" resource. |

6.3.3.2	Resource: Training Subscriptions

6.3.3.2.1	Description

This resource represents the collection of Training Subscription(s) managed by the AF.

6.3.3.2.2	Resource Definition

Resource URI: {apiRoot}/naf-train/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.3.3.2.2-1.

Table 6.3.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.3.1. |

6.3.3.2.3	Resource Standard Methods

6.3.3.2.3.1	POST

The HTTP POST method allows a service consumer to request the creation of a Training Subscription at the AF.

This method shall support the URI query parameters specified in table 6.3.3.2.3.1-1.

Table 6.3.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.2.3.1-2 and the response data structures and response codes specified in table 6.3.3.2.3.1-3.

Table 6.3.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrainEventsSubsc | M | 1 | Represents the parameters to request the creation of a Training Subscription. |

Table 6.3.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrainEventsSubsc | M | 1 | 201 Created | Successful case. The Training Subscription is successfully created and a representation of the created "Individual Training Subscription" resource shall be returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| ProblemDetails |  | O |  | 0..1 | 403 Forbidden | (NOTE 2) |  |
| NOTE 1: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: Failure cases are described in clause 6.3.7. |  |  |  |  |

Table 6.3.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/naf-train/<apiVersion>/subscriptions/{subscriptionId} |

6.3.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

6.3.3.3	Resource: Individual Training Subscription

6.3.3.3.1	Description

This resource represents a Training Subscription managed by the AF.

6.3.3.3.2	Resource Definition

Resource URI: {apiRoot}/naf-train/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.3.3.3.2-1.

Table 6.3.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.3.1. |
| subscriptionId | string | Represents the unique identifier of the "Individual Training Subscription" resource. |

6.3.3.3.3	Resource Standard Methods

6.3.3.3.3.1	GET

The GET method allows an NF service consumer to retrieve an existing "Individual Training Subscription" resource managed by the AF.

This method shall support the URI query parameters specified in table 6.3.3.3.3.1-1.

Table 6.3.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.3.3.1-2 and the response data structures and response codes specified in table 6.3.3.3.3.1-3.

Table 6.3.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.3.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrainEventsSubsc | M | 1 | 200 OK | Successful case. The requested "Individual Training Subscription" resource is returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.30.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.3.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.30.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

Table 6.3.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.30.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

6.3.3.3.3.2	PUT

The PUT method allows an NF service consumer to request the update of an existing "Individual Training Subscription" resource managed by the AF.

This method shall support the URI query parameters specified in table 6.3.3.3.3.2-1.

Table 6.3.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.3.3.2-2 and the response data structures and response codes specified in table 6.3.3.3.3.2-3.

Table 6.3.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrainEventsSubsc | M | 1 | Contains the updated representation of the "Individual Training Subscription" resource. |

Table 6.3.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrainEventsSubsc | M | 1 | 200 OK | Successful case. The "Individual Training Subscription" resource is successfully updated and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Training Subscription" resource is successfully updated and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 3) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.30.9.1 of 3GPP TS 29.500 [4]). NOTE 3: Failure cases are described in clause 6.3.7. |  |  |  |  |

Table 6.3.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

Table 6.3.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

6.3.3.3.3.3	PATCH

The PATCH method allows an NF service consumer to request the modification of an existing "Individual Training Subscription" resource managed by the AF.

This method shall support the URI query parameters specified in table 6.3.3.3.3.3-1.

Table 6.3.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.3.3.3-2 and the response data structures and response codes specified in table 6.3.3.3.3.3-3.

Table 6.3.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrainEventsSubscPatch | M | 1 | Contains the parameters to request the modification of the "Individual Training Subscription" resource. |

Table 6.3.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrainEventsSubsc | M | 1 | 200 OK | Successful case. The "Individual Training Subscription" resource is successfully modified and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Training Subscription" resource is successfully modified and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 3) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). NOTE 3: Failure cases are described in clause 6.3.7. |  |  |  |  |

Table 6.3.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

Table 6.3.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

6.3.3.3.3.4	DELETE

The DELETE method allows an NF service consumer to request the deletion of an existing "Individual Training Subscription" resource managed by the AF.

This method shall support the URI query parameters specified in table 6.3.3.3.3.4-1.

Table 6.3.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.3.3.4-2 and the response data structures and response codes specified in table 6.3.3.3.3.4-3.

Table 6.3.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.3.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual Training Subscription" resource is successfully deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.3.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

Table 6.3.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

6.3.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

6.3.5	Notifications

6.3.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

Table 6.3.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Training Notification | {notifUri} | POST | Enables the AF to notify a previously subscribed NF service consumer on Training report(s). |

6.3.5.2	Training Notification

6.3.5.2.1	Description

The Training Notification is used by the AF to notify a previously subscribed NF service consumer on Training report(s).

6.3.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.3.5.2.2-1.

Table 6.3.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Represents the callback URI encoded as a string formatted as a URI. |

6.3.5.2.3	Standard Methods

6.3.5.2.3.1	POST

This method shall support the request data structures specified in table 6.3.5.2.3.1-1 and the response data structures and response codes specified in table 6.3.5.2.3.1-2.

Table 6.3.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrainEventsNotif | M | 1 | Represents the Training Notification. |

Table 6.3.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Training Notification is successfully received. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.3.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF service consumer (service) instance towards which the notification request is redirected. |

Table 6.3.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF service consumer (service) instance towards which the notification request is redirected. |

6.3.6	Data Model

6.3.6.1	General

This clause specifies the application data model supported by the API.

Table 6.3.6.1-1 specifies the data types defined for the Naf_Training service-based interface protocol.

Table 6.3.6.1-1: Naf_Training API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| EventNotif | 6.3.6.2.7 | Represents notification on event training. |  |
| EventSubsc | 6.3.6.2.4 | Represents an event training subscription. |  |
| MlModelMonitorInfo | 6.3.6.2.5 | Represents the ML model monitoring information. |  |
| TrainEventsNotif | 6.3.6.2.6 | Represents notification on training event(s) that occurred. |  |
| TrainEventsSubsc | 6.3.6.2.2 | Represents event(s) training subscription. |  |
| TrainEventsSubscPatch | 6.3.6.2.3 | Represents the requested modifications to an event training subscription. |  |

Table 6.3.6.1-2 specifies data types re-used by the Naf_Training service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Naf_Training service-based interface.

Table 6.3.6.1-2: Naf_Training API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Accuracy | 3GPP TS 29.520 [18] | Represents accuracy levels of interest for ML models. |  |
| DateTime | 3GPP TS 29.571 [16] | Identifies the time. |  |
| DelayEventNotif | 3GPP TS 29.520 [18] | Contains information related to the inability to provide the notification as requested. |  |
| DurationSec | 3GPP TS 29.571 [16] | Represents a time duration expressed in units of seconds. |  |
| EventFilter | 3GPP TS 29.520 [18] | Represents the event filters used to identify the subscribed analytics. |  |
| InferenceDataForModelTrain | 3GPP TS 29.520 [18] | Represents the inference data for model training. |  |
| InputDataInfo | 3GPP TS 29.520 [18] | Represents the metrics of the input data. |  |
| MLModelMetric | 3GPP TS 29.520 [18] | Represents the ML Model Metric. |  |
| NetworkAreaInfo | 3GPP TS 29.554 [23] | Identifies the network area. |  |
| NwdafEvent | 3GPP TS 29.520 [18] | Represents the NWDAF event. |  |
| ReportingInformation | 3GPP TS 29.523 [17] | Represents the event reporting requirements. |  |
| SupportedFeatures | 3GPP TS 29.571 [16] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| TargetUeInformation | 3GPP TS 29.520 [18] | Identifies the target UE information. |  |
| TimeWindow | 3GPP TS 29.122 [22] | Represents a time window. |  |
| Uinteger | 3GPP TS 29.571 [16] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.571 [16] | Represents a URI. |  |
| VflTermCause | 3GPP TS 29.520 [18] | Identifies the VFL termination cause. |  |

6.3.6.2	Structured data types

6.3.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.3.6.2.2	Type: TrainEventsSubsc

Table 6.3.6.2.2-1: Definition of type TrainEventsSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| trainEventSubs | map(EventSubsc) | M | 1..N | Contains the subscribed training events. The key of the map shall be set to the value of the "event" attribute in the EventSubsc data structure. |  |
| notifUri | Uri | M | 1 | Contains the URI via which the training related notifications shall be delivered. |  |
| notifCorreId | string | M | 1 | The value of Notification Correlation ID in the corresponding notification. |  |
| reportingReqs | ReportingInformation | O | 0..1 | Contains reporting requirement information of the subscription. If omitted, the default values within the ReportingInformation data type apply. |  |
| eventNotifs | array(EventNotif) | O | 1..N | Contains the training related even(s) report(s). This attribute may be present only if immediate reporting was requested via the "reportingReqs" attribute. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.3.8. This attribute shall be present only when feature negotiation is required. |  |

6.3.6.2.3	Type: TrainEventsSubscPatch

Table 6.3.6.2.3-1: Definition of type TrainEventsSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| trainEventSubs | map(EventSubsc) | O | 1..N | Contains the updated subscribed training events. The key of the map shall be set to the value of the "event" attribute in the EventSubsc data structure. |  |
| notifUri | Uri | O | 0..1 | Contains the updated URI via which the training related notifications shall be delivered. |  |
| notifCorreId | string | O | 0..1 | The value of Notification Correlation ID in the corresponding notification. |  |
| reportingReqs | ReportingInformation | O | 0..1 | Contains the reporting requirement information of the subscription. |  |

6.3.6.2.4	Type: EventSubsc

Table 6.3.6.2.4-1: Definition of type EventSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | NwdafEvent | M | 1 | Event that is subscribed. |  |
| useCaseCxt | string | O | 0..1 | Indicates the context of usage of the analytics. The value and format of this parameter are not standardized. |  |
| trainFilter | EventFilter | O | 0..1 | Identifies the training filter information for the monitored event. |  |
| tgtUe | TargetUeInformation | O | 0..1 | Identifies target UE information. (NOTE) |  |
| exterGroupIds | array(ExternalGroupId) | O | 1..N | Identifies the external group of UE(s) to which the inference applies. (NOTE) |  |
| repRatio | Uinteger | O | 0..1 | Minimum percentage of UEs whose data is used for training an ML model when the target of ML model reporting is a group of UEs. Minimum = 0. Maximum = 100. |  |
| targetPeriod | TimeWindow | O | 0..1 | Indicates the time interval for which the ML model for the analytics is requested. |  |
| inferInputData | InputDataInfo | O | 0..1 | Indicates the Inference Input Data information. |  |
| timeModelNeeded | DateTime | O | 0..1 | Indicating the latest time when the consumer expects to receive the ML Model(s). |  |
| modelMonInfo | MlModelMonitorInfo | O | 0..1 | Contains the ML Model Monitoring Information. |  |
| accuLevel | Accuracy | O | 0..1 | Contains the accuracy level of interest. |  |
| NOTE: The "gpsis" attribute within the TargetUeInformation data structure and "exterGroupIds" attribute are not applicable for the AF services if the AF is a trusted AF. The "intGroupIds" and "supis" attributes within the TargetUeInformation data structure are not applicable for the AF services if the AF is an untrusted AF. |  |  |  |  |  |

6.3.6.2.5	Type: MlModelMonitorInfo

Table 6.3.6.2.5-1: Definition of type MlModelMonitorInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| modelMetric | MLModelMetric | O | 0..1 | Indicates the ML model metric. |  |
| accuThreshold | Uinteger | O | 0..1 | Accuracy reporting threshold. Indicates the accuracy threshold of the ML Model requested by the consumer Minimum = 0. Maximum = 100. |  |
| accuPeriod | DurationSec | O | 0..1 | Indicates the periodic reporting time. (NOTE) |  |
| storedData | InferenceDataForModelTrain | O | 0..1 | Indicates the inference data stored in the ADRF. |  |
| NOTE: When present, this attribute shall take precedence over the periodic reporting time indicated by the "repPeriod" attribute within the "reportingReqs" attribute (of data type ReportingInformation) in the TrainEventsSubsc data type. |  |  |  |  |  |

6.3.6.2.6	Type: TrainEventsNotif

Table 6.3.6.2.6-1: Definition of type TrainEventsNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifCorreId | string | M | 1 | Contains the notification correlation identifier. |  |
| eventNotifs | array(EventNotif) | M | 1..N | Contains the Training related event(s) notification(s). |  |

6.3.6.2.7	Type: EventNotif

Table 6.3.6.2.7-1: Definition of type EventNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | NwdafEvent | M | 1 | Contains the identifier of the analytics to which the training notification is related. |  |
| vflNotOnTimeInd | DelayEventNotif | O | 0..1 | Contains information indicating that the AF is not able to provide the training results within the maximum response time. (NOTE 1)(NOTE 2) |  |
| trainingInd | boolean | O | 0..1 | Set to "true" to indicate that the ML model training is ongoing. Set to "false" to indicate that the ML model training is done. Default value is "false" if omitted. (NOTE 1) |  |
| validityPeriod | TimeWindow | O | 0..1 | Indicates the time period when the provided ML model applies. (NOTE 1) |  |
| spatialValidity | NetworkAreaInfo | O | 0..1 | Indicates the area where the provided ML model applies. (NOTE 1) |  |
| repRatio | Uinteger | O | 0..1 | Minimum percentage of UEs whose data is used for training an ML model when the target of ML model reporting is a group of UEs. Minimum = 0. Maximum = 100. (NOTE 1) |  |
| accMLModel | Uinteger | O | 0..1 | Indicates the accuracy value of the ML model. Minimum = 0. Maximum = 100. (NOTE 1) |  |
| termCause | VflTermCause | O | 0..1 | Contains the termination cause. (NOTE 1) |  |
| vflCorrId | string | O | 0..1 | Contains the VFL Correlation ID. (NOTE 1) |  |
| NOTE 1: If the "termCause" attribute is present, the rest of the attributes indicated by this NOTE shall not be present. NOTE 2: The "ML_MODEL_TRAIN_FAILURE" value within the "delayCause" attribute in the "vflNotOnTimeInd" attribute is not applicable. The attributes within the "vflNotOnTimeInd" attribute apply for VFL here. |  |  |  |  |  |

6.3.6.3	Simple data types and enumerations

6.3.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.3.6.3.2	Simple data types

The simple data types defined in table 6.3.6.3.2-1 shall be supported.

Table 6.3.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

6.3.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

6.3.6.5	Binary data

6.3.6.5.1	Binary Data Types

Table 6.3.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

6.3.7	Error Handling

6.3.7.1	General

For the Naf_Training API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Naf_Training API.

6.3.7.2	Protocol Errors

No specific procedures for the Naf_Training service are specified.

6.3.7.3	Application Errors

The application errors defined for the Naf_Training service are listed in Table 6.3.7.3-1.

Table 6.3.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| OVERLOAD | 403 Forbidden | Indicates the AF is overloaded. |  |
| NOTE: Including a "ProblemDetails" data structure with the "cause" attribute in the HTTP response is optional unless explicitly mandated in the service operation clauses. |  |  |  |

6.3.8	Feature negotiation

The optional features in table 6.3.8-1 are defined for the Naf_Training API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.3.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

6.3.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Naf_Training API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Naf_Training API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in clause 5.4.2.2 of 3GPP TS 29.510 [10].

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Naf_Training service.

The Naf_Training API defines a single scope "naf-train" for the entire service, and it does not define any additional scopes at resource or operation level.

6.3.10	HTTP redirection

An HTTP request may be redirected to a different AF service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different AF producer instance will return the NF Instance ID of the new AF producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an AF redirects a service request to a different AF using an HTTP "307 Temporary Redirect" or "308 Permanent Redirect" status code, the identity of the new AF towards which the service request is redirected shall be indicated in the "3gpp-Sbi-Target-Nf-Id" header of the HTTP "307 Temporary Redirect" or "308 Permanent Redirect" response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

6.4	Naf_Inference Service API

6.4.1	Introduction

The Naf_Inference service shall use the Naf_Inference API.

The API URI of the Naf_Inference API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "naf-inference".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.4.3.

6.4.2	Usage of HTTP

6.4.2.1	General

If the AF is untrusted, support of HTTP/1.1 (IETF RFC 9112 [19], IETF RFC 9110 [20] and IETF RFC 9111[21] over TLS is mandatory and support of HTTP/2 (IETF RFC 9113 [11]) over TLS is recommended. TLS shall be used as specified in clause 12.3 and clause 13.1 of 3GPP TS 33.501 [8].

If the AF is trusted, HTTP/2, IETF RFC 9113 [11], shall be used as specified in clause 5.2 of 3GPP TS 29.500 [4].

HTTP/2, IETF RFC 9113 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.5 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Naf_Inference API is contained in Annex A.

6.4.2.2	HTTP standard headers

6.4.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

6.4.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 9457 [13].

6.4.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.5.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.5.3.3 of 3GPP TS 29.500 [4] may be supported.

In this Release of the specification, no specific custom headers are defined for the Naf_Inference API.

6.4.3	Resources

6.4.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.4.3.1-1 depicts the resource URIs structure for the Naf_Inference API.

Figure 6.4.3.1-1: Resource URI structure of the Naf_Inference API

Table 6.4.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.4.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Inference Subscriptions | /subscriptions | POST | Creates a new Individual Inference Subscription resource. |
| Individual Inference Subscription | /subscriptions/{subscriptionId} | PUT | Updates an existing Individual Inference Subscription identified by subresource {subscriptionId}. |
|  |  | PATCH | Modifies an existing Individual Inference Subscription identified by subresource {subscriptionId}. |
|  |  | DELETE | Deletes an Individual Inference Subscription identified by subresource {subscriptionId}. |

6.4.3.2	Resource: Inference Subscriptions

6.4.3.2.1	Description

The Inference Subscriptions resource represents all Inference subscriptions to the Naf_Inference service at a given AF. The resource allows an NF service consumer to create a new Individual Inference Subscription resource.

6.4.3.2.2	Resource Definition

Resource URI: {apiRoot}/naf-inference/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.4.3.2.2-1.

Table 6.4.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.4.1 |

6.4.3.2.3	Resource Standard Methods

6.4.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.4.3.2.3.1-1.

Table 6.4.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.2.3.1-2 and the response data structures and response codes specified in table 6.4.3.2.3.1-3.

Table 6.4.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| InferEventSubsc | M | 1 | Contains the information required for the creation of a new Individual Inference Subscription resource. "intGroupIds" and "supis" target identities on "inferAnaSubs" attribute under this structure are not applicable to this API if the NF consumer is a NEF, i.e., the AF is an untrusted AF. "exterGroupIds" and "gpsis" target identities on "inferAnaSubs" attribute under this structure are not applicable to this API if the NF consumer is an NWDAF, i.e., the AF is a trusted AF. |

Table 6.4.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| InferEventSubsc | M | 1 | 201 Created | Contains the representation of the Individual Inference Subscription resource. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the POST method listed in Table 5.5.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: Failure cases are described in clause 6.4.7. |  |  |  |  |

Table 6.4.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/naf-inference/<apiVersion>/subscriptions/{subscriptionId} |

6.4.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

6.4.3.3	Resource: Individual Inference Subscription

6.4.3.3.1	Description

The Individual Inference Subscription resource represents a single inference subscription to the Naf_Inference service at a given AF.

6.4.3.3.2	Resource Definition

Resource URI: {apiRoot}/naf-inference/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.4.3.3.2-1.

Table 6.4.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.4.1 |
| subscriptionId | string | Identifies an inference subscription to the Naf_Inference service. |

6.4.3.3.3	Resource Standard Methods

6.4.3.3.3.1	PUT

This method shall support the URI query parameters specified in table 6.4.3.3.3.1-1.

Table 6.4.3.3.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.1-2 and the response data structures and response codes specified in table 6.4.3.3.3.1-3.

Table 6.4.3.3.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| InferEventSubsc | M | 1 | Parameters to replace a subscription to an Individual Inference Subscription resource. |

Table 6.4.3.3.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| InferEventSubsc | M | 1 | 200 OK | Successful case. The Individual Inference Subscription resource was modified successfully and a representation is returned. "intGroupIds" and "supis" target identities on "inferAnaSubs" attribute under this structure are not applicable to this API if the NF consumer is a NEF, i.e., the AF is an untrusted AF. "exterGroupIds" and "gpsis" target identities on "inferAnaSubs" attribute under this structure are not applicable to this API if the NF consumer is an NWDAF, i.e., the AF is a trusted AF. |
| n/a |  |  | 204 No Content | Successful case. The Individual Inference Subscription resource was modified successfully. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 3) |
| NOTE 1: The mandatory HTTP error status codes for the PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). NOTE 3: Failure cases are described in clause 6.4.7. |  |  |  |  |

Table 6.4.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.4.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the request is redirected. |

6.4.3.3.3.2	PATCH

This method shall support the URI query parameters specified in table 6.4.3.3.3.2-1.

Table 6.4.3.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.2-2 and the response data structures and response codes specified in table 6.4.3.3.3.2-3.

Table 6.4.3.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| InferEventSubscPatch | M | 1 | Partial update of parameters to a subscription to an Individual Inference Subscription resource |

Table 6.4.3.3.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| InferEventSubsc | M | 1 | 200 OK | The Individual Inference Subscription resource was partial modified successfully and a representation of that resource is returned. "intGroupIds" and "supis" target identities on "inferAnaSubs" attribute under this structure are not applicable to this API if the NF consumer is a NEF, i.e., the AF is an untrusted AF. "exterGroupIds" and "gpsis" target identities on "inferAnaSubs" attribute under this structure are not applicable to this API if the NF consumer is an NWDAF, i.e., the AF is a trusted AF. |
| n/a |  |  | 204 No Content | The Individual AF Inference Subscription resource was partial modified successfully. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 3) |
| NOTE 1: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). NOTE 3: Failure cases are described in clause 6.4.7. |  |  |  |  |

Table 6.4.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.4.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the request is redirected. |

6.4.3.3.3.3	DELETE

This method shall support the URI query parameters specified in table 6.4.3.3.3.3-1.

Table 6.4.3.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.3-2 and the response data structures and response codes specified in table 6.4.3.3.3.3-3.

Table 6.4.3.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.4.3.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Individual Inference Subscription resource matching the subscriptionId was deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.4.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.4.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

6.4.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

6.4.4	Custom Operations without associated resources

There are no custom operations defined for this resource in this release of the specification.

6.4.5	Notifications

6.4.5.1	General

Notifications shall comply to clause 6.4 of 3GPP TS 29.500 [4] and clause 4.6.4.3 of 3GPP TS 29.501 [5].

Table 6.4.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Inference Notification | {notifUri} | POST | Provides Information about observed events. |

6.4.5.5	Inference notification

6.4.5.5.1	Description

The Inference Event Notification is used by the AF to report one or several observed Inference Events to an NF service consumer that has subscribed to such Notifications via the Individual Inference Subscription resource.

6.4.5.5.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.4.5.5.2-1.

Table 6.4.5.5.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | String formatted as URI with the Callback Uri |

6.4.5.5.3	Standard Methods

6.4.5.5.3.1	POST

This method shall support the request data structures specified in table 6.4.5.5.3.1-1 and the response data structures and response codes specified in table 6.4.5.5.3.1-1.

Table 6.4.5.5.3.1-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| InferNotif | M | 1 | Provides Information about observed events. "intGroupIds" and "supis" target identities on "inferResults" attribute under this structure are not applicable to this API if the NF consumer is a NEF, i.e., the AF is an untrusted AF. "exterGroupIds" and "gpsis" target identities on "inferResults" attribute under this structure are not applicable to this API if the NF consumer is an NWDAF, i.e., the AF is a trusted AF. |

Table 6.4.5.5.3.1-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during event notification. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during event notification. (NOTE 2) |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.5.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.4.5.5.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected. |

Table 6.4.5.5.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected. |

6.4.6	Data Model

6.4.6.1	General

This clause specifies the application data model supported by the API.

Table 6.4.6.1-1 specifies the data types defined for the Naf_Inference service based interface protocol.

Table 6.4.6.1-1: Naf_Inference specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| InferAnaSub | 6.4.6.2.4 | Represents notification of an inference subscription per analytics id. |  |
| InferNotif | 6.4.6.2.5 | Represents notification of an inference subscription. |  |
| InferEventSubsc | 6.4.6.2.2 | Represents an inference subscription. |  |
| InferEventSubscPatch | 6.4.6.2.3 | Represents parameters to request the modification of an inference subscription. |  |
| InferResult | 6.4.6.2.6 | Represents inference results. |  |

Table 6.4.6.1-2 specifies data types re-used by the Naf_Inference service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Naf_Inference service based interface.

Table 6.4.6.1-2: Naf_Inference re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DelayEventNotif | 3GPP TS 29.520 [18] | Indicates that the AF is not able to complete inference within the maximum response time. |  |
| AnalyticsMetadata | 3GPP TS 29.520 [18] | Represents the types of analytics metadata information that can be requested. |  |
| AnalyticsMetadataInfo | 3GPP TS 29.520 [18] | Contains analytics metadata information required for analytics aggregation. |  |
| DatasetStatisticalProperty | 3GPP TS 29.520 [18] | Dataset statistical properties of the data used for inference. |  |
| DateTime | 3GPP TS 29.571 [16] | Represents a date and time. |  |
| DurationSec | 3GPP TS 29.571 [16] | Represents a time duration expressed in units of seconds. |  |
| EventFilter | 3GPP TS 29.520 [18] | Identifies the filter for the subscribed event. |  |
| EventNotification | 3GPP TS 29.520 [18] | Describes Notifications about events that occurred. |  |
| EventReportingRequirement | 3GPP TS 29.520 [18] | Represents the analytics reporting requirement information. |  |
| ExternalGroupId | 3GPP TS 29.571 [16] | External Group Identifier for a user group. |  |
| Gpsi | 3GPP TS 29.571 [16] | The GPSI for a UE. |  |
| GroupId | 3GPP TS 29.571 [16] | Identifies a group of UEs. |  |
| NwdafEvent | 3GPP TS 29.520 [18] | Describes the NWDAF Events. |  |
| ReportingInformation | 3GPP TS 29.523 [17] | Represents the type of reporting a subscription requires. |  |
| RedirectResponse | 3GPP TS 29.571 [16] | Contains redirection related information. |  |
| Supi | 3GPP TS 29.571 [16] | The SUPI for a UE. |  |
| SupportedFeatures | 3GPP TS 29.571 [16] | Represents the list of supported features. |  |
| TimeWindow | 3GPP TS 29.122 [22] | Represents a time window. |  |
| Uinteger | 3GPP TS 29.571 [16] | Unsigned Integer, i.e. only value 0 and integers above 0 are permissible. |  |
| Uri | 3GPP TS 29.571 [16] | Represents a URI. |  |
| VflTermCause | 3GPP TS 29.520 [18] | Represents a cause of termination. |  |

6.4.6.2	Structured data types

6.4.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.4.6.2.2	Type InferEventSubsc

Table 6.4.6.2.2-1: Definition of type InferEventSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifCorreId | string | M | 1 | The value of Notification Correlation ID in the corresponding notification. |  |
| notifUri | Uri | M | 1 | URI at which the NF service consumer requests to receive notifications. |  |
| suppFeats | SupportedFeatures | C | 0..1 | List of Supported features used as described in clause 5.10.8. It shall be supplied by NF service consumer in the POST requests that request the creation of an Inference Subscriptions resource and shall be supplied by the AF in the reply of corresponding request. |  |
| inferAnaSubs | map(InferAnaSub) | M | 1..N | Identifies the inference subscription information for the subscribed analytics ID(s). The key of the map shall be set to the value of the "anaEvent" attribute in the InferAnaSub data structure. |  |
| inferResults | array(InferResult) | O | 1..N | Represents inference results. This attribute may be present only in the response message and if immediate reporting was requested via the "reportInfo" attribute. |  |
| reportInfo | ReportingInformation | O | 0..1 | Reporting requirement information of the inference subscription. If omitted, the default values within the ReportingInformation data type apply. |  |

6.4.6.2.3	Type InferEventSubscPatch

Table 6.4.6.2.3-1: Definition of type InferEventSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | O | 0..1 | URI at which the NF service consumer requests to receive notifications. |  |
| notifCorreId | string | O | 0..1 | The value of Notification Correlation ID in the corresponding notification. |  |
| inferAnaSubs | map(InferAnaSub) | O | 1..N | Contains the updated subscribed inference events. The key of the map shall be set to the value of the "anaEvent" attribute in the InferAnaSub data structure. |  |
| reportInfo | ReportingInformation | O | 0..1 | Reporting requirement information of the inference subscription. If omitted, the default values within the ReportingInformation data type apply. |  |

6.4.6.2.4	Type InferAnaSub

Table 6.4.6.2.4-1: Definition of type InferAnaSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anaEvent | NwdafEvent | M | 1 | Type of analytics for which inference is required. |  |
| eventFilter | EventFilter | O | 0..1 | Inference filter information. |  |
| exterGroupIds | array(ExternalGroupId) | C | 1..N | Identifies the external group of UE(s) to which the inference applies. (NOTE 1) (NOTE 2) |  |
| gpsis | array(Gpsi) | C | 1..N | Each element identifies a GPSI of an UE to which the inference applies. (NOTE 1) (NOTE 2) |  |
| intGroupIds | array(GroupId) | C | 1..N | Each element represents an internal group identifier of the UEs to which the inference applies. (NOTE 1) (NOTE 2) |  |
| supis | array(Supi) | C | 1..N | Each element identifies a SUPI of an UE to which the inference applies. (NOTE 1) (NOTE 2) |  |
| anaMeta | array(AnalyticsMetadata) | O | 1..N | List of analytics metadata that are requested to be included in the response. Only "NUM_OF_SAMPLES", "DATA_WINDOW", "DATA_STAT_PROPS" and “DATA_SOURCES” values are applicable. |  |
| dataStatProps | array(DatasetStatisticalProperty) | O | 1..N | List of dataset statistical properties of the data to be used to perform inference. |  |
| timeWindows | array(TimeWindow) | O | 1..N | The time periods for inference. |  |
| resTime | DateTime | O | 0..1 | Time when the local result is needed. |  |
| NOTE 1: The "exterGroupIds" and "gpsis" attributes are not applicable for the AF services if the AF is a trusted AF. The "intGroupIds" and "supis" attributes of the data type are not applicable for the AF services if the AF is an untrusted AF. NOTE 2: Only one of "exterGroupIds", "gpsis", "intGroupIds", or "supis" attributes shall be provided. |  |  |  |  |  |

6.4.6.2.5	Type InferNotif

Table 6.4.6.2.5-1: Definition of type InferNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifCorreId | string | M | 1 | The value of Notification Correlation ID in the corresponding notification. |  |
| inferResults | array(InferResult) | M | 1..N | Represents inference results. |  |

6.4.6.2.6	Type InferResult

Table 6.4.6.2.6-1: Definition of type InferResult

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| inferRes | EventNotification | C | 0..1 | Represents inference results. (NOTE) |  |
| vflNotOnTimeInd | DelayEventNotif | C | 0..1 | Indicates that the AF is not able to provide the inference results within the maximum response time. (NOTE) |  |
| termCause | VflTermCause | C | 0..1 | A cause for which the AF client will send no further notifications for this subscription to the event specified by "event" attribute. Its presence indicates that the AF requests the termination of the subscription to the event specified by "event" attribute. (NOTE) |  |
| NOTE: One of the "inferRes", "termCause" or "vflNotOnTimeInd" attributes shall be provided. The "ML_MODEL_TRAIN_FAILURE" value within the "delayCause" attribute in the "vflNotOnTimeInd" attribute is not applicable. The attributes within the "vflNotOnTimeInd" attribute here apply for VFL. |  |  |  |  |  |

6.4.6.3	Simple data types and enumerations

6.4.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.4.6.3.2	Simple data types

The simple data types defined in table 6.4.6.3.2-1 shall be supported.

Table 6.4.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

6.4.7	Error Handling

6.4.7.1	General

For the Naf_Inference API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.5.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.5.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Naf_Inference API.

6.4.7.2	Protocol Errors

No specific procedures for the Naf_Inference service are specified.

6.4.7.3	Application Errors

The application errors defined for the Naf_Inference service are listed in Table 6.4.7.3-1.

Table 6.4.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| OVERLOAD | 403 Forbidden | Indicates the AF is overloaded. |  |
| UE_LEFT_AREA | 403 Forbidden | Indicates the UE has moved out of the serving area. |  |
| INFERENCE_REQS_NOT_MET | 403 Forbidden | Indicates the inference requirements are not met. |  |
| NOT_AVAILABLE_FOR_VFL_PROCESS | 403 Forbidden | Indicates the AF is not available for inference process anymore. |  |
| NOTE: Including a "ProblemDetails" data structure with the "cause" attribute in the HTTP response is optional unless explicitly mandated in the service operation clauses. |  |  |  |

6.4.8	Feature negotiation

The optional features in table 6.4.8-1 are defined for the Naf_Inference API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.4.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

6.4.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Naf_Inference API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Naf_Inference API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Naf_Inference service.

The Naf_Inference API defines a single scope "naf-inference" for the entire service, and it does not define any additional scopes at resource or operation level.

6.4.10	HTTP redirection

An HTTP request may be redirected to a different AF service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different AF producer instance will return the NF Instance ID of the new AF producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.40.3.4 of 3GPP TS 29.500 [4].

If an AF redirects a service request to a different AF using an HTTP 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new AF towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the HTTP 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.40.9.1 of 3GPP TS 29.500 [4].

Annex A (normative):
OpenAPI specification

A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5.3.1 of 3GPP TS 29.501 [5] and clause 5B of 3GPP TR 21.900 [7]).

A.2	Naf_VFLTraining API

openapi: 3.0.0

info:

title: Naf_VFLTraining Service API

version: 1.0.0

description: |

AF VFL Training Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.530 V19.0.0; 5G System; Application Function Artificial Intelligence/Machine Learning

(AI/ML) Services.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.530/

servers:

- url: '{apiRoot}/naf-vfl-train/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- naf-vfl-train

paths:

/subscriptions:

post:

summary: Request the creation of a VFL Training Subscription.

operationId: CreateVflTrainSubsc

tags:

- VFL Training Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VflTrainingSubs'

responses:

'201':

description: >

Created. The VFL Training Subscription is successfully created and a representation of

the created Individual VFL Training Subscription resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/VflTrainingSubs'

headers:

Location:

description: >

Contains the URI of the created Individual VFL Training Subscription resource.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29520_Nnwdaf_VFLTraining.yaml#/components/schemas/VflTrainingNotify'

responses:

'204':

description: >

No Content. The VFL Training Notification is successfully received

and acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of the Individual VFL Training Subscription

resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual VFL Training Subscription resource.

operationId: GetIndVflTrainSubsc

tags:

- Individual VFL Training Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual VFL Training Subscription resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/VflTrainingSubs'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual VFL Training Subscription resource.

operationId: UpdateIndVflTrainSubsc

tags:

- Individual VFL Training Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VflTrainingSubs'

responses:

'200':

description: >

OK. The Individual VFL Training Subscription resource is successfully updated and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/VflTrainingSubs'

'204':

description: >

No Content. The Individual VFL Training Subscription resource is successfully updated

and no content is returned in the response body.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual VFL Training Subscription resource.

operationId: ModifyIndVflTrainSubsc

tags:

- Individual VFL Training Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/VflTrainingSubsPatch'

responses:

'200':

description: >

OK. The Individual VFL Training Subscription resource is successfully modified and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/VflTrainingSubs'

'204':

description: >

No Content. The Individual VFL Training Subscription resource is successfully modified

and no content is returned in the response body.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual VFL Training Subscription resource.

operationId: DeleteIndVflTrainSubsc

tags:

- Individual VFL Training Subscription (Document)

responses:

'204':

description: >

No Content. The Individual VFL Training Subscription resource is successfully deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

naf-vfl-train: >

Enables to access all the resources and custom operations of the Naf_VFLTraining API.

schemas:

#

# STRUCTURED DATA TYPES

#

VflTrainingSubs:

description: >

Represents a VFL Training Subscription.

type: object

properties:

vflTrainSubs:

type: array

items:

$ref: 'TS29520_Nnwdaf_VFLTraining.yaml#/components/schemas/VflTrainingSub'

minItems: 1

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorrId:

type: string

vflPreFlag:

type: boolean

description: >

Indicates the subscription is for preparation of VFL, when it is included and set to

"true". The default value is "false" if omitted.

reportingReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

trainReports:

type: array

items:

$ref: 'TS29520_Nnwdaf_VFLTraining.yaml#/components/schemas/VflTrainingNotify'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- vflTrainSubs

VflTrainingSubsPatch:

description: >

Represents the requested modifications to a VFL Training Subscription.

type: object

properties:

vflTrainSubs:

type: array

items:

$ref: 'TS29520_Nnwdaf_VFLTraining.yaml#/components/schemas/VflTrainingSub'

minItems: 1

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorrId:

type: string

reportingReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

# Data types describing alternative data types or combinations of data types:

#

A.3	Naf_VFLInference API

openapi: 3.0.0

info:

title: Naf_VFLInference

version: 1.0.1

description: |

Naf_VflInference API Service.

© 2026, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.530 V19.1.0; 5G System; Application Function Artificial Intelligence/Machine

Learning (AI/ML) Services

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.530/

servers:

- url: '{apiRoot}/naf-vflinference/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- naf-vflinference

paths:

/subscriptions:

post:

summary: Create a new Individual VFL Inference Subscription resource.

operationId: CreateVFLInferenceSubcription

tags:

- Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VflInferSub'

responses:

'201':

description: Create a new Individual VFL Inference Subscription resource.

content:

application/json:

schema:

$ref: '#/components/schemas/VflInferSub'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the

structure

{apiRoot}/naf-vflinference/v1/subscriptions/{subscriptionId}.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflInferNotif'

responses:

'204':

description: No Content, Notification was succesfull

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: String identifying a subscription to the Naf_VFLInference Service.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual VFL Inference Subscription resource.

operationId: GetIndVFLTrainSubsc

tags:

- Individual VFL Inference Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual VFL Inference Subscription resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/VflInferSub'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Update an existing Individual VFL Inference Subscription

operationId: UpdateVFLInferenceSubcription

tags:

- Individual VFL Inference Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VflInferSub'

responses:

'200':

description: >

The Individual VFL Inference Subscription resource was modified

successfully and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/VflInferSub'

'204':

description: >

The Individual VFL Inference Subscription resource was modified successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Partial update an existing Individual VFL Inference Subscription

operationId: PartialUpdateVFLInferenceSubcription

tags:

- Individual VFL Inference Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/VflInferSubPatch'

responses:

'200':

description: >

The Individual VFL Inference Subscription resource was partial modified successfully

and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/VflInferSub'

'204':

description: >

The Individual VFL Inference Subscription resource was partial modified

successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual VFL Inference Subscription.

operationId: DeleteVFLInferenceSubcription

tags:

- Individual VFL Inference Subscription (Document)

responses:

'204':

description: >

No Content. The Individual VFL Inference Subscription matching the

subscriptionId was deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

naf-vflinference: Access to the Naf_VflInference API

schemas:

VflInferSub:

description: Represents a VFL Inference subscription.

type: object

properties:

notifCorrId:

type: string

description: >

String identifying the Notification Correlation ID in the corresponding

notification.

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

reportingReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

suppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

vflInferAnaSubs:

type: array

items:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflInferAnaSub'

minItems: 1

description: >

Contains VFL inference subscription information for the subscribed analytics ID(s).

vflInferResults:

type: array

items:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflInferResult'

minItems: 1

description: Represents intermediate VFL Inference result.

required:

- notifUri

- notifCorrId

- vflInferAnaSubs

VflInferSubPatch:

description: >

Represents parameters to request the modification of a VFL Inference

subscription.

type: object

properties:

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorrId:

type: string

description: >

String identifying the Notification Correlation ID in the corresponding

notification.

reportingReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

vflInferAnaSubs:

type: array

items:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflInferAnaSub'

minItems: 1

description: >

Contains updated VFL inference subscription information for the subscribed

analytics ID(s).

#

# ENUMERATIONS DATA TYPES

#

A.4	Naf_Training API

openapi: 3.0.0

info:

title: Naf_Training Service API

version: 1.0.1

description: |

AF Training Service.

© 2026, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.530 V19.1.0; 5G System; Application Function Artificial Intelligence/Machine Learning

(AI/ML) Services.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.530/

servers:

- url: '{apiRoot}/naf-train/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- naf-train

paths:

/subscriptions:

post:

summary: Request the creation of a Training Subscription.

operationId: CreateTrainSubsc

tags:

- Training Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TrainEventsSubsc'

responses:

'201':

description: >

Created. The Training Subscription is successfully created and a representation of

the created Individual Training Subscription resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/TrainEventsSubsc'

headers:

Location:

description: >

Contains the URI of the created Individual Training Subscription resource.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

TrainEventsNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TrainEventsNotif'

responses:

'204':

description: >

No Content. The Training Notification is successfully received

and acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of the Individual Training Subscription

resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Training Subscription resource.

operationId: GetIndTrainSubsc

tags:

- Individual Training Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual Training Subscription resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/TrainEventsSubsc'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Training Subscription resource.

operationId: UpdateIndTrainSubsc

tags:

- Individual Training Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TrainEventsSubsc'

responses:

'200':

description: >

OK. The Individual Training Subscription resource is successfully updated and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/TrainEventsSubsc'

'204':

description: >

No Content. The Individual Training Subscription resource is successfully updated

and no content is returned in the response body.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Training Subscription resource.

operationId: ModifyIndTrainSubsc

tags:

- Individual Training Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/TrainEventsSubscPatch'

responses:

'200':

description: >

OK. The Individual Training Subscription resource is successfully modified and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/TrainEventsSubsc'

'204':

description: >

No Content. The Individual Training Subscription resource is successfully modified

and no content is returned in the response body.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Training Subscription resource.

operationId: DeleteIndTrainSubsc

tags:

- Individual Training Subscription (Document)

responses:

'204':

description: >

No Content. The Individual Training Subscription resource is successfully deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

naf-train: >

Enables to access all the resources and custom operations of the Naf_Training API.

schemas:

#

# STRUCTURED DATA TYPES

#

TrainEventsSubsc:

description: Represents a Training Subscription.

type: object

properties:

trainEventSubs:

type: object

additionalProperties:

$ref: '#/components/schemas/EventSubsc'

minProperties: 1

description: >

Contains the subscribed training events. The key of the map shall be set to the

value of the "event" attribute in the EventSubsc data structure.

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorreId:

type: string

reportingReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

eventNotifs:

type: array

items:

$ref: '#/components/schemas/EventNotif'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- trainEventSubs

- notifUri

- notifCorreId

TrainEventsSubscPatch:

description: Represents the requested modifications to a Training Subscription.

type: object

properties:

trainEventSubs:

type: object

additionalProperties:

$ref: '#/components/schemas/EventSubsc'

minProperties: 1

description: >

Contains the updated subscribed training events. The key of the map shall be set to the

value of the "event" attribute in the EventSubsc data structure.

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorreId:

type: string

reportingReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

EventSubsc:

description: Represents events training subscription.

type: object

properties:

event:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NwdafEvent'

useCaseCxt:

type: string

trainFilter:

$ref: 'TS29520_Nnwdaf_AnalyticsInfo.yaml#/components/schemas/EventFilter'

tgtUe:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/TargetUeInformation'

exterGroupIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExternalGroupId'

minItems: 1

repRatio:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

targetPeriod:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

inferInputData:

$ref: 'TS29520_Nnwdaf_MLModelProvision.yaml#/components/schemas/InputDataInfo'

timeModelNeeded:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

modelMonInfo:

$ref: '#/components/schemas/MlModelMonitorInfo'

accuLevel:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/Accuracy'

required:

- event

MlModelMonitorInfo:

description: Represents an event training subscription.

type: object

properties:

modelMetric:

$ref: 'TS29520_Nnwdaf_MLModelProvision.yaml#/components/schemas/MLModelMetric'

accuThreshold:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

accuPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

storedData:

$ref: 'TS29520_Nnwdaf_MLModelProvision.yaml#/components/schemas/InferenceDataForModelTrain'

TrainEventsNotif:

description: Represents a events training notification.

type: object

properties:

notifCorreId:

type: string

eventNotifs:

type: array

items:

$ref: '#/components/schemas/EventNotif'

minItems: 1

required:

- notifCorreId

- eventNotifs

EventNotif:

description: Represents an event training report.

type: object

properties:

event:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NwdafEvent'

vflNotOnTimeInd:

$ref: 'TS29520_Nnwdaf_MLModelTraining.yaml#/components/schemas/DelayEventNotif'

trainingInd:

type: boolean

description: >

Set to "true" to indicate that the ML model training is ongoing. Set to "false" to

indicate that the ML model training is done. Default value is "false" if omitted.

validityPeriod:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

spatialValidity:

$ref: 'TS29554_Npcf_BDTPolicyControl.yaml#/components/schemas/NetworkAreaInfo'

repRatio:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

accMLModel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

termCause:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflTermCause'

vflCorrId:

type: string

required:

- event

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

# Data types describing alternative data types or combinations of data types:

#

A.5	Naf_Inference API

openapi: 3.0.0

info:

title: Naf_Inference

version: 1.0.1

description: |

Naf_Inference API Service.

© 2026, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.530 V19.1.0; 5G System; Application Function Artificial

Intelligence/Machine Learning (AI/ML) Services.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.530/

servers:

- url: '{apiRoot}/naf-inference/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- naf-inference

paths:

/subscriptions:

post:

summary: Create a new Individual Inference Subscription resource.

operationId: CreateInferenceSubcription

tags:

- Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/InferEventSubsc'

responses:

'201':

description: Create a new Individual Inference Subscription resource.

content:

application/json:

schema:

$ref: '#/components/schemas/InferEventSubsc'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the

structure

{apiRoot}/naf-inference/v1/subscriptions/{subscriptionId}.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/InferNotif'

responses:

'204':

description: No Content, Notification was successful

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

put:

summary: Update an existing Individual Inference Subscription

operationId: UpdateInferenceSubcription

tags:

- Individual Inference Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/InferEventSubsc'

parameters:

- name: subscriptionId

in: path

description: String identifying a subscription to the Naf_Inference Service.

required: true

schema:

type: string

responses:

'200':

description: >

The Individual Inference Subscription resource was modified

successfully and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/InferEventSubsc'

'204':

description: >

The Individual Inference Subscription resource was modified

successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Partial update an existing Individual Inference Subscription

operationId: PartialUpdateInferenceSubcription

tags:

- Individual Inference Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/InferEventSubscPatch'

parameters:

- name: subscriptionId

in: path

description: String identifying a subscription to the Naf_Inference Service.

required: true

schema:

type: string

responses:

'200':

description: >

The Individual Inference Subscription resource was partial

modified successfully and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/InferEventSubsc'

'204':

description: >

The Individual Inference Subscription resource was partial

modified successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual Inference Subscription.

operationId: DeleteInferenceSubcription

tags:

- Individual Inference Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: >

String identifying a subscription to the Naf_Inference Service.

required: true

schema:

type: string

responses:

'204':

description: >

No Content. The Individual Inference Subscription matching the

subscriptionId was deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

naf-inference: Access to the Naf_Inference API

schemas:

InferEventSubsc:

description: Represents an Inference subscription.

type: object

properties:

notifCorreId:

type: string

description: >

String identifying the Notification Correlation ID in the corresponding notification.

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

suppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

inferAnaSubs:

type: object

additionalProperties:

$ref: '#/components/schemas/InferAnaSub'

minItems: 1

description: >

Represents inference subscription per analytics Id. The key of the map shall

be set to the value of the "anaEvent" attribute in the InferAnaSub data

structure.

inferResults:

type: array

items:

$ref: '#/components/schemas/InferResult'

minItems: 1

description: Represents Inference result.

reportInfo:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

required:

- notifUri

- notifCorreId

- inferAnaSubs

InferNotif:

description: Represents notifications on events that occurred.

type: object

properties:

notifCorreId:

type: string

description: >

String identifying the Notification Correlation ID in the corresponding notification.

inferResults:

type: array

items:

$ref: '#/components/schemas/InferResult'

minItems: 1

description: Represents inference results.

required:

- notifCorreId

- inferResults

InferEventSubscPatch:

description: Represents parameters to request the modification of an Inference subscription.

type: object

properties:

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorreId:

type: string

description: >

String identifying the Notification Correlation ID in the corresponding notification.

inferAnaSubs:

type: object

additionalProperties:

$ref: '#/components/schemas/InferAnaSub'

minItems: 1

description: >

Represents inference subscription per analytics Id. The key of the map shall

be set to the value of the "anaEvent" attribute in the InferAnaSub data

structure.

reportInfo:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

InferAnaSub:

description: Represents a subscription to a single event.

type: object

properties:

anaEvent:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NwdafEvent'

exterGroupIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExternalGroupId'

minItems: 1

gpsis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minItems: 1

intGroupIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

minItems: 1

supis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

eventFilter:

$ref: 'TS29520_Nnwdaf_AnalyticsInfo.yaml#/components/schemas/EventFilter'

anaMeta:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/AnalyticsMetadata'

minItems: 1

dataStatProps:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/DatasetStatisticalProperty'

minItems: 1

timeWindows:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

minItems: 1

resTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- anaEvent

oneOf:

- required: [exterGroupIds]

- required: [gpsis]

- required: [intGroupIds]

- required: [supis]

InferResult:

description: Represents Inference result per target UE.

type: object

properties:

inferRes:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/EventNotification'

vflNotOnTimeInd:

$ref: 'TS29520_Nnwdaf_MLModelTraining.yaml#/components/schemas/DelayEventNotif'

termCause:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflTermCause'

oneOf:

- required: [inferRes]

- required: [termCause]

- required: [vflNotOnTimeInd]

#

# ENUMERATIONS DATA TYPES

#

Annex B (informative):
Withdrawn API versions

B.1	General

This Annex lists withdrawn API versions of the APIs defined in the present specification. Clause 4.3.1.6 of 3GPP TS 29.501 [5] describes the withdrawal of API versions.

B.2	Naf_VFLTraining API

The API versions listed in table B.2-1 are withdrawn for the Naf_VFLTraining API.

Table B.2-1: Withdrawn API versions of the Naf_VFLTraining service

| API version number | Remarks |
| --- | --- |

B.3	Naf_VFLInference API

The API versions listed in table B.3-1 are withdrawn for the Naf_VFLInference API.

Table B.3-1: Withdrawn API versions of the Naf_VFLInference service

| API version number | Remarks |
| --- | --- |

B.4	Naf_Training API

The API versions listed in table B.4-1 are withdrawn for the Naf_Training API.

Table B.4-1: Withdrawn API versions of the Naf_Training service

| API version number | Remarks |
| --- | --- |

B.5	Naf_Inference API

The API versions listed in table B.5-1 are withdrawn for the Naf_Inference API.

Table B.5-1: Withdrawn API versions of the Naf_Inference service

| API version number | Remarks |
| --- | --- |

Annex C (normative):
ABNF grammar for 3GPP SBI HTTP custom headers

C.1	General

This Annex contains a self-contained set of ABNF rules, comprising the re-used rules from IETF RFCs, and the rules defined by the 3GPP custom headers defined in this specification (see clause 6.x.y.z).

This grammar may be used as input to existing tools to help implementations to parse 3GPP custom headers.

Annex D (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | CT3#142 | C3-253650 |  |  |  | Inclusion of agreed pCRs C3-253362, C3-253363, C3-253544, C3-253546, C3-253547, C3-253548, C3-253549, C3-253550, C3-253551, C3-253552, C3-253636, C3-253637, C3-253638 | 0.1.0 |
| 2025-09 | CT#109 | CP-252068 |  |  |  | Presentation to TSG CT for information | 1.0.0 |
| 2025-10 | CT#143 | C3-254474 |  |  |  | Inclusion of agreed pCRs C3-254367, C3-254127, C3-254556, C3-254384, C3-254369, C3-254269, C3-254386 | 1.1.0 |
| 2025-11 | CT#144 | C3-255656 |  |  |  | Inclusion of agreed pCRs C3-255159, C3-255358, C3-255404, C3-255405, C3-255504, C3-255507, C3-255508, C3-255526, C3-255527, C3-255602, C3-255635, C3-255636, C3-255647, C3-255654 | 1.2.0 |
| 2025-12 | CT#110 | CP-253020 |  |  |  | Presentation to TSG CT for approval | 2.0.0 |
| 2025-12 | CT#110 | CP-253020 |  |  |  | Approved by TSG CT | 19.0.0 |
| 2026-01 |  |  |  |  |  | Corrected yaml files coding: to be encoded as UTF-8 without BOM | 19.0.1 |
| 2026-03 | CT#111 | C3-260440 | 0001 | 1 | F | Correction on periodic reporting of ML model accuracy inforamtion | 19.1.0 |
| 2026-03 | CT#111 | C3-260338 | 0003 | 1 | F | Missing 403 Forbidden response code in Naf_Training API | 19.1.0 |
| 2026-03 | CT#111 | C3-260340 | 0004 | 1 | F | Missing 403 Forbidden response code in Naf_VFLInference API | 19.1.0 |
| 2026-03 | CT#111 | C3-260495 | 0006 | 3 | F | Corrections on the attribute name and HTTP status codes for Naf_Inference API | 19.1.0 |
| 2026-03 | CT#111 | C3-260341 | 0007 | 1 | F | Corrections on the resource name and consumer of Naf_VFLInference API | 19.1.0 |
| 2026-03 | CT#111 | C3-260342 | 0008 | 1 | F | Corrections on the status code and attribute presence of Naf_VFLTraining API | 19.1.0 |
| 2026-03 | CT#111 | C3-260339 | 0009 | 1 | F | Enhancements and corrections on the Naf_Training API | 19.1.0 |
| 2026-03 | CT#111 | C3-260389 | 0010 | 1 | F | VFL Inference Requirements corrections | 19.1.0 |
| 2026-03 | CT#111 | C3-260559 | 0011 | - | F | Update of info and externalDocs fields | 19.1.0 |