| 3GPP TS 29.566 V1.2.0 (2025-10) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Energy Information Function Services Stage 3 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions, symbols and abbreviations	8

3.1	Definitions	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Overview	8

5	Services offered by the EIF	9

5.1	Introduction	9

5.2	Neif_EventExposure Service	9

5.2.1	Service Description	9

5.2.2	Service Operations	9

5.2.2.1	Introduction	9

5.2.2.2	Neif_EventExposure_Subscribe	10

5.2.2.2.1	General	10

5.2.2.2.2	Energy Event Exposure Subscription Creation	10

5.2.2.2.3	Energy Event Exposure Subscription Update	10

5.2.2.3	Neif_EventExposure_Unsubscribe	11

5.2.2.3.1	General	11

5.2.2.3.2	Energy Event Exposure Subscription Deletion	11

5.2.2.4	Neif_EventExposure_Notify	12

5.2.2.4.1	General	12

5.2.2.4.2	Energy Event Exposure Notification	12

6	API Definitions	13

6.1	Neif_EventExposure Service API	13

6.1.1	Introduction	13

6.1.2	Usage of HTTP	13

6.1.2.1	General	13

6.1.2.2	HTTP standard headers	13

6.1.2.2.1	General	13

6.1.2.2.2	Content type	13

6.1.2.3	HTTP custom headers	14

6.1.3	Resources	14

6.1.3.1	Overview	14

6.1.3.2	Resource: Energy Event Exposure Subscriptions	15

6.1.3.2.1	Description	15

6.1.3.2.2	Resource Definition	15

6.1.3.2.3	Resource Standard Methods	15

6.1.3.2.4	Resource Custom Operations	17

6.1.3.3	Resource: Individual Energy Event Exposure Subscription	17

6.1.3.3.1	Description	17

6.1.3.3.2	Resource Definition	17

6.1.3.3.3	Resource Standard Methods	17

6.1.3.3.4	Resource Custom Operations	22

6.1.4	Custom Operations without associated resources	22

6.1.5	Notifications	22

6.1.5.1	General	22

6.1.5.2	Energy Event Exposure Notification	23

6.1.5.2.1	Description	23

6.1.5.2.2	Target URI	23

6.1.5.2.3	Standard Methods	23

6.1.6	Data Model	24

6.1.6.1	General	24

6.1.6.2	Structured data types	25

6.1.6.2.1	Introduction	25

6.1.6.2.2	Type: EnergyEeSubsc	25

6.1.6.2.3	Type: EnergyEeSubscPatch	26

6.1.6.2.4	Type: EnergyEeNotif	26

6.1.6.2.5	Type: EnergyEeSubscSet	27

6.1.6.2.6	Type: EnergyEeReport	28

6.1.6.3	Simple data types and enumerations	28

6.1.6.3.1	Introduction	28

6.1.6.3.2	Simple data types	28

6.1.6.3.3	Enumeration: EnergyEeEvent	28

6.1.6.4	Data types describing alternative data types or combinations of data types	28

6.1.6.5	Binary data	29

6.1.6.5.1	Binary Data Types	29

6.1.7	Error Handling	29

6.1.7.1	General	29

6.1.7.2	Protocol Errors	29

6.1.7.3	Application Errors	29

6.1.8	Feature negotiation	29

6.1.9	Security	29

6.1.10	HTTP redirection	30

Annex A (normative): OpenAPI specification	31

A.1	General	31

A.2	Neif_EventExposure API	31

Annex B (informative): Withdrawn API versions	38

B.1	General	38

B.2	Neif_EventExposure API	38

Annex C (normative): ABNF grammar for 3GPP SBI HTTP custom headers	39

C.1	General	39

Annex <D> (informative): Change history	39

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the Neif Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the EIF.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 23.503 [19].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 9113: "HTTP/2".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 9457: "Problem Details for HTTP APIs".

[14]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[15]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[16]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[17]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[18]	3GPP TS 29.523: "5G System; Policy Control Event Exposure Service; Stage 3".

[19]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

EIF	Energy Information Function

# 4	Overview

In the frame of Energy Efficiency and Energy Saving functionalities and services, the Energy Information Function (EIF) provides services to NF service consumers (e.g., NEF, AF) via the Neif service-based interface. The EIF supports for this purpose the functionalities defined in 3GPP TS 23.502 [3] to manage Energy information and its exposure.

Figures°4-1 and 4.2 depict the Energy related reference architecture of the EIF respectively in SBI representation and reference point representation.

Figure 4-1: Reference model for the EIF Services – SBI representation

Figure 4-2: Reference Model for the EIF Services – Reference point representation

# 5	Services offered by the EIF

## 5.1	Introduction

The EIF provides the following services:

-	Neif_EventExposure

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Neif_EventExposure | 6.1 | EIF Event Exposure Service | TS29566_Neif_EventExposure.yaml | neif-ee | A.2 |

## 5.2	Neif_EventExposure Service

### 5.2.1	Service Description

The Neif_EventExposure service exposed by the EIF enables an NF service consumer to:

-	create/update/delete an Energy Event Exposure Subscription at the EIF; and

-	receive energy consumption information related event(s) reporting.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The service operations defined for the Neif_EventExposure Service are shown in table 5.2.2.1-1.

Table 5.2.2.1-1: Neif_EventExposure Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Neif_EventExposure_Subscribe | This service operation is used by an NF service consumer to create/update an Energy Event Exposure Subscription. | e.g., NEF, AF |
| Neif_EventExposure_Unsubscribe | This service operation is used by an NF service consumer to delete an existing Energy Event Exposure Subscription. | e.g. NEF, AF |
| Neif_EventExposure_Notify | This service operation is used by the EIF to report energy consumption information related event(s). | EIF |

#### 5.2.2.2	Neif_EventExposure_Subscribe

##### 5.2.2.2.1	General

This service operation is used by an NF service consumer to create/update an Energy Event Exposure Subscription.

The following procedures  are supported by the "Neif_EventExposure_Subscribe" service operation:

-	Energy Event Exposure Subscription Creation.

-	Energy Event Exposure Subscription Update.

##### 5.2.2.2.2	Energy Event Exposure Subscription Creation

Figure 5.2.2.2.2-1 depicts a scenario where an NF service consumer requests to create an Energy Event Exposure Subscription (see also clause 4.29 of 3GPP TS 23.502 [3]).

Figure 5.2.2.2.2-1: Energy Event Exposure Subscription Creation

1.	To subscribe to energy consumption related event(s) reporting, the NF service consumer shall send an HTTP POST request targeting the URI of the "Energy Event Exposure Subscriptions" collection resource, with the request body including the EnergyEeSubsc data structure.

2a.	Upon success, the EIF shall respond to the NF service consumer with an HTTP "201 Created" status code with the response body containing the representation of the created resource within the EnergyEeSubsc data structure, and an HTTP "Location" header field containing the URI of the created "Individual Energy Event Exposure Subscription" resource.

2b.	On failure, the EIF shall take proper error handling actions, as specified in clause 6.1.7, and respond to the NF service consumer with an appropriate error status code.

##### 5.2.2.2.3	Energy Event Exposure Subscription Update

Figure 5.2.2.2.3-1 depicts a scenario where an NF service consumer requests to update an existing Energy Event Exposure Subscription (see also clause 4.29 of 3GPP TS 23.502 [3]).

Figure 5.2.2.2.3-1: Energy Event Exposure Subscription Update

1.	In order to request the update of an existing subscription to energy consumption related event(s) reporting, the NF service consumer shall send an HTTP PUT/PATCH request to the EIF, targeting the URI of the corresponding "Individual Energy Event Exposure Subscription" resource, with the request body including either:

-	the updated representation of the resource within the EnergyEeSubsc data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the EnergyEeSubscPatch data structure, in case the HTTP PATCH method is used.

2a.	Upon success, the EIF shall update the targeted "Individual Energy Event Exposure Subscription" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Energy Event Exposure Subscription" resource within the EnergyEeSubsc data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the EIF shall take proper error handling actions, as specified in clause 6.1.7, and respond to the NF service consumer with an appropriate error status code.

#### 5.2.2.3	Neif_EventExposure_Unsubscribe

##### 5.2.2.3.1	General

This service operation is used by an NF service consumer to delete an existing Energy Event Exposure Subscription.

The following procedures are supported by the "Neif_EventExposure_UnSubscribe" service operation:

-	Energy Event Exposure Subscription Deletion.

##### 5.2.2.3.2	Energy Event Exposure Subscription Deletion

Figure 5.2.2.3.2-1 depicts a scenario where an NF service consumer requests to delete an existing Energy Event Exposure Subscription (see also clause 4.29 of 3GPP TS 23.502 [3]).

Figure 5.2.2.3.2-1: Energy Event Exposure Subscription Deletion

1.	To unsubscribe from energy consumption related event(s) reporting, the NF service consumer shall send an HTTP DELETE request to the EIF targeting the URI of the corresponding "Individual Energy Event Exposure Subscription" resource.

2a.	Upon success, the EIF shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the EIF shall take proper error handling actions, as specified in clause 6.1.7, and respond to the NF service consumer with an appropriate error status code.

#### 5.2.2.4	Neif_EventExposure_Notify

##### 5.2.2.4.1	General

This service operation is used by the EIF to report energy consumption information related event(s).

The following procedures are supported by the "Neif_EventExposure_Notify" service operation:

-	Energy Event Exposure Notification.

##### 5.2.2.4.2	Energy Event Exposure Notification

Figure 5.2.2.4.2-1 depicts a scenario where the EIF sends a request to notify a previously subscribed NF service consumer on energy consumption information related event(s) (see also clause 4.29 of 3GPP TS 23.502 [3]).

Figure 5.2.2.4.2-1: Energy Event Exposure Notification

1.	In order to notify a previously subscribed NF service consumer on energy consumption information related event(s), the EIF shall send an HTTP POST request to the NF service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the NF service consumer during the creation/update of the corresponding Energy Event Exposure Subscription using the procedures defined in clause 5.2.2.2, and the request body including the EnergyEeNotif data structure.

2a.	Upon success, the NF service consumer shall respond to the EIF with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the NF service consumer shall take proper error handling actions, as specified in clause 6.1.7, and respond to the EIF with an appropriate error status code.

# 6	API Definitions

## 6.1	Neif_EventExposure Service API

### 6.1.1	Introduction

The Neif_EventExposure Service shall use the Neif_EventExposure API.

The API URI of the Neif_EventExposure API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "neif-ee".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clauses 6.1.3 and 6.1.4.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, IETF RFC 9113 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Neif_EventExposure API is contained in Annex A.

#### 6.1.2.2	HTTP standard headers

##### 6.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

##### 6.1.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 9457 [13].

#### 6.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

In this Release of the specification, no specific custom headers are defined for the Neif_EventExposure API.

### 6.1.3	Resources

#### 6.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.3.1-1 depicts the resource URIs structure for the Neif_EventExposure API.

Figure 6.1.3.1-1: Resource URI structure of the Neif_EventExposure API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods for the Neif_EventExposure API.

.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Energy Event Exposure Subscriptions | /subscriptions | POST | Create a new Energy Event Exposure Subscription. |
|  |  | GET | Retrieve all the Energy Event Exposure Subscription(s) managed by the EIF. |
| Individual Energy Event Exposure Subscription | /subscriptions/{subId} | GET | Retrieve an existing "Individual Energy Event Exposure Subscription" resource. |
|  |  | PUT | Update an existing "Individual Energy Event Exposure Subscription" resource. |
|  |  | PATCH | Modify an existing "Individual Energy Event Exposure Subscription" resource. |
|  |  | DELETE | Delete an existing "Individual Energy Event Exposure Subscription" resource. |

#### 6.1.3.2	Resource: Energy Event Exposure Subscriptions

##### 6.1.3.2.1	Description

The "Energy Event Exposure Subscriptions" resource represents the collection of Energy Event Exposure Subscriptions managed by the EIF.

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/neif-ee/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |

##### 6.1.3.2.3	Resource Standard Methods

###### 6.1.3.2.3.1	POST

The HTTP POST method enables an NF service consumer to request the creation of an Energy Event Exposure Subscription at the EIF.

This method shall support the URI query parameters specified in table 6.1.3.2.3.2-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| <n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.2-2 and the response data structures and response codes specified in table 6.1.3.2.3.2-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EnergyEeSubsc | M | 1 | Represents the parameters to request the creation of a new Energy Event Exposure Subscription. |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EnergyEeSubsc | M | 1 | 201 Created | Successful case. The Energy Event Exposure Subscription is successfully created and a representation of the created "Individual Energy Event Exposure Subscription" resource shall be returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/neif-ee/<apiVersion>/subscriptions/{subId} |

###### 6.1.3.2.3.2	GET

The HTTP GET method enables an NF service consumer to retrieve all the Energy Event Exposure Subscription(s) managed by the EIF.

This method shall support the URI query parameters specified in table 6.1.3.2.3.2-1.

Table 6.1.3.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.2-2 and the response data structures and response codes specified in table 6.1.3.2.3.2-3.

Table 6.1.3.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(EnergyEeSubsc) | M | 0..N | 200 OK | Successful case. All the active "Individual Energy Event Exposure Subscription" resource(s) managed by the EIF are returned. If there are no active "Individual Energy Event Exposure Subscription" resource(s) at the EIF, an empty array is returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target EIF (service) instance towards which the request is redirected. |

Table 6.1.3.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target EIF (service) instance towards which the request is redirected. |

##### 6.1.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.1.3.3	Resource: Individual Energy Event Exposure Subscription

##### 6.1.3.3.1	Description

The "Individual Energy Event Exposure Subscription" resource represents an Energy Event Exposure Subscription managed by the EIF.

##### 6.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/neif-ee/<apiVersion>/subscriptions{subId}

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |
| subId | string | Represents the identifier of the "Individual Energy Event Exposure Subscription" resource. |

##### 6.1.3.3.3	Resource Standard Methods

6.1.3.3.3.1	GET

The HTTP GET method enables an NF service consumer to retrieve an existing "Individual Energy Event Exposure Subscription" resource at the EIF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |

Table 6.1.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EnergyEeSubsc | M | 1 | 200 OK | Successful case. The requested "Individual Energy Event Exposure Subscription" resource is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target EIF (service) instance towards which the request is redirected |

Table 6.1.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target EIF (service) instance towards which the request is redirected |

6.1.3.3.3.2	PUT

The HTTP PUT method enables an NF service consumer to request the update of an existing "Individual Energy Event Exposure Subscription" resource at the EIF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.2-1.

Table 6.1.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.3.3.3.2-3.

Table 6.1.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EnergyEeSubsc | M | 1 | Represents the updated "Individual Energy Event Exposure Subscription" resource representation. |

Table 6.1.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EnergyEeSubsc | M | 1 | 200 OK | Successful case. The "Individual Energy Event Exposure Subscription" resource is successfully updated and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Energy Event Exposure Subscription" resource is successfully updated and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target EIF (service) instance towards which the request is redirected. |

Table 6.1.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target EIF (service) instance towards which the request is redirected. |

6.1.3.3.3.3	PATCH

The HTTP PUT method enables an NF service consumer to request the modification of an existing "Individual Energy Event Exposure Subscription" resource at the EIF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.3-1.

Table 6.1.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.3-2 and the response data structures and response codes specified in table 6.1.3.3.3.3-3.

Table 6.1.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EnergyEeSubscPatch | M | 1 | Contains the parameters to request the modification of the "Individual Energy Event Exposure Subscription" resource. |

Table 6.1.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EnergyEeSubsc | M | 1 | 200 OK | Successful case. The "Individual Energy Event Exposure Subscription" resource is successfully modified and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Energy Event Exposure Subscription" resource is successfully modified and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target EIF (service) instance towards which the request is redirected. |

Table 6.1.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target EIF (service) instance towards which the request is redirected. |

6.1.3.3.3.4	DELETE

The HTTP PUT method enables an NF service consumer to request the deletion of an existing "Individual Energy Event Exposure Subscription" resource at the EIF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.4-1.

Table 6.1.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.4-2 and the response data structures and response codes specified in table 6.1.3.3.3.4-3.

Table 6.1.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual Energy Event Exposure Subscription" resource is successfully deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target EIF (service) instance towards which the request is redirected. |

Table 6.1.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative EIF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target EIF (service) instance towards which the request is redirected. |

##### 6.1.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 6.1.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 6.1.5	Notifications

#### 6.1.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

Table 6.1.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Energy Event Exposure Notification | {notifUri} | POST | Enables the EIF to notify a previously subscribed NF service consumer on energy consumption information related event(s). |

#### 6.1.5.2	Energy Event Exposure Notification

##### 6.1.5.2.1	Description

The Energy Event Exposure Notification is used by the EIF to notify a previously subscribed NF service consumer on energy consumption information related event(s).

##### 6.1.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.1.5.2.2-1.

Table 6.1.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Represents the callback URI encoded as a string formatted as a URI. |

##### 6.1.5.2.3	Standard Methods

6.1.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.5.2.3.1-1 and the response data structures and response codes specified in table 6.1.5.2.3.1-2.

Table 6.1.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EnergyEeNotif | M | 1 | Represents the Energy Event Exposure Notification. |

Table 6.1.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Energy Event Exposure Notification is successfully received and acknowledged. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative NF service consumer (service) instance towards which the notification should be redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF service consumer (service) instance towards which the notification request is redirected. |

Table 6.1.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative NF service consumer (service) instance towards which the notification should be redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF service consumer (service) instance towards which the notification request is redirected. |

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Neif_EventExposure service-based interface protocol.

Table 6.1.6.1-1: Neif_EventExposure specific Data Type

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| EnergyEeEvent | 6.1.6.3.3 | Represents the Energy related event. |  |
| EnergyEeNotif | 6.1.6.2.4 | Represents the Energy Event Exposure Notification. |  |
| EnergyEeReport | 6.1.6.2.6 | Represents the Energy Event Exposure report. |  |
| EnergyEeSubsc | 6.1.6.2.2 | Represents the Energy Event Exposure Subscription. |  |
| EnergyEeSubscPatch | 6.1.6.2.3 | Represents the requested modifications to an Energy Event Exposure Subscription. |  |
| EnergyEeSubscSet | 6.1.6.2.5 | Represents the Energy Event Exposure Subscription set. |  |

Table 6.1.6.1-2 specifies data types re-used by the Neif_EventExposure service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Neif_EventExposure service-based interface.

Table 6.1.6.1-2: Neif_EventExposure re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ApplicationId | 3GPP TS 29.571 [14] | Represents the application identifier. |  |
| DateTime | 3GPP TS 29.571 [14] | Represents a date and a time. |  |
| Dnn | 3GPP TS 29.571 [14] | Represents a DNN. |  |
| DurationSec | 3GPP TS 29.571 [14] | Represents a time duration in units of seconds. |  |
| EnergyInfo | 3GPP TS 29.122 [17] | Represents the Energy consumption information. |  |
| FlowDescription | 3GPP TS 29.514 [16] | Represents an IP flow description. |  |
| Gpsi | 3GPP TS 29.571 [14] | Represents a GPSI. |  |
| RedirectResponse | 3GPP TS 29.571 [14] | Represents redirection related information. |  |
| ReportingInformation | 3GPP TS 29.523 [18] | Represents the reporting requirements. |  |
| Snssai | 3GPP TS 29.571 [14] | Represents an S-NSSAI. |  |
| Supi | 3GPP TS 29.571 [14] | Represents a SUPI. |  |
| SupportedFeatures | 3GPP TS 29.571 [14] | Represents the list of supported feature(s) and is used to negotiate the applicability of optional features. |  |
| TimeWindow | 3GPP TS 29.122 [17] | Represents a time window. |  |
| Uinteger | 3GPP TS 29.571 [14] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.571 [14] | Represents a URI. |  |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: EnergyEeSubsc

Table 6.1.6.2.2-1: Definition of type EnergyEeSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | M | 1 | Contains the URI via which Energy consumption information related event(s) reporting shall be delivered. |  |
| repReqs | ReportingInformation | O | 0..1 | Contains the reporting requirements of the subscription. |  |
| eventsSubscSets | map(EnergyEeSubscSet) | M | 1..N | Contains the set(s) of Energy Event Exposure subscription related details. The key of the map shall be set to the value of the "event" attribute of the corresponding map value encoded using the EnergyEeSubscSet data structure. |  |
| reports | array(EnergyEeReport) | C | 1..N | Contains the Energy Event Exposure report(s). This attribute shall be present only when immediate reporting was requested in the corresponding subscription request and there are available report(s). |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features used as defined in clause 6.1.8. This attribute shall be present only when feature negotiation is required. |  |

##### 6.1.6.2.3	Type: EnergyEeSubscPatch

Table 6.1.6.2.3-1: Definition of type EnergyEeSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | O | 0..1 | Contains the updated URI via which Energy consumption information related event(s) reporting shall be delivered. |  |
| repReqs | ReportingInformation | O | 0..1 | Contains the reporting requirements of the subscription. |  |
| eventsSubscSets | maps(EnergyEeSubscSet) | O | 1..N | Contains the set(s) of Energy Event Exposure subscription related details. The key of the map shall be set to the value of the "event" attribute of the corresponding map value encoded using the EnergyEeSubscSet data structure. |  |

##### 6.1.6.2.4	Type: EnergyEeNotif

Table 6.1.6.2.4-1: Definition of type EnergyEeNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subId | string | M | 1 | Contains the identifier of the Energy Event Exposure Subscription to which the notification is related. |  |
| reports | array(EnergyEeReport) | M | 1..N | Contains the Energy Event Exposure report(s). |  |

##### 6.1.6.2.5	Type: EnergyEeSubscSet

Table 6.1.6.2.5-1: Definition of type  EnergyEeSubscSet

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | EnergyEeEvent | M | 1 | Contains the subscribed Energy related event. |  |
| subscSetId | string | M | 1 | Contains the identifier of the Energy event exposure subscription set. |  |
| supi | Supi | C | 0..1 | Contains the target SUPI. (NOTE 1) |  |
| gpsi | Gpsi | C | 0..1 | Contains the target GPSI. (NOTE 1) |  |
| dnn | Dnn | C | 0..1 | Contains the target DNN. (NOTE 2) |  |
| snssai | Snssai | C | 0..1 | Contains the target S-NSSAI. (NOTE 2, NOTE 4) |  |
| appId | ApplicationId | C | 0..1 | Contains the identifier of the target application. (NOTE 3) |  |
| flowDescs | array(FlowDescription) | C | 1..N | Contains the service flow information description for a target application. (NOTE 3) |  |
| repTimePeriod | TimeWindow | O | 0..1 | Contains the reporting time period. (NOTE 6) |  |
| enrgRepThres | EnergyInfo | O | 0..1 | Contains the reporting thresholds for the Energy consumption information event exposure. This attribute shall be present in case threshold-based reporting is requested. |  |
| repReqs | ReportingInformation | O | 0..1 | Contains the reporting requirements of the subscription set. The "repPeriod" within the ReportingInformation data structure shall be present in case periodic reporting is requested. (NOTE 5) |  |
| NOTE 1: These attributes are mutually exclusive and only one of them shall be present. NOTE 2: If the value of the "event" attribute is set to "PDU_SESSION_ENERGY" or "SERVICE_FLOW_ENERGY", then at least one of these attributes shall be present. NOTE 3: If the value of the "event" attribute is set to "SERVICE_FLOW_ENERGY", then only one of these attributes shall be present. NOTE 4: if the value of the "event" attribute is set to "UE_SNSSAI_ENERGY", then this attribute shall be present. NOTE 5: When this attribute is present, the event reporting requirements provided within this attribute shall take precedence over the common events reporting requirements provided within the "repReqs" attribute of the parent EnergyEeSubsc data structure. NOTE 6: In this Release of the specification, the values of both the start time and end time within the "repTimePeriod" attribute shall be in the future. |  |  |  |  |  |

##### 6.1.6.2.6	Type: EnergyEeReport

Table 6.1.6.2.6-1: Definition of type EnergyEeReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | EnergyEeEvent | M | 1 | Contains the reported Energy related event. |  |
| subscSetId | string | M | 1 | Contains the identifier of the Energy event exposure subscription set to which this report is related. |  |
| timeStamp | DateTime | M | 1 | Contains the time at which the event is observed. |  |
| timeWindow | TimeWindow | O | 0..1 | Contains the time window representing a start time and a stop time of the reported data collection period. |  |
| energyInfo | EnergyInfo | O | 0..1 | Contains the energy consumption information. |  |

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 6.1.6.3.3	Enumeration: EnergyEeEvent

The enumeration EnergyEeEvent represents the Energy event. It shall comply with the provisions defined in table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration EnergyEeEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UE_ENERGY | Indicates that the Energy Exposure event is the energy consumed by a UE. |  |
| PDU_SESSION_ENERGY | Indicates that the Energy Exposure event is the energy consumed by a PDU session of a UE. |  |
| SERVICE_FLOW_ENERGY | Indicates that the Energy Exposure event is the energy consumed by an application or service data flow of an application traffic of a UE. |  |
| UE_SNSSAI_ENERGY | Indicates that the Energy Exposure event is the energy consumed by a UE in an S-NSSAI. |  |

#### 6.1.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 6.1.6.5	Binary data

##### 6.1.6.5.1	Binary Data Types

Table 6.1.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 6.1.7	Error Handling

#### 6.1.7.1	General

For the Neif_EventExposure API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Neif_EventExposure API.

#### 6.1.7.2	Protocol Errors

No specific procedures for the Neif_EventExposure service are specified.

#### 6.1.7.3	Application Errors

The application errors defined for the Neif_EventExposure service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

### 6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the Neif_EventExposure API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Neif_EventExposure API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Neif_EventExposure API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in clause 5.4.2.2 of 3GPP TS 29.510 [10].

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Neif_EventExposure service.

The Neif_EventExposure API defines a single scope "neif-ee" for the entire service, and it does not define any additional scopes at resource or operation level.

### 6.1.10	HTTP redirection

An HTTP request may be redirected to a different Neif_EventExposure service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different EIF producer instance will return the NF Instance ID of the new EIF producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an EIF redirects a service request to a different EIF using an HTTP "307 Temporary Redirect" or "308 Permanent Redirect"’ status code, the identity of the new EIF towards which the service request is redirected shall be indicated in the "3gpp-Sbi-Target-Nf-Id"’ header of the HTTP "307 Temporary Redirect" or "308 Permanent Redirect" response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

Annex A (normative):
OpenAPI specification

# A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5.3.1 of 3GPP TS 29.501 [5] and clause 5B of 3GPP TR 21.900 [7]).

# A.2	Neif_EventExposure API

openapi: 3.0.0

info:

title: EIF Event Exposure Service

version: 1.0.0-alpha.1

description: |

API for the Neif_EventExposure Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.566 V1.2.0; 5G System; Energy Information Function Services; Stage 3.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.566/

servers:

- url: '{apiRoot}/neif-ee/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- neif-ee

paths:

/subscriptions:

get:

summary: Retrieve all the active Energy Event Exposure Subscription(s) managed by the EIF.

operationId: GetEnergyEESubscs

tags:

- Energy Event Exposure Subscriptions (Collection)

responses:

'200':

description: >

OK. All the active Individual Energy Event Exposure Subscription resource(s) managed by

the EIF are returned.

If there are no active Individual Energy Event Exposure Subscription resource(s) at the

EIF, an empty array is returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/EnergyEeSubsc'

minItems: 0

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

post:

summary: Request the creation of an Energy Event Exposure Subscription.

operationId: CreateEnergyEESubsc

tags:

- Energy Event Exposure Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EnergyEeSubsc'

responses:

'201':

description: >

Created. The Energy Event Exposure Subscription is successfully created and a

representation of the created Individual Energy Event Exposure Subscription resource

shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/EnergyEeSubsc'

headers:

Location:

description: >

Contains the URI of the created Individual Energy Event Exposure Subscription

resource.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

EnergyEENotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EnergyEeNotif'

responses:

'204':

description: >

No Content. The Energy Event Exposure notification is successfully

received and acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subId}:

parameters:

- name: subId

in: path

description: >

Represents the identifier of the Individual Energy Event Exposure Subscription

resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Energy Event Exposure Subscription resource.

operationId: GetIndEnergyEESubsc

tags:

- Individual Energy Event Exposure Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual Energy Event Exposure Subscription resource

shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/EnergyEeSubsc'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Energy Event Exposure Subscription resource.

operationId: UpdateIndEnergyEESubsc

tags:

- Individual Energy Event Exposure Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EnergyEeSubsc'

responses:

'200':

description: >

OK. The Individual Energy Event Exposure Subscription resource is successfully updated

and a representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/EnergyEeSubsc'

'204':

description: >

No Content. The Individual Energy Event Exposure Subscription resource is successfully

updated and no content is returned in the response body.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Energy Event Exposure Subscription resource.

operationId: ModifyIndEnergyEESubsc

tags:

- Individual Energy Event Exposure Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/EnergyEeSubscPatch'

responses:

'200':

description: >

OK. The Individual Energy Event Exposure Subscription resource is successfully modified

and a representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/EnergyEeSubsc'

'204':

description: >

No Content. The Individual Energy Event Exposure Subscription resource is successfully

modified and no content is returned in the response body.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Energy Event Exposure Subscription resource.

operationId: DeleteIndEnergyEESubsc

tags:

- Individual Energy Event Exposure Subscription (Document)

responses:

'204':

description: >

No Content. The Individual Energy Event Exposure Subscription resource is

successfully deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

neif-ee: >

Enables to access all the resources and custom operations of the Neif_EventExposure

API.

schemas:

#

# STRUCTURED DATA TYPES

#

EnergyEeSubsc:

description: >

Represents an Energy Event Exposure Subscription.

type: object

properties:

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

repReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

eventsSubscSets:

type: object

additionalProperties:

$ref: '#/components/schemas/EnergyEeSubscSet'

minProperties: 1

description: >

Contains the set(s) of Energy Event Exposure subscription related details.

The key of the map shall be set to the value of the event attribute of the

corresponding map value encoded using the EnergyEeSubscSet data structure.

reports:

type: array

items:

$ref: '#/components/schemas/EnergyEeReport'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notifUri

- eventsSubscSets

EnergyEeSubscPatch:

description: >

Represents the requested modifications to an Energy Event Exposure Subscription.

type: object

properties:

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

repReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

eventsSubscSets:

type: object

additionalProperties:

$ref: '#/components/schemas/EnergyEeSubscSet'

minProperties: 1

description: >

Contains the set(s) of Energy Event Exposure subscription related details.

The key of the map shall be set to the value of the event attribute of the

corresponding map value encoded using the EnergyEeSubscSet data structure.

EnergyEeNotif:

description: >

Represents the Energy Event Exposure Notification.

type: object

properties:

subId:

type: string

reports:

type: array

items:

$ref: '#/components/schemas/EnergyEeReport'

minItems: 1

required:

- subId

- reports

EnergyEeSubscSet:

description: >

Represents the Energy Event Exposure Subscription set.

type: object

properties:

event:

$ref: '#/components/schemas/EnergyEeEvent'

subscSetId:

type: string

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

appId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

flowDescs:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/FlowDescription'

minItems: 1

repTimePeriod:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

enrgRepThres:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/EnergyInfo'

repReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

required:

- event

- subscSetId

oneOf:

- required: [supi]

- required: [gpsi]

not:

required: [appId, flowDescs]

EnergyEeReport:

description: >

Represents the Energy Event Exposure report.

type: object

properties:

event:

$ref: '#/components/schemas/EnergyEeEvent'

subscSetId:

type: string

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

timeWindow:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

energyInfo:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/EnergyInfo'

required:

- event

- subscSetId

- timeStamp

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

EnergyEeEvent:

anyOf:

- type: string

enum:

- UE_ENERGY

- PDU_SESSION_ENERGY

- SERVICE_FLOW_ENERGY

- UE_SNSSAI_ENERGY

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the Energy event.

Possible values are:

- UE_ENERGY: Indicates that the Energy Exposure event is the energy consumed by a UE.

- PDU_SESSION_ENERGY: Indicates that the Energy Exposure event is the energy consumed

by a PDU session of a UE.

- SERVICE_FLOW_ENERGY: Indicates that the Energy Exposure event is the energy consumed

by an application or service data flow of an application traffic of a UE.

- UE_SNSSAI_ENERGY: Indicates that the Energy Exposure event is the energy consumed by

a UE in an S-NSSAI.

# Data types describing alternative data types or combinations of data types:

#

Annex B (informative):
Withdrawn API versions

# B.1	General

This Annex lists withdrawn API versions of the APIs defined in the present specification. Clause 4.3.1.6 of 3GPP TS 29.501 [5] clause 4.3.1.6 describes the withdrawal of API versions.

# B.2	Neif_EventExposure API

The API versions listed in table B.2-1 are withdrawn for the Neif_EventExposure API.

Table B.2-1: Withdrawn API versions of the Neif_EventExposure service

| API version number | Remarks |
| --- | --- |

Annex C (normative):
ABNF grammar for 3GPP SBI HTTP custom headers

# C.1	General

This Annex contains a self-contained set of ABNF rules, comprising the re-used rules from IETF RFCs, and the rules defined by the 3GPP custom headers defined in this specification (see clause 6.1.2.3).

This grammar may be used as input to existing tools to help implementations to parse 3GPP custom headers.

Annex <D> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-04 | CT3#140 | C3-251540 |  |  |  | Skeleton | 0.0.0 |
| 2025-04 | CT3#140 | C3-251622 |  |  |  | Cleanup of specification to be agreed by CT3 | 0.1.0 |
| 2025-05 | CT3#141 | C3-252389 |  |  |  | Agreed version at CT3#141, implementing: C3-252097, C3-252469, C3-252470, C3-252471, C3-252472 | 0.2.0 |
| 2025-06 | CT#108 | CP-251139 |  |  |  | Presentation to TSG CT for information. | 1.0.0 |
| 2025-08 | CT3#142 | C3-253653 |  |  |  | Agreed version at CT3#142, implementing: C3-253339, C3-253340, C3-253341, C3-253690 | 1.1.0 |
| 2025-10 | CT3#143 | C3-254475 |  |  |  | Agreed version at CT3#143, implementing: C3-254424, C3-254425, C3-254426 | 1.2.0 |