| 3GPP TS 29.562 V19.4.0 (2025-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Home Subscriber Server (HSS) Services; Stage 3 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	14

1	Scope	15

2	References	15

3	Definitions of terms, symbols and abbreviations	17

3.1	Terms	17

3.2	Symbols	18

3.3	Abbreviations	18

4	Overview	18

4.1	Introduction	18

5	Services offered by the HSS	19

5.1	Introduction	19

5.2	Nhss_imsUEContextManagement Service	20

5.2.1	Service Description	20

5.2.2	Service Operations	20

5.2.2.1	Introduction	20

5.2.2.2	Registration	21

5.2.2.2.1	General	21

5.2.2.2.2	S-CSCF Registration	21

5.2.2.2.3	IMS AS Registration	22

5.2.2.3	DeregistrationNotification	23

5.2.2.3.1	General	23

5.2.2.3.2	HSS initiated S-CSCF Deregistration	23

5.2.2.3.3	HSS initiated IMS AS Deregistration	24

5.2.2.4	Deregistration	24

5.2.2.4.1	General	24

5.2.2.4.2	S-CSCF deregistration	25

5.2.2.4.3	IMS AS deregistration	25

5.2.2.5	Authorize	26

5.2.2.5.1	General	26

5.2.2.5.2	Authorization request	26

5.2.2.6	RestorationInfoGet	27

5.2.2.6.1	General	27

5.2.2.6.2	Restoration Information Retrieval	27

5.2.2.7	RestorationInfoUpdate	27

5.2.2.7.1	General	27

5.2.2.7.2	Restoration Information Update	28

5.2.2.8	RestorationInfoDelete	28

5.2.2.8.1	General	28

5.2.2.8.2	Restoration Information Delete	29

5.2.2.9	AsInfoGet	29

5.2.2.9.1	General	29

5.2.2.9.2	IMS AS Information Retrieval	29

5.2.2.10	IndividualImsasRegistrationInfoGet	30

5.2.2.10.1	General	30

5.2.2.10.2	Individual IMS AS Registration Information Retrieval	30

5.2.2.11	AsReassignmentNotification	31

5.2.2.11.1	General	31

5.2.2.11.2	HSS initiated IMS AS Reassignment Notification	31

5.3	Nhss_imsSubscriberDataManagement Service	31

5.3.1	Service Description	31

5.3.2	Service Operations	31

5.3.2.1	Introduction	31

5.3.2.2	Get	32

5.3.2.2.1	General	32

5.3.2.2.2	Identity Data	33

5.3.2.2.3	IMS Location Data and Status	35

5.3.2.2.4	IMS Profile Data	38

5.3.2.2.5	Access Data	41

5.3.2.2.6	SRVCC Data	45

5.3.2.2.7	Service Data	45

5.3.2.2.8	Repository Data	47

5.3.2.2.9	Shared Subscription Data	48

5.3.2.3	Subscribe	49

5.3.2.3.1	General	49

5.3.2.3.2	Subscription to notifications of data change	49

5.3.2.3.3	Subscription to notifications of shared data change	50

5.3.2.3.4	Subscriptions to notification of UE Reachability for IP	50

5.3.2.4	Unsubscribe	51

5.3.2.4.1	General	51

5.3.2.4.2	Unsubscribe to notifications of data change	51

5.3.2.4.3	Unsubscribe to notifications of shared data change	52

5.3.2.4.4	Unsubscribe to notification of UE Reachability for IP	52

5.3.2.5	ModifySubscription	53

5.3.2.5.1	General	53

5.3.2.5.2	Modification of a subscription to notifications of data change	53

5.3.2.5.3	Modification of a subscription to notifications of shared data change	54

5.3.2.5.4	Modification of a subscription to notifications of data change	54

5.3.2.6	Notification	55

5.3.2.6.1	General	55

5.3.2.6.2	Data Change Notification To NF	55

5.3.2.6.3	UE Reachability for IP Notification to NF	55

5.3.2.7	Update	56

5.3.2.7.1	General	56

5.3.2.7.2	Repository Data Creation or Update	56

5.3.2.7.3	Repository Data Deletion	57

5.3.2.7.4	STN-SR update	58

5.3.2.7.5	PSI activation state update	58

5.3.2.7.6	SMS Registration Information Creation or Update	59

5.3.2.7.7	SMS Registration Information Deletion	59

5.3.2.7.8	DSAI tag activation state update	60

5.4	Nhss_imsUEAuthentication Service	61

5.4.1	Service Description	61

5.4.2	Service Operations	61

5.4.2.1	Introduction	61

5.4.2.2	Get	61

5.4.2.2.1	General	61

5.4.2.2.2	Authentication Information Retrieval	61

5.5	Nhss_gbaSubscriberDataManagement Service	62

5.5.1	Service Description	62

5.5.2	Service Operations	62

5.5.2.1	Introduction	62

5.5.2.2	Get	62

5.5.2.2.1	General	62

5.5.2.2.2	Retrieval of GBA subscriber data	62

5.5.2.3	Subscribe	63

5.5.2.3.1	General	63

5.5.2.3.2	Subscription to changes on the GBA subscriber data	63

5.5.2.4	Unsubscribe	64

5.5.2.4.1	General	64

5.5.2.4.2	Unsubscribe to changes on the GBA subscriber data	64

5.5.2.5	Modification of Subscription	64

5.5.2.5.1	General	64

5.5.2.5.2	Modification of Subscription to changes on the GBA subscriber data	64

5.5.2.5	Notify	65

5.5.2.5.1	General	65

5.5.2.5.2	Notification of changes on the GBA subscriber data	65

5.6	Nhss_gbaUEAuthentication Service	66

5.6.1	Service Description	66

5.6.2	Service Operations	66

5.6.2.1	Introduction	66

5.6.2.2	Get	66

5.6.2.2.1	General	66

5.6.2.2.2	Request UE authentication data	66

5.7	Nhss_imsEventExposure Service	67

5.7.1	Service Description	67

5.7.2	Service Operations	67

5.7.2.1	Introduction	67

5.7.2.2	Subscribe	67

5.7.2.2.1	General	67

5.7.2.2.2	Subscription to a specific subscriber related IMS event	68

5.7.2.3	Unsubscribe	68

5.7.2.3.1	General	68

5.7.2.3.2	Unsubscribe to a specific subscriber related IMS event	68

5.7.2.4	Modification of Subscription	69

5.7.2.4.1	General	69

5.7.2.4.2	Modification of Subscription to a specific subscriber related IMS event	69

5.7.2.5	Notification	70

5.7.2.5.1	General	70

5.7.2.5.2	IMS Monitoring Revocation Notification	70

6	API Definitions	71

6.1	Nhss_imsUEContextManagement Service API	71

6.1.1	Introduction	71

6.1.2	Usage of HTTP	71

6.1.2.1	General	71

6.1.2.2	HTTP standard headers	71

6.1.2.2.1	General	71

6.1.2.2.2	Content type	71

6.1.2.3	HTTP custom headers	71

6.1.2.3.1	General	71

6.1.3	Resources	72

6.1.3.1	Overview	72

6.1.3.2	Resource: Scscf Registration	72

6.1.3.2.1	Description	72

6.1.3.2.2	Resource Definition	73

6.1.3.2.3	Resource Standard Methods	73

6.1.3.2.4	Resource Custom Operations	74

6.1.3.3	Resource: Scscf Restoration Information	75

6.1.3.3.1	Description	75

6.1.3.3.2	Resource Definition	75

6.1.3.3.3	Resource Standard Methods	75

6.1.3.4	Resource: Individual Imsas Registration Information	79

6.1.3.4.1	Description	79

6.1.3.4.2	Resource Definition	79

6.1.3.4.3	Resource Standard Methods	79

6.1.3.4.4	Resource Custom Operations	82

6.1.3.5	Resource: Imsas Registrations	82

6.1.3.5.1	Description	82

6.1.3.5.2	Resource Definition	82

6.1.3.5.3	Resource Standard Methods	83

6.1.3.5.4	Resource Custom Operations	83

6.1.4	Custom Operations without associated resources	83

6.1.4.1	Overview	83

6.1.4.2	Operation: Authorize	84

6.1.4.2.1	Description	84

6.1.4.2.2	Operation Definition	84

6.1.5	Notifications	85

6.1.5.1	General	85

6.1.5.2	S-CSCF Deregistration Notification	85

6.1.5.3	IMS AS Deregistration Notification	86

6.1.5.4	IMS AS Reassignment Notification	87

6.1.6	Data Model	88

6.1.6.1	General	88

6.1.6.2	Structured data types	89

6.1.6.2.1	Introduction	89

6.1.6.2.2	Type: AuthorizationRequest	90

6.1.6.2.3	Type: AuthorizationResponse	90

6.1.6.2.4	Type: ScscfRestorationInfo	90

6.1.6.2.5	Type: ScscfRestorationInfoRequest	91

6.1.6.2.6	Type: ScscfRestorationInfoResponse	91

6.1.6.2.7	Type: RestorationInfo	91

6.1.6.2.8	Type: UeSubscriptionInfo	91

6.1.6.2.9	Type: PcscfSubscriptionInfo	92

6.1.6.2.10	Type: ScscfRegistration	93

6.1.6.2.11	Type: ExtendedProblemDetails	95

6.1.6.2.12	Type: AdditionalInfo	95

6.1.6.2.13	Type: DeregistrationData	96

6.1.6.2.14	Type: DeregistrationReason	96

6.1.6.2.15	Type: EmergencyRegisteredIdentity	96

6.1.6.2.16	Type: ImsasRegistration	97

6.1.6.2.17	Type: ImsAsRegistrationInfo	97

6.1.6.2.18	Type: AsDeregistrationData	97

6.1.6.2.19	Type: AsReassignmentData	98

6.1.6.3	Simple data types and enumerations	98

6.1.6.3.1	Introduction	98

6.1.6.3.2	Simple data types	98

6.1.6.3.3	Enumeration: AuthorizationType	98

6.1.6.3.4	Enumeration: AuthorizationResult	98

6.1.6.3.5	Enumeration: ImsRegistrationType	98

6.1.6.3.6	Enumeration: LooseRouteIndication	99

6.1.6.3.7	Enumeration: DeregistrationReasonCode	99

6.1.6.3.8	Enumeration: AsType	100

6.1.6.3.9	Enumeration: AsDeregistrationReason	100

6.1.6.4	Data types describing alternative data types or combinations of data types	101

6.1.7	Error Handling	101

6.1.7.1	General	101

6.1.7.2	Protocol Errors	101

6.1.7.3	Application Errors	101

6.1.8	Feature negotiation	101

6.1.9	Security	102

6.1.10	HTTP redirection	102

6.2	Nhss_imsSubscriberDataManagement Service API	102

6.2.1	API URI	102

6.2.2	Usage of HTTP	103

6.2.2.1	General	103

6.2.2.2	HTTP standard headers	103

6.2.2.2.1	General	103

6.2.2.2.2	Content type	103

6.2.2.3	HTTP custom headers	103

6.2.2.3.1	General	103

6.2.3	Resources	104

6.2.3.1	Overview	104

6.2.3.2	Resource: IMS Associated Identities	107

6.2.3.2.1	Description	107

6.2.3.2.2	Resource Definition	108

6.2.3.2.3	Resource Standard Methods	108

6.2.3.3	Resource: MSISDNS	109

6.2.3.3.1	Description	109

6.2.3.3.2	Resource Definition	109

6.2.3.3.3	Resource Standard Methods	109

6.2.3.4	Resource: S-CSCF Capabilities	110

6.2.3.4.1	Description	110

6.2.3.4.2	Resource Definition	111

6.2.3.4.3	Resource Standard Methods	111

6.2.3.5	Resource: IMS Profile Data	112

6.2.3.5.1	Description	112

6.2.3.5.2	Resource Definition	112

6.2.3.5.3	Resource Standard Methods	112

6.2.3.6	Resource: Subscriptions	114

6.2.3.6.1	Description	114

6.2.3.6.2	Resource Definition	114

6.2.3.6.3	Resource Standard Methods	114

6.2.3.7	Resource: Individual subscription	115

6.2.3.7.1	Description	115

6.2.3.7.2	Resource Definition	116

6.2.3.7.3	Resource Standard Methods	116

6.2.3.8	Resource: SharedDataSubscriptions	118

6.2.3.8.1	Description	118

6.2.3.8.2	Resource Definition	118

6.2.3.8.3	Resource Standard Methods	119

6.2.3.9	Resource: Shared Data Individual subscription	120

6.2.3.9.1	Description	120

6.2.3.9.2	Resource Definition	120

6.2.3.9.3	Resource Standard Methods	120

6.2.3.10	Resource: IMS Registration Status	122

6.2.3.10.1	Description	122

6.2.3.10.2	Resource Definition	123

6.2.3.10.3	Resource Standard Methods	123

6.2.3.11	Resource: Service Priority Information	124

6.2.3.11.1	Description	124

6.2.3.11.2	Resource Definition	124

6.2.3.11.3	Resource Standard Methods	124

6.2.3.12	Resource: Initial Filter Criteria	125

6.2.3.12.1	Description	125

6.2.3.12.2	Resource Definition	125

6.2.3.12.3	Resource Standard Methods	126

6.2.3.13	Resource: IMS Location Data	127

6.2.3.13.1	Description	127

6.2.3.13.2	Resource Definition	127

6.2.3.13.3	Resource Standard Methods	127

6.2.3.14	Resource: Service Level Trace Information	128

6.2.3.14.1	Description	128

6.2.3.14.2	Resource Definition	128

6.2.3.14.3	Resource Standard Methods	129

6.2.3.15	Resource: PS location Information	130

6.2.3.15.1	Description	130

6.2.3.15.2	Resource Definition	130

6.2.3.15.3	Resource Standard Methods	130

6.2.3.16	Resource: CS location Information	132

6.2.3.16.1	Description	132

6.2.3.16.2	Resource Definition	132

6.2.3.16.3	Resource Standard Methods	133

6.2.3.17	Resource: SRVCC data	134

6.2.3.17.1	Description	134

6.2.3.17.2	Resource Definition	134

6.2.3.17.3	Resource Standard Methods	135

6.2.3.18	Resource: PSI activation state	137

6.2.3.18.1	Description	137

6.2.3.18.2	Resource Definition	138

6.2.3.18.3	Resource Standard Methods	138

6.2.3.19	Resource: Repository Data	140

6.2.3.19.1	Description	140

6.2.3.19.2	Resource Definition	140

6.2.3.19.3	Resource Standard Methods	141

6.2.3.20	Resource: SharedData	145

6.2.3.20.1	Description	145

6.2.3.20.2	Resource Definition	145

6.2.3.20.3	Resource Standard Methods	145

6.2.3.21	Resource: ImeiSvInformation	146

6.2.3.21.1	Description	146

6.2.3.21.2	Resource Definition	146

6.2.3.21.3	Resource Standard Methods	146

6.2.3.22	Resource: IP Address	148

6.2.3.22.1	Description	148

6.2.3.22.2	Resource Definition	148

6.2.3.22.3	Resource Standard Methods	148

6.2.3.23	Resource: T-ADS Info	149

6.2.3.23.1	Description	149

6.2.3.23.2	Resource Definition	149

6.2.3.23.3	Resource Standard Methods	150

6.2.3.24	Resource: UeReachabilitySubscriptions	151

6.2.3.24	Description	151

6.2.3.24.2	Resource Definition	151

6.2.3.24.3	Resource Standard Methods	151

6.2.3.25	Resource: UE reachability Individual subscription	153

6.2.3.25.1	Description	153

6.2.3.25.2	Resource Definition	153

6.2.3.25.3	Resource Standard Methods	153

6.2.3.26	Resource: PS User State	156

6.2.3.26.1	Description	156

6.2.3.26.2	Resource Definition	156

6.2.3.26.3	Resource Standard Methods	156

6.2.3.27	Resource: CS User State	157

6.2.3.27.1	Description	157

6.2.3.27.2	Resource Definition	157

6.2.3.27.3	Resource Standard Methods	158

6.2.3.28	Resource: Csrn	159

6.2.3.28.1	Description	159

6.2.3.28.2	Resource Definition	159

6.2.3.28.3	Resource Standard Methods	159

6.2.3.29	Resource: ReferenceAccessLocation	161

6.2.3.29.1	Description	161

6.2.3.29.2	Resource Definition	161

6.2.3.29.3	Resource Standard Methods	161

6.2.3.30	Resource: SMSRegistrationInfo	163

6.2.3.30.1	Description	163

6.2.3.30.2	Resource Definition	163

6.2.3.30.3	Resource Standard Methods	163

6.2.3.31	Resource: Dsai	167

6.2.3.31.1	Description	167

6.2.3.31.2	Resource Definition	167

6.2.3.31.3	Resource Standard Methods	168

6.2.3.32	Resource: Private Identities	170

6.2.3.32.1	Description	170

6.2.3.32.2	Resource Definition	170

6.2.3.32.3	Resource Standard Methods	171

6.2.3.33	Resource: S-CSCF Selection Assistance Information	172

6.2.3.33.1	Description	172

6.2.3.33.2	Resource Definition	172

6.2.3.33.3	Resource Standard Methods	172

6.2.3.34	Resource: Charging Information	173

6.2.3.34.1	Description	173

6.2.3.34.2	Resource Definition	173

6.2.3.34.3	Resource Standard Methods	174

6.2.3.35	Resource: Repository Data List	174

6.2.3.35.1	Description	174

6.2.3.35.2	Resource Definition	174

6.2.3.35.3	Resource Standard Methods	175

6.2.5	Notifications	176

6.2.5.1	General	176

6.2.5.2	Data Change Notification	176

6.2.6	Data Model	177

6.2.6.1	General	177

6.2.6.2	Structured data types	180

6.2.6.2.1	Introduction	180

6.2.6.2.2	Type: ScscfCapabilityList	180

6.2.6.2.3	Type: Capabilities	181

6.2.6.2.4	Type: ImsProfileData	181

6.2.6.2.5	Type: SharedData	181

6.2.6.2.6	Void	181

6.2.6.2.7	Type: RepositoryData	181

6.2.6.2.8	Type: MsisdnList	182

6.2.6.2.9	Type: PublicIdentities	182

6.2.6.2.10	Type: PublicIdentity	182

6.2.6.2.11	Type: ImsSdmSubscription	182

6.2.6.2.12	Type: ImsRegistrationStatus	183

6.2.6.2.13	Type: PriorityLevels	183

6.2.6.2.14	Type: Ifcs	183

6.2.6.2.15	Type: Ifc	184

6.2.6.2.16	Type: TriggerPoint	184

6.2.6.2.17	Type: Spt	185

6.2.6.2.18	Type: HeaderSipRequest	185

6.2.6.2.19	Type: SdpDescription	185

6.2.6.2.20	Type: ApplicationServer	186

6.2.6.2.21	Type: ImsLocationData	186

6.2.6.2.22	Type: ServiceLevelTraceInformation	186

6.2.6.2.23	Type: PsLocation	186

6.2.6.2.24	Type: SgsnLocationData	187

6.2.6.2.25	Type: MmeLocationData	187

6.2.6.2.26	Type: AmfLocationData	187

6.2.6.2.27	Type: TwanLocationData	188

6.2.6.2.28	Type: CsLocation	188

6.2.6.2.29	Type: CsgInformation	188

6.2.6.2.30	Type: SrvccData	189

6.2.6.2.31	Type: PsiActivationState	189

6.2.6.2.32	Type: ImsServiceProfile	189

6.2.6.2.33	Type: PublicIdentifier	189

6.2.6.2.34	Type: ImeiSvInformation	190

6.2.6.2.35	Void	190

6.2.6.2.36	Type: TadsInformation	190

6.2.6.2.37	Type: UeReachabilitySubscription	190

6.2.6.2.38	Type: UeReachabilityNotification	190

6.2.6.2.39	Type: PsUserState	190

6.2.6.2.40	Type: CsUserState	191

6.2.6.2.41	Type: Csrn	191

6.2.6.2.42	Type: ReferenceLocationInformation	191

6.2.6.2.43	Type: SmsRegistrationInfo	191

6.2.6.2.44	Type: IpSmGwAddress	192

6.2.6.2.45	Type: ImsAssociatedIdentities	192

6.2.6.2.46	Type: DsaiTagInformation	192

6.2.6.2.47	Type: DsaiTagStatus	192

6.2.6.2.48	Type: CreatedUeReachabilitySubscription	192

6.2.6.2.49	Type: PrivateIdentities	193

6.2.6.2.50	Type: PrivateIdentity	193

6.2.6.2.51	Type: ScscfSelectionAssistanceInformation	193

6.2.6.2.52	Type: ChargingInfo	193

6.2.6.2.53	Type: RepositoryDataList	193

6.2.6.2.54	Type: CoreNetworkServiceAuthorization	194

6.2.6.3	Simple data types and enumerations	194

6.2.6.3.1	Introduction	194

6.2.6.3.2	Simple data types	194

6.2.6.3.3	Enumeration: DataSetName	195

6.2.6.3.4	Enumeration:  IdentityType	195

6.2.6.3.5	Enumeration:  ImsRegistrationState	195

6.2.6.3.6	Enumeration: TypeOfCondition	195

6.2.6.3.7	Enumeration: RegistrationType	195

6.2.6.3.8	Enumeration: RequestDirection	196

6.2.6.3.9	Enumeration: ServiceInformation	196

6.2.6.3.10	Enumeration: RequestedNode	196

6.2.6.3.11	Enumeration:  SrvccCapability	196

6.2.6.3.12	Enumeration:  ActivationState	197

6.2.6.3.13	Enumeration: ImsVoiceOverPsSessionSupport	197

6.2.6.3.14	Enumeration: AccessType	197

6.2.6.3.15	Enumeration: UserStatePs	197

6.2.6.3.16	Enumeration: UserStateCs	198

6.2.6.3.17	Enumeration: PrivateIdentityType	198

6.2.7	Error Handling	198

6.2.7.1	General	198

6.2.7.2	Protocol Errors	198

6.2.7.3	Application Errors	198

6.2.8	Feature negotiation	199

6.2.9	Security	199

6.2.10	HTTP redirection	200

6.3	Nhss_imsUEAuthentication Service API	201

6.3.1	API URI	201

6.3.2	Usage of HTTP	201

6.3.2.1	General	201

6.3.2.2	HTTP standard headers	201

6.3.2.2.1	General	201

6.3.2.2.2	Content type	201

6.3.2.3	HTTP custom headers	201

6.3.2.3.1	General	201

6.3.3	Resources	202

6.3.3.1	Overview	202

6.3.3.2	Resource: SecurityInformation	202

6.3.3.2.1	Description	202

6.3.3.2.2	Resource Definition	202

6.3.3.2.3	Resource Standard Methods	203

6.3.3.2.4	Resource Custom Operations	203

6.3.4	Custom Operations without associated resources	204

6.3.5	Notifications	204

6.3.6	Data Model	204

6.3.6.1	General	204

6.3.6.2	Structured data types	205

6.3.6.2.1	Introduction	205

6.3.6.2.2	Type: SipAuthenticationInfoRequest	205

6.3.6.2.3	Type: SipAuthenticationInfoResult	205

6.3.6.2.4	Type: ResynchronizationInfo	206

6.3.6.2.5	Type: 3GAkaAv	206

6.3.6.2.6	Type: DigestAuthentication	206

6.3.6.2.7	Void	206

6.3.6.3	Simple data types and enumerations	206

6.3.6.3.1	Introduction	206

6.3.6.3.2	Simple data types	206

6.3.6.3.3	Enumeration: SipAuthenticationScheme	206

6.3.6.3.4	Enumeration: SipDigestAlgorithm	207

6.3.6.3.5	Enumeration: SipDigestQop	207

6.3.7	Error Handling	207

6.3.7.1	General	207

6.3.7.2	Protocol Errors	207

6.3.7.3	Application Errors	207

6.3.8	Feature Negotiation	208

6.3.9	Security	208

6.3.10	HTTP redirection	208

6.4	Nhss_gbaSubscriberDataManagement Service API	208

6.4.1	API URI	208

6.4.2	Usage of HTTP	208

6.4.2.1	General	208

6.4.2.2	HTTP standard headers	209

6.4.2.2.1	General	209

6.4.2.2.2	Content type	209

6.4.2.3	HTTP custom headers	209

6.4.2.3.1	General	209

6.4.3	Resources	209

6.4.3.1	Overview	209

6.4.3.2	Resource: GBA Subscriber Data	210

6.4.3.2.1	Description	210

6.4.3.2.2	Resource Definition	210

6.4.3.2.3	Resource Standard Methods	210

6.4.3.3	Resource: Subscriptions	211

6.4.3.3.1	Description	211

6.4.3.3.2	Resource Definition	211

6.4.3.3.3	Resource Standard Methods	212

6.4.3.4	Resource: Individual subscription	213

6.4.3.4.1	Description	213

6.4.3.4.2	Resource Definition	213

6.4.3.4.3	Resource Standard Methods	213

6.4.4	Custom Operations without associated resources	215

6.4.5	Notifications	215

6.4.5.1	General	215

6.4.5.2	Data Change Notification	216

6.4.6	Data Model	217

6.4.6.1	General	217

6.4.6.2	Structured data types	217

6.4.6.2.1	Introduction	217

6.4.6.2.2	Type: GbaSubscriberData	217

6.4.6.2.3	Type: Guss	217

6.4.6.2.4	Type: BsfInfo	218

6.4.6.2.5	Type: GbaSdmSubscription	218

6.4.6.3	Simple data types and enumerations	218

6.4.6.3.1	Introduction	218

6.4.6.3.2	Simple data types	218

6.4.6.3.3	Enumeration: UiccType	218

6.4.7	Error Handling	219

6.4.7.1	General	219

6.4.7.2	Protocol Errors	219

6.4.7.3	Application Errors	219

6.4.8	Feature Negotiation	219

6.4.9	Security	219

6.4.10	HTTP redirection	220

6.5	Nhss_gbaUEAuthentication Service API	220

6.5.1	API URI	220

6.5.2	Usage of HTTP	220

6.5.2.1	General	220

6.5.2.2	HTTP standard headers	220

6.5.2.2.1	General	220

6.5.2.2.2	Content type	220

6.5.2.3	HTTP custom headers	220

6.5.2.3.1	General	220

6.5.3	Resources	221

6.5.3.1	Overview	221

6.5.3.2	Resource: SecurityInformation (Custom Operation)	221

6.5.3.2.1	Description	221

6.5.3.2.2	Resource Definition	221

6.5.3.2.3	Resource Standard Methods	221

6.5.3.2.3	Resource Custom Operations	222

6.5.5	Notifications	223

6.5.6	Data Model	223

6.5.6.1	General	223

6.5.6.2	Structured data types	223

6.5.6.2.1	Introduction	223

6.5.6.2.2	Type: AuthenticationInfoRequest	224

6.5.6.2.3	Type: AuthenticationInfoResult	224

6.5.6.2.4	Void	224

6.5.6.2.5	Void	224

6.5.6.2.6	Type: DigestAuthentication	224

6.5.6.3	Simple data types and enumerations	224

6.5.6.3.1	Introduction	224

6.5.6.3.2	Simple data types	224

6.5.6.3.3	Enumeration: AuthenticationScheme	224

6.5.6.3.4	Enumeration: DigestAlgorithm	225

6.5.6.3.5	Enumeration: DigestQop	225

6.5.7	Error Handling	225

6.5.7.1	General	225

6.5.7.2	Protocol Errors	225

6.5.7.3	Application Errors	225

6.5.8	Feature Negotiation	225

6.5.9	Security	226

6.5.10	HTTP redirection	226

6.6	Nhss_imsEventExposure Service API	226

6.6.1	API URI	226

6.6.2	Usage of HTTP	226

6.6.2.1	General	226

6.6.2.2	HTTP standard headers	227

6.6.2.2.1	General	227

6.6.2.2.2	Content type	227

6.6.2.3	HTTP custom headers	227

6.6.2.3.1	General	227

6.6.3	Resources	227

6.6.3.1	Overview	227

6.6.3.2	Resource: IMS Event Subscriptions	228

6.6.3.2.1	Description	228

6.6.3.2.2	Resource Definition	228

6.6.3.2.3	Resource Standard Methods	228

6.6.3.2.4	Resource Custom Operations	229

6.6.3.3	Resource: Individual IMS Event Subscription	229

6.6.3.3.1	Description	229

6.6.3.3.2	Resource Definition	230

6.6.3.3.3	Resource Standard Methods	230

6.6.3.3.4	Resource Custom Operations	232

6.6.4	Custom Operations without associated resources	232

6.6.5	Notifications	233

6.6.5.1	General	233

6.6.5.2	Monitoring Revocation Notification	233

6.6.6	Data Model	234

6.6.6.1	General	234

6.6.6.2	Structured data types	234

6.6.6.2.1	Introduction	234

6.6.6.2.2	Type: ImsEeSubscription	235

6.6.6.2.3	Type: CreatedImsEeSubscription	235

6.6.6.2.4	Void	236

6.6.6.2.5	Void	236

6.6.6.2.6	Type: ImsEeMonitoringRevoked	236

6.6.6.2.7	Type: ImsEeMonitoringEvent	236

6.6.6.3	Simple data types and enumerations	236

6.6.6.3.1	Introduction	236

6.6.6.3.2	Simple data types	236

6.6.6.3.3	Enumeration: ImsRevokedCause	237

6.6.6.4	Data types describing alternative data types or combinations of data types	237

6.6.6.5	Binary data	237

6.6.7	Error Handling	237

6.6.7.1	General	237

6.6.7.2	Protocol Errors	237

6.6.7.3	Application Errors	237

6.6.8	Feature negotiation	237

6.6.9	Security	238

6.6.10	HTTP redirection	238

Annex A (normative): OpenAPI specification	239

A.1	General	239

A.2	Nhss_imsUECM API	239

A.3	Nhss_imsSDM API	254

A.4	Nhss_imsUEAU API	307

A.5	Nhss_gbaSDM API	311

A.6	Nhss_gbaUEAU API	316

A.7	Nhss_imsEE API	319

Annex B (informative): Withdrawn API versions	325

B.1	General	325

Annex C (informative): Change history	326

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol, including message flows and API specification details, for the Nhss services, as part of the 5G Service-Based Architecture, offered by the HSS.

The HSS services specified in the present document include:

-	Services for interworking with the IP Multimedia Subsystem (IMS)

-	Services for interworking with the Generic Bootstrapping Architecture (GBA)

NOTE:	The HSS services for Interworking with the Unified Data Management (UDM) Network Function are specified in 3GPP TS 29.563 [39].

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3].

The IP Multimedia Subsystem (IMS) stage 2 architecture and procedures are specified in 3GPP TS 23.228 [6].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

The stage 2 architecture and procedures of SBA-enabled GBA is specified in 3GPP TS 33.220 [40] and 3GPP TS 33.223 [41].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 29.335: "User Data Repository Access Protocol over the Ud interface; Stage 3".

[8]	IETF RFC 9113: "HTTP/2".

[9]	OpenAPI Initiative, "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[10]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[11]	IETF RFC 9457: "Problem Details for HTTP APIs".

[12]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[13]	3GPP TS 23.003: "Numbering, addressing and identification".

[14]	3GPP TS 33.203: "Access security for IP-based services".

[15]	3GPP TS 29.503: "Unified Data Management Services; Stage 3".

[16]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[17]	IETF RFC 4740: "Diameter Session Initiation Protocol (SIP) Application".

[18]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[19]	IETF RFC 4412: "Communications Resource Priority for the Session Initiation Protocol".

[20]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx interfaces;Signalling flows and message contents".

[21]	3GPP TS 29.218: "IP Multimedia (IM) session handling; IM call model; Stage 2".

[22]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[23]	IETF RFC 8497: "Marking SIP messages to be logged".

[24]	3GPP TS 24.323: "3GPP IP Multimedia Subsystem (IMS) service level tracing Management Object (MO)".

[25]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[26]	3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[27]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

[28]	IETF RFC 2045: "Multipurpose Internet Mail Extensions(MIME) Part One: Format of Internet Message Bodies".

[29]	ETSI ES 283 034: "Network Attachment Sub-System (NASS); e4 interface based on the DIAMETER protocol".

[30]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[31]	3GPP TS 23.078: "Customised Applications for Mobile network Enhanced Logic (CAMEL) Phase 3 - Stage 2".

[32]	3GPP TS 29.272: "Evolved Packet System; MME and SGSN Related Interfaces Based on Diameter Protocol".

[33]	3GPP TS 29.518: 5G System; Access and Mobility Management Services; Stage 3".

[34]	3GPP TR 21.900: "Technical Specification Group working methods".

[35]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[36]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[37]	3GPP TS 23.380: "IP Multimedia Subsystem (IMS); IMS Restoration Procedures".

[38]	3GPP TS 29.328: "IP Multimedia (IM) Subsystem Sh interface; Signalling flows and message contents".

[39]	3GPP TS 29.563: "Home Subscriber Server (HSS) services for interworking with Unified Data Management (UDM); Stage 3".

[40]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[41]	3GPP TS 33.223: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) Push function".

[42]	3GPP TS 29.309: "Bootstrapping Server Function (GBA BSF) Services; Stage 3".

[43]	3GPP TS 23.218: "IP Multimedia (IM) session handling; IM call model; Stage 2".

[44]	3GPP TS 29.175: "IP Multimedia Subsystem (IMS) Application Server (AS) Services; Stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

The following terms and definitions given in TS 23.003 [13] apply:

Distinct Public Service Identity

Distinct Public User Identity

Public Service Identity

Public User Identity

Wildcarded Public Service Identity

Wildcarded Public User Identity

In the present document, the following terms and definitions apply.

Common Part (of a user profile): Contains Initial Filter Criteria instances that should be evaluated both for registered and unregistered Public User Identities, or for unregistered Public Service Identities in the S-CSCF.

Complete user profile: Contains the Initial Filter Criteria instances of all three different user profile parts; registered part, unregistered part and common part.

IP Multimedia session: IP Multimedia session and IP Multimedia call are treated as equivalent in this specification.

Authentication pending flag: A flag that indicates that the authentication of a Public User Identity - Private User Identity pair is pending and waiting for confirmation.

Implicitly registered Public User Identity set: A set of Public User Identities, which are registered and de-registered simultaneously when any of the Public User Identities belonging to that set is registered or de-registered.

Not Registered State: Public Identity is not Registered and has no S-CSCF assigned.

Private Identity: Either a Private User Identity or a Private Service Identity.

Public Identity: Either a Public User Identity or a Public Service Identity.

Registered State: Public User Identity is Registered at the request of the user and has an S-CSCF assigned.

Unregistered State: Public Identity is not Registered but has a serving S-CSCF assigned to execute Unregistered state services as a consequence of a terminating request, or an originating request from an AS on behalf of a user, or there is an S-CSCF keeping the user profile stored.

User information: The user related data that the S-CSCF requests from the HSS or HSS pushes to the S-CSCF, e.g. user profile, charging information, allowed WAF and/or WWSF identities and authentication information.

User profile: Data that is sent in the resource associated to the complete user's subscription data.

IP Multimedia session: IP Multimedia session and IP Multimedia call are treated as equivalent in this specification.

Transparent data: Data that is understood syntactically but not semantically by the HSS. It is data that an AS may store in the HSS to support its service logic. One example is data that an AS stores in the HSS, using it as a repository.

Non-transparent data: Data that is understood both syntactically and semantically by the HSS.

AS (Application Server): a term used to denote either of a SIP Application Server or an OSA Service Capability Server.

## 3.2	Symbols

Void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

AS	Application Server

AMF	Access and Mobility Management Function

CSCF	Call Session Control Function

CSG	Closed Subscriber Group

DCSF	Data Channel Signalling Function

FQDN	Fully Qualified Domain Name

GIBA	GPRS-IMS-Bundled-Authentication

GPSI	Generic Public Subscription Identifier

HSS	Home Subscriber Server

IMS	IP Multimedia Subsystem

I-CSCF	Interrogating CSCF

JSON	Javascript Object Notation

MPS	Multimedia Priority Service

NAI	Network Access Identifier

NEF	Network Exposure Function

NRF	Network Repository Function

NSSAI	Network Slice Selection Assistance Information

NWDAF	Network Data Analytics Function

P-CSCF	Proxy CSCF

SBI	Service Based Interface

SIP	Session Initiation Protocol

SPT	Service Point Trigger

S-CSCF	Serving CSCF

UDR	Unified Data Repository

# 4	Overview

## 4.1	Introduction

Within the 5GC, the HSS offers services to the S-CSCF, I-CSCF, IMS-AS, DCSF (on the IP Multimedia Subsystem), GBA BSF and NEF via the Nhss service based interface (see 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3]).

Figure 4.1-1 provides the reference model in service-based interface representation with focus on the HSS.

Figure 4.1-1: Reference model – HSS

# 5	Services offered by the HSS

## 5.1	Introduction

The SBI capable HSS offers the following services via the Nhss_ims interface:

-	Nhss_imsUEContextManagement Service

-	Nhss_imsSubscriberDataManagement Service

-	Nhss_imsUEAuthentication Service

-	Nhss_gbaSubscriberDataManagement Service

-	Nhss_gbaUEAuthentication Service

-	Nhss_imsEventExposure Service

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Nhss_imsUEContextManagement | 6.1 | Nhss UE Context Management Service for IMS | TS29562_Nhss_imsUECM.yaml | nhss-ims-uecm | A.2 |
| Nhss_imsSubscriberDataManagement | 6.2 | Nhss Subscriber Data Management Service for IMS | TS29562_Nhss_imsSDM.yaml | nhss-ims-sdm | A.3 |
| Nhss_imsUEAuthentication | 6.3 | Nhss UE Authentication Service for IMS | TS29562_Nhss_imsUEAU.yaml | nhss-ims-ueau | A.4 |
| Nhss_gbaSubscriberDataManagement | 6.4 | Nhss Subscriber Data Management Service for GBA | TS29562_Nhss_gbaSDM.yaml | nhss-gba-sdm | A.5 |
| Nhss_gbaUEAuthentication | 6.5 | Nhss UE Authentication Service for GBA | TS29562_Nhss_gbaUEAU.yaml | nhss-gba-ueau | A.6 |
| Nhss_imsEventExposure | 6.6 | Nhss subscriber related events exposure | TS29.562_Nhss_imsEE.yaml | nhss-ims-ee | A.7 |

All scenarios shown in the following clauses assume that the SBI capable HSS is stateful and stores information in local memory. However, the SBI capable HSS may be stateless and store information externally in the UDR. If so, the stateless SBI capable HSS may use Ud interface as specified in 3GPP TS 29.335 [7] to retrieve required data from the UDR and store them locally before processing an incoming request. Processing the incoming request may then include updating data in the UDR. After processing the incoming request, the SBI capable HSS may delete the locally stored data. When data stored in UDR is then shared among the different SBI capable HSS instances of the same group, as identified by HSS Group ID (see 3GPP TS 23.501 [2], clause 6.2.6), bulk subscriptions, as described in clause 4.15.3.2.4 of 3GPP TS 23.502 [3], are not applicable, i.e. an NF consumer (e.g. IMS-AS) only subscribes towards one of the SBI capable HSS instances within the group.

Editor's Note:	It is FFS if an informative Annex needs to be included to show how an IMS-AS/I-CSCF/S-CSCF can interact with a group of stateless SBI capable HSS instances.

## 5.2	Nhss_imsUEContextManagement Service

### 5.2.1	Service Description

See 3GPP TS 23.228 [6], clause AA.2.1.2.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

For the Nhss_imsUEContextManagement service the following service operations are defined:

-	Registration

-	DeregistrationNotification

-	Deregistration

-	Authorize

-	Update

-	RestorationInfoGet

-	RestorationInfoUpdate

-	RestorationInfoDelete

-	AsInfoGet

-	IndividualImsasRegistrationInfoGet

IMS restoration procedures are specified in 3GPP TS 23.380 [37].

The Nhss_imsUEContextManagement Service is used by Consumer NFs (I-CSCF/S-CSCF or IMS AS) to:

-	register at the HSS by means of the Registration service operation (S-CSCF or IMS AS)

-	get notified by means of the DeregistrationNotification service operation when HSS decides to deregister the registered consumer NF (S-CSCF)

-	deregister from the HSS by means of the Deregistration service operation (S-CSCF)

-	request registration authorization from HSS by means of the Authorize service operation (I-CSCF)

-	update registration information stored at the HSS by means of the Update service operation (S-CSCF or IMS AS)

-	retrieve Restoration Information stored at the HSS by means of the RestorationInfoGet service operation (S-CSCF)

-	update Restoration Information stored at the HSS by means of the RestorationInfoUpdate service operation (S-CSCF)

-	delete Restoration Information stored at the HSS by means of the RestorationInfoDelete service operation (S-CSCF)

The Nhss_imsUEContextManagement Service is used by Consumer NFs (NEF) to:

-	retrieve the information of the IMS AS served the specific subscriber from the HSS

-	retrieve registration information of an individual IMS AS instance from HSS by means of the IndividualImsasRegistrationInfoGet service operation.

#### 5.2.2.2	Registration

##### 5.2.2.2.1	General

The Registration service operation is invoked by a NF that has been selected to provide service to the UE to store related IMS UE Context Management information in HSS.

NF Consumer is S-CSCF or IMS AS.

As part of this procedure, the HSS updates the registration state of the requested IMS Public Identity and IMS Private Identity. The state of the IMS Public Identities in the same Implicit Registration Set is also updated.

The following procedures using the Registration service operation are supported:

-	S-CSCF registration

-	IMS AS registration

##### 5.2.2.2.2	S-CSCF Registration

Figure 5.2.2.2.2-1 shows a scenario where the S-CSCF sends a request to register at the HSS (see also 3GPP TS 23.228 [6] clause 5.2). The request contains the IMS UE's identity (/{imsUeId}) which shall be an IMPU, the S-CSCF Registration Information and the Registration Type.

Figure 5.2.2.2.2-1: S-CSCF registration

1.	The S-CSCF sends a PUT request to the resource representing the UE's S-CSCF registration to update or create S-CSCF registration information. The S-CSCF shall check whether there is a reg-id parameter in the Contact header in the SIP REGISTER message. If a reg-id parameter exists, the S-CSCF shall indicate to the HSS that the registration is related to a multiple registration as specified in clause 4.2.3 of 3GPP TS 23.380 [37]. If the S-CSCF received a S-CSCF reselection indicator from the I-CSCF, the S-CSCF shall include that indicator to the HSS.

2a.	If the resource already exists, and if the S-CSCF registered is the same or if another S-CSCF is registered and S-CSCF reselection indication is received, the HSS updates the scscfRegistration resource by replacing it with the received resource information (e.g. S-CSCF name, state of the related IMS public identity based on the imsRegistrationType received) and responds with "204 No Content" or "200 OK" with the updated resource information and HSS supported features.

Additionally, if the S-CSCF is reselected, the HSS shall invoke the Deregistration Notification service operation towards the old S-CSCF using the callback URI provided by the old S-CSCF with deregistration reason set to "NEW SERVER ASSIGNED".

When receiving the "200 OK" response and if a multiple registration indicator has indicated to the HSS, the S-CSCF shall retrieve and update the restoration information as specified in clause 5.2.2.6 and clause 5.2.2.7.

2b.	If the resource does not exist (there is no previous S-CSCF information stored in HSS), HSS stores the received S-CSCF registration data and responds with HTTP Status Code "201 created". A response body is included with the created resource and the features supported by HSS.

2c.	If the operation is not authorized due to e.g. received IMS identities do not match, or a different S-CSCF is registered and S-CSCF reselection is not authorized, HTTP status code "403 Forbidden" should be returned including additional error information in the response body (in "ExtendedProblemDetails" element).

If the operation is not authorized due to received IMS identities do not exist, HTTP status code "404 Not Found" should be returned including additional error information in the response body (in "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PUT response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

If the S-CSCF receives from HSS a response message containing the 3gpp-Sbi-Oci HTTP header with the parameter "Extend-Registration-Timer" set to "true", as described in 3GPP TS 29.500 [4], the S-CSCF shall abate the received traffic by extending the registraton timers of the UEs trying to get access to the network; this situation shall be enforced during the time period indicated in the "Period-of-Validity" parameter of this HTTP header, or until the "Extend-Registration-Timer" parameter is received with value "false".

##### 5.2.2.2.3	IMS AS Registration

Figure 5.2.2.2.3-1 shows a scenario where the IMS AS sends a request to register at the HSS. The request contains the IMS UE's identity (/{imsUeId}) which shall be an IMPU, the IMS AS Registration Information and the Registration Type.

Figure 5.2.2.2.3-1: IMS AS registration

1.	The IMS AS sends a PUT request to the resource representing the UE's IMS AS registration to create or update IMS AS registration information.

2a.	If the resource already exists, the HSS updates the ImsasRegistrations resource by replacing it with the received resource information and responds with HTTP status code "204 No Content" or "200 OK" with the updated resource information.

2b.	If the resource does not exist (there is no previous IMS AS information stored in HSS for the IMS UE's identity and the IMS AS instance ID), HSS stores the received IMS AS registration data and responds with HTTP Status Code "201 Created". A response body is included with the created resource.

The HSS checks whether another IMS AS instance of the same IMS AS type is registered for the IMS UE's identity. If the IMS AS includes "IMS AS reassignment for new sessions" indication, HSS keeps the existing IMS AS registration and accepts the newly assigned IMS AS registration and sends a notification to the existing IMS AS to indicate that a new IMS AS is assigned for the IMS UE’s identity, using the AsReassignmentCallbackUri included during the registration of the existing IMS AS. Otherwise, HSS removes the registration information associated to the old IMS AS instance(s) of the same IMS AS Type and initiates a de-registration towards the IMS AS.

If the registration information is removed, the HSS invokes the Deregistration Notification service operation towards the deregistered IMS AS using the callback URI provided by the previously registered IMS AS during registration, as specified in clause 5.2.2.3.3.

2c.	If the operation is not authorized due to e.g. received IMS UE's identity does not exist, HTTP status code "404 Not Found" should be returned including additional error information in the response body (in "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PUT response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

#### 5.2.2.3	DeregistrationNotification

##### 5.2.2.3.1	General

The following procedure using the DeregistrationNotification service operation is supported:

-	HSS initiated S-CSCF Deregistration

-	HSS initiated IMS AS Deregistration

##### 5.2.2.3.2	HSS initiated S-CSCF Deregistration

Figure 5.2.2.3.2-1 shows a scenario where the HSS notifies the registered S-CSCF about the deregistration of one or more IMS Public Identities (see also 3GPP TS 23.228 [6] clause 5.3.2.2.1). The request contains the callback URI for deregistration notification as received by the HSS during registration, and Deregistration Data.

Figure 5.2.2.3.2-1: HSS initiated S-CSCF Deregistration

1.	The HSS determines the need to deregister one or more public identities and sends a POST request to the callbackReference as provided by the NF service consumer (S-CSCF) during the registration.

NOTE:	The callbackReference identifies a public identity or an IRS or a Wildcarded PSI that shall be deregistered.

HSS includes DeregistrationData with the Deregistration Reason that triggered the notification and additional information about the public identities that shall be deregistered such as:

-	Private Identity of the public identity to be deregistered,

-	if several Private Identities with all associated Public Identities need to be deregistered, the Associated Private Identities.

2a.	On success, the NF service consumer responds with "204 No Content" or with "200 OK" including additional information about the deregistration result such as:

-	Associated Private Identities: it contains all Private Identities that have been deregistered together with the one in the Private User Identity received in the request,

-	Identities with Emergency Registration: This information element indicates a list of pairs of private and public user identities which have not been de-registered due to emergency registration.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

In the case of redirection, the NF Service Consumer shall return 3xx status code, which shall contain a Location header with an URI pointing to an alternative notification endpoint.

##### 5.2.2.3.3	HSS initiated IMS AS Deregistration

Figure 5.2.2.3.3-1 shows a scenario where the HSS notifies the registered IMS AS about the deregistration of an IMS Public Identity. The request contains the callback URI for deregistration notification as received by the HSS during registration, and AsDeregistrationData for the IMS AS.

Figure 5.2.2.3.3-1: HSS initiated IMS AS Deregistration

1.	The HSS determines the need to deregister the IMS public identity and sends a POST request to the callbackReference as provided by the NF service consumer (IMS AS) during the registration.

HSS includes AsDeregistrationData with the Deregistration Reason that triggered the notification.

2a.	On success, the NF service consumer (IMS AS) responds with "204 No Content" or with "200 OK".

The IMS AS shall remove all information for the Public Identity indicated in the request. This includes the removal of any subscription to specific subscriber related IMS events, if any.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

In the case of redirection, the NF Service Consumer shall return 3xx status code, which shall contain a Location header with an URI pointing to an alternative notification endpoint.

#### 5.2.2.4	Deregistration

##### 5.2.2.4.1	General

The following procedures using the Deregistration service operation are supported:

-	S-CSCF deregistration

##### 5.2.2.4.2	S-CSCF deregistration

Figure 5.2.2.4.2-1 shows a scenario where the S-CSCF sends a request to the HSS to deregister a user (see also 3GPP TS 23.228 [6] clause 5.3.1). The request contains the IMS UE's identity (/{imsUeId}) which shall be an IMPI or an IMPU and the deregistration information.

Figure 5.2.2.4.2-1: S-CSCF deregistering

1.	The S-CSCF sends a PUT request to the resource representing the UE's CSCF registration.

2a.	The HSS shall check whether the received S-CSCF name matches the stored S-CSCF. If the S-CSCF name is different, the HSS shall check whether the S-CSCF reassignment is allowed (i.e. S-CSCF reassignment pending flag is set).

If an IMPI is received as ImsUeId, HSS shall deregister all IMPUs associated to that IMPI.

If an IMPU is received as ImsUeId, HSS shall deregister the IMPU and related IMPUs in the Implicit Registration Set for the related IMPI.

On success, HSS shall respond "204 No Content" or "200 OK".

2b.	Otherwise the HSS responds with "403 Forbidden" (e.g. S-CSCF is not the current S-CSCF assigned) or "404 Not Found" (e.g. IMPU or IMPI do not exist).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PUT response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.2.2.4.3	IMS AS deregistration

Figure 5.2.2.4.3-1 shows a scenario where the IMS AS sends a request to the HSS to deregister a user when the user initialed IMS de-registration procedure. The request contains the IMS UE's identity (/{imsUeId}) which shall be an IMPU and the deregistration information.

Figure 5.2.2.4.3-1: IMS AS deregistration

1.	The IMS AS sends a DELETE request to the resource representing the UE's Individual Imsas Registration information registration.

2a.	On success, HSS shall respond "204 No Content" or "200 OK".

2b.	Otherwise the HSS responds with "403 Forbidden" or "404 Not Found" (e.g. IMPU or IMS AS instanceId do not exist).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PUT response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

#### 5.2.2.5	Authorize

##### 5.2.2.5.1	General

The Authorize service operation is invoked by a NF to request UE authorization to register or establish terminating /originating sessions in the IMS network.

NF Consumer is I-CSCF for IMS service.

As part of this procedure, the HSS authorizes or rejects the subscriber to use the IMS service based on the authorization information provided in the request and subscription data (e.g. roaming restrictions, barrings).

The following procedures using the Authorize service operation are supported:

-	Authorization request

##### 5.2.2.5.2	Authorization request

Figure 5.2.2.5.2-1 shows a scenario where the I-CSCF sends a request to the HSS to authorize the UE to register in a given PLMN's P-CSCF or initiate a session for unregistered services (see also 3GPP TS 23.228 [6] clause 5.2.2.3 or 5.12.2). The request contains the IMS UE's identity (/{imsUeId}) which shall be an IMPU, and the AuthorizationRequest.

Figure 5.2.2.5.2-1: Authorization

1.	The I-CSCF sends a POST request (custom method: authorize) to request for UE's authorization to register or establish an originating/terminating session.

2a.	If the operation cannot be authorized due to e.g. UE does not have required subscription data, HTTP status code "403 Forbidden" should be returned including additional error information in the response body (in "ProblemDetails" element).

2b.	If successful, HSS responds with HTTP Status Code "200 OK" and may provide the S-CSCF identity which is serving the user, if any.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

If the I-CSCF receives from HSS a response message containing the "3gpp-Sbi-Oci" HTTP header with the parameter "Extend-Registration-Timer" set to true, as described in 3GPP TS 29.500 [4], the I-CSCF shall abate the received traffic by extending the registraton timers of the UEs trying to get access to the network; this situation shall be enforced during the time period indicated in the "Period-of-Validity" parameter of this HTTP header, or until the "Extend-Registration-Timer" parameter is received with value false.

#### 5.2.2.6	RestorationInfoGet

##### 5.2.2.6.1	General

The RestorationInfoGet service operation is invoked by a NF to retrieve restoration information in HSS.

NF Consumer is S-CSCF.

The following procedures using the RestorationInfoGet service operation are supported:

-	Restoration Information Retrieval

##### 5.2.2.6.2	Restoration Information Retrieval

Figure 5.2.2.6.2-1 shows a scenario where the NF service consumer (S-CSCF) sends a request to the HSS to retrieve the information related to a specific registration for a registered user (e.g. during a registration procedure, during an originating request procedure or during a terminating request procedure). The request contains the IMS UE's identity (/{imsUeId}) which shall be an IMPU.

Figure 5.2.2.6.2-1: Restoration Information Retrieval

1.	The NF service consumer (S-CSCF) sends a GET request to the resource representing the UE's S-CSCF restoration information.

2a.	On success, the HSS responds with "200 OK" with the message body containing the UE's S-CSCF restoration information. The S-CSCF shall send a request to the HSS to retrieve the UE's IMS profile as specified in clause 5.3.2.2.4.

2b.	If there is no valid restoration data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

#### 5.2.2.7	RestorationInfoUpdate

##### 5.2.2.7.1	General

The RestorationInfoUpdate service operation is invoked by a NF to update restoration information in HSS.

NF Consumer is S-CSCF.

The following procedures using the RestorationInfoUpdate service operation are supported:

-	Restoration Information Update

##### 5.2.2.7.2	Restoration Information Update

S-CSCF backup and update of S-CSCF restoration information in HSS is specified in clause 4.6 of 3GPP TS 23.380 [37].

Figure 5.2.2.7.2-1 shows a scenario where the S-CSCF sends a request to update restoration information at the HSS (see also 3GPP TS 23.228 [6] clause 4.24). The request contains the IMS UE's identity (/{imsUeId}) which shall be an IMPU.

Figure 5.2.2.7.2-1: Restoration Information Update

1.	The S-CSCF sends a PUT request to the resource representing the UE's S-CSCF restoration information to update or create S-CSCF restoration information.

2a.	If there are S-CSCF restoration information related to the IMS Public Identity stored in the HSS, the HSS updates the Scscf Restoration Information resource by replacing it with the received restoration information and responds with "200 OK" or "204 No Content".

2b.	If the resource does not exist (there is no previous S-CSCF restoration information stored in HSS for that user), HSS stores the received S-CSCF restoration data and responds with HTTP Status Code "201 created".

2c.	If the request can't be accepted (e.g. the S-CSCF is not allowed to create/update the restoration information), HTTP status code "403 Forbidden" should be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PUT response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

#### 5.2.2.8	RestorationInfoDelete

##### 5.2.2.8.1	General

The RestorationInfoDelete service operation is invoked by a NF to delete restoration information in HSS.

NF Consumer is S-CSCF.

The S-CSCF consumes the RestorationInfoDelete service operation as a consequence of subscriber deregistration (TIMEOUT_DEREGISTRATION, USER_DEREGISTRATION, ADMINISTRATIVE_DEREGISTRATION). In restoration scenarios, the new S-CSCF after successful retrieval of restoration data, consumes the RestorationInfoDelete service operation to delete the restoration data stored by the old S-CSCF.

The following procedures using the RestorationInfoDelete service operation are supported:

-	Restoration Information Delete

##### 5.2.2.8.2	Restoration Information Delete

Figure 5.2.2.Z.2-1 shows a scenario where the S-CSCF sends a request to delete restoration information at the HSS. The request contains the IMS UE's identity (/{imsUeId}) which shall be an IMPU.

Figure 5.2.2.8.2-1: Restoration Information Delete

1.	The S-CSCF sends a DELETE request to the resource representing the UE's S-CSCF restoration information to delete S-CSCF restoration information.

2a.	If there are S-CSCF restoration information related to the IMS Public Identity stored in the HSS, the HSS deletes the Scscf Restoration Information resource by replacing it with the received restoration information and responds with "204 No Content".

2b.	If the request can't be accepted (e.g. the S-CSCF is not allowed to delete the restoration information), HTTP status code "403 Forbidden" should be returned including additional error information in the response body (in the "ProblemDetails" element).

2c.	If there is no valid S-CSCF restoration information, HTTP status code "404 Not Found" should be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the DELETE response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

#### 5.2.2.9	AsInfoGet

##### 5.2.2.9.1	General

The AsInfoGet service operation is invoked by a NF to retrieve the information of the IMS AS served the specific subscriber in HSS.

NF Consumer is NEF.

The following procedures using the AsInfoGet service operation are supported:

-	IMS AS Information Retrieval

##### 5.2.2.9.2	IMS AS Information Retrieval

Figure 5.2.2.9.2-1 shows a scenario where the NF service consumer (NEF) sends a request to the HSS to retrieve the information related to a specific registered user.The request contains the IMS UE's identity (/{imsUeId}) which shall be an IMPU.

Figure 5.2.2.9.2-1: IMS AS Information Retrieval

1.	The NF service consumer (NEF) sends a GET request to the resource representing the UE's IMS AS registration information.

2a.	On success, the HSS responds with "200 OK" with the message body containing the UE's IMS AS registration information.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

-	If there is no valid IMS AS registration data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

2c.	In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

#### 5.2.2.10	IndividualImsasRegistrationInfoGet

##### 5.2.2.10.1	General

The IndividualImsasRegistrationInfoGet service operation is invoked by a NF service consumer (NEF) to retrieve registration information of a given IMS AS in HSS.

The following procedures using the IndividualImsasRegistrationInfoGet service operation are supported:

-	Individual IMS AS Registration Information Retrieval

##### 5.2.2.10.2	Individual IMS AS Registration Information Retrieval

Figure 5.2.2.10.2-1 shows a scenario where the NF service consumer (NEF) sends a request to the HSS to retrieve the information related to a specific IMS AS registration. The request contains the IMS UE's identity (/{imsUeId}) which shall be an IMPU and the IMS AS's instance identity (/{instanceId}).

Figure 5.2.2.10.2-1: IMS AS Registration Information Retrieval

1.	The NF service consumer (NEF) sends a GET request to the resource representing the IMS AS Registration information.

2a.	On success, the HSS responds with "200 OK" with the message body containing the IMS AS registration information.

2b.	If there is no valid IMS AS registration information, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

#### 5.2.2.11	AsReassignmentNotification

##### 5.2.2.11.1	General

The following procedure using the AsReassignmentNotification service operation is supported:

-	HSS initiated IMS AS reassignment notification

##### 5.2.2.11.2	HSS initiated IMS AS Reassignment Notification

Figure 5.2.2.11.2-1 shows a scenario where the HSS notifies the registered IMS AS about a new IMS AS being assigned for new sessions. The request contains the AsReassignmentCallbackUri URI as received by the HSS during registration of the IMS AS and IMS AS reassignment data.

Figure 5.2.2.11.2-1: HSS initiated IMS AS Reassignment Notification

1.	The HSS sends a POST request to the asReassignmentCallbackUri as provided by the NF service consumer during the registration.

2a.	On success, the IMS AS responds with "204 No Content". The IMS AS checks if there are ongoing sessions and deregisters from HSS when all sessions for this UE using IMS AS Deregistration operation and clearing locally all IMS previously subscribed, if any,

2b.	On failure or redirection, one of the appropriate HTTP status code listed in Table 6.1.5.4-3 shall be returned. For a 4xx/5xx response, the message body may contain appropriate additional error information.

## 5.3	Nhss_imsSubscriberDataManagement Service

### 5.3.1	Service Description

See 3GPP TS 23.228 [6], clause AA.2.1.3.

### 5.3.2	Service Operations

#### 5.3.2.1	Introduction

For the Nhss_imsSubscriberDataManagement service the following service operations are defined:

-	Get

-	Subscribe to notification of data changes and UE reachability for IP

-	Unsubscribe to notification of data changes and UE reachability for IP

-	Notification of data changes and UE reachability for IP

-	Update

The Nhss_imsSubscriberDataManagement Service is used by Consumer NFs (S-CSCF, IMS-AS and DCSF) to retrieve the UE's individual subscription data relevant to the consumer NF, or by I-CSCF to retrieve S-CSCF capabilities or the assigned S-CSCF, from the SBI capable HSS by means of the Get service operation. If the consumer NF supports the feature "sharedData", the retrieved individual subscription data for the UE may contain shared data identifier identifying additional parts of subscription data shared by several UEs. If so, the Nhss_imsSubscriberDataManagement Service is also used by Consumer NFs to:

-	retrieve shared subscription data from the HSS by means of the Get service operation.

The Nhss_imsSubscriberDataManagement Service is also used by Consumer NFs to:

-	subscribe to notifications of data change by means of the Subscribe service operation. If the consumer NF supports the feature "sharedData", the consumer NF may also subscribe to notifications of shared data change by means of the Subscribe service operation.

-	to unsubscribe from notifications of data changes by means of the Unsubscribe service operation. If the feature "sharedData" is supported, it may also be used to unsubscribe from notifications of shared data changes by means of the Unsubscribe service operation.

-	get notified by means of the Notification service operation when HSS decides to modify the subscribed data. If the feature "sharedData" is supported by the consumer NF and the consumer NF has previously subscribed to notifications of shared data change, it is also used by the consumer NF to get notified by means of the Notification service operation when the HSS decides to modify the subscribed shared data.

It is also used by Consumer NFs to:

-	provide the information about the status of the subscription data management procedures.

-	update information (e.g. transparent data) associated to the subscription data management.

#### 5.3.2.2	Get

##### 5.3.2.2.1	General

The following procedures using the Get service operation are supported:

-	Repository Data Retrieval

-	IMS Identity Data (e.g. IMS associated identities including Implicit Registration Set and Alias Group) Retrieval

-	IMS Profile Data (e.g. IFCs) Retrieval

-	IMS Location Data Retrieval

-	IMS Registration Status Data Retrieval

-	Access Data (e.g. T-ADS) Retrieval

-	SRVCC Data (e.g. STN-SR) Retrieval

-	Service Data (e.g. DSAI) Retrieval

-	Retrieval Of Multiple Data Sets

-	Shared Subscription Data Retrieval

-	Trace Data Retrieval

-	PSI Activation State Retrieval

-	DSAI information Retrieval

-	S-CSCF Selection Assistance Information Retrieval

When the feature SharedData is supported and the retrieved UE-individual data (i.e. data other than Shared Subscription Data) contain SharedDataIds, the NF service consumer shall also retrieve the shared data identified by the received shared data Ids unless the identified shared data are already available at the NF service consumer. The order of sequence of sharedDataIds within UE-individual data is significant: Individual data take precedence over shared data; shared data "SharedDataX" identified by a sharedDataId X takes precedence over shared data "SharedDataY" identified by a sharedDataId Y if X appears before Y within the list of SharedDataIds in the UE-individual data.

##### 5.3.2.2.2	Identity Data

5.3.2.2.2.1	IMS Identity Data Retrieval

Figure 5.3.2.2.2.1-1 shows a scenario where the NF service consumer (IMS-AS) sends a request to the HSS to receive the UE's IMS Identity Data for the IMS associated Identities (IRS and Alias group information). The request contains the UE's identity (/{imsUeId}), the type of the requested information (/identities/ims-associated-identities) and query parameters (e.g. supported-features).

Figure 5.3.2.2.2.1-1: IMS Identity Data Retrieval

1.	The NF service consumer (IMS-AS) sends a GET request to the resource representing the UE's Identity Data for the IMS associated identities with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the UE's Implicit Registration Set (if any) and the Alias Group information (if any) which the UE's identity belongs to as relevant for the requesting NF service consumer. Additional information (e.g. distinct or wildcarded type of identity, registration state of the IRS) is also included in the response.

2b.	If there is no valid subscription data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

5.3.2.2.2.2	MSISDN(s) Retrieval

Figure 5.3.2.2.2.2-1 shows a scenario where the NF service consumer (IMS-AS) sends a request to the HSS to receive the UE's MSISDN(s). The request contains the UE's identity (/{imsUeId}), the type of the requested information (/identities/msisdns) and query parameters (e.g. supported-features).

Figure 5.3.2.2.2.2-1: MSISDN(s) Retrieval

1.	The NF service consumer (IMS-AS) sends a GET request to the resource representing the UE's MSISDN(s) with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the UE's MSISDN(s) as relevant for the requesting NF service consumer. The type of MSISDN (basic/C-MSISDN or additional MSISDN) is included in the response.

2b.	If there is no valid subscription data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

5.3.2.2.2.3	Private Identities Retrieval

Figure 5.3.2.2.2.3-1 shows a scenario where the NF service consumer (IMS-AS) sends a request to the HSS to receive the UE's Private Identities (IMS Private User Identity, IMSI). The request contains the UE's identity (/{imsUeId}), the type of the requested information (/identities/private-identities) and query parameters (e.g. supported-features).

Figure 5.3.2.2.2.3-1: Private Identities Retrieval

1.	The NF service consumer (IMS-AS) sends a GET request to the resource representing the UE's Private Identities with query optional parameters indicating the supported-features and/or impi.

2a.	On success, the HSS responds with "200 OK" with the message body containing the UE's Private Identities as relevant for the requesting NF service consumer. The type of Private Identity (e.g. IMPI, IMSI) is included in the response. If an impi was provided as query parameter in the GET request, only the Private Identity of type IMSI that is associated with the provided impi shall be returned.

2b.	If there is no valid subscription data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

5.3.2.2.2.4	IMEI(SV) Retrieval

Figure 5.3.2.2.2.4-1 shows a scenario where the NF service consumer (e.g. IMS AS) sends a request to the HSS to receive the IMEI(SV) for the UE. The request contains the UE's identity (/{imsUeId}), the type of the requested information (/identities/imeisv) and query parameters (e.g. supported-features).

Figure 5.3.2.2.2.4-1: IMEI(SV) Retrieval

1.	The NF service consumer (e.g. IMS AS) sends a GET request to the resource representing the UE's IMEI(SV) information with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the last known IMEI(SV) associated to the UE.

2b.	If there is no valid subscription data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.2.3	IMS Location Data and Status

5.3.2.2.3.1	S-CSCF Capabilities Retrieval

Figure 5.3.2.2.3.1-1 shows a scenario where the NF service consumer (I-CSCF), after authorization information has been performed without obtaining an S-CSCF name, sends a request to the HSS to receive the UE's S-CSCF capabilities (mandatory and/or optional). The request contains the UE's identity (/{imsUeId}), the type of the requested information (/ims-data/location-data/scscf-capabilities) and query parameters (e.g. supported-features).

Figure 5.3.2.2.3.1-1: S-CSCF Capabilities Retrieval

1.	The NF service consumer (I-CSCF) sends a GET request to the resource representing the UE's S-CSCF capabilities with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the UE's S-CSCF capabilities relevant for the requesting NF service consumer. The type of S-CSCF capabilities (mandatory and/or optional) is included in the response.

2b.	If there is no valid subscription data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

5.3.2.2.3.2	Server Name Retrieval

Figure 5.3.2.2.3.2-1 shows a scenario where the NF service consumer (e.g. AS) sends a request to the HSS to receive the Server Name (S-CSCF name) associated to the UE. The request contains the UE's identity (/{imsUeId}), the type of the requested information (ims-data/location-data/server-name) and query parameters (e.g. supported-features).

Figure 5.3.2.2.3.2-1: Server Name Retrieval

1.	The NF service consumer (e.g. AS) sends a GET request to the resource representing the UE's IMS location information with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the IMS location information (S-CSCF name) as relevant for the service consumer.

2b.	If there is no valid data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

5.3.2.2.3.3	IMS Registration Status Retrieval

Figure 5.3.2.2.3.3-1 shows a scenario where the NF service consumer (e.g. AS) sends a request to the HSS to IMS registration status of a user. The request contains the UE's identity (/{imsUeId}), the type of the requested information (ims-data/registration-status) and query parameters (e.g. supported-features).

Figure 5.3.2.2.3.3-1: IMS Registration Status Retrieval

1.	The NF service consumer (e.g. AS) sends a GET request to the resource representing the UE's IMS registration status with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the IMS registration status (e.g. unregistered) as relevant for the service consumer.

2b.	If there is no valid data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

5.3.2.2.3.4	S-CSCF Selection Assistance Information Retrieval

Figure 5.3.2.2.3.4-1 shows a scenario where the NF service consumer (I-CSCF), sends a request to the HSS to receive the UE's S-CSCF Selection Assistance Information. The request contains the UE's identity (/{imsUeId}), the type of the requested information (/ims-data/location-data/scscf-selection-assistance-info) and query parameters (e.g. supported-features).

5.3.2.2.3.4-1: S-CSCF Selection assistance Information Retrieval

1.	The NF service consumer (I-CSCF) sends a GET request to the resource representing the UE's S-CSCF Selection Assistance Information with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the UE's S-CSCF Selection Assistance Information.

2b.	If there is no valid subscription data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.2.4	IMS Profile Data

5.3.2.2.4.1	IMS User Profile Retrieval

Figure 5.3.2.2.4.1-1 shows a scenario where the NF service consumer (S-CSCF) sends a request to the HSS to receive the UE's IMS profile data. The request contains the UE's identity (/{imsUeId}), the type of the requested information (/ims-data/profile-data) and query parameters (e.g. supported-features).

Figure 5.3.2.2.4.1-1: IMS User Profile Retrieval

1.	The NF service consumer (S-CSCF) sends a GET request to the resource representing the UE's IMS profile with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the UE's Profile Data as relevant for the requesting NF service consumer. The Initial Filter Criteria (IFC) is included in the response. Additional information (e.g. service priority information, chargingInfo, serviceLevelTraceInfo, maxAllowedSimulReg), if any, may also be included.

2b.	If there is no valid subscription data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

5.3.2.2.4.2	Service Priority Level Information Retrieval

Figure 5.3.2.2.4.2-1 shows a scenario where the NF service consumer (e.g. AS) sends a request to the HSS to receive the Service Priority levels allowed for the UE. The request contains the UE's identity (/{imsUeId}), the type of the requested information (ims-data/profile-data/priority-levels) and query parameters (e.g. supported-features).

Figure 5.3.2.2.4.2-1: Service Priority Level Information Retrieval

1.	The NF service consumer (e.g. AS) sends a GET request to the resource representing the UE's Service Priority Level with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the Namespaces and priority levels allowed for the user.

2b.	If there is no valid subscription data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

5.3.2.2.4.3	Initial Filter Criteria Retrieval

Figure 5.3.2.2.4.3-1 shows a scenario where the NF service consumer (e.g. AS, S-CSCF) sends a request to the HSS to receive the UE's Initial Filter Criteria. The request contains the UE's identity (/{imsUeId}), the type of the requested information (ims-data/profile-data/ifcs) and query parameters (e.g. supported-features).

Figure 5.3.2.2.4.3-1: Initial Filter Criteria Retrieval

1.	The NF service consumer (e.g. AS) sends a GET request to the resource representing the UE's IMS profile with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the UE's Initial Filter Criteria (possibly containing S-CSCF-Specific IFC Set Identifiers, if supported and configured in S-CSCF) as relevant for the requesting NF service consumer.

2b.	If there is no valid subscription data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

5.3.2.2.4.4	Service Trace Level Information Retrieval

Figure 5.3.2.2.4.4-1 shows a scenario where the NF service consumer (e.g. AS) sends a request to the HSS to receive the UE's IMS service level trace information associated to the UE. The request contains the UE's identity (/{imsUeId}), the type of the requested information (ims-data/profile-data/ service-level-trace-info) and query parameters (e.g. supported-features).

Figure 5.3.2.2.4.4-1: Service Trace Level Information Retrieval

1.	The NF service consumer (e.g. AS) sends a GET request to the resource representing the UE's IMS service level trace information with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the IMS service level trace information (see 3GPP TS 24.323 [24]) so that the service consumer performs logging at IMS service level as described in IETF RFC 8497 [23].

2b.	If there is no valid subscription data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

5.3.2.2.4.5	Charging Information Retrieval

Figure 5.3.2.2.4.5-1 shows a scenario where the NF service consumer (e.g. AS) sends a request to the HSS to receive the UE's charging information. The request contains the UE's identity (/{imsUeId}), the type of the requested information (ims-data/profile-data/ charging-info) and query parameters (e.g. supported-features).

Figure 5.3.2.2.4.5-1: Charging Information Retrieval

1.	The NF service consumer (e.g. AS) sends a GET request to the resource representing the UE's charging information with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the ChargingInfo.

2b.	If there is no valid subscription data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.2.5	Access Data

5.3.2.2.5.1	Location Information Retrieval for CS/PS Domains

Figure 5.3.2.2.5.1-1 shows a scenario where the NF service consumer (e.g. AS) sends a request to the HSS to receive the Location Information associated to the UE The request contains the UE's identity (/{imsUeId}), the type of the requested information, the domain (cs-domain or ps-domain) requested (e.g. ims-data/access-data/ps-domain/location-data) and query parameters (e.g. supported-features, requested-nodes).

Figure 5.3.2.2.5.1-1: Location Information Retrieval for CS/PS Domains

1a.	The NF service consumer (e.g. AS) sends a GET request to the resource representing the UE's location information at PS domain with query parameters indicating the supported-features and optionally the requested nodes (e.g. MME, AMF)

1b.	The NF service consumer (e.g. AS) sends a GET request to the resource representing the UE's location information at CS domain with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the location information for the requested domain (PS) and the requested nodes or all nodes (if no requested nodes were present in the request)

2b.	On success, the HSS responds with "200 OK" with the message body containing the location information for the requested domain (CS).

2c.	If there is no valid subscription data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

5.3.2.2.5.2	IP Address Information Retrieval

Figure 5.3.2.2.5.2-1 shows a scenario where the NF service consumer (e.g. SCC-AS) sends a request to the HSS to receive the IP Address of the UE. The request contains the UE's identity (/{imsUeId}), the type of the requested information (/access-data/ps-domain/ip-address) and query parameters (e.g. supported-features).

Figure 5.3.2.2.5.2-1: IP Address Information Retrieval

1.	The NF service consumer (e.g. SCC-AS) sends a GET request to the resource representing the UE's IP address information with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing either an IPv4 address, IPv6 address or IPv6 prefix and/or IPv6 interface identifier.

2b.	If there is no valid subscription data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

5.3.2.2.5.3	T-ADS Information Retrieval

Figure 5.3.2.2.5.3-1 shows a scenario where the NF service consumer (e.g. SCC-AS) sends a request to the HSS to receive the T-ADS information for the UE. The request contains the UE's identity (/{imsUeId}), the type of the requested information (/access-data/ps-domain/tads-info) and query parameters (e.g. supported-features).

Figure 5.3.2.2.5.3-1: T-ADS Information Retrieval

1.	The NF service consumer (e.g. SCC-AS) sends a GET request to the resource representing the UE's T-ADS information with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the most recent IMS Voice over PS Sessions support indication as indicated by the serving nodes.for the user.

2b.	If there is no valid subscription data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

5.3.2.2.5.4	User State Retrieval for CS/PS Domains

Figure 5.3.2.2.5.4-1 shows a scenario where the NF service consumer (e.g. AS) sends a request to the HSS to receive the User State associated to the UE The request contains the UE's identity (/{imsUeId}), the type of the requested information, the domain (cs-domain or ps-domain) requested (e.g. ims-data/access-data/ps-domain/user-state) and query parameters (e.g. supported-features, requested-nodes).

Figure 5.3.2.2.5.4-1: User State Retrieval for CS/PS Domains

1a.	The NF service consumer (e.g. AS) sends a GET request to the resource representing the UE's user state information at PS domain with query parameters indicating the supported-features and optionally the requested nodes (e.g. MME, AMF)

1b.	The NF service consumer (e.g. AS) sends a GET request to the resource representing the UE's User State information at CS domain with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the User State information for the requested domain (PS) and the requested nodes or all nodes (if no requested nodes were present in the request)

2b.	On success, the HSS responds with "200 OK" with the message body containing the User State information for the requested domain (CS).

2c.	If there is no valid subscription data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

5.3.2.2.5.5	CSRN Retrieval

Figure 5.3.2.2.5.5-1 shows a scenario where the NF service consumer (e.g. SCC-AS) sends a request to the HSS to receive a CS Domain Routeing Number (CSRN, see 3GPP TS 23.003 [13]) for the UE. The request contains the UE's identity (/{imsUeId}), the type of the requested information (/access-data/cs-domain/csrn) and query parameters (e.g. supported-features, pre-paging).

Figure 5.3.2.2.5.5-1: T-ADS Information Retrieval

1.	The NF service consumer (e.g. SCC-AS) sends a GET request to the resource representing the UE's CSRN information with query parameters indicating the supported-features and/or pre-paging indication.

2a.	On success, the HSS responds with "200 OK" with the message body containing the CSRN as provided by the MSC/VLR.

2b.	If there is no valid subscription data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

5.3.2.2.5.6	Reference Location Information Retrieval

Figure 5.3.2.2.5.6-1 shows a scenario where the NF service consumer (e.g. IMS-AS) sends a request to the HSS to receive the reference location information for the UE. The request contains the UE's identity (/{imsUeId}), the type of the requested information (/access-data/wireline-domain/reference-location) and query parameters (e.g. supported-features).

Figure 5.3.2.2.5.6-1: Reference Location Information Retrieval

1.	The NF service consumer (e.g. IMS-AS) sends a GET request to the resource representing the UE's reference location information with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the access type and/or the access info and associated value (line identifier in fixed access networks).

2b.	If there is no valid subscription data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.2.6	SRVCC Data

5.3.2.2.6.1	SRVCC Data Retrieval

Figure 5.3.2.2.6.1-1 shows a scenario where the NF service consumer (e.g. AS) sends a request to the HSS to retrieve the SRVCC data for the UE. The request contains the UE's identity (/{imsUeId}), the type of the requested information (/srvcc-data) and query parameters (e.g. supported-features).

Figure 5.3.2.2.6.2-1: SRVCC Data Retrieval

1.	The NF service consumer (e.g. AS) sends a GET request to the resource representing the UE's SRVCC data information with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the STN-SR and the UE SRVCC capability for the user.

2b.	If there is no valid SRVCC data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.2.7	Service Data

5.3.2.2.7.1	PSI Activation State Retrieval

Figure 5.3.2.2.7.1-1 shows a scenario where the NF service consumer (e.g. AS) sends a request to the HSS to retrieve the PSI activation state. The request contains the Public Service Identity (/{imsUeId}), the type of the requested information (/service-data/psi-status) and query parameters (e.g. supported-features).

Figure 5.3.2.2.7.1-1: PSI Activation State Retrieval

1.	The NF service consumer (e.g. AS) sends a GET request to the resource representing the PSI activation state with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the PSI activation state (active or inactive).

2b.	If there is no valid activation state for the PSI, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

5.3.2.2.7.2	SMS Registration Information retrieval

Figure 5.3.2.2.7.2-1 shows a scenario where the NF service consumer (e.g. IP-SM-GW) sends a request to the HSS to retrieve the SMS registration information associated to a UE. The request contains the UE identity (/{imsUeId}), the type of the requested information (/service-data/sms-registration-info) and query parameters (e.g. supported-features).

Figure 5.3.2.2.7.2-1: SMS Registration Information Retrieval

1.	The NF service consumer (e.g. IP-SM-GW) sends a GET request to the resource representing the SMS registration information with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the IP-SM-GW number and optionally the SC address.

2b.	If there is no valid SMS registration information, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

5.3.2.2.7.3	DSAI Information retrieval

Figure 5.3.2.2.7.3-1 shows a scenario where the NF service consumer (e.g. IMS-AS) sends a request to the HSS to retrieve the DSAI information associated to an Application Server. The request contains the UE identity (/{imsUeId}), the type of the requested information (/service-data/dsai) and query parameters (e.g. Application Server Name).

Figure 5.3.2.2.7.3-1: DSAI Information Retrieval

1.	The NF service consumer (e.g. IMS-AS) sends a GET request to the resource representing the DSAI information with query parameter indicating the Application Server Name. Additional query parameters (e.g.supported-features) may be included.

2a.	On success, the HSS responds with "200 OK" with the message body containing the DSAI tags and their respective status associated to the received Application Server Name.

2b.	If there are no DSAI tags for the Application Server, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.2.8	Repository Data

5.3.2.2.8.1	Repository Data Retrieval

Figure 5.3.2.2.8.1-1 shows a scenario where the NF service consumer (IMS-AS or DCSF) sends a request to the HSS to receive the UE's Repository Data for a given Service Indication. The request contains the UE's identity (/{imsUeId}), the type of the requested information (/repository-data/{serviceIndication}) and query parameters (e.g. supported-features).

Figure 5.3.2.2.8.1-1: Repository Data Retrieval

1.	The NF service consumer sends a GET request to the resource representing the UE's Repository Data for a given Service Indication, with query parameters indicating the supported-features.

2a.	On success, the HSS responds with "200 OK" with the message body containing the UE's Repository Data for the requested Service Indication as relevant for the requesting NF service consumer.

2b.	If there is no valid subscription data or Repository Data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

5.3.2.2.8.2	Repository Data Retrieval for multiple service indications

Figure 5.3.2.2.8.2-1 shows a scenario where the NF service consumer (IMS-AS or DCSF) sends a request to the HSS to receive the UE's Repository Data for multiple Service Indications. The request contains the UE's identity (/{imsUeId}), the type of the requested information (/repository-data) and query parameters (e.g. list of service indications, supported-features).

Figure 5.3.2.2.8.2-1: Repository Data Retrieval for multiple service indications

1.	The NF service consumer sends a GET request to the resource representing the UE's Repository Data, with query parameters indicating the supported-features and list of service indications.

2a.	On success, the HSS responds with "200 OK" with the message body containing the UE's Repository Data for the requested Service Indications as relevant for the requesting NF service consumer.

2b.	If there is no valid subscription data or Repository Data for the UE, HTTP status code "404 Not Found" shall be returned and it should include additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.2.9	Shared Subscription Data

5.3.2.2.9.1	Shared Subscription Data Retrieval

Figure 5.3.2.2.9.1-1 shows a scenario where the NF service consumer (e.g.S-CSCF) sends a request to the HSS to receive the shared subscription data. The request contains the type of the requested information (/shared-data) and query parameters (supportedFeatures, shared-data-id).

Figure 5.3.2.2.9.1-1: Shared Subscription Data Retrieval

1.	The NF service consumer (e.g. S-CSCF) sends a GET request to the resource representing the SharedData, with query parameters indicating the supportedFeatures and shared-data-id.

2a.	On success, the HSS responds with "200 OK" with the message body containing the SharedData.

2b.	If there is no valid shared data for one or more of the shared-data-ids, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

#### 5.3.2.3	Subscribe

##### 5.3.2.3.1	General

The following procedures using the Subscribe service operation are supported:

-	Subscription to notification of data change

-	Subscription to notification of UE reachability for IP

##### 5.3.2.3.2	Subscription to notifications of data change

Figure 5.3.2.3.2-1 shows a scenario where the NF service consumer (e.g. S-CSCF) sends a request to the HSS to subscribe to notifications of data change. The request contains a callback URI and the URI of the monitored resource.

Figure 5.3.2.3.2-1: NF service consumer subscribes to notifications

1.	The NF service consumer sends a POST request to the parent resource (collection of subscriptions) (.../{imsUeId}/sdm-subscriptions), to create a subscription as present in message body.

2a.	On success, the HSS responds with "201 Created" with the message body containing a representation of the created subscription. The Location HTTP header shall contain the URI of the created subscription.

2b.	If there is no valid subscription data for the UE, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in the "ProblemDetails" element).

2c.	If the UE subscription data exist, but the requested subscription to data change notification cannot be created (e.g. due to an invalid/unsupported data reference to be monitored, contained in the ImsSdmSubscription parameter), HTTP status code "501 Not Implemented" shall be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.3.3	Subscription to notifications of shared data change

Figure 5.3.2.3.3-1 shows a scenario where the NF service consumer sends a request to the HSS to subscribe to notifications of shared data change. The request contains a callback URI and the URI of the monitored resource.

Figure 5.3.2.3.3-1: NF service consumer subscribes to notifications of shared data change

1.	The NF service consumer sends a POST request to the parent resource (collection of subscriptions) (.../shared-data-subscriptions), to create a subscription as present in message body.

2a.	On success, the HSS responds with "201 Created" with the message body containing a representation of the created subscription. The Location HTTP header shall contain the URI of the created subscription.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.3.4	Subscriptions to notification of UE Reachability for IP

Figure 5.3.2.3.4-1 shows a scenario where the NF service consumer (e.g. IP-SM-GW) sends a request to the HSS to subscribe to UE reachability for IP. The request contains the UE's identity (/{imsUeId}), the resource to be monitored (/access-data/ps-domain/ue-reach-subscriptions), an expiry time, a callback URI and query parameters (e.g. supported-features).

Subscriptions to notification on UE Reachability for IP are requests for a one-time notification, i.e. after notification the subscription becomes invalid.

Figure 5.3.2.3.4-1: Subscription to notification of UE reachability for IP

1.	The NF service consumer (e.g. IP-SM-GW) sends a POST request to the parent resource (collection of UE reachability subscriptions) (.../{ueIdentity}/ue-reach-subscriptions), to create a subscription to UE reachability for IP. The request contains an expiry time, suggested by the NF Service Consumer, representing the time upto which the subscription is desired to be kept active and the time after which the UE reachability event shall not generate a notification.

2a.	On success, the HSS sets the URRP flag for the relevant serving nodes and may contact UDM (see 3GPP TS 23.632, clause 5.4.5). The HSS responds with "201 Created" with the message body containing a representation of the created subscription. The Location HTTP header shall contain the URI of the created subscription.

The response contains the expiry time, as determined by the HSS (possible based on operator policy) after which the subscription becomes invalid. Once the subscription expires, if the NF Service Consumer wants to keep receiving notifications, it shall create a new subscription in the HSS. The NF Service Producer shall not provide the same expiry time for many subscriptions in order to avoid all of them expiring and recreating the subscription at the same time.

2b.	If the user does not exist, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

#### 5.3.2.4	Unsubscribe

##### 5.3.2.4.1	General

The following procedures using the Unsubscribe service operation are supported:

-	Unsubscribe to notification of data change

-	Unsubscribe to notification of UE reachability for IP

##### 5.3.2.4.2	Unsubscribe to notifications of data change

Figure 5.3.2.4.2-1 shows a scenario where the NF service consumer (e.g. S-CSCF) sends a request to the HSS to unsubscribe from notifications of data changes. The request contains the URI previously received in the Location HTTP header of the response to the subscription.

Figure 5.3.2.4.2-1: NF service consumer unsubscribes to notifications

1.	The NF service consumer sends a DELETE request to the resource identified by the URI previously received during subscription creation.

2a.	On success, the HSS responds with "204 No Content".

2b.	If there is no valid subscription available (e.g. due to an unknown subscriptionId value), HTTP status code "404 Not Found" should be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the DELETE response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.4.3	Unsubscribe to notifications of shared data change

Figure 5.3.2.4.3-1 shows a scenario where the NF service consumer sends a request to the HSS to unsubscribe from notifications of shared data changes. The request contains the URI previously received in the Location HTTP header of the response to the subscription.

Figure 5.3.2.4.3-1: NF service consumer unsubscribes to notifications for shared data

1.	The NF service consumer sends a DELETE request to the resource identified by the URI previously received during subscription creation.

2a.	On success, the HSS responds with "204 No Content".

2b.	If there is no valid subscription available (e.g. due to an unknown subscriptionId value), HTTP status code "404 Not Found" should be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the DELETE response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.4.4	Unsubscribe to notification of UE Reachability for IP

Figure 5.3.2.4.4-1 shows a scenario where the NF service consumer (e.g. IP-SM-GW) sends a request to the HSS to unsubscribe from notifications of UE Reachability for IP. The request contains the URI previously received in the Location HTTP header of the response to the subscription.

Figure 5.3.2.4.4-1: Unsubscription to notification of UE reachability for IP

1.	The NF service consumer sends a DELETE request to the resource identified by the URI previously received during subscription creation.

2a.	On success, the HSS responds with "204 No Content".

2b.	If there is no valid subscription available (e.g. due to an unknown subscriptionId value), HTTP status code "404 Not Found" should be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the DELETE response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

#### 5.3.2.5	ModifySubscription

##### 5.3.2.5.1	General

The following procedures using the ModifySubscription service operation are supported:

-	Modification of a Subscription to notification of data change (for UE individual data)

-	Modification of a Subscription to notification of shared data change

-	Modification of a Subscription to notification of UE Reachability for IP

The ModifySubscription service operation can be used for the following purpose:

-	Extend the expiry time of ImsSdmSubscription;

-	Extend the expiry time of UeReachabilitySubscription;

-	Modify the resource URIs to be monitored, e.g. add/remove resource URIs to/from the monitored resource URI list.

##### 5.3.2.5.2	Modification of a subscription to notifications of data change

Figure 5.3.2.5.2-1 shows a scenario where the NF service consumer sends a request to the HSS to modify a subscription to notifications of data changes. The request contains the URI previously received in the Location HTTP header of the response to the subscription.

Figure 5.3.2.5.2-1: NF service consumer modifies a subscription to notifications

1.	The NF service consumer sends a PATCH request to the resource identified by the URI previously received during subscription creation.

The NF service consumer may include "monitoredResourceUris" to replace the existing monitored resource URIs, e.g. to add/remove specific resource URIs from the monitored resource URI list.

2a.	On success, the HSS responds with "200 OK" or "204 No Content".

2b.	If there is no valid subscription available (e.g. due to an unknown subscriptionId value), HTTP status code "404 Not Found" should be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PATCH response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.5.3	Modification of a subscription to notifications of shared data change

Figure 5.3.2.z.3-1 shows a scenario where the NF service consumer sends a request to the HSS to modify a subscription to notifications of shared data changes. The request contains the URI previously received in the Location HTTP header of the response to the subscription.

Figure 5.3.2.5.3-1: NF service consumer modifies a subscription to notifications for shared data

1.	The NF service consumer sends a PATCH request to the resource identified by the URI previously received during subscription creation.

The NF service consumer may include "monitoredResourceUris" to replace the existing monitored resource URIs, e.g. for the purposes to add/remove specific resource URIs from the monitored resource URI list.

2a.	On success, the HSS responds with "200 OK" or "204 No Content".

2b.	If there is no valid subscription available (e.g. due to an unknown subscriptionId value), HTTP status code "404 Not Found" should be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PATCH response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.5.4	Modification of a subscription to notifications of data change

Figure 5.3.2.5.4-1 shows a scenario where the NF service consumer sends a request to the HSS to modify a subscription to notifications of data changes. The request contains the URI previously received in the Location HTTP header of the response to the subscription.

Figure 5.3.2.5.4-1: NF service consumer modifies a subscription to notifications

1.	The NF service consumer sends a PATCH request to the resource identified by the URI previously received during subscription creation.

The NF service consumer includes a new expiry time, e.g. to extend the monitoring time of the event.

2a.	On success, the HSS responds with "200 OK" or "204 No Content".

2b.	If there is no valid subscription available (e.g. due to an unknown subscriptionId value), HTTP status code "404 Not Found" should be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PATCH response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

#### 5.3.2.6	Notification

##### 5.3.2.6.1	General

The following procedures using the Notification service operation are supported:

-	Data change notification to NF;

-	UE Reachability for IP notification to NF.

##### 5.3.2.6.2	Data Change Notification To NF

Figure 5.3.2.6.2-1 shows a scenario where the HSS notifies the NF service consumer (that has subscribed to receive such notification) about subscription data change. The request contains the callbackReference URI as previously received in the ImsSdmSubscription.

Figure 5.3.2.6.2-1: Subscription Data Change Notification

1.	The HSS sends a POST request to the callbackReference as provided by the NF service consumer during the subscription.

2.	The NF service consumer responds with "204 No Content".

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

In the case of redirection, the NF Service Consumer shall return 3xx status code, which shall contain a Location header with an URI pointing to an alternative notification endpoint.

##### 5.3.2.6.3	UE Reachability for IP Notification to NF

Figure 5.3.2.6.3-1 shows a scenario where the HSS notifies the NF service consumer (that has subscribed to receive such notification) about subscription data change. The request contains the callbackReference URI as previously received in the UeReachabilitySubscription.

Figure 5.3.2.6.3-1: UE Reachability for IP Notification

1.	The HSS sends a POST request to the callbackReference as provided by the NF service consumer during the subscription. The notification includes information about UE reachability for IP event detected.

2.	The NF service consumer responds with "204 No Content".

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

In the case of redirection, the NF Service Consumer shall return 3xx status code, which shall contain a Location header with an URI pointing to an alternative notification endpoint.

#### 5.3.2.7	Update

##### 5.3.2.7.1	General

The following procedures using the Update service operation are supported:

-	Repository Data Create/Update/Delete

-	STN-SR update

-	SMS registration information Create/Update/Delete

-	PSI activation state Update

-	DSAI-tag activation state Update

##### 5.3.2.7.2	Repository Data Creation or Update

Figure 5.3.2.7.2-1 shows a scenario where the NF service consumer (IMS AS or DCSF) sends a request to the HSS to create or update the repository data for a given service indication. The request contains the UE's identity (/{imsUeId}) which shall be an IMS Public Identity, the service indication and repository data with the associated version (sequence number).

Figure 5.3.2.7.2-1: Creating or updating repository data

1.	The NF service consumer sends a PUT request to the resource representing the UE's Repository Data for a requested Service Indication.

The NF service consumer may request the creation of the Repository Data (sequence number equal to zero) or may request the update of the Repository Data (sequence number greater than zero)

2a.	On success, the HSS updates the RepositoryData resource by replacing it with the received resource information, and responds with "200 OK" or "204 No Content".

2b.	If the resource does not exist (there is no repository data associated to the requested service indication), HSS stores the received Repository Data and responds with HTTP Status Code "201 created". A response body may be included to convey additional information to the NF consumer (e.g., features supported by HSS).

2c.	If the operation is not authorized due to e.g the NF service consumer does not have the required permissions for the service indication and the operation requested, HTTP status code "403 Forbidden" should be returned including additional error information in the response body (in "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PUT response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.7.3	Repository Data Deletion

Figure 5.3.2.7.3-1 shows a scenario where the NF service consumer sends a request to the HSS to delete the repository data for a given service indication. The request contains the UE's identity (/{imsUeId}) which shall be an IMS Public Identity and the service indication.

Figure 5.3.2.7.3-1: Deleting repository data

1.	The NF service consumer sends a DELETE request to the resource representing the UE's Repository Data for a requested Service Indication.

2a.	On success, the HSS deletes the resource and responds with "204 No Content".

2b.	If the repository data does not exist in the HSS, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in the "ProblemDetails" element).

2c.	If the operation is not authorized due to e.g the NF service consumer does not have the required permissions for the service indication and operation requested, HTTP status code "403 Forbidden" should be returned including additional error information in the response body (in "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the DELETE response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.7.4	STN-SR update

Figure 5.3.2.7.4-1 shows a scenario where the IMS-AS sends a request to the HSS to update the STN-SR. The request contains the UE's identity (/{imsUeId}) which shall be an IMS Public Identity or a Private Identity and an instruction to modify the STN-SR.

Figure 5.3.2.7.4-1: IMS AS updating STN-SR

1.	The IMS AS sends a PATCH request to the resource representing the UE's SRVCC Data.

2a.	On success, the HSS responds with "204 No Content".

2b.	If the resource does not exist (i.e. the user is not subscribed to SRVCC), HTTP status code "404 Not Found" should be returned including additional error information in the response body (in the "ProblemDetails" element).

2c.	If the operation is not authorized due to e.g IMS AS does not have the required permissions for the operation requested, HTTP status code "403 Forbidden" should be returned including additional error information in the response body (in "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PATCH response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.7.5	PSI activation state update

Figure 5.3.2.7.5-1 shows a scenario where the IMS-AS sends a request to the HSS to update the PSI activation state. The request contains the Public Service Identity (/{imsUeId}) and an instruction to modify the activation state.

Figure 5.3.2.7.5-1: IMS AS updating a PSI activation state

1.	The IMS AS sends a PATCH request to the resource representing the PSI activation state.

2a.	On success, the HSS responds with "204 No Content".

2b.	If the resource does not exist (i.e. the service does not exist), HTTP status code "404 Not Found" should be returned including additional error information in the response body (in the "ProblemDetails" element).

2c.	If the operation is not authorized due to e.g IMS AS does not have the required permissions for the operation requested, HTTP status code "403 Forbidden" should be returned including additional error information in the response body (in "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PATCH response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.7.6	SMS Registration Information Creation or Update

Figure 5.3.2.7.6-1 shows a scenario where the IMS-AS sends a request to the HSS to create or update the SMS registration information. The request contains the UE's identity (/{imsUeId}) which shall be an IMS Public Identity and the SMS registration information (IP-SM-GW number).

Figure 5.3.2.7.6-1: IMS AS creating or updating the SMS registration information

1.	The IMS AS sends a PUT request to the resource representing the UE's SMS registration information. The request contains the IP-SM-GW number and, if s6c is supported, the Diameter Identity and Diameter Realm of the IP-SM-GW.

If the IP-SM-GW supports SBI-based MT SM transmit, the "SBI support indication" of the IP-SM-GW shall be included in the SMS Registration Information.

2a.	On success, the HSS updates the SMS registration information resource by replacing it with the received resource information and responds with "200 OK" or "204 No Content".

2b.	If the resource does not exist (there is no IP-SM-GW number associated to the user), HSS stores the received SMS registration information and responds with HTTP Status Code "201 created". A response body may be included to convey additional information to the NF consumer (e.g., features supported by HSS).

2c.	If the operation is not authorized due to e.g IMS AS does not have the required permissions or the IP-SM-GW number is pre-configured and cannot be overwritten, HTTP status code "403 Forbidden" should be returned including additional error information in the response body (in "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PUT response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.7.7	SMS Registration Information Deletion

Figure 5.3.2.7.7-1 shows a scenario where the IMS-AS sends a request to the HSS to delete the SMS registration information. The request contains the UE's identity (/{imsUeId}) which shall be an IMS Public Identity.

Figure 5.3.2.7.7-1: IMS AS deleting SMS registration information

1.	The IMS AS sends a DELETE request to the resource representing the UE's SMS registration information.

2a.	On success, the HSS deletes the resource and responds with "204 No Content".

2b.	If the SMS registration information does not exist in the HSS, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in the "ProblemDetails" element).

2c.	If the operation is not authorized due to e.g IMS AS does not have the required permissions or the IP-SM-GW number is pre-configured and cannot be deleted, HTTP status code "403 Forbidden" should be returned including additional error information in the response body (in "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the DELETE response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

##### 5.3.2.7.8	DSAI tag activation state update

Figure 5.3.2.7.8-1 shows a scenario where the IMS-AS sends a request to the HSS to update the DSAI tag activation state. The request contains the Public Service Identity (/{imsUeId}) and an instruction to modify the activation state of DSAI tag.

Figure 5.3.2.7.8-1: IMS AS updating DSAI tag activation state

1.	The IMS AS sends a PATCH request to the resource representing the DSAI tag activation state.

2a.	On success, the HSS responds with "204 No Content".

2b.	If the resource does not exist (i.e. the service does not exist), HTTP status code "404 Not Found" should be returned including additional error information in the response body (in the "ProblemDetails" element).

2c.	If the operation is not authorized due to e.g IMS AS does not have the required permissions for the operation requested, HTTP status code "403 Forbidden" should be returned including additional error information in the response body (in "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PATCH response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

## 5.4	Nhss_imsUEAuthentication Service

### 5.4.1	Service Description

See 3GPP TS 23.228 [6], clause AA.2.1.4.

### 5.4.2	Service Operations

#### 5.4.2.1	Introduction

For the Nhss_ imsUEAuthentication service the following service operation is defined:

-	Get

The Nhss_ imsUEAuthentication Service is used by the S-CSCF to request the SBI capable HSS to select an authentication method, calculate a fresh authentication vector (AV) if required for the selected method, and provide it to the S-CSCF by means of the Get service operation.

#### 5.4.2.2	Get

##### 5.4.2.2.1	General

The following procedures using the Get service operation are supported:

-	Authentication Information Retrieval

##### 5.4.2.2.2	Authentication Information Retrieval

Figure 5.4.2.2.2-1 shows a scenario where the NF service consumer (S-CSCF) retrieves authentication information for the UE from the HSS. The request contains the UE's identity (IMS Private Identity) and the SIP authentication scheme and may contain resynchronization info.

Figure 5.4.2.2.2-1: NF service consumer requesting authentication information

1.	The NF service consumer sends a POST request (custom method: generate-sip-auth-data) to the resource representing the UE's security information. The content shall contain the S-CSCF name, SIP authentication scheme and the resynchronization info.

2a.	The HSS responds with "200 OK" with the message body containing the authentication data information.

2b.	If the operation cannot be authorized due to e.g UE does not have required subcription data, HTTP status code "403 Forbidden" should be returned including additional error information in the response body (in "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

## 5.5	Nhss_gbaSubscriberDataManagement Service

### 5.5.1	Service Description

See 3GPP TS 33.220 [40], clause N.2.1.2.

### 5.5.2	Service Operations

#### 5.5.2.1	Introduction

For the Nhss_gbaSubscriberDataManagement service the following service operations are defined:

-	Get

-	Subscribe

-	Unsubscribe

-	Modification of subscription

-	Notification

The Nhss_gbaSubscriberDataManagement Service is used by Consumer NFs (GBA BSF) to:

-	fetch the GBA subscriber data for the UE

-	subscribe/unsubscribe, and to be notified, when data previously requested have changed

#### 5.5.2.2	Get

##### 5.5.2.2.1	General

The following procedures using the Get service operation are supported:

-	Retrieval of GBA subscriber data

##### 5.5.2.2.2	Retrieval of GBA subscriber data

Figure 5.5.2.2.2-1 shows a scenario where the GBA BSF sends a request to the HSS to retrieve the GBA subscriber data. The request contains the UE's identity (/{ueId}) which shall be one of IMSI, MSISDN, IMPI, IMPU.

Figure 5.5.2.2.2-1: Retrieval of GBA subscriber data

1.	The GBA BSF sends a GET request to the resource representing the UE's GBA subscriber data.

2a.	Upon success, the HSS responds with "200 OK" with the GBA subscriber data (containing, e.g. the GBA User Security Settings, GUSS) in the response body, and HSS supported features.

2b.	If the operation is not authorized due to, e.g. received UE identity not being allowed for GBA services, or the requesting node not being authorized to fetch the requested data, HTTP status code "403 Forbidden" shall be returned including additional error information in the response body (in "ProblemDetails" data structure).

2c.	If the UE identity is not found in HSS, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in "ProblemDetails" data structure).

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

#### 5.5.2.3	Subscribe

##### 5.5.2.3.1	General

The following procedures using the Subscribe service operation are supported:

-	Subscription to changes on the GBA subscriber data

##### 5.5.2.3.2	Subscription to changes on the GBA subscriber data

Figure 5.5.2.3.2-1 shows a scenario where the GBA BSF sends a request to the HSS to subscribe to changes on a previously retrieved GBA subscriber data. The request contains the UE's identity (/{ueId}) which shall be one of IMSI, MSISDN, IMPI, IMPU.

Figure 5.5.2.3.2-1: Subscription to changes on the GBA subscriber data

1.	The GBA BSF sends a POST request to the collection resource representing the subscriptions to changes on the UE's GBA subscriber data. The request body contains a URI where subsequent notification shall be sent by HSS.

2a.	Upon success, the HSS responds with "200 OK" with the GBA SDM subscription data in the response body, and HSS supported features.

2b.	If the operation is not authorized due to, e.g. received UE identity not being allowed for GBA services, or the requesting node not being authorized to subscribe to changes on GBA subscriber data, HTTP status code "403 Forbidden" shall be returned including additional error information in the response body (in "ProblemDetails" data structure).

2c.	If the UE identity is not found in HSS, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in "ProblemDetails" data structure).

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

#### 5.5.2.4	Unsubscribe

##### 5.5.2.4.1	General

The following procedures using the Unsubscribe service operation are supported:

-	Unsubscribe to changes on the GBA subscriber data

##### 5.5.2.4.2	Unsubscribe to changes on the GBA subscriber data

Figure 5.5.2.4.2-1 shows a scenario where the GBA BSF sends a request to the HSS to unsubscribe to changes on a previously retrieved GBA subscriber data. The request contains the UE's identity (/{ueId}) which shall be one of IMSI, MSISDN, IMPI, IMPU.

Figure 5.5.2.4.2-1: Deletion of a subscription to changes on the GBA subscriber data

1.	The GBA BSF sends a DELETE request to the resource representing the individual subscription (subscriptionID) to changes on the UE's GBA subscriber data.

2a.	Upon success, the HSS responds with "204 No Content".

2b.	If the operation is not authorized due to, e.g. received UE identity not being allowed for GBA services, or the requesting node not being authorized to subscribe/unsubscribe to changes on GBA subscriber data, HTTP status code "403 Forbidden" shall be returned including additional error information in the response body (in "ProblemDetails" data structure).

2c.	If the UE identity is not found in HSS, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in "ProblemDetails" data structure).

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

#### 5.5.2.5	Modification of Subscription

##### 5.5.2.5.1	General

The following procedures using the Modification of Subscription service operation are supported:

-	Modification of Subscription to changes on the GBA subscriber data

##### 5.5.2.5.2	Modification of Subscription to changes on the GBA subscriber data

Figure 5.5.2.5.2-1 shows a scenario where the GBA BSF sends a request to the HSS to modify a subscription to changes (e.g. to extend its expiration lifetime) on a previously retrieved GBA subscriber data. The request contains the UE's identity (/{ueId}) which shall be one of IMSI, MSISDN, IMPI, IMPU.

Figure 5.5.2.5.2-1: Modification of Subscription to changes on the GBA subscriber data

1.	The GBA BSF sends a PATCH request to the resource representing the individual subscription to changes on the UE's GBA subscriber data (identified by its subscriptionID).

2a.	Upon success, if some of the changes, but not all, have been accepted, the HSS responds with "200 OK" with the new GBA SDM subscription data in the response body.

2b.	Upon success, if all the changes have been accepted, the HSS responds with "204 No Content".

2c.	If the operation is not authorized due to the requesting node not being authorized to modify the subscription to changes on GBA subscriber data, HTTP status code "403 Forbidden" shall be returned including additional error information in the response body (in "ProblemDetails" data structure).

2d.	If the subscriptionId is not found in HSS, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in "ProblemDetails" data structure).

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

#### 5.5.2.5	Notify

##### 5.5.2.5.1	General

The following procedures using the Notify service operation are supported:

-	Notification of changes on the GBA subscriber data

##### 5.5.2.5.2	Notification of changes on the GBA subscriber data

Figure 5.5.2.5.2-1 shows a scenario where the HSS sends a notification to the GBA BSF to inform of changes on a previously retrieved GBA subscriber data.

Figure 5.5.2.5.2-1: Subscription to changes on of GBA User Security data

1.	The GBA BSF sends a POST request to the notification URI previously provided by the GBA BSF during the subscription request (see clause 5.5.2.3.1).

2a.	Upon success, the HSS responds with "204 No Content".

2b.	If the UE identity contained in the NotificationData is not found in GBA BSF, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in "ProblemDetails" data structure).

In the case of redirection, the GBA BSF shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another GBA BSF instance capable of handling the notification request.

## 5.6	Nhss_gbaUEAuthentication Service

### 5.6.1	Service Description

See 3GPP TS 33.220 [40], clause N.2.1.3.

### 5.6.2	Service Operations

#### 5.6.2.1	Introduction

For the Nhss_gbaUEAuthentication service the following service operations are defined:

-	Get

The Nhss_gbaUEAuthentication Service is used by Consumer NFs (GBA BSF) to:

-	request the authentication data of the UE

#### 5.6.2.2	Get

##### 5.6.2.2.1	General

The following procedures using the Get service operation are supported:

-	Request UE authentication data

##### 5.6.2.2.2	Request UE authentication data

Figure 5.6.2.2.2-1 shows a scenario where the GBA BSF sends a request to the HSS to retrieve UE authentication data (authentication vectors) for GBA. The request contains the UE's identity (/{ueId}) which shall be one of IMSI, MSISDN, IMPI, IMPU.

Figure 5.6.2.2.2-1: Request UE authentication data

1.	The GBA BSF sends a POST request (custom method: generate-auth-data) to the HSS.

2a.	Upon success, the HSS responds with "200 OK" with the AuthenticationInfoResult data structure (containing authentication vectors) in the response body, and HSS supported features.

2b.	If the operation is not authorized due to, e.g. received UE identity not being allowed for GBA services, or the requesting node not being authorized to fetch the requested data, HTTP status code "403 Forbidden" shall be returned including additional error information in the response body (in "ProblemDetails" data structure).

2c.	If the UE identity is not found in HSS, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in "ProblemDetails" data structure).

In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

## 5.7	Nhss_imsEventExposure Service

### 5.7.1	Service Description

See 3GPP TS 23.228 [6], clause AA.2.1.5.

### 5.7.2	Service Operations

#### 5.7.2.1	Introduction

For the Nhss_imsEventExposure service the following service operations are defined:

-	Subscribe

-	Unsubscribe

-	Modification of subscription

The Nhss_imsEventExposure Service is used by Consumer NFs (e.g., NEF) to:

-	subscribe/unsubscribe for a specific subscriber related IMS event.

#### 5.7.2.2	Subscribe

##### 5.7.2.2.1	General

The following procedures using the Subscribe service operation are supported:

-	Subscription to specific subscriber related IMS events, e.g., Bootstrap Data Channel Establishment, Application Data Channel Establishment.

##### 5.7.2.2.2	Subscription to a specific subscriber related IMS event

Figure 5.7.2.2.2-1 shows a scenario where the consumer sends a request to the HSS to subscribe a specific subscriber related IMS event. The request contains a UE's identity (/{imsUeId}) which shall be IMPU.

Figure 5.7.2.2.2-1: Subscription to a specific subscriber related IMS event

1.	The NF consumer sends a POST request to the collection resource "ee-subscriptions" representing the subscriptions on the UE's IMS events. The request body shall contain a callback URI where subsequent event notifications shall be sent by IMS AS and may contain another callback URI where revocation notifications may be sent by HSS.

2a.	Upon success, the HSS shall include a HTTP Location header to provide the location of the newly created resource (ee-subscriptions) together with the status code "201 Created" in the response message indicating that the requested resource is created.

2b.	On failure, one of the HTTP status code listed in table 6.6.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in table 6.6.3.2.3.1-3.

-	If the operation is not authorized due to, e.g. the requesting node not being authorized to subscribe to a specific subscriber related IMS event, HTTP status code "403 Forbidden" shall be returned including additional error information in the response body (in "ProblemDetails" data structure).

-	If the UE identity is not found in HSS, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in "ProblemDetails" data structure).

2c.	In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

#### 5.7.2.3	Unsubscribe

##### 5.7.2.3.1	General

The following procedures using the Unsubscribe service operation are supported:

-	Unsubscribe to a specific subscriber related IMS event.

##### 5.7.2.3.2	Unsubscribe to a specific subscriber related IMS event

Figure 5.7.2.3.2-1 shows a scenario where the NF consumer sends a request to the HSS to unsubscribe to a specific subscriber related IMS event. The request contains the UE's identity (/{imsUeId}) which shall be IMPU.

Figure 5.7.2.3.2-1: Deletion of a subscription to a specific subscriber related IMS event

1.	The consumer sends a DELETE request to the resource representing the individual subscription (subscriptionID) to a specific subscriber related IMS event.

2a.	Upon success, the HSS responds with "204 No Content" in the response message to indicate that the resource identified by the subscription ID has been successfully deleted.

2b.	On failure, one of the HTTP status code listed in table 6.6.3.3.3.1-3 indicating the error shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in table 6.6.3.3.3.1-3.

2c.	In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

#### 5.7.2.4	Modification of Subscription

##### 5.7.2.4.1	General

The following procedures using the Modification of Subscription service operation are supported:

-	Modification of Subscription to a specific subscriber related IMS event.

##### 5.7.2.4.2	Modification of Subscription to a specific subscriber related IMS event

Figure 5.7.2.4.2-1 shows a scenario where the NF consumer sends a request to the HSS to modify a previously subscribed IMS event for a specific subscriber. The request contains the UE's identity (/{imsUeId}) which shall be IMPU.

Figure 5.7.2.4.2-1: Modification of Subscription to a specific subscriber related IMS event

1.	The NF consumer sends a PATCH request to the resource representing the individual subscription to a specific subscriber (identified by its subscriptionID) to modify the subscription. The content of the PATCH request shall contain the list of operations (add/delete/replace) to be applied to the subscription; these operations may be directed to individual parameters of the subscription or to the list of the subscribed events.

2a.	Upon success, if the change is to delete the events to be subscribed and HSS accepts the request change, the HSS shall return the status code 204 No Content.

2b. On success, if the change is to add new events to be subscribed or to update the existing subscribed event within a subscription and the HSS accepts the request change, the HSS shall return the status code 200 OK. The response shall contain the new resource representation of the resource.

2c.	On failure, one of the HTTP status code listed in table 6.6.3.3.3.2-3 indicating the error shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in table 6.6.3.3.3.2-3.

-	If the operation is not authorized due to the requesting node not being authorized to modify the subscription to a specific subscriber related IMS event, HTTP status code "403 Forbidden" shall be returned including additional error information in the response body (in "ProblemDetails" data structure).

-	If the subscriptionId is not found in HSS, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in "ProblemDetails" data structure).

2d.	In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

#### 5.7.2.5	Notification

##### 5.7.2.5.1	General

The following procedures using the Notification service operation are supported:

-	Monitoring Revocation Notification;

##### 5.7.2.5.2	IMS Monitoring Revocation Notification

Figure 5.7.2.5.2-1 shows a scenario where the HSS notifies the NF service consumer (that has subscribed to receive such notification) about revocation of the Event Exposure Subscription. The request contains the notification URI as previously received in the ImsEeSubscription.

Figure 5.7.2.5.2-1: IMS Event Exposure Monitoring Revocation

1.	The HSS sends a POST request to the revocation notificationURI as provided by the NF service consumer during the subscription.

2.	The NF service consumer responds with "204 No Content".

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

In the case of redirection, the NF Service Consumer shall return 3xx status code, which shall contain a Location header with an URI pointing to an alternative notification endpoint.

# 6	API Definitions

## 6.1	Nhss_imsUEContextManagement Service API

### 6.1.1	Introduction

URIs of this API shall have the following root:

{apiRoot}/{apiName}/{apiVersion}/

where "apiRoot" is defined in clause 4.4.1 of 3GPP TS 29.501 [5], the "apiName" shall be set to "nhss-ims-uecm" and the "apiVersion" shall be set to "v1" for the current version of this specification.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, as defined in IETF RFC 9113 [8], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

HTTP messages and bodies for the Nhss_imsUECM service shall comply with the OpenAPI [9] specification contained in Annex A.2.

#### 6.1.2.2	HTTP standard headers

##### 6.1.2.2.1	General

The usage of HTTP standard headers shall be supported as specified in clause 5.2.2 of 3GPP TS 29.500 [4].

##### 6.1.2.2.2	Content type

The following content types shall be supported:

-	JSON, as defined in IETF RFC 8259 [10], signalled by the content type "application/json".

-	The Problem Details JSON Object (IETF RFC 9457 [11] signalled by the content type "application/problem+json".

-	JSON Patch, as defined in IETF RFC 6902 [12], signalled by the content type "application/json-patch+json".

#### 6.1.2.3	HTTP custom headers

##### 6.1.2.3.1	General

The usage of HTTP custom headers shall be supported as specified in clause 5.2.3 of 3GPP TS 29.500 [4].

### 6.1.3	Resources

#### 6.1.3.1	Overview

Figure 6.1.3.1-1: Resource URI structure of the Nhss_imsUECM API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name (Archetype) | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| ScscfRegistration (Document) | /{imsUeId}/scscf-registration | PUT | Create or update the S-CSCF registration state of the UE |
| Authorize (Custom operation) | /{impu}/authorize | authorize (POST) | Authorize the requested operation for the UE |
| Scscf Restoration Information (Document) | /{imsUeId}/scscf-registration/scscf-restoration-info | GET | Retrieve the S-CSCF restoration information. |
|  |  | PUT | Update the S-CSCF restoration information. |
|  |  | DELETE | Delete the S-CSCF restoration information. |
| ImsAs Registrations (Store) | /{imsUeId}/imsas-registrations | GET | Retrieve the information of IMS AS served the UE. |
| Individual Imsas Registration Information (Document) | /{imsUeId}/imsas-registrations/{instanceId} | GET | Retrieve the IMS AS of the UE |
|  |  | PUT | Create or update the IMS AS registration state of the UE |
|  |  | DELETE | Delete the IMS AS registration of the UE |

#### 6.1.3.2	Resource: Scscf Registration

##### 6.1.3.2.1	Description

This resource represents the registered S-CSCF.

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-uecm/{apiVersion}/{imsUeId}/scscf-registration

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| apiVersion | See clause 6.1.1 |
| imsUeId | Represents the IMS Subscription Identifier, either the IMS Public Identity (i.e. IMS Public User Identity or Public PSI) or the Private Identity (i.e. IMPI or PSI). See 3GPP TS 23.228 [6] clause 4.3). IMS Public Identity is used when the PUT method is used for registration; IMS Public Identity or IMS Private Identity is used when the PUT method is used for deregistration. Pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.1.3.2.3	Resource Standard Methods

6.1.3.2.3.1	PUT

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ScscfRegistration | M | 1 | S-CSCF registration for indicated IMS Public Identity is updated with the received information. |

Table 6.1.3.2.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ScscfRegistration | M | 1 | 201 Created | A response body containing a representation of the created individual scscfRegistration resource for the received IMS Public Identity shall be returned. |
| ScscfRegistration | M | 1 | 200 OK | A response body containing a representation of the updated individual scscfRegistration resource for the received IMS Public Identity shall be returned. |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - USER_NOT_FOUND |
| ExtendedProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - IDENTITY_ALREADY_REGISTERED In this case include invalidParams with cscfServerName as param and with value of the currently assigned S-CSCF name in the AdditionalInfo - IDENTITIES_DO_NOT_MATCH - ERROR_IN_REGISTRATION_TYPE |

Table 6.1.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.1.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.1.3.2.3.1-6: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nhss-ims-uecm/<apiVersion>/{imsUeId}/scscf-registration |

##### 6.1.3.2.4	Resource Custom Operations

There are no custom operations associated to resources.

#### 6.1.3.3	Resource: Scscf Restoration Information

##### 6.1.3.3.1	Description

This resource represents the IMS restoration information. It is queried by the service consumer (S-CSCF) to retrieve the information required for an S-CSCF to handle the requests for a user.

##### 6.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-uecm/{apiVersion}/{imsUeId}/scscf-registration/scscf-restoration-info

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| apiVersion | See clause 6.1.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦.+)$" |

##### 6.1.3.3.3	Resource Standard Methods

6.1.3.3.3.1	GET

This method shall support the URI query parameters specified in table 6.1.3.3.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ScscfRestorationInfoResponse | M | 1 | 200 OK | A response body containing information for the S-CSCF to handle the requests for a user. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - DATA_NOT_FOUND NOTE: DATA_NOT_FOUND indicates that there is no restoration info stored under the scscf-registration resource. |

Table 6.1.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.1.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

6.1.3.3.3.2	PUT

This method shall support the URI query parameters specified in table 6.1.3.3.3.2-1.

Table 6.1.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.3.3.3.2-3.

Table 6.1.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ScscfRestorationInfoRequest | M | 1 | S-CSCF restoration information for indicated IMS Public Identity is updated with the received information. |

Table 6.1.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ScscfRestorationInfoResponse | M | 1 | 201 Created | Upon success, a response body containing a representation of the created individual ScscfRestorationInfo resource for the received IMS Public Identity shall be returned. |
| ScscfRestorationInfoResponse | M | 1 | 200 OK | Upon success, a response body containing a representation of the updated individual ScscfRestorationInfo resource shall be returned. |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - OPERATION_NOT_ALLOWED NOTE: OPERATION_NOT_ALLOWED indicates the S-CSCF is not allowed to create/update the restoration information. |

Table 6.1.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.1.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.1.3.3.3.2-6: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nhss-ims-uecm/<apiVersion>/{imsUeId}/scscf-registration/scscf-restoration-info |

6.1.3.3.3.3	DELETE

This method shall support the URI query parameters specified in table 6.1.3.3.3.3-1.

Table 6.1.3.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.3-2 and the response data structures and response codes specified in table 6.1.3.3.3.3-3.

Table 6.1.3.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a | M | 1 | The request body shall be empty. |

Table 6.1.3.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - OPERATION_NOT_ALLOWED NOTE: OPERATION_NOT_ALLOWED indicates the S-CSCF is not allowed to delete the restoration information stored in the HSS. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - DATA_NOT_FOUND NOTE: DATA_NOT_FOUND indicates that there is no restoration info stored under the scscf-registration resource. |

Table 6.1.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.1.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.1.3.4	Resource: Individual Imsas Registration Information

##### 6.1.3.4.1	Description

This resource represents the individual registered IMS AS information.

##### 6.1.3.4.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-uecm/<apiVersion>/{imsUeId}/imsas-registrations/{instanceId}

This resource shall support the resource URI variables defined in table 6.1.3.4.2-1.

Table 6.1.3.4.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMPU). Pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15})$" |
| instanceId | Represents the NF instance ID of the individual registered IMS AS. |

##### 6.1.3.4.3	Resource Standard Methods

6.1.3.4.3.1	PUT

This method shall support the URI query parameters specified in table 6.1.3.4.3.1-1.

Table 6.1.3.4.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.4.3.1-2 and the response data structures and response codes specified in table 6.1.3.4.3.1-3.

Table 6.1.3.4.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ImsasRegistration | M | 1 | IMS AS registration for indicated IMS Public Identity is updated with the received information. |

Table 6.1.3.4.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsasRegistration | M | 1 | 201 Created | A response body containing a representation of the created individual ImsasRegistration resource for the received IMS Public Identity shall be returned. |
| ImsasRegistration | M | 1 | 200 OK | A response body containing a representation of the updated individual ImsasRegistration resource for the received IMS Public Identity shall be returned. |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - DATA_NOT_FOUND - USER_NOT_FOUND |

Table 6.1.3.4.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.1.3.4.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.1.3.4.3.1-6: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nhss-ims-uecm/<apiVersion>/{imsUeId}/scscf-registration |

6.1.3.4.3.2	DELETE

This method shall support the URI query parameters specified in table 6.1.3.4.3.2-1.

Table 6.1.3.4.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.4.3.2-2 and the response data structures and response codes specified in table 6.1.3.4.3.2-3.

Table 6.1.3.4.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

Table 6.1.3.4.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - DATA_NOT_FOUND |

Table 6.1.3.4.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.1.3.4.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

6.1.3.4.3.3	GET

This method shall support the URI query parameters specified in table 6.1.3.4.3.3.1-1.

Table 6.1.3.4.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.4.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.4.3.3.1-3.

Table 6.1.3.4.3.2.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.4.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsasRegistration | M | 1 | 200 OK | A response body containing information of IMS AS registration |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - DATA_NOT_FOUND |

Table 6.1.3.4.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.1.3.4.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

##### 6.1.3.4.4	Resource Custom Operations

There are no custom operations associated to resources.

#### 6.1.3.5	Resource: Imsas Registrations

##### 6.1.3.5.1	Description

This resource is used to represent IMS AS registrations.

##### 6.1.3.5.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-uecm/<apiVersion>/{imsUeId}/imsas-registrations

This resource shall support the resource URI variables defined in table 6.1.3.5.2-1.

Table 6.1.3.5.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.4.1 |
| imsUeId | string | Represents the IMS Public Identity (i.e. IMPU). Pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15})$" |

##### 6.1.3.5.3	Resource Standard Methods

6.1.3.5.3.1	GET

This method shall support the URI query parameters specified in table 6.1.3.5.3.1-1.

Table 6.1.3.5.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| asType | Astype | O | 0..1 | The type of the IMS AS required. |

This method shall support the request data structures specified in table 6.1.3.5.3.1-2 and the response data structures and response codes specified in table 6.1.3.5.3.1-3.

Table 6.1.3.5.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.5.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsAsRegistrationInfo | M | 1 | 200 OK | Upon success, a response body containing the ImsAsRegistrationInfo shall be returned. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be set to one of the following application errors: - CONTEXT_NOT_FOUND - USER_NOT_FOUND |
| NOTE: In addition common data structures as listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] are supported. |  |  |  |  |

##### 6.1.3.5.4	Resource Custom Operations

There are no custom operations associated to resources.

### 6.1.4	Custom Operations without associated resources

#### 6.1.4.1	Overview

Table 6.1.4.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| /{impu}/authorize | POST | Authorize the requested operation for the UE |

#### 6.1.4.2	Operation: Authorize

##### 6.1.4.2.1	Description

This resource represents the information that is needed to authorize a UE to register or establish sessions in the IMS domain. See 3GPP TS 23.228 [6].

##### 6.1.4.2.2	Operation Definition

This resource shall support the resource URI variables defined in table 6.1.4.2.2-1.

Table 6.1.4.2.2-1: URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| apiVersion | See clause 6.1.1 |
| impu | Represents the IMS Public Identity (i.e. IMPU or Public PSI). See 3GPP TS 23.228 [6] clause 4.3). |

This operation shall support the request data structures specified in table 6.1.4.2.2-1 and the response data structure and response codes specified in table 6.1.4.2.2-2.

Table 6.1.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AuthorizationRequest | M | 1 | Contains input for authorization, such as the authorization type or the visited network identifier. |

Table 6.1.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AuthorizationResponse | M | 1 | 200 OK | A response body containing result information or the registered S-CSCF shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - IDENTITY_NOT_REGISTERED |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - AUTHORIZATION_REJECTED - IDENTITIES_DONT_MATCH |

Table 6.1.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.1.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

### 6.1.5	Notifications

#### 6.1.5.1	General

This clause specifies the use of notifications and corresponding protocol details if required for the specific service. When notifications are supported by the API, it will include a reference to the general description of notifications support over the 5G SBIs specified in 3GPP TS 29.500 [4] / 3GPP TS 29.501 [5].

Table 6.1.5.1-1: Notifications overview

| Notification | Resource URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| S-CSCF Deregistration Notification | {deregCallbackUri} | POST |  |
| IMS AS Deregistration Notification | {deregCallbackUri} | POST |  |

#### 6.1.5.2	S-CSCF Deregistration Notification

The POST method shall be used for Deregistration Notifications sent to S-CSCF instance previously registered in HSS and the URI shall be as provided during the registration procedure.

Resource URI: {deregCallbackUri}

Support of URI query parameters is specified in table 6.1.5.2-1.

Table 6.1.5.2-1: URI query parameters supported by the POST method

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Support of request data structures is specified in table 6.1.5.2-2 and of response data structures and response codes is specified in table 6.1.5.2-3.

Table 6.1.5.2-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DeregistrationData | M | 1 | Includes Deregistration Reason and identities to be deregistered |

Table 6.1.5.2-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| DeregistrationData |  |  | 200 OK | Upon success or partial success, DeregistrationData shall be returned including additional information about the result of the procedure. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the notification endpoint of the subscribing NF Service Consumer. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the notification endpoint of the subscribing NF Service Consumer. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - CONTEXT_NOT_FOUND |

Table 6.1.5.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the notification endpoint of the subscribing NF Service Consumer. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.1.5.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the notification endpoint of the subscribing NF Service Consumer. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.1.5.3	IMS AS Deregistration Notification

The POST method shall be used for Deregistration Notifications sent to IMS AS instances previously registered in HSS and the URI shall be as provided during the registration procedure.

Resource URI: {deregCallbackUri}

Support of URI query parameters is specified in table 6.1.5.3-1.

Table 6.1.5.3-1: URI query parameters supported by the POST method

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Support of request data structures is specified in table 6.1.5.3-2 and of response data structures and response codes is specified in table 6.1.5.3-3.

Table 6.1.5.3-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AsDeregistrationData | M | 1 | Includes Deregistration Reason and identity to be deregistered |

Table 6.1.5.3-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the notification endpoint of the subscribing NF Service Consumer. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the notification endpoint of the subscribing NF Service Consumer. |

Table 6.1.5.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the notification endpoint of the subscribing NF Service Consumer. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.1.5.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the notification endpoint of the subscribing NF Service Consumer. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.1.5.4	IMS AS Reassignment Notification

The POST method shall be used for IMS AS Reassignment Notifications and the URI shall be as provided during the registration procedure.

Resource URI: {asReassignmentCallbackUri}

Support of URI query parameters is specified in table 6.1.5.4-1.

Table 6.1.5.4-1: URI query parameters supported by the POST method

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Support of request data structures is specified in table 6.1.5.4-2 and of response data structures and response codes is specified in table 6.1.5.4-3.

Table 6.1.5.4-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AsReassignmentnData | M | 1 | Contains the NF instance Id of the new IMS AS assigned. |

Table 6.1.5.4-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - CONTEXT_NOT_FOUND |
| NOTE: In addition common data structures as listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] are supported. |  |  |  |  |

Table 6.1.5.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the Callback URI of the target NF Service Consumer to which the request is redirected. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.5.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the Callback URI of the target NF Service Consumer to which the request is redirected. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nhss_imsUECM service-based interface protocol.

Table 6.1.6.1-1: Nhss_imsUECM specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AuthorizationRequest | 6.1.6.2.2 | Information for the type of authorization requested and the visited PLMN-ID |  |
| AuthorizationResponse | 6.1.6.2.3 | Result of the authorization requested for registration. |  |
| ScscfRestorationInfo | 6.1.6.2.4 | Information required for an S-CSCF to handle the requests for a user. |  |
| ScscfRestorationInfoRequest | 6.1.6.2.5 | Scscf restoration information request. |  |
| ScscfRestorationInfoResponse | 6.1.6.2.6 | Result of the scscf restoration information request. |  |
| RestorationInfo | 6.1.6.2.7 | Information related to a specific registration required for an S-CSCF to handle the requests for a user. |  |
| UeSubscriptionInfo | 6.1.6.2.8 | UE's subscription information. |  |
| PcscfSubscriptionInfo | 6.1.6.2.9 | P-CSCF's subscription information. |  |
| ScscfRegistration | 6.1.6.2.10 | Registration information of the S-CSCF for the registered UE. |  |
| ExtendedProblemDetails | 6.1.6.2.11 |  |  |
| AdditionalInfo | 6.1.6.2.12 |  |  |
| DeregistrationData | 6.1.6.2.13 |  |  |
| DeregistrationReason | 6.1.6.2.14 |  |  |
| EmergencyRegisteredIdentity | 6.1.6.2.15 |  |  |
| ImsasRegistration | 6.1.6.2.16 | Registration information of the IMS AS for the registered UE. |  |

Table 6.1.6.1-2 specifies data types re-used by the Nhss_imsUECM service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nhss_imsUECM service-based interface.

Table 6.1.6.1-2: Nhss_imsUECM re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Uri | 3GPP TS 29.571 [16] | Uniform Resource Identifier |  |
| SupportedFeatures | 3GPP TS 29.571 [16] | see 3GPP TS 29.500 [4] clause 6.6 |  |
| DateTime | 3GPP TS 29.571 [16] | String with format "date-time" as defined in OpenAPI Specification [9] |  |
| ProblemDetails | 3GPP TS 29.571 [16] | Response body of error response messages. |  |
| RedirectResponse | 3GPP TS 29.571 [16] | Response body of redirect response messages. |  |
| NfInstanceId | 3GPP TS 29.571 [16] | String uniquely identifying a NF instance |  |
| ScscfSelectionAssistanceInformation | 6.2.6.2.51 | This information shall be used by the I-CSCF to select an S-CSCF for the UE |  |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: AuthorizationRequest

Table 6.1.6.2.2-1: Definition of type AuthorizationRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| authorizationType | AuthorizationType | M | 1 | Contains the type of the authorization requested |  |
| impi | Impi | O | 1 | Contains an IMS Private User Identity. It shall be present when the authorization requested is for a registration or de-registration. |  |
| visitedNetworkIdentifier | string | O | 1 | Contains the identifier of the visited network where the P-CSCF is located. It shall be present when the authorization requested is for a registration or de-registration. |  |

##### 6.1.6.2.3	Type: AuthorizationResponse

Table 6.1.6.2.3-1: Definition of type AuthorizationResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| authorizationResult | AuthorizationResult | M | 1 | Contains the details of the granted authorization. |  |
| cscfServerName | string | C | 0..1 | Contains the stored S-CSCF name in SIP URI format. It shall be present if there is a S-CSCF name stored for the associated IMS subscription. |  |
| scscfSelectionAssistanceInfo | ScscfSelectionAssistanceInformation | C | 0..1 | Contains information needed by the I-CSCF to select an S-CSCF for the UE. Shall be present if cscfServerName is absent. |  |

##### 6.1.6.2.4	Type: ScscfRestorationInfo

Table 6.1.6.2.4-1: Definition of type ScscfRestorationInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| userName | Uri | O | 0..1 | Registered Private Identity of the user for whom the data is required. If the userName is NULL, it means that an S-CSCF request all registrations for all the private identities registered in the S-CSCF. |  |
| restorationInfo | array(RestorationInfo) | O | 0..N | This IE shall contain the information related to a specific registration required for an S-CSCF to handle the requests for a user. |  |
| registrationTimeOut | DateTime | O | 1 | This IE indicates the point of time at which the UE's registration expires. |  |
| sipAuthenticationScheme | SipAuthenticationScheme | O | 1 | This IE indicates the authentication scheme to be used in the authentication of SIP requests. |  |

##### 6.1.6.2.5	Type: ScscfRestorationInfoRequest

Table 6.1.6.2.5-1: Definition of type ScscfRestorationInfoRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| scscfRestorationInfoRequest | ScscfRestorationInfo | M | 1 | This IE shall contain the information related to a specific registration required for an S-CSCF to handle the requests for the users. |  |

##### 6.1.6.2.6	Type: ScscfRestorationInfoResponse

Table 6.1.6.2.6-1: Definition of type ScscfRestorationInfoResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| scscfRestorationInfoResponse | array(ScscfRestorationInfo) | M | 1..N | This IE shall contain the response information related to a specific or all registrations required for an S-CSCF. |  |

##### 6.1.6.2.7	Type: RestorationInfo

Table 6.1.6.2.7-1: Definition of type RestorationInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| path | string | M | 1 | This IE shall contain a comma separated list of SIP proxies in the Path header. |  |
| contact | string | M | 1 | This IE shall contain the Contact Addresses and Parameters in the Contact header. |  |
| initialCSeqSequenceNumber | Uint32 | O | 1 | This IE contains the sequence number of the CSeq header field contained in the initial successful REGISTER request. |  |
| callIdSipHeader | string | O | 1 | This IE contains the information in the Call-ID header. |  |
| uesubscriptionInfo | UeSubscriptionInfo | O | 1 | This IE contains the UE's subscription information. |  |
| pcscfSubscriptionInfo | PcscfSubscriptionInfo | O | 1 | This IE contains the P-CSCF's subscription information. |  |
| imsSdmSubscriptions | map(ImsSdmSubscription) | O | 1..N | A map (list of key-value pairs where subscriptionId serves as key) of imsSdmSubscriptions. |  |

##### 6.1.6.2.8	Type: UeSubscriptionInfo

Table 6.1.6.2.8-1: Definition of type UeSubscriptionInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| callIdSipHeader | string | M | 1 | This IE contains the information in the Call-ID header. |  |
| fromSipHeader | string | M | 1 | This IE shall contain the information in the From header. |  |
| toSipHeader | string | M | 1 | This IE shall contain the information in the To header. |  |
| recordRoute | string | M | 1 | This IE shall contain a comma separated list of Record Route header(s). |  |
| contact | string | M | 1 | This IE shall contain the Contact Addresses and Parameters in the Contact header. |  |

##### 6.1.6.2.9	Type: PcscfSubscriptionInfo

Table 6.1.6.2.9-1: Definition of type PcscfSubscriptionInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| callIdSipHeader | string | M | 1 | This IE contains the information in the Call-ID header. |  |
| fromSipHeader | string | M | 1 | This IE shall contain the information in the From header. |  |
| toSipHeader | string | M | 1 | This IE shall contain the information in the To header. |  |
| contact | string | M | 1 | This IE shall contain the Contact Addresses and Parameters in the Contact header. |  |

##### 6.1.6.2.10	Type: ScscfRegistration

Table 6.1.6.2.10-1: Definition of type ScscfRegistration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| imsRegistrationType | ImsRegistrationType | M | 1 | Contains the type of the registration/deregistration requested. |  |
| impi | Impi | C | 1 | Contains an IMS Private Identity. It may be absent during the originating or terminating request to an unregistered Public Identity (imsRegistrationType shall contain the value UNREGISTERED_USER) |  |
| cscfServerName | String | M | 1 | Contains the S-CSCF name in SIP URI format. |  |
| scscfInstanceId | NfInstanceId | M | 1 | The NF instance id of the S-CSCF. |  |
| deregCallbackUri | Uri | C | 1 | A URI provided by the S-CSCF to receive (implicitly subscribed) notifications on deregistration. The deregistration callback URI shall have unique information within S-CSCF set to identify the IMS Public Identity to be deregistered. If there are implicitly identities registered (i.e. the identity belongs to an IRS) in the response, the deregistration callback URI identifies the IRS to be deregistered. If there is a wildcarded PSI unregistered received in the response, the deregistration callback URI identifies the wildcarded PSI to be deregistered. |  |
| associatedImpis | array(Impi) | O | 1..N | Associated IMS private Identities in the subscription. If the IMS subscription contains only single Private Identity this attribute shall not be present. (NOTE) |  |
| associatedRegisteredImpis | array(Impi) | O | 1..N | Associated registered IMS private Identities in the subscription. (NOTE) |  |
| irsImpus | array(Impu) | O | 1...N | List of non barred IMS Public User Identities (distinct or wildcarded) which are in the Implicit Registration Set. This attribute shall be present when the received public identity belongs to an IRS. (NOTE) |  |
| wildcardedPui | Impu | O | 1 | It shall be present when the Public Identity in the request fell within the range of a Wildcarded Public User Identity present in the IRS. (NOTE) |  |
| wildcardedPsi | Impu | O | 1 | It shall be present if the Public Identity in the request fell within the range of a Wildcarded Public Service Identity in the HSS whose state is unregistered. If this element is present, it shall be used by the S-CSCF to identify the identity affected by the request. (NOTE) |  |
| looseRouteIndicator | LooseRouteIndication | O | 1 | Loose Routing Indicator (NOTE) |  |
| supportedFeatures | SupportedFeatures | O | 0..1 | See clause 6.2.8 |  |
| multipleRegistrationIndicator | boolean | C | 1 | Contains the multiple registration indication. If included, when it is set to true it indicates multiple registration. |  |
| pcscfRestorationIndicator | boolean | C | 0..1 | true: indicates that the P-CSCF-Restoration-mechanism shall be executed by the HSS. false or absent: P-CSCF-Restoration is not requested. May be present if imsRegistrationType is "UNREGISTERED_USER" or "ADMINISTRATIVE_DEREGISTRATION"; otherwise shall be absent. |  |
| scscfReselectionIndicator | boolean | C | 0..1 | true: indicates that the S-CSCF has been selected due to, e.g. failure of the previously assigned S-CSCF. false or absent: no S-CSCF reselection requested |  |
| NOTE: Optional attribute applicable in response messages only. |  |  |  |  |  |

##### 6.1.6.2.11	Type: ExtendedProblemDetails

Table 6.1.6.2.11-1: Definition of type ExtendedProblemDetails

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| problemDetails | ProblemDetails | M | 1 | More information on the error shall be provided in the "cause" attribute of the "ProblemDetails" structure. |
| additionalInfo | AdditionalInfo | O | 1 | Contains the currently assigned S-CSCF name in SIP URI format. |

##### 6.1.6.2.12	Type: AdditionalInfo

Table 6.1.6.2.12-1: Definition of type AdditionalInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| cscfServerName | String | O | 1 | Current S-CSCF assigned to the user (in SIP URI format). This attribute shall be included when "cause" is IDENTITY_ALREADY_REGISTERED. |

##### 6.1.6.2.13	Type: DeregistrationData

Table 6.1.6.2.13-1: Definition of type DeregistrationData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| deregReason | DeregistrationReason | M | 1 | Indicates the reason for the deregistration. |  |
| impi | Impi | M | 1 | Contains the IMS Private User Identity known by the SCSCF. |  |
| associatedImpis | Array(Impi) | O | 0..N | Associated IMS private Identities in the subscription that are deregistered. This attribute may be present in responses, otherwise, it shall be absent. |  |
| emergencyRegisteredIdentities | array(EmergencyRegisteredIdentity) | O | 1..N | A list of pairs of private and public user identities which have not been de-registered due to emergency registration. This attribute may be present in responses, otherwise, it shall be absent. |  |

##### 6.1.6.2.14	Type: DeregistrationReason

Table 6.1.6.2.14-1: Definition of type DeregistrationReason

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| reasonCode | DeregistrationReasonCode | M | 1 | Defines the reason for the network initiated de-registration |  |
| reasonText | string | O | 0..1 | Contains textual information to inform the user about the reason for a de-registration |  |

##### 6.1.6.2.15	Type: EmergencyRegisteredIdentity

Table 6.1.6.2.15-1: Definition of type EmergencyRegisteredIdentity

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| impi | Impi | M | 1 | IMS Private User Identity |  |
| impu | Impu | M | 1 | IMS Public User Identity |  |

##### 6.1.6.2.16	Type: ImsasRegistration

Table 6.1.6.2.16-1: Definition of type ImsasRegistration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| imsasInstanceId | NfInstanceId | M | 1 | NF Instance Id of the IMS AS |  |
| asType | AsType | O | 0..1 | The IMS AS type. |  |
| deregCallbackUri | Uri | O | 0..1 | A URI provided by the IMS AS to receive (implicitly subscribed) notifications on deregistration. The deregistration callback URI shall have unique information within IMS AS set to identify the IMS Public Identity to be deregistered. |  |
| reassignmentInd | boolean | O | 0..1 | Indicates whether this IMS AS has been assigned to handle new sessions for this user, as indicated by the S-CSCF. In case the value is: - true: HSS shall not remove any IMS AS registrations of the same AS type (possibly handling ongoing sessions). This attribute can only be set to true. When not present, HSS shall replace existing IMS AS of the same IMS AS type by the new one and shall initiate a deregistration towards the replaced IMS AS. |  |
| asReassignmentCallbackUri | Uri | C | 1 | A URI provided by the IMS AS to receive (implicitly subscribed) notifications on IMS AS Reassignment Notification. The callback URI shall have unique information within IMS AS set to identify the IMS Public Identity. |  |

##### 6.1.6.2.17	Type: ImsAsRegistrationInfo

Table 6.1.6.2.17-1: Definition of type ImsAsRegistrationInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| imsAsRegistrationList | array(ImsasRegistration) | M | 1..N | List of ImsasRegistration. |

##### 6.1.6.2.18	Type: AsDeregistrationData

Table 6.1.6.2.18-1: Definition of type AsDeregistrationData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| asDeregReason | AsDeregistrationReason | M | 1 | Indicates the reason for the deregistration. |  |
| impu | Impu | O | 0..1 | Contains the IMS Public User Identity deregistered by HSS. |  |

##### 6.1.6.2.19	Type: AsReassignmentData

Table 6.1.6.2.19-1: Definition of type AsReassignmentData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| imsasInstanceId | NfInstanceId | M | 1 | NF instance Id of the new IMS AS. |  |
| impu | Impu | O | 0..1 | Contains the IMS Public User Identity deregistered by HSS. |  |

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| Impu | string | Contains an IMS Public Identity |  |
| Impi | string | Contains an IMS Private User Identity |  |

##### 6.1.6.3.3	Enumeration: AuthorizationType

The enumeration AuthorizationType represents the type of authorization requested. It shall comply with the provisions defined in table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration AuthorizationType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "REGISTRATION" | The authorization requested if for a user's IMS registration or re-registration. |  |
| "DEREGISTRATION" | The authorization requested if for a user's IMS de-registration. |  |

##### 6.1.6.3.4	Enumeration: AuthorizationResult

The enumeration AuthorizationResult represents the details of the granted authorization. It shall comply with the provisions defined in table 6.1.6.3.4-1.

Table 6.1.6.3.4-1: AuthorizationResult

| Enumeration value | Description |
| --- | --- |
| "FIRST_REGISTRATION" | The registration is an initial registration for the IMS Public User Identity |
| "SUBSEQUENT_REGISTRATION" | The registration is an subsequent registration for the IMS Public User Identity |

##### 6.1.6.3.5	Enumeration: ImsRegistrationType

The enumeration ImsRegistrationType represents the type of registration associated to the REGISTER request.

Table 6.1.6.3.5-1: Enumeration ImsRegistrationType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "INITIAL_REGISTRATION" | The request is related to an initial registration |  |
| "RE_REGISTRATION" | The request is related to a re-registration |  |
| "TIMEOUT_DEREGISTRATION" | The registration timer of an identity has expired. |  |
| "USER_DEREGISTRATION" | The S-CSCF has received a user initiated de-registration request |  |
| "ADMINISTRATIVE_DEREGISTRATION" | The S-CSCF, due to administrative reasons or network issues, has performed the de-registration of an identity. |  |
| "AUTHENTICATION_FAILURE" | The authentication of a user has failed. |  |
| "AUTHENTICATION_TIMEOUT" | Authentication timeout has occurred. |  |
| "UNREGISTERED_USER" | The request is related to a request for a Public Identity that is not registered. |  |

##### 6.1.6.3.6	Enumeration: LooseRouteIndication

The enumeration LooseRouteIndication represents whether the "loose routing" mechanism is required, or not, to serve the user.

Table 6.1.6.3.6-1: Enumeration LooseRouteIndication

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "LOOSE_ROUTE_NOT_REQUIRED" | Indicates that strict routing mechanism is required to serve the user. |  |
| "LOOSE_ROUTE_REQUIRED " | Indicates that loose routing mechanism is required to serve the user. |  |

##### 6.1.6.3.7	Enumeration: DeregistrationReasonCode

The enumeration DeregistrationReasonCode indicates the reason for the network initiated de-registration.

Table 6.1.6.3.7-1: Enumeration DeregistrationReasonCode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "PERMANENT_TERMINATION" | The HSS indicates to the S-CSCF that the S-CSCF will no longer be assigned to the Public Identity and associated implicitly registered/unregistered Public Identities (if any) for the Private Identity(ies) indicated in the request (e.g. due to an IMS subscription cancellation, or modification, or a removal of IP-address secure binding information when GIBA is used). |  |
| "NEW_SERVER_ASSIGNED" | The HSS indicates to the S-CSCF that a new S-CSCF has been allocated to the IMS Subscription e.g. because the previous assigned S-CSCF was unavailable during a registration procedure. The S-CSCF shall remove all information for all of the Public Identities indicated in the request. |  |
| "SERVER_CHANGE" | The HSS indicates to the S-CSCF that the de-registration is requested to force the selection of new S-CSCF to assign to the IMS Subscription (e.g. when the S-CSCF capabilities are changed in the HSS or when the S-CSCF indicates that it has not enough memory for the updated User Profile). If the S-CSCF does not indicate in the response all the associated Private Identities, the HSS shall repeat this request for each of the remaining Private Identities in the IMS Subscription that are known to the S-CSCF. The S-CSCF should start the network initiated de-registration towards the user, i.e. all registrations within the IMS Subscription are de-registered and the user is asked to re-register to all existing registrations. |  |
| "REMOVE_S-CSCF" | The HSS indicates to the S-CSCF that the S-CSCF will no longer be assigned to an unregistered Public Identity(ies) (i.e registered as a consequence of an originating or terminating request or there is a S-CSCF keeping the user profile stored) for a given IMS Subscription. |  |

##### 6.1.6.3.8	Enumeration: AsType

The enumeration AsType represents the type of the IMS AS. It shall comply with the provisions defined in table 6.1.6.3.8-1.

Table 6.1.6.3.8-1: AsType

| Enumeration value | Description |
| --- | --- |
| "MMTEL_AS" | The type of the IMS AS is MMTel AS. |

##### 6.1.6.3.9	Enumeration: AsDeregistrationReason

The enumeration AsDeregistrationReason indicates the reason for the deregistration notification.

Table 6.1.6.3.9-1: Enumeration AsDeregistrationReason

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "ADMINISTRATIVE_TERMINATION" | The HSS indicates to the IMS AS that the IMS AS is no longer assigned to the Public Identity indicated in the request (e.g. due to an administrative IMS AS cancellation). |  |
| "NEW_SERVER_ASSIGNED" | The HSS indicates to the IMS AS that the HSS has de-registered the IMS AS due to a new IMS AS registration without the reassignment indication received by HSS. |  |

#### 6.1.6.4	Data types describing alternative data types or combinations of data types

There are no alternative data types, neither combinations of data types

### 6.1.7	Error Handling

#### 6.1.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [4].

#### 6.1.7.2	Protocol Errors

Protocol errors handling shall be supported as specified in clause 5.2.7 of 3GPP TS 29.500 [4].

#### 6.1.7.3	Application Errors

The common application errors defined in the Table 5.2.7.2-1 in 3GPP TS 29.500 [4] may also be used for the Nhss_imsUECM service. The following application errors listed in Table 6.1.7.3-1 are specific for the Nhss_imsUECM service.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| USER_NOT_FOUND | 404 Not Found | The received IMS UE identity is unknown. |
| CONTEXT_NOT_FOUND | 404 Not Found | The context identified by the callback reference is not found |
| IDENTITIES_DO_NOT_MATCH | 403 Forbidden | A message was received with a public identity and a private identity for a user, and the server determines that the public identity does not correspond to the private identity. |
| ERROR_IN_REGISTRATION_TYPE | 403 Forbidden | The requested registrationType is not allowed or the Public Identity type received in the request is not allowed for the requested registrationType. |
| IDENTITY_ALREADY_REGISTERED | 403 Forbidden | The identity has already a server assigned, and the registration information cannot be overwritten. |

### 6.1.8	Feature negotiation

The optional features in Table 6.1.8-1 are defined for the Nhss_imsUEContextManagement API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.1.9	Security

As indicated in 3GPP TS 33.501 [5], the access to the Nhss_imsUECM API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [35]), using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [36]) plays the role of the authorization server.

If Oauth2 authorization is used, an NF Service Consumer, prior to consuming services offered by the Nhss_imsUECM API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [36], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nhss_imsUECM service.

The Nhss_imsUECM API defines the following scopes for OAuth2 authorization:

Table 6.1.9-1: Oauth2 scopes defined in Nhss_imsUECM API

| Scope | Description |
| --- | --- |
| "nhss-ims-uecm" | Access to the Nhss IMS UE Context Management API |
| "nhss-ims-uecm:authorize:invoke" | Access to invoke the Authorize custom operation |
| "nhss-ims-uecm:registration:create" | Access to create S-CSCF Registration resources |
| "nhss-ims-uecm:restoration:read" | Access to read the S-CSCF Restoration resource |
| "nhss-ims-uecm:restoration:modify" | Access to create/update/delete the S-CSCF Restoration resource |
| "nhss-ims-uecm:imsasregistration:create" | Access to create the Individual Imsas Registration Information resource |
| "nhss-ims-uecm:imsasregistration:modify" | Access to delete the Individual Imsas Registration Information resource |
| "nhss-ims-uecm:imsasregistration:read" | Access to read IMS AS Registration resources |

### 6.1.10	HTTP redirection

An HTTP request may be redirected to a different HSS service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different HSS producer instance will return the NF Instance ID of the new HSS producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an HSS redirects a service request to a different HSS using an 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new HSS towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

## 6.2	Nhss_imsSubscriberDataManagement Service API

### 6.2.1	API URI

URIs of this API shall have the following root:

{apiRoot}/{apiName}/{apiVersion}/

where "apiRoot" is defined in clause 4.4.1 of 3GPP TS 29.501 [5], the "apiName" shall be set to "nhss-ims-sdm" and the "apiVersion" shall be set to "v1" for the current version of this specification.

### 6.2.2	Usage of HTTP

#### 6.2.2.1	General

HTTP/2, as defined in IETF RFC 9113 [8], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

HTTP messages and bodies for the Nhss_imsSubscriberDataManagement service shall comply with the OpenAPI [9] specification contained in Annex A.3.

#### 6.2.2.2	HTTP standard headers

##### 6.2.2.2.1	General

The usage of HTTP standard headers shall be supported as specified in clause 5.2.2 of 3GPP TS 29.500 [4].

##### 6.2.2.2.2	Content type

The following content types shall be supported:

-	JSON, as defined in IETF RFC 8259 [10], signalled by the content type "application/json".

-	The Problem Details JSON Object (IETF RFC 9457 [11] signalled by the content type "application/problem+json"

-	JSON Patch, as defined in IETF RFC 6902 [12], signalled by the content type "application/json-patch+json"

#### 6.2.2.3	HTTP custom headers

##### 6.2.2.3.1	General

The usage of HTTP custom headers shall be supported as specified in clause 5.2.3 of 3GPP TS 29.500 [4].

### 6.2.3	Resources

#### 6.2.3.1	Overview

Figure 6.2.3.1-1: Resource URI structure of the Nhss_imsSDM API

Figure 6.2.3.1-2: Resource URI structure of the Nhss_imsSDM API

Table 6.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.3.1-1: Resources and methods overview

| Resource name (Archetype) | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Repository Data (Document) | /{imsUeId}/repository-data/{serviceIndication} | GET | Retrieve repository data for a service indication |
|  |  | DELETE | Delete repository data for a service indication |
|  |  | PUT | Update repository data for a service indication |
| Repository Data List (Document) | /{imsUeId}/repository-data | GET | Retrieve multiple repository data for a list of service indications |
| IMS Associated Identities (Document) | /{imsUeId}/identities/ims-associated-identities | GET | Retrieve the UE's IMS associated identities (implicit and alias) |
| MSISDNs (Document) | /{imsUeId}/identities/msisdns | GET | Retrieve the UE's MSISDNs (basic and additional) |
| PrivateIdentities (Document) | /{imsUeId}/identities/private-identities | GET | Retrieve the UE's Private Identities (IMPI and IMSI) |
| ImeiSvInformation (Document) | /{imsUeId}/identities/imeisv | GET | Retrieve the UE's IMEISV |
| IMS Profile Data (Document) | /{imsUeId}/ims-data/profile-data | GET | Retrieve the UE's subscribed Profile |
| Initial Filter Criteria (Document) | /{imsUeId}/ims-data/ profile-data/ifcs | GET | Retrieve the UE's subscribed Initial Filter Criteria |
| Service Level Trace Information (Document) | /{imsUeId}/ims-data/profile-data/service-level-trace-info | GET | Retrieve the UE's subscribed Service Level Trace Information |
| Service Priority Information (Document) | /{imsUeId}/ims-data/ profile-data/priority-levels | GET | Retrieve the UE's subscribed Service Priority Levels |
| Charging Information (Document) | /{imsUeId}/ims-data/ profile-data/chargingInfo | GET | Retrieve the UE's subscribed Charging Information |
| IMS Location Data (Document) | /{imsUeId}/ims-data/location-data/server-name | GET | Retrieve the UE's S-CSCF name |
| S-CSCF Capabilities (Document) | /{imsUeId}/ims-data/location-data/scscf-capabilities | GET | Retrieve the UE's S-CSCF capabilities (mandatory and/or optional) |
| S-CSCF Selection Assistance Information (Document) | /{imsUeId}/ims-data/location-data/scscf-selection-assistance-info | GET | Retrieve the UE's S-CSCF Selection Assistance Information |
| IMS Registration Status (Document) | /{imsUeId}/ims-data/ registration-status | GET | Retrieve the UE's IMS registration status. |
| Subscriptions (Collection) | /{imsUeId}/subscriptions | POST | Create a subscription |
| Individual subscription (Document) | /{imsUeId}/subscriptions/ {subscriptionId} | DELETE | Delete the subscription identified by {subscriptionId}, i.e. unsubscribe |
|  |  | PATCH | Modify the sdm-subscription identified by {subscriptionId} |
| PS Location Information(Document) | /{imsUeId}/access-data/ ps-domain/location-data | GET | Retrieve the UE's location in PS domain |
| CS Location Information(Document) | /{imsUeId}/access-data/ cs-domain/location-data | GET | Retrieve the UE's location in CS domain |
| PsUserState (Document) | /{imsUeId}/access-data/ ps-domain/user-state | GET | Retrieve the UE's state in PS domain |
| CsUserState (Document) | /{imsUeId}/access-data/ cs-domain/user-state | GET | Retrieve the UE's state in CS domain |
| TadsInfo (Document) | /{imsUeId}/access-data/ ps-domain/tads-info | GET | Retrieve the UE's T-ADS information |
| UeReachabilityIpSubscriptions (Collection) | /{imsUeId}/access-data/ ps-domain/ue-reach-subscriptions | POST | Create a subscription to UE Reachability for IP |
| UE reachability individual subscription (Document) | /{imsUeId}/access-data/ ps-domain/ue-reach-subscriptions/ {subscriptionId} | DELETE | Delete the subscription to UE reachability for IP identified by {subscriptionId}, i.e. unsubscribe |
|  |  | PATCH | Modify the subscription to UE reachability for IP identified by {subscriptionId} |
| IpAddress (Document) | /{imsUeId}/access-data/ ps-domain/ip-address | GET | Retrieve the UE's stored IP Address Secure Binding Information |
| Csrn (Document) | /{imsUeId}/access-data/ cs-domain/csrn | GET | Retrieve a CSRN for the UE |
| ReferenceAccessLocation (Document) | /{imsUeId}/access-data/wireline-domain/reference-location | GET | Retrieve the UE's subscribed Reference Access Location |
| SRVCC Data (Document) | /{imsUeId}/srvcc-data | GET | Retrieve the UE's STN-SR and SRVCC capability |
|  |  | PATCH | Update the UE's STN-SR. |
| PSI Activation State (Document) | /{imsUeId}/service-data/ psi-status | GET | Retrieve a PSI activation status |
|  |  | PATCH | Update a PSI activation status |
| Dsai (Document) | /{imsUeId}/service-data/dsai | GET | Retrieve DSAI data (i.e. DSAI tags and status) associated to an Application Server |
|  |  | PATCH | Update DSAI data associated to an Application Server |
| SMSRegistrationInfo (Document) | /{imsUeId}/service-data/ sms-registration-info | GET | Retrieve the SMS Registration Information (e.g. IP-SM-GW address) |
|  |  | PUT | Creates or updates the SMS Registration Information (e.g. IP-SM-GW address) |
|  |  | DELETE | Deletes the SMS Registration Information (e.g. IP-SM-GW address) |
| SharedData (Collection) | /shared-data | GET | Retrieve shared data |
| SharedDataSubscriptions (Collection) | /shared-data-subscriptions | POST | Create a subscription |
| Shared Data Individual Subscription (Document) | /shared-data-subscriptions/{subscriptionId} | DELETE | Delete the subscription identified by {subscriptionId}, i.e. unsubscribe |
|  |  | PATCH | Modify the subscription identified by {subscriptionId} |

#### 6.2.3.2	Resource: IMS Associated Identities

##### 6.2.3.2.1	Description

This resource represents the IMS associated identities. It is queried by the service consumer (IMS-AS) to retrieve the list of identities in the same Implicit Registration Set, together with the Alias Group Information and additional information (e.g. identity type)

##### 6.2.3.2.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/identities/ims-associated-identities

This resource shall support the resource URI variables defined in table 6.2.3.2.2-1.

Table 6.2.3.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.2.3	Resource Standard Methods

6.2.3.2.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.2.3.1-1.

Table 6.2.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.3.2.3.1-3.

Table 6.2.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsAssociatedIdentities | M | 1 | 200 OK | A response body containing the list of identities belonging to the same Implicit Registration Set shall be returned, together with the related information (Alias Group, if any, identity type, IRS default identity, registration state of the IRS). |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - USER_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - OPERATION_NOT_ALLOWED |

Table 6.2.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.3	Resource: MSISDNS

##### 6.2.3.3.1	Description

This resource represents the MSISDNs associated to the UE. It is queried by the service consumer (IMS-AS) to retrieve the list of MSISDNs (basic and additional). If the UE is shared among multiple IMS Private Identities and the consumer queries for the basic MSISDN (C-MSISDN), the URI query parameter Private Identity (IMPI) shall be included.

##### 6.2.3.3.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/identities/msisdns

This resource shall support the resource URI variables defined in table 6.2.3.3.2-1.

Table 6.2.3.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.3.3	Resource Standard Methods

6.2.3.3.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.3.3.1-1.

Table 6.2.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| private-id | PrivateId | O | 0..1 | IMS Private Identity (IMPI) of the UE. It shall be included if there are multiple Private Identities associated to the Public Identity received and the basic MSISDN (C-MSISDN) is requested. |

This method shall support the request data structures specified in table 6.2.3.3.3.1-2 and the response data structures and response codes specified in table 6.2.3.3.3.1-3.

Table 6.2.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MsisdnList | M | 1 | 200 OK | A response body containing the list of MSISDNs associated to the UE and all Private Identities shall be returned. If the response is for a basic MSISDN (C-MSISDN) query (query parameter IMPI was present), only one MSISDN shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.4	Resource: S-CSCF Capabilities

##### 6.2.3.4.1	Description

This resource represents the S-CSCF capabilities. It is queried by the service consumer (I-CSCF) to retrieve S-CSCF capabilities associated to the UE (mandatory capabilities and/or optional capabilities).

##### 6.2.3.4.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/ims-data/location-data/scscf-capabilities

This resource shall support the resource URI variables defined in table 6.2.3.4.2-1.

Table 6.2.3.4.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.4.3	Resource Standard Methods

6.2.3.4.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.4.3.1-1.

Table 6.2.3.4.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.4.3.1-2 and the response data structures and response codes specified in table 6.2.3.4.3.1-3.

Table 6.2.3.4.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.4.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ScscfCapabilityList | M | 1 | 200 OK | A response body containing the list mandatory and/or optional capabilities shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND NOTE: DATA_NOT_FOUND indicates that any S-CSCF can be selected, e.g. based on I-CSCF local policy. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.4.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.4.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.5	Resource: IMS Profile Data

##### 6.2.3.5.1	Description

This resource represents the IMS Profile Data. It is queried by the service consumer (S-CSCF) to retrieve the user's IMS profile data, containing the required information to handle the multimedia sessions (e.g. Initial Filter Criteria)

##### 6.2.3.5.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/ims-data/profile-data

This resource shall support the resource URI variables defined in table 6.2.3.5.2-1.

Table 6.2.3.5.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.5.3	Resource Standard Methods

6.2.3.5.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.5.3.1-1.

NOTE:	The retrieval of these data sets can also be achieved by sending individual GET requests to the corresponding sub-resources under the {imsUeId} resource. When multiple data sets need to be retrieved by the NF Service consumer, it is recommended to use a single GET request with query parameters rather than issuing multiple GET requests.

Table 6.2.3.5.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dataset-names | array(DataSetName) | O | 1..N | Datasets to be retrieved |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 6.2.3.5.3.1-2 and the response data structures and response codes specified in table 6.2.3.5.3.1-3.

Table 6.2.3.5.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.5.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsProfileData | M | 1 | 200 OK | Upon success, a response body containing the User's Profile Data shall be included. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application errors: - OPERATION_NOT_ALLOWED |

Table 6.2.3.5.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.5.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.6	Resource: Subscriptions

##### 6.2.3.6.1	Description

This resource is used to represent subscriptions to notifications of data change.

##### 6.2.3.6.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/subscriptions

This resource shall support the resource URI variables defined in table 6.2.3.6.2-1.

Table 6.2.3.6.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.6.3	Resource Standard Methods

6.2.3.6.3.1	POST

This method shall support the URI query parameters specified in table 6.2.3.6.3.1-1.

Table 6.2.3.6.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.6.3.1-2 and the response data structures and response codes specified in table 6.2.3.6.3.1-3.

Table 6.2.3.6.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ImsSdmSubscription | M | 1 | The subscription that is to be created. |

Table 6.2.3.6.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsSdmSubscription | M | 1 | 201 Created | Upon success, a response body containing a representation of the created Individual subscription resource shall be returned. The HTTP response shall include a "Location" HTTP header that contains the resource URI of the created resource. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - USER_NOT_FOUND |
| ProblemDetails | O | 0..1 | 501 Not Implemented | The "cause" attribute may be used to indicate the following application error: - UNSUPPORTED_RESOURCE_URI This response shall not be cached. |

Table 6.2.3.6.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.6.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.6.3.1-6: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nhss-ims-sdm/<apiVersion>/{imsUeId}/subscriptions/{subscriptionId} |

#### 6.2.3.7	Resource: Individual subscription

##### 6.2.3.7.1	Description

This resource is used to represent an individual subscription to notifications of data change.

##### 6.2.3.7.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.2.3.7.2-1.

Table 6.2.3.7.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |
| subscriptionId | The subscriptionId identifies an individual subscription to notifications. |

##### 6.2.3.7.3	Resource Standard Methods

6.2.3.7.3.1	DELETE

This method shall support the URI query parameters specified in table 6.2.3.7.3.1-1.

Table 6.2.3.7.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.7.3.1-2 and the response data structures and response codes specified in table 6.2.3.7.3.1-3.

Table 6.2.3.7.3.1-2: Data structures supported by the Delete Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

Table 6.2.3.7.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - SUBSCRIPTION_NOT_FOUND, see 3GPP TS 29.500 [4] table 5.2.7.2-1. |

Table 6.2.3.7.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.7.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

6.2.3.7.3.2	PATCH

This method shall support the URI query parameters specified in table 6.2.3.7.3.2-1.

Table 6.2.3.7.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 6.2.3.7.3.2-2 and the response data structures and response codes specified in table 6.2.3.7.3.2-3.

Table 6.2.3.7.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1 | It contains the list of changes to be made to the resource representing the individual subscription, according to the JSON PATCH format specified in IETF RFC 6902 [12]. |

Table 6.2.3.7.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, a response with no content is returned. (NOTE 1) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 1) |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - SUBSCRIPTION_NOT_FOUND, see 3GPP TS 29.500 [4] table 5.2.7.2-1. |
| ProblemDetails | O | 0..1 | 403 Forbidden | One or more attributes are not allowed to be modified. The "cause" attribute may be used to indicate the following application error: - MODIFICATION_NOT_ALLOWED, see 3GPP TS 29.500 [4] table 5.2.7.2-1. |
| NOTE 1: If all the modification instructions in the PATCH request have been implemented, the HSS shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the HSS shall respond with PatchResult. |  |  |  |  |

Table 6.2.3.7.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.7.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.8	Resource: SharedDataSubscriptions

##### 6.2.3.8.1	Description

This resource is used to represent subscriptions to notifications for shared data.

##### 6.2.3.8.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/shared-data-subscriptions

This resource shall support the resource URI variables defined in table 6.2.3.8.2-1.

Table 6.2.3.8.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |

##### 6.2.3.8.3	Resource Standard Methods

6.2.3.8.3.1	POST

This method shall support the URI query parameters specified in table 6.2.3.8.3.1-1.

Table 6.2.3.8.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.8.3.1-2 and the response data structures and response codes specified in table 6.2.3.8.3.1-3.

Table 6.2.3.8.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ImsSdmSubscription | M | 1 | The subscription that is to be created. |

Table 6.2.3.8.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsSdmSubscription | M | 1 | 201 Created | Upon success, a response body containing a representation of the created Individual subscription resource shall be returned. The HTTP response shall include a "Location" HTTP header that contains the resource URI of the created resource. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 501 Not Implemented | The "cause" attribute may be used to indicate the following application error: - UNSUPPORTED_RESOURCE_URI This response shall not be cached. |

Table 6.2.3.8.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.8.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.8.3.1-6: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nhss-ims-sdm/<apiVersion>/shared-data-subscriptions/{subscriptionId} |

#### 6.2.3.9	Resource: Shared Data Individual subscription

##### 6.2.3.9.1	Description

This resource is used to represent an individual subscription to notifications for shared data.

##### 6.2.3.9.2	Resource Definition

Resource URI: {apiRoot}/nhss-sdm/{apiVersion}/shared-data-subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.2.3.9.2-1.

Table 6.2.3.9.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| subscriptionId | The subscriptionId identifies an individual subscription to notifications. |

##### 6.2.3.9.3	Resource Standard Methods

6.2.3.9.3.1	DELETE

This method shall support the URI query parameters specified in table 6.2.3.9.3.1-1.

Table 6.2.3.9.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.zz.3.1-2 and the response data structures and response codes specified in table 6.2.3.zz.3.1-3.

Table 6.2.3.9.3.1-2: Data structures supported by the Delete Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

Table 6.2.3.9.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The resource corresponding to the SubscriptionId can't be found. The "cause" attribute may be used to indicate the following application error: - SUBSCRIPTION_NOT_FOUND, see 3GPP TS 29.500 [4] table 5.2.7.2-1. |

Table 6.2.3.9.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.9.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

6.2.3.9.3.2	PATCH

This method shall support the URI query parameters specified in table 6.2.3.9.3.2-1.

Table 6.2.3.9.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 6.2.3.9.3.2-2 and the response data structures and response codes specified in table 6.2.3.9.3.2-3.

Table 6.2.3.9.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1 | It contains the list of changes to be made to the resource representing the individual subscription, according to the JSON PATCH format specified in IETF RFC 6902 [12]. |

Table 6.2.3.9.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, a response with no content is returned. (NOTE 1) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 1) |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The resource corresponding to the SubscriptionId can't be found. The "cause" attribute may be used to indicate the following application error: - SUBSCRIPTION_NOT_FOUND, see 3GPP TS 29.500 [4] table 5.2.7.2-1. |
| ProblemDetails | O | 0..1 | 403 Forbidden | One or more attributes are not allowed to be modified. The "cause" attribute may be used to indicate the following application error: - MODIFICATION_NOT_ALLOWED, see 3GPP TS 29.500 [4] table 5.2.7.2-1. |
| NOTE 1: If all the modification instructions in the PATCH request have been implemented, the HSS shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the HSS shall respond with PatchResult. |  |  |  |  |

Table 6.2.3.9.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.9.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.10	Resource: IMS Registration Status

##### 6.2.3.10.1	Description

This resource represents the IMS Registration Status. It is queried by the service consumer (e.g. AS) to retrieve the IMS registration status of the user.

##### 6.2.3.10.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/ims-data/registration-status

This resource shall support the resource URI variables defined in table 6.2.3.10.2-1.

Table 6.2.3.10.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.10.3	Resource Standard Methods

6.2.3.10.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.10.3.1-1.

Table 6.2.3.10.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |

This method shall support the request data structures specified in table 6.2.3.10.3.1-2 and the response data structure and response codes specified in table 6.2.3.10.3.1-3.

Table 6.2.3.10.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.10.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsRegistrationStatus | M | 1 | 200 OK | A response body containing the IMS registration status of the user. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND DATA_NOT_FOUND indicates the user has not performed yet any IMS registration. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.10.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.10.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.11	Resource: Service Priority Information

##### 6.2.3.11.1	Description

This resource represents the Service Priority Levels. It is queried by the service consumer (e.g. AS) to retrieve the Namespaces and priority levels allowed for the user.

##### 6.2.3.11.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/ims-data/profile-data/priority-levels

This resource shall support the resource URI variables defined in table 6.2.3.11.2-1.

Table 6.2.3.11.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.11.3	Resource Standard Methods

6.2.3.11.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.11.3.1-1.

Table 6.2.3.11.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP 1 29.500 [4] clause 6.6 |  |

This method shall support the request data structures specified in table 6.2.3.11.3.1-2 and the response data structures and response codes specified in table 6.2.3.11.3.1-3.

Table 6.2.3.11.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.11.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PriorityLevels | M | 1 | 200 OK | A response body containing the service priority information (priority namespaces and associated levels) allowed for the user shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND DATA_NOT_FOUND indicates that the user does not have priority service. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.11.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.11.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.12	Resource: Initial Filter Criteria

##### 6.2.3.12.1	Description

This resource represents the Initial Filter Criteria (IFCs). It is queried by the service consumer (e.g. S-CSCF, AS) to retrieve the list of IFCs associated to the UE (including Shared IFC Set Identifiers, if provisioned).

##### 6.2.3.12.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/ims-data/profile-data/ifcs

This resource shall support the resource URI variables defined in table 6.2.3.12.2-1.

Table 6.2.3.12.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.12.3	Resource Standard Methods

6.2.3.12.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.12.3.1-1.

Table 6.2.3.12.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |
| application-server-name | SipServerName | O | 0..1 | This information element indicates application server's SIP URI. |  |

This method shall support the request data structures specified in table 6.2.3.12.3.1-2 and the response data structures and response codes specified in table 6.2.3.12.3.1-3.

Table 6.2.3.12.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.12.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| Ifcs | M | 1 | 200 OK | A response body containing the list of complete IFCs and/or shared IFC set identifiers shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND DATA_NOT_FOUND indicates that there is no Initial Filter Criteria associated to the user (i.e. no AS is triggered by S-CSCF for the user) |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.12.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.12.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.13	Resource: IMS Location Data

##### 6.2.3.13.1	Description

This resource represents the IMS location data. It is queried by the service consumer (e.g. AS) to retrieve the S-CSCF name stored for the user.

##### 6.2.3.13.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/ims-data/location-data/server-name

This resource shall support the resource URI variables defined in table 6.2.3.13.2-1.

Table 6.2.3.13.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.13.3	Resource Standard Methods

6.2.3.13.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.13.3.1-1.

Table 6.2.3.13.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |

This method shall support the request data structures specified in table 6.2.3.13.3.1-2 and the response data structure and response codes specified in table 6.2.3.13.3.1-3.

Table 6.2.3.13.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.13.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsLocationData | M | 1 | 200 OK | A response body containing the S-CSCF name stored for the user. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND DATA_NOT_FOUND indicates the user has not performed yet any IMS registration. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.13.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.13.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.14	Resource: Service Level Trace Information

##### 6.2.3.14.1	Description

This resource represents the Service Level Trace Information. It is queried by the service consumer (e.g. AS) to retrieve the Trace Level information so that the consumer can perform logging for the user as described in IETF RFC 8497 [23].

##### 6.2.3.14.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/ims-data/profile-data/service-trace-level-information

This resource shall support the resource URI variables defined in table 6.2.3.14.2-1.

Table 6.2.3.14.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.14.3	Resource Standard Methods

6.2.3.14.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.14.3.1-1.

Table 6.2.3.14.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |

This method shall support the request data structures specified in table 6.2.3.14.3.1-2 and the response data structures and response codes specified in table 6.2.3.14.3.1-3.

Table 6.2.3.14.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.14.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceLevelTraceInformation | M | 1 | 200 OK | A response body containing the IMS service level trace information shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND DATA_NOT_FOUND indicates that the service level tracing is disabled for the user. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.14.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.14.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.15	Resource: PS location Information

##### 6.2.3.15.1	Description

This resource represents the Location Information in PS domain. It is queried by the service consumer (e.g. AS) to retrieve the location information in PS domain as retrieved from the serving node(s). The service consumer may indicate the requested nodes for which the request is applicable (e.g. AMF only)

##### 6.2.3.15.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/access-data/ps-domain/location-data

This resource shall support the resource URI variables defined in table 6.2.3.15.2-1.

Table 6.2.3.15.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) or the IMS private Identity. If both the IMS Public Identity and the IMS Private Identity are available at the service consumer, the imsUeId represents the IMS Public Identity. pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.15.3	Resource Standard Methods

6.2.3.15.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.15.3.1-1.

Table 6.2.3.15.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| requested-nodes | array(RequestedNode) | O | 1..N | Indicates the serving node(s) for which the request is applicable. |
| serving-node | boolean | O | 0..1 | Indicates that only the stored NF id/address of the serving node(s) requested and PLMN identity are required, that is, other location data (e.g. Global Cell ID) is not required. It shall be absent if current-location is present with value true. true: only the requested node(s) address(es) are requested false or absent: location data as retrieved from the requested node(s) is requested. |
| local-time | boolean | O | 0..1 | Indicates that only the Local Time Zone information (Time Zone and Daylight Saving Time) and the PLMN identity of the location in the visited network where the UE is attached is requested, that is, other location data (e.g. Global Cell ID) is not required. true: only the local time is requested false or absent: location data as retrieved from the requested node(s) is requested. |
| current-location | boolean | O | 0..1 | Indicates whether an active location retrieval has to be initiated by the requested node(s). true: active location retrieval is requested false or absent: active location retrieval is not requested |
| rat-type | boolean | O | 0..1 | Indicates whether retrieval of RAT type is requested. true: requested false: not requested |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |
| private-identity | PrivateId | C | 0..1 | Represents the IMS Private Identity. Shall be present if the imsUeId variable in the resource URI takes the value of an IMS Public User Identity that is shared by several Private User Identities. |

If "requested-nodes" is not included, HSS shall return location information as retrieved from all the nodes (AMF, MME and SGSN)

This method shall support the request data structures specified in table 6.2.3.15.3.1-2 and the response data structures and response codes specified in table 6.2.3.15.3.1-3.

Table 6.2.3.15.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.15.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PsLocation | M | 1 | 200 OK | A response body containing the PS location information as requested shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND DATA_NOT_FOUND indicates that the location information is unknown or unavailable. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.15.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.15.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.16	Resource: CS location Information

##### 6.2.3.16.1	Description

This resource represents the Location Information in CS domain. It is queried by the service consumer (e.g. AS) to retrieve the location information in CS domain as retrieved from the MSC/VLR.

##### 6.2.3.16.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/access-data/cs-domain/location-data

This resource shall support the resource URI variables defined in table 6.2.3.16.2-1.

Table 6.2.3.16.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) or the IMS private Identity. If both the IMS Public Identity and the IMS Private Identity are available at the service consumer, the imsUeId represents the IMS Public Identity. pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.16.3	Resource Standard Methods

6.2.3.16.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.16.3.1-1.

Table 6.2.3.16.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| serving-node | boolean | O | 0..1 | Indicates that only the MSC/VLR address and PLMN identity are required, that is, other location data (e.g. Global Cell ID) is not required. It shall be absent if current-location is present. true: only the MSC/VLR number requested false or absent: location data as retrieved from the MSC/VLR is requested. |
| local-time | boolean | O | 0..1 | Indicates that only the Local Time Zone information (Time Zone and Daylight Saving Time) and the PLMN identity of the location in the visited network where the UE is attached is requested, that is, other location data (e.g. Global Cell ID) is not required. It shall be absent if current-location is present. |
| current-location | boolean | O | 0..1 | Indicates whether an active location retrieval has to be initiated by the MSC/VLR. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |
| private-identity | PrivateId | C | 0..1 | Represents the IMS Private Identity. Shall be present if the imsUeId variable in the resource URI takes the value of an IMS Public User Identity that is shared by several Private User Identities. |

If "serving-node" is not included, HSS shall return the location data as retrieved from the MSC/VLR.

If "local-time" is not included, HSS shall return the location data as retrieved from the MSC/VLR.

This method shall support the request data structures specified in table 6.2.3.16.3.1-2 and the response data structures and response codes specified in table 6.2.3.16.3.1-3.

Table 6.2.3.16.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.16.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CsLocation | M | 1 | 200 OK | A response body containing the CS location information as requested shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND DATA_NOT_FOUND indicates that the location information is unknown or unavailable. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.16.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.16.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.17	Resource: SRVCC data

##### 6.2.3.17.1	Description

This resource represents the SRVCC data. It is queried by the service consumer (e.g. AS) to retrieve the UE SRVCC capability and the STN-SR for the user.

##### 6.2.3.17.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/srvcc-data

This resource shall support the resource URI variables defined in table 6.2.3.17.2-1.

Table 6.2.3.17.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) or the IMS private Identity. If both the IMS Public Identity and the IMS Private Identity are available at the service consumer, the imsUeId represents the IMS Public Identity. pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.17.3	Resource Standard Methods

6.2.3.17.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.17.3.1-1.

Table 6.2.3.17.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |
| private-identity | PrivateId | C | 0..1 | Represents the IMS Private Identity. Shall be present if the imsUeId variable in the resource URI takes the value of an IMS Public User Identity that is shared by several Private User Identities. |  |

This method shall support the request data structures specified in table 6.2.3.17.3.1-2 and the response data structures and response codes specified in table 6.2.3.17.3.1-3.

Table 6.2.3.17.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.17.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SrvccData | M | 1 | 200 OK | A response body containing the STN-SR and the UE SRVCC capabilities for the user shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND DATA_NOT_FOUND indicates that the user is not subscribed to SRVCC. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.17.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.17.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

6.2.3.17.3.2	PATCH

This method shall support the URI query parameters specified in table 6.2.3.17.3.2-1.

Table 6.2.3.17.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |
| private-identity | PrivateId | C | 0..1 | Represents the IMS Private Identity. Shall be present if the imsUeId variable in the resource URI takes the value of an IMS Public User Identity that is shared by several Private User Identities. |

This method shall support the request data structures specified in table 6.2.3.17.3.2-2 and the response data structures and response codes specified in table 6.2.3.17.3.2-3.

Table 6.2.3.17.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Items describe the modifications to the SRVCC data. Modification are not applicable to UE SRVCC capabilities. |

Table 6.2.3.17.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. (NOTE 1) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 1) |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND DATA_NOT_FOUND indicates that the user is not subscribed to SRVCC. |
| ProblemDetails | O | 0..1 | 403 Forbidden | One or more attributes are not allowed to be modified. The "cause" attribute may be used to indicate one of the following application errors: - MODIFICATION_NOT_ALLOWED, see GPP TS 29.500 [4] table 5.2.7.2-1. - OPERATION_NOT_ALLOWED |
| NOTE 1: If all the modification instructions in the PATCH request have been implemented, the HSS shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the HSS shall respond with PatchResult. |  |  |  |  |

Table 6.2.3.17.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.17.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.18	Resource: PSI activation state

##### 6.2.3.18.1	Description

This resource represents the PSI activation state. It is queried by the service consumer (e.g. AS) to retrieve the activation state for the IMS Public Service Identity.

##### 6.2.3.18.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/psi-status

This resource shall support the resource URI variables defined in table 6.2.3.18.2-1.

Table 6.2.3.18.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Service Identity pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.18.3	Resource Standard Methods

6.2.3.18.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.18.3.1-1.

Table 6.2.3.18.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |

This method shall support the request data structures specified in table 6.2.3.18.3.1-2 and the response data structures and response codes specified in table 6.2.3.18.3.1-3.

Table 6.2.3.18.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.18.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PsiActivationState | M | 1 | 200 OK | A response body containing the PSI activation state shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - SERVICE_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.18.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.18.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

6.2.3.18.3.2	PATCH

This method shall support the URI query parameters specified in table 6.2.3.18.3.2-1.

Table 6.2.3.18.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 6.2.3.18.3.2-2 and the response data structures and response codes specified in table 6.2.3.18.3.2-3.

Table 6.2.3.18.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Items describe the modifications to the PSI activation state. |

Table 6.2.3.18.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. (NOTE 1) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 1) |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - SERVICE_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | One or more attributes are not allowed to be modified. The "cause" attribute may be used to indicate one of the following application errors: - MODIFICATION_NOT_ALLOWED, see GPP TS 29.500 [4] table 5.2.7.2-1. - OPERATION_NOT_ALLOWED |
| NOTE 1: If all the modification instructions in the PATCH request have been implemented, the HSS shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the HSS shall respond with PatchResult. |  |  |  |  |

Table 6.2.3.18.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.18.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.19	Resource: Repository Data

##### 6.2.3.19.1	Description

This resource represents the Repository Data. It is used by the service consumer (e.g. IMS-AS) to create, retrieve, update or delete the repository data for a given service indication.

##### 6.2.3.19.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/repository-data/{serviceIndication}

This resource shall support the resource URI variables defined in table 6.2.3.19.2-1.

Table 6.2.3.19.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |
| serviceIndication | Represents the identifier of a service related data. |

##### 6.2.3.19.3	Resource Standard Methods

6.2.3.19.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.19.3.1-1.

Table 6.2.3.19.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 6.2.3.19.3.1-2 and the response data structures and response codes specified in table 6.2.3.19.3.1-3.

Table 6.2.3.19.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.19.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RepositoryData | M | 1 | 200 OK | A response body containing the Repository Data for the requested Service Indication. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.19.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.19.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

6.2.3.19.3.2	PUT

This method shall support the URI query parameters specified in table 6.2.3.19.3.2-1.

Table 6.2.3.19.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.19.3.2-2 and the response data structures and response codes specified in table 6.2.3.19.3.2-3.

Table 6.2.3.19.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| RepositoryData | M | 1 | The Repository Data to be created or replaced. |

Table 6.2.3.19.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RepositoryData | M | 1 | 201 Created | Upon success, a response body containing a representation of the created Individual RepositoryData resource shall be returned. |
| RepositoryData | M | 1 | 200 OK | Upon success, a response body containing a representation of the updated Individual RepositoryData resource shall be returned. |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - USER_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |
| ProblemDetails | O | 0..1 | 409 Conflict | The "cause" attribute may be used to indicate the following application error: - OUT_OF_SYNC |
| ProblemDetails | O | 0..1 | 413 Content Too Large | The "cause" attribute may be used to indicate the following application error: - TOO_MUCH_DATA |

Table 6.2.3.19.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.19.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.19.3.2-6: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nhss-ims-sdm/<apiVersion>/{imsUeId}/repository-data/{serviceIndication} |

6.2.3.19.3.3	DELETE

This method shall support the URI query parameters specified in table 6.2.3.19.3.3-1.

Table 6.2.3.19.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.19.3.3-2 and the response data structures and response codes specified in table 6.2.3.19.3.3-3.

Table 6.2.3.19.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

Table 6.2.3.19.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND DATA_NOT_FOUND indicates that no repository data exists for the requested service indication. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.19.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.19.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.20	Resource: SharedData

##### 6.2.3.20.1	Description

This resource represents the collection of data that can be shared by multiple UEs.

##### 6.2.3.20.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/<apiVersion>/shared-data

This resource shall support the resource URI variables defined in table 6.2.3.20.2-1.

Table 6.2.3.20.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |

##### 6.2.3.20.3	Resource Standard Methods

6.2.3.20.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.20.3.1-1.

Table 6.2.3.20.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| shared-data-ids | array(SharedDataId) | M | 1..N |  |

This method shall support the request data structures specified in table 6.2.3.20.3.1-2 and the response data structures and response codes specified in table 6.2.3.20.3.1-3.

Table 6.2.3.20.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.20.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(SharedData) | M | 1..N | 200 OK | Upon success, a response body containing a list of SharedData shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - DATA_NOT_FOUND |

Table 6.2.3.20.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.20.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.21	Resource: ImeiSvInformation

##### 6.2.3.21.1	Description

This resource represents the IMEI(SV) information. It is queried by the service consumer (e.g. IMS AS) to retrieve the IMEI(SV) for the user.

##### 6.2.3.21.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/identities/imeisv

This resource shall support the resource URI variables defined in table 6.2.3.21.2-1.

Table 6.2.3.21.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity) or the IMS Private Identity. If both the IMS Public Identity and the IMS Private Identity are available at the service consumer, the imsUeId represents the IMS Public Identity. pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.21.3	Resource Standard Methods

6.2.3.21.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.21.3.1-1.

Table 6.2.3.21.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |
| private-identity | PrivateId | C | 0..1 | Represents the IMS Private Identity. Shall be present if the imsUeId variable in the resource URI takes the value of an IMS Public User Identity that is shared by several Private User Identities. |  |

This method shall support the request data structures specified in table 6.2.3.21.3.1-2 and the response data structures and response codes specified in table 6.2.3.21.3.1-3.

Table 6.2.3.21.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.21.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImeiSvInformation | M | 1 | 200 OK | A response body containing the IMEI(SV) information for the user shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.21.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.21.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.22	Resource: IP Address

##### 6.2.3.22.1	Description

This resource represents the IP Address Information. It is queried by the service consumer (e.g. SCC-AS) to retrieve the IP Address Information for the user.

##### 6.2.3.22.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/access-data/ps-domain/ip-address

This resource shall support the resource URI variables defined in table 6.2.3.22.2-1.

Table 6.2.3.22.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. non-shared IMS Public User identity) or the IMS Private Identity. pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.22.3	Resource Standard Methods

6.2.3.22.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.22.3.1-1.

Table 6.2.3.22.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |

This method shall support the request data structures specified in table 6.2.3.22.3.1-2 and the response data structures and response codes specified in table 6.2.3.22.3.1-3.

Table 6.2.3.22.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.22.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| IpAddr | M | 1 | 200 OK | A response body containing the IP address (IPv4, Ipv6, Ipv6 prefix and/or IPv6 interface identifier) for the user shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND DATA_NOT_FOUND indicates that no IP Address is currently allocated for the user. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.22.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.22.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.23	Resource: T-ADS Info

##### 6.2.3.23.1	Description

This resource represents the T-ADS Information. It is queried by the service consumer (e.g. SCC-AS) to retrieve the T-ADS Information for the user.

##### 6.2.3.23.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/access-data/ps-domain/tads-info

This resource shall support the resource URI variables defined in table 6.2.3.23.2-1.

Table 6.2.3.23.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity) or the IMS Private Identity. If both the IMS Public Identity and the IMS Private Identity are available at the service consumer, the imsUeId represents the IMS Public Identity. pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.23.3	Resource Standard Methods

6.2.3.23.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.23.3.1-1.

Table 6.2.3.23.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |
| private-identity | PrivateId | C | 0..1 | Represents the IMS Private Identity. Shall be present if the imsUeId variable in the resource URI takes the value of an IMS Public User Identity that is shared by several Private User Identities. |  |

This method shall support the request data structures specified in table 6.2.3.23.3.1-2 and the response data structures and response codes specified in table 6.2.3.23.3.1-3.

Table 6.2.3.23.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.23.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TadsInformation | M | 1 | 200 OK | A response body containing the T-ADS information (most recent VoPS support) for the user shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.23.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.23.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.24	Resource: UeReachabilitySubscriptions

#### 6.2.3.24	Description

This resource is used to represent subscriptions to notifications of UE Reachability for IP.

##### 6.2.3.24.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/access-data/ps-domain/ue-reach-subscriptions

This resource shall support the resource URI variables defined in table 6.2.3.24.2-1.

Table 6.2.3.24.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.24.3	Resource Standard Methods

6.2.3.24.3.1	POST

This method shall support the URI query parameters specified in table 6.2.3.24.3.1-1.

Table 6.2.3.24.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| private-identity | PrivateId | C | 0..1 | Represents the IMS Private Identity. Shall be present if the imsUeId variable in the resource URI takes the value of an IMS Public User Identity that is shared by several Private User Identities. |

This method shall support the request data structures specified in table 6.2.3.24.3.1-2 and the response data structures and response codes specified in table 6.2.3.24.3.1-3.

Table 6.2.3.24.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UeReachabilitySubscription | M | 1 | The subscription to UE reachability for IP that is to be created. |

Table 6.2.3.24.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CreatedUeReachabilitySubscription | M | 1 | 201 Created | Upon success, a response body containing a representation of the created Individual subscription resource shall be returned. The HTTP response shall include a "Location" HTTP header that contains the resource URI of the created resource. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - USER_NOT_FOUND |

Table 6.2.3.24.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.24.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.24.3.1-6: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nhss-ims-sdm/<apiVersion>/{imsUeId}/access-data/ps-domain/ue-reach-subscriptions/{subscriptionId} |

#### 6.2.3.25	Resource: UE reachability Individual subscription

##### 6.2.3.25.1	Description

This resource is used to represent an individual subscription to notification of UE reachability for IP.

##### 6.2.3.25.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/access-data/ps-domain/ue-reach-subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.2.3.25.2-1.

Table 6.2.3.25.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |
| subscriptionId | The subscriptionId identifies an individual subscription to notification of UE reachability for IP. |

##### 6.2.3.25.3	Resource Standard Methods

6.2.3.25.3.1	DELETE

This method shall support the URI query parameters specified in table 6.2.3.25.3.1-1.

Table 6.2.3.25.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| private-identity | PrivateId | C | 0..1 | Represents the IMS Private Identity. Shall be present if the imsUeId variable in the resource URI takes the value of an IMS Public User Identity that is shared by several Private User Identities. |

This method shall support the request data structures specified in table 6.2.3.7.3.1-2 and the response data structures and response codes specified in table 6.2.3.7.3.1-3.

Table 6.2.3.25.3.1-2: Data structures supported by the Delete Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

Table 6.2.3.25.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - SUBSCRIPTION_NOT_FOUND, see 3GPP TS 29.500 [4] table 5.2.7.2-1. |

Table 6.2.3.25.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.25.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

6.2.3.25.3.2	PATCH

This method shall support the URI query parameters specified in table 6.2.3.25.3.2-1.

Table 6.2.3.25.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |
| private-identity | PrivateId | C | 0..1 | Represents the IMS Private Identity. Shall be present if the imsUeId variable in the resource URI takes the value of an IMS Public User Identity that is shared by several Private User Identities. |

This method shall support the request data structures specified in table 6.2.3.25.3.2-2 and the response data structures and response codes specified in table 6.2.3.25.3.2-3.

Table 6.2.3.25.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1 | It contains the list of changes to be made to the resource representing the individual subscription, according to the JSON PATCH format specified in IETF RFC 6902 [12]. |

Table 6.2.3.25.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, a response with no content is returned. (NOTE 1) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 1) |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - SUBSCRIPTION_NOT_FOUND, see 3GPP TS 29.500 [4] table 5.2.7.2-1. |
| ProblemDetails | O | 0..1 | 403 Forbidden | One or more attributes are not allowed to be modified. The "cause" attribute may be used to indicate the following application error: - MODIFICATION_NOT_ALLOWED, see 3GPP TS 29.500 [4] table 5.2.7.2-1. |
| NOTE 1: If all the modification instructions in the PATCH request have been implemented, the HSS shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the HSS shall respond with PatchResult. |  |  |  |  |

Table 6.2.3.25.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.25.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.26	Resource: PS User State

##### 6.2.3.26.1	Description

This resource represents the User State information in PS domain. It is queried by the service consumer (e.g. AS) to retrieve the User State information in PS domain as retrieved from the serving node(s). The service consumer may indicate the requested nodes for which the request is applicable (e.g. AMF only)

##### 6.2.3.26.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/access-data/ps-domain/user-state

This resource shall support the resource URI variables defined in table 6.2.3.26.2-1.

Table 6.2.3.26.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) or the IMS private Identity. If both the IMS Public Identity and the IMS Private Identity are available at the service consumer, the imsUeId represents the IMS Public Identity. pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.26.3	Resource Standard Methods

6.2.3.26.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.26.3.1-1.

Table 6.2.3.26.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| requested-nodes | array(RequestedNode) | O | 1..N | Indicates the serving node(s) for which the request is applicable. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |
| private-identity | PrivateId | C | 0..1 | Represents the IMS Private Identity. Shall be present if the imsUeId variable in the resource URI takes the value of an IMS Public User Identity that is shared by several Private User Identities. |

If "requested-nodes" is not included, HSS shall return the user state as retrieved from all the nodes (AMF, MME and SGSN)

This method shall support the request data structures specified in table 6.2.3.26.3.1-2 and the response data structures and response codes specified in table 6.2.3.26.3.1-3.

Table 6.2.3.26.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.26.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PsUserState | M | 1 | 200 OK | A response body containing the PS user state information as requested shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND DATA_NOT_FOUND indicates that the user state information is unknown or unavailable. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.26.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.26.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.27	Resource: CS User State

##### 6.2.3.27.1	Description

This resource represents the User State information in CS domain. It is queried by the service consumer (e.g. AS) to retrieve the User State information in CS domain as retrieved from the MSC/VLR.

##### 6.2.3.27.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/access-data/cs-domain/user-state

This resource shall support the resource URI variables defined in table 6.2.3.27.2-1.

Table 6.2.3.27.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) or the IMS private Identity. If both the IMS Public Identity and the IMS Private Identity are available at the service consumer, the imsUeId represents the IMS Public Identity. pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.27.3	Resource Standard Methods

6.2.3.27.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.27.3.1-1.

Table 6.2.3.27.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |
| private-identity | PrivateId | C | 0..1 | Represents the IMS Private Identity. Shall be present if the imsUeId variable in the resource URI takes the value of an IMS Public User Identity that is shared by several Private User Identities. |

This method shall support the request data structures specified in table 6.2.3.27.3.1-2 and the response data structures and response codes specified in table 6.2.3.27.3.1-3.

Table 6.2.3.27.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.27.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CsUserState | M | 1 | 200 OK | A response body containing the CS user state information as requested shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.27.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.27.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.28	Resource: Csrn

##### 6.2.3.28.1	Description

This resource represents the CSRN. It is queried by the service consumer (e.g. SCC-AS) to retrieve a CSRN for the user.

##### 6.2.3.28.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/access-data/cs-domain/csrn

This resource shall support the resource URI variables defined in table 6.2.3.28.2-1.

Table 6.2.3.28.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity) or the IMS Private Identity. If both the IMS Public Identity and the IMS Private Identity are available at the service consumer, the imsUeId represents the IMS Public Identity. pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.28.3	Resource Standard Methods

6.2.3.28.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.28.3.1-1.

Table 6.2.3.28.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |
| pre-paging | PrePaging | O | 0..1 | Indicates if pre-paging is supported true: pre-paging is supported false or absent: pre-paging is not supported |  |
| private-identity | PrivateId | C | 0..1 | Represents the IMS Private Identity. Shall be present if the imsUeId variable in the resource URI takes the value of an IMS Public User Identity that is shared by several Private User Identities. |  |
| call-reference-info | CallReferenceInfo | O | 0..1 | Contains a Call Reference Number and the AS-Number. It allows a later retry of the call setup in the context of MTRR. See 3GPP TS 29.328 [38] |  |

This method shall support the request data structures specified in table 6.2.3.28.3.1-2 and the response data structures and response codes specified in table 6.2.3.28.3.1-3.

Table 6.2.3.28.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.28.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| Csrn | M | 1 | 200 OK | A response body containing the CSRN for the user shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND DATA_NOT_FOUND indicates that CSRN retrieval from MSC/VLR failed. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.28.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.28.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.29	Resource: ReferenceAccessLocation

##### 6.2.3.29.1	Description

This resource represents the Reference Location Information. It is queried by the service consumer (e.g. IMS-AS) to retrieve the reference location information for the user (e.g. line identifier in fixed access networks).

##### 6.2.3.29.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/access-data/wireline-domain/reference-location

This resource shall support the resource URI variables defined in table 6.2.3.29.2-1.

Table 6.2.3.29.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity) or the IMS Private Identity. If both the IMS Public Identity and the IMS Private Identity are available at the service consumer, the imsUeId represents the IMS Public Identity. pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.29.3	Resource Standard Methods

6.2.3.29.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.29.3.1-1.

Table 6.2.3.29.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |
| private-identity | PrivateId | C | 0..1 | Represents the IMS Private Identity. Shall be present if the imsUeId variable in the resource URI takes the value of an IMS Public User Identity that is shared by several Private User Identities. |  |

This method shall support the request data structures specified in table 6.2.3.29.3.1-2 and the response data structures and response codes specified in table 6.2.3.29.3.1-3.

Table 6.2.3.29.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.29.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ReferenceLocationInformation | M | 1 | 200 OK | A response body containing the reference location information (e.g. access type, line identifier) for the user shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND DATA_NOT_FOUND indicates that no reference location information exist for the user. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.29.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.29.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.30	Resource: SMSRegistrationInfo

##### 6.2.3.30.1	Description

This resource represents the SMS registration information. It is used by the service consumer (e.g. IP-SM-GW) to create, retrieve, update or delete the SMS registration information.

##### 6.2.3.30.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/service-data/sms-registration-info

This resource shall support the resource URI variables defined in table 6.2.3.30.2-1.

Table 6.2.3.30.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity) or the IMS private Identity. If both the IMS Public Identity and the IMS Private Identity are available at the service consumer, the imsUeId represents the IMS Public Identity. pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.30.3	Resource Standard Methods

6.2.3.30.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.30.3.1-1.

Table 6.2.3.30.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |
| private-identity | PrivateId | C | 0..1 | Represents the IMS Private Identity. Shall be present if the imsUeId variable in the resource URI takes the value of an IMS Public User Identity that is shared by several Private User Identities. |

This method shall support the request data structures specified in table 6.2.3.30.3.1-2 and the response data structures and response codes specified in table 6.2.3.30.3.1-3.

Table 6.2.3.30.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.30.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmsRegistrationInfo | M | 1 | 200 OK | A response body containing the SMS registration information. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - USER_NOT_FOUND - DATA_NOT_FOUND DATA_NOT_FOUND indicates that no IP-SM-GW number is preconfigured or created previously for the user. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - OPERATION_NOT_ALLOWED |

Table 6.2.3.30.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.30.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

6.2.3.30.3.2	PUT

This method shall support the URI query parameters specified in table 6.2.3.30.3.2-1.

Table 6.2.3.30.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| private-identity | PrivateId | C | 0..1 | Represents the IMS Private Identity. Shall be present if the imsUeId variable in the resource URI takes the value of an IMS Public User Identity that is shared by several Private User Identities. |

This method shall support the request data structures specified in table 6.2.3.30.3.2-2 and the response data structures and response codes specified in table 6.2.3.30.3.2-3.

Table 6.2.3.30.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| IpSmGwAddress | M | 1 | The SMS Registration Information to be created or replaced. |

Table 6.2.3.30.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmsRegistrationInfo | M | 1 | 201 Created | Upon success, a response body containing a representation of the created Individual RepositoryData resource shall be returned. |
| SmsRegistrationInfo | M | 1 | 200 OK | Upon success, a response body containing a representation of the updated Individual RepositoryData resource shall be returned. |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - USER_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |
| ProblemDetails | O | 0..1 | 409 Conflict | The "cause" attribute may be used to indicate the following application error: - IP_SM_GW_CANNOT_BE_MODIFIED |

Table 6.2.3.30.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.30.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.30.3.2-6: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nhss-ims-sdm/<apiVersion>/{imsUeId}/service-data/sms-registration-info |

6.2.3.30.3.3	DELETE

This method shall support the URI query parameters specified in table 6.2.3.30.3.3-1.

Table 6.2.3.30.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| private-identity | PrivateId | C | 0..1 | Represents the IMS Private Identity. Shall be present if the imsUeId variable in the resource URI takes the value of an IMS Public User Identity that is shared by several Private User Identities. |

This method shall support the request data structures specified in table 6.2.3.30.3.3-2 and the response data structures and response codes specified in table 6.2.3.30.3.3-3.

Table 6.2.3.30.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

Table 6.2.3.30.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND DATA_NOT_FOUND indicates that no SMS registration information exists for the user. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |
| ProblemDetails | O | 0..1 | 409 Conflict | The "cause" attribute may be used to indicate the following application error: - IP_SM_GW_CANNOT_BE_MODIFIED |

Table 6.2.3.30.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.30.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.31	Resource: Dsai

##### 6.2.3.31.1	Description

This resource represents the DSAI Tag Information for the requested Application Server Name. It is used by the service consumer (e.g. IMS-AS) to retrieve or update the DSAI Tag Status.

##### 6.2.3.31.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/service-data/dsai

This resource shall support the resource URI variables defined in table 6.2.3.31.2-1.

Table 6.2.3.31.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. non-shared IMS Public User identity) or the IMS private Identity. pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.31.3	Resource Standard Methods

6.2.3.31.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.31.3.1-1.

Table 6.2.3.31.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| application-server-name | SipServerName | M | 1 | The requested Application Server Name. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |
| dsai-tag | string | O | 0..1 | This information element identify the requested instance of Dynamic Service Activation Info (DSAI). |

This method shall support the request data structures specified in table 6.2.3.31.3.1-2 and the response data structures and response codes specified in table 6.2.3.31.3.1-3.

Table 6.2.3.31.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.31.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DsaiTagInformation | M | 1 | 200 OK | A response body containing the DSAI information for the requested Application Server. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND DATA_NOT_FOUND indicates that no DSAI Tag was found for the requested Application Server. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.31.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.31.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

6.2.3.31.3.2	PATCH

This method shall support the URI query parameters specified in table 6.2.3.31.3.2-1.

Table 6.2.3.31.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 6.2.3.31.3.2-2 and the response data structures and response codes specified in table 6.2.3.31.3.2-3.

Table 6.2.3.31.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Items describe the modifications to the DSAI tag(s) activation state. |

Table 6.2.3.31.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. (NOTE 1) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 1) |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - DSAI_TAG_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | One or more attributes are not allowed to be modified. The "cause" attribute may be used to indicate one of the following application errors: - MODIFICATION_NOT_ALLOWED, see GPP TS 29.500 [4] table 5.2.7.2-1. - OPERATION_NOT_ALLOWED |
| NOTE 1: If all the modification instructions in the PATCH request have been implemented, the HSS shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the HSS shall respond with PatchResult. |  |  |  |  |

Table 6.2.3.31.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.31.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.32	Resource: Private Identities

##### 6.2.3.32.1	Description

This resource represents the private identities. It is queried by the service consumer (IMS-AS) to retrieve the list of private identities.

##### 6.2.3.32.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/identities/private-identities

This resource shall support the resource URI variables defined in table 6.2.3.32.2-1.

Table 6.2.3.32.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.32.3	Resource Standard Methods

6.2.3.32.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.32.3.1-1.

Table 6.2.3.32.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |
| impi | PrivateId | O | 0..1 | Private Identity of type IMPI |  |

This method shall support the request data structures specified in table 6.2.3.32.3.1-2 and the response data structures and response codes specified in table 6.2.3.32.3.1-3.

Table 6.2.3.32.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.32.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PrivateIdentities | M | 1 | 200 OK | A response body containing the list of private identities belonging to the IMS UE. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - USER_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.32.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.32.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.33	Resource: S-CSCF Selection Assistance Information

##### 6.2.3.33.1	Description

This resource represents the S-CSCF selection assistance information. It is queried by the service consumer (I-CSCF) before S-CSCF selection.

##### 6.2.3.33.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/ims-data/location-data/scscf-selection-assistance-info

This resource shall support the resource URI variables defined in table 6.2.3.33.2-1.

Table 6.2.3.33.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.33.3	Resource Standard Methods

6.2.3.33.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.33.3.1-1.

Table 6.2.3.33.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.33.3.1-2 and the response data structures and response codes specified in table 6.2.3.33.3.1-3.

Table 6.2.3.33.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.33.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ScscfSelectionAssistanceInformation | M | 1 | 200 OK | A response body containing the S-CSCF Selection Assistance Information. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND NOTE: DATA_NOT_FOUND indicates that any S-CSCF can be selected, e.g. based on I-CSCF local policy. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.33.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.33.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.2.3.34	Resource: Charging Information

##### 6.2.3.34.1	Description

This resource represents the Charging Information. It is queried by the service consumer (e.g. AS) to retrieve the Diameter addresses Charging Functions for the user.

##### 6.2.3.34.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/ims-data/profile-data/charging-info

This resource shall support the resource URI variables defined in table 6.2.3.34.2-1.

Table 6.2.3.34.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.34.3	Resource Standard Methods

6.2.3.34.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.34.3.1-1.

Table 6.2.3.34.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP 1 29.500 [4] clause 6.6 |  |

This method shall support the request data structures specified in table 6.2.3.34.3.1-2 and the response data structures and response codes specified in table 6.2.3.34.3.1-3.

Table 6.2.3.34.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.34.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ChargingInfo | M | 1 | 200 OK | A response body containing the Charging Information for the user shall be returned. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

#### 6.2.3.35	Resource: Repository Data List

##### 6.2.3.35.1	Description

This resource represents the Repository Data. It is used by the service consumer (e.g. IMS-AS) to  retrieve multiple repository data for a given list of service indications.

##### 6.2.3.35.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-sdm/{apiVersion}/{imsUeId}/repository-data

This resource shall support the resource URI variables defined in table 6.2.3.19.2-1.

Table 6.2.3.35.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.2.1 |
| apiVersion | See clause 6.2.1 |
| imsUeId | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity) pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

##### 6.2.3.35.3	Resource Standard Methods

6.2.3.35.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.35.3.1-1.

Table 6.2.3.35.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |
| service-indications | array(ServiceIndication) | M | 1..N |  |

This method shall support the request data structures specified in table 6.2.3.35.3.1-2 and the response data structures and response codes specified in table 6.2.3.35.3.1-3.

Table 6.2.3.35.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.35.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RepositoryDataList | M | 1 | 200 OK | A response body containing the Repository Data List for the requested Service Indications. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - OPERATION_NOT_ALLOWED |

Table 6.2.3.35.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.3.35.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

### 6.2.5	Notifications

#### 6.2.5.1	General

This clause specifies the use of notifications and corresponding protocol details.

#### 6.2.5.2	Data Change Notification

The POST method shall be used for Data Change Notifications and the URI shall be as provided during the subscription procedure.

Resource URI: {callbackReference}

Support of URI query parameters is specified in table 6.2.5.2-1.

Table 6.2.5.2-1: URI query parameters supported by the POST method

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Support of request data structures is specified in table 6.2.5.2-2 and of response data structures and response codes is specified in table 6.2.5.2-3.

Table 6.2.5.2-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ModificationNotification | M | 1 |  |

Table 6.2.5.2-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the notification endpoint of the subscribing NF Service Consumer. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the notification endpoint of the subscribing NF Service Consumer. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - CONTEXT_NOT_FOUND See table 6.2.7.3-1 for the description of this error. |

Table 6.2.5.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the notification endpoint of the subscribing NF. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.2.5.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the notification endpoint of the subscribing NF. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

### 6.2.6	Data Model

#### 6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the Nhss_imsSDM service API.

Table 6.2.6.1-1: Nhss_imsSDM specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| ScscfCapabilityList | 6.2.6.2.2 | Information about mandatory and optional S-CSCF capabilities |
| ImsProfileData | 6.2.6.2.4 | User's IMS profile data |
| RepositoryData | 6.2.6.2.7 | Repository Data for the requested Service Indication |
| MsisdnList | 6.2.6.2.8 | List of MSISDNs associated to the IMS public Identity |
| PublicIdentities | 6.2.6.2.9 | IMS Public Identities which belong to the same Implicit Registration Set (if any) as the requested IMS Public Identity |
| PublicIdentity | 6.2.6.2.10 | IMS Public Identity and the related data (Alias Group Id, IRS default indication, Identity Type) |
| ImsSdmSubscription | 6.2.6.2.11 | A subscription to notifications of data change |
| ImsRegistrationStatus | 6.2.6.2.12 | Registration status of the user. |
| PriorityLevels | 6.2.6.2.13 | Namespaces and priority levels allowed for the IMS public Identity. |
| Ifcs | 6.2.6.2.14 | List of IFCs associated to the IMS public Identity |
| Ifc | 6.2.6.2.15 | Data associated to an individual IFC (Initial Filter Criteria) |
| TriggerPoint | 6.2.6.2.16 | Contains the conditions that should be checked in order to find out if an Application Server should be contacted or not |
| Spt | 6.2.6.2.17 | Contains the data of a Service Point Trigger |
| HeaderSipRequest | 6.2.6.2.18 | Contains a header (and optionally value of the header) in the SIP request |
| SdpDescription | 6.2.6.2.19 | Contains a SDP line (and optionally the value in the line) within the body (if any) of a SIP request |
| ApplicationServer | 6.2.6.2.20 | Application Server which shall be triggered if the conditions of a certain IFC are met, and its associated data |
| ImsLocationData | 6.2.6.2.21 | IMS Location Data (S-CSCF name) |
| ServiceLevelTraceInformation | 6.2.6.2.22 | IMS Service Level Trace Information |
| PsLocation | 6.2.6.2.23 | Location data in PS domain. |
| SgsnLocationData | 6.2.6.2.24 | Location information as retrieved from the SGSN serving node |
| MmeLocationData | 6.2.6.2.25 | Location information as retrieved from the MME serving node |
| AmfLocationData | 6.2.6.2.26 | Location information as retrieved from the AMF serving node |
| TwanLocationData | 6.2.6.2.27 | Location information of the 3GPP AAA Server for Trusted W-LAN Access |
| CsLocation | 6.2.6.2.28 | Location data in CS domain. |
| CsgInformation | 6.2.6.2.29 | Information about a Closed Subscriber Group (CSG) |
| SrvccData | 6.2.6.2.30 | UE SRVCC capability (if available) and STN-SR (if subscribed) |
| PsiActivationState | 6.2.6.2.31 | Public Service Identity activation state |
| ImeiSvInformation | 6.2.6.2.34 | IMEI(SV) information |
| TadsInformation | 6.2.6.2.36 | T-ADS Information |
| UeReachabilitySubscription | 6.2.6.2.37 | A subscription to notifications of UE reachability for IP. |
| UeReachabilityNotification | 6.2.6.2.38 | A notification of UE reachability for IP. |
| PsUserState | 6.2.6.2.39 | User state in PS domain |
| CsUserState | 6.2.6.2.40 | User state in CS domain |
| Csrn | 6.2.6.2.41 | CS domain routeing number |
| ReferenceLocationInformation | 6.2.6.2.42 | Reference Location Information for the user in fixed access networks. |
| SmsRegistrationInfo | 6.2.6.2.43 | SMS Registration Information (IP-SM-GW number and SC address) |
| IpSmGwAddress | 6.2.6.2.44 | IP-SM-GW number and diameter URI/realm |
| ImsAssociatedIdentities | 6.2.6.2.45 | A list of identities belonging to the same Implicit Registration Set (IRS), along with the registration state of the IRS |
| DsaiTagInformation | 6.2.6.2.46 | DSAI tag information (list of DSAI tags activation state for an Application Server) |
| DsaiTagStatus | 6.2.6.2.47 | DSAI tag status (DSAI tag and activation state) |
| CreatedUeReachabilitySubscription | 6.2.6.2.48 | A subscription to notifications of UE reachability for IP. |
| PrivateIdentities | 6.2.6.2.49 | IMS Private Identities |
| PrivateIdentity | 6.3.6.2.50 | IMS Private Identity and the related data (e.g. Identity Type) |
| ScscfSelectionAssistanceInformation | 6.2.6.2.51 | Information used by the I-CSCF to select an S-CSCF for the UE |
| ChargingInfo | 6.2.6.2.52 | Diameter addresses of the charging function |
| RepositoryDataList | 6.2.6.2.53 | List of Repository Data for the requested Service Indications |
| CoreNetworkServiceAuthorization | 6.2.6.2.54 | Core Network Service Authorization |

Table 6.2.6.1-2 specifies data types re-used by the Nhss_imsSDM service API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nhss_imsSDM.

Table 6.2.6.1-2: Nhss_imsSDM re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Uri | 3GPP TS 29.571 [16] | Uniform Resource Identifier |
| SupportedFeatures | 3GPP TS 29.571 [16] | see 3GPP TS 29.500 [4] clause 6.6 |
| ModificationNotification | 3GPP TS 29.503 [15] | Content of the notification after a data change |
| EutraLocation | 3GPP TS 29.571 [16] | E-UTRAN user location |
| NrLocation | 3GPP TS 29.571 [16] | NR user location |
| PlmnId | 3GPP TS 29.571 [16] | PLMN Identity |
| TimeZone | 3GPP TS 29.571 [16] | Time Zone and Daylight Saving Time |
| UtraLocation | 3GPP TS 29.571 [16] | UTRAN user location |
| GeraLocation | 3GPP TS 29.571 [16] | GERAN user location |
| DiameterIdentity | 3GPP TS 29.571 [16] | Diameter URI |
| NfInstanceId | 3GPP TS 29.571 [16] | String uniquely identifying a NF instance |
| RatType | 3GPP TS 29.571 [16] | RAT type |
| StnSr | 3GPP TS 29.571 [16] | Session Transfer Number for SRVCC |
| PatchItem | 3GPP TS 29.571 [16] | Patch Item |
| PatchResult | 3GPP TS 29.571 [16] | Patch Result |
| IpAddr | 3GPP TS 29.571 [16] | IP address (IPv4 address, IPv6 address, or IPv6 Prefix) |
| DateTime | 3GPP TS 29.571 [16] | UTC time |
| AccessType | 3GPP TS 29.571 [16] | Type of access |
| ProblemDetails | 3GPP TS 29.571 [16] | Response body of error response messages. |
| RedirectResponse | 3GPP TS 29.571 [16] | Response body of redirect response messages. |
| Ecgi | 3GPP TS 29.571 [16] | E-UTRAN Cell Global Identity |
| Tai | 3GPP TS 29.571 [16] | Tracking Area Identity |

#### 6.2.6.2	Structured data types

##### 6.2.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.2.6.2.2	Type: ScscfCapabilityList

Table 6.2.6.2.2-1: Definition of type ScscfCapabilityList

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mandatoryCapabilityList | Capabilities | O | 1 | List of S-CSCF capabilities which are mandatory to be supported by the S-CSCF allocated to the subscription the user belongs to. |
| optionalCapabilityList | Capabilities | O | 1 | List of S-CSCF capabilities which are optional or preferred (but not mandatory) to be supported by the S-CSCF allocated to the subscription the user belongs to. |
| NOTE 1: At least one of mandatory or optional capabilities shall be present. |  |  |  |  |

##### 6.2.6.2.3	Type: Capabilities

Table 6.2.6.2.3-1: Definition of type Capabilities

| Type Name | Type Definition | Description |
| --- | --- | --- |
| Capabilities | array(Capability) | This IE contains a list of capabilities |

##### 6.2.6.2.4	Type: ImsProfileData

Table 6.2.6.2.4-1: Definition of type ImsProfileData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supportedFeatures | SupportedFeatures | O | 0..1 | See clause 6.2.8 |
| imsServiceProfiles | array(ImsServiceProfile) | M | 1..N | IMS service profile shared by several public identities when they belong to an IRS. |
| chargingInfo | ChargingInfo | O | 0..1 | A response body containing the Charging Information for the user. |
| serviceLevelTraceInfo | ServiceLevelTraceInformation | O | 0..1 | IMS Service Level Trace information. Syntax described in 3GPP TS 24.323 [24]. |
| servicePriorityLevelList | array(NameSpacePriority) | O | 1..N | List of namespaces and their associated priorities allowed for the user. |
| maxAllowedSimulReg | integer | O | 0..1 | Maximum number of allowed simultaneous registrations for the Public User Identity. |
| servicePriorityLevel | integer | O | 0..1 | Possible values: 0 (Highest priority) 1 2 3 4 (Lowest priority) |

##### 6.2.6.2.5	Type: SharedData

Table 6.2.6.2.5-1: SharedData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sharedDataId | SharedDataId | M | 1 | Identifier of the shared data |
| sharedImsIfcData | Ifcs | O | 0..1 | Shared IFC Subscription Data |
| NOTE 1: The attribute sharedImsIfcData shall not contain sharedDataIds. NOTE 2: When shared data clash with individual data, individual data shall take precedence. |  |  |  |  |

##### 6.2.6.2.6	Void

##### 6.2.6.2.7	Type: RepositoryData

Table 6.2.6.2.7-1: Definition of type RepositoryData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| serviceData | string | M | 1 | The Repository Data for the requested Service Indication. Base64 encoded. |
| sequenceNumber | SequenceNumber | M | 1 | The sequence number associated to the current version of Repository Data. |

##### 6.2.6.2.8	Type: MsisdnList

Table 6.2.6.2.8-1: Definition of type MsisdnList

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| basicMsisdn | Msisdn | M | 1 | The basic (or correlation) MSISDN associated to the IMS Public Identity for the IMS Private Identity indicated (if any) |
| additionalMsisdns | array(Msisdn) | O | 1..N | Additional MSISDNs |

##### 6.2.6.2.9	Type: PublicIdentities

Table 6.2.6.2.9-1: Definition of type PublicIdentities

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| publicIdentities | array(PublicIdentity) | M | 1..N | List of IMS Public Identities associated |

##### 6.2.6.2.10	Type: PublicIdentity

Table 6.2.6.2.10-1: Definition of type PublicIdentity

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| imsPublicId | ImsPublicId | M | 1 | IMS Public Identity (distinct or wildcarded) |
| identityType | IdentityType | M | 1 | Type of IMS Public Identity (e.g. Wildcarded Public Service Identity type) |
| irsIsDefault | boolean | O | 0..1 | Indicates whether the IMS Public Identity is the default within the Implicit Registration Set. Absence and false indicates "non-default". Only one identity in SIP URI format and one identity in TEL URI format (if any) shall have value True for this attribute. |
| aliasGroupId | string | O | 0..1 | Indicates the Alias Public User Identity Group to which the Public User Identity belongs. Absence indicates that the identity does not belong to any Alias Group |

##### 6.2.6.2.11	Type: ImsSdmSubscription

Table 6.2.6.2.11-1: ImsSdmSubscription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| nfInstanceId | NfInstanceId | M | 1 | Identity of the NF Instance creating the subscription. |
| callbackReference | Uri | M | 1 | URI provided by the NF service consumer to receive notifications |
| monitoredResourceUris | array(Uri) | M | 1..N | A set of URIs that identify the resources for which a change triggers a notification. The URI shall take the form of either an absolute URI or an absolute-path reference as defined in IETF RFC 3986 [31]. See NOTE. |
| expires | DateTime | C | 0..1 | If present, indicates the point in time at which the subscription expires. Within a POST request the proposed expiry time is conveyed whereas in a POST response or PATCH response the confirmed expiry time is returned. |
| NOTE: The HSS should handle only the relative-path part (apiSpecificResourceUriPart, see 3GPP TS 29.501 [5] clause 4.4.1) and ignore possible inconsistencies in the base URI part. |  |  |  |  |

##### 6.2.6.2.12	Type: ImsRegistrationStatus

Table 6.2.6.2.12-1: Definition of type ImsRegistrationStatus

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| imsUserStatus | ImsRegistrationState | M | 1 | It contains the registration state of the user or the IRS, as described in 3GPP TS 29.328 [38], clause 7.6.3. |

##### 6.2.6.2.13	Type: PriorityLevels

Table 6.2.6.2.13-1: Definition of type PriorityLevels

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| servicePriorityLevelList | array(NameSpacePriority) | M | 1..N | List of namespaces and their associated priorities allowed for the user. |
| servicePriorityLevel | integer | O | 0..1 | Possible values: 0 (Highest priority) 1 2 3 4 (Lowest priority) |
| messagingPriority | boolean | O | 0..1 | Priority for MPS for messaging true: enabled false or absent: not enabled May be present only if at least one entry in the Service Priority Level List includes a "WPS" namespace |

##### 6.2.6.2.14	Type: Ifcs

Table 6.2.6.2.14-1: Definition of type Ifcs

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ifcList | array(Ifc) | C | 1..N | List of complete IFCs which are associated to the user. |
| cscfFilterSetIdList | array(cscfFilterSetId) | C | 1..N | List of S-CSCF specific filter set identifiers. |
| NOTE: At least one of IfcList or cscfFilterSetIdList shall be present. |  |  |  |  |

##### 6.2.6.2.15	Type: Ifc

Table 6.2.6.2.15-1: Definition of type Ifc

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| priority | integer | M | 1 | Contains the priority of the IFC. The higher the Priority Number the lower the priority of the Filter Criteria is. Minimum: 1 |
| trigger | TriggerPoint | O | 1 | Contains the conditions that should be checked in order to find out if the indicated Application Server should be contacted or not. The absence of this attribute will indicate an unconditional triggering to Application Server. |
| appServer | ApplicationServer | M | 1 | Contains the Application Server which shall be triggered if the conditions are met and its associated data (e.g. default handling) |
| NOTE : See 3GPP TS 29.228 [20], Annex B.2.2 for more details on the terms and concepts associated to Initial Filter Criteria. The ProfilePartIndicator attribute described in Annex B.2.2 is removed and it shall be derived from the SessionCase attribute described in Annex B.2.3. |  |  |  |  |

##### 6.2.6.2.16	Type: TriggerPoint

Table 6.2.6.2.16-1: Definition of type TriggerPoint

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| conditionType | TypeOfCondition | M | 1 | Indicates how the set of SPTs are expressed, i.e. either an Ored set of ANDed sets of SPT statements or an ANDed set of Ored sets of statements. These combinations are termed, respectively, Disjunctive Normal Form (DNF) and Conjunctive Normal Form (CNF) for the SPT (see 3GPP TS 29.228 [20], Annex C). |
| sptList | array(Spt) | M | 1..N | Contains the list of Service Point Triggers |

##### 6.2.6.2.17	Type: Spt

Table 6.2.6.2.17-1: Definition of type Spt

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| conditionNegated | boolean | M | 1 | Indicates if the Service Point Trigger instance is negated (i.e. NOT logical expression) |
| sptGroup | array(SptGroupId) | M | 1..N | Contains the SPT group or list of SPT groups assigned to the SPT. |
| regType | array(RegistrationType) | O | 1..2 | Indicates the type of registration. If included, it shall contain the values that define whether the SPT matches to REGISTER messages that are related to initial registrations, re-registrations, and/or de-registrations. This attribute shall be included only if the SPT is for SIP method with a value "REGISTER" and only if it does not apply to all types of registration; otherwise, it shall be absent (i.e. the SIP Method SPT matches to all REGISTER messages). The attribute regType shall be discarded if it is present in an SPT other than SIP Method with value "REGISTER". |
| requestURI | string | C | 1 | Contains the request-URI of the SIP request (NOTE 2) |
| sipMethod | string | C | 1 | Contains the method of the SIP request (NOTE 2) |
| sipHeader | HeaderSipRequest | C | 1 | Contains a header (and optionally value of the header) in the SIP request (NOTE 2) |
| sessionCase | RequestDirection | O | 1 | Contains the direction of the SIP request as evaluated by the S-CSCF. |
| sessionDescription | SdpDescription | C | 1 | Contains a SDP line (and optionally the valuein the line) within the body (if any) of a SIP request (NOTE 2) |
| NOTE 1: See 3GPP TS 29.228 [20], Annex B.2.3 for more details on the terms and concepts associated to Service Point Trigger. The ProfilePartIndicator attribute described in Annex B.2.2 is removed and it shall be derived from the SessionCase attribute described in Annex B.2.3. NOTE 2: Exactly one of requestURI, method, header and sessionDescription shall be present. See TS 29.228 [20], Annex F for the definition of these attributes. |  |  |  |  |

##### 6.2.6.2.18	Type: HeaderSipRequest

Table 6.2.6.2.18-1: Definition of type HeaderSipRequest

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| header | string | M | 1 | Contains a SIP header |
| content | string | O | 1 | Contains the content of the SIP header. If this attribute is absent, it shall indicate that the SPT is the presence (if conditionNegated="FALSE") or absence (if conditionNegated="TRUE") of the SIP header. |

##### 6.2.6.2.19	Type: SdpDescription

Table 6.2.6.2.19-1: Definition of type SdpDescription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| line | string | M | 1 | Contains a line in the session description |
| content | string | O | 1 | Contains the content of the SDP line. If this attribute is absent, it shall indicate that the SPT is the presence (if conditionNegated="FALSE") or absence (if conditionNegated="TRUE") of the SDP line. |

##### 6.2.6.2.20	Type: ApplicationServer

Table 6.2.6.2.20-1: Definition of type ApplicationServer

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| asUri | string | M | 1 | Contains the Application Server SIP URI, as described in IETF RFC 3261 [22] or 3GPP TS 23.003 [13]. |
| sessionContinue | boolean | O | 1 | This IE indicates whether the SIP dialog shall be continued or released if the Application Server could not be reached. true: indicates that the session shall be continued false or absent: indicates that the session shall be released |
| serviceInfoList | array(ServiceInformation) | O | 1..N | Indicates a list of service information that shall be sent to the Application Server. |

##### 6.2.6.2.21	Type: ImsLocationData

Table 6.2.6.2.21-1: Definition of type ImsLocationData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| scscfName | string | M | 1 | S-CSCF name assigned to the IMS subscription the UE belongs to in the form of a SIP URI, as defined in IETF RFC 3261 [22]. |

##### 6.2.6.2.22	Type: ServiceLevelTraceInformation

Table 6.2.6.2.22-1: Definition of type ServiceLevelTraceInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| serviceLevelTraceInfo | string | M | 1 | IMS Service Level Trace information. Syntax described in 3GPP TS 24.323 [24] |

##### 6.2.6.2.23	Type: PsLocation

Table 6.2.6.2.23-1: Definition of type PsLocation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sgsnLocation | SgsnLocationData | O | 0..1 | Location information as retrieved from SGSN |
| mmeLocation | MmeLocationData | O | 0..1 | Location information as retrieved from MME |
| amfLocation | AmfLocationData | O | 0..1 | Location information as retrieved from AMF |
| twanLocation | TwanLocationData | O | 0..1 | 3GPP AAA Server for TWAN Location information. |
| NOTE: At least one of the "sgsnLocation", "mmeLocation", "amfLocation" or "twanLocation" shall be included. |  |  |  |  |

##### 6.2.6.2.24	Type: SgsnLocationData

Table 6.2.6.2.24-1: Definition of type SgsnLocationData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sgsnNumber | string | M | 1 | SGSN number. See 3GPP TS 23.003 [13] clause 5.1. |
| plmnId | PlmnId | M | 1 | PLMN identity |
| sgsnLocation | UtraLocation | O | 0..1 | User location as retrieved from SGSN. |
| csgInformation | CsgInformation | O | 0..1 | Closed Subscriber Group Information. See 3GPP TS 23.060 [25]. |
| timeZone | TimeZone | O | 0..1 | Local Time Zone information (Time Zone and Daylight Saving Time) of the location in the visited network where the UE is located. |
| ratType | RatType | O | 0..1 | RAT type |

##### 6.2.6.2.25	Type: MmeLocationData

Table 6.2.6.2.25-1: Definition of type MmeLocationData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mmeAddress | DiameterIdentity | M | 1 | MME diameter identity |
| plmnId | PlmnId | M | 1 | PLMN identity |
| mmeLocation | EutraLocation | O | 0..1 | User location as retrieved from MME. |
| csgInformation | CsgInformation | O | 0..1 | Closed Subscriber Group Information. See 3GPP TS 23.060 [25]. |
| timeZone | TimeZone | O | 0..1 | Local Time Zone information (Time Zone and Daylight Saving Time) of the location in the visited network where the UE is located. |
| ratType | RatType | O | 0..1 | RAT type |

##### 6.2.6.2.26	Type: AmfLocationData

Table 6.2.6.2.26-1: Definition of type AmfLocationData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| amfAddress | NfInstanceId | M | 1 | AMF identity. |
| plmnId | PlmnId | M | 1 | PLMN identity |
| amfLocation | NrLocation | O | 0..1 | User location as retrieved from AMF. |
| smsfAddress | NfInstanceId | O | 0..1 | SMSF identity |
| timeZone | TimeZone | O | 0..1 | Local Time Zone information (Time Zone and Daylight Saving Time) of the location in the visited network where the UE is located. |
| ratType | RatType | O | 0..1 | RAT type |

##### 6.2.6.2.27	Type: TwanLocationData

Table 6.2.6.2.27-1: Definition of type TwanLocationData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| twanSsid | string | M | 1 | TWAN SSID, as defined in 3GPP TS 29.273 [26]. |
| plmnId | PlmnId | M | 1 | PLMN identity of the TWAN |
| twanBssid | string | O | 0..1 | TWAN BSSID, as defined in 3GPP TS 29.273 [26]. |
| civicAddress | string | O | 0..1 | Defined in clause 3.4 of IETF RFC 4776 [27] excluding the first 3 octets. Base64 encoded, according to IETF RFC 2045 [28]). |
| twanOperatorName | string | O | 0..1 | TWAN Operator Name, as defined in clause 19.8 of 3GPP TS 23.003 [13]. |
| timeZone | TimeZone | O | 0..1 | Local Time Zone information (Time Zone and Daylight Saving Time) of the location in the visited network where the UE is located. |
| logicalAccessId | string | O | 0..1 | Logical Access ID, as defined in ETSI ES 283 034 [29]. |

##### 6.2.6.2.28	Type: CsLocation

Table 6.2.6.2.28-1: Definition of type CsLocation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mscNumber | string | M | 0..1 | MSC number. See 3GPP TS 23.003 [13] clause 5.1. |
| vlrNumber | string | M | 0..1 | VLR number. See 3GPP TS 23.003 [13] clause 5.1. |
| plmnId | PlmnId | M | 0..1 | PLMN identity |
| vlrLocation | GeraLocation | O | 0..1 | User location as retrieved from MSC/VLR. |
| csgInformation | CsgInformation | O | 0..1 | Closed Subscriber Group Information. See 3GPP TS 23.060 [25]. |
| timeZone | TimeZone | O | 0..1 | Local Time Zone information (Time Zone and Daylight Saving Time) of the location in the visited network where the UE is located. |
| eUtranCgi | Ecgi | O | 0..1 | E-UTRAN Cell Global Identity |
| tai | Tai | O | 0..1 | Tracking Area ID |

##### 6.2.6.2.29	Type: CsgInformation

Table 6.2.6.2.29-1: Definition of type CsgInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| csgId | string | M | 1 | CSG Identity. Syntax described in 3GPP TS 29.002 [30] i.e. 5 octets BER encoded value of 27-bit BIT STRING (Base64 encoded according to IETF RFC 2045 [28]). Length = 8. |
| accessMode | string | O | 0..1 | CSG Access Mode. Syntax described in 3GPP TS 29.002 [30] (Base64 encoded according to IETF RFC 2045 [28]). Length = 4. |
| cMi | boolean | O | 0..1 | CSG Membership Indication 0: CSG membership 1: non CSG membership |

##### 6.2.6.2.30	Type: SrvccData

Table 6.2.6.2.30-1: Definition of type SrvccData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| stnSr | StnSr | M | 1 | Iindicates the STN-SR (Session Transfer Number for SRVCC) of the UE. |
| ueSrvccCapabilities | array(SrvccCapability) | O | 1..N | List of accesses supported by UE SRVCC capability (e.g. 4G, 5G). Absence of this attribute indicates that UE is not SRVCC capable. |

##### 6.2.6.2.31	Type: PsiActivationState

Table 6.2.6.2.31-1: Definition of type PsiActivationState

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| activationState | ActivationState | M | 1 | Indicates whether the PSI is active or inactive |

##### 6.2.6.2.32	Type: ImsServiceProfile

Table 6.2.6.2.32-1: Definition of type ImsServiceProfile

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| publicIdentifierList | array(PublicIdentifier) | M | 1..N | List of public identities sharing the same service profile and its individual data |
| ifcs | Ifcs | O | 1 | List of IFCs and/or shared IFC set identifiers |
| cnServiceAuthorization | CoreNetworkServiceAuthorization | O | 0..1 | Core Network Service Authorization |

##### 6.2.6.2.33	Type: PublicIdentifier

Table 6.2.6.2.33-1: Definition of type PublicIdentifier

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| publicIdentity | PublicIdentity | M | 1 | Public identity information |
| displayName | string | O | 0..1 | Name associated with the public identity |
| imsServicePriority | PriorityLevels | O | 0..1 | Multimedia service priority. Includes MPS for messaging priority indication. |
| serviceLevelTraceInfo | ServiceLevelTraceInformation | O | 0..1 | Service Level Tracing Information |
| barringIndicator | boolean | O | 0..1 | Indicates whether the identity is barred, i.e. it can not be used in any IMS communication except registrations, re-registrations and de-registrations. true: identitiy is barred false or absent: identity is not barred |
| wildcardedImpu | string | O | 0..1 | When identityType is wildcarded IMPU it contains the wildcard id as stored by HSS |

##### 6.2.6.2.34	Type: ImeiSvInformation

Table 6.2.6.2.34-1: Definition of type ImeiSvInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| imei | Imei | C | 1 | Indicates the IMEI |
| imeiSv | ImeiSv | C | 1 | Indicates the IMEISV |
| NOTE: Either imei or imeiSv shall be present. |  |  |  |  |

##### 6.2.6.2.35	Void

##### 6.2.6.2.36	Type: TadsInformation

Table 6.2.6.2.36-1: Definition of type TadsInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| voiceOverPsSessionSupport | ImsVoiceOverPsSessionSupport | M | 1 | Indicates the support for Voice over PS |
| accessType | AccessType | O | 0..1 | Indicates the type of access |
| ratType | RatType | O | 0..1 | Indicates the RAT Type |
| lastUeActivityTime | DateTime | O | 0..1 | Indicates the time of the last radio contact with the UE. |

##### 6.2.6.2.37	Type: UeReachabilitySubscription

Table 6.2.6.2.37-1: Definition of type UeReachabilitySubscription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| expiry | DateTime | M | 1 | Indicates the suggested time at which monitoring should cease and the subscription should become invalid. |
| callbackReference | Uri | M | 1 | Callback URI provided by the subscribing NF service consumer. |

##### 6.2.6.2.38	Type: UeReachabilityNotification

Table 6.2.6.2.38-1: Definition of type UeReachabilityNotification

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| reachabilityIndicator | boolean | M | 1 | Indicates that the UE has been detected reachable for IP. |
| detectingNode | DetectingNode | M | 1 | Indicates the serving node which detected the UE reachability for IP. |
| accessType | AccessType | C | 1 | Indicates the access type (e.g. non 3GPP) where the reachability was detected. It shall be present when the serving node is AMF. |

##### 6.2.6.2.39	Type: PsUserState

Table 6.2.6.2.39-1: Definition of type PsUserState

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sgsnUserState | UserStatePs | C | 0..1 | User state as retrieved from SGSN |
| mmeUserState | UserStatePs | C | 0..1 | User state as retrieved from MME |
| amfUserState | UserStatePs | C | 0..1 | User state as retrieved from AMF |
| NOTE: At least one of the "sgsnUserState", "mmeUserState" or "amfUserState" shall be included. |  |  |  |  |

##### 6.2.6.2.40	Type: CsUserState

Table 6.2.6.2.40-1: Definition of type PsUserState

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mscVlrUserState | UserStateCs | M | 1 | User state as retrieved from MSC/VLR |

##### 6.2.6.2.41	Type: Csrn

Table 6.2.6.2.41-1: Definition of type CsrnInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| csrn | Msisdn | M | 1 | Contains a CS domain routeing number as retrieved from MSC/VLR |

##### 6.2.6.2.42	Type: ReferenceLocationInformation

Table 6.2.6.2.42-1: Definition of type ReferenceLocationInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| accessType | string | C | 1 | Indicates the type of access for which the reference location of the user is defined (e.g. ADSL). |
| accessInfo | string | C | 1 | Indicates the type of the access information defined for the reference location of the user (e.g. dsl-location). |
| accessValue | string | C | 1 | It contains the location information (e.g. line identifier in fixed access networks) as configured by the operator. |
| NOTE 1: At least one of accessType or accessInfo or accessValue shall be present. NOTE 2: The syntax of accessType, accessInfo and accessValue is as described in 3GPP TS 24.229 [18] for P-Access-Network-Info header fields: accessType corresponds to the "access-type" field whereas accessInfo and accessValue correspond to the type and associated value defined for the "access-info" field. |  |  |  |  |

##### 6.2.6.2.43	Type: SmsRegistrationInfo

Table 6.2.6.2.43-1: Definition of type SmsRegistrationInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ipSmGwNumber | Msisdn | M | 1 | IP-SM-GW number |
| ipSmGwAddress | IpSmGwAddress | O | 0..1 | IP-SM-GW addressing information |
| scAddress | Msisdn | O | 0..1 | Short Message Service Centre Address |

##### 6.2.6.2.44	Type: IpSmGwAddress

Table 6.2.6.2.44-1: Definition of type IpSmGwAddress

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ipSmGwNumber | Msisdn | M | 1 | IP-SM-GW number |
| ipSmGwDiaUri | DiameterIdentity | O | 0..1 | IP-SM-GW Diameter Uri. It shall be present if s6c is supported. |
| ipSmGwDiaRealm | DiameterIdentity | O | 0..1 | IP-SM-GW Diameter Realm. It shall be present if s6c is supported. |
| ipSmGwSbiSupInd | boolean | C | 0..1 | SBI Support Indication of the IP-SM-GW. it shall be present if provided in SMS Registration Information Creation or Update procedure (see clauses 5.3.2.7.6) Contains the indication on whether or not the IP-SM-GW is expecting to support SBI-based MT SM transmit. - true: the IP-SM-GW supports SBI for MT SM transmit. - false, or absence of this attribute: the IP-SM-GW do not support SBI for MT SM transmit. |
| ipSmGwInstanceId | NfInstanceId | O | 0..1 | NF Instance ID of the IP-SM-GW |

##### 6.2.6.2.45	Type: ImsAssociatedIdentities

Table 6.2.6.2.45-1: Definition of type ImsAssociatedIdentities

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| irsState | ImsRegistrationState | M | 1 | It contains the registration state of the Implicit Registration Set. |
| publicIdentities | PublicIdentities | M | 1 | List of IMS Public Identities associated. |

##### 6.2.6.2.46	Type: DsaiTagInformation

Table 6.2.6.2.46-1: Definition of type DsaiTagInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dsaiTagStatusList | array(DsaiTagStatus) | M | 1..N | List of DSAI Tags associated to the Application Server Name. |

##### 6.2.6.2.47	Type: DsaiTagStatus

Table 6.2.6.2.47-1: Definition of type DsaiTagStatus

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dsaiTag | string | M | 1 | DSAI tag associated to the Application Server Name. |
| dsaiStatus | ActivationState | M | 1 | Status of DSAI tag |

##### 6.2.6.2.48	Type: CreatedUeReachabilitySubscription

Table 6.2.6.2.48-1: Definition of type CreatedUeReachabilitySubscription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| expiry | DateTime | M | 1 | Indicates the granted time at which monitoring will cease and the subscription becomes invalid. |

##### 6.2.6.2.49	Type: PrivateIdentities

Table 6.2.6.2.49-1: Definition of type PrivateIdentities

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| privateIdentities | array(PrivateIdentity) | M | 1..N | List of IMS Private Identities associated. |

##### 6.2.6.2.50	Type: PrivateIdentity

Table 6.2.6.2.50-1: Definition of type PrivateIdentity

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| privateIdentity | PrivateId | M | 1 | IMS Private Identity. |
| privateIdentityType | PrivateIdentityType | M | 1 | Type of IMS Private Identity (e.g. IMPI or IMSI). |

##### 6.2.6.2.51	Type: ScscfSelectionAssistanceInformation

Table 6.2.6.2.51-1: Definition of type ScscfSelectionAssistanceInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| scscfCapabilityList | ScscfCapabilityList | C | 0..1 | Information about mandatory and optional S-CSCF capabilities. Shall be present if scscfNames is absent; may be present otherwise. |
| scscfNames | array(string) | C | 1..N | a list of S-CSCF names the UE can be steered to. Each strings shall contain a SIP-URL as defined in IETF RFC 3261 [22] to identify a SIP server (S-CSCF) Shall be present if scscfCapabilityList is absent; may be present otherwise. |

##### 6.2.6.2.52	Type: ChargingInfo

Table 6.2.6.2.52-1: Definition of type ChargingInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| primaryEventChargingFunctionName | DiameterIdentity | C | 0..1 | See 3GPP TS 29.328 |
| secondaryEventChargingFunctionName | DiameterIdentity | O | 0..1 | See 3GPP TS 29.328 |
| primaryChargingCollectionFunctionName | DiameterIdentity | C | 0..1 | See 3GPP TS 29.328 |
| secondaryChargingCollectionFunctionName | DiameterIdentity | O | 0..1 | See 3GPP TS 29.328 |
| NOTE: At least primaryEventChargingFunctionName or primaryChargingCollectionFunctionName shall be present |  |  |  |  |

##### 6.2.6.2.53	Type: RepositoryDataList

Table 6.2.6.2.53-1: Definition of type RepositoryDataList

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| repositoryDataMap | map(RepositoryData) | M | 1..N | A map (list of key-value pairs where ServiceIndication serves as key) of RepositoryData. |

##### 6.2.6.2.54	Type: CoreNetworkServiceAuthorization

Table 6.2.6.2.54-1: Definition of type CoreNetworkServiceAuthorization

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| subscribedMediaProfileId | integer | O | 0..1 | Subscribed Media Profile ID |

#### 6.2.6.3	Simple data types and enumerations

##### 6.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.2.6.3.2	Simple data types

The simple data types defined in table 6.2.6.3.2-1 shall be supported.

Table 6.2.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| Capability | integer | S-CSCF capability |
| SequenceNumber | integer | Unsigned integer containing the sequence number associated to the current version of Repository Data |
| ServiceIndication | string | String containing the Service Indication or Service Identifier |
| Msisdn | string | String containing an additional or basic MSISDN Pattern: "[0-9]{5,15}$" |
| PrivateId | string | String containing a Private User Identity or a Private Service Identity. |
| ImsPublicId | string | String containing an IMS Public Identity in SIP URI format or TEL URI format, as specified 3GPP TS 23.003 [13] Pattern: "^(sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦tel\:\+[0-9]{5,15})$" |
| NameSpacePriority | string | String containing the namespace and priority. It shall be formatted as "r-value" parameter described in IETF RFC 4412 [19]. See also 3GPP TS 24.229 [18]. Pattern: "^[0-9a-zA-Z-\!%\*_\+`\'~]+.[0-9a-zA-Z-\!%\*_\+`\'~]+$" |
| CscfFilterSetId | integer | Contains the identifier of a specific filter set in the S-CSCF. Minimum=0 |
| SptGroupId | integer | Contains the identifier of the Group of SPTs. Minimum=0 |
| SharedDataId | string | Identifies globally and uniquely a piece of subscription data shared by multiple UEs. The value shall start with the HPLMN id (MCC/MNC) followed by a hyphen followed by a local Id as allocated by the home network operator. Pattern: "[0-9]{5,6}-.+" |
| PrePaging | boolean | Inidicates the support of pre-paging. |
| SipServerName | string | Server Name (e.g. AS name, CSCF name) in SIP URI format Pattern: "^(sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,})$" |

##### 6.2.6.3.3	Enumeration: DataSetName

The enumeration DataSetName represents a Set of Data within the IMS User Profile. It shall comply with the provisions defined in table 6.2.6.3.3-1.

Table 6.2.6.3.3-1: Enumeration DataSetName

| Enumeration value | Description |
| --- | --- |
| "CHARGING_DATA" | Charging Information Data |
| "IFC_DATA" | Initial Filter Criteria Data |
| "TRACE_DATA" | IMS service level trace information |
| "PRIORITY_DATA" | Service Priority Levels |

##### 6.2.6.3.4	Enumeration:  IdentityType

The enumeration IdentityType represents the type of IMS Public Identity. It shall comply with the provisions defined in table 6.2.6.3.4-1.

Table 6.2.6.3.4-1: Enumeration IdentityType

| Enumeration value | Description |
| --- | --- |
| "DISTINCT_IMPU" | The identity is an IMS Public User Identity |
| "DISTINCT_PSI" | The identity is an IMS Public Service Identity |
| "WILDCARDED_IMPU" | The identity is an IMS Wildcarded Public User Identity |
| "WILDCARDED_PSI" | The identity is an IMS Wildcarded Public Service Identity |

##### 6.2.6.3.5	Enumeration:  ImsRegistrationState

The enumeration ImsRegistrationState represents the IMS status of the user or Public Identity or the IRS. It shall comply with the provisions defined in table 6.2.6.3.5-1

Table 6.2.6.3.5-1: Enumeration ImsRegistrationState

| Enumeration value | Description |
| --- | --- |
| "REGISTERED" | The identity is explicitly registered by the user. |
| "NOT_REGISTERED" | The identity is not registered. |
| "AUTHENTICATION_PENDING" | The identity initiated a registration and its authentication is ongoing (authentication pending flag is set). |
| "REGISTERED_UNREG_SERVICES" | The identity is not registered but has an S-CSCF assigned. |
| NOTE: See clause 3.1 for more details on the IMS registration states and terms. |  |

##### 6.2.6.3.6	Enumeration: TypeOfCondition

The enumeration TypeOfCondition represents the type of condition to be applied to the logical expression of SPT groups and SPTs. See 3GPP TS 29.228 [20], Annex C.

Table 6.2.6.3.6-1: Enumeration TypeOfCondition

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "CNF" | Conjunctive form |  |
| "DNF" | Disjunctive form |  |

##### 6.2.6.3.7	Enumeration: RegistrationType

The enumeration RegistrationType represents the type of registration associated to the REGISTER request.

Table 6.2.6.3.7-1: Enumeration RegistrationType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "INITIAL_REGISTRATION" | The request is related to an initial registration |  |
| "RE_REGISTRATION" | The request is related to a re-registration |  |
| "DE_REGISTRATION" | The request is related to a de-registration |  |

##### 6.2.6.3.8	Enumeration: RequestDirection

The enumeration RequestDirection represents the direction of the request in combination with the registration status of the user as evaluated in the S-CSCF.

Table 6.2.6.3.8-1: Enumeration RequestDirection

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "ORIGINATING_REGISTERED" | The request is originated by an explicitly registered user. |  |
| "ORIGINATING_UNREGISTERED" | The request is originated by a non-registered user. |  |
| "ORIGINATING_CDIV " | The REGISTER request is related to an originating request after call diversion service (regardless of the registration state). |  |
| "TERMINATING_REGISTERED" | The request is targeted to an explicitly registered user. |  |
| "TERMINATING _UNREGISTERED" | The request is targeted to a non-registered user. |  |

##### 6.2.6.3.9	Enumeration: ServiceInformation

The enumeration ServiceInformation indicates the type of additional information to be included in the body of the SIP request towards the Application Server.

Table 6.2.6.3.9-1: Enumeration ServiceInformation

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "INCLUDE_REGISTER_REQUEST" | Indicates to the S-CSCF that the incoming SIP REGISTER request is to be transferred to the Application Server. |  |
| "INCLUDE_REGISTER_RESPONSE" | Indicates to the S-CSCF that the final SIP response to the incoming SIP REGISTER request is to be transferred to the Application Server. |  |
| NOTE: See 3GPP TS 23.218 [43] for the use of this information. |  |  |

##### 6.2.6.3.10	Enumeration: RequestedNode

The enumeration RequestedNode represents serving node requested. It shall comply with the provisions defined in table 6.2.6.3.10-1.

Table 6.2.6.3.10-1: Enumeration RequestedNode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "SGSN" | The requested node is SGSN. |  |
| "MME" | The requested node is MME. |  |
| "AMF" | The requested node is AMF. |  |
| "3GPP_AAA_SERVER_TWAN" | The requested node is 3GPP-AAA-SERVER-TWAN |  |

##### 6.2.6.3.11	Enumeration:  SrvccCapability

The enumeration SrvccCapability represents the type of UE support for SRVCC. It shall comply with the provisions defined in table 6.2.6.3.11-1.

Table 6.2.6.3.11-1: Enumeration IdentityType

| Enumeration value | Description |
| --- | --- |
| "UE_4G_SRVCC_CAPABLE" | UE is 4G SRVCC capable |
| "UE_5G_SRVCC_CAPABLE" | UE is 5G SRVCC capable |

##### 6.2.6.3.12	Enumeration:  ActivationState

The enumeration ActivationState represents the activation state of a PSI or DSAI tag. It shall comply with the provisions defined in table 6.2.6.3.12-1.

Table 6.2.6.3.12-1: Enumeration ActivationState

| Enumeration value | Description |
| --- | --- |
| "ACTIVE" |  |
| "INACTIVE" |  |

##### 6.2.6.3.13	Enumeration: ImsVoiceOverPsSessionSupport

The enumeration ImsVoiceOverPsSessionSupport represents the support for Voice over PS associated to the UE.

Table 6.2.6.3.13-1: Enumeration ImsVoiceOverPsSessionSupport

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "IMS_VOICE_OVER_PS_NOT_SUPPORTED" | Voice over PS is supported for the UE |  |
| "IMS_VOICE_OVER_PS_SUPPORTED" | Voice over PS is not supported for the UE |  |
| "IMS_VOICE_OVER_PS_SUPPORT_UNKNOWN" | Voice over PS support for the UE is unknown |  |

##### 6.2.6.3.14	Enumeration: AccessType

The enumeration AccessType represents the type of access. It shall comply with the provisions defined in table 6.2.6.3.14-1.

Table 6.2.6.3.14-1: Enumeration AccessType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "3GPP_ACCESS" |  |  |
| "NON_3GPP_ACCESS" |  |  |

##### 6.2.6.3.15	Enumeration: UserStatePs

The enumeration UserStatePs represents the user state in PS domain.

Table 6.2.6.3.15-1: Enumeration ImsVoiceOverPsSessionSupport

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "DETACHED" |  |  |
| "ATTACHED_NOT_REACHABLE_FOR_PAGING" |  |  |
| "ATTACHED_REACHABLE_FOR_PAGING" |  |  |
| "CONNECTED_NOT_REACHABLE_FOR_PAGING" |  |  |
| "CONNECTED_REACHABLE_FOR_PAGING" |  |  |
| "NOT_PROVIDED_FROM_SGSN_OR_MME_OR_AMF" |  |  |
| "NETWORK_DETERMINED_NOT_REACHABLE" |  |  |
| NOTE: See 3GPP TS 29.272 [32] for PS/EPS User State and 3GPP TS 29.518 [33] for the 5GS User State |  |  |

##### 6.2.6.3.16	Enumeration: UserStateCs

The enumeration UserStateCs represents the user state in CS domain.

Table 6.2.6.3.16-1: Enumeration ImsVoiceOverPsSessionSupport

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "CAMEL_BUSY" |  |  |
| "NETWORK_DETERMINED_NOT_REACHABLE" |  |  |
| "ASSUMED_IDLE" |  |  |
| "NOT_PROVIDED_FROM_VLR" |  |  |
| NOTE: See 3GPP TS 23.078 [31] for CS Subscriber State |  |  |

##### 6.2.6.3.17	Enumeration: PrivateIdentityType

The enumeration PrivateIdentityType represents the type of IMS Private Identity. It shall comply with the provisions defined in table 6.2.6.3.17-1.

Table 6.2.6.3.17-1: Enumeration PrivateIdentityType

| Enumeration value | Description |
| --- | --- |
| "IMPI" | The identity is an IMS Private User Identity. |
| "IMSI" | The identity is an IMSI. |

### 6.2.7	Error Handling

#### 6.2.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [4].

#### 6.2.7.2	Protocol Errors

Protocol errors handling shall be supported as specified in clause 5.2.7 of 3GPP TS 29.500 [4].

#### 6.2.7.3	Application Errors

The common application errors defined in the Table 5.2.7.2-1 in 3GPP TS 29.500 [4] may also be used for the Nhss_imsSDM service. The following application errors listed in Table 6.2.7.3-1 are specific for the Nhss_imsSDM service.

Table 6.2.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| USER_NOT_FOUND | 404 Not Found | The user does not exist in the HPLMN |
| SERVICE_NOT_FOUND | 404 Not Found | The service (PSI) does not exist in the HPLMN |
| DATA_NOT_FOUND | 404 Not Found | The requested data is not found/does not exist. Additional information (if any) about this application error is indicated for each specific resource. |
| CONTEXT_NOT_FOUND | 404 Not Found | It is used during the modification of an existing subscription when no corresponding context exists. |
| OPERATION_NOT_ALLOWED | 403 Forbidden | The requested data is not allowed to be accessed by the service consumer, e.g. the IMS-AS does not have permissions for the requested resource and/or HTTP method. |
| OUT_OF_SYNC | 409 Conflict | The request could not be completed due to a conflict with the current state of the target resource (wrong version or sequence number) |
| TOO_MUCH_DATA | 413 Content Too Large | The request could not be completed due to large amount of data (e.g. size of repository data is too large). This response may be due to constraints applied by the service producer, e.g. based on local policy. |

### 6.2.8	Feature negotiation

The optional features in table 6.2.8-1 are defined for the Nhss_imsSDM API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | SharedData | When receiving a Nhss_ismSDM_Get service operation request to retrieve a UE's IMS profile data, and the request does not contain a supported-features query parameter indicating support of this feature, the HSS shall not include Shared Data Ids in the response. Instead the HSS may – based on operator policy – take no further action (i.e. allow the UE to get services based on only the UE's individual IMS profile data), or send the shared data as individual data (this may result in notifications of individual subscription data change – if so subscribed – when shared data, which are sent as individual data, are modified, and/or when the UE's Shared Data IDs are modified). |
| 2 | PatchReport | If some of the modifications included in the PATCH request are not successfully implemented, the HSS reports the result of PATCH request execution to the consumer. See clause 5.2.7.2 of 3GPP TS 29.500 [4]. |

### 6.2.9	Security

As indicated in 3GPP TS 33.501 [5], the access to the Nhss_imsSDM API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [35]), using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [36]) plays the role of the authorization server.

If Oauth2 authorization is used, an NF Service Consumer, prior to consuming services offered by the Nhss_imsSDM API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [36], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nhss_imsSDM service.

The Nhss_imsSDM API defines the following scopes for OAuth2 authorization:

Table 6.2.9-1: Oauth2 scopes defined in Nhss_imsSDM API

| Scope | Description |
| --- | --- |
| "nhss-ims-sdm" | Access to the Nhss IMS Subscription Data Management API |
| "nhss-ims-sdm:registration-status:read" | Access to read the Registration Status resource |
| "nhss-ims-sdm:profile-data:read" | Access to read the Profile Data resource |
| "nhss-ims-sdm:priority-levels:read" | Access to read the Priority Levels resource |
| "nhss-ims-sdm:ifcs:read" | Access to read the Initial Filter Criteria resource |
| "nhss-ims-sdm:service-level-trace-information:read" | Access to read the Service Level Trace Information resource |
| "nhss-ims-sdm:server-name:read" | Access to read the Server Name resource |
| "nhss-ims-sdm:scscf-capabilities:read" | Access to read the S-CSCF Capabilities resource |
| "nhss-ims-sdm:ps-domain:location-data:read" | Access to read the PS-Domain Location Data resource |
| "nhss-ims-sdm:ps-domain:ip-address:read" | Access to read the PS-Domain IP Address resource |
| "nhss-ims-sdm:ps-domain:tads-info:read" | Access to read the PS-Domain TADS Info resource |
| "nhss-ims-sdm:ps-domain:ue-reach-subscriptions:create" | Access to create PS-Domain UE Reachability Subscriptions resources |
| "nhss-ims-sdm:ps-domain:ue-reach-subscriptions:modify" | Access to update/delete a PS-Domain UE Reachability Subscription resource |
| "nhss-ims-sdm:ps-domain:user-state:read" | Access to read the PS-Domain User State resource |
| "nhss-ims-sdm:cs-domain:location-data:read" | Access to read the CS-Domain Location Data resource |
| "nhss-ims-sdm:cs-domain:user-state:read" | Access to read the CS-Domain User State resource |
| "nhss-ims-sdm:cs-domain:csrn:read" | Access to read the CS-Domain CSRN resource |
| "nshh-ims-sdm:wireline-domain:reference-location:read" | Access to read the Wireline-Domain Reference Location resource |
| "nhss-ims-sdm:repository-data:modify" | Access to create/update/delete the Repository Data resource |
| "nhss-ims-sdm:repository-data:read" | Access to read the Repository Data resource |
| "nhss-ims-sdm:identities:read" | Access to read the Identities resource |
| "nhss-ims-sdm:srvcc:read" | Access to read the SRVCC resource |
| "nhss-ims-sdm:srvcc:modify" | Acess to update the SRVCC resource |
| "nhss-ims-sdm:psi-status:read" | Access to read the PSI Status resource |
| "nhss-ims-sdm:psi-status:modify" | Acess to update the PSI Status resource |
| "nhss-ims-sdm:dsai:read" | Access to read the DSAI resource |
| "nhss-ims-sdm:dsai:modify" | Access to update the DSAI resource |
| "nhss-ims-sdm:sms-registration-info:read" | Access to read the SMS Registration Info resource |
| "nhss-ims-sdm:sms-registration-info:modify" | Access to create/update/delete the SMS Registration Info resource |
| "nhss-ims-sdm:subscriptions:create" | Access to create Subscriptions resources |
| "nhss-ims-sdm:subscription:modify" | Access to update/delete a Subscription resource |
| "nhss-ims-sdm:shared-subscriptions:create" | Access to create a Shared-Data Subscriptions resource |
| "nhss-ims-sdm:shared-subscriptions:modify" | Access to update/delete a Shared-Data Subscription resource |
| "nhss-ims-sdm:shared-subscriptions:read" | Access to read the Shared-Data resource |
| "nhss-ims-sdm:charging-info:read" | Access to read the Charging-Information resource |

### 6.2.10	HTTP redirection

An HTTP request may be redirected to a different HSS service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different HSS producer instance will return the NF Instance ID of the new HSS producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an HSS redirects a service request to a different HSS using an 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new HSS towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

## 6.3	Nhss_imsUEAuthentication Service API

### 6.3.1	API URI

URIs of this API shall have the following root:

{apiRoot}/{apiName}/{apiVersion}/

where "apiRoot" is defined in clause 4.4.1 of 3GPP TS 29.501 [5], the "apiName" shall be set to "nhss-ims-ueau" and the "apiVersion" shall be set to "v1" for the current version of this specification.

### 6.3.2	Usage of HTTP

#### 6.3.2.1	General

HTTP/2, as defined in IETF RFC 9113 [8], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

HTTP messages and bodies for the Nhss_imsUEAU service shall comply with the OpenAPI [9] specification contained in Annex A.3.

#### 6.3.2.2	HTTP standard headers

##### 6.3.2.2.1	General

The usage of HTTP standard headers shall be supported as specified in clause 5.2.2 of 3GPP TS 29.500 [4].

##### 6.3.2.2.2	Content type

The following content types shall be supported:

-	JSON, as defined in IETF RFC 8259 [10], signalled by the content type "application/json".

-	The Problem Details JSON Object (IETF RFC 9457 [11] signalled by the content type "application/problem+json"

-	JSON Patch, as defined in IETF RFC 6902 [12], signalled by the content type "application/json-patch+json"

#### 6.3.2.3	HTTP custom headers

##### 6.3.2.3.1	General

The usage of HTTP custom headers shall be supported as specified in clause 5.2.3 of 3GPP TS 29.500 [4].

### 6.3.3	Resources

#### 6.3.3.1	Overview

Figure 6.3.3.1-1: Resource URI structure of the Nhss_imsUEAU API

Table 6.3.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.3.3.1-1: Resources and methods overview

| Resource name (Archetype) | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| SecurityInformation (Custom operation) | /{impi}/security-information/generate-sip-auth-data | generate-sip-auth-data (POST) | The HSS takes the variable {impi} and, if applicable, fetches the corresponding SUPI. The HSS calculates a fresh authentication vector based on the received information and the stored security information, if IMS-AKA is selected. Otherwise, the HSS provides the corresponding authentication information. |

#### 6.3.3.2	Resource: SecurityInformation

##### 6.3.3.2.1	Description

This resource represents the information that is needed together with the SIP authenticacion scheme to calculate a fresh authentication vector.

##### 6.3.3.2.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-ueau/v1/{impi}/security-information

This resource shall support the resource URI variables defined in table 6.3.3.2.2-1.

Table 6.3.3.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.3.1 |
| impi | Represents the IMS Private Identity (see 3GPP TS 23.003 [13] clause 13.3. Pattern: "^(impi-.+¦.+)$" |

##### 6.3.3.2.3	Resource Standard Methods

No Standard Methods are supported for this resource.

##### 6.3.3.2.4	Resource Custom Operations

6.3.3.2.4.1	Overview

Table 6.3.3.2.4.1-1: Custom operations

| Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- |
| /generate-sip-auth-data | POST | Select the authentication method and calculate a fresh AV if IMS-AKA is selected or provides corresponding authentication information. |

6.3.3.2.4.2	Operation: generate-sip-auth-data

6.3.3.2.4.2.1	Description

This custom operation is used by the NF service consumer (S-CSCF) to request authentication information data for the IMPI from the HSS. The HSS calculates an authentication vector taking into account the information received from the NF service consumer (S-CSCF) and the current representation of this resource if IMS AKA is selected. For details see 3GPP TS 33.203 [14].

6.3.3.2.4.2.2	Operation Definition

This operation shall support the request data structures specified in table 6.3.3.2.4.2.2-1 and the response data structure and response codes specified in table 6.3.3.2.4.2.2-2.

Table 6.3.3.2.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SipAuthenticationInfoRequest | M | 1 | Contains the SIP Authentication Scheme, the number of authentication items and Resynchronization Information |

Table 6.3.3.2.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SipAuthenticationInfoResult | M | 1 | 200 OK | Upon success, a response body containing the selected authentication method and an authentication vector if IMS AKA has been selected shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - USER_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - AUTHENTICATION_REJECTED - UNSUPPORTED_SIP_AUTH_SCHEME |

Table 6.3.3.2.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.3.3.2.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

### 6.3.4	Custom Operations without associated resources

In this release of this specification, no custom operations without associated resources are defined for the Nhss_imsUEAuthentication Service.

### 6.3.5	Notifications

In this release of this specification, no notifications are defined for the Nhss_imsUEAuthentication Service.

### 6.3.6	Data Model

#### 6.3.6.1	General

This clause specifies the application data model supported by the API.

Table 6.3.6.1-1 specifies the structured data types defined for the Nhss_imsUEAU service API.

Table 6.3.6.1-1: Nhss_imsUEAU specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SipAuthenticationInfoRequest | 6.3.6.2.2 | Contains SIP Authentication Schema, Authentication Items and Resynchronization Information |
| SipAuthenticationInfoResult | 6.3.6.2.3 | Contains authentication information (e.g. authentication vectors, line id) |
| ResynchronizationInfo | 6.3.6.2.4 | Contains RAND and AUTS |
| 3GAkaAv | 6.3.6.2.5 | Contains RAND, XRES, AUTN, CK, and IK |

Table 6.3.6.1-2 specifies data types re-used by the Nhss_imsUEAU service API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nhss_imsUEAU.

Table 6.3.6.1-2: Nhss_imsUEAU re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| ProblemDetails | 3GPP TS 29.571 [16] | Response body of error response messages. |
| RedirectResponse | 3GPP TS 29.571 [16] | Response body of redirect response messages. |
| IpAddr | 3GPP TS 29.571 [16] | IP Address (either IPv4 address, IPv6 address or IPv6 prefix) |
| Autn | 3GPP TS 29.503 [15] | AUTN component of the Authentication Vector |
| Auts | 3GPP TS 29.503 [15] | AUTS component of the Resynchronization Info |
| Rand | 3GPP TS 29.503 [15] | RAND component of the Authentication Vector or the Resynchronization Info |
| Xres | 3GPP TS 29.503 [15] | XRES component of the Authentication Vector |
| ConfidentialityKey | 3GPP TS 29.503 [15] | CK component of the Authentication Vector |
| IntegrityKey | 3GPP TS 29.503 [15] | IK component of the Authentication Vector |

#### 6.3.6.2	Structured data types

##### 6.3.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.3.6.2.2	Type: SipAuthenticationInfoRequest

Table 6.3.6.2.3-1: Definition of type AuthenticationInfoRequest

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| cscfServerName | string | M | 1 | S-CSCF name in SIP URI format |
| sipAuthenticationScheme | SipAuthenticationScheme | M | 1 | SIP Authentication Scheme requested by service consumer (S-CSCF) |
| sipNumberAuthItems | SipNumberAuthItems | O | 0..1 | Number of authentication items (AVs). Absence indicates that only one AV is requested. This attribute shall be absent for authentication schemes other than IMS AKA. |
| resynchronizationInfo | ResynchronizationInfo | O | 0..1 | Contains RAND and AUTS; see 3GPP TS 33.203 [14]. This attribute shall be included only if a resynchronization procedure needs to be triggered. This attribute shall be absent for authentication schemes other than IMS AKA. |

##### 6.3.6.2.3	Type: SipAuthenticationInfoResult

Table 6.3.6.2.3-1: Definition of type AuthenticationInfoResult

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sipAuthenticationScheme | SipAuthenticationScheme | M | 1 | SIP Authentication Scheme used by service producer (HSS) |
| 3gAkaAvs | array(3GAkaAv) | O | 1..N | If IMS AKA is requested, it shall be present and will contains Authentication Vectors. |
| digestAuth | DigestAuthentication | O | 0..1 | If SIP Digest is used, it shall be present and shall contain the digest realm, algorithm, QoP and HA1. |
| lineIdentifierList | array(LineIdentifier) | O | 0..N | If NBA is used, it shall be present and shall contain a list of fixed broadband access line identifiers associated to the user. |
| ipAddress | IpAddr | O | 0..1 | If GIBA is used, it shall be present and shall contain an IP address (IPv4, or IPv6, or IPv6 prefix) |

##### 6.3.6.2.4	Type: ResynchronizationInfo

Table 6.3.6.2.4-1: Definition of type ResynchronizationInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| rand | Rand | M | 1 |  |
| auts | Auts | M | 1 |  |

##### 6.3.6.2.5	Type: 3GAkaAv

Table 6.3.6.2.5-1: Definition of type 3gAkaAv

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| rand | Rand | M | 1 | RAND component of the Authentication Vector |
| xres | Xres | M | 1 | XRES component of the Authentication Vector |
| autn | Autn | M | 1 | AUTN component of the Authentication Vector |
| ck | ConfidentialityKey | M | 1 | CK component of the Authentication Vector |
| ik | IntegrityKey | M | 1 | IK component of the Authentication Vector |

##### 6.3.6.2.6	Type: DigestAuthentication

Table 6.3.6.2.6-1: Definition of type DigestAuthentication

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| digestRealm | string | M | 1 | Digest realm. See IETF RFC 4740 [17]. |
| digestAlgorithm | SipDigestAlgorithm | M | 1 | Digest Algorithm. See IETF RFC 4740 [17]. |
| digestQop | SipDigestQop | M | 1 | Quality of Protection. See IETF RFC 4740 [17]. |
| ha1 | string | M | 1 | Hash H(A1). See IETF RFC 4740 [17]. |

##### 6.3.6.2.7	Void

#### 6.3.6.3	Simple data types and enumerations

##### 6.3.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.3.6.3.2	Simple data types

The simple data types defined in table 6.3.6.3.2-1 shall be supported.

Table 6.3.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| SipNumberAuthItems | integer | Number of authentication items Minimum: 1 |
| Impi | string | IMS Private User Identity |
| LineIdentifier | string | Line Identifier for the wireline access |

##### 6.3.6.3.3	Enumeration: SipAuthenticationScheme

The enumeration SipAuthenticationScheme represents an IMS authentication scheme. It shall comply with the provisions defined in table 6.3.6.3.3-1.

Table 6.3.6.3.3-1: Enumeration SipAuthenticationScheme

| Enumeration value | Description |
| --- | --- |
| "DIGEST-AKAV1-MD5" | IMS-AKA authentication scheme |
| "DIGEST-HTTP" | SIP Digest authentication scheme |
| "NBA" | NASS Bundled authentication scheme |
| "GIBA" | GPRS-IMS-Bundled authentication scheme |
| "UNKNOWN" | Authentication scheme to be used is unknown by the service consumer |

##### 6.3.6.3.4	Enumeration: SipDigestAlgorithm

The enumeration SipDigestAlgorithm represents the algorithm for SIP Digest scheme. It shall comply with the provisions defined in table 6.3.6.3.4-1.

Table 6.3.6.3.4-1: Enumeration SipDigestAlgorithm

| Enumeration value | Description |
| --- | --- |
| "MD5" |  |
| "MD5_SESS" |  |

##### 6.3.6.3.5	Enumeration: SipDigestQop

The enumeration SipDigestQop represents the quality of protection for SIP Digest scheme. It shall comply with the provisions defined in table 6.3.6.3.5-1.

Table 6.3.6.3.5-1: Enumeration SipDigestQop

| Enumeration value | Description |
| --- | --- |
| "AUTH" | Indicates authentication |
| "AUTH_INT" | Indicates authentication with integrity protection |

### 6.3.7	Error Handling

#### 6.3.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [4].

#### 6.3.7.2	Protocol Errors

Protocol errors handling shall be supported as specified in clause 5.2.7 of 3GPP TS 29.500 [4].

#### 6.3.7.3	Application Errors

The common application errors defined in the Table 5.2.7.2-1 in 3GPP TS 29.500 [4] may also be used for the Nhss_imsUEAuthentication service. The following application errors listed in Table 6.3.7.3-1 are specific for the Nhss_imsUEAuthentication service.

Table 6.3.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| AUTHENTICATION_REJECTED | 403 Forbidden | The user cannot be authenticated with this authentication method |
| USER_NOT_FOUND | 404 Not Found | The user does not exist in the HPLMN |
| UNSUPPORTED_SIP_AUTHENTICATION_SCHEME | 501 Not implemented | The received SIP authentication scheme is not supported by HPLMN |

### 6.3.8	Feature Negotiation

### 6.3.9	Security

As indicated in 3GPP TS 33.501 [5], the access to the Nhss_imsUEAU API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [35]), using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [36]) plays the role of the authorization server.

If Oauth2 authorization is used, an NF Service Consumer, prior to consuming services offered by the Nhss_imsUEAU API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [36], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nhss_imsUEAU service.

The Nhss_imsUEAU API defines the following scopes for OAuth2 authorization:

Table 6.3.9-1: Oauth2 scopes defined in Nhss_imsUEAU API

| Scope | Description |
| --- | --- |
| "nhss-ims-ueau" | Access to the Nhss IMS UE Context Management API |
| "nhss-ims-ueau:generate-sip-auth-data:invoke" | Access to invoke the Generate SIP Auth Data custom method |

### 6.3.10	HTTP redirection

An HTTP request may be redirected to a different HSS service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different HSS producer instance will return the NF Instance ID of the new HSS producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an HSS redirects a service request to a different HSS using an 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new HSS towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

## 6.4	Nhss_gbaSubscriberDataManagement Service API

### 6.4.1	API URI

URIs of this API shall have the following root:

{apiRoot}/{apiName}/{apiVersion}

where "apiRoot" is defined in clause 4.4.1 of 3GPP TS 29.501 [5], the "apiName" shall be set to "nhss-gba-sdm" and the "apiVersion" shall be set to "v1" for the current version of this specification.

### 6.4.2	Usage of HTTP

#### 6.4.2.1	General

HTTP/2, as defined in IETF RFC 9113 [8], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

HTTP messages and bodies for the Nhss_gbaSubscriberDataManagement service shall comply with the OpenAPI [9] specification contained in Annex A.5.

#### 6.4.2.2	HTTP standard headers

##### 6.4.2.2.1	General

The usage of HTTP standard headers shall be supported as specified in clause 5.2.2 of 3GPP TS 29.500 [4].

##### 6.4.2.2.2	Content type

The following content types shall be supported:

-	JSON, as defined in IETF RFC 8259 [10], signalled by the content type "application/json".

-	The Problem Details JSON Object (IETF RFC 9457 [11] signalled by the content type "application/problem+json"

-	JSON Patch, as defined in IETF RFC 6902 [12], signalled by the content type "application/json-patch+json"

#### 6.4.2.3	HTTP custom headers

##### 6.4.2.3.1	General

The usage of HTTP custom headers shall be supported as specified in clause 5.2.3 of 3GPP TS 29.500 [4].

### 6.4.3	Resources

#### 6.4.3.1	Overview

Figure 6.4.3.1-1: Resource URI structure of the Nhss_gbaSDM API

Table 6.4.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.4.3.1-1: Resources and methods overview

| Resource name (Archetype) | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| GBA Subscriber Data (Document) | /{ueId}/gba-subscriber-data | GET |  |
| GBA SDM Subscriptions (Collection) | /{ueId}/subscriptions | POST |  |
| GBA SDM Individual | /{ueId}/subscriptions/{subscriptionId} | PATCH |  |
| Subscription (Document) |  | DELETE |  |

#### 6.4.3.2	Resource: GBA Subscriber Data

##### 6.4.3.2.1	Description

This resource represents the subscriber data of the UE for GBA, including the GBA User Security Settings (GUSS).

##### 6.4.3.2.2	Resource Definition

Resource URI: {apiRoot}/nhss-gba-sdm/v1/{ueId}/gba-subscriber-data

This resource shall support the resource URI variables defined in table 6.4.3.2.2-1.

Table 6.4.3.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.4.1 |
| ueId | Represents the UE Identity. See clause 6.4.6.3.2. |

##### 6.4.3.2.3	Resource Standard Methods

6.4.3.2.3.1	GET

This method shall support the URI query parameters specified in table 6.4.3.2.3.1-1.

Table 6.4.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.2.3.1-2 and the response data structures and response codes specified in table 6.4.3.2.3.1-3.

Table 6.4.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.4.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| GbaSubscriberData | M | 1 | 200 OK | A response body containing the GBA subscriber data of the UE; it includes the GBA User Security Settings (GUSS) data. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - OPERATION_NOT_ALLOWED |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - USER_NOT_FOUND |

Table 6.4.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.4.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

#### 6.4.3.3	Resource: Subscriptions

##### 6.4.3.3.1	Description

This resource is used to represent subscriptions to notifications of data change.

##### 6.4.3.3.2	Resource Definition

Resource URI: {apiRoot}/nhss-gba-sdm/{apiVersion}/{ueId}/subscriptions

This resource shall support the resource URI variables defined in table 6.4.3.3.2-1.

Table 6.4.3.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.4.1 |
| apiVersion | See clause 6.4.1 |
| ueId | Represents the UE Identity. See clause 6.4.6.3.2. |

##### 6.4.3.3.3	Resource Standard Methods

6.4.3.3.3.1	POST

This method shall support the URI query parameters specified in table 6.4.3.3.3.1-1.

Table 6.4.3.3.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.1-2 and the response data structures and response codes specified in table 6.4.3.3.3.1-3.

Table 6.4.3.3.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| GbaSdmSubscription | M | 1 | The subscription that is to be created. |

Table 6.4.3.3.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| GbaSdmSubscription | M | 1 | 201 Created | Upon success, a response body containing a representation of the created Individual subscription resource shall be returned. The HTTP response shall include a "Location" HTTP header that contains the resource URI of the created resource. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - USER_NOT_FOUND |
| ProblemDetails | O | 0..1 | 501 Not Implemented | The "cause" attribute may be used to indicate the following application error: - UNSUPPORTED_RESOURCE_URI This response shall not be cached. |

Table 6.4.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.4.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.4.3.3.3.1-6: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nhss-gba-sdm/<apiVersion>/{ueId}/subscriptions/{subscriptionId} |

#### 6.4.3.4	Resource: Individual subscription

##### 6.4.3.4.1	Description

This resource is used to represent an individual subscription to notifications of data change.

##### 6.4.3.4.2	Resource Definition

Resource URI: {apiRoot}/nhss-gba-sdm/{apiVersion}/{ueId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.4.3.4.2-1.

Table 6.4.3.4.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.4.1 |
| apiVersion | See clause 6.4.1 |
| ueId | Represents the UE Identity. See clause 6.4.6.3.2. |
| subscriptionId | The subscriptionId identifies an individual subscription to notifications. |

##### 6.4.3.4.3	Resource Standard Methods

6.4.3.4.3.1	DELETE

This method shall support the URI query parameters specified in table 6.4.3.4.3.1-1.

Table 6.4.3.4.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.4.3.1-2 and the response data structures and response codes specified in table 6.4.3.4.3.1-3.

Table 6.4.3.4.3.1-2: Data structures supported by the Delete Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

Table 6.4.3.4.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - SUBSCRIPTION_NOT_FOUND, see 3GPP TS 29.500 [4] table 5.2.7.2-1. |

Table 6.4.3.4.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.4.3.4.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

6.4.3.4.3.2	PATCH

This method shall support the URI query parameters specified in table 6.4.3.4.3.2-1.

Table 6.4.3.4.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 6.4.3.4.3.2-2 and the response data structures and response codes specified in table 6.4.3.4.3.2-3.

Table 6.4.3.4.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1 | It contains the list of changes to be made to the resource representing the individual subscription, according to the JSON PATCH format specified in IETF RFC 6902 [12]. |

Table 6.4.3.4.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, a response with no content is returned. (NOTE 1) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 1) |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - SUBSCRIPTION_NOT_FOUND, see 3GPP TS 29.500 [4] table 5.2.7.2-1. |
| ProblemDetails | O | 0..1 | 403 Forbidden | One or more attributes are not allowed to be modified. The "cause" attribute may be used to indicate the following application error: - MODIFICATION_NOT_ALLOWED, see 3GPP TS 29.500 [4] table 5.2.7.2-1. |
| NOTE 1: If all the modification instructions in the PATCH request have been implemented, the HSS shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the HSS shall respond with PatchResult. |  |  |  |  |

Table 6.4.3.4.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.4.3.4.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

### 6.4.4	Custom Operations without associated resources

In this release of this specification, no custom operations without associated resources are defined for the Nhss_gbaSDM Service.

### 6.4.5	Notifications

#### 6.4.5.1	General

This clause specifies the use of notifications and corresponding protocol details.

#### 6.4.5.2	Data Change Notification

The POST method shall be used for Data Change Notifications and the URI shall be as provided during the subscription procedure.

Resource URI: {callbackReference}

Support of URI query parameters is specified in table 6.4.5.2-1.

Table 6.4.5.2-1: URI query parameters supported by the POST method

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Support of request data structures is specified in table 6.4.5.2-2 and of response data structures and response codes is specified in table 6.4.5.2-3.

Table 6.4.5.2-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ModificationNotification | M | 1 |  |

Table 6.4.5.2-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the notification endpoint of the subscribing NF Service Consumer. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the notification endpoint of the subscribing NF Service Consumer. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - CONTEXT_NOT_FOUND See table 6.4.7.3-1 for the description of this error. |

Table 6.4.5.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the notification endpoint of the subscribing NF. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.4.5.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the notification endpoint of the subscribing NF. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

### 6.4.6	Data Model

#### 6.4.6.1	General

This clause specifies the application data model supported by the API.

Table 6.4.6.1-1 specifies the structured data types defined for the Nhss_gbaSDM service API.

Table 6.4.6.1-1: Nhss_gbaSDM specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| GbaSubscriberData | 6.4.6.2.2 |  |
| Guss | 6.4.6.2.3 |  |
| BsfInfo | 6.4.6.2.4 |  |
| GbaSdmSubscription | 6.4.6.2.5 |  |
| UeId | 6.4.6.3.2 |  |

Table 6.4.6.1-2 specifies data types re-used by the Nhss_gbaSDM service API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nhss_gbaSDM.

Table 6.4.6.1-2: Nhss_gbaSDM re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| UssListItem | 3GPP TS 29.309 [42] |  |
| SecFeature | 3GPP TS 29.309 [42] |  |
| ModificationNotification | 3GPP TS 29.503 [15] |  |
| NfInstanceId | 3GPP TS 29.571 [16] |  |
| Uri | 3GPP TS 29.571 [16] |  |
| DateTime | 3GPP TS 29.571 [16] |  |
| DurationSec | 3GPP TS 29.571 [16] |  |
| ProblemDetails | 3GPP TS 29.571 [16] | Response body of error response messages. |
| RedirectResponse | 3GPP TS 29.571 [16] | Response body of redirect response messages. |
| ImsUeId | 3GPP TS 29.562 | IMS UE ID (IMPU or IMPI) |

#### 6.4.6.2	Structured data types

##### 6.4.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.4.6.2.2	Type: GbaSubscriberData

Table 6.4.6.2.2-1: Definition of type GbaSubscriberData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| guss | Guss | M | 1 |  |

##### 6.4.6.2.3	Type: Guss

Table 6.4.6.2.3-1: Definition of type Guss

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| bsfInfo | BsfInfo | O | 0..1 | BSF-specific information. |
| ussList | array(UssListItem) | O | 1..N |  |

##### 6.4.6.2.4	Type: BsfInfo

Table 6.4.6.2.4-1: Definition of type BsfInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| uiccType | UiccType | O | 0..1 | It indicates whether GBA or GBA_U is to be used; default value is GBA. |
| lifeTime | DurationSec | O | 0..1 | It indicates a user-specific key lifetime (duration in seconds). If the lifeTime element is missing the default value in the BSF is used. |
| securityFeatures | array(SecFeature) | O | 1..N | It indicates a user-specific security feature list that the UE supports. If the securityFeature element is missing, then the security features are not defined. Standard security features are defined in 3GPP TS 29.309 [42], clause 6.1.6.3.5. |

##### 6.4.6.2.5	Type: GbaSdmSubscription

Table 6.4.6.2.5-1: Definition of type GbaSdmSubscription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| nfInstanceId | NfInstanceId | M | 1 |  |
| callbackReference | Uri | M | 1 |  |
| monitoredResourceUris | array(Uri) | M | 1..N |  |
| expires | DateTime | O | 0..1 |  |

#### 6.4.6.3	Simple data types and enumerations

##### 6.4.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.4.6.3.2	Simple data types

The simple data types defined in table 6.4.6.3.2-1 shall be supported.

Table 6.4.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| UeId | string | Identity of the UE. It can be an MSISDN, an IMSI an IMPI or an IMPU. |
| Msisdn | string | MS-ISDN of the UE. pattern: '^msisdn-[0-9]{5,15}$' |
| Imsi | string | IMSI of the UE. pattern: '^imsi-[0-9]{5,15}$' |

##### 6.4.6.3.3	Enumeration: UiccType

The enumeration UiccType represents whether GBA or GBA_U is to be used. It shall comply with the provisions defined in table 6.4.6.3.3-1.

Table 6.4.6.3.4-1: Enumeration UiccType

| Enumeration value | Description |
| --- | --- |
| "GBA" | GBA: Basic GBA case (no generation of Ks_int_NAF at the BSF). |
| "GBA_U" | GBA_U: Generation of Ks_int_NAF is required at the BSF. |

### 6.4.7	Error Handling

#### 6.4.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [4].

#### 6.4.7.2	Protocol Errors

Protocol errors handling shall be supported as specified in clause 5.2.7 of 3GPP TS 29.500 [4].

#### 6.4.7.3	Application Errors

The common application errors defined in the Table 5.2.7.2-1 in 3GPP TS 29.500 [4] may also be used for the Nhss_gbaSDM service. The following application errors listed in Table 6.4.7.3-1 are specific for the Nhss_gbaSDM service.

Table 6.4.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| USER_NOT_FOUND | 404 Not Found | The user does not exist in the HPLMN |
| CONTEXT_NOT_FOUND | 404 Not Found | The service consumer has received a notification from a service producer for a subscription that is unknown to the service consumer. |

### 6.4.8	Feature Negotiation

The optional features in table 6.4.8-1 are defined for the Nhss_gbaSDM API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.4.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.4.9	Security

As indicated in 3GPP TS 33.501 [5], the access to the Nhss_gbaSDM API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [35]), using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [36]) plays the role of the authorization server.

If Oauth2 authorization is used, an NF Service Consumer, prior to consuming services offered by the Nhss_gbaSDM API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [36], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nhss_gbaSDM service.

The Nhss_gbaSDM API defines the following scopes for OAuth2 authorization:

Table 6.4.9-1: Oauth2 scopes defined in Nhss_gbaSDM API

| Scope | Description |
| --- | --- |
| "nhss-gba-sdm" | Access to the Nhss_gbaSDM API |
| "nhss-gba-sdm:gba-subscriber-data:read" | Access to read the GBA Subscriber Data resource |

### 6.4.10	HTTP redirection

An HTTP request may be redirected to a different HSS service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different HSS producer instance will return the NF Instance ID of the new HSS producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an HSS redirects a service request to a different HSS using an 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new HSS towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

## 6.5	Nhss_gbaUEAuthentication Service API

### 6.5.1	API URI

URIs of this API shall have the following root:

{apiRoot}/{apiName}/{apiVersion}

where "apiRoot" is defined in clause 4.4.1 of 3GPP TS 29.501 [5], the "apiName" shall be set to "nhss-gba-ueau" and the "apiVersion" shall be set to "v1" for the current version of this specification.

### 6.5.2	Usage of HTTP

#### 6.5.2.1	General

HTTP/2, as defined in IETF RFC 9113 [8], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

HTTP messages and bodies for the Nhss_gbaUEAuthentication service shall comply with the OpenAPI [9] specification contained in Annex A.6.

#### 6.5.2.2	HTTP standard headers

##### 6.5.2.2.1	General

The usage of HTTP standard headers shall be supported as specified in clause 5.2.2 of 3GPP TS 29.500 [4].

##### 6.5.2.2.2	Content type

The following content types shall be supported:

-	JSON, as defined in IETF RFC 8259 [10], signalled by the content type "application/json".

-	The Problem Details JSON Object (IETF RFC 9457 [11] signalled by the content type "application/problem+json"

#### 6.5.2.3	HTTP custom headers

##### 6.5.2.3.1	General

The usage of HTTP custom headers shall be supported as specified in clause 5.2.3 of 3GPP TS 29.500 [4].

### 6.5.3	Resources

#### 6.5.3.1	Overview

Figure 6.5.3.1-1: Resource URI structure of the Nhss_gbaUEAU API

Table 6.5.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.5.3.1-1: Resources and methods overview

| Resource name (Archetype) | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| SecurityInformation (Custom operation) | /{ueId}/security-information/generate-auth-data | generate-auth-data (POST) |  |

#### 6.5.3.2	Resource: SecurityInformation (Custom Operation)

##### 6.5.3.2.1	Description

This resource represents the information that is needed to calculate a fresh authentication vector for GBA.

##### 6.5.3.2.2	Resource Definition

Resource URI: {apiRoot}/nhss-gba-ueau/v1/{ueId}/security-information

This resource shall support the resource URI variables defined in table 6.5.3.2.2-1.

Table 6.5.3.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.5.1 |
| ueId | Represents the UE Identity. It can be an MSISDN, an IMSI an IMPI or an IMPU. |

##### 6.5.3.2.3	Resource Standard Methods

No Standard Methods are supported for this resource.

##### 6.5.3.2.3	Resource Custom Operations

6.5.3.2.4.1	Overview

Table 6.5.3.2.4.1-1: Custom operations

| Operation Name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| generate-auth-data | /generate-auth-data | POST |  |

6.5.3.2.4.2	Operation: generate-auth-data

6.5.3.2.4.2.1	Description

This custom operation is used by the NF service consumer (BSF) to request authentication information data for the UE from the HSS.

6.5.3.2.4.2.2	Operation Definition

This operation shall support the request data structures specified in table 6.5.3.2.4.2.2-1 and the response data structure and response codes specified in table 6.5.3.2.4.2.2-2.

Table 6.5.3.2.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AuthenticationInfoRequest | M | 1 |  |

Table 6.5.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AuthenticationInfoResult | M | 1 | 200 OK |  |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - OPERATION_NOT_ALLOWED |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - USER_NOT_FOUND |

Table 6.5.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.5.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

### 6.5.5	Notifications

In this release of this specification, no notifications are defined for the Nhss_gbaUEAuthentication Service.

### 6.5.6	Data Model

#### 6.5.6.1	General

This clause specifies the application data model supported by the API.

Table 6.5.6.1-1 specifies the structured data types defined for the Nhss_gbaUEAU service API.

Table 6.5.6.1-1: Nhss_gbaUEAU specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| AuthenticationInfoRequest | 6.5.6.2.2 |  |
| AuthenticationInfoResult | 6.5.6.2.3 |  |
| DigestAuthentication | 6.5.6.2.6 |  |
| Impi | 6.5.6.3.2 |  |
| AuthenticationScheme | 6.5.6.3.3 |  |
| DigestAlgorithm | 6.5.6.3.4 |  |
| DigestQop | 6.5.6.3.5 |  |

Table 6.5.6.1-2 specifies data types re-used by the Nhss_gbaUEAU service API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nhss_gbaUEAU.

Table 6.5.6.1-2: Nhss_gbaUEAU re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Autn | 3GPP TS 29.503 [15] |  |
| Auts | 3GPP TS 29.503 [15] |  |
| Rand | 3GPP TS 29.503 [15] |  |
| Xres | 3GPP TS 29.503 [15] |  |
| ProblemDetails | 3GPP TS 29.571 [16] | Response body of error response messages. |
| RedirectResponse | 3GPP TS 29.571 [16] | Response body of redirect response messages. |
| SupportedFeatures | 3GPP TS 29.571 [16] |  |
| ResynchronizationInfo | 3GPP TS 29.562 | Clause 6.3.6.2.4 |
| 3GAkaAv | 3GPP TS 29.562 | Clause 6.3.6.2.5 |
| UeId | 3GPP TS 29.562 | Clause 6.4.6.3.2 |

#### 6.5.6.2	Structured data types

##### 6.5.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.5.6.2.2	Type: AuthenticationInfoRequest

Table 6.5.6.2.2-1: Definition of type AuthenticationInfoRequest

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| authenticationScheme | AuthenticationScheme | M | 1 |  |
| resynchronizationInfo | ResynchronizationInfo | O | 0..1 |  |
| supportedFeatures | SupportedFeatures | O | 0..1 |  |

##### 6.5.6.2.3	Type: AuthenticationInfoResult

Table 6.5.6.2.3-1: Definition of type AuthenticationInfoResult

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| impi | Impi | O | 0..1 |  |
| 3gAkaAv | 3gAkaAv | O | 0..1 |  |
| digestAuth | DigestAuthentication | O | 0..1 |  |
| supportedFeatures | SupportedFeatures | O | 0..1 |  |

##### 6.5.6.2.4	Void

##### 6.5.6.2.5	Void

##### 6.5.6.2.6	Type: DigestAuthentication

Table 6.5.6.2.6-1: Definition of type DigestAuthentication

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| digestRealm | string | M | 1 |  |
| digestAlgorithm | DigestAlgorithm | M | 1 |  |
| digestQop | DigestQop | M | 1 |  |
| ha1 | string | M | 1 |  |

#### 6.5.6.3	Simple data types and enumerations

##### 6.5.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.5.6.3.2	Simple data types

The simple data types defined in table 6.5.6.3.2-1 shall be supported.

Table 6.5.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| Impi | string | IMS Private User Identity |
| Ck | string | pattern: "^[A-Fa-f0-9]{32}$" |
| Ik | string | pattern: "^[A-Fa-f0-9]{32}$" |

##### 6.5.6.3.3	Enumeration: AuthenticationScheme

The enumeration AuthenticationScheme represents the authentication scheme used in GBA. It shall comply with the provisions defined in table 6.5.6.3.3-1.

Table 6.5.6.3.3-1: Enumeration AuthenticationScheme

| Enumeration value | Description |
| --- | --- |
| "DIGEST_AKAV1_MD5" | IMS-AKA authentication scheme |
| "DIGEST_HTTP" | Digest authentication scheme |

##### 6.5.6.3.4	Enumeration: DigestAlgorithm

The enumeration DigestAlgorithm represents the algorithm for the Digest scheme. It shall comply with the provisions defined in table 6.5.6.3.4-1.

Table 6.5.6.3.4-1: Enumeration DigestAlgorithm

| Enumeration value | Description |
| --- | --- |
| "MD5" | MD5 digest algorithm |
| "MD5_SESS" | Session variant of the MD5 digest algorithm |

##### 6.5.6.3.5	Enumeration: DigestQop

The enumeration DigestQop represents the quality of protection for the Digest scheme. It shall comply with the provisions defined in table 6.5.6.3.5-1.

Table 6.5.6.3.5-1: Enumeration DigestQop

| Enumeration value | Description |
| --- | --- |
| "AUTH" | Indicates authentication |
| "AUTH_INT" | Indicates authentication with integrity protection |

### 6.5.7	Error Handling

#### 6.5.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [4].

#### 6.5.7.2	Protocol Errors

Protocol errors handling shall be supported as specified in clause 5.2.7 of 3GPP TS 29.500 [4].

#### 6.5.7.3	Application Errors

The common application errors defined in the Table 5.2.7.2-1 in 3GPP TS 29.500 [4] may also be used for the Nhss_gbaUEAU service. The following application errors listed in Table 6.5.7.3-1 are specific for the Nhss_gbaUEAU service.

Table 6.5.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| OPERATION_NOT_ALLOWED | 403 Forbidden | The NF Service Consumer is not authorized to invoke the requested service operation. |
| USER_NOT_FOUND | 404 Not Found | The user does not exist in the HPLMN. |

### 6.5.8	Feature Negotiation

The optional features in table 6.5.8-1 are defined for the Nhss_gbaUEAU API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.5.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.5.9	Security

As indicated in 3GPP TS 33.501 [5], the access to the Nhss_gbaUEAU API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [35]), using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [36]) plays the role of the authorization server.

If Oauth2 authorization is used, an NF Service Consumer, prior to consuming services offered by the Nhss_gbaUEAU API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [36], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nhss_gbaUEAU service.

The Nhss_gbaUEAU API defines the following scopes for OAuth2 authorization:

Table 6.5.9-1: Oauth2 scopes defined in Nhss_gbaUEAU API

| Scope | Description |
| --- | --- |
| "nhss-gba-ueau" | Access to the Nhss_gbaUEAU API |
| "nhss-gba-ueau:generate-auth-data:invoke" | Access to invoke the Generate Auth Data operation |

### 6.5.10	HTTP redirection

An HTTP request may be redirected to a different HSS service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different HSS producer instance will return the NF Instance ID of the new HSS producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an HSS redirects a service request to a different HSS using an 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new HSS towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

## 6.6	Nhss_imsEventExposure Service API

### 6.6.1	API URI

URIs of this API shall have the following root:

{apiRoot}/{apiName}/{apiVersion}

where "apiRoot" is defined in clause 4.4.1 of 3GPP TS 29.501 [5], the "apiName" shall be set to "nhss-ims-ee" and the "apiVersion" shall be set to "v1" for the current version of this specification.

### 6.6.2	Usage of HTTP

#### 6.6.2.1	General

HTTP/2, as defined in IETF RFC 9113 [8], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

HTTP messages and bodies for the Nhss_imsEventExposure service shall comply with the OpenAPI [9] specification contained in Annex A.5.

#### 6.6.2.2	HTTP standard headers

##### 6.6.2.2.1	General

The usage of HTTP standard headers shall be supported as specified in clause 5.2.2 of 3GPP TS 29.500 [4].

##### 6.6.2.2.2	Content type

The following content types shall be supported:

-	JSON, as defined in IETF RFC 8259 [10], signalled by the content type "application/json".

-	The Problem Details JSON Object (IETF RFC 9457 [11] signalled by the content type "application/problem+json"

-	JSON Patch, as defined in IETF RFC 6902 [12], signalled by the content type "application/json-patch+json"

#### 6.6.2.3	HTTP custom headers

##### 6.6.2.3.1	General

The usage of HTTP custom headers shall be supported as specified in clause 5.2.3 of 3GPP TS 29.500 [4].

### 6.6.3	Resources

#### 6.6.3.1	Overview

Figure 6.6.3.1-1: Resource URI structure of the Nhss_imsEventExposure API

Table 6.6.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.6.3.1-1: Resources and methods overview

| Resource name (Archetype) | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| IMS Event Subscriptions (Collection) | /{imsUeId}/ee-subscriptions | POST | Subscribe service operation, creating a new subscription. |
| Individual IMS Event Subscription (Document) | /{imsUeId}/ee-subscriptions/{subscriptionId} | DELETE | Unsubscribe service operation. |
|  |  | PATCH | Subscribe service operation, modification of a subscription. |

#### 6.6.3.2	Resource: IMS Event Subscriptions

##### 6.6.3.2.1	Description

The IMS Event Subscriptions resource represents the subscriptions to a specific subscriber related IMS event. The resource is modelled as Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [5]). The resource allows an NF service consumer to create a new Individual IMS Event Subscription resource.

##### 6.6.3.2.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-ee/<apiVersion>/{imsUeId}/ee-subscriptions

This resource shall support the resource URI variables defined in table 6.6.3.2.2-1.

Table 6.6.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.6.1 |
| imsUeId | string | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity). See 3GPP TS 23.228 [6] clause 4.3. Pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦.+)$" |

##### 6.6.3.2.3	Resource Standard Methods

6.6.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.6.3.2.3.1-1.

Table 6.6.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.6.3.2.3.1-2 and the response data structures and response codes specified in table 6.6.3.2.3.1-3.

Table 6.6.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ImsEeSubscription | M | 1 | Creates a new Individual IMS Event Subscription resource. |

Table 6.6.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CreatedImsEeSubscription | M | 1 | 201 Created | The creation of an Individual IMS Event Subscription resource is confirmed and a representation of that resource is returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - OPERATION_NOT_ALLOWED |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND |
| NOTE 1: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.6.3.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nhss-ims-ee/<apiVersion>/{imsUeId}/ee-subscriptions/{subscriptionId}. |

Table 6.6.3.2.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative HSS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target HSS instance ID towards which the request is redirected. |

Table 6.6.3.2.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative HSS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target HSS instance ID towards which the request is redirected. |

##### 6.6.3.2.4	Resource Custom Operations

None.

#### 6.6.3.3	Resource: Individual IMS Event Subscription

##### 6.6.3.3.1	Description

The Individual IMS Event Subscription resource represents an individual IMS event subscription created for the NF service consumers of Nhss_ImsEE service. This resource is modelled as the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [5]).

##### 6.6.3.3.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-ee/<apiVersion>/{imsUeId}/ee-subscriptions/{subscriptionId}

The <apiVersion> shall be set as described in clause 6.6.1.

This resource shall support the resource URI variables defined in table 6.6.3.3.2-1.

Table 6.6.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.6.1 |
| imsUeId | string | Represents the IMS Public Identity (i.e. IMS Public User identity or Public Service Identity). See 3GPP TS 23.228 [6] clause 4.3). Pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦.+)$" |
| subscriptionId | string | Identifies an individual IMS event subscription to the Nhss_ImsEE service. |

##### 6.6.3.3.3	Resource Standard Methods

6.6.3.3.3.1	DELETE

This method shall support the URI query parameters specified in table 6.6.3.3.3.1-1.

Table 6.6.3.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.6.3.3.3.1-2 and the response data structures and response codes specified in table 6.6.3.3.3.1-3.

Table 6.6.3.3.3.1-2: Data structures supported by the Delete Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

Table 6.6.3.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - SUBSCRIPTION_NOT_FOUND, see 3GPP TS 29.500 [4] table 5.2.7.2-1. |

Table 6.6.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.6.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

6.6.3.3.3.2	PATCH

This method shall support the URI query parameters specified in table 6.6.3.3.3.2-1.

Table 6.6.3.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.6.3.3.3.2-2 and the response data structures and response codes specified in table 6.6.3.3.3.2-3.

Table 6.6.3.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Parameters to be modified to an individual IMS Event Subscription resource. |

Table 6.6.3.3.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CreatedImsEeSubscription | M | 1..N | 200 OK | Represents a successful update on the subscription and the representation of that resource is returned. |
| n/a |  |  | 204 No Content | Upon success, a response with no content is returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - SUBSCRIPTION_NOT_FOUND, see 3GPP TS 29.500 [4] table 5.2.7.2-1. |
| ProblemDetails | O | 0..1 | 403 Forbidden | One or more attributes are not allowed to be modified. The "cause" attribute may be used to indicate the following application error: - MODIFICATION_NOT_ALLOWED, see 3GPP TS 29.500 [4] table 5.2.7.2-1. |
| NOTE 1: The manadatory HTTP error status code for the PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.6.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.6.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same HSS (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

##### 6.6.3.3.4	Resource Custom Operations

None.

### 6.6.4	Custom Operations without associated resources

None in this release of specification.

### 6.6.5	Notifications

#### 6.6.5.1	General

This clause specifies the use of notifications and corresponding protocol details.

#### 6.6.5.2	Monitoring Revocation Notification

The POST method shall be used for Monitoring Revocation Notifications and the URI shall be as provided during the subscription procedure.

Resource URI: {revocationNotifURI}

Support of URI query parameters is specified in table 6.6.5.2-1.

Table 6.6.5.2-1: URI query parameters supported by the POST method

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Support of request data structures is specified in table 6.6.5.2-2 and of response data structures and response codes is specified in table 6.6.5.2-3.

Table 6.6.5.2-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ImsEeMonitoringRevoked | M | 1 |  |

Table 6.6.5.2-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the notification endpoint of the subscribing NF Service Consumer. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the notification endpoint of the subscribing NF Service Consumer. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - CONTEXT_NOT_FOUND See table 6.6.7.3-1 for the description of this error. |

Table 6.6.5.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the notification endpoint of the subscribing NF. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.6.5.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the notification endpoint of the subscribing NF. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

### 6.6.6	Data Model

#### 6.6.6.1	General

This clause specifies the application data model supported by the API.

Table 6.6.6.1-1 specifies the data types defined for the Nhss_ImsEE service based interface protocol.

Table 6.6.6.1-1: Nhss_ImsEE specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| ImsEeSubscription | 6.6.6.2.2 | Represents an Individual IMS Event Subscription. |
| CreatedImsEeSubscription | 6.6.6.2.3 | Represents the created Individual IMS Event Subscription. |
| ImsEeMonitoringRevoked | 6.6.6.2.6 | Represents the IMS Revoked Monitoring events information |
| ImsEeMonitoringEvent | 6.6.6.2.7 | Represents the IMS Monitoring event |

Table 6.6.6.1-2 specifies data types re-used by the Nhss_ImsEE service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nhss_ImsEE service based interface.

Table 6.6.6.1-2: Nhss_ImsEE re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| DateTime | 3GPP TS 29.571 [16] |  |
| ImsEvent | 3GPP TS 29.571 [16] | The IMS events. |
| ImsEventConfiguration | 3GPP TS 29.571 [16] | The IMS event configuration. |
| ImsEventReport | 3GPP TS 29.571 [16] | The IMS event report. |
| ImsReportingOptions | 3GPP TS 29.571 [16] | The IMS reporting options. |
| ImsPublicId | 3GPP TS 29.562 [15] | IMS Public Identity. |
| Uri | 3GPP TS 29.571 [16] |  |

#### 6.6.6.2	Structured data types

##### 6.6.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.6.6.2.2	Type: ImsEeSubscription

Table 6.6.6.2.2-1: Definition of type ImsEeSubscription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| notifId | string | O | 0..1 | Notification Correlation ID provided by the NF service consumer. |
| notificationURI | Uri | M | 1 | Identifies the recipient of Notifications sent by the NF consumer. This parameter shall be supplied by the NF service consumer in the HTTP POST requests that create the subscriptions for event notifications. |
| subscrImsEvents | map(ImsEventConfiguration) | M | 1..N | A map (list of key-value pairs where referenceId converted from integer to string serves as key. Describes the IMS events to be subscribed in a subscription request or the events successfully subscribed for this subscription in a subscription response. |
| afId | string | O | 0..1 | The identifier of the originating AF, may be used by the HSS to perform necessary authorization prior to handling the request. |
| imsReportOptions | ImsReportingOptions | O | 0..1 | This IE may be included if the NF service consumer wants to describe how the reports of the event to be generated, If this attribute is absent, the default report mode shall be "ON_EVENT_DETECTION". |
| revocationNotifURI | Uri | O | 0..1 | Identifies the recipient of Monitoring Revocation Notifications sent by the NF consumer. |

##### 6.6.6.2.3	Type: CreatedImsEeSubscription

Table 6.6.6.2.3-1: Definition of type CreatedImsEeSubscription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| imsEeSubscription | ImsEeSubscription | M | 1 | The Subscribed IMS events information in a subscription response. |
| statusEvents | array(ImsEventReport) | O | 1..N | If immediate report is requested, this IE shall contain the current statuses of the subscribed events. This IE shall be included if the status related to at least one of the subscribed events is available at the time of subscription. |

##### 6.6.6.2.4	Void

##### 6.6.6.2.5	Void

##### 6.6.6.2.6	Type: ImsEeMonitoringRevoked

Table 6.6.6.2.6-1: Definition of type ImsEeMonitoringRevoked

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| revokedMonitoringEventList | map(ImsEeMonitoringEvent) | O | 1..N | Shall contain a map (list of key-value pairs where referenceId converted from integer to string serves as key) of the ImsEeMonitoringEvents (see clause 6.6.6.2.y) that shall be revoked. The Reference ID is provided as the key of the subscribed IMS Events in subscription request. The consumer can use this IE to uniquely associate the revoked monitoring event with the corresponding event that was requested to be monitored. |
| revokedImsUeId | ImsPublicId | O | 0..1 | Represents the IMS Subscription Identifier, either the IMS Public User Identity (IMPU) or Public Service Identity (PSI). See 3GPP TS 23.228 [6] clause 4.3. Pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}-.+)$" |

##### 6.6.6.2.7	Type: ImsEeMonitoringEvent

Table 6.6.6.2.7-1: Definition of type ImsEeMonitoringEvent

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| eventType | ImsEvent | M | 1 | IMS Event; see clause 5.11.3.6 of 3GPP TS 29.571 [16] |
| revokedCause | ImsRevokedCause | O | 0..1 | When present, it contains the reason for revocation of the monitoring event. |

#### 6.6.6.3	Simple data types and enumerations

##### 6.6.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.6.6.3.2	Simple data types

The simple data types defined in table 6.6.6.3.2-1 shall be supported.

Table 6.6.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 6.6.6.3.3	Enumeration: ImsRevokedCause

Table 6.4.6.3.3-1: Enumeration ImsRevokedCause

| Enumeration value | Description |
| --- | --- |
| "MONITORING_NOT_ALLOWED" | IMS Event Exposure Monitoring is not allowed |
| "IMS_UE_ID_REMOVED" | The IMS Subscription Identifier, either the IMS Public Identity (IMPU) or Public Service Identity (PSI) used in the Event Exposure subscription is no longer valid for this individual subscription |

#### 6.6.6.4	Data types describing alternative data types or combinations of data types

None in this release of the specification.

#### 6.6.6.5	Binary data

None in this release of the specification.

### 6.6.7	Error Handling

#### 6.6.7.1	General

For the Nhss_ImsEE API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nhss_ImsEE API.

#### 6.6.7.2	Protocol Errors

No specific procedures for the Nhss_ImsEE service are specified.

#### 6.6.7.3	Application Errors

The common application errors defined in the Table 5.2.7.2-1 in 3GPP TS 29.500 [4] may also be used for the Nhss_imsEE service. The following application errors listed in Table 6.6.7.3-1 are specific for the Nhss_imsEE service.

Table 6.6.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| USER_NOT_FOUND | 404 Not Found | The received IMS UE identity is unknown. |
| CONTEXT_NOT_FOUND | 404 Not Found | It is used during the modification of an existing subscription when no corresponding context exists. |
| OPERATION_NOT_ALLOWED | 403 Forbidden | Indicates the IMS Event subscription is not allowed to be created. |

### 6.6.8	Feature negotiation

The optional features in table 6.6.8-1 are defined for the Nhss_ImsEE API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.6.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.6.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nhss_ImsEE API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nhss_ImsEE API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nhss_EE service.

The Nhss_ImsEE API defines a single scope "nhss-ims-ee" for the entire service, and it does not define any additional scopes at resource or operation level.

### 6.6.10	HTTP redirection

An HTTP request may be redirected to a different HSS service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different NF provider (e.g. HSS) instance will return the NF Instance ID of the new NF provider instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an NF provider (e.g. HSS) redirects a service request to a different NF provider using a 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new NF provider towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

Annex A (normative):
OpenAPI specification

# A.1	General

This Annex specifies the formal definition of the Nhss IMS API(s). It consists of OpenAPI 3.0.0 specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository, that uses the GitLab software version control system (see 3GPP TS 29.501 [5] clause 5.3.1 and 3GPP TR 21.900 [34] clause 5B).

# A.2	Nhss_imsUECM API

openapi: 3.0.0

info:

version: '1.3.0'

title: 'Nhss_imsUECM'

description: |

Nhss UE Context Management Service for IMS.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.562 Home Subscriber Server (HSS) Services, version 19.4.0

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.562/'

servers:

- url: '{apiRoot}/nhss-ims-uecm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

paths:

/{impu}/authorize:

post:

summary: >

Authorize IMS Identities to register in the network or establish multimedia sessions

and return CSCF location if it is stored

operationId: Authorize

tags:

- Authorize (Custom Operation)

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:authorize:invoke

parameters:

- name: impu

in: path

description: Public identity of the user.

required: true

schema:

$ref: '#/components/schemas/Impu'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AuthorizationRequest'

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/AuthorizationResponse'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/scscf-registration:

put:

summary: SCSCF registration information

operationId: SCSCF registration

tags:

- S-CSCF Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:registration:create

parameters:

- name: imsUeId

in: path

description: IMS Public Identity or IMS Private Identity

required: true

schema:

type: string

pattern: '(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|impu-tel\:\+[0-9]{5,15}|impi-.+|.+)$'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRegistration'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRegistration'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/nhss-ims-uecm/v1/{imsUeId}/scscf-registration

required: true

schema:

type: string

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRegistration'

'204':

description: No content

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

description: Forbidden

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtendedProblemDetails'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

deregistrationNotification:

'{$request.body#/deregCallbackUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DeregistrationData'

responses:

'204':

description: Successful Notification response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/scscf-registration/scscf-restoration-info:

put:

summary: Update the S-CSCF restoration information of the UE

operationId: UpdateScscfRestorationInfo

tags:

- S-CSCF Restoration Information (Document)

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:restoration:modify

parameters:

- name: imsUeId

in: path

description: IMS Public Identity.

required: true

schema:

type: string

pattern: '^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|impu-tel\:\+[0-9]{5,15}|.+)$'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRestorationInfoRequest'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRestorationInfoResponse'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/nhss-ims-uecm/v1/{imsUeId}/scscf-registration/scscf-restoration-info

required: true

schema:

type: string

'200':

description: OK

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRestorationInfoResponse'

'204':

description: No content

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieve the S-CSCF restoration information of the UE

operationId: GetScscfRestorationInfo

tags:

- S-CSCF Restoration Information (Document)

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:restoration:read

parameters:

- name: impu

in: path

description: Public identity of the user.

required: true

schema:

$ref: '#/components/schemas/Impu'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRestorationInfoResponse'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete the S-CSCF restoration information of the UE

operationId: DeleteScscfRestorationInfo

tags:

- S-CSCF Restoration Information (Document)

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:restoration:modify

parameters:

- name: impu

in: path

description: Public identity of the user.

required: true

schema:

$ref: '#/components/schemas/Impu'

responses:

'204':

description: No content

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/imsas-registrations:

get:

summary: Retrieve the IMS AS information of the UE

operationId: AsInfoGet

tags:

- ImsAs Registrations (Store)

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:imsasregistration:read

parameters:

- name: imsUeId

in: path

description: IMS Public Identity of the user.

required: true

schema:

type: string

pattern: '^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|impu-tel\:\+[0-9]{5,15}|.+)$'

- name: asType

in: query

description: IMS AS type

schema:

$ref: '#/components/schemas/AsType'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ImsAsRegistrationInfo'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/imsas-registrations/{instanceId}:

put:

summary: IMS AS registration information

operationId: CreateImsasRegistration

tags:

- Individual Imsas Registration Information (Document)

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:imsasregistration:create

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

type: string

pattern: '^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|impu-tel\:\+[0-9]{5,15}|.+)$'

- name: instanceId

in: path

description: indicates the instance Id of the IMS AS.

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ImsasRegistration'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/ImsasRegistration'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/nhss-ims-uecm/v1/{imsUeId}/imsas-registration

required: true

schema:

type: string

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ImsasRegistration'

'204':

description: No content

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

ImsasDeregistrationNotification:

'{$request.body#/deregCallbackUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AsDeregistrationData'

responses:

'204':

description: Successful Notification response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

ImsasReassignmentNotification:

'{$request.body#/asReassignmentCallbackUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AsReassignmentData'

responses:

'204':

description: Successful Notification response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: delete the IMS AS registration information

operationId: DeleteImsasRegistration

tags:

- Individual Imsas Registration Information (Document)

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:imsasregistration:modify

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

type: string

pattern: '^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|impu-tel\:\+[0-9]{5,15}|.+)$'

- name: instanceId

in: path

description: indicates the instance Id of the IMS AS.

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

responses:

'204':

description: No content

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieve the IMS AS registration information of the UE

operationId: AsInfoGet

tags:

- Imsas Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:imsasregistration:read

parameters:

- name: impu

in: path

description: Public identity of the user.

required: true

schema:

$ref: '#/components/schemas/Impu'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ImsasRegistration'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nhss-ims-uecm: Access to the Nhss IMS UE Context Management API

nhss-ims-uecm:authorize:invoke: Access to invoke the Authorize custom operation

nhss-ims-uecm:registration:create: Access to create the S-CSCF Registration resource

nhss-ims-uecm:restoration:read: Access to read the S-CSCF Restoration resource

nhss-ims-uecm:restoration:modify: >-

Access to create/update/delete the S-CSCF Restoration resource

nhss-ims-uecm:imsasregistration:create: >-

Access to create the Individual Imsas Registration Information resource

nhss-ims-uecm:imsasregistration:modify: >-

Access to delete the Individual Imsas Registration Information resource

nhss-ims-uecm:imsasregistration:read: Access to read IMS AS Registration resources

schemas:

#

# COMPLEX TYPES:

#

AuthorizationRequest:

description: Ims authorization request data

type: object

required:

- authorizationType

properties:

impi:

$ref: '#/components/schemas/Impi'

authorizationType:

$ref: '#/components/schemas/AuthorizationType'

visitedNetworkIdentifier:

type: string

emergencyIndicator:

type: boolean

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

AuthorizationResponse:

description: Ims Registration authorization information result

type: object

properties:

authorizationResult:

$ref: '#/components/schemas/AuthorizationResult'

cscfServerName:

type: string

scscfSelectionAssistanceInfo:

$ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ScscfSelectionAssistanceInformation'

required:

- authorizationResult

oneOf:

- required:

- cscfServerName

- required:

- scscfSelectionAssistanceInfo

ScscfRegistration:

description: Scscf Registration

type: object

required:

- imsRegistrationType

- cscfServerName

properties:

impi:

$ref: '#/components/schemas/Impi'

imsRegistrationType:

$ref: '#/components/schemas/ImsRegistrationType'

cscfServerName:

type: string

scscfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

deregCallbackUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

associatedImpis:

type: array

items:

$ref: '#/components/schemas/Impi'

associatedRegisteredImpis:

type: array

items:

$ref: '#/components/schemas/Impi'

irsImpus:

type: array

items:

$ref: '#/components/schemas/Impu'

minItems: 1

uniqueItems: true

wildcardedPui:

$ref: '#/components/schemas/Impu'

looseRouteIndicator:

$ref: '#/components/schemas/LooseRouteIndication'

wildcardedPsi:

$ref: '#/components/schemas/Impu'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

multipleRegistrationIndicator:

type: boolean

pcscfRestorationIndicator:

type: boolean

default: false

scscfReselectionIndicator:

type: boolean

default: false

ExtendedProblemDetails:

description: >

Extension of the ProblemDetails data type, to include additional information

in an error response message

allOf:

- $ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

- $ref: '#/components/schemas/AdditionalInfo'

AdditionalInfo:

description: >

Additional information to be sent on error response messages, along with the

common content in ProblemDetails

type: object

properties:

scscfServerName:

type: string

ScscfRestorationInfo:

description: S-CSCF restoration information

type: object

properties:

userName:

$ref: '#/components/schemas/Impi'

restorationInfo:

type: array

items:

$ref: '#/components/schemas/RestorationInfo'

registrationTimeOut:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

sipAuthenticationScheme:

$ref: 'TS29562_Nhss_imsUEAU.yaml#/components/schemas/SipAuthenticationScheme'

ScscfRestorationInfoRequest:

description: S-CSCF restoration information request

type: object

properties:

scscfRestorationInfoRequest:

$ref: '#/components/schemas/ScscfRestorationInfo'

ScscfRestorationInfoResponse:

description: S-CSCF restoration information response

type: object

properties:

scscfRestorationInfoResponse:

type: array

items:

$ref: '#/components/schemas/ScscfRestorationInfo'

RestorationInfo:

description: >

The information relevant to a specific registration required for an S-CSCF

to handle the requests for a user

type: object

required:

- path

- contact

properties:

path:

type: string

contact:

type: string

initialCSeqSequenceNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

callIdSipHeader:

type: string

uesubscriptionInfo:

$ref: '#/components/schemas/UeSubscriptionInfo'

pcscfSubscriptionInfo:

$ref: '#/components/schemas/PcscfSubscriptionInfo'

imsSdmSubscriptions:

description: >

A map (list of key-value pairs where subscriptionId serves as key) of ImsSdmSubscription

type: object

additionalProperties:

$ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsSdmSubscription'

UeSubscriptionInfo:

description: Subscription information of the UE for the SIP Registration State event

type: object

required:

- callIdSipHeader

- fromSipHeader

- toSipHeader

- recordRoute

- contact

properties:

callIdSipHeader:

type: string

fromSipHeader:

type: string

toSipHeader:

type: string

recordRoute:

type: string

contact:

type: string

PcscfSubscriptionInfo:

description: Subscription information of the P-CSCF for the SIP Registration State event

type: object

required:

- callIdSipHeader

- fromSipHeader

- toSipHeader

- contact

properties:

callIdSipHeader:

type: string

fromSipHeader:

type: string

toSipHeader:

type: string

contact:

type: string

DeregistrationData:

description: Data related to the de-registration information of a S-CSCF in HSS

type: object

required:

- deregReason

- impi

properties:

deregReason:

$ref: '#/components/schemas/DeregistrationReason'

impi:

$ref: '#/components/schemas/Impi'

associatedImpis:

type: array

items:

$ref: '#/components/schemas/Impi'

emergencyRegisteredIdentities:

type: array

items:

$ref: '#/components/schemas/EmergencyRegisteredIdentity'

minItems: 1

EmergencyRegisteredIdentity:

description: >

A pair of private and public user identities which have not been de-registered

due to emergency registration

type: object

required:

- impi

- impu

properties:

impi:

$ref: '#/components/schemas/Impi'

impu:

$ref: '#/components/schemas/Impu'

DeregistrationReason:

description: >

Contains the reason for the network initiated de-registration (including a reason code,

and a human-readable reason text)

type: object

required:

- reasonCode

- reasonText

properties:

reasonCode:

$ref: '#/components/schemas/DeregistrationReasonCode'

reasonText:

type: string

ImsasRegistration:

description: Imsas Registration

type: object

required:

- imsasInstanceId

properties:

imsasInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

asType:

$ref: '#/components/schemas/AsType'

deregCallbackUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

asReassignementInd:

type: boolean

enum:

- true

asReassignmentCallbackUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ImsAsRegistrationInfo:

description: IMS AS Registration information list

type: object

required:

- imsAsRegistrationList

properties:

imsAsRegistrationList:

type: array

items:

$ref: '#/components/schemas/ImsasRegistration'

minItems: 1

AsDeregistrationData:

description: Data related to the de-registration information of a IMS AS in HSS

type: object

required:

- asDeregReason

properties:

asDeregReason:

$ref: '#/components/schemas/AsDeregistrationReason'

impu:

$ref: '#/components/schemas/Impu'

AsReassignmentData:

description: Data related to the IMS AS reassignment information

type: object

required:

- imsasInstanceId

properties:

imsasInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

impu:

$ref: '#/components/schemas/Impu'

#

# SIMPLE TYPES

#

ImsUeId:

description: IMS UE Identity (IMPU or IMPI)

type: string

pattern: '^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|impu-tel\:\+[0-9]{5,15}|impi-.+|.+)$'

Impu:

description: IMS Public Identity of the UE (sip URI or tel URI)

type: string

pattern: '^(sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|tel\:\+[0-9]{5,15})$'

Impi:

description: IMS Private Identity of the UE

type: string

#

# ENUMS:

#

AuthorizationType:

description: Represents the type of authorization requested by the UE

anyOf:

- type: string

enum:

- REGISTRATION

- DEREGISTRATION

- type: string

AuthorizationResult:

description: Represents the details of the granted authorization to the UE

anyOf:

- type: string

enum:

- FIRST_REGISTRATION

- SUBSEQUENT_REGISTRATION

- type: string

ImsRegistrationType:

description: Represents the type of registration associated to the REGISTER request

anyOf:

- type: string

enum:

- INITIAL_REGISTRATION

- RE_REGISTRATION

- TIMEOUT_DEREGISTRATION

- USER_DEREGISTRATION

- ADMINISTRATIVE_DEREGISTRATION

- AUTHENTICATION_FAILURE

- AUTHENTICATION_TIMEOUT

- UNREGISTERED_USER

- type: string

LooseRouteIndication:

description: Indicates whether the loose routing mechanism is required to serve the user

anyOf:

- type: string

enum:

- LOOSE_ROUTE_NOT_REQUIRED

- LOOSE_ROUTE_REQUIRED

- type: string

DeregistrationReasonCode:

description: Indicates the reason for the network-initiated deregistration

anyOf:

- type: string

enum:

- PERMANENT_TERMINATION

- NEW_SERVER_ASSIGNED

- REMOVE_S-CSCF

- SERVER_CHANGE

- type: string

AsType:

description: Represents the type of the IMS AS

anyOf:

- type: string

enum:

- MMTEL_AS

- type: string

AsDeregistrationReason:

description: >

It represents the reason of Deregistration Notification.

anyOf:

- type: string

enum:

- ADMINISTRATIVE_TERMINATION

- NEW_SERVER_ASSIGNED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

# A.3	Nhss_imsSDM API

openapi: 3.0.0

info:

version: '1.3.0'

title: 'Nhss_imsSDM'

description: |

Nhss Subscriber Data Management Service for IMS.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.562 HSS Services for interworking with IMS, version 19.4.0

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.562/'

servers:

- url: '{apiRoot}/nhss-ims-sdm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

paths:

/{imsUeId}/ims-data/registration-status:

get:

summary: Retrieve the registration status of a user

operationId: GetRegistrationStatus

tags:

- Registration Status retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:registration-status:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ImsRegistrationStatus'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/ims-data/profile-data:

get:

summary: >

Retrieve the complete IMS profile for a given IMS public identity

(and public identities in the same IRS)

operationId: GetProfileData

tags:

- IMS Profile Data Retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:profile-data:read

parameters:

- name: imsUeId

in: path

description: IMS Identity. In this case it shall be an IMS public identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: dataset-names

in: query

description: Datasets to be retrieved

style: form

explode: false

schema:

$ref: '#/components/schemas/DataSetNames'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ImsProfileData'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/ims-data/profile-data/priority-levels:

get:

summary: Retrieve the service priority levels associated to the user

operationId: GetPriorityInfo

tags:

- Priority Info Retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:priority-levels:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/PriorityLevels'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/ims-data/profile-data/ifcs:

get:

summary: Retrieve the Initial Filter Criteria for the associated IMS subscription

operationId: GetIfcs

tags:

- IFCs Retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:ifcs:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: application-server-name

in: query

description: SIP URI of the Application Server Name

schema:

$ref: '#/components/schemas/SipServerName'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/Ifcs'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/ims-data/profile-data/service-level-trace-information:

get:

summary: Retrieve the IMS service level trace information for the associated user

operationId: GetServiceTraceInfo

tags:

- Service Trace Info Retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:service-level-trace-information:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceLevelTraceInformation'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/ims-data/profile-data/charging-info:

get:

summary: Retrieve the charging information for to the user

operationId: GetChargingInfo

tags:

- Charging Info Retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:charging-info:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingInfo'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/ims-data/location-data/server-name:

get:

summary: Retrieve the server name for the associated user

operationId: GetServerName

tags:

- Server Name retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:server-name:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ImsLocationData'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/ims-data/location-data/scscf-capabilities:

get:

summary: Retrieve the S-CSCF capabilities for the associated IMS subscription

operationId: GetScscfCapabilities

tags:

- Retrieval of the S-CSCF capabilities for the IMS subscription

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:scscf-capabilities:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfCapabilityList'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/ims-data/location-data/scscf-selection-assistance-info:

get:

summary: Retrieve the S-CSCF selection assistance info

operationId: GetScscfSelectionAssistanceInfo

tags:

- Retrieval of the S-CSCF selection assistance information for the IMS subscription

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:scscf-selection-assistance-info:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfSelectionAssistanceInformation'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/access-data/ps-domain/location-data:

get:

summary: Retrieve the location data in PS domain

operationId: GetLocPsDomain

tags:

- PS location retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:ps-domain:location-data:read

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: requested-nodes

in: query

description: Indicates the serving node(s) for which the request is applicable.

style: form

explode: false

schema:

type: array

items:

$ref: '#/components/schemas/RequestedNode'

- name: serving-node

in: query

description: >

Indicates that only the stored NF id/address of the serving node(s) is required

schema:

type: boolean

- name: local-time

in: query

description: >

Indicates that only the Local Time Zone information of the location in the visited

network where the UE is attached is requested

schema:

type: boolean

- name: current-location

in: query

description: >

Indicates whether an active location retrieval has to be initiated by the requested node

schema:

type: boolean

- name: rat-type

in: query

description: Indicates whether RAT Type retrieval is requested

schema:

type: boolean

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: private-identity

in: query

description: IMS Private Identity

schema:

$ref: '#/components/schemas/PrivateId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/PsLocation'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/access-data/cs-domain/location-data:

get:

summary: Retrieve the location data in CS domain

operationId: GetLocCsDomain

tags:

- CS location retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:cs-domain:location-data:read

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: serving-node

in: query

description: >

Indicates that only the stored NF id/address of the serving node(s) is required

schema:

type: boolean

- name: local-time

in: query

description: >

Indicates that only the Local Time Zone information of the location in the visited

network where the UE is attached is requested

schema:

type: boolean

- name: current-location

in: query

description: >

Indicates whether an active location retrieval has to be initiated by the requested node

schema:

type: boolean

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: private-identity

in: query

description: IMS Private Identity

schema:

$ref: '#/components/schemas/PrivateId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/CsLocation'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/repository-data/{serviceIndication}:

put:

summary: Update the repository data associated to an IMPU and service indication

operationId: UpdateRepositoryDataServInd

tags:

- Update repository data

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:repository-data:modify

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: serviceIndication

in: path

description: Identifier of a service related data

required: true

schema:

$ref: '#/components/schemas/ServiceIndication'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/RepositoryData'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/RepositoryData'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/nhss-ims-sdm/v1/{imsUeId}/repository-data/{serviceIndication}

required: true

schema:

type: string

'200':

description: OK

content:

application/json:

schema:

$ref: '#/components/schemas/RepositoryData'

'204':

description: No content

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'409':

$ref: 'TS29571_CommonData.yaml#/components/responses/409'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

delete:

summary: delete the Repository Data for a Service Indication

operationId: DeleteRepositoryDataServInd

tags:

- Delete repository data

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:repository-data:modify

parameters:

- name: imsUeId

in: path

description: Identifier of the UE

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: serviceIndication

in: path

description: Identifier of a service related data

required: true

schema:

$ref: '#/components/schemas/ServiceIndication'

responses:

'204':

description: Expected response to a valid request

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

get:

summary: Retrieve the repository data associated to an IMPU and service indication

operationId: GetRepositoryDataServInd

tags:

- Repository data

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:repository-data:read

parameters:

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: serviceIndication

in: path

description: Identifier of a service related data

required: true

schema:

$ref: '#/components/schemas/ServiceIndication'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/RepositoryData'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/identities/msisdns:

get:

summary: retrieve the Msisdns associated to requested identity

operationId: GetMsisdns

tags:

- Retrieval of the associated Msisdns

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:identities:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: private-id

in: query

description: Private identity

schema:

$ref: '#/components/schemas/PrivateId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/MsisdnList'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/identities/ims-associated-identities:

get:

summary: >

Retrieve the associated identities to the IMS public identity included

in the service request

operationId: GetImsAssocIds

tags:

- Retrieval of associated IMS public identities

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:identities:read

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ImsAssociatedIdentities'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/identities/private-identities:

get:

summary: >

Retrieve the associated private identities to the IMS public identity included

in the service request

operationId: GetImsPrivateIds

tags:

- Retrieval of associated IMS private identities

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:identities:read

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: impi

in: query

description: Private Identity of type IMPI

schema:

$ref: '#/components/schemas/PrivateId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/PrivateIdentities'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/srvcc-data:

get:

summary: Retrieve the srvcc data

operationId: GetSrvccData

tags:

- Retrieval of UE SRVCC capability and STN-SR

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:srvcc:read

parameters:

- name: imsUeId

in: path

description: IMS Public Identity or IMS Private Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: private-identity

in: query

description: IMS Private Identity

schema:

$ref: '#/components/schemas/PrivateId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SrvccData'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Patch

operationId: UpdateSrvccData

tags:

- Update SRVCC data

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:srvcc:modify

parameters:

- name: imsUeId

in: path

description: IMS Public Identity or IMS Private Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: private-identity

in: query

description: IMS Private Identity

schema:

$ref: '#/components/schemas/PrivateId'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'204':

description: Successful response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{imsUeId}/service-data/psi-status:

get:

summary: Retrieve the PSI activation state data

operationId: GetPsiState

tags:

- Retrieval of PSI activation state

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:psi-status:read

parameters:

- name: imsUeId

in: path

description: IMS Private Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/PsiActivationState'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Patch

operationId: UpdatePsiState

tags:

- Update PSI state data

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:psi-status:modify

parameters:

- name: imsUeId

in: path

description: IMS Public Service Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'204':

description: Successful response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{imsUeId}/service-data/dsai:

get:

summary: Retrieve the DSAI information associated to an Application Server

operationId: GetDsaiInfo

tags:

- DSAI registration information

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:dsai:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: application-server-name

in: query

description: SIP URI of the Application Server Name

required: true

schema:

$ref: '#/components/schemas/SipServerName'

- name: dsai-tag

in: query

description: The requested instance of Dynamic Service Activation Info

schema:

type: string

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/DsaiTagInformation'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Patch

operationId: UpdateDsaiState

tags:

- Update Dsai State

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:dsai:modify

parameters:

- name: imsUeId

in: path

description: IMS Public Service Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'204':

description: Successful response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{imsUeId}/service-data/sms-registration-info:

put:

summary: Update the SMS registration information associated to a user

operationId: UpdateSmsRegistrationInfo

tags:

- Update SMS registration info

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:sms-registration-info:modify

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: private-identity

in: query

description: IMS Private Identity

schema:

$ref: '#/components/schemas/PrivateId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/IpSmGwAddress'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/SmsRegistrationInfo'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/nhss-ims-sdm/v1/{imsUeId}/service-data/sms-registration-info

required: true

schema:

type: string

'200':

description: OK

content:

application/json:

schema:

$ref: '#/components/schemas/SmsRegistrationInfo'

'204':

description: No content

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'409':

$ref: 'TS29571_CommonData.yaml#/components/responses/409'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

delete:

summary: delete the SMS registration information

operationId: DeleteSmsRegistrationInfo

tags:

- Delete SMS registration information

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:sms-registration-info:modify

parameters:

- name: imsUeId

in: path

description: Identifier of the UE

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: private-identity

in: query

description: IMS Private Identity

schema:

$ref: '#/components/schemas/PrivateId'

responses:

'204':

description: Expected response to a valid request

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

get:

summary: Retrieve the SMS registration information associated to a user

operationId: GetSmsRegistrationInfo

tags:

- SMS registration information

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:sms-registration-info:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: private-identity

in: query

description: IMS Private Identity

schema:

$ref: '#/components/schemas/PrivateId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SmsRegistrationInfo'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/identities/imeisv:

get:

summary: Retrieve the IMEISV information

operationId: GetIMEISVInfo

tags:

- IMEISV Retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:identities:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: private-identity

in: query

description: IMS Private Identity

schema:

$ref: '#/components/schemas/PrivateId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ImeiSvInformation'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/access-data/ps-domain/ip-address:

get:

summary: Retrieve the IP address information

operationId: GetIpAddressInfo

tags:

- IP address Info Retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:ps-domain:ip-address:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/access-data/ps-domain/tads-info:

get:

summary: Retrieve the T-ADS information

operationId: GetTadsInfo

tags:

- TADS Info Retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:ps-domain:tads-info:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: private-identity

in: query

description: IMS Private Identity

schema:

$ref: '#/components/schemas/PrivateId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/TadsInformation'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/access-data/ps-domain/ue-reach-subscriptions:

post:

summary: subscribe to notifications of UE reachability

operationId: UeReachIpSubscribe

tags:

- UE IP reachability Subscription Creation

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:ps-domain:ue-reach-subscriptions:create

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: private-identity

in: query

description: IMS Private Identity

schema:

$ref: '#/components/schemas/PrivateId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/UeReachabilitySubscription'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/CreatedUeReachabilitySubscription'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/nhss-ims-sdm/v1/{imsUeId}/access-data/ps-domain/ue-reach-subscriptions/{subscriptionId}

required: true

schema:

type: string

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

callbacks:

ueReachabilityNotification:

'{$request.body#/callbackReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UeReachabilityNotification'

responses:

'204':

description: Successful Notification response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{imsUeId}/access-data/ps-domain/ue-reach-subscriptions/{subscriptionId}:

delete:

summary: unsubscribe from notifications to UE reachability

operationId: UeReachUnsubscribe

tags:

- Reachability Subscription Deletion

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:ps-domain:ue-reach-subscriptions:modify

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: subscriptionId

in: path

description: Id of the Subscription

required: true

schema:

type: string

- name: private-identity

in: query

description: IMS Private Identity

schema:

$ref: '#/components/schemas/PrivateId'

responses:

'204':

description: Successful response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

patch:

summary: modify the subscription

operationId: UeReachSubsModify

tags:

- Reachability Subscription Modification

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:ps-domain:ue-reach-subscriptions:modify

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: subscriptionId

in: path

description: Id of the Subscription

required: true

schema:

type: string

- name: private-identity

in: query

description: IMS Private Identity

schema:

$ref: '#/components/schemas/PrivateId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'204':

description: Successful modification

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{imsUeId}/access-data/ps-domain/user-state:

get:

summary: Retrieve the user state information in PS domain

operationId: GetPsUserStateInfo

tags:

- PS User State Info Retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:ps-domain:user-state:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: requested-nodes

in: query

description: Indicates the serving node(s) for which the request is applicable.

style: form

explode: false

schema:

type: array

items:

$ref: '#/components/schemas/RequestedNode'

minItems: 1

- name: private-identity

in: query

description: IMS Private Identity

schema:

$ref: '#/components/schemas/PrivateId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/PsUserState'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/access-data/cs-domain/user-state:

get:

summary: Retrieve the user state information in CS domain

operationId: GetCsUserStateInfo

tags:

- CS User State Info Retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:cs-domain:user-state:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: private-identity

in: query

description: IMS Private Identity

schema:

$ref: '#/components/schemas/PrivateId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/CsUserState'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/access-data/cs-domain/csrn:

get:

summary: Retrieve the routeing number in CS domain

operationId: GetCsrn

tags:

- CSRN retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:cs-domain:csrn:read

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: pre-paging

in: query

description: Indicates pre-paging support

schema:

$ref: '#/components/schemas/PrePaging'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: private-identity

in: query

description: IMS Private Identity

schema:

$ref: '#/components/schemas/PrivateId'

- name: call-reference-info

in: query

description: Indicates Call-Reference Number and AS-Number

content:

application/json:

schema:

$ref: '#/components/schemas/CallReferenceInfo'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/Csrn'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/access-data/wireline-domain/reference-location:

get:

summary: Retrieve the reference location information

operationId: GetReferenceLocationInfo

tags:

- Reference Location Info Retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:wireline-domain:reference-location:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: private-identity

in: query

description: IMS Private Identity

schema:

$ref: '#/components/schemas/PrivateId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ReferenceLocationInformation'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/subscriptions:

post:

summary: subscribe to notifications

operationId: ImsSdmSubscribe

tags:

- SDM Subscription Creation

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:subscriptions:create

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSdmSubscription'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSdmSubscription'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/nhss-ims-sdm/v1/{imsUeId}/subscriptions/{subscriptionId}

required: true

schema:

type: string

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

datachangeNotification:

'{$request.body#/callbackReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ModificationNotification'

responses:

'204':

description: Successful Notification response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/subscriptions/{subscriptionId}:

delete:

summary: unsubscribe from notifications

operationId: ImsSdmUnsubscribe

tags:

- SDM Subscription Deletion

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:subscription:modify

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: subscriptionId

in: path

description: Id of the Subscription

required: true

schema:

type: string

responses:

'204':

description: Successful response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: modify the subscription

operationId: ImsSdmSubsModify

tags:

- SDM Subscription Modification

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:subscription:modify

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: subscriptionId

in: path

description: Id of the Subscription

required: true

schema:

type: string

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'204':

description: Successful modification

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/shared-data-subscriptions:

post:

summary: subscribe to notifications for shared data

operationId: SubscribeToSharedData

tags:

- Subscription Creation for shared data

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:shared-subscriptions:create

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSdmSubscription'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSdmSubscription'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/nhss-ims-sdm/v1/shared-data-subscriptions/{subscriptionId}

required: true

schema:

type: string

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

datachangeNotification:

'{$request.body#/callbackReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ModificationNotification'

responses:

'204':

description: Successful Notification response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/shared-data-subscriptions/{subscriptionId}:

delete:

summary: unsubscribe from notifications for shared data

operationId: UnsubscribeForSharedData

tags:

- Subscription Deletion for shared data

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:shared-subscription:modify

parameters:

- name: subscriptionId

in: path

description: Id of the Shared data Subscription

required: true

schema:

type: string

responses:

'204':

description: Successful response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: modify the subscription

operationId: ModifySharedDataSubs

tags:

- Subscription Modification

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:shared-subscription:modify

parameters:

- name: subscriptionId

in: path

description: Id of the SDM Subscription

required: true

schema:

type: string

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'204':

description: Successful modification

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/shared-data:

get:

summary: retrieve shared data

operationId: GetSharedData

tags:

- Retrieval of shared data

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:shared-data:read

parameters:

- name: shared-data-ids

in: query

description: List of shared data ids

required: true

style: form

explode: false

schema:

type: array

items:

$ref: '#/components/schemas/SharedDataId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/SharedData'

minItems: 1

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/repository-data:

get:

summary: Retrieve the repository data associated to an IMPU and service indication list

operationId: GetRepositoryDataServIndList

tags:

- Repository data list

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:repository-data:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: service-indications

in: query

description: Identifiers of a services related data

required: true

style: form

explode: false

schema:

type: array

items:

$ref: '#/components/schemas/ServiceIndication'

minItems: 1

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/RepositoryDataList'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nhss-ims-sdm: Access to the Nhss IMS Subscription Data Management API

nhss-ims-sdm:registration-status:read: Access to read the Registration Status resource

nhss-ims-sdm:profile-data:read: Access to read the Profile Data resource

nhss-ims-sdm:priority-levels:read: Access to read the Priority Levels resource

nhss-ims-sdm:ifcs:read: Access to read the Initial Filter Criteria resource

nhss-ims-sdm:service-level-trace-information:read: >-

Access to read the Service Level Trace Information resource

nhss-ims-sdm:server-name:read: Access to read the Server Name resource

nhss-ims-sdm:scscf-capabilities:read: Access to read the S-CSCF Capabilities resource

nhss-ims-sdm:ps-domain:location-data:read: >-

Access to read the PS-Domain Location Data resource

nhss-ims-sdm:ps-domain:ip-address:read: Access to read the PS-Domain IP Address resource

nhss-ims-sdm:ps-domain:tads-info:read: Access to read the PS-Domain TADS Info resource

nhss-ims-sdm:ps-domain:ue-reach-subscriptions:create: >-

Access to create PS-Domain UE Reachability Subscriptions resources

nhss-ims-sdm:ps-domain:ue-reach-subscriptions:modify: >-

Access to update/delete a PS-Domain UE Reachability Subscription resource

nhss-ims-sdm:ps-domain:user-state:read: Access to read the PS-Domain User State resource

nhss-ims-sdm:cs-domain:location-data:read: >-

Access to read the CS-Domain Location Data resource

nhss-ims-sdm:cs-domain:user-state:read: Access to read the CS-Domain User State resource

nhss-ims-sdm:cs-domain:csrn:read: Access to read the CS-Domain CSRN resource

nhss-ims-sdm:wireline-domain:reference-location:read: >-

Access to read the Wireline-Domain Reference Location resource

nhss-ims-sdm:repository-data:modify: >-

Access to create/update/delete the Repository Data resource

nhss-ims-sdm:repository-data:read: Access to read the Repository Data resource

nhss-ims-sdm:identities:read: Access to read the Identities resource

nhss-ims-sdm:srvcc:read: Access to read the SRVCC resource

nhss-ims-sdm:srvcc:modify: Acess to update the SRVCC resource

nhss-ims-sdm:psi-status:read: Access to read the PSI Status resource

nhss-ims-sdm:psi-status:modify: Acess to update the PSI Status resource

nhss-ims-sdm:dsai:read: Acess to read the DSAI resource

nhss-ims-sdm:dsai:modify: Acess to update the DSAI resource

nhss-ims-sdm:sms-registration-info:read: >-

Acess to read the SMS Registration Info resource

nhss-ims-sdm:sms-registration-info:modify: >-

Acess to create/update/delete the SMS Registration Info resource

nhss-ims-sdm:subscriptions:create: Access to create Subscriptions resources

nhss-ims-sdm:subscription:modify: Access to update/delete a Subscription resource

nhss-ims-sdm:shared-subscriptions:create: >-

Access to create a Shared-Data Subscriptions resource

nhss-ims-sdm:shared-subscription:modify: >-

Access to update/delete a Shared-Data Subscription resource

nhss-ims-sdm:shared-data:read: Access to read the Shared-Data resource

nhss-ims-sdm:charging-info:read: Access to read the ChargingInfo resource

schemas:

#

# COMPLEX TYPES:

#

ScscfCapabilityList:

description: Information about mandatory and optional S-CSCF capabilities

type: object

properties:

mandatoryCapabilityList:

$ref: '#/components/schemas/Capabilities'

optionalCapabilityList:

$ref: '#/components/schemas/Capabilities'

anyOf:

- required: [mandatoryCapabilityList]

- required: [optionalCapabilityList]

Capabilities:

description: A list of capabilities of the S-CSCF

type: array

items:

$ref: '#/components/schemas/Capability'

minItems: 1

uniqueItems: true

RepositoryData:

description: Repository Data for the requested Service Indication

type: object

required:

- serviceData

- sequenceNumber

properties:

sequenceNumber:

$ref: '#/components/schemas/SequenceNumber'

serviceData:

type: string

format: byte

MsisdnList:

description: List of MSISDNs associated to the IMS public Identity

type: object

required:

- basicMsisdn

properties:

basicMsisdn:

$ref: '#/components/schemas/Msisdn'

additionalMsisdns:

type: array

minItems: 1

items:

$ref: '#/components/schemas/Msisdn'

PublicIdentities:

description: >

IMS Public Identities which belong to the same Implicit Registration Set (if any)

as the requested IMS Public Identity

type: object

required:

- publicIdentities

properties:

publicIdentities:

type: array

items:

$ref: '#/components/schemas/PublicIdentity'

minItems: 1

uniqueItems: true

PrivateIdentities:

description: A list of IMS Private Identities

type: object

required:

- privateIdentities

properties:

privateIdentities:

type: array

items:

$ref: '#/components/schemas/PrivateIdentity'

minItems: 1

uniqueItems: true

PublicIdentity:

description: >

IMS Public Identity and related data (Alias Group Id, IRS default indication,

Identity Type)

type: object

required:

- imsPublicId

- identityType

properties:

imsPublicId:

$ref: '#/components/schemas/ImsPublicId'

identityType:

$ref: '#/components/schemas/IdentityType'

irsIsDefault:

type: boolean

aliasGroupId:

type: string

PrivateIdentity:

description: IMS Private Identity and its identity type (IMPI or IMSI)

type: object

required:

- privateIdentity

- privateIdentityType

properties:

privateIdentity:

$ref: '#/components/schemas/PrivateId'

privateIdentityType:

$ref: '#/components/schemas/PrivateIdentityType'

ImsAssociatedIdentities:

description: >

A list of identities belonging to the same Implicit Registration Set (IRS),

along with the registration state of the IRS

type: object

required:

- irsState

- publicIdentities

properties:

irsState:

$ref: '#/components/schemas/ImsRegistrationState'

publicIdentities:

$ref: '#/components/schemas/PublicIdentities'

ImsSdmSubscription:

description: A subscription to notifications of data change

type: object

required:

- nfInstanceId

- callbackReference

- monitoredResourceUris

properties:

nfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

callbackReference:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

monitoredResourceUris:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

expires:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

ImsRegistrationStatus:

description: Registration status of the user

type: object

required:

- imsUserStatus

properties:

imsUserStatus:

$ref: '#/components/schemas/ImsRegistrationState'

PriorityLevels:

description: Namespaces and priority levels allowed for the IMS public Identity

type: object

required:

- servicePriorityLevelList

properties:

servicePriorityLevelList:

type: array

items:

$ref: '#/components/schemas/NameSpacePriority'

minItems: 1

uniqueItems: true

servicePriorityLevel:

type: integer

minimum: 0

maximum: 4

messagingPriority:

type: boolean

default: false

Ifcs:

description: List of IFCs associated to the IMS public Identity

type: object

properties:

ifcList:

type: array

items:

$ref: '#/components/schemas/Ifc'

minItems: 1

cscfFilterSetIdList:

type: array

items:

$ref: '#/components/schemas/CscfFilterSetId'

minItems: 1

anyOf:

- required: [ ifcList ]

- required: [ cscfFilterSetIdList ]

Ifc:

description: Data associated to an individual IFC (Initial Filter Criteria)

type: object

required:

- priority

- appServer

properties:

priority:

type: integer

minimum: 1

trigger:

$ref: '#/components/schemas/TriggerPoint'

appServer:

$ref: '#/components/schemas/ApplicationServer'

TriggerPoint:

description: >

Contains the conditions that should be checked in order to find out if an

Application Server should be contacted or not

type: object

required:

- conditionType

- sptList

properties:

conditionType:

$ref: '#/components/schemas/TypeOfCondition'

sptList:

type: array

items:

$ref: '#/components/schemas/Spt'

minItems: 1

Spt:

description: Contains the data of a Service Point Trigger

type: object

required:

- conditionNegated

- sptGroup

properties:

conditionNegated:

type: boolean

sptGroup:

type: array

items:

$ref: '#/components/schemas/SptGroupId'

minItems: 1

regType:

items:

$ref: '#/components/schemas/RegistrationType'

minItems: 1

maxItems: 2

requestUri:

type: string

sipMethod:

type: string

sipHeader:

$ref: '#/components/schemas/HeaderSipRequest'

sessionCase:

$ref: '#/components/schemas/RequestDirection'

sessionDescription:

$ref: '#/components/schemas/SdpDescription'

HeaderSipRequest:

description: Contains a header (and optionally value of the header) in the SIP request

type: object

required:

- header

properties:

header:

type: string

content:

type: string

SdpDescription:

description: >

Contains a SDP line (and optionally the value in the line) within the body (if any)

of a SIP request

type: object

required:

- line

properties:

line:

type: string

content:

type: string

ApplicationServer:

description: >

Application Server which shall be triggered if the conditions of a certain IFC are met,

and its associated data

type: object

required:

- asUri

properties:

asUri:

type: string

sessionContinue:

type: boolean

serviceInfoList:

type: array

items:

$ref: '#/components/schemas/ServiceInformation'

minItems: 1

ImsLocationData:

description: IMS Location Data (S-CSCF name)

type: object

required:

- scscfName

properties:

scscfName:

type: string

PsLocation:

description: Location data in PS domain

type: object

oneOf:

- required:

- sgsnLocationData

- required:

- mmeLocationData

- required:

- amfLocationData

- required:

- twanLocationData

properties:

sgsnLocationData:

$ref: '#/components/schemas/SgsnLocationData'

mmeLocationData:

$ref: '#/components/schemas/MmeLocationData'

amfLocationData:

$ref: '#/components/schemas/AmfLocationData'

twanLocationData:

$ref: '#/components/schemas/TwanLocationData'

SgsnLocationData:

description: Location information as retrieved from the SGSN serving node

type: object

required:

- sgsnNumber

- plmnId

properties:

sgsnNumber:

type: string

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

sgsnLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UtraLocation'

csgInformation:

$ref: '#/components/schemas/CsgInformation'

timeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

MmeLocationData:

description: Location information as retrieved from the MME serving node

type: object

required:

- mmeAddress

- plmnId

properties:

mmeAddress:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DiameterIdentity'

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

mmeLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/EutraLocation'

csgInformation:

$ref: '#/components/schemas/CsgInformation'

timeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

AmfLocationData:

description: Location information as retrieved from the AMF serving node

type: object

required:

- amfAddress

- plmnId

properties:

amfAddress:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

amfLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NrLocation'

smsfAddress:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

timeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

TwanLocationData:

description: Location information of the 3GPP AAA Server for Trusted W-LAN Access

type: object

required:

- twanSsid

- plmnId

properties:

twanSsid:

type: string

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

twanBssid:

type: string

civicAddress:

type: string

format: byte

twanOperatorName:

type: string

timeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

logicalAccessId:

type: string

CsLocation:

description: Location data in CS domain

type: object

required:

- mscNumber

- vlrNumber

- plmnId

properties:

mscNumber:

type: string

vlrNumber:

type: string

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

vlrLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GeraLocation'

csgInformation:

$ref: '#/components/schemas/CsgInformation'

timeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

eUtranCgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

tai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

CsgInformation:

description: Information about a Closed Subscriber Group (CSG)

type: object

required:

- csgId

properties:

csgId:

type: string

format: byte

accessMode:

type: string

format: byte

cMi:

type: boolean

SrvccData:

description: UE SRVCC capability (if available) and STN-SR (if subscribed)

type: object

required:

- stnSr

properties:

stnSr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/StnSr'

ueSrvccCapabilities:

type: array

items:

$ref: '#/components/schemas/SrvccCapability'

minItems: 1

uniqueItems: true

PsiActivationState:

description: Public Service Identity activation state

type: object

required:

- activationState

properties:

activationState:

$ref: '#/components/schemas/ActivationState'

SharedData:

description: Represents a part of the IMS Profile data shared by several UEs

type: object

required:

- sharedDataId

properties:

sharedDataId:

$ref: '#/components/schemas/SharedDataId'

sharedImsIfcData:

$ref: '#/components/schemas/Ifcs'

minProperties: 1

ImsProfileData:

description: User's IMS profile data

type: object

required:

- imsServiceProfiles

properties:

imsServiceProfiles:

type: array

items:

$ref: '#/components/schemas/ImsServiceProfile'

chargingInfo:

$ref: '#/components/schemas/ChargingInfo'

serviceLevelTraceInfo:

$ref: '#/components/schemas/ServiceLevelTraceInformation'

servicePriorityLevelList:

type: array

items:

$ref: '#/components/schemas/NameSpacePriority'

minItems: 1

uniqueItems: true

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

maxAllowedSimulReg:

type: integer

servicePriorityLevel:

type: integer

minimum: 0

maximum: 4

ImsServiceProfile:

description: >

IMS Service Profile of the UE, containing the list of Public Identifiers and

optionally a list of IFCs

type: object

required:

- publicIdentifierList

properties:

publicIdentifierList:

type: array

items:

$ref: '#/components/schemas/PublicIdentifier'

ifcs:

$ref: '#/components/schemas/Ifcs'

cnServiceAuthorization:

$ref: '#/components/schemas/CoreNetworkServiceAuthorization'

CoreNetworkServiceAuthorization:

description: >

Core Network Service Authorization

type: object

properties:

subscribedMediaProfileId:

type: integer

PublicIdentifier:

description: >

Distinct or wildcarded public identity and its associated priority, trace and

barring information

type: object

required:

- publicIdentity

properties:

publicIdentity:

$ref: '#/components/schemas/PublicIdentity'

displayName:

type: string

imsServicePriority:

$ref: '#/components/schemas/PriorityLevels'

serviceLevelTraceInfo:

$ref: '#/components/schemas/ServiceLevelTraceInformation'

barringIndicator:

type: boolean

wildcardedImpu:

type: string

ServiceLevelTraceInformation:

description: IMS Service Level Trace Information

type: object

properties:

serviceLevelTraceInfo:

type: string

ImeiSvInformation:

description: IMEI(SV) information

type: object

oneOf:

- required:

- imei

- required:

- imeiSv

properties:

imei:

$ref: '#/components/schemas/Imei'

imeiSv:

$ref: '#/components/schemas/ImeiSv'

TadsInformation:

description: T-ADS Information

type: object

required:

- voiceOverPsSessionSupport

properties:

voiceOverPsSessionSupport:

$ref: '#/components/schemas/ImsVoiceOverPsSessionSupport'

accessType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

lastUeActivityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

UeReachabilitySubscription:

description: >

Contains the request parameters received by the HSS for a subscription to

notifications of UE reachability for IP

type: object

required:

- expiry

- callbackReference

properties:

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

callbackReference:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

CreatedUeReachabilitySubscription:

description: >

Contains the response data returned by HSS after the subscription to

notifications of UE reachability for IP was created

type: object

required:

- expiry

properties:

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

UeReachabilityNotification:

description: >

Represents the contents of a notification of UE reachability for IP sent by the HSS

type: object

required:

- reachabilityIndicator

- detectingNode

- accessType

properties:

reachabilityIndicator:

type: boolean

detectingNode:

$ref: '#/components/schemas/DetectingNode'

accessType:

$ref: '#/components/schemas/AccessType'

PsUserState:

description: User state in PS domain

type: object

oneOf:

- required:

- sgsnUserState

- required:

- mmeUserState

- required:

- amfUserState

properties:

sgsnUserState:

$ref: '#/components/schemas/UserStatePs'

mmeUserState:

$ref: '#/components/schemas/UserStatePs'

amfUserState:

$ref: '#/components/schemas/UserStatePs'

CsUserState:

description: User state in CS domain

type: object

required:

- mscVlrUserState

properties:

mscVlrUserState:

$ref: '#/components/schemas/UserStateCs'

Csrn:

description: CS domain routeing number

type: object

required:

- csrn

properties:

csrn:

$ref: '#/components/schemas/Msisdn'

ReferenceLocationInformation:

description: Reference Location Information for the user in fixed access networks

type: object

properties:

accessType:

type: string

accessInfo:

type: string

accessValue:

type: string

anyOf:

- required: [accessType]

- required: [accessInfo]

- required: [accessValue]

SmsRegistrationInfo:

description: SMS Registration Information (IP-SM-GW number and SC address)

type: object

required:

- ipSmGwNumber

properties:

ipSmGwNumber:

$ref: '#/components/schemas/Msisdn'

ipSmGwAddress:

$ref: '#/components/schemas/IpSmGwAddress'

scAddress:

$ref: '#/components/schemas/Msisdn'

IpSmGwAddress:

description: IP-SM-GW number and diameter URI/realm

type: object

required:

- ipSmGwNumber

properties:

ipSmGwNumber:

$ref: '#/components/schemas/Msisdn'

ipSmGwDiaUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DiameterIdentity'

ipSmGwDiaRealm:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DiameterIdentity'

ipSmGwSbiSupInd:

type: boolean

default: false

ipSmGwInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

DsaiTagInformation:

description: >

DSAI tag information (list of DSAI tags activation state for an Application Server)

type: object

required:

- dsaiTagStatusList

properties:

dsaiTagStatusList:

type: array

items:

$ref: '#/components/schemas/DsaiTagStatus'

minItems: 1

uniqueItems: true

DsaiTagStatus:

description: DSAI tag status (DSAI tag and activation state)

type: object

required:

- dsaiTag

- dsaiState

properties:

dsaiTag:

type: string

dsaiState:

$ref: '#/components/schemas/ActivationState'

ScscfSelectionAssistanceInformation:

description: Information used by the I-CSCF to select an S-CSCF for the UE

type: object

properties:

scscfCapabilityList:

$ref: '#/components/schemas/ScscfCapabilityList'

scscfNames:

type: array

items:

type: string

minItems: 1

anyOf:

- required: [scscfCapabilityList]

- required: [scscfNames]

ChargingInfo:

description: Diameter addresses of the charging function

type: object

anyOf:

- required:

- primaryEventChargingFunctionName

- required:

- primaryChargingCollectionFunctionName

properties:

primaryEventChargingFunctionName:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DiameterIdentity'

secondaryEventChargingFunctionName:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DiameterIdentity'

primaryChargingCollectionFunctionName:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DiameterIdentity'

secondaryChargingCollectionFunctionName:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DiameterIdentity'

CallReferenceInfo:

description: >

Contains a Call Reference Number and the AS-Number; it allows a later retry of the call

setup in the context of MTRR

type: object

required:

- callRefNumber

- asNumber

properties:

callRefNumber:

type: string

format: byte

description: >

The content is according to CallReferenceNumber type described in 3GPP TS 29.002 [30].

Base64 encoded according to IETF RFC 2045 [28]

asNumber:

type: string

format: byte

description: >

The content is according to ISDN-AddressString type described in 3GPP TS 29.002 [30].

Base64 encoded according to IETF RFC 2045 [28]

DataSetNames:

description: Contains the data sets to be retrieved from the IMS User Profile

type: array

items:

$ref: '#/components/schemas/DataSetName'

minItems: 1

uniqueItems: true

RepositoryDataList:

description: List of Repository Data for the requested Service Indications

type: object

required:

- repositoryDataMap

properties:

repositoryDataMap:

description: >

A map (list of key-value pairs where ServiceIndication serves as key) of RepositoryData

type: object

additionalProperties:

$ref: '#/components/schemas/RepositoryData'

#

# SIMPLE TYPES:

#

Capability:

description: S-CSCF capability

type: integer

ImsUeId:

description: >

Represents the IMS Public Identity (i.e. IMS Public User identity or

Public Service Identity)

type: string

pattern: '^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|impu-tel\:\+[0-9]{5,15}|impi-.+|.+)$'

SequenceNumber:

description: >

Unsigned integer containing the sequence number associated to the current version

of Repository Data

type: integer

minimum: 0

ServiceIndication:

description: String containing the Service Indication or Service Identifier

type: string

Msisdn:

description: String containing an additional or basic MSISDN

type: string

pattern: '[0-9]{5,15}$'

PrivateId:

description: String containing a Private User Identity or a Private Service Identity

type: string

ImsPublicId:

description: String containing an IMS Public Identity in SIP URI format or TEL URI format

type: string

pattern: '^(sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|tel\:\+[0-9]{5,15})$'

NameSpacePriority:

description: >

String containing the namespace and priority formatted as "r-value" parameter

described in IETF RFC 4412

type: string

pattern: '^[0-9a-zA-Z-\!%\*_\+`''~]+.[0-9a-zA-Z-\!%\*_\+`''~]+$'

CscfFilterSetId:

description: Contains the identifier of a specific filter set in the S-CSCF

type: integer

minimum: 0

SptGroupId:

description: Contains the identifier of the Group of SPTs

type: integer

minimum: 0

SharedDataId:

description: >

Identifies globally and uniquely a piece of subscription data shared by multiple UEs;

the value shall start with the HPLMN id (MCC/MNC) followed by a hyphen followed by a

local Id as allocated by the home network operator

type: string

pattern: '^[0-9]{5,6}-.+$'

Imei:

description: IMEI of the UE

type: string

pattern: '^[0-9]{15}$'

ImeiSv:

description: IMEISV of the UE

type: string

pattern: '^[0-9]{16}$'

PrePaging:

description: Inidicates the support of pre-paging

type: boolean

SipServerName:

description: Server Name (e.g. AS name, CSCF name) in SIP URI format

type: string

pattern: '^(sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,})$'

#

# ENUMS:

#

IdentityType:

description: Represents the type of IMS Public Identity

anyOf:

- type: string

enum:

- DISTINCT_IMPU

- DISTINCT_PSI

- WILDCARDED_IMPU

- WILDCARDED_PSI

- type: string

PrivateIdentityType:

description: Represents the type of IMS Private Identity (IMPI or IMSI)

anyOf:

- type: string

enum:

- IMPI

- IMSI

- type: string

ImsRegistrationState:

description: Represents the IMS status of the user or Public Identity or the IRS

anyOf:

- type: string

enum:

- REGISTERED

- NOT_REGISTERED

- AUTHENTICATION_PENDING

- REGISTERED_UNREG_SERVICES

- type: string

TypeOfCondition:

description: >

Represents the type of condition to be applied to the logical expression of SPT groups

and SPTs (Conjunctive Normal Form, or Disjunctive Normal Form)

anyOf:

- type: string

enum:

- CNF

- DNF

- type: string

RegistrationType:

description: Represents the type of registration associated to the REGISTER request

anyOf:

- type: string

enum:

- INITIAL_REGISTRATION

- RE_REGISTRATION

- DE_REGISTRATION

- type: string

RequestDirection:

description: >

Represents the direction of the request in combination with the registration status

of the user as evaluated in the S-CSCF

anyOf:

- type: string

enum:

- ORIGINATING_REGISTERED

- ORIGINATING_UNREGISTERED

- ORIGINATING_CDIV

- TERMINATING_REGISTERED

- TERMINATING_UNREGISTERED

- type: string

ServiceInformation:

description: >

Indicates the type of additional information to be included in the body of the SIP request

towards the Application Server

anyOf:

- type: string

enum:

- INCLUDE_REGISTER_REQUEST

- INCLUDE_REGISTER_RESPONSE

- type: string

RequestedNode:

description: Represents the type of serving node for which certain data is requested

anyOf:

- type: string

enum:

- SGSN

- MME

- AMF

- 3GPP_AAA_SERVER_TWAN

- type: string

SrvccCapability:

description: Represents the type of SRVCC support of the UE

anyOf:

- type: string

enum:

- UE_4G_SRVCC_CAPABLE

- UE_5G_SRVCC_CAPABLE

- type: string

ActivationState:

description: Represents the activation state of a PSI or DSAI tag

anyOf:

- type: string

enum:

- ACTIVE

- INACTIVE

- type: string

ImsVoiceOverPsSessionSupport:

description: Represents the support for Voice-over-PS of the UE

anyOf:

- type: string

enum:

- IMS_VOICE_OVER_PS_NOT_SUPPORTED

- IMS_VOICE_OVER_PS_SUPPORTED

- IMS_VOICE_OVER_PS_SUPPORT_UNKNOWN

- type: string

DetectingNode:

description: Represents the type of serving node that detected the reachability of the UE

anyOf:

- type: string

enum:

- SGSN

- MME

- AMF

- type: string

AccessType:

description: Represents the type of access (3GPP or non-3GPP)

anyOf:

- type: string

enum:

- 3GPP_ACCESS

- NON_3GPP_ACCESS

- type: string

UserStatePs:

description: Represents the user state in PS domain

anyOf:

- type: string

enum:

- DETACHED

- ATTACHED_NOT_REACHABLE_FOR_PAGING

- ATTACHED_REACHABLE_FOR_PAGING

- CONNECTED_NOT_REACHABLE_FOR_PAGING

- CONNECTED_REACHABLE_FOR_PAGING

- NOT_PROVIDED_FROM_SGSN_OR_MME_OR_AMF

- NETWORK_DETERMINED_NOT_REACHABLE

- type: string

UserStateCs:

description: Represents the user state in CS domain

anyOf:

- type: string

enum:

- CAMEL_BUSY

- NETWORK_DETERMINED_NOT_REACHABLE

- ASSUMED_IDLE

- NOT_PROVIDED_FROM_VLR

- type: string

DataSetName:

description: A set of data within the IMS User Profile

anyOf:

- type: string

enum:

- CHARGING_DATA

- IFC_DATA

- TRACE_DATA

- PRIORITY_DATA

- type: string

# A.4	Nhss_imsUEAU API

openapi: 3.0.0

info:

version: '1.3.0'

title: 'Nhss_imsUEAU'

description: |

Nhss UE Authentication Service for IMS.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.562 Home Subscriber Server (HSS) Services, version 19.4.0

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.562/'

servers:

- url: '{apiRoot}/nhss-ims-ueau/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-ueau

paths:

/{impi}/security-information/generate-sip-auth-data:

post:

summary: Generate authentication data for the UE based on the Auth-Scheme provided

operationId: GenerateSipAuthData

tags:

- Generate SIP Auth Data (Custom Operation)

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-ueau

- oAuth2ClientCredentials:

- nhss-ims-ueau

- nhss-ims-ueau:generate-sip-auth-data:invoke

parameters:

- name: impi

in: path

description: IMS Private Identity for the UE (IMPI)

required: true

schema:

$ref: '#/components/schemas/Impi'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/SipAuthenticationInfoRequest'

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SipAuthenticationInfoResult'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nhss-ims-ueau: Access to the Nhss IMS UE Authentication API

nhss-ims-ueau:generate-sip-auth-data:invoke: >

Access to invoke the Generate SIP Auth Data custom operation

schemas:

#

# COMPLEX TYPES:

#

SipAuthenticationInfoRequest:

description: >

Contains input data to the SIP authentication request message (e.g. SIP authentication

scheme, requested number of authentication items, resynchronization information)

type: object

required:

- cscfServerName

- sipAuthenticationScheme

properties:

cscfServerName:

type: string

sipAuthenticationScheme:

$ref: '#/components/schemas/SipAuthenticationScheme'

sipNumberAuthItems:

$ref: '#/components/schemas/SipNumberAuthItems'

resynchronizationInfo:

$ref: '#/components/schemas/ResynchronizationInfo'

ResynchronizationInfo:

description: Contains RAND and AUTS

type: object

required:

- rand

- auts

properties:

rand:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Rand'

auts:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Auts'

SipAuthenticationInfoResult:

description: >

Contains authentication information returned in the SIP authentication response message

(e.g. authentication vectors, digest authentication parameters, line identifiers)

type: object

required:

- impi

properties:

impi:

$ref: '#/components/schemas/Impi'

3gAkaAvs:

type: array

items:

$ref: '#/components/schemas/3GAkaAv'

minItems: 1

digestAuth:

$ref: '#/components/schemas/DigestAuthentication'

lineIdentifierList:

type: array

items:

$ref: '#/components/schemas/LineIdentifier'

minItems: 1

ipAddress:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

3GAkaAv:

description: Contains RAND, XRES, AUTN, CK, and IK

type: object

required:

- rand

- xres

- autn

- ck

- ik

properties:

rand:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Rand'

xres:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Xres'

autn:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Autn'

ck:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/ConfidentialityKey'

ik:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/IntegrityKey'

DigestAuthentication:

description: Parameters used for the Digest authentication scheme

type: object

required:

- digestRealm

- digestAlgorithm

- digestQop

- ha1

properties:

digestRealm:

type: string

digestAlgorithm:

$ref: '#/components/schemas/SipDigestAlgorithm'

digestQop:

$ref: '#/components/schemas/SipDigestQop'

ha1:

type: string

#

# SIMPLE TYPES:

#

SipNumberAuthItems:

description: Indicates the number of requested SIP authentication items

type: integer

minimum: 1

Impi:

description: IMS Private Identity of the UE

type: string

LineIdentifier:

description: Line Identifier for the wireline access

type: string

#

# ENUMS:

#

SipAuthenticationScheme:

description: Authentication scheme to be used in the SIP authentication request message

anyOf:

- type: string

enum:

- DIGEST-AKAV1-MD5

- DIGEST-HTTP

- NBA

- GIBA

- UNKNOWN

- type: string

SipDigestAlgorithm:

description: Algorithm used for the SIP Digest authentication scheme

anyOf:

- type: string

enum:

- MD5

- MD5_SESS

- type: string

SipDigestQop:

description: Quality of Protection for the SIP Digest authentication scheme

anyOf:

- type: string

enum:

- AUTH

- AUTH_INT

- type: string

# A.5	Nhss_gbaSDM API

openapi: 3.0.0

info:

version: '1.2.0'

title: 'Nhss_gbaSDM'

description: |

Nhss Subscriber Data Management Service for GBA.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.562 Home Subscriber Server (HSS) Services, version 19.4.0

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.562/'

servers:

- url: '{apiRoot}/nhss-gba-sdm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- nhss-gba-sdm

paths:

/{ueId}/subscriber-data:

get:

summary: Retrieve the subscriber data of a user

operationId: GetSubscriberData

tags:

- Subscriber Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nhss-gba-sdm

- oAuth2ClientCredentials:

- nhss-gba-sdm

- nhss-gba-sdm:subscriber-data:read

parameters:

- name: ueId

in: path

description: UE Identity

required: true

schema:

$ref: '#/components/schemas/UeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/GbaSubscriberData'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{ueId}/subscriptions:

post:

summary: Subscribe to notifications

operationId: GbaSdmSubscribe

tags:

- GBA SDM Subscriptions (Collection)

security:

- {}

- oAuth2ClientCredentials:

- nhss-gba-sdm

- oAuth2ClientCredentials:

- nhss-gba-sdm

- nhss-gba-sdm:subscriptions:create

parameters:

- name: ueId

in: path

description: UE Identity

required: true

schema:

$ref: '#/components/schemas/UeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/GbaSdmSubscription'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/GbaSdmSubscription'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/nhss-gba-sdm/<apiVersion>/{ueId}/subscriptions/{subscriptionId}

required: true

schema:

type: string

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

datachangeNotification:

'{$request.body#/callbackReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ModificationNotification'

responses:

'204':

description: Successful Notification response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{ueId}/subscriptions/{subscriptionId}:

delete:

summary: Unsubscribe from notifications

operationId: GbaSdmUnsubscribe

tags:

- GBA SDM Subscription (Document)

security:

- {}

- oAuth2ClientCredentials:

- nhss-gba-sdm

- oAuth2ClientCredentials:

- nhss-gba-sdm

- nhss-gba-sdm:subscription:modify

parameters:

- name: ueId

in: path

description: UE Identity

required: true

schema:

$ref: '#/components/schemas/UeId'

- name: subscriptionId

in: path

description: Id of the Subscription

required: true

schema:

type: string

responses:

'204':

description: Successful response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify the subscription

operationId: GbaSdmSubsModify

tags:

- GBA SDM Subscription (Document)

security:

- {}

- oAuth2ClientCredentials:

- nhss-gba-sdm

- oAuth2ClientCredentials:

- nhss-gba-sdm

- nhss-gba-sdm:subscription:modify

parameters:

- name: ueId

in: path

description: UE Identity

required: true

schema:

$ref: '#/components/schemas/UeId'

- name: subscriptionId

in: path

description: Id of the Subscription

required: true

schema:

type: string

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'204':

description: Successful modification

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nhss-gba-sdm: Access to the Nhss GBA Subscription Data Management API

nhss-gba-sdm:subscriber-data:read: Access to read the GBA Subscriber Data resource

nhss-gba-sdm:subscriptions:create: Access to create Subscriptions resources

nhss-gba-sdm:subscription:modify: Access to update/delete a Subscription resource

schemas:

#

# COMPLEX TYPES:

#

GbaSubscriberData:

description: >

GBA subscriber data of the UE; it includes the GBA User Security Settings (GUSS) data

type: object

properties:

guss:

$ref: '#/components/schemas/Guss'

Guss:

description: >

GBA User Security Settings of the UE; it includes information about the BSF,

and a list of User Security Settings

type: object

properties:

bsfInfo:

$ref: '#/components/schemas/BsfInfo'

ussList:

type: array

items:

$ref: 'TS29309_Nbsp_GBA.yaml#/components/schemas/UssListItem'

minItems: 1

BsfInfo:

description: >

Information about the BSF; it includes an indication of whether GBA or GBA_U is to be used,

the key lifetime and a user-specific list of security features

type: object

properties:

uiccType:

$ref: '#/components/schemas/UiccType'

lifeTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

securityFeatures:

type: array

items:

$ref: 'TS29309_Nbsp_GBA.yaml#/components/schemas/SecFeature'

minItems: 1

GbaSdmSubscription:

description: >

Information about a subscription created in the HSS, so the consumer (e.g. BSF) can be

notified when there are changes on the monitored data

type: object

required:

- nfInstanceId

- callbackReference

- monitoredResourceUris

properties:

nfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

callbackReference:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

monitoredResourceUris:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

expires:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

#

# SIMPLE TYPES:

#

UeId:

description: UE ID can be MSISDN, IMSI, IMPI or IMPU

anyOf:

- $ref: '#/components/schemas/Msisdn'

- $ref: '#/components/schemas/Imsi'

- $ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsUeId'

Msisdn:

description: MS-ISDN of the UE

type: string

pattern: '^msisdn-[0-9]{5,15}$'

Imsi:

description: IMSI of the UE

type: string

pattern: '^imsi-[0-9]{5,15}$'

#

# ENUMS:

#

UiccType:

description: Indicates whether GBA or GBA_U is to be used

anyOf:

- type: string

enum:

- GBA

- GBA_U

- type: string

# A.6	Nhss_gbaUEAU API

openapi: 3.0.0

info:

version: '1.2.0'

title: 'Nhss_gbaUEAU'

description: |

Nhss UE Authentication Service for GBA.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.562 Home Subscriber Server (HSS) Services, version 19.4.0

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.562/'

servers:

- url: '{apiRoot}/nhss-gba-ueau/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- oAuth2ClientCredentials:

- nhss-gba-ueau

- {}

paths:

/{ueId}/security-information/generate-auth-data:

post:

summary: Generate GBA authentication data for the UE

operationId: GenerateAuthData

tags:

- Generate Auth Data (Custom Operation)

security:

- {}

- oAuth2ClientCredentials:

- nhss-gba-ueau

- oAuth2ClientCredentials:

- nhss-gba-ueau

- nhss-gba-ueau:generate-auth-data:invoke

parameters:

- name: ueId

in: path

description: UE identity of the user

required: true

schema:

$ref: '#/components/schemas/UeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AuthenticationInfoRequest'

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/AuthenticationInfoResult'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nhss-gba-ueau: Access to the nhss-gba-ueau API

nhss-gba-ueau:generate-auth-data:invoke: >

Access to invoke the Generate Auth Data custom operation

schemas:

#

# COMPLEX TYPES:

#

AuthenticationInfoRequest:

description: >

Contains input data to the authentication request message (e.g. authentication

scheme, resynchronization information)

type: object

properties:

authenticationScheme:

$ref: '#/components/schemas/AuthenticationScheme'

resynchronizationInfo:

$ref: 'TS29562_Nhss_imsUEAU.yaml#/components/schemas/ResynchronizationInfo'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

AuthenticationInfoResult:

description: >

Contains authentication information returned in the authentication response message

(e.g. authentication vector, digest authentication parameters)

type: object

properties:

impi:

$ref: '#/components/schemas/Impi'

3gAkaAv:

$ref: 'TS29562_Nhss_imsUEAU.yaml#/components/schemas/3GAkaAv'

digestAuth:

$ref: '#/components/schemas/DigestAuthentication'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

DigestAuthentication:

description: Parameters used for the Digest authentication scheme

type: object

required:

- digestRealm

- digestAlgorithm

- digestQop

- ha1

properties:

digestRealm:

type: string

digestAlgorithm:

$ref: '#/components/schemas/DigestAlgorithm'

digestQop:

$ref: '#/components/schemas/DigestQop'

ha1:

type: string

#

# SIMPLE TYPES:

#

UeId:

$ref: 'TS29562_Nhss_gbaSDM.yaml#/components/schemas/UeId'

Impi:

description: IMS Private Identity of the UE

type: string

#

# ENUMS:

#

AuthenticationScheme:

description: Authentication scheme to be used in the authentication request message

anyOf:

- type: string

enum:

- DIGEST_AKAV1_MD5

- DIGEST_HTTP

- type: string

DigestAlgorithm:

description: Algorithm used for the Digest authentication scheme

anyOf:

- type: string

enum:

- MD5

- MD5_SESS

- type: string

DigestQop:

description: Quality of Protection for the Digest authentication scheme

anyOf:

- type: string

enum:

- AUTH

- AUTH_INT

- type: string

# A.7	Nhss_imsEE API

openapi: 3.0.0

info:

version: '1.0.0'

title: 'Nhss_imsEE'

description: |

Nhss Event Exposure Service for IMS.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.562 V19.4.0; Home Subscriber Server (HSS) Services

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.562/

servers:

- url: '{apiRoot}/nhss-ims-ee/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-ee

paths:

/{imsUeId}/ee-subscriptions:

post:

summary: Subscribe to IMS Event

operationId: ImsEventSubscribe

tags:

- IMS Event Subscription Creation

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-ee

- oAuth2ClientCredentials:

- nhss-ims-ee

- nhss-ims-ee:ee-subscriptions:create

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

type: string

pattern: '^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|impu-tel\:\+[0-9]{5,15}|.+)$'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ImsEeSubscription'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/CreatedImsEeSubscription'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/nhss-ims-ee/v1/{imsUeId}/ee-subscriptions/{subscriptionId}

required: true

schema:

type: string

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

imsMonitoringRevocationNotification:

'{$request.body#/revocationNotifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ImsEeMonitoringRevoked'

responses:

'204':

description: Successful Notification response

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{imsUeId}/ee-subscriptions/{subscriptionId}:

delete:

summary: unsubscribe from Individual IMS Event

operationId: ImsEventUnsubscribe

tags:

- Individual IMS Event Subscription Deletion

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-ee

- oAuth2ClientCredentials:

- nhss-ims-ee

- nhss-ims-ee:ee-subscriptions:modify

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

type: string

pattern: '^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|impu-tel\:\+[0-9]{5,15}|.+)$'

- name: subscriptionId

in: path

description: Id of the Subscription

required: true

schema:

type: string

responses:

'204':

description: Successful response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: modify the subscription

operationId: ImsEventSubsModify

tags:

- Individual IMS Event Subscription Modification

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-ee

- oAuth2ClientCredentials:

- nhss-ims-ee

- nhss-ims-ee:subscription:modify

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

type: string

pattern: '^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|impu-tel\:\+[0-9]{5,15}|.+)$'

- name: subscriptionId

in: path

description: Id of the Subscription

required: true

schema:

type: string

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/CreatedImsEeSubscription'

'204':

description: Successful response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nhss-ims-ee: Access to the Nhss IMS Event Exposure API

nhss-ims-ee:ee-subscriptions:create: >-

Access to create IMS Event Subscriptions resource

nhss-ims-ee:ee-subscriptions:modify: >-

Access to update/delete the Individual IMS Event Subscription resource

schemas:

#

# COMPLEX TYPES:

#

ImsEeSubscription:

description: An Individual IMS Event Subscription

type: object

required:

- notificationURI

- subscrImsEvents

properties:

notifId:

type: string

afId:

type: string

notificationURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

subscrImsEvents:

description: >

the IMS events to be subscribed or successfully subscribed in a

subscription reponse within the referenceId converted from integer

to string is the key of the map.

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ImsEventConfiguration'

minProperties: 1

imsReportingOptions:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ImsReportingOptions'

revocationNotifURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

CreatedImsEeSubscription:

description: The created Individual IMS Event Subscription

type: object

required:

- imsEeSubscription

properties:

imsEeSubscription:

$ref: '#/components/schemas/ImsEeSubscription'

statusEvents:

type: array

minItems: 1

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ImsEventReport'

ImsEeMonitoringRevoked:

type: object

required:

- revokedMonitoringEventList

properties:

revokedMonitoringEventList:

description: >

A map (list of key-value pairs where ReferenceId serves as key)

of ImsEeMonitoringEvents

type: object

additionalProperties:

$ref: '#/components/schemas/ImsEeMonitoringEvent'

minProperties: 1

revokedImsUeId:

$ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsPublicId'

ImsEeMonitoringEvent:

type: object

required:

- eventType

properties:

eventType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ImsEvent'

revokedCause:

$ref: '#/components/schemas/ImsRevokedCause'

#

# SIMPLE TYPES

#

#

# ENUMS

#

ImsRevokedCause:

description: Revocation Cause for the IMS monitoring event.

anyOf:

- type: string

enum:

- MONITORING_NOT_ALLOWED

- IMS_UE_ID_REMOVED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

Annex B (informative):
Withdrawn API versions

# B.1	General

This Annex lists withdrawn API versions of the APIs defined in the present specification. 3GPP TS 29.501 [5] clause 4.3.1.6 describes the withdrawal of API versions.

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-09 | CT4#93 | C4-193851 |  |  |  | Initial Draft (C4-193264) and incorporation of agreed pCRs at CT4#93: C4-193576, C4-193272, C4-193273, C4-193579, C4-193706, C4-193838. | 0.1.0 |
| 2019-10 | CT4#94 | C4-194525 |  |  |  | Incorporation of agreed pCRs at CT4#94: C4-194361, C4-194362, C4-194364, C4-194365, C4-194556 | 0.2.0 |
| 2019-11 | CT4#95 | C4-195640 |  |  |  | Incorporation of agreed pCRs at CT4#95: C4-195468, C4-195469, C4-195475, C4-195478, C4-195583, C4-195650 | 0.3.0 |
| 2020-03 | CT4#96 | C4-201270 |  |  |  | Incorporation of agreed pCRs at CT4#96: C4-200572, C4-200578, C4-200582, C4-200803, C4-200804, C4-200927, C4-200929, C4-200963, C4-200977, C4-201098, C4-201104, C4-201105, C4-201106, C4-201107, C4-201108, C4-201109, C4-201110, C4-201111, C4-201112, C4-201113, C4-201114, C4-201115, C4-201116 | 0.4.0 |
| 2020-03 | CT#87 | CP-200064 |  |  |  | TS presented for information and approval | 1.0.0 |
| 2020-03 | CT#87 |  |  |  |  | Approved at CT#87 | 16.0.0 |
| 2020-07 | CT#88 | CP-201038 | 0001 | - | B | IMEISV retrieval | 16.1.0 |
| 2020-07 | CT#88 | CP-201038 | 0002 | - | B | IP address retrieval | 16.1.0 |
| 2020-07 | CT#88 | CP-201038 | 0003 | - | B | T-ADS info retrieval | 16.1.0 |
| 2020-07 | CT#88 | CP-201038 | 0004 | - | B | UE Reachability subscription | 16.1.0 |
| 2020-07 | CT#88 | CP-201038 | 0005 | - | B | User State retrieval | 16.1.0 |
| 2020-07 | CT#88 | CP-201038 | 0006 | - | B | Storage of YAML files in ETSI Forge | 16.1.0 |
| 2020-07 | CT#88 | CP-201038 | 0007 | - | B | Resource-Level Authorization | 16.1.0 |
| 2020-07 | CT#88 | CP-201038 | 0008 | - | F | Common data structures in the response body | 16.1.0 |
| 2020-07 | CT#88 | CP-201038 | 0009 | - | B | CSRN retrieval | 16.1.0 |
| 2020-07 | CT#88 | CP-201038 | 0010 | 1 | B | Wireline domain info retrieval | 16.1.0 |
| 2020-07 | CT#88 | CP-201038 | 0011 | - | B | SDM SMS Registration Info | 16.1.0 |
| 2020-07 | CT#88 | CP-201038 | 0012 | 1 | F | Miscellaneous corrections | 16.1.0 |
| 2020-07 | CT#88 | CP-201038 | 0013 | 1 | F | IMS restoration amendments | 16.1.0 |
| 2020-07 | CT#88 | CP-201038 | 0014 | 1 | F | Support of Multiple Registration | 16.1.0 |
| 2020-07 | CT#88 | CP-201038 | 0015 | 2 | F | IMS Identity Data Retrieval amendments | 16.1.0 |
| 2020-07 | CT#88 | CP-201038 | 0016 | - | F | SDM Wireline-domain Line-Id Removal | 16.1.0 |
| 2020-07 | CT#88 | CP-201038 | 0017 | 1 | B | SDM - DSAI handling | 16.1.0 |
| 2020-07 | CT#88 | CP-201038 | 0018 | 1 | F | Cleanup of TS | 16.1.0 |
| 2020-07 | CT#88 | CP-201038 | 0019 | - | F | Miscellaneous Corrections | 16.1.0 |
| 2020-07 | CT#88 | CP-201073 | 0020 | - | F | 29.562 Rel-16 API version and External doc update | 16.1.0 |
| 2020-09 | CT#89 | CP-202118 | 0021 | - | F | P-CSCF Restoration | 16.2.0 |
| 2020-09 | CT#89 | CP-202118 | 0022 | - | F | Missing Resource Scopes | 16.2.0 |
| 2020-09 | CT#89 | CP-202118 | 0023 | 1 | F | IMS restoration amendments II | 16.2.0 |
| 2020-09 | CT#89 | CP-202118 | 0024 | - | F | Miscellaneous Corrections | 16.2.0 |
| 2020-09 | CT#89 | CP-202096 | 0025 | - | F | 29.562 Rel-16 API version and External doc update | 16.2.0 |
| 2020-12 |  |  |  |  |  | Corrupted figures fixed | 16.2.1 |
| 2020-12 | CT#90 | CP-203053 | 0027 | - | F | Storage of YAML files in 3GPP Forge | 16.3.0 |
| 2020-12 | CT#90 | CP-203053 | 0028 | - | F | RAT type retrieval | 16.3.0 |
| 2020-12 | CT#90 | CP-203053 | 0029 | - | F | Resource ImsServerNameAndCapabilities | 16.3.0 |
| 2020-12 | CT#90 | CP-203053 | 0030 | - | F | S-CSCF Restoration | 16.3.0 |
| 2020-12 | CT#90 | CP-203053 | 0031 | 1 | F | S-CSCF Selection | 16.3.0 |
| 2020-12 | CT#90 | CP-203053 | 0032 | - | F | S-CSCF name | 16.3.0 |
| 2020-12 | CT#90 | CP-203053 | 0033 | - | F | Text alignment with yaml definitions | 16.3.0 |
| 2020-12 | CT#90 | CP-203053 | 0034 | - | F | Type name alignments | 16.3.0 |
| 2020-12 | CT#90 | CP-203053 | 0035 | - | F | UE-Reachability subscription | 16.3.0 |
| 2020-12 | CT#90 | CP-203066 | 0039 | 1 | F | Private identities | 16.3.0 |
| 2020-12 | CT#90 | CP-203067 | 0040 | 1 | F | Authorization Response | 16.3.0 |
| 2020-12 | CT#90 | CP-203036 | 0041 | - | F | 29.562 Rel-16 API version and External doc update | 16.3.0 |
| 2021-03 | CT#91 | CP-210038 | 0044 | - | F | Authorization Response | 16.4.0 |
| 2021-03 | CT#91 | CP-210054 | 0047 | - | F | 29.562 Rel-16 API version and External doc update | 16.4.0 |
| 2021-03 | CT#91 | CP-210023 | 0045 | - | F | Title and Scope of TS 29.562 | 17.0.0 |
| 2021-06 | CT#92 | CP-211060 | 0049 | 1 | A | Alias Group Id | 17.1.0 |
| 2021-06 | CT#92 | CP-211060 | 0051 | 2 | A | IMSI retrieval | 17.1.0 |
| 2021-06 | CT#92 | CP-211060 | 0054 | 1 | A | Charging Information retrieval | 17.1.0 |
| 2021-06 | CT#92 | CP-211060 | 0056 | - | A | Shared IMPU | 17.1.0 |
| 2021-06 | CT#92 | CP-211028 | 0057 | - | F | OpenAPI Reference | 17.1.0 |
| 2021-06 | CT#92 | CP-211060 | 0059 | - | A | S-CSCF reselection in eIMS | 17.1.0 |
| 2021-06 | CT#92 | CP-211060 | 0061 | - | A | CSRN retrieval | 17.1.0 |
| 2021-06 | CT#92 | CP-211059 | 0063 | - | A | Redirect Responses | 17.1.0 |
| 2021-06 | CT#92 | CP-211060 | 0064 | 1 | A | Correction to schema of requested-nodes | 17.1.0 |
| 2021-06 | CT#92 | CP-211060 | 0065 | - | A | DSAI resource query parameters | 17.1.0 |
| 2021-06 | CT#92 | CP-211060 | 0066 | - | A | Initial Filter Criteria resource query parameter | 17.1.0 |
| 2021-06 | CT#92 | CP-211170 | 0067 | 1 | A | Correction to IMS User Profile | 17.1.0 |
| 2021-06 | CT#92 | CP-211050 | 0073 | - | F | 29.562 Rel-17 API version and External doc update | 17.1.0 |
| 2021-09 | CT#93 | CP-212066 | 0053 | 1 | A | Query parameter name alignment | 17.2.0 |
| 2021-09 | CT#93 | CP-212066 | 0078 | - | A | imsUeId pattern | 17.2.0 |
| 2021-09 | CT#93 | CP-212026 | 0079 | 1 | F | Patch alignment | 17.2.0 |
| 2021-09 | CT#93 | CP-212183 | 0080 | 2 | F | Retrieval of multiple Repository Data | 17.2.0 |
| 2021-09 | CT#93 | CP-212066 | 0082 | - | A | RequestedNodes data type definition | 17.2.0 |
| 2021-09 | CT#93 | CP-212066 | 0084 | 1 | F | S-CSCF Restoration Information Retrieval | 17.2.0 |
| 2021-09 | CT#93 | CP-212059 | 0086 | - | F | 29.562 Rel-17 API version and External doc update | 17.2.0 |
| 2021-09 | CT#93 | CP-212066 | 0088 | 1 | A | Correction to shared IMPU | 17.2.0 |
| 2021-12 | CT#94 | CP-213091 | 0090 | 3 | B | HSS GBA SBI Services Definition | 17.3.0 |
| 2021-12 | CT#94 | CP-213132 | 0092 | - | A | Monitored resource URI | 17.3.0 |
| 2021-12 | CT#94 | CP-213132 | 0094 | - | A | Supported Feature | 17.3.0 |
| 2021-12 | CT#94 | CP-213091 | 0095 | 1 | B | HSS GBA SDM API Definition | 17.3.0 |
| 2021-12 | CT#94 | CP-213091 | 0096 | 1 | B | HSS GBA UEAU API Definition | 17.3.0 |
| 2021-12 | CT#94 | CP-213121 | 0097 | - | F | 29.562 Rel-17 API version and External doc update | 17.3.0 |
| 2022-03 | CT#95 | CP-220023 | 0099 | - | F | Wrong Oauth2 scope name | 17.4.0 |
| 2022-03 | CT#95 | CP-220029 | 0100 | 1 | F | User Identifier Pattern | 17.4.0 |
| 2022-03 | CT#95 | CP-220066 | 0101 | - | F | 29.562 Rel-17 API version and External doc update | 17.4.0 |
| 2022-06 | CT#96 | CP-221094 | 0103 | 1 | F | Missing Description Fields and Additional Corrections | 17.5.0 |
| 2022-06 | CT#96 | CP-221058 | 0104 | - | F | Corrections to the Nhss_imsUEAU API | 17.5.0 |
| 2022-06 | CT#96 | CP-221058 | 0105 | 1 | F | Corrections to the Nhss_imsUECM API | 17.5.0 |
| 2022-06 | CT#96 | CP-221028 | 0106 | - | F | RestorationInfoDelete clarification | 17.5.0 |
| 2022-06 | CT#96 | CP-221029 | 0107 | 1 | F | Corrections Nhss_imsSDM API | 17.5.0 |
| 2022-06 | CT#96 | CP-221051 | 0108 | - | F | 29.562 Rel-17 API version and External doc update | 17.5.0 |
| 2022-09 | CT#97 | CP-222027 | 0109 | 1 | B | Add SBI support Indication for IP-SM-GW in HSS | 17.6.0 |
| 2022-09 | CT#97 | CP-222060 | 0111 | - | A | Obvious yaml error correction | 17.6.0 |
| 2022-09 | CT#97 | CP-222026 | 0112 | - | F | Charging Info retrieval | 17.6.0 |
| 2022-09 | CT#97 | CP-222026 | 0113 | - | F | Core Network Service Authorization | 17.6.0 |
| 2022-09 | CT#97 | CP-222026 | 0114 | - | F | IMS User Profile Retrieval | 17.6.0 |
| 2022-09 | CT#97 | CP-222026 | 0116 | - | F | ImsProfileData | 17.6.0 |
| 2022-09 | CT#97 | CP-222026 | 0117 | - | F | IMS ServicePriority Level | 17.6.0 |
| 2022-09 | CT#97 | CP-222026 | 0118 | - | F | 403 Forbidden | 17.6.0 |
| 2022-09 | CT#97 | CP-222060 | 0120 | - | A | CsLocation | 17.6.0 |
| 2022-09 | CT#97 | CP-222060 | 0122 | - | A | CallReferenceInfo | 17.6.0 |
| 2022-09 | CT#97 | CP-222058 | 0123 | - | F | 29.562 Rel-17 API version and External doc update | 17.6.0 |
| 2022-12 | CT#98 | CP-223028 | 0125 | 1 | F | Missing Mandatory Status Codes in OpenAPI | 18.0.0 |
| 2022-12 | CT#98 | CP-223033 | 0127 | - | F | 29.562 Rel-18 API version and External doc update | 18.0.0 |
| 2023-06 | CT#100 | CP-231027 | 0128 | 4 | F | Location header and missing Redirection clause | 18.1.0 |
| 2023-09 | CT#101 | CP-232045 | 0130 | - | B | Add DCSF as service consumer | 18.2.0 |
| 2023-09 | CT#101 | CP-232063 | 0132 | - | A | Add Location description for 201 response | 18.2.0 |
| 2023-09 | CT#101 | CP-232063 | 0134 | 1 | A | Update OpenAPI for Scscf Registration and Scscf Restoration Information Resource URI | 18.2.0 |
| 2023-09 | CT#101 | CP-232060 | 0135 | - | F | 29.562 Rel-18 API version and External doc update | 18.2.0 |
| 2023-12 | CT#102 | CP-233029 | 0137 | 1 | B | HTTP RFCs obsoleted by IETF RFC 9110, 9111 and 9113 | 18.3.0 |
| 2023-12 | CT#102 | CP-233029 | 0138 | 1 | B | ProblemDetails RFC 7807 obsoleted by 9457 | 18.3.0 |
| 2023-12 | CT#102 | CP-233048 | 0139 | 1 | B | Update the procedure to support DCSF get and update the repository data | 18.3.0 |
| 2024-03 | CT#103 | CP-240073 | 0142 | - | A | Correction on SmsRegistrationInfo data type | 18.4.0 |
| 2024-03 | CT#103 | CP-240058 | 0144 | - | A | IP SM GW Registration | 18.4.0 |
| 2024-03 | CT#103 | CP-240075 | 0147 | - | A | Remove the unused data type | 18.4.0 |
| 2024-03 | CT#103 | CP-240056 | 0148 | - | F | 29.562 Rel-18 API version and External doc update | 18.4.0 |
| 2024-06 | CT#104 | CP-241028 | 0151 | 1 | F | Callbacks | 18.5.0 |
| 2024-06 | CT#104 | CP-241052 | 0152 | - | F | 29.562 Rel-18 API version and External doc update | 18.5.0 |
| 2024-12 | CT#106 | CP-243022 | 0154 | 1 | F | Reference correction | 18.6.0 |
| 2024-12 | CT#106 | CP-243045 | 0155 | 1 | B | Support of IMS subscribe and Notify framework in HSS | 19.0.0 |
| 2024-12 | CT#106 | CP-243055 | 0157 | 2 | B | 29.562 MPS for IMS Messaging Indication on Nhss | 19.0.0 |
| 2024-12 | CT#106 | CP-243247 | 0158 | 3 | B | Support of IMS AS registration to HSS | 19.0.0 |
| 2024-12 | CT#106 | CP-243069 | 0161 | - | F | API version and External doc update | 19.0.0 |
| 2025-03 | CT#107 | CP-250032 | 0162 | - | F | Query parameter correction | 19.1.0 |
| 2025-03 | CT#107 | CP-250047 | 0163 | 2 | B | Add Nhss_imsEE service in HSS | 19.1.0 |
| 2025-03 | CT#107 | CP-250047 | 0164 | 2 | B | Add Nhss_imsUECM_AsInfoGet service operation | 19.1.0 |
| 2025-03 | CT#107 | CP-250047 | 0165 | 1 | B | Update the Individual Imsas Registration Resource | 19.1.0 |
| 2025-03 | CT#107 | CP-250047 | 0169 | 4 | B | Retrieval of IMS AS registration info from HSS | 19.1.0 |
| 2025-03 | CT#107 | CP-250137 | 0174 | - | F | API version and External doc update | 19.1.0 |
| 2025-06 | CT#108 | CP-251068 | 0175 | 1 | F | Update Nhss_imsEE Service | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0176 | - | F | IMS AS registration of the same IMS AS type | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0177 | 1 | B | Immediate report in Nhss_imsEE subscribe response | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0179 | 2 | B | AF Identifier in ImsEeSubscription | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0180 | 2 | B | Add Nhss_ImsEE OpenAPI | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0181 | - | F | Correction on application error of Nhss_ImsUECM service | 19.2.0 |
| 2025-06 | CT#108 | CP-251060 | 0185 | 1 | F | 29.562 Correction for MPS attribute | 19.2.0 |
| 2025-06 | CT#108 | CP-251079 | 0186 | - | F | API version and External doc update | 19.2.0 |
| 2025-09 | CT#109 | CP-252050 | 0187 | - | F | Incorrect "imsUeId" pattern in Nhss_imsEE service | 19.3.0 |
| 2025-09 | CT#109 | CP-252050 | 0189 | 1 | F | Add 403 Forbidden in data structure of IMS AS registration in Nimsas_ImsUECM | 19.3.0 |
| 2025-09 | CT#109 | CP-252176 | 0190 | - | F | API version and External doc update | 19.3.0 |
| 2025-12 | CT#110 | C4-254279 | 0191 | 1 | B | Update the name of ImsasRegistrationInfoGet service operation | 19.4.0 |
| 2025-12 | CT#110 | C4-255266 | 0192 | 3 | B | Cancel procedure for subscriber specific IMS Events | 19.4.0 |
| 2025-12 | CT#110 | C4-255360 | 0196 | 1 | A | S-CSCF Deregistration Notification | 19.4.0 |
| 2025-12 | CT#110 | C4-255295 | 0197 | 1 | B | Registration Timer Extension | 19.4.0 |
| 2025-12 | CT#110 | C4-255272 | 0198 | 1 | B | HSS initiated IMS AS De-registration procedure | 19.4.0 |
| 2025-12 | CT#110 | C4-255273 | 0199 | 1 | B | IMS AS registration with re-assignment for new sessions | 19.4.0 |
| 2025-12 | CT#110 | C4-255231 | 0200 | - | F | Correction on S-CSCF registration procedure in Nhss_imsUECM | 19.4.0 |
| 2025-12 | CT#110 | C4-255486 | 0204 | - | F | API version and External doc update | 19.4.0 |