3GPP TSG RAN meeting #110										    RP-253685

Baltimore, USA, December 8-11, 2025

## Status Report to TSG

Agenda item:			10.6.2

| WI / SI Name | Introduction of IoT-NTN TDD mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | IoT_NTN_TDD |  |  |  |  |
| Unique ID | 1050123 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-252935 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: N/A | Core part: Overall: 100% RAN1: 100% RAN2: 100% RAN4: 100% | Performance Part: Overall:67% RAN4: 67% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Andjela Ilic-Savoia |
|  | Company | Iridium Satellite LLC |
|  | Email | andjela.savoia@iridium.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

2.1.1 	Agreements during RAN1#122bis (Maintenance)

RAN1 #122bis UE Features for IOT NTN TDD

Agreement

- Confirm “TDD only” in “Need of FDD/TDD differentiation” column in FG 2-1.

- Confirm “N/A” in “Capability interpretation for mixture of FDD/TDD” column in FG 2-1.

#### 2.1.2	Agreements during RAN1#123 (Maintenance)

RAN1 #123 List of Agreements and Conclusions

Conclusion

RAN1 does not specify further enhancements for NPUSCH format 1 scheduling delays in Rel-19 IoT NTN TDD mode.

Conclusion

RAN1 does not specify further enhancements for NRS availability in non-anchor carriers in Rel-19 IoT NTN TDD mode.

#### 2.1.3	Remaining open issues

  - Awaiting LS Reply from RAN4 confirming RAN1 agreement on segmented compensation.

    - (RAN4 sent the LS Reply at the end of RAN4 #117: R4-2523100)

## 2.2	RAN2

#### 2.2.1	Agreements during RAN2#131bis

1.	IoT NTN TDD mode capability is conditionally mandatory (as in the draft CR) if the UE supports the relevant TDD band

2.	No need to introduce a new list of neighbour cells operating in TDD mode in Rel-19

3.	RAN2 assume that for IoT NTN TDD mode, serving cell and neighbor cells are not always synchronized.

4.	There is no benefit in introducing a new idle mode DRX cycle length for IoT NTN TDD mode.

5.	There is no specification impact for RAN2 (36.304) related to paging scheduling.

6.	There is no specification change required in the existing SIB1 scheduling mechanism.

7.	RAN2 assumes the network will take into account the invalid subframes when setting the explicit EpochTime

8.	RAN2 assumes that OCC defined for IoT NTN Rel-19 can be supported for IoT NTN TDD mode but ask RAN1 for confirmation

9.	We aim at introducing the needed changes (if any) to support CB-Msg3 and CB-Msg4 for IoT NTN TDD mode during the ASN.1 review, including at least

Adding configurations that are aligned with the 90 ms periodicity for npusch-Periodicity and windowPeriodicity-NB

(FFS other needed enhancements based on the progress of the CB-Msg3-EDT discussion this week)

10.	IoT NTN PWS Rel-19 can be supported in IoT NTN TDD (no spec impact)

11.	TDD radio frame offset between serving cell and neighbor cell(s) can be provided via SIB33 to the UE for neighbor cell measurement.

#### 2.2.2	Agreements during RAN2#131bis Post-meeting email discussion

- CRs endorsed:

  - 36.331 CR: Huawei (R2-2507788)

#### 2.2.3	Agreements during RAN2#132

Agreements:

1. Orthogonal Cover Codes are not supported for IoT NTN TDD mode.

2. N022 is postponed until RAN4 confirms the RAN1 agreement on segment compensation

3. [H600] For CB-msg3-EDT we introduce configurations that are multiple of the 90 ms npusch-Periodicity

4. [H600] For CB-msg3-EDT if the subframe of the NPUSCH transmission is not a U-subframe, the NPUSCH transmission is postponed to the nearest U-subframe

5. RAN2 confirms that IoT NTN TDD follows FDD behaviour for the scheduling restriction on DL and UL transmissions.

6. RAN2 understands that it would be up to NW implementation whether and how to configure SPS for IoT NTN TDD (we don’t clarify anything further in the specs)

- CRs agreed:

  - 36.300 CR: Iridium (R2-2509206)

#### 2.2.4 	Remaining open issues

  - Awaiting LS Reply from RAN4 confirming RAN1 agreement on segmented compensation.

    - (RAN4 sent the LS Reply at the end of RAN4 #117: R4-2523100)

#### 2.2.5 	Agreements during RAN2#132 Post-meeting email discussion

- CRs agreed:

  - 36.331 CR: Huawei (R2-2509368)

  - 36.321 CR: Toyota ITC (R2-2509372)

## 2.3	RAN3

## 2.4	RAN4

2.4.1 	Agreements during RAN4#116bis

## 2.4.1.1	SAN Conformance Requirements

Way forward:

-	Issue 2-1-1: Selected impacted clauses under TS 36.181

Agreement: As starting point, the following clauses are impacted/not impacted (tables i through vi below)

- i/ General requirements for IoT-NTN TDD band 249

| Requirements in TS 36.181 | Expected impact |
| --- | --- |
| 4 General test conditions and declarations |  |
| 4.1 Measurement uncertainties and test requirements | No specification impact. |
| 4.2 Requirement reference points | No specification impact. |
| 4.3 Satellite Access Node classes | No specification impact. |
| 4.4 Regional requirements | Need to add band 249 in Table 4.4-1. |
| 4.5 SAN configurations | No specification impact. |
| 4.6 Manufacturer declarations | No specification impact. |
| 4.7 Test configurations | The carrier bandwidth in section 4.7.3.1 needs to be updated. |
| 4.8 Applicability of requirements | Need to revise the applicability Transmit ON/OFF power in Table 4.8.2-1. |
| 4.9 RF channels and test models | No specification impact. |
| 4.10 Requirements for contiguous and non-contiguous spectrum | No specification impact. |
| 4.11 Requirements for SAN capable of multi-band operation | No specification impact. |
| 4.12 Reference coordinate system | No specification impact. |
| 4.13 Format and interpretation of tests | No specification impact. |

ii/ Conducted Tx requirements for IoT-NTN TDD band 249

| Requirements in TS 36.181 | Expected impact |
| --- | --- |
| 6 Conducted transmitter characteristics |  |
| 6.1 General | No specification impact. |
| 6.2 Satellite Access Node output power | No specification impact. |
| 6.3 Output power dynamics |  |
| 6.3.1 General | No specification impact. |
| 6.3.2 RE power control dynamic range | No specification impact. |
| 6.3.3 Total power dynamic range | No specification impact. |
| 6.4 Transmit ON/OFF power | Need to update transmit ON/OFF power requirement for NB-IoT TDD SAN. |
| 6.5 Transmitted signal quality |  |
| 6.5.1 General | No specification impact. |
| 6.5.2 Frequency error | No specification impact. |
| 6.5.3 Modulation quality | No specification impact. |
| 6.5.4 Time alignment error | No specification impact. |
| 6.5.5 DL RS power | No specification impact. |
| 6.6 Unwanted emissions |  |
| 6.6.1 General | No specification impact. |
| 6.6.2 Occupied bandwidth | No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio (ACLR) | No specification impact. |
| 6.6.4 Out-of-band emissions | No specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | No specification impact. |

iii/ Conducted Rx requirements for NTN IoT-NTN TDD band 249

| Requirements in TS 36.181 | Expected impact |
| --- | --- |
| 7 Conducted receiver characteristics |  |
| 7.1 General | No specification impact. |
| 7.2 Reference sensitivity level | Need to add the following REFSENS requirement for IoT-NTN TDD band. Table 7.2.5-5: Reference sensitivity levels of SAN supporting standalone NB-IoT operation and NB-IoT operation in NTN NR in-band in TDD operating bands (LEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, PREFSENS (dBm). 200; 15; FRC A14-3 in Annex A.14; -126.1. |
| 7.3 Dynamic range | Need to add the following dynamic range requirement for IoT-NTN TDD band. Table 7.3.5-4: Dynamic range of SAN supporting standalone NB-IoT operation in TDD bands (LEO class payload) SAN channel bandwidth (kHz); Reference measurement channel; Wanted signal mean power (dBm); Interfering signal mean power (dBm) / BWChannel; Type of interfering signal. 200; FRC A15-1 in Annex A.15; -105.1; -101.7; AWGN. |
| 7.4 In-band selectivity and blocking |  |
| 7.4.1 Adjacent channel selectivity (ACS) | Need to add the following ACS requirement for IoT-NTN TDD band. Table 7.4.1.5-3: ACS requirement of SAN supporting standalone NB-IoT operation for TDD bands SAN channel bandwidth of the lowest/highest carrier received (kHz); Wanted signal mean power (dBm); Interfering signal mean power (dBm); Interfering signal centre frequency offset to the lower/upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz); Type of interfering signal. 200; PREFSENS + 19.5dB (Note); LEO SAN class: -85.5; ±100; 180 kHz NB-IoT signal. Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-5.. |
| 7.4.2 In-band blocking | No specification impact. |
| 7.5 Out-of-band blocking | No specification impact. |
| 7.6 Receiver spurious emissions | No specification impact. |
| 7.7 Receiver intermodulation | No specification impact. |
| 7.8 In-channel selectivity | No specification impact. |

iv/ Radiated Tx requirements for NTN IoT-NTN TDD band 249

| Requirements in TS 36.181 | Expected impact |
| --- | --- |
| 9 Radiated transmitter characteristics |  |
| 9.1 General | No specification impact. |
| 9.2 Radiated transmit power | No specification impact. |
| 9.3 OTA SAN output power | No specification impact. |
| 9.4 OTA output power dynamics | No specification impact. |
| 9.5 OTA transmit ON/OFF power | N/A, according to WF from RAN4#114bis |
| 9.6 OTA transmitted signal quality | No specification impact. |
| 9.7 OTA unwanted emissions | No specification impact. |
| 9.8 OTA transmitter intermodulation | No specification impact. |

v/ Radiated Rx requirements for NTN IoT-NTN TDD band 249

| Requirements in TS 36.181 | Expected impact |
| --- | --- |
| 10 Radiated receiver characteristics |  |
| 10.1 General | No specification impact. |
| 10.2 OTA sensitivity | Need to add the following OTA sensitivity requirement for IoT-NTN TDD band. Table10.2.5-3: SAN LEO class EIS levels for standalone NB-IoT operation and NB-IoT operation in NTN NR in-band SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel (NOTE); OTA sensitivity level, EIS (dBm). 200; 15; A14-3 in Annex A.14; Declared minimum EIS + 1.6. NOTE: EIS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth.. |
| 10.3 OTA reference sensitivity level | Need to add the following OTA reference sensitivity level requirement for IoT-NTN TDD band. Table10.3.5-5: Reference sensitivity levels of SAN supporting standalone NB-IoT operation and NB-IoT operation in NTN NR in-band for TDD frequency bands (LEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, EISREFSENS (dBm). 200; 15; FRC A14-3 in Annex A.14; -125.8 - ΔOTAREFSENS. |
| 10.4 OTA dynamic range | Need to add the following dynamic range requirement for IoT-NTN TDD band. Table 10.4.5.2-4: Dynamic range of SAN supporting standalone NB-IoT operation in TDD frequency bands (LEO class payload) SAN channel bandwidth (kHz); Reference measurement channel; Wanted signal mean power (dBm); Interfering signal mean power (dBm) / BWChannel; Type of interfering signal. 200; FRC A15-1 in annex A.15; -105.1 - ΔOTAREFSENS; -101.7 - ΔOTAREFSENS; AWGN. |
| 10.5 OTA in-band selectivity and blocking | Need to add the following ACS requirement for IoT-NTN TDD band. Table 10.5.1.5-3: ACS requirement of SAN supporting standalone NB-IoT operation for TDD frequency bands SAN channel bandwidth of the lowest/highest carrier received (kHz); Wanted signal mean power (dBm); Interfering signal mean power (dBm); Interfering signal centre frequency offset to the lower/upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz); Type of interfering signal. 200; PREFSENS + 19.5dB - ΔOTAREFSENS(Note); LEO SAN class: -85.5- ΔOTAREFSENS; ±100; 180 kHz NB-IoT signal. Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-5.. |
| 10.5.1 OTA adjacent channel selectivity | No specification impact. |
| 10.5.2 OTA in-band blocking | No specification impact. |
| 10.6 OTA out-of-band blocking | No specification impact. |
| 10.7 OTA receiver spurious emissions | No specification impact. |
| 10.8 OTA receiver intermodulation | No specification impact. |
| 10.9 OTA in-channel selectivity | No specification impact. |

vi/ Annex

| Requirements in TS 36.181 | Expected impact |
| --- | --- |
| Annex A (normative): Reference measurement channel | Need to add new FRC A14-3 in Table A.14-1 for IoT-NTN TDD band. Need to add new FRC A15-3 in Table A.15-1 for IoT-NTN TDD band. |
| Annex B (informative): Environmental requirements for SAN equipment | No specification impact. |
| Annex C (informative): Test tolerances and derivation of test requirements | No specification impact. |
| Annex D (informative): Measurement system set-up | No specification impact. |
| Annex E (normative): Characteristics of the interfering signals | No specification impact. |
| Annex F (normative): Propagation conditions | No specification impact. |
| Annex G (normative): In-channel Tx test | Need to revise averaging length for the transmission time to 8ms in section G.7.3 Averaged EVM. |

Issue 2-1-2: Draft CR for endorsement

Draft CR R4-2514411  Revised to R4-2515064 according to the above Issue 2-1-1 proposal that was agreed and already existing material from R4-2514411.

Agreement: Endorse Draft CR R4-2515064.

## 2.4.1.2	UE Requirements

- None

## 2.4.1.3	Demodulation Requirements

## Sub-topic 3-1: SAN Demodulation – NPUSCH Format 1

#### Channel Model

Agreement

NTN-TDLA100-1

NTN-TDLC5-1

#### Antenna Configuration

Agreement

1T1R

#### Repetition Number

Agreement

4

#### Sub Carrier Spacing

Agreement

15 kHz

#### Allocated Subcarriers

Agreement

12

#### TDD Pattern

Agreement

D/U = 8

## Sub-topic 3-2: SAN Demodulation – NPUSCH Format 2

#### Channel Model

Agreement

NTN-TDLA100-1

#### Antenna Configuration

Agreement

1T1R

#### Repetition Number

Options:

Option 1: 8 with Tx Duration of 16

Option 2: 4 with Tx Duration of 2 (Total Duration of 8 ms)

#### Sub Carrier Spacing

Agreement

15 kHz

#### Allocated Subcarriers

Agreement

1

## Sub-topic 3-3: SAN Demodulation – NPRACH

#### Channel Model

Options:

Option 1: NTN-TDLA100-1

Option 2: NTN-TDLC5-1

#### Antenna Configuration

Agreement

1T1R

#### Repetition Number and Tx Duration

Options:

- For Format 0:

  - Option 1: 4 with 2 Tx duration (Total duration 8 ms)

  - Option 2: 8 with 8 Tx duration

  - Option 3: 16 with 16 Tx duration

- For Format 1:

  - Option 1: 2 with 4 Tx duration (Total duration 8 ms)

  - Option 2: 8 with 8 Tx duration

#### TDD Pattern

Agreement

D/U = 8

## Sub-topic 3-4: UE Demodulation - NPDSCH

#### Channel Model

Agreement

NTN-TDLC5-200

NTN-TDLA100-10

#### Carriers Type

Agreement

Anchor

Non Anchor

#### Antenna Configuration

Agreement

1T1R

#### Repetition Number

Agreement

4

#### TDD Pattern

Agreement

D/U = 8

2.4.1.4	Remaining open issues (RAN4 UE/SAN RF)

  - None.

2.4.2 	Agreements during RAN4#116bis RRM

#### Issue 4-1-1: Introduction of new band group

Agreement

- Band groups for NB-IoT for satellite access in Table 3.5.1A-1 shall be updated to include NTN TDD band 249 as follows:

- Table 3.5.1A-1: Band groups for NB-IoT for satellite access

| Group | E-UTRA FDD |  |
| --- | --- | --- |
|  | Band group notation | Operating bands |
| A | NFDD_SAB_A |  |
| B | NFDD_SAB_B |  |
| C | NFDD_SAB_C |  |
| D | NFDD_SAB_D |  |
| E | NFDD_SAB_E |  |
| F | NFDD_SAB_F |  |
| G | NFDD_SAB_G | 253, 254, 255, 256 |
| H | NFDD_SAB_H |  |
| I | NFDD_SAB_I |  |
| J | NFDD_SAB_J |  |
| K | NFDD_SAB_K |  |
| L | NFDD_SAB_L |  |
| M | NFDD_SAB_M |  |
| N | NFDD_SAB_N |  |
| X | NTDD_SAB_N | 249 |

- Update Annex B.3.25A to include IoT NTN TDD band 249 and if required with different minimum NRSRP value as follows:

- Table B.3.25A-1: NB-IoT intra-frequency absolute NRSRP and NRSRQ Accuracy Requirements

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum NRSRP |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_SAB _G | -139.8 |
|  | NTDD_SAB_X | FFS |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |

#### Issue 4-1-2: Measurement accuracy – General

Tentative Agreement in AH

- RAN4 to define accuracy requirements for NRSRP, NRSRQ and downlink channel quality report.

RAN4 to consider adding section 9.1.22X (based on the existing 9.1.22A) with updated side conditions and band groups.

#### Issue 4-1-3: Measurement accuracy – Detail

Agreement

- RAN4 to consider introducing intra frequency absolute NRSRP accuracy requirement for IoT NTN TDD as follows:

Table 9.1.22X.1-1: NRSRP Intra frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| 6 | 9 | -1 dB | NTDD_SAB_X | -122.9 | N/A | -70 |
| 8 | 11 | -1 dB | NTDD_SAB_X | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

- RAN4 to consider introducing intra frequency absolute NRSRQ accuracy requirement for IoT NTN TDD as follows:

- Table 9.1.22X.2-1: NRSRQ Intra frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 3 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel |
| 5.2 | 8.2 | -1 dB-3 dB | NTDD_SAB_X | -122.9 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |

- RAN4 to consider adding inter frequency absolute NRSRP accuracy requirement for IoT NTN TDD as follows:

Table 9.1.22X.3-1: NRSRP Inter frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| 6 | 9 | -1 dB -6 dB | NTDD_SAB_X | -122.9 | N/A | -70 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

- RAN4 to consider inter frequency inter frequency absolute NRSRPQ accuracy requirement for IoT NTN TDD as follows:

Table 9.1.22X.4-1: NRSRQ Inter frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 3 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel |
| 5.2 | 8.2 | -1 dB-3 dB | NTDD_SAB_X | -122.9 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |

- RAN4 to consider introducing downlink channel quality reporting accuracy requirement for IoT NTN TDD as follows:

Table 9.1.22X.8-1: Downlink channel quality reporting accuracy for UE Category NB1

| NPDCCH Repetition | Pm-Dsg (%) | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ês/Iot | Io NOTE 1 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups NOTE 2 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -1 dB-6 dB | NTDD_SAB_X | -122.9 | N/A | -70 |
| R/4 NOTE 1 | >1 | -1 dB -6 dB | NTDD_SAB_X | -122.9 | N/A | -70 |
| NOTE 1: R is the reported NPDCCH repetition level that UE has reported in CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### Issue 4-1-4: Measurement report mapping

Tentative Agreement in AH

- The existing NRSRP, NRSRQ, and msg3-based measurement report mapping can be re-used for IoT NTN TDD mode without modification.

#### Issue 4-1-5: PHR requirement

Tentative Agreement in AH

- UE power class 6 should be excluded from the power headroom performance requirements for IoT NTN TDD mode.

- The power headroom performance requirements specified for UE category NB1 for satellite access in section 9.1.23A shall also apply to IoT NTN TDD mode.

#### Issue 4-2-1: Updated Reference Channel

Tentative Agreement in AH

- For UE category NB1 operating in IoT NTN TDD mode,

  - Update the NPDSCH reference channel based on Table A.3.1.5.3.1 as follows:

Table X.3.1.5.3-1: NPDSCH Reference Channel for UE category NB1 for standalone operation

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDSCH Reference channel | - | R.18 HD-FDD | R.19 HD-FDD |
| Channel bandwidth | KHz | 200 | 200 |
| Number of transmitter antennas | - | 1 | 2 |
| Allocated subframes per Radio Frame | subframes | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits | 72 Note 2 | 72 Note 2 |
| For Sub-Frame 4, 9 | Bits | Note 3 | Note 3 |
| For Sub-Frame 0, 5 | Bits | 0 | 0 |
| Number of Code Blocks per Sub-Frame |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  | 1 Note 4 | 1 Note 4 |
| For Sub-Frame 4, 9 |  | Note 5 | Note 5 |
| For Sub-Frame 0, 5 |  | 0 | 0 |
| Maximum number of repetitions | - | Note 6 | Note 6 |
| Cell ID | - | Note 7 | Note 7 |
| Note 1: Shall depend upon the NPDSCH scheduling Note 2: Only apply for subframes scheduled with NPDSCH. Note 3: 72 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. Note 4: only apply for subframes scheduled with NPDSCH.. Note 5: 1 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. Note 6: Maximum number of repetitions shall depend upon the test configuration. Note 7: Cell ID shall depend upon the test configuration. |  |  |  |

  - The existing NPDCCH reference channel parameters specified for NB-IoT NTN FDD for standalone mode can also be applicable for IoT NTN TDD mode

  - Update the OCNG parameters based on Table A.3.2.3.3.1 as follows:

Table X.3.2.3.3-1: NOP.3 FDD: OCNG FDD Pattern 3

| Allocation | Relative power level [dB] |  |  | NPDCCH and NPDSCH Data |
| --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |
|  | 0, 4 | 5, 9 | 1-3, 6-8 |  |
| 0 | 0 (Note 2) | 0 (Note 2) | 0 | Note 1 |
| Note 1: This physical resource block is assigned to a virtual UE for transmission of NPDCCH or NPDSCH; the data transmitted over the NOCNG NPDCCH or NPDSCH shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of NPDCCH and NPDSCH. If two transmit antennas with NBRS are used in the test, the NPDCCH and NPDSCH parts of NOCNG shall be transmitted to the virtual users by both transmit antennas with NBRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the NPDCCH and NPDSCH parts of NOCNG is equal between both the transmit antennas with NBRS used in the test. The antenna transmission modes are specified in clause 7.1 in TS 36.213. Note 2: Value of is applicable to PRBs not used for transmission of NPSS, NSSS and NPBCH in anchor cell (= 0). Note 3: SI transmissions, and NPDCCH and NPDSCH for the UE under test, have precedence over NOCNG. |  |  |  |  |

  - Update the NPRACH reference configurations based on Table A.3.18-1 as follows:

Table X.3.18-1: NPRACH.R-1: HD-FDD Reference NPRACH Configuration

| Field | Value |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH coverage level |  |  |  |  |  |
| rsrp-ThresholdsPrach | {rsrp1, rsrp2} |  |  | The values of NPRACH RSRP thresholds for will be set according the requirement of individual test cases |  |
| nprach-CP-Length | us66dot7 |  |  | NPRACH format 0 |  |
| Parameters per NPRACH coverage Level |  |  |  |  |  |
| CE Level | Level 0 | Level 1 | Level 2 |  | Valid values as defined in TS 36.331 [2] |  |
| nprach-Periodicity | ms640ms40 | ms640ms40 | FFS ms40 |  | {ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560} |  |
| nprach-StartTime | ms8 | ms8 | ms8 |  | {ms8, ms16, ms32, ms64, ms128, ms256, ms512, ms1024} |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  | {n0, n12, n24, n36, n2, n18, n34} |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  | {n12, n24, n36, n48} |  |
| nprach-SubcarrierMSG3-RangeStart | {one} | {one} | {one} |  | {zero, oneThird, twoThird, one} |  |
| maxNumPreambleAttemptCE | n3 | n5 | n7 |  | {n3, n4, n5, n6, n7, n8, n10} |  |
| numRepetitionsPerPreambleAttempt | n1 | n8 | n16n32 |  | {n1, n2, n4, n8, n16, n32, n64, n128} |  |
| npdcch-NumRepetitions-RA | r1 | r8 | r16r32 |  | {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048} |  |
| npdcch-StartSF-CSS-RA | v64v8 | v32v2 | V64v2 |  | {v1dot5, v2, v4, v8, v16, v32, v48, v64} |  |
| npdcch-Offset-RA | zero | zero | zero |  | {zero, oneEighth, oneFourth, threeEighth} |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  | {n8, n10, n11, n12, n20, n22, n23, n24, n32, n34, n35, n36, n40, n44, n48} |  |
| Note 1: See Clause 6.7.3.2 in TS 36.331 [2] for further information on the parameters in this table. |  |  |  |  |  |

#### Issue 4-2-2: Test case list

Agreement

- Following list of test cases from the existing NB-IoT NTN FDD mode are applicable for IoT NTN TDD:

| Test case | Description |
| --- | --- |
| Re-selection | A.13.1.1.1 HD – FDD and IoT NTN TDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage A.13.1.1.3 HD – FDD and IoT NTN TDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX |
|  | A.13.1.1.4 HD – FDD and IoT NTN TDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX |
| RRC Re-establishment | A.13.3.1.1 HD-FDD and IoT NTN TDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage |
| Random Access | A.13.3.2.1 Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage |
| UE transmit timing | A.13.4.1.1 E-UTRAN HD-FDD and IoT NTN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access |
| Timing Advance | A.13.4.2.1 HD-FDD and IoT NTN TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access |
| Radio Link Monitoring | - A.13.4.3.1 HD-FDD and IoT NTN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage - A.13.4.3.4 HD-FDD and IoT NTN TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Normal Coverage - A.13.4.3.5 HD-FDD and IoT NTN TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage - A.13.4.3.7 HD-FDD and IoT NTN TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage |
| Measurements | A.13.5.1 HD-FDD and IoT NTN TDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access |
|  | A.13.5.2 HD-FDD and IoT NTN TDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access |
|  | - A.13.5.3 HD-FDD and IoT NTN TDD Intra-frequency location-based neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access |
| Downlink channel quality reporting accuracy | A.13.6.2.1 E-UTRAN HD-FDD and IoT NTN TDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage A.13.6.2.3 E-UTRAN HD-FDD and IoT NTN TDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage A.13.6.2.5 E-UTRAN HD-FDD and IoT NTN TDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage |

#### Issue 4-2-3: Test configuration - General

Agreement

- Only NGSO configuration is applicable.

- For existing NB-IoT NTN test cases, create a new test configuration with the new reference channel models and reuse the test description. The test requirements when the new configuration is used might be adapted to reflect the agreements of the core requirements.

#### Issue 4-2-4: Principle of testing TDD and FDD frame structure

WayForward

- Further discuss the principle of testing TDD and FDD frame structure.

  - Option 1: A UE capable of both TDD and FDD, the UE is required to pass either TDD or FDD per the DUT’s declaration.

  - Option 2: define a test case list to be tested for each configuration for the UE that supports both TDD and FDD frame structures

  - Other options are not precluded.

#### Issue 4-2-5: Test configuration for random access test

Agreement

- Update the NPRACH procedure test configuration parameters, based on Table A.13.3.2.1.1-2.

- FFS the suitable values.

- FFS the applicable CE level in this test case

Table X.13.3.2.1.1-3: NPRACH-Configuration parameters for HD-FDD contention based random access test for UE category NB1 Standalone mode in Normal Coverage

| Field | Value |  |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH resource |  |  |  |  |  |  |
| RSRP-ThresholdsNPRACH-InfoList | {39, 24} |  |  |  | Corresponding to {-101, -116} dBm as defined in Table 9.1.4-1 |  |
| nprach-CP-Length | us66dot7 |  |  |  |  |  |
| nrs-Power | -5 dBm/15 kHz |  |  |  | As defined in clause 6.7.3 in TS 36.331. |  |
| Backoff Parameter Index | 1 |  |  |  | As defined in table 7.2-2 in TS 36.321 |  |
| Configured UE transmitted power () | 23 dBm for power class 3, 20 dBm for power class 5 |  |  |  | As defined in clause 6.2B.4 in TS 36.102 |  |
| powerRampingStep | dB2 |  |  |  |  |  |
| preambleInitialReceivedTargetPower | dBm-112 |  |  |  |  |  |
| preambleTransMax-CE | n6 |  |  |  |  |  |
| Parameters per NPRACH Resource |  |  |  |  |  |  |
| NPRACH Resource | Level 0 | Level 1 | Level 2 |  |  |
| nprach-Periodicity | ms40 | ms240 | ms1280 |  |  |
| nprach-StartTime | ms8 | ms64 | ms512 |  |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart | zero | zero | zero |  |  |
| maxNumPreambleAttemptCE | n3 | n6 | n10 |  |  |
| numRepetitionsPerPreambleAttempt | n2 | n8 | n64 |  |  |
| npdcch-NumRepetitions-RA | r4 | r16 | r128 |  |  |
| npdcch-StartSF-CSS-RA | v1dot5 | v1dot5 | v1dot5 |  |  |
| npdcch-Offset-RA | zero | zero | Zero |  |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  |  |
| ra-ResponseWindowSize (per NPRACH Resource) | pp2 | pp2 | pp2 |  |  |
| mac-ContentionResolutionTimer (per NPRACH Resource) | pp8 | pp8 | pp8 |  |  |
| Note 1: See Clause 6.7.3 in TS 36.331 for further information on the parameters in this table. |  |  |  |  |  |  |

#### Issue 4-2-6: Test configuration for timing test

Tentative Agreement in AH

- Update the timing advance accuracy test configuration parameters, based on Table A.13.4.2.1.1-2.

- FFS the suitable values.

- FFS the applicable CE level in this test case

Table X.13.4.2.1.1-2: General Test Parameters for E-UTRAN Timing Advance Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |  |
| CP Length |  |  | Normal |  |
| Satellite information | Config 1 |  | SSC.1 |  |
|  | Config 2 |  | SSC.2 |  |
| Timing Advance Command (TA) value during T1 |  |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  |  | 39 | NTA = 128 |
| Number of repetitons | NPDCCH |  | 128 | Smaller repetition values for NTN TDD |
|  | NPDSCH |  | 128 |  |
|  | NPUSCH |  | 32 |  |
| DRX |  |  | OFF |  |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

#### Issue 4-2-7: Test configuration for RLM test

Agreement

- RAN4 to discuss the Qin and Qout for RLM test cases based on Rmax =2.

2.4.3		Remaining open issues (RAN4 RRM)

  - None

## 2.4.4	Agreements during RAN4#117 Demodulation

- SAN Demodulation – NPUSCH Format 1

Frequency Offset with 12 Tones

Agreement:

2 Hz

Transmission Timing Drift

Agreement:

Apply additional transmitting timing drift to NPUSCH format 1. Not consider additional transmitting timing drift for NPRACH performance requirements. The values were not considered in TS 36.108, and consider it in TS 36.181 with following value.

| Channel | Step size Δt |
| --- | --- |
| NPUSCH format 1, 15KHz, 12 tones | [0.01] µs per RU |

- SAN Demodulation – NPUSCH Format 2

Repetition Number

Agreement:

4 with total Tx Duration of 8

Frequency Offset

Agreement:

4Hz

Test Metric

Agreement:

1% of ACK missed detection probability

Transmission Timing Drift

Agreement:

Apply additional transmitting timing drift to NPUSCH format 2. Not consider additional transmitting timing drift for NPRACH performance requirements. The values were not considered in TS 36.108, and consider it in TS 36.181 with following value

| Channel | Step size Δt |
| --- | --- |
| NPUSCH format 2, 15KHz, 1 tone | [0.02] µs per RU |

- SAN Demodulation – NPRACH

Channel Model

Agreement:

NTN-TDLA100-1

NTN-TDLC5-1

Number of Repetition

Options:

- Option 1

- 4 for Format 0

- 2 for Format 1

- Option 2

- 16 for Format 0

- 8 for Format 1

Companies encouraged to provide results for both options so that the requirement can be defined at RAN4#118, when an option is downselected.

- Test Applicability

Test applicability rule for different antenna configuration

Agreement:

RAN4 should introduce the test applicability rule for IoT NTN requirement, where the test of IoT NTN requirement is only applied to one antenna, while the specific connector used for testing it based on manufacturer declaration

FFS on the wording.

Test applicability rule for TDD

Agreement:

The requirements for TDD are optional and only valid for SAN supporting TDD pattern. RAN4 should introduce applicability rule for IoT NTN TDD requirement. And the related test should be performed based on manufacturer declaration.

[Post 117][300] BDaT email approval:

  - R4-2523102	Big draft CR on 36.133 for IoT_NTN_TDD   MediaTek  Endorsed

#### 2.4.5	Agreements during RAN4#117 RRM

Issue 4-1-1: Measurement accuracy

WayForward

- Discuss option 1 and 2 for Minimum NRSRP on the following table.

Table B.3.25A-1: NB-IoT intra-frequency absolute NRSRP and NRSRQ Accuracy Requirements

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum NRSRP |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_SAB _G | -139.8 |
|  | NTDD_SAB_O | -128 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |

Issue 4-2-1: Principle of testing TDD and FDD frame structure

Agreement

- UE supporting both FDD and TDD should pass the test cases with each of the FDD and TDD configurations as default.

Issue 4-2-2: Test case list Test parameters - TSI_NB-IoT  and TPRACH_NB-IoT

WayForward

- FFS TSI_NB-IoT  and TPRACH_NB-IoT  for TDD.

Issue 4-2-3: Test configuration for random access test

- For IoT NTN TDD Reference NPRACH Configuration, keep coverage level 2.

- Please check the proposed values directly in the corresponding CR.

Issue 4-2-4: Test configuration for timing advance accuracy Test

- The following test parameters are adopted.

| Number of repetitions | NPDCCH | 4 |
| --- | --- | --- |
|  | NPDSCH | 8 |
|  | NPUSCH | 8 |

- Note that the timing advance accuracy should be measured independently for each UL burst of 8 ms.

Issue 4-2-5: Test configuration for RLM test

- The Qin = 1 dB and Qout = -5 dB can be used as a reference for specifying SNR values in RLM test cases.

- Please check the proposed values directly in the corresponding CR.

## 4.	References

## 4.1	RAN1

## RAN1#122bis meeting, Prague, CZ , Oct 13th – 17th, 2025

IoT-NTN TDD mode – papers

- R1-2506718	LS on OCC for IoT-NTN TDD mode	RAN2, Huawei

- R1-2506731	LS Reply on precompensation for NB-IoT NTN TDD mode	RAN4, Iridium

- R1-2506859	Draft reply LS on OCC support for IoT NTN TDD	vivo

- R1-2506860	Discussion on OCC support for IoT NTN TDD	vivo

- R1-2506864	Draft reply LS on pre-compensation for IoT TDD mode	vivo

- R1-2506865	Discussion on pre-compensation for IoT TDD mode	vivo

- R1-2506908	Discussion on the LS on OCC for IoT-NTN TDD mode	ZTE Corporation, Sanechips

- R1-2506909	Discussion on the LS Reply on precompensation for NB-IoT NTN TDD mode	ZTE Corporation, Sanechips

- R1-2506911	Remaining issues on Rel-19 IoT NTN	ZTE Corporation, Sanechips

- R1-2506937	Maintenance for IoT NTN Phase 3 and TDD mode	Huawei, HiSilicon

- R1-2506940	UE features for IoT-NTN TDD mode	Huawei, HiSilicon

- R1-2506954	Discussion on the LS on OCC for IoT-NTN TDD mode	xiaomi

- R1-2506968	Maintenance for Rel-19  IoT NTN	Xiaomi

- R1-2507084	Discussion on LS on OCC for IoT-NTN TDD mode	CATT

- R1-2507086	Discussion on LS Reply on precompensation for NB-IoT NTN TDD mode	CATT

- R1-2507138	Discussion on UE features for IoT-NTN TDD mode	OPPO

- R1-2507145	Discussion on LS on OCC for IoT-NTN TDD mode	OPPO

- R1-2507146	Draft reply LS on OCC for IoT-NTN TDD mode	OPPO

- R1-2507147	Discussion on LS Reply on precompensation for NB-IoT NTN TDD mode	OPPO

- R1-2507148	Draft reply on LS on precompensation for NB-IoT NTN TDD mode	OPPO

- R1-2507296	Discussion on RAN2 LS on OCC for IoT-NTN TDD mode	Nokia, Nokia Shanghai Bell

- R1-2507684	Precompensation for IoT-NTN TDD mode	Qualcomm Incorporated

- R1-2507686	OCC for IoT-NTN TDD mode	Qualcomm Incorporated

- R1-2507915	Discussion on OCC for IoT-NTN TDD mode	Huawei, HiSilicon

- R1-2507916	Draft LS reply on OCC for IoT-NTN TDD mode	Huawei, HiSilicon

- R1-2507701	Maintenance for Rel-19 IOT NTN	Qualcomm Incorporated

- R1-2507993	Feature lead summary #1 for IoT NTN TDD mode	Moderator (Qualcomm)

IoT-NTN TDD mode – papers related to UE features

- R1-2506940	UE features for IoT-NTN TDD mode	Huawei, HiSilicon

- R1-2507126	Discussion on UE features for IoT-NTN TDD mode		CATT

- R1-2507138	Discussion on UE features for IoT-NTN TDD mode		OPPO

- R1-2507275	UE features for IoT NTN TDD mode		Samsung

## RAN1#123 meeting, Dallas, USA , Nov 17th – 21st, 2025

IoT-NTN TDD mode – papers

- R1-2508413	Maintenance on Rel-19 IoT-NTN	vivo

- R1-2508480	Maintenance for Rel19 IoT-NTN	Nokia, Nokia Shanghai Bell

- R1-2508488	Maintenance for IoT NTN Phase 3 and TDD mode	Huawei, HiSilicon

- R1-2508600	Maintenance for Rel-19 IoT NTN	CATT

- R1-2508665	Maintenance for Rel-19  IoT NTN	Xiaomi

- R1-2508703	Maintenance for Rel-19 IoT NTN	OPPO

- R1-2508782	Maintenance for Rel-19 IoT NTN	Samsung

- R1-2508851	Remaining issues on Rel-19 IoT NTN	ZTE Corporation, Sanechips

- R1-2508867	Maintenance for Rel-19 IoT-NTN	Ericsson

- R1-2509063	FL Summary #1 for Rel-19 IoT-NTN maintenance	Moderator(Sony)

- R1-2509064	FL Summary #2 for Rel-19 IoT-NTN maintenance	Moderator(Sony)

- R1-2509065	Final FL summary for Rel-19 IoT-NTN maintenance	Moderator(Sony)

- R1-2509135	On scheduling enhancement to IoT NTN TDD	Nordic Semiconductor ASA

- R1-2509206	Maintenance for Rel-19 IOT NTN	Qualcomm Incorporated

- R1-2509207	Feature Lead Summary #1 on maintenance for IoT NTN TDD mode	Moderator (Qualcomm Incorporated)

## 4.2	RAN2

## RAN2#131bis meeting, Prague, CZ , Oct 13th – 17th, 2025

IoT-NTN TDD mode – papers

- R2-2506714	LS on updated text proposal for 36.300 for IoT NTN TDD mode (R1-2506535; contact: Qualcomm)	RAN1

- R2-2506741	LS Reply on precompensation for NB-IoT NTN TDD mode (R4-2512550; contact: Iridium)	RAN4

- R2-2507048	Discussion on IoT NTN TDD mode	OPPO

- R2-2507062	Rapporteur correction on IoT NTN TDD	Huawei, HiSilicon

- R2-2507063	RIL status on IoT NTN TDD	Huawei, HiSilicon

- R2-2507064	Neighbour cell measurement in IoT NTN TDD	Huawei, HiSilicon

- R2-2507262	Rapporteur correction on IoT NTN TDD	Huawei, HiSilicon

- R2-2507442	Remaining issues for NB-IoT NTN TDD mode	Qualcomm Incorporated

- R2-2507456	Discussion on RIL [X501] for IoT NTN TDD	Samsung

- R2-2507612	Remaining issues on support of TDD mode for NB-IoT-NTN	Nokia, Nokia Shanghai Bell

- R2-2507674	Discussion on the RIL X501 (radioFrameOffset-r19) for IoT NTN TDD	Beijing Xiaomi Electronics

- R2-2507710	Reply LS on precompensation for NB-IoT NTN TDD mode (R1-2508064; contact: Qualcomm)	RAN1

- R2-2507772	Rapporteur correction on IoT NTN TDD	Huawei, HiSilicon

- R2-2507773	Report of [AT131bis][311][IoT NTN TDD] RRC open issues	Huawei

- R2-2507778	Report of [AT131bis][309][IoT NTN TDD] radioFrameoffset per cell	Samsung

- R2-2507788	Rapporteur correction on IoT NTN TDD	Huawei, HiSilicon

## RAN2#132 meeting, Dallas, USA , Nov 17th – 21st, 2025

IoT-NTN TDD mode – papers

- R2-2508013	LS reply on OCC for IoT-NTN TDD mode (R1-2508110; contact: Huawei)	RAN1

- R2-2508042	Remaining Issues on IoT NTN TDD Operation	vivo

- R2-2508131	Clarification on inter-freq neighbor cell meas for IOT NTN TDD	Iridium (Rapporteur)

- R2-2508160	Discussion on SPS collision issues for IoT NTN TDD	CATT

- R2-2508199	Reply LS on precompensation for NB-IoT NTN TDD mode (R1-2508064; contact: Qualcomm)	RAN1

- R2-2508276	Rapporteur correction on IoT NTN TDD	Huawei, HiSilicon

- R2-2508277	RIL status on IoT NTN TDD	Huawei, HiSilicon

- R2-2508278	Remaining issues for IoT NTN TDD	Huawei, HiSilicon

- R2-2508299	Discussion on SPS collision in IoT NTN TDD mode	OPPO

- R2-2508314	Discussion on remaining open issues for IoT NTN TDD	Nokia, Nokia Shanghai Bell

- R2-2508637	Correction on IoT NTN TDD for DRX	Toyota ITC (Rapporteur)

- R2-2508793	Corrections on neighbour cell measurement for IOT NTN TDD	Iridium (Rapporteur), Huawei

- R2-2508811	SPS for NB-IoT NTN TDD mode	Qualcomm Incorporated

- R2-2508912	Discussion on remaining issue for IoT NTN TDD mode	ZTE Corporation,  Sanechips

- R2-2508915	Discussion on the remaining issues for the IoT NTN TDD	Xiaomi

- R2-2509206	Corrections on neighbour cell measurement for IOT NTN TDD	Iridium (Rapporteur), Huawei

- R2-2509207	Report of [AT132][304][IoT NTN TDD] SPS	CATT

- R2-2509368	Rapporteur correction on IoT NTN TDD	Huawei, HiSilicon

- R2-2509372	Correction on IoT NTN TDD for DRX	Toyota ITC (Rapporteur)

## 4.3	RAN3

## 4.4	RAN4

RAN4#116bis meeting, Prague, CZ , Oct 13th – 17th, 2025

IoT-NTN TDD mode – papers

- R4-2513406	View on demoduation requirement for IoT NTN TDD	Samsung

- R4-2513639	Discussion on RRM performance requirements for IoT-NTN TDD mode	MediaTek inc.

- R4-2513824	Maintenance of RRM core requirements for IoT NTN TDD	Iridium, CCL, THALES

- R4-2513825	Demodulation performance requirements for IOT NTN TDD	Iridium, CCL

- R4-2513826	On RRM performance requirements for IOT NTN TDD	Iridium, CCL, THALES

- R4-2514027	DraftCR to TS36.108 Revise section 9.2, 9.3 and 9.7.5 for IoT-NTN TDD band	ZTE Corporation, Sanechips

- R4-2514029	Discussion on SAN conformance testing requirements for Iot-NTN TDD band	ZTE Corporation, Sanechips

- R4-2514074	On demodulation performance requirements of IoT-NTN TDD mode	Nordic Semiconductor ASA

- R4-2514180	Discussion on core requirements for IoT NTN TDD	Huawei, HiSilicon

- R4-2514181	draftCR on core requirements for IoT NTN TDD	Huawei, HiSilicon

- R4-2515128	draftCR on core requirements for IoT NTN TDD	Huawei, HiSilicon

- R4-2514182	Discussion on performance requirements for IoT NTN TDD	Huawei, HiSilicon

- R4-2514247	On demodulation performance requirements of IoT-NTN TDD mode	Nordic Semiconductor ASA

- R4-2514250	On remaining issues of IoT_NTN_TDD related to RRM core  Nordic Semiconductor ASA

- R4-2514342	Open issues the measurement framework for the new IOT TDD Punctured frame	Nokia

- R4-2514343	DraftCR to TS 36.133 on Editorial Correction for IoT NTN TDD Requirements	Nokia

- R4-2514344	Test case configurations for NTN IOT TDD	Nokia

- R4-2514345	Issues on pre-compensation gap in TS 36.102	Nokia

- R4-2514346	DraftCR to TS 36.133 on Corrections on the segmentation gap in IoT TDD NTN	Nokia

- R4-2514411	Draft CR on TS 36.181 - IoT_NTN_TDD	THALES, Iridium Satellite LLC

- R4-2515065 	WF Rel-19_BS_SAN_conformance_Part2 [R116b][308]	Moderator (Iridium)

- R4-2514431	TopicSummary_116bis_308 Rel-19_BS_SAN_conf_Part2	 Moderator (Iridium)

- R4-2515055 	WF [116bis][310] R19 RRMperf_Part2_NTN_V2  Moderator (MediaTek inc.)

- R4-2515160 	Way Forward for [116bis][312] Rel-19 Demodulation_Part2  Moderator (Nokia)

- R4-2514532	TopicSummary 116b_325 NR_IoT_NTN_Maintenance_RRM  Moderator (Iridium)

- R4-2514339	On test cases for CB-MSG3 in TS 36.133  Nokia

- R4-2514422	Demodulation performance requirements for NTN NB-IoT TDD  THALES

- R4-2514423	General discussion on IoT_NTN_TDD demodulation performance requirements  THALES , Iridium, Cambridge Consultants (CCL)

- R4-2515064	Draft CR on TS 36.181 - IoT_NTN_TDD   THALES, Iridium Satellite LLC, ZTE

- R4-2514435	Topic summary for [116bis][312] Rel-19 Demodulation_Part2  Moderator (Nokia)

- R4-2515041	Way Forward for [116bis][312] Rel-19 Demodulation_Part  Moderator (Nokia)

- R4-2514433	Topic summary for [116bis][310] Rel-19 RRM performance_Part2  Moderator (MediaTek)

RAN4#117 meeting, Dallas, USA , Nov 17th – 21st, 2025

IoT-NTN TDD mode – papers

- R4-2520015	(IoT_NTN_TDD-Perf) DraftCR Applicability Rule IoT NTN TR 36.102	Nokia

- R4-2523076	(IoT_NTN_TDD-Perf) DraftCR Applicability Rule IoT NTN TR 36.102	Nokia

- R4-2520038	Introduction of test case of RRC Re-establishment and Random Access for NB-IoT in TDD mode with Satellite Access	MediaTek inc.

- R4-2523066	Introduction of test case of RRC Re-establishment and Random Access for NB-IoT in TDD mode with Satellite Access	MediaTek inc.

- R4-2520261	Draft CR for 36.133 RRM meas accuracy, reference NPRACH config for IoT_NTN_TDD	CAMBRIDGE CONSULTANTS LTD, Iridium Satellite LLC, THALES

- R4-2523067	Draft CR for 36.133 RRM meas accuracy, reference NPRACH config for IoT_NTN_TDD	CAMBRIDGE CONSULTANTS LTD, Iridium Satellite LLC, THALES

- R4-2520269	LS Reply on precompensation for NB-IoT NTN TDD mode	Iridium Satellite LLC

- R4-2523058	LS Reply on precompensation for NB-IoT NTN TDD mode	Iridium Satellite LLC

- R4-2520547	Draft CR on 36.133 introduction of RLM test cases for  IoT_NTN_TDD	CAMBRIDGE CONSULTANTS LTD, Iridium Satellite LLC, THALES

- R4-2523068	Draft CR on 36.133 introduction of RLM test cases for  IoT_NTN_TDD	CAMBRIDGE CONSULTANTS LTD, Iridium Satellite LLC, THALES

- R4-2520607	On RRM Performance Requirements for IOT NTN TDD	Iridium, Cambridge Consultants (CCL), THALES

- R4-2520608	Draft CR on 36.133 introduction of DL CQI reporting accuracy test cases for IoT_NTN_TDD	CAMBRIDGE CONSULTANTS LTD, Iridium Satellite LLC, THALES

- R4-2523069	Draft CR on 36.133 introduction of DL CQI reporting accuracy test cases for IoT_NTN_TDD	CAMBRIDGE CONSULTANTS LTD, Iridium Satellite LLC, THALES

- R4-2520698	Draft CR on demodulation performance for TS 36.102 - IoT_NTN_TDD	Nordic Semiconductor, THALES, Iridium Satellite LLC, CCL

- R4-2520701	IoT_NTN_TDD RRM performance	Nordic Semiconductor ASA

- R4-2520890	CR on pre-compensation for timing error requirement for IoT NTN TDD (Clause 7.20B.2)	LG Electronics Inc.

- R4-2521359	Discussion on IoT NTN TDD demodulation performance requirements	Huawei,HiSilicon

- R4-2521492	View on demodulation requirement for IoT NTN TDD	Samsung

- R4-2521558	Test case on timing advance	ZTECorporation,Sanechips

- R4-2523070	Test case on timing advance	ZTECorporation,Sanechips

- R4-2521559	Test case on transmit timing	ZTECorporation,Sanechips

- R4-2523071	Test case on transmit timing	ZTECorporation,Sanechips

- R4-2521651	Discussion on time pre-compensation for IoT NTN TDD	Huawei, HiSilicon

- R4-2521652	CR on core requirements for IoT NTN TDD	Huawei, HiSilicon

- R4-2521653	Discussion on performance requirements for IoT NTN TDD	Huawei, HiSilicon

- R4-2521654	draftCR on measurement test cases for IoT NTN TDD	Huawei, HiSilicon

- R4-2521733	CR  with Editorial Correction for IoT NTN TDD Requirements	Nokia

- R4-2521734	On removing support of segmented pre-compensation for IoT NTN TDD	Nokia

- R4-2521735	(IoT_NTN_TDD-Perf) CR on removing support for segmented precompensation	Nokia

- R4-2521742	CR  Reselection test cases for IoT NTN TDD	Nokia

- R4-2521872	On RRM Performance Requirements for IOT NTN TDD	Iridium,CCL, Thales

- R4-2522036	(IoT_NTN_TDD-Perf) CR  on removing support for segmented precompensation	Nokia

- R4-2522136	Discussion on precompensation for NB-IoT NTN TDD mode	Huawei, HiSilicon

- R4-2522148	Draft CR on demodulation performance for TS 36.102 - IoT_NTN_TDD	Nordic Semiconductor ASA

- R4-2523077	Draft CR on demodulation performance for TS 36.102 - IoT_NTN_TDD	Nordic Semiconductor ASA, Thales, Iridium, CCL

- R4-2522195	Confirmation of demodulation performance requirements values for NTN NB-IoT TDD	THALES, Iridium Satellite LLC, CCL, Nordic Semiconductor

- R4-2522196	Draft CR on demodulation performance for TS 36.108 - IoT_NTN_TDD	THALES, Iridium Satellite LLC, CCL

- R4-2523078	Draft CR on demodulation performance for TS 36.108 - IoT_NTN_TDD	THALES, Iridium Satellite LLC, CCL

- R4-2522203	Draft CR on the performance conformance testing for TS 36.181 - IoT_NTN_TDD	THALES, Iridium Satellite LLC, CCL

- R4-2523080	Draft CR on the performance conformance testing for TS 36.181 - IoT_NTN_TDD	THALES, Iridium Satellite LLC, CCL

- R4-2522265	CR for core requirements under TS 36.108 - IoT_NTN_TDD	THALES, ZTE, Iridium

- R4-2520005	Reply LS on precompensation for NB-IoT NTN TDD mode	RAN1

- R4-2523100 	LS Reply on precompensation for NB-IoT NTN TDD mode	RAN4

- R4-2523057  	CR for 36.133 on removing support for segmented precompensation  Nokia, LG

- R4-2522036	CR for 36.102 on removing support for segmented precompensation  Nokia

- R4-2522216	Topic summary for [117][308] Rel-19_BS_SAN_conf_Part2_NTN  Moderator(THALES)

- R4-2522222	Topic summary for [117][312] Rel-19 Demod_Part2_NTN  Moderator(Nokia)

- R4-2522932 	Way Forward for [117][312] Rel-19 Demod_Part2_NTN  Moderator(Nokia)

- R4-2523085 	Way Forward for [117][312] Rel-19 Demod_Part2_NTN  Moderator(Nokia)

- R4-2522933	Offline meeting minutes for [117][312] Rel-19 Demod_Part2_NTN  Moderator(Nokia)

- R4-2522949	Way Forward for [117][308] Rel-19_BS_SAN_conf_Part2_NTN  Moderator(THALES)

- R4-2523079	TDD Band Simulation Collection Summary   Thales, Nokia, Iridium, CCL, Nordic

- R4-2523102	Big draft CR on 36.133 for IoT_NTN_TDD   MediaTek

- R4-2522219	Topic summary for [117][310] Rel-19 RRM perf_Part2_NTN  MediaTek

- R4-2522936	Way Forward for [117][310] Rel-19 RRM perf_Part2_NTN  MediaTek