3GPP TSG-RAN WG1 Meeting #98	Tdoc R1-19xxxxx

Prague, CZ, Aug 26th –30th, 2019

Agenda Item:	x.x.x

Source:	Ericsson

Title:	Meeting report from informal F2F meeting on indoor industrial channel model

Document for:	Information

# 1	Introduction

An informal F2F meeting was conducted on July 1-2 in Berlin, Germany. The purpose of this meeting was to facilitate technical discussions and consensus building among channel model experts to support the SI on Indoor Industrial channel modeling. This tdoc contains a meeting report summarizing the discussions and views that were exchanged during the meeting.

# 2	Participation

The following participants attended the meeting:

- Raschkowski, Leszek, Fraunhofer HHI

- Henrik Asplund, Ericsson

- Rodriguez Larrad, Ignacio, Nokia

- Koshiro Kitao, DOCOMO

- Prasanth Karunakaran, Fraunhofer IIS

- Schmieder, Mathis, Fraunhofer HHI

- Jaeckel, Stephan, Fraunhiofer HHI

- Tommi Jamsa, Huawei

- Landmann, Markus, Fraunhofer IIS

- Müller, Robert, Fraunhofer IIS

- Guo Bolun, Huawei

Remote participants:

- Piyush Gupta, Qualcomm

- Raffaele D'Errico, CEA-LETI

- Jianwu Dou, ZTE

- Georg Wannemacher, Deutsche Telekom

- Rakesh Tamrakar, Vivo

- Xiaodong Sun, Vivo

- Jian Lou(spelling?), Huawei

# 3	Company input

Company input is available at ftp://ftp.3gpp.org/Email_Discussions/RAN1/RAN1_IIChM_July19/Docs/

# 4	Technical discussions

## 4.1	New measurements

Fraunhofer IIS presented the contribution R1-XXXX-Single-band.docx containing new measurements in a high clutter density environment.

- Comments on how to take into account the very large relative bandwidth in the path loss analysis

- What percentage of floor area is covered by clutter? 3D scanner data available that can be processed

- Discussion on how this clutter density compares with some other available measurements

- Raw path loss data for 6 GHz has been shared

- 30 GHz may be added before RAN1#98

Fraunhofer HHI presented the contribution 2019-07-01 Industrial Indoor Measurements.pptx containing new measurements in high clutter density environments.

- Can share path loss data points (5500 points in total)

- Angular spread is based on specular only (100 paths)

- Question on ratio between power of specular components and dense multipath. Has been analyzed…

- Which sub-scenario? 2 and 4: high clutter density, 3 m clutter height, BS height 2-8 m

- Fraunhofer have added their path loss and LSP results (possibly split into sub-scenarios) into R1-19xxxxx Path loss model parameters_v3_CEA_HHI.xlsx, raw path loss data will be shared

## 4.2	Scenario description

Discussion on threshold between low and high clutter density:

- Nokia has estimated the clutter densities as above 50% for all high clutter density scenarios

- Fraunhofer IIS has calculated clutter density to 35% for a high clutter scenario

- Ask CMCC about clutter density in their low clutter scenario

- DOCOMO has estimated 29% in their scenario which is in between low and high clutter density, Huawei: DOCOMO path loss seems different compared to high density results

Outcome: High clutter density is above [35%] and low clutter density is below [35%]. Classify DOCOMO results as low clutter density

Discussion on clutter height:

- Ericsson proposes 1-10 m, Nokia proposes an open range with h_clutter lower than ceiling height

- Nokia: open scenarios have higher ceiling height than high clutter density scenarios, suggest to couple ceiling height range with clutter density

- Nokia has compiled a complete scenario description table in R1-xxxxxxxx_scenario_parametrization_nokia_v2.docx.

  - Discussion outcome: Use this scenario description with the following changes:

    - Change the ceiling height for sub-scenario 5 to [5-25] m

    - “Clutter height” -> “Effective clutter height”

    - Add “typical clutter size”: Big = 10 m, Small to medium = 2 m

## 4.3	Path loss

Huawei presented a first attempt at producing merged path loss models in IIot Path Loss measured data expressions and curves_v1.3.xlsx based on (a sub-set of) the available raw path loss data. The following improvements were discussed:

- Classify/re-classify sub-scenario association

- Include missing raw data, e.g. Huawei/CEA-LETI/Fraunhofer (AP: Huawei)

- Include random raw data generated from contributed path loss parameters (AP: Huawei, Nokia)

  - How many data points? If not specified, check the reference if there is some information. Otherwise, handle later.

- How to apply weights to the data from different sources (low vs high number of samples in different data sets)

  - No additional weighting as a starting point. Revisit later.

Nokia: will provide random path loss data based on the v3 email discussion summary by end of Monday 1 July. Huawei has updated to v1.4 to include “all LOS” and “all NLOS”. Huawei will update the excel file with CMCC, CEA-LETI, Fraunhofer HHI raw data and random raw data by Tuesday afternoon.

Discussion on embedded devices:

- Qualcomm may do new measurements on other aspects than path loss, trying to complete before August meeting

- Wait with further discussion until August, Qualcomm encouraged to provide a complete model proposal

## 4.4	LOS probability

Discussion on LOS probability:

- Agreement from Reno: PrLOS,subsced2D=psubsce,d2D≤dsubsceme-d2D-dsubsceksubsce⋅psubsce,d2D>dsubscem

- Two proposals: empirical or analytical as captured in ZTEs contribution ZTE_LOS probability.docx.

Discussion outcome: As a starting point for calibration, use proposal 2 from ZTE:

Proposal 2: For analytical LOS probability model, use following formula to calculate ksubsce, where r is the effective clutter density after removing the clutters no higher than UE, dclutter represents the clutter size, {hc, hBS, hMS} are the heights of clutter, base station and terminal, respectively.

ksubsce=-dclutterln1-rBS embedded-dclutterln1-r∙hBS-hUThc-hUTBS elevated

- psubsce and dsubsce are FFS.

Discussion outcome: use the following values as starting point and for calibration:

- For elevated and high clutter density: d_subsce=[1], p_subsce=[0.6]

- For all other cases, d_subsce=[0], p_subsce=[1]

Discussion on LOS autocorrelation:

- Two proposals: 18/20 m or equal to dclutter

- Nokia: Related to dclutter, to make it different for different sub-scenarios

- Huawei: 18/20 m is seen as too long, perhaps use a smaller distance such as 10/20?

- Ericsson: good to have short correlation distance to get better statistical significance of simulation of a limited area

- Fraunhofer HHI: prefer single number

- Outcome of discussion: Use [10 m] for calibration

## 4.5	Fast fading modelling

Discussion on how to merge LSP proposals

- Ericsson provided an updated plot of DS vs hall volume to include new results that were not part of the tdoc in Reno in Delay spread vs hall volume_v3.pptx. Discussion:

  - “Raw data” available in the ppt and in R1-1905202. Further possible analysis to look at reverberation theory using surface area and volume [Fraunhofer IIS, this week]

- Huawei has provided updated LSP proposals in R1-19xxxxx LSP parameter proposals_v3_CEA_HW2.xlsx

- Fraunhofer has provided updated LSP proposals in R1-19xxxxx LSP parameter proposals_v3_CEA_HW_HHI.xlsx

- These two need to be merged [Huawei, by Tuesday afternoon]

- Possible merging principles: Revisit after the table is updated

  - Treat DS separately

  - Treat K-factor separately

  - Averaging all other LSP proposals

Discussion on dense multipath

- The experience from Fraunhofer IIS is that up to 80-90% of indoor power is captured in the dense multipath, more dense multipath in sub-6 GHz than at mmw

- Different views on the spatial properties of the DMC, white or not?

- How to capture dense multipath? Proposals:

  - Optional model: Consider if 38.901 sec 7.6.2 approach can be used to emulate DMC using more rays

  - Optional model: Colored noise process to model the transfer function (of frequency and tx/rx antenna elements), using frequency correlation function and angular correlation functions. Few analysis available on the correlation properties of the DMC in f and antenna dimensions

  - Baseline model: Change (increase) the number of clusters and rays in the baseline model

  - Room electromagnetics for DMC (Univ of Ghent): 30% of the power in the DMC

  - Do nothing

- Conclusion: encourage companies to analyze DMC in their measurements and if necessary propose models and parameterizations until Prague. If no such proposals, don’t do anything.

## 4.6	Additional modelling components

Presentation on new measurements and modeling of additional delay by Fraunhofer IIS and Huawei in Berlin_Meeting_NLOS_Relative_Delay_Results_v2.pptx. Discussion:

- New results show clear dependence on distance

- Lower bound at shorter distances may be due to obstructed LOS (diffraction). Suggestion to use lower bound for  that is larger than 0

- Can the distance trend be extrapolated beyond 40 m? Will give >700 ns DS for 280 m…

- Perhaps fitting against log(distance) instead?

- Select strongest or first peak? The first peak within -5 dB of the strongest one

- 6 and 60 GHz data may be available later, also analysis with different thresholds

Discussion on absolute delay modeling

- How to select from the different proposals (including the new one from H/IIS)?

- Outcome: Encourage companies to share data: (f,d,) samples and report the used detection threshold

  - Analyze based on merging of data from multiple sources [Huawei volunteers]

  - Further discussion on upper bound and cross-correlations over email

## 4.7	Channel model calibration

- Table 7.8-2: Simulation assumptions for large scale calibration for the indoor industrial scenario

| Parameter | Values |
| --- | --- |
| Scenario | Indoor industrial – sub-scenarios 1-4 |
| Room size | 100x100 m |
| Room height | 10 m |
| Sectorization | None |
| BS antenna configurations | 1 element (vertically polarized), Isotropic antenna gain pattern |
| UT antenna configurations | 1 element (vertically polarized), Isotropic antenna gain pattern |
| Handover margin (for calibration) | 0dB |
| BS deployment | Rectangular grid with ISD = 20 m, FFS on exact grid and number BS height = [1.5] m or 8 m |
| UT distribution | uniform dropping for indoor with minimum distance ([2D or 3D]) of [1] m UT height = 1.5 m |
| UT attachment | Based on pathloss |

- Outcome of discussion: Use the following simulation assumptions for calibration of the indoor industrial scenario:

  - Hall sizes: sub-scenario 1: 60x120 m, 2: 300x150 m, 300x150 m, 4: 60x120 m

  - Use a minimum 2D dropping distance of 1 m

  - BS height is 1.5 m for the clutter-embedded scenarios

  - BS tx power: 30 dBm

  - BS deployment: in rectangular grid with 20 m spacing and 10 m from the walls for the small halls, and in rectangular grid with 50 m spacing and 25 m from the walls for the big halls

  - UT noise figure: 9 dB

  - Carrier frequency: 3.5 GHz, 28 GHz

  - Bandwidth: 100 MHz

- Outcome of discussion: The following metrics are proposed for the channel model calibration:

  - 1) Coupling loss – serving cell

  - 2) Geometry with and without noise

  - 3) CDF of delay and angle spread (ASD, ZSD, ASA, ZSA) according to definition in Annex A.1 of TR 38.901

  - 4) CDF of first path excess delay for serving cell

Discussion outcome: Use the following parameters for calibration:

|  | Low clutter density | High clutter density |
| --- | --- | --- |
| Clutter density: r | 20% | 60% |
| Clutter height: hc | 2 m | 6 m |
| Clutter size: dclutter | 10 m | 2 m |

# References

RP-182138, SID on Channel Modeling for Indoor Industrial Scenarios, Ericsson, 3GPP TSG-RAN Meeting #81, Gold Coast, Australia, September 10th – 13th 2018.

R1-1907920, List of agreements, Ericsson, RAN1#97, Reno, USA, May 13-17, 2019.

R1-1907940, Addition of indoor industrial channel model, Ericsson, RAN1#97, Reno, USA, May 13-17, 2019.

R1-1907405, Summary of email discussion on path loss, Ericsson, RAN1#97, Reno, USA, May 13-17, 2019.

R1-1907407, Summary of email discussion on fast fading, Ericsson, RAN1#97, Reno, USA, May 13-17, 2019.