6.5.10.2	NR DL-TDOA Assistance Data Request

–	NR-DL-TDOA-RequestAssistanceData

The IE NR-DL-TDOA-RequestAssistanceData is used by the target device to request assistance data from a location server.

-- ASN1START

NR-DL-TDOA-RequestAssistanceData-r16 ::= SEQUENCE {

nr-PhysCellID-r16				NR-PhysCellID-r16							OPTIONAL,

nr-AdType-r16					BIT STRING {	dl-prs	(0),

posCalc (1) } (SIZE (1..8)),

...,

[[

nr-PosCalcAssistanceRequest-r17	BIT STRING {	trpLoc		(0),

beamInfo	(1),

rtdInfo		(2),

losNlosInfo	(3),

trpTEG-Info	(4),

integrityBounds-r18 (5),

pruInfo-r18	(6)

}	(SIZE (1..8))				OPTIONAL,

nr-on-demand-DL-PRS-Request-r17	NR-On-Demand-DL-PRS-Request-r17				OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Request-r17

ENUMERATED { eAoD, eAoA }					OPTIONAL,

pre-configured-AssistanceDataRequest-r17

ENUMERATED { true }							OPTIONAL

]],

[[

nr-PeriodicAssistDataReq-r18 NR-PeriodicAssistDataReq-r18	OPTIONAL -- Cond PerADReq,

nr-IntegrityAssistanceParaRequest-r18	BIT STRING {

riskPara		(0),

servicePara 	(1),

serviceAlert		(2),

integrityParaTRP-Loc	(3),

integrityParaBeamInfo	(4),

integrityParaRTD-Info	(5),

}	(SIZE (1..8)) OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| PerADReq | This field is mandatory present if the target device requests periodic NR assistance data delivery. This field may only be included if 'pruInfo' bit in nr-PosCalcAssistanceRequest is set to '1'. |

NR-DL-TDOA-RequestAssistanceData field descriptions

nr-PhysCellID This field specifies the NR physical cell identity of the current primary cell of the target device.

nr-AdType This field indicates the requested assistance data. dl-prs means requested assistance data is nr-DL-PRS-AssistanceData, posCalc means requested assistance data is nr-PositionCalculationAssistance for UE based positioning.

nr-PosCalcAssistanceRequest This field indicates the Position Calculation Assistance Data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is requested or not; - bit 3 indicates whether the field nr-DL-PRS-Expected-LOS-NLOS-Assistance in IE NR-PositionCalculationAssistance is requested or not; - bit 4 indicates whether the field nr-DL-PRS-TRP-TEG-Info in IE NR-PositionCalculationAssistance is requested or not. - bit 5 together with the bit 0 indicate whether the field integrityTRP-LocationBounds in IE NR-TRP-LocationInfo is requested or not. bit 5 together with the bit 1 indicate whether the field integrityBeamInfoBounds in IE NR-DL-PRS-BeamInfo is requested or not. bit 5 together with the bit 2 indicate whether the field integrityRTD-InfoBounds in IE NR-RTD-Info is requested or not. - bit 6 indicates whether the field nr-PRU-DL-Info in IE NR-PositionCalculationAssistance is requested or not. This field may only be present if the 'posCalc' bit in nr-AdType is set to value '1'.

nr-on-demand-DL-PRS-Request This field indicates the on-demand DL-PRS requested for DL-TDOA. This field may be included when the dl-prs bit in nr-AdType is set to value '1'.

nr-DL-PRS-ExpectedAoD-or-AoA-Request This field, if present, indicates that the IE NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData is requested. Enumerated value 'eAoD' indicates that expected AoD information is requested; value 'eAoA' indicates that expected AoA information is requested. This field may only be present if the 'dl-prs' bit in nr-AdType is set to value '1'.

pre-configured-AssistanceDataRequest This field, if present, indicates that the target device requests pre-configured assistance data with area validity.

nr-PeriodicAssistDataReq This field indicates the Periodic Position Calculation Assistance Data requested.

nr-IntegrityAssistanceParaRequest This field indicates the Integrity Calculation Assistance Data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. - bit 0 indicates whether the field nr-IntegrityServiceParameters in IE NR-PositionCalculationAssistance is requested or not; - bit 1 indicates whether the field nr-IntegrityServiceAlert in IE NR-PositionCalculationAssistance is requested or not; - bit 2 indicates whether the field nr-IntegrityRiskParameters in IE NR-PositionCalculationAssistance is requested or not; - bit 3 indicates whether the field nr-IntegrityParametersTRP-LocationInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 4 indicates whether the field nr-IntegrityParametersDL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is requested or not. - bit 5 indicates whether the field nr-IntegrityParametersRTD-Info in IE NR-PositionCalculationAssistance is requested or not.

-------------------------------NEXT CHANGE-----------------------------------------------------------------------------

6.5.11.2	NR DL-AoD Assistance Data Request

–	NR-DL-AoD-RequestAssistanceData

The IE NR-DL-AoD-RequestAssistanceData is used by the target device to request assistance data from a location server.

-- ASN1START

NR-DL-AoD-RequestAssistanceData-r16 ::= SEQUENCE {

nr-PhysCellID-r16				NR-PhysCellID-r16						OPTIONAL,

nr-AdType-r16					BIT STRING {	dl-prs	(0),

posCalc (1) } (SIZE (1..8)),

...,

[[

nr-PosCalcAssistanceRequest-r17				BIT STRING {	trpLoc		(0),

beamInfo	(1),

rtdInfo		(2),

beamAntInfo	(3),

losNlosInfo	(4),

integrityBounds-r18 (5)

}	(SIZE (1..8))	OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Request-r17	ENUMERATED { eAoD, eAoA }		OPTIONAL,

nr-DL-PRS-BeamInfoRequest-r17				ENUMERATED { requested }		OPTIONAL,

nr-on-demand-DL-PRS-Request-r17				NR-On-Demand-DL-PRS-Request-r17	OPTIONAL,

pre-configured-AssistanceDataRequest-r17	ENUMERATED { true }				OPTIONAL,

]]

[[

nr-IntegrityAssistanceParaRequest-r18	BIT STRING {

riskPara		(0),

servicePara 	(1),

serviceAlert		(2),

integrityParaTRP-Loc	(3),

integrityParaBeamInfo	(4),

integrityParaRTD-Info	(5),

integrityParaBeamAntInfo(6)

}	(SIZE (1..8)) OPTIONAL

]]

}

-- ASN1STOP

NR-DL-AoD-RequestAssistanceData field descriptions

nr-PhysCellID This field specifies the NR physical cell identity of the current primary cell of the target device.

nr-AdType This field indicates the requested assistance data. dl-prs means requested assistance data is nr-DL-PRS-AssistanceData, posCalc means requested assistance data is nr-PositionCalculationAssistance for UE based positioning.

nr-PosCalcAssistanceRequest This field indicates the Position Calculation Assistance Data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is requested or not; - bit 3 indicates whether the field nr-TRP-BeamAntennaInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 4 indicates whether the field nr-DL-PRS-Expected-LOS-NLOS-Assistance in IE NR-PositionCalculationAssistance is requested or not. - bit 5 together with the bit 0 indicate whether the field integrityTRP-LocationBounds in IE NR-TRP-LocationInfo is requested or not. bit 5 together with the bit 1 indicate whether the field integrityBeamInfoBounds in IE NR-DL-PRS-BeamInfo is requested or not. bit 5 together with the bit 2 indicate whether the field integrityRTD-InfoBounds in IE NR-RTD-Info is requested or not. bit 5 together with the bit 3 indicate whether the field integrityBeamPowerBounds in IE NR-TRP-BeamAntennaInfo is requested or not. This field may only be present if the 'posCalc' bit in nr-AdType is set to value '1'.

nr-DL-PRS-ExpectedAoD-or-AoA-Request This field, if present, indicates that the IE NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData is requested. Enumerated value 'eAoD' indicates that expected AoD information is requested; value 'eAoA' indicates that expected AoA information is requested. This field may only be present if the 'dl-prs' bit in nr-AdType is set to value '1'.

nr-DL-PRS-BeamInfoRequest This field, if present, indicates that the IE NR-DL-PRS-BeamInfo is requested.

nr-on-demand-DL-PRS-Request This field indicates the on-demand DL-PRS requested for DL-AoD. This field may be included when the dl-prs bit in nr-AdType is set to value '1'.

pre-configured-AssistanceDataRequest This field, if present, indicates that the target device requests pre-configured assistance data with area validity.

nr-IntegrityAssistanceParaRequest This field indicates the Integrity Calculation Assistance Data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. - bit 0 indicates whether the field nr-IntegrityServiceParameters in IE NR-PositionCalculationAssistance is requested or not; - bit 1 indicates whether the field nr-IntegrityServiceAlert in IE NR-PositionCalculationAssistance is requested or not; - bit 2 indicates whether the field nr-IntegrityRiskParameters in IE NR-PositionCalculationAssistance is requested or not; - bit 3 indicates whether the field nr-IntegrityParametersTRP-LocationInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 4 indicates whether the field nr-IntegrityParametersDL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is requested or not. - bit 5 indicates whether the field nr-IntegrityParametersRTD-Info in IE NR-PositionCalculationAssistance is requested or not. - bit 6 indicates whether the field nr-IntegrityParametersTRP-BeamAntennaInfo in IE NR-PositionCalculationAssistance is requested or not.