3GPP TSG-RAN WG2 Meeting #103bis	R2-181xxxx

Chengdu, China, 8-12 August 2018

Title:	DRAFT Reply LS on RAN4 design on channel bandwidth

Response to:	R4-1811905 / R2-1813541

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	Qualcomm Incorporated

To:	RAN4

Cc:	RAN1

Contact Person:

#### Name:	Masato Kitazoe

E-mail Address:	mkitazoe [at] qti.qualcomm.com

Attachments: -

1. Overall Description:

RAN2 would like to thank RAN4 for their LS on RAN4 design on channel bandwidth.

RAN2 included already earlier a list of SCS-SpecificCarrier IEs into the cell-specific information that is provided in SIB1, upon PSCell/SCell addition and during handover. This structure provides the UE with the following information:

offsetToCarrier                     INTEGER (0..2199),

subcarrierSpacing                   SubcarrierSpacing,

carrierBandwidth                    INTEGER (1..maxNrofPhysicalResourceBlocks),

The above-mentioned fields are provided per subcarrier spacing to account for possible offsets among the PRB grids for different SCSs. The structure also supports cases where no such offset is present or where a cell uses only a single SCS.

These parameters were introduced based on RAN1 input [1] on RRC parameters as follows.

| Field in TS38.331 | “Parameter name” [1] | “Value range” [1] | Comment |
| --- | --- | --- | --- |
| offsetToCarrier | offset-pointA-low-scs | 0,1,2,3,..,275*8-1 | According to [1] - Offset between point A and the lowest subcarrier of the lowest usable PRB of a given SCS. - The offset is indicated in the unit of PRB based on the given SCS. |
| subcarrierSpacing | ref-scs | 15,30,60,120 | Reference subcarrier spacing based on which the offsetToCarrier is calculated. |
| carrierBandwidth | BW | 1,2,3,…275 | According to [1] - Channel BW of the carrier configured to the UE. - Include both UL and DL BW. - For Pcell DL in paired spectrum and Pcell DL & UL in unpaired spectrum, the above information is signaled to a UE is indicated in RMSI. - For Pcell UL in paired spectrum, the above information is indicated in RMSI and it’s also used to determine the frequency location of initial active UL BWP in paired spectrum. |

RAN2 would like to also inform RAN4 that BWP IE, in particular locationAndBandwitth field, can configure the bandwidth of each BWP of a serving cell. The first PRB is a PRB determined by subcarrierSpacing of the concerned BWP and offsetToCarrier as configured in SCS-SpecificCarrier IE mentioned above, RAN2 however understand that this is not the channel bandwidth parameter that RAN4 is seeking for.

BWP ::=                             SEQUENCE {

locationAndBandwidth                INTEGER (0..37949),

subcarrierSpacing                   SubcarrierSpacing,

cyclicPrefix                        ENUMERATED { extended }              OPTIONAL    -- Need R

}

RAN2 think the following two solutions are possible and would like to ask RAN4 to provide their feedback.

Solution 1:

As mentioned above, the existing structure is in the cell specific signaling branch (ServingCellConfigCommon / ServingCellConfigCommonSIB). Hence, it does not allow configuring selected UEs with UE specific channel bandwidth(s). To enable that, the same signalling structure can be introduced in the UE specific part of RRC configuration (ServingCellConfig) for UL and DL.

In this solution, RAN2 assumes that the channel bandwidth tables in 38-101-1 and in 38.101-2 provide a mapping from the carrierBandwidth (given in PRBs) to the corresponding channel bandwidth (in MHz).

Solution 2:

A new field indicating the channel BW value (e.g. defined as ChannelBandwidth below) can be introduced in the UE specific part of RRC configuration (ServingCellConfig) for UL and DL, directly or as a part of SCS-SpecificCarrier IE.

ChannelBandwidth::=	ENUMERATED	{5MHz,10MHz,20MHz,25MHz,30MHz,40MHz,50MHz,60MHz,80MHz,90MHz,100MHz,200MHz,400MHz, ...}

2. Actions:

To RAN4:

RAN2 would like to ask RAN4 to take the above information into account, and inform RAN2 which solution is preferred by RAN4.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #104		12 – 16 November 2018, Spokane, USA

TSG-RAN2 Meeting #105		25 February – 1 March 2019, Athens, Greece

4. Reference

[1]	R2-1801719 (R1-1801281)	LS on correction to RRC parameters for NR	RAN1