3GPP TSG-RAN WG2 Meeting #103bis	R2-181xxxx

Chengdu, China, 8-12 August 2018

Title:	DRAFT Reply LS on RAN4 design on channel bandwidth

Response to:	R4-1811905 / R2-1813541

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	Qualcomm Incorporated

To:	RAN4

Cc:	RAN1

Contact Person:

#### Name:	Masato Kitazoe

E-mail Address:	mkitazoe [at] qti.qualcomm.com

Attachments: -

1. Overall Description:

RAN2 would like to thank RAN4 for their LS on RAN4 design on channel bandwidth.

RAN2 would like to clarify that there are currently two types of bandwidth parameters provided to UE via RRC:

- Bandwidth indicated by carrierBandwidth per SCSin SCS-SpecificCarrier: cell specific parameter provided by dedicated signalling per serving cell as part of the IE ServingCellConfigCommon, and contains the same information that the cell broadcasts in SIB1 (if SIB1 is broadcast by the cell).

- Bandwidth indicated by locationAndBandwidth within BWP configuration of each serving cell:

  - For initial BWP, cell specific parameter provided by dedicated signallling as part of the IE ServingCellConfigCommon, and contains the same information that the cell broadcasts in SIB1 (if SIB1 is broadcast by the cell).

  - For other BWP, UE specific parameter provided by dedicated signaling as part of the IE ServingCellConfig.

  - The operating bandwidth of active BWP can be changed with dedicated signalling when UE is in RRC_CONNECTED.

| Field in TS38.331 | “Parameter name” [1] |  |  |
| --- | --- | --- | --- |
|  |  |  | According to [1] - Offset between point A and the lowest subcarrier of the lowest usable PRB of a given SCS. - The offset is indicated in the unit of PRB based on the given SCS. |
|  |  |  | Reference subcarrier spacing based on which the offsetToCarrier is calculated. |
|  |  |  | According to [1] - Channel BW of the carrier configured to the UE. - Include both UL and DL BW. - For Pcell DL in paired spectrum and Pcell DL & UL in unpaired spectrum, the above information is signaled to a UE is indicated in RMSI. - For Pcell UL in paired spectrum, the above information is indicated in RMSI and it’s also used to determine the frequency location of initial active UL BWP in paired spectrum. |

Further details of these parameters are given in the Annex within this LS below.

RAN2 would like to ask RAN4 whether the existing configurations mentioned above sufficiently address the RAN4 requirement. If the answer is ‘no’, then RAN2 would like to ask RAN4 to provide the detail of additional signaling and configurations that are necessary.

2. Actions:

To RAN4:

RAN2 would like to ask RAN4 whether the existing configurations mentioned above sufficiently address the RAN4 requirement. If the answer is ‘no’, then RAN2 would like to ask RAN4 to provide the detail of additional signaling and configurations that are necessary.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #104		12 – 16 November 2018, Spokane, USA

TSG-RAN2 Meeting #105		25 February – 1 March 2019, Athens, Greece

4. Reference

[1]	R2-1801719 (R1-1801281)	LS on correction to RRC parameters for NR	RAN1

5. Annex: Signalling details of BW configuration parameters in RRC

carrierBandwidth per SCS in SCS-SpecificCarrier

SCS-SpecificCarrier IE contains the following parameters.

offsetToCarrier                     INTEGER (0..2199),

subcarrierSpacing                   SubcarrierSpacing,

carrierBandwidth                    INTEGER (1..maxNrofPhysicalResourceBlocks),

The above-mentioned fields are provided per subcarrier spacing to account for possible offsets among the PRB grids for different SCSs. The structure also supports cases where no such offset is present or where a cell uses only a single SCS.

These parameters were introduced based on RAN1 input [1] on RRC parameters as follows.

| Field in TS38.331 | “Parameter name” [1] | “Value range” [1] | Comment |
| --- | --- | --- | --- |
| offsetToCarrier | offset-pointA-low-scs | 0,1,2,3,..,275*8-1 | According to [1] - Offset between point A and the lowest subcarrier of the lowest usable PRB of a given SCS. - The offset is indicated in the unit of PRB based on the given SCS. |
| subcarrierSpacing | ref-scs | 15,30,60,120 | Reference subcarrier spacing based on which the offsetToCarrier is calculated. |
| carrierBandwidth | BW | 1,2,3,…275 | According to [1] - Channel BW of the carrier configured to the UE. - Include both UL and DL BW. - For Pcell DL in paired spectrum and Pcell DL & UL in unpaired spectrum, the above information is signaled to a UE is indicated in RMSI. - For Pcell UL in paired spectrum, the above information is indicated in RMSI and it’s also used to determine the frequency location of initial active UL BWP in paired spectrum. |

BWP configuration

The BWP bandwidth information is provided via the following signallling structure:

BWP ::=                             SEQUENCE {

locationAndBandwidth                INTEGER (0..37949),

subcarrierSpacing                   SubcarrierSpacing,

cyclicPrefix                        ENUMERATED { extended }              OPTIONAL    -- Need R

}

These parameters were introduced based on RAN1 input [1] on RRC parameters as follows.

| Field in TS38.331 | “Parameter name” [1] | “Value range” [1] | Comment |
| --- | --- | --- | --- |
| locationAndBandwidth | DL-BWP-BW UL-BWP-BW | 1, 2, 3, … 2^16 | According to [1] The frequency position and applicable bandwidth for the configured DL/UL BWP • A DL (or UL) BWP is configured to a UE by resource allocation Type 1 with granularity as follows o Granularity of starting frequency location: 1 PRB o Granularity of bandwidth size: 1 PRB o Note: The above granularity doesn’t imply that a UE shall adapt its RF channel bandwidth accordingly |
| subcarrierSpacing | DL-BWP-mu UL-BWP-mu | 0,1, 2, 3, 4 or 5 | According to [1] • The subcarrier spacing of DL/UL BWP configuration is applied to at least PDCCH, PDSCH & corresponding DMRS. Table 4.1-2 in 38.211 |
| cyclicPrefix | DL-BWP-cp UL-BWP-cp | Normal CP, Extended CP | Configure the CP for the bandwidth part. Support CPs are normal CP (for all numerologies and slot formats), extended CP for 60 kHz numerology |