3GPP TSG-RAN WG2 Meeting #103bis                                                          R2-18xxxxx

Chengdu, China, Oct 8th – 12th, 2018

Agenda item:	11.1.2

Source: 		Qualcomm Incorporated

Title: 	Email discussion [103bis#31][NR - IAB] Adaptation layer in MT

Document for:	Discussion

# 1. Introduction

This document contains email discussion:

- [103bis#xx][NR - IAB] Adaptation layer in MT  (Qualcomm)

Intended outcome: TP

Deadline:  Thursday 2018-11-02

This email discussion is based on online discussion on adaptation layer in MT UP transport. Here are the chairman’s notes from the online discussion:

R2-1814999	Adaptation layer in IAB MT	Nokia, Nokia Shanghai Bell	discussion	Rel-16	FS_NR_IAB	R2-1812303

Adaptation layer is always present for all the traffic exchanged between IAB nodes, i.e. for both traffic of Access UEs as well as for traffic terminated and originating in IAB node.

-	Qualcomm indicates that we haven’t even discussed MT UP support.

R2-1814073	UP support for IAB node in arch 1a	Qualcomm Inc	discussion

-	Ericsson points out that GTP-U should be in Node 1.

=>	Fix the figures with GTP-U the Node 1

-	Ericsson thinks that Nokia’s proposal can go in the section of advantages/disadvantages of TR

=>	Include table with advantages/disadvantages and proposals from R2-1814999 in the TP.

=>	Capture in the TP Protocol stack with adaptation layer and without adaptation layer

# 2. Discussion

Contribution R2-1814073 by Qualcomm contains a TP with protocol stacks for the MT’s UP transport. This TP aims to keep FFS if the MT’s access link holds the adapt layer. The protocol stacks therefore show the adaptation layer on the MT’s access link with dashed lines.

During online session, it was observed that some of the figures need corrections. It was also proposed to show separate stacks for each case, i.e. MT UP access link with and without Adapt layer.

Section 5 of this document holds a TP with the corrected protocol stacks, one set of stacks showing MT’s UP access link with Adapt layer and the other without. The TP below further holds original text from R2-1814073, which compares MT access with and without Adapt layer.

The TP below further holds a protocol stack for UE-based UP traffic using architecture 1b (Figure 8.2.2-2). This figure corrects TR 38.874 v0.50 in that it changes the dashed lines to solid lines for the adapt layer on the upstream link of the UE’s parent node.

Contribution R2-184999 by Nokia contains the following discussion and proposal (without TP):

This aspect was shortly discussed, but not concluded during the e-mail discussion mentioned above. In the table below, we summarize pros and cons of having adaptation layer always included in the serving IAB node, some of which were also indicated during the course of the aforementioned e-mail discussion:

| Adaptation layer present in the serving IAB node | Adaptation layer not present in the serving IAB node |
| --- | --- |
| - All traffic destined to the IAB node can use the same routing principles based on IAB node identifier in the adaptation layer header - Logical channel space is not decreased due to separation of traffic terminated in or originating from the IAB node - Same processing rules are used for all the packets arriving to the MT part of the IAB node (both terminated in IAB node and the ones destined for Access UEs or next hop IAB node) - More overhead on the last hop due to adaptation layer existence for traffic generated by or terminated in the IAB node | - Traffic terminated in the IAB node needs to be routed differently compared to traffic, which is to be relayed to the Access UEs connected to this IAB node - Separate logical channel needs to be assigned for traffic terminated in the IAB node, which reduces the number of logical channels available for QoS differentiation - Different processing rules are used for all the packets arriving to the MT part of the IAB node (both terminated in IAB node and the ones destined for Access UEs or next hop IAB node) - Reduced overhead on the last hop |

It can be seen that the advantage of omitting adaptation layer is reduced overhead of the traffic terminated or originating in the IAB node. However, it comes at the expense of complications in terms of packet processing and routing by the IAB node as well as reduced logical channel ID space, which limits flexibility in terms of QoS differentiation. Since the number of packets terminated/originating in the IAB nodes is expected to be very small compared to the one related to Access UEs, we think the gains of overhead reduction will be negligible, so in order to avoid issues brought by omitting adaptation layer for traffic terminated/originating in IAB nodes, we propose the following:

Proposal 1: Adaptation layer is always present for all the traffic exchanged between IAB nodes, i.e. for both traffic of Access UEs as well as for traffic terminated and originating in IAB node.

Using comparisons from R2-1814073 and the table from R2-184999 as a starting point, this email discussion aims to establish a consolidated table in the TP with a comparison between MT’s UP access link with vs. without adaptation layer.

The following text captures comments from the email discussion:

Comments:

- LG thinks that Adapt on MT’s access link for UP option d) should not include GTP-U and for UP option e), it should not include GTP-U/IP.

- Rapporteur (QC) believes that these headers are integral part of Adapt. Otherwise, there would be two different solutions of Adapt.

- Rapporteur (QC) thinks that bullet 1 in Nokia’s table is technically incorrect. If the same routing rules are applied for UE and MT traffic, the routing of the MT’s access traffic is terminated at the MT’s parent node. In fact, it would not be obvious what routing addresses the Adapt layer would carry on the MT’s access link.

# 3. Conclusion

…

# 5. Text Proposal for TR 38.874

The following changes to TR 38.874 are proposed:

********* Start of Change **********

# 8	Radio protocol aspects

Editor’s note:	Primary responsible WG for this clause is RAN2.

## 8.1	Packet Processing

## 8.2 	User-plane considerations for architecture group 1

### 8.2.1 	General

…

### 8.2.2 	Adaptation Layer

…

Figure 8.2.2 - 2: Protocol stack example for UE-access using L2-relaying with adaptation layer for architecture

…

### 8.2.3 	Multi-hop RLC ARQ

…

### 8.2.x 	UP support of IAB-node

Figure 8.2.x-1: Protocol stack examples for MT-access using L2-relaying with adaptation layer for architecture 1a

Figure 8.2.x-2: Protocol stack examples for MT-access using L2-relaying with adaptation layer for architecture 1b

Figure 8.2.x-3: Protocol stack examples for MT-access using L2-relaying without adaptation layer for architecture 1a

Figure 8.2.x-4: Protocol stack examples for MT-access using L2-relaying without adaptation layer for architecture 1b

The MT on the IAB-node also sustains an access link, which is used by the IAB-node for its own traffic, e.g., for OAM support. Figures 8.2.x-1, -2, -3 and -4 show examples of protocol stacks for MT-access, which are analogue to those for UE-access.

In Figures 8.2.x-1 and -2, the adaptation layer is included on RLC-channels the MT uses for its own access traffic. In Figures 8.2.x-3 and -4, the adaptation layer is not included on RLC-channels the MT uses for its own access traffic.

Table 8.2.x-1 shows comparisons between using vs. not using the adaptation layer on the MT-access link.

Table 8.2.x-1: Comparison between using vs. not using adaptation layer on MT’s access link

| Adaptation layer present on MT’s access link | Adaptation layer not present on MT’s access link |
| --- | --- |
| - Adapt layer can be used to multiplex MT access bearers and BH traffic onto the same logical channel. Logical channel space is not decreased due to separation of MT traffic and BH traffic. | - Separate logical channel needs to be assigned for MT traffic, which reduces the number of logical channels available for BH traffic. |
| - Same processing rules are used for MT’s access traffic and BH traffic. | - Different processing rules are used for MT’s access traffic than for BH traffic. |
| - More overhead on last hop of MT’s access traffic. | - Reduced overhead on last hop of MT’s access traffic. |

## Control-plane considerations for architecture group 1

…

********* End of Change **********