3GPP TSG-RAN WG2 Meeting #105bis                                                          R2-19xxxxx

Xi’an, China, April 8th – 12th, 2019

Agenda item:	11.1.1

Source: 		Ericsson (Rapporteur)

Title: 	Email discussion [105#47] [NR_IAB-Core] Bearer Mapping

Document for:	Discussion

# Introduction

This document contains email discussion:

R2-1902645	Email Discussions 		Qualcomm Inc

discussion

- 	Huawei think that Adapt entities should be discussed separately.

- 	Chair proposes to remove the part on entities

- [105#47] [NR_IAB-Core] Bearer mapping (Ericsson)

Intended outcome: a report to identify options and possible “easy” agreements.

Deadline:  Thursday 2019-03-28

Discussion:

The TR defines the term “bearer mapping” as “UE-bearer to RLC-channel mapping” but this is too restrictive since it does not apply to F1-C. Further, F1-C messages of different type might also have different priorities and therefore use different RLC channels. This raises the following questions:

- What are the criteria we want to allow for the mapping of an Adapt PDU to a BH RLC channel? (e.g. QoS, UP vs CP, PDU session, F1 connection, others).

- What information must be available at the access-IAB-node to enable this mapping in upstream direction? (e.g. mapping table with (F1-connection Id, BH LCID)-pairs).

- Which of these criteria require a re-mapping on intermediate IAB-nodes?

- What information must be available at the intermediate-IAB-node to enable this re-mapping? (e.g. none if remapping is not required).

- What information needs to be carried in the adapt header to enable the remapping on the intermediate-IAB-node?

# 2. Discussion

The purpose of this email discussion is to apprehend different options for bearer mapping in an IAB network and identify common themes on this topic to be presented in RAN2#105bis. To make this discussion more productive, we have rephrased the above questions by bringing up explicitly the underlying issues of mapping in Donor DU, intermediate-IAB-node and Access-IAB-node. In addition, to provide companies the opportunity to express their point of view on how these issues can be tackled for N:1 and 1:1 mapping, as well as the mapping aspect of F1-C and OAM traffic, we have included three sets of questions.

It should be noted that we have not included the case of the DL mapping at the Access-IAB-node and UL mapping at the Donor DU because there is no BH RLC channel at the next hop and we can rely on F1/IP mechanisms.

Companies are invited to share their point of view, specifically about what information is used/needed for the mapping in each case, how the node(s) obtain the information, whether the information is provided via configuration or inband (e.g. Adaptation/IP header), and motivation behind remapping at the intermediate-IAB-node(s).

In the following, two sets of questions one for each type of mapping are provided, covering the bearer mapping issues raised in R2-1902645:

Question 1: N:1 mapping UP

1a: How is the mapping performed for the UL at the Access-IAB-node?

| ## Company | ## Comments |
| --- | --- |
| ## Qualcomm | ## Based on F1-U GTP TEID |

Summary:

1b: How is the mapping performed for the DL at the Donor DU?

| ## Company | ## Comments |
| --- | --- |
| ## Qualcomm | Upfront: If RAN2 wants to have a new transport format on the wireline network, they need to get RAN3 involved. Same applies if RAN2 wants E2E security protection of F1 to be broken at the Donor DU. QC does not want to see a new transport format for IAB support on wireline network, or split up of E2E security layer for F1. We propose: - Derive mapping from IPv6 flow label and DSCP on IP header. - For IPv4, only DSCP is available. If operator wants to support fine-granular QoS, they can deploy IPv6. |

Summary:

1c: How is the mapping performed for the UL/DL at the intermediate-IAB-node(s)?

| ## Company | ## Comments |
| --- | --- |
| ## Qualcomm | Based on 1:1 mapping between ingress and egress RLC channels. There is no need for a “remapping”. We don’t see why the reasons for the bearer mapping, e.g. such as bearer’s QoS, PDU session, slice, etc., would change on the intermediate IAB-node. |

Summary:

1d: Additional comments/aspects related to N:1 mapping?

| ## Company | ## Comments |
| --- | --- |
| ## Qualcomm | There should be no difference between N:1 and 1:1 bearer mapping. 1:1 bearer mapping should simply be a special case of N:1 mapping with N=1. |

Summary:

Question 2: 1:1 mapping UP

2a: How is the mapping performed for the UL at the Access-IAB-node?

| ## Company | ## Comments |
| --- | --- |
| ## Qualcomm | Same as for N:1 bearer mapping: Based on GTP-U TEID. |

Summary:

2b: How is the mapping performed for the DL at the Donor DU?

| ## Company | ## Comments |
| --- | --- |
| ## Qualcomm | Same as for N:1 bearer mapping. - Based on DSCP and IPv6 flow label. - If operators wish to perform fine granular 1:1 mapping they can deploy IPv6. |

Summary:

2c: How is the mapping performed for the UL/DL at the intermediate-IAB-node(s)?

| ## Company | ## Comments |
| --- | --- |
| ## Qualcomm | Same as for N:1 bearer mapping: Based on 1:1 mapping between ingress and egress RLC channels. There is no need for a “remapping”. |

Summary:

2d: Additional comments/aspects regarding 1:1 mapping?

| ## Company | ## Comments |
| --- | --- |
| ## Qualcomm | Should be same as for N:1 mapping with N=1. |

Summary:

Question 3: Mapping of F1-C and OAM

3a: How is the mapping performed for the UL at the Access-IAB-node?

| ## Company | ## Comments |
| --- | --- |
| ## Qualcomm | ## F1-C should be provided with a separate RLC channel. ## OAM is essentially UP traffic and could be aggregated with mobile UP traffic. De-multiplexing is no issue since OAM traffic and mobile UP traffic use different dst IP addresses in upstream direction. |

Summary:

3b: How is the mapping performed for the DL at the Donor DU?

| ## Company | ## Comments |
| --- | --- |
| ## Qualcomm | ## F1-C PDUs should be marked with unique DSCP on wireline network, so that F1-C priority can also be achieved for IPv4 deployments. OAM can use same marking as mobile UP traffic. Demultiplexing in downstream direction will be based on L4 (i.e. UDP with F1-U port number for mobile UP traffic vs. TCP connection for OAM traffic) |

Summary:

3c: How is the mapping performed for the UL/DL at the intermediate-IAB-node(s)?

| ## Company | ## Comments |
| --- | --- |
| ## Qualcomm | Same as UP: There should be 1:1 mapping between ingress and egress RLC channels. |

Summary:

3d: Additional comments/aspects regarding F1-C/OAM mapping?

| ## Company | ## Comments |
| --- | --- |
|  | CP and UP should use the same principles. Further, F1-C should be aggregated onto same BH RLC channel. OAM can be aggregated with BE UP traffic. |

Summary:

# 3. Summary