3GPP TSG-RAN WG2 Meeting #110 electronic	R2-200xxxx

1st – 12th June, 2020

Agenda item:       6.1.3

Source: 	Huawei

Title: 	Summary of email discussion [Post109bis-e][019][IAB] BAP

Document for:		Discussion and Decision

- 1.	Introduction

This is a summary of the following email discussion:

- [Post109bis-e][019][IAB] BAP (Huawei)

Scope: Update running CR with latest RAN3 agreement on bearer mapping, DL mapping, UL mapping. Including discussion on terminology/notation on the various identifiers to sync up between RAN2 and RAN3 TSs. Review for issue identification and corrections. 
Intended outcome: Agreeable CR update, Report. 
Deadline : Next meeting.

The draft CR is provided in a separate Tdoc, and this document is only for issue identification and handling.

- 2. Discussion

- 2.1	Using default configuration during IAB migration

In RAN3 LS R3-202851, RAN3 asks RAN2 to take the following RAN3 agreement into account:

Donor CU configures a default UL BAP routing ID and a default BH RLC channel to migrating IAB node in order to configure UL mapping for F1-C traffic on the target path, e.g. IKE handshake, SCTP chunks, during the handover preparation procedure.

This agreement may concern the following procedure in TS 38.340.

##### 5.2.1.2.1 BAP routing identity selection at IAB-node <text removed here> At the IAB-node, for a BAP SDU received from upper layers and to be transmitted in upstream direction, the BAP entity shall: - if the Uplink Traffic to Routing ID Mapping Configuration is not configured in accordance with TS 38.473 [5]: - select the BAP address and the BAP path identity as configured by defaultUL-BAProutingID in TS 38.331 [3]; - else: - for the BAP SDU encapsulating an F1-U packet: - select an entry from the Uplink Traffic to Routing ID Mapping Configuration with its traffic type specifier corresponds to the destination IP address and TEID of this BAP SDU; - for the BAP SDU encapsulating a non-F1-U packet: - select an entry from the Uplink Traffic to Routing ID Mapping Configuration with its traffic type specifier corresponds to the traffic type of this BAP SDU; - select the BAP address and the BAP path identity from the BAP routing ID in the entry selected above; 5.2.1.4.2 Mapping to BH RLC Channel for BAP SDUs from upper layers at IAB –node For a BAP SDU received from upper layers at the IAB-node and to be transmitted in upstream direction, whose egress link has been selected as specified in clause 5.2.1.3, the BAP entity shall: - if the Uplink Traffic to BH RLC Channel Mapping Configuration is not configured in accordance with TS 38.473 [5]: - select the egress BH RLC channel corresponding to defaultUL-BH-RLC-Channel configured in TS 38.331 [3]; - else: - for the BAP SDU encapsulating an F1-U packet: - if there is an entry in the Uplink Traffic to BH RLC Channel Mapping Configuration with its traffic type specifier corresponds to the destination IP address and TEID of this BAP SDU and its egress link ID corresponding to the selected egress link; - select the egress BH RLC channel corresponding to [egressBH-RLC-ID] of the this entry; - else: - select any egress BH RLC channel on the selected egress link; - for the BAP SDU encapsulating non-F1-U packet: - if there is an entry from the Uplink Traffic to BH RLC Channel Mapping Configuration with its traffic type specifier corresponds to the traffic type of this BAP SDU and its egress link ID corresponding to the selected egress link; - select the egress BH RLC channel corresponding to [egressBH-RLC-ID] of this entry; - else: - select any egress BH RLC channel on the selected egress link;

In the existing procedure in BAP, as long as the F1AP configuration for the Uplink Traffic to Routing ID Mapping and the Uplink Traffic to BH RLC Channel Mapping are configured, the IAB node will not use the default configuration of routing ID and BH RLC channel for F1-C and non-F1 transmission. It means that the default configuration is only used in the very early stage, e.g. when the IAB node starts up. During IAB node handover, although the RRC configuration of the IAB-MT, including the default routing ID and BH RLC channel, has been updated according to the target node in Handover Command message, the F1AP configuration for routing ID and BH RLC channel are still in place according to the source node. The F1AP configuration can only be updated by the donor-CU after the IAB-MT completes the handover procedure.

Moreover, it should be noted that IAB node migration may not only refer to the case of handover. It is the similar situation when IAB nodes re-establish or resume on another cell than the previous serving cell, and the default configuration of Routing ID and BH RLC channel as configured in RRC Reconfiguration message should also be applied before the F1AP configuration updated.

In order to use the default configuration for Routing ID and BH RLC channel as configured in RRC during IAB migration (including handover, RRC re-establishment and RRC resume), there are at least the following option(s):

- Option-1: the old BAP configuration configured by F1AP should be released, and then the default configuration configured by RRC would be applied in BAP, upon receiving the RRC reconfiguration message in HO command or after RRC resume/RRC re-establishment. Note that to be simple, this option is to release all F1AP configuration for BAP.

- (Other solutions to be added).

Q1) Which option do you prefer to enable the default configuration for routing ID and BH RLC channel for F1-C and non-F1 transmission during IAB migration?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |

- 2.2	Other open issues to be discussed

Companies are welcome to provide issues in the following table:

| Company | Issues and solutions |
| --- | --- |

- 3. Conclusion

TBD.