3GPP TSG-RAN WG2 Meeting #110 electronic				R2-200xxxx

Elbonia, June 1st  – 12nd  2020

Agenda item:		x.x.x

Source:			Intel Corporation

Title:		[109b#930] UE capabilities for NR mobility

Document for:	 	Discussion and decision

# Introduction

This is the email discussion report on below email discussion:

- [Post109bis-e][930][NR MOB] UE capabilities for NR mobility (Intel)

Intended outcome: Discuss remaining issues with UE capabilities for NR mobility based on RAN1 input and updates from RAN2#109bis-e (if any)

Deadline: Long (until next meeting)

This email discussion only focus on RAN2 features and is split into 2 phases:

Phase 1: companies to provide the comments on the discussion paper and the draft CRs;

Deadline: 2020-05-15 23:59 PST;

Phase 2: companies to provide the comments on the summary of the discussion paper and the updated CRs; Deadline: 2020-05-20 23:59 PST;

# Discussion

RAN2 made following agreements on capabilities:

- 1: Agree the capabilities (x1-1, x1-3, x2, x3) including the revisions as indicated in the table for NR. May be revised based on further inputs

- 2: Agree the capabilities (x1-1, x1-3) including the revisions as indicated in the table for LTE. May be revised based on further inputs

- 3: Intra freq DAPS can be supported for bandwidthClass B/C and above UE (e.g. bandwidthClass B/C UE, the UE supports intraF DAPS with bandwidth class A for the band against source and target). The capability intra-FreqDAPS is put under bandParameter.

- 4: For inter freq DAPS, the capability inter-FreqDAPS is specified per BC (for intra band, inter band cases). .It  is put under existing CA bandcombiantion, and same as CA, the CCs in the bandcombination with UL can all be source or target PCell.

- 5: Below RAN4 capabilities are introduced as baseline, and may be revised if more inputs are received from RAN4.

•	Per BC: AsyncDAPS, supportedNumberTAG, singleUL-Transmission;

•	Per Band per BC: intraBandDiffSCS, intraFreq-DAPS;

- 6: Reuse CA capability supportedNumberTAG for DAPS handover.

- 7: Per Band per BC capability (intraBandDiffSCS, intraFreq-DAPS) is put in BandParameters.

- 8: Baseline is UplinkPowerSharingDAPS-HO, pdcch-BlindDetectionMCG1-UE and pdcch-BlindDetectionMCG2-UE are introduced as per BC capabilities. May  be revised if more inputs are received from RAN1., .e.g. on whether pdcch-BlindDetectionMCG1-UE and pdcch-BlindDetectionMCG2-UE can be per UE as DC.

- 9: pdcch-BlindDetectionSource, pdcch-BlindDetectionTarget and supportedNumberTAG(>=2) are mandatory with capability for DAPS capable UE. FFS on intraFreqDAPS, uplinkPowerSharing and Sync-DAPS.

- 10: As agreed in RAN4, intraBandDiffSCS is different for intra/inter DAPS, i.e. separate capability.

- 11: Agree the ASN.1 part in section 7 as baseline for LTE and NR.

- 12: Source decides source configuration to be used during DAPS handover and the restriction in target; target determines the target configuration based on received source configuration to be used during DAPS HO and UE capabilities, and generates the DAPS handover command and the target node sends the DAPS handover command to the source node in the X2/Xn HANDOVER REQUEST ACKNOWLEDGE which transparently forwards it to the UE.

- 13: Same as legacy reconfiguration procedure, modification of target configuration can be sent in the same message for source release;

- 15 For LTE, the DAPS network coordination is based on source link configuration to be used during DAPS HO, UE capabilities, maxSCH-TB-BitsDL, maxSCH-TB-BitsUL, powerCoordinationInfo within HandoverPreparationInformation message;

- 16:For NR, the DAPS network coordination is based on source link configuration to be used during DAPS HO, UE capabilities, maxSCH-TB-BitsDL (to be redefined for NR), maxSCH-TB-BitsUL (to be redefined for NR), powerCoordinationInfo within HandoverPreparationInformation message; To discuss whether  coordination on maxSCH-TB-BitsDL, maxSCH-TB-BitsUL are needed.

- 19: SCells are released in HO command, and not configured in HO command. To clarify this in UE capability,i.e. intra/inter-F DAPS capability indicates that the UE can only do DAPS handover with source and target PCell and no SCells. There should no other specification impact.

RAN2 features related agreements are highlighted in red. In addition, there were some contributions on CHO, CPC and T312 capabilities.

In Summary:

CHO:  per UE, FDD/TDD diff, FR1/FR2 diff;

- Handling of CHO configuration, including execution condition and candidate cell configuration; Agreed in RAN2-109

Agree: Ericsson [4]

- FFS Maximum candidate cells the UE can support; Default value FFS

No: LG [3],

- CHO based failure handling;							Agreed in RAN2-109

Not needed: Ericsson [4]

- FFS support of 2 trigger events for one execution condition

Yes: LG[3],

T312: per UE, FDD/TDD diff, FR1/FR2 diff;

- T312 based PCell fast failure recovery		Agreed in RAN2-109

- T312 based PSCell fast failure recovery		Agreed in RAN2-109

Rapporteur would like to check companies’ view again on the open issues above and would like to avoid repeating the discussion on the capabilities those we already agreed.

Question 1: For CHO, do companies see the need to introduce a capability on maximum candidate cells the UE can support? If yes, pls indicate what default value should be.

| Company | Yes/No | Remark |
| --- | --- | --- |
| Nokia | No | If RRC defines the maximum number of CHO/CPC candidate cells then we can implicitly assume the UE which is CHO/CPC capable, can support up to this amount of CHO/CPC candidates. The actual CHO procedures are also no different regardless of how many candidates are configured. We would also be fine with having “1” as baseline and then “8” as optional capability. |
| Qualcomm | No | The current maximum value of 8 cells is acceptable as baseline CHO capability. |
| Intel | No | Do not see the need to have such additional capability |

Question 2:For CHO, do companies see the need to introduce a capability on support of 2 trigger events for one execution condition?

| Company | Yes/No | Remark |
| --- | --- | --- |
| Nokia | Yes | Two triggering events for one execution condition is not a default CHO operation, so in our opinion this shall be subject to a separate capability or IOT bit. This also allows the basic CHO to progress even if the IODT for two execution conditions never appears. |
| Qualcomm | Yes | This is a new feature (not used in LTE or NR before) introduced only for CHO and it is an optimization so should be a UE capability. |
| Intel |  | No strong opinion. It is not complex to support it. But tend to agree, it is also related to whether there is IODT test case or not. |

CPC (did not discuss before):

- Handling of CPC configuration, including execution condition and candidate cell configuration;

Yes: LG [6], Ericsson [7]

- Maximum candidate cells the UE can support; Default value FFS

No: LG [6], Ericsson [7]

- CPC based failure handling;=> not applied for CPC based on agreements;					No: LG [6], Ericsson [7]

- support of 2 trigger events for one execution condition

Yes: LG[6],

No: Ericsson [7]

Question 3: For CPC, do companies agree that same as CHO (cho-r16), capability on handling of CPC configuration is needed, i.e. cpc-r16?

| Company | Yes/No | Remark |
| --- | --- | --- |
| Nokia | Yes | The way this question is formulated is a bit vague, but we support having a separate capability for CPC. |
| Qualcomm | Yes | Assuming the intention is to have separate capabilities of CHO and CPC. |
| Intel | Yes |  |

Question 4: For CPC, do companies see the need to introduce a capability on maximum candidate cells the UE can support? If yes, pls indicate what default value should be.

| Company | Yes/No | Remark |
| --- | --- | --- |
| Nokia | No | Same as in Q1. Supporting CPC means the UE supports up to max number of CPC candidates. |
| Qualcomm | No | Same as in Q1. |
| Intel | No |  |

Question 5: For CPC, do companies see the need to introduce a capability on support of 2 trigger events for one execution condition?

| Company | Yes/No | Remark |
| --- | --- | --- |
| Nokia | Yes | Same reasoning as for Q2. |
| Qualcomm | Yes | Same as Q2 |
| Intel |  | Same as Q2 |

Question 6: Any other issues?

| Company | Remark |
| --- | --- |

# Summary

To be added:

# References

[1] R2-2003368	UE Capability for Rel-16 NR mobility enhancement	Intel Corporation

[2] R2-2003369	UE Capability for Rel-16 NR mobility enhancement	Intel Corporation

[3] R2-2002902	Consideration on CHO capability	LG Electronics Inc.

[4] R2-2003037	UE capabilities for CHO	Ericsson

[5] R2-2003579	Discussion on UE capabilities for CHO and T312	Huawei, HiSilicon

[6] R2-2002904	Consideration on CPC capability	LG Electronics Inc.

[7] R2-2003039	UE capabilities for conditional PSCell change	Ericsson

[8] R2-2003581	Discussion on UE capabilities for CPC	Huawei, HiSilicon

[9] R2-2002592	Inter-node signalling for DAPS handover	Ericsson

[10]R2-2002905	Consideration on DAPS Capability	LG Electronics Inc.

[11] R2-2003047	Discussion on open issues for UE capability coordination	Huawei, HiSilicon

[12] R2-2003367	Discussion on  capabilities for MOB	Intel Corporation

[13] R2-2001727	Report of [AT109e][211][MOB] UE capabilities for DAPS and CHO (Intel)	Intel Corporation