3GPP TSG-RAN WG2 Meeting#110-e	Draft_R2-2005610

Online, 1st -12th Jun. 2020

Agenda Item:	6.11.2

Source:	Huawei

Title:	Report of email discussion [Post109bis-e][938][PowSav] MAC open issues

Document for:	Discussion and Decision

# Introduction

This document is for the following email discussion on the remaining MAC open issues:

- [Post109bis-e][938][PowSav] MAC open issues (Huawei)

Address stage-3 remaining open issues. Capture identified NEW, if any, stage-3 corrections/issues from other companies.  Issues that have already been discussed and not pursued should not be brought up again.

Intended outcome: CR for 38.321 addressing open issues (including editorials received offline)

Deadline: Wednesday May 20th 23.59 PST

# Discussion

## RAN1 LS

In the RAN2#109-e, one LS has been sent to RAN1 to inform RAN2 understanding on MAC-PHY interactions for DCP [1]. In the RAN1#100bis-e, the following RAN1 reply LS has been approved to inform the following RAN1 understanding [2]:

L1 procedure of DCI format 2_6 detection - Successful decoding of DCI format 2_6 - L1 sends a positive indication to MAC when the value of wakeup indication bit is “1” - L1 sends a negative indication to MAC when the value of wakeup indication bit is “0” - Miss-detection - all CRC checks fails on DCI format 2_6 - L1 does not send any indication to MAC - All invalid monitoring occasions – following legacy behavior to wake up - L1 sends a positive indication to MAC Text proposal of CR to TS38.213 of the above physical layer procedure were also agreed.

Whether the endorsed MAC CR needs to be updated according to the RAN1 LS needs to be discussed.

Discussion point 1. Is there any impact on the endorsed MAC CR due to the reply LS from RAN1?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| CATT | Yes | We think the MAC CR wording can be updated to better match the latest RAN1 spec as follows: 3> if DCP indication associated with the current DRX Cycle received from lower layer was a positive indicationindicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or |
| Qualcomm | No | We do not think the change proposed by CATT is necessary. It is better to have a more explicit text on what UE’s behavior should be (e.g. start on duration timer), instead of a rather generic word “positive/negative”. In our view, “positive” or “negative” is more of an internal modelling issue. |
| LG | No | We have similar understanding with Qualcomm. There is no need to change the current text. |
| Xiaomi | Yes? | According to the LS, the physical layer procedure regarding to invalid DCP occasions was changed to: An indication will be received from PHY. In our view, it seems that the second bullet 3 can be merged to the previous bullet. Because MAC does not need to care about whether the indication is triggered by a successful decoding of DCP or invalid monitoring occasions. 2> if DCP is configured for the active DL BWP: 3> if DCP indication associated with the current DRX Cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or 3> if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX Cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or within BWP switching interruption length, or during a measurement gap; or 3> if ps-Wakeup is configured with value true and DCP indication associated with the current DRX Cycle has not been received from lower layers: 4> start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe. |
| Nokia, Nokia Shanghai Bell | No | We agree with Qualcomm |
| Intel | Partially | For the case of “Successful decoding of DCI format 2_6”, we agree with that MAC spec should focus on whether UE starts (or not) the on duration timer (as it is already captured in current CR). For case of “All invalid monitoring occasions – following legacy behavior to wake up”, RAN2 IF condition does not seem to cover all invalid cases described in 38.213, with reference copied below to clauses 10, 11.1, 12 (as MAC IF condition mainly addresses the overlapping with active time, BWP switching and measurement gap): If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE - is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [14, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next DRX cycle, or Therefore, we suggest adding a reference to the MO invalid cases described in RAN1 TS e.g. with the following TP change: 3> if DCP indication associated with the current DRX Cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or 3> if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX Cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or within BWP switching interruption length, or during a measurement gap; or 3> if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX Cycle are invalid; or 3> if ps-Wakeup is configured with value true and DCP indication associated with the current DRX Cycle has not been received from lower layers: 4> start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe. |
| ZTE | No | We tend to share the same view with Qualcomm |
| Ericsson | No | We think it is rather clear when lower layers indicate to start the drx-OnDurationTimer, i.e. no further clarifications are needed: no-Wake-up indication value ‘0’ to higher layers Wake-up indication value ‘1’ to higher layers But based on the comment from Intel above, we have further comments/questions. We had a preference from the beginning for a stricter separation of what to capture in RAN1 and RAN2 spec, i.e. PDCCH monitoring aspects only in RAN1. We think the current status is confusing, and not clear: - PHY is now referring to MAC when to send a wake-up indication to MAC : “as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [14, TS 38.321]”? - When PHY talks about “UE is not required to monitor PDCCH for detection of DCI format 2_6” (aka invalid DCP occasion) it is not clear that this refers to “if all DCP occasion(s) in time domain, as specified in TS 38.213 [6]” in 38.321? 38.213: If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [14, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next DRX cycle, or does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next DRX cycle the UE shall send the Wake-up indication value “1” to higher layers start the drx-onDurationTimer for the next DRX cycle. 38.321: 3> if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX Cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or within BWP switching interruption length, or during a measurement gap; or |
| OPPO | No | There is no need to update the MAC procedure based on RAN1 LS. |
| Convida Wireless | No | We agree with the views of Qualcomm that it is better to have more explicit text on what the UE’s behavior should be. |
| Apple | No | Agree with Qualcomm to have a more explicit text to clarify on the UE behaviour. |
| vivo | Partially Yes | According to the RAN1 LS and TP, positive or negative indication to MAC is defined in PHY spec, it is better to clearly align with RAN1 specification, i.e. MAC starts drx-onDurationTimer if DCP indication associated with the current DRX Cycle with the value of “1” is received from lower layer. If the majority companies want to keep the current MAC specification, we are also OK that lower layers indicate to start the drx-OnDurationTimer. For safe, we would better to reply LS to RAN1 or companies can bring RAN1 CR to update the wording for RAN1 specification. But for DCP MO invalid cases, RAN1's latest TP for TS38.213 is as followings: If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE - is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [14, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next DRX cycle, or - does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next DRX cycle the UE shall send the Wake-up indication value “1” to higher layers start the drx-onDurationTimer for the next DRX cycle. From our understanding, one part of DCP MO invalid cases can be handled in PHY via sending the wake-up indication value "1" to higher layers, and the rest of DCP MO invalid cases are handled in MAC: 3> if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX Cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or within BWP switching interruption length, or during a measurement gap; or With latest TS38.213 and TS38.321, UE can work correctly. But there are some overlaps conditions between PHY and MAC. It is also better to update the description to avoid cross reference. |
| Huawei, HiSilicon | No | Regarding “positive/negative” indication in the RAN1 LS, we share the same view as above that the current MAC CR is clear enough and “indicated to start drx-onDurationTimer” is better wording used in TS38.321. For Xiaomi’s comment: RAN1 specification does not capture the relation between Active Time and the ambiguous period, i.e. “4 ms prior to…” is not considered in TS38.213. Thus we think the removed bullet 3> is needed For Intel and Ericsson comments: We agree that the current MAC CR does not cover all invalided case, i.e. Clauses 10, 11.1, 12. But since those clauses are all RAN1 invalid cases, we think those cases should be handled in PHY spec as the agreed modelling in R2-2002201: PHY specification: - PHY specifies DCP monitoring, including: - When to start the monitoring (ps_offset) and stop the monitoring (minimum gap based on UE capability) - In case DCP is considered invalid from PHY perspective (scenarios FFS in RAN1), PHY should not monitor DCP and indicates to MAC to start the drx-onDurationTimer for the next DRX cycle For those invalid cases, as in RAN1 LS, a positive indication will be sent to MAC thus the cases can be covered by the following existing “if”: 3> if DCP indication associated with the current DRX Cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or |

## Other remaining issues

In the last RAN2#109bis-e meeting [3], it seems all MAC open issues in the submitted documents have been discussed / addressed. The only remaining issue is the collision between DCP and RAR window. One LS has been sent to RAN1 to confirm RAN2 understanding that RAR addressed all RNTIs is prioritized over DCP by the UE and ask where to capture the behaviour [4]. An Editor’s Note has been captured in the endorsed MAC CR [5]. Our understanding is that we need to wait for RAN1 reply before further discuss this issue.

Companies are invited to raise new MAC remaining issues (including editorials) based on the endorsed MAC CR [5], if any.

Discussion point 2. Companies are invited to raise new MAC remaining issues (including editorials) based on the endorsed MAC CR [5], if any.

| Company name | Open issues | Comments |
| --- | --- | --- |

# Summary

13 companies participated the email discussion: CATT, Qualcomm, LG, Xiaomi, Nokia/Nokia Shanghai Bell, Intel, ZTE, Ericsson, OPPO, Convida Wireless, Apple, Vivo and Huawei/HiSilicon.

Discussion point 1 about the impact on the endorsed MAC CR due to the reply LS from RAN1

- 9 companies think that there is no further impact on the endorsed MAC CR due the reply LS from RAN1:

- 4 companies think that there is impact:

- 2 companies think that according to the LS, the text in MAC CR needs to be aligned with the RAN1 specification, i.e. referring to “positive indication” in the MAC CR.

- 1 company proposes to remove the condition regarding DCP overlapping cases in the MAC CR.

- 1 company proposes to add the case that all DCP occasion(s) are invalid in the MAC CR.

Proposal 1: No further update on the endorsed MAC CR is needed due to the RAN1 reply LS on MAC-RRC modelling.

Discussion point 2 on other MAC open issue

No other MAC open issue was raised in this discussion point.

# Conclusion

This email discussion focused on MAC open issues in Rel-16 UE power saving. The following proposal is made according to the summary:

Proposal 1: No further update on the endorsed MAC CR is needed due to the RAN1 reply LS on MAC-RRC modelling.

# References

- R2-2002201, “LS on DCP”, RAN2, RAN2#109e

- R1-2003068, “Reply LS on DCP”, RAN1, RAN1#100bis-e

- R2-2003804, “Meeting Report for NR-U, Power Savings and 2-step RACH”, Session Chair (InterDigital), RAN2#109bis-e

- R2-2004175, “LS on RAN2 DCP Open Issues”, RAN2, RAN2#109bis-e

- R2-2003975, “MAC CR for Rel-16 UE power saving”, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, RAN2#109bis-e